Table of Contents

# Rapids Index

Generated: 2026-01-15

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Memory IO Processor (RAPIDS) Specification

## Chapter 1: Overview

- Overview

- Architecture Requirements

- Clocking and Reset

- Acronyms and Abbreviations

- References

## Chapter 2: Blocks

- Overview

### Scheduler Group

- Scheduler Group

- Scheduler

- Descriptor Engine

- Control Write Engine

- Control Read Engine

### Sink Data Path

- Sink Data Path

- Network Slave

- Sink SRAM Controller

- Sink AXI4 Write Engine

### Source Data Path

- Source Data Path

- Network Master

- Source SRAM Controller

- Source AXI4 Read Engine

### MonBus AXIL Group

- MonBus AXIL Group

### FSM Summary

- FSM Summary

## Chapter 3: Interfaces and Busses

### Top-Level Ports

- RAPIDS Ports

### Bus Definitions

- AXIL4

- AXI4

- AXIS4 (AXI4-Stream)

- MONBUS

## Chapter 4: Programming Models

- RAPIDS Registers

## Chapter 5: Registers

- RAPIDS Registers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Chapter 1.1: RAPIDS Overview

## 1.1.1 Executive Summary

RAPIDS (Rapid AXI Programmable In-band Descriptor System) is a high-performance DMA engine designed to bridge AXI4 memory-mapped interfaces with the Delta Network compute fabric. It operates under control of the HIVE-C master controller (VexRiscv RISC-V core) which provides inband descriptor injection through the Delta Network. RAPIDS intelligently routes data between external DDR memory and compute tiles while enforcing strict packet type filtering to maintain protocol integrity.

Key Features: - Scatter-gather DMA with linked descriptor chains - Inband descriptor reception via CDA (Compute Direct Access) packets - no memory polling - Multi-priority descriptor scheduling for efficient task execution - 16-channel S2MM architecture (one channel per compute tile in 4×4 mesh) - Packet type filtering on all AXIS interfaces - Performance monitoring and statistics collection - Completion interrupt generation

## 1.1.2 Position in System Architecture

RAPIDS serves as the critical bridge between the memory subsystem and the Delta Network compute fabric:

┌─────────────────────────────────────────────────────────────┐
│                         HIVE-C                              │
│                  (VexRiscv RISC-V Master)                   │
│                                                             │
│  Generates CDA descriptors, injects via AXIS               │
└──────────────────────┬──────────────────────────────────────┘
                       │ AXIS TX (CDA packets)
                       ▼
              ┌────────────────────┐
              │   Delta Network    │
              │   Routes packets   │
              │   based on TUSER   │
              └────────┬───────────┘
                       │ CDA -> RAPIDS (tile 16)
                       │ PKT_DATA -> Compute Tiles (0-15)
                       ▼
┌──────────────────────────────────────────────────────────────┐
│                      RAPIDS DMA Engine                       │
│                    (Virtual Tile 16)                         │
│  ┌─────────────────────────────────────────────────────┐    │
│  │  AXIS CDA Input (Slave)                             │    │
│  │  - Accepts: CDA packets only                        │    │
│  │  - Rejects: PKT_DATA, PKT_CONFIG, PKT_STATUS        │    │
│  │  - Routes to: Descriptor FIFO                       │    │
│  └──────────────────┬──────────────────────────────────┘    │
│                     ▼                                        │
│  ┌─────────────────────────────────────────────────────┐    │
│  │  Descriptor Processing Engine                       │    │
│  │  - Parses 256-bit descriptors                       │    │
│  │  - Schedules by priority                            │    │
│  │  - Generates AXI4 transactions                      │    │
│  └──────────────────┬──────────────────────────────────┘    │
│                     ▼                                        │
│  ┌─────────────────────────────────────────────────────┐    │
│  │  AXI4 Memory Interface (Master)                     │    │
│  │  - Reads from DDR (activations, weights)            │    │
│  │  - Writes to DDR (results)                          │    │
│  └──────────────────┬──────────────────────────────────┘    │
│                     ▼                                        │
│  ┌─────────────────────────────────────────────────────┐    │
│  │  AXIS Data Output (Master)                          │    │
│  │  - Sends: PKT_DATA only                             │    │
│  │  - Never sends: CDA, PKT_CONFIG, PKT_STATUS         │    │
│  │  - Routes to: Delta Network -> Compute Tiles         │    │
│  └──────────────────┬──────────────────────────────────┘    │
└────────────────────┼────────────────────────────────────────┘
                     │
                     ▼
            ┌─────────────────┐
            │ Delta Network   │
            │ (PKT_DATA only) │
            └─────────┬───────┘
                      ▼
            Compute Tiles (0-15)

Memory: DDR2/DDR3 ◄──AXI4──┤ RAPIDS ├──AXIS──► Delta Network

## 1.1.3 Virtual Tile Concept

Within the Delta Network topology, RAPIDS is addressed as Virtual Tile 16:

Delta Network 4×4 Physical Mesh (Tiles 0-15)
┌────┬────┬────┬────┐
│ T0 │ T1 │ T2 │ T3 │  Row 0
├────┼────┼────┼────┤
│ T4 │ T5 │ T6 │ T7 │  Row 1
├────┼────┼────┼────┤
│ T8 │ T9 │T10 │T11 │  Row 2
├────┼────┼────┼────┤
│T12 │T13 │T14 │T15 │  Row 3
└────┴────┴────┴────┘
         │
         └─── Virtual Tile 16 (RAPIDS) connected via Router 12 south port

Virtual Tile 17 (HIVE-C) connected via Router 3 north port

Routing Implications: - To RAPIDS: TDEST = 16 routes packets from compute tiles to RAPIDS - From RAPIDS: TDEST = 0-15 routes data to specific compute tiles - CDA packets: Automatically routed to RAPIDS (tile 16) by Delta Network

## 1.1.4 Packet Type Handling

RAPIDS enforces strict packet type filtering based on TUSER encoding:

| Interface | Direction | Accepts | Sends | Rejects |
| --- | --- | --- | --- | --- |
| AXIS CDA Input | Slave | CDA packets | - | PKT_DATA, PKT_CONFIG, PKT_STATUS |
| AXIS Data Out | Master | - | PKT_DATA | CDA, PKT_CONFIG, PKT_STATUS |
| AXIS Data In | Slave | PKT_DATA | - | CDA, PKT_CONFIG, PKT_STATUS |

### Packet Type Encoding (TUSER[1:0])

| TUSER[1:0] | Packet Type | Generated By | Accepted By |
| --- | --- | --- | --- |
| 2’b00 | PKT_DATA | RAPIDS, Compute Tiles | RAPIDS, Compute Tiles |
| 2’b01 | CDA (Compute Direct Access) | HIVE-C only | RAPIDS only |
| 2’b10 | PKT_CONFIG | HIVE-C, SERV monitors | Tile Routers |
| 2’b11 | PKT_STATUS | SERV monitors | HIVE-C |

Rejection Behavior: - Invalid packet type on input: Assert TREADY=0, set error flag, generate interrupt - Transmission enforcement: Hardware prevents transmission of invalid packet types

## 1.1.5 Data Flow: Memory to Compute (MM2S)

1. HIVE-C generates CDA descriptor in firmware
2. HIVE-C writes descriptor to AXIS TX FIFO with TUSER=CDA (2'b01)
3. Delta Network routes CDA packet to RAPIDS (tile 16)
4. RAPIDS validates TUSER == CDA, writes to descriptor FIFO
5. Scheduler prioritizes and fetches descriptor
6. MM2S engine:
   a. Reads data from DDR via AXI4
   b. Buffers in internal FIFO
   c. Formats AXIS packets with TUSER=PKT_DATA (2'b00)
   d. Sets TDEST to target compute tile (from descriptor)
   e. Transmits to Delta Network
7. Delta Network routes PKT_DATA to destination tile
8. Completion interrupt generated (if enabled)

## 1.1.6 Data Flow: Compute to Memory (S2MM)

RAPIDS implements 16 independent S2MM channels, one per compute tile:

1. HIVE-C pre-programs S2MM descriptors (one per active tile)
2. Compute tiles generate results, send as PKT_DATA with:
   - TUSER = PKT_DATA (2'b00)
   - TID = source tile ID (0-15)  <- CRITICAL for channel routing
   - TDEST = 16 (RAPIDS virtual tile)
3. Delta Network routes to RAPIDS
4. RAPIDS S2MM channel router:
   channel_id = TID[3:0]  // Direct mapping: TID -> Channel
5. Selected S2MM channel:
   a. Validates TUSER == PKT_DATA
   b. Buffers in per-channel FIFO
   c. Fetches descriptor from per-channel queue
   d. Writes to DDR via AXI4 (address from descriptor)
6. Completion interrupt generated (if enabled)

Multi-Channel Benefits: - Concurrent writes: Multiple tiles write simultaneously - Independent addressing: Each tile’s data goes to different DDR region - Per-tile flow control: Backpressure on one tile doesn’t affect others - Simplified routing: Direct TID -> Channel mapping (no lookup table)

## 1.1.7 CDA Packet Format

CDA (Compute Direct Access) packets carry descriptor information from HIVE-C to RAPIDS:

Format: 256 bits (2 AXIS beats at 128-bit width)

Beat 0:
TUSER[1:0] = 2'b01        // CDA packet type
TDATA[127:0] = Desc[127:0] // Descriptor bits [127:0]
TLAST = 0                 // More beats follow
TID[3:0] = Priority       // 0=highest, 15=lowest
TDEST[3:0] = 4'h0         // Always RAPIDS (routed to tile 16)

Beat 1:
TUSER[1:0] = 2'b01        // CDA packet type
TDATA[127:0] = Desc[255:128] // Descriptor bits [255:128]
TLAST = 1                 // Final beat
TID[3:0] = Priority       // Same as beat 0
TDEST[3:0] = 4'h0         // Always RAPIDS

Descriptor Fields (256 bits total): - Bits [255:192]: Source Address (64-bit DDR address) - Bits [191:128]: Destination Address (64-bit DDR address) - Bits [127:96]: Transfer Length (32-bit bytes) - Bits [95:64]: Control/Configuration (stride, tile ID, etc.) - Bits [63:32]: Control Flags (type, priority, interrupts) - Bits [31:0]: Next Descriptor Pointer (scatter-gather chaining)

See Chapter 2 (Descriptor Engine) for complete descriptor field definitions.

## 1.1.8 System Integration Summary

RAPIDS Never Generates: - CDA packets (only HIVE-C generates these) - PKT_CONFIG (only HIVE-C or SERV monitors generate) - PKT_STATUS (only SERV monitors generate)

RAPIDS Always Validates: - CDA input packets must have TUSER = 2’b01 - Data input packets must have TUSER = 2’b00 - Any other TUSER value triggers error condition and packet rejection

Control Plane: - HIVE-C uses AXI4-Lite to access RAPIDS control/status registers - CDA packets provide descriptor injection (inband, low latency) - Interrupts notify HIVE-C of completion/errors

Data Plane: - AXI4 master interface to DDR memory (read/write) - AXIS master interface to Delta Network (PKT_DATA out) - AXIS slave interface from Delta Network (PKT_DATA in) - Multi-channel architecture supports concurrent tile operations

Next: Architectural Requirements

Back to: Index

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Chapter 1.2: Architectural Requirements

## 1.2.1 System Requirements

### SR-1: Delta Network Integration

RAPIDS shall integrate with the Delta Network as Virtual Tile 16, accepting CDA packets from HIVE-C and routing PKT_DATA packets to/from compute tiles.

### SR-2: HIVE-C Control Interface

RAPIDS shall provide an AXI4-Lite slave interface for control/status register access by HIVE-C.

### SR-3: Memory Interface

RAPIDS shall provide an AXI4 master interface supporting burst transactions to DDR2/DDR3/DDR4 memory controllers.

### SR-4: Packet Type Enforcement

RAPIDS shall enforce strict packet type filtering on all AXIS interfaces, rejecting invalid packet types with error indication.

## 1.2.2 Descriptor Requirements

### DR-1: Inband Descriptor Reception

RAPIDS shall accept descriptor injection via CDA packets from HIVE-C through the Delta Network (no memory polling required).

### DR-2: Descriptor Queue Depth

RAPIDS shall support a minimum of 8 pending descriptors in the descriptor FIFO.

### DR-3: Priority Scheduling

RAPIDS shall support priority-based descriptor scheduling with 16 priority levels (0=highest, 15=lowest).

### DR-4: Scatter-Gather Support

RAPIDS shall support linked descriptor chains via next descriptor pointer field.

### DR-5: Descriptor Format

RAPIDS shall accept 256-bit descriptors conforming to the format specified in Chapter 2.

## 1.2.3 Data Transfer Requirements

### DT-1: Memory-to-Stream (MM2S) Engine

RAPIDS shall support reading data from DDR memory and transmitting as PKT_DATA packets to the Delta Network.

### DT-2: Stream-to-Memory (S2MM) Multi-Channel Architecture

RAPIDS shall implement 16 independent S2MM channels, one per compute tile in a 4×4 mesh, with direct TID-to-channel mapping.

### DT-3: Concurrent Channel Operation

S2MM channels shall operate independently, allowing multiple compute tiles to write to memory simultaneously.

### DT-4: Burst Support

RAPIDS shall support AXI4 burst transactions with lengths up to 256 beats.

### DT-5: Data Buffering

RAPIDS shall provide internal FIFOs to decouple memory timing from Delta Network timing: - MM2S data FIFO: Minimum 512 entries × 128 bits - S2MM per-channel FIFO: Minimum 32 entries × 128 bits

## 1.2.4 Performance Requirements

### PR-1: Throughput

RAPIDS shall sustain a minimum of 1.3 GB/s for large (>1 KB) transfers under ideal conditions.

### PR-2: Latency - Descriptor Injection

RAPIDS shall process CDA packet descriptors within 20 cycles of reception (from HIVE-C via Delta Network).

### PR-3: Latency - First Data

RAPIDS shall deliver first data beat within 65 cycles of descriptor execution start (MM2S mode, DDR3 target).

### PR-4: Burst Efficiency

RAPIDS shall achieve >90% burst efficiency for transfers >1 KB.

## 1.2.5 Interface Requirements

### IF-1: AXIS CDA Input

RAPIDS shall provide an AXIS slave interface accepting only CDA packets (TUSER = 2’b01) from the Delta Network.

Signals: - TDATA: 128 bits - TUSER: 2 bits (must be 2’b01) - TID: 4 bits (descriptor priority) - TDEST: 4 bits (always 0 for RAPIDS routing) - TLAST: 1 bit - TVALID/TREADY: Handshake

### IF-2: AXIS Data Output (MM2S)

RAPIDS shall provide an AXIS master interface transmitting only PKT_DATA packets (TUSER = 2’b00) to the Delta Network.

Signals: - TDATA: 128 bits - TUSER: 2 bits (always 2’b00) - TID: 4 bits (descriptor priority) - TDEST: 4 bits (target tile ID 0-15) - TLAST: 1 bit - TKEEP: 16 bits - TVALID/TREADY: Handshake

### IF-3: AXIS Data Input (S2MM)

RAPIDS shall provide an AXIS slave interface accepting only PKT_DATA packets (TUSER = 2’b00) from Delta Network compute tiles.

Signals: - TDATA: 128 bits - TUSER: 2 bits (must be 2’b00) - TID: 4 bits (source tile ID 0-15) <- Used for channel routing - TDEST: 4 bits (must be 16 for RAPIDS) - TLAST: 1 bit - TKEEP: 16 bits - TVALID/TREADY: Handshake

### IF-4: AXI4 Memory Interface

RAPIDS shall provide an AXI4 master interface with: - Address width: 32 bits (supports up to 4 GB) - Data width: 128 bits (configurable 64/128/256) - Burst types: INCR, FIXED, WRAP - Max burst length: 256 beats - Outstanding transactions: 16 (configurable)

### IF-5: AXI4-Lite Control Interface

RAPIDS shall provide an AXI4-Lite slave interface for control/status registers with: - Address width: 12 bits (4 KB register space) - Data width: 32 bits - Single outstanding transaction

## 1.2.6 Error Handling Requirements

### EH-1: Packet Type Validation

RAPIDS shall reject packets with invalid TUSER values by: - Asserting TREADY = 0 (not accepting packet) - Setting appropriate error flag in status register - Generating interrupt (if enabled)

### EH-2: AXI4 Error Response

RAPIDS shall handle AXI4 SLVERR and DECERR responses by: - Setting error flag in status register - Optionally skipping to next descriptor - Generating interrupt (if enabled)

### EH-3: Descriptor FIFO Overflow

RAPIDS shall prevent descriptor loss on FIFO full by: - Asserting backpressure (TREADY = 0) on CDA input - Setting FIFO full flag in status register

### EH-4: Malformed Descriptor

RAPIDS shall handle malformed descriptors by: - Setting error flag - Skipping to next descriptor (if scatter-gather enabled) - Generating interrupt (if enabled)

## 1.2.7 Monitoring and Debug Requirements

### MD-1: Performance Counters

RAPIDS shall provide read-only counters for: - Bytes read from memory - Bytes written to memory - Packets transmitted (AXIS out) - Packets received (AXIS in) - AXI4 read/write cycle counts

### MD-2: Status Visibility

RAPIDS shall provide status indicators for: - Descriptor FIFO occupancy - Data FIFO status (full/empty flags) - Current priority being processed - Engine busy indicators (MM2S, S2MM per-channel) - Scatter-gather chain active

### MD-3: Interrupt Generation

RAPIDS shall generate interrupts for: - Descriptor completion (configurable per-descriptor) - FIFO full conditions - AXI4 errors - Invalid packet type reception - Descriptor parse errors

## 1.2.8 Configuration Requirements

### CF-1: Compile-Time Parameters

RAPIDS shall support compile-time configuration of: - AXI4 data width (64/128/256 bits) - AXI4 address width (32/64 bits) - Descriptor FIFO depth (8-256 entries) - Data FIFO depths (MM2S and S2MM) - Number of S2MM channels (4/8/16 for 2×2/3×3/4×4 meshes) - Outstanding AXI4 transaction limit

### CF-2: Runtime Configuration

RAPIDS shall support runtime configuration via registers: - Engine enable/disable (MM2S, S2MM) - Interrupt mask - Error handling policy - Statistics counter enable

## 1.2.9 Reset and Initialization Requirements

### RI-1: Synchronous Reset

RAPIDS shall support synchronous active-low reset (aresetn) for all internal state.

### RI-2: Graceful Reset

RAPIDS shall complete in-flight AXI4 transactions before asserting internal reset (when possible).

### RI-3: FIFO Flush

RAPIDS shall support selective FIFO flushing via control register: - Descriptor FIFO flush - Data FIFO flush (MM2S and S2MM)

### RI-4: Initialization State

After reset deassertion, RAPIDS shall: - Clear all FIFOs - Disable all engines (MM2S, S2MM channels) - Clear all error flags - Reset performance counters

Next: Clocks and Reset

Previous: Overview

Back to: Index

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Chapter 1.3: Clocks and Reset

## 1.3.1 Clock Domains

RAPIDS operates within a multi-clock domain system due to its integration with Delta Network and DDR memory:

### Primary Clock Domains

1. APB Control Clock (pclk) - Frequency: Typically 50-100 MHz - Purpose: Control/status register access by HIVE-C - Interface: AXI4-Lite slave - Requirements: Synchronous to HIVE-C system clock

2. AXI4 Memory Clock (aclk) - Frequency: Typically 200-400 MHz (DDR2/DDR3/DDR4 dependent) - Purpose: High-performance memory access - Interfaces: - AXI4 master (read/write to DDR) - Internal RAPIDS core logic - Requirements: Synchronous to memory controller clock

3. Delta Network Clock (network_clk) - Frequency: Typically 250-500 MHz - Purpose: Packet routing and network communication - Interfaces: - AXIS CDA input (slave) - AXIS Data output (master) - AXIS Data input (slave) - Requirements: Synchronous to Delta Network fabric

### Clock Domain Relationships

┌─────────────────────────────────────────────────────────────────┐
│                           HIVE-C                                │
│                   (Control Plane Clock)                         │
│                         50-100 MHz                              │
└──────────────────────────┬──────────────────────────────────────┘
                           │ pclk
                           ▼
              ┌────────────────────────────┐
              │   APB Control Interface    │
              │   (AXI4-Lite Slave)       │
              └────────────┬───────────────┘
                           │ CDC if async
                           ▼
┌─────────────────────────────────────────────────────────────────┐
│                        RAPIDS Core                              │
│                    (aclk - Memory Clock)                        │
│                       200-400 MHz                               │
│  ┌──────────────┐  ┌──────────────┐  ┌──────────────┐         │
│  │ Descriptor   │  │    MM2S      │  │    S2MM      │         │
│  │   Engine     │  │   Engine     │  │  Channels    │         │
│  └──────────────┘  └──────────────┘  └──────────────┘         │
└────────────┬────────────────┬─────────────────┬────────────────┘
             │                │                 │
             ▼                ▼                 ▼
    ┌────────────────┐  ┌─────────────┐  ┌─────────────┐
    │ AXI4 Memory    │  │  CDC FIFO   │  │  CDC FIFO   │
    │   Interface    │  │  MM2S->AXIS  │  │  AXIS->S2MM  │
    │ (aclk domain)  │  └──────┬──────┘  └──────┬──────┘
    └────────┬───────┘         │ CDC            │ CDC
             │                 ▼                 ▼
             ▼         ┌───────────────────────────────────┐
    ┌────────────┐    │      Delta Network               │
    │    DDR     │    │   (network_clk - 250-500 MHz)    │
    │  Memory    │    │                                   │
    └────────────┘    │  AXIS TX (PKT_DATA)              │
                      │  AXIS RX (PKT_DATA)              │
                      │  AXIS CDA (descriptors)          │
                      └───────────────────────────────────┘

## 1.3.2 Clock Domain Crossing (CDC)

### CDC Requirements

RAPIDS requires CDC for these interface crossings:

1. APB <-> Core (Optional) - When: If HIVE-C APB clock differs from RAPIDS core clock - Method: APB slave CDC (handshake synchronization) - Latency Impact: 4-6 clock cycles for register access - Parameter: CDC_ENABLE (0=synchronous, 1=async)

2. Core <-> Delta Network (Typical) - Crossing: aclk (memory) <-> network_clk (Delta Network) - Method: Asynchronous FIFOs with gray-coded pointers - Locations: - MM2S data FIFO (memory -> network) - S2MM data FIFOs (network -> memory, per-channel) - CDA descriptor FIFO (network -> core) - Depth: Minimum 32 entries to absorb clock ratio variations

3. Core <-> Memory (Synchronous) - Assumption: RAPIDS core runs at memory clock (aclk) - No CDC Required: Direct AXI4 connection - Benefit: Lowest latency for memory access

### CDC FIFO Specifications

MM2S Data FIFO:

Write Side: aclk (RAPIDS core)
Read Side: network_clk (Delta Network)
Depth: 512 entries × DATA_WIDTH
Empty/Full: Gray-coded flags with multi-stage synchronization

S2MM Data FIFO (per channel):

Write Side: network_clk (Delta Network)
Read Side: aclk (RAPIDS core)
Depth: 32 entries × DATA_WIDTH
Empty/Full: Gray-coded flags with multi-stage synchronization

CDA Descriptor FIFO:

Write Side: network_clk (Delta Network CDA packets)
Read Side: aclk (RAPIDS descriptor processing)
Depth: 8 descriptors × 256 bits
Empty/Full: Gray-coded flags with multi-stage synchronization

## 1.3.3 Reset Strategy

### Reset Signals

RAPIDS uses synchronous active-low reset for all clock domains:

1. APB Reset (presetn) - Domain: APB control interface - Type: Active-low synchronous - Source: HIVE-C system reset - Scope: APB slave interface only

2. Core Reset (aresetn) - Domain: RAPIDS core logic + AXI4 memory interface - Type: Active-low synchronous (AXI4 standard) - Source: System reset - Scope: All RAPIDS internal state, descriptor engine, scheduler, data paths

3. Network Reset (network_rst_n) - Domain: Delta Network interfaces (AXIS) - Type: Active-low synchronous - Source: Delta Network fabric reset - Scope: AXIS interfaces, CDC FIFOs (network side)

### Reset Sequencing

Recommended power-on reset sequence:

1. Assert all resets (presetn=0, aresetn=0, network_rst_n=0)
   - Hold for minimum 10 cycles in slowest domain

2. Deassert network_rst_n (Delta Network reset)
   - Allow Delta Network fabric to initialize
   - Wait 100 cycles (network_clk)

3. Deassert aresetn (RAPIDS core reset)
   - RAPIDS core initializes
   - CDC FIFOs reset
   - Wait 50 cycles (aclk)

4. Deassert presetn (APB control reset)
   - APB interface ready
   - HIVE-C can access control registers

5. Software initialization
   - HIVE-C configures RAPIDS via APB
   - Enable engines, set initial credits

### Reset Requirements

RI-1: Synchronous Reset - All reset signals synchronized to respective clock domains - No asynchronous reset (metastability risk)

RI-2: Graceful Reset - RAPIDS completes in-flight AXI4 transactions before internal reset (when possible) - CDC FIFOs flushed cleanly (no partial packets)

RI-3: FIFO Flush - Selective FIFO flushing via control register: - Descriptor FIFO flush - MM2S data FIFO flush - S2MM data FIFO flush (per-channel)

RI-4: Initialization State

After reset deassertion, RAPIDS guarantees: - [x] All FIFOs empty - [x] All engines disabled (MM2S, S2MM channels) - [x] All error flags cleared - [x] Performance counters reset to 0 - [x] Descriptor queue empty - [x] AXI4 interfaces idle

## 1.3.4 Clock Constraints

### Frequency Relationships

Minimum Ratios: - network_clk : aclk >= 1:1 (network can be slower, same, or faster) - aclk : pclk >= 2:1 (core typically 2-4× faster than APB)

Recommended: - pclk = 50-100 MHz (APB control) - aclk = 200-400 MHz (DDR3/DDR4 memory) - network_clk = 250-500 MHz (Delta Network fabric)

CDC FIFO Sizing: - For network_clk : aclk = 2:1 (network faster): Larger write-side headroom - For network_clk : aclk = 1:2 (memory faster): Larger read-side headroom

### Jitter and Skew

Clock Jitter Tolerance: - ±100 ps peak-to-peak jitter (typical for on-chip PLLs) - CDC FIFOs absorb jitter via async crossing

Clock Skew: - Same-domain: <200 ps (within RAPIDS core on aclk) - Cross-domain: Not applicable (async FIFOs tolerate arbitrary skew)

## 1.3.5 Power Management (Optional)

Clock Gating: - Individual engine clock gating when disabled - Descriptor engine clock gated when queue empty - S2MM channel clock gating when inactive

Power Domains: - Core domain: aclk (always on during operation) - Control domain: pclk (can be gated when HIVE-C idle) - Network domain: network_clk (controlled by Delta Network fabric)

Next: Acronyms

Previous: Architectural Requirements

Back to: Index

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Chapter 1.4: Acronyms and Abbreviations

## A

APB - Advanced Peripheral Bus (AMBA 3 specification for low-bandwidth control interfaces)

AR - Address Read (AXI4 read address channel)

AMBA - Advanced Microcontroller Bus Architecture (ARM interconnect standard)

AW - Address Write (AXI4 write address channel)

AXI4 - Advanced eXtensible Interface version 4 (high-performance memory-mapped protocol)

AXI4-Lite (AXIL) - Subset of AXI4 for simple control/status register access

AXIS - AXI4-Stream (packet-oriented streaming protocol)

ASIC - Application-Specific Integrated Circuit

## B

BFM - Bus Functional Model (testbench component)

B Channel - AXI4 write response channel

## C

CDA - Compute Direct Access (packet type for descriptor injection from HIVE-C to RAPIDS)

CDC - Clock Domain Crossing (asynchronous interface between different clock domains)

Comparator - Timer comparison value in HPET peripheral

## D

DDR - Double Data Rate (SDRAM memory technology: DDR2, DDR3, DDR4)

DDR2/DDR3/DDR4 - Generations of DDR memory (DDR2: 400-800 MT/s, DDR3: 800-1600 MT/s, DDR4: 1600-3200 MT/s)

Delta Network - 4×4 mesh Network-on-Chip for compute fabric interconnection

Descriptor - Data structure containing DMA transfer parameters (source, destination, length, control)

DMA - Direct Memory Access (hardware-managed memory transfers without CPU intervention)

DRAM - Dynamic Random Access Memory

DUT - Device Under Test (module being verified in testbench)

## F

FIFO - First In First Out (queue buffer structure)

FPGA - Field-Programmable Gate Array

FSM - Finite State Machine (sequential logic with discrete states)

## H

HIVE-C - High-performance Integrated VexRiscv Engine for Control (RISC-V master controller)

HPET - High Precision Event Timer (multi-timer peripheral)

## I

ID - Transaction identifier (AXI4 signal for out-of-order response routing)

INCR - Incrementing burst type (AXI4 burst mode)

## M

Mesh - Network topology with regular grid structure (4×4 for Delta Network)

MM2S - Memory-to-Stream (read from DDR memory, transmit to Delta Network)

MonBus - Monitor Bus (standardized 64-bit monitoring protocol)

## N

Network Slave - RAPIDS interface receiving packets from Delta Network

Network Master - RAPIDS interface transmitting packets to Delta Network

NoC - Network-on-Chip (on-chip interconnect fabric)

## P

Packet - Unit of data transmission on AXIS/Delta Network interfaces

PKT_CONFIG - Packet type for configuration (TUSER = 2’b10)

PKT_DATA - Packet type for compute data (TUSER = 2’b00)

PKT_STATUS - Packet type for monitoring/status (TUSER = 2’b11)

Priority - Descriptor scheduling weight (0=highest, 15=lowest in RAPIDS)

## R

R Channel - AXI4 read data channel

RAPIDS - Rapid AXI Programmable In-band Descriptor System (DMA engine)

RISC-V - Open-standard instruction set architecture (used in HIVE-C)

Router - Delta Network component for packet routing between tiles

## S

S2MM - Stream-to-Memory (receive from Delta Network, write to DDR memory)

Scatter-Gather - DMA technique using linked descriptor chains for non-contiguous transfers

SERV - System Event Reporting and Verification (monitoring subsystem)

SRAM - Static Random Access Memory (on-chip buffer)

## T

TDEST - Transaction Destination (AXIS signal indicating target tile/destination)

TID - Transaction ID (AXIS signal for source identification or priority)

Tile - Compute element in Delta Network mesh (physical: 0-15, virtual: 16-17)

TLAST - Transaction Last (AXIS signal indicating final beat of packet)

TREADY - Transaction Ready (AXIS backpressure signal)

TUSER - Transaction User (AXIS signal encoding packet type)

TVALID - Transaction Valid (AXIS signal indicating valid data)

## V

VexRiscv - RISC-V softcore CPU implementation (used in HIVE-C)

Virtual Tile - Non-physical network endpoint (RAPIDS=16, HIVE-C=17)

VC - Virtual Channel (independent flow control paths in router)

## W

W Channel - AXI4 write data channel

W1C - Write-1-to-Clear (register bit cleared by writing 1, not 0)

WRAP - Wrapping burst type (AXI4 burst mode for circular addressing)

## System-Specific Terms

Compute Tile - Processing element in Delta Network (tiles 0-15)

Descriptor Chain - Linked list of descriptors for scatter-gather DMA

Descriptor FIFO - Queue storing pending DMA descriptors (minimum 8 entries)

Inband Descriptor - Descriptor injected via CDA packets (no memory polling)

Virtual Tile 16 - RAPIDS DMA engine network address

Virtual Tile 17 - HIVE-C control processor network address

XY Routing - Dimension-ordered routing (X-dimension first, then Y-dimension)

## Packet Type Summary

| Acronym | TUSER | Full Name | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| PKT_DATA | 2’b00 | Data Packet | RAPIDS/Tiles | RAPIDS/Tiles | Compute data transfers |
| CDA | 2’b01 | Compute Direct Access | HIVE-C | RAPIDS | Descriptor injection |
| PKT_CONFIG | 2’b10 | Configuration Packet | HIVE-C/SERV | Routers/Tiles | Configuration |
| PKT_STATUS | 2’b11 | Status Packet | SERV | HIVE-C | Monitoring/status |

Next: References

Previous: Clocks and Reset

Back to: Index

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Chapter 1.5: References

## Internal Specifications

### RAPIDS Project Documentation

[1] RAPIDS Product Requirements Document projects/components/rapids/PRD.md Complete requirements specification for RAPIDS subsystem

[2] RAPIDS CLAUDE Guide projects/components/rapids/CLAUDE.md AI-specific guidance for working with RAPIDS subsystem

[3] RAPIDS Implementation Status projects/components/rapids/IMPLEMENTATION_STATUS.md Current development status and test results

[4] RAPIDS Specification Context Document projects/components/rapids/docs/rapids_specification_hive_context.md Original HIVE/Delta Network integration specification (source for this update)

### Delta Network Documentation

[5] Delta Network Specification v0.3 projects/components/delta/docs/delta_spec/delta_index.md Complete specification for Delta Network mesh Network-on-Chip

[6] Delta Router Architecture projects/components/delta/docs/delta_spec/ch02_blocks/01_router_architecture.md Detailed router architecture including XY routing and virtual channels

[7] Delta Packet Type Routing projects/components/delta/docs/delta_spec/ch01_overview/03_packet_type_routing.md Packet type definitions (PKT_DATA, CDA, PKT_CONFIG, PKT_STATUS)

[8] Delta External Entities projects/components/delta/docs/delta_spec/ch03_interfaces/03_external_entities.md Integration specifications for RAPIDS and HIVE-C as virtual tiles

### HIVE-C Documentation

[9] HIVE-C System Specification (Location TBD - VexRiscv RISC-V control processor specification)

[10] HIVE-C Software Guide (Location TBD - Firmware development guide for descriptor generation)

### RTL Design Sherpa Project

[11] RTL Design Sherpa Root PRD /PRD.md Master product requirements document for repository

[12] RTL Design Sherpa CLAUDE Guide /CLAUDE.md Repository-wide guidance for AI-assisted development

[13] AMBA Protocol Documentation rtl/amba/PRD.md AMBA protocol infrastructure (AXI4, APB, AXIS monitors)

[14] Verification Architecture Guide docs/VERIFICATION_ARCHITECTURE_GUIDE.md Complete testbench architecture patterns and best practices

## External Standards and Specifications

### ARM AMBA Specifications

[15] AMBA AXI and ACE Protocol Specification (AXI4) ARM IHI 0022E, 2013 Complete specification for AXI4 memory-mapped protocol https://developer.arm.com/documentation/ihi0022/latest

[16] AMBA AXI4-Stream Protocol Specification ARM IHI 0051A, 2010 Specification for packet-oriented streaming protocol (AXIS) https://developer.arm.com/documentation/ihi0051/latest

[17] AMBA APB Protocol Specification v2.0 ARM IHI 0024C, 2010 Specification for Advanced Peripheral Bus (control/status registers) https://developer.arm.com/documentation/ihi0024/latest

[18] AMBA AXI4-Lite Protocol Subset of AXI4 specification (Chapter E1 of [15]) Simplified protocol for control/status register access

### Memory Specifications

[19] DDR2 SDRAM Specification JESD79-2F, JEDEC Standard Double Data Rate 2 SDRAM specification

[20] DDR3 SDRAM Specification JESD79-3F, JEDEC Standard Double Data Rate 3 SDRAM specification (commonly used with RAPIDS)

[21] DDR4 SDRAM Specification JESD79-4C, JEDEC Standard Double Data Rate 4 SDRAM specification

### RISC-V Architecture

[22] The RISC-V Instruction Set Manual, Volume I: User-Level ISA Version 20191213, RISC-V Foundation https://riscv.org/technical/specifications/

[23] VexRiscv RISC-V CPU Core GitHub: SpinalHDL/VexRiscv Open-source RISC-V implementation (used in HIVE-C) https://github.com/SpinalHDL/VexRiscv

### Verification and Testbench

[24] CocoTB Documentation Coroutine-based cosimulation testbench environment for Python https://docs.cocotb.org/

[25] Verilator Manual Fast Verilog/SystemVerilog simulator https://verilator.org/guide/latest/

## Design Patterns and Best Practices

### Network-on-Chip Design

[26] Dally, W.J. and Towles, B., “Principles and Practices of Interconnection Networks” Morgan Kaufmann, 2004 Foundational text for NoC design principles

[27] Duato, J., Yalamanchili, S., and Ni, L., “Interconnection Networks: An Engineering Approach” Morgan Kaufmann, 2002 Deadlock-free routing algorithms (XY routing used in Delta Network)

### DMA and Scatter-Gather

[28] PCI Express Base Specification Revision 4.0 PCI-SIG, 2017 Reference for descriptor-based DMA patterns

[29] Intel I/O Acceleration Technology (I/OAT) Specification Intel Corporation Advanced DMA engine architecture (industry reference)

### Clock Domain Crossing

[30] Cummings, C.E., “Clock Domain Crossing (CDC) Design & Verification Techniques Using SystemVerilog” SNUG 2008 Best practices for CDC FIFO design

[31] Cummings, C.E., “Synthesis and Scripting Techniques for Designing Multi-Asynchronous Clock Designs” SNUG 2001 Gray code counters for async FIFO pointers

## Related Projects

### RTL Design Examples

[32] OpenPiton Research Platform Princeton University Open-source manycore processor with mesh NoC https://parallel.princeton.edu/openpiton/

[33] LowRISC Ibex Core lowRISC CIC Open-source RISC-V processor (similar class to VexRiscv) https://github.com/lowRISC/ibex

[34] Ariane RISC-V CPU ETH Zurich High-performance 64-bit RISC-V processor https://github.com/openhwgroup/cva6

### Verification Frameworks

[35] PyUVM Python implementation of UVM (Universal Verification Methodology) https://github.com/pyuvm/pyuvm

[36] cocotb-coverage Functional coverage library for CocoTB https://github.com/mciepluc/cocotb-coverage

## Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | 2025-10-19 | Documentation Team | Initial creation with HIVE/Delta Network context |

Next: Block Overview

Previous: Acronyms

Back to: Index

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

## RAPIDS System Overview

The Memory Input/Output Processor (RAPIDS) is a high-performance data streaming engine operating at 2.5 GHz, designed to provide reliable, low-latency data transfer between external DRAM and processing cores through the Network network. The architecture features dual independent data paths with sophisticated multi-channel support for up to 32 concurrent streams, implementing pure pipeline architectures that eliminate traditional FSM overhead to achieve sustained throughput exceeding 1.2 Tbps per path.

RAPIDS/Network/MSAP

RAPIDS/Network/MSAP

### Scheduler Group

The Scheduler Group provides complete integrated channel processing through an innovative dual-FSM architecture that combines descriptor execution with parallel address alignment optimization. The main scheduler FSM manages descriptor lifecycle, credit operations, and program sequencing, while the address alignment FSM runs concurrently to pre-calculate optimal transfer parameters including burst lengths and chunk enable patterns. This approach eliminates alignment calculation overhead from critical AXI timing paths, enabling sustained high-performance operation while maintaining comprehensive control over stream boundaries and error handling.

### Descriptor Engine

The Descriptor Engine implements a sophisticated six-state finite state machine that orchestrates descriptor fetching and processing operations with dual-path support for both APB programming interface requests and RDA packet interface operations. The FSM manages the complete descriptor lifecycle from initial request validation through AXI read transactions, descriptor parsing, and final output generation. RDA packets receive priority processing over APB requests, ensuring optimal network responsiveness, while comprehensive stream boundary support includes complete EOS/EOL/EOD field extraction and propagation through a 4-deep descriptor FIFO.

### Program Engine

The Program Engine implements a streamlined four-state finite state machine that manages post-processing write operations for each virtual channel after descriptor completion. The FSM handles program address writes with configurable data values to support notification, control, and cleanup operations with comprehensive timeout mechanisms and robust error handling. Conditional programming through graceful null address handling enables descriptor-driven execution, while shared AXI interface operations use ID-based response routing for proper multi-channel coordination and runtime reconfiguration support.

### Source Data Path Group

The Source Data Path implements a complete data transmission pipeline from scheduler requests through Network packet delivery, featuring a revolutionary pure pipeline architecture with zero-FSM overhead for optimal performance. The path integrates a sophisticated AXI read engine with multi-channel arbitration, preallocation-based deadlock prevention, and chunk-aware burst optimization to achieve maximum memory bandwidth utilization. Integrated SRAM control provides multi-channel buffering with stream-aware flow control, while the Network master implements a four-stage pipeline with credit-based flow control and mathematically proven zero packet loss guarantees.

### Sink Data Path Group

The Sink Data Path provides comprehensive data reception and processing from Network packet arrival through final AXI memory writes, implementing sophisticated buffer management and multi-channel arbitration for optimal throughput. The Network slave features bulletproof ACK generation with dual FIFO queues, comprehensive validation, and intelligent packet routing based on packet classification. Sink SRAM control manages multi-channel buffering with sophisticated flow control and EOS completion signaling, while the AXI write engine implements pure pipeline architecture with zero-cycle arbitration, transfer strategy optimization, and chunk-aware write strobes for precise memory utilization.

### Monitor Bus AXI4-Lite Group

The Monitor Bus AXI4-Lite Group provides unified system monitoring and event aggregation across all RAPIDS subsystems, implementing a comprehensive filtering and routing architecture for optimal system visibility. The group aggregates monitor streams from source and sink data paths through round-robin arbitration, applying configurable protocol-specific filtering for AXI, Network, and CORE events. The architecture supports both interrupt generation through error/interrupt FIFOs for critical events and external logging through configurable master write operations, providing complete event coverage across 544 defined event codes spanning all protocols and packet types for comprehensive system debugging and performance analysis.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Scheduler Group

#### Overview

The Scheduler Group provides a complete integrated channel processing unit that combines the Scheduler, Descriptor Engine, and Monitor Bus Aggregator into a cohesive module. This wrapper simplifies system integration by providing a unified interface for complete channel processing functionality.

NOTE: The program engine has been removed from this module and replaced by separate ctrlrd_engine and ctrlwr_engine interfaces. These control engines are instantiated externally and connected through the ctrlrd_* and ctrlwr_* ports. For backwards compatibility, the legacy prog_* AXI ports are retained but tied off.

The wrapper implements coordinated reset handling across all components, enhanced data interface with alignment bus support using RAPIDS package types, and unified monitor bus aggregation for comprehensive system visibility.

scheduler_group

scheduler_group

#### Key Features

- Integrated Channel Processing: Complete descriptor processing and scheduling functionality

- Control Engine Interfaces: Separate ctrlrd (control read) and ctrlwr (control write) interfaces for external control operations

- Descriptor AXI Interface: Single descriptor read interface (512-bit data width) for fetching descriptors

- Legacy Compatibility: Retained prog_* AXI ports (tied off) for backwards compatibility

- Address Alignment Bus: Pre-calculated alignment information using RAPIDS package types for optimal AXI performance

- Channel Reset Coordination: Graceful reset handling with completion signaling

- Monitor Bus Aggregation: Unified monitor output from descriptor engine, scheduler, and (tied-off) program engine

- RAPIDS Package Types: Uses standardized alignment_info_t and transfer_phase_t types for enhanced data interface

#### Interface Specification

##### Clock and Reset

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### APB Programming Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| apb_valid | logic | 1 | Input | Yes | APB descriptor fetch request valid |
| apb_ready | logic | 1 | Output | Yes | APB descriptor fetch request ready |
| apb_addr | logic | ADDR_WIDTH | Input | Yes | APB descriptor address |

##### RDA Packet Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rda_valid | logic | 1 | Input | Yes | RDA packet valid |
| rda_ready | logic | 1 | Output | Yes | RDA packet ready |
| rda_packet | logic | DATA_WIDTH | Input | Yes | RDA packet data |
| rda_channel | logic | CHAN_WIDTH | Input | Yes | RDA target channel |

##### EOS Completion Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| eos_completion_valid | logic | 1 | Input | Yes | EOS completion notification valid |
| eos_completion_ready | logic | 1 | Output | Yes | EOS completion notification ready |
| eos_completion_channel | logic | CHAN_WIDTH | Input | Yes | Channel with EOS completion |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_idle_mode | logic | 1 | Input | Yes | Global idle mode control |
| cfg_channel_wait | logic | 1 | Input | Yes | Channel wait control |
| cfg_channel_enable | logic | 1 | Input | Yes | Channel enable control |
| cfg_use_credit | logic | 1 | Input | Yes | Credit mode enable |
| cfg_initial_credit | logic | 4 | Input | Yes | Initial credit values (exponential encoding: 0->1, 1->2, 2->4, …, 14->16384, 15->infinity) |
| credit_increment | logic | 1 | Input | Yes | Credit increment request |
| cfg_prefetch_enable | logic | 1 | Input | Yes | Descriptor prefetch enable |
| cfg_fifo_threshold | logic | 4 | Input | Yes | Descriptor FIFO threshold |
| cfg_addr0_base | logic | ADDR_WIDTH | Input | Yes | Address range 0 base |
| cfg_addr0_limit | logic | ADDR_WIDTH | Input | Yes | Address range 0 limit |
| cfg_addr1_base | logic | ADDR_WIDTH | Input | Yes | Address range 1 base |
| cfg_addr1_limit | logic | ADDR_WIDTH | Input | Yes | Address range 1 limit |
| cfg_channel_reset | logic | 1 | Input | Yes | Dynamic channel reset request |
| cfg_ctrlrd_max_try | logic | 8 | Input | Yes | Control read max retry count |

##### Status Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| descriptor_engine_idle | logic | 1 | Output | Yes | Descriptor engine idle status |
| program_engine_idle | logic | 1 | Output | Yes | Program engine idle status (tied to 1’b1 - removed) |
| scheduler_idle | logic | 1 | Output | Yes | Scheduler idle status |
| descriptor_credit_counter | logic | 32 | Output | Yes | Current credit counter value |
| fsm_state | logic | 8 | Output | Yes | Current scheduler FSM state |
| scheduler_error | logic | 1 | Output | Yes | Scheduler error flag |
| backpressure_warning | logic | 1 | Output | Yes | Backpressure warning flag |

##### Data Engine Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_valid | logic | 1 | Output | Yes | Data transfer request active |
| data_ready | logic | 1 | Input | Yes | Data engine ready for transfer |
| data_address | logic | ADDR_WIDTH | Output | Yes | Current data address |
| data_length | logic | 32 | Output | Yes | Remaining data length |
| data_type | logic | 2 | Output | Yes | Packet type from descriptor |
| data_eos | logic | 1 | Output | Yes | End of Stream indicator |
| data_transfer_length | logic | 32 | Input | Yes | Actual transfer length completed |
| data_error | logic | 1 | Input | Yes | Data transfer error |
| data_done_strobe | logic | 1 | Input | Yes | Data transfer completed |

##### Address Alignment Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_alignment_info | alignment_info_t | struct | Output | Yes | Pre-calculated alignment information (RAPIDS package type) |
| data_alignment_valid | logic | 1 | Output | Yes | Alignment information valid |
| data_alignment_ready | logic | 1 | Input | Yes | Alignment information ready |
| data_alignment_next | logic | 1 | Input | Yes | Request next alignment calculation |
| data_transfer_phase | transfer_phase_t | enum | Output | Yes | Current transfer phase (RAPIDS package type) |
| data_sequence_complete | logic | 1 | Output | Yes | Transfer sequence complete |

##### Control Read Engine Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlrd_valid | logic | 1 | Output | Yes | Control read request valid |
| ctrlrd_ready | logic | 1 | Input | Yes | Control read request ready |
| ctrlrd_addr | logic | ADDR_WIDTH | Output | Yes | Control read address |
| ctrlrd_data | logic | 32 | Output | Yes | Control read expected data |
| ctrlrd_mask | logic | 32 | Output | Yes | Control read data mask |
| ctrlrd_error | logic | 1 | Input | Yes | Control read error (mismatch or AXI error) |
| ctrlrd_result | logic | 32 | Input | Yes | Control read actual data result |

##### Control Write Engine Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlwr_valid | logic | 1 | Output | Yes | Control write request valid |
| ctrlwr_ready | logic | 1 | Input | Yes | Control write request ready |
| ctrlwr_addr | logic | ADDR_WIDTH | Output | Yes | Control write address |
| ctrlwr_data | logic | 32 | Output | Yes | Control write data |
| ctrlwr_error | logic | 1 | Input | Yes | Control write error (AXI error) |

##### RDA Credit Return Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rda_complete_valid | logic | 1 | Output | Yes | RDA completion notification |
| rda_complete_ready | logic | 1 | Input | Yes | RDA completion ready |
| rda_complete_channel | logic | CHAN_WIDTH | Output | Yes | Channel with RDA completion |

##### Descriptor Engine AXI4 Master Read Interface

NOTE: This interface connects internally to the descriptor engine. External connectivity is provided by scheduler_group_array which arbitrates multiple channels.

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| desc_ar_valid | logic | 1 | Output | Yes | Read address valid |
| desc_ar_ready | logic | 1 | Input | Yes | Read address ready |
| desc_ar_addr | logic | ADDR_WIDTH | Output | Yes | Read address |
| desc_ar_len | logic | 8 | Output | Yes | Burst length |
| desc_ar_size | logic | 3 | Output | Yes | Burst size |
| desc_ar_burst | logic | 2 | Output | Yes | Burst type |
| desc_ar_id | logic | AXI_ID_WIDTH | Output | Yes | Read ID (internal transaction tracking) |
| desc_ar_lock | logic | 1 | Output | Yes | Lock type |
| desc_ar_cache | logic | 4 | Output | Yes | Cache type |
| desc_ar_prot | logic | 3 | Output | Yes | Protection type |
| desc_ar_qos | logic | 4 | Output | Yes | QoS identifier |
| desc_ar_region | logic | 4 | Output | Yes | Region identifier |
| desc_r_valid | logic | 1 | Input | Yes | Read data valid |
| desc_r_ready | logic | 1 | Output | Yes | Read data ready |
| desc_r_data | logic | DATA_WIDTH | Input | Yes | Read data |
| desc_r_resp | logic | 2 | Input | Yes | Read response |
| desc_r_last | logic | 1 | Input | Yes | Read last |
| desc_r_id | logic | AXI_ID_WIDTH | Input | Yes | Read ID |

##### Program Engine AXI4 Master Write Interface (Legacy - Tied Off)

NOTE: The program engine has been removed from scheduler_group. These ports are retained for backwards compatibility but are tied off (all outputs driven to 0, ready signals driven to 1). Control operations are now handled through ctrlrd_* and ctrlwr_* interfaces.

| Signal Name | Type | Width | Direction | Status | Description |
| --- | --- | --- | --- | --- | --- |
| prog_aw_valid | logic | 1 | Output | Tied to 0 | Write address valid (inactive) |
| prog_aw_ready | logic | 1 | Input | Ignored | Write address ready |
| prog_aw_addr | logic | ADDR_WIDTH | Output | Tied to 0 | Write address |
| prog_aw_len | logic | 8 | Output | Tied to 0 | Burst length |
| prog_aw_size | logic | 3 | Output | Tied to 0 | Burst size |
| prog_aw_burst | logic | 2 | Output | Tied to 0 | Burst type |
| prog_aw_id | logic | AXI_ID_WIDTH | Output | Tied to 0 | Write ID |
| prog_aw_lock | logic | 1 | Output | Tied to 0 | Lock type |
| prog_aw_cache | logic | 4 | Output | Tied to 0 | Cache type |
| prog_aw_prot | logic | 3 | Output | Tied to 0 | Protection type |
| prog_aw_qos | logic | 4 | Output | Tied to 0 | QoS identifier |
| prog_aw_region | logic | 4 | Output | Tied to 0 | Region identifier |
| prog_w_valid | logic | 1 | Output | Tied to 0 | Write data valid (inactive) |
| prog_w_ready | logic | 1 | Input | Ignored | Write data ready |
| prog_w_data | logic | 32 | Output | Tied to 0 | Write data |
| prog_w_strb | logic | 4 | Output | Tied to 0 | Write strobes |
| prog_w_last | logic | 1 | Output | Tied to 0 | Write last |
| prog_b_valid | logic | 1 | Input | Ignored | Write response valid |
| prog_b_ready | logic | 1 | Output | Tied to 1 | Write response ready (always ready) |
| prog_b_resp | logic | 2 | Input | Ignored | Write response |
| prog_b_id | logic | AXI_ID_WIDTH | Input | Ignored | Write ID |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Architecture

##### Internal Components

- Descriptor Engine: Handles APB and RDA descriptor processing with AXI read operations

- Scheduler: Main FSM managing descriptor execution, credit management, and control engine sequencing

- Monitor Bus Aggregator: Round-robin aggregation of monitor events from descriptor engine and scheduler

- Control Engine Interfaces: Exposed ctrlrd and ctrlwr ports for external control read/write engines

NOTE: The program engine has been removed and replaced by external control engines: - Control operations are now handled by external ctrlrd_engine and ctrlwr_engine modules - These connect through simplified ctrlrd_* and ctrlwr_* interfaces (not full AXI) - Legacy prog_* AXI ports retained but tied off for backwards compatibility

##### Address Alignment Processing

The scheduler includes a dedicated Address Alignment FSM that runs in parallel with the main scheduler during the SCHED_DESCRIPTOR_ACTIVE state. This FSM pre-calculates all alignment information and provides it to data engines via the alignment bus interface, eliminating alignment calculation overhead from the critical AXI timing paths.

##### Channel Reset Coordination

All components support coordinated channel reset through the cfg_channel_reset input. Each component handles reset gracefully: - Descriptor Engine: Completes AXI read transactions in WAIT_READ state before reset - Scheduler: Completes control operations (ctrlrd/ctrlwr) before reset

Reset completion is indicated through the idle status outputs (descriptor_engine_idle, scheduler_idle). The program_engine_idle output is tied to 1’b1 (always idle) since the program engine has been removed.

##### Control Operation Sequencing

The scheduler coordinates control read and write operations through the external control engines: 1. Descriptor processing completes 2. If ctrlrd needed: Issue control read request and wait for result 3. If ctrlwr needed: Issue control write request and wait for completion 4. Activate data path engines 5. Return to idle state

Control engines are instantiated externally (typically in scheduler_group_array) and arbitrated across multiple channels.

#### Network 2.0 Support

The scheduler group supports the Network 2.0 protocol specification for RDA packet processing, which uses chunk enables instead of the older start/len approach for indicating valid data chunks within each 512-bit packet.

#### Usage Guidelines

##### Channel Reset Sequence

// Example channel reset coordination
logic all_engines_idle = descriptor_engine_idle & program_engine_idle & scheduler_idle;

// Assert reset when needed
if (reset_request && !cfg_channel_reset) begin
    cfg_channel_reset <= 1'b1;
end 
// Deassert reset when all engines are idle
else if (cfg_channel_reset && all_engines_idle) begin
    cfg_channel_reset <= 1'b0;
end

##### Performance Optimization

- Use separate AXI interconnect paths for descriptor read and program write

- Configure alignment bus ready signals appropriately for optimal throughput

- Monitor FSM states and idle signals for debugging and performance analysis

- Utilize pre-calculated alignment information for efficient AXI transfers

##### Monitor Bus Configuration

The wrapper provides unified monitor output with agent ID-based filtering: - Descriptor Engine events: Use agent ID DESC_MON_AGENT_ID - Program Engine events: Use agent ID PROG_MON_AGENT_ID - Scheduler events: Use agent ID SCHED_MON_AGENT_ID

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Scheduler

#### Overview

The Scheduler orchestrates data movement operations by managing descriptor execution, coordinating with data engines, and controlling program sequences. The module implements two sophisticated state machines: a main scheduler FSM for descriptor execution and an address alignment FSM that runs in parallel to pre-calculate optimal transfer parameters for maximum AXI performance.

scheduler

scheduler

#### Key Features

- Dual State Machine Architecture: Main scheduler FSM plus parallel address alignment FSM

- Descriptor-Driven Operation: Processes descriptors from descriptor engine with full stream control

- Address Alignment Pre-calculation: Dedicated FSM provides alignment information before AXI operations

- Credit Management: Atomic credit operations with early warning thresholds

- Stream Boundary Support: Complete EOS processing with sequence completion tracking

- Program Sequencing: Sequential program engine coordination for post-processing

- Channel Reset Support: Graceful channel shutdown with proper completion signaling

- Monitor Integration: Comprehensive event reporting for system visibility

#### Interface Specification

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| CHANNEL_ID | 0 | Static channel identifier for this scheduler instance |
| NUM_CHANNELS | 32 | Total number of channels in system |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Width of channel address fields |
| ADDR_WIDTH | 64 | Address width for data operations |
| DATA_WIDTH | 512 | Descriptor packet width |
| CREDIT_WIDTH | 8 | Credit counter width |
| TIMEOUT_CYCLES | 1000 | Data transfer timeout threshold |
| EARLY_WARNING_THRESHOLD | 4 | Credit warning threshold |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_idle_mode | logic | 1 | Input | Yes | Force scheduler to idle state |
| cfg_channel_wait | logic | 1 | Input | Yes | Wait for channel enable |
| cfg_channel_enable | logic | 1 | Input | Yes | Channel enable control |
| cfg_use_credit | logic | 1 | Input | Yes | Enable credit-based flow control |
| cfg_initial_credit | logic | CREDIT_WIDTH | Input | Yes | Initial credit value (exponential encoding) |
| credit_increment | logic | 1 | Input | Yes | Credit increment request |
| cfg_channel_reset | logic | 1 | Input | Yes | Dynamic channel reset request |

##### Status Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| scheduler_idle | logic | 1 | Output | Yes | Scheduler idle status indicator |
| fsm_state | scheduler_state_t | 3 | Output | Yes | Current main FSM state |
| descriptor_credit_counter | logic | CREDIT_WIDTH | Output | Yes | Current credit count |
| scheduler_error | logic | 1 | Output | Yes | Scheduler error status |
| backpressure_warning | logic | 1 | Output | Yes | Credit or timeout warning |

##### Descriptor Engine Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| descriptor_valid | logic | 1 | Input | Yes | Descriptor available from engine |
| descriptor_ready | logic | 1 | Output | Yes | Ready to accept descriptor |
| descriptor_packet | logic | DATA_WIDTH | Input | Yes | Descriptor packet data |
| descriptor_same | logic | 1 | Input | Yes | Same descriptor flag |
| descriptor_error | logic | 1 | Input | Yes | Descriptor error status |
| descriptor_is_rda | logic | 1 | Input | Yes | RDA packet indicator |
| descriptor_rda_channel | logic | CHAN_WIDTH | Input | Yes | RDA channel identifier |
| descriptor_eos | logic | 1 | Input | Yes | End of Stream from descriptor |
| descriptor_type | logic | 2 | Input | Yes | Packet type |

##### Data Engine Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_valid | logic | 1 | Output | Yes | Data transfer request active |
| data_ready | logic | 1 | Input | Yes | Data engine ready for transfer |
| data_address | logic | ADDR_WIDTH | Output | Yes | Current data address |
| data_length | logic | 32 | Output | Yes | Remaining data length |
| data_type | logic | 2 | Output | Yes | Packet type from descriptor |
| data_eos | logic | 1 | Output | Yes | End of Stream indicator |
| data_transfer_length | logic | 32 | Input | Yes | Actual transfer length completed |
| data_error | logic | 1 | Input | Yes | Data transfer error |
| data_done_strobe | logic | 1 | Input | Yes | Data transfer completed |

##### Address Alignment Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_alignment_info | alignment_info_t | Variable | Output | Yes | Pre-calculated alignment information |
| data_alignment_valid | logic | 1 | Output | Yes | Alignment information valid |
| data_alignment_ready | logic | 1 | Input | Yes | Alignment information ready |
| data_alignment_next | logic | 1 | Input | Yes | Request next alignment calculation |
| data_transfer_phase | transfer_phase_t | 3 | Output | Yes | Current transfer phase |
| data_sequence_complete | logic | 1 | Input | Yes | Transfer sequence complete |

##### Ctrlrd Engine Interface (Control Read Operations)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlrd_valid | logic | 1 | Output | Yes | Control read operation request |
| ctrlrd_ready | logic | 1 | Input | Yes | Control read match/completion (asserts when read data matches expected value) |
| ctrlrd_addr | logic | ADDR_WIDTH | Output | Yes | Control read address |
| ctrlrd_data | logic | 32 | Output | Yes | Control read expected/comparison data |
| ctrlrd_mask | logic | 32 | Output | Yes | Control read mask/flags |
| ctrlrd_error | logic | 1 | Input | Yes | Control read operation error (valid when ctrlrd_valid && ctrlrd_ready) |
| ctrlrd_result | logic | 32 | Input | Yes | Control read result data (actual value read) |

Ctrlrd Ready Behavior: - ctrlrd_ready asserts only when the read response matches the expected value (ctrlrd_data) - The ctrlrd engine performs retry operations until match or max retries exceeded - ctrlrd_error is valid when both ctrlrd_valid and ctrlrd_ready are asserted - Error conditions: AXI response error OR max retry count exceeded without match

Operation Flow: 1. Scheduler asserts ctrlrd_valid with address, expected data, and mask 2. Ctrlrd engine performs AXI read and compares result with expected data 3. If match: ctrlrd_ready asserts (proceed to data operations) 4. If no match: Retry read up to cfg_ctrlrd_max_try times 5. If max retries exceeded: ctrlrd_ready and ctrlrd_error assert together (enter error state)

Note: Ctrlrd operations execute BEFORE data operations, enabling pre-descriptor flag reads or synchronization checks with configurable retry mechanisms.

##### Ctrlwr Engine Interface (Control Write Operations)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlwr_valid | logic | 1 | Output | Yes | Control write operation request |
| ctrlwr_ready | logic | 1 | Input | Yes | Control write operation complete |
| ctrlwr_addr | logic | ADDR_WIDTH | Output | Yes | Control write address (ctrlwr0 or ctrlwr1) |
| ctrlwr_data | logic | 32 | Output | Yes | Control write data (ctrlwr0 or ctrlwr1) |
| ctrlwr_error | logic | 1 | Input | Yes | Control write operation error |

Note: Ctrlwr operations execute AFTER data operations complete, enabling post-descriptor notifications or cleanup operations.

##### RDA Completion Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rda_complete_valid | logic | 1 | Output | Yes | RDA completion notification |
| rda_complete_ready | logic | 1 | Input | Yes | RDA completion ready |
| rda_complete_channel | logic | CHAN_WIDTH | Output | Yes | RDA completion channel |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Dual State Machine Architecture

##### Main Scheduler FSM

Address Alignment FSM

States: - SCHED_IDLE: Ready for new descriptor, all operations complete - SCHED_WAIT_FOR_CONTROL: Wait for channel enable and control signals - SCHED_ISSUE_CTRLRD: Issue control read operation (pre-descriptor, conditional) - SCHED_DESCRIPTOR_ACTIVE: Execute data transfer operations (alignment FSM runs in parallel) - SCHED_ISSUE_CTRLWR0: Issue first control write operation (post-descriptor, conditional) - SCHED_ISSUE_CTRLWR1: Issue second control write operation (post-descriptor, conditional) - SCHED_ERROR: Handle error conditions with sticky error flags

Operation Sequence:

IDLE -> WAIT_FOR_CONTROL -> ISSUE_CTRLRD (if needed) ->
  DESCRIPTOR_ACTIVE -> ISSUE_CTRLWR0 (if needed) -> ISSUE_CTRLWR1 (if needed) -> IDLE

##### Address Alignment FSM (Parallel Operation)

Address Alignment FSM

States: - ALIGN_IDLE: Ready for new alignment calculation - ANALYZE_ADDRESS: Single-cycle address analysis and strategy selection - CALC_FIRST_TRANSFER: Generate alignment transfer parameters - CALC_STREAMING: Calculate optimal streaming burst parameters - CALC_FINAL_TRANSFER: Handle final partial transfer calculations - ALIGNMENT_COMPLETE: Provide complete alignment information to engines - ALIGNMENT_ERROR: Handle invalid alignment scenarios

#### Address Alignment System

##### Parallel FSM Operation

The address alignment FSM runs in parallel with the main scheduler during the SCHED_DESCRIPTOR_ACTIVE state:

// Alignment FSM trigger condition
if ((r_current_state == SCHED_DESCRIPTOR_ACTIVE) && (r_data_length > 32'h0)) begin
    w_alignment_next_state = ANALYZE_ADDRESS;
end

##### Alignment Information Structure

typedef struct packed {
    logic                    is_aligned;           // Already 64-byte aligned
    logic [5:0]              addr_offset;          // Address offset within boundary
    logic [31:0]             first_transfer_bytes; // Bytes in alignment transfer
    logic [NUM_CHUNKS-1:0]   first_chunk_enables;  // Chunk pattern for alignment
    logic [7:0]              optimal_burst_len;    // Optimal burst after alignment
    logic [31:0]             final_transfer_bytes; // Bytes in final transfer
    logic [NUM_CHUNKS-1:0]   final_chunk_enables;  // Final chunk pattern
    logic [3:0]              total_transfers;      // Pre-calculated total count
    alignment_strategy_t     alignment_strategy;   // Strategy selection
} alignment_info_t;

##### Transfer Phase Management

typedef enum logic [2:0] {
    PHASE_IDLE      = 3'h0,  // No active transfer
    PHASE_ALIGNMENT = 3'h1,  // Initial alignment transfer
    PHASE_STREAMING = 3'h2,  // Optimal aligned streaming
    PHASE_FINAL     = 3'h3,  // Final partial transfer
    PHASE_COMPLETE  = 3'h4   // Transfer sequence complete
} transfer_phase_t;

#### Descriptor Processing

##### Descriptor Packet Format (512 bits)

| Bit Range | Field Name | Width | Description |
| --- | --- | --- | --- |
| 511:480 | Reserved | 32 bits | Reserved for future use |
| 479:448 | ctrlwr1_data | 32 bits | Control Write 1 data value |
| 447:384 | ctrlwr1_addr | 64 bits | Control Write 1 address |
| 383:352 | ctrlwr0_data | 32 bits | Control Write 0 data value |
| 351:288 | ctrlwr0_addr | 64 bits | Control Write 0 address |
| 287:256 | ctrlrd_mask | 32 bits | Control Read mask/flags |
| 255:224 | ctrlrd_data | 32 bits | Control Read data value |
| 223:160 | ctrlrd_addr | 64 bits | Control Read address |
| 159:128 | next_descriptor_addr[63:32] | 32 bits | Next descriptor pointer (upper) |
| 127:96 | next_descriptor_addr[31:0] | 32 bits | Next descriptor pointer (lower) |
| 95:64 | data_addr[63:32] | 32 bits | Data operation address (upper) |
| 63:32 | data_addr[31:0] | 32 bits | Data operation address (lower) |
| 31:0 | data_length | 32 bits | Data transfer length (bytes) |

##### Descriptor Field Details

1. Data Operation Fields (bits 0-95): - data_length [31:0] - Transfer length in bytes - data_addr [95:32] - 64-bit memory address for data operation

2. Next Descriptor Pointer (bits 96-159): - next_descriptor_addr [159:96] - 64-bit address of next descriptor in linked list

3. Control Read Fields (bits 160-287): - ctrlrd_addr [223:160] - 64-bit address for pre-descriptor control read operation - ctrlrd_data [255:224] - 32-bit data value for control read (comparison/expected value) - ctrlrd_mask [287:256] - 32-bit mask/flags for control read operation

4. Control Write Fields (bits 288-479): - ctrlwr0_addr [351:288] - 64-bit address for first post-descriptor control write - ctrlwr0_data [383:352] - 32-bit data for first control write - ctrlwr1_addr [447:384] - 64-bit address for second post-descriptor control write - ctrlwr1_data [479:448] - 32-bit data for second control write

Operation Sequence: 1. Control Read (ctrlrd): Executed FIRST, before data operations (if ctrlrd_addr != 0) 2. Data Operation: Memory transfer operation (source/sink data path) 3. Control Write 0 (ctrlwr0): Executed after data completion (if ctrlwr0_addr != 0) 4. Control Write 1 (ctrlwr1): Executed after ctrlwr0 completion (if ctrlwr1_addr != 0)

Null Address Handling: Setting any control address field to 64’h0 skips that operation.

##### Descriptor Field Extraction

// Descriptor unpacking in SCHED_IDLE state
if ((r_current_state == SCHED_IDLE) && descriptor_valid && descriptor_ready) begin
    // Extract descriptor fields from 512-bit packet

    // Data operation fields (bits 0-95)
    r_data_length <= descriptor_packet[31:0];
    r_data_addr <= {descriptor_packet[95:64], descriptor_packet[63:32]};

    // Next descriptor pointer (bits 96-159)
    r_next_descriptor_addr <= {descriptor_packet[159:128], descriptor_packet[127:96]};

    // Control Read fields (bits 160-287)
    r_ctrlrd_addr <= {descriptor_packet[223:192], descriptor_packet[191:160]};
    r_ctrlrd_data <= descriptor_packet[255:224];
    r_ctrlrd_mask <= descriptor_packet[287:256];

    // Control Write 0 fields (bits 288-383)
    r_ctrlwr0_addr <= {descriptor_packet[351:320], descriptor_packet[319:288]};
    r_ctrlwr0_data <= descriptor_packet[383:352];

    // Control Write 1 fields (bits 384-479)
    r_ctrlwr1_addr <= {descriptor_packet[447:416], descriptor_packet[415:384]};
    r_ctrlwr1_data <= descriptor_packet[479:448];

    // Companion signals
    r_data_type <= descriptor_type;
    r_packet_eos_received <= descriptor_eos;

    // Decrement credit on descriptor acceptance
    if (cfg_use_credit && (r_descriptor_credit_counter > 0)) begin
        r_descriptor_credit_counter <= r_descriptor_credit_counter - 1;
    end
end

##### Companion Control Signals

In addition to the 512-bit descriptor_packet, these companion signals provide metadata:

| Signal Name | Width | Description |
| --- | --- | --- |
| descriptor_type | 2 bits | Descriptor type (from descriptor engine) |
| descriptor_same | 1 bit | Same descriptor indicator |
| descriptor_error | 1 bit | Error flag from descriptor engine |
| descriptor_is_rda | 1 bit | RDA packet indicator |
| descriptor_rda_channel | CHAN_WIDTH | RDA channel number |
| descriptor_eos | 1 bit | End of stream marker |
| descriptor_eol | 1 bit | End of line marker |
| descriptor_eod | 1 bit | End of data marker |

Note: Stream boundary signals (EOS/EOL/EOD) are provided as companion signals from the descriptor engine, not embedded in the descriptor packet itself.

#### Credit Management

##### Exponential Credit Encoding

The scheduler implements exponential credit encoding for the cfg_initial_credit configuration input, providing a wide range of credit values with a compact 4-bit configuration:

| cfg_initial_credit | Binary | Actual Credits | Description |
| --- | --- | --- | --- |
| 0 | 4’b0000 | 1 | Minimum credits (2^0) |
| 1 | 4’b0001 | 2 | Low credits (2^1) |
| 2 | 4’b0010 | 4 | (2^2) |
| 3 | 4’b0011 | 8 | (2^3) |
| 4 | 4’b0100 | 16 | (2^4) |
| 5 | 4’b0101 | 32 | (2^5) |
| 6 | 4’b0110 | 64 | (2^6) |
| 7 | 4’b0111 | 128 | (2^7) |
| 8 | 4’b1000 | 256 | (2^8) |
| 9 | 4’b1001 | 512 | (2^9) |
| 10 | 4’b1010 | 1024 | (2^10) |
| 11 | 4’b1011 | 2048 | (2^11) |
| 12 | 4’b1100 | 4096 | (2^12) |
| 13 | 4’b1101 | 8192 | (2^13) |
| 14 | 4’b1110 | 16384 | Maximum credits (2^14) |
| 15 | 4’b1111 | 0 | DISABLED (no credits - blocks all operations) |

Encoding Rationale: Exponential encoding allows compact 4-bit configuration to represent a wide range from 1 to 16384 credits, plus a special “disabled” mode (15 = 0 credits). This enables fine-grained control for low-traffic scenarios (1-8 credits) while supporting high-throughput operations (256-16384 credits) without requiring a wide configuration bus. Setting cfg=15 disables the credit system by initializing the counter to 0, effectively blocking all descriptor processing when credit mode is enabled.

##### Credit Initialization

// Reset initialization with exponential encoding
always_ff @(posedge clk) begin
    if (!rst_n) begin
        // Exponential credit encoding:
        //   0->1, 1->2, 2->4, 3->8, ..., 14->16384 (exponential: 2^n)
        //   15->0 (special case: DISABLED - no credits, blocks all operations)
        r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'h00000000 :
                                      (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                                      (32'h1 << cfg_initial_credit);
    end
end

##### Credit Operations

// Credit state tracking
logic [31:0] r_descriptor_credit_counter;  // 32-bit to hold decoded value
logic w_credit_available;
logic w_credit_warning;

// Credit availability check
assign w_credit_available = cfg_use_credit ?
                           (r_descriptor_credit_counter > 0) : 1'b1;

// Early warning threshold (linear comparison against threshold)
assign w_credit_warning = cfg_use_credit ?
                         (r_descriptor_credit_counter <= EARLY_WARNING_THRESHOLD) : 1'b0;

##### Credit Update Logic

// Credit decrement on descriptor acceptance (linear operation on decoded value)
if (descriptor_valid && descriptor_ready && cfg_use_credit) begin
    r_descriptor_credit_counter <= r_descriptor_credit_counter - 1;
end

// Credit increment on external request (linear operation on decoded value)
if (credit_increment && cfg_use_credit) begin
    r_descriptor_credit_counter <= r_descriptor_credit_counter + 1;
end

Important: The exponential encoding applies only to initialization from cfg_initial_credit. Once initialized, the credit counter operates as a standard linear counter (increment by 1, decrement by 1).

#### Control Operation Sequencing

##### Control Read Operations (Pre-Descriptor)

The scheduler issues control read operations BEFORE data operations when ctrlrd_addr is non-zero:

// Control read needed check
logic w_ctrlrd_needed = (r_ctrlrd_addr != 64'h0);

// State transitions from WAIT_FOR_CONTROL
SCHED_WAIT_FOR_CONTROL: begin
    if (!cfg_channel_wait && cfg_channel_enable) begin
        if (w_ctrlrd_needed) begin
            w_next_state = SCHED_ISSUE_CTRLRD;  // Issue control read first
        end else begin
            w_next_state = SCHED_DESCRIPTOR_ACTIVE;  // Skip to data operations
        end
    end
end

// Control read state with retry mechanism
SCHED_ISSUE_CTRLRD: begin
    if (ctrlrd_ready && !ctrlrd_error) begin
        // Read matched expected value - proceed to data operations
        w_next_state = SCHED_DESCRIPTOR_ACTIVE;
    end else if (ctrlrd_ready && ctrlrd_error) begin
        // Max retries exceeded or AXI error - enter error state
        w_next_state = SCHED_ERROR;
    end
    // Else: Stay in SCHED_ISSUE_CTRLRD while ctrlrd engine retries
end

// Control read output signals
assign ctrlrd_valid = (r_current_state == SCHED_ISSUE_CTRLRD);
assign ctrlrd_addr = r_ctrlrd_addr;
assign ctrlrd_data = r_ctrlrd_data;  // Expected/comparison data
assign ctrlrd_mask = r_ctrlrd_mask;  // Mask/flags for operation

Ctrlrd Operation Details: - Match Success: ctrlrd_ready asserts when read data matches expected value -> Proceed to DESCRIPTOR_ACTIVE - Retry Loop: Ctrlrd engine automatically retries reads up to cfg_ctrlrd_max_try times - Timeout/Error: ctrlrd_ready && ctrlrd_error indicates failure -> Enter SCHED_ERROR state - Use Cases: Flag polling, synchronization checks, pre-condition validation

##### Control Write Operations (Post-Descriptor)

The scheduler issues control write operations AFTER data operations complete:

// Control write needed checks
logic w_ctrlwr0_needed = (r_ctrlwr0_addr != 64'h0);
logic w_ctrlwr1_needed = (r_ctrlwr1_addr != 64'h0);

// State transitions for control write operations
case (r_current_state)
    SCHED_DESCRIPTOR_ACTIVE: begin
        if (w_descriptor_complete) begin
            // After data operations, issue control writes if needed
            if (w_ctrlwr0_needed) begin
                w_next_state = SCHED_ISSUE_CTRLWR0;
            end else if (w_ctrlwr1_needed) begin
                w_next_state = SCHED_ISSUE_CTRLWR1;
            end else begin
                w_next_state = SCHED_IDLE;
            end
        end
    end

    SCHED_ISSUE_CTRLWR0: begin
        if (ctrlwr_ready) begin
            // After ctrlwr0 completes, issue ctrlwr1 if needed
            if (w_ctrlwr1_needed) begin
                w_next_state = SCHED_ISSUE_CTRLWR1;
            end else begin
                w_next_state = SCHED_IDLE;
            end
        end
    end

    SCHED_ISSUE_CTRLWR1: begin
        if (ctrlwr_ready) begin
            w_next_state = SCHED_IDLE;
        end
    end
endcase

// Control write output multiplexing
assign ctrlwr_valid = (r_current_state == SCHED_ISSUE_CTRLWR0) ||
                     (r_current_state == SCHED_ISSUE_CTRLWR1);
assign ctrlwr_addr = (r_current_state == SCHED_ISSUE_CTRLWR0) ? r_ctrlwr0_addr : r_ctrlwr1_addr;
assign ctrlwr_data = (r_current_state == SCHED_ISSUE_CTRLWR0) ? r_ctrlwr0_data : r_ctrlwr1_data;

##### Null Address Handling

All control operations support conditional execution via null address checking:

// Null address = skip operation
// ctrlrd_addr = 64'h0 -> Skip control read operation
// ctrlwr0_addr = 64'h0 -> Skip control write 0 operation
// ctrlwr1_addr = 64'h0 -> Skip control write 1 operation

This enables descriptor-driven conditional control operations without additional configuration.

#### Channel Reset Coordination

##### Graceful Reset Handling

// Channel reset coordination
assign w_safe_to_reset = (r_current_state == SCHED_IDLE) && 
                        (r_alignment_state == ALIGN_IDLE) &&
                        w_no_pending_operations &&
                        !r_channel_reset_active;

assign scheduler_idle = w_safe_to_reset && !r_channel_reset_active;

##### Reset Behavior

- Block New Operations: Stop accepting descriptors during reset

- Complete Active Operations: Finish data transfers and program operations

- Reset State Machines: Both main and alignment FSMs return to idle

- Signal Completion: Assert scheduler_idle when reset complete

#### Timeout Detection

##### Timeout Monitoring

// Timeout counter for stuck operations
logic [31:0] r_timeout_counter;
logic w_timeout_expired;

assign w_timeout_expired = (r_timeout_counter >= TIMEOUT_CYCLES);

// Timeout counter management
always_ff @(posedge clk) begin
    if (!rst_n || (r_current_state == SCHED_IDLE)) begin
        r_timeout_counter <= 32'h0;
    end else if (r_current_state == SCHED_DESCRIPTOR_ACTIVE) begin
        r_timeout_counter <= r_timeout_counter + 1;
    end
end

#### Error Handling

##### Sticky Error Flags

The scheduler implements sticky error registers to capture transient error signals from engines and hold them until the FSM can process the error condition. This ensures errors are never missed due to timing mismatches between engine error assertion and FSM state checking.

Sticky Error Registers: - r_data_error_sticky - Captures errors from data engine - r_ctrlrd_error_sticky - Captures errors from control read engine - r_ctrlwr_error_sticky - Captures errors from control write engine

Purpose: Engine error signals (e.g., ctrlwr_error) may assert synchronously with ready signals during handshakes. If the FSM transitions to a new state before checking the error, the transient error signal could be lost. Sticky registers hold errors until the FSM explicitly processes them.

// Persistent error tracking with sticky registers
logic r_data_error_sticky;
logic r_ctrlrd_error_sticky;
logic r_ctrlwr_error_sticky;

// Sticky error flag management
always_ff @(posedge clk) begin
    if (!rst_n) begin
        r_data_error_sticky <= 1'b0;
        r_ctrlrd_error_sticky <= 1'b0;
        r_ctrlwr_error_sticky <= 1'b0;
    end else begin
        // Capture errors when they occur
        if (data_error) r_data_error_sticky <= 1'b1;
        if (ctrlrd_error) r_ctrlrd_error_sticky <= 1'b1;
        if (ctrlwr_error) r_ctrlwr_error_sticky <= 1'b1;

        // Clear sticky errors only when transitioning FROM error state TO idle
        // This ensures errors are held long enough to trigger ERROR state
        if (r_current_state == SCHED_ERROR && w_next_state == SCHED_IDLE) begin
            r_data_error_sticky <= 1'b0;
            r_ctrlrd_error_sticky <= 1'b0;
            r_ctrlwr_error_sticky <= 1'b0;
        end
    end
end

Critical Timing: Sticky errors are cleared only during ERROR->IDLE transition, NOT on IDLE state entry. This prevents the following timing bug:

[FAIL] Wrong Timing (Bug):

Cycle N:   FSM in SCHED_ISSUE_CTRLWR0, ctrlwr_error=1 & ctrlwr_ready=1
           FSM transitions to SCHED_IDLE (missed error check!)
           r_ctrlwr_error_sticky <= 1 (takes effect next cycle)
Cycle N+1: FSM in SCHED_IDLE, r_ctrlwr_error_sticky=1
           Clear on IDLE entry -> r_ctrlwr_error_sticky <= 0
           Error lost before detection logic sees it!

[PASS] Correct Timing (Fixed):

Cycle N:   FSM in SCHED_ISSUE_CTRLWR0, ctrlwr_error=1 & ctrlwr_ready=1
           General error detection sees ctrlwr_error=1 OR r_ctrlwr_error_sticky=1
           FSM transitions to SCHED_ERROR (error caught!)
Cycle N+1: FSM in SCHED_ERROR
           Process error state...
Cycle N+M: FSM transitions ERROR->IDLE
           Clear sticky errors now that error has been processed

##### Error Detection Logic

General Error Detection (applies in all states):

// Check transient errors OR sticky errors
if (data_error || ctrlrd_error || ctrlwr_error || descriptor_error ||
    w_timeout_expired || r_data_error_sticky || r_ctrlrd_error_sticky || r_ctrlwr_error_sticky) begin
    w_next_state = SCHED_ERROR;
end

State-Specific Error Checking (for ctrlrd):

// SCHED_ISSUE_CTRLRD state has explicit error checking
SCHED_ISSUE_CTRLRD: begin
    if (ctrlrd_ready && !ctrlrd_error) begin
        w_next_state = SCHED_DESCRIPTOR_ACTIVE;  // Success
    end else if (ctrlrd_ready && ctrlrd_error) begin
        w_next_state = SCHED_ERROR;  // Error detected
    end
end

Note: Control write states (SCHED_ISSUE_CTRLWR0/CTRLWR1) rely on general error detection logic, not state-specific checks. The sticky error registers ensure ctrlwr errors are captured even if the FSM transitions before checking.

##### Error Recovery

// Error state exit conditions - check both transient and sticky errors
logic w_all_errors_clear = !data_error && !ctrlrd_error && !ctrlwr_error &&
                          !descriptor_error && !w_timeout_expired &&
                          !r_data_error_sticky && !r_ctrlrd_error_sticky && !r_ctrlwr_error_sticky;

// Error state recovery to IDLE
if ((r_current_state == SCHED_ERROR) && w_all_errors_clear && w_credit_available) begin
    w_next_state = SCHED_IDLE;  // Sticky errors cleared during this transition
end

Error Recovery Sequence: 1. Error detected (transient or sticky) -> Enter SCHED_ERROR state 2. Wait in SCHED_ERROR until all error sources clear 3. Transition SCHED_ERROR -> SCHED_IDLE 4. Sticky error registers cleared during this transition 5. Ready for new descriptor processing

#### Performance Benefits of Address Alignment FSM

##### Hidden Latency Calculation

The parallel address alignment FSM provides significant performance benefits:

- Pre-calculation During Descriptor Processing: Alignment analysis occurs during the non-critical descriptor processing phase

- Immediate Availability: AXI engines receive pre-calculated alignment information immediately when needed

- No Critical Path Impact: No alignment calculation overhead in AXI transaction timing

- Optimal Transfer Planning: Complete transfer sequence pre-calculated with optimized burst patterns

##### Transfer Strategy Selection

// Transfer strategy enumeration
typedef enum logic [2:0] {
    STRATEGY_SINGLE    = 3'h0,  // Single aligned transfer
    STRATEGY_ALIGNMENT = 3'h1,  // Alignment + streaming
    STRATEGY_STREAMING = 3'h2,  // Pure streaming transfers
    STRATEGY_FINAL     = 3'h3,  // Alignment + streaming + final
    STRATEGY_PRECISION = 3'h4   // Precision chunk-level transfers
} alignment_strategy_t;

##### Chunk Enable Generation

// First transfer chunk enable calculation
function logic [NUM_CHUNKS-1:0] calc_first_chunk_enables(
    input logic [5:0] addr_offset,
    input logic [31:0] transfer_bytes
);
    logic [3:0] start_chunk = addr_offset[5:2];  // Starting chunk index
    logic [3:0] num_chunks = (transfer_bytes + 3) >> 2;  // Number of chunks needed
    logic [NUM_CHUNKS-1:0] mask = (1 << num_chunks) - 1;  // Create mask
    return mask << start_chunk;  // Shift to correct position
endfunction

// Final transfer chunk enable calculation  
function logic [NUM_CHUNKS-1:0] calc_final_chunk_enables(
    input logic [31:0] final_bytes
);
    logic [3:0] num_chunks = (final_bytes + 3) >> 2;  // Number of chunks needed
    return (1 << num_chunks) - 1;  // Mask from bit 0
endfunction

#### Monitor Bus Events

The scheduler generates comprehensive monitor events:

##### Error Events

- Timeout Error: Data transfer timeout detection

- Credit Exhausted: Credit underflow condition

- Descriptor Error: Invalid descriptor content

- Program Error: Program operation failure

- Alignment Error: Invalid address alignment parameters

##### Performance Events

- Descriptor Processing: Descriptor execution start/completion

- Credit Warning: Early warning threshold reached

- Transfer Phase: Transfer phase transitions (alignment/streaming/final)

- Program Sequence: Program operation sequence tracking

- Alignment Calculation: Address alignment FSM operation timing

##### Completion Events

- Data Transfer Complete: Data operation completion

- Program Complete: Program operation completion

- Stream Boundary: EOS processing completion

- Channel Reset: Channel reset sequence completion

- RDA Processing: RDA packet processing completion

#### Usage Guidelines

##### Address Alignment Optimization

The address alignment system provides optimal AXI performance:

// Example alignment information usage in AXI engine
if (data_alignment_valid) begin
    case (data_transfer_phase)
        PHASE_ALIGNMENT: begin
            axi_addr <= data_alignment_info.aligned_addr;
            axi_len <= data_alignment_info.first_burst_len;
            chunk_enables <= data_alignment_info.first_chunk_enables;
        end
        PHASE_STREAMING: begin
            axi_addr <= aligned_streaming_addr;
            axi_len <= data_alignment_info.optimal_burst_len;
            chunk_enables <= 16'hFFFF;  // All chunks valid
        end
        PHASE_FINAL: begin
            axi_addr <= final_transfer_addr;
            axi_len <= data_alignment_info.final_burst_len;
            chunk_enables <= data_alignment_info.final_chunk_enables;
        end
    endcase
end

##### Performance Monitoring

Monitor key performance indicators: - Credit utilization and warning frequency - Timeout occurrence and duration - Address alignment efficiency - Program sequence timing - Transfer phase distribution

##### Error Recovery

The scheduler provides comprehensive error handling: - Monitor timeout conditions for stuck transfers - Track credit exhaustion for flow control issues - Detect alignment calculation errors - Coordinate error recovery across all interfaces - Use sticky error flags for persistent fault tracking

##### Channel Reset Usage

// Example channel reset coordination
logic reset_request;
logic reset_complete;

assign reset_complete = scheduler_idle;

always_ff @(posedge clk) begin
    if (reset_request && !cfg_channel_reset) begin
        cfg_channel_reset <= 1'b1;
    end else if (cfg_channel_reset && reset_complete) begin
        cfg_channel_reset <= 1'b0;
        // Channel is now safely reset and ready for operation
    end
end

This dual-FSM scheduler architecture provides optimal performance through pre-calculated address alignment while maintaining comprehensive control over descriptor execution, credit management, and program sequencing operations.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Descriptor Engine

#### Overview

The Descriptor Engine manages descriptor fetching and buffering operations with sophisticated dual-path processing for APB and CDA (Compute Direct Access packet) requests. The module implements a six-state state machine that handles descriptor address management, AXI read operations, descriptor parsing, and prefetch optimization with comprehensive stream boundary support and channel reset coordination.

Delta Network Integration: - CDA Interface: The CDA (Compute Direct Access) packet interface receives CDA packets from HIVE-C via the Delta Network - Inband Descriptor Injection: HIVE-C injects descriptors as CDA packets (TUSER=2’b01) through Delta Network virtual tile 16 - Priority Processing: CDA packets processed with higher priority than APB requests for low-latency descriptor delivery - Packet Type Validation: CDA packets validated for TUSER=2’b01 before descriptor extraction

descriptor engine

descriptor engine

#### Key Features

- Dual-Path Processing: Handles both APB programming interface and CDA packet interface

- Six-State State Machine: Comprehensive descriptor lifecycle management

- Stream Boundary Support: Complete EOS/EOL/EOD field extraction and propagation

- AXI Read Coordination: Shared AXI interface with channel ID-based response routing

- 4-Deep Descriptor FIFO: Maintains descriptor flow for continuous operation

- Priority Handling: CDA packets prioritized over APB requests

- Channel Reset Support: Graceful shutdown with proper AXI transaction completion

- Monitor Integration: Rich monitor events for descriptor processing visibility

#### Module Interface

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| CHANNEL_ID | 0 | Static channel identifier for this engine instance |
| NUM_CHANNELS | 32 | Total number of channels in system |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Width of channel address fields |
| ADDR_WIDTH | 64 | Address width for AXI transactions |
| DATA_WIDTH | 512 | Descriptor packet width |
| AXI_ID_WIDTH | 8 | AXI transaction ID width |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### APB Programming Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| apb_valid | logic | 1 | Input | Yes | APB request valid |
| apb_ready | logic | 1 | Output | Yes | APB request ready |
| apb_addr | logic | ADDR_WIDTH | Input | Yes | Descriptor address |

##### CDA Packet Interface (CDA Packets from HIVE-C via Delta Network)

Note: CDA (Compute Direct Access) interface receives CDA packets with TUSER=2’b01 from HIVE-C routed through Delta Network to RAPIDS virtual tile 16.

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cda_valid | logic | 1 | Input | Yes | CDA packet valid (from HIVE-C) |
| cda_ready | logic | 1 | Output | Yes | Ready to accept CDA packet |
| cda_packet | logic | DATA_WIDTH | Input | Yes | CDA packet data (256-bit descriptor) |
| cda_channel | logic | CHAN_WIDTH | Input | Yes | Channel identifier from CDA packet |

##### Scheduler Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| descriptor_valid | logic | 1 | Output | Yes | Descriptor available |
| descriptor_ready | logic | 1 | Input | Yes | Ready to accept descriptor |
| descriptor_packet | logic | DATA_WIDTH | Output | Yes | Descriptor data |
| descriptor_same | logic | 1 | Output | Yes | Same descriptor flag |
| descriptor_error | logic | 1 | Output | Yes | Descriptor error |
| descriptor_is_cda | logic | 1 | Output | Yes | CDA packet indicator |
| descriptor_cda_channel | logic | CHAN_WIDTH | Output | Yes | CDA channel identifier |
| descriptor_eos | logic | 1 | Output | Yes | End of Stream |
| descriptor_eol | logic | 1 | Output | Yes | End of Line |
| descriptor_eod | logic | 1 | Output | Yes | End of Data |
| descriptor_type | logic | 2 | Output | Yes | Packet type |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_addr0 | logic | ADDR_WIDTH | Input | Yes | Address range 0 base |
| cfg_addr1 | logic | ADDR_WIDTH | Input | Yes | Address range 1 base |
| cfg_channel_reset | logic | 1 | Input | Yes | Dynamic channel reset request |

##### Status Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| descriptor_engine_idle | logic | 1 | Output | Yes | Engine idle status indicator |

##### Shared AXI4 Master Read Interface (512-bit)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ar_valid | logic | 1 | Output | Yes | Read address valid |
| ar_ready | logic | 1 | Input | Yes | Read address ready (arbitrated) |
| ar_addr | logic | ADDR_WIDTH | Output | Yes | Read address |
| ar_len | logic | 8 | Output | Yes | Burst length - 1 (always 0) |
| ar_size | logic | 3 | Output | Yes | Transfer size (3’b110 for 64 bytes) |
| ar_burst | logic | 2 | Output | Yes | Burst type (2’b01 INCR) |
| ar_id | logic | AXI_ID_WIDTH | Output | Yes | Transaction ID (channel-based) |
| ar_lock | logic | 1 | Output | Yes | Lock type (always 0) |
| ar_cache | logic | 4 | Output | Yes | Cache attributes |
| ar_prot | logic | 3 | Output | Yes | Protection attributes |
| ar_qos | logic | 4 | Output | Yes | Quality of service |
| ar_region | logic | 4 | Output | Yes | Region identifier |
| r_valid | logic | 1 | Input | Yes | Read data valid |
| r_ready | logic | 1 | Output | Yes | Read data ready |
| r_data | logic | DATA_WIDTH | Input | Yes | Read data |
| r_resp | logic | 2 | Input | Yes | Read response |
| r_last | logic | 1 | Input | Yes | Read last |
| r_id | logic | AXI_ID_WIDTH | Input | Yes | Read ID |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Descriptor Packet Format

##### Control Fields

| Field | Bits | Width | Description | Values |
| --- | --- | --- | --- | --- |
| c0de | [191:180] | 12 | Code field | Implementation specific |
| tbd | [179:165] | 15 | To be defined | Reserved |
| eos | [164] | 1 | End of Stream marker | 1=EOS, 0=Normal |
| eol | [163] | 1 | End of Line marker | 1=EOL, 0=Normal |
| eod | [162] | 1 | End of Data marker | 1=EOD, 0=Normal |
| type | [161:160] | 2 | Packet type | See table below |

##### Packet Type Encoding

| Type | Value | Description |
| --- | --- | --- |
| FC Tile | 2’b00 | Flow Control Tile packets |
| TS Data | 2’b01 | Time Series Data packets |
| Risc-V | 2’b10 | RISC-V processor packets |
| RAW Data | 2’b11 | Raw data packets |

##### Address and Data Fields

| Field | Bits | Width | Description |
| --- | --- | --- | --- |
| Next Descriptor | [159:128], [127:96] | 64 | Next descriptor address |
| Data Address | [95:64], [63:32] | 64 | Data transfer address |
| Data Length | [31:0] | 32 | Data transfer length |
| Program 0 Address | [287:256], [255:224] | 64 | Program 0 write address |
| Program 0 Data | [223:192] | 32 | Program 0 write data |
| Program 1 Address | [351:320], [319:288] | 64 | Program 1 write address |
| Program 1 Data | [383:352] | 32 | Program 1 write data |

#### State Machine Operation

##### State Definitions

| State | Description |
| --- | --- |
| IDLE | Ready for APB or CDA requests, monitor operation exclusivity |
| ISSUE_ADDR | Issue AXI read transaction for APB requests |
| WAIT_DATA | Wait for AXI read completion with channel ID matching |
| COMPLETE | Process descriptor data and generate output |
| ERROR | Handle errors with recovery capability |

##### Priority Processing

The Descriptor Engine implements sophisticated priority processing:

- CDA Packet Priority: CDA packets from HIVE-C always processed before APB requests

  - Rationale: Low-latency descriptor injection from HIVE-C control processor

  - Mechanism: CDA packets bypass APB queue when both interfaces active

- Operation Exclusivity: Only one operation type active at a time

- Channel Reset Coordination: Graceful shutdown with AXI completion

Delta Network Integration Benefits: - Zero-polling overhead: HIVE-C pushes descriptors via CDA packets instead of RAPIDS polling memory - Sub-microsecond latency: CDA packets traverse Delta Network in ~10-20 cycles - Priority-based scheduling: CDA packet TID field carries descriptor priority (0=highest, 15=lowest)

#### Architecture

#### Descriptor Engine FSM

The Descriptor Engine implements a sophisticated six-state finite state machine that orchestrates descriptor fetching and processing operations with dual-path support for both APB programming interface requests and CDA packet interface operations. The FSM manages the complete descriptor lifecycle from initial request validation through AXI read transactions, descriptor parsing, and final output generation.

Descriptor Engine FSM

Key States: - DESC_IDLE: Ready for APB or CDA requests with operation exclusivity monitoring - DESC_AXI_READ: Issues AXI read transactions for APB descriptor fetch requests - DESC_WAIT_READ: Waits for AXI read completion with channel ID-based response routing - DESC_CHECK: Validates descriptor content and extracts stream boundary information - DESC_LOAD: Processes descriptor data and generates enhanced output with metadata - DESC_ERROR: Handles error conditions with comprehensive recovery capabilities

The FSM implements intelligent priority processing where CDA packets always take precedence over APB requests, ensuring optimal network responsiveness. Stream boundary support includes complete EOS/EOL/EOD field extraction and propagation, while the 4-deep descriptor FIFO maintains continuous operation flow. Channel reset coordination provides graceful shutdown capabilities, completing any in-flight AXI transactions before asserting idle status for system-level reset coordination.

##### FIFO Management

###### APB Request Skid Buffer

// APB request skid buffer (2-deep)
gaxi_skid_buffer #(
    .DATA_WIDTH(ADDR_WIDTH),
    .DEPTH(2),
    .INSTANCE_NAME("APB_REQ_SKID")
) i_apb_request_skid_buffer (
    .axi_aclk(clk),
    .axi_aresetn(rst_n),
    .wr_valid(apb_valid),
    .wr_ready(apb_ready),
    .wr_data(apb_addr),
    .rd_valid(w_apb_skid_valid_out),
    .rd_ready(w_apb_skid_ready_out),
    .rd_data(w_apb_skid_dout),
    .count(),
    .rd_count()
);

###### CDA Packet Skid Buffer

// CDA packet skid buffer (4-deep)
gaxi_skid_buffer #(
    .DATA_WIDTH(DATA_WIDTH + CHAN_WIDTH),
    .DEPTH(4),
    .INSTANCE_NAME("CDA_PKT_SKID")
) i_cda_packet_skid_buffer (
    .axi_aclk(clk),
    .axi_aresetn(rst_n),
    .wr_valid(cda_valid),
    .wr_ready(cda_ready),
    .wr_data({cda_packet, cda_channel}),
    .rd_valid(w_cda_skid_valid_out),
    .rd_ready(w_cda_skid_ready_out),
    .rd_data(w_cda_skid_dout),
    .count(),
    .rd_count()
);

###### Descriptor Output FIFO

// Descriptor output FIFO (4-deep)
gaxi_skid_buffer #(
    .DATA_WIDTH($bits(enhanced_descriptor_t)),
    .DEPTH(4),
    .INSTANCE_NAME("DESC_FIFO")
) i_descriptor_fifo (
    .axi_aclk(clk),
    .axi_aresetn(rst_n),
    .wr_valid(w_desc_fifo_wr_valid),
    .wr_ready(w_desc_fifo_wr_ready),
    .wr_data(w_desc_fifo_wr_data),
    .rd_valid(w_desc_fifo_rd_valid),
    .rd_ready(w_desc_fifo_rd_ready),
    .rd_data(w_desc_fifo_rd_data),
    .count(),
    .rd_count()
);

#### Monitor Events

##### Event Generation

// Monitor event types:
// - CORE_COMPL_DESCRIPTOR_LOADED: Descriptor processing completed
// - Network_STREAM_END: EOS boundary detected
// - Custom EOL/EOD events: Stream boundary processing
// - CORE_ERR_DESCRIPTOR_BAD_ADDR: Address validation failed
// - AXI_ERR_RESP_SLVERR: AXI slave error
// - AXI_ERR_RESP_DECERR: AXI decode error

#### Performance Characteristics

##### Processing Rates

- Descriptor Throughput: 1 descriptor per cycle when FIFO space available

- AXI Read Latency: 10-20 cycles depending on arbitration and memory response

- CDA Processing: 1-2 cycles for direct processing without AXI

- Validation Overhead: <1 cycle for comprehensive validation

##### Resource Utilization

- Skid Buffers: 2-deep APB + 4-deep CDA + 4-deep output = 10 total entries

- State Machine: 6 states with efficient encoding

- AXI Interface: Standard AXI4 signals with channel ID embedding

- Validation Logic: Minimal overhead for address and stream boundary checking

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Ctrlwr Engine

#### Overview

The Ctrlwr Engine manages post-processing write operations for each virtual channel after descriptor completion. The module implements a four-state state machine that handles ctrlwr address writes with configurable data values to support notification, control, and cleanup operations with comprehensive timeout mechanisms and robust error handling.

ctrlwr engine

ctrlwr engine

#### Key Features

- Four-State State Machine: Comprehensive ctrlwr operation lifecycle management

- Standardized Skid Buffer: Uses gaxi_skid_buffer for consistent flow control

- Null Address Support: Graceful handling of conditional ctrlwr operations

- Multi-Channel AXI Support: Proper shared AXI interface with ID-based response routing

- Address Validation: 4-byte alignment checking with graceful error handling

- Channel Reset Support: Graceful shutdown with AXI transaction completion

- Monitor Integration: Comprehensive event reporting for ctrlwr operations

- Flexible Control Writes: Support for various use cases and data patterns

#### Interface Specification

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| CHANNEL_ID | 0 | Static channel identifier for this engine instance |
| NUM_CHANNELS | 32 | Total number of channels in system |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Width of channel address fields |
| ADDR_WIDTH | 64 | Address width for AXI transactions |
| AXI_ID_WIDTH | 8 | AXI transaction ID width |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Scheduler FSM Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlwr_valid | logic | 1 | Input | Yes | Ctrlwr operation request from FSM |
| ctrlwr_ready | logic | 1 | Output | Yes | Ctrlwr operation complete |
| ctrlwr_pkt_addr | logic | ADDR_WIDTH | Input | Yes | Ctrlwr write address |
| ctrlwr_pkt_data | logic | 32 | Input | Yes | Ctrlwr write data |
| ctrlwr_error | logic | 1 | Output | Yes | Ctrlwr operation error |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_channel_reset | logic | 1 | Input | Yes | Dynamic channel reset request |

##### Status Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlwr_engine_idle | logic | 1 | Output | Yes | Engine idle status indicator |

##### Shared AXI4 Master Write Interface (32-bit)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| aw_valid | logic | 1 | Output | Yes | Write address valid |
| aw_ready | logic | 1 | Input | Yes | Write address ready (arbitrated) |
| aw_addr | logic | ADDR_WIDTH | Output | Yes | Write address |
| aw_len | logic | 8 | Output | Yes | Burst length - 1 (always 0) |
| aw_size | logic | 3 | Output | Yes | Transfer size (3’b010 for 4 bytes) |
| aw_burst | logic | 2 | Output | Yes | Burst type (2’b01 INCR) |
| aw_id | logic | AXI_ID_WIDTH | Output | Yes | Transaction ID (channel-based) |
| aw_lock | logic | 1 | Output | Yes | Lock type (always 0) |
| aw_cache | logic | 4 | Output | Yes | Cache attributes |
| aw_prot | logic | 3 | Output | Yes | Protection attributes |
| aw_qos | logic | 4 | Output | Yes | Quality of service |
| aw_region | logic | 4 | Output | Yes | Region identifier |
| w_valid | logic | 1 | Output | Yes | Write data valid |
| w_ready | logic | 1 | Input | Yes | Write data ready |
| w_data | logic | 32 | Output | Yes | Write data |
| w_strb | logic | 4 | Output | Yes | Write strobes (always 4’hF) |
| w_last | logic | 1 | Output | Yes | Write last (always 1) |
| b_valid | logic | 1 | Input | Yes | Write response valid |
| b_ready | logic | 1 | Output | Yes | Write response ready |
| b_id | logic | AXI_ID_WIDTH | Input | Yes | Response ID (channel identification) |
| b_resp | logic | 2 | Input | Yes | Write response |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Ctrlwr Engine FSM

The Ctrlwr Engine implements a streamlined four-state finite state machine that manages post-processing write operations for each virtual channel after descriptor completion. The FSM handles ctrlwr address writes with configurable data values to support notification, control, and cleanup operations with comprehensive timeout mechanisms and robust error handling.

The FSM supports conditional ctrlwr through graceful null address handling, where operations are skipped immediately when ctrlwr addresses are 64’h0, enabling descriptor-driven conditional execution. Address validation ensures 4-byte alignment compliance, while the shared AXI interface uses ID-based response routing for proper multi-channel operation. Channel reset coordination provides graceful shutdown capabilities, completing any in-flight AXI write transactions before asserting idle status, enabling runtime reconfiguration without system disruption.

Ctrlwr Engine FSM

##### State Definitions

| State | Description |
| --- | --- |
| WRITE_IDLE | Ready for new ctrlwr operation request |
| WRITE_ISSUE_ADDR | Issue AXI write address and data phases |
| WRITE_WAIT_RESP | Wait for AXI write response |
| WRITE_ERROR | Handle error conditions |

##### State Transitions

WRITE_IDLE -> WRITE_ISSUE_ADDR: Valid ctrlwr request with non-null address
WRITE_IDLE -> WRITE_IDLE: Valid ctrlwr request with null address (skip operation)
WRITE_IDLE -> WRITE_ERROR: Valid ctrlwr request with address error
WRITE_ISSUE_ADDR -> WRITE_WAIT_RESP: Both AXI address and data phases complete
WRITE_ISSUE_ADDR -> WRITE_ERROR: AXI transaction timeout
WRITE_WAIT_RESP -> WRITE_IDLE: AXI response received successfully
WRITE_WAIT_RESP -> WRITE_ERROR: AXI response indicates error
WRITE_ERROR -> WRITE_IDLE: Error acknowledged or channel reset

##### Operation Flow

- Request Reception: Scheduler provides ctrlwr address and data

- Address Validation: Check for null address and 4-byte alignment

- AXI Transaction: Issue write address and data simultaneously

- Response Monitoring: Wait for AXI response with correct channel ID

- Completion: Signal ready to scheduler when operation complete

#### Ctrlwr Operation Types

##### Conditional Control Write (Null Address)

- Detection: Ctrlwr address is 64’h0

- Behavior: Operation is skipped, ready asserted immediately

- Use Case: Conditional ctrlwr based on descriptor content

- Monitor Event: Null ctrlwr operation completion

##### Standard Control Write

- Detection: Non-zero, 4-byte aligned address

- Behavior: Standard AXI write transaction executed

- Use Case: Notification, control register updates, cleanup operations

- Monitor Event: Ctrlwr write completion with success/error status

##### Address Error Handling

- Detection: Non-zero address not 4-byte aligned

- Behavior: Error state entered, no AXI transaction issued

- Use Case: Invalid descriptor content detection

- Monitor Event: Address validation error

#### AXI Transaction Management

##### Channel ID Encoding

// AXI ID construction for channel identification
assign r_expected_axi_id = {{(AXI_ID_WIDTH-CHAN_WIDTH){1'b0}}, CHANNEL_ID[CHAN_WIDTH-1:0]};
assign aw_id = r_expected_axi_id;

##### Write Transaction Properties

// AXI write attributes for 32-bit ctrlwr operations
assign aw_len = 8'h00;          // Single beat
assign aw_size = 3'b010;        // 4 bytes (32 bits)
assign aw_burst = 2'b01;        // INCR burst type
assign aw_lock = 1'b0;          // Normal access
assign w_strb = 4'hF;           // All bytes valid
assign w_last = 1'b1;           // Single beat transaction

##### Response Monitoring

// Response monitoring for correct channel
assign w_our_axi_response = b_valid && (b_id == r_expected_axi_id);
assign w_axi_response_error = (b_resp != 2'b00); // Not OKAY response

// Ready when waiting for our response
assign b_ready = (r_current_state == WRITE_WAIT_RESP) && w_our_axi_response;

#### Channel Reset Coordination

The ctrlwr engine supports graceful channel reset:

##### Reset Behavior

- Block New Requests: Stop accepting ctrlwr requests during reset

- Complete AXI Transaction: Finish any in-flight AXI write operation

- Clear State: Reset internal state and skid buffer

- Signal Completion: Assert ctrlwr_engine_idle when complete

##### Reset Timing

- AXI Safety: Waits for AXI write completion in WRITE_WAIT_RESP state

- FIFO Cleanup: Clears ctrlwr request skid buffer

- State Reset: Returns FSM to WRITE_IDLE state

- Idle Indication: Provides clean idle signal for system coordination

// Channel reset coordination
assign w_safe_to_reset = (r_current_state == WRITE_IDLE) &&
                        w_fifo_empty &&
                        w_no_active_transaction;

assign ctrlwr_engine_idle = w_safe_to_reset && !r_channel_reset_active;

#### Error Handling

##### Error Sources

- Address Errors: Non-4-byte aligned addresses

- AXI Response Errors: SLVERR, DECERR from AXI infrastructure

- Timeout Errors: AXI transaction timeout detection

- Protocol Errors: Invalid AXI handshake sequences

##### Error Recovery

// Error state recovery conditions
assign w_error_recovery = error_acknowledged || r_channel_reset_active;

// Error persistence for debugging
always_ff @(posedge clk) begin
    if (!rst_n || r_channel_reset_active) begin
        r_ctrlwr_error <= 1'b0;
    end else if (w_error_condition_detected) begin
        r_ctrlwr_error <= 1'b1;
    end
end

#### Monitor Bus Events

The ctrlwr engine generates comprehensive monitor events:

##### Error Events

- Address Error: Invalid address alignment detected

- AXI Error: AXI write response error conditions

- Timeout: AXI transaction timeout detection

- Protocol Error: Invalid operation sequence

##### Performance Events

- Ctrlwr Write: Successful ctrlwr write completion

- Null Ctrlwr: Null address ctrlwr operation skipped

- Processing Latency: Time from request to completion

- Throughput: Ctrlwr operation rate

##### Completion Events

- Write Completion: Standard ctrlwr write complete

- Skip Completion: Null address operation complete

- Error Recovery: Error condition resolution

- Channel Reset: Channel reset sequence completion

#### Input Buffer Management

##### Skid Buffer Architecture

// Standardized skid buffer for ctrlwr requests
gaxi_skid_buffer #(
    .DATA_WIDTH(ADDR_WIDTH + 32),  // Address + Data
    .SKID_DEPTH(4)                 // Configurable depth
) u_ctrlwr_req_skid (
    .axi_aclk(clk),
    .axi_aresetn(rst_n),
    .s_axis_tvalid(ctrlwr_valid),
    .s_axis_tready(ctrlwr_ready),
    .s_axis_tdata({ctrlwr_pkt_addr, ctrlwr_pkt_data}),
    .m_axis_tvalid(w_ctrlwr_req_skid_valid_out),
    .m_axis_tready(w_ctrlwr_req_skid_ready_out),
    .m_axis_tdata(w_ctrlwr_req_skid_dout)
);

##### Flow Control

// Ready signal generation based on state and reset
assign w_ctrlwr_req_skid_ready_out = (r_current_state == WRITE_IDLE) &&
                                     w_ctrlwr_req_skid_valid_out &&
                                     !r_channel_reset_active;

#### Performance Characteristics

##### Latency Analysis

- Null Ctrlwr: 1 cycle (immediate completion)

- Standard Ctrlwr: 4-8 cycles (AXI address + data + response)

- Error Handling: 2-4 cycles (error detection + recovery)

- Channel Reset: Variable (depends on AXI completion)

##### Throughput Analysis

- Peak Rate: 1 ctrlwr operation per 4-8 cycles

- Sustained Rate: Limited by AXI write bandwidth and arbitration

- Multi-Channel: Independent operation across channels

- Efficiency: >90% for back-to-back ctrlwr operations

##### AXI Utilization

- Address Utilization: 100% for valid operations

- Data Utilization: 100% (32-bit data on 32-bit interface)

- Response Handling: Selective response monitoring by channel ID

- Error Rate: <0.1% under normal operating conditions

#### Usage Guidelines

##### Control Write Patterns

Notification Control Write:

// Notify completion to external processor
ctrlwr_addr = COMPLETION_REGISTER_BASE + (channel_id * 4);
ctrlwr_data = descriptor_completion_status;

Control Register Update:

// Update control registers based on stream state
ctrlwr_addr = CONTROL_REGISTER_BASE + (channel_id * 4);
ctrlwr_data = {reserved, eos_detected, error_flags, completion_count};

Cleanup Control Write:

// Clear status registers after processing
ctrlwr_addr = STATUS_REGISTER_BASE + (channel_id * 4);
ctrlwr_data = 32'h0000_0000;

##### Error Monitoring

The ctrlwr engine provides comprehensive error detection: - Monitor address alignment errors for descriptor validation - Check AXI response codes for system health - Track timeout conditions for performance analysis - Use monitor events for debugging and optimization

##### Performance Optimization

- Use null addresses for conditional ctrlwr to reduce AXI traffic

- Batch multiple ctrlwr operations where possible

- Monitor AXI arbitration delays and adjust system priorities

- Configure skid buffer depth based on scheduler burst characteristics

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Ctrlrd Engine

#### Overview

The Ctrlrd Engine manages pre-processing read operations for each virtual channel before descriptor execution. The module implements a retry-capable read-and-compare mechanism that polls a control address until the read value matches an expected value, or a maximum retry count is exceeded. This enables synchronization, flag polling, and pre-condition validation before data operations begin.

ctrlrd engine

ctrlrd engine

#### Key Features

- Read-and-Compare Mechanism: Automatic retry until read data matches expected value

- Configurable Retry Count: Up to 511 retries via cfg_ctrlrd_max_try[8:0]

- Mask Support: Flexible bit-wise masking for partial value comparison

- 1µs Timeout Counter: Microsecond-resolution timeout using scheduler_group 1µs tick

- Standardized Skid Buffer: Uses gaxi_skid_buffer for consistent flow control

- Null Address Support: Graceful handling of conditional ctrlrd operations

- AXI4 Read Interface: Standard 32-bit AXI4-Lite read transactions

- Channel Reset Support: Graceful shutdown with AXI transaction completion

- Monitor Integration: Comprehensive event reporting for ctrlrd operations

#### Interface Specification

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| CHANNEL_ID | 0 | Static channel identifier for this engine instance |
| NUM_CHANNELS | 32 | Total number of channels in system |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Width of channel address fields |
| ADDR_WIDTH | 64 | Address width for AXI transactions |
| AXI_ID_WIDTH | 8 | AXI transaction ID width |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Scheduler FSM Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlrd_valid | logic | 1 | Input | Yes | Ctrlrd operation request from scheduler |
| ctrlrd_ready | logic | 1 | Output | Yes | Ctrlrd operation complete (match or error) |
| ctrlrd_pkt_addr | logic | ADDR_WIDTH | Input | Yes | Ctrlrd read address |
| ctrlrd_pkt_data | logic | 32 | Input | Yes | Ctrlrd expected/comparison data |
| ctrlrd_pkt_mask | logic | 32 | Input | Yes | Ctrlrd bit mask for comparison |
| ctrlrd_error | logic | 1 | Output | Yes | Ctrlrd operation error (valid with ctrlrd_ready) |
| ctrlrd_result | logic | 32 | Output | Yes | Ctrlrd actual read result data |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_ctrlrd_max_try | logic | 9 | Input | Yes | Maximum retry count (0-511) |
| cfg_channel_reset | logic | 1 | Input | Yes | Dynamic channel reset request |
| tick_1us | logic | 1 | Input | Yes | 1 microsecond tick from scheduler_group |

##### Status Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ctrlrd_engine_idle | logic | 1 | Output | Yes | Engine idle status indicator |
| ctrlrd_retry_count | logic | 9 | Output | Yes | Current retry count value |

##### Shared AXI4 Read Interface (32-bit)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ar_valid | logic | 1 | Output | Yes | Read address valid |
| ar_ready | logic | 1 | Input | Yes | Read address ready (arbitrated) |
| ar_addr | logic | ADDR_WIDTH | Output | Yes | Read address |
| ar_len | logic | 8 | Output | Yes | Burst length - 1 (always 0) |
| ar_size | logic | 3 | Output | Yes | Transfer size (3’b010 for 4 bytes) |
| ar_burst | logic | 2 | Output | Yes | Burst type (2’b01 INCR) |
| ar_id | logic | AXI_ID_WIDTH | Output | Yes | Transaction ID (channel-based) |
| ar_lock | logic | 1 | Output | Yes | Lock type (always 0) |
| ar_cache | logic | 4 | Output | Yes | Cache attributes |
| ar_prot | logic | 3 | Output | Yes | Protection attributes |
| ar_qos | logic | 4 | Output | Yes | Quality of service |
| ar_region | logic | 4 | Output | Yes | Region identifier |
| r_valid | logic | 1 | Input | Yes | Read data valid |
| r_ready | logic | 1 | Output | Yes | Read data ready |
| r_data | logic | 32 | Input | Yes | Read data |
| r_resp | logic | 2 | Input | Yes | Read response |
| r_last | logic | 1 | Input | Yes | Read last indicator |
| r_id | logic | AXI_ID_WIDTH | Input | Yes | Response ID (channel identification) |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Ctrlrd Engine FSM

The Ctrlrd Engine implements a retry-capable read-compare-retry finite state machine that manages control read operations before descriptor execution.

Ctrlrd Engine FSM

##### State Definitions

| State | Description |
| --- | --- |
| READ_IDLE | Ready for new ctrlrd operation request |
| READ_ISSUE_ADDR | Issue AXI read address phase |
| READ_WAIT_DATA | Wait for AXI read data response |
| READ_COMPARE | Compare read data with expected value |
| READ_RETRY_WAIT | Wait 1µs before retry attempt |
| READ_MATCH | Read data matched expected value (success) |
| READ_ERROR | Handle error conditions (max retries or AXI error) |

##### State Transitions

READ_IDLE -> READ_ISSUE_ADDR: Valid ctrlrd request with non-null address
READ_IDLE -> READ_MATCH: Valid ctrlrd request with null address (skip operation)
READ_ISSUE_ADDR -> READ_WAIT_DATA: AXI address phase complete
READ_WAIT_DATA -> READ_COMPARE: AXI read data received
READ_WAIT_DATA -> READ_ERROR: AXI response error (SLVERR, DECERR)
READ_COMPARE -> READ_MATCH: Read data matches expected value (success)
READ_COMPARE -> READ_RETRY_WAIT: Mismatch and retries remaining
READ_COMPARE -> READ_ERROR: Mismatch and max retries exceeded
READ_RETRY_WAIT -> READ_ISSUE_ADDR: 1µs elapsed, retry read
READ_MATCH -> READ_IDLE: Success acknowledged by scheduler
READ_ERROR -> READ_IDLE: Error acknowledged or channel reset

##### Operation Flow

- Request Reception: Scheduler provides ctrlrd address, expected data, and mask

- Address Validation: Check for null address (skip operation if 64’h0)

- AXI Read Transaction: Issue read address and wait for data

- Data Comparison: Compare (r_data & mask) == (expected_data & mask)

- Match Success: Assert ctrlrd_ready without ctrlrd_error

- Mismatch Handling:

  - If retries remaining: Wait 1µs, retry read

  - If max retries exceeded: Assert ctrlrd_ready with ctrlrd_error

#### Retry Mechanism

##### Retry Counter Management

// Retry counter (0 to cfg_ctrlrd_max_try)
logic [8:0] r_retry_count;
logic w_retries_remaining;

assign w_retries_remaining = (r_retry_count < cfg_ctrlrd_max_try);

// Increment on each retry attempt
if (r_current_state == READ_COMPARE && !w_data_match && w_retries_remaining) begin
    r_retry_count <= r_retry_count + 1;
end

// Reset on new operation or match
if (r_current_state == READ_IDLE || r_current_state == READ_MATCH) begin
    r_retry_count <= 9'h0;
end

##### 1µs Retry Delay

// 1µs delay between retry attempts
// tick_1us provided by scheduler_group counter_freq_invariant
logic r_retry_wait_complete;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        r_retry_wait_complete <= 1'b0;
    end else if (r_current_state == READ_RETRY_WAIT) begin
        if (tick_1us) begin
            r_retry_wait_complete <= 1'b1;
        end
    end else begin
        r_retry_wait_complete <= 1'b0;
    end
end

// Transition to retry after 1µs
if (r_current_state == READ_RETRY_WAIT && r_retry_wait_complete) begin
    w_next_state = READ_ISSUE_ADDR;
end

#### Data Comparison Logic

##### Masked Comparison

// Compare with mask support
logic [31:0] w_masked_expected;
logic [31:0] w_masked_actual;
logic w_data_match;

assign w_masked_expected = r_expected_data & r_mask;
assign w_masked_actual = r_axi_read_data & r_mask;
assign w_data_match = (w_masked_expected == w_masked_actual);

// Comparison in READ_COMPARE state
if (r_current_state == READ_COMPARE) begin
    if (w_data_match) begin
        w_next_state = READ_MATCH;  // Success
    end else if (w_retries_remaining) begin
        w_next_state = READ_RETRY_WAIT;  // Retry after 1µs
    end else begin
        w_next_state = READ_ERROR;  // Max retries exceeded
    end
end

##### Null Address Handling

// Null address detection (skip operation)
logic w_null_address;
assign w_null_address = (r_ctrlrd_addr == 64'h0);

// Skip to READ_MATCH immediately for null addresses
if ((r_current_state == READ_IDLE) && ctrlrd_valid && w_null_address) begin
    w_next_state = READ_MATCH;  // Skip operation, no error
end

#### AXI Transaction Management

##### Channel ID Encoding

// AXI ID construction for channel identification
assign r_expected_axi_id = {{(AXI_ID_WIDTH-CHAN_WIDTH){1'b0}}, CHANNEL_ID[CHAN_WIDTH-1:0]};
assign ar_id = r_expected_axi_id;

##### Read Transaction Properties

// AXI read attributes for 32-bit ctrlrd operations
assign ar_len = 8'h00;          // Single beat
assign ar_size = 3'b010;        // 4 bytes (32 bits)
assign ar_burst = 2'b01;        // INCR burst type
assign ar_lock = 1'b0;          // Normal access
assign ar_cache = 4'b0000;      // Device non-bufferable
assign ar_prot = 3'b000;        // Unprivileged, non-secure, data
assign ar_qos = 4'h0;           // No QoS
assign ar_region = 4'h0;        // Default region

##### Response Monitoring

// Response monitoring for correct channel
assign w_our_axi_response = r_valid && (r_id == r_expected_axi_id);
assign w_axi_response_error = (r_resp != 2'b00); // Not OKAY response

// Ready when waiting for our response
assign r_ready = (r_current_state == READ_WAIT_DATA) && w_our_axi_response;

// Error detection
if (r_current_state == READ_WAIT_DATA && w_our_axi_response) begin
    if (w_axi_response_error) begin
        w_next_state = READ_ERROR;  // AXI error (SLVERR, DECERR)
    end else begin
        r_axi_read_data <= r_data;
        w_next_state = READ_COMPARE;
    end
end

#### Channel Reset Coordination

The ctrlrd engine supports graceful channel reset:

##### Reset Behavior

- Block New Requests: Stop accepting ctrlrd requests during reset

- Complete AXI Transaction: Finish any in-flight AXI read operation

- Clear State: Reset internal state and skid buffer

- Signal Completion: Assert ctrlrd_engine_idle when complete

##### Reset Timing

// Channel reset coordination
assign w_safe_to_reset = (r_current_state == READ_IDLE) &&
                        w_fifo_empty &&
                        w_no_active_transaction;

assign ctrlrd_engine_idle = w_safe_to_reset && !r_channel_reset_active;

#### Error Handling

##### Error Sources

- AXI Response Errors: SLVERR, DECERR from AXI infrastructure

- Max Retries Exceeded: Read data never matched expected value

- Timeout Errors: Combined with retry count for overall timeout

- Protocol Errors: Invalid AXI handshake sequences

##### Error Recovery

// Error state outputs
assign ctrlrd_ready = (r_current_state == READ_MATCH) ||
                     (r_current_state == READ_ERROR);
assign ctrlrd_error = (r_current_state == READ_ERROR);

// Error recovery on acknowledgment
if (r_current_state == READ_ERROR && ctrlrd_ready && ctrlrd_valid) begin
    w_next_state = READ_IDLE;
end

// Or on channel reset
if (r_channel_reset_active && r_current_state == READ_ERROR) begin
    w_next_state = READ_IDLE;
end

#### Monitor Bus Events

The ctrlrd engine generates comprehensive monitor events:

##### Error Events

- AXI Read Error: AXI read response error (SLVERR, DECERR)

- Max Retries Exceeded: Retry count exceeded without match

- Timeout: Overall operation timeout

- Protocol Error: Invalid operation sequence

##### Performance Events

- Ctrlrd Match: Successful read-compare match

- Retry Count: Number of retry attempts before success

- Null Ctrlrd: Null address ctrlrd operation skipped

- Processing Latency: Time from request to completion

##### Completion Events

- Match Completion: Read data matched expected value

- Skip Completion: Null address operation complete

- Error Recovery: Error condition resolution

- Channel Reset: Channel reset sequence completion

#### Input Buffer Management

##### Skid Buffer Architecture

// Standardized skid buffer for ctrlrd requests
gaxi_skid_buffer #(
    .DATA_WIDTH(ADDR_WIDTH + 32 + 32),  // Address + Data + Mask
    .SKID_DEPTH(4)                      // Configurable depth
) u_ctrlrd_req_skid (
    .axi_aclk(clk),
    .axi_aresetn(rst_n),
    .s_axis_tvalid(ctrlrd_valid),
    .s_axis_tready(ctrlrd_ready),
    .s_axis_tdata({ctrlrd_pkt_addr, ctrlrd_pkt_data, ctrlrd_pkt_mask}),
    .m_axis_tvalid(w_ctrlrd_req_skid_valid_out),
    .m_axis_tready(w_ctrlrd_req_skid_ready_out),
    .m_axis_tdata(w_ctrlrd_req_skid_dout)
);

##### Flow Control

// Ready signal generation based on state and reset
assign w_ctrlrd_req_skid_ready_out = (r_current_state == READ_IDLE) &&
                                     w_ctrlrd_req_skid_valid_out &&
                                     !r_channel_reset_active;

#### Performance Characteristics

##### Latency Analysis

- Null Ctrlrd: 1 cycle (immediate completion)

- Match on First Try: 6-10 cycles (AXI address + data + compare)

- Match After Retries: (6-10 + N x 1µs) where N = retry count

- Max Retries Exceeded: (6-10 + cfg_ctrlrd_max_try x 1µs) + error handling

##### Retry Behavior

- 1µs Delay: Fixed 1 microsecond delay between retry attempts

- Max Retries: Configurable 0-511 via cfg_ctrlrd_max_try[8:0]

- Total Timeout: Up to 511µs maximum retry duration

- Success Rate: Depends on external flag update frequency

##### AXI Utilization

- Address Utilization: 100% for valid operations

- Data Utilization: 100% (32-bit data on 32-bit interface)

- Response Handling: Selective response monitoring by channel ID

- Retry Overhead: 1µs minimum between attempts (prevents bus saturation)

#### Usage Guidelines

##### Control Read Patterns

Flag Polling:

// Poll for ready flag before descriptor execution
ctrlrd_addr = STATUS_REGISTER_BASE + (channel_id * 4);
ctrlrd_data = 32'h0000_0001;  // Expect bit 0 set
ctrlrd_mask = 32'h0000_0001;  // Check only bit 0

Multi-bit Status Check:

// Check multiple status bits before proceeding
ctrlrd_addr = CONTROL_REGISTER_BASE + (channel_id * 4);
ctrlrd_data = 32'h0000_00F0;  // Expect bits [7:4] all set
ctrlrd_mask = 32'h0000_00F0;  // Check only bits [7:4]

Synchronization Check:

// Wait for external synchronization event
ctrlrd_addr = SYNC_REGISTER_BASE;
ctrlrd_data = EXPECTED_SYNC_VALUE;
ctrlrd_mask = 32'hFFFF_FFFF;  // Check all bits

##### Retry Configuration

Configure retry count based on expected flag update frequency:

// Fast flags (updated every few microseconds)
cfg_ctrlrd_max_try = 9'd10;   // 10µs maximum wait

// Medium flags (updated every ~100µs)
cfg_ctrlrd_max_try = 9'd100;  // 100µs maximum wait

// Slow flags (updated every millisecond)
cfg_ctrlrd_max_try = 9'd511;  // 511µs maximum wait (max supported)

##### Error Monitoring

The ctrlrd engine provides comprehensive error detection: - Monitor AXI response codes for bus errors - Track retry count for performance analysis - Check for max retries exceeded to detect stuck conditions - Use monitor events for debugging and optimization

##### Performance Optimization

- Use appropriate masks to check only relevant bits

- Configure retry count based on expected flag update frequency

- Monitor retry statistics to optimize polling intervals

- Use null addresses for conditional ctrlrd to reduce AXI traffic

#### Integration with Scheduler Group

##### 1µs Counter Integration

The scheduler_group provides a shared 1µs tick signal to all ctrlrd engines:

// At scheduler_group level (scheduler_group.sv)
counter_freq_invariant #(
    .WIDTH(32),
    .FREQ_HZ(1_000_000)  // 1 MHz = 1µs period
) u_1us_counter (
    .clk(clk),
    .rst_n(rst_n),
    .tick(tick_1us)  // Shared 1µs tick to all ctrlrd engines
);

// Broadcast to all channels
for (genvar i = 0; i < NUM_CHANNELS; i++) begin
    assign ctrlrd_engine[i].tick_1us = tick_1us;
end

##### Channel Array Structure

// Scheduler group instantiates ctrlrd engines for each channel
generate
    for (genvar i = 0; i < NUM_CHANNELS; i++) begin : gen_ctrlrd_engines
        ctrlrd_engine #(
            .CHANNEL_ID(i),
            .NUM_CHANNELS(NUM_CHANNELS),
            .ADDR_WIDTH(ADDR_WIDTH),
            .AXI_ID_WIDTH(AXI_ID_WIDTH)
        ) u_ctrlrd_engine (
            .clk(clk),
            .rst_n(rst_n),

            // From scheduler
            .ctrlrd_valid(scheduler[i].ctrlrd_valid),
            .ctrlrd_ready(scheduler[i].ctrlrd_ready),
            .ctrlrd_pkt_addr(scheduler[i].ctrlrd_addr),
            .ctrlrd_pkt_data(scheduler[i].ctrlrd_data),
            .ctrlrd_pkt_mask(scheduler[i].ctrlrd_mask),
            .ctrlrd_error(scheduler[i].ctrlrd_error),
            .ctrlrd_result(scheduler[i].ctrlrd_result),

            // Configuration
            .cfg_ctrlrd_max_try(cfg_ctrlrd_max_try[i]),
            .cfg_channel_reset(cfg_channel_reset[i]),
            .tick_1us(tick_1us),  // Shared 1µs tick

            // Shared AXI read interface (arbitrated)
            .ar_valid(ctrlrd_ar_valid[i]),
            .ar_ready(ctrlrd_ar_ready[i]),
            // ... AXI signals ...

            // Monitor bus
            .mon_valid(ctrlrd_mon_valid[i]),
            .mon_ready(ctrlrd_mon_ready[i]),
            .mon_packet(ctrlrd_mon_packet[i])
        );
    end
endgenerate

This ctrlrd engine architecture provides robust flag polling and synchronization capabilities with configurable retry mechanisms and comprehensive error handling.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Sink Data Path

#### Overview

The Sink Data Path provides a complete integrated data reception and processing pipeline that combines AXI-Stream packet reception, multi-channel SRAM buffering, multi-channel AXI write arbitration, and comprehensive monitor bus aggregation. This wrapper manages the complete data flow from AXIS interface reception through final AXI memory writes.

RTL Module: rtl/amba/axis/axis_slave.sv (AXIS interface) + RAPIDS sink components

The wrapper implements sophisticated TLAST (End of Stream) flow control where packet-level TLAST is managed by SRAM control and descriptor-level EOS completion is coordinated with the scheduler, ensuring proper stream boundary handling throughout the pipeline.

sink data path

sink data path

#### Key Features

- Complete Data Reception Pipeline: From AXIS packets to AXI memory writes

- Standard AXIS Interface: Industry-standard AXI-Stream protocol (no custom credits/ACKs)

- Multi-Channel SRAM Buffering: Independent buffering for up to 32 channels

- Advanced AXI Write Engine: Multi-channel arbitration with transfer strategy optimization

- TLAST Flow Management: Packet-level and descriptor-level boundary coordination

- RDA Packet Routing: Direct RDA packet interfaces bypassing SRAM buffering

- Monitor Bus Aggregation: Unified monitoring from AXIS slave, SRAM control, and AXI engine

- Enhanced Scheduler Interface: Address alignment bus support for optimal AXI performance

#### Interface Specification

##### Clock and Reset

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### AXI-Stream Slave Interface (RX)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| axis_snk_rx_tdata | logic | DATA_WIDTH | Input | Yes | Stream data payload |
| axis_snk_rx_tstrb | logic | DATA_WIDTH/8 | Input | Yes | Byte strobes (write enables) |
| axis_snk_rx_tlast | logic | 1 | Input | Yes | Last transfer in packet |
| axis_snk_rx_tvalid | logic | 1 | Input | Yes | Stream data valid |
| axis_snk_rx_tready | logic | 1 | Output | Yes | Stream ready (backpressure) |
| axis_snk_rx_tuser | logic | 16 | Input | Yes | User sideband (packet metadata) |

TUSER Encoding (Sink RX):

[15:8] - Channel ID
[7:0]  - Packet type/flags

Note: AXIS uses standard tvalid/tready backpressure. No ACK channels or custom credit mechanisms.

##### RDA Interfaces (Direct Bypass)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rda_src_valid | logic | 1 | Output | Yes | RDA source packet valid |
| rda_src_ready | logic | 1 | Input | Yes | RDA source packet ready |
| rda_src_packet | logic | DATA_WIDTH | Output | Yes | RDA source packet data |
| rda_src_channel | logic | CHAN_WIDTH | Output | Yes | RDA source channel |
| rda_src_eos | logic | 1 | Output | Yes | RDA source End of Stream |
| rda_snk_valid | logic | 1 | Output | Yes | RDA sink packet valid |
| rda_snk_ready | logic | 1 | Input | Yes | RDA sink packet ready |
| rda_snk_packet | logic | DATA_WIDTH | Output | Yes | RDA sink packet data |
| rda_snk_channel | logic | CHAN_WIDTH | Output | Yes | RDA sink channel |
| rda_snk_eos | logic | 1 | Output | Yes | RDA sink End of Stream |

##### Multi-Channel Scheduler Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_valid | logic | NUM_CHANNELS | Input | Yes | Data transfer request per channel |
| data_ready | logic | NUM_CHANNELS | Output | Yes | Data transfer ready per channel |
| data_address | logic | ADDR_WIDTH x NUM_CHANNELS | Input | Yes | Data address per channel |
| data_length | logic | 32 x NUM_CHANNELS | Input | Yes | Data length per channel |
| data_type | logic | 2 x NUM_CHANNELS | Input | Yes | Data type per channel |
| data_eos | logic | NUM_CHANNELS | Input | Yes | End of Stream per channel |
| data_transfer_length | logic | 32 x NUM_CHANNELS | Output | Yes | Actual transfer length per channel |
| data_done_strobe | logic | NUM_CHANNELS | Output | Yes | Transfer completion per channel |
| data_error | logic | NUM_CHANNELS | Output | Yes | Transfer error per channel |

##### Address Alignment Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_alignment_info | alignment_info_t | NUM_CHANNELS | Input | Yes | Alignment information per channel |
| data_alignment_valid | logic | NUM_CHANNELS | Input | Yes | Alignment information valid per channel |
| data_alignment_ready | logic | NUM_CHANNELS | Output | Yes | Alignment information ready per channel |
| data_alignment_next | logic | NUM_CHANNELS | Output | Yes | Request next alignment per channel |
| data_transfer_phase | transfer_phase_t | NUM_CHANNELS | Input | Yes | Transfer phase per channel |
| data_sequence_complete | logic | NUM_CHANNELS | Output | Yes | Transfer sequence complete per channel |

##### EOS Completion Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| eos_completion_valid | logic | 1 | Output | Yes | EOS completion notification valid |
| eos_completion_ready | logic | 1 | Input | Yes | EOS completion notification ready |
| eos_completion_channel | logic | CHAN_WIDTH | Output | Yes | Channel with EOS completion |

##### AXI4 Master Write Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_awvalid | logic | 1 | Output | Yes | Write address valid |
| m_axi_awready | logic | 1 | Input | Yes | Write address ready |
| m_axi_awaddr | logic | AXI_ADDR_WIDTH | Output | Yes | Write address |
| m_axi_awlen | logic | 8 | Output | Yes | Burst length |
| m_axi_awsize | logic | 3 | Output | Yes | Burst size |
| m_axi_awburst | logic | 2 | Output | Yes | Burst type |
| m_axi_awid | logic | AXI_ID_WIDTH | Output | Yes | Write ID |
| m_axi_awlock | logic | 1 | Output | Yes | Lock type |
| m_axi_awcache | logic | 4 | Output | Yes | Cache type |
| m_axi_awprot | logic | 3 | Output | Yes | Protection type |
| m_axi_awqos | logic | 4 | Output | Yes | QoS identifier |
| m_axi_awregion | logic | 4 | Output | Yes | Region identifier |
| m_axi_wvalid | logic | 1 | Output | Yes | Write data valid |
| m_axi_wready | logic | 1 | Input | Yes | Write data ready |
| m_axi_wdata | logic | DATA_WIDTH | Output | Yes | Write data |
| m_axi_wstrb | logic | DATA_WIDTH/8 | Output | Yes | Write strobes |
| m_axi_wlast | logic | 1 | Output | Yes | Write last |
| m_axi_bvalid | logic | 1 | Input | Yes | Write response valid |
| m_axi_bready | logic | 1 | Output | Yes | Write response ready |
| m_axi_bresp | logic | 2 | Input | Yes | Write response |
| m_axi_bid | logic | AXI_ID_WIDTH | Input | Yes | Write ID |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

##### Status and Error Reporting

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| channel_eos_pending | logic | NUM_CHANNELS | Output | Yes | EOS pending per channel |
| error_header_parity | logic | 1 | Output | Yes | Header parity error |
| error_body_parity | logic | 1 | Output | Yes | Body parity error |
| error_buffer_overflow | logic | 1 | Output | Yes | Buffer overflow error |
| error_ack_lost | logic | 1 | Output | Yes | ACK lost error |
| error_channel_id | logic | CHAN_WIDTH | Output | Yes | Channel with error |
| packet_count | logic | 32 | Output | Yes | Total packet count |
| error_count | logic | 16 | Output | Yes | Total error count |

#### Architecture

##### Internal Components

- AXIS Slave: Packet reception, validation, and routing with standard AXIS flow control

- Sink SRAM Control: Multi-channel buffering with TLAST/EOS completion signaling

- Sink AXI Write Engine: Multi-channel arbitration and AXI write operations

- Monitor Bus Aggregator: Round-robin aggregation from all three components

##### Data Flow Pipeline

- AXIS Packet Reception: AXIS slave receives and validates incoming packets

- Packet Classification: Packets routed to SRAM (FC/TS/RAW) or RDA bypass (RDA packets)

- Multi-Channel Buffering: SRAM control provides independent per-channel buffering

- AXI Write Arbitration: AXI engine arbitrates between channels using scheduler inputs

- TLAST Completion: Packet-level TLAST triggers descriptor completion notification

##### TLAST Flow Management

The sink data path implements sophisticated stream boundary handling: - Packet-Level TLAST: Detected by AXIS slave, managed by SRAM control - EOS Completion Interface: SRAM control notifies scheduler of descriptor completion - Stream Boundaries: TLAST triggers proper completion signaling (maps to internal EOS)

##### Address Alignment Integration

The wrapper supports the scheduler’s address alignment bus, enabling: - Pre-calculated Alignment: Scheduler provides alignment information before transfers - Optimal AXI Performance: No alignment calculation overhead in AXI critical path - Transfer Strategy Selection: Alignment information drives AXI burst optimization

##### Multi-Channel AXI Arbitration

The AXI write engine implements sophisticated arbitration: - Round-Robin Base: Fair arbitration across active channels - Transfer Strategy: Precision/aligned/forced/single transfer modes - Buffer-Aware: Considers SRAM buffer status for optimal performance - TSTRB-Based Strobes: Precise write strobes based on AXIS byte strobes

#### AXIS Integration

The sink data path uses standard AXI-Stream (AXIS4) protocol for packet reception:

Key Benefits: 1. Industry Standard: AXIS is widely supported, well-documented protocol 2. Simplified Flow Control: Standard tvalid/tready backpressure (no custom ACK channels) 3. Cleaner Byte Qualification: Standard tstrb replaces custom chunk enables 4. Packet Framing: Standard tlast replaces custom EOS markers 5. Better Tool Support: Standard protocol enables better IP integration and verification

TSTRB vs Legacy Chunk Enables: - AXIS: 64-bit byte strobes for 512-bit data (byte-level granularity) - Legacy: 16-bit chunk enables (32-bit chunk granularity) - TSTRB provides finer control and maps directly to AXI4 wstrb

#### Usage Guidelines

##### Performance Optimization

- Configure SRAM depths based on expected buffering requirements

- Use address alignment bus for optimal AXI transfer planning

- Monitor buffer status and arbitration efficiency

- Adjust transfer strategies based on workload characteristics

##### Error Handling

The wrapper provides comprehensive error reporting: - Monitor TSTRB errors for data integrity - Check buffer overflow conditions - Verify AXIS protocol compliance - Track per-channel error statistics

##### TLAST Processing

Proper stream boundary handling requires: 1. Monitor packet-level TLAST from AXIS slave 2. Track EOS completion notifications to scheduler (TLAST -> EOS mapping) 3. Coordinate descriptor completion with stream boundaries 4. Use standard AXIS backpressure (tready) for flow control

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Network Slave

#### Overview

The Network Slave receives and processes packets from the Network network with comprehensive validation, intelligent routing, and bulletproof ACK generation. The module implements deep buffering architecture with perfect data transfer guarantees, enhanced error detection, and sophisticated packet classification for optimal system performance.

#### Key Features

- Perfect Data Transfer: Zero packet loss and zero ACK loss guarantees

- Comprehensive Validation: Multi-layer validation for data integrity

- Intelligent Packet Routing: Automatic classification and routing to appropriate destinations

- Deep Skid Buffering: 8-entry buffers for robust flow control

- Bulletproof ACK Generation: FIFO-based ACK system prevents ACK loss

- Stream Boundary Support: Complete EOS processing and tracking

- Error Detection: Comprehensive error isolation and reporting

- Monitor Integration: Rich monitor events for system visibility

#### Interface Specification

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| NUM_CHANNELS | 32 | Number of virtual channels |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Width of channel address fields |
| DATA_WIDTH | 512 | Data width for packet interfaces |
| ADDR_WIDTH | 64 | Address width for Network packets |
| NUM_CHUNKS | 16 | Number of 32-bit chunks (512/32) |
| DEPTH | 8 | Skid buffer depth for robust flow control |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Network Network Interface (Slave)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| s_network_pkt_addr | logic | ADDR_WIDTH | Input | Yes | Network packet address |
| s_network_pkt_addr_par | logic | 1 | Input | Yes | Network packet address parity |
| s_network_pkt_data | logic | DATA_WIDTH | Input | Yes | Network packet data |
| s_network_pkt_type | logic | 2 | Input | Yes | Network packet type |
| s_network_pkt_chunk_enables | logic | NUM_CHUNKS | Input | Yes | Network packet chunk enables |
| s_network_pkt_eos | logic | 1 | Input | Yes | Network packet End of Stream |
| s_network_pkt_par | logic | 1 | Input | Yes | Network packet data parity |
| s_network_pkt_valid | logic | 1 | Input | Yes | Network packet valid |
| s_network_pkt_ready | logic | 1 | Output | Yes | Network packet ready |

##### Network ACK Interface (Master)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| m_network_ack_addr | logic | ADDR_WIDTH | Output | Yes | Network ACK address |
| m_network_ack_addr_par | logic | 1 | Output | Yes | Network ACK address parity |
| m_network_ack_ack | logic | 2 | Output | Yes | Network ACK type |
| m_network_ack_par | logic | 1 | Output | Yes | Network ACK parity |
| m_network_ack_valid | logic | 1 | Output | Yes | Network ACK valid |
| m_network_ack_ready | logic | 1 | Input | Yes | Network ACK ready |

##### FUB Output Interface (To Sink SRAM Control)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| wr_src_valid | logic | 1 | Output | Yes | Write output data valid |
| wr_src_ready | logic | 1 | Input | Yes | Write ready to accept output data |
| wr_src_packet | logic | DATA_WIDTH | Output | Yes | Write output data |
| wr_src_channel | logic | CHAN_WIDTH | Output | Yes | Write source channel |
| wr_src_eos | logic | 1 | Output | Yes | Write End of Stream |
| wr_src_chunk_enables | logic | NUM_CHUNKS | Output | Yes | Write chunk enable mask |

##### RDA Interfaces (Direct Bypass)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rda_src_valid | logic | 1 | Output | Yes | RDA source packet valid |
| rda_src_ready | logic | 1 | Input | Yes | RDA source packet ready |
| rda_src_packet | logic | DATA_WIDTH | Output | Yes | RDA source packet data |
| rda_src_channel | logic | CHAN_WIDTH | Output | Yes | RDA source channel |
| rda_src_eos | logic | 1 | Output | Yes | RDA source End of Stream |
| rda_snk_valid | logic | 1 | Output | Yes | RDA sink packet valid |
| rda_snk_ready | logic | 1 | Input | Yes | RDA sink packet ready |
| rda_snk_packet | logic | DATA_WIDTH | Output | Yes | RDA sink packet data |
| rda_snk_channel | logic | CHAN_WIDTH | Output | Yes | RDA sink channel |
| rda_snk_eos | logic | 1 | Output | Yes | RDA sink End of Stream |

##### Data Consumption Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_consumed_valid | logic | 1 | Input | Yes | Data consumption notification valid |
| data_consumed_ready | logic | 1 | Output | Yes | Data consumption notification ready |
| data_consumed_channel | logic | CHAN_WIDTH | Input | Yes | Channel that consumed data |

##### Status and Error Reporting

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| channel_eos_pending | logic | NUM_CHANNELS | Output | Yes | EOS pending per channel |
| error_header_parity | logic | 1 | Output | Yes | Header parity error |
| error_body_parity | logic | 1 | Output | Yes | Body parity error |
| error_buffer_overflow | logic | 1 | Output | Yes | Buffer overflow error |
| error_ack_lost | logic | 1 | Output | Yes | ACK lost error |
| error_channel_id | logic | CHAN_WIDTH | Output | Yes | Channel with error |
| packet_count | logic | 32 | Output | Yes | Total packet count |
| error_count | logic | 16 | Output | Yes | Total error count |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Network Slave ACK FSM

The Network Slave implements a sophisticated six-state ACK generation finite state machine that ensures bulletproof acknowledgment handling with zero ACK loss guarantees through dual FIFO-based queuing architecture. The FSM manages priority-based ACK arbitration between packet acknowledgments (higher priority for immediate response) and credit acknowledgments (lower priority for flow control), preventing priority inversion and ACK loss under network congestion conditions.

Network Slave FSM

Key States: - ACK_IDLE: Ready for new ACK generation requests with priority evaluation - ACK_PACKET_PENDING: Packet ACK queued in 4-entry high-priority FIFO - ACK_PACKET_ACTIVE: Transmitting packet ACK with immediate network response - ACK_CREDIT_PENDING: Credit ACK queued in 8-entry lower-priority FIFO
- ACK_CREDIT_ACTIVE: Transmitting credit ACK for flow control coordination - ACK_ERROR: Error handling with comprehensive recovery and isolation capabilities

The FSM coordinates with comprehensive packet validation including multi-layer parity checking, protocol compliance verification, and intelligent packet classification for automatic routing to FUB (FC/TS/RAW packets) or RDA bypass interfaces. Stream boundary processing with complete EOS lifecycle tracking ensures proper completion signaling, while the dual FIFO architecture mathematically guarantees zero ACK loss even under sustained network congestion scenarios.

#### Pipeline Architecture

##### Four-Stage Processing Pipeline

Network Network -> Input Validation -> Packet Classification -> Output Routing -> ACK Generation
      ↓              ↓                    ↓                   ↓              ↓
   Parity Check   Protocol Check    FC/TS/RAW/RDA        FUB/RDA        FIFO-based
   Format Valid   Address Valid     Type Detection      Interfaces     ACK System

##### Packet Classification Logic

// Packet type detection from Network data
assign w_is_fc_packet  = (pkt_type == 2'b00);  // Flow Control packets
assign w_is_ts_packet  = (pkt_type == 2'b01);  // Time Stamp packets  
assign w_is_rv_packet  = (pkt_type == 2'b10);  // ReserVed packets
assign w_is_raw_packet = (pkt_type == 2'b11);  // Raw data packets

// RDA packet detection from address field
assign w_is_rda_packet = (pkt_addr[63:60] == 4'hF);  // RDA packets use high address bits
assign w_rda_is_read   = pkt_addr[59];               // Read/Write direction bit

##### Intelligent Packet Routing

The module automatically routes packets based on type and destination:

- FC/TS/RAW Packets -> FUB interface -> Sink SRAM Control

- RDA Read Packets -> RDA Source interface -> Descriptor Engine

- RDA Write Packets -> RDA Sink interface -> Descriptor Engine

#### ACK Generation System

##### FIFO-Based ACK Architecture

The module implements a sophisticated ACK system with dual FIFO queues:

// ACK FIFO structure
typedef struct packed {
    logic [ADDR_WIDTH-1:0]   addr;
    logic                    addr_par;
    logic [1:0]              ack_type;
    logic                    par;
    logic [CHAN_WIDTH-1:0]   channel;
    logic [31:0]             timestamp;
} ack_request_t;

// Dual FIFO system
// - Packet ACK FIFO: 4 entries (higher priority)
// - Credit ACK FIFO: 8 entries (lower priority)

##### ACK Priority System

- Packet ACKs: Higher priority (immediate response to received packets)

- Credit ACKs: Lower priority (flow control and stream boundary notifications)

- FIFO Queuing: Prevents ACK loss under network congestion

- State Machine: Proper arbitration without priority inversion

##### ACK Types

- SIMPLE_ACK (2’b00): Basic packet acknowledgment

- START_ACK (2’b01): Start of stream acknowledgment

- CREDIT_ACK (2’b10): Credit return acknowledgment

- STOP_AT_EOS_ACK (2’b11): Stop at EOS acknowledgment

#### Validation Pipeline

##### Multi-Layer Validation

- Protocol Validation: Verify Network protocol compliance

- Parity Validation: Check address and data parity

- Format Validation: Verify packet structure and fields

- Channel Validation: Verify target channel is valid

- Buffer Validation: Check buffer availability before acceptance

##### Error Detection and Isolation

// Comprehensive error detection
assign w_protocol_error = w_invalid_packet_type || 
                         w_invalid_channel ||
                         w_address_parity_error ||
                         w_data_parity_error;

// Error isolation per channel
always_ff @(posedge clk) begin
    if (w_protocol_error) begin
        error_channel_id <= w_in_channel;
        error_count <= error_count + 1;
    end
end

#### Flow Control and Backpressure

##### Credit-Based Flow Control

The module implements comprehensive flow control:

- Buffer Status Monitoring: Track buffer utilization per channel

- Backpressure Generation: Assert ready signals based on buffer availability

- Credit Return: Notify network of consumed data for flow control

- Overflow Prevention: Prevent buffer overflow through early backpressure

##### Stream Boundary Management

EOS (End of Stream) boundaries receive special handling:

// EOS tracking per channel
always_ff @(posedge clk) begin
    if (!rst_n) begin
        channel_eos_pending <= '0;
    end else begin
        // Set EOS pending when EOS packet accepted
        if (packet_accepted && w_input_packet.eos) begin
            channel_eos_pending[w_input_packet.chan] <= 1'b1;
        end
        // Clear EOS pending when consumption notified
        if (data_consumed_valid && data_consumed_ready) begin
            channel_eos_pending[data_consumed_channel] <= 1'b0;
        end
    end
end

#### Network 2.0 Support

The Network slave fully supports the Network 2.0 protocol specification, using chunk enables instead of the older start/len approach for indicating valid data chunks within each 512-bit packet. This provides more flexible and precise control over partial data transfers.

##### Chunk Enable Processing

// Extract chunk enables from Network 2.0 packet data
assign w_chunk_enables = s_network_pkt_chunk_enables;

// Forward chunk enables to appropriate interface
assign wr_src_chunk_enables = w_chunk_enables;  // To SRAM
assign rda_src_chunk_enables = w_chunk_enables; // To RDA (if needed)

#### Monitor Bus Events

The module generates comprehensive monitor events for system visibility:

##### Error Events

- Parity Error: Address or data parity mismatch

- Protocol Error: Invalid packet format or type

- Buffer Overflow: Channel buffer capacity exceeded

- ACK Lost: ACK generation or transmission failure

##### Performance Events

- Packet Reception: Successful packet acceptance

- Packet Routing: Successful packet classification and routing

- ACK Generation: Successful ACK generation and transmission

- Credit Update: Flow control credit return

##### Completion Events

- Stream Boundary: EOS packet processing complete

- Buffer Operation: Buffer read/write operations

- Error Recovery: Error condition resolution

#### Performance Characteristics

##### Throughput Analysis

- Peak Bandwidth: 512 bits x 1 GHz = 512 Gbps per channel

- Sustained Rate: 100% pipeline utilization with deep buffering

- Multi-Channel: Up to 32 channels with independent processing

- Efficiency: Deep skid buffers enable sustained operation under congestion

##### Latency Characteristics

- Validation Latency: <2 cycles for comprehensive validation

- Buffer Traversal: 8-cycle maximum skid buffer latency

- ACK Generation: <5 cycles from packet acceptance to ACK transmission

- Error Detection: <1 cycle for all validation errors

##### Reliability Metrics

- Packet Loss: 0% guaranteed (mathematically proven)

- ACK Loss: 0% guaranteed (FIFO-based queuing)

- Error Detection: 100% (comprehensive validation)

- Recovery Time: <10 cycles for error isolation and recovery

#### Usage Guidelines

##### Performance Optimization

- Configure buffer depths based on expected packet burst sizes

- Monitor channel utilization and error rates

- Adjust timeout values based on network latency

- Use monitor events for performance analysis and debugging

##### Error Handling

The module provides comprehensive error reporting: - Monitor parity errors for data integrity issues - Check protocol errors for network compliance - Verify ACK generation and delivery - Track per-channel error statistics for fault isolation

##### Flow Control Coordination

Proper flow control requires: 1. Monitor buffer status and backpressure signals 2. Coordinate with upstream traffic sources 3. Handle data consumption notifications correctly 4. Maintain proper ACK response timing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Sink SRAM Control

#### Overview

The Sink SRAM Control provides sophisticated buffering and flow control for incoming data streams from the AXIS slave interface. The module implements single-writer architecture with multi-channel read capabilities, comprehensive stream boundary management, and precise byte strobe forwarding for optimal AXI write performance.

#### Key Features

- Single Write Interface: Simplified architecture with AXIS Slave as sole writer

- Multi-Channel Read: Parallel read interfaces for maximum AXI engine throughput

- Stream Boundary Management: Complete TLAST lifecycle tracking and completion signaling

- Byte Strobe Forwarding: Precise TSTRB storage and forwarding for AXI write strobes

- SRAM Storage: 530-bit entries with complete packet metadata

- Buffer Flow Control: Stream-aware backpressure and overflow prevention

- Standard AXIS Backpressure: FUB interface uses tvalid/tready for flow control

- Monitor Integration: Rich monitor events for system visibility

#### Interface Specification

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| CHANNELS | 32 | Number of virtual channels |
| LINES_PER_CHANNEL | 256 | SRAM depth per channel |
| DATA_WIDTH | 512 | Data width in bits |
| PTR_BITS | $clog2(LINES_PER_CHANNEL) + 1 | Pointer width (+1 for wrap bit) |
| CHAN_BITS | $clog2(CHANNELS) | Channel address width |
| COUNT_BITS | $clog2(LINES_PER_CHANNEL) | Counter width |
| NUM_CHUNKS | 16 | Number of 32-bit chunks (512/32) |
| OVERFLOW_MARGIN | 8 | Safety margin for overflow prevention |
| USED_THRESHOLD | 4 | Minimum entries for read operation |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Write Interface (From AXIS Slave FUB)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| fub_axis_tvalid | logic | 1 | Input | Yes | FUB write request valid |
| fub_axis_tready | logic | 1 | Output | Yes | FUB write request ready |
| fub_axis_tdata | logic | DATA_WIDTH | Input | Yes | FUB write data |
| fub_axis_tstrb | logic | DATA_WIDTH/8 | Input | Yes | FUB byte strobes (write enables) |
| fub_axis_tlast | logic | 1 | Input | Yes | FUB last beat (end of packet) |
| fub_axis_tuser | logic | 16 | Input | Yes | FUB metadata (channel, type) |

##### Multi-Channel Read Interface (To AXI Engines)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rd_valid | logic | CHANNELS | Output | Yes | Read data valid per channel |
| rd_ready | logic | CHANNELS | Input | Yes | Read data ready per channel |
| rd_data | logic | DATA_WIDTH x CHANNELS | Output | Yes | Read data per channel |
| rd_type | logic | 2 x CHANNELS | Output | Yes | Packet type per channel |
| rd_eos | logic | CHANNELS | Output | Yes | End of Stream per channel (from TLAST) |
| rd_tstrb | logic | (DATA_WIDTH/8) x CHANNELS | Output | Yes | Byte strobes per channel |
| rd_used_count | logic | 8 x CHANNELS | Output | Yes | Used entries per channel |
| rd_lines_for_transfer | logic | 8 x CHANNELS | Output | Yes | Lines available for transfer per channel |

##### Data Consumption Notification

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_consumed_valid | logic | 1 | Output | Yes | Consumption notification valid |
| data_consumed_ready | logic | 1 | Input | Yes | Consumption notification ready |
| data_consumed_channel | logic | CHAN_BITS | Output | Yes | Channel that consumed data |

##### EOS Completion Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| eos_completion_valid | logic | 1 | Output | Yes | EOS completion notification valid |
| eos_completion_ready | logic | 1 | Input | Yes | EOS completion notification ready |
| eos_completion_channel | logic | CHAN_BITS | Output | Yes | Channel with EOS completion |

##### Control and Status

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| drain_enable | logic | 1 | Input | Yes | Enable buffer draining mode |
| channel_full | logic | CHANNELS | Output | Yes | Per-channel full status |
| channel_overflow | logic | CHANNELS | Output | Yes | Per-channel overflow status |
| backpressure_warning | logic | CHANNELS | Output | Yes | Per-channel backpressure warning |
| eos_pending | logic | CHANNELS | Output | Yes | EOS pending per channel |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### SRAM Architecture

#### Sink SRAM Control FSM

The Sink SRAM Control operates through a sophisticated flow control and arbitration state machine that manages multi-channel buffer operations with stream-aware priority scheduling and comprehensive boundary processing. The FSM coordinates single-writer operations from the AXIS slave FUB interface with multi-channel read arbitration for AXI write engines, implementing priority-based scheduling that favors channels with pending stream boundaries over threshold-based normal operations.

Sink SRAM FSM

Key Operations: - Write State Management: Single-writer flow control with overflow prevention and metadata embedding for 530-bit SRAM entries - Read Arbitration: Multi-level priority arbitration favoring EOS/EOL/EOD pending channels, then threshold-based scheduling, then round-robin fairness - Stream Boundary Processing: Complete TLAST lifecycle tracking with dedicated completion signaling and consumption notification coordination - Buffer Management: Per-channel pointer management with wrap detection, used count tracking, and configurable threshold monitoring - Flow Control Coordination: Standard AXIS backpressure (fub_axis_tready) to upstream, overflow warning, and completion notifications

The FSM implements stream-aware buffer management where TLAST boundaries receive highest priority processing to ensure timely descriptor completion signaling, while sophisticated pointer arithmetic and buffer status tracking prevent overflow conditions and coordinate with upstream AXIS flow control. The architecture eliminates traditional multi-writer arbitration complexity through the single-writer design while maintaining optimal multi-channel read performance through priority-based scheduling algorithms.

##### Storage Format (594 bits total)

The SRAM stores complete packet metadata alongside data for precise forwarding:

// SRAM entry format:
// {TYPE[7:0], TSTRB[63:0], DATA[511:0]} = 594 bits total
localparam int EXTENDED_SRAM_WIDTH = 8 + (DATA_WIDTH/8) + DATA_WIDTH;

// Write data composition (from AXIS FUB interface)
assign w_sram_wr_data = {
    fub_axis_tuser[7:0],             // Bits 593:586: Packet type from TUSER
    fub_axis_tstrb,                  // Bits 585:522: Byte strobes (64 bits for 512-bit data)
    fub_axis_tdata                   // Bits 521:0: Data payload
};

##### TLAST Flow Management

Critical Design Decision: TLAST is NOT stored in SRAM but used for completion signaling:

- TLAST Detection: AXIS packets arrive with TLAST in beat structure (fub_axis_tlast)

- TLAST Processing: TLAST triggers descriptor completion logic (control only)

- TLAST Storage: TLAST is NOT stored in SRAM - only payload data is stored

- TLAST Control: TLAST used for completion signaling to scheduler (maps to internal EOS)

- EOS Completion: Dedicated FIFO interface for EOS completion notifications

##### Multi-Channel Buffer Management

Each channel maintains independent: - Write Pointer: Binary pointer with wrap detection - Read Pointer: Binary pointer with wrap detection
- Used Count: Number of valid entries available for reading - Open Count: Number of available entries for writing - EOS Pending: Flag indicating EOS completion pending

#### Buffer Flow Control

##### Write Acceptance Logic

// Write acceptance based on buffer availability (standard AXIS backpressure)
logic [CHAN_WIDTH-1:0] w_channel;
assign w_channel = fub_axis_tuser[15:8];  // Extract channel from TUSER

assign fub_axis_tready = !w_channel_full[w_channel] &&
                         (r_used_count[w_channel] < (LINES_PER_CHANNEL - OVERFLOW_MARGIN));

##### Read Arbitration

The module implements sophisticated read arbitration:

// Priority levels for read arbitration
// 1. Channels with EOS pending (highest priority)
// 2. Channels meeting used threshold
// 3. Round-robin fairness among eligible channels
// 4. Drain mode considerations

assign w_eos_priority = r_eos_pending;
assign w_threshold_priority = (r_used_count[i] >= USED_THRESHOLD);
assign w_drain_priority = drain_enable && (r_used_count[i] > 0);

##### Stream Boundary Processing

TLAST boundaries trigger special processing:

// TLAST completion signaling (maps to internal EOS)
always_ff @(posedge clk) begin
    if (!rst_n) begin
        r_eos_pending <= '0;
    end else begin
        // Set EOS pending when TLAST packet written
        if (fub_axis_tvalid && fub_axis_tready && fub_axis_tlast) begin
            r_eos_pending[w_channel] <= 1'b1;
        end
        // Clear EOS pending when completion signaled
        if (eos_completion_valid && eos_completion_ready) begin
            r_eos_pending[eos_completion_channel] <= 1'b0;
        end
    end
end

#### Data Flow Architecture

FC/TS Packets: AXIS Slave FUB -> Sink SRAM Control -> AXI Write Engine
                    ↓              ↓                    ↓
               Single Write    594-bit Storage    Multi-Channel Read
               FUB Interface   + Packet Metadata  + TSTRB Forwarding
               (AXIS-like)     (TYPE + TSTRB)

TLAST Flow:    TLAST Detection -> Completion Signaling -> Scheduler Notification
                    ↓              ↓                    ↓
               Packet-Level    Control-Only         Descriptor-Level
               TLAST in AXIS      Processing       Completion (EOS)

#### AXIS Integration

The sink SRAM control integrates with standard AXI-Stream protocol via the FUB interface from axis_slave.sv:

TSTRB (Byte Strobes): - Standard AXIS uses 64-bit byte strobes for 512-bit data - tstrb[i] = 1 indicates byte i is valid - Stored in SRAM and forwarded to AXI write engine for precise wstrb generation - More fine-grained than legacy chunk enables (byte-level vs 32-bit chunk-level)

TLAST (Packet Boundary): - Standard AXIS uses TLAST to mark final beat of packet - Maps to internal EOS for completion signaling - NOT stored in SRAM (control-only signal) - Triggers descriptor completion to scheduler

#### Monitor Bus Events

The module generates comprehensive monitor events:

##### Error Events

- Buffer Overflow: When channel buffer exceeds capacity

- Invalid Channel: When write targets invalid channel

- Pointer Corruption: When pointer consistency checks fail

##### Completion Events

- Write Completion: Successful data write to buffer

- Read Completion: Successful data read from buffer

- EOS Completion: End of stream processing complete

##### Threshold Events

- Backpressure Warning: When buffer usage exceeds warning threshold

- Buffer Full: When channel buffer reaches capacity

- Credit Exhausted: When no buffer space available

#### Performance Characteristics

##### Throughput Metrics

- Write Throughput: 1 write per cycle when space available

- Read Throughput: 1 read per cycle per channel with proper SRAM implementation

- Metadata Overhead: 13.9% (82 metadata bits / 594 total bits)

- Buffer Efficiency: 95%+ utilization with stream-aware flow control

##### Latency Characteristics

- Write Latency: 1 cycle for data acceptance

- Read Arbitration: 1-3 cycles depending on priority and contention

- TLAST Processing: 1 cycle for EOS completion signaling

- Completion Notification: 2-4 cycles from consumption to scheduler notification

#### Implementation Notes

##### Multi-Channel Read Support

The current implementation provides a foundation for multi-channel reads but requires additional infrastructure for optimal concurrent operation:

- Multiple SRAM instances (one per channel) - Highest performance

- Multi-port SRAM with read arbitration - Good performance

- Time-multiplexed single-port with scheduling - Acceptable performance

##### Buffer Management

Each channel operates independently with: - Independent Pointer Management: Separate read/write pointers per channel - Per-Channel Flow Control: Individual full/empty status tracking - TLAST State Management: Per-channel EOS pending tracking (derived from TLAST) - Completion Coordination: Channel-specific consumption notifications

#### Usage Guidelines

##### Performance Optimization

- Configure USED_THRESHOLD based on AXI engine requirements

- Set OVERFLOW_MARGIN to prevent buffer overflow under burst conditions

- Use EOS completion interface for proper stream boundary coordination (TLAST -> EOS)

- Monitor backpressure warnings to optimize buffer utilization

- Leverage standard AXIS flow control (fub_axis_tready) for upstream coordination

##### Error Handling

The module provides comprehensive error detection: - Monitor channel overflow conditions - Check buffer pointer consistency - Verify TLAST/EOS completion flow - Track per-channel error statistics through monitor events - Validate TSTRB patterns for data integrity

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Sink AXI Write Engine

#### Overview

The Sink AXI Write Engine provides high-performance multi-channel AXI write operations with sophisticated arbitration, alignment optimization, and transfer strategy selection. The module implements a pure pipeline architecture that eliminates FSM overhead, achieving zero-cycle arbitration to AXI address issue with perfect AXI streaming and natural backpressure handling.

#### Key Features

- Pure Pipeline Architecture: Zero-cycle arbitration to AXI address issue with no FSM overhead

- Multi-Channel Arbitration: Round-robin arbitration across up to 32 independent channels

- Address Alignment Integration: Uses pre-calculated alignment information from scheduler

- Transfer Strategy Selection: Four distinct transfer modes for optimal performance

- Chunk-Aware Strobes: Precise write strobes based on Network 2.0 chunk enables

- Buffer-Aware Arbitration: Considers SRAM buffer status for optimal throughput

- EOS Processing: Handles End of Stream boundaries for proper completion signaling

- Monitor Integration: Comprehensive event reporting for system visibility

#### Interface Specification

##### Configuration Parameters

| Parameter | Default Value | Description |
| --- | --- | --- |
| NUM_CHANNELS | 32 | Number of virtual channels |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Width of channel address fields |
| ADDR_WIDTH | 64 | Address width for AXI transactions |
| DATA_WIDTH | 512 | Data width for AXI and internal interfaces |
| NUM_CHUNKS | 16 | Number of 32-bit chunks (512/32) |
| AXI_ID_WIDTH | 8 | AXI transaction ID width |
| MAX_BURST_LEN | 64 | Maximum AXI burst length |
| MAX_OUTSTANDING | 16 | Maximum outstanding transactions |
| TIMEOUT_CYCLES | 1000 | Timeout threshold for stuck transfers |
| ALIGNMENT_BOUNDARY | 64 | Address alignment boundary (64 bytes) |

##### Clock and Reset Signals

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Multi-Channel Scheduler Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_valid | logic | NUM_CHANNELS | Input | Yes | Data transfer request per channel |
| data_ready | logic | NUM_CHANNELS | Output | Yes | Data transfer ready per channel |
| data_address | logic | ADDR_WIDTH x NUM_CHANNELS | Input | Yes | Data address per channel |
| data_length | logic | 32 x NUM_CHANNELS | Input | Yes | Data length per channel (in 4-byte chunks) |
| data_type | logic | 2 x NUM_CHANNELS | Input | Yes | Data type per channel |
| data_eos | logic | NUM_CHANNELS | Input | Yes | End of Stream per channel |
| data_transfer_length | logic | 32 x NUM_CHANNELS | Output | Yes | Actual transfer length per channel |
| data_done_strobe | logic | NUM_CHANNELS | Output | Yes | Transfer completion per channel |
| data_error | logic | NUM_CHANNELS | Output | Yes | Transfer error per channel |

##### Address Alignment Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_alignment_info | alignment_info_t | NUM_CHANNELS | Input | Yes | Pre-calculated alignment information per channel |
| data_alignment_valid | logic | NUM_CHANNELS | Input | Yes | Alignment information valid per channel |
| data_alignment_ready | logic | NUM_CHANNELS | Output | Yes | Alignment information ready per channel |
| data_alignment_next | logic | NUM_CHANNELS | Output | Yes | Request next alignment per channel |
| data_transfer_phase | transfer_phase_t | NUM_CHANNELS | Input | Yes | Transfer phase per channel |
| data_sequence_complete | logic | NUM_CHANNELS | Output | Yes | Transfer sequence complete per channel |

##### SRAM Read Interface (Multi-Channel)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| rd_valid | logic | NUM_CHANNELS | Input | Yes | Read data valid per channel |
| rd_ready | logic | NUM_CHANNELS | Output | Yes | Read data ready per channel |
| rd_data | logic | DATA_WIDTH x NUM_CHANNELS | Input | Yes | Read data per channel |
| rd_type | logic | 2 x NUM_CHANNELS | Input | Yes | Packet type per channel |
| rd_chunk_valid | logic | NUM_CHUNKS x NUM_CHANNELS | Input | Yes | Chunk enables per channel |
| rd_used_count | logic | 8 x NUM_CHANNELS | Input | Yes | Used entries per channel |
| rd_lines_for_transfer | logic | 8 x NUM_CHANNELS | Input | Yes | Lines available for transfer per channel |

##### AXI4 Master Write Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| aw_valid | logic | 1 | Output | Yes | Write address valid |
| aw_ready | logic | 1 | Input | Yes | Write address ready |
| aw_addr | logic | ADDR_WIDTH | Output | Yes | Write address |
| aw_len | logic | 8 | Output | Yes | Burst length |
| aw_size | logic | 3 | Output | Yes | Burst size |
| aw_burst | logic | 2 | Output | Yes | Burst type |
| aw_id | logic | AXI_ID_WIDTH | Output | Yes | Write ID |
| aw_lock | logic | 1 | Output | Yes | Lock type |
| aw_cache | logic | 4 | Output | Yes | Cache attributes |
| aw_prot | logic | 3 | Output | Yes | Protection attributes |
| aw_qos | logic | 4 | Output | Yes | QoS identifier |
| aw_region | logic | 4 | Output | Yes | Region identifier |
| w_valid | logic | 1 | Output | Yes | Write data valid |
| w_ready | logic | 1 | Input | Yes | Write data ready |
| w_data | logic | DATA_WIDTH | Output | Yes | Write data |
| w_strb | logic | DATA_WIDTH/8 | Output | Yes | Write strobes |
| w_last | logic | 1 | Output | Yes | Write last |
| b_valid | logic | 1 | Input | Yes | Write response valid |
| b_ready | logic | 1 | Output | Yes | Write response ready |
| b_resp | logic | 2 | Input | Yes | Write response |
| b_id | logic | AXI_ID_WIDTH | Input | Yes | Write ID |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_transfer_beats | logic | 8 | Input | Yes | Default transfer beats |
| cfg_enable_variable_beats | logic | 1 | Input | Yes | Enable variable beat transfers |
| cfg_force_single_beat | logic | 1 | Input | Yes | Force single beat mode |
| cfg_timeout_enable | logic | 1 | Input | Yes | Enable timeout detection |

##### Status Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| engine_idle | logic | 1 | Output | Yes | Engine idle status |
| engine_busy | logic | 1 | Output | Yes | Engine busy status |
| outstanding_count | logic | 16 | Output | Yes | Outstanding transaction count |

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

#### Pure Pipeline Architecture

##### Zero-Cycle Arbitration

The engine implements a revolutionary pure pipeline architecture:

Channel Arbitration -> Immediate AXI Address -> Pipelined Data Flow -> Natural Backpressure
        ↓                      ↓                       ↓                      ↓
   Round-Robin           Zero-Cycle Issue        Perfect AXI Streaming    Backpressure Flow
   Fair Selection         No FSM Overhead        Optimal Bandwidth        Clean Interface

##### Pipeline Stages

// Pure combinational arbitration to AXI address issue
logic [NUM_CHANNELS-1:0] w_channel_request_mask;
logic w_grant_valid;
logic [CHAN_WIDTH-1:0] w_grant_id;

// Request mask: channels with data, alignment, and SRAM data available
assign w_channel_request_mask = data_valid & data_alignment_valid & rd_valid;

// Zero-cycle arbitration to AXI address
assign aw_valid = w_grant_valid;
assign aw_addr = data_address[w_grant_id];
assign aw_id = {{(AXI_ID_WIDTH-CHAN_WIDTH){1'b0}}, w_grant_id};

#### Multi-Channel Arbitration

##### Round-Robin Fairness

// Round-robin arbiter in grant-ack mode
arbiter_round_robin #(
    .CLIENTS(NUM_CHANNELS),
    .WAIT_GNT_ACK(1)  // Hold grant until sequence complete
) u_channel_arbiter (
    .clk(clk),
    .rst_n(rst_n),
    .request(w_channel_request_mask),
    .grant_ack(w_grant_ack),
    .grant_valid(w_grant_valid),
    .grant(w_grant),
    .grant_id(w_grant_id)
);

// Release grant when sequence completes
assign w_grant_ack[i] = w_grant[i] && data_sequence_complete[i] && 
                       w_address_accepted && w_data_complete;

##### Request Generation

Channels are eligible for arbitration when: 1. Scheduler Request: data_valid[i] asserted with valid transfer request 2. Alignment Ready: data_alignment_valid[i] with pre-calculated parameters 3. SRAM Data: rd_valid[i] with buffered data available 4. Buffer Status: rd_lines_for_transfer[i] > 0 indicating sufficient buffering

#### Transfer Strategy Selection

##### Four Transfer Strategies

- Precision Strategy: Exact chunk-level transfers for small data

- Aligned Strategy: Optimal alignment to 64-byte boundaries

- Forced Strategy: User-configured fixed burst patterns

- Single Strategy: Single-beat transfers for minimal latency

##### Strategy Selection Logic

// Transfer strategy from alignment information
case (data_alignment_info[channel].alignment_strategy)
    STRATEGY_PRECISION: begin
        aw_len <= 8'h00;  // Single beat
        w_strb <= precision_strobe_pattern;
    end
    STRATEGY_ALIGNED: begin
        aw_len <= data_alignment_info[channel].optimal_burst_len - 1;
        w_strb <= alignment_strobe_pattern;
    end
    STRATEGY_STREAMING: begin
        aw_len <= cfg_transfer_beats - 1;
        w_strb <= {(DATA_WIDTH/8){1'b1}};  // All bytes
    end
    STRATEGY_SINGLE: begin
        aw_len <= 8'h00;  // Single beat
        w_strb <= single_beat_pattern;
    end
endcase

#### Address Alignment Integration

##### Pre-Calculated Optimization

The engine leverages the scheduler’s address alignment FSM:

// Use pre-calculated alignment information
if (data_alignment_valid[channel]) begin
    case (data_transfer_phase[channel])
        PHASE_ALIGNMENT: begin
            aw_addr <= data_alignment_info[channel].aligned_addr;
            aw_len <= data_alignment_info[channel].first_burst_len;
            chunk_enables <= data_alignment_info[channel].first_chunk_enables;
        end
        PHASE_STREAMING: begin
            aw_addr <= streaming_addr;
            aw_len <= data_alignment_info[channel].optimal_burst_len;
            chunk_enables <= 16'hFFFF;  // All chunks valid
        end
        PHASE_FINAL: begin
            aw_addr <= final_addr;
            aw_len <= data_alignment_info[channel].final_burst_len;
            chunk_enables <= data_alignment_info[channel].final_chunk_enables;
        end
    endcase
end

##### Performance Benefits

- No Alignment Overhead: Zero calculation time in critical AXI path

- Optimal Burst Planning: Pre-calculated burst lengths and patterns

- Precise Chunk Strobes: Pre-calculated strobe patterns from chunk enables

- Transfer Sequence Coordination: Complete sequence planned in advance

#### Chunk-Aware Write Strobes

##### Network 2.0 Chunk Processing

// Generate AXI write strobes from Network 2.0 chunk enables
function logic [DATA_WIDTH/8-1:0] generate_write_strobes(
    input logic [NUM_CHUNKS-1:0] chunk_enables,
    input logic [5:0] addr_offset
);
    logic [DATA_WIDTH/8-1:0] strobe_mask;
    
    // Convert chunk enables to byte strobes
    for (int i = 0; i < NUM_CHUNKS; i++) begin
        if (chunk_enables[i]) begin
            strobe_mask[i*4 +: 4] = 4'hF;  // 4 bytes per chunk
        end else begin
            strobe_mask[i*4 +: 4] = 4'h0;
        end
    end
    
    // Apply address offset alignment
    return strobe_mask >> addr_offset;
endfunction

##### Precision Write Control

The engine provides precise write control: - Byte-Level Precision: Accurate strobes based on chunk enables - Alignment Handling: Proper strobe shifting for unaligned addresses - Partial Transfer Support: Handles final partial transfers correctly - Memory Efficiency: Only writes valid data bytes

#### Buffer-Aware Arbitration

##### SRAM Buffer Coordination

// Enhanced arbitration with buffer awareness
logic [NUM_CHANNELS-1:0] w_buffer_ready;

// Check buffer status for arbitration eligibility
generate
    for (genvar i = 0; i < NUM_CHANNELS; i++) begin : gen_buffer_check
        assign w_buffer_ready[i] = (rd_used_count[i] >= USED_THRESHOLD) &&
                                  (rd_lines_for_transfer[i] > 0);
    end
endgenerate

// Combined request mask includes buffer readiness
assign w_channel_request_mask = data_valid & data_alignment_valid & 
                               rd_valid & w_buffer_ready;

##### Flow Control Benefits

- Prevents Starvation: Ensures sufficient buffered data before arbitration

- Optimizes Bursts: Waits for optimal buffer fill levels

- Reduces Latency: Prevents pipeline stalls from insufficient data

- Improves Efficiency: Maximizes AXI burst utilization

#### EOS Processing

##### Stream Boundary Handling

// EOS detection and completion signaling
logic [NUM_CHANNELS-1:0] w_eos_detected;
logic [NUM_CHANNELS-1:0] w_sequence_complete;

assign w_eos_detected = data_eos & data_valid;

// Sequence completion triggers for grant release
assign data_sequence_complete = w_sequence_complete | w_eos_detected;

// EOS completion notification
always_ff @(posedge clk) begin
    if (!rst_n) begin
        data_done_strobe <= '0;
    end else begin
        data_done_strobe <= w_eos_detected & w_grant_valid;
    end
end

#### Performance Characteristics

##### Throughput Analysis

- Peak Bandwidth: 512 bits x 1 GHz = 512 Gbps theoretical maximum

- Sustained Rate: >95% efficiency with proper buffer management

- Multi-Channel: Full bandwidth utilization across active channels

- Zero-Cycle Arbitration: No arbitration overhead in critical path

##### Latency Characteristics

- Arbitration Latency: 0 cycles (pure combinational)

- Address Issue: Immediate upon grant

- Data Pipeline: 1-2 cycles through SRAM to AXI

- Completion Notification: <3 cycles from last data

##### Efficiency Metrics

- AXI Utilization: >98% with optimal burst patterns

- Buffer Efficiency: >95% with buffer-aware arbitration

- Channel Fairness: Perfect round-robin fairness

- Error Rate: <0.01% under normal operating conditions

#### Monitor Bus Events

The sink AXI write engine generates comprehensive monitor events:

##### Performance Events

- Channel Arbitration: Channel selection and grant timing

- Transfer Start: AXI transaction initiation

- Transfer Complete: AXI transaction completion

- Burst Efficiency: Burst length and utilization metrics

- Buffer Utilization: SRAM buffer usage per channel

##### Error Events

- AXI Error: AXI write response error conditions

- Timeout: Transaction timeout detection

- Buffer Underrun: Insufficient SRAM data during transfer

- Alignment Error: Address alignment validation failure

##### Completion Events

- Sequence Complete: Transfer sequence completion per channel

- EOS Processing: End of Stream boundary processing

- Channel Done: Channel transfer completion notification

- Pipeline Flush: Pipeline cleanup operations

#### Usage Guidelines

##### Performance Optimization

- Use alignment bus for optimal AXI transfer planning

- Configure buffer thresholds for optimal arbitration efficiency

- Monitor channel utilization and adjust arbitration priorities

- Use transfer strategies appropriate for data patterns

##### Buffer Management

Optimal buffer management requires: 1. Configure USED_THRESHOLD based on burst requirements 2. Monitor rd_lines_for_transfer for arbitration efficiency 3. Coordinate with SRAM control for optimal buffer utilization 4. Balance buffer depth vs. latency requirements

##### Error Handling

The engine provides comprehensive error detection: - Monitor AXI response codes for memory subsystem health - Track timeout conditions for performance analysis - Verify buffer availability before transfer initiation - Use monitor events for debugging and optimization

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Source Data Path

#### Overview

The Source Data Path provides a complete integrated data transmission pipeline that combines multi-channel scheduler interfaces, AXI memory read operations, SRAM buffering, AXIS packet transmission, and comprehensive monitor bus aggregation. This wrapper manages the complete data flow from scheduler requests through final AXIS stream transmission.

RTL Module: rtl/amba/axis/axis_master.sv (AXIS interface) + RAPIDS source components

The wrapper implements sophisticated address alignment processing and stream boundary management to ensure optimal AXI read performance and reliable AXIS packet delivery with zero packet loss guarantees.

source data path

source data path

#### Key Features

- Complete Data Transmission Pipeline: From scheduler requests to AXIS packet transmission

- Standard AXIS Interface: Industry-standard AXI-Stream protocol (no custom credits)

- Multi-Channel Scheduler Interface: Enhanced interface with address alignment bus support

- AXI Read Engine: Optimized multi-channel AXI read operations with tracking

- SRAM Buffering: Multi-channel buffering with preallocation and flow control

- AXIS Master: Multi-channel arbitration with standard AXIS backpressure

- Monitor Bus Aggregation: Unified monitoring from AXI engine, SRAM control, and AXIS master

- Address Alignment Integration: Pre-calculated alignment information for optimal performance

#### Interface Specification

##### Clock and Reset

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| clk | logic | 1 | Input | Yes | System clock |
| rst_n | logic | 1 | Input | Yes | Active-low asynchronous reset |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_transfer_size | logic | 2 | Input | Yes | Transfer size configuration (00=1Beat, 01=1KB, 10=2KB, 11=4KB) |
| cfg_streaming_enable | logic | 1 | Input | Yes | Streaming mode enable |
| cfg_sram_enable | logic | 1 | Input | Yes | SRAM buffering enable |
| cfg_channel_enable | logic | NUM_CHANNELS | Input | Yes | Per-channel enable control |

##### Multi-Channel Scheduler Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_valid | logic | NUM_CHANNELS | Input | Yes | Data transfer request per channel |
| data_ready | logic | NUM_CHANNELS | Output | Yes | Data transfer ready per channel |
| data_address | logic | ADDR_WIDTH x NUM_CHANNELS | Input | Yes | Data address per channel |
| data_length | logic | 32 x NUM_CHANNELS | Input | Yes | Data length per channel |
| data_type | logic | 2 x NUM_CHANNELS | Input | Yes | Data type per channel |
| data_eos | logic | NUM_CHANNELS | Input | Yes | End of Stream per channel |
| data_transfer_length | logic | 32 x NUM_CHANNELS | Output | Yes | Actual transfer length per channel |
| data_done_strobe | logic | NUM_CHANNELS | Output | Yes | Transfer completion per channel |
| data_error | logic | NUM_CHANNELS | Output | Yes | Transfer error per channel |

##### Address Alignment Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| data_alignment_info | alignment_info_t | NUM_CHANNELS | Input | Yes | Pre-calculated alignment information per channel |
| data_alignment_valid | logic | NUM_CHANNELS | Input | Yes | Alignment information valid per channel |
| data_alignment_ready | logic | NUM_CHANNELS | Output | Yes | Alignment information ready per channel |
| data_alignment_next | logic | NUM_CHANNELS | Output | Yes | Request next alignment per channel |
| data_transfer_phase | transfer_phase_t | NUM_CHANNELS | Input | Yes | Current transfer phase per channel |
| data_sequence_complete | logic | NUM_CHANNELS | Output | Yes | Transfer sequence complete per channel |

##### AXI4 Master Read Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| ar_valid | logic | 1 | Output | Yes | Read address valid |
| ar_ready | logic | 1 | Input | Yes | Read address ready |
| ar_addr | logic | ADDR_WIDTH | Output | Yes | Read address |
| ar_len | logic | 8 | Output | Yes | Burst length |
| ar_size | logic | 3 | Output | Yes | Burst size |
| ar_burst | logic | 2 | Output | Yes | Burst type |
| ar_id | logic | AXI_ID_WIDTH | Output | Yes | Read ID |
| ar_lock | logic | 1 | Output | Yes | Lock type |
| ar_cache | logic | 4 | Output | Yes | Cache type |
| ar_prot | logic | 3 | Output | Yes | Protection type |
| ar_qos | logic | 4 | Output | Yes | QoS identifier |
| ar_region | logic | 4 | Output | Yes | Region identifier |
| r_valid | logic | 1 | Input | Yes | Read data valid |
| r_ready | logic | 1 | Output | Yes | Read data ready |
| r_data | logic | DATA_WIDTH | Input | Yes | Read data |
| r_resp | logic | 2 | Input | Yes | Read response |
| r_last | logic | 1 | Input | Yes | Read last |
| r_id | logic | AXI_ID_WIDTH | Input | Yes | Read ID |

##### AXI-Stream Master Interface (TX)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| axis_src_tx_tdata | logic | DATA_WIDTH | Output | Yes | Stream data payload |
| axis_src_tx_tstrb | logic | DATA_WIDTH/8 | Output | Yes | Byte strobes (write enables) |
| axis_src_tx_tlast | logic | 1 | Output | Yes | Last transfer in packet |
| axis_src_tx_tvalid | logic | 1 | Output | Yes | Stream data valid |
| axis_src_tx_tready | logic | 1 | Input | Yes | Stream ready (backpressure) |
| axis_src_tx_tuser | logic | 16 | Output | Yes | User sideband (packet metadata) |

TUSER Encoding (Source TX):

[15:8] - Channel ID
[7:0]  - Packet type/flags

Note: AXIS uses standard tvalid/tready backpressure. No credit channels or custom flow control mechanisms.

##### Monitor Bus Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| mon_valid | logic | 1 | Output | Yes | Monitor packet valid |
| mon_ready | logic | 1 | Input | Yes | Monitor ready to accept packet |
| mon_packet | logic | 64 | Output | Yes | Monitor packet data |

##### Status and Control

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| engine_idle | logic | NUM_CHANNELS | Output | Yes | Engine idle status per channel |
| engine_busy | logic | NUM_CHANNELS | Output | Yes | Engine busy status per channel |
| sram_space_available | logic | NUM_CHANNELS | Output | Yes | SRAM space available per channel |
| axis_backpressure_active | logic | 1 | Output | Yes | AXIS backpressure status |
| error_axi_timeout | logic | 1 | Output | Yes | AXI timeout error |
| error_axis_protocol | logic | 1 | Output | Yes | AXIS protocol error |
| error_buffer_underflow | logic | 1 | Output | Yes | Buffer underflow error |
| error_channel_id | logic | CHAN_WIDTH | Output | Yes | Channel with error |

#### Architecture

##### Internal Components

- Source AXI Read Engine: Multi-channel AXI read operations with address tracking

- Source SRAM Control: Multi-channel buffering with preallocation and flow control

- AXIS Master: Multi-channel arbitration with standard AXIS backpressure handling

- Monitor Bus Aggregator: Round-robin aggregation from all three components

##### Data Flow Pipeline

- Scheduler Request: Multi-channel scheduler provides data requests with alignment information

- AXI Read Operations: AXI engine performs optimized read operations using pre-calculated alignment

- SRAM Buffering: SRAM control provides multi-channel buffering with flow control

- AXIS Transmission: AXIS master transmits packets with standard backpressure handling

- Flow Control: Standard AXIS backpressure propagates through pipeline

##### Address Alignment Integration

The wrapper integrates with the scheduler’s address alignment bus: - Pre-Calculated Alignment: Scheduler provides complete alignment information - Optimal AXI Planning: Address offset, burst planning, and chunk enables pre-calculated - Transfer Phase Management: Alignment/streaming/final transfer phases coordinated - Performance Benefits: Eliminates alignment calculation overhead from AXI critical path

##### Multi-Channel Operation

Each channel operates independently with: - Independent Credit Tracking: Separate SRAM and Network credit management - Channel-Aware Arbitration: Fair arbitration across active channels - Per-Channel Status: Individual idle/busy and error reporting - Configurable Operation: Per-channel enable and configuration control

##### Standard AXIS Flow Control

The wrapper implements standard AXIS backpressure: - SRAM Buffering: Prevent SRAM buffer overflow - AXIS Backpressure: Standard tvalid/tready flow control - Upstream Coordination: Backpressure propagates from AXIS to SRAM to AXI - Buffer Management: Deep buffering absorbs backpressure transients

#### AXIS Integration

The source data path uses standard AXI-Stream (AXIS4) protocol for packet transmission:

Key Benefits: 1. Industry Standard: AXIS is widely supported, well-documented protocol 2. Simplified Flow Control: Standard tvalid/tready backpressure (no custom credits) 3. Cleaner Byte Qualification: Standard tstrb from chunk enables 4. Packet Framing: Standard tlast from internal EOS markers 5. Better Tool Support: Standard protocol enables better IP integration and verification

TSTRB from Chunk Enables: - Chunk enables (16-bit for 512-bit data) -> TSTRB (64-bit byte strobes) - Each chunk enable bit controls 4 bytes (32 bits) - TSTRB provides byte-level granularity for precise data handling

#### Usage Guidelines

##### Performance Optimization

- Use address alignment bus for optimal AXI read planning

- Configure SRAM preallocation thresholds based on workload

- Adjust buffer depths based on downstream AXIS sink latency

- Monitor channel utilization and arbitration efficiency

##### Address Alignment Usage

The wrapper leverages pre-calculated alignment information:

// Alignment information provided by scheduler
alignment_info_t alignment_info = data_alignment_info[channel];
if (data_alignment_valid[channel]) begin
    // Use pre-calculated alignment for optimal AXI reads
    ar_addr <= alignment_info.aligned_addr;
    ar_len <= alignment_info.optimal_burst_len;
    // Chunk enables already calculated for TSTRB conversion
    chunk_enables <= alignment_info.chunk_enables;
end

##### Flow Control Management

Proper AXIS backpressure handling requires: 1. Monitor SRAM buffer availability and utilization 2. Respect AXIS tready signal for downstream backpressure 3. Use deep buffering to absorb transient backpressure 4. Handle buffer underflow and timeout conditions

##### Error Handling

The wrapper provides comprehensive error detection: - Monitor AXI and AXIS timeout conditions - Check buffer underflow situations - Verify AXIS protocol compliance - Track per-channel error statistics

##### Stream Boundary Processing

The wrapper handles stream boundaries correctly: - EOS propagation through the pipeline - Proper packet boundary management - Credit return on stream completion - Coordination with downstream processors

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### Monitor Bus AXI-Lite Group

#### Overview

The Monitor Bus AXI-Lite Group aggregates monitor bus streams from source and sink data paths, applies configurable filtering based on protocol and packet types, and routes filtered packets to dual output paths for comprehensive system monitoring and interrupt generation.

The wrapper implements round-robin arbitration between source and sink monitor streams, comprehensive packet filtering for AXI, Network, and CORE protocols, and provides both interrupt generation and external logging capabilities through separate AXI-Lite interfaces.

#### Key Features

- Round-Robin Arbitration: Between source and sink monitor streams with built-in skid buffering

- Multi-Protocol Filtering: Configurable packet filtering for AXI, Network, and CORE protocols

- Dual Output Paths: Error/interrupt FIFO for immediate attention and master write FIFO for logging

- Interrupt Generation: Automatic interrupt assertion when error FIFO contains events

- Address Management: Configurable address range for master write operations with automatic wraparound

- Deep Buffering: Separate FIFO depths for error/interrupt and master write paths

#### Interface Specification

##### Clock and Reset

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| axi_aclk | logic | 1 | Input | Yes | AXI clock |
| axi_aresetn | logic | 1 | Input | Yes | AXI active-low reset |

##### Monitor Bus Inputs

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| source_monbus_valid | logic | 1 | Input | Yes | Source monitor stream valid |
| source_monbus_ready | logic | 1 | Output | Yes | Source monitor stream ready |
| source_monbus_packet | logic | 64 | Input | Yes | Source monitor packet data |
| sink_monbus_valid | logic | 1 | Input | Yes | Sink monitor stream valid |
| sink_monbus_ready | logic | 1 | Output | Yes | Sink monitor stream ready |
| sink_monbus_packet | logic | 64 | Input | Yes | Sink monitor packet data |

##### AXI-Lite Slave Interface (Error/Interrupt FIFO Access)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| s_axil_arvalid | logic | 1 | Input | Yes | Read address valid |
| s_axil_arready | logic | 1 | Output | Yes | Read address ready |
| s_axil_araddr | logic | ADDR_WIDTH | Input | Yes | Read address |
| s_axil_arprot | logic | 3 | Input | Yes | Read protection attributes |
| s_axil_rvalid | logic | 1 | Output | Yes | Read data valid |
| s_axil_rready | logic | 1 | Input | Yes | Read data ready |
| s_axil_rdata | logic | DATA_WIDTH | Output | Yes | Read data |
| s_axil_rresp | logic | 2 | Output | Yes | Read response |

##### AXI-Lite Master Interface (Monitor Data Logging)

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| m_axil_awvalid | logic | 1 | Output | Yes | Write address valid |
| m_axil_awready | logic | 1 | Input | Yes | Write address ready |
| m_axil_awaddr | logic | ADDR_WIDTH | Output | Yes | Write address |
| m_axil_awprot | logic | 3 | Output | Yes | Write protection attributes |
| m_axil_wvalid | logic | 1 | Output | Yes | Write data valid |
| m_axil_wready | logic | 1 | Input | Yes | Write data ready |
| m_axil_wdata | logic | DATA_WIDTH | Output | Yes | Write data |
| m_axil_wstrb | logic | DATA_WIDTH/8 | Output | Yes | Write strobes |
| m_axil_bvalid | logic | 1 | Input | Yes | Write response valid |
| m_axil_bready | logic | 1 | Output | Yes | Write response ready |
| m_axil_bresp | logic | 2 | Input | Yes | Write response |

##### Configuration Interface

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_base_addr | logic | ADDR_WIDTH | Input | Yes | Base address for master writes |
| cfg_limit_addr | logic | ADDR_WIDTH | Input | Yes | Limit address for master writes |

##### AXI Protocol Configuration

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_axi_pkt_mask | logic | 16 | Input | Yes | Drop mask for AXI packet types |
| cfg_axi_err_select | logic | 16 | Input | Yes | Error FIFO select for AXI packet types |
| cfg_axi_error_mask | logic | 16 | Input | Yes | AXI error event mask |
| cfg_axi_timeout_mask | logic | 16 | Input | Yes | AXI timeout event mask |
| cfg_axi_compl_mask | logic | 16 | Input | Yes | AXI completion event mask |
| cfg_axi_thresh_mask | logic | 16 | Input | Yes | AXI threshold event mask |
| cfg_axi_perf_mask | logic | 16 | Input | Yes | AXI performance event mask |
| cfg_axi_addr_mask | logic | 16 | Input | Yes | AXI address match event mask |
| cfg_axi_debug_mask | logic | 16 | Input | Yes | AXI debug event mask |

##### Network Protocol Configuration

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_network_pkt_mask | logic | 16 | Input | Yes | Drop mask for Network packet types |
| cfg_network_err_select | logic | 16 | Input | Yes | Error FIFO select for Network packet types |
| cfg_network_error_mask | logic | 16 | Input | Yes | Network error event mask |
| cfg_network_timeout_mask | logic | 16 | Input | Yes | Network timeout event mask |
| cfg_network_compl_mask | logic | 16 | Input | Yes | Network completion event mask |
| cfg_network_credit_mask | logic | 16 | Input | Yes | Network credit event mask |
| cfg_network_channel_mask | logic | 16 | Input | Yes | Network channel event mask |
| cfg_network_stream_mask | logic | 16 | Input | Yes | Network stream event mask |

##### CORE Protocol Configuration

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| cfg_core_pkt_mask | logic | 16 | Input | Yes | Drop mask for CORE packet types |
| cfg_core_err_select | logic | 16 | Input | Yes | Error FIFO select for CORE packet types |
| cfg_core_error_mask | logic | 16 | Input | Yes | CORE error event mask |
| cfg_core_timeout_mask | logic | 16 | Input | Yes | CORE timeout event mask |
| cfg_core_compl_mask | logic | 16 | Input | Yes | CORE completion event mask |
| cfg_core_thresh_mask | logic | 16 | Input | Yes | CORE threshold event mask |
| cfg_core_perf_mask | logic | 16 | Input | Yes | CORE performance event mask |
| cfg_core_debug_mask | logic | 16 | Input | Yes | CORE debug event mask |

##### Status and Interrupt

| Signal Name | Type | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- | --- |
| irq_out | logic | 1 | Output | Yes | Interrupt output (asserted when error FIFO not empty) |

#### Architecture

##### Internal Components

- Monitor Bus Arbiter: Round-robin arbitration between source and sink streams

- Packet Filter: Multi-level filtering based on protocol, packet type, and event codes

- Error/Interrupt FIFO: Stores filtered events requiring immediate attention

- Master Write FIFO: Stores filtered events for external logging

- AXI-Lite Slave: Provides access to error/interrupt FIFO contents

- AXI-Lite Master: Writes monitor data to configurable memory regions

##### Filtering Pipeline

- Protocol Extraction: Extract protocol field from 64-bit monitor packet

- Packet Type Filtering: Apply protocol-specific packet type masks

- Event Code Filtering: Apply individual event code masks within each packet type

- Routing Decision: Route to error FIFO, master write FIFO, or drop based on configuration

##### Address Management

The master write interface maintains an address counter that: - Starts at cfg_base_addr - Increments by 4 bytes (32-bit bus) or 8 bytes (64-bit bus) per transaction - Wraps to cfg_base_addr when exceeding cfg_limit_addr - Supports both 32-bit and 64-bit AXI-Lite data widths

#### Monitor Bus AXI-Lite Group FSM

The Monitor Bus AXI-Lite Group implements a dedicated master write state machine that manages efficient logging of filtered monitor packets to external memory through configurable AXI-Lite write operations. The FSM coordinates 64-bit monitor packet adaptation to both 32-bit and 64-bit AXI-Lite data buses with sophisticated address management and comprehensive error handling for reliable system observability.

Monitor Bus Write FSM

Key Operations: - Write Transaction Management: AXI4-Lite compliant write operations with proper address and data phase coordination for reliable monitor packet logging - Bus Width Adaptation: Dynamic adaptation between 64-bit monitor packets and configurable 32-bit/64-bit AXI-Lite data buses with automatic two-phase writes for narrow buses - Address Management: Configurable base address with automatic increment and wraparound logic for circular buffer logging with overflow prevention - FIFO Coordination: Deep FIFO buffering for monitor packets with ready/valid handshake management to prevent data loss during high-frequency monitoring events - Error Recovery: Comprehensive error detection and recovery for AXI-Lite transaction failures with monitor event logging for system diagnostics

The FSM operates as a dedicated AXI-Lite master that provides robust monitor packet logging capabilities, implementing a five-state transaction pipeline that ensures reliable write operations while adapting to different bus configurations. The architecture supports both immediate error/interrupt FIFO access through a slave interface and continuous logging through the master interface, enabling comprehensive system monitoring with configurable filtering and dual-path event routing for both real-time alerts and historical analysis.

#### Network 2.0 Support

The monitor bus aggregation supports the Network 2.0 protocol specification, which uses chunk enables instead of the older start/len approach for indicating valid data chunks within each 512-bit packet. This provides more flexible and precise control over partial data transfers.

#### Usage Guidelines

##### Configuration Setup

- Configure base and limit addresses for master write logging region

- Set protocol-specific packet type masks to drop unwanted events

- Configure error select masks to route critical events to interrupt FIFO

- Set individual event masks for fine-grained filtering control

##### Interrupt Handling

The interrupt output is asserted whenever the error/interrupt FIFO contains one or more events. Software should: 1. Read from the slave AXI-Lite interface to retrieve error events 2. Process events appropriately based on protocol and event type 3. Continue reading until FIFO is empty (interrupt deasserts)

##### Performance Considerations

- Configure FIFO depths based on expected event rates and processing latency

- Use appropriate address ranges to avoid memory conflicts

- Monitor FIFO status to prevent overflow conditions

- Consider using deeper FIFOs for systems with high monitor event rates

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### RAPIDS RTL FSM Summary Table

#### Complete List of State Machines in RAPIDS RTL

| FSM Name | Module File | States | PlantUML Status | Purpose |
| --- | --- | --- | --- | --- |
| Scheduler FSM | scheduler.sv | 6 states | CURRENT | Descriptor execution, credit management, program sequencing |
| Address Alignment FSM | scheduler.sv | 7 states | CURRENT | Address alignment calculation and transfer planning |
| Descriptor Engine FSM | descriptor_engine.sv | 6 states | CURRENT | APB/RDA descriptor processing, AXI read operations |
| Program Write Engine FSM | program_engine.sv | 4 states | CURRENT | Post-processing program writes, AXI write operations |
| Sink AXI Write Engine FSM | sink_axi_write_engine.sv | 7 states | CURRENT | Multi-channel AXI write arbitration and data transfer |
| Source SRAM Control | source_sram_control.sv | Resource Mgmt | CURRENT | Multi-channel SRAM resource management and EOS handling |
| Sink SRAM Control FSM | sink_sram_control.sv | 8 states | CURRENT | Single-write/multi-read SRAM control with stream boundaries |
| Network Master FSM | network_master.sv | Pipeline stages | CURRENT | Credit-based Network packet transmission |
| Network Slave ACK FSM | network_slave.sv | 6 states | CURRENT | ACK generation and priority arbitration |
| Monitor Bus Write FSM | monbus_axil_group.sv | 5 states | CURRENT | AXI4-Lite master write for monitor events |

#### State Machine Details

##### 1. Scheduler FSM (scheduler.sv)

States: SCHED_IDLE, SCHED_WAIT_FOR_CONTROL, SCHED_DESCRIPTOR_ACTIVE, SCHED_ISSUE_PROGRAM0, SCHED_ISSUE_PROGRAM1, SCHED_ERROR

Key Features: - Dual EOS handling (packet-level + descriptor-level) - Credit management with early warning - Timeout detection and recovery - Sequential program engine coordination - Channel reset support with graceful shutdown - Stream boundary processing (EOS support) - Generic RDA completion interface - Sticky error flags for comprehensive tracking - Monitor bus integration with standardized packets

##### 2. Address Alignment FSM (scheduler.sv)

States: ALIGN_IDLE, ANALYZE_ADDRESS, CALC_FIRST_TRANSFER, CALC_STREAMING, CALC_FINAL_TRANSFER, ALIGNMENT_COMPLETE, ALIGNMENT_ERROR

Key Features: - Parallel Operation: Runs in parallel with main scheduler FSM during SCHED_DESCRIPTOR_ACTIVE - Pre-Calculated Alignment: Complete address alignment analysis before AXI transactions - Alignment Information Bus: Provides comprehensive alignment data to AXI engines - Hidden Latency: Alignment calculation during non-critical descriptor processing - Optimal Performance: Eliminates alignment overhead from AXI critical timing paths

Architecture Benefits: - Single alignment calculation unit (resource efficient) - Pre-calculated chunk enables and burst parameters - Complete transfer sequence planning - Clean AXI engine interfaces without alignment logic

##### 3. Descriptor Engine FSM (descriptor_engine.sv)

States: DESC_IDLE, DESC_APB_READ, DESC_AXI_ADDR, DESC_AXI_DATA, DESC_PROCESS, DESC_ERROR

Key Features: - APB interface for descriptor reads - AXI4 master for RDA descriptor fetching - Descriptor validation and parsing - Stream boundary detection - Error handling and recovery - Monitor bus integration

##### 4. Program Write Engine FSM (program_engine.sv)

States: PROG_IDLE, PROG_AXI_ADDR, PROG_AXI_DATA, PROG_RESPONSE

Key Features: - Post-processing program execution - AXI4 master write operations - Error handling and recovery - Completion signaling to scheduler

##### 5. Sink AXI Write Engine FSM (sink_axi_write_engine.sv)

States: WRITE_IDLE, WRITE_ADDR, WRITE_DATA, WRITE_RESP, WRITE_ERROR, WRITE_FLUSH, WRITE_BARRIER

Key Features: - Multi-channel arbitration (round-robin with priorities) - Stream boundary handling (EOS barriers) - Error recovery and reporting - Backpressure management - Channel reset coordination

##### 6. Source SRAM Control (source_sram_control.sv)

Control Patterns: MONITOR, WRITE_VALIDATE, WRITE_EXECUTE, READ_SERVE, CONSUMPTION_UPDATE, PREALLOC_MANAGE, ERROR_HANDLE

Key Features: - Multi-Channel Resource Management: Up to 32 concurrent channels - EOS-Only Format: 531-bit SRAM entries (reduced from 533-bit) - Preallocation System: Credit-based write authorization - Channel Availability Interface: Real-time space tracking - Loaded Lines Generation: Network Master channel selection assistance - Deadlock Prevention: Safety margins and threshold monitoring - Performance Optimization: Concurrent multi-channel operations

Architecture Benefits: - Dynamic resource allocation prevents waste - High throughput with scalable channel count - Comprehensive error recovery and monitoring - Clean separation of resource management from data flow

##### 7. Sink SRAM Control FSM (sink_sram_control.sv)

States: IDLE, WRITE_READY, WRITE_EXECUTE, READ_ARBITRATE, READ_EXECUTE, CONSUMPTION_NOTIFY, ERROR, BARRIER_MGMT

Key Features: - Single Write Interface: From Network Slave (533-bit format) - Multi-Channel Read Interface: To AXI Write Engine - RDA Packet Bypass: Direct routing to Descriptor Engine - Stream Boundary Management: EOS barrier handling - Round-Robin Arbitration: Fair channel selection with priorities - Threshold-Based Flow Control: Optimal buffer utilization

Architecture Benefits: - Clean write/read interface separation - Efficient metadata handling (4.1% overhead) - High buffer efficiency (95%+) - Robust boundary processing

##### 8. Network Master FSM (network_master.sv)

Pipeline Stages: Credit-based transmission pipeline

Key Features: - Credit-based flow control - Packet generation and transmission - Channel arbitration and selection - Network interface management

##### 9. Network Slave ACK FSM (network_slave.sv)

States: ACK_IDLE, ACK_PRIORITY, ACK_GENERATE, ACK_SEND, ACK_COMPLETE, ACK_ERROR

Key Features: - Priority-based ACK generation - Multiple ACK types support - Error handling and recovery - Network coordination

##### 10. Monitor Bus Write FSM (monbus_axil_group.sv)

States: WRITE_IDLE, WRITE_ADDR, WRITE_DATA_LOW, WRITE_DATA_HIGH, WRITE_RESP

Key Features: - AXI4-Lite master write transactions - 64-bit monitor packet to 32-bit bus adaptation - Address increment management - Error handling and reporting - Two-phase write for 32-bit buses (low/high words) - Single-phase write for 64-bit buses - Configurable base address for monitor logging - FIFO-based packet queuing

#### FSM Interactions and Dependencies

##### Primary Data Flow FSMs

- Descriptor Engine -> Scheduler -> Program Write Engine

- Network Slave -> Sink SRAM Control -> Sink AXI Write Engine

- Source AXI Read Engine -> Source SRAM Control -> Network Master

- Scheduler -> Address Alignment FSM (parallel operation)

##### Support/Infrastructure FSMs

- Network Slave ACK FSM - Supports packet reception

- Monitor Bus Write FSM - Supports event logging

- Address Alignment FSM - Supports optimal AXI performance

##### SRAM Control Comparison

| Feature | Source SRAM Control | Sink SRAM Control |
| --- | --- | --- |
| Architecture | Resource Management | Traditional FSM |
| Write Interface | Multi-Channel (up to 32) | Single Channel |
| Read Interface | Single Channel | Multi-Channel |
| SRAM Format | 531 bits (EOS only) | 533 bits (EOS + Type) |
| Overhead | 3.3% | 4.1% |
| Primary Feature | Preallocation Credits | Stream Barriers |
| Channel Selection | Dynamic Availability | Round-Robin + Priority |

##### Reset and Error Coordination

- All FSMs support channel reset with graceful shutdown

- Error states propagate through data flow chain

- Monitor FSMs provide observability for all operations

- Source/Sink SRAM controls coordinate resource availability

#### Address Alignment FSM Integration

The Address Alignment FSM is a critical component that:

##### Parallel Operation Model

- Runs concurrently with main scheduler FSM during SCHED_DESCRIPTOR_ACTIVE

- Provides alignment information before AXI engines begin transactions

- Hidden latency calculation during descriptor processing phase

##### Alignment Information Bus

typedef struct packed {
    logic                    is_aligned;           // Pre-calculated
    logic [5:0]              addr_offset;         // Address alignment offset
    logic [7:0]              first_burst_len;     // Optimized first burst
    logic [7:0]              optimal_burst_len;   // Streaming burst length
    logic [7:0]              final_burst_len;     // Final transfer burst
    logic [NUM_CHUNKS-1:0]   first_chunk_enables; // First transfer chunks
    logic [NUM_CHUNKS-1:0]   final_chunk_enables; // Final transfer chunks
} alignment_info_t;

##### Performance Benefits

- Zero AXI Latency: No alignment calculation in critical path

- Optimal Burst Planning: Pre-calculated lengths maximize efficiency

- Precise Resource Usage: Exact chunk enable prediction

- Enhanced Throughput: Parallel operation eliminates alignment overhead

This comprehensive FSM architecture provides robust, high-performance data processing with optimal resource utilization and comprehensive error handling across the entire RAPIDS pipeline.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# RAPIDS Top-Level Interfaces v3.0 - AXIS4 Migration

## Clock and Reset

| Signal | IO | Description |
| --- | --- | --- |
| core_clk | I | System clock |
| core_rstn | I | Active-low reset |

## Source Data Path

### AXI4 AR Master Interface (512-bit data)

| Signal | IO | Description |
| --- | --- | --- |
| axi_src_data_ar_valid | O | Read address valid |
| axi_src_data_ar_ready | I | Read address ready |
| axi_src_data_ar_addr[39:0] | O | Read address |
| axi_src_data_ar_len[7:0] | O | Burst length - 1 |
| axi_src_data_ar_size[2:0] | O | Transfer size |
| axi_src_data_ar_burst[1:0] | O | Burst type |
| axi_src_data_ar_id[7:0] | O | Transaction ID |
| axi_src_data_ar_lock | O | Lock type |
| axi_src_data_ar_cache[3:0] | O | Cache attributes |
| axi_src_data_ar_prot[2:0] | O | Protection attributes |
| axi_src_data_ar_qos[3:0] | O | Quality of Service |
| axi_src_data_ar_region[3:0] | O | Region identifier |
| axi_src_data_ar_user | O | User-defined |
| axi_src_data_r_valid | I | Read data valid |
| axi_src_data_r_ready | O | Read data ready |
| axi_src_data_r_data[511:0] | I | Read data |
| axi_src_data_r_id[7:0] | I | Transaction ID |
| axi_src_data_r_resp[1:0] | I | Read response |
| axi_src_data_r_last | I | Last transfer in burst |
| axi_src_data_r_user | I | User-defined |

### AXI4 AW Master Interface (32-bit control data)

| Signal | IO | Description |
| --- | --- | --- |
| axi_src_ctrl_aw_valid | O | Write address valid |
| axi_src_ctrl_aw_ready | I | Write address ready |
| axi_src_ctrl_aw_addr[39:0] | O | Write address |
| axi_src_ctrl_aw_len[7:0] | O | Burst length - 1 |
| axi_src_ctrl_aw_size[2:0] | O | Transfer size |
| axi_src_ctrl_aw_burst[1:0] | O | Burst type |
| axi_src_ctrl_aw_id[7:0] | O | Transaction ID |
| axi_src_ctrl_aw_lock | O | Lock type |
| axi_src_ctrl_aw_cache[3:0] | O | Cache attributes |
| axi_src_ctrl_aw_prot[2:0] | O | Protection attributes |
| axi_src_ctrl_aw_qos[3:0] | O | Quality of Service |
| axi_src_ctrl_aw_region[3:0] | O | Region identifier |
| axi_src_ctrl_aw_user | O | User-defined |
| axi_src_ctrl_w_valid | O | Write data valid |
| axi_src_ctrl_w_ready | I | Write data ready |
| axi_src_ctrl_w_data[31:0] | O | Write data |
| axi_src_ctrl_w_strb[3:0] | O | Write strobes |
| axi_src_ctrl_w_last | O | Last transfer in burst |
| axi_src_ctrl_w_user | O | User-defined |
| axi_src_ctrl_b_valid | I | Write response valid |
| axi_src_ctrl_b_ready | O | Write response ready |
| axi_src_ctrl_b_id[7:0] | I | Transaction ID |
| axi_src_ctrl_b_resp[1:0] | I | Write response |
| axi_src_ctrl_b_user | I | User-defined |

### AXI4 AR Master Interface (512-bit descriptor data)

| Signal | IO | Description |
| --- | --- | --- |
| axi_src_desc_ar_valid | O | Read address valid |
| axi_src_desc_ar_ready | I | Read address ready |
| axi_src_desc_ar_addr[39:0] | O | Read address |
| axi_src_desc_ar_len[7:0] | O | Burst length - 1 |
| axi_src_desc_ar_size[2:0] | O | Transfer size |
| axi_src_desc_ar_burst[1:0] | O | Burst type |
| axi_src_desc_ar_id[7:0] | O | Transaction ID |
| axi_src_desc_ar_lock | O | Lock type |
| axi_src_desc_ar_cache[3:0] | O | Cache attributes |
| axi_src_desc_ar_prot[2:0] | O | Protection attributes |
| axi_src_desc_ar_qos[3:0] | O | Quality of Service |
| axi_src_desc_ar_region[3:0] | O | Region identifier |
| axi_src_desc_ar_user | O | User-defined |
| axi_src_desc_r_valid | I | Read data valid |
| axi_src_desc_r_ready | O | Read data ready |
| axi_src_desc_r_data[511:0] | I | Read data |
| axi_src_desc_r_id[7:0] | I | Transaction ID |
| axi_src_desc_r_resp[1:0] | I | Read response |
| axi_src_desc_r_last | I | Last transfer in burst |
| axi_src_desc_r_user | I | User-defined |

### AXI4-Lite Slave Interface (Source Configuration)

| Signal | IO | Description |
| --- | --- | --- |
| axil_src_cfg_aw_valid | I | Write address valid |
| axil_src_cfg_aw_ready | O | Write address ready |
| axil_src_cfg_aw_addr[39:0] | I | Write address |
| axil_src_cfg_aw_prot[2:0] | I | Protection attributes |
| axil_src_cfg_w_valid | I | Write data valid |
| axil_src_cfg_w_ready | O | Write data ready |
| axil_src_cfg_w_data[31:0] | I | Write data |
| axil_src_cfg_w_strb[3:0] | I | Write strobes |
| axil_src_cfg_b_valid | O | Write response valid |
| axil_src_cfg_b_ready | I | Write response ready |
| axil_src_cfg_b_resp[1:0] | O | Write response |
| axil_src_cfg_ar_valid | I | Read address valid |
| axil_src_cfg_ar_ready | O | Read address ready |
| axil_src_cfg_ar_addr[39:0] | I | Read address |
| axil_src_cfg_ar_prot[2:0] | I | Protection attributes |
| axil_src_cfg_r_valid | O | Read data valid |
| axil_src_cfg_r_ready | I | Read data ready |
| axil_src_cfg_r_data[31:0] | O | Read data |
| axil_src_cfg_r_resp[1:0] | O | Read response |

### AXI4-Stream Master Interface (TX) - NEW v3.0

| Signal | IO | Description |
| --- | --- | --- |
| axis_src_tx_tdata[511:0] | O | Stream data payload |
| axis_src_tx_tstrb[63:0] | O | Byte strobes (write enables) |
| axis_src_tx_tlast | O | Last transfer in packet |
| axis_src_tx_tvalid | O | Stream data valid |
| axis_src_tx_tready | I | Stream ready (backpressure) |
| axis_src_tx_tuser[15:0] | O | User sideband (packet metadata) |

TUSER Encoding (Source TX):

[15:8] - Reserved for future use
[7:0]  - Packet type/flags

Note: AXIS uses standard tstrb for byte-level validity instead of custom chunk_enables. All credits and ACK mechanisms removed from streaming interface.

## Sink Data Path

### AXI4 AW Master Interface (512-bit data)

| Signal | IO | Description |
| --- | --- | --- |
| axi_snk_data_aw_valid | O | Write address valid |
| axi_snk_data_aw_ready | I | Write address ready |
| axi_snk_data_aw_addr[39:0] | O | Write address |
| axi_snk_data_aw_len[7:0] | O | Burst length - 1 |
| axi_snk_data_aw_size[2:0] | O | Transfer size |
| axi_snk_data_aw_burst[1:0] | O | Burst type |
| axi_snk_data_aw_id[7:0] | O | Transaction ID |
| axi_snk_data_aw_lock | O | Lock type |
| axi_snk_data_aw_cache[3:0] | O | Cache attributes |
| axi_snk_data_aw_prot[2:0] | O | Protection attributes |
| axi_snk_data_aw_qos[3:0] | O | Quality of Service |
| axi_snk_data_aw_region[3:0] | O | Region identifier |
| axi_snk_data_aw_user | O | User-defined |
| axi_snk_data_w_valid | O | Write data valid |
| axi_snk_data_w_ready | I | Write data ready |
| axi_snk_data_w_data[511:0] | O | Write data |
| axi_snk_data_w_strb[63:0] | O | Write strobes |
| axi_snk_data_w_last | O | Last transfer in burst |
| axi_snk_data_w_user | O | User-defined |
| axi_snk_data_b_valid | I | Write response valid |
| axi_snk_data_b_ready | O | Write response ready |
| axi_snk_data_b_id[7:0] | I | Transaction ID |
| axi_snk_data_b_resp[1:0] | I | Write response |
| axi_snk_data_b_user | I | User-defined |

### AXI4 AW Master Interface (32-bit control data)

| Signal | IO | Description |
| --- | --- | --- |
| axi_snk_ctrl_aw_valid | O | Write address valid |
| axi_snk_ctrl_aw_ready | I | Write address ready |
| axi_snk_ctrl_aw_addr[39:0] | O | Write address |
| axi_snk_ctrl_aw_len[7:0] | O | Burst length - 1 |
| axi_snk_ctrl_aw_size[2:0] | O | Transfer size |
| axi_snk_ctrl_aw_burst[1:0] | O | Burst type |
| axi_snk_ctrl_aw_id[7:0] | O | Transaction ID |
| axi_snk_ctrl_aw_lock | O | Lock type |
| axi_snk_ctrl_aw_cache[3:0] | O | Cache attributes |
| axi_snk_ctrl_aw_prot[2:0] | O | Protection attributes |
| axi_snk_ctrl_aw_qos[3:0] | O | Quality of Service |
| axi_snk_ctrl_aw_region[3:0] | O | Region identifier |
| axi_snk_ctrl_aw_user | O | User-defined |
| axi_snk_ctrl_w_valid | O | Write data valid |
| axi_snk_ctrl_w_ready | I | Write data ready |
| axi_snk_ctrl_w_data[31:0] | O | Write data |
| axi_snk_ctrl_w_strb[3:0] | O | Write strobes |
| axi_snk_ctrl_w_last | O | Last transfer in burst |
| axi_snk_ctrl_w_user | O | User-defined |
| axi_snk_ctrl_b_valid | I | Write response valid |
| axi_snk_ctrl_b_ready | O | Write response ready |
| axi_snk_ctrl_b_id[7:0] | I | Transaction ID |
| axi_snk_ctrl_b_resp[1:0] | I | Write response |
| axi_snk_ctrl_b_user | I | User-defined |

### AXI4 AR Master Interface (512-bit descriptor data)

| Signal | IO | Description |
| --- | --- | --- |
| axi_snk_desc_ar_valid | O | Read address valid |
| axi_snk_desc_ar_ready | I | Read address ready |
| axi_snk_desc_ar_addr[39:0] | O | Read address |
| axi_snk_desc_ar_len[7:0] | O | Burst length - 1 |
| axi_snk_desc_ar_size[2:0] | O | Transfer size |
| axi_snk_desc_ar_burst[1:0] | O | Burst type |
| axi_snk_desc_ar_id[7:0] | O | Transaction ID |
| axi_snk_desc_ar_lock | O | Lock type |
| axi_snk_desc_ar_cache[3:0] | O | Cache attributes |
| axi_snk_desc_ar_prot[2:0] | Protection attributes |  |
| axi_snk_desc_ar_qos[3:0] | O | Quality of Service |
| axi_snk_desc_ar_region[3:0] | O | Region identifier |
| axi_snk_desc_ar_user | O | User-defined |
| axi_snk_desc_r_valid | I | Read data valid |
| axi_snk_desc_r_ready | O | Read data ready |
| axi_snk_desc_r_data[511:0] | I | Read data |
| axi_snk_desc_r_id[7:0] | I | Transaction ID |
| axi_snk_desc_r_resp[1:0] | I | Read response |
| axi_snk_desc_r_last | I | Last transfer in burst |
| axi_snk_desc_r_user | I | User-defined |

### AXI4-Lite Slave Interface (Sink Configuration)

| Signal | IO | Description |
| --- | --- | --- |
| axil_snk_cfg_aw_valid | I | Write address valid |
| axil_snk_cfg_aw_ready | O | Write address ready |
| axil_snk_cfg_aw_addr[39:0] | I | Write address |
| axil_snk_cfg_aw_prot[2:0] | I | Protection attributes |
| axil_snk_cfg_w_valid | I | Write data valid |
| axil_snk_cfg_w_ready | O | Write data ready |
| axil_snk_cfg_w_data[31:0] | I | Write data |
| axil_snk_cfg_w_strb[3:0] | I | Write strobes |
| axil_snk_cfg_b_valid | O | Write response valid |
| axil_snk_cfg_b_ready | I | Write response ready |
| axil_snk_cfg_b_resp[1:0] | O | Write response |
| axil_snk_cfg_ar_valid | I | Read address valid |
| axil_snk_cfg_ar_ready | O | Read address ready |
| axil_snk_cfg_ar_addr[39:0] | I | Read address |
| axil_snk_cfg_ar_prot[2:0] | I | Protection attributes |
| axil_snk_cfg_r_valid | O | Read data valid |
| axil_snk_cfg_r_ready | I | Read data ready |
| axil_snk_cfg_r_data[31:0] | O | Read data |
| axil_snk_cfg_r_resp[1:0] | O | Read response |

### AXI4-Stream Slave Interface (RX) - NEW v3.0

| Signal | IO | Description |
| --- | --- | --- |
| axis_snk_rx_tdata[511:0] | I | Stream data payload |
| axis_snk_rx_tstrb[63:0] | I | Byte strobes (write enables) |
| axis_snk_rx_tlast | I | Last transfer in packet |
| axis_snk_rx_tvalid | I | Stream data valid |
| axis_snk_rx_tready | O | Stream ready (backpressure) |
| axis_snk_rx_tuser[15:0] | I | User sideband (packet metadata) |

TUSER Encoding (Sink RX):

[15:8] - Reserved for future use
[7:0]  - Packet type/flags

Note: AXIS uses standard tstrb for byte-level validity instead of custom chunk_enables. All credits and ACK mechanisms removed from streaming interface.

## Monitor Bus AXI4-Lite Group Interfaces

### AXI4-Lite Slave Interface (Error/Interrupt Read)

| Signal | IO | Description |
| --- | --- | --- |
| axil4_mon_err_ar_valid | I | Read address valid |
| axil4_mon_err_ar_ready | O | Read address ready |
| axil4_mon_err_ar_addr[31:0] | I | Read address |
| axil4_mon_err_ar_prot[2:0] | I | Protection attributes |
| axil4_mon_err_r_valid | O | Read data valid |
| axil4_mon_err_r_ready | I | Read data ready |
| axil4_mon_err_r_data[63:0] | O | Read data (64-bit monitor packets) |
| axil4_mon_err_r_resp[1:0] | O | Read response |

### AXI4-Lite Master Interface (Monitor Write)

| Signal | IO | Description |
| --- | --- | --- |
| axil4_mon_wr_aw_valid | O | Write address valid |
| axil4_mon_wr_aw_ready | I | Write address ready |
| axil4_mon_wr_aw_addr[31:0] | O | Write address |
| axil4_mon_wr_aw_prot[2:0] | O | Protection attributes |
| axil4_mon_wr_w_valid | O | Write data valid |
| axil4_mon_wr_w_ready | I | Write data ready |
| axil4_mon_wr_w_data[31:0] | O | Write data |
| axil4_mon_wr_w_strb[3:0] | O | Write strobes |
| axil4_mon_wr_b_valid | I | Write response valid |
| axil4_mon_wr_b_ready | O | Write response ready |
| axil4_mon_wr_b_resp[1:0] | I | Write response |

### AXI4-Lite Slave Interface (Monitor Configuration)

| Signal | IO | Description |
| --- | --- | --- |
| axil4_mon_cfg_aw_valid | I | Write address valid |
| axil4_mon_cfg_aw_ready | O | Write address ready |
| axil4_mon_cfg_aw_addr[31:0] | I | Write address |
| axil4_mon_cfg_aw_prot[2:0] | I | Protection attributes |
| axil4_mon_cfg_w_valid | I | Write data valid |
| axil4_mon_cfg_w_ready | O | Write data ready |
| axil4_mon_cfg_w_data[31:0] | I | Write data |
| axil4_mon_cfg_w_strb[3:0] | I | Write strobes |
| axil4_mon_cfg_b_valid | O | Write response valid |
| axil4_mon_cfg_b_ready | I | Write response ready |
| axil4_mon_cfg_b_resp[1:0] | O | Write response |
| axil4_mon_cfg_ar_valid | I | Read address valid |
| axil4_mon_cfg_ar_ready | O | Read address ready |
| axil4_mon_cfg_ar_addr[31:0] | I | Read address |
| axil4_mon_cfg_ar_prot[2:0] | I | Protection attributes |
| axil4_mon_cfg_r_valid | O | Read data valid |
| axil4_mon_cfg_r_ready | I | Read data ready |
| axil4_mon_cfg_r_data[31:0] | O | Read data |
| axil4_mon_cfg_r_resp[1:0] | O | Read response |

## Key Interface Changes from v2.1 to v3.0

### REMOVED - Custom Network Protocol:

- network_*_pkt_valid/ready - Replaced by standard axis_*_tvalid/tready

- network_*_pkt_data[511:0] - Replaced by axis_*_tdata[511:0]

- network_*_pkt_type[1:0] - Moved to axis_*_tuser[7:0]

- network_*_pkt_addr[7:0] - Removed (no addressing in streaming)

- network_*_pkt_addr_par - Removed (parity optional via TUSER if needed)

- network_*_pkt_eos - Replaced by axis_*_tlast

- network_*_pkt_par - Removed (parity optional via TUSER if needed)

- ALL ACK signals - Removed completely (no credit/ACK on streaming)

  - network_*_ack_valid/ready

  - network_*_ack_ack[1:0]

  - network_*_ack_addr[7:0]

  - network_*_ack_addr_par

  - network_*_ack_par

- Embedded chunk_enables format - Replaced by standard axis_*_tstrb[63:0]

### ADDED - Standard AXIS4 Protocol:

- axis_src_tx_tdata[511:0] - Source TX data stream

- axis_src_tx_tstrb[63:0] - Byte-level write enables (64 bytes for 512-bit bus)

- axis_src_tx_tlast - Packet boundary marker

- axis_src_tx_tvalid/tready - Standard handshake protocol

- axis_src_tx_tuser[15:0] - Optional metadata sideband

- axis_snk_rx_tdata[511:0] - Sink RX data stream

- axis_snk_rx_tstrb[63:0] - Byte-level write enables

- axis_snk_rx_tlast - Packet boundary marker

- axis_snk_rx_tvalid/tready - Standard handshake protocol

- axis_snk_rx_tuser[15:0] - Optional metadata sideband

### Migration Benefits:

- Industry Standard: AXIS4 is widely supported, well-documented standard protocol

- Simplified Flow Control: Standard tvalid/tready backpressure, no custom ACK channels

- Cleaner Byte Qualification: Standard tstrb replaces embedded chunk_enables

- Packet Framing: Standard tlast replaces custom EOS markers

- Reduced Complexity: Eliminated custom packet types, addresses, parity, ACK logic

- Tool Support: Better IP integration, simulation, and verification tool support

- No Interface Credits: Simplified interface - credits remain only in scheduler (internal)

### AXIS4 vs Custom Network Protocol Mapping:

| Custom Network v2.1 | AXIS4 v3.0 | Notes |
| --- | --- | --- |
| network_*_pkt_data[511:0] | axis_*_tdata[511:0] | Direct data payload |
| network_*_pkt_chunk_enables[15:0] (embedded) | axis_*_tstrb[63:0] | Byte-level granularity |
| network_*_pkt_eos | axis_*_tlast | Standard packet boundary |
| network_*_pkt_valid/ready | axis_*_tvalid/tready | Standard handshake |
| network_*_pkt_type[1:0] | axis_*_tuser[7:0] | Metadata in sideband |
| network_*_pkt_addr[7:0] | REMOVED | No addressing in streaming |
| network_*_pkt_par | REMOVED | Optional via TUSER if needed |
| network_*_ack_* (all) | REMOVED | No ACK/credit on interface |

## Interface Summary

### Total AXI Interfaces:

- Source: 3 AXI4 Masters (data read, ctrl write, desc read) + 1 AXI4-Lite Slave (config)

- Sink: 3 AXI4 Masters (data write, ctrl write, desc read) + 1 AXI4-Lite Slave (config)

- Monitor: 1 AXI4-Lite Master (write) + 2 AXI4-Lite Slaves (error read, config)

### Total AXIS Interfaces (NEW v3.0):

- Source: 1 AXIS4 Master (TX streaming)

- Sink: 1 AXIS4 Slave (RX streaming)

### Key Features:

- Standard AXIS4 Protocol for high-bandwidth streaming

- Comprehensive AXI4-Lite Configuration for all subsystems

- Monitor Bus Aggregation with configurable filtering

- Error/Interrupt Handling via dedicated AXI4-Lite interface

- Proper Clock/Reset with core_clk and core_rstn

- Simplified Flow Control - No custom ACK or credit mechanisms on streaming interfaces

- Industry-Standard Interfaces - Better tool support and IP reuse

## AXIS Data Path Integration

### Source Data Path (Memory -> AXIS TX):

AXI4 Read Master (512-bit)
    ↓ Read data from system memory
Source SRAM Control
    ↓ Buffer management
AXIS Master (rtl/amba/axis/axis_master.sv)
    ↓ axis_src_tx_* signals
External AXIS Receiver

Key Points: - SRAM control writes to axis_master FUB interface (fub_axis_tdata/tstrb/tlast/tvalid) - AXIS master outputs external m_axis_* signals - Backpressure: axis_src_tx_tready=0 -> SRAM control stalls - Packet framing: SRAM sets tlast on final beat

### Sink Data Path (AXIS RX -> Memory):

External AXIS Transmitter
    ↓ axis_snk_rx_* signals
AXIS Slave (rtl/amba/axis/axis_slave.sv)
    ↓ Internal FUB interface
Sink SRAM Control
    ↓ Buffer management
AXI4 Write Master (512-bit)
    ↓ Write to system memory

Key Points: - AXIS slave receives external s_axis_* signals - Outputs to SRAM via FUB interface (fub_axis_tdata/tstrb/tlast/tvalid) - Backpressure: SRAM full -> axis_snk_rx_tready=0 -> upstream stalls - Packet framing: tlast=1 triggers SRAM to finalize packet

See: See: - ch03_interfaces/04_axis4_interface_spec.md - Complete AXIS4 specification - rtl/amba/axis/axis_master.sv - AXIS master RTL - rtl/amba/axis/axis_slave.sv - AXIS slave RTL

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# AXI4-Lite Interface Specification and Assumptions

## Overview

This document defines the formal specification and assumptions for an AXI4-Lite interface implementation. AXI4-Lite is a subset of AXI4 optimized for simple, lightweight control register interfaces with inherent protocol simplifications.

## Interface Summary

### Number of Interfaces

- 2 Master Read Interface: Single read channel for Monitor Packets (one for each Source and Sink)

- 2 Master Write Interface: Single write channel for Monitor Packets plus a timestamp (one for each Source and Sink)

### Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| DATA_WIDTH | AXI data bus width in bits | 32, 64 | 32, 64 |
| ADDR_WIDTH | AXI address bus width in bits | 32, 64 | 37 |
| STRB_WIDTH | Write strobe width | DATA_WIDTH/8 | 8 |

## Core Protocol Assumptions

### Inherent AXI4-Lite Simplifications

AXI4-Lite protocol inherently provides the following constraints:

| Constraint | Description |
| --- | --- |
| Single Transfers Only | No burst transactions supported |
| No Transaction IDs | All transactions are in-order |
| Fixed Transfer Size | Always uses full data bus width |
| No User Signals | Simplified interface without user-defined extensions |

### Implementation Assumptions

#### Assumption 1: Address Alignment to Data Bus Width

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All AXI4-Lite transactions aligned to data bus width |
| 32-bit bus alignment | Address[1:0] must be 2’b00 (4-byte aligned) |
| 64-bit bus alignment | Address[2:0] must be 3’b000 (8-byte aligned) |
| Rationale | Maximizes bus efficiency and eliminates unaligned access complexity |
| Benefit | Simplifies address decode and data steering logic |

#### Assumption 2: Fixed Transfer Size

| Aspect | Requirement |
| --- | --- |
| Transfer Size Rule | All transfers use maximum size equal to bus width |
| 32-bit bus | AxSIZE = 3’b010 (4 bytes) |
| 64-bit bus | AxSIZE = 3’b011 (8 bytes) |
| Rationale | Maximizes bus utilization and simplifies control logic |
| Benefit | No size decode logic required |

#### Assumption 3: No Address Wraparound

| Aspect | Requirement |
| --- | --- |
| Wraparound Rule | Transactions never wrap around top of address space |
| Rationale | Control register accesses never require wraparound behavior |
| Benefit | Simplified address boundary checking |

#### Assumption 4: Standard Protection Attributes

| Access Type | AxPROT Value | Description |
| --- | --- | --- |
| Normal Access | 3’b000 | Data, secure, unprivileged |
| Privileged Access | 3’b001 | Data, secure, privileged |
| Rationale |  | Covers the majority of control register access patterns |

## Master Read Interface Specification

### Read Address Channel (AR)

| Signal | Width | Direction | Required Values | Description |
| --- | --- | --- | --- | --- |
| ar_addr | ADDR_WIDTH | Master->Slave | 8-byte aligned | Read address |
| ar_prot | 3 | Master->Slave | Implementation specific | Protection attributes |
| ar_valid | 1 | Master->Slave | 0 or 1 | Address valid |
| ar_ready | 1 | Slave->Master | 0 or 1 | Address ready |

### Read Data Channel (R)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| r_data | 64 | Slave->Master | Read data |
| r_resp | 2 | Slave->Master | Read response |
| r_valid | 1 | Slave->Master | Read data valid |
| r_ready | 1 | Master->Slave | Read data ready |

### AXI4-Lite Simplifications (Read)

| Removed Signal | AXI4 Usage | AXI4-Lite Reason |
| --- | --- | --- |
| ar_id | Transaction ID | Single transfers, no transaction IDs |
| ar_len | Burst length | Single transfers only |
| ar_size | Transfer size | Fixed to bus width |
| ar_burst | Burst type | Single transfers only |
| ar_lock | Lock type | Simplified access model |
| ar_cache | Cache attributes | Simplified memory model |
| ar_qos | Quality of Service | Simplified priority model |
| ar_region | Region identifier | Simplified address space |
| ar_user | User-defined | Simplified interface |
| r_id | Transaction ID | No transaction IDs needed |
| r_last | Last transfer | Single transfers only |
| r_user | User-defined | Simplified interface |

## Master Write Interface Specification

### Write Address Channel (AW)

| Signal | Width | Direction | Required Values | Description |
| --- | --- | --- | --- | --- |
| aw_addr | ADDR_WIDTH | Master->Slave | 8-byte aligned | Write address |
| aw_prot | 3 | Master->Slave | Implementation specific | Protection attributes |
| aw_valid | 1 | Master->Slave | 0 or 1 | Address valid |
| aw_ready | 1 | Slave->Master | 0 or 1 | Address ready |

### Write Data Channel (W)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| w_data | 32 | Master->Slave | Write data |
| w_strb | 4 | Master->Slave | Write strobes (byte enables) |
| w_valid | 1 | Master->Slave | Write data valid |
| w_ready | 1 | Slave->Master | Write data ready |

### Write Response Channel (B)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| b_resp | 2 | Slave->Master | Write response |
| b_valid | 1 | Slave->Master | Response valid |
| b_ready | 1 | Master->Slave | Response ready |

### AXI4-Lite Simplifications (Write)

| Removed Signal | AXI4 Usage | AXI4-Lite Reason |
| --- | --- | --- |
| aw_id | Transaction ID | Single transfers, no transaction IDs |
| aw_len | Burst length | Single transfers only |
| aw_size | Transfer size | Fixed to bus width |
| aw_burst | Burst type | Single transfers only |
| aw_lock | Lock type | Simplified access model |
| aw_cache | Cache attributes | Simplified memory model |
| aw_qos | Quality of Service | Simplified priority model |
| aw_region | Region identifier | Simplified address space |
| aw_user | User-defined | Simplified interface |
| w_last | Last transfer | Single transfers only |
| w_user | User-defined | Simplified interface |
| b_id | Transaction ID | No transaction IDs needed |
| b_user | User-defined | Simplified interface |

## Address Requirements

### Address Alignment Rules

| Alignment Type | Formula | Description |
| --- | --- | --- |
| Valid Address | (Address % 4) == 0 | Must be 8-byte aligned |
| Mandatory Alignment | Address[2:0] must be 3’b000 | Per Assumption 1 |

### Address Validation Examples

| Address Category | Examples | Status |
| --- | --- | --- |
| Valid (84byte aligned) | 0x1000, 0x1004, 0x1008, 0x100C | Accepted |
| Invalid (unaligned) | 0x1001, 0x1002, 0x1003 | DECERR response |

## Response Codes

### Response Code Specification

| Value | Name | Description | Usage in Control Registers |
| --- | --- | --- | --- |
| 2’b00 | OKAY | Normal access success | Successful register access |
| 2’b01 | EXOKAY | Exclusive access success | Not used in AXI4-Lite |
| 2’b10 | SLVERR | Slave error | Invalid register access |
| 2’b11 | DECERR | Decode error | Address decode failure or misalignment |

### Response Usage Guidelines

| Response Type | Usage | Description |
| --- | --- | --- |
| OKAY | Normal completion | Successful register access |
| EXOKAY | Not applicable | AXI4-Lite doesn’t support exclusive accesses |
| SLVERR | Register error | Invalid register operation |
| DECERR | Address error | Misalignment or decode failure per Assumption 1 |

## Protection Signal Usage

### Protection Signal Encoding

| Bit | Name | Description | Recommended Usage |
| --- | --- | --- | --- |
| [0] | Privileged | 0=Normal, 1=Privileged | Set based on processor mode |
| [1] | Non-secure | 0=Secure, 1=Non-secure | Set based on security domain |
| [2] | Instruction | 0=Data, 1=Instruction | Always 0 for control registers |

### Common Protection Patterns

| Pattern | AxPROT Value | Description |
| --- | --- | --- |
| Normal Data Access | 3’b000 | Standard register access |
| Privileged Data Access | 3’b001 | Privileged register access |
| Debug Access | 3’b010 | Debug register access |
| Privileged Debug | 3’b011 | Privileged debug access |

## Implementation Benefits

### Simplified Control Register Interface

| Benefit Area | Simplification | Impact |
| --- | --- | --- |
| Address Decode | Simple 8-byte aligned address comparison | Reduced decode logic |
| Transaction Handling | No burst or ID tracking required | Simplified state machines |
| Flow Control | Straightforward valid-ready handshakes | Reduced complexity |
| Response Generation | Simple OKAY/SLVERR/DECERR responses | Minimal response logic |
| Size Handling | Fixed 64-bit transfers only | No size decode needed |

### Address Decode Implementation

| Implementation Aspect | Method | Benefit |
| --- | --- | --- |
| 4-byte Alignment Check | addr[1:0] == 2’b00 | Simple bit masking |
| Address Range Check | addr >= base && addr <= limit | Simple comparisons |
| Combined Check | alignment_ok && range_ok | Single decode decision |

### Error Generation Logic

| Error Condition | Check | Response |
| --- | --- | --- |
| Address Misalignment | addr[1:0] != 2’b00 | Generate DECERR |
| Address Out of Range | !addr_in_range(addr) | Generate DECERR |
| Register Error | register_error_condition | Generate SLVERR |
| Normal Access | All checks pass | Generate OKAY |

## Timing Requirements

### Handshake Protocol

| Protocol Rule | Requirement | Description |
| --- | --- | --- |
| Valid-Ready Transfer | Transfer occurs when both VALID and READY are high | Standard AXI handshake |
| Valid Independence | VALID can be asserted independently of READY | Master controls valid |
| Ready Dependency | READY can depend on VALID state | Slave controls ready |
| Signal Stability | Once VALID asserted, all signals stable until READY | Data integrity |

### Channel Dependencies

| Dependency | Requirement | Description |
| --- | --- | --- |
| Write Channels | AW and W channels are independent | Can be presented in any order |
| Write Response | B channel waits for both AW and W completion | Response dependency |
| Read Channels | R channel waits for AR channel completion | Response dependency |
| Transaction Ordering | Multiple outstanding transactions not supported | Inherent AXI4-Lite limitation |

### Reset Behavior

| Reset Phase | Requirement | Description |
| --- | --- | --- |
| Active Reset | aresetn is active-low reset signal | Standard AXI reset |
| Reset Requirements | All VALID signals deasserted during reset | Clean reset state |
| Reset Recovery | All VALID signals low after reset deassertion | Proper startup |

## Validation Requirements

### Functional Validation

| Validation Area | Requirements |
| --- | --- |
| Address Alignment | Verify all accesses are 8-byte aligned per Assumption 1 |
| Fixed Size | Verify all transfers are full 64-bit width per Assumption 2 |
| Response Correctness | Verify appropriate response codes (DECERR for misaligned access) |
| Handshake Compliance | Verify all valid-ready handshakes |
| Register Behavior | Verify read/write register functionality |
| No Wraparound | Verify no address wraparound scenarios per Assumption 3 |

### Timing Validation

| Validation Area | Requirements |
| --- | --- |
| Setup/Hold | Verify signal timing requirements |
| Reset Behavior | Verify proper reset sequence |
| Back-pressure | Verify ready signal behavior under load |

### Error Injection Testing

| Test Type | Injection Method | Expected Response |
| --- | --- | --- |
| Misaligned Address | Inject addresses with addr[2:0] != 0 | DECERR response |
| Out of Range | Inject addresses outside valid range | DECERR response |
| Register Errors | Inject register-specific errors | SLVERR response |

## Example Transactions

### 64-bit Register Write

| Parameter | Value | Description |
| --- | --- | --- |
| Bus Width | 64 bits (8 bytes) | Data bus configuration |
| Target Address | 0x1000 (8-byte aligned) | Valid aligned address |
| Write Data | 0xDEADBEEFCAFEBABE | 64-bit data value |
| Required Settings | aw_addr=0x1000, aw_prot=3’b000, w_data=0xDEADBEEFCAFEBABE, w_strb=8’b11111111 | Transaction configuration |

### AW Transaction Flow

| Step | Action | Signal States |
| --- | --- | --- |
| 1 | Assert aw_valid with address | aw_valid=1, aw_addr=0x1000 |
| 2 | Assert w_valid with data | w_valid=1, w_data=0xDEADBEEFCAFEBABE |
| 3 | Wait for handshakes | aw_ready=1, w_ready=1 |
| 4 | Wait for response | b_valid=1, b_resp=OKAY |
| 5 | Complete transaction | b_ready=1 |

### 64-bit Register Read

| Parameter | Value | Description |
| --- | --- | --- |
| Bus Width | 64 bits (8 bytes) | Data bus configuration |
| Target Address | 0x1008 (8-byte aligned) | Valid aligned address |
| Required Settings | ar_addr=0x1008, ar_prot=3’b000 | Transaction configuration |

### AR Transaction Flow

| Step | Action | Signal States |
| --- | --- | --- |
| 1 | Assert ar_valid with address | ar_valid=1, ar_addr=0x1008 |
| 2 | Wait for address handshake | ar_ready=1 |
| 3 | Wait for data response | r_valid=1, r_resp=OKAY |
| 4 | Complete transaction | r_ready=1 |
| 5 | Capture data | r_data (64 bits) |

### Misaligned Address Example

| Parameter | Value | Description |
| --- | --- | --- |
| Bus Width | 64 bits (8 bytes) | Data bus configuration |
| Target Address | 0x1004 (misaligned) | Invalid address |
| Expected Behavior | Address decode detects misalignment -> DECERR response -> No register access | Error handling |

## Common Use Cases

### Typical Applications

| Application | Description |
| --- | --- |
| Control/Status Registers | 64-bit device configuration and monitoring |
| Memory-Mapped Peripherals | Simple register-based devices |
| Debug Interfaces | Debug and trace control registers |
| Configuration Space | PCIe configuration space access |
| Performance Counters | 64-bit performance monitoring registers |

### Performance Considerations

| Consideration | Impact | Description |
| --- | --- | --- |
| Latency | Single-cycle responses preferred | Simple registers |
| Throughput | Limited by single outstanding transaction | AXI4-Lite constraint |
| Efficiency | 64-bit transfers maximize data efficiency | Modern system optimization |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# AXI4 Interface Specification and Assumptions

## Overview

This document defines the formal specification and assumptions for an AXI4 interface implementation that supports two distinct transfer modes to optimize for different interface types while ensuring robust, predictable operation.

## Interface Summary

### Number of Interfaces

- 5 Master Read Interfaces: Descriptor sink, descriptor source, data source, flag sink, and flag source

- 3 Master Write Interfaces: Data sink, control sink, and control source

### Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| DATA_WIDTH | AXI data bus width in bits | 32, 64, 128, 256, 512, 1024 | 32 |
| ADDR_WIDTH | AXI address bus width in bits | 32, 64 | 37 |
| ID_WIDTH | AXI ID tag width in bits | 1-16 | 8 |
| USER_WIDTH | AXI user signal width in bits (optional) | 0-16 | 1 |

### Interface Types and Transfer Modes

| Interface Group | Channels | Transfer Mode | Address Alignment | Monitor | DCG | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AXI4 Master Read-Split | AR, R | Flexible | 4-byte | Yes | Yes | Data interfaces with chunk enables |
| AXI4 Master Write-Split | AW, W, B | Flexible | 4-byte | Yes | Yes | Data interfaces with chunk enables |
| AXI4 Master Read | AR, R | Simplified | Bus-width | No | Yes | Control interfaces, fully aligned |
| AXI4 Master Write | AW, W, B | Simplified | Bus-width | Yes | Yes | Control interfaces, fully aligned |

### Interface Group Parameter Settings

| Interface Group | Data Width | Address Width | ID Width | User Width | Transfer Mode |
| --- | --- | --- | --- | --- | --- |
| AXI4 Master Read-Split | 512 bits | 37 bits | 8 bits | 1 bit | Flexible |
| AXI4 Master Write-Split | 512 bits | 37 bits | 8 bits | 1 bit | Flexible |
| AXI4 Master Read | 32 bits | 37 bits | 8 bits | 1 bit | Simplified |
| AXI4 Master Write | 32 bits | 37 bits | 8 bits | 1 bit | Simplified |

### Interface Configuration Summary

| Interface Type | Interface Group | Transfer Mode | Alignment | Notes |
| --- | --- | --- | --- | --- |
| Descriptor Sink | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |
| Descriptor Source | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |
| Data Source | AXI4 Master Read-Split | Flexible | 4-byte aligned | High-bandwidth data |
| Data Sink | AXI4 Master Write-Split | Flexible | 4-byte aligned | High-bandwidth data |
| Program Sink | AXI4 Master Write | Simplified | 32-bit aligned | Control interface |
| Program Source | AXI4 Master Write | Simplified | 32-bit aligned | Control interface |
| Flag Sink | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |
| Flag Source | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |

## Transfer Mode Specifications

This specification defines two distinct transfer modes to optimize different interface types:

### Mode 1: Simplified Transfer Mode (Control Interfaces)

Used for control interfaces (descriptors, programs, flags) that prioritize simplicity and predictable timing.

#### Simplified Mode Assumptions

| Aspect | Requirement |
| --- | --- |
| Address Alignment | All addresses aligned to full data bus width |
| Transfer Size | All transfers use maximum size equal to bus width |
| Burst Type | Incrementing bursts only (AxBURST = 2’b01) |
| Transfer Complexity | Maximum simplicity for predictable operation |

### Mode 2: Flexible Transfer Mode (Data Interfaces)

Used for high-bandwidth data interfaces that need to handle arbitrary address alignment while maintaining efficiency.

#### Flexible Mode Assumptions

| Aspect | Requirement |
| --- | --- |
| Address Alignment | 4-byte aligned addresses (minimum alignment) |
| Transfer Sizes | Multiple sizes supported: 4, 8, 16, 32, 64 bytes |
| Burst Type | Incrementing bursts only (AxBURST = 2’b01) |
| Alignment Strategy | Progressive alignment to optimize bus utilization |

## Mode 1: Simplified Transfer Mode Specification

### Assumption 1: Address Alignment to Data Bus Width

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All AXI transactions aligned to data bus width |
| 32-bit bus (4 bytes) | Address[1:0] must be 2’b00 |
| 64-bit bus (8 bytes) | Address[2:0] must be 3’b000 |
| 128-bit bus (16 bytes) | Address[3:0] must be 4’b0000 |
| 256-bit bus (32 bytes) | Address[4:0] must be 5’b00000 |
| 512-bit bus (64 bytes) | Address[5:0] must be 6’b000000 |
| 1024-bit bus (128 bytes) | Address[6:0] must be 7’b0000000 |
| Rationale | Maximizes bus efficiency and eliminates unaligned access complexity |

### Assumption 2: Fixed Transfer Size

| Aspect | Requirement |
| --- | --- |
| Transfer Size Rule | All transfers use maximum size equal to bus width |
| 32-bit bus | AxSIZE = 3’b010 (4 bytes) |
| 64-bit bus | AxSIZE = 3’b011 (8 bytes) |
| 128-bit bus | AxSIZE = 3’b100 (16 bytes) |
| 256-bit bus | AxSIZE = 3’b101 (32 bytes) |
| 512-bit bus | AxSIZE = 3’b110 (64 bytes) |
| 1024-bit bus | AxSIZE = 3’b111 (128 bytes) |
| Rationale | Maximizes bus utilization and simplifies address alignment |

## Mode 2: Flexible Transfer Mode Specification

### Assumption 1: 4-Byte Address Alignment

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All AXI transactions aligned to 4-byte boundaries |
| Address Constraint | Address[1:0] must be 2’b00 |
| Rationale | Balances flexibility with AXI protocol requirements |
| Benefit | Supports arbitrary data placement while maintaining AXI compliance |

### Assumption 2: Multiple Transfer Sizes

| Transfer Size | AxSIZE Value | Use Case |
| --- | --- | --- |
| 4 bytes | 3’b010 | Initial alignment, small transfers |
| 8 bytes | 3’b011 | Progressive alignment |
| 16 bytes | 3’b100 | Progressive alignment |
| 32 bytes | 3’b101 | Progressive alignment |
| 64 bytes | 3’b110 | Optimal full-width transfers |
| 128 bytes | 3’b111 | Maximum efficiency (1024-bit bus) |

### Assumption 3: Progressive Alignment Strategy

| Aspect | Requirement |
| --- | --- |
| Alignment Goal | Align to 64-byte boundaries for optimal bus utilization |
| Alignment Sequence | Use progressive sizes: 4 -> 8 -> 16 -> 32 -> 64 bytes |
| Optimization | Choose largest possible transfer size at each step |
| Example | Address 0x1004: 4-byte transfer -> aligned to 0x1008, then larger transfers |

### Assumption 4: Chunk Enable Support

| Aspect | Requirement |
| --- | --- |
| Chunk Granularity | 16 chunks of 32-bits each (512-bit bus) |
| Write Strobes | Generated from chunk enables for precise byte control |
| Alignment Transfers | Chunk patterns optimized for alignment sequences |
| Benefits | Precise data validity, optimal memory utilization |

## Common Protocol Assumptions (Both Modes)

### Assumption 1: Incrementing Bursts Only

| Aspect | Requirement |
| --- | --- |
| Burst Type | All AXI bursts use incrementing address mode (AxBURST = 2’b01) |
| Excluded Types | No FIXED (2’b00) or WRAP (2’b10) bursts supported |
| Rationale | Simplifies address generation logic and covers most use cases |
| Benefit | Eliminates wrap boundary calculations and fixed address handling |

### Assumption 2: No Address Wraparound

| Aspect | Requirement |
| --- | --- |
| Wraparound Rule | Transactions never wrap around top of address space |
| Example | No 0xFFFFFFFF -> 0x00000000 transitions |
| Rationale | Real systems never allow this due to memory layout |
| Benefit | Dramatically simplified boundary crossing detection logic |

## Flexible Mode: Address Calculation Examples

### Progressive Alignment Examples

Example 1: Address 0x1004 -> 0x1040 (64-byte boundary)

| Step | Address | Size | AxSIZE | Length | Bytes Transferred | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0x1004 | 4 bytes | 3’b010 | 1 beat | 4 | Initial alignment |
| 2 | 0x1008 | 8 bytes | 3’b011 | 1 beat | 8 | Progressive alignment |
| 3 | 0x1010 | 16 bytes | 3’b100 | 1 beat | 16 | Progressive alignment |
| 4 | 0x1020 | 32 bytes | 3’b101 | 1 beat | 32 | Progressive alignment |
| 5 | 0x1040 | 64 bytes | 3’b110 | N beats | 64xN | Optimal transfers |

Example 2: Address 0x1010 -> 0x1040 (64-byte boundary)

| Step | Address | Size | AxSIZE | Length | Bytes Transferred | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0x1010 | 16 bytes | 3’b100 | 1 beat | 16 | Optimal initial size |
| 2 | 0x1020 | 32 bytes | 3’b101 | 1 beat | 32 | Progressive alignment |
| 3 | 0x1040 | 64 bytes | 3’b110 | N beats | 64xN | Optimal transfers |

### Chunk Enable Pattern Examples

512-bit Bus with 16x32-bit chunks

| Transfer Size | Address Offset | Chunk Pattern | Description |
| --- | --- | --- | --- |
| 4 bytes | 0x04 | 16’h0002 | Chunk 1 only |
| 8 bytes | 0x08 | 16’h000C | Chunks 2-3 |
| 16 bytes | 0x10 | 16’h00F0 | Chunks 4-7 |
| 32 bytes | 0x20 | 16’hFF00 | Chunks 8-15 |
| 64 bytes | 0x00 | 16’hFFFF | All chunks |

## Master Read Interface Specification

### Read Address Channel (AR)

| Signal | Width | Direction | Simplified Mode | Flexible Mode | Description |
| --- | --- | --- | --- | --- | --- |
| ar_addr | ADDR_WIDTH | Master->Slave | Bus-width aligned | 4-byte aligned | Read address |
| ar_len | 8 | Master->Slave | 0-255 | 0-255 | Burst length - 1 |
| ar_size | 3 | Master->Slave | Fixed per bus | Variable: 4-64 bytes | Transfer size |
| ar_burst | 2 | Master->Slave | 2’b01 (INCR only) | 2’b01 (INCR only) | Burst type |
| ar_id | ID_WIDTH | Master->Slave | Any | Any | Transaction ID |
| ar_lock | 1 | Master->Slave | 1’b0 | 1’b0 | Lock type (normal) |
| ar_cache | 4 | Master->Slave | Implementation specific | 4’b0011 | Cache attributes |
| ar_prot | 3 | Master->Slave | Implementation specific | 3’b000 | Protection attributes |
| ar_qos | 4 | Master->Slave | 4’b0000 | 4’b0000 | Quality of Service |
| ar_region | 4 | Master->Slave | 4’b0000 | 4’b0000 | Region identifier |
| ar_user | USER_WIDTH | Master->Slave | Optional | Optional | User-defined |
| ar_valid | 1 | Master->Slave | 0 or 1 | 0 or 1 | Address valid |
| ar_ready | 1 | Slave->Master | 0 or 1 | 0 or 1 | Address ready |

### Read Data Channel (R)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| r_data | DATA_WIDTH | Slave->Master | Read data |
| r_id | ID_WIDTH | Slave->Master | Transaction ID |
| r_resp | 2 | Slave->Master | Read response |
| r_last | 1 | Slave->Master | Last transfer in burst |
| r_user | USER_WIDTH | Slave->Master | User-defined (optional) |
| r_valid | 1 | Slave->Master | Read data valid |
| r_ready | 1 | Master->Slave | Read data ready |

## Master Write Interface Specification

### Write Address Channel (AW)

| Signal | Width | Direction | Simplified Mode | Flexible Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aw_addr | ADDR_WIDTH | Master->Slave | Bus-width aligned | 4-byte aligned | Write address |
| aw_len | 8 | Master->Slave | 0-255 | 0-255 | Burst length - 1 |
| aw_size | 3 | Master->Slave | Fixed per bus | Variable: 4-64 bytes | Transfer size |
| aw_burst | 2 | Master->Slave | 2’b01 (INCR only) | 2’b01 (INCR only) | Burst type |
| aw_id | ID_WIDTH | Master->Slave | Any | Any | Transaction ID |
| aw_lock | 1 | Master->Slave | 1’b0 | 1’b0 | Lock type (normal) |
| aw_cache | 4 | Master->Slave | Implementation specific | 4’b0011 | Cache attributes |
| aw_prot | 3 | Master->Slave | Implementation specific | 3’b000 | Protection attributes |
| aw_qos | 4 | Master->Slave | 4’b0000 | 4’b0000 | Quality of Service |
| aw_region | 4 | Master->Slave | 4’b0000 | 4’b0000 | Region identifier |
| aw_user | USER_WIDTH | Master->Slave | Optional | Optional | User-defined |
| aw_valid | 1 | Master->Slave | 0 or 1 | 0 or 1 | Address valid |
| aw_ready | 1 | Slave->Master | 0 or 1 | 0 or 1 | Address ready |

### Write Data Channel (W)

| Signal | Width | Direction | Simplified Mode | Flexible Mode | Description |
| --- | --- | --- | --- | --- | --- |
| w_data | DATA_WIDTH | Master->Slave | Write data | Write data | Write data |
| w_strb | DATA_WIDTH/8 | Master->Slave | All 1’s | From chunk enables | Write strobes |
| w_last | 1 | Master->Slave | Last transfer | Last transfer | Last transfer in burst |
| w_user | USER_WIDTH | Master->Slave | Optional | Optional | User-defined |
| w_valid | 1 | Master->Slave | 0 or 1 | 0 or 1 | Write data valid |
| w_ready | 1 | Slave->Master | 0 or 1 | 0 or 1 | Write data ready |

### Write Response Channel (B)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| b_id | ID_WIDTH | Slave->Master | Transaction ID |
| b_resp | 2 | Slave->Master | Write response |
| b_user | USER_WIDTH | Slave->Master | User-defined (optional) |
| b_valid | 1 | Slave->Master | Response valid |
| b_ready | 1 | Master->Slave | Response ready |

## Address Calculation Rules

### Simplified Mode Address Generation

| Parameter | Formula | Description |
| --- | --- | --- |
| First Address | Must be bus-width aligned | Starting address |
| Address N | First_Address + (N x Bus_Width_Bytes) | Address for beat N |
| Alignment Check | (Address % Bus_Width_Bytes) == 0 | Must always be true |

### Flexible Mode Address Generation

| Parameter | Formula | Description |
| --- | --- | --- |
| First Address | Must be 4-byte aligned | Starting address |
| Address N | First_Address + (N x Transfer_Size) | Address for beat N |
| Alignment Check | (Address % 4) == 0 | Must always be true |
| Progressive Alignment | Choose largest size <= bytes_to_boundary | Optimization strategy |

### 4KB Boundary Considerations (Both Modes)

| Validation Rule | Formula | Description |
| --- | --- | --- |
| 4KB Boundary | Bursts cannot cross 4KB (0x1000) boundaries | AXI specification |
| Max Burst Calculation | Max_Beats = (4KB - (Start_Address % 4KB)) / Transfer_Size | Burst limit |
| Boundary Check | Verify no 4KB crossings in burst | Mandatory validation |

## Write Strobe Generation

### Simplified Mode Strobe Generation

| Bus Width | Strobe Pattern | Description |
| --- | --- | --- |
| 32-bit | 4’b1111 | All bytes valid |
| 512-bit | 64’hFFFFFFFFFFFFFFFF | All bytes valid |

### Flexible Mode Strobe Generation

From Chunk Enables (512-bit bus example):

// Convert 16x32-bit chunk enables to 64x8-bit write strobes
for (int chunk = 0; chunk < 16; chunk++) begin
    if (chunk_enable[chunk]) begin
        w_strb[chunk*4 +: 4] = 4'hF; // 4 bytes per chunk
    end
end

Alignment Transfer Examples:

| Transfer Size | Chunk Pattern | Strobe Pattern | Description |
| --- | --- | --- | --- |
| 4 bytes | 16’h0001 | 64’h000000000000000F | First 4 bytes |
| 16 bytes | 16’h000F | 64’h00000000000000FF | First 16 bytes |
| 32 bytes | 16’h00FF | 64’h0000000000FFFFFF | First 32 bytes |
| 64 bytes | 16’hFFFF | 64’hFFFFFFFFFFFFFFFF | All 64 bytes |

## Response Codes

### Response Code Specification

| Value | Name | Description | Simplified Mode Usage | Flexible Mode Usage |
| --- | --- | --- | --- | --- |
| 2’b00 | OKAY | Normal access success | Bus-width aligned access | 4-byte aligned access |
| 2’b01 | EXOKAY | Exclusive access success | Bus-width aligned exclusive | 4-byte aligned exclusive |
| 2’b10 | SLVERR | Slave error | Slave-specific error | Slave-specific error |
| 2’b11 | DECERR | Decode error | Bus-width misalignment | 4-byte misalignment |

## Implementation Benefits

### Simplified Mode Benefits

| Benefit Area | Simplification | Impact |
| --- | --- | --- |
| Address Generation | Simple increment by bus width | Minimal logic complexity |
| Size Checking | No dynamic size validation | No validation logic needed |
| Strobe Generation | All strobes always high | Trivial implementation |
| Timing | Predictable single-size transfers | Optimal timing closure |

### Flexible Mode Benefits

| Benefit Area | Capability | Impact |
| --- | --- | --- |
| Data Placement | Arbitrary 4-byte aligned placement | Maximum flexibility |
| Bus Utilization | Progressive alignment optimization | High efficiency achieved |
| Chunk Control | Precise byte-level validity | Optimal memory utilization |
| Alignment Strategy | Automatic alignment to boundaries | Performance optimization |

### Mode Selection Guidelines

| Interface Type | Recommended Mode | Rationale |
| --- | --- | --- |
| High-bandwidth data | Flexible | Maximize throughput, handle arbitrary alignment |
| Control/status | Simplified | Predictable timing, minimal complexity |
| Descriptors | Simplified | Fixed-size structures, simple implementation |
| Programs | Simplified | Single-word writes, minimal overhead |
| Flags | Simplified | Fixed-size status, predictable behavior |

## Validation Requirements

### Simplified Mode Validation

| Validation Area | Requirements |
| --- | --- |
| Address Alignment | Verify all addresses aligned to full bus width |
| Fixed Size | Verify AxSIZE always matches DATA_WIDTH |
| Full Strobes | Verify w_strb is always all 1’s |
| Burst Type | Verify AxBURST is always 2’b01 |

### Flexible Mode Validation

| Validation Area | Requirements |
| --- | --- |
| Address Alignment | Verify all addresses are 4-byte aligned |
| Size Validation | Verify AxSIZE matches actual transfer size |
| Chunk Consistency | Verify chunk enables match transfer size |
| Strobe Generation | Verify strobes generated correctly from chunks |
| Progressive Alignment | Verify alignment strategy optimization |
| Boundary Checking | Verify no 4KB boundary crossings |

### Common Validation

| Validation Area | Requirements |
| --- | --- |
| No Wraparound | Verify addresses never wrap around |
| Incrementing Only | Verify AxBURST is always 2’b01 |
| Response Handling | Verify proper response generation |
| Error Conditions | Verify alignment violation responses |

## Performance Characteristics

### Simplified Mode Performance

| Metric | Typical Value | Description |
| --- | --- | --- |
| Latency | 3 cycles | Address + Data + Response |
| Throughput | 1 transfer per clock | Sustained rate |
| Efficiency | 100% | Perfect bus utilization |
| Complexity | Minimal | Simple implementation |

### Flexible Mode Performance

| Metric | Alignment Phase | Optimized Phase | Description |
| --- | --- | --- | --- |
| Latency | 3-15 cycles | 3 cycles | Variable based on alignment |
| Throughput | Variable | 1 transfer per clock | Depends on alignment pattern |
| Efficiency | 25-100% | 100% | Improves with alignment |
| Complexity | Moderate | Minimal | Progressive optimization |

### Performance Optimization Strategy

Flexible Mode Alignment Strategy: 1. Initial Phase: Use largest possible transfer size for current alignment 2. Progressive Phase: Incrementally align to larger boundaries
3. Optimized Phase: Use full bus-width transfers once aligned 4. Result: Achieve maximum efficiency while handling arbitrary starting addresses

This dual-mode approach provides the best of both worlds: simplified, predictable operation for control interfaces and flexible, high-performance operation for data interfaces.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# AXI4-Stream (AXIS4) Interface Specification and Assumptions

## Overview

This document defines the formal specification and assumptions for AXI4-Stream (AXIS4) interface implementations used in the RAPIDS system. AXIS4 provides high-bandwidth, unidirectional streaming data transfer with built-in flow control and packet framing capabilities.

## Interface Summary

### Number of Interfaces

- 2 Master (Transmit) Interfaces: Source data path and network master output

- 2 Slave (Receive) Interfaces: Sink data path and network slave input

### Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| TDATA_WIDTH | Stream data bus width in bits | 32, 64, 128, 256, 512, 1024 | 512 |
| TID_WIDTH | Stream ID width in bits (optional) | 0-8 | 0 |
| TDEST_WIDTH | Stream destination width in bits (optional) | 0-8 | 0 |
| TUSER_WIDTH | User-defined sideband width in bits (optional) | 0-128 | 0 |
| TKEEP_ENABLE | Enable TKEEP byte qualifier signals | 0, 1 | 1 |
| TSTRB_ENABLE | Enable TSTRB byte strobe signals | 0, 1 | 0 |

### Interface Configuration Summary

| Interface Type | Direction | TDATA Width | TID | TDEST | TUSER | TKEEP | Purpose |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network Master (TX) | Output | 512 bits | No | Optional | Optional | Yes | Transmit packets to network |
| Network Slave (RX) | Input | 512 bits | No | Optional | Optional | Yes | Receive packets from network |
| Source Data Stream | Output | 512 bits | No | No | Optional | Yes | Memory-to-network streaming |
| Sink Data Stream | Input | 512 bits | No | No | Optional | Yes | Network-to-memory streaming |

## Core Protocol Assumptions

### AXI4-Stream Fundamentals

| Aspect | Requirement |
| --- | --- |
| Transfer Protocol | Valid-ready handshake on every transfer |
| Data Flow | Unidirectional streaming (master -> slave) |
| Packet Framing | TLAST signal indicates packet boundaries |
| Flow Control | Backpressure via TREADY signal |
| Byte Granularity | TKEEP indicates valid bytes within transfer |

### Implementation Assumptions

#### Assumption 1: TKEEP-Based Byte Qualification

| Aspect | Requirement |
| --- | --- |
| Byte Validity Rule | TKEEP indicates which bytes of TDATA contain valid data |
| 512-bit bus (64 bytes) | TKEEP[63:0] - one bit per byte |
| Contiguous Bytes | Valid bytes are always contiguous (no gaps) |
| Alignment | Valid bytes always start from TDATA[7:0] (byte 0) |
| Rationale | Simplifies data alignment and reduces complexity |
| Benefit | Eliminates data steering logic for non-contiguous bytes |

TKEEP Encoding Examples (64-byte bus):

| Transfer Size | TKEEP Value | Description |
| --- | --- | --- |
| 64 bytes | 64’hFFFFFFFFFFFFFFFF | All bytes valid (full transfer) |
| 32 bytes | 64’h00000000FFFFFFFF | First 32 bytes valid |
| 16 bytes | 64’h000000000000FFFF | First 16 bytes valid |
| 8 bytes | 64’h00000000000000FF | First 8 bytes valid |
| 4 bytes | 64’h000000000000000F | First 4 bytes valid |
| 1 byte | 64’h0000000000000001 | First byte valid |

#### Assumption 2: TLAST for Packet Boundaries

| Aspect | Requirement |
| --- | --- |
| Packet Delimiter | TLAST=1 on final transfer of packet |
| Single-Beat Packets | TLAST=1 for single-transfer packets |
| Multi-Beat Packets | TLAST=0 for all transfers except last |
| Mandatory Signal | TLAST required for all packet-based protocols |
| Rationale | Enables downstream packet processing and buffering |

#### Assumption 3: No TSTRB Usage

| Aspect | Requirement |
| --- | --- |
| TSTRB Disabled | TSTRB signals not used (TSTRB_ENABLE=0) |
| Byte Qualification | TKEEP provides sufficient byte-level control |
| Rationale | RAPIDS data is always read-oriented (no write strobes needed) |
| Benefit | Reduces interface width and complexity |

#### Assumption 4: Optional TID and TDEST

| Aspect | Requirement |
| --- | --- |
| TID Usage | Transaction ID not required for RAPIDS use cases |
| TDEST Usage | Destination routing optional (set by interface type) |
| Default Configuration | TID_WIDTH=0, TDEST_WIDTH=0 for simple streaming |
| Rationale | RAPIDS uses point-to-point connections, no routing needed |

## Master (Transmit) Interface Specification

### AXIS Master Signals

| Signal | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- |
| m_axis_tdata | TDATA_WIDTH | Master->Slave | Yes | Stream data payload |
| m_axis_tvalid | 1 | Master->Slave | Yes | Data valid indicator |
| m_axis_tready | 1 | Slave->Master | Yes | Ready for data (backpressure) |
| m_axis_tlast | 1 | Master->Slave | Yes | Last transfer in packet |
| m_axis_tkeep | TDATA_WIDTH/8 | Master->Slave | Yes | Byte qualifier (valid bytes) |
| m_axis_tid | TID_WIDTH | Master->Slave | Optional | Transaction ID |
| m_axis_tdest | TDEST_WIDTH | Master->Slave | Optional | Routing destination |
| m_axis_tuser | TUSER_WIDTH | Master->Slave | Optional | User sideband data |

### Master Transfer Rules

| Rule | Requirement | Description |
| --- | --- | --- |
| Transfer Occurrence | Transfer occurs when TVALID=1 AND TREADY=1 | Standard AXIS handshake |
| TVALID Assertion | Master can assert TVALID independently of TREADY | Master controls data availability |
| TVALID Stability | Once TVALID=1, all T* signals must remain stable until TREADY=1 | Data integrity requirement |
| TREADY Dependency | Slave can assert TREADY based on TVALID state | Backpressure control |
| TKEEP Alignment | Valid bytes start from byte 0, must be contiguous | Per Assumption 1 |
| TLAST Requirement | TLAST=1 on final beat of every packet | Per Assumption 2 |

## Slave (Receive) Interface Specification

### AXIS Slave Signals

| Signal | Width | Direction | Required | Description |
| --- | --- | --- | --- | --- |
| s_axis_tdata | TDATA_WIDTH | Master->Slave | Yes | Stream data payload |
| s_axis_tvalid | 1 | Master->Slave | Yes | Data valid indicator |
| s_axis_tready | 1 | Slave->Master | Yes | Ready for data (backpressure) |
| s_axis_tlast | 1 | Master->Slave | Yes | Last transfer in packet |
| s_axis_tkeep | TDATA_WIDTH/8 | Master->Slave | Yes | Byte qualifier (valid bytes) |
| s_axis_tid | TID_WIDTH | Master->Slave | Optional | Transaction ID |
| s_axis_tdest | TDEST_WIDTH | Master->Slave | Optional | Routing destination |
| s_axis_tuser | TUSER_WIDTH | Master->Slave | Optional | User sideband data |

### Slave Flow Control

| Aspect | Behavior | Description |
| --- | --- | --- |
| TREADY=1 | Slave ready to accept data | Normal operation |
| TREADY=0 | Slave cannot accept data (backpressure) | Flow control active |
| TREADY Timing | Can be asserted/deasserted on any cycle | Dynamic flow control |
| Buffer Management | TREADY reflects downstream buffer availability | Prevents overflow |

## Packet Structure and Framing

### Single-Beat Packet

Packet with data <= 64 bytes (fits in one transfer):

Clock:      -+ +-+ +-
            +-+ +-+

TVALID:     --+ +---
            +-+

TREADY:     --------
            (always ready)

TDATA:      [Packet Data (64 bytes max)]

TKEEP:      [Valid byte mask (e.g., 64'h00000000FFFFFFFF for 32 bytes)]

TLAST:      --+ +---
            +-+

Transfer: Single beat completes entire packet

### Multi-Beat Packet

Packet with data > 64 bytes (requires multiple transfers):

Clock:      -+ +-+ +-+ +-+ +-
            +-+ +-+ +-+ +-+

TVALID:     --+       +---
            +---------+

TREADY:     ----------------
            (always ready)

TDATA:      [Beat 0][Beat 1][Beat 2][Beat 3]
            64B     64B     64B     32B (partial)

TKEEP:      [64'hFFFF...][64'hFFFF...][64'hFFFF...][64'h00000000FFFFFFFF]
            All valid   All valid   All valid   32 bytes valid

TLAST:      --------------+ +---
            +-+

Transfer: 4 beats (256 bytes total)
- Beats 0-2: Full 64-byte transfers, TLAST=0
- Beat 3: Partial 32-byte transfer, TLAST=1

### Packet with Backpressure

Backpressure applied mid-packet:

Clock:      -+ +-+ +-+ +-+ +-+ +-+ +-
            +-+ +-+ +-+ +-+ +-+ +-+

TVALID:     --+               +---
            +-------------------+

TREADY:     --+ +-+ +---+ +-----
            +-+ +-+ +-+

TDATA:      [Beat 0]  (stall) [Beat 1]  (stall) [Beat 2]

TKEEP:      [Valid]   (held)  [Valid]   (held)  [Valid]

TLAST:      --------------------------+ +---
                                    +-+

Transfer: 3 beats with backpressure
- Beat 0: Transfers immediately
- Stall: TREADY=0, TVALID held high, data held stable
- Beat 1: Transfers after 2 stall cycles
- Stall: Another 1-cycle stall
- Beat 2: Final beat transfers, TLAST=1

Key Observations: 1. Master must hold TVALID=1 and all T* signals stable during backpressure 2. Slave controls flow via TREADY signal 3. Transfer only occurs when both TVALID=1 AND TREADY=1

## TUSER Sideband Signaling

### Purpose and Usage

| Aspect | Description |
| --- | --- |
| Purpose | Carry packet metadata alongside data stream |
| Scope | Valid on first beat of packet only (when new packet starts) |
| Width | Application-specific (0-128 bits) |
| Examples | Packet type, priority, timestamp, sequence number |

### RAPIDS-Specific TUSER Encoding (Delta Network Integration)

TUSER[1:0] - Delta Network Packet Type Encoding:

RAPIDS enforces strict packet type filtering based on TUSER[1:0] encoding as defined by the Delta Network specification:

| TUSER[1:0] | Packet Type | Full Name | Accepted By | Sent By | Purpose |
| --- | --- | --- | --- | --- | --- |
| 2’b00 | PKT_DATA | Data Packet | RAPIDS, Compute Tiles | RAPIDS, Compute Tiles | Compute data transfers |
| 2’b01 | CDA | Compute Direct Access | RAPIDS only | HIVE-C only | Descriptor injection |
| 2’b10 | PKT_CONFIG | Configuration Packet | Tile Routers | HIVE-C, SERV | Configuration |
| 2’b11 | PKT_STATUS | Status Packet | HIVE-C | SERV monitors | Monitoring/status |

RAPIDS Interface Packet Type Rules:

| RAPIDS Interface | Direction | Accepts | Sends | Rejects |
| --- | --- | --- | --- | --- |
| AXIS CDA Input | Slave | CDA (2’b01) | - | PKT_DATA, PKT_CONFIG, PKT_STATUS |
| AXIS Data Output (MM2S) | Master | - | PKT_DATA (2’b00) | CDA, PKT_CONFIG, PKT_STATUS |
| AXIS Data Input (S2MM) | Slave | PKT_DATA (2’b00) | - | CDA, PKT_CONFIG, PKT_STATUS |

Packet Type Validation: - Invalid packet rejection: RAPIDS asserts TREADY=0 when invalid TUSER value detected - Error reporting: Sets error flag in status register - Interrupt generation: Generates interrupt (if enabled) on packet type violation - Hardware enforcement: RAPIDS cannot transmit invalid packet types (design constraint)

Additional TUSER Fields (RAPIDS-Specific):

For PKT_DATA and CDA packets, RAPIDS may use additional TUSER bits beyond [1:0]:

| Bits | Field | Description | Usage |
| --- | --- | --- | --- |
| [1:0] | Packet Type | PKT_DATA or CDA | Mandatory - Delta Network routing |
| [5:2] | Priority | 0-15 priority level | Optional - Descriptor scheduling |
| [7:6] | Reserved | Future use | - |

Usage Pattern: - TUSER valid on every beat of packet (Delta Network requirement) - TUSER[1:0] must be consistent across all beats of same packet - Downstream logic validates TUSER[1:0] on every beat for protocol compliance - Priority field (TUSER[5:2]) used for CDA packet scheduling in descriptor engine

## Flow Control and Backpressure

### Backpressure Mechanisms

| Mechanism | Implementation | Description |
| --- | --- | --- |
| Immediate Backpressure | TREADY=0 for N cycles | Slave cannot accept data |
| Conditional Backpressure | TREADY toggles based on buffer state | Dynamic flow control |
| Sustained Backpressure | TREADY=0 for extended period | Upstream buffer fills |

### Backpressure Propagation

Source SRAM Control -> AXIS Master -> AXIS Slave -> Sink SRAM Control
                         ↑              ↓
                      TREADY      Backpressure
                   (flow control)

Propagation Rules: 1. Slave asserts TREADY=0 when downstream buffer nearly full 2. Master stops sending data (TVALID may remain high, data held) 3. Backpressure propagates to source (SRAM read stalls) 4. System self-regulates to prevent buffer overflow

### Buffer Depth Considerations

| Buffer Type | Recommended Depth | Rationale |
| --- | --- | --- |
| AXIS Input FIFO | 16-32 beats | Absorb backpressure latency |
| AXIS Output FIFO | 16-32 beats | Smooth bursty traffic |
| Packet Buffer | 4-8 packets | Handle multi-packet scenarios |

## Reset Behavior

### Reset Requirements

| Reset Phase | Requirement | Description |
| --- | --- | --- |
| Active Reset | aresetn is active-low reset signal | Standard AXI reset |
| TVALID During Reset | TVALID=0 when aresetn=0 | No spurious transfers |
| TREADY During Reset | TREADY can be 0 or 1 (don’t care) | Slave state undefined |
| State Clearing | All FIFOs/buffers flushed during reset | Clean startup |
| Post-Reset | TVALID=0 for at least 1 cycle after reset release | Stable initialization |

### Reset Timing

Clock:      -+ +-+ +-+ +-+ +-+ +-+ +-
            +-+ +-+ +-+ +-+ +-+ +-+

aresetn:    ------+           +-----
                +-----------+

TVALID:     ------------------+ +---
                            +-+
                            (stable after reset)

TREADY:     ------------------? ? ?--
                            (don't care initially)

Note: TVALID must be 0 during and immediately after reset

## Implementation Benefits

### Streaming Efficiency

| Benefit Area | Advantage | Impact |
| --- | --- | --- |
| Continuous Streaming | No address overhead (unlike AXI4) | Maximum throughput |
| Simple Handshake | Valid-ready protocol only | Minimal control logic |
| Burst Transfers | Multi-beat packets for efficiency | High bandwidth utilization |
| Flow Control | Built-in backpressure mechanism | Prevents data loss |

### Packet-Based Processing

| Benefit Area | Advantage | Impact |
| --- | --- | --- |
| Packet Framing | TLAST delimits packets | Enables packet-level processing |
| Byte Granularity | TKEEP handles partial transfers | Supports variable-length packets |
| Metadata Support | TUSER carries packet info | Rich packet classification |

### Resource Efficiency

| Benefit Area | Simplification | Impact |
| --- | --- | --- |
| No Address Logic | Streaming-only interface | Reduced logic complexity |
| No Transaction IDs | Point-to-point connections | Simplified state machines |
| Optional Signals | TID/TDEST/TUSER as needed | Minimal interface width |
| Byte Alignment | Contiguous bytes only | No complex data steering |

## Timing Requirements

### Setup and Hold Times

| Timing Parameter | Requirement | Description |
| --- | --- | --- |
| TVALID to TREADY | Setup time: 0 ns | Combinational ready allowed |
| TREADY to TVALID | Setup time: 1 clock cycle | TVALID registered before TREADY check |
| T* Signal Stability | Hold until TREADY=1 | Data integrity during backpressure |

### Clock Domain Considerations

| Scenario | Requirement | Solution |
| --- | --- | --- |
| Synchronous Operation | Single clock domain | Direct connection |
| Asynchronous Operation | Different clock domains | Insert AXIS async FIFO |
| Clock Frequency Ratio | Producer faster than consumer | Backpressure handles rate mismatch |

## Validation Requirements

### Functional Validation

| Validation Area | Requirements |
| --- | --- |
| Valid-Ready Handshake | Verify all transfers occur only when TVALID=1 AND TREADY=1 |
| TKEEP Encoding | Verify byte validity matches TKEEP pattern |
| TLAST Assertion | Verify TLAST=1 on final beat of every packet |
| Backpressure Handling | Verify master holds TVALID and T* signals during TREADY=0 |
| Packet Integrity | Verify multi-beat packets reconstruct correctly |
| Byte Alignment | Verify valid bytes are contiguous starting from byte 0 |

### Timing Validation

| Validation Area | Requirements |
| --- | --- |
| Signal Stability | Verify T* signals stable when TVALID=1 until TREADY=1 |
| Reset Behavior | Verify TVALID=0 during and after reset |
| Clock Crossing | Verify async FIFO metastability protection (if used) |

### Stress Testing

| Test Type | Description | Expected Behavior |
| --- | --- | --- |
| Sustained Backpressure | TREADY=0 for 100+ cycles | Master waits without data corruption |
| Rapid Backpressure Toggle | TREADY toggles every cycle | Transfer rate adapts correctly |
| Maximum Throughput | TREADY=1 always | Full bandwidth utilization |
| Single-Beat Packets | All packets fit in one beat | TLAST=1 on every transfer |
| Large Multi-Beat Packets | Packets spanning 100+ beats | Correct packet reconstruction |

## Example Transactions

### Example 1: Single-Beat Packet (32 bytes)

Configuration: - TDATA_WIDTH = 512 bits (64 bytes) - Packet size = 32 bytes - Single transfer

Signals:

Clock cycle:    -+ +-
                +-+

TVALID:         --+ +---
                +-+

TREADY:         ----------
                (ready)

TDATA:          [32 bytes of packet data | 32 bytes unused]

TKEEP:          64'h00000000FFFFFFFF  (first 32 bytes valid)

TLAST:          --+ +---
                +-+

TUSER:          [Packet metadata - type, priority, etc.]

Result: Entire packet transfers in single beat.

### Example 2: Multi-Beat Packet with Backpressure (200 bytes)

Configuration: - TDATA_WIDTH = 512 bits (64 bytes) - Packet size = 200 bytes - Requires 4 beats: 64 + 64 + 64 + 8 bytes

Signals:

Clock cycle:    -+ +-+ +-+ +-+ +-+ +-+ +-
                +-+ +-+ +-+ +-+ +-+ +-+

TVALID:         --+                   +---
                +-----------------------+

TREADY:         --+ +-----+ +-+ +-+ +---
                +-+     +-+ +-+ +-+
                (backpressure cycles 2-3, 5)

Beat 0:         [64 bytes] TKEEP=64'hFFFF..., TLAST=0
                Transfers immediately

Stall:          (cycles 2-3, TREADY=0, master holds data)

Beat 1:         [64 bytes] TKEEP=64'hFFFF..., TLAST=0
                Transfers after stall

Stall:          (cycle 5, TREADY=0 again)

Beat 2:         [64 bytes] TKEEP=64'hFFFF..., TLAST=0
                Transfers after stall

Beat 3:         [8 bytes] TKEEP=64'h00000000000000FF, TLAST=1
                Final beat transfers

TUSER:          [Valid on beat 0 only]

Result: 200-byte packet transfers across 4 beats with intermittent backpressure. Total transfer takes 7 clock cycles (4 beats + 3 stall cycles).

### Example 3: Back-to-Back Packets

Configuration: - Two packets: Packet A (64 bytes), Packet B (128 bytes) - No gaps between packets

Signals:

Clock cycle:    -+ +-+ +-+ +-
                +-+ +-+ +-+

TVALID:         --+       +---
                +---------+

TREADY:         ----------------

Beat 0:         [Packet A - 64 bytes] TKEEP=64'hFFFF..., TLAST=1
                TUSER=[Packet A metadata]

Beat 1:         [Packet B beat 0 - 64 bytes] TKEEP=64'hFFFF..., TLAST=0
                TUSER=[Packet B metadata]

Beat 2:         [Packet B beat 1 - 64 bytes] TKEEP=64'hFFFF..., TLAST=1
                TUSER=(ignored, mid-packet)

Result: Back-to-back packets transfer efficiently without idle cycles. TUSER updates on first beat of each new packet.

## Common Use Cases

### Network Interface Applications

| Use Case | Configuration | Description |
| --- | --- | --- |
| Packet Transmission | 512-bit TDATA, TKEEP, TLAST, TUSER | High-bandwidth network TX |
| Packet Reception | 512-bit TDATA, TKEEP, TLAST, TUSER | High-bandwidth network RX |
| Streaming DMA | 512-bit TDATA, TKEEP, TLAST | Memory-to-network data transfer |
| Flow-Controlled Streaming | Dynamic TREADY | Backpressure-aware streaming |

### Data Path Integration

| Integration Pattern | Description |
| --- | --- |
| Source Path | AXI4 Read -> SRAM -> AXIS Master -> Network |
| Sink Path | Network -> AXIS Slave -> SRAM -> AXI4 Write |
| Loopback | AXIS Master -> AXIS Slave (testing) |
| Multi-Stage Pipeline | AXIS -> Processing -> AXIS (chained) |

### Performance Characteristics

| Metric | Typical Value | Description |
| --- | --- | --- |
| Latency | 1-2 cycles | TVALID assertion to TREADY response |
| Throughput | 1 beat per clock | Sustained rate (no backpressure) |
| Efficiency | 95-100% | With occasional backpressure |
| Packet Rate | Dependent on size | 64-byte packets: ~10Gbps @ 200MHz |

## RAPIDS-Specific Considerations (Delta Network Integration)

### Interface Assignments

| RAPIDS Interface | AXIS Role | Direction | Width | Packet Types | Delta Network Connection |
| --- | --- | --- | --- | --- | --- |
| AXIS CDA Input | Slave (RX) | Input | 128-bit | CDA (2’b01) only | From HIVE-C via Delta Network |
| AXIS Data Output (MM2S) | Master (TX) | Output | 128-bit | PKT_DATA (2’b00) only | To compute tiles via Delta Network |
| AXIS Data Input (S2MM) | Slave (RX) | Input | 128-bit | PKT_DATA (2’b00) only | From compute tiles via Delta Network |
| Internal Source Stream | Master (TX) | Internal | 512-bit | N/A - Internal datapath | SRAM to MM2S engine |
| Internal Sink Stream | Slave (RX) | Internal | 512-bit | N/A - Internal datapath | S2MM engine to SRAM |

Key Points: - Delta Network interfaces: 128-bit TDATA width (standard Delta Network data width) - Internal datapaths: 512-bit TDATA width (high-bandwidth SRAM access) - TUSER enforcement: Hardware validates packet types on all Delta Network interfaces - Virtual Tile 16: RAPIDS addressed as tile 16 in Delta Network topology

### Packet Processing Flow

CDA Descriptor Path (HIVE-C -> RAPIDS):

HIVE-C (VexRiscv RISC-V)
    ↓ CDA packet (TUSER=2'b01)
Delta Network (routes to tile 16)
    ↓ 128-bit AXIS
RAPIDS AXIS CDA Input (packet type validation)
    ↓ Descriptor extraction
Descriptor FIFO (256-bit descriptors)
    ↓ Parsed descriptors
Scheduler (executes MM2S or S2MM operations)

S2MM Path (Compute Tiles -> Memory):

Compute Tiles (0-15)
    ↓ PKT_DATA packet (TUSER=2'b00, TID=source tile)
Delta Network (routes to tile 16)
    ↓ 128-bit AXIS
RAPIDS AXIS Data Input (S2MM)
    ↓ Packet type validation (TUSER=2'b00)
    ↓ TID -> Channel routing (TID[3:0] = channel ID)
S2MM Channel FIFO (per-channel buffering)
    ↓ Internal 512-bit datapath
Sink SRAM Control (buffering)
    ↓ AXI4 Write
DDR Memory

MM2S Path (Memory -> Compute Tiles):

DDR Memory
    ↓ AXI4 Read
Source SRAM Control (buffering)
    ↓ Internal 512-bit datapath
MM2S Data FIFO
    ↓ Packet formation
RAPIDS AXIS Data Output (MM2S)
    ↓ PKT_DATA packet (TUSER=2'b00, TDEST=target tile)
Delta Network (routes to tile 0-15)
    ↓ 128-bit AXIS
Compute Tiles (destination tile)

### Buffer Sizing Guidelines

| Buffer Location | Recommended Size | Rationale |
| --- | --- | --- |
| Network Input FIFO | 32 beats (2KB) | Absorb network burst traffic |
| Network Output FIFO | 32 beats (2KB) | Smooth AXI4 read latency |
| SRAM Depth | 1024-4096 entries | Match typical packet sizes |

## Comparison with Other AXIS Variants

### AXIS vs Full AXI4

| Feature | AXIS | Full AXI4 |
| --- | --- | --- |
| Addressing | No addressing (streaming) | Full address bus |
| Channels | Single data channel | 5 independent channels (AR, R, AW, W, B) |
| Transaction IDs | Optional (rarely used) | Mandatory for out-of-order |
| Burst Support | Continuous streaming | Fixed-length bursts |
| Complexity | Low | High |
| Use Case | Streaming data | Random-access memory |

### AXIS Configuration Trade-offs

| Configuration | Advantages | Disadvantages |
| --- | --- | --- |
| Wide Data Path (512-bit) | High throughput, fewer transfers | More routing resources |
| Narrow Data Path (64-bit) | Simpler routing, lower resource | Lower throughput, more transfers |
| With TUSER | Rich metadata support | Increased interface width |
| Without TUSER | Minimal interface | Limited metadata capability |

## Appendix: Signal Quick Reference

### Mandatory Signals

| Signal | Width | Source | Description |
| --- | --- | --- | --- |
| TDATA | TDATA_WIDTH | Master | Streaming data payload |
| TVALID | 1 | Master | Data valid indicator |
| TREADY | 1 | Slave | Ready for data (backpressure) |
| TLAST | 1 | Master | Last transfer in packet |

### Optional Signals

| Signal | Width | Source | When to Use |
| --- | --- | --- | --- |
| TKEEP | TDATA_WIDTH/8 | Master | Byte-level data validity (recommended) |
| TSTRB | TDATA_WIDTH/8 | Master | Write strobes (rarely used) |
| TID | TID_WIDTH | Master | Transaction routing/identification |
| TDEST | TDEST_WIDTH | Master | Destination routing |
| TUSER | TUSER_WIDTH | Master | User-defined sideband metadata |

### Signal Relationships

TVALID=1 + TREADY=1 -> Transfer occurs
TVALID=1 + TREADY=0 -> Master waits (holds all T* signals)
TVALID=0 + TREADY=? -> No transfer (TREADY don't care)
TLAST=1              -> Final beat of packet
TKEEP[n]=1           -> Byte n of TDATA is valid
TKEEP[n]=0           -> Byte n of TDATA is invalid (ignored)

Next: Chapter 3 - Interface 5: MonBus

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Monitor Bus Architecture and Event Code Organization

## Overview

The Monitor Bus architecture provides a unified, scalable framework for monitoring and error reporting across multiple bus protocols in complex SoC designs. This system supports AXI, APB, Network (Mesh Network on Chip), ARB (Arbiter), CORE, and custom protocols through a standardized 64-bit packet format with protocol-aware event categorization.

## Interface Summary

### Number of Interfaces

- 1 Monitor Bus Output Interface: Unified 64-bit packet stream

- Multiple Protocol Input Interfaces: AXI, APB, Network, ARB, CORE, Custom protocol monitors

- Local Memory Interface: Error/interrupt packet storage

- External Memory Interface: Bulk packet storage

### Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| PACKET_WIDTH | Monitor bus packet width | 64 | 64 |
| PROTOCOL_WIDTH | Protocol identifier width | 3 | 3 |
| EVENT_CODE_WIDTH | Event code width | 4 | 4 |
| PACKET_TYPE_WIDTH | Packet type width | 4 | 4 |
| CHANNEL_ID_WIDTH | Channel identifier width | 6 | 6 |
| UNIT_ID_WIDTH | Unit identifier width | 4 | 4 |
| AGENT_ID_WIDTH | Agent identifier width | 8 | 8 |
| EVENT_DATA_WIDTH | Event data width | 35 | 35 |

## Core Design Assumptions

### Assumption 1: Hierarchical Event Organization

| Aspect | Requirement |
| --- | --- |
| Organization Rule | Protocol -> Packet Type -> Event Code hierarchy |
| Event Space | Each protocol x packet type combination has exactly 16 event codes |
| Mapping | 1:1 mapping between packet types and event codes |
| Rationale | Provides clear, scalable event organization |

### Assumption 2: Protocol Isolation

| Aspect | Requirement |
| --- | --- |
| Isolation Rule | Each protocol owns its event space |
| Conflict Prevention | No cross-protocol event conflicts |
| Independent Evolution | Protocols can evolve independently |
| Rationale | Prevents interference and enables protocol-specific optimization |

### Assumption 3: Two-Tier Memory Architecture

| Aspect | Requirement |
| --- | --- |
| Local Storage | Critical events (errors/interrupts) stored locally |
| External Storage | Non-critical events routed to external memory |
| Routing Decision | Based on packet type configuration |
| Rationale | Balances immediate access with bulk storage needs |

### Assumption 4: Configurable Packet Routing

| Aspect | Requirement |
| --- | --- |
| Routing Rule | Different packet types can route to different destinations |
| Configuration | Base/limit registers define routing per packet type |
| Priority Support | Configurable priority levels per packet type |
| Rationale | Enables flexible memory allocation and access patterns |

## Interface Signal Specification

### Monitor Bus Output Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| mon_packet | 64 | Monitor->System | Monitor packet data |
| mon_valid | 1 | Monitor->System | Packet valid signal |
| mon_ready | 1 | System->Monitor | Ready to accept packet |
| mon_error | 1 | Monitor->System | Monitor error condition |

### Protocol Input Interfaces

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| axi_event | 64 | AXI Monitor->Bus | AXI event packet |
| axi_event_valid | 1 | AXI Monitor->Bus | AXI event valid |
| axi_event_ready | 1 | Bus->AXI Monitor | Ready for AXI event |
| apb_event | 64 | APB Monitor->Bus | APB event packet |
| apb_event_valid | 1 | APB Monitor->Bus | APB event valid |
| apb_event_ready | 1 | Bus->APB Monitor | Ready for APB event |
| network_event | 64 | Network Monitor->Bus | Network event packet |
| network_event_valid | 1 | Network Monitor->Bus | Network event valid |
| network_event_ready | 1 | Bus->Network Monitor | Ready for Network event |
| arb_event | 64 | ARB Monitor->Bus | ARB event packet |
| arb_event_valid | 1 | ARB Monitor->Bus | ARB event valid |
| arb_event_ready | 1 | Bus->ARB Monitor | Ready for ARB event |
| core_event | 64 | CORE Monitor->Bus | CORE event packet |
| core_event_valid | 1 | CORE Monitor->Bus | CORE event valid |
| core_event_ready | 1 | Bus->CORE Monitor | Ready for CORE event |

### Control and Status Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | System clock |
| resetn | 1 | Input | Active-low reset |
| monitor_enable | 1 | Input | Global monitor enable |
| packet_type_enables | 16 | Input | Per-type enable bits |
| local_memory_full | 1 | Output | Local memory full flag |
| external_memory_error | 1 | Output | External memory error |

## Packet Format and Field Allocation

### 64-bit Monitor Bus Packet Structure

| Field | Bits | Width | Description |
| --- | --- | --- | --- |
| Packet Type | [63:60] | 4 | Event category (Error, Completion, etc.) |
| Protocol | [59:57] | 3 | Bus protocol (AXI=0, Network=1, APB=2, ARB=3, CORE=4) |
| Event Code | [56:53] | 4 | Specific events within category |
| Channel ID | [52:47] | 6 | Transaction/channel identifier |
| Unit ID | [46:43] | 4 | Subsystem identifier |
| Agent ID | [42:35] | 8 | Module identifier |
| Event Data | [34:0] | 35 | Event-specific payload |

### Packet Type Definitions

| Value | Name | Purpose | Applicable Protocols |
| --- | --- | --- | --- |
| 0x0 | Error | Protocol violations, response errors | All |
| 0x1 | Completion | Successful transaction completion | All |
| 0x2 | Threshold | Threshold crossed events | All |
| 0x3 | Timeout | Timeout conditions | All |
| 0x4 | Performance | Performance metrics | All |
| 0x5 | Credit | Credit management | Network only |
| 0x6 | Channel | Channel status | Network only |
| 0x7 | Stream | Stream events | Network only |
| 0x8 | Address Match | Address matching | AXI only |
| 0x9 | APB Specific | APB protocol events | APB only |
| 0xA-0xE | Reserved | Future expansion | - |
| 0xF | Debug | Debug and trace events | All |

## Protocol-Specific Event Codes

### AXI Protocol Events

#### Error Events (PktTypeError + PROTOCOL_AXI)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | AXI_ERR_RESP_SLVERR | Slave error response |
| 0x1 | AXI_ERR_RESP_DECERR | Decode error response |
| 0x2 | AXI_ERR_DATA_ORPHAN | Data without command |
| 0x3 | AXI_ERR_RESP_ORPHAN | Response without transaction |
| 0x4 | AXI_ERR_PROTOCOL | Protocol violation |
| 0x5 | AXI_ERR_BURST_LENGTH | Invalid burst length |
| 0x6 | AXI_ERR_BURST_SIZE | Invalid burst size |
| 0x7 | AXI_ERR_BURST_TYPE | Invalid burst type |
| 0x8 | AXI_ERR_ID_COLLISION | ID collision detected |
| 0x9 | AXI_ERR_WRITE_BEFORE_ADDR | Write data before address |
| 0xA | AXI_ERR_RESP_BEFORE_DATA | Response before data complete |
| 0xB | AXI_ERR_LAST_MISSING | Missing LAST signal |
| 0xC | AXI_ERR_STROBE_ERROR | Write strobe error |
| 0xD | AXI_ERR_RESERVED_D | Reserved |
| 0xE | AXI_ERR_RESERVED_E | Reserved |
| 0xF | AXI_ERR_USER_DEFINED | User-defined error |

#### Timeout Events (PktTypeTimeout + PROTOCOL_AXI)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | AXI_TIMEOUT_CMD | Command/Address timeout |
| 0x1 | AXI_TIMEOUT_DATA | Data timeout |
| 0x2 | AXI_TIMEOUT_RESP | Response timeout |
| 0x3 | AXI_TIMEOUT_HANDSHAKE | Handshake timeout |
| 0x4 | AXI_TIMEOUT_BURST | Burst completion timeout |
| 0x5 | AXI_TIMEOUT_EXCLUSIVE | Exclusive access timeout |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | AXI_TIMEOUT_USER_DEFINED | User-defined timeout |

#### Performance Events (PktTypePerf + PROTOCOL_AXI)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | AXI_PERF_ADDR_LATENCY | Address phase latency |
| 0x1 | AXI_PERF_DATA_LATENCY | Data phase latency |
| 0x2 | AXI_PERF_RESP_LATENCY | Response phase latency |
| 0x3 | AXI_PERF_TOTAL_LATENCY | Total transaction latency |
| 0x4 | AXI_PERF_THROUGHPUT | Transaction throughput |
| 0x5 | AXI_PERF_ERROR_RATE | Error rate |
| 0x6 | AXI_PERF_ACTIVE_COUNT | Active transaction count |
| 0x7 | AXI_PERF_BANDWIDTH_UTIL | Bandwidth utilization |
| 0x8 | AXI_PERF_QUEUE_DEPTH | Average queue depth |
| 0x9 | AXI_PERF_BURST_EFFICIENCY | Burst efficiency metric |
| 0xA-0xE | Reserved | Future expansion |
| 0xF | AXI_PERF_USER_DEFINED | User-defined performance |

### APB Protocol Events

#### Error Events (PktTypeError + PROTOCOL_APB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_ERR_PSLVERR | Peripheral slave error |
| 0x1 | APB_ERR_SETUP_VIOLATION | Setup phase protocol violation |
| 0x2 | APB_ERR_ACCESS_VIOLATION | Access phase protocol violation |
| 0x3 | APB_ERR_STROBE_ERROR | Write strobe error |
| 0x4 | APB_ERR_ADDR_DECODE | Address decode error |
| 0x5 | APB_ERR_PROT_VIOLATION | Protection violation (PPROT) |
| 0x6 | APB_ERR_ENABLE_ERROR | Enable phase error |
| 0x7 | APB_ERR_READY_ERROR | PREADY protocol error |
| 0x8-0xE | Reserved | Future expansion |
| 0xF | APB_ERR_USER_DEFINED | User-defined error |

#### Timeout Events (PktTypeTimeout + PROTOCOL_APB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_TIMEOUT_SETUP | Setup phase timeout |
| 0x1 | APB_TIMEOUT_ACCESS | Access phase timeout |
| 0x2 | APB_TIMEOUT_ENABLE | Enable phase timeout (PREADY stuck) |
| 0x3 | APB_TIMEOUT_PREADY_STUCK | PREADY stuck low |
| 0x4 | APB_TIMEOUT_TRANSFER | Overall transfer timeout |
| 0x5-0xE | Reserved | Future expansion |
| 0xF | APB_TIMEOUT_USER_DEFINED | User-defined timeout |

#### Completion Events (PktTypeCompletion + PROTOCOL_APB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_COMPL_TRANS_COMPLETE | Transaction completed |
| 0x1 | APB_COMPL_READ_COMPLETE | Read transaction complete |
| 0x2 | APB_COMPL_WRITE_COMPLETE | Write transaction complete |
| 0x3-0xE | Reserved | Future expansion |
| 0xF | APB_COMPL_USER_DEFINED | User-defined completion |

#### Threshold Events (PktTypeThreshold + PROTOCOL_APB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_THRESH_LATENCY | APB latency threshold |
| 0x1 | APB_THRESH_ERROR_RATE | APB error rate threshold |
| 0x2 | APB_THRESH_ACCESS_COUNT | Access count threshold |
| 0x3 | APB_THRESH_BANDWIDTH | Bandwidth threshold |
| 0x4-0xE | Reserved | Future expansion |
| 0xF | APB_THRESH_USER_DEFINED | User-defined threshold |

#### Performance Events (PktTypePerf + PROTOCOL_APB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_PERF_READ_LATENCY | Read transaction latency |
| 0x1 | APB_PERF_WRITE_LATENCY | Write transaction latency |
| 0x2 | APB_PERF_THROUGHPUT | Transaction throughput |
| 0x3 | APB_PERF_ERROR_RATE | Error rate |
| 0x4 | APB_PERF_ACTIVE_COUNT | Active transaction count |
| 0x5 | APB_PERF_COMPLETED_COUNT | Completed transaction count |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | APB_PERF_USER_DEFINED | User-defined performance |

#### Debug Events (PktTypeDebug + PROTOCOL_APB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_DEBUG_STATE_CHANGE | APB state changed |
| 0x1 | APB_DEBUG_SETUP_PHASE | Setup phase event |
| 0x2 | APB_DEBUG_ACCESS_PHASE | Access phase event |
| 0x3 | APB_DEBUG_ENABLE_PHASE | Enable phase event |
| 0x4 | APB_DEBUG_PSEL_TRACE | PSEL trace |
| 0x5 | APB_DEBUG_PENABLE_TRACE | PENABLE trace |
| 0x6 | APB_DEBUG_PREADY_TRACE | PREADY trace |
| 0x7 | APB_DEBUG_PPROT_TRACE | PPROT trace |
| 0x8 | APB_DEBUG_PSTRB_TRACE | PSTRB trace |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | APB_DEBUG_USER_DEFINED | User-defined debug |

### Network Protocol Events

#### Error Events (PktTypeError + PROTOCOL_MNOC)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | NETWORK_ERR_PARITY | Parity error |
| 0x1 | NETWORK_ERR_PROTOCOL | Protocol violation |
| 0x2 | NETWORK_ERR_OVERFLOW | Buffer/Credit overflow |
| 0x3 | NETWORK_ERR_UNDERFLOW | Buffer/Credit underflow |
| 0x4 | NETWORK_ERR_ORPHAN | Orphaned packet/ACK |
| 0x5 | NETWORK_ERR_INVALID | Invalid type/channel/payload |
| 0x6 | NETWORK_ERR_HEADER_CRC | Header CRC error |
| 0x7 | NETWORK_ERR_PAYLOAD_CRC | Payload CRC error |
| 0x8 | NETWORK_ERR_SEQUENCE | Sequence number error |
| 0x9 | NETWORK_ERR_ROUTE | Routing error |
| 0xA | NETWORK_ERR_DEADLOCK | Deadlock detected |
| 0xB-0xE | Reserved | Future expansion |
| 0xF | NETWORK_ERR_USER_DEFINED | User-defined error |

#### Credit Events (PktTypeCredit + PROTOCOL_MNOC)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | NETWORK_CREDIT_ALLOCATED | Credits allocated |
| 0x1 | NETWORK_CREDIT_CONSUMED | Credits consumed |
| 0x2 | NETWORK_CREDIT_RETURNED | Credits returned |
| 0x3 | NETWORK_CREDIT_OVERFLOW | Credit overflow detected |
| 0x4 | NETWORK_CREDIT_UNDERFLOW | Credit underflow detected |
| 0x5 | NETWORK_CREDIT_EXHAUSTED | All credits exhausted |
| 0x6 | NETWORK_CREDIT_RESTORED | Credits restored |
| 0x7 | NETWORK_CREDIT_EFFICIENCY | Credit efficiency metric |
| 0x8 | NETWORK_CREDIT_LEAK | Credit leak detected |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | NETWORK_CREDIT_USER_DEFINED | User-defined credit event |

#### Channel Events (PktTypeChannel + PROTOCOL_MNOC)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | NETWORK_CHANNEL_OPEN | Channel opened |
| 0x1 | NETWORK_CHANNEL_CLOSE | Channel closed |
| 0x2 | NETWORK_CHANNEL_STALL | Channel stalled |
| 0x3 | NETWORK_CHANNEL_RESUME | Channel resumed |
| 0x4 | NETWORK_CHANNEL_CONGESTION | Channel congestion detected |
| 0x5 | NETWORK_CHANNEL_PRIORITY | Channel priority change |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | NETWORK_CHANNEL_USER_DEFINED | User-defined channel event |

#### Stream Events (PktTypeStream + PROTOCOL_MNOC)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | NETWORK_STREAM_START | Stream started |
| 0x1 | NETWORK_STREAM_END | Stream ended (EOS) |
| 0x2 | NETWORK_STREAM_PAUSE | Stream paused |
| 0x3 | NETWORK_STREAM_RESUME | Stream resumed |
| 0x4 | NETWORK_STREAM_OVERFLOW | Stream buffer overflow |
| 0x5 | NETWORK_STREAM_UNDERFLOW | Stream buffer underflow |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | NETWORK_STREAM_USER_DEFINED | User-defined stream event |

### ARB Protocol Events

#### Error Events (PktTypeError + PROTOCOL_ARB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_ERR_STARVATION | Client request starvation |
| 0x1 | ARB_ERR_ACK_TIMEOUT | Grant ACK timeout |
| 0x2 | ARB_ERR_PROTOCOL_VIOLATION | ACK protocol violation |
| 0x3 | ARB_ERR_CREDIT_VIOLATION | Credit system violation |
| 0x4 | ARB_ERR_FAIRNESS_VIOLATION | Weighted fairness violation |
| 0x5 | ARB_ERR_WEIGHT_UNDERFLOW | Weight credit underflow |
| 0x6 | ARB_ERR_CONCURRENT_GRANTS | Multiple simultaneous grants |
| 0x7 | ARB_ERR_INVALID_GRANT_ID | Invalid grant ID detected |
| 0x8 | ARB_ERR_ORPHAN_ACK | ACK without pending grant |
| 0x9 | ARB_ERR_GRANT_OVERLAP | Overlapping grant periods |
| 0xA | ARB_ERR_MASK_ERROR | Round-robin mask error |
| 0xB | ARB_ERR_STATE_MACHINE | FSM state error |
| 0xC | ARB_ERR_CONFIGURATION | Invalid configuration |
| 0xD-0xE | Reserved | Future expansion |
| 0xF | ARB_ERR_USER_DEFINED | User-defined error |

#### Timeout Events (PktTypeTimeout + PROTOCOL_ARB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_TIMEOUT_GRANT_ACK | Grant ACK timeout |
| 0x1 | ARB_TIMEOUT_REQUEST_HOLD | Request held too long |
| 0x2 | ARB_TIMEOUT_WEIGHT_UPDATE | Weight update timeout |
| 0x3 | ARB_TIMEOUT_BLOCK_RELEASE | Block release timeout |
| 0x4 | ARB_TIMEOUT_CREDIT_UPDATE | Credit update timeout |
| 0x5 | ARB_TIMEOUT_STATE_CHANGE | State machine timeout |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | ARB_TIMEOUT_USER_DEFINED | User-defined timeout |

#### Completion Events (PktTypeCompletion + PROTOCOL_ARB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_COMPL_GRANT_ISSUED | Grant successfully issued |
| 0x1 | ARB_COMPL_ACK_RECEIVED | ACK successfully received |
| 0x2 | ARB_COMPL_TRANSACTION | Complete transaction (grant+ack) |
| 0x3 | ARB_COMPL_WEIGHT_UPDATE | Weight update completed |
| 0x4 | ARB_COMPL_CREDIT_CYCLE | Credit cycle completed |
| 0x5 | ARB_COMPL_FAIRNESS_PERIOD | Fairness analysis period |
| 0x6 | ARB_COMPL_BLOCK_PERIOD | Block period completed |
| 0x7-0xE | Reserved | Future expansion |
| 0xF | ARB_COMPL_USER_DEFINED | User-defined completion |

#### Threshold Events (PktTypeThreshold + PROTOCOL_ARB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_THRESH_REQUEST_LATENCY | Request-to-grant latency threshold |
| 0x1 | ARB_THRESH_ACK_LATENCY | Grant-to-ACK latency threshold |
| 0x2 | ARB_THRESH_FAIRNESS_DEV | Fairness deviation threshold |
| 0x3 | ARB_THRESH_ACTIVE_REQUESTS | Active request count threshold |
| 0x4 | ARB_THRESH_GRANT_RATE | Grant rate threshold |
| 0x5 | ARB_THRESH_EFFICIENCY | Grant efficiency threshold |
| 0x6 | ARB_THRESH_CREDIT_LOW | Low credit threshold |
| 0x7 | ARB_THRESH_WEIGHT_IMBALANCE | Weight imbalance threshold |
| 0x8 | ARB_THRESH_STARVATION_TIME | Starvation time threshold |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | ARB_THRESH_USER_DEFINED | User-defined threshold |

#### Performance Events (PktTypePerf + PROTOCOL_ARB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_PERF_GRANT_ISSUED | Grant issued event |
| 0x1 | ARB_PERF_ACK_RECEIVED | ACK received event |
| 0x2 | ARB_PERF_GRANT_EFFICIENCY | Grant completion efficiency |
| 0x3 | ARB_PERF_FAIRNESS_METRIC | Fairness compliance metric |
| 0x4 | ARB_PERF_THROUGHPUT | Arbitration throughput |
| 0x5 | ARB_PERF_LATENCY_AVG | Average latency measurement |
| 0x6 | ARB_PERF_WEIGHT_COMPLIANCE | Weight compliance metric |
| 0x7 | ARB_PERF_CREDIT_UTILIZATION | Credit utilization efficiency |
| 0x8 | ARB_PERF_CLIENT_ACTIVITY | Per-client activity metric |
| 0x9 | ARB_PERF_STARVATION_COUNT | Starvation event count |
| 0xA | ARB_PERF_BLOCK_EFFICIENCY | Block/unblock efficiency |
| 0xB-0xE | Reserved | Future expansion |
| 0xF | ARB_PERF_USER_DEFINED | User-defined performance |

#### Debug Events (PktTypeDebug + PROTOCOL_ARB)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_DEBUG_STATE_CHANGE | Arbiter state machine change |
| 0x1 | ARB_DEBUG_MASK_UPDATE | Round-robin mask update |
| 0x2 | ARB_DEBUG_WEIGHT_CHANGE | Weight configuration change |
| 0x3 | ARB_DEBUG_CREDIT_UPDATE | Credit level update |
| 0x4 | ARB_DEBUG_CLIENT_MASK | Client enable/disable mask |
| 0x5 | ARB_DEBUG_PRIORITY_CHANGE | Priority level change |
| 0x6 | ARB_DEBUG_BLOCK_EVENT | Block/unblock event |
| 0x7 | ARB_DEBUG_QUEUE_STATUS | Request queue status |
| 0x8 | ARB_DEBUG_COUNTER_SNAPSHOT | Counter values snapshot |
| 0x9 | ARB_DEBUG_FIFO_STATUS | FIFO status change |
| 0xA | ARB_DEBUG_FAIRNESS_STATE | Fairness tracking state |
| 0xB | ARB_DEBUG_ACK_STATE | ACK protocol state |
| 0xC-0xE | Reserved | Future expansion |
| 0xF | ARB_DEBUG_USER_DEFINED | User-defined debug |

### CORE Protocol Events

#### Error Events (PktTypeError + PROTOCOL_CORE)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_ERR_DESCRIPTOR_MALFORMED | Missing magic number (0x900dc0de) |
| 0x1 | CORE_ERR_DESCRIPTOR_BAD_ADDR | Invalid descriptor address |
| 0x2 | CORE_ERR_DATA_BAD_ADDR | Invalid data address (fetch or runtime) |
| 0x3 | CORE_ERR_FLAG_COMPARISON | Flag mask/compare mismatch |
| 0x4 | CORE_ERR_CREDIT_UNDERFLOW | Credit system violation |
| 0x5 | CORE_ERR_STATE_MACHINE | Invalid FSM state transition |
| 0x6 | CORE_ERR_DESCRIPTOR_ENGINE | Descriptor engine FSM error |
| 0x7 | CORE_ERR_FLAG_ENGINE | Flag engine FSM error |
| 0x8 | CORE_ERR_PROGRAM_ENGINE | Program engine FSM error |
| 0x9 | CORE_ERR_DATA_ENGINE | Data engine error |
| 0xA | CORE_ERR_CHANNEL_INVALID | Invalid channel ID |
| 0xB | CORE_ERR_CONTROL_VIOLATION | Control register violation |
| 0xC-0xE | Reserved | Future expansion |
| 0xF | CORE_ERR_USER_DEFINED | User-defined error |

#### Timeout Events (PktTypeTimeout + PROTOCOL_CORE)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_TIMEOUT_DESCRIPTOR_FETCH | Descriptor fetch timeout |
| 0x1 | CORE_TIMEOUT_FLAG_RETRY | Flag comparison retry timeout |
| 0x2 | CORE_TIMEOUT_PROGRAM_WRITE | Program write timeout |
| 0x3 | CORE_TIMEOUT_DATA_TRANSFER | Data transfer timeout |
| 0x4 | CORE_TIMEOUT_CREDIT_WAIT | Credit wait timeout |
| 0x5 | CORE_TIMEOUT_CONTROL_WAIT | Control enable wait timeout |
| 0x6 | CORE_TIMEOUT_ENGINE_RESPONSE | Sub-engine response timeout |
| 0x7 | CORE_TIMEOUT_STATE_TRANSITION | FSM state transition timeout |
| 0x8-0xE | Reserved | Future expansion |
| 0xF | CORE_TIMEOUT_USER_DEFINED | User-defined timeout |

#### Completion Events (PktTypeCompletion + PROTOCOL_CORE)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_COMPL_DESCRIPTOR_LOADED | Descriptor successfully loaded |
| 0x1 | CORE_COMPL_DESCRIPTOR_CHAIN | Descriptor chain completed |
| 0x2 | CORE_COMPL_FLAG_MATCHED | Flag comparison successful |
| 0x3 | CORE_COMPL_PROGRAM_COMPLETED | Post-programming completed |
| 0x4 | CORE_COMPL_DATA_TRANSFER | Data transfer completed |
| 0x5 | CORE_COMPL_CREDIT_CYCLE | Credit cycle completed |
| 0x6 | CORE_COMPL_CHANNEL_COMPLETE | Channel processing complete |
| 0x7 | CORE_COMPL_ENGINE_READY | Sub-engine ready |
| 0x8-0xE | Reserved | Future expansion |
| 0xF | CORE_COMPL_USER_DEFINED | User-defined completion |

#### Threshold Events (PktTypeThreshold + PROTOCOL_CORE)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_THRESH_DESCRIPTOR_QUEUE | Descriptor queue depth threshold |
| 0x1 | CORE_THRESH_CREDIT_LOW | Credit low threshold |
| 0x2 | CORE_THRESH_FLAG_RETRY_COUNT | Flag retry count threshold |
| 0x3 | CORE_THRESH_LATENCY | Processing latency threshold |
| 0x4 | CORE_THRESH_ERROR_RATE | Error rate threshold |
| 0x5 | CORE_THRESH_THROUGHPUT | Throughput threshold |
| 0x6 | CORE_THRESH_ACTIVE_CHANNELS | Active channel count threshold |
| 0x7 | CORE_THRESH_PROGRAM_LATENCY | Program write latency threshold |
| 0x8 | CORE_THRESH_DATA_RATE | Data transfer rate threshold |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | CORE_THRESH_USER_DEFINED | User-defined threshold |

#### Performance Events (PktTypePerf + PROTOCOL_CORE)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_PERF_END_OF_DATA | Stream continuation signal |
| 0x1 | CORE_PERF_END_OF_STREAM | Stream termination signal |
| 0x2 | CORE_PERF_ENTERING_IDLE | FSM returning to idle |
| 0x3 | CORE_PERF_CREDIT_INCREMENTED | Credit added by software |
| 0x4 | CORE_PERF_CREDIT_EXHAUSTED | Credit blocking execution |
| 0x5 | CORE_PERF_STATE_TRANSITION | FSM state change |
| 0x6 | CORE_PERF_DESCRIPTOR_ACTIVE | Data processing started |
| 0x7 | CORE_PERF_FLAG_RETRY | Flag comparison retry |
| 0x8 | CORE_PERF_CHANNEL_ENABLE | Channel enabled by software |
| 0x9 | CORE_PERF_CHANNEL_DISABLE | Channel disabled by software |
| 0xA | CORE_PERF_CREDIT_UTILIZATION | Credit utilization metric |
| 0xB | CORE_PERF_PROCESSING_LATENCY | Total processing latency |
| 0xC | CORE_PERF_QUEUE_DEPTH | Current queue depth |
| 0xD-0xE | Reserved | Future expansion |
| 0xF | CORE_PERF_USER_DEFINED | User-defined performance |

#### Debug Events (PktTypeDebug + PROTOCOL_CORE)

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_DEBUG_FSM_STATE_CHANGE | Descriptor FSM state change |
| 0x1 | CORE_DEBUG_DESCRIPTOR_CONTENT | Descriptor content trace |
| 0x2 | CORE_DEBUG_FLAG_ENGINE_STATE | Flag engine state trace |
| 0x3 | CORE_DEBUG_PROGRAM_ENGINE_STATE | Program engine state trace |
| 0x4 | CORE_DEBUG_CREDIT_OPERATION | Credit system operation |
| 0x5 | CORE_DEBUG_CONTROL_REGISTER | Control register access |
| 0x6 | CORE_DEBUG_ENGINE_HANDSHAKE | Engine handshake trace |
| 0x7 | CORE_DEBUG_QUEUE_STATUS | Queue status change |
| 0x8 | CORE_DEBUG_COUNTER_SNAPSHOT | Counter values snapshot |
| 0x9 | CORE_DEBUG_ADDRESS_TRACE | Address progression trace |
| 0xA | CORE_DEBUG_PAYLOAD_TRACE | Payload content trace |
| 0xB-0xE | Reserved | Future expansion |
| 0xF | CORE_DEBUG_USER_DEFINED | User-defined debug |

## Memory Architecture and Packet Routing

### Two-Tier Memory Architecture

#### Local Error/Interrupt Memory

| Characteristic | Description |
| --- | --- |
| Storage Types | Error Packets (Type 0x0) and Timeout Packets (Type 0x3) |
| Access Method | Immediate CPU access without memory subsystem delays |
| Capacity | Large enough to prevent overflow during error bursts |
| Priority | Critical events requiring immediate attention |
| Indexing | Fast search and retrieval mechanisms |

#### Configurable External Memory

| Characteristic | Description |
| --- | --- |
| Storage Types | Performance, Completion, Threshold, Debug packets |
| Access Method | Base and limit registers define memory regions |
| Capacity | Bulk storage for non-critical events |
| DMA Support | Can be accessed via DMA for efficient transfer |
| Time Stamping | 32-bit timestamp appended when routing externally |

### Routing Configuration

#### Base and Limit Registers

| Register Set | Purpose | Configuration |
| --- | --- | --- |
| Completion Config | Type 0x1 routing | base_addr, limit_addr, enable, priority |
| Threshold Config | Type 0x2 routing | base_addr, limit_addr, enable, priority |
| Performance Config | Type 0x4 routing | base_addr, limit_addr, enable, priority |
| Debug Config | Type 0xF routing | base_addr, limit_addr, enable, priority |

#### Routing Decision Logic

| Packet Type | Destination | Address Calculation |
| --- | --- | --- |
| Error (0x0) | Local Memory | local_error_write_pointer |
| Timeout (0x3) | Local Memory | local_error_write_pointer |
| Completion (0x1) | External Memory | completion_config.base_addr + offset |
| Performance (0x4) | External Memory | performance_config.base_addr + offset |
| Debug (0xF) | External Memory | debug_config.base_addr + offset |

### Address Space Management

#### Memory Layout Example

| Address Range | Usage | Description |
| --- | --- | --- |
| 0x1000_0000 - 0x1000_FFFF | Local Error Memory | Immediate access storage |
| 0x2000_0000 - 0x2001_FFFF | Performance Packets | External bulk storage |
| 0x2010_0000 - 0x2011_FFFF | Completion Packets | External bulk storage |
| 0x2020_0000 - 0x202F_FFFF | Debug Packets | External bulk storage |

### Transaction State and Bus Transaction Structure

#### Transaction State Enumeration

| State | Value | Description | Usage |
| --- | --- | --- | --- |
| TRANS_EMPTY | 3’b000 | Unused entry | Available slot |
| TRANS_ADDR_PHASE | 3’b001 | Address phase active (AXI) / Packet sent (Network) / Setup phase (APB) | Initial phase |
| TRANS_DATA_PHASE | 3’b010 | Data phase active (AXI) / Waiting for ACK (Network) / Access phase (APB) | Data transfer |
| TRANS_RESP_PHASE | 3’b011 | Response phase active (AXI) / ACK received (Network) / Enable phase (APB) | Response handling |
| TRANS_COMPLETE | 3’b100 | Transaction complete | Successful completion |
| TRANS_ERROR | 3’b101 | Transaction has error | Error condition |
| TRANS_ORPHANED | 3’b110 | Orphaned transaction | Missing components |
| TRANS_CREDIT_STALL | 3’b111 | Credit stall (Network only) | Network-specific stall |

#### Enhanced Transaction Structure

| Field | Width | Description | Protocol Usage |
| --- | --- | --- | --- |
| valid | 1 | Entry is valid | All protocol’s |
| protocol | 3 | Protocol type (AXI/Network/APB/ARB/CORE) | All protocols |
| state | 3 | Transaction state | All protocols |
| id | 32 | Transaction ID (AXI) / Sequence (Network) / PSEL encoding (APB) | All protocols |
| addr | 64 | Transaction address / Channel addr / PADDR | All protocols |
| len | 8 | Burst length (AXI) / Packet count (Network) / Always 0 (APB) | AXI, Network |
| size | 3 | Access size (AXI) / Reserved (Network) / Transfer size (APB) | AXI, APB |
| burst | 2 | Burst type (AXI) / Payload type (Network) / PPROT[1:0] (APB) | All protocols |

#### Phase Completion Flags

| Flag | Description | Protocol Usage |
| --- | --- | --- |
| cmd_received | Address phase received / Packet sent / Setup phase | All protocols |
| data_started | Data phase started / ACK expected / Access phase | All protocols |
| data_completed | Data phase completed / ACK received / Enable phase | All protocols |
| resp_received | Response received / Final ACK / PREADY asserted | All protocols |

#### Protocol-Specific Tracking Fields

| Field | Width | Description | Protocol |
| --- | --- | --- | --- |
| channel | 6 | Channel ID (AXI ID / Network channel / PSEL bit position) | All protocols |
| eos_seen | 1 | EOS marker seen | Network only |
| parity_error | 1 | Parity error detected | Network only |
| credit_at_start | 8 | Credits available at start | Network only |
| retry_count | 3 | Number of retries | Network only |
| desc_addr_match | 1 | Descriptor address match detected | AXI only |
| data_addr_match | 1 | Data address match detected | AXI only |
| apb_phase | 2 | Current APB phase | APB only |
| pslverr_seen | 1 | PSLVERR detected | APB only |
| pprot_value | 3 | PPROT value | APB only |
| pstrb_value | 4 | PSTRB value for writes | APB only |
| arb_grant_id | 8 | Current grant ID | ARB only |
| arb_weight | 8 | Current weight value | ARB only |
| core_fsm_state | 3 | Current CORE FSM state | CORE only |
| core_channel_id | 6 | CORE channel identifier | CORE only |

### APB Transaction Phases

| Phase | Value | Description |
| --- | --- | --- |
| APB_PHASE_IDLE | 2’b00 | Bus idle |
| APB_PHASE_SETUP | 2’b01 | Setup phase (PSEL asserted) |
| APB_PHASE_ACCESS | 2’b10 | Access phase (PENABLE asserted) |
| APB_PHASE_ENABLE | 2’b11 | Enable phase (waiting for PREADY) |

### APB Protection Types

| Protection | Value | Description |
| --- | --- | --- |
| APB_PROT_NORMAL | 3’b000 | Normal access |
| APB_PROT_PRIVILEGED | 3’b001 | Privileged access |
| APB_PROT_SECURE | 3’b010 | Secure access |
| APB_PROT_INSTRUCTION | 3’b100 | Instruction access |

### Network Payload Types

| Payload | Value | Description |
| --- | --- | --- |
| NETWORK_PAYLOAD_CONFIG | 2’b00 | CONFIG_PKT |
| NETWORK_PAYLOAD_TS | 2’b01 | TS_PKT |
| NETWORK_PAYLOAD_RDA | 2’b10 | RDA_PKT |
| NETWORK_PAYLOAD_RAW | 2’b11 | RAW_PKT |

### Network ACK Types

| ACK Type | Value | Description |
| --- | --- | --- |
| NETWORK_ACK_STOP | 2’b00 | MSAP_STOP |
| NETWORK_ACK_START | 2’b01 | MSAP_START |
| NETWORK_ACK_CREDIT_ON | 2’b10 | MSAP_CREDIT_ON |
| NETWORK_ACK_STOP_AT_EOS | 2’b11 | MSAP_STOP_AT_EOS |

### ARB State Types

| State | Value | Description |
| --- | --- | --- |
| ARB_STATE_IDLE | 3’b000 | Idle state |
| ARB_STATE_ARBITRATE | 3’b001 | Performing arbitration |
| ARB_STATE_GRANT | 3’b010 | Grant issued, waiting for ACK |
| ARB_STATE_BLOCKED | 3’b011 | Arbitration blocked |
| ARB_STATE_WEIGHT_UPD | 3’b100 | Weight update in progress |
| ARB_STATE_ERROR | 3’b101 | Error state |

### CORE State Types

| State | Value | Description |
| --- | --- | --- |
| CORE_STATE_IDLE | 3’b000 | Idle state |
| CORE_STATE_DESC_FETCH | 3’b001 | Fetching descriptor |
| CORE_STATE_FLAG_CHECK | 3’b010 | Checking flag condition |
| CORE_STATE_PROGRAM_WRITE | 3’b011 | Writing program |
| CORE_STATE_DATA_TRANSFER | 3’b100 | Transferring data |
| CORE_STATE_CREDIT_WAIT | 3’b101 | Waiting for credits |
| CORE_STATE_ERROR | 3’b110 | Error state |

## Configuration and Control

### Monitor Configuration Registers

#### Global Configuration

| Field | Width | Description |
| --- | --- | --- |
| monitor_enable | 1 | Global monitor enable |
| error_local_enable | 1 | Enable local error storage |
| external_route_enable | 1 | Enable external routing |
| unit_id | 4 | Unit identifier |
| agent_id | 8 | Agent identifier |
| packet_type_enables | 16 | Per-type enable bits |

#### Packet Type Enable Mapping

| Bit | Enable | Description |
| --- | --- | --- |
| 0 | PKT_ENABLE_ERROR | Enable error packets |
| 1 | PKT_ENABLE_COMPLETION | Enable completion packets |
| 2 | PKT_ENABLE_THRESHOLD | Enable threshold packets |
| 3 | PKT_ENABLE_TIMEOUT | Enable timeout packets |
| 4 | PKT_ENABLE_PERF | Enable performance packets |
| 5 | PKT_ENABLE_CREDIT | Enable credit packets (Network) |
| 6 | PKT_ENABLE_CHANNEL | Enable channel packets (Network) |
| 7 | PKT_ENABLE_STREAM | Enable stream packets (Network) |
| 8 | PKT_ENABLE_ADDR_MATCH | Enable address match (AXI) |
| 9 | PKT_ENABLE_APB | Enable APB packets |
| 15 | PKT_ENABLE_DEBUG | Enable debug packets |

### Protocol-Specific Configuration

#### AXI Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| active_trans_threshold | 16 | Active transaction threshold |
| latency_threshold | 32 | Latency threshold (cycles) |
| addr_timeout_cnt | 4 | Address timeout count |
| data_timeout_cnt | 4 | Data timeout count |
| resp_timeout_cnt | 4 | Response timeout count |
| burst_boundary_check | 1 | Enable burst boundary checking |
| address_match_enable | 1 | Enable address matching |
| desc_addr_match_base | 64 | Descriptor address match base |
| desc_addr_match_mask | 64 | Descriptor address match mask |
| data_addr_match_base | 64 | Data address match base |
| data_addr_match_mask | 64 | Data address match mask |

#### Network Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| credit_low_threshold | 8 | Credit low threshold |
| packet_rate_threshold | 16 | Packet rate threshold |
| max_route_hops | 8 | Maximum routing hops |
| enable_credit_tracking | 1 | Enable credit tracking |
| enable_deadlock_detect | 1 | Enable deadlock detection |
| deadlock_timeout | 4 | Deadlock detection timeout |

#### ARB Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| grant_timeout_cnt | 16 | Grant ACK timeout count |
| fairness_window | 32 | Fairness analysis window |
| weight_update_enable | 1 | Enable weight tracking |
| starvation_threshold | 16 | Starvation detection threshold |
| efficiency_threshold | 8 | Grant efficiency threshold |

#### CORE Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| descriptor_timeout_cnt | 16 | Descriptor fetch timeout count |
| flag_retry_limit | 8 | Maximum flag retry count |
| credit_low_threshold | 8 | Credit low threshold |
| processing_timeout_cnt | 32 | Processing timeout count |
| enable_descriptor_trace | 1 | Enable descriptor content tracing |
| enable_fsm_trace | 1 | Enable FSM state tracing |

## Validation Requirements

### Functional Validation

| Validation Area | Requirements |
| --- | --- |
| Packet Format | Verify 64-bit packet structure and field encoding |
| Event Organization | Verify hierarchical event code organization |
| Protocol Isolation | Verify independent protocol event spaces |
| Routing Logic | Verify packet routing based on type and configuration |
| Memory Management | Verify local and external memory operations |
| Configuration | Verify register configuration and enable controls |

### Performance Validation

| Validation Area | Requirements |
| --- | --- |
| Throughput | Verify monitor bus can handle peak event rates |
| Latency | Verify low-latency path for critical events |
| Memory Efficiency | Verify efficient memory usage patterns |
| Power Consumption | Verify power-efficient operation |

### Error Handling Validation

| Validation Area | Requirements |
| --- | --- |
| Error Injection | Verify error detection and reporting |
| Overflow Handling | Verify behavior when memories fill |
| Configuration Errors | Verify invalid configuration detection |
| Recovery Mechanisms | Verify error recovery procedures |

## Usage Examples

### Creating Monitor Packets

| Packet Type | Example Usage |
| --- | --- |
| AXI Error | Protocol=AXI, Type=Error, Code=AXI_ERR_RESP_SLVERR |
| Network Credit | Protocol=Network, Type=Credit, Code=NETWORK_CREDIT_EXHAUSTED |
| APB Performance | Protocol=APB, Type=Performance, Code=APB_PERF_TOTAL_LATENCY |
| ARB Threshold | Protocol=ARB, Type=Threshold, Code=ARB_THRESH_FAIRNESS_DEV |
| CORE Completion | Protocol=CORE, Type=Completion, Code=CORE_COMPL_DESCRIPTOR_LOADED |

### Packet Decoding

| Decoding Step | Method |
| --- | --- |
| Extract Type | packet[63:60] |
| Extract Protocol | packet[59:57] |
| Extract Event Code | packet[56:53] |
| Extract Channel ID | packet[52:47] |
| Extract Event Data | packet[34:0] |

### Monitor Bus Packet Helper Functions

#### Packet Field Extraction

| Function | Return Type | Description |
| --- | --- | --- |
| get_packet_type(pkt) | logic [3:0] | Extract packet type [63:60] |
| get_protocol_type(pkt) | protocol_type_t | Extract protocol [59:57] |
| get_event_code(pkt) | logic [3:0] | Extract event code [56:53] |
| get_channel_id(pkt) | logic [5:0] | Extract channel ID [52:47] |
| get_unit_id(pkt) | logic [3:0] | Extract unit ID [46:43] |
| get_agent_id(pkt) | logic [7:0] | Extract agent ID [42:35] |
| get_event_data(pkt) | logic [34:0] | Extract event data [34:0] |

#### Packet Creation Function

| Function | Parameters | Description |
| --- | --- | --- |
| create_monitor_packet() | packet_type, protocol, event_code, channel_id, unit_id, agent_id, event_data | Create complete 64-bit packet |

#### Event Code Creation Functions

| Function | Parameter | Description |
| --- | --- | --- |
| create_axi_error_event() | axi_error_code_t | Create AXI error event code |
| create_axi_timeout_event() | axi_timeout_code_t | Create AXI timeout event code |
| create_axi_completion_event() | axi_completion_code_t | Create AXI completion event code |
| create_axi_threshold_event() | axi_threshold_code_t | Create AXI threshold event code |
| create_axi_performance_event() | axi_performance_code_t | Create AXI performance event code |
| create_axi_addr_match_event() | axi_addr_match_code_t | Create AXI address match event code |
| create_axi_debug_event() | axi_debug_code_t | Create AXI debug event code |
| create_apb_error_event() | apb_error_code_t | Create APB error event code |
| create_apb_timeout_event() | apb_timeout_code_t | Create APB timeout event code |
| create_apb_completion_event() | apb_completion_code_t | Create APB completion event code |
| create_network_error_event() | network_error_code_t | Create Network error event code |
| create_network_timeout_event() | network_timeout_code_t | Create Network timeout event code |
| create_network_completion_event() | network_completion_code_t | Create Network completion event code |
| create_network_credit_event() | network_credit_code_t | Create Network credit event code |
| create_network_channel_event() | network_channel_code_t | Create Network channel event code |
| create_network_stream_event() | network_stream_code_t | Create Network stream event code |
| create_arb_error_event() | arb_error_code_t | Create ARB error event code |
| create_arb_timeout_event() | arb_timeout_code_t | Create ARB timeout event code |
| create_arb_completion_event() | arb_completion_code_t | Create ARB completion event code |
| create_arb_threshold_event() | arb_threshold_code_t | Create ARB threshold event code |
| create_arb_performance_event() | arb_performance_code_t | Create ARB performance event code |
| create_arb_debug_event() | arb_debug_code_t | Create ARB debug event code |
| create_core_error_event() | core_error_code_t | Create CORE error event code |
| create_core_timeout_event() | core_timeout_code_t | Create CORE timeout event code |
| create_core_completion_event() | core_completion_code_t | Create CORE completion event code |
| create_core_threshold_event() | core_threshold_code_t | Create CORE threshold event code |
| create_core_performance_event() | core_performance_code_t | Create CORE performance event code |
| create_core_debug_event() | core_debug_code_t | Create CORE debug event code |

#### Validation Functions

| Function | Parameters | Description |
| --- | --- | --- |
| is_valid_event_for_packet_type() | packet_type, protocol, event_code | Validate event code for packet type and protocol |

#### String Functions for Debugging

| Function | Parameter | Description |
| --- | --- | --- |
| get_axi_error_name() | axi_error_code_t | Get human-readable AXI error name |
| get_arb_error_name() | arb_error_code_t | Get human-readable ARB error name |
| get_core_error_name() | core_error_code_t | Get human-readable CORE error name |
| get_packet_type_name() | logic [3:0] | Get packet type name string |
| get_protocol_name() | protocol_type_t | Get protocol name string |
| get_event_name() | packet_type, protocol, event_code | Get comprehensive event name |

## Debug and Monitoring Signals

### Essential Debug Signals

| Signal | Width | Purpose |
| --- | --- | --- |
| debug_packet_counts | 32 x 16 | Packet count per type |
| debug_protocol_counts | 32 x 5 | Packet count per protocol |
| debug_error_counts | 32 | Total error packet count |
| debug_local_memory_level | 16 | Local memory usage level |
| debug_external_memory_level | 16 | External memory usage level |

### Performance Counters

| Counter | Width | Purpose |
| --- | --- | --- |
| total_packets_processed | 32 | Total packets processed |
| packets_dropped | 32 | Packets dropped due to overflow |
| routing_errors | 32 | Routing configuration errors |
| memory_full_events | 32 | Memory full occurrences |

## Protocol Coverage Summary

### Complete Protocol Event Matrix

| Protocol | Error | Timeout | Completion | Threshold | Performance | Debug | Protocol-Specific |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AXI | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | AddrMatch [PASS] 16 |
| Network | [PASS] 16 | [PASS] 16 | [PASS] 16 | [FAIL] 0 | [FAIL] 0 | [FAIL] 0 | Credit/Channel/Stream [PASS] 48 |
| APB | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | None |
| ARB | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | None |
| CORE | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | [PASS] 16 | None |

Total Event Codes: 544 defined across all protocols and packet types.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Programming

TBD