RTL Design Sherpa

HIVE Hardware Architecture Specification 0.2

October 13, 2026

Table of Contents

1 Hive Has Index	12

2 Document Information	13

2.1 HIVE Hardware Architecture Specification	13

2.2 Document Purpose	13

2.3 References	13

2.4 Terminology	14

2.5 Revision History	15

3 Purpose and Scope	17

3.1 Document Purpose	17

3.2 What HIVE Is	17

3.2.1 Status: specification phase, no RTL yet	18

3.3 Scope	18

3.3.1 In Scope	18

3.3.2 Explicit Non-Goals	18

3.4 Intended Audience	19

3.5 Specification Conventions	19

4 System Overview and Context	21

4.1 Executive Summary	21

4.2 Hierarchical Organization	22

4.2.1 Figure 2.1: HIVE control plane	22

4.3 Position in the RAPIDS / Delta / HIVE Ecosystem	22

4.3.1 Figure 2.2: HIVE in the RAPIDS / Delta / HIVE system	23

4.4 Control Flow Example: Matrix Multiply	23

4.5 Virtual Tile Addressing	24

4.5.1 Figure 2.3: Delta mesh and the HIVE virtual tiles	25

4.6 Packet Type Usage by Component	25

4.7 Key Architectural Features	26

5 Subsystem Architecture	28

5.1 Block 1: HIVE-C Master Controller	28

5.2 Block 2: SERV Monitors (16 instances)	29

5.3 Block 3: Control Network	29

5.4 Block 4: Configuration Manager	30

5.5 Inter-Block Communication	31

5.6 Resource Budget (NexysA7 100T)	32

6 Top-Level Interfaces	34

6.1 External Interface Summary	34

6.2 Communication Protocol Rule (owner, 2026-10-13)	34

6.3 Host Interface	35

6.4 AXIS CDA TX (HIVE-C → Delta)	35

6.5 AXIS SERV TX (16×, per-monitor injection)	35

6.6 AXIS HIVE-C RX (status return)	36

6.7 Control Network (internal)	36

6.8 Router Monitor Taps (internal)	36

7 Packet Formats and Routing Rules	37

7.1 AXIS Packet Classification	37

7.2 CDA Descriptor Format (256-bit)	37

7.3 PKT_CONFIG Format (128-bit)	38

7.4 PKT_STATUS Format (128-bit)	38

7.5 TID Enforcement Rule	39

8 Memory Map and Register Spaces	41

8.1 HIVE-C Firmware Memory Map	41

8.2 Router Configuration Registers (per tile)	41

8.3 Soft Reset Register	42

8.4 Fabric IDLE Status Register	42

8.5 Register Access Rules	42

9 HIVE-C Firmware Architecture	43

9.1 Main Loop	43

9.2 Descriptor Injection API	44

9.3 Interrupts	44

10 Descriptor Scheduling	46

10.1 Priority Levels	46

10.2 Scheduling Policy	46

10.3 Channel-Aware S2MM Programming	46

10.4 Worked Example: Matrix-Multiply Layer	47

11 SERV Monitor Programming	49

11.1 Trigger Conditions	49

11.2 Inband Packet Injection	50

11.3 Reporting Discipline	50

12 Network Reconfiguration Workflow	51

12.1 Atomic Switch Protocol	51

12.2 Worked Flow: Mesh to Tree Reduction	51

12.3 Safety Rules	51

13 Performance Characteristics	53

13.1 Latency Targets	53

13.2 Throughput and Overhead Targets	53

13.3 Design Tradeoffs	54

13.4 Success Criteria	54

14 Integration and Verification	56

14.1 RAPIDS Integration	56

14.2 Delta Integration	56

14.2.1 Figure 7.1: HIVE/Delta tile seam — SERV monitor above the Delta-owned NI	57

14.3 System Top Integration	57

14.4 Verification Strategy	58

14.5 Implementation Roadmap	59

14.6 Expansion Path (PoC → Production)	60

15 HIVE Micro-Architecture Specification Index	61

15.1 Document Organization	61

15.1.1 Front Matter	61

15.1.2 Chapter 1: Overview	61

15.1.3 Chapter 2: Functional Blocks	61

15.1.4 Chapter 3: Internal Interfaces	61

15.1.5 Chapter 4: Micro-Architecture Behavior	61

15.1.6 Chapter 5: Verification Hooks	62

15.1.7 Chapter 6: Configuration	62

15.2 Related Documentation	62

16 Product Requirements Document (PRD)	63

16.1 HIVE - Hierarchical Intelligent Vector Environment	63

16.2 1. Executive Summary	63

16.2.1 1.1 Quick Stats	63

16.2.2 1.2 Subsystem Goals	63

16.3 2. Documentation Structure	63

16.3.1 HIVE Hardware Architecture Specification (HAS) — external contract	63

16.3.2 HIVE Micro-Architecture Specification (MAS) — implementation intent	64

16.3.3 Other Documentation	64

16.4 3. Architecture Overview	64

16.4.1 3.1 Top-Level Block Diagram	64

16.4.2 3.2 Resource Budget (NexysA7 100T)	65

16.5 4. Key Features	66

16.5.1 4.1 Hierarchical RISC-V Architecture	66

16.5.2 4.2 Distributed Monitoring	67

16.5.3 4.3 Network Reconfiguration	67

16.5.4 4.4 RAPIDS Integration	67

16.6 5. Interfaces	68

16.6.1 5.1 External Interfaces	68

16.6.2 5.2 Virtual Tile Mapping	68

16.7 6. Use Cases	68

16.7.1 6.1 Descriptor Scheduling	68

16.7.2 6.2 Congestion Management	69

16.7.3 6.3 Network Reconfiguration	69

16.8 7. Test Coverage	69

16.8.1 7.1 Current Status	69

16.9 8. Performance Targets	69

16.9.1 8.1 Latency Targets	69

16.9.2 8.2 Monitoring Overhead	70

16.10 9. Integration Guidelines	70

16.10.1 9.1 HIVE + RAPIDS + Delta Integration	70

16.11 10. Development Status	71

16.11.1 10.1 Current Phase	71

16.11.2 10.2 Roadmap	71

16.11.3 10.3 HIVE-C core strategy — VexRiscv now, falcon-flavored later	71

16.12 11. Educational Value	72

16.13 13. Documentation Generation	72

16.13.1 13.1 Generating PDF/DOCX from the Specifications	72

16.14 14. References	73

16.14.1 14.1 Internal Documentation	73

16.14.2 14.2 Related Subsystems	73

16.14.3 14.3 External References	73

16.15 Navigation	73

17 Claude Code Guide: HIVE Component	75

17.1 Quick Context	75

17.2 Critical Rules for This Component	75

17.2.1 Rule #2: Always Reference the Specifications	75

17.2.2 Rule #3: HIVE System Context	76

17.3 Architecture Quick Reference	76

17.3.1 Block Organization (HIVE System)	76

17.4 Common User Questions and Responses	77

17.4.1 Q: “What’s the difference between HIVE-C and SERV monitors?”	77

17.4.2 Q: “How does HIVE-C send descriptors to RAPIDS?”	77

17.4.3 Q: “How does network reconfiguration work?”	78

17.4.4 Q: “What do SERV monitors track?”	78

17.5 Key Documentation Links	79

17.5.1 Primary Technical Specifications	79

17.5.2 Related Component Specifications	79

17.6 Documentation Generation	79

17.6.1 Generating PDF/DOCX from the Specifications	79

17.7 Remember	80

18 RAPIDS Beats Architecture HAS	81

18.1 Document Organization	81

18.1.1 Front Matter	81

18.1.2 Chapter 1: Overview	81

18.1.3 Chapter 2: Architecture	81

18.1.4 Chapter 3: Interfaces	81

18.1.5 Chapter 4: Use Cases	81

18.1.6 Chapter 5: Programming Model	82

18.1.7 Chapter 6: Performance	82

18.2 Quick Reference	82

18.2.1 RAPIDS Beats at a Glance	82

18.2.2 Interface Summary	82

18.3 Related Documentation	83

18.4 Specification Conventions	83

18.4.1 Signal Naming	83

18.4.2 Diagram Conventions	83

19 Delta Hardware Architecture Specification Index	85

19.1 Overview	85

19.2 Related Modules	85

19.3 Navigation	85

19.3.1 Front Matter	85

19.3.2 Chapter 1: Introduction	85

19.3.3 Chapter 2: System Overview	85

19.3.4 Chapter 3: Architecture	85

19.3.5 Chapter 4: Interfaces	86

19.3.6 Chapter 5: Performance	86

19.3.7 Chapter 6: Integration	86

19.3.8 Chapter 7: Mesh Network	86

20 RISC-V Falcon Suite	88

20.1 The ladder	88

20.1.1 Moved: hive-serv	89

20.1.2 Tetrad engine: gatling-rv32itetrad	89

20.2 Goals per rung	90

20.3 Packaging (per rung, the MC-line treatment)	91

20.4 References and layout	91

List of Tables

Table 1: Reference Documents	13

Table 2: Terminology and Definitions	14

Table 3: Document Revision History	15

Table 4: Explicit non-goals of HIVE	18

Table 5: Intended audience	19

Table 6: Packet type usage by component	25

Table 7: HIVE-C key specifications	28

Table 8: SERV monitor key specifications	29

Table 9: Control network key specifications	30

Table 10: Configuration Manager key specifications	30

Table 11: Virtual configuration contexts	31

Table 12: Inter-block communication summary	31

Table 13: Resource budget, NexysA7 100T	32

Table 14: External interface summary	34

Table 15: Firmware module responsibilities	43

Table 16: HIVE-C interrupt inventory	44

Table 17: Descriptor priority classes	46

Table 18: SERV trigger conditions	49

Table 19: Latency targets	53

Table 20: Throughput and overhead targets	53

Table 21: VC tradeoff (per router estimates)	54

Table 22: SERV interval tradeoff	54

Table 23: Formal property inventory	59

Table 24: Implementation roadmap	59

Table 25: RAPIDS Beats Feature Summary	82

Table 26: External Interface Summary	82

# 1	Hive Has Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	HIVE Hardware Architecture Specification

Document Number: HIVE-HAS-001 Version: 0.3 Status: Draft (pre-RTL — specification phase) Classification: Open Source - MIT License

## 2.2	Document Purpose

This Hardware Architecture Specification (HAS) defines the external, system-level architecture of the HIVE subsystem (Hierarchical Intelligent Vector Environment): what it is, what it controls, how it connects to the host, the RAPIDS DMA engine, and the Delta Network, how packets and registers are encoded at its boundaries, and how it is integrated into a larger system. It is written for consumers of the subsystem — integrators, firmware engineers, and verification engineers — and is deliberately interface-first: ports, packet formats, encodings, and protocols, with rationale where boundary behavior is a design decision rather than an obvious default.

This document does not describe the internal microarchitecture. Block internals, cycle-level behavior, clock/reset implementation detail, and the verification hook plan live in the companion Micro-Architecture Specification (MAS).

Target Audience: - System architects evaluating HIVE for integration alongside RAPIDS and Delta - Hardware engineers wiring HIVE into a system top level - Firmware engineers writing HIVE-C control software or SERV monitor firmware - Verification engineers planning subsystem-level and system-level test coverage

Companion Documents: - HIVE Micro-Architecture Specification (MAS) - internal block-level implementation intent - HIVE PRD (PRD.md) - product requirements, use cases, and roadmap - RAPIDS Beats HAS/MAS - the DMA engine HIVE-C controls - Delta HAS/MAS - the network HIVE-C and the SERV monitors inject packets into

## 2.3	References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | HIVE MAS v0.2 (docs/hive_mas/) | Micro-Architecture Specification (companion) |
| [REF-2] | HIVE PRD (PRD.md) | Product requirements and use cases |
| [REF-3] | RAPIDS Beats HAS (../../../../../dma-ip/rapids/docs/rapids_beats_has/rapids_beats_has_index.md) | RAPIDS DMA engine; CDA consumer |
| [REF-4] | Delta HAS (../../../../../fabric-gen-ip/delta/docs/delta_has/delta_has_index.md) | 4×4 mesh NoC; packet routing fabric |
| [REF-5] | AXI4-Stream Protocol Specification, ARM IHI 0051A | AXIS signal semantics (TUSER/TID/TDEST/TLAST) |
| [REF-6] | VexRiscv (https://github.com/SpinalHDL/VexRiscv) | HIVE-C master core generator |
| [REF-7] | SERV (https://github.com/olofk/serv) | Bit-serial RV32I core used for the 16 monitors |
| [REF-8] | RISC-V Falcon Suite README (../README.md) | Suite ladder and hive-serv placement beside it |

## 2.4	Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| CDA | Compute Direct Access — the packet class (TUSER=2’b01) HIVE-C uses to inject RAPIDS descriptors inband |
| Context | One of 4 pre-loaded routing configurations (systolic, mesh, tree, custom) held by the Configuration Manager |
| Control Network | Dedicated low-bandwidth star network between HIVE-C and the 16 SERV monitors, separate from the Delta data path |
| HIVE-C | The HIVE master controller: one VexRiscv RV32IM core, Delta Virtual Tile 17 |
| Inband Control | Control messages carried through the data network as packets rather than over a separate memory-mapped bus |
| Monitors | The 16 SERV-based per-tile traffic monitors (not to be confused with verification monitors) |
| PKT_DATA / PKT_CONFIG / PKT_STATUS | AXIS packet classes encoded in TUSER[1:0]: 2’b00 compute data, 2’b10 configuration, 2’b11 status |
| Quiesce | The drain-and-hold protocol that empties the network before an atomic context switch |
| SERV | Small Embedded RISC-V — bit-serial RV32I core, ~125 LUTs per instance |
| Virtual Tile | A Delta Network endpoint that is not a physical compute tile: 16 = RAPIDS, 17 = HIVE-C |
| VexRiscv | SpinalHDL-generated RISC-V core family; HIVE-C uses the “Small” RV32IM configuration |

## 2.5	Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-10-12 | seang | Initial HAS release. Migrates and supersedes the v0.3 hive_spec/ + hive_specification.md drafts; resource-budget table corrected (HIVE-only vs. system subtotals disambiguated); delta references repointed to fabric-gen-ip/delta |
| 0.2 | 2026-10-13 | seang | Owner decisions recorded: fabric IDLE status register (PeakRDL bank) is the quiesce check; kestrel loader pattern for firmware load; descriptor mailbox parameter-controlled with dedicated register file default (custom-instruction alternative rejected). Addendum: delta relationship closed — the AXIS fabric generator is the fabric (same fabric as the archived NoC draft); AXIS-only communication rule — all block-boundary traffic is AXI4-Stream, control network AXIS-shaped, AXI4-Lite for registers only, no AXI4 full |
| 0.3 | 2026-10-20 | seang | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |

Last Updated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	Document Purpose

This Hardware Architecture Specification is the integration contract for the HIVE subsystem. It specifies:

- System architecture - the hierarchical control plane (one VexRiscv master, sixteen SERV monitors) and its position in the RAPIDS/Delta ecosystem

- Subsystem architecture - the four functional blocks (HIVE-C, SERV monitors, Control Network, Configuration Manager), their responsibilities, and the resource budget on the proof-of-concept FPGA

- External interfaces - top-level interface classes, the AXIS packet type encoding, the 256-bit CDA descriptor format, PKT_CONFIG/PKT_STATUS formats, and the memory-mapped register spaces

- Programming model - HIVE-C firmware architecture, descriptor scheduling, SERV monitor programming, and the network reconfiguration workflow

- Performance targets - latency, throughput, overhead, and monitoring budget commitments, plus the tradeoff tables those commitments come from

- Integration and verification - RAPIDS/Delta integration recipes, test levels, formal property inventory, and the implementation roadmap

## 3.2	What HIVE Is

HIVE (Hierarchical Intelligent Vector Environment) is a distributed control and monitoring subsystem for the RAPIDS DMA engine and the Delta Network compute fabric. One VexRiscv master controller (HIVE-C) coordinates global operations; sixteen SERV bit-serial cores, one per Delta compute tile, provide distributed traffic monitoring and inband configuration/status injection. HIVE consumes no DSP blocks and a minority of FPGA logic, leaving the fabric free for compute.

HIVE lives in projects/components/compute-eng-ip/hive-serv/, beside the RISC-V falcon suite ladder. It is deliberately not a falcon rung: the ladder teaches one core’s microarchitecture per rung, while HIVE uses RISC-V cores (VexRiscv and SERV, both external projects) as building blocks for a control system. The educational payload here is hierarchical control, distributed monitoring, and inband control packets — not a new CPU.

### 3.2.1	Status: specification phase, no RTL yet

This HAS describes a planned subsystem. No RTL, firmware, or testbenches exist yet; the companion MAS documents implementation intent to seed that work. Where a number in this document is a target rather than a measured result, it is said so. Content migrated here from the earlier v0.3 spec drafts is treated as design intent and remains subject to change during RTL bring-up.

## 3.3	Scope

### 3.3.1	In Scope

- The four HIVE blocks and their responsibilities (HIVE-C, 16× SERV monitors, Control Network, Configuration Manager)

- AXIS packet classification (TUSER[1:0]) and routing rules on the Delta Network

- The 256-bit CDA descriptor format and HIVE-C → RAPIDS injection contract

- PKT_CONFIG and PKT_STATUS formats, including the SERV → HIVE-C reporting path

- Network reconfiguration: the 4 virtual contexts and the atomic switch protocol

- HIVE-C firmware architecture, descriptor scheduling policy, and memory map

- Resource budget on the Digilent NexysA7 100T proof-of-concept platform

- Performance targets and the tradeoff tables behind them

- Integration recipes against RAPIDS and Delta, and the verification strategy

### 3.3.2	Explicit Non-Goals

The following are not provided by HIVE, and integrators must not rely on them:

Explicit non-goals of HIVE

| Non-goal | What provides it instead |
| --- | --- |
| Compute elements (DSP MACs, PE arrays) | Delta Network compute tiles; HIVE only monitors and configures them |
| DDR memory controller | RAPIDS’ AXI4 memory interface; HIVE never touches DDR directly |
| Data-plane packet switching / routing tables | Delta Network routers; HIVE writes their configuration, it does not forward packets |
| General-purpose host OS / driver stack | Bare-metal host commands over UART or AXI4-Lite; no OS driver exists |
| Real-time guarantees (hard deadlines) | Best-effort latency targets; deadline-aware descriptor scheduling is a policy, not a guarantee |
| Cache coherence | None in the PoC; coherent-access flag in the descriptor is reserved for future use |
| RTL, firmware binaries, SimPy models | Planned deliverables; this document is the specification they will be verified against |

## 3.4	Intended Audience

Intended audience

| Audience | Primary use of this document |
| --- | --- |
| System architects | Decide whether HIVE fits the control plane; understand the contracts with RAPIDS and Delta |
| Hardware engineers | Wire hive_top into a system; close timing against the interface classes; plan clock domains |
| Firmware engineers | Structure HIVE-C control software; write SERV monitor firmware; link images for both cores |
| Verification engineers | Plan subsystem-level coverage against Chapter 7 and the MAS verification hooks |

## 3.5	Specification Conventions

- Signal, register, and packet-field names appear in code font exactly as used in the RTL plan (axis_cda_tvalid, MODE_SELECT) so every claim can be grepped once RTL lands.

- Hexadecimal values use 0x or SystemVerilog sizing (32'hFFFF_FFFE); packet type encodings use sized binary (2'b01).

- “Cycle” means one cycle of the relevant clock domain (see MAS clocking chapter for the four-domain map); cross-domain latencies are given as approximate ranges.

- Monitor in this book means an SERV traffic monitor (a HIVE block). The repository’s verification monitor system (Ocelus) is referred to by name to avoid the collision.

- Packet type names follow the Delta convention: PKT_DATA, CDA (called PKT_DESC in early drafts), PKT_CONFIG, PKT_STATUS in TUSER[1:0].

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	System Overview and Context

## 4.1	Executive Summary

HIVE is a distributed control and monitoring subsystem coordinating the RAPIDS DMA engine and the Delta Network compute fabric. A hierarchical RISC-V architecture provides it: one VexRiscv master controller manages global operations, and sixteen lightweight SERV cores — one per Delta compute tile — provide per-tile traffic monitoring and inband descriptor/configuration injection.

Primary design goals: - Low-latency, inband control of RAPIDS DMA through AXIS-native descriptor injection (~10-20 cycles vs. 100+ for memory-mapped polling) - Distributed traffic monitoring across the Delta compute network - Reconfigurable network topologies through virtual configuration switching - A comprehensive educational platform with performance models and modular design

Target platforms: - Proof of Concept: Digilent NexysA7 100T (Xilinx Artix-7 XC7A100T) - Production target: Digilent Genesys2 (Xilinx Kintex-7 325T) - Alternative: Terasic DE10 Standard (Cyclone V SX, Quartus toolchain)

## 4.2	Hierarchical Organization

### 4.2.1	Figure 2.1: HIVE control plane

HIVE control plane: HIVE-C, Control Network, sixteen SERV monitors, over the Delta network

The compute tiles, routers, and network interfaces belong to Delta (see the Delta HAS). HIVE owns the control plane above them: HIVE-C, the sixteen SERV monitors, the Control Network, and the Configuration Manager.

## 4.3	Position in the RAPIDS / Delta / HIVE Ecosystem

Delta relationship (owner, 2026-10-13): the delta component (projects/components/fabric-gen-ip/delta/) — the AXIS fabric generator — is the fabric HIVE communicates with. The archived NoC-mesh draft (noc-ip/delta_spec/) and the generator describe the same fabric; the 2026-10-12 re-home from noc-ip/ was organizational. HIVE’s architecture stands as specified against delta, and HIVE talks to it exclusively over AXI4-Stream (§4.1).

### 4.3.1	Figure 2.2: HIVE in the RAPIDS / Delta / HIVE system

System data flow: host, HIVE-C, Delta network, RAPIDS, compute tiles, DDR

## 4.4	Control Flow Example: Matrix Multiply

The end-to-end flow that exercises every HIVE block:

- Setup (HIVE-C): Host sends “matmul layer N, tiles 0-7”. HIVE-C switches Delta to mesh mode if needed, then generates MM2S descriptors (DDR → tiles) and S2MM descriptors (tiles → DDR).

- Descriptor injection (HIVE-C → RAPIDS): 256-bit descriptors are sent as CDA packets (TUSER=2’b01, TDEST=16); Delta routes them to the RAPIDS descriptor engine.

- Data movement (RAPIDS → tiles): RAPIDS executes MM2S descriptors, reads activations from DDR over AXI4, and emits PKT_DATA (TUSER=2’b00, TDEST=0-7).

- Computation (tiles): Compute elements produce results and send PKT_DATA with TID = source tile. The tile Network Interface overrides TID with the local tile ID (hardware-enforced, see §4.2).

- Result collection (tiles → RAPIDS → DDR): PKT_DATA routed to RAPIDS (TDEST=16); S2MM channels match on TID; results written to DDR; completion interrupt to HIVE-C.

- Monitoring (SERV → HIVE-C): SERV monitors track packet counts and buffer occupancy, alert on congestion, and send PKT_STATUS to HIVE-C (TDEST=17).

## 4.5	Virtual Tile Addressing

HIVE participates in the Delta topology as two virtual tiles:

### 4.5.1	Figure 2.3: Delta mesh and the HIVE virtual tiles

Delta 4x4 mesh of compute tiles with virtual tile 16 (RAPIDS) and 17 (HIVE-C)

Routing implications: - TDEST = 16 routes to the RAPIDS DMA engine - TDEST = 17 routes to the HIVE-C status aggregator - From HIVE-C: CDA packets (TUSER=2’b01) route to RAPIDS - From tiles: PKT_STATUS (TUSER=2’b11) routes to HIVE-C

## 4.6	Packet Type Usage by Component

Packet type usage by component

| Component | Sends | Receives | Purpose |
| --- | --- | --- | --- |
| HIVE-C | CDA (2’b01), PKT_CONFIG (2’b10) | PKT_STATUS (2’b11) | Descriptor injection, network config, monitoring |
| RAPIDS | PKT_DATA (2’b00) | CDA (2’b01), PKT_DATA (2’b00) | DMA data transfers |
| Compute Tiles | PKT_DATA (2’b00) | PKT_DATA (2’b00), PKT_CONFIG (2’b10) | Computation, reconfiguration |
| SERV Monitors | PKT_STATUS (2’b11), PKT_CONFIG (2’b10) | PKT_CONFIG (2’b10) | Traffic reporting, tile control |
| Delta Routers | - | PKT_CONFIG (2’b10) | Routing table updates |

Validation rules (checkable by protocol checkers at tile interfaces): - HIVE-C never sends PKT_DATA or PKT_STATUS - RAPIDS never sends CDA, PKT_CONFIG, or PKT_STATUS - Compute tiles never send CDA, PKT_CONFIG, or PKT_STATUS - SERV monitors send PKT_STATUS and PKT_CONFIG only

## 4.7	Key Architectural Features

1. Hierarchical control. HIVE-C is the single global coordinator; the 16 SERV monitors are lightweight agents. One powerful core where decisions are made, sixteen tiny cores where the data is.

2. Inband control. HIVE-C pushes descriptors as CDA packets through the Delta data fabric — no polling, no sideband descriptor bus. The control path reuses the network that already exists.

3. Distributed monitoring. Each SERV tracks its local router’s traffic and autonomously injects PKT_STATUS on trigger conditions (congestion, errors, periodic reporting). HIVE-C aggregates rather than polls.

4. Reconfigurable topology. Four pre-loaded routing contexts (systolic, mesh, tree, custom) switch atomically in 10-20 cycles under the Configuration Manager’s quiesce protocol.

5. Zero data-path ownership. HIVE carries control and status traffic only; the Delta data plane and the RAPIDS AXI4 path never traverse HIVE logic.

Last Updated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Subsystem Architecture

HIVE consists of four primary functional blocks. Each has one clear purpose; together they provide distributed control and monitoring for the RAPIDS/Delta system.

## 5.1	Block 1: HIVE-C Master Controller

Single VexRiscv RISC-V processor serving as the global coordinator.

Key specifications:

HIVE-C key specifications

| Attribute | Value |
| --- | --- |
| Core | VexRiscv “Small” configuration |
| ISA | RV32IM (no compressed, no floating point) |
| Pipeline | 5-stage |
| Frequency | 100 MHz (PoC), 150+ MHz (optimized target) |
| Memory | 32 KB tightly-coupled I-mem + 32 KB tightly-coupled D-mem (BRAM) |
| External interfaces | UART/AXI4-Lite slave (host), AXIS master (CDA injection), control network master (SERVs) |

Responsibilities: 1. Parse high-level transfer commands from the host 2. Generate RAPIDS DMA descriptors (256-bit CDA packets) 3. Inject descriptors into Delta via AXIS (TUSER=2’b01, TDEST=16) 4. Aggregate traffic statistics from the 16 SERV monitors 5. Manage network reconfiguration sequences 6. Handle interrupt routing and exception processing

Performance targets: descriptor injection < 30 cycles (HIVE-C → RAPIDS); command response < 1 µs @ 100 MHz; all-16-SERV status aggregation < 100 cycles.

## 5.2	Block 2: SERV Monitors (16 instances)

Lightweight per-tile traffic monitors built on SERV bit-serial RISC-V cores.

Key specifications (per instance):

SERV monitor key specifications

| Attribute | Value |
| --- | --- |
| Core | SERV (Small Embedded RISC-V) |
| ISA | RV32I, bit-serial execution (1 bit/cycle) |
| Frequency | 50-100 MHz (shared serv_clk) |
| Resources | ~125 LUTs, 164 FFs, 1 BRAM (2 KB I-mem + 2 KB D-mem) |
| Interfaces | Control network slave (from HIVE-C), AXIS master (PKT_CONFIG/PKT_STATUS), router monitoring taps |

Responsibilities (per tile): 1. Monitor local router traffic (packet counts, buffer occupancy) 2. Detect congestion and anomalies against programmable thresholds 3. Generate inband control packets on trigger conditions 4. Inject configuration updates into the local tile 5. Report statistics to HIVE-C via the control network 6. Support debug/trace packet injection

Monitored metrics (exposed to firmware): - Counters (32-bit, wrapping): pkt_rx_count[4], pkt_tx_count[4] (per direction N/S/E/W), pkt_local_inject, pkt_local_consume, buffer_overflow_count, stall_cycles - Real-time: buffer_occupancy[4] (8-bit per direction), congestion_flag, error_flags (parity, protocol violations)

Performance targets: monitoring overhead < 5% of compute time; congestion detection < 10 cycles; periodic reporting interval 1000 cycles (configurable).

## 5.3	Block 3: Control Network

Dedicated low-bandwidth infrastructure for HIVE-C ↔ SERV command and status exchange. Deliberately separate from the Delta data path: control traffic must not contend with compute traffic, and must remain live during data-plane quiesce.

Control network key specifications

| Attribute | Value |
| --- | --- |
| Topology | Star (HIVE-C master, 16 SERV slaves) |
| Data width | 32 bits (commands and status responses) |
| Arbitration | Round-robin (SERV → HIVE-C uplink) |
| Protocol | Valid/ready handshake, in-order, lossless |

Responsibilities: 1. Deliver control commands from HIVE-C to individual SERV monitors 2. Aggregate status reports from SERVs to HIVE-C 3. Provide reliable, in-order delivery (no packet loss) 4. Prioritize urgent alerts over periodic statistics

Communication patterns: - HIVE-C → SERV (multicast or unicast): configuration commands, threshold updates, firmware control (soft reset, debug mode) - SERV → HIVE-C (uplink): periodic statistics (every 1000 cycles), congestion alerts (immediate), error notifications

Performance targets: command delivery < 10 cycles; status aggregation < 20 cycles across all 16 SERVs; round-robin fairness (no SERV starvation).

## 5.4	Block 4: Configuration Manager

Manages virtual configuration contexts and orchestrates network topology switching.

Configuration Manager key specifications

| Attribute | Value |
| --- | --- |
| Virtual contexts | 4 pre-loaded routing configurations |
| Context storage | BRAM-based context memory (routing tables per tile) |
| Switching protocol | Atomic context switch with packet draining (quiesce) |
| Distribution | PKT_CONFIG broadcast via Delta Network |

The four contexts:

Virtual configuration contexts

| Context | Name | Character |
| --- | --- | --- |
| 0 | Systolic | Nearest-neighbor N/S/E/W fixed routing; router bypass; 1-2 cycles/hop |
| 1 | Packet-switched mesh | Full XY dimension-ordered routing; credit flow control; 2 virtual channels; 3-4 cycles/hop |
| 2 | Tree reduction | Hierarchical aggregation; tiles 12-15 as roots; E-W reduction tree |
| 3 | Custom/debug | User-programmable routing tables; per-tile independent configuration |

Responsibilities: 1. Store multiple virtual configuration contexts 2. Coordinate atomic topology switching across all tiles 3. Manage the quiesce protocol (drain in-flight packets) 4. Broadcast configuration updates via PKT_CONFIG packets 5. Verify successful context-switch completion

Switching sequence (detailed protocol in §5.4): CONFIG_PREPARE → drain/quiesce (5-10 cycles) → SET_ROUTING_MODE broadcast → CONFIG_ACTIVATE. Total latency 10-20 cycles, deterministic; budgeted < 25 cycles including quiesce.

## 5.5	Inter-Block Communication

Inter-block communication summary

| Path | Medium | Direction | Bandwidth | Latency |
| --- | --- | --- | --- | --- |
| HIVE-C ↔ SERVs | Control Network | Bidirectional | Low (32-bit command/status) | < 10 cycles |
| HIVE-C → Delta | AXIS (CDA) | Master | High (256-bit descriptors) | ~10-20 cycles to RAPIDS |
| SERVs → Delta | AXIS (PKT_CONFIG/STATUS) | Master (per SERV) | Medium (128-bit) | ~10-20 cycles through Delta |
| Config Manager → routers | PKT_CONFIG broadcast | Broadcast | Low (infrequent) | < 5 cycles |

## 5.6	Resource Budget (NexysA7 100T)

FPGA target: Xilinx Artix-7 XC7A100T-1CSG324C — 63,400 LUTs, 126,800 FFs, 240 DSPs, 135 BRAM (36Kb) blocks.

Resource budget, NexysA7 100T

| Component | LUTs | FFs | DSPs | BRAM (36Kb) | Notes |
| --- | --- | --- | --- | --- | --- |
| Block 1: HIVE-C Controller | 1,400 | 900 | 0 | 8 | VexRiscv “Small” config |
| Block 2: SERV Monitors (16×) | 2,000 | 2,624 | 0 | 16 | ~125 LUTs per SERV |
| Block 3: Control Network | 1,200 | 800 | 0 | 4 | HIVE-C ↔ SERV comm |
| Block 4: Configuration Manager | 1,500 | 1,200 | 0 | 2 | Virtual contexts |
| HIVE control plane subtotal | 6,100 | 5,524 | 0 | 30 | 9.6% LUT, 22% BRAM |
| Delta Network 4×4 mesh (16 routers) | 8,000 | 6,000 | 0 | 32 | Delta-owned; see Delta HAS |
| HIVE + NoC total | 14,100 | 11,524 | 0 | 62 | 22% LUT, 46% BRAM |
| Available for compute/RAPIDS | 49,300 | 115,276 | 240 | 73 | Remaining resources |

Correction note (v0.1): earlier drafts presented the 14,100-LUT subtotal as “HIVE” while omitting the Delta NoC row from the table — the NoC’s 8,000 LUTs / 6,000 FFs / 32 BRAMs were silently included in the subtotal. The table above separates the HIVE control plane (6,100 LUTs) from the Delta NoC it configures, then gives the combined figure. The “22% LUT / 46% BRAM” occupancy that motivated the PoC budget refers to the combined figure.

Key design constraints: - HIVE consumes zero DSP blocks (all 240 available to compute) - HIVE uses no AXI4 master of its own — it never touches DDR directly - The combined control plane leaves 78% of logic resources for RAPIDS and Delta compute elements

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Top-Level Interfaces

hive_top is the subsystem boundary. Signal groups below are architectural — final port names are fixed with the RTL (see MAS, Chapter 3); integrators should treat the classes here as the contract.

## 6.1	External Interface Summary

External interface summary

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| Host interface | Slave | 32-bit | UART and/or AXI4-Lite for host commands, status, and firmware load |
| AXIS CDA TX | Master | 256-bit | CDA descriptor injection toward RAPIDS (via Delta) |
| AXIS SERV TX (16×) | Master | 128-bit | PKT_CONFIG / PKT_STATUS injection from each SERV monitor |
| AXIS HIVE-C RX | Slave | 128-bit | PKT_STATUS aggregation return into HIVE-C |
| Control Network | Internal | 32-bit | HIVE-C ↔ SERV communication (Block 3) |
| Router monitor taps (16×) | Internal | per Delta | Buffer occupancy, packet counts, error flags from each tile router |

## 6.2	Communication Protocol Rule (owner, 2026-10-13)

Everything that crosses a block boundary toward the fabric is a stream; only register files are memory-mapped.

- AXI4-Stream carries all packet traffic: CDA injection, SERV PKT_CONFIG/PKT_STATUS, and the status return path. AXIS is delta’s native interface, and the tuser/tdest/tid packet classification (§4.2) with the ~10-20 cycle inband latency target only exist on the stream path.

- AXI4-Lite is used exclusively for register windows: the host interface and HIVE-C’s AXIL master window to RAPIDS/Delta/SERV configuration.

- No AXI4 full anywhere in HIVE. HIVE never touches DDR and moves no burst traffic; the full protocol buys nothing here.

- The internal control network is AXIS-shaped too (32-bit frames on tvalid/tready/tlast, tuser[0] = urgent flag) so the repo stream BFMs and the Ocelus monitor system cover it without bespoke protocol work (MAS §2.4).

## 6.3	Host Interface

Two physical options, one command set:

- UART — serial console-style command channel; default on the PoC board (NexysA7 USB-UART bridge). Fixed 115200 8N1 in the PoC; baud is a firmware configuration value, not a synthesis parameter.

- AXI4-Lite slave — memory-mapped command/status window for SoC integration (e.g., a Zynq or PCIe-attached host). Same command encodings as the UART path; selection is a synthesis-time option, not runtime.

The host issues high-level transfer commands (“matmul layer N, tiles 0-7”); HIVE-C decomposes them into descriptor batches. The host never writes RAPIDS registers directly — all DMA control flows through HIVE-C firmware.

## 6.4	AXIS CDA TX (HIVE-C → Delta)

- Width: 256-bit data beats, one descriptor per packet, tlast on every beat (descriptors are single-beat).

- Sideband: tuser[1:0] = 2’b01 (CDA) on every beat; tdest = 16 (RAPIDS virtual tile) for descriptor packets.

- Rate: sustained injection must not stall the descriptor engine; target is back-to-back descriptor issue with < 30-cycle HIVE-C → RAPIDS latency.

## 6.5	AXIS SERV TX (16×, per-monitor injection)

- Width: 128-bit beats; packets are single-beat in the common case.

- Sideband: tuser[1:0] = 2’b10 (PKT_CONFIG) or 2’b11 (PKT_STATUS); tdest = target tile for config writes, 17 (HIVE-C) for status packets routed back through Delta where applicable, or local-tile distribution as defined by the Configuration Manager.

- TID: SERV monitors own their tid for identification in status traffic; the hardware TID-override rule (§4.2) does not apply to SERV-originated packets — only to compute-element PKT_DATA.

## 6.6	AXIS HIVE-C RX (status return)

- Width: 128-bit beats.

- Purpose: receives PKT_STATUS traffic addressed to HIVE-C (virtual tile

  - through the Delta fabric, where the source elected Delta routing rather than the control network (e.g., aggregated tile-level status).

## 6.7	Control Network (internal)

Star topology, HIVE-C master / 16 SERV slaves. 32-bit command/status frames with valid/ready handshake; in-order, lossless delivery. Frame layout is an implementation decision and lives in the MAS (Chapter 3); the architectural guarantees are the latency targets and the ordering/fairness contract in Chapter 3 above.

## 6.8	Router Monitor Taps (internal)

Each SERV monitor observes its local tile router through a passive tap — buffer occupancy per direction, packet counters, error flags. Taps are read-only observation; the SERV cannot inject values into the router data path. SERV-originated actions travel as PKT_CONFIG packets through the normal injection path, never through the tap.

Last Updated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Packet Formats and Routing Rules

All HIVE control traffic is AXIS packets classified by tuser[1:0]. These encodings are shared with the Delta Network contract — Delta owns the routing, HIVE owns the packet classes and payloads.

## 7.1	AXIS Packet Classification

// AXIS TUSER[1:0] encoding
typedef enum logic [1:0] {
    PKT_DATA   = 2'b00,  // Compute data (activations, weights, results)
    PKT_CDA    = 2'b01,  // DMA descriptor ("CDA", Compute Direct Access)
    PKT_CONFIG = 2'b10,  // Configuration command
    PKT_STATUS = 2'b11   // Status/monitoring data
} packet_type_t;

Naming note: early drafts called 2’b01 PKT_DESC; current docs and the Delta contract call it CDA. PKT_CDA is the spelling to use in RTL.

Routing rules:

- PKT_DATA — routed through the Delta mesh to compute tiles

- PKT_CDA — routed to the RAPIDS DMA descriptor queue (virtual tile 16)

- PKT_CONFIG — routed to tile configuration registers / router config

- PKT_STATUS — routed to the HIVE-C monitoring aggregator (virtual tile 17)

## 7.2	CDA Descriptor Format (256-bit)

RAPIDS descriptors injected by HIVE-C (one per packet, single beat):

Bits [255:192]: Source Address (64-bit)
Bits [191:128]: Destination Address (64-bit)
Bits [127:96]:  Transfer Length (32-bit, in bytes)
Bits [95:64]:   2D Stride Configuration (32-bit)
Bits [63:32]:   Control Flags (32-bit)
Bits [31:0]:    Next Descriptor Pointer (32-bit)

Control Flags [31:0]:
  [31:28] - Burst Size (AXI beats)
  [27:24] - Burst Type (FIXED/INCR/WRAP)
  [23:20] - Priority Level (0-15)
  [19:16] - Destination Tile ID
  [15:12] - Source Tile ID (S2MM channel select; see below)
  [11:8]  - Completion Interrupt Vector
  [7]     - Enable 2D Transfer
  [6]     - Enable Scatter-Gather (follow next ptr)
  [5]     - Generate Completion Interrupt
  [4]     - Cache Coherent Access (reserved in PoC; no coherence fabric)
  [3:0]   - Descriptor Type (future expansion)

: CDA descriptor field map

Channel-aware programming: for S2MM (stream-to-memory) descriptors, the source tile ID field selects which RAPIDS S2MM channel the descriptor arms; RAPIDS matches that channel against the tid of incoming result packets from the tiles. HIVE-C must program S2MM descriptors with the exact tile IDs it expects results from — the identity mapping channel = tile_id is the PoC convention.

## 7.3	PKT_CONFIG Format (128-bit)

Bits [127:96]:  Target Tile Mask (32-bit, one bit per tile)
Bits [95:64]:   Configuration Command (32-bit)
Bits [63:0]:    Payload Data (64-bit)

Command Types:
  0x0000_0001: Set Routing Mode (payload = context ID)
  0x0000_0002: Update Router Table Entry
  0x0000_0004: Set Traffic Priority Weights
  0x0000_0008: Enable/Disable Tile
  0x0000_0010: Flush Local Buffers
  0x0000_0020: Reset Statistics Counters
  0x0000_0040: Update Clock Divider
  0x0000_0080: Set Debug Trace Mode

: PKT_CONFIG field map

The target tile mask allows a single broadcast packet to configure multiple tiles simultaneously — the mechanism that makes atomic context switching possible (all routers update mode in the same cycle).

## 7.4	PKT_STATUS Format (128-bit)

Bits [127:120]: Source Tile ID (8-bit)
Bits [119:112]: Status Type (8-bit)
Bits [111:64]:  Timestamp (48-bit cycle counter)
Bits [63:0]:    Payload (type-dependent)

Status Types:
  0x01: Traffic Statistics (packet counts)
  0x02: Buffer Occupancy Report
  0x04: Congestion Alert
  0x08: Error/Exception Report
  0x10: Performance Counter Snapshot
  0x20: Debug Trace Data

: PKT_STATUS field map

The 48-bit timestamp is in the producer’s clock domain; HIVE-C interprets status freshness by interval, not by absolute timestamp comparison across domains.

## 7.5	TID Enforcement Rule

When compute elements send PKT_DATA back toward RAPIDS, the tile Network Interface overrides tid with the local tile ID:

// Network Interface (NI) TX path for compute-element traffic
always_comb begin
    if (source == COMPUTE_ELEMENT && axis_tuser == PKT_DATA) begin
        // Override: RAPIDS must receive the true source tile ID
        axis_tx_tid_out = MY_TILE_ID[3:0];
    end else begin
        // SERV packets (CONFIG, STATUS) pass TID through
        axis_tx_tid_out = axis_tx_tid_in;
    end
end

This guarantees four properties: 1. Tiles cannot spoof source ID (security and correctness) 2. RAPIDS always receives the accurate tid for S2MM channel routing 3. Compute elements need not know their own tile ID 4. SERV monitors retain free use of tid for status identification

Protocol checkers at tile boundaries should flag any compute-element PKT_DATA whose tid disagrees with the egress port’s tile ID.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Memory Map and Register Spaces

Three memory-mapped views exist: the HIVE-C firmware address space, the router configuration registers (written via PKT_CONFIG or HIVE-C’s AXIL master), and the soft-reset register. All offsets are byte addresses.

## 8.1	HIVE-C Firmware Memory Map

0x0000_0000 - 0x0000_7FFF: Instruction memory (32 KB)
0x0000_8000 - 0x0000_FFFF: Data memory (32 KB)
0x4000_0000 - 0x4000_0FFF: RAPIDS register interface (AXIL master window)
0x4000_1000 - 0x4000_1FFF: Delta network control
0x4000_2000 - 0x4000_2FFF: SERV monitor status (16 × 256 bytes)
0x4000_3000 - 0x4000_3FFF: Configuration registers
0x4000_4000 - 0x4000_4FFF: Host interface (UART/AXI4-Lite)

: HIVE-C firmware memory map

The SERV status window (0x4000_2000) exposes one 256-byte record per monitor: counters, occupancy snapshot, error flags, and firmware heartbeat. Layout is fixed with the SERV firmware (see MAS Chapter 3).

## 8.2	Router Configuration Registers (per tile)

Each Delta tile router exposes this register block. HIVE-C reaches them through its AXI4-Lite master window (Delta network control region) or via PKT_CONFIG broadcast.

Base + 0x00: MODE_SELECT     (RW) - Context ID [1:0]
Base + 0x04: ROUTE_TABLE_0   (RW) - Custom routing entry 0
Base + 0x08: ROUTE_TABLE_1   (RW) - Custom routing entry 1
     ...
Base + 0x20: FLOW_CTRL_CFG   (RW) - Credit limits, VC allocation
Base + 0x24: PRIORITY_MASK   (RW) - QoS priority weights
Base + 0x28: STATUS          (RO) - Buffer occupancy, active packets
Base + 0x2C: STATISTICS      (RO) - Packet counters
Base + 0x30: ERROR_FLAGS     (RO) - Overflow, underflow, parity errors
Base + 0x34: DEBUG_TRACE     (RW) - Trace filter configuration

: Router configuration register map

ROUTE_TABLE_n blocks are only meaningful in Context 3 (custom/debug); contexts 0-2 use hardwired routing functions, and the table registers read back as zero.

## 8.3	Soft Reset Register

HIVE supports selective soft resets from configuration space (offset 0x0000 in the HIVE configuration region):

Bits [31:16] - SERV Reset Mask (one bit per SERV monitor)
Bit  [8]     - Delta Network Soft Reset
Bit  [4]     - Control Network Soft Reset
Bit  [0]     - HIVE-C Soft Reset (use with caution)

: Soft reset register

Write 1 to assert, 0 to deassert. Soft resets are synchronous to the target clock domain and follow the reset sequencing rules in the MAS clocking chapter. Use cases: SERV hang recovery without system disruption, clearing Delta state during reconfiguration, and component-level debug.

## 8.4	Fabric IDLE Status Register

A read-only status register in the HIVE configuration region carries the aggregated fabric IDLE signal — the quiescent indication from the Delta fabric covering all participating routers (and the SERV injection paths). HIVE-C firmware reads it to verify quiesce before issuing SET_ROUTING_MODE (§5.4), and the host may read it for bring-up diagnostics. The register is part of the PeakRDL-managed HIVE register bank (generated from rdl/ sources per the repo convention, see MAS §2.5): bit [0] = fabric idle, bits [16:1] reserved (per-router idle granularity is a possible future extension).

## 8.5	Register Access Rules

- Host → HIVE-C: host interface only; the host has no path to RAPIDS, Delta, or SERV registers except through HIVE-C firmware actions.

- HIVE-C → RAPIDS/Delta/SERV: AXI4-Lite master window and PKT_CONFIG broadcast; descriptor data never travels over the register path — it travels as CDA packets.

- SERV → registers: SERV monitors write tile configuration only via PKT_CONFIG injection; their router tap is observation-only.

Last Updated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	HIVE-C Firmware Architecture

HIVE-C runs bare-metal RV32IM firmware. The architecture below is the reference structure; module boundaries are a firmware convention, not an RTL constraint.

main.c              - Initialization and main loop
  ├── rapids_mgr.c  - DMA descriptor management
  ├── config_mgr.c  - Network reconfiguration
  ├── stats_mgr.c   - Performance monitoring aggregation
  ├── interrupt.c   - IRQ handlers (RAPIDS completion, SERV alerts)
  └── host_if.c     - UART/AXI4-Lite command interface

: HIVE-C firmware module structure

Responsibilities by module:

Firmware module responsibilities

| Module | Responsibility |
| --- | --- |
| main.c | Hardware init, context load, main loop |
| rapids_mgr.c | Descriptor pool, batching, channel-aware S2MM programming |
| config_mgr.c | Context selection, quiesce/activate orchestration |
| stats_mgr.c | SERV status collection, congestion policy, performance model updates |
| interrupt.c | RAPIDS completion, SERV alert routing, error handling |
| host_if.c | Command parsing, response formatting, firmware load support |

## 9.1	Main Loop

void hive_main_loop(void) {
    init_hardware();
    load_network_config(CONTEXT_MESH); // Start in mesh mode (Context 1)

    while (1) {
        if (host_command_pending()) {
            handle_host_command();
        }
        if (timer_expired(STATS_INTERVAL)) {
            collect_serv_statistics();
            update_performance_model();
        }
        if (congestion_detected()) {
            adaptive_route_balancing();
        }
        if (rapids_ready() && desc_queue_has_work()) {
            issue_descriptor_batch(MAX_INFLIGHT);
        }
        if (irq_pending()) {
            process_interrupts();
        }
    }
}

The loop is poll-plus-interrupt: fast paths (descriptor issue) poll, slow paths (statistics, host commands) share the same iteration. No RTOS; latency comes from staying in this loop.

## 9.2	Descriptor Injection API

// Inject one 256-bit CDA descriptor toward RAPIDS (virtual tile 16).
// Blocks until the AXIS master accepts the beat.
void inject_descriptor(const descriptor_t *desc);

inject_descriptor is the single path by which descriptors reach RAPIDS — firmware never writes RAPIDS descriptor registers directly. The injection firmware sets tuser = PKT_CDA, tdest = 16, and waits for tready.

## 9.3	Interrupts

HIVE-C interrupt inventory

| Source | Signal | Handler action |
| --- | --- | --- |
| RAPIDS | Descriptor completion | Bookkeeping, next-batch issue |
| RAPIDS | Error | Log, halt or degrade per policy |
| SERV (via control network) | Congestion alert | Policy decision: throttle, reroute, or context switch |
| SERV (via control network) | Error notification | Mark tile, notify host |
| Host interface | Command ready | Wake command parser |

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Descriptor Scheduling

HIVE-C’s scheduler admits descriptor chains — descriptors linked through the next-descriptor pointer with scatter-gather enabled (HAS §4.2) — and schedules the kick: when each chain starts, and on which RAPIDS channel. The kick/flow split is deliberate (owner, 2026-10-13): HIVE-C owns admission and kick ordering; RAPIDS’ built-in flow control owns pacing — producers and consumers (e.g., one cell producing, another consuming) synchronize through hardware handshakes via flags in memory, so a kicked chain executes at the rate the producer/consumer pair sustains without HIVE-C polling.

Scheduling policy orders the kicks:

## 10.1	Priority Levels

Descriptor priority classes

| Level | Class | Examples |
| --- | --- | --- |
| 0 (highest) | Real-time / latency-sensitive | Small control transfers, urgent spills |
| 1 | Standard compute data movement | Activation/weight loads, result drains |
| 2 | Background / prefetch | Next-layer prefetch, cache warming |
| 3 (lowest) | Debug / trace | Trace dumps, instrumentation |

## 10.2	Scheduling Policy

- Round-robin within a priority level

- Strict priority across levels: a non-empty higher level preempts issue (already-injected descriptors are never reordered)

- Age-based promotion: a descriptor waiting 1000 cycles promotes one level

- Deadline-aware tie-break for level-0 descriptors

## 10.3	Channel-Aware S2MM Programming

The critical correctness rule: S2MM descriptors must name the tile that will send the results, because RAPIDS matches incoming PKT_DATA tid against armed S2MM channels.

// Direct mapping: Tile N sends with TID=N, routes to Channel N
#define TILE_TO_CHANNEL(tile_id)  (tile_id)  // Identity mapping

void program_s2mm_descriptor(uint8_t source_tile) {
    descriptor_t desc;
    desc.type         = DESC_S2MM;                  // Network → Memory
    desc.src_addr     = 0;                          // Not used for S2MM
    desc.dst_addr     = result_buffer[source_tile]; // DDR write address
    desc.length       = result_size;
    desc.source_tile  = source_tile;                // Descriptor[15:12]
    desc.dest_tile    = 16;                         // RAPIDS virtual tile
    inject_descriptor(&desc);
    // RAPIDS arms S2MM Channel[source_tile]; a PKT_DATA with tid == source_tile
    // consumes this descriptor.
}

## 10.4	Worked Example: Matrix-Multiply Layer

void execute_matmul_layer(uint8_t num_tiles) {
    // Phase 1: send activations to all participating tiles (MM2S)
    for (int tile = 0; tile < num_tiles; tile++) {
        descriptor_t mm2s_desc;
        mm2s_desc.type        = DESC_MM2S;          // Memory → Network
        mm2s_desc.src_addr    = activation_buffer[tile];
        mm2s_desc.dst_addr    = 0;                  // Not used for MM2S
        mm2s_desc.length      = activation_size;
        mm2s_desc.dest_tile   = tile;               // TDEST routing to tile
        mm2s_desc.source_tile = 0;                  // Not used for MM2S
        mm2s_desc.priority    = 0;
        inject_descriptor(&mm2s_desc);
    }

    // Phase 2: pre-program S2MM descriptors for result collection.
    // CRITICAL: one per expected source tile, channel = tile_id.
    for (int tile = 0; tile < num_tiles; tile++) {
        program_s2mm_descriptor(tile);
    }

    wait_for_all_tiles_complete();
}

If a result arrives with a tid that matches no armed channel, RAPIDS flags it; HIVE-C’s error policy (log, halt, or degrade) applies. A missing S2MM descriptor is a firmware bug, not a hardware fault — this is the channel-mapping invariant the firmware must maintain.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	SERV Monitor Programming

Each SERV runs a small monitor firmware image (hand-optimized RV32I assembly is expected; a SERV executes one bit per cycle, so instruction count directly buys reaction latency). All sixteen instances run the same image; per-tile identity comes from a read-only tile-ID register the wrapper exposes at address 0xF000_0000.

## 11.1	Trigger Conditions

The monitor firmware evaluates five trigger classes:

SERV trigger conditions

| # | Trigger | Default threshold | Action |
| --- | --- | --- | --- |
| 1 | Congestion | buffer_occupancy > 75% | Send PKT_STATUS congestion alert to HIVE-C; optionally throttle local injection |
| 2 | Error | parity error or protocol violation | Send error status packet; halt local injection pending recovery |
| 3 | Periodic report | 1000 cycles | Send statistics snapshot to HIVE-C |
| 4 | Descriptor queue event | RAPIDS completion signal | Notify HIVE-C for next-batch scheduling |
| 5 | Custom | user-defined via debug registers | Inject debug trace packet |

Thresholds, intervals, and enable bits are configurable by HIVE-C over the control network at runtime.

## 11.2	Inband Packet Injection

// SERV code to inject a packet through the AXIS TX FIFO.
// Example: 2D transfer descriptor (PKT_CDA) — descriptors may be
// injected by SERVs as well as by HIVE-C.
void serv_inject_packet(uint32_t *words, uint8_t nwords, uint8_t pkt_type,
                        uint8_t tdest) {
    for (int i = 0; i < nwords; i++) {
        AXIS_TX_FIFO[i] = words[i];
    }
    AXIS_TX_TUSER  = pkt_type;   // 2'b10 PKT_CONFIG, 2'b11 PKT_STATUS, ...
    AXIS_TX_TDEST  = tdest;
    AXIS_TX_TLAST  = 1;
    AXIS_TX_TVALID = 1;
    while (!AXIS_TX_READY) { }   // Wait for handshake
    AXIS_TX_TVALID = 0;
}

The TX FIFO abstracts the AXIS master; depth and handshaking are implemented in the wrapper (MAS Chapter 2). A SERV should never block on tready while holding time-critical counters — the reference firmware snapshots metrics first, then transmits.

## 11.3	Reporting Discipline

- Periodic snapshots ride the control network uplink (32-bit frames, round-robin arbitrated).

- Urgent alerts (congestion, errors) may use either the control network fast path or a PKT_STATUS injection through Delta, per HIVE-C’s runtime configuration; the control network is preferred during data-plane quiesce.

- Timestamps are local serv_clk cycle counts; HIVE-C treats them as sequence indicators, not comparable clocks.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Network Reconfiguration Workflow

Network topology switching is HIVE-C firmware driving the Configuration Manager hardware through a fixed three-command protocol. The sequence is identical every time; only the context ID changes.

## 12.1	Atomic Switch Protocol

1. HIVE-C issues CONFIG_PREPARE
   - Target tiles flush in-flight packets
   - Routers enter quiescent state (5-10 cycles)

2. HIVE-C broadcasts SET_ROUTING_MODE (PKT_CONFIG, command 0x0000_0001)
   - Target tile mask = all participating tiles
   - Payload = context ID (0-3)
   - All tiles simultaneously update MODE_SELECT

3. HIVE-C issues CONFIG_ACTIVATE
   - Tiles resume operation under the new routing

Total latency 10-20 cycles, deterministic; budget < 25 cycles including quiesce. Overhead target < 1% for workloads whose phases run 1000+ cycles.

## 12.2	Worked Flow: Mesh to Tree Reduction

- Host command (or scheduler policy): “switch to tree reduction mode”

- HIVE-C broadcasts CONFIG_PREPARE over the control network; SERV monitors quiesce their local injection

- Routers drain in-flight packets (5-10 cycles; SERV taps confirm occupancy at zero via the STATUS registers)

- HIVE-C broadcasts SET_ROUTING_MODE, payload = context 2, tile mask = all 16 tiles — one PKT_CONFIG packet updates every router in the same cycle

- HIVE-C issues CONFIG_ACTIVATE; the network resumes with the E-W reduction tree active

## 12.3	Safety Rules

- Never switch without quiesce. A SET_ROUTING_MODE issued while packets are in flight is a protocol violation; the Configuration Manager does not hardware-enforce the drain, so firmware must verify the fabric IDLE status register (below) before step 2. A hardware interlock is optional future hardening (MAS §2.5).

- Context 3 tables must be loaded before activation. Custom routing entries are written via Update Router Table Entry (0x0000_0002) packets while the router is quiescent.

- Rollback: on CONFIG_ACTIVATE failure (tile error flags), firmware re-runs the sequence back to the previous context; there is no partial commit by construction (single-cycle mode register update).

Last Updated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Performance Characteristics

All values in this chapter are targets, not measured results — they are the acceptance criteria RTL bring-up will be verified against.

## 13.1	Latency Targets

Latency targets

| Metric | Target | Notes |
| --- | --- | --- |
| Descriptor injection (HIVE-C → RAPIDS) | < 30 cycles | Includes AXIS handshake + Delta traversal |
| CDA packet latency through Delta | ~10-20 cycles | One hop per router; tile 16 is edge-attached |
| Command response (host → HIVE-C action) | < 1 µs @ 100 MHz | Host interface round trip |
| Context switch (incl. quiesce) | < 25 cycles | Deterministic; 10-20 typical |
| Congestion detection (occupancy → alert sent) | < 10 cycles | SERV trigger evaluation + packetize |
| Status packet generation | < 5 cycles | From trigger to tvalid |
| SERV aggregation (all 16 via control network) | < 100 cycles | HIVE-C-side collection; per-uplink < 20 |

## 13.2	Throughput and Overhead Targets

Throughput and overhead targets

| Metric | Target |
| --- | --- |
| Delta Network throughput | ≥ 80% of theoretical maximum |
| SERV monitoring overhead | < 5% of compute time |
| Context-switch overhead | < 1% for 1000+ cycle phases |
| Descriptor processing | Continuous, no stalls at full injection rate |

## 13.3	Design Tradeoffs

Virtual channels per router (Delta-owned; HIVE is the consumer that cares about deadlock freedom and latency):

VC tradeoff (per router estimates)

| VCs | Area (LUTs) | Max Freq (MHz) | Deadlock Free | Latency (cycles) | Verdict |
| --- | --- | --- | --- | --- | --- |
| 1 | 400 | 250 | No | 3.2 | Minimal area, unsafe for XY |
| 2 | 500 | 220 | Yes (XY) | 3.5 | PoC choice |
| 4 | 650 | 180 | Yes | 4.1 | QoS headroom, revisit at 8×8 |

SERV reporting interval (how often each monitor reports):

SERV interval tradeoff

| Interval (cycles) | Overhead (%) | Detection Latency | Memory (bytes) |
| --- | --- | --- | --- |
| 100 | 8.5 | Excellent | 512 |
| 500 | 2.1 | Good | 256 |
| 1000 | 1.2 | Acceptable | PoC default (128) |
| 5000 | 0.3 | Poor | 64 |

## 13.4	Success Criteria

Functional: inband descriptor control demonstrated end-to-end; 16 monitors tracking accurately; context switch < 25 cycles; correct routing for all four packet classes; zero packet loss under normal operation; congestion alert < 10 cycles.

Performance: ≥ 100 MHz on NexysA7 100T; descriptor injection < 30 cycles; NoC ≥ 80% theoretical throughput; SERV overhead < 5%; switch overhead < 1% on 1000+ cycle phases.

Educational: SimPy models predict hardware within ±10%; 100% of RTL modules with passing unit tests; API docs with examples; ≥ 5 tutorials; modular replacement of any block without system redesign.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Integration and Verification

## 14.1	RAPIDS Integration

HIVE-C connects to RAPIDS through two complementary interfaces:

Control interface (HIVE-C AXI4-Lite master → RAPIDS registers): status reads, queue base addresses, DMA parameter configuration. Used for setup and status; not on the data path.

Descriptor injection (HIVE-C AXIS master → Delta → RAPIDS): 256-bit CDA packets classified by tuser=2'b01 and routed to the RAPIDS descriptor FIFO at virtual tile 16. No memory round-trip — 10-20 cycles vs. 100+ for memory-mapped descriptor writes. Execution pacing: RAPIDS’ producer/ consumer flow control (memory-flag handshakes between producer and consumer endpoints) paces chains after the kick — one cell can produce while another consumes without HIVE-C polling per-chain progress; HIVE-C tracks completion via the interrupt inventory (§5.1).

The contract with RAPIDS is exactly the CDA descriptor format in §4.2 plus the channel-aware S2MM rule in §5.2. See the RAPIDS Beats HAS for the consumer side.

## 14.2	Delta Integration

HIVE and Delta meet at three seams:

- Packet classes — HIVE originates CDA/PKT_CONFIG/PKT_STATUS; Delta routes them by tuser/tdest under contexts HIVE selects.

- Virtual tiles — RAPIDS = 16, HIVE-C = 17, edge-attached to the mesh.

- Router configuration — the register block in §4.3, written via PKT_CONFIG broadcast (Configuration Manager) or HIVE-C’s AXIL window.

The tile internals (NI, router, compute element) are Delta’s architecture:

### 14.2.1	Figure 7.1: HIVE/Delta tile seam — SERV monitor above the Delta-owned NI

HIVE/Delta tile seam: SERV monitor, network interface, router, compute element

## 14.3	System Top Integration

// Top-level integration example (architectural port names; see MAS ch.3)
module system_top (
    input  logic        clk_100mhz,
    input  logic        rst_n,
    // Host interface
    input  logic [31:0] host_cmd,
    output logic [31:0] host_resp,
    // External memory (for RAPIDS)
    // ... AXI4 memory interface ...
);

    // HIVE instance
    hive_top u_hive (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .host_cmd       (host_cmd),
        .host_resp      (host_resp),
        .axis_cda_*     (hive_to_delta_cda),
        .axis_status_*  (delta_to_hive_status)
    );

    // Delta Network instance
    delta_network_4x4 u_delta (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .tile16_*       (rapids_interface),  // RAPIDS virtual tile
        .tile17_*       (hive_interface)     // HIVE-C virtual tile
    );

    // RAPIDS instance
    rapids_top u_rapids (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .axis_cda_*     (rapids_interface),  // From Delta
        .m_axi_*        (memory_interface)   // To external memory
    );

endmodule

## 14.4	Verification Strategy

Level 1 — Unit (cocotb): every RTL module gets a dedicated testbench. Coverage target: 100% line, 95%+ branch. Example (SERV congestion trigger):

@cocotb.test()
async def test_congestion_detection(dut):
    """SERV triggers alert when buffer exceeds threshold"""
    clock = Clock(dut.clk, 10, units="ns")
    cocotb.start_soon(clock.start())
    dut.rst_n.value = 0
    await RisingEdge(dut.clk)
    dut.rst_n.value = 1

    dut.congestion_threshold.value = 120  # 75% of 160-entry FIFO
    for i in range(125):                   # Inject with backpressure
        dut.pkt_rx_valid.value = 1
        dut.pkt_rx_ready.value = 0
        await RisingEdge(dut.clk)

    await RisingEdge(dut.clk)
    assert dut.congestion_alert.value == 1
    assert dut.axis_tx_valid.value == 1
    assert dut.axis_tx_tuser.value == 0b11  # PKT_STATUS

Level 2 — Integration: multi-module scenarios (SERV + router, HIVE-C + control network + 16 SERVs, Configuration Manager + Delta); protocol compliance checkers; randomized traffic stress.

Level 3 — System: full HIVE + RAPIDS + Delta against real workload traces (matmul, convolution); SimPy prediction vs. RTL-measured performance (±10% acceptance).

Level 4 — Hardware: NexysA7 bring-up, ILA capture, performance counters vs. model validation.

Formal property inventory (critical properties, formal-verified):

Formal property inventory

| Property | Statement |
| --- | --- |
| Router deadlock freedom | A request denied a grant eventually receives it (XY + 2 VC) |
| Descriptor FIFO ordering | Enqueue order preserved at dequeue per queue |
| Context switch atomicity | No packet crosses a router between SET_ROUTING_MODE and CONFIG_ACTIVATE |
| TID integrity | No compute-element PKT_DATA egresses with tid ≠ tile_id |
| Quiesce completeness | STATUS.active_packets == 0 before mode update |

## 14.5	Implementation Roadmap

Implementation roadmap

| Phase | Scope | Weeks |
| --- | --- | --- |
| 1 — Core infrastructure | VexRiscv integration + memories; one SERV monitor with counters; basic AXIS injection; unit tests | 1-3 |
| 2 — 2×2 prototype | 2×2 mesh + 4 SERVs; control network; config register bank; descriptor-injection integration test | 4-6 |
| 3 — Full 4×4 | 16 routers + SERVs; ≥ 2 virtual contexts; switch logic; HIVE-C scheduler firmware; RAPIDS + Delta end-to-end | 7-10 |
| 4 — Optimization | 100 MHz closure; Artix-7 fit; SimPy-guided tuning | 11-12 |
| 5 — Validation | NexysA7 hardware-in-loop; model-vs-hardware; stress; tutorials | 13-14 |

## 14.6	Expansion Path (PoC → Production)

Kintex-7 325T scaling: 8×8 mesh (64 tiles), 64 SERV monitors, 4 VexRiscv cores under hierarchical control, 150-200 MHz, 8 routing contexts. Estimated control overhead: 50,000 LUTs + 288 BRAM — still leaving 153,800 LUTs and 840 DSPs for compute. Feature extensions beyond scaling: adaptive routing, quality-of-service guarantees, fault tolerance with dynamic rerouting, and ML-assisted traffic prediction on the SERV fabric.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	HIVE Micro-Architecture Specification Index

Hierarchical Intelligent Vector Environment

Version: 0.2 Date: 2026-10-13 Purpose: Block-level micro-architecture specification for the HIVE distributed control and monitoring subsystem — implementation intent for the RTL that has not been written yet.

## 15.1	Document Organization

Note: All chapters linked below for automated document generation.

### 15.1.1	Front Matter

- Document Information

### 15.1.2	Chapter 1: Overview

- Design Philosophy and Machine Organization

- Clocking and Reset

### 15.1.3	Chapter 2: Functional Blocks

- hive_top

- hive_c_controller

- serv_monitor

- control_network

- config_manager

- Common Utilities

### 15.1.4	Chapter 3: Internal Interfaces

- Packages and Wiring

### 15.1.5	Chapter 4: Micro-Architecture Behavior

- The Descriptor Injection Cycle

- The Monitoring Cycle

- The Context Switch Sequence

- Reset and Boot Behavior

### 15.1.6	Chapter 5: Verification Hooks

- Verification Hooks and Observability

### 15.1.7	Chapter 6: Configuration

- Parameters, Defines, and Filelists

## 15.2	Related Documentation

- HIVE HAS - Hardware Architecture Specification (external contract)

- HIVE PRD - Product requirements and roadmap

- RAPIDS Beats MAS - the DMA engine’s internals HIVE-C drives

- Delta MAS - router/NI internals HIVE configures and monitors

- RISC-V Falcon Suite README - suite overview (hive-serv moved to the compute-eng-ip family 2026-10-13)

Last Updated: 2026-10-13 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Product Requirements Document (PRD)

## 16.1	HIVE - Hierarchical Intelligent Vector Environment

Version: 0.2 Date: 2026-10-12 Status: HAS/MAS v0.2 published; Phase 1 RTL in progress (3 of 5 blocks verified, see vault hive-serv TASK-000) Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 16.2	1. Executive Summary

The Hierarchical Intelligent Vector Environment (HIVE) is a distributed control and monitoring subsystem that coordinates the RAPIDS DMA engine and Delta Network. It demonstrates hierarchical RISC-V processor architecture, distributed monitoring, and dynamic network reconfiguration.

### 16.2.1	1.1 Quick Stats

- Architecture: 4 major blocks (HIVE-C, SERV monitors, Control network, Config manager)

- Processors: 1 VexRiscv (master) + 16 SERV (monitors) = 17 RISC-V cores

- Interfaces: Control Network (internal), AXIS (to Delta Network)

- Resource Budget: 6,100 LUTs HIVE control plane (14,100 including the Delta NoC), 30 BRAM on NexysA7 100T

- Status: Specification phase; HAS/MAS v0.2 published 2026-10-12

### 16.2.2	1.2 Subsystem Goals

- Primary: Demonstrate hierarchical control system design patterns

- Secondary: Enable dynamic network reconfiguration and distributed monitoring

- Tertiary: Educational reference for RISC-V integration and NoC control

## 16.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 16.3.1	HIVE Hardware Architecture Specification (HAS) — external contract

Location: projects/components/compute-eng-ip/hive-serv/docs/hive_has/

- Index - HAS structure

- System overview, subsystem architecture (4 blocks), interfaces & packet formats, programming model, performance targets, integration & verification

### 16.3.2	HIVE Micro-Architecture Specification (MAS) — implementation intent

Location: projects/components/compute-eng-ip/hive-serv/docs/hive_mas/

- Index - MAS structure

- Block-level module trees, clocking/reset, internal interfaces, cycle-level behavior of the key operations, verification hooks, parameters/filelists

### 16.3.3	Other Documentation

- README - Quick start and integration guide (to be created)

- CLAUDE - AI assistance guide for this subsystem

- Tasks - none tracked. The vault area was deleted 2026-09-27: hive has no .sv, no tests and no filelists, so its 24 open items described work that had not begun. They are in git history at vault/tracking/projects/components/compute-eng-ip/hive-serv/.

History note: the earlier docs/hive_spec/ + docs/hive_specification.md drafts (v0.3) were migrated into the HAS/MAS and retired 2026-10-12; they remain in git history.

## 16.4	3. Architecture Overview

### 16.4.1	3.1 Top-Level Block Diagram

HIVE System Architecture
┌─────────────────────────────────────────────────────────────┐
│                     HIVE Control Plane                      │
│  ┌────────────────────────────────────────────────────┐    │
│  │   Block 1: HIVE-C Master Controller (VexRiscv)    │    │
│  │   - Global coordination                            │    │
│  │   - Descriptor scheduling                          │    │
│  │   - Performance aggregation                        │    │
│  └──────────────────┬─────────────────────────────────┘    │
│                     │ Block 3: Control Network              │
│          ┌──────────┴──────────┬──────────────┐           │
│          ▼          ▼          ▼              ▼           │
│  ┌──────────┐ ┌──────────┐ ... ┌──────────┐ (16×)        │
│  │ Block 2: │ │ Block 2: │     │ Block 2: │              │
│  │ SERV-0   │ │ SERV-1   │     │ SERV-15  │              │
│  │ Monitor  │ │ Monitor  │     │ Monitor  │              │
│  └────┬─────┘ └────┬─────┘     └────┬─────┘              │
│       │            │                 │                     │
│       │  Block 4: Configuration Manager                    │
│       │  - Virtual context storage                         │
│       │  - Topology switching                              │
│       └────────────┴──────────────────┘                    │
└─────────────────────────────────────────────────────────────┘
         │ AXIS Packets (CDA, PKT_CONFIG, PKT_STATUS)
         ▼
    Delta Network (4×4 Mesh) + RAPIDS DMA Engine

See: docs/hive_has/ch02_system_overview/01_system_overview.md and docs/hive_has/ch03_architecture/01_subsystem_architecture.md for the architecture; docs/hive_mas/ for block internals.

### 16.4.2	3.2 Resource Budget (NexysA7 100T)

FPGA Target: Xilinx Artix-7 XC7A100T-1CSG324C

| Component | LUTs | FFs | DSPs | BRAM (36Kb) | Notes |
| --- | --- | --- | --- | --- | --- |
| Block 1: HIVE-C Controller | 1,400 | 900 | 0 | 8 | VexRiscv “Small” config |
| Block 2: SERV Monitors (16×) | 2,000 | 2,624 | 0 | 16 | 125 LUTs per SERV |
| Block 3: Control Network | 1,200 | 800 | 0 | 4 | HIVE-C ↔ SERV comm |
| Block 4: Configuration Manager | 1,500 | 1,200 | 0 | 2 | Virtual contexts |
| HIVE control plane subtotal | 6,100 | 5,524 | 0 | 30 | 9.6% LUT, 22% BRAM |
| Delta Network 4×4 mesh (16 routers) | 8,000 | 6,000 | 0 | 32 | Delta-owned; see Delta HAS |
| HIVE + NoC total | 14,100 | 11,524 | 0 | 62 | 22% LUT, 46% BRAM |
| Available for Compute/RAPIDS | 49,300 | 115,276 | 240 | 73 | Remaining resources |
| Total Budget | 63,400 | 126,800 | 240 | 135 | 100% |

Correction note (2026-10-12): earlier tables presented 14,100 LUTs as the “HIVE subtotal” while omitting the Delta NoC row; the NoC’s 8,000 LUTs were silently included. The table now separates the HIVE control plane from the Delta NoC. The 22%-LUT / 46%-BRAM occupancy figures refer to the combined total.

Key Design Constraints: - HIVE consumes zero DSP blocks (all 240 DSPs available for compute) - HIVE’s own memory footprint is 30 BRAM blocks (22%); combined with the NoC it is 62 (46%) - HIVE uses 9.6% of LUTs on its own (22% combined with the NoC) - Leaves 78% of logic resources for RAPIDS and Delta Network compute elements

## 16.5	4. Key Features

### 16.5.1	4.1 Hierarchical RISC-V Architecture

| Feature | Status | Description |
| --- | --- | --- |
| VexRiscv master (HIVE-C) | note | RV32IM, 5-stage pipeline, 32KB I-mem + 32KB D-mem |
| SERV monitors (16×) | note | RV32I bit-serial, 2KB I-mem + 2KB D-mem each |
| Control network | note | Star topology, round-robin arbitration |
| Independent firmware | note | Separate binaries for HIVE-C and SERV |

### 16.5.2	4.2 Distributed Monitoring

| Feature | Status | Description |
| --- | --- | --- |
| Per-tile traffic counters | note | Packet counts per direction (N/S/E/W) |
| Buffer occupancy tracking | note | Real-time FIFO fill levels |
| Congestion detection | note | Programmable thresholds, immediate alerts |
| Error detection | note | Parity errors, protocol violations |
| Periodic reporting | note | Configurable interval (default 1000 cycles) |

### 16.5.3	4.3 Network Reconfiguration

| Feature | Status | Description |
| --- | --- | --- |
| Virtual configuration contexts | note | 4 pre-loaded routing modes |
| Atomic context switching | note | Deterministic latency (<25 cycles) |
| Quiesce protocol | note | Drain in-flight packets before switch |
| Broadcast configuration | note | PKT_CONFIG packets to all tiles |

### 16.5.4	4.4 RAPIDS Integration

| Feature | Status | Description |
| --- | --- | --- |
| Inband descriptor injection | note | CDA packets via Delta Network to RAPIDS |
| Low-latency delivery | note | ~10-20 cycles vs. 100+ for memory-mapped |
| AXIS master interface | note | Direct injection into Delta Network |
| Completion tracking | note | PKT_STATUS packets from RAPIDS |

## 16.6	5. Interfaces

### 16.6.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| Host Interface | Slave | 32-bit | UART/AXI4-Lite for host commands |
| AXIS (HIVE-C TX) | Master | 256-bit | CDA packet injection to RAPIDS |
| AXIS (SERV TX, 16×) | Master | 128-bit | PKT_CONFIG, PKT_STATUS injection |
| AXIS (HIVE-C RX) | Slave | 128-bit | PKT_STATUS from network |
| Control Network | Internal | 32-bit | HIVE-C ↔ SERV communication |

See: docs/hive_has/ch04_interfaces/ for the complete interface, packet-format, and register-map specifications.

### 16.6.2	5.2 Virtual Tile Mapping

| Virtual Tile | Entity | Connection |
| --- | --- | --- |
| Tile 16 | RAPIDS DMA | Edge-attached to the 4×4 mesh |
| Tile 17 | HIVE-C Controller | Edge-attached to the 4×4 mesh |

## 16.7	6. Use Cases

### 16.7.1	6.1 Descriptor Scheduling

Scenario: HIVE-C receives high-level transfer command from host

Flow: 1. Host sends command via UART/AXI4-Lite to HIVE-C 2. HIVE-C generates 256-bit CDA descriptor 3. HIVE-C injects CDA packet via AXIS master (TUSER=2’b01, TDEST=16) 4. Delta Network routes to RAPIDS (Virtual Tile 16) 5. RAPIDS descriptor engine processes 6. Completion reported via PKT_STATUS to HIVE-C

### 16.7.2	6.2 Congestion Management

Scenario: SERV monitor detects buffer congestion on tile 7

Flow: 1. SERV-7 monitors router buffer occupancy 2. Threshold exceeded (buffer_occupancy > cfg_threshold) 3. SERV-7 generates PKT_STATUS alert packet 4. Alert routed to HIVE-C (Virtual Tile 17) 5. HIVE-C aggregates alerts, initiates mitigation 6. HIVE-C may trigger network reconfiguration

### 16.7.3	6.3 Network Reconfiguration

Scenario: Switch from packet-switched mesh to tree reduction mode

Flow: 1. HIVE-C issues CONFIG_PREPARE command via control network 2. All SERV monitors flush in-flight packets 3. Routers enter quiescent state 4. HIVE-C broadcasts SET_ROUTING_MODE (PKT_CONFIG, context=2) 5. All tiles simultaneously update routing mode 6. HIVE-C issues CONFIG_ACTIVATE command 7. Network resumes with tree reduction routing

## 16.8	7. Test Coverage

### 16.8.1	7.1 Current Status

Overall: Phase 1 RTL in progress (2026-10-13)

| Component | Spec Status | Implementation Status |
| --- | --- | --- |
| HIVE-C Controller | HAS/MAS v0.2 | Phase 1 in progress (descriptor_gen, axis_cda_master verified) |
| SERV Monitors | HAS/MAS v0.2 | Submodules written; top pending (TASK-000) |
| Control Network | HAS/MAS v0.2 | CN-01..03 verified; rotation scenarios blocked (TASK-000) |
| Configuration Manager | HAS/MAS v0.1 | Pending |
| Integration | HAS/MAS v0.2 | Pending |

## 16.9	8. Performance Targets

### 16.9.1	8.1 Latency Targets

- Descriptor Injection: < 30 cycles (HIVE-C → RAPIDS)

- Command Response: < 1 µs @ 100 MHz

- Monitoring Aggregation: < 100 cycles for all 16 SERV status reads

- Context Switch: < 25 cycles (including quiesce)

### 16.9.2	8.2 Monitoring Overhead

- SERV Monitor Overhead: < 5% of compute time

- Congestion Detection Latency: < 10 cycles

- Periodic Reporting Interval: 1000 cycles (configurable)

See: docs/hive_has/ch06_performance/01_performance.md for the full target set and the tradeoff tables behind them.

## 16.10	9. Integration Guidelines

### 16.10.1	9.1 HIVE + RAPIDS + Delta Integration

// Top-level integration example (architectural port names)
module system_top (
    input  logic        clk_100mhz,
    input  logic        rst_n,
    // Host interface
    input  logic [31:0] host_cmd,
    output logic [31:0] host_resp,
    // External memory (for RAPIDS)
    // ... AXI4 memory interface ...
);

    // HIVE instance
    hive_top u_hive (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .host_cmd       (host_cmd),
        .host_resp      (host_resp),
        .axis_cda_*     (hive_to_delta_cda),
        .axis_status_*  (delta_to_hive_status)
    );

    // Delta Network instance
    delta_network_4x4 u_delta (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .tile16_*       (rapids_interface),  // RAPIDS virtual tile
        .tile17_*       (hive_interface)     // HIVE-C virtual tile
    );

    // RAPIDS instance
    rapids_top u_rapids (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .axis_cda_*     (rapids_interface),  // From Delta
        .m_axi_*        (memory_interface)   // To external memory
    );

endmodule

See: docs/hive_has/ch07_integration/01_integration_and_verification.md for the full integration and verification strategy.

## 16.11	10. Development Status

### 16.11.1	10.1 Current Phase

Phase: Specification (HAS/MAS v0.2 complete, 2026-10-12)

- Hardware Architecture Specification: complete (7 chapters)

- Micro-Architecture Specification: complete (6 chapters)

- RTL: pending (roadmap in the HAS, Chapter 7)

See: nothing tracked – the vault area was deleted 2026-09-27 (unstarted); the former items are in git history.

### 16.11.2	10.2 Roadmap

Near-Term (Q4 2025): - HIVE-C RTL implementation (Phase 1, per HAS ch.7 roadmap) - SERV monitor wrapper implementation - Control network implementation

Long-Term (2026+): - Configuration manager implementation - HIVE-C firmware development - SERV firmware development - Integration with RAPIDS and Delta - FPGA deployment and testing

### 16.11.3	10.3 HIVE-C core strategy — VexRiscv now, falcon-flavored later

HIVE-C’s core sits behind the hive_c_loader contract (AXIL loader window, load-then-run); the LDR testplan verifies that contract, not the core. That deliberate isolation makes the core swappable by parameter (CORE_FLAVOR, to be recorded in the MAS): vexriscv is the default today, peregrine is the intended successor when falcon rung 3 lands. The swap touches only the wrapper’s core-facing glue (VexRiscv native ibus/dbus cmd/rsp vs peregrine’s caches-behind-AXI4); the loader, firmware, and testplan carry over unchanged.

Two purposes, per the owner (2026-10-13):

- System proving ground for the falcon ladder. Every falcon rung is verified in isolation (spike lockstep, riscv-formal, ISA batteries) but never in a system. HIVE-C is the vehicle: real control firmware, real interrupts, a real fabric — the LDR plan becomes peregrine’s bring-up acceptance test the day its RTL exists.

- The “does a more powerful RV make a difference” experiment. The PoC’s headline latencies are fabric-bound (a stronger core will not move the 30-cycle injection target). The differences to measure: interrupt-driven firmware architecture (alert-to-action latency vs polling), scheduler worst case under load, and headroom at the 64-SERV expansion scale. Same firmware, same testplan, measure the deltas — if there is no system-level gain, that is a finding about HIVE’s workload class, not a failed experiment.

## 16.12	11. Educational Value

HIVE demonstrates: - Hierarchical processor architecture (master + agents) - Distributed monitoring and control - RISC-V processor integration (VexRiscv, SERV) - Inband control packet injection - Dynamic network reconfiguration - Resource budget management - Control network design patterns

Target Audience: - Advanced RTL designers - Network-on-chip architects - Embedded systems engineers - RISC-V integration specialists

## 16.13	13. Documentation Generation

### 16.13.1	13.1 Generating PDF/DOCX from the Specifications

Tool: bin/md_to_docx.py (repo root) via the wrapper scripts:

cd projects/components/compute-eng-ip/hive-serv/docs
./generate_has_pdf.sh            # HIVE_HAS_v0.2.docx / .pdf
./generate_mas_pdf.sh            # HIVE_MAS_v0.2.docx / .pdf
# Optional revision override:
./generate_has_pdf.sh --rev 0.2

The scripts check dependencies (python-docx, PyYAML, pandoc, LibreOffice), then invoke md_to_docx.py with the book’s index, styles yaml, title page, ToC, and page breaks. Built books are checked in alongside their sources.

Requirements: Python 3.6+, pandoc, LibreOffice (PDF export), and the repo’s bin/md_to_docx.py.

## 16.14	14. References

### 16.14.1	14.1 Internal Documentation

- HIVE HAS: docs/hive_has/ ← Primary external-contract reference

- HIVE MAS: docs/hive_mas/ ← Primary implementation-intent reference

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 16.14.2	14.2 Related Subsystems

- RAPIDS: projects/components/dma-ip/rapids/ - DMA engine controlled by HIVE-C

- Delta Network: projects/components/fabric-gen-ip/delta/ - AXIS fabric generator (the HIVE fabric; owner: same fabric as the archived NoC draft)

- AMBA/Ocelus: rtl/amba/ocelus/ - Monitor infrastructure for observability

- Common: rtl/common/ - Building blocks

### 16.14.3	14.3 External References

- VexRiscv: https://github.com/SpinalHDL/VexRiscv

- SERV: https://github.com/olofk/serv

- RISC-V ISA: https://riscv.org/technical/specifications/

- AXI4-Stream: ARM IHI 0051A

- HIVE Reference Library (docs/references.md) — annotated bibliography (NoC/interconnect, network processors, RISC-V, SoC integration, modeling, verification, FPGA practice), compiled from the owner calibre library + spec-cited papers 2026-10-13

Document Version: 0.2 Last Updated: 2026-10-12 Review Cycle: Monthly during active development Owner: RTL Design Sherpa Project

## 16.15	Navigation

- ← Back to Root: /PRD.md

- Hardware Architecture Specification: docs/hive_has/hive_has_index.md

- Micro-Architecture Specification: docs/hive_mas/hive_mas_index.md

- Quick Start: README.md (to be created)

- AI Guidance: CLAUDE.md

- Tasks: none tracked (vault area deleted 2026-09-27, unstarted)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Claude Code Guide: HIVE Component

Version: 0.2 Last Updated: 2026-10-12 Purpose: AI-specific guidance for working with HIVE subsystem

## 17.1	Quick Context

What: HIVE (Hierarchical Intelligent Vector Environment) - Distributed control and monitoring subsystem for RAPIDS/Delta Network Status: HAS/MAS v0.2 published 2026-10-13; Phase 1 RTL in progress (descriptor_gen, axis_cda_master, ctrl_network CN-01..03 verified at gate/func; remaining blocks + the CN-04 rotation issue tracked in vault hive-serv TASK-000) Your Role: Help users understand hierarchical RISC-V architecture, distributed monitoring, and network reconfiguration

Primary Specifications: - HAS (projects/components/compute-eng-ip/hive-serv/docs/hive_has/) — external contract - MAS (projects/components/compute-eng-ip/hive-serv/docs/hive_mas/) — block-level implementation intent

## 17.2	Critical Rules for This Component

### 17.2.1	Rule #2: Always Reference the Specifications

This subsystem’s technical documentation lives in: - projects/components/compute-eng-ip/hive-serv/docs/hive_has/ — architecture, interfaces, packet formats, programming model - projects/components/compute-eng-ip/hive-serv/docs/hive_mas/ — module trees, clocking/reset, cycle-level behavior, verification hooks

Before answering technical questions:

ls projects/components/compute-eng-ip/hive-serv/docs/hive_has/
ls projects/components/compute-eng-ip/hive-serv/docs/hive_mas/
cat projects/components/compute-eng-ip/hive-serv/docs/hive_has/hive_has_index.md

Your answer should: 1. Provide direct answer/code 2. Then link to the spec: “See projects/components/compute-eng-ip/hive-serv/docs/hive_has/{file}.md (or hive_mas/) for the full specification”

### 17.2.2	Rule #3: HIVE System Context

IMPORTANT: HIVE is part of the larger RAPIDS/Delta/HIVE ecosystem:

- RAPIDS - DMA engine with inband descriptor injection

  - Virtual Tile 16 in Delta Network

  - Controlled by HIVE-C via CDA packets

- Delta Network - 4×4 mesh Network-on-Chip (projects/components/fabric-gen-ip/delta/)

  - 16 physical compute tiles (0-15)

  - 2 virtual tiles (16=RAPIDS, 17=HIVE-C)

  - Four packet types (PKT_DATA, CDA, PKT_CONFIG, PKT_STATUS)

- HIVE - Distributed control and monitoring

  - HIVE-C master controller (VexRiscv) - Virtual Tile 17

  - 16 SERV monitors (one per physical tile)

  - Control network for HIVE-C ↔ SERV communication

  - Configuration manager for network reconfiguration

When user mentions HIVE: Clarify whether they’re asking about the entire ecosystem or just the HIVE control subsystem.

## 17.3	Architecture Quick Reference

### 17.3.1	Block Organization (HIVE System)

HIVE System Architecture
├── Block 1: HIVE-C Master Controller (VexRiscv)
│   ├── VexRiscv "Small" RV32IM core
│   ├── 32KB instruction memory
│   ├── 32KB data memory
│   ├── Descriptor generation (CDA packets)
│   ├── Control network master
│   └── AXIS master (to Delta Network)
│
├── Block 2: SERV Monitors (16 instances)
│   ├── SERV bit-serial RISC-V (RV32I)
│   ├── 2KB instruction + 2KB data memory each
│   ├── Traffic counters (pkt_rx/tx_count)
│   ├── Buffer occupancy monitors
│   ├── Congestion detection logic
│   └── AXIS injector (PKT_CONFIG, PKT_STATUS)
│
├── Block 3: Control Network
│   ├── Star topology (HIVE-C ↔ 16 SERVs)
│   ├── Round-robin arbiter (SERV → HIVE-C)
│   ├── Command delivery (HIVE-C → SERV)
│   └── Status aggregation (SERV → HIVE-C)
│
└── Block 4: Configuration Manager
    ├── 4 virtual configuration contexts
    ├── Context 0: Systolic Mode
    ├── Context 1: Packet-Switched Mesh (XY routing)
    ├── Context 2: Tree Reduction
    ├── Context 3: Custom/Debug
    └── Atomic topology switching protocol

See: - docs/hive_has/ch03_architecture/01_subsystem_architecture.md (block specs) - docs/hive_mas/ch02_blocks/ (module trees)

## 17.4	Common User Questions and Responses

### 17.4.1	Q: “What’s the difference between HIVE-C and SERV monitors?”

A: Hierarchical control architecture:

| Aspect | HIVE-C Controller | SERV Monitors (16×) |
| --- | --- | --- |
| Core | VexRiscv RV32IM | SERV RV32I (bit-serial) |
| Resources | 1,400 LUTs, 8 BRAM | 125 LUTs, 1 BRAM each |
| Role | Global coordination | Per-tile monitoring |
| Responsibilities | Descriptor scheduling, network reconfiguration, performance aggregation | Traffic monitoring, congestion detection, local packet injection |
| Network Position | Virtual Tile 17 | Co-located with physical tiles 0-15 |

Key Point: HIVE-C is the master controller, SERV monitors are lightweight per-tile agents.

See: - docs/hive_has/ch03_architecture/01_subsystem_architecture.md - docs/hive_mas/ch02_blocks/02_hive_c_controller.md and 03_serv_monitor.md

### 17.4.2	Q: “How does HIVE-C send descriptors to RAPIDS?”

A: Inband CDA packet injection via Delta Network:

1. HIVE-C generates 256-bit CDA descriptor
   ↓
2. AXIS master transmits to Delta Network
   - TUSER = 2'b01 (CDA packet type)
   - TDEST = 16 (RAPIDS virtual tile)
   ↓
3. Delta Network routes to RAPIDS (virtual tile 16)
   ↓
4. RAPIDS receives CDA packet
   - Descriptor engine processes
   - Scheduler activates appropriate data path

Advantages over memory-mapped descriptors: - Lower latency (~10-20 cycles vs. 100+) - No memory bus contention - Inband delivery through existing network

See: - docs/hive_has/ch04_interfaces/02_packet_formats.md (descriptor format) - docs/hive_mas/ch04_behavior/01_descriptor_injection_cycle.md (cycle-by-cycle)

### 17.4.3	Q: “How does network reconfiguration work?”

A: Atomic context switching via Configuration Manager:

Switching Sequence:
1. HIVE-C issues CONFIG_PREPARE command
   - Target tiles flush in-flight packets
   - Routers enter quiescent state (5-10 cycles)

2. HIVE-C broadcasts SET_ROUTING_MODE (PKT_CONFIG packet)
   - TUSER = 2'b10 (PKT_CONFIG type)
   - All tiles simultaneously update mode register
   - Context switch occurs in single cycle

3. HIVE-C issues CONFIG_ACTIVATE command
   - Tiles resume operation with new routing
   - Total latency: 10-20 cycles (deterministic)

Virtual Configuration Contexts: - Context 0: Systolic Mode (nearest-neighbor) - Context 1: Packet-Switched Mesh (XY routing) - Context 2: Tree Reduction (hierarchical aggregation) - Context 3: Custom/Debug (user-programmable)

See: - docs/hive_has/ch05_programming_model/04_network_reconfiguration.md (workflow) - docs/hive_mas/ch04_behavior/03_context_switch_sequence.md (FSM/cycles)

### 17.4.4	Q: “What do SERV monitors track?”

A: Per-tile traffic and congestion metrics:

Counters (32-bit, wrapping): - pkt_rx_count[4] - Received packets per direction (N/S/E/W) - pkt_tx_count[4] - Transmitted packets per direction - pkt_local_inject - Packets injected by local compute element - pkt_local_consume - Packets consumed by local compute element - buffer_overflow_count - FIFO overflow events - stall_cycles - Cycles with valid packet waiting for credits

Real-Time Metrics: - buffer_occupancy[4] - Current FIFO fill levels (8-bit per direction) - congestion_flag - Boolean indicating occupancy > threshold - error_flags - Parity errors, protocol violations

Reporting: - Periodic status to HIVE-C (every 1000 cycles, configurable) - Immediate alerts on congestion/errors via PKT_STATUS packets

See: - docs/hive_has/ch03_architecture/01_subsystem_architecture.md (Block 2) - docs/hive_mas/ch02_blocks/03_serv_monitor.md (counter/tap design)

## 17.5	Key Documentation Links

### 17.5.1	Primary Technical Specifications

- docs/hive_has/hive_has_index.md - Hardware Architecture Specification index

- docs/hive_mas/hive_mas_index.md - Micro-Architecture Specification index

- docs/hive_has/ch04_interfaces/ - Interfaces, packet formats, register maps

- docs/hive_mas/ch04_behavior/ - Cycle-level behavior of key operations

### 17.5.2	Related Component Specifications

- projects/components/dma-ip/rapids/docs/rapids_beats_has/ and rapids_beats_mas/ - RAPIDS DMA specification (Beats architecture)

- projects/components/fabric-gen-ip/delta/docs/delta_has/ and delta_mas/ - Delta Network specification

## 17.6	Documentation Generation

### 17.6.1	Generating PDF/DOCX from the Specifications

Tool: bin/md_to_docx.py (repo root) via the wrapper scripts:

cd projects/components/compute-eng-ip/hive-serv/docs
./generate_has_pdf.sh            # HIVE_HAS_v0.2.docx / .pdf
./generate_mas_pdf.sh            # HIVE_MAS_v0.2.docx / .pdf

The scripts check dependencies (python-docx, PyYAML, pandoc, LibreOffice), then invoke md_to_docx.py with the book’s index, styles yaml, title page, ToC, and page breaks. Built books are checked in alongside their sources.

Requirements: Python 3.6+, pandoc, LibreOffice (PDF export), and the repo’s bin/md_to_docx.py.

Debug mode (merged markdown inspection):

python bin/md_to_docx.py \
    projects/components/compute-eng-ip/hive-serv/docs/hive_has/hive_has_index.md \
    -o output.docx --debug-md

## 17.7	Remember

- Link to the specs - docs/hive_has/ (architecture) and docs/hive_mas/ (implementation intent)

- Hierarchical architecture - HIVE-C (master) + 16 SERV monitors (agents)

- Virtual tiles - RAPIDS=16, HIVE-C=17 in Delta Network topology

- Four packet types - PKT_DATA, CDA, PKT_CONFIG, PKT_STATUS (TUSER[1:0])

- Configuration contexts - 4 virtual routing modes with atomic switching

- Control network - Separate star topology for HIVE-C ↔ SERV communication

- Monitoring - Per-tile traffic stats, congestion detection, error reporting

- Pre-RTL - everything is specification; the MAS marks open items explicitly

Version: 0.2 Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	RAPIDS Beats Architecture HAS

Version: 0.14 Date: 2026-10-20 Purpose: Hardware Architecture Specification for RAPIDS Phase 1 “Beats” Architecture Classification: External Interface Specification

## 18.1	Document Organization

Note: All chapters linked below for automated document generation.

### 18.1.1	Front Matter

- Document Information

- Revision History

- Acronyms and Definitions

### 18.1.2	Chapter 1: Overview

- Product Overview

- Key Features

- System Context

### 18.1.3	Chapter 2: Architecture

- Block Diagram

- Data Flow

- Channel Architecture

### 18.1.4	Chapter 3: Interfaces

- Interface Summary

- AXI4 Master Interface

- AXIS Interface

- APB Configuration Interface

- Monitor Bus Interface

- CXS Network Interface

### 18.1.5	Chapter 4: Use Cases

- Network-to-Memory Transfer

- Memory-to-Network Transfer

- Descriptor Chaining

- Multi-Channel Operation

### 18.1.6	Chapter 5: Programming Model

- Descriptor Format

- Register Map

- Initialization Sequence

- Error Handling

### 18.1.7	Chapter 6: Performance

- Throughput Targets

- Latency Characteristics

- Resource Estimates

## 18.2	Quick Reference

### 18.2.1	RAPIDS Beats at a Glance

RAPIDS Beats Feature Summary

| Feature | Specification |
| --- | --- |
| Architecture | Network-to-Memory Accelerator |
| Channels | 8 independent DMA channels |
| Data Width | 512-bit (configurable) |
| Address Width | 64-bit |
| Descriptor Size | 256-bit |
| AXI Protocol | AXI4 (Read/Write Masters) |
| Network Protocol | AXI-Stream (Sink/Source) |
| Monitoring | 64-bit MonBus packets |

### 18.2.2	Interface Summary

External Interface Summary

| Interface | Type | Direction | Width | Purpose |
| --- | --- | --- | --- | --- |
| Descriptor AXI | AXI4 Master | Bidirectional | 256-bit | Descriptor fetch |
| Sink AXI | AXI4 Master | Write | 512-bit | Memory write |
| Source AXI | AXI4 Master | Read | 512-bit | Memory read |
| Sink AXIS | AXIS Slave | Input | 512-bit | Network ingress |
| Source AXIS | AXIS Master | Output | 512-bit | Network egress |
| MonBus | Custom | Output | 64-bit | Monitoring |

## 18.3	Related Documentation

- RAPIDS Beats MAS - Module Architecture Specification

- PRD.md - Product requirements

- ADDRESS_INCREMENT_PATTERNS.md - DMA addressing patterns

## 18.4	Specification Conventions

### 18.4.1	Signal Naming

- Clock: aclk (AXI clock domain)

- Reset: aresetn (active-low, async assert, sync deassert)

- AXI Signals: Standard ARM AMBA naming

- AXIS Signals: Standard AXI-Stream naming

### 18.4.2	Diagram Conventions

- Mermaid: Block diagrams and flowcharts

- WaveDrom: Timing diagrams

- Tables: Interface specifications and register maps

Last Updated: 2026-07-02 Maintained By: RAPIDS Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Delta Hardware Architecture Specification Index

## 19.1	Overview

Version: 1.2 Date: 2026-10-20 Purpose: High-level hardware architecture specification for the Delta AXI4-Stream crossbar generator, including the planned mesh network mode

## 19.2	Related Modules

- Delta MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- QUICK_START.md - Hands-on generator guide

## 19.3	Navigation

Note: Every chapter below is one source file; the document build assembles the spec from these links.

### 19.3.1	Front Matter

- Document Information

### 19.3.2	Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### 19.3.3	Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### 19.3.4	Chapter 3: Architecture

- Block Diagram

- Data Flow

- Protocol Support

### 19.3.5	Chapter 4: Interfaces

- AXI-Stream Base Interface

- AXI-Stream Rich Interface (TSTRB + AMBA5)

- Clock and Reset

### 19.3.6	Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resources

### 19.3.7	Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

- Synthesis and Implementation

### 19.3.8	Chapter 7: Mesh Network

- Hierarchy

- Cell and Flit

- Routing and Configuration

- Flow Control and Deadlock

- Quality of Service

- Multicast

- Systolic Support and Fabric Modes

- Observability and DFT

- Performance and Verification

- Deferred Items

- Resilient Routing: Open Hazards

Last Updated: 2026-10-14 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	RISC-V Falcon Suite

Five independent, clean-slate RISC-V cores whose primary product is education, in the same spirit as the pumice/scoria/andesite memory-controller line. Each rung is a complete, verified, documented design small enough to hold in your head; the rungs ascend in microarchitectural complexity so the diff between neighbors is the lesson. Rungs do not share RTL — each core is the cleanest possible example of its level, not the cheapest to produce. Educational benefit over IP; no rung chases performance or benchmarks.

Design spec (source of truth for scope, ISA, verification, packaging): docs/superpowers/specs/2026-10-06-riscv-falcon-suite-design.md.

## 20.1	The ladder

Each rung carries a raptor-codename, real falcons first (species in strict size order, tracking complexity). The falcon run ends at gyrfalcon — the largest living species — so the top rung steps deliberately to mythology: garuda, the giant divine raptor, king of birds, one past the largest falcon. The codename is the RTL identifier prefix and the module/package prefix; the DIRECTORY is the compound <codename>-<isa> form — kestrel-rv32i, not kestrel. The bare codename is how the core is referred to in prose (“kestrel sees the whole machine in one cycle”).

| Directory | Codename | ISA | Microarchitecture | Status |
| --- | --- | --- | --- | --- |
| kestrel-rv32i/ | kestrel | RV32I | single-cycle | RTL complete; rv32ui battery 42/42 with spike lockstep; riscv-formal 42/42; doc book v0.1 published; board loader sim-verified |
| merlin-rv32i/ | merlin | RV32I | 5-stage in-order pipeline | Planned; structure only |
| peregrine-rv32im/ | peregrine | RV32IM | advanced in-order: predictor, traps, L1 caches, AXI master | Planned; structure only |
| gyrfalcon-rv32ima/ | gyrfalcon | RV32IM baseline; gyrfalcon-A flavor RV32IMA | out-of-order capstone: rename, ROB, reservation stations; two flavors — baseline and gyrfalcon-A (A_EXT=1: LR/SC + AMO atomics study) | Planned; structure only |
| garuda-rv32imaf/ | garuda | RV32IMAF | gyrfalcon’s OoO plus an FPU: FP register file + rename, fcsr precise flags, non-pipelined long-latency units | Planned; structure only (penciled in; beyond the 2026-10-06 design spec, which pins four rungs) |

### 20.1.1	Moved: hive-serv

hive-serv — the SERV-based control/monitoring system (1 VexRiscv control plane + 16 SERV monitor cores) — left this area 2026-10-13 for compute-eng-ip/hive-serv: it uses RISC-V cores rather than teaching one, and it is the control half of the compute family (its other half is the cell CGRA), not a falcon-ladder rung. This directory is the ladder again, nothing more.

### 20.1.2	Tetrad engine: gatling-rv32itetrad

gatling-rv32itetrad/ is not a ladder rung either — it is the compute family’s tetrad engine (owner PRD 2026-10-17): a barrel RV32I core with the Xcell extension (CFG/POP/PUSH/ECTX, custom-0), one engine per tetrad with four contexts — the four cells — sharing one F/D/X/W datapath. It replaces the cell PRD’s ALU + execution-FSM block (“the program is the config”) and answers the kestrel-for-the-compute-FSM question in kestrel’s own lineage: same RV32I contract and lockstep flow, inverted pipeline lesson — N contexts hide one shared datapath’s latency instead of stages hiding latency for one stream. Open decisions and its interface to the bf16 alu-infer block are tracked in vault TASK-001.

## 20.2	Goals per rung

kestrel — see everything. Single-cycle RV32I: PC, register file, ALU, immediate generator, control as an explicit decode truth table, a plainly documented combinational-read memory contract. The hovering falcon: the whole machine is visible in one cycle — including its cost, since the critical path is the machine, which is exactly the motivation for merlin. Delivers: the ISA (R/I/S/B/U/J formats, the full base integer set), the datapath, honest control logic.

merlin — flow it. The classic IF/ID/EX/MEM/WB pipeline on the deliberately identical RV32I ISA, so students diff behavior, not ISA. Forwarding from EX/MEM and MEM/WB with explicit priority, load-use interlock stall, branches resolved in EX predict-not-taken with flush. Delivers: pipeline registers, RAW hazards, forwarding priority, the load-use bubble, control-hazard flushes, why branch resolution stage matters.

peregrine — speed it up. Advanced in-order RV32IM: iterative multicycle multiply/divide (the structural-hazard lesson), BTB + 2-bit bimodal prediction with mispredict recovery, machine-mode CSRs with precise exceptions and CLINT-shaped interrupts, split blocking write-back L1 caches behind an AXI4 master on the repo fabric with MonBus visibility. Delivers: speculation and recovery, precise exceptions, the memory hierarchy, bus protocol — and ties the whole repo together (memory controllers, cache IP line, AMBA fabric).

gyrfalcon — hide everything. Out-of-order capstone in two flavors (added 2026-10-11): the baseline — peregrine’s RV32IM exactly — and gyrfalcon-A (A_EXT=1): LR/SC reservations, store-conditional failure, and AMOs, the atomicity study — so every peregrine battery still carries in both. Dual-issue dispatch, explicit rename map + free list, 32-entry ROB with in-order commit, three reservation stations, wakeup/select over common data buses, speculative dispatch with checkpoint-restore rename rollback. Delivers: register renaming and WAR/WAW elimination, the ROB as an undo log, out-of-order completion with in-order retirement — the honest, complete OOO story — and the atomicity study: reservations, SC failure, AMOs, acquire/release as memory-model operations.

garuda — float it. The RV32IMAF delta on gyrfalcon’s OoO: a 32-entry FP register file with its own rename, an fcsr whose five accrued exception flags (NV/DZ/OF/UF/NX) stay precise-exception-clean through the ROB, and the repo’s ieee754 fp32 arithmetic behind the reservation stations — multi-cycle FMUL/FADD and the non-pipelined Goldschmidt FDIV / Newton-rsqrt FSQRT (9–11 cycle class) as the long-latency-unit lesson. Rounding hard-wired RNE; any other requested mode raises NV, keeping the CSR story honest without a dynamic rounding-mode datapath. Delivers: long-latency execution scheduling, FP architectural state, and the F-extension ISA delta (F regs, fcsr, FMV/FCVT/FSGNJ/FCLASS) — consuming the math library’s math_ieee754_2008_fp32_* blocks end-to-end.

## 20.3	Packaging (per rung, the MC-line treatment)

- Doc book — Simplified RV32I: kestrel and successors, Memory Notes branding, built through bin/md_to_docx.py to DOCX/PDF, following the Simplified DFI books’ precedent. Each book lives in its rung’s docs/; the kestrel edition is published at kestrel-rv32i/docs/simplified_rv32i/.

- Formal — riscv-formal-style ISA compliance where it pays: full on kestrel, retire-interface on merlin, targeted properties (exception precision, recovery, in-order commit, deadlock freedom) on peregrine/gyrfalcon.

- Golden model — spike ISS lockstep in the rds-dv cocotb framework; the classic rv32ui/rv32um riscv-tests batteries as the compliance gate.

- Drill app — bin/apps/core_drills (ddr_drills architecture): pipeline stage-viewer with forwarding arrows, hazard quiz, branch-predictor drill, and a rename/ROB viewer. App design gets its own doc once merlin’s RTL is fixed.

## 20.4	References and layout

Common primary sources (the RISC-V ISA manuals) live in references/ and are cited by chapter, not page.

Per-rung skeleton follows the dma-ip/stream component layout convention: rtl/, docs/, dv/ (cocotb + lockstep testbenches). Shared suite doctrine lands in docs/ as it is written. Per-rung PRD.md/CLAUDE.md arrive with each rung’s implementation plan.