RTL Design Sherpa

Delta Hardware Architecture Specification 1.2

October 20, 2026

Table of Contents

1 Delta Has Index	12

2 Document Information	13

2.1 Delta Hardware Architecture Specification	13

2.2 Revision History	14

2.3 Document Purpose	15

2.4 Intended Audience	16

2.5 Related Documents	17

3 Purpose and Scope	18

3.1 What is Delta?	18

3.2 The Problem Delta Solves	19

3.3 Scope	20

3.4 Out of Scope	21

3.5 Non-Goal: Memory-Mapped Protocol Conversion	22

4 Document Conventions	23

4.1 Naming Conventions	23

4.2 Signal Direction Conventions	24

4.3 Numbering Conventions	25

4.4 Typographic Conventions	26

4.5 Versioning Conventions	27

5 Definitions and Acronyms	28

5.1 Acronyms	28

5.2 Key Terms	29

6 Use Cases	30

6.1 Primary: RISC Cores + DSP Array	30

6.2 Secondary: Sensor/Peripheral Fabric	31

6.3 Tertiary: AMBA 5-Aware Streaming Interconnect	32

6.4 Educational Demonstration	33

6.5 Mesh-Mode: Monitored Manycore Fabric	34

7 Key Features	35

7.1 Flat M×N Crossbar	35

7.2 TDEST-Based Routing	36

7.3 Packet-Atomic Round-Robin Arbitration	37

7.4 Combinational Data Mux with Registered Grant	38

7.5 Optional Richness (Default Off)	39

7.6 Optional Packet Counters	40

7.7 Formal Assertions	41

7.8 Code Generation	42

7.9 Mesh-Mode Features	43

8 System Context	45

8.1 Where Delta Fits	45

8.1.1 Figure 2.1: Delta System Context	45

8.2 Upstream Requirements	46

8.3 Downstream Requirements	47

8.4 Clocking and Reset	48

8.5 Relationship to Other Components	49

8.6 Mesh-Mode Context	50

9 Block Diagram	51

9.1 Delta Architecture Overview	51

9.1.1 Figure 3.1: Delta High-Level Block Diagram	51

9.2 Functional Blocks	52

9.2.1 TDEST Decode	52

9.2.2 Per-Slave Round-Robin Arbiter	52

9.2.3 Crossbar Mux	52

9.2.4 TREADY Steering	52

9.2.5 TWAKEUP Steering (AMBA5, Optional)	52

9.2.6 Interface Checks (AMBA5, Optional)	52

9.2.7 Packet Counters (Optional)	52

9.3 Scalability	53

9.3.1 Resource Scaling	53

10 Data Flow	54

10.1 Request Flow	54

10.2 Payload Flow	55

10.3 Backpressure Flow	56

10.4 Wakeup Flow (AMBA5)	57

10.5 Packet Boundaries	58

11 Protocol Support	59

11.1 AXI4-Stream Base Flavor	59

11.2 Optional TSTRB Byte Qualifiers	60

11.3 Optional AMBA 5 Sidebands	61

11.4 What Delta Does Not Support	62

11.5 Protocol Compliance Summary	63

12 AXI-Stream Base Interface	64

12.1 Master Interface (S_AXIS)	64

12.2 Slave Interface (M_AXIS)	65

12.3 Signal Semantics	66

13 AXI-Stream Rich Interface (TSTRB + AMBA5)	67

13.1 Rich Master Interface (S_AXIS)	67

13.2 Rich Slave Interface (M_AXIS)	69

13.3 TSTRB Semantics	70

13.4 TWAKEUP Semantics	71

13.5 Check-Signal Semantics (IHI 0051B Chapter 5)	72

13.6 Parameter Implications	73

14 Clock and Reset	74

14.1 Clock	74

14.2 Reset	75

14.3 Clocking Rules	76

14.4 Reset Recommendations	77

15 Throughput Characteristics	78

15.1 Aggregate Throughput	78

15.2 Per-Slave Throughput	79

15.3 Head-of-Line Blocking	80

15.4 Throughput Targets	81

15.5 Factors That Reduce Throughput	82

16 Latency Analysis	83

16.1 Cycle Breakdown	83

16.2 Arbitration Delay	84

16.3 Backpressure Latency	85

16.4 Latency Targets	86

16.5 Frequency Target	87

17 Resources	88

17.1 Estimates from the Performance Model	88

17.2 Scaling Laws	89

17.3 Area Optimization Knobs	90

18 System Requirements	91

18.1 Toolchain Requirements	91

18.2 Runtime Requirements	92

18.3 Interface Assumptions	93

18.4 Integration Checklist	94

19 Parameter Configuration	95

19.1 Generated Module Parameters	95

19.2 Generator CLI Options	96

19.3 Example Configurations	97

19.3.1 Base flavor 4×16 @ 64-bit	97

19.3.2 Rich flavor 2×4 @ 32-bit with TSTRB and AMBA5	97

19.3.3 Base flavor without counters	97

20 Verification Strategy	98

20.1 Current Status	98

20.2 Generator Verification	99

20.3 Lint Verification	100

20.4 Planned RTL Verification	101

20.5 Formal Properties	102

21 Synthesis and Implementation	103

21.1 Synthesis Flow	103

21.2 Timing Considerations	104

21.3 Floorplanning Suggestions	105

21.4 Implementation Checklist	106

22 Mesh Hierarchy	107

22.1 Placement Levels	108

22.1.1 Figure 7.1: Delta Mesh Placement Hierarchy	108

22.2 Cell	109

22.3 Tetrad	110

22.4 Tile	111

22.5 Fabric	112

22.6 Why This Grouping	113

22.7 Observation Modules (2026-10-15 amendment)	114

23 Cell and Flit	115

23.1 Cell Ports	115

23.2 Clock and Reset	116

23.3 Flit Format	117

23.3.1 Head-Flit Header Fields	117

24 Routing and Configuration	119

24.1 Routing Tables	119

24.2 Reset Default	120

24.3 Table Constraints	121

24.4 In-Band Configuration	122

24.5 Broadcast Mode-Switch	123

25 Flow Control and Deadlock	124

25.1 Credit-Based Flow Control	124

25.2 Two Virtual Channels	125

25.3 VC Transition Rules	126

25.4 Torus Wrap and the Dateline Rule	127

25.5 Livelock	128

26 Quality of Service	129

26.1 Three Traffic Classes	129

26.2 Strict Priority With Aging	130

26.3 Non-Interference	131

26.4 Per-Class Routes	132

26.5 Within-Class Fairness	133

27 Multicast	134

27.1 VC0 Only	134

27.2 BRCP-Conformed Paths	135

27.3 Source-Side Replication	136

27.4 Multiple Delivery VCs	137

27.5 No Responses	138

28 Systolic Support and Fabric Modes	139

28.1 Torus Wrap Links	139

28.2 Route Lookahead	140

28.3 Source-Route Mode	141

28.4 Speculative Allocation	142

28.5 Standard Fabric Modes	143

28.5.1 Standard Fabric Modes	143

29 Observability and DFT	145

29.1 Ocelus Counter Suite	145

29.2 DFT Test Mode	146

30 Performance and Verification	147

30.1 Performance Model	147

30.2 Verification Plan	148

31 Deferred Items	149

32 Resilient Routing: Open Hazards	151

32.1 Design Fork: Which Leg Adapts	152

32.2 Multi-Path Tables and Failover	153

32.3 Deadlock	155

32.4 Ordering	156

List of Figures

Figure 2.1: Delta System Context	45

Figure 3.1: Delta High-Level Block Diagram	51

Figure 7.1: Delta Mesh Placement Hierarchy	108

List of Tables

Table 1: Document Properties	13

Table 2: Revision History	14

Table 3: Naming Conventions	23

Table 4: Acronyms and Definitions	28

Table 5: Delta Component Context	49

Table 6: Topology Parameter Scaling	53

Table 7: Protocol Support Matrix	63

Table 8: Base Master Interface Signals	64

Table 9: Base Slave Interface Signals	65

Table 10: Rich Master Interface Additional Signals	67

Table 11: Rich Slave Interface Additional Signals	69

Table 12: Rich-Flavor Parameter-to-Port Mapping	73

Table 13: Clock Signal	74

Table 14: Reset Signal	75

Table 15: Throughput Targets	81

Table 16: Latency Cycle Breakdown	83

Table 17: Latency Targets	86

Table 18: Resource Estimates	88

Table 19: Area Optimization Options	90

Table 20: Generated Module Parameters	95

Table 21: Generator Command-Line Options	96

Table 22: Head-flit header fields carried in TUSER on the head beat.	117

Table 23: Standard fabric modes selected by broadcast mode-switch.	143

Table 24: Ocelus counter bank indices.	145

Table 25: Adaptive-routing options	152

Table 26: Failover hazards	153

Table 27: Ordering contract	156

# 1	Delta Has Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Delta Hardware Architecture Specification

Document Properties

| Property | Value |
| --- | --- |
| Document Title | Delta Hardware Architecture Specification |
| Version | 1.2 |
| Date | 2026-10-20 |
| Status | Released |
| Classification | Open Source - MIT License |

## 2.2	Revision History

Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-10-12 | RTL Design Sherpa | Initial HAS/MAS split; documents the 2026-10-12 richness options (TSTRB and AMBA5 TWAKEUP/TPARITY) and flat-only topology. |
| 1.1 | 2026-10-12 | RTL Design Sherpa | AMBA 5 interface parity per IHI 0051B ch. 5 (issue #102): TPARITY/PW_WIDTH/--parity-width replaced by the odd-parity check signals TDATACHK … TWAKEUPCHK (spec widths, checked at every port, sticky *_parity_error), matching rtl/amba/axis5. First dv/ suite. |
| 1.2 | 2026-10-20 | RTL Design Sherpa | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |

## 2.3	Document Purpose

This Hardware Architecture Specification (HAS) is the high-level view of the Delta AXI4-Stream crossbar generator — the map you read before you integrate, not the schematic. It covers:

- System-level architecture and data flow

- AXI4-Stream protocol support (base and rich flavors)

- Interface definitions and requirements

- Performance characteristics

- Integration guidelines

For micro-architecture, FSM designs, and signal-level detail, see the companion Delta Micro-Architecture Specification (MAS).

## 2.4	Intended Audience

- System architects defining streaming interconnect topology

- Hardware engineers integrating Delta into SoC designs

- Verification engineers planning test strategies

- Technical managers evaluating Delta capabilities

## 2.5	Related Documents

- Delta MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI assistant integration guide

- QUICK_START.md - Hands-on generator guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	What is Delta?

Delta is a Python-based AXI4-Stream crossbar generator. You provide the number of masters, the number of slaves, the data width, and a few optional sideband knobs; it hands you back parameterized, synthesizable SystemVerilog. The name follows the water theme used elsewhere in RTL Design Sherpa (like RAPIDS): a river delta is where one channel splits into many branches, analogous to crossbar routing.

## 3.2	The Problem Delta Solves

A hand-written AXI4-Stream crossbar is error-prone. Building one means writing per-slave arbitration, packet-atomic grant logic, data multiplexing for TDATA/TVALID/TLAST/TDEST/TID/TUSER, and TREADY backpressure steering — then doing it again every time the master or slave count changes.

Delta automates all of it:

- Define the crossbar in a single command line

- Run bin/delta_generator.py

- Get production-ready SystemVerilog RTL

## 3.3	Scope

This specification covers:

- High-level architecture and block organization

- AXI4-Stream protocol support, including the optional base + rich signal sets

- Interface requirements and signal conventions

- Performance characteristics and resource estimates

- Integration guidelines and verification status

## 3.4	Out of Scope

These live in the companion MAS or are deliberate non-goals:

- Detailed FSM state diagrams and transitions (MAS)

- Signal-level timing diagrams (MAS)

- Internal pipeline structures (MAS)

- AXI4/AXI4-Lite/APB protocol conversion (deliberate non-goal; see Bridge or APB crossbar siblings)

- Dynamic reconfiguration or advanced routing algorithms

## 3.5	Non-Goal: Memory-Mapped Protocol Conversion

Delta is AXI4-Stream only. It does not convert to or from AXI4, AXI4-Lite, or APB. If you need address-mapped fabrics, use the sibling bridge or lattice-apbx-xbar components.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Naming Conventions

Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| s_axis_* | Signals on the master side of the crossbar (inputs) | s_axis_tdata[m] |
| m_axis_* | Signals on the slave side of the crossbar (outputs) | m_axis_tready[s] |
| [NUM_MASTERS] | Unpacked array dimension, one entry per master | s_axis_tvalid [NUM_MASTERS] |
| [NUM_SLAVES] | Unpacked array dimension, one entry per slave | m_axis_tvalid [NUM_SLAVES] |
| m | Master index | for (int m = 0; m < NUM_MASTERS; m++) |
| s | Slave index | for (int s = 0; s < NUM_SLAVES; s++) |

## 4.2	Signal Direction Conventions

All directions are stated from the crossbar’s point of view:

- s_axis_* signals are driven by upstream masters and received by Delta.

- m_axis_* signals are driven by Delta and received by downstream slaves.

- aclk and aresetn are driven by the system.

## 4.3	Numbering Conventions

- Masters are numbered 0 to NUM_MASTERS-1.

- Slaves are numbered 0 to NUM_SLAVES-1.

- TDEST is interpreted as a slave index; legal values are 0 to NUM_SLAVES-1.

## 4.4	Typographic Conventions

- code font is used for signal names, parameters, and file paths.

- Bold is used for emphasis and defined terms.

- Tables use the RTL Design Sherpa corporate document style.

## 4.5	Versioning Conventions

This document uses semantic-style major.minor revision numbers. The initial release is 1.0.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

## 5.1	Acronyms

Acronyms and Definitions

| Acronym | Definition |
| --- | --- |
| AXIS | AXI4-Stream (ARM AMBA streaming protocol) |
| AMBA5 | ARM AMBA 5 protocol extensions, including TWAKEUP and the T*CHK interface check signals |
| TDEST | Transaction destination (slave ID in AXI4-Stream) |
| TID | Transaction ID (master identifier in AXI4-Stream) |
| TUSER | User-defined sideband in AXI4-Stream |
| TSTRB | Byte strobe / byte qualifier in AXI4-Stream |
| TWAKEUP | AMBA 5 wakeup hint signal |
| T*CHK | AXI5-Stream interface check signals (TDATACHK, TVALIDCHK, …), odd parity, IHI 0051B chapter 5 |
| RR | Round-robin arbitration |
| LUT | Look-up table (FPGA logic resource) |
| FF | Flip-flop (FPGA register resource) |

## 5.2	Key Terms

Base flavor

The default generated crossbar with the seven-signal set: tdata, tvalid, tready, tlast, tdest, tid, tuser.

Rich flavor

A generated crossbar with optional additional signals enabled: tstrb and/or AMBA 5 twakeup with the t*chk check signals.

Packet atomicity

The property that once a master is granted access to a slave, it retains that grant until the current packet’s TLAST beat has been transferred. No other master may interleave beats on that slave during the packet.

Request matrix

A per-slave collection of master request bits. request_matrix[s][m] is true when master m has a valid transaction targeting slave s.

Grant matrix

A per-slave one-hot (or zero-hot) collection of master grant bits. grant_matrix[s][m] is true when master m currently owns slave s.

TDEST decode

The combinational translation of s_axis_tdest[m] into a slave index in the request matrix. Delta does not use an address map.

TWAKEUP steering

The AMBA 5 sideband path that forwards a master’s s_axis_twakeup hint to the slave selected by s_axis_tdest. It is an OR-per-slave and may assert before TVALID.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Use Cases

## 6.1	Primary: RISC Cores + DSP Array

Scenario: Four small RISC-V cores route compute tasks to sixteen DSP accelerators.

Configuration:

- 4 masters (RISC cores)

- 16 slaves (DSP arrays)

- 64-bit data width

- Flat topology

Benefits:

- Each RISC core can target any DSP (full flexibility).

- Per-slave round-robin arbitration prevents starvation.

- Two-cycle latency keeps task dispatch responsive.

- All sixteen DSPs can operate concurrently when traffic targets different slaves.

## 6.2	Secondary: Sensor/Peripheral Fabric

Scenario: Multiple sensor streams feed a smaller set of processing blocks.

Configuration:

- 2-8 masters (sensor front-ends)

- 2-4 slaves (aggregation or filter blocks)

- 32-bit data width with optional TSTRB byte qualifiers

Benefits:

- Direct TDEST routing removes the need for an address decoder.

- Packet atomicity preserves frame boundaries from each sensor.

- Optional TSTRB supports sparse byte-valid data from byte-packed sensors.

## 6.3	Tertiary: AMBA 5-Aware Streaming Interconnect

Scenario: A low-power subsystem needs wakeup hints and AXI5-Stream interface parity protection.

Configuration:

- Masters and slaves that implement AMBA 5 AXI-Stream

- --amba5 enabled

Benefits:

- TWAKEUP lets a master wake a target slave before presenting data.

- The IHI 0051B chapter 5 check signals (TDATACHK and the rest, odd parity) are carried across the fabric and checked at each crossbar port. A sticky per-port flag reports a fault.

- Generated ports port-match the rtl/amba/axis5/* interface conventions.

## 6.4	Educational Demonstration

Scenario: Teach interconnect design trade-offs and code generation.

Configuration:

- Generate flat crossbars of varying sizes into rtl_test/.

- Compare latency, throughput, and resource estimates from delta_performance_model.py.

Benefits:

- Hands-on learning with working RTL.

- Clear performance comparisons.

- Demonstrates specification-driven design and Python-based RTL generation.

## 6.5	Mesh-Mode: Monitored Manycore Fabric

Scenario: A 3-D manycore fabric of SERV cores and CGRA compute cells exchanges stream traffic while ocelus observers collect statistics and rapids DMA feeds the boundary.

Configuration:

- --topology mesh with fabric dimensions cells-x, cells-y, cells-z.

- One delta_mesh_cell at every fabric coordinate; rapids attaches at boundary cells’ local ports.

- ocelus observers tap cell ports for flit/packet/stall/latency counters.

- Cell contents (SERV, CGRA, ocelus) are out of scope — delta sees only the AXI4-Stream at the local port.

IMPLEMENTED — Mesh mode RTL is generated by delta_generator.py --topology mesh. The design described here is per the owner-approved delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md). The flat crossbar remains the implemented default.

Still deferred: CXS migration, MonBus observation aggregation in the tile IP, 1 cycle/hop lookahead (issue #120), and SRAM FIFO_IMPL (issue #118).

Benefits:

- Table-routed 3-D mesh scales without the all-to-all wiring of a flat crossbar.

- Per-cell routing tables and broadcast mode-switches let the same physical fabric run different communication patterns.

- Built-in observability gives software-visible performance data from the fabric.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Key Features

## 7.1	Flat M×N Crossbar

Delta generates a single self-contained SystemVerilog module implementing a full M×N crossbar. Every master can reach every slave; each slave has its own round-robin arbiter.

## 7.2	TDEST-Based Routing

There is no address map. The target slave is taken directly from s_axis_tdest. This makes request decode simpler than an address-based crossbar and removes configuration risk.

## 7.3	Packet-Atomic Round-Robin Arbitration

Each slave arbiter grants one master at a time and holds that grant until the current packet’s TLAST beat is accepted. This guarantees:

- No packet interleaving on a slave.

- Fair bandwidth allocation among contending masters.

- Streaming packet boundaries are preserved end-to-end.

## 7.4	Combinational Data Mux with Registered Grant

The data path from the granted master to the selected slave is combinational. The grant decision is registered on aclk, giving a predictable two-cycle latency while keeping the critical path short.

## 7.5	Optional Richness (Default Off)

Two command-line options add sidebands without changing the default output:

- --strb-width N adds TSTRB byte qualifiers (port-matches rtl/amba/axis4).

- --amba5 adds TWAKEUP and the IHI 0051B chapter 5 odd-parity check signals (TDATACHK … TWAKEUPCHK), with sticky per-port error flags. The ports match rtl/amba/axis5.

When neither option is used, the generated module is byte-identical to pre-2026-10-12 output.

## 7.6	Optional Packet Counters

Per-master and per-slave 32-bit packet counters count completed packets (TVALID && TREADY && TLAST). Disable them with --no-counters to save area.

## 7.7	Formal Assertions

When FORMAL is defined, the generated RTL includes assertions that check:

- One-hot (or zero-hot) grants per slave.

- TDEST is within range when s_axis_tvalid is asserted.

## 7.8	Code Generation

The generator is a Python script (bin/delta_generator.py) that reuses the unified Module framework. It produces lint-clean SystemVerilog and is the single source of truth for all checked-in RTL variants.

## 7.9	Mesh-Mode Features

IMPLEMENTED — Mesh mode is a generator option that emits delta_mesh_cell and a fabric top. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md). The flat TDEST crossbar remains the default and is untouched.

Still deferred: CXS migration (owner plan, parallel rapids_cxs work), MonBus observation aggregation in the tile IP (parallel session), 1 cycle/hop lookahead (issue #120), and SRAM FIFO_IMPL (issue #118).

- 3-D table-routed mesh of identical cells — one delta_mesh_cell per fabric coordinate, connected to ±X/±Y/±Z neighbors, the two diagonal-axis links (P_DIA for tetrad-scale, P_DIA2 for tile-scale), and one local port.

- Placement hierarchy — cell → tetrad (2×2×1 = 4 cells, fully-connected K4) → tile (2×2×1 tetrads = 16 cells, K4 at the tetrad level). Owner-named 2026-10-15; the datapath stays one module per cell, and the tetrad/tile wrappers exist only for MonBus observation aggregation (outside delta).

- 2 VCs with escape-channel deadlock freedom — VC0 carries the connected, acyclic escape subrelation; VC1 carries minimal-or-budgeted adaptive routes. VC1 may drain into VC0, but VC0 may never enter VC1.

- 3 QoS classes — config, latency, and bulk traffic get separate VCs and injection buffers, with strict priority, programmable aging, and a bulk minimum reservation.

- In-band config with broadcast mode-switch — per-cell RAM tables load through the data path; a broadcast mode-switch message switches all cells to one of 10 standard fabric modes atomically after a fabric-wide ack barrier.

- 10 standard fabric modes — mesh, systolic-1D, wavefront-2D, stencil-3D, broadcast, convergecast/reduce, all-to-all/transpose, pipeline+rapids-stream, tetrad all-to-all (K4 via P_DIA), and tile all-to-all (K4-of-tetrads via P_DIA2).

- Multicast — VC0-only, BRCP-conformed paths with source-side replication, multiple delivery VCs, and no responses.

- Systolic/torus package — per-dimension torus wrap (--torus=x,y,z), source-route mode (--source-route), and speculative allocation (--speculation). Route lookahead is the remaining latency step (issue #120).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	System Context

## 8.1	Where Delta Fits

Delta sits between upstream AXI4-Stream masters and downstream AXI4-Stream slaves. It does not interpret addresses, bursts, or memory attributes; it only routes streams based on TDEST.

A typical Delta deployment looks like:

### 8.1.1	Figure 2.1: Delta System Context

Delta System Context

Multiple masters and multiple slaves attach in parallel. Each master drives one index of the s_axis_* arrays; each slave receives one index of the m_axis_* arrays.

## 8.2	Upstream Requirements

- Masters must drive valid AXI4-Stream signals.

- s_axis_tdest[m] must be a valid slave index (0 to NUM_SLAVES-1) when s_axis_tvalid[m] is asserted.

- Masters must respect s_axis_tready[m] backpressure.

## 8.3	Downstream Requirements

- Slaves must accept valid AXI4-Stream signals.

- Slaves drive m_axis_tready[s] to throttle traffic.

## 8.4	Clocking and Reset

Delta uses a single clock (aclk) and an active-low asynchronous reset (aresetn). All registered state is reset; combinational decode and mux logic is not registered.

## 8.5	Relationship to Other Components

Delta Component Context

| Component | Role | When to Use |
| --- | --- | --- |
| Delta | AXI4-Stream crossbar generator | Streaming fabrics, DSP arrays, sensors |
| Bridge | Multi-protocol crossbar generator | Memory-mapped AXI4/AXI4-Lite/APB systems |
| APB Crossbar | APB-only crossbar | Low-bandwidth register-mapped peripherals |
| RAPIDS | DMA subsystem | Descriptor-based memory transfers |

Delta is intentionally narrow in scope: it does one thing (streaming crossbars) and does it without protocol conversion.

## 8.6	Mesh-Mode Context

Mesh mode extends the same bin/delta_generator.py with a --topology mesh option. Instead of a flat M×N crossbar it emits a 3-D grid of delta_mesh_cell instances with rapids DMA at the boundary and ocelus observers on the links. Cell contents (SERV, CGRA, observers) are outside delta’s scope; delta only carries the AXI4-Stream traffic between them.

IMPLEMENTED — Mesh mode RTL is generated by delta_generator.py --topology mesh; the design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md). The flat crossbar shown in Figure 2.1 remains the default topology.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Block Diagram

## 9.1	Delta Architecture Overview

### 9.1.1	Figure 3.1: Delta High-Level Block Diagram

Delta HAS Block Diagram

The diagram follows one slave s; every slave repeats it. Each master’s TVALID and TDEST are decoded into a request for s (center), the per-slave round-robin arbiter (the trapezium) grants one master and holds the grant until that packet’s TLAST, and the crossbar mux forwards the granted master’s TVALID and payload, which arrive on their own lane at the left. TREADY returns up the right lane to the granted master only. The dashed blocks are build options: --amba5 adds TWAKEUP steering and the *CHK parity checks, and the packet counters are built unless --no-counters.

## 9.2	Functional Blocks

### 9.2.1	TDEST Decode

Receives s_axis_tvalid and s_axis_tdest from every master and produces the per-slave request_matrix. A request bit is set when a master has a valid transaction whose TDEST selects that slave.

### 9.2.2	Per-Slave Round-Robin Arbiter

One arbiter per slave. When the slave is idle, it picks the next requesting master in round-robin order. Once a master is granted, the grant is held until TLAST is accepted. This block is the scheduler analog in the house style and is drawn as an inverted trapezium.

### 9.2.3	Crossbar Mux

A bank of per-slave multiplexers. Each slave mux selects the payload and control signals from the master that currently holds the grant for that slave.

### 9.2.4	TREADY Steering

Steers each slave’s m_axis_tready back to the single master that currently owns the grant for that slave. Ungranted masters see s_axis_tready == 0.

### 9.2.5	TWAKEUP Steering (AMBA5, Optional)

When --amba5 is enabled, this block ORs each master’s s_axis_twakeup to the slave indicated by s_axis_tdest. Because TWAKEUP is a hint, it does not depend on the grant and may assert before TVALID.

### 9.2.6	Interface Checks (AMBA5, Optional)

With --amba5, each master’s payload check bits (TDATACHK … TUSERCHK) are checked for odd parity while TVALID is high and then carried through the mux with the beat; TVALIDCHK, TWAKEUPCHK and TREADYCHK are generated by the side that drives the signal and checked by the other (dataint_parity). A failed check sets the port’s sticky s_parity_error / m_parity_error.

### 9.2.7	Packet Counters (Optional)

32-bit counters per master and per slave that increment on every completed packet (TVALID && TREADY && TLAST). Removed from the build with --no-counters.

## 9.3	Scalability

Topology Parameter Scaling

| Parameter | Typical Range | Impact |
| --- | --- | --- |
| NUM_MASTERS | 1-32 | Linear arbitration and mux growth |
| NUM_SLAVES | 1-256 | Linear decode and grant-matrix growth |
| DATA_WIDTH | 8-1024 | Linear data-path width |
| DEST_WIDTH | auto (clog2(NUM_SLAVES)) | Sets TDEST width |
| ID_WIDTH | auto (clog2(NUM_MASTERS)) | Sets TID width |

### 9.3.1	Resource Scaling

- Logic: O(NUM_MASTERS × NUM_SLAVES) for the grant and request matrices.

- Data path: O(NUM_SLAVES × DATA_WIDTH) for the output muxes.

- Memory: No BRAM; all state is flip-flops.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Data Flow

## 10.1	Request Flow

- A master asserts s_axis_tvalid[m] and places the target slave index on s_axis_tdest[m].

- The TDEST decode block sets request_matrix[s][m] for the selected slave.

- If the slave is not already serving another packet, the per-slave round-robin arbiter grants one requesting master.

- The grant is registered and held until the packet ends on TLAST.

## 10.2	Payload Flow

- While a grant is active, the crossbar mux selects the granted master’s payload.

- m_axis_tdata[s], m_axis_tlast[s], m_axis_tdest[s], m_axis_tid[s], and m_axis_tuser[s] are driven from the granted master.

- If TSTRB is enabled, m_axis_tstrb[s] is also forwarded.

- If --amba5 is enabled, the payload check bits (m_axis_tdatachk[s] through m_axis_tuserchk[s]) are forwarded with the beat. They were already checked at the master port, where a failure sets s_parity_error[m].

## 10.3	Backpressure Flow

- Each downstream slave drives m_axis_tready[s].

- The TREADY steering block routes the granted slave’s ready back to the owning master.

- Only the granted master sees s_axis_tready[m] == 1; all other masters are stalled.

## 10.4	Wakeup Flow (AMBA5)

- A master may assert s_axis_twakeup[m] before asserting s_axis_tvalid[m].

- The TWAKEUP steering block decodes s_axis_tdest[m] and ORs the hint to the matching slave.

- m_axis_twakeup[s] may assert with no active grant.

## 10.5	Packet Boundaries

TLAST marks the final beat of a packet. The arbiter releases the grant on the cycle in which m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s] is true. The next arbitration decision can occur on the following cycle.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Protocol Support

## 11.1	AXI4-Stream Base Flavor

The default generated crossbar implements the standard AXI4-Stream signal set:

- TDATA — data payload

- TVALID / TREADY — handshake

- TLAST — packet boundary

- TDEST — destination slave index

- TID — transaction/master ID (pass-through)

- TUSER — user sideband (pass-through)

This is the byte-identical default; no sideband options are enabled unless requested.

## 11.2	Optional TSTRB Byte Qualifiers

The --strb-width N option adds TSTRB to every master and slave interface. TSTRB is treated as a pass-through sideband: the mux forwards the granted master’s TSTRB to the selected slave. Delta does not interpret or modify strobe bits.

## 11.3	Optional AMBA 5 Sidebands

The --amba5 option adds:

- TWAKEUP: a wakeup hint, decoded from TDEST and ORed per slave.

- The AXI5-Stream interface-protection signals of IHI 0051B chapter 5 (Check_Type = Odd_Parity_Byte_All): TDATACHK, TSTRBCHK (with TSTRB), TLASTCHK, TIDCHK, TDESTCHK, TUSERCHK, TVALIDCHK, TREADYCHK and TWAKEUPCHK, all odd parity. The spec fixes the widths at one check bit per 8 bits.

- Sticky s_parity_error / m_parity_error flags per port.

The crossbar checks the payload check bits on ingress and carries them to the granted slave unchanged. It generates the control check bits for the signals it drives and checks the ones it receives. The generated ports match the rtl/amba/axis5/* modules. See Chapter 4, AXI-Stream Rich Interface, for the full table.

## 11.4	What Delta Does Not Support

Delta is intentionally AXI4-Stream only. It does not support:

- AXI4 memory-mapped transactions (AW/AR/W/B/R channels).

- AXI4-Lite register-mapped access.

- APB transfers.

- Burst lengths, address phases, or memory attributes.

- Dynamic reconfiguration of the crossbar dimensions.

For memory-mapped fabrics, use the sibling bridge or lattice-apbx-xbar components.

## 11.5	Protocol Compliance Summary

Protocol Support Matrix

| Feature | Base Flavor | Rich Flavor | Notes |
| --- | --- | --- | --- |
| TVALID/TREADY handshake | Yes | Yes | Standard AXI4-Stream |
| TLAST packet boundary | Yes | Yes | Grant release on TLAST |
| TDEST routing | Yes | Yes | Direct slave index |
| TID/TUSER pass-through | Yes | Yes | Unmodified |
| TSTRB | No | Yes | --strb-width N |
| TWAKEUP | No | Yes | --amba5 |
| T*CHK interface parity (IHI 0051B ch. 5, odd) | No | Yes | --amba5 |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI-Stream Base Interface

## 12.1	Master Interface (S_AXIS)

Each master port is indexed by m in the range 0 to NUM_MASTERS-1.

Base Master Interface Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_tdata[m] | DATA_WIDTH | Input | Data payload. |
| s_axis_tvalid[m] | 1 | Input | Valid indicator. When high, s_axis_tdest[m] selects the target slave. |
| s_axis_tready[m] | 1 | Output | Ready / backpressure. Driven from the granted slave’s m_axis_tready. |
| s_axis_tlast[m] | 1 | Input | Packet boundary. The final beat of a packet. |
| s_axis_tdest[m] | DEST_WIDTH | Input | Target slave index. Must be < NUM_SLAVES when tvalid is asserted. |
| s_axis_tid[m] | ID_WIDTH | Input | Transaction ID. Pass-through to m_axis_tid. |
| s_axis_tuser[m] | USER_WIDTH | Input | User sideband. Pass-through to m_axis_tuser. |

## 12.2	Slave Interface (M_AXIS)

Each slave port is indexed by s in the range 0 to NUM_SLAVES-1.

Base Slave Interface Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_tdata[s] | DATA_WIDTH | Output | Data payload from the granted master. |
| m_axis_tvalid[s] | 1 | Output | Valid indicator. Asserted when a grant is active and the selected master is valid. |
| m_axis_tready[s] | 1 | Input | Ready / backpressure from the downstream slave. |
| m_axis_tlast[s] | 1 | Output | Packet boundary. Forwarded from the granted master. |
| m_axis_tdest[s] | DEST_WIDTH | Output | Target slave index. Pass-through from the granted master. |
| m_axis_tid[s] | ID_WIDTH | Output | Transaction ID. Pass-through from the granted master. |
| m_axis_tuser[s] | USER_WIDTH | Output | User sideband. Pass-through from the granted master. |

## 12.3	Signal Semantics

- TVALID may be asserted without TREADY; the transfer completes only when both are high.

- TREADY for an ungranted master is always low.

- TDEST must be stable while TVALID is asserted and the master is waiting for grant or transfer.

- TID and TUSER are pass-through and must be stable on every valid beat.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	AXI-Stream Rich Interface (TSTRB + AMBA5)

## 13.1	Rich Master Interface (S_AXIS)

When --strb-width N and/or --amba5 are used, the master interface adds the following signals in addition to the base set. The check signals are IHI 0051B chapter 5 (Check_Type = Odd_Parity_Byte_All). Names and widths match rtl/amba/axis5/*.

Rich Master Interface Additional Signals

| Signal | Width | Direction | Presence | Description |
| --- | --- | --- | --- | --- |
| s_axis_tstrb[m] | STRB_WIDTH | Input | --strb-width N | Byte qualifier / strobe. Pass-through to m_axis_tstrb. |
| s_axis_twakeup[m] | 1 | Input | --amba5 | Wakeup hint. May assert before TVALID. |
| s_axis_tdatachk[m] | TDATACHK_W = DATA_WIDTH/8 | Input | --amba5 | Odd parity per TDATA byte. Checked, then carried. |
| s_axis_tstrbchk[m] | TSTRBCHK_W = ceil(STRB_WIDTH/8) | Input | --amba5 and --strb-width N | Odd parity over TSTRB. Checked, then carried. |
| s_axis_tlastchk[m] | 1 | Input | --amba5 | ~TLAST. Checked, then carried. |
| s_axis_tidchk[m] | TIDCHK_W = ceil(ID_WIDTH/8) | Input | --amba5 | Odd parity over TID. Checked, then carried. |
| s_axis_tdestchk[m] | TDESTCHK_W = ceil(DEST_WIDTH/8) | Input | --amba5 | Odd parity over TDEST. Checked, then carried. |
| s_axis_tuserchk[m] | TUSERCHK_W = ceil(USER_WIDTH/8) | Input | --amba5 | Odd parity over TUSER. Checked, then carried. |
| s_axis_tvalidchk[m] | 1 | Input | --amba5 | ~TVALID. Checked every cycle out of reset. |
| s_axis_treadychk[m] | 1 | Output | --amba5 | ~TREADY, generated by the crossbar. |
| s_axis_twakeupchk[m] | 1 | Input | --amba5 | ~TWAKEUP. Checked every cycle out of reset. |
| s_parity_error[m] | 1 | Output | --amba5 | Sticky. Set by any check failure on master port m. Cleared by aresetn. |

## 13.2	Rich Slave Interface (M_AXIS)

Rich Slave Interface Additional Signals

| Signal | Width | Direction | Presence | Description |
| --- | --- | --- | --- | --- |
| m_axis_tstrb[s] | STRB_WIDTH | Output | --strb-width N | Byte qualifier from the granted master. |
| m_axis_twakeup[s] | 1 | Output | --amba5 | Wakeup hint. OR of all masters whose TDEST selects slave s. |
| m_axis_tdatachk[s], m_axis_tstrbchk[s], m_axis_tlastchk[s], m_axis_tidchk[s], m_axis_tdestchk[s], m_axis_tuserchk[s] | as on the master side | Output | --amba5 (tstrbchk also needs --strb-width N) | Payload check bits of the granted master, carried unchanged. All ones while no master is granted. |
| m_axis_tvalidchk[s] | 1 | Output | --amba5 | ~TVALID, generated by the crossbar. |
| m_axis_treadychk[s] | 1 | Input | --amba5 | ~TREADY. Checked every cycle out of reset. |
| m_axis_twakeupchk[s] | 1 | Output | --amba5 | ~TWAKEUP, generated by the crossbar. |
| m_parity_error[s] | 1 | Output | --amba5 | Sticky. Set by a bad TREADYCHK on slave port s. Cleared by aresetn. |

## 13.3	TSTRB Semantics

- TSTRB is treated as an opaque byte-qualifier sideband.

- The crossbar forwards the granted master’s TSTRB to the selected slave.

- Delta does not gate TVALID based on TSTRB or modify strobe bits.

## 13.4	TWAKEUP Semantics

TWAKEUP is an AMBA 5 hint, not a handshake signal:

- A master may assert s_axis_twakeup[m] before s_axis_tvalid[m] to wake the target slave.

- The crossbar decodes s_axis_tdest[m] and ORs the hint to the indicated slave.

- m_axis_twakeup[s] may be high while m_axis_tvalid[s] is low.

- No grant is required for TWAKEUP steering.

## 13.5	Check-Signal Semantics (IHI 0051B Chapter 5)

- Parity is odd. A check bit and the bits it covers hold an odd number of ones. Bit [n] covers bits [8n+7:8n], and the top bit covers a short remainder (section 5.3).

- Payload checks (TDATACHK through TUSERCHK) are checked only while TVALID is high. TVALIDCHK, TREADYCHK and TWAKEUPCHK are checked on every cycle out of reset.

- Payload check bits are carried end to end with the beat, never regenerated. A bad bit at a master port stays bad at the slave port.

- A failed check sets the port’s sticky error flag. It does not stall, drop or alter the beat.

- Every check bit is computed or checked by a rtl/common/dataint_parity instance. The compile closure of a generated --amba5 crossbar includes rtl/common/filelists/dataint_parity.f.

## 13.6	Parameter Implications

Rich-Flavor Parameter-to-Port Mapping

| Option | Generated Parameter | Port Effect |
| --- | --- | --- |
| --strb-width N | STRB_WIDTH = N | Adds s_axis_tstrb and m_axis_tstrb arrays. |
| --amba5 | TDATACHK_W, TIDCHK_W, TDESTCHK_W, TUSERCHK_W (plus TSTRBCHK_W with --strb-width), each derived from its payload width | Adds TWAKEUP, the nine check signals and the two *_parity_error arrays. Requires a whole-byte --data-width. |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Clock and Reset

## 14.1	Clock

Clock Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| aclk | 1 | Input | Positive-edge clock for all registered state. |

All registered logic (grants, last_grant, packet-active flags, counters) is clocked on the rising edge of aclk. Combinational decode and mux logic is not registered.

## 14.2	Reset

Reset Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| aresetn | 1 | Input | Active-low asynchronous reset. |

aresetn resets all registered state:

- grant_matrix to zero.

- last_grant to zero.

- packet_active to zero.

- Packet counters to zero.

The reset assertion is asynchronous; de-assertion is synchronous to aclk in the checked-in RTL through the ALWAYS_FF_RST macro.

## 14.3	Clocking Rules

- All masters and slaves share the same aclk.

- Delta does not contain clock-domain crossing logic.

- Inputs are sampled on the rising edge of aclk; outputs are driven combinationally or from registers.

## 14.4	Reset Recommendations

- Assert aresetn for at least several clock cycles after power and clock stability.

- Do not rely on Delta’s outputs during reset; combinational mux outputs may be zero or undefined during reset.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Throughput Characteristics

## 15.1	Aggregate Throughput

Because each slave has its own arbiter and mux, aggregate throughput scales with the number of slaves. In the best case — every master targets a different slave and every slave is ready — the crossbar can accept one beat per master per cycle.

## 15.2	Per-Slave Throughput

When multiple masters contend for the same slave, the round-robin arbiter gives each contending master an equal share of the slave’s bandwidth. With K contending masters, each sees approximately 1/K of the slave’s available bandwidth.

## 15.3	Head-of-Line Blocking

A master blocked on one slave does not block traffic to other slaves. Each slave arbiter is independent. However, a master that targets a heavily contended slave will wait until its round-robin turn and until the current packet completes.

## 15.4	Throughput Targets

Throughput Targets

| Metric | Target | Notes |
| --- | --- | --- |
| Per-slave throughput | Up to 1 transfer/cycle | When one master continuously feeds one ready slave. |
| Aggregate throughput | Up to NUM_MASTERS transfers/cycle | When all masters target different ready slaves. |
| Mixed-traffic throughput | ≥ 0.7 transfers/cycle per active master | Under moderate contention in the performance model. |

## 15.5	Factors That Reduce Throughput

- Contention for the same slave.

- Downstream backpressure (m_axis_tready[s] == 0).

- Multi-beat packets that lock the slave until TLAST.

- Arbitration overhead of one cycle when switching masters after TLAST.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 16	Latency Analysis

## 16.1	Cycle Breakdown

The generated flat crossbar has a fixed two-cycle latency from master input to slave output:

Latency Cycle Breakdown

| Cycle | Action |
| --- | --- |
| 0 | Master asserts s_axis_tvalid and s_axis_tdest. |
| 1 | TDEST decode produces request_matrix; arbiter registers the grant. |
| 2 | Crossbar mux drives m_axis_tdata, m_axis_tvalid, etc. to the slave. |

## 16.2	Arbitration Delay

If the target slave is idle when the request arrives, the grant is registered on the next rising edge of aclk. If the slave is busy serving another packet, the new request waits until the current packet’s TLAST is accepted.

## 16.3	Backpressure Latency

m_axis_tready[s] propagates combinationally through the TREADY steering logic to s_axis_tready[m]. There is no additional registered delay on the backpressure path.

## 16.4	Latency Targets

Latency Targets

| Configuration | Latency | Notes |
| --- | --- | --- |
| Flat crossbar, idle slave | 2 cycles | Arbitration + registered grant. |
| Flat crossbar, contended slave | 2 + wait cycles | Wait depends on current packet length. |

## 16.5	Frequency Target

The flat crossbar is intended to run at ≥ 300 MHz on Xilinx UltraScale+ devices for typical configurations (e.g., 4×16 @ 64-bit). Actual Fmax depends on device, tool settings, and floorplanning.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 17	Resources

## 17.1	Estimates from the Performance Model

The bin/delta_performance_model.py script provides empirical resource estimates based on the flat topology. Values below are planning estimates, not synthesis results.

Resource Estimates

| Configuration | LUTs | FFs | Notes |
| --- | --- | --- | --- |
| 4×16 @ 64-bit | ~1,536 | ~1,536 | Base flavor, with counters. |
| 2×4 @ 32-bit | ~200 | ~150 | Small test configuration. |
| 4×16 @ 64-bit, no counters | ~1,400 | ~1,200 | --no-counters saves FF area. |

## 17.2	Scaling Laws

- LUTs: Grow roughly as NUM_MASTERS × NUM_SLAVES for arbitration plus NUM_SLAVES × DATA_WIDTH for the data mux.

- FFs: Grow roughly as NUM_MASTERS × NUM_SLAVES for the grant matrix and last_grant storage, plus NUM_MASTERS + NUM_SLAVES for packet counters when enabled.

- BRAM: None. Delta is a pure-logic implementation.

## 17.3	Area Optimization Knobs

Area Optimization Options

| Knob | Effect |
| --- | --- |
| --no-counters | Removes pkt_count_master and pkt_count_slave arrays. |
| Smaller NUM_MASTERS/NUM_SLAVES | Linear reduction in control logic. |
| Smaller DATA_WIDTH | Linear reduction in data mux area. |
| Disable richness options | Removes TSTRB muxing, and with --amba5 off removes the TWAKEUP decode, the check-signal mux and the per-port dataint_parity checkers. |

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 18	System Requirements

## 18.1	Toolchain Requirements

- Synthesis: Any SystemVerilog-2017-capable synthesis tool (Vivado, Yosys, Genus, etc.).

- Simulation: Any SystemVerilog simulator; cocotb is the planned verification framework.

- Lint: Verilator (--lint-only) is used for quick sanity checks.

## 18.2	Runtime Requirements

- Single clock domain: all s_axis_* and m_axis_* interfaces use aclk.

- Active-low asynchronous reset: aresetn must be asserted at power-up.

- All upstream masters and downstream slaves must implement the AXI4-Stream handshake.

## 18.3	Interface Assumptions

- Masters drive valid TDEST values (0 to NUM_SLAVES-1).

- Slaves drive m_axis_tready independently.

- No master changes TDEST while waiting for a grant.

- TID and TUSER widths match the generated parameters.

- Packets may interleave and reorder (2026-10-19, owner direction, replacing the same-day “packets never interleave” assumption). Delta guarantees the order of flits within one packet and nothing between packets; an endpoint must not infer arrival order from send order. The tile’s cells order their writes to RAPIDS by waiting for every credit back (RAPIDS MAS, Tile Packet Header, Ordering Across the Fabric, O4). Open: whether the AXIS and CXS plane ports still deliver one packet’s flits together; RAPIDS’s header-then-payload framing depends on it.

- Pencilled in: a tag on every flit (2026-10-19, rapids TASK-033). For RAPIDS to take interleaved and reordered packets, every flit, body flits included, carries {sender[4:0], pos[3:0], head} from the source endpoint to the destination on AXIS tuser and CXS user. Delta forwards it and never reads it. Today a body flit’s tuser is zero.

## 18.4	Integration Checklist

- ☐ Confirm DATA_WIDTH, NUM_MASTERS, and NUM_SLAVES match the system.

- ☐ Confirm DEST_WIDTH and ID_WIDTH are wide enough for the generated module.

- ☐ Decide whether TSTRB or AMBA 5 sidebands are needed.

- ☐ Decide whether packet counters are needed (default on; use --no-counters to remove).

- ☐ Connect aclk and aresetn from the system.

- ☐ Verify downstream slaves can accept backpressure.

- ☐ Run verilator --lint-only on the generated RTL.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 19	Parameter Configuration

## 19.1	Generated Module Parameters

Generated Module Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| DATA_WIDTH | CLI --data-width | Width of TDATA in bits. |
| DEST_WIDTH | clog2(NUM_SLAVES) | Width of TDEST. |
| ID_WIDTH | clog2(NUM_MASTERS) | Width of TID. |
| USER_WIDTH | CLI --user-width | Width of TUSER. |
| NUM_MASTERS | CLI --masters | Number of master interfaces. |
| NUM_SLAVES | CLI --slaves | Number of slave interfaces. |
| STRB_WIDTH | CLI --strb-width | Width of TSTRB (only when --strb-width is used). |
| TDATACHK_W | DATA_WIDTH / 8 | Width of TDATACHK (only with --amba5). |
| TSTRBCHK_W | (STRB_WIDTH + 7) / 8 | Width of TSTRBCHK (only with --amba5 and --strb-width). |
| TIDCHK_W | (ID_WIDTH + 7) / 8 | Width of TIDCHK (only with --amba5). |
| TDESTCHK_W | (DEST_WIDTH + 7) / 8 | Width of TDESTCHK (only with --amba5). |
| TUSERCHK_W | (USER_WIDTH + 7) / 8 | Width of TUSERCHK (only with --amba5). |

## 19.2	Generator CLI Options

Generator Command-Line Options

| Option | Values | Default | Description |
| --- | --- | --- | --- |
| --topology | flat only | flat | Crossbar topology (tree/both retired 2026-10-12). |
| --masters | 1-32 | required | Number of master interfaces. |
| --slaves | 1-256 | required | Number of slave interfaces. |
| --data-width | 8-1024 | 64 | TDATA width in bits. |
| --user-width | 1-32 | 1 | TUSER width in bits. |
| --strb-width | ≥ 0 | 0 | TSTRB width; 0 omits TSTRB. |
| --amba5 | flag | off | Emit AMBA 5 TWAKEUP, the IHI 0051B ch. 5 check signals and the per-port *_parity_error flags. Requires --data-width to be a multiple of 8. |
| --no-counters | flag | off | Disable packet counters. |
| --output-dir | path | ../rtl | Output directory for generated SystemVerilog. |

## 19.3	Example Configurations

### 19.3.1	Base flavor 4×16 @ 64-bit

python3 bin/delta_generator.py \
    --topology flat --masters 4 --slaves 16 --data-width 64 \
    --output-dir rtl/

### 19.3.2	Rich flavor 2×4 @ 32-bit with TSTRB and AMBA5

python3 bin/delta_generator.py \
    --topology flat --masters 2 --slaves 4 --data-width 32 \
    --strb-width 4 --amba5 --output-dir /tmp/delta_amba5/
cp /tmp/delta_amba5/delta_axis_flat_2x4.sv rtl_test/delta_axis_flat_2x4_strb_amba5.sv

The generator names its output by shape. Writing straight into rtl_test/ would overwrite the plain delta_axis_flat_2x4.sv. The compile closure is rtl_test/filelists/delta_axis_flat_2x4_strb_amba5.f, which -f includes rtl/common/filelists/dataint_parity.f.

### 19.3.3	Base flavor without counters

python3 bin/delta_generator.py \
    --topology flat --masters 4 --slaves 16 --data-width 64 \
    --no-counters --output-dir rtl/

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 20	Verification Strategy

## 20.1	Current Status

As of 2026-10-12, Delta has one cocotb suite in dv/tests/. It covers the --amba5 flavor through the checked-in example rtl_test/delta_axis_flat_2x4_strb_amba5.sv (2x4, 32-bit, TSTRB), using a thin flattening harness (dv/tb/delta_axis_flat_2x4_strb_amba5_th.sv) and RDS-DV AXIS5 BFMs on every port:

- routing: every master sends to every slave at once. Each slave must receive exactly its beats, every beat must pass the slave BFM’s odd-parity check, the crossbar-driven TVALIDCHK/TWAKEUPCHK/TREADYCHK must equal the inversion of their signals on every cycle, and no error flag may set. At func and full the BFMs add random VALID gaps and READY stalls.

- wakeup: TWAKEUP with no TVALID reaches only the TDEST slave, with TWAKEUPCHK inverted.

- chk_errors: a bad TDATACHK, a bad TVALIDCHK and a bad TREADYCHK each set the right sticky flag and no other. Reset clears it.

Run it with make clean-all && make run-all-func-parallel in dv/tests/. The base flavor (rtl/delta_axis_flat_4x16.sv and the plain rtl_test/ shapes) still has no simulation suite. It is verified by lint and review only.

## 20.2	Generator Verification

- Run bin/delta_generator.py for several configurations.

- Compare output against checked-in reference RTL (rtl/delta_axis_flat_4x16.sv, rtl_test/delta_axis_flat_*.sv).

- Verify that default output is byte-identical to pre-2026-10-12 generations.

## 20.3	Lint Verification

verilator --lint-only rtl/delta_axis_flat_4x16.sv

Or use the provided rtl/Makefile:

cd rtl && make verilator

## 20.4	Planned RTL Verification

Still to add:

- All master-to-slave routing combinations.

- Concurrent traffic to the same slave (round-robin fairness).

- Concurrent traffic to different slaves (no crosstalk).

- Backpressure stress tests.

- Packet atomicity verification (no interleaving before TLAST).

- Base-flavor equivalents of the --amba5 suite (the AMBA 5 check signals are covered today).

- Reset behavior.

## 20.5	Formal Properties

When FORMAL is defined, the generated RTL includes:

- One-hot grant assertion per slave.

- TDEST range assertion per master.

These can be exercised with a SystemVerilog formal tool.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 21	Synthesis and Implementation

## 21.1	Synthesis Flow

The generated module is a flat SystemVerilog file intended for standard synthesis:

# Example Vivado flow
read_verilog rtl/delta_axis_flat_4x16.sv
synth_design -top delta_axis_flat_4x16 -part xczu7ev-ffvc1156-2-e

## 21.2	Timing Considerations

- The critical path is typically from s_axis_tvalid/tdest through the decode and mux to m_axis_tdata/tvalid.

- Because the grant is registered, the mux select is stable for a full cycle once a grant is made.

- The TREADY path is combinational from m_axis_tready to s_axis_tready.

## 21.3	Floorplanning Suggestions

- For large crossbars, consider registering s_axis_tdata at the master boundary if the combinational mux path is too long.

- Keep related master/slave groups physically close to reduce routing congestion.

- If Fmax is insufficient, the generator supports --no-counters and reduced DATA_WIDTH to ease timing.

## 21.4	Implementation Checklist

- ☐ Synthesize the generated module in the target tool.

- ☐ Check setup/hold timing on aclk.

- ☐ Verify aresetn deassertion is synchronous to aclk.

- ☐ Run a post-implementation functional simulation if the design is modified after generation.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Mesh Hierarchy

IMPLEMENTED (2026-10-15 amendment) — Mesh mode emits one module, delta_mesh_cell, and a fabric top. The owner re-named the hierarchy levels on 2026-10-15: tetrad = 2×2×1 cells, tile = 2×2×1 tetrads. For the datapath these are placement and naming; the tetrad/tile wrappers exist only to aggregate MonBus observation streams and live in the compute-engine tile IP, not in delta. The canonical tile definition is TILE_SPEC.md. This chapter’s old name “confluence” is pre-2026-10-15 history.

## 22.1	Placement Levels

Delta’s mesh groups identical cells into named levels so that routing tables, config images, and rapids attachment points can be named without forcing a separate datapath wrapper at every level.

### 22.1.1	Figure 7.1: Delta Mesh Placement Hierarchy

Delta Mesh Hierarchy

## 22.2	Cell

A cell is delta_mesh_cell, the one RTL module the generator emits for mesh mode. It has nine ports: six axis neighbor ports (±X, ±Y, ±Z), two instances of the conceptual diagonal axis (P_DIA at the tetrad scale and P_DIA2 at the tile scale), and one local port. A cell forwards flits by looking up a destination coordinate in a RAM routing table and applying credit-based flow control per VC.

## 22.3	Tetrad

A tetrad is 2×2×1 cells — four cells arranged in X and Y, one deep in Z. The four cells are fully connected as a K4 clique: perimeter edges over ±X/±Y plus both diagonals over P_DIA. The tetrad is a placement/config grouping and the first level of MonBus aggregation; an intra-tetrad hop is an ordinary cell hop.

## 22.4	Tile

A tile is 2×2×1 tetrads — sixteen cells, or four tetrads. The four tetrads are themselves fully connected as a K4 at tetrad scale: shared ±X/±Y boundaries for edge-adjacent tetrads and P_DIA2 for diagonal-tetrad pairs. Rapids DMA attaches at the local ports of boundary cells. The tile wrapper (outside delta) is the second level of MonBus aggregation.

## 22.5	Fabric

A fabric is 4×4×K tiles stacked in Z. The recursion is planar: the Z dimension lives between tile layers. The generator produces a fabric top that instantiates the cells-x × cells-y × cells-z grid of delta_mesh_cell, parameterizes coordinate widths and per-dimension wrap links, and exposes boundary local ports as rapids attachment points.

## 22.6	Why This Grouping

The grouping borrows the superswitch idea seen in ECLIPSE: clustering cells so a message can reach any cell in a local group cheaply. Delta keeps the naming and table-bounding benefit but does not promise single-cycle traversal across a tetrad or tile. That trade keeps the cell simple — one module, one set of tables, one set of allocators — while still letting software reason about locality.

## 22.7	Observation Modules (2026-10-15 amendment)

Every cell emits exactly one MonBus observer stream. The tetrad and tile wrappers (compute-eng-ip tile IP, not delta) aggregate these streams up the hierarchy: four cell streams merge into one tetrad-level MonBus, and four tetrad streams merge into one tile-top-level MonBus. The datapath is unchanged; the wrappers are placement-only observation aggregation. MonBus aggregation is a parallel effort and is not part of this delta task.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Cell and Flit

## 23.1	Cell Ports

IMPLEMENTED — delta_mesh_cell is generated by delta_generator.py --topology mesh. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Each cell has nine AXI4-Stream interfaces:

- 6 axis neighbor ports — +X, −X, +Y, −Y, +Z, −Z. Wrap links for any dimension are parameterizable with --torus=x,y,z.

- 1 diagonal tetrad port (P_DIA) — connects the cell to its diagonal mate inside the same 2×2×1 tetrad, making every tetrad a fully-connected K4.

- 1 diagonal tile port (P_DIA2) — connects the cell to its diagonal-tetrad corner mate inside the same 4×4×1 tile, making the four tetrads of a tile a fully-connected K4.

- 1 local port — host-facing. Boundary cells expose this port for rapids DMA attachment; interior cells connect to local compute (SERV, CGRA, ocelus) outside delta’s scope.

Conceptually, P_DIA and P_DIA2 are two instances of a single diagonal axis: one in-plane diagonal axis at tetrad scale and one at tile scale, plus the out-of-plane Z axis and the in-plane X/Y axes, give the cell four axes and nine ports.

All ports carry TDATA, TVALID, TREADY, and TLAST. The head beat also carries the packet header in TUSER. AMBA 5 sidebands (TWAKEUP, TDATACHK and the chapter 5 check signals) are flat-mode only in v1 — mesh mode rejects --amba5 at generation time (mesh ports carry no sidebands; the check-signal budget is an open item if a monitored mesh port is needed).

## 23.2	Clock and Reset

The mesh cell uses a single clock (aclk) and the active-low asynchronous reset (aresetn) common to the rest of delta. This is a deliberate simplification against the GALS direction common in the NoC literature. The trade: timing closure and generator predictability win over per-link clock flexibility. If a future system needs independent clock domains, that becomes a boundary problem solved outside the cell.

## 23.3	Flit Format

A flit is one DATA_WIDTH beat. A packet is a head flit, zero or more body flits, and a TLAST tail beat. The head flit carries the header in TUSER; body and tail flits carry only payload and TLAST.

### 23.3.1	Head-Flit Header Fields

Head-flit header fields carried in TUSER on the head beat.

| Field | Width | Purpose |
| --- | --- | --- |
| dest coord | 3×COORD_W | X, Y, Z destination. COORD_W is sized by the fabric dimensions. |
| src coord | 3×COORD_W | X, Y, Z source, used for responses and counters. |
| QoS class | 2 | 0 = config, 1 = latency, 2 = bulk. |
| VC | 1 | Assigned at injection per §6. |
| multicast | 1 | Set for multicast packets; group/mask reference is mode-specific. |
| misroute budget | 4 | Decremented on each non-minimal hop; packet dropped and counted at 0. |
| source-route ptr | 8 | Indexes an inline port list when source-route mode is active. |
| packet ID | 4 | Tolerates reordering at endpoints under multi-VC or adaptive routing. |

Note (2026-10-19): The checked-in RTL (delta_mesh_cell.sv, generated from bin/delta_mesh/header.py) is binding for these widths.

Proposed (2026-10-16, not implemented): the compute-engine tile proposes extending this header with a level field in the coordinates, a payload type (cfg / prog / mem), a length and a target address. The description lives in TILE_SPEC.md Section 11, not here.

Payload streams pass transparently through the cell once the head beat has supplied the route. This is the same pass-through principle used by flat mode’s TDEST handling, only the routing information now lives in TUSER rather than TDEST.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Routing and Configuration

## 24.1	Routing Tables

IMPLEMENTED — delta_mesh_cell contains a RAM routing table. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Each delta_mesh_cell holds a RAM routing table. The lookup key is (destination coordinate, QoS class) and the result is (output port mask, VC). Because QoS classes are isolated, each class can have an independent route to the same destination, giving path diversity without changing the cell datapath. The output port mask is 9 bits wide, one bit per cell port.

The table is a standard node-table implementation: one lookup per hop. That lookup penalty is why source-route mode (§7.3) exists for latency-critical systolic schedules.

## 24.2	Reset Default

At reset every cell loads a hardwired dimension-ordered route toward the rapids boundary. The route is connected and acyclic by construction, so it forms a safe bootstrap path even before software loads custom tables. In-band config packets ride this default route.

## 24.3	Table Constraints

The generator and the table-load checker enforce three rules that keep the mesh deadlock-free:

- VC0 entries must form a connected, acyclic subrelation in the extended channel-dependency graph. This is checked at table-load time, not just at reset.

- VC1 entries must be minimal, or must carry a misroute budget and create no indirect dependence on VC0.

- VC1→VC0 transitions are permitted at any hop; VC0→VC1 transitions are forbidden.

## 24.4	In-Band Configuration

Config packets target a reserved destination and are intercepted by the cell’s ejection block instead of being delivered to the local port. The config format is {op (read/write), entry index, entry data}. Reads return response packets to the source so software can load-verify.

Per-cell unicast loading is the primary way to install custom routes: N cells × M entries are written at boot, and individual entries can be rewritten at runtime. Config traffic uses QoS class 0 and the reset-default route; a boundary cell or rapids injects the config stream.

## 24.5	Broadcast Mode-Switch

A broadcast mode-switch message switches the entire fabric to one of the standard modes. Every cell stores generator-emitted images for the standard modes in ROM or preloaded RAM. The new mode applies only after a fabric-wide ack barrier: each cell returns a config read-response ack, and the injector treats the mode as active only when all acks are in. No packet straddles two modes.

Mode 0 is custom — the per-cell unicast tables. Modes 1-10 are the standard fabric modes (modes 9 and 10 added 2026-10-15 for the P_DIA/P_DIA2 diagonal-axis K4 images). Switching among standard modes does not touch the custom tables.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Flow Control and Deadlock

## 25.1	Credit-Based Flow Control

IMPLEMENTED — delta_mesh_cell uses credit-based flow control per VC. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Delta’s mesh uses credit-based flow control per VC. Each output port tracks the number of free buffer slots in the downstream neighbor. A flit leaves only when a credit is available, and a credit returns only after the downstream cell has consumed the flit. Credit-counter depth is sized to cover round-trip latency; undersizing drops the channel duty factor below one.

## 25.2	Two Virtual Channels

The mesh has 2 VCs:

- VC0 — the escape channel. It carries only reset-default and turn-model-conforming routes that form a connected, acyclic subrelation.

- VC1 — the adaptive channel. It may carry minimal routes or routes with a bounded misroute budget.

This is Duato’s Protocol: one adaptive channel set plus one deterministic escape set. The escape set breaks any cycle that could form in the adaptive set.

## 25.3	VC Transition Rules

A packet may move from VC1 to VC0 at any hop. VC0 to VC1 is forbidden. Because every VC1 packet can always drain into VC0, cycles in the adaptive channel cannot close into a deadlock.

## 25.4	Torus Wrap and the Dateline Rule

Per-dimension torus wrap (--torus=x,y,z) restores edge symmetry and halves diameter, but a wrap channel creates a cycle that plain dimension-ordered routing cannot break. For each wrapped dimension, one designated wrap channel participates in VC0 only through the escape subrelation. Other wrap traffic uses VC1 with the misroute budget. This dateline rule keeps the 2-VC budget sufficient even with wrap links.

## 25.5	Livelock

Non-minimal routing is the only source of livelock. The misroute budget in the head flit bounds it: every non-minimal hop decrements the budget, and when it reaches zero the packet is dropped and counted. Minimal routing is the normal case; the budget exists to bound VC1 alternates.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Quality of Service

## 26.1	Three Traffic Classes

IMPLEMENTED — QoS arbitration is built into delta_mesh_cell. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Delta’s mesh defines 3 QoS classes:

- Class 0 — config/control — highest priority; must never be blocked by data traffic.

- Class 1 — latency — interactive or latency-sensitive traffic.

- Class 2 — bulk — throughput traffic that can tolerate delay.

This split mirrors the control/data-plane priority split common in NPU practice.

## 26.2	Strict Priority With Aging

Every output port uses strict priority among the classes. To prevent starvation of lower-priority traffic, the arbiter includes programmable aging: a lower-priority request that has waited too long is temporarily promoted above a newer higher-priority request.

Aging alone is not enough for class 2 under heavy class-0/1 load. A bulk minimum reservation guarantees that some fraction of each cycle is spent serving bulk traffic.

## 26.3	Non-Interference

Each class has separate VCs and separate injection buffers. Shared buffers let tree-saturation traffic in one class block unrelated traffic even with priority bits, so delta keeps the resources separate. Config class traffic gets its own VC0 slice so control can never be bulk-blocked.

## 26.4	Per-Class Routes

The routing table is keyed by (destination, QoS class), so each class can follow an independent route to the same destination. Latency class might take a low-contention path while bulk class takes the shortest physical path.

## 26.5	Within-Class Fairness

Per-flow weighted round-robin or deficit round-robin within a class is a known enhancement. It is deferred until a user needs it; the current design provides class isolation and class-level fairness only.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Multicast

## 27.1	VC0 Only

IMPLEMENTED — multicast is built into delta_mesh_cell. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Multicast packets use VC0 only. Restricting multicast to the escape channel keeps the multicast paths inside the same connected, acyclic subrelation used by unicast traffic, so multicast adds no new channel dependencies beyond the unicast deadlock proof.

## 27.2	BRCP-Conformed Paths

Multicast routes are table entries that produce an output port mask instead of a single output. The set of paths a multicast can take is constrained to conform to the base unicast routing (BRCP — base routing conformed path-based multicast). A cell replicates a packet only onto ports that the unicast tables would also use.

## 27.3	Source-Side Replication

When a packet must fan out, the branching cell enqueues one copy per destination port. This source-side replication discipline avoids broadcasting every packet everywhere and filtering later. Incremental allocation serves whichever masked outputs are free each cycle and releases the input only after all destinations have been served.

## 27.4	Multiple Delivery VCs

Multicast reception can deadlock even when the forward paths are correct, because multiple deliveries can compete for the same buffer. Each cell therefore provides multiple delivery VCs for multicast reception.

## 27.5	No Responses

Multicast connections do not carry responses. Responses create merge volume and ordering problems that are better handled by unicast many-to-one traffic. If software needs an acknowledgment, it sends a separate unicast gather after the multicast completes.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	Systolic Support and Fabric Modes

## 28.1	Torus Wrap Links

IMPLEMENTED — per-dimension torus wrap is generated with --torus=x,y,z. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Each dimension can be wrapped independently with --torus=x,y,z. Periodic boundaries restore edge symmetry and halve diameter. The wrap links are folded so wire length doubles rather than growing with the dimension size. Deadlock freedom on wraps is handled by the dateline rule in §4.

## 28.2	Route Lookahead

The cell computes the next hop’s output port one hop ahead of the flit’s arrival. In steady state this collapses per-hop latency toward one cycle, which is the target for systolic repeat-cycle timing. The measured pipeline is 4 cycles/hop today (issue #120); the 1 cycle/hop target is deferred.

## 28.3	Source-Route Mode

For static systolic schedules, the injection shim can place an inline port list in the head-flit header. Intermediate cells forward without a table lookup, giving fixed-latency, table-free paths. Node-table routing remains the default for dynamic traffic. Source-route mode is generated with --source-route.

## 28.4	Speculative Allocation

The generator can enable speculative VC and switch allocation with --speculation. A head flit requests both a VC and a switch slot before the route is fully known, overlapping the allocators with the lookup. Mis-speculated grants are discarded and retried.

## 28.5	Standard Fabric Modes

A broadcast mode-switch (§3) selects one of 10 standard fabric modes. Every cell stores generator-emitted images for these modes. Mode 0 is custom (per-cell unicast tables).

### 28.5.1	Standard Fabric Modes

Standard fabric modes selected by broadcast mode-switch.

| Mode | Name | Pattern |
| --- | --- | --- |
| 1 | Mesh (default) | Generic packet switching using RAM tables; reset default is dimension-ordered routing. |
| 2 | Systolic-1D | Linear nearest-neighbor chains, row or column selected, optional torus ring; fixed source-route paths. |
| 3 | Wavefront-2D | 2-D diagonal ripple with skewed E/W and N/S streams; per-diagonal static tables or source routes. |
| 4 | Stencil-3D | 3-D halo/ghost-zone exchange with periodic boundaries; nearest-neighbor tables plus torus wrap on all dims. |
| 5 | Broadcast | One-to-all; multicast on VC0 with port masks to every cell. |
| 6 | Convergecast/reduce | All-to-one reduction tree; reverse-broadcast tables plus ocelus counter reduction. |
| 7 | All-to-all/transpose | Permutations such as transpose and shuffle/butterfly stages; per-stage table images. |
| 8 | Pipeline + rapids-stream | Boustrophedon serpentine chain; boundary cells stream to and from rapids DMA. |
| 9 | Tetrad all-to-all (K4) | Every 2×2×1 tetrad fully connected: perimeter via ±X/±Y, both diagonals via P_DIA; edge-adjacent and diagonal mates reach in one hop. |
| 10 | Tile all-to-all (K4 of tetrads) | The 4 tetrads of a tile fully connected: shared boundaries via ±X/±Y, diagonal tetrad pairs via P_DIA2; intra-tile dsts reach in 1..2 hops. |

Mode images are data, not new hardware. Adding a mode means adding an image and a verification suite.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Observability and DFT

## 29.1	Ocelus Counter Suite

IMPLEMENTED — delta_mesh_cell exposes an eleven-entry 32-bit counter bank read through the config port. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Each cell exposes saturating 32-bit counters readable through the config port. The suite is informed by NPU observability practice rather than by the sparse NoC literature. Counter indices 0..10 are defined; indices above 10 read back 0. The counter bank is selected by setting table_index[15] = 1 on a config read.

Ocelus counter bank indices.

| Index | Counter | Description |
| --- | --- | --- |
| 0 | C_FLITS_IN | Flits received by the cell. |
| 1 | C_FLITS_OUT | Flits transmitted by the cell. |
| 2 | C_PKTS_IN | Packets received (TLAST handshakes). |
| 3 | C_PKTS_OUT | Packets transmitted (TLAST handshakes). |
| 4 | C_STALL_CYCLES | Cycles the cell saw output stall. |
| 5 | C_CREDIT_STALL | Cycles a granted output waited for downstream credit. |
| 6 | C_ERR_DROP | Malformed-destination drops (also mirrored on o_err_drop_cnt). |
| 7 | C_MODE_ACKS | Broadcast mode-switch acks received (also mirrored on o_mode_acks). |
| 8 | C_BULK_STARVE | Cycles bulk reservation was due. |
| 9 | C_SPEC_COMMITS | Speculative grants that committed. |
| 10 | C_SPEC_REVOKES | Speculative grants that were revoked. |

Counters are frozen when ENABLE_COUNTERS == 0. They clear on aresetn.

## 29.2	DFT Test Mode

Test mode configures each cell into one of three deterministic routing patterns so the mesh wiring behaves as a concatenated bus. With three patterns, stuck-at and bridge faults are testable with approximately ⌈log₂(n)⌉ patterns. The mode is controlled through the config port using a DFT escape write (table_index[15:12] = 4'hD, payload bit 0 enables DFT, bits 2:1 select the pattern), and each cell drives an o_dft_active output.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Performance and Verification

## 30.1	Performance Model

IMPLEMENTED — bin/delta_mesh_perf.py models the mesh and is calibrated by the regression suite. The design is per the delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

The bin/delta_mesh_perf.py model uses the standard interconnection-network cost model:

- Zero-load latency T0 = H·tr + D/v + L/b, where H is the hop count, tr is the per-hop router delay, D is the distance, v is the propagation speed, L is the packet length, and b is the channel bandwidth.

- Queuing is modeled as M/D/1 under deterministic flit service.

- Saturation throughput is derived from the maximum channel load γ_max, using bisection-bound and multicommodity-flow analysis. The model reports minimum delivered flow, not average.

- Traffic patterns include uniform random, nearest-neighbor/stencil, tornado, bit-complement, and transpose.

- Methodology uses infinite source queues, warm-up/measurement/drain phases, and latency measured from generation to departure.

Loaded-latency microbenchmarks follow NPU benchmarking practice. Network-calculus delay and backlog bounds for the fixed-priority classes are a phase-2 addition.

## 30.2	Verification Plan

The mesh verification plan is an 8-suite cocotb program. Each suite is a generator configuration plus a dedicated test file:

- Routing — reset default DOR, programmed-table routing, table load via config port with readback verify.

- QoS — strict priority order, aging inversion under saturation, bulk minimum reservation, class isolation under tree-saturation attack.

- Multicast — fanout correctness, no-response rule, delivery-VC deadlock soak.

- Deadlock soak — random traffic under adversarial-but-valid random tables, plus VC1 stress with VC0 escape, soaking to N×10⁶ flits.

- Systolic — torus nearest-neighbor ring/wavefront with measured per-hop latency, source-route fixed-latency path check, and 3-D stencil halo exchange.

- Counters — every ocelus counter against a golden model.

- DFT mode — three patterns route correctly; pattern count check.

- Perf model calibration — measured versus modeled latency and throughput on the design-spec §10 patterns; report deltas.

Formal assists include an offline VC0-table acyclicity checker, one-hot grant assertions per output, and a TLAST-before-grant-release assertion that preserves packet atomicity. Detailed microarchitecture verification lives in the Delta MAS; the design spec dated 2026-10-13 is the source of truth for the architecture described here.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	Deferred Items

STATUS — The mesh cell RTL is implemented. The items below remain deferred or are tracked in other workstreams.

The following features were considered and deferred:

- CXS interface mode — IMPLEMENTED (owner decision 2026-10-16): --ifc {axis,cxs} selects the interface plane; CXS mode wraps the unchanged AXIS core with native AMBA 5 CXS ports so a CXS rapids attaches without adapters. Validated for directed traffic; concurrent-load CXS shows a duplication/loss corner tracked as issue #122.

- MonBus observation aggregation — the tetrad/tile wrappers that merge cell MonBus streams live in the compute-eng-ip tile IP, not in delta (parallel session; datapath unchanged).

- Route lookahead to 1 cycle/hop — the aspirational systolic target; current measured pipeline is 4 cycles/hop (issue #120).

- SRAM FIFO_IMPL — the --fifo-impl sram option hangs in the current RTL (issue #118); regression uses --fifo-impl reg.

- GT/TDM slot-table circuits — Ethereal/NoC slot tables, headerless GT flits, end-to-end credit flow control, and optional VOQ. Phase-2 candidate.

- Fault-tolerant routing — block fault regions, software absorb/reinject, and fault-status registers. Multi-path tables with failover as a table write are now being worked: hazards in Resilient Routing: Open Hazards, delta DELTA-013.

- Barrier/superstep sync wave — ECLIPSE-style synchronization wave; lands with the CGRA work, not delta.

- Express channels/cubes — little use at the target radix; skipped.

- Logical-to-physical address mapping — revisit if remapping or fault-recovery demands it.

- GALS/async links — single clock is a deliberate simplification contrary to this literature direction.

- VOQ option — area/throughput trade; add when a user needs it.

- Within-class per-flow fairness (WRR/DRR) — deferred until a user asks.

- Flit-reservation flow control — revisit if systolic latency targets are not met by route lookahead.

- Reliable-mode resend/NACK — only if a control-plane user needs lossless delivery beyond class-0 VC isolation.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 32	Resilient Routing: Open Hazards

OPEN — design hazards recorded 2026-10-18 for delta DELTA-013. Nothing here is built. The solutions are worked through in that task.

Resilience means a link or node failure changes the topology. Delta absorbs it by rewriting route tables in band with the existing config packets (Routing and Configuration, In-Band Configuration), with no new wires. To make failover a table write, each table entry holds a set of next hops, a primary and an alternate. A flit’s path then depends on table state even with no failure, so a request and its reply can take different paths. This section records what that breaks, under correct configuration.

## 32.1	Design Fork: Which Leg Adapts

Delta routes per packet: the head flit picks the output and the body follows it (wormhole), so adaptivity is a per-packet choice at each node, never per flit. “Forward” below is the request direction, from the requester; “return” is the reply and the credit traffic it causes.

Adaptive-routing options

| Option | Forward leg | Return leg | Verification |
| --- | --- | --- | --- |
| Fully adaptive | Best next hop at every node | Best next hop at every node | Every path is a function of load; the escape-VC argument must cover both legs at once |
| Pinned forward, adaptive return | The table’s chosen next hop, deterministic for a given table state | Best next hop at every node | One leg is deterministic, which keeps the escape-VC story simple |

Pinned forward is the leading option: it is much easier to verify, and for a resilience goal it is usually enough, because the failure being survived is on the forward path (failover rewrites it) and the return recomputes anyway. It does not restore ordering: packets before and after a failover flip take different forward paths, and the return leg adapts, so the ordering rules below apply under either option.

## 32.2	Multi-Path Tables and Failover

Today an entry is one (output port mask, VC) per (destination, QoS class) (Routing and Configuration, Routing Tables); path diversity comes only from routing classes differently. Failover needs the alternate in the entry, or a second table image, so the switch is a write and not a recompute.

Failover hazards

| # | Hazard | Mechanism | Candidate direction |
| --- | --- | --- | --- |
| R1 | Mixed tables during an update | Unicast config writes land one cell at a time, so mid-update the fabric runs a mix of old and new entries. The load-time checker validates one table, not the mix; a mix can loop or black-hole a destination, and a VC0 mix can close a cycle | Install the alternates everywhere first, then flip every cell at once through the broadcast mode-switch ack barrier (Routing and Configuration), which already guarantees no packet straddles two modes |
| R2 | The escape channel loses connectivity | Duato’s protocol needs VC0 to reach every destination. A failed link on the VC0 subrelation leaves VC1 traffic that drains into VC0 with nowhere to go | Failover recomputes VC0 on the surviving graph (connected and acyclic, e.g. up/down over the tile hierarchy), not only the VC1 alternates |
| R3 | The config path crosses the failure | Config rides QoS 0 on the reset-default route. If the failed link is on that route, the cells beyond it cannot be reached to fail them over | The bootstrap route carries its own alternate; in the tile, the path through the other root |
| R4 | A misroute drop loses state | A VC1 packet whose misroute budget reaches 0 is dropped and counted (Flow Control and Deadlock, Livelock). Alternates add non-minimal hops, so drops become reachable in normal traffic. A dropped data or credit packet breaks the tile rule that credits always return (TILE_SPEC §11.9): the sender’s strap leaks until a resync | Tile traffic that carries credits never misroutes (minimal or VC0 only), and every drop is reported on MonBus |

Packets in flight on the failed link are lost. That is the existing root failover assumption (TILE_SPEC §11.4): RAPIDS times the transfer out and software re-issues it, and a resync set (TILE_SPEC §11.10) restores leaked straps.

## 32.3	Deadlock

Delta already has the standard fix: two VCs per class (Flow Control and Deadlock), VC0 the deterministic escape channel, VC1 adaptive, VC1→VC0 permitted at any hop and VC0→VC1 forbidden, with the dateline rule for torus wrap. The two-VC budget is met and stays the rule for every QoS class, since each class has its own VCs (Quality of Service).

What changes with failover is that the escape subrelation stops being a design-time artefact. Today it is the reset default plus generator-checked tables. After a failure it is whatever the surviving graph allows, so R2 is the deadlock hazard: the escape rule must be re-derived and re-checked on every failover, including the mixed state of R1.

## 32.4	Ordering

Ordering contract

| Guaranteed | Not guaranteed |
| --- | --- |
| The flits of one packet, in order (one packet, one path) | Order between packets, even from one source to one destination in one class, once tables hold alternates or a failover flips mid-stream |

Implicit ordering across the fabric is gone, and this is documented so nothing is built on it. Order becomes a requester-side property: a transaction ID on every request, echoed in its reply, with the requester reordering, or delivering in order per ID as AXI IDs do. The head flit’s 4-bit packet ID (Cell and Flit) is the existing hook; 16 outstanding IDs per requester is its ceiling before aliasing. The tile and RAPIDS consequences are in TILE_SPEC and the RAPIDS MAS (ch03 Tile Packet Header, Ordering Across the Fabric).