RTL Design Sherpa

Delta Micro-Architecture Specification 1.2

October 20, 2026

Table of Contents

1 Delta Mas Index	11

2 Delta Hardware Architecture Specification Index	12

2.1 Overview	12

2.2 Related Modules	13

2.3 Navigation	14

2.3.1 Front Matter	14

2.3.2 Chapter 1: Introduction	14

2.3.3 Chapter 2: System Overview	14

2.3.4 Chapter 3: Architecture	14

2.3.5 Chapter 4: Interfaces	14

2.3.6 Chapter 5: Performance	14

2.3.7 Chapter 6: Integration	14

2.3.8 Chapter 7: Mesh Network	15

3 Document Information	16

3.1 Delta Micro-Architecture Specification	16

3.2 Revision History	17

3.3 Document Purpose	18

3.4 Intended Audience	19

3.5 Related Documents	20

4 Overview	21

4.1 What This Document Describes	21

4.2 Generated Module Structure	22

4.3 Key Micro-Architectural Decisions	23

4.4 Block Diagram	24

4.4.1 Figure 1.1: Delta Micro-Architecture Block Diagram	24

4.5 Reference RTL	25

5 Request Decode	26

5.1 Purpose	26

5.2 Implementation	27

5.3 TDEST Range Check	28

5.4 Relationship to Arbitration	29

5.5 Notes	30

6 Per-Slave Round-Robin Arbiter	31

6.1 State Variables	31

6.2 FSM Behavior	32

6.2.1 Idle State (packet_active[s] == 0)	32

6.2.2 Active State (packet_active[s] == 1)	32

6.3 Round-Robin Pseudocode	33

6.4 Fairness	34

6.5 Reset Behavior	35

7 Crossbar Multiplexer	36

7.1 Purpose	36

7.2 Implementation	37

7.3 Signal Selection	38

7.4 Default Values	39

7.5 Timing	40

8 TREADY Steering	41

8.1 Purpose	41

8.2 Implementation	42

8.3 Behavior	43

8.4 Deadlock Avoidance	44

9 TWAKEUP Steering (AMBA5)	45

9.1 Purpose	45

9.2 Implementation	46

9.3 Semantics	47

9.4 Integration Notes	48

10 Interface Check Signals (AMBA5)	49

10.1 Purpose	49

10.2 Parity Rule	50

10.3 Implementation	51

10.4 Design Notes	52

10.5 Verification	53

11 Packet Counters	54

11.1 Purpose	54

11.2 Implementation	55

11.3 Behavior	56

11.4 Disabling Counters	57

12 Assertions	58

12.1 Overview	58

12.2 One-Hot Grant Check	59

12.3 TDEST Range Check	60

12.4 Enabling Assertions	61

12.5 Notes	62

13 Multi-Master Contention	63

13.1 Scenario	63

13.2 Cycle Trace	64

13.3 Observations	66

13.4 Same-Cycle Release and Re-Arbitration	67

14 Packet Interleave Prevention	68

14.1 Rule	68

14.2 Mechanism	69

14.3 Why It Matters	70

14.4 Example: Attempted Interleave	71

14.5 Corner Cases	72

15 Module Structure	73

15.1 Generated File	73

15.2 Module Header	74

15.2.1 Base Flavor Header	74

15.2.2 Rich Flavor Additions	74

15.3 Internal Organization	75

15.4 Filelist	76

16 Parameters and Logic Implications	77

16.1 Parameter Summary	77

16.2 Width Calculations	78

16.3 Parameter Interaction	79

16.4 RTL Constants	80

17 Test Strategy	81

17.1 Current Status	81

17.2 Recommended Test Plan	82

17.2.1 1. Routing Coverage	82

17.2.2 2. Arbitration Fairness	82

17.2.3 3. Packet Atomicity	82

17.2.4 4. Backpressure	82

17.2.5 5. Reset	82

17.2.6 6. Rich Flavor	82

17.2.7 7. Formal Assertions	82

17.3 Lint and Generation Checks	84

18 Buffers and Virtual Channels	85

18.1 Per-Port Input Buffer Organization	86

18.2 Why Per-VC Reservation Is Required	87

18.3 Credit-Based Flow Control	88

18.4 Failure Mode: Ready Drops Mid-Burst	89

18.5 References	90

19 Pipeline Stages	91

19.0.1 Figure 6.1: delta_mesh_cell internal data path	91

19.0.2 Figure 6.1: delta_mesh_cell internal data path	91

19.1 Route Lookahead	93

19.2 Speculative VC and Switch Allocation	94

19.3 Handshake Discipline	95

19.4 Failure Mode: TLAST Arrives Without a Matching Grant	96

19.5 References	97

20 Routing Table	98

20.1 RAM Table Structure	99

20.2 Reset Image	100

20.3 In-Band Load and Readback	101

20.4 VC0 Acyclicity Check	102

20.5 Torus Wrap and the Dateline Rule	103

20.6 Standard Fabric Modes	104

20.7 References	105

21 Allocators	106

21.1 VC Allocation	107

21.2 Switch Allocation	108

21.3 One-Hot Grant Assertions	109

21.4 Multicast Allocation	110

21.5 Conservative Output-VC Reallocation	111

21.6 References	112

22 QoS Mechanisms	113

22.1 Strict Priority and Programmable Aging	114

22.2 Bulk Minimum Reservation	115

22.3 Class Non-Interference	116

22.4 Failure Mode: Aging Inversion Misconfiguration	117

22.5 References	118

23 Multicast Path	119

23.1 Source-Side Replication	120

23.2 Delivery-VC Assignment	121

23.3 The No-Responses Rule	122

23.4 Failure Mode: Tree-Based Multicast in Wormhole Networks	123

23.5 References	124

24 Observability Counters and DFT	125

24.1 Ocelus Counter Bank	126

24.2 DFT Test Mode	127

24.3 References	128

25 Verification Targets	129

25.1 cocotb Suites	130

25.2 Formal Assists	132

25.3 Performance Model Calibration	133

25.4 References	134

List of Figures

Figure 1.1: Delta Micro-Architecture Block Diagram	24

Figure 6.1: delta_mesh_cell internal data path	91

List of Tables

Table 1: Document Properties	16

Table 2: Revision History	17

Table 3: Key Micro-Architectural Decisions	23

Table 4: Arbiter State Variables	31

Table 5: Contention Trace	64

Table 6: Interleave Prevention	71

Table 7: Parameter-to-Logic Mapping	77

Table 8: Input buffer organization	86

Table 9: Credit counter sizing	88

Table 10: Cell pipeline stages	91

Table 11: Routing table entry layout	99

Table 12: Standard fabric mode catalog	104

Table 13: Allocator mapping	106

Table 14: QoS class mapping	113

Table 15: Class isolation resources	116

Table 16: Multicast discipline	122

Table 17: Ocelus counter bank indices.	126

Table 18: DFT routing patterns	127

Table 19: Mesh cocotb verification suites	130

# 1	Delta Mas Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Delta Hardware Architecture Specification Index

## 2.1	Overview

Version: 1.2 Date: 2026-10-20 Purpose: High-level hardware architecture specification for the Delta AXI4-Stream crossbar generator, including the planned mesh network mode

## 2.2	Related Modules

- Delta MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- QUICK_START.md - Hands-on generator guide

## 2.3	Navigation

Note: Every chapter below is one source file; the document build assembles the spec from these links.

### 2.3.1	Front Matter

- Document Information

### 2.3.2	Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### 2.3.3	Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### 2.3.4	Chapter 3: Architecture

- Block Diagram

- Data Flow

- Protocol Support

### 2.3.5	Chapter 4: Interfaces

- AXI-Stream Base Interface

- AXI-Stream Rich Interface (TSTRB + AMBA5)

- Clock and Reset

### 2.3.6	Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resources

### 2.3.7	Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

- Synthesis and Implementation

### 2.3.8	Chapter 7: Mesh Network

- Hierarchy

- Cell and Flit

- Routing and Configuration

- Flow Control and Deadlock

- Quality of Service

- Multicast

- Systolic Support and Fabric Modes

- Observability and DFT

- Performance and Verification

- Deferred Items

- Resilient Routing: Open Hazards

Last Updated: 2026-10-14 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Document Information

## 3.1	Delta Micro-Architecture Specification

Document Properties

| Property | Value |
| --- | --- |
| Document Title | Delta Micro-Architecture Specification |
| Version | 1.2 |
| Date | 2026-10-20 |
| Status | Released |
| Classification | Open Source - MIT License |

## 3.2	Revision History

Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-10-12 | RTL Design Sherpa | Initial release; describes the generated flat crossbar RTL including request decode, packet-atomic RR arbitration, mux, TREADY/TWAKEUP steering, counters, and assertions. |
| 1.1 | 2026-10-12 | RTL Design Sherpa | AMBA 5 interface parity per IHI 0051B ch. 5 (issue #102): TPARITY/PW_WIDTH/--parity-width replaced by the odd-parity check signals TDATACHK … TWAKEUPCHK (spec widths, checked at every port, sticky *_parity_error), matching rtl/amba/axis5. First dv/ suite. |
| 1.2 | 2026-10-20 | RTL Design Sherpa | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |

## 3.3	Document Purpose

This Micro-Architecture Specification (MAS) is the internals document for the Delta AXI4-Stream crossbar generator — the one you open when you need to know what the generated RTL actually does, block by block. It covers:

- Block-level implementation of the generated SystemVerilog

- Arbitration state behavior and grant hold/release rules

- Cycle-level behavior examples

- Parameter-to-logic implications

- Verification strategy

For the high-level architecture, interface requirements, and integration guidelines, see the companion Delta Hardware Architecture Specification (HAS).

## 3.4	Intended Audience

- RTL engineers reading or modifying the generated crossbar

- Verification engineers writing protocol checks

- System architects resolving timing or arbitration questions

- Technical reviewers auditing the implementation

## 3.5	Related Documents

- Delta HAS - Hardware Architecture Specification (high-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI assistant integration guide

- QUICK_START.md - Hands-on generator guide

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 4	Overview

## 4.1	What This Document Describes

This Micro-Architecture Specification (MAS) describes the internals of the SystemVerilog module produced by bin/delta_generator.py. It is intended for readers who need to understand, debug, or modify the generated RTL.

## 4.2	Generated Module Structure

The generator emits a single module named delta_axis_flat_MxN where M is NUM_MASTERS and N is NUM_SLAVES. The module contains:

- TDEST decode — builds the per-slave request matrix.

- Per-slave round-robin arbiter — packet-atomic grant logic.

- Crossbar mux — combinational selection of payload and control.

- TREADY steering — backpressure routing.

- TWAKEUP steering — AMBA 5 wakeup hint routing (optional).

- Packet counters — per-master and per-slave counters (optional).

- Assertions — ifdef FORMAL checks.

## 4.3	Key Micro-Architectural Decisions

Key Micro-Architectural Decisions

| Decision | Rationale |
| --- | --- |
| Registered grant, combinational mux | Two-cycle latency with a short critical path. |
| Per-slave arbiter | Independent scheduling; no head-of-line blocking across slaves. |
| TDEST direct decode | No address map; simpler and lower latency than address-range decode. |
| Packet-atomic grant | Preserves AXI4-Stream packet semantics; prevents interleaving. |
| Optional sidebands default off | Keeps base output byte-identical to earlier versions. |

## 4.4	Block Diagram

### 4.4.1	Figure 1.1: Delta Micro-Architecture Block Diagram

Delta MAS Block Diagram

The diagram follows one slave s of the generated RTL; every slave repeats it. Each master’s TVALID and TDEST become one bit of request_matrix[s]; the round-robin pick (the trapezium) chooses the first requester after last_grant[s] while no packet is active; the grant-state flops hold grant_matrix[s] until the TLAST handshake; and the combinational mux forwards the granted master’s TVALID and payload, which arrive on their own lane at the left. TREADY climbs the right lane to the granted master only. Dashed blocks are build options: TWAKEUP steering and the ingress and egress *CHK checks with --amba5, the packet counters unless --no-counters, and the assertions under `ifdef FORMAL.

## 4.5	Reference RTL

The descriptions in this document are verified against:

- rtl/delta_axis_flat_4x16.sv (base flavor)

- rtl_test/delta_axis_flat_2x4_strb_amba5.sv (rich flavor)

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 5	Request Decode

## 5.1	Purpose

The request decode block translates each master’s s_axis_tvalid and s_axis_tdest into a per-slave request matrix. A set bit in request_matrix[s][m] indicates that master m is requesting slave s.

## 5.2	Implementation

The decode is combinational:

logic [NUM_MASTERS-1:0] request_matrix [NUM_SLAVES];

always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++) begin
        request_matrix[s] = '0;
    end
    for (int m = 0; m < NUM_MASTERS; m++) begin
        if (s_axis_tvalid[m] &&
            (s_axis_tdest[m] < NUM_SLAVES[$clog2(NUM_SLAVES+1)-1:0])) begin
            request_matrix[s_axis_tdest[m]][m] = 1'b1;
        end
    end
end

## 5.3	TDEST Range Check

The comparison s_axis_tdest[m] < NUM_SLAVES prevents out-of-range indices from corrupting the request_matrix array. The width of the comparison is sized to cover the full NUM_SLAVES value.

## 5.4	Relationship to Arbitration

The request matrix is the input to each slave’s round-robin arbiter. Only slaves with at least one request bit set are candidates for a new grant.

## 5.5	Notes

- The decode does not depend on TREADY; a master may request even while stalled.

- TDEST is sampled combinationally and must be stable while TVALID is asserted.

- There is no address map; TDEST is the slave index directly.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 6	Per-Slave Round-Robin Arbiter

## 6.1	State Variables

Each slave s maintains three registered variables:

Arbiter State Variables

| Variable | Width | Purpose |
| --- | --- | --- |
| grant_matrix[s] | NUM_MASTERS bits | One-hot (or zero-hot) grant vector. |
| last_grant[s] | clog2(NUM_MASTERS) bits | Pointer to the most recently granted master. |
| packet_active[s] | 1 bit | Set while a packet is in progress on this slave. |

## 6.2	FSM Behavior

The arbiter has two operational states for each slave:

### 6.2.1	Idle State (packet_active[s] == 0)

- If request_matrix[s] is non-zero, perform round-robin arbitration.

- The search starts at last_grant[s] + 1 and wraps through all masters.

- The first requesting master is granted; last_grant[s] is updated and packet_active[s] is set.

- If no master requests, grant_matrix[s] remains zero.

### 6.2.2	Active State (packet_active[s] == 1)

- The current grant is held.

- On the cycle where m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s] is true, packet_active[s] is cleared and grant_matrix[s] is reset to zero.

- If TLAST is accepted, the next cycle may re-arbitrate for a new master.

## 6.3	Round-Robin Pseudocode

if (packet_active[s]) begin
    if (m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s]) begin
        packet_active[s] <= 1'b0;
        grant_matrix[s]  <= '0;
    end
end else if (|request_matrix[s]) begin
    grant_matrix[s] = '0;
    for (int i = 0; i < NUM_MASTERS; i++) begin
        int m = (last_grant[s] + 1 + i) % NUM_MASTERS;
        if (request_matrix[s][m] && grant_matrix[s] == '0) begin
            grant_matrix[s][m] = 1'b1;
            last_grant[s]      = m[$clog2(NUM_MASTERS)-1:0];
            packet_active[s]   = 1'b1;
        end
    end
end else begin
    grant_matrix[s] <= '0;
end

## 6.4	Fairness

Round-robin guarantees that no requesting master is starved. The longest wait for a contending master is bounded by the number of other contending masters and the length of their packets.

## 6.5	Reset Behavior

On reset:

- grant_matrix[s] <= '0

- last_grant[s] <= '0

- packet_active[s] <= 1'b0

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 7	Crossbar Multiplexer

## 7.1	Purpose

The crossbar mux selects the payload and control signals from the granted master and drives them onto the selected slave interface. There is one mux per slave.

## 7.2	Implementation

Each slave mux is combinational:

generate
    for (genvar s = 0; s < NUM_SLAVES; s++) begin : gen_slave_mux
        always_comb begin
            m_axis_tdata[s]  = '0;
            m_axis_tvalid[s] = 1'b0;
            m_axis_tlast[s]  = 1'b0;
            m_axis_tdest[s]  = '0;
            m_axis_tid[s]    = '0;
            m_axis_tuser[s]  = '0;

            for (int m = 0; m < NUM_MASTERS; m++) begin
                if (grant_matrix[s][m]) begin
                    m_axis_tdata[s]  = s_axis_tdata[m];
                    m_axis_tvalid[s] = s_axis_tvalid[m];
                    m_axis_tlast[s]  = s_axis_tlast[m];
                    m_axis_tdest[s]  = s_axis_tdest[m];
                    m_axis_tid[s]    = s_axis_tid[m];
                    m_axis_tuser[s]  = s_axis_tuser[m];
                end
            end
        end
    end
endgenerate

## 7.3	Signal Selection

The mux forwards the following signals from the granted master to the selected slave:

- TDATA

- TVALID

- TLAST

- TDEST

- TID

- TUSER

- TSTRB (when enabled)

- The payload check bits TDATACHK, TSTRBCHK, TLASTCHK, TIDCHK, TDESTCHK, TUSERCHK (with --amba5). They are carried unchanged, not regenerated. See Interface Check Signals (AMBA5).

## 7.4	Default Values

When no grant is active for a slave, the mux drives all outputs to zero and m_axis_tvalid[s] to low. This is a safe idle state. With --amba5, the payload check outputs idle at all ones instead, which is the odd parity of the all-zero idle payload.

## 7.5	Timing

Because the mux is combinational, the output settles after the grant and the input signals are stable. The grant is registered, so the mux select is stable for a full clock cycle once a packet begins.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 8	TREADY Steering

## 8.1	Purpose

TREADY steering routes each slave’s m_axis_tready back to the master that currently holds the grant for that slave. Ungranted masters see s_axis_tready[m] == 0.

## 8.2	Implementation

generate
    for (genvar m = 0; m < NUM_MASTERS; m++) begin : gen_master_tready
        always_comb begin
            s_axis_tready[m] = 1'b0;
            for (int s = 0; s < NUM_SLAVES; s++) begin
                if (grant_matrix[s][m]) begin
                    s_axis_tready[m] = m_axis_tready[s];
                end
            end
        end
    end
endgenerate

## 8.3	Behavior

- Only one slave can grant a given master at a time (packets are atomic and a master targets one slave per packet), so the loop effectively selects a single m_axis_tready[s].

- If a master is not currently granted by any slave, it is stalled.

- Backpressure propagates combinationally from slave to master.

## 8.4	Deadlock Avoidance

The design avoids deadlock because:

- Grants are released after TLAST, freeing the slave for new requests.

- TREADY is combinationally steered; a slave ready transition immediately reaches the granted master.

- There are no FIFOs or credit loops in the base crossbar.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 9	TWAKEUP Steering (AMBA5)

## 9.1	Purpose

When --amba5 is enabled, the generated module includes s_axis_twakeup and m_axis_twakeup ports. TWAKEUP is an AMBA 5 hint that a master will soon send a transaction; it may assert before TVALID.

## 9.2	Implementation

The wakeup steering is combinational and independent of the grant:

always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++) begin
        m_axis_twakeup[s] = 1'b0;
        for (int m = 0; m < NUM_MASTERS; m++) begin
            if (s_axis_twakeup[m] &&
                (s_axis_tdest[m] == s[$clog2(NUM_SLAVES)-1:0])) begin
                m_axis_twakeup[s] = 1'b1;
            end
        end
    end
end

## 9.3	Semantics

- m_axis_twakeup[s] is the OR of s_axis_twakeup[m] from every master whose TDEST points to slave s.

- No grant is required; the hint is decoded directly from TDEST.

- TWAKEUP may be high while TVALID is low.

- The hint is not a handshake; the slave is not required to respond before data arrives.

## 9.4	Integration Notes

- TWAKEUP is only emitted when --amba5 is specified.

- --amba5 also emits TWAKEUPCHK. The crossbar generates m_axis_twakeupchk[s] from this OR and checks s_axis_twakeupchk[m]. See Interface Check Signals (AMBA5).

- TWAKEUP does not affect arbitration timing.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 10	Interface Check Signals (AMBA5)

## 10.1	Purpose

When --amba5 is enabled, the generated module carries the AXI5-Stream interface-protection signals of IHI 0051B chapter 5 (Check_Type = Odd_Parity_Byte_All) and checks them at its own ports. This is the generate_check_logic() section of bin/delta_generator.py. Names and widths match rtl/amba/axis5/*.

## 10.2	Parity Rule

Odd parity: a check bit and the bits it covers hold an odd number of ones. Check bit [n] covers bits [8n+7:8n], and the top bit covers a short remainder (section 5.3). A one-bit check (TLASTCHK, TVALIDCHK, TREADYCHK, TWAKEUPCHK) is the inversion of its signal. Every check bit is one rtl/common/dataint_parity instance with CHUNKS=1 and parity_type = 1'b0 (odd), so the spec’s per-byte layout holds at any width.

## 10.3	Implementation

Two generate loops follow the TWAKEUP steering block.

Per master port (gen_s_chk[m]). The crossbar is the receiver here.

for (genvar g = 0; g < TDATACHK_W; g++) begin : g_tdatachk
    localparam int LO = 8 * g;
    localparam int PIECE_W = (DATA_WIDTH - LO < 8) ? (DATA_WIDTH - LO) : 8;
    dataint_parity #(.CHUNKS(1), .WIDTH(PIECE_W)) u_par (
        .data_in     (s_axis_tdata[m][LO +: PIECE_W]),
        .parity_in   (s_axis_tdatachk[m][g]),
        .parity_type (1'b0),
        .parity      (w_tdatachk_exp[g]),
        .parity_err  (w_tdatachk_err[g])
    );
end
// ... the same for TSTRB (with --strb-width), TID, TDEST, TUSER; TLAST is 1 bit
assign w_payload_err = s_axis_tvalid[m] && (|w_tdatachk_err || ...);

- The payload checks (TDATACHK, TSTRBCHK, TLASTCHK, TIDCHK, TDESTCHK, TUSERCHK) are enabled by TVALID.

- TVALIDCHK and TWAKEUPCHK are checked on every cycle out of reset.

- s_axis_treadychk[m] is generated: the parity output of a 1-bit dataint_parity over the s_axis_tready[m] the crossbar drives.

- Any failure sets s_parity_error[m], a sticky flop that only aresetn clears.

Per slave port (gen_m_chk[s]). The crossbar is the transmitter here.

- m_axis_tvalidchk[s] and m_axis_twakeupchk[s] are generated from the TVALID / TWAKEUP the crossbar drives.

- m_axis_treadychk[s] is checked on every cycle out of reset. A failure sets the sticky m_parity_error[s].

Carry. The payload check bits are not recomputed at the slave port. They travel through the crossbar mux (see Crossbar Multiplexer) with the same grant as TDATA. While no master is granted they idle at all ones, which is the odd parity of the all-zero idle payload.

## 10.4	Design Notes

- Carrying the check bits keeps protection end to end. A beat corrupted upstream of the crossbar still fails at the downstream receiver. If the crossbar regenerated the check bits from its own mux output, the fault would be hidden.

- A failed check does not stall or drop the beat. The fabric’s job is to report the fault, and the system decides how to respond.

- TWAKEUPCHK on the slave port is generated from the OR in the TWAKEUP steering block, so it is valid before any grant.

## 10.5	Verification

dv/tests/test_delta_axis_flat_2x4_strb_amba5.py (routing, wakeup and chk_errors) exercises every row above. A mutation check of the generated RTL showed the suite failing in each of these cases: even parity instead of odd, the ingress payload check removed, TDATACHK corrupted on the carry path, and TREADYCHK not inverted.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 11	Packet Counters

## 11.1	Purpose

Optional 32-bit packet counters track completed packets at the master boundary and the slave boundary. They are enabled by default and can be disabled with --no-counters.

## 11.2	Implementation

logic [31:0] pkt_count_master [NUM_MASTERS];
logic [31:0] pkt_count_slave [NUM_SLAVES];

generate
    for (genvar m = 0; m < NUM_MASTERS; m++) begin : gen_master_counters
        always_ff @(posedge aclk or negedge aresetn) begin
            if (!aresetn)
                pkt_count_master[m] <= '0;
            else if (s_axis_tvalid[m] && s_axis_tready[m] && s_axis_tlast[m])
                pkt_count_master[m] <= pkt_count_master[m] + 1;
        end
    end

    for (genvar s = 0; s < NUM_SLAVES; s++) begin : gen_slave_counters
        always_ff @(posedge aclk or negedge aresetn) begin
            if (!aresetn)
                pkt_count_slave[s] <= '0;
            else if (m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s])
                pkt_count_slave[s] <= pkt_count_slave[s] + 1;
        end
    end
endgenerate

## 11.3	Behavior

- A master counter increments when the master completes a packet (last beat accepted).

- A slave counter increments when the slave completes a packet (last beat accepted).

- Counters wrap at 32 bits.

- Counters are not exposed on module ports; they are internal observation points.

## 11.4	Disabling Counters

Use --no-counters to remove the counter logic and save area:

python3 bin/delta_generator.py --topology flat --masters 4 --slaves 16 \
    --data-width 64 --no-counters --output-dir rtl/

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 12	Assertions

## 12.1	Overview

All generated Delta modules include an ifdef FORMAL block with SystemVerilog assertions for debug and formal verification.

## 12.2	One-Hot Grant Check

Each slave must grant at most one master at a time:

assert property (@(posedge aclk) disable iff (!aresetn)
    $onehot0(grant_matrix[s])
) else $error("Grant matrix not one-hot for slave %0d", s);

$onehot0 allows the grant vector to be all-zero (idle) but not multi-hot.

## 12.3	TDEST Range Check

Each master must drive a valid slave index when it asserts TVALID:

assert property (@(posedge aclk) disable iff (!aresetn)
    s_axis_tvalid[m] |-> s_axis_tdest[m] < NUM_SLAVES
) else $error("TDEST out of range for master %0d", m);

## 12.4	Enabling Assertions

Define FORMAL during simulation or formal analysis:

verilator --lint-only -DFORMAL rtl/delta_axis_flat_4x16.sv

## 12.5	Notes

- Assertions are disabled by default to avoid simulation overhead.

- The TDEST range assertion duplicates the combinational safety check in the request decode block.

- Additional protocol assertions (TVALID stability, TLAST packet integrity) are candidates for the planned cocotb testbench.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 13	Multi-Master Contention

## 13.1	Scenario

Two masters, M0 and M1, both want to send packets to the same slave S0. S0 is initially idle. M0 sends a two-beat packet; M1 sends a one-beat packet.

## 13.2	Cycle Trace

Assumption: last_grant[S0] was 1 before the trace begins.

Contention Trace

| Cycle | Event | request_matrix[S0] | grant_matrix[S0] | packet_active[S0] | Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | M0 and M1 both request S0 | 2'b11 | 2'b00 | 0 | Slave idle. |
| 1 | Arbiter picks M0 (RR after last_grant=1) | 2'b11 | 2'b01 | 1 | Grant registered. |
| 2 | M0 beat 0 accepted | 2'b10 | 2'b01 | 1 | M1 waits. |
| 3 | M0 beat 1 (TLAST) accepted | 2'b10 | 2'b00 | 0 | Packet ends; grant released. |
| 4 | Arbiter picks M1 | 2'b10 | 2'b10 | 1 | M1 now granted. |
| 5 | M1 beat 0 (TLAST) accepted | 2'b00 | 2'b00 | 0 | Both packets complete. |

## 13.3	Observations

- M1 waits while M0 completes its packet.

- The arbiter does not re-arbitrate inside a packet.

- After TLAST, the next arbitration cycle grants M1 because M0 was the previous grant.

- Round-robin ensures M1 is not starved.

## 13.4	Same-Cycle Release and Re-Arbitration

On the cycle that TLAST is accepted, the grant is released. The next rising edge can register a new grant if a request is pending. This gives a one-cycle gap between back-to-back packets from different masters.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 14	Packet Interleave Prevention

## 14.1	Rule

A slave can source data from only one master within a single packet. The grant is locked from the first beat of the packet until the TLAST beat is accepted.

## 14.2	Mechanism

The packet_active[s] flag is set when a new grant is made. While packet_active[s] is high:

- grant_matrix[s] is held constant.

- The crossbar mux continues to select the same master.

- s_axis_tready for the granted master follows only m_axis_tready[s].

When m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s] is true, packet_active[s] clears and the grant is released.

## 14.3	Why It Matters

AXI4-Stream slaves often expect packet-atomic streams. If beats from different masters were interleaved, a slave receiving the mixed stream could misinterpret packet boundaries, corrupt state, or violate protocol.

## 14.4	Example: Attempted Interleave

Suppose M0 and M1 both target S0 and both have multi-beat packets:

Interleave Prevention

| Cycle | Without atomicity (hypothetical) | With Delta packet atomicity |
| --- | --- | --- |
| 0 | M0 beat 0 | M0 beat 0 |
| 1 | M1 beat 0 (interleaved!) | M0 beat 1 |
| 2 | M0 beat 1 | M0 beat 2 (TLAST) |
| 3 | M1 beat 1 (TLAST) | M1 beat 0 |

Delta prevents the interleaved case by holding M0’s grant until its TLAST.

## 14.5	Corner Cases

- If the granted master drops TVALID mid-packet, the slave sees no valid beats until the master reasserts it. The grant is still held.

- If the downstream slave is never ready, the packet can stall indefinitely; there is no timeout. This is consistent with standard AXI4-Stream behavior.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 15	Module Structure

## 15.1	Generated File

The generator produces one file per configuration:

delta_axis_flat_MxN.sv

where M is NUM_MASTERS and N is NUM_SLAVES.

## 15.2	Module Header

The module begins with parameters and port declarations. The exact port list depends on the selected richness options.

### 15.2.1	Base Flavor Header

module delta_axis_flat_4x16 #(
    parameter int DATA_WIDTH  = 64,
    parameter int DEST_WIDTH  = 4,
    parameter int ID_WIDTH    = 2,
    parameter int USER_WIDTH  = 1,
    parameter int NUM_MASTERS = 4,
    parameter int NUM_SLAVES  = 16
)(
    input  logic aclk,
    input  logic aresetn,
    // s_axis and m_axis base ports...
);

### 15.2.2	Rich Flavor Additions

When --strb-width is used, STRB_WIDTH is added to the parameter list and s_axis_tstrb/m_axis_tstrb are added to the ports.

When --amba5 is used, the check-width parameters TDATACHK_W, TIDCHK_W, TDESTCHK_W and TUSERCHK_W are added, plus TSTRBCHK_W with --strb-width. Each is derived from its payload width. The ports gain s_/m_axis_twakeup, the nine IHI 0051B check signals per side (*_axis_tdatachk … *_axis_twakeupchk), and the sticky s_parity_error[NUM_MASTERS] / m_parity_error[NUM_SLAVES]. The module then instantiates rtl/common/dataint_parity, so its filelist must -f include rtl/common/filelists/dataint_parity.f.

## 15.3	Internal Organization

Inside the module, the generator emits sections in this order:

- Header comment with configuration.

- Request decode.

- Per-slave round-robin arbiter.

- Crossbar data mux.

- TREADY steering.

- TWAKEUP steering (if enabled).

- Interface check signals (if --amba5).

- Performance counters (unless disabled).

- Assertions (under ifdef FORMAL).

## 15.4	Filelist

The checked-in RTL includes $REPO_ROOT-relative filelists under rtl/filelists/ for use with simulation and synthesis flows.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 16	Parameters and Logic Implications

## 16.1	Parameter Summary

Parameter-to-Logic Mapping

| Parameter | Generated From | Logic Implication |
| --- | --- | --- |
| DATA_WIDTH | --data-width | Width of TDATA; with --amba5 it also sets TDATACHK_W = DATA_WIDTH/8. |
| DEST_WIDTH | clog2(NUM_SLAVES) | Width of TDEST; determines request decode comparison width. |
| ID_WIDTH | clog2(NUM_MASTERS) | Width of TID; pass-through only. |
| USER_WIDTH | --user-width | Width of TUSER; pass-through only. |
| NUM_MASTERS | --masters | Size of s_axis_* arrays and grant/request matrix columns. |
| NUM_SLAVES | --slaves | Size of m_axis_* arrays and grant/request matrix rows. |
| STRB_WIDTH | --strb-width | Width of TSTRB; adds mux ports when > 0. |
| TDATACHK_W, TSTRBCHK_W, TIDCHK_W, TDESTCHK_W, TUSERCHK_W | derived in the header from the payload widths (IHI 0051B Table 5-1) | Check-signal widths with --amba5. Each sizes one carried *CHK mux field and one bank of dataint_parity checkers per master. TSTRBCHK_W appears only with --strb-width. |

## 16.2	Width Calculations

The generator computes:

dest_width = max(1, ceil(log2(num_slaves)))
id_width   = max(1, ceil(log2(num_masters)))

A single master or slave still gets a 1-bit width to avoid zero-width signals.

## 16.3	Parameter Interaction

- STRB_WIDTH == 0 omits TSTRB entirely.

- The check-width parameters exist only when --amba5 is set. Without it, no TWAKEUP or *CHK ports are generated.

- --amba5 with a --data-width that is not a multiple of 8 is rejected by the CLI parser. The old --parity-width option has been removed, because the spec fixes the check widths.

## 16.4	RTL Constants

The generator uses generate/genvar loops sized by NUM_MASTERS and NUM_SLAVES. Changing these parameters after generation requires regenerating the module; the parameter is set at generation time, not truly configurable post-generation. This is intentional: it allows the synthesizer to optimize the fixed-size structure.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 17	Test Strategy

## 17.1	Current Status

As of 2026-10-12, Delta has one cocotb suite, dv/tests/test_delta_axis_flat_2x4_strb_amba5.py. It runs against the checked-in --amba5 example through a flattening harness (dv/tb/), with RDS-DV AXIS5 BFMs (enable_parity=True) on all six ports and the TB class in dv/tbclasses/delta_axis_amba5_tb.py. Three tests run at gate, func and full: routing with every check signal verified (random VALID gaps and READY stalls at func and full), TWAKEUP steering, and check-error injection on every flag. The base flavor still has no simulation suite. The plan below remains the target for it.

## 17.2	Recommended Test Plan

When a testbench is created, it should cover the following areas.

### 17.2.1	1. Routing Coverage

- For every master m and every slave s, send at least one packet from m to s.

- Verify that only the target slave receives the packet.

- Verify that m_axis_tdest[s] matches the sent TDEST.

### 17.2.2	2. Arbitration Fairness

- Have multiple masters send continuous packets to the same slave.

- Verify that each master receives approximately equal bandwidth over time.

- Verify that no master is starved.

### 17.2.3	3. Packet Atomicity

- Have two masters send multi-beat packets to the same slave.

- Verify that slave receives the first packet’s beats contiguously up to TLAST before any beat of the second packet.

### 17.2.4	4. Backpressure

- Randomly deassert m_axis_tready[s].

- Verify that only the granted master stalls and that no data is lost.

### 17.2.5	5. Reset

- Assert aresetn mid-transaction.

- Verify all grants clear and counters reset.

### 17.2.6	6. Rich Flavor

- With --strb-width, verify TSTRB pass-through.

- With --amba5, verify TWAKEUP OR-per-slave, the odd-parity check signals (payload *CHK carried, TVALIDCHK/TWAKEUPCHK/TREADYCHK generated or checked) and the sticky *_parity_error flags. The existing suite already covers this.

### 17.2.7	7. Formal Assertions

- Run with +define+FORMAL or equivalent.

- Verify the one-hot grant and TDEST-range assertions never fire.

## 17.3	Lint and Generation Checks

For the base flavor, which has no testbench yet, verification consists of:

python3 bin/delta_generator.py --topology flat --masters 4 --slaves 16 \
    --data-width 64 --output-dir rtl/
verilator --lint-only rtl/delta_axis_flat_4x16.sv

The planned mesh cell verification suites are summarized in Chapter 6, Section 8.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Buffers and Virtual Channels

IMPLEMENTED: This chapter describes the delta_mesh_cell microarchitecture as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

## 18.1	Per-Port Input Buffer Organization

Each delta_mesh_cell has nine AXI4-Stream input ports: six axis neighbor ports (±X, ±Y, ±Z), one diagonal tetrad port (P_DIA), one diagonal tile port (P_DIA2), and one local port. P_DIA and P_DIA2 are the two instances of the single conceptual diagonal axis. Every input port keeps two virtual channels, so the buffering resource is 9 ports × 2 VCs. Each VC is a physically separate buffer partition with its own single-port readout; the two partitions on one port share only the incoming flit stream up to the VC demux.

Input buffer organization

| Resource | Count per cell | Notes |
| --- | --- | --- |
| Input ports | 9 | 6 axis + P_DIA + P_DIA2 + 1 local |
| VCs per port | 2 | VC0 = escape, VC1 = adaptive/QoS alternate |
| Buffer partitions | 18 | one single-port readout array per VC |
| Read ports consumed | 9 | one per input port, multiplexing its two VCs into the pipeline |

The single-port-per-input choice is deliberate. It is cheaper than a fully-ported VC-per-input arrangement, and it matches the expected throughput because only one flit per input port advances in a given cycle. The real cost is area, not bandwidth: Proteo reported that FIFO flops dominated a NoC node at 81% of area [N8: §4.5], so the generator exposes a regs-vs-SRAM buffer knob.

## 18.2	Why Per-VC Reservation Is Required

A shared input buffer that any VC can draw from looks efficient until adaptive routing starts bouncing packets. Here is the part that bites: a packet can hold the last free slot in a shared buffer while waiting for a credit that is held by another packet, which in turn is waiting for a slot in the same buffer. That cycle is a deadlock, and it is exactly why the literature insists on per-VC buffer reservation for VC-based deadlock avoidance. Duato’s sufficient condition for deadlock freedom assumes that a packet queued on one channel cannot block resources belonging to another channel [D2][D7]; Dally & Towles restate the same requirement for per-VC input buffer organization [P18: §17.1]. Partitioning the buffer keeps the channel-dependency graph clean and makes the escape-channel proof hold.

Design note: A shared read port is fine; a shared storage pool is not. The demux that writes a flit into VC0 or VC1 must be deterministic from the flit’s VC field; do not let a full VC0 spill into VC1 space, or the deadlock analysis collapses.

## 18.3	Credit-Based Flow Control

Every (output port, VC) pair maintains a credit counter that tracks the number of free buffer slots at the downstream input VC. A flit may leave this cell only if the corresponding credit is non-zero. When the downstream cell consumes the flit, it returns a credit. The counter depth must cover the round-trip latency so the channel can run at full duty factor:

F · Lf / tcrt ≥ b

where F is the flow-control frequency, Lf is the forward channel latency, tcrt is the credit-return latency, and b is the minimum number of credits needed to keep the pipe full. If the inequality fails, the link will go idle waiting for credits even though both ends have data [P10][P15].

Credit counter sizing

| Counter | Location | Width driver |
| --- | --- | --- |
| Downstream credit | Per (output port, VC) | round-trip latency × flits per cycle |
| Consumed credit | Per (output port, VC) | same as credit counter |

## 18.4	Failure Mode: Ready Drops Mid-Burst

So what happens when TREADY drops mid-burst on an output? The flit stays in the upstream buffer, the credit is not returned, and the upstream pipeline stalls for that VC only. Because each VC has its own credit counter, a backpressured VC1 path does not block VC0 escape traffic. That isolation is the whole point of splitting the buffers.

## 18.5	References

- Duato, Yalamanchili, Ni, Interconnection Networks: An Engineering Approach, 2nd ed., Morgan Kaufmann, 2003 — channel-dependency theory and VC-based deadlock avoidance [D2][D7].

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann, 2004 — per-VC input buffer reservation and credit sizing [P10][P15][P18].

- Jantsch & Tenhunen (eds.), Networks on Chip, Kluwer, 2003 — Proteo node area and FIFO cost data [N8: §4.5].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Pipeline Stages

IMPLEMENTED: This chapter describes the delta_mesh_cell pipeline as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

The cell pipeline is organized as five stages. Each stage is a valid/ready skid so a downstream stall does not corrupt state.

### 19.0.1	Figure 6.1: delta_mesh_cell internal data path

### 19.0.2	Figure 6.1: delta_mesh_cell internal data path

Cell pipeline stages

| Stage | Function | Cycle-critical notes |
| --- | --- | --- |
| Buffer write | Demux arriving flit into VC0 or VC1 partition | Only head flit carries the route header |
| Route compute | Table lookup or source-route decode; produce (output port, VC) | Lookahead pre-computes the next hop’s output port |
| VC allocation | Pick an available downstream VC for the chosen output | Separable per-output matrix arbiters |
| Switch allocation | Pick a crossbar input for each output port | iSLIP or round-robin; one-hot grant |
| Crossbar traversal | Mux the winning flit onto the output port | Combinational data path, registered valid |

## 19.1	Route Lookahead

The steady-state target is one cycle per hop. That only works if the route decision for hop n+1 is computed while the flit is still traversing hop n. The head flit carries enough sideband in TUSER to let the next cell determine its own output port without an extra lookup cycle [P17]. Body flits follow the same path until TLAST releases the switch grant.

Measured latency on the ring and tetrad suites is 4 cycles/hop today (issue #120). The 1 cycle/hop steady-state target is deferred; the current pipeline pays the full table-lookup and grant latency per hop. Source-route mode can bypass the table lookup for fixed-latency systolic schedules.

## 19.2	Speculative VC and Switch Allocation

The generator can enable speculative allocation: a head flit requests both a VC and a switch slot before the route is fully known, overlapping the allocators with the lookup. If the route guess is wrong, the grant is discarded and the flit retries the next cycle. The steady-state benefit is largest under light load; under saturation, mis-speculation wastes allocator cycles [P17]. The flag is per-build (--speculation), not dynamic.

## 19.3	Handshake Discipline

A granted input holds its output until the packet’s TLAST handshake completes, identical in spirit to the flat crossbar’s packet-atomic arbitration. The switch allocator releases the crossbar input only after TLAST && TVALID && TREADY. Body beats advance on every ready cycle; if TREADY falls mid-burst, the beat waits in the output stage and the grant remains held.

## 19.4	Failure Mode: TLAST Arrives Without a Matching Grant

That should never happen. If it does, the cell is in an inconsistent state: the flit has no output port and will be dropped. The verification suite includes a formal assertion that TLAST may only be asserted while a switch grant is active for the same input, mirroring the flat-mode packet-atomicity check.

## 19.5	References

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann, 2004 — router speculation and lookahead [P17].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Routing Table

IMPLEMENTED: This chapter describes the delta_mesh_cell routing table as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

## 20.1	RAM Table Structure

Each cell contains a RAM lookup table. The key is the packet’s destination coordinate plus its QoS class; the entry gives the output port mask and the VC assignment. The mask supports multicast on VC0 and unicast on either VC.

Routing table entry layout

| Field | Width | Purpose |
| --- | --- | --- |
| Key: dest_x, dest_y, dest_z | 3 × COORD_W | Destination coordinate, sized by fabric dimensions |
| Key: QoS class | 2 | 0=config, 1=latency, 2=bulk |
| Entry: output port mask | 9 bits (one per output port) | Next-hop direction(s); one-hot for unicast, multi-hot for multicast |
| Entry: VC | 1 | 0 or 1, assigned by the table |

## 20.2	Reset Image

At reset the table is loaded with a dimension-ordered route toward the rapids boundary. The route is minimal, connected, and acyclic by construction, which satisfies Duato’s sufficient condition for the escape VC [D5][P11]. This bootstrap path is the only way to load custom tables in-band; every other mode image is loaded later through the config port.

## 20.3	In-Band Load and Readback

Configuration packets target a reserved address. The ejection block intercepts them instead of delivering them to the local port. The command format is {op, entry_index, entry_data}; reads return response packets to the source for load-verify. Config traffic rides QoS class 0 on the reset-default route. A boundary cell or rapids DMA injects the stream.

Design note: The generator can also emit preloaded ROM images for the standard fabric modes. In that build the cell still uses the same table RAMs; the image is just initialized differently.

## 20.4	VC0 Acyclicity Check

VC0 is the escape channel. Its table entries must form a connected, acyclic subrelation in the extended channel-dependency graph; that is Duato’s sufficient condition for deadlock freedom [D2][D7]. This check is performed offline on a table image before it is loaded, not by the hardware at runtime. The verification target is the table-load flow: any image that fails the acyclicity test is rejected before it reaches the fabric.

## 20.5	Torus Wrap and the Dateline Rule

When a dimension is configured as a torus (--torus=x,y,z), the wrap link creates a cycle that plain dimension-order routing cannot break. The fix is a dateline: one designated wrap channel per dimension participates in VC0 only as part of the escape subrelation; ordinary wrap traffic uses VC1 and may consume a misroute budget [P11]. The hardware enforces the rule by gating wrap-link VC0 grants with the dateline condition.

## 20.6	Standard Fabric Modes

Alongside custom per-cell tables, the generator defines ten standard modes switched by a broadcast mode-switch message. Each cell holds generator-emitted images; the switch applies only after a fabric-wide ack barrier returns through the config read-response path.

Standard fabric mode catalog

| Mode | Pattern | Routing source |
| --- | --- | --- |
| 1 | Mesh (default) | RAM tables, reset DOR image |
| 2 | Systolic-1D | Fixed source-route paths |
| 3 | Wavefront-2D | Per-diagonal static tables |
| 4 | Stencil-3D | Nearest-neighbor tables + torus wrap |
| 5 | Broadcast | Multicast VC0 port masks to all |
| 6 | Convergecast/reduce | Reverse-broadcast tables |
| 7 | All-to-all/transpose | Per-stage table images |
| 8 | Pipeline + rapids-stream | Fixed chain + boundary tables |
| 9 | Tetrad all-to-all (K4) | P_DIA for diagonal mates; DOR for inter-tetrad dsts |
| 10 | Tile all-to-all (K4 of tetrads) | P_DIA2 for diagonal-tetrad mates; DOR for inter-tile dsts |

## 20.7	References

- Duato, Yalamanchili, Ni, Interconnection Networks: An Engineering Approach, 2nd ed., Morgan Kaufmann, 2003 — deterministic routing, Duato’s protocol, and VC0 acyclicity [D2][D5][D7].

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann, 2004 — table-based routing, deadlock avoidance, and the dateline technique [P8][P11][P12].

- Franklin et al. (eds.), Network Processor Design: Issues and Practices, Vol. 1, Morgan Kaufmann, 2002 — in-band control-program precedent [Q6].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Allocators

IMPLEMENTED: This chapter describes the delta_mesh_cell allocators as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

The cell reuses the house arbiters rather than inventing new allocation hardware. The two allocator stages are VC allocation and switch allocation.

Allocator mapping

| Allocator | House block | Role |
| --- | --- | --- |
| VC allocation | arbiter_matrix | For each output port, pick one requesting input VC that has a free downstream VC |
| Switch allocation | Round-robin or iSLIP-style | For each output port, pick one input flit that already won a VC |

## 21.1	VC Allocation

VC allocation is a many-to-one pick per output port: several input VCs request the same downstream VC, and the arbiter grants at most one. The arbiter_matrix is Dally & Towles’ recommended choice for small input counts [P19: §18.5]. It keeps state in a priority matrix, provides fairness, and is small enough to place one per output port without exploding area. The grant is one-hot per output.

Design note: A separable allocator would allocate VC and switch in one combined stage. That saves a cycle but at the cost of more hardware and lower saturation throughput. The two-stage separable approach is the conservative starting point; combined allocation is deferred until measurements justify it.

## 21.2	Switch Allocation

Switch allocation picks which input port drives each output port. The baseline is a round-robin arbiter per output. An iSLIP-style allocator is selectable at generation; iSLIP’s iterative accept/reject loop reaches 100% throughput under uniform traffic, while a one-pass separable allocator saturates around 66% [P19: §19.3, §19.7]. The choice is a throughput-vs-area knob.

## 21.3	One-Hot Grant Assertions

Every allocator grant is asserted to be one-hot (or zero-hot). Under ifdef FORMAL, a $onehot0 assertion checks each output’s grant vector. This is the same discipline as the flat crossbar: a multi-hot grant means two inputs would collide on the same output.

## 21.4	Multicast Allocation

Multicast entries use a multi-hot output port mask. The cell does not allocate all outputs at once; instead it incrementally serves whichever masked outputs are free each cycle and releases the input only after all requested outputs have been served [P18: §17.2][P19: exercises]. Until then the head flit stays at the front of its input VC and the body flits behind it cannot advance. That serialization is the price of source-side replication within one cell.

## 21.5	Conservative Output-VC Reallocation

When a downstream VC drains, the cell waits for the returning credit before assigning that VC to a new packet. Conservative reallocation prevents a subtle failure: if a VC were reassigned before the credit returned, the new packet could establish a dependency that the original deadlock analysis did not cover [P16]. Waiting one round trip is cheap insurance.

## 21.6	References

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann, 2004 — matrix arbiters, separable and iSLIP allocators, and multicast allocation [P16][P18][P19].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	QoS Mechanisms

IMPLEMENTED: This chapter describes the delta_mesh_cell QoS mechanisms as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Delta defines three QoS classes with strict priority at every output port. The split follows the control/data-plane separation common in NPU practice: class 0 for config, class 1 for latency traffic, class 2 for bulk [Q4][Q5].

QoS class mapping

| Class | Name | Priority | Typical use |
| --- | --- | --- | --- |
| 0 | Config / control | Highest | In-band table loads, mode switches, rapids control |
| 1 | Latency | Middle | Compute traffic with deadline requirements |
| 2 | Bulk | Lowest | Background data movement |

## 22.1	Strict Priority and Programmable Aging

The arbiter uses strict priority by default: a class-0 request always wins over class 1 and 2. To prevent indefinite starvation of lower classes, each class has a programmable aging counter. After the counter expires, the effective priority inverts so the lower class gets service [D13]. The counter is per-output-port and reloads automatically.

## 22.2	Bulk Minimum Reservation

Aging alone is not enough. Under sustained latency traffic, bulk could still be starved because every fresh class-1 request would preempt the aged class-2 grant. The cell therefore maintains a bulk minimum reservation counter: after a programmable number of consecutive higher-priority grants, the next grant is forced to class 2 regardless of pending higher-priority requests [D1][Q5]. This guarantees a floor of bulk bandwidth without relying on traffic patterns.

## 22.3	Class Non-Interference

Priority bits are insufficient when traffic classes share buffers. Tree-saturation in one class can block unrelated traffic through a shared input buffer even if that traffic has a priority bit [P14: §15.5]. Delta avoids this by giving each class its own VC and its own injection buffer at the local port. Config traffic rides a dedicated slice of VC0 so control can never be bulk-blocked.

Class isolation resources

| Isolation mechanism | Where applied |
| --- | --- |
| Separate VCs per class | Every input port |
| Separate injection buffers per class | Local port only |
| Independent table region per class | Per-destination route diversity |

## 22.4	Failure Mode: Aging Inversion Misconfiguration

If the aging counter is set too large, class 2 starves. If it is set too small, class 1 loses its latency advantage. The verification suite sweeps the counter and checks that bulk gets at least its reserved fraction under a tree-saturation attack.

## 22.5	References

- Duato, Yalamanchili, Ni, Interconnection Networks: An Engineering Approach, 2nd ed., Morgan Kaufmann, 2003 — starvation under priority and bulk reservation need [D1][D13].

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann, 2004 — separation of resources to avoid tree saturation [P14].

- Franklin et al. (eds.), Network Processor Design: Issues and Practices, Vol. 1, Morgan Kaufmann, 2002 — NPU QoS practice and ASPEN scheduler starvation prevention [Q4][Q5].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Multicast Path

IMPLEMENTED: This chapter describes the delta_mesh_cell multicast path as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

Multicast is restricted to VC0. The routing table supplies a multi-hot output port mask, and the cell builds a BRCP-conformed path: every branch follows the base unicast routing function so multicast adds no new channel dependencies beyond the unicast proof [D9]. The implementation floods a multicast packet along a BFS tree from the source, replicating at branching cells.

## 23.1	Source-Side Replication

When a head flit reaches a cell whose table entry says “send out ports A and B,” the cell creates one copy per destination output. This is source-side replication at the branching cell, not a broadcast-and-filter scheme. It costs buffer slots at the branch but avoids wasting downstream bandwidth on flits that would be dropped [Q7].

## 23.2	Delivery-VC Assignment

Multicast reception uses multiple delivery VCs at each destination cell. The literature is clear: even with correct routing, a single delivery channel for multicast creates a deadlock that the unicast VC analysis does not cover [D9: §5.5.3]. The cell assigns a distinct delivery VC per multicast group or per source, depending on the build-time parameterization.

## 23.3	The No-Responses Rule

Multicast connections do not carry responses. If every destination tried to acknowledge delivery, the acknowledgments would converge on the source and create a many-to-one merge problem; worse, a response could wait for a multicast channel that is held by another response, closing a dependency cycle [N3]. Many-to-one communication uses unicast, not multicast.

Multicast discipline

| Rule | Rationale |
| --- | --- |
| VC0 only | Keeps multicast on the escape channel; avoids adaptive-route cycles |
| BRCP paths | Adds no channel dependencies beyond unicast proof |
| Source-side replication | No wasted downstream bandwidth |
| Multiple delivery VCs | Breaks delivery-channel deadlock |
| No responses | Avoids response merge and cyclic waits |

## 23.4	Failure Mode: Tree-Based Multicast in Wormhole Networks

A naive tree multicast that forks at every hop in a wormhole network deadlocks readily because forked branches can hold buffer resources in a cycle [D9: §5.5.2]. The BRCP + source replication + delivery-VC combination is the book-recommended discipline for this reason.

## 23.5	References

- Duato, Yalamanchili, Ni, Interconnection Networks: An Engineering Approach, 2nd ed., Morgan Kaufmann, 2003 — hardware multicast deadlock, BRCP, and delivery-channel VCs [D9].

- Jantsch & Tenhunen (eds.), Networks on Chip, Kluwer, 2003 — iEthereal connection model: multicast with no responses [N3].

- Franklin et al. (eds.), Network Processor Design: Issues and Practices, Vol. 1, Morgan Kaufmann, 2002 — source-side replication precedent [Q7].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Observability Counters and DFT

IMPLEMENTED: This chapter describes the delta_mesh_cell counter bank and DFT mode as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

## 24.1	Ocelus Counter Bank

The cell exposes eleven 32-bit saturating counters through the config-read path. The counter bank is selected by setting table_index[15] = 1; indices 0..10 are defined and indices above 10 read back 0. Counters are frozen when ENABLE_COUNTERS == 0 and clear on aresetn.

Ocelus counter bank indices.

| Index | Name | What it counts |
| --- | --- | --- |
| 0 | C_FLITS_IN | Flits received by the cell |
| 1 | C_FLITS_OUT | Flits transmitted by the cell |
| 2 | C_PKTS_IN | Packets received (TLAST handshakes) |
| 3 | C_PKTS_OUT | Packets transmitted (TLAST handshakes) |
| 4 | C_STALL_CYCLES | Output stall cycles |
| 5 | C_CREDIT_STALL | Cycles a granted output waited for downstream credit |
| 6 | C_ERR_DROP | Malformed-destination drops (mirrors o_err_drop_cnt) |
| 7 | C_MODE_ACKS | Broadcast mode-switch acks received (mirrors o_mode_acks) |
| 8 | C_BULK_STARVE | Cycles bulk reservation was due |
| 9 | C_SPEC_COMMITS | Speculative grants that committed |
| 10 | C_SPEC_REVOKES | Speculative grants that were revoked |

The suite is ahead of the NoC textbook literature, which says little about hardware observability, and is instead modeled on NPU practice [Q4][Q2][Q8].

## 24.2	DFT Test Mode

DFT mode reconfigures each cell into one of three deterministic routing patterns so the mesh wiring behaves as a concatenated bus. Stuck-at and bridge faults become testable with approximately ceil(log2(n)) patterns, where n is the number of cells in the fabric [N6]. The mode is entered through a DFT escape write on the config port (table_index[15:12] = 4'hD, payload bit 0 enables DFT, bits 2:1 select the pattern); each cell drives an o_dft_active output.

DFT routing patterns

| Pattern | Routing behavior | Fault target |
| --- | --- | --- |
| Pattern 0 | All cells route +X | Horizontal wiring |
| Pattern 1 | All cells route +Y | Vertical wiring |
| Pattern 2 | All cells route +Z | Depth wiring |

Design note: DFT mode is mutually exclusive with normal traffic. The cell gates non-test flits while test_active is asserted; returning to normal mode requires a fabric-wide mode-switch sequence.

## 24.3	References

- Jantsch & Tenhunen (eds.), Networks on Chip, Kluwer, 2003 — testing the communication infrastructure with three switch configurations [N6].

- Franklin et al. (eds.), Network Processor Design: Issues and Practices, Vol. 1, Morgan Kaufmann, 2002 — IXP2400 PMU, Toaster per-queue stats, Agere ASI RMON counters [Q4][Q2][Q8].

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Verification Targets

IMPLEMENTED: This chapter describes the delta_mesh_cell verification suites as built. The design is per the 2026-10-13 design spec (docs/superpowers/specs/2026-10-13-delta-mesh-design.md).

## 25.1	cocotb Suites

The mesh verification plan is ten cocotb suites, each driven by a different generator configuration.

Mesh cocotb verification suites

| Suite | Focus | Pass criteria |
| --- | --- | --- |
| Cell routing | Reset DOR, programmed tables, config load + readback | Correct output port per table; readback matches write |
| Fabric 2x2 | All-pairs delivery across multi-hop paths | Every packet delivers exactly once |
| Flow control | Credit exhaustion, atomicity, steady-state throughput | No loss, no interleaving, sustained 1 flit/clk |
| Tables | Malformed-dst drop, cross-VC drop drain, table reroute | Drop counted once; drain stays on its own VC |
| Modes | Broadcast mode-switch, ack barrier, mode images 1/2/3/5/6 | Barrier completes; traffic routes per selected image |
| Tetrad | Mode 9 K4 all-to-all, P_DIA one-hop latency | All 12 ordered pairs deliver in 1 hop |
| Tile | Mode 10 K4-of-tetrads, P_DIA2 one-hop latency | Diagonal-tetrad pairs deliver in 1 hop |
| QoS | Strict priority, aging inversion, bulk reservation | Class order under saturation; bulk minimum bandwidth |
| Multicast | Fanout correctness, no-response rule, deadlock soak | All destinations receive exactly one copy; 10^6-flit soak green |
| Counters | Counter bank readback, DFT escape | Readback matches scripted traffic; DFT pattern switches |
| Systolic | Torus ring, dateline, source-route, speculation | Ring latency measured; dateline forces VC0; SR bypasses table drop; speculation parity |
| Perf model calibration | Tornado pattern at three loads vs bin/delta_mesh_perf.py | Zero-load T0 within 20% of model; loaded deltas recorded |

## 25.2	Formal Assists

Three formal checks assist the cocotb suites:

- VC0 acyclicity checker. Run offline on table images before load; rejects any VC0 subrelation that violates Duato’s sufficient condition [D2].

- One-hot grant assertions. Per-output switch and VC grants must be one-hot or zero-hot, identical to the flat-mode assertion discipline.

- TLAST-before-grant-release packet atomicity. A grant may not be released before the TLAST handshake completes, preventing packet interleaving on an output.

## 25.3	Performance Model Calibration

The Python performance model (bin/delta_mesh_perf.py) predicts zero-load latency and saturation throughput for uniform, nearest-neighbor, tornado, bit-complement, and transpose patterns. The perf-model suite compares measured results against the model and reports deltas; the model itself is verified by the same methodology used for network-processor benchmarks [P20][Q1].

## 25.4	References

- Duato, Yalamanchili, Ni, Interconnection Networks: An Engineering Approach, 2nd ed., Morgan Kaufmann, 2003 — acyclicity condition for deadlock-free escape VC [D2].

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann, 2004 — performance analysis methodology [P20].

- Franklin et al. (eds.), Network Processor Design: Issues and Practices, Vol. 1, Morgan Kaufmann, 2002 — hardware-level benchmarking practice [Q1].