RTL Design Sherpa

Lattice Hardware Architecture Specification 1.2

October 13, 2026

Table of Contents

Lattice Has Index	15

Lattice Hardware Architecture Specification Index	16

Document Organization	17

Front Matter	17

Chapter 1: Introduction	17

Chapter 2: System Overview	17

Chapter 3: Architecture	17

Chapter 4: Interfaces	17

Chapter 5: Performance	17

Chapter 6: Integration	17

Related Documentation	19

Document Information	20

Lattice Hardware Architecture Specification	20

Document Purpose	21

References	22

Terminology	23

Revision History	24

Purpose and Scope	25

Document Purpose	25

Component Overview	26

Document Scope	27

In Scope	27

Out of Scope	27

Specification Level	28

Document Conventions	29

Notation	29

Signal Naming	29

Parameter Naming	29

Diagrams	30

Block Diagrams	30

Address Notation	30

Table Captions	31

Requirement References	32

Definitions and Acronyms	33

Acronyms	33

Terms	34

APB Protocol Signals	35

Use Cases	36

Primary Use Cases	36

UC-1: Simple Peripheral Interconnect (1x4)	36

UC-2: Multi-Master System (2x4)	36

UC-3: Protocol Conversion (1x1)	36

Variant Selection Guide	37

Key Features	38

Feature Summary	38

F1: Parametric MxN Configuration	38

F2: Automatic Address-Based Routing	38

F3: Round-Robin Arbitration	38

F4: Back-to-Back Transactions	38

F5: Proven Building Blocks	39

Feature Comparison	40

Design Philosophy	41

Composition Over Complexity	41

Scalability	41

System Context	42

System Context Diagram	42

Figure 2.1: System Context Diagram	42

Typical Integration Topology	43

Master-Side Connections	43

Slave-Side Connections	43

Interface Boundaries	44

Input Boundary (Master-Side)	44

Output Boundary (Slave-Side)	44

Clock and Reset	44

System Integration Considerations	45

Address Map Planning	45

Multi-Master Considerations	45

Block Diagram	46

Top-Level Architecture	46

Figure 3.1: Lattice Block Diagram	46

Functional Blocks	47

Master-Side Protocol Conversion	47

Internal Crossbar Fabric	47

Slave-Side Protocol Conversion	47

Module Hierarchy	48

Data Path Width	49

Data Flow	50

Transaction Flow Overview	50

Figure 3.2: Transaction Data Flow	50

Write Transaction Flow	51

Stage 1: Master Request	51

Stage 2: Protocol Conversion (Master-Side)	51

Stage 3: Address Decode	51

Stage 4: Arbitration (if needed)	51

Stage 5: Protocol Conversion (Slave-Side)	51

Stage 6: Response Return	51

Read Transaction Flow	52

Timing Summary	53

Back-to-Back Transactions	54

Address Mapping	55

Address Map Structure	55

Figure 3.3: Address Mapping Structure	55

Default Address Map	56

Address Decode Logic	57

Example Decode	57

Customizing Base Address	58

Address Map Constraints	59

Fixed Slave Region Size	59

Maximum Configuration	59

Address Width Requirement	59

Master-Side APB Interfaces	60

Interface Overview	60

Signal Definition	61

APB5 Sideband (mixed-version variants only)	61

Signal Descriptions	63

PSEL (Input)	63

PENABLE (Input)	63

PADDR (Input)	63

PWRITE (Input)	63

PWDATA (Input)	63

PSTRB (Input)	63

PPROT (Input)	63

PRDATA (Output)	63

PSLVERR (Output)	63

PREADY (Output)	64

Transaction Timing	65

Uncontended Write (No Arbitration Wait)	65

Figure 4.1: Uncontended Write at a Master Port	65

Contended Transaction (Arbitration Wait)	65

Figure 4.2: Contended Transfer	66

Slave-Side APB Interfaces	67

Interface Overview	67

Signal Definition	68

APB5 Sideband (mixed-version variants only)	68

Signal Behavior	70

Output Signals (Crossbar to Slave)	70

Input Signals (Slave to Crossbar)	70

Slave Response Handling	71

Wait States	71

Figure 4.3: Slave Wait States	71

Error Response	71

Multi-Slave Considerations	72

Address Overlap Prevention	72

Independent Operation	72

Clock and Reset	73

Clock Interface	73

Signal Definition	73

Clock Requirements	73

Reset Interface	74

Signal Definition	74

Reset Behavior	74

Reset Safe Values	74

Clock-Reset Relationship	75

Reset Assertion	75

Reset Deassertion	75

Figure 4.4: Reset Sequence	75

Integration Notes	76

Clock Domain Crossing	76

Clock Gating	76

Throughput Characteristics	77

Maximum Throughput	77

Single Master	77

Multi-Master (Uncontended)	77

Multi-Master (Contended)	77

Arbitration Handoff	79

Grant Persistence	79

Back-to-Back Timing	79

Throughput Factors	80

Factors That Reduce Throughput	80

Optimal Usage Pattern	80

Latency Analysis	81

Transaction Latency	81

Uncontended Access	81

Contended Access	82

Latency Breakdown	84

Forward Path (Master to Slave)	84

Response Path (Slave to Master)	85

Latency Timing Diagram	86

Best Case (No Waits, No Contention)	86

Figure 5.1: Best-Case Latency Timeline	86

Worst Case (Contention + Slave Wait States)	86

Figure 5.2: Arbitration and Slave Wait States	86

Latency Optimization	87

Design Recommendations	87

Resource Estimates	88

FPGA Resource Usage	88

Pre-Generated Variants	88

Scaling Factors	88

Lines of Code	90

Power Considerations	91

Static Power	91

Dynamic Power	91

Power Optimization	91

Area Comparison	92

Custom Configuration Estimation	93

System Requirements	94

Hardware Requirements	94

Clock and Reset	94

APB Compliance	94

Software Requirements	95

Address Map Configuration	95

Driver Considerations	95

Constraints	96

Address Map Constraints	96

Transaction Constraints	96

Integration Checklist	97

Pre-Integration	97

Post-Integration	97

Parameter Configuration	98

Module Parameters	98

Core Parameters	98

Derived Parameters	99

Parameter Usage Examples	100

Default Configuration	100

Custom Base Address	100

64-bit Data Width	100

Wide Address	100

Parameter Validation	101

Address Width Check	101

Data Width Constraints	101

Custom Generation Parameters	102

Verification Strategy	103

Pre-Verified Component	103

Test Categories	104

Basic Connectivity	104

Address Decode Verification	104

Arbitration Testing	104

Stress Testing	104

APB5 Sideband Gating (mixed variant)	104

Timing and Protocol Compliance	104

Integration Testing Recommendations	105

Level 1: Basic Integration	105

Level 2: Functional Integration	105

Level 3: System Integration	105

Verification Collateral	106

Available Test Infrastructure	106

Running Verification	106

Known Limitations	107

Not Tested	107

Related Documentation	108

List of Figures

Figure 2.1: System Context Diagram	42

Figure 3.1: Lattice Block Diagram	46

Figure 3.2: Transaction Data Flow	50

Figure 3.3: Address Mapping Structure	55

Figure 4.1: Uncontended Write at a Master Port	65

Figure 4.2: Contended Transfer	66

Figure 4.3: Slave Wait States	71

Figure 4.4: Reset Sequence	75

Figure 5.1: Best-Case Latency Timeline	86

Figure 5.2: Arbitration and Slave Wait States	86

List of Tables

Table 1: Reference Documents	22

Table 2: Terminology and Definitions	23

Table 3: Document Revision History	24

Table 4: HAS vs MAS Coverage Comparison	28

Table 5: Signal Naming Conventions	29

Table 6: Parameter Naming Conventions	29

Table 7: Acronym Definitions	33

Table 8: Term Definitions	34

Table 9: APB Protocol Signal Definitions	35

Table 10: Variant Selection Guide	37

Table 11: Pre-Generated Variant Comparison	40

Table 12: Typical Master Connections	43

Table 13: Typical Slave Connections (1to4 example)	43

Table 14: Data Path Width Parameters	49

Table 15: Transaction Timing Summary	53

Table 16: Default Address Map (4-slave example)	56

Table 17: Address Decode Example	57

Table 18: Master Port Signal Definition	61

Table 19: APB5 Master Sideband Signals	61

Table 20: Slave Port Signal Definition	68

Table 21: APB5 Slave Sideband Signals	68

Table 22: Clock Signal	73

Table 23: Reset Signal	74

Table 24: Reset Safe Values	74

Table 25: Single-Master-Active Throughput, by Variant Class	77

Table 26: Contended Access Throughput	78

Table 27: Throughput Reduction Factors	80

Table 28: Uncontended Transaction Latency	81

Table 29: Measured Timing, Always-Ready Slave	82

Table 30: Arbitration Latency	82

Table 31: Forward Path Latency	84

Table 32: Response Path Latency	85

Table 33: Latency Optimization Recommendations	87

Table 34: FPGA Resource Estimates (32-bit)	88

Table 35: Resource Scaling Factors	88

Table 36: Source Code Metrics	90

Table 37: Power Optimization Techniques	91

Table 38: Relative Area Comparison	92

Table 39: Clock and Reset Requirements	94

Table 40: APB Compliance Requirements	94

Table 41: Driver Considerations	95

Table 42: Address Map Constraints	96

Table 43: Transaction Constraints	96

Table 44: Core Parameter Definition	98

Table 45: Derived Parameters	99

Table 46: Valid Data Width Configurations	101

Table 47: Generator Command Line Options	102

Table 48: Pre-Verification Status	103

Table 49: Level 1 Integration Tests	105

Table 50: Level 2 Integration Tests	105

Table 51: Level 3 Integration Tests	105

Table 52: Test Infrastructure	106

Table 53: Known Limitations	107

# Lattice Has Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Lattice Hardware Architecture Specification Index

Version: 1.3 Date: 2026-10-20 Purpose: High-level hardware architecture specification for Lattice component

## Document Organization

Note: All chapters are linked below; this index doubles as the manifest for automated document generation.

### Front Matter

- Document Information

### Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### Chapter 3: Architecture

- Block Diagram

- Data Flow

- Address Mapping

### Chapter 4: Interfaces

- Master-Side APB Interfaces

- Slave-Side APB Interfaces

- Clock and Reset

### Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resource Estimates

### Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

## Related Documentation

- Lattice MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Document Information

## Lattice Hardware Architecture Specification

Document Number: LATTICE-HAS-001 Version: 1.3 Status: Production Ready Classification: Open Source - MIT License (matches the SPDX tags on every RTL file)

## Document Purpose

This Hardware Architecture Specification (HAS) is the high-level architectural view of the Lattice: system-level design, external interfaces, performance characteristics, and integration requirements. Internal implementation specifics are deliberately left to the MAS.

Target Audience: - System architects evaluating Lattice for SoC integration - Hardware engineers planning peripheral interconnect design - Software engineers developing drivers and firmware - Verification engineers planning system-level testing

Companion Documents: - Lattice Micro-Architecture Specification (MAS) - Detailed block-level implementation - Lattice Product Requirements Document (PRD) - Requirements and rationale

## References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | Lattice MAS v1.2 | Micro-Architecture Specification |
| [REF-2] | Lattice PRD | Product Requirements Document |
| [REF-3] | ARM AMBA APB | APB Protocol Specification (IHI 0024C) |

## Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| APB | Advanced Peripheral Bus - ARM AMBA low-power peripheral bus |
| Arbiter | Logic that selects between competing requests using round-robin |
| Crossbar | NxM interconnect fabric connecting multiple masters to multiple slaves |
| Master | APB initiator device that initiates read/write transactions |
| PSEL | APB peripheral select signal indicating slave selection |
| PENABLE | APB enable signal indicating data phase of transaction |
| PREADY | APB ready signal indicating slave response completion |
| Slave | APB target device that responds to master transactions |
| Transaction | Complete APB read or write operation (setup + data phase) |

## Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | seang | Initial HAS release |
| 1.1 | 2026-10-12 | seang | Refresh: update LOC counts, verification status, and generator CLI options |
| 1.2 | 2026-10-13 | seang | Rebrand: component codename lattice, directory lattice-apbx-xbar; book titles and document number (LATTICE-HAS-001) updated; RTL module names unchanged (apbx_xbar_*) |
| 1.3 | 2026-10-20 | seang | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |

Last Updated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Purpose and Scope

## Document Purpose

This Hardware Architecture Specification (HAS) describes the Lattice at the system integration level, with enough detail for:

- System architects to evaluate the component for SoC integration

- Hardware engineers to plan interconnect topology and address mapping

- Software engineers to understand register access paths and address spaces

- Verification engineers to develop system-level test plans

## Component Overview

The Lattice is a parametric MxN interconnect fabric connecting multiple APB masters to multiple APB slaves:

- Automatic address-based routing to the appropriate slave

- Fair round-robin arbitration when multiple masters access the same slave

- Back-to-back transactions with no master-side idle cycles (they do not overlap inside the fabric – 10 pclk cycles each PREADY-to-PREADY with one master, 11 on an arbitrated variant, where the round-robin grant is registered; see 5.1)

- Scalable configuration from 1x1 to 16x16

## Document Scope

### In Scope

- System-level architecture and block organization

- External interface specifications (all APB signals)

- Address mapping and routing strategy

- Arbitration policy and fairness guarantees

- Performance characteristics (latency, throughput)

- Integration requirements and parameters

### Out of Scope

- Internal RTL implementation details (see MAS)

- Detailed timing diagrams for internal signals

- Synthesis and implementation guidance

- Test infrastructure details

## Specification Level

This HAS is a black-box view of the Lattice:

HAS vs MAS Coverage Comparison

| Aspect | HAS Coverage | MAS Coverage |
| --- | --- | --- |
| External ports | Complete | Complete |
| Functional behavior | Complete | Complete |
| Internal architecture | Overview only | Detailed |
| FSM states | - | Complete |
| Timing paths | Interface only | All paths |
| RTL structure | - | Complete |

Next: Document Conventions

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Document Conventions

## Notation

### Signal Naming

Signal Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| m<i>_apb_* | Master-side APB port i – BOTH directions: requests in (PSEL, PADDR, …), responses out (PRDATA, PREADY, PSLVERR) | m0_apb_PSEL (in), m0_apb_PREADY (out) |
| s<j>_apb_* | Slave-side APB port j – BOTH directions: requests out, responses in | s2_apb_PSEL (out), s2_apb_PREADY (in) |
| P* | APB protocol signal prefix | PADDR, PWRITE, PRDATA |
| <i>, <j> | Master/slave index placeholder | m0, s3 |

### Parameter Naming

Parameter Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| UPPER_CASE | Module parameters | ADDR_WIDTH, DATA_WIDTH |
| NUM_* | Count parameters | NUM_MASTERS, NUM_SLAVES |
| *_WIDTH | Bit width parameters | DATA_WIDTH = 32 |
| BASE_* | Base address parameters | BASE_ADDR = 0x10000000 |

## Diagrams

### Block Diagrams

Block diagrams here use:

- Rectangles for functional blocks

- Arrows showing data flow direction

- Dashed lines for configuration/control paths

- Labels on connections indicating signal groups

### Address Notation

Addresses appear in hexadecimal with the 0x prefix:

- 0x1000_0000 - Underscores separate 16-bit groups for readability

- [31:0] - Bit range notation (MSB:LSB)

- 64KB - Size notation using standard abbreviations

## Table Captions

Tables are numbered sequentially within each chapter. A colon (:) followed by the caption text appears below each table.

## Requirement References

Requirements from the PRD are referenced as:

- FR-n - Functional Requirement number n

- PR-n - Performance Requirement number n

- IR-n - Interface Requirement number n

Next: Definitions and Acronyms

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Definitions and Acronyms

## Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| CPU | Central Processing Unit |
| DMA | Direct Memory Access |
| FSM | Finite State Machine |
| GPIO | General Purpose Input/Output |
| HAS | Hardware Architecture Specification |
| I2C | Inter-Integrated Circuit |
| LOC | Lines of Code |
| LUT | Lookup Table (FPGA resource) |
| MAS | Micro-Architecture Specification |
| PRD | Product Requirements Document |
| RTL | Register Transfer Level |
| SoC | System on Chip |
| SPI | Serial Peripheral Interface |
| UART | Universal Asynchronous Receiver/Transmitter |

## Terms

Term Definitions

| Term | Definition |
| --- | --- |
| Address Decode | Logic that determines which slave should receive a transaction based on address |
| Arbitration | Process of selecting one request when multiple masters compete for the same slave |
| Back-to-back | Consecutive transactions with no idle cycles between them |
| Crossbar | Interconnect topology allowing any master to communicate with any slave |
| Grant Persistence | Maintaining arbitration grant through entire transaction duration |
| Master | Device that initiates APB transactions (CPU, DMA, etc.) |
| Round-Robin | Arbitration scheme that rotates priority among requesters |
| Slave | Device that responds to APB transactions (peripherals) |
| Starvation | Condition where a requester is indefinitely denied access |
| Transaction | Complete APB read or write operation including setup and data phases |
| Back-to-Back | Consecutive transactions with no master-side idle cycles. Note this crossbar does not OVERLAP them, and cadence is one cycle longer than single-transfer latency: 10 PREADY-to-PREADY against 9 SETUP-to-PREADY with one master, 11 against 10 when arbitrated (see 5.2) |

## APB Protocol Signals

APB Protocol Signal Definitions

| Signal | Direction | Description |
| --- | --- | --- |
| PCLK | Input | APB clock (all signals synchronous to rising edge) |
| PRESETn | Input | Active-low reset. Assertion is asynchronous in every build; deassertion must be synchronised to PCLK (see 4.3) |
| PADDR | Master to Slave | Address bus |
| PSEL | Master to Slave | Peripheral select (one per slave) |
| PENABLE | Master to Slave | Enable signal (high during data phase) |
| PWRITE | Master to Slave | Write/read direction (1=write, 0=read) |
| PWDATA | Master to Slave | Write data bus |
| PSTRB | Master to Slave | Write strobes (byte enables) |
| PPROT | Master to Slave | Protection attributes |
| PRDATA | Slave to Master | Read data bus |
| PREADY | Slave to Master | Ready signal (can extend transaction) |
| PSLVERR | Slave to Master | Slave error response |

Next: Chapter 2: System Overview

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Use Cases

## Primary Use Cases

### UC-1: Simple Peripheral Interconnect (1x4)

Scenario: Single CPU accessing multiple peripherals

A microcontroller reaches UART, GPIO, Timer, and SPI peripherals through a single APB master interface.

Configuration: - 1 Master (CPU) - 4 Slaves (UART, GPIO, Timer, SPI) - Variant: apbx_xbar_1to4

Address Map:

0x1000_0000 - 0x1000_FFFF : UART  (64KB)
0x1001_0000 - 0x1001_FFFF : GPIO  (64KB)
0x1002_0000 - 0x1002_FFFF : Timer (64KB)
0x1003_0000 - 0x1003_FFFF : SPI   (64KB)

### UC-2: Multi-Master System (2x4)

Scenario: CPU and DMA both accessing peripherals

An SoC where both the CPU and a DMA controller share the same peripherals.

Configuration: - 2 Masters (CPU, DMA) - 4 Slaves (peripherals) - Variant: apbx_xbar_2to4

Key Behavior: - Round-robin arbitration when both masters access same peripheral - No master starvation guaranteed - Back-to-back transactions on uncontended access (10 pclk cycles each for M = 1, 11 when arbitrated; they do not overlap – see 5.1)

### UC-3: Protocol Conversion (1x1)

Scenario: APB timing boundary or clock domain preparation

Insert the APB crossbar as a timing stage, or as preparation for clock domain crossing.

Configuration: - 1 Master - 1 Slave - Variant: apbx_xbar_1to1

Benefits: - Isolates master from slave timing - Consistent transaction buffering - Lowest resource footprint of the family

## Variant Selection Guide

Variant Selection Guide

| Use Case | Recommended Variant | Rationale |
| --- | --- | --- |
| Simple peripheral bus | apbx_xbar_1to4 | Most common SoC configuration |
| CPU + DMA system | apbx_xbar_2to4 | Fair access for both masters |
| Timing isolation | apbx_xbar_1to1 | Registered command and response paths on a single pairing |
| Multi-master arbitration | apbx_xbar_2to1 | Focus on arbitration only |
| Custom topology | Generator | Any MxN configuration |

Next: Key Features

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Key Features

## Feature Summary

### F1: Parametric MxN Configuration

- Any combination of M masters and N slaves

- Pre-generated variants for common configurations (1x1, 2x1, 1x4, 2x4, and the mixed-version 2x2)

- Python generator for custom configurations up to 16x16

- A single GENERATOR serves all configurations – not a single RTL source. Each variant is a distinct file with its M×N baked into the module name and port list; only ADDR_WIDTH, DATA_WIDTH, STRB_WIDTH and BASE_ADDR are elaboration parameters (see 6.2). The parameterized-core reading was true of the retired apbx_xbar_thin and has not been true since 2026-08-27.

### F2: Automatic Address-Based Routing

- Parallel address decode for all slaves

- Fixed 64KB address space per slave

- Configurable base address via BASE_ADDR parameter

- Zero-decode-latency routing

Address Calculation:

slave_index = ((PADDR - BASE_ADDR) >> 16)[$clog2(S)-1:0]

### F3: Round-Robin Arbitration

- Independent arbiter per slave

- Fair rotation of master priority

- No master starvation guaranteed

- Grant persistence through transaction completion

### F4: Back-to-Back Transactions

- Accepted with no master-side idle cycles required

- Grant held for the duration of one transaction, released at the response handshake

- They do NOT overlap inside the fabric: apb4_slave is one-command-at-a-time. Sustained cadence measures 10 pclk cycles PREADY-to-PREADY, one more than the 9-cycle SETUP-to-PREADY single transfer, the extra cycle being the next transfer’s mandatory SETUP phase (see 5.1/5.2)

### F5: Proven Building Blocks

- Built from production-tested apb4_slave.sv and apb4_master.sv modules

- No new protocol logic - pure composition

- Each component independently verified

## Feature Comparison

Pre-Generated Variant Comparison

| Feature | apbx_xbar_1to1 | apbx_xbar_2to1 | apbx_xbar_1to4 | apbx_xbar_2to4 | apbx_xbar_2to2_mixed |
| --- | --- | --- | --- | --- | --- |
| Masters | 1 | 2 | 1 | 2 | 2 |
| Slaves | 1 | 1 | 4 | 4 | 2 |
| Arbitration | No | Yes | No | Yes | Yes |
| Address Decode | No | No | Yes | Yes | Yes |
| APB5 sideband | No | No | No | No | Yes – per-port version gating |

All five are generator output. LOC per variant is in HAS ch05_performance/03_resources.md, the single source of truth for it – the row that used to sit in this table drifted stale within two days of being reconciled. apbx_xbar_thin, a hand-written parameterized core that once occupied a sixth column here, was retired and deleted on 2026-08-27 along with its tests, formal harnesses and testplan; nothing in the tree depends on it.

## Design Philosophy

### Composition Over Complexity

The crossbar is a composition of well-tested building blocks:

- APB Slaves (M instances) - Convert incoming APB to internal cmd/rsp

- Arbiters (N instances) - Select winning master per slave

- Address Decode - Route commands to appropriate slave

- Response Routing - Return responses to originating masters

- APB Masters (N instances) - Convert internal cmd/rsp back to APB

### Scalability

Resource usage scales predictably: - M x N routing paths - N independent arbiters - Linear growth with configuration size

Next: System Context

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# System Context

## System Context Diagram

The Lattice in a typical SoC context:

### Figure 2.1: System Context Diagram

System Context Diagram

## Typical Integration Topology

### Master-Side Connections

The crossbar’s master ports face upstream APB masters:

Typical Master Connections

| Master Type | Typical Source | Connection Point |
| --- | --- | --- |
| CPU | AHB-to-APB Bridge | m0_apb_* |
| DMA | DMA Controller APB Port | m1_apb_* |
| Debug | Debug Access Port | m2_apb_* (if present) |

### Slave-Side Connections

The crossbar’s slave ports face downstream APB peripherals:

Typical Slave Connections (1to4 example)

| Slave Type | Address Offset | Connection Point |
| --- | --- | --- |
| UART | +0x0_0000 | s0_apb_* |
| GPIO | +0x1_0000 | s1_apb_* |
| Timer | +0x2_0000 | s2_apb_* |
| SPI | +0x3_0000 | s3_apb_* |

## Interface Boundaries

### Input Boundary (Master-Side)

Each master port presents a complete APB slave interface:

- Receives PSEL, PENABLE, PADDR, PWRITE, PWDATA, PSTRB, PPROT

- Returns PRDATA, PSLVERR, PREADY

### Output Boundary (Slave-Side)

Each slave port presents a complete APB master interface:

- Drives PSEL, PENABLE, PADDR, PWRITE, PWDATA, PSTRB, PPROT

- Receives PRDATA, PSLVERR, PREADY

### Clock and Reset

Single clock domain: - pclk - APB clock (all signals synchronous) - presetn - Active-low asynchronous reset

## System Integration Considerations

### Address Map Planning

The fixed 64KB per-slave allocation requires planning:

- Peripheral fit: Ensure each peripheral needs <= 64KB address space

- Base address: Set BASE_ADDR to avoid conflicts with other subsystems

- Address width: ADDR_WIDTH must accommodate BASE_ADDR + (NUM_SLAVES x 64KB)

### Multi-Master Considerations

When using multiple masters:

- Arbitration overhead: a contended access waits a FULL transaction, not 1-2 cycles. The grant is held to the response handshake (WAIT_GNT_ACK(1)), so each master ahead in the round-robin costs ~11 cycles (the arbitrated back-to-back period). Uncontended, the registered grant still costs exactly 1 cycle – see HAS 5.2.

- Priority: All masters have equal priority (round-robin)

- Bandwidth: Shared access reduces per-master bandwidth

Next: Chapter 3: Architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Block Diagram

## Top-Level Architecture

The Lattice has three functional layers:

### Figure 3.1: Lattice Block Diagram

Block Diagram

One master’s path to one slave; every master and slave repeats it. The request takes the center column: the master-side apb4_slave turns the APB transfer into a cmd packet, the address decode picks the slave window, and the slave’s round-robin arbiter (the trapezium, where every master’s cmd for that slave contends) grants one master and holds the grant until the response handshake. The completion climbs the right column, routed back by the slave index registered when the command was accepted; an address outside every window completes locally with PSLVERR.

## Functional Blocks

### Master-Side Protocol Conversion

Component: apb4_slave[M] instances (one per master)

Each APB slave instance: - Accepts incoming APB transactions from a master - Converts the APB protocol to the internal command/response format - Buffers the transaction - Generates the response timing

Interface: - Input: Complete APB slave signals from external master - Output: Internal cmd/rsp bus to crossbar fabric

### Internal Crossbar Fabric

Components: - Address Decode - Parallel decode logic - Per-Slave Arbiters - Round-robin arbiter per slave (N instances) - Response Routing - Combinational response mux (the registers on this path live in the boundary IP’s skid buffers, not the mux)

The crossbar fabric: 1. Decodes transaction address to determine target slave 2. Arbitrates when multiple masters request the same slave 3. Routes commands to the selected slave 4. Routes responses back to the originating master

### Slave-Side Protocol Conversion

Component: apb4_master[N] instances (one per slave)

Each APB master instance: - Accepts internal cmd/rsp from crossbar fabric - Generates compliant APB transactions to the slave - Handles wait states via PREADY - Propagates error responses via PSLVERR

Interface: - Input: Internal cmd/rsp bus from crossbar fabric - Output: Complete APB master signals to external slave

## Module Hierarchy

apbx_xbar_MxN
+-- apb4_slave[0..M-1]          # Master-side conversion
+-- addr_decode                 # Parallel address decode
+-- arbiter[0..N-1]            # Per-slave round-robin
+-- response_mux               # Response routing
+-- apb4_master[0..N-1]         # Slave-side conversion

## Data Path Width

Data path widths are consistent throughout:

Data Path Width Parameters

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | 32 | 18-64 (decoding variants: the slave-index part-select reads bit 17); 1-64 for 1to1/2to1 | Address bus width |
| DATA_WIDTH | 32 | 8-64 | Data bus width |
| STRB_WIDTH | 4 | 1-8 | Byte strobe width (DATA_WIDTH/8) |

Next: Data Flow

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Data Flow

## Transaction Flow Overview

### Figure 3.2: Transaction Data Flow

Data Flow Diagram

## Write Transaction Flow

A write transaction passes through six stages:

### Stage 1: Master Request

The external master initiates an APB write: 1. Master asserts PSEL and PADDR with target address 2. Master sets PWRITE=1 and provides PWDATA 3. Next cycle: Master asserts PENABLE

### Stage 2: Protocol Conversion (Master-Side)

The apb4_slave module converts to internal format: 1. Captures transaction parameters (address, data, write) 2. Generates internal command 3. Routes to address decode

### Stage 3: Address Decode

Parallel decode determines target slave:

slave_index = offset[16 +: $clog2(S)]      // offset = PADDR - BASE_ADDR

A part-select, not a mask. & (S-1) appeared here until 2026-08-30 and is only equivalent when S is a power of two – for a generated 3-slave variant the mask would make slave 1 unreachable. The generator emits a part-select (slave_sel = m0_cmd_offset[13:12] for S = 3 at the generator’s default 4KB window; the shipped 64KB variants read offset[16 +: $clog2(S)]), and the sibling formulas in ch02/ch03 always did.

### Stage 4: Arbitration (if needed)

If multiple masters target the same slave: 1. Arbiter selects one master (round-robin priority) 2. Other masters wait (PREADY low) 3. Grant held until transaction completes

### Stage 5: Protocol Conversion (Slave-Side)

The apb4_master module generates the APB transaction: 1. Asserts PSEL to selected slave 2. Provides PADDR, PWRITE, PWDATA 3. Asserts PENABLE next cycle 4. Waits for slave PREADY

### Stage 6: Response Return

The response flows back to the originating master: 1. Slave asserts PREADY (with optional PSLVERR) 2. apb4_master captures response 3. Response routed to correct apb4_slave 4. apb4_slave asserts PREADY to original master

## Read Transaction Flow

Read transactions follow the same flow with these differences:

- PWRITE=0 indicates read operation

- No PWDATA on forward path

- PRDATA returned on response path

## Timing Summary

Transaction Timing Summary

| Transaction Phase | Clock Cycles | Notes |
| --- | --- | --- |
| Setup phase | 1 | PSEL asserted, PENABLE low |
| Data phase | 1+ | PENABLE high, wait for PREADY |
| Decode | 0 | Combinational (parallel decode) |
| Arbitration | 0 or 1 | 0 when M = 1 – the generator emits no arbiter at all. 1 when M > 1, even uncontended, because the grant is REGISTERED |

## Back-to-Back Transactions

The crossbar accepts back-to-back transactions – but they do NOT overlap:

- A master may hold PSEL and start the next SETUP the cycle after PREADY

- apb4_slave is one-command-at-a-time, so the next command is not captured until the previous transaction completes

- Sustained cadence is therefore one cycle LONGER than single-transfer latency: measured PREADY-to-PREADY = 10 pclk cycles at the earliest legal turnaround, against 9 cycles SETUP-to-PREADY for one transfer (M = 1; 11 and 10 on an arbitrated variant). The extra cycle is the next transfer’s mandatory SETUP phase, which cannot overlap the previous ACCESS (see 5.1/5.2)

Next: Address Mapping

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Address Mapping

## Address Map Structure

The shipped Lattice variants use a 64KB address region per slave. The size is chosen at generation (--slave-size, generator default 4KB) and then baked into the decode part-select, so it is fixed for a given generated module:

### Figure 3.3: Address Mapping Structure

Address Mapping

## Default Address Map

With default BASE_ADDR = 0x1000_0000:

Default Address Map (4-slave example)

| Slave Index | Start Address | End Address | Size |
| --- | --- | --- | --- |
| 0 | 0x1000_0000 | 0x1000_FFFF | 64KB |
| 1 | 0x1001_0000 | 0x1001_FFFF | 64KB |
| 2 | 0x1002_0000 | 0x1002_FFFF | 64KB |
| 3 | 0x1003_0000 | 0x1003_FFFF | 64KB |
| … | … | … | … |
| N-1 | BASE + (N-1)*64KB | BASE + N*64KB - 1 | 64KB |

## Address Decode Logic

The target slave comes from simple bit extraction:

offset = PADDR - BASE_ADDR
slave_index = offset[16 +: $clog2(S)]   // ceil(log2(S)) bits: [17:16] for S=4
local_addr = offset[15:0]      // Bits 15:0 are local address within slave

### Example Decode

For address 0x1002_3456 with BASE_ADDR = 0x1000_0000:

Address Decode Example

| Step | Calculation | Result |
| --- | --- | --- |
| 1. Compute offset | 0x1002_3456 - 0x1000_0000 | 0x0002_3456 |
| 2. Extract slave index | offset[17:16] (S=4) | 0x2 (Slave 2) |
| 3. Extract local address | offset[15:0] | 0x3456 |

## Customizing Base Address

The BASE_ADDR parameter shifts the entire address map:

// Peripheral space at 0x4000_0000
apbx_xbar_1to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_xbar (...);

// Resulting map:
// Slave 0: 0x4000_0000 - 0x4000_FFFF
// Slave 1: 0x4001_0000 - 0x4001_FFFF
// Slave 2: 0x4002_0000 - 0x4002_FFFF
// Slave 3: 0x4003_0000 - 0x4003_FFFF

## Address Map Constraints

### Fixed Slave Region Size

Each slave occupies exactly 64KB (0x10000 bytes). This is a current design constraint.

Implications: - Peripherals requiring more than 64KB need multiple slave ports - Peripherals using less than 64KB have unused address space

### Maximum Configuration

With 4-bit slave index extraction: - Maximum 16 slaves - Total address space: 16 x 64KB = 1MB

### Address Width Requirement

The ADDR_WIDTH parameter must be sufficient to address: - BASE_ADDR + (NUM_SLAVES x 64KB)

Example: For 4 slaves at BASE_ADDR = 0x8000_0000, need: - 0x8000_0000 + 0x4_0000 = 0x8004_0000 - Minimum ADDR_WIDTH: 32 bits

Next: Chapter 4: Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Master-Side APB Interfaces

## Interface Overview

Each master port presents a complete APB slave interface, accepting transactions from an external APB master (CPU, DMA, etc.).

## Signal Definition

For each master index i (0 to M-1):

Master Port Signal Definition

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m<i>_apb_PSEL | 1 | Input | Peripheral select |
| m<i>_apb_PENABLE | 1 | Input | Transfer enable |
| m<i>_apb_PADDR | ADDR_WIDTH | Input | Address bus |
| m<i>_apb_PWRITE | 1 | Input | Write/read direction |
| m<i>_apb_PWDATA | DATA_WIDTH | Input | Write data bus |
| m<i>_apb_PSTRB | STRB_WIDTH | Input | Write strobes |
| m<i>_apb_PPROT | 3 | Input | Protection attributes |
| m<i>_apb_PRDATA | DATA_WIDTH | Output | Read data bus |
| m<i>_apb_PSLVERR | 1 | Output | Slave error response |
| m<i>_apb_PREADY | 1 | Output | Transfer ready |

### APB5 Sideband (mixed-version variants only)

An APB5 master port carries five additional pins. They exist ONLY on ports the generator was told are APB5 – in the shipped apbx_xbar_2to2_mixed that is m1 alone, and m0 has none of them. That structural absence IS the version gate; there is no mask to set.

APB5 Master Sideband Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m<i>_apb_PAUSER | 1 | Input | Request user attributes; forwarded only to an APB5 slave |
| m<i>_apb_PWUSER | 1 | Input | Write-data user attributes; likewise |
| m<i>_apb_PRUSER | 1 | Output | Completer read-data user attributes, from an APB5 slave |
| m<i>_apb_PBUSER | 1 | Output | Completer response user attributes, from an APB5 slave |
| m<i>_apb_PWAKEUP | 1 | Input | Requester wake; forwarded to every APB5 slave port – see below |

Reading from an APB4 slave returns '0 on PRUSER/PBUSER rather than a stale or fabricated value, and that is proven, not just tested (formal/apbx_xbar/apbx_xbar_2to2_mixed/, ap_gate_no_fab_*).

PWAKEUP is an input here. AMBA APB (IHI 0024E section 3.7.2, Table 3-7) defines PWAKEUP as a Requester output, and the external master is the Requester on this port. The crossbar ORs it into PWAKEUP on every APB5 slave port, without registering it, so a wake raised a cycle ahead of PSEL reaches the completer a cycle ahead of PSEL. An APB4 master has no PWAKEUP. Its transfers to an APB5 slave are still covered by the crossbar’s own wake, as described in Slave Side. The forwarding is proven (ap_pwakeup_forwarded in formal/apbx_xbar/apbx_xbar_2to2_mixed/). Earlier crossbars drove this pin as an output tied to '0, under the old reversed direction. That was corrected in issue #102 item 7.

The user-signal widths are fixed at 1 bit, not parameterised. This table previously named AUSER_WIDTH/WUSER_WIDTH/RUSER_WIDTH/BUSER_WIDTH as though an integrator could size them. There are no such parameters: the whole module takes only ADDR_WIDTH, BASE_ADDR, DATA_WIDTH and STRB_WIDTH, and the generator hardcodes AUSER_WIDTH(1) .. BUSER_WIDTH(1) at each boundary shim. Widening a user field means regenerating the crossbar, not overriding a parameter.

Parity pins are absent entirely: every shipped variant is generated with parity off. A variant generated with enable_parity=True adds the IHI 0024E chapter 5 check signals on each APB5 port. Inputs are PADDRCHK, PCTRLCHK, PSELCHK, PENABLECHK, PWDATACHK, PSTRBCHK, PAUSERCHK, PWUSERCHK and PWAKEUPCHK. Outputs are PRDATACHK, PREADYCHK, PSLVERRCHK, PRUSERCHK and PBUSERCHK. All are odd parity, plus m<i>_parity_error_wdata and m<i>_parity_error_ctrl. See the GAS chapter Mixed APB4/APB5 and Parity for the widths.

## Signal Descriptions

### PSEL (Input)

Peripheral select signal. When asserted: - Indicates the master wants to access the crossbar - Must remain asserted throughout the transaction - Combined with PADDR determines target slave

### PENABLE (Input)

Transfer enable signal. APB state machine: - PSEL=1, PENABLE=0: Setup phase - PSEL=1, PENABLE=1: Access phase (data transfer)

### PADDR (Input)

Address bus. Used for: - Target slave selection (upper bits) - Local address within slave (lower bits) - Must be stable while PSEL asserted

### PWRITE (Input)

Transfer direction: - PWRITE=1: Write transaction (data from master) - PWRITE=0: Read transaction (data to master)

### PWDATA (Input)

Write data bus: - Valid during write transactions - Sampled when PREADY and PENABLE both high - Width set by DATA_WIDTH parameter

### PSTRB (Input)

Write byte strobes: - One bit per byte of DATA_WIDTH - Indicates which bytes are valid in PWDATA - Ignored during read transactions

### PPROT (Input)

Protection attributes (3 bits): - Bit 0: Privileged/normal access - Bit 1: Secure/non-secure access - Bit 2: Instruction/data access

### PRDATA (Output)

Read data bus: - Valid during read transactions when PREADY high - Undefined during write transactions - Width matches DATA_WIDTH parameter

### PSLVERR (Output)

Slave error response: - Sampled when PREADY and PENABLE both high - PSLVERR=1 indicates error condition - Propagated from the downstream slave, or generated locally by this port when the address decodes outside the map: on the decoding variants a miss completes here with PSLVERR=1 and never reaches a slave (projects/components/fabric-gen-ip/lattice-apbx-xbar BUG-002, was APBX-005). The 1to1 and 2to1 variants have no decode and so never generate it locally.

### PREADY (Output)

Transfer ready signal: - PREADY=1 allows the transaction to complete - PREADY=0 inserts wait states - Reflects downstream slave PREADY (or arbitration wait)

## Transaction Timing

### Uncontended Write (No Arbitration Wait)

Drawn to scale for a single-master variant: 9 pclk edges SETUP-to-PREADY, 10 on an arbitrated one (see 5.2).

### Figure 4.1: Uncontended Write at a Master Port

Uncontended Write at a Master Port

### Contended Transaction (Arbitration Wait)

When another master holds the target slave:

### Figure 4.2: Contended Transfer

Contended Transfer

Next: Slave-Side Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Slave-Side APB Interfaces

## Interface Overview

Each slave port presents a complete APB master interface, driving transactions to an external APB slave (peripheral).

## Signal Definition

For each slave index j (0 to N-1):

Slave Port Signal Definition

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s<j>_apb_PSEL | 1 | Output | Peripheral select |
| s<j>_apb_PENABLE | 1 | Output | Transfer enable |
| s<j>_apb_PADDR | ADDR_WIDTH | Output | Address bus |
| s<j>_apb_PWRITE | 1 | Output | Write/read direction |
| s<j>_apb_PWDATA | DATA_WIDTH | Output | Write data bus |
| s<j>_apb_PSTRB | STRB_WIDTH | Output | Write strobes |
| s<j>_apb_PPROT | 3 | Output | Protection attributes |
| s<j>_apb_PRDATA | DATA_WIDTH | Input | Read data bus |
| s<j>_apb_PSLVERR | 1 | Input | Slave error response |
| s<j>_apb_PREADY | 1 | Input | Transfer ready |

### APB5 Sideband (mixed-version variants only)

Symmetric with the master side, and equally structural: these pins exist ONLY on slave ports the generator was told are APB5. In the shipped apbx_xbar_2to2_mixed that is s0 alone – s1 has none of them – so the absence IS the version gate, with no mask to set. Note the pairing in that variant is crosswise: the APB5 port is m1 on the master side and s0 here.

APB5 Slave Sideband Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s<j>_apb_PAUSER | 1 | Output | Request user attributes, forwarded from an APB5 master |
| s<j>_apb_PWUSER | 1 | Output | Write-data user attributes, likewise |
| s<j>_apb_PRUSER | 1 | Input | Read-data user attributes returned by the slave |
| s<j>_apb_PBUSER | 1 | Input | Response user attributes returned by the slave |
| s<j>_apb_PWAKEUP | 1 | Output | Crossbar wake toward the slave – see below |

Widths are fixed at 1 bit. The generator hardcodes AUSER_WIDTH(1) .. BUSER_WIDTH(1) on the boundary shim and exposes no parameter for them.

s<j>_apb_PWAKEUP is driven by the crossbar. On this interface the crossbar is the Requester, and PWAKEUP is a Requester output (IHI 0024E section 3.7.2, Table 3-7). It is the OR of two terms:

- The boundary apb5_master IP’s own registered wake. It is high while a command for this slave is pending or a transfer is in flight, it leads PSEL by a cycle, and it holds through PREADY. This term covers transfers from APB4 masters, which have no PWAKEUP. 3.7.2 lets a slave wait for PWAKEUP before PREADY, so a PSEL without it could deadlock.

- Every APB5 master’s m<i>_apb_PWAKEUP. All masters can address all slaves, so the wake goes to every APB5 slave port, not to the decoded one. The address is not known until PSEL, and 3.7.2 recommends the wake lead PSEL.

3.7.2 names a glitch-free OR tree as a valid construction. Three properties are proven in formal/apbx_xbar/apbx_xbar_2to2_mixed/:

- ap_pwakeup_forwarded: a master’s wake always reaches the slave.

- ap_pwakeup_covers_psel: the slave is never selected without PWAKEUP.

- ap_pwakeup_not_fabricated: from reset, with no master wake and no traffic, the wake stays low.

The slave’s own wake is not forwarded to any master. Earlier crossbars took this pin as an input and dropped it, under the old reversed direction. That was corrected in issue #102 item 7. With parity on, the port’s PWAKEUPCHK is ~s<j>_apb_PWAKEUP, computed over the OR.

Reading from an APB4 slave returns '0 on PRUSER/PBUSER rather than a stale or fabricated value, proven by ap_gate_no_fab_* in formal/apbx_xbar/apbx_xbar_2to2_mixed/.

## Signal Behavior

### Output Signals (Crossbar to Slave)

PSEL: Asserted when a transaction targets this slave. Remains high throughout the transaction.

PENABLE: Follows standard APB timing – exactly one setup cycle (PSEL high, PENABLE low) before ACCESS, on every transfer.

Until 2026-08-28 apb4_master/apb5_master drove two setup cycles when launching out of IDLE, measured [2,2,2] at a downstream port; only back-to-back transfers taking the ACCESS->SETUP shortcut were compliant. Fixed as AMBA TASK-071 by not asserting PSEL in the IDLE launch cycle, which cost no latency because the state sequence was unchanged. Now measured [1,1,1,1,1] downstream, asserted by val/amba/test_apb_master_setup_phase.py and by the downstream check in dv/tests/test_apbx_xbar_timing.py.

- Low during setup phase

- High during access phase

PADDR: Contains the full address from the master. The local address within a 64KB region is offset[15:0] – note this is the OFFSET’s low bits, not PADDR’s, so it only equals PADDR[15:0] when BASE_ADDR is 64KB-aligned (which is permitted but not required).

PWRITE, PWDATA, PSTRB, PPROT: Passed through from the winning master without modification.

### Input Signals (Slave to Crossbar)

PRDATA: Captured by crossbar when both PREADY and PENABLE are high. Routed to originating master.

PSLVERR: Captured with PRDATA. Propagated to originating master as error indication.

PREADY: Controls transaction completion: - PREADY=1: Transaction completes this cycle - PREADY=0: Insert wait states

## Slave Response Handling

### Wait States

Slaves can insert wait states by holding PREADY low:

### Figure 4.3: Slave Wait States

Slave Wait States

The crossbar: - Holds master PREADY low while waiting - Does not timeout (assumes slaves always respond) - Passes through all wait states to master

### Error Response

When slave asserts PSLVERR:

- Crossbar captures PSLVERR with PRDATA

- Routes PSLVERR to originating master

- Master sees PSLVERR=1 when its PREADY goes high

No error handling or recovery - error indication only.

## Multi-Slave Considerations

### Address Overlap Prevention

Each slave port is mutually exclusive: - Only one slave receives PSEL for any given address - No overlapping address regions - Address decode is deterministic

### Independent Operation

Each slave operates independently: - Own PREADY timing - No synchronization between slaves - Concurrent transactions to different slaves allowed (different masters)

Next: Clock and Reset

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Clock and Reset

## Clock Interface

### Signal Definition

Clock Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| pclk | 1 | Input | APB clock |

### Clock Requirements

Single Clock Domain: - All crossbar logic operates on the pclk rising edge - All APB signals sampled/driven synchronous to pclk - No internal clock generation or division

Frequency: - No minimum frequency requirement - Maximum frequency set by the critical path - Typical: 100-250 MHz (depends on target technology)

Duty Cycle: - No specific duty cycle requirement - Standard 50% duty cycle recommended

## Reset Interface

### Signal Definition

Reset Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| presetn | 1 | Input | Active-low asynchronous reset |

### Reset Behavior

Assertion (presetn goes low): - All internal state cleared immediately - All output signals driven to safe values - Master ports: PREADY=0, PRDATA=0, PSLVERR=0 - Slave ports: PSEL=0, PENABLE=0, PADDR=0, etc.

Deassertion (presetn goes high): - Internal state begins normal operation - Reset release is NOT synchronized internally – presetn is applied directly as an asynchronous reset (ALWAYS_FF_RST) in every module. The integrator must synchronize reset DEASSERTION to pclk externally (recovery/removal timing) - Ready to accept transactions from the first pclk edge after a correctly synchronized deassertion – NOT “immediately” in the sense of the deassertion instant. Until that externally synchronized removal lands cleanly, the fabric’s state is not defined; the “Reset Deassertion” guidance later on this page (wait 1 cycle before the first transaction) is the operative rule.

### Reset Safe Values

Reset Safe Values

| Signal Group | Reset Value | Rationale |
| --- | --- | --- |
| Master PREADY | 0 | apb4_slave asserts PREADY only in BUSY |
| Master PRDATA | 0 | No false data |
| Master PSLVERR | 0 | No false errors |
| Slave PSEL | 0 | No false transactions |
| Slave PENABLE | 0 | No false enables |
| Arbiter grants | 0 | No master selected |

## Clock-Reset Relationship

### Reset Assertion

Reset can be asserted at any time: - Asynchronous assertion (immediate effect), in every build. This comes from ALWAYS_FF_RST in rtl/amba/includes/reset_defs.svh, which is unconditionally async; the old USE_ASYNC_RESET switch is a no-op. - Deassertion must be synchronous to pclk – see “Reset Deassertion” below. This bullet previously also permitted asynchronous deassertion, contradicting that section three lines later. - All in-flight transactions aborted

### Reset Deassertion

For clean operation: 1. Assert reset for minimum 2 pclk cycles 2. Deassert reset synchronous to pclk rising edge 3. Wait 1 cycle before first transaction

### Figure 4.4: Reset Sequence

Reset Sequence

## Integration Notes

### Clock Domain Crossing

If masters or slaves are in different clock domains: - Insert CDC logic external to the crossbar - The crossbar assumes a single clock domain - Consider an APB clock bridge if needed

### Clock Gating

The crossbar supports clock gating: - No internal activity when idle - Safe to gate pclk when no transactions pending - All registers use standard flip-flops (no clock gating cells)

Next: Chapter 5: Performance

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Throughput Characteristics

## Maximum Throughput

### Single Master

With a single master accessing any slave, the figures split by arbiter presence: the generator emits an arbiter only when M > 1, and its registered grant costs a cycle on every one of them. See 5.2.

Single-Master-Active Throughput, by Variant Class

| Metric | M = 1 | M > 1 | Notes |
| --- | --- | --- | --- |
| Cycles per transaction (uncontended) | 9 SETUP->PREADY; 8 fabric | 10 SETUP->PREADY; 9 fabric | measured, zero-wait slave |
| Sustained cycles per transaction | 10 (PREADY->PREADY) | 11 (PREADY->PREADY) | measured back-to-back, one active master |
| Maximum transactions per cycle | ~0.100 | ~0.091 | 1 transaction / 10 or 11 cycles |
| Data throughput (32-bit @ 100MHz) | ~40 MB/s | ~36 MB/s | 4 B per period |
| Data throughput (32-bit @ 250MHz) | ~100 MB/s | ~91 MB/s | 4 B per period |

M = 1: apbx_xbar_1to1, apbx_xbar_1to4. M > 1: apbx_xbar_2to1, apbx_xbar_2to4, apbx_xbar_2to2_mixed – so the 2x4 the PRD calls the typical SoC case is the slower column. This table published the M = 1 figures unconditionally until 2026-08-29.

### Multi-Master (Uncontended)

When masters access different slaves: - Each master achieves single-master throughput - Aggregate throughput = M x single-master throughput - No CONTENTION overhead – but the registered grant still costs one cycle on every M > 1 variant, contended or not (see 5.2). Only the single-master variants have no arbiter at all.

### Multi-Master (Contended)

When masters compete for the same slave:

Contended Access Throughput

| Masters | Effective Throughput | Notes |
| --- | --- | --- |
| 2 | 50% per master | Round-robin sharing |
| 3 | 33% per master | Round-robin sharing |
| 4 | 25% per master | Round-robin sharing |

## Arbitration Handoff

### Grant Persistence

The crossbar implements grant persistence: - A master holds the grant for the duration of ONE transaction - The grant is released at the RESPONSE handshake, not when PSEL drops (grant_ack = grant && rsp_valid && rsp_ready, arbiter instantiated with WAIT_GNT_ACK(1)), after which the round-robin mask rotates – holding PSEL does not keep the grant if another master is requesting - A single master with no competitor re-wins immediately, so its transactions still stream back-to-back – at the fabric’s own cadence, not at APB’s 2-cycle minimum

### Back-to-Back Timing

A “zero-bubble” 4-cycle two-transaction diagram appeared here in earlier revisions. It described a pipeline this RTL does not have: the apb4_slave front end is a one-command-at-a-time FSM (IDLE -> BUSY -> WAIT), so the next command is not captured until the previous transaction completes. But sustained cadence is not equal to single-transfer latency either – it is one cycle longer. A master holding PSEL high and dropping PENABLE for exactly one cycle after each PREADY (the earliest LEGAL turnaround) measures PREADY-to-PREADY = 10 pclk cycles, against a 9-cycle SETUP-to-PREADY single transfer. The extra cycle is the next transfer’s mandatory SETUP phase, which cannot overlap the previous transfer’s ACCESS. See 5.2 for the convention and the cycle-by-cycle breakdown. There is still no overlap between consecutive transactions to draw.

## Throughput Factors

### Factors That Reduce Throughput

Throughput Reduction Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Slave wait states | Variable | Choose faster peripherals |
| Arbitration conflicts | ~11 cycles per master ahead in the round-robin | Distribute access across slaves |
| Slave response time | Variable | Design slaves for quick response |

### Optimal Usage Pattern

For maximum throughput: 1. Distribute master access across different slaves 2. Minimize slave wait states 3. Use burst-like access patterns (same master, same slave) 4. Avoid rapid master switching on same slave

Next: Latency Analysis

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Latency Analysis

## Transaction Latency

### Uncontended Access

When a master has exclusive access to a slave:

Uncontended Transaction Latency

| Phase | Cycles | Description |
| --- | --- | --- |
| Master SETUP | 1 | PSEL high, PENABLE low |
| apb4_slave capture + cmd skid | 2 | command registered out |
| apb4_master IDLE-launch / SETUP / ACCESS | 3 | downstream PSEL then PENABLE |
| Response + rsp skid | 2 | slave PREADY through the return skid |
| apb4_slave BUSY -> PREADY | 1 | master-visible completion |
| Total | 9 | PSEL to PREADY (8 of them ACCESS to PREADY) |

These are measured numbers, not protocol minimums. A crossbar transfer is NOT the 2-cycle APB minimum: the fabric converts APB to an internal cmd/rsp protocol through apb4_slave and back through apb4_master, and both directions cross REGISTERED skid buffers.

Measurement convention. Every figure below counts rising pclk edges and names the two edges it spans. State the edges or the number means nothing – this page has been wrong twice from not doing so.

The numbers depend on whether the variant has an arbiter, and the generator only emits one when there is more than one master. A single-master variant has no arbiter_round_robin at all; a multi-master variant has one per slave, and its grant is a flop (grant <= w_next_grant), which costs exactly one cycle on every quantity below.

Measured Timing, Always-Ready Slave

| Quantity | M = 1 | M > 1 | Spans |
| --- | --- | --- | --- |
| Fabric latency | 8 | 9 | first ACCESS edge -> edge where PREADY is high |
| Single-transfer latency | 9 | 10 | SETUP edge -> edge where PREADY is high |
| Back-to-back period | 10 | 11 | PREADY edge -> next PREADY edge |

M = 1 is apbx_xbar_1to1 and apbx_xbar_1to4 (0 arbiters). M > 1 is apbx_xbar_2to1, apbx_xbar_2to4 and apbx_xbar_2to2_mixed (1, 4 and 2 arbiters). All six columns are measured, not derived – see dv/tests/test_apbx_xbar_timing.py, which asserts both classes.

Until 2026-08-29 this page published the M = 1 numbers unconditionally, under the heading “Uncontended Access”, having measured only apbx_xbar_1to1. They were one cycle optimistic for every arbitrated variant – including the 2x4 that the PRD calls the typical SoC case.

The period is 10, not 9, and the difference is structural. After PREADY at cycle N the bus is still in ACCESS for that cycle, so the next transfer’s mandatory SETUP cycle cannot begin before N+1; its ACCESS follows at N+2 and its PREADY at N+2+8 = N+10. Reaching 9 would require a SETUP cycle overlapping the previous transfer’s ACCESS, which is not a legal APB waveform. So sustained cadence does not equal single-transfer latency here – it is exactly one cycle longer.

Earlier revisions of this page claimed 2 cycles (the bare APB minimum for a directly-attached slave, which does not apply through this fabric), and later claimed a 9-cycle sustained cadence, which came from a probe whose turnaround was not legal APB. A qc reviewer flagged the 9-cycle cadence and was told it was a false positive; the reviewer was right. dv/tests/test_apbx_xbar_timing.py now asserts all three numbers against the RTL so the question is settled by the suite rather than by argument.

### Contended Access

When multiple masters compete for the same slave:

Arbitration Latency

| Scenario | Additional Latency | Description |
| --- | --- | --- |
| Win arbitration | 1 cycle | grant is REGISTERED in arbiter_round_robin |
| Lose to 1 master | ~10+ cycles | wait one full transaction (see above) |
| Lose to N masters | ~10N+ cycles | wait N transactions |

## Latency Breakdown

### Forward Path (Master to Slave)

Forward Path Latency

| Component | M = 1 | M > 1 | Type |
| --- | --- | --- | --- |
| apb4_slave capture | 1 | 1 | Registered |
| apb4_slave cmd skid out | 1 | 1 | Registered |
| Address decode | 0 | 0 | Combinational |
| Arbitration | 0 | 1 | No arbiter is emitted when M = 1; when one is, its grant is a flop |
| apb4_master cmd skid in | 1 | 1 | Registered |
| apb4_master cmd skid out | 1 | 1 | Registered |
| apb4_master IDLE -> SETUP | 1 | 1 | FSM |
| Total | 5 | 6 | master SETUP edge to downstream PSEL edge |

Both columns measured: 5 on apbx_xbar_1to1, 6 on apbx_xbar_2to1. The arbitration row is the whole difference between the two variant classes. This table carried a single unconditional column with a 1-cycle arbitration row and a 5-cycle total until 2026-08-29 – which cannot both be true, since the 5 was measured on the variant that has no arbiter.

The three numbers on this page must sum, and now do:

M = 1 M > 1 5 forward 6 forward + 1 downstream SETUP -> ACCESS + 1 downstream SETUP -> ACCESS + 3 response + 3 response = 9 SETUP-to-PREADY = 10 SETUP-to-PREADY

(With a zero-wait slave the downstream ACCESS and PREADY edges are the same cycle, which is why the response row can be read from either. A slave with wait states adds its own cycles between them and shifts the total.)

This table read 4 until 2026-08-29, which could not be reconciled with the page’s own 9-cycle total – 4 + 3 left two cycles unaccounted for. Measured on apbx_xbar_1to1 with an always-ready slave: master SETUP at cycle 1, downstream PSEL at cycle 6, downstream ACCESS at cycle 7, master PREADY at cycle 10.

### Response Path (Slave to Master)

Response Path Latency

| Component | Cycles | Type |
| --- | --- | --- |
| Slave response | Variable | The slave’s own PREADY timing, excluded below |
| apb4_master captures into rsp skid | 1 | Registered |
| rsp skid out, through the response mux | 1 | Skid registered; the mux itself is combinational |
| apb4_slave drives PREADY | 1 | Registered |
| Total | 3 | downstream PREADY edge to master PREADY edge |

Same on both variant classes – arbitration is on the command path only. The rows read “0-1 cycle” each until 2026-08-29, which could not sum to the stated 3.

## Latency Timing Diagram

### Best Case (No Waits, No Contention)

### Figure 5.1: Best-Case Latency Timeline

Best-Case Latency Timeline

### Worst Case (Contention + Slave Wait States)

### Figure 5.2: Arbitration and Slave Wait States

Arbitration and Slave Wait States

## Latency Optimization

### Design Recommendations

Latency Optimization Recommendations

| Goal | Recommendation |
| --- | --- |
| Minimize contention | Use more slaves, spread access |
| Reduce arbitration wait | Lower master count per slave |
| Reduce slave latency | Design slaves with quick PREADY |

Next: Resource Estimates

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Resource Estimates

## FPGA Resource Usage

### Pre-Generated Variants

Estimated resource usage for common configurations (32-bit data width):

FPGA Resource Estimates (32-bit)

| Variant | LUTs | FFs | Description |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | ~150 | ~80 | Full 1x1 with buffering |
| apbx_xbar_2to1 | ~300 | ~150 | 2 masters, arbitration |
| apbx_xbar_1to4 | ~400 | ~200 | 4 slaves, decode |
| apbx_xbar_2to4 | ~800 | ~400 | Full 2x4 crossbar |

Hand estimates, not synthesis results, and the single source of truth for this component. PRD §9.3 previously carried a second, different set of numbers (2-3x smaller in every cell); that table now refers here rather than competing with it. Nothing in the repo has been synthesized to confirm either set – treat these as order-of-magnitude only.

### Scaling Factors

Resource usage scales approximately as:

Resource Scaling Factors

| Component | Scaling | Notes |
| --- | --- | --- |
| apb4_slave instances | M x base | Linear with masters |
| apb4_master instances | N x base | Linear with slaves |
| Arbiters | N x log2(M) | Per slave, sized for masters |
| Address decode | N | Comparators per slave |
| Response mux | N x M | Full crosspoint for responses |

## Lines of Code

Source code metrics for pre-generated variants:

Source Code Metrics

| Variant | Lines of Code | Complexity |
| --- | --- | --- |
| apbx_xbar_1to1 | 165 | Low |
| apbx_xbar_2to1 | 314 | Medium |
| apbx_xbar_1to4 | 417 | Medium |
| apbx_xbar_2to2_mixed | 625 | Medium |
| apbx_xbar_2to4 | 806 | Medium-High |

This table is the single source of truth for LOC, as it already is for the LUT/FF estimates above. Exact wc -l of rtl/*.sv, measured 2026-10-12; the PRD and the key-features page point here rather than carrying their own copies.

Three tables disagreed before that – by up to 33% on the same file (2to4 as ~1000, ~751 and 751) – because each was corrected on a different day. Even the most precise-looking set was stale: 165/314/384/751/570 was right when reconciled on 2026-08-27 and wrong two days later, once the projects/components/fabric-gen-ip/lattice-apbx-xbar BUG-002 (was APBX-005) decode-miss logic added lines. Numbers that must be hand-synced across three pages will desync; keeping one and pointing at it is the fix.

## Power Considerations

### Static Power

Static power scales with: - Total register count (FFs) - Routing resources used - Technology node

### Dynamic Power

Dynamic power depends on: - Transaction rate (activity factor) - Data width (toggling bits) - Clock frequency

### Power Optimization

Power Optimization Techniques

| Technique | Benefit | Trade-off |
| --- | --- | --- |
| Clock gating | Reduce idle power | Adds gating logic |
| Lower data width | Fewer toggling bits | Reduced throughput |
| Fewer slaves | Less decode logic | Reduced flexibility |

## Area Comparison

Relative area comparison (normalized to apbx_xbar_1to1):

Relative Area Comparison

| Variant | Relative Area |
| --- | --- |
| apbx_xbar_1to1 | 1.0x (baseline) |
| apbx_xbar_2to1 | 2.0x |
| apbx_xbar_1to4 | 2.5x |
| apbx_xbar_2to4 | 5.0x |

## Custom Configuration Estimation

For custom MxN configuration:

Estimated LUTs ~= 50 + (M * 75) + (N * 50) + (M * N * 10)
Estimated FFs  ~= 20 + (M * 40) + (N * 30)

Example: 4x8 crossbar - LUTs: 50 + (475) + (850) + (4810) = 50 + 300 + 400 + 320 = ~1070 - FFs: 20 + (440) + (830) = 20 + 160 + 240 = ~420

Next: Chapter 6: Integration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# System Requirements

## Hardware Requirements

### Clock and Reset

Clock and Reset Requirements

| Requirement | Specification |
| --- | --- |
| Clock input | Single clock (pclk) |
| Clock frequency | No minimum, technology-dependent maximum |
| Reset input | Active-low (presetn); asynchronous assertion, synchronous deassertion |
| Reset duration | Minimum 2 clock cycles |

### APB Compliance

The crossbar requires APB-compliant masters and slaves:

APB Compliance Requirements

| Requirement | Description |
| --- | --- |
| Protocol version | APB4 (with PSTRB, PPROT); APB5 sideband supported only on mixed-version variants such as apbx_xbar_2to2_mixed |
| Wait states | Supported (PREADY-based) |
| Error response | Supported (PSLVERR) |
| Slave timeout | Not handled internally |

## Software Requirements

### Address Map Configuration

Software must be configured with the correct address map:

// Example address definitions
#define PERIPHERAL_BASE     0x10000000
#define UART_BASE          (PERIPHERAL_BASE + 0x00000)
#define GPIO_BASE          (PERIPHERAL_BASE + 0x10000)
#define TIMER_BASE         (PERIPHERAL_BASE + 0x20000)
#define SPI_BASE           (PERIPHERAL_BASE + 0x30000)

### Driver Considerations

Driver Considerations

| Consideration | Recommendation |
| --- | --- |
| Polling vs interrupt | Both supported |
| Atomic operations | Not guaranteed across arbitration |
| Cache coherency | APB typically uncached |

## Constraints

### Address Map Constraints

Address Map Constraints

| Constraint | Value | Notes |
| --- | --- | --- |
| Slave region size | 64KB in the shipped variants | Set at generation via --slave-size; fixed once generated |
| Maximum slaves | 16 | Generator limit |
| Maximum masters | 16 | Generator limit |
| Address alignment | Byte aligned | No alignment requirement |

### Transaction Constraints

Transaction Constraints

| Constraint | Value | Notes |
| --- | --- | --- |
| Outstanding transactions | 1 per master | APB protocol limit |
| Transaction interleaving | Not supported | APB is non-pipelined |
| Burst transactions | Not supported | APB single-beat only |

## Integration Checklist

### Pre-Integration

- ☐ Verify BASE_ADDR does not conflict with other subsystems

- ☐ Confirm all slaves fit within 64KB regions

- ☐ Validate ADDR_WIDTH is sufficient for address map

- ☐ Check DATA_WIDTH matches system data width

### Post-Integration

- ☐ Verify address decode routing to correct slaves

- ☐ Test all master-to-slave paths

- ☐ Validate arbitration behavior (if multi-master)

- ☐ Check reset behavior

Next: Parameter Configuration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Parameter Configuration

## Module Parameters

### Core Parameters

Core Parameter Definition

| Parameter | Type | Default | Range | Description |
| --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | 18-64 for decoding variants; 1-64 for 1to1/2to1 | Address bus width in bits. The decode part-select reads bits 16 +: $clog2(S), so a 2-slave variant needs only bit 16 – but the generator emits the range-check span as a fixed 32-bit constant (paddr < BASE_ADDR + 32'h...), so anything below 18 width-expands and fails a warnings-fatal build. 17 lints clean only with -Wno-fatal; it is not usable. |
| DATA_WIDTH | int | 32 | 8, 16, 32, 64 | Data bus width in bits |
| STRB_WIDTH | int | DATA_WIDTH/8 | derived | Write strobe width. Declared as a parameter, but it is computed from DATA_WIDTH and overriding it is not supported |
| BASE_ADDR | logic[ADDR_WIDTH-1:0] | 0x10000000 | any address; the slave index is decoded from the OFFSET, so no span alignment is required | Base of the slave map. UNUSED on 1to1 and 2to1 – they have a single slave and no decode, so the parameter is declared only for interface uniformity and setting it changes nothing (see those modules’ RTL headers) |

### Derived Parameters

These are calculated automatically:

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| STRB_WIDTH | DATA_WIDTH / 8 | Byte strobe width |
| (none) | — | Each variant declares only the four parameters above; master/slave counts are baked into the module name and port list, NOT exposed as parameters. |

## Parameter Usage Examples

### Default Configuration

apbx_xbar_2to4 u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections
);
// Uses defaults: ADDR_WIDTH=32, DATA_WIDTH=32, BASE_ADDR=0x10000000

### Custom Base Address

apbx_xbar_2to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections
);
// Slaves at 0x4000_0000, 0x4001_0000, 0x4002_0000, 0x4003_0000

### 64-bit Data Width

apbx_xbar_2to4 #(
    .DATA_WIDTH(64)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections with 64-bit data buses
);
// STRB_WIDTH automatically becomes 8

### Wide Address

apbx_xbar_2to4 #(
    .ADDR_WIDTH(40),
    .BASE_ADDR(40'h80_0000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections with 40-bit address buses
);

## Parameter Validation

### Address Width Check

Ensure ADDR_WIDTH can accommodate the full address range:

Required minimum: log2(BASE_ADDR + NUM_SLAVES * 64KB)

Example: 4 slaves at BASE_ADDR=0x1000_0000 - Max address: 0x1000_0000 + 4*0x10000 = 0x1004_0000 - Required: 29 bits minimum - ADDR_WIDTH=32 is sufficient

### Data Width Constraints

Valid Data Width Configurations

| DATA_WIDTH | STRB_WIDTH | Valid APB Widths |
| --- | --- | --- |
| 8 | 1 | Yes |
| 16 | 2 | Yes |
| 32 | 4 | Yes (most common) |
| 64 | 8 | Yes |

## Custom Generation Parameters

When using the Python generator for custom configurations:

python generate_xbars.py \
    --masters 3 \
    --slaves 6 \
    --base-addr 0x80000000 \
    --slave-size 0x10000

Generator Command Line Options

| Option | Description | Default |
| --- | --- | --- |
| –masters | Number of master ports | Required |
| –slaves | Number of slave ports | Required |
| –base-addr | Base address (hex) | 0x10000000 |
| –slave-size | Address space per slave (0x1000 = 4KB, 0x10000 = 64KB) | 0x10000 |

--slave-size decides which address bits select the slave: the crossbar decodes offset[log2(slave_size) +: ceil(log2(S))], so 64KB windows give offset[16 +: n] and 4KB windows give offset[12 +: n]. Until 2026-09-07 this script did not pass the value through on the custom path, so a custom variant silently came out with 4KB windows while all four shipped variants had 64KB – one family, two address maps. It now defaults to 64KB to match them.

The lower-level apbx_xbar_generator.py takes the same flag but keeps its own historical default of 4KB; prefer generate_xbars.py so the family stays consistent.

Next: Verification Strategy

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Verification Strategy

## Pre-Verified Component

The Lattice is pre-verified:

Pre-Verification Status

| Test / Variant | Transactions | Status | Coverage |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | 100+ | Pass | Basic connectivity |
| apbx_xbar_2to1 | 130+ | Pass | Arbitration stress |
| apbx_xbar_1to4 | 200+ | Pass | Address decode |
| apbx_xbar_2to4 | 350+ | Pass | Full crossbar stress |
| apbx_xbar_2to2_mixed | 8 directed | Pass | All four APB4/APB5 pairings, sideband gating, decode miss, structural pin check |
| apbx_xbar_timing | 2 variants | Pass | Published latency/cadence figures, both variant classes |

Overall: 100% passing (9/9), >750 transactions plus the directed mixed-version and timing checks. apbx_xbar_2to2_mixed also has a formal harness (formal/apbx_xbar/apbx_xbar_2to2_mixed/).

## Test Categories

### Basic Connectivity

Tests for each variant include: - Single read transaction to each slave - Single write transaction to each slave - Read-modify-write sequences - All address regions exercised

### Address Decode Verification

For multi-slave variants: - Verify each slave selected by correct address range - Boundary address testing - Invalid address behavior (decoding variants complete the access with PSLVERR; they do not hang – see 4.1)

### Arbitration Testing

For multi-master variants: - Round-robin fairness verification - Contention handling - Grant persistence validation - No starvation testing

### Stress Testing

High-intensity scenarios: - Back-to-back transactions - Random delays - Mixed read/write patterns - Concurrent multi-master access

### APB5 Sideband Gating (mixed variant)

For apbx_xbar_2to2_mixed: - All four APB4/APB5 master/slave pairings exercised - Sideband signals flow only where both ends are APB5 - APB4 ports carry no sideband pins (structural check) - Decode-miss completes with PSLVERR

### Timing and Protocol Compliance

test_apbx_xbar_timing asserts the published figures against the RTL: - Single-transfer latency for single-master and arbitrated classes - Back-to-back cadence (PREADY-to-PREADY) - Forward-path and response-path breakdown - Downstream setup-phase count ([1,1,1,1,1])

## Integration Testing Recommendations

### Level 1: Basic Integration

Level 1 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| Reset test | Apply reset, verify outputs | All outputs at safe values |
| Single transaction | Read from each slave | Correct data returned |
| Write verify | Write and readback | Data matches |

### Level 2: Functional Integration

Level 2 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| All masters | Each master accesses each slave | No errors |
| Arbitration | Concurrent master access | Fair grant rotation |
| Error propagation | Force PSLVERR from slave | Error reaches master |

### Level 3: System Integration

Level 3 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| CPU access | Software reads/writes peripherals | Correct behavior |
| DMA access | DMA transfers via crossbar | Data integrity |
| Mixed traffic | CPU and DMA concurrent | No deadlock, fair access |

## Verification Collateral

### Available Test Infrastructure

Test Infrastructure

| Item | Location | Description |
| --- | --- | --- |
| CocoTB tests | dv/tests/ | Python testbenches |
| Wave configs | dv/tests/GTKW/ | GTKWave configurations |
| Coverage | dv/tests/coverage_reports/ | Merged functional coverage reports |
| Build orchestration | dv/tests/Makefile | Area Makefile delegating to make/tests.mk |
| Formal harness (mixed variant) | formal/apbx_xbar/apbx_xbar_2to2_mixed/ | Sideband-gating proofs |

### Running Verification

Tests are run through the area Makefile (which delegates to the shared make/tests.mk infrastructure); bare pytest is not the supported entry point because the Makefile supplies filelists, levels, and coverage options.

cd projects/components/fabric-gen-ip/lattice-apbx-xbar/dv/tests/

# Run all crossbar tests -- clean-all first for a run you intend to trust
make clean-all && make run-all-gate

# Run specific variant
make run-apbx_xbar_2to4-gate

# Run with waveforms (WAVES=1 via the target)
make run-apbx_xbar_2to4-gate-waves
# create_view_cmd() writes the gtkwave command beside the log

## Known Limitations

### Not Tested

Known Limitations

| Scenario | Reason | Recommendation |
| --- | --- | --- |
| Slave timeout | Not implemented | Add external watchdog |
| >16 masters/slaves | Generator limit | Custom modification needed |
| Non-64KB regions | Not supported | Use multiple crossbars |

End of Hardware Architecture Specification

## Related Documentation

- Lattice MAS - Detailed implementation

- PRD.md - Product requirements

- README.md - Quick start guide