RTL Design Sherpa

RAPIDS Characterization Nexys A7 Project Guide 0.91

July 2026

Table of Contents

1 Document Information	8

1.1 References	8

1.2 Terminology	8

1.3 Revision History	9

2 Overview	11

2.1 What it characterizes	11

2.2 Boards and tools	11

2.3 Datapath geometry	11

2.4 Status	12

3 How It Works	13

3.1 The spine and region decode	13

3.1.1 Figure 2.1: RAPIDS Characterization Harness Spine	13

3.2 Key RTL blocks	14

3.3 The DUT and the on-chip harness	15

3.4 Atomic launch	16

4 Test Methodology	17

4.1 What is under test	17

4.2 What is measured, and how	17

4.2.1 Figure 3.1: Measurement Pipeline	18

4.3 Workloads	18

4.4 The oracle	19

4.5 Measurement pitfalls	19

5 Build and Run	20

5.1 Make targets (flows-rapids-beats/Makefile)	20

5.2 The host campaign (flows-rapids-beats/host/)	21

5.3 Typical sequence	22

6 Harness CSR Configuration	23

6.1 Regions and by-name access	23

6.1.1 Waveform 5.1: Atomic Launch	24

6.2 DESC-LOAD region (region 1, byte offsets)	24

6.3 HARNESS CSR — control (region 2)	24

6.4 Atomic-launch staging (region 2)	26

6.5 HARNESS CSR — status and observability	27

7 Simulation vs Silicon	29

7.1 The two paths	29

7.2 The cocotb testbench	29

7.3 Why no UART-in-sim here	30

8 Troubleshooting	31

8.1 Current state	31

8.2 Known gotchas	31

8.3 Cannot find the board	32

8.4 Board-fit vs sim geometry	32

8.5 Three different “0x1000”s	32

8.6 Vivado-only package collisions	32

List of Figures

Figure 2.1: RAPIDS Characterization Harness Spine	13

Figure 3.1: Measurement Pipeline	18

List of Tables

Table 1: Reference documents	8

Table 2: Revision history	9

Table 3: Boards and tool requirements	11

Table 4: Datapath geometry	11

Table 5: Host address regions	13

Table 6: Key RTL blocks	14

Table 7: On-chip harness stimulus and checkers	15

Table 8: What is measured	17

Table 9: Characterization workloads	18

Table 10: flows-rapids-beats make targets	20

Table 11: Host address regions	23

Table 12: DESC-LOAD registers	24

Table 13: Harness CSR — control registers	24

Table 14: Atomic-launch staging registers	26

Table 15: Harness CSR — status and observability	27

Table 16: The two characterization paths	29

Table 17: Known issues and workarounds	31

List of Waveforms

Waveform 5.1: Atomic Launch	24

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 1	Document Information

This is the operator / developer guide for the Nexys A7 RAPIDS Characterization project (projects/fpga-systems/Genesys2/dma-ip/rapids_beats). It explains what the project characterizes, how the harness works, how to build / simulate / program / run it, and how the harness CSR is configured.

The device under test is the RAPIDS beats DMA (rapids_beats_top, from projects/components/dma-ip/rapids/). This guide documents how the harness drives it and the harness control surface — not the DMA core’s internals.

## 1.1	References

Reference documents

| Source | Title |
| --- | --- |
| RTL Design Sherpa | README.md (project overview + quick start) |
| RTL Design Sherpa | docs/rapids_beats_findings.md (architecture, timing, golden CRC) |
| RTL Design Sherpa | flows-rapids-beats/host/README.md (AXIL word map + campaign semantics) |
| RTL Design Sherpa | reports/perf/README.md (8-channel line-rate performance) |
| RTL Design Sherpa | bin/TBClasses/ (shared harness / RegisterMap collateral) |
| ARM | AMBA AXI4 / AXI4-Stream / APB Protocol Specifications |

## 1.2	Terminology

SRC / SNK

The two wholly-separate RAPIDS engines: rapids_src_beats reads memory and emits to the AXIS network; rapids_snk_beats receives from the network and writes memory. They share one APB decode (SRC @ 0x0000, SNK @ 0x1000).

Beats

The fixed-quantum transfer variant of RAPIDS. The Genesys 2 build is DATA_WIDTH = 256 (32 bytes per beat, AXI4 and AXIS alike) since 2026-09-29; peak per-direction bandwidth = 32 B × 100 MHz = 3.2 GB/s. Perf reports v1.0 to v1.5 measured the earlier 512-bit build (64 B/beat, 6.4 GB/s). The bitstream reports its own geometry in the harness BUILD register and the host takes its byte and bandwidth math from there.

Region

The top 4 bits of the host word address (addr[19:16]) select one of three regions: DUT-REG (0), DESC-LOAD (1), HARNESS CSR (2).

Golden CRC

The deterministic CRC-32 (Ethernet 0x04C11DB7, LFSR seed 0xDEADBEEF) that both the on-chip checkers and the host golden model compute; a pass is a CRC match.

## 1.3	Revision History

Revision history

| Version | Date | Notes |
| --- | --- | --- |
| 0.90 | 2026-07-18 | Initial project guide |
| 0.91 | 2026-10-15 | The on-chip run timer and CRC verdict (char_run_timer, shared with STREAM): register map rows at 0x160-0x180, the methodology and Figure 3.1 name it, troubleshooting no longer calls the generator valid flag a non-fatal flake. |
| 0.92 | 2026-10-20 | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |

# 2	Overview

## 2.1	What it characterizes

This project performs on-silicon characterization of the split RAPIDS beats DMA (rapids_beats_top) — refactored into two wholly-separate engines behind one shared APB decode:

- rapids_src_beats — read-only: memory → AXIS network

- rapids_snk_beats — write-only: AXIS network → memory

The harness drives both data paths on real hardware over UART and validates every beat against a deterministic golden CRC-32. It reports per-direction throughput and channel scaling, with an all-on-chip stimulus/checker so the board runs at line rate without a host bottleneck.

There is a single flow, flows-rapids-beats/ (the “beats” fixed-quantum variant of RAPIDS).

## 2.2	Boards and tools

Boards and tool requirements

| Item | Detail |
| --- | --- |
| Default board | Digilent Nexys A7-100T (xc7a100tcsg324-1), 100 MHz, NUM_CHANNELS = 4 |
| Optional board | Genesys 2 (xc7k325tffg900-2) via BOARD=genesys2, 8 channels (200→100 MHz MMCM) |
| Build | Vivado (synth / impl / bitgen / program) |
| Host | Python 3 + pyserial |
| Sim | Verilator + cocotb / cocotb_test / pytest |

## 2.3	Datapath geometry

Datapath geometry

| Parameter | Value |
| --- | --- |
| Data width | 256 bits (32 B/beat), AXI4 and AXIS; 512 bits (64 B) through perf report v1.5 |
| Address width | 64 bits |
| Descriptor | fixed 256-bit |
| SRAM per channel | 128 beats = 4 KB (was 256 x 64 B = 16 KB) |
| Peak per direction | 3.2 GB/s (32 B × 100 MHz) |
| Peak full-duplex | 6.4 GB/s |

## 2.4	Status

Silicon-validated: make smoke passes both paths; make suite passes 48/48 (channels {1,2,4} × beats {1,4,8,16} × backpressure {off,on} × 2 seeds). Timing closes at 100 MHz (WNS +0.007 ns, 0 failing endpoints) at the board-fit geometry NUM_CHANNELS = 4, SRAM_DEPTH = 256. On Genesys 2 the 8-channel 512-bit build reached 99.8–100% line rate (6.40 GB/s per direction, perf report v1.5); the 256-bit / 4 KB-per-channel build is the current design point (Makefile DATA_WIDTH / SRAM_DEPTH, exported to the build and to verify-sim) — see reports/perf/.

# 3	How It Works

## 3.1	The spine and region decode

Host commands arrive over one UART link and are steered by the top 4 bits of the word address (addr[19:16]) into four regions:

### 3.1.1	Figure 2.1: RAPIDS Characterization Harness Spine

RAPIDS characterization harness spine

Source: 01_harness_spine.dot

Host address regions

| Region | Base | Contents |
| --- | --- | --- |
| 0 — DUT-REG | 0x0_0000 | AXIL → apb4_master → the DUT’s APB (SRC @ 0x0000, SNK @ 0x1000) |
| 1 — DESC-LOAD | 0x1_0000 | 256-bit descriptor assembly + single-beat AXI4 write into descriptor RAM |
| 2 — HARNESS CSR | 0x2_0000 | gen / chk / mem / mon / obs control + status readback |
| 3 — OBSERVERS | 0x3_0000 | USE_OBSERVERS=1 only: the shared interface observers’ obs_regs maps, AXI observer @ 0x0000, AXIS observer @ 0x1000 (paddr[12]); reads 0 with PSLVERR when not built |

The UART wire protocol is ASCII (W <addr> <data>\n / R <addr>\n), 115200 8N1, decoded by uart_axil_bridge.

## 3.2	Key RTL blocks

Key RTL blocks

| File | Role |
| --- | --- |
| flows-rapids-beats/rtl/rapids_char_top.sv | FPGA pin-level top: pins to harness ports, LEDs and 7-seg. It no longer owns the host front-end: that moved into rapids_char_harness on 2026-09-23, so the harness simulation drives the board’s own launch path. |
| flows-rapids-beats/rtl/rapids_char_harness.sv | Synthesizable harness: the host front-end (uart_axil_bridge, the region decode, apb4_master, the DESC-LOAD path, the harness CSR register file and the atomic-launch kick sequencer; all CSR/DESC offsets are localparams here, the single source of truth for the host regmaps), the DUT rapids_beats_top, and the on-chip stimulus / checkers / memories. This is the cocotb DUT top. |
| flows-rapids-beats/rtl/rapids_char_genesys2_top.sv | Genesys 2 wrapper (MMCM 200→100 MHz) around rapids_char_top, default 8 channels. |

## 3.3	The DUT and the on-chip harness

The DUT is rapids_beats_top (u_dut), from projects/components/dma-ip/rapids/. Two build-time overrides matter for the char build: USE_AXI_MONITORS = 0 and GEN_MON = 0 — the in-core AXI/descriptor monitors and MonBus egress are compiled out so utilization is metered externally and 8-channel timing closes. Two more knobs, also default off, add the shared interface observers beside the DUT’s data ports (rapids TASK-001): USE_OBSERVERS = 1 builds axi4_intf_master_observer on the rd/wr data masters and axis4_intf_observer on s_axis / m_axis (port 0 sink ingress, port 1 source egress), each on its own 4 KB obs_regs window in region 3; OBS_ENABLE_MON_TAPS = 1 arms their monbus event taps (the meters and latency histograms count without them). Their monbus egress lands in an always-accept responder, like the DUT’s own; the bare axi_bus_meter / axis_bus_meter instances and the region-2 OBS_* CSRs the host tools read are unchanged. The harness also carries STREAM’s memory-latency model: axi_response_delay on the source read’s R channel and the sink write’s B channel, programmed by the RESP_DELAY CSR (region 2, 0xC4) in aclk cycles, so the same latency-vs-throughput knee STREAM reports can be swept here (--suite-delay).

Measured 2026-09-27 (synth, 8ch, 100 MHz): the pair adds 10,943 LUTs as instruments and 18,733 with the taps armed over the 60,651-LUT bare build, and timing closes in every variant (+3.703 / +0.869 ns synth WNS vs +3.683).

Because all data paths are on-chip, the board runs at line rate with no host bottleneck. The harness instantiates (all sharing LFSR 0xDEADBEEF, taps {32,22,2,1}, CRC-32 0x04C11DB7):

On-chip harness stimulus and checkers

| Block | Role |
| --- | --- |
| axis4_master_injector | drives the DUT s_axis (sink ingress stimulus) |
| axis4_slave_pattern_check | checks the DUT m_axis (source egress) |
| axi4_slave_rd_injector | backs the DUT m_axi_rd (DATA_WIDTH-bit source data; 256 on the Genesys 2 build) |
| axi4_slave_wr_crc_check | backs the DUT m_axi_wr (DATA_WIDTH-bit sink data verify) |
| sdpram_slave_axi4_axi4 ×4 | descriptor + control RAM per half (DUT reads, host writes) |
| axi_bus_meter / axis_bus_meter | prod/bp/starv/idle buckets + exact AXIS byte/packet counters |

## 3.4	Atomic launch

To start a measured run coherently, the host stages the kick parameters (CSR_KICK_MASK, CSR_KICK_BASE_*, CSR_KICK_STRIDE) and then writes a single CSR_GO, which arms the bus meters, starts the generator, and fires the channel kicks together (Chapter 5). A run latches PASS (0x0123) or FAIL (0x9999) on the LEDs and 7-seg once gen_done & src_idle & snk_idle.

# 4	Test Methodology

This chapter describes what is exercised on the FPGA block (Figure 2.1) and how throughput and data integrity are measured.

## 4.1	What is under test

The split RAPIDS beats DMA — the rapids_src_beats (read → AXIS) and rapids_snk_beats (AXIS → write) engines. Because all stimulus, checking, and memory are on-chip, the board runs at line rate and every measurement is taken at the DUT’s own AXI/AXIS ports.

## 4.2	What is measured, and how

What is measured

| Quantity | How it is obtained |
| --- | --- |
| Data integrity | Per-channel golden CRC-32: SINK WR_CRC[ch] vs GEN_EXP_CRC[ch]; SOURCE CHK_ACT_CRC[ch] vs RD_CRC[ch]; plus STATUS.data_error |
| Throughput | AXIS/AXI bus meters + exact byte/packet counters (OBS_SIN/SOUT_BYTES/PKTS) over the timed window |
| Utilization | OBS_{RD,WR,SIN,SOUT}_{PROD,BP,STARV,IDLE} buckets |
| Scaling | Repeat across the active-channel mask |
| Observer telemetry (USE_OBSERVERS=1) | Region-3 obs_regs by name: per-port cycle buckets, AXIS bytes/packets, AXI burst counts and latency histograms, tap drop counts (OBS_STICKY) |
| Sweep axes | --suite-channels, --suite-beats, --suite-descs (chains), --suite-xfer (burst AxLEN), --suite-delay (RESP_DELAY) – STREAM’s five knobs |

A measurement run uses atomic launch (Waveform 5.1): configure the DUT by name over APB, load descriptors into the per-half descriptor RAM, stage the kick parameters (CSR_KICK_MASK, CSR_KICK_BASE_*, CSR_KICK_STRIDE), then write a single CSR_GO that arms the bus meters, starts the generator, and fires the channel kicks together. CSR_OBS_TARGET = channels × beats freezes the meter window deterministically; the host then reads the CRCs and counters.

### 4.2.1	Figure 3.1: Measurement Pipeline

Measurement pipeline

Source: 02_measurement_pipeline.dot

## 4.3	Workloads

Characterization workloads

| Axis | Values |
| --- | --- |
| Channels | 1, 2, 4 (Nexys) / up to 8 (Genesys 2) |
| Beats | 1, 4, 8, 16 |
| Backpressure | off, on (SOURCE only, via chk_ready_en) |
| Seed | default, alternate (SINK only — SOURCE read-gen is fixed 0xDEADBEEF) |

## 4.4	The oracle

Every beat is checked against a deterministic golden CRC-32 (LFSR seed 0xDEADBEEF, taps {32,22,2,1}, poly 0x04C11DB7). The host’s rapids_char_golden.py computes the same value the on-chip checkers do; a pass is a match on both paths (make suite = 48/48).

The harness also judges each run on chip. Its run timer (char_run_timer, the module the STREAM harness uses) compares the active half’s input CRC with its output CRC per channel and latches PASS when the run ends. The host requires both: the golden match and the on-chip PASS.

## 4.5	Measurement pitfalls

- Arm every run — the bus meters have no auto-reset; use OBS_CTRL.arm or the CSR_GO write, or the window stays frozen on the first capture.

- SINK re-arm — the sink is fully trustworthy only on the first arm after a board reset; the runner pulses CHANNEL_RESET before each run.

- The generator self-CRC (GEN_EXP_CRC) is one side of the on-chip SINK verdict. Its valid flag read 0 intermittently in the 2026-07-15 board runs, before the generator start became a one-cycle pulse; no board run since shows it. A recurrence now fails the run instead of logging a warning.

# 5	Build and Run

All commands assume the Python environment is sourced (the Makefile hard-fails without REPO_ROOT):

cd /path/to/RTLDesignSherpa && source env_python
cd projects/fpga-systems/Genesys2/dma-ip/rapids_beats/flows-rapids-beats

## 5.1	Make targets (flows-rapids-beats/Makefile)

flows-rapids-beats make targets

| Target | What it does |
| --- | --- |
| make sim | cocotb harness self-check (sink + source, multi-channel) |
| make verify-sim | pre-bitstream sim gate (-k sink); bypass with BITSTREAM_SKIP_VERIFY=1 |
| make project / make synth | create Vivado project / synth-only + utilization |
| make bitstream | full synth + impl + bitgen (depends on verify-sim) |
| make program | flash the board over JTAG |
| make smoke | fast golden-validated UART confidence check |
| make suite | full UART sweep → JSON under reports/ |
| make characterize | full UART campaign on PORT |
| make flow | sim → bitstream → program → characterize |
| make utilization / make timing | print the latest reports |
| make clean / make clean-all | remove artifacts |

Knobs: CHANNELS (4 on nexys, 8 on genesys2 — passed as both the build generic and the host --channels, kept in lockstep), BEATS=8, PORT=/dev/ttyUSB1, BOARD=nexys|genesys2, BITSTREAM_SKIP_VERIFY=1.

make sim                                  # cocotb self-check
make bitstream                            # Nexys A7, 4 channels
make bitstream BOARD=genesys2 CHANNELS=8  # Genesys 2, 8 channels
make program
make smoke PORT=/dev/ttyUSB1
make suite CHANNELS=4

The Vivado build targets part xc7a100tcsg324-1, top rapids_char_top, with board-fit generics NUM_CHANNELS=4, SRAM_DEPTH=256, DESC_RAM_ENTRIES=256. The Genesys 2 build (BOARD=genesys2) is 8 channels at the 256-bit / 4 KB per channel design point: Makefile DATA_WIDTH=256 SRAM_DEPTH=128, exported as the RAPIDS_DATA_WIDTH / RAPIDS_SRAM_DEPTH generics and handed to verify-sim, so the sim gate builds the same geometry as the bitstream. Override both on the make line to build another point; the host reads the result back from the harness BUILD register and refuses a --channels that disagrees with it. Programming is pinned to a specific JTAG serial so the flash and the UART campaign land on the same board (override RAPIDS_CHAR_JTAG_SERIAL).

## 5.2	The host campaign (flows-rapids-beats/host/)

The port defaults to --port auto, which probes each /dev/ttyUSB* and keeps the board whose region-2 CTRL/ID reads 0x52415031 (“RAP1”).

run_characterization.py — the campaign runner (sink then source):

# Full campaign, verbose
./run_characterization.py --port /dev/ttyUSB1 --channels 4 --active 4 --beats 8 -v

# One path only
./run_characterization.py --sink-only   --channels 4
./run_characterization.py --source-only --channels 4

# Sink with the generator round-robining the active channels beat by beat
# (GEN_MODE.INTERLEAVE): every sink channel holds data at once, so the
# measured write window is the DUT's, not one channel's (rapids TASK-018)
./run_characterization.py --sink-only --interleave --channels 8 --active 8

# Smoke (both paths, quick) and full suite (matrix → JSON)
./run_characterization.py --smoke  --channels 4
./run_characterization.py --suite  --suite-channels 1,2,4 --suite-beats 1,4,8,16 \
        --suite-bp off,on --suite-seeds default,0x12345678

Exit codes: 0 all pass, 1 a self-check failed, 2 no UART link / wrong ID.

dump_status.py — a STATUS / totals / CRC snapshot (./dump_status.py --port /dev/ttyUSB1 --channels 4). Other host modules: descriptor_builder.py (256-bit descriptor builder), rapids_char_golden.py (standalone golden CRC), rapids_char_io.py (transport + autodetect).

## 5.3	Typical sequence

make sim                            # prove the harness self-check
make bitstream                      # (runs verify-sim first)
make program
make smoke PORT=/dev/ttyUSB1        # golden-validated link check
make suite CHANNELS=4               # full sweep -> reports/*.json

# 6	Harness CSR Configuration

## 6.1	Regions and by-name access

The host address splits into four regions by addr[19:16]:

Host address regions

| Region | Base | Contents |
| --- | --- | --- |
| DUT-REG | 0x0_0000 | APB into the RAPIDS DUT (SRC 0x0000, SNK 0x1000) |
| DESC-LOAD | 0x1_0000 | descriptor holding registers + kick |
| HARNESS CSR | 0x2_0000 | gen / chk / mem / mon / obs + status |
| OBSERVERS | 0x3_0000 | USE_OBSERVERS=1 builds only: obs_regs of the AXI observer @ 0x0000 and of the AXIS observer @ 0x1000 (projects/components/utility-ip/misc/rdl/obs/obs_regs.rdl, by name through obs_regs_top_regmap.py); PSLVERR + 0 when not built |

No PeakRDL RDL exists for the harness CSR. The registers are decoded by hand in rapids_char_top.sv (localparams) and mirrored by two hand-maintained Python regmaps, regenerated from the SV by flows-rapids-beats/bin/gen_rapids_harness_regmap.py:

- flows-rapids-beats/rtl/rapids_harness_csr_regmap.py

- flows-rapids-beats/rtl/rapids_harness_desc_regmap.py

The DUT (RAPIDS core) does use PeakRDL — projects/components/dma-ip/rapids/rtl/rapids_regmap.py — loaded by the host as two RegisterMap instances (SRC start_address=0x0000, SNK 0x1000, 13-bit APB) for by-name DUT config.

The identity register: region-2 CTRL/ID @ 0x000 reads 0x5241_5031 (“RAP1”) — the BUILD_ID equivalent used for ping() and port autodetect.

### 6.1.1	Waveform 5.1: Atomic Launch

Atomic launch sequence

Source: 01_atomic_launch.json

## 6.2	DESC-LOAD region (region 1, byte offsets)

DESC-LOAD registers

| Register | Offset | Field | Bits | Access | Description |
| --- | --- | --- | --- | --- | --- |
| DESC_WORD0..7 | 0x00–0x1C | value | [31:0] | RW | Words of the 256-bit descriptor |
| DESC_ADDR | 0x20 | value | [31:0] | RW | Target byte address in descriptor RAM |
| DESC_KICK | 0x24 | half | [0] | W | 0 = SRC, 1 = SNK — write issues one AXI4 write |
| DESC_STATUS | 0x28 | ok | [0] | R | Last BRESP was OKAY |

## 6.3	HARNESS CSR — control (region 2)

Harness CSR — control registers

| Register | Offset | Field | Bits | Access | Description |
| --- | --- | --- | --- | --- | --- |
| CTRL | 0x00 | cam_clear | [0] | W | Pulse: clear the sticky-error CAM |
| GEN_CTRL | 0x10 | gen_start | [0] | W | Pulse: start the AXIS generator |
| GEN_SEED | 0x14 | value | [31:0] | RW | LFSR seed (SINK honors it) |
| GEN_NBEATS | 0x18 | value | [31:0] | RW | Beats per channel |
| GEN_BPP | 0x1C | value | [31:0] | RW | Beats per packet |
| GEN_CHMASK | 0x20 | value | [N-1:0] | RW | Active-channel mask |
| GEN_TDEST | 0x24 | value | — | RW | AXIS TDEST |
| GEN_MODE | 0x28 | interleave | [0] | RW | 1: round-robin the active channels beat by beat (every sink channel holds data at once); 0: one channel’s whole run, then the next |
| CHK_CTRL | 0x30 | chk_start | [0] | RW | Pulse: start the checker |
| CHK_CTRL | 0x30 | chk_ready_en | [1] | RW | Level: checker backpressure enable |
| CHK_SEED | 0x34 | value | [31:0] | RW | Checker seed |
| MEM_CTRL | 0x40 | rd_crc_lfsr_reset | [0] | W | Pulse: reset read-CRC LFSR |
| MEM_CTRL | 0x40 | wr_crc_reset | [1] | W | Pulse: reset write-CRC |
| MON_BASE / MON_LIMIT | 0x50 / 0x54 | value | [31:0] | RW | DUT monitor egress window (host default 0x1000 / 0x4FFF) |
| MON_FLUSHWM | 0x58 | value | [15:0] | RW | Monitor flush watermark |
| CH_SEL | 0x60 | value | — | RW | Selects the channel for indexed CRC reads |
| OBS_CTRL | 0xC0 | arm | [0] | W | Pulse: re-arm the bus meters |
| RESP_DELAY | 0xC4 | rd_delay / wr_delay | [15:0] / [31:16] | RW | Memory-latency model (STREAM knob 5): per-beat hold, in aclk cycles, injected by axi_response_delay on the source read’s R channel and the sink write’s B channel. 0 = one register stage. |

## 6.4	Atomic-launch staging (region 2)

These raw offsets are declared in rapids_char_top.sv and mirrored in run_characterization.py (they are not in the by-name regmap):

Atomic-launch staging registers

| Register | Offset | Field | Bits | Description |
| --- | --- | --- | --- | --- |
| CSR_KICK_CFG | 0x64 | half / start_gen_on_go | [0] / [1] | Which half; start gen on GO |
| CSR_KICK_MASK | 0x68 | mask | [N-1:0] | Kick channel mask |
| CSR_KICK_BASE_LO/HI | 0x6C / 0x70 | value | [31:0] | Descriptor base address |
| CSR_KICK_STRIDE | 0x74 | value | [31:0] | Per-channel byte stride |
| CSR_GO | 0x78 | go | [0] | Pulse: arm meter + start gen + fire kicks |
| CSR_OBS_TARGET | 0x7C | value | [31:0] | Freeze window at N productive beats (0 = off) |

## 6.5	HARNESS CSR — status and observability

Harness CSR — status and observability

| Register | Offset | Field | Bits | Description |
| --- | --- | --- | --- | --- |
| STATUS | 0x80 | mon_irq, src_idle, snk_idle, gen_busy, gen_done, data_error, rd_mem_busy, wr_mem_busy | [0]…[7] | Harness state |
| GEN_BEATS_T / CHK_BEATS_T | 0x84 / 0x88 | value | [31:0] | Gen / checker beat totals |
| PKT_CNT | 0x8C | value | [31:0] | Packet count |
| RD_BEATS_T / WR_BEATS_T | 0x90 / 0x94 | value | [31:0] | Read / write beat totals |
| SRC_SCHERR / SNK_SCHERR | 0x98 / 0x9C | value | [N-1:0] | Per-channel scheduler error |
| GEN_EXP_CRC / CHK_ACT_CRC | 0xA0 / 0xA4 | value | [31:0] | CRC for the CH_SEL channel |
| RD_CRC / WR_CRC | 0xA8 / 0xAC | value | [31:0] | Read / write CRC for CH_SEL |
| *_VLD | 0xB0–0xBC | value | [N-1:0] | Per-channel CRC valid masks |
| OBS_RD_* / OBS_WR_* | 0x100–0x11C | value | [31:0] | AXI4 read / write meter buckets |
| OBS_SIN_* / OBS_SOUT_* | 0x120–0x13C | value | [31:0] | AXIS in / out meter buckets |
| OBS_SIN_BYTES_LO/HI, OBS_SIN_PKTS | 0x140–0x148 | value | [31:0] | Exact AXIS-in bytes / packets |
| OBS_SOUT_BYTES_LO/HI, OBS_SOUT_PKTS | 0x14C–0x154 | value | [31:0] | Exact AXIS-out bytes / packets |
| TIMER_STATUS | 0x160 | done, running, pass, crc_any_valid, crc_match, verdict_valid | [0]…[5] | On-chip run timer and CRC verdict (char_run_timer); pass is latched at done |
| TIMER_CYC_LO/HI | 0x164 / 0x168 | value | [31:0] | Run length in aclk cycles, GO to the OBS_TARGET-th output beat |
| TIMER_R_FIRST/R_LAST/W_FIRST/W_LAST | 0x16C–0x178 | value | [31:0] | Cycle stamps of the first and last input-side and output-side beat (low word) |
| CRC_VALID_MASK / CRC_MATCH_MASK | 0x17C / 0x180 | value | [N-1:0] | Per channel: both CRCs of the pair valid / valid and equal |

Meter buckets and byte/packet counters are cleared by CTRL.cam_clear and each run must be armed (via OBS_CTRL.arm or the CSR_GO write) or the window stays frozen on the first capture.

The run timer is the same char_run_timer the STREAM harness uses (projects/components/utility-ip/misc/rtl/char_run_timer.sv; BUILD.RUN_TIMER says it is built). GO clears it and starts it a cycle later; it stops when the output-side beat count reaches OBS_TARGET. Its verdict compares the active half’s input CRC with its output CRC: on SOURCE, RD_CRC against CHK_ACT_CRC; on SINK, GEN_EXP_CRC against WR_CRC. The host fails a run when the timer did not finish, when pass is clear, or when an active channel is missing from CRC_MATCH_MASK.

# 7	Simulation vs Silicon

RAPIDS characterization reaches the same golden CRC two ways — but, unlike the ddr2/stream siblings, the simulation does not run the host over a simulated UART. The UART bridge and CSR router live only in the board top (rapids_char_top.sv); the cocotb DUT top is the harness (rapids_char_harness), and the testbench drives its control surface directly.

## 7.1	The two paths

The two characterization paths

|  | Silicon | Simulation |
| --- | --- | --- |
| Config | host RegisterMap over UART → APB | cocotb RegisterMap over the harness APB port |
| Descriptors | host DESC-LOAD region → descriptor RAM | cocotb 256-bit AXI4 host-write port |
| Kick | host CSR_GO atomic launch | cocotb per-half kick windows |
| Data paths | all on-chip (harness gen/check/mem) | the same on-chip blocks, elaborated in Verilator |
| Pass criterion | golden CRC match (host rapids_char_golden.py) | golden CRC match (checker CRC == expected) |

The equivalence is by shared logic, not a shared transport: the same by-name DUT config (rapids_regmap.py), the same descriptor bytes (the TB’s create_descriptor() mirrors flows-rapids-beats/host/descriptor_builder.py), and the same golden CRC-32 asserted in both places.

## 7.2	The cocotb testbench

- DUT top: rapids_char_harness (Verilator). Runner flows-rapids-beats/dv/test_rapids_char_harness.py; TB class RapidsCharHarnessTB (flows-rapids-beats/dv/rapids_char_harness_tb.py).

- Two self-checks:

  - cocotb_test_sink_selfcheck — s_axis → sink → m_axi_wr; asserts wr_crc[ch] == gen_expected_crc[ch] for each active channel.

  - cocotb_test_source_selfcheck — m_axi_rd → source → m_axis; asserts chk_actual_crc[ch] == rd_crc[ch] and data_error == 0.

- Params: NUM_CHANNELS=8 (env TEST_NUM_CHANNELS), NUM_ACTIVE=4, NUM_BEATS=8, DATA_WIDTH=512, SRAM_DEPTH=512 when run bare; make sim / make verify-sim pass the build’s DATA_WIDTH / SRAM_DEPTH (256 / 128 on the Genesys 2) as TEST_DATA_WIDTH / TEST_SRAM_DEPTH. Waves via WAVES=1.

- Gate: make verify-sim runs the sink self-check before a bitstream build, so a broken elaboration is caught before a long Vivado run.

## 7.3	Why no UART-in-sim here

The harness data paths are entirely on-chip, so there is nothing for a simulated host to drive beyond the control surface — the TB pokes that surface directly (APB + descriptor RAM + kick) and reads back the same CRCs the board’s host reads over UART. If a UART-level equivalence sim is wanted later, the shared make_uart_channel / CocotbUartChannel collateral in bin/TBClasses/harness/ (used by ddr2/stream) can wrap the board top — but it is not wired today.

# 8	Troubleshooting

## 8.1	Current state

Silicon-validated. make smoke passes both paths; make suite passes 48/48 (3 channels × 4 beats × 2 backpressure × 2 seeds). Timing closes at 100 MHz (setup WNS +0.007 ns, 0 failing endpoints; hold WHS +0.011 ns). Post-route utilization (4-channel, xc7a100t-1): 37,555 LUTs (59.2%), 28,683 FF (22.6%), 22 BRAM (16.3%), 0 DSP. There is no separate KNOWN_ISSUES file — the known-state material lives in docs/rapids_beats_findings.md.

## 8.2	Known gotchas

Known issues and workarounds

| Symptom | Cause / workaround |
| --- | --- |
| SINK over-produces / desyncs | The generator start must be a one-cycle pulse, not a held level (a UART-held level runs the generator repeatedly). Fixed in RTL; residual: SINK is fully trustworthy only on the first arm after a board reset — a full SINK sweep needs a per-config reset. SOURCE re-arms cleanly. |
| Stale scheduler/descriptor state | reset_channels() pulses CHANNEL_RESET on both halves before each run — the sink does not self-clean after a transfer. |
| Meter reads 0 / frozen | The bus meters have no per-run auto-reset. Arm each run via OBS_CTRL.arm or the CSR_GO write; use CSR_OBS_TARGET = channels × beats for a deterministic freeze window. |
| gen_expected_crc_valid intermittently 0 | Seen only in the 2026-07-15 board runs, before the generator start became a one-cycle pulse; none since. The on-chip SINK verdict now needs it, so a recurrence fails the run (hardware CRC verdict FAIL) instead of warning. Treat it as a bug to find, not a flake. |
| hardware timer not done | GO did not start the run timer, or the output-side beat count never reached OBS_TARGET. Check OBS_TARGET = channels x beats and the beat totals. |
| Spurious read failure | csr_read() retries up to 3× on malformed/short UART frames. |

## 8.3	Cannot find the board

The USB-UART re-enumerates across reboots/replugs. --port auto (default) probes each /dev/ttyUSB* and picks the board whose region-2 CTRL/ID reads 0x52415031 (“RAP1”). make program and the UART campaign are pinned to the same board by JTAG serial (override RAPIDS_CHAR_JTAG_SERIAL) so the flash and the run land together.

## 8.4	Board-fit vs sim geometry

The harness sim defaults (SRAM_DEPTH = 4096, DESC_RAM_ENTRIES = 2048) inflate to ~148 BRAM tiles — over the 100T’s 135 — so the board build trims both to 256 (~22 tiles). This is a build-time generic only; the RTL is unchanged. Set CHANNELS once (it drives both the build generic and the host --channels) so the two never drift.

## 8.5	Three different “0x1000”s

Do not conflate them: the SNK APB half is at APB 0x1000; the host default MON_BASE is 0x1000; and the in-core DUT monitors (which would live in a 0x1000 monitor window) are compiled out (GEN_MON = 0) for this build.

## 8.6	Vivado-only package collisions

rapids_pkg vs stream_pkg enum-label/type clashes (RD_*, CH_*, channel_state_t, descriptor_t, …) surface only under Vivado’s flattened $unit scope — Verilator does not catch them. They are resolved by rapids_pkg:: qualification; if a Vivado elaboration errors on a duplicate enum label, look here first.