RTL Design Sherpa

STREAM on the Genesys 2 The FPGA System: Harness, Builds, Flow -- Version 1.1

September 2026

Table of Contents

1 Document Information	8

1.1 Revision History	8

1.2 How to read this book	9

2 The System	10

2.1 What it is	10

2.1.1 Figure 1.1: The bench	10

2.2 The board	11

2.3 The two USB chips	11

2.4 The stack, top to bottom	12

2.5 What the host sees	12

3 The Harness	14

3.0.1 Figure 2.1: The harness block map	14

3.1 The host front-end and the bridge	14

3.2 The harness CSR	15

3.3 The run timer and the verdict	16

3.4 Stimulus and the memory model	16

3.5 The three instruments	17

4 The Address Map	19

4.0.1 Figure 2.2: Host address map	19

4.1 Bridge slave map	19

4.2 Harness CSR detail	21

4.3 By-name access	24

5 One RTL, Three Builds	25

5.1 The rule	25

5.1.1 Figure 3.1: One source tree, three bitstreams	26

5.2 Why three, and why not two	26

5.3 What the builds measured	27

5.4 The knobs behind the rows	29

6 Host Tools and Flow	31

6.1 The shared flow	31

6.2 The host layers	31

6.2.1 Figure 4.1: What each build measures, end to end	33

6.3 One program, two targets	33

6.4 Where the results go	33

List of Figures

Figure 1.1: The bench	10

Figure 2.1: The harness block map	14

Figure 2.2: Host address map	19

Figure 3.1: One source tree, three bitstreams	26

Figure 4.1: What each build measures, end to end	33

List of Tables

Table 1: Table 0.1: Document information	8

Table 2: Table 0.2: Revision history	8

Table 3: Table 1.1: The part and what the builds use of it	11

Table 4: Table 1.2: The stack	12

Table 5: Table 2.1: Harness CSR, the registers that define a run	15

Table 6: Table 2.2: Stimulus and memory model	16

Table 7: Table 2.3: Bridge address map (from bridge_stream_mon_axil.toml)	19

Table 8: Table 2.4: harness_csr register map (base 0x0001_0000)	21

Table 9: Table 3.1: The three builds	25

Table 10: Table 3.2: Sign-off builds	27

Table 11: Table 3.3: The knobs	29

Table 12: Table 4.1: Flow targets (from make/fpga_flow.mk)	31

Table 13: Table 4.2: Per-build host programs	32

Table 14: Table 4.3: Reports	33

List of Waveforms

No waveforms in this document.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 1	Document Information

Table 0.1: Document information

| Field | Value |
| --- | --- |
| Title | STREAM on the Genesys 2 – The FPGA System |
| Version | 1.2 |
| Date | 2026-10-20 |
| Scope | The STREAM characterization and monitor-validation system: board, harness, the three builds, host flow |
| Not in scope | The STREAM core itself (HAS/MAS under projects/components/dma-ip/stream/docs/); the measured results (reports/, stable/MANIFEST.md) |
| Status | Current; written against the tree at the stream BUG-018 close (commit 022fa0e3e), run timer updated 2026-10-15 |

## 1.1	Revision History

Table 0.2: Revision history

| Version | Date | Change |
| --- | --- | --- |
| 1.0 | 2026-09-29 | First edition, after the component-level consolidation (one harness, one top, three parameter sets) and the mon and obs sign-off builds of September 2026. |
| 1.1 | 2026-10-15 | The run timer and CRC verdict are now char_run_timer from utility-ip/misc, shared with the RAPIDS harnesses: Figure 2.1 adds the block, Section 2 describes it, Figure 4.1’s verdict names it, and the address map no longer calls TIMER_CYCLES free-running. |
| 1.2 | 2026-10-20 | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |

## 1.2	How to read this book

Chapter 1 is the bench. Chapter 2 is the bitstream around the DUT, block by block, with the bridge address map every host tool depends on. Chapter 3 is why there are three bitstreams from one source and what each is for; it is the chapter to read before trusting a zero. Chapter 4 is the shared flow, the per-build host layers, and the one-program-two-targets rule. Every diagram is a Graphviz source under assets/graphviz/, rendered to PNG by regenerate_all_graphviz.sh.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	The System

## 2.1	What it is

The STREAM system on the Genesys 2 is two things sharing one bitstream source. It is a characterization harness: a synthesizable wrapper that feeds the STREAM scatter-gather DMA from an on-chip LFSR memory model, sinks its writes into a CRC checker, adds programmable memory latency, and counts every cycle of every interface. And it is a monitor-validation environment: the same wrapper carries the tally memories, capture SRAM and drains that let the host see every MonBus packet type from every agent in the DUT, whether the packets come from the in-core monitors or from interface observers standing outside the DUT.

Which of those it is on a given day is a question of which build is programmed. Chapter 3 is about that. The rest of this chapter is what does not change between them.

### 2.1.1	Figure 1.1: The bench

The bench: host, two USB chips, the FPGA

Source: 01_board_and_host.dot

## 2.2	The board

Digilent Genesys 2, Kintex-7 XC7K325T-2. The Nexys A7 version of this system (stream_char_top) was the original; the Genesys 2 top, stream_genesys2_top, adds an IBUFDS and an MMCM for the board’s 200 MHz LVDS system clock and drops the seven-segment display for the eight user LEDs. Everything below the top is the same RTL on both boards.

The harness clock is a parameter, not a constant. The MMCM runs its VCO at 1200 MHz and the harness clock is 1200 divided by CLKOUT0_DIVIDE: 12 gives 100 MHz, 15 gives 80, 20 gives 60. FPGA_CLK_HZ, which sizes the UART divisor, the heartbeat and the timer, is derived from the same parameter, so the two can never disagree. The mon sign-off build ran at 60 MHz, which is where the full-cone monitor build closed with margin; the perf build is the one that cares about the clock, and it is small enough to run faster.

Table 1.1: The part and what the builds use of it

| Resource | XC7K325T-2 | The builds |
| --- | --- | --- |
| Slice LUTs | 203,800 | mon sign-off 139,293 (68%); the two instruments together would need 217,761, which is why they are never built together (2026-10-03 re-measurement: 86,865 LUTs / 42.6%, WNS +1.914 ns; RTL has moved since the original sign-off) |
| Block RAM tiles | 445 | 20 to 24 on the current reports |
| User LEDs | 8 | status bank; PASS and FAIL are one-byte patterns |
| Reset | red CPU reset button, active low | synchronized in the top |

## 2.3	The two USB chips

The Genesys 2 carries JTAG on an FT2232 (serial 200300B818A0, the one FPGA_JTAG_SERIAL names, because a Nexys A7 shares the chain in this lab) and the UART on a separate FT232R that enumerates as its own /dev/ttyUSB*. The host tools probe for the harness rather than pin a device node. The link is 115200-8N1 with the ASCII W addr data / R addr protocol, one 32-bit AXI4-Lite access per line.

## 2.4	The stack, top to bottom

Table 1.2: The stack

| Layer | Module | Role |
| --- | --- | --- |
| Board top | stream_genesys2_top | LVDS clock in, MMCM, reset sync, LEDs, the geometry generics |
| Harness | stream_harness | UART to AXI4-Lite bridge, the generated one-to-many bridge, harness CSRs, descriptor RAM, tallies, capture SRAM, DMA slaves, latency model, observers |
| DUT | stream_top_ch8 | the STREAM DMA: APB configuration and kicks, eight channels (four on the obs build), one descriptor AXI4 master, read and write data masters, MonBus egress |

The harness is component-level: one stream_harness.sv, one stream_genesys2_top.sv, one harness_csr.sv, one axi_response_delay.sv, one set of generated bridges, one Tcl set and one XDC, shared by every build. Every geometry default the harness uses comes from stream_cfg_pkg, which is the single source the board top and the simulation share. A literal in the harness is how the simulation and the board once drifted apart, with the perf characterization run at sixteen outstanding transactions against a board built at two.

## 2.5	What the host sees

One AXI4-Lite master, fanned out by a generated bridge to every slave in the harness: the STREAM APB window, the harness CSRs, the descriptor RAM, the MonBus error drains, the two tally memories and their configuration, the slave-monitor and observer register blocks, and the capture SRAM. Chapter 2 has the map. Every register the host touches is reached by name through a register map generated from regs/harness_csr.rdl; a hand-kept Python table used to exist, drifted, and declared ten registers the RTL did not decode, which three host tools then read as zeros off a running board.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	The Harness

stream_harness is the bitstream minus the pins. Unlike the RAPIDS harness, whose host front-end was moved inside it late, this one was built with the UART bridge and the CSRs inside from the start, so the cosim and the board have always run the same host programs over the same bytestream. Figure 2.1 is the block map.

### 3.0.1	Figure 2.1: The harness block map

The harness: front-end, stimulus, DUT, the three instruments

Source: 02_harness_blocks.dot

## 3.1	The host front-end and the bridge

uart_axil_bridge turns the UART bytestream into one 32-bit AXI4-Lite master. A generated bridge (bridge_stream_mon_axil, from the bridge generator in projects/components/fabric-gen-ip/bridge) fans it out to every slave, converting protocol and width per port; it is regenerated by bin/regen_bridges.sh as the flow’s PREBUILD step, so a slave added to the map is a generator input, not hand-written glue. The complete host address map – bridge slaves and the harness CSR register detail – is drawn in The Address Map.

Every MonBus-carrying AXI path is 64 bits wide. The descriptor read path is a direct 256-bit AXI4 from the DUT to desc_ram, bypassing the bridge, and the data masters are 128-bit into the pattern generator and CRC checker.

## 3.2	The harness CSR

harness_csr is the host’s control and status surface, separate from the STREAM registers in the APB window. Its map is generated from regs/harness_csr.rdl; the RTL is hand-written against it and every external AXI4-Lite channel sits behind a skid buffer for timing.

Table 2.1: Harness CSR, the registers that define a run

| Register | Role |
| --- | --- |
| CTRL | start (opens the measurement window), clear_stats, freeze_trace (stops capture writes), soft_reset, cam_clear (clears every CAM in the DUT’s monitors and the tally) |
| STATUS | latched stream_irq, sticky any_error, sticky trace_overflow, clear_busy (poll to zero after clear_stats before the next capture) |
| DBG_WR_PTR, DBG_OVERFLOW | how much of the capture memory is written, and whether it overflowed |
| CRC_RD_EXPECTED, CRC_WR_EXPECTED, CRC_WR_COMPUTED | the source’s pseudo-random CRC, the expected sink CRC, and what the sink actually wrote |
| (no meter registers) | 0x100-0x128, where the retired harness bus meters were read back, is unallocated; the per-interface cycle classification is read from the observers’ own register blocks (OBS_STAT_SEL / OBS_STAT_DATA) and from STREAM’s in-core perf CSRs |

soft_reset clears every instrument in the harness, which is what lets one programming run a long sequence of host programs without inheriting state; the runner hooks on the host issue it between programs.

## 3.3	The run timer and the verdict

char_run_timer (u_char_run_timer) times each run and judges it. It lives in projects/components/utility-ip/misc/rtl/. The two Genesys 2 RAPIDS harnesses use the same module and the fabric benchmark harness will, so every harness built on the LFSR source and CRC sink measures and judges a run the same way.

- Start: the first m_axi_desc AR handshake after a TIMER_CTRL clear, so the run begins with the first descriptor fetch.

- Stop: the sink’s write-beat count reaching TIMER_EXPECTED_BEATS. TIMER_CYCLES freezes there, so the throughput is exact.

- Pass: sampled 5 cycles after the stop, once the last beat’s CRC has settled. Every channel that ran must have a valid sink CRC equal to its source CRC; a channel that never ran neither passes nor fails. No channel is masked out of the verdict.

- Stamps: the cycle of the first and last read and write beat, so the host can separate steady-state engine throughput from descriptor fill and drain.

TIMER_STATUS.pass and the per-channel CRC registers in Table 2.1 are this block’s outputs.

## 3.4	Stimulus and the memory model

Table 2.2: Stimulus and memory model

| Block | Role |
| --- | --- |
| axi4_dma_slaves | an LFSR read-source pattern generator behind m_axi_rd and a write-sink CRC checker behind m_axi_wr; both accept the next transaction on the last beat of the current one (an earlier two-state version inserted a dead cycle per burst and cost the datapath a phantom 6 percent) |
| axi_response_delay x2 | a programmable memory-latency model on the read and on the write path, the knob of the latency sweeps |
| desc_ram | the descriptor store the host preloads and the DUT fetches from directly |

## 3.5	The three instruments

The harness carries three ways of measuring, and Chapter 3 is about which build has which.

- Bus meters, always built, inside the interface observers and STREAM’s in-core datapath (the harness’s own axi_bus_meter blocks are retired). Per interface, every cycle of the window is productive, backpressure, starvation or idle. They sit outside the monitor gate on purpose: the perf build has no monitor and must still count. That separation regressed once, when the gate over-reached and the perf build reported zeros, and the monitor test now asserts the meters are non-zero in that flavour.

- Tallies, monbus_tally_axil twice. A direct count over every possible MonBus identity would be a 20,480-cell matrix of which about 245 tuples are legal; crossed with a 16-bit agent id it would be 99 percent empty. The tally instead loads the legal {agent, protocol, packet type, event} tuples into a CAM (MON_N_PROFILE slots, 32 on the sign-off builds) and counts densely, so the host can prove every agent emitted every packet type it may legally emit. stream_tally counts the in-core monitors, slave_tally the observers.

- Capture, comp_sram. The MonBus stream itself, compressed on the mon build, downloadable by the host and decoded against the cosim reference; stream_err and slave_err drain the error and IRQ FIFOs.

The two optional sources of packets are the DUT’s in-core monitors (USE_AXI_MONITORS) and the interface observers standing on the DUT’s masters and on the slaves (OBS_ENABLE_MON_TAPS). Both feed the same tally and capture machinery; they are never built together, and the reason is in Chapter 3.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	The Address Map

The host sees one 32-bit AXI4-Lite master. uart_axil_bridge turns the ASCII W addr data / R addr lines into that master, and the generated bridge_stream_mon_axil fans it out to every slave in the harness. The bridge is generated from rtl/bridges/configs/bridge_stream_mon_axil.toml; the table below is the as-built map from that file. Anything not listed decodes to a tied-off dma_axil port or returns DECERR.

### 4.0.1	Figure 2.2: Host address map

Host address map

Source: 05_address_map.dot

## 4.1	Bridge slave map

Table 2.3: Bridge address map (from bridge_stream_mon_axil.toml)

| Base | Size | Protocol | Slave | Purpose |
| --- | --- | --- | --- | --- |
| 0x0000_0000 | 8 KB | APB | stream_apb | STREAM DUT config and in-core monitor perf CSRs |
| 0x0001_0000 | 4 KB | AXIL | harness_csr | Harness control/status/timer/CRC/build identity |
| 0x0002_0000 | 64 KB | AXI4/AXIL | desc_ram | Descriptor preload: host writes 32-bit, STREAM reads 256-bit |
| 0x0003_0000 | 4 KB | AXIL | stream_err | STREAM in-core monitor error / IRQ FIFO drain |
| 0x0004_0000 | 256 KB | AXIL | stream_tally | In-core monitor packet tally histogram readback |
| 0x0008_0000 | 4 KB | AXIL | dma_axil | Tied off / DECERR – not used in this harness |
| 0x0009_0000 | 4 KB | AXIL | slave_err | Slave observer error / IRQ FIFO drain |
| 0x000C_0000 | 256 KB | AXIL | slave_tally | Slave observer packet tally histogram readback |
| 0x0010_0000 | 256 KB | AXIL | stream_tally_cfg | stream_tally legal-set CAM config + bin readback |
| 0x0014_0000 | 256 KB | AXIL | slave_tally_cfg | slave_tally legal-set CAM config + bin readback |
| 0x0018_0000 | 4 KB | APB | slvmon_apb | Slave-role observer (obs_regs_top) configuration |
| 0x0019_0000 | 4 KB | APB | obs_apb | Master-role observer (obs_regs_top) configuration |
| 0x001A_0000 | 64 KB | AXIL | comp_sram | Compressed MonBus capture memory, host download |

The two tally windows (stream_tally / slave_tally) are read-only for the host in normal use: records arrive directly from the observers or in-core monitors, and the host reads the resulting histogram. Their companion _cfg windows are where the host loads the legal {agent, protocol, type, event} CAM profile and where direct bin readback also lives.

## 4.2	Harness CSR detail

harness_csr occupies 0x0001_0000 and is the source of truth for run control, build identity, and the memory-latency knob. The map is generated from regs/harness_csr_regs.rdl and decoded by hand in rtl/harness_csr.sv; a test (bin/tests/test_harness_regmap.py) asserts every RDL register has a matching hardware decode.

Table 2.4: harness_csr register map (base 0x0001_0000)

| Offset | Register | Access | Content |
| --- | --- | --- | --- |
| 0x000 | CTRL | RW | start, clear_stats, freeze_trace, soft_reset, cam_clear pulses |
| 0x004 | STATUS | R | stream_irq, any_error, trace_overflow, clear_busy |
| 0x008 | DBG_WR_PTR | R | Capture SRAM write pointer (words) |
| 0x00C | DBG_OVERFLOW | R | Capture overflow flag |
| 0x010 | CRC_RD_EXPECTED | R | Aggregate expected read-source CRC |
| 0x014 | CRC_WR_EXPECTED | R | Aggregate expected write-sink CRC |
| 0x018 | CRC_WR_COMPUTED | R | Actual CRC computed by write sink |
| 0x01C | CRC_MATCH | R | match[0], valid[1] |
| 0x020 | SCRATCH | RW | Host ping / port autodetect (magic 0xC0FFEE5A) |
| 0x024 | BUILD_ID | R | 0x5354_5243 (“STRC”) – harness family |
| 0x028 | TIMER_CTRL | W | Timer clear pulse |
| 0x02C | TIMER_STATUS | R | done, running, pass |
| 0x030 / 0x034 | TIMER_CYCLES_LO / HI | R | Run length in cycles, first descriptor AR to the expected write beat (char_run_timer; frozen at the end of the run, not free-running) |
| 0x038 | TIMER_EXPECTED_BEATS | RW | Sink beat count that stops the timer |
| 0x03C | RESP_DELAY | RW | rd_delay[15:0], wr_delay[31:16] latency knobs |
| 0x040 - 0x05C | TIMER_R_FIRST_* / TIMER_R_LAST_* / TIMER_W_FIRST_* / TIMER_W_LAST_* | R | Per-engine first/last beat cycle stamps |
| 0x060 - 0x07C | CRC_RD_PER_CH0..7 | R | Per-channel read CRC |
| 0x080 - 0x09C | CRC_WR_PER_CH0..7 | R | Per-channel write CRC |
| 0x0A0 / 0x0A4 | CRC_VALID_MASK / CRC_MATCH_MASK | R | Per-channel valid/match bitmasks |
| 0x0D4 / 0x0D8 | DESC_SRAM_AR_HS / DESC_SRAM_R_HS | R | Descriptor SRAM AXIL handshake counters |
| 0x0E0 - 0x0FC | DESC_AR_HS, DESC_AR_STALL, DESC_R_HS, DESC_R_STALL, DESC_AW_HS, DESC_W_HS, DESC_B_HS, DESC_VR_LIVE | R | STREAM descriptor AXI4 handshake + live valid/ready snapshot |
| 0x100 - 0x1CF | – | – | Unallocated (retired AXI bus-meter readback) |
| 0x1D0 | BUILD_VERSION | R | Functional build number |
| 0x1D4 | BUILD_CONFIG | R | num_channels, error_flavor, use_monitors, gen_mon, data_width_b, main_cones |
| 0x1D8 | BUILD_N_PROFILE | R | Tally CAM legal-set size |
| 0x1DC | BUILD_CLK_HZ | R | Harness clock frequency in Hz |
| 0x1E0 - 0x1FC | COMP_TIER1_A/B/C, COMP_TIER0, COMP_CAM_MISS, COMP_DELTA_TS_OVF, COMP_EVENT_DATA_OVF, COMP_ED_DELTA_OVF | R | MonBus compressor statistics (0 unless USE_MON_COMPRESSION=1) |

## 4.3	By-name access

Host tools must resolve addresses by name, not by hardcoded offset:

- bin/harness_addrs.py H("CTRL") resolves harness CSR registers against rtl/harness_csr_regmap.py (generated from the RDL).

- bin/stream_addrs.py A("SCHED_CONFIG") resolves STREAM APB registers against the component-level stream_regmap.py.

- bin/obs_addrs.py O("MON_CTRL") resolves observer APB registers against obs_regs_top_regmap.py; use base=SLAVE_OBS_APB_BASE (0x0018_0000) for the slave-role observer.

This indirection is what keeps host scripts valid when a register moves inside its window.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	One RTL, Three Builds

## 5.1	The rule

There is one harness, one board top, one CSR block, one latency model, one set of generated bridges, one Tcl set and one XDC, all at component level. build-mon/, build-obs/ and build-perf/ each hold a Makefile of variables and nothing else; the flow logic is the shared make/fpga_flow.mk. Which build you want is a question about what is under test, and the answer is a row in this table:

Table 3.1: The three builds

| Build | USE_AXI_MONITORS | OBS_ENABLE_MON_TAPS | Channels | Measures |
| --- | --- | --- | --- | --- |
| mon | 1 | 0 | 8 | the in-core monitors: coverage, compression, error and fault probes |
| obs | 0 | 1 | 4 | the interface observers, watching the same DUT from outside it |
| perf | 0 | 0 | 8 | the datapath itself, with no instrument in the way |

Two near-identical harnesses, one per flavour, used to exist. They were retired because a variant that is a copy drifts: the board and the simulation once disagreed on the outstanding-transaction limit for exactly that reason. Now a variant is a parameter, the parameter is a generic handed to Vivado, and stream_cfg_pkg stays every build’s default; editing the package to retarget one build silently retargets the other two.

### 5.1.1	Figure 3.1: One source tree, three bitstreams

One RTL, the knobs, the three builds

Source: 03_build_variants.dot

## 5.2	Why three, and why not two

The mon and obs builds are complementary, not redundant. They exercise the monitor code from two directions: the in-core monitors see transactions from inside the DUT, the observers see them on the interfaces outside it, and both push their packets through the same tally and capture machinery. So build-obs has no in-core STREAM monitor at all, and a zero on any mon_* probe there is structural; build-mon has its observer taps off, and a zero on an observer probe there is equally structural. Reading either zero as a defect has cost an afternoon before, which is why the Makefile says so in capitals.

They cannot be one build. With every monitor cone and both observers at eight channels the design needs 217,761 LUTs against the 203,800 on the part, and the placer never runs. Even the obs build alone does not fit at eight channels: it reached 99.33 percent of the LUTs and “could not place all instances”, so obs is pinned at four (six does not route either). The eight-channel design point is perf’s requirement; the obs channel count is incidental, not a traded-away requirement.

The perf build exists so that a number about the datapath is a number about the datapath. Its bus meters are instantiated outside the monitor gate and keep counting with every instrument removed; that arrangement is what lets the design fit a small part, and it is guarded by a test after it regressed once.

## 5.3	What the builds measured

Table 3.2: Sign-off builds

| Build | Sign-off | Clock | Result |
| --- | --- | --- | --- |
| mon | 2026-09-09, stable/MANIFEST.md | 60 MHz | all cones plus the error flavour, compression built, 8 channels, MON_N_PROFILE=32: WNS +1.513 ns, 139,293 LUTs (68.35%), 0 failed nets; 512 packets in 516 slots on the board, 66.4% smaller than the raw encoding, CRC match, cosim reference within 4 points (2026-10-03 re-measurement: 66.0% / 1.02 slots/packet, within data-dependent noise; bit sha256 aceec688043ad630879779ddae211814fcc4dc457365d7c06ec95b8a4f348e6b, WNS +1.914 ns, 86,865 LUTs) |
| obs | 2026-09-07, stable-obs/MANIFEST.md | as built | 4 channels, 32-entry tally CAM, WNS +2.191 ns, archived the day a make clean-all destroyed the previous obs bitstream and it turned out obs had been synthesizing at 100.01 percent of the LUTs and placing by luck (2026-10-03 re-measurement: build-obs/results/obs_board_2026-10-03.txt, bit sha256 60696ace63fed2c77ad4d3f1f511d06dd85801c9b5a9dac1d4078454192bfa5f, WNS +3.925 ns, 73,809 LUTs; the observer instrument changed 2026-09-27, commit 78cddb5e2 — lite taps, no perf/debug cone, caps0 bit4/bit5 read 0) |
| perf | current reports on disk | as built | 68,139 LUTs (33.4%), 24 BRAM tiles in build-perf/fpga/reports; the sweeps behind reports/perf and reports/ext_addressing (2026-10-03 re-measurement: build-perf/results/perf_sweep_2026-10-03.{csv,json}, bit sha256 d26f9b7b3ac534a1507ab5a519e728d541b1045cf67000329fbedf77e3b9512c, WNS +1.175 ns, 64,305 LUTs) |

The stable/ directories hold exactly one build each, with the bitstream, the Vivado reports for that bitstream, the board results measured on it and a manifest naming the tree SHA it was built from. They exist because make clean-all in a build directory deletes tracked bitstreams and reports, and that has destroyed a verified bitstream more than once; stable/ is a sibling of the build directories, outside that blast radius.

## 5.4	The knobs behind the rows

Table 3.3: The knobs

| Knob | Where | What it selects |
| --- | --- | --- |
| STREAM_NUM_CHANNELS | build Makefile | channel count: 8 for mon and perf, 4 for obs |
| USE_AXI_MONITORS | build Makefile, DUT generic | the in-core rd/wr/descriptor monitors, their CAMs, and the compression pipeline (gated on this flavour only) |
| OBS_ENABLE_MON_TAPS | build Makefile, harness generic | arm the observers’ MonBus event taps; the observers’ meters and histograms count without them |
| MON_ERROR_FLAVOR, MON_CONES | build Makefile | which monitor cones are built; the sign-off is the union including error |
| MON_N_PROFILE | build Makefile | tally CAM depth |
| CLKOUT0_DIVIDE | board top parameter | 100, 80 or 60 MHz harness clock |
| stream_env | host | which build’s host/ layer goes on the Python path (default mon; perf sets it explicitly so a perf program cannot import mon’s copy of a sibling) |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Host Tools and Flow

## 6.1	The shared flow

The area Makefile is a dispatcher: make bitstream BUILD=obs runs the target in build-obs/, whose Makefile is variables plus an include of make/fpga_flow.mk. The flow’s targets are therefore identical across the three builds and across the other Genesys 2 and Nexys A7 systems that use it.

Table 4.1: Flow targets (from make/fpga_flow.mk)

| Target | What it does |
| --- | --- |
| lint, flat-filelist, lint-decl-order | Verilator lint of the whole harness against the expanded filelist, before any Vivado run |
| prebuild | the build’s PREBUILD step: here, regenerate the bridges |
| project, synth, bitstream | create the Vivado project; synthesis only with utilization and failing-path reports; the full flow with every report |
| bitstream-ila | the same design plus an ILA on the marked debug nets |
| sim | this build’s harness cosim (cocotb, Verilator) |
| program | flash the board, naming the file, falling back to the hold copy after a clean |
| run SEQUENCES="...", seq-list | drive the programmed board through the area’s sequences |
| utilization, timing | print the latest reports |
| keep | copy the bitstream to RDS_HOLD_DIR outside the repo and the reports to ../stable/ |
| clean, clean-build, clean-all | remove Vivado artifacts, cosim trees, everything; clean-all is what stable/ protects against |

## 6.2	The host layers

Two levels of Python. The component bin/ holds what every build uses: stream_device.py (the UART link and register access by name), harness_kick.py (staging the channel kicks), bus_meters.py (the meter packing, one definition shared with the cosim so both measure the same thing), tally.py and dump_monbus.py (reading the tallies and decoding a capture), characterization.py and stream_ext_suite.py (the campaign runners), stream_monitors.py (monitor and observer configuration), and the plotting scripts. Each build then has its own host/ with the programs that only make sense on that bitstream:

Table 4.2: Per-build host programs

| Build | Programs |
| --- | --- |
| perf | host_characterize, host_bus_meters, host_desc_perf, host_rw_perf, host_ext_char, host_ext_soak, host_probe_multichannel, host_verify_descriptors, host_status, host_perf_json_to_csv |
| mon | host_mon_coverage, host_mon_compress, host_mon_matrix, host_mon_err_probe, host_mon_fault_probe, host_reg_walk |
| obs | host_obs_campaign, host_obs_matrix, host_reg_walk |

stream_env selects which build’s host/ goes on the Python path. Its default is mon; build-perf sets it explicitly because without that a perf program importing a sibling from its own directory would get mon’s copy, and it would appear to work, since Python also puts the running script’s own directory first.

### 6.2.1	Figure 4.1: What each build measures, end to end

The three measurement paths

Source: 04_measurement_path.dot

## 6.3	One program, two targets

The same host program runs against the cosim and against the board. The cocotb harness testbench presents the UART bytestream to stream_harness exactly as the FT232R does, the harness carries the bridge and the CSRs, and the generics are the build’s. A monitor coverage result on the board is therefore compared with the same program’s result in cosim, and the sign-off manifest records both (512 packets in 516 slots on the board against 1.12 slots per packet in cosim, for the compression program). A host program that only works on one target is a defect in the harness or the program, not a difference in kind.

The sign-off discipline before a build is the same on every flavour: the FULL-level sim suites of all three builds green on a verified-clean tree, recorded in the manifest with the tree SHA, so the bitstream is reproducible from the recorded source.

## 6.4	Where the results go

Table 4.3: Reports

| Report | Source build | Content |
| --- | --- | --- |
| reports/perf | perf | utilization sweeps: channels, descriptors, sizes, latency (the CSV data under docs/data/) |
| reports/compression | mon | MonBus compression on the board against the cosim |
| reports/area | any | what the monitors and the readback logic cost in LUTs per channel count |
| reports/ext_addressing | perf | the extended row/column addressing campaign |
| stable/results | mon | the board sweeps measured on the sign-off bitstream |

Each report directory has its own README and, where a deliverable exists, the versioned DOCX and PDF built through the house pipeline.