RTL Design Sherpa

DDR2 / LPDDR2 Family Controller Micro-Architecture Specification 0.1

June 13, 2026

Table of Contents

1 Ddr2 Lpddr2 Mas Index	13

2 Document Information	13

2.1 DDR2 / LPDDR2 Family Controller Micro-Architecture Specification	13

2.2 Document Purpose	13

2.3 Document Scope	14

2.4 Revision History	14

3 Architecture and Datapath	15

3.1 Top-Level FUB Topology	15

3.2 Read / Write Datapath: AXI → addr-hash → CAM	16

3.2.1 Why Two CAMs, Not One	16

3.2.2 Why the XOR Happens Before the CAM	18

3.2.3 Why the AXI-side metadata stays in axi4_slave_fub	18

3.3 Clocking Topology	18

3.4 DFI Phase / Gear Topology	19

4 Top-Level Port List	19

4.1 Parameter-Dependent Widths (re-stated)	20

4.2 AXI4 Slave Port List	20

4.3 DFI v2.1 Master Port List	20

4.4 APB CSR Slave Port List	20

4.5 Clocks, Resets, IRQs, Debug	21

4.6 SystemVerilog Header Snippet	21

5 Clocks and Reset	22

5.1 Clock Domains	22

5.2 Reset Topology	22

5.3 CDC	23

5.4 Quiet Points	23

5.5 Reset Sequence	24

6 Top-Level Integration (ddr2_lpddr2_ctrl)	24

6.1 Purpose	24

6.2 Instantiation Inventory	25

6.3 Generate Blocks	26

6.4 What the Top Does Not Do	26

7 AXI4 Slave (axi4_slave_fub)	27

7.1 Purpose	27

7.2 External Interface (AXI side)	27

7.3 Internal Buffers	27

7.4 Data Flow	28

7.4.1 Write Path	28

7.4.2 Read Path	28

7.5 Outstanding Burst Tracking	29

7.6 Timing Budget	29

7.7 CSR Hooks	30

7.8 TODO (week-of-MAS work)	30

8 Address XOR / Hash (addr_mapper_fub)	30

8.1 Purpose	30

8.2 Mirror to the Python AddressMapping Class	31

8.3 Interface	31

8.4 Scheme Decode	31

8.4.1 Scheme 1: ROW_MAJOR	31

8.4.2 Scheme 2: BANK_INTERLEAVE	32

8.4.3 Scheme 3: XOR_HASH	32

8.5 Runtime Scheme Mux	32

8.6 Timing Budget	32

8.7 Verification	33

8.8 TODO	33

9 Read Command CAM (rd_cmd_cam_fub)	33

9.1 Purpose	34

9.2 Parameters	34

9.3 Storage Per Slot	34

9.4 Interfaces	35

9.4.1 Push (from axi4_slave_fub)	35

9.4.2 Scheduler Query	35

9.4.3 Issue Notification	35

9.4.4 Beat Return (from rd_data_path_fub)	36

9.5 Lookup Mechanism	36

9.6 Per-ID Beat Counting	36

9.7 Timing Budget	36

9.8 CSR Hooks	37

9.9 TODO	37

10 Write Command CAM (wr_cmd_cam_fub)	37

10.1 Purpose	37

10.2 Parameters	38

10.3 Storage Per Slot	38

10.4 Interfaces	39

10.4.1 Push (from axi4_slave_fub)	39

10.4.2 Scheduler Query	39

10.4.3 Issue Notification	39

10.4.4 Beat-Pull (to wr_data_path_fub)	39

10.4.5 Completion (from DFI write window expiration)	39

10.5 Difference From Read CAM	39

10.6 Timing Budget	40

10.7 CSR Hooks	40

10.8 TODO	40

11 Transaction Queue (txn_queue_fub)	41

11.1 Purpose	41

11.2 Relationship to the CAMs	41

11.3 Synthesis Parameters	42

11.4 Entry Format	42

11.5 Entry Lifecycle	44

11.6 Ports	45

11.6.1 Insert (from CAMs)	45

11.6.2 Parallel Read (to scheduler — every cycle)	46

11.6.3 Bank-State Watchpoint (from bank machines)	46

11.6.4 Scheduler Pick (from scheduler)	46

11.6.5 CAM Completion (from CAMs)	47

11.6.6 Backpressure Output	47

11.7 Insert Path	47

11.8 Row-Hit Cache Coherency	48

11.9 Age Counter Saturation	49

11.10 Storage Choice	49

11.11 CSR Hooks	50

11.12 Verification Notes (cocotb test plan)	50

11.13 Open Questions / Future Work	51

12 FR-FCFS Scheduler (scheduler_fub)	52

12.1 Purpose	52

12.2 Synthesis Parameters	52

12.3 Interface	53

12.3.1 Inputs	53

12.3.2 Outputs	54

12.4 Decision Pipeline	55

12.4.1 Stage 1 — Per-Entry Readiness (combinational, N parallel comparators)	56

12.4.2 Stage 2 — Priority Masking (combinational)	57

12.4.3 Stage 3 — Selection (combinational priority encoders)	58

12.4.4 Stage 4 — Command Formation (registered)	58

12.4.5 Refresh Window Behavior	59

12.5 Cache Coherency: row_hit_cached	59

12.6 Pipeline Staging	60

12.7 Strict In-Order Mode (collapse)	60

12.8 Critical-Path Analysis (rough budget at 500 MHz target, 2 ns cycle)	61

12.9 CSR Hooks	61

12.10 Verification Notes (cocotb test plan)	62

12.11 Open Questions / Future Work	63

13 HAPPY Page Predictor (page_predictor_fub)	63

13.1 TODO (Week-of-MAS work)	64

14 Bank Machine (bank_machine_fub)	64

14.1 Purpose	64

14.2 Synthesis Parameters	65

14.3 Instantiation Pattern	65

14.4 Block Internals	67

14.5 FSM States and Transition Triggers	68

14.6 Counter Pool	69

14.7 Outputs to Scheduler (accepts_*)	70

14.8 Refresh Handshake Protocol	70

14.9 Open-Row Change Broadcast	71

14.10 Per-Instance vs. Aggregated Storage	72

14.11 Per-Instance Area Estimate	73

14.12 CSR Hooks	73

14.13 Verification Notes (cocotb test plan)	74

14.14 Open Questions / Future Work	75

15 Cross-Bank Timers (xbank_timers_fub)	75

15.1 TODO (Week-of-MAS work)	76

16 Refresh Manager (refresh_mgr_fub)	76

16.1 TODO (Week-of-MAS work)	76

17 Init Engine (init_engine_fub)	77

17.1 TODO (Week-of-MAS work)	77

18 Power-State FSM (power_state_fub)	77

18.1 TODO (Week-of-MAS work)	77

19 Command Encoder (cmd_encoder_fub)	78

19.1 TODO (Week-of-MAS work)	78

20 DFI Gear (gear_dfi_fub)	78

20.1 TODO (Week-of-MAS work)	79

21 ODT Control (odt_ctrl_fub)	79

21.1 TODO (Week-of-MAS work)	79

22 Write Data Path (wr_data_path_fub)	80

22.1 TODO (Week-of-MAS work)	80

23 Read Data Path (rd_data_path_fub)	80

23.1 TODO (Week-of-MAS work)	80

24 AXI4 Slave Protocol	81

24.1 Scope of This Section	81

24.2 Supported Features (v1)	81

24.3 Optional Features (build-time)	81

24.4 Unsupported Features	81

24.5 TODO	81

25 DFI v2.1 Master Protocol	82

25.1 Scope	82

25.2 Sub-Interfaces Supported	82

25.3 Sub-Interfaces Not Supported (v1)	82

25.4 Phase / Gear Topology	82

25.5 TODO	83

26 APB CSR Slave Protocol	83

26.1 Protocol Compliance	83

26.2 Behavior	83

26.3 CDC	83

26.4 TODO	83

27 Register Map	84

27.1 Source of Truth	84

27.2 Staging vs. Live State	84

27.3 TODO	85

28 Runtime Overrides and Quiet Points	85

28.1 Override Categories	85

28.2 Quiet-Point Sequence	86

28.3 TODO	86

29 Family-Wide Config-Bit Applicability	86

29.1 Per-Bit Implementation Notes for DDR2 / LPDDR2	86

29.2 Soft-N/A Behavior Examples	87

29.3 TODO	87

30 Initialization Sequence	87

30.1 TODO	87

31 Refresh and Power-State Programming	88

31.1 TODO	88

32 Multi-Rank Programming	88

32.1 TODO	88

33 Error Handling	89

33.1 TODO	89

34 Build-Time Configuration Reference	89

34.1 TODO	90

35 Runtime Configuration Reference	90

35.1 TODO	90

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# 1	Ddr2 Lpddr2 Mas Index

Generated: 2026-06-16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	DDR2 / LPDDR2 Family Controller Micro-Architecture Specification

Document Number: DDR2-LPDDR2-MAS-001 Version: 0.1 Status: Draft - Skeleton Classification: Open Source - Apache 2.0 License

## 2.2	Document Purpose

This Micro-Architecture Specification (MAS) is the implementation-level companion to the DDR2 / LPDDR2 Hardware Architecture Specification (HAS). Where the HAS answers “what is the architecture and why,” this MAS answers “what is in the RTL and how does it work.”

The MAS is the document RTL designers, verification engineers, and integrators consult when writing or reviewing SystemVerilog code, building testbenches, and stitching the controller into an SoC.

Target Audience:

- RTL designers implementing the FUBs described in this document

- Verification engineers writing per-FUB cocotb tests

- Integrators stitching the controller into an SoC

- Software engineers writing low-level memory configuration code (driver authors, bring-up engineers)

Companion Documents:

- DDR2/LPDDR2 Family Controller HAS (../ddr2_lpddr2_has/) — high-level architecture and design rationale

- DDR2/LPDDR2 DFI BFM documentation (in RTLDesignSherpa-DV) — verification-side reference

## 2.3	Document Scope

The MAS is the micro-architecture view: per-FUB inputs/outputs, internal state, FSM diagrams, datapath timing, register-level pipeline stages, and per-block timing budgets. It assumes the reader has read the HAS for context.

This document covers:

- Top-level integration wiring (ddr2_lpddr2_ctrl)

- Each FUB’s interface signal list, internal storage, datapath flow, FSM, and timing budget

- AXI4 and DFI v2.1 wire-level protocol details specific to this controller

- APB programming interface and the full CSR register map

- Programming sequences (init, refresh, power-down, multi-rank, error handling)

- Build-time and runtime configuration reference

This document does not cover:

- Higher-level architectural rationale (see HAS)

- Verification plans or coverage models (see dv/README.md)

- Floorplan or layout guidance (project-specific)

- Bit-level CSR YAML — that lives in regs/ and is the source of truth for CSR generation

## 2.4	Revision History

| Version | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.1 | 2026-06-14 | RTL Design Sherpa | Initial skeleton — chapter outline, FUB list, port list scaffold |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Architecture and Datapath

This chapter is the implementation-level architectural orientation: the top-level FUB stitching, the read/write datapath flow, and the clocking topology. Per-FUB detail is in §2.

## 3.1	Top-Level FUB Topology

The controller’s top is ddr2_lpddr2_ctrl.sv — a pure structural integration of leaf FUBs. The 18 leaf FUBs are partitioned into 7 functional groups (see HAS §2.5 for the SWAG-level group description; this section is the wire-level topology view).

Top-Level FUB Topology

Source: 01_top_fub_topology.mmd

The salient property: there is exactly one stage where AXI-layer concepts (burst, ID, write strobe) cross into DRAM-layer concepts (rank, bank, row, column). That stage is addr_mapper → {rd,wr}_cmd_cam. Upstream of addr_mapper everything is AXI; downstream of the CAMs everything is DRAM.

## 3.2	Read / Write Datapath: AXI → addr-hash → CAM

The user-facing entry point is axi4_slave_fub. From the slave, the read and write paths share addr_mapper (one cycle) and then split into two separate CAMs:

AXI → addr_mapper → CAMs Datapath

Source: 02_axi_to_cam_datapath.mmd

### 3.2.1	Why Two CAMs, Not One

The CAM split happens here, not at the scheduler, because read and write CAMs hold different metadata and have different lifetimes:

| CAM | Key | Data | Cleared on |
| --- | --- | --- | --- |
| wr_cmd_cam | AXI ID | (rank, bank, row, col, burst_len, w_buf_ptr, strb_ptr) | B-channel response push |
| rd_cmd_cam | AXI ID | (rank, bank, row, col, burst_len, rid_counter, expected_beats) | last R beat of the burst |

A single unified CAM would either carry both metadata sets (wasteful) or force an awkward “is_write” predicate on every lookup. Keeping the CAMs separate also means the read and write paths can be sized independently — the read path typically deeper because read latency is longer; the write path can be shallower because write completion is fire-and-forget.

The two CAMs have visibly different lifecycles — note in particular that the write side stays “alive” through the post-issue write window (waiting on b_complete from xbank_timers) while the read side just counts beats:

CAM Slot Lifecycles (read vs. write)

Source: 03_cam_lifecycles.mmd

### 3.2.2	Why the XOR Happens Before the CAM

Address XOR / hash (addr_mapper) must happen before the CAM push because the CAM stores the post-decode (rank, bank, row, col) tuple — not the raw AXI address. Two reasons:

- Stable CAM key data. The CAM is queried by the scheduler in the (rank, bank) dimension — for example, “do any pending writes target rank 0 bank 3, row R?” Storing the post-decode tuple makes this a direct comparison; storing the raw AXI address would force the scheduler to re-XOR on every match.

- Address-mapping scheme is per-controller, not per-burst. The ADDR_MAP_TUNING.scheme_or field is global to the controller. If we stored raw addresses, a runtime scheme change (which can happen at any quiet point) would force a CAM walk to re-decode every entry. Storing the decoded tuple means a scheme change only affects new AXI bursts — old in-flight bursts continue with their decoded address. This matches the quiet-point semantics in HAS §6.3.

The addr_mapper is combinational — no pipeline stage between AXI intake and CAM push — so the timing budget for the XOR network is one cycle minus the AXI register-slice slack. See ch02_blocks/03_addr_mapper.md for the timing breakdown.

### 3.2.3	Why the AXI-side metadata stays in axi4_slave_fub

Some AXI fields (awcache, awprot, awqos, awregion, bid/rid) never need to cross into DRAM-layer state. These are held in small side tables inside axi4_slave_fub, indexed by the same AXI ID that keys the CAMs. The CAM holds the DRAM state; the side table holds the AXI state. On B/R completion, the CAM produces the completion signal and the side table produces the response metadata; the two are joined at the AXI return port.

This split keeps the CAM narrow (16 ID slots × ~36 bits in the default config) and pushes the AXI-specific clutter into the FUB that already owns the protocol.

## 3.3	Clocking Topology

Two external clocks; one CDC.

| Clock | Frequency Range | Drives |
| --- | --- | --- |
| mc_clk | 100–500 MHz | All DRAM-layer FUBs: scheduler, bank machines, refresh, init, power, cmd_encoder, gear_dfi, odt_ctrl, data paths, and axi4_slave |
| apb_pclk | 25–100 MHz | CSR slave only |

csr_apb_fub owns the single CDC between apb_pclk and mc_clk. CSR overrides are handed off across this CDC into a quiet-point staging register; the actual override-application happens at the next quiet point on mc_clk. Quiet-point detection is in power_state_fub (no commands in flight, no refresh in progress).

There is no CDC in the AXI4 datapath — axi4_slave_fub runs in the mc_clk domain. If the SoC’s AXI master is on a different clock, an external clock converter is required upstream of the controller.

## 3.4	DFI Phase / Gear Topology

The DFI v2.1 master interface is N_PHASES-deep. The MC clock drives one frame of N_PHASES DFI cycles per MC clock cycle. The gear_dfi_fub rewinds scheduler-issued commands and AXI data beats into the right per-phase slot per WRPHASE / RDPHASE. The scheduler does not see the phase dimension — its issue rate is one command per MC clock; the gear absorbs all of the phase-level scheduling.

The block diagrams and FSM diagrams referenced inline above are in assets/mermaid/ (TBD; placeholders until the per-block detail in §2 is complete).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Top-Level Port List

This chapter is the wire-level port list of the ddr2_lpddr2_ctrl top module — the exact signal names, directions, widths, and parameter dependencies that the integration script and SoC top-level use.

The implementation-level signal list is the same as the HAS overview view (HAS §2.4); this chapter restates it in the SystemVerilog-derivable form, with notes on bit-ordering, default tie-offs, and synthesis attributes where they matter.

The canonical machine-generated source is regs/ddr2_lpddr2_ctrl_ports.svh — what follows is the human-readable mirror.

## 4.1	Parameter-Dependent Widths (re-stated)

- AW = AXI_ADDR_WIDTH

- DW = AXI_DATA_WIDTH

- SW = DW/8

- IDW = AXI_ID_WIDTH

- DFI_DW = DFI_DATA_WIDTH

- DFI_AW = DFI_ADDR_WIDTH

- NP = N_PHASES

- NR = NUM_RANKS

- NB = NUM_BANKS

- BA = $clog2(NUM_BANKS)

## 4.2	AXI4 Slave Port List

See HAS §2.4 §1 for the table. The MAS adds nothing beyond what the HAS already published; the SystemVerilog declaration uses the same field names directly. AXI side-band signals (awqos, awregion, arqos, arregion) are observed but do not currently drive scheduler priority in v1; the wiring exists for the v2 QoS feature.

## 4.3	DFI v2.1 Master Port List

See HAS §2.4 §2. Additional MAS-level notes:

- The dfi_cs_n[NR] packed dimension is outer-most in SV declaration: output [NP-1:0][NR-1:0] dfi_cs_n;

- Per-rank dfi_cke[NR] is outer-most: output [NR-1:0] dfi_cke;

- The DDR2 ras/cas/we strobes are present even in LPDDR2 builds (driven all-1s, tied off by synthesis); the elaboration-time MEMTYPE parameter does not remove them from the port list, so the same SV header is usable for both flavors.

## 4.4	APB CSR Slave Port List

See HAS §2.4 §3. The APB clock and reset (apb_pclk, apb_presetn) are in the misc port group below; only the protocol signals are in this APB block.

## 4.5	Clocks, Resets, IRQs, Debug

See HAS §2.4 §4.

Additional MAS notes:

- mc_rst_n is asynchronous-assert, synchronous-deassert inside the controller (the deassert synchronizer lives in power_state_fub’s reset domain).

- apb_presetn is independently synchronized in csr_apb_fub.

- irq_* outputs are asserted in the mc_clk domain; the SoC’s interrupt controller is responsible for re-synchronizing if it runs on a different clock.

## 4.6	SystemVerilog Header Snippet

The first 30 lines of the generated port header — the rest is per the tables above:

module ddr2_lpddr2_ctrl #(
    parameter string MEMTYPE          = "DDR2",
    parameter int    N_PHASES         = 4,
    parameter int    NUM_RANKS        = 1,
    parameter int    NUM_BANKS        = 8,
    parameter int    AXI_DATA_WIDTH   = 64,
    parameter int    AXI_ID_WIDTH     = 4,
    parameter int    AXI_ADDR_WIDTH   = 32,
    parameter int    ROW_WIDTH        = 14,
    parameter int    COL_WIDTH        = 10,
    parameter int    DFI_DATA_WIDTH   = 32,
    parameter int    DFI_ADDR_WIDTH   = 14,
    parameter int    WRPHASE          = 0,
    parameter int    RDPHASE          = 0,
    parameter int    TXN_QUEUE_DEPTH  = 16,
    parameter string SCHEDULER_MODE   = "OOO",
    parameter int    LOOKAHEAD_DEPTH_MAX = 4,
    parameter string PAGE_POLICY      = "HAPPY_HYBRID",
    parameter int    PAGE_PREDICTOR_TABLE_BITS = 12,
    parameter int    AGE_MAX          = 256,
    parameter int    REFRESH_DEFER_MAX = 8,
    parameter string REFPB_POLICY     = "DARP",
    parameter string ODT_RULE_MULTIRANK = "JEDEC_DDR2",
    parameter int    SIM_INIT_SCALE   = 1
) (
    // Clocks and resets
    input  logic                       mc_clk,
    input  logic                       mc_rst_n,
    input  logic                       apb_pclk,
    input  logic                       apb_presetn,

    // AXI4 slave, DFI master, APB slave, IRQs, debug — see chapter tables.

    // ...
);

The full module declaration is in regs/ddr2_lpddr2_ctrl_ports.svh (machine-generated).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Clocks and Reset

## 5.1	Clock Domains

| Clock | Frequency | Polarity | Domain Members |
| --- | --- | --- | --- |
| mc_clk | 100–500 MHz | Posedge | All DRAM-layer FUBs, axi4_slave, data paths |
| apb_pclk | 25–100 MHz | Posedge | csr_apb_fub only |

The DFI v2.1 interface is driven on mc_clk (the controller side of DFI is always controller-clock; the DFI PHY runs at the DRAM clock and the gear mapping is the gear FUB’s responsibility).

## 5.2	Reset Topology

| Reset | Polarity | Type | Domain |
| --- | --- | --- | --- |
| mc_rst_n | Active-low | Async assert, sync deassert | mc_clk |
| apb_presetn | Active-low | Async assert, sync deassert | apb_pclk |

Each reset has its own 2-flop synchronizer for the deassert edge, instantiated in the FUB that owns the domain:

- mc_rst_n sync — in power_state_fub

- apb_presetn sync — in csr_apb_fub

Both synchronizers are clear-on-async-assert, hold-during-async-low. No reset-glitch filter is in the controller — the SoC’s PMU is expected to drive a clean reset.

## 5.3	CDC

Exactly one CDC in the controller: APB → MC for CSR overrides and MC → APB for CSR readback. Both are handled in csr_apb_fub. The two directions use different mechanisms:

- APB → MC (write side): 4-flop synchronizer per register field, with a per-field apb_write_strobe edge-detect on the MC side to latch the new value into the staging register. Staging held until next quiet point.

- MC → APB (read side): per-counter pulse synchronizers feed Gray-coded saturating counters; APB reads sample the Gray code and decode locally. The MC-side counters never wrap synchronously between reads.

Clock Domains and CDC Topology

Source: 04_clocks_cdc.mmd

## 5.4	Quiet Points

A configuration quiet point is the only safe moment to propagate CSR overrides from staging registers into the live datapath. Quiet points are defined by:

- txn_queue_fub is empty or all pending entries are in PENDING state (no ISSUED/COMPLETING)

- refresh_mgr_fub is in IDLE (not WAIT_BANK_GNTS, DO_REFRESH, DO_ZQCS)

- power_state_fub is in ACTIVE (not transitioning)

- No DFI command is in flight (the encoder’s per-phase pipeline has drained)

When all four hold, power_state_fub asserts quiet_point. Override-staging in csr_apb_fub watches this signal and commits all queued overrides in one MC-clock edge.

Software triggers quiet-point drain via CTRL.config_apply. The CSR slave returns STATUS.config_settled when commit is complete.

## 5.5	Reset Sequence

On power-on:

- mc_rst_n and apb_presetn are asserted (both low). PHY drives dfi_init_complete = 0.

- SoC deasserts apb_presetn. csr_apb_fub comes out of reset; CSRs are R/W-able. Software loads timing CSRs (MR0..MR3, timings, address-map scheme, refresh tuning).

- SoC deasserts mc_rst_n. All DRAM-layer FUBs come out of reset. Controller idles in POST_RESET state.

- Software writes CTRL.init_start = 1. init_engine_fub walks the per-memtype step table, driving dfi_reset_n, MR loads, ZQCAL, etc.

- On completion, init_engine_fub asserts irq_init_done (one cycle) and STATUS.init_done = 1. Controller transitions to ACTIVE.

- AXI traffic from the host is honored.

The minimum gap between apb_presetn and mc_rst_n deassertion is 16 mc_clk cycles (the synchronizer chain depth). Software does not need to wait — the controller stalls at POST_RESET until both resets are released.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Top-Level Integration (ddr2_lpddr2_ctrl)

Module: ddr2_lpddr2_ctrl.sv Location: rtl/top/ Category: TOP (Structural only — no behavioral logic) Status: Skeleton

## 6.1	Purpose

ddr2_lpddr2_ctrl is the controller’s top-level integration. It is pure structural wiring — every behavioral statement belongs in a FUB. The role of the top is:

- Declare the external port list (see §1.2)

- Instantiate every FUB

- Wire FUB↔FUB interfaces

- Fan-out per-rank / per-bank / per-phase signals where the FUB count is parametric

## 6.2	Instantiation Inventory

| Instance Name | FUB | Count | Notes |
| --- | --- | --- | --- |
| u_axi4_slave | axi4_slave_fub | 1 | Single AXI4 slave port |
| u_addr_mapper | addr_mapper_fub | 1 | Combinational; no clock |
| u_rd_cmd_cam | rd_cmd_cam_fub | 1 | Read-side CAM |
| u_wr_cmd_cam | wr_cmd_cam_fub | 1 | Write-side CAM |
| u_txn_queue | txn_queue_fub | 1 | Unified pending queue |
| u_scheduler | scheduler_fub | 1 |  |
| u_page_predictor | page_predictor_fub | 0 or 1 | Conditional on PAGE_POLICY == HAPPY_HYBRID |
| u_bank_machine[r][b] | bank_machine_fub | NUM_RANKS × NUM_BANKS | generate block, two-dimensional |
| u_xbank_timers | xbank_timers_fub | 1 | Internally per-rank tRRD/tFAW + global tCCD |
| u_refresh_mgr | refresh_mgr_fub | 1 |  |
| u_init_engine | init_engine_fub | 1 |  |
| u_power_state | power_state_fub | 1 |  |
| u_cmd_encoder | cmd_encoder_fub | 1 | Parameterized by MEMTYPE |
| u_gear_dfi | gear_dfi_fub | 1 |  |
| u_odt_ctrl | odt_ctrl_fub | 1 |  |
| u_wr_data_path | wr_data_path_fub | 1 |  |
| u_rd_data_path | rd_data_path_fub | 1 |  |
| u_csr_apb | csr_apb_fub | 1 | Only FUB in apb_pclk domain |

## 6.3	Generate Blocks

The only parametric-fanout instances are the bank machines, fan-out arrays of per-rank CSRs, and per-rank ODT/CKE outputs:

generate
    for (genvar r = 0; r < NUM_RANKS; r++) begin : g_rank
        for (genvar b = 0; b < NUM_BANKS; b++) begin : g_bank
            bank_machine_fub #(.ROW_WIDTH(ROW_WIDTH)) u_bank_machine (
                .mc_clk         (mc_clk),
                .mc_rst_n       (mc_rst_n),
                .scheduler_if   (sched_to_bank[r][b]),
                .refresh_if     (refresh_to_bank[r][b]),
                .xbank_if       (xbank_to_bank[r][b]),
                .state_o        (bank_state[r][b]),
                .open_row_o     (bank_open_row[r][b]),
                .last_ref_age_o (bank_last_ref_age[r][b])
            );
        end
    end
endgenerate

The for (genvar r) outer loop is the per-rank dimension; the inner is the per-bank dimension. The aggregation of bank_state[r][b] into the scheduler input is a structural concatenation only — no behavioral aggregation in the top.

## 6.4	What the Top Does Not Do

- No combinational logic beyond signal renaming or pure concatenation

- No CSR field expansion (the CSR slave handles its own field encoding)

- No clock gating (clock-gate insertion is a synthesis-script concern, not RTL)

- No reset synchronization (handled per-domain in power_state_fub and csr_apb_fub)

- No assign statements except for tying-off optional DFI signals (DDR2 vs LPDDR2 strobe ties)

This intentional poverty of the top file makes it the obvious place for the structural-only synthesis sanity check: “every line in this file is either an instantiation or a port-mapping; if a line is anything else, it is a bug.”

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	AXI4 Slave (axi4_slave_fub)

Module: axi4_slave_fub.sv Location: rtl/fub/ Category: FUB Parent: ddr2_lpddr2_ctrl Status: Skeleton

## 7.1	Purpose

axi4_slave_fub is the AXI4 host-facing protocol engine. It accepts AW/W/AR transactions from the SoC, hands off the address to addr_mapper_fub for decode, and pushes the resulting transaction into the appropriate CAM (wr_cmd_cam_fub or rd_cmd_cam_fub). On B and R returns, it pulls completion signals from the CAMs and merges them with AXI-side metadata to produce protocol-compliant responses.

The FUB owns the AXI ID side table — the per-ID set of fields (awcache, awprot, awqos, awregion, bid echo, rid echo) that never need to cross into the DRAM-layer state.

## 7.2	External Interface (AXI side)

Per HAS §2.4 §1 — full AW/W/B/AR/R channel signal list. No additions in MAS.

## 7.3	Internal Buffers

| Buffer | Depth | Width | Purpose |
| --- | --- | --- | --- |
| aw_buf | TXN_QUEUE_DEPTH/2 (default 8) | AW + IDW + 8(len) + 3(size) + 2(burst) + 4(cache) + 3(prot) + 4(qos) + 4(region) | Pending AW until CAM push |
| w_buf | TXN_QUEUE_DEPTH × max(burst_len) | DW + SW + 1(last) per beat | Write data staging |
| ar_buf | TXN_QUEUE_DEPTH/2 (default 8) | same AW fields | Pending AR until CAM push |
| b_response_fifo | small (4) | IDW + 2(resp) | Pending B responses |
| r_response_fifo | small (4) | IDW + DW + 2(resp) + 1(last) per beat | Pending R responses |
| id_side_table | 2^IDW | 4(cache) + 3(prot) + 4(qos) + 4(region) | Per-ID AXI metadata, indexed by awid/arid |

The buffers above are FUB-internal; they do not appear in the architecture-level transaction queue (txn_queue_fub). The transaction queue holds the DRAM-layer view of pending work; these buffers hold the AXI-layer view of in-flight bursts.

## 7.4	Data Flow

### 7.4.1	Write Path

- AW intake: capture aw* fields; push to aw_buf. Compute outstanding-count and assert awready if aw_buf not full.

- W intake: accept w* beats into w_buf. Each W beat carries wstrb and wlast. The W path does not wait for AW — W can lead AW or trail it within AXI ordering rules.

- CAM push: when both aw_buf head and the matching w_buf head are present (or when a write-only / address-only burst’s prerequisites are satisfied), push to wr_cmd_cam_fub with:

  - key = awid

  - data = (rank, bank, row, col, burst_len, w_buf_ptr, strb_ptr)

  - where (rank, bank, row, col) come from addr_mapper_fub (combinational, same cycle)

- Scheduler issue: when scheduler picks this entry, the FUB hands w_data_path_fub the w_buf_ptr and strb_ptr for it to walk the data beats.

- B response: when wr_cmd_cam_fub reports the entry complete (last W beat issued + tDFI write to DFI), the FUB pops the AXI ID, joins with id_side_table[id].axi_metadata, and pushes b_response_fifo.

- B emit: standard B-channel handshake; pop b_response_fifo.

### 7.4.2	Read Path

- AR intake: capture ar* fields; push to ar_buf. Assert arready if not full.

- CAM push: pop ar_buf head; push to rd_cmd_cam_fub with:

  - key = arid

  - data = (rank, bank, row, col, burst_len, rid_counter, expected_beats)

  - where (rank, bank, row, col) come from addr_mapper_fub

  - rid_counter is the AXI ID echo

- Scheduler issue: when scheduler picks this entry, it commands the issue of RD / RDA. rd_data_path_fub watches for the corresponding DFI rddata beats.

- R response: as each R beat completes, rd_data_path_fub strobes rd_cmd_cam_fub to decrement expected_beats; the CAM emits r_beat_to_axi with rid, beat data, rlast when the last beat arrives.

- R emit: standard R-channel handshake; pop r_response_fifo.

## 7.5	Outstanding Burst Tracking

The FUB enforces:

| Limit | Constraint |
| --- | --- |
| Total in-flight reads | ≤ rd_cmd_cam_fub depth (default 16) |
| Total in-flight writes | ≤ wr_cmd_cam_fub depth (default 16) |
| Per-ID in-flight reads | ≤ MAX_PER_ID_READS parameter (default 4) |
| Per-ID in-flight writes | ≤ MAX_PER_ID_WRITES parameter (default 4) |

If AXI_OOO_ACROSS_IDS == false, the per-ID counts reduce to “one in-flight per ID” — the AXI completion is strictly in-order per ID, but the issue order at the scheduler is still OoO.

## 7.6	Timing Budget

| Path | Budget |
| --- | --- |
| awvalid → awready (combinational) | ≤ 0.7 of cycle |
| aw_buf push to addr_mapper valid | 0 cycles (combinational) |
| addr_mapper valid to CAM push | 1 cycle |
| wr_cmd_cam push to scheduler-visible | 1 cycle |
| Worst path: awvalid → CAM-push register | 2 cycles |

The two-cycle AXI→CAM latency is the limit on AXI sustained issue rate; with default config it allows 1 burst per 2 cycles, well within the AXI burst rate at 200 MHz.

## 7.7	CSR Hooks

- STATUS.axi_inflight_rd (R only) — current count of in-flight read bursts

- STATUS.axi_inflight_wr (R only) — current count of in-flight write bursts

- STATUS.axi_id_table_occ (R only) — number of distinct AXI IDs currently in the side table

- OBS_AXI_R_LATENCY_AVG, OBS_AXI_W_LATENCY_AVG — driven from R/B-channel sampling in this FUB

## 7.8	TODO (week-of-MAS work)

- FSM diagram for the AW/W join and the AR intake (mermaid)

- B/R response-ordering corner cases (per HAS §3.1 OoO-across-IDs)

- Waveform examples (wavedrom) for back-pressure scenarios

- Detailed table of id_side_table field widths and reset values

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Address XOR / Hash (addr_mapper_fub)

Module: addr_mapper_fub.sv Location: rtl/fub/ Category: FUB (combinational; no clock) Parent: ddr2_lpddr2_ctrl Status: Skeleton

## 8.1	Purpose

addr_mapper_fub decodes a flat AXI_ADDR_WIDTH-bit address into the DRAM-layer (rank, bank, row, column) tuple per the currently-active address-mapping scheme. The XOR / hash happens here — upstream of the read/write CAMs — so that the CAMs store the decoded tuple and not the raw AXI address.

The FUB is pure combinational — there is no clock, no flop, no FSM. The output is valid the same cycle as the input. The path is one of the timing-critical signatures in the design and is treated as a multi-cycle path only if synthesis cannot meet timing.

## 8.2	Mirror to the Python AddressMapping Class

This RTL is the mirror of the AddressMapping Python class in the DV repository (CocoTBFramework.components.dfi.address_mapping). Both produce bit-for-bit identical decoded outputs for the same input. The mirror is enforced by:

- Same scheme list (ROW_MAJOR, BANK_INTERLEAVE, XOR_HASH)

- Same field-bit positions per scheme

- Same XOR_HASH seed and tap matrix

The shared decode function eliminates a common class of bugs where simulation passes (because the BFM decodes one way) but RTL fails (because the controller decodes a different way).

## 8.3	Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_addr_i | input | AW | Flat AXI byte address |
| scheme_active_i | input | 2 | Runtime scheme select from ADDR_MAP_TUNING.scheme_or |
| xor_seed_i | input | 8 | Runtime seed for XOR_HASH (from CSR) |
| bg_field_pos_i | input | 3 | Bank-group field bit position (unused in DDR2/LPDDR2; reserved) |
| rank_o | output | $clog2(NR) | Decoded rank |
| bank_o | output | BA | Decoded bank |
| row_o | output | ROW_WIDTH | Decoded row |
| col_o | output | COL_WIDTH | Decoded column (byte-aligned within burst) |

## 8.4	Scheme Decode

### 8.4.1	Scheme 1: ROW_MAJOR

The simplest mapping; one contiguous row per rank/bank stride. Bit layout:

| ... reserved ... | rank | row [ROW_WIDTH-1:0] | bank [BA-1:0] | col [COL_WIDTH-1:0] | byte_in_dq |

The byte-in-DQ field is the controller’s column-byte offset and is consumed by wr_data_path_fub for wstrb masking; it is not part of the decoded col_o (which is column-word aligned).

ROW_MAJOR is best for sequential streaming workloads where a long burst stays in one row.

### 8.4.2	Scheme 2: BANK_INTERLEAVE

| ... reserved ... | rank | row [ROW_WIDTH-1:0] | col_hi [...] | bank [BA-1:0] | col_lo [...] | byte_in_dq |

Bank bits are placed between column-low and column-high bits, so consecutive cache lines round-robin across banks. This is the recommended default for general-purpose workloads with mixed access patterns; it produces higher bank-parallelism in the scheduler.

### 8.4.3	Scheme 3: XOR_HASH

Same layout as BANK_INTERLEAVE, but the bank field and (a subset of) the column-high bits are XOR-hashed with row bits:

bank_hashed[i] = bank_raw[i] XOR row_raw[i] XOR row_raw[i + BA] XOR xor_seed[i]

The XOR_HASH scheme defeats pathological row-conflict patterns (e.g., a power-of-two stride that lands on the same bank in BANK_INTERLEAVE). The seed xor_seed_i is runtime-modifiable so a bring-up engineer can change the hash without rebuilding.

## 8.5	Runtime Scheme Mux

The output is multiplexed across the synthesized schemes per scheme_active_i. Only schemes in ADDR_MAP_SCHEMES_SYNTH are routed to the mux; un-synthesized schemes tie off and writing them via CSR returns pslverr.

axi_addr_i [AW-1:0]
                       │
       ┌───────────────┼───────────────────┐
       ▼               ▼                   ▼
  ROW_MAJOR     BANK_INTERLEAVE      XOR_HASH
  decoder        decoder              decoder
       │               │                   │
       └───────┬───────┴───────────────────┘
               ▼
           scheme_active_i ──► 3:1 mux
               │
               ▼
        (rank, bank, row, col) ────► CAMs

## 8.6	Timing Budget

The decoder is purely combinational. The longest path in XOR_HASH is the XOR of (a) several address bits, (b) the seed, and (c) the row bits — about 5–6 levels of XOR gates plus a 3:1 mux. At 500 MHz this is comfortably within 0.5 ns of MC clock cycle time.

| Path | Estimate (gate-level) |
| --- | --- |
| axi_addr_i → bank_o (ROW_MAJOR) | 2 levels of MUX |
| axi_addr_i → bank_o (BANK_INTERLEAVE) | 2 levels of MUX |
| axi_addr_i → bank_o (XOR_HASH) | 5 levels XOR + 2 MUX |
| axi_addr_i → row_o | 1 level MUX |

If XOR_HASH does not meet timing at the target frequency, the FUB can be re-fitted with a single pipeline stage (the seed-XOR can absorb a flop without changing the per-tuple invariant — the seed only changes on quiet points anyway).

## 8.7	Verification

The RTL decoder and the Python AddressMapping class are co-validated by a small cocotb test that picks 10000 random addresses, asks each side to decode, and asserts bit-equality on the (rank, bank, row, col) tuple. This is the primary regression for addr_mapper_fub.

## 8.8	TODO

- Tap matrix for XOR_HASH — the exact bit mapping

- Synthesis hint for the XOR_HASH critical path

- Edge cases for AW < (ROW_WIDTH + COL_WIDTH + BA + clog2(NR)) (under-decoded address — should it tie-off or error?)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Read Command CAM (rd_cmd_cam_fub)

Module: rd_cmd_cam_fub.sv Location: rtl/fub/ Category: FUB Parent: ddr2_lpddr2_ctrl Status: Skeleton

## 9.1	Purpose

rd_cmd_cam_fub is the read-side content-addressable storage of in-flight read commands. It holds the DRAM-layer view (rank, bank, row, col) of every read burst that has been pushed by axi4_slave_fub and not yet fully returned on the AXI R-channel.

The CAM is keyed by AXI ID, allowing the scheduler to find all reads to a given (rank, bank) without iterating the queue and allowing rd_data_path_fub to find the right entry to decrement when a read beat returns.

The read CAM and write CAM are intentionally separate FUBs — see §1.1 for the rationale.

## 9.2	Parameters

| Parameter | Default | Purpose |
| --- | --- | --- |
| RD_CAM_DEPTH | 16 | Number of in-flight read slots |
| IDW | AXI_ID_WIDTH | Key width |
| BURST_LEN_WIDTH | 8 | Burst-length field width |

## 9.3	Storage Per Slot

| Field | Width | Description |
| --- | --- | --- |
| valid | 1 | Slot occupied |
| axi_id | IDW | Key — the originating AXI ID |
| rank | $clog2(NR) | Decoded rank |
| bank | BA | Decoded bank |
| row | ROW_WIDTH | Decoded row |
| col_start | COL_WIDTH | Starting column |
| burst_len | BURST_LEN_WIDTH | Total beats in burst |
| beats_returned | BURST_LEN_WIDTH | Beats already returned on R |
| issued | 1 | Scheduler has picked this entry |
| last_beat_at | small | Cycle of last beat return (for latency telemetry) |

Total per-slot width (default config): ~46 bits.

Total storage (16 entries × 46 b): ~92 bytes — small enough to fit comfortably in distributed flops with no SRAM macro.

## 9.4	Interfaces

### 9.4.1	Push (from axi4_slave_fub)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| push_valid | input | 1 | New read burst arriving |
| push_ready | output | 1 | CAM has a free slot |
| push_id | input | IDW | AXI ID |
| push_rank | input | $clog2(NR) | from addr_mapper |
| push_bank | input | BA |  |
| push_row | input | ROW_WIDTH |  |
| push_col | input | COL_WIDTH |  |
| push_len | input | BURST_LEN_WIDTH |  |

### 9.4.2	Scheduler Query

The scheduler queries the CAM to: - Find all unissued entries matching a target (rank, bank) — for next-issue selection - Find row-hit candidates (entries whose row == open_row[rank][bank])

| Signal | Direction | Description |
| --- | --- | --- |
| q_rank_i | input | Query rank |
| q_bank_i | input | Query bank |
| q_row_i | input | Query row (for row-hit check) |
| match_unissued_o | output | One-hot vector of slots matching (rank, bank) and not issued |
| match_rowhit_o | output | One-hot vector of slots matching (rank, bank, row) and not issued |
| match_data_o | output | The selected slot’s payload (rank, bank, row, col, len) |

### 9.4.3	Issue Notification

When the scheduler picks an entry from this CAM:

| Signal | Direction | Description |
| --- | --- | --- |
| issued_slot_i | input | Slot index to mark as issued |
| issued_we_i | input | Mark-issued strobe |

### 9.4.4	Beat Return (from rd_data_path_fub)

| Signal | Direction | Description |
| --- | --- | --- |
| beat_id_i | input | AXI ID of returning beat |
| beat_we_i | input | Beat-arrived strobe |
| entry_complete_o | output | One-hot vector of slots that just completed (last beat of their burst) |

On entry-complete the slot’s valid is cleared in the next cycle.

## 9.5	Lookup Mechanism

The CAM uses a parallel match-line structure: each slot has a small comparator for {rank, bank} that drives a per-slot match wire. The scheduler reads the full match-vector and picks among matches using a priority encoder (age-priority — oldest slot wins ties).

For the typical RD_CAM_DEPTH of 16 with BA=3, $clog2(NR)=1..2 (1 for single-rank, 2 for quad-rank), this is 16 4–5-bit equality comparators in parallel — trivial silicon.

## 9.6	Per-ID Beat Counting

Because reads return in-order per AXI ID (an AXI ordering rule), the read CAM does not need a per-beat slot table. A slot’s beats_returned increments on each beat that returns with that ID, and when beats_returned == burst_len the slot completes and clears.

If AXI_OOO_ACROSS_IDS == false (rare), the SoC has asked for in-order return across all IDs, and the read CAM enforces this by issuing a head-of-queue priority hint to the scheduler — entries are still cleared in arrival order. The OoO-across-IDs default is true; per-ID in-order is the AXI default and is preserved.

## 9.7	Timing Budget

| Path | Budget |
| --- | --- |
| push_valid → slot occupy (1 cycle) | 1 cycle |
| q_rank_i / q_bank_i → match_*_o | combinational |
| beat_id_i → beats_returned[slot]+1 | 1 cycle |
| entry_complete_o → slot clear | 1 cycle |

The combinational match path is on the scheduler’s critical loop and is shared with wr_cmd_cam_fub and the bank machines’ state matrix. See ch02_blocks/07_scheduler.md for the full critical-path breakdown.

## 9.8	CSR Hooks

- STATUS.rd_cam_occ — current occupancy

- OBS_RD_CAM_HIGH_WATER — peak occupancy observed since last read

## 9.9	TODO

- Mermaid for the parallel match-line topology

- Wavedrom: push → scheduler-match → issue → beats return → clear

- Per-ID head-of-queue ordering corner case spec

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Write Command CAM (wr_cmd_cam_fub)

Module: wr_cmd_cam_fub.sv Location: rtl/fub/ Category: FUB Parent: ddr2_lpddr2_ctrl Status: Skeleton

## 10.1	Purpose

wr_cmd_cam_fub is the write-side content-addressable storage of in-flight write commands. It mirrors the read CAM’s role for write traffic but carries different per-slot metadata:

- Pointer into the AXI-side write-data buffer (w_buf_ptr)

- Pointer into the AXI-side write-strobe buffer (strb_ptr)

- Per-slot beats-issued counter (so the FUB knows when the last W beat has been pushed to DFI write data)

Separation from the read CAM lets the write path sit shallow — writes complete on B response, which is a single AXI event, so the write CAM does not need to track per-beat return.

## 10.2	Parameters

| Parameter | Default | Purpose |
| --- | --- | --- |
| WR_CAM_DEPTH | 16 | Number of in-flight write slots |
| IDW | AXI_ID_WIDTH | Key width |
| W_BUF_PTR_WIDTH | $clog2(W_BUF_DEPTH) | Pointer into AXI W buffer |
| BURST_LEN_WIDTH | 8 | Burst-length field width |

## 10.3	Storage Per Slot

| Field | Width | Description |
| --- | --- | --- |
| valid | 1 | Slot occupied |
| axi_id | IDW | Key — the originating AXI ID |
| rank | $clog2(NR) | Decoded rank |
| bank | BA | Decoded bank |
| row | ROW_WIDTH | Decoded row |
| col_start | COL_WIDTH | Starting column |
| burst_len | BURST_LEN_WIDTH | Total beats |
| beats_issued | BURST_LEN_WIDTH | Beats already pushed to DFI write data |
| w_buf_ptr | W_BUF_PTR_WIDTH | Pointer into the AXI W-buffer for next beat |
| strb_ptr | W_BUF_PTR_WIDTH | Pointer into the per-beat strobe buffer |
| issued | 1 | Scheduler has picked this entry |
| b_pending | 1 | Awaiting B response from DFI write completion |

Total per-slot width: ~58 bits in the default config. Total storage (16 × 58): ~116 bytes, distributed flops.

## 10.4	Interfaces

### 10.4.1	Push (from axi4_slave_fub)

Same shape as the read CAM push, plus w_buf_ptr and strb_ptr (which the AXI slave allocates at push time).

### 10.4.2	Scheduler Query

Same shape as the read CAM. The scheduler matches by (rank, bank) and selects with row-hit priority.

### 10.4.3	Issue Notification

When the scheduler picks a write, the CAM: 1. Marks issued = 1 2. Begins handing per-beat data to wr_data_path_fub via the beat_data_pull interface (below)

### 10.4.4	Beat-Pull (to wr_data_path_fub)

| Signal | Direction | Description |
| --- | --- | --- |
| beat_pull_o | output | Strobe — pull next beat for this slot |
| beat_pull_slot_o | output | Slot index whose beat is being pulled |
| beat_pull_ptr_o | output | w_buf_ptr + beats_issued |
| beat_pull_strb_o | output | strb_ptr + beats_issued |
| beat_pull_last_o | output | 1 when this is the burst’s final beat |

wr_data_path_fub walks the AXI W buffer at the supplied pointer and pushes to DFI wrdata. On the last beat, it asserts b_pending_set to the CAM.

### 10.4.5	Completion (from DFI write window expiration)

| Signal | Direction | Description |
| --- | --- | --- |
| b_pending_set_i | input | Last W beat pushed; await tCWL+tWR window |
| b_complete_i | input | Write window expired (from xbank_timers) |
| entry_complete_o | output | Burst is fully complete; signal axi4_slave_fub to push B response |

## 10.5	Difference From Read CAM

| Aspect | Read CAM | Write CAM |
| --- | --- | --- |
| Lifecycle | Push → issue → beats return → clear | Push → issue → beats issued → write window expires → B → clear |
| Beat tracking | Counts beats returned from DFI | Counts beats issued to DFI; also tracks the post-issue write window |
| AXI ID echo | rid returned with each R beat | bid returned once at B-response time |
| Pointer fields | none (just a beat counter) | w_buf_ptr, strb_ptr into AXI-side buffers |
| Completion driver | rd_data_path (last beat of R) | xbank_timers (write window expiration) → wr_data_path |

The asymmetry justifies the separate FUBs. Trying to share a CAM would either inflate the read slot to carry write-only fields, or vice versa.

## 10.6	Timing Budget

Same shape as the read CAM. The combinational match-line path is shared with rd_cmd_cam_fub in the scheduler’s match aggregation.

## 10.7	CSR Hooks

- STATUS.wr_cam_occ — current occupancy

- STATUS.wr_b_pending — count of slots awaiting B response

- OBS_WR_CAM_HIGH_WATER — peak occupancy

## 10.8	TODO

- Mermaid for the issued → beat-pull → b_pending → b_complete lifecycle

- Wavedrom showing a multi-beat write burst from CAM push to B response

- Backpressure path when wr_data_path can’t keep up with beat_pull (rare; DFI wrdata is wider than AXI W)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Transaction Queue (txn_queue_fub)

Module: txn_queue_fub.sv Location: rtl/fub/ Category: FUB Parent: ddr2_lpddr2_ctrl Status: Draft v0.1

Architectural context: HAS §3.2 txn_queue. This block-level MAS section is the implementation detail: entry format, port arrangement, insert/clear paths, row_hit_cached coherency, backpressure, storage choice.

## 11.1	Purpose

txn_queue_fub is the scheduling view of in-flight DRAM transactions. It is the data structure the scheduler reads every MC clock to pick the next command. The queue is not the source of truth for AXI metadata or beat tracking — that lives in rd_cmd_cam_fub and wr_cmd_cam_fub. The queue carries only the fields the scheduler’s priority function needs, plus a reverse pointer back to the CAM slot.

The split exists because the scheduler’s read path is the hottest in the design: every cycle it touches every entry in parallel. Making the entry narrow (~32 bits, see below) keeps the parallel-comparator fan-in manageable. The CAMs carry the wider per-burst metadata (w_buf_ptr, strb_ptr, beats_returned, etc.) which is only touched on insert, on scheduler pick, and on per-beat completion — never in the per-cycle scheduling loop.

## 11.2	Relationship to the CAMs

Each entry in txn_queue reflects exactly one slot in rd_cmd_cam or wr_cmd_cam. The cam_slot_idx field is the reverse pointer:

txn_queue[q].cam_slot_idx ──► (is_write ? wr_cmd_cam[cam_slot_idx] : rd_cmd_cam[cam_slot_idx])

When the scheduler picks txn_queue[q], it issues an “mark issued” strobe to the corresponding CAM using cam_slot_idx (see scheduler_fub, §2.7). When the CAM signals entry-complete (read: last beat returned; write: B-channel quiet point reached), it strobes back to clear txn_queue[q].

The queue and CAMs are sized independently:

| Structure | Default depth | Reason |
| --- | --- | --- |
| txn_queue | 16 | Scheduling pool — depth bounded by parallel-comparator cost |
| rd_cmd_cam | 16 | One slot per pending read |
| wr_cmd_cam | 16 | One slot per pending write |

In the default 16+16+16 configuration, the queue is the bottleneck — at most 16 transactions can be queued for scheduling at any time, regardless of how many slots are free in each CAM individually. The CAMs are not allowed to oversubscribe the queue (the axi4_slave_fub push path stalls on txn_queue full, not on CAM-full). The two CAMs can be sized larger than the queue if the read-path and write-path have very different burst-completion latencies — but that’s not v1.

## 11.3	Synthesis Parameters

| Parameter | Source | Effect |
| --- | --- | --- |
| TXN_QUEUE_DEPTH | top (default 16) | Number of parallel-readable entries |
| AGE_MAX | top (default 256) | Age counter saturation; per-entry clog2(AGE_MAX) = 8 bits |
| ROW_WIDTH | top | Width of the row field |
| NUM_RANKS | top | Width of the rank field |
| NUM_BANKS | top | Width of the bank field |

## 11.4	Entry Format

Each slot carries the scheduling view — the minimum metadata the scheduler’s priority function needs.

| Field | Width | Description |
| --- | --- | --- |
| valid | 1 | Slot occupied |
| state | 2 | {FREE, PENDING, ISSUED, COMPLETING} (see lifecycle below) |
| is_write | 1 | Picks rd vs wr CAM for the reverse pointer |
| rank | $clog2(NUM_RANKS) | Target rank (1-2 bits) |
| bank | $clog2(NUM_BANKS) | Target bank (3 bits for NB=8) |
| row | ROW_WIDTH | Target row (14 bits default) |
| row_hit_cached | 1 | Cached at insert; refreshed on bank-state change |
| age | $clog2(AGE_MAX) | Saturating age counter (8 bits default) |
| cam_slot_idx | $clog2(CAM_DEPTH) | Reverse pointer to rd_cmd_cam or wr_cmd_cam slot (4 bits) |
| Total per slot | ~32 bits | (default: NR=1, NB=8, ROW=14, AGE_MAX=256, CAM=16) |

Total queue storage at depth 16: ~64 bytes (16 × 32 b). This is distributed flops — no SRAM macro. Even at depth 64 it’s 256 bytes, still trivially distributed.

Fields not in the queue entry (these stay in the CAM):

- col_start, burst_len — only consulted at issue time (Stage 4 of scheduler), not every cycle

- axi_id — opaque to the scheduler; only the CAM and axi4_slave_fub need it

- w_buf_ptr, strb_ptr, beats_returned, beats_issued — beat-level tracking, not scheduling

At issue time, the scheduler reads col_start and burst_len via the cam_slot_idx reverse pointer — a single read from the CAM’s lookup port. This is one cycle slower than carrying them in the queue, but it keeps the queue narrow.

## 11.5	Entry Lifecycle

txn_queue Entry State Diagram

Source: 06_txn_queue_entry_state.mmd

Four states tracked per slot:

| State | Visible to scheduler? | Counts toward queue depth? | Notes |
| --- | --- | --- | --- |
| FREE | No | No | Empty slot, available for insert |
| PENDING | Yes (eligible) | Yes | Scheduler is searching for this entry |
| ISSUED | No (masked out) | Yes | Picked by scheduler; column command in flight |
| COMPLETING | No (masked out) | Yes | Last beat / write-window draining |

The scheduler’s pick_valid_mask (see §2.7 Stage 2) explicitly excludes ISSUED and COMPLETING so the same entry can never be picked twice. The mask is (state == PENDING).

ISSUED → COMPLETING is the same edge in both rd and wr cases but triggered by different signals:

- Read: first R beat returns from DFI → rd_data_path_fub strobes entry_transition_completing via the CAM.

- Write: last W beat is pushed to DFI write data → wr_data_path_fub strobes the same edge.

The COMPLETING → FREE edge is driven by the CAM’s entry_complete_o strobe (see §2.4 and §2.5). The queue does not own completion-detection logic; it follows the CAM.

## 11.6	Ports

### 11.6.1	Insert (from CAMs)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ins_valid_i | input | 1 | New entry to push |
| ins_ready_o | output | 1 | Queue has a free slot |
| ins_is_write_i | input | 1 |  |
| ins_rank_i | input | $clog2(NR) | from addr_mapper |
| ins_bank_i | input | $clog2(NB) |  |
| ins_row_i | input | ROW_WIDTH |  |
| ins_cam_slot_i | input | $clog2(CAM_DEPTH) | Reverse pointer |

row_hit_cached is not an insert port — it’s computed inside this FUB at insert time by querying bank_state_i[ins_rank][ins_bank].open_row (see “Insert Path” below). This keeps the insert-time row-hit computation local to the queue.

### 11.6.2	Parallel Read (to scheduler — every cycle)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| q_entries_o[TXN_QUEUE_DEPTH-1:0] | output | per entry-format table | Live snapshot of all slots, parallel |

This is a wide bus — at the default TXN_QUEUE_DEPTH = 16 and ~32 bits per entry, it’s a 512-bit bus. The scheduler reads it combinationally; there’s no read-port arbitration because there’s only one reader.

### 11.6.3	Bank-State Watchpoint (from bank machines)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| bank_state_i[NR][NB] | input | per bank_machine | Live bank state (used at insert for row_hit_cached) |
| bank_open_row_changed_i[NR][NB] | input | NR×NB | One-cycle strobe when a bank’s open row changes |
| bank_new_open_row_i[NR][NB] | input | NR×NB × ROW_WIDTH | New open-row value, valid the cycle after the strobe |

The _changed + _new_open_row pair is what drives the per-entry row_hit_cached refresh path (see “Row-Hit Cache Coherency” below).

### 11.6.4	Scheduler Pick (from scheduler)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| pick_one_hot_i | input | TXN_QUEUE_DEPTH | One-hot vector of which slot the scheduler picked |
| pick_strobe_i | input | 1 | Pick happened this cycle |

On pick_strobe_i, the indicated slot transitions PENDING → ISSUED.

### 11.6.5	CAM Completion (from CAMs)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| entry_transition_i | input | 2 | 01 = ISSUED → COMPLETING, 10 = COMPLETING → FREE |
| entry_transition_slot_i | input | $clog2(TXN_QUEUE_DEPTH) | Slot index that’s transitioning |
| entry_transition_strobe_i | input | 1 | Transition strobe |

### 11.6.6	Backpressure Output

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| q_high_water_o | output | 1 | High when occupancy ≥ SCHED_TUNING.txn_queue_high_water |
| q_full_o | output | 1 | All slots are non-FREE |
| q_occupancy_o | output | $clog2(DEPTH+1) | Current occupancy (number of slots not in FREE state) |

q_high_water_o drives the AXI backpressure path in axi4_slave_fub — awready / arready deasserts when the queue is near full. q_full_o is the hard backpressure (no inserts accepted).

## 11.7	Insert Path

When axi4_slave_fub has decoded a new burst and pushed it to either CAM, the CAM in turn pushes to this queue:

cycle T:   CAM push from axi4_slave  →  ins_valid asserted, ins_ready asserted
cycle T:   queue selects free_slot via priority encoder over (state == FREE)
cycle T:   row_hit_cached_init = (bank_state[ins_rank][ins_bank].state == ACTIVE
                                 AND bank_state[ins_rank][ins_bank].open_row == ins_row)
cycle T+1: free_slot.{is_write, rank, bank, row, row_hit_cached, cam_slot_idx} latched
cycle T+1: free_slot.state = PENDING, age = 0

The insert is a two-cycle latency from CAM push to first scheduler visibility, but the queue can accept one new insert per cycle (the comparator network for free_slot finds the next available slot combinationally; the latch is registered).

row_hit_cached_init is the only nontrivial computation at insert time. It’s a single comparison: does the target bank’s open row match the inserted row? If the bank is IDLE, it’s automatically 0 (no row is open). If the bank is in the middle of a row change (ACTIVATING or PRECHARGING), it’s also 0 — the cached value will be refreshed in the next cycle when the bank settles via the watchpoint path below.

## 11.8	Row-Hit Cache Coherency

The row_hit_cached field is computed at insert (above) and refreshed any time a bank’s open row changes.

The refresh path:

cycle T:   bank_machine[r,b] transitions ACTIVE → ACTIVE-on-new-row
           (via PRE then ACT)  OR  IDLE → ACTIVE (via ACT)
cycle T:   bank_machine[r,b] strobes bank_open_row_changed[r][b] = 1
           and presents bank_new_open_row[r][b] = R_new
cycle T+1: txn_queue refresh path:
           for each slot i:
             if entries[i].state in {PENDING}
                AND entries[i].rank == r AND entries[i].bank == b:
               entries[i].row_hit_cached = (entries[i].row == R_new)

The refresh is broadcast-parallel: all TXN_QUEUE_DEPTH entries are checked in parallel (their (rank, bank) matches are computed; the matching set has its row_hit_cached recomputed). The refresh fires for ISSUED and COMPLETING entries too, but those are masked out by the scheduler anyway — refreshing them is harmless and cheaper than gating.

Why a one-cycle lag is acceptable. In the cycle the bank state changes, the scheduler may still observe stale row_hit_cached. The worst outcome is the scheduler picks an entry as a “row hit” that isn’t, or vice versa. The bank_machine catches this — accepts_rd / accepts_wr are gated on the current open_row, so the scheduler can’t issue an incorrect column command. It would issue a bare RD/WR when an RDA/WRA was warranted (or vice versa), costing at most one extra PRE later. This is the trade documented in scheduler §2.7.

The synchronous broadcast (rather than asynchronous combinational re-eval) keeps the scheduler’s Stage-1 path off the bank_machine state machines. If we tried to make this combinational, the Stage-1 critical path would route through every bank machine’s open_row register, every queue entry’s row comparator, and every priority mask — easily 8-10 LUT levels, ruining the timing budget.

## 11.9	Age Counter Saturation

Every cycle, every PENDING entry’s age increments by 1. The counter saturates at AGE_MAX - 1. Concretely with AGE_MAX = 256:

- New entry: age = 0

- After 1 cycle: age = 1

- After 255 cycles: age = 255

- After 256+ cycles: age = 255 (saturated)

The scheduler’s age priority encoder treats saturated entries equally — once an entry has been waiting “long enough,” it has maximum priority and ties break by slot index. In normal traffic, ages rarely saturate; in pathological cases (a row-conflict victim behind a stream of row-hits) saturation provides the anti-starvation guarantee.

The runtime override SCHED_TUNING.age_max_runtime clips the effective saturation point lower than the build-time AGE_MAX — useful for characterization sweeps to shorten the saturation timescale without rebuilding.

Increment gating. Age does not increment when state is ISSUED or COMPLETING. Those entries are out of the scheduling pool; aging them further would skew the priority for any subsequent entries that get re-queued (which doesn’t currently happen in v1, but the gate is cheap insurance).

## 11.10	Storage Choice

At the default TXN_QUEUE_DEPTH = 16, total storage is ~64 bytes. Distributed flops are the right answer — an SRAM macro would have higher access latency and the macro overhead is significant for so little data.

The crossover point where SRAM becomes attractive is around TXN_QUEUE_DEPTH ≥ 64 (256 bytes), and even then only if the scheduler can be re-architected to issue from a partial snapshot rather than the full queue. Currently, the scheduler reads all entries in parallel — SRAM ports can’t deliver that. So in v1 we cap at distributed flops; the parameter range is 4 … 64.

For depth = 64, the entry bus is 64 × 32 = 2048 bits — wide but routable on 7-series. Stage-1 priority comparators scale linearly with depth (16 → 64 is 4× the comparators), and the age priority encoder tournament grows by log₂(4) = 2 levels. The critical-path budget in §2.7 absorbs this at the 200 MHz target; at 500 MHz the scheduler will need flop insertion as noted.

## 11.11	CSR Hooks

| CSR field | Source signal | Use case |
| --- | --- | --- |
| STATUS.txn_queue_occ (R) | q_occupancy_o | Live occupancy |
| OBS_TXN_QUEUE_DEPTH_MAX (R) | High-water-mark register | Peak occupancy since last read (HAS §6.3) |
| OBS_TXN_QUEUE_DEPTH_AVG (R) | Rolling time-average | Time-averaged occupancy |
| STATUS.txn_queue_full_pulses (R) | Counter — number of times q_full_o asserted | Hard-backpressure event counter |
| SCHED_TUNING.txn_queue_high_water (R/W) | Threshold for q_high_water_o | Software-tunable soft-backpressure point |

The _high_water threshold defaults to TXN_QUEUE_DEPTH - 2 so AXI has a few-burst headroom before hard backpressure. Software can tune it lower for tighter latency control.

## 11.12	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Single insert, single pick, single clear | Smoke: FREE → PENDING → ISSUED → COMPLETING → FREE |
| Insert at full speed (1 per cycle) until full | Insert throughput; q_full_o asserts on the 17th |
| Bank-state change while N entries target that (rank, bank) | Broadcast row-hit refresh works on all matching slots |
| Bank-state change during a pick on the same slot | Pick is honored; refresh applies to next cycle’s view |
| Age saturation across all queue slots | All saturate at AGE_MAX - 1; tie-break by slot index |
| age_max_runtime = 16, age saturates earlier than AGE_MAX | Runtime clip works |
| Insert + pick + complete in the same cycle (different slots) | The four ports are non-conflicting |
| txn_queue_high_water = 8; AXI backpressure asserts at occupancy 8 | Soft backpressure threshold |
| q_full_o blocks CAM push; CAM stalls AXI | Hard backpressure path |
| Insert with is_write = 0 and is_write = 1 interleaved | Reverse-pointer routing to correct CAM |
| entry_transition with slot out-of-range (illegal) | Assertion fires; entry untouched |
| Reset during mid-life: all slots clear, no spurious completions emitted | Reset behavior |

## 11.13	Open Questions / Future Work

- Should the queue allow re-queue of a COMPLETING entry that fails (e.g., a DFI rddata timeout)? Currently the CAM owns failure-recovery; if it decides to retry, it would have to push a new queue entry. A “re-queue without losing age” path would be cheaper but adds complexity. Defer to v2.

- Should cam_slot_idx be wider than $clog2(CAM_DEPTH) to allow over-sized CAMs? Currently sized to match the smaller of the two CAM depths. If the rd-CAM grows to 32 and wr-CAM stays at 16, the field width adapts at elaboration; this is fine but the field is technically over-provisioned for write entries. Not worth fixing in v1.

- The broadcast row_hit_cached refresh fires for ISSUED / COMPLETING entries too. These are masked at the scheduler anyway. The gate would save a few comparator switches but no flops — drop the gate optimization. Document the decision and move on.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	FR-FCFS Scheduler (scheduler_fub)

Module: scheduler_fub.sv Location: rtl/fub/ Category: FUB Parent: ddr2_lpddr2_ctrl Status: Draft v0.1

Architectural context: HAS §3.2 (scheduler, priority function, lookahead, force_inorder, refresh-priority). This block-level MAS section is the implementation view: signals, internal datapath, pipeline staging, critical paths, and verification scenarios.

## 12.1	Purpose

scheduler_fub is the central command issue engine. Every MC clock cycle it:

- Snapshots txn_queue and all bank_machine states

- Computes a per-entry readiness vector (which queue entries could issue right now)

- Applies priority masking (init, refresh, force-inorder, row-hit-first, age tie-break)

- Picks at most one winner via parallel priority encoders

- Generates the DRAM command (issue_op, operands) and the CAM “mark issued” strobe

Issue rate is one command per MC clock. Multi-cycle commands (LPDDR2’s 2-cycle CA encodings) are absorbed inside cmd_encoder_fub and gear_dfi_fub; the scheduler still sees one issue per cycle.

The block is the single hardest synthesis-timing FUB in the design — the parallel match-line into per-entry readiness, then into priority encoders, is what sets the MC clock frequency ceiling. Everything else has slack.

## 12.2	Synthesis Parameters

| Parameter | Source | Effect on this FUB |
| --- | --- | --- |
| SCHEDULER_MODE | top | "OOO" synthesizes the full FR-FCFS comparator network; "INORDER" synthesizes only the first-ready FIFO path (smaller area, no row-hit reordering). |
| TXN_QUEUE_DEPTH | top (default 16) | Number of parallel readiness comparators |
| LOOKAHEAD_DEPTH_MAX | top (default 4) | Width of the lookahead window peek into txn_queue |
| PAGE_POLICY | top | Picks which Stage-4 auto-precharge decoder is synthesized |
| NUM_RANKS, NUM_BANKS | top | Width of the bank-state input matrix |
| AGE_MAX | top (default 256) | Width of per-entry age counter (clog2(AGE_MAX)) |

The "INORDER" synthesis variant is a different decoder tree, not a runtime gate of the OoO tree. The runtime SCHED_TUNING.force_inorder bit is honored only when SCHEDULER_MODE == "OOO" was synthesized (the bit becomes a tied observation in INORDER builds, per HAS §3.2).

## 12.3	Interface

### 12.3.1	Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk, mc_rst_n | input | 1, 1 | MC clock + async-assert / sync-deassert reset |
| q_entries_i[TXN_QUEUE_DEPTH-1:0] | input | per HAS §3.2 | Live queue snapshot: per entry {valid, is_write, rank, bank, row, col, burst_len, row_hit_cached, age, state, cam_slot_idx} |
| bank_state_i[NUM_RANKS][NUM_BANKS] | input | per bank_machine_fub | Per-(rank, bank): state, open_row, accepts_act/rd/wr/pre/ref |
| xbank_blocks_i | input | struct | blocks_act_per_rank[NR], blocks_xrank_rd, blocks_xrank_wr |
| refresh_wants_i | input | 1 | One-bit “refresh due now” from refresh_mgr_fub |
| refresh_done_for_i[NUM_RANKS][NUM_BANKS] | input | NR×NB | Per-(rank, bank) “refresh handshake granted” from refresh_mgr_fub |
| init_in_progress_i | input | 1 | From init_engine_fub — blocks all normal traffic |
| predict_hit_i[NUM_RANKS][NUM_BANKS] | input | NR×NB | HAPPY predictor output (tied 0 when PAGE_POLICY != HAPPY_HYBRID) |
| cfg_force_inorder_i | input | 1 | SCHED_TUNING.force_inorder runtime override |
| cfg_lookahead_active_i | input | 4 | SCHED_TUNING.lookahead_active |
| cfg_page_policy_or_i | input | 2 | REFRESH_TUNING.page_policy_or |
| cfg_happy_enable_i | input | 1 | SCHED_TUNING.happy_enable |

### 12.3.2	Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| issue_valid_o | output | 1 | A command is being issued this cycle |
| issue_op_o | output | 4 | One of {NOP, ACT, RD, RDA, WR, WRA, PRE, PREA, REF, REFPB, MRS, ZQCS, ZQCL} |
| issue_rank_o | output | $clog2(NR) | Target rank |
| issue_bank_o | output | $clog2(NB) | Target bank |
| issue_row_o | output | ROW_WIDTH | Target row (for ACT) or open-row pass-through (for RD/WR) |
| issue_col_o | output | COL_WIDTH | Target column (for RD/WR) |
| issue_axi_id_o | output | AXI_ID_WIDTH | Originating AXI ID (passed to the CAM for completion routing) |
| issue_cam_slot_o | output | clog2(CAM_DEPTH) | Index into rd_cmd_cam or wr_cmd_cam to mark “issued” |
| issue_is_write_o | output | 1 | Picks wr_cmd_cam vs rd_cmd_cam for the mark-issued strobe |
| predict_query_o | output | struct | (rank, bank, row) query to page_predictor_fub for stage-4 auto-pre |
| dbg_inorder_active_o | output | 1 | (debug) High when current cycle picked the in-order path |
| dbg_refresh_blocking_o | output | 1 | (debug) High when wants_refresh reduced the candidate pool |

## 12.4	Decision Pipeline

The decision is a single MC-clock-cycle combinational pipeline broken into four conceptual stages, finishing in a registered output. Stages 1–3 are combinational; Stage 4 latches the final command into output flops.

Scheduler Priority Pipeline

Source: 05_scheduler_priority_pipeline.mmd

### 12.4.1	Stage 1 — Per-Entry Readiness (combinational, N parallel comparators)

For each of TXN_QUEUE_DEPTH entries, in parallel:

ready[i] = q_entries[i].valid
         AND  (q_entries[i].state == PENDING)
         AND  bank_machine[r,b].accepts_X
         AND  NOT xbank_blocks_X
         AND  ( NOT refresh_wants
                OR  bank_machine[r,b].in_refresh_done_set )

is_row_hit[i] = q_entries[i].row_hit_cached
                ( cached at queue insert; refreshed when bank_machine[r,b]
                  state changes — see "Cache Coherency" below )

needs_act[i]  = NOT bank_active[r,b]
                OR  bank_open_row[r,b] != q_entries[i].row

Where X is rd if is_write == 0, wr if is_write == 1. r = q_entries[i].rank, b = q_entries[i].bank.

ready[i] answers “could I issue some command for this entry this cycle without violating any per-bank or cross-bank timer?” is_row_hit[i] answers “if I do, is it a column command on the already-open row?” needs_act[i] answers “is the preliminary command actually ACT (or PRE+ACT) rather than RD/WR?”

### 12.4.2	Stage 2 — Priority Masking (combinational)

Five masks, computed in parallel from the Stage-1 vector and the config inputs:

| Mask | Definition | Effect |
| --- | --- | --- |
| init_mask | init_in_progress ? 0 : ready[i] | Block all normal entries during init |
| refresh_mask | refresh_wants ? (ready[i] AND bank_in_refresh_done_set) : ready[i] | Allow only entries whose target rank+bank has already been granted the refresh handshake (i.e., already transitioned out of REFRESHING, or never entered it) |
| inorder_mask | force_inorder ? oldest_ready_one_hot : ready[i] | Collapse to first-ready FIFO |
| row_hit_mask | ready[i] AND is_row_hit[i] AND NOT needs_act[i] | Row-hit candidates (column-only) |
| row_miss_mask | ready[i] AND (NOT is_row_hit[i] OR needs_act[i]) | Row-miss candidates (need ACT or PRE+ACT) |

The composed candidate vectors are:

cands_row_hit  = init_mask AND refresh_mask AND inorder_mask AND row_hit_mask
cands_row_miss = init_mask AND refresh_mask AND inorder_mask AND row_miss_mask

### 12.4.3	Stage 3 — Selection (combinational priority encoders)

Two parallel age-priority encoders, then a 2:1 mux:

winner_hit  = age_pe(cands_row_hit)    // index of oldest entry in cands_row_hit, or none
winner_miss = age_pe(cands_row_miss)   // index of oldest entry in cands_row_miss, or none

picked = winner_hit  IS_VALID ? winner_hit
       : winner_miss IS_VALID ? winner_miss
                              : NONE   // issue NOP

The age priority encoder is the slowest path in this stage — it’s an N-entry tournament on clog2(AGE_MAX) = 8-bit ages. For N = 16 (the default), that’s 4 levels of pairwise compare-and-select. Synthesizable in two LUT layers per level on 7-series; ~30 LUT-delays in the typical config.

### 12.4.4	Stage 4 — Command Formation (registered)

Once an entry is picked, Stage 4 turns it into a DRAM command. This stage may pre-flop intermediate signals for timing closure on the highest clock targets (see “Pipeline Staging” below).

- Lookahead peek — examine the next cfg_lookahead_active queue entries after the picked one to see if any target the same (rank, bank).

- Auto-precharge decision — per HAS §3.2 (lookahead first, fallback to PAGE_POLICY):

  - Same-bank entry found AND its row matches the open row → keep open → bare RD / WR

  - Same-bank entry found AND its row differs → close → RDA / WRA

  - No same-bank entry within window → consult cfg_page_policy_or:

    - OPEN → bare RD / WR

    - CLOSE → RDA / WRA

    - HAPPY_HYBRID → query predict_hit_i[r][b]; if predicted hit → bare; else RDA/WRA

- Op encoding — generate the 4-bit issue_op_o from (is_write, needs_act, auto_pre):

  - needs_act and bank state == IDLE → ACT

  - needs_act and bank state == ACTIVE (different row) → PRE (the column command follows next cycle after tRP)

  - NOT needs_act and is_write → WR or WRA

  - NOT needs_act and NOT is_write → RD or RDA

- Latch outputs into the issue flops.

### 12.4.5	Refresh Window Behavior

When refresh_wants_i is high, refresh_mgr_fub is asserting refresh_req to one or more bank machines. Those bank machines drive refresh_done_for_i[r][b] = 1 only after reaching IDLE and handing over the grant. While refresh_done_for_i[r][b] == 0, that bank’s entries are masked out — no new column commands to a bank that’s about to refresh.

Bank machines for other ranks (in the multi-rank REFab round-robin) or other banks (in REFpb) remain available, so the scheduler continues to drain those during the refresh window. This is what makes per-rank REFab dispatch beneficial — the non-refreshing ranks keep their throughput.

When all required refreshes complete and refresh_wants_i drops, the masks lift and normal FR-FCFS resumes the next cycle.

## 12.5	Cache Coherency: row_hit_cached

q_entries[i].row_hit_cached is computed at queue insertion (by axi4_slave_fub via a one-cycle query to bank_machine[r,b].open_row) and stored in the queue entry. This avoids re-querying every cycle. But the bank state can change between insertion and selection:

- When bank_machine[r,b] transitions ACTIVE → ACTIVE on a different row (via PRE + ACT to a new row), all q_entries with (rank, bank) == (r, b) need their row_hit_cached recomputed against the new open_row.

- When bank_machine[r,b] transitions IDLE → ACTIVATING, the queue entries get row_hit_cached = 1 for entries whose row matches the in-flight ACT.

This is implemented as a broadcast update: on every bank state transition that affects the open row, the bank machine emits open_row_changed[r][b] + the new open_row. The queue has a per-entry update path that, in parallel, recomputes row_hit_cached. This is a one-cycle update — the recomputed value is visible to the scheduler in the cycle after the bank transition.

The one-cycle lag means the scheduler may make an “outdated row-hit” decision once per row change. The downside is one mis-categorized RD/WR (issued as bare when RDA was warranted, or vice versa); the worst case is one extra PRE later. This is a deliberate trade — the alternative is a combinational broadcast from bank state through every queue entry’s row-hit comparator, which would inflate the Stage-1 critical path.

## 12.6	Pipeline Staging

The default build is a single-cycle combinational scheduler — Stages 1–3 are combinational; Stage 4’s auto-precharge decoder is combinational; only the final issue-flop is registered. On 7-series FPGA at 200 MHz this closes timing comfortably. For 14 nm targets at 500+ MHz, two timing-closure escape hatches:

- Stage 3 → Stage 4 flop insertion — register picked between stages. Adds one cycle of issue latency. Throughput is unchanged because the pipeline is single-issue anyway; the only visible difference is that a request inserted at cycle T can first issue at cycle T+3 instead of T+2.

- Stage 1 partition by bank — partition the readiness comparator network by (rank, bank) so each partition produces a per-bank ready vector independently, then aggregate. Useful when NUM_RANKS × NUM_BANKS is small (the default 1×8 = 8 partitions) and routing congestion dominates.

These are not on by default; they’re synthesis-script switches, not RTL parameters.

## 12.7	Strict In-Order Mode (collapse)

When cfg_force_inorder_i = 1 (and SCHEDULER_MODE == "OOO" is synthesized):

- inorder_mask collapses to oldest_ready_one_hot — a one-hot vector that picks the oldest valid ready entry by age.

- This drops the row-hit prioritization (no row-hit vs row-miss split — both go through cands_row_miss since age order rules).

- Refresh priority is preserved (JEDEC requirement).

- Lookahead and HAPPY remain on at elaboration but can be independently disabled at runtime (cfg_lookahead_active = 0, cfg_happy_enable = 0) for a pure first-ready FIFO.

When SCHEDULER_MODE == "INORDER" is synthesized:

- The Stage-2 / Stage-3 priority encoders are different RTL: the row-hit/row-miss split is omitted; only the oldest-ready selector is built.

- cfg_force_inorder_i becomes a tied observation bit at STATUS.force_inorder_obs = 1.

## 12.8	Critical-Path Analysis (rough budget at 500 MHz target, 2 ns cycle)

| Path | Levels (approx) | Budget |
| --- | --- | --- |
| bank_state_i.accepts_X → ready[i] → row-hit mask → cands_row_hit[i] | 4 LUT levels | 0.7 ns |
| cands_row_hit → age priority encoder → winner_hit (16-entry tournament) | 4 levels × 2 LUT each = 8 | 1.2 ns |
| winner_hit / winner_miss → 2:1 mux → picked | 1 LUT level | 0.2 ns |
| Routing / setup margin |  | 0.3 ns |
| Total Stage 1–3 |  | 2.4 ns |

So at 500 MHz the path misses by ~400 ps and needs Stage 3 → Stage 4 flop insertion (the first escape hatch above). At 200 MHz target (5 ns cycle, embedded SoC default), the path closes with ~2.6 ns of slack — no escape hatch needed.

The 500 MHz analysis is FPGA-pessimistic; ASIC targets close 500 MHz on this path comfortably with standard pipelining.

## 12.9	CSR Hooks

The scheduler drives several CSR observability fields:

| CSR field | Source signal | Use case |
| --- | --- | --- |
| STATUS.issue_op_obs (R) | Last issued issue_op_o | Bring-up debug: what was the last DRAM command |
| STATUS.scheduler_idle_pct (R) | Rolling counter — fraction of cycles issue_valid_o = 0 | Bandwidth-headroom telemetry |
| OBS_ROW_HIT_RANK<R>_BANK<N> (R) | Incremented when row-hit path wins for that bank | Per-bank row-hit rate (HAS §6.3) |
| OBS_INORDER_FALLBACKS (R) | Incremented when force_inorder ate a row-hit win | Tells software how much OoO is currently giving up |
| OBS_LOOKAHEAD_HITS (R) | Incremented when Stage-4 lookahead was conclusive | Tunes LOOKAHEAD_DEPTH_MAX characterization sweep |
| STATUS.force_inorder_obs (R) | Echo of effective force-inorder state | Software-readable confirmation |

## 12.10	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Single-write, single-bank, single-rank | Smoke test: ACT → WR → b_complete cycle |
| Single-read, single-bank, single-rank | Smoke test: ACT → RD → beat return |
| Two reads, same bank, same row, oldest first | Row-hit reordering not invoked (already in order) |
| Two reads, same bank, different rows, oldest is row-conflict | OoO: younger row-hit wins over older row-miss |
| Same as above with force_inorder = 1 | OoO disabled: older row-miss wins despite younger row-hit |
| Cross-bank: rd to bank 0 and rd to bank 1, bank 0 row-miss, bank 1 row-hit | Bank parallelism: bank-1 issue fires first |
| Cross-rank (NUM_RANKS=2): rd to rank 0 bank 3 and rd to rank 1 bank 3 | Per-rank bank parallelism |
| Refresh during in-flight burst | Refresh window correctly drains and resumes |
| Refresh during multi-rank traffic (REFab on rank 0, traffic on rank 1) | Per-rank REFab dispatch: rank-1 traffic continues |
| Age-saturation under sustained row-conflict from younger IDs | Older row-conflict eventually wins via age priority |
| LOOKAHEAD_DEPTH_MAX = 4, lookahead concludes auto-precharge correctly | Lookahead path |
| LOOKAHEAD_DEPTH_MAX = 0, HAPPY predictor used | Fallback path through page predictor |
| PAGE_POLICY = CLOSE, every column command is auto-pre | CLOSE policy |
| Init-in-progress blocks normal traffic | init_mask gating |
| Queue full + back-pressure to AXI | axi4_slave does not accept new bursts |

## 12.11	Open Questions / Future Work

- Should QoS (awqos / arqos) be promoted from the v1 “no behavior” pass-through to a real priority boost in the FR-FCFS function? Currently HAS §3.2 leaves QoS as a side-band hint for v2. The hook would be a qos_boost_mask between the row-hit and row-miss stages.

- The cross-rank read-to-write turnaround (tRTRS + write window) lives in xbank_timers_fub today; the scheduler consumes the gate but does not pre-plan around it. A smarter scheduler could batch by rank (issue all ready reads on rank 0, then all on rank 1) to amortize tRTRS. This is a v2 feature; v1 takes the simpler per-cycle gating.

- Whether OBS_INORDER_FALLBACKS is useful for characterization is not yet validated; the counter is cheap, but if it’s never read by the bring-up team it can be dropped in v2.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	HAPPY Page Predictor (page_predictor_fub)

Module: page_predictor_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Synthesized only when PAGE_POLICY == HAPPY_HYBRID. See HAS §3.2 page_predictor for algorithm details (Ghasempour et al. 2015).

## 13.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Bank Machine (bank_machine_fub)

Module: bank_machine_fub.sv Location: rtl/fub/ Category: FUB Parent: ddr2_lpddr2_ctrl (instantiated NUM_RANKS × NUM_BANKS times) Status: Draft v0.1

Architectural context: HAS §3.3. The FSM state diagram is in ddr2_lpddr2_has/assets/mermaid/03_bank_machine_fsm.png and is the canonical state reference — this section is the implementation view (port interface, counter pool, broadcast updates, multi-rank instantiation, timing).

## 14.1	Purpose

bank_machine_fub is the per-(rank, bank) state machine that enforces JEDEC per-bank timing constraints. One FSM instance per (rank, bank) — so a 1-rank 8-bank build has 8 instances, a 2-rank 8-bank build has 16, a 4-rank 8-bank build has 32. Each instance tracks:

- Open-row state and the row identifier

- All per-bank timing counters (tRCD, tRAS, tRP, tRC, tWR, tRFCpb, plus per-RD CL and per-WR CWL windows)

- The “last refreshed” age used by DARP per-bank refresh selection

- The refresh handshake state to refresh_mgr_fub

The instance fans its outputs into:

- A per-(rank, bank) row of the scheduler’s bank-state matrix

- The txn_queue’s row-hit-cache refresh broadcast (open_row_changed[r][b] + new_open_row[r][b])

- The refresh manager’s refresh_gnt[r][b] array and the last_ref_age[r][b] DARP selection input

The bank machine is the most-replicated FUB in the design. Per-instance area is small (~150–300 LUTs, see below), but the replication factor (NR × NB, up to 32) makes total bank-machine area the second-largest after the scheduler.

## 14.2	Synthesis Parameters

| Parameter | Source | Effect on this FUB |
| --- | --- | --- |
| ROW_WIDTH | top | Width of open_row register and new_open_row broadcast |
| COL_WIDTH | top | Width of column passthrough (consumed by cmd_encoder, not stored) |
| MEMTYPE | top | Controls whether the REFRESHING state uses tRFCab or tRFCpb (LPDDR2 only has REFpb) |
| T_*_WIDTH_MAX | derived | Width of each timing-counter register (set by max CSR-loaded tREFI, tRC, tRFCpb, etc.) |
| LAST_REF_AGE_WIDTH | derived | Width of last_ref_age counter; sized to span 8 × tREFI_cycles without saturation |

The bank machine takes no rank/bank identifier as a parameter — every instance is identical. Identity comes from the position in the top-level generate block and is passed in via wire-level bank_id_i and rank_id_i ports (used only for assertion messages and CSR routing; the FSM logic is identity-agnostic).

## 14.3	Instantiation Pattern

Recap from §2.1 top-integration:

generate
    for (genvar r = 0; r < NUM_RANKS; r++) begin : g_rank
        for (genvar b = 0; b < NUM_BANKS; b++) begin : g_bank
            bank_machine_fub #(.ROW_WIDTH(ROW_WIDTH)) u_bank_machine (
                .mc_clk             (mc_clk),
                .mc_rst_n           (mc_rst_n),
                .rank_id_i          (r[$clog2(NR)-1:0]),
                .bank_id_i          (b[$clog2(NB)-1:0]),
                .sched_issue_if     (sched_to_bank[r][b]),
                .xbank_blocks_i     (xbank_to_bank[r][b]),
                .refresh_req_i      (refresh_req[r][b]),
                .refresh_gnt_o      (refresh_gnt[r][b]),
                .init_cmd_if        (init_to_bank[r][b]),
                .state_o            (bank_state[r][b]),
                .open_row_o         (bank_open_row[r][b]),
                .accepts_o          (bank_accepts[r][b]),
                .open_row_changed_o (orw_changed[r][b]),
                .new_open_row_o     (new_orw[r][b]),
                .last_ref_age_o     (bank_last_ref_age[r][b])
            );
        end
    end
endgenerate

The outputs are 2-D-indexed arrays at the top level. The scheduler and txn_queue consume them by name; no aggregation logic in the top — pure structural wiring.

## 14.4	Block Internals

Bank-Machine Internals — port boundary, counter pool, FSM, broadcasts

Source: 07_bank_machine_internals.mmd

The diagram above is the implementation view. The HAS state-diagram view is at ddr2_lpddr2_has/assets/mermaid/03_bank_machine_fsm.png — refer to it for state transitions; this section is the port + counter detail.

## 14.5	FSM States and Transition Triggers

Recap of the seven states (full state diagram in HAS):

| State | Held while | Exits via |
| --- | --- | --- |
| IDLE | No row open; bank free | ACT or REFpb issued by scheduler |
| ACTIVATING | t_rcd_cnt > 0 | t_rcd_cnt == 0 → ACTIVE |
| ACTIVE | Row open; awaiting column command or PRE | RD/RDA, WR/WRA, or PRE issued by scheduler |
| RD_BUSY | t_cl_cnt > 0 (CL + BL/2 beats remaining) | Last beat returned → ACTIVE (bare RD) or PRECHARGING (RDA) |
| WR_BUSY | t_cwl_cnt > 0 (CWL + BL/2 + tWR window) | Window expired → ACTIVE (bare WR) or PRECHARGING (WRA) |
| PRECHARGING | t_rp_cnt > 0 | t_rp_cnt == 0 → IDLE |
| REFRESHING | t_rfc_cnt > 0 (tRFCpb for LPDDR2, tRFCab for DDR2 REFab) | t_rfc_cnt == 0 → IDLE |

Auto-precharge handling. RDA / WRA do not require a separate state — they are encoded as an “auto-pre pending” flag set when the FSM enters RD_BUSY / WR_BUSY. When the busy window expires, the flag steers the next-state to PRECHARGING instead of ACTIVE. No PRE command is consumed from the scheduler — the precharge timing is purely internal. This is the same trick the cmd_encoder uses to fold RDA into one DRAM command.

Init-engine bypass. During init, the init_cmd_if path can issue ACT, PRE, MRS, REF directly without going through the scheduler. The FSM honors these as if they came from the scheduler — same state transitions, same counter reloads. The init engine knows the JEDEC sequence; the bank machine just executes it.

## 14.6	Counter Pool

All counters are down-counters that saturate at 0. Each is loaded with a CSR-supplied cycle count on the relevant trigger.

| Counter | Load Trigger | Reload Value (cycles) | Saturates At | Width |
| --- | --- | --- | --- | --- |
| t_rcd_cnt | ACT issued | TIMINGS_RC_RCD_RP.tRCD | 0 | 8 bits |
| t_ras_cnt | ACT issued | TIMINGS_RC_RCD_RP.tRAS | 0 | 8 bits |
| t_rp_cnt | PRE or PREA issued; or RDA/WRA window expiration | TIMINGS_RC_RCD_RP.tRP | 0 | 8 bits |
| t_rc_cnt | ACT issued | TIMINGS_RC_RCD_RP.tRC | 0 | 8 bits |
| t_wr_cnt | WR or WRA last beat issued | TIMINGS_CL_CWL_WR.tWR | 0 | 8 bits |
| t_rfcpb_cnt | REF or REFpb issued | LPDDR2 tRFCpb / DDR2 tRFCab | 0 | 16 bits |
| t_cl_cnt | RD or RDA issued | TIMINGS_CL_CWL_WR.CL + BL/2 | 0 | 8 bits |
| t_cwl_cnt | WR or WRA issued | TIMINGS_CL_CWL_WR.CWL + BL/2 + tWR | 0 | 8 bits |
| last_ref_age | REF or REFpb completion (t_rfcpb_cnt → 0) | reload to 0; otherwise +1 per cycle | does not saturate (wide enough for 8 × tREFI) | 24 bits |

The CSR-supplied values are read from TIMINGS_* registers in csr_apb_fub; the values are live (not staged) since timing parameters are typically loaded once at init and not changed at runtime. Software can change them at quiet points, but the bank machine doesn’t enforce quiet-point staging — it just samples them on each counter load.

last_ref_age is non-saturating because DARP needs to distinguish an entry that was refreshed 10 cycles ago from one refreshed 10,000 cycles ago. A 24-bit width supports 2^24 / 200 MHz = 84 ms of age, well past 8 × tREFI (typically 50 µs).

## 14.7	Outputs to Scheduler (accepts_*)

The accepts_o struct is the combinational readiness output consumed by the scheduler’s Stage-1 readiness computation. Each member is the AND of “FSM state permits this command” and “all relevant counters are 0” and “xbank constraints permit this command”:

accepts_act = (state == IDLE)
           AND (t_rp_cnt == 0)
           AND (t_rc_cnt == 0)
           AND NOT xbank_blocks_act

accepts_rd  = (state == ACTIVE)
           AND (t_rcd_cnt == 0)
           AND NOT xbank_blocks_rd

accepts_wr  = (state == ACTIVE)
           AND (t_rcd_cnt == 0)
           AND NOT xbank_blocks_wr

accepts_pre = (state == ACTIVE)
           AND (t_ras_cnt == 0)

accepts_ref = (state == IDLE)
           AND (t_rp_cnt == 0)
           AND (t_rc_cnt == 0)

These are pure combinational — no flop between the state register and the scheduler. The scheduler’s Stage-1 critical path (per §2.7) routes through these comparators.

Why xbank lives outside. Cross-bank constraints (tRRD, tFAW, tCCD, tWTR, tRTW) are shared across all banks of a rank or globally across the channel — they can’t be enforced inside a per-bank FSM. The xbank_timers_fub (§2.10) aggregates them and presents the per-(rank, bank) AND-mask via xbank_blocks_i.

## 14.8	Refresh Handshake Protocol

The bank machine cooperates with refresh_mgr_fub via a two-wire handshake per (rank, bank):

| Signal | Direction | Meaning |
| --- | --- | --- |
| refresh_req_i | refresh_mgr → bank | “Drain to a quiet state and await my REF / REFpb command” |
| refresh_gnt_o | bank → refresh_mgr | “I am in IDLE (or REFRESHING) and will not issue any column command” |

The bank machine’s protocol:

- refresh_req asserts while the bank is in ACTIVE / RD_BUSY / WR_BUSY: the bank does not assert refresh_gnt yet. It finishes its current column command (if any) and falls back to ACTIVE. The scheduler is masked from issuing new column commands to this bank during the refresh window (txn_queue watchpoint — see §2.6 and §2.7).

- refresh_req asserts while the bank is ACTIVE and no column command is in flight: the bank issues an internal precharge (driving accepts_pre high to signal the scheduler can issue PRE; or, if the scheduler is gated for the refresh window, the bank can self-issue PRE on t_ras_cnt == 0). Once state == IDLE and t_rp_cnt == 0, refresh_gnt asserts.

- refresh_req asserts while the bank is in IDLE: refresh_gnt asserts the same cycle.

- REF/REFpb arrives from the scheduler (via the refresh manager’s priority path): FSM transitions IDLE → REFRESHING, t_rfcpb_cnt loads, last_ref_age resets to 0.

- refresh_req deasserts after refresh completes: bank machine drops refresh_gnt, transitions REFRESHING → IDLE on t_rfcpb_cnt == 0, and is free to accept new ACTs.

For multi-rank REFab dispatch (per §2.11 refresh_mgr), the refresh manager asserts refresh_req[r][*] for all banks on the target rank simultaneously and waits for all refresh_gnt[r][*] before issuing REF. Other ranks’ bank machines see refresh_req[r' ≠ r] = 0 throughout and continue normal operation.

## 14.9	Open-Row Change Broadcast

When the FSM transitions in a way that changes the open row, the bank machine asserts a one-cycle broadcast to txn_queue_fub:

| Transition | open_row_changed_o | new_open_row_o |
| --- | --- | --- |
| IDLE → ACTIVATING (via ACT) | 1 cycle | The ACT’s row |
| ACTIVE → PRECHARGING → IDLE (after RDA/WRA or explicit PRE) | 1 cycle (on entering IDLE) | 0 (no row open) |
| REFRESHING → IDLE | 1 cycle | 0 (no row open) |
| ACTIVATING → ACTIVE | not asserted (row was already valid as of ACTIVATING) | — |

This is the path that drives txn_queue’s row_hit_cached refresh, as described in §2.6. The one-cycle lag (broadcast at T, queue refresh visible at T+1) is the deliberate trade described in both this block and the scheduler (§2.7).

new_open_row_o = 0 when the bank is leaving an open-row state is a convention — the queue’s refresh path compares against the new row only when the bank ends in an active state. When ending in IDLE, the queue sets row_hit_cached = 0 for all entries targeting this (rank, bank) regardless of new_open_row_o.

## 14.10	Per-Instance vs. Aggregated Storage

What lives per-instance (NR × NB copies):

- state register (3 bits)

- open_row register (ROW_WIDTH bits)

- All counters (~80 bits per instance)

- auto_pre_pending flag (1 bit)

What is aggregated at the top level (one copy each):

- bank_state[NR][NB] — concatenation of per-instance state outputs into a 2-D array

- bank_accepts[NR][NB] — concatenation of accepts structs

- The DARP candidate-set vector (computed by refresh_mgr_fub from the array of last_ref_age outputs)

- The scheduler’s match vector (computed by the scheduler from bank_state and bank_accepts)

This separation matters for synthesis: the per-instance flops are local to each bank machine (good for placement); the aggregation buses are wide (NR × NB × ~30 bits = up to ~1000 bits at 4×8) and route across the chip to the scheduler and refresh manager.

Routing concern. At NUM_RANKS=4, NUM_BANKS=8, the bank-state array routes 32 instances’ worth of state and accepts wires to the scheduler. This is a ~1000-bit bus and is the second-widest in the design (after txn_queue.q_entries_o). The scheduler’s Stage-1 critical path absorbs this in the 200 MHz target with slack; the 500 MHz target requires registered bank-state outputs (which adds one cycle to the scheduler-to-issue latency but is otherwise harmless — bank states change slowly compared to scheduler issue).

## 14.11	Per-Instance Area Estimate

At default config (ROW_WIDTH=14, COL_WIDTH=10, 8-bit timing counters, 24-bit last_ref_age):

| Item | Per instance |
| --- | --- |
| FSM state (3 bits + 3-bit next) | ~10 LUTs |
| Counter pool (8 down-counters) | ~60 flops + 30 LUTs |
| open_row register (14 bits) | 14 flops |
| last_ref_age (24 bits) | 24 flops |
| auto_pre_pending + combinational accepts_* glue | ~20 LUTs |
| Per-instance total | ~120 flops + 60 LUTs ≈ 100–150 LUTs |
| 32-instance total (4-rank × 8-bank) | ~3,200–4,800 LUTs |

This is ~25% of the controller’s total LUT budget at the max-rank config — significant. At single-rank, the bank machines fall to ~800–1,200 LUTs, comfortably in budget.

## 14.12	CSR Hooks

| CSR field | Source signal | Use case |
| --- | --- | --- |
| OBS_ROW_HIT_RANK<R>_BANK<N> (R) | Driven by scheduler when row-hit picks this bank | Per-bank row-hit telemetry (HAS §6.3) |
| OBS_REF_LATENCY_RANK<R>_BANK<N> (R) | Time from refresh_req to refresh_gnt | Per-bank refresh-blocking time |
| OBS_BANK_OPEN_ROW_R<R>_B<N> (R) | Current open_row value | Debug: which row is currently open |
| OBS_BANK_STATE_R<R>_B<N> (R) | Current FSM state (3-bit encoding) | Debug: bank state for bring-up |
| STATUS.bank_idle_mask (R) | One bit per (rank, bank): state == IDLE | Quick visibility into bank availability |

The per-(rank, bank) observation registers are sparse at multi-rank — at NR=4, NB=8 there are 32 of each. The CSR slave generates them from a YAML template (see §4.2). Software queries the cap_max_ranks and NUM_BANKS capability bits (§4.4) to know which registers exist.

## 14.13	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Single ACT → RD → PRE → IDLE on bank 0 | Smoke: each counter loads and decrements correctly |
| ACT → RDA: bank auto-precharges without explicit PRE from scheduler | Auto-precharge flag steers RD_BUSY → PRECHARGING |
| ACT → WRA: bank auto-precharges; t_wr_cnt window respected | Write recovery before precharge |
| Scheduler issues RD while t_rcd_cnt > 0 | Assertion fires; accepts_rd was 0 — should be impossible |
| refresh_req asserts during ACTIVATING | refresh_gnt waits until bank reaches IDLE |
| refresh_req asserts during RD_BUSY | Bank completes RD, returns to ACTIVE, then drains via PRE |
| refresh_req asserts when bank is IDLE | refresh_gnt asserts same cycle |
| REF completion → last_ref_age resets to 0 | DARP age-tracking correctness |
| Two adjacent ACTs to different rows on same bank — second blocks on t_rp_cnt | Per-bank tRP enforcement |
| Cross-bank ACT (handled by xbank_timers) — should NOT affect this bank | Per-bank FSM is rank/bank-local |
| Multi-rank (NR=2): refresh on rank 0 bank 3; rank 1 bank 3 continues normal ops | Per-rank refresh isolation |
| Open-row change broadcast: ACT issued, open_row_changed asserts for 1 cycle | Broadcast timing correctness |
| Soft reset during WR_BUSY — counters clear, state returns to IDLE | Reset behavior |

## 14.14	Open Questions / Future Work

- CSR coverage of per-bank state. At NR=4, NB=8 the OBS_BANK_OPEN_ROW_* registers consume 32 CSR slots — non-trivial. Worth gating behind a cap_per_bank_obs capability bit so smaller builds skip them entirely?

- Per-instance vs centralized last_ref_age. Could the age counters live in refresh_mgr_fub and free up flops per bank machine? Slight area win but adds a wide read port from refresh_mgr to bank_machine for age comparison; punt to v2.

- Auto-pre flag clear timing. Currently the auto_pre_pending flag clears on entering PRECHARGING. If a refresh request arrives mid-RD with auto-pre pending, the bank still does the auto-pre (drains via tRP) before granting refresh. This is correct per JEDEC, but worth a verification scenario (added above).

- tWR vs tRP composition. A WRA → PRECHARGING transition needs max(t_wr_cnt, t_rp_load) cycles. Implementation: the FSM enters PRECHARGING after t_wr_cnt expires and then loads t_rp_cnt. This is the simplest correct sequence; alternative is to start t_rp_cnt early and gate IDLE on max(t_wr_cnt, t_rp_cnt). The simpler form is preferred unless characterization shows it costs measurable bandwidth.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Cross-Bank Timers (xbank_timers_fub)

Module: xbank_timers_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Per-rank tRRD/tFAW, global tCCD/tWTR/tRTW, cross-rank tRTRS/tCS. See HAS §3.3 xbank_timers and the Per-Rank vs. Cross-Rank Scope table.

## 15.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Refresh Manager (refresh_mgr_fub)

Module: refresh_mgr_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Round-robin rank dispatch, DARP bank selection, postponer, periodic ZQCS, per-rank PASR mask. See HAS §3.4.

## 16.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Init Engine (init_engine_fub)

Module: init_engine_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Microprogram-driven cold-boot. Step-table ROM in rtl/macro/ per memtype. See HAS §3.5.

## 17.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Power-State FSM (power_state_fub)

Module: power_state_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Active / Active-PD / Self-Refresh / Deep-PD; per-rank CKE; SR coordination; quiet-point detector. See HAS §3.5.

## 18.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Command Encoder (cmd_encoder_fub)

Module: cmd_encoder_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

DDR2 ras/cas/we vs LPDDR2 20-bit CA bus, selected at elaboration by MEMTYPE. See HAS §3.6.

## 19.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	DFI Gear (gear_dfi_fub)

Module: gear_dfi_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Per-phase packing per N_PHASES / WRPHASE / RDPHASE. See HAS §3.6 gear_dfi.

## 20.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	ODT Control (odt_ctrl_fub)

Module: odt_ctrl_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

Per-rank dfi_odt[r] per ODT_RULE_MULTIRANK. The home of the cross-rank read/write ODT rules. See HAS §3.6.

## 21.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Write Data Path (wr_data_path_fub)

Module: wr_data_path_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

AXI W → DFI wrdata. Width conversion AXI DW → NP × DFI_DW. Strobe propagation. See HAS §3.7.

## 22.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Read Data Path (rd_data_path_fub)

Module: rd_data_path_fub.sv Location: rtl/fub/ Status: Skeleton — to be authored during MAS week

DFI rddata → AXI R. CL/CWL-aware capture, ID-tagged OoO completion. See HAS §3.7.

## 23.1	TODO (Week-of-MAS work)

- Interface signal list (inputs/outputs, widths, parameter dependencies)

- Internal state and storage

- FSM diagram (mermaid in assets/mermaid/)

- Datapath timing and per-stage flops

- Timing budget on critical paths

- CSR hooks (observability registers driven by this FUB)

- Verification notes (cocotb test plan)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	AXI4 Slave Protocol

Status: Skeleton — to be authored during MAS week

The AXI4 slave port carries host requests into the controller.

## 24.1	Scope of This Section

This section is the wire-level AXI4 contract specific to this controller — what’s supported, what’s relaxed, what’s omitted. It is not an AXI4 tutorial; refer to ARM IHI 0022 for the full protocol.

## 24.2	Supported Features (v1)

- AW/W/B/AR/R 5-channel handshake

- INCR burst type (mandatory)

- ID-aware OoO completion (when AXI_OOO_ACROSS_IDS = true)

- 64 / 128 / 256-bit data widths

- 1–16-bit ID widths

- Per-ID in-order completion

## 24.3	Optional Features (build-time)

- FIXED burst (SUPPORT_FIXED_BURST)

- WRAP burst (SUPPORT_WRAP_BURST)

## 24.4	Unsupported Features

- AXI exclusive accesses

- AXI4 user signals (no USER_* widths)

- AXI4 cache-coherent extensions

## 24.5	TODO

- Per-channel timing diagrams (wavedrom)

- Backpressure semantics in detail

- Response-ordering examples for OoO-across-IDs

- Burst-boundary corner cases (4 KB boundary, address-alignment)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	DFI v2.1 Master Protocol

Status: Skeleton — to be authored during MAS week

The DFI v2.1 master port drives the DRAM PHY.

## 25.1	Scope

This section is the wire-level DFI v2.1 contract as implemented by this controller. The full DFI v2.1 spec covers more — training, frequency change, low-power negotiation — see HAS §2.1 for what is and isn’t in scope here.

## 25.2	Sub-Interfaces Supported

- Command sub-interface

- Write-data sub-interface

- Read-data sub-interface

- Status sub-interface

- Update sub-interface (controller-initiated and PHY-initiated)

## 25.3	Sub-Interfaces Not Supported (v1)

- Training (not required at v2.1 for DDR2/LPDDR2)

- Frequency change

- Low-power negotiation (CKE used directly)

- CRC (not in DDR2/LPDDR2)

## 25.4	Phase / Gear Topology

N_PHASES per MC clock; gear FUB (see §2.15) handles packing.

## 25.5	TODO

- Per-signal timing diagrams (rise-to-rise)

- WRPHASE / RDPHASE selection guidelines

- ctrlupd / phyupd protocol detail (rare in v2.1 but spec-mandated)

- DDR2 ras/cas/we vs LPDDR2 CA encoding (cross-ref to §2.14)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	APB CSR Slave Protocol

Status: Skeleton — to be authored during MAS week

The APB CSR slave is the configuration and observation interface.

## 26.1	Protocol Compliance

APB3 (pslverr supported); APB4 (pstrb) optional.

## 26.2	Behavior

- Two-cycle access (SETUP + ENABLE) per the APB spec

- pslverr returns 1 for:

  - Writes to unimplemented schemes in ADDR_MAP_TUNING.scheme_or

  - Writes to ranks beyond NUM_RANKS (PASR registers, observation registers)

  - Writes to read-only registers

  - Reads from reserved addresses

- All registers are 32-bit; sub-32-bit accesses are aligned to 32-bit boundaries

## 26.3	CDC

The APB↔MC CDC lives in this FUB. See §1.3 and §2.18 csr_apb_fub for the CDC topology.

## 26.4	TODO

- pslverr decision tree

- APB-side timing diagram

- Sub-32-bit access policy

- CDC implementation detail

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Register Map

Status: Skeleton — to be authored during MAS week

The full CSR map is in HAS §6.3. This MAS section is the implementation-level view: how each register is generated, what the reset value generation rule is, how runtime overrides are staged, and the canonical CSR YAML source.

## 27.1	Source of Truth

The CSR map is generated from regs/ddr2_lpddr2_csrs.yaml (PeakRDL- or RDL-style YAML). The build runs the generator to produce:

- regs/ddr2_lpddr2_csr_pkg.sv — SV package with offsets, masks, reset values

- regs/ddr2_lpddr2_csr_block.sv — the wrapped register file

- regs/ddr2_lpddr2_csrs.h — C header for software

- regs/ddr2_lpddr2_csr_block_uvm.sv — UVM register model

The MAS does not re-publish the bit-level register table; it documents the generation pipeline and the runtime semantics (quiet-point staging, etc.).

## 27.2	Staging vs. Live State

Every R/W field has two storage cells:

- APB-side staging register — written immediately on the APB write

- MC-side live register — copied from staging on the next quiet point

The staging→live commit is one MC cycle. Reads can return either side depending on the field — observation fields (OBS_*) read live state; configuration fields (*_TUNING) read staging by default but switch to live after CTRL.config_apply is acknowledged.

## 27.3	TODO

- CSR YAML schema reference

- Generation script invocation

- Quiet-point handshake protocol detail

- Software access pattern (config-apply sequence)

- Generated UVM register model overview

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	Runtime Overrides and Quiet Points

Status: Skeleton — to be authored during MAS week

This section documents the runtime override mechanism — how a CSR write becomes a live datapath change without rebuilding or resetting.

## 28.1	Override Categories

Three flavors of override exist:

- Synthesized-MAX, runtime-active — the silicon includes the maximum-capability logic; runtime CSR selects how much of it is active. Examples: SCHED_TUNING.lookahead_active, REFRESH_TUNING.refresh_defer_active.

- Synthesized-set, runtime-mux — the silicon includes a discrete set of variants; runtime CSR picks one. Examples: ADDR_MAP_TUNING.scheme_or, REFRESH_TUNING.refpb_policy_or.

- Pure runtime — no silicon-area cost beyond the register itself. Examples: REFRESH_TUNING.zqcs_freq_hz, INIT_TUNING.zq_retries.

## 28.2	Quiet-Point Sequence

- Software writes the staging field via APB.

- Software writes CTRL.config_apply = 1.

- Controller drives txn_queue to drain, blocks new AXI accepts, completes any in-flight refresh.

- power_state_fub asserts quiet_point for one cycle.

- csr_apb_fub copies staging → live for every R/W field.

- STATUS.config_settled rises.

- Software reads STATUS.config_settled, clears CTRL.config_apply, and resumes traffic.

The drain phase is bounded — typically < 256 MC cycles for a healthy queue — and is the only operational disruption from a runtime override.

## 28.3	TODO

- Worst-case drain latency analysis

- Override propagation timing for each category

- Multi-write coalescing (if multiple staging writes are queued, commit them all at one quiet point)

- Software examples (init-time vs. runtime override patterns)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Family-Wide Config-Bit Applicability

Status: Skeleton — to be authored during MAS week

The full family-wide config-bit registry is in HAS §5.4. This MAS section documents the implementation-level shape of those bits in this controller specifically.

## 29.1	Per-Bit Implementation Notes for DDR2 / LPDDR2

Bits flagged (reserved) in HAS §5.4 ECC_TUNING are present in the CSR map at the documented offsets but read as zero and writes are ignored. The CSR slave does not raise pslverr on these — software portability requires writing the family-wide config word to succeed on flavors that haven’t implemented every bit.

Bits flagged DDR3+, DDR4+, LPDDR3+, LPDDR4-only likewise read as zero and ignore writes in this DDR2/LPDDR2 controller.

The 0xFF8 capability vector reports the build’s actual capabilities; software queries this once at boot to decide which family-wide writes are meaningful.

## 29.2	Soft-N/A Behavior Examples

- Writing SCHED_TUNING.bank_group_balance = 1 in DDR2 build: silently ignored, no pslverr. Read-back returns 0.

- Writing POWER_TUNING.dpd_enable = 1 in DDR2 build: silently ignored. (DPD is LPDDR-only.)

- Reading INIT_TUNING.cbt_enable in any v1 controller: returns 0. (LPDDR4 only.)

## 29.3	TODO

- Per-bit implementation note for every family-wide bit

- Test plan for each soft-N/A bit

- Software bring-up sequence demonstrating capability-vector-gated writes

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Initialization Sequence

Status: Skeleton — to be authored during MAS week

The init sequence from power-on through DRAM ready, as seen by software.

## 30.1	TODO

- Step-by-step software pseudocode

- Timing of each step (CSR write → controller state advance)

- Error-detection at each step

- Multi-rank init differences (per-rank ZQ, per-rank MR loads)

- Sim shortcut path (SIM_INIT_SCALE > 1)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	Refresh and Power-State Programming

Status: Skeleton — to be authored during MAS week

Programming the refresh subsystem and the power-state FSM from software.

## 31.1	TODO

- Setting tREFI from CSR

- Selecting REFab vs REFpb per rank

- LPDDR2 PASR mask programming per rank

- Temperature-derate read-back (MR4 / TEMP_DERATE_RANK)

- ZQCS frequency tuning

- Entering / exiting self-refresh

- Entering / exiting Deep Power Down (LPDDR2)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 32	Multi-Rank Programming

Status: Skeleton — to be authored during MAS week

The multi-rank-specific programming concerns: per-rank MR loads, per-rank PASR, cross-rank ODT, refresh fairness.

## 32.1	TODO

- Per-rank MR0/1/2/3 programming sequence

- Per-rank PASR programming

- ODT rule selection (ODT_RULE_MULTIRANK)

- Cross-rank ODT verification with the BFM

- Rank-disable semantics (RANK_TUNING.rank_enable_mask)

- Multi-rank refresh schedule observation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 33	Error Handling

Status: Skeleton — to be authored during MAS week

Error reporting paths and the software response model.

## 33.1	TODO

- Error categories: init failure, refresh-deadline miss, AXI queue overflow, DFI rddata-valid timeout

- IRQ topology (irq_init_done, irq_init_error, irq_overflow)

- Clearing latched errors via CSR

- Diagnosis sequence: history register (STATUS_HISTORY), debug observation FUBs

- Software recovery patterns (re-init, soft-reset, full reset)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 34	Build-Time Configuration Reference

Status: Skeleton — to be authored during MAS week

The full build-time parameter table is in HAS §5.2. This MAS section is the build-script view: how each parameter is wired through filelists, includes, and synthesis scripts.

## 34.1	TODO

- Parameter override on synthesis command line

- Filelist composition per parameter combination

- rtl/includes/ddr2_lpddr2_config.svh macro definitions

- Conditional FUB inclusion (e.g., page_predictor only when PAGE_POLICY == HAPPY_HYBRID)

- Per-memtype filelists

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 35	Runtime Configuration Reference

Status: Skeleton — to be authored during MAS week

The full runtime CSR is in HAS §6.3. This MAS section is the driver-author view: which fields can be tuned at runtime, in what order, and with what side-effects.

## 35.1	TODO

- Field-by-field runtime tuning cookbook

- Recommended bring-up sequence

- Recommended characterization sweep sequence

- Telemetry registers driving each tuning decision