RTL Design Sherpa

RAPIDS Beats Micro-Architecture Specification 0.6

July 2, 2026

Table of Contents

1 Rapids Beats Mas Index	34

2 Document Information	34

2.1 References	34

2.1.1 Related Documents	34

2.2 Terminology	35

2.3 “Beats” Architecture Concept	36

2.4 Revision History	37

3 Architecture Overview	38

3.1 Overview	38

3.1.1 Key Features	38

3.1.2 Block Diagram	39

3.1.3 Figure 1.1.1: RAPIDS Beats Architecture Block Diagram	39

3.2 Data Flow Overview	40

3.2.1 Sink Path (Network to Memory)	40

3.2.2 Figure 1.1.2: Sink Path Data Flow	41

3.2.3 Source Path (Memory to Network)	41

3.2.4 Figure 1.1.3: Source Path Data Flow	41

3.3 Key Architectural Decisions	42

3.3.1 Beat-Level Tracking	42

3.3.2 Concurrent Read/Write	42

3.3.3 Virtual FIFOs (Alloc/Drain)	42

3.4 Module Hierarchy	43

3.5 Timing Diagram: Basic Transfer	43

3.5.1 Figure 1.1.4: Basic Sink Path Transfer Timing	43

3.6 Related Documentation	44

4 Top-Level Port List	44

4.1 Parameters	44

4.1.1 Primary Parameters	44

4.1.2 Monitor Bus Parameters	45

4.2 Clock and Reset	45

4.3 APB Programming Interface	45

4.4 Configuration Interface	46

4.4.1 Per-Channel Configuration	46

4.4.2 Scheduler Configuration	46

4.4.3 Descriptor Engine Configuration	47

4.4.4 AXI Transfer Configuration	47

4.5 Status Interface	47

4.6 Sink Path Interface (Network to Memory)	48

4.6.1 Fill Interface (External to SRAM)	48

4.6.2 Sink AXI Write Master	48

4.7 Source Path Interface (Memory to Network)	50

4.7.1 Source AXI Read Master	50

4.7.2 Drain Interface (SRAM to External)	50

4.8 Descriptor AXI Master	51

4.9 MonBus Interface	52

5 Clocks and Reset	52

5.1 Clock Domain	52

5.1.1 Clock Specifications	53

5.1.2 Clock Distribution	53

5.2 Reset Specification	53

5.2.1 Reset Signal	53

5.2.2 Reset Timing Diagram	54

5.2.3 Figure 1.3.1: Reset Timing	54

5.3 Reset Behavior	54

5.3.1 On Reset Assert	54

5.3.2 Reset Sequence	55

5.4 Configuration During Reset	55

5.5 Reset Macros	55

5.6 Power-On Reset Considerations	56

6 Scheduler Specification	56

6.1 Overview	56

6.1.1 Key Features	56

6.1.2 Block Diagram	57

6.1.3 Figure 2.1.1: Scheduler Block Diagram	57

6.2 Parameters	57

6.3 Port List	58

6.3.1 Clock and Reset	58

6.3.2 Configuration Interface	58

6.3.3 Descriptor Engine Interface	59

6.3.4 Data Read Interface	59

6.3.5 Data Write Interface	60

6.4 FSM States	60

6.4.1 Figure 2.1.2: Scheduler FSM State Diagram	60

6.4.2 State Encoding	61

6.5 Operation	62

6.5.1 Descriptor Format	62

6.5.2 Transfer Sequence	62

6.5.3 Figure 2.1.3: Basic Transfer Timing	62

6.6 Error Handling	62

6.6.1 Recoverable Write-Progress Timeout	63

6.7 MonBus Events	63

7 Descriptor Engine Specification	64

7.1 Overview	64

7.1.1 Key Features	64

7.1.2 Block Diagram	64

7.1.3 Figure 2.2.1: Descriptor Engine Block Diagram	64

7.2 Parameters	65

7.3 Port List	65

7.3.1 Clock and Reset	65

7.3.2 APB Programming Interface	66

7.3.3 Configuration Interface	66

7.3.4 AXI4 Read Master Interface	66

7.3.5 Scheduler Interface	67

7.4 FSM States	67

7.4.1 Figure 2.2.2: Descriptor Engine FSM	67

7.5 Operation	68

7.5.1 Descriptor Chain Processing	68

7.5.2 Figure 2.2.3: Descriptor Chain Timing	68

7.6 Address Range Checking	69

7.7 Prefetch and Autonomous Chaining	69

7.7.1 Chaining Decision	69

7.7.2 Deferred (Throttle-Blocked) Chain Fetch	70

8 AXI Read Engine Specification	70

8.1 Overview	70

8.1.1 Key Features	70

8.1.2 Block Diagram	71

8.1.3 Figure 2.3.1: AXI Read Engine Block Diagram	71

8.2 Parameters	71

8.3 Port List	72

8.3.1 Clock and Reset	72

8.3.2 Scheduler Interface	72

8.3.3 AXI4 Read Master Interface	72

8.3.4 SRAM Write Interface	73

8.4 Operation	73

8.4.1 Streaming Pipeline Architecture	73

8.4.2 Timing Diagram	74

8.4.3 Figure 2.3.2: AXI Read Burst Timing	74

8.5 Burst Segmentation	74

8.6 Error Handling	74

9 AXI Write Engine Specification	75

9.1 Overview	75

9.1.1 Key Features	75

9.1.2 Block Diagram	75

9.1.3 Figure 2.4.1: AXI Write Engine Block Diagram	75

9.2 Parameters	76

9.3 Port List	76

9.3.1 Clock and Reset	76

9.3.2 Scheduler Interface	77

9.3.3 AXI4 Write Master Interface	78

9.3.4 SRAM Read Interface	78

9.4 Operation	79

9.4.1 Write Transaction Phases	79

9.4.2 Timing Diagram	79

9.4.3 Figure 2.4.2: AXI Write Burst Timing	79

9.5 Error Handling	80

10 Beats Alloc Control Specification	80

10.1 Overview	80

10.1.1 Key Features	80

10.1.2 Block Diagram	81

10.1.3 Figure 2.5.1: Beats Alloc Control Block Diagram	81

10.2 Concept: Virtual FIFO	81

10.2.1 Use Case: Pre-Allocation for AXI Reads	81

10.3 Parameters	82

10.4 Port List	82

10.4.1 Clock and Reset	82

10.4.2 Write Interface (Allocation Requests)	82

10.4.3 Read Interface (Data Written)	82

10.4.4 Status Outputs	83

10.5 Operation	83

10.5.1 Pointer Arithmetic	83

10.5.2 Timing Diagram	83

10.5.3 Figure 2.5.2: Allocation and Release Timing	83

10.6 Integration Example	84

11 Beats Drain Control Specification	85

11.1 Overview	85

11.1.1 Key Features	85

11.1.2 Block Diagram	85

11.1.3 Figure 2.6.1: Beats Drain Control Block Diagram	85

11.2 Concept: Data Availability Tracking	85

11.2.1 Complementary Roles	86

11.3 Parameters	86

11.4 Port List	86

11.4.1 Clock and Reset	86

11.4.2 Write Interface (Data Written to SRAM)	86

11.4.3 Read Interface (Drain Requests)	87

11.4.4 Status Outputs	87

11.5 Operation	87

11.5.1 Pointer Arithmetic	87

11.5.2 Timing Diagram	88

11.5.3 Figure 2.6.2: Data Arrival and Drain Timing	88

11.6 Integration Example	88

11.7 Relationship: alloc_ctrl vs drain_ctrl	89

12 Beats Latency Bridge Specification	89

12.1 Overview	89

12.1.1 Key Features	89

12.1.2 Block Diagram	90

12.1.3 Figure 2.7.1: Beats Latency Bridge Block Diagram	90

12.2 Concept: Why Latency Compensation?	90

12.3 Parameters	90

12.4 Port List	91

12.4.1 Clock and Reset	91

12.4.2 Input Interface	91

12.4.3 Output Interface	91

12.5 Operation	92

12.5.1 Timing Diagram	92

12.5.2 Figure 2.7.2: Latency Bridge Timing (Depth=2)	92

12.6 Integration Context	92

12.7 Design Considerations	93

13 Beats Scheduler Group Specification	93

13.1 Overview	93

13.1.1 Key Features	93

13.1.2 Block Diagram	94

13.1.3 Figure 3.1.1: Beats Scheduler Group Block Diagram	94

13.2 Parameters	94

13.3 Port List	95

13.3.1 Clock and Reset	95

13.3.2 APB Programming Interface	95

13.3.3 Configuration Interface	95

13.3.4 Descriptor AXI Master Interface	96

13.3.5 Scheduler Data Interfaces	96

13.3.6 Status	97

13.3.7 MonBus Interface	97

13.4 Internal Architecture	98

14 Beats Scheduler Group Array Specification	98

14.1 Overview	98

14.1.1 Key Features	98

14.1.2 Block Diagram	99

14.1.3 Figure 3.2.1: Beats Scheduler Group Array Block Diagram	99

14.2 Parameters	100

14.3 Port List	100

14.3.1 Clock and Reset	100

14.3.2 Per-Channel APB Programming	101

14.3.3 Per-Channel Configuration	101

14.3.4 Shared Descriptor AXI Master Interface	102

14.3.5 Per-Channel Scheduler Data Interfaces	102

14.3.6 Aggregate Status	103

14.3.7 Unified MonBus Interface	103

14.4 AXI Arbitration	104

14.4.1 Figure 3.2.2: Descriptor AXI Arbitration Sequence	104

14.5 MonBus Aggregation	104

14.6 Integration Context	105

15 Sink Data Path Specification	106

15.1 Overview	106

15.1.1 Key Features	106

15.1.2 Block Diagram	106

15.1.3 Figure 3.3.1: Sink Data Path Block Diagram	106

15.2 Parameters	107

15.3 Data Flow	107

15.3.1 Figure 3.3.2: Sink Path Data Flow Sequence	107

15.4 Timing Diagram	108

15.4.1 Figure 3.3.3: Sink Path Transfer Timing	108

15.5 Interface Summary	109

15.5.1 Fill Interface (Input)	109

15.5.2 Scheduler Interface	109

16 Sink Data Path AXIS Wrapper Specification	110

16.1 Overview	110

16.1.1 Key Features	110

16.1.2 Block Diagram	110

16.1.3 Figure 3.4.1: Sink Data Path AXIS Block Diagram	110

16.2 Parameters	111

16.3 Port List	112

16.3.1 Clock and Reset	112

16.3.2 AXI-Stream Slave Interface	112

16.3.3 Space Allocation Interface	112

16.3.4 Scheduler Interface	113

16.3.5 AXI Write Master Interface	113

16.4 Signal Mapping	114

16.4.1 Figure 3.4.2: AXIS to Fill Interface Mapping	114

16.5 Timing Diagram	114

16.5.1 Figure 3.4.3: AXIS Ingress Timing	114

16.6 Usage Notes	115

17 Sink SRAM Controller Specification	115

17.1 Overview	115

17.1.1 Key Features	115

17.1.2 Block Diagram	116

17.1.3 Figure 3.5.1: Sink SRAM Controller Block Diagram	116

17.2 Parameters	117

17.3 Port List	117

17.3.1 Clock and Reset	117

17.3.2 Per-Channel Fill Interface	117

17.3.3 Arbitrated Drain Interface (to AXI Write Engine)	118

17.3.4 Per-Channel Status	118

17.4 Arbitration Logic	119

17.4.1 Figure 3.5.2: Channel Arbitration Timing	119

17.5 Integration with AXI Write Engine	119

18 Sink SRAM Controller Unit Specification	120

18.1 Overview	120

18.1.1 Key Features	120

18.1.2 Block Diagram	120

18.1.3 Figure 3.6.1: Sink SRAM Controller Unit Block Diagram	120

18.2 Parameters	122

18.3 Port List	122

18.3.1 Clock and Reset	122

18.3.2 Fill Interface (from Network)	122

18.3.3 Drain Interface (to AXI Write Engine)	123

18.3.4 Status	123

18.4 Internal Architecture	124

18.4.1 Figure 3.6.2: Internal Data Flow	124

18.5 Timing Diagram	124

18.5.1 Figure 3.6.3: Fill and Drain Timing	124

18.6 Virtual FIFO Concept	125

18.6.1 Figure 3.6.4: Virtual FIFO Operation	125

18.7 Integration Example	125

19 Source Data Path Specification	126

19.1 Overview	126

19.1.1 Key Features	126

19.1.2 Block Diagram	127

19.1.3 Figure 3.7.1: Source Data Path Block Diagram	127

19.2 Parameters	127

19.3 Data Flow	128

19.3.1 Figure 3.7.2: Source Path Data Flow Sequence	128

19.4 Port List	129

19.4.1 Clock and Reset	129

19.4.2 Scheduler Interface	129

19.4.3 AXI Read Master Interface	129

19.4.4 Drain Interface (Output to Network)	130

19.5 Timing Diagram	130

19.5.1 Figure 3.7.3: Source Path Transfer Timing	130

19.6 Integration Example	131

20 Source Data Path AXIS Wrapper Specification	132

20.1 Overview	132

20.1.1 Key Features	132

20.1.2 Block Diagram	132

20.1.3 Figure 3.8.1: Source Data Path AXIS Block Diagram	132

20.2 Parameters	133

20.3 Port List	134

20.3.1 Clock and Reset	134

20.3.2 Scheduler Interface	134

20.3.3 AXI Read Master Interface	134

20.3.4 AXI-Stream Master Interface	135

20.4 Signal Mapping	135

20.4.1 Figure 3.8.2: Drain to AXIS Interface Mapping	135

20.5 Timing Diagram	136

20.5.1 Figure 3.8.3: AXIS Egress Timing	136

20.6 Usage Notes	136

20.7 Integration Example	136

21 Source SRAM Controller Specification	137

21.1 Overview	138

21.1.1 Key Features	138

21.1.2 Block Diagram	138

21.1.3 Figure 3.9.1: Source SRAM Controller Block Diagram	138

21.2 Parameters	139

21.3 Port List	139

21.3.1 Clock and Reset	139

21.3.2 Per-Channel Fill Interface (from AXI Read Engine)	140

21.3.3 Arbitrated Drain Interface (to Network)	140

21.3.4 Per-Channel Status	141

21.4 Arbitration Logic	141

21.4.1 Figure 3.9.2: Source Channel Arbitration	141

21.5 Comparison: Sink vs Source SRAM Controller	141

21.6 Integration Example	142

22 Source SRAM Controller Unit Specification	143

22.1 Overview	143

22.1.1 Key Features	143

22.1.2 Block Diagram	143

22.1.3 Figure 3.10.1: Source SRAM Controller Unit Block Diagram	143

22.2 Parameters	144

22.3 Port List	145

22.3.1 Clock and Reset	145

22.3.2 Fill Interface (from AXI Read Engine)	145

22.3.3 Drain Interface (to Network)	145

22.3.4 Status	146

22.4 Internal Architecture	146

22.4.1 Figure 3.10.2: Internal Data Flow	146

22.5 Comparison: Sink vs Source Unit	147

22.6 Timing Diagram	147

22.6.1 Figure 3.10.3: Source Unit Fill and Drain Timing	147

22.7 Integration Example	148

23 RAPIDS Core Beats Specification	149

23.1 Overview	149

23.1.1 Key Features	149

23.1.2 Block Diagram	149

23.1.3 Figure 3.11.1: RAPIDS Core Beats Block Diagram	149

23.2 Parameters	150

23.3 Port List	151

23.3.1 Clock and Reset	151

23.3.2 Per-Channel APB Programming	151

23.3.3 Per-Channel Configuration	152

23.3.4 Descriptor AXI Master Interface	152

23.3.5 Sink AXI Write Master Interface	153

23.3.6 Source AXI Read Master Interface	154

23.3.7 Sink Fill Interface (or AXIS Slave)	154

23.3.8 Source Drain Interface (or AXIS Master)	154

23.3.9 Unified MonBus Interface	155

23.3.10 Aggregate Status	155

23.4 Data Flow	156

23.4.1 Figure 3.11.2: RAPIDS Core Complete Data Flow	156

23.5 MonBus Aggregation	156

23.6 Integration Example	157

24 RAPIDS Registers Specification	159

24.1 Overview	159

24.2 Base Register Map (0x100-0x3FF)	159

24.2.1 Key Register Fields	161

24.3 Monitor Register Map (rapids_mon_regs @ 0x1000)	162

25 RAPIDS Config Block Specification	163

25.1 Overview	163

25.2 Mapping Groups	164

25.2.1 Base Configuration	164

25.2.2 Monitor Configuration (hwif_out.MON.*)	164

25.2.3 Performance and Observation	164

26 RAPIDS Beats Top Specification	165

26.1 Overview	165

26.2 Integration Datapath	165

26.2.1 Figure 3.14.1: RAPIDS Beats Top Integration	165

26.2.2 Address Decode	166

26.3 AXI Monitors (USE_AXI_MONITORS)	166

26.4 Parameters	167

26.5 Top-Level Interfaces	167

27 Interface Overview	168

27.1 External Interfaces	169

27.1.1 AXI4 Master Interfaces	169

27.1.2 Streaming Interfaces	169

27.1.3 Control/Monitor Interfaces	169

27.2 Interface Documentation	170

27.3 Interface Signal Naming Conventions	170

28 AXI4 Interface Specification	170

28.1 Overview	171

28.2 Descriptor AXI Master Interface	171

28.2.1 Purpose	171

28.2.2 Signal Table	171

28.2.3 Transaction Characteristics	172

28.2.4 Timing Diagram	172

28.2.5 Figure 4.1.1: Descriptor Fetch Timing	172

28.3 Sink AXI Write Master Interface	173

28.3.1 Purpose	173

28.3.2 Signal Table	173

28.3.3 Transaction Characteristics	174

28.3.4 Timing Diagram	174

28.3.5 Figure 4.1.2: Sink AXI Write Burst Timing	174

28.4 Source AXI Read Master Interface	174

28.4.1 Purpose	174

28.4.2 Signal Table	175

28.4.3 Transaction Characteristics	175

28.4.4 Timing Diagram	176

28.4.5 Figure 4.1.3: Source AXI Read Burst Timing	176

28.5 AXI Protocol Compliance	176

28.5.1 Supported Features	176

28.5.2 Error Handling	177

29 AXI-Stream Interface Specification	177

29.1 Overview	177

29.2 AXIS Sink Slave Interface	177

29.2.1 Purpose	177

29.2.2 Signal Table	177

29.2.3 Signal Details	178

29.2.4 Timing Diagram	179

29.2.5 Figure 4.2.1: AXIS Sink Slave Timing	179

29.2.6 Backpressure	179

29.3 AXIS Source Master Interface	179

29.3.1 Purpose	179

29.3.2 Signal Table	179

29.3.3 Timing Diagram	180

29.3.4 Figure 4.2.2: AXIS Source Master Timing	180

29.3.5 Flow Control	180

29.4 AXIS vs Custom Fill/Drain Interfaces	181

29.4.1 Comparison	181

29.4.2 Selection Guide	181

29.5 Configuration Parameters	181

30 MonBus Interface Specification	182

30.1 Overview	182

30.2 MonBus Signal Interface	182

30.2.1 Signal Table	182

30.2.2 Handshaking Protocol	182

30.2.3 Timing Diagram	182

30.2.4 Figure 4.3.1: MonBus Packet Transfer Timing	182

30.3 MonBus Packet Format	183

30.3.1 64-bit Packet Structure	183

30.3.2 Figure 4.3.2: MonBus Packet Format	183

30.4 Packet Types	183

30.4.1 Type Code Definitions	183

30.4.2 Payload Formats by Type	184

30.5 MonBus Source Aggregation	185

30.5.1 RAPIDS Core MonBus Sources	185

30.5.2 Aggregation Hierarchy	186

30.5.3 Figure 4.3.3: MonBus Aggregation Tree	186

30.6 Integration Guidelines	186

30.6.1 Consumer Requirements	186

30.6.2 Recommended Consumer Patterns	186

30.7 Timing Requirements	187

31 Product Requirements Document (PRD)	188

31.1 RAPIDS - Rapid AXI Programmable In-band Descriptor System	188

31.2 1. Executive Summary	188

31.2.1 1.1 Quick Stats	188

31.2.2 1.2 Subsystem Goals	188

31.3 2. Documentation Structure	188

31.3.1 📚 Complete RAPIDS Specification	188

31.3.2 🐛 Known Issues	189

31.3.3 📖 Other Documentation	190

31.4 2.4 Organizational Standards - RAPIDS Code Location	190

31.4.1 Code Organization Principle	190

31.4.2 RAPIDS Directory Structure	190

31.4.3 What Goes Where?	191

31.4.4 Import Pattern for RAPIDS Tests	191

31.4.5 Benefits of This Organization	192

31.4.6 Compliance Status	192

31.5 3. Architecture Overview	192

31.5.1 3.1 Top-Level Block Diagram	192

31.5.2 3.2 Data Flow	193

31.6 4. Key Features	193

31.6.1 4.1 Descriptor-Based Operation	193

31.6.2 4.2 Data Path Features	193

31.6.3 4.3 Scheduler Features	194

31.6.4 4.4 Monitoring Integration	194

31.7 5. Interfaces	194

31.7.1 5.1 External Interfaces	194

31.7.2 5.2 Configuration Parameters	195

31.8 6. Use Cases	196

31.8.1 6.1 DMA-Style Transfers	196

31.8.2 6.2 Network Packet Processing	196

31.8.3 6.3 Custom Data Path Acceleration	196

31.9 7. Test Coverage	196

31.9.1 7.1 Current Status	196

31.9.2 7.2 Test Strategy	197

31.10 8. Known Issues Summary	197

31.10.1 8.1 Critical Issues	197

31.10.2 8.2 Medium Priority Issues	198

31.11 9. Integration Guidelines	198

31.11.1 9.1 Quick Start	198

31.11.2 9.2 Configuration Steps	199

31.12 10. Development Status	199

31.12.1 10.1 Current Phase	199

31.12.2 10.2 Roadmap	200

31.13 11. Performance Characteristics	200

31.13.1 11.1 Throughput	200

31.13.2 11.2 Latency	200

31.13.3 11.3 Resource Utilization	200

31.14 12. Verification Infrastructure	200

31.14.1 12.1 Test Organization	200

31.14.2 12.2 CocoTB Framework	201

31.14.3 12.2.1 MANDATORY: BFM Usage for FUB Tests	201

31.14.4 12.3 Test File Structure (Standard Pattern)	202

31.15 13. Quick Reference	207

31.15.1 13.1 Key Files	207

31.15.2 13.2 Commands	207

31.16 14. Success Criteria	208

31.16.1 14.1 Functional	208

31.16.2 14.2 Quality	208

31.16.3 14.3 Documentation	208

31.17 15. Educational Value	208

31.18 15. Attribution and Contribution Guidelines	209

31.18.1 15.1 Git Commit Attribution	209

31.19 16. Documentation Generation	209

31.19.1 16.1 Generating PDF/DOCX from Specification	209

31.20 16.2 PDF Generation Location	210

31.21 17. References	211

31.21.1 16.1 Internal Documentation	211

31.21.2 16.2 Related Subsystems	211

31.21.3 16.3 External References	211

31.22 Navigation	211

32 Claude Code Guide: RAPIDS Subsystem	212

32.1 Quick Context	212

32.2 📖 Global Requirements Reference	212

32.3 Critical Rules for This Subsystem	213

32.3.1 Rule #0: Attribution Format for Git Commits	213

32.3.2 Rule #0.1: Testbench Location and Test Structure (MANDATORY)	213

32.3.3 Rule #0.5: Three Mandatory TB Methods (MANDATORY)	215

32.3.4 Rule #0.75: Audit Signal Naming BEFORE Writing Testbenches	216

32.3.5 Rule #1: MANDATORY BFM Usage for FUB-Level Tests	217

32.3.6 Rule #2: Always Reference Detailed Specification	220

32.3.7 Rule #3: Know the Known Issues	220

32.3.8 Rule #4: RAPIDS is Complex - Understand Block Interactions	220

32.3.9 Rule #5: Test Strategy is Multi-Layered	221

32.4 Architecture Quick Reference	221

32.4.1 Block Organization	221

32.4.2 Module Quick Reference	222

32.4.3 Interface Summary	223

32.5 Common User Questions and Responses	223

32.5.1 Q: “How does the scheduler work?”	223

32.5.2 Q: “How do I configure RAPIDS?”	224

32.5.3 Q: “What’s the data flow for network to memory transfer?”	225

32.5.4 Q: “What’s the credit counter bug and how does exponential encoding work?”	225

32.5.5 Q: “How do I run RAPIDS tests?”	226

32.6 Integration Patterns	227

32.6.1 Pattern 1: Basic RAPIDS Instantiation	227

32.6.2 Pattern 2: Configuration Sequence	228

32.6.3 Pattern 3: MonBus Integration	229

32.7 Anti-Patterns to Catch	229

32.7.1 ✗ Anti-Pattern 1: Not Understanding Exponential Credit Encoding	229

32.7.2 ✗ Anti-Pattern 2: Insufficient SRAM Depth	230

32.7.3 ✗ Anti-Pattern 3: No MonBus Downstream Handling	230

32.7.4 ✗ Anti-Pattern 4: Testing Individual Blocks in Isolation	230

32.8 Debugging Workflow	230

32.8.1 Issue: Scheduler Not Processing Descriptors	230

32.8.2 Issue: Data Path Stalls	231

32.8.3 Issue: MonBus Packets Not Generated	231

32.9 Testing Guidance	231

32.9.1 Test Organization	231

32.9.2 Running Tests	232

32.9.3 Test Coverage Status	232

32.10 Key Documentation Links	233

32.10.1 Always Reference These	233

32.11 Quick Commands	233

32.12 Verification Patterns and Best Practices	234

32.12.1 Pattern: Continuous Background Monitoring	234

32.12.2 Pattern: Using Existing CocoTBFramework Components	235

32.12.3 Test Scalability Patterns	237

32.13 Documentation Generation	239

32.13.1 Generating PDF/DOCX from Specification	239

32.14 PDF Generation Location	241

32.15 Remember	241

List of Figures

Figure 1.1.1: RAPIDS Beats Architecture Block Diagram	39

Figure 1.1.2: Sink Path Data Flow	41

Figure 1.1.3: Source Path Data Flow	41

Figure 1.1.4: Basic Sink Path Transfer Timing	43

Figure 1.3.1: Reset Timing	54

Figure 2.1.1: Scheduler Block Diagram	57

Figure 2.1.2: Scheduler FSM State Diagram	60

Figure 2.1.3: Basic Transfer Timing	62

Figure 2.2.1: Descriptor Engine Block Diagram	64

Figure 2.2.2: Descriptor Engine FSM	67

Figure 2.2.3: Descriptor Chain Timing	68

Figure 2.3.1: AXI Read Engine Block Diagram	71

Figure 2.3.2: AXI Read Burst Timing	74

Figure 2.4.1: AXI Write Engine Block Diagram	75

Figure 2.4.2: AXI Write Burst Timing	79

Figure 2.5.1: Beats Alloc Control Block Diagram	81

Figure 2.5.2: Allocation and Release Timing	83

Figure 2.6.1: Beats Drain Control Block Diagram	85

Figure 2.6.2: Data Arrival and Drain Timing	88

Figure 2.7.1: Beats Latency Bridge Block Diagram	90

Figure 2.7.2: Latency Bridge Timing (Depth=2)	92

Figure 3.1.1: Beats Scheduler Group Block Diagram	94

Figure 3.2.1: Beats Scheduler Group Array Block Diagram	99

Figure 3.2.2: Descriptor AXI Arbitration Sequence	104

Figure 3.3.1: Sink Data Path Block Diagram	106

Figure 3.3.2: Sink Path Data Flow Sequence	107

Figure 3.3.3: Sink Path Transfer Timing	108

Figure 3.4.1: Sink Data Path AXIS Block Diagram	110

Figure 3.4.2: AXIS to Fill Interface Mapping	114

Figure 3.4.3: AXIS Ingress Timing	114

Figure 3.5.1: Sink SRAM Controller Block Diagram	116

Figure 3.5.2: Channel Arbitration Timing	119

Figure 3.6.1: Sink SRAM Controller Unit Block Diagram	120

Figure 3.6.2: Internal Data Flow	124

Figure 3.6.3: Fill and Drain Timing	124

Figure 3.6.4: Virtual FIFO Operation	125

Figure 3.7.1: Source Data Path Block Diagram	127

Figure 3.7.2: Source Path Data Flow Sequence	128

Figure 3.7.3: Source Path Transfer Timing	130

Figure 3.8.1: Source Data Path AXIS Block Diagram	132

Figure 3.8.2: Drain to AXIS Interface Mapping	135

Figure 3.8.3: AXIS Egress Timing	136

Figure 3.9.1: Source SRAM Controller Block Diagram	138

Figure 3.9.2: Source Channel Arbitration	141

Figure 3.10.1: Source SRAM Controller Unit Block Diagram	143

Figure 3.10.2: Internal Data Flow	146

Figure 3.10.3: Source Unit Fill and Drain Timing	147

Figure 3.11.1: RAPIDS Core Beats Block Diagram	149

Figure 3.11.2: RAPIDS Core Complete Data Flow	156

Figure 3.14.1: RAPIDS Beats Top Integration	165

Figure 4.1.1: Descriptor Fetch Timing	172

Figure 4.1.2: Sink AXI Write Burst Timing	174

Figure 4.1.3: Source AXI Read Burst Timing	176

Figure 4.2.1: AXIS Sink Slave Timing	179

Figure 4.2.2: AXIS Source Master Timing	180

Figure 4.3.1: MonBus Packet Transfer Timing	182

Figure 4.3.2: MonBus Packet Format	183

Figure 4.3.3: MonBus Aggregation Tree	186

List of Tables

Table 1: Table 0.1: Related Documents and Specifications	34

Table 2: Table 0.2: RAPIDS Beats MAS Document Revision History	37

Table 3: Table 1.1.1: Beat Granularity Examples	42

Table 4: Table 1.2.3: Clock and Reset Signals	45

Table 5: Table 1.2.4: APB Programming Interface	45

Table 6: Table 1.2.5: Per-Channel Configuration	46

Table 7: Table 1.2.6: Scheduler Configuration	46

Table 8: Table 1.2.7: Descriptor Engine Configuration	47

Table 9: Table 1.2.8: AXI Transfer Configuration	47

Table 10: Table 1.2.9: Status Interface	47

Table 11: Table 1.2.10: Sink Fill Interface	48

Table 12: Table 1.2.11: Sink AXI Write Master Interface	48

Table 13: Table 1.2.12: Source AXI Read Master Interface	50

Table 14: Table 1.2.13: Source Drain Interface	50

Table 15: Table 1.2.14: Descriptor AXI Master Interface	51

Table 16: Table 1.2.15: MonBus Interface	52

Table 17: Table 1.3.1: Clock Specifications	53

Table 18: Table 1.3.2: Reset Specifications	53

Table 19: Table 1.3.3: Reset States	54

Table 20: Table 1.3.4: Configuration Timing Requirements	55

Table 21: Table 2.1.2: Clock and Reset	58

Table 22: Table 2.1.3: Configuration Interface	58

Table 23: Table 2.1.4: Descriptor Engine Interface	59

Table 24: Table 2.1.5: Data Read Interface	59

Table 25: Table 2.1.6: Data Write Interface	60

Table 26: Table 2.1.7: FSM State Encoding	61

Table 27: Table 2.1.8: Error Handling	62

Table 28: Table 2.1.9: MonBus Event Codes	63

Table 29: Table 2.2.2: Clock and Reset	65

Table 30: Table 2.2.3: APB Programming Interface	66

Table 31: Table 2.2.4: Configuration Interface	66

Table 32: Table 2.2.5: AXI4 Read Master Interface	66

Table 33: Table 2.2.6: Scheduler Interface	67

Table 34: Table 2.3.2: Clock and Reset	72

Table 35: Table 2.3.3: Scheduler Interface	72

Table 36: Table 2.3.4: AXI4 Read Master Interface	72

Table 37: Table 2.3.5: SRAM Write Interface	73

Table 38: Table 2.3.6: Error Handling	74

Table 39: Table 2.4.2: Clock and Reset	76

Table 40: Table 2.4.3: Scheduler Interface	77

Table 41: Table 2.4.4: AXI4 Write Master Interface	78

Table 42: Table 2.4.5: SRAM Read Interface	78

Table 43: Table 2.4.6: Error Handling	80

Table 44: Table 2.5.2: Clock and Reset	82

Table 45: Table 2.5.3: Write Interface	82

Table 46: Table 2.5.4: Read Interface	82

Table 47: Table 2.5.5: Status Outputs	83

Table 48: Table 2.6.2: Clock and Reset	86

Table 49: Table 2.6.3: Write Interface	86

Table 50: Table 2.6.4: Read Interface	87

Table 51: Table 2.6.5: Status Outputs	87

Table 52: Table 2.6.6: alloc_ctrl vs drain_ctrl Comparison	89

Table 53: Table 2.7.2: Clock and Reset	91

Table 54: Table 2.7.3: Input Interface	91

Table 55: Table 2.7.4: Output Interface	91

Table 56: Table 2.7.5: Bridge Depth Selection	93

Table 57: Table 3.1.2: Clock and Reset	95

Table 58: Table 3.1.3: APB Programming Interface	95

Table 59: Table 3.1.4: Configuration Interface	95

Table 60: Table 3.1.5: Descriptor AXI Master Interface	96

Table 61: Table 3.1.6: Scheduler Data Interfaces	96

Table 62: Table 3.1.7: Status Interface	97

Table 63: Table 3.1.8: MonBus Interface	97

Table 64: Table 3.2.2: Clock and Reset	100

Table 65: Table 3.2.3: APB Programming Interface	101

Table 66: Table 3.2.4: Per-Channel Configuration	101

Table 67: Table 3.2.5: Shared Descriptor AXI Master Interface	102

Table 68: Table 3.2.6: Per-Channel Scheduler Data Interfaces	102

Table 69: Table 3.2.7: Aggregate Status	103

Table 70: Table 3.2.8: Unified MonBus Interface	103

Table 71: Table 3.2.9: MonBus Source Assignment	104

Table 72: Table 3.3.2: Fill Interface	109

Table 73: Table 3.3.3: Scheduler Interface	109

Table 74: Table 3.4.2: Clock and Reset	112

Table 75: Table 3.4.3: AXI-Stream Slave Interface	112

Table 76: Table 3.4.4: Space Allocation Interface	112

Table 77: Table 3.4.5: Scheduler Interface	113

Table 78: Table 3.4.6: AXI Write Master Interface	113

Table 79: Table 3.5.2: Clock and Reset	117

Table 80: Table 3.5.3: Per-Channel Fill Interface	117

Table 81: Table 3.5.4: Arbitrated Drain Interface	118

Table 82: Table 3.5.5: Per-Channel Status	118

Table 83: Table 3.6.2: Clock and Reset	122

Table 84: Table 3.6.3: Fill Interface	122

Table 85: Table 3.6.4: Drain Interface	123

Table 86: Table 3.6.5: Status Interface	123

Table 87: Table 3.7.2: Clock and Reset	129

Table 88: Table 3.7.3: Scheduler Interface	129

Table 89: Table 3.7.4: AXI Read Master Interface	129

Table 90: Table 3.7.5: Drain Interface	130

Table 91: Table 3.8.2: Clock and Reset	134

Table 92: Table 3.8.3: Scheduler Interface	134

Table 93: Table 3.8.4: AXI Read Master Interface	134

Table 94: Table 3.8.5: AXI-Stream Master Interface	135

Table 95: Table 3.9.2: Clock and Reset	139

Table 96: Table 3.9.3: Per-Channel Fill Interface	140

Table 97: Table 3.9.4: Arbitrated Drain Interface	140

Table 98: Table 3.9.5: Per-Channel Status	141

Table 99: Table 3.9.6: Sink vs Source Controller Comparison	141

Table 100: Table 3.10.2: Clock and Reset	145

Table 101: Table 3.10.3: Fill Interface	145

Table 102: Table 3.10.4: Drain Interface	145

Table 103: Table 3.10.5: Status Interface	146

Table 104: Table 3.10.6: Sink vs Source Unit Comparison	147

Table 105: Table 3.11.2: Clock and Reset	151

Table 106: Table 3.11.3: APB Programming Interface	151

Table 107: Table 3.11.4: Per-Channel Configuration	152

Table 108: Table 3.11.5: Descriptor AXI Master Interface	152

Table 109: Table 3.11.6: Sink AXI Write Master Interface	153

Table 110: Table 3.11.7: Source AXI Read Master Interface	154

Table 111: Table 3.11.8: Sink Fill Interface	154

Table 112: Table 3.11.9: Source Drain Interface	154

Table 113: Table 3.11.10: Unified MonBus Interface	155

Table 114: Table 3.11.11: Aggregate Status	155

Table 115: Table 3.11.12: MonBus Source Assignment	156

Table 116: Table 3.12.1: Base Register Map	159

Table 117: Table 3.12.2: Monitor Register Map (base 0x1000)	162

Table 118: Table 3.13.1: Monitor Register-to-Config Mapping	164

Table 119: Table 3.14.1: Top-Level Address Decode	166

Table 120: Table 3.14.3: Top-Level Interfaces	167

Table 121: Table 4.0.1: AXI4 Master Interfaces	169

Table 122: Table 4.0.2: Streaming Interfaces	169

Table 123: Table 4.0.3: Control and Monitor Interfaces	169

Table 124: Table 4.0.4: Signal Naming Conventions	170

Table 125: Table 4.1.1: Descriptor AXI Master Interface Signals	171

Table 126: Table 4.1.2: Descriptor AXI Transaction Characteristics	172

Table 127: Table 4.1.3: Sink AXI Write Master Interface Signals	173

Table 128: Table 4.1.4: Sink AXI Write Transaction Characteristics	174

Table 129: Table 4.1.5: Source AXI Read Master Interface Signals	175

Table 130: Table 4.1.6: Source AXI Read Transaction Characteristics	175

Table 131: Table 4.1.7: AXI Feature Support	176

Table 132: Table 4.2.1: AXIS Sink Slave Interface Signals	177

Table 133: Table 4.2.2: TID Channel Mapping	178

Table 134: Table 4.2.3: TKEEP Configuration	179

Table 135: Table 4.2.4: AXIS Source Master Interface Signals	179

Table 136: Table 4.2.5: Interface Comparison	181

Table 137: Table 4.2.6: Interface Selection Guide	181

Table 138: Table 4.3.1: MonBus Interface Signals	182

Table 139: Table 4.3.2: MonBus Packet Fields	183

Table 140: Table 4.3.3: MonBus Packet Types	183

Table 141: Table 4.3.4: Descriptor Packet Payload	184

Table 142: Table 4.3.5: Scheduler Packet Payload	184

Table 143: Table 4.3.6: AXI Event Packet Payload	185

Table 144: Table 4.3.7: Error Packet Payload	185

Table 145: Table 4.3.8: MonBus Source Assignment	185

Table 146: Table 4.3.9: MonBus Timing Parameters	187

List of Waveforms

No waveforms in this document.

# 1	Rapids Beats Mas Index

Generated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

This document describes the RAPIDS “Beats” architecture, a Phase 1 implementation of the Rapid AXI Programmable In-band Descriptor System. The beats architecture provides network-to-memory and memory-to-network data transfer capabilities using descriptor-based DMA with 8-channel support. It emphasizes “beat-level” tracking for precise flow control and latency management.

## 2.1	References

### 2.1.1	Related Documents

Table 0.1: Related Documents and Specifications

| Source | Title | Version |
| --- | --- | --- |
| RTL Design Sherpa | RAPIDS Product Requirements Document | 1.0 |
| RTL Design Sherpa | RAPIDS Original Specification | 0.25 |
| RTL Design Sherpa | STREAM MAS (Reference Architecture) | 0.90 |
| ARM | AMBA AXI and ACE Protocol Specification | IHI0022H |
| ARM | AMBA AXI-Stream Protocol Specification | IHI0051A |

## 2.2	Terminology

AXIS

AXI-Stream. ARM’s streaming protocol for high-bandwidth, low-latency data transfer without addressing.

AXI4

Version 4 of the AXI protocol, supporting burst transfers up to 256 beats.

Beat

A single data transfer within an AXI burst. For RAPIDS with 512-bit data width, one beat = 64 bytes.

Beats Architecture

The Phase 1 RAPIDS implementation that tracks transfers at beat granularity for precise flow control.

Burst

A group of consecutive data transfers (beats) initiated by a single address phase.

Channel

One of 8 independent DMA channels in RAPIDS. Each channel has its own descriptor chain and SRAM buffer allocation.

Descriptor

A 256-bit data structure containing transfer parameters: address, length, channel ID, and control flags.

Descriptor Chain

A linked list of descriptors where each descriptor points to the next, enabling scatter-gather operations.

DMA

Direct Memory Access. Hardware-controlled data movement between memory locations without CPU intervention.

FUB

Functional Unit Block. A self-contained RTL module with defined interfaces and testbench.

Latency Bridge

A module that provides buffering to compensate for pipeline latency between alloc/drain controllers.

MAC

Macro. An integration-level block that instantiates and connects multiple FUBs.

MonBus

Monitor Bus. A 64-bit internal bus for performance monitoring and debug event reporting.

RAPIDS

Rapid AXI Programmable In-band Descriptor System. A DMA accelerator for network-to-memory transfers.

Sink Path

The data path from network (AXIS slave) to system memory (AXI write master).

Source Path

The data path from system memory (AXI read master) to network (AXIS master).

SRAM

Static Random Access Memory. Used for internal data buffering between network and memory interfaces.

Virtual FIFO

A tracking mechanism (alloc_ctrl/drain_ctrl) that manages SRAM space without moving actual data.

## 2.3	“Beats” Architecture Concept

The “beats” architecture name reflects the key design decision to track all transfers at beat granularity (data-width units) rather than byte or chunk granularity:

- Beat-level Allocation: Space is allocated in beats (e.g., 64-byte units for 512-bit datapath)

- Beat-level Drain: Data availability is tracked in beats

- Beat-level Latency: The latency bridge compensates for pipeline delays in beat counts

- Beat-level Completion: AXI engines report completion in beats

This simplifies the math and aligns naturally with AXI burst semantics where each beat represents one data-width transfer.

## 2.4	Revision History

Revisions follow the convention x.y, where x is the major version and y is the minor version.

Table 0.2: RAPIDS Beats MAS Document Revision History

| Rev | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.25 | 2025-01-10 | seang | Initial RAPIDS Beats MAS structure |
| 0.6 | 2026-07-02 | seang | Resynced to RTL: scheduler recoverable write-progress timeout (cfg_sched_timeout_limit, strike escalation, scheduler_idle excludes CH_ERROR) and B-response commit gating; functional descriptor prefetch (cfg_prefetch_enable/cfg_fifo_threshold, deferred chaining); AXI write engine COMMIT strobes; commit/timeout-limit plumbing through rapids_core_beats; added rapids_regs register block (monitors @ 0x1000), rapids_config_block, and rapids_beats_top integration with the MonBus AXI-Lite group |

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Architecture Overview

Module: rapids_core_beats.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 3.1	Overview

The RAPIDS “Beats” architecture is a Phase 1 implementation providing network-to-memory and memory-to-network data transfer capabilities. The name “beats” reflects the design decision to track all transfers at beat granularity (data-width units) for simplified flow control.

### 3.1.1	Key Features

- 8 Independent Channels: Each channel has its own descriptor chain and SRAM allocation

- Separate Data Paths: Sink (network-to-memory) and source (memory-to-network) paths

- Beat-Level Tracking: All flow control operates at beat granularity

- SRAM Buffering: Separate sink and source SRAM buffers for data staging

- MonBus Integration: Comprehensive monitoring for all subsystems

- Streaming Pipelines: No FSM in data engines - pure streaming for maximum throughput

### 3.1.2	Block Diagram

### 3.1.3	Figure 1.1.1: RAPIDS Beats Architecture Block Diagram

rapids_core_beats
    +-------------------------------------------------------------------------+
    |                                                                         |
    |  +-------------------------------------------------------------------+  |
    |  |              beats_scheduler_group_array (8 channels)             |  |
    |  |  +----------------+  +----------------+       +----------------+  |  |
    |  |  | scheduler_grp  |  | scheduler_grp  |  ...  | scheduler_grp  |  |  |
    |  |  |    [0]         |  |    [1]         |       |    [7]         |  |  |
    |  |  +----------------+  +----------------+       +----------------+  |  |
    |  |         |                   |                        |           |  |
    |  |         v                   v                        v           |  |
    |  |  +----------------------------------------------------------+    |  |
    |  |  |           Shared Descriptor AXI Master                   |    |  |
    |  |  |              (Round-Robin Arbitration)                   |    |  |
    |  |  +----------------------------------------------------------+    |  |
    |  +-------------------------------------------------------------------+  |
    |         |                                              |                |
    |         v                                              v                |
    |  +--------------------+                    +------------------------+   |
    |  |   SINK DATA PATH   |                    |   SOURCE DATA PATH     |   |
    |  | (Network -> Memory)|                    | (Memory -> Network)    |   |
    |  |                    |                    |                        |   |
    |  |  Fill Interface    |                    |  Drain Interface       |   |
    |  |       |            |                    |        ^               |   |
    |  |       v            |                    |        |               |   |
    |  |  snk_sram_ctrl     |                    |  src_sram_ctrl         |   |
    |  |       |            |                    |        ^               |   |
    |  |       v            |                    |        |               |   |
    |  |  axi_write_engine  |                    |  axi_read_engine       |   |
    |  |       |            |                    |        ^               |   |
    |  +-------|------------+                    +--------|---------------+   |
    |          v                                          |                   |
    +----------|------------------------------------------|-----------------+
               |                                          |
               v                                          |
        AXI Write Master                           AXI Read Master
        (to System Memory)                         (from System Memory)

Source: 01_architecture_block.mmd

## 3.2	Data Flow Overview

### 3.2.1	Sink Path (Network to Memory)

The sink path receives data from an external source (via Fill interface) and writes it to system memory:

### 3.2.2	Figure 1.1.2: Sink Path Data Flow

1. External Fill Valid
        |
        v
2. beats_alloc_ctrl (Space Allocation)
        |
        v
3. snk_sram_controller_unit (SRAM Write)
        |
        v
4. beats_drain_ctrl (Data Available)
        |
        v
5. axi_write_engine (AXI Burst Write)
        |
        v
6. System Memory

Source: 01_sink_path_flow.mmd

### 3.2.3	Source Path (Memory to Network)

The source path reads data from system memory and sends it to an external destination (via Drain interface):

### 3.2.4	Figure 1.1.3: Source Path Data Flow

1. Scheduler Request
        |
        v
2. beats_alloc_ctrl (Space Allocation)
        |
        v
3. axi_read_engine (AXI Burst Read)
        |
        v
4. src_sram_controller_unit (SRAM Write)
        |
        v
5. beats_drain_ctrl (Data Available)
        |
        v
6. External Drain Ready

Source: 01_source_path_flow.mmd

## 3.3	Key Architectural Decisions

### 3.3.1	Beat-Level Tracking

All flow control uses beat granularity:

Table 1.1.1: Beat Granularity Examples

| Operation | Granularity | Example (512-bit DW) |
| --- | --- | --- |
| Space allocation | Beats | Request 8 beats = 512 bytes |
| Data availability | Beats | 16 beats ready = 1024 bytes |
| AXI burst length | Beats | ARLEN=15 = 16 beats |
| Latency compensation | Beats | 2-beat pipeline delay |

### 3.3.2	Concurrent Read/Write

To prevent deadlock with large transfers:

Example: 100MB transfer with 2KB SRAM buffer

Sequential operation (WRONG):
1. Read 100MB -> DEADLOCK at 2KB (SRAM full, can't complete read)

Concurrent operation (CORRECT):
1. Read starts filling SRAM -> SRAM becomes full (2KB)
2. Read pauses (natural backpressure)
3. Write drains SRAM -> SRAM has free space
4. Read resumes -> Both continue until 100MB complete

### 3.3.3	Virtual FIFOs (Alloc/Drain)

The alloc_ctrl and drain_ctrl modules are “virtual FIFOs” that track space/data without storing actual data:

- beats_alloc_ctrl: Tracks allocated space (write pointer advances on allocation, read pointer on actual write)

- beats_drain_ctrl: Tracks available data (write pointer advances on data arrival, read pointer on drain)

## 3.4	Module Hierarchy

rapids_core_beats
├── beats_scheduler_group_array
│   ├── beats_scheduler_group [0..7]
│   │   ├── descriptor_engine
│   │   └── scheduler
│   └── Shared Descriptor AXI Master
│
├── sink_data_path
│   ├── snk_sram_controller
│   │   └── snk_sram_controller_unit [0..7]
│   │       ├── beats_alloc_ctrl
│   │       ├── simple_sram
│   │       └── beats_drain_ctrl
│   └── axi_write_engine
│       └── beats_latency_bridge
│
└── source_data_path
    ├── axi_read_engine
    │   └── beats_latency_bridge
    └── src_sram_controller
        └── src_sram_controller_unit [0..7]
            ├── beats_alloc_ctrl
            ├── simple_sram
            └── beats_drain_ctrl

## 3.5	Timing Diagram: Basic Transfer

### 3.5.1	Figure 1.1.4: Basic Sink Path Transfer Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    fill_valid     _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____
                    :       :       :       :       :       :
    fill_ready     ‾\_______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_______/‾‾‾‾‾
                    :  [1]  :       :       :       :  [2]  :
    fill_data      X|==D0===|==D1===|==D2===|==D3===|=====XX|XXXXX
                    :       :       :       :       :       :
    sram_wr_en     _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\____________
                    :       :       :       :       :       :
    data_avail     ___/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\____
                    :       :       :       :       :       :

    [1] = fill_ready deasserted (SRAM full or allocation pending)
    [2] = fill_ready deasserted (end of burst)

TODO: Replace with simulation-generated waveform showing actual signals

## 3.6	Related Documentation

- Top-Level Port List - Complete port specification

- Clocks and Reset - Timing requirements

- Beats Scheduler Group - Scheduler integration

- Sink Data Path - Sink path details

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Top-Level Port List

Module: rapids_core_beats.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 4.1	Parameters

### 4.1.1	Primary Parameters

parameter int NUM_CHANNELS = 8;               // Number of DMA channels
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS);
parameter int ADDR_WIDTH = 64;                // Address bus width
parameter int DATA_WIDTH = 512;               // Data bus width
parameter int AXI_ID_WIDTH = 8;               // AXI ID width
parameter int SRAM_DEPTH = 512;               // SRAM depth per channel
parameter int SEG_COUNT_WIDTH = $clog2(SRAM_DEPTH) + 1;
parameter int PIPELINE = 0;                   // Pipeline stages
parameter int AR_MAX_OUTSTANDING = 8;         // Max outstanding AR transactions
parameter int AW_MAX_OUTSTANDING = 8;         // Max outstanding AW transactions
parameter int W_PHASE_FIFO_DEPTH = 64;        // Write data FIFO depth
parameter int B_PHASE_FIFO_DEPTH = 16;        // Write response FIFO depth

: Table 1.2.1: Primary Parameters

### 4.1.2	Monitor Bus Parameters

parameter int DESC_MON_BASE_AGENT_ID = 16;    // 0x10 - Descriptor Engines (16-23)
parameter int SCHED_MON_BASE_AGENT_ID = 48;   // 0x30 - Schedulers (48-55)
parameter int DESC_AXI_MON_AGENT_ID = 8;      // 0x08 - Descriptor AXI Master Monitor
parameter int MON_UNIT_ID = 1;                // 0x1
parameter int MON_MAX_TRANSACTIONS = 16;

: Table 1.2.2: Monitor Bus Parameters

## 4.2	Clock and Reset

Table 1.2.3: Clock and Reset Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock (100-500 MHz) |
| rst_n | input | 1 | Active-low asynchronous reset |

## 4.3	APB Programming Interface

Table 1.2.4: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | NC | Per-channel kick-off valid |
| apb_ready | output | NC | Per-channel ready |
| apb_addr | input | NC x AW | Per-channel descriptor start address |

## 4.4	Configuration Interface

### 4.4.1	Per-Channel Configuration

Table 1.2.5: Per-Channel Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | NC | Enable each channel |
| cfg_channel_reset | input | NC | Per-channel soft reset |

### 4.4.2	Scheduler Configuration

Table 1.2.6: Scheduler Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_enable | input | 1 | Global scheduler enable |
| cfg_sched_timeout_cycles | input | 16 | Timeout threshold in cycles |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_sched_err_enable | input | 1 | Enable error reporting |
| cfg_sched_compl_enable | input | 1 | Enable completion reporting |
| cfg_sched_perf_enable | input | 1 | Enable performance monitoring |

### 4.4.3	Descriptor Engine Configuration

Table 1.2.7: Descriptor Engine Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desceng_enable | input | 1 | Global descriptor engine enable |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetching |
| cfg_desceng_fifo_thresh | input | 4 | Prefetch FIFO threshold |
| cfg_desceng_addr0_base | input | AW | Address range 0 base |
| cfg_desceng_addr0_limit | input | AW | Address range 0 limit |
| cfg_desceng_addr1_base | input | AW | Address range 1 base |
| cfg_desceng_addr1_limit | input | AW | Address range 1 limit |

### 4.4.4	AXI Transfer Configuration

Table 1.2.8: AXI Transfer Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | AXI read burst length (beats) |
| cfg_axi_wr_xfer_beats | input | 8 | AXI write burst length (beats) |

## 4.5	Status Interface

Table 1.2.9: Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| system_idle | output | 1 | All components idle |
| descriptor_engine_idle | output | NC | Per-channel descriptor engine idle |
| scheduler_idle | output | NC | Per-channel scheduler idle |
| scheduler_state | output | NC x 7 | Per-channel FSM state (one-hot) |
| sched_error | output | NC | Per-channel error flag |

## 4.6	Sink Path Interface (Network to Memory)

### 4.6.1	Fill Interface (External to SRAM)

Table 1.2.10: Sink Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_fill_alloc_req | input | 1 | Allocation request |
| snk_fill_alloc_size | input | 8 | Size in beats to allocate |
| snk_fill_alloc_id | input | CIW | Channel ID for allocation |
| snk_fill_space_free | output | NC x SCW | Available space per channel |
| snk_fill_valid | input | 1 | Fill data valid |
| snk_fill_ready | output | 1 | Ready to accept fill data |
| snk_fill_data | input | DW | Fill data |
| snk_fill_last | input | 1 | Last beat of fill burst |
| snk_fill_id | input | CIW | Fill channel ID |

### 4.6.2	Sink AXI Write Master

Table 1.2.11: Sink AXI Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_snk_axi_awid | output | IW | Write address ID |
| m_snk_axi_awaddr | output | AW | Write address |
| m_snk_axi_awlen | output | 8 | Burst length (beats - 1) |
| m_snk_axi_awsize | output | 3 | Burst size |
| m_snk_axi_awburst | output | 2 | Burst type |
| m_snk_axi_awvalid | output | 1 | Write address valid |
| m_snk_axi_awready | input | 1 | Write address ready |
| m_snk_axi_wdata | output | DW | Write data |
| m_snk_axi_wstrb | output | DW/8 | Write strobes |
| m_snk_axi_wlast | output | 1 | Last write beat |
| m_snk_axi_wvalid | output | 1 | Write data valid |
| m_snk_axi_wready | input | 1 | Write data ready |
| m_snk_axi_bid | input | IW | Response ID |
| m_snk_axi_bresp | input | 2 | Write response |
| m_snk_axi_bvalid | input | 1 | Response valid |
| m_snk_axi_bready | output | 1 | Response ready |

## 4.7	Source Path Interface (Memory to Network)

### 4.7.1	Source AXI Read Master

Table 1.2.12: Source AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_src_axi_arid | output | IW | Read address ID |
| m_src_axi_araddr | output | AW | Read address |
| m_src_axi_arlen | output | 8 | Burst length (beats - 1) |
| m_src_axi_arsize | output | 3 | Burst size |
| m_src_axi_arburst | output | 2 | Burst type |
| m_src_axi_arvalid | output | 1 | Read address valid |
| m_src_axi_arready | input | 1 | Read address ready |
| m_src_axi_rid | input | IW | Read data ID |
| m_src_axi_rdata | input | DW | Read data |
| m_src_axi_rresp | input | 2 | Read response |
| m_src_axi_rlast | input | 1 | Last read beat |
| m_src_axi_rvalid | input | 1 | Read data valid |
| m_src_axi_rready | output | 1 | Read data ready |

### 4.7.2	Drain Interface (SRAM to External)

Table 1.2.13: Source Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_drain_valid | output | 1 | Drain data valid |
| src_drain_ready | input | 1 | External ready for drain |
| src_drain_data | output | DW | Drain data |
| src_drain_last | output | 1 | Last beat of drain burst |
| src_drain_id | output | CIW | Drain channel ID |

## 4.8	Descriptor AXI Master

Table 1.2.14: Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_desc_axi_arid | output | IW | Read address ID |
| m_desc_axi_araddr | output | AW | Read address |
| m_desc_axi_arlen | output | 8 | Burst length |
| m_desc_axi_arsize | output | 3 | Burst size |
| m_desc_axi_arburst | output | 2 | Burst type |
| m_desc_axi_arvalid | output | 1 | Read address valid |
| m_desc_axi_arready | input | 1 | Read address ready |
| m_desc_axi_rid | input | IW | Read data ID |
| m_desc_axi_rdata | input | 256 | Descriptor data (256-bit) |
| m_desc_axi_rresp | input | 2 | Read response |
| m_desc_axi_rlast | input | 1 | Last read beat |
| m_desc_axi_rvalid | input | 1 | Read data valid |
| m_desc_axi_rready | output | 1 | Read data ready |

## 4.9	MonBus Interface

Table 1.2.15: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_valid | output | 1 | Monitor packet valid |
| monbus_ready | input | 1 | Monitor consumer ready |
| monbus_data | output | 64 | Monitor packet data |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Clocks and Reset

Module: rapids_core_beats.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 5.1	Clock Domain

RAPIDS Beats operates in a single clock domain for simplified design and maximum throughput.

### 5.1.1	Clock Specifications

Table 1.3.1: Clock Specifications

| Parameter | Specification |
| --- | --- |
| Clock Signal | clk (also axi_aclk in some submodules) |
| Frequency Range | 100 - 500 MHz |
| Duty Cycle | 45% - 55% |
| Clock Jitter | < 100 ps peak-to-peak |

### 5.1.2	Clock Distribution

clk (system clock)
                        |
    +-------------------+-------------------+
    |                   |                   |
    v                   v                   v
beats_scheduler    sink_data_path    source_data_path
_group_array            |                   |
    |              +----+----+         +----+----+
    |              |         |         |         |
    v              v         v         v         v
descriptor    snk_sram   axi_write  axi_read  src_sram
_engine       _ctrl      _engine    _engine   _ctrl

## 5.2	Reset Specification

RAPIDS uses asynchronous assert, synchronous deassert reset methodology.

### 5.2.1	Reset Signal

Table 1.3.2: Reset Specifications

| Parameter | Specification |
| --- | --- |
| Reset Signal | rst_n (also axi_aresetn in some submodules) |
| Polarity | Active-low |
| Assert | Asynchronous (immediate) |
| Deassert | Synchronous (on rising clock edge) |
| Minimum Duration | 10 clock cycles |

### 5.2.2	Reset Timing Diagram

### 5.2.3	Figure 1.3.1: Reset Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    rst_n    ‾‾‾‾‾‾‾\_______________________________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
                    :  [1]  :       :       :  [2]  :       :
    internal_regs   X|======RESET VALUES============|=NORMAL OP====
                    :       :       :       :       :       :
    fsm_state       X|======IDLE====================|=ACTIVE=======

    [1] = Reset asserted (asynchronous)
    [2] = Reset deasserted (synchronous to clock rising edge)

TODO: Replace with simulation-generated waveform

## 5.3	Reset Behavior

### 5.3.1	On Reset Assert

All modules initialize to known states:

Table 1.3.3: Reset States

| Component | Reset State |
| --- | --- |
| Scheduler FSM | IDLE |
| Descriptor Engine | IDLE, FIFOs empty |
| AXI Read Engine | Idle, no outstanding transactions |
| AXI Write Engine | Idle, no outstanding transactions |
| SRAM Pointers | All zeros |
| Alloc/Drain Controllers | Empty, full space available |
| MonBus | No packets pending |

### 5.3.2	Reset Sequence

1. External reset asserted (rst_n = 0)
   - All flip-flops async reset
   - All outputs driven to safe states
   - No AXI transactions

2. System stabilization (minimum 10 cycles)
   - Clock running and stable
   - Configuration registers programmed (optional)

3. Reset deasserted (rst_n = 1, sync to clk rising edge)
   - FSMs begin operation
   - Ready to accept descriptors/data

## 5.4	Configuration During Reset

Some configuration signals MUST be stable before reset deassertion:

Table 1.3.4: Configuration Timing Requirements

| Signal | Requirement |
| --- | --- |
| cfg_channel_enable | Stable before rst_n=1 |
| cfg_sched_timeout_cycles | Stable before rst_n=1 |
| cfg_axi_rd_xfer_beats | Stable before rst_n=1 |
| cfg_axi_wr_xfer_beats | Stable before rst_n=1 |

## 5.5	Reset Macros

RAPIDS uses standardized reset macros from reset_defs.svh:

`include "reset_defs.svh"

// Standard reset pattern
`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
        r_counter <= '0;
    end else begin
        r_state <= w_next_state;
        r_counter <= w_next_counter;
    end
)

## 5.6	Power-On Reset Considerations

- External reset should be held for minimum 10 clock cycles after clock is stable

- Configuration registers should be programmed before or during reset

- No AXI transactions until reset is deasserted and channels are enabled

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Scheduler Specification

Module: scheduler.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 6.1	Overview

The Scheduler coordinates descriptor-based data transfers for a single RAPIDS channel. It receives descriptors from the descriptor engine, parses transfer parameters, and coordinates read/write operations through the data paths.

### 6.1.1	Key Features

- Descriptor Processing: Parses 256-bit descriptors for transfer parameters

- Beat-Based Tracking: All lengths tracked in beats (data-width units)

- Concurrent Read/Write: Simultaneous read and write to prevent deadlock

- Timeout Detection: Configurable watchdog for stalled transfers

- MonBus Integration: State transition and error event reporting

- Error Aggregation: Combines errors from descriptor engine and data paths

### 6.1.2	Block Diagram

### 6.1.3	Figure 2.1.1: Scheduler Block Diagram

+---------------------------+
    descriptor_valid -->|                           |--> sched_rd_valid
    descriptor_ready <--|                           |--> sched_rd_addr
    descriptor_packet-->|                           |--> sched_rd_beats
                        |        SCHEDULER          |
    cfg_channel_enable->|                           |<-- sched_rd_done_strobe
    cfg_timeout_cycles->|                           |<-- sched_rd_beats_done
    cfg_timeout_enable->|                           |<-- sched_rd_error
                        |                           |
                        |                           |--> sched_wr_valid
                        |                           |--> sched_wr_addr
                        |                           |--> sched_wr_beats
                        |                           |
                        |                           |<-- sched_wr_done_strobe
                        |                           |<-- sched_wr_beats_done
    scheduler_idle   <--|                           |<-- sched_wr_error
    scheduler_state  <--|                           |
    sched_error      <--|                           |--> monbus_pkt_valid
                        +---------------------------+--> monbus_pkt_data

Source: 02_scheduler_block.mmd

## 6.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels in system
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Channel ID width
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width (beats)

// Monitor Bus Parameters
parameter logic [7:0] MON_AGENT_ID = 8'h30;      // RAPIDS Scheduler Agent ID
parameter logic [3:0] MON_UNIT_ID = 4'h1;        // Unit identifier
parameter logic [5:0] MON_CHANNEL_ID = 6'h0;     // Base channel ID

// Descriptor Width
parameter int DESC_WIDTH = 256;                  // RAPIDS descriptor width

: Table 2.1.1: Scheduler Parameters

## 6.3	Port List

### 6.3.1	Clock and Reset

Table 2.1.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 6.3.2	Configuration Interface

Table 2.1.3: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable this channel |
| cfg_channel_reset | input | 1 | Channel soft reset |
| cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window (cycles) |
| cfg_sched_timeout_limit | input | 8 | Consecutive-timeout windows before fatal escalation (0 = never) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |

### 6.3.3	Descriptor Engine Interface

Table 2.1.4: Descriptor Engine Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | input | 1 | Descriptor valid |
| descriptor_ready | output | 1 | Ready to accept descriptor |
| descriptor_packet | input | 256 | 256-bit descriptor |
| descriptor_error | input | 1 | Error from descriptor engine |

### 6.3.4	Data Read Interface

Table 2.1.5: Data Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Read request valid |
| sched_rd_addr | output | AW | Source address |
| sched_rd_beats | output | 32 | Beats to read |
| sched_rd_done_strobe | input | 1 | Read complete strobe |
| sched_rd_beats_done | input | 32 | Beats completed |
| sched_rd_error | input | 1 | Read error |

### 6.3.5	Data Write Interface

Table 2.1.6: Data Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | output | 1 | Write request valid (gated on remaining ISSUE beats) |
| sched_wr_addr | output | AW | Destination address |
| sched_wr_beats | output | 32 | Beats to write |
| sched_wr_done_strobe | input | 1 | Write AW-issue strobe (burst accepted on AW) |
| sched_wr_beats_done | input | 32 | Beats issued this strobe |
| sched_wr_commit_strobe | input | 1 | Write COMMIT strobe (B response received) |
| sched_wr_commit_beats | input | 32 | Beats committed this strobe |
| sched_wr_error | input | 1 | Write error |

Completion is gated on write-data COMMIT, not AW issue. The scheduler maintains two independent beat counters for the write side:

- r_write_beats_remaining decrements on sched_wr_done_strobe (AW handshake) and drives the destination address and sched_wr_valid (writes are requested only while ISSUE beats remain).

- r_write_beats_to_commit decrements on sched_wr_commit_strobe (B response). A transfer is considered write-complete only when this counter reaches zero (w_write_complete), so a channel is never reported done until every issued burst has been acknowledged on the B channel.

## 6.4	FSM States

### 6.4.1	Figure 2.1.2: Scheduler FSM State Diagram

+-------+
        rst_n=0 --> | IDLE  |<-----------------+
                    +---+---+                  |
                        |                      |
             descriptor_valid                  |
                        |                      |
                        v                      |
                    +-------+                  |
                    | PARSE |                  |
                    +---+---+                  |
                        |                      |
                  parse complete               |
                        |                      |
                        v                      |
                +---------------+              |
                | CH_XFER_DATA  |              |
                | (concurrent   |              |
                |  rd + wr)     |              |
                +-------+-------+              |
                        |                      |
            rd_beats=0 && wr_beats=0           |
                        |                      |
                        v                      |
                    +-------+                  |
                    | DONE  |------------------+
                    +-------+

Source: 02_scheduler_fsm.mmd

### 6.4.2	State Encoding

Table 2.1.7: FSM State Encoding

| State | Encoding | Description |
| --- | --- | --- |
| IDLE | 7’b0000001 | Waiting for descriptor |
| PARSE | 7’b0000010 | Parsing descriptor fields |
| CH_XFER_DATA | 7’b0000100 | Concurrent read/write transfer |
| DONE | 7’b0001000 | Transfer complete, MonBus report |
| ERROR | 7’b0010000 | Error handling |
| TIMEOUT | 7’b0100000 | Timeout occurred |
| SOFT_RESET | 7’b1000000 | Soft reset handling |

## 6.5	Operation

### 6.5.1	Descriptor Format

Bits [63:0]    - src_addr: Source memory address
Bits [127:64]  - dst_addr: Destination memory address
Bits [159:128] - length: Transfer length in beats
Bits [191:160] - next_ptr: Next descriptor pointer (0 = last)
Bits [195:192] - channel_id: Channel identifier
Bits [196]     - last: Last descriptor flag
Bits [197]     - gen_irq: Generate interrupt
Bits [255:198] - reserved

### 6.5.2	Transfer Sequence

### 6.5.3	Figure 2.1.3: Basic Transfer Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    state          |  IDLE  | PARSE |    CH_XFER_DATA      | DONE |
                    :       :       :       :       :       :
    desc_valid     ‾‾\______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
                    :       :       :       :       :       :
    sched_rd_valid _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____________
                    :       :       :       :       :       :
    sched_wr_valid _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____________
                    :       :       :       :       :       :
    rd_done_strobe ___________________________/‾\_________________
                    :       :       :       :       :       :
    wr_done_strobe _____________________________/‾\_______________

TODO: Replace with simulation-generated waveform

## 6.6	Error Handling

Table 2.1.8: Error Handling

| Error Source | Detection | Response |
| --- | --- | --- |
| Descriptor engine | descriptor_error | Transition to CH_ERROR state |
| Read engine | sched_rd_error | Set sticky error flag |
| Write engine | sched_wr_error | Set sticky error flag |
| Write-progress timeout | Recoverable, escalates after limit | See below |

### 6.6.1	Recoverable Write-Progress Timeout

The write-progress watchdog is recoverable and only escalates to a fatal fault after a configurable number of consecutive stalled windows:

- Window counting: r_timeout_counter increments while sched_wr_valid && !sched_wr_ready (a write request is stalled). The window expires when it reaches cfg_sched_timeout_cycles.

- Re-arm: On expiry the counter re-arms (resets to 0) and the scheduler keeps waiting; a single expired window is not fatal.

- Strike counting: r_timeout_strikes (8-bit) increments once per expired window. Any write progress – sched_wr_done_strobe (AW issue) or sched_wr_commit_strobe (B response) – clears both the window counter and the strike counter.

- Escalation: When cfg_sched_timeout_limit != 0 and r_timeout_strikes >= cfg_sched_timeout_limit, the channel escalates to a sticky CH_ERROR (surfaced on sched_error). Total time to escalate is approximately cfg_sched_timeout_limit * cfg_sched_timeout_cycles.

- Never-escalate mode: cfg_sched_timeout_limit == 0 is a pure soft timeout – the event may be reported, but the channel never faults.

scheduler_idle excludes CH_ERROR. Only CH_IDLE (with no active channel reset) is reported idle; a faulted channel in CH_ERROR is deliberately NOT reported idle so a wedged channel cannot read back as idle. The fault is surfaced via sched_error instead.

## 6.7	MonBus Events

Table 2.1.9: MonBus Event Codes

| Event Code | Description |
| --- | --- |
| 0x01 | Descriptor received |
| 0x02 | Transfer started |
| 0x03 | Transfer complete |
| 0x04 | Error occurred |
| 0x05 | Timeout |

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Descriptor Engine Specification

Module: descriptor_engine.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 7.1	Overview

The Descriptor Engine fetches and manages 256-bit descriptors from memory via AXI4. It maintains a prefetch FIFO to hide memory latency and provides parsed descriptors to the scheduler.

### 7.1.1	Key Features

- AXI4 Descriptor Fetch: Single-beat 256-bit reads

- Prefetch FIFO: Configurable depth for latency hiding

- Address Range Checking: Validates descriptor addresses

- Error Detection: Address violations, AXI errors

- MonBus Integration: Fetch events and error reporting

### 7.1.2	Block Diagram

### 7.1.3	Figure 2.2.1: Descriptor Engine Block Diagram

+---------------------------+
    apb_valid        -->|                           |
    apb_ready        <--|                           |--> m_axi_araddr
    apb_addr         -->|                           |--> m_axi_arvalid
                        |                           |<-- m_axi_arready
    cfg_enable       -->|   DESCRIPTOR ENGINE       |
    cfg_prefetch     -->|                           |<-- m_axi_rdata
    cfg_fifo_thresh  -->|                           |<-- m_axi_rvalid
    cfg_addr0_base   -->|                           |--> m_axi_rready
    cfg_addr0_limit  -->|                           |<-- m_axi_rresp
                        |                           |
    descriptor_valid <--|                           |
    descriptor_ready -->|                           |--> monbus_pkt_valid
    descriptor_packet<--|                           |--> monbus_pkt_data
    descriptor_error <--|                           |
                        +---------------------------+

Source: 02_descriptor_engine_block.mmd

## 7.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int FIFO_DEPTH = 4;                    // Prefetch FIFO depth

// Monitor Bus Parameters
parameter logic [7:0] MON_AGENT_ID = 8'h10;      // Descriptor Engine Agent ID
parameter logic [3:0] MON_UNIT_ID = 4'h1;        // Unit identifier

: Table 2.2.1: Descriptor Engine Parameters

## 7.3	Port List

### 7.3.1	Clock and Reset

Table 2.2.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 7.3.2	APB Programming Interface

Table 2.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Kick-off valid (start descriptor chain) |
| apb_ready | output | 1 | Ready to accept kick-off |
| apb_addr | input | AW | First descriptor address |

### 7.3.3	Configuration Interface

Table 2.2.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_prefetch_enable | input | 1 | Enable prefetch chaining (0 = on-demand, ~1 ahead) |
| cfg_fifo_threshold | input | 4 | Prefetch depth (descriptors buffered ahead when enabled) |
| cfg_addr0_base | input | AW | Valid address range 0 base |
| cfg_addr0_limit | input | AW | Valid address range 0 limit |
| cfg_addr1_base | input | AW | Valid address range 1 base |
| cfg_addr1_limit | input | AW | Valid address range 1 limit |

### 7.3.4	AXI4 Read Master Interface

Table 2.2.5: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_araddr | output | AW | Read address |
| m_axi_arvalid | output | 1 | Address valid |
| m_axi_arready | input | 1 | Address ready |
| m_axi_rdata | input | 256 | Read data (descriptor) |
| m_axi_rvalid | input | 1 | Data valid |
| m_axi_rready | output | 1 | Data ready |
| m_axi_rresp | input | 2 | Read response |

### 7.3.5	Scheduler Interface

Table 2.2.6: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | output | 1 | Descriptor valid |
| descriptor_ready | input | 1 | Scheduler ready |
| descriptor_packet | output | 256 | Parsed descriptor |
| descriptor_error | output | 1 | Error flag |

## 7.4	FSM States

### 7.4.1	Figure 2.2.2: Descriptor Engine FSM

+-------+
        rst_n=0 --> | IDLE  |<-----------------+
                    +---+---+                  |
                        |                      |
                  apb_valid                    |
                        |                      |
                        v                      |
                +---------------+              |
                | CHECK_RANGE   |              |
                +-------+-------+              |
                        |                      |
               range OK | range error          |
                        |         |            |
                        v         v            |
                +-------+   +-------+          |
                | FETCH |   | ERROR |----------+
                +---+---+   +-------+          |
                    |                          |
            axi_rvalid                         |
                    |                          |
                    v                          |
                +-------+                      |
                | PUSH  |                      |
                +---+---+                      |
                    |                          |
            last descriptor?                   |
                    |                          |
         no         |         yes              |
    +---------------+---------------+          |
    |                               |          |
    v                               v          |
+-------+                       +-------+      |
|PREFETCH|                      | DONE  |------+
+---+---+                       +-------+
    |
    +---> back to FETCH

Source: 02_descriptor_engine_fsm.mmd

## 7.5	Operation

### 7.5.1	Descriptor Chain Processing

### 7.5.2	Figure 2.2.3: Descriptor Chain Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    apb_valid      _/‾\_____:_______:_______:_______:_______:_______
                    :       :       :       :       :       :
    m_axi_arvalid  _________/‾‾‾‾‾‾‾\_______/‾‾‾‾‾‾‾\_______
                    :       :       :       :       :       :
    m_axi_araddr   X|=DESC0=|XXXXXXX|=DESC1=|XXXXXXX|XXXXXXX
                    :       :       :       :       :       :
    m_axi_rvalid   _________________/‾\_____________/‾\_____
                    :       :       :       :       :       :
    desc_valid     _____________________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾

TODO: Replace with simulation-generated waveform

## 7.6	Address Range Checking

Descriptors must fall within configured valid address ranges:

Valid if: (addr >= cfg_addr0_base && addr <= cfg_addr0_limit)
       || (addr >= cfg_addr1_base && addr <= cfg_addr1_limit)

Invalid addresses generate an error and halt the descriptor chain.

## 7.7	Prefetch and Autonomous Chaining

cfg_prefetch_enable and cfg_fifo_threshold are functional prefetch controls (previously declared-but-unused, which left chaining permanently greedy). They throttle how far ahead the engine fetches chained descriptors by comparing the output descriptor FIFO occupancy (w_desc_fifo_count) against a computed w_prefetch_limit:

if      (!cfg_prefetch_enable)      w_prefetch_limit = 1;   // on-demand
else if (cfg_fifo_threshold == 0)   w_prefetch_limit = 1;   // guard (0 would stall)
else                                w_prefetch_limit = cfg_fifo_threshold;

w_prefetch_allows = (w_desc_fifo_count < w_prefetch_limit);

- Prefetch OFF (cfg_prefetch_enable = 0): limit forced to 1, so a chained fetch is only allowed when the output FIFO has drained – the engine stays roughly one descriptor ahead of the scheduler (on-demand).

- Prefetch ON: the engine buffers up to cfg_fifo_threshold descriptors ahead.

### 7.7.1	Chaining Decision

A chain fetch issues immediately when all of the following hold:

w_should_chain = w_chain_eligible     // valid non-last descriptor, in range, no error
              && w_desc_committed      // current descriptor accepted into output FIFO
              && w_prefetch_allows     // occupancy below prefetch limit
              && w_desc_addr_fifo_wr_ready;  // address FIFO has a slot

### 7.7.2	Deferred (Throttle-Blocked) Chain Fetch

If a descriptor commits and is chain-eligible but the fetch cannot issue this cycle (throttled by w_prefetch_allows, or the address FIFO is full), the owed fetch is recorded rather than dropped:

- r_chain_pending is set and the next-descriptor pointer is held in r_pending_chain_addr.

- The deferred fetch is reissued (w_pending_push_fire) once the output FIFO drains enough that w_prefetch_allows is true again and the address FIFO has a slot, clearing r_chain_pending.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	AXI Read Engine Specification

Module: axi_read_engine.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 8.1	Overview

The AXI Read Engine performs burst reads from system memory and writes data to the SRAM buffer. It operates as a streaming pipeline with no FSM, maximizing throughput through continuous data flow.

### 8.1.1	Key Features

- Streaming Pipeline: No FSM - pure streaming architecture

- Multi-Channel Support: Channel ID passed through AXI ID field

- Configurable Burst Length: Up to 256 beats per burst

- Latency Compensation: Integrated beats_latency_bridge

- Error Handling: AXI RRESP error detection and reporting

### 8.1.2	Block Diagram

### 8.1.3	Figure 2.3.1: AXI Read Engine Block Diagram

+---------------------------+
    sched_rd_valid   -->|                           |--> m_axi_arid
    sched_rd_addr    -->|                           |--> m_axi_araddr
    sched_rd_beats   -->|    AXI READ ENGINE        |--> m_axi_arlen
    sched_rd_id      -->|                           |--> m_axi_arvalid
                        |    (Streaming Pipeline)   |<-- m_axi_arready
    cfg_xfer_beats   -->|                           |
                        |                           |<-- m_axi_rid
    sched_rd_done    <--|                           |<-- m_axi_rdata
    sched_rd_beats_d <--|                           |<-- m_axi_rresp
    sched_rd_error   <--|                           |<-- m_axi_rlast
                        |                           |<-- m_axi_rvalid
    sram_wr_en       <--|                           |--> m_axi_rready
    sram_wr_addr     <--|                           |
    sram_wr_data     <--|                           |
    sram_wr_id       <--|                           |
                        +---------------------------+

Source: 02_axi_read_engine_block.mmd

## 8.2	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width
parameter int AXI_ID_WIDTH = 8;                  // AXI ID width
parameter int MAX_OUTSTANDING = 8;               // Max outstanding AR transactions
parameter int PIPELINE = 0;                      // Pipeline stages

// Derived
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS);

: Table 2.3.1: AXI Read Engine Parameters

## 8.3	Port List

### 8.3.1	Clock and Reset

Table 2.3.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 8.3.2	Scheduler Interface

Table 2.3.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | 1 | Read request valid |
| sched_rd_addr | input | AW | Source address |
| sched_rd_beats | input | 32 | Total beats to read |
| sched_rd_id | input | CW | Channel ID |
| sched_rd_done_strobe | output | 1 | Burst complete strobe |
| sched_rd_beats_done | output | 32 | Beats completed |
| sched_rd_error | output | 1 | Error flag |

### 8.3.3	AXI4 Read Master Interface

Table 2.3.4: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arid | output | IW | Read address ID |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length (beats - 1) |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type (INCR) |
| m_axi_arvalid | output | 1 | Address valid |
| m_axi_arready | input | 1 | Address ready |
| m_axi_rid | input | IW | Read data ID |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rlast | input | 1 | Last beat |
| m_axi_rvalid | input | 1 | Data valid |
| m_axi_rready | output | 1 | Data ready |

### 8.3.4	SRAM Write Interface

Table 2.3.5: SRAM Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sram_wr_en | output | 1 | SRAM write enable |
| sram_wr_addr | output | AW | SRAM write address |
| sram_wr_data | output | DW | SRAM write data |
| sram_wr_id | output | CW | Channel ID for data |

## 8.4	Operation

### 8.4.1	Streaming Pipeline Architecture

The AXI read engine uses a streaming pipeline with no FSM:

Scheduler Request --> AR Channel --> R Channel --> SRAM Write
      |                   |             |              |
      +--- beat count     +-- addr      +-- data       +-- wr_en
           tracking           issue         receive        generate

### 8.4.2	Timing Diagram

### 8.4.3	Figure 2.3.2: AXI Read Burst Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    sched_rd_valid _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____
    sched_rd_beats |=============== 8 ================|XXXXX|XXXXX
                    :       :       :       :       :       :
    m_axi_arvalid  _/‾\_____:_______:_______:_______:_______:_______
    m_axi_arlen    X| 7 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
    m_axi_arready  ‾‾‾‾‾\___/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
                    :       :       :       :       :       :
    m_axi_rvalid   _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____
    m_axi_rdata    X:XXXXXXX|==D0===|==D1===|...|==D7===|XXXXXXX
    m_axi_rlast    _________:_______:_______:_______/‾\_____:_______
                    :       :       :       :       :       :
    sram_wr_en     _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____
    sram_wr_data   X:XXXXXXX|==D0===|==D1===|...|==D7===|XXXXXXX
                    :       :       :       :       :       :
    sched_rd_done  _________:_______:_______:_______:_______/‾\___

TODO: Replace with simulation-generated waveform

## 8.5	Burst Segmentation

Large transfers are segmented into AXI-compliant bursts:

Total beats = 256
cfg_xfer_beats = 64

Bursts issued:
  AR[0]: addr=0x1000, len=63 (64 beats)
  AR[1]: addr=0x2000, len=63 (64 beats)
  AR[2]: addr=0x3000, len=63 (64 beats)
  AR[3]: addr=0x4000, len=63 (64 beats)

## 8.6	Error Handling

Table 2.3.6: Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | m_axi_rresp == 2'b10 | Set sched_rd_error, continue |
| AXI DECERR | m_axi_rresp == 2'b11 | Set sched_rd_error, continue |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	AXI Write Engine Specification

Module: axi_write_engine.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 9.1	Overview

The AXI Write Engine reads data from SRAM and performs burst writes to system memory. It operates as a streaming pipeline and includes write response handling for completion tracking.

### 9.1.1	Key Features

- Streaming Pipeline: No FSM - pure streaming architecture

- Multi-Channel Support: Channel ID in AXI ID field

- Write Response Tracking: B channel completion handling

- Configurable Burst Length: Up to 256 beats per burst

- Latency Compensation: Integrated beats_latency_bridge

### 9.1.2	Block Diagram

### 9.1.3	Figure 2.4.1: AXI Write Engine Block Diagram

+---------------------------+
    sched_wr_valid   -->|                           |--> m_axi_awid
    sched_wr_addr    -->|                           |--> m_axi_awaddr
    sched_wr_beats   -->|    AXI WRITE ENGINE       |--> m_axi_awlen
    sched_wr_id      -->|                           |--> m_axi_awvalid
                        |    (Streaming Pipeline)   |<-- m_axi_awready
    cfg_xfer_beats   -->|                           |
                        |                           |--> m_axi_wdata
    sched_wr_done    <--|                           |--> m_axi_wstrb
    sched_wr_beats_d <--|                           |--> m_axi_wlast
    sched_wr_error   <--|                           |--> m_axi_wvalid
                        |                           |<-- m_axi_wready
    sram_rd_en       <--|                           |
    sram_rd_addr     <--|                           |<-- m_axi_bid
    sram_rd_data     -->|                           |<-- m_axi_bresp
    sram_rd_id       <--|                           |<-- m_axi_bvalid
                        |                           |--> m_axi_bready
                        +---------------------------+

Source: 02_axi_write_engine_block.mmd

## 9.2	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width
parameter int AXI_ID_WIDTH = 8;                  // AXI ID width
parameter int MAX_OUTSTANDING = 8;               // Max outstanding AW transactions
parameter int W_FIFO_DEPTH = 64;                 // Write data FIFO depth
parameter int B_FIFO_DEPTH = 16;                 // Write response FIFO depth
parameter int PIPELINE = 0;                      // Pipeline stages

// Derived
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS);

: Table 2.4.1: AXI Write Engine Parameters

## 9.3	Port List

### 9.3.1	Clock and Reset

Table 2.4.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 9.3.2	Scheduler Interface

Table 2.4.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | input | 1 | Write request valid |
| sched_wr_addr | input | AW | Destination address |
| sched_wr_beats | input | 32 | Total beats to write |
| sched_wr_id | input | CW | Channel ID |
| sched_wr_done_strobe | output | NC | AW-issue strobe (pulses on AW handshake) |
| sched_wr_beats_done | output | NC*32 | Beats issued this strobe (awlen + 1) |
| sched_wr_commit_strobe | output | NC | COMMIT strobe (pulses on B response) |
| sched_wr_commit_beats | output | NC*32 | Beats committed this strobe |
| sched_wr_error | output | 1 | Error flag |

Two completion strobes per channel. The engine reports write progress to the scheduler at two points, letting the scheduler gate completion on data actually landing in memory:

- sched_wr_done_strobe / sched_wr_beats_done pulse when the AW command handshakes (m_axi_awvalid && m_axi_awready); the beat count is taken directly from m_axi_awlen + 1. This advances the scheduler’s destination address (ISSUE tracking).

- sched_wr_commit_strobe / sched_wr_commit_beats pulse when the matching B response arrives (m_axi_bvalid && m_axi_bready for the channel); the beat count is recovered from the per-channel B-phase transaction FIFO. This is the COMMIT the scheduler uses to declare the transfer complete.

### 9.3.3	AXI4 Write Master Interface

Table 2.4.4: AXI4 Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awid | output | IW | Write address ID |
| m_axi_awaddr | output | AW | Write address |
| m_axi_awlen | output | 8 | Burst length (beats - 1) |
| m_axi_awsize | output | 3 | Burst size |
| m_axi_awburst | output | 2 | Burst type (INCR) |
| m_axi_awvalid | output | 1 | Address valid |
| m_axi_awready | input | 1 | Address ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat |
| m_axi_wvalid | output | 1 | Data valid |
| m_axi_wready | input | 1 | Data ready |
| m_axi_bid | input | IW | Response ID |
| m_axi_bresp | input | 2 | Write response |
| m_axi_bvalid | input | 1 | Response valid |
| m_axi_bready | output | 1 | Response ready |

### 9.3.4	SRAM Read Interface

Table 2.4.5: SRAM Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sram_rd_en | output | 1 | SRAM read enable |
| sram_rd_addr | output | AW | SRAM read address |
| sram_rd_data | input | DW | SRAM read data |
| sram_rd_id | output | CW | Channel ID for read |

## 9.4	Operation

### 9.4.1	Write Transaction Phases

AXI writes have three phases that can overlap:

Phase 1: AW (Address)    Phase 2: W (Data)    Phase 3: B (Response)
    |                        |                      |
    +-- Issue address        +-- Stream data        +-- Receive response
        before data             from SRAM               track completion

### 9.4.2	Timing Diagram

### 9.4.3	Figure 2.4.2: AXI Write Burst Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    sched_wr_valid _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_____
    sched_wr_beats |=============== 4 ================|XXXXX|XXXXX
                    :       :       :       :       :       :
    m_axi_awvalid  _/‾\_____:_______:_______:_______:_______:_______
    m_axi_awlen    X| 3 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    sram_rd_en     _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_______:_______:_______
                    :       :       :       :       :       :
    m_axi_wvalid   _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_______:_______
    m_axi_wdata    X:XXXXXXX|==D0===|==D1===|==D2===|==D3===|XXXXXXX
    m_axi_wlast    _________:_______:_______:_______/‾\_____:_______
                    :       :       :       :       :       :
    m_axi_bvalid   _________:_______:_______:_______:_______/‾\_____
    m_axi_bresp    X:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX| OK|XXX
                    :       :       :       :       :       :
    sched_wr_done  _________:_______:_______:_______:_______:_/‾\___

TODO: Replace with simulation-generated waveform

## 9.5	Error Handling

Table 2.4.6: Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | m_axi_bresp == 2'b10 | Set sched_wr_error |
| AXI DECERR | m_axi_bresp == 2'b11 | Set sched_wr_error |

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Beats Alloc Control Specification

Module: beats_alloc_ctrl.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 10.1	Overview

The Beats Alloc Control is a “virtual FIFO” that tracks space allocation without storing actual data. It uses FIFO pointer logic to manage pre-allocation for AXI engines, preventing race conditions between allocation and actual data arrival.

### 10.1.1	Key Features

- Virtual FIFO: Pointer tracking without data storage

- Variable-Size Allocation: Supports multi-beat allocation requests

- Single-Beat Release: Data writes release one beat at a time

- Status Flags: Full, almost full, empty, almost empty

- Space Tracking: Reports available space for flow control

### 10.1.2	Block Diagram

### 10.1.3	Figure 2.5.1: Beats Alloc Control Block Diagram

+---------------------------+
    wr_valid -->|                           |--> wr_ready
    wr_size  -->|    BEATS_ALLOC_CTRL       |--> space_free
                |                           |
    rd_valid -->|     (Virtual FIFO)        |--> wr_full
                |                           |--> wr_almost_full
                |  [No data storage]        |--> rd_empty
                |  [Pointer arithmetic]     |--> rd_almost_empty
                +---------------------------+

Source: 02_beats_alloc_ctrl_block.mmd

## 10.2	Concept: Virtual FIFO

Unlike a traditional FIFO that stores data, beats_alloc_ctrl only tracks space reservations:

Traditional FIFO:          Virtual FIFO (alloc_ctrl):

  wr_data --> [storage] -> rd_data    wr_size --> [pointers] --> space_free

  Data moves through FIFO             Only pointers move, no data

### 10.2.1	Use Case: Pre-Allocation for AXI Reads

1. AXI engine requests N beats of space
   - alloc_ctrl.wr_valid = 1
   - alloc_ctrl.wr_size = N
   - wr_ptr advances by N

2. As AXI read data arrives (1 beat at a time)
   - alloc_ctrl.rd_valid = 1 (each beat)
   - rd_ptr advances by 1

3. Space becomes available again
   - space_free reflects available capacity

## 10.3	Parameters

parameter int DEPTH = 512;                      // Virtual FIFO depth
parameter int ALMOST_WR_MARGIN = 1;             // Almost full margin
parameter int ALMOST_RD_MARGIN = 1;             // Almost empty margin
parameter int REGISTERED = 1;                   // Registered outputs

// Derived
parameter int AW = $clog2(DEPTH);               // Address width

: Table 2.5.1: Beats Alloc Control Parameters

## 10.4	Port List

### 10.4.1	Clock and Reset

Table 2.5.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low reset |

### 10.4.2	Write Interface (Allocation Requests)

Table 2.5.3: Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | Allocate space |
| wr_size | input | 8 | Number of beats to allocate |
| wr_ready | output | 1 | Space available for allocation |

### 10.4.3	Read Interface (Data Written)

Table 2.5.4: Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | One beat of data written |
| rd_ready | output | 1 | Not full (can accept) |

### 10.4.4	Status Outputs

Table 2.5.5: Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| space_free | output | AW+1 | Available space in beats |
| wr_full | output | 1 | No space available |
| wr_almost_full | output | 1 | Near full |
| rd_empty | output | 1 | No allocations pending |
| rd_almost_empty | output | 1 | Near empty |

## 10.5	Operation

### 10.5.1	Pointer Arithmetic

space_free = DEPTH - (wr_ptr - rd_ptr)

Full condition:    wr_ptr - rd_ptr >= DEPTH
Empty condition:   wr_ptr == rd_ptr

### 10.5.2	Timing Diagram

### 10.5.3	Figure 2.5.2: Allocation and Release Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    wr_valid       _/‾\_____:_______:_______:_______:_______:_______
    wr_size        X| 8 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    rd_valid       _________/‾\_____/‾\_____/‾\_____/‾\_____/‾\_____
                    :       :       :       :       :       :
    space_free     |==512===|==504==|==505==|==506==|==507==|==508==
                    :       :       :       :       :       :
    wr_ptr         |== 0 ===|== 8 ==|== 8 ==|== 8 ==|== 8 ==|== 8 ==
    rd_ptr         |== 0 ===|== 0 ==|== 1 ==|== 2 ==|== 3 ==|== 4 ==

TODO: Replace with simulation-generated waveform showing actual flow control

## 10.6	Integration Example

// Source path: Pre-allocate SRAM space before AXI read
beats_alloc_ctrl #(
    .DEPTH(SRAM_DEPTH),
    .ALMOST_WR_MARGIN(8)
) u_src_alloc (
    .axi_aclk       (clk),
    .axi_aresetn    (rst_n),

    // Scheduler allocates space before issuing AXI read
    .wr_valid       (sched_rd_valid),
    .wr_size        (sched_rd_beats[7:0]),
    .wr_ready       (alloc_ready),

    // AXI read data arrival releases allocation
    .rd_valid       (m_axi_rvalid && m_axi_rready),
    .rd_ready       (),  // Not used for single-beat

    // Status
    .space_free     (sram_space_available),
    .wr_full        (sram_full),
    .wr_almost_full (sram_almost_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Beats Drain Control Specification

Module: beats_drain_ctrl.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 11.1	Overview

The Beats Drain Control is a “virtual FIFO” that tracks data availability without storing actual data. It complements beats_alloc_ctrl by tracking when data has been written to SRAM and is available for draining.

### 11.1.1	Key Features

- Virtual FIFO: Pointer tracking without data storage

- Single-Beat Write: Data arrivals tracked one beat at a time

- Variable-Size Drain: Supports multi-beat drain requests

- Status Flags: Full, almost full, empty, almost empty

- Data Availability: Reports beats available for drain

### 11.1.2	Block Diagram

### 11.1.3	Figure 2.6.1: Beats Drain Control Block Diagram

+---------------------------+
    wr_valid -->|                           |--> wr_ready
                |    BEATS_DRAIN_CTRL       |--> data_avail
    rd_valid -->|                           |
    rd_size  -->|     (Virtual FIFO)        |--> wr_full
                |                           |--> wr_almost_full
                |  [No data storage]        |--> rd_empty
                |  [Pointer arithmetic]     |--> rd_almost_empty
                +---------------------------+

Source: 02_beats_drain_ctrl_block.mmd

## 11.2	Concept: Data Availability Tracking

The drain_ctrl tracks when data has been written to SRAM:

alloc_ctrl:  Tracks SPACE reservations (allocation -> actual write)
drain_ctrl:  Tracks DATA availability (data write -> drain request)

### 11.2.1	Complementary Roles

Source Path:
  1. Scheduler allocates space (alloc_ctrl.wr)
  2. AXI reads data into SRAM
  3. Data written to SRAM (drain_ctrl.wr)
  4. Drain consumer reads (drain_ctrl.rd)

Sink Path:
  1. Fill allocates space (alloc_ctrl.wr)
  2. Fill writes to SRAM (alloc_ctrl.rd)
  3. Data available in SRAM (drain_ctrl.wr)
  4. AXI write drains SRAM (drain_ctrl.rd)

## 11.3	Parameters

parameter int DEPTH = 512;                      // Virtual FIFO depth
parameter int ALMOST_WR_MARGIN = 1;             // Almost full margin
parameter int ALMOST_RD_MARGIN = 1;             // Almost empty margin
parameter int REGISTERED = 1;                   // Registered outputs

// Derived
parameter int AW = $clog2(DEPTH);               // Address width

: Table 2.6.1: Beats Drain Control Parameters

## 11.4	Port List

### 11.4.1	Clock and Reset

Table 2.6.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low reset |

### 11.4.2	Write Interface (Data Written to SRAM)

Table 2.6.3: Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | One beat written to SRAM |
| wr_ready | output | 1 | Can track more data |

### 11.4.3	Read Interface (Drain Requests)

Table 2.6.4: Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | Drain request |
| rd_size | input | 8 | Number of beats to drain |
| rd_ready | output | 1 | Data available for drain |

### 11.4.4	Status Outputs

Table 2.6.5: Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| data_avail | output | AW+1 | Beats available to drain |
| wr_full | output | 1 | Tracking full |
| wr_almost_full | output | 1 | Near full |
| rd_empty | output | 1 | No data to drain |
| rd_almost_empty | output | 1 | Near empty |

## 11.5	Operation

### 11.5.1	Pointer Arithmetic

data_avail = wr_ptr - rd_ptr

Full condition:    wr_ptr - rd_ptr >= DEPTH
Empty condition:   wr_ptr == rd_ptr

### 11.5.2	Timing Diagram

### 11.5.3	Figure 2.6.2: Data Arrival and Drain Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    wr_valid       _/‾\_____/‾\_____/‾\_____/‾\_____:_______:_______
                    :       :       :       :       :       :
    rd_valid       _________:_______:_______:_______/‾\_____:_______
    rd_size        X:XXXXXXX:XXXXXXX:XXXXXXX| 4 |XXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    data_avail     |== 0 ===|== 1 ==|== 2 ==|== 3 ==|== 4 ==|== 0 ==
                    :       :       :       :       :       :
    rd_empty       ‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾/‾‾‾‾‾‾‾

TODO: Replace with simulation-generated waveform showing actual flow control

## 11.6	Integration Example

// Sink path: Track data availability for AXI write engine
beats_drain_ctrl #(
    .DEPTH(SRAM_DEPTH),
    .ALMOST_RD_MARGIN(8)
) u_snk_drain (
    .axi_aclk       (clk),
    .axi_aresetn    (rst_n),

    // Fill writes data to SRAM
    .wr_valid       (fill_valid && fill_ready),
    .wr_ready       (),  // Single-beat, always ready

    // AXI write engine drains data
    .rd_valid       (axi_wr_drain_req),
    .rd_size        (axi_wr_burst_len),
    .rd_ready       (data_ready_for_drain),

    // Status
    .data_avail     (beats_to_write),
    .rd_empty       (no_data_to_write)
);

## 11.7	Relationship: alloc_ctrl vs drain_ctrl

Table 2.6.6: alloc_ctrl vs drain_ctrl Comparison

| Aspect | beats_alloc_ctrl | beats_drain_ctrl |
| --- | --- | --- |
| Tracks | Space reservations | Data availability |
| Write port | Multi-beat allocation | Single-beat arrival |
| Read port | Single-beat release | Multi-beat drain |
| Output | space_free | data_avail |
| Use case | Pre-allocate before fill | Know when to drain |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Beats Latency Bridge Specification

Module: beats_latency_bridge.sv Location: projects/components/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 12.1	Overview

The Beats Latency Bridge provides buffering to compensate for pipeline latency between alloc_ctrl and drain_ctrl in the SRAM controller. It prevents race conditions where drain signals arrive before allocation is complete.

### 12.1.1	Key Features

- Latency Compensation: Buffers requests to match pipeline delays

- Configurable Depth: Matches expected pipeline latency

- Flow Control: Standard valid/ready handshaking

- Pass-Through Data: Preserves beat count and channel ID

### 12.1.2	Block Diagram

### 12.1.3	Figure 2.7.1: Beats Latency Bridge Block Diagram

+---------------------------+
    in_valid -->|                           |--> out_valid
    in_ready <--|    BEATS_LATENCY_BRIDGE   |--> out_ready (input)
    in_beats -->|                           |--> out_beats
    in_id    -->|    [Buffered Pipeline]    |--> out_id
                +---------------------------+

Source: 02_beats_latency_bridge_block.mmd

## 12.2	Concept: Why Latency Compensation?

The SRAM controller has a timing challenge:

Without Bridge:                        With Bridge:

  alloc_ctrl.wr ----+                    alloc_ctrl.wr ----+
                    |                                      |
                    v                                      v
  +----------+  (immediate)              +----------+  +--------+
  |   SRAM   |<--- drain_ctrl.rd         |   SRAM   |  |LATENCY |
  +----------+                           +----------+  | BRIDGE |
                                                       +---+----+
  Problem: drain_ctrl.rd may arrive                        |
  before alloc_ctrl has reserved                           v
  the space -> race condition!            drain_ctrl.rd (delayed)

                                         Solution: Bridge delays drain
                                         signals to match alloc timing

## 12.3	Parameters

parameter int DEPTH = 4;                         // Bridge FIFO depth
parameter int BEATS_WIDTH = 8;                   // Beat count width
parameter int ID_WIDTH = 3;                      // Channel ID width
parameter int REGISTERED = 1;                    // Registered outputs

: Table 2.7.1: Beats Latency Bridge Parameters

## 12.4	Port List

### 12.4.1	Clock and Reset

Table 2.7.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 12.4.2	Input Interface

Table 2.7.3: Input Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| in_valid | input | 1 | Input request valid |
| in_ready | output | 1 | Bridge ready to accept |
| in_beats | input | BEATS_WIDTH | Beat count |
| in_id | input | ID_WIDTH | Channel ID |

### 12.4.3	Output Interface

Table 2.7.4: Output Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| out_valid | output | 1 | Output request valid |
| out_ready | input | 1 | Downstream ready |
| out_beats | output | BEATS_WIDTH | Beat count (delayed) |
| out_id | output | ID_WIDTH | Channel ID (delayed) |

## 12.5	Operation

### 12.5.1	Timing Diagram

### 12.5.2	Figure 2.7.2: Latency Bridge Timing (Depth=2)

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    in_valid       _/‾\_____/‾\_____:_______:_______:_______:_______
    in_beats       X| 8 |XXX| 4 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    out_valid      _________:_______/‾\_____/‾\_____:_______:_______
    out_beats      X:XXXXXXX:XXXXXXX| 8 |XXX| 4 |XXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
                    :       :       :       :       :       :
                    +-------+       +-------+
                    | 2-cycle latency through bridge |

TODO: Replace with simulation-generated waveform

## 12.6	Integration Context

The latency bridge is used within the SRAM controller:

// In snk_sram_controller_unit
beats_alloc_ctrl u_alloc (
    // Allocation on fill requests
    .wr_valid       (fill_valid),
    .wr_size        (fill_size),
    // Release on actual data write
    .rd_valid       (sram_wr_complete)
);

beats_latency_bridge u_bridge (
    // Input from SRAM write path
    .in_valid       (sram_wr_complete),
    .in_beats       (beats_written),
    // Output to drain controller (delayed)
    .out_valid      (drain_trigger),
    .out_beats      (drain_beats)
);

beats_drain_ctrl u_drain (
    // Delayed notification of data available
    .wr_valid       (drain_trigger),
    // Drain requests from write engine
    .rd_valid       (axi_drain_req),
    .rd_size        (axi_drain_size)
);

## 12.7	Design Considerations

Table 2.7.5: Bridge Depth Selection

| Depth | Use Case | Latency |
| --- | --- | --- |
| 2 | Minimal pipeline | 2 cycles |
| 4 | Standard pipeline | 4 cycles |
| 8 | Deep pipeline | 8 cycles |

The bridge depth should match the maximum pipeline latency from alloc_ctrl write to SRAM data being valid for drain_ctrl.

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Beats Scheduler Group Specification

Module: beats_scheduler_group.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 13.1	Overview

The Beats Scheduler Group wraps a single channel’s scheduler and descriptor engine together with MonBus aggregation. It provides a clean integration point for the scheduler_group_array.

### 13.1.1	Key Features

- Single Channel Integration: One scheduler + one descriptor engine

- MonBus Aggregation: Combines scheduler and descriptor engine MonBus outputs

- Configuration Pass-Through: Routes configuration to both sub-modules

- Status Aggregation: Combined idle and error status

### 13.1.2	Block Diagram

### 13.1.3	Figure 3.1.1: Beats Scheduler Group Block Diagram

beats_scheduler_group
    +-------------------------------------------------------+
    |                                                       |
    |  +-------------------+    +-------------------+       |
    |  | descriptor_engine |    |     scheduler     |       |
    |  |                   |--->|                   |       |
    |  | - AXI AR/R        |    | - FSM             |       |
    |  | - Prefetch FIFO   |desc| - Beat tracking   |       |
    |  | - Address check   |--->| - Error handling  |       |
    |  +--------+----------+    +---------+---------+       |
    |           |                         |                 |
    |           | monbus                  | monbus          |
    |           v                         v                 |
    |  +-----------------------------------------------+    |
    |  |           MonBus Arbiter (2:1)                |    |
    |  +----------------------+------------------------+    |
    |                         |                             |
    +-------------------------|-----------------------------+
                              v
                       monbus_pkt_valid/data

Source: 03_beats_scheduler_group_block.mmd

## 13.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width

// Monitor Bus Parameters
parameter int DESC_MON_AGENT_ID = 16 + CHANNEL_ID;  // Descriptor engine agent
parameter int SCHED_MON_AGENT_ID = 48 + CHANNEL_ID; // Scheduler agent
parameter int MON_UNIT_ID = 1;

: Table 3.1.1: Beats Scheduler Group Parameters

## 13.3	Port List

### 13.3.1	Clock and Reset

Table 3.1.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 13.3.2	APB Programming Interface

Table 3.1.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Channel kick-off |
| apb_ready | output | 1 | Ready for kick-off |
| apb_addr | input | AW | First descriptor address |

### 13.3.3	Configuration Interface

Table 3.1.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable channel |
| cfg_channel_reset | input | 1 | Soft reset |
| cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window (cycles) |
| cfg_sched_timeout_limit | input | 8 | Consecutive-timeout windows before fatal escalation (0 = never) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout |
| cfg_desceng_enable | input | 1 | Enable descriptor engine |
| cfg_desceng_prefetch | input | 1 | Enable prefetch chaining |
| cfg_desceng_fifo_thresh | input | 4 | Prefetch threshold |
| cfg_desceng_addr0_base | input | AW | Address range 0 base |
| cfg_desceng_addr0_limit | input | AW | Address range 0 limit |

### 13.3.4	Descriptor AXI Master Interface

Table 3.1.5: Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | AR address |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | 256 | R data (descriptor) |
| m_axi_rresp | input | 2 | R response |

### 13.3.5	Scheduler Data Interfaces

Table 3.1.6: Scheduler Data Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Read request |
| sched_rd_addr | output | AW | Read address |
| sched_rd_beats | output | 32 | Read beats |
| sched_rd_done_strobe | input | 1 | Read complete |
| sched_wr_valid | output | 1 | Write request |
| sched_wr_addr | output | AW | Write address |
| sched_wr_beats | output | 32 | Write beats |
| sched_wr_done_strobe | input | 1 | Write AW-issue strobe |
| sched_wr_commit_strobe | input | 1 | Write COMMIT strobe (B response) |
| sched_wr_commit_beats | input | 32 | Beats committed this strobe |

### 13.3.6	Status

Table 3.1.7: Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| scheduler_idle | output | 1 | Scheduler idle |
| descriptor_engine_idle | output | 1 | Descriptor engine idle |
| scheduler_state | output | 7 | FSM state |
| sched_error | output | 1 | Error flag |

### 13.3.7	MonBus Interface

Table 3.1.8: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | output | 1 | Packet valid |
| monbus_pkt_ready | input | 1 | Consumer ready |
| monbus_pkt_data | output | 64 | Packet data |

## 13.4	Internal Architecture

Internal Signal Flow:

apb_valid/addr -----> descriptor_engine -----> scheduler
                             |                     |
                      desc_valid/packet           |
                             |                     |
                             +--> descriptor_ready <--+

MonBus Aggregation:

desc_engine.monbus_pkt ----+
                           |---> round_robin_arbiter ---> monbus_out
scheduler.monbus_pkt   ----+

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Beats Scheduler Group Array Specification

Module: beats_scheduler_group_array.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 14.1	Overview

The Beats Scheduler Group Array instantiates 8 scheduler groups with a shared descriptor AXI master and unified MonBus output. It provides the complete scheduler infrastructure for all RAPIDS channels.

### 14.1.1	Key Features

- 8-Channel Array: One scheduler_group per channel

- Shared Descriptor AXI: Round-robin arbitration for descriptor fetches

- Unified MonBus: Aggregates 9 MonBus sources (8 scheduler + 1 arbiter)

- Per-Channel Configuration: Individual enable/reset per channel

- Aggregate Status: Combined idle and error flags

### 14.1.2	Block Diagram

### 14.1.3	Figure 3.2.1: Beats Scheduler Group Array Block Diagram

beats_scheduler_group_array
    +------------------------------------------------------------------+
    |                                                                  |
    |  +------------------+  +------------------+  +------------------+ |
    |  | scheduler_group  |  | scheduler_group  |  | scheduler_group  | |
    |  |      [0]         |  |      [1]         |  |      [7]         | |
    |  +--------+---------+  +--------+---------+  +--------+---------+ |
    |           |                     |                     |          |
    |           | AXI AR/R            | AXI AR/R            | AXI AR/R |
    |           v                     v                     v          |
    |  +----------------------------------------------------------+   |
    |  |              Round-Robin AXI AR/R Arbiter                |   |
    |  +----------------------------+-----------------------------+   |
    |                               |                                  |
    |                               v                                  |
    |                     Shared Descriptor AXI                        |
    |                                                                  |
    |  MonBus from each group:                                        |
    |           |                     |                     |          |
    |           v                     v                     v          |
    |  +----------------------------------------------------------+   |
    |  |            MonBus Round-Robin Arbiter (9 sources)        |   |
    |  +----------------------------+-----------------------------+   |
    |                               |                                  |
    +-------------------------------|----------------------------------+
                                    v
                          monbus_pkt_valid/data

Source: 03_beats_scheduler_group_array_block.mmd

## 14.2	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width
parameter int DESC_DATA_WIDTH = 256;             // Descriptor width

// AXI ID Management
parameter int AXI_ID_WIDTH = 8;                  // ID field width
parameter int MAX_OUTSTANDING = 8;               // Outstanding descriptors

// Monitor Bus Parameters
parameter int MON_UNIT_ID = 1;                   // Unit ID for MonBus

: Table 3.2.1: Beats Scheduler Group Array Parameters

## 14.3	Port List

### 14.3.1	Clock and Reset

Table 3.2.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 14.3.2	Per-Channel APB Programming

Table 3.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | NC | Channel kick-off (per-channel) |
| apb_ready | output | NC | Ready for kick-off |
| apb_addr | input | NC*AW | First descriptor address |

### 14.3.3	Per-Channel Configuration

Table 3.2.4: Per-Channel Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | NC | Enable per channel |
| cfg_channel_reset | input | NC | Soft reset per channel |
| cfg_sched_timeout_cycles | input | NC*16 | Timeout threshold |
| cfg_sched_timeout_enable | input | NC | Enable timeout |
| cfg_desceng_enable | input | NC | Enable descriptor engine |
| cfg_desceng_prefetch | input | NC | Enable prefetching |
| cfg_desceng_fifo_thresh | input | NC*4 | Prefetch threshold |
| cfg_desceng_addr0_base | input | NC*AW | Address range 0 base |
| cfg_desceng_addr0_limit | input | NC*AW | Address range 0 limit |

### 14.3.4	Shared Descriptor AXI Master Interface

Table 3.2.5: Shared Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | AR address |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_arlen | output | 8 | Burst length |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | 256 | R data |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rresp | input | 2 | R response |
| m_axi_rlast | input | 1 | Last beat |

### 14.3.5	Per-Channel Scheduler Data Interfaces

Table 3.2.6: Per-Channel Scheduler Data Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | NC | Read request per channel |
| sched_rd_addr | output | NC*AW | Read address |
| sched_rd_beats | output | NC*32 | Read beats |
| sched_rd_done_strobe | input | NC | Read complete |
| sched_wr_valid | output | NC | Write request per channel |
| sched_wr_addr | output | NC*AW | Write address |
| sched_wr_beats | output | NC*32 | Write beats |
| sched_wr_done_strobe | input | NC | Write complete |

### 14.3.6	Aggregate Status

Table 3.2.7: Aggregate Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| all_channels_idle | output | 1 | All channels idle |
| scheduler_idle | output | NC | Per-channel scheduler idle |
| descriptor_engine_idle | output | NC | Per-channel desc eng idle |
| scheduler_state | output | NC*7 | FSM states |
| sched_error | output | NC | Error flags |

### 14.3.7	Unified MonBus Interface

Table 3.2.8: Unified MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | output | 1 | Packet valid |
| monbus_pkt_ready | input | 1 | Consumer ready |
| monbus_pkt_data | output | 64 | Packet data |

## 14.4	AXI Arbitration

### 14.4.1	Figure 3.2.2: Descriptor AXI Arbitration Sequence

Channel[0]    Channel[1]    Arbiter        AXI Master
            |             |            |               |
    AR REQ  |------------>|            |               |
            |             | AR REQ     |               |
            |             |----------->|               |
            |             |            | GRANT[0]      |
            |<-------------------------|               |
            |             |            |  AR VALID     |
            |             |            |-------------->|
            |             |            |  AR READY     |
            |             |            |<--------------|
            |             |            | GRANT[1]      |
            |             |<-----------|               |
            |             |            |  AR VALID     |
            |             |            |-------------->|
            |             |            |  R DATA       |
            |             |            |<--------------|
            |    R DATA   |            |               |
            |<-------------------------|               |
            |             |    R DATA  |               |
            |             |<-----------|               |
            |             |            |               |

Source: 03_scheduler_group_array_arbitration.mmd

## 14.5	MonBus Aggregation

The unified MonBus output aggregates 9 sources using round-robin arbitration:

Table 3.2.9: MonBus Source Assignment

| Source ID | Origin | Description |
| --- | --- | --- |
| 0-7 | scheduler_group[0:7] | Per-channel aggregated MonBus |
| 8 | AXI Arbiter | Arbitration events |

Each scheduler_group provides a single MonBus output that combines both scheduler and descriptor engine packets (2:1 internal arbitration).

## 14.6	Integration Context

beats_scheduler_group_array #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512)
) u_scheduler_array (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // APB programming (per-channel)
    .apb_valid              (apb_kick_valid),
    .apb_ready              (apb_kick_ready),
    .apb_addr               (apb_kick_addr),

    // Configuration (per-channel)
    .cfg_channel_enable     (cfg_ch_enable),
    .cfg_sched_timeout_cycles(cfg_timeout_cycles),
    // ... additional config ...

    // Shared descriptor AXI
    .m_axi_arvalid          (desc_axi_arvalid),
    .m_axi_arready          (desc_axi_arready),
    .m_axi_araddr           (desc_axi_araddr),
    .m_axi_rvalid           (desc_axi_rvalid),
    .m_axi_rready           (desc_axi_rready),
    .m_axi_rdata            (desc_axi_rdata),

    // Per-channel scheduler outputs
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_done_strobe   (sched_rd_done_strobe),
    .sched_wr_valid         (sched_wr_valid),
    .sched_wr_addr          (sched_wr_addr),
    .sched_wr_beats         (sched_wr_beats),
    .sched_wr_done_strobe   (sched_wr_done_strobe),

    // Status
    .all_channels_idle      (all_schedulers_idle),

    // Unified MonBus
    .monbus_pkt_valid       (sched_array_monbus_valid),
    .monbus_pkt_ready       (sched_array_monbus_ready),
    .monbus_pkt_data        (sched_array_monbus_data)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Sink Data Path Specification

Module: sink_data_path.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 15.1	Overview

The Sink Data Path integrates the SRAM controller and AXI write engine for network-to-memory transfers. Data enters through the fill interface, is buffered in SRAM, and is written to system memory via AXI4.

### 15.1.1	Key Features

- 8-Channel SRAM Controller: Per-channel buffering with flow control

- AXI Write Engine: Streaming burst writes to system memory

- Beat-Level Tracking: Precise flow control at beat granularity

- Scheduler Integration: Receives transfer requests from scheduler

### 15.1.2	Block Diagram

### 15.1.3	Figure 3.3.1: Sink Data Path Block Diagram

sink_data_path
    +-------------------------------------------------------+
    |                                                       |
    |    Fill Interface (External)                          |
    |         |                                             |
    |         v                                             |
    |  +---------------------------------------------+      |
    |  |         snk_sram_controller                 |      |
    |  |  +-------+  +-------+       +-------+       |      |
    |  |  | unit  |  | unit  |  ...  | unit  |       |      |
    |  |  |  [0]  |  |  [1]  |       |  [7]  |       |      |
    |  |  +-------+  +-------+       +-------+       |      |
    |  |      |          |              |            |      |
    |  |      v          v              v            |      |
    |  |  +-------------------------------------+    |      |
    |  |  |        Channel Arbitration          |    |      |
    |  |  +------------------+------------------+    |      |
    |  +---------------------|----------------------+       |
    |                        |                              |
    |                        v                              |
    |  +---------------------------------------------+      |
    |  |            axi_write_engine                 |      |
    |  |  - Burst generation                         |      |
    |  |  - W channel streaming                      |      |
    |  |  - B channel response tracking              |      |
    |  +---------------------+-----------------------+      |
    |                        |                              |
    +------------------------|------------------------------+
                             v
                     AXI Write Master
                   (to System Memory)

Source: 03_sink_data_path_block.mmd

## 15.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int AW_MAX_OUTSTANDING = 8;
parameter int W_PHASE_FIFO_DEPTH = 64;
parameter int B_PHASE_FIFO_DEPTH = 16;

: Table 3.3.1: Sink Data Path Parameters

## 15.3	Data Flow

### 15.3.1	Figure 3.3.2: Sink Path Data Flow Sequence

1. Fill Request
   - snk_fill_alloc_req = 1
   - snk_fill_alloc_size = N (beats to allocate)
   - snk_fill_alloc_id = channel
        |
        v
2. Space Check (beats_alloc_ctrl)
   - Check snk_fill_space_free[channel] >= N
   - Allocate N beats of space
        |
        v
3. Fill Data Transfer
   - snk_fill_valid = 1
   - snk_fill_data = data beats
   - snk_fill_id = channel
        |
        v
4. SRAM Write
   - Data written to channel's SRAM partition
   - beats_drain_ctrl notified of data availability
        |
        v
5. AXI Write Request
   - Scheduler requests write: sched_wr_valid
   - Address and beat count provided
        |
        v
6. SRAM Read + AXI Write
   - axi_write_engine reads from SRAM
   - Issues AXI bursts to memory
        |
        v
7. Completion
   - B channel response received
   - sched_wr_done_strobe = 1

Source: 03_sink_data_flow.mmd

## 15.4	Timing Diagram

### 15.4.1	Figure 3.3.3: Sink Path Transfer Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    fill_alloc_req _/‾\_____:_______:_______:_______:_______:_______
    fill_alloc_size| 4 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    fill_valid     _________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\_______:_______:_______
    fill_data      X:XXXXXXX|==D0==|==D1==|==D2==|==D3==|XXXXXXXXX
                    :       :       :       :       :       :
    sched_wr_valid _________:_______:_______:_______/‾‾‾‾‾‾‾\_______
    sched_wr_beats X:XXXXXXX:XXXXXXX:XXXXXXX| 4 |XXXXXXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    m_axi_awvalid  _________:_______:_______:_______/‾\_____:_______
    m_axi_wvalid   _________:_______:_______:_______:_/‾‾‾‾‾\_______
    m_axi_bvalid   _________:_______:_______:_______:_______:_/‾\___
                    :       :       :       :       :       :
    sched_wr_done  _________:_______:_______:_______:_______:___/‾\_

TODO: Replace with simulation-generated waveform showing complete sink transfer

## 15.5	Interface Summary

### 15.5.1	Fill Interface (Input)

Table 3.3.2: Fill Interface

| Signal | Direction | Description |
| --- | --- | --- |
| snk_fill_alloc_req | input | Allocation request |
| snk_fill_alloc_size | input | Beats to allocate |
| snk_fill_alloc_id | input | Channel ID |
| snk_fill_space_free | output | Available space per channel |
| snk_fill_valid | input | Data valid |
| snk_fill_ready | output | Ready for data |
| snk_fill_data | input | Fill data |
| snk_fill_last | input | Last beat |
| snk_fill_id | input | Channel ID |

### 15.5.2	Scheduler Interface

Table 3.3.3: Scheduler Interface

| Signal | Direction | Description |
| --- | --- | --- |
| sched_wr_valid | input | Write request |
| sched_wr_addr | input | Destination address |
| sched_wr_beats | input | Beats to write |
| sched_wr_id | input | Channel ID |
| sched_wr_done_strobe | output | Write complete |
| sched_wr_beats_done | output | Beats completed |
| sched_wr_error | output | Error flag |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Sink Data Path AXIS Wrapper Specification

Module: sink_data_path_axis.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 16.1	Overview

The Sink Data Path AXIS wrapper adds an AXI-Stream slave interface to the sink data path, enabling standard AXIS ingress for network-to-memory transfers.

### 16.1.1	Key Features

- AXI-Stream Slave Interface: Standard AXIS for data ingress

- Per-Channel TID Mapping: AXIS TID maps to fill channel ID

- TLAST to Last Mapping: Packet boundary tracking

- Core Sink Integration: Wraps sink_data_path module

### 16.1.2	Block Diagram

### 16.1.3	Figure 3.4.1: Sink Data Path AXIS Block Diagram

sink_data_path_axis
    +-------------------------------------------------------+
    |                                                       |
    |    AXI-Stream Slave Interface                         |
    |    +-------------------------------------------+      |
    |    | s_axis_tvalid   s_axis_tready             |      |
    |    | s_axis_tdata    s_axis_tlast              |      |
    |    | s_axis_tid      s_axis_tkeep              |      |
    |    +--------------------+----------------------+      |
    |                         |                             |
    |                         v                             |
    |    +-------------------------------------------+      |
    |    |          AXIS to Fill Converter           |      |
    |    |  - TID -> fill_id                         |      |
    |    |  - TLAST -> fill_last                     |      |
    |    |  - TVALID/TREADY -> fill handshaking     |      |
    |    +--------------------+----------------------+      |
    |                         |                             |
    |                         v                             |
    |    +-------------------------------------------+      |
    |    |            sink_data_path                 |      |
    |    |  (snk_sram_controller + axi_write_engine) |      |
    |    +--------------------+----------------------+      |
    |                         |                             |
    +-------------------------|-----------------------------+
                              v
                      AXI Write Master

Source: 03_sink_data_path_axis_block.mmd

## 16.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int TID_WIDTH = 3;                     // log2(NUM_CHANNELS)
parameter int TDEST_WIDTH = 1;
parameter int TUSER_WIDTH = 1;

: Table 3.4.1: Sink Data Path AXIS Parameters

## 16.3	Port List

### 16.3.1	Clock and Reset

Table 3.4.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 16.3.2	AXI-Stream Slave Interface

Table 3.4.3: AXI-Stream Slave Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tvalid | input | 1 | Data valid |
| s_axis_tready | output | 1 | Ready for data |
| s_axis_tdata | input | DATA_WIDTH | Data payload |
| s_axis_tkeep | input | DATA_WIDTH/8 | Byte enables |
| s_axis_tlast | input | 1 | Last beat of packet |
| s_axis_tid | input | TID_WIDTH | Stream ID (channel) |
| s_axis_tdest | input | TDEST_WIDTH | Destination |
| s_axis_tuser | input | TUSER_WIDTH | User sideband |

### 16.3.3	Space Allocation Interface

Table 3.4.4: Space Allocation Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_alloc_req | input | 1 | Allocation request |
| snk_alloc_size | input | 16 | Beats to allocate |
| snk_alloc_id | input | 3 | Channel ID |
| snk_space_free | output | NC*16 | Available space per channel |

### 16.3.4	Scheduler Interface

Table 3.4.5: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | input | 1 | Write request |
| sched_wr_addr | input | AW | Destination address |
| sched_wr_beats | input | 32 | Beats to write |
| sched_wr_id | input | 3 | Channel ID |
| sched_wr_done_strobe | output | 1 | Write complete |
| sched_wr_beats_done | output | 32 | Beats completed |
| sched_wr_error | output | 1 | Error flag |

### 16.3.5	AXI Write Master Interface

Table 3.4.6: AXI Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awvalid | output | 1 | AW channel valid |
| m_axi_awready | input | 1 | AW channel ready |
| m_axi_awaddr | output | AW | Write address |
| m_axi_awlen | output | 8 | Burst length |
| m_axi_awsize | output | 3 | Burst size |
| m_axi_awburst | output | 2 | Burst type |
| m_axi_awid | output | ID_W | Transaction ID |
| m_axi_wvalid | output | 1 | W channel valid |
| m_axi_wready | input | 1 | W channel ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat |
| m_axi_bvalid | input | 1 | B channel valid |
| m_axi_bready | output | 1 | B channel ready |
| m_axi_bresp | input | 2 | Write response |
| m_axi_bid | input | ID_W | Response ID |

## 16.4	Signal Mapping

### 16.4.1	Figure 3.4.2: AXIS to Fill Interface Mapping

AXIS Slave                    Fill Interface
    +----------------+            +----------------+
    | s_axis_tvalid  |----------->| snk_fill_valid |
    | s_axis_tready  |<-----------| snk_fill_ready |
    | s_axis_tdata   |----------->| snk_fill_data  |
    | s_axis_tlast   |----------->| snk_fill_last  |
    | s_axis_tid     |----------->| snk_fill_id    |
    | s_axis_tkeep   |----------->| snk_fill_strb  |
    +----------------+            +----------------+

Source: 03_sink_axis_mapping.mmd

## 16.5	Timing Diagram

### 16.5.1	Figure 3.4.3: AXIS Ingress Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    s_axis_tvalid  _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    s_axis_tready  _______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    s_axis_tdata   X:XXXXX|=D0==|=D1==|=D2==|=D3==|XXX:XXXXXXX:XXXXXXX
    s_axis_tid     X:XXXXX| CH2 | CH2 | CH2 | CH2 |XXX:XXXXXXX:XXXXXXX
    s_axis_tlast   ___________________:_______/‾\___:_______:_______
                    :       :       :       :       :       :
    snk_fill_valid _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    snk_fill_id    X:XXXXX| CH2 | CH2 | CH2 | CH2 |XXX:XXXXXXX:XXXXXXX
    snk_fill_last  ___________________:_______/‾\___:_______:_______

TODO: Replace with simulation-generated waveform showing AXIS packet reception

## 16.6	Usage Notes

- TID Usage: AXIS TID directly maps to channel ID for multi-channel routing

- Allocation Required: Space must be allocated via snk_alloc_* before AXIS data arrives

- Backpressure: s_axis_tready deasserts when SRAM space exhausted

- Packet Boundaries: TLAST marks end of AXIS packets, mapped to fill_last

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Sink SRAM Controller Specification

Module: snk_sram_controller.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 17.1	Overview

The Sink SRAM Controller manages 8 SRAM controller units, providing per-channel buffering with channel arbitration for the shared AXI write engine.

### 17.1.1	Key Features

- 8-Channel SRAM Array: Instantiates 8 snk_sram_controller_unit modules

- Channel Arbitration: Round-robin access to shared AXI write engine

- Flow Control Integration: Per-channel alloc_ctrl and drain_ctrl

- Space Tracking: Reports available space per channel

### 17.1.2	Block Diagram

### 17.1.3	Figure 3.5.1: Sink SRAM Controller Block Diagram

snk_sram_controller
    +------------------------------------------------------------------+
    |                                                                  |
    |  Fill Interface (per-channel)                                    |
    |         |         |                   |                          |
    |         v         v                   v                          |
    |  +----------+ +----------+     +----------+                      |
    |  |snk_sram_ | |snk_sram_ | ... |snk_sram_ |                      |
    |  |ctrl_unit | |ctrl_unit |     |ctrl_unit |                      |
    |  |   [0]    | |   [1]    |     |   [7]    |                      |
    |  +----+-----+ +----+-----+     +----+-----+                      |
    |       |            |                |                            |
    |       |drain_req   |drain_req       |drain_req                   |
    |       v            v                v                            |
    |  +----------------------------------------------------------+   |
    |  |         Round-Robin Channel Arbiter                      |   |
    |  +----------------------------+-----------------------------+   |
    |                               |                                  |
    |                               v                                  |
    |                    SRAM Read Interface                           |
    |                    (to AXI Write Engine)                         |
    |                                                                  |
    +------------------------------------------------------------------+

Source: 03_snk_sram_controller_block.mmd

## 17.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;                  // Per-channel depth
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);

: Table 3.5.1: Sink SRAM Controller Parameters

## 17.3	Port List

### 17.3.1	Clock and Reset

Table 3.5.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 17.3.2	Per-Channel Fill Interface

Table 3.5.3: Per-Channel Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_fill_alloc_req | input | NC | Allocation request per channel |
| snk_fill_alloc_size | input | NC*16 | Beats to allocate |
| snk_fill_space_free | output | NC*16 | Available space per channel |
| snk_fill_valid | input | NC | Fill data valid |
| snk_fill_ready | output | NC | Ready for fill data |
| snk_fill_data | input | NC*DW | Fill data |
| snk_fill_last | input | NC | Last beat marker |

### 17.3.3	Arbitrated Drain Interface (to AXI Write Engine)

Table 3.5.4: Arbitrated Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_req | output | 1 | Drain request (any channel) |
| drain_gnt | input | 1 | Drain grant |
| drain_id | output | 3 | Granted channel ID |
| drain_beats | output | 16 | Beats available to drain |
| sram_rd_en | input | 1 | SRAM read enable |
| sram_rd_addr | input | AW | SRAM read address |
| sram_rd_data | output | DW | SRAM read data |

### 17.3.4	Per-Channel Status

Table 3.5.5: Per-Channel Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| channel_data_avail | output | NC | Data available per channel |
| channel_empty | output | NC | Channel empty flags |
| channel_full | output | NC | Channel full flags |

## 17.4	Arbitration Logic

### 17.4.1	Figure 3.5.2: Channel Arbitration Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    drain_req[0]   _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______:_______
    drain_req[1]   _______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    drain_req[2]   _______________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______
                    :       :       :       :       :       :
    drain_id       X| CH0  | CH0  | CH1  | CH1  | CH2  | CH2  |XXXXX
    drain_gnt      _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___
                    :       :       :       :       :       :

TODO: Replace with simulation-generated waveform showing round-robin arbitration

## 17.5	Integration with AXI Write Engine

snk_sram_controller #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_snk_sram_ctrl (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Per-channel fill interface
    .snk_fill_alloc_req     (fill_alloc_req),
    .snk_fill_alloc_size    (fill_alloc_size),
    .snk_fill_space_free    (fill_space_free),
    .snk_fill_valid         (fill_valid),
    .snk_fill_ready         (fill_ready),
    .snk_fill_data          (fill_data),
    .snk_fill_last          (fill_last),

    // Arbitrated drain interface
    .drain_req              (sram_drain_req),
    .drain_gnt              (sram_drain_gnt),
    .drain_id               (sram_drain_id),
    .drain_beats            (sram_drain_beats),
    .sram_rd_en             (sram_rd_en),
    .sram_rd_addr           (sram_rd_addr),
    .sram_rd_data           (sram_rd_data),

    // Status
    .channel_data_avail     (channel_data_avail),
    .channel_empty          (channel_empty),
    .channel_full           (channel_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Sink SRAM Controller Unit Specification

Module: snk_sram_controller_unit.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 18.1	Overview

The Sink SRAM Controller Unit manages a single channel’s SRAM buffer with beat-level flow control using virtual FIFOs (alloc_ctrl and drain_ctrl).

### 18.1.1	Key Features

- Single-Channel SRAM: Dedicated SRAM buffer for one channel

- Virtual FIFO Flow Control: beats_alloc_ctrl + beats_drain_ctrl

- Latency Bridge: Compensates for pipeline timing

- Simple SRAM: No reset on memory, only on control logic

### 18.1.2	Block Diagram

### 18.1.3	Figure 3.6.1: Sink SRAM Controller Unit Block Diagram

snk_sram_controller_unit
    +------------------------------------------------------------------+
    |                                                                  |
    |  Fill Interface                                                  |
    |       |                                                          |
    |       v                                                          |
    |  +-----------------+                                             |
    |  | beats_alloc_ctrl|  <- Space allocation                        |
    |  +--------+--------+                                             |
    |           |                                                      |
    |           v                                                      |
    |  +-----------------+     +------------------+                    |
    |  |   SRAM Write    |---->|   simple_sram    |                    |
    |  |     Logic       |     |                  |                    |
    |  +-----------------+     +--------+---------+                    |
    |                                   |                              |
    |                                   v                              |
    |  +-----------------+     +------------------+                    |
    |  |beats_latency_   |<----|   SRAM Read      |                    |
    |  |    bridge       |     |     Logic        |                    |
    |  +--------+--------+     +------------------+                    |
    |           |                                                      |
    |           v                                                      |
    |  +-----------------+                                             |
    |  | beats_drain_ctrl|  <- Data availability                       |
    |  +--------+--------+                                             |
    |           |                                                      |
    +-----------|------------------------------------------------------+
                v
          Drain Interface (to AXI Write Engine)

Source: 03_snk_sram_controller_unit_block.mmd

## 18.2	Parameters

parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);
parameter int LATENCY_BRIDGE_DEPTH = 4;

: Table 3.6.1: Sink SRAM Controller Unit Parameters

## 18.3	Port List

### 18.3.1	Clock and Reset

Table 3.6.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 18.3.2	Fill Interface (from Network)

Table 3.6.3: Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Allocate space |
| fill_alloc_size | input | 16 | Beats to allocate |
| fill_space_free | output | 16 | Available space |
| fill_valid | input | 1 | Fill data valid |
| fill_ready | output | 1 | Ready for fill data |
| fill_data | input | DW | Fill data |
| fill_last | input | 1 | Last beat marker |

### 18.3.3	Drain Interface (to AXI Write Engine)

Table 3.6.4: Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_req | output | 1 | Data available to drain |
| drain_gnt | input | 1 | Drain grant |
| drain_beats | output | 16 | Beats available |
| sram_rd_en | input | 1 | SRAM read enable |
| sram_rd_addr | input | AW | SRAM read address |
| sram_rd_data | output | DW | SRAM read data |

### 18.3.4	Status

Table 3.6.5: Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| data_avail | output | 1 | Data available flag |
| empty | output | 1 | Buffer empty |
| full | output | 1 | Buffer full |

## 18.4	Internal Architecture

### 18.4.1	Figure 3.6.2: Internal Data Flow

Fill Path:

   fill_alloc_req -----> beats_alloc_ctrl -----> wr_ptr management
                              |
   fill_valid/data ---------> | -----> SRAM write
                              |
   fill_ready <-------------- | <----- space_free > 0


Drain Path:

   SRAM read <--------------- | <----- sram_rd_en
        |                     |
        v                     |
   beats_latency_bridge ------+-----> timing compensation
        |                     |
        v                     |
   beats_drain_ctrl ----------+-----> data_avail tracking
        |
        v
   drain_req -----------------> channel arbiter

## 18.5	Timing Diagram

### 18.5.1	Figure 3.6.3: Fill and Drain Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    fill_alloc_req _/‾\___:_______:_______:_______:_______:_______
    fill_alloc_size| 4 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
    fill_space_free|512|508|XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    fill_valid     _______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    fill_data      X:XXXXX|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX:XXXXXXX
    fill_ready     _‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______
                    :       :       :       :       :       :
    (latency)       :       :       :       :       :       :
                    :       :       :       :       :       :
    data_avail     _______________:_______:_/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    drain_req      _______________:_______:_/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    drain_beats    X:XXXXXXX:XXXXXXX:XXXXXXX| 4 |XXXXXXXXX:XXXXXXX

TODO: Replace with simulation-generated waveform showing complete fill-to-drain flow

## 18.6	Virtual FIFO Concept

The controller uses “virtual FIFOs” - pointer-based flow control without duplicating data storage:

### 18.6.1	Figure 3.6.4: Virtual FIFO Operation

SRAM Buffer (actual data storage)
    +-----------------------------------------------------------+
    |  [0]  |  [1]  |  [2]  |  [3]  |  [4]  | ... | [511]      |
    +-----------------------------------------------------------+
         ^                      ^
         |                      |
      rd_ptr                 wr_ptr

    beats_alloc_ctrl: Tracks wr_ptr, manages free space
    beats_drain_ctrl: Tracks rd_ptr, manages data availability

    No data duplication - just pointer arithmetic!

## 18.7	Integration Example

snk_sram_controller_unit #(
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512),
    .LATENCY_BRIDGE_DEPTH(4)
) u_snk_sram_unit (
    .clk                (clk),
    .rst_n              (rst_n),

    // Fill interface
    .fill_alloc_req     (fill_alloc_req[ch]),
    .fill_alloc_size    (fill_alloc_size[ch]),
    .fill_space_free    (fill_space_free[ch]),
    .fill_valid         (fill_valid[ch]),
    .fill_ready         (fill_ready[ch]),
    .fill_data          (fill_data[ch]),
    .fill_last          (fill_last[ch]),

    // Drain interface
    .drain_req          (drain_req[ch]),
    .drain_gnt          (drain_gnt[ch]),
    .drain_beats        (drain_beats[ch]),
    .sram_rd_en         (sram_rd_en[ch]),
    .sram_rd_addr       (sram_rd_addr[ch]),
    .sram_rd_data       (sram_rd_data[ch]),

    // Status
    .data_avail         (data_avail[ch]),
    .empty              (empty[ch]),
    .full               (full[ch])
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Source Data Path Specification

Module: source_data_path.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 19.1	Overview

The Source Data Path integrates the AXI read engine and SRAM controller for memory-to-network transfers. Data is read from system memory via AXI4, buffered in SRAM, and delivered through the drain interface to the network.

### 19.1.1	Key Features

- AXI Read Engine: Streaming burst reads from system memory

- 8-Channel SRAM Controller: Per-channel buffering with flow control

- Beat-Level Tracking: Precise flow control at beat granularity

- Scheduler Integration: Receives transfer requests from scheduler

### 19.1.2	Block Diagram

### 19.1.3	Figure 3.7.1: Source Data Path Block Diagram

source_data_path
    +-------------------------------------------------------+
    |                                                       |
    |    AXI Read Master (from System Memory)               |
    |         |                                             |
    |         v                                             |
    |  +---------------------------------------------+      |
    |  |            axi_read_engine                  |      |
    |  |  - AR channel management                    |      |
    |  |  - R channel streaming                      |      |
    |  |  - Burst generation                         |      |
    |  +------------------+------------------------+        |
    |                     |                                 |
    |                     v                                 |
    |  +---------------------------------------------+      |
    |  |         src_sram_controller                 |      |
    |  |  +-------+  +-------+       +-------+       |      |
    |  |  | unit  |  | unit  |  ...  | unit  |       |      |
    |  |  |  [0]  |  |  [1]  |       |  [7]  |       |      |
    |  |  +-------+  +-------+       +-------+       |      |
    |  +------------------+------------------------+        |
    |                     |                                 |
    |                     v                                 |
    +---------------------|--------------------------------+
                          |
                   Drain Interface
                 (to Network Master)

Source: 03_source_data_path_block.mmd

## 19.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int AR_MAX_OUTSTANDING = 8;
parameter int R_PHASE_FIFO_DEPTH = 64;

: Table 3.7.1: Source Data Path Parameters

## 19.3	Data Flow

### 19.3.1	Figure 3.7.2: Source Path Data Flow Sequence

1. Scheduler Read Request
   - sched_rd_valid = 1
   - sched_rd_addr = source address
   - sched_rd_beats = N
   - sched_rd_id = channel
        |
        v
2. Space Check (beats_alloc_ctrl)
   - Check src_drain_space_free[channel] >= N
   - Allocate N beats of space
        |
        v
3. AXI Read Transaction
   - axi_read_engine issues AR
   - m_axi_araddr = sched_rd_addr
   - m_axi_arlen = calculated from beats
        |
        v
4. R Channel Data
   - Data arrives on m_axi_rdata
   - Written to channel's SRAM partition
        |
        v
5. SRAM Write Complete
   - beats_drain_ctrl notified via latency bridge
   - Data available for drain
        |
        v
6. Drain Interface
   - src_drain_valid = 1
   - Data delivered to network master
        |
        v
7. Completion
   - sched_rd_done_strobe = 1
   - Scheduler notified of completion

Source: 03_source_data_flow.mmd

## 19.4	Port List

### 19.4.1	Clock and Reset

Table 3.7.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 19.4.2	Scheduler Interface

Table 3.7.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | 1 | Read request |
| sched_rd_addr | input | AW | Source address |
| sched_rd_beats | input | 32 | Beats to read |
| sched_rd_id | input | 3 | Channel ID |
| sched_rd_done_strobe | output | 1 | Read complete |
| sched_rd_beats_done | output | 32 | Beats completed |
| sched_rd_error | output | 1 | Error flag |

### 19.4.3	AXI Read Master Interface

Table 3.7.4: AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rlast | input | 1 | Last beat |

### 19.4.4	Drain Interface (Output to Network)

Table 3.7.5: Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_drain_valid | output | 1 | Drain data valid |
| src_drain_ready | input | 1 | Ready for drain data |
| src_drain_data | output | DW | Drain data |
| src_drain_last | output | 1 | Last beat marker |
| src_drain_id | output | 3 | Channel ID |
| src_drain_data_avail | output | NC*16 | Available data per channel |

## 19.5	Timing Diagram

### 19.5.1	Figure 3.7.3: Source Path Transfer Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    sched_rd_valid _/‾‾‾‾‾‾‾\___:_______:_______:_______:_______
    sched_rd_beats X| 4 |XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
    sched_rd_addr  X|ADDR|XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    m_axi_arvalid  _______/‾\___:_______:_______:_______:_______
    m_axi_arlen    X:XXXXX| 3 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    m_axi_rvalid   ___________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______
    m_axi_rdata    X:XXXXXXX|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX
    m_axi_rlast    _________________________/‾\___:_______:_______
                    :       :       :       :       :       :
    src_drain_valid ___________________:_______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    src_drain_data X:XXXXXXX:XXXXXXX:XXXXXXX|=D0=|=D1=|=D2=|=D3=|
                    :       :       :       :       :       :
    sched_rd_done  _________________________________:_______/‾\___

TODO: Replace with simulation-generated waveform showing complete source transfer

## 19.6	Integration Example

source_data_path #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_source_path (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Scheduler interface
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_id            (sched_rd_id),
    .sched_rd_done_strobe   (sched_rd_done_strobe),
    .sched_rd_beats_done    (sched_rd_beats_done),
    .sched_rd_error         (sched_rd_error),

    // AXI read master
    .m_axi_arvalid          (src_axi_arvalid),
    .m_axi_arready          (src_axi_arready),
    .m_axi_araddr           (src_axi_araddr),
    .m_axi_arlen            (src_axi_arlen),
    .m_axi_rvalid           (src_axi_rvalid),
    .m_axi_rready           (src_axi_rready),
    .m_axi_rdata            (src_axi_rdata),
    .m_axi_rlast            (src_axi_rlast),

    // Drain interface
    .src_drain_valid        (src_drain_valid),
    .src_drain_ready        (src_drain_ready),
    .src_drain_data         (src_drain_data),
    .src_drain_last         (src_drain_last),
    .src_drain_id           (src_drain_id),
    .src_drain_data_avail   (src_data_avail)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Source Data Path AXIS Wrapper Specification

Module: source_data_path_axis.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 20.1	Overview

The Source Data Path AXIS wrapper adds an AXI-Stream master interface to the source data path, enabling standard AXIS egress for memory-to-network transfers.

### 20.1.1	Key Features

- AXI-Stream Master Interface: Standard AXIS for data egress

- Per-Channel TID Mapping: Drain channel ID maps to AXIS TID

- TLAST Generation: Packet boundary marking

- Core Source Integration: Wraps source_data_path module

### 20.1.2	Block Diagram

### 20.1.3	Figure 3.8.1: Source Data Path AXIS Block Diagram

source_data_path_axis
    +-------------------------------------------------------+
    |                                                       |
    |    AXI Read Master (from System Memory)               |
    |         |                                             |
    |         v                                             |
    |    +-------------------------------------------+      |
    |    |           source_data_path                |      |
    |    |  (axi_read_engine + src_sram_controller)  |      |
    |    +--------------------+----------------------+      |
    |                         |                             |
    |                         v                             |
    |    +-------------------------------------------+      |
    |    |          Drain to AXIS Converter          |      |
    |    |  - drain_id -> TID                        |      |
    |    |  - drain_last -> TLAST                    |      |
    |    |  - drain handshaking -> TVALID/TREADY    |      |
    |    +--------------------+----------------------+      |
    |                         |                             |
    |                         v                             |
    |    +-------------------------------------------+      |
    |    | m_axis_tvalid   m_axis_tready             |      |
    |    | m_axis_tdata    m_axis_tlast              |      |
    |    | m_axis_tid      m_axis_tkeep              |      |
    |    +-------------------------------------------+      |
    |    AXI-Stream Master Interface                        |
    |                                                       |
    +-------------------------------------------------------+

Source: 03_source_data_path_axis_block.mmd

## 20.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int TID_WIDTH = 3;                     // log2(NUM_CHANNELS)
parameter int TDEST_WIDTH = 1;
parameter int TUSER_WIDTH = 1;

: Table 3.8.1: Source Data Path AXIS Parameters

## 20.3	Port List

### 20.3.1	Clock and Reset

Table 3.8.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 20.3.2	Scheduler Interface

Table 3.8.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | 1 | Read request |
| sched_rd_addr | input | AW | Source address |
| sched_rd_beats | input | 32 | Beats to read |
| sched_rd_id | input | 3 | Channel ID |
| sched_rd_done_strobe | output | 1 | Read complete |
| sched_rd_beats_done | output | 32 | Beats completed |
| sched_rd_error | output | 1 | Error flag |

### 20.3.3	AXI Read Master Interface

Table 3.8.4: AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rlast | input | 1 | Last beat |

### 20.3.4	AXI-Stream Master Interface

Table 3.8.5: AXI-Stream Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tvalid | output | 1 | Data valid |
| m_axis_tready | input | 1 | Ready for data |
| m_axis_tdata | output | DATA_WIDTH | Data payload |
| m_axis_tkeep | output | DATA_WIDTH/8 | Byte enables |
| m_axis_tlast | output | 1 | Last beat of packet |
| m_axis_tid | output | TID_WIDTH | Stream ID (channel) |
| m_axis_tdest | output | TDEST_WIDTH | Destination |
| m_axis_tuser | output | TUSER_WIDTH | User sideband |

## 20.4	Signal Mapping

### 20.4.1	Figure 3.8.2: Drain to AXIS Interface Mapping

Drain Interface               AXIS Master
    +----------------+            +----------------+
    | src_drain_valid|----------->| m_axis_tvalid  |
    | src_drain_ready|<-----------| m_axis_tready  |
    | src_drain_data |----------->| m_axis_tdata   |
    | src_drain_last |----------->| m_axis_tlast   |
    | src_drain_id   |----------->| m_axis_tid     |
    | src_drain_strb |----------->| m_axis_tkeep   |
    +----------------+            +----------------+

Source: 03_source_axis_mapping.mmd

## 20.5	Timing Diagram

### 20.5.1	Figure 3.8.3: AXIS Egress Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    sched_rd_valid _/‾‾‾‾‾‾‾\___:_______:_______:_______:_______
    sched_rd_id    X| CH3 |XXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    (AXI read + SRAM buffering)
                    :       :       :       :       :       :
    m_axis_tvalid  _______________:_/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___
    m_axis_tready  _______________:_/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    m_axis_tdata   X:XXXXXXX:XXXXX|=D0=|=D1=|=D2=|=D3=|XXXXXXX
    m_axis_tid     X:XXXXXXX:XXXXX| CH3| CH3| CH3| CH3|XXXXXXX
    m_axis_tlast   _______________:_______________/‾\___:_______
                    :       :       :       :       :       :
    sched_rd_done  _________________________________:_/‾\___:___

TODO: Replace with simulation-generated waveform showing AXIS packet transmission

## 20.6	Usage Notes

- TID Generation: Drain channel ID directly maps to AXIS TID

- TLAST Generation: Drain last signal maps to AXIS TLAST

- Backpressure: Network TREADY backpressure propagates to SRAM drain

- Packet Boundaries: TLAST marks end of AXIS packets

## 20.7	Integration Example

source_data_path_axis #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .TID_WIDTH(3)
) u_source_axis (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Scheduler interface
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_id            (sched_rd_id),
    .sched_rd_done_strobe   (sched_rd_done_strobe),

    // AXI read master
    .m_axi_arvalid          (src_axi_arvalid),
    .m_axi_arready          (src_axi_arready),
    .m_axi_araddr           (src_axi_araddr),
    .m_axi_rvalid           (src_axi_rvalid),
    .m_axi_rready           (src_axi_rready),
    .m_axi_rdata            (src_axi_rdata),

    // AXIS master
    .m_axis_tvalid          (network_tvalid),
    .m_axis_tready          (network_tready),
    .m_axis_tdata           (network_tdata),
    .m_axis_tlast           (network_tlast),
    .m_axis_tid             (network_tid)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Source SRAM Controller Specification

Module: src_sram_controller.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 21.1	Overview

The Source SRAM Controller manages 8 SRAM controller units for the source data path, providing per-channel buffering with channel arbitration for data delivery to the network.

### 21.1.1	Key Features

- 8-Channel SRAM Array: Instantiates 8 src_sram_controller_unit modules

- Channel Arbitration: Round-robin access for drain requests

- Flow Control Integration: Per-channel alloc_ctrl and drain_ctrl

- Data Availability Tracking: Reports available data per channel

### 21.1.2	Block Diagram

### 21.1.3	Figure 3.9.1: Source SRAM Controller Block Diagram

src_sram_controller
    +------------------------------------------------------------------+
    |                                                                  |
    |    SRAM Write Interface (from AXI Read Engine)                   |
    |         |         |                   |                          |
    |         v         v                   v                          |
    |  +----------+ +----------+     +----------+                      |
    |  |src_sram_ | |src_sram_ | ... |src_sram_ |                      |
    |  |ctrl_unit | |ctrl_unit |     |ctrl_unit |                      |
    |  |   [0]    | |   [1]    |     |   [7]    |                      |
    |  +----+-----+ +----+-----+     +----+-----+                      |
    |       |            |                |                            |
    |       |drain_req   |drain_req       |drain_req                   |
    |       v            v                v                            |
    |  +----------------------------------------------------------+   |
    |  |         Round-Robin Channel Arbiter                      |   |
    |  +----------------------------+-----------------------------+   |
    |                               |                                  |
    |                               v                                  |
    |                    Drain Interface                               |
    |                    (to Network Master)                           |
    |                                                                  |
    +------------------------------------------------------------------+

Source: 03_src_sram_controller_block.mmd

## 21.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;                  // Per-channel depth
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);

: Table 3.9.1: Source SRAM Controller Parameters

## 21.3	Port List

### 21.3.1	Clock and Reset

Table 3.9.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 21.3.2	Per-Channel Fill Interface (from AXI Read Engine)

Table 3.9.3: Per-Channel Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_fill_alloc_req | input | NC | Allocation request per channel |
| src_fill_alloc_size | input | NC*16 | Beats to allocate |
| src_fill_space_free | output | NC*16 | Available space per channel |
| src_fill_valid | input | NC | Fill data valid |
| src_fill_ready | output | NC | Ready for fill data |
| src_fill_data | input | NC*DW | Fill data |
| src_fill_last | input | NC | Last beat marker |

### 21.3.3	Arbitrated Drain Interface (to Network)

Table 3.9.4: Arbitrated Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_valid | output | 1 | Drain data valid |
| drain_ready | input | 1 | Ready for drain |
| drain_data | output | DW | Drain data |
| drain_id | output | 3 | Source channel ID |
| drain_last | output | 1 | Last beat of transfer |

### 21.3.4	Per-Channel Status

Table 3.9.5: Per-Channel Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| channel_data_avail | output | NC*16 | Data available per channel |
| channel_empty | output | NC | Channel empty flags |
| channel_full | output | NC | Channel full flags |

## 21.4	Arbitration Logic

### 21.4.1	Figure 3.9.2: Source Channel Arbitration

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    data_avail[0]  _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    data_avail[1]  _______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______
    data_avail[2]  _______________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___
                    :       :       :       :       :       :
    drain_valid    _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    drain_id       X| CH0 | CH0 | CH1 | CH1 | CH2 | CH2 |XXXXX
    drain_ready    _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾

TODO: Replace with simulation-generated waveform showing round-robin drain

## 21.5	Comparison: Sink vs Source SRAM Controller

Table 3.9.6: Sink vs Source Controller Comparison

| Aspect | Sink Controller | Source Controller |
| --- | --- | --- |
| Fill Source | Network ingress | AXI read data |
| Drain Destination | AXI write engine | Network egress |
| Fill Interface | Per-channel from network | Per-channel from AXI R |
| Drain Interface | Arbitrated SRAM read | Arbitrated drain to network |
| Primary Use | Network -> Memory | Memory -> Network |

## 21.6	Integration Example

src_sram_controller #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_src_sram_ctrl (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Per-channel fill interface (from AXI read engine)
    .src_fill_alloc_req     (fill_alloc_req),
    .src_fill_alloc_size    (fill_alloc_size),
    .src_fill_space_free    (fill_space_free),
    .src_fill_valid         (fill_valid),
    .src_fill_ready         (fill_ready),
    .src_fill_data          (fill_data),
    .src_fill_last          (fill_last),

    // Arbitrated drain interface (to network)
    .drain_valid            (src_drain_valid),
    .drain_ready            (src_drain_ready),
    .drain_data             (src_drain_data),
    .drain_id               (src_drain_id),
    .drain_last             (src_drain_last),

    // Status
    .channel_data_avail     (channel_data_avail),
    .channel_empty          (channel_empty),
    .channel_full           (channel_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Source SRAM Controller Unit Specification

Module: src_sram_controller_unit.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 22.1	Overview

The Source SRAM Controller Unit manages a single channel’s SRAM buffer for the source data path, with beat-level flow control using virtual FIFOs.

### 22.1.1	Key Features

- Single-Channel SRAM: Dedicated SRAM buffer for one channel

- Virtual FIFO Flow Control: beats_alloc_ctrl + beats_drain_ctrl

- Latency Bridge: Compensates for pipeline timing

- Simple SRAM: No reset on memory, only on control logic

### 22.1.2	Block Diagram

### 22.1.3	Figure 3.10.1: Source SRAM Controller Unit Block Diagram

src_sram_controller_unit
    +------------------------------------------------------------------+
    |                                                                  |
    |  Fill Interface (from AXI Read Engine)                           |
    |       |                                                          |
    |       v                                                          |
    |  +-----------------+                                             |
    |  | beats_alloc_ctrl|  <- Space allocation                        |
    |  +--------+--------+                                             |
    |           |                                                      |
    |           v                                                      |
    |  +-----------------+     +------------------+                    |
    |  |   SRAM Write    |---->|   simple_sram    |                    |
    |  |     Logic       |     |                  |                    |
    |  +-----------------+     +--------+---------+                    |
    |                                   |                              |
    |                                   v                              |
    |  +-----------------+     +------------------+                    |
    |  |beats_latency_   |<----|   SRAM Read      |                    |
    |  |    bridge       |     |     Logic        |                    |
    |  +--------+--------+     +------------------+                    |
    |           |                                                      |
    |           v                                                      |
    |  +-----------------+                                             |
    |  | beats_drain_ctrl|  <- Data availability                       |
    |  +--------+--------+                                             |
    |           |                                                      |
    +-----------|------------------------------------------------------+
                v
          Drain Interface (to Network)

Source: 03_src_sram_controller_unit_block.mmd

## 22.2	Parameters

parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);
parameter int LATENCY_BRIDGE_DEPTH = 4;

: Table 3.10.1: Source SRAM Controller Unit Parameters

## 22.3	Port List

### 22.3.1	Clock and Reset

Table 3.10.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 22.3.2	Fill Interface (from AXI Read Engine)

Table 3.10.3: Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Allocate space |
| fill_alloc_size | input | 16 | Beats to allocate |
| fill_space_free | output | 16 | Available space |
| fill_valid | input | 1 | Fill data valid |
| fill_ready | output | 1 | Ready for fill data |
| fill_data | input | DW | Fill data |
| fill_last | input | 1 | Last beat marker |

### 22.3.3	Drain Interface (to Network)

Table 3.10.4: Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_req | output | 1 | Data available to drain |
| drain_gnt | input | 1 | Drain grant from arbiter |
| drain_beats | output | 16 | Beats available |
| drain_valid | output | 1 | Drain data valid |
| drain_ready | input | 1 | Network ready |
| drain_data | output | DW | Drain data |
| drain_last | output | 1 | Last beat marker |

### 22.3.4	Status

Table 3.10.5: Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| data_avail | output | 16 | Data available count |
| empty | output | 1 | Buffer empty |
| full | output | 1 | Buffer full |

## 22.4	Internal Architecture

### 22.4.1	Figure 3.10.2: Internal Data Flow

Fill Path (from AXI Read Engine):

   fill_alloc_req -----> beats_alloc_ctrl -----> wr_ptr management
                              |
   fill_valid/data ---------> | -----> SRAM write
                              |
   fill_ready <-------------- | <----- space_free > 0


Drain Path (to Network):

   SRAM read <--------------- | <----- drain_gnt && drain_ready
        |                     |
        v                     |
   beats_latency_bridge ------+-----> timing compensation
        |                     |
        v                     |
   beats_drain_ctrl ----------+-----> data_avail tracking
        |
        v
   drain_req -----------------> channel arbiter
   drain_valid/data ----------> network interface

## 22.5	Comparison: Sink vs Source Unit

Table 3.10.6: Sink vs Source Unit Comparison

| Aspect | Sink Unit | Source Unit |
| --- | --- | --- |
| Fill Source | Network | AXI Read Engine |
| Drain Destination | AXI Write Engine | Network |
| Fill Handshake | fill_valid/ready | fill_valid/ready |
| Drain Handshake | SRAM read interface | drain_valid/ready |
| Primary Direction | Network -> SRAM -> AXI | AXI -> SRAM -> Network |

## 22.6	Timing Diagram

### 22.6.1	Figure 3.10.3: Source Unit Fill and Drain Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    fill_alloc_req _/‾\___:_______:_______:_______:_______:_______
    fill_alloc_size| 4 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    fill_valid     _______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    fill_data      X:XXXXX|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    (latency bridge)
                    :       :       :       :       :       :
    data_avail     _______________:_______:_| 4  |XXXXXXXXX:XXXXXXX
    drain_req      _______________:_______:_/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    drain_gnt      _______________:_______:_______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    drain_valid    _______________:_______:_______/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
    drain_data     X:XXXXXXX:XXXXXXX:XXXXXXX:XXXXX|=D0=|=D1=|=D2=|

TODO: Replace with simulation-generated waveform showing complete flow

## 22.7	Integration Example

src_sram_controller_unit #(
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512),
    .LATENCY_BRIDGE_DEPTH(4)
) u_src_sram_unit (
    .clk                (clk),
    .rst_n              (rst_n),

    // Fill interface (from AXI read engine)
    .fill_alloc_req     (fill_alloc_req[ch]),
    .fill_alloc_size    (fill_alloc_size[ch]),
    .fill_space_free    (fill_space_free[ch]),
    .fill_valid         (fill_valid[ch]),
    .fill_ready         (fill_ready[ch]),
    .fill_data          (fill_data[ch]),
    .fill_last          (fill_last[ch]),

    // Drain interface (to network)
    .drain_req          (drain_req[ch]),
    .drain_gnt          (drain_gnt[ch]),
    .drain_beats        (drain_beats[ch]),
    .drain_valid        (drain_valid[ch]),
    .drain_ready        (drain_ready[ch]),
    .drain_data         (drain_data[ch]),
    .drain_last         (drain_last[ch]),

    // Status
    .data_avail         (data_avail[ch]),
    .empty              (empty[ch]),
    .full               (full[ch])
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	RAPIDS Core Beats Specification

Module: rapids_core_beats.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 23.1	Overview

The RAPIDS Core Beats module is the top-level integration of the “beats” architecture, combining the scheduler group array with sink and source data paths.

### 23.1.1	Key Features

- Complete RAPIDS Core: Scheduler array + sink path + source path

- 8-Channel Architecture: Full multi-channel support

- Unified MonBus: Aggregated monitoring from all subsystems

- Configurable Parameters: Data width, address width, SRAM depths

### 23.1.2	Block Diagram

### 23.1.3	Figure 3.11.1: RAPIDS Core Beats Block Diagram

rapids_core_beats
    +---------------------------------------------------------------------+
    |                                                                     |
    |  +---------------------------------------------------------------+  |
    |  |            beats_scheduler_group_array                        |  |
    |  |  [8 channels x (scheduler + descriptor_engine)]               |  |
    |  +----------------------------+----------------------------------+  |
    |                               |                                     |
    |            +------------------+------------------+                   |
    |            |                                    |                   |
    |            v                                    v                   |
    |  +-----------------------+        +-----------------------+        |
    |  |   sink_data_path      |        |  source_data_path     |        |
    |  | (or sink_data_path_   |        | (or source_data_path_ |        |
    |  |      axis)            |        |      axis)            |        |
    |  |                       |        |                       |        |
    |  | - snk_sram_controller |        | - src_sram_controller |        |
    |  | - axi_write_engine    |        | - axi_read_engine     |        |
    |  +-----------+-----------+        +-----------+-----------+        |
    |              |                                |                     |
    |              v                                v                     |
    |       AXI Write Master               AXI Read Master               |
    |                                                                     |
    |  MonBus Aggregation:                                               |
    |  +---------------------------------------------------------------+  |
    |  |  Scheduler Array MonBus + Sink Path MonBus + Source Path      |  |
    |  +----------------------------+----------------------------------+  |
    |                               |                                     |
    +-------------------------------|-------------------------------------+
                                    v
                          Unified MonBus Output

Source: 03_rapids_core_beats_block.mmd

## 23.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int DESC_DATA_WIDTH = 256;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;

// AXI Parameters
parameter int AR_MAX_OUTSTANDING = 8;
parameter int AW_MAX_OUTSTANDING = 8;
parameter int R_PHASE_FIFO_DEPTH = 64;
parameter int W_PHASE_FIFO_DEPTH = 64;
parameter int B_PHASE_FIFO_DEPTH = 16;

// MonBus Parameters
parameter int MON_UNIT_ID = 1;

// Feature Enables
parameter bit ENABLE_AXIS_WRAPPERS = 0;          // Use AXIS interfaces

: Table 3.11.1: RAPIDS Core Beats Parameters

## 23.3	Port List

### 23.3.1	Clock and Reset

Table 3.11.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 23.3.2	Per-Channel APB Programming

Table 3.11.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | NC | Channel kick-off (per-channel) |
| apb_ready | output | NC | Ready for kick-off |
| apb_addr | input | NC*AW | First descriptor address |

### 23.3.3	Per-Channel Configuration

Table 3.11.4: Per-Channel Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | NC | Enable per channel |
| cfg_channel_reset | input | NC | Soft reset per channel |
| cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window (cycles) |
| cfg_sched_timeout_limit | input | 8 | Consecutive-timeout windows before fatal escalation (0 = never) |
| cfg_sched_timeout_enable | input | NC | Enable timeout |
| cfg_desceng_enable | input | NC | Enable descriptor engine |
| cfg_desceng_prefetch | input | NC | Enable prefetch chaining |

cfg_sched_timeout_limit is passed straight through to the beats_scheduler_group_array instance (recoverable-timeout escalation, see the Scheduler FUB). The write-side COMMIT strobes (sched_wr_commit_strobe / sched_wr_commit_beats) generated by the sink data path’s axi_write_engine_beats are routed internally from the sink data path up into the scheduler array so completion is gated on B responses.

### 23.3.4	Descriptor AXI Master Interface

Table 3.11.5: Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_m_axi_arvalid | output | 1 | AR channel valid |
| desc_m_axi_arready | input | 1 | AR channel ready |
| desc_m_axi_araddr | output | AW | AR address |
| desc_m_axi_rvalid | input | 1 | R channel valid |
| desc_m_axi_rready | output | 1 | R channel ready |
| desc_m_axi_rdata | input | 256 | R data |

### 23.3.5	Sink AXI Write Master Interface

Table 3.11.6: Sink AXI Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_awvalid | output | 1 | AW channel valid |
| snk_m_axi_awready | input | 1 | AW channel ready |
| snk_m_axi_awaddr | output | AW | Write address |
| snk_m_axi_awlen | output | 8 | Burst length |
| snk_m_axi_wvalid | output | 1 | W channel valid |
| snk_m_axi_wready | input | 1 | W channel ready |
| snk_m_axi_wdata | output | DW | Write data |
| snk_m_axi_wlast | output | 1 | Last beat |
| snk_m_axi_bvalid | input | 1 | B channel valid |
| snk_m_axi_bready | output | 1 | B channel ready |
| snk_m_axi_bresp | input | 2 | Write response |

### 23.3.6	Source AXI Read Master Interface

Table 3.11.7: Source AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_arvalid | output | 1 | AR channel valid |
| src_m_axi_arready | input | 1 | AR channel ready |
| src_m_axi_araddr | output | AW | Read address |
| src_m_axi_arlen | output | 8 | Burst length |
| src_m_axi_rvalid | input | 1 | R channel valid |
| src_m_axi_rready | output | 1 | R channel ready |
| src_m_axi_rdata | input | DW | Read data |
| src_m_axi_rlast | input | 1 | Last beat |

### 23.3.7	Sink Fill Interface (or AXIS Slave)

Table 3.11.8: Sink Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_fill_valid | input | 1 | Fill data valid |
| snk_fill_ready | output | 1 | Ready for fill |
| snk_fill_data | input | DW | Fill data |
| snk_fill_id | input | 3 | Channel ID |
| snk_fill_last | input | 1 | Last beat |

### 23.3.8	Source Drain Interface (or AXIS Master)

Table 3.11.9: Source Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_drain_valid | output | 1 | Drain data valid |
| src_drain_ready | input | 1 | Ready for drain |
| src_drain_data | output | DW | Drain data |
| src_drain_id | output | 3 | Channel ID |
| src_drain_last | output | 1 | Last beat |

### 23.3.9	Unified MonBus Interface

Table 3.11.10: Unified MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | output | 1 | Packet valid |
| monbus_pkt_ready | input | 1 | Consumer ready |
| monbus_pkt_data | output | 64 | Packet data |

### 23.3.10	Aggregate Status

Table 3.11.11: Aggregate Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| all_channels_idle | output | 1 | All channels idle |
| scheduler_idle | output | NC | Per-channel scheduler idle |
| sink_idle | output | 1 | Sink path idle |
| source_idle | output | 1 | Source path idle |
| error_flags | output | 32 | Combined error flags |

## 23.4	Data Flow

### 23.4.1	Figure 3.11.2: RAPIDS Core Complete Data Flow

SOFTWARE
                        |
                        v
            +------------------------+
            |     APB Kick-Off       |
            | (per-channel address)  |
            +------------------------+
                        |
                        v
            +------------------------+
            |  Scheduler Group Array |
            |  - Descriptor fetch    |
            |  - Channel scheduling  |
            +------------------------+
                 /            \
                v              v
    +----------------+    +----------------+
    | Sink Path      |    | Source Path    |
    | (Network->Mem) |    | (Mem->Network) |
    +----------------+    +----------------+
          |                      |
          v                      v
    +----------------+    +----------------+
    | AXI Write      |    | AXI Read       |
    | Master         |    | Master         |
    +----------------+    +----------------+
          |                      |
          v                      v
    +----------------------------------------+
    |           SYSTEM MEMORY                |
    +----------------------------------------+

## 23.5	MonBus Aggregation

The unified MonBus output aggregates sources from all subsystems:

Table 3.11.12: MonBus Source Assignment

| Source Range | Origin | Description |
| --- | --- | --- |
| 0-15 | Scheduler Array | Per-channel scheduler + desc engine |
| 16-23 | Sink Data Path | Sink SRAM controllers |
| 24-31 | Source Data Path | Source SRAM controllers |
| 32 | AXI Write Engine | Write completions |
| 33 | AXI Read Engine | Read completions |

## 23.6	Integration Example

rapids_core_beats #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512),
    .ENABLE_AXIS_WRAPPERS(0)
) u_rapids_core (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // APB kick-off
    .apb_valid              (apb_kick_valid),
    .apb_ready              (apb_kick_ready),
    .apb_addr               (apb_kick_addr),

    // Configuration
    .cfg_channel_enable     (cfg_ch_enable),
    .cfg_sched_timeout_cycles(cfg_timeout),

    // Descriptor AXI
    .desc_m_axi_arvalid     (desc_arvalid),
    .desc_m_axi_arready     (desc_arready),
    .desc_m_axi_araddr      (desc_araddr),
    .desc_m_axi_rvalid      (desc_rvalid),
    .desc_m_axi_rready      (desc_rready),
    .desc_m_axi_rdata       (desc_rdata),

    // Sink AXI write
    .snk_m_axi_awvalid      (snk_awvalid),
    .snk_m_axi_awready      (snk_awready),
    .snk_m_axi_awaddr       (snk_awaddr),
    .snk_m_axi_wvalid       (snk_wvalid),
    .snk_m_axi_wready       (snk_wready),
    .snk_m_axi_wdata        (snk_wdata),
    .snk_m_axi_bvalid       (snk_bvalid),
    .snk_m_axi_bready       (snk_bready),

    // Source AXI read
    .src_m_axi_arvalid      (src_arvalid),
    .src_m_axi_arready      (src_arready),
    .src_m_axi_araddr       (src_araddr),
    .src_m_axi_rvalid       (src_rvalid),
    .src_m_axi_rready       (src_rready),
    .src_m_axi_rdata        (src_rdata),

    // Sink fill interface
    .snk_fill_valid         (network_rx_valid),
    .snk_fill_ready         (network_rx_ready),
    .snk_fill_data          (network_rx_data),
    .snk_fill_id            (network_rx_id),

    // Source drain interface
    .src_drain_valid        (network_tx_valid),
    .src_drain_ready        (network_tx_ready),
    .src_drain_data         (network_tx_data),
    .src_drain_id           (network_tx_id),

    // MonBus
    .monbus_pkt_valid       (mon_valid),
    .monbus_pkt_ready       (mon_ready),
    .monbus_pkt_data        (mon_data),

    // Status
    .all_channels_idle      (rapids_idle)
);

Note: rapids_core_beats is a core-level block. The register interface, configuration mapping, and top-level integration (APB slave, descriptor kick-off, AXI monitors, and the MonBus AXI-Lite group) are provided by the rapids_regs, rapids_config_block, and rapids_beats_top modules – see RAPIDS Registers, RAPIDS Config Block, and RAPIDS Beats Top.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	RAPIDS Registers Specification

Module: rapids_regs.sv (generated) Source: projects/components/rapids/rtl/macro_beats/rapids_regs.rdl, rapids_mon_regs.rdl Generated: projects/components/rapids/regs/generated/rtl/rapids_regs.sv Status: Implemented (PeakRDL-generated)

## 24.1	Overview

rapids_regs is the PeakRDL-generated register block for the RAPIDS Beats top-level. It exposes configuration and status through a single command/response CPU interface and drives the hardware via a hwif_out structure that rapids_config_block maps onto the core/monitor cfg_* signals.

A single addrmap (one APB slave) contains two regfiles:

- Base config/status registers at 0x100-0x3FF.

- A nested rapids_mon_regs monitor regfile at base 0x1000, decoded under hwif_out.MON.* (all DAXMON / RDMON / WRMON / MON_FIFO and per-monitor performance registers).

Because the monitor regfile lives at 0x1000, the register-block address decode requires at least 13 bits (cpuif_addr is a 13-bit compare in the generated RTL). The APB address bus feeding the block (APB_ADDR_WIDTH) must therefore be >= 13 to reach the monitor regfile.

## 24.2	Base Register Map (0x100-0x3FF)

Table 3.12.1: Base Register Map

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | RW | GLOBAL_EN[0], GLOBAL_RST[1] (self-clearing) |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE[0] (all channels idle) |
| 0x108 | VERSION | RO | MINOR[7:0]=0x5A, MAJOR[15:8], NUM_CHANNELS[23:16]=0x08 |
| 0x120 | CHANNEL_ENABLE | RW | CH_EN[7:0] per-channel enable |
| 0x124 | CHANNEL_RESET | RW | CH_RST[7:0] per-channel reset (self-clearing) |
| 0x140 | CHANNEL_IDLE | RO | CH_IDLE[7:0] per-channel idle |
| 0x144 | DESC_ENGINE_IDLE | RO | Per-channel descriptor-engine idle |
| 0x148 | SCHEDULER_IDLE | RO | Per-channel scheduler idle (excludes CH_ERROR) |
| 0x150-0x16C | CH_STATE[0..7].STATE | RO | Per-channel FSM state (stride 0x4) |
| 0x170 | SCHED_ERROR | RO | Per-channel sticky scheduler error |
| 0x174 | AXI_RD_COMPLETE | RO | Per-channel read-complete flags |
| 0x178 | AXI_WR_COMPLETE | RO | Per-channel write-complete flags |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | TIMEOUT_CYCLES[31:0] write-progress window (reset 1000) |
| 0x204 | SCHED_CONFIG | RW | SCHED_EN, TIMEOUT_EN, ERR_EN, COMPL_EN, PERF_EN |
| 0x208 | SCHED_TIMEOUT_LIMIT | RW | LIMIT[7:0] consecutive-timeout escalation limit (reset 4, 0 = never) |
| 0x220 | DESCENG_CONFIG | RW | DESCENG_EN[0], PREFETCH_EN[1], FIFO_THRESH[5:2] (reset 8) |
| 0x224 | DESCENG_ADDR0_BASE | RW | Descriptor address range 0 base [31:0] |
| 0x228 | DESCENG_ADDR0_LIMIT | RW | Descriptor address range 0 limit [31:0] |
| 0x22C | DESCENG_ADDR1_BASE | RW | Descriptor address range 1 base [31:0] |
| 0x230 | DESCENG_ADDR1_LIMIT | RW | Descriptor address range 1 limit [31:0] |
| 0x2A0 | AXI_XFER_CONFIG | RW | AXI transfer sizing configuration |
| 0x2B0 | PERF_CONFIG | RW | Performance profiler configuration |
| 0x2C0 | OBS_CTRL | RW | Observation mux control (channel/category select) |
| 0x2C4 | OBS_FLAGS | RO | Observation flags |
| 0x2C8 | OBS_DATA0 | RO | Observation data word 0 |
| 0x2CC | OBS_DATA1 | RO | Observation data word 1 |
| 0x35C | PERF_CH_SEL | RW | Performance channel select |
| 0x378 | HIST_SEL | RW | Histogram bucket select |
| 0x37C | HIST_DATA | RO | Histogram bucket data |
| 0x380 | HIST_TOTAL | RO | Histogram total count |

### 24.2.1	Key Register Fields

SCHED_TIMEOUT_LIMIT (0x208) – LIMIT[7:0] sets the number of consecutive write-progress timeout windows a channel tolerates before the recoverable timeout escalates to a fatal, sticky CH_ERROR. 0 = never escalate (pure soft timeout). Total time to escalate is approximately LIMIT x TIMEOUT_CYCLES. Reset value is 4.

DESCENG_CONFIG (0x220) – PREFETCH_EN[1] and FIFO_THRESH[5:2] control the now-functional descriptor prefetch: prefetch off = on-demand chaining (~1 ahead), prefetch on = buffer up to FIFO_THRESH descriptors ahead.

## 24.3	Monitor Register Map (rapids_mon_regs @ 0x1000)

The monitor regfile is instantiated at base 0x1000 under hwif_out.MON.*. It holds the MonBus FIFO status, the three AXI-monitor configuration groups (DAXMON = descriptor monitor, RDMON = read monitor, WRMON = write monitor), and per-monitor performance counters.

Table 3.12.2: Monitor Register Map (base 0x1000)

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x1000 | MON.MON_FIFO_STATUS | RO | MonBus capture/error FIFO status |
| 0x1004 | MON.MON_FIFO_COUNT | RO | MonBus FIFO occupancy |
| 0x10C0 | MON.DAXMON_ENABLE | RW | Descriptor-monitor enables (incl. COMPRESS_EN) |
| 0x10C4 | MON.DAXMON_TIMEOUT | RW | Descriptor-monitor timeout cycles |
| 0x10C8 | MON.DAXMON_LATENCY_THRESH | RW | Descriptor-monitor latency threshold |
| 0x10CC | MON.DAXMON_PKT_MASK | RW | Descriptor-monitor packet mask |
| 0x10D0 | MON.DAXMON_ERR_CFG | RW | Descriptor-monitor error select/mask |
| 0x10D4-0x10DC | MON.DAXMON_MASK1/2/3 | RW | Descriptor-monitor category masks |
| 0x10E0-0x10FC | MON.RDMON_* | RW | Read-monitor config (same layout as DAXMON) |
| 0x1100-0x111C | MON.WRMON_* | RW | Write-monitor config (same layout as DAXMON) |
| 0x1150-0x1178 | MON.DAXMON_PERF_* | RO/RW | Descriptor-monitor performance counters |
| 0x1180-0x11A8 | MON.RDMON_PERF_* | RO/RW | Read-monitor performance counters |
| 0x11B0-0x11D8 | MON.WRMON_PERF_* | RO/RW | Write-monitor performance counters |
| 0x11E0-0x11EC | MON.{RD,WR}MON_PERF_CH_* | RO | Per-channel producer/backpressure/starve/idle |
| 0x11F0 | MON.RDMON_PERF_CH_OVERFLOW | RO | Read-monitor per-channel overflow |
| 0x11F4 | MON.WRMON_PERF_CH_OVERFLOW | RO | Write-monitor per-channel overflow |

Each *_ENABLE register carries MON_EN, ERR_EN, COMPL_EN, TIMEOUT_EN (and COMPRESS_EN on the write monitor); each *_PERF_* group provides window, producer, backpressure, starve, idle, beat, byte (lo/hi), and burst counters.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	RAPIDS Config Block Specification

Module: rapids_config_block.sv Location: projects/components/rapids/rtl/macro_beats/ Status: Implemented

## 25.1	Overview

rapids_config_block is a combinational adapter that maps the register block’s hwif_out structure onto the flat cfg_* signals consumed by rapids_core_beats and the AXI monitors. It contains no state; every output is a direct assignment (with a small number of width expansions and global-enable gates).

The monitor registers arrive under hwif_out.MON.*; base configuration and status arrive at the top level of hwif_out.

## 25.2	Mapping Groups

### 25.2.1	Base Configuration

Routes scheduler, channel, and descriptor-engine configuration to the core:

- cfg_channel_enable = CHANNEL_ENABLE.CH_EN & {8{GLOBAL_CTRL.GLOBAL_EN}} (per-channel enable gated by the global master enable).

- cfg_sched_timeout_cycles = SCHED_TIMEOUT_CYCLES.TIMEOUT_CYCLES (32-bit).

- cfg_sched_timeout_limit  = SCHED_TIMEOUT_LIMIT.LIMIT (8-bit, direct).

- cfg_desceng_* from DESCENG_CONFIG (enable / prefetch / FIFO threshold).

- Descriptor address ranges (cfg_desceng_addr{0,1}_{base,limit}) are zero-extended from the 32-bit register fields to the 64-bit ADDR_WIDTH.

### 25.2.2	Monitor Configuration (hwif_out.MON.*)

Three parallel groups map to the descriptor, read, and write AXI monitors:

Table 3.13.1: Monitor Register-to-Config Mapping

| Register group | cfg_* target |
| --- | --- |
| MON.DAXMON_* | cfg_desc_mon_* (descriptor monitor) |
| MON.RDMON_* | cfg_rdeng_mon_* (read monitor) |
| MON.WRMON_* | cfg_wreng_mon_* (write monitor) |

Within each group the enable/timeout/latency/mask fields map 1:1, e.g. cfg_desc_mon_enable = MON.DAXMON_ENABLE.MON_EN & GLOBAL_CTRL.GLOBAL_EN, cfg_desc_mon_timeout_cycles = MON.DAXMON_TIMEOUT.TIMEOUT_CYCLES, and the category masks (timeout/compl/thresh/perf/addr/debug) pass through directly. Key enables are gated by GLOBAL_CTRL.GLOBAL_EN; masks pass through ungated.

### 25.2.3	Performance and Observation

PERF_CONFIG, PERF_CH_SEL, and OBS_CTRL map to the profiler and channel-observation-mux cfg_* signals.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	RAPIDS Beats Top Specification

Module: rapids_beats_top.sv Location: projects/components/rapids/rtl/top_beats/ Status: Implemented

## 26.1	Overview

rapids_beats_top is the synthesizable top-level integration of the RAPIDS Beats accelerator. It wraps the register block, config-block adapter, and core with a single APB slave for all software access, and adds optional AXI transaction monitors whose events are merged with the core’s descriptor-monitor packet and delivered through a MonBus AXI-Lite group (error-drain slave, capture master, and interrupt).

## 26.2	Integration Datapath

### 26.2.1	Figure 3.14.1: RAPIDS Beats Top Integration

s_apb_* (APB4 slave)
        |
     apb_slave  (APB -> cmd/rsp)
        |
   cmdrsp_router (address decode)
     |                         |
     | 0x000-0x03F             | 0x100+ (and 0x1000 MON)
     v                         v
  apbtodescr            peakrdl_to_cmdrsp
  (per-channel           |
   descriptor kickoff)   v
     |               rapids_regs  ---> hwif_out
     |                   |
     |                   v
     |            rapids_config_block  (hwif_out -> cfg_*)
     |                   |
     +---------> rapids_core_beats <---+
                        |
        core AXI rd/wr, descriptor-monitor packet
                        |
   USE_AXI_MONITORS ? insert axi4_master_rd_mon / axi4_master_wr_mon
                        |
     +------------------+------------------+
     | m_axi_rd    m_axi_wr   rd/wr mon packets + core mon packet
     |                        |
     |                   monbus_arbiter (3:1)
     |                        |
     |               monbus_axil_axil_group
     |                 |          |         |
     |          s_axil_err_*  m_axil_mon_*  mon_irq
     v
   (external memory)

### 26.2.2	Address Decode

Table 3.14.1: Top-Level Address Decode

| Range | Target | Purpose |
| --- | --- | --- |
| 0x000-0x03F | apbtodescr | Per-channel descriptor kick-off |
| 0x100-0x3FF | rapids_regs (base) | Configuration / status registers |
| 0x1000+ | rapids_regs (MON regfile) | Monitor configuration / performance |

Because the monitor regfile is at 0x1000, the APB address bus must be at least 13 bits wide (APB_ADDR_WIDTH >= 13) to reach it.

## 26.3	AXI Monitors (USE_AXI_MONITORS)

When USE_AXI_MONITORS = 1, an axi4_master_rd_mon is inserted on the read master (m_axi_rd) and an axi4_master_wr_mon on the write master (m_axi_wr). Their monitor_packet_t outputs are combined with the core’s descriptor-monitor packet (zero-extended to 128 bits) by a 3-input monbus_arbiter (round-robin, with input/output skid buffers). The combined stream feeds monbus_axil_axil_group, which provides:

- s_axil_err_* – AXI-Lite (32-bit) error-drain slave: CPU reads captured error events from the error FIFO.

- m_axil_mon_* – AXI-Lite (64-bit) capture master: bulk-writes the MonBus trace to system memory (base/limit/watermark from cfg_mon_*).

- mon_irq – interrupt on error/threshold events.

When USE_AXI_MONITORS = 0 the monitor taps are bypassed (core AXI passes straight through to m_axi_rd/m_axi_wr), the core MonBus is dropped (always-ready), and the AXI-Lite group outputs are tied off (s_axil_err read-inactive, m_axil_mon write-inactive, mon_irq = 0).

## 26.4	Parameters

parameter int NUM_CHANNELS        = 8;
parameter int DATA_WIDTH          = 512;
parameter int ADDR_WIDTH          = 64;
parameter int AXI_ID_WIDTH        = 8;
parameter int SRAM_DEPTH          = 4096;
parameter int APB_ADDR_WIDTH      = 12;   // must be >= 13 to reach MON regfile @ 0x1000
parameter int APB_DATA_WIDTH      = 32;
parameter bit USE_AXI_MONITORS    = 0;    // 1 = insert rd/wr monitors + MonBus group
parameter int MON_MAX_TRANSACTIONS = 16;
parameter int AR_MAX_OUTSTANDING  = 8;
parameter int AW_MAX_OUTSTANDING  = 8;

: Table 3.14.2: RAPIDS Beats Top Parameters

## 26.5	Top-Level Interfaces

Table 3.14.3: Top-Level Interfaces

| Interface | Signals | Notes |
| --- | --- | --- |
| Clock / reset | aclk, aresetn, cam_clear | Single clock domain; cam_clear for monitor CAMs |
| APB slave | s_apb_paddr (APB_ADDR_WIDTH), s_apb_psel/penable/pwrite/pwdata/pstrb, s_apb_prdata/pready/pslverr | 32-bit data |
| Descriptor AXI (master) | m_axi_desc_ar*, m_axi_desc_r* | 256-bit read data |
| Data read AXI (master) | m_axi_rd_ar*, m_axi_rd_r* | DATA_WIDTH; monitored when enabled |
| Data write AXI (master) | m_axi_wr_aw*, m_axi_wr_w*, m_axi_wr_b* | DATA_WIDTH; monitored when enabled |
| Sink fill | snk_fill_alloc_*, snk_fill_valid/ready/id/data, snk_fill_space_free | Network ingress |
| Source drain | src_drain_data_avail/req/size, src_drain_valid/read/id/data | Network egress |
| MonBus error-drain slave | s_axil_err_ar*, s_axil_err_r* | AXI-Lite, 32-bit read |
| MonBus capture master | m_axil_mon_aw*, m_axil_mon_w* (64-bit wdata), m_axil_mon_b* | AXI-Lite write |
| MonBus interrupt | mon_irq | Error/threshold interrupt |
| MonBus config | cfg_mon_base_addr, cfg_mon_limit_addr, cfg_mon_flush_watermark | Capture region + flush watermark |
| Status | system_idle, sched_error[NC-1:0] | Aggregate status |

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Interface Overview

Last Updated: 2025-01-10

## 27.1	External Interfaces

The RAPIDS Beats architecture exposes the following external interfaces:

### 27.1.1	AXI4 Master Interfaces

Table 4.0.1: AXI4 Master Interfaces

| Interface | Purpose | Data Width |
| --- | --- | --- |
| Descriptor AXI | Fetch descriptors from memory | 256-bit |
| Sink AXI Write | Write sink data to memory | 512-bit (param) |
| Source AXI Read | Read source data from memory | 512-bit (param) |

### 27.1.2	Streaming Interfaces

Table 4.0.2: Streaming Interfaces

| Interface | Type | Purpose |
| --- | --- | --- |
| Fill Interface | Custom | Network data ingress (to SRAM) |
| Drain Interface | Custom | Network data egress (from SRAM) |
| AXIS Sink | AXI-Stream Slave | Optional AXIS wrapper for fill |
| AXIS Source | AXI-Stream Master | Optional AXIS wrapper for drain |

### 27.1.3	Control/Monitor Interfaces

Table 4.0.3: Control and Monitor Interfaces

| Interface | Type | Purpose |
| --- | --- | --- |
| APB | APB slave | Per-channel kick-off |
| Configuration | Custom | Global/per-channel config |
| Status | Custom | Idle, state, error flags |
| MonBus | Custom 64-bit | Monitor packet output |

## 27.2	Interface Documentation

- AXI4 Interface Specification - Descriptor, sink write, source read

- AXIS Interface Specification - Optional streaming wrappers

- MonBus Interface Specification - Monitor packet format

## 27.3	Interface Signal Naming Conventions

Table 4.0.4: Signal Naming Conventions

| Prefix | Meaning | Example |
| --- | --- | --- |
| m_axi_ | AXI master port | m_axi_arvalid |
| s_axi_ | AXI slave port | s_axi_awready |
| m_axis_ | AXIS master port | m_axis_tvalid |
| s_axis_ | AXIS slave port | s_axis_tready |
| cfg_ | Configuration input | cfg_enable |
| snk_ | Sink data path | snk_fill_valid |
| src_ | Source data path | src_drain_ready |
| sched_ | Scheduler interface | sched_rd_valid |
| monbus_ | Monitor bus | monbus_pkt_data |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	AXI4 Interface Specification

Last Updated: 2025-01-10

## 28.1	Overview

The RAPIDS Beats architecture uses three AXI4 master interfaces: 1. Descriptor AXI: Fetches descriptors from memory (256-bit data) 2. Sink AXI Write: Writes sink data to memory (512-bit data, configurable) 3. Source AXI Read: Reads source data from memory (512-bit data, configurable)

## 28.2	Descriptor AXI Master Interface

### 28.2.1	Purpose

Fetches descriptor packets from system memory. Shared across all 8 channels with round-robin arbitration.

### 28.2.2	Signal Table

Table 4.1.1: Descriptor AXI Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_arlen | output | 8 | Burst length (fixed: 0 = 1 beat) |
| m_axi_arsize | output | 3 | Burst size (fixed: 5 = 32 bytes) |
| m_axi_arburst | output | 2 | Burst type (fixed: INCR) |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | 256 | Read data (descriptor) |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rlast | input | 1 | Last beat |

### 28.2.3	Transaction Characteristics

Table 4.1.2: Descriptor AXI Transaction Characteristics

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 256 bits | Fixed descriptor size |
| Burst Length | 1 beat | Single descriptor per transaction |
| Burst Type | INCR | Incrementing burst |
| Max Outstanding | 8 | Configurable via parameter |
| ID Usage | Per-channel | ID encodes source channel |

### 28.2.4	Timing Diagram

### 28.2.5	Figure 4.1.1: Descriptor Fetch Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    m_axi_arvalid  _/‾\___:_______:_______:_______:_______:_______
    m_axi_arready  _/‾\___:_______:_______:_______:_______:_______
    m_axi_araddr   X|DESC_ADDR|XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
    m_axi_arid     X| CH2 |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    m_axi_rvalid   _______:_/‾\___:_______:_______:_______:_______
    m_axi_rready   _______:_/‾\___:_______:_______:_______:_______
    m_axi_rdata    X:XXXXXXX|=DESCRIPTOR=|XXX:XXXXXXX:XXXXXXX
    m_axi_rid      X:XXXXXXX| CH2 |XXX:XXXXXXX:XXXXXXX:XXXXXXX
    m_axi_rlast    _______:_/‾\___:_______:_______:_______:_______

TODO: Replace with simulation-generated waveform

## 28.3	Sink AXI Write Master Interface

### 28.3.1	Purpose

Writes sink data from SRAM to system memory. Supports burst transactions for efficient memory bandwidth.

### 28.3.2	Signal Table

Table 4.1.3: Sink AXI Write Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awvalid | output | 1 | AW channel valid |
| m_axi_awready | input | 1 | AW channel ready |
| m_axi_awaddr | output | AW | Write address |
| m_axi_awid | output | ID_W | Transaction ID |
| m_axi_awlen | output | 8 | Burst length (0-255) |
| m_axi_awsize | output | 3 | Burst size |
| m_axi_awburst | output | 2 | Burst type |
| m_axi_wvalid | output | 1 | W channel valid |
| m_axi_wready | input | 1 | W channel ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat |
| m_axi_bvalid | input | 1 | B channel valid |
| m_axi_bready | output | 1 | B channel ready |
| m_axi_bid | input | ID_W | Response ID |
| m_axi_bresp | input | 2 | Write response |

### 28.3.3	Transaction Characteristics

Table 4.1.4: Sink AXI Write Transaction Characteristics

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 512 bits | Configurable |
| Burst Length | 1-256 beats | Based on transfer size |
| Burst Type | INCR | Incrementing burst |
| Max Outstanding AW | 8 | Configurable |
| W FIFO Depth | 64 | Configurable |
| B FIFO Depth | 16 | Configurable |

### 28.3.4	Timing Diagram

### 28.3.5	Figure 4.1.2: Sink AXI Write Burst Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    m_axi_awvalid  _/‾\___:_______:_______:_______:_______:_______
    m_axi_awready  _/‾\___:_______:_______:_______:_______:_______
    m_axi_awaddr   X|ADDR |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
    m_axi_awlen    X| 3  |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    m_axi_wvalid   _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    m_axi_wready   _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    m_axi_wdata    X|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX:XXXXXXX
    m_axi_wlast    _______________/‾\___:_______:_______:_______
                    :       :       :       :       :       :
    m_axi_bvalid   ___________________:_/‾\___:_______:_______
    m_axi_bready   _____________________/‾\___:_______:_______
    m_axi_bresp    X:XXXXXXX:XXXXXXX:XXXXXXX|OK |XXX:XXXXXXX

TODO: Replace with simulation-generated waveform

## 28.4	Source AXI Read Master Interface

### 28.4.1	Purpose

Reads source data from system memory into SRAM. Supports burst transactions for efficient memory bandwidth.

### 28.4.2	Signal Table

Table 4.1.5: Source AXI Read Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_arlen | output | 8 | Burst length (0-255) |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rlast | input | 1 | Last beat |

### 28.4.3	Transaction Characteristics

Table 4.1.6: Source AXI Read Transaction Characteristics

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 512 bits | Configurable |
| Burst Length | 1-256 beats | Based on transfer size |
| Burst Type | INCR | Incrementing burst |
| Max Outstanding AR | 8 | Configurable |
| R FIFO Depth | 64 | Configurable |

### 28.4.4	Timing Diagram

### 28.4.5	Figure 4.1.3: Source AXI Read Burst Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    m_axi_arvalid  _/‾\___:_______:_______:_______:_______:_______
    m_axi_arready  _/‾\___:_______:_______:_______:_______:_______
    m_axi_araddr   X|ADDR |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
    m_axi_arlen    X| 3  |XXX:XXXXXXX:XXXXXXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
    m_axi_rvalid   ___________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______
    m_axi_rready   ___________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______
    m_axi_rdata    X:XXXXXXX|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX
    m_axi_rlast    _______________________/‾\___:_______:_______

TODO: Replace with simulation-generated waveform

## 28.5	AXI Protocol Compliance

### 28.5.1	Supported Features

Table 4.1.7: AXI Feature Support

| Feature | Descriptor | Sink Write | Source Read |
| --- | --- | --- | --- |
| Burst Type INCR | Yes | Yes | Yes |
| Burst Type FIXED | No | No | No |
| Burst Type WRAP | No | No | No |
| Exclusive Access | No | No | No |
| Locked Access | No | No | No |
| Unaligned Access | No | Yes | Yes |
| Narrow Transfers | No | Yes | Yes |

### 28.5.2	Error Handling

- SLVERR: Transaction aborted, error reported to scheduler

- DECERR: Transaction aborted, error reported to scheduler

- Timeout: Configurable watchdog, error reported to scheduler

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	AXI-Stream Interface Specification

Last Updated: 2025-01-10

## 29.1	Overview

The RAPIDS Beats architecture provides optional AXI-Stream (AXIS) wrappers for network interfaces: 1. AXIS Sink Slave: Receives network data for memory writes 2. AXIS Source Master: Transmits network data from memory reads

These wrappers are available when ENABLE_AXIS_WRAPPERS = 1.

## 29.2	AXIS Sink Slave Interface

### 29.2.1	Purpose

Receives streaming network data and routes to per-channel SRAM buffers based on TID.

### 29.2.2	Signal Table

Table 4.2.1: AXIS Sink Slave Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tvalid | input | 1 | Data valid |
| s_axis_tready | output | 1 | Ready to accept data |
| s_axis_tdata | input | DATA_WIDTH | Data payload |
| s_axis_tkeep | input | DATA_WIDTH/8 | Byte enables |
| s_axis_tlast | input | 1 | Last beat of packet |
| s_axis_tid | input | TID_WIDTH | Stream ID (channel select) |
| s_axis_tdest | input | TDEST_WIDTH | Destination routing |
| s_axis_tuser | input | TUSER_WIDTH | User sideband |

### 29.2.3	Signal Details

#### 29.2.3.1	TID (Transaction ID)

TID is used for per-channel routing:

Table 4.2.2: TID Channel Mapping

| TID Value | Destination |
| --- | --- |
| 0 | Channel 0 SRAM |
| 1 | Channel 1 SRAM |
| 2 | Channel 2 SRAM |
| 3 | Channel 3 SRAM |
| 4 | Channel 4 SRAM |
| 5 | Channel 5 SRAM |
| 6 | Channel 6 SRAM |
| 7 | Channel 7 SRAM |

#### 29.2.3.2	TKEEP (Byte Enables)

TKEEP indicates valid bytes within TDATA:

Table 4.2.3: TKEEP Configuration

| DATA_WIDTH | TKEEP Width | Usage |
| --- | --- | --- |
| 512 bits | 64 bits | Per-byte valid |
| 256 bits | 32 bits | Per-byte valid |

#### 29.2.3.3	TLAST (Packet Boundary)

TLAST marks the end of an AXIS packet. This is mapped to the internal fill_last signal for packet boundary tracking.

### 29.2.4	Timing Diagram

### 29.2.5	Figure 4.2.1: AXIS Sink Slave Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    s_axis_tvalid  _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    s_axis_tready  _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    s_axis_tdata   X|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX:XXXXXXX
    s_axis_tid     X| CH2| CH2| CH2| CH2|XXX:XXXXXXX:XXXXXXX
    s_axis_tkeep   X|0xFF|0xFF|0xFF|0x0F|XXX:XXXXXXX:XXXXXXX
    s_axis_tlast   _______________/‾\___:_______:_______:_______
                    :       :       :       :       :       :

TODO: Replace with simulation-generated waveform

### 29.2.6	Backpressure

The sink interface asserts backpressure (s_axis_tready = 0) when: - Target channel SRAM is full - No space allocated for incoming data - Internal pipeline stalls

## 29.3	AXIS Source Master Interface

### 29.3.1	Purpose

Transmits streaming network data from per-channel SRAM buffers with TID indicating source channel.

### 29.3.2	Signal Table

Table 4.2.4: AXIS Source Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tvalid | output | 1 | Data valid |
| m_axis_tready | input | 1 | Ready to accept data |
| m_axis_tdata | output | DATA_WIDTH | Data payload |
| m_axis_tkeep | output | DATA_WIDTH/8 | Byte enables |
| m_axis_tlast | output | 1 | Last beat of packet |
| m_axis_tid | output | TID_WIDTH | Stream ID (source channel) |
| m_axis_tdest | output | TDEST_WIDTH | Destination routing |
| m_axis_tuser | output | TUSER_WIDTH | User sideband |

### 29.3.3	Timing Diagram

### 29.3.4	Figure 4.2.2: AXIS Source Master Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    m_axis_tvalid  _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    m_axis_tready  _/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___:_______:_______
    m_axis_tdata   X|=D0=|=D1=|=D2=|=D3=|XXX:XXXXXXX:XXXXXXX
    m_axis_tid     X| CH5| CH5| CH5| CH5|XXX:XXXXXXX:XXXXXXX
    m_axis_tkeep   X|0xFF|0xFF|0xFF|0xFF|XXX:XXXXXXX:XXXXXXX
    m_axis_tlast   _______________/‾\___:_______:_______:_______
                    :       :       :       :       :       :

TODO: Replace with simulation-generated waveform

### 29.3.5	Flow Control

The source interface respects network backpressure: - Data held when m_axis_tready = 0 - No data loss on backpressure - SRAM drain pauses until ready

## 29.4	AXIS vs Custom Fill/Drain Interfaces

### 29.4.1	Comparison

Table 4.2.5: Interface Comparison

| Aspect | Custom Fill/Drain | AXIS |
| --- | --- | --- |
| Standard | Proprietary | AXI-Stream |
| Channel ID | fill_id / drain_id | TID |
| Packet Boundary | fill_last / drain_last | TLAST |
| Byte Enables | fill_strb / drain_strb | TKEEP |
| Interoperability | RAPIDS only | Industry standard |

### 29.4.2	Selection Guide

Table 4.2.6: Interface Selection Guide

| Use Case | Recommended Interface |
| --- | --- |
| Internal RAPIDS testing | Custom Fill/Drain |
| Network IP integration | AXIS |
| FPGA board-level design | AXIS |
| Maximum performance | Custom Fill/Drain |

## 29.5	Configuration Parameters

// AXIS Interface Parameters
parameter int DATA_WIDTH = 512;       // Data payload width
parameter int TID_WIDTH = 3;          // Stream ID width (log2 channels)
parameter int TDEST_WIDTH = 1;        // Destination width
parameter int TUSER_WIDTH = 1;        // User sideband width

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	MonBus Interface Specification

Last Updated: 2025-01-10

## 30.1	Overview

The MonBus (Monitor Bus) is a 64-bit packet-based interface for reporting internal events, status, and performance data. All RAPIDS subsystems generate MonBus packets that are aggregated and output through a unified interface.

## 30.2	MonBus Signal Interface

### 30.2.1	Signal Table

Table 4.3.1: MonBus Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | output | 1 | Packet valid |
| monbus_pkt_ready | input | 1 | Consumer ready |
| monbus_pkt_data | output | 64 | Packet data |

### 30.2.2	Handshaking Protocol

Standard valid/ready handshaking: - Data transfers when valid & ready both high - Producer holds valid until ready seen - Consumer asserts ready when able to accept

### 30.2.3	Timing Diagram

### 30.2.4	Figure 4.3.1: MonBus Packet Transfer Timing

____    ____    ____    ____    ____    ____    ____
    clk      |    |__|    |__|    |__|    |__|    |__|    |__|    |__
                    :       :       :       :       :       :
    pkt_valid      _/‾\___/‾‾‾‾‾‾‾\___/‾\___:_______:_______
    pkt_ready      _/‾\___:_______/‾\___/‾\___:_______:_______
    pkt_data       X|PKT1|X|==PKT2==|X|PKT3|XXX:XXXXXXX:XXXXXXX
                    :       :       :       :       :       :
                    ^       ^       ^       ^
                    |       |       |       |
                  Xfer    Wait   Xfer   Xfer

TODO: Replace with simulation-generated waveform

## 30.3	MonBus Packet Format

### 30.3.1	64-bit Packet Structure

63      56 55      48 47      40 39      32 31              0
    +--------+---------+---------+---------+-------------------+
    | Type   | Unit ID | Agent ID| Channel | Payload (32 bits) |
    +--------+---------+---------+---------+-------------------+

### 30.3.2	Figure 4.3.2: MonBus Packet Format

Table 4.3.2: MonBus Packet Fields

| Field | Bits | Description |
| --- | --- | --- |
| Type | [63:56] | Packet type code |
| Unit ID | [55:48] | Reporting unit identifier |
| Agent ID | [47:40] | Source agent within unit |
| Channel | [39:32] | Channel number (0-7) |
| Payload | [31:0] | Type-specific payload |

## 30.4	Packet Types

### 30.4.1	Type Code Definitions

Table 4.3.3: MonBus Packet Types

| Type Code | Name | Description |
| --- | --- | --- |
| 0x01 | DESC_FETCH | Descriptor fetch started |
| 0x02 | DESC_COMPLETE | Descriptor fetch completed |
| 0x03 | SCHED_START | Scheduler started transfer |
| 0x04 | SCHED_DONE | Scheduler transfer complete |
| 0x10 | AXI_RD_START | AXI read burst started |
| 0x11 | AXI_RD_DONE | AXI read burst completed |
| 0x12 | AXI_WR_START | AXI write burst started |
| 0x13 | AXI_WR_DONE | AXI write burst completed |
| 0x20 | SRAM_FILL | SRAM fill event |
| 0x21 | SRAM_DRAIN | SRAM drain event |
| 0x30 | ERROR | Error event |
| 0x31 | TIMEOUT | Timeout event |
| 0xFF | HEARTBEAT | Periodic heartbeat |

### 30.4.2	Payload Formats by Type

#### 30.4.2.1	DESC_FETCH / DESC_COMPLETE (0x01, 0x02)

Table 4.3.4: Descriptor Packet Payload

| Bits | Description |
| --- | --- |
| [31:16] | Descriptor address [47:32] |
| [15:0] | Descriptor address [31:16] |

#### 30.4.2.2	SCHED_START / SCHED_DONE (0x03, 0x04)

Table 4.3.5: Scheduler Packet Payload

| Bits | Description |
| --- | --- |
| [31:16] | Transfer ID |
| [15:0] | Beat count |

#### 30.4.2.3	AXI_RD/WR_START/DONE (0x10-0x13)

Table 4.3.6: AXI Event Packet Payload

| Bits | Description |
| --- | --- |
| [31:24] | AXI ID |
| [23:16] | Burst length |
| [15:0] | Reserved |

#### 30.4.2.4	ERROR (0x30)

Table 4.3.7: Error Packet Payload

| Bits | Description |
| --- | --- |
| [31:24] | Error code |
| [23:16] | Error source |
| [15:0] | Error details |

## 30.5	MonBus Source Aggregation

### 30.5.1	RAPIDS Core MonBus Sources

Table 4.3.8: MonBus Source Assignment

| Source ID | Agent | Description |
| --- | --- | --- |
| 0-7 | Scheduler[0:7] | Per-channel scheduler events |
| 8-15 | DescEngine[0:7] | Per-channel descriptor events |
| 16-23 | SnkSRAM[0:7] | Sink SRAM controller events |
| 24-31 | SrcSRAM[0:7] | Source SRAM controller events |
| 32 | AXI Write Engine | Write transaction events |
| 33 | AXI Read Engine | Read transaction events |
| 34 | Error Handler | Error events |

### 30.5.2	Aggregation Hierarchy

### 30.5.3	Figure 4.3.3: MonBus Aggregation Tree

Level 0 (Sources)           Level 1 (Groups)        Level 2 (Top)
    +----------------+
    | Scheduler[0]   |------+
    | DescEngine[0]  |------+---> scheduler_group[0] ---+
    +----------------+                                   |
    +----------------+                                   |
    | Scheduler[1]   |------+                           |
    | DescEngine[1]  |------+---> scheduler_group[1] ---+
    +----------------+                                   |
         ...                            ...              +---> rapids_core
    +----------------+                                   |     monbus_out
    | Scheduler[7]   |------+                           |
    | DescEngine[7]  |------+---> scheduler_group[7] ---+
    +----------------+                                   |
    +----------------+                                   |
    | SnkSRAM[0:7]   |--------> snk_sram_group --------+
    +----------------+                                   |
    +----------------+                                   |
    | SrcSRAM[0:7]   |--------> src_sram_group --------+
    +----------------+                                   |
    +----------------+                                   |
    | AXI Engines    |--------> axi_engine_group ------+
    +----------------+

## 30.6	Integration Guidelines

### 30.6.1	Consumer Requirements

MonBus consumers must: 1. Provide sufficient ready cycles to prevent packet loss 2. Handle bursty traffic (multiple sources may have packets simultaneously) 3. Process or buffer packets at wire rate

### 30.6.2	Recommended Consumer Patterns

// Pattern 1: Direct FIFO buffering
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_monbus_fifo (
    .i_clk      (clk),
    .i_rst_n    (rst_n),
    .i_valid    (monbus_pkt_valid),
    .i_data     (monbus_pkt_data),
    .o_ready    (monbus_pkt_ready),
    .o_valid    (fifo_valid),
    .o_data     (fifo_data),
    .i_ready    (consumer_ready)
);

// Pattern 2: Software register interface
always_ff @(posedge clk) begin
    if (monbus_pkt_valid && monbus_pkt_ready) begin
        sw_monbus_data <= monbus_pkt_data;
        sw_monbus_valid <= 1'b1;
    end
end
assign monbus_pkt_ready = !sw_monbus_valid || sw_read;

## 30.7	Timing Requirements

Table 4.3.9: MonBus Timing Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Min Ready | 1 cycle | Must assert ready within N cycles |
| Max Latency | Configurable | Before packet dropped |
| Packet Rate | Variable | Depends on activity |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	Product Requirements Document (PRD)

## 31.1	RAPIDS - Rapid AXI Programmable In-band Descriptor System

Version: 1.0 Date: 2025-09-30 Status: Active Development - Validation in Progress Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 31.2	1. Executive Summary

The Rapid AXI Programmable In-band Descriptor System (RAPIDS) is a custom hardware accelerator designed for efficient memory-to-memory data movement with network interface integration. It demonstrates complex FSM coordination, descriptor-based DMA operations, and comprehensive monitoring capabilities.

### 31.2.1	1.1 Quick Stats

- Modules: 17 SystemVerilog files

- Architecture: 3 major blocks (Scheduler, Sink, Source)

- Interfaces: AXI4 (memory), Network (network), AXIL4 (control), MonBus (monitoring)

- Test Coverage: ~80% functional (basic scenarios validated)

- Status: Active validation, known issues documented

### 31.2.2	1.2 Subsystem Goals

- Primary: Demonstrate complex accelerator design patterns

- Secondary: Provide DMA-style memory transfer capability

- Tertiary: Educational reference for descriptor-based engines

## 31.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 31.3.1	📚 Complete RAPIDS Specification

Location: projects/components/rapids/docs/rapids_spec/

- Index - Complete specification structure

#### 31.3.1.1	Chapter 1: Overview

- Overview

- Architecture Requirements

- Clocking and Reset

- Acronyms

- References

#### 31.3.1.2	Chapter 2: Block Specifications

- Blocks Overview

Scheduler Group: - Scheduler Group - Scheduler - Task management FSM - Descriptor Engine - Descriptor parsing - Program Engine - Program sequencing

Sink Data Path (Network → Memory): - Sink Data Path - Network Slave - Network ingress - Sink SRAM Control - Buffer management - Sink AXI Write Engine - Memory writes

Source Data Path (Memory → Network): - Source Data Path - Network Master - Network egress - Source SRAM Control - Buffer management - Source AXI Read Engine - Memory reads

Monitoring: - MonBus AXIL Group - Control/status - FSM Summary - State machine overview

#### 31.3.1.3	Chapter 3: Interfaces

- Top-Level Ports

- AXIL4 Interface

- AXI4 Interface

- Network Interface

- MonBus Interface

#### 31.3.1.4	Chapter 4 & 5: Programming

- Programming Model

- Register Definitions

### 31.3.2	🐛 Known Issues

Location: projects/components/rapids/known_issues/

- Scheduler - Credit counter initialization bug

- Sink Data Path - Minor issues

- Sink SRAM Control - Edge cases

### 31.3.3	📖 Other Documentation

- README - Quick start and integration guide

- CLAUDE - AI assistance guide for this subsystem

- TASKS - Current work items (to be created)

- Validation Report - Test results

## 31.4	2.4 Organizational Standards - RAPIDS Code Location

⚠ MANDATORY: All RAPIDS-specific code must be in the project area ⚠

### 31.4.1	Code Organization Principle

“All RAPIDS-specific verification code MUST reside in projects/components/rapids/dv/ for easy discovery.”

This subsystem follows the repository-wide organizational standard (see /PRD.md Section 2.3) requiring all project-specific code to be located in the project area, NOT the framework area.

### 31.4.2	RAPIDS Directory Structure

projects/components/rapids/
├── rtl/                          # RTL source code
│   ├── includes/                 # RAPIDS packages (rapids_pkg.sv)
│   ├── rapids_fub/               # Functional unit blocks
│   └── rapids_macro/             # Top-level integration
│
└── dv/                           # Design verification (all RAPIDS-specific)
    ├── tbclasses/                # ★ RAPIDS TB classes HERE (not framework!)
    │   ├── scheduler_tb.py       # Scheduler testbench class
    │   ├── descriptor_engine_tb.py
    │   ├── program_engine_tb.py
    │   └── rapids_integration_tb.py
    │
    ├── components/               # ★ RAPIDS-specific BFMs
    │   └── (project-specific components if needed)
    │
    ├── scoreboards/              # ★ RAPIDS-specific scoreboards
    │   └── (verification scoreboards)
    │
    └── tests/                    # Test runners (import TB classes)
        ├── fub_tests/            # Functional unit block tests
        │   ├── scheduler/
        │   │   └── test_scheduler.py
        │   ├── descriptor_engine/
        │   └── program_engine/
        ├── integration_tests/    # Multi-block scenarios
        └── system_tests/         # Full RAPIDS operation

### 31.4.3	What Goes Where?

| Code Type | ✓ CORRECT Location | ✗ WRONG Location |
| --- | --- | --- |
| RAPIDS TB Classes | projects/components/rapids/dv/tbclasses/ | bin/TBClasses/rapids/ |
| RAPIDS-Specific BFMs | projects/components/rapids/dv/components/ | bin/TBClasses/components/rapids/ |
| RAPIDS Scoreboards | projects/components/rapids/dv/scoreboards/ | bin/TBClasses/scoreboards/rapids/ |
| Test Runners | projects/components/rapids/dv/tests/ | Anywhere else |
| Shared AXI4/APB BFMs | bin/TBClasses/components/{protocol}/ | Project area |

### 31.4.4	Import Pattern for RAPIDS Tests

✓ CORRECT - Import from Project Area:

# Import framework utilities (PYTHONPATH includes bin/)
import os, sys
from TBClasses.shared.utilities import get_repo_root
from TBClasses.shared.tbbase import TBBase

# Add repo root to Python path using robust git-based method
repo_root = get_repo_root()
sys.path.insert(0, repo_root)

# Import RAPIDS TB classes from PROJECT AREA
from projects.components.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from projects.components.rapids.dv.tbclasses.descriptor_engine_tb import DescriptorEngineTB

# Shared framework components
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

✗ WRONG - Don’t Import from Framework:

# DON'T DO THIS!
from TBClasses.rapids.scheduler_tb import SchedulerTB  # ✗ WRONG!

### 31.4.5	Benefits of This Organization

- Easy Discovery - All RAPIDS code in ONE place: projects/components/rapids/

- Clear Ownership - RAPIDS team owns their dv/ area completely

- No Confusion - Never wonder “where does this TB class live?”

- Maintainability - Changes isolated to RAPIDS area don’t affect other projects

- Framework Stays Clean - Only truly shared cross-project code in framework

### 31.4.6	Compliance Status

✓ RAPIDS is now compliant - All TB classes moved to project area as of 2025-10-18

Migration History: - Before: TB classes incorrectly in bin/TBClasses/rapids/ - After: TB classes correctly in projects/components/rapids/dv/tbclasses/ - Test Imports: Updated to import from project area

📖 Complete Documentation: See /PRD.md Section 2.3 for repository-wide organizational standards.

## 31.5	3. Architecture Overview

### 31.5.1	3.1 Top-Level Block Diagram

RAPIDS (Rapid AXI Programmable In-band Descriptor System)
├── Scheduler Group
│   ├── Scheduler          (Task FSM, credit management)
│   ├── Descriptor Engine  (Descriptor FIFO, parsing)
│   └── Program Engine     (Program sequencing, alignment)
│
├── Sink Data Path (Network → SRAM → System Memory)
│   ├── Network Slave        (Network ingress interface)
│   ├── Sink SRAM Control (Buffering, flow control)
│   └── Sink AXI Writer   (AXI4 write to system memory)
│
├── Source Data Path (System Memory → SRAM → Network)
│   ├── Source AXI Reader (AXI4 read from system memory)
│   ├── Source SRAM Control (Buffering, flow control)
│   └── Network Master       (Network egress interface)
│
└── MonBus AXIL Group
    ├── AXIL4 Slave       (Control/status registers)
    └── MonBus Reporter   (Monitor packet generation)

📖 See: rapids_spec/ch02_blocks/00_overview.md for detailed architecture

### 31.5.2	3.2 Data Flow

Sink Path (Receive): 1. Network packets arrive via Network Slave 2. Buffered in Sink SRAM 3. DMA’d to system memory via AXI4 Write Engine 4. Completion reported via MonBus

Source Path (Transmit): 1. Descriptor specifies data location in system memory 2. Source AXI Reader fetches data to Source SRAM 3. Network Master transmits to network 4. Completion reported via MonBus

Scheduler Coordination: - Parses descriptors from Descriptor Engine - Manages credit-based flow control - Sequences program engine operations - Coordinates sink/source data paths

## 31.6	4. Key Features

### 31.6.1	4.1 Descriptor-Based Operation

| Feature | Status | Description |
| --- | --- | --- |
| Descriptor FIFO | ✓ | Queued descriptor processing |
| Multi-field parsing | ✓ | Address, length, control fields |
| Chained descriptors | ⏳ | Future enhancement |
| Completion reporting | ✓ | Via MonBus packets |

### 31.6.2	4.2 Data Path Features

| Feature | Status | Description |
| --- | --- | --- |
| SRAM buffering | ✓ | Decouple network from memory |
| AXI4 burst support | ✓ | Efficient memory transfers |
| Backpressure handling | ✓ | Flow control on all interfaces |
| Data alignment | ✓ | Handle unaligned transfers |

### 31.6.3	4.3 Scheduler Features

| Feature | Status | Description |
| --- | --- | --- |
| Task FSM | ✓ | Multi-state coordination |
| Credit management | ✓ | Exponential encoding (0→1, 1→2, 2→4, …, 15→∞) |
| Program sequencing | ✓ | Coordinated operations |
| Error detection | ✓ | Timeout, overflow detection |

Credit Management Details: - Uses exponential credit encoding for compact configuration - 4-bit cfg_initial_credit decodes to actual credit counts: - 0 → 1 credit (2^0), 4 → 16 credits (2^4), 8 → 256 credits (2^8) - 15 → ∞ (unlimited credits, 0xFFFFFFFF) - Encoding applied at initialization; runtime operations are linear (increment/decrement by 1) - Provides wide range (1 to 16384) with minimal configuration overhead

📖 See: rapids_spec/ch02_blocks/01_01_scheduler.md for complete encoding table

### 31.6.4	4.4 Monitoring Integration

| Feature | Status | Description |
| --- | --- | --- |
| MonBus packets | ✓ | StandardAMBA 64-bit format |
| Descriptor events | ✓ | Start/complete reporting |
| Error events | ✓ | Timeout, overflow, underflow |
| Performance metrics | ⏳ | Future enhancement |

## 31.7	5. Interfaces

### 31.7.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXIL4 | Slave | 32-bit | Control/status registers |
| AXI4 (Sink) | Master | Configurable | Write to system memory |
| AXI4 (Source) | Master | Configurable | Read from system memory |
| Network (Sink) | Slave | Configurable | Network ingress |
| Network (Source) | Master | Configurable | Network egress |
| MonBus | Master | 64-bit | Monitor packet output |

📖 See: rapids_spec/ch03_interfaces/ for complete interface specs

### 31.7.2	5.2 Configuration Parameters

// Example RAPIDS instantiation parameters
rapids_top #(
    .AXI_ADDR_WIDTH(32),
    .AXI_DATA_WIDTH(64),
    .Network_DATA_WIDTH(64),
    .SRAM_DEPTH(1024),
    .MAX_DESCRIPTORS(16)
) u_rapids (
    .aclk               (clk),
    .aresetn            (rst_n),
    // AXIL4 control interface
    .s_axil_*           (...),
    // AXI4 memory interfaces
    .m_axi_sink_*       (...),
    .m_axi_source_*     (...),
    // Network network interfaces
    .s_network_*           (...),
    .m_network_*           (...),
    // MonBus output
    .monbus_pkt_*       (...)
);

## 31.8	6. Use Cases

### 31.8.1	6.1 DMA-Style Transfers

Scenario: Move data from network to system memory

Flow: 1. Software writes descriptor to Descriptor Engine 2. Scheduler parses descriptor, activates Sink path 3. Network packets arrive via Network Slave 4. Data buffered in Sink SRAM 5. AXI4 Write Engine DMAs to system memory 6. Completion packet on MonBus

### 31.8.2	6.2 Network Packet Processing

Scenario: Read data from memory, transmit to network

Flow: 1. Descriptor specifies source address, length 2. Source AXI Reader fetches data to SRAM 3. Network Master transmits to network 4. Completion/error reporting via MonBus

### 31.8.3	6.3 Custom Data Path Acceleration

Educational value: Shows how to build custom accelerators - Descriptor-based control - Multi-block FSM coordination - Buffering strategies - Error handling - Performance monitoring

## 31.9	7. Test Coverage

### 31.9.1	7.1 Current Status

Overall: ~85% functional coverage (basic scenarios validated, descriptor engine complete)

| Component | Test Coverage | Status |
| --- | --- | --- |
| Scheduler | ~95% | Credit encoding fixed and verified (43/43 tests passing) |
| Descriptor Engine | ✓ 100% | All tests passing (14/14 tests, 100% success rate) |
| Program Engine | ~85% | Alignment tested |
| Sink Data Path | ~75% | Basic flows working |
| Source Data Path | ~70% | Basic flows working |
| SRAM Controllers | ~80% | Buffer management tested |
| Integration | ~60% | More stress testing needed |

Test Location: projects/components/rapids/dv/tests/fub_tests/ and projects/components/rapids/dv/tests/integration_tests/

Recent Achievements: - ✓ Descriptor Engine (2025-10-13): Achieved 100% test pass rate using continuous background monitoring pattern - 14/14 tests passing across all test levels (basic, medium, full) - All test classes passing (APB_ONLY, MIXED) - All delay profiles passing (fast_producer, fast_consumer, fixed_delay, minimal_delay) - Applied continuous monitoring methodology for asynchronous output capture

📖 See: docs/RAPIDS_Validation_Status_Report.md for detailed results

### 31.9.2	7.2 Test Strategy

FUB (Functional Unit Block) Tests: - Individual block testing - Located in projects/components/rapids/dv/tests/fub_tests/ - Focus on module-level functionality

Integration Tests: - Multi-block scenarios - Located in projects/components/rapids/dv/tests/integration_tests/ - End-to-end data flow validation

System Tests: - Full RAPIDS operation - Located in projects/components/rapids/dv/tests/system_tests/ - Realistic traffic patterns

## 31.10	8. Known Issues Summary

### 31.10.1	8.1 Critical Issues

✓ Scheduler Credit Counter Initialization - FIXED (2025-10-11) - File: projects/components/rapids/rtl/rapids_fub/scheduler.sv:567-570 - Issue: Credit counter was initializing to 0 instead of using exponential encoding - Fix Applied: Implemented exponential credit encoding - Status: Fixed, awaiting test verification - Documentation: known_issues/scheduler.md

Fix Details:

// Previous (wrong):
r_descriptor_credit_counter <= 32'h0;

// Fixed - Exponential encoding:
// 0→1, 1→2, 2→4, 3→8, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Encoding Rationale: - Compact 4-bit configuration covers 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞) - Exponential encoding applied at initialization only; runtime operations are linear

### 31.10.2	8.2 Medium Priority Issues

Descriptor Engine Edge Cases: - Some stress test failures under high load - Edge case handling needs improvement - Priority: P2

SRAM Control Timing: - Rare timing issues in back-to-back operations - Priority: P2

📖 See: known_issues/ directory for complete issue tracking

## 31.11	9. Integration Guidelines

### 31.11.1	9.1 Quick Start

rapids_top #(
    .AXI_ADDR_WIDTH(32),
    .AXI_DATA_WIDTH(64),
    .Network_DATA_WIDTH(64)
) u_rapids (
    // Clocking & Reset
    .aclk               (system_clk),
    .aresetn            (system_rst_n),

    // AXIL4 Control (connect to control fabric)
    .s_axil_awaddr      (ctrl_awaddr),
    .s_axil_awvalid     (ctrl_awvalid),
    .s_axil_awready     (ctrl_awready),
    // ... other AXIL signals

    // AXI4 Memory (connect to memory controller)
    .m_axi_sink_awaddr  (mem_awaddr),
    .m_axi_sink_awvalid (mem_awvalid),
    // ... AXI write channel for sink
    // ... AXI read channel for source

    // Network Network (connect to network fabric)
    .s_network_tdata       (net_rx_data),
    .s_network_tvalid      (net_rx_valid),
    // ... Network slave (receive)
    // ... Network master (transmit)

    // MonBus Output (connect to monitor aggregator)
    .monbus_pkt_valid   (rapids_mon_valid),
    .monbus_pkt_ready   (rapids_mon_ready),
    .monbus_pkt_data    (rapids_mon_data)
);

### 31.11.2	9.2 Configuration Steps

- Initialize via AXIL4:

  - Configure SRAM depths

  - Set timeout thresholds

  - Enable credit management (or disable if using workaround)

- Load Descriptors:

  - Write descriptors to Descriptor Engine FIFO

  - Each descriptor specifies: address, length, control bits

- Enable Operation:

  - Set enable bits via AXIL4 registers

  - Monitor MonBus for completion/error packets

📖 See: rapids_spec/ch04_programming_models/01_programming.md for register details

## 31.12	10. Development Status

### 31.12.1	10.1 Current Phase

Phase: Validation and Bug Fixing (In Progress)

- ✓ Core architecture implemented

- ✓ Basic functionality verified

- ✓ Scheduler credit counter bug fixed (exponential encoding implemented)

- ⏳ Credit management tests need verification (remove workarounds)

- ⏳ Stress testing ongoing

- ⏳ Edge case refinement

📖 See: TASKS.md for detailed work items

### 31.12.2	10.2 Roadmap

Near-Term (Q4 2025): - ✓ Fix scheduler credit counter bug (completed 2025-10-11) - ⏳ Verify credit management tests (remove workarounds) - ⏳ Complete descriptor engine stress testing - ⏳ Integration test suite expansion - ⏳ Performance benchmarking

Long-Term (2026+): - Chained descriptor support - Advanced error recovery - Performance optimizations - Multi-channel support

## 31.13	11. Performance Characteristics

### 31.13.1	11.1 Throughput

Target: Match network/memory interface bandwidth

Bottlenecks: - SRAM buffer size - AXI4 burst efficiency - Scheduler overhead

Optimization: - Increase SRAM depth for larger packets - Tune AXI4 burst parameters - Pipeline scheduler operations

### 31.13.2	11.2 Latency

Components: - Descriptor parsing: ~10 cycles - SRAM buffering: Configurable depth - AXI4 memory access: System dependent - End-to-end: Typically <100 cycles for small packets

### 31.13.3	11.3 Resource Utilization

Area: - Scheduler: ~2K LUTs - Each data path: ~3K LUTs - SRAM buffers: Configurable (dominant area) - Total: ~10K LUTs + SRAM

Power: - Clock gating opportunities in idle blocks - SRAM power depends on depth/width

## 31.14	12. Verification Infrastructure

### 31.14.1	12.1 Test Organization

Location: projects/components/rapids/dv/tests/

Structure:

projects/components/rapids/dv/tests/
├── fub_tests/              # Functional Unit Block tests
│   ├── scheduler/
│   ├── descriptor_engine/
│   ├── program_engine/
│   ├── network_interfaces/
│   └── simple_sram/
├── integration_tests/      # Multi-block scenarios
└── system_tests/           # Full RAPIDS operation

### 31.14.2	12.2 CocoTB Framework

Location: bin/TBClasses/rapids/

Components: - RAPIDS-specific drivers - Descriptor generators - Traffic patterns - Monitor checkers

📖 See: docs/markdown/TBClasses/ (once created)

### 31.14.3	12.2.1 MANDATORY: BFM Usage for FUB Tests

⚠ CRITICAL DESIGN REQUIREMENT ⚠

All RAPIDS FUB (Functional Unit Block) level tests MUST use CocoTB Framework BFMs. Manual handshake driving is NOT allowed.

Required BFM Components:

| Interface Type | Framework Location | BFM Component |
| --- | --- | --- |
| Custom valid/ready | bin/TBClasses/components/gaxi/ | GAXI Master/Slave |
| AXI4 | bin/TBClasses/components/axi4/ | AXI4 Master/Slave |
| AXI4-Lite (AXIL) | bin/TBClasses/components/axil4/ | AXIL Master/Slave |
| APB | bin/TBClasses/components/apb/ | APB Master/Slave |
| AXI-Stream (AXIS) | bin/TBClasses/components/axis4/ | AXIS Master/Slave |
| Network | bin/TBClasses/components/network/ | Network Master/Slave |
| MonBus | bin/TBClasses/components/monbus/ | MonBus drivers |

Rationale: 1. Consistency: All tests use standardized handshake protocols 2. Correctness: BFMs handle complex timing scenarios (backpressure, randomization) 3. Reusability: Same BFM across all RAPIDS tests 4. Maintainability: Fix once in BFM, all tests benefit 5. Coverage: BFMs include comprehensive timing profiles

Example - Program Engine:

# ✗ WRONG: Manual handshake (violates design requirement)
async def send_request(self, addr, data):
    self.dut.program_valid.value = 1
    self.dut.program_pkt_addr.value = addr
    # ... manual handshaking logic ...

# ✓ CORRECT: Use GAXI Master BFM
from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster

class ProgramEngineTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.program_master = GAXIMaster(
            dut=dut,
            clock=dut.clk,
            valid_signal='program_valid',
            ready_signal='program_ready',
            data_signals=['program_pkt_addr', 'program_pkt_data'],
            data_widths=[64, 32]
        )

    async def send_request(self, addr, data):
        await self.program_master.write({'program_pkt_addr': addr, 'program_pkt_data': data})

📖 See: - projects/components/rapids/CLAUDE.md - Rule #1 for complete BFM usage guidelines - bin/TBClasses/components/gaxi/README.md - GAXI BFM documentation - bin/TBClasses/components/axi4/README.md - AXI4 BFM documentation

### 31.14.4	12.3 Test File Structure (Standard Pattern)

⚠ MANDATORY: All RAPIDS tests must follow this structure ⚠

RAPIDS tests follow the same pattern as AMBA tests for consistency across the repository:

# Example: projects/components/rapids/dv/tests/fub_tests/scheduler/test_scheduler.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# Import REUSABLE testbench class (NOT defined in this file!)
from TBClasses.rapids.scheduler_tb import SchedulerTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic_flow(dut):
    """Test basic descriptor flow."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_basic_descriptor_flow()
    assert result, "Basic descriptor flow test failed"

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_credit_encoding(dut):
    """Test exponential credit encoding."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_exponential_encoding_all_values()
    assert result, "Credit encoding test failed"

# ===========================================================================
# PARAMETER GENERATION - at bottom of file
# ===========================================================================

def generate_scheduler_test_params():
    """Generate test parameters for scheduler tests."""
    return [
        # (channel_id, num_channels, data_width, credit_width)
        (0, 8, 512, 8),  # Standard configuration
        # Add more parameter sets as needed
    ]

scheduler_params = generate_scheduler_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - at bottom of file
# ===========================================================================

@pytest.mark.parametrize("channel_id, num_channels, data_width, credit_width", scheduler_params)
def test_basic_flow(request, channel_id, num_channels, data_width, credit_width):
    """
    Scheduler basic flow test.

    Run with: pytest projects/components/rapids/dv/tests/fub_tests/scheduler/test_scheduler.py::test_basic_flow -v
    """
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_rapids_fub': '../../rtl/rapids_fub'
    })

    dut_name = "scheduler"
    toplevel = dut_name

    verilog_sources = [
        os.path.join(repo_root, 'rtl', 'amba', 'includes', 'monitor_pkg.sv'),
        os.path.join(repo_root, 'rtl', 'rapids', 'includes', 'rapids_pkg.sv'),
        os.path.join(rtl_dict['rtl_rapids_fub'], f'{dut_name}.sv'),
    ]

    # Format parameters for unique test name
    cid_str = TBBase.format_dec(channel_id, 2)
    nc_str = TBBase.format_dec(num_channels, 2)
    dw_str = TBBase.format_dec(data_width, 4)
    cw_str = TBBase.format_dec(credit_width, 2)
    test_name_plus_params = f"test_{dut_name}_cid{cid_str}_nc{nc_str}_dw{dw_str}_cw{cw_str}"

    # Add worker ID for pytest-xdist parallel execution
    worker_id = os.environ.get('PYTEST_XDIST_WORKER', '')
    if worker_id:
        test_name_plus_params = f"{test_name_plus_params}_{worker_id}"

    log_path = os.path.join(log_dir, f'{test_name_plus_params}.log')
    sim_build = os.path.join(tests_dir, 'local_sim_build', test_name_plus_params)
    os.makedirs(sim_build, exist_ok=True)
    os.makedirs(log_dir, exist_ok=True)

    rtl_parameters = {
        'CHANNEL_ID': channel_id,
        'NUM_CHANNELS': num_channels,
        'DATA_WIDTH': data_width,
        'CREDIT_WIDTH': credit_width,
        # Add other RTL parameters as needed
    }

    extra_env = {
        'LOG_PATH': log_path,
        'TEST_CHANNEL_ID': str(channel_id),
        'TEST_NUM_CHANNELS': str(num_channels),
        'TEST_DATA_WIDTH': str(data_width),
    }

    compile_args = ["-Wno-TIMESCALEMOD"]
    sim_args = []
    plusargs = []

    cmd_filename = create_view_cmd(log_dir, log_path, sim_build, module, test_name_plus_params)

    try:
        run(
            python_search=[tests_dir],
            verilog_sources=verilog_sources,
            includes=[
                os.path.join(repo_root, 'rtl', 'rapids', 'includes'),
                os.path.join(repo_root, 'rtl', 'amba', 'includes'),
            ],
            toplevel=toplevel,
            module=module,
            testcase="cocotb_test_basic_flow",  # ← cocotb test function name
            parameters=rtl_parameters,
            sim_build=sim_build,
            extra_env=extra_env,
            waves=False,
            keep_files=True,
            compile_args=compile_args,
            sim_args=sim_args,
            plusargs=plusargs,
        )

        print(f"✓ Scheduler basic flow test completed!")
        print(f"Logs: {log_path}")

    except Exception as e:
        print(f"✗ Scheduler basic flow test failed: {str(e)}")
        print(f"Logs preserved at: {log_path}")
        raise

Key Structure Requirements:

- Testbench Class Location:

  - ALWAYS in bin/TBClasses/rapids/

  - NEVER inline in test file

  - Reusable across multiple test files

- CocoTB Test Functions:

  - Prefix with cocotb_ to prevent pytest collection

  - Located at top of test file

  - Use @cocotb.test() decorator

  - Call testbench methods

- Parameter Generation:

  - Function returns list of parameter tuples

  - Located near bottom of file (before pytest wrappers)

  - Stored in variable (e.g., scheduler_params)

- Pytest Wrapper Functions:

  - Located at bottom of file

  - Use @pytest.mark.parametrize() with parameter variable

  - Build unique test names with TBBase.format_dec()

  - Call run() with testcase="cocotb_test_function_name"

  - Handle parallel execution (PYTEST_XDIST_WORKER)

- Mandatory TB Methods:

  - async def setup_clocks_and_reset(self) - Complete initialization

  - async def assert_reset(self) - Assert reset signal(s)

  - async def deassert_reset(self) - Deassert reset signal(s)

📖 See: - val/amba/test_apb_slave.py - Reference example - projects/components/rapids/CLAUDE.md - Detailed TB requirements

## 31.15	13. Quick Reference

### 31.15.1	13.1 Key Files

| File | Purpose |
| --- | --- |
| projects/components/rapids/PRD.md | This document (overview) |
| projects/components/rapids/README.md | Quick start guide |
| projects/components/rapids/CLAUDE.md | AI assistance guide |
| projects/components/rapids/TASKS.md | Work items (to be created) |
| projects/components/rapids/docs/rapids_spec/ | Complete specification |
| projects/components/rapids/known_issues/ | Bug tracking |
| docs/RAPIDS_Validation_Status_Report.md | Test results |

### 31.15.2	13.2 Commands

# Run RAPIDS tests
pytest projects/components/rapids/dv/tests/fub_tests/ -v          # Individual blocks
pytest projects/components/rapids/dv/tests/integration_tests/ -v   # Multi-block
pytest projects/components/rapids/dv/tests/system_tests/ -v        # Full system

# Run specific FUB test
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ -v

# Lint RAPIDS RTL
verilator --lint-only projects/components/rapids/rtl/rapids_fub/scheduler.sv

# View specifications
cat projects/components/rapids/docs/rapids_spec/rapids_index.md
cat projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md

## 31.16	14. Success Criteria

### 31.16.1	14.1 Functional

- ✓ All major blocks implemented

- ✓ Basic data flows working

- ✓ Scheduler credit bug fixed (exponential encoding implemented)

- ⏳ Credit management tests verified (remove workarounds, run full suite)

- ⏳ 100% descriptor test pass rate (currently ~80%)

- ⏳ Stress tests passing

### 31.16.2	14.2 Quality

- ⏳ >90% functional coverage (currently ~80%)

- ⏳ >85% code coverage

- ✓ All FSMs documented

- ⏳ Integration guide complete

### 31.16.3	14.3 Documentation

- ✓ Complete specification in rapids_spec/

- ✓ Known issues documented

- ⏳ Integration examples

- ⏳ Performance characterization

## 31.17	15. Educational Value

RAPIDS demonstrates: - ✓ Complex FSM coordination (scheduler ↔ data paths) - ✓ Descriptor-based DMA design patterns - ✓ Buffer management strategies - ✓ Credit-based flow control with exponential encoding - ✓ Multi-interface integration - ✓ Comprehensive monitoring - ✓ Error detection and reporting - ✓ Compact configuration encoding strategies

Target Audience: - Advanced RTL designers - Accelerator architects - DMA engine developers - System integration engineers

## 31.18	15. Attribution and Contribution Guidelines

### 31.18.1	15.1 Git Commit Attribution

When creating git commits for RAPIDS documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: RAPIDS documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 31.19	16. Documentation Generation

### 31.19.1	16.1 Generating PDF/DOCX from Specification

Tool: /mnt/data/github/rtldesignsherpa/bin/md_to_docx.py

Use this tool to convert the linked specification index into a single all-inclusive PDF or DOCX file.

Basic Usage:

# Generate DOCX from rapids_spec index
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc \
    --title-page

# Generate both DOCX and PDF
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc \
    --title-page \
    --pdf

# With custom template (optional)
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    -t path/to/template.dotx \
    --toc \
    --title-page \
    --pdf

Key Features: - Recursive Collection: Follows all markdown links in the index file - Heading Demotion: Automatically adjusts heading levels for included files - Table of Contents: --toc flag generates automatic ToC - Title Page: --title-page flag creates title page from first heading - PDF Export: --pdf flag generates both DOCX and PDF - Image Support: Resolves images relative to source directory - Template Support: Optional custom DOCX/DOTX template via -t flag

Common Workflow:

# 1. Update version number in index file (rapids_index.md)
# 2. Generate documentation
cd /mnt/data/github/rtldesignsherpa
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc --title-page --pdf

# 3. Output files created:
#    - RAPIDS_Specification_v0.25.docx
#    - RAPIDS_Specification_v0.25.pdf (if --pdf used)

Debug Mode:

# Generate debug markdown to see combined output
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o output.docx \
    --debug-md

# This creates debug.md showing the complete merged content

Tool Requirements: - Python 3.6+ - Pandoc installed and in PATH - For PDF generation: LaTeX (e.g., texlive) or use Pandoc’s built-in PDF writer

📖 See: /mnt/data/github/rtldesignsherpa/bin/md_to_docx.py for complete implementation details

## 31.20	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/rapids/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/rapids/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the rapids_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 31.21	17. References

### 31.21.1	16.1 Internal Documentation

- Complete Spec: rapids_spec/ ← Primary technical reference

- Validation: docs/RAPIDS_Validation_Status_Report.md

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 31.21.2	16.2 Related Subsystems

- AMBA: rtl/amba/ - Monitor infrastructure used in RAPIDS

- Common: rtl/common/ - Building blocks (counters, FIFOs, etc.)

- CocoTB Framework: bin/TBClasses/rapids/

### 31.21.3	16.3 External References

- AXI4 Specification: ARM IHI0022E

- AXIL4 Specification: ARM IHI0022E (subset)

- Network interface specs (custom Network protocol)

Document Version: 1.0 Last Updated: 2025-09-30 Review Cycle: Monthly during active development Next Review: 2025-10-30 Owner: RTL Design Sherpa Project

## 31.22	Navigation

- ← Back to Root: /PRD.md

- Complete Specification: rapids_spec/rapids_index.md

- Quick Start: README.md

- AI Guidance: CLAUDE.md

- Tasks: TASKS.md (to be created)

- Issues: known_issues/

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 32	Claude Code Guide: RAPIDS Subsystem

Version: 1.0 Last Updated: 2025-10-11 Purpose: AI-specific guidance for working with RAPIDS subsystem

## 32.1	Quick Context

What: Rapid AXI Programmable In-band Descriptor System - Custom DMA-style accelerator with network interfaces Status: 🟡 Active development - ~80% test coverage, validation in progress Your Role: Help users understand architecture, fix bugs, extend functionality

📖 Complete Specification: projects/components/rapids/docs/rapids_spec/ ← Always reference this for technical details

## 32.2	📖 Global Requirements Reference

IMPORTANT: Check /GLOBAL_REQUIREMENTS.md before starting RAPIDS work

All mandatory requirements are consolidated in the global requirements document: - See: /GLOBAL_REQUIREMENTS.md - Repository-wide mandatory requirements - RAPIDS-Specific: Attribution format, BFM usage requirements - Universal: TB location, three methods, TBBase inheritance, 100% success

This CLAUDE.md provides RAPIDS-specific guidance. Also review: - Root /CLAUDE.md - Repository-wide patterns - projects/components/CLAUDE.md - Project area standards (reset macros, FPGA attributes) - bin/TBClasses/CLAUDE.md - Framework usage patterns

## 32.3	Critical Rules for This Subsystem

### 32.3.1	Rule #0: Attribution Format for Git Commits

IMPORTANT: When creating git commit messages for RAPIDS documentation or code:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: RAPIDS receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

### 32.3.2	Rule #0.1: Testbench Location and Test Structure (MANDATORY)

📖 See: /GLOBAL_REQUIREMENTS.md Section 2.1 for complete requirement

RAPIDS-Specific Directory Structure:

projects/components/rapids/dv/
├── tbclasses/                    # ★ RAPIDS TB classes (project area!)
│   ├── scheduler_tb.py           # Scheduler testbench
│   ├── descriptor_engine_tb.py   # Descriptor engine testbench
│   └── rapids_integration_tb.py  # Integration testbench
└── tests/                        # Test runners
    ├── fub_tests/scheduler/test_scheduler.py
    ├── fub_tests/descriptor_engine/test_desc_engine.py
    └── integration_tests/test_rapids_integration.py

RAPIDS Import Pattern:

# Import framework utilities (PYTHONPATH includes bin/)
import os, sys
from TBClasses.shared.utilities import get_repo_root
from TBClasses.shared.tbbase import TBBase

# Add repo root to Python path using robust git-based method
repo_root = get_repo_root()
sys.path.insert(0, repo_root)

# Import RAPIDS TB from project area
from projects.components.rapids.dv.tbclasses.scheduler_tb import SchedulerTB

RAPIDS Test File Organization:

# 1. CocoTB functions at top (prefix "cocotb_test_*")
@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic_flow(dut):
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()
    # ... test logic

# 2. Parameter generation near bottom
def generate_scheduler_test_params():
    return [(0, 8, 512, 8)]  # (channel_id, num_channels, data_width, credit_width)

scheduler_params = generate_scheduler_test_params()

# 3. Pytest wrappers at bottom
@pytest.mark.parametrize("channel_id, ...", scheduler_params)
def test_basic_flow(request, channel_id, ...):
    run(..., testcase="cocotb_test_basic_flow", ...)

Why RAPIDS Tests Use Project Area: 1. Reusability: Same TB in FUB tests, integration tests, system tests 2. Composition: Scheduler TB + Descriptor TB → Integration TB 3. Discovery: All RAPIDS code under projects/components/rapids/

📖 Complete Pattern: val/amba/test_apb_slave.py lines 1251-1346 (reference example)

📖 Queue-Based Verification Pattern: See /GLOBAL_REQUIREMENTS.md Section 2.4

RAPIDS-Specific Usage:

RAPIDS uses queue-based verification for in-order verification of program engine, descriptor engine, and AXI transactions.

Example - Program Engine Verification:

# Direct queue access for in-order RAPIDS operations
class ProgramEngineTB(TBBase):
    async def verify_write_operation(self, expected_addr, expected_data):
        """RAPIDS program engine: In-order writes, simple queue verification"""
        # Wait for AXI transaction
        await self.wait_for_transaction(self.aw_monitor)

        # Get from queue - program engine writes are always in-order
        aw_pkt = self.aw_monitor._recvQ.popleft()
        w_pkt = self.w_monitor._recvQ.popleft()

        # Verify
        assert aw_pkt.addr == expected_addr
        assert w_pkt.data == expected_data

When RAPIDS Uses Memory Models: - ✗ Descriptor engine (in-order) - Use queue access - ✗ Program engine (in-order) - Use queue access - ✓ Integration tests with multiple masters - Memory model tracks state

📖 Complete Patterns: docs/VERIFICATION_ARCHITECTURE_GUIDE.md

### 32.3.3	Rule #0.5: Three Mandatory TB Methods (MANDATORY)

📖 See: /GLOBAL_REQUIREMENTS.md Section 2.2 for complete requirement

RAPIDS-Specific Context:

Many RAPIDS modules (especially scheduler and credit management) require configuration signals set BEFORE reset is released. This is because some counters/state are initialized during reset based on these config values.

Critical Example - Exponential Credit Encoding:

async def setup_clocks_and_reset(self):
    """RAPIDS-specific: Config signals MUST be set before reset"""
    # Start clock
    await self.start_clock('clk', freq=10, units='ns')

    # ⚠ CRITICAL: Set exponential credit config BEFORE reset
    # Scheduler's credit counter initializes during reset based on this value
    self.dut.cfg_initial_credit.value = 4  # 4 = 16 credits (2^4)
    self.dut.cfg_use_credit.value = 1

    # Now perform reset sequence
    await self.assert_reset()
    await self.wait_clocks('clk', 10)
    await self.deassert_reset()
    await self.wait_clocks('clk', 5)

async def assert_reset(self):
    """Assert active-low reset"""
    self.dut.rst_n.value = 0

async def deassert_reset(self):
    """Release active-low reset"""
    self.dut.rst_n.value = 1

Why Config-Before-Reset Matters for RAPIDS: - Scheduler credit counter: Initialized to (1 << cfg_initial_credit) during reset - Descriptor engine depth: Configuration read during reset sequence - SRAM parameters: Must be stable before memory controllers come out of reset

Common RAPIDS Configs to Set Before Reset: - cfg_initial_credit - Exponential credit encoding (0→1, 1→2, 2→4, etc.) - cfg_use_credit - Enable/disable credit-based flow control - cfg_timeout_threshold - Watchdog timer configuration - cfg_sram_depth - SRAM buffer size parameters

### 32.3.4	Rule #0.75: Audit Signal Naming BEFORE Writing Testbenches

⚠ CRITICAL: Check for Signal Naming Conflicts BEFORE Factory Usage ⚠

Before writing testbench code that uses AXI factory functions, audit your RTL for signal naming conflicts.

The Problem: AXI factory pattern matching searches for signals using prefix + channel patterns (e.g., {prefix}ar_valid, {prefix}r_ready). If you have: - Internal signals: desc_valid, desc_ready (simple handshake) - External AXI ports: desc_ar_valid, desc_ar_ready (AXI AR channel)

Both match desc_*valid → Factory finds BOTH signals → Initialization FAILS!

Solution - Signal Naming Audit Tool:

# Audit single file before writing testbench
../../bin/audit_signal_naming_conflicts.py projects/components/rapids/rtl/rapids_macro/scheduler_group.sv

# Audit entire RAPIDS subsystem
../../bin/audit_signal_naming_conflicts.py projects/components/rapids/rtl/

# Generate markdown report
../../bin/audit_signal_naming_conflicts.py projects/components/rapids/rtl/ --markdown signal_conflicts.md

Known RAPIDS Conflicts: See projects/components/rapids/known_issues/scheduler_group_signal_naming_conflicts.md for documented conflicts in scheduler_group.sv: - desc prefix: 4 internal + 4 external (AR/R channels) - prog prefix: 4 internal + 6 external (AW/W/B channels)

Recommended Workflow: 1. ✓ Write RTL module with signals 2. ✓ Run audit script to detect conflicts 3. ✓ Fix naming conflicts (rename internal signals with _to_sched suffix) 4. ✓ Write testbench using factory pattern matching

Three Solutions When Conflicts Found: 1. Rename internal signals (recommended): desc_valid → desc_to_sched_valid 2. Use explicit signal_map: Bypass pattern matching with manual signal mapping 3. Test at higher level: Where internal signals aren’t visible (e.g., rapids_top)

📖 Complete Guide: bin/SIGNAL_NAMING_AUDIT.md

### 32.3.5	Rule #1: MANDATORY BFM Usage for FUB-Level Tests

⚠ CRITICAL DESIGN INTENTION - USE FRAMEWORK BFMs ⚠

For all RAPIDS FUB (Functional Unit Block) level tests, you MUST use appropriate BFMs from the CocoTB Framework:

Interface Type → Required BFM Component:

| Interface Type | Framework Component Location | Usage |
| --- | --- | --- |
| Custom valid/ready | bin/TBClasses/components/gaxi/ | GAXI Master/Slave BFMs |
| AXI4 | bin/TBClasses/components/axi4/ | AXI4 Master/Slave drivers |
| AXI4-Lite (AXIL) | bin/TBClasses/components/axil4/ | AXIL Master/Slave drivers |
| APB | bin/TBClasses/components/apb/ | APB Master/Slave drivers |
| AXI-Stream (AXIS) | bin/TBClasses/components/axis4/ | AXIS Master/Slave drivers |
| Network | bin/TBClasses/components/network/ | Network Master/Slave drivers |
| MonBus | bin/TBClasses/components/monbus/ | MonBus drivers |

Critical Rules:

- NEVER manually drive valid/ready handshakes - Use GAXI BFM components

- NEVER create custom protocol drivers - Framework components already exist

- NEVER embed simple handshake logic - Extract to GAXI Master/Slave

Example - Program Engine Interface:

# ✗ WRONG: Manual handshake driving
async def send_program_request(self, addr, data):
    # Manually driving program_valid/program_ready - DON'T DO THIS!
    self.dut.program_valid.value = 1
    self.dut.program_pkt_addr.value = addr
    self.dut.program_pkt_data.value = data
    while int(self.dut.program_ready.value) == 0:
        await self.wait_clocks(self.clk_name, 1)
    await self.wait_clocks(self.clk_name, 1)
    self.dut.program_valid.value = 0

# ✓ CORRECT: Use GAXI Master BFM
from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster

class ProgramEngineTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        # Create GAXI master for program interface
        self.program_master = GAXIMaster(
            dut=dut,
            clock=dut.clk,
            valid_signal='program_valid',
            ready_signal='program_ready',
            data_signals=['program_pkt_addr', 'program_pkt_data'],
            data_widths=[64, 32],
            name='program_interface'
        )

    async def send_program_request(self, addr, data):
        # Use GAXI master for proper handshaking
        await self.program_master.write({'program_pkt_addr': addr, 'program_pkt_data': data})

Why This Rule Exists:

- Consistency: All tests use same handshake protocol

- Correctness: GAXI BFMs handle complex timing scenarios correctly

- Reusability: Same BFM used across all RAPIDS tests

- Maintainability: Fix once in BFM, all tests benefit

- Coverage: GAXI BFMs include timing randomization and backpressure

When Manual Driving is Acceptable:

- ✓ Quick debug/prototype - Temporary, will be replaced with BFM

- ✓ Clock/reset initialization - Not part of protocol handshaking

- ✗ Production testbenches - MUST use BFMs

Search Before Creating:

# Find existing GAXI components
find bin/TBClasses/components/gaxi/ -name "*.py"

# Find example usage
grep -r "GAXIMaster\|GAXISlave" projects/components/rapids/dv/tests/fub_tests/
grep -r "from.*gaxi" bin/TBClasses/

BFM Selection Guide:

Need to drive interface with valid/ready?
├─ Standard protocol (AXI4, APB, AXIS, Network)?
│  └─ Use protocol-specific BFM (axi4/, apb/, axis4/, network/)
│
├─ Custom RAPIDS interface with valid/ready signals?
│  └─ Use GAXI Master/Slave BFM (gaxi/)
│
└─ MonBus monitor packet interface?
   └─ Use MonBus BFM (monbus/)

📖 See: - bin/TBClasses/components/gaxi/README.md - GAXI BFM documentation - bin/TBClasses/components/axi4/README.md - AXI4 BFM documentation - val/amba/test_*.py - Reference examples using framework BFMs

### 32.3.6	Rule #2: Always Reference Detailed Specification

This subsystem has extensive documentation in projects/components/rapids/docs/rapids_spec/

Before answering technical questions:

# Check complete specification
ls projects/components/rapids/docs/rapids_spec/
cat projects/components/rapids/docs/rapids_spec/rapids_index.md
cat projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md

Your answer should: 1. Provide direct answer/code 2. Then link to detailed spec: “See projects/components/rapids/docs/rapids_spec/{file}.md for complete specification”

### 32.3.7	Rule #3: Know the Known Issues

Fixed Critical Issue: - ✓ Scheduler Credit Counter - FIXED (projects/components/rapids/rtl/rapids_fub/scheduler.sv:567-570) - Was: Credit counter hardcoded to 0 - Now: Implements exponential encoding (0→1, 1→2, 2→4, …, 15→∞) - Impact: Credit-based flow control now functional with proper encoding - Tests: Verify exponential decoding works correctly - Priority: P0 (Critical - COMPLETED)

Always check: projects/components/rapids/known_issues/ before diagnosing bugs

ls projects/components/rapids/known_issues/
cat projects/components/rapids/known_issues/scheduler.md

### 32.3.8	Rule #4: RAPIDS is Complex - Understand Block Interactions

Key Interaction Patterns:

- Descriptor Flow:

- Software → AXIL4 → Descriptor Engine → Scheduler → Data Paths

- Sink Data Path (Network → Memory):

- Network Slave → Sink SRAM Control → Sink AXI Write Engine → System Memory

- Source Data Path (Memory → Network):

- System Memory → Source AXI Read Engine → Source SRAM Control → Network Master

- Monitoring:

- All Blocks → MonBus Reporter → MonBus Output

Never work on one block in isolation without understanding its upstream/downstream dependencies!

### 32.3.9	Rule #5: Test Strategy is Multi-Layered

RAPIDS testing follows this hierarchy:

- FUB (Functional Unit Block) Tests: projects/components/rapids/dv/tests/fub_tests/

  - Individual block testing

  - Focus: Module-level functionality

  - Example: Scheduler FSM, Descriptor Engine FIFO

- Integration Tests: projects/components/rapids/dv/tests/integration_tests/

  - Multi-block scenarios

  - Focus: Block-to-block interfaces

  - Example: Scheduler + Descriptor Engine, Sink data path end-to-end

- System Tests: projects/components/rapids/dv/tests/system_tests/

  - Full RAPIDS operation

  - Focus: Realistic traffic patterns

  - Example: Complete DMA transfer with monitoring

When creating/modifying tests, ensure appropriate test layer is used!

## 32.4	Architecture Quick Reference

### 32.4.1	Block Organization

RAPIDS Architecture (17 SystemVerilog modules)
├── Scheduler Group
│   ├── scheduler.sv              (Task FSM, credit management)
│   ├── descriptor_engine.sv      (Descriptor FIFO, parsing)
│   └── program_engine.sv         (Program sequencing, alignment)
│
├── Sink Data Path (Network → Memory)
│   ├── network_slave.sv             (Network ingress)
│   ├── sink_sram_control.sv      (Buffer management)
│   └── sink_axi_write_engine.sv  (AXI4 write to system memory)
│
├── Source Data Path (Memory → Network)
│   ├── source_axi_read_engine.sv (AXI4 read from system memory)
│   ├── source_sram_control.sv    (Buffer management)
│   └── network_master.sv            (Network egress)
│
└── MonBus AXIL Group
    ├── axil4_slave.sv            (Control/status registers)
    └── monbus_reporter.sv        (Monitor packet generation)

📖 See: projects/components/rapids/docs/rapids_spec/ch02_blocks/00_overview.md for detailed block descriptions

### 32.4.2	Module Quick Reference

| Module | Location | Purpose | Documentation |
| --- | --- | --- | --- |
| scheduler.sv | rapids_fub/ | Task FSM, credit management | rapids_spec/ch02_blocks/01_01_scheduler.md |
| descriptor_engine.sv | rapids_fub/ | Descriptor FIFO, parsing | rapids_spec/ch02_blocks/01_02_descriptor_engine.md |
| program_engine.sv | rapids_fub/ | Program sequencing | rapids_spec/ch02_blocks/01_03_program_engine.md |
| network_slave.sv | rapids_fub/ | Network ingress | rapids_spec/ch02_blocks/02_01_network_slave.md |
| sink_sram_control.sv | rapids_fub/ | Sink buffer management | rapids_spec/ch02_blocks/02_02_sink_sram_control.md |
| sink_axi_write_engine.sv | rapids_fub/ | Memory writes | rapids_spec/ch02_blocks/02_03_sink_axi_write_engine.md |
| source_axi_read_engine.sv | rapids_fub/ | Memory reads | rapids_spec/ch02_blocks/03_03_source_axi_read_engine.md |
| source_sram_control.sv | rapids_fub/ | Source buffer management | rapids_spec/ch02_blocks/03_02_source_sram_control.md |
| network_master.sv | rapids_fub/ | Network egress | rapids_spec/ch02_blocks/03_01_network_master.md |
| rapids_top.sv | rapids_macro/ | Top-level integration | rapids_spec/ch03_interfaces/01_top_level.md |

### 32.4.3	Interface Summary

| Interface | Type | Width | Purpose | Specification |
| --- | --- | --- | --- | --- |
| AXIL4 | Slave | 32-bit | Control/status registers | rapids_spec/ch03_interfaces/02_axil_interface_spec.md |
| AXI4 (Sink) | Master | Configurable | Write to system memory | rapids_spec/ch03_interfaces/03_axi4_interface_spec.md |
| AXI4 (Source) | Master | Configurable | Read from system memory | rapids_spec/ch03_interfaces/03_axi4_interface_spec.md |
| Network (Sink) | Slave | Configurable | Network ingress | rapids_spec/ch03_interfaces/04_network_interface_spec.md |
| Network (Source) | Master | Configurable | Network egress | rapids_spec/ch03_interfaces/04_network_interface_spec.md |
| MonBus | Master | 64-bit | Monitor packet output | rapids_spec/ch03_interfaces/05_monbus_interface_spec.md |

## 32.5	Common User Questions and Responses

### 32.5.1	Q: “How does the scheduler work?”

A: Direct answer:

The scheduler is a complex FSM that coordinates RAPIDS operations:

- Idle State: Waits for descriptors from Descriptor Engine

- Parse State: Extracts descriptor fields (address, length, control)

- Credit Check: Verifies credit availability (if credit mode enabled)

- Execute State: Activates appropriate data path (sink or source)

- Monitor State: Tracks operation progress

- Complete State: Generates completion packets, updates credits

Key FSM States: - IDLE → PARSE → CREDIT_CHECK → EXECUTE → MONITOR → COMPLETE → IDLE

Credit Management: - ✓ FIXED: Exponential credit encoding now implemented (lines 567-570) - Encoding: 0→1, 1→2, 2→4, 3→8, …, 14→16384, 15→∞ (unlimited) - See Q&A below for complete encoding table and details

📖 See: - projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md - Complete FSM specification - projects/components/rapids/known_issues/scheduler.md - Known bugs and workarounds

### 32.5.2	Q: “How do I configure RAPIDS?”

A: Configuration via AXIL4 interface:

// 1. Initialize RAPIDS via AXIL4 writes
// Configure SRAM depths
write_axil(ADDR_SINK_SRAM_DEPTH, 1024);
write_axil(ADDR_SOURCE_SRAM_DEPTH, 1024);

// Set timeout thresholds
write_axil(ADDR_TIMEOUT_THRESHOLD, 1000);

// Configure initial credits (exponential encoding)
// 0→1, 1→2, 2→4, 3→8, 4→16, etc.
write_axil(ADDR_INITIAL_CREDIT, 4);  // 4 = 16 credits (2^4)
write_axil(ADDR_CREDIT_ENABLE, 1);   // ✓ Enable credit mode (now fixed!)

// 2. Load descriptors
write_descriptor(addr, length, control_bits);

// 3. Enable operation
write_axil(ADDR_ENABLE, 1);

📖 See: - projects/components/rapids/docs/rapids_spec/ch04_programming_models/01_programming.md - Programming model - projects/components/rapids/docs/rapids_spec/ch05_registers/01_registers.md - Register definitions

### 32.5.3	Q: “What’s the data flow for network to memory transfer?”

A: Sink path data flow:

1. Network Packet Arrives
   ↓
2. Network Slave receives packet
   - Validates packet format
   - Handshakes with Network interface
   ↓
3. Sink SRAM Control buffers data
   - Writes to SRAM
   - Manages write pointers
   - Handles backpressure
   ↓
4. Sink AXI Write Engine
   - Reads from SRAM
   - Generates AXI4 write transactions
   - Bursts data to system memory
   ↓
5. Completion Reporting
   - Generates MonBus completion packet
   - Updates scheduler state

Key Considerations: - SRAM acts as buffer to decouple network from memory timing - AXI4 bursts used for efficient memory access - Backpressure propagates from memory to network

📖 See: projects/components/rapids/docs/rapids_spec/ch02_blocks/02_00_sink_data_path.md

### 32.5.4	Q: “What’s the credit counter bug and how does exponential encoding work?”

A: Scheduler credit counter exponential encoding:

The scheduler uses exponential credit encoding to provide a wide range of credit values (1 to 16384) with a compact 4-bit configuration:

Encoding Table: | cfg_initial_credit | Actual Credits | Use Case | |———————|—————-|———-| | 0 | 1 | Minimum (2^0) | | 1 | 2 | Very low traffic (2^1) | | 2 | 4 | Low traffic (2^2) | | 3 | 8 | (2^3) | | 4 | 16 | Typical (2^4) | | 5 | 32 | (2^5) | | 6 | 64 | Medium traffic (2^6) | | 7 | 128 | (2^7) | | 8 | 256 | High traffic (2^8) | | 10 | 1024 | Very high traffic (2^10) | | 14 | 16384 | Maximum finite (2^14) | | 15 | ∞ (0xFFFFFFFF) | Unlimited credits |

✓ FIXED Implementation:

// projects/components/rapids/rtl/rapids_fub/scheduler.sv:567-570
// Exponential credit encoding: 0→1, 1→2, 2→4, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Was Broken (Before Fix):

r_descriptor_credit_counter <= 32'h0;  // ✗ WRONG: Hardcoded to 0

Impact (Before Fix): - Credit-based flow control didn’t work - All operations blocked if credit mode enabled - Descriptors never processed

Encoding Rationale: - Compact 4-bit config covers wide range: 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞)

Important: Exponential encoding applies only at initialization. Once running, the counter operates linearly (increment/decrement by 1).

📖 See: - projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md - Complete encoding table - projects/components/rapids/known_issues/scheduler.md - Bug details and fix verification

### 32.5.5	Q: “How do I run RAPIDS tests?”

A: Multi-layered test approach:

# 1. FUB (Functional Unit Block) Tests - Individual blocks
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ -v
pytest projects/components/rapids/dv/tests/fub_tests/descriptor_engine/ -v
pytest projects/components/rapids/dv/tests/fub_tests/ -v  # All FUB tests

# 2. Integration Tests - Multi-block scenarios
pytest projects/components/rapids/dv/tests/integration_tests/ -v

# 3. System Tests - Full RAPIDS operation
pytest projects/components/rapids/dv/tests/system_tests/ -v

# Run with waveforms for debugging
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ --vcd=debug.vcd
gtkwave debug.vcd

Test Organization: - FUB tests: Focus on individual block functionality - Integration tests: Verify block-to-block interfaces - System tests: Validate complete data flows

Current Status: ~80% functional coverage, basic scenarios validated

📖 See: docs/RAPIDS_Validation_Status_Report.md for detailed test results

## 32.6	Integration Patterns

### 32.6.1	Pattern 1: Basic RAPIDS Instantiation

rapids_top #(
    .AXI_ADDR_WIDTH(32),
    .AXI_DATA_WIDTH(64),
    .Network_DATA_WIDTH(64),
    .SRAM_DEPTH(1024),
    .MAX_DESCRIPTORS(16)
) u_rapids (
    // Clock and Reset
    .aclk               (system_clk),
    .aresetn            (system_rst_n),

    // AXIL4 Control Interface
    .s_axil_awaddr      (ctrl_awaddr),
    .s_axil_awvalid     (ctrl_awvalid),
    .s_axil_awready     (ctrl_awready),
    .s_axil_wdata       (ctrl_wdata),
    .s_axil_wstrb       (ctrl_wstrb),
    .s_axil_wvalid      (ctrl_wvalid),
    .s_axil_wready      (ctrl_wready),
    .s_axil_bresp       (ctrl_bresp),
    .s_axil_bvalid      (ctrl_bvalid),
    .s_axil_bready      (ctrl_bready),
    .s_axil_araddr      (ctrl_araddr),
    .s_axil_arvalid     (ctrl_arvalid),
    .s_axil_arready     (ctrl_arready),
    .s_axil_rdata       (ctrl_rdata),
    .s_axil_rresp       (ctrl_rresp),
    .s_axil_rvalid      (ctrl_rvalid),
    .s_axil_rready      (ctrl_rready),

    // AXI4 Memory Interface (Sink - Write)
    .m_axi_sink_awaddr  (mem_sink_awaddr),
    .m_axi_sink_awlen   (mem_sink_awlen),
    .m_axi_sink_awsize  (mem_sink_awsize),
    .m_axi_sink_awburst (mem_sink_awburst),
    .m_axi_sink_awvalid (mem_sink_awvalid),
    .m_axi_sink_awready (mem_sink_awready),
    // ... additional AXI4 sink write channel signals

    // AXI4 Memory Interface (Source - Read)
    .m_axi_source_araddr  (mem_source_araddr),
    .m_axi_source_arlen   (mem_source_arlen),
    .m_axi_source_arsize  (mem_source_arsize),
    .m_axi_source_arburst (mem_source_arburst),
    .m_axi_source_arvalid (mem_source_arvalid),
    .m_axi_source_arready (mem_source_arready),
    // ... additional AXI4 source read channel signals

    // Network Network Interface (Sink - Receive)
    .s_network_tdata       (net_rx_data),
    .s_network_tvalid      (net_rx_valid),
    .s_network_tready      (net_rx_ready),
    .s_network_tlast       (net_rx_last),
    // ... additional Network sink signals

    // Network Network Interface (Source - Transmit)
    .m_network_tdata       (net_tx_data),
    .m_network_tvalid      (net_tx_valid),
    .m_network_tready      (net_tx_ready),
    .m_network_tlast       (net_tx_last),
    // ... additional Network source signals

    // MonBus Output
    .monbus_pkt_valid   (rapids_mon_valid),
    .monbus_pkt_ready   (rapids_mon_ready),
    .monbus_pkt_data    (rapids_mon_data)
);

### 32.6.2	Pattern 2: Configuration Sequence

// Recommended initialization sequence
initial begin
    // 1. Assert reset
    aresetn = 0;
    repeat(10) @(posedge aclk);
    aresetn = 1;

    // 2. Configure RAPIDS via AXIL4
    axil_write(ADDR_SINK_SRAM_DEPTH, 1024);
    axil_write(ADDR_SOURCE_SRAM_DEPTH, 1024);
    axil_write(ADDR_TIMEOUT_THRESHOLD, 1000);
    axil_write(ADDR_INITIAL_CREDIT, 4);  // 4 = 16 credits (2^4, exponential encoding)
    axil_write(ADDR_CREDIT_ENABLE, 1);  // ✓ Enable credit mode (now fixed!)

    // 3. Load descriptors
    for (int i = 0; i < num_descriptors; i++) begin
        axil_write(ADDR_DESC_ADDR, descriptor[i].addr);
        axil_write(ADDR_DESC_LEN, descriptor[i].length);
        axil_write(ADDR_DESC_CTRL, descriptor[i].control);
        axil_write(ADDR_DESC_COMMIT, 1);
    end

    // 4. Enable RAPIDS operation
    axil_write(ADDR_ENABLE, 1);
end

### 32.6.3	Pattern 3: MonBus Integration

// Always add downstream FIFO for MonBus
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_rapids_mon_fifo (
    .i_clk      (aclk),
    .i_rst_n    (aresetn),
    .i_data     (monbus_pkt_data),
    .i_valid    (monbus_pkt_valid),
    .o_ready    (monbus_pkt_ready),
    .o_data     (fifo_mon_data),
    .o_valid    (fifo_mon_valid),
    .i_ready    (consumer_ready)
);

## 32.7	Anti-Patterns to Catch

### 32.7.1	✗ Anti-Pattern 1: Not Understanding Exponential Credit Encoding

✗ WRONG:
.cfg_initial_credit(4'd16),  // Thinks this gives 16 credits - it doesn't!

✓ CORRECTED:
"Credits use exponential encoding:
- cfg_initial_credit = 4 → 16 credits (2^4)
- cfg_initial_credit = 8 → 256 credits (2^8)
- cfg_initial_credit = 15 → ∞ credits (unlimited)
See projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md for encoding table."

### 32.7.2	✗ Anti-Pattern 2: Insufficient SRAM Depth

✗ WRONG:
.SRAM_DEPTH(16)  // Too small for realistic packets

✓ CORRECTED:
"SRAM depth should match typical packet sizes.
Recommended: 1024-4096 entries depending on data width and packet sizes."

### 32.7.3	✗ Anti-Pattern 3: No MonBus Downstream Handling

✗ WRONG:
assign monbus_pkt_ready = 1'b1;  // Always ready = potential packet loss

✓ CORRECTED:
"Connect to FIFO or proper consumer:
gaxi_fifo_sync #(.DATA_WIDTH(64), .DEPTH(256)) u_mon_fifo (
    .i_valid(monbus_pkt_valid),
    .i_data(monbus_pkt_data),
    .o_ready(monbus_pkt_ready),
    ...
);"

### 32.7.4	✗ Anti-Pattern 4: Testing Individual Blocks in Isolation

✗ WRONG:
"Only test scheduler without descriptor engine"

✓ CORRECTED:
"RAPIDS blocks are tightly coupled. Always test:
1. FUB tests for basic block functionality
2. Integration tests for block interactions
3. System tests for complete flows"

## 32.8	Debugging Workflow

### 32.8.1	Issue: Scheduler Not Processing Descriptors

Check in order: 1. ✓ Credit configuration correct? (Remember exponential encoding!) 2. ✓ Descriptors loaded via AXIL4? 3. ✓ RAPIDS enabled? (ADDR_ENABLE = 1) 4. ✓ Reset properly deasserted? 5. ✓ Descriptor engine FIFO not empty?

Debug commands:

pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ -v -s  # Verbose test
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ --vcd=debug.vcd
gtkwave debug.vcd  # Inspect FSM state transitions

### 32.8.2	Issue: Data Path Stalls

Check in order: 1. ✓ SRAM depth sufficient? 2. ✓ Downstream interfaces ready? 3. ✓ AXI4 backpressure handling? 4. ✓ Network flow control? 5. ✓ Buffer overflow/underflow detection?

Waveform Analysis: - Check SRAM read/write pointers - Verify AXI4 handshakes - Inspect Network TREADY signals

### 32.8.3	Issue: MonBus Packets Not Generated

Check in order: 1. ✓ Operations completing successfully? 2. ✓ MonBus ready signal asserted? 3. ✓ MonBus reporter enabled? 4. ✓ Downstream FIFO not full?

## 32.9	Testing Guidance

### 32.9.1	Test Organization

projects/components/rapids/dv/tests/
├── fub_tests/                  # Individual block tests
│   ├── scheduler/
│   │   ├── test_scheduler.py   # FSM, credit management
│   │   └── conftest.py
│   ├── descriptor_engine/
│   │   ├── test_desc_engine.py # FIFO, parsing
│   │   └── conftest.py
│   ├── program_engine/
│   ├── network_interfaces/
│   └── simple_sram/
│
├── integration_tests/          # Multi-block scenarios
│   ├── test_scheduler_desc.py  # Scheduler + Descriptor Engine
│   ├── test_sink_path.py       # Complete sink data path
│   └── test_source_path.py     # Complete source data path
│
└── system_tests/               # Full RAPIDS operation
    ├── test_full_dma.py        # Complete DMA transfer
    └── test_bidirectional.py   # Simultaneous sink/source

### 32.9.2	Running Tests

# Single block test
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ -v

# All FUB tests
pytest projects/components/rapids/dv/tests/fub_tests/ -v

# Integration tests
pytest projects/components/rapids/dv/tests/integration_tests/ -v

# System tests
pytest projects/components/rapids/dv/tests/system_tests/ -v

# All RAPIDS tests
pytest projects/components/rapids/dv/tests/ -v

# With waveforms
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ --vcd=waves.vcd

### 32.9.3	Test Coverage Status

Current Status: ~80% functional coverage

| Component | Coverage | Status |
| --- | --- | --- |
| Scheduler | ~90% | Exponential credit encoding implemented |
| Descriptor Engine | ~80% | Basic tests passing |
| Program Engine | ~85% | Alignment tested |
| Sink Data Path | ~75% | Basic flows working |
| Source Data Path | ~70% | Basic flows working |
| Integration | ~60% | More stress testing needed |

## 32.10	Key Documentation Links

### 32.10.1	Always Reference These

Primary Technical Specification: - projects/components/rapids/docs/rapids_spec/rapids_index.md - Complete specification index - projects/components/rapids/docs/rapids_spec/ch01_overview/ - Architecture overview - projects/components/rapids/docs/rapids_spec/ch02_blocks/ - Block specifications - projects/components/rapids/docs/rapids_spec/ch03_interfaces/ - Interface specifications - projects/components/rapids/docs/rapids_spec/ch04_programming_models/ - Programming model - projects/components/rapids/docs/rapids_spec/ch05_registers/ - Register definitions

This Subsystem: - projects/components/rapids/PRD.md - Requirements overview - projects/components/rapids/README.md - Quick start guide (if exists) - projects/components/rapids/TASKS.md - Current work items - projects/components/rapids/known_issues/ - Bug tracking

Validation: - docs/RAPIDS_Validation_Status_Report.md - Test results and status

Root: - /PRD.md - Master requirements - /CLAUDE.md - Repository guide

## 32.11	Quick Commands

# View complete specification
cat projects/components/rapids/docs/rapids_spec/rapids_index.md
cat projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md

# Check known issues
ls projects/components/rapids/known_issues/
cat projects/components/rapids/known_issues/scheduler.md

# Run tests
pytest projects/components/rapids/dv/tests/fub_tests/ -v
pytest projects/components/rapids/dv/tests/integration_tests/ -v

# Lint
verilator --lint-only projects/components/rapids/rtl/rapids_fub/scheduler.sv

# Search for modules
find projects/components/rapids/rtl/rapids_fub/ -name "*.sv" -exec grep -H "^module" {} \;

## 32.12	Verification Patterns and Best Practices

### 32.12.1	Pattern: Continuous Background Monitoring

Problem: When testing components that output data asynchronously (descriptors, packets, etc.), tests that only check outputs at specific points will miss data that arrives during other operations.

Solution: Use continuous background monitoring coroutines that run throughout the test.

Example - Descriptor Engine Testing:

class DescriptorEngineTB(TBBase):
    async def run_apb_only_test(self, num_packets: int, profile: DelayProfile):
        """Test with continuous descriptor monitoring"""
        descriptors_collected = []  # Shared list
        monitor_active = True  # Control flag

        # Background coroutine monitors continuously
        async def descriptor_monitor():
            """Continuously monitor for descriptors throughout the test"""
            while monitor_active:
                await self.wait_clocks(self.clk_name, 1)
                if int(self.dut.descriptor_valid.value) == 1:
                    desc_data = int(self.dut.descriptor_packet.value)
                    descriptors_collected.append(desc_data)
                    if len(descriptors_collected) % 5 == 1:
                        self.log.info(f"📦 Descriptor {len(descriptors_collected)}: 0x{desc_data:X}")

        # Start background monitor
        monitor_task = cocotb.start_soon(descriptor_monitor())

        # Send all requests (monitor captures outputs during this phase)
        for i in range(num_packets):
            # Send APB request
            self.dut.apb_valid.value = 1
            self.dut.apb_addr.value = test_addr
            await self.wait_clocks(self.clk_name, 1)
            # ... wait for ready ...
            self.dut.apb_valid.value = 0

        # Wait for final descriptors (monitor still running)
        for cycle in range(final_window):
            await self.wait_clocks(self.clk_name, 1)
            if len(descriptors_collected) >= num_packets:
                break

        # Stop background monitor
        monitor_active = False
        await self.wait_clocks(self.clk_name, 2)  # Let monitor finish

        # Verify results
        return len(descriptors_collected) == num_packets

Key Points: 1. Shared State: Use list/dict accessible from both main test and monitor coroutine 2. Control Flag: monitor_active allows clean shutdown 3. Background Task: cocotb.start_soon() runs monitor concurrently 4. Continuous Capture: Monitor checks every clock cycle, never missing data 5. Clean Shutdown: Set flag to False, wait for monitor to finish

When to Use: - ✓ Asynchronous output interfaces (descriptors, packets, responses) - ✓ Tests with rapid-fire input operations - ✓ Multi-stage pipelines where outputs don’t align with inputs - ✗ Simple synchronous request-response patterns

Benefits: - 100% capture rate (no missed transactions) - Decouples input stimulus from output monitoring - Realistic timing coverage (captures outputs at any point)

Results: Descriptor engine tests improved from 42% success (5/12 descriptors) to 100% success (14/14 tests passing) by applying this pattern.

### 32.12.2	Pattern: Using Existing CocoTBFramework Components

Critical Rule: Always search for existing components before creating new ones.

Framework Structure:

bin/TBClasses/
├── components/           # Protocol-specific BFMs and drivers
│   ├── axi4/            # Complete AXI4 infrastructure
│   ├── axil4/           # AXI4-Lite components
│   ├── apb/             # APB drivers and monitors
│   ├── axis4/           # AXI-Stream components
│   └── rapids/            # RAPIDS-specific BFMs
├── tbclasses/           # Reusable testbench classes
│   ├── amba/            # AMBA testbenches
│   ├── rapids/            # RAPIDS testbenches
│   └── shared/          # Common utilities (TBBase, etc.)
└── scoreboards/         # Transaction checkers

Decision Tree: Create New BFM vs Use Existing?

Need to model protocol behavior?
├─ Is it a standard protocol (AXI4, APB, AXIS)?
│  └─ YES → Use components/axi4/, components/apb/, etc.
│           ✓ DO NOT create new implementation
│
├─ Is it RAPIDS-specific custom behavior?
│  ├─ Is it >100 lines?
│  │  └─ YES → Extract to components/rapids/
│  └─ Is it <50 lines and test-specific?
│     └─ YES → Keep embedded in testbench
│
└─ Is it generic helper (not protocol-specific)?
   └─ Add to tbclasses/shared/utilities.py

Example - Descriptor Engine AXI Responder:

# ✗ WRONG: Creating new AXI4 read responder BFM
# File: components/rapids/axi_read_responder_bfm.py
class AXIReadResponderBFM:
    """New AXI4 read responder - DON'T DO THIS!"""
    # ... 200 lines of AXI4 protocol implementation ...

# ✓ CORRECT: Use existing AXI4 components
# File: tbclasses/rapids/descriptor_engine_tb.py
async def axi_read_responder(self):
    """Simple test-specific responder - 50 lines, embedded"""
    while True:
        await self.wait_clocks(self.clk_name, 1)
        if int(self.dut.ar_valid.value) == 1 and int(self.dut.ar_ready.value) == 1:
            # Generate response data
            self.dut.r_valid.value = 1
            self.dut.r_data.value = response_data
            # ... wait for r_ready ...
            self.dut.r_valid.value = 0

Why This Matters: - Existing AXI4 components: Comprehensive, tested, feature-complete - Simple embedded responder: Test-specific, minimal protocol simulation - No duplication: Framework already has robust AXI4 infrastructure

BFM Extraction Criteria:

| Factor | Extract to BFM | Keep Embedded |
| --- | --- | --- |
| Lines of code | >100 lines | <50 lines |
| Complexity | Complex protocol logic | Simple stimulus/response |
| Reusability | Used across multiple tests | Test-specific |
| Protocol | Custom RAPIDS-specific | Standard protocol (use framework) |
| Dependencies | Standalone | Tightly coupled to one test |

Examples:

✓ Extracted to BFM: DataMoverBFM (components/rapids/data_mover_bfm.py) - 150+ lines - Complex RAPIDS data mover protocol - Used across scheduler tests - Reusable for integration tests

✓ Kept Embedded: AXI read responder in descriptor engine - 50 lines - Simple test-specific behavior - Framework already has full AXI4 support - No need for extraction

### 32.12.3	Test Scalability Patterns

Principle: Tests should scale from basic validation to comprehensive stress testing.

Pattern: Delay Profiles for Timing Coverage

class DelayProfile(Enum):
    """Delay profiles for comprehensive timing coverage"""
    FAST_PRODUCER = "fast_producer"      # Producer faster than consumer
    FAST_CONSUMER = "fast_consumer"      # Consumer faster than producer
    FIXED_DELAY = "fixed_delay"          # Predictable timing
    MINIMAL_DELAY = "minimal_delay"      # Stress test - minimal delays
    BACKPRESSURE = "backpressure"        # Heavy backpressure scenarios

# Test method scales with profile
async def run_apb_only_test(self, num_packets: int, profile: DelayProfile):
    """Scalable test with configurable timing"""
    params = self.delay_params[profile]

    for i in range(num_packets):
        # Apply profile-specific delays
        producer_delay = self.get_delay_value(params['producer_delay'])
        await self.wait_clocks(self.clk_name, producer_delay)

        # Send request...

        # Profile-specific backpressure
        if random.random() < params.get('backpressure_freq', 0.1):
            backpressure_cycles = random.randint(5, 25)
            await self.wait_clocks(self.clk_name, backpressure_cycles)

Pattern: Hierarchical Test Levels

# Test runner with multiple levels
@pytest.mark.parametrize("test_level", ["basic", "medium", "full"])
def test_descriptor_engine(test_level, ...):
    """Hierarchical test levels for different coverage needs"""

    if test_level == "basic":
        # Quick validation: 10 packets, simple timing
        num_packets = 10
        test_class = TestClass.APB_ONLY

    elif test_level == "medium":
        # Moderate coverage: 3 packets × 4 profiles
        num_packets = 3
        test_classes = [TestClass.APB_ONLY, TestClass.MIXED]

    elif test_level == "full":
        # Comprehensive: 5 packets × all profiles × all test classes
        num_packets = 5
        test_classes = [TestClass.APB_ONLY, TestClass.MIXED]

Benefits: - Basic: Quick smoke tests (CI/CD) - Medium: Developer validation - Full: Comprehensive regression

Pattern: Parametrized Test Generation

def generate_test_params():
    """Generate comprehensive parameter combinations"""
    params = []

    # Basic configurations
    for num_channels in [8, 32, 64]:
        for data_width in [64, 512]:
            for addr_width in [32, 64]:
                params.append((num_channels, data_width, addr_width))

    return params

# Pytest automatically runs all combinations
@pytest.mark.parametrize("num_channels, data_width, addr_width", generate_test_params())
def test_scheduler(num_channels, data_width, addr_width):
    """Test scales across all parameter combinations"""
    # ...

Test Success Criteria:

⚠ CRITICAL: All tests must achieve 100% success rate.

# ✗ WRONG: Accepting partial success
success_rate = (descriptors_received / total_sent) * 100
return success_rate >= 70  # "Mostly good" - NOT ACCEPTABLE

# ✓ CORRECT: Require 100% success
success_rate = (descriptors_received / total_sent) * 100
return success_rate >= 100  # Must receive ALL expected outputs

Why 100% Required: - Partial success indicates bugs, not acceptable tolerance - RTL should be deterministic - 100% success is achievable - Lower thresholds mask real issues

## 32.13	Documentation Generation

### 32.13.1	Generating PDF/DOCX from Specification

Tool: /mnt/data/github/rtldesignsherpa/bin/md_to_docx.py

Use this tool to convert the linked specification index into a single all-inclusive PDF or DOCX file.

Basic Usage:

# Generate DOCX from rapids_spec index
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc \
    --title-page

# Generate both DOCX and PDF
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc \
    --title-page \
    --pdf

# With custom template (optional)
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    -t path/to/template.dotx \
    --toc \
    --title-page \
    --pdf

Key Features: - Recursive Collection: Follows all markdown links in the index file - Heading Demotion: Automatically adjusts heading levels for included files - Table of Contents: --toc flag generates automatic ToC - Title Page: --title-page flag creates title page from first heading - PDF Export: --pdf flag generates both DOCX and PDF - Image Support: Resolves images relative to source directory - Template Support: Optional custom DOCX/DOTX template via -t flag

Common Workflow:

# 1. Update version number in index file (rapids_index.md)
# 2. Generate documentation
cd /mnt/data/github/rtldesignsherpa
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc --title-page --pdf

# 3. Output files created:
#    - RAPIDS_Specification_v0.25.docx
#    - RAPIDS_Specification_v0.25.pdf (if --pdf used)

Debug Mode:

# Generate debug markdown to see combined output
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o output.docx \
    --debug-md

# This creates debug.md showing the complete merged content

Tool Requirements: - Python 3.6+ - Pandoc installed and in PATH - For PDF generation: LaTeX (e.g., texlive) or use Pandoc’s built-in PDF writer

📖 See: /mnt/data/github/rtldesignsherpa/bin/md_to_docx.py for complete implementation details

## 32.14	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/rapids/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/rapids/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the rapids_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 32.15	Remember

- 🎛️ MANDATORY: Use BFMs - GAXI Master/Slave for custom valid/ready, protocol-specific BFMs for AXI4/APB/AXIS/Network

- 📖 Link to detailed spec - projects/components/rapids/docs/rapids_spec/ has complete architecture docs

- 🔢 Exponential credit encoding - Remember: 0→1, 1→2, 2→4, not linear!

- 🐛 Check known issues - Before diagnosing bugs

- 🔗 Block interactions - RAPIDS blocks are tightly coupled

- 🧪 Multi-layered testing - FUB → Integration → System tests

- 🏗️ Testbench reuse - Always create TB classes in bin/TBClasses/rapids/

- 🎯 Continuous monitoring - Use background coroutines for asynchronous output capture

- 🔍 Search first - Use existing CocoTBFramework components before creating new ones

- 📊 100% success - All tests must achieve 100% success rate, no exceptions

- 🏛️ Three-layer architecture - TB (infrastructure) + Test (intelligence) + Scoreboard (verification)

- 🎪 Queue-based verification - Use monitor._recvQ.popleft() for simple tests, not memory models

Version: 1.2 Last Updated: 2025-10-14 Maintained By: RTL Design Sherpa Project