RTL Design Sherpa

Reed-Solomon Board Validation Report Genesys 2 FPGA board evidence

October 2026

Table of Contents

1 Document Information	8

1.1 Revision History	8

1.2 Scope	9

1.3 Provenance	10

2 Scope and Claims	11

2.1 What this book is	11

2.2 What is claimed	12

2.3 What is not claimed	13

3 Images and Codec Profile	14

3.1 Image matrix	14

3.2 Codec profile	15

4 Harness Architecture	16

4.1 RTL inventory	16

4.2 Host and build inventory	18

4.2.1 Figure 2.1: Reed-Solomon AXIS datapath and register path.	18

4.3 Register path, clock, and reset	19

5 Campaigns and the Oracle Argument	20

5.1 Campaign inventory	20

5.2 Determinism and cross-solver agreement	21

5.3 The dual-solver loop as oracle	22

6 Testing Choices	23

6.1 Injector modes mapped to failure mechanisms	23

6.2 Post-encoder injection protects parity symbols	24

6.3 UART and the safety pair	25

6.4 Genesys 2 vs Nexys A7	26

6.5 AXIS and AXI4 flavors cover both integration styles	27

6.6 Deliberately not board-tested	28

7 Erasure Campaign	29

7.1 What erasure decoding adds	29

7.2 Board findings	30

7.3 TASK-005 boundary	31

8 Battery Results	32

8.1 Per-image campaign status	32

8.2 Correction boundary	33

8.2.1 Figure 4.1: Reed-Solomon correction boundary across all four images.	33

8.3 Decode cost	34

8.3.1 Figure 4.2: Reed-Solomon decode cost across all four images.	34

8.4 Sweep verdict table	35

8.5 Solver agreement	37

8.5.1 Figure 4.3: Reed-Solomon solver agreement matrix (riBM vs Euclid).	37

9 Soak Run Scheduled	38

9.1 Status at publication time	38

9.2 Target	39

9.3 What will be reported	40

10 Honest Limits	41

10.1 What the board evidence does not cover	41

10.2 Traceability	42

List of Figures

Figure 2.1: Reed-Solomon AXIS datapath and register path.	18

Figure 4.1: Reed-Solomon correction boundary across all four images.	33

Figure 4.2: Reed-Solomon decode cost across all four images.	34

Figure 4.3: Reed-Solomon solver agreement matrix (riBM vs Euclid).	37

List of Tables

Table 1: Table 0.1: Revision history.	8

Table 2: Table 1.1: Reed-Solomon Genesys 2 image matrix.	14

Table 3: Table 2.1: RTL blocks in the Reed-Solomon harness.	16

Table 4: Table 2.2: Host scripts and build flow.	18

Table 5: Table 3.1: Reed-Solomon board campaign inventory.	20

Table 6: Table 3.2: Reed-Solomon erasure boundary results.	30

Table 7: Table 4.1: Reed-Solomon 2026-10-05 battery per-image status.	32

Table 8: Table 4.2: Reed-Solomon sweep verdicts.	35

Table 9: Table 5.1: Traceability from claim to artifact.	42

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 1	Document Information

## 1.1	Revision History

Table 0.1: Revision history.

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-10-05 | RTL Design Sherpa | Initial board validation roll-up for the Reed-Solomon Genesys 2 harness. |

## 1.2	Scope

This book covers the RS(252,236) t=8 codec as exercised on the Digilent Genesys 2 board. It describes the harness, the campaign methodology, the 2026-10-05 battery, the erasure boundary finding, and the scheduled million-block soak.

## 1.3	Provenance

All timing and campaign results in this book come from stable/MANIFEST.md and stable/results/2026-10-05_battery/. The board used for the runs is Digilent Genesys 2 serial 200300B818A0 (Kintex-7 xc7k325t-2), programmed and measured with Vivado 2025.1.

# 2	Scope and Claims

## 2.1	What this book is

This report is the board’s statement about the RS(252,236) t=8 codec. The Genesys 2 harness runs the same campaigns in cocotb simulation and on real silicon, so when the board passes we have evidence that simulation matches hardware at 100 MHz with real I/O, real clocks, and real register traffic.

## 2.2	What is claimed

The 2026-10-05 battery demonstrates that four single-decoder bitstreams (two flavours times two solvers) pass the same deterministic campaign set:

- init, smoke, sweep, random, clusters, localized, and badblock all PASS on axis_ribm, axis_euclid, axi4_ribm, and axi4_euclid.

- The sweep shows e <= 8 corrected, e = 9 .. 18 uncorrectable, on every image.

- The random campaign recorded 64 runs x 16 blocks with 0 failing runs across all images.

- The matching verdict counts between riBM and Euclid images are themselves a cross-solver check.

That is board evidence that both key-equation solvers behave identically under the same workload and that the codec honours the correction boundary.

## 2.3	What is not claimed

This book does not replace the component DV matrix. It does not prove every possible error pattern or every smaller RS profile. The dual-decoder on-chip comparator is present in simulation but not in the production board matrix; agreement is proven by running identical deterministic campaigns on the two single-decoder images. The erasure f = 2t boundary is a known open issue (TASK-005) and is called out in Chapter 3 and Chapter 5. The scheduled soak has not completed at publication time; Chapter 4 says what it will report.

# 3	Images and Codec Profile

## 3.1	Image matrix

Four Genesys 2 bitstreams were built and kept in stable/reports/:

Table 1.1: Reed-Solomon Genesys 2 image matrix.

| Image | Solver | Bitstream | WNS (ns) | Failing endpoints | Notes |
| --- | --- | --- | --- | --- | --- |
| axis_ribm | riBM | rs_loop_genesys2_axis_ribm.bit | +1.072 | 0 | Streaming pipe. |
| axis_euclid | Euclid | rs_loop_genesys2_axis_euclid.bit | +0.875 | 0 | Streaming pipe. |
| axi4_ribm | riBM | rs_loop_genesys2_axi4_ribm.bit | +1.822 | 0 | Memory-to-memory chain. |
| axi4_euclid | Euclid | rs_loop_genesys2_axi4_euclid.bit | +1.431 | 0 | Memory-to-memory chain. |

All four images are timing-clean at 100 MHz on the k325t-2. The WNS values were read from the four stable/reports/genesys2_*/timing_summary.txt files.

## 3.2	Codec profile

The geometry is frozen in build-loop/rtl/rs_loop_cfg_pkg.sv:

- Code: RS(252,236), t=8, m=8.

- Symbols per beat: 4 (S=4), so a 32-bit beat carries four 8-bit symbols.

- Shortened from the natural 255-symbol RS block so that n=252 and k=236 are both multiples of the beat size. A full codeword is 63 beats; the message is 59 beats.

The shortening was deliberate: it makes the AXIS datapath line-rate at 63 cycles per block and keeps the AXI4 memory alignment clean. The host reads PROFILE to discover the geometry and TOPOLOGY to discover which datapath and solver are loaded.

# 4	Harness Architecture

## 4.1	RTL inventory

Table 2.1: RTL blocks in the Reed-Solomon harness.

| Module | File | Role |
| --- | --- | --- |
| uart_axil_bridge | projects/components/utility-ip/converters/rtl/uart_to_axil4/uart_axil_bridge.sv | UART 115200 8N1 to AXI4-Lite master. |
| bridge_rs_loop_axil | Generated fabric | 1-master x 3-slave AXI4-Lite/APB fabric. |
| rs_loop_genesys2_top / rs_loop_top | build-loop/rtl/rs_loop_genesys2_top.sv / build-loop/rtl/rs_loop_top.sv | Genesys 2 MMCM wrapper, or Nexys A7 100 MHz direct clock. |
| rs_loop_harness | build-loop/rtl/rs_loop_harness.sv | Codec loop, register decode, bandwidth meters, observers, verdict tallies. |
| rs_loop_cfg_pkg | build-loop/rtl/rs_loop_cfg_pkg.sv | Single source of geometry: RS(252,236), 4 symbols/beat. |
| rs_loop_regs | Generated from build-loop/rtl/rs_loop_regs.rdl | Host-visible CSR block. |
| axis4_master_pattern_gen | rtl/amba/shared/axis4_master_pattern_gen.sv | LFSR data source plus expected CRC-32, one packet per block. |
| axis4_slave_pattern_check | rtl/amba/shared/axis4_slave_pattern_check.sv | Regenerates the same LFSR pattern and compares beats per decoder. |
| rs_encoder_core | Component RS RTL | RS(252,236) encoder. |
| error_injector | Shared utility RTL | Post-encoder corruption: modes 0..7 plus erasure mark. |
| rs_decoder_core | Component RS RTL | Decoder with selectable KES_ALGO and erasure path. |
| rs_erasure_unit | Component RS RTL | Erasure locator / transform used when ERASURE_SUPPORT=1. |
| rs_axi4_pipeline | build-loop/rtl/rs_axi4_pipeline.sv | Memory-to-memory job chain for the AXI4 flavor. |
| axis4_intf_observer | Shared utility RTL | Four AXIS seam observer on window 0x20000. |
| axi4_intf_master_observer | Shared utility RTL | Four codec AXI4 master-port observer on window 0x10000. |

## 4.2	Host and build inventory

Table 2.2: Host scripts and build flow.

| Script / target | File | Role |
| --- | --- | --- |
| rs_env.py | bin/rs_env.py | Path anchor for shared host code. |
| run_smoke.py | bin/run_smoke.py | Campaign runner. |
| init / smoke / sweep / random / soak / erasure | bin/seq_*.py | Deterministic campaign sequences. |
| RsLoopDriver | build-loop/host/rs_loop.py | By-name register access and status collection. |
| rs_loop_programs | build-loop/host/rs_loop_programs.py | Shared authored-once programs. |
| host_rs_loop.py | build-loop/host/host_rs_loop.py | CLI front-end. |
| make -C build-loop bitstream | build-loop/Makefile | Vivado build; set RS_TARGET=genesys2. |
| bin/build_image_matrix.sh | bin/build_image_matrix.sh | Builds {axis,axi4} x {riBM,Euclid}. |

Datapath diagram

### 4.2.1	Figure 2.1: Reed-Solomon AXIS datapath and register path.

## 4.3	Register path, clock, and reset

The host register path is Python over pyserial to the UART, into uart_axil_bridge, through the generated bridge_rs_loop_axil 1x3 fabric, and out as three APB windows. Window 0 at 0x00000000 holds rs_loop_regs (BUILD_ID, SCRATCH, PROFILE, TOPOLOGY, INJ_CFG, etc.). Window 1 at 0x00010000 hosts the AXI4 master observer on AXI4 flavours and is a read-zero stub on AXIS. Window 2 at 0x00020000 is always live and hosts the AXIS seam observer.

On the Genesys 2 the 200 MHz LVDS system clock passes through IBUFDS into an MMCME2_BASE with VCO = 1200 MHz and CLKOUT0_DIVIDE_F = 12, producing the 100 MHz harness clock. The Nexys A7 top uses the on-board 100 MHz clock directly.

# 5	Campaigns and the Oracle Argument

## 5.1	Campaign inventory

Table 3.1: Reed-Solomon board campaign inventory.

| Campaign | What it proves | Determinism |
| --- | --- | --- |
| init | BUILD_ID, SCRATCH round-trip, PROFILE, TOPOLOGY. | Deterministic. |
| smoke | Bypass, clean, e=t, e=t+1, deterministic DEBUG walk. | Deterministic. |
| sweep | Exact correction counts for e = 0 .. 2t+2. | Deterministic. |
| random | Fresh seeds, mixed injector modes, 64 runs x 16 blocks. | Host-seeded; reproducible on demand. |
| clusters | Spatially correlated errors. | Deterministic combo matrix. |
| localized | Windowed errors. | Deterministic combo matrix. |
| badblock | Retention-tail / worn-block behaviour. | Deterministic combo matrix. |
| erasure | Marked-symbol runs at f = t, 2t, 2t+1. | Deterministic. |
| soak | Long-running mixed-mode stress; the million-block run. | Host-seeded; replayable. |

## 5.2	Determinism and cross-solver agreement

The host-side RNG seed defaults to 1. With that default, the random, soak, clusters, localized, and badblock campaigns are fully deterministic. Because the riBM and Euclid images run bit-identical workloads, the matching verdict counts between them are the cross-check. You do not need the on-chip comparator to know the two solvers agree; the campaigns prove it.

## 5.3	The dual-solver loop as oracle

The AXIS datapath in simulation feeds the same corrupted blocks to two rs_decoder_core instances, one with KES_ALGO=RIBM and one with KES_ALGO=EUCLID. Each decoder’s output is compared beat-by-beat against the generator’s regenerated LFSR pattern in its own axis4_slave_pattern_check, so a correction is judged by a reference that never saw the errors. A separate comparator then requires the two decoders to agree on every beat and on every block’s verdict. On the board images this comparator is absent by construction; the cross-check is the deterministic campaign battery.

# 6	Testing Choices

## 6.1	Injector modes mapped to failure mechanisms

The shared error_injector operates on 8-bit symbols. Mode 1 exact-count places exactly e errors per block at uniformly random distinct positions, which is what makes the e = t / e = t + 1 boundary sharp. Mode 2 burst and mode 3 rate model raw channel bit-error behaviour. Modes 4 clusters and 5 localized model spatially correlated faults. Mode 6 badblock models the tail of NAND retention. Mode 7 debug is the deterministic bring-up walk.

## 6.2	Post-encoder injection protects parity symbols

Errors are injected after the encoder, not in the generator. An error in the generator’s data would be encoded faithfully and would be invisible to the code. Placing the injector on the coded stream means parity symbols are corrupted too, which is the workload the decoder actually has to handle.

## 6.3	UART and the safety pair

The harness uses a plain UART instead of a proprietary debug core, so the host side is just pyserial and Python scripts. Board safety is a pair: board_lock.sh serializes programming flows by JTAG serial, and fpga_board.py reads the JTAG chain before programming to confirm the expected Genesys 2 serial (200300B818A0) is present with a device behind it.

## 6.4	Genesys 2 vs Nexys A7

The Kintex-7 XC7K325T-2 was chosen because it has room for the full harness plus observers and still closes timing with positive slack. The Nexys A7-100T flow is kept as a target option (RS_TARGET=nexys_a7_100t) but the Genesys 2 is the primary target for harness-class work.

## 6.5	AXIS and AXI4 flavors cover both integration styles

The AXIS flavor is a single streaming pipe; the AXI4 flavor replaces the middle with rs_axi4_pipeline, a memory-to-memory job chain over block RAM. Both use the same generator, checker, CSRs, and verdict logic, so a run reports itself identically whichever middle is built. Running the same deterministic campaign on both flavours is a free sanity check. The AXI4 flavor refuses a GEN_BLOCKS value that would exceed the job memories.

## 6.6	Deliberately not board-tested

The dual-decoder comparator with ENABLE_COMPARE=1 is the sim/DV domain. The soak campaign has a default time budget and is scheduled separately. The component DV matrix exercises the codec in ways the board harness does not replicate. The erasure f = 2t boundary is a known open issue, detailed in the next section.

# 7	Erasure Campaign

## 7.1	What erasure decoding adds

When INJ_CFG.mark is set, the injector’s hit mask rides the decoder in_erasure sideband. The decoder is then told which symbols were corrupted, so the correction bound doubles: the decoder can correct e errors plus f erasures whenever 2e + f <= 2t. For RS(252,236) t=8, that means:

- f = t = 8 erasures alone should correct.

- f = 2t = 16 erasures alone should correct.

- f = 2t + 1 = 17 erasures should be refused by inspection.

## 7.2	Board findings

The erasure sequence was run on the Genesys 2 board:

Table 3.2: Reed-Solomon erasure boundary results.

| f | Expected | Observed |
| --- | --- | --- |
| 8 (t) | Correct | Correct: 16/16 blocks corrected, A=B solver agreement. |
| 16 (2t) | Correct | Flagged uncorrectable: ok/corr/unc = 0/0/16. |
| 17 (2t+1) | Refuse by inspection | Refused: 16/16 blocks uncorrectable. |

## 7.3	TASK-005 boundary

The f = 2t case is the known open issue TASK-005 (vault/Tasks/projects/components/ecc-ip/reed-solomon/task/open/TASK-005.md): the B-stage t_zero bypass intends to pass f = 16, but the final verdict fires downstream. The f = t and f = 2t+1 paths are correct; the f = 2t path must be fixed in the component DV and then re-run on the board. This report documents the finding exactly as measured and does not fabricate a corrected result.

# 8	Battery Results

## 8.1	Per-image campaign status

All four images passed every campaign in the 2026-10-05 battery:

Table 4.1: Reed-Solomon 2026-10-05 battery per-image status.

| Image | Solver | Init | Smoke | Sweep | Random | Clusters | Localized | Badblock |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| axis_ribm | riBM | PASS | PASS | PASS | PASS | PASS | PASS | PASS |
| axis_euclid | Euclid | PASS | PASS | PASS | PASS | PASS | PASS | PASS |
| axi4_ribm | riBM | PASS | PASS | PASS | PASS | PASS | PASS | PASS |
| axi4_euclid | Euclid | PASS | PASS | PASS | PASS | PASS | PASS | PASS |

## 8.2	Correction boundary

fig_1_correction_boundary.png

### 8.2.1	Figure 4.1: Reed-Solomon correction boundary across all four images.

## 8.3	Decode cost

fig_2_decode_cost.png

### 8.3.1	Figure 4.2: Reed-Solomon decode cost across all four images.

## 8.4	Sweep verdict table

The sweep ran e = 0 .. 18 with 16 blocks per count on every image:

Table 4.2: Reed-Solomon sweep verdicts.

| e | Verdict | Blocks | Notes |
| --- | --- | --- | --- |
| 0 | Correct | 16 | Clean decode. |
| 1 | Correct | 16 | 16 symbols corrected. |
| 2 | Correct | 16 | 32 symbols corrected. |
| 3 | Correct | 16 | 48 symbols corrected. |
| 4 | Correct | 16 | 64 symbols corrected. |
| 5 | Correct | 16 | 80 symbols corrected. |
| 6 | Correct | 16 | 96 symbols corrected. |
| 7 | Correct | 16 | 112 symbols corrected. |
| 8 | Correct | 16 | 128 symbols corrected. |
| 9 | Uncorrectable | 16 | Past correction boundary. |
| 10 | Uncorrectable | 16 | Past correction boundary. |
| 11 | Uncorrectable | 16 | Past correction boundary. |
| 12 | Uncorrectable | 16 | Past correction boundary. |
| 13 | Uncorrectable | 16 | Past correction boundary. |
| 14 | Uncorrectable | 16 | Past correction boundary. |
| 15 | Uncorrectable | 16 | Past correction boundary. |
| 16 | Uncorrectable | 16 | Past correction boundary. |
| 17 | Uncorrectable | 16 | Past correction boundary. |
| 18 | Uncorrectable | 16 | Past correction boundary. |

## 8.5	Solver agreement

fig_5_solver_agreement.png

### 8.5.1	Figure 4.3: Reed-Solomon solver agreement matrix (riBM vs Euclid).

The matrix is entirely green with a Y in every cell. That means no image, at any injected error count from 0 to 18, disagreed with its sibling solver on the same deterministic workload. The absence of any red cell is the board evidence that the two key-equation solvers are consistent.

# 9	Soak Run Scheduled

## 9.1	Status at publication time

The million-block soak for Reed-Solomon was scheduled at publication time and had not yet completed. This chapter records the target and the reporting plan; it contains no fabricated numbers.

## 9.2	Target

- Code: RS(252,236) t=8, S=4.

- Board: Digilent Genesys 2 serial 200300B818A0.

- Target image: genesys2_axis_ribm or genesys2_axis_euclid; the AXIS flavour is used because the AXI4 job-memory cap makes long soaks impractical.

- Target volume: 1,000,000 blocks, matching the BCH soak depth.

- Host interface: UART 115200-8N1, so the effective block rate will be host-bound rather than datapath-bound.

## 9.3	What will be reported

When the soak completes, the following will be added to this chapter and the battery JSON artifact:

- Final block count, wall time, and blocks per second.

- Clean, corrected, and uncorrectable block counts.

- Total symbols corrected.

- Number of blocks with more than t errors and whether every one was flagged.

- Mis-decode count (must be 0 for a pass).

- The soak-timeline figure.

Until those numbers exist, the honest statement is that the run is scheduled.

# 10	Honest Limits

## 10.1	What the board evidence does not cover

- The soak is scheduled but not complete at publication time; Chapter 4 contains no soak numbers.

- The soak, once complete, will be statistical, not exhaustive. One million blocks is large, but it will not visit every possible error pattern.

- The erasure f = 2t boundary is an open issue (TASK-005). f = t corrects and f = 2t+1 refuses on the board; f = 2t is flagged uncorrectable even though the design intent says it should correct.

- The dual-decoder comparator with ENABLE_COMPARE=1 is sim-only on the board matrix. Solver agreement on the board is proven by running identical deterministic campaigns on the single-decoder images.

- BCH has no erasure path by design; that comparison is RS-only.

- The board validates only the RS(252,236) t=8, S=4 profile. Other profiles are covered in simulation and component DV.

- The component DV matrix exercises the codec in ways the board harness does not replicate.

## 10.2	Traceability

Every quantitative claim in this book can be traced to an artifact under stable/:

Table 5.1: Traceability from claim to artifact.

| Claim | Artifact |
| --- | --- |
| Four images timing-clean at 100 MHz | stable/reports/genesys2_{axis_ribm,axis_euclid,axi4_ribm,axi4_euclid}/timing_summary.txt, all WNS positive, 0 failing endpoints. |
| 7/7 campaigns PASS on all images | stable/results/2026-10-05_battery/battery.json and FINDINGS.md. |
| e <= 8 corrected, e > 8 uncorrectable | stable/results/2026-10-05_battery/battery.json, sweep array. |
| Solver agreement matrix all green | stable/results/2026-10-05_battery/fig_5_solver_agreement.png. |
| f = 2t erasure boundary open | vault/Tasks/projects/components/ecc-ip/reed-solomon/task/open/TASK-005.md. |