RTL Design Sherpa

APB Crossbar Hardware Architecture Specification 1.0

January 3, 2026

Table of Contents

Apbx Xbar Has Index	19

APB Crossbar Hardware Architecture Specification Index	20

Document Organization	21

Front Matter	21

Chapter 1: Introduction	21

Chapter 2: System Overview	21

Chapter 3: Architecture	21

Chapter 4: Interfaces	21

Chapter 5: Performance	21

Chapter 6: Integration	21

Related Documentation	23

Document Information	24

APB Crossbar Hardware Architecture Specification	24

Document Purpose	25

References	26

Terminology	27

Revision History	28

Purpose and Scope	29

Document Purpose	29

Component Overview	30

Document Scope	31

In Scope	31

Out of Scope	31

Specification Level	32

Document Conventions	33

Notation	33

Signal Naming	33

Parameter Naming	33

Diagrams	34

Block Diagrams	34

Address Notation	34

Table Captions	35

Requirement References	36

Definitions and Acronyms	37

Acronyms	37

Terms	38

APB Protocol Signals	39

Use Cases	40

Primary Use Cases	40

UC-1: Simple Peripheral Interconnect (1x4)	40

UC-2: Multi-Master System (2x4)	40

UC-3: Protocol Conversion (1x1)	40

Variant Selection Guide	41

Key Features	42

Feature Summary	42

F1: Parametric MxN Configuration	42

F2: Automatic Address-Based Routing	42

F3: Round-Robin Arbitration	42

F4: Back-to-Back Transactions	42

F5: Proven Building Blocks	43

Feature Comparison	44

Design Philosophy	45

Composition Over Complexity	45

Scalability	45

System Context	46

System Context Diagram	46

Figure 2.1: System Context Diagram	46

Typical Integration Topology	47

Master-Side Connections	47

Slave-Side Connections	47

Interface Boundaries	48

Input Boundary (Master-Side)	48

Output Boundary (Slave-Side)	48

Clock and Reset	48

System Integration Considerations	49

Address Map Planning	49

Multi-Master Considerations	49

Block Diagram	50

Top-Level Architecture	50

Figure 3.1: APB Crossbar Block Diagram	50

Functional Blocks	51

Master-Side Protocol Conversion	51

Internal Crossbar Fabric	51

Slave-Side Protocol Conversion	51

Module Hierarchy	52

Data Path Width	53

Data Flow	54

Transaction Flow Overview	54

Figure 3.2: Transaction Data Flow	54

Write Transaction Flow	55

Stage 1: Master Request	55

Stage 2: Protocol Conversion (Master-Side)	55

Stage 3: Address Decode	55

Stage 4: Arbitration (if needed)	55

Stage 5: Protocol Conversion (Slave-Side)	55

Stage 6: Response Return	55

Read Transaction Flow	56

Timing Summary	57

Back-to-Back Transactions	58

Address Mapping	59

Address Map Structure	59

Figure 3.3: Address Mapping Structure	60

Default Address Map	61

Address Decode Logic	62

Example Decode	62

Customizing Base Address	63

Address Map Constraints	64

Fixed Slave Region Size	64

Maximum Configuration	64

Address Width Requirement	64

Master-Side APB Interfaces	65

Interface Overview	65

Signal Definition	66

APB5 Sideband (mixed-version variants only)	66

Signal Descriptions	68

PSEL (Input)	68

PENABLE (Input)	68

PADDR (Input)	68

PWRITE (Input)	68

PWDATA (Input)	68

PSTRB (Input)	68

PPROT (Input)	68

PRDATA (Output)	68

PSLVERR (Output)	68

PREADY (Output)	69

Transaction Timing	70

Uncontended Write (No Arbitration Wait) – PHASE SHAPE ONLY, not to scale:	70

Contended Transaction (Arbitration Wait)	70

Slave-Side APB Interfaces	71

Interface Overview	71

Signal Definition	72

APB5 Sideband (mixed-version variants only)	72

Signal Behavior	74

Output Signals (Crossbar to Slave)	74

Input Signals (Slave to Crossbar)	74

Slave Response Handling	75

Wait States	75

Error Response	75

Multi-Slave Considerations	76

Address Overlap Prevention	76

Independent Operation	76

Clock and Reset	77

Clock Interface	77

Signal Definition	77

Clock Requirements	77

Reset Interface	78

Signal Definition	78

Reset Behavior	78

Reset Safe Values	78

Clock-Reset Relationship	79

Reset Assertion	79

Reset Deassertion	79

Reset Sequence Diagram	79

Integration Notes	80

Clock Domain Crossing	80

Clock Gating	80

Throughput Characteristics	81

Maximum Throughput	81

Single Master	81

Multi-Master (Uncontended)	81

Multi-Master (Contended)	81

Arbitration Handoff	83

Grant Persistence	83

Back-to-Back Timing	83

Throughput Factors	84

Factors That Reduce Throughput	84

Optimal Usage Pattern	84

Latency Analysis	85

Transaction Latency	85

Uncontended Access	85

Contended Access	86

Latency Breakdown	88

Forward Path (Master to Slave)	88

Response Path (Slave to Master)	89

Latency Timing Diagram	90

Best Case (No Waits, No Contention)	90

Worst Case (Contention + Slave Wait States)	90

Latency Optimization	91

Design Recommendations	91

Resource Estimates	92

FPGA Resource Usage	92

Pre-Generated Variants	92

Scaling Factors	92

Lines of Code	94

Power Considerations	95

Static Power	95

Dynamic Power	95

Power Optimization	95

Area Comparison	96

Custom Configuration Estimation	97

System Requirements	98

Hardware Requirements	98

Clock and Reset	98

APB Compliance	98

Software Requirements	99

Address Map Configuration	99

Driver Considerations	99

Constraints	100

Address Map Constraints	100

Transaction Constraints	100

Integration Checklist	101

Pre-Integration	101

Post-Integration	101

Parameter Configuration	102

Module Parameters	102

Core Parameters	102

Derived Parameters	103

Parameter Usage Examples	104

Default Configuration	104

Custom Base Address	104

64-bit Data Width	104

Wide Address	104

Parameter Validation	105

Address Width Check	105

Data Width Constraints	105

Custom Generation Parameters	106

Verification Strategy	107

Pre-Verified Component	107

Test Categories	108

Basic Connectivity	108

Address Decode Verification	108

Arbitration Testing	108

Stress Testing	108

APB5 Sideband Gating (mixed variant)	108

Timing and Protocol Compliance	108

Integration Testing Recommendations	109

Level 1: Basic Integration	109

Level 2: Functional Integration	109

Level 3: System Integration	109

Verification Collateral	110

Available Test Infrastructure	110

Running Verification	110

Known Limitations	111

Not Tested	111

Related Documentation	112

APB Crossbar Micro-Architecture Specification Index	113

Document Organization	114

Main Documentation	114

Chapter 1: Architecture	115

Chapter 2: Address Decode and Arbitration	116

Chapter 3: RTL Generator	117

Quick Navigation	118

For New Users	118

For Integration	118

Common Questions	118

Visual Assets	119

Architecture Diagrams	119

Timing Diagrams	119

Component Overview	120

Key Features	120

Pre-Generated Variants	120

Design Philosophy	120

Related Documentation	122

Companion Specifications	122

Project-Level	122

Test Infrastructure	122

RTL	122

Version History	123

Product Requirements Document (PRD)	124

APB Crossbar Generator	124

1. Executive Summary	125

1.1 Quick Stats	125

1.2 Project Goals	125

2. Key Design Principles	126

2.1 Architecture Philosophy	126

2.2 Design Trade-offs	126

3. Architecture Overview	127

3.1 Top-Level Block Diagram	127

3.2 Address Mapping	128

3.3 Arbitration Strategy	129

4. Available Variants	131

4.1 Pre-Generated Modules	131

4.2 Wrapper Modules	131

5. Functional Requirements	133

FR-1: Arbitrary MxN Configuration	133

FR-2: Round-Robin Arbitration	133

FR-3: Address-Based Routing	133

FR-4: Back-to-Back Transactions	133

FR-5: Configurable Address Map	133

6. Interface Specifications	134

6.1 Master-Side Interface (APB Slave)	134

6.2 Slave-Side Interface (APB Master)	134

7. Parameter Configuration	135

8. Generator Usage	136

8.1 Pre-Generated Variants	136

8.2 Custom Generation	136

8.3 Generator Options	136

9. Performance Characteristics	137

9.1 Latency	137

9.2 Throughput	137

9.3 Resource Utilization (Estimated)	137

10. Verification Status	138

10.1 Test Coverage	138

10.2 Test Methodology	138

11. Integration Guide	139

11.1 Simple Integration (1to4)	139

11.2 Multi-Master Integration (2to4)	139

12. Design Constraints	141

12.1 Limitations	141

12.2 Assumptions	141

13. Future Enhancements	142

13.1 Potential Features	142

13.2 Generator Improvements	142

14. References	143

14.1 Internal Documentation	143

14.2 External Standards	143

14.3 Related Components	143

Claude Code Guide: APB Crossbar Component	144

Quick Context	145

Critical Rules for This Component	146

Rule #0: This is a GENERATOR, Not Just Modules	146

Rule #1: Address Map is Fixed Per-Slave	146

Rule #2: Know the Pre-Generated Variants	146

Rule #3: Generator Syntax	147

Architecture Quick Reference	148

Module Organization	148

Block Diagram (2×4 Example)	148

Common User Questions and Responses	150

Q: “How do I connect a CPU to 4 peripherals?”	150

Q: “I need CPU and DMA to access peripherals. Which crossbar?”	150

Q: “What if I need 3 masters and 8 slaves?”	151

Q: “How does arbitration work?”	151

Q: “Can I change the address map?”	152

Q: “How do I run tests?”	152

Integration Patterns	154

Pattern 1: Simple Peripheral Interconnect	154

Pattern 2: Multi-Master System	154

Pattern 3: Hierarchical Interconnect	155

Anti-Patterns to Catch	156

Anti-Pattern 1: Generating When Pre-Generated Exists	156

Anti-Pattern 2: Assuming Custom Per-Slave Sizes	156

Anti-Pattern 3: Not Mentioning Address Map	156

Anti-Pattern 4: Forgetting About Wrappers	156

Debugging Workflow	158

Issue: Address Not Routing Correctly	158

Issue: Arbitration Not Fair	158

Issue: Back-to-Back Transactions Stalling	158

Quick Commands	159

Remember	160

List of Figures

Figure 2.1: System Context Diagram	46

Figure 3.1: APB Crossbar Block Diagram	50

Figure 3.2: Transaction Data Flow	54

Figure 3.3: Address Mapping Structure	60

List of Tables

Table 1: Reference Documents	26

Table 2: Terminology and Definitions	27

Table 3: Document Revision History	28

Table 4: HAS vs MAS Coverage Comparison	32

Table 5: Signal Naming Conventions	33

Table 6: Parameter Naming Conventions	33

Table 7: Acronym Definitions	37

Table 8: Term Definitions	38

Table 9: APB Protocol Signal Definitions	39

Table 10: Variant Selection Guide	41

Table 11: Pre-Generated Variant Comparison	44

Table 12: Typical Master Connections	47

Table 13: Typical Slave Connections (1to4 example)	47

Table 14: Data Path Width Parameters	53

Table 15: Transaction Timing Summary	57

Table 16: Default Address Map (4-slave example)	61

Table 17: Address Decode Example	62

Table 18: Master Port Signal Definition	66

Table 19: APB5 Master Sideband Signals	66

Table 20: Slave Port Signal Definition	72

Table 21: APB5 Slave Sideband Signals	72

Table 22: Clock Signal	77

Table 23: Reset Signal	78

Table 24: Reset Safe Values	78

Table 25: Single-Master-Active Throughput, by Variant Class	81

Table 26: Contended Access Throughput	82

Table 27: Throughput Reduction Factors	84

Table 28: Uncontended Transaction Latency	85

Table 29: Measured Timing, Always-Ready Slave	86

Table 30: Arbitration Latency	86

Table 31: Forward Path Latency	88

Table 32: Response Path Latency	89

Table 33: Latency Optimization Recommendations	91

Table 34: Resource Scaling Factors	92

Table 35: Source Code Metrics	94

Table 36: Power Optimization Techniques	95

Table 37: Relative Area Comparison	96

Table 38: Clock and Reset Requirements	98

Table 39: APB Compliance Requirements	98

Table 40: Driver Considerations	99

Table 41: Address Map Constraints	100

Table 42: Transaction Constraints	100

Table 43: Core Parameter Definition	102

Table 44: Derived Parameters	103

Table 45: Valid Data Width Configurations	105

Table 46: Generator Command Line Options	106

Table 47: Level 1 Integration Tests	109

Table 48: Level 2 Integration Tests	109

Table 49: Level 3 Integration Tests	109

Table 50: Test Infrastructure	110

Table 51: Known Limitations	111

# Apbx Xbar Has Index

Generated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Hardware Architecture Specification Index

Version: 1.1 Date: 2026-10-12 Purpose: High-level hardware architecture specification for APB Crossbar component

## Document Organization

Note: All chapters are linked below; this index doubles as the manifest for automated document generation.

### Front Matter

- Document Information

### Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### Chapter 3: Architecture

- Block Diagram

- Data Flow

- Address Mapping

### Chapter 4: Interfaces

- Master-Side APB Interfaces

- Slave-Side APB Interfaces

- Clock and Reset

### Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resource Estimates

### Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

## Related Documentation

- APB Crossbar MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Document Information

## APB Crossbar Hardware Architecture Specification

Document Number: APB-XBAR-HAS-001 Version: 1.1 Status: Production Ready Classification: Open Source - MIT License (matches the SPDX tags on every RTL file)

## Document Purpose

This Hardware Architecture Specification (HAS) is the high-level architectural view of the APB Crossbar: system-level design, external interfaces, performance characteristics, and integration requirements. Internal implementation specifics are deliberately left to the MAS.

Target Audience: - System architects evaluating APB Crossbar for SoC integration - Hardware engineers planning peripheral interconnect design - Software engineers developing drivers and firmware - Verification engineers planning system-level testing

Companion Documents: - APB Crossbar Micro-Architecture Specification (MAS) - Detailed block-level implementation - APB Crossbar Product Requirements Document (PRD) - Requirements and rationale

## References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | APB Crossbar MAS v1.1 | Micro-Architecture Specification |
| [REF-2] | APB Crossbar PRD | Product Requirements Document |
| [REF-3] | ARM AMBA APB | APB Protocol Specification (IHI 0024C) |

## Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| APB | Advanced Peripheral Bus - ARM AMBA low-power peripheral bus |
| Arbiter | Logic that selects between competing requests using round-robin |
| Crossbar | NxM interconnect fabric connecting multiple masters to multiple slaves |
| Master | APB initiator device that initiates read/write transactions |
| PSEL | APB peripheral select signal indicating slave selection |
| PENABLE | APB enable signal indicating data phase of transaction |
| PREADY | APB ready signal indicating slave response completion |
| Slave | APB target device that responds to master transactions |
| Transaction | Complete APB read or write operation (setup + data phase) |

## Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | seang | Initial HAS release |
| 1.1 | 2026-10-12 | seang | Refresh: update LOC counts, verification status, and generator CLI options |

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Purpose and Scope

## Document Purpose

This Hardware Architecture Specification (HAS) describes the APB Crossbar at the system integration level, with enough detail for:

- System architects to evaluate the component for SoC integration

- Hardware engineers to plan interconnect topology and address mapping

- Software engineers to understand register access paths and address spaces

- Verification engineers to develop system-level test plans

## Component Overview

The APB Crossbar is a parametric MxN interconnect fabric connecting multiple APB masters to multiple APB slaves:

- Automatic address-based routing to the appropriate slave

- Fair round-robin arbitration when multiple masters access the same slave

- Back-to-back transactions with no master-side idle cycles (they do not overlap inside the fabric – 10 pclk cycles each PREADY-to-PREADY with one master, 11 on an arbitrated variant, where the round-robin grant is registered; see 5.1)

- Scalable configuration from 1x1 to 16x16

## Document Scope

### In Scope

- System-level architecture and block organization

- External interface specifications (all APB signals)

- Address mapping and routing strategy

- Arbitration policy and fairness guarantees

- Performance characteristics (latency, throughput)

- Integration requirements and parameters

### Out of Scope

- Internal RTL implementation details (see MAS)

- Detailed timing diagrams for internal signals

- Synthesis and implementation guidance

- Test infrastructure details

## Specification Level

This HAS is a black-box view of the APB Crossbar:

HAS vs MAS Coverage Comparison

| Aspect | HAS Coverage | MAS Coverage |
| --- | --- | --- |
| External ports | Complete | Complete |
| Functional behavior | Complete | Complete |
| Internal architecture | Overview only | Detailed |
| FSM states | - | Complete |
| Timing paths | Interface only | All paths |
| RTL structure | - | Complete |

Next: Document Conventions

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Document Conventions

## Notation

### Signal Naming

Signal Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| m<i>_apb_* | Master-side APB port i – BOTH directions: requests in (PSEL, PADDR, …), responses out (PRDATA, PREADY, PSLVERR) | m0_apb_PSEL (in), m0_apb_PREADY (out) |
| s<j>_apb_* | Slave-side APB port j – BOTH directions: requests out, responses in | s2_apb_PSEL (out), s2_apb_PREADY (in) |
| P* | APB protocol signal prefix | PADDR, PWRITE, PRDATA |
| <i>, <j> | Master/slave index placeholder | m0, s3 |

### Parameter Naming

Parameter Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| UPPER_CASE | Module parameters | ADDR_WIDTH, DATA_WIDTH |
| NUM_* | Count parameters | NUM_MASTERS, NUM_SLAVES |
| *_WIDTH | Bit width parameters | DATA_WIDTH = 32 |
| BASE_* | Base address parameters | BASE_ADDR = 0x10000000 |

## Diagrams

### Block Diagrams

Block diagrams here use:

- Rectangles for functional blocks

- Arrows showing data flow direction

- Dashed lines for configuration/control paths

- Labels on connections indicating signal groups

### Address Notation

Addresses appear in hexadecimal with the 0x prefix:

- 0x1000_0000 - Underscores separate 16-bit groups for readability

- [31:0] - Bit range notation (MSB:LSB)

- 64KB - Size notation using standard abbreviations

## Table Captions

Tables are numbered sequentially within each chapter. A colon (:) followed by the caption text appears below each table.

## Requirement References

Requirements from the PRD are referenced as:

- FR-n - Functional Requirement number n

- PR-n - Performance Requirement number n

- IR-n - Interface Requirement number n

Next: Definitions and Acronyms

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Definitions and Acronyms

## Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| CPU | Central Processing Unit |
| DMA | Direct Memory Access |
| FSM | Finite State Machine |
| GPIO | General Purpose Input/Output |
| HAS | Hardware Architecture Specification |
| I2C | Inter-Integrated Circuit |
| LOC | Lines of Code |
| LUT | Lookup Table (FPGA resource) |
| MAS | Micro-Architecture Specification |
| PRD | Product Requirements Document |
| RTL | Register Transfer Level |
| SoC | System on Chip |
| SPI | Serial Peripheral Interface |
| UART | Universal Asynchronous Receiver/Transmitter |

## Terms

Term Definitions

| Term | Definition |
| --- | --- |
| Address Decode | Logic that determines which slave should receive a transaction based on address |
| Arbitration | Process of selecting one request when multiple masters compete for the same slave |
| Back-to-back | Consecutive transactions with no idle cycles between them |
| Crossbar | Interconnect topology allowing any master to communicate with any slave |
| Grant Persistence | Maintaining arbitration grant through entire transaction duration |
| Master | Device that initiates APB transactions (CPU, DMA, etc.) |
| Round-Robin | Arbitration scheme that rotates priority among requesters |
| Slave | Device that responds to APB transactions (peripherals) |
| Starvation | Condition where a requester is indefinitely denied access |
| Transaction | Complete APB read or write operation including setup and data phases |
| Back-to-Back | Consecutive transactions with no master-side idle cycles. Note this crossbar does not OVERLAP them, and cadence is one cycle longer than single-transfer latency: 10 PREADY-to-PREADY against 9 SETUP-to-PREADY with one master, 11 against 10 when arbitrated (see 5.2) |

## APB Protocol Signals

APB Protocol Signal Definitions

| Signal | Direction | Description |
| --- | --- | --- |
| PCLK | Input | APB clock (all signals synchronous to rising edge) |
| PRESETn | Input | Active-low reset. Assertion is asynchronous in every build; deassertion must be synchronised to PCLK (see 4.3) |
| PADDR | Master to Slave | Address bus |
| PSEL | Master to Slave | Peripheral select (one per slave) |
| PENABLE | Master to Slave | Enable signal (high during data phase) |
| PWRITE | Master to Slave | Write/read direction (1=write, 0=read) |
| PWDATA | Master to Slave | Write data bus |
| PSTRB | Master to Slave | Write strobes (byte enables) |
| PPROT | Master to Slave | Protection attributes |
| PRDATA | Slave to Master | Read data bus |
| PREADY | Slave to Master | Ready signal (can extend transaction) |
| PSLVERR | Slave to Master | Slave error response |

Next: Chapter 2: System Overview

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Use Cases

## Primary Use Cases

### UC-1: Simple Peripheral Interconnect (1x4)

Scenario: Single CPU accessing multiple peripherals

A microcontroller reaches UART, GPIO, Timer, and SPI peripherals through a single APB master interface.

Configuration: - 1 Master (CPU) - 4 Slaves (UART, GPIO, Timer, SPI) - Variant: apbx_xbar_1to4

Address Map:

0x1000_0000 - 0x1000_FFFF : UART  (64KB)
0x1001_0000 - 0x1001_FFFF : GPIO  (64KB)
0x1002_0000 - 0x1002_FFFF : Timer (64KB)
0x1003_0000 - 0x1003_FFFF : SPI   (64KB)

### UC-2: Multi-Master System (2x4)

Scenario: CPU and DMA both accessing peripherals

An SoC where both the CPU and a DMA controller share the same peripherals.

Configuration: - 2 Masters (CPU, DMA) - 4 Slaves (peripherals) - Variant: apbx_xbar_2to4

Key Behavior: - Round-robin arbitration when both masters access same peripheral - No master starvation guaranteed - Back-to-back transactions on uncontended access (10 pclk cycles each for M = 1, 11 when arbitrated; they do not overlap – see 5.1)

### UC-3: Protocol Conversion (1x1)

Scenario: APB timing boundary or clock domain preparation

Insert the APB crossbar as a timing stage, or as preparation for clock domain crossing.

Configuration: - 1 Master - 1 Slave - Variant: apbx_xbar_1to1

Benefits: - Isolates master from slave timing - Consistent transaction buffering - Lowest resource footprint of the family

## Variant Selection Guide

Variant Selection Guide

| Use Case | Recommended Variant | Rationale |
| --- | --- | --- |
| Simple peripheral bus | apbx_xbar_1to4 | Most common SoC configuration |
| CPU + DMA system | apbx_xbar_2to4 | Fair access for both masters |
| Timing isolation | apbx_xbar_1to1 | Registered command and response paths on a single pairing |
| Multi-master arbitration | apbx_xbar_2to1 | Focus on arbitration only |
| Custom topology | Generator | Any MxN configuration |

Next: Key Features

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Key Features

## Feature Summary

### F1: Parametric MxN Configuration

- Any combination of M masters and N slaves

- Pre-generated variants for common configurations (1x1, 2x1, 1x4, 2x4, and the mixed-version 2x2)

- Python generator for custom configurations up to 16x16

- A single GENERATOR serves all configurations – not a single RTL source. Each variant is a distinct file with its M×N baked into the module name and port list; only ADDR_WIDTH, DATA_WIDTH, STRB_WIDTH and BASE_ADDR are elaboration parameters (see 6.2). The parameterized-core reading was true of the retired apbx_xbar_thin and has not been true since 2026-08-27.

### F2: Automatic Address-Based Routing

- Parallel address decode for all slaves

- Fixed 64KB address space per slave

- Configurable base address via BASE_ADDR parameter

- Zero-decode-latency routing

Address Calculation:

slave_index = ((PADDR - BASE_ADDR) >> 16)[$clog2(S)-1:0]

### F3: Round-Robin Arbitration

- Independent arbiter per slave

- Fair rotation of master priority

- No master starvation guaranteed

- Grant persistence through transaction completion

### F4: Back-to-Back Transactions

- Accepted with no master-side idle cycles required

- Grant held for the duration of one transaction, released at the response handshake

- They do NOT overlap inside the fabric: apb4_slave is one-command-at-a-time. Sustained cadence measures 10 pclk cycles PREADY-to-PREADY, one more than the 9-cycle SETUP-to-PREADY single transfer, the extra cycle being the next transfer’s mandatory SETUP phase (see 5.1/5.2)

### F5: Proven Building Blocks

- Built from production-tested apb4_slave.sv and apb4_master.sv modules

- No new protocol logic - pure composition

- Each component independently verified

## Feature Comparison

Pre-Generated Variant Comparison

| Feature | apbx_xbar_1to1 | apbx_xbar_2to1 | apbx_xbar_1to4 | apbx_xbar_2to4 | apbx_xbar_2to2_mixed |
| --- | --- | --- | --- | --- | --- |
| Masters | 1 | 2 | 1 | 2 | 2 |
| Slaves | 1 | 1 | 4 | 4 | 2 |
| Arbitration | No | Yes | No | Yes | Yes |
| Address Decode | No | No | Yes | Yes | Yes |
| APB5 sideband | No | No | No | No | Yes – per-port version gating |

All five are generator output. LOC per variant is in HAS ch05_performance/03_resources.md, the single source of truth for it – the row that used to sit in this table drifted stale within two days of being reconciled. apbx_xbar_thin, a hand-written parameterized core that once occupied a sixth column here, was retired and deleted on 2026-08-27 along with its tests, formal harnesses and testplan; nothing in the tree depends on it.

## Design Philosophy

### Composition Over Complexity

The crossbar is a composition of well-tested building blocks:

- APB Slaves (M instances) - Convert incoming APB to internal cmd/rsp

- Arbiters (N instances) - Select winning master per slave

- Address Decode - Route commands to appropriate slave

- Response Routing - Return responses to originating masters

- APB Masters (N instances) - Convert internal cmd/rsp back to APB

### Scalability

Resource usage scales predictably: - M x N routing paths - N independent arbiters - Linear growth with configuration size

Next: System Context

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# System Context

## System Context Diagram

The APB Crossbar in a typical SoC context:

### Figure 2.1: System Context Diagram

System Context Diagram

## Typical Integration Topology

### Master-Side Connections

The crossbar’s master ports face upstream APB masters:

Typical Master Connections

| Master Type | Typical Source | Connection Point |
| --- | --- | --- |
| CPU | AHB-to-APB Bridge | m0_apb_* |
| DMA | DMA Controller APB Port | m1_apb_* |
| Debug | Debug Access Port | m2_apb_* (if present) |

### Slave-Side Connections

The crossbar’s slave ports face downstream APB peripherals:

Typical Slave Connections (1to4 example)

| Slave Type | Address Offset | Connection Point |
| --- | --- | --- |
| UART | +0x0_0000 | s0_apb_* |
| GPIO | +0x1_0000 | s1_apb_* |
| Timer | +0x2_0000 | s2_apb_* |
| SPI | +0x3_0000 | s3_apb_* |

## Interface Boundaries

### Input Boundary (Master-Side)

Each master port presents a complete APB slave interface:

- Receives PSEL, PENABLE, PADDR, PWRITE, PWDATA, PSTRB, PPROT

- Returns PRDATA, PSLVERR, PREADY

### Output Boundary (Slave-Side)

Each slave port presents a complete APB master interface:

- Drives PSEL, PENABLE, PADDR, PWRITE, PWDATA, PSTRB, PPROT

- Receives PRDATA, PSLVERR, PREADY

### Clock and Reset

Single clock domain: - pclk - APB clock (all signals synchronous) - presetn - Active-low asynchronous reset

## System Integration Considerations

### Address Map Planning

The fixed 64KB per-slave allocation requires planning:

- Peripheral fit: Ensure each peripheral needs <= 64KB address space

- Base address: Set BASE_ADDR to avoid conflicts with other subsystems

- Address width: ADDR_WIDTH must accommodate BASE_ADDR + (NUM_SLAVES x 64KB)

### Multi-Master Considerations

When using multiple masters:

- Arbitration overhead: a contended access waits a FULL transaction, not 1-2 cycles. The grant is held to the response handshake (WAIT_GNT_ACK(1)), so each master ahead in the round-robin costs ~11 cycles (the arbitrated back-to-back period). Uncontended, the registered grant still costs exactly 1 cycle – see HAS 5.2.

- Priority: All masters have equal priority (round-robin)

- Bandwidth: Shared access reduces per-master bandwidth

Next: Chapter 3: Architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Block Diagram

## Top-Level Architecture

The APB Crossbar has three functional layers:

### Figure 3.1: APB Crossbar Block Diagram

Block Diagram

## Functional Blocks

### Master-Side Protocol Conversion

Component: apb4_slave[M] instances (one per master)

Each APB slave instance: - Accepts incoming APB transactions from a master - Converts the APB protocol to the internal command/response format - Buffers the transaction - Generates the response timing

Interface: - Input: Complete APB slave signals from external master - Output: Internal cmd/rsp bus to crossbar fabric

### Internal Crossbar Fabric

Components: - Address Decode - Parallel decode logic - Per-Slave Arbiters - Round-robin arbiter per slave (N instances) - Response Routing - Combinational response mux (the registers on this path live in the boundary IP’s skid buffers, not the mux)

The crossbar fabric: 1. Decodes transaction address to determine target slave 2. Arbitrates when multiple masters request the same slave 3. Routes commands to the selected slave 4. Routes responses back to the originating master

### Slave-Side Protocol Conversion

Component: apb4_master[N] instances (one per slave)

Each APB master instance: - Accepts internal cmd/rsp from crossbar fabric - Generates compliant APB transactions to the slave - Handles wait states via PREADY - Propagates error responses via PSLVERR

Interface: - Input: Internal cmd/rsp bus from crossbar fabric - Output: Complete APB master signals to external slave

## Module Hierarchy

apbx_xbar_MxN
+-- apb4_slave[0..M-1]          # Master-side conversion
+-- addr_decode                 # Parallel address decode
+-- arbiter[0..N-1]            # Per-slave round-robin
+-- response_mux               # Response routing
+-- apb4_master[0..N-1]         # Slave-side conversion

## Data Path Width

Data path widths are consistent throughout:

Data Path Width Parameters

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | 32 | 18-64 (decoding variants: the slave-index part-select reads bit 17); 1-64 for 1to1/2to1 | Address bus width |
| DATA_WIDTH | 32 | 8-64 | Data bus width |
| STRB_WIDTH | 4 | 1-8 | Byte strobe width (DATA_WIDTH/8) |

Next: Data Flow

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Data Flow

## Transaction Flow Overview

### Figure 3.2: Transaction Data Flow

Data Flow Diagram

## Write Transaction Flow

A write transaction passes through six stages:

### Stage 1: Master Request

The external master initiates an APB write: 1. Master asserts PSEL and PADDR with target address 2. Master sets PWRITE=1 and provides PWDATA 3. Next cycle: Master asserts PENABLE

### Stage 2: Protocol Conversion (Master-Side)

The apb4_slave module converts to internal format: 1. Captures transaction parameters (address, data, write) 2. Generates internal command 3. Routes to address decode

### Stage 3: Address Decode

Parallel decode determines target slave:

slave_index = offset[16 +: $clog2(S)]      // offset = PADDR - BASE_ADDR

A part-select, not a mask. & (S-1) appeared here until 2026-08-30 and is only equivalent when S is a power of two – for a generated 3-slave variant the mask would make slave 1 unreachable. The generator emits a part-select (slave_sel = m0_cmd_offset[13:12] for S = 3 at the generator’s default 4KB window; the shipped 64KB variants read offset[16 +: $clog2(S)]), and the sibling formulas in ch02/ch03 always did.

### Stage 4: Arbitration (if needed)

If multiple masters target the same slave: 1. Arbiter selects one master (round-robin priority) 2. Other masters wait (PREADY low) 3. Grant held until transaction completes

### Stage 5: Protocol Conversion (Slave-Side)

The apb4_master module generates the APB transaction: 1. Asserts PSEL to selected slave 2. Provides PADDR, PWRITE, PWDATA 3. Asserts PENABLE next cycle 4. Waits for slave PREADY

### Stage 6: Response Return

The response flows back to the originating master: 1. Slave asserts PREADY (with optional PSLVERR) 2. apb4_master captures response 3. Response routed to correct apb4_slave 4. apb4_slave asserts PREADY to original master

## Read Transaction Flow

Read transactions follow the same flow with these differences:

- PWRITE=0 indicates read operation

- No PWDATA on forward path

- PRDATA returned on response path

## Timing Summary

Transaction Timing Summary

| Transaction Phase | Clock Cycles | Notes |
| --- | --- | --- |
| Setup phase | 1 | PSEL asserted, PENABLE low |
| Data phase | 1+ | PENABLE high, wait for PREADY |
| Decode | 0 | Combinational (parallel decode) |
| Arbitration | 0 or 1 | 0 when M = 1 – the generator emits no arbiter at all. 1 when M > 1, even uncontended, because the grant is REGISTERED |

## Back-to-Back Transactions

The crossbar accepts back-to-back transactions – but they do NOT overlap:

- A master may hold PSEL and start the next SETUP the cycle after PREADY

- apb4_slave is one-command-at-a-time, so the next command is not captured until the previous transaction completes

- Sustained cadence is therefore one cycle LONGER than single-transfer latency: measured PREADY-to-PREADY = 10 pclk cycles at the earliest legal turnaround, against 9 cycles SETUP-to-PREADY for one transfer (M = 1; 11 and 10 on an arbitrated variant). The extra cycle is the next transfer’s mandatory SETUP phase, which cannot overlap the previous ACCESS (see 5.1/5.2)

Next: Address Mapping

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Address Mapping

## Address Map Structure

The shipped APB Crossbar variants use a 64KB address region per slave. The size is chosen at generation (--slave-size, generator default 4KB) and then baked into the decode part-select, so it is fixed for a given generated module:

### Figure 3.3: Address Mapping Structure

Address Mapping

## Default Address Map

With default BASE_ADDR = 0x1000_0000:

Default Address Map (4-slave example)

| Slave Index | Start Address | End Address | Size |
| --- | --- | --- | --- |
| 0 | 0x1000_0000 | 0x1000_FFFF | 64KB |
| 1 | 0x1001_0000 | 0x1001_FFFF | 64KB |
| 2 | 0x1002_0000 | 0x1002_FFFF | 64KB |
| 3 | 0x1003_0000 | 0x1003_FFFF | 64KB |
| … | … | … | … |
| N-1 | BASE + (N-1)*64KB | BASE + N*64KB - 1 | 64KB |

## Address Decode Logic

The target slave comes from simple bit extraction:

offset = PADDR - BASE_ADDR
slave_index = offset[16 +: $clog2(S)]   // ceil(log2(S)) bits: [17:16] for S=4
local_addr = offset[15:0]      // Bits 15:0 are local address within slave

### Example Decode

For address 0x1002_3456 with BASE_ADDR = 0x1000_0000:

Address Decode Example

| Step | Calculation | Result |
| --- | --- | --- |
| 1. Compute offset | 0x1002_3456 - 0x1000_0000 | 0x0002_3456 |
| 2. Extract slave index | offset[17:16] (S=4) | 0x2 (Slave 2) |
| 3. Extract local address | offset[15:0] | 0x3456 |

## Customizing Base Address

The BASE_ADDR parameter shifts the entire address map:

// Peripheral space at 0x4000_0000
apbx_xbar_1to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_xbar (...);

// Resulting map:
// Slave 0: 0x4000_0000 - 0x4000_FFFF
// Slave 1: 0x4001_0000 - 0x4001_FFFF
// Slave 2: 0x4002_0000 - 0x4002_FFFF
// Slave 3: 0x4003_0000 - 0x4003_FFFF

## Address Map Constraints

### Fixed Slave Region Size

Each slave occupies exactly 64KB (0x10000 bytes). This is a current design constraint.

Implications: - Peripherals requiring more than 64KB need multiple slave ports - Peripherals using less than 64KB have unused address space

### Maximum Configuration

With 4-bit slave index extraction: - Maximum 16 slaves - Total address space: 16 x 64KB = 1MB

### Address Width Requirement

The ADDR_WIDTH parameter must be sufficient to address: - BASE_ADDR + (NUM_SLAVES x 64KB)

Example: For 4 slaves at BASE_ADDR = 0x8000_0000, need: - 0x8000_0000 + 0x4_0000 = 0x8004_0000 - Minimum ADDR_WIDTH: 32 bits

Next: Chapter 4: Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Master-Side APB Interfaces

## Interface Overview

Each master port presents a complete APB slave interface, accepting transactions from an external APB master (CPU, DMA, etc.).

## Signal Definition

For each master index i (0 to M-1):

Master Port Signal Definition

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m<i>_apb_PSEL | 1 | Input | Peripheral select |
| m<i>_apb_PENABLE | 1 | Input | Transfer enable |
| m<i>_apb_PADDR | ADDR_WIDTH | Input | Address bus |
| m<i>_apb_PWRITE | 1 | Input | Write/read direction |
| m<i>_apb_PWDATA | DATA_WIDTH | Input | Write data bus |
| m<i>_apb_PSTRB | STRB_WIDTH | Input | Write strobes |
| m<i>_apb_PPROT | 3 | Input | Protection attributes |
| m<i>_apb_PRDATA | DATA_WIDTH | Output | Read data bus |
| m<i>_apb_PSLVERR | 1 | Output | Slave error response |
| m<i>_apb_PREADY | 1 | Output | Transfer ready |

### APB5 Sideband (mixed-version variants only)

An APB5 master port carries five additional pins. They exist ONLY on ports the generator was told are APB5 – in the shipped apbx_xbar_2to2_mixed that is m1 alone, and m0 has none of them. That structural absence IS the version gate; there is no mask to set.

APB5 Master Sideband Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m<i>_apb_PAUSER | 1 | Input | Request user attributes; forwarded only to an APB5 slave |
| m<i>_apb_PWUSER | 1 | Input | Write-data user attributes; likewise |
| m<i>_apb_PRUSER | 1 | Output | Completer read-data user attributes, from an APB5 slave |
| m<i>_apb_PBUSER | 1 | Output | Completer response user attributes, from an APB5 slave |
| m<i>_apb_PWAKEUP | 1 | Output | Tied '0 – see below |

Reading from an APB4 slave returns '0 on PRUSER/PBUSER rather than a stale or fabricated value, and that is proven, not just tested (formal/apbx_xbar/apbx_xbar_2to2_mixed/, ap_gate_no_fab_*).

PWAKEUP is always '0. The boundary IP leaves rsp_pwakeup unconnected and ties wakeup_request to '0, so an APB5 slave asserting PWAKEUP is never seen here. This is a known limitation rather than a guarantee, and it is asserted formally (ap_pwakeup_dropped) so that wiring it up later fails a proof instead of changing behaviour silently.

The user-signal widths are fixed at 1 bit, not parameterised. This table previously named AUSER_WIDTH/WUSER_WIDTH/RUSER_WIDTH/BUSER_WIDTH as though an integrator could size them. There are no such parameters: the whole module takes only ADDR_WIDTH, BASE_ADDR, DATA_WIDTH and STRB_WIDTH, and the generator hardcodes AUSER_WIDTH(1) .. BUSER_WIDTH(1) at each boundary shim. Widening a user field means regenerating the crossbar, not overriding a parameter.

Parity pins are absent entirely: every shipped variant is generated with parity off, and a mixed pairing carries no parity in any case.

## Signal Descriptions

### PSEL (Input)

Peripheral select signal. When asserted: - Indicates the master wants to access the crossbar - Must remain asserted throughout the transaction - Combined with PADDR determines target slave

### PENABLE (Input)

Transfer enable signal. APB state machine: - PSEL=1, PENABLE=0: Setup phase - PSEL=1, PENABLE=1: Access phase (data transfer)

### PADDR (Input)

Address bus. Used for: - Target slave selection (upper bits) - Local address within slave (lower bits) - Must be stable while PSEL asserted

### PWRITE (Input)

Transfer direction: - PWRITE=1: Write transaction (data from master) - PWRITE=0: Read transaction (data to master)

### PWDATA (Input)

Write data bus: - Valid during write transactions - Sampled when PREADY and PENABLE both high - Width set by DATA_WIDTH parameter

### PSTRB (Input)

Write byte strobes: - One bit per byte of DATA_WIDTH - Indicates which bytes are valid in PWDATA - Ignored during read transactions

### PPROT (Input)

Protection attributes (3 bits): - Bit 0: Privileged/normal access - Bit 1: Secure/non-secure access - Bit 2: Instruction/data access

### PRDATA (Output)

Read data bus: - Valid during read transactions when PREADY high - Undefined during write transactions - Width matches DATA_WIDTH parameter

### PSLVERR (Output)

Slave error response: - Sampled when PREADY and PENABLE both high - PSLVERR=1 indicates error condition - Propagated from the downstream slave, or generated locally by this port when the address decodes outside the map: on the decoding variants a miss completes here with PSLVERR=1 and never reaches a slave (projects/components/fabric-gen-ip/apbx-xbar BUG-002, was APBX-005). The 1to1 and 2to1 variants have no decode and so never generate it locally.

### PREADY (Output)

Transfer ready signal: - PREADY=1 allows the transaction to complete - PREADY=0 inserts wait states - Reflects downstream slave PREADY (or arbitration wait)

## Transaction Timing

### Uncontended Write (No Arbitration Wait) – PHASE SHAPE ONLY, not to scale:

a real transfer is 9 pclk cycles SETUP-to-PREADY on a single-master variant, 10 on an arbitrated one (see 5.2)

Cycle 1    Cycle 2    Cycle 3
PSEL   __|-----------|----------|____
PENABLE__|________---|----------|____
PADDR  --<  Address  >--
PWRITE --<  1        >--
PWDATA --<  Data     >--
PREADY _________|----------|____

### Contended Transaction (Arbitration Wait)

When another master holds the target slave:

Cycle 1    Cycle 2    Cycle 3    Cycle 4
PSEL   __|--------------------------|____
PENABLE__|_____________|--------------|____
PADDR  --<  Address                   >--
PREADY _______________________|------|____
                     ^ Wait for grant

Next: Slave-Side Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Slave-Side APB Interfaces

## Interface Overview

Each slave port presents a complete APB master interface, driving transactions to an external APB slave (peripheral).

## Signal Definition

For each slave index j (0 to N-1):

Slave Port Signal Definition

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s<j>_apb_PSEL | 1 | Output | Peripheral select |
| s<j>_apb_PENABLE | 1 | Output | Transfer enable |
| s<j>_apb_PADDR | ADDR_WIDTH | Output | Address bus |
| s<j>_apb_PWRITE | 1 | Output | Write/read direction |
| s<j>_apb_PWDATA | DATA_WIDTH | Output | Write data bus |
| s<j>_apb_PSTRB | STRB_WIDTH | Output | Write strobes |
| s<j>_apb_PPROT | 3 | Output | Protection attributes |
| s<j>_apb_PRDATA | DATA_WIDTH | Input | Read data bus |
| s<j>_apb_PSLVERR | 1 | Input | Slave error response |
| s<j>_apb_PREADY | 1 | Input | Transfer ready |

### APB5 Sideband (mixed-version variants only)

Symmetric with the master side, and equally structural: these pins exist ONLY on slave ports the generator was told are APB5. In the shipped apbx_xbar_2to2_mixed that is s0 alone – s1 has none of them – so the absence IS the version gate, with no mask to set. Note the pairing in that variant is crosswise: the APB5 port is m1 on the master side and s0 here.

APB5 Slave Sideband Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s<j>_apb_PAUSER | 1 | Output | Request user attributes, forwarded from an APB5 master |
| s<j>_apb_PWUSER | 1 | Output | Write-data user attributes, likewise |
| s<j>_apb_PRUSER | 1 | Input | Read-data user attributes returned by the slave |
| s<j>_apb_PBUSER | 1 | Input | Response user attributes returned by the slave |
| s<j>_apb_PWAKEUP | 1 | Input | Driven by the slave, and dropped – see below |

Widths are fixed at 1 bit. The generator hardcodes AUSER_WIDTH(1) .. BUSER_WIDTH(1) on the boundary shim and exposes no parameter for them.

s<j>_apb_PWAKEUP is accepted at the pin and goes nowhere. The boundary IP leaves rsp_pwakeup unconnected and ties wakeup_request to '0, so an APB5 slave asserting PWAKEUP is never observed by any master. That is a known limitation rather than a guarantee, and it is asserted formally (ap_pwakeup_dropped) so that wiring it up later fails a proof instead of changing behaviour silently – the same contract described from the master’s side in Master Side.

Reading from an APB4 slave returns '0 on PRUSER/PBUSER rather than a stale or fabricated value, proven by ap_gate_no_fab_* in formal/apbx_xbar/apbx_xbar_2to2_mixed/.

## Signal Behavior

### Output Signals (Crossbar to Slave)

PSEL: Asserted when a transaction targets this slave. Remains high throughout the transaction.

PENABLE: Follows standard APB timing – exactly one setup cycle (PSEL high, PENABLE low) before ACCESS, on every transfer.

Until 2026-08-28 apb4_master/apb5_master drove two setup cycles when launching out of IDLE, measured [2,2,2] at a downstream port; only back-to-back transfers taking the ACCESS->SETUP shortcut were compliant. Fixed as AMBA TASK-071 by not asserting PSEL in the IDLE launch cycle, which cost no latency because the state sequence was unchanged. Now measured [1,1,1,1,1] downstream, asserted by val/amba/test_apb_master_setup_phase.py and by the downstream check in dv/tests/test_apbx_xbar_timing.py.

- Low during setup phase

- High during access phase

PADDR: Contains the full address from the master. The local address within a 64KB region is offset[15:0] – note this is the OFFSET’s low bits, not PADDR’s, so it only equals PADDR[15:0] when BASE_ADDR is 64KB-aligned (which is permitted but not required).

PWRITE, PWDATA, PSTRB, PPROT: Passed through from the winning master without modification.

### Input Signals (Slave to Crossbar)

PRDATA: Captured by crossbar when both PREADY and PENABLE are high. Routed to originating master.

PSLVERR: Captured with PRDATA. Propagated to originating master as error indication.

PREADY: Controls transaction completion: - PREADY=1: Transaction completes this cycle - PREADY=0: Insert wait states

## Slave Response Handling

### Wait States

Slaves can insert wait states by holding PREADY low:

Cycle 1    Cycle 2    Cycle 3    Cycle 4
PSEL   __|-------------------------------|____
PENABLE__|____________|------------------|____
PREADY _____________________|-----------|____
                      ^ Wait states

The crossbar: - Holds master PREADY low while waiting - Does not timeout (assumes slaves always respond) - Passes through all wait states to master

### Error Response

When slave asserts PSLVERR:

- Crossbar captures PSLVERR with PRDATA

- Routes PSLVERR to originating master

- Master sees PSLVERR=1 when its PREADY goes high

No error handling or recovery - error indication only.

## Multi-Slave Considerations

### Address Overlap Prevention

Each slave port is mutually exclusive: - Only one slave receives PSEL for any given address - No overlapping address regions - Address decode is deterministic

### Independent Operation

Each slave operates independently: - Own PREADY timing - No synchronization between slaves - Concurrent transactions to different slaves allowed (different masters)

Next: Clock and Reset

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Clock and Reset

## Clock Interface

### Signal Definition

Clock Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| pclk | 1 | Input | APB clock |

### Clock Requirements

Single Clock Domain: - All crossbar logic operates on the pclk rising edge - All APB signals sampled/driven synchronous to pclk - No internal clock generation or division

Frequency: - No minimum frequency requirement - Maximum frequency set by the critical path - Typical: 100-250 MHz (depends on target technology)

Duty Cycle: - No specific duty cycle requirement - Standard 50% duty cycle recommended

## Reset Interface

### Signal Definition

Reset Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| presetn | 1 | Input | Active-low asynchronous reset |

### Reset Behavior

Assertion (presetn goes low): - All internal state cleared immediately - All output signals driven to safe values - Master ports: PREADY=0, PRDATA=0, PSLVERR=0 - Slave ports: PSEL=0, PENABLE=0, PADDR=0, etc.

Deassertion (presetn goes high): - Internal state begins normal operation - Reset release is NOT synchronized internally – presetn is applied directly as an asynchronous reset (ALWAYS_FF_RST) in every module. The integrator must synchronize reset DEASSERTION to pclk externally (recovery/removal timing) - Ready to accept transactions from the first pclk edge after a correctly synchronized deassertion – NOT “immediately” in the sense of the deassertion instant. Until that externally synchronized removal lands cleanly, the fabric’s state is not defined; the “Reset Deassertion” guidance later on this page (wait 1 cycle before the first transaction) is the operative rule.

### Reset Safe Values

Reset Safe Values

| Signal Group | Reset Value | Rationale |
| --- | --- | --- |
| Master PREADY | 0 | apb4_slave asserts PREADY only in BUSY |
| Master PRDATA | 0 | No false data |
| Master PSLVERR | 0 | No false errors |
| Slave PSEL | 0 | No false transactions |
| Slave PENABLE | 0 | No false enables |
| Arbiter grants | 0 | No master selected |

## Clock-Reset Relationship

### Reset Assertion

Reset can be asserted at any time: - Asynchronous assertion (immediate effect), in every build. This comes from ALWAYS_FF_RST in rtl/amba/includes/reset_defs.svh, which is unconditionally async; the old USE_ASYNC_RESET switch is a no-op. - Deassertion must be synchronous to pclk – see “Reset Deassertion” below. This bullet previously also permitted asynchronous deassertion, contradicting that section three lines later. - All in-flight transactions aborted

### Reset Deassertion

For clean operation: 1. Assert reset for minimum 2 pclk cycles 2. Deassert reset synchronous to pclk rising edge 3. Wait 1 cycle before first transaction

### Reset Sequence Diagram

pclk      _|~|_|~|_|~|_|~|_|~|_|~|_|~|_

presetn   ________|~~~~~~~~~~~~~~~~

Internal  xxxxxxxx|--- Valid State --
State

Ready for ________|--- Transactions --
Traffic

## Integration Notes

### Clock Domain Crossing

If masters or slaves are in different clock domains: - Insert CDC logic external to the crossbar - The crossbar assumes a single clock domain - Consider an APB clock bridge if needed

### Clock Gating

The crossbar supports clock gating: - No internal activity when idle - Safe to gate pclk when no transactions pending - All registers use standard flip-flops (no clock gating cells)

Next: Chapter 5: Performance

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Throughput Characteristics

## Maximum Throughput

### Single Master

With a single master accessing any slave, the figures split by arbiter presence: the generator emits an arbiter only when M > 1, and its registered grant costs a cycle on every one of them. See 5.2.

Single-Master-Active Throughput, by Variant Class

| Metric | M = 1 | M > 1 | Notes |
| --- | --- | --- | --- |
| Cycles per transaction (uncontended) | 9 SETUP->PREADY; 8 fabric | 10 SETUP->PREADY; 9 fabric | measured, zero-wait slave |
| Sustained cycles per transaction | 10 (PREADY->PREADY) | 11 (PREADY->PREADY) | measured back-to-back, one active master |
| Maximum transactions per cycle | ~0.100 | ~0.091 | 1 transaction / 10 or 11 cycles |
| Data throughput (32-bit @ 100MHz) | ~40 MB/s | ~36 MB/s | 4 B per period |
| Data throughput (32-bit @ 250MHz) | ~100 MB/s | ~91 MB/s | 4 B per period |

M = 1: apbx_xbar_1to1, apbx_xbar_1to4. M > 1: apbx_xbar_2to1, apbx_xbar_2to4, apbx_xbar_2to2_mixed – so the 2x4 the PRD calls the typical SoC case is the slower column. This table published the M = 1 figures unconditionally until 2026-08-29.

### Multi-Master (Uncontended)

When masters access different slaves: - Each master achieves single-master throughput - Aggregate throughput = M x single-master throughput - No CONTENTION overhead – but the registered grant still costs one cycle on every M > 1 variant, contended or not (see 5.2). Only the single-master variants have no arbiter at all.

### Multi-Master (Contended)

When masters compete for the same slave:

Contended Access Throughput

| Masters | Effective Throughput | Notes |
| --- | --- | --- |
| 2 | 50% per master | Round-robin sharing |
| 3 | 33% per master | Round-robin sharing |
| 4 | 25% per master | Round-robin sharing |

## Arbitration Handoff

### Grant Persistence

The crossbar implements grant persistence: - A master holds the grant for the duration of ONE transaction - The grant is released at the RESPONSE handshake, not when PSEL drops (grant_ack = grant && rsp_valid && rsp_ready, arbiter instantiated with WAIT_GNT_ACK(1)), after which the round-robin mask rotates – holding PSEL does not keep the grant if another master is requesting - A single master with no competitor re-wins immediately, so its transactions still stream back-to-back – at the fabric’s own cadence, not at APB’s 2-cycle minimum

### Back-to-Back Timing

A “zero-bubble” 4-cycle two-transaction diagram appeared here in earlier revisions. It described a pipeline this RTL does not have: the apb4_slave front end is a one-command-at-a-time FSM (IDLE -> BUSY -> WAIT), so the next command is not captured until the previous transaction completes. But sustained cadence is not equal to single-transfer latency either – it is one cycle longer. A master holding PSEL high and dropping PENABLE for exactly one cycle after each PREADY (the earliest LEGAL turnaround) measures PREADY-to-PREADY = 10 pclk cycles, against a 9-cycle SETUP-to-PREADY single transfer. The extra cycle is the next transfer’s mandatory SETUP phase, which cannot overlap the previous transfer’s ACCESS. See 5.2 for the convention and the cycle-by-cycle breakdown. There is still no overlap between consecutive transactions to draw.

## Throughput Factors

### Factors That Reduce Throughput

Throughput Reduction Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Slave wait states | Variable | Choose faster peripherals |
| Arbitration conflicts | ~11 cycles per master ahead in the round-robin | Distribute access across slaves |
| Slave response time | Variable | Design slaves for quick response |

### Optimal Usage Pattern

For maximum throughput: 1. Distribute master access across different slaves 2. Minimize slave wait states 3. Use burst-like access patterns (same master, same slave) 4. Avoid rapid master switching on same slave

Next: Latency Analysis

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Latency Analysis

## Transaction Latency

### Uncontended Access

When a master has exclusive access to a slave:

Uncontended Transaction Latency

| Phase | Cycles | Description |
| --- | --- | --- |
| Master SETUP | 1 | PSEL high, PENABLE low |
| apb4_slave capture + cmd skid | 2 | command registered out |
| apb4_master IDLE-launch / SETUP / ACCESS | 3 | downstream PSEL then PENABLE |
| Response + rsp skid | 2 | slave PREADY through the return skid |
| apb4_slave BUSY -> PREADY | 1 | master-visible completion |
| Total | 9 | PSEL to PREADY (8 of them ACCESS to PREADY) |

These are measured numbers, not protocol minimums. A crossbar transfer is NOT the 2-cycle APB minimum: the fabric converts APB to an internal cmd/rsp protocol through apb4_slave and back through apb4_master, and both directions cross REGISTERED skid buffers.

Measurement convention. Every figure below counts rising pclk edges and names the two edges it spans. State the edges or the number means nothing – this page has been wrong twice from not doing so.

The numbers depend on whether the variant has an arbiter, and the generator only emits one when there is more than one master. A single-master variant has no arbiter_round_robin at all; a multi-master variant has one per slave, and its grant is a flop (grant <= w_next_grant), which costs exactly one cycle on every quantity below.

Measured Timing, Always-Ready Slave

| Quantity | M = 1 | M > 1 | Spans |
| --- | --- | --- | --- |
| Fabric latency | 8 | 9 | first ACCESS edge -> edge where PREADY is high |
| Single-transfer latency | 9 | 10 | SETUP edge -> edge where PREADY is high |
| Back-to-back period | 10 | 11 | PREADY edge -> next PREADY edge |

M = 1 is apbx_xbar_1to1 and apbx_xbar_1to4 (0 arbiters). M > 1 is apbx_xbar_2to1, apbx_xbar_2to4 and apbx_xbar_2to2_mixed (1, 4 and 2 arbiters). All six columns are measured, not derived – see dv/tests/test_apbx_xbar_timing.py, which asserts both classes.

Until 2026-08-29 this page published the M = 1 numbers unconditionally, under the heading “Uncontended Access”, having measured only apbx_xbar_1to1. They were one cycle optimistic for every arbitrated variant – including the 2x4 that the PRD calls the typical SoC case.

The period is 10, not 9, and the difference is structural. After PREADY at cycle N the bus is still in ACCESS for that cycle, so the next transfer’s mandatory SETUP cycle cannot begin before N+1; its ACCESS follows at N+2 and its PREADY at N+2+8 = N+10. Reaching 9 would require a SETUP cycle overlapping the previous transfer’s ACCESS, which is not a legal APB waveform. So sustained cadence does not equal single-transfer latency here – it is exactly one cycle longer.

Earlier revisions of this page claimed 2 cycles (the bare APB minimum for a directly-attached slave, which does not apply through this fabric), and later claimed a 9-cycle sustained cadence, which came from a probe whose turnaround was not legal APB. A qc reviewer flagged the 9-cycle cadence and was told it was a false positive; the reviewer was right. dv/tests/test_apbx_xbar_timing.py now asserts all three numbers against the RTL so the question is settled by the suite rather than by argument.

### Contended Access

When multiple masters compete for the same slave:

Arbitration Latency

| Scenario | Additional Latency | Description |
| --- | --- | --- |
| Win arbitration | 1 cycle | grant is REGISTERED in arbiter_round_robin |
| Lose to 1 master | ~10+ cycles | wait one full transaction (see above) |
| Lose to N masters | ~10N+ cycles | wait N transactions |

## Latency Breakdown

### Forward Path (Master to Slave)

Forward Path Latency

| Component | M = 1 | M > 1 | Type |
| --- | --- | --- | --- |
| apb4_slave capture | 1 | 1 | Registered |
| apb4_slave cmd skid out | 1 | 1 | Registered |
| Address decode | 0 | 0 | Combinational |
| Arbitration | 0 | 1 | No arbiter is emitted when M = 1; when one is, its grant is a flop |
| apb4_master cmd skid in | 1 | 1 | Registered |
| apb4_master cmd skid out | 1 | 1 | Registered |
| apb4_master IDLE -> SETUP | 1 | 1 | FSM |
| Total | 5 | 6 | master SETUP edge to downstream PSEL edge |

Both columns measured: 5 on apbx_xbar_1to1, 6 on apbx_xbar_2to1. The arbitration row is the whole difference between the two variant classes. This table carried a single unconditional column with a 1-cycle arbitration row and a 5-cycle total until 2026-08-29 – which cannot both be true, since the 5 was measured on the variant that has no arbiter.

The three numbers on this page must sum, and now do:

M = 1 M > 1 5 forward 6 forward + 1 downstream SETUP -> ACCESS + 1 downstream SETUP -> ACCESS + 3 response + 3 response = 9 SETUP-to-PREADY = 10 SETUP-to-PREADY

(With a zero-wait slave the downstream ACCESS and PREADY edges are the same cycle, which is why the response row can be read from either. A slave with wait states adds its own cycles between them and shifts the total.)

This table read 4 until 2026-08-29, which could not be reconciled with the page’s own 9-cycle total – 4 + 3 left two cycles unaccounted for. Measured on apbx_xbar_1to1 with an always-ready slave: master SETUP at cycle 1, downstream PSEL at cycle 6, downstream ACCESS at cycle 7, master PREADY at cycle 10.

### Response Path (Slave to Master)

Response Path Latency

| Component | Cycles | Type |
| --- | --- | --- |
| Slave response | Variable | The slave’s own PREADY timing, excluded below |
| apb4_master captures into rsp skid | 1 | Registered |
| rsp skid out, through the response mux | 1 | Skid registered; the mux itself is combinational |
| apb4_slave drives PREADY | 1 | Registered |
| Total | 3 | downstream PREADY edge to master PREADY edge |

Same on both variant classes – arbitration is on the command path only. The rows read “0-1 cycle” each until 2026-08-29, which could not sum to the stated 3.

## Latency Timing Diagram

### Best Case (No Waits, No Contention)

Master PSEL   ___|-----------|___
Master PENABLE___|_____------|___
               ^         ^
Slave PSEL    _____|---------|___
Slave PENABLE _____|___------|___
Slave PREADY  _________|-----|___
                       ^
Master PREADY _________|-----|___
                       ^
              Total: 9 cycles (SETUP edge to PREADY edge)

### Worst Case (Contention + Slave Wait States)

Master PSEL   ___|--------------------------|___
Master PENABLE___|________________----------|___
               ^  Arbitration    ^
               |  wait           |
Slave PSEL    ____________|---------------|___
Slave PENABLE ____________|______---------|___
Slave PREADY  _____________________|------|___
                                   ^
Master PREADY _____________________|------|___
                                   ^
              Total: 9+ cycles (plus slave wait states and contention)

## Latency Optimization

### Design Recommendations

Latency Optimization Recommendations

| Goal | Recommendation |
| --- | --- |
| Minimize contention | Use more slaves, spread access |
| Reduce arbitration wait | Lower master count per slave |
| Reduce slave latency | Design slaves with quick PREADY |

Next: Resource Estimates

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Resource Estimates

## FPGA Resource Usage

### Pre-Generated Variants

Estimated resource usage for common configurations (32-bit data width):

| Variant | LUTs | FFs | Description |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | ~150 | ~80 | Full 1x1 with buffering |
| apbx_xbar_2to1 | ~300 | ~150 | 2 masters, arbitration |
| apbx_xbar_1to4 | ~400 | ~200 | 4 slaves, decode |
| apbx_xbar_2to4 | ~800 | ~400 | Full 2x4 crossbar |

Hand estimates, not synthesis results, and the single source of truth for this component. PRD §9.3 previously carried a second, different set of numbers (2-3x smaller in every cell); that table now refers here rather than competing with it. Nothing in the repo has been synthesized to confirm either set – treat these as order-of-magnitude only.

: FPGA Resource Estimates (32-bit)

### Scaling Factors

Resource usage scales approximately as:

Resource Scaling Factors

| Component | Scaling | Notes |
| --- | --- | --- |
| apb4_slave instances | M x base | Linear with masters |
| apb4_master instances | N x base | Linear with slaves |
| Arbiters | N x log2(M) | Per slave, sized for masters |
| Address decode | N | Comparators per slave |
| Response mux | N x M | Full crosspoint for responses |

## Lines of Code

Source code metrics for pre-generated variants:

Source Code Metrics

| Variant | Lines of Code | Complexity |
| --- | --- | --- |
| apbx_xbar_1to1 | 165 | Low |
| apbx_xbar_2to1 | 314 | Medium |
| apbx_xbar_1to4 | 417 | Medium |
| apbx_xbar_2to2_mixed | 625 | Medium |
| apbx_xbar_2to4 | 806 | Medium-High |

This table is the single source of truth for LOC, as it already is for the LUT/FF estimates above. Exact wc -l of rtl/*.sv, measured 2026-10-12; the PRD and the key-features page point here rather than carrying their own copies.

Three tables disagreed before that – by up to 33% on the same file (2to4 as ~1000, ~751 and 751) – because each was corrected on a different day. Even the most precise-looking set was stale: 165/314/384/751/570 was right when reconciled on 2026-08-27 and wrong two days later, once the projects/components/fabric-gen-ip/apbx-xbar BUG-002 (was APBX-005) decode-miss logic added lines. Numbers that must be hand-synced across three pages will desync; keeping one and pointing at it is the fix.

## Power Considerations

### Static Power

Static power scales with: - Total register count (FFs) - Routing resources used - Technology node

### Dynamic Power

Dynamic power depends on: - Transaction rate (activity factor) - Data width (toggling bits) - Clock frequency

### Power Optimization

Power Optimization Techniques

| Technique | Benefit | Trade-off |
| --- | --- | --- |
| Clock gating | Reduce idle power | Adds gating logic |
| Lower data width | Fewer toggling bits | Reduced throughput |
| Fewer slaves | Less decode logic | Reduced flexibility |

## Area Comparison

Relative area comparison (normalized to apbx_xbar_1to1):

Relative Area Comparison

| Variant | Relative Area |
| --- | --- |
| apbx_xbar_1to1 | 1.0x (baseline) |
| apbx_xbar_2to1 | 2.0x |
| apbx_xbar_1to4 | 2.5x |
| apbx_xbar_2to4 | 5.0x |

## Custom Configuration Estimation

For custom MxN configuration:

Estimated LUTs ~= 50 + (M * 75) + (N * 50) + (M * N * 10)
Estimated FFs  ~= 20 + (M * 40) + (N * 30)

Example: 4x8 crossbar - LUTs: 50 + (475) + (850) + (4810) = 50 + 300 + 400 + 320 = ~1070 - FFs: 20 + (440) + (830) = 20 + 160 + 240 = ~420

Next: Chapter 6: Integration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# System Requirements

## Hardware Requirements

### Clock and Reset

Clock and Reset Requirements

| Requirement | Specification |
| --- | --- |
| Clock input | Single clock (pclk) |
| Clock frequency | No minimum, technology-dependent maximum |
| Reset input | Active-low (presetn); asynchronous assertion, synchronous deassertion |
| Reset duration | Minimum 2 clock cycles |

### APB Compliance

The crossbar requires APB-compliant masters and slaves:

APB Compliance Requirements

| Requirement | Description |
| --- | --- |
| Protocol version | APB4 (with PSTRB, PPROT); APB5 sideband supported only on mixed-version variants such as apbx_xbar_2to2_mixed |
| Wait states | Supported (PREADY-based) |
| Error response | Supported (PSLVERR) |
| Slave timeout | Not handled internally |

## Software Requirements

### Address Map Configuration

Software must be configured with the correct address map:

// Example address definitions
#define PERIPHERAL_BASE     0x10000000
#define UART_BASE          (PERIPHERAL_BASE + 0x00000)
#define GPIO_BASE          (PERIPHERAL_BASE + 0x10000)
#define TIMER_BASE         (PERIPHERAL_BASE + 0x20000)
#define SPI_BASE           (PERIPHERAL_BASE + 0x30000)

### Driver Considerations

Driver Considerations

| Consideration | Recommendation |
| --- | --- |
| Polling vs interrupt | Both supported |
| Atomic operations | Not guaranteed across arbitration |
| Cache coherency | APB typically uncached |

## Constraints

### Address Map Constraints

Address Map Constraints

| Constraint | Value | Notes |
| --- | --- | --- |
| Slave region size | 64KB in the shipped variants | Set at generation via --slave-size; fixed once generated |
| Maximum slaves | 16 | Generator limit |
| Maximum masters | 16 | Generator limit |
| Address alignment | Byte aligned | No alignment requirement |

### Transaction Constraints

Transaction Constraints

| Constraint | Value | Notes |
| --- | --- | --- |
| Outstanding transactions | 1 per master | APB protocol limit |
| Transaction interleaving | Not supported | APB is non-pipelined |
| Burst transactions | Not supported | APB single-beat only |

## Integration Checklist

### Pre-Integration

- ☐ Verify BASE_ADDR does not conflict with other subsystems

- ☐ Confirm all slaves fit within 64KB regions

- ☐ Validate ADDR_WIDTH is sufficient for address map

- ☐ Check DATA_WIDTH matches system data width

### Post-Integration

- ☐ Verify address decode routing to correct slaves

- ☐ Test all master-to-slave paths

- ☐ Validate arbitration behavior (if multi-master)

- ☐ Check reset behavior

Next: Parameter Configuration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Parameter Configuration

## Module Parameters

### Core Parameters

Core Parameter Definition

| Parameter | Type | Default | Range | Description |
| --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | 18-64 for decoding variants; 1-64 for 1to1/2to1 | Address bus width in bits. The decode part-select reads bits 16 +: $clog2(S), so a 2-slave variant needs only bit 16 – but the generator emits the range-check span as a fixed 32-bit constant (paddr < BASE_ADDR + 32'h...), so anything below 18 width-expands and fails a warnings-fatal build. 17 lints clean only with -Wno-fatal; it is not usable. |
| DATA_WIDTH | int | 32 | 8, 16, 32, 64 | Data bus width in bits |
| STRB_WIDTH | int | DATA_WIDTH/8 | derived | Write strobe width. Declared as a parameter, but it is computed from DATA_WIDTH and overriding it is not supported |
| BASE_ADDR | logic[ADDR_WIDTH-1:0] | 0x10000000 | any address; the slave index is decoded from the OFFSET, so no span alignment is required | Base of the slave map. UNUSED on 1to1 and 2to1 – they have a single slave and no decode, so the parameter is declared only for interface uniformity and setting it changes nothing (see those modules’ RTL headers) |

### Derived Parameters

These are calculated automatically:

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| STRB_WIDTH | DATA_WIDTH / 8 | Byte strobe width |
| (none) | — | Each variant declares only the four parameters above; master/slave counts are baked into the module name and port list, NOT exposed as parameters. |

## Parameter Usage Examples

### Default Configuration

apbx_xbar_2to4 u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections
);
// Uses defaults: ADDR_WIDTH=32, DATA_WIDTH=32, BASE_ADDR=0x10000000

### Custom Base Address

apbx_xbar_2to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections
);
// Slaves at 0x4000_0000, 0x4001_0000, 0x4002_0000, 0x4003_0000

### 64-bit Data Width

apbx_xbar_2to4 #(
    .DATA_WIDTH(64)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections with 64-bit data buses
);
// STRB_WIDTH automatically becomes 8

### Wide Address

apbx_xbar_2to4 #(
    .ADDR_WIDTH(40),
    .BASE_ADDR(40'h80_0000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections with 40-bit address buses
);

## Parameter Validation

### Address Width Check

Ensure ADDR_WIDTH can accommodate the full address range:

Required minimum: log2(BASE_ADDR + NUM_SLAVES * 64KB)

Example: 4 slaves at BASE_ADDR=0x1000_0000 - Max address: 0x1000_0000 + 4*0x10000 = 0x1004_0000 - Required: 29 bits minimum - ADDR_WIDTH=32 is sufficient

### Data Width Constraints

Valid Data Width Configurations

| DATA_WIDTH | STRB_WIDTH | Valid APB Widths |
| --- | --- | --- |
| 8 | 1 | Yes |
| 16 | 2 | Yes |
| 32 | 4 | Yes (most common) |
| 64 | 8 | Yes |

## Custom Generation Parameters

When using the Python generator for custom configurations:

python generate_xbars.py \
    --masters 3 \
    --slaves 6 \
    --base-addr 0x80000000 \
    --slave-size 0x10000

Generator Command Line Options

| Option | Description | Default |
| --- | --- | --- |
| –masters | Number of master ports | Required |
| –slaves | Number of slave ports | Required |
| –base-addr | Base address (hex) | 0x10000000 |
| –slave-size | Address space per slave (0x1000 = 4KB, 0x10000 = 64KB) | 0x10000 |

--slave-size decides which address bits select the slave: the crossbar decodes offset[log2(slave_size) +: ceil(log2(S))], so 64KB windows give offset[16 +: n] and 4KB windows give offset[12 +: n]. Until 2026-09-07 this script did not pass the value through on the custom path, so a custom variant silently came out with 4KB windows while all four shipped variants had 64KB – one family, two address maps. It now defaults to 64KB to match them.

The lower-level apbx_xbar_generator.py takes the same flag but keeps its own historical default of 4KB; prefer generate_xbars.py so the family stays consistent.

Next: Verification Strategy

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Verification Strategy

## Pre-Verified Component

The APB Crossbar is pre-verified:

| Test / Variant | Transactions | Status | Coverage |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | 100+ | Pass | Basic connectivity |
| apbx_xbar_2to1 | 130+ | Pass | Arbitration stress |
| apbx_xbar_1to4 | 200+ | Pass | Address decode |
| apbx_xbar_2to4 | 350+ | Pass | Full crossbar stress |
| apbx_xbar_2to2_mixed | 8 directed | Pass | All four APB4/APB5 pairings, sideband gating, decode miss, structural pin check |
| apbx_xbar_timing | 2 variants | Pass | Published latency/cadence figures, both variant classes |

Overall: 100% passing (9/9), >750 transactions plus the directed mixed-version and timing checks. apbx_xbar_2to2_mixed also has a formal harness (formal/apbx_xbar/apbx_xbar_2to2_mixed/).

: Pre-Verification Status

## Test Categories

### Basic Connectivity

Tests for each variant include: - Single read transaction to each slave - Single write transaction to each slave - Read-modify-write sequences - All address regions exercised

### Address Decode Verification

For multi-slave variants: - Verify each slave selected by correct address range - Boundary address testing - Invalid address behavior (decoding variants complete the access with PSLVERR; they do not hang – see 4.1)

### Arbitration Testing

For multi-master variants: - Round-robin fairness verification - Contention handling - Grant persistence validation - No starvation testing

### Stress Testing

High-intensity scenarios: - Back-to-back transactions - Random delays - Mixed read/write patterns - Concurrent multi-master access

### APB5 Sideband Gating (mixed variant)

For apbx_xbar_2to2_mixed: - All four APB4/APB5 master/slave pairings exercised - Sideband signals flow only where both ends are APB5 - APB4 ports carry no sideband pins (structural check) - Decode-miss completes with PSLVERR

### Timing and Protocol Compliance

test_apbx_xbar_timing asserts the published figures against the RTL: - Single-transfer latency for single-master and arbitrated classes - Back-to-back cadence (PREADY-to-PREADY) - Forward-path and response-path breakdown - Downstream setup-phase count ([1,1,1,1,1])

## Integration Testing Recommendations

### Level 1: Basic Integration

Level 1 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| Reset test | Apply reset, verify outputs | All outputs at safe values |
| Single transaction | Read from each slave | Correct data returned |
| Write verify | Write and readback | Data matches |

### Level 2: Functional Integration

Level 2 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| All masters | Each master accesses each slave | No errors |
| Arbitration | Concurrent master access | Fair grant rotation |
| Error propagation | Force PSLVERR from slave | Error reaches master |

### Level 3: System Integration

Level 3 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| CPU access | Software reads/writes peripherals | Correct behavior |
| DMA access | DMA transfers via crossbar | Data integrity |
| Mixed traffic | CPU and DMA concurrent | No deadlock, fair access |

## Verification Collateral

### Available Test Infrastructure

Test Infrastructure

| Item | Location | Description |
| --- | --- | --- |
| CocoTB tests | dv/tests/ | Python testbenches |
| Wave configs | dv/tests/GTKW/ | GTKWave configurations |
| Coverage | dv/tests/coverage_reports/ | Merged functional coverage reports |
| Build orchestration | dv/tests/Makefile | Area Makefile delegating to make/tests.mk |
| Formal harness (mixed variant) | formal/apbx_xbar/apbx_xbar_2to2_mixed/ | Sideband-gating proofs |

### Running Verification

Tests are run through the area Makefile (which delegates to the shared make/tests.mk infrastructure); bare pytest is not the supported entry point because the Makefile supplies filelists, levels, and coverage options.

cd projects/components/fabric-gen-ip/apbx-xbar/dv/tests/

# Run all crossbar tests -- clean-all first for a run you intend to trust
make clean-all && make run-all-gate

# Run specific variant
make run-apbx_xbar_2to4-gate

# Run with waveforms (WAVES=1 via the target)
make run-apbx_xbar_2to4-gate-waves
# create_view_cmd() writes the gtkwave command beside the log

## Known Limitations

### Not Tested

Known Limitations

| Scenario | Reason | Recommendation |
| --- | --- | --- |
| Slave timeout | Not implemented | Add external watchdog |
| >16 masters/slaves | Generator limit | Custom modification needed |
| Non-64KB regions | Not supported | Use multiple crossbars |

End of Hardware Architecture Specification

## Related Documentation

- APB Crossbar MAS - Detailed implementation

- PRD.md - Product requirements

- README.md - Quick start guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Micro-Architecture Specification Index

Component: APB Crossbar (MxN Interconnect) Version: 1.1 Date: 2026-10-12 Purpose: Detailed micro-architecture specification for APB Crossbar component

## Document Organization

This specification covers the APB Crossbar: a parametric MxN interconnect that routes multiple APB masters to multiple APB slaves with automatic address-based routing and round-robin arbitration.

### Main Documentation

README.md

## Chapter 1: Architecture

chapters/01_architecture.md

## Chapter 2: Address Decode and Arbitration

chapters/02_address_and_arbitration.md

## Chapter 3: RTL Generator

chapters/03_rtl_generator.md

## Quick Navigation

### For New Users

- Start with README.md for overview and quick start

- Read 01_architecture.md to understand the design

- Study 02_address_and_arbitration.md for operational details

- Reference 03_rtl_generator.md if you need a custom configuration

### For Integration

- Pre-generated variants: See 01_architecture.md Section “Pre-Generated Variants”

- Custom generation: See 03_rtl_generator.md Section “Quick Start”

- Address mapping: See 02_address_and_arbitration.md Section “Address Decode”

- Arbitration behavior: See 02_address_and_arbitration.md Section “Arbitration”

### Common Questions

All answered in README.md, section “Common Questions”.

## Visual Assets

Every diagram in this specification is checked in as both source and rendered output:

- Source Files:

  - assets/graphviz/*.gv - Graphviz source diagrams

  - assets/wavedrom/*.json - WaveJSON timing diagrams

- Rendered Files:

  - assets/png/*.png - PNG format (for document generation)

### Architecture Diagrams

- APB Crossbar Architecture (2x4 Example)

  - Source: assets/graphviz/apbx_xbar_architecture.gv

  - Rendered: assets/png/apbx_xbar_architecture.png

- Address Decode Flow

  - Source: assets/graphviz/address_decode_flow.gv

  - Rendered: assets/png/address_decode_flow.png

### Timing Diagrams

- Round-Robin Arbitration

  - Source: assets/wavedrom/arbitration_round_robin.json

  - Rendered: assets/wavedrom/arbitration_round_robin.png

## Component Overview

### Key Features

- Parametric MxN Configuration: Any combination of M masters and N slaves (up to 16x16)

- Automatic Address Decode: 64KB per slave, simple offset-based routing

- Round-Robin Arbitration: Per-slave fair arbitration, no master starvation

- Back-to-Back Transactions: Accepted without master-side idle cycles, though they do not overlap inside the fabric (10 pclk cycles each for M = 1, 11 for arbitrated variants; one more than that variant’s single-transfer latency — see HAS 5.2)

- Grant Persistence: Grant held through transaction completion

- RTL Generation: Python-based generator for custom configurations

### Pre-Generated Variants

| Module | M×N | Use Case |
| --- | --- | --- |
| apbx_xbar_1to1 | 1×1 | Passthrough, protocol conversion |
| apbx_xbar_2to1 | 2×1 | Multi-master arbitration |
| apbx_xbar_1to4 | 1×4 | Simple SoC peripheral bus |
| apbx_xbar_2to4 | 2×4 | Typical SoC with CPU+DMA |
| apbx_xbar_2to2_mixed | 2×2 | Mixed APB4/APB5 (s0 APB5) |

### Design Philosophy

Proven Building Blocks: - Built from production-tested apb4_slave and apb4_master modules - No new protocol logic - pure composition - Each component independently verified

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants for fast integration - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol → cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp → protocol

## Related Documentation

### Companion Specifications

- APB Crossbar HAS - Hardware Architecture Specification (high-level)

### Project-Level

- PRD.md: ../../PRD.md - Complete product requirements document

- CLAUDE.md: ../../CLAUDE.md - AI assistant integration guide

- README.md: ../../README.md - Quick start guide

### Test Infrastructure

- Test Directory: ../../dv/tests/ - CocoTB + pytest test suite

- Test Results: 100% passing (9/9) — all pre-generated variants plus the dedicated timing test; apbx_xbar_2to2_mixed additionally has a formal harness

### RTL

- Core Modules: ../../rtl/apbx_xbar_*.sv - Pre-generated crossbars

- Wrappers: ../../rtl/wrappers/ - testbench scaffolds (pclk/presetn only), not integration wrappers

- Generator: ../../bin/generate_xbars.py - Python generator script

## Version History

Version 1.0 (2025-10-25): - Initial specification release - Complete visual documentation (3 diagrams) - Generator documentation - All pre-generated variants verified (100% passing)

Version 1.1 (2026-10-12): - Refreshed against current RTL, generator, and DV - Updated verification status (9/9 tests passing, mixed-variant formal harness) - Added --slave-size CLI option to generator table - Updated line-of-code counts (single source of truth remains HAS ch05_performance/03_resources.md)

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Product Requirements Document (PRD)

## APB Crossbar Generator

Version: 1.0 Date: 2025-10-19 Status: Production Ready Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 1. Executive Summary

The APB Crossbar is a parametric generator for configurable MxN APB interconnect fabrics connecting multiple APB masters to multiple APB slaves. It is built from the proven apb4_slave and apb4_master modules: independent round-robin arbitration per slave, automatic address-based routing, and no new protocol logic.

### 1.1 Quick Stats

- Modules: 5 pre-generated variants + generator for custom sizes

- Max Capacity: Up to 16x16 (configurable)

- Architecture: apb4_slave → arbitration + decode → apb4_master

- Status: Production ready, all tests passing

- Generator: Python-based parametric code generation

### 1.2 Project Goals

- Primary: Provide production-quality APB interconnect for SoC integration

- Secondary: Demonstrate parametric RTL generation best practices

- Tertiary: Support both fixed and dynamically-generated variants

## 2. Key Design Principles

### 2.1 Architecture Philosophy

Proven Building Blocks: - Uses apb4_slave.sv and apb4_master.sv from rtl/amba/apb4 - Each module independently tested and production-proven - Crossbar = composition of proven components

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants (1to1, 2to1, 1to4, 2to4, 2to2_mixed) - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol to cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp back to protocol

### 2.2 Design Trade-offs

| Feature | Choice | Rationale |
| --- | --- | --- |
| Arbitration | Round-robin per slave | Fair, simple, predictable |
| Address Map | 64KB per slave | Sufficient for most peripherals |
| Grant Persistence | Hold through response | One transaction per grant; mask rotates after |
| Address Decode | Parallel decode | Low latency, simple logic |

## 3. Architecture Overview

### 3.1 Top-Level Block Diagram

APB Crossbar (M masters × N slaves)
┌─────────────────────────────────────────────────────────────────┐
│                                                                  │
│  Master-Side (APB slaves)                                        │
│  ┌──────────────┐  ┌──────────────┐       ┌──────────────┐     │
│  │ apb4_slave[0] │  │ apb4_slave[1] │  ...  │ apb4_slave[M-1]│     │
│  │ (Master 0)   │  │ (Master 1)   │       │ (Master M-1)  │     │
│  └──────┬───────┘  └──────┬───────┘       └──────┬────────┘     │
│         │                  │                      │              │
│         └──────────────────┴──────────────────────┘              │
│                            │                                     │
│                     cmd/rsp buses                                │
│                            │                                     │
│         ┌──────────────────┴──────────────────────┐              │
│         │                                          │              │
│  ┌──────▼──────┐  ┌──────────────┐  ┌────────────▼──┐          │
│  │ Address     │  │ Arbitration  │  │ Response       │          │
│  │ Decode      │  │ per Slave    │  │ Routing        │          │
│  │ (parallel)  │  │ (round-robin)│  │(combinational) │          │
│  └──────┬──────┘  └──────┬───────┘  └────────┬───────┘          │
│         │                 │                   │                  │
│         └─────────────────┴───────────────────┘                  │
│                            │                                     │
│                     cmd/rsp buses                                │
│                            │                                     │
│         ┌──────────────────┴──────────────────────┐              │
│         │                  │                      │              │
│  ┌──────▼───────┐  ┌──────▼───────┐       ┌──────▼────────┐    │
│  │ apb4_master[0]│  │ apb4_master[1]│  ...  │ apb4_master[N-1]│    │
│  │ (Slave 0)    │  │ (Slave 1)    │       │ (Slave N-1)    │    │
│  └──────────────┘  └──────────────┘       └───────────────┘    │
│                                                                  │
│  Slave-Side (APB masters)                                        │
└──────────────────────────────────────────────────────────────────┘

Figure: APB Crossbar top-level architecture showing 2 masters connected to 4 slaves. Source: docs/apbx_xbar_mas/assets/graphviz/apbx_xbar_architecture.gv | SVG

### 3.2 Address Mapping

BASE_ADDR + 0x0000_0000 → 0x0000_FFFF : Slave 0 (64KB)
BASE_ADDR + 0x0001_0000 → 0x0001_FFFF : Slave 1 (64KB)
BASE_ADDR + 0x0002_0000 → 0x0002_FFFF : Slave 2 (64KB)
BASE_ADDR + 0x0003_0000 → 0x0003_FFFF : Slave 3 (64KB)
...
BASE_ADDR + 0x000F_0000 → 0x000F_FFFF : Slave 15 (64KB)

Default BASE_ADDR: 0x1000_0000

Address Decode Example:

The crossbar extracts the slave index from the upper bits of the address offset:

slave_index = (address - BASE_ADDR) >> 16  // Divide by 64KB (0x10000)

Figure: Address decode flow showing how address 0x10023456 routes to Slave 2. Source: docs/apbx_xbar_mas/assets/graphviz/address_decode_flow.gv | SVG

### 3.3 Arbitration Strategy

Per-Slave Round-Robin: - Each slave has independent arbiter - Master priority rotates after each grant - Grant held from command acceptance through response completion - No master can starve another master

Example:

Slave 0 accessed by M0, M1, M0 → Next grant goes to M1
Slave 1 accessed by M1, M1, M0 → Next grant goes to M1
  (the winner cannot immediately re-win: after an M0 grant the
   mask leaves only M1 eligible)

Figure: Round-robin arbitration timing showing 2 masters competing for Slave 0. Master priority rotates after each grant to ensure fair access. Source: docs/apbx_xbar_mas/assets/wavedrom/arbitration_round_robin.json

## 4. Available Variants

### 4.1 Pre-Generated Modules

| Module | Masters | Slaves | Use Case |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | 1 | 1 | Protocol conversion, testing |
| apbx_xbar_2to1 | 2 | 1 | Multi-master arbitration |
| apbx_xbar_1to4 | 1 | 4 | Address decode, simple SoC |
| apbx_xbar_2to4 | 2 | 4 | Full crossbar, typical SoC |
| apbx_xbar_2to2_mixed | 2 | 2 | Mixed APB4/APB5 (s0 is APB5) |

LOC figures live in HAS ch05_performance/03_resources.md, which is the single source of truth for them. A duplicate column here disagreed with that table by up to 33%.

### 4.2 Wrapper Modules

These are TESTBENCH SCAFFOLDS, not integration building blocks. Each exposes pclk and presetn and nothing else — every APB signal is an internal net driven by the cocotb testbench. They exist because the four legacy tests drive a wrapper rather than the bare crossbar; a design cannot connect anything to one.

| Wrapper | Configuration | Purpose |
| --- | --- | --- |
| apbx_xbar_1to1_wrap | 1×1 | Test scaffold for test_apbx_xbar_1to1.py |
| apbx_xbar_2to1_wrap | 2×1 | Test scaffold for test_apbx_xbar_2to1.py |
| apbx_xbar_1to4_wrap | 1×4 | Test scaffold for test_apbx_xbar_1to4.py |
| apbx_xbar_2to4_wrap | 2×4 | Test scaffold for test_apbx_xbar_2to4.py |

To integrate, instantiate the crossbar itself. apbx_xbar_2to2_mixed has no wrapper precisely because its test drives the DUT directly, which is the better pattern.

## 5. Functional Requirements

### FR-1: Arbitrary MxN Configuration

Priority: P0 (Critical) Status: Implemented and verified

Description: Generate crossbar for any M masters and N slaves (up to 16x16)

Verification: Generator creates valid RTL for all tested configurations

### FR-2: Round-Robin Arbitration

Priority: P0 (Critical) Status: Implemented and verified

Description: Fair per-slave arbitration with rotating master priority

Verification: Arbitration stress tests (130+ transactions for 2to1)

### FR-3: Address-Based Routing

Priority: P0 (Critical) Status: Implemented and verified

Description: Automatic slave selection based on address ranges

Verification: Address decode validation (200+ transactions for 1to4)

### FR-4: Back-to-Back Transactions

Priority: P1 (High) Status: Implemented and verified

Description: Consecutive transactions are accepted with no master-side idle cycles required. They do NOT overlap inside the fabric — apb4_slave is one-command-at-a-time. Sustained cadence is 10 pclk cycles PREADY-to-PREADY for a single-master variant and 11 for an arbitrated one, in each case one more than that variant’s SETUP-to-PREADY latency of 9 or 10 (HAS 5.1/5.2).

Verification: Performance tests show consecutive transactions

### FR-5: Configurable Address Map

Priority: P1 (High) Status: Implemented and verified

Description: BASE_ADDR parameter sets base of address map

Verification: Tests with multiple BASE_ADDR values

## 6. Interface Specifications

### 6.1 Master-Side Interface (APB Slave)

Per-master APB slave interface:

input  logic                    m<i>_apb_PSEL
input  logic                    m<i>_apb_PENABLE
input  logic [ADDR_WIDTH-1:0]   m<i>_apb_PADDR
input  logic                    m<i>_apb_PWRITE
input  logic [DATA_WIDTH-1:0]   m<i>_apb_PWDATA
input  logic [STRB_WIDTH-1:0]   m<i>_apb_PSTRB
input  logic [2:0]              m<i>_apb_PPROT
output logic [DATA_WIDTH-1:0]   m<i>_apb_PRDATA
output logic                    m<i>_apb_PSLVERR
output logic                    m<i>_apb_PREADY

### 6.2 Slave-Side Interface (APB Master)

Per-slave APB master interface:

output logic                    s<j>_apb_PSEL
output logic                    s<j>_apb_PENABLE
output logic [ADDR_WIDTH-1:0]   s<j>_apb_PADDR
output logic                    s<j>_apb_PWRITE
output logic [DATA_WIDTH-1:0]   s<j>_apb_PWDATA
output logic [STRB_WIDTH-1:0]   s<j>_apb_PSTRB
output logic [2:0]              s<j>_apb_PPROT
input  logic [DATA_WIDTH-1:0]   s<j>_apb_PRDATA
input  logic                    s<j>_apb_PSLVERR
input  logic                    s<j>_apb_PREADY

## 7. Parameter Configuration

| Parameter | Type | Default | Range | Description |
| --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | 18-64 for multi-slave variants (the decode part-select needs bit 17); 1-64 for 1to1/2to1 | Address bus width |
| DATA_WIDTH | int | 32 | 8,16,32,64 | Data bus width |
| STRB_WIDTH | int | DATA_WIDTH/8 | - | Strobe width (auto-calc) |
| BASE_ADDR | logic [ADDR_WIDTH-1:0] | 0x10000000 | Any except the top S x 64KB, where BASE_ADDR + span wraps and the range check can never pass | Base address for slave map. Inert on 1to1/2to1 (no decode) |

## 8. Generator Usage

### 8.1 Pre-Generated Variants

Use existing modules directly:

# Available in projects/components/fabric-gen-ip/apbx-xbar/rtl/
apbx_xbar_1to1.sv
apbx_xbar_2to1.sv
apbx_xbar_1to4.sv
apbx_xbar_2to4.sv
apbx_xbar_2to2_mixed.sv

### 8.2 Custom Generation

Generate custom MxN crossbar:

cd projects/components/fabric-gen-ip/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6
python generate_xbars.py --masters 4 --slaves 8 --base-addr 0x80000000

### 8.3 Generator Options

python generate_xbars.py --help

Options:
  --masters M       Number of masters (1-16)
  --slaves N        Number of slaves (1-16)
  --base-addr ADDR  Base address (default: 0x10000000)

## 9. Performance Characteristics

### 9.1 Latency

| Path | M = 1 | M > 1 | Notes |
| --- | --- | --- | --- |
| Command path | 6 | 7 | master SETUP (1) + forward path (5, or 6 with a registered arbiter grant) |
| Response path | 3 | 3 | arbitration is on the command side only |
| Total | 9 | 10 | SETUP->PREADY, uncontended, zero-wait slave (8 / 9 ACCESS->PREADY) |

M = 1 is 1to1 and 1to4, which the generator emits with no arbiter at all. M > 1 is 2to1, 2to4 and 2to2_mixed — including the 2x4 this document calls the typical SoC case. This table gave only the M = 1 figures, unlabelled, until 2026-08-30.

APB’s 2-cycle minimum applies to a directly-attached slave, not through this fabric: the crossbar converts APB→cmd/rsp→APB across registered skid buffers in both directions. See HAS 5.2 for the cycle-by-cycle breakdown and the measurement.

### 9.2 Throughput

- Back-to-back transactions: Supported, but not overlapped — apb4_slave is a one-command-at-a-time FSM, so the next command is captured only after the previous transaction completes

- Maximum rate: ~1 transaction per 10 pclk cycles (1to1, 1to4) or 11 (2to1, 2to4, 2to2_mixed) per master (measured back-to-back at an always-ready slave)

### 9.3 Resource Utilization (Estimated)

See HAS ch05_performance/03_resources.md — that table is the single source of truth. An earlier duplicate here quoted figures 2-3x smaller in every cell; both were unsourced hand estimates, so the duplicate is withdrawn rather than reconciled.

## 10. Verification Status

### 10.1 Test Coverage

| Test | Transactions | Status | Coverage |
| --- | --- | --- | --- |
| test_apbx_xbar_1to1 | 100+ | Pass | Basic connectivity |
| test_apbx_xbar_2to1 | 130+ | Pass | Arbitration stress |
| test_apbx_xbar_1to4 | 200+ | Pass | Address decode |
| test_apbx_xbar_2to4 | 350+ | Pass | Full crossbar stress |
| test_apbx_xbar_2to2_mixed | 8 directed | Pass | All four APB4/APB5 pairings, sideband gating, decode miss, structural pin check |
| test_apbx_xbar_timing | 2 variants | Pass | Published latency/cadence figures, both variant classes |

Overall: 100% passing (9/9), >750 transactions plus the directed mixed-version and timing checks. 2to2_mixed also has a formal harness (formal/apbx_xbar/apbx_xbar_2to2_mixed/).

### 10.2 Test Methodology

Test Structure: - CocoTB framework - Random transaction generation - Variable delay profiles - Address range validation - Arbitration fairness checks

Verification Points: - Correct address routing - Round-robin arbitration - Response integrity - Back-to-back transactions - Error propagation

## 11. Integration Guide

### 11.1 Simple Integration (1to4)

apbx_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master interface
    .m0_apb_PSEL(cpu_psel),
    .m0_apb_PENABLE(cpu_penable),
    .m0_apb_PADDR(cpu_paddr),
    .m0_apb_PWRITE(cpu_pwrite),
    .m0_apb_PWDATA(cpu_pwdata),
    .m0_apb_PSTRB(cpu_pstrb),
    .m0_apb_PPROT(cpu_pprot),
    .m0_apb_PRDATA(cpu_prdata),
    .m0_apb_PSLVERR(cpu_pslverr),
    .m0_apb_PREADY(cpu_pready),

    // Slave 0: UART (0x1000_0000 - 0x1000_FFFF)
    .s0_apb_PSEL(uart_psel),
    .s0_apb_PENABLE(uart_penable),
    // ... (similar connections)

    // Slave 1: GPIO (0x1001_0000 - 0x1001_FFFF)
    .s1_apb_PSEL(gpio_psel),
    // ... (similar connections)

    // Slave 2: Timer (0x1002_0000 - 0x1002_FFFF)
    // Slave 3: SPI (0x1003_0000 - 0x1003_FFFF)
);

### 11.2 Multi-Master Integration (2to4)

apbx_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    // ... (full interface)

    // Master 1: DMA
    .m1_apb_PSEL(dma_psel),
    // ... (full interface)

    // Slaves 0-3: Peripherals
    // ... (slave connections)
);

## 12. Design Constraints

### 12.1 Limitations

| Constraint | Value | Rationale |
| --- | --- | --- |
| Max masters | 16 | Generator limit (configurable) |
| Max slaves | 16 | Generator limit (configurable) |
| Slave region size | 64KB in every shipped variant | Baked in at generation (--slave-size, default 4KB); not overridable at elaboration |
| No slave disable | N/A | All slaves always active |
| No timeout | N/A | Assumes slaves always respond |

### 12.2 Assumptions

- Slave responses: All slaves respond eventually (no timeout handling)

- Address ranges: Slaves occupy 64KB regions starting at BASE_ADDR

- APB compliance: All masters and slaves follow APB protocol

- Single clock domain: All signals synchronous to pclk

## 13. Future Enhancements

### 13.1 Potential Features

- Configurable slave region sizes: Not just 64KB

- Slave enable/disable: Dynamic slave activation

- Timeout detection: Watchdog for hung slaves

- Transaction ordering: Optional in-order completion

- Priority arbitration: Weighted instead of round-robin

### 13.2 Generator Improvements

- HDL output formats: Support Verilog, VHDL, Chisel

- Automated testing: Generate tests alongside RTL

- Documentation generation: Auto-generate integration guides

- Synthesis scripts: Generate vendor-specific scripts

## 14. References

### 14.1 Internal Documentation

- README.md - Quick start guide

- CLAUDE.md - AI assistant guidelines

- bin/generate_xbars.py - Generator script

- dv/tests/ - Test implementations

### 14.2 External Standards

- AMBA APB Protocol v2.0 - ARM IHI 0024C

- APB4 Extensions - AMBA 5 specification

### 14.3 Related Components

- rtl/amba/apb4/apb4_slave.sv - Master-side building block

- rtl/amba/apb4/apb4_master.sv - Slave-side building block

- projects/components/fabric-gen-ip/apbx-xbar/bin/apbx_xbar_generator.py - Main generator

Document Version: 1.0 Last Review: 2025-10-19 Next Review: 2026-01-01 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Claude Code Guide: APB Crossbar Component

Version: 1.0 Last Updated: 2025-10-19 Purpose: AI-specific guidance for working with APB Crossbar component

## Quick Context

What: APB Crossbar Generator - Parametric APB interconnect for connecting M masters to N slaves Status: Production Ready (all tests passing) Your Role: Help users generate, integrate, and customize APB crossbars

Complete Specification: projects/components/fabric-gen-ip/apbx-xbar/PRD.md ← Always reference this for technical details

## Critical Rules for This Component

### Rule #0: This is a GENERATOR, Not Just Modules

IMPORTANT: APB Crossbar is both: 1. Pre-generated modules (1to1, 2to1, 1to4, 2to4, 2to2_mixed) in rtl/ 2. Python generator (bin/generate_xbars.py) for custom configurations

When users ask for crossbar: - Check if pre-generated variant exists first - Only suggest generation if custom size needed

Decision Tree:

User needs crossbar?
├─ 1x1, 2x1, 1x4, 2x4? → Use pre-generated module
├─ Mixed APB4/APB5 2x2? → Use apbx_xbar_2to2_mixed
└─ Custom MxN? → Run generator script

### Rule #1: Address Map is Fixed Per-Slave

Each slave occupies 64KB region:

Slave 0: BASE_ADDR + 0x0000_0000 → 0x0000_FFFF
Slave 1: BASE_ADDR + 0x0001_0000 → 0x0001_FFFF
Slave 2: BASE_ADDR + 0x0002_0000 → 0x0002_FFFF
...

The window size is not a module parameter, but it IS a generator option. Every shipped variant uses 64KB windows because bin/generate_xbars.py passes slave_size=0x10000. The generator’s own default is 4KB, and apbx_xbar_generator.py --slave-size accepts any power of 2 >= 256 bytes. Once generated the size is baked into the decode part-select, so it cannot be overridden at elaboration — only BASE_ADDR can, and only on the decoding variants.

If user asks for different sizes:

WRONG: "Let me modify the generator to support custom sizes per slave"

CORRECT: "The shipped variants use 64KB per slave. You can:
1. Use BASE_ADDR to shift the whole map (decoding variants only)
2. For a different window, regenerate with --slave-size (power of 2, >= 256B)
3. Or use multiple crossbars with different BASE_ADDR values"

### Rule #2: Know the Pre-Generated Variants

Available in rtl/ directory:

| Module | M×N | Use Case | When to Recommend |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | 1×1 | Passthrough | Protocol conversion, testing |
| apbx_xbar_2to1 | 2×1 | Arbitration | Multi-master to single peripheral |
| apbx_xbar_1to4 | 1×4 | Decode | Single CPU to multiple peripherals |
| apbx_xbar_2to4 | 2×4 | Full crossbar | CPU + DMA to peripherals |
| apbx_xbar_2to2_mixed | 2×2 | Mixed APB4/APB5 | APB5 peripheral alongside APB4 |

Always suggest pre-generated first!

### Rule #3: Generator Syntax

Generate custom crossbar:

cd projects/components/fabric-gen-ip/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

Options:

--masters M         Number of masters (1-16)
--slaves N          Number of slaves (1-16)
--base-addr ADDR    Base address (default: 0x10000000)

Example:

# Generate 3x8 crossbar with custom base
python generate_xbars.py --masters 3 --slaves 8 --base-addr 0x80000000

# Generate a 5x5 variant
python generate_xbars.py --masters 5 --slaves 5

## Architecture Quick Reference

### Module Organization

projects/components/fabric-gen-ip/apbx-xbar/
├── rtl/
│   ├── apbx_xbar_1to1.sv        Core crossbar modules
│   ├── apbx_xbar_2to1.sv
│   ├── apbx_xbar_1to4.sv
│   ├── apbx_xbar_2to4.sv
│   ├── apbx_xbar_2to2_mixed.sv  (m0=APB4, m1=APB5, s0=APB5, s1=APB4)
│   └── wrappers/               testbench scaffolds, not integration wrappers
│       ├── apbx_xbar_1to1_wrap.sv
│       ├── apbx_xbar_2to1_wrap.sv
│       ├── apbx_xbar_1to4_wrap.sv
│       └── apbx_xbar_2to4_wrap.sv
├── bin/
│   └── generate_xbars.py       Generator script
├── dv/
│   └── tests/
│       ├── test_apbx_xbar_1to1.py
│       ├── test_apbx_xbar_2to1.py
│       ├── test_apbx_xbar_1to4.py
│       ├── test_apbx_xbar_2to4.py
│       ├── test_apbx_xbar_2to2_mixed.py
│       ├── test_apbx_xbar_timing.py     published latency/cadence figures
│       └── GTKW/                Waveform configs
├── PRD.md                       Complete specification
├── CLAUDE.md                    This file
└── README.md                    Quick start guide

### Block Diagram (2×4 Example)

Master 0      Master 1
               │             │
               ▼             ▼
         ┌─────────────────────┐
         │  APB Slaves (M-side) │
         └──────────┬────────────┘
                    │
         ┌──────────┴────────────┐
         │ Address Decode        │
         │ + Arbitration         │
         │ + Response Routing    │
         └──────────┬────────────┘
                    │
         ┌──────────┴────────────┐
         │  APB Masters (S-side) │
         └──────┬────────┬───────┘
                │        │   (more)
           Slave 0   Slave 1 ... Slave 3

## Common User Questions and Responses

### Q: “How do I connect a CPU to 4 peripherals?”

A: Use the pre-generated 1to4 crossbar:

apbx_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)  // Start of peripheral space
) u_peripheral_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // CPU connection (master 0)
    .m0_apb_PSEL(cpu_psel),
    .m0_apb_PENABLE(cpu_penable),
    .m0_apb_PADDR(cpu_paddr),
    .m0_apb_PWRITE(cpu_pwrite),
    .m0_apb_PWDATA(cpu_pwdata),
    .m0_apb_PSTRB(cpu_pstrb),
    .m0_apb_PPROT(cpu_pprot),
    .m0_apb_PRDATA(cpu_prdata),
    .m0_apb_PSLVERR(cpu_pslverr),
    .m0_apb_PREADY(cpu_pready),

    // Peripheral 0: UART @ 0x1000_0000
    .s0_apb_PSEL(uart_psel),
    .s0_apb_PENABLE(uart_penable),
    // ... (full interface)

    // Peripheral 1: GPIO @ 0x1001_0000
    // Peripheral 2: Timer @ 0x1002_0000
    // Peripheral 3: SPI @ 0x1003_0000
);

Address map: - UART: 0x1000_0000 - 0x1000_FFFF - GPIO: 0x1001_0000 - 0x1001_FFFF - Timer: 0x1002_0000 - 0x1002_FFFF - SPI: 0x1003_0000 - 0x1003_FFFF

See: PRD.md Section 11.1

### Q: “I need CPU and DMA to access peripherals. Which crossbar?”

A: Use the 2to4 crossbar:

apbx_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_soc_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    // ... (full interface)

    // Master 1: DMA
    .m1_apb_PSEL(dma_psel),
    // ... (full interface)

    // Slaves 0-3: Peripherals
    // ... (slave connections)
);

Key feature: Round-robin arbitration per slave ensures fair access between CPU and DMA.

See: PRD.md Section 11.2

### Q: “What if I need 3 masters and 8 slaves?”

A: Generate custom crossbar:

cd projects/components/fabric-gen-ip/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 8   # writes apbx_xbar_3to8.sv next to the script

# Check generated file, then move it into rtl/
ls -lh apbx_xbar_3to8.sv
mv apbx_xbar_3to8.sv ../rtl/

Then instantiate like any other crossbar:

apbx_xbar_3to8 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_custom_xbar (
    // ... connections for 3 masters and 8 slaves
);

See: PRD.md Section 8

### Q: “How does arbitration work?”

A: Per-slave round-robin:

Each slave has its own arbiter that rotates master priority:

Example with 2 masters accessing Slave 0:

Transaction 1: M0 requests → M0 granted
Transaction 2: M0 and M1 request → M1 granted (rotated)
Transaction 3: M1 requests → M1 granted
Transaction 4: M0 and M1 request → M0 granted (rotated)

Key points: - Independent per slave: Each slave arbitrates independently - Fair: No master can starve another - Grant persistence: Once granted, master holds slave until response completes - Back-to-back: supported, but not overlapped — 10 pclk cycles per transfer for 1to1/1to4, 11 for the arbitrated variants (2to1, 2to4, 2to2_mixed), PREADY-to-PREADY (see HAS 5.1/5.2)

See: PRD.md Section 3.3

### Q: “Can I change the address map?”

A: BASE_ADDR parameter shifts entire map, but per-slave size is fixed:

// Default: 0x1000_0000
apbx_xbar_1to4 #(.BASE_ADDR(32'h1000_0000)) u_xbar1 (...);
// Slaves at: 0x1000_0000, 0x1001_0000, 0x1002_0000, 0x1003_0000

// Shifted: 0x8000_0000
apbx_xbar_1to4 #(.BASE_ADDR(32'h8000_0000)) u_xbar2 (...);
// Slaves at: 0x8000_0000, 0x8001_0000, 0x8002_0000, 0x8003_0000

Limitation: Each slave always occupies 64KB (0x10000 bytes)

Workaround for different sizes: 1. Use multiple crossbars with different BASE_ADDR 2. Modify generator’s addr_offset calculation 3. Use address masking in slaves

See: PRD.md Section 3.2

### Q: “How do I run tests?”

A: Use the area Makefile (never bare pytest):

cd projects/components/fabric-gen-ip/apbx-xbar/dv/tests/

# Run specific test
make run-apbx_xbar_1to4-gate

# Run all tests -- clean-all first for a run you intend to trust
make clean-all && make run-all-gate

# Run with waveforms (WAVES=1 via the target, not --vcd)
make run-apbx_xbar_2to4-gate-waves
# create_view_cmd() writes the gtkwave command beside the log

Test coverage: - test_apbx_xbar_1to1: 100+ transactions (passthrough) - test_apbx_xbar_2to1: 130+ transactions (arbitration) - test_apbx_xbar_1to4: 200+ transactions (address decode) - test_apbx_xbar_2to4: 350+ transactions (full stress) - test_apbx_xbar_2to2_mixed: 8 directed — all four APB4/APB5 pairings, sideband gating, decode miss, structural pin check - test_apbx_xbar_timing: both variant classes — asserts the published latency, cadence and per-path breakdown so the numbers cannot drift

All tests passing (9/9) — and 2to2_mixed additionally has a formal harness at formal/apbx_xbar/apbx_xbar_2to2_mixed/.

See: PRD.md Section 10

## Integration Patterns

### Pattern 1: Simple Peripheral Interconnect

// CPU to 4 peripherals
apbx_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (
    .pclk(sys_clk),
    .presetn(sys_rst_n),

    // CPU side
    .m0_apb_PSEL(cpu_apb_psel),
    .m0_apb_PENABLE(cpu_apb_penable),
    .m0_apb_PADDR(cpu_apb_paddr),
    .m0_apb_PWRITE(cpu_apb_pwrite),
    .m0_apb_PWDATA(cpu_apb_pwdata),
    .m0_apb_PSTRB(cpu_apb_pstrb),
    .m0_apb_PPROT(cpu_apb_pprot),
    .m0_apb_PRDATA(cpu_apb_prdata),
    .m0_apb_PSLVERR(cpu_apb_pslverr),
    .m0_apb_PREADY(cpu_apb_pready),

    // Peripherals
    .s0_apb_PSEL(uart_psel), /* ... full interface ... */
    .s1_apb_PSEL(gpio_psel), /* ... */
    .s2_apb_PSEL(timer_psel), /* ... */
    .s3_apb_PSEL(spi_psel) /* ... */
);

### Pattern 2: Multi-Master System

// CPU + DMA to peripherals
apbx_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h4000_0000)
) u_soc_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    /* ... full CPU interface ... */

    // Master 1: DMA Controller
    .m1_apb_PSEL(dma_psel),
    /* ... full DMA interface ... */

    // Slave 0-3: Memory-mapped peripherals
    .s0_apb_PSEL(mem_ctrl_psel), /* ... */
    .s1_apb_PSEL(uart_psel), /* ... */
    .s2_apb_PSEL(i2c_psel), /* ... */
    .s3_apb_PSEL(adc_psel) /* ... */
);

### Pattern 3: Hierarchical Interconnect

// Top-level crossbar
apbx_xbar_1to4 u_top_xbar (
    // .m0_apb_PSEL(cpu_apb_PSEL), .m0_apb_PENABLE(...), ...  (elided)
    .s0_apb_PSEL(periph_bus0_PSEL), /* ...rest of the port... */  // To sub-crossbar 0
    .s1_apb_PSEL(periph_bus1_PSEL), /* ...rest of the port... */  // To sub-crossbar 1
    .s2_apb_PSEL(mem_ctrl_PSEL), /* ...rest of the port... */     // Direct to memory controller
    .s3_apb_PSEL(dma_ctrl_PSEL), /* ...rest of the port... */      // Direct to DMA
);

// Sub-crossbar 0 for low-speed peripherals
apbx_xbar_1to4 u_periph_xbar0 (
    .m0_apb_PSEL(periph_bus0_PSEL), /* ...rest of the port... */
    .s0_apb_PSEL(uart0_PSEL), /* ...rest of the port... */
    .s1_apb_PSEL(gpio0_PSEL), /* ...rest of the port... */
    .s2_apb_PSEL(i2c0_PSEL), /* ...rest of the port... */
    .s3_apb_PSEL(spi0_PSEL), /* ...rest of the port... */
);

// Sub-crossbar 1 for high-speed peripherals
apbx_xbar_1to4 u_periph_xbar1 (
    .m0_apb_PSEL(periph_bus1_PSEL), /* ...rest of the port... */
    .s0_apb_PSEL(uart1_PSEL), /* ...rest of the port... */
    .s1_apb_PSEL(timer_PSEL), /* ...rest of the port... */
    .s2_apb_PSEL(pwm_PSEL), /* ...rest of the port... */
    .s3_apb_PSEL(adc_PSEL), /* ...rest of the port... */
);

## Anti-Patterns to Catch

### Anti-Pattern 1: Generating When Pre-Generated Exists

WRONG:
User: "I need a 2×4 crossbar"
You: "Let me generate that for you..."
python generate_xbars.py --masters 2 --slaves 4

CORRECTED:
"Use the pre-generated apbx_xbar_2to4.sv in the rtl/ directory.
No generation needed!"

### Anti-Pattern 2: Assuming Custom Per-Slave Sizes

WRONG:
User: "Can I make slave 0 256KB and slave 1 4KB?"
You: "Sure, let me modify the parameters..."

CORRECTED:
"The shipped variants use 64KB per slave. For a different window:
1. Regenerate with --slave-size (power of 2, >= 256B)
2. Or use multiple crossbars with different BASE_ADDR
3. Or implement address masking in slaves"

### Anti-Pattern 3: Not Mentioning Address Map

WRONG:
User: "How do I integrate the crossbar?"
You: *Shows port connections only*

CORRECTED:
"Here's the integration with address map:
apbx_xbar_1to4 #(.BASE_ADDR(32'h1000_0000)) u_xbar (...);

Address map:
- Slave 0: 0x1000_0000 - 0x1000_FFFF
- Slave 1: 0x1001_0000 - 0x1001_FFFF
- Slave 2: 0x1002_0000 - 0x1002_FFFF
- Slave 3: 0x1003_0000 - 0x1003_FFFF"

### Anti-Pattern 4: Forgetting About Wrappers

WRONG:
User: "I need a quick 10×10 crossbar"
You: "Run the generator with --masters 10 --slaves 10"

CORRECTED:
"We have pre-configured wrappers in rtl/wrappers/:
- apbx_xbar_1to1_wrap.sv, apbx_xbar_2to1_wrap.sv,
  apbx_xbar_1to4_wrap.sv, apbx_xbar_2to4_wrap.sv

Those four are TESTBENCH SCAFFOLDS, not integration wrappers: each has only
pclk and presetn, with every APB signal internal to the cocotb bench, so a
design cannot connect to one. Instantiate the crossbar directly. For any
other MxS, generate one with bin/generate_xbars.py -- and there is no
m10_s10 wrapper in the tree."

## Debugging Workflow

### Issue: Address Not Routing Correctly

Check in order: 1. BASE_ADDR parameter set correctly? 2. Address within slave’s 64KB region? 3. PSEL signal asserted by master? 4. All APB signals properly connected?

Calculate expected slave:

slave_index = (address - BASE_ADDR) >> 16  // Divide by 64KB

Debug commands:

cd dv/tests && make run-apbx_xbar_1to4-gate-waves
# gtkwave command is written beside the log by create_view_cmd()

### Issue: Arbitration Not Fair

Symptoms: - One master dominates - Other masters starved

Check: 1. Arbiters instantiated per slave? 2. Round-robin logic correct? 3. Grant persistence working?

Verify with tests:

cd dv/tests && make run-apbx_xbar_2to1-gate   # Arbitration stress test

### Issue: Back-to-Back Transactions Stalling

Check: 1. Grant persistence enabled? 2. Slaves responding with PREADY? 3. No unintended pipeline bubbles?

View waveforms:

cd dv/tests && make run-apbx_xbar_2to4-gate-waves
# gtkwave command is written beside the log by create_view_cmd()

## Quick Commands

# List available crossbars
ls projects/components/fabric-gen-ip/apbx-xbar/rtl/*.sv

# Generate custom crossbar
cd projects/components/fabric-gen-ip/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

# Run all tests
cd projects/components/fabric-gen-ip/apbx-xbar/dv/tests
make clean-all && make run-all-gate

# Run specific test with waveforms
make run-apbx_xbar_2to4-gate-waves

# View waveforms: create_view_cmd() writes a ready-made command
# beside the log; no hand-built gtkwave path needed

# Check documentation
cat projects/components/fabric-gen-ip/apbx-xbar/PRD.md
cat projects/components/fabric-gen-ip/apbx-xbar/README.md

## Remember

- Check pre-generated first - Don’t generate unnecessarily

- Address map matters - Always mention BASE_ADDR + 64KB regions

- Fair arbitration - Round-robin per slave

- Complete connections - All APB signals must be wired

- Tests available - 100% passing, comprehensive coverage

- Wrappers are TEST SCAFFOLDS - rtl/wrappers/ carries only pclk/presetn; instantiate the crossbar directly (see Anti-Pattern 4)

- Generator limits - Up to 16×16 (configurable)

Version: 1.0 Last Updated: 2025-10-19 Maintained By: RTL Design Sherpa Project