RTL Design Sherpa

Delta Hardware Architecture Specification 1.0

October 12, 2026

Table of Contents

1 Delta Has Index	14

2 Delta Micro-Architecture Specification Index	15

2.1 Overview	16

2.1.1 Key Features	16

2.1.2 Companion Specifications	16

2.2 Navigation	17

2.2.1 Front Matter	17

2.2.2 Chapter 1: Introduction	17

2.2.3 Chapter 2: Block Descriptions	17

2.2.4 Chapter 3: Cycle-Level Behavior	17

2.2.5 Chapter 4: Generated RTL	17

2.2.6 Chapter 5: Verification	17

3 Delta: AXI-Stream Crossbar Generator - Product Requirements Document	18

3.1 Executive Summary	19

3.2 1. Project Goals	20

3.2.1 1.1 Primary Goals	20

3.2.2 1.2 Non-Goals	20

3.3 2. Architecture	21

3.3.1 2.1 Flat Crossbar Topology	21

3.3.2 2.2 Tree Topology (retired 2026-10-12 — never implemented in the framework generator)	21

3.4 3. Functional Requirements	23

3.4.1 3.1 Code Generation	23

3.4.2 3.2 Performance Modeling	23

3.4.3 3.3 Protocol Compliance	23

3.5 4. Non-Functional Requirements	25

3.5.1 4.1 Performance Targets	25

3.5.2 4.2 Resource Targets	25

3.5.3 4.3 Code Quality	25

3.6 5. Interface Specifications	26

3.6.1 5.1 AXI-Stream Master Interface (S_AXIS)	26

3.6.2 5.2 AXI-Stream Slave Interface (M_AXIS)	26

3.6.3 5.3 Clock and Reset	27

3.7 6. Key Design Decisions	28

3.7.1 6.1 Why AXI-Stream vs AXI4?	28

3.7.2 6.2 Why Round-Robin vs Fixed Priority?	28

3.7.3 6.3 Why Flat AND Tree Topologies?	28

3.7.4 6.4 Why Python Generator vs Manual RTL?	28

3.8 7. Comparison to APB Crossbar	29

3.8.1 7.1 Similarities (~95% Code Reuse)	29

3.8.2 7.2 Differences	29

3.8.3 7.3 Migration Effort	29

3.9 8. Use Cases	30

3.9.1 8.1 Primary: RISC Cores + DSP Array	30

3.9.2 8.2 Secondary: Educational Demonstration	30

3.9.3 8.3 Future: Integration with RAPIDS	30

3.10 9. Verification Strategy	31

3.10.1 9.1 Generator Verification	31

3.10.2 9.2 RTL Verification	31

3.10.3 9.3 Model Validation	31

3.11 10. Documentation Requirements	32

3.11.1 10.1 Specifications	32

3.11.2 10.2 Code Documentation	32

3.12 11. Success Metrics	33

3.12.1 11.1 Functional Metrics	33

3.12.2 11.2 Performance Metrics	33

3.12.3 11.3 Quality Metrics	33

3.13 12. Timeline and Milestones	34

3.13.1 Week 1: Generator and Models	34

3.13.2 Week 2: RTL and Verification	34

3.13.3 Week 3: Integration and Documentation	34

3.14 13. Open Questions	35

3.15 14. Revision History	36

3.16 16. Documentation	37

3.17 Appendix A: Glossary	38

4 Claude Code Guide: Delta (AXI-Stream Crossbar Generator)	39

4.1 Quick Context	40

4.2 How It Actually Works (verified against rtl/delta_axis_flat_4x16.sv)	41

4.3 Repository Map	42

4.4 Critical Rules	43

4.5 Common Tasks	44

5 Delta Project - Quick Start Guide	45

5.1 What You Have	46

5.1.1 [PASS] Working Code Generator (697 lines)	46

5.1.2 [PASS] Performance Modeling (487 lines)	46

5.1.3 [PASS] Complete Specifications	46

5.1.4 [PASS] Generated RTL Example	46

5.2 How Delta Differs from Your APB Crossbar Generator	47

5.2.1 Key Insight: 95% Code Reuse, ~75 Minutes to Adapt	47

5.2.2 Detailed Comparison	47

5.2.3 Why AXIS is Actually Simpler Than APB	47

5.3 Quick Test Drive	48

5.3.1 1. Generate Your First Crossbar (30 seconds)	48

5.3.2 2. Run Performance Analysis	48

5.3.3 3. Inspect Generated RTL	48

5.3.4 4. Generate Test Configurations	48

5.4 Project Structure	50

5.5 Next Steps	51

5.5.1 Option A: Use Provided Generator Immediately	51

5.5.2 Option B: Adapt Your APB Generator	51

5.5.3 Option C: Review Specs First (Your Preferred Approach)	51

5.6 Generator Command Reference	52

5.6.1 Basic Usage	52

5.6.2 Examples	52

5.6.3 Options	52

5.7 Performance Model Command Reference	54

5.7.1 Basic Usage	54

5.7.2 Examples	54

5.8 Key Features	55

5.8.1 1. Dual Topology Support	55

5.8.2 2. Performance Modeling	55

5.8.3 3. Complete Specifications	55

5.9 Demonstrations of Rigor	56

5.9.1 1. Complete Specifications (Before Code)	56

5.9.2 2. Performance Modeling (Before Implementation)	56

5.9.3 3. Architecture Comparison	56

5.9.4 4. Integration with Existing Automation	56

5.10 Educational Value	57

5.11 Status Summary	58

5.11.1 [PASS] Complete	58

5.11.2 [ ] TODO (Future Work)	58

5.12 Quick Reference	59

5.13 Summary	60

6 Document Information	61

6.1 Delta Hardware Architecture Specification	61

6.2 Revision History	62

6.3 Document Purpose	63

6.4 Intended Audience	64

6.5 Related Documents	65

7 Purpose and Scope	66

7.1 What is Delta?	66

7.2 The Problem Delta Solves	67

7.3 Scope	68

7.4 Out of Scope	69

7.5 Non-Goal: Memory-Mapped Protocol Conversion	70

8 Document Conventions	71

8.1 Naming Conventions	71

8.2 Signal Direction Conventions	72

8.3 Numbering Conventions	73

8.4 Typographic Conventions	74

8.5 Versioning Conventions	75

9 Definitions and Acronyms	76

9.1 Acronyms	76

9.2 Key Terms	77

10 Use Cases	78

10.1 Primary: RISC Cores + DSP Array	78

10.2 Secondary: Sensor/Peripheral Fabric	79

10.3 Tertiary: AMBA 5-Aware Streaming Interconnect	80

10.4 Educational Demonstration	81

11 Key Features	82

11.1 Flat M×N Crossbar	82

11.2 TDEST-Based Routing	83

11.3 Packet-Atomic Round-Robin Arbitration	84

11.4 Combinational Data Mux with Registered Grant	85

11.5 Optional Richness (Default Off)	86

11.6 Optional Packet Counters	87

11.7 Formal Assertions	88

11.8 Code Generation	89

12 System Context	90

12.1 Where Delta Fits	90

12.2 Upstream Requirements	91

12.3 Downstream Requirements	92

12.4 Clocking and Reset	93

12.5 Relationship to Other Components	94

13 Block Diagram	95

13.1 Delta Architecture Overview	95

13.1.1 Figure 3.1: Delta High-Level Block Diagram	95

13.2 Functional Blocks	97

13.2.1 TDEST Decode	97

13.2.2 Per-Slave Round-Robin Arbiter	97

13.2.3 Crossbar Mux	97

13.2.4 TREADY Steering	97

13.2.5 TWAKEUP Steering (AMBA5, Optional)	97

13.2.6 Packet Counters (Optional)	97

13.3 Scalability	98

13.3.1 Resource Scaling	98

14 Data Flow	99

14.1 Request Flow	99

14.2 Payload Flow	100

14.3 Backpressure Flow	101

14.4 Wakeup Flow (AMBA5)	102

14.5 Packet Boundaries	103

15 Protocol Support	104

15.1 AXI4-Stream Base Flavor	104

15.2 Optional TSTRB Byte Qualifiers	105

15.3 Optional AMBA 5 Sidebands	106

15.4 What Delta Does Not Support	107

15.5 Protocol Compliance Summary	108

16 AXI-Stream Base Interface	109

16.1 Master Interface (S_AXIS)	109

16.2 Slave Interface (M_AXIS)	110

16.3 Signal Semantics	111

17 AXI-Stream Rich Interface (TSTRB + AMBA5)	112

17.1 Rich Master Interface (S_AXIS)	112

17.2 Rich Slave Interface (M_AXIS)	113

17.3 TSTRB Semantics	114

17.4 TWAKEUP Semantics	115

17.5 TPARITY Semantics	116

17.6 Parameter Implications	117

18 Clock and Reset	118

18.1 Clock	118

18.2 Reset	119

18.3 Clocking Rules	120

18.4 Reset Recommendations	121

19 Throughput Characteristics	122

19.1 Aggregate Throughput	122

19.2 Per-Slave Throughput	123

19.3 Head-of-Line Blocking	124

19.4 Throughput Targets	125

19.5 Factors That Reduce Throughput	126

20 Latency Analysis	127

20.1 Cycle Breakdown	127

20.2 Arbitration Delay	128

20.3 Backpressure Latency	129

20.4 Latency Targets	130

20.5 Frequency Target	131

21 Resources	132

21.1 Estimates from the Performance Model	132

21.2 Scaling Laws	133

21.3 Area Optimization Knobs	134

22 System Requirements	135

22.1 Toolchain Requirements	135

22.2 Runtime Requirements	136

22.3 Interface Assumptions	137

22.4 Integration Checklist	138

23 Parameter Configuration	139

23.1 Generated Module Parameters	139

23.2 Generator CLI Options	140

23.3 Example Configurations	141

23.3.1 Base flavor 4×16 @ 64-bit	141

23.3.2 Rich flavor 2×4 @ 32-bit with TSTRB and AMBA5	141

23.3.3 Base flavor without counters	141

24 Verification Strategy	142

24.1 Current Status	142

24.2 Generator Verification	143

24.3 Lint Verification	144

24.4 Planned RTL Verification	145

24.5 Formal Properties	146

25 Synthesis and Implementation	147

25.1 Synthesis Flow	147

25.2 Timing Considerations	148

25.3 Floorplanning Suggestions	149

25.4 Implementation Checklist	150

List of Figures

Figure 3.1: Delta High-Level Block Diagram	95

List of Tables

Table 1: Table 3.1: Topology Parameter Scaling	98

Table 2: Table 3.2: Protocol Support Matrix	108

Table 3: Table 4.1: Base Master Interface Signals	109

Table 4: Table 4.2: Base Slave Interface Signals	110

Table 5: Table 4.3: Rich Master Interface Additional Signals	112

Table 6: Table 4.4: Rich Slave Interface Additional Signals	113

Table 7: Table 4.5: Rich-Flavor Parameter-to-Port Mapping	117

Table 8: Table 5.1: Throughput Targets	125

Table 9: Table 5.2: Latency Cycle Breakdown	127

Table 10: Table 5.3: Latency Targets	130

Table 11: Table 5.4: Resource Estimates	132

Table 12: Table 5.5: Area Optimization Options	134

Table 13: Table 6.1: Generated Module Parameters	139

Table 14: Table 6.2: Generator Command-Line Options	140

# 1	Delta Has Index

Generated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Delta Micro-Architecture Specification Index

Component: Delta (AXI4-Stream Crossbar Generator) Version: 1.0 Date: 2026-10-12 Purpose: Detailed micro-architecture specification for the generated Delta RTL

## 2.1	Overview

Delta is a Python generator that emits a single self-contained SystemVerilog module per configuration — a flat M×N AXI4-Stream crossbar routing s_axis_* masters to m_axis_* slaves. This is the internals document: the one you open when you need to know what the generated RTL actually does, block by block, rather than what the high-level spec says.

### 2.1.1	Key Features

- Flat crossbar only: Tree generation was retired on 2026-10-12.

- TDEST-based routing: No address map; the destination slave is taken directly from s_axis_tdest.

- Packet-atomic round-robin arbitration: A granted master holds the slave until TLAST.

- Combinational data mux: The grant is the registered stage; payload passes through combinatorially.

- Optional richness: TSTRB byte qualifiers and AMBA 5 TWAKEUP/TPARITY sidebands (default off).

- Optional counters: Per-master and per-slave packet counters (disable with --no-counters).

- Formal assertions: ifdef FORMAL checks for one-hot grants and in-range TDEST.

### 2.1.2	Companion Specifications

- Delta HAS - Hardware Architecture Specification (high-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

## 2.2	Navigation

### 2.2.1	Front Matter

- Document Information

### 2.2.2	Chapter 1: Introduction

- Overview

### 2.2.3	Chapter 2: Block Descriptions

- Request Decode

- Per-Slave Round-Robin Arbiter

- Crossbar Multiplexer

- TREADY Steering

- TWAKEUP Steering (AMBA5)

- Packet Counters

- Assertions

### 2.2.4	Chapter 3: Cycle-Level Behavior

- Multi-Master Contention

- Packet Interleave Prevention

### 2.2.5	Chapter 4: Generated RTL

- Module Structure

- Parameters and Logic Implications

### 2.2.6	Chapter 5: Verification

- Test Strategy

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Delta: AXI-Stream Crossbar Generator - Product Requirements Document

Project: Delta Version: 1.0 Date: 2025-10-18 Status: Active Development

## 3.1	Executive Summary

Delta is an AXI-Stream crossbar generator that produces parameterized RTL for routing data between multiple masters and multiple slaves. The name “Delta” follows the water/river theme (like RAPIDS) - deltas are where rivers split into multiple branches, analogous to crossbar routing.

Key Features: - Python-based RTL generation (similar to APB crossbar automation) - Single topology support: flat crossbar (tree topology retired 2026-10-12 — see QUICK_START.md) - Performance modeling (analytical + simulation) - Complete AXI-Stream protocol compliance - Educational focus with rigorous specifications

## 3.2	1. Project Goals

### 3.2.1	1.1 Primary Goals

- Code Generation Excellence

  - Python generator produces clean, parameterized SystemVerilog

  - Reuses patterns from existing APB crossbar automation (~95% code reuse)

  - Single tool generates multiple topologies

- Performance Rigor

  - Analytical models (closed-form latency/throughput)

  - Discrete event simulation (SimPy)

  - Resource estimation

  - Validation against RTL synthesis

- Educational Value

  - Demonstrates specification-driven design

  - Shows code generation techniques

  - Compares topology trade-offs

  - Suitable for instruction on GitHub

- Integration Ready

  - Works with existing CocoTBFramework

  - Compatible with RAPIDS subsystem

  - Supports RISC core + DSP array use case

### 3.2.2	1.2 Non-Goals

- Vendor-specific IP (using custom generators only)

- Protocol conversion (AXIS only, not AXI4/APB)

- Advanced routing algorithms (static TDEST-based only)

- Dynamic reconfiguration

## 3.3	2. Architecture

### 3.3.1	2.1 Flat Crossbar Topology

Description: Full MxN crossbar with per-slave arbiters

Characteristics: - Latency: 2 cycles (arbitration + output register) - Throughput: High aggregate (each slave independent) - Resources: ~1,920 LUTs for 4x16 @ 64-bit - Best For: Production systems requiring low latency

Block Diagram:

Masters (M)          Arbiters (N)         Slaves (N)

M0 ------+----------+-[Arbiter_S0]----- S0
         |          |
M1 ------+          +-[Arbiter_S1]----- S1
         |          |
M2 ------+          +-[Arbiter_S2]----- S2
         |          |
M3 ------+          +-[Arbiter_S15]---- S15

Request Matrix      Grant Matrix
(decode TDEST)      (round-robin)

### 3.3.2	2.2 Tree Topology (retired 2026-10-12 — never implemented in the framework generator)

Description: Hierarchical composition of 1:2 splitters and 2:1 mergers

Characteristics: - Latency: 4-6 cycles (multi-stage pipeline) - Throughput: Lower aggregate (bottleneck at root) - Resources: ~1,600 LUTs for 4x16 @ 64-bit (fewer LUTs but more instances) - Best For: Educational examples, demonstrating modularity

Block Diagram:

Masters     Mergers        Splitters       Slaves

M0 --+
     +-[2:1]--[1:2]--+-[1:2]--+- S0
M1 --+              |         +- S1
                    |
M2 --+              +-[1:2]--+- S2
     +-[2:1]--------+         +- S3
M3 --+

Tree depth: log2(N) stages

## 3.4	3. Functional Requirements

### 3.4.1	3.1 Code Generation

REQ-GEN-001: Python generator shall produce SystemVerilog RTL - Input: Command-line parameters (masters, slaves, data width, topology) - Output: Synthesizable SystemVerilog modules - Verification: Verilator lint clean, synthesis clean

REQ-GEN-002: Generator shall support flat crossbar topology (tree topology retired 2026-10-12 — see QUICK_START.md) REQ-GEN-003: Generator shall support optional TSTRB byte qualifiers (--strb-width N; default 0 = base signal set) so generated crossbars port-match rtl/amba/axis4/* (added 2026-10-12). REQ-GEN-004: Generator shall support optional AMBA 5 sidebands (--amba5): TWAKEUP steering — TDEST-decoded OR per slave, valid before TVALID — and TPARITY (--parity-width, default DATA_WIDTH/8), matching rtl/amba/axis5/* (added 2026-10-12). Both options default off; default output is byte-identical to pre-2026-10-12 generations. - Flat: Single crossbar module - Tree: Hierarchical node composition - Both: Generate both variants with --topology both (retired 2026-10-12; flat only)

REQ-GEN-003: Generated RTL shall be parameterized - NUM_MASTERS, NUM_SLAVES (up to 32x256) - DATA_WIDTH (8, 16, 32, 64, 128, 256, 512, 1024 bits) - DEST_WIDTH, ID_WIDTH (auto-calculated)

REQ-GEN-004: Generator shall follow APB crossbar patterns - ~95% code reuse from APB automation - Same request generation, arbitration, mux patterns - Signal name mapping (pclk->aclk, psel->tvalid, etc.)

### 3.4.2	3.2 Performance Modeling

REQ-MODEL-001: Analytical model shall provide closed-form results - Latency calculation (cycles and nanoseconds) - Throughput estimation (transfers/cycle, Gbps) - Resource estimation (LUTs, FFs)

REQ-MODEL-002: Simulation model shall use discrete event simulation - SimPy-based cycle-accurate model - Statistical analysis (mean, percentiles) - Traffic pattern support (uniform, hotspot, localized)

REQ-MODEL-003: Models shall be validated against RTL - Compare analytical vs simulation results - Validate against synthesis reports - Document discrepancies

### 3.4.3	3.3 Protocol Compliance

REQ-PROTO-001: Generated RTL shall comply with AXI-Stream specification - TVALID/TREADY handshaking - TLAST packet boundaries - TDEST, TID, TUSER sideband signals

REQ-PROTO-002: Arbitration shall provide packet atomicity - Grant locked until TLAST - No packet interleaving - Fair round-robin arbitration

REQ-PROTO-003: Backpressure shall propagate correctly - TREADY from slave to granted master - No deadlocks - No data loss

## 3.5	4. Non-Functional Requirements

### 3.5.1	4.1 Performance Targets

NFR-PERF-001: Flat crossbar latency <= 2 cycles NFR-PERF-002: Flat crossbar Fmax >= 300 MHz (UltraScale+) NFR-PERF-003: Tree topology latency <= 6 cycles (retired with tree topology, 2026-10-12) NFR-PERF-004: Throughput >= 0.7 transfers/cycle under mixed traffic

### 3.5.2	4.2 Resource Targets

NFR-RES-001: Flat 4x16 @ 64-bit <= 2,500 LUTs NFR-RES-002: Tree 4x16 @ 64-bit <= 2,000 LUTs NFR-RES-003: No BRAM usage (pure logic implementation)

### 3.5.3	4.3 Code Quality

NFR-QUAL-001: Generated RTL shall pass Verilator lint NFR-QUAL-002: Generator code shall follow PEP 8 style NFR-QUAL-003: Comprehensive inline documentation NFR-QUAL-004: Assertions for formal verification

## 3.6	5. Interface Specifications

### 3.6.1	5.1 AXI-Stream Master Interface (S_AXIS)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_tdata[m] | DATA_WIDTH | Input | Data payload |
| s_axis_tvalid[m] | 1 | Input | Valid indicator |
| s_axis_tready[m] | 1 | Output | Ready (backpressure) |
| s_axis_tlast[m] | 1 | Input | Packet boundary |
| s_axis_tdest[m] | DEST_WIDTH | Input | Target slave ID |
| s_axis_tid[m] | ID_WIDTH | Input | Transaction ID |
| s_axis_tuser[m] | USER_WIDTH | Input | User sideband |

Array: [NUM_MASTERS] (one per master)

### 3.6.2	5.2 AXI-Stream Slave Interface (M_AXIS)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_tdata[s] | DATA_WIDTH | Output | Data payload |
| m_axis_tvalid[s] | 1 | Output | Valid indicator |
| m_axis_tready[s] | 1 | Input | Ready (backpressure) |
| m_axis_tlast[s] | 1 | Output | Packet boundary |
| m_axis_tdest[s] | DEST_WIDTH | Output | Target slave ID (pass-through) |
| m_axis_tid[s] | ID_WIDTH | Output | Transaction ID (pass-through) |
| m_axis_tuser[s] | USER_WIDTH | Output | User sideband (pass-through) |

Array: [NUM_SLAVES] (one per slave)

### 3.6.3	5.3 Clock and Reset

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| aclk | 1 | Input | Clock |
| aresetn | 1 | Input | Active-low reset |

## 3.7	6. Key Design Decisions

### 3.7.1	6.1 Why AXI-Stream vs AXI4?

Decision: Use AXI-Stream instead of full AXI4

Rationale: - Streaming workloads (RISC cores, DSP arrays) - Simpler protocol (no address phases, burst management) - Better fit for compute fabrics - Still allows future upgrade to AXI4 if needed

### 3.7.2	6.2 Why Round-Robin vs Fixed Priority?

Decision: Default to round-robin arbitration

Rationale: - Fair bandwidth allocation - No starvation - Suitable for general-purpose compute - Can add fixed-priority mode later if needed

### 3.7.3	6.3 Why Flat AND Tree Topologies?

Decision: Support both, not either/or

Rationale: - Flat for production (low latency, high throughput) - Tree for education (modular, demonstrates composition) - Python generator makes both easy (~zero extra cost) - Shows trade-off analysis

### 3.7.4	6.4 Why Python Generator vs Manual RTL?

Decision: Python-based code generation

Rationale: - Reuse existing APB crossbar automation (~95%) - Parameterization without manual edits - Consistent code quality - Educational value (code generation techniques) - Rapid design space exploration

## 3.8	7. Comparison to APB Crossbar

### 3.8.1	7.1 Similarities (~95% Code Reuse)

| Component | APB Crossbar | Delta (AXIS) | Reusable? |
| --- | --- | --- | --- |
| Request generation | Address decode | TDEST decode | [PASS] Pattern same |
| Per-slave arbitration | Round-robin | Round-robin | [PASS] Identical logic |
| Grant matrix | MxN grants | MxN grants | [PASS] Identical |
| Data multiplexing | Mux PRDATA | Mux TDATA/TVALID/TLAST | [PASS] Same pattern |
| Backpressure | PREADY | TREADY | [PASS] Renamed only |

### 3.8.2	7.2 Differences

| Aspect | APB Crossbar | Delta (AXIS) |
| --- | --- | --- |
| Address decode | Range check (base/end) | Direct TDEST (simpler!) |
| Packet concept | Single transaction | Multi-beat packets with TLAST |
| Arbitration | Re-arbitrate every cycle | Lock until TLAST (packet atomicity) |
| Signals | PRDATA, PSLVERR | TDATA, TVALID, TLAST, TDEST, TID, TUSER |
| Read/Write | Separate paths | Unified data path (simpler!) |

### 3.8.3	7.3 Migration Effort

Estimated Time: ~75 minutes (from existing APB generator to AXIS)

Tasks: 1. Rename signals (10 min) 2. Simplify address decode to TDEST (5 min) 3. Add packet atomicity logic (15 min) 4. Add new signals (TLAST, TID, TUSER) to ports and mux (10 min) 5. Update module naming (5 min) 6. Test with 2x2 configuration (30 min)

## 3.9	8. Use Cases

### 3.9.1	8.1 Primary: RISC Cores + DSP Array

Scenario: 4 small RISC-V cores need to route compute tasks to 16 DSP accelerators

Configuration: - 4 masters (RISC cores) - 16 slaves (DSP arrays) - 64-bit data width - Flat topology (low latency critical)

Benefits: - Each RISC core can target any DSP (full flexibility) - Round-robin ensures fair DSP access - Low 2-cycle latency for task dispatch

### 3.9.2	8.2 Secondary: Educational Demonstration

Scenario: Teach students about interconnect design trade-offs

Configuration: - Generate flat crossbars (tree topology retired 2026-10-12) - Compare latency, throughput, resources - Show code generation techniques - Demonstrate verification methodology

Benefits: - Hands-on learning with working RTL - Clear performance comparisons - Demonstrates real-world design process

### 3.9.3	8.3 Future: Integration with RAPIDS

Scenario: Use Delta for compute fabric routing, RAPIDS for memory DMA

Configuration: - RAPIDS handles descriptor-based memory transfers - Delta routes compute tasks between processors - Protocol adapter (Network 2.0 <-> AXI-Stream)

Benefits: - Separate concerns (memory vs compute) - Reusable components - Scalable architecture

## 3.10	9. Verification Strategy

### 3.10.1	9.1 Generator Verification

Approach: 1. Unit tests for each generator function (Python unittest) 2. Lint generated RTL (Verilator) 3. Synthesis test (Vivado/Yosys) 4. Compare output for different parameters

Success Criteria: - All Python tests pass - Generated RTL lints clean - Synthesis completes without errors

### 3.10.2	9.2 RTL Verification

Approach: 1. CocoTB testbench framework (reuse AMBA patterns) 2. Test all MxS routing combinations 3. Concurrent traffic scenarios 4. Backpressure stress tests 5. Packet atomicity verification

Success Criteria: - 100% routing coverage - No deadlocks under stress - Packet atomicity confirmed - Performance matches model (±10%)

### 3.10.3	9.3 Model Validation

Approach: 1. Analytical model vs simulation model comparison 2. Simulation model vs RTL CocoTB results 3. Synthesis reports vs resource estimates

Success Criteria: - Latency within ±1 cycle - Throughput within ±15% - Resources within ±20%

## 3.11	10. Documentation Requirements

### 3.11.1	10.1 Specifications

PRD (this document) - Requirements, architecture, use cases

Technical Specification - Detailed block diagrams - Interface timing diagrams - Performance analysis

User Guide - Generator usage examples - Integration patterns - Best practices

### 3.11.2	10.2 Code Documentation

Generator Code: - Docstrings for all functions - Inline comments for complex logic - Usage examples in header

Generated RTL: - Module header with configuration - Block-level comments - Signal descriptions

Performance Models: - Algorithm explanations - Formula derivations - Validation methodology

## 3.12	11. Success Metrics

### 3.12.1	11.1 Functional Metrics

- [PASS] Generator produces lint-clean RTL

- [PASS] All routing combinations verified

- [PASS] No protocol violations

- [PASS] Packet atomicity enforced

### 3.12.2	11.2 Performance Metrics

- [PASS] Flat crossbar: 2-cycle latency

- [PASS] Tree topology: <=6-cycle latency

- [PASS] Throughput: >=0.7 transfers/cycle

- [PASS] Fmax: >=300 MHz

### 3.12.3	11.3 Quality Metrics

- [PASS] Code reuse: >=90% from APB generator

- [PASS] Model accuracy: ±10% vs RTL

- [PASS] Resource estimate: ±20% vs synthesis

- [PASS] Documentation: Complete and clear

## 3.13	12. Timeline and Milestones

### 3.13.1	Week 1: Generator and Models

- [PASS] Python generator (flat topology)

- [PASS] Analytical performance model

- [PASS] Simulation model (SimPy)

- [PASS] Specifications

### 3.13.2	Week 2: RTL and Verification

- Generate RTL variants (flat 4x16; tree variant retired 2026-10-12)

- CocoTB testbench framework

- Basic functional tests

- Performance validation

### 3.13.3	Week 3: Integration and Documentation

- RISC + DSP integration example

- Complete user guide

- Performance comparison report

- Educational materials

## 3.14	13. Open Questions

Q1: Should we add configurable FIFO insertion for burst buffering? - Status: Deferred to v1.1 (keep v1.0 simple)

Q2: Support for weighted round-robin arbitration? - Status: Deferred (default round-robin sufficient for now)

Q3: Integration with existing APB crossbar generator (unified tool)? - Status: Under consideration (separate for now, may merge later)

## 3.15	14. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2025-10-18 | RTL Design Sherpa | Initial release |

## 3.16	16. Documentation

The aspirational DELTA mesh NoC specification was split out of this component on 2026-10-12 and lives at projects/components/noc-ip/delta_spec/ (agent guide: projects/components/noc-ip/CLAUDE.md). To render it:

cd projects/components/noc-ip
./generate_pdf.sh --rev 0.25

Rendered DOCX/PDF land in noc-ip/delta_spec/. This component — the AXIS crossbar generator — is documented by README.md, QUICK_START.md, and delta_index.md.

## 3.17	Appendix A: Glossary

- AXIS: AXI-Stream (streaming variant of AXI protocol)

- Delta: Project name (river delta = branching flow, like crossbar routing)

- Flat Topology: Full MxN crossbar with all connections

- Tree Topology: Hierarchical composition of 1:2 and 2:1 nodes

- Packet Atomicity: Locking grant until TLAST (prevent interleaving)

- TDEST: Transaction destination (slave ID in AXI-Stream)

- TID: Transaction ID (master identifier in AXI-Stream)

END OF PRD

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Claude Code Guide: Delta (AXI-Stream Crossbar Generator)

Version: 1.0 Last Updated: 2026-10-12 Purpose: AI-specific guidance for the Delta component (fabric-gen-ip)

## 4.1	Quick Context

What: Delta is a Python-based AXI4-Stream (AXIS) crossbar generator. It emits a single self-contained SystemVerilog module per configuration — a flat M×N stream crossbar routing s_axis_* masters to m_axis_* slaves.

Status: Active development. RTL generates and simulates; there is no dv/ test suite yet (the CocoTB suite is the known TODO). Re-homed from noc-ip/delta on 2026-10-12 — the move did not change any behavior.

Your Role: Help users generate, parameterize, and validate AXIS crossbars; keep generated RTL and this documentation in sync.

Not this component: the aspirational DELTA mesh NoC design draft (routers, virtual channels, XY routing) lives separately at projects/components/noc-ip/delta_spec/ — no NoC RTL exists. When “delta” comes up without qualification, this crossbar generator is what is meant.

## 4.2	How It Actually Works (verified against rtl/delta_axis_flat_4x16.sv)

- Route decode: the target slave comes straight from s_axis_tdest (DEST_WIDTH bits) — there is no address map.

- Arbitration: per-slave round-robin; the grant is packet-atomic — once granted, a master holds the slave until TLAST.

- Data path: TDATA/TVALID/TLAST/TDEST/TID/TUSER crossbar mux is combinational; the grant is the registered stage.

- Backpressure: TREADY from the granted slave is steered back to the granted master only.

- Optional flavors (2026-10-12, default off, default output byte-identical): TSTRB (--strb-width) byte qualifiers, and AMBA 5 TWAKEUP/TPARITY (--amba5, --parity-width default DATA_WIDTH/8) so generated crossbars port-match rtl/amba/axis4/* and rtl/amba/axis5/*. TWAKEUP is TDEST-decoded (OR per slave) and may assert before TVALID, per the AMBA 5 hint semantics.

- Optional: packet counters.

Parameters on the generated module: DATA_WIDTH, DEST_WIDTH, ID_WIDTH, USER_WIDTH, NUM_MASTERS, NUM_SLAVES.

## 4.3	Repository Map

| Path | What |
| --- | --- |
| bin/delta_generator.py | The generator (DeltaConfig, DeltaFlatCrossbar, CLI) |
| bin/delta_performance_model.py | Analytical + simulation performance model |
| bin/complete_tree_generator.py | Retired tree-topology prototype (kept for history) |
| rtl/delta_axis_flat_4x16.sv | Reference generated crossbar (checked in) |
| rtl_test/*.sv | Generated variants (2×4, 3×8, 2×16, 4×16) |
| rtl_test/delta_axis_flat_2x4_strb_amba5.sv | Rich-flavor example (TSTRB + TWAKEUP/TPARITY) |
| rtl/filelists/*.f | Filelists ($REPO_ROOT-relative) |
| README.md / PRD.md / QUICK_START.md | Product docs (PRD REQ-GEN-002 is flat-only since 2026-10-12) |
| delta_index.md | Crossbar doc index |
| TREE_TOPOLOGY_TEST_RESULTS.md | Historical results for the retired tree path |

## 4.4	Critical Rules

- Flat only. --topology tree (and both) prints “not yet implemented in framework version”. Do not promise tree generation; REQ-GEN-002 was amended 2026-10-12.

- Regenerate, don’t hand-edit. rtl/delta_axis_flat_4x16.sv and the rtl_test/ variants are generator outputs. Fix bin/delta_generator.py and re-emit; note --topology tree was retired — regenerate with flat.

- AXIS only. No AXI4/AXI4-Lite/APB conversion here (PRD non-goal). For memory-mapped fabrics see siblings ../bridge/ and ../apbx-xbar/.

- Filelist paths in rtl/filelists/*.f are $REPO_ROOT-relative — keep them working from any cwd.

- Do not resurrect the NoC (noc-ip/delta_spec/) unless the owner explicitly asks for NoC work.

## 4.5	Common Tasks

# Generate a fresh crossbar
python3 bin/delta_generator.py --topology flat --masters 4 --slaves 16 \
    --data-width 64 --output-dir rtl/

# Rich flavor: TSTRB + AMBA 5 sidebands (matches rtl/amba/axis4 + axis5)
python3 bin/delta_generator.py --topology flat --masters 2 --slaves 4 \
    --data-width 32 --strb-width 4 --amba5 --output-dir rtl_test/

# Performance comparison (historical flat/tree data retained)
python3 bin/delta_performance_model.py --topology compare

Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Delta Project - Quick Start Guide

Created: 2025-10-18 Status: [PASS] Complete and Ready to Use

Status (2026-07-22): updated - the tree-topology generation originally described here was retired; delta now generates flat AXIS crossbars only. Current flow: bin/delta_generator.py --topology flat ... -> rtl/delta_axis_flat_4x16.sv (test configurations in rtl_test/), linted via rtl/Makefile (make verilator). The docs/DELTA_VS_APB_GENERATOR.md migration guide was removed; current the NoC design draft moved to ../../noc-ip/delta_spec/ (unimplemented, not this component; rendered PDF kept there).

## 5.1	What You Have

Delta is a complete AXI-Stream crossbar generator project with:

### 5.1.1	[PASS] Working Code Generator (697 lines)

- bin/delta_generator.py - Python RTL generator

- Produces parameterized SystemVerilog

- Flat crossbar topology (tree retired 2026-10-12)

- Optional TSTRB byte qualifiers + AMBA 5 TWAKEUP/TPARITY sidebands (--strb-width N, --amba5; default off = base 7-signal set, byte-identical to earlier output) — port-matches rtl/amba/axis4 + axis5; see rtl_test/delta_axis_flat_2x4_strb_amba5.sv

- Tested and working (example RTL generated + verilator-linted)

### 5.1.2	[PASS] Performance Modeling (487 lines)

- bin/delta_performance_model.py - Analytical + simulation models

- Latency/throughput analysis

- Resource estimation

- Flat vs tree comparison

### 5.1.3	[PASS] Complete Specifications

- PRD.md (525 lines) - Product requirements document

- README.md (502 lines) - User guide

- (NoC design draft — unimplemented, NOT this component: ../../noc-ip/delta_spec/, rendered PDF alongside)

### 5.1.4	[PASS] Generated RTL Example

- rtl/delta_axis_flat_4x16.sv - Working 4x16 crossbar

- Verilator lint clean

- Ready for synthesis

Total: ~2,826 lines of specifications, code, and documentation

## 5.2	How Delta Differs from Your APB Crossbar Generator

### 5.2.1	Key Insight: 95% Code Reuse, ~75 Minutes to Adapt

| Component | APB | Delta (AXIS) | Difference |
| --- | --- | --- | --- |
| Request generation | Address range decode | TDEST decode | SIMPLER! (no address map) |
| Arbitration | Round-robin | Round-robin + atomicity | +10 lines for TLAST handling |
| Data mux | Mux PRDATA | Mux TDATA+more signals | Same pattern, +3 signals |
| Backpressure | PREADY | TREADY | Just rename signals |

### 5.2.2	Detailed Comparison

The original docs/DELTA_VS_APB_GENERATOR.md migration guide (side-by-side code comparison, migration checklist, effort estimation) was removed after the migration was completed - bin/delta_generator.py is the adapted, framework-based generator. The comparison summary above captures the key differences.

### 5.2.3	Why AXIS is Actually Simpler Than APB

APB Address Decode (64 comparisons for 4x16):

if (paddr[0] >= 32'h10000000 && paddr[0] < 32'h10001000) request_matrix[0][0] = 1'b1;
if (paddr[0] >= 32'h10001000 && paddr[0] < 32'h10002000) request_matrix[1][0] = 1'b1;
// ... 62 more comparisons

AXIS TDEST Decode (4 decodes for 4x16):

if (s_axis_tvalid[m])
    request_matrix[s_axis_tdest[m]][m] = 1'b1;  // Done!

Result: AXIS is 7x simpler in request generation!

## 5.3	Quick Test Drive

### 5.3.1	1. Generate Your First Crossbar (30 seconds)

cd projects/components/fabric-gen-ip/delta

# Generate flat 4x16 for RISC cores + DSP arrays
python bin/delta_generator.py \
    --topology flat \
    --masters 4 \
    --slaves 16 \
    --data-width 64 \
    --output-dir rtl/

# OK Output: rtl/delta_axis_flat_4x16.sv

### 5.3.2	2. Run Performance Analysis

# Compare flat vs tree topology
python bin/delta_performance_model.py --topology compare

# Output:
# ================================================================================
# Flat:  2 cycles latency, 12 xfers/cyc (76.8 Gbps @ 100MHz)
# Tree:  6 cycles latency, 0.8 xfers/cyc (5.1 Gbps @ 100MHz)
# Recommendation: Flat for production, Tree for education
# ================================================================================

### 5.3.3	3. Inspect Generated RTL

# View module header
head -80 rtl/delta_axis_flat_4x16.sv

# Check for lint errors (should be clean)
verilator --lint-only rtl/delta_axis_flat_4x16.sv

### 5.3.4	4. Generate Test Configurations

# Generate additional flat crossbar sizes into rtl_test/
python bin/delta_generator.py --topology flat --masters 2 --slaves 4  --data-width 32 --output-dir rtl_test/
python bin/delta_generator.py --topology flat --masters 2 --slaves 16 --data-width 64 --output-dir rtl_test/
python bin/delta_generator.py --topology flat --masters 3 --slaves 8  --data-width 64 --output-dir rtl_test/

# Lint everything under rtl/ via the Makefile
cd rtl && make verilator

Note (2026-07-22): the tree-topology flow (1:2 splitters, 2:1 mergers, fan-out/fan-in trees via bin/complete_tree_generator.py) was retired; only flat crossbars are generated and kept in the tree now. See TREE_TOPOLOGY_TEST_RESULTS.md (historical) for what the retired flow produced.

## 5.4	Project Structure

projects/components/fabric-gen-ip/delta/
+-- bin/                              # Automation
|   +-- delta_generator.py            # RTL generator (framework version)
|   +-- delta_generator_v1_backup.py  # Previous generator (backup)
|   +-- complete_tree_generator.py    # Retired tree-topology generator (historical)
|   +-- delta_performance_model.py    # Performance models
|
+-- docs/                             # Documentation
|   +-- delta_spec/                   # Chaptered specification (delta_index.md)
|   +-- Delta_Specification_v1.0.pdf  # Rendered specification
|
+-- rtl/                              # Generated RTL
|   +-- delta_axis_flat_4x16.sv       # Production 4x16 crossbar
|   +-- Makefile                      # Lint/synthesis checks (make verilator)
|
+-- rtl_test/                         # Generated test configurations
|   +-- delta_axis_flat_{2x4,2x16,3x8,4x16}.sv
|
+-- PRD.md                            # Requirements
+-- README.md                         # User guide
+-- QUICK_START.md                    # This file

## 5.5	Next Steps

### 5.5.1	Option A: Use Provided Generator Immediately

# Generate production RTL
python bin/delta_generator.py --topology flat --masters 4 --slaves 16 --data-width 64 --output-dir rtl/

# Lint check
verilator --lint-only rtl/delta_axis_flat_4x16.sv

# Synthesize (Vivado/Yosys)
# ... synthesis script ...

# Create CocoTB testbench
# (following patterns in bin/TBClasses/)

Timeline: Immediate RTL, 1-2 days for verification

### 5.5.2	Option B: Adapt Your APB Generator

Done (historical): this adaptation was completed - bin/delta_generator.py is the framework-based generator that resulted from adapting the APB crossbar generator (signal renames, simplified TDEST decode, packet atomicity, TLAST/TID/TUSER support). The previous version is kept as bin/delta_generator_v1_backup.py.

### 5.5.3	Option C: Review Specs First (Your Preferred Approach)

# 1. Read complete specifications
cat PRD.md
cat README.md
cat ../../noc-ip/delta_spec/delta_index.md   # NoC draft (unimplemented, reference)

# 2. Review performance models
python bin/delta_performance_model.py --topology compare

# 3. Understand generated RTL
cat rtl/delta_axis_flat_4x16.sv

# 4. Plan integration with RISC cores + DSP arrays
# 5. Create verification plan
# 6. Generate production RTL

Timeline: 1 week review/planning, 1-2 weeks implementation/verification

## 5.6	Generator Command Reference

### 5.6.1	Basic Usage

python bin/delta_generator.py \
    --topology <flat|tree|both> \
    --masters <num> \
    --slaves <num> \
    --data-width <bits> \
    --output-dir <dir>

### 5.6.2	Examples

# Flat 4x16 crossbar @ 64-bit (production)
python bin/delta_generator.py --topology flat --masters 4 --slaves 16 --data-width 64 --output-dir rtl/

# Tree 4x16 crossbar @ 64-bit (educational)
python bin/delta_generator.py --topology tree --masters 4 --slaves 16 --data-width 64 --output-dir rtl/ --nodes

# Both topologies for comparison
python bin/delta_generator.py --topology both --masters 4 --slaves 16 --data-width 64 --output-dir rtl/ --nodes

# Small 2x4 test configuration
python bin/delta_generator.py --topology flat --masters 2 --slaves 4 --data-width 32 --output-dir test/

### 5.6.3	Options

| Option | Values | Default | Description |
| --- | --- | --- | --- |
| --topology | flat, tree, both | flat | Crossbar topology |
| --masters | 1-32 | required | Number of master interfaces |
| --slaves | 1-256 | required | Number of slave interfaces |
| --data-width | 8-1024 | 64 | TDATA width in bits |
| --user-width | 1-32 | 1 | TUSER width in bits |
| --output-dir | path | ../rtl | Output directory |
| --nodes | flag | false | Generate node primitives (1:2, 2:1) |
| --no-counters | flag | false | Disable performance counters |

## 5.7	Performance Model Command Reference

### 5.7.1	Basic Usage

python bin/delta_performance_model.py --topology <flat|tree|compare>

### 5.7.2	Examples

# Compare flat vs tree (recommended first step)
python bin/delta_performance_model.py --topology compare

# Analyze flat topology only
python bin/delta_performance_model.py --topology flat --masters 4 --slaves 16 --data-width 64

# Run discrete event simulation (requires simpy)
python bin/delta_performance_model.py --topology flat --simulate

# Custom configuration
python bin/delta_performance_model.py --topology flat --masters 8 --slaves 32 --data-width 128

## 5.8	Key Features

### 5.8.1	1. Dual Topology Support

Flat Crossbar: - Latency: 2 cycles - Throughput: 12 transfers/cycle (4x16) - Resources: ~1,536 LUTs - Use case: Production (RISC + DSP)

Tree Topology: - Latency: 6 cycles - Throughput: 0.8 transfers/cycle (4x16) - Resources: ~921 LUTs - Use case: Education (modularity demo)

### 5.8.2	2. Performance Modeling

Analytical Model: - Closed-form latency calculation - Throughput estimation (queuing theory) - Resource estimation (empirical) - No simulation needed - instant results

Simulation Model (SimPy): - Cycle-accurate discrete event simulation - Statistical analysis (mean, p50, p99) - Traffic patterns (uniform, hotspot, localized) - Validation against RTL

### 5.8.3	3. Complete Specifications

PRD (525 lines): - Functional requirements - Performance targets - Interface specifications - Use cases and success criteria

Technical Docs: - Generator architecture - APB migration guide - Integration examples - Verification strategy

## 5.9	Demonstrations of Rigor

As requested, Delta demonstrates rigor through:

### 5.9.1	1. Complete Specifications (Before Code)

- PRD written first (requirements -> architecture -> design)

- All interfaces documented

- Success criteria defined

- Trade-offs analyzed

### 5.9.2	2. Performance Modeling (Before Implementation)

- Analytical model (closed-form math)

- Simulation model (discrete events)

- Resource estimation

- Validation methodology

### 5.9.3	3. Architecture Comparison

- Flat vs tree topology analysis

- Latency/throughput/resources trade-offs

- Use case matching

- Recommendation with rationale

### 5.9.4	4. Integration with Existing Automation

- Reuses APB crossbar patterns (95%)

- Migration guide with effort estimates

- Side-by-side code comparison

- Detailed diff showing changes

## 5.10	Educational Value

Delta teaches:

- Specification-Driven Design - Complete specs before coding

- Code Generation - Python automation for parameterized RTL

- Performance Modeling - Analytical + simulation validation

- Architecture Trade-offs - Flat vs tree comparison

- Verification - CocoTB framework integration (TODO)

Perfect for GitHub instruction repository!

## 5.11	Status Summary

### 5.11.1	[PASS] Complete

- RTL generator (697 lines Python)

- Performance models (487 lines Python)

- Specifications (PRD, README, migration guide)

- Example generated RTL (4x16 crossbar)

- Command-line interface with full options

- Tree-topology generation (splitters/mergers/fan-out/fan-in) - since retired in favor of flat crossbars (see status note at top)

### 5.11.2	[ ] TODO (Future Work)

- CocoTB testbench framework

- RISC + DSP integration example

- Weighted round-robin arbiter variant

## 5.12	Quick Reference

Generate RTL:

python bin/delta_generator.py --topology flat --masters 4 --slaves 16 --output-dir rtl/

Run Performance Analysis:

python bin/delta_performance_model.py --topology compare

Read Specs:

cat PRD.md                            # Requirements
cat README.md                         # User guide
cat ../../noc-ip/delta_spec/delta_index.md   # NoC draft (unimplemented, reference)

View Generated RTL:

cat rtl/delta_axis_flat_4x16.sv

Lint Check:

verilator --lint-only rtl/delta_axis_flat_4x16.sv

## 5.13	Summary

Delta provides everything you need:

- [PASS] Working generator (tested, produces lint-clean RTL)

- [PASS] Performance models (analytical + simulation)

- [PASS] Complete specs (PRD + docs demonstrate rigor)

- [PASS] APB migration guide (~95% reuse, ~75 min effort)

- [PASS] Example RTL (4x16 crossbar generated and verified)

Ready for your 4 RISC cores + 16 DSP arrays project!

Project Delta - Where data flows branch like river deltas

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Document Information

## 6.1	Delta Hardware Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Delta Hardware Architecture Specification |
| Version | 1.0 |
| Date | 2026-10-12 |
| Status | Released |
| Classification | Open Source - MIT License |

## 6.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-10-12 | RTL Design Sherpa | Initial HAS/MAS split; documents the 2026-10-12 richness options (TSTRB and AMBA5 TWAKEUP/TPARITY) and flat-only topology. |

## 6.3	Document Purpose

This Hardware Architecture Specification (HAS) is the high-level view of the Delta AXI4-Stream crossbar generator — the map you read before you integrate, not the schematic. It covers:

- System-level architecture and data flow

- AXI4-Stream protocol support (base and rich flavors)

- Interface definitions and requirements

- Performance characteristics

- Integration guidelines

For micro-architecture, FSM designs, and signal-level detail, see the companion Delta Micro-Architecture Specification (MAS).

## 6.4	Intended Audience

- System architects defining streaming interconnect topology

- Hardware engineers integrating Delta into SoC designs

- Verification engineers planning test strategies

- Technical managers evaluating Delta capabilities

## 6.5	Related Documents

- Delta MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI assistant integration guide

- QUICK_START.md - Hands-on generator guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Purpose and Scope

## 7.1	What is Delta?

Delta is a Python-based AXI4-Stream crossbar generator. You provide the number of masters, the number of slaves, the data width, and a few optional sideband knobs; it hands you back parameterized, synthesizable SystemVerilog. The name follows the water theme used elsewhere in RTL Design Sherpa (like RAPIDS): a river delta is where one channel splits into many branches, analogous to crossbar routing.

## 7.2	The Problem Delta Solves

A hand-written AXI4-Stream crossbar is error-prone. Building one means writing per-slave arbitration, packet-atomic grant logic, data multiplexing for TDATA/TVALID/TLAST/TDEST/TID/TUSER, and TREADY backpressure steering — then doing it again every time the master or slave count changes.

Delta automates all of it:

- Define the crossbar in a single command line

- Run bin/delta_generator.py

- Get production-ready SystemVerilog RTL

## 7.3	Scope

This specification covers:

- High-level architecture and block organization

- AXI4-Stream protocol support, including the optional base + rich signal sets

- Interface requirements and signal conventions

- Performance characteristics and resource estimates

- Integration guidelines and verification status

## 7.4	Out of Scope

These live in the companion MAS or are deliberate non-goals:

- Detailed FSM state diagrams and transitions (MAS)

- Signal-level timing diagrams (MAS)

- Internal pipeline structures (MAS)

- AXI4/AXI4-Lite/APB protocol conversion (deliberate non-goal; see Bridge or APB crossbar siblings)

- Dynamic reconfiguration or advanced routing algorithms

## 7.5	Non-Goal: Memory-Mapped Protocol Conversion

Delta is AXI4-Stream only. It does not convert to or from AXI4, AXI4-Lite, or APB. If you need address-mapped fabrics, use the sibling bridge or apbx-xbar components.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Document Conventions

## 8.1	Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| s_axis_* | Signals on the master side of the crossbar (inputs) | s_axis_tdata[m] |
| m_axis_* | Signals on the slave side of the crossbar (outputs) | m_axis_tready[s] |
| [NUM_MASTERS] | Unpacked array dimension, one entry per master | s_axis_tvalid [NUM_MASTERS] |
| [NUM_SLAVES] | Unpacked array dimension, one entry per slave | m_axis_tvalid [NUM_SLAVES] |
| m | Master index | for (int m = 0; m < NUM_MASTERS; m++) |
| s | Slave index | for (int s = 0; s < NUM_SLAVES; s++) |

## 8.2	Signal Direction Conventions

All directions are stated from the crossbar’s point of view:

- s_axis_* signals are driven by upstream masters and received by Delta.

- m_axis_* signals are driven by Delta and received by downstream slaves.

- aclk and aresetn are driven by the system.

## 8.3	Numbering Conventions

- Masters are numbered 0 to NUM_MASTERS-1.

- Slaves are numbered 0 to NUM_SLAVES-1.

- TDEST is interpreted as a slave index; legal values are 0 to NUM_SLAVES-1.

## 8.4	Typographic Conventions

- code font is used for signal names, parameters, and file paths.

- Bold is used for emphasis and defined terms.

- Tables use the RTL Design Sherpa corporate document style.

## 8.5	Versioning Conventions

This document uses semantic-style major.minor revision numbers. The initial release is 1.0.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Definitions and Acronyms

## 9.1	Acronyms

| Acronym | Definition |
| --- | --- |
| AXIS | AXI4-Stream (ARM AMBA streaming protocol) |
| AMBA5 | ARM AMBA 5 protocol extensions, including TWAKEUP and TPARITY |
| TDEST | Transaction destination (slave ID in AXI4-Stream) |
| TID | Transaction ID (master identifier in AXI4-Stream) |
| TUSER | User-defined sideband in AXI4-Stream |
| TSTRB | Byte strobe / byte qualifier in AXI4-Stream |
| TWAKEUP | AMBA 5 wakeup hint signal |
| TPARITY | AMBA 5 parity signal |
| RR | Round-robin arbitration |
| LUT | Look-up table (FPGA logic resource) |
| FF | Flip-flop (FPGA register resource) |

## 9.2	Key Terms

Base flavor

The default generated crossbar with the seven-signal set: tdata, tvalid, tready, tlast, tdest, tid, tuser.

Rich flavor

A generated crossbar with optional additional signals enabled: tstrb and/or AMBA 5 twakeup/tparity.

Packet atomicity

The property that once a master is granted access to a slave, it retains that grant until the current packet’s TLAST beat has been transferred. No other master may interleave beats on that slave during the packet.

Request matrix

A per-slave collection of master request bits. request_matrix[s][m] is true when master m has a valid transaction targeting slave s.

Grant matrix

A per-slave one-hot (or zero-hot) collection of master grant bits. grant_matrix[s][m] is true when master m currently owns slave s.

TDEST decode

The combinational translation of s_axis_tdest[m] into a slave index in the request matrix. Delta does not use an address map.

TWAKEUP steering

The AMBA 5 sideband path that forwards a master’s s_axis_twakeup hint to the slave selected by s_axis_tdest. It is an OR-per-slave and may assert before TVALID.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Use Cases

## 10.1	Primary: RISC Cores + DSP Array

Scenario: Four small RISC-V cores route compute tasks to sixteen DSP accelerators.

Configuration:

- 4 masters (RISC cores)

- 16 slaves (DSP arrays)

- 64-bit data width

- Flat topology

Benefits:

- Each RISC core can target any DSP (full flexibility).

- Per-slave round-robin arbitration prevents starvation.

- Two-cycle latency keeps task dispatch responsive.

- All sixteen DSPs can operate concurrently when traffic targets different slaves.

## 10.2	Secondary: Sensor/Peripheral Fabric

Scenario: Multiple sensor streams feed a smaller set of processing blocks.

Configuration:

- 2-8 masters (sensor front-ends)

- 2-4 slaves (aggregation or filter blocks)

- 32-bit data width with optional TSTRB byte qualifiers

Benefits:

- Direct TDEST routing removes the need for an address decoder.

- Packet atomicity preserves frame boundaries from each sensor.

- Optional TSTRB supports sparse byte-valid data from byte-packed sensors.

## 10.3	Tertiary: AMBA 5-Aware Streaming Interconnect

Scenario: A low-power subsystem needs wakeup hints and parity sidebands.

Configuration:

- Masters and slaves that implement AMBA 5 AXI-Stream

- --amba5 enabled, optionally with --parity-width

Benefits:

- TWAKEUP lets a master wake a target slave before presenting data.

- TPARITY carries a user-defined parity sideband across the fabric.

- Generated ports port-match the rtl/amba/axis5/* interface conventions.

## 10.4	Educational Demonstration

Scenario: Teach interconnect design trade-offs and code generation.

Configuration:

- Generate flat crossbars of varying sizes into rtl_test/.

- Compare latency, throughput, and resource estimates from delta_performance_model.py.

Benefits:

- Hands-on learning with working RTL.

- Clear performance comparisons.

- Demonstrates specification-driven design and Python-based RTL generation.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Key Features

## 11.1	Flat M×N Crossbar

Delta generates a single self-contained SystemVerilog module implementing a full M×N crossbar. Every master can reach every slave; each slave has its own round-robin arbiter.

## 11.2	TDEST-Based Routing

There is no address map. The target slave is taken directly from s_axis_tdest. This makes request decode simpler than an address-based crossbar and removes configuration risk.

## 11.3	Packet-Atomic Round-Robin Arbitration

Each slave arbiter grants one master at a time and holds that grant until the current packet’s TLAST beat is accepted. This guarantees:

- No packet interleaving on a slave.

- Fair bandwidth allocation among contending masters.

- Streaming packet boundaries are preserved end-to-end.

## 11.4	Combinational Data Mux with Registered Grant

The data path from the granted master to the selected slave is combinational. The grant decision is registered on aclk, giving a predictable two-cycle latency while keeping the critical path short.

## 11.5	Optional Richness (Default Off)

Two command-line options add sidebands without changing the default output:

- --strb-width N adds TSTRB byte qualifiers (port-matches rtl/amba/axis4).

- --amba5 [--parity-width M] adds TWAKEUP and TPARITY (port-matches rtl/amba/axis5).

When neither option is used, the generated module is byte-identical to pre-2026-10-12 output.

## 11.6	Optional Packet Counters

Per-master and per-slave 32-bit packet counters count completed packets (TVALID && TREADY && TLAST). Disable them with --no-counters to save area.

## 11.7	Formal Assertions

When FORMAL is defined, the generated RTL includes assertions that check:

- One-hot (or zero-hot) grants per slave.

- TDEST is within range when s_axis_tvalid is asserted.

## 11.8	Code Generation

The generator is a Python script (bin/delta_generator.py) that reuses the unified Module framework. It produces lint-clean SystemVerilog and is the single source of truth for all checked-in RTL variants.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	System Context

## 12.1	Where Delta Fits

Delta sits between upstream AXI4-Stream masters and downstream AXI4-Stream slaves. It does not interpret addresses, bursts, or memory attributes; it only routes streams based on TDEST.

A typical Delta deployment looks like:

┌─────────────┐     ┌───────┐     ┌─────────────┐
│   Master    │────▶│       │────▶│   Slave     │
│   (source)  │     │ Delta │     │  (sink)     │
│             │◀────│       │◀────│             │
└─────────────┘     └───────┘     └─────────────┘
     s_axis              ▲              m_axis
                     aclk / aresetn

Multiple masters and multiple slaves attach in parallel. Each master drives one index of the s_axis_* arrays; each slave receives one index of the m_axis_* arrays.

## 12.2	Upstream Requirements

- Masters must drive valid AXI4-Stream signals.

- s_axis_tdest[m] must be a valid slave index (0 to NUM_SLAVES-1) when s_axis_tvalid[m] is asserted.

- Masters must respect s_axis_tready[m] backpressure.

## 12.3	Downstream Requirements

- Slaves must accept valid AXI4-Stream signals.

- Slaves drive m_axis_tready[s] to throttle traffic.

## 12.4	Clocking and Reset

Delta uses a single clock (aclk) and an active-low asynchronous reset (aresetn). All registered state is reset; combinational decode and mux logic is not registered.

## 12.5	Relationship to Other Components

| Component | Role | When to Use |
| --- | --- | --- |
| Delta | AXI4-Stream crossbar generator | Streaming fabrics, DSP arrays, sensors |
| Bridge | Multi-protocol crossbar generator | Memory-mapped AXI4/AXI4-Lite/APB systems |
| APB Crossbar | APB-only crossbar | Low-bandwidth register-mapped peripherals |
| RAPIDS | DMA subsystem | Descriptor-based memory transfers |

Delta is intentionally narrow in scope: it does one thing (streaming crossbars) and does it without protocol conversion.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Block Diagram

## 13.1	Delta Architecture Overview

### 13.1.1	Figure 3.1: Delta High-Level Block Diagram

Delta HAS Block Diagram

The diagram shows the three traceable flows of the Delta crossbar: the request/control flow down the center, the payload flow on the left, and the backpressure flow returning up the right/left side as a dashed sideband. The scheduler is the wide-edge-up trapezium (per-slave round-robin arbiter). The gray bar is the slave / m_axis boundary.

## 13.2	Functional Blocks

### 13.2.1	TDEST Decode

Receives s_axis_tvalid and s_axis_tdest from every master and produces the per-slave request_matrix. A request bit is set when a master has a valid transaction whose TDEST selects that slave.

### 13.2.2	Per-Slave Round-Robin Arbiter

One arbiter per slave. When the slave is idle, it picks the next requesting master in round-robin order. Once a master is granted, the grant is held until TLAST is accepted. This block is the scheduler analog in the house style and is drawn as an inverted trapezium.

### 13.2.3	Crossbar Mux

A bank of per-slave multiplexers. Each slave mux selects the payload and control signals from the master that currently holds the grant for that slave.

### 13.2.4	TREADY Steering

Steers each slave’s m_axis_tready back to the single master that currently owns the grant for that slave. Ungranted masters see s_axis_tready == 0.

### 13.2.5	TWAKEUP Steering (AMBA5, Optional)

When --amba5 is enabled, this block ORs each master’s s_axis_twakeup to the slave indicated by s_axis_tdest. Because TWAKEUP is a hint, it does not depend on the grant and may assert before TVALID.

### 13.2.6	Packet Counters (Optional)

32-bit counters per master and per slave that increment on every completed packet (TVALID && TREADY && TLAST). Removed from the build with --no-counters.

## 13.3	Scalability

Table 3.1: Topology Parameter Scaling

| Parameter | Typical Range | Impact |
| --- | --- | --- |
| NUM_MASTERS | 1-32 | Linear arbitration and mux growth |
| NUM_SLAVES | 1-256 | Linear decode and grant-matrix growth |
| DATA_WIDTH | 8-1024 | Linear data-path width |
| DEST_WIDTH | auto (clog2(NUM_SLAVES)) | Sets TDEST width |
| ID_WIDTH | auto (clog2(NUM_MASTERS)) | Sets TID width |

### 13.3.1	Resource Scaling

- Logic: O(NUM_MASTERS × NUM_SLAVES) for the grant and request matrices.

- Data path: O(NUM_SLAVES × DATA_WIDTH) for the output muxes.

- Memory: No BRAM; all state is flip-flops.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Data Flow

## 14.1	Request Flow

- A master asserts s_axis_tvalid[m] and places the target slave index on s_axis_tdest[m].

- The TDEST decode block sets request_matrix[s][m] for the selected slave.

- If the slave is not already serving another packet, the per-slave round-robin arbiter grants one requesting master.

- The grant is registered and held until the packet ends on TLAST.

## 14.2	Payload Flow

- While a grant is active, the crossbar mux selects the granted master’s payload.

- m_axis_tdata[s], m_axis_tlast[s], m_axis_tdest[s], m_axis_tid[s], and m_axis_tuser[s] are driven from the granted master.

- If TSTRB is enabled, m_axis_tstrb[s] is also forwarded.

- If TPARITY is enabled, m_axis_tparity[s] is forwarded.

## 14.3	Backpressure Flow

- Each downstream slave drives m_axis_tready[s].

- The TREADY steering block routes the granted slave’s ready back to the owning master.

- Only the granted master sees s_axis_tready[m] == 1; all other masters are stalled.

## 14.4	Wakeup Flow (AMBA5)

- A master may assert s_axis_twakeup[m] before asserting s_axis_tvalid[m].

- The TWAKEUP steering block decodes s_axis_tdest[m] and ORs the hint to the matching slave.

- m_axis_twakeup[s] may assert with no active grant.

## 14.5	Packet Boundaries

TLAST marks the final beat of a packet. The arbiter releases the grant on the cycle in which m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s] is true. The next arbitration decision can occur on the following cycle.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Protocol Support

## 15.1	AXI4-Stream Base Flavor

The default generated crossbar implements the standard AXI4-Stream signal set:

- TDATA — data payload

- TVALID / TREADY — handshake

- TLAST — packet boundary

- TDEST — destination slave index

- TID — transaction/master ID (pass-through)

- TUSER — user sideband (pass-through)

This is the byte-identical default; no sideband options are enabled unless requested.

## 15.2	Optional TSTRB Byte Qualifiers

The --strb-width N option adds TSTRB to every master and slave interface. TSTRB is treated as a pass-through sideband: the mux forwards the granted master’s TSTRB to the selected slave. Delta does not interpret or modify strobe bits.

## 15.3	Optional AMBA 5 Sidebands

The --amba5 option adds two AMBA 5 signals:

- TWAKEUP — wakeup hint, decoded from TDEST and ORed per slave.

- TPARITY — parity sideband, treated as pass-through like TUSER.

The parity width defaults to DATA_WIDTH / 8 and can be overridden with --parity-width M. The generated ports port-match the rtl/amba/axis5/* conventions.

## 15.4	What Delta Does Not Support

Delta is intentionally AXI4-Stream only. It does not support:

- AXI4 memory-mapped transactions (AW/AR/W/B/R channels).

- AXI4-Lite register-mapped access.

- APB transfers.

- Burst lengths, address phases, or memory attributes.

- Dynamic reconfiguration of the crossbar dimensions.

For memory-mapped fabrics, use the sibling bridge or apbx-xbar components.

## 15.5	Protocol Compliance Summary

Table 3.2: Protocol Support Matrix

| Feature | Base Flavor | Rich Flavor | Notes |
| --- | --- | --- | --- |
| TVALID/TREADY handshake | Yes | Yes | Standard AXI4-Stream |
| TLAST packet boundary | Yes | Yes | Grant release on TLAST |
| TDEST routing | Yes | Yes | Direct slave index |
| TID/TUSER pass-through | Yes | Yes | Unmodified |
| TSTRB | No | Yes | --strb-width N |
| TWAKEUP | No | Yes | --amba5 |
| TPARITY | No | Yes | --amba5 [--parity-width M] |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	AXI-Stream Base Interface

## 16.1	Master Interface (S_AXIS)

Each master port is indexed by m in the range 0 to NUM_MASTERS-1.

Table 4.1: Base Master Interface Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_tdata[m] | DATA_WIDTH | Input | Data payload. |
| s_axis_tvalid[m] | 1 | Input | Valid indicator. When high, s_axis_tdest[m] selects the target slave. |
| s_axis_tready[m] | 1 | Output | Ready / backpressure. Driven from the granted slave’s m_axis_tready. |
| s_axis_tlast[m] | 1 | Input | Packet boundary. The final beat of a packet. |
| s_axis_tdest[m] | DEST_WIDTH | Input | Target slave index. Must be < NUM_SLAVES when tvalid is asserted. |
| s_axis_tid[m] | ID_WIDTH | Input | Transaction ID. Pass-through to m_axis_tid. |
| s_axis_tuser[m] | USER_WIDTH | Input | User sideband. Pass-through to m_axis_tuser. |

## 16.2	Slave Interface (M_AXIS)

Each slave port is indexed by s in the range 0 to NUM_SLAVES-1.

Table 4.2: Base Slave Interface Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_tdata[s] | DATA_WIDTH | Output | Data payload from the granted master. |
| m_axis_tvalid[s] | 1 | Output | Valid indicator. Asserted when a grant is active and the selected master is valid. |
| m_axis_tready[s] | 1 | Input | Ready / backpressure from the downstream slave. |
| m_axis_tlast[s] | 1 | Output | Packet boundary. Forwarded from the granted master. |
| m_axis_tdest[s] | DEST_WIDTH | Output | Target slave index. Pass-through from the granted master. |
| m_axis_tid[s] | ID_WIDTH | Output | Transaction ID. Pass-through from the granted master. |
| m_axis_tuser[s] | USER_WIDTH | Output | User sideband. Pass-through from the granted master. |

## 16.3	Signal Semantics

- TVALID may be asserted without TREADY; the transfer completes only when both are high.

- TREADY for an ungranted master is always low.

- TDEST must be stable while TVALID is asserted and the master is waiting for grant or transfer.

- TID and TUSER are pass-through and must be stable on every valid beat.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	AXI-Stream Rich Interface (TSTRB + AMBA5)

## 17.1	Rich Master Interface (S_AXIS)

When --strb-width N and/or --amba5 are used, the master interface adds the following signals in addition to the base set.

Table 4.3: Rich Master Interface Additional Signals

| Signal | Width | Direction | Presence | Description |
| --- | --- | --- | --- | --- |
| s_axis_tstrb[m] | STRB_WIDTH | Input | --strb-width N | Byte qualifier / strobe. Pass-through to m_axis_tstrb. |
| s_axis_twakeup[m] | 1 | Input | --amba5 | Wakeup hint. May assert before TVALID. |
| s_axis_tparity[m] | PW_WIDTH | Input | --amba5 | Parity sideband. Pass-through to m_axis_tparity. |

## 17.2	Rich Slave Interface (M_AXIS)

Table 4.4: Rich Slave Interface Additional Signals

| Signal | Width | Direction | Presence | Description |
| --- | --- | --- | --- | --- |
| m_axis_tstrb[s] | STRB_WIDTH | Output | --strb-width N | Byte qualifier from the granted master. |
| m_axis_twakeup[s] | 1 | Output | --amba5 | Wakeup hint. OR of all masters whose TDEST selects slave s. |
| m_axis_tparity[s] | PW_WIDTH | Output | --amba5 | Parity sideband from the granted master. |

## 17.3	TSTRB Semantics

- TSTRB is treated as an opaque byte-qualifier sideband.

- The crossbar forwards the granted master’s TSTRB to the selected slave.

- Delta does not gate TVALID based on TSTRB or modify strobe bits.

## 17.4	TWAKEUP Semantics

TWAKEUP is an AMBA 5 hint, not a handshake signal:

- A master may assert s_axis_twakeup[m] before s_axis_tvalid[m] to wake the target slave.

- The crossbar decodes s_axis_tdest[m] and ORs the hint to the indicated slave.

- m_axis_twakeup[s] may be high while m_axis_tvalid[s] is low.

- No grant is required for TWAKEUP steering.

## 17.5	TPARITY Semantics

- TPARITY is treated as a pass-through sideband like TUSER.

- The width defaults to DATA_WIDTH / 8 and can be overridden with --parity-width M.

- The crossbar forwards the granted master’s TPARITY to the selected slave.

## 17.6	Parameter Implications

Table 4.5: Rich-Flavor Parameter-to-Port Mapping

| Option | Generated Parameter | Port Effect |
| --- | --- | --- |
| --strb-width N | STRB_WIDTH = N | Adds s_axis_tstrb and m_axis_tstrb arrays. |
| --amba5 | PW_WIDTH = DATA_WIDTH/8 (default) | Adds s_axis_twakeup/tparity and m_axis_twakeup/tparity arrays. |
| --amba5 --parity-width M | PW_WIDTH = M | Overrides the default parity width. |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Clock and Reset

## 18.1	Clock

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| aclk | 1 | Input | Positive-edge clock for all registered state. |

All registered logic (grants, last_grant, packet-active flags, counters) is clocked on the rising edge of aclk. Combinational decode and mux logic is not registered.

## 18.2	Reset

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| aresetn | 1 | Input | Active-low asynchronous reset. |

aresetn resets all registered state:

- grant_matrix to zero.

- last_grant to zero.

- packet_active to zero.

- Packet counters to zero.

The reset assertion is asynchronous; de-assertion is synchronous to aclk in the checked-in RTL through the ALWAYS_FF_RST macro.

## 18.3	Clocking Rules

- All masters and slaves share the same aclk.

- Delta does not contain clock-domain crossing logic.

- Inputs are sampled on the rising edge of aclk; outputs are driven combinationally or from registers.

## 18.4	Reset Recommendations

- Assert aresetn for at least several clock cycles after power and clock stability.

- Do not rely on Delta’s outputs during reset; combinational mux outputs may be zero or undefined during reset.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Throughput Characteristics

## 19.1	Aggregate Throughput

Because each slave has its own arbiter and mux, aggregate throughput scales with the number of slaves. In the best case — every master targets a different slave and every slave is ready — the crossbar can accept one beat per master per cycle.

## 19.2	Per-Slave Throughput

When multiple masters contend for the same slave, the round-robin arbiter gives each contending master an equal share of the slave’s bandwidth. With K contending masters, each sees approximately 1/K of the slave’s available bandwidth.

## 19.3	Head-of-Line Blocking

A master blocked on one slave does not block traffic to other slaves. Each slave arbiter is independent. However, a master that targets a heavily contended slave will wait until its round-robin turn and until the current packet completes.

## 19.4	Throughput Targets

Table 5.1: Throughput Targets

| Metric | Target | Notes |
| --- | --- | --- |
| Per-slave throughput | Up to 1 transfer/cycle | When one master continuously feeds one ready slave. |
| Aggregate throughput | Up to NUM_MASTERS transfers/cycle | When all masters target different ready slaves. |
| Mixed-traffic throughput | ≥ 0.7 transfers/cycle per active master | Under moderate contention in the performance model. |

## 19.5	Factors That Reduce Throughput

- Contention for the same slave.

- Downstream backpressure (m_axis_tready[s] == 0).

- Multi-beat packets that lock the slave until TLAST.

- Arbitration overhead of one cycle when switching masters after TLAST.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 20	Latency Analysis

## 20.1	Cycle Breakdown

The generated flat crossbar has a fixed two-cycle latency from master input to slave output:

Table 5.2: Latency Cycle Breakdown

| Cycle | Action |
| --- | --- |
| 0 | Master asserts s_axis_tvalid and s_axis_tdest. |
| 1 | TDEST decode produces request_matrix; arbiter registers the grant. |
| 2 | Crossbar mux drives m_axis_tdata, m_axis_tvalid, etc. to the slave. |

## 20.2	Arbitration Delay

If the target slave is idle when the request arrives, the grant is registered on the next rising edge of aclk. If the slave is busy serving another packet, the new request waits until the current packet’s TLAST is accepted.

## 20.3	Backpressure Latency

m_axis_tready[s] propagates combinationally through the TREADY steering logic to s_axis_tready[m]. There is no additional registered delay on the backpressure path.

## 20.4	Latency Targets

Table 5.3: Latency Targets

| Configuration | Latency | Notes |
| --- | --- | --- |
| Flat crossbar, idle slave | 2 cycles | Arbitration + registered grant. |
| Flat crossbar, contended slave | 2 + wait cycles | Wait depends on current packet length. |

## 20.5	Frequency Target

The flat crossbar is intended to run at ≥ 300 MHz on Xilinx UltraScale+ devices for typical configurations (e.g., 4×16 @ 64-bit). Actual Fmax depends on device, tool settings, and floorplanning.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 21	Resources

## 21.1	Estimates from the Performance Model

The bin/delta_performance_model.py script provides empirical resource estimates based on the flat topology. Values below are planning estimates, not synthesis results.

Table 5.4: Resource Estimates

| Configuration | LUTs | FFs | Notes |
| --- | --- | --- | --- |
| 4×16 @ 64-bit | ~1,536 | ~1,536 | Base flavor, with counters. |
| 2×4 @ 32-bit | ~200 | ~150 | Small test configuration. |
| 4×16 @ 64-bit, no counters | ~1,400 | ~1,200 | --no-counters saves FF area. |

## 21.2	Scaling Laws

- LUTs: Grow roughly as NUM_MASTERS × NUM_SLAVES for arbitration plus NUM_SLAVES × DATA_WIDTH for the data mux.

- FFs: Grow roughly as NUM_MASTERS × NUM_SLAVES for the grant matrix and last_grant storage, plus NUM_MASTERS + NUM_SLAVES for packet counters when enabled.

- BRAM: None. Delta is a pure-logic implementation.

## 21.3	Area Optimization Knobs

Table 5.5: Area Optimization Options

| Knob | Effect |
| --- | --- |
| --no-counters | Removes pkt_count_master and pkt_count_slave arrays. |
| Smaller NUM_MASTERS/NUM_SLAVES | Linear reduction in control logic. |
| Smaller DATA_WIDTH | Linear reduction in data mux area. |
| Disable richness options | Removes TSTRB/TPARITY muxing and TWAKEUP decode logic. |

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 22	System Requirements

## 22.1	Toolchain Requirements

- Synthesis: Any SystemVerilog-2017-capable synthesis tool (Vivado, Yosys, Genus, etc.).

- Simulation: Any SystemVerilog simulator; cocotb is the planned verification framework.

- Lint: Verilator (--lint-only) is used for quick sanity checks.

## 22.2	Runtime Requirements

- Single clock domain: all s_axis_* and m_axis_* interfaces use aclk.

- Active-low asynchronous reset: aresetn must be asserted at power-up.

- All upstream masters and downstream slaves must implement the AXI4-Stream handshake.

## 22.3	Interface Assumptions

- Masters drive valid TDEST values (0 to NUM_SLAVES-1).

- Slaves drive m_axis_tready independently.

- No master changes TDEST while waiting for a grant.

- TID and TUSER widths match the generated parameters.

## 22.4	Integration Checklist

- ☐ Confirm DATA_WIDTH, NUM_MASTERS, and NUM_SLAVES match the system.

- ☐ Confirm DEST_WIDTH and ID_WIDTH are wide enough for the generated module.

- ☐ Decide whether TSTRB or AMBA 5 sidebands are needed.

- ☐ Decide whether packet counters are needed (default on; use --no-counters to remove).

- ☐ Connect aclk and aresetn from the system.

- ☐ Verify downstream slaves can accept backpressure.

- ☐ Run verilator --lint-only on the generated RTL.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 23	Parameter Configuration

## 23.1	Generated Module Parameters

Table 6.1: Generated Module Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| DATA_WIDTH | CLI --data-width | Width of TDATA in bits. |
| DEST_WIDTH | clog2(NUM_SLAVES) | Width of TDEST. |
| ID_WIDTH | clog2(NUM_MASTERS) | Width of TID. |
| USER_WIDTH | CLI --user-width | Width of TUSER. |
| NUM_MASTERS | CLI --masters | Number of master interfaces. |
| NUM_SLAVES | CLI --slaves | Number of slave interfaces. |
| STRB_WIDTH | CLI --strb-width | Width of TSTRB (only when --strb-width is used). |
| PW_WIDTH | DATA_WIDTH/8 (or --parity-width) | Width of TPARITY (only when --amba5 is used). |

## 23.2	Generator CLI Options

Table 6.2: Generator Command-Line Options

| Option | Values | Default | Description |
| --- | --- | --- | --- |
| --topology | flat only | flat | Crossbar topology (tree/both retired 2026-10-12). |
| --masters | 1-32 | required | Number of master interfaces. |
| --slaves | 1-256 | required | Number of slave interfaces. |
| --data-width | 8-1024 | 64 | TDATA width in bits. |
| --user-width | 1-32 | 1 | TUSER width in bits. |
| --strb-width | ≥ 0 | 0 | TSTRB width; 0 omits TSTRB. |
| --amba5 | flag | off | Emit AMBA 5 TWAKEUP and TPARITY. |
| --parity-width | ≥ 0 | 0 | Override TPARITY width (requires --amba5). |
| --no-counters | flag | off | Disable packet counters. |
| --output-dir | path | ../rtl | Output directory for generated SystemVerilog. |

## 23.3	Example Configurations

### 23.3.1	Base flavor 4×16 @ 64-bit

python3 bin/delta_generator.py \
    --topology flat --masters 4 --slaves 16 --data-width 64 \
    --output-dir rtl/

### 23.3.2	Rich flavor 2×4 @ 32-bit with TSTRB and AMBA5

python3 bin/delta_generator.py \
    --topology flat --masters 2 --slaves 4 --data-width 32 \
    --strb-width 4 --amba5 --output-dir rtl_test/

### 23.3.3	Base flavor without counters

python3 bin/delta_generator.py \
    --topology flat --masters 4 --slaves 16 --data-width 64 \
    --no-counters --output-dir rtl/

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 24	Verification Strategy

## 24.1	Current Status

As of 2026-10-12, there is no dv/ test suite for Delta. The generator output is verified by linting (Verilator) and by visual/code review. A cocotb-based testbench is the known TODO.

## 24.2	Generator Verification

- Run bin/delta_generator.py for several configurations.

- Compare output against checked-in reference RTL (rtl/delta_axis_flat_4x16.sv, rtl_test/delta_axis_flat_*.sv).

- Verify that default output is byte-identical to pre-2026-10-12 generations.

## 24.3	Lint Verification

verilator --lint-only rtl/delta_axis_flat_4x16.sv

Or use the provided rtl/Makefile:

cd rtl && make verilator

## 24.4	Planned RTL Verification

When a cocotb testbench is added, the recommended coverage includes:

- All master-to-slave routing combinations.

- Concurrent traffic to the same slave (round-robin fairness).

- Concurrent traffic to different slaves (no crosstalk).

- Backpressure stress tests.

- Packet atomicity verification (no interleaving before TLAST).

- Rich-flavor tests (TSTRB, TWAKEUP, TPARITY).

- Reset behavior.

## 24.5	Formal Properties

When FORMAL is defined, the generated RTL includes:

- One-hot grant assertion per slave.

- TDEST range assertion per master.

These can be exercised with a SystemVerilog formal tool.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 25	Synthesis and Implementation

## 25.1	Synthesis Flow

The generated module is a flat SystemVerilog file intended for standard synthesis:

# Example Vivado flow
read_verilog rtl/delta_axis_flat_4x16.sv
synth_design -top delta_axis_flat_4x16 -part xczu7ev-ffvc1156-2-e

## 25.2	Timing Considerations

- The critical path is typically from s_axis_tvalid/tdest through the decode and mux to m_axis_tdata/tvalid.

- Because the grant is registered, the mux select is stable for a full cycle once a grant is made.

- The TREADY path is combinational from m_axis_tready to s_axis_tready.

## 25.3	Floorplanning Suggestions

- For large crossbars, consider registering s_axis_tdata at the master boundary if the combinational mux path is too long.

- Keep related master/slave groups physically close to reduce routing congestion.

- If Fmax is insufficient, the generator supports --no-counters and reduced DATA_WIDTH to ease timing.

## 25.4	Implementation Checklist

- ☐ Synthesize the generated module in the target tool.

- ☐ Check setup/hold timing on aclk.

- ☐ Verify aresetn deassertion is synchronous to aclk.

- ☐ Run a post-implementation functional simulation if the design is modified after generation.