RTL Design Sherpa

Delta Micro-Architecture Specification 1.0

October 12, 2026

Table of Contents

1 Delta Mas Index	12

2 Delta Hardware Architecture Specification Index	13

2.1 Overview	13

2.2 Related Modules	14

2.3 Navigation	15

2.3.1 Front Matter	15

2.3.2 Chapter 1: Introduction	15

2.3.3 Chapter 2: System Overview	15

2.3.4 Chapter 3: Architecture	15

2.3.5 Chapter 4: Interfaces	15

2.3.6 Chapter 5: Performance	15

2.3.7 Chapter 6: Integration	15

3 Delta: AXI-Stream Crossbar Generator - Product Requirements Document	17

3.1 Executive Summary	18

3.2 1. Project Goals	19

3.2.1 1.1 Primary Goals	19

3.2.2 1.2 Non-Goals	19

3.3 2. Architecture	20

3.3.1 2.1 Flat Crossbar Topology	20

3.3.2 2.2 Tree Topology (retired 2026-10-12 — never implemented in the framework generator)	20

3.4 3. Functional Requirements	22

3.4.1 3.1 Code Generation	22

3.4.2 3.2 Performance Modeling	22

3.4.3 3.3 Protocol Compliance	22

3.5 4. Non-Functional Requirements	24

3.5.1 4.1 Performance Targets	24

3.5.2 4.2 Resource Targets	24

3.5.3 4.3 Code Quality	24

3.6 5. Interface Specifications	25

3.6.1 5.1 AXI-Stream Master Interface (S_AXIS)	25

3.6.2 5.2 AXI-Stream Slave Interface (M_AXIS)	25

3.6.3 5.3 Clock and Reset	26

3.7 6. Key Design Decisions	27

3.7.1 6.1 Why AXI-Stream vs AXI4?	27

3.7.2 6.2 Why Round-Robin vs Fixed Priority?	27

3.7.3 6.3 Why Flat AND Tree Topologies?	27

3.7.4 6.4 Why Python Generator vs Manual RTL?	27

3.8 7. Comparison to APB Crossbar	28

3.8.1 7.1 Similarities (~95% Code Reuse)	28

3.8.2 7.2 Differences	28

3.8.3 7.3 Migration Effort	28

3.9 8. Use Cases	29

3.9.1 8.1 Primary: RISC Cores + DSP Array	29

3.9.2 8.2 Secondary: Educational Demonstration	29

3.9.3 8.3 Future: Integration with RAPIDS	29

3.10 9. Verification Strategy	30

3.10.1 9.1 Generator Verification	30

3.10.2 9.2 RTL Verification	30

3.10.3 9.3 Model Validation	30

3.11 10. Documentation Requirements	31

3.11.1 10.1 Specifications	31

3.11.2 10.2 Code Documentation	31

3.12 11. Success Metrics	32

3.12.1 11.1 Functional Metrics	32

3.12.2 11.2 Performance Metrics	32

3.12.3 11.3 Quality Metrics	32

3.13 12. Timeline and Milestones	33

3.13.1 Week 1: Generator and Models	33

3.13.2 Week 2: RTL and Verification	33

3.13.3 Week 3: Integration and Documentation	33

3.14 13. Open Questions	34

3.15 14. Revision History	35

3.16 16. Documentation	36

3.17 Appendix A: Glossary	37

4 Claude Code Guide: Delta (AXI-Stream Crossbar Generator)	38

4.1 Quick Context	39

4.2 How It Actually Works (verified against rtl/delta_axis_flat_4x16.sv)	40

4.3 Repository Map	41

4.4 Critical Rules	42

4.5 Common Tasks	43

5 Document Information	44

5.1 Delta Micro-Architecture Specification	44

5.2 Revision History	45

5.3 Document Purpose	46

5.4 Intended Audience	47

5.5 Related Documents	48

6 Overview	49

6.1 What This Document Describes	49

6.2 Generated Module Structure	50

6.3 Key Micro-Architectural Decisions	51

6.4 Block Diagram	52

6.4.1 Figure 1.1: Delta Micro-Architecture Block Diagram	52

6.5 Reference RTL	55

7 Request Decode	56

7.1 Purpose	56

7.2 Implementation	57

7.3 TDEST Range Check	58

7.4 Relationship to Arbitration	59

7.5 Notes	60

8 Per-Slave Round-Robin Arbiter	61

8.1 State Variables	61

8.2 FSM Behavior	62

8.2.1 Idle State (packet_active[s] == 0)	62

8.2.2 Active State (packet_active[s] == 1)	62

8.3 Round-Robin Pseudocode	63

8.4 Fairness	64

8.5 Reset Behavior	65

9 Crossbar Multiplexer	66

9.1 Purpose	66

9.2 Implementation	67

9.3 Signal Selection	68

9.4 Default Values	69

9.5 Timing	70

10 TREADY Steering	71

10.1 Purpose	71

10.2 Implementation	72

10.3 Behavior	73

10.4 Deadlock Avoidance	74

11 TWAKEUP Steering (AMBA5)	75

11.1 Purpose	75

11.2 Implementation	76

11.3 Semantics	77

11.4 Integration Notes	78

12 Packet Counters	79

12.1 Purpose	79

12.2 Implementation	80

12.3 Behavior	81

12.4 Disabling Counters	82

13 Assertions	83

13.1 Overview	83

13.2 One-Hot Grant Check	84

13.3 TDEST Range Check	85

13.4 Enabling Assertions	86

13.5 Notes	87

14 Multi-Master Contention	88

14.1 Scenario	88

14.2 Cycle Trace	89

14.3 Observations	91

14.4 Same-Cycle Release and Re-Arbitration	92

15 Packet Interleave Prevention	93

15.1 Rule	93

15.2 Mechanism	94

15.3 Why It Matters	95

15.4 Example: Attempted Interleave	96

15.5 Corner Cases	97

16 Module Structure	98

16.1 Generated File	98

16.2 Module Header	99

16.2.1 Base Flavor Header	99

16.2.2 Rich Flavor Additions	99

16.3 Internal Organization	100

16.4 Filelist	101

17 Parameters and Logic Implications	102

17.1 Parameter Summary	102

17.2 Width Calculations	103

17.3 Parameter Interaction	104

17.4 RTL Constants	105

18 Test Strategy	106

18.1 Current Status	106

18.2 Recommended Test Plan	107

18.2.1 1. Routing Coverage	107

18.2.2 2. Arbitration Fairness	107

18.2.3 3. Packet Atomicity	107

18.2.4 4. Backpressure	107

18.2.5 5. Reset	107

18.2.6 6. Rich Flavor	107

18.2.7 7. Formal Assertions	107

18.3 Lint and Generation Checks	108

List of Figures

Figure 1.1: Delta Micro-Architecture Block Diagram	52

List of Tables

Table 1: Table 1.1: Key Micro-Architectural Decisions	51

Table 2: Table 2.1: Arbiter State Variables	61

Table 3: Table 3.1: Contention Trace	89

Table 4: Table 3.2: Interleave Prevention	96

Table 5: Table 4.1: Parameter-to-Logic Mapping	102

List of Waveforms

No waveforms in this document.

# 1	Delta Mas Index

Generated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Delta Hardware Architecture Specification Index

## 2.1	Overview

Version: 1.0 Date: 2026-10-12 Purpose: High-level hardware architecture specification for the Delta AXI4-Stream crossbar generator

## 2.2	Related Modules

- Delta MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- QUICK_START.md - Hands-on generator guide

## 2.3	Navigation

Note: Every chapter below is one source file; the document build assembles the spec from these links.

### 2.3.1	Front Matter

- Document Information

### 2.3.2	Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### 2.3.3	Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### 2.3.4	Chapter 3: Architecture

- Block Diagram

- Data Flow

- Protocol Support

### 2.3.5	Chapter 4: Interfaces

- AXI-Stream Base Interface

- AXI-Stream Rich Interface (TSTRB + AMBA5)

- Clock and Reset

### 2.3.6	Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resources

### 2.3.7	Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

- Synthesis and Implementation

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Delta: AXI-Stream Crossbar Generator - Product Requirements Document

Project: Delta Version: 1.0 Date: 2025-10-18 Status: Active Development

## 3.1	Executive Summary

Delta is an AXI-Stream crossbar generator that produces parameterized RTL for routing data between multiple masters and multiple slaves. The name “Delta” follows the water/river theme (like RAPIDS) - deltas are where rivers split into multiple branches, analogous to crossbar routing.

Key Features: - Python-based RTL generation (similar to APB crossbar automation) - Single topology support: flat crossbar (tree topology retired 2026-10-12 — see QUICK_START.md) - Performance modeling (analytical + simulation) - Complete AXI-Stream protocol compliance - Educational focus with rigorous specifications

## 3.2	1. Project Goals

### 3.2.1	1.1 Primary Goals

- Code Generation Excellence

  - Python generator produces clean, parameterized SystemVerilog

  - Reuses patterns from existing APB crossbar automation (~95% code reuse)

  - Single tool generates multiple topologies

- Performance Rigor

  - Analytical models (closed-form latency/throughput)

  - Discrete event simulation (SimPy)

  - Resource estimation

  - Validation against RTL synthesis

- Educational Value

  - Demonstrates specification-driven design

  - Shows code generation techniques

  - Compares topology trade-offs

  - Suitable for instruction on GitHub

- Integration Ready

  - Works with existing CocoTBFramework

  - Compatible with RAPIDS subsystem

  - Supports RISC core + DSP array use case

### 3.2.2	1.2 Non-Goals

- Vendor-specific IP (using custom generators only)

- Protocol conversion (AXIS only, not AXI4/APB)

- Advanced routing algorithms (static TDEST-based only)

- Dynamic reconfiguration

## 3.3	2. Architecture

### 3.3.1	2.1 Flat Crossbar Topology

Description: Full MxN crossbar with per-slave arbiters

Characteristics: - Latency: 2 cycles (arbitration + output register) - Throughput: High aggregate (each slave independent) - Resources: ~1,920 LUTs for 4x16 @ 64-bit - Best For: Production systems requiring low latency

Block Diagram:

Masters (M)          Arbiters (N)         Slaves (N)

M0 ------+----------+-[Arbiter_S0]----- S0
         |          |
M1 ------+          +-[Arbiter_S1]----- S1
         |          |
M2 ------+          +-[Arbiter_S2]----- S2
         |          |
M3 ------+          +-[Arbiter_S15]---- S15

Request Matrix      Grant Matrix
(decode TDEST)      (round-robin)

### 3.3.2	2.2 Tree Topology (retired 2026-10-12 — never implemented in the framework generator)

Description: Hierarchical composition of 1:2 splitters and 2:1 mergers

Characteristics: - Latency: 4-6 cycles (multi-stage pipeline) - Throughput: Lower aggregate (bottleneck at root) - Resources: ~1,600 LUTs for 4x16 @ 64-bit (fewer LUTs but more instances) - Best For: Educational examples, demonstrating modularity

Block Diagram:

Masters     Mergers        Splitters       Slaves

M0 --+
     +-[2:1]--[1:2]--+-[1:2]--+- S0
M1 --+              |         +- S1
                    |
M2 --+              +-[1:2]--+- S2
     +-[2:1]--------+         +- S3
M3 --+

Tree depth: log2(N) stages

## 3.4	3. Functional Requirements

### 3.4.1	3.1 Code Generation

REQ-GEN-001: Python generator shall produce SystemVerilog RTL - Input: Command-line parameters (masters, slaves, data width, topology) - Output: Synthesizable SystemVerilog modules - Verification: Verilator lint clean, synthesis clean

REQ-GEN-002: Generator shall support flat crossbar topology (tree topology retired 2026-10-12 — see QUICK_START.md) REQ-GEN-003: Generator shall support optional TSTRB byte qualifiers (--strb-width N; default 0 = base signal set) so generated crossbars port-match rtl/amba/axis4/* (added 2026-10-12). REQ-GEN-004: Generator shall support optional AMBA 5 sidebands (--amba5): TWAKEUP steering — TDEST-decoded OR per slave, valid before TVALID — and TPARITY (--parity-width, default DATA_WIDTH/8), matching rtl/amba/axis5/* (added 2026-10-12). Both options default off; default output is byte-identical to pre-2026-10-12 generations. - Flat: Single crossbar module - Tree: Hierarchical node composition - Both: Generate both variants with --topology both (retired 2026-10-12; flat only)

REQ-GEN-003: Generated RTL shall be parameterized - NUM_MASTERS, NUM_SLAVES (up to 32x256) - DATA_WIDTH (8, 16, 32, 64, 128, 256, 512, 1024 bits) - DEST_WIDTH, ID_WIDTH (auto-calculated)

REQ-GEN-004: Generator shall follow APB crossbar patterns - ~95% code reuse from APB automation - Same request generation, arbitration, mux patterns - Signal name mapping (pclk->aclk, psel->tvalid, etc.)

### 3.4.2	3.2 Performance Modeling

REQ-MODEL-001: Analytical model shall provide closed-form results - Latency calculation (cycles and nanoseconds) - Throughput estimation (transfers/cycle, Gbps) - Resource estimation (LUTs, FFs)

REQ-MODEL-002: Simulation model shall use discrete event simulation - SimPy-based cycle-accurate model - Statistical analysis (mean, percentiles) - Traffic pattern support (uniform, hotspot, localized)

REQ-MODEL-003: Models shall be validated against RTL - Compare analytical vs simulation results - Validate against synthesis reports - Document discrepancies

### 3.4.3	3.3 Protocol Compliance

REQ-PROTO-001: Generated RTL shall comply with AXI-Stream specification - TVALID/TREADY handshaking - TLAST packet boundaries - TDEST, TID, TUSER sideband signals

REQ-PROTO-002: Arbitration shall provide packet atomicity - Grant locked until TLAST - No packet interleaving - Fair round-robin arbitration

REQ-PROTO-003: Backpressure shall propagate correctly - TREADY from slave to granted master - No deadlocks - No data loss

## 3.5	4. Non-Functional Requirements

### 3.5.1	4.1 Performance Targets

NFR-PERF-001: Flat crossbar latency <= 2 cycles NFR-PERF-002: Flat crossbar Fmax >= 300 MHz (UltraScale+) NFR-PERF-003: Tree topology latency <= 6 cycles (retired with tree topology, 2026-10-12) NFR-PERF-004: Throughput >= 0.7 transfers/cycle under mixed traffic

### 3.5.2	4.2 Resource Targets

NFR-RES-001: Flat 4x16 @ 64-bit <= 2,500 LUTs NFR-RES-002: Tree 4x16 @ 64-bit <= 2,000 LUTs NFR-RES-003: No BRAM usage (pure logic implementation)

### 3.5.3	4.3 Code Quality

NFR-QUAL-001: Generated RTL shall pass Verilator lint NFR-QUAL-002: Generator code shall follow PEP 8 style NFR-QUAL-003: Comprehensive inline documentation NFR-QUAL-004: Assertions for formal verification

## 3.6	5. Interface Specifications

### 3.6.1	5.1 AXI-Stream Master Interface (S_AXIS)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_tdata[m] | DATA_WIDTH | Input | Data payload |
| s_axis_tvalid[m] | 1 | Input | Valid indicator |
| s_axis_tready[m] | 1 | Output | Ready (backpressure) |
| s_axis_tlast[m] | 1 | Input | Packet boundary |
| s_axis_tdest[m] | DEST_WIDTH | Input | Target slave ID |
| s_axis_tid[m] | ID_WIDTH | Input | Transaction ID |
| s_axis_tuser[m] | USER_WIDTH | Input | User sideband |

Array: [NUM_MASTERS] (one per master)

### 3.6.2	5.2 AXI-Stream Slave Interface (M_AXIS)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_tdata[s] | DATA_WIDTH | Output | Data payload |
| m_axis_tvalid[s] | 1 | Output | Valid indicator |
| m_axis_tready[s] | 1 | Input | Ready (backpressure) |
| m_axis_tlast[s] | 1 | Output | Packet boundary |
| m_axis_tdest[s] | DEST_WIDTH | Output | Target slave ID (pass-through) |
| m_axis_tid[s] | ID_WIDTH | Output | Transaction ID (pass-through) |
| m_axis_tuser[s] | USER_WIDTH | Output | User sideband (pass-through) |

Array: [NUM_SLAVES] (one per slave)

### 3.6.3	5.3 Clock and Reset

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| aclk | 1 | Input | Clock |
| aresetn | 1 | Input | Active-low reset |

## 3.7	6. Key Design Decisions

### 3.7.1	6.1 Why AXI-Stream vs AXI4?

Decision: Use AXI-Stream instead of full AXI4

Rationale: - Streaming workloads (RISC cores, DSP arrays) - Simpler protocol (no address phases, burst management) - Better fit for compute fabrics - Still allows future upgrade to AXI4 if needed

### 3.7.2	6.2 Why Round-Robin vs Fixed Priority?

Decision: Default to round-robin arbitration

Rationale: - Fair bandwidth allocation - No starvation - Suitable for general-purpose compute - Can add fixed-priority mode later if needed

### 3.7.3	6.3 Why Flat AND Tree Topologies?

Decision: Support both, not either/or

Rationale: - Flat for production (low latency, high throughput) - Tree for education (modular, demonstrates composition) - Python generator makes both easy (~zero extra cost) - Shows trade-off analysis

### 3.7.4	6.4 Why Python Generator vs Manual RTL?

Decision: Python-based code generation

Rationale: - Reuse existing APB crossbar automation (~95%) - Parameterization without manual edits - Consistent code quality - Educational value (code generation techniques) - Rapid design space exploration

## 3.8	7. Comparison to APB Crossbar

### 3.8.1	7.1 Similarities (~95% Code Reuse)

| Component | APB Crossbar | Delta (AXIS) | Reusable? |
| --- | --- | --- | --- |
| Request generation | Address decode | TDEST decode | [PASS] Pattern same |
| Per-slave arbitration | Round-robin | Round-robin | [PASS] Identical logic |
| Grant matrix | MxN grants | MxN grants | [PASS] Identical |
| Data multiplexing | Mux PRDATA | Mux TDATA/TVALID/TLAST | [PASS] Same pattern |
| Backpressure | PREADY | TREADY | [PASS] Renamed only |

### 3.8.2	7.2 Differences

| Aspect | APB Crossbar | Delta (AXIS) |
| --- | --- | --- |
| Address decode | Range check (base/end) | Direct TDEST (simpler!) |
| Packet concept | Single transaction | Multi-beat packets with TLAST |
| Arbitration | Re-arbitrate every cycle | Lock until TLAST (packet atomicity) |
| Signals | PRDATA, PSLVERR | TDATA, TVALID, TLAST, TDEST, TID, TUSER |
| Read/Write | Separate paths | Unified data path (simpler!) |

### 3.8.3	7.3 Migration Effort

Estimated Time: ~75 minutes (from existing APB generator to AXIS)

Tasks: 1. Rename signals (10 min) 2. Simplify address decode to TDEST (5 min) 3. Add packet atomicity logic (15 min) 4. Add new signals (TLAST, TID, TUSER) to ports and mux (10 min) 5. Update module naming (5 min) 6. Test with 2x2 configuration (30 min)

## 3.9	8. Use Cases

### 3.9.1	8.1 Primary: RISC Cores + DSP Array

Scenario: 4 small RISC-V cores need to route compute tasks to 16 DSP accelerators

Configuration: - 4 masters (RISC cores) - 16 slaves (DSP arrays) - 64-bit data width - Flat topology (low latency critical)

Benefits: - Each RISC core can target any DSP (full flexibility) - Round-robin ensures fair DSP access - Low 2-cycle latency for task dispatch

### 3.9.2	8.2 Secondary: Educational Demonstration

Scenario: Teach students about interconnect design trade-offs

Configuration: - Generate flat crossbars (tree topology retired 2026-10-12) - Compare latency, throughput, resources - Show code generation techniques - Demonstrate verification methodology

Benefits: - Hands-on learning with working RTL - Clear performance comparisons - Demonstrates real-world design process

### 3.9.3	8.3 Future: Integration with RAPIDS

Scenario: Use Delta for compute fabric routing, RAPIDS for memory DMA

Configuration: - RAPIDS handles descriptor-based memory transfers - Delta routes compute tasks between processors - Protocol adapter (Network 2.0 <-> AXI-Stream)

Benefits: - Separate concerns (memory vs compute) - Reusable components - Scalable architecture

## 3.10	9. Verification Strategy

### 3.10.1	9.1 Generator Verification

Approach: 1. Unit tests for each generator function (Python unittest) 2. Lint generated RTL (Verilator) 3. Synthesis test (Vivado/Yosys) 4. Compare output for different parameters

Success Criteria: - All Python tests pass - Generated RTL lints clean - Synthesis completes without errors

### 3.10.2	9.2 RTL Verification

Approach: 1. CocoTB testbench framework (reuse AMBA patterns) 2. Test all MxS routing combinations 3. Concurrent traffic scenarios 4. Backpressure stress tests 5. Packet atomicity verification

Success Criteria: - 100% routing coverage - No deadlocks under stress - Packet atomicity confirmed - Performance matches model (±10%)

### 3.10.3	9.3 Model Validation

Approach: 1. Analytical model vs simulation model comparison 2. Simulation model vs RTL CocoTB results 3. Synthesis reports vs resource estimates

Success Criteria: - Latency within ±1 cycle - Throughput within ±15% - Resources within ±20%

## 3.11	10. Documentation Requirements

### 3.11.1	10.1 Specifications

PRD (this document) - Requirements, architecture, use cases

Technical Specification - Detailed block diagrams - Interface timing diagrams - Performance analysis

User Guide - Generator usage examples - Integration patterns - Best practices

### 3.11.2	10.2 Code Documentation

Generator Code: - Docstrings for all functions - Inline comments for complex logic - Usage examples in header

Generated RTL: - Module header with configuration - Block-level comments - Signal descriptions

Performance Models: - Algorithm explanations - Formula derivations - Validation methodology

## 3.12	11. Success Metrics

### 3.12.1	11.1 Functional Metrics

- [PASS] Generator produces lint-clean RTL

- [PASS] All routing combinations verified

- [PASS] No protocol violations

- [PASS] Packet atomicity enforced

### 3.12.2	11.2 Performance Metrics

- [PASS] Flat crossbar: 2-cycle latency

- [PASS] Tree topology: <=6-cycle latency

- [PASS] Throughput: >=0.7 transfers/cycle

- [PASS] Fmax: >=300 MHz

### 3.12.3	11.3 Quality Metrics

- [PASS] Code reuse: >=90% from APB generator

- [PASS] Model accuracy: ±10% vs RTL

- [PASS] Resource estimate: ±20% vs synthesis

- [PASS] Documentation: Complete and clear

## 3.13	12. Timeline and Milestones

### 3.13.1	Week 1: Generator and Models

- [PASS] Python generator (flat topology)

- [PASS] Analytical performance model

- [PASS] Simulation model (SimPy)

- [PASS] Specifications

### 3.13.2	Week 2: RTL and Verification

- Generate RTL variants (flat 4x16; tree variant retired 2026-10-12)

- CocoTB testbench framework

- Basic functional tests

- Performance validation

### 3.13.3	Week 3: Integration and Documentation

- RISC + DSP integration example

- Complete user guide

- Performance comparison report

- Educational materials

## 3.14	13. Open Questions

Q1: Should we add configurable FIFO insertion for burst buffering? - Status: Deferred to v1.1 (keep v1.0 simple)

Q2: Support for weighted round-robin arbitration? - Status: Deferred (default round-robin sufficient for now)

Q3: Integration with existing APB crossbar generator (unified tool)? - Status: Under consideration (separate for now, may merge later)

## 3.15	14. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2025-10-18 | RTL Design Sherpa | Initial release |

## 3.16	16. Documentation

The aspirational DELTA mesh NoC specification was split out of this component on 2026-10-12 and lives at projects/components/noc-ip/delta_spec/ (agent guide: projects/components/noc-ip/CLAUDE.md). To render it:

cd projects/components/noc-ip
./generate_pdf.sh --rev 0.25

Rendered DOCX/PDF land in noc-ip/delta_spec/. This component — the AXIS crossbar generator — is documented by README.md, QUICK_START.md, and delta_index.md.

## 3.17	Appendix A: Glossary

- AXIS: AXI-Stream (streaming variant of AXI protocol)

- Delta: Project name (river delta = branching flow, like crossbar routing)

- Flat Topology: Full MxN crossbar with all connections

- Tree Topology: Hierarchical composition of 1:2 and 2:1 nodes

- Packet Atomicity: Locking grant until TLAST (prevent interleaving)

- TDEST: Transaction destination (slave ID in AXI-Stream)

- TID: Transaction ID (master identifier in AXI-Stream)

END OF PRD

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Claude Code Guide: Delta (AXI-Stream Crossbar Generator)

Version: 1.0 Last Updated: 2026-10-12 Purpose: AI-specific guidance for the Delta component (fabric-gen-ip)

## 4.1	Quick Context

What: Delta is a Python-based AXI4-Stream (AXIS) crossbar generator. It emits a single self-contained SystemVerilog module per configuration — a flat M×N stream crossbar routing s_axis_* masters to m_axis_* slaves.

Status: Active development. RTL generates and simulates; there is no dv/ test suite yet (the CocoTB suite is the known TODO). Re-homed from noc-ip/delta on 2026-10-12 — the move did not change any behavior.

Your Role: Help users generate, parameterize, and validate AXIS crossbars; keep generated RTL and this documentation in sync.

Not this component: the aspirational DELTA mesh NoC design draft (routers, virtual channels, XY routing) lives separately at projects/components/noc-ip/delta_spec/ — no NoC RTL exists. When “delta” comes up without qualification, this crossbar generator is what is meant.

## 4.2	How It Actually Works (verified against rtl/delta_axis_flat_4x16.sv)

- Route decode: the target slave comes straight from s_axis_tdest (DEST_WIDTH bits) — there is no address map.

- Arbitration: per-slave round-robin; the grant is packet-atomic — once granted, a master holds the slave until TLAST.

- Data path: TDATA/TVALID/TLAST/TDEST/TID/TUSER crossbar mux is combinational; the grant is the registered stage.

- Backpressure: TREADY from the granted slave is steered back to the granted master only.

- Optional flavors (2026-10-12, default off, default output byte-identical): TSTRB (--strb-width) byte qualifiers, and AMBA 5 TWAKEUP/TPARITY (--amba5, --parity-width default DATA_WIDTH/8) so generated crossbars port-match rtl/amba/axis4/* and rtl/amba/axis5/*. TWAKEUP is TDEST-decoded (OR per slave) and may assert before TVALID, per the AMBA 5 hint semantics.

- Optional: packet counters.

Parameters on the generated module: DATA_WIDTH, DEST_WIDTH, ID_WIDTH, USER_WIDTH, NUM_MASTERS, NUM_SLAVES.

## 4.3	Repository Map

| Path | What |
| --- | --- |
| bin/delta_generator.py | The generator (DeltaConfig, DeltaFlatCrossbar, CLI) |
| bin/delta_performance_model.py | Analytical + simulation performance model |
| bin/complete_tree_generator.py | Retired tree-topology prototype (kept for history) |
| rtl/delta_axis_flat_4x16.sv | Reference generated crossbar (checked in) |
| rtl_test/*.sv | Generated variants (2×4, 3×8, 2×16, 4×16) |
| rtl_test/delta_axis_flat_2x4_strb_amba5.sv | Rich-flavor example (TSTRB + TWAKEUP/TPARITY) |
| rtl/filelists/*.f | Filelists ($REPO_ROOT-relative) |
| README.md / PRD.md / QUICK_START.md | Product docs (PRD REQ-GEN-002 is flat-only since 2026-10-12) |
| delta_index.md | Crossbar doc index |
| TREE_TOPOLOGY_TEST_RESULTS.md | Historical results for the retired tree path |

## 4.4	Critical Rules

- Flat only. --topology tree (and both) prints “not yet implemented in framework version”. Do not promise tree generation; REQ-GEN-002 was amended 2026-10-12.

- Regenerate, don’t hand-edit. rtl/delta_axis_flat_4x16.sv and the rtl_test/ variants are generator outputs. Fix bin/delta_generator.py and re-emit; note --topology tree was retired — regenerate with flat.

- AXIS only. No AXI4/AXI4-Lite/APB conversion here (PRD non-goal). For memory-mapped fabrics see siblings ../bridge/ and ../apbx-xbar/.

- Filelist paths in rtl/filelists/*.f are $REPO_ROOT-relative — keep them working from any cwd.

- Do not resurrect the NoC (noc-ip/delta_spec/) unless the owner explicitly asks for NoC work.

## 4.5	Common Tasks

# Generate a fresh crossbar
python3 bin/delta_generator.py --topology flat --masters 4 --slaves 16 \
    --data-width 64 --output-dir rtl/

# Rich flavor: TSTRB + AMBA 5 sidebands (matches rtl/amba/axis4 + axis5)
python3 bin/delta_generator.py --topology flat --masters 2 --slaves 4 \
    --data-width 32 --strb-width 4 --amba5 --output-dir rtl_test/

# Performance comparison (historical flat/tree data retained)
python3 bin/delta_performance_model.py --topology compare

Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Document Information

## 5.1	Delta Micro-Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Delta Micro-Architecture Specification |
| Version | 1.0 |
| Date | 2026-10-12 |
| Status | Released |
| Classification | Open Source - MIT License |

## 5.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-10-12 | RTL Design Sherpa | Initial release; describes the generated flat crossbar RTL including request decode, packet-atomic RR arbitration, mux, TREADY/TWAKEUP steering, counters, and assertions. |

## 5.3	Document Purpose

This Micro-Architecture Specification (MAS) is the internals document for the Delta AXI4-Stream crossbar generator — the one you open when you need to know what the generated RTL actually does, block by block. It covers:

- Block-level implementation of the generated SystemVerilog

- Arbitration state behavior and grant hold/release rules

- Cycle-level behavior examples

- Parameter-to-logic implications

- Verification strategy

For the high-level architecture, interface requirements, and integration guidelines, see the companion Delta Hardware Architecture Specification (HAS).

## 5.4	Intended Audience

- RTL engineers reading or modifying the generated crossbar

- Verification engineers writing protocol checks

- System architects resolving timing or arbitration questions

- Technical reviewers auditing the implementation

## 5.5	Related Documents

- Delta HAS - Hardware Architecture Specification (high-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI assistant integration guide

- QUICK_START.md - Hands-on generator guide

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 6	Overview

## 6.1	What This Document Describes

This Micro-Architecture Specification (MAS) describes the internals of the SystemVerilog module produced by bin/delta_generator.py. It is intended for readers who need to understand, debug, or modify the generated RTL.

## 6.2	Generated Module Structure

The generator emits a single module named delta_axis_flat_MxN where M is NUM_MASTERS and N is NUM_SLAVES. The module contains:

- TDEST decode — builds the per-slave request matrix.

- Per-slave round-robin arbiter — packet-atomic grant logic.

- Crossbar mux — combinational selection of payload and control.

- TREADY steering — backpressure routing.

- TWAKEUP steering — AMBA 5 wakeup hint routing (optional).

- Packet counters — per-master and per-slave counters (optional).

- Assertions — ifdef FORMAL checks.

## 6.3	Key Micro-Architectural Decisions

Table 1.1: Key Micro-Architectural Decisions

| Decision | Rationale |
| --- | --- |
| Registered grant, combinational mux | Two-cycle latency with a short critical path. |
| Per-slave arbiter | Independent scheduling; no head-of-line blocking across slaves. |
| TDEST direct decode | No address map; simpler and lower latency than address-range decode. |
| Packet-atomic grant | Preserves AXI4-Stream packet semantics; prevents interleaving. |
| Optional sidebands default off | Keeps base output byte-identical to earlier versions. |

## 6.4	Block Diagram

### 6.4.1	Figure 1.1: Delta Micro-Architecture Block Diagram

Delta MAS Block Diagram

The micro-architecture diagram shows the internal blocks of the generated RTL. The request/control flow descends through TDEST decode, the request matrix, the packet-atomic round-robin arbiter (inverted-trapezium scheduler), the grant matrix, and the crossbar mux to the slave boundary. The payload flow follows its own route on the left. The backpressure flow returns up through TREADY steering as a dashed sideband. Optional blocks (TWAKEUP steering, packet counters, and SVA assertions) are dashed.

## 6.5	Reference RTL

The descriptions in this document are verified against:

- rtl/delta_axis_flat_4x16.sv (base flavor)

- rtl_test/delta_axis_flat_2x4_strb_amba5.sv (rich flavor)

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 7	Request Decode

## 7.1	Purpose

The request decode block translates each master’s s_axis_tvalid and s_axis_tdest into a per-slave request matrix. A set bit in request_matrix[s][m] indicates that master m is requesting slave s.

## 7.2	Implementation

The decode is combinational:

logic [NUM_MASTERS-1:0] request_matrix [NUM_SLAVES];

always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++) begin
        request_matrix[s] = '0;
    end
    for (int m = 0; m < NUM_MASTERS; m++) begin
        if (s_axis_tvalid[m] &&
            (s_axis_tdest[m] < NUM_SLAVES[$clog2(NUM_SLAVES+1)-1:0])) begin
            request_matrix[s_axis_tdest[m]][m] = 1'b1;
        end
    end
end

## 7.3	TDEST Range Check

The comparison s_axis_tdest[m] < NUM_SLAVES prevents out-of-range indices from corrupting the request_matrix array. The width of the comparison is sized to cover the full NUM_SLAVES value.

## 7.4	Relationship to Arbitration

The request matrix is the input to each slave’s round-robin arbiter. Only slaves with at least one request bit set are candidates for a new grant.

## 7.5	Notes

- The decode does not depend on TREADY; a master may request even while stalled.

- TDEST is sampled combinationally and must be stable while TVALID is asserted.

- There is no address map; TDEST is the slave index directly.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 8	Per-Slave Round-Robin Arbiter

## 8.1	State Variables

Each slave s maintains three registered variables:

Table 2.1: Arbiter State Variables

| Variable | Width | Purpose |
| --- | --- | --- |
| grant_matrix[s] | NUM_MASTERS bits | One-hot (or zero-hot) grant vector. |
| last_grant[s] | clog2(NUM_MASTERS) bits | Pointer to the most recently granted master. |
| packet_active[s] | 1 bit | Set while a packet is in progress on this slave. |

## 8.2	FSM Behavior

The arbiter has two operational states for each slave:

### 8.2.1	Idle State (packet_active[s] == 0)

- If request_matrix[s] is non-zero, perform round-robin arbitration.

- The search starts at last_grant[s] + 1 and wraps through all masters.

- The first requesting master is granted; last_grant[s] is updated and packet_active[s] is set.

- If no master requests, grant_matrix[s] remains zero.

### 8.2.2	Active State (packet_active[s] == 1)

- The current grant is held.

- On the cycle where m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s] is true, packet_active[s] is cleared and grant_matrix[s] is reset to zero.

- If TLAST is accepted, the next cycle may re-arbitrate for a new master.

## 8.3	Round-Robin Pseudocode

if (packet_active[s]) begin
    if (m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s]) begin
        packet_active[s] <= 1'b0;
        grant_matrix[s]  <= '0;
    end
end else if (|request_matrix[s]) begin
    grant_matrix[s] = '0;
    for (int i = 0; i < NUM_MASTERS; i++) begin
        int m = (last_grant[s] + 1 + i) % NUM_MASTERS;
        if (request_matrix[s][m] && grant_matrix[s] == '0) begin
            grant_matrix[s][m] = 1'b1;
            last_grant[s]      = m[$clog2(NUM_MASTERS)-1:0];
            packet_active[s]   = 1'b1;
        end
    end
end else begin
    grant_matrix[s] <= '0;
end

## 8.4	Fairness

Round-robin guarantees that no requesting master is starved. The longest wait for a contending master is bounded by the number of other contending masters and the length of their packets.

## 8.5	Reset Behavior

On reset:

- grant_matrix[s] <= '0

- last_grant[s] <= '0

- packet_active[s] <= 1'b0

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 9	Crossbar Multiplexer

## 9.1	Purpose

The crossbar mux selects the payload and control signals from the granted master and drives them onto the selected slave interface. There is one mux per slave.

## 9.2	Implementation

Each slave mux is combinational:

generate
    for (genvar s = 0; s < NUM_SLAVES; s++) begin : gen_slave_mux
        always_comb begin
            m_axis_tdata[s]  = '0;
            m_axis_tvalid[s] = 1'b0;
            m_axis_tlast[s]  = 1'b0;
            m_axis_tdest[s]  = '0;
            m_axis_tid[s]    = '0;
            m_axis_tuser[s]  = '0;

            for (int m = 0; m < NUM_MASTERS; m++) begin
                if (grant_matrix[s][m]) begin
                    m_axis_tdata[s]  = s_axis_tdata[m];
                    m_axis_tvalid[s] = s_axis_tvalid[m];
                    m_axis_tlast[s]  = s_axis_tlast[m];
                    m_axis_tdest[s]  = s_axis_tdest[m];
                    m_axis_tid[s]    = s_axis_tid[m];
                    m_axis_tuser[s]  = s_axis_tuser[m];
                end
            end
        end
    end
endgenerate

## 9.3	Signal Selection

The mux forwards the following signals from the granted master to the selected slave:

- TDATA

- TVALID

- TLAST

- TDEST

- TID

- TUSER

- TSTRB (when enabled)

- TPARITY (when enabled)

## 9.4	Default Values

When no grant is active for a slave, the mux drives all outputs to zero and m_axis_tvalid[s] to low. This is a safe idle state.

## 9.5	Timing

Because the mux is combinational, the output settles after the grant and the input signals are stable. The grant is registered, so the mux select is stable for a full clock cycle once a packet begins.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 10	TREADY Steering

## 10.1	Purpose

TREADY steering routes each slave’s m_axis_tready back to the master that currently holds the grant for that slave. Ungranted masters see s_axis_tready[m] == 0.

## 10.2	Implementation

generate
    for (genvar m = 0; m < NUM_MASTERS; m++) begin : gen_master_tready
        always_comb begin
            s_axis_tready[m] = 1'b0;
            for (int s = 0; s < NUM_SLAVES; s++) begin
                if (grant_matrix[s][m]) begin
                    s_axis_tready[m] = m_axis_tready[s];
                end
            end
        end
    end
endgenerate

## 10.3	Behavior

- Only one slave can grant a given master at a time (packets are atomic and a master targets one slave per packet), so the loop effectively selects a single m_axis_tready[s].

- If a master is not currently granted by any slave, it is stalled.

- Backpressure propagates combinationally from slave to master.

## 10.4	Deadlock Avoidance

The design avoids deadlock because:

- Grants are released after TLAST, freeing the slave for new requests.

- TREADY is combinationally steered; a slave ready transition immediately reaches the granted master.

- There are no FIFOs or credit loops in the base crossbar.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 11	TWAKEUP Steering (AMBA5)

## 11.1	Purpose

When --amba5 is enabled, the generated module includes s_axis_twakeup and m_axis_twakeup ports. TWAKEUP is an AMBA 5 hint that a master will soon send a transaction; it may assert before TVALID.

## 11.2	Implementation

The wakeup steering is combinational and independent of the grant:

always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++) begin
        m_axis_twakeup[s] = 1'b0;
        for (int m = 0; m < NUM_MASTERS; m++) begin
            if (s_axis_twakeup[m] &&
                (s_axis_tdest[m] == s[$clog2(NUM_SLAVES)-1:0])) begin
                m_axis_twakeup[s] = 1'b1;
            end
        end
    end
end

## 11.3	Semantics

- m_axis_twakeup[s] is the OR of s_axis_twakeup[m] from every master whose TDEST points to slave s.

- No grant is required; the hint is decoded directly from TDEST.

- TWAKEUP may be high while TVALID is low.

- The hint is not a handshake; the slave is not required to respond before data arrives.

## 11.4	Integration Notes

- TWAKEUP is only emitted when --amba5 is specified.

- The default parity width is DATA_WIDTH / 8; override with --parity-width.

- TWAKEUP does not affect arbitration timing.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 12	Packet Counters

## 12.1	Purpose

Optional 32-bit packet counters track completed packets at the master boundary and the slave boundary. They are enabled by default and can be disabled with --no-counters.

## 12.2	Implementation

logic [31:0] pkt_count_master [NUM_MASTERS];
logic [31:0] pkt_count_slave [NUM_SLAVES];

generate
    for (genvar m = 0; m < NUM_MASTERS; m++) begin : gen_master_counters
        always_ff @(posedge aclk or negedge aresetn) begin
            if (!aresetn)
                pkt_count_master[m] <= '0;
            else if (s_axis_tvalid[m] && s_axis_tready[m] && s_axis_tlast[m])
                pkt_count_master[m] <= pkt_count_master[m] + 1;
        end
    end

    for (genvar s = 0; s < NUM_SLAVES; s++) begin : gen_slave_counters
        always_ff @(posedge aclk or negedge aresetn) begin
            if (!aresetn)
                pkt_count_slave[s] <= '0;
            else if (m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s])
                pkt_count_slave[s] <= pkt_count_slave[s] + 1;
        end
    end
endgenerate

## 12.3	Behavior

- A master counter increments when the master completes a packet (last beat accepted).

- A slave counter increments when the slave completes a packet (last beat accepted).

- Counters wrap at 32 bits.

- Counters are not exposed on module ports; they are internal observation points.

## 12.4	Disabling Counters

Use --no-counters to remove the counter logic and save area:

python3 bin/delta_generator.py --topology flat --masters 4 --slaves 16 \
    --data-width 64 --no-counters --output-dir rtl/

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 13	Assertions

## 13.1	Overview

All generated Delta modules include an ifdef FORMAL block with SystemVerilog assertions for debug and formal verification.

## 13.2	One-Hot Grant Check

Each slave must grant at most one master at a time:

assert property (@(posedge aclk) disable iff (!aresetn)
    $onehot0(grant_matrix[s])
) else $error("Grant matrix not one-hot for slave %0d", s);

$onehot0 allows the grant vector to be all-zero (idle) but not multi-hot.

## 13.3	TDEST Range Check

Each master must drive a valid slave index when it asserts TVALID:

assert property (@(posedge aclk) disable iff (!aresetn)
    s_axis_tvalid[m] |-> s_axis_tdest[m] < NUM_SLAVES
) else $error("TDEST out of range for master %0d", m);

## 13.4	Enabling Assertions

Define FORMAL during simulation or formal analysis:

verilator --lint-only -DFORMAL rtl/delta_axis_flat_4x16.sv

## 13.5	Notes

- Assertions are disabled by default to avoid simulation overhead.

- The TDEST range assertion duplicates the combinational safety check in the request decode block.

- Additional protocol assertions (TVALID stability, TLAST packet integrity) are candidates for the planned cocotb testbench.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 14	Multi-Master Contention

## 14.1	Scenario

Two masters, M0 and M1, both want to send packets to the same slave S0. S0 is initially idle. M0 sends a two-beat packet; M1 sends a one-beat packet.

## 14.2	Cycle Trace

Assumption: last_grant[S0] was 1 before the trace begins.

Table 3.1: Contention Trace

| Cycle | Event | request_matrix[S0] | grant_matrix[S0] | packet_active[S0] | Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | M0 and M1 both request S0 | 2'b11 | 2'b00 | 0 | Slave idle. |
| 1 | Arbiter picks M0 (RR after last_grant=1) | 2'b11 | 2'b01 | 1 | Grant registered. |
| 2 | M0 beat 0 accepted | 2'b10 | 2'b01 | 1 | M1 waits. |
| 3 | M0 beat 1 (TLAST) accepted | 2'b10 | 2'b00 | 0 | Packet ends; grant released. |
| 4 | Arbiter picks M1 | 2'b10 | 2'b10 | 1 | M1 now granted. |
| 5 | M1 beat 0 (TLAST) accepted | 2'b00 | 2'b00 | 0 | Both packets complete. |

## 14.3	Observations

- M1 waits while M0 completes its packet.

- The arbiter does not re-arbitrate inside a packet.

- After TLAST, the next arbitration cycle grants M1 because M0 was the previous grant.

- Round-robin ensures M1 is not starved.

## 14.4	Same-Cycle Release and Re-Arbitration

On the cycle that TLAST is accepted, the grant is released. The next rising edge can register a new grant if a request is pending. This gives a one-cycle gap between back-to-back packets from different masters.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 15	Packet Interleave Prevention

## 15.1	Rule

A slave can source data from only one master within a single packet. The grant is locked from the first beat of the packet until the TLAST beat is accepted.

## 15.2	Mechanism

The packet_active[s] flag is set when a new grant is made. While packet_active[s] is high:

- grant_matrix[s] is held constant.

- The crossbar mux continues to select the same master.

- s_axis_tready for the granted master follows only m_axis_tready[s].

When m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s] is true, packet_active[s] clears and the grant is released.

## 15.3	Why It Matters

AXI4-Stream slaves often expect packet-atomic streams. If beats from different masters were interleaved, a slave receiving the mixed stream could misinterpret packet boundaries, corrupt state, or violate protocol.

## 15.4	Example: Attempted Interleave

Suppose M0 and M1 both target S0 and both have multi-beat packets:

Table 3.2: Interleave Prevention

| Cycle | Without atomicity (hypothetical) | With Delta packet atomicity |
| --- | --- | --- |
| 0 | M0 beat 0 | M0 beat 0 |
| 1 | M1 beat 0 (interleaved!) | M0 beat 1 |
| 2 | M0 beat 1 | M0 beat 2 (TLAST) |
| 3 | M1 beat 1 (TLAST) | M1 beat 0 |

Delta prevents the interleaved case by holding M0’s grant until its TLAST.

## 15.5	Corner Cases

- If the granted master drops TVALID mid-packet, the slave sees no valid beats until the master reasserts it. The grant is still held.

- If the downstream slave is never ready, the packet can stall indefinitely; there is no timeout. This is consistent with standard AXI4-Stream behavior.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 16	Module Structure

## 16.1	Generated File

The generator produces one file per configuration:

delta_axis_flat_MxN.sv

where M is NUM_MASTERS and N is NUM_SLAVES.

## 16.2	Module Header

The module begins with parameters and port declarations. The exact port list depends on the selected richness options.

### 16.2.1	Base Flavor Header

module delta_axis_flat_4x16 #(
    parameter int DATA_WIDTH  = 64,
    parameter int DEST_WIDTH  = 4,
    parameter int ID_WIDTH    = 2,
    parameter int USER_WIDTH  = 1,
    parameter int NUM_MASTERS = 4,
    parameter int NUM_SLAVES  = 16
)(
    input  logic aclk,
    input  logic aresetn,
    // s_axis and m_axis base ports...
);

### 16.2.2	Rich Flavor Additions

When --strb-width is used, STRB_WIDTH is added to the parameter list and s_axis_tstrb/m_axis_tstrb are added to the ports.

When --amba5 is used, PW_WIDTH is added to the parameter list and s_axis_twakeup/s_axis_tparity/m_axis_twakeup/m_axis_tparity are added to the ports.

## 16.3	Internal Organization

Inside the module, the generator emits sections in this order:

- Header comment with configuration.

- Request decode.

- Per-slave round-robin arbiter.

- Crossbar data mux.

- TREADY steering.

- TWAKEUP steering (if enabled).

- Performance counters (unless disabled).

- Assertions (under ifdef FORMAL).

## 16.4	Filelist

The checked-in RTL includes $REPO_ROOT-relative filelists under rtl/filelists/ for use with simulation and synthesis flows.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 17	Parameters and Logic Implications

## 17.1	Parameter Summary

Table 4.1: Parameter-to-Logic Mapping

| Parameter | Generated From | Logic Implication |
| --- | --- | --- |
| DATA_WIDTH | --data-width | Width of all TDATA and TPARITY default calculations. |
| DEST_WIDTH | clog2(NUM_SLAVES) | Width of TDEST; determines request decode comparison width. |
| ID_WIDTH | clog2(NUM_MASTERS) | Width of TID; pass-through only. |
| USER_WIDTH | --user-width | Width of TUSER; pass-through only. |
| NUM_MASTERS | --masters | Size of s_axis_* arrays and grant/request matrix columns. |
| NUM_SLAVES | --slaves | Size of m_axis_* arrays and grant/request matrix rows. |
| STRB_WIDTH | --strb-width | Width of TSTRB; adds mux ports when > 0. |
| PW_WIDTH | --parity-width or DATA_WIDTH/8 | Width of TPARITY; adds mux ports when --amba5 is set. |

## 17.2	Width Calculations

The generator computes:

dest_width = max(1, ceil(log2(num_slaves)))
id_width   = max(1, ceil(log2(num_masters)))

A single master or slave still gets a 1-bit width to avoid zero-width signals.

## 17.3	Parameter Interaction

- STRB_WIDTH == 0 omits TSTRB entirely.

- PW_WIDTH is meaningful only when --amba5 is set; otherwise TPARITY and TWAKEUP ports are not generated.

- --parity-width without --amba5 is rejected by the CLI parser.

## 17.4	RTL Constants

The generator uses generate/genvar loops sized by NUM_MASTERS and NUM_SLAVES. Changing these parameters after generation requires regenerating the module; the parameter is set at generation time, not truly configurable post-generation. This is intentional: it allows the synthesizer to optimize the fixed-size structure.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 18	Test Strategy

## 18.1	Current Status

As of 2026-10-12, Delta has no dv/ test suite. The RTL is lint-clean and the generator output matches checked-in reference files, but a cocotb-based verification environment is the known TODO.

## 18.2	Recommended Test Plan

When a testbench is created, it should cover the following areas.

### 18.2.1	1. Routing Coverage

- For every master m and every slave s, send at least one packet from m to s.

- Verify that only the target slave receives the packet.

- Verify that m_axis_tdest[s] matches the sent TDEST.

### 18.2.2	2. Arbitration Fairness

- Have multiple masters send continuous packets to the same slave.

- Verify that each master receives approximately equal bandwidth over time.

- Verify that no master is starved.

### 18.2.3	3. Packet Atomicity

- Have two masters send multi-beat packets to the same slave.

- Verify that slave receives the first packet’s beats contiguously up to TLAST before any beat of the second packet.

### 18.2.4	4. Backpressure

- Randomly deassert m_axis_tready[s].

- Verify that only the granted master stalls and that no data is lost.

### 18.2.5	5. Reset

- Assert aresetn mid-transaction.

- Verify all grants clear and counters reset.

### 18.2.6	6. Rich Flavor

- With --strb-width, verify TSTRB pass-through.

- With --amba5, verify TWAKEUP OR-per-slave and TPARITY pass-through.

### 18.2.7	7. Formal Assertions

- Run with +define+FORMAL or equivalent.

- Verify the one-hot grant and TDEST-range assertions never fire.

## 18.3	Lint and Generation Checks

Until a testbench exists, the verification consists of:

python3 bin/delta_generator.py --topology flat --masters 4 --slaves 16 \
    --data-width 64 --output-dir rtl/
verilator --lint-only rtl/delta_axis_flat_4x16.sv