Table of Contents

# Smbus Index

Generated: 2025-12-07

# APB SMBus Specification - Table of Contents

Component: APB System Management Bus (SMBus) Controller Version: 1.0 Last Updated: 2025-12-01 Status: Production Ready

## Document Organization

### Chapter 1: Overview

- 01_overview.md - Component overview

- 02_architecture.md - Architecture

### Chapter 2: Blocks

- 00_overview.md - Block hierarchy

### Chapter 3: Interfaces

- 00_overview.md - Interface summary

### Chapter 4: Programming Model

- 00_overview.md - Programming overview

### Chapter 5: Registers

- 01_register_map.md - Register map

## Block Diagram

SMBus Block Diagram

## Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2025-12-01 | RTL Design Sherpa | Initial specification |

# APB SMBus - Overview

## Introduction

The APB SMBus controller provides System Management Bus communication with APB interface. It supports host controller functionality for accessing SMBus devices.

## Key Features

- SMBus 2.0 compatible

- Host controller mode

- Quick Command, Send/Receive Byte

- Read/Write Byte/Word

- Block Read/Write (up to 32 bytes)

- PEC (Packet Error Checking) support

- Programmable clock divider

- Interrupt-driven operation

- Timeout detection

## Applications

- Temperature monitoring

- Voltage monitoring

- Fan control

- EEPROM access

- Power management

- System health monitoring

## Block Diagram

SMBus Block Diagram

## Timing Diagrams

### Byte Write (Start + Address)

Shows the START condition and 7-bit address transmission.

SMBus Byte Write

START condition is SDA falling while SCL is high. The 7-bit slave address plus R/W bit is clocked out, followed by slave ACK (SDA low during 9th clock).

### Byte Read

Shows slave-to-master data transfer.

SMBus Byte Read

Slave drives 8 data bits while master clocks SCL. Master samples each bit on SCL rising edge, then provides ACK (more data) or NACK (last byte).

### Clock Stretching

Slave flow control by holding SCL low.

SMBus Clock Stretch

When the slave needs processing time, it holds SCL low after the master releases it. Master waits for SCL to rise before continuing. This provides backpressure without data loss.

### Multi-Master Arbitration

Collision detection when multiple masters start simultaneously.

SMBus Arbitration

Both masters monitor SDA while transmitting. If a master drives 1 but reads 0 (wired-AND bus), it loses arbitration and backs off. The winner continues the transaction.

### Packet Error Check (PEC)

CRC-8 error detection for data integrity.

SMBus PEC

PEC is calculated over address, command, and data bytes using CRC-8. The PEC byte is transmitted after data and verified by the receiver to detect transmission errors.

## Register Summary

| Offset | Name | Description |
| --- | --- | --- |
| 0x00 | SMBUS_STATUS | Status register |
| 0x04 | SMBUS_CONTROL | Control register |
| 0x08 | SMBUS_COMMAND | Command type |
| 0x0C | SMBUS_ADDRESS | Target address |
| 0x10 | SMBUS_DATA0 | Data byte 0 |
| 0x14 | SMBUS_DATA1 | Data byte 1 |
| 0x18 | SMBUS_BLOCK | Block data count |
| 0x1C | SMBUS_PEC | PEC value |
| 0x20 | SMBUS_AUXCTL | Auxiliary control |
| 0x80-0x9F | SMBUS_BLOCKDATA | Block data buffer |

Next: 02_architecture.md

# APB SMBus - Register Map

## Register Summary

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | SMBUS_STATUS | RO/W1C | Status |
| 0x04 | SMBUS_CONTROL | RW | Control |
| 0x08 | SMBUS_COMMAND | RW | Command byte |
| 0x0C | SMBUS_ADDRESS | RW | Slave address |
| 0x10 | SMBUS_DATA0 | RW | Data byte 0 |
| 0x14 | SMBUS_DATA1 | RW | Data byte 1 |
| 0x18 | SMBUS_BLOCK_CNT | RW | Block count |
| 0x1C | SMBUS_PEC | RO | PEC value |
| 0x20 | SMBUS_AUXCTL | RW | Auxiliary control |
| 0x80-0x9F | SMBUS_BLOCK_DATA | RW | Block buffer (32 bytes) |

## SMBUS_STATUS (0x00)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | BUSY | RO | Transaction in progress |
| 1 | INTR | W1C | Interrupt pending |
| 2 | DEV_ERR | W1C | Device error |
| 3 | BUS_ERR | W1C | Bus error |
| 4 | FAILED | W1C | Transaction failed |
| 5 | ALERT | W1C | SMBus alert received |
| 6 | TIMEOUT | W1C | Timeout occurred |
| 7 | PEC_ERR | W1C | PEC error |

## SMBUS_CONTROL (0x04)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 2:0 | CMD_TYPE | RW | Command type |
| 3 | START | RW | Start transaction |
| 4 | PEC_EN | RW | Enable PEC |
| 5 | INTREN | RW | Enable interrupt |
| 6 | KILL | RW | Abort transaction |
| 7 | RESET | RW | Soft reset |

### Command Types

| CMD_TYPE | Description |
| --- | --- |
| 000 | Quick Command |
| 001 | Send Byte |
| 010 | Receive Byte |
| 011 | Write Byte |
| 100 | Read Byte |
| 101 | Write Word |
| 110 | Read Word |
| 111 | Block Read/Write |

## SMBUS_ADDRESS (0x0C)

| Bits | Name | Description |
| --- | --- | --- |
| 6:0 | ADDR | 7-bit slave address |
| 7 | RW | 0=Write, 1=Read |

Back to: SMBus Specification Index