RTL Design Sherpa

Converters Micro-Architecture Specification 1.0

January 4, 2026

Table of Contents

1 Converter Mas Index	22

2 Document Information	22

2.1 Document Control	22

2.2 Purpose	22

2.3 Audience	22

2.4 Related Documents	23

2.5 Revision History	23

2.6 Conventions	23

2.6.1 Notation	23

2.6.2 Diagrams	23

2.6.3 Code Examples	23

3 Chapter 1: Introduction - Overview	24

3.1 1.1 Purpose	24

3.2 1.2 Problem Statement	24

3.2.1 1.2.1 Data Width Mismatch	24

3.2.2 1.2.2 Protocol Incompatibility	24

3.3 1.3 Solution Architecture	25

3.3.1 Figure 1.1: Converter Module Hierarchy	25

3.3.2 1.3.1 Data Width Converters	25

3.3.3 1.3.2 Protocol Converters	25

3.4 1.4 Key Design Decisions	26

3.4.1 1.4.1 Generic vs. Full Modules	26

3.4.2 1.4.2 Throughput vs. Area Trade-offs	26

3.4.3 1.4.3 Sideband Signal Handling	26

3.5 1.5 Performance Characteristics	27

3.5.1 1.5.1 Latency	27

3.5.2 1.5.2 Throughput	27

3.6 1.6 Scope	27

3.6.1 In Scope	27

3.6.2 Out of Scope	28

3.7 1.7 Target Applications	28

4 2.1 Generic Building Blocks	28

4.1 2.1.1 Module Hierarchy	28

4.2 2.1.2 Design Philosophy	29

4.2.1 Separation of Concerns	29

4.2.2 Interface Pattern	29

4.3 2.1.3 Width Ratio Calculation	29

4.4 2.1.4 Sideband Modes	30

4.4.1 Upsize Sideband Modes	30

4.4.2 Downsize Sideband Modes	30

4.5 2.1.5 Performance Comparison	31

4.5.1 Figure 2.1: Performance Comparison	31

4.6 2.1.6 Resource Utilization	31

4.6.1 Upsize Resources (NARROW=64, WIDE=512)	31

4.6.2 Downsize Resources (WIDE=512, NARROW=64)	32

4.7 2.1.7 Integration Guidelines	32

4.7.1 When to Use Generic Blocks	32

4.7.2 Composition Example	32

5 2.2 axi_data_upsize Module	33

5.1 2.2.1 Purpose and Function	33

5.2 2.2.2 Block Diagram	34

5.2.1 Figure 2.2: axi_data_upsize Architecture	34

5.3 2.2.3 Interface Specification	34

5.3.1 Parameters	34

5.3.2 Ports	35

5.4 2.2.4 Operation	36

5.4.1 Accumulation Cycle	36

5.4.2 Early LAST Handling	36

5.4.3 State Machine	36

5.5 2.2.5 Sideband Handling	36

5.5.1 Concatenate Mode (SB_OR_MODE=0)	36

5.5.2 OR Mode (SB_OR_MODE=1)	37

5.6 2.2.6 Implementation	37

5.6.1 Core Logic	37

5.7 2.2.7 Timing Characteristics	38

5.7.1 Latency	38

5.7.2 Throughput	39

5.7.3 Critical Paths	39

5.8 2.2.8 Resource Utilization	39

5.8.1 Typical Resources (64-bit to 512-bit)	39

5.8.2 Scaling	39

5.9 2.2.9 Usage Example	39

5.9.1 64-bit to 512-bit Write Data	39

6 2.3 axi_data_dnsize Module	40

6.1 2.3.1 Purpose and Function	40

6.2 2.3.2 Block Diagram	41

6.2.1 Figure 2.3: axi_data_dnsize Single-Buffer Architecture	41

6.2.2 Figure 2.4: axi_data_dnsize Dual-Buffer Architecture	41

6.3 2.3.3 Interface Specification	42

6.3.1 Parameters	42

6.3.2 Ports	42

6.4 2.3.4 Single-Buffer Mode Operation	43

6.4.1 Split Cycle	43

6.4.2 State Machine	43

6.4.3 Throughput Analysis	43

6.5 2.3.5 Dual-Buffer Mode Operation	44

6.5.1 Ping-Pong Operation	44

6.5.2 State Machine (Dual)	44

6.5.3 100% Throughput	44

6.6 2.3.6 Sideband Handling	44

6.6.1 Slice Mode (SB_BROADCAST=0)	44

6.6.2 Broadcast Mode (SB_BROADCAST=1)	45

6.7 2.3.7 Burst Tracking	45

6.7.1 Purpose	45

6.7.2 Operation	45

6.8 2.3.8 Implementation	46

6.8.1 Single-Buffer Core Logic	46

6.9 2.3.9 Resource Utilization	47

6.9.1 Single-Buffer (512-bit to 64-bit)	47

6.9.2 Dual-Buffer (512-bit to 64-bit)	47

6.9.3 Comparison	47

6.10 2.3.10 Usage Example	48

6.10.1 512-bit to 64-bit Read Data (High Performance)	48

7 2.4 Dual-Buffer Architecture	48

7.1 2.4.1 Problem Statement	49

7.1.1 Single-Buffer Limitation	49

7.1.2 High-Performance Requirements	49

7.2 2.4.2 Solution: Ping-Pong Buffering	49

7.2.1 Concept	49

7.2.2 Figure 2.5: Dual-Buffer Ping-Pong Operation	49

7.3 2.4.3 Implementation	49

7.3.1 Buffer State Machine	49

7.3.2 Output Arbiter	50

7.3.3 Load Controller	50

7.3.4 Buffer State Transitions	50

7.4 2.4.4 Timing Analysis	51

7.4.1 Continuous Operation	51

7.4.2 Latency	52

7.5 2.4.5 Edge Cases	52

7.5.1 Empty Pipeline Start	52

7.5.2 Pipeline Drain	52

7.5.3 Backpressure Handling	52

7.6 2.4.6 Resource Comparison	52

7.6.1 Resource Breakdown	52

7.6.2 Trade-off Summary	53

7.7 2.4.7 Configuration Guidelines	53

7.7.1 When to Use Dual Buffer	53

7.7.2 Integration Example	53

8 2.5 axi4_dwidth_converter_wr	54

8.1 2.5.1 Purpose and Function	54

8.2 2.5.2 Block Diagram	55

8.2.1 Figure 2.6: Write Converter Architecture	55

8.3 2.5.3 Interface Specification	55

8.3.1 Parameters	55

8.3.2 Ports	56

8.4 2.5.4 Burst Length Conversion	57

8.4.1 Ratio Calculation	57

8.4.2 Examples	57

8.4.3 Non-Aligned Bursts	57

8.5 2.5.5 Address Channel Handling	58

8.5.1 AW Passthrough with Adjustment	58

8.5.2 Skid Buffer for AW	58

8.6 2.5.6 Write Data Channel	58

8.6.1 Upsize Instance	58

8.7 2.5.7 Response Channel	59

8.7.1 B Channel Passthrough	59

8.7.2 Response Ordering	59

8.8 2.5.8 AW/W Synchronization	59

8.8.1 Challenge	59

8.8.2 Solution	59

8.9 2.5.9 Resource Utilization	60

8.9.1 Typical Resources (64→512, ID=4, ADDR=64)	60

8.10 2.5.10 Timing Characteristics	60

8.10.1 Latency	60

8.10.2 Throughput	60

8.11 2.5.11 Usage Example	60

9 2.6 axi4_dwidth_converter_rd	61

9.1 2.6.1 Purpose and Function	61

9.2 2.6.2 Block Diagram	62

9.2.1 Figure 2.7: Read Converter Architecture	62

9.3 2.6.3 Interface Specification	62

9.3.1 Parameters	62

9.3.2 Ports	63

9.4 2.6.4 Burst Length Conversion	64

9.4.1 Ratio Calculation	64

9.4.2 Examples	64

9.5 2.6.5 Address Channel Handling	64

9.5.1 AR Passthrough with Adjustment	64

9.5.2 AR Information FIFO	64

9.6 2.6.6 Read Data Channel	65

9.6.1 Downsize Instance	65

9.7 2.6.7 RLAST Generation	65

9.7.1 Challenge	65

9.7.2 Solution: Burst Tracker	66

9.8 2.6.8 RID Handling	66

9.8.1 ID Passthrough	66

9.9 2.6.9 Dual-Buffer Impact	66

9.9.1 Performance Comparison	66

9.9.2 When to Use Dual Buffer	67

9.10 2.6.10 Resource Utilization	67

9.10.1 Typical Resources (512→64, ID=4, Dual Buffer)	67

9.10.2 Single Buffer Version	67

9.11 2.6.11 Timing Characteristics	67

9.11.1 Latency	67

9.11.2 Throughput	67

9.12 2.6.12 Usage Example	67

10 3.1 Protocol Conversion Overview	68

10.1 3.1.1 Available Converters	69

10.1.1 AXI4 to AXI4-Lite (Protocol Downgrade)	69

10.1.2 AXI4-Lite to AXI4 (Protocol Upgrade)	69

10.1.3 Other Protocol Converters	69

10.2 3.1.2 Protocol Comparison	69

10.2.1 Figure 3.1: Protocol Feature Comparison	69

10.2.2 Feature Matrix	70

10.3 3.1.3 Conversion Strategies	70

10.3.1 AXI4 to AXI4-Lite	70

10.3.2 AXI4-Lite to AXI4	70

10.3.3 AXI4 to APB	70

10.4 3.1.4 Performance Characteristics	71

10.4.1 Key Observations	71

10.5 3.1.5 Use Case Guidelines	71

10.5.1 When to Use AXI4 to AXI4-Lite	71

10.5.2 When to Use AXI4-Lite to AXI4	71

10.5.3 When to Use AXI4 to APB	71

10.6 3.1.6 Integration Patterns	71

10.6.1 Pattern 1: CPU to Peripherals	71

10.6.2 Pattern 2: Simple IP in AXI4 Fabric	72

10.6.3 Pattern 3: Mixed System	72

11 3.2 AXI4 to AXI4-Lite Converter	72

11.1 3.2.1 Module Organization	72

11.1.1 Design Philosophy	72

11.2 3.2.2 Read Path (axi4_to_axil4_rd)	72

11.2.1 Block Diagram	72

11.2.2 Figure 3.2: AXI4 to AXI4-Lite Read Path	72

11.2.3 Operation	73

11.2.4 State Machine	73

11.2.5 Implementation	73

11.3 3.2.3 Write Path (axi4_to_axil4_wr)	74

11.3.1 Block Diagram	74

11.3.2 Figure 3.3: AXI4 to AXI4-Lite Write Path	74

11.3.3 Operation	74

11.3.4 AW/W Synchronization Challenge	75

11.3.5 Solution: Dual Accept Logic	75

11.3.6 Response Aggregation	75

11.4 3.2.4 Bidirectional Wrapper (axi4_to_axil4)	76

11.4.1 Composition Pattern	76

11.5 3.2.5 Resource Utilization	77

11.6 3.2.6 Performance Analysis	77

11.6.1 Throughput	77

11.6.2 Latency	77

11.7 3.2.7 Test Coverage	77

12 3.3 AXI4-Lite to AXI4 Converter	78

12.1 3.3.1 Module Organization	78

12.2 3.3.2 Design Philosophy	78

12.3 3.3.3 Signal Mapping	79

12.3.1 Address Channel Signals	79

12.3.2 Data Channel Signals	79

12.4 3.3.4 Read Path (axil4_to_axi4_rd)	80

12.4.1 Block Diagram	80

12.4.2 Figure 3.4: AXI4-Lite to AXI4 Read Path	80

12.4.3 Implementation	80

12.5 3.3.5 Write Path (axil4_to_axi4_wr)	81

12.5.1 Block Diagram	81

12.5.2 Figure 3.5: AXI4-Lite to AXI4 Write Path	81

12.5.3 Implementation	81

12.6 3.3.6 Bidirectional Wrapper	83

12.7 3.3.7 Resource Utilization	84

12.8 3.3.8 Performance	84

12.9 3.3.9 Test Coverage	84

12.10 3.3.10 Usage Example	84

13 3.4 AXI4 to APB Converter	85

13.1 3.4.1 Purpose	85

13.2 3.4.2 Block Diagram	87

13.2.1 Figure 3.6: AXI4 to APB Converter	87

13.3 3.4.3 Interface Specification	88

13.3.1 Parameters	88

13.3.2 Ports	88

13.4 3.4.4 State Machine	90

13.4.1 Figure 3.7: AXI4 to APB FSM	90

13.4.2 States	91

13.4.3 Transitions	91

13.5 3.4.5 Burst Handling	91

13.5.1 Burst Decomposition	91

13.5.2 Address Calculation	92

13.6 3.4.6 Address Width Adaptation	92

13.6.1 64-bit to 32-bit Conversion	92

13.7 3.4.7 Error Response Mapping	92

13.7.1 Error Aggregation	92

13.8 3.4.8 Implementation	93

13.8.1 Core FSM	93

13.9 3.4.9 Resource Utilization	94

13.10 3.4.10 Performance	94

13.10.1 Timing Analysis	94

13.10.2 Throughput	95

13.11 3.4.11 Usage Example	95

14 AXI4 to APB5 Shim	95

14.1 Design	95

14.2 Rationale	96

14.3 Consumers	96

15 3.5 PeakRDL Adapter	97

15.1 3.5.1 Purpose	97

15.2 3.5.2 Block Diagram	98

15.2.1 Figure 3.8: PeakRDL Adapter	98

15.3 3.5.3 Interface Specification	100

15.3.1 Parameters	100

15.3.2 Ports	100

15.4 3.5.4 Operation	101

15.4.1 Write Transaction	101

15.4.2 Read Transaction	101

15.5 3.5.5 Implementation	101

15.6 3.5.6 Resource Utilization	102

15.7 3.5.7 Use Cases	102

15.7.1 1. PeakRDL to Custom Control	102

15.7.2 2. Register Access Logging	103

15.7.3 3. Pipeline Insertion	103

15.8 3.5.8 Integration Example	103

16 4.1 Width Converter FSMs	104

16.1 4.1.1 Upsize FSM	104

16.1.1 Figure 4.1: Upsize FSM	105

16.1.2 States	105

16.1.3 Transitions	105

16.1.4 Implementation	106

16.2 4.1.2 Downsize FSM (Single Buffer)	106

16.2.1 Figure 4.2: Downsize Single-Buffer FSM	107

16.2.2 States	109

16.2.3 Transitions	109

16.3 4.1.3 Downsize FSM (Dual Buffer)	109

16.3.1 Buffer State Machine	109

16.3.2 Arbiter Logic	109

16.4 4.1.4 Full Converter FSMs	110

16.4.1 Write Converter (axi4_dwidth_converter_wr)	110

16.4.2 Read Converter (axi4_dwidth_converter_rd)	110

16.5 4.1.5 Timing Diagrams	111

16.5.1 Upsize Timing (8:1 ratio)	111

16.5.2 Downsize Timing (8:1 ratio, single buffer)	111

17 4.2 Protocol Converter FSMs	111

17.1 4.2.1 AXI4 to AXI4-Lite Read FSM	111

17.1.1 Figure 4.3: AXI4 to AXIL4 Read FSM	111

17.1.2 States	112

17.1.3 Transitions	112

17.1.4 Implementation	112

17.2 4.2.2 AXI4 to AXI4-Lite Write FSM	113

17.2.1 States	113

17.2.2 AW/W Synchronization	114

17.3 4.2.3 AXI4 to APB FSM	115

17.3.1 Figure 4.4: AXI4 to APB FSM	115

17.3.2 States	116

17.3.3 APB Protocol Phases	116

17.3.4 Implementation	116

17.4 4.2.4 Timing Analysis	117

17.4.1 AXI4 to AXIL4 Timing	117

17.4.2 AXI4 to APB Timing	118

18 4.3 Burst Decomposition	118

18.1 4.3.1 Width Converter Burst Handling	118

18.1.1 Burst Length Adjustment	118

18.1.2 Figure 4.5: Width Burst Conversion	118

18.1.3 Non-Aligned Bursts	119

18.2 4.3.2 Protocol Converter Burst Handling	119

18.2.1 AXI4 to AXI4-Lite Decomposition	119

18.2.2 Address Increment Calculation	119

18.3 4.3.3 Response Aggregation	120

18.3.1 Read Response Aggregation	120

18.3.2 Write Response Aggregation	120

18.4 4.3.4 Burst Tracking Registers	121

18.4.1 Required State	121

18.4.2 Initialization	121

18.5 4.3.5 Timing Impact	122

18.5.1 Decomposition Overhead	122

18.5.2 Pipeline Considerations	122

19 5.1 Test Strategy	122

19.1 5.1.1 Test Organization	122

19.1.1 Test Hierarchy	122

19.2 5.1.2 Test Levels	123

19.2.1 Level 1: Unit Tests	123

19.2.2 Level 2: Integration Tests	123

19.2.3 Level 3: System Tests	123

19.3 5.1.3 Test Categories	124

19.3.1 Functional Tests	124

19.3.2 Stress Tests	124

19.3.3 Error Tests	124

19.4 5.1.4 Testbench Architecture	125

19.4.1 Components	125

19.4.2 Driver Implementation	125

19.5 5.1.5 Coverage Model	126

19.5.1 Functional Coverage	126

19.5.2 Code Coverage	126

19.6 5.1.6 Test Execution	127

19.6.1 Running Tests	127

19.6.2 CI/CD Integration	127

20 5.2 Debug Guide	127

20.1 5.2.1 Common Issues	128

20.1.1 Width Converter Issues	128

20.1.2 Protocol Converter Issues	128

20.2 5.2.2 Debug Signals	129

20.2.1 Recommended Internal Signals	129

20.2.2 ILA Configuration	129

20.3 5.2.3 Simulation Debug	130

20.3.1 Waveform Analysis	130

20.3.2 Timing Diagram Template	130

20.4 5.2.4 Common Mistakes	131

20.4.1 Mistake 1: Wrong Width Ratio	131

20.4.2 Mistake 2: Missing Sideband Handling	131

20.4.3 Mistake 3: Incorrect LAST Timing	131

20.4.4 Mistake 4: Burst Length Calculation Error	131

20.5 5.2.5 Verification Checklist	131

20.6 5.2.6 Performance Validation	132

20.6.1 Throughput Measurement	132

20.6.2 Expected Throughput	132

List of Figures

Figure 1.1: Converter Module Hierarchy	25

Figure 2.1: Performance Comparison	31

Figure 2.2: axi_data_upsize Architecture	34

Figure 2.3: axi_data_dnsize Single-Buffer Architecture	41

Figure 2.4: axi_data_dnsize Dual-Buffer Architecture	41

Figure 2.5: Dual-Buffer Ping-Pong Operation	49

Figure 2.6: Write Converter Architecture	55

Figure 2.7: Read Converter Architecture	62

Figure 3.1: Protocol Feature Comparison	69

Figure 3.2: AXI4 to AXI4-Lite Read Path	72

Figure 3.3: AXI4 to AXI4-Lite Write Path	74

Figure 3.4: AXI4-Lite to AXI4 Read Path	80

Figure 3.5: AXI4-Lite to AXI4 Write Path	81

Figure 3.6: AXI4 to APB Converter	87

Figure 3.7: AXI4 to APB FSM	90

Figure 3.8: PeakRDL Adapter	98

Figure 4.1: Upsize FSM	105

Figure 4.2: Downsize Single-Buffer FSM	107

Figure 4.3: AXI4 to AXIL4 Read FSM	111

Figure 4.4: AXI4 to APB FSM	115

Figure 4.5: Width Burst Conversion	118

List of Tables

Table 1: Table 0.1: Document Control Information	22

Table 2: Table 0.2: Related Documents	23

Table 3: Table 0.3: Revision History	23

Table 4: Table 1.1: Common Data Width Configurations	24

Table 5: Table 1.2: Protocol Mismatch Examples	24

Table 6: Table 1.3: Throughput vs. Area Trade-offs	26

Table 7: Table 1.4: Sideband Handling Modes	27

Table 8: Table 1.5: Latency Characteristics	27

Table 9: Table 1.6: Throughput Characteristics	27

Table 10: Table 2.1: Upsize Sideband Modes	30

Table 11: Table 2.2: Downsize Sideband Modes	30

Table 12: Table 2.3: Performance Comparison	31

Table 13: Table 2.4: axi_data_upsize Parameters	34

Table 14: Table 2.5: Upsize Latency	38

Table 15: Table 2.6: Upsize Resource Scaling	39

Table 16: Table 2.7: axi_data_dnsize Parameters	42

Table 17: Table 2.8: Single-Buffer Throughput by Ratio	44

Table 18: Table 2.9: Resource Comparison	47

Table 19: Table 2.10: Dual-Buffer Cycle Timing	51

Table 20: Table 2.11: Dual-Buffer Latency	52

Table 21: Table 2.12: Resource Breakdown	52

Table 22: Table 2.13: Mode Trade-offs	53

Table 23: Table 2.14: Write Converter Parameters	55

Table 24: Table 2.15: Burst Length Conversion Examples	57

Table 25: Table 2.16: Write Converter Latency	60

Table 26: Table 2.17: Read Converter Parameters	62

Table 27: Table 2.18: Read Burst Length Conversion	64

Table 28: Table 2.19: Read Converter Performance	66

Table 29: Table 2.20: Read Converter Latency	67

Table 30: Table 3.1: AXI4 to AXI4-Lite Converters	69

Table 31: Table 3.2: AXI4-Lite to AXI4 Converters	69

Table 32: Table 3.3: Other Protocol Converters	69

Table 33: Table 3.4: Protocol Feature Comparison	70

Table 34: Table 3.5: Protocol Converter Performance	71

Table 35: Table 3.6: AXI4 to AXIL4 Resources	77

Table 36: Table 3.7: AXI4 to AXIL4 Throughput	77

Table 37: Table 3.8: AXI4 to AXIL4 Latency	77

Table 38: Table 3.9: Test Coverage Summary	77

Table 39: Table 3.10: AR Channel Mapping	79

Table 40: Table 3.11: R Channel Mapping	79

Table 41: Table 3.12: AXIL4 to AXI4 Resources	84

Table 42: Table 3.13: AXIL4 to AXI4 Performance	84

Table 43: Table 3.14: Test Coverage Summary	84

Table 44: Table 3.15: AXI4 vs APB Comparison	85

Table 45: Table 3.16: AXI4 to APB Parameters	88

Table 46: Table 3.17: FSM Transitions	91

Table 47: Table 3.18: Error Mapping	92

Table 48: Table 3.19: APB Converter Timing	94

Table 49: Table 3.20: PeakRDL Adapter Parameters	100

Table 50: Table 4.1: Upsize FSM States	105

Table 51: Table 4.2: Downsize FSM States	109

Table 52: Table 4.3: AXI4 to AXIL4 Read FSM States	112

Table 53: Table 4.4: AXI4 to AXIL4 Write FSM States	113

Table 54: Table 4.5: AXI4 to APB FSM States	116

Table 55: Table 4.6: Decomposition Overhead	122

Table 56: Table 5.1: Functional Test Categories	124

Table 57: Table 5.2: Stress Test Categories	124

Table 58: Table 5.3: Error Test Categories	124

Table 59: Table 5.4: Coverage Targets	126

Table 60: Table 5.5: Expected Throughput	132

# 1	Converter Mas Index

Generated: 2026-08-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Document Control

Table 0.1: Document Control Information

| Field | Value |
| --- | --- |
| Document Title | Converters Micro-Architecture Specification |
| Document Version | 1.0 |
| Component | Converters |
| Status | Active |
| Classification | Internal Technical |
| Last Updated | 2026-01-03 |

## 2.2	Purpose

This Micro-Architecture Specification (MAS) provides detailed implementation guidance for the Converters component. It covers:

- Internal block architectures

- State machine designs

- Signal timing and handshaking

- Resource utilization estimates

- Debug and verification strategies

## 2.3	Audience

This document is intended for:

- RTL designers implementing or modifying converter modules

- Verification engineers creating testbenches

- Integration engineers connecting converters to systems

- Performance engineers optimizing throughput and latency

## 2.4	Related Documents

Table 0.2: Related Documents

| Document | Purpose |
| --- | --- |
| Converters Spec | High-level feature specification |
| Component PRD | Product requirements and goals |
| Bridge MAS | Related crossbar micro-architecture |
| Stream MAS | Related datapath micro-architecture |

## 2.5	Revision History

Table 0.3: Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial MAS release |

## 2.6	Conventions

### 2.6.1	Notation

- signal_name - RTL signals and parameters

- ModuleName - Module names and key concepts

- Figure X.X - Figure references

### 2.6.2	Diagrams

All diagrams use Mermaid format rendered to PNG: - Source: assets/mermaid/*.mmd - Rendered: assets/mermaid/*.png

### 2.6.3	Code Examples

SystemVerilog code snippets are provided for implementation guidance. These represent the intended design pattern but may differ slightly from actual RTL.

Next: Chapter 1: Introduction

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Chapter 1: Introduction - Overview

## 3.1	1.1 Purpose

The Converters component provides essential data width conversion and protocol conversion modules that enable seamless integration between components with different data widths or communication protocols.

## 3.2	1.2 Problem Statement

Modern SoC designs frequently encounter two integration challenges:

### 3.2.1	1.2.1 Data Width Mismatch

Table 1.1: Common Data Width Configurations

| Component | Typical Data Width |
| --- | --- |
| CPU | 64-bit |
| DDR Controller | 512-bit |
| PCIe Endpoint | 128-bit |
| GPU | 256-bit |

Challenge: Direct connection between mismatched widths is impossible. Width conversion is required.

### 3.2.2	1.2.2 Protocol Incompatibility

Table 1.2: Protocol Mismatch Examples

| Master Type | Protocol | Slave Type | Protocol |
| --- | --- | --- | --- |
| CPU | AXI4 | DDR | AXI4 |
| DMA | AXI4 | UART | APB |
| CPU | AXI4 | GPIO | APB |
| Custom IP | AXI4-Lite | Fabric | AXI4 |

Challenge: Different protocols require protocol bridges for communication.

## 3.3	1.3 Solution Architecture

### 3.3.1	Figure 1.1: Converter Module Hierarchy

Converter Module Hierarchy

### 3.3.2	1.3.1 Data Width Converters

Generic Building Blocks: - axi_data_upsize - Accumulates N narrow beats into 1 wide beat - axi_data_dnsize - Splits 1 wide beat into N narrow beats

Full AXI4 Converters: - axi4_dwidth_converter_wr - Complete write path (AW + W + B channels) - axi4_dwidth_converter_rd - Complete read path (AR + R channels)

### 3.3.3	1.3.2 Protocol Converters

AXI4 to AXI4-Lite: - axi4_to_axil4_rd - Read path burst decomposition - axi4_to_axil4_wr - Write path burst decomposition - axi4_to_axil4 - Full bidirectional wrapper

AXI4-Lite to AXI4: - axil4_to_axi4_rd - Read path protocol upgrade - axil4_to_axi4_wr - Write path protocol upgrade - axil4_to_axi4 - Full bidirectional wrapper

Other Converters: - axi4_to_apb4_convert - Full AXI4-to-APB bridge - peakrdl_to_cmdrsp - Register interface adapter

## 3.4	1.4 Key Design Decisions

### 3.4.1	1.4.1 Generic vs. Full Modules

The converter architecture uses a layered approach:

Layer 1: Generic Building Blocks
         axi_data_upsize, axi_data_dnsize
         - Protocol-agnostic data manipulation
         - Reusable across different contexts

Layer 2: Full AXI4 Converters
         axi4_dwidth_converter_wr, axi4_dwidth_converter_rd
         - Compose generic blocks with AXI4 channel management
         - Handle burst length adjustment, ID tracking

Layer 3: Protocol Converters
         axi4_to_axil4, axi4_to_apb4, etc.
         - Full protocol translation
         - State machine control

### 3.4.2	1.4.2 Throughput vs. Area Trade-offs

Table 1.3: Throughput vs. Area Trade-offs

| Configuration | Throughput | Area | Use Case |
| --- | --- | --- | --- |
| Upsize (single buffer) | 100% | 1x | All narrow-to-wide |
| Downsize (single buffer) | 80% | 1x | Area-constrained |
| Downsize (dual buffer) | 100% | 2x | High-performance |

Design Decision: Single-buffer upsize is always optimal (100% throughput at minimal cost). Downsize mode is configurable based on system requirements.

### 3.4.3	1.4.3 Sideband Signal Handling

Different modes for handling sideband signals (WSTRB, RRESP, etc.):

Table 1.4: Sideband Handling Modes

| Mode | Upsize Behavior | Downsize Behavior |
| --- | --- | --- |
| Concatenate | Pack narrow strobes | Slice wide strobes |
| Broadcast | N/A | Repeat value |
| OR | Combine with OR | N/A |

## 3.5	1.5 Performance Characteristics

### 3.5.1	1.5.1 Latency

Table 1.5: Latency Characteristics

| Module | Single-Beat | Burst (N beats) |
| --- | --- | --- |
| axi_data_upsize | 0 cycles | N cycles to accumulate |
| axi_data_dnsize (single) | 1 cycle | N cycles + gap |
| axi_data_dnsize (dual) | 1 cycle | N cycles |
| axi4_to_axil4 | 0 cycles | 2xN cycles |
| axil4_to_axi4 | 0 cycles | N/A (single only) |

### 3.5.2	1.5.2 Throughput

Table 1.6: Throughput Characteristics

| Module | Configuration | Peak Throughput |
| --- | --- | --- |
| axi_data_upsize | Single buffer | 1 beat/cycle |
| axi_data_dnsize | Single buffer | 0.8 beats/cycle |
| axi_data_dnsize | Dual buffer | 1 beat/cycle |
| axi4_to_axil4 | Burst | 0.5 beats/cycle |
| axil4_to_axi4 | Any | 1 beat/cycle |

## 3.6	1.6 Scope

### 3.6.1	In Scope

- Integer width ratios (2:1, 4:1, 8:1, 16:1, etc.)

- AXI4, AXI4-Lite, and APB protocol support

- Configurable throughput vs. area trade-offs

- Generic building blocks for custom converters

- Burst-aware width conversion

### 3.6.2	Out of Scope

- Non-integer width ratios (e.g., 3:2 conversion)

- AXI4-Stream protocol (see Stream component)

- Complex buffering beyond dual-buffer

- Clock domain crossing (use separate CDC modules)

- Address translation (handled by crossbar)

## 3.7	1.7 Target Applications

- CPU-to-DDR Integration - 64-bit CPU to 512-bit memory controller

- DMA Engines - Variable width data movers

- Mixed Protocol Systems - AXI4 fabric with APB peripheral bus

- FPGA Fabric Interfaces - Width matching for IP integration

- Register Access - PeakRDL to custom control protocols

Next: Chapter 2: Data Width Converter Blocks

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	2.1 Generic Building Blocks

The Converters component provides two generic building blocks for data width conversion: axi_data_upsize (narrow-to-wide) and axi_data_dnsize (wide-to-narrow). These modules are protocol-agnostic and can be composed into full AXI4 converters or used directly in custom designs.

## 4.1	2.1.1 Module Hierarchy

Generic Building Blocks
|
+-- axi_data_upsize.sv      # Narrow-to-Wide accumulator
|   +-- Accumulator buffer
|   +-- Beat counter
|   +-- Sideband packing
|
+-- axi_data_dnsize.sv      # Wide-to-Narrow splitter
    +-- Data buffer (single or dual)
    +-- Beat counter
    +-- Sideband extraction
    +-- Optional burst tracking

## 4.2	2.1.2 Design Philosophy

### 4.2.1	Separation of Concerns

The generic blocks handle only data manipulation: - Data packing/unpacking - Sideband signal handling - Valid/ready handshaking

They do not handle: - Address manipulation - Burst length adjustment - ID tracking - Protocol-specific signals (ARLEN, AWLEN, etc.)

This separation enables: - Reuse in multiple contexts - Simpler verification - Cleaner composition into full converters

### 4.2.2	Interface Pattern

Both modules use a consistent valid/ready interface:

// Input (narrow for upsize, wide for dnsize)
input  logic                    i_valid,
output logic                    o_ready,
input  logic [DATA_WIDTH-1:0]   i_data,
input  logic [SB_WIDTH-1:0]     i_sideband,
input  logic                    i_last,      // Optional

// Output (wide for upsize, narrow for dnsize)
output logic                    o_valid,
input  logic                    i_ready,
output logic [DATA_WIDTH-1:0]   o_data,
output logic [SB_WIDTH-1:0]     o_sideband,
output logic                    o_last       // Optional

## 4.3	2.1.3 Width Ratio Calculation

Both modules calculate the width ratio at elaboration time:

localparam int RATIO = WIDE_WIDTH / NARROW_WIDTH;
localparam int RATIO_LOG2 = $clog2(RATIO);

// Example: 64-bit to 512-bit
// RATIO = 512 / 64 = 8
// RATIO_LOG2 = 3

Constraints: - WIDE_WIDTH must be an integer multiple of NARROW_WIDTH - Minimum ratio: 2 - Maximum ratio: Typically 16 (limited by timing)

## 4.4	2.1.4 Sideband Modes

### 4.4.1	Upsize Sideband Modes

Table 2.1: Upsize Sideband Modes

| Mode | Parameter | Behavior | Use Case |
| --- | --- | --- | --- |
| Concatenate | SB_OR_MODE=0 | Pack N narrow sidebands | WSTRB packing |
| OR | SB_OR_MODE=1 | OR all narrow sidebands | Error aggregation |

Concatenate Example (WSTRB):

8 beats of 8-bit WSTRB → 1 beat of 64-bit WSTRB
[0]: 0xFF → output[7:0]   = 0xFF
[1]: 0xF0 → output[15:8]  = 0xF0
...
[7]: 0x0F → output[63:56] = 0x0F

OR Example (Error flags):

8 beats of error flags → 1 beat with any error
[0]: 0 → output = 0
[1]: 1 → output = 1 (error detected)
...
[7]: 0 → output remains 1

### 4.4.2	Downsize Sideband Modes

Table 2.2: Downsize Sideband Modes

| Mode | Parameter | Behavior | Use Case |
| --- | --- | --- | --- |
| Slice | SB_BROADCAST=0 | Extract slice per beat | WSTRB extraction |
| Broadcast | SB_BROADCAST=1 | Repeat full value | RRESP broadcast |

Slice Example (WSTRB):

1 beat of 64-bit WSTRB → 8 beats of 8-bit WSTRB
input = 0xFF_F0_..._0F
[0]: output = 0xFF (bits [7:0])
[1]: output = 0xF0 (bits [15:8])
...
[7]: output = 0x0F (bits [63:56])

Broadcast Example (RRESP):

1 beat of 2-bit RRESP → 8 beats of 2-bit RRESP
input = OKAY (2'b00)
[0-7]: output = OKAY (all beats get same response)

## 4.5	2.1.5 Performance Comparison

### 4.5.1	Figure 2.1: Performance Comparison

Performance Comparison

Performance Comparison

Table 2.3: Performance Comparison

| Module | Mode | Throughput | Latency | Area |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single | 100% | N cycles | 1x |
| axi_data_dnsize | Single | 80% | 1 cycle | 1x |
| axi_data_dnsize | Dual | 100% | 1 cycle | 2x |

Why 80% for single-buffer downsize? - Single buffer requires one cycle gap between wide beats - Wide beat loaded → N narrow beats output → gap → next wide beat - Gap cycle = 1/(N+1) throughput loss - For large N, approaches 100% but never reaches it

Why 100% for dual-buffer downsize? - Ping-pong between two buffers - While one buffer outputs, other loads - No gap cycles required

## 4.6	2.1.6 Resource Utilization

### 4.6.1	Upsize Resources (NARROW=64, WIDE=512)

Accumulator buffer:   512 bits (output register)
Beat counter:         3 bits (clog2(8))
Sideband accumulator: 64 bits (WSTRB)
Control logic:        ~50 LEs

Total: ~70-100 LEs, ~580 registers

### 4.6.2	Downsize Resources (WIDE=512, NARROW=64)

Single Buffer:

Data buffer:          512 bits
Beat counter:         3 bits
Sideband logic:       ~20 LEs
Control logic:        ~50 LEs

Total: ~70-100 LEs, ~520 registers

Dual Buffer:

Data buffers (2x):    1024 bits
Beat counters (2x):   6 bits
Sideband logic:       ~40 LEs
Control logic:        ~100 LEs (ping-pong FSM)

Total: ~140-180 LEs, ~1040 registers

## 4.7	2.1.7 Integration Guidelines

### 4.7.1	When to Use Generic Blocks

Use directly when: - Building custom data pipelines - Data width conversion without AXI4 protocol - Simple valid/ready streaming interfaces

Use full converters when: - Need AXI4 channel management (AW, W, B, AR, R) - Burst length adjustment required - ID tracking needed

### 4.7.2	Composition Example

// Full AXI4 write path composition
axi4_dwidth_converter_wr #(.S_DATA_WIDTH(64), .M_DATA_WIDTH(512)) u_wr (
    // This module internally instantiates:
    // 1. Address phase skid buffer
    // 2. axi_data_upsize for write data
    // 3. Response path passthrough
    // 4. Burst length adjustment logic
);

Next: axi_data_upsize Module

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	2.2 axi_data_upsize Module

The axi_data_upsize module accumulates N narrow beats into 1 wide beat. It is the core building block for narrow-to-wide data width conversion.

## 5.1	2.2.1 Purpose and Function

The upsize module serves several critical functions:

- Data Accumulation: Collects N narrow data beats into accumulator buffer

- Sideband Packing: Concatenates or ORs sideband signals (WSTRB, etc.)

- Flow Control: Manages valid/ready handshaking with single-cycle latency

- LAST Tracking: Detects input LAST to generate output LAST

## 5.2	2.2.2 Block Diagram

### 5.2.1	Figure 2.2: axi_data_upsize Architecture

axi_data_upsize Architecture

## 5.3	2.2.3 Interface Specification

### 5.3.1	Parameters

Table 2.4: axi_data_upsize Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NARROW_WIDTH | int | 64 | Input data width (bits) |
| WIDE_WIDTH | int | 512 | Output data width (bits) |
| NARROW_SB_WIDTH | int | 8 | Input sideband width (bits) |
| WIDE_SB_WIDTH | int | 64 | Output sideband width |
| SB_OR_MODE | bit | 0 | 0=concatenate, 1=OR sidebands |
| USE_LAST | bit | 1 | Enable LAST signal tracking |

### 5.3.2	Ports

module axi_data_upsize #(
    parameter int NARROW_WIDTH    = 64,
    parameter int WIDE_WIDTH      = 512,
    parameter int NARROW_SB_WIDTH = 8,
    parameter int WIDE_SB_WIDTH   = 64,
    parameter bit SB_OR_MODE      = 0,
    parameter bit USE_LAST        = 1
) (
    input  logic                        clk,
    input  logic                        rst_n,

    // Narrow input interface
    input  logic                        s_valid,
    output logic                        s_ready,
    input  logic [NARROW_WIDTH-1:0]     s_data,
    input  logic [NARROW_SB_WIDTH-1:0]  s_sideband,
    input  logic                        s_last,

    // Wide output interface
    output logic                        m_valid,
    input  logic                        m_ready,
    output logic [WIDE_WIDTH-1:0]       m_data,
    output logic [WIDE_SB_WIDTH-1:0]    m_sideband,
    output logic                        m_last
);

## 5.4	2.2.4 Operation

### 5.4.1	Accumulation Cycle

Cycle 0: s_data[0] → buffer[63:0],   count = 0 → 1
Cycle 1: s_data[1] → buffer[127:64], count = 1 → 2
...
Cycle 7: s_data[7] → buffer[511:448], count = 7 → 0, m_valid = 1
Cycle 8: m_ready handshake, output complete

### 5.4.2	Early LAST Handling

If s_last arrives before buffer is full:

Cycle 0: s_data[0] → buffer[63:0],   count = 0 → 1
Cycle 1: s_data[1] + s_last → buffer[127:64], count = 1 → 0
         m_valid = 1, m_last = 1
         Remaining bytes = don't care (masked by WSTRB)

### 5.4.3	State Machine

IDLE (count=0):
  - s_valid=1 → load beat, increment count
  - count < RATIO-1 → stay in IDLE
  - count = RATIO-1 OR s_last → OUTPUT

OUTPUT (m_valid=1):
  - m_ready=1 → clear buffer, → IDLE
  - m_ready=0 → hold output

## 5.5	2.2.5 Sideband Handling

### 5.5.1	Concatenate Mode (SB_OR_MODE=0)

Used for WSTRB packing:

// Pack narrow sidebands into wide sideband
always_ff @(posedge clk) begin
    if (s_valid && s_ready) begin
        r_sideband[r_count * NARROW_SB_WIDTH +: NARROW_SB_WIDTH] <= s_sideband;
    end
end

Example: 8 beats of 8-bit WSTRB to 64-bit WSTRB

Beat 0: WSTRB = 0xFF → output[7:0]   = 0xFF
Beat 1: WSTRB = 0xF0 → output[15:8]  = 0xF0
Beat 2: WSTRB = 0x0F → output[23:16] = 0x0F
...
Beat 7: WSTRB = 0xAA → output[63:56] = 0xAA
Final:  output = 0xAA_..._0F_F0_FF

### 5.5.2	OR Mode (SB_OR_MODE=1)

Used for error flag aggregation:

// OR narrow sidebands together
always_ff @(posedge clk) begin
    if (s_valid && s_ready) begin
        if (r_count == 0)
            r_sideband <= {{(WIDE_SB_WIDTH-NARROW_SB_WIDTH){1'b0}}, s_sideband};
        else
            r_sideband <= r_sideband | s_sideband;
    end
end

Example: Any error in burst propagates

Beat 0: error = 0 → output = 0
Beat 1: error = 0 → output = 0
Beat 2: error = 1 → output = 1 (error detected)
Beat 3: error = 0 → output = 1 (remains set)
...
Final:  output = 1 (error occurred in burst)

## 5.6	2.2.6 Implementation

### 5.6.1	Core Logic

// Beat counter
logic [$clog2(RATIO)-1:0] r_count;

// Accumulator buffer
logic [WIDE_WIDTH-1:0] r_data;
logic [WIDE_SB_WIDTH-1:0] r_sideband;
logic r_last;

// Output valid when buffer full or early LAST
logic w_output_valid;
assign w_output_valid = (r_count == RATIO - 1) || r_last;

// Ready when not outputting or downstream ready
assign s_ready = !w_output_valid || m_ready;

// Main accumulation logic
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_count <= '0;
        r_data <= '0;
        r_sideband <= '0;
        r_last <= 1'b0;
    end else if (s_valid && s_ready) begin
        // Pack data into buffer
        r_data[r_count * NARROW_WIDTH +: NARROW_WIDTH] <= s_data;

        // Handle sideband based on mode
        if (SB_OR_MODE)
            r_sideband <= (r_count == 0) ? s_sideband : (r_sideband | s_sideband);
        else
            r_sideband[r_count * NARROW_SB_WIDTH +: NARROW_SB_WIDTH] <= s_sideband;

        // Track LAST
        r_last <= s_last;

        // Update counter
        if (s_last || r_count == RATIO - 1)
            r_count <= '0;
        else
            r_count <= r_count + 1'b1;
    end else if (m_valid && m_ready) begin
        r_last <= 1'b0;
    end
end

// Output assignments
assign m_valid = w_output_valid;
assign m_data = r_data;
assign m_sideband = r_sideband;
assign m_last = r_last;

## 5.7	2.2.7 Timing Characteristics

### 5.7.1	Latency

Table 2.5: Upsize Latency

| Scenario | Latency |
| --- | --- |
| Full buffer (N beats) | N cycles |
| Early LAST (M beats) | M cycles |
| Output handshake | 0-1 cycles |

### 5.7.2	Throughput

100% throughput - No gaps required between input beats.

The accumulator accepts one beat per cycle continuously. When the output buffer is ready, it completes the handshake and immediately starts accumulating the next wide beat.

### 5.7.3	Critical Paths

Typical critical paths: - s_data → accumulator buffer → m_data - r_count → comparison → s_ready

Timing closure: The module is designed for single-cycle operation with combinatorial paths only within registered stages.

## 5.8	2.2.8 Resource Utilization

### 5.8.1	Typical Resources (64-bit to 512-bit)

Accumulator buffer:   512 flip-flops
Sideband buffer:      64 flip-flops (WSTRB)
Beat counter:         3 flip-flops
Control logic:        ~20 flip-flops
                      ~50-70 LUTs

Total: ~600 flip-flops, ~50-70 LUTs

### 5.8.2	Scaling

Table 2.6: Upsize Resource Scaling

| Configuration | Registers | LUTs |
| --- | --- | --- |
| 32 → 128 (4:1) | ~170 | ~30 |
| 64 → 256 (4:1) | ~330 | ~40 |
| 64 → 512 (8:1) | ~600 | ~60 |
| 128 → 1024 (8:1) | ~1150 | ~80 |

## 5.9	2.2.9 Usage Example

### 5.9.1	64-bit to 512-bit Write Data

axi_data_upsize #(
    .NARROW_WIDTH(64),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(8),    // WSTRB
    .WIDE_SB_WIDTH(64),
    .SB_OR_MODE(0),         // Concatenate WSTRB
    .USE_LAST(1)
) u_wdata_upsize (
    .clk        (aclk),
    .rst_n      (aresetn),
    .s_valid    (s_wvalid),
    .s_ready    (s_wready),
    .s_data     (s_wdata),
    .s_sideband (s_wstrb),
    .s_last     (s_wlast),
    .m_valid    (m_wvalid),
    .m_ready    (m_wready),
    .m_data     (m_wdata),
    .m_sideband (m_wstrb),
    .m_last     (m_wlast)
);

Next: axi_data_dnsize Module

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	2.3 axi_data_dnsize Module

The axi_data_dnsize module splits 1 wide beat into N narrow beats. It supports both single-buffer (80% throughput) and dual-buffer (100% throughput) modes.

## 6.1	2.3.1 Purpose and Function

The downsize module serves several critical functions:

- Data Splitting: Extracts N narrow beats from one wide beat

- Sideband Extraction: Slices or broadcasts sideband signals

- Dual-Buffer Mode: Optional ping-pong buffering for 100% throughput

- Burst Tracking: Optional LAST signal generation based on burst length

## 6.2	2.3.2 Block Diagram

### 6.2.1	Figure 2.3: axi_data_dnsize Single-Buffer Architecture

axi_data_dnsize Single Buffer

### 6.2.2	Figure 2.4: axi_data_dnsize Dual-Buffer Architecture

axi_data_dnsize Dual Buffer

## 6.3	2.3.3 Interface Specification

### 6.3.1	Parameters

Table 2.7: axi_data_dnsize Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| WIDE_WIDTH | int | 512 | Input data width (bits) |
| NARROW_WIDTH | int | 64 | Output data width (bits) |
| WIDE_SB_WIDTH | int | 2 | Input sideband width |
| NARROW_SB_WIDTH | int | 2 | Output sideband width |
| SB_BROADCAST | bit | 1 | 0=slice, 1=broadcast sidebands |
| DUAL_BUFFER | bit | 0 | 0=single, 1=dual buffer mode |
| USE_BURST_TRACKER | bit | 0 | Enable burst-aware LAST generation |
| BURST_LEN_WIDTH | int | 8 | Width of burst length input |

### 6.3.2	Ports

module axi_data_dnsize #(
    parameter int WIDE_WIDTH       = 512,
    parameter int NARROW_WIDTH     = 64,
    parameter int WIDE_SB_WIDTH    = 2,
    parameter int NARROW_SB_WIDTH  = 2,
    parameter bit SB_BROADCAST     = 1,
    parameter bit DUAL_BUFFER      = 0,
    parameter bit USE_BURST_TRACKER = 0,
    parameter int BURST_LEN_WIDTH  = 8
) (
    input  logic                       clk,
    input  logic                       rst_n,

    // Wide input interface
    input  logic                       s_valid,
    output logic                       s_ready,
    input  logic [WIDE_WIDTH-1:0]      s_data,
    input  logic [WIDE_SB_WIDTH-1:0]   s_sideband,
    input  logic                       s_last,

    // Burst length input (optional)
    input  logic [BURST_LEN_WIDTH-1:0] burst_len,

    // Narrow output interface
    output logic                       m_valid,
    input  logic                       m_ready,
    output logic [NARROW_WIDTH-1:0]    m_data,
    output logic [NARROW_SB_WIDTH-1:0] m_sideband,
    output logic                       m_last
);

## 6.4	2.3.4 Single-Buffer Mode Operation

### 6.4.1	Split Cycle

Cycle 0: s_data loaded → buffer, s_ready = 0
Cycle 1: m_data = buffer[63:0],   count = 0, m_valid = 1
Cycle 2: m_data = buffer[127:64], count = 1
...
Cycle 8: m_data = buffer[511:448], count = 7, m_last possible
Cycle 9: s_ready = 1, gap cycle (80% throughput loss)

### 6.4.2	State Machine

IDLE:
  - s_valid=1 → load buffer → SPLITTING

SPLITTING:
  - Output beats 0 to RATIO-1
  - m_ready=1 → increment count
  - count=RATIO-1 AND m_ready → IDLE

### 6.4.3	Throughput Analysis

Why 80%?

For ratio N, the cycle utilization is: - N cycles outputting narrow beats - 1 cycle loading next wide beat

Throughput = N / (N + 1)

Table 2.8: Single-Buffer Throughput by Ratio

| Ratio | Cycles Active | Cycles Total | Throughput |
| --- | --- | --- | --- |
| 2:1 | 2 | 3 | 66.7% |
| 4:1 | 4 | 5 | 80.0% |
| 8:1 | 8 | 9 | 88.9% |
| 16:1 | 16 | 17 | 94.1% |

## 6.5	2.3.5 Dual-Buffer Mode Operation

### 6.5.1	Ping-Pong Operation

Buffer A          Buffer B          Output
--------          --------          ------
Load beat 0       (empty)           (idle)
Outputting 0      Load beat 1       beat 0[0]
Outputting 0      Outputting 1      beat 0[1]
...               ...               ...
Outputting 0      Outputting 1      beat 0[N-1]
Load beat 2       Outputting 1      beat 1[0]
Outputting 2      Outputting 1      beat 1[1]
...

### 6.5.2	State Machine (Dual)

Buffer A State: LOADING | OUTPUTTING | DONE
Buffer B State: LOADING | OUTPUTTING | DONE

Arbiter selects active output buffer
When output complete, swap buffers

### 6.5.3	100% Throughput

Dual-buffer achieves 100% throughput because: - While buffer A outputs, buffer B loads - While buffer B outputs, buffer A loads - No gap cycles required

Trade-off: 2x register resources

## 6.6	2.3.6 Sideband Handling

### 6.6.1	Slice Mode (SB_BROADCAST=0)

Used for WSTRB extraction:

// Extract sideband slice per beat
assign m_sideband = r_sideband[r_count * NARROW_SB_WIDTH +: NARROW_SB_WIDTH];

Example: 64-bit WSTRB to 8-bit WSTRB

Input: 0xAA_BB_CC_DD_EE_FF_00_11

Beat 0: output WSTRB = 0x11 (bits [7:0])
Beat 1: output WSTRB = 0x00 (bits [15:8])
Beat 2: output WSTRB = 0xFF (bits [23:16])
...
Beat 7: output WSTRB = 0xAA (bits [63:56])

### 6.6.2	Broadcast Mode (SB_BROADCAST=1)

Used for RRESP:

// Broadcast same sideband to all beats
assign m_sideband = r_sideband[NARROW_SB_WIDTH-1:0];

Example: RRESP = OKAY for all beats

Input: RRESP = 2'b00 (OKAY)

Beat 0: output RRESP = 2'b00
Beat 1: output RRESP = 2'b00
...
Beat 7: output RRESP = 2'b00

## 6.7	2.3.7 Burst Tracking

### 6.7.1	Purpose

When USE_BURST_TRACKER=1, the module generates correct m_last based on AXI4 burst length instead of relying on input s_last.

### 6.7.2	Operation

// Track narrow beats across burst
logic [BURST_LEN_WIDTH+RATIO_LOG2-1:0] r_burst_beats_remaining;

// Initialize on first beat
if (first_beat)
    r_burst_beats_remaining <= (burst_len + 1) * RATIO - 1;

// Decrement on each output
if (m_valid && m_ready)
    r_burst_beats_remaining <= r_burst_beats_remaining - 1;

// Generate LAST
assign m_last = (r_burst_beats_remaining == 0);

Example: ARLEN=3 (4 beats), ratio 8:1

Total narrow beats = 4 * 8 = 32
Beat 0-30: m_last = 0
Beat 31: m_last = 1

## 6.8	2.3.8 Implementation

### 6.8.1	Single-Buffer Core Logic

// Beat counter
logic [$clog2(RATIO)-1:0] r_count;
logic r_active;

// Data buffer
logic [WIDE_WIDTH-1:0] r_data;
logic [WIDE_SB_WIDTH-1:0] r_sideband;
logic r_last_wide;

// Load/output control
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_active <= 1'b0;
        r_count <= '0;
    end else begin
        if (!r_active && s_valid) begin
            // Load new wide beat
            r_data <= s_data;
            r_sideband <= s_sideband;
            r_last_wide <= s_last;
            r_active <= 1'b1;
            r_count <= '0;
        end else if (r_active && m_ready) begin
            if (r_count == RATIO - 1) begin
                r_active <= 1'b0;  // Done with this beat
            end else begin
                r_count <= r_count + 1'b1;
            end
        end
    end
end

// Output data slice
assign m_data = r_data[r_count * NARROW_WIDTH +: NARROW_WIDTH];

// Sideband (slice or broadcast)
assign m_sideband = SB_BROADCAST ?
    r_sideband[NARROW_SB_WIDTH-1:0] :
    r_sideband[r_count * NARROW_SB_WIDTH +: NARROW_SB_WIDTH];

// Control signals
assign m_valid = r_active;
assign s_ready = !r_active;
assign m_last = r_last_wide && (r_count == RATIO - 1);

## 6.9	2.3.9 Resource Utilization

### 6.9.1	Single-Buffer (512-bit to 64-bit)

Data buffer:          512 flip-flops
Sideband buffer:      64 flip-flops
Beat counter:         3 flip-flops
Control logic:        ~10 flip-flops
                      ~30-50 LUTs

Total: ~590 flip-flops, ~30-50 LUTs

### 6.9.2	Dual-Buffer (512-bit to 64-bit)

Data buffers (2x):    1024 flip-flops
Sideband buffers:     128 flip-flops
Beat counters (2x):   6 flip-flops
Control logic:        ~30 flip-flops
Ping-pong FSM:        ~50 LUTs

Total: ~1190 flip-flops, ~80-100 LUTs

### 6.9.3	Comparison

Table 2.9: Resource Comparison

| Mode | Registers | LUTs | Throughput |
| --- | --- | --- | --- |
| Single | 590 | 40 | 80% |
| Dual | 1190 | 90 | 100% |

Decision Guide: - Area-constrained: Use single buffer, accept 80% throughput - Performance-critical: Use dual buffer, accept 2x resources

## 6.10	2.3.10 Usage Example

### 6.10.1	512-bit to 64-bit Read Data (High Performance)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(64),
    .WIDE_SB_WIDTH(2),       // RRESP
    .NARROW_SB_WIDTH(2),
    .SB_BROADCAST(1),        // Broadcast RRESP
    .DUAL_BUFFER(1),         // 100% throughput
    .USE_BURST_TRACKER(1),   // Generate RLAST
    .BURST_LEN_WIDTH(8)
) u_rdata_dnsize (
    .clk        (aclk),
    .rst_n      (aresetn),
    .s_valid    (s_rvalid),
    .s_ready    (s_rready),
    .s_data     (s_rdata),
    .s_sideband (s_rresp),
    .s_last     (s_rlast),
    .burst_len  (ar_len),
    .m_valid    (m_rvalid),
    .m_ready    (m_rready),
    .m_data     (m_rdata),
    .m_sideband (m_rresp),
    .m_last     (m_rlast)
);

Next: Dual-Buffer Architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	2.4 Dual-Buffer Architecture

The dual-buffer mode for axi_data_dnsize achieves 100% throughput by using ping-pong buffering to eliminate the gap cycle between wide beat loads.

## 7.1	2.4.1 Problem Statement

### 7.1.1	Single-Buffer Limitation

In single-buffer mode, there is an unavoidable gap cycle between completing output of one wide beat and starting output of the next:

Cycle N:   Output last narrow beat of wide beat A
Cycle N+1: Load wide beat B (gap - no output)
Cycle N+2: Output first narrow beat of wide beat B

This gap cycle reduces throughput to N/(N+1), or approximately 80-90% depending on the width ratio.

### 7.1.2	High-Performance Requirements

Some applications require continuous 100% throughput: - DDR memory controllers with sustained bandwidth - DMA engines with continuous data flows - Real-time video/audio processing

## 7.2	2.4.2 Solution: Ping-Pong Buffering

### 7.2.1	Concept

Use two buffers that alternate between loading and outputting:

Time:     0   1   2   3   4   5   6   7   8   9  10  11
Buffer A: LD  O0  O1  O2  O3  O4  O5  O6  O7  LD  O0  O1
Buffer B:     LD  --  --  --  --  --  --  --  LD  --  --
Output:   --  A0  A1  A2  A3  A4  A5  A6  A7  B0  B1  B2

LD = Loading, On = Outputting beat n

While buffer A outputs its 8 narrow beats, buffer B loads the next wide beat. When buffer A completes, buffer B immediately starts outputting while buffer A loads.

### 7.2.2	Figure 2.5: Dual-Buffer Ping-Pong Operation

Dual-Buffer Operation

Dual-Buffer Operation

## 7.3	2.4.3 Implementation

### 7.3.1	Buffer State Machine

Each buffer has three states:

typedef enum logic [1:0] {
    BUF_IDLE      = 2'b00,  // Empty, ready to load
    BUF_LOADED    = 2'b01,  // Full, waiting to output
    BUF_OUTPUTTING = 2'b10  // Active output
} buf_state_t;

buf_state_t r_buf_a_state, r_buf_b_state;

### 7.3.2	Output Arbiter

// Select which buffer outputs
logic w_output_sel;  // 0 = buffer A, 1 = buffer B

always_comb begin
    if (r_buf_a_state == BUF_OUTPUTTING)
        w_output_sel = 1'b0;
    else if (r_buf_b_state == BUF_OUTPUTTING)
        w_output_sel = 1'b1;
    else if (r_buf_a_state == BUF_LOADED)
        w_output_sel = 1'b0;  // A ready first
    else if (r_buf_b_state == BUF_LOADED)
        w_output_sel = 1'b1;  // B ready
    else
        w_output_sel = 1'b0;  // Default
end

### 7.3.3	Load Controller

// Determine which buffer can accept new data
logic w_load_sel;  // 0 = buffer A, 1 = buffer B

always_comb begin
    if (r_buf_a_state == BUF_IDLE)
        w_load_sel = 1'b0;
    else if (r_buf_b_state == BUF_IDLE)
        w_load_sel = 1'b1;
    else
        w_load_sel = 1'b0;  // No buffer available
end

assign s_ready = (r_buf_a_state == BUF_IDLE) || (r_buf_b_state == BUF_IDLE);

### 7.3.4	Buffer State Transitions

// Buffer A state machine
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_buf_a_state <= BUF_IDLE;
    end else begin
        case (r_buf_a_state)
            BUF_IDLE: begin
                if (s_valid && s_ready && !w_load_sel) begin
                    r_buf_a_data <= s_data;
                    r_buf_a_state <= BUF_LOADED;
                end
            end

            BUF_LOADED: begin
                if (!w_output_sel || r_buf_b_state != BUF_OUTPUTTING)
                    r_buf_a_state <= BUF_OUTPUTTING;
            end

            BUF_OUTPUTTING: begin
                if (m_ready && r_buf_a_count == RATIO - 1)
                    r_buf_a_state <= BUF_IDLE;
            end
        endcase
    end
end

// Buffer B follows same pattern with opposite selection

## 7.4	2.4.4 Timing Analysis

### 7.4.1	Continuous Operation

With dual buffering, the output is continuous:

Table 2.10: Dual-Buffer Cycle Timing

| Cycle | Buffer A | Buffer B | Output |
| --- | --- | --- | --- |
| 0 | Load W0 | Idle | - |
| 1 | Output W0[0] | Load W1 | W0[0] |
| 2 | Output W0[1] | Wait | W0[1] |
| … | … | … | … |
| 8 | Output W0[7] | Wait | W0[7] |
| 9 | Load W2 | Output W1[0] | W1[0] |
| 10 | Wait | Output W1[1] | W1[1] |

### 7.4.2	Latency

Table 2.11: Dual-Buffer Latency

| Metric | Value |
| --- | --- |
| First beat latency | 1 cycle (load) |
| Sustained throughput | 100% |
| Buffer switch latency | 0 cycles |

## 7.5	2.4.5 Edge Cases

### 7.5.1	Empty Pipeline Start

When starting from empty state: 1. First wide beat loads into buffer A 2. Buffer A starts outputting 3. Buffer B loads next wide beat 4. Continuous operation begins

### 7.5.2	Pipeline Drain

When input stops (last beat processed): 1. Currently outputting buffer completes 2. Other buffer (if loaded) takes over 3. Pipeline drains to empty

### 7.5.3	Backpressure Handling

If m_ready goes low: 1. Output stalls on current beat 2. Loading continues if buffer available 3. If both buffers full, s_ready goes low 4. Resumes when m_ready returns high

## 7.6	2.4.6 Resource Comparison

### 7.6.1	Resource Breakdown

Table 2.12: Resource Breakdown

| Component | Single Buffer | Dual Buffer |
| --- | --- | --- |
| Data registers | 512 bits | 1024 bits |
| Sideband registers | 64 bits | 128 bits |
| Beat counters | 3 bits | 6 bits |
| State machines | 1 | 2 + arbiter |
| Control logic | ~40 LEs | ~90 LEs |

### 7.6.2	Trade-off Summary

Table 2.13: Mode Trade-offs

| Mode | Area | Throughput | Use Case |
| --- | --- | --- | --- |
| Single | 1x | 80-90% | Area-constrained, bursty traffic |
| Dual | 2x | 100% | Performance-critical, sustained traffic |

## 7.7	2.4.7 Configuration Guidelines

### 7.7.1	When to Use Dual Buffer

Recommended: - DDR/HBM memory interfaces - DMA engines with sustained transfers - Video/audio streaming paths - Any path where 10-20% throughput loss is unacceptable

Not Recommended: - Area-constrained designs - Bursty traffic with gaps - Control paths (low bandwidth) - When latency is more critical than throughput

### 7.7.2	Integration Example

// High-performance read path for DDR controller
axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(64),
    .DUAL_BUFFER(1),          // Enable dual buffer
    .USE_BURST_TRACKER(1),
    .SB_BROADCAST(1)
) u_rdata_dnsize (
    // ... connections
);

// Low-bandwidth control path (save area)
axi_data_dnsize #(
    .WIDE_WIDTH(128),
    .NARROW_WIDTH(32),
    .DUAL_BUFFER(0),          // Single buffer OK
    .USE_BURST_TRACKER(0),
    .SB_BROADCAST(1)
) u_ctrl_dnsize (
    // ... connections
);

Next: axi4_dwidth_converter_wr

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	2.5 axi4_dwidth_converter_wr

The axi4_dwidth_converter_wr module provides complete AXI4 write path conversion, handling AW, W, and B channels with burst length adjustment and proper protocol compliance.

## 8.1	2.5.1 Purpose and Function

The write converter combines the generic axi_data_upsize with AXI4 protocol handling:

- Address Channel (AW): Passes through with burst length adjustment

- Write Data Channel (W): Uses axi_data_upsize for data packing

- Response Channel (B): Passes through unchanged

- Burst Length Adjustment: Converts AWLEN based on width ratio

## 8.2	2.5.2 Block Diagram

### 8.2.1	Figure 2.6: Write Converter Architecture

Write Converter Architecture

## 8.3	2.5.3 Interface Specification

### 8.3.1	Parameters

Table 2.14: Write Converter Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| S_DATA_WIDTH | int | 64 | Slave-side (narrow) data width |
| M_DATA_WIDTH | int | 512 | Master-side (wide) data width |
| ADDR_WIDTH | int | 64 | Address width |
| ID_WIDTH | int | 4 | Transaction ID width |
| SKID_DEPTH | int | 2 | Pipeline buffer depth |

### 8.3.2	Ports

module axi4_dwidth_converter_wr #(
    parameter int S_DATA_WIDTH = 64,
    parameter int M_DATA_WIDTH = 512,
    parameter int ADDR_WIDTH   = 64,
    parameter int ID_WIDTH     = 4,
    parameter int SKID_DEPTH   = 2
) (
    input  logic clk,
    input  logic rst_n,

    // Slave interface (narrow, from master)
    input  logic                      s_awvalid,
    output logic                      s_awready,
    input  logic [ADDR_WIDTH-1:0]     s_awaddr,
    input  logic [7:0]                s_awlen,
    input  logic [2:0]                s_awsize,
    input  logic [1:0]                s_awburst,
    input  logic [ID_WIDTH-1:0]       s_awid,
    // ... other AW signals

    input  logic                      s_wvalid,
    output logic                      s_wready,
    input  logic [S_DATA_WIDTH-1:0]   s_wdata,
    input  logic [S_DATA_WIDTH/8-1:0] s_wstrb,
    input  logic                      s_wlast,

    output logic                      s_bvalid,
    input  logic                      s_bready,
    output logic [ID_WIDTH-1:0]       s_bid,
    output logic [1:0]                s_bresp,

    // Master interface (wide, to slave)
    output logic                      m_awvalid,
    input  logic                      m_awready,
    output logic [ADDR_WIDTH-1:0]     m_awaddr,
    output logic [7:0]                m_awlen,
    output logic [2:0]                m_awsize,
    output logic [1:0]                m_awburst,
    output logic [ID_WIDTH-1:0]       m_awid,
    // ... other AW signals

    output logic                      m_wvalid,
    input  logic                      m_wready,
    output logic [M_DATA_WIDTH-1:0]   m_wdata,
    output logic [M_DATA_WIDTH/8-1:0] m_wstrb,
    output logic                      m_wlast,

    input  logic                      m_bvalid,
    output logic                      m_bready,
    input  logic [ID_WIDTH-1:0]       m_bid,
    input  logic [1:0]                m_bresp
);

## 8.4	2.5.4 Burst Length Conversion

### 8.4.1	Ratio Calculation

localparam int RATIO = M_DATA_WIDTH / S_DATA_WIDTH;
localparam int RATIO_LOG2 = $clog2(RATIO);

// New AWLEN = (original AWLEN + 1) / RATIO - 1
// = (AWLEN + 1) >> RATIO_LOG2 - 1

### 8.4.2	Examples

Table 2.15: Burst Length Conversion Examples

| S_DATA | M_DATA | Ratio | S_AWLEN | S_beats | M_AWLEN | M_beats |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | 512 | 8 | 7 | 8 | 0 | 1 |
| 64 | 512 | 8 | 15 | 16 | 1 | 2 |
| 64 | 256 | 4 | 3 | 4 | 0 | 1 |
| 64 | 256 | 4 | 7 | 8 | 1 | 2 |

### 8.4.3	Non-Aligned Bursts

When burst length is not a multiple of ratio:

S_AWLEN = 5 (6 beats), RATIO = 8
M_AWLEN = 0 (1 beat)

The 6 narrow beats pack into 1 wide beat.
Remaining 2 positions have WSTRB = 0 (no write).

## 8.5	2.5.5 Address Channel Handling

### 8.5.1	AW Passthrough with Adjustment

// Burst length adjustment
logic [7:0] w_adjusted_awlen;
assign w_adjusted_awlen = ((s_awlen + 1) >> RATIO_LOG2) - 1;

// Size adjustment (wider data = larger size)
logic [2:0] w_adjusted_awsize;
assign w_adjusted_awsize = s_awsize + RATIO_LOG2;

// Address alignment check
logic w_aligned;
assign w_aligned = (s_awaddr[RATIO_LOG2+2:0] == '0);

### 8.5.2	Skid Buffer for AW

axi_skid_buffer #(
    .DATA_WIDTH(AW_CHANNEL_WIDTH)
) u_aw_skid (
    .clk     (clk),
    .rst_n   (rst_n),
    .s_valid (s_awvalid),
    .s_ready (s_awready),
    .s_data  ({s_awid, s_awaddr, w_adjusted_awlen, ...}),
    .m_valid (m_awvalid),
    .m_ready (m_awready),
    .m_data  ({m_awid, m_awaddr, m_awlen, ...})
);

## 8.6	2.5.6 Write Data Channel

### 8.6.1	Upsize Instance

axi_data_upsize #(
    .NARROW_WIDTH(S_DATA_WIDTH),
    .WIDE_WIDTH(M_DATA_WIDTH),
    .NARROW_SB_WIDTH(S_DATA_WIDTH/8),  // WSTRB
    .WIDE_SB_WIDTH(M_DATA_WIDTH/8),
    .SB_OR_MODE(0),                     // Concatenate WSTRB
    .USE_LAST(1)
) u_wdata_upsize (
    .clk        (clk),
    .rst_n      (rst_n),
    .s_valid    (s_wvalid),
    .s_ready    (s_wready),
    .s_data     (s_wdata),
    .s_sideband (s_wstrb),
    .s_last     (s_wlast),
    .m_valid    (m_wvalid),
    .m_ready    (m_wready),
    .m_data     (m_wdata),
    .m_sideband (m_wstrb),
    .m_last     (m_wlast)
);

## 8.7	2.5.7 Response Channel

### 8.7.1	B Channel Passthrough

The response channel passes through unchanged:

// Simple passthrough (or via skid buffer)
assign s_bvalid = m_bvalid;
assign m_bready = s_bready;
assign s_bid    = m_bid;
assign s_bresp  = m_bresp;

### 8.7.2	Response Ordering

Responses return in order because: - Single outstanding transaction per ID - Upsize doesn’t reorder data - B response generated after all W beats accepted

## 8.8	2.5.8 AW/W Synchronization

### 8.8.1	Challenge

AXI4 allows AW to arrive before, with, or after W data. The converter must handle all cases:

- AW before W: Normal pipelining

- W before AW: Data buffered until AW arrives

- Interleaved: Multiple transactions in flight

### 8.8.2	Solution

Use FIFO for AW information needed by upsize logic:

// FIFO stores AWLEN for burst tracking
fifo_sync #(.WIDTH(8), .DEPTH(4)) u_aw_info_fifo (
    .clk     (clk),
    .rst_n   (rst_n),
    .wr_en   (s_awvalid && s_awready),
    .wr_data (s_awlen),
    .rd_en   (m_wvalid && m_wready && m_wlast),
    .rd_data (current_awlen),
    .full    (aw_fifo_full),
    .empty   (aw_fifo_empty)
);

## 8.9	2.5.9 Resource Utilization

### 8.9.1	Typical Resources (64→512, ID=4, ADDR=64)

AW skid buffer:     ~200 flip-flops
W upsize:           ~600 flip-flops, ~60 LUTs
B skid buffer:      ~20 flip-flops
AW info FIFO:       ~50 flip-flops
Control logic:      ~100 LUTs

Total: ~870 flip-flops, ~160 LUTs

## 8.10	2.5.10 Timing Characteristics

### 8.10.1	Latency

Table 2.16: Write Converter Latency

| Path | Latency |
| --- | --- |
| AW passthrough | 1-2 cycles (skid) |
| W upsize | N cycles (accumulation) |
| B passthrough | 1 cycle (skid) |

### 8.10.2	Throughput

- AW channel: 1 transaction/cycle

- W channel: 100% (upsize is 100%)

- B channel: 1 response/cycle

## 8.11	2.5.11 Usage Example

axi4_dwidth_converter_wr #(
    .S_DATA_WIDTH(64),
    .M_DATA_WIDTH(512),
    .ADDR_WIDTH(64),
    .ID_WIDTH(4),
    .SKID_DEPTH(2)
) u_wr_converter (
    .clk     (aclk),
    .rst_n   (aresetn),

    // 64-bit slave interface (from CPU)
    .s_awvalid (cpu_awvalid),
    .s_awready (cpu_awready),
    .s_awaddr  (cpu_awaddr),
    .s_awlen   (cpu_awlen),
    // ... other s_* signals

    // 512-bit master interface (to DDR)
    .m_awvalid (ddr_awvalid),
    .m_awready (ddr_awready),
    .m_awaddr  (ddr_awaddr),
    .m_awlen   (ddr_awlen),
    // ... other m_* signals
);

Next: axi4_dwidth_converter_rd

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	2.6 axi4_dwidth_converter_rd

The axi4_dwidth_converter_rd module provides complete AXI4 read path conversion, handling AR and R channels with burst length adjustment and burst-aware RLAST generation.

## 9.1	2.6.1 Purpose and Function

The read converter combines the generic axi_data_dnsize with AXI4 protocol handling:

- Address Channel (AR): Passes through with burst length adjustment

- Read Data Channel (R): Uses axi_data_dnsize for data splitting

- Burst Tracking: Generates correct RLAST based on original ARLEN

- Response Broadcasting: Propagates RRESP to all narrow beats

## 9.2	2.6.2 Block Diagram

### 9.2.1	Figure 2.7: Read Converter Architecture

Read Converter Architecture

## 9.3	2.6.3 Interface Specification

### 9.3.1	Parameters

Table 2.17: Read Converter Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| S_DATA_WIDTH | int | 64 | Slave-side (narrow) data width |
| M_DATA_WIDTH | int | 512 | Master-side (wide) data width |
| ADDR_WIDTH | int | 64 | Address width |
| ID_WIDTH | int | 4 | Transaction ID width |
| DUAL_BUFFER | bit | 1 | Enable dual-buffer for 100% throughput |
| SKID_DEPTH | int | 2 | Pipeline buffer depth |

### 9.3.2	Ports

module axi4_dwidth_converter_rd #(
    parameter int S_DATA_WIDTH = 64,
    parameter int M_DATA_WIDTH = 512,
    parameter int ADDR_WIDTH   = 64,
    parameter int ID_WIDTH     = 4,
    parameter bit DUAL_BUFFER  = 1,
    parameter int SKID_DEPTH   = 2
) (
    input  logic clk,
    input  logic rst_n,

    // Slave interface (narrow, to master)
    input  logic                      s_arvalid,
    output logic                      s_arready,
    input  logic [ADDR_WIDTH-1:0]     s_araddr,
    input  logic [7:0]                s_arlen,
    input  logic [2:0]                s_arsize,
    input  logic [1:0]                s_arburst,
    input  logic [ID_WIDTH-1:0]       s_arid,

    output logic                      s_rvalid,
    input  logic                      s_rready,
    output logic [S_DATA_WIDTH-1:0]   s_rdata,
    output logic [ID_WIDTH-1:0]       s_rid,
    output logic [1:0]                s_rresp,
    output logic                      s_rlast,

    // Master interface (wide, from slave)
    output logic                      m_arvalid,
    input  logic                      m_arready,
    output logic [ADDR_WIDTH-1:0]     m_araddr,
    output logic [7:0]                m_arlen,
    output logic [2:0]                m_arsize,
    output logic [1:0]                m_arburst,
    output logic [ID_WIDTH-1:0]       m_arid,

    input  logic                      m_rvalid,
    output logic                      m_rready,
    input  logic [M_DATA_WIDTH-1:0]   m_rdata,
    input  logic [ID_WIDTH-1:0]       m_rid,
    input  logic [1:0]                m_rresp,
    input  logic                      m_rlast
);

## 9.4	2.6.4 Burst Length Conversion

### 9.4.1	Ratio Calculation

Same as write converter:

localparam int RATIO = M_DATA_WIDTH / S_DATA_WIDTH;
localparam int RATIO_LOG2 = $clog2(RATIO);

// New ARLEN = (original ARLEN + 1) / RATIO - 1

### 9.4.2	Examples

Table 2.18: Read Burst Length Conversion

| S_DATA | M_DATA | Ratio | S_ARLEN | S_beats | M_ARLEN | M_beats |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | 512 | 8 | 7 | 8 | 0 | 1 |
| 64 | 512 | 8 | 15 | 16 | 1 | 2 |
| 64 | 512 | 8 | 31 | 32 | 3 | 4 |

## 9.5	2.6.5 Address Channel Handling

### 9.5.1	AR Passthrough with Adjustment

// Burst length adjustment
logic [7:0] w_adjusted_arlen;
assign w_adjusted_arlen = ((s_arlen + 1) >> RATIO_LOG2) - 1;

// Size adjustment
logic [2:0] w_adjusted_arsize;
assign w_adjusted_arsize = s_arsize + RATIO_LOG2;

### 9.5.2	AR Information FIFO

Store original ARLEN for RLAST generation:

// FIFO to track original burst length
fifo_sync #(.WIDTH(8+ID_WIDTH), .DEPTH(4)) u_ar_info_fifo (
    .clk     (clk),
    .rst_n   (rst_n),
    .wr_en   (s_arvalid && s_arready),
    .wr_data ({s_arid, s_arlen}),
    .rd_en   (s_rvalid && s_rready && s_rlast),
    .rd_data ({current_arid, current_arlen}),
    .full    (ar_fifo_full),
    .empty   (ar_fifo_empty)
);

## 9.6	2.6.6 Read Data Channel

### 9.6.1	Downsize Instance

axi_data_dnsize #(
    .WIDE_WIDTH(M_DATA_WIDTH),
    .NARROW_WIDTH(S_DATA_WIDTH),
    .WIDE_SB_WIDTH(2),          // RRESP
    .NARROW_SB_WIDTH(2),
    .SB_BROADCAST(1),           // Broadcast RRESP
    .DUAL_BUFFER(DUAL_BUFFER),
    .USE_BURST_TRACKER(1),
    .BURST_LEN_WIDTH(8)
) u_rdata_dnsize (
    .clk        (clk),
    .rst_n      (rst_n),
    .s_valid    (m_rvalid),
    .s_ready    (m_rready),
    .s_data     (m_rdata),
    .s_sideband (m_rresp),
    .s_last     (m_rlast),
    .burst_len  (current_arlen),
    .m_valid    (s_rvalid),
    .m_ready    (s_rready),
    .m_data     (s_rdata),
    .m_sideband (s_rresp),
    .m_last     (s_rlast)
);

## 9.7	2.6.7 RLAST Generation

### 9.7.1	Challenge

The wide interface generates RLAST based on M_ARLEN, but the narrow interface needs RLAST based on S_ARLEN:

Wide RLAST: Asserted on beat (M_ARLEN + 1)
Narrow RLAST: Asserted on beat (S_ARLEN + 1) = (M_ARLEN + 1) * RATIO

### 9.7.2	Solution: Burst Tracker

// Track narrow beats within burst
logic [15:0] r_beat_count;
logic [7:0]  r_total_narrow_beats;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_beat_count <= '0;
        r_total_narrow_beats <= '0;
    end else begin
        if (new_burst_start) begin
            r_beat_count <= '0;
            r_total_narrow_beats <= (current_arlen + 1) * RATIO - 1;
        end else if (s_rvalid && s_rready) begin
            r_beat_count <= r_beat_count + 1;
        end
    end
end

assign s_rlast = (r_beat_count == r_total_narrow_beats);

## 9.8	2.6.8 RID Handling

### 9.8.1	ID Passthrough

RID passes through unchanged:

// RID from wide interface propagates to all narrow beats
assign s_rid = m_rid;

// Or use tracked ID from FIFO
assign s_rid = current_arid;

## 9.9	2.6.9 Dual-Buffer Impact

### 9.9.1	Performance Comparison

Table 2.19: Read Converter Performance

| Mode | Throughput | Latency | Resources |
| --- | --- | --- | --- |
| Single | 80-90% | 1 cycle | 1x |
| Dual | 100% | 1 cycle | 2x |

### 9.9.2	When to Use Dual Buffer

Use dual buffer for: - DDR read paths with high bandwidth - Streaming read applications - DMA read operations

Use single buffer for: - Control register reads - Low-bandwidth paths - Area-constrained designs

## 9.10	2.6.10 Resource Utilization

### 9.10.1	Typical Resources (512→64, ID=4, Dual Buffer)

AR skid buffer:     ~150 flip-flops
R downsize (dual):  ~1200 flip-flops, ~100 LUTs
AR info FIFO:       ~100 flip-flops
Burst tracker:      ~30 flip-flops, ~20 LUTs
Control logic:      ~80 LUTs

Total: ~1480 flip-flops, ~200 LUTs

### 9.10.2	Single Buffer Version

R downsize (single): ~600 flip-flops, ~50 LUTs

Total: ~880 flip-flops, ~120 LUTs

## 9.11	2.6.11 Timing Characteristics

### 9.11.1	Latency

Table 2.20: Read Converter Latency

| Path | Latency |
| --- | --- |
| AR passthrough | 1-2 cycles (skid) |
| First R beat | 1 cycle (load buffer) |
| Subsequent R beats | 1 beat/cycle |

### 9.11.2	Throughput

- AR channel: 1 transaction/cycle

- R channel: 80% (single) or 100% (dual)

## 9.12	2.6.12 Usage Example

axi4_dwidth_converter_rd #(
    .S_DATA_WIDTH(64),
    .M_DATA_WIDTH(512),
    .ADDR_WIDTH(64),
    .ID_WIDTH(4),
    .DUAL_BUFFER(1),      // High-performance mode
    .SKID_DEPTH(2)
) u_rd_converter (
    .clk     (aclk),
    .rst_n   (aresetn),

    // 64-bit slave interface (to CPU)
    .s_arvalid (cpu_arvalid),
    .s_arready (cpu_arready),
    .s_araddr  (cpu_araddr),
    .s_arlen   (cpu_arlen),
    .s_rvalid  (cpu_rvalid),
    .s_rready  (cpu_rready),
    .s_rdata   (cpu_rdata),
    // ... other s_* signals

    // 512-bit master interface (from DDR)
    .m_arvalid (ddr_arvalid),
    .m_arready (ddr_arready),
    .m_araddr  (ddr_araddr),
    .m_arlen   (ddr_arlen),
    .m_rvalid  (ddr_rvalid),
    .m_rready  (ddr_rready),
    .m_rdata   (ddr_rdata),
    // ... other m_* signals
);

Next: Chapter 3: Protocol Converter Blocks

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	3.1 Protocol Conversion Overview

Protocol converters enable communication between components using different communication protocols, essential for integrating diverse IP blocks in complex SoC designs.

## 10.1	3.1.1 Available Converters

### 10.1.1	AXI4 to AXI4-Lite (Protocol Downgrade)

Table 3.1: AXI4 to AXI4-Lite Converters

| Module | Function | Test Status |
| --- | --- | --- |
| axi4_to_axil4_rd | Read burst decomposition | 14/14 passing |
| axi4_to_axil4_wr | Write burst decomposition | 14/14 passing |
| axi4_to_axil4 | Full bidirectional wrapper | Composed |

### 10.1.2	AXI4-Lite to AXI4 (Protocol Upgrade)

Table 3.2: AXI4-Lite to AXI4 Converters

| Module | Function | Test Status |
| --- | --- | --- |
| axil4_to_axi4_rd | Read protocol upgrade | 7/7 passing |
| axil4_to_axi4_wr | Write protocol upgrade | 7/7 passing |
| axil4_to_axi4 | Full bidirectional wrapper | Composed |

### 10.1.3	Other Protocol Converters

Table 3.3: Other Protocol Converters

| Module | Function | Status |
| --- | --- | --- |
| axi4_to_apb4_convert | Full AXI4-to-APB bridge | Production |
| peakrdl_to_cmdrsp | Register interface adapter | Production |
| uart_axil_bridge | UART to AXI4-Lite | Planned |

## 10.2	3.1.2 Protocol Comparison

### 10.2.1	Figure 3.1: Protocol Feature Comparison

Protocol Comparison

Protocol Comparison

### 10.2.2	Feature Matrix

Table 3.4: Protocol Feature Comparison

| Feature | AXI4 | AXI4-Lite | APB |
| --- | --- | --- | --- |
| Channels | 5 (AW, W, B, AR, R) | 5 (simplified) | 1 (combined) |
| Bursts | Up to 256 beats | Single beat only | Single beat |
| Out-of-order | Yes (ID-based) | No | No |
| Pipelining | Yes | Optional | No (2-phase) |
| Data widths | 8-1024 bits | 32/64 bits | 8-32 bits |

## 10.3	3.1.3 Conversion Strategies

### 10.3.1	AXI4 to AXI4-Lite

Challenge: Decompose multi-beat bursts into sequential single beats.

Strategy: 1. Accept AXI4 burst transaction 2. Issue N single-beat AXI4-Lite transactions 3. Aggregate responses 4. Return combined response to AXI4 master

Complexity: Medium (FSM-based decomposition)

### 10.3.2	AXI4-Lite to AXI4

Challenge: Add AXI4 burst signals with appropriate defaults.

Strategy: 1. Pass through all AXI4-Lite signals 2. Add default values for burst signals (LEN=0, SIZE=2, BURST=INCR) 3. Add default IDs (configurable)

Complexity: Very low (combinational only)

### 10.3.3	AXI4 to APB

Challenge: Bridge 5-channel AXI4 to 2-phase APB protocol.

Strategy: 1. Accept AXI4 transaction 2. Execute APB setup phase 3. Execute APB access phase 4. Return AXI4 response

Complexity: High (full protocol FSM)

## 10.4	3.1.4 Performance Characteristics

Table 3.5: Protocol Converter Performance

| Converter | Single-Beat | Burst (N) | Area |
| --- | --- | --- | --- |
| axi4_to_axil4 | 0 cycles | 2N cycles | ~450 LUTs |
| axil4_to_axi4 | 0 cycles | N/A | ~110 LUTs |
| axi4_to_apb4 | 3-5 cycles | (3-5)N cycles | ~300 LUTs |

### 10.4.1	Key Observations

- Zero-overhead upgrade: AXI4-Lite to AXI4 is purely combinational

- Burst penalty: AXI4 to AXI4-Lite doubles cycle count for bursts

- APB overhead: 3-5 cycle minimum per APB transaction

## 10.5	3.1.5 Use Case Guidelines

### 10.5.1	When to Use AXI4 to AXI4-Lite

Use when: - CPU/DMA with burst support needs simple peripheral access - Want to simplify peripheral design (no burst handling) - Data widths match (no width conversion needed) - Burst performance is not critical

Avoid when: - High-bandwidth streaming data - Latency-critical paths - Many back-to-back bursts

### 10.5.2	When to Use AXI4-Lite to AXI4

Use when: - Legacy AXI4-Lite IP connects to AXI4 fabric - Designing simple peripheral for AXI4 system - Want zero-overhead protocol upgrade - Don’t need burst capability

Avoid when: - Need actual burst support (use native AXI4) - Width conversion needed (use width converters)

### 10.5.3	When to Use AXI4 to APB

Use when: - AXI4 masters need APB peripheral access - Building CPU-to-peripheral bridges - Integrating legacy APB devices

Avoid when: - High-performance paths (APB is slow) - Streaming data (APB is sequential)

## 10.6	3.1.6 Integration Patterns

### 10.6.1	Pattern 1: CPU to Peripherals

CPU (AXI4) → AXI4-to-AXIL4 → AXIL4 Peripherals
                          → AXI4-to-APB → APB Peripherals

### 10.6.2	Pattern 2: Simple IP in AXI4 Fabric

Simple IP (AXIL4) → AXIL4-to-AXI4 → AXI4 Crossbar

### 10.6.3	Pattern 3: Mixed System

CPU (AXI4) → AXI4 Crossbar → DDR (AXI4)
                           → AXI4-to-AXIL4 → Config Regs (AXIL4)
                           → AXI4-to-APB → UART, GPIO (APB)

Next: AXI4 to AXI4-Lite

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	3.2 AXI4 to AXI4-Lite Converter

The axi4_to_axil4 converter family decomposes AXI4 burst transactions into sequential AXI4-Lite single-beat transactions.

## 11.1	3.2.1 Module Organization

axi4_to_axil4.sv          # Full bidirectional wrapper
├── axi4_to_axil4_rd.sv   # Read path converter
└── axi4_to_axil4_wr.sv   # Write path converter

### 11.1.1	Design Philosophy

Separate read and write paths enable: - Independent optimization - Selective instantiation (read-only, write-only, or both) - Simpler verification (test paths independently)

## 11.2	3.2.2 Read Path (axi4_to_axil4_rd)

### 11.2.1	Block Diagram

### 11.2.2	Figure 3.2: AXI4 to AXI4-Lite Read Path

AXI4 to AXIL4 Read

AXI4 to AXIL4 Read

### 11.2.3	Operation

Single-Beat (ARLEN=0):

Cycle 0: AR accepted (passthrough)
Cycle 1: R returned (passthrough)
Total: 0 extra cycles (pure passthrough)

Multi-Beat (ARLEN=N-1):

Cycle 0:   AR[0] issued to AXIL4
Cycle 1:   R[0] received, AR[1] issued
Cycle 2:   R[1] received, AR[2] issued
...
Cycle 2N-1: R[N-1] received (RLAST)
Total: 2N cycles (1 cycle per AR + 1 cycle per R)

### 11.2.4	State Machine

typedef enum logic [1:0] {
    IDLE       = 2'b00,  // Wait for AR
    DECOMPOSE  = 2'b01,  // Issuing single beats
    WAIT_R     = 2'b10   // Wait for last R
} rd_state_t;

### 11.2.5	Implementation

// Beat counter and address tracking
logic [7:0] r_beat_count;
logic [7:0] r_arlen_saved;
logic [ADDR_WIDTH-1:0] r_current_addr;

// State machine
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
        r_beat_count <= '0;
    end else begin
        case (r_state)
            IDLE: begin
                if (s_arvalid && s_arready) begin
                    r_arlen_saved <= s_arlen;
                    r_current_addr <= s_araddr;
                    if (s_arlen == 0) begin
                        // Single beat - passthrough
                        r_state <= WAIT_R;
                    end else begin
                        r_state <= DECOMPOSE;
                        r_beat_count <= 8'd1;
                    end
                end
            end

            DECOMPOSE: begin
                if (m_arvalid && m_arready) begin
                    r_current_addr <= r_current_addr + (1 << s_arsize);
                    r_beat_count <= r_beat_count + 1;
                    if (r_beat_count == r_arlen_saved) begin
                        r_state <= WAIT_R;
                    end
                end
            end

            WAIT_R: begin
                if (s_rvalid && s_rready && s_rlast) begin
                    r_state <= IDLE;
                end
            end
        endcase
    end
end

// AXIL4 AR generation
assign m_arvalid = (r_state == DECOMPOSE) || (r_state == IDLE && s_arvalid);
assign m_araddr = r_current_addr;

// R aggregation
assign s_rlast = (r_beat_count == r_arlen_saved) || (r_arlen_saved == 0);

## 11.3	3.2.3 Write Path (axi4_to_axil4_wr)

### 11.3.1	Block Diagram

### 11.3.2	Figure 3.3: AXI4 to AXI4-Lite Write Path

AXI4 to AXIL4 Write

AXI4 to AXIL4 Write

### 11.3.3	Operation

Single-Beat (AWLEN=0):

Cycle 0: AW+W accepted (passthrough)
Cycle 1: B returned (passthrough)
Total: 0 extra cycles

Multi-Beat (AWLEN=N-1):

Cycle 0:   AW[0] + W[0] issued
Cycle 1:   B[0] received, AW[1] + W[1] issued
...
Cycle 2N-1: B[N-1] received
Total: 2N cycles

### 11.3.4	AW/W Synchronization Challenge

AXI4 allows AW and W to arrive in any order: - AW before W - W before AW - Interleaved

### 11.3.5	Solution: Dual Accept Logic

// Track AW and W arrival independently
logic r_aw_accepted;
logic r_w_accepted;

// Accept both when ready to issue
always_ff @(posedge clk) begin
    if (w_issue_axil4) begin
        r_aw_accepted <= 1'b0;
        r_w_accepted <= 1'b0;
    end else begin
        if (s_awvalid && s_awready)
            r_aw_accepted <= 1'b1;
        if (s_wvalid && s_wready)
            r_w_accepted <= 1'b1;
    end
end

// Issue AXIL4 when both available
assign w_issue_axil4 = (r_aw_accepted || s_awvalid) &&
                       (r_w_accepted || s_wvalid) &&
                       m_ready;

### 11.3.6	Response Aggregation

// Track worst response in burst
logic [1:0] r_worst_bresp;

always_ff @(posedge clk) begin
    if (r_state == IDLE)
        r_worst_bresp <= 2'b00;  // OKAY
    else if (m_bvalid && m_bready)
        r_worst_bresp <= (m_bresp > r_worst_bresp) ? m_bresp : r_worst_bresp;
end

// Return worst response on final beat
assign s_bresp = (r_beat_count == r_awlen_saved) ?
                 ((m_bresp > r_worst_bresp) ? m_bresp : r_worst_bresp) :
                 r_worst_bresp;

## 11.4	3.2.4 Bidirectional Wrapper (axi4_to_axil4)

### 11.4.1	Composition Pattern

module axi4_to_axil4 #(
    parameter int DATA_WIDTH = 32,
    parameter int ADDR_WIDTH = 32,
    parameter int ID_WIDTH   = 4
) (
    // ... ports
);

    // Instantiate read path
    axi4_to_axil4_rd #(
        .DATA_WIDTH(DATA_WIDTH),
        .ADDR_WIDTH(ADDR_WIDTH),
        .ID_WIDTH(ID_WIDTH)
    ) u_rd (
        // ... read channel connections
    );

    // Instantiate write path
    axi4_to_axil4_wr #(
        .DATA_WIDTH(DATA_WIDTH),
        .ADDR_WIDTH(ADDR_WIDTH),
        .ID_WIDTH(ID_WIDTH)
    ) u_wr (
        // ... write channel connections
    );

endmodule

## 11.5	3.2.5 Resource Utilization

Table 3.6: AXI4 to AXIL4 Resources

| Module | Registers | LUTs | BRAM |
| --- | --- | --- | --- |
| axi4_to_axil4_rd | ~120 | ~180 | 0 |
| axi4_to_axil4_wr | ~150 | ~220 | 0 |
| axi4_to_axil4 (combined) | ~270 | ~400 | 0 |

## 11.6	3.2.6 Performance Analysis

### 11.6.1	Throughput

Table 3.7: AXI4 to AXIL4 Throughput

| Transaction Type | Throughput |
| --- | --- |
| Single-beat | 100% (passthrough) |
| 2-beat burst | 50% |
| 4-beat burst | 50% |
| N-beat burst | ~50% |

### 11.6.2	Latency

Table 3.8: AXI4 to AXIL4 Latency

| Transaction Type | Latency |
| --- | --- |
| Single-beat | 0 extra cycles |
| N-beat burst | 2N - 1 cycles |

## 11.7	3.2.7 Test Coverage

Test Suite: 42 tests passing

Table 3.9: Test Coverage Summary

| Test Category | Tests | Status |
| --- | --- | --- |
| Single-beat read | 4 | Pass |
| Multi-beat read | 6 | Pass |
| Single-beat write | 4 | Pass |
| Multi-beat write | 6 | Pass |
| Mixed traffic | 8 | Pass |
| Error injection | 6 | Pass |
| Edge cases | 8 | Pass |

Next: AXI4-Lite to AXI4

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	3.3 AXI4-Lite to AXI4 Converter

The axil4_to_axi4 converter family upgrades AXI4-Lite single-beat transactions to full AXI4 protocol by adding default burst signals.

## 12.1	3.3.1 Module Organization

axil4_to_axi4.sv          # Full bidirectional wrapper
├── axil4_to_axi4_rd.sv   # Read path converter
└── axil4_to_axi4_wr.sv   # Write path converter

## 12.2	3.3.2 Design Philosophy

Zero-Overhead Upgrade: - Purely combinational logic - No state machines or buffers - Adds default values for missing AXI4 signals

Why This Works: - AXI4-Lite is a subset of AXI4 - All AXI4-Lite transactions are single-beat - Missing AXI4 signals have well-defined defaults

## 12.3	3.3.3 Signal Mapping

### 12.3.1	Address Channel Signals

Table 3.10: AR Channel Mapping

| AXI4-Lite Signal | AXI4 Signal | Default/Mapping |
| --- | --- | --- |
| ARADDR | ARADDR | Passthrough |
| ARPROT | ARPROT | Passthrough |
| ARVALID | ARVALID | Passthrough |
| ARREADY | ARREADY | Passthrough |
| - | ARLEN | 8’h00 (single beat) |
| - | ARSIZE | $clog2(DATA_WIDTH/8) |
| - | ARBURST | 2’b01 (INCR) |
| - | ARLOCK | 1’b0 (normal) |
| - | ARCACHE | 4’b0000 (non-cacheable) |
| - | ARQOS | 4’b0000 (no QoS) |
| - | ARID | Configurable default |

### 12.3.2	Data Channel Signals

Table 3.11: R Channel Mapping

| AXI4-Lite Signal | AXI4 Signal | Default/Mapping |
| --- | --- | --- |
| RDATA | RDATA | Passthrough |
| RRESP | RRESP | Passthrough |
| RVALID | RVALID | Passthrough |
| RREADY | RREADY | Passthrough |
| - | RLAST | 1’b1 (always last) |
| - | RID | Matches ARID |

## 12.4	3.3.4 Read Path (axil4_to_axi4_rd)

### 12.4.1	Block Diagram

### 12.4.2	Figure 3.4: AXI4-Lite to AXI4 Read Path

AXIL4 to AXI4 Read

AXIL4 to AXI4 Read

### 12.4.3	Implementation

module axil4_to_axi4_rd #(
    parameter int DATA_WIDTH = 32,
    parameter int ADDR_WIDTH = 32,
    parameter int ID_WIDTH   = 4,
    parameter logic [ID_WIDTH-1:0] DEFAULT_ARID = '0
) (
    // AXI4-Lite slave interface (input)
    input  logic                    s_arvalid,
    output logic                    s_arready,
    input  logic [ADDR_WIDTH-1:0]   s_araddr,
    input  logic [2:0]              s_arprot,

    output logic                    s_rvalid,
    input  logic                    s_rready,
    output logic [DATA_WIDTH-1:0]   s_rdata,
    output logic [1:0]              s_rresp,

    // AXI4 master interface (output)
    output logic                    m_arvalid,
    input  logic                    m_arready,
    output logic [ADDR_WIDTH-1:0]   m_araddr,
    output logic [7:0]              m_arlen,
    output logic [2:0]              m_arsize,
    output logic [1:0]              m_arburst,
    output logic                    m_arlock,
    output logic [3:0]              m_arcache,
    output logic [2:0]              m_arprot,
    output logic [3:0]              m_arqos,
    output logic [ID_WIDTH-1:0]     m_arid,

    input  logic                    m_rvalid,
    output logic                    m_rready,
    input  logic [DATA_WIDTH-1:0]   m_rdata,
    input  logic [1:0]              m_rresp,
    input  logic                    m_rlast,
    input  logic [ID_WIDTH-1:0]     m_rid
);

    // AR channel - passthrough with defaults
    assign m_arvalid = s_arvalid;
    assign s_arready = m_arready;
    assign m_araddr  = s_araddr;
    assign m_arprot  = s_arprot;

    // AXI4 burst defaults
    assign m_arlen   = 8'h00;                    // Single beat
    assign m_arsize  = $clog2(DATA_WIDTH/8);     // Full width
    assign m_arburst = 2'b01;                    // INCR
    assign m_arlock  = 1'b0;                     // Normal access
    assign m_arcache = 4'b0000;                  // Non-cacheable
    assign m_arqos   = 4'b0000;                  // No QoS
    assign m_arid    = DEFAULT_ARID;             // Configurable ID

    // R channel - passthrough (ignore RLAST and RID)
    assign s_rvalid  = m_rvalid;
    assign m_rready  = s_rready;
    assign s_rdata   = m_rdata;
    assign s_rresp   = m_rresp;

endmodule

Key Points: - No registers - purely combinational - RLAST from AXI4 is ignored (always 1 for AXIL4) - RID from AXI4 is ignored (no ID tracking in AXIL4)

## 12.5	3.3.5 Write Path (axil4_to_axi4_wr)

### 12.5.1	Block Diagram

### 12.5.2	Figure 3.5: AXI4-Lite to AXI4 Write Path

AXIL4 to AXI4 Write

AXIL4 to AXI4 Write

### 12.5.3	Implementation

module axil4_to_axi4_wr #(
    parameter int DATA_WIDTH = 32,
    parameter int ADDR_WIDTH = 32,
    parameter int ID_WIDTH   = 4,
    parameter logic [ID_WIDTH-1:0] DEFAULT_AWID = '0
) (
    // AXI4-Lite slave interface (input)
    input  logic                    s_awvalid,
    output logic                    s_awready,
    input  logic [ADDR_WIDTH-1:0]   s_awaddr,
    input  logic [2:0]              s_awprot,

    input  logic                    s_wvalid,
    output logic                    s_wready,
    input  logic [DATA_WIDTH-1:0]   s_wdata,
    input  logic [DATA_WIDTH/8-1:0] s_wstrb,

    output logic                    s_bvalid,
    input  logic                    s_bready,
    output logic [1:0]              s_bresp,

    // AXI4 master interface (output)
    output logic                    m_awvalid,
    input  logic                    m_awready,
    output logic [ADDR_WIDTH-1:0]   m_awaddr,
    output logic [7:0]              m_awlen,
    output logic [2:0]              m_awsize,
    output logic [1:0]              m_awburst,
    output logic                    m_awlock,
    output logic [3:0]              m_awcache,
    output logic [2:0]              m_awprot,
    output logic [3:0]              m_awqos,
    output logic [ID_WIDTH-1:0]     m_awid,

    output logic                    m_wvalid,
    input  logic                    m_wready,
    output logic [DATA_WIDTH-1:0]   m_wdata,
    output logic [DATA_WIDTH/8-1:0] m_wstrb,
    output logic                    m_wlast,

    input  logic                    m_bvalid,
    output logic                    m_bready,
    input  logic [1:0]              m_bresp,
    input  logic [ID_WIDTH-1:0]     m_bid
);

    // AW channel
    assign m_awvalid = s_awvalid;
    assign s_awready = m_awready;
    assign m_awaddr  = s_awaddr;
    assign m_awprot  = s_awprot;
    assign m_awlen   = 8'h00;
    assign m_awsize  = $clog2(DATA_WIDTH/8);
    assign m_awburst = 2'b01;
    assign m_awlock  = 1'b0;
    assign m_awcache = 4'b0000;
    assign m_awqos   = 4'b0000;
    assign m_awid    = DEFAULT_AWID;

    // W channel
    assign m_wvalid  = s_wvalid;
    assign s_wready  = m_wready;
    assign m_wdata   = s_wdata;
    assign m_wstrb   = s_wstrb;
    assign m_wlast   = 1'b1;  // Always last (single beat)

    // B channel
    assign s_bvalid  = m_bvalid;
    assign m_bready  = s_bready;
    assign s_bresp   = m_bresp;

endmodule

## 12.6	3.3.6 Bidirectional Wrapper

module axil4_to_axi4 #(
    parameter int DATA_WIDTH = 32,
    parameter int ADDR_WIDTH = 32,
    parameter int ID_WIDTH   = 4,
    parameter logic [ID_WIDTH-1:0] DEFAULT_ID = '0
) (
    // ... all port declarations
);

    axil4_to_axi4_rd #(
        .DATA_WIDTH(DATA_WIDTH),
        .ADDR_WIDTH(ADDR_WIDTH),
        .ID_WIDTH(ID_WIDTH),
        .DEFAULT_ARID(DEFAULT_ID)
    ) u_rd (/* connections */);

    axil4_to_axi4_wr #(
        .DATA_WIDTH(DATA_WIDTH),
        .ADDR_WIDTH(ADDR_WIDTH),
        .ID_WIDTH(ID_WIDTH),
        .DEFAULT_AWID(DEFAULT_ID)
    ) u_wr (/* connections */);

endmodule

## 12.7	3.3.7 Resource Utilization

Table 3.12: AXIL4 to AXI4 Resources

| Module | Registers | LUTs |
| --- | --- | --- |
| axil4_to_axi4_rd | 0 | ~50 |
| axil4_to_axi4_wr | 0 | ~60 |
| axil4_to_axi4 (combined) | 0 | ~110 |

Note: Zero registers - purely combinational logic.

## 12.8	3.3.8 Performance

Table 3.13: AXIL4 to AXI4 Performance

| Metric | Value |
| --- | --- |
| Latency | 0 cycles |
| Throughput | 100% |
| Max frequency | Wire speed |

## 12.9	3.3.9 Test Coverage

Test Suite: 14 tests passing

Table 3.14: Test Coverage Summary

| Test Category | Tests | Status |
| --- | --- | --- |
| Single-beat read | 3 | Pass |
| Single-beat write | 3 | Pass |
| Mixed traffic | 4 | Pass |
| Default ID verification | 2 | Pass |
| Edge cases | 2 | Pass |

## 12.10	3.3.10 Usage Example

// Upgrade simple register block to AXI4 fabric
axil4_to_axi4 #(
    .DATA_WIDTH(32),
    .ADDR_WIDTH(32),
    .ID_WIDTH(4),
    .DEFAULT_ID(4'h5)  // Unique ID for this IP
) u_protocol_upgrade (
    // Connect AXIL4 register block
    .s_arvalid (reg_block_arvalid),
    .s_arready (reg_block_arready),
    .s_araddr  (reg_block_araddr),
    // ... other AXIL4 signals

    // Connect to AXI4 crossbar
    .m_arvalid (xbar_arvalid),
    .m_arready (xbar_arready),
    .m_araddr  (xbar_araddr),
    .m_arlen   (xbar_arlen),
    // ... other AXI4 signals
);

Next: AXI4 to APB

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	3.4 AXI4 to APB Converter

The axi4_to_apb4_convert module provides full protocol translation from AXI4 to APB, enabling AXI4 masters to access APB peripherals.

## 13.1	3.4.1 Purpose

Bridge the significant protocol differences between AXI4 and APB:

Table 3.15: AXI4 vs APB Comparison

| Aspect | AXI4 | APB |
| --- | --- | --- |
| Channels | 5 (AW, W, B, AR, R) | 1 (combined) |
| Phases | Pipelined | 2-phase (setup, access) |
| Bursts | Up to 256 beats | Single transfer |
| Address width | Up to 64 bits | Typically 32 bits |
| Data width | 8-1024 bits | 8-32 bits |

## 13.2	3.4.2 Block Diagram

### 13.2.1	Figure 3.6: AXI4 to APB Converter

AXI4 to APB

## 13.3	3.4.3 Interface Specification

### 13.3.1	Parameters

Table 3.16: AXI4 to APB Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| AXI_ADDR_WIDTH | int | 64 | AXI4 address width |
| AXI_DATA_WIDTH | int | 32 | AXI4 data width |
| AXI_ID_WIDTH | int | 4 | AXI4 ID width |
| APB_ADDR_WIDTH | int | 32 | APB address width |
| APB_DATA_WIDTH | int | 32 | APB data width |

### 13.3.2	Ports

module axi4_to_apb4_convert #(
    parameter int AXI_ADDR_WIDTH = 64,
    parameter int AXI_DATA_WIDTH = 32,
    parameter int AXI_ID_WIDTH   = 4,
    parameter int APB_ADDR_WIDTH = 32,
    parameter int APB_DATA_WIDTH = 32
) (
    input  logic clk,
    input  logic rst_n,

    // AXI4 slave interface
    // AW channel
    input  logic                        s_awvalid,
    output logic                        s_awready,
    input  logic [AXI_ADDR_WIDTH-1:0]   s_awaddr,
    input  logic [7:0]                  s_awlen,
    input  logic [AXI_ID_WIDTH-1:0]     s_awid,

    // W channel
    input  logic                        s_wvalid,
    output logic                        s_wready,
    input  logic [AXI_DATA_WIDTH-1:0]   s_wdata,
    input  logic [AXI_DATA_WIDTH/8-1:0] s_wstrb,
    input  logic                        s_wlast,

    // B channel
    output logic                        s_bvalid,
    input  logic                        s_bready,
    output logic [AXI_ID_WIDTH-1:0]     s_bid,
    output logic [1:0]                  s_bresp,

    // AR channel
    input  logic                        s_arvalid,
    output logic                        s_arready,
    input  logic [AXI_ADDR_WIDTH-1:0]   s_araddr,
    input  logic [7:0]                  s_arlen,
    input  logic [AXI_ID_WIDTH-1:0]     s_arid,

    // R channel
    output logic                        s_rvalid,
    input  logic                        s_rready,
    output logic [AXI_DATA_WIDTH-1:0]   s_rdata,
    output logic [AXI_ID_WIDTH-1:0]     s_rid,
    output logic [1:0]                  s_rresp,
    output logic                        s_rlast,

    // APB master interface
    output logic                        psel,
    output logic                        penable,
    output logic                        pwrite,
    output logic [APB_ADDR_WIDTH-1:0]   paddr,
    output logic [APB_DATA_WIDTH-1:0]   pwdata,
    output logic [APB_DATA_WIDTH/8-1:0] pstrb,
    input  logic                        pready,
    input  logic [APB_DATA_WIDTH-1:0]   prdata,
    input  logic                        pslverr
);

## 13.4	3.4.4 State Machine

### 13.4.1	Figure 3.7: AXI4 to APB FSM

AXI4 to APB FSM

### 13.4.2	States

typedef enum logic [2:0] {
    IDLE        = 3'b000,  // Wait for AXI4 transaction
    APB_SETUP   = 3'b001,  // APB setup phase
    APB_ACCESS  = 3'b010,  // APB access phase (wait PREADY)
    AXI_RESP_B  = 3'b011,  // Send AXI4 B response
    AXI_RESP_R  = 3'b100,  // Send AXI4 R response
    BURST_NEXT  = 3'b101   // Next beat in burst
} apb_state_t;

### 13.4.3	Transitions

Table 3.17: FSM Transitions

| Current State | Condition | Next State |
| --- | --- | --- |
| IDLE | s_awvalid | APB_SETUP (write) |
| IDLE | s_arvalid | APB_SETUP (read) |
| APB_SETUP | always | APB_ACCESS |
| APB_ACCESS | pready && is_write | AXI_RESP_B |
| APB_ACCESS | pready && is_read | AXI_RESP_R |
| AXI_RESP_B | s_bready && !more_beats | IDLE |
| AXI_RESP_B | s_bready && more_beats | BURST_NEXT |
| AXI_RESP_R | s_rready && !more_beats | IDLE |
| AXI_RESP_R | s_rready && more_beats | BURST_NEXT |
| BURST_NEXT | always | APB_SETUP |

## 13.5	3.4.5 Burst Handling

### 13.5.1	Burst Decomposition

AXI4 bursts are decomposed into sequential APB transfers:

AXI4: AWADDR=0x1000, AWLEN=3 (4 beats)

APB sequence:
  Transfer 0: PADDR=0x1000
  Transfer 1: PADDR=0x1004
  Transfer 2: PADDR=0x1008
  Transfer 3: PADDR=0x100C

### 13.5.2	Address Calculation

// Calculate next address for INCR burst
logic [APB_ADDR_WIDTH-1:0] r_current_addr;
logic [2:0] r_awsize;

always_ff @(posedge clk) begin
    if (r_state == APB_ACCESS && pready) begin
        r_current_addr <= r_current_addr + (1 << r_awsize);
    end
end

## 13.6	3.4.6 Address Width Adaptation

### 13.6.1	64-bit to 32-bit Conversion

// Truncate upper address bits
assign paddr = s_awaddr[APB_ADDR_WIDTH-1:0];

// Optional: Check for out-of-range access
wire w_addr_oor = |s_awaddr[AXI_ADDR_WIDTH-1:APB_ADDR_WIDTH];

## 13.7	3.4.7 Error Response Mapping

Table 3.18: Error Mapping

| APB Signal | AXI4 Response |
| --- | --- |
| PSLVERR = 0 | OKAY (2’b00) |
| PSLVERR = 1 | SLVERR (2’b10) |

### 13.7.1	Error Aggregation

// Track worst error in burst
logic r_error_seen;

always_ff @(posedge clk) begin
    if (r_state == IDLE)
        r_error_seen <= 1'b0;
    else if (r_state == APB_ACCESS && pready && pslverr)
        r_error_seen <= 1'b1;
end

// Final response
assign s_bresp = r_error_seen ? 2'b10 : 2'b00;

## 13.8	3.4.8 Implementation

### 13.8.1	Core FSM

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
        r_is_write <= 1'b0;
    end else begin
        case (r_state)
            IDLE: begin
                if (s_awvalid && s_wvalid) begin
                    r_state <= APB_SETUP;
                    r_is_write <= 1'b1;
                    r_current_addr <= s_awaddr[APB_ADDR_WIDTH-1:0];
                    r_beat_count <= '0;
                    r_awlen <= s_awlen;
                    r_awid <= s_awid;
                end else if (s_arvalid) begin
                    r_state <= APB_SETUP;
                    r_is_write <= 1'b0;
                    r_current_addr <= s_araddr[APB_ADDR_WIDTH-1:0];
                    r_beat_count <= '0;
                    r_arlen <= s_arlen;
                    r_arid <= s_arid;
                end
            end

            APB_SETUP: begin
                r_state <= APB_ACCESS;
            end

            APB_ACCESS: begin
                if (pready) begin
                    r_rdata_saved <= prdata;
                    r_error_seen <= r_error_seen || pslverr;
                    if (r_is_write)
                        r_state <= AXI_RESP_B;
                    else
                        r_state <= AXI_RESP_R;
                end
            end

            AXI_RESP_B: begin
                if (s_bready) begin
                    if (r_beat_count == r_awlen)
                        r_state <= IDLE;
                    else
                        r_state <= BURST_NEXT;
                end
            end

            AXI_RESP_R: begin
                if (s_rready) begin
                    if (r_beat_count == r_arlen)
                        r_state <= IDLE;
                    else
                        r_state <= BURST_NEXT;
                end
            end

            BURST_NEXT: begin
                r_beat_count <= r_beat_count + 1;
                r_current_addr <= r_current_addr + (1 << r_size);
                r_state <= APB_SETUP;
            end
        endcase
    end
end

## 13.9	3.4.9 Resource Utilization

State machine:        ~50 LUTs, ~20 regs
Address logic:        ~30 LUTs, ~40 regs
Data buffering:       ~10 LUTs, ~70 regs
Control:              ~60 LUTs, ~20 regs

Total: ~150 LUTs, ~150 regs

## 13.10	3.4.10 Performance

### 13.10.1	Timing Analysis

Table 3.19: APB Converter Timing

| Operation | Cycles |
| --- | --- |
| Single write | 3-4 (setup + access + B) |
| Single read | 3-4 (setup + access + R) |
| N-beat write burst | 3N + 1 |
| N-beat read burst | 3N + 1 |

### 13.10.2	Throughput

Best case: 1 transfer per 3 cycles With slow PREADY: Additional cycles per transfer

## 13.11	3.4.11 Usage Example

axi4_to_apb4_convert #(
    .AXI_ADDR_WIDTH(64),
    .AXI_DATA_WIDTH(32),
    .AXI_ID_WIDTH(4),
    .APB_ADDR_WIDTH(32),
    .APB_DATA_WIDTH(32)
) u_axi2apb4 (
    .clk     (aclk),
    .rst_n   (aresetn),

    // AXI4 slave (from CPU)
    .s_awvalid (cpu_awvalid),
    .s_awready (cpu_awready),
    // ... other AXI4 signals

    // APB master (to peripherals)
    .psel    (uart_psel),
    .penable (uart_penable),
    .pwrite  (uart_pwrite),
    .paddr   (uart_paddr),
    .pwdata  (uart_pwdata),
    .pready  (uart_pready),
    .prdata  (uart_prdata),
    .pslverr (uart_pslverr)
);

Next: PeakRDL Adapter

# 14	AXI4 to APB5 Shim

Module: axi4_to_apb5_shim.sv Filelist: rtl/filelists/axi4_to_apb5_shim.f (-f’s the APB4 shim’s closure)

## 14.1	Design

APB5 keeps the APB4 transfer protocol — PSEL/PENABLE phases, PREADY, PSLVERR are unchanged — and adds only sideband: requester-driven user signals and completer-driven wakeup/user signals. So this block is a thin pin-superset wrapper over axi4_to_apb4_shim: the entire protocol engine (CDC, cmd/rsp FIFOs, burst decomposition, response assembly) is the APB4 shim instantiated unchanged, with all 58 ports and 16 parameters forwarded 1:1.

The APB5 additions on the requester surface:

| Signal | Dir | Handling |
| --- | --- | --- |
| m_apb_PAUSER[APB_AUSER_WIDTH-1:0] | out | tied '0 — nothing upstream sources it (AXI USER bits do not map onto APB user semantics) |
| m_apb_PWUSER[APB_WUSER_WIDTH-1:0] | out | tied '0 |
| m_apb_PWAKEUP | in | accepted and terminated |
| m_apb_PRUSER[APB_RUSER_WIDTH-1:0] | in | accepted and terminated |
| m_apb_PBUSER[APB_BUSER_WIDTH-1:0] | in | accepted and terminated |

User-signal widths default to 1 and match rtl/amba/apb5/apb5_slave.sv, which this requester drops onto pin-for-pin (including the repo’s convention that PWAKEUP rides completer→requester).

## 14.2	Rationale

Deriving the wrapper from the APB4 shim’s actual port surface (rather than reimplementing the conversion) means the APB4 engine’s fixes and its dependency closure are inherited automatically — the closure filelist simply -f’s the APB4 shim’s. When a future consumer needs real PAUSER/PWUSER sourcing or PWAKEUP-gated clocking, those grow here without touching the conversion core.

## 14.3	Consumers

The bridge generator instantiates this shim for protocol = "apb5" slaves (BRIDGE-002 A5-3c) through the same component path as the APB4 shim — the Axi4ToApbShim component takes protocol='apb5' and wires the five extra pairs. Verified by projects/components/bridge/dv/tests/test_bridge_1x2_rw_apb5.py (APB4 BFM legally drives the port: same transfer protocol).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	3.5 PeakRDL Adapter

The peakrdl_to_cmdrsp module adapts PeakRDL-generated register interfaces to a custom command/response protocol, enabling protocol decoupling and flexible register implementations.

## 15.1	3.5.1 Purpose

PeakRDL generates register blocks with an APB-style interface. This adapter:

- Decouples register interface from implementation

- Provides clean handshake protocol

- Enables pipelined register access

- Supports custom control logic integration

## 15.2	3.5.2 Block Diagram

### 15.2.1	Figure 3.8: PeakRDL Adapter

PeakRDL Adapter

## 15.3	3.5.3 Interface Specification

### 15.3.1	Parameters

Table 3.20: PeakRDL Adapter Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | Address width |
| DATA_WIDTH | int | 32 | Data width |

### 15.3.2	Ports

module peakrdl_to_cmdrsp #(
    parameter int ADDR_WIDTH = 32,
    parameter int DATA_WIDTH = 32
) (
    input  logic clk,
    input  logic rst_n,

    // Register interface (from PeakRDL)
    input  logic [ADDR_WIDTH-1:0]   reg_addr,
    input  logic [DATA_WIDTH-1:0]   reg_wdata,
    input  logic                    reg_write,
    input  logic                    reg_read,
    output logic [DATA_WIDTH-1:0]   reg_rdata,
    output logic                    reg_error,
    output logic                    reg_ack,

    // Command interface (output)
    output logic                    cmd_valid,
    input  logic                    cmd_ready,
    output logic [ADDR_WIDTH-1:0]   cmd_addr,
    output logic [DATA_WIDTH-1:0]   cmd_wdata,
    output logic                    cmd_write,

    // Response interface (input)
    input  logic                    rsp_valid,
    output logic                    rsp_ready,
    input  logic [DATA_WIDTH-1:0]   rsp_rdata,
    input  logic                    rsp_error
);

## 15.4	3.5.4 Operation

### 15.4.1	Write Transaction

Cycle 0: reg_write asserted
         cmd_valid = 1, cmd_write = 1
Cycle 1: cmd_ready = 1 (downstream accepts)
         Wait for response
Cycle N: rsp_valid = 1
         reg_ack = 1
Cycle N+1: Transaction complete

### 15.4.2	Read Transaction

Cycle 0: reg_read asserted
         cmd_valid = 1, cmd_write = 0
Cycle 1: cmd_ready = 1 (downstream accepts)
         Wait for response
Cycle N: rsp_valid = 1
         reg_rdata = rsp_rdata
         reg_ack = 1
Cycle N+1: Transaction complete

## 15.5	3.5.5 Implementation

// State machine
typedef enum logic [1:0] {
    IDLE    = 2'b00,
    CMD     = 2'b01,
    RSP     = 2'b10
} state_t;

state_t r_state;
logic r_is_write;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
    end else begin
        case (r_state)
            IDLE: begin
                if (reg_write || reg_read) begin
                    r_state <= CMD;
                    r_is_write <= reg_write;
                end
            end

            CMD: begin
                if (cmd_ready) begin
                    r_state <= RSP;
                end
            end

            RSP: begin
                if (rsp_valid) begin
                    r_state <= IDLE;
                end
            end
        endcase
    end
end

// Command interface
assign cmd_valid = (r_state == CMD);
assign cmd_addr = reg_addr;
assign cmd_wdata = reg_wdata;
assign cmd_write = r_is_write;

// Response interface
assign rsp_ready = (r_state == RSP);

// Register interface
assign reg_rdata = rsp_rdata;
assign reg_error = rsp_error;
assign reg_ack = (r_state == RSP) && rsp_valid;

## 15.6	3.5.6 Resource Utilization

State machine:  ~20 LUTs, ~10 regs
Data paths:     ~10 LUTs, ~40 regs
Control:        ~20 LUTs, ~5 regs

Total: ~50 LUTs, ~55 regs

## 15.7	3.5.7 Use Cases

### 15.7.1	1. PeakRDL to Custom Control

PeakRDL Registers → Adapter → Custom State Machine
                            → Hardware Accelerator
                            → Debug Controller

### 15.7.2	2. Register Access Logging

PeakRDL Registers → Adapter → Logger → Registers
                            ↓
                         Log Buffer

### 15.7.3	3. Pipeline Insertion

PeakRDL Registers → Adapter → Pipeline → Slow Registers
                             (for timing)

## 15.8	3.5.8 Integration Example

// Instantiate PeakRDL-generated register block
my_regs u_regs (
    .clk        (clk),
    .rst_n      (rst_n),

    // APB-style interface from CPU
    .s_apb_psel    (apb_psel),
    .s_apb_penable (apb_penable),
    // ... other APB signals

    // Register interface to adapter
    .reg_addr   (reg_addr),
    .reg_wdata  (reg_wdata),
    .reg_write  (reg_write),
    .reg_read   (reg_read),
    .reg_rdata  (reg_rdata),
    .reg_error  (reg_error),
    .reg_ack    (reg_ack)
);

// Adapter to custom protocol
peakrdl_to_cmdrsp #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32)
) u_adapter (
    .clk        (clk),
    .rst_n      (rst_n),

    // From PeakRDL registers
    .reg_addr   (reg_addr),
    .reg_wdata  (reg_wdata),
    .reg_write  (reg_write),
    .reg_read   (reg_read),
    .reg_rdata  (reg_rdata),
    .reg_error  (reg_error),
    .reg_ack    (reg_ack),

    // To custom control logic
    .cmd_valid  (ctrl_cmd_valid),
    .cmd_ready  (ctrl_cmd_ready),
    .cmd_addr   (ctrl_cmd_addr),
    .cmd_wdata  (ctrl_cmd_wdata),
    .cmd_write  (ctrl_cmd_write),

    .rsp_valid  (ctrl_rsp_valid),
    .rsp_ready  (ctrl_rsp_ready),
    .rsp_rdata  (ctrl_rsp_rdata),
    .rsp_error  (ctrl_rsp_error)
);

Next: Chapter 4: FSM Design

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	4.1 Width Converter FSMs

This section describes the state machines used in data width converter modules.

## 16.1	4.1.1 Upsize FSM

The axi_data_upsize module uses a simple accumulation state machine.

### 16.1.1	Figure 4.1: Upsize FSM

Upsize FSM

### 16.1.2	States

Table 4.1: Upsize FSM States

| State | Description |
| --- | --- |
| ACCUMULATE | Collecting narrow beats into buffer |
| OUTPUT | Buffer full, outputting wide beat |

### 16.1.3	Transitions

ACCUMULATE:
  - s_valid && count < RATIO-1 → stay, increment count
  - s_valid && (count == RATIO-1 || s_last) → OUTPUT

OUTPUT:
  - m_ready → ACCUMULATE, reset count
  - !m_ready → stay

### 16.1.4	Implementation

typedef enum logic {
    ACCUMULATE = 1'b0,
    OUTPUT     = 1'b1
} upsize_state_t;

upsize_state_t r_state;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= ACCUMULATE;
    end else begin
        case (r_state)
            ACCUMULATE: begin
                if (s_valid && s_ready) begin
                    if (s_last || r_count == RATIO - 1)
                        r_state <= OUTPUT;
                end
            end

            OUTPUT: begin
                if (m_ready)
                    r_state <= ACCUMULATE;
            end
        endcase
    end
end

## 16.2	4.1.2 Downsize FSM (Single Buffer)

The axi_data_dnsize single-buffer mode uses a load/output state machine.

### 16.2.1	Figure 4.2: Downsize Single-Buffer FSM

Downsize FSM

### 16.2.2	States

Table 4.2: Downsize FSM States

| State | Description |
| --- | --- |
| IDLE | Waiting for wide input |
| LOAD | Loading wide beat |
| OUTPUT | Outputting narrow beats |

### 16.2.3	Transitions

IDLE:
  - s_valid → LOAD

LOAD:
  - always → OUTPUT (combinational)

OUTPUT:
  - m_ready && count < RATIO-1 → stay, increment count
  - m_ready && count == RATIO-1 → IDLE

## 16.3	4.1.3 Downsize FSM (Dual Buffer)

Dual-buffer mode uses two parallel state machines with an arbiter.

### 16.3.1	Buffer State Machine

typedef enum logic [1:0] {
    BUF_EMPTY     = 2'b00,
    BUF_LOADED    = 2'b01,
    BUF_OUTPUTTING = 2'b10
} buf_state_t;

buf_state_t r_buf_a_state, r_buf_b_state;

### 16.3.2	Arbiter Logic

// Select which buffer outputs
always_comb begin
    if (r_buf_a_state == BUF_OUTPUTTING)
        output_sel = 1'b0;  // Buffer A
    else if (r_buf_b_state == BUF_OUTPUTTING)
        output_sel = 1'b1;  // Buffer B
    else if (r_buf_a_state == BUF_LOADED)
        output_sel = 1'b0;  // A loaded first
    else
        output_sel = 1'b1;  // B loaded first
end

// Select which buffer loads
always_comb begin
    if (r_buf_a_state == BUF_EMPTY)
        load_sel = 1'b0;
    else
        load_sel = 1'b1;
end

## 16.4	4.1.4 Full Converter FSMs

### 16.4.1	Write Converter (axi4_dwidth_converter_wr)

IDLE:
  - AW valid → accept AW, store info
  - W valid → buffer W data

AW_ACCEPT:
  - downstream AW ready → forward adjusted AW

W_CONVERT:
  - upsize accumulating narrow W beats
  - on output → forward wide W beat

B_FORWARD:
  - B from downstream → forward to master

### 16.4.2	Read Converter (axi4_dwidth_converter_rd)

IDLE:
  - AR valid → accept AR, store info, forward adjusted AR

AR_FORWARD:
  - downstream AR ready → wait for R

R_CONVERT:
  - downsize splitting wide R into narrow beats
  - track burst count for RLAST

R_FORWARD:
  - narrow R beat ready → forward to master
  - on RLAST → IDLE

## 16.5	4.1.5 Timing Diagrams

### 16.5.1	Upsize Timing (8:1 ratio)

clk     __|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|
s_valid ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯|_______|¯¯¯¯¯
s_data    D0   D1   D2   D3   D4   D5   D6   D7       -     D8
s_ready ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯|_______|¯¯¯¯¯¯¯¯¯¯¯¯¯
m_valid ____________________________________________|¯¯¯¯¯¯¯|______
m_data                                              WIDE0
m_ready ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯

### 16.5.2	Downsize Timing (8:1 ratio, single buffer)

clk     __|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|
s_valid ¯¯¯¯¯¯¯|_______________________________________________|¯¯¯
s_data    WIDE0                                                 WIDE1
s_ready ¯¯¯¯¯¯¯|_______________________________________________|¯¯¯
m_valid ________|¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯|___
m_data          D0   D1   D2   D3   D4   D5   D6   D7
m_ready ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯

Next: Protocol Converter FSMs

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	4.2 Protocol Converter FSMs

This section describes the state machines used in protocol converter modules.

## 17.1	4.2.1 AXI4 to AXI4-Lite Read FSM

### 17.1.1	Figure 4.3: AXI4 to AXIL4 Read FSM

AXI4 to AXIL4 Read FSM

AXI4 to AXIL4 Read FSM

### 17.1.2	States

Table 4.3: AXI4 to AXIL4 Read FSM States

| State | Description |
| --- | --- |
| IDLE | Waiting for AR transaction |
| SINGLE | Single-beat passthrough |
| DECOMPOSE | Issuing burst as single beats |
| WAIT_R | Waiting for final R response |

### 17.1.3	Transitions

IDLE:
  - s_arvalid && s_arlen == 0 → SINGLE (passthrough)
  - s_arvalid && s_arlen > 0 → DECOMPOSE

SINGLE:
  - m_arready → forward AR, wait for R
  - m_rvalid && m_rready → IDLE

DECOMPOSE:
  - m_arready → issue single AR
  - increment address, decrement remaining
  - remaining == 0 → WAIT_R

WAIT_R:
  - s_rvalid && s_rready && s_rlast → IDLE

### 17.1.4	Implementation

typedef enum logic [1:0] {
    IDLE      = 2'b00,
    SINGLE    = 2'b01,
    DECOMPOSE = 2'b10,
    WAIT_R    = 2'b11
} rd_state_t;

rd_state_t r_state;
logic [7:0] r_beats_remaining;
logic [ADDR_WIDTH-1:0] r_current_addr;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
    end else begin
        case (r_state)
            IDLE: begin
                if (s_arvalid && s_arready) begin
                    r_current_addr <= s_araddr;
                    r_beats_remaining <= s_arlen;
                    r_state <= (s_arlen == 0) ? SINGLE : DECOMPOSE;
                end
            end

            SINGLE: begin
                if (m_rvalid && s_rready)
                    r_state <= IDLE;
            end

            DECOMPOSE: begin
                if (m_arvalid && m_arready) begin
                    r_current_addr <= r_current_addr + (1 << r_arsize);
                    if (r_beats_remaining == 0)
                        r_state <= WAIT_R;
                    else
                        r_beats_remaining <= r_beats_remaining - 1;
                end
            end

            WAIT_R: begin
                if (s_rvalid && s_rready && s_rlast)
                    r_state <= IDLE;
            end
        endcase
    end
end

## 17.2	4.2.2 AXI4 to AXI4-Lite Write FSM

### 17.2.1	States

Table 4.4: AXI4 to AXIL4 Write FSM States

| State | Description |
| --- | --- |
| IDLE | Waiting for AW/W transactions |
| SYNC_AW_W | Synchronizing AW and W channels |
| DECOMPOSE | Issuing burst as single beats |
| WAIT_B | Waiting for all B responses |
| SEND_B | Sending aggregated B response |

### 17.2.2	AW/W Synchronization

// Track which channels have been accepted
logic r_aw_pending, r_w_pending;

always_ff @(posedge clk) begin
    // Accept AW
    if (s_awvalid && s_awready && !r_aw_pending)
        r_aw_pending <= 1'b1;

    // Accept W
    if (s_wvalid && s_wready && !r_w_pending)
        r_w_pending <= 1'b1;

    // Issue AXIL4 when both ready
    if (w_issue_axil4) begin
        r_aw_pending <= 1'b0;
        r_w_pending <= 1'b0;
    end
end

assign w_issue_axil4 = (r_aw_pending || s_awvalid) &&
                       (r_w_pending || s_wvalid) &&
                       m_awready && m_wready;

## 17.3	4.2.3 AXI4 to APB FSM

### 17.3.1	Figure 4.4: AXI4 to APB FSM

AXI4 to APB FSM

### 17.3.2	States

Table 4.5: AXI4 to APB FSM States

| State | Description |
| --- | --- |
| IDLE | Waiting for AXI4 transaction |
| APB_SETUP | APB setup phase (PSEL=1, PENABLE=0) |
| APB_ACCESS | APB access phase (PSEL=1, PENABLE=1) |
| AXI_RESP | Sending AXI4 response |
| BURST_NEXT | Preparing next beat in burst |

### 17.3.3	APB Protocol Phases

Setup Phase:
  - PSEL = 1
  - PENABLE = 0
  - PADDR, PWDATA, PWRITE stable
  - Duration: 1 cycle

Access Phase:
  - PSEL = 1
  - PENABLE = 1
  - Wait for PREADY
  - Sample PRDATA (read) or complete write
  - Duration: 1+ cycles (depends on PREADY)

### 17.3.4	Implementation

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
        psel <= 1'b0;
        penable <= 1'b0;
    end else begin
        case (r_state)
            IDLE: begin
                psel <= 1'b0;
                penable <= 1'b0;
                if (s_awvalid || s_arvalid) begin
                    r_state <= APB_SETUP;
                    psel <= 1'b1;
                end
            end

            APB_SETUP: begin
                r_state <= APB_ACCESS;
                penable <= 1'b1;
            end

            APB_ACCESS: begin
                if (pready) begin
                    psel <= 1'b0;
                    penable <= 1'b0;
                    r_state <= AXI_RESP;
                end
            end

            AXI_RESP: begin
                if ((r_is_write && s_bready) ||
                    (!r_is_write && s_rready)) begin
                    if (r_beat_count == r_burst_len)
                        r_state <= IDLE;
                    else
                        r_state <= BURST_NEXT;
                end
            end

            BURST_NEXT: begin
                r_beat_count <= r_beat_count + 1;
                r_current_addr <= r_current_addr + (1 << r_size);
                r_state <= APB_SETUP;
                psel <= 1'b1;
            end
        endcase
    end
end

## 17.4	4.2.4 Timing Analysis

### 17.4.1	AXI4 to AXIL4 Timing

Single-beat transaction:

Cycle 0: AR accepted
Cycle 1: AR forwarded to AXIL4
Cycle 2: R received from AXIL4
Cycle 3: R forwarded to AXI4
Total: ~2 cycles overhead (can be pipelined)

N-beat burst:

Cycle 0: AR accepted
Cycles 1-2N: Decomposed transactions
Total: 2N cycles

### 17.4.2	AXI4 to APB Timing

Single transfer:

Cycle 0: AXI4 AR/AW accepted
Cycle 1: APB setup phase
Cycle 2+: APB access phase (wait PREADY)
Cycle N: APB complete, AXI4 response
Total: 3+ cycles (minimum)

Next: Burst Decomposition

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	4.3 Burst Decomposition

This section describes how converters decompose AXI4 bursts into smaller transactions.

## 18.1	4.3.1 Width Converter Burst Handling

### 18.1.1	Burst Length Adjustment

When converting widths, burst length changes inversely with data width:

M_AWLEN = (S_AWLEN + 1) / RATIO - 1

Example (64-bit to 512-bit, RATIO=8):
  S_AWLEN = 7 (8 beats × 64 bits = 512 bits)
  M_AWLEN = (7 + 1) / 8 - 1 = 0 (1 beat × 512 bits)

  S_AWLEN = 15 (16 beats × 64 bits = 1024 bits)
  M_AWLEN = (15 + 1) / 8 - 1 = 1 (2 beats × 512 bits)

### 18.1.2	Figure 4.5: Width Burst Conversion

Width Burst Conversion

Width Burst Conversion

### 18.1.3	Non-Aligned Bursts

When burst length is not a multiple of ratio:

S_AWLEN = 5 (6 beats), RATIO = 8
M_AWLEN = (5 + 1) / 8 - 1 = -1 → 0 (1 beat)

The 6 narrow beats pack into 1 wide beat.
Last 2 positions have WSTRB = 0 (no write).

## 18.2	4.3.2 Protocol Converter Burst Handling

### 18.2.1	AXI4 to AXI4-Lite Decomposition

AXI4-Lite only supports single-beat transactions, so all bursts must be decomposed:

AXI4 Burst:
  ARADDR = 0x1000
  ARLEN = 3 (4 beats)
  ARSIZE = 2 (4 bytes)

AXIL4 Sequence:
  Transaction 0: ARADDR = 0x1000
  Transaction 1: ARADDR = 0x1004
  Transaction 2: ARADDR = 0x1008
  Transaction 3: ARADDR = 0x100C

### 18.2.2	Address Increment Calculation

// Calculate address increment based on burst type and size
function automatic [ADDR_WIDTH-1:0] next_address(
    input [ADDR_WIDTH-1:0] current_addr,
    input [2:0] size,
    input [1:0] burst,
    input [7:0] len,
    input [7:0] beat
);
    logic [ADDR_WIDTH-1:0] increment;
    logic [ADDR_WIDTH-1:0] wrap_mask;

    increment = 1 << size;

    case (burst)
        2'b00: // FIXED
            return current_addr;  // No increment

        2'b01: // INCR
            return current_addr + increment;

        2'b10: // WRAP
            wrap_mask = ((len + 1) << size) - 1;
            return (current_addr & ~wrap_mask) |
                   ((current_addr + increment) & wrap_mask);

        default:
            return current_addr + increment;
    endcase
endfunction

## 18.3	4.3.3 Response Aggregation

### 18.3.1	Read Response Aggregation

For burst reads decomposed into multiple single reads:

// Track responses as they arrive
logic [7:0] r_response_count;
logic [1:0] r_worst_rresp;

always_ff @(posedge clk) begin
    if (start_new_burst) begin
        r_response_count <= '0;
        r_worst_rresp <= 2'b00;  // OKAY
    end else if (m_rvalid && m_rready) begin
        r_response_count <= r_response_count + 1;
        // Keep worst response
        if (m_rresp > r_worst_rresp)
            r_worst_rresp <= m_rresp;
    end
end

// Generate RLAST on final beat
assign s_rlast = (r_response_count == r_original_arlen);

// Forward individual responses or aggregate
assign s_rresp = m_rresp;  // Forward each response
// Or: assign s_rresp = r_worst_rresp; // Aggregate

### 18.3.2	Write Response Aggregation

For burst writes:

// Track worst response across burst
logic [1:0] r_worst_bresp;
logic r_all_beats_done;

always_ff @(posedge clk) begin
    if (start_new_burst)
        r_worst_bresp <= 2'b00;
    else if (m_bvalid && m_bready)
        r_worst_bresp <= (m_bresp > r_worst_bresp) ? m_bresp : r_worst_bresp;
end

// Send single aggregated B response
assign s_bvalid = r_all_beats_done;
assign s_bresp = r_worst_bresp;

## 18.4	4.3.4 Burst Tracking Registers

### 18.4.1	Required State

// Burst tracking registers
logic [ADDR_WIDTH-1:0] r_base_addr;
logic [ADDR_WIDTH-1:0] r_current_addr;
logic [7:0]            r_original_len;
logic [7:0]            r_remaining_beats;
logic [2:0]            r_size;
logic [1:0]            r_burst;
logic [ID_WIDTH-1:0]   r_id;
logic                  r_is_write;

### 18.4.2	Initialization

always_ff @(posedge clk) begin
    if (accept_new_transaction) begin
        r_base_addr <= s_axaddr;
        r_current_addr <= s_axaddr;
        r_original_len <= s_axlen;
        r_remaining_beats <= s_axlen;
        r_size <= s_axsize;
        r_burst <= s_axburst;
        r_id <= s_axid;
        r_is_write <= is_write_transaction;
    end else if (beat_complete) begin
        r_current_addr <= next_address(...);
        r_remaining_beats <= r_remaining_beats - 1;
    end
end

## 18.5	4.3.5 Timing Impact

### 18.5.1	Decomposition Overhead

Table 4.6: Decomposition Overhead

| Transaction Type | Overhead |
| --- | --- |
| Single-beat | 0 cycles (passthrough) |
| 2-beat burst | 2 cycles (sequential) |
| N-beat burst | 2N cycles (2 per beat) |

### 18.5.2	Pipeline Considerations

Decomposition is inherently sequential: - Cannot issue next AR until previous R received (AXIL4) - Cannot issue next AW/W until previous B received (AXIL4)

Optimization: Use response pipelining when downstream supports it.

Next: Chapter 5: Verification

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	5.1 Test Strategy

This section describes the verification approach for converter modules.

## 19.1	5.1.1 Test Organization

### 19.1.1	Test Hierarchy

projects/components/converters/dv/tests/
├── width_converters/
│   ├── test_axi_data_upsize.py
│   ├── test_axi_data_dnsize.py
│   ├── test_axi4_dwidth_converter_wr.py
│   └── test_axi4_dwidth_converter_rd.py
├── protocol_converters/
│   ├── test_axi4_to_axil4_rd.py
│   ├── test_axi4_to_axil4_wr.py
│   ├── test_axil4_to_axi4_rd.py
│   ├── test_axil4_to_axi4_wr.py
│   ├── test_axi4_to_apb4.py
│   └── test_peakrdl_adapter.py
└── integration/
    ├── test_width_protocol_chain.py
    └── test_full_system.py

## 19.2	5.1.2 Test Levels

### 19.2.1	Level 1: Unit Tests

Purpose: Verify individual module functionality in isolation.

Coverage: - All parameters combinations - Edge cases (min/max values) - Error injection

Example:

@pytest.mark.parametrize("narrow_width,wide_width", [
    (32, 64),
    (32, 128),
    (64, 256),
    (64, 512),
    (128, 1024),
])
async def test_upsize_ratios(dut, narrow_width, wide_width):
    """Test various width ratios."""
    tb = UpsizeTB(dut, narrow_width, wide_width)
    await tb.reset()
    await tb.run_basic_transfer(count=100)
    assert tb.scoreboard.check_passed()

### 19.2.2	Level 2: Integration Tests

Purpose: Verify module combinations work together.

Coverage: - Width converter + protocol converter chains - Back-to-back converters - Mixed traffic patterns

### 19.2.3	Level 3: System Tests

Purpose: Verify in realistic system context.

Coverage: - CPU-to-DDR paths - Peripheral access paths - Full bandwidth stress

## 19.3	5.1.3 Test Categories

### 19.3.1	Functional Tests

Table 5.1: Functional Test Categories

| Category | Description | Example |
| --- | --- | --- |
| Basic | Single transactions | Single read, single write |
| Burst | Multi-beat transactions | INCR burst, WRAP burst |
| Mixed | Interleaved R/W | Read-modify-write sequences |
| Edge | Boundary conditions | Max burst length, min width |

### 19.3.2	Stress Tests

Table 5.2: Stress Test Categories

| Category | Description | Duration |
| --- | --- | --- |
| Throughput | Maximum bandwidth | 10,000 transactions |
| Backpressure | Ready signal variations | Random delays |
| Reset | Reset during operation | Mid-transaction reset |

### 19.3.3	Error Tests

Table 5.3: Error Test Categories

| Category | Description | Expected Behavior |
| --- | --- | --- |
| SLVERR | Slave error injection | Error propagation |
| DECERR | Decode error | Error response |
| Timeout | Response timeout | Error handling |

## 19.4	5.1.4 Testbench Architecture

### 19.4.1	Components

Testbench
├── Drivers
│   ├── AXI4 Master Driver
│   ├── AXI4 Slave Driver
│   ├── AXIL4 Master/Slave
│   └── APB Master/Slave
├── Monitors
│   ├── AXI4 Monitor
│   ├── AXIL4 Monitor
│   └── APB Monitor
├── Scoreboard
│   ├── Transaction Queue
│   ├── Response Checker
│   └── Coverage Collector
└── Generators
    ├── Random Transaction Generator
    └── Directed Sequence Generator

### 19.4.2	Driver Implementation

class AXI4MasterDriver:
    def __init__(self, dut, clock, prefix="s_axi"):
        self.dut = dut
        self.clock = clock
        self.prefix = prefix

    async def write(self, addr, data, burst_len=0):
        """Issue AXI4 write transaction."""
        # AW phase
        self.dut.awvalid.value = 1
        self.dut.awaddr.value = addr
        self.dut.awlen.value = burst_len
        await self._wait_ready("awready")

        # W phase
        for i, d in enumerate(data):
            self.dut.wvalid.value = 1
            self.dut.wdata.value = d
            self.dut.wlast.value = (i == len(data) - 1)
            await self._wait_ready("wready")

        # B phase
        await self._wait_valid("bvalid")
        return self.dut.bresp.value

## 19.5	5.1.5 Coverage Model

### 19.5.1	Functional Coverage

@coverage
class ConverterCoverage:
    # Width ratio coverage
    ratio_cp = coverpoint(
        lambda: self.width_ratio,
        bins=[2, 4, 8, 16]
    )

    # Burst length coverage
    burst_len_cp = coverpoint(
        lambda: self.burst_len,
        bins=list(range(0, 256, 16)) + [255]
    )

    # Burst type coverage
    burst_type_cp = coverpoint(
        lambda: self.burst_type,
        bins=["FIXED", "INCR", "WRAP"]
    )

    # Cross coverage
    ratio_x_burst = cross(ratio_cp, burst_len_cp)

### 19.5.2	Code Coverage

Target: >95% line coverage, >90% branch coverage

Table 5.4: Coverage Targets

| Module | Line | Branch | FSM |
| --- | --- | --- | --- |
| axi_data_upsize | 98% | 95% | 100% |
| axi_data_dnsize | 97% | 93% | 100% |
| axi4_to_axil4_rd | 96% | 91% | 100% |
| axi4_to_axil4_wr | 95% | 90% | 100% |

## 19.6	5.1.6 Test Execution

### 19.6.1	Running Tests

# Run all converter tests
cd projects/components/converters/dv/tests
pytest -v

# Run specific module tests
pytest test_axi_data_upsize.py -v

# Run with coverage
pytest --cov=projects/components/converters/rtl -v

# Run with waveform dump
pytest test_axi_data_upsize.py -v --waves

### 19.6.2	CI/CD Integration

converter_tests:
  stage: test
  script:
    - cd projects/components/converters/dv/tests
    - pytest -v --junitxml=results.xml
    - pytest --cov=rtl --cov-report=html
  artifacts:
    reports:
      junit: results.xml
    paths:
      - htmlcov/

Next: Debug Guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	5.2 Debug Guide

This section provides debugging guidance for converter module issues.

## 20.1	5.2.1 Common Issues

### 20.1.1	Width Converter Issues

#### 20.1.1.1	Issue: Data Corruption

Symptoms: - Output data doesn’t match expected - Random bits flipped or missing

Debug Steps: 1. Check width ratio calculation: RATIO = WIDE_WIDTH / NARROW_WIDTH 2. Verify beat counter is correct: $clog2(RATIO) bits 3. Check data packing/unpacking slice indices 4. Verify sideband mode matches use case

Waveform Checkpoints:

r_count         - Should cycle 0 to RATIO-1
r_data          - Check each slice is populated
s_data/m_data   - Compare input/output patterns

#### 20.1.1.2	Issue: LAST Signal Incorrect

Symptoms: - Transaction ends early or late - Master receives wrong beat count

Debug Steps: 1. Check USE_LAST parameter 2. Verify burst tracker logic if enabled 3. Check s_last input timing

Solution:

// Verify LAST generation
assign m_last = (r_count == RATIO - 1) || r_input_last_seen;

#### 20.1.1.3	Issue: Throughput Lower Than Expected

Symptoms: - Gaps between output beats - Single-buffer achieving <80%

Debug Steps: 1. Check downstream ready signal behavior 2. Verify DUAL_BUFFER parameter for downsize 3. Look for backpressure stalls

### 20.1.2	Protocol Converter Issues

#### 20.1.2.1	Issue: Burst Decomposition Incorrect

Symptoms: - Wrong number of single transactions - Address increment wrong

Debug Steps: 1. Check burst type (FIXED, INCR, WRAP) 2. Verify size calculation 3. Check address increment logic

Waveform Checkpoints:

r_current_addr  - Should increment by (1 << size)
r_beat_count    - Should count to s_arlen/s_awlen
m_arvalid       - Should assert for each decomposed beat

#### 20.1.2.2	Issue: Response Aggregation Wrong

Symptoms: - Wrong BRESP/RRESP value - Error not propagated correctly

Debug Steps: 1. Check worst-case response tracking 2. Verify response counter 3. Check RLAST generation

Solution:

// Proper error aggregation
always_ff @(posedge clk) begin
    if (new_burst)
        r_worst_resp <= 2'b00;
    else if (m_rvalid && m_rready)
        r_worst_resp <= (m_rresp > r_worst_resp) ? m_rresp : r_worst_resp;
end

## 20.2	5.2.2 Debug Signals

### 20.2.1	Recommended Internal Signals

For width converters:

// Add debug outputs
output logic [$clog2(RATIO)-1:0] dbg_beat_count,
output logic                     dbg_buffer_valid,
output logic [1:0]               dbg_state

For protocol converters:

// Add debug outputs
output logic [7:0]  dbg_remaining_beats,
output logic [2:0]  dbg_state,
output logic [1:0]  dbg_worst_resp,
output logic        dbg_in_burst

### 20.2.2	ILA Configuration

# Create ILA for converter debug
create_debug_core u_ila ila

# Add probes
set_property probe_count 10 [get_debug_cores u_ila]
connect_debug_port u_ila/clk [get_nets aclk]

# Key signals
connect_debug_port u_ila/probe0 [get_nets r_state]
connect_debug_port u_ila/probe1 [get_nets r_beat_count]
connect_debug_port u_ila/probe2 [get_nets s_arvalid]
connect_debug_port u_ila/probe3 [get_nets m_arvalid]

## 20.3	5.2.3 Simulation Debug

### 20.3.1	Waveform Analysis

Key Signal Groups:

- Input Channel:

  - s_valid, s_ready, s_data, s_last

- Output Channel:

  - m_valid, m_ready, m_data, m_last

- Control:

  - r_state, r_count, r_burst_remaining

- Sideband:

  - s_wstrb/m_wstrb (write path)

  - s_rresp/m_rresp (read path)

### 20.3.2	Timing Diagram Template

___     ___     ___     ___     ___
clk     ___|   |___|   |___|   |___|   |___|   |
            _______________________________
s_valid ___/                               \___
           [D0 ][D1 ][D2 ][D3 ][D4 ][D5 ][D6 ][D7 ]
s_data
        ___________________________________________
s_ready

           ________________________________
m_valid __|                                |___
                                   [WIDE_DATA ]
m_data

Check:
1. s_ready stays high during accumulation
2. m_valid asserts after RATIO beats
3. Data packing is correct

## 20.4	5.2.4 Common Mistakes

### 20.4.1	Mistake 1: Wrong Width Ratio

// WRONG: Manual ratio
localparam RATIO = 8;  // May not match actual widths

// CORRECT: Calculated ratio
localparam RATIO = WIDE_WIDTH / NARROW_WIDTH;

### 20.4.2	Mistake 2: Missing Sideband Handling

// WRONG: Forgetting sideband
assign m_data = r_data;
// Missing: assign m_wstrb = ...

// CORRECT: Handle both
assign m_data = r_data;
assign m_wstrb = r_sideband;

### 20.4.3	Mistake 3: Incorrect LAST Timing

// WRONG: LAST on wrong beat
assign m_last = r_count == 0;  // First beat!

// CORRECT: LAST on final beat
assign m_last = (r_count == RATIO - 1) || r_early_last;

### 20.4.4	Mistake 4: Burst Length Calculation Error

// WRONG: Off-by-one
assign m_awlen = s_awlen / RATIO;  // Wrong!

// CORRECT: Account for LEN encoding
assign m_awlen = ((s_awlen + 1) >> RATIO_LOG2) - 1;

## 20.5	5.2.5 Verification Checklist

Before signoff, verify:

- ☐ All parameter combinations tested

- ☐ Single-beat transactions work

- ☐ Multi-beat bursts work (INCR, WRAP, FIXED)

- ☐ Backpressure handling correct

- ☐ Error propagation correct

- ☐ LAST signal timing correct

- ☐ Sideband signals handled correctly

- ☐ Reset behavior verified

- ☐ Coverage targets met

## 20.6	5.2.6 Performance Validation

### 20.6.1	Throughput Measurement

async def measure_throughput(tb, transaction_count=1000):
    start_time = get_sim_time()

    for _ in range(transaction_count):
        await tb.send_transaction()

    end_time = get_sim_time()
    elapsed_cycles = (end_time - start_time) / clock_period

    throughput = transaction_count / elapsed_cycles
    print(f"Throughput: {throughput:.2f} transactions/cycle")

    return throughput

### 20.6.2	Expected Throughput

Table 5.5: Expected Throughput

| Module | Mode | Expected |
| --- | --- | --- |
| axi_data_upsize | Single | 1.0 trans/cycle |
| axi_data_dnsize | Single | 0.8 trans/cycle |
| axi_data_dnsize | Dual | 1.0 trans/cycle |
| axi4_to_axil4 | Single-beat | 1.0 trans/cycle |
| axi4_to_axil4 | Burst | 0.5 trans/cycle |

End of Micro-Architecture Specification