RTL Design Sherpa

Converters Micro-Architecture Specification 1.3

September 11, 2026

Table of Contents

1 Converter Mas Index	22

2 Document Information	23

2.1 Document Control	23

2.2 Purpose	24

2.3 Audience	25

2.4 Related Documents	26

2.5 Revision History	27

2.6 Conventions	28

2.6.1 Notation	28

2.6.2 Diagrams	28

2.6.3 Code Examples	28

3 Chapter 1: Introduction - Overview	29

3.1 1.1 Purpose	29

3.2 1.2 Problem Statement	30

3.2.1 1.2.1 Data Width Mismatch	30

3.2.2 1.2.2 Protocol Incompatibility	30

3.3 1.3 Solution Architecture	31

3.3.1 Figure 1.1: Converter Module Hierarchy	31

3.3.2 1.3.1 Data Width Converters	31

3.3.3 1.3.2 Protocol Converters	31

3.4 1.4 Key Design Decisions	32

3.4.1 1.4.1 Generic vs. Full Modules	32

3.4.2 1.4.2 Throughput vs. Area Trade-offs	32

3.4.3 1.4.3 Sideband Signal Handling	32

3.5 1.5 Performance Characteristics	34

3.5.1 1.5.1 Latency	34

3.5.2 1.5.2 Throughput	34

3.6 1.6 Scope	35

3.6.1 In Scope	35

3.6.2 Out of Scope	35

3.7 1.7 Target Applications	36

4 2.1 Generic Building Blocks	37

4.1 2.1.1 Module Hierarchy	38

4.2 2.1.2 Design Philosophy	39

4.2.1 Separation of Concerns	39

4.2.2 Interface Pattern	39

4.3 2.1.3 Width Ratio Calculation	40

4.4 2.1.4 Sideband Modes	41

4.4.1 Upsize Sideband Modes	41

4.4.2 Downsize Sideband Modes	41

4.5 2.1.5 Performance Comparison	43

4.5.1 Figure 2.1: Performance Comparison	43

4.6 2.1.6 Resource Utilization	44

4.6.1 Upsize Resources (NARROW=64, WIDE=512)	44

4.6.2 Downsize Resources (WIDE=512, NARROW=64)	44

4.7 2.1.7 Integration Guidelines	45

4.7.1 When to Use Generic Blocks	45

4.7.2 Composition Example	45

5 2.2 axi_data_upsize	46

5.1 2.2.1 Overview	47

5.2 2.2.2 Block Diagram	48

5.2.1 Figure 2.2: axi_data_upsize Architecture	48

5.3 2.2.3 Interface Specification	49

5.3.1 Parameters	49

5.3.2 Ports	49

5.4 2.2.4 Operation	51

5.4.1 Accumulation Cycle	51

5.4.2 Early LAST Handling	51

5.4.3 State Machine	51

5.5 2.2.5 Sideband Handling	52

5.5.1 Concatenate Mode (SB_OR_MODE=0)	52

5.5.2 Severity-Fold Mode (SB_OR_MODE=1)	52

5.6 2.2.6 Implementation	54

5.6.1 Core Logic	54

5.7 2.2.7 Timing	56

5.7.1 Latency	56

5.7.2 Throughput	56

5.7.3 Critical Paths	56

5.8 2.2.8 Resource Utilization	57

5.8.1 Typical Resources (64-bit to 512-bit)	57

5.8.2 Scaling	57

5.9 2.2.9 Usage Example	58

6 2.3 axi_data_dnsize	59

6.1 2.3.1 Overview	60

6.2 2.3.2 Block Diagram	61

6.2.1 Figure 2.3: axi_data_dnsize Single-Buffer Architecture	61

6.3 2.3.3 Interface Specification	62

6.3.1 Parameters	62

6.3.2 Ports	62

6.4 2.3.4 Single-Buffer Mode Operation	64

6.4.1 Split Cycle	64

6.4.2 State Machine	64

6.4.3 Throughput Analysis	64

6.5 2.3.5 Sideband Handling	66

6.5.1 Slice Mode (SB_BROADCAST=0)	66

6.5.2 Broadcast Mode (SB_BROADCAST=1)	66

6.6 2.3.6 Burst Tracking	67

6.6.1 Purpose	67

6.6.2 Operation	67

6.7 2.3.7 Implementation	68

6.7.1 Single-Buffer Core Logic	68

6.8 2.3.8 Resource Utilization	69

6.8.1 Single-Buffer (512-bit to 64-bit)	69

6.8.2 Measured Throughput	69

6.9 2.3.9 Usage Example	70

7 2.5 axi4_dwidth_converter_wr	71

7.1 2.5.1 Overview	72

7.2 2.5.2 Block Diagram	73

7.2.1 Figure 2.6: Write Converter Architecture	73

7.3 2.5.3 Interface Specification	74

7.3.1 Parameters	74

7.3.2 Ports	74

7.4 2.5.4 Burst Length Conversion	78

7.4.1 Ratio Calculation	78

7.4.2 Examples	78

7.4.3 Non-Aligned Bursts	78

7.5 2.5.5 Address Channel Handling	79

7.5.1 AW Passthrough with Adjustment	79

7.5.2 Skid Buffer for AW	80

7.6 2.5.6 Write Data Channel	81

7.6.1 Upsize Instance	81

7.7 2.5.7 Response Channel	82

7.7.1 B Channel	82

7.7.2 Response Ordering	82

7.7.3 Ordering Constraint (applies to both directions)	82

7.8 2.5.8 AW/W Synchronization	84

7.8.1 Challenge	84

7.8.2 Solution	84

7.9 2.5.9 Resource Utilization	85

7.9.1 Typical Resources (64→512, ID=4, ADDR=64)	85

7.10 2.5.10 Timing	86

7.10.1 Latency	86

7.10.2 Throughput	86

7.11 2.5.11 Usage Example	87

8 2.6 axi4_dwidth_converter_rd	88

8.1 2.6.1 Overview	89

8.2 2.6.2 Block Diagram	90

8.2.1 Figure 2.7: Read Converter Architecture	90

8.3 2.6.3 Interface Specification	91

8.3.1 Parameters	91

8.3.2 Ports	91

8.4 2.6.4 Burst Length Conversion	94

8.4.1 Ratio Calculation	94

8.4.2 Examples	94

8.5 2.6.5 Address Channel Handling	95

8.5.1 AR Passthrough with Adjustment	95

8.5.2 Burst-Length FIFO	96

8.6 2.6.6 Read Data Channel	97

8.6.1 Downsize Instance	97

8.7 2.6.7 RLAST Generation	98

8.8 2.6.8 RID Handling	99

8.8.1 ID Passthrough	99

8.8.2 Ordering Constraint (applies to both directions)	99

8.9 2.6.9 Resource Utilization	100

8.9.1 Typical Resources (64→512 UPSIZE, ratio 8, ID=4)	100

8.10 2.6.10 Timing	101

8.10.1 Latency	101

8.10.2 Throughput	101

8.11 2.6.11 Usage Example	102

9 2.7 AXIL Wide-Alignment Converters	103

9.1 2.7.1 Why They Exist	104

9.2 2.7.2 Parameters	105

10 3.1 Protocol Conversion Overview	106

10.1 3.1.1 Available Converters	107

10.1.1 AXI4 to AXI4-Lite (Protocol Downgrade)	107

10.1.2 AXI4-Lite to AXI4 (Protocol Upgrade)	107

10.1.3 Other Protocol Converters	107

10.2 3.1.2 Protocol Comparison	109

10.2.1 Figure 3.1: Protocol Feature Comparison	109

10.2.2 Feature Matrix	109

10.3 3.1.3 Conversion Strategies	110

10.3.1 AXI4 to AXI4-Lite	110

10.3.2 AXI4-Lite to AXI4	110

10.3.3 AXI4 to APB	110

10.4 3.1.4 Performance Characteristics	111

10.4.1 Key Observations	111

10.5 3.1.5 Use Case Guidelines	112

10.5.1 When to Use AXI4 to AXI4-Lite	112

10.5.2 When to Use AXI4-Lite to AXI4	112

10.5.3 When to Use AXI4 to APB	112

10.6 3.1.6 Integration Patterns	113

10.6.1 Pattern 1: CPU to Peripherals	113

10.6.2 Pattern 2: Simple IP in AXI4 Fabric	113

10.6.3 Pattern 3: Mixed System	113

11 3.2 AXI4 to AXI4-Lite Converter	114

11.1 3.2.1 Module Organization	115

11.1.1 Design Philosophy	115

11.2 3.2.2 Read Path (axi4_to_axil4_rd)	116

11.2.1 Figure 3.2: AXI4 to AXI4-Lite Read Path	116

11.2.2 Operation	116

11.2.3 State Machine	116

11.2.4 Implementation	117

11.3 3.2.3 Write Path (axi4_to_axil4_wr)	119

11.3.1 Figure 3.3: AXI4 to AXI4-Lite Write Path	119

11.3.2 Operation	119

11.3.3 AW/W Synchronization Challenge	120

11.3.4 Solution: W gated on the AW that owns it	120

11.3.5 Response Aggregation	121

11.4 3.2.4 Bidirectional Wrapper (axi4_to_axil4)	122

11.4.1 Composition Pattern	122

11.5 3.2.5 Resource Utilization	123

11.6 3.2.6 Timing	124

11.6.1 Throughput	124

11.6.2 Latency	124

11.7 3.2.7 Testing	125

12 3.3 AXI4-Lite to AXI4 Converter	126

12.1 3.3.1 Module Organization	127

12.2 3.3.2 Design Philosophy	128

12.3 3.3.3 Signal Mapping	129

12.3.1 Address Channel Signals	129

12.3.2 Data Channel Signals	129

12.4 3.3.4 Read Path (axil4_to_axi4_rd)	130

12.4.1 Figure 3.4: AXI4-Lite to AXI4 Read Path	130

12.4.2 Implementation	130

12.5 3.3.5 Write Path (axil4_to_axi4_wr)	133

12.5.1 Figure 3.5: AXI4-Lite to AXI4 Write Path	133

12.5.2 Implementation	133

12.6 3.3.6 Bidirectional Wrapper	136

12.7 3.3.7 Resource Utilization	137

12.8 3.3.8 Timing	138

12.9 3.3.9 Testing	139

12.10 3.3.10 Usage Example	140

13 3.4 AXI4 to APB Converter	141

13.1 3.4.1 Overview	142

13.2 3.4.2 Block Diagram	143

13.2.1 Figure 3.6: AXI4 to APB Converter	143

13.3 3.4.3 Interface Specification	144

13.3.1 Parameters	144

13.3.2 Ports	145

13.4 3.4.4 Clock Domains	148

13.5 3.4.5 State Machine	149

13.5.1 Figure 3.7: AXI4 to APB FSM	149

13.5.2 States	150

13.6 3.4.6 Burst Handling	151

13.6.1 Burst Decomposition	151

13.6.2 Address Calculation	151

13.7 3.4.7 Address Width Adaptation	152

13.7.1 64-bit to 32-bit Conversion	152

13.8 3.4.8 Error Response Mapping	153

13.8.1 Error Aggregation	153

13.9 3.4.9 Implementation	154

13.10 3.4.10 Resource Utilization	155

13.11 3.4.11 Timing	156

13.11.1 Timing Analysis	156

13.11.2 Throughput	156

13.12 3.4.12 Usage Example	157

14 AXI4 to APB5 Shim	159

14.1 Overview	160

14.2 Parameters	161

14.3 Design Notes	162

14.4 Related Modules	163

14.5 Navigation	164

15 AXI4 to AXI5-Lite	165

15.1 Overview	166

15.2 Where each signal gets its value	167

15.3 The AW/AR sideband is held, not passed through	168

15.4 Reset style	169

15.5 Verification	170

15.6 Related Modules	171

15.7 Navigation	172

16 3.5 PeakRDL Adapter	173

16.1 Overview	174

16.2 Parameters	175

16.3 Ports	176

16.4 Functional Description	178

16.4.1 Write	178

16.4.2 Read	178

16.4.3 3.5.5 Implementation	178

16.4.4 3.5.6 Resource Utilization	179

16.5 Waveforms	180

16.5.1 Figure 3.8: PeakRDL Adapter	180

16.6 Usage Example	182

16.6.1 3.5.7 Use Cases	182

16.6.2 3.5.8 Integration Example	182

16.7 Related Modules	184

16.7.1 3.5.9 APB4 Front End (apb4_to_peakrdl)	184

17 3.7 UART to AXI4-Lite Bridge	187

17.1 3.7.1 Module Organization	188

17.2 3.7.2 Command Protocol	189

17.3 3.7.3 Error Responses	190

17.4 3.7.4 Parameters	191

17.5 3.7.5 Ports	192

17.5.1 uart_axil_bridge	192

17.5.2 uart_rx	193

17.5.3 uart_tx	193

18 3.8 Width-plus-Protocol Chains	195

18.1 3.8.1 axi4_dwidth_to_axil4_wr_chain	196

18.2 3.8.2 axi4_dwidth_to_apb4_chain	197

19 3.10 AXI4-Lite to Wishbone B4 Converter	199

19.1 3.10.1 Module Organization	200

19.2 3.10.2 Design Philosophy	201

19.3 3.10.3 Block Diagram	202

19.3.1 Figure 3.12: AXI4-Lite to Wishbone B4 Converter	202

19.4 3.10.4 Interface Specification	203

19.4.1 Parameters	203

19.4.2 Ports	203

19.5 3.10.5 Command Formation	205

19.6 3.10.6 Response Mapping	206

19.7 3.10.7 Timing	207

19.8 3.10.8 Formal	208

19.9 3.10.9 Testing	209

19.10 3.10.10 Usage Example	210

20 3.11 Wishbone B4 to AXI4-Lite Converter	211

20.1 3.11.1 Module Organization	212

20.2 3.11.2 Why This Direction Is Harder	213

20.3 3.11.3 Interface Specification	214

20.3.1 Parameters	214

20.3.2 Ports	214

20.4 3.11.4 Command Formation	216

20.5 3.11.5 Response Mapping	217

20.6 3.11.6 Formal	218

20.7 3.11.7 Testing	219

20.8 3.11.8 Usage Example	220

20.9 Navigation	221

21 APB to AXI4 (Requester)	222

21.1 Overview	223

21.2 Fields the requester invents	224

21.3 APB5 sideband	225

21.4 Parameters	226

21.5 Verification	227

21.6 Related Modules	228

21.7 Navigation	229

22 4.1 Width Converter FSMs	230

22.1 Overview	230

22.2 Functional Description	231

22.2.1 4.1.1 Upsize Structure (no FSM)	231

22.2.2 4.1.2 Downsize Structure (single buffer, no FSM)	231

22.2.3 4.1.3 Full Converter FSMs	232

22.3 Timing	233

22.3.1 Upsize (8:1 ratio, m_ready held high)	233

22.3.2 Downsize (8:1 ratio, single buffer, atomic replace)	233

23 4.2 Protocol Converter FSMs	234

23.1 Overview	234

23.2 Functional Description	235

23.2.1 4.2.1 AXI4 to AXI4-Lite Read FSM	235

23.2.2 4.2.2 AXI4 to AXI4-Lite Write FSM	235

23.2.3 4.2.3 AXI4 to APB FSMs	235

23.3 Timing	236

23.3.1 4.2.4 Timing Analysis	236

24 4.3 Burst Decomposition	237

24.1 Overview	237

24.2 Functional Description	238

24.2.1 4.3.1 Width Converter Burst Handling	238

24.2.2 4.3.2 Protocol Converter Burst Handling	238

24.2.3 4.3.3 Response Aggregation	239

24.2.4 4.3.4 Burst Tracking Registers	240

24.3 Timing	242

24.3.1 4.3.5 Timing Impact	242

24.4 Waveforms	243

24.4.1 Figure 4.5: Width Burst Conversion	243

24.5 Navigation	244

25 5.1 Test Strategy	245

25.1 Overview	245

25.2 Testing	246

25.2.1 5.1.1 Test Organization	246

25.2.2 5.1.2 Test Levels	246

25.2.3 5.1.3 Test Categories	247

25.2.4 5.1.4 Testbench Architecture	248

25.2.5 5.1.5 Coverage Model	249

25.2.6 5.1.6 Test Execution	250

25.3 Navigation	251

26 5.2 Debug Guide	252

26.1 Overview	252

26.2 Testing	253

26.2.1 5.2.1 Common Issues	253

26.2.2 5.2.2 Debug Signals	254

26.2.3 5.2.3 Simulation Debug	255

26.2.4 5.2.4 Common Mistakes	256

26.2.5 5.2.5 Verification Checklist	257

26.2.6 5.2.6 Performance Validation	257

26.3 Navigation	259

List of Figures

Figure 1.1: Converter Module Hierarchy	31

Figure 2.1: Performance Comparison	43

Figure 2.2: axi_data_upsize Architecture	48

Figure 2.3: axi_data_dnsize Single-Buffer Architecture	61

Figure 2.6: Write Converter Architecture	73

Figure 2.7: Read Converter Architecture	90

Figure 3.1: Protocol Feature Comparison	109

Figure 3.2: AXI4 to AXI4-Lite Read Path	116

Figure 3.3: AXI4 to AXI4-Lite Write Path	119

Figure 3.4: AXI4-Lite to AXI4 Read Path	130

Figure 3.5: AXI4-Lite to AXI4 Write Path	133

Figure 3.6: AXI4 to APB Converter	143

Figure 3.7: AXI4 to APB FSM	149

Figure 3.8: PeakRDL Adapter	180

Figure 3.12: AXI4-Lite to Wishbone B4 Converter	202

Figure 4.5: Width Burst Conversion	243

List of Tables

Table 1: Table 0.1: Document Control Information	23

Table 2: Table 0.2: Related Documents	26

Table 3: Table 0.3: Revision History	27

Table 4: Table 1.1: Common Data Width Configurations	30

Table 5: Table 1.2: Protocol Mismatch Examples	30

Table 6: Table 1.3: Throughput vs. Area Trade-offs	32

Table 7: Table 1.4: Sideband Handling Modes	32

Table 8: Table 1.5: Latency Characteristics	34

Table 9: Table 1.6: Throughput Characteristics	34

Table 10: Table 2.1: Upsize Sideband Modes	41

Table 11: Table 2.2: Downsize Sideband Modes	41

Table 12: Table 2.3: Performance Comparison	43

Table 13: Table 2.4: axi_data_upsize Parameters	49

Table 14: Table 2.5: Upsize Latency	56

Table 15: Table 2.6: Upsize Resource Scaling	57

Table 16: Table 2.7: axi_data_dnsize Parameters	62

Table 17: Table 2.8: Measured Throughput by Mode	65

Table 18: Table 2.9: Resources and measured rate	69

Table 19: Table 2.14: Write Converter Parameters	74

Table 20: Table 2.15: Burst Length Conversion Examples	78

Table 21: Table 2.16: Write Converter Latency	86

Table 22: Table 2.17: Read Converter Parameters	91

Table 23: Table 2.18: Read Burst Length Conversion	94

Table 24: Table 2.20: Read Converter Latency	101

Table 25: Wide-Alignment Converter Parameters	105

Table 26: Table 3.1: AXI4 to AXI4-Lite Converters	107

Table 27: Table 3.2: AXI4-Lite to AXI4 Converters	107

Table 28: Table 3.3: Other Protocol Converters	107

Table 29: Table 3.4: Protocol Feature Comparison	109

Table 30: Table 3.5: Protocol Converter Performance	111

Table 31: Table 3.6: AXI4 to AXIL4 Resources	123

Table 32: Table 3.7: AXI4 to AXIL4 Throughput	124

Table 33: Table 3.8: AXI4 to AXIL4 Latency	124

Table 34: Table 3.9: Test Coverage Summary	125

Table 35: Table 3.10: AR Channel Mapping	129

Table 36: Table 3.11: R Channel Mapping	129

Table 37: Table 3.12: AXIL4 to AXI4 Resources	137

Table 38: Table 3.13: AXIL4 to AXI4 Performance	138

Table 39: Table 3.14: Test Coverage Summary	139

Table 40: Table 3.15: AXI4 vs APB Comparison	142

Table 41: Table 3.16: AXI4 to APB Parameters	144

Table 42: Table 3.17: AXI4 to APB Shim Parameters	144

Table 43: Table 3.18: Error Mapping	153

Table 44: Table 3.19: APB Converter Timing	156

Table 45: AXI4 to APB5 Shim – Inert Parameter	161

Table 46: AXI4 to AXI5-Lite – Sideband Disposition	167

Table 47: Table 3.20: PeakRDL Adapter Parameters	175

Table 48: apb4_to_peakrdl Parameters	184

Table 49: apb4_to_peakrdl Ports	185

Table 50: uart_axil_bridge Parameters	191

Table 51: uart_axil_bridge Ports	192

Table 52: uart_rx Ports	193

Table 53: uart_tx Ports	193

Table 54: axi4_dwidth_to_axil4_wr_chain Parameters	196

Table 55: axi4_dwidth_to_apb4_chain Parameters	197

Table 56: Table 3.26: AXI4-Lite to Wishbone Parameters	203

Table 57: Table 3.27: AXI4-Lite to Wishbone Ports	203

Table 58: Table 3.28: Command Formation	205

Table 59: Table 3.29: Response Mapping	206

Table 60: Table 3.30: AXI4-Lite to Wishbone Timing	207

Table 61: Table 3.31: Wishbone to AXI4-Lite Parameters	214

Table 62: Table 3.32: Wishbone to AXI4-Lite Ports	214

Table 63: Table 3.33: Command Formation	216

Table 64: Table 3.34: Response Mapping	217

Table 65: Table 3.35: APB-to-AXI4 requester pieces	223

Table 66: Table 3.36: AXI4 request fields from an APB transfer	224

Table 67: Table 3.37: APB5 USER mapping	225

Table 68: Table 3.38: APB-to-AXI4 parameters	226

Table 69: Table 4.6: Decomposition Overhead	242

Table 70: Table 5.1: Functional Test Categories	247

Table 71: Table 5.2: Stress Test Categories	247

Table 72: Table 5.3: Error Test Categories	247

Table 73: Table 5.4: Coverage Targets	249

Table 74: Table 5.5: Expected Throughput	257

# 1	Converter Mas Index

Generated: 2026-09-11

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Document Control

Table 0.1: Document Control Information

| Field | Value |
| --- | --- |
| Document Title | Converters Micro-Architecture Specification |
| Document Version | 1.0 |
| Component | Converters |
| Status | Active |
| Classification | Internal Technical |
| Last Updated | 2026-01-03 |

## 2.2	Purpose

This Micro-Architecture Specification (MAS) describes how the Converters component is actually built — the internal detail behind the feature set. It covers:

- Internal block architectures

- State machine designs

- Signal timing and handshaking

- Resource utilization estimates

- Debug and verification strategies

## 2.3	Audience

This document is for:

- RTL designers implementing or modifying converter modules

- Verification engineers creating testbenches

- Integration engineers connecting converters to systems

- Performance engineers optimizing throughput and latency

## 2.4	Related Documents

Table 0.2: Related Documents

| Document | Purpose |
| --- | --- |
| Bridge MAS | Related crossbar micro-architecture |
| Stream MAS | Related datapath micro-architecture |

(The old converter spec tree was migrated into this MAS, and converters has no standalone PRD — this book is the authoritative document.)

## 2.5	Revision History

Table 0.3: Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial MAS release |
| 1.2 | 2026-09-09 | RTL Design Sherpa | Added chapter 3.10, AXI4-Lite to Wishbone B4 |

## 2.6	Conventions

### 2.6.1	Notation

- signal_name - RTL signals and parameters

- ModuleName - Module names and key concepts

- Figure X.X - Figure references

### 2.6.2	Diagrams

All diagrams use Mermaid format rendered to PNG: - Source: assets/mermaid/*.mmd - Rendered: assets/mermaid/*.png

### 2.6.3	Code Examples

Treat the SystemVerilog snippets as implementation guidance: they show the intended design pattern but may differ slightly from the actual RTL.

Next: Chapter 1: Introduction

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Chapter 1: Introduction - Overview

## 3.1	1.1 Purpose

The Converters component provides data width and protocol conversion — the modules that let blocks with different data widths or communication protocols talk to each other.

## 3.2	1.2 Problem Statement

Sooner or later, every SoC design runs into the same two integration problems:

### 3.2.1	1.2.1 Data Width Mismatch

Table 1.1: Common Data Width Configurations

| Component | Typical Data Width |
| --- | --- |
| CPU | 64-bit |
| DDR Controller | 512-bit |
| PCIe Endpoint | 128-bit |
| GPU | 256-bit |

Challenge: You can’t wire mismatched widths together directly — a width converter has to sit between them.

### 3.2.2	1.2.2 Protocol Incompatibility

Table 1.2: Protocol Mismatch Examples

| Master Type | Protocol | Slave Type | Protocol |
| --- | --- | --- | --- |
| CPU | AXI4 | DDR | AXI4 |
| DMA | AXI4 | UART | APB |
| CPU | AXI4 | GPIO | APB |
| Custom IP | AXI4-Lite | Fabric | AXI4 |

Challenge: Different protocols can’t exchange a single transfer without a bridge between them.

## 3.3	1.3 Solution Architecture

### 3.3.1	Figure 1.1: Converter Module Hierarchy

Converter Module Hierarchy

### 3.3.2	1.3.1 Data Width Converters

Generic Building Blocks: - axi_data_upsize - Accumulates N narrow beats into 1 wide beat - axi_data_dnsize - Splits 1 wide beat into N narrow beats

Full AXI4 Converters: - axi4_dwidth_converter_wr - Complete write path (AW + W + B channels) - axi4_dwidth_converter_rd - Complete read path (AR + R channels)

### 3.3.3	1.3.2 Protocol Converters

AXI4 to AXI4-Lite: - axi4_to_axil4_rd - Read path burst decomposition - axi4_to_axil4_wr - Write path burst decomposition - axi4_to_axil4 - Full bidirectional wrapper

AXI4-Lite to AXI4: - axil4_to_axi4_rd - Read path protocol upgrade - axil4_to_axi4_wr - Write path protocol upgrade - axil4_to_axi4 - Full bidirectional wrapper

Other Converters: - axi4_to_apb4_convert - Full AXI4-to-APB bridge - peakrdl_to_cmdrsp - Register interface adapter

## 3.4	1.4 Key Design Decisions

### 3.4.1	1.4.1 Generic vs. Full Modules

The converters are built in three layers:

Layer 1: Generic Building Blocks
         axi_data_upsize, axi_data_dnsize
         - Protocol-agnostic data manipulation
         - Reusable across different contexts

Layer 2: Full AXI4 Converters
         axi4_dwidth_converter_wr, axi4_dwidth_converter_rd
         - Compose generic blocks with AXI4 channel management
         - Handle burst length adjustment, ID tracking

Layer 3: Protocol Converters
         axi4_to_axil4, axi4_to_apb4, etc.
         - Full protocol translation
         - State machine control

### 3.4.2	1.4.2 Throughput vs. Area Trade-offs

Table 1.3: Throughput vs. Area Trade-offs

| Configuration | Throughput | Area | Use Case |
| --- | --- | --- | --- |
| Upsize (single buffer) | 100% | 1x | All narrow-to-wide |
| Downsize (single buffer) | 0.992 beats/cycle | 1x | Area-constrained |

Design Decision: One buffer per direction. The downsize accepts its next wide beat during the last narrow beat of the current one — measured at 0.992 beats/cycle — so a second buffer has nothing to recover. A ping-pong mode existed for this and was removed once the single buffer was fixed.

### 3.4.3	1.4.3 Sideband Signal Handling

Sideband signals (WSTRB, RRESP, etc.) support three handling modes:

Table 1.4: Sideband Handling Modes

| Mode | Upsize Behavior | Downsize Behavior |
| --- | --- | --- |
| Concatenate | Pack narrow strobes | Slice wide strobes |
| Broadcast | N/A | Repeat value |
| Severity fold | Keep numeric max | N/A |

## 3.5	1.5 Performance Characteristics

### 3.5.1	1.5.1 Latency

Table 1.5: Latency Characteristics

| Module | Single-Beat | Burst (N beats) |
| --- | --- | --- |
| axi_data_upsize | 1 cycle (registered output) | RATIO narrow beats to fill a group |
| axi_data_dnsize (single) | 1 cycle | N cycles (replacement wide beat accepted during the last narrow beat — no gap) |
| axi4_to_axil4 | 0 cycles | slave-limited (see 3.2.6) |
| axil4_to_axi4 | 0 cycles | N/A (single only) |

### 3.5.2	1.5.2 Throughput

Table 1.6: Throughput Characteristics

| Module | Configuration | Peak Throughput |
| --- | --- | --- |
| axi_data_upsize | Single buffer | 1 beat/cycle |
| axi_data_dnsize | Single buffer | 0.992 beats/cycle (measured, ratio 4) |
| axi4_to_axil4 | Burst | slave-limited; requests stream independently of responses |
| axil4_to_axi4 | Any | 1 beat/cycle |

## 3.6	1.6 Scope

### 3.6.1	In Scope

- Integer width ratios (2:1, 4:1, 8:1, 16:1, etc.)

- AXI4, AXI4-Lite, and APB protocol support

- Configurable throughput vs. area trade-offs

- Generic building blocks for custom converters

- Burst-aware width conversion

### 3.6.2	Out of Scope

- Non-integer width ratios (e.g., 3:2 conversion)

- AXI4-Stream protocol (see Stream component)

- Buffering deeper than one beat per direction

- Clock domain crossing (use separate CDC modules)

- Address translation (handled by crossbar)

## 3.7	1.7 Target Applications

- CPU-to-DDR Integration - 64-bit CPU to 512-bit memory controller

- DMA Engines - Variable width data movers

- Mixed Protocol Systems - AXI4 fabric with APB peripheral bus

- FPGA Fabric Interfaces - Width matching for IP integration

- Register Access - PeakRDL to custom control protocols

Next: Chapter 2: Data Width Converter Blocks

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	2.1 Generic Building Blocks

Two generic building blocks handle the data movement: axi_data_upsize (narrow-to-wide) and axi_data_dnsize (wide-to-narrow). Both are protocol-agnostic — compose them into full AXI4 converters or use them directly in custom designs.

## 4.1	2.1.1 Module Hierarchy

Generic Building Blocks
|
+-- axi_data_upsize.sv      # Narrow-to-Wide accumulator
|   +-- Accumulator buffer
|   +-- Beat counter
|   +-- Sideband packing
|
+-- axi_data_dnsize.sv      # Wide-to-Narrow splitter
    +-- Data buffer
    +-- Beat counter
    +-- Sideband extraction
    +-- Optional burst tracking

## 4.2	2.1.2 Design Philosophy

### 4.2.1	Separation of Concerns

The generic blocks handle only data manipulation: - Data packing/unpacking - Sideband signal handling - Valid/ready handshaking

They do not handle: - Address manipulation - Burst length adjustment - ID tracking - Protocol-specific signals (ARLEN, AWLEN, etc.)

That separation buys you three things: - Reuse in multiple contexts - Simpler verification - Cleaner composition into full converters

### 4.2.2	Interface Pattern

Both modules use a consistent valid/ready interface:

// Input (narrow for upsize, wide for dnsize)
input  logic                    i_valid,
output logic                    o_ready,
input  logic [DATA_WIDTH-1:0]   i_data,
input  logic [SB_WIDTH-1:0]     i_sideband,
input  logic                    i_last,      // Optional

// Output (wide for upsize, narrow for dnsize)
output logic                    o_valid,
input  logic                    i_ready,
output logic [DATA_WIDTH-1:0]   o_data,
output logic [SB_WIDTH-1:0]     o_sideband,
output logic                    o_last       // Optional

## 4.3	2.1.3 Width Ratio Calculation

Both modules calculate the width ratio at elaboration time:

localparam int RATIO = WIDE_WIDTH / NARROW_WIDTH;
localparam int RATIO_LOG2 = $clog2(RATIO);

// Example: 64-bit to 512-bit
// RATIO = 512 / 64 = 8
// RATIO_LOG2 = 3

Constraints: - WIDE_WIDTH must be an integer multiple of NARROW_WIDTH - Minimum ratio: 2 - Maximum ratio: Typically 16 (limited by timing)

## 4.4	2.1.4 Sideband Modes

### 4.4.1	Upsize Sideband Modes

Table 2.1: Upsize Sideband Modes

| Mode | Parameter | Behavior | Use Case |
| --- | --- | --- | --- |
| Concatenate | SB_OR_MODE=0 | Pack N narrow sidebands | WSTRB packing |
| Severity fold | SB_OR_MODE=1 | Keep the numeric max of the narrow sidebands | RRESP worst-case fold |

Concatenate Example (WSTRB):

8 beats of 8-bit WSTRB → 1 beat of 64-bit WSTRB
[0]: 0xFF → output[7:0]   = 0xFF
[1]: 0xF0 → output[15:8]  = 0xF0
...
[7]: 0x0F → output[63:56] = 0x0F

OR Example (Error flags):

8 beats of error flags → 1 beat with any error
[0]: 0 → output = 0
[1]: 1 → output = 1 (error detected)
...
[7]: 0 → output remains 1

### 4.4.2	Downsize Sideband Modes

Table 2.2: Downsize Sideband Modes

| Mode | Parameter | Behavior | Use Case |
| --- | --- | --- | --- |
| Slice | SB_BROADCAST=0 | Extract slice per beat | WSTRB extraction |
| Broadcast | SB_BROADCAST=1 | Repeat full value | RRESP broadcast |

Slice Example (WSTRB):

1 beat of 64-bit WSTRB → 8 beats of 8-bit WSTRB
input = 0xFF_F0_..._0F
[0]: output = 0xFF (bits [7:0])
[1]: output = 0xF0 (bits [15:8])
...
[7]: output = 0x0F (bits [63:56])

Broadcast Example (RRESP):

1 beat of 2-bit RRESP → 8 beats of 2-bit RRESP
input = OKAY (2'b00)
[0-7]: output = OKAY (all beats get same response)

## 4.5	2.1.5 Performance Comparison

### 4.5.1	Figure 2.1: Performance Comparison

Performance Comparison

Table 2.3: Performance Comparison

| Module | Mode | Throughput | Latency | Area |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single | 100% | N cycles | 1x |
| axi_data_dnsize | Single | 0.992 beats/cycle | 1 cycle | 1x |

Single-buffer downsize does not stall between wide beats.

wide_ready is asserted during the last narrow beat, so the next wide beat is accepted as the current one finishes draining. With TRACK_BURSTS=1 the replacement is suppressed at burst boundaries only.

(The former dual-buffer mode is deleted; the atomic replace above is the whole story.)

## 4.6	2.1.6 Resource Utilization

### 4.6.1	Upsize Resources (NARROW=64, WIDE=512)

Accumulator buffer:   512 bits (output register)
Beat counter:         3 bits (clog2(8))
Sideband accumulator: 64 bits (WSTRB)
Control logic:        ~50 LEs

Total: ~70-100 LEs, ~580 registers

### 4.6.2	Downsize Resources (WIDE=512, NARROW=64)

Single Buffer:

Data buffer:          512 bits
Beat counter:         3 bits
Sideband logic:       ~20 LEs
Control logic:        ~50 LEs

Total: ~70-100 LEs, ~520 registers

## 4.7	2.1.7 Integration Guidelines

### 4.7.1	When to Use Generic Blocks

Use directly when: - Building custom data pipelines - Data width conversion without AXI4 protocol - Simple valid/ready streaming interfaces

Use full converters when: - Need AXI4 channel management (AW, W, B, AR, R) - Burst length adjustment required - ID tracking needed

### 4.7.2	Composition Example

// Full AXI4 write path composition
axi4_dwidth_converter_wr #(.S_AXI_DATA_WIDTH(32), .M_AXI_DATA_WIDTH(128)) u_wr (
    // This module internally instantiates:
    // 1. Address phase skid buffer
    // 2. axi_data_upsize for write data
    // 3. Response path passthrough
    // 4. Burst length adjustment logic
);

Next: axi_data_upsize Module

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	2.2 axi_data_upsize

The axi_data_upsize module accumulates N narrow beats into 1 wide beat. It’s the core building block for narrow-to-wide data width conversion.

## 5.1	2.2.1 Overview

The upsize module does four things:

- Data Accumulation: Collects N narrow data beats into accumulator buffer

- Sideband Packing: Concatenates or ORs sideband signals (WSTRB, etc.)

- Flow Control: Manages valid/ready handshaking with single-cycle latency

- LAST Tracking: Detects input LAST to generate output LAST

## 5.2	2.2.2 Block Diagram

### 5.2.1	Figure 2.2: axi_data_upsize Architecture

axi_data_upsize Architecture

## 5.3	2.2.3 Interface Specification

### 5.3.1	Parameters

Table 2.4: axi_data_upsize Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NARROW_WIDTH | int | 32 | Input data width (bits) |
| WIDE_WIDTH | int | 128 | Output data width (bits) |
| NARROW_SB_WIDTH | int | 0 | Input sideband width (bits), 0 if unused |
| WIDE_SB_WIDTH | int | 0 | Output sideband width, 0 if unused |
| SB_OR_MODE | int | 0 | 0=concatenate, 1=severity fold (numeric max) |

### 5.3.2	Ports

module axi_data_upsize #(
    // Width Configuration
    parameter int NARROW_WIDTH    = 32,
    parameter int WIDE_WIDTH      = 128,
    parameter int NARROW_SB_WIDTH = 0,        // Sideband width (0 if unused)
    parameter int WIDE_SB_WIDTH   = 0,        // Wide sideband width
    parameter int SB_OR_MODE      = 0,        // 0=concatenate, 1=severity fold (numeric max)

    // Calculated Parameters
    localparam int WIDTH_RATIO = WIDE_WIDTH / NARROW_WIDTH,
    localparam int PTR_WIDTH   = $clog2(WIDTH_RATIO),
    // Ensure sideband widths are at least 1 for port declarations (unused if actual width is 0)
    localparam int NARROW_SB_PORT_WIDTH = (NARROW_SB_WIDTH > 0) ? NARROW_SB_WIDTH : 1,
    localparam int WIDE_SB_PORT_WIDTH = (WIDE_SB_WIDTH > 0) ? WIDE_SB_WIDTH : 1
) (
    input  logic                            aclk,
    input  logic                            aresetn,

    // Narrow Input (from slave or master)
    input  logic                            narrow_valid,
    output logic                            narrow_ready,
    input  logic [NARROW_WIDTH-1:0]         narrow_data,
    input  logic [NARROW_SB_PORT_WIDTH-1:0] narrow_sideband,  // Min width 1 to avoid [-1:0]
    input  logic                            narrow_last,

    // Lane the FIRST narrow beat of a burst occupies inside the wide
    // word (addr % wide_bytes / narrow_bytes); later groups start at
    // lane 0. Tie '0 for aligned-only behavior.
    input  logic [PTR_WIDTH-1:0]            start_lane,

    // Wide Output (to master or slave)
    output logic                            wide_valid,
    input  logic                            wide_ready,
    output logic [WIDE_WIDTH-1:0]           wide_data,
    output logic [WIDE_SB_PORT_WIDTH-1:0]   wide_sideband,  // Min width 1 to avoid [-1:0]
    output logic                            wide_last
);

## 5.4	2.2.4 Operation

### 5.4.1	Accumulation Cycle

Cycle 0: s_data[0] → buffer[63:0],   count = 0 → 1
Cycle 1: s_data[1] → buffer[127:64], count = 1 → 2
...
Cycle 7: s_data[7] → buffer[511:448], count = 7 → 0, m_valid = 1
Cycle 8: m_ready handshake, output complete

### 5.4.2	Early LAST Handling

If s_last arrives before buffer is full:

Cycle 0: s_data[0] → buffer[63:0],   count = 0 → 1
Cycle 1: s_data[1] + s_last → buffer[127:64], count = 1 → 0
         m_valid = 1, m_last = 1
         Remaining bytes = don't care (masked by WSTRB)

### 5.4.3	State Machine

IDLE (count=0):
  - s_valid=1 → load beat, increment count
  - count < RATIO-1 → stay in IDLE
  - count = RATIO-1 OR s_last → OUTPUT

OUTPUT (m_valid=1):
  - m_ready=1 → clear buffer, → IDLE
  - m_ready=0 → hold output

## 5.5	2.2.5 Sideband Handling

### 5.5.1	Concatenate Mode (SB_OR_MODE=0)

Used for WSTRB packing:

// Pack narrow sidebands into wide sideband
always_ff @(posedge clk) begin
    if (s_valid && s_ready) begin
        r_sideband[r_count * NARROW_SB_WIDTH +: NARROW_SB_WIDTH] <= s_sideband;
    end
end

Example: 8 beats of 8-bit WSTRB to 64-bit WSTRB

Beat 0: WSTRB = 0xFF → output[7:0]   = 0xFF
Beat 1: WSTRB = 0xF0 → output[15:8]  = 0xF0
Beat 2: WSTRB = 0x0F → output[23:16] = 0x0F
...
Beat 7: WSTRB = 0xAA → output[63:56] = 0xAA
Final:  output = 0xAA_..._0F_F0_FF

### 5.5.2	Severity-Fold Mode (SB_OR_MODE=1)

Folds per-sub-beat responses into one wide-beat response by keeping the numeric maximum, not a bitwise OR. The distinction matters for the mode’s primary use case, 2-bit RRESP: with the AXI encoding (OKAY=00, EXOKAY=01, SLVERR=10, DECERR=11), SLVERR | EXOKAY = DECERR — an OR inflates the error class the moment an exclusive-read beat mixes with a slave error (CONV-005). Numeric max is exactly severity order for RRESP, so the fold keeps the worst response instead:

// gen_or_mode: keep the numeric max across the group's sub-beats
if (narrow_valid && narrow_ready) begin
    if (r_beat_ptr == '0)
        r_sideband_accumulator <= WIDE_SB_PORT_WIDTH'(narrow_sideband);
    else if (WIDE_SB_PORT_WIDTH'(narrow_sideband) > r_sideband_accumulator)
        r_sideband_accumulator <= WIDE_SB_PORT_WIDTH'(narrow_sideband);
end

For 1-bit sidebands (a bare error flag) max and OR coincide, so “any error in the group propagates” still holds:

Beat 0: RRESP = OKAY   (00) → accumulator = 00
Beat 1: RRESP = EXOKAY (01) → accumulator = 01
Beat 2: RRESP = SLVERR (10) → accumulator = 10 (worst so far)
Beat 3: RRESP = OKAY   (00) → accumulator = 10 (max retained)
Final:  wide RRESP = SLVERR — not DECERR, which a bitwise OR would fabricate

## 5.6	2.2.6 Implementation

### 5.6.1	Core Logic

Wide valid is registered, not combinational. It is set by the narrow beat that completes a group, so the wide word is only offered after its final lane has actually been written:

// Lane the incoming narrow beat lands in (start_lane on a fresh burst)
assign w_lane = (r_beat_ptr == '0) ? start_lane : r_beat_ptr;

// The accepted narrow beat that closes a group: last lane, or an early LAST
logic narrow_completes_group;
assign narrow_completes_group = narrow_valid && narrow_ready &&
                                (w_lane == PTR_WIDTH'(WIDTH_RATIO-1) ||
                                 narrow_last);

logic wide_accept;
assign wide_accept = r_wide_valid && wide_ready;

always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        r_data_accumulator <= '0;
        r_beat_ptr         <= '0;
        r_wide_valid       <= 1'b0;
        r_last_buffered    <= 1'b0;
        r_burst_fresh      <= 1'b1;
    end else begin
        if (narrow_valid && narrow_ready) begin
            // A group-opening beat ZEROES the whole accumulator and places
            // its data in one whole-register assignment; later beats write
            // only their own lane. Without the zeroing, an early narrow_last
            // (or a burst starting mid-word) leaves residual data and WSTRB
            // in the unfilled lanes and they leak into the emitted beat.
            if (r_beat_ptr == '0)
                r_data_accumulator <= {{(WIDE_WIDTH-NARROW_WIDTH){1'b0}}, narrow_data}
                                      << (w_lane * NARROW_WIDTH);
            else
                r_data_accumulator[w_lane*NARROW_WIDTH +: NARROW_WIDTH] <= narrow_data;

            r_beat_ptr    <= narrow_completes_group ? '0 : (w_lane + 1'b1);
            r_burst_fresh <= narrow_last;
        end

        // ONE point of control for r_wide_valid. A newly completed group
        // dominates an in-flight wide accept -- when both land on the same
        // cycle, two separate NBAs to this register would race.
        if (narrow_completes_group) begin
            r_wide_valid    <= 1'b1;
            r_last_buffered <= narrow_last;
        end else if (wide_accept) begin
            r_wide_valid    <= 1'b0;
            r_last_buffered <= 1'b0;
        end
    end
end

assign narrow_ready = !r_wide_valid || wide_ready;
assign wide_valid   = r_wide_valid;
assign wide_last    = r_last_buffered;

The registered form is what makes the throughput claim in 2.2.7 true. An earlier version of this page showed a combinational w_output_valid = (r_count == RATIO-1) || r_last driving m_valid, with s_ready gated on m_ready. That machine is wrong in both directions: it asserts valid during the cycle the completing narrow beat is still being accepted, so with m_ready high the wide beat handshakes before the final lane’s non-blocking write lands (a wide word missing its last narrow beat), and with m_ready low the completing narrow beat stalls (a bubble every group). It also contradicted 2.2.4, which correctly places the transition after the completing beat.

## 5.7	2.2.7 Timing

### 5.7.1	Latency

Table 2.5: Upsize Latency

| Scenario | Latency |
| --- | --- |
| Full buffer (N beats) | N cycles |
| Early LAST (M beats) | M cycles |
| Output handshake | 0-1 cycles |

### 5.7.2	Throughput

100% throughput — no gaps required between input beats.

The accumulator accepts one beat per cycle, every cycle. When the output buffer completes its handshake, accumulation of the next wide beat starts immediately.

### 5.7.3	Critical Paths

Typical critical paths: - s_data → accumulator buffer → m_data - r_count → comparison → s_ready

Timing closure: The module is designed for single-cycle operation, with combinatorial paths only within registered stages.

## 5.8	2.2.8 Resource Utilization

### 5.8.1	Typical Resources (64-bit to 512-bit)

Accumulator buffer:   512 flip-flops
Sideband buffer:      64 flip-flops (WSTRB)
Beat counter:         3 flip-flops
Control logic:        ~20 flip-flops
                      ~50-70 LUTs

Total: ~600 flip-flops, ~50-70 LUTs

### 5.8.2	Scaling

Table 2.6: Upsize Resource Scaling

| Configuration | Registers | LUTs |
| --- | --- | --- |
| 32 → 128 (4:1) | ~170 | ~30 |
| 64 → 256 (4:1) | ~330 | ~40 |
| 64 → 512 (8:1) | ~600 | ~60 |
| 128 → 1024 (8:1) | ~1150 | ~80 |

## 5.9	2.2.9 Usage Example

W-channel upsize (32 → 128) with WSTRB as the concatenated sideband:

axi_data_upsize #(
    .NARROW_WIDTH    (32),
    .WIDE_WIDTH      (128),
    .NARROW_SB_WIDTH (4),    // WSTRB, narrow side
    .WIDE_SB_WIDTH   (16),   // WSTRB, wide side
    .SB_OR_MODE      (0)     // concatenate strobes
) u_w_upsize (
    .aclk            (aclk),
    .aresetn         (aresetn),
    .narrow_valid    (s_wvalid),
    .narrow_ready    (s_wready),
    .narrow_data     (s_wdata),
    .narrow_sideband (s_wstrb),
    .narrow_last     (s_wlast),
    // '0 = aligned-only; wire addr%wide_bytes/narrow_bytes for
    // mid-word burst starts (see the wr converter's AW-lane queue)
    .start_lane      ('0),
    .wide_valid      (m_wvalid),
    .wide_ready      (m_wready),
    .wide_data       (m_wdata),
    .wide_sideband   (m_wstrb),
    .wide_last       (m_wlast)
);

Next: axi_data_dnsize Module

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	2.3 axi_data_dnsize

The axi_data_dnsize module splits 1 wide beat into N narrow beats. It accepts the next wide beat during the last narrow beat of the current one, so a steady stream costs no per-beat stall.

## 6.1	2.3.1 Overview

The downsize module does three things:

- Data Splitting: Extracts N narrow beats from one wide beat

- Sideband Extraction: Slices or broadcasts sideband signals

- Burst Tracking: Optional LAST signal generation based on burst length

## 6.2	2.3.2 Block Diagram

### 6.2.1	Figure 2.3: axi_data_dnsize Single-Buffer Architecture

axi_data_dnsize Single Buffer

## 6.3	2.3.3 Interface Specification

### 6.3.1	Parameters

Table 2.7: axi_data_dnsize Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| WIDE_WIDTH | int | 128 | Input data width (bits) |
| NARROW_WIDTH | int | 32 | Output data width (bits) |
| WIDE_SB_WIDTH | int | 0 | Input sideband width, 0 if unused |
| NARROW_SB_WIDTH | int | 0 | Output sideband width, 0 if unused |
| SB_BROADCAST | int | 1 | 0=slice, 1=broadcast sidebands |
| TRACK_BURSTS | int | 0 | Enable burst-aware LAST generation |
| BURST_LEN_WIDTH | int | 8 | Width of burst length input |

### 6.3.2	Ports

module axi_data_dnsize #(
    // Width Configuration
    parameter int WIDE_WIDTH        = 128,
    parameter int NARROW_WIDTH      = 32,
    parameter int WIDE_SB_WIDTH     = 0,        // Sideband width (0 if unused)
    parameter int NARROW_SB_WIDTH   = 0,
    parameter int SB_BROADCAST      = 1,        // 1=broadcast, 0=slice
    parameter int TRACK_BURSTS      = 0,        // 1=track bursts for LAST
    parameter int BURST_LEN_WIDTH   = 8,        // Burst length counter width

    // Calculated Parameters
    localparam int WIDTH_RATIO = WIDE_WIDTH / NARROW_WIDTH,
    localparam int PTR_WIDTH   = $clog2(WIDTH_RATIO),
    // Ensure sideband widths are at least 1 for port declarations
    localparam int WIDE_SB_PORT_WIDTH = (WIDE_SB_WIDTH > 0) ? WIDE_SB_WIDTH : 1,
    localparam int NARROW_SB_PORT_WIDTH = (NARROW_SB_WIDTH > 0) ? NARROW_SB_WIDTH : 1
) (
    input  logic                            aclk,
    input  logic                            aresetn,

    // Burst Control (only if TRACK_BURSTS=1)
    input  logic [BURST_LEN_WIDTH-1:0]      burst_len,       // From address channel (ARLEN/AWLEN)
    input  logic                            burst_start,     // Pulse to start new burst
    // TRACK_BURSTS only: lane the burst's first narrow beat occupies in
    // the FIRST wide word; later wide words slice from lane 0.
    input  logic [PTR_WIDTH-1:0]            start_lane,

    // Wide Input (from slave or master)
    input  logic                            wide_valid,
    output logic                            wide_ready,
    input  logic [WIDE_WIDTH-1:0]           wide_data,
    input  logic [WIDE_SB_PORT_WIDTH-1:0]   wide_sideband,  // Min width 1 to avoid [-1:0]
    input  logic                            wide_last,

    // Narrow Output (to master or slave)
    output logic                            narrow_valid,
    input  logic                            narrow_ready,
    output logic [NARROW_WIDTH-1:0]         narrow_data,
    output logic [NARROW_SB_PORT_WIDTH-1:0] narrow_sideband,  // Min width 1 to avoid [-1:0]
    output logic                            narrow_last
);

## 6.4	2.3.4 Single-Buffer Mode Operation

### 6.4.1	Split Cycle

Cycle 0: s_data loaded → buffer, s_ready = 0
Cycle 1: m_data = buffer[63:0],   count = 0, m_valid = 1
Cycle 2: m_data = buffer[127:64], count = 1
...
Cycle 8: m_data = buffer[511:448], count = 7, m_last possible
Cycle 9: wide_ready already asserted during cycle 8's last narrow beat -- no gap

### 6.4.2	State Machine

IDLE:
  - s_valid=1 → load buffer → SPLITTING

SPLITTING:
  - Output beats 0 to RATIO-1
  - m_ready=1 → increment count
  - count=RATIO-1 AND m_ready → IDLE

### 6.4.3	Throughput Analysis

No per-beat load cycle

For ratio N, a wide beat costs N cycles outputting narrow beats. The load of the next wide beat is not an extra cycle: wide_ready is asserted during the Nth narrow beat, so the replacement is accepted as the current beat finishes.

TRACK_BURSTS=1 is the exception. Its ready condition is mid_burst_replace, which excludes the final beat of a burst, so a cycle is given up at each burst boundary — not at each wide beat.

Measured on axi_data_dnsize with both sides driven by the shared backtoback randomizer profile, 64 wide beats per run, timing the drain only (see measure_throughput in the dnsize TB):

| Configuration | Narrow beats | Cycles | Beats/cycle |
| --- | --- | --- | --- |
| ratio 4, single buffer | 256 | 258 | 0.992 |
| ratio 2, single buffer | 128 | 130 | 0.985 |

Both buffering modes sustain a narrow beat every cycle, which is the most the narrow side can carry. The shortfall is a constant 2-cycle pipeline fill, not a per-beat cost — it does not grow with the run, which is how the earlier “one gap cycle per wide beat” model was ruled out.

TRACK_BURSTS=1 gives up one cycle per BURST boundary (its replace condition excludes each burst’s final beat) and measures 0.914-0.941 beats/cycle over 8 framed bursts of 32 narrow beats — the shortfall being the same fixed fill, paid once per burst (measure_burst_throughput in the dnsize TB).

Table 2.8: Measured Throughput by Mode

| Mode | Measured | Cost model |
| --- | --- | --- |
| Simple, ratio 4 | 0.992 beats/cycle | fixed 2-cycle fill only |
| Simple, ratio 2 | 0.985 beats/cycle | fixed 2-cycle fill only |
| TRACK_BURSTS, ratio 4 | 0.914-0.941 | one bubble per burst boundary |

## 6.5	2.3.5 Sideband Handling

### 6.5.1	Slice Mode (SB_BROADCAST=0)

Used for WSTRB extraction:

// Extract sideband slice per beat
assign m_sideband = r_sideband[r_count * NARROW_SB_WIDTH +: NARROW_SB_WIDTH];

Example: 64-bit WSTRB to 8-bit WSTRB

Input: 0xAA_BB_CC_DD_EE_FF_00_11

Beat 0: output WSTRB = 0x11 (bits [7:0])
Beat 1: output WSTRB = 0x00 (bits [15:8])
Beat 2: output WSTRB = 0xFF (bits [23:16])
...
Beat 7: output WSTRB = 0xAA (bits [63:56])

### 6.5.2	Broadcast Mode (SB_BROADCAST=1)

Used for RRESP:

// Broadcast same sideband to all beats
assign m_sideband = r_sideband[NARROW_SB_WIDTH-1:0];

Example: RRESP = OKAY for all beats

Input: RRESP = 2'b00 (OKAY)

Beat 0: output RRESP = 2'b00
Beat 1: output RRESP = 2'b00
...
Beat 7: output RRESP = 2'b00

## 6.6	2.3.6 Burst Tracking

### 6.6.1	Purpose

When TRACK_BURSTS=1, the module generates narrow_last from the AXI4 burst length instead of relying on the incoming wide_last.

### 6.6.2	Operation

// from the RTL: burst_len is already in NARROW beats (the caller frames
// the burst in output units -- ARLEN/AWLEN scaled by the instantiator),
// so there is no xRATIO multiply here
if (TRACK_BURSTS != 0 && burst_start && !r_burst_active) begin
    r_slave_total_beats <= burst_len + 1'b1;
    r_slave_beat_count  <= '0;
    r_burst_active      <= 1'b1;
end

// count every narrow beat sent; LAST on the final one
assign narrow_last = r_wide_buffered && r_burst_active &&
                     (r_slave_beat_count + 1'b1 >= r_slave_total_beats);

An earlier revision showed the module multiplying burst_len by RATIO internally. It never did — the units contract is narrow beats in, and mis-framing in wide beats makes LAST fire WIDTH_RATIO times early (a real bug this exact confusion caused in the TB).

Example: ARLEN=3 (4 beats), ratio 8:1

Total narrow beats = 4 * 8 = 32
Beat 0-30: m_last = 0
Beat 31: m_last = 1

## 6.7	2.3.7 Implementation

### 6.7.1	Single-Buffer Core Logic

// from the RTL -- the whole point is that wide_ready re-asserts DURING
// the last narrow beat (atomic replace), so there is NO
// `s_ready = !r_active` style stall between wide beats:
assign wide_ready = !gen_single_buffer.r_wide_buffered ||
                    (narrow_ready && w_last_narrow_beat);

// output slice selection
assign narrow_data = gen_single_buffer.r_data_buffer
                     [r_beat_ptr * NARROW_WIDTH +: NARROW_WIDTH];
assign narrow_valid = gen_single_buffer.r_wide_buffered;

// simple mode: LAST = buffered wide_last AND final slice
assign narrow_last = gen_single_buffer.r_wide_buffered &&
                     gen_single_buffer.r_last_buffered &&
                     w_last_narrow_beat;

An earlier pseudocode block here used s_ready = !r_active — the pre-fix behaviour that stalls one cycle per wide beat, which the measured 0.992 beats/cycle disproves.

## 6.8	2.3.8 Resource Utilization

### 6.8.1	Single-Buffer (512-bit to 64-bit)

Data buffer:          512 flip-flops
Sideband buffer:      64 flip-flops
Beat counter:         3 flip-flops
Control logic:        ~10 flip-flops
                      ~30-50 LUTs

Total: ~590 flip-flops, ~30-50 LUTs

### 6.8.2	Measured Throughput

Table 2.9: Resources and measured rate

| Registers | LUTs | Throughput |
| --- | --- | --- |
| 590 | 40 | 0.992 beats/cycle (ratio 4) |

A narrow beat every cycle, so there is nothing left for a second buffer to recover. The ping-pong DUAL_BUFFER mode this table used to compare against was removed once the single buffer was fixed to accept its replacement during the last narrow beat — nothing instantiated it and it measured no faster.

## 6.9	2.3.9 Usage Example

R-channel downsize (128 → 32) with RRESP broadcast and burst tracking. burst_start/burst_len are NOT optional with TRACK_BURSTS(1): r_burst_active only sets on a burst_start pulse, so leaving them unconnected silently produces framing with no LAST at all. burst_len is in NARROW beats minus one.

axi_data_dnsize #(
    .WIDE_WIDTH      (128),
    .NARROW_WIDTH    (32),
    .WIDE_SB_WIDTH   (2),    // RRESP
    .NARROW_SB_WIDTH (2),
    .SB_BROADCAST    (1),
    .TRACK_BURSTS    (1),
    .BURST_LEN_WIDTH (8)
) u_r_dnsize (
    .aclk            (aclk),
    .aresetn         (aresetn),
    .burst_len       (8'(narrow_beats - 1)),  // narrow-beat units
    .burst_start     (ar_accepted),
    // '0 = aligned-only; wire addr%wide_bytes/narrow_bytes for
    // mid-word burst starts (see the rd converter's length+lane FIFO)
    .start_lane      ('0),
    .wide_valid      (s_rvalid),
    .wide_ready      (s_rready),
    .wide_data       (s_rdata),
    .wide_sideband   (s_rresp),
    .wide_last       (1'b0),                  // counter drives LAST here
    .narrow_valid    (m_rvalid),
    .narrow_ready    (m_rready),
    .narrow_data     (m_rdata),
    .narrow_sideband (m_rresp),
    .narrow_last     (m_rlast)
);

Next: axi4_dwidth_converter_wr

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	2.5 axi4_dwidth_converter_wr

The axi4_dwidth_converter_wr module is the complete AXI4 write path — AW, W, and B channels with burst length adjustment for the new width.

## 7.1	2.5.1 Overview

The write converter combines the generic axi_data_upsize with AXI4 protocol handling:

- Address Channel (AW): Passes through with burst length adjustment

- Write Data Channel (W): Uses axi_data_upsize for data packing

- Response Channel (B): Passes through unchanged

- Burst Length Adjustment: Converts AWLEN based on width ratio

## 7.2	2.5.2 Block Diagram

### 7.2.1	Figure 2.6: Write Converter Architecture

Write Converter Architecture

## 7.3	2.5.3 Interface Specification

### 7.3.1	Parameters

Table 2.14: Write Converter Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| S_AXI_DATA_WIDTH | int | 32 | Slave-side data width |
| M_AXI_DATA_WIDTH | int | 128 | Master-side data width |
| AXI_ID_WIDTH | int | 8 | Transaction ID width |
| AXI_ADDR_WIDTH | int | 32 | Address width |
| AXI_USER_WIDTH | int | 1 | User-signal width |
| SKID_DEPTH_AW | int | 2 | AW skid buffer depth |
| SKID_DEPTH_W | int | 4 | W skid buffer depth |
| SKID_DEPTH_B | int | 2 | B skid buffer depth |

### 7.3.2	Ports

module axi4_dwidth_converter_wr #(
    // Width Configuration
    parameter int S_AXI_DATA_WIDTH  = 32,
    parameter int M_AXI_DATA_WIDTH  = 128,
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,

    // Skid Buffer Depths (for timing closure)
    parameter int SKID_DEPTH_AW     = 2,
    parameter int SKID_DEPTH_W      = 4,
    parameter int SKID_DEPTH_B      = 2,

    // Calculated Parameters
    localparam int S_STRB_WIDTH = S_AXI_DATA_WIDTH / 8,
    localparam int M_STRB_WIDTH = M_AXI_DATA_WIDTH / 8,
    localparam int WIDTH_RATIO  = (S_AXI_DATA_WIDTH < M_AXI_DATA_WIDTH) ?
                                  (M_AXI_DATA_WIDTH / S_AXI_DATA_WIDTH) :
                                  (S_AXI_DATA_WIDTH / M_AXI_DATA_WIDTH),
    localparam bit UPSIZE       = (S_AXI_DATA_WIDTH < M_AXI_DATA_WIDTH) ? 1'b1 : 1'b0,
    localparam bit DOWNSIZE     = (S_AXI_DATA_WIDTH > M_AXI_DATA_WIDTH) ? 1'b1 : 1'b0,

    // Skid buffer packed widths
    localparam int AW_WIDTH = AXI_ID_WIDTH + AXI_ADDR_WIDTH + 8 + 3 + 2 + 1 + 4 + 3 + 4 + 4 + AXI_USER_WIDTH,
    localparam int W_WIDTH  = S_AXI_DATA_WIDTH + S_STRB_WIDTH + 1 + AXI_USER_WIDTH,
    localparam int B_WIDTH  = AXI_ID_WIDTH + 2 + AXI_USER_WIDTH
) (
    // Clock and Reset
    input  logic                        aclk,
    input  logic                        aresetn,

    //==========================================================================
    // Slave AXI Write Interface
    //==========================================================================

    // Write Address Channel
    input  logic [AXI_ID_WIDTH-1:0]     s_axi_awid,
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axi_awaddr,
    input  logic [7:0]                  s_axi_awlen,
    input  logic [2:0]                  s_axi_awsize,
    input  logic [1:0]                  s_axi_awburst,
    input  logic                        s_axi_awlock,
    input  logic [3:0]                  s_axi_awcache,
    input  logic [2:0]                  s_axi_awprot,
    input  logic [3:0]                  s_axi_awqos,
    input  logic [3:0]                  s_axi_awregion,
    input  logic [AXI_USER_WIDTH-1:0]   s_axi_awuser,
    input  logic                        s_axi_awvalid,
    output logic                        s_axi_awready,

    // Write Data Channel
    input  logic [S_AXI_DATA_WIDTH-1:0] s_axi_wdata,
    input  logic [S_STRB_WIDTH-1:0]     s_axi_wstrb,
    input  logic                        s_axi_wlast,
    input  logic [AXI_USER_WIDTH-1:0]   s_axi_wuser,
    input  logic                        s_axi_wvalid,
    output logic                        s_axi_wready,

    // Write Response Channel
    output logic [AXI_ID_WIDTH-1:0]     s_axi_bid,
    output logic [1:0]                  s_axi_bresp,
    output logic [AXI_USER_WIDTH-1:0]   s_axi_buser,
    output logic                        s_axi_bvalid,
    input  logic                        s_axi_bready,

    //==========================================================================
    // Master AXI Write Interface
    //==========================================================================

    // Write Address Channel
    output logic [AXI_ID_WIDTH-1:0]     m_axi_awid,
    output logic [AXI_ADDR_WIDTH-1:0]   m_axi_awaddr,
    output logic [7:0]                  m_axi_awlen,
    output logic [2:0]                  m_axi_awsize,
    output logic [1:0]                  m_axi_awburst,
    output logic                        m_axi_awlock,
    output logic [3:0]                  m_axi_awcache,
    output logic [2:0]                  m_axi_awprot,
    output logic [3:0]                  m_axi_awqos,
    output logic [3:0]                  m_axi_awregion,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_awuser,
    output logic                        m_axi_awvalid,
    input  logic                        m_axi_awready,

    // Write Data Channel
    output logic [M_AXI_DATA_WIDTH-1:0] m_axi_wdata,
    output logic [M_STRB_WIDTH-1:0]     m_axi_wstrb,
    output logic                        m_axi_wlast,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_wuser,
    output logic                        m_axi_wvalid,
    input  logic                        m_axi_wready,

    // Write Response Channel
    input  logic [AXI_ID_WIDTH-1:0]     m_axi_bid,
    input  logic [1:0]                  m_axi_bresp,
    input  logic [AXI_USER_WIDTH-1:0]   m_axi_buser,
    input  logic                        m_axi_bvalid,
    output logic                        m_axi_bready
);

## 7.4	2.5.4 Burst Length Conversion

### 7.4.1	Ratio Calculation

// Direction-aware: wider / narrower (a plain M/S divide evaluates to
// 0 in DOWNSIZE mode and $clog2(0) is illegal)
localparam int RATIO = (S_AXI_DATA_WIDTH < M_AXI_DATA_WIDTH)
                       ? (M_AXI_DATA_WIDTH / S_AXI_DATA_WIDTH)
                       : (S_AXI_DATA_WIDTH / M_AXI_DATA_WIDTH);
localparam int RATIO_LOG2 = $clog2(RATIO);

// upsize: New AWLEN = ceil((AWLEN + 1) / RATIO) - 1  (round UP --
// a floor divide underflows on non-multiples, see 2.5.5)
// downsize: split into master bursts of <= 256 beats (see 2.5.5)

### 7.4.2	Examples

Table 2.15: Burst Length Conversion Examples

| S_DATA | M_DATA | Ratio | S_AWLEN | S_beats | M_AWLEN | M_beats |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | 512 | 8 | 7 | 8 | 0 | 1 |
| 64 | 512 | 8 | 15 | 16 | 1 | 2 |
| 64 | 256 | 4 | 3 | 4 | 0 | 1 |
| 64 | 256 | 4 | 7 | 8 | 1 | 2 |

### 7.4.3	Non-Aligned Bursts

When the burst length is not a multiple of the ratio:

S_AWLEN = 5 (6 beats), RATIO = 8
M_AWLEN = 0 (1 beat)

The 6 narrow beats pack into 1 wide beat.
Remaining 2 positions have WSTRB = 0 (no write).

## 7.5	2.5.5 Address Channel Handling

### 7.5.1	AW Passthrough with Adjustment

The rewrite differs by direction, so the RTL has two generate branches.

Narrow → wide (upsize). Beats combine, so the count divides — and it must round UP, because a burst that is not a whole multiple of the ratio still needs a final partial wide beat:

assign m_axi_awlen = ((int_awlen + 8'(WIDTH_RATIO)) / 8'(WIDTH_RATIO)) - 8'd1;

Wide → narrow (downsize). Each wide beat becomes RATIO narrow ones — and the product does not fit a burst. AXI4 allows 256 beats, so a full-length slave burst needs up to 256 * WIDTH_RATIO narrow beats, which is neither expressible in the 8-bit AWLEN nor a legal burst. One slave burst is therefore split into as many master bursts of <= 256 beats as it takes:

// remaining-beat counter, 9 + $clog2(WIDTH_RATIO) bits -- 8 is exactly
// what a multiply-into-AWLEN overflows
r_split_remaining <= (CNTW'(int_awlen) + CNTW'(1)) * CNTW'(WIDTH_RATIO);
...
assign m_axi_awlen = 8'(w_this_beats - 9'd1);   // w_this_beats = min(remaining, 256)

Each issued master burst is recorded in a split queue — its beat count frames that burst’s WLAST on the W path, and a final-burst flag drives the B fold (several master responses collapse into one slave response, worst case wins). The address advances by 256 * M_STRB_WIDTH per burst for INCR and holds for FIXED; WRAP never reaches a split, since AXI4 caps it at 16 beats and 16 * RATIO <= 256 for every supported ratio. The slave’s AW is consumed only when its final master burst has been issued.

Rounding up is the whole point of the + WIDTH_RATIO term. A floor divide underflows: AWLEN=5 (6 beats) at RATIO=8 gives (6 >> 3) - 1, which is -1 — 8’hFF, a 256-beat burst — where the correct answer is 0, one wide beat.

Size is not derived from the incoming AWSIZE. Both branches drive the master side at its own full width:

assign m_axi_awsize = MASTER_SIZE[2:0];

Neither converter checks address alignment on the downsize path. The read converter aligns the address it issues on its UPSIZE path only — a wide access cannot start mid-word (see 2.6.5).

Upsize INCR bursts may start mid-wide-word. The AXI-correct behavior: the first wide beat carries the narrow data in the byte lanes the ADDRESS selects, with WSTRB covering only those lanes (leading lanes byte-disabled, not clobbered). Three pieces cooperate:

- m_axi_awlen counts the lane offset: ceil((start_lane + narrow_beats) / RATIO) wide beats. AWADDR passes through unchanged — unaligned is legal AXI; the first beat is partial within its size container.

- an AW-lane queue (same inline pattern as the read converter’s burst-length FIFO) records each accepted AW’s start lane; W beats are held off until their AW is queued (the packer’s lane comes from AWADDR), and the entry pops on the NARROW side’s last beat — the head must track the burst the CURRENT narrow beat belongs to, and a wide-side pop raced back-to-back bursts.

- axi_data_upsize takes a start_lane input: the first narrow beat of a burst lands at that lane (data AND WSTRB shifted, leading slots ’0); later wide groups of the burst start at lane 0.

FIXED and WRAP keep the wide-aligned requirement (asserted in simulation) — their lane semantics through the packer are not defined.

### 7.5.2	Skid Buffer for AW

The channel buffers are gaxi_skid_buffer (there is no axi_skid_buffer), and they carry the UNMODIFIED slave fields — the length rewrite happens after the skid, in the splitter:

gaxi_skid_buffer #(
    .DEPTH(SKID_DEPTH_AW),
    .DATA_WIDTH(AW_WIDTH)
) aw_skid (
    .axi_aclk   (aclk),
    .axi_aresetn(aresetn),
    .wr_valid   (s_axi_awvalid),
    .wr_ready   (s_axi_awready),
    .wr_data    ({s_axi_awid, s_axi_awaddr, s_axi_awlen, s_axi_awsize,
                  s_axi_awburst, s_axi_awlock, s_axi_awcache, s_axi_awprot,
                  s_axi_awqos, s_axi_awregion, s_axi_awuser}),
    .rd_valid   (int_aw_valid),
    .rd_ready   (int_aw_ready),
    .rd_data    (int_aw_data),
    .count      (),
    .rd_count   ()
);

## 7.6	2.5.6 Write Data Channel

### 7.6.1	Upsize Instance

axi_data_upsize #(
    .NARROW_WIDTH    (S_AXI_DATA_WIDTH),
    .WIDE_WIDTH      (M_AXI_DATA_WIDTH),
    .NARROW_SB_WIDTH (S_STRB_WIDTH),   // WSTRB
    .WIDE_SB_WIDTH   (M_STRB_WIDTH),
    .SB_OR_MODE      (0)               // Concatenate WSTRB
) u_w_upsize (
    .aclk            (aclk),
    .aresetn         (aresetn),
    .narrow_valid    (int_w_valid),
    .narrow_ready    (int_w_ready),
    .narrow_data     (int_wdata),
    .narrow_sideband (int_wstrb),
    .narrow_last     (int_wlast),
    .start_lane      (w_upsize_start_lane[$clog2(WIDTH_RATIO)-1:0]),
    .wide_valid      (m_axi_wvalid),
    .wide_ready      (m_axi_wready),
    .wide_data       (m_axi_wdata),
    .wide_sideband   (m_axi_wstrb),
    .wide_last       (m_axi_wlast)
);

## 7.7	2.5.7 Response Channel

### 7.7.1	B Channel

Upsize passes the response through unchanged. Downsize cannot: a split slave burst receives several master responses, and the slave expects ONE. Non-final responses are consumed immediately and folded worst-case- wins; the final one carries the folded result (see 2.5.5). The passthrough below is the UPSIZE branch:

// gen_b_pass (upsize)
assign s_bvalid = m_bvalid;
assign m_bready = s_bready;
assign s_bid    = m_bid;
assign s_bresp  = m_bresp;

### 7.7.2	Response Ordering

Responses return in order provided the master-side slave returns them in issue order across all IDs – see the constraint below, which is a real restriction and not a formality. Given that, the fold is safe because: - Upsize doesn’t reorder data - B response generated after all W beats accepted

An earlier version of this list gave “single outstanding transaction per ID” as sufficient. It is not: the B fold compares no IDs at all.

### 7.7.3	Ordering Constraint (applies to both directions)

The master-side slave must return responses in AR/AW issue order across ALL IDs. Neither converter carries a transaction ID through its data path, so neither can attribute an out-of-order completion to the burst it belongs to.

Read side: the burst-length queue holds {narrow ARLEN, start lane} and nothing else, and s_axi_rid is a single register latched on every master R handshake. If a slave returns ID1’s data before ID0’s, ID1’s beats are counted against ID0’s queue head, s_axi_rlast fires at the wrong boundary, one aggregated wide beat can mix two transactions, and s_axi_rid is whichever ID handshaked most recently.

Write side: the split queue is popped by split_b_pop = m_axi_bvalid && m_axi_bready – any arriving B pops the head, whatever its ID – and m_axi_bready = split_b_final ? int_b_ready : 1'b1 swallows a non-final response unconditionally. A B that arrives out of order is folded into the wrong burst’s accumulator: the burst it actually belonged to never gets its response (the master hangs), the other burst’s response is built from the wrong beats, and every later burst’s framing is shifted by one.

AXI4 permits both behaviours. A slave may complete different-ID transactions out of order and may interleave read data across IDs, and a multi-ported DDR controller normally does. Satisfying “one outstanding transaction per ID” is not sufficient – two IDs with one transaction each meet that and still break the fold.

Safe configurations are: an in-order master-side slave, or a single ID outstanding at a time. Nothing in the RTL enforces or detects a violation, so a breach corrupts data silently rather than failing (tracked as CONV-001).

## 7.8	2.5.8 AW/W Synchronization

### 7.8.1	Challenge

AXI4 allows AW to arrive before, with, or after W data. The converter must handle all cases:

- AW before W: Normal pipelining

- W before AW: Data buffered until AW arrives

- Interleaved: Multiple transactions in flight

### 7.8.2	Solution

The upsize path carries an AW-lane queue (see 2.5.5): each accepted AW pushes its start lane, and W beats are held off until their AW is queued — the packer’s lane placement comes from AWADDR, so W genuinely cannot run ahead of its address. The entry pops on the narrow side’s last beat.

The downsize path does carry one — the split queue of 2.5.5. Each issued master burst pushes {final-burst flag, beat count}; the beat count frames that burst’s WLAST on the W path and the flag drives the B fold. AW runs ahead of W, which is precisely why that AW-derived information must be queued.

Compare the read converter, which needs a queue in its UPSIZE mode: its downsize block has no length queue of its own, so overlapping read bursts would lose their framing without an external ARLEN queue.

## 7.9	2.5.9 Resource Utilization

### 7.9.1	Typical Resources (64→512, ID=4, ADDR=64)

AW skid buffer:     ~200 flip-flops
W upsize:           ~600 flip-flops, ~60 LUTs
B skid buffer:      ~20 flip-flops
Control logic:      ~100 LUTs

Total: ~820 flip-flops, ~160 LUTs

Hand estimates, not synthesis results. AW-derived storage: the
downsize split queue, or the upsize AW-lane queue (16 x lane-width
entries + two 5-bit pointers) -- see 2.5.5 for both.

## 7.10	2.5.10 Timing

### 7.10.1	Latency

Table 2.16: Write Converter Latency

| Path | Latency |
| --- | --- |
| AW passthrough | 1-2 cycles (skid) |
| W upsize | N cycles (accumulation) |
| B passthrough | 1 cycle (skid) |

### 7.10.2	Throughput

- AW channel: 1 transaction/cycle

- W channel: 100% (upsize is 100%)

- B channel: 1 response/cycle

## 7.11	2.5.11 Usage Example

axi4_dwidth_converter_wr #(
    .S_AXI_DATA_WIDTH (128),   // slave wide
    .M_AXI_DATA_WIDTH (32),    // master narrow (downsize)
    .AXI_ID_WIDTH     (8),
    .AXI_ADDR_WIDTH   (32),
    .SKID_DEPTH_AW    (2),
    .SKID_DEPTH_W     (4),
    .SKID_DEPTH_B     (2)
) u_conv (
    .aclk             (aclk),
    .aresetn          (aresetn),
    // slave side: s_axi_aw*/... (full AXI4 channel set)
    .s_axi_awvalid   (cpu_awvalid),
    .s_axi_awready   (cpu_awready),
    // ...
    // master side: m_axi_* toward the narrow fabric
    .m_axi_awvalid   (mem_awvalid),
    .m_axi_awready   (mem_awready)
    // ...
);

All channel ports carry the s_axi_/m_axi_ prefix; the full list is the module header. Skid depths are per channel — there is no single SKID_DEPTH.

Next: axi4_dwidth_converter_rd

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	2.6 axi4_dwidth_converter_rd

The axi4_dwidth_converter_rd module is the complete AXI4 read path — AR and R channels with burst length adjustment and burst-aware RLAST generation.

## 8.1	2.6.1 Overview

The read converter combines the generic axi_data_dnsize with AXI4 protocol handling:

- Address Channel (AR): Passes through with burst length adjustment

- Read Data Channel (R): Uses axi_data_dnsize for data splitting

- Burst Tracking: Generates correct RLAST based on original ARLEN

- Response Broadcasting: Propagates RRESP to all narrow beats

## 8.2	2.6.2 Block Diagram

### 8.2.1	Figure 2.7: Read Converter Architecture

Read Converter Architecture

## 8.3	2.6.3 Interface Specification

### 8.3.1	Parameters

Table 2.17: Read Converter Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| S_AXI_DATA_WIDTH | int | 32 | Slave-side data width |
| M_AXI_DATA_WIDTH | int | 128 | Master-side data width |
| AXI_ID_WIDTH | int | 8 | Transaction ID width |
| AXI_ADDR_WIDTH | int | 32 | Address width |
| AXI_USER_WIDTH | int | 1 | User-signal width |
| SKID_DEPTH_AR | int | 2 | AR skid buffer depth |
| SKID_DEPTH_R | int | 4 | R skid buffer depth |

### 8.3.2	Ports

module axi4_dwidth_converter_rd #(
    // Width Configuration
    parameter int S_AXI_DATA_WIDTH  = 32,
    parameter int M_AXI_DATA_WIDTH  = 128,
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,

    // Skid Buffer Depths (for timing closure)
    parameter int SKID_DEPTH_AR     = 2,
    parameter int SKID_DEPTH_R      = 4,

    // Calculated Parameters
    localparam int S_STRB_WIDTH = S_AXI_DATA_WIDTH / 8,
    localparam int M_STRB_WIDTH = M_AXI_DATA_WIDTH / 8,
    localparam int WIDTH_RATIO  = (S_AXI_DATA_WIDTH < M_AXI_DATA_WIDTH) ?
                                  (M_AXI_DATA_WIDTH / S_AXI_DATA_WIDTH) :
                                  (S_AXI_DATA_WIDTH / M_AXI_DATA_WIDTH),
    localparam bit UPSIZE       = (S_AXI_DATA_WIDTH < M_AXI_DATA_WIDTH) ? 1'b1 : 1'b0,
    localparam bit DOWNSIZE     = (S_AXI_DATA_WIDTH > M_AXI_DATA_WIDTH) ? 1'b1 : 1'b0,

    // Skid buffer packed widths
    localparam int AR_WIDTH = AXI_ID_WIDTH + AXI_ADDR_WIDTH + 8 + 3 + 2 + 1 + 4 + 3 + 4 + 4 + AXI_USER_WIDTH,
    localparam int R_WIDTH  = S_AXI_DATA_WIDTH + 2 + AXI_USER_WIDTH + 1 + AXI_ID_WIDTH
) (
    // Clock and Reset
    input  logic                        aclk,
    input  logic                        aresetn,

    //==========================================================================
    // Slave AXI Read Interface
    //==========================================================================

    // Read Address Channel
    input  logic [AXI_ID_WIDTH-1:0]     s_axi_arid,
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axi_araddr,
    input  logic [7:0]                  s_axi_arlen,
    input  logic [2:0]                  s_axi_arsize,
    input  logic [1:0]                  s_axi_arburst,
    input  logic                        s_axi_arlock,
    input  logic [3:0]                  s_axi_arcache,
    input  logic [2:0]                  s_axi_arprot,
    input  logic [3:0]                  s_axi_arqos,
    input  logic [3:0]                  s_axi_arregion,
    input  logic [AXI_USER_WIDTH-1:0]   s_axi_aruser,
    input  logic                        s_axi_arvalid,
    output logic                        s_axi_arready,

    // Read Data Channel
    output logic [AXI_ID_WIDTH-1:0]     s_axi_rid,
    output logic [S_AXI_DATA_WIDTH-1:0] s_axi_rdata,
    output logic [1:0]                  s_axi_rresp,
    output logic                        s_axi_rlast,
    output logic [AXI_USER_WIDTH-1:0]   s_axi_ruser,
    output logic                        s_axi_rvalid,
    input  logic                        s_axi_rready,

    //==========================================================================
    // Master AXI Read Interface
    //==========================================================================

    // Read Address Channel
    output logic [AXI_ID_WIDTH-1:0]     m_axi_arid,
    output logic [AXI_ADDR_WIDTH-1:0]   m_axi_araddr,
    output logic [7:0]                  m_axi_arlen,
    output logic [2:0]                  m_axi_arsize,
    output logic [1:0]                  m_axi_arburst,
    output logic                        m_axi_arlock,
    output logic [3:0]                  m_axi_arcache,
    output logic [2:0]                  m_axi_arprot,
    output logic [3:0]                  m_axi_arqos,
    output logic [3:0]                  m_axi_arregion,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_aruser,
    output logic                        m_axi_arvalid,
    input  logic                        m_axi_arready,

    // Read Data Channel
    input  logic [AXI_ID_WIDTH-1:0]     m_axi_rid,
    input  logic [M_AXI_DATA_WIDTH-1:0] m_axi_rdata,
    input  logic [1:0]                  m_axi_rresp,
    input  logic                        m_axi_rlast,
    input  logic [AXI_USER_WIDTH-1:0]   m_axi_ruser,
    input  logic                        m_axi_rvalid,
    output logic                        m_axi_rready
);

## 8.4	2.6.4 Burst Length Conversion

### 8.4.1	Ratio Calculation

Same as write converter:

// Direction-aware, same as the write converter (2.5.4)
localparam int RATIO = (S_AXI_DATA_WIDTH < M_AXI_DATA_WIDTH)
                       ? (M_AXI_DATA_WIDTH / S_AXI_DATA_WIDTH)
                       : (S_AXI_DATA_WIDTH / M_AXI_DATA_WIDTH);
localparam int RATIO_LOG2 = $clog2(RATIO);

// upsize: New ARLEN = ceil((start_lane + ARLEN + 1) / RATIO) - 1
// (round UP; the start lane counts for mid-word bursts, see 2.6.5)
// downsize: split into master bursts of <= 256 beats (see 2.6.5)

### 8.4.2	Examples

Table 2.18: Read Burst Length Conversion

| S_DATA | M_DATA | Ratio | S_ARLEN | S_beats | M_ARLEN | M_beats |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | 512 | 8 | 7 | 8 | 0 | 1 |
| 64 | 512 | 8 | 15 | 16 | 1 | 2 |
| 64 | 512 | 8 | 31 | 32 | 3 | 4 |

## 8.5	2.6.5 Address Channel Handling

### 8.5.1	AR Passthrough with Adjustment

Same two directions as the write converter (see 2.5.4 for why the upsize divide rounds up):

// narrow -> wide (upsize): beats combine, round up; the start lane
// counts toward the wide total for mid-word bursts
assign m_axi_arlen  = 8'(((w_ar_lane + 10'(int_arlen) + 10'(WIDTH_RATIO))
                          / 10'(WIDTH_RATIO)) - 10'd1);

// wide -> narrow (downsize): each wide beat becomes RATIO narrow beats,
// and the product can exceed both AWLEN's 8 bits and the 256-beat legal
// maximum -- so one slave burst is SPLIT into master bursts of <= 256
// beats (same mechanism as the write converter, see 2.5.5)
assign m_axi_arlen  = 8'(w_this_beats - 9'd1);  // min(remaining, 256)

// size is the master's own full width, not a shift of ARSIZE
assign m_axi_arsize = MASTER_SIZE[2:0];

For a split read the slave must still see ONE burst: each master burst returns its own RLAST, and every one except the final master burst’s is masked out of the upsize, whose accumulation simply continues across the boundary. The masking is safe by construction — 256 narrow beats is a whole number of wide beats at every ratio, so a masked boundary can never land mid-accumulation. A one-bit flag queue, pushed per issued AR and popped per master RLAST, says which burst is final.

On the UPSIZE path the issued address is aligned down to the master data width — the slave returns whole wide words — and mid-word INCR starts are handled by the R slicer: the burst-length FIFO carries the start lane alongside the narrow length, m_axi_arlen counts it (ceil((start_lane + narrow_beats) / RATIO) wide beats), and axi_data_dnsize slices the burst’s FIRST wide word from that lane, so the narrow master receives the bytes it actually addressed. Later wide words slice from lane 0. FIXED/WRAP keep the wide-aligned requirement (asserted in simulation). The downsize path issues narrow accesses and passes the address through the splitter unmodified apart from the per-burst advance:

localparam int ALIGN_BITS = $clog2(M_STRB_WIDTH);
assign aligned_araddr = {int_araddr[AXI_ADDR_WIDTH-1:ALIGN_BITS],
                         {ALIGN_BITS{1'b0}}};
assign m_axi_araddr   = aligned_araddr;

### 8.5.2	Burst-Length FIFO

Only the wide→narrow R data path needs one — that is the converter’s UPSIZE mode (S narrower than M: wide master read data sliced down to narrow slave beats through axi_data_dnsize). The downsize block ignores a burst_start pulse while a burst is active and keeps no length queue of its own, so framing only the first burst would collapse N read bursts into one — bursts 2..N would drain with narrow_last never asserting. A small queue holds one narrow ARLEN per outstanding burst: its head feeds the downsize, and it pops as each narrow burst completes, so every burst is framed however they overlap.

It is an inline circular buffer rather than a fifo_sync instance, deliberately: this converter is widely instantiated and a submodule here would add a filelist dependency to every consumer. It stores the narrow length AND the burst’s start lane (mid-word INCR starts, CONV-006) — ID is carried on the AXI channels, not through this queue — and AR is back-pressured when full, so it cannot overflow.

localparam int BLEN_FIFO_DEPTH = 16;
            localparam int BLEN_AW         = $clog2(BLEN_FIFO_DEPTH);

            localparam int R_LANE_W = $clog2(WIDTH_RATIO);
            logic [7:0]          blen_mem      [BLEN_FIFO_DEPTH];
            logic [R_LANE_W-1:0] blen_lane_mem [BLEN_FIFO_DEPTH];  // start lane
            logic [BLEN_AW:0]    blen_wptr, blen_rptr;   // extra MSB for full/empty
            logic               w_blen_push, w_blen_pop;

            // AR accepted -> enqueue its narrow length. A slave-side (narrow)
            // burst completes on its last-beat handshake -> dequeue.
            assign w_blen_push = int_ar_valid && int_ar_ready;
            assign w_blen_pop  = int_r_valid && int_r_ready && int_rlast;

The narrow→wide R data path (the converter’s DOWNSIZE mode) needs no such queue; its generate branch ties the shared handshake wires off inert.

## 8.6	2.6.6 Read Data Channel

### 8.6.1	Downsize Instance

axi_data_dnsize #(
    .WIDE_WIDTH      (M_AXI_DATA_WIDTH),
    .NARROW_WIDTH    (S_AXI_DATA_WIDTH),
    .WIDE_SB_WIDTH   (2),          // RRESP
    .NARROW_SB_WIDTH (2),
    .SB_BROADCAST    (1),          // Broadcast RRESP
    .TRACK_BURSTS    (1),
    .BURST_LEN_WIDTH (8)
) u_r_dnsize (
    .aclk            (aclk),
    .aresetn         (aresetn),
    // burst framing is MANDATORY with TRACK_BURSTS(1): burst_len is in
    // NARROW beats - 1, and burst_start is LEVEL-HELD while the length
    // FIFO is non-empty (the dnsize samples it only when idle, which is
    // what frames every overlapping burst); leaving them off silently
    // produces framing with no LAST (see 2.3.9)
    .burst_len       (w_blen_rd_data),
    .burst_start     (w_blen_rd_valid),
    .start_lane      (w_blen_rd_lane[$clog2(WIDTH_RATIO)-1:0]),
    .wide_valid      (m_axi_rvalid),
    .wide_ready      (m_axi_rready),
    .wide_data       (m_axi_rdata),
    .wide_sideband   (m_axi_rresp),
    .wide_last       (m_axi_rlast),
    .narrow_valid    (int_r_valid),
    .narrow_ready    (int_r_ready),
    .narrow_data     (int_rdata),
    .narrow_sideband (int_rresp),
    .narrow_last     (int_rlast)
);

## 8.7	2.6.7 RLAST Generation

There is no local RLAST tracker in the converter. The downsize block generates narrow_last itself in TRACK_BURSTS mode, framed by the burst-length FIFO of 2.6.5: each accepted AR pushes its narrow-beat length, the FIFO head drives burst_len/burst_start, and the dnsize counts narrow beats against it — int_rlast comes out of the dnsize and passes to s_axi_rlast through the R skid.

(An earlier revision showed a standalone counter loading (arlen + 1) * RATIO - 1, the same xRATIO framing 2.3.4 calls out as the classic mis-framing bug. No such multiply exists anywhere in the converter: in UPSIZE mode the slave side is already narrow, so the ARLEN pushed into the FIFO is already in narrow-beat units — blen_mem[...] <= int_arlen; stores it unchanged.)

## 8.8	2.6.8 RID Handling

### 8.8.1	ID Passthrough

RID is sampled from the master side and HELD:

// from the RTL: latch rid/ruser on every master R handshake; AXI4
// keeps RID constant across a transaction's beats, so "most recent
// rid" is correct for whatever aggregated beat is being emitted
`ALWAYS_FF_RST(aclk, aresetn,
    ... else if (m_axi_rvalid && m_axi_rready) begin
        r_rid_held   <= m_axi_rid;
        r_ruser_held <= m_axi_ruser;
    end
)
assign int_rid = r_rid_held;

### 8.8.2	Ordering Constraint (applies to both directions)

The master-side slave must return responses in AR/AW issue order across ALL IDs. Neither converter carries a transaction ID through its data path, so neither can attribute an out-of-order completion to the burst it belongs to.

Read side: the burst-length queue holds {narrow ARLEN, start lane} and nothing else, and s_axi_rid is a single register latched on every master R handshake. If a slave returns ID1’s data before ID0’s, ID1’s beats are counted against ID0’s queue head, s_axi_rlast fires at the wrong boundary, one aggregated wide beat can mix two transactions, and s_axi_rid is whichever ID handshaked most recently.

Write side: the split queue is popped by split_b_pop = m_axi_bvalid && m_axi_bready – any arriving B pops the head, whatever its ID – and m_axi_bready = split_b_final ? int_b_ready : 1'b1 swallows a non-final response unconditionally. A B that arrives out of order is folded into the wrong burst’s accumulator: the burst it actually belonged to never gets its response (the master hangs), the other burst’s response is built from the wrong beats, and every later burst’s framing is shifted by one.

AXI4 permits both behaviours. A slave may complete different-ID transactions out of order and may interleave read data across IDs, and a multi-ported DDR controller normally does. Satisfying “one outstanding transaction per ID” is not sufficient – two IDs with one transaction each meet that and still break the fold.

Safe configurations are: an in-order master-side slave, or a single ID outstanding at a time. Nothing in the RTL enforces or detects a violation, so a breach corrupts data silently rather than failing (tracked as CONV-001).

## 8.9	2.6.9 Resource Utilization

### 8.9.1	Typical Resources (64→512 UPSIZE, ratio 8, ID=4)

The burst-length FIFO exists only in the converter’s UPSIZE mode (see 2.6.5), so the configuration here is upsize — an earlier revision labeled this table 512→64 (DOWNSIZE), a mode in which that FIFO is tied off and does not exist. Hand estimates, not synthesis results, except the FIFO, which is counted from its declarations.

AR skid buffer:      ~150 flip-flops
R dnsize data path:  ~600 flip-flops, ~50 LUTs  (single-buffer slicer)
Burst-length FIFO:   186 flip-flops
                     (16 x 8b length + 16 x 3b start lane + two 5b pointers)
Control logic:       ~80 LUTs

Total: ~940 flip-flops, ~130 LUTs (sum of the lines above)

There is no separate “burst tracker” block — 2.6.7 explains RLAST comes from the dnsize’s own counter, framed by the FIFO. The R data path is the single-buffer axi_data_dnsize — the only implementation; its ping-pong DUAL_BUFFER variant was removed, see 2.3.8.

## 8.10	2.6.10 Timing

### 8.10.1	Latency

Table 2.20: Read Converter Latency

| Path | Latency |
| --- | --- |
| AR passthrough | 1-2 cycles (skid) |
| First R beat | 1 cycle (load buffer) |
| Subsequent R beats | 1 beat/cycle |

### 8.10.2	Throughput

- AR channel: 1 transaction/cycle

- R channel: the downsize accepts its next wide beat during the last narrow beat — 0.992 beats/cycle measured in simple mode (TRACK_BURSTS=1 pays one bubble per burst boundary and measures ~0.93; see 2.3)

## 8.11	2.6.11 Usage Example

axi4_dwidth_converter_rd #(
    .S_AXI_DATA_WIDTH (128),
    .M_AXI_DATA_WIDTH (32),
    .AXI_ID_WIDTH     (8),
    .AXI_ADDR_WIDTH   (32),
    .SKID_DEPTH_AR    (2),
    .SKID_DEPTH_R     (4)
) u_conv (
    .aclk             (aclk),
    .aresetn          (aresetn),
    // slave side: s_axi_ar*/... (full AXI4 channel set)
    .s_axi_arvalid   (cpu_arvalid),
    .s_axi_arready   (cpu_arready),
    // ...
    // master side: m_axi_* toward the narrow fabric
    .m_axi_arvalid   (mem_arvalid),
    .m_axi_arready   (mem_arready)
    // ...
);

All channel ports carry the s_axi_/m_axi_ prefix; the full list is the module header. Skid depths are per channel — there is no single SKID_DEPTH.

Next: Protocol Conversion Overview

# 9	2.7 AXIL Wide-Alignment Converters

axil_to_axi4_wide_align_wr and axil_to_axi4_wide_align_rd are drop-in replacements for axi4_dwidth_converter_wr / _rd for the one case those converters get wrong: a slave-side master that is effectively AXI4-Lite – every transfer single-beat, narrow, at a sub-row address.

## 9.1	2.7.1 Why They Exist

The generic converters place data by beat counter. axi_data_upsize aggregates N narrow beats into one wide beat in arrival order, and axi_data_dnsize emits narrow beats the same way. That is correct for a true multi-beat narrow burst.

An AXI4-Lite master never issues one. Every transfer is a single beat with awlen=0, and AXI4 requires the data for a narrow single-beat transfer to sit in the byte lanes selected by the address, not in lane 0. With the generic upsize, every such write lands at lane 0 whatever awaddr says, and every such read is taken from lane 0 rather than the requested slot.

These two modules place the beat at byte offset (addr & wide_mask) * 8 instead, which is what the specification requires.

The path this matters on is the bridge generator’s: it wraps an AXI4-Lite master with axi4_slave_wr, which presents fub_axi_* with awlen=0, a narrow awsize and awburst=01. That is exactly the shape the generic upsize mishandles.

## 9.2	2.7.2 Parameters

Both modules take the same set. Defaults differ from the generic converters because the intended direction is narrow master to wide slave.

Wide-Alignment Converter Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| S_AXI_DATA_WIDTH | 32 | Slave-side (narrow, AXIL-like) data width |
| M_AXI_DATA_WIDTH | 256 | Master-side (wide) data width |
| AXI_ID_WIDTH | 8 | ID width |
| AXI_ADDR_WIDTH | 32 | Address width |
| AXI_USER_WIDTH | 1 | User-signal width |
| SKID_DEPTH_AW | 2 | Accepted for API parity; unused. Tied into the module’s _unused sink |
| SKID_DEPTH_W | 2 | Likewise unused (write module) |
| SKID_DEPTH_B | 2 | Likewise unused (write module) |
| SKID_DEPTH_AR | 2 | Likewise unused (read module) |
| SKID_DEPTH_R | 4 | Likewise unused (read module) |

The SKID_DEPTH_* parameters exist so these modules can be swapped in for the generic converters without editing the instantiation. They buffer nothing – the RTL lists them explicitly in a verilator lint_off UNUSED sink rather than leaving the non-use implicit.

Several AW/AR sideband inputs are declared and deliberately unused for the same reason – awlen, awsize, awburst, awlock, awqos, awregion and wlast on the write side. A single-beat AXIL transfer carries no useful burst description, so the module ignores it rather than trusting it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	3.1 Protocol Conversion Overview

Protocol converters bridge components that speak different bus protocols — the glue in any SoC that mixes AXI4, AXI4-Lite, and APB.

## 10.1	3.1.1 Available Converters

### 10.1.1	AXI4 to AXI4-Lite (Protocol Downgrade)

Table 3.1: AXI4 to AXI4-Lite Converters

| Module | Function | Test Status |
| --- | --- | --- |
| axi4_to_axil4_rd | Read burst decomposition | 5/5 configs passing |
| axi4_to_axil4_wr | Write burst decomposition | 5/5 configs passing |
| axi4_to_axil4 | Full bidirectional wrapper | Composed |

### 10.1.2	AXI4-Lite to AXI4 (Protocol Upgrade)

Table 3.2: AXI4-Lite to AXI4 Converters

| Module | Function | Test Status |
| --- | --- | --- |
| axil4_to_axi4_rd | Read protocol upgrade | 7/7 passing |
| axil4_to_axi4_wr | Write protocol upgrade | 7/7 passing |
| axil4_to_axi4 | Full bidirectional wrapper | Composed |

### 10.1.3	Other Protocol Converters

Table 3.3: Other Protocol Converters

| Module | Function | Status |
| --- | --- | --- |
| axi4_to_apb4_convert | Full AXI4-to-APB bridge | Production |
| peakrdl_to_cmdrsp | Register interface adapter | Production |
| uart_axil_bridge | UART to AXI4-Lite | Implemented (rtl/uart_to_axil4/, dv/tests/test_uart_axil_bridge.py) |
| axil4_to_wb4 | AXI4-Lite to Wishbone B4 (pipelined or classic) | Implemented (dv/tests/test_axil4_to_wb4.py, formal on the core) |
| wb4_to_axil4 | Wishbone B4 to AXI4-Lite; merges AXI’s two response channels back into B4’s in-order termination | Implemented (dv/tests/test_wb4_to_axil4.py, formal on the core) |
| apb4_to_axi4 / apb5_to_axi4 | APB4 / APB5 completer in, single-beat AXI4 requester out (the mirror of axi4_to_apb4); APB5 USER mapped onto AXI USER | Implemented (dv/tests/test_apb{4,5}_to_axi4.py; the bridge’s APB master ports, BRIDGE-014) |
| apb_cmdrsp_to_axi4 | The requester half of the two above: APB cmd/rsp stream to one AXI4 transaction per transfer | Implemented (covered by the wrappers’ tests) |

## 10.2	3.1.2 Protocol Comparison

### 10.2.1	Figure 3.1: Protocol Feature Comparison

Protocol Comparison

### 10.2.2	Feature Matrix

Table 3.4: Protocol Feature Comparison

| Feature | AXI4 | AXI4-Lite | APB |
| --- | --- | --- | --- |
| Channels | 5 (AW, W, B, AR, R) | 5 (simplified) | 1 (combined) |
| Bursts | Up to 256 beats | Single beat only | Single beat |
| Out-of-order | Yes (ID-based) | No | No |
| Pipelining | Yes | Optional | No (2-phase) |
| Data widths | 8-1024 bits | 32/64 bits | 8-32 bits |

## 10.3	3.1.3 Conversion Strategies

### 10.3.1	AXI4 to AXI4-Lite

Challenge: Decompose multi-beat bursts into sequential single beats.

Strategy: 1. Accept AXI4 burst transaction 2. Issue N single-beat AXI4-Lite transactions 3. Aggregate responses 4. Return combined response to AXI4 master

Complexity: Medium (FSM-based decomposition)

### 10.3.2	AXI4-Lite to AXI4

Challenge: Add AXI4 burst signals with appropriate defaults.

Strategy: 1. Pass through all AXI4-Lite signals 2. Add default values for burst signals (LEN=0, SIZE=$clog2(DATA_WIDTH/8), BURST=INCR) 3. Add default IDs (configurable)

Complexity: Very low (combinational only)

### 10.3.3	AXI4 to APB

Challenge: Bridge 5-channel AXI4 to 2-phase APB protocol.

Strategy: 1. Accept AXI4 transaction 2. Execute APB setup phase 3. Execute APB access phase 4. Return AXI4 response

Complexity: High (full protocol FSM)

## 10.4	3.1.4 Performance Characteristics

Table 3.5: Protocol Converter Performance

| Converter | Single-Beat | Burst (N) | Area |
| --- | --- | --- | --- |
| axi4_to_axil4 | 0 cycles | slave-limited, ~N cycles best case (see 3.2.6) | ~450 LUTs |
| axil4_to_axi4 | 0 cycles | N/A | ~110 LUTs |
| axi4_to_apb4 | 3-5 cycles | ~2N+1 APB cycles sustained (see 3.4.11) | ~150 LUTs (hand estimate; matches 3.4.10) |

### 10.4.1	Key Observations

- Zero-overhead upgrade: AXI4-Lite to AXI4 is purely combinational

- Burst rate: AXI4 to AXI4-Lite adds no wait state between beats — decomposed requests stream at whatever rate the AXIL4 slave sustains (a zero-wait-state slave is served one beat per cycle)

- APB overhead: 3-5 cycle minimum per APB transaction

## 10.5	3.1.5 Use Case Guidelines

### 10.5.1	When to Use AXI4 to AXI4-Lite

Use when: - CPU/DMA with burst support needs simple peripheral access - Want to simplify peripheral design (no burst handling) - Data widths match (no width conversion needed) - Burst performance is not critical

Avoid when: - High-bandwidth streaming data - Latency-critical paths - Many back-to-back bursts

### 10.5.2	When to Use AXI4-Lite to AXI4

Use when: - Legacy AXI4-Lite IP connects to AXI4 fabric - Designing simple peripheral for AXI4 system - Want zero-overhead protocol upgrade - Don’t need burst capability

Avoid when: - Need actual burst support (use native AXI4) - Width conversion needed (use width converters)

### 10.5.3	When to Use AXI4 to APB

Use when: - AXI4 masters need APB peripheral access - Building CPU-to-peripheral bridges - Integrating legacy APB devices

Avoid when: - High-performance paths (APB is slow) - Streaming data (APB is sequential)

## 10.6	3.1.6 Integration Patterns

### 10.6.1	Pattern 1: CPU to Peripherals

CPU (AXI4) → AXI4-to-AXIL4 → AXIL4 Peripherals
                          → AXI4-to-APB → APB Peripherals

### 10.6.2	Pattern 2: Simple IP in AXI4 Fabric

Simple IP (AXIL4) → AXIL4-to-AXI4 → AXI4 Crossbar

### 10.6.3	Pattern 3: Mixed System

CPU (AXI4) → AXI4 Crossbar → DDR (AXI4)
                           → AXI4-to-AXIL4 → Config Regs (AXIL4)
                           → AXI4-to-APB → UART, GPIO (APB)

Next: AXI4 to AXI4-Lite

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	3.2 AXI4 to AXI4-Lite Converter

The axi4_to_axil4 converter family decomposes AXI4 burst transactions into sequential AXI4-Lite single-beat transactions.

## 11.1	3.2.1 Module Organization

axi4_to_axil4.sv          # Full bidirectional wrapper
├── axi4_to_axil4_rd.sv   # Read path converter
└── axi4_to_axil4_wr.sv   # Write path converter

### 11.1.1	Design Philosophy

Read and write are separate modules, which buys: - Independent optimization - Selective instantiation (read-only, write-only, or both) - Simpler verification (test paths independently)

## 11.2	3.2.2 Read Path (axi4_to_axil4_rd)

### 11.2.1	Figure 3.2: AXI4 to AXI4-Lite Read Path

AXI4 to AXIL4 Read

### 11.2.2	Operation

Single-Beat (ARLEN=0):

Cycle 0: AR accepted (passthrough)
Cycle 1: R returned (passthrough)
Total: 0 extra cycles (pure passthrough)

Multi-Beat (ARLEN=N-1):

Cycle 0:   AR[0] issued to AXIL4
Cycle 1:   R[0] received, AR[1] issued
Cycle 2:   R[1] received, AR[2] issued
...
           R[N-1] received (RLAST)

AR issue and R return are fully independent: the address FSM streams one AXIL4 AR per beat as fast as m_axil_arready accepts, with no reference to R at all — nothing in the AR path looks at m_axil_rvalid. Against a slave with multi-cycle response latency the requests pipeline ahead of the data, and end-to-end duration is set by the slave, not the converter.

Bursts do not overlap each other: AR is held off until the burst in flight has delivered its last beat, because the burst-tracking registers hold one burst at a time.

### 11.2.3	State Machine

typedef enum logic [1:0] {
    RD_IDLE       = 2'b00,   // No burst in flight
    RD_BURST      = 2'b01,   // Issuing the AXIL4 reads of a burst
    RD_LAST_BEAT  = 2'b10    // Issuing the burst's final read
} rd_state_t;

A single-beat read (ARLEN == 0) never leaves RD_IDLE: it takes the passthrough leg rather than the decomposition FSM. That is also why the one-outstanding-burst guard cannot rely on the FSM state alone.

### 11.2.4	Implementation

// Read state machine
    typedef enum logic [1:0] {
        RD_IDLE       = 2'b00,
        RD_BURST      = 2'b01,
        RD_LAST_BEAT  = 2'b10
    } rd_state_t;

    rd_state_t r_rd_state, w_rd_next_state;

    always_ff @(posedge aclk or negedge aresetn) begin
        if (!aresetn) begin
            r_rd_state <= RD_IDLE;
        end else begin
            r_rd_state <= w_rd_next_state;
        end
    end

    // Read next state logic
    always_comb begin
        w_rd_next_state = r_rd_state;
        case (r_rd_state)
            RD_IDLE: begin
                if (s_axi_arvalid && s_axi_arready) begin
                    if (s_axi_arlen == 0)
                        w_rd_next_state = RD_IDLE;  // Single beat, stay idle
                    else
                        w_rd_next_state = RD_BURST;  // Start burst decomposition
                end
            end
            RD_BURST: begin
                if (m_axil_arvalid && m_axil_arready) begin
                    if (r_ar_beat_count == r_ar_len - 1)
                        w_rd_next_state = RD_LAST_BEAT;
                end
            end
            RD_LAST_BEAT: begin
                if (m_axil_arvalid && m_axil_arready)
                    w_rd_next_state = RD_IDLE;
            end
            default: w_rd_next_state = RD_IDLE;
        endcase
    end

## 11.3	3.2.3 Write Path (axi4_to_axil4_wr)

### 11.3.1	Figure 3.3: AXI4 to AXI4-Lite Write Path

AXI4 to AXIL4 Write

### 11.3.2	Operation

Single-Beat (AWLEN=0):

Cycle 0: AW+W accepted (passthrough)
Cycle 1: B returned (passthrough)
Total: 0 extra cycles

Multi-Beat (AWLEN=N-1):

Cycle 0:   AW[0] + W[0] issued
Cycle 1:   B[0] received, AW[1] + W[1] issued
...
           B[N-1] received (timing set by the AXIL4 slave)
           B[N-1] received

As on the read path, AW/W issue is independent of B return — the write FSM streams beats as fast as the AXIL4 slave accepts them, and mid-burst B responses are consumed immediately (m_axil_bready = s_axi_bready || !w_b_all_beats_done). The single B the master receives is emitted once every decomposed write has responded. Bursts do not overlap each other: the next AW is held off until the burst in flight has its B accepted.

### 11.3.3	AW/W Synchronization Challenge

AXI4 allows AW and W to arrive in any order: - AW before W - W before AW - Interleaved

### 11.3.4	Solution: W gated on the AW that owns it

W is gated directly against the state of the AW it belongs to, rather than through pending-arrival flags, so a data beat cannot reach the AXIL4 slave ahead of its own address:

- Burst capture cycle. When a burst’s AW is being taken into the registers, W is held off for that cycle (w_burst_capture). Without it, the first W beat of the next burst slips out while the previous burst is still finishing, and the AXIL4 slave pairs an address from burst N with data from burst N+1.

- During a burst. W passes only once this burst’s AW has actually been sent (r_aw_sent), or is being sent in this very cycle.

- Single beats. Straight passthrough; AW and W are presented together.

// s_axi_awready restricts the block to the ACCEPT cycle. Without it
    // a burst AW merely PARKED by the one-outstanding guard (awvalid=1,
    // awready=0) deadlocked the write already in flight (CONV-007).
    wire w_burst_capture = !r_aw_active && s_axi_awvalid && s_axi_awready &&
                           (s_axi_awlen > 0);

    assign m_axil_wvalid = w_burst_capture ? 1'b0 :
                           r_aw_active     ? (s_axi_wvalid &&
                                              (r_aw_sent ||
                                               (m_axil_awvalid && m_axil_awready))) :
                                             s_axi_wvalid;
    assign s_axi_wready  = w_burst_capture ? 1'b0 :
                           r_aw_active     ? (m_axil_wready &&
                                              (r_aw_sent ||
                                               (m_axil_awvalid && m_axil_awready))) :
                                             m_axil_wready;

The failure this guards against only appears back to back — the next AW arriving the cycle after WR_LAST_BEAT completes. A sequential test with a cooldown between bursts never opens that window, which is why it survived the FUB tests and was caught by a bridge-level probe. Classic bring-up bug, in other words: it hides from every directed test and fires the moment real traffic shows up.

### 11.3.5	Response Aggregation

// from the RTL: accumulate per response, reset on AW acceptance
always_ff @(posedge aclk or negedge aresetn) begin
    ...
    if (s_axi_awvalid && s_axi_awready)
        r_b_resp_accum <= 2'b00;
    if (m_axil_bvalid && m_axil_bready)
        if (m_axil_bresp > r_b_resp_accum)
            r_b_resp_accum <= m_axil_bresp;
end

// live-beat fold at emission (see 4.3.3 for why the registered-only
// form was a shipped bug)
assign w_b_resp_worst = (m_axil_bresp > r_b_resp_accum) ? m_axil_bresp
                                                        : r_b_resp_accum;
assign s_axi_bresp = w_b_resp_worst;

## 11.4	3.2.4 Bidirectional Wrapper (axi4_to_axil4)

### 11.4.1	Composition Pattern

module axi4_to_axil4 #(
    parameter int AXI_ID_WIDTH   = 8,
    parameter int AXI_ADDR_WIDTH = 32,
    parameter int AXI_DATA_WIDTH = 32,
    parameter int AXI_USER_WIDTH = 1
) (
    // aclk/aresetn + full s_axi_*/m_axil_* channel set
);

    axi4_to_axil4_rd #(
        .AXI_ID_WIDTH   (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH (AXI_USER_WIDTH)
    ) u_rd_converter ( /* read channels */ );

    axi4_to_axil4_wr #(
        .AXI_ID_WIDTH   (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH (AXI_USER_WIDTH)
    ) u_wr_converter ( /* write channels */ );

endmodule

## 11.5	3.2.5 Resource Utilization

Table 3.6: AXI4 to AXIL4 Resources

| Module | Registers | LUTs | BRAM |
| --- | --- | --- | --- |
| axi4_to_axil4_rd | ~120 | ~180 | 0 |
| axi4_to_axil4_wr | ~150 | ~220 | 0 |
| axi4_to_axil4 (combined) | ~270 | ~400 | 0 |

## 11.6	3.2.6 Timing

### 11.6.1	Throughput

Table 3.7: AXI4 to AXIL4 Throughput

| Transaction Type | Behaviour |
| --- | --- |
| Single-beat | Passthrough; no converter-inserted wait state |
| N-beat burst | One AXIL4 access per beat; requests stream at the slave’s accept rate, independent of responses |
| Back-to-back bursts | Serialized: the next AR/AW waits for the previous burst’s last beat |

The previous “50% for any burst” figure was not measured and does not follow from the design: the converter adds no wait state between beats, so the achievable rate is whatever the AXIL4 slave sustains. A measured characterization per slave latency has not been done and is not claimed here.

### 11.6.2	Latency

Table 3.8: AXI4 to AXIL4 Latency

| Transaction Type | Latency |
| --- | --- |
| Single-beat | 0 extra cycles |
| N-beat burst | slave-limited; requests stream independently of responses (see 3.2.6) |

## 11.7	3.2.7 Testing

Test Suite: both directions green — test_axi4_to_axil4_{wr,rd}.py, 5 parametrized configurations each, every scenario suite asserted (the counts below are scenario groups within those runs)

Table 3.9: Test Coverage Summary

| Test Category | Tests | Status |
| --- | --- | --- |
| Single-beat read | 4 | Pass |
| Multi-beat read | 6 | Pass |
| Single-beat write | 4 | Pass |
| Multi-beat write | 6 | Pass |
| Mixed traffic | 8 | Pass |
| Error injection | 6 | Pass |
| Edge cases | 8 | Pass |

Next: AXI4-Lite to AXI4

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	3.3 AXI4-Lite to AXI4 Converter

The axil4_to_axi4 converter family upgrades AXI4-Lite single-beat transactions to full AXI4 protocol by adding default burst signals.

## 12.1	3.3.1 Module Organization

axil4_to_axi4.sv          # Full bidirectional wrapper
├── axil4_to_axi4_rd.sv   # Read path converter
└── axil4_to_axi4_wr.sv   # Write path converter

## 12.2	3.3.2 Design Philosophy

Zero-Overhead Upgrade: - Purely combinational logic - No state machines or buffers - Adds default values for missing AXI4 signals

Why This Works: - AXI4-Lite is a subset of AXI4 - All AXI4-Lite transactions are single-beat - Missing AXI4 signals have well-defined defaults

## 12.3	3.3.3 Signal Mapping

### 12.3.1	Address Channel Signals

Table 3.10: AR Channel Mapping

| AXI4-Lite Signal | AXI4 Signal | Default/Mapping |
| --- | --- | --- |
| ARADDR | ARADDR | Passthrough |
| ARPROT | ARPROT | Passthrough |
| ARVALID | ARVALID | Passthrough |
| ARREADY | ARREADY | Passthrough |
| - | ARLEN | 8’h00 (single beat) |
| - | ARSIZE | $clog2(DATA_WIDTH/8) |
| - | ARBURST | 2’b01 (INCR) |
| - | ARLOCK | 1’b0 (normal) |
| - | ARCACHE | 4’b0011 (bufferable, modifiable) |
| - | ARQOS | 4’(DEFAULT_QOS), parameter, default 0 |
| - | ARID | Configurable default |

### 12.3.2	Data Channel Signals

Table 3.11: R Channel Mapping

| AXI4-Lite Signal | AXI4 Signal | Default/Mapping |
| --- | --- | --- |
| RDATA | RDATA | Passthrough |
| RRESP | RRESP | Passthrough |
| RVALID | RVALID | Passthrough |
| RREADY | RREADY | Passthrough |
| - | RLAST | 1’b1 (always last) |
| - | RID | Matches ARID |

## 12.4	3.3.4 Read Path (axil4_to_axi4_rd)

### 12.4.1	Figure 3.4: AXI4-Lite to AXI4 Read Path

AXIL4 to AXI4 Read

### 12.4.2	Implementation

module axil4_to_axi4_rd #(
    // Width Configuration
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_DATA_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,

    // Default Values for AXI4-only Fields
    parameter int DEFAULT_ID        = 0,
    parameter int DEFAULT_REGION    = 0,
    parameter int DEFAULT_QOS       = 0,

    // Skid Buffer Depths (for timing closure)


    // Calculated Parameters
    localparam int STRB_WIDTH = AXI_DATA_WIDTH / 8,
    localparam int SIZE_VAL   = $clog2(STRB_WIDTH)  // ARSIZE for full width
) (
    // Clock and Reset
    input  logic                        aclk,
    input  logic                        aresetn,

    //==========================================================================
    // Slave AXI4-Lite Read Interface (Input - Simplified Protocol)
    //==========================================================================

    // Read Address Channel
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axil_araddr,
    input  logic [2:0]                  s_axil_arprot,
    input  logic                        s_axil_arvalid,
    output logic                        s_axil_arready,

    // Read Data Channel
    output logic [AXI_DATA_WIDTH-1:0]   s_axil_rdata,
    output logic [1:0]                  s_axil_rresp,
    output logic                        s_axil_rvalid,
    input  logic                        s_axil_rready,

    //==========================================================================
    // Master AXI4 Read Interface (Output - Full Protocol)
    //==========================================================================

    // Read Address Channel
    output logic [AXI_ID_WIDTH-1:0]     m_axi_arid,
    output logic [AXI_ADDR_WIDTH-1:0]   m_axi_araddr,
    output logic [7:0]                  m_axi_arlen,
    output logic [2:0]                  m_axi_arsize,
    output logic [1:0]                  m_axi_arburst,
    output logic                        m_axi_arlock,
    output logic [3:0]                  m_axi_arcache,
    output logic [2:0]                  m_axi_arprot,
    output logic [3:0]                  m_axi_arqos,
    output logic [3:0]                  m_axi_arregion,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_aruser,
    output logic                        m_axi_arvalid,
    input  logic                        m_axi_arready,

    // Read Data Channel
    input  logic [AXI_ID_WIDTH-1:0]     m_axi_rid,
    input  logic [AXI_DATA_WIDTH-1:0]   m_axi_rdata,
    input  logic [1:0]                  m_axi_rresp,
    input  logic                        m_axi_rlast,
    input  logic [AXI_USER_WIDTH-1:0]   m_axi_ruser,
    input  logic                        m_axi_rvalid,
    output logic                        m_axi_rready
);

Key Points: - No registers - purely combinational - RLAST from AXI4 is ignored (always 1 for AXIL4) - RID from AXI4 is ignored (no ID tracking in AXIL4)

## 12.5	3.3.5 Write Path (axil4_to_axi4_wr)

### 12.5.1	Figure 3.5: AXI4-Lite to AXI4 Write Path

AXIL4 to AXI4 Write

### 12.5.2	Implementation

module axil4_to_axi4_wr #(
    // Width Configuration
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_DATA_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,

    // Default Values for AXI4-only Fields
    parameter int DEFAULT_ID        = 0,
    parameter int DEFAULT_REGION    = 0,
    parameter int DEFAULT_QOS       = 0,

    // Skid Buffer Depths (for timing closure)



    // Calculated Parameters
    localparam int STRB_WIDTH = AXI_DATA_WIDTH / 8,
    localparam int SIZE_VAL   = $clog2(STRB_WIDTH)  // AWSIZE for full width
) (
    // Clock and Reset
    input  logic                        aclk,
    input  logic                        aresetn,

    //==========================================================================
    // Slave AXI4-Lite Write Interface (Input - Simplified Protocol)
    //==========================================================================

    // Write Address Channel
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axil_awaddr,
    input  logic [2:0]                  s_axil_awprot,
    input  logic                        s_axil_awvalid,
    output logic                        s_axil_awready,

    // Write Data Channel
    input  logic [AXI_DATA_WIDTH-1:0]   s_axil_wdata,
    input  logic [STRB_WIDTH-1:0]       s_axil_wstrb,
    input  logic                        s_axil_wvalid,
    output logic                        s_axil_wready,

    // Write Response Channel
    output logic [1:0]                  s_axil_bresp,
    output logic                        s_axil_bvalid,
    input  logic                        s_axil_bready,

    //==========================================================================
    // Master AXI4 Write Interface (Output - Full Protocol)
    //==========================================================================

    // Write Address Channel
    output logic [AXI_ID_WIDTH-1:0]     m_axi_awid,
    output logic [AXI_ADDR_WIDTH-1:0]   m_axi_awaddr,
    output logic [7:0]                  m_axi_awlen,
    output logic [2:0]                  m_axi_awsize,
    output logic [1:0]                  m_axi_awburst,
    output logic                        m_axi_awlock,
    output logic [3:0]                  m_axi_awcache,
    output logic [2:0]                  m_axi_awprot,
    output logic [3:0]                  m_axi_awqos,
    output logic [3:0]                  m_axi_awregion,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_awuser,
    output logic                        m_axi_awvalid,
    input  logic                        m_axi_awready,

    // Write Data Channel
    output logic [AXI_DATA_WIDTH-1:0]   m_axi_wdata,
    output logic [STRB_WIDTH-1:0]       m_axi_wstrb,
    output logic                        m_axi_wlast,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_wuser,
    output logic                        m_axi_wvalid,
    input  logic                        m_axi_wready,

    // Write Response Channel
    input  logic [AXI_ID_WIDTH-1:0]     m_axi_bid,
    input  logic [1:0]                  m_axi_bresp,
    input  logic [AXI_USER_WIDTH-1:0]   m_axi_buser,
    input  logic                        m_axi_bvalid,
    output logic                        m_axi_bready
);

## 12.6	3.3.6 Bidirectional Wrapper

module axil4_to_axi4 #(
    parameter int AXI_ID_WIDTH   = 8,
    parameter int AXI_ADDR_WIDTH = 32,
    parameter int AXI_DATA_WIDTH = 32,
    parameter int AXI_USER_WIDTH = 1,
    parameter int DEFAULT_ID     = 0,
    parameter int DEFAULT_REGION = 0,
    parameter int DEFAULT_QOS    = 0
) (
    // ... all port declarations
);

    axil4_to_axi4_rd #(
        .AXI_ID_WIDTH    (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH  (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH  (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH  (AXI_USER_WIDTH),
        .DEFAULT_ID      (DEFAULT_ID),
        .DEFAULT_REGION  (DEFAULT_REGION),
        .DEFAULT_QOS     (DEFAULT_QOS)
    ) u_rd_converter (/* connections */);

    axil4_to_axi4_wr #(
        .AXI_ID_WIDTH    (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH  (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH  (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH  (AXI_USER_WIDTH),
        .DEFAULT_ID      (DEFAULT_ID),
        .DEFAULT_REGION  (DEFAULT_REGION),
        .DEFAULT_QOS     (DEFAULT_QOS)
    ) u_wr_converter (/* connections */);

endmodule

## 12.7	3.3.7 Resource Utilization

Table 3.12: AXIL4 to AXI4 Resources

| Module | Registers | LUTs |
| --- | --- | --- |
| axil4_to_axi4_rd | 0 | ~50 |
| axil4_to_axi4_wr | 0 | ~60 |
| axil4_to_axi4 (combined) | 0 | ~110 |

Note: Zero registers — purely combinational logic.

## 12.8	3.3.8 Timing

Table 3.13: AXIL4 to AXI4 Performance

| Metric | Value |
| --- | --- |
| Latency | 0 cycles |
| Throughput | 100% |
| Max frequency | Wire speed |

## 12.9	3.3.9 Testing

Test Suite: 14 tests passing

Table 3.14: Test Coverage Summary

| Test Category | Tests | Status |
| --- | --- | --- |
| Single-beat read | 3 | Pass |
| Single-beat write | 3 | Pass |
| Mixed traffic | 4 | Pass |
| Default ID verification | 2 | Pass |
| Edge cases | 2 | Pass |

## 12.10	3.3.10 Usage Example

axil4_to_axi4 #(
    .AXI_DATA_WIDTH (32),
    .AXI_ADDR_WIDTH (32),
    .AXI_ID_WIDTH   (8),
    .DEFAULT_ID     (8'h05),
    .DEFAULT_QOS    (0)
) u_axil2axi (
    .aclk           (aclk),
    .aresetn        (aresetn),
    // AXI4-Lite slave side: s_axil_*
    .s_axil_arvalid (lite_arvalid),
    .s_axil_arready (lite_arready),
    .s_axil_araddr  (lite_araddr),
    // ...
    // AXI4 master side: m_axi_* (ID/len/size driven from parameters;
    // the conversion is combinational, no internal pipelining)
    .m_axi_arvalid  (fab_arvalid),
    .m_axi_arready  (fab_arready)
    // ...
);

Next: AXI4 to APB

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	3.4 AXI4 to APB Converter

The axi4_to_apb4_convert module provides full protocol translation from AXI4 to APB, enabling AXI4 masters to access APB peripherals.

## 13.1	3.4.1 Overview

AXI4 and APB differ on just about every axis:

Table 3.15: AXI4 vs APB Comparison

| Aspect | AXI4 | APB |
| --- | --- | --- |
| Channels | 5 (AW, W, B, AR, R) | 1 (combined) |
| Phases | Pipelined | 2-phase (setup, access) |
| Bursts | Up to 256 beats | Single transfer |
| Address width | Up to 64 bits | Typically 32 bits |
| Data width | 8-1024 bits | 8-32 bits |

## 13.2	3.4.2 Block Diagram

### 13.2.1	Figure 3.6: AXI4 to APB Converter

AXI4 to APB

## 13.3	3.4.3 Interface Specification

### 13.3.1	Parameters

axi4_to_apb4_convert — the conversion core:

Table 3.16: AXI4 to APB Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| AXI_ADDR_WIDTH | int | 32 | AXI4 address width |
| AXI_DATA_WIDTH | int | 32 | AXI4 data width |
| AXI_ID_WIDTH | int | 8 | AXI4 ID width |
| AXI_USER_WIDTH | int | 1 | AXI4 user-signal width |
| APB_ADDR_WIDTH | int | 32 | APB address width |
| APB_DATA_WIDTH | int | 32 | APB data width |
| SIDE_DEPTH | int | 6 | Side-channel FIFO depth (ID/user carried past the APB hop) |

axi4_to_apb4_shim — the wrapper integrators actually instantiate. It adds the channel FIFOs around the core, and is what the bridge generator emits:

Table 3.17: AXI4 to APB Shim Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| DEPTH_AW / DEPTH_AR | int | 2 | Write/read address channel FIFO depth |
| DEPTH_W / DEPTH_R | int | 4 | Write/read data channel FIFO depth |
| DEPTH_B | int | 2 | Write response channel FIFO depth |
| SIDE_DEPTH | int | 4 | Side-channel FIFO depth |
| APB_CMD_DEPTH | int | 4 | APB command FIFO depth |
| APB_RSP_DEPTH | int | 4 | APB response FIFO depth |
| USE_JOHNSON | int | 0 | CDC-FIFO pointer encoding: 0 = Gray (power-of-2 depth only), 1 = Johnson |
| USE_2_PHASE_CDC | bit | 1 | Deprecated and ignored. Declared for compatibility and never referenced, so setting it changes nothing |
| AXI_/APB_ widths | int | as above | Same width parameters as the core |

The shim’s defaults differ from the core’s SIDE_DEPTH (4 vs 6); it sets its own rather than inheriting.

### 13.3.2	Ports

// The conversion CORE. It does not sit on the AXI4 channels directly:
// the shim packs each channel into a packet plus a count and hands those
// in, which is why the ports below are r_s_axi_*_pkt rather than the raw
// AXI signals. Integrators instantiate axi4_to_apb4_shim instead; its
// parameters are in the shim table above.

module axi4_to_apb4_convert #(
    parameter int SIDE_DEPTH        = 6,
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_DATA_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,
    parameter int APB_ADDR_WIDTH    = 32,
    parameter int APB_DATA_WIDTH    = 32,
    parameter int AXI_WSTRB_WIDTH   = AXI_DATA_WIDTH / 8,
    parameter int APB_WSTRB_WIDTH   = APB_DATA_WIDTH / 8,
    // short and calculated params
    parameter int AW                = AXI_ADDR_WIDTH,
    parameter int DW                = AXI_DATA_WIDTH,
    parameter int IW                = AXI_ID_WIDTH,
    parameter int UW                = AXI_USER_WIDTH,
    parameter int SW                = AXI_DATA_WIDTH / 8,
    parameter int APBAW             = APB_ADDR_WIDTH,
    parameter int APBDW             = APB_DATA_WIDTH,
    parameter int APBSW             = APB_DATA_WIDTH / 8,
    parameter int AXI2APBRATIO      = DW / APBDW,
    parameter int AWSize            = IW + AW + 8 + 3 + 2 + 1 + 4 + 3 + 4 + 4 + UW,
    parameter int WSize             = DW + SW + 1 + UW,
    parameter int BSize             = IW + 2 + UW,
    parameter int ARSize            = IW + AW + 8 + 3 + 2 + 1 + 4 + 3 + 4 + 4 + UW,
    parameter int RSize             = IW + DW + 2 + 1 + UW,
    parameter int APBCmdWidth       = APBAW + APBDW + APBSW + 3 + 1 + 1 + 1,
    parameter int APBRspWidth       = APBDW + 1 + 1 + 1,
    parameter int SideSize          = 1 + IW + 1 + UW
) (
    // Clock and Reset
    input  logic                    aclk,
    input  logic                    aresetn,

    // Inputs from axi_slave_stub
    input  logic [AWSize-1:0]       r_s_axi_aw_pkt,
    input  logic [3:0]              r_s_axi_aw_count,
    input  logic                    r_s_axi_awvalid,
    output logic                    w_s_axi_awready,

    input  logic [WSize-1:0]        r_s_axi_w_pkt,
    input  logic                    r_s_axi_wvalid,
    output logic                    w_s_axi_wready,

    output logic [BSize-1:0]        r_s_axi_b_pkt,
    output logic                    w_s_axi_bvalid,
    input  logic                    r_s_axi_bready,

    input  logic [ARSize-1:0]       r_s_axi_ar_pkt,
    input  logic [3:0]              r_s_axi_ar_count,
    input  logic                    r_s_axi_arvalid,
    output logic                    w_s_axi_arready,

    output logic [RSize-1:0]        r_s_axi_r_pkt,
    output logic                    w_s_axi_rvalid,
    input  logic                    r_s_axi_rready,

    // APB Master Interface
    output logic                    w_cmd_valid,
    input  logic                    r_cmd_ready,
    output logic [APBCmdWidth-1:0]  r_cmd_data,

    input  logic                    r_rsp_valid,
    output logic                    w_rsp_ready,
    input  logic [APBRspWidth-1:0]  r_rsp_data
);

## 13.4	3.4.4 Clock Domains

The shim is a two-clock block. aclk runs the AXI side and pclk the APB side, with independent resets (aresetn, presetn). Commands cross aclk→pclk and responses pclk→aclk through gray-pointer asynchronous FIFOs (gaxi_fifo_async).

The reset behaviour is the part worth knowing. Each domain resets its own pointer and its crossed copy of the remote pointer from its LOCAL reset, so resetting one side alone leaves that side self-consistent — both pointers at 0, meaning empty. Because the pointers are absolute positions rather than toggle parity, an independent reset of one side cannot fabricate or swallow a transfer. The earlier 2-phase handshake could, and the failure was permanent rather than transient: the response stream ends up offset by one and every read returns the previous read’s data.

CDC FIFO depths follow the APB cmd/rsp queue depths, floored at 4:

localparam int CDC_CMD_DEPTH = (APB_CMD_DEPTH < 4) ? 4 : APB_CMD_DEPTH;
localparam int CDC_RSP_DEPTH = (APB_RSP_DEPTH < 4) ? 4 : APB_RSP_DEPTH;

A power-of-2 depth is preferred: the default USE_JOHNSON=0 Gray encoding requires it (see the parameter table above).

## 13.5	3.4.5 State Machine

### 13.5.1	Figure 3.7: AXI4 to APB FSM

AXI4 to APB FSM

### 13.5.2	States

The convert core runs TWO state machines, and neither of them touches PSEL/PENABLE — the APB setup/access phases belong to apb4_master, on the far side of the shim’s CDC. The core’s job is packetizing.

Command FSM — walks the accepted AXI burst and emits one APB command packet per APB-width beat into the cmd stream:

typedef enum logic [2:0] {
    IDLE  = 3'b001,   // wait for an accepted AW+W pair or AR
    READ  = 3'b010,   // emit one read command per beat
    WRITE = 3'b100    // emit one write command per beat
} apb_state_t;

Writes are preferred over reads when both are pending. Each command carries first/last flags; last is set on the final beat, which is also when the AXI address channel is released (awready/arready). A data pointer sub-divides each AXI beat into AXI2APBRATIO APB beats when the widths differ, and the next address comes from the shared axi_gen_addr (INCR/FIXED/WRAP per the AXI burst type).

Response FSM — assembles returning rsp packets into AXI responses:

typedef enum logic [1:0] {
    RSP_IDLE   = 2'b01,   // no response in flight
    RSP_ACTIVE = 2'b10    // consuming rsp packets for one transaction
} rsp_state_t;

Read data re-packs APB-width beats into AXI-width R beats; write responses collapse into the single B, worst response winning. The first flag in the command stream is what arms this FSM — stamping it from transaction progress rather than the previous command state is the fix for a hang recorded in the RTL history (a first=0 first command froze the response FSM in RSP_IDLE).

## 13.6	3.4.6 Burst Handling

### 13.6.1	Burst Decomposition

AXI4 bursts are decomposed into sequential APB transfers:

AXI4: AWADDR=0x1000, AWLEN=3 (4 beats)

APB sequence:
  Transfer 0: PADDR=0x1000
  Transfer 1: PADDR=0x1004
  Transfer 2: PADDR=0x1008
  Transfer 3: PADDR=0x100C

### 13.6.2	Address Calculation

The per-beat address comes from the shared axi_gen_addr block, keyed on the AXI burst type (INCR/FIXED/WRAP) and size — the convert core does not roll its own increment, and there is no APB-phase state here (PREADY pacing happens in apb4_master beyond the CDC; see 3.4.5).

## 13.7	3.4.7 Address Width Adaptation

### 13.7.1	64-bit to 32-bit Conversion

// Truncate upper address bits
assign paddr = s_awaddr[APB_ADDR_WIDTH-1:0];

// Optional: Check for out-of-range access
wire w_addr_oor = |s_awaddr[AXI_ADDR_WIDTH-1:APB_ADDR_WIDTH];

## 13.8	3.4.8 Error Response Mapping

Table 3.18: Error Mapping

| APB Signal | AXI4 Response |
| --- | --- |
| PSLVERR = 0 | OKAY (2’b00) |
| PSLVERR = 1 | SLVERR (2’b10) |

### 13.8.1	Error Aggregation

Each rsp packet carries the APB pslverr for its beat. The response FSM accumulates across the transaction’s packets and maps any error to SLVERR on the AXI side — B for writes, per-beat RRESP for reads (the error lands on the beats it belongs to, not smeared across the burst). There is no APB-phase sampling here; pslverr is captured by apb4_master and travels back in the packet.

## 13.9	3.4.9 Implementation

The implementation is the two FSMs of 3.4.5 plus the packet plumbing: the shim skid-buffers each AXI channel, packs AW/W/AR into per-channel packets for the core, and carries the core’s cmd/rsp streams through gray-pointer async FIFOs to apb4_master, which owns the actual APB setup/access phases. The full source is projects/components/converters/rtl/axi4_to_apb4_convert.sv (core) and axi4_to_apb4_shim.sv (integration); their headers document the packet formats. No separate simplified listing is maintained here — an earlier one drifted into describing states the RTL never had.

## 13.10	3.4.10 Resource Utilization

State machine:        ~50 LUTs, ~20 regs
Address logic:        ~30 LUTs, ~40 regs
Data buffering:       ~10 LUTs, ~70 regs
Control:              ~60 LUTs, ~20 regs

Total: ~150 LUTs, ~150 regs

## 13.11	3.4.11 Timing

### 13.11.1	Timing Analysis

Table 3.19: APB Converter Timing

| Operation | Cycles |
| --- | --- |
| Single write | 3-4 APB-side + ~2 pclk in + ~2 aclk out for the CDC crossings (see 3.4.4) |
| Single read | 3-4 (setup + access + R) |
| N-beat write burst | ~2N+1 APB-side + the same two CDC crossings, paid once per command run |
| N-beat read burst | ~2N+1 APB-side |

The 2N+1 shape falls out of the FSM: the first transfer pays IDLE→SETUP→ACCESS (3 pclk), but with commands queued in the skid the ACCESS state hands straight back to SETUP — every subsequent transfer is SETUP→ACCESS, 2 pclk, PREADY permitting.

### 13.11.2	Throughput

Best case (sustained): 1 transfer per 2 pclk cycles once the first transfer’s extra IDLE cycle is paid With slow PREADY: Additional cycles per transfer

## 13.12	3.4.12 Usage Example

Integrators instantiate the SHIM (the core’s cmd/rsp packet ports are internal plumbing):

axi4_to_apb4_shim #(
    .AXI_ID_WIDTH     (8),
    .AXI_ADDR_WIDTH   (32),
    .AXI_DATA_WIDTH   (32),
    .APB_ADDR_WIDTH   (32),
    .APB_DATA_WIDTH   (32),
    .APB_CMD_DEPTH    (4),
    .APB_RSP_DEPTH    (4)
) u_axi2apb (
    // AXI side clock/reset
    .aclk             (aclk),
    .aresetn          (aresetn),
    // APB side clock/reset -- this is a TWO-CLOCK block (see 3.4.4)
    .pclk             (pclk),
    .presetn          (presetn),

    // AXI4 slave interface: s_axi_aw*/w*/b*/ar*/r* (full signal list in
    // the module header)
    .s_axi_awvalid    (cpu_awvalid),
    .s_axi_awready    (cpu_awready),
    // ...

    // APB master interface
    .m_apb_PSEL       (periph_psel),
    .m_apb_PENABLE    (periph_penable),
    .m_apb_PADDR      (periph_paddr),
    .m_apb_PWRITE     (periph_pwrite),
    .m_apb_PWDATA     (periph_pwdata),
    .m_apb_PSTRB      (periph_pstrb),
    .m_apb_PPROT      (periph_pprot),
    .m_apb_PRDATA     (periph_prdata),
    .m_apb_PSLVERR    (periph_pslverr),
    .m_apb_PREADY     (periph_pready)
);

Next: AXI4 to APB5 Shim

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	AXI4 to APB5 Shim

Module: axi4_to_apb5_shim.sv Filelist: rtl/filelists/axi4_to_apb5_shim.f (-f’s the APB4 shim’s closure)

## 14.1	Overview

APB5 changes less than the version bump suggests. The transfer protocol is pure APB4 — PSEL/PENABLE phases, PREADY, PSLVERR, all unchanged — and everything new lives in the sideband: requester-driven user signals on one side, completer-driven wakeup/user signals on the other. So this block is a thin pin-superset wrapper over axi4_to_apb4_shim. The entire protocol engine — CDC, cmd/rsp FIFOs, burst decomposition, response assembly — is the APB4 shim instantiated unchanged, with all 58 ports and 18 parameters forwarded 1:1.

The APB5 additions on the requester surface:

| Signal | Dir | Handling |
| --- | --- | --- |
| m_apb_PAUSER[APB_AUSER_WIDTH-1:0] | out | tied '0 — nothing upstream sources it (AXI USER bits do not map onto APB user semantics) |
| m_apb_PWUSER[APB_WUSER_WIDTH-1:0] | out | tied '0 |
| m_apb_PWAKEUP | in | accepted and terminated |
| m_apb_PRUSER[APB_RUSER_WIDTH-1:0] | in | accepted and terminated |
| m_apb_PBUSER[APB_BUSER_WIDTH-1:0] | in | accepted and terminated |

One thing to get right before you wire this up. User-signal widths default to 1 here, but rtl/amba/apb5/apb5_slave.sv defaults its own to 4, so the two do NOT line up out of the box — set APB_{A,W,R,B}USER_WIDTH to match whatever completer you attach. The signal set is otherwise pin-for-pin with that slave, including the repo’s convention that PWAKEUP rides completer→requester.

## 14.2	Parameters

The width and depth parameters are the same as the APB4 shim’s – see Table 3.17 – with the APB5 user-signal widths (APB_AUSER_WIDTH, APB_WUSER_WIDTH, APB_RUSER_WIDTH, APB_BUSER_WIDTH) added for the sideband above.

One parameter is inert, and I’d rather call it out here than watch someone wire a knob to it:

AXI4 to APB5 Shim – Inert Parameter

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| USE_2_PHASE_CDC | bit | 1 | Deprecated and ignored. This shim wraps axi4_to_apb4_shim and forwards the parameter to it; that module declares it // deprecated, ignored and never references it, so it has no effect at any level |

## 14.3	Design Notes

Deriving the wrapper from the APB4 shim’s actual port surface — rather than reimplementing the conversion — means the APB4 engine’s fixes and its dependency closure are inherited automatically. The closure filelist simply -f’s the APB4 shim’s. When a future consumer needs real PAUSER/PWUSER sourcing or PWAKEUP-gated clocking, those grow here without touching the conversion core.

## 14.4	Related Modules

The bridge generator instantiates this shim for protocol = "apb5" slaves (BRIDGE-002 A5-3c) through the same component path as the APB4 shim — the Axi4ToApbShim component takes protocol='apb5' and wires the five extra pairs. Verified by projects/components/bridge/dv/tests/test_bridge_1x2_rw_apb5.py (APB4 BFM legally drives the port: same transfer protocol).

## 14.5	Navigation

Next: AXI4 to AXI5-Lite

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	AXI4 to AXI5-Lite

Modules: axi4_to_axil5.sv, axi4_to_axil5_wr.sv, axi4_to_axil5_rd.sv Filelists: rtl/filelists/axi4_to_axil5{,_wr,_rd}.f (each -f’s the AXI4-Lite converter’s closure)

## 15.1	Overview

AXI5-Lite changes less than the version bump suggests, and in exactly the way APB5 does: the transfer protocol is pure AXI4-Lite – single-beat, no burst, no ID, same handshakes and response codes – and everything new is sideband. So these blocks are thin wrappers over axi4_to_axil4_{wr,rd}. Burst decomposition, address incrementing, response folding and the one-outstanding guards are that module, instantiated unchanged. What lives here is the sideband, and one timing decision it forces.

axi4_to_axil5 pairs the two halves exactly as axi4_to_axil4 pairs its own, and contains nothing but the two instantiations.

The AXI4 slave surface is the full AXI4 interface, not a subset. A master upstream of this converter may be feeding a fabric whose other branches reach AXI4 slaves, so nothing is dropped from the boundary on the grounds that this particular downstream path cannot use it.

## 15.2	Where each signal gets its value

Every AXI5-Lite signal is in one of three groups. This is the whole content of the block, so it is worth reading as a table rather than as prose.

AXI4 to AXI5-Lite – Sideband Disposition

| Group | Signals | Handling |
| --- | --- | --- |
| FORWARDED | awlock/arlock, awuser/aruser, wuser | carries the AXI4 value, gated by ENABLE_LOCK / ENABLE_USER |
| FORWARDED (response) | buser -> s_axi_buser, ruser -> s_axi_ruser | returned onto the AXI4 response channel, gated by ENABLE_USER |
| TIED | awloop, awmpam, awmecid, awnsaid, awtrace, wpoison (and the AR equivalents) | driven '0; AXI4 has no source for them |
| TERMINATED | bloop, btrace, rloop, rtrace, rpoison | completer-driven, nothing on the AXI4 side to return them to |

Two consequences of that table are worth stating outright.

The tied group has no ENABLE_ parameter, and that is deliberate. A knob that cannot change the design’s behaviour is worse than no knob: a reader sets it and believes something happened. ENABLE_LOCK and ENABLE_USER exist because they gate a real choice; ENABLE_TRACE, ENABLE_MPAM and the rest do not exist at all. The width parameters (USER_WIDTH, LOOP_WIDTH, MPAM_WIDTH, MECID_WIDTH, NSAID_WIDTH) do remain, because they set the port shape.

The tied group’s ports still exist. An AXI5-Lite boundary whose shape changes with a config knob cannot be wired to a fixed external completer, so the ports are always present and always driven – zero rather than floating.

rpoison deserves its own note. AXI4 has no poison bit, so a poisoned read beat arriving from an AXI5-Lite completer cannot be signalled to the AXI4 master through any protocol field. It is terminated rather than folded into RRESP, because turning poison into SLVERR would invent an error the completer never reported. A design that must observe poison needs an AXI5 master port, not an AXI4 one.

## 15.3	The AW/AR sideband is held, not passed through

This is the one place the wrapper is not trivial, and it is worth understanding before modifying it.

The core decomposes: one AXI4 AW handshake becomes N AXI5-Lite AW handshakes, one per beat. And axi4_to_axil4_wr drops s_axi_awready as soon as it accepts the AW (s_axi_awready = !r_aw_active && !r_wr_outstanding && ...). So from the cycle after acceptance, an AXI4 master is free to present the next transaction’s AW signals – new address, new USER, new LOCK – while beats 2..N of the current burst are still going out.

A combinational passthrough of the sideband therefore stamps those later beats with the wrong values. The core has the identical problem with the address and solves it with a capture-and-hold mux:

assign m_axil_awaddr = r_aw_active ? r_aw_addr : s_axi_awaddr;

r_aw_active is registered, so it is still 0 during the accept cycle itself and 1 for every beat after. The wrapper cannot see that signal, so it reconstructs the same selector from outside:

wire w_aw_accept = s_axi_awvalid && s_axi_awready;
// ... captured into r_held_aw{lock,user} on w_aw_accept ...
wire [AXI_USER_WIDTH-1:0] w_awuser_sel = w_aw_accept ? s_axi_awuser
                                                     : r_held_awuser;

Accept cycle takes the live value; every beat after takes the captured one – identical to the core’s mux, so the sideband travels with the address it was issued against.

The first version of this RTL passed it through combinationally. The failure is not subtle once you can see it: with two overlapping writes, burst A’s beats carry burst B’s USER from beat 0, because B’s AW is already on the bus. It is invisible to sequential traffic, which is why the test that catches it (test_axi4_to_axil5.py, full level) issues two writes concurrently rather than one after another.

W beats need no hold. The core assigns m_axil_wdata = s_axi_wdata directly – W is 1:1 with the AXI4 side – so wuser is a plain passthrough.

## 15.4	Reset style

These modules use always_ff @(posedge aclk or negedge aresetn) rather than the components-area ALWAYS_FF_RST macro, matching the AXI4-Lite converters they wrap. Using the macro here while the core stays manual would make one compiled design half synchronous-reset and half asynchronous – Verilator says so with SYNCASYNCNET. Converting the whole area is tracked as CONV-009, with what it actually costs recorded there.

## 15.5	Verification

projects/components/converters/dv/tests/test_axi4_to_axil5.py covers only what the wrapper adds; the transfer protocol is already covered by the AXI4-Lite suites, and duplicating it would test code this module does not contain. Eight configurations across three levels check that forwarded values arrive and stay with their burst, that tied signals read 0 (never X) at every handshake, that the response sideband returns on the AXI4 channels, and that an ENABLE_*=0 build produces zeros from non-zero AXI4 traffic. The overlapping-burst case was confirmed RED against the unfixed RTL and GREEN after.

## 15.6	Related Modules

The bridge generator instantiates these converters for protocol = "axil5" slaves through the same component path as the AXI4-Lite ones – the Axi4ToAxilShim component takes protocol='axil5'. Which sideband groups are live comes from the port’s axi5_features, which on an axil5 port accepts only user and exclusive: naming a tied group there would suggest it changes something, so the validator rejects it rather than ignoring it. Verified by projects/components/bridge/dv/tests/test_bridge_1x2_rw_axil5.py.

## 15.7	Navigation

Next: PeakRDL Adapter

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	3.5 PeakRDL Adapter

The peakrdl_to_cmdrsp module drives a PeakRDL-generated register block from a command/response stream. Data flows cmd/rsp -> PeakRDL: cmd_* are inputs and regblk_* request signals are outputs, with the register block’s acks and read data coming back in. Fair warning — the name reads in the opposite order to the dataflow. The ports below are authoritative.

## 16.1	Overview

PeakRDL generates register blocks with a selectable cpuif, and this adapter mates with the passthrough cpuif (regblk_req / req_is_wr / wr_biten / stall / ack / err — NOT the APB cpuif’s PSEL/PENABLE pins, so if you came here looking for PSEL, back up). It:

- Decouples the register interface from the implementation

- Provides a clean handshake protocol

- Enables pipelined register access

- Supports custom control logic integration

## 16.2	Parameters

Table 3.20: PeakRDL Adapter Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | int | 12 | Address width (register-block sized) |
| DATA_WIDTH | int | 32 | Data width |

## 16.3	Ports

The full port list, straight from the RTL:

module peakrdl_to_cmdrsp #(
    parameter int ADDR_WIDTH = 12,  // Address width for cmd/rsp interface
    parameter int DATA_WIDTH = 32   // Must match PeakRDL generation (typically 32)
) (
    // Clock and Reset
    input  logic                    aclk,
    input  logic                    aresetn,

    // =========================================================================
    // CMD/RSP Interface (rtldesignsherpa standard)
    // =========================================================================
    // Command Channel
    input  logic                    cmd_valid,
    output logic                    cmd_ready,
    input  logic                    cmd_pwrite,         // 1=write, 0=read
    input  logic [ADDR_WIDTH-1:0]   cmd_paddr,          // Byte address
    input  logic [DATA_WIDTH-1:0]   cmd_pwdata,         // Write data
    input  logic [DATA_WIDTH/8-1:0] cmd_pstrb,          // Byte strobes

    // Response Channel
    output logic                    rsp_valid,
    input  logic                    rsp_ready,
    output logic [DATA_WIDTH-1:0]   rsp_prdata,         // Read data
    output logic                    rsp_pslverr,        // Error flag

    // =========================================================================
    // PeakRDL Passthrough Interface
    // =========================================================================
    output logic                    regblk_req,         // Request -- HELD until ack
    output logic                    regblk_req_is_wr,   // Write flag
    output logic [ADDR_WIDTH-1:0]   regblk_addr,        // Address
    output logic [DATA_WIDTH-1:0]   regblk_wr_data,     // Write data
    output logic [DATA_WIDTH-1:0]   regblk_wr_biten,    // Write bit enables
    input  logic                    regblk_req_stall_wr, // Write stall
    input  logic                    regblk_req_stall_rd, // Read stall
    input  logic                    regblk_rd_ack,      // Read acknowledge
    input  logic                    regblk_rd_err,      // Read error
    input  logic [DATA_WIDTH-1:0]   regblk_rd_data,     // Read data
    input  logic                    regblk_wr_ack,      // Write acknowledge
    input  logic                    regblk_wr_err       // Write error
);

## 16.4	Functional Description

Direction: cmd/rsp -> PeakRDL. cmd_* arrive from upstream (e.g. the APB shim’s packet stream); the adapter drives the register block’s regblk_* request signals and returns its acks as rsp packets.

### 16.4.1	Write

Cycle 0: cmd_valid, cmd_pwrite=1 accepted (cmd_ready high in CMD_IDLE);
         regblk_req driven the same cycle, strobes converted to bit
         enables
Cycle N: regblk_wr_ack -> rsp queued (RSP_VALID), pslverr from
         regblk_wr_err
         (if regblk_req_stall_wr: state parks in CMD_STALLED and
         retries when the stall clears)
Cycle M: rsp_valid && rsp_ready -> back to idle

`regblk_req` is HELD asserted from acceptance through the ack cycle
(`(cmd_state == CMD_WAIT_ACK) || (CMD_IDLE && cmd_valid)`) -- it is
not a one-cycle strobe. The generated passthrough regblock samples the
request level and needs the hold; a consumer that starts a new access
every cycle req is high would double-fire non-idempotent registers
(TASK-064 records the earlier strobe mis-documentation).

### 16.4.2	Read

Same shape via regblk_rd_ack/regblk_rd_data/regblk_rd_err, with rsp_prdata carried in the response.

### 16.4.3	3.5.5 Implementation

Two small FSMs do all the work, from the RTL:

typedef enum logic [1:0] {
    CMD_IDLE     = 2'b00,   // ready to accept a command
    CMD_WAIT_ACK = 2'b01,   // register block has the request
    CMD_STALLED  = 2'b10    // req_stall_* asserted; retry
} cmd_state_t;

typedef enum logic {
    RSP_IDLE  = 1'b0,
    RSP_VALID = 1'b1        // response held until rsp_ready
} rsp_state_t;

The command is registered on acceptance so the request can be replayed out of CMD_STALLED; in CMD_IDLE the request muxes straight from the live cmd inputs, so an unstalled single-cycle ack costs no extra latency. cmd_ready = (cmd_state == CMD_IDLE) gives one outstanding command, which is what a register block wants.

### 16.4.4	3.5.6 Resource Utilization

FSM state:            3 regs (cmd_state 2b + rsp_state 1b)
Command capture:      77 regs (pwrite 1 + paddr 12 + pwdata 32 + wr_biten 32)
Response capture:     33 regs (prdata 32 + pslverr 1)
Request replay muxes: ~77 LUTs (four 2:1 selects, 1+12+32+32 bits --
                      IDLE-vs-registered request presentation)
FSM + control decode: ~25 LUTs

Total: ~113 regs, ~100 LUTs  (counted from the declarations at the
documented defaults ADDR_WIDTH=12, DATA_WIDTH=32; all of it is live
state -- the command capture replays the request out of
CMD_STALLED/CMD_WAIT_ACK, and the response capture holds the rsp packet)

## 16.5	Waveforms

### 16.5.1	Figure 3.8: PeakRDL Adapter

PeakRDL Adapter

## 16.6	Usage Example

### 16.6.1	3.5.7 Use Cases

The adapter sits between a command/response packet stream and a PeakRDL-generated register block:

peakrdl_to_cmdrsp #(
    .ADDR_WIDTH (12),
    .DATA_WIDTH (32)
) u_adapter (
    .aclk                (aclk),
    .aresetn             (aresetn),

    // command/response stream (e.g. from apb4_slave_cdc's unpacker)
    .cmd_valid           (cmd_valid),
    .cmd_ready           (cmd_ready),
    .cmd_pwrite          (cmd_pwrite),
    .cmd_paddr           (cmd_paddr),
    .cmd_pwdata          (cmd_pwdata),
    .cmd_pstrb           (cmd_pstrb),
    .rsp_valid           (rsp_valid),
    .rsp_ready           (rsp_ready),
    .rsp_prdata          (rsp_prdata),
    .rsp_pslverr         (rsp_pslverr),

    // PeakRDL register block hookup
    .regblk_req          (hwif_req),
    .regblk_req_is_wr    (hwif_req_is_wr),
    .regblk_addr         (hwif_addr),
    .regblk_wr_data      (hwif_wr_data),
    .regblk_wr_biten     (hwif_wr_biten),
    .regblk_req_stall_wr (hwif_req_stall_wr),
    .regblk_req_stall_rd (hwif_req_stall_rd),
    .regblk_rd_ack       (hwif_rd_ack),
    .regblk_rd_err       (hwif_rd_err),
    .regblk_rd_data      (hwif_rd_data),
    .regblk_wr_ack       (hwif_wr_ack),
    .regblk_wr_err       (hwif_wr_err)
);

Typical deployments: CSR blocks behind the APB shim (see 3.4), or any fabric whose endpoint speaks the cmd/rsp packet convention.

### 16.6.2	3.5.8 Integration Example

The flow a real deployment uses (CSRs behind the APB shim):

AXI4 master → axi4_to_apb4_shim → APB bus → apb4_to_peakrdl
                                              ├─ apb4_slave_cdc  (APB -> cmd/rsp)
                                              └─ peakrdl_to_cmdrsp
                                                        │
                                              regblk_* request/ack
                                                        ↓
                                          PeakRDL-generated register block

The shim’s own cmd/rsp stream is internal — its external pins are APB. The block that turns a bus back into the cmd/rsp surface this adapter consumes is apb4_to_peakrdl (which wraps apb4_slave_cdc + peakrdl_to_cmdrsp); wire the adapter directly only when your fabric already speaks cmd/rsp packets.

The wiring is exactly the instantiation in 3.5.7; the earlier example here showed the adapter hanging off a register block’s APB port through reg_* signals that do not exist — the reversed-direction reading this page used to make.

## 16.7	Related Modules

### 16.7.1	3.5.9 APB4 Front End (apb4_to_peakrdl)

peakrdl_to_cmdrsp above takes a cmd/rsp stream. apb4_to_peakrdl is the module that produces one from an APB4 slave port, and it crosses clock domains while doing it – APB on pclk, the register block on aclk. It is a two-stage composition and adds no register logic of its own:

| Stage | Module | Domain |
| --- | --- | --- |
| 1 | apb4_slave_cdc | pclk in, aclk out |
| 2 | peakrdl_to_cmdrsp | aclk |

apb4_to_peakrdl Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | int | 12 | APB and cpuif byte address width |
| DATA_WIDTH | int | 32 | Must match the PeakRDL generation |
| PROT_WIDTH | int | 3 | PPROT width |
| STRB_WIDTH | int | DATA_WIDTH/8 | Derived; do not override |
| CDC_DEPTH | int | 2 | Command/response CDC FIFO depth (>= 2) |
| USE_JOHNSON | int | 0 | CDC pointer encoding: 0 = Gray (power-of-2 depth only), 1 = Johnson (any depth) |
| USE_2_PHASE_CDC | bit | 1 | Deprecated and ignored. Accepted for compatibility; apb4_slave_cdc does not reference it, so setting it changes nothing |

apb4_to_peakrdl Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| aclk | 1 | Input | Register-block clock; drives the cpuif master |
| aresetn | 1 | Input | Active-low reset, aclk domain |
| pclk | 1 | Input | APB clock |
| presetn | 1 | Input | Active-low reset, pclk domain |
| s_apb_PSEL | 1 | Input | APB select |
| s_apb_PENABLE | 1 | Input | APB enable |
| s_apb_PREADY | 1 | Output | APB ready |
| s_apb_PADDR | ADDR_WIDTH | Input | APB address |
| s_apb_PWRITE | 1 | Input | Write/read direction |
| s_apb_PWDATA | DATA_WIDTH | Input | Write data |
| s_apb_PSTRB | STRB_WIDTH | Input | Write strobes |
| s_apb_PPROT | PROT_WIDTH | Input | Protection attributes |
| s_apb_PRDATA | DATA_WIDTH | Output | Read data |
| s_apb_PSLVERR | 1 | Output | Slave error |
| cpuif_req | 1 | Output | Passthrough request |
| cpuif_req_is_wr | 1 | Output | Request is a write |
| cpuif_addr | ADDR_WIDTH | Output | Request address |
| cpuif_wr_data | DATA_WIDTH | Output | Write data |
| cpuif_wr_biten | DATA_WIDTH | Output | Per-bit write enables, expanded from PSTRB |
| cpuif_req_stall_wr | 1 | Input | Register block stalls a write |
| cpuif_req_stall_rd | 1 | Input | Register block stalls a read |
| cpuif_rd_ack | 1 | Input | Read acknowledge |
| cpuif_rd_err | 1 | Input | Read error, returned as PSLVERR |
| cpuif_rd_data | DATA_WIDTH | Input | Read data |
| cpuif_wr_ack | 1 | Input | Write acknowledge |
| cpuif_wr_err | 1 | Input | Write error, returned as PSLVERR |

# 17	3.7 UART to AXI4-Lite Bridge

The uart_to_axil4 block turns a two-wire serial link into an AXI4-Lite master, so a host can read and write a design’s register space over a USB serial adapter with no JTAG, no debug core and no vendor tooling. It is the access path the characterization harnesses use for board bring-up.

Three modules ship in rtl/uart_to_axil4/: the byte-level uart_rx and uart_tx, and uart_axil_bridge, which parses commands and drives the bus.

## 17.1	3.7.1 Module Organization

uart_axil_bridge instantiates four blocks:

| Instance | Module | Role |
| --- | --- | --- |
| u_uart_rx | uart_rx | Serial to byte, with a ready/valid handshake |
| u_uart_tx | uart_tx | Byte to serial |
| u_axil_wr_timing | axil4_master_wr | Write-channel skid isolation |
| u_axil_rd_timing | axil4_master_rd | Read-channel skid isolation |

The two axil4_master_* instances are the same AMBA library blocks documented in the RTL AMBA AXI4-Lite book; the bridge does not hand-roll a bus master. The SKID_DEPTH_* parameters below are passed straight through to them.

uart_rx and uart_tx use the i_clk/i_rst_n common-block naming while the bridge presents aclk/aresetn. The bridge maps them directly – there is one clock domain and no CDC anywhere in this block.

## 17.2	3.7.2 Command Protocol

Commands are ASCII, terminated by newline:

| Command | Form | Response |
| --- | --- | --- |
| Write | W <addr_hex> <data_hex>\n | OK\n |
| Read | R <addr_hex>\n | 0x<data_hex>\n |

The command letter is case-insensitive: w/W and r/R are both accepted.

Hex digits are consumed as nibbles and shifted in, with ' ', '\n' and '\r' skipped as separators. The field widths follow the bus parameters – ceil(AXIL_ADDR_WIDTH/4) address digits and ceil(AXIL_DATA_WIDTH/4) data digits, so 8 of each at the 32-bit defaults.

Because digits shift rather than fill a fixed field, the parser does not validate length. Supplying too few digits left-aligns nothing and yields a small address; supplying too many silently discards the oldest nibbles. The host is responsible for sending exactly the digit count the bus width implies.

## 17.3	3.7.3 Error Responses

A transfer that returns a non-OKAY response answers ERR\n:

| Outcome | Response |
| --- | --- |
| Write accepted (bresp == OKAY) | OK\n |
| Write rejected (SLVERR/DECERR) | ERR\n |
| Read accepted (rresp == OKAY) | 0x<data>\n |
| Read rejected (SLVERR/DECERR) | ERR\n |

A failed read returns no data at all rather than 0x followed by whatever sat on rdata, because a value that looks well-formed is worse than no value.

Until 2026-09-03 the bridge did not do this. It brought bresp and rresp back from the two axil4_master_* instances and never examined either, so a write that returned SLVERR still answered OK and a failed read returned stale bus data formatted as a normal result. A host had no way to tell a failure from a success.

The host side needed no change: uart_axi_bridge.py already returns response.strip() == b'OK' from write(), and read() already requires a 0x prefix. Both were written expecting error signalling the RTL was not producing.

## 17.4	3.7.4 Parameters

uart_axil_bridge:

uart_axil_bridge Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| AXIL_ADDR_WIDTH | 32 | AXI4-Lite address width; sets the address digit count |
| AXIL_DATA_WIDTH | 32 | AXI4-Lite data width; sets the data digit count and m_axil_wstrb width |
| CLKS_PER_BIT | 868 | Baud divisor, aclk cycles per bit. The default is 100 MHz / 115200 baud |
| SKID_DEPTH_AR | 2 | Passed to axil4_master_rd |
| SKID_DEPTH_R | 4 | Passed to axil4_master_rd |
| SKID_DEPTH_AW | 2 | Passed to axil4_master_wr |
| SKID_DEPTH_W | 4 | Passed to axil4_master_wr |
| SKID_DEPTH_B | 2 | Passed to axil4_master_wr |

uart_rx and uart_tx each take one parameter, CLKS_PER_BIT (default 868), with the same meaning.

## 17.5	3.7.5 Ports

### 17.5.1	uart_axil_bridge

uart_axil_bridge Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| aclk | 1 | Input | Bridge clock; also clocks both UART blocks |
| aresetn | 1 | Input | Active-low reset |
| i_uart_rx | 1 | Input | Serial receive line |
| o_uart_tx | 1 | Output | Serial transmit line |
| m_axil_awaddr | AXIL_ADDR_WIDTH | Output | Write address |
| m_axil_awprot | 3 | Output | Write protection attributes |
| m_axil_awvalid | 1 | Output | Write address valid |
| m_axil_awready | 1 | Input | Write address ready |
| m_axil_wdata | AXIL_DATA_WIDTH | Output | Write data |
| m_axil_wstrb | AXIL_DATA_WIDTH/8 | Output | Write byte strobes |
| m_axil_wvalid | 1 | Output | Write data valid |
| m_axil_wready | 1 | Input | Write data ready |
| m_axil_bresp | 2 | Input | Write response; a non-OKAY value yields ERR\n (3.7.3) |
| m_axil_bvalid | 1 | Input | Write response valid |
| m_axil_bready | 1 | Output | Write response ready |
| m_axil_araddr | AXIL_ADDR_WIDTH | Output | Read address |
| m_axil_arprot | 3 | Output | Read protection attributes |
| m_axil_arvalid | 1 | Output | Read address valid |
| m_axil_arready | 1 | Input | Read address ready |
| m_axil_rdata | AXIL_DATA_WIDTH | Input | Read data, returned in the 0x... response |
| m_axil_rresp | 2 | Input | Read response; a non-OKAY value yields ERR\n (3.7.3) |
| m_axil_rvalid | 1 | Input | Read data valid |
| m_axil_rready | 1 | Output | Read data ready |

### 17.5.2	uart_rx

uart_rx Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| i_clk | 1 | Input | Clock |
| i_rst_n | 1 | Input | Active-low reset |
| i_rx | 1 | Input | Serial receive line |
| o_rx_data | 8 | Output | Received byte |
| o_rx_valid | 1 | Output | Byte valid |
| i_rx_ready | 1 | Input | Consumer ready |

### 17.5.3	uart_tx

uart_tx Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| i_clk | 1 | Input | Clock |
| i_rst_n | 1 | Input | Active-low reset |
| o_tx | 1 | Output | Serial transmit line |
| i_tx_data | 8 | Input | Byte to send |
| i_tx_valid | 1 | Input | Byte valid |
| o_tx_ready | 1 | Output | Transmitter ready for the next byte |

# 18	3.8 Width-plus-Protocol Chains

A chain is a FUB-level module that wires one width converter directly to one protocol converter. Neither chain adds logic of its own: they exist because that pairing is what a real bridge instantiates, and because the bugs worth catching live at the seam between the two, not inside either one.

Both are downsizing chains – a wide AXI4 master talking to a narrow peripheral – and both were built to reproduce the back-to-back page-boundary class of defect at FUB level, where it can be driven directly, instead of only at the bridge level where it was first seen.

## 18.1	3.8.1 axi4_dwidth_to_axil4_wr_chain

Write path. The slave port is wide AXI4 (S_AXI_DATA_WIDTH); the master port is AXI4-Lite at M_AXIL_DATA_WIDTH.

| Stage | Module | What it does |
| --- | --- | --- |
| 1 | axi4_dwidth_converter_wr | Downsizes the W beats and rewrites awlen/awsize, so the next stage sees a narrow AXI4 burst of M_AXIL_DATA_WIDTH-wide beats |
| 2 | axi4_to_axil4_wr | Decomposes that narrow burst into single-beat AXI4-Lite writes |

axi4_dwidth_to_axil4_wr_chain Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| S_AXI_DATA_WIDTH | 64 | Slave-side (wide) AXI4 data width |
| M_AXIL_DATA_WIDTH | 32 | Master-side AXI4-Lite data width; the width beats are downsized to |
| AXI_ID_WIDTH | 8 | Slave-side ID width |
| AXI_ADDR_WIDTH | 32 | Address width, both sides |
| AXI_USER_WIDTH | 1 | User-signal width |

Ports are the slave-side AXI4 write channels and the master-side AXI4-Lite write channels, unchanged from the two modules it wraps – see AXI4 to AXI4-Lite for the AXI4-Lite side and the width block chapters for the AXI4 side.

## 18.2	3.8.2 axi4_dwidth_to_apb4_chain

Read path only. The write channels on the slave port are tied off inside the chain – this is a read chain, not a general bridge.

| Stage | Module | What it does |
| --- | --- | --- |
| 1 | axi4_dwidth_converter_rd | Rewrites arlen/arsize down to APB_DATA_WIDTH-wide narrow beats, and upsizes the returning R beats back to S_AXI_DATA_WIDTH |
| 2 | axi4_to_apb4_shim | Decomposes the narrow burst into single-beat APB reads |

axi4_dwidth_to_apb4_chain Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| S_AXI_DATA_WIDTH | 64 | Slave-side (wide) AXI4 data width |
| APB_DATA_WIDTH | 32 | APB data width on the master side |
| AXI_ID_WIDTH | 8 | Slave-side ID width |
| AXI_ADDR_WIDTH | 32 | AXI address width |
| APB_ADDR_WIDTH | 32 | APB address width |
| AXI_USER_WIDTH | 1 | User-signal width |
| USE_JOHNSON | 0 | CDC-FIFO pointer encoding forwarded to the shim: 0 = Gray (power-of-2 depth only), 1 = Johnson |

The direction is worth stating plainly, because the two readings of “downsize” point opposite ways. The wide master issues a wide-beat AR burst; the read converter rewrites it into narrow APB_DATA_WIDTH beats for the shim; the R data coming back is reassembled into wide beats for the master.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	3.10 AXI4-Lite to Wishbone B4 Converter

The axil4_to_wb4 converter presents an AXI4-Lite slave and drives a Wishbone B4 master. It is the bridge to drop in when an AXI4-Lite interconnect has to reach a Wishbone peripheral, and it is built from parts the library already has: the AXI4-Lite slave skids, a small conversion core, and wb4_master from rtl/amba/wb4/.

## 19.1	3.10.1 Module Organization

axil4_to_wb4.sv              # Wrapper: skids + core + wb4_master
├── axil4_slave_wr.sv        # AW / W / B skid buffers (rtl/amba/axil4)
├── axil4_slave_rd.sv        # AR / R skid buffers (rtl/amba/axil4)
├── axil4_to_wb4_core.sv     # Five channels -> one command and one response queue
└── wb4_master.sv            # Queues -> Wishbone B4 bus (rtl/amba/wb4)

## 19.2	3.10.2 Design Philosophy

One queue, in order. AXI4-Lite has separate write and read paths; Wishbone has one bus. The core merges the two paths into the single command queue of wb4_master and relies on a B4 rule to route the answers back: terminations return in issue order. A one-bit side queue, written at issue with the direction of each command, tells the core whether the oldest response belongs on B or on R. There is no ID, no reorder buffer, and no state machine.

No starvation. A write is eligible when both AW and W are present, a read when AR is. When both are eligible in the same clock the side not served last is picked, so a stream of writes cannot hold reads off the bus and vice versa.

Wishbone status becomes an AXI response, nothing more. ACK maps to OKAY, ERR to SLVERR, and RTY to the RTY_RESP parameter (SLVERR by default). AXI has no retry, so a retrying Wishbone slave is reported to the requester as an error; a retry wrapper on the FUB side of wb4_master is the place to absorb RTY transparently.

No width conversion. The AXI4-Lite and Wishbone address and data widths are the same. Put axi4_dwidth_* or the wide-alignment blocks in front when the interconnect width differs.

## 19.3	3.10.3 Block Diagram

### 19.3.1	Figure 3.12: AXI4-Lite to Wishbone B4 Converter

s_axil_aw ──► skid ──► fub_aw ─┐
 s_axil_w  ──► skid ──► fub_w  ─┤ pick        cmd_{we,adr,dat,sel} ──► wb4_master ──► m_wb_*
 s_axil_ar ──► skid ──► fub_ar ─┘ (alternate)         │
                                     │ side queue (we per issue)
 s_axil_b  ◄── skid ◄── fub_b  ◄─┐   ▼
 s_axil_r  ◄── skid ◄── fub_r  ◄─┴─ steer ◄── rsp_{status,dat} ◄── wb4_master

## 19.4	3.10.4 Interface Specification

### 19.4.1	Parameters

Table 3.26: AXI4-Lite to Wishbone Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| ADDR_WIDTH | 32 | Address width, both sides |
| DATA_WIDTH | 32 | Data width, both sides |
| SKID_DEPTH_AW/W/B/AR/R | 2 | AXI4-Lite channel skid depths (2..8) |
| CMD_DEPTH | 4 | wb4_master command queue depth |
| RSP_DEPTH | 4 | wb4_master response queue depth; bounds the transfers on the bus |
| SIDE_DEPTH | 8 | Core direction queue; at least CMD_DEPTH + RSP_DEPTH so the master sets the pipeline depth |
| CLASSIC | 0 | 0 = B4 pipelined, 1 = B4 standard (classic) mode; match the slave |
| RTY_RESP | 2’b10 | AXI response returned for a Wishbone RTY |

The core alone (axil4_to_wb4_core) takes ADDR_WIDTH, DATA_WIDTH, SIDE_DEPTH and RTY_RESP.

### 19.4.2	Ports

Table 3.27: AXI4-Lite to Wishbone Ports

| Port group | Direction | Description |
| --- | --- | --- |
| s_axil_aw*, s_axil_w*, s_axil_b* | slave | AXI4-Lite write channels (AWPROT accepted, not carried) |
| s_axil_ar*, s_axil_r* | slave | AXI4-Lite read channels |
| m_wb_CYC/STB/WE/ADR/DAT_W/SEL | out | Wishbone request |
| m_wb_STALL/ACK/ERR/RTY/DAT_R | in | Wishbone flow control and termination |
| busy | out | Anything buffered in the skids or a cycle open on the bus |

The core’s ports are the five fub_* channels of the two slave blocks on one side and the cmd_* / rsp_* queues of wb4_master on the other.

## 19.5	3.10.5 Command Formation

Table 3.28: Command Formation

| AXI4-Lite | Wishbone command |
| --- | --- |
| AWADDR, WDATA, WSTRB | we=1, adr, dat, sel = WSTRB |
| ARADDR | we=0, adr, sel = all ones |
| AWPROT / ARPROT | not carried (Wishbone has no protection bits) |

A write consumes AW and W in the same clock (fub_awready and fub_wready are the same signal), so a W beat that arrives before its AW simply waits in its skid. A command is only issued when the side queue has room; when the side queue is full the core stalls, which with SIDE_DEPTH >= CMD_DEPTH + RSP_DEPTH never happens before the master’s own queues are full.

## 19.6	3.10.6 Response Mapping

Table 3.29: Response Mapping

| Wishbone status | B / R response |
| --- | --- |
| ACK | OKAY (2’b00) |
| ERR | SLVERR (2’b10) |
| RTY | RTY_RESP (default SLVERR) |

RDATA is the Wishbone DAT_R of the termination. A response that arrives with nothing recorded in the side queue is a master protocol violation (wb4_master reports it in simulation); the core drops it so the queue cannot wedge on it.

## 19.7	3.10.7 Timing

Table 3.30: AXI4-Lite to Wishbone Timing

| Path | Clocks |
| --- | --- |
| AW+W or AR accepted at s_axil_* to STB on the bus | 2 (skid, master command queue) |
| Termination on the bus to BVALID / RVALID | 2 (master response queue, skid) |
| Sustained rate, pipelined slave | one transfer per clock per direction pair, up to RSP_DEPTH in flight |
| Sustained rate, CLASSIC=1 | one transfer at a time (B4 standard mode) |

## 19.8	3.10.8 Formal

formal/converters/axil4_to_wb4_core/ (sv2v flatten, then SymbiYosys) proves on the core: a command is issued only with a request present and carries the payload of the channel it took; B and R never fire together and each consumes exactly one response; the response steered to B or R is the oldest open transfer’s; the open count never exceeds SIDE_DEPTH; OKAY / SLVERR / RTY_RESP follow the status; and a write is never issued straight after a write while a read is pending. Covers reach both directions, the alternation, an SLVERR on B, a retry on R, and a full side queue.

## 19.9	3.10.9 Testing

dv/tests/test_axil4_to_wb4.py drives s_axil_* with the AXI4-Lite master BFMs and answers m_wb_* with the framework’s Wishbone slave BFM over a memory model (ERR and RTY windows by address) while the Wishbone monitor checks the B4 rules on the wire. The scoreboard mirrors every OKAY write byte-by-byte (strobed writes included) and checks every read against the mirror, and every response code against the window its address is in. Phases: sequential; several transfers in flight against a stalling, a slow-terminating and a mixed slave; drain with one monitor transfer per transaction and no violations. The classic build runs the same phases against a classic-mode slave and returns DECERR for a retry to prove the RTY_RESP path. Four RTL mutations (direction bit inverted, RTY mapped to OKAY, write strobes dropped, read address taken from AW) all fail the test.

## 19.10	3.10.10 Usage Example

axil4_to_wb4 #(
    .ADDR_WIDTH (32),
    .DATA_WIDTH (32),
    .CMD_DEPTH  (4),
    .RSP_DEPTH  (4),
    .SIDE_DEPTH (8),
    .CLASSIC    (0)
) u_axil_to_wb (
    .aclk           (clk),
    .aresetn        (rst_n),
    .s_axil_awaddr  (awaddr),  .s_axil_awprot (awprot), .s_axil_awvalid (awvalid), .s_axil_awready (awready),
    .s_axil_wdata   (wdata),   .s_axil_wstrb  (wstrb),  .s_axil_wvalid  (wvalid),  .s_axil_wready  (wready),
    .s_axil_bresp   (bresp),   .s_axil_bvalid (bvalid), .s_axil_bready  (bready),
    .s_axil_araddr  (araddr),  .s_axil_arprot (arprot), .s_axil_arvalid (arvalid), .s_axil_arready (arready),
    .s_axil_rdata   (rdata),   .s_axil_rresp  (rresp),  .s_axil_rvalid  (rvalid),  .s_axil_rready  (rready),
    .m_wb_CYC (wb_cyc), .m_wb_STB (wb_stb), .m_wb_WE (wb_we), .m_wb_ADR (wb_adr),
    .m_wb_DAT_W (wb_dat_w), .m_wb_SEL (wb_sel),
    .m_wb_STALL (wb_stall), .m_wb_ACK (wb_ack), .m_wb_ERR (wb_err), .m_wb_RTY (wb_rty),
    .m_wb_DAT_R (wb_dat_r),
    .busy ()
);

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	3.11 Wishbone B4 to AXI4-Lite Converter

The wb4_to_axil4 converter presents a Wishbone B4 slave and drives an AXI4-Lite master. It is the mirror of 3.10 and the harder direction, for one reason given below.

## 20.1	3.11.1 Module Organization

wb4_to_axil4.sv              # Wrapper: slave + core + the two masters
├── wb4_slave.sv             # Wishbone bus -> cmd/rsp queues (rtl/amba/wb4)
├── wb4_to_axil4_core.sv     # Queues -> five AXI4-Lite channels, responses merged
├── axil4_master_wr.sv       # AW / W / B channels (rtl/amba/axil4)
└── axil4_master_rd.sv       # AR / R channels (rtl/amba/axil4)

## 20.2	3.11.2 Why This Direction Is Harder

Going AXI4-Lite to Wishbone, the two request paths merge into one bus and the answers come back in issue order for free, because that is what Wishbone does.

Coming back the other way the ordering runs against you. Wishbone B4 terminates in issue order. AXI4-Lite’s B and R channels are independent and a slave may answer them in any order. A read issued behind a slow write will routinely finish first. If the converter passed each response straight through, the Wishbone master would see its transfers terminate out of order, which the protocol forbids and its own bookkeeping would mispair.

So the core records the direction of every command it issues in an in-order side queue and releases only the response at the head. A response that completed early waits in its channel’s skid until its predecessors have retired.

Ordering cost. A slow write at the head holds back a read that finished behind it. OUTSTANDING = 1 avoids the question entirely and is the default; raise it only when the AXI slave answers roughly in order, or head-of-line waiting eats the benefit.

## 20.3	3.11.3 Interface Specification

### 20.3.1	Parameters

Table 3.31: Wishbone to AXI4-Lite Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| ADDR_WIDTH | 32 | Address width, both sides |
| DATA_WIDTH | 32 | Data width, both sides |
| CMD_DEPTH / RSP_DEPTH | 2 | The wb4_slave’s queue depths |
| MAX_OUTSTANDING | 16 | The wb4_slave’s bus-side bound |
| CLASSIC | 0 | 0 = B4 pipelined, 1 = B4 standard; match the Wishbone master |
| OUTSTANDING | 1 | Commands in flight through the core. 1 = strictly serialised |
| SKID_DEPTH_AW/W/B/AR/R | 2 | The AXI4-Lite masters’ channel skids |
| AXIL_PROT | 3’b000 | AWPROT/ARPROT driven on every command |

### 20.3.2	Ports

Table 3.32: Wishbone to AXI4-Lite Ports

| Port group | Direction | Description |
| --- | --- | --- |
| s_wb_CYC/STB/WE/ADR/DAT_W/SEL/CTI/BTE | in | Wishbone request |
| s_wb_STALL/ACK/ERR/RTY/DAT_R | out | Flow control and termination |
| m_axil_aw*, m_axil_w*, m_axil_b* | master | AXI4-Lite write channels |
| m_axil_ar*, m_axil_r* | master | AXI4-Lite read channels |
| busy | out | Anything buffered, or a cycle open on the Wishbone side |

## 20.4	3.11.4 Command Formation

Table 3.33: Command Formation

| Wishbone | AXI4-Lite |
| --- | --- |
| WE=1, ADR, DAT_W, SEL | AWADDR + WDATA, WSTRB = SEL |
| WE=0, ADR | ARADDR |
| CTI / BTE | dropped (see below) |
| - | AWPROT/ARPROT = AXIL_PROT |

A write only launches when both AWREADY and WREADY are high in the same clock. Letting AW go while W stalled would leave a half-issued transfer that the outstanding count could not describe honestly.

The burst hints stop here. CTI and BTE describe a registered-feedback burst, and AXI4-Lite is single-beat with no field to carry them. A burst-aware target behind an AXI4-Lite bus is a contradiction, so the converter drops them rather than inventing a mapping.

## 20.5	3.11.5 Response Mapping

Table 3.34: Response Mapping

| AXI4-Lite BRESP / RRESP | Wishbone termination |
| --- | --- |
| OKAY | ACK |
| SLVERR | ERR |
| DECERR | ERR |
| - | RTY is never produced |

B4 has one error termination and no way to say which kind, so SLVERR and DECERR both become ERR. RTY never appears because an AXI slave has no way to ask for a retry; a Wishbone master that wants retries needs wb4_retry on the other side of the bus, not here.

## 20.6	3.11.6 Formal

formal/converters/wb4_to_axil4_core/ proves: a write takes AW and W in the same clock and never one alone; a command becomes a write or a read, never both; the response consumed is the head’s channel and is backed by a real response on that channel; the open count never exceeds OUTSTANDING; OKAY becomes ACK, any error becomes ERR, and RTY is never produced.

The backing property earned its place. An earlier property set proved every ordering rule above and still passed a mutation that asserted rsp_valid whenever either channel had a response, returning a status read off a channel that had nothing. Simulation caught it as a hang; formal did not, until the property was added.

## 20.7	3.11.7 Testing

dv/tests/test_wb4_to_axil4.py drives s_wb_* with the framework’s Wishbone master BFM and answers m_axil_* with the AXI4-Lite slave BFMs over one shared memory model, so a write really lands where a later read finds it. Addresses beyond the memory model answer SLVERR and must arrive as ERR.

The phases are sequential traffic, pipelined traffic under gappy and sparse request pacing, and a directed ordering probe: write-then-read pairs where the testbench answers the write channel slowly and the read channel quickly, so the AXI side completes the read first every time. The scoreboard pairs terminations with issue order, so a response that overtook its predecessor shows up as a status or data mismatch rather than as silence.

Three RTL mutations fail the test: completing from whichever channel is ready, corrupting read data, and launching AW without W.

## 20.8	3.11.8 Usage Example

wb4_to_axil4 #(
    .ADDR_WIDTH  (32),
    .DATA_WIDTH  (32),
    .OUTSTANDING (1),      // raise only if the AXI slave answers in order
    .CLASSIC     (0)
) u_wb_to_axil (
    .aclk (clk), .aresetn (rst_n),
    .s_wb_CYC (wb_cyc), .s_wb_STB (wb_stb), .s_wb_WE (wb_we), .s_wb_ADR (wb_adr),
    .s_wb_DAT_W (wb_dat_w), .s_wb_SEL (wb_sel), .s_wb_CTI (wb_cti), .s_wb_BTE (wb_bte),
    .s_wb_STALL (wb_stall), .s_wb_ACK (wb_ack), .s_wb_ERR (wb_err), .s_wb_RTY (wb_rty),
    .s_wb_DAT_R (wb_dat_r),
    .m_axil_awaddr (awaddr), .m_axil_awprot (awprot), .m_axil_awvalid (awvalid), .m_axil_awready (awready),
    .m_axil_wdata (wdata), .m_axil_wstrb (wstrb), .m_axil_wvalid (wvalid), .m_axil_wready (wready),
    .m_axil_bresp (bresp), .m_axil_bvalid (bvalid), .m_axil_bready (bready),
    .m_axil_araddr (araddr), .m_axil_arprot (arprot), .m_axil_arvalid (arvalid), .m_axil_arready (arready),
    .m_axil_rdata (rdata), .m_axil_rresp (rresp), .m_axil_rvalid (rvalid), .m_axil_rready (rready),
    .busy ()
);

## 20.9	Navigation

Next: APB to AXI4 (Requester)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	APB to AXI4 (Requester)

Modules: apb4_to_axi4.sv, apb5_to_axi4.sv, apb_cmdrsp_to_axi4.sv Filelists: rtl/filelists/apb4_to_axi4.f, rtl/filelists/apb5_to_axi4.f (each -f’s its rtl/amba APB completer and the shared requester core)

## 21.1	Overview

The mirror of AXI4 to APB. There an AXI4 requester is turned into an APB requester; here an APB requester – a small CPU, a debug port, a register sequencer – becomes an AXI4 requester. The bridge generator puts one of these in front of a master port declared protocol = "apb" or "apb5" (BRIDGE-014).

Two proven pieces and one small new one:

Table 3.35: APB-to-AXI4 requester pieces

| Piece | Job |
| --- | --- |
| apb4_slave / apb5_slave (rtl/amba) | The APB completer surface: PSEL/PENABLE/PREADY handshake, skid-buffered cmd/rsp pair, the orphan-response guard |
| apb_cmdrsp_to_axi4 | One command -> one single-beat AXI4 transaction -> one response |
| apb4_to_axi4 / apb5_to_axi4 | The wiring, and for APB5 the USER mapping |

APB is strictly one-outstanding, so there is never more than one AXI4 transaction in flight and the response needs no ID matching: every request carries the constant DEFAULT_ID. AW and W are presented together and each is held until its own handshake, so the requester is legal against a completer that takes W before AW or the other way round.

pwrite=1 : AW + W (awlen=0, wlast=1)  ->  B  ->  rsp  (PSLVERR = bresp[1])
pwrite=0 : AR      (arlen=0)          ->  R  ->  rsp  (PRDATA, PSLVERR = rresp[1])

Both SLVERR and DECERR fold to PSLVERR: APB has one error bit, so a requester cannot tell an unmapped address from a failing completer. R is drained to RLAST even though every request asks for one beat, so a completer that answers with more cannot wedge the port; only the first beat’s data is returned.

## 21.2	Fields the requester invents

Table 3.36: AXI4 request fields from an APB transfer

| Field | Value | Why |
| --- | --- | --- |
| AxID | DEFAULT_ID | APB has no ID; one value is enough with one transaction outstanding |
| AxLEN | 0 | one beat per transfer |
| AxSIZE | $clog2(DATA_WIDTH/8) | the full data width; PSTRB becomes WSTRB unchanged |
| AxBURST | INCR |  |
| AxCACHE | DEFAULT_CACHE (device non-bufferable) | the natural attribute for a peripheral-bus requester |
| AxPROT | PPROT | forwarded |
| AxLOCK, AxQOS, AxREGION | 0 |  |
| WLAST | 1 |  |

## 21.3	APB5 sideband

apb5_to_axi4 maps the APB5 USER signals onto the AXI USER fields with a plain size cast – the low bits travel, anything wider is zero-filled or dropped:

Table 3.37: APB5 USER mapping

| APB5 | AXI4 |
| --- | --- |
| PAUSER | awuser / aruser |
| PWUSER | wuser |
| buser | PBUSER |
| ruser | PRUSER |

PWAKEUP is requester-driven (AMBA APB5, IHI 0024E) and is an input here, accepted and terminated – there is nothing on the AXI side for it to become. apb5_slave’s own PWAKEUP output (its wake-up request toward the requester) is left unconnected with wakeup_request tied low. Parity is not part of this surface: apb5_slave is instantiated with ENABLE_PARITY=0 and its parity pins tied off, matching axi4_to_apb5_shim.

Inside the bridge the AXI USER width is 1, so one bit of PAUSER reaches the fabric – the same width the AXI5-Lite slave side exposes, which is how an APB5 requester’s user bit can be observed at an AXI5-Lite completer. The return direction is not symmetrical: the bridge’s master adapters tie response-side USER to zero (see the bridge PRD), so PBUSER/PRUSER read 0 on a bridge port even though the converter itself carries them.

## 21.4	Parameters

Table 3.38: APB-to-AXI4 parameters

| Parameter | Default | Description |
| --- | --- | --- |
| APB_ADDR_WIDTH | 32 | PADDR and AxADDR width (one address space; no windowing) |
| APB_DATA_WIDTH | 32 | PWDATA/PRDATA and the AXI data width – equal by construction |
| AXI_ID_WIDTH | 1 | width of the constant ID |
| AXI_USER_WIDTH | 1 | AXI USER width (APB4 wrapper drives 0) |
| DEFAULT_ID | '0 | the ID on every request |
| DEPTH | 2 | cmd/rsp skid depth inside the APB completer |
| APB_{A,W,R,B}USER_WIDTH | 1 | APB5 only; independent of AXI_USER_WIDTH |

DEFAULT_CACHE on the core is not exposed by the wrappers; a consumer that needs a different cache attribute instantiates apb_cmdrsp_to_axi4 directly.

## 21.5	Verification

dv/tests/test_apb4_to_axi4.py and dv/tests/test_apb5_to_axi4.py share dv/tbclasses/apb_to_axi4_tb.py: an APB master BFM drives the completer surface, memory-backed AXI4 slave BFMs answer the requester surface, and a monitor samples every AW/W/AR handshake. Checked: the data round trip (the completer’s memory is read directly as well as through a second transfer, so a converter echoing its own write data is caught), the single-beat shape of every request, PPROT->AxPROT, PSTRB->WSTRB against a byte-accurate shadow, out-of-range SLVERR folding to PSLVERR with nothing written and the port working afterwards, the APB5 USER mapping in both directions, and a slow completer holding AW/W/AR to their handshakes. Twelve cells at FULL (data widths 16/32/64, AXI USER 1/2/4, APB USER 2/4).

## 21.6	Related Modules

The bridge generator’s master adapter instantiates apb4_to_axi4 / apb5_to_axi4 for protocol = "apb" / "apb5" master ports and feeds the result to the same axi4_slave_{wr,rd} timing wrapper an AXI4 master port gets, so decode, width adaptation (including the wide-slave aligner) and the response mux are untouched. Sign-off: projects/components/bridge/dv/tests/test_bridge_2x3_apb_req_paths.py.

## 21.7	Navigation

Previous: Wishbone B4 to AXI4-Lite

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	4.1 Width Converter FSMs

## 22.1	Overview

Here’s the twist with the width converters: the upsize and downsize primitives don’t have a state machine at all, and that’s deliberate — what they have instead is tighter. The full AXI4 dwidth converters do run explicit FSMs, and those are small.

## 22.2	Functional Description

### 22.2.1	4.1.1 Upsize Structure (no FSM)

The axi_data_upsize module has NO state machine. Earlier revisions of this chapter documented an ACCUMULATE/OUTPUT FSM; the RTL is a register-and-gating structure built deliberately so that output and the next accumulation OVERLAP — an explicit OUTPUT state would insert the very bubble the design avoids.

The real structure:

- r_beat_ptr walks slots as narrow beats land in the accumulator. A fresh BURST’s first beat lands at start_lane (mid-word INCR starts, CONV-006), so the effective lane is w_lane = (ptr=='0) ? (burst_fresh ? start_lane : 0) : ptr;

- a group completes when the LANE reaches WIDTH_RATIO-1 or on an early narrow_last — a mid-word burst’s first wide group holds WIDTH_RATIO - start_lane beats, not WIDTH_RATIO; later groups start at lane 0 and hold the full count;

- r_wide_valid presents the completed group on the wide side while the NEXT group starts accumulating — when a wide handshake and a completing narrow beat coincide, a single priority selection keeps the freshly completed group. (Last-write-wins ordering across two NBA blocks was the BUG here, not the fix — the wide-accept block’s clear clobbered the fresh group’s valid; the dnsize is the one that legitimately relies on last-write-wins, for its atomic replace.)

- narrow_ready = !r_wide_valid || wide_ready, so the narrow side stalls only when the wide side is holding a beat hostage.

On an early narrow_last, unwritten accumulator slots are zeroed in the same single write that deposits the first beat of a group — residual data from the previous group must not leak into a partial wide beat.

### 22.2.2	4.1.2 Downsize Structure (single buffer, no FSM)

Same story: no IDLE/LOAD/OUTPUT machine. The single buffer performs an atomic replace — wide_ready asserts during the LAST narrow beat of the current wide beat, so the replacement lands as the drain finishes and a steady stream pays no per-beat bubble (measured 0.992 beats/cycle, see 2.3):

assign wide_ready = !r_wide_buffered || (narrow_ready && w_last_narrow_beat);

With TRACK_BURSTS=1 the condition narrows to mid_burst_replace, which excludes each burst’s final beat — one bubble per burst boundary, not per beat. A beat pointer (r_beat_ptr) selects the slice driven onto the narrow side; on the burst’s FIRST wide word it starts at start_lane (mid-word INCR starts, CONV-006 — the narrow master receives the bytes it addressed, not the aligned-down word’s bytes), and at lane 0 for every later wide word. narrow_last in tracked mode comes only from the beat counter reaching burst_len + 1.

### 22.2.3	4.1.3 Full Converter FSMs

#### 22.2.3.1	Write Converter (axi4_dwidth_converter_wr)

IDLE:
  - AW valid → accept AW, store info
  - W valid → buffer W data

AW_ACCEPT:
  - downstream AW ready → forward adjusted AW

W_CONVERT:
  - upsize accumulating narrow W beats
  - on output → forward wide W beat

B_FORWARD:
  - B from downstream → forward to master

#### 22.2.3.2	Read Converter (axi4_dwidth_converter_rd)

IDLE:
  - AR valid → accept AR, store info, forward adjusted AR

AR_FORWARD:
  - downstream AR ready → wait for R

R_CONVERT:
  - downsize splitting wide R into narrow beats
  - track burst count for RLAST

R_FORWARD:
  - narrow R beat ready → forward to master
  - on RLAST → IDLE

## 22.3	Timing

### 22.3.1	Upsize (8:1 ratio, m_ready held high)

No s_ready bubble: the wide handshake overlaps the next group’s first narrow beat.

clk     __|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|
s_valid ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯
s_data    D0   D1   D2   D3   D4   D5   D6   D7   D8   D9   ...
s_ready ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯
m_valid _______________________________________|¯¯¯¯|_______________
m_data                                          WIDE0
m_ready ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯

s_ready only drops if the wide side stalls with a completed group waiting (!r_wide_valid || wide_ready fails).

### 22.3.2	Downsize (8:1 ratio, single buffer, atomic replace)

s_ready re-asserts DURING the last narrow beat, so WIDE1 is accepted as D7 drains — back-to-back wide beats with no gap:

clk     __|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|__|¯¯|
s_valid ¯¯¯¯¯¯¯|_________________________________________|¯¯¯¯|_____
s_data    WIDE0                                            WIDE1
s_ready ¯¯¯¯¯¯¯|__________________________________________|¯¯¯¯|____
m_valid ________|¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯
m_data           D0   D1   D2   D3   D4   D5   D6   D7   E0   E1 ...
m_ready ¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯¯

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	4.2 Protocol Converter FSMs

## 23.1	Overview

The state machines inside the protocol converters: every one of them is smaller than the earlier documentation made it look.

## 23.2	Functional Description

### 23.2.1	4.2.1 AXI4 to AXI4-Lite Read FSM

The real machine has THREE states, and none of them waits for R:

typedef enum logic [1:0] {
    RD_IDLE       = 2'b00,   // no burst in flight
    RD_BURST      = 2'b01,   // issuing the AXIL4 reads of a burst
    RD_LAST_BEAT  = 2'b10    // issuing the burst's final read
} rd_state_t;

Earlier revisions documented IDLE/SINGLE/DECOMPOSE/WAIT_R. Two of those never existed: a single-beat read (ARLEN==0) takes a passthrough leg and never leaves RD_IDLE, and there is no wait-for-R state at all — the address and data paths are decoupled, with ARs issuing at the downstream accept rate regardless of R (see 3.2). What serializes bursts is not an FSM state but the one-outstanding-burst guard: the burst-tracking registers (r_r_id/r_r_len/r_r_beat_count) hold exactly one burst, so s_axi_arready is held off until the in-flight burst delivers its last beat. Response aggregation is the live-beat worst-case fold of 4.3.3.

### 23.2.2	4.2.2 AXI4 to AXI4-Lite Write FSM

Also three states (WR_IDLE/WR_BURST/WR_LAST_BEAT), same passthrough leg for AWLEN==0, same one-outstanding guard on the B side. There are no r_aw_pending/r_w_pending flags: W is gated directly against the state of the AW that owns it —

- during the burst-capture cycle W is held off (w_burst_capture), or the first W beat of the next burst would slip out while the previous burst is finishing and pair with the wrong address;

- during a burst, W passes only once this burst’s AW has been sent (r_aw_sent) or is being sent in the same cycle;

- single beats pass straight through.

See 3.2’s “W gated on the AW that owns it” for the exact equations.

### 23.2.3	4.2.3 AXI4 to APB FSMs

The convert core runs a command FSM (IDLE/READ/WRITE, writes preferred) that packetizes each burst into per-APB-beat commands, and a response FSM (RSP_IDLE/RSP_ACTIVE) that reassembles rsp packets into AXI responses. PSEL/PENABLE setup/access phases are NOT here — they belong to apb4_master on the far side of the shim’s CDC. Full description in 3.4.5.

## 23.3	Timing

### 23.3.1	4.2.4 Timing Analysis

AXI4 to AXIL4. A single-beat transfer is a combinational passthrough — no converter-inserted wait state. In a burst the decomposed requests issue at the downstream ACCEPT rate, independent of responses: m_axil_arvalid in RD_BURST holds every cycle and the beat tracker advances on m_axil_arvalid && m_axil_arready — no signal in the request path samples m_axil_rvalid/m_axil_bvalid. Against a pipelining slave, N requests issue in N consecutive cycles and the burst finishes one response latency later; the old “2 cycles overhead / 2N cycles per burst” figures described a serialization the RTL does not have. Bursts do not overlap each other (the one-outstanding guard above); that is the only converter-imposed serialization.

AXI4 to APB. Per APB beat: command packet -> CDC (2-flop gray crossing each way) -> APB setup + access (2 pclk minimum, plus PREADY stalls) -> response packet -> CDC back. Latency is dominated by the two clock crossings and the APB protocol itself; no measured characterization is published here.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	4.3 Burst Decomposition

## 24.1	Overview

Burst handling is where the two converter families diverge: width converters rescale the burst length, protocol converters split the burst into single beats. Same AXI burst in — two very different kinds of bookkeeping out.

## 24.2	Functional Description

### 24.2.1	4.3.1 Width Converter Burst Handling

#### 24.2.1.1	Burst Length Adjustment

When converting widths, burst length changes inversely with data width:

M_AWLEN = ceil((S_AWLEN + 1) / RATIO) - 1  =  ((S_AWLEN + 1) + RATIO - 1) / RATIO - 1

Example (64-bit to 512-bit, RATIO=8):
  S_AWLEN = 7 (8 beats × 64 bits = 512 bits)
  M_AWLEN = (7 + 1) / 8 - 1 = 0 (1 beat × 512 bits)

  S_AWLEN = 15 (16 beats × 64 bits = 1024 bits)
  M_AWLEN = (15 + 1) / 8 - 1 = 1 (2 beats × 512 bits)

#### 24.2.1.2	Non-Aligned Bursts

When the burst length is not a multiple of the ratio:

S_AWLEN = 5 (6 beats), RATIO = 8
M_AWLEN = ((5 + 1) + 7) / 8 - 1 = 0  (1 wide beat -- the ceiling term is what prevents the -1 a floor divide gives)

The 6 narrow beats pack into 1 wide beat.
Last 2 positions have WSTRB = 0 (no write).

### 24.2.2	4.3.2 Protocol Converter Burst Handling

#### 24.2.2.1	AXI4 to AXI4-Lite Decomposition

AXI4-Lite only supports single-beat transactions, so all bursts must be decomposed:

AXI4 Burst:
  ARADDR = 0x1000
  ARLEN = 3 (4 beats)
  ARSIZE = 2 (4 bytes)

AXIL4 Sequence:
  Transaction 0: ARADDR = 0x1000
  Transaction 1: ARADDR = 0x1004
  Transaction 2: ARADDR = 0x1008
  Transaction 3: ARADDR = 0x100C

#### 24.2.2.2	Address Increment Calculation

// Calculate address increment based on burst type and size
function automatic [ADDR_WIDTH-1:0] next_address(
    input [ADDR_WIDTH-1:0] current_addr,
    input [2:0] size,
    input [1:0] burst,
    input [7:0] len,
    input [7:0] beat
);
    logic [ADDR_WIDTH-1:0] increment;
    logic [ADDR_WIDTH-1:0] wrap_mask;

    increment = 1 << size;

    case (burst)
        2'b00: // FIXED
            return current_addr;  // No increment

        2'b01: // INCR
            return current_addr + increment;

        2'b10: // WRAP
            wrap_mask = ((len + 1) << size) - 1;
            return (current_addr & ~wrap_mask) |
                   ((current_addr + increment) & wrap_mask);

        default:
            return current_addr + increment;
    endcase
endfunction

### 24.2.3	4.3.3 Response Aggregation

Several downstream responses collapse into one upstream response, worst case winning (OKAY < EXOKAY < SLVERR < DECERR by numeric compare). The subtlety is WHERE the final beat’s own response enters the result, and getting it wrong is not hypothetical: the converters shipped with the registered-only version below, and every single-beat error was reported upstream as OKAY.

Wrong (the shipped bug). Accumulate in a register and emit the register:

always_ff @(posedge aclk or negedge aresetn) begin
    ...
    else if (m_axil_bvalid && m_axil_bready)
        if (m_axil_bresp > r_b_resp_accum)
            r_b_resp_accum <= m_axil_bresp;
end

assign s_axi_bresp = r_b_resp_accum;   // WRONG

The accumulator is a non-blocking assign: on the handshake that emits the response it does not yet contain THAT beat’s bresp. For a single-beat transfer the only beat is the final one, so its error is exactly the one dropped.

Right (the current RTL). Fold the live beat combinationally at the point of emission:

assign w_b_resp_worst = (m_axil_bresp > r_b_resp_accum) ? m_axil_bresp
                                                        : r_b_resp_accum;
assign s_axi_bresp    = w_b_resp_worst;

The read path is the same shape, gated to the final beat:

assign s_axi_rresp = (r_r_beat_count == r_r_len) ? w_r_resp_worst
                                                 : m_axil_rresp;

Both were fixed with tests that inject SLVERR from the downstream slave and assert it reaches the upstream master — including the last-beat-only case, where earlier beats cannot mask the loss. See test_error_response in the axi4_to_axil4 testbenches.

### 24.2.4	4.3.4 Burst Tracking Registers

#### 24.2.4.1	Required State

The listing below is the generic PATTERN, not the RTL’s register names: the real modules split this state between separate read and write converters and count UP (r_ar_beat_count/r_aw_beat_count against a stored length) rather than down — grep for those names, not these.

// Burst tracking registers (generic pattern)
logic [ADDR_WIDTH-1:0] r_base_addr;
logic [ADDR_WIDTH-1:0] r_current_addr;
logic [7:0]            r_original_len;
logic [7:0]            r_remaining_beats;
logic [2:0]            r_size;
logic [1:0]            r_burst;
logic [ID_WIDTH-1:0]   r_id;
logic                  r_is_write;

#### 24.2.4.2	Initialization

always_ff @(posedge clk) begin
    if (accept_new_transaction) begin
        r_base_addr <= s_axaddr;
        r_current_addr <= s_axaddr;
        r_original_len <= s_axlen;
        r_remaining_beats <= s_axlen;
        r_size <= s_axsize;
        r_burst <= s_axburst;
        r_id <= s_axid;
        r_is_write <= is_write_transaction;
    end else if (beat_complete) begin
        r_current_addr <= next_address(...);
        r_remaining_beats <= r_remaining_beats - 1;
    end
end

## 24.3	Timing

### 24.3.1	4.3.5 Timing Impact

#### 24.3.1.1	Decomposition Overhead

Table 4.6: Decomposition Overhead

| Transaction Type | Overhead |
| --- | --- |
| Single-beat | 0 cycles (passthrough) |
| N-beat burst | none per beat – requests stream at the downstream accept rate, independent of responses (see 4.2.4) |
| Back-to-back bursts | serialized by the one-outstanding-burst guard |

The old “2 cycles per beat” figure described a request-response lockstep the RTL does not have; nothing in the address path waits on responses.

#### 24.3.1.2	Pipeline Considerations

Decomposed requests are independent of responses (Table 4.6 above): - ARs issue at the downstream accept rate — nothing in the AR path samples m_axil_rvalid - AW/W beats advance on their own handshakes; intermediate Bs are absorbed (m_axil_bready = s_axi_bready || !w_b_all_beats_done) without stalling issuance

Against a pipelining AXIL4 slave, an N-beat burst costs N issue cycles plus one response latency — not N round trips. The only serialization the converter imposes is between whole bursts (the one-outstanding guard).

## 24.4	Waveforms

### 24.4.1	Figure 4.5: Width Burst Conversion

Width Burst Conversion

## 24.5	Navigation

Next: Chapter 5: Verification

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	5.1 Test Strategy

## 25.1	Overview

How the converter modules get verified, from unit tests to system stress: three test levels, a coverage model, and the exact commands to run it all.

## 25.2	Testing

### 25.2.1	5.1.1 Test Organization

projects/components/converters/dv/tests/
├── width_converters/
│   ├── test_axi_data_upsize.py
│   ├── test_axi_data_dnsize.py
│   ├── test_axi4_dwidth_converter_wr.py
│   └── test_axi4_dwidth_converter_rd.py
├── protocol_converters/
│   ├── test_axi4_to_axil4_rd.py
│   ├── test_axi4_to_axil4_wr.py
│   ├── test_axil4_to_axi4_rd.py
│   ├── test_axil4_to_axi4_wr.py
│   ├── test_axi4_to_apb4.py
│   └── test_peakrdl_adapter.py
└── integration/
    ├── test_width_protocol_chain.py
    └── test_full_system.py

### 25.2.2	5.1.2 Test Levels

#### 25.2.2.1	Level 1: Unit Tests

Purpose: Verify individual module functionality in isolation.

Coverage: - All parameters combinations - Edge cases (min/max values) - Error injection

Example:

@pytest.mark.parametrize("narrow_width,wide_width", [
    (32, 64),
    (32, 128),
    (64, 256),
    (64, 512),
    (128, 1024),
])
async def test_upsize_ratios(dut, narrow_width, wide_width):
    """Test various width ratios."""
    tb = UpsizeTB(dut, narrow_width, wide_width)
    await tb.reset()
    await tb.run_basic_transfer(count=100)
    assert tb.scoreboard.check_passed()

#### 25.2.2.2	Level 2: Integration Tests

Purpose: Verify module combinations work together.

Coverage: - Width converter + protocol converter chains - Back-to-back converters - Mixed traffic patterns

#### 25.2.2.3	Level 3: System Tests

Purpose: Verify in realistic system context.

Coverage: - CPU-to-DDR paths - Peripheral access paths - Full bandwidth stress

### 25.2.3	5.1.3 Test Categories

#### 25.2.3.1	Functional Tests

Table 5.1: Functional Test Categories

| Category | Description | Example |
| --- | --- | --- |
| Basic | Single transactions | Single read, single write |
| Burst | Multi-beat transactions | INCR burst, WRAP burst |
| Mixed | Interleaved R/W | Read-modify-write sequences |
| Edge | Boundary conditions | Max burst length, min width |

#### 25.2.3.2	Stress Tests

Table 5.2: Stress Test Categories

| Category | Description | Duration |
| --- | --- | --- |
| Throughput | Maximum bandwidth | 10,000 transactions |
| Backpressure | Ready signal variations | Random delays |
| Reset | Reset during operation | Mid-transaction reset |

#### 25.2.3.3	Error Tests

Table 5.3: Error Test Categories

| Category | Description | Expected Behavior |
| --- | --- | --- |
| SLVERR | Slave error injection | Error propagation |
| DECERR | Decode error | Error response |
| Timeout | Response timeout | Error handling |

### 25.2.4	5.1.4 Testbench Architecture

#### 25.2.4.1	Components

Testbench
├── Drivers
│   ├── AXI4 Master Driver
│   ├── AXI4 Slave Driver
│   ├── AXIL4 Master/Slave
│   └── APB Master/Slave
├── Monitors
│   ├── AXI4 Monitor
│   ├── AXIL4 Monitor
│   └── APB Monitor
├── Scoreboard
│   ├── Transaction Queue
│   ├── Response Checker
│   └── Coverage Collector
└── Generators
    ├── Random Transaction Generator
    └── Directed Sequence Generator

#### 25.2.4.2	Driver Implementation

class AXI4MasterDriver:
    def __init__(self, dut, clock, prefix="s_axi"):
        self.dut = dut
        self.clock = clock
        self.prefix = prefix

    async def write(self, addr, data, burst_len=0):
        """Issue AXI4 write transaction."""
        # AW phase
        # the DUT ports are prefixed (s_axi_awvalid, ...) -- resolve
        # through the stored prefix or every access AttributeErrors
        sig = lambda name: getattr(self.dut, f"{self.prefix}_{name}")
        sig("awvalid").value = 1
        sig("awaddr").value = addr
        sig("awlen").value = burst_len
        await self._wait_ready("awready")

        # W phase
        for i, d in enumerate(data):
            sig("wvalid").value = 1
            sig("wdata").value = d
            sig("wlast").value = (i == len(data) - 1)
            await self._wait_ready("wready")

        # B phase
        await self._wait_valid("bvalid")
        return sig("bresp").value

### 25.2.5	5.1.5 Coverage Model

#### 25.2.5.1	Functional Coverage

@coverage
class ConverterCoverage:
    # Width ratio coverage
    ratio_cp = coverpoint(
        lambda: self.width_ratio,
        bins=[2, 4, 8, 16]
    )

    # Burst length coverage
    burst_len_cp = coverpoint(
        lambda: self.burst_len,
        bins=list(range(0, 256, 16)) + [255]
    )

    # Burst type coverage
    burst_type_cp = coverpoint(
        lambda: self.burst_type,
        bins=["FIXED", "INCR", "WRAP"]
    )

    # Cross coverage
    ratio_x_burst = cross(ratio_cp, burst_len_cp)

#### 25.2.5.2	Code Coverage

Target: >95% line coverage, >90% branch coverage

Table 5.4: Coverage Targets

| Module | Line | Branch | FSM |
| --- | --- | --- | --- |
| axi_data_upsize | 98% | 95% | n/a (no FSM – see 4.1.1) |
| axi_data_dnsize | 97% | 93% | n/a (no FSM – see 4.1.2) |
| axi4_to_axil4_rd | 96% | 91% | 100% |
| axi4_to_axil4_wr | 95% | 90% | 100% |

### 25.2.6	5.1.6 Test Execution

#### 25.2.6.1	Running Tests

# Run all converter tests
cd projects/components/converters/dv/tests
pytest -v

# Run specific module tests
pytest test_axi_data_upsize.py -v

# Run with coverage
pytest --cov=projects/components/converters/rtl -v

# Run with waveform dump
pytest test_axi_data_upsize.py -v --waves

#### 25.2.6.2	CI/CD Integration

converter_tests:
  stage: test
  script:
    - cd projects/components/converters/dv/tests
    - pytest -v --junitxml=results.xml
    - pytest --cov=rtl --cov-report=html
  artifacts:
    reports:
      junit: results.xml
    paths:
      - htmlcov/

## 25.3	Navigation

Next: Debug Guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	5.2 Debug Guide

## 26.1	Overview

Start here when a converter misbehaves. Each entry runs symptoms, probe points, then fix — the order you want them in at the bench.

## 26.2	Testing

### 26.2.1	5.2.1 Common Issues

#### 26.2.1.1	Width Converter Issues

##### 26.2.1.1.1	Issue: Data Corruption

Symptoms: - Output data doesn’t match expected - Random bits flipped or missing

Debug Steps: 1. Check width ratio calculation: RATIO = WIDE_WIDTH / NARROW_WIDTH 2. Verify beat counter is correct: $clog2(RATIO) bits 3. Check data packing/unpacking slice indices 4. Verify sideband mode matches use case

Waveform Checkpoints:

r_beat_ptr             - slot/slice pointer, cycles 0 to RATIO-1
r_data_accumulator     - (upsize) each slot fills as narrow beats land
r_data_buffer          - (dnsize) captured wide beat being sliced
narrow_/wide_ data     - compare input/output patterns

##### 26.2.1.1.2	Issue: LAST Signal Incorrect

Symptoms: - Transaction ends early or late - Master receives wrong beat count

Debug Steps: 1. Check TRACK_BURSTS parameter (axi_data_dnsize); there is no USE_LAST 2. Verify burst tracker logic if enabled 3. Check s_last input timing

Solution:

// LAST is the buffered last flag AND the final beat -- from the RTL:
//   upsize:  wide_last   = r_last_buffered && r_wide_valid;
//   dnsize:  narrow_last = r_wide_buffered && r_last_buffered
//                          && w_last_narrow_beat;
// An OR of (final beat, saw-last) asserts LAST at the end of EVERY
// group and truncates any burst spanning more than one wide beat.

##### 26.2.1.1.3	Issue: Throughput Lower Than Expected

Symptoms: - Gaps between output beats - Single-buffer stalling between wide beats (it should replace during the last narrow beat)

Debug Steps: 1. Check downstream ready signal behavior 2. Look for backpressure stalls

#### 26.2.1.2	Protocol Converter Issues

##### 26.2.1.2.1	Issue: Burst Decomposition Incorrect

Symptoms: - Wrong number of single transactions - Address increment wrong

Debug Steps: 1. Check burst type (FIXED, INCR, WRAP) 2. Verify size calculation 3. Check address increment logic

Waveform Checkpoints:

r_ar_addr / r_aw_addr - walk by (1 << size) per issued beat
r_ar_beat_count / r_aw_beat_count - count to arlen/awlen
m_axil_arvalid  - asserts for each decomposed beat

##### 26.2.1.2.2	Issue: Response Aggregation Wrong

Symptoms: - Wrong BRESP/RRESP value - Error not propagated correctly

Debug Steps: 1. Check worst-case response tracking 2. Verify response counter 3. Check RLAST generation

Solution:

// Accumulate per beat...
always_ff @(posedge aclk or negedge aresetn) begin
    ...
    else if (m_axil_rvalid && m_axil_rready)
        if (m_axil_rresp > r_r_resp_accum)
            r_r_resp_accum <= m_axil_rresp;
end

// ...and ALWAYS fold the live beat in at emission. The accumulator
// alone is the shipped bug (see 4.3.3): it does not yet contain the
// final beat's own response, so a single-beat error reports OKAY.
assign w_r_resp_worst = (m_axil_rresp > r_r_resp_accum) ? m_axil_rresp
                                                        : r_r_resp_accum;
assign s_axi_rresp = (r_r_beat_count == r_r_len) ? w_r_resp_worst
                                                 : m_axil_rresp;

### 26.2.2	5.2.2 Debug Signals

#### 26.2.2.1	Recommended Internal Signals

For width converters:

// Add debug outputs
output logic [$clog2(RATIO)-1:0] dbg_beat_count,
output logic                     dbg_buffer_valid,
output logic [1:0]               dbg_state

For protocol converters:

// Add debug outputs
output logic [7:0]  dbg_remaining_beats,
output logic [2:0]  dbg_state,
output logic [1:0]  dbg_worst_resp,
output logic        dbg_in_burst

#### 26.2.2.2	ILA Configuration

# Create ILA for converter debug
create_debug_core u_ila ila

# Add probes
set_property probe_count 10 [get_debug_cores u_ila]
connect_debug_port u_ila/clk [get_nets aclk]

# Key signals
connect_debug_port u_ila/probe0 [get_nets r_beat_ptr]
connect_debug_port u_ila/probe1 [get_nets r_ar_beat_count]
connect_debug_port u_ila/probe2 [get_nets s_axi_arvalid]
connect_debug_port u_ila/probe3 [get_nets m_axil_arvalid]

### 26.2.3	5.2.3 Simulation Debug

#### 26.2.3.1	Waveform Analysis

Key Signal Groups (real port names — the width primitives are named by WIDTH, not direction: narrow_* is the INPUT of the upsize but the OUTPUT of the dnsize):

- Narrow side:

  - narrow_valid, narrow_ready, narrow_data, narrow_sideband, narrow_last (upsize input / dnsize output)

- Wide side:

  - wide_valid, wide_ready, wide_data, wide_sideband, wide_last (upsize output / dnsize input)

- Control:

  - r_beat_ptr, r_slave_beat_count, r_burst_active (tracked mode), start_lane, burst_len/burst_start (dnsize tracked mode)

- Sideband contents:

  - WSTRB rides narrow_/wide_sideband on the write path

  - RRESP rides narrow_/wide_sideband on the read path

#### 26.2.3.2	Timing Diagram Template

___     ___     ___     ___     ___
clk     ___|   |___|   |___|   |___|   |___|   |
            _______________________________
s_valid ___/                               \___
           [D0 ][D1 ][D2 ][D3 ][D4 ][D5 ][D6 ][D7 ]
s_data
        ___________________________________________
s_ready

           ________________________________
m_valid __|                                |___
                                   [WIDE_DATA ]
m_data

Check:
1. s_ready stays high during accumulation
2. m_valid asserts after RATIO beats
3. Data packing is correct

### 26.2.4	5.2.4 Common Mistakes

#### 26.2.4.1	Mistake 1: Wrong Width Ratio

// WRONG: Manual ratio
localparam RATIO = 8;  // May not match actual widths

// CORRECT: Calculated ratio
localparam RATIO = WIDE_WIDTH / NARROW_WIDTH;

#### 26.2.4.2	Mistake 2: Missing Sideband Handling

// WRONG: Forgetting sideband
assign m_data = r_data;
// Missing: assign m_wstrb = ...

// CORRECT: Handle both
assign m_data = r_data;
assign m_wstrb = r_sideband;

#### 26.2.4.3	Mistake 3: Incorrect LAST Timing

// WRONG: LAST on wrong beat
assign narrow_last = (r_beat_ptr == 0);  // First slice!

// CORRECT: LAST on final beat
// AND of final-beat with the buffered last flag (see 5.2.1); the OR
// form truncates multi-wide-beat bursts
assign narrow_last = r_wide_buffered && r_last_buffered && w_last_narrow_beat;

#### 26.2.4.4	Mistake 4: Burst Length Calculation Error

// WRONG: Off-by-one
assign m_awlen = s_awlen / RATIO;  // Wrong!

// CORRECT: Account for LEN encoding
assign m_axi_awlen = ((int_awlen + 8'(WIDTH_RATIO)) / 8'(WIDTH_RATIO)) - 8'd1;  // round UP

### 26.2.5	5.2.5 Verification Checklist

Before signoff, verify:

- ☐ All parameter combinations tested

- ☐ Single-beat transactions work

- ☐ Multi-beat bursts work (INCR, WRAP, FIXED)

- ☐ Backpressure handling correct

- ☐ Error propagation correct

- ☐ LAST signal timing correct

- ☐ Sideband signals handled correctly

- ☐ Reset behavior verified

- ☐ Coverage targets met

### 26.2.6	5.2.6 Performance Validation

#### 26.2.6.1	Throughput Measurement

async def measure_throughput(tb, transaction_count=1000):
    start_time = get_sim_time()

    for _ in range(transaction_count):
        await tb.send_transaction()

    end_time = get_sim_time()
    elapsed_cycles = (end_time - start_time) / clock_period

    throughput = transaction_count / elapsed_cycles
    print(f"Throughput: {throughput:.2f} transactions/cycle")

    return throughput

#### 26.2.6.2	Expected Throughput

Table 5.5: Expected Throughput

| Module | Mode | Expected |
| --- | --- | --- |
| axi_data_upsize | Single | 1.0 trans/cycle |
| axi_data_dnsize | Single | 0.992 narrow beats/cycle (measured) |
| axi4_to_axil4 | Single-beat | ~1/(2+slave latency) trans/cycle (one-outstanding guard) |
| axi4_to_axil4 | Burst | ~N/(N+2) beats/cycle vs a pipelining slave (N issue cycles + one response latency, see 4.2.4); whole bursts serialize on the one-outstanding guard |

## 26.3	Navigation

End of Micro-Architecture Specification