RTL Design Sherpa

mem-ctrl-ip Common IP Micro-Architecture Specification 0.1

October 10, 2026

Table of Contents

1 Common Mas Index	10

2 mem-ctrl-ip Family — Overarching Hardware Architecture Specification	11

2.1 Document Organization	11

2.1.1 Front Matter	11

2.1.2 Chapter 1: Introduction	11

2.1.3 Chapter 2: The Plane Stack	11

2.1.4 Chapter 3: The DFI Generations	12

2.1.5 Chapter 4: Doctrines Added by the Common-IP Campaign	12

2.1.6 Chapter 5: The Line-Up	12

3 Document Information	13

3.1 Revision history	13

3.2 Chapter conventions	14

4 Chapter 1: mc_common_pkg — the family package	15

4.1 1.1 Package role	15

4.2 1.2 memtype_e	15

4.3 1.3 Shared type inventory	16

4.3.1 dram_op_e encoding	17

4.3.2 decoded_addr_t field layout	18

4.4 1.4 Helper-function implementation constraint	18

4.5 1.5 What stays rock-side	18

4.5.1 Legacy CSR widening argument	19

4.6 1.6 Migration plan reference	19

4.7 1.7 Open question: do the rocks import mc_common_pkg directly?	20

5 mc_axi4_plane — AXI4 host interface and read-return ring	21

5.1 Child-instance map	21

5.2 Burst chop, split, and final coalescing	23

5.3 Read return ring: ordering contract	24

5.4 Parameters	25

5.5 Ports	26

5.5.1 Clock, reset, address map, and status	26

5.5.2 Host AXI4 slave	27

5.5.3 Downstream interfaces	29

5.6 What this plane does not know	32

5.7 Open questions	33

5.8 References	33

6 mc_storage_plane — Storage Plane and Entry-Pool FUBs	34

6.1 mc_storage_plane parameters	34

6.2 mc_storage_plane ports	35

6.3 Internal wiring	39

6.4 mc_wr_data_cam — write-data CAM / write buffer	39

6.5 mc_rd_cmd_cam — read-command scheduling window	42

6.6 mc_addr_mapper — flat address to DRAM tuple	44

6.7 Open questions	46

6.8 References	46

7 Chapter 4 — Scheduler Plane and Pick Machinery	47

7.1 1. Top-level parameters	47

7.2 2. Top-level ports	49

7.3 3. Internal structure	55

7.4 4. Schedulable-entry view and unified prospect pool	57

7.5 5. Generic maintenance slot	57

7.6 6. Bank and global timer parameterization	57

7.7 7. Output command descriptor	57

7.8 8. Fire-stage authority	58

7.9 9. Page-policy context	59

7.10 10. P1 init accept/ack gating	59

7.11 11. Open questions	59

7.12 References	60

8 mc_training_plane — LPDDR2 training clients on the generic shell	61

8.1 Composition	61

8.1.1 Slot mapping	62

8.1.2 Maintenance-slot arbitration (shell)	62

8.1.3 One-active trn_cmd mux (shell)	62

8.1.4 mc_clk/dfi_clk CDC placement (shell)	63

8.1.5 Status aggregation	63

8.2 Boundary doctrine: training searches stay PHY-side	63

8.3 Parameters	63

8.4 Ports	64

8.4.1 Clock and reset	64

8.4.2 Runtime enable / build guard	65

8.4.3 CSR controls	65

8.4.4 Maintenance command channel to the scheduler	66

8.4.5 DFI-plane calibration sideband	67

8.4.6 Telemetry / status	68

8.5 Product integration	68

8.5.1 Pumice	69

8.6 ZQ placement as a maintenance client	69

8.7 Plane-only versus pumice-proven	69

8.8 Open questions	70

9 Chapter 6 — mc_ras_plane	71

9.1 Fixed decisions	71

9.2 Parameters	72

9.3 Ports	72

9.3.1 Write-data forward seam (upstream → downstream)	72

9.3.2 Read-return backward seam (downstream → upstream)	73

9.3.3 Reserved CSR surface	74

9.4 Bypass property	75

9.5 Explicitly not decided	75

9.6 Open questions	75

10 mc_dfi_2p1_plane — DFI 2.1 Command / Write / Read Pipeline	76

10.1 Purpose	76

10.2 Sub-FUBs and clock domains	76

10.3 The single controller↔PHY clock-domain crossing	77

10.4 DFI_RATE gearing and bus geometry	78

10.5 Command path	78

10.6 Write serializer	79

10.7 Read aligner	79

10.8 PHY timing knobs	79

10.9 Parameters	80

10.10 Ports	82

10.10.1 Controller domain (ctl_clk)	82

10.10.2 PHY DFI domain (dfi_clk)	83

10.11 Open questions	84

11 Chapter 8 — mc_dfi_3p1_plane	86

11.1 Purpose and scope	86

11.2 What is inherited unchanged from the 2.1 pipeline	86

11.3 What the 3.1 delta adds	87

11.3.1 Command path: mc_dfi_3p1_cmd_path	87

11.3.2 Write-leveling CDC	87

11.3.3 Calibration sideband removed	88

11.4 Parameters	88

11.5 Ports	90

11.5.1 Controller domain (ctl_clk)	90

11.5.2 PHY DFI domain (dfi_clk)	91

11.5.3 Write-leveling crossing ports	93

11.6 Family-boundary doctrine	93

11.7 Open questions and verification notes	94

12 mc_dfi_4p0_plane — DFI 4.0 Generation Delta	95

12.1 Inherited shared pipeline	95

12.2 What the 4.0 delta adds	95

12.2.1 Command path (mc_dfi_4p0_cmd_path)	95

12.3 What the plane leaves unimplemented	96

12.4 Parameters	97

12.4.1 mc_dfi_4p0_plane	97

12.5 Ports	98

12.5.1 mc_dfi_4p0_plane — controller domain (ctl_clk)	98

12.5.2 mc_dfi_4p0_plane — PHY DFI domain (dfi_clk)	100

12.5.3 mc_dfi_4p0_cmd_path — dfi_clk domain	101

12.6 Family boundary rule	102

12.7 Verification record	102

List of Figures

No figures in this document.

List of Tables

Table 1: Table 8.1: mc_dfi_3p1_plane parameters	88

Table 2: Table 8.2: Controller-domain ports	90

Table 3: Table 8.3: PHY DFI-domain ports	91

Table 4: Table 8.4: Write-leveling crossing ports	93

# 1	Common Mas Index

Generated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	mem-ctrl-ip Family — Overarching Hardware Architecture Specification

Version: 0.1 Date: 2026-10-10 Status: v0.1, the vision edition. This book is the family-level HAS: the plane-stack architecture the partner set out during the Phase-2 common-IP campaign, written down as the binding contract the rocks and the products are built against. The per-block microarchitecture lives in common_mas; this book owns the shape of the whole and the rules no single block owns.

Read this first. A controller book (pumice/scoria/andesite HAS/MAS) and a family doc disagree → the family doc wins and the book is the defect. That rule predates this book (the family INDEX states it); this book is one of the docs it protects. Every claim here is either (a) measured in a named rock, (b) a cited JEDEC/DFI clause, or (c) an explicitly marked open question. Nothing else is asserted.

Warning: every timing CSR in this family is a count of MC-clock cycles, correct only at the MC clock frequency it was derived for. A frequency change invalidates the whole timing set at once; software must re-derive and reprogram it before traffic resumes. Stale values present as data corruption, not errors. Each rock’s HAS carries this warning at the register map; this book makes it family doctrine (Ch 4).

## 2.1	Document Organization

Note: All chapters linked below for automated document generation.

### 2.1.1	Front Matter

- Document Information

### 2.1.2	Chapter 1: Introduction

- Purpose, Scope, and the Two Lines

### 2.1.3	Chapter 2: The Plane Stack

- The Architecture: axi4 → storage → scheduler → (ras) → training → dfi

- The Descriptor Seam

### 2.1.4	Chapter 3: The DFI Generations

- Why dfi_2p1_plane / dfi_3p1_plane / dfi_4p0_plane Are Separate Blocks

### 2.1.5	Chapter 4: Doctrines Added by the Common-IP Campaign

- New Family Doctrines

### 2.1.6	Chapter 5: The Line-Up

- The common-ip Layout

- Products and Research: granite / gabbro / diorite

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Document Information

| Field | Value |
| --- | --- |
| Document | mem-ctrl-ip Common IP — Micro-Architecture Specification |
| Version | 0.1 |
| Date | 2026-10-10 |
| Status | Partner-review draft, land-and-review with the common_has v0.1 book |
| Owner | mem-ctrl-ip family (common-ip/docs/) |
| Scope | One chapter per common block: package, eight planes, and the shared FUBs inside their owning plane’s chapter |

## 3.1	Revision history

| Version | Date | Change |
| --- | --- | --- |
| 0.1 | 2026-10-10 | First edition, written against the Phase-2 extraction as landed: mc_common_pkg, eight planes (mc_axi4_plane, mc_storage_plane, mc_scheduler_plane, mc_training_plane, mc_ras_plane, mc_dfi_2p1_plane, mc_dfi_3p1_plane, mc_dfi_4p0_plane), and seventeen shared FUBs. Skeleton blocks (mc_ras_plane, mc_training_plane shell, per-generation DFI deltas) are documented honestly thin. |

## 3.2	Chapter conventions

- RTL citations use path:line against common-ip/rtl/ unless noted.

- Provenance citations use the rock books (pumice_mas, scoria_mas, andesite_mas) and the rock bug/issue records (pumice BUG-021, scoria BUG-001/002, ISSUE-002, and the campaign ledger).

- Parameter/knob detail lives in the knobs inventories (mc_*_knobs.md beside this book) and is summarized, not duplicated.

- The evidentiary rule (index “Read this first”) applies to every chapter.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Chapter 1: mc_common_pkg — the family package

mc_common_pkg is the single source of family-shared types for the memory-controller research IPs (pumice DDR2/LPDDR2, scoria DDR3/LPDDR3, andesite DDR4/LPDDR4). The package was extracted on 2026-10-10 during the Phase 2 common-IP reorganization (includes/mc_common_pkg.sv:17-18). Its encodings are the family encodings; the design authority is family doc 01 (01_mem_ctrl_pkg.md). The plane-stack context is in the family HAS (common_has_index.md); this chapter documents only the package contents and the rock-side boundary.

## 4.1	1.1 Package role

mc_common_pkg holds the types that were previously duplicated, near-identically, across pumice_pkg, scoria_pkg, and andesite_pkg (includes/mc_common_pkg.sv:19-23). It is imported directly by every common plane and shared FUB in common-ip/rtl/ (for example, macro/mc_axi4_plane.sv:178, macro/mc_scheduler_plane.sv:26, fub/mc_cmd_arbiter.sv:34, fub/mc_addr_mapper.sv:55). The package contains no parameters, no build-time knobs, and no CSR definitions; it is a pure type-and-function package meant to be imported with ::* at module scope.

## 4.2	1.2 memtype_e

memtype_e is a 3-bit packed enum (includes/mc_common_pkg.sv:41-48) whose encoding is owned by family doc 01 Table 1.0. Bit [2] is the LP axis; bits [1:0] form a generation field.

| memtype_e | Member | memtype_e[1:0] | memtype_e[2] LP |
| --- | --- | --- | --- |
| 3’b000 | MEMTYPE_DDR2 | 2’b00 (generation 2) | 0 |
| 3’b001 | MEMTYPE_DDR3 | 2’b01 (generation 3) | 0 |
| 3’b010 | MEMTYPE_DDR4 | 2’b10 (generation 4) | 0 |
| 3’b100 | MEMTYPE_LPDDR2 | 2’b00 | 1 |
| 3’b101 | MEMTYPE_LPDDR3 | 2’b01 | 1 |
| 3’b110 | MEMTYPE_LPDDR4 | 2’b10 | 1 |
| 3’b011, 3’b111 | reserved | 2’b11 (generation 5, basalt) | — |

The reserved codes (3'b011 and 3'b111) are illegal and must decode as an error rather than alias to a legal member (includes/mc_common_pkg.sv:36-39).

## 4.3	1.3 Shared type inventory

The package exports the following enums, struct, and helper functions. Every item is used by at least one common plane or FUB.

| Symbol | Kind | File:line | Notes |
| --- | --- | --- | --- |
| memtype_e | enum | includes/mc_common_pkg.sv:41-48 | 3-bit, six legal members, two reserved codes |
| dram_op_e | enum | includes/mc_common_pkg.sv:59-78 | 5-bit superset of legacy 4-bit op tables; OP_MPC = 5'h10 is the LPDDR4 addition |
| bank_state_e | enum | includes/mc_common_pkg.sv:84-92 | bank-machine state |
| page_policy_e | enum | includes/mc_common_pkg.sv:101-105 | open/close plus reserved; pumice’s PAGE_POLICY_HAPPY_HYBRID (2’h2) was retired 2026-08-25 |
| decoded_addr_t | packed struct | includes/mc_common_pkg.sv:114-119 | rank/bank/row/col, padded to 4/4/18/14 bits |
| is_column_op | function | includes/mc_common_pkg.sv:128-130 | true for RD/RDA/WR/WRA |
| is_write_op | function | includes/mc_common_pkg.sv:132-134 | true for WR/WRA |
| is_read_op | function | includes/mc_common_pkg.sv:136-138 | true for RD/RDA |
| is_refresh_op | function | includes/mc_common_pkg.sv:140-142 | true for REF/REFPB |
| has_auto_pre | function | includes/mc_common_pkg.sv:144-146 | true for RDA/WRA |
| is_zq_op | function | includes/mc_common_pkg.sv:148-150 | true for ZQCS/ZQCL |

### 4.3.1	dram_op_e encoding

dram_op_e is widened to 5 bits so that OP_MPC exists once in the family (includes/mc_common_pkg.sv:21-23). Values 5'h00 through 5'h0F are identical to every rock’s legacy 4-bit table (includes/mc_common_pkg.sv:56-57); the wire-level translation per memtype lives in each rock’s DFI command formatter.

| Value | Member | Meaning |
| --- | --- | --- |
| 5’h00 | OP_NOP | no operation |
| 5’h01 | OP_ACT | activate |
| 5’h02 | OP_RD | read |
| 5’h03 | OP_RDA | read with auto-precharge |
| 5’h04 | OP_WR | write |
| 5’h05 | OP_WRA | write with auto-precharge |
| 5’h06 | OP_PRE | precharge |
| 5’h07 | OP_PREA | precharge all banks |
| 5’h08 | OP_REF | all-bank refresh |
| 5’h09 | OP_REFPB | per-bank refresh |
| 5’h0A | OP_MRS | mode register set |
| 5’h0B | OP_ZQCS | ZQ calibration short |
| 5’h0C | OP_ZQCL | ZQ calibration long |
| 5’h0D | OP_SREFE | self-refresh entry |
| 5’h0E | OP_SREFX | self-refresh exit |
| 5’h0F | OP_DPDE | deep-power-down entry |
| 5’h10 | OP_MPC | LPDDR4 multipurpose command; CA-bus encoding per the andesite kmap book |

### 4.3.2	decoded_addr_t field layout

| Field | Width | Notes |
| --- | --- | --- |
| rank | 4 bits | pad to 4; actual width is $clog2(NUM_RANKS) |
| bank | 4 bits | pad to 4; DDR3 uses 3 |
| row | 18 bits | pad to 18; MT41J256M16 uses 15 |
| col | 14 bits | pad to 14; MT41J256M16 uses 10 |

## 4.4	1.4 Helper-function implementation constraint

Each helper returns a single-line expression (includes/mc_common_pkg.sv:124-126). The RTL comment records that multi-line return expressions break yosys’ frontend, which is why pumice’s formal flow historically had to sv2v-flatten wrappers that imported its package.

## 4.5	1.5 What stays rock-side

The three rock packages still exist, but their contents are now thin.

| Rock package | Remaining content | File |
| --- | --- | --- |
| scoria_pkg | import/export shim for mc_common_pkg only | research-ip/scoria-ddr3-lpddr3/rtl/includes/scoria_pkg.sv:29-33 |
| andesite_pkg | import/export shim for mc_common_pkg only | research-ip/andesite-ddr4-lpddr4/rtl/includes/andesite_pkg.sv:30-34 |
| pumice_pkg | import/export shim plus addr_map_scheme_e and odt_rule_e | research-ip/pumice-ddr2-lpddr2/rtl/includes/pumice_pkg.sv:28-60 |

addr_map_scheme_e is retained only because retired macro/OLD modules still reference the type in regression sentinels (pumice_pkg.sv:36-40). odt_rule_e is pumice-specific ODT rule select (pumice_pkg.sv:49-59).

### 4.5.1	Legacy CSR widening argument

The rocks’ PHY_TIMING.memtype CSR field stays 1-bit wide for Phase 2, with per-rock legacy values (includes/mc_common_pkg.sv:25-27; mc_common_pkg_knobs.md). Pumice maps 0→MEMTYPE_DDR2 and 1→MEMTYPE_LPDDR2 at research-ip/pumice-ddr2-lpddr2/rtl/top/pumice_top.sv:199-207. Scoria maps 0→MEMTYPE_DDR3 and 1→MEMTYPE_LPDDR3 at research-ip/scoria-ddr3-lpddr3/rtl/top/scoria_top.sv:237-245. Andesite’s CSR rebuild is a follow-up: andesite_core.sv currently takes memtype_i as a top-level port (research-ip/andesite-ddr4-lpddr4/rtl/top/andesite_core.sv:90-91) and no andesite_top.sv exists yet, so the legacy cast site is not yet in RTL.

The same config block in pumice and scoria also carries a page-policy cast warning: REFRESH_TUNING.page_policy_or uses a software encoding while page_policy_e uses OPEN=0/CLOSE=1. The RTL comment records that the old raw cast made software-OPEN run CLOSE and software-CLOSE run HYBRID, corrupting the open_page/reorder config axis (research-ip/pumice-ddr2-lpddr2/rtl/top/pumice_top.sv:208-212, research-ip/scoria-ddr3-lpddr3/rtl/top/scoria_top.sv:246-250; issue #42). Note that mc_common_pkg::page_policy_e retired the HYBRID member in favor of PAGE_POLICY_RSVD = 2'h3 (includes/mc_common_pkg.sv:97-105).

## 4.6	1.6 Migration plan reference

The migration that created mc_common_pkg is the Phase 2 migration deferred in family doc 01 (01_mem_ctrl_pkg.md). The conditions were: execute once, when andesite RTL bring-up starts, with both shipping controllers moving together. The knob inventory (mc_common_pkg_knobs.md) records what moved, what stayed, the legacy-CSR ruling, and the width-change consequences expected during adoption (e.g., dram_op_e re-width 4→5 and memtype_e re-width 1→3).

## 4.7	1.7 Open question: do the rocks import mc_common_pkg directly?

No. As of the Phase 2 extraction, common planes and FUBs import mc_common_pkg::* directly, but rock RTL still imports its own rock package (pumice_pkg::*, scoria_pkg::*, or andesite_pkg::*). Each rock package in turn imports and exports mc_common_pkg::* (research-ip/scoria-ddr3-lpddr3/rtl/includes/scoria_pkg.sv:31-32, research-ip/andesite-ddr4-lpddr4/rtl/includes/andesite_pkg.sv:32-33, research-ip/pumice-ddr2-lpddr2/rtl/includes/pumice_pkg.sv:30-31). Therefore mc_common_pkg exports every family-shared symbol the rocks need, but the rocks do not source it directly. pumice_pkg additionally exports pumice-only symbols (addr_map_scheme_e, odt_rule_e) that the common package does not and should not carry.

This is a stable, intentional shim: it leaves rock import statements untouched while making the common package the single source of family-shared encodings.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	mc_axi4_plane — AXI4 host interface and read-return ring

Module: macro/mc_axi4_plane.sv Child FUBs: fub/mc_wr_splitter.sv, fub/mc_axi_burst_chopper.sv, fub/mc_wr_intake.sv, fub/mc_rd_intake.sv, fub/mc_rd_return_ring.sv

mc_axi4_plane is the host-facing AXI4 slave boundary of the common memory-controller stack. It transforms host AXI4 read and write bursts into DFI-burst-sized descriptors that cross into mc_storage_plane, sources read-your-write snarf data from the write-data CAM, and presents the final AXI4 read-data and write-response channels to the host (macro/mc_axi4_plane.sv:8-12). It is the N=1 provider of the descriptor seam in lite builds (see common_has chapter 2).

mc_axi4_plane block diagram

## 5.1	Child-instance map

| Instance | Module | Parameter bindings in this plane | Role |
| --- | --- | --- | --- |
| u_wr_split | mc_wr_splitter | AXI_ID_WIDTH=IW, AXI_ADDR_WIDTH=AW, AXI_DATA_WIDTH=DW, AXI_USER_WIDTH=UW, AXI_BEATS_PER_BURST=AXI_BEATS_PER_BURST | Split host AW/W into DFI-burst sub-bursts and reframe WLAST (macro/mc_axi4_plane.sv:207-256) |
| u_rd_split | mc_axi_burst_chopper | AXI_ID_WIDTH=IW, AXI_ADDR_WIDTH=AW, AXI_USER_WIDTH=UW, STRB_BYTES=SW, AXI_BEATS_PER_BURST=AXI_BEATS_PER_BURST | Split host AR into DFI-burst sub-commands (macro/mc_axi4_plane.sv:259-296) |
| u_wr_intake | mc_wr_intake | full address-map set plus AXI_BEATS_PER_BURST | Write descriptor push + write-data pop + commit-driven B (macro/mc_axi4_plane.sv:304-370) |
| u_rd_intake | mc_rd_intake | as u_wr_intake, plus ORDER_FIFO_DEPTH=RD_RET_DEPTH+8 | Read descriptor push + snarf probe + R return (macro/mc_axi4_plane.sv:373-445) |
| u_rd_ring | mc_rd_return_ring | DEPTH=RD_RET_DEPTH, AXI_DATA_WIDTH=DW, AXI_BEATS_PER_BURST=AXI_BEATS_PER_BURST | In-flight read return buffer (macro/mc_axi4_plane.sv:450-475) |

## 5.2	Burst chop, split, and final coalescing

mc_axi_burst_chopper is an FSM-free AR/AW burst chopper with one active bit, a remaining-beats counter, and a running address. Each sub-command spans at most AXI_BEATS_PER_BURST AXI beats; addresses step by AXI_BURST_BYTES so each sub-command lands on a DRAM-burst boundary (fub/mc_axi_burst_chopper.sv:8-14, fub/mc_axi_burst_chopper.sv:44-64). A ragged final chunk takes min(remaining, AXI_BEATS_PER_BURST) beats (fub/mc_axi_burst_chopper.sv:27-29).

mc_wr_splitter wraps the same chopper for AW with PAD_TO_CHUNK=1 and reframes the W stream so every sub-command sees exactly one WLAST at its AXI_BEATS_PER_BURST boundary (fub/mc_wr_splitter.sv:8-12). The WLAST re-frame is driven by a W-beat down-counter, not by the AW FSM, so it stays exact for any AXI_BEATS_PER_BURST including 1 (fub/mc_wr_splitter.sv:15-22). Short write bursts are completed with zero-strobe filler beats; strb=0 becomes DM=1 in the DFI write serializer (fub/mc_wr_splitter.sv:134-139).

Both splitters emit an aggregation sideband: m_ax_agg is high when the host command splits into more than one sub-command, and m_ax_last is high on the final sub-command (fub/mc_axi_burst_chopper.sv:31-37). The intakes push these bits through their FIFOs so the return path can collapse per-sub B/RLAST into a single host response without a separate aggregator (macro/mc_axi4_plane.sv:200-204). For reads, the intake forwards only the beats the host requested and drops excess DRAM-burst beats; RLAST is generated on the last requested beat, and for a split burst it is asserted only when w_head_last is also set (fub/mc_rd_intake.sv:380-384, fub/mc_rd_intake.sv:465-473).

## 5.3	Read return ring: ordering contract

mc_rd_return_ring separates the in-flight read count from the read-command CAM depth. The CAM entry lives from insert to issue only; after issue, a ticket follows the read through DRAM (fub/mc_rd_return_ring.sv:8-12). The ordering rules are:

- Allocation order equals AR order. A ticket is allocated at the ring tail when alloc_valid_i is asserted with the CAM insert (fub/mc_rd_return_ring.sv:53-55, fub/mc_rd_return_ring.sv:97-100).

- DFI returns are accepted in issue order, mapped by an internal ticket FIFO (fub/mc_rd_return_ring.sv:60-63, fub/mc_rd_return_ring.sv:106-120).

- Drain order equals ring/AR order. The ring drains from its head once the head slot’s data is complete; because allocation order is AR order, per-ID AXI reads return in request order structurally (fub/mc_rd_return_ring.sv:74-80, fub/mc_rd_return_ring.sv:135-143).

- A slot is freed only when its last beat is consumed by the drain (fub/mc_rd_return_ring.sv:185-186).

The ring is sized by RD_RET_DEPTH (default 32). This decoupling fixed a bandwidth ceiling: with the CAM holding reads for the full DRAM round trip, eight entries over a ~27-cycle round trip bounded board reads to about 180 MB/s; the ring fix took board reads to ~572 MB/s (research-ip/pumice-ddr2-lpddr2/docs/pumice_mas/ch02_blocks/22_rd_return_ring.md:32-42, inherited from the pumice measured history).

## 5.4	Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| AXI_ID_WIDTH | 8 | Host AXI ID width |
| AXI_ADDR_WIDTH | 32 | Host AXI address width |
| AXI_DATA_WIDTH | 64 | Host AXI data width (and DFI word width in this plane) |
| AXI_USER_WIDTH | 1 | Host AXI user width |
| DRAM_BEAT_WIDTH | 64 | Retained only to size DRAM_BURST_BYTES; pumice instantiates this with the AXI data width, not the DRAM beat width (macro/mc_axi4_plane.sv:29-34) |
| NUM_RANKS | 1 | Rank count passed to the address mapper |
| NUM_BANKS | 8 | Bank count passed to the address mapper |
| ROW_WIDTH | 14 | Row address width passed to the address mapper |
| COL_WIDTH | 10 | Column address width passed to the address mapper |
| BYTE_OFFSET_WIDTH | 3 | Byte-offset width passed to the address mapper |
| BG_WIDTH | 1 | Bank-group width; HAS_BG=0 keeps generations 2/3 bit-identical (macro/mc_axi4_plane.sv:41-44) |
| HAS_BG | 0 | Bank-group enable; andesite passes 1/2 (macro/mc_axi4_plane.sv:41-44) |
| AXI_BEATS_PER_BURST | 4 | AXI beats per DFI burst; this is what pumice passes as BURST_WORDS, not the JEDEC burst length (macro/mc_axi4_plane.sv:45-51) |
| NUM_ENTRIES | 8 | Storage/scheduler entry count passed from core |
| N_SRAM_SLOTS | NUM_ENTRIES | SRAM slot count |
| N_SCHED_LU | 4 | Scheduler lookup port count |
| AGE_WIDTH | 16 | Age field width |
| RD_RET_DEPTH | 32 | In-flight read count (ring depth) |
| IW, AW, DW, UW, SW, BKW, PTRW, RKW, DRAM_BURST_BYTES | derived | Shorthand widths; DRAM_BURST_BYTES = AXI_BEATS_PER_BURST * (DRAM_BEAT_WIDTH / 8) |

## 5.5	Ports

### 5.5.1	Clock, reset, address map, and status

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| aclk | input | 1 | Plane clock |
| aresetn | input | 1 | Active-low reset |
| bank_lsb_i | input | 5 | Address-map bank LSB selector |
| hash_en_i | input | 1 | Bank hash enable |
| hash_seed_i | input | 8 | Bank hash seed |
| busy_o | output | 1 | OR of write-intake, read-intake, and return-ring busy flags (macro/mc_axi4_plane.sv:477) |

### 5.5.2	Host AXI4 slave

| Channel | Port | Direction | Width | Description |
| --- | --- | --- | --- | --- |
| AW | s_axi_awid | input | IW | Write address ID |
| AW | s_axi_awaddr | input | AW | Write address |
| AW | s_axi_awlen | input | 8 | Burst length |
| AW | s_axi_awsize | input | 3 | Burst size |
| AW | s_axi_awburst | input | 2 | Burst type |
| AW | s_axi_awlock | input | 1 | Lock signal |
| AW | s_axi_awcache | input | 4 | Cache signal |
| AW | s_axi_awprot | input | 3 | Protection signal |
| AW | s_axi_awqos | input | 4 | QoS signal |
| AW | s_axi_awregion | input | 4 | Region signal |
| AW | s_axi_awuser | input | UW | User sideband |
| AW | s_axi_awvalid | input | 1 | Write address valid |
| AW | s_axi_awready | output | 1 | Write address ready |
| W | s_axi_wdata | input | DW | Write data |
| W | s_axi_wstrb | input | SW | Write strobe |
| W | s_axi_wlast | input | 1 | Write data last |
| W | s_axi_wuser | input | UW | Write data user |
| W | s_axi_wvalid | input | 1 | Write data valid |
| W | s_axi_wready | output | 1 | Write data ready |
| B | s_axi_bid | output | IW | Write response ID |
| B | s_axi_bresp | output | 2 | Write response |
| B | s_axi_buser | output | UW | Write response user |
| B | s_axi_bvalid | output | 1 | Write response valid |
| B | s_axi_bready | input | 1 | Write response ready |
| AR | s_axi_arid | input | IW | Read address ID |
| AR | s_axi_araddr | input | AW | Read address |
| AR | s_axi_arlen | input | 8 | Burst length |
| AR | s_axi_arsize | input | 3 | Burst size |
| AR | s_axi_arburst | input | 2 | Burst type |
| AR | s_axi_arlock | input | 1 | Lock signal |
| AR | s_axi_arcache | input | 4 | Cache signal |
| AR | s_axi_arprot | input | 3 | Protection signal |
| AR | s_axi_arqos | input | 4 | QoS signal |
| AR | s_axi_arregion | input | 4 | Region signal |
| AR | s_axi_aruser | input | UW | User sideband |
| AR | s_axi_arvalid | input | 1 | Read address valid |
| AR | s_axi_arready | output | 1 | Read address ready |
| R | s_axi_rid | output | IW | Read data ID |
| R | s_axi_rdata | output | DW | Read data |
| R | s_axi_rresp | output | 2 | Read response |
| R | s_axi_rlast | output | 1 | Read data last |
| R | s_axi_ruser | output | UW | Read data user |
| R | s_axi_rvalid | output | 1 | Read data valid |
| R | s_axi_rready | input | 1 | Read data ready |

### 5.5.3	Downstream interfaces

| Group | Port | Direction | Width | Description |
| --- | --- | --- | --- | --- |
| WR push | aw_push_valid_o | output | 1 | Write command valid |
| WR push | aw_push_ready_i | input | 1 | Write command ready |
| WR push | aw_push_bank_o | output | BKW | Decoded bank |
| WR push | aw_push_row_o | output | ROW_WIDTH | Decoded row |
| WR push | aw_push_col_o | output | COL_WIDTH | Decoded column |
| WR push | aw_push_id_o | output | IW | Write AXI ID |
| WR push | aw_push_qos_o | output | 4 | Write QoS |
| WR push | aw_push_agg_o | output | 1 | Part of a split host burst |
| WR push | aw_push_last_o | output | 1 | Final sub-command of host burst |
| WR data | wd_valid_o | output | 1 | Write data valid |
| WR data | wd_ready_i | input | 1 | Write data ready |
| WR data | wd_data_o | output | DW | Write data |
| WR data | wd_strb_o | output | SW | Write strobe |
| WR data | wd_last_o | output | 1 | Write data last |
| WR done | wr_done_valid_i | input | 1 | Write commit completion |
| WR done | wr_done_id_i | input | IW | Completion ID |
| Snarf probe | snarf_probe_valid_o | output | 1 | Probe valid |
| Snarf probe | snarf_probe_bank_o | output | BKW | Probe bank |
| Snarf probe | snarf_probe_row_o | output | ROW_WIDTH | Probe row |
| Snarf probe | snarf_probe_col_o | output | COL_WIDTH | Probe column |
| Snarf probe | snarf_probe_id_o | output | IW | Read AXI ID |
| Snarf probe | snarf_probe_len_o | output | 8 | Read AXI length |
| Snarf hit | snarf_hit_i | input | 1 | Probe hit (combinational, one cycle delayed by the CAM register) |
| Snarf hit | snarf_accept_o | output | 1 | AR admitted as a snarf hit |
| Snarf data | snarf_rd_valid_i | input | 1 | Snarf data valid |
| Snarf data | snarf_rd_ready_o | output | 1 | Snarf data ready |
| Snarf data | snarf_rd_data_i | input | DW | Snarf data |
| Snarf data | snarf_rd_last_i | input | 1 | Snarf data last |
| RD push | ar_push_valid_o | output | 1 | Read command valid |
| RD push | ar_push_ready_i | input | 1 | Read command ready |
| RD push | ar_push_bank_o | output | BKW | Decoded bank |
| RD push | ar_push_row_o | output | ROW_WIDTH | Decoded row |
| RD push | ar_push_col_o | output | COL_WIDTH | Decoded column |
| RD push | ar_push_id_o | output | IW | Read AXI ID |
| RD push | ar_push_qos_o | output | 4 | Read QoS |
| Ring alloc | rt_alloc_ready_o | output | 1 | Ring has space to allocate a ticket |
| Ring alloc | rt_alloc_ticket_o | output | $clog2(RD_RET_DEPTH) | Allocated ticket (ring tail) |
| Ring issue | rd_iss_ready_o | output | 1 | Ring ready for issue notify |
| Ring issue | rd_iss_valid_i | input | 1 | Issue notify valid |
| Ring issue | rd_iss_ticket_i | input | $clog2(RD_RET_DEPTH) | Ticket that issued |
| Ring gate | rd_cam_ins_ready_i | input | 1 | Combined gate for ring allocation with ar_push_valid_o (macro/mc_axi4_plane.sv:457) |
| DFI return | rd_dfi_ret_valid_i | input | 1 | DFI return data valid |
| DFI return | rd_dfi_ret_ready_o | output | 1 | DFI return data ready |
| DFI return | rd_dfi_ret_data_i | input | DW | DFI return data |
| DFI return | rd_dfi_ret_resp_i | input | 2 | DFI return response |
| DFI return | rd_dfi_ret_last_i | input | 1 | DFI return last beat |

## 5.6	What this plane does not know

- DRAM timing. No tRAS, tRCD, CAS latency, refresh, or power-state logic resides here.

- Scheduling. The plane produces descriptor pushes and accepts scheduler handshakes through the storage plane; it does not pick oldest entries or arbitrate banks.

- DFI protocol generation. All DFI command, data, and timing formatting is delegated to the DFI planes.

- Write-data CAM internals. The CAM fill, commit, and snarf datapaths are in mc_storage_plane; this plane exposes only the raw push, snarf probe, and completion ports.

## 5.7	Open questions

- Whether NUM_ENTRIES, N_SRAM_SLOTS, N_SCHED_LU, and AGE_WIDTH are still consumed by mc_axi4_plane itself or are pass-through remnants from an earlier integration; they appear only in the parameter list and do not drive any local logic in the reviewed RTL (macro/mc_axi4_plane.sv:52-55).

- Whether bank_lsb_i is allowed to place the bank boundary inside the AXI beat; the intakes mask the beat-offset bits before address mapping, but the resulting column alignment behavior for data-width converters wider than the device word is not verified here.

## 5.8	References

- Smith, Pleszkun, “Implementation of Precise Interrupts in Pipelined Processors,” IEEE Trans. Computers 1988 (orig. ISCA 1985). DOI (conceptual convergence: reorder buffer)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	mc_storage_plane — Storage Plane and Entry-Pool FUBs

Files: macro/mc_storage_plane.sv, fub/mc_wr_data_cam.sv, fub/mc_rd_cmd_cam.sv, fub/mc_addr_mapper.sv

The storage plane holds the family’s schedulable-entry pools: the write-data CAM (write buffer + snarf source) and the read-command CAM (read scheduling window). It presents the scheduler-facing bank/row/col image and accepts the read-snoop probe from the upstream AXI4 plane. Within the eight-plane family stack it is the N=8 full provider of the descriptor seam; the stack architecture is documented in common_has.

mc_storage_plane internal block diagram

## 6.1	mc_storage_plane parameters

| Parameter | Default | Description |
| --- | --- | --- |
| AXI_ID_WIDTH | 8 | AXI ID width (IW) |
| AXI_DATA_WIDTH | 64 | AXI data width (DW) |
| NUM_RANKS | 1 | Rank count |
| NUM_BANKS | 8 | Bank count (BKW = $clog2) |
| ROW_WIDTH | 14 | Row field width |
| COL_WIDTH | 10 | Column field width |
| AXI_BEATS_PER_BURST | 4 | Beats per AXI burst |
| NUM_ENTRIES | 8 | CAM / scheduling-window depth (PTRW = $clog2) |
| N_SRAM_SLOTS | NUM_ENTRIES | Write-data SRAM slots |
| N_SCHED_LU | 4 | Parallel scheduler lookup ports |
| AGE_WIDTH | 16 | Free-running age width |
| RD_RET_DEPTH | 32 | Return-ring depth / ticket space; power of 2 |
| IW | AXI_ID_WIDTH | Derived alias |
| DW | AXI_DATA_WIDTH | Derived alias |
| SW | AXI_DATA_WIDTH/8 | Derived strobe width |
| BKW | $clog2(NUM_BANKS) | Derived bank-index width |
| PTRW | $clog2(NUM_ENTRIES) | Derived pointer width |
| RKW | (NUM_RANKS>1) ? ... : 1 | Derived rank-index width |

Parameter block: macro/mc_storage_plane.sv:22-45.

## 6.2	mc_storage_plane ports

| Group | Signals | Direction | Width | Description |
| --- | --- | --- | --- | --- |
| Clock/reset | aclk, aresetn | input | 1, 1 | Clock, active-low reset |
| WR intake cmd | aw_push_valid_i, aw_push_ready_o | in / out | 1, 1 | AW push handshake |
| WR intake cmd | aw_push_bank_i, aw_push_row_i, aw_push_col_i, aw_push_id_i, aw_push_qos_i, aw_push_agg_i, aw_push_last_i | input | BKW, ROW_WIDTH, COL_WIDTH, IW, 4, 1, 1 | Decoded key, ID, QoS, aggregate, last |
| WR intake data | wd_valid_i, wd_ready_o, wd_data_i, wd_strb_i, wd_last_i | in / out / in / in / in | 1, 1, DW, SW, 1 | W-data handshake, data, strobes, last |
| WR done | wr_done_valid_o, wr_done_id_o | output | 1, IW | Commit-done B response |
| RD snarf probe | snarf_probe_valid_i, snarf_probe_bank_i, snarf_probe_row_i, snarf_probe_col_i, snarf_probe_id_i, snarf_probe_len_i, snarf_hit_o, snarf_accept_i | in / in / in / in / in / in / out / in | 1, BKW, ROW_WIDTH, COL_WIDTH, IW, 8, 1, 1 | Read probe key/id/len, hit, accept |
| RD snarf stream | snarf_rd_valid_o, snarf_rd_ready_i, snarf_rd_data_o, snarf_rd_last_o | out / in / out / out | 1, 1, DW, 1 | Forwarded write-data stream |
| RD intake cmd | ar_push_valid_i, ar_push_ready_o, ar_push_bank_i, ar_push_row_i, ar_push_col_i, ar_push_id_i, ar_push_qos_i | in / out / in / in / in / in / in | 1, 1, BKW, ROW_WIDTH, COL_WIDTH, IW, 4 | AR push key/id/QoS |
| Return ring | rt_alloc_ready_i, rt_alloc_ticket_i, rd_iss_ready_i, rd_iss_valid_o, rd_iss_ticket_o, rd_cam_ins_ready_o | in / in / in / out / out / out | 1, $clog2(RD_RET_DEPTH), 1, 1, $clog2(RD_RET_DEPTH), 1 | Ticket allocation, issue notify, CAM ready |
| Scheduler ctrl | sched_age_thresh_i | input | 8 | Age threshold / 16 (0 = off) |
| Scheduler ctrl | wr_commit_valid_i, wr_commit_ready_o, wr_commit_slot_i | in / out / in | 1, 1, PTRW | Write commit |
| Scheduler ctrl | rd_issue_valid_i, rd_issue_ready_o, rd_issue_slot_i | in / out / in | 1, 1, PTRW | Read issue |
| WR sched vectors | wr_sch_valid_o, wr_sch_bank_o, wr_sch_row_o, wr_sch_col_o, wr_sch_older_o, wr_sch_age_exceed_o, wr_sch_qos_o, wr_sch_head_rel_o | output | NUM_ENTRIES, NUM_ENTRIES*BKW, NUM_ENTRIES*ROW_WIDTH, NUM_ENTRIES*COL_WIDTH, NUM_ENTRIES*NUM_ENTRIES, NUM_ENTRIES, NUM_ENTRIES*4, 16 | Per-entry WR scheduling image |
| RD sched vectors | rd_sch_valid_o, rd_sch_bank_o, rd_sch_row_o, rd_sch_col_o, rd_sch_older_o, rd_sch_age_exceed_o, rd_sch_qos_o, rd_sch_head_rel_o | output | (same as WR) | Per-entry RD scheduling image |
| WR commit data | wr_cm_rd_valid_o, wr_cm_rd_ready_i, wr_cm_rd_data_o, wr_cm_rd_strb_o, wr_cm_rd_last_o | out / in / out / out / out | 1, 1, DW, SW, 1 | Commit data stream |
| Status | busy_o | output | 1 | Plane busy |

Port block: macro/mc_storage_plane.sv:46-159.

## 6.3	Internal wiring

mc_storage_plane instantiates mc_wr_data_cam at macro/mc_storage_plane.sv:165 and mc_rd_cmd_cam at macro/mc_storage_plane.sv:250. The legacy oldest_* and sched_lu_* ports on both CAMs are tied off and pruned by synthesis (macro/mc_storage_plane.sv:205-207 and macro/mc_storage_plane.sv:269-284). A read is admitted only when both the CAM and the return ring have room: assign ar_push_ready_o = rd_cam_ins_ready_o && rt_alloc_ready_i; (macro/mc_storage_plane.sv:247). The return ring hands out a ticket at insert (macro/mc_storage_plane.sv:104-105) that the CAM stores and forwards on issue (macro/mc_storage_plane.sv:297-299).

## 6.4	mc_wr_data_cam — write-data CAM / write buffer

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_ENTRIES | 8 | CAM depth (PTRW) |
| N_SCHED_LU | 4 | Parallel scheduler lookups |
| NUM_BANKS | 8 | Bank count (BKW) |
| ROW_WIDTH | 14 | Row width |
| COL_WIDTH | 10 | Column width |
| AXI_ID_WIDTH | 8 | AXI ID width (IW) |
| AXI_DATA_WIDTH | 64 | Data width (DW) |
| AXI_BEATS_PER_BURST | 4 | Beats per burst |
| AGE_WIDTH | 16 | Age-counter width |
| N_SRAM_SLOTS | NUM_ENTRIES | SRAM data slots |
| WR_DRAIN_AHEAD | 2 | Bursts allowed in drain queue |
| IW / DW / SW / BKW / PTRW / SPTRW / BCW | derived | Width aliases |

Parameter block: fub/mc_wr_data_cam.sv:24-45.

| Signals | Direction | Width | Description |
| --- | --- | --- | --- |
| aclk, aresetn | input | 1, 1 | Clock, reset |
| ins_valid_i, ins_ready_o | in / out | 1, 1 | Insert handshake |
| ins_bank_i, ins_row_i, ins_col_i, ins_id_i, ins_qos_i, ins_agg_i, ins_last_i | input | BKW, ROW_WIDTH, COL_WIDTH, IW, 4, 1, 1 | Command key/id/QoS/aggregate/last |
| wd_valid_i, wd_ready_o, wd_data_i, wd_strb_i, wd_last_i | in / out / in / in / in | 1, 1, DW, SW, 1 | Fill data handshake and beat |
| snarf_probe_valid_i, snarf_probe_bank_i, snarf_probe_row_i, snarf_probe_col_i, snarf_probe_id_i, snarf_probe_len_i | input | 1, BKW, ROW_WIDTH, COL_WIDTH, IW, 8 | Read probe |
| snarf_hit_o, snarf_accept_i | out / in | 1, 1 | Hit / accepted |
| snarf_rd_valid_o, snarf_rd_ready_i, snarf_rd_data_o, snarf_rd_last_o | out / in / out / out | 1, 1, DW, 1 | Snarf data stream |
| oldest_valid_o, oldest_bank_o, oldest_row_o, oldest_col_o, oldest_id_o, oldest_slot_o | output | 1, BKW, ROW_WIDTH, COL_WIDTH, IW, PTRW | Oldest schedulable entry |
| sched_lu_valid_i, sched_lu_bank_i, sched_lu_row_i | input | N_SCHED_LU, N_SCHED_LU*BKW, N_SCHED_LU*ROW_WIDTH | Scheduler lookup requests |
| sched_lu_hit_o, sched_lu_slot_o, sched_lu_col_o, sched_lu_id_o, sched_lu_age_o | output | N_SCHED_LU, N_SCHED_LU*PTRW, N_SCHED_LU*COL_WIDTH, N_SCHED_LU*IW, N_SCHED_LU*AGE_WIDTH | Scheduler lookup results |
| sch_valid_o, sch_bank_o, sch_row_o, sch_col_o, sch_older_o, age_thresh_i, sch_age_exceed_o, sch_qos_o, sch_head_rel_o | out / out / out / out / out / in / out / out / out | NUM_ENTRIES, NUM_ENTRIES*BKW, NUM_ENTRIES*ROW_WIDTH, NUM_ENTRIES*COL_WIDTH, NUM_ENTRIES*NUM_ENTRIES, 8, NUM_ENTRIES, NUM_ENTRIES*4, AGE_WIDTH | Per-entry scheduling image |
| commit_valid_i, commit_ready_o, commit_slot_i | in / out / in | 1, 1, PTRW | Commit handshake |
| cm_rd_valid_o, cm_rd_ready_i, cm_rd_data_o, cm_rd_strb_o, cm_rd_last_o | out / in / out / out / out | 1, 1, DW, SW, 1 | Commit data stream |
| commit_done_valid_o, commit_done_id_o | output | 1, IW | B-response strobe |
| busy_o | output | 1 | CAM busy |

Port block: fub/mc_wr_data_cam.sv:47-153.

The per-entry state is r_valid, r_bank, r_row, r_col, r_id, r_qos, r_age, r_ptr (SRAM slot), r_pv, r_fdone (fill complete), r_sched (committed), r_agg, r_last, plus the age-order matrix r_older (fub/mc_wr_data_cam.sv:158-189). The burst data lives in one BRAM word per (slot, beat) holding {strb, data}, packed as SRW = DW + SW (fub/mc_wr_data_cam.sv:212-225). A single read port is shared by the snarf and commit streams; a 2-deep prefetch skid decouples fetch from consume (fub/mc_wr_data_cam.sv:228-247). Commit has priority, but snarf anti-starvation caps priority preemption at SN_STARVE_LIMIT = 16 cycles (fub/mc_wr_data_cam.sv:249-258).

Read snooping against the un-coalesced write pool (the snarf path) is limited to the safe case documented in the RTL: a read may snarf a write only when (1) the write is not yet scheduled (!r_sched), (2) the AXI IDs match, and (3) the read burst length equals the write burst length, which reduces to arlen == AXI_BEATS_PER_BURST - 1 because every admitted write is exactly AXI_BEATS_PER_BURST beats (fub/mc_wr_data_cam.sv:314-325). The match also requires r_fdone; among candidates the youngest match wins, computed from the age-order matrix rather than a 16-bit relative-age reduction (fub/mc_wr_data_cam.sv:340-364).

The scheduler lookup ports return the oldest {bank, row} match (max rel) among entries that are valid, fill-complete, and not yet scheduled (fub/mc_wr_data_cam.sv:390-414), and the per-entry scheduling vectors expose the same qualified state plus the age-order matrix (fub/mc_wr_data_cam.sv:451-466). The oldest-schedulable pick was replaced by the age-order matrix for timing: the RTL records that the previous 16-bit age-reduce path was the worst path in the design after the mode work, measuring 63.6 ns against a 15 ns period, 89 logic levels, 30 carry chains, and 17.8 ns of logic alone — pumice ISSUE-005 (was PUMICE-017) (fub/mc_wr_data_cam.sv:416-426).

Commit is rate-matched: WR_DRAIN_AHEAD = 2 bounds how far the command stream can run ahead of the data drain. The RTL comments note that the old FIFO-room gate allowed a command to run ~20 aclk ahead and produced a measured 29373-cycle DFI hold (fub/mc_wr_data_cam.sv:507-524). Entry eviction happens at fetch-last so the slot frees before the DFI serialization wait (fub/mc_wr_data_cam.sv:754-769).

## 6.5	mc_rd_cmd_cam — read-command scheduling window

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_ENTRIES | 8 | Scheduling-window depth (PTRW) |
| N_SCHED_LU | 4 | Parallel scheduler lookups |
| NUM_BANKS | 8 | Bank count (BKW) |
| ROW_WIDTH | 14 | Row width |
| COL_WIDTH | 10 | Column width |
| AXI_ID_WIDTH | 8 | AXI ID width (IW) |
| AGE_WIDTH | 16 | Age-counter width |
| RD_RET_DEPTH | 32 | Return-ring ticket space (TW) |
| IW / BKW / PTRW / TW | derived | Width aliases |

Parameter block: fub/mc_rd_cmd_cam.sv:33-46.

| Signals | Direction | Width | Description |
| --- | --- | --- | --- |
| aclk, aresetn | input | 1, 1 | Clock, reset |
| ins_valid_i, ins_ready_o | in / out | 1, 1 | Insert handshake |
| ins_bank_i, ins_row_i, ins_col_i, ins_id_i, ins_qos_i, ins_ticket_i | input | BKW, ROW_WIDTH, COL_WIDTH, IW, 4, TW | Read key/id/QoS + ring ticket |
| sched_lu_valid_i, sched_lu_bank_i, sched_lu_row_i | input | N_SCHED_LU, N_SCHED_LU*BKW, N_SCHED_LU*ROW_WIDTH | Scheduler lookup requests |
| sched_lu_hit_o, sched_lu_slot_o, sched_lu_col_o, sched_lu_id_o, sched_lu_age_o | output | N_SCHED_LU, N_SCHED_LU*PTRW, N_SCHED_LU*COL_WIDTH, N_SCHED_LU*IW, N_SCHED_LU*AGE_WIDTH | Scheduler lookup results |
| sch_valid_o, sch_bank_o, sch_row_o, sch_col_o, sch_older_o, age_thresh_i, sch_age_exceed_o, sch_qos_o, sch_head_rel_o | out / out / out / out / out / in / out / out / out | NUM_ENTRIES, NUM_ENTRIES*BKW, NUM_ENTRIES*ROW_WIDTH, NUM_ENTRIES*COL_WIDTH, NUM_ENTRIES*NUM_ENTRIES, 8, NUM_ENTRIES, NUM_ENTRIES*4, AGE_WIDTH | Per-entry scheduling image |
| oldest_valid_o, oldest_bank_o, oldest_row_o, oldest_col_o, oldest_id_o, oldest_slot_o | output | 1, BKW, ROW_WIDTH, COL_WIDTH, IW, PTRW | Oldest valid entry |
| issue_valid_i, issue_ready_o, issue_slot_i | in / out / in | 1, 1, PTRW | Issue notify |
| iss_valid_o, iss_ready_i, iss_ticket_o | out / in / out | 1, 1, TW | Issue ticket forward |
| busy_o | output | 1 | CAM busy |

Port block: fub/mc_rd_cmd_cam.sv:48-121.

mc_rd_cmd_cam is the mirror of the write CAM on the read issue side. It stores the read key, ID, QoS, and the ring ticket (fub/mc_rd_cmd_cam.sv:126-133). The entry lives from insert to issue only; the return ring owns the DRAM round trip (fub/mc_rd_cmd_cam.sv:13-18). Until 2026-09-08 this CAM also buffered returned data, which made NUM_ENTRIES the in-flight read limit and capped board read bandwidth at roughly 180 MB/s by Little’s law (fub/mc_rd_cmd_cam.sv:20-23). Free-slot allocation is priority-encoded from the high index down (fub/mc_rd_cmd_cam.sv:148-159). Scheduler lookups return the oldest {bank, row} match (fub/mc_rd_cmd_cam.sv:190-212), and the per-entry scheduling vectors plus age-order matrix follow the same idiom as the write CAM (fub/mc_rd_cmd_cam.sv:214-255). On issue the entry is freed and its ticket is forwarded to the return ring (fub/mc_rd_cmd_cam.sv:165-170).

## 6.6	mc_addr_mapper — flat address to DRAM tuple

| Parameter | Default | Description |
| --- | --- | --- |
| AXI_ADDR_WIDTH | 32 | Flat byte-address width |
| NUM_RANKS | 1 | 1, 2, or 4 |
| NUM_BANKS | 8 | 4 or 8 |
| ROW_WIDTH | 14 | Row width |
| COL_WIDTH | 10 | Column width |
| BYTE_OFFSET_WIDTH | 3 | log2(beat byte size) |
| BG_WIDTH | 1 | Bank-group width (DDR4) |
| HAS_BG | 0 | Enable bank-group field |

Parameter block: fub/mc_addr_mapper.sv:57-67.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_addr_i | input | AXI_ADDR_WIDTH | Flat AXI byte address |
| bank_lsb_i | input | 5 | Bank-field LSB in word address |
| hash_en_i | input | 1 | Enable bank XOR-hash |
| hash_seed_i | input | 8 | XOR-hash seed |
| rank_o | output | $clog2(max(NR,2)) | Decoded rank |
| bank_o | output | $clog2(NUM_BANKS) | Decoded bank |
| bg_o | output | BG_WIDTH | Bank group ('0 when !HAS_BG) |
| row_o | output | ROW_WIDTH | Decoded row |
| col_o | output | COL_WIDTH | Decoded column |

Port block: fub/mc_addr_mapper.sv:69-82.

The mapper strips byte offsets to a 32-bit word address and slides a BW-bit bank field through the column region at bank_lsb_i; the column is split into col_lo below the bank and col_hi above it, while row and rank sit at invariant positions (fub/mc_addr_mapper.sv:96-120). bank_lsb_i is clamped to [0, COL_WIDTH] (fub/mc_addr_mapper.sv:102-103). bank_lsb == COL_WIDTH gives ROW_MAJOR placement, bank_lsb == log2(DRAM_BL) gives maximal bank interleaving, and intermediate values give partial interleave. The RTL comment notes that the safe lower bound was measured, not derived, and that two older formulas were wrong in opposite directions — pumice BUG-022 (fub/mc_addr_mapper.sv:36-41).

When hash_en_i is set, each bank bit is folded with row bits and the seed: bank_hashed[i] = bank_raw[i] ^ row[i] ^ row[MID] ^ hash_seed[i] where MID is clamped to ROW_WIDTH-1 (fub/mc_addr_mapper.sv:127-133). HAS_BG adds the bank-group field above bank for generation-4 geometries while keeping generation-2/3 arithmetic bit-identical (fub/mc_addr_mapper.sv:63-65).

mc_addr_mapper was extracted on 2026-10-10 (fub/mc_addr_mapper.sv:43). It is not instantiated in this plane: mc_wr_intake and mc_rd_intake each instantiate one (fub/mc_wr_intake.sv:322, fub/mc_rd_intake.sv:236), so it sits in mc_axi4_plane, and all three research rocks reach it through that plane (pumice_core, scoria_core and andesite_core each instantiate mc_axi4_plane). It is documented here because it produces the {bank,row,col} key the pools are searched on.

## 6.7	Open questions

- The legacy sched_lu_* and oldest_* ports on both CAMs are tied off inside mc_storage_plane. Will they be removed or reconnected for a future scheduler variant?

## 6.8	References

- Tomasulo, “An Efficient Algorithm for Exploiting Multiple Arithmetic Units,” IBM JRD 1967. PDF (conceptual convergence: CAM scoreboard)

- Jacob, Ng, Wang, Memory Systems: Cache, DRAM, Disk, Morgan Kaufmann 2007. Publisher page (wr2rd snarf/forwarding; citation-only, book)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Chapter 4 — Scheduler Plane and Pick Machinery

This chapter documents the common mc_scheduler_plane and its pick core mc_cmd_arbiter. The surrounding plane-stack and descriptor-seam architecture are defined in the Hardware Architecture Specification (HAS); this chapter does not restate them.

mc_scheduler_plane internal block diagram

## 7.1	1. Top-level parameters

All parameters are declared at macro/mc_scheduler_plane.sv:27-68.

| Parameter | Default | Notes |
| --- | --- | --- |
| NUM_RANKS | 1 | Rank count; drives RKW and NUM_CS. |
| CMD_HISTORY_EN | 0 | Arms the command-history checker child (built only when HAS_CMD_HISTORY is also 1). |
| HIST_T_RCD, HIST_T_RP, HIST_T_RAS, HIST_T_RFC, HIST_T_WTR, HIST_T_RRD, HIST_T_FAW, HIST_T_RTW | varies | History-checker timing parameters. |
| NUM_BANKS | 8 | Banks per rank; drives BKW. |
| NUM_BG | 1 | Bank groups; >1 enables DDR4 L/S windows and cmd_bg_o. |
| ROW_WIDTH | 14 | Row-address width. |
| COL_WIDTH | 10 | Column-address width. |
| AXI_ID_WIDTH | 8 | AXI ID width; drives IW. |
| NUM_ENTRIES | 8 | CAM entries per direction; drives PTRW. |
| AGE_WIDTH | 16 | Age-counter width. |
| CMD_FIFO_DEPTH | 16 | Scheduler-to-DFI command FIFO depth. |
| CMD_DELAY | 6 | Fixed token delay between FIFO push and cmd_valid_o. |
| HAS_ZQ | 1’b0 | Include ZQ calibration branch and tZQCS block. |
| MAINT_EXTRA_WIDTH | 1 | Width of cmd_extra_o / maint_extra_i (pumice=MRR, andesite=MPC[5:0]). |
| HAS_INIT_SEQUENCER | 1’b1 | Build the mc_xddr2_init_sequencer child; 0 takes the init stream from the init_*_i ports. |
| HAS_MODE_REGISTER | 1’b1 | Build the mc_xddr2_mode_register child (drives cl_o/cwl_o/bl_o). |
| HAS_REFRESH_CTRL | 1’b1 | Build the mc_xddr2_refresh_ctrl child; 0 takes refresh from the refresh_*_i ports. |
| HAS_PAGE_POLICY | 1’b1 | Build the mc_page_policy child; 0 takes the AP/timeout-PRE controls from the ap_*_i / timeout_pre_*_i ports and ties the page statistics to 0. |
| HAS_CMD_HISTORY | 1’b0 | Build the mc_cmd_history_checker child (simulation scoreboard). |
| REF_HAS_POSTPONE, REF_HAS_PULLIN, REF_HAS_REFPB, REF_HAS_DRAIN_BURST | 1’b1 | Feature reduction passed to mc_xddr2_refresh_ctrl. |
| N_LU | NUM_BANKS | Legacy lookup-width hint; kept for port compatibility with the pumice-era CAM lookup interface. |
| RKW | derived | (NUM_RANKS>1) ? $clog2(NUM_RANKS) : 1. |
| NUM_CS | NUM_RANKS | Chip-select count for rank-side tracking. |
| CSW | derived | (NUM_CS>1) ? $clog2(NUM_CS) : 1. |
| BKW | derived | $clog2(NUM_BANKS). |
| PTRW | derived | $clog2(NUM_ENTRIES). |
| IW | derived | AXI_ID_WIDTH. |
| BGW_EFF | derived | (NUM_BG>1) ? $clog2(NUM_BG) : 1. |

## 7.2	2. Top-level ports

Ports are declared at macro/mc_scheduler_plane.sv:69-240.

| Group | Signals | Direction | Width / type | Description |
| --- | --- | --- | --- | --- |
| Clock/reset | aclk, aresetn | in | 1 | MC clock and active-low reset. |
| Config | page_policy_i | in | page_policy_e | OPEN / CLOSE / HAPPY_HYBRID selector. |
| Scheduler CSR | sched_order_mode_i, sched_row_sel_i, sched_col_sel_i, sched_access_pref_i | in | 2 each | ORDER mode, row/col select, class preference. |
| Scheduler CSR | sched_wr_high_wm_i, sched_wr_batch_max_i, sched_wr_low_wm_i | in | 8 each | Write-batch watermarks and max. |
| Scheduler CSR | sched_prio_sub_i, sched_qos_en_i | in | 2, 1 | Read-vs-write tie-break and QoS enable. |
| Page-policy CSR | page_mode_i, page_tr_init_i | in | 3, 8 | Consumed by the mc_page_policy child (HAS_PAGE_POLICY=1). |
| Stall telemetry | stall_bp_o, stall_refresh_o, stall_turnaround_o, stall_tccd_o, stall_actlimit_o, stall_banktimer_o, stall_noreq_o, stall_zq_o | out | 32 each | Per-cause stall counters. |
| Page telemetry | stat_page_hit_o, stat_row_hit_o, stat_page_miss_o, stat_page_empty_o, stat_act_o, stat_pre_o, stat_ref_o, stat_ref_busy_o | out | 32 / [NUM_BANKS][32] | Page/policy telemetry. |
| Timing CSR | memtype_i | in | memtype_e | DRAM generation selector. |
| Timing CSR | t_rcd_i, t_rp_i, t_ras_i, t_rc_i, t_wr_i, t_rtp_i, t_faw_i, t_rrd_i, t_wtr_i, t_rtw_i, t_ccd_i | in | 8 each | Core JEDEC timing. |
| Timing CSR | t_ccd_l_i, t_ccd_s_i, t_rrd_l_i, t_rrd_s_i | in | 8 each | DDR4 long/short pair timing. |
| Refresh | refresh_req_i, refresh_drain_i, refresh_kind_i, refresh_bank_i, refresh_grant_o, t_rfc_i, t_rfc_pb_i | in/out | BKW/16/8 | Refresh request/grant/recovery; the request ports are used only when HAS_REFRESH_CTRL=0. |
| ZQ | zq_req_i, zq_grant_o, t_zqcs_i | in/out | 1/1/16 | ZQ calibration (present when HAS_ZQ=1). |
| Maintenance | maint_req_i, maint_grant_o, maint_op_i, maint_bank_i, maint_row_i, maint_col_i, maint_extra_i, maint_busy_i | in/out | varies | Generic maintenance/training slot. |
| Init passthrough | init_done_i, init_cmd_valid_i, init_cmd_op_i, init_cmd_bank_i, init_cmd_row_i | in | varies | External init command stream; used only when HAS_INIT_SEQUENCER=0. |
| Init / MR child | t_init_wait_i, t_dll_wait_i, t_mrd_wait_i, t_rp_wait_i, t_rfc_wait_i, mr0_i..mr3_i, init_restart_i, dfi_init_start_o, dfi_init_complete_i, init_done_o, cl_o, cwl_o, bl_o | in/out | varies | CSRs and outputs of the built-in mc_xddr2_init_sequencer / mc_xddr2_mode_register. |
| Refresh child | t_refi_i, refi_reload_i, refresh_burst_i, ref_postpone_i, ref_pullin_i, ref_mode_i, ref_trefi_pb_i, ref_trfc_pb_i | in | varies | CSRs of the built-in mc_xddr2_refresh_ctrl. |
| Page-policy engine | ap_mode_en_i, ap_close_i, timeout_pre_req_i, timeout_pre_bank_i | in | 1/[NUM_BANKS]/1/[BKW] | External runtime AP and timeout-PRE controls; used only when HAS_PAGE_POLICY=0. |
| Write CAM | wr_sch_valid_i, wr_sch_bank_i, wr_sch_row_i, wr_sch_col_i, wr_sch_older_i, wr_sch_age_exceed_i, wr_sch_qos_i, wr_sch_head_rel_i, wr_commit_ready_i, wr_commit_valid_o, wr_commit_slot_o | in/out | varies | Write schedulable-entry vectors and commit handshake. |
| Read CAM | rd_sch_valid_i, rd_sch_bank_i, rd_sch_row_i, rd_sch_col_i, rd_sch_older_i, rd_sch_age_exceed_i, rd_sch_qos_i, rd_sch_head_rel_i, rd_issue_ready_i, rd_issue_valid_o, rd_issue_slot_o | in/out | varies | Read schedulable-entry vectors and issue handshake. |
| Command output | cmd_valid_o, cmd_ready_i, cmd_op_o, cmd_rank_o, cmd_bank_o, cmd_bg_o, cmd_row_o, cmd_col_o, cmd_ap_o, cmd_extra_o | out/in | varies | DFI command descriptor. |
| Bank state | bank_row_active_o, bank_open_row_o | out | [NUM_RANKS][NUM_BANKS], [NUM_RANKS][NUM_BANKS][ROW_WIDTH-1:0] | Registered bank-state image. |
| Status | busy_o | out | 1 | !init_done \¦\¦ refresh_req \¦\¦ cmd_fifo_valid \¦\¦ any_bank_active (internal init/refresh when the children are built). |
| Pre-FIFO accept | cmd_accept_o, cmd_accept_op_o | out | 1, dram_op_e | Same-cycle accept pulse and opcode. |

## 7.3	3. Internal structure

The plane instantiates the blocks at macro/mc_scheduler_plane.sv:288-772. The first four rows are the DDR2/LPDDR2-generation (“v2”) children, each behind its HAS_* selector; with the selector at 0 the plane takes that function from its external ports instead. Until 2026-10-11 these children lived in pumice’s pumice_scheduler_plane wrapper; they are now common-ip FUBs composed here, and pumice instantiates this plane directly with the defaults.

| Instance | Module | Built when | Key parameters | Role |
| --- | --- | --- | --- | --- |
| u_init | mc_xddr2_init_sequencer | HAS_INIT_SEQUENCER | NUM_BANKS, ROW_WIDTH | DDR2/LPDDR2 JEDEC init; drives dfi_init_start_o and the init command stream; produces init_done. |
| u_mode_reg | mc_xddr2_mode_register | HAS_MODE_REGISTER | NUM_RANKS | MR shadow updated by init writes; decodes cl_o / cwl_o / bl_o. |
| u_refresh | mc_xddr2_refresh_ctrl | HAS_REFRESH_CTRL | NUM_BANKS, REF_HAS_* | tREFI accumulator, postpone/pull-in, REFab/REFpb selection; enabled after init_done. |
| u_page_policy | mc_page_policy | HAS_PAGE_POLICY | NUM_RANKS, NUM_BANKS, ROW_WIDTH | Runtime page-policy engine and page telemetry. |
| u_bank_timers | mc_bank_timers | always | NUM_RANKS, NUM_BANKS, ROW_WIDTH, BANK_LA=4 | Per-bank safe timers and open-row state. |
| u_global_timers | mc_global_timers | always | NUM_BG, HAS_LS_PAIRS=(NUM_BG>1) | Rank/global windows (tFAW, tRRD, tWTR, tRTW, tCCD, L/S pairs). |
| u_arbiter | mc_cmd_arbiter | always | NUM_BG, HAS_ZQ, MAINT_EXTRA_WIDTH | Single-issue pick core. |
| u_cmd_fifo | gaxi_fifo_sync | always | DATA_WIDTH=CMD_W, DEPTH=CMD_FIFO_DEPTH | Scheduler-to-DFI command FIFO. |
| u_cmd_history | mc_cmd_history_checker | HAS_CMD_HISTORY && CMD_HISTORY_EN != 0 | NUM_RANKS, NUM_BANKS, HIST_T_* | Simulation-only JEDEC sequencing scoreboard. |

busy_o is !init_done || refresh_req || w_cmd_rd_valid || (|w_bank_row_active[0]) (macro/mc_scheduler_plane.sv:744-745).

## 7.4	4. Schedulable-entry view and unified prospect pool

The arbiter folds the two CAMs into one POOL = 2*NUM_ENTRIES vector: upper half writes, lower half reads (fub/mc_cmd_arbiter.sv:491-501). Each entry carries {valid, bank, row, col} and is classified as column, activate, or precharge against the registered bank image (fub/mc_cmd_arbiter.sv:537-589). Column eligibility further requires row hit, per-bank bank_rdwr_ready, the forward tCCD guard, safe turnaround, sink-FIFO room, and clear in-flight guards. This unified-pool refactor (adopted from pumice Task 4.5) is bit-identical to the earlier split loops (../../mc_scheduler_knobs.md §4.1).

## 7.5	5. Generic maintenance slot

Rock-specific training/init/ZQ requesters collapse to a generic maintenance slot: {maint_req_i, maint_grant_o, maint_op_i, maint_bank_i, maint_row_i, maint_col_i, maint_extra_i, maint_busy_i} (macro/mc_scheduler_plane.sv:185-192). maint_extra_i is rock-defined (pumice=MRR, andesite=MPC[5:0]) (fub/mc_cmd_arbiter.sv:45; ../../mc_scheduler_knobs.md §6.3). ZQ keeps its own zq_req_i/zq_grant_o/t_zqcs_i because the tZQCS window must block every other command (fub/mc_cmd_arbiter.sv:96-101).

## 7.6	6. Bank and global timer parameterization

mc_bank_timers stamps one mc_bank_timer per (rank,bank) with advisory lookahead BANK_LA=4 (fub/mc_bank_timers.sv:29-35; fub/mc_bank_timer.sv:41-44). Each timer is FSM-free — preset/decrement counters plus a row-open register — so a just-issued command is reflected one cycle later (fub/mc_bank_timer.sv:15-17).

mc_global_timers exposes DDR4 L/S windows via NUM_BG, BGW, and HAS_LS_PAIRS (fub/mc_global_timers.sv:33-44). The plane derives HAS_LS_PAIRS = (NUM_BG > 1) and takes the fired command’s bank group from the upper bits of evt_bank (macro/mc_scheduler_plane.sv:517-519). When HAS_LS_PAIRS=0 the L/S readiness outputs tie to 1 (fub/mc_global_timers.sv:341-346).

## 7.7	7. Output command descriptor

The FIFO write data is packed {ap, col, row, bg, bank, rank, extra, op} (macro/mc_scheduler_plane.sv:697) and unpacked in the same order at the output (macro/mc_scheduler_plane.sv:738). The extra field carries the maintenance value for generic maintenance commands; otherwise it is zero. This is the scheduler-side seam; DFI-generation packing is described in DFI Boundary Lineage.

## 7.8	8. Fire-stage authority

The output stage is the sole timing authority. A registered pick fires only when the live per-bank and global readiness agree:

ACT:  bank_act_ready_i && tfaw_ok_i && trrd_ok_i && w_trrd_ls_ok && !w_zq_busy && !maint_busy_i
RD/WR: bank_rdwr_ready_i && w_tccd_ls_ok && !w_zq_busy && !maint_busy_i
PRE:  bank_pre_ready_i && !w_zq_busy && !maint_busy_i

These terms are at fub/mc_cmd_arbiter.sv:294-306. Upstream classify and STAGE-1b masks are advisory.

A pick that fails the live check is normally dropped, not held, because CAM commit/issue are qualified by w_fire_out and the entry stays schedulable (fub/mc_cmd_arbiter.sv:252-257). The single exception is an ACT blocked only by the rank-global tFAW/tRRD windows: w_out_hold keeps that ACT in the output register (fub/mc_cmd_arbiter.sv:307-311). This exception is measured history:

- pumice BUG-021: dropping such an ACT regressed close-page throughput from the accepted 30.77% calibration to 18.57%; holding recovered to 30.14% (fub/mc_cmd_arbiter.sv:259-265; ../../mc_scheduler_knobs.md §6.4).

- scoria BUG-001: 3/3 seeds failed with a global tRRD violation (“ACT only 1 cyc after another ACT”), leading to the tfaw_ok_i/trrd_ok_i fire-stage re-check (fub/mc_cmd_arbiter.sv:267-268; ../../mc_scheduler_knobs.md §6.4).

- ISSUE-002: moving tFAW/tRRD to the classify mask drained the 3-stage pipeline and dropped close-page throughput from 30.77% to ~18% (fub/mc_cmd_arbiter.sv:513-518; ../../mc_scheduler_knobs.md §6.4).

Other proven guard terms:

- ONE-CYCLE BLIND SPOT: turnaround history registers load the cycle after a fire, so a command picked in the same cycle another fires sees stale history. The fix adds an in-flight fire term to the live turnaround equations (fub/mc_cmd_arbiter.sv:849-858; fub/mc_global_timers.sv:100-124).

- Parked-victim PRE guard: a PRE leaves the registered row image stale for up to three cycles; a column picked against it lands on the closed row and wedges the AR-order drain. The column mask includes !w_pre_col_guard (fub/mc_cmd_arbiter.sv:441-453).

- BUG-002 / tZQCS fire-stage block: the tZQCS counter loads on accepted fire, so a classify-time check is one cycle early. Every demand-class fire gate includes !w_zq_busy (fub/mc_cmd_arbiter.sv:278-284). Refresh is placed above ZQ in the priority cone because inside tZQCS the refresh branch cannot run; w_ref_safe carries !w_rfc_busy, while refresh is kept out of the ZQ window by cone priority, and the code warns not to “tidy” this into symmetry (fub/mc_cmd_arbiter.sv:1104-1116, 1378-1386).

## 7.9	9. Page-policy context

The auto-precharge decision is w_ap = (page_policy_i == PAGE_POLICY_CLOSE) unless the runtime engine is enabled, in which case it is per-bank via ap_close_i[b] (fub/mc_cmd_arbiter.sv:197-202). PAGE_POLICY_HAPPY_HYBRID was retired and maps to OPEN in the current build (pumice scheduler MAS, lines 200-201). With HAS_PAGE_POLICY=1 the built-in mc_page_policy child supplies the AP-mode enable, per-bank close, and timeout-PRE controls; with it at 0 they come from ap_mode_en_i, ap_close_i, timeout_pre_req_i, and timeout_pre_bank_i.

## 7.10	10. P1 init accept/ack gating

For DDR4 P1 init the product’s scheduler wrapper (andesite) must ack its own init sequencer in the same cycle the arbiter accepts the command. Because the arbiter’s registered pick re-arms every cycle, a held init request would be picked twice, producing MRS twins one cycle apart. The wrapper gates init_cmd_valid_i with a combinational accept pulse from cmd_accept_o/cmd_accept_op_o (macro/mc_scheduler_plane.sv:199-200; ../../../../research-ip/andesite-ddr4-lpddr4/rtl/macro/andesite_scheduler_plane.sv:255-269; ../../mc_scheduler_knobs.md §6.4).

## 7.11	11. Open questions

- The exact encodings of page_mode_i and page_tr_init_i belong to the mc_page_policy child, not to the plane’s own logic; this chapter treats them as opaque forwarded CSRs.

- Multi-rank arbitration is fixed at RK0 = 0 in mc_cmd_arbiter.sv:194; whether the same fire-stage authority scales to NUM_RANKS > 1 is not exercised by the present core.

## 7.12	References

- Rixner, Dally, Kapasi, Mattson, Owens, “Memory Access Scheduling,” ISCA 2000. PDF

- Mutlu, Moscibroda, “Stall-Time Fair Memory Access Scheduling for Chip Multiprocessors,” MICRO 2007. PDF

- Tomasulo, “An Efficient Algorithm for Exploiting Multiple Arithmetic Units,” IBM JRD 1967. PDF (conceptual convergence: schedulable-entry view)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	mc_training_plane — LPDDR2 training clients on the generic shell

Modules: mc_training_plane.sv, mc_training_shell.sv
Location: common-ip/rtl/macro/
Category: Plane-4 training plane (maintenance-traffic client)
Status: implemented; instantiated directly by pumice_core (both clients enabled)

The plane-stack role of this block — where it sits between the scheduler and the PHY calibration sideband — is defined in the family architecture book (common_has). This chapter describes the plane’s composition, the generic shell inside it, and how a product configures it.

mc_training_plane block diagram

## 8.1	Composition

mc_training_plane is two layers (macro/mc_training_plane.sv:7-15):

- The LPDDR2 training clients, each behind a build parameter: mc_lpddr2_zq_ctrl (u_zq, slot 0, HAS_LP_ZQ) and mc_lpddr2_dq_cal (u_lp_cal, slot 1, HAS_LP_DQ_CAL) (macro/mc_training_plane.sv:112-195). With a parameter at 0 the client is not elaborated, its slot request is tied low, and its status outputs read 0.

- mc_training_shell (u_shell), the generic, dependency-inverted part: a fixed-priority maintenance-slot bus, a single generic trn_cmd output, and the mc_clk/dfi_clk calibration-data crossing (macro/mc_training_shell.sv:7-16). Its slot_* ports are internal to the plane; the plane drives them from the clients (macro/mc_training_plane.sv:200-225).

The plane, not a product wrapper, owns the slot mapping, the command shapes, and the sideband wiring. A product chooses only parameters. Further training children (CA training, write leveling) would be added as more slots on the shell.

### 8.1.1	Slot mapping

| Slot | Client | Priority | Command shape | slot_extra (MRR flag) |
| --- | --- | --- | --- | --- |
| 0 | mc_lpddr2_zq_ctrl | highest | MRW to MR10 (OP_MRS, row {4'd0, MR10, OP}, OP 0x56 ZQCS / 0xAB ZQCL) | 1'b0 |
| 1 | mc_lpddr2_dq_cal | lowest | MRR to MR32 then MR40 (OP_MRS, row from mrr_row(MA)) | 1'b1 |

ZQ is periodic maintenance and outranks DQ calibration, which is a firmware-initiated one-shot. For NUM_SLOTS == 2 the mapping is fixed; for other values the plane assigns ZQ to slot 0 and DQ calibration to slot 1, sets slot_extra[s] = s[0], and leaves any further slots idle (macro/mc_training_plane.sv:212-223). The shell’s trn_cmd_extra_o[0] leaves the plane as trn_cmd_mrr_o (macro/mc_training_plane.sv:265).

### 8.1.2	Maintenance-slot arbitration (shell)

The highest-priority requesting slot drives the generic trn_cmd_* output to the scheduler; lower-priority slots wait. Priority is fixed, not round-robin: slot 0 > slot 1 > … > slot NUM_SLOTS-1 (macro/mc_training_shell.sv:64-74). Grant is returned only to the selected slot: slot_grant_o[g] = slot_req_i[g] && trn_cmd_grant_i && (w_sel == g) (macro/mc_training_shell.sv:80-85).

### 8.1.3	One-active trn_cmd mux (shell)

Only one slot’s payload reaches the scheduler at a time. The output command is assembled directly from the selected slot’s slot_op_i, slot_bank_i, slot_row_i, and slot_extra_i (macro/mc_training_shell.sv:75-78). The shell never re-encodes the command; the clients produce the opcode, bank, row, and MRR flag.

### 8.1.4	mc_clk/dfi_clk CDC placement (shell)

All calibration-sideband crossing for the plane lives inside the shell, using the repo’s standard cells:

- cal_expect (mc_clk domain, from mc_lpddr2_dq_cal) crosses down to cal_expect_o (dfi_clk domain) through sync_pulse with three stages (macro/mc_training_shell.sv:90-97).

- cal_data_i (dfi_clk domain) crosses up to the mc_clk domain through cdc_synchronizer with WIDTH = CAL_DATA_WIDTH and three flops (macro/mc_training_shell.sv:99-104).

- cal_valid_i (dfi_clk domain) crosses up to the mc_clk domain through sync_pulse with three stages (macro/mc_training_shell.sv:106-113).

The synchronized data and valid feed mc_lpddr2_dq_cal directly, so both clients run entirely in the mc_clk domain.

### 8.1.5	Status aggregation

cal_busy_o = zq_busy_o || dq_cal busy; cal_done_o, cal_err_o, mrr32_data_o and mrr40_data_o come from mc_lpddr2_dq_cal; zq_busy_o, zq_overdue_o and zqcs_total_o from mc_lpddr2_zq_ctrl (macro/mc_training_plane.sv:270-274).

## 8.2	Boundary doctrine: training searches stay PHY-side

The common training plane owns the JEDEC maintenance-command sequencing and the calibration-control CSR plane. The actual delay-tap, bitslip, and phase-search mechanisms stay on the PHY side of the documented contract (../../phy_cal_csr_contract.md). In other words, the core issues ZQ (MRW to MR10) and MRR commands and captures returned beats; firmware plus the PHY walk the taps and compare patterns.

## 8.3	Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_SLOTS | int | 2 | Number of fixed-priority maintenance slots on the shell. (macro/mc_training_plane.sv:27) |
| BKW | int | 3 | Bank field width for the slots and trn_cmd_bank_o. (macro/mc_training_plane.sv:28) |
| ROW_WIDTH | int | 14 | Row field width for the slots and trn_cmd_row_o. (macro/mc_training_plane.sv:29) |
| EXTRA_WIDTH | int | 1 | Side-band width per slot; bit 0 is the MRR flag. (macro/mc_training_plane.sv:30) |
| CAL_DATA_WIDTH | int | 1 | Width of cal_data_i and the captured MRR data; set to the DFI read-data width. (macro/mc_training_plane.sv:31) |
| HAS_LP_ZQ | bit | 1 | Build mc_lpddr2_zq_ctrl on slot 0. (macro/mc_training_plane.sv:32) |
| HAS_LP_DQ_CAL | bit | 1 | Build mc_lpddr2_dq_cal on slot 1. (macro/mc_training_plane.sv:33) |

## 8.4	Ports

### 8.4.1	Clock and reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk | in | 1 | Controller clock. (macro/mc_training_plane.sv:35) |
| mc_rst_n | in | 1 | Active-low reset. (macro/mc_training_plane.sv:36) |

### 8.4.2	Runtime enable / build guard

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| init_done_i | in | 1 | Training FUBs run only after DRAM init completes. (macro/mc_training_plane.sv:39) |
| memtype_i | in | memtype_e | DRAM type (DDR2 / LPDDR2). (macro/mc_training_plane.sv:40) |

### 8.4.3	CSR controls

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| zq_en_i | in | 1 | Enable periodic LPDDR2 ZQ calibration. (macro/mc_training_plane.sv:43) |
| zq_defer_en_i | in | 1 | Enable demand-deferred ZQ placement. (macro/mc_training_plane.sv:44) |
| zq_interval_i | in | 32 | ZQCS interval in controller clocks. (macro/mc_training_plane.sv:45) |
| t_zqcs_i | in | 16 | ZQCS command post-grant hold time. (macro/mc_training_plane.sv:46) |
| t_zqcl_i | in | 16 | ZQCL command post-grant hold time. (macro/mc_training_plane.sv:47) |
| zq_overdue_max_i | in | 13 | Overdue-escalation threshold to ZQCL. (macro/mc_training_plane.sv:48) |
| cal_start_i | in | 1 | One-shot start for LPDDR2 DQ calibration. (macro/mc_training_plane.sv:50) |
| cal_abort_i | in | 1 | Abort in-flight DQ calibration. (macro/mc_training_plane.sv:51) |
| t_mrr_i | in | 16 | MRR command interval. (macro/mc_training_plane.sv:52) |
| t_readout_i | in | 16 | Readout capture window. (macro/mc_training_plane.sv:53) |

### 8.4.4	Maintenance command channel to the scheduler

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| trn_cmd_req_o | out | 1 | Aggregate maintenance request. (macro/mc_training_plane.sv:56) |
| trn_cmd_grant_i | in | 1 | Arbiter grant. (macro/mc_training_plane.sv:57) |
| trn_cmd_op_o | out | dram_op_e | Opcode of the selected slot. (macro/mc_training_plane.sv:58) |
| trn_cmd_bank_o | out | BKW | Bank of the selected slot. (macro/mc_training_plane.sv:59) |
| trn_cmd_row_o | out | ROW_WIDTH | Row of the selected slot. (macro/mc_training_plane.sv:60) |
| trn_cmd_mrr_o | out | 1 | MRR side-band flag to the scheduler. (macro/mc_training_plane.sv:61) |

### 8.4.5	DFI-plane calibration sideband

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_clk | in | 1 | DFI/PHY clock. (macro/mc_training_plane.sv:64) |
| dfi_rstn | in | 1 | Active-low DFI reset. (macro/mc_training_plane.sv:65) |
| cal_expect_o | out | 1 | dfi_clk-domain arm pulse to the PHY/read aligner. (macro/mc_training_plane.sv:66) |
| cal_data_i | in | CAL_DATA_WIDTH | dfi_clk-domain captured beat. (macro/mc_training_plane.sv:67) |
| cal_valid_i | in | 1 | dfi_clk-domain “data landed” pulse. (macro/mc_training_plane.sv:68) |

### 8.4.6	Telemetry / status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cal_busy_o | out | 1 | Either training client is active. (macro/mc_training_plane.sv:71) |
| cal_done_o | out | 1 | DQ calibration completed (sticky). (macro/mc_training_plane.sv:72) |
| cal_err_o | out | 1 | DQ calibration error (sticky). (macro/mc_training_plane.sv:73) |
| mrr32_data_o | out | CAL_DATA_WIDTH | Captured MR32 read data. (macro/mc_training_plane.sv:74) |
| mrr40_data_o | out | CAL_DATA_WIDTH | Captured MR40 read data. (macro/mc_training_plane.sv:75) |
| zq_busy_o | out | 1 | ZQ calibration is active. (macro/mc_training_plane.sv:76) |
| zq_overdue_o | out | 1 | ZQ is overdue. (macro/mc_training_plane.sv:77) |
| zqcs_total_o | out | 16 | Count of completed ZQCS operations. (macro/mc_training_plane.sv:78) |

## 8.5	Product integration

A product instantiates mc_training_plane directly and:

- Chooses its clients with HAS_LP_ZQ / HAS_LP_DQ_CAL.

- Sets BKW, ROW_WIDTH, EXTRA_WIDTH to match its scheduler’s maintenance slot, and CAL_DATA_WIDTH to its DFI read-data width.

- Connects trn_cmd_* to the scheduler’s generic maintenance slot (maint_req_i / maint_grant_o / maint_op_i / maint_bank_i / maint_row_i, with trn_cmd_mrr_o on maint_extra_i[0] and cal_busy_o on maint_busy_i).

- Connects cal_expect_o / cal_data_i / cal_valid_i to the DFI plane’s read-aligner calibration sideband, and the CSR controls and status to its register block.

Run gating (init_done, memtype == LPDDR2) and JEDEC command packing live in the client FUBs, not in the shell, so a DDR2 run leaves the plane inert without a rebuild.

### 8.5.1	Pumice

pumice_core instantiates the plane as u_train_plane with NUM_SLOTS = 2, EXTRA_WIDTH = 1, CAL_DATA_WIDTH = DFI_DATA_WIDTH, and both clients enabled (research-ip/pumice-ddr2-lpddr2/rtl/top/pumice_core.sv:756-797). Until 2026-10-11 pumice reached the shell through its own pumice_training_plane wrapper around pumice-local pumice_zq_ctrl / pumice_lp_cal; those FUBs became mc_lpddr2_zq_ctrl / mc_lpddr2_dq_cal, the composition moved into this plane, and the wrapper was removed. The clients’ behaviour and measured history are still documented in the pumice MAS (ZQ control, DQ calibration) and the pumice HAS Training chapter.

## 8.6	ZQ placement as a maintenance client

ZQ calibration is a maintenance-class request: it raises slot 0, waits for its grant, and never preempts an in-flight demand command. The plane preserves the BUG-003 discipline: the scheduler holds demand-class traffic during the post-grant hold (cal_busy_o onto maint_busy_i, folded into the fire stage) so that w_fire_out == cmd_valid_o && cmd_ready_i holds by construction, and the training plane never touches the pick cone (pumice MAS 23_zq_ctrl).

mc_lpddr2_zq_ctrl.demand_i is tied to 1'b0 inside the plane (macro/mc_training_plane.sv:125). This means the Mode-C deferral entry condition (zq_defer_en_i && demand_i) can never fire, the overdue/ZQCL escalation path is dormant, and zq_overdue_o never sets. The FSM keeps the policy; it needs only one wire to make it live.

## 8.7	Plane-only versus pumice-proven

| Capability | Provenance |
| --- | --- |
| 2-slot fixed-priority arbitration (ZQ > DQ cal) | pumice-proven |
| Generic trn_cmd_* output mapped to the scheduler maintenance slot | pumice-proven |
| sync_pulse + cdc_synchronizer calibration sideband crossing | pumice-proven at CAL_DATA_WIDTH = 128 |
| HAS_LP_ZQ = 0 / HAS_LP_DQ_CAL = 0 builds | plane-only; no product builds them |
| NUM_SLOTS > 2 | plane-only; extra slots stay idle |
| EXTRA_WIDTH > 1 or multi-bit side-band semantics | plane-only |
| Re-arbitration/round-robin policy | not implemented |
| Multi-rank ZQ overlap policy | not implemented; pumice is single-rank |
| cal_expect from more than one client | not implemented; only mc_lpddr2_dq_cal drives it |

## 8.8	Open questions

- Demand wiring for Mode-C ZQ placement. Should the plane expose a demand_i port so the scheduler’s demand indicator reaches mc_lpddr2_zq_ctrl instead of the internal tie-off?

- Slot-width defaults. The clients produce 3-bit bank and 18-bit row fields whose top four row bits are zero, so ROW_WIDTH must be at least 14 to carry {MA[5:0], OP[7:0]}; the defaults (BKW = 3, ROW_WIDTH = 14) are exactly that floor. Is the default EXTRA_WIDTH = 1 still useful once MRR flags graduate to a dedicated scheduler field?

- Multi-client cal_expect. If the plane ever composes more than one calibration child that needs cal_expect, the shell’s single cal_expect_i must be muxed or widened. No design exists yet.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Chapter 6 — mc_ras_plane

mc_ras_plane is the Reliability/Availability/Serviceability transform plane on the two memory-data seams: write-data forward and read-return backward (macro/mc_ras_plane.sv:8-9). The common-ip extraction intentionally leaves it a skeleton; the only binding decisions are the module boundary, the transform-pair placement, the HAS_ECC parameter intent, and a reserved CSR surface. The architecture context is in the family HAS (common_has); this chapter does not restate it.

The plane is not yet instantiated into any research controller (mc_ras_plane_knobs.md).

mc_ras_plane

## 9.1	Fixed decisions

- Module boundary. Two AXI-like streaming seams, one in each direction, plus a four-signal CSR surface and a busy_o indicator (macro/mc_ras_plane.sv:27-69).

- Transform-pair placement. ECC insertion belongs on the write-data forward seam; ECC check belongs on the read-return backward seam. The pair wraps either the storage plane or a DFI plane, depending on where the code word is carried. This placement is the only RAS structure with two customers today (common_has doctrine 9).

- HAS_ECC intent. 0 = ECC_OFF (default and only implemented value), 1 = ECC_DETECT reserved, 2 = ECC_CORRECT reserved (macro/mc_ras_plane.sv:22-23).

- Reserved CSR surface. csr_ecc_en_i, csr_ecc_mode_i, and csr_ecc_stat_o exist at the boundary; when HAS_ECC = ECC_OFF the status port reads zero (macro/mc_ras_plane.sv:62-66, 86). The full field layout is recorded in mc_ras_plane_knobs.md.

- Scrubber relationship. When added, the scrubber will be a maintenance-class client of the scheduler/prospect pool, peer to refresh and training, injecting read descriptors that reuse existing read-snoop hazard rules (mc_ras_plane_knobs.md).

## 9.2	Parameters

| Parameter | Default | Description | RTL |
| --- | --- | --- | --- |
| DATA_WIDTH | 64 | Data beat width on both seams | macro/mc_ras_plane.sv:20 |
| STRB_WIDTH | DATA_WIDTH/8 | Byte-strobe width on the write seam | macro/mc_ras_plane.sv:21 |
| HAS_ECC | 0 | 0=OFF, 1=DETECT (reserved), 2=CORRECT (reserved) | macro/mc_ras_plane.sv:23 |
| ECC_MODE | 0 | Reserved knob for in-band vs. side-band code placement | macro/mc_ras_plane.sv:25 |
| ECC_CODE_WIDTH | 8 | Reserved knob for ECC code-word width | macro/mc_ras_plane.sv:26 |

## 9.3	Ports

### 9.3.1	Write-data forward seam (upstream → downstream)

| Port | Direction | Width | Description | RTL |
| --- | --- | --- | --- | --- |
| wdata_valid_i | input | 1 | Upstream write-data valid | macro/mc_ras_plane.sv:34 |
| wdata_ready_o | output | 1 | Upstream write-data ready | macro/mc_ras_plane.sv:35 |
| wdata_data_i | input | DATA_WIDTH | Upstream write data | macro/mc_ras_plane.sv:36 |
| wdata_strb_i | input | STRB_WIDTH | Upstream write strobe | macro/mc_ras_plane.sv:37 |
| wdata_last_i | input | 1 | Upstream write last beat | macro/mc_ras_plane.sv:38 |
| wdata_valid_o | output | 1 | Downstream write-data valid | macro/mc_ras_plane.sv:40 |
| wdata_ready_i | input | 1 | Downstream write-data ready | macro/mc_ras_plane.sv:41 |
| wdata_data_o | output | DATA_WIDTH | Downstream write data | macro/mc_ras_plane.sv:42 |
| wdata_strb_o | output | STRB_WIDTH | Downstream write strobe | macro/mc_ras_plane.sv:43 |
| wdata_last_o | output | 1 | Downstream write last beat | macro/mc_ras_plane.sv:44 |

### 9.3.2	Read-return backward seam (downstream → upstream)

| Port | Direction | Width | Description | RTL |
| --- | --- | --- | --- | --- |
| rdata_valid_i | input | 1 | Downstream read-return valid | macro/mc_ras_plane.sv:49 |
| rdata_ready_o | output | 1 | Downstream read-return ready | macro/mc_ras_plane.sv:50 |
| rdata_data_i | input | DATA_WIDTH | Downstream read data | macro/mc_ras_plane.sv:51 |
| rdata_resp_i | input | 2 | Downstream read response | macro/mc_ras_plane.sv:52 |
| rdata_last_i | input | 1 | Downstream read last beat | macro/mc_ras_plane.sv:53 |
| rdata_valid_o | output | 1 | Upstream read-return valid | macro/mc_ras_plane.sv:55 |
| rdata_ready_i | input | 1 | Upstream read-return ready | macro/mc_ras_plane.sv:56 |
| rdata_data_o | output | DATA_WIDTH | Upstream read data | macro/mc_ras_plane.sv:57 |
| rdata_resp_o | output | 2 | Upstream read response | macro/mc_ras_plane.sv:58 |
| rdata_last_o | output | 1 | Upstream read last beat | macro/mc_ras_plane.sv:59 |

### 9.3.3	Reserved CSR surface

| Port | Direction | Width | Description | RTL |
| --- | --- | --- | --- | --- |
| csr_ecc_en_i | input | 1 | Master ECC enable; no effect when HAS_ECC = OFF | macro/mc_ras_plane.sv:64 |
| csr_ecc_mode_i | input | 2 | ECC operating mode; reserved | macro/mc_ras_plane.sv:65 |
| csr_ecc_stat_o | output | 32 | ECC status; reads zero when HAS_ECC = OFF | macro/mc_ras_plane.sv:66 |
| busy_o | output | 1 | Plane busy indicator; tied low | macro/mc_ras_plane.sv:68 |

## 9.4	Bypass property

A build that does not need RAS features omits this plane entirely; absence is a true bypass, not a stub. When the plane is present, both gen_ecc_off and gen_ecc_reserved reduce to bit-identical passthrough assigns (macro/mc_ras_plane.sv:71-105). The RTL comment notes that the HAS_ECC = 0 branch contains “no combinational logic beyond the assigns themselves” (macro/mc_ras_plane.sv:72-73).

## 9.5	Explicitly not decided

Per doctrine 9, any RAS feature beyond the transform pair (ECC insert/check) plus scrub-as-maintenance-client is out of scope until a real enabling customer appears (common_has doctrine 9). In particular:

- The ECC encoder, decoder, and syndrome logic are not implemented (macro/mc_ras_plane.sv:88-90).

- ECC_MODE and ECC_CODE_WIDTH are documented reserved knobs but have no RTL effect (macro/mc_ras_plane.sv:24-26).

- The scrubber has no implementation; its scheduler interface is deferred to the scheduler-merge task (mc_ras_plane_knobs.md).

## 9.6	Open questions

- Which side of the storage/DFI boundary hosts the ECC code word? The transform-pair placement is fixed, but the partner has not selected the code-word carrier.

- What error-response encoding appears on rdata_resp_o when HAS_ECC = DETECT/CORRECT? The rdata_resp width is two bits (macro/mc_ras_plane.sv:52), but no encoding is assigned.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	mc_dfi_2p1_plane — DFI 2.1 Command / Write / Read Pipeline

Module: mc_dfi_2p1_plane.sv · Location: common-ip/rtl/macro/

Architectural context is in the family HAS (common_has); this chapter documents the block microarchitecture only.

mc_dfi_2p1_plane internal block diagram

## 10.1	Purpose

mc_dfi_2p1_plane is the common DFI 2.1 PHY-facing plane. It presents controller-clock command, write-data, and read-data streams on one side and the DFI 2.1 pin bus on the other, and owns the single controller-to-PHY clock-domain crossing plus the DFI-clock command path, write serializer, and read aligner (macro/mc_dfi_2p1_plane.sv:8-12, macro/mc_dfi_2p1_plane.sv:32). Its datapath unit is the DFI word; one FIFO word equals one DFI cycle, so the path is bubble-free at rate (macro/mc_dfi_2p1_plane.sv:24-25).

## 10.2	Sub-FUBs and clock domains

| Instance | Domain | Role | File |
| --- | --- | --- | --- |
| u_cdc | ctl_clk ↔ dfi_clk | The single family CDC: async gaxi_fifo_async FIFOs for cmd, wrdata, rddata; token crossings for init_start/init_complete | fub/mc_dfi_cdc.sv:30 |
| u_cmd | dfi_clk | Command FIFO → multi-phase DFI command bus; emits wr_fire/rd_fire strobes | fub/mc_dfi_cmd_path.sv:47 |
| u_wr | dfi_clk | Write-data FIFO → dfi_wrdata/dfi_wrdata_en/dfi_wrdata_mask at t_phy_wrlat | fub/mc_dfi_wr_serializer.sv:29 |
| u_rd | dfi_clk | dfi_rddata capture → read-return FIFO at t_rddata_en | fub/mc_dfi_rd_aligner.sv:38 |

## 10.3	The single controller↔PHY clock-domain crossing

mc_dfi_cdc is the only place the controller and PHY clock domains meet (fub/mc_dfi_cdc.sv:8-12). It uses gaxi_fifo_async for every crossing — cmd, wrdata, rddata, plus one-bit token FIFOs for init_start and init_complete — and prohibits hand-rolled synchronizers or open-loop bit crossings (fub/mc_dfi_cdc.sv:11-12).

The level signals init_start_i and dfi_init_complete_i cross as event tokens: a rising edge on the source pushes a 1-bit token; the sink pops it into a sticky latch (fub/mc_dfi_cdc.sv:20-22, fub/mc_dfi_cdc.sv:188-248). The write path also pushes a pwr_staged token per complete write burst so the command path cannot issue a WR ahead of its data (fub/mc_dfi_cdc.sv:86-93).

## 10.4	DFI_RATE gearing and bus geometry

DFI_RATE is the compile-time maximum number of DRAM beats per DFI word (phase count). The plane derives the DFI bus widths from it (macro/mc_dfi_2p1_plane.sv:70-82):

| Derived width | Formula |
| --- | --- |
| DFI_DATA_WIDTH | DRAM_BEAT_WIDTH * DFI_RATE |
| DFI_STRB_WIDTH | DFI_DATA_WIDTH / 8 |
| DFI_EN_WIDTH | DFI_RATE |
| DFI_VALID_WIDTH | DFI_RATE |
| DFI_ADDR_BUS_W | ROW_WIDTH * DFI_RATE |
| DFI_BANK_BUS_W | $clog2(NUM_BANKS) * DFI_RATE |
| DFI_CTRL_BUS_W | 1 * DFI_RATE |
| DFI_CS_BUS_W | NUM_RANKS * DFI_RATE |

At runtime gear_i selects the active rate within the compile-time maximum: active_rate = 1 << gear_i (macro/mc_dfi_2p1_plane.sv:143-148). The plane builds a per-phase active mask w_phase_active and gates dfi_wrdata_en_o/dfi_rddata_en_o to the active phases (macro/mc_dfi_2p1_plane.sv:202-211). At gear_i = $clog2(DFI_RATE) the mask is all-ones and the outputs are bit-identical to the unmasked FUB outputs.

## 10.5	Command path

mc_dfi_cmd_path pops the abstract command stream from the CDC FIFO, unpacks {mrr, ap, col, row, bank, rank, op}, and drives the multi-phase DFI command bus (fub/mc_dfi_cmd_path.sv:8-10). It places READ on rd_phase_i and WRITE on wr_phase_i; everything else lands on phase 0 (fub/mc_dfi_cmd_path.sv:231-238).

The path performs no JEDEC timing enforcement: all delays come from the scheduler. The only remaining holds are structural — a read may not issue unless the aligner has a free slot, and a write may not issue before its data is staged (fub/mc_dfi_cmd_path.sv:175-214). This policy followed pumice BUG-017 (was PUMICE-042): holding columns for tRTW/tWTR here compressed REF→ACT spacing in the in-order FIFO and caused 180 consecutive reads to return an undriven DQ bus (fub/mc_dfi_cmd_path.sv:181-190; see pumice MAS 15_gear_dfi.md).

For sub-DFI-word packing, N_SUBCMD sub-column-commands of one scheduled column command are issued in a single DFI cycle at phases {base, base+sub_phase_stride_i, ...} and columns {col, col+sub_col_stride_i, ...} (fub/mc_dfi_cmd_path.sv:216-227). The per-sub formatter buses merge by OR for address/bank/odt and AND for the active-low control strobes (fub/mc_dfi_cmd_path.sv:308-332). One wr_fire/rd_fire group strobe is emitted per packed group, so the data paths drive/capture the single DFI word once (fub/mc_dfi_cmd_path.sv:396-411).

## 10.6	Write serializer

mc_dfi_wr_serializer is stateless. On wr_fire_i it matures after t_phy_wrlat_i DFI cycles, then streams one DFI word per cycle from the FIFO onto dfi_wrdata_o until wd_last_i (fub/mc_dfi_wr_serializer.sv:8-11, fub/mc_dfi_wr_serializer.sv:64-75). It tracks matured-but-not-finished bursts in r_owed and drives whenever a burst is owed and the FIFO has a word (fub/mc_dfi_wr_serializer.sv:91-99). dfi_wrdata_mask_o = ~wd_strb_i while driving (fub/mc_dfi_wr_serializer.sv:22, fub/mc_dfi_wr_serializer.sv:113).

## 10.7	Read aligner

mc_dfi_rd_aligner admits a read on op_valid_i && op_ready_o. op_ready_o deasserts when MAX_OUTSTANDING reads are already in flight (fub/mc_dfi_rd_aligner.sv:18-25, fub/mc_dfi_rd_aligner.sv:91-98). On admission it drives dfi_rddata_en_o for EN_CYC cycles starting t_rddata_en_i cycles later (fub/mc_dfi_rd_aligner.sv:100-120). It captures dfi_rddata_i whenever dfi_rddata_valid_i is asserted and a read is outstanding, pushing {last, resp=OKAY, data} into the read FIFO (fub/mc_dfi_rd_aligner.sv:127-166).

The capture uses a combinational enable-window credit to reject the PHY preamble valid before the read window; a registered credit dropped the first word in the a7gated model (fub/mc_dfi_rd_aligner.sv:132-146). The 2.1 plane enables the calibration sideband: cal_expect_i arms a one-shot capture of the first valid beat into cal_data_o/cal_valid_o (fub/mc_dfi_rd_aligner.sv:169-201).

The read-return FIFO must cover the whole read-admission domain; RD_FIFO_DEPTH defaults to 32 because depth 16 dropped beats under a paced-but-adjacent burst schedule (macro/mc_dfi_2p1_plane.sv:55-63).

## 10.8	PHY timing knobs

| Knob | Width | Programmed by | Measured pumice value |
| --- | --- | --- | --- |
| t_phy_wrlat_i | [7:0] | PHY_TIMING CSR, bits 7:0 | 1 on Nexys A7 a7ddrphy (research-ip/pumice-ddr2-lpddr2/docs/pumice_mas/ch04_apb_config/02_csr_map.md:272) |
| t_rddata_en_i | [7:0] | PHY_TIMING CSR, bits 15:8 | 6 on Nexys A7 a7ddrphy (research-ip/pumice-ddr2-lpddr2/docs/pumice_mas/ch04_apb_config/02_csr_map.md:273) |
| rd_phase_i / wr_phase_i | PHW | DFI_PHASE CSR | 0 / 0 on Nexys A7 (research-ip/pumice-ddr2-lpddr2/docs/pumice_mas/ch05_programming/04_error_handling.md:67) |

These PHY-integration knobs are not datapath constants; pumice is the measured customer (Nexys A7, 66.67 / 75 MHz) (common-ip/docs/common_has/ch02_architecture/02_descriptor_seam.md:97). The board-validated tuple (2026-07-21) is rd_phase=0, t_rddata_en=6, t_phy_wrlat=1, plus harness DFI_TUNING.rddata_delay=7 (research-ip/pumice-ddr2-lpddr2/docs/pumice_mas/ch04_apb_config/03_runtime_overrides.md:97).

## 10.9	Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_RANKS | 1 | CS/rank fan-out |
| NUM_BANKS | 8 | Number of banks |
| ROW_WIDTH | 14 | Row / address width |
| COL_WIDTH | 10 | Column width |
| DFI_RATE | 2 | Compile-time max DRAM beats per DFI word |
| DRAM_BEAT_WIDTH | 64 | One DRAM beat data width |
| DFI_BEATS_PER_BURST | 8 | DRAM beats per burst |
| N_SUBCMD | 1 | Max sub-column-commands packed into one DFI word |
| SUB_COL_STRIDE | 1 | Compile-time max device-word column stride |
| SUB_PHASE_STRIDE | derived | Compile-time max DFI-phase stride between packed sub-bursts |
| SUBW_MAX | derived | Width holding N_SUBCMD |
| BURST_WORDS | derived | DFI words per burst-group |
| CMD_FIFO_DEPTH | 8 | CDC command FIFO depth |
| WD_FIFO_DEPTH | 16 | CDC write-data FIFO depth |
| RD_FIFO_DEPTH | 32 | CDC read-return FIFO depth |
| N_FLOP_CROSS | 2 | Async-FIFO pointer synchronizer stages |
| USE_JOHNSON | 0 | Pointer encoding: 0=Gray, 1=Johnson |
| DFI_DATA_WIDTH | derived | DRAM_BEAT_WIDTH * DFI_RATE |
| DFI_STRB_WIDTH | derived | DFI_DATA_WIDTH / 8 |
| DFI_EN_WIDTH | derived | DFI_RATE |
| DFI_VALID_WIDTH | derived | DFI_RATE |
| DFI_ADDR_WIDTH | derived | ROW_WIDTH |
| DFI_BANK_WIDTH | derived | $clog2(NUM_BANKS) |
| DFI_CTRL_WIDTH | 1 | Control strobe width |
| DFI_CS_WIDTH | derived | NUM_RANKS |
| DFI_ADDR_BUS_W | derived | DFI_ADDR_WIDTH * DFI_RATE |
| DFI_BANK_BUS_W | derived | DFI_BANK_WIDTH * DFI_RATE |
| DFI_CTRL_BUS_W | derived | DFI_CTRL_WIDTH * DFI_RATE |
| DFI_CS_BUS_W | derived | DFI_CS_WIDTH * DFI_RATE |
| RKW | derived | Rank index width |
| BKW | derived | Bank index width |
| PHW | derived | Phase index width |
| CMD_HISTORY_EN | 0 | DFI-wire command-history scoreboard enable (sim only) |
| HIST_T_RCD / HIST_T_RP / HIST_T_RAS / HIST_T_RFC / HIST_T_WTR / HIST_T_RTW | 0 | Command-history timing constants |
| BL_WORDS | derived | Alias of BURST_WORDS |
| RD_MAX_OUTSTANDING | 16 | Read-aligner outstanding-read tracking depth |
| RD_EN_CYC | derived | rddata_en window width |
| CMD_DW | derived | Command FIFO payload width |
| WD_DW | derived | Write-data FIFO payload width {last,strb,data} |
| RD_DW | derived | Read-return FIFO payload width {last,resp,data} |

## 10.10	Ports

### 10.10.1	Controller domain (ctl_clk)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctl_clk | in | 1 | Controller clock |
| ctl_rstn | in | 1 | Controller reset (active low) |
| cmd_valid_i | in | 1 | Command stream valid (from scheduler) |
| cmd_ready_o | out | 1 | Command stream ready |
| cmd_data_i | in | [CMD_DW-1:0] | Packed {op,rank,bank,row,col,ap} |
| wd_valid_i | in | 1 | Write-data stream valid (from WR CAM) |
| wd_ready_o | out | 1 | Write-data stream ready |
| wd_data_i | in | [WD_DW-1:0] | Packed {last,strb,data} DFI-word granular |
| rd_valid_o | out | 1 | Read-return stream valid (to RD CAM) |
| rd_ready_i | in | 1 | Read-return stream ready |
| rd_data_o | out | [RD_DW-1:0] | Packed {last,resp,data} |
| init_start_i | in | 1 | Init-start request (controller side) |
| init_complete_o | out | 1 | Init-complete indication (controller side) |

### 10.10.2	PHY DFI domain (dfi_clk)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_clk | in | 1 | DFI / PHY clock |
| dfi_rstn | in | 1 | DFI reset (active low) |
| memtype_i | in | memtype_e | DDR2 / LPDDR2 selector |
| rd_phase_i | in | [PHW-1:0] | DFI phase carrying READ command |
| wr_phase_i | in | [PHW-1:0] | DFI phase carrying WRITE command |
| t_phy_wrlat_i | in | [7:0] | WR cmd → dfi_wrdata_en delay |
| t_rddata_en_i | in | [7:0] | RD cmd → dfi_rddata_en window delay |
| gear_i | in | [1:0] | Active gear: active_rate = 1 << gear_i |
| n_subcmd_i | in | [SUBW_MAX-1:0] | Active sub-column commands per DFI word |
| sub_col_stride_i | in | [COL_WIDTH-1:0] | Active device-word column stride |
| sub_phase_stride_i | in | [PHW-1:0] | Active DFI-phase stride between sub-bursts |
| dfi_address_o | out | [DFI_ADDR_BUS_W-1:0] | DFI address bus |
| dfi_bank_o | out | [DFI_BANK_BUS_W-1:0] | DFI bank bus |
| dfi_cas_n_o | out | [DFI_CTRL_BUS_W-1:0] | DFI CAS# (active low) |
| dfi_ras_n_o | out | [DFI_CTRL_BUS_W-1:0] | DFI RAS# (active low) |
| dfi_we_n_o | out | [DFI_CTRL_BUS_W-1:0] | DFI WE# (active low) |
| dfi_cs_n_o | out | [DFI_CS_BUS_W-1:0] | DFI chip-select (active low) |
| dfi_odt_o | out | [DFI_CS_BUS_W-1:0] | DFI ODT |
| dfi_wrdata_o | out | [DFI_DATA_WIDTH-1:0] | DFI write data |
| dfi_wrdata_en_o | out | [DFI_EN_WIDTH-1:0] | DFI write-data enable |
| dfi_wrdata_mask_o | out | [DFI_STRB_WIDTH-1:0] | DFI write-data mask |
| dfi_rddata_en_o | out | [DFI_EN_WIDTH-1:0] | DFI read-data enable |
| dfi_rddata_i | in | [DFI_DATA_WIDTH-1:0] | DFI read data |
| dfi_rddata_valid_i | in | [DFI_VALID_WIDTH-1:0] | DFI read-data valid |
| cal_expect_i | in | 1 | Calibration capture arm |
| cal_data_o | out | [DFI_DATA_WIDTH-1:0] | Calibration captured data |
| cal_valid_o | out | 1 | Calibration capture valid |
| dfi_init_start_o | out | 1 | DFI init-start to PHY |
| dfi_init_complete_i | in | 1 | DFI init-complete from PHY |

## 10.11	Open questions

- The fire_rank_o output of mc_dfi_cmd_path is currently unused inside mc_dfi_2p1_plane (macro/mc_dfi_2p1_plane.sv:375 ties it off). Open question: is this a reserved hook for per-rank ODT or training, or should it be removed?

- db_wr_dbi_en_i/wd_dbi_i and rd_dbi_en_i/dfi_rddata_dbi_i are tied off in the 2.1 plane (macro/mc_dfi_2p1_plane.sv:336-337, macro/mc_dfi_2p1_plane.sv:367-369), matching the DFI 2.1 no-DBI contract. The serializer/aligner HAS_DBI parameters default to 0 and are not exposed by the plane.

- Re-leveling (re-runs of init_start/init_complete) is marked future work in the CDC comments because the event-token crossing is monotonic (fub/mc_dfi_cdc.sv:22).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Chapter 8 — mc_dfi_3p1_plane

Module: macro/mc_dfi_3p1_plane.sv
Family boundary: DFI 3.1 controller/PHY seam
Status: extracted from scoria_dfi_plane during the Phase 2 common-plane reorg; consumed today by scoria_core (research-ip/scoria-ddr3-lpddr3/rtl/top/scoria_core.sv:866).

## 11.1	Purpose and scope

mc_dfi_3p1_plane holds the single ctl_clk/dfi_clk crossing in the controller and presents the controller-clock command/write-data/read-data streams on one side and the DFI 3.1 pin bus on the other (macro/mc_dfi_3p1_plane.sv:8-12). It is a constant-latency conduit: all JEDEC spacing is enforced upstream in the scheduler, and this plane must not stall the in-order command stream (fub/mc_dfi_3p1_cmd_path.sv:160-184).

mc_dfi_3p1_plane internal block diagram

## 11.2	What is inherited unchanged from the 2.1 pipeline

The internal datapath unit is one DFI word; one FIFO word equals one DFI cycle, so the crossing is bubble-free (macro/mc_dfi_3p1_plane.sv:19-20). The 2.1 and 3.1 planes share these child instances and behaviours:

- mc_dfi_cdc — the single async-FIFO crossing for command, write data, read data, and init start/complete (macro/mc_dfi_3p1_plane.sv:254).

- mc_dfi_wr_serializer — delays write data by t_phy_wrlat_i and drives dfi_wrdata_o/dfi_wrdata_en_o/dfi_wrdata_mask_o (macro/mc_dfi_3p1_plane.sv:350).

- mc_dfi_rd_aligner — delays dfi_rddata_en_o by t_rddata_en_i and captures read data into the return FIFO (macro/mc_dfi_3p1_plane.sv:373).

- Active-gear phase mask — gear_i selects the active DFI rate; the unmasked dfi_wrdata_en/dfi_rddata_en from the serializer/aligner are gated by w_phase_active before the pins. At gear_i == log2(DFI_RATE) the mask is all-ones (macro/mc_dfi_3p1_plane.sv:221-238).

## 11.3	What the 3.1 delta adds

The generation delta is concentrated in the command path and the write-leveling surface.

### 11.3.1	Command path: mc_dfi_3p1_cmd_path

The 3.1 plane instantiates fub/mc_dfi_3p1_cmd_path.sv (macro/mc_dfi_3p1_plane.sv:300) in place of the 2.1 mc_dfi_cmd_path. The 3.1 path:

- Drops the optional 2.1 mode-register-read bit; the command word is {ap, col, row, bank, rank, op} (fub/mc_dfi_3p1_cmd_path.sv:99-101, fub/mc_dfi_3p1_cmd_path.sv:158).

- Formats commands through the silicon-proven scoria_dfi_cmd_formatter (fub/mc_dfi_3p1_cmd_path.sv:255).

- Supports sub-DFI-word burst packing via N_SUBCMD, SUB_COL_STRIDE, and SUB_PHASE_STRIDE: when N_SUBCMD > 1, one scheduled column command fans out into N_SUBCMD sub-commands placed on distinct DFI phases of the same cycle, and the per-sub buses merge by AND for control/CS and OR for address/bank/ODT (fub/mc_dfi_3p1_cmd_path.sv:199-215, fub/mc_dfi_3p1_cmd_path.sv:291-315).

- Uses the scoria_cmd_history_checker when CMD_HISTORY_EN is set (fub/mc_dfi_3p1_cmd_path.sv:333-352).

Clause-level DFI 3.1 pin semantics and the per-PHY integration contract are owned by the scoria book (../../../research-ip/scoria-ddr3-lpddr3/docs/scoria_mas/ch03_interfaces/02_dfi_v31_pins.md); this chapter does not restate them.

### 11.3.2	Write-leveling CDC

The 3.1 plane adds the DDR3 write-leveling handshake crossing. Because the plane owns the only ctl_clk/dfi_clk boundary, routing these signals through it avoids an unsynchronized crossing (macro/mc_dfi_3p1_plane.sv:184-189). Slow levels use cdc_synchronizer flop chains; the one-cycle wl_strobe_i pulse uses sync_pulse so the pulse count survives arbitrary clock ratios (macro/mc_dfi_3p1_plane.sv:192-195, macro/mc_dfi_3p1_plane.sv:425-470). Active-low _n signals are inverted to active-high for synchronization and inverted again at the output to avoid spurious assertions out of reset (macro/mc_dfi_3p1_plane.sv:409-422).

The scoria book states that this PHY family does not implement the DFI leveling handshake, so on the Genesys 2 build these pins are unused and write leveling is the PHY’s own (../../../research-ip/scoria-ddr3-lpddr3/docs/scoria_mas/ch03_interfaces/02_dfi_v31_pins.md:123). The protocol state machine is documented in the scoria MAS (../../../research-ip/scoria-ddr3-lpddr3/docs/scoria_mas/ch02_blocks/20_wrlvl_ifc.md).

### 11.3.3	Calibration sideband removed

The 2.1 plane exposes cal_expect_i, cal_data_o, and cal_valid_o on the dfi_clk domain (macro/mc_dfi_2p1_plane.sv:173-176). The 3.1 plane removes these ports and ties HAS_CAL_CAPTURE to 0 inside mc_dfi_rd_aligner (macro/mc_dfi_3p1_plane.sv:380).

## 11.4	Parameters

Table 8.1: mc_dfi_3p1_plane parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_RANKS | 1 | DRAM ranks. |
| NUM_CS | NUM_RANKS | Chip selects. |
| NUM_BANKS | 8 | DRAM banks. |
| ROW_WIDTH | 14 | Row address width. |
| COL_WIDTH | 10 | Column address width. |
| DFI_RATE | 2 | Compile-time max DFI phases per clock. |
| DRAM_BEAT_WIDTH | 64 | DRAM beat width. |
| DFI_BEATS_PER_BURST | 8 | DRAM beats per burst (BL_PUMICE). |
| N_SUBCMD | 1 | Compile-time max sub-commands packed into one DFI word. |
| SUB_COL_STRIDE | 1 | Compile-time max column stride between sub-bursts. |
| SUB_PHASE_STRIDE | (N_SUBCMD>1)?(DFI_RATE/N_SUBCMD):1 | Compile-time max DFI-phase stride between packed sub-bursts. |
| SUBW_MAX | $clog2(N_SUBCMD+1) | Width holding active N_SUBCMD. |
| BURST_WORDS | (DFI_BEATS_PER_BURST>=DFI_RATE)?(DFI_BEATS_PER_BURST/DFI_RATE):1 | DFI words per burst group; clamped to 1 in the sub-word regime. |
| CMD_FIFO_DEPTH | 8 | Command async FIFO depth. |
| WD_FIFO_DEPTH | 16 | Write-data async FIFO depth. |
| RD_FIFO_DEPTH | 32 | Read-data async FIFO depth. Required because dfi_rddata_valid is fire-and-forget; depth 16 dropped beats under paced adjacent bursts (macro/mc_dfi_3p1_plane.sv:56-64). |
| N_FLOP_CROSS | 2 | Pointer synchronizer flop stages. |
| USE_JOHNSON | 0 | Async FIFO pointer encoding: 0 = Gray, 1 = Johnson. |
| CMD_HISTORY_EN | 0 | Enable DFI-wire command-history scoreboard (sim only). |
| HIST_T_RCD | 0 | History checker tRCD. |
| HIST_T_RP | 0 | History checker tRP. |
| HIST_T_RAS | 0 | History checker tRAS. |
| HIST_T_RFC | 0 | History checker tRFC. |
| HIST_T_WTR | 0 | History checker tWTR. |
| HIST_T_RTW | 0 | History checker tRTW. |
| BL_WORDS | BURST_WORDS | Alias exposed to the read aligner. |
| RD_MAX_OUTSTANDING | 16 | Read aligner outstanding-read tracking depth. |
| RD_EN_CYC | BL_WORDS | dfi_rddata_en window width. The default is wrong when DRAM_BEAT_WIDTH > DRAM_DEVICE_WIDTH; scoria overrides explicitly (macro/mc_dfi_3p1_plane.sv:104-112). |

Derived geometry parameters (DFI_DATA_WIDTH, DFI_STRB_WIDTH, DFI_EN_WIDTH, DFI_VALID_WIDTH, DFI_ADDR_WIDTH, DFI_BANK_WIDTH, DFI_CTRL_WIDTH, DFI_CS_WIDTH, the *_BUS_W widths, RKW, BKW, PHW, and FIFO payload widths CMD_DW/WD_DW/RD_DW) are computed in macro/mc_dfi_3p1_plane.sv:71-116.

## 11.5	Ports

### 11.5.1	Controller domain (ctl_clk)

Table 8.2: Controller-domain ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctl_clk | in | 1 | Controller clock. |
| ctl_rstn | in | 1 | Controller reset, active-low. |
| cmd_valid_i | in | 1 | Command valid from scheduler. |
| cmd_ready_o | out | 1 | Command ready to scheduler. |
| cmd_data_i | in | CMD_DW | Packed command {ap, col, row, bank, rank, op}. |
| wd_valid_i | in | 1 | Write-data valid. |
| wd_ready_o | out | 1 | Write-data ready. |
| wd_data_i | in | WD_DW | Write data {last, strb, data}. |
| rd_valid_o | out | 1 | Read-data valid. |
| rd_ready_i | in | 1 | Read-data ready. |
| rd_data_o | out | RD_DW | Read data {last, resp, data}. |
| init_start_i | in | 1 | Init start level. |
| init_complete_o | out | 1 | Init complete sticky. |

### 11.5.2	PHY DFI domain (dfi_clk)

Table 8.3: PHY DFI-domain ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_clk | in | 1 | DFI clock. |
| dfi_rstn | in | 1 | DFI reset, active-low. |
| memtype_i | in | memtype_e | DRAM type (DDR3/LPDDR3). |
| rd_phase_i | in | PHW | Read-command DFI phase. |
| wr_phase_i | in | PHW | Write-command DFI phase. |
| t_phy_wrlat_i | in | 8 | WR command to dfi_wrdata_en delay. |
| t_rddata_en_i | in | 8 | RD command to dfi_rddata_en delay. |
| gear_i | in | 2 | Active DFI gear ratio selector. |
| n_subcmd_i | in | SUBW_MAX | Active sub-commands per DFI word. |
| sub_col_stride_i | in | COL_WIDTH | Active column stride. |
| sub_phase_stride_i | in | PHW | Active phase stride. |
| dfi_address_o | out | DFI_ADDR_BUS_W | DFI address bus. |
| dfi_bank_o | out | DFI_BANK_BUS_W | DFI bank bus. |
| dfi_cas_n_o | out | DFI_CTRL_BUS_W | DFI CAS#. |
| dfi_ras_n_o | out | DFI_CTRL_BUS_W | DFI RAS#. |
| dfi_we_n_o | out | DFI_CTRL_BUS_W | DFI WE#. |
| dfi_cs_n_o | out | DFI_CS_BUS_W | DFI chip select. |
| dfi_odt_o | out | DFI_CS_BUS_W | DFI ODT. |
| dfi_wrdata_o | out | DFI_DATA_WIDTH | DFI write data. |
| dfi_wrdata_en_o | out | DFI_EN_WIDTH | DFI write data enable. |
| dfi_wrdata_mask_o | out | DFI_STRB_WIDTH | DFI write data mask. |
| dfi_rddata_en_o | out | DFI_EN_WIDTH | DFI read data enable. |
| dfi_rddata_i | in | DFI_DATA_WIDTH | DFI read data. |
| dfi_rddata_valid_i | in | DFI_VALID_WIDTH | DFI read data valid. |
| dfi_init_start_o | out | 1 | DFI init start sticky. |
| dfi_init_complete_i | in | 1 | DFI init complete level. |

### 11.5.3	Write-leveling crossing ports

Table 8.4: Write-leveling crossing ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wl_req_cs_n_i | in | NUM_CS | Leveling request from scheduler, active-low. |
| wl_wrlvl_cs_n_i | in | NUM_CS | WRLVL mode from scheduler, active-low. |
| wl_strobe_i | in | 1 | One-cycle strobe from scheduler. |
| wl_ack_cs_n_o | out | NUM_CS | PHY ack to scheduler, active-low. |
| wl_prime_dq_o | out | 1 | Prime DQ to scheduler. |
| dfi_phylvl_req_cs_n_o | out | NUM_CS | Leveled request to PHY, active-low. |
| dfi_phylvl_ack_cs_n_i | in | NUM_CS | PHY ack, active-low. |
| dfi_phy_wrlvl_cs_n_o | out | NUM_CS | Leveled WRLVL mode to PHY, active-low. |
| dfi_wrlvl_strobe_o | out | 1 | Leveled one-cycle strobe to PHY. |
| dfi_prime_dq_i | in | 1 | Prime DQ from PHY. |

## 11.6	Family-boundary doctrine

DFI revision is a family boundary, not a tunable parameter: a given controller instance commits to one DFI generation at compile time. The rationale and lineage are in the architecture book (../common_has/ch03_dfi_generations/01_dfi_planes.md).

## 11.7	Open questions and verification notes

- scoria_core.sv at research-ip/scoria-ddr3-lpddr3/rtl/top/scoria_core.sv:866 instantiates mc_dfi_3p1_plane and wires every write-leveling port (research-ip/scoria-ddr3-lpddr3/rtl/top/scoria_core.sv:937-947), yet a stale comment at research-ip/scoria-ddr3-lpddr3/rtl/top/scoria_core.sv:234 claims the handshake does not go through the plane. The RTL wiring is the operative fact; the comment is inconsistent.

- The sibling scoria MAS chapter (../../../research-ip/scoria-ddr3-lpddr3/docs/scoria_mas/ch02_blocks/03_dfi_plane.md) still documents the removed scoria_dfi_plane; no scoria_dfi_plane.sv or scoria_dfi_cmd_path.sv remains in the scoria RTL tree (Glob returned no matches). That chapter is therefore out of date with respect to current RTL.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	mc_dfi_4p0_plane — DFI 4.0 Generation Delta

mc_dfi_4p0_plane is the common DFI 4.0 boundary block extracted from andesite_dfi_plane during the Phase 2 common-plane reorganization (mc_dfi_4p0_plane.sv:20). It holds the single controller-clock to DFI-clock crossing and the dfi_clk-domain datapath that turns the family streams into the DFI 4.0 pin surface.

mc_dfi_4p0_plane internal block diagram

DFI 4.0 = shared pipeline + DDR4/LPDDR4 command surface

## 12.1	Inherited shared pipeline

The plane instantiates four shared FUBs (mc_dfi_4p0_plane.sv:14-18): mc_dfi_cdc for the async command/write/read/init-token crossings (mc_dfi_4p0_plane.sv:220-259), mc_dfi_4p0_cmd_path for the widened scheduler word to DFI 4.0 pins (mc_dfi_4p0_plane.sv:266-297), mc_dfi_wr_serializer with HAS_DBI=1 (mc_dfi_4p0_plane.sv:302-321), and mc_dfi_rd_aligner with HAS_DBI=1 (mc_dfi_4p0_plane.sv:326-354). Training pins live on rock-specific mc_training_plane wrappers, not here (mc_dfi_4p0_plane.sv:25).

## 12.2	What the 4.0 delta adds

### 12.2.1	Command path (mc_dfi_4p0_cmd_path)

The command path accepts the widened word {ap, col, row, bg, bank, rank, op} (mc_dfi_4p0_cmd_path.sv:91-99); the bg field is the structural delta over the 3.1 word (mc_dfi_4p0_cmd_path.sv:22-23). It presents each accepted command to the rock-local P1 single-shot formatter andesite_dfi_cmd_formatter for one cycle and exposes the registered DFI pin vector plus fire strobes (mc_dfi_4p0_cmd_path.sv:8-13). Structural readiness is the only backpressure (mc_dfi_4p0_cmd_path.sv:104-109), per the carried scoria TASK-007 contract (mc_dfi_4p0_cmd_path.sv:107). The CMD_HISTORY_EN parameter is audit-only and not instantiated in v1 (mc_dfi_4p0_cmd_path.sv:41-45). Address routing: OP_MRS/OP_ZQCL payload rides ROW; OP_RD/OP_RDA/OP_WR/OP_WRA merge ap into JEDEC A10; OP_PREA forces A10 high and OP_PRE clears it (mc_dfi_4p0_cmd_path.sv:145-152).

ACT_n. dfi_act_n_o is driven by andesite_dfi_cmd_formatter together with dfi_ras_n_o, dfi_cas_n_o, and dfi_we_n_o (mc_dfi_4p0_cmd_path.sv:178-181), forming the DFI 4.0 activate surface (andesite HAS §3.1 Control Interface).

Bank groups. dfi_bg_o is driven from the unpacked w_bg field (mc_dfi_4p0_cmd_path.sv:95, 142), giving the DFI 4.0 bank-group address (andesite HAS §3.1 Control Interface).

CA parity. parity_en_i feeds the formatter to generate dfi_parity_in_o (mc_dfi_4p0_cmd_path.sv:60, 176, 186; andesite HAS §3.5.7 CRC and CA Parity). The hw/fw split of any parity-error counting machinery is an open question inherited from the andesite HAS §TBC(TASK-005).

Gear-down. gear_i[1:0] selects the active DFI rate as 1 << gear_i; the phase-active mask is applied to dfi_wrdata_en_o and dfi_rddata_en_o (mc_dfi_4p0_plane.sv:182-195). The andesite HAS states that the init sequencer drives gear-down entry after init (andesite HAS §3.13 Geardown Mode); that sequencing is not visible in this common block’s RTL §TBC(TASK-005).

DBI. Write DBI travels with the CDC word {last,strb,dbi,data} and is driven through dfi_wrdata_mask_o (mc_dfi_4p0_plane.sv:75, 96, 148, 302-321). Read DBI enters on dfi_rddata_dbi_i and exits on rd_dbi_o (mc_dfi_4p0_plane.sv:103, 155, 326-354); both child instances set HAS_DBI=1 (mc_dfi_4p0_plane.sv:305, 333). This matches the andesite HAS correction that DFI 4.0 has no dfi_wrdata_dbi pin (andesite HAS §3.2.1 Write Data Mask/Write DBI, §3.3.1 Read DBI).

Alert_n. dfi_alert_n* handling is not in this plane; the andesite HAS places it in a boundary alert_n submodule surfaced through dfi_error (andesite HAS §3.5.7 CRC and CA Parity), while the common plane leaves alerts to the rock training wrapper (mc_dfi_4p0_plane.sv:25) §TBC(TASK-005).

## 12.3	What the plane leaves unimplemented

| Omission | Rationale | Citation |
| --- | --- | --- |
| DDR5-oriented 4.x features | No DDR5 target | andesite HAS §2.2.1, §2.2.2 |
| Frequency ratios beyond inherited 1:4 | No board PHY to consume them | andesite HAS §4.9 |
| dfi_lp_* request consumption | Dormant-pair condition; requests stay presented | andesite HAS §3.7 |
| Training pins | Live on mc_training_plane rock wrappers | mc_dfi_4p0_plane.sv:25; andesite HAS §3.6 |

dfi_cke_o is registered from cke_i, not produced by the command-path formatter (mc_dfi_4p0_plane.sv:125, 362-373). At NUM_RANKS=1, dfi_wrdata_cs_o and dfi_rddata_cs_o are driven zero, matching the scoria v3.1 precedent (mc_dfi_4p0_plane.sv:30-31, 359-360).

## 12.4	Parameters

### 12.4.1	mc_dfi_4p0_plane

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_RANKS | 1 | Ranks |
| NUM_BANKS | 8 | Banks per group |
| NUM_BG | 4 | Bank groups |
| ROW_WIDTH | 14 | Row width |
| COL_WIDTH | 10 | Column width |
| ADDR_WIDTH | 18 | DFI address width |
| DFI_RATE | 2 | DFI:DRAM clock ratio minus one (1:4 default) |
| DRAM_BEAT_WIDTH | 64 | DRAM beat width |
| DFI_DATA_WIDTH | DRAM_BEAT_WIDTH * DFI_RATE | DFI data width |
| CMD_FIFO_DEPTH | 8 | Command FIFO depth |
| WD_FIFO_DEPTH | 16 | Write-data FIFO depth |
| RD_FIFO_DEPTH | 32 | Read-data FIFO depth |
| RD_MAX_OUTSTANDING | 16 | Read-aligner outstanding depth |
| RD_EN_CYC | 4 | dfi_rddata_en window (DFI cycles) |
| BL_WORDS | RD_EN_CYC | DFI words per read |
| N_FLOP_CROSS | 2 | CDC synchronizer flops |
| USE_JOHNSON | 0 | 0 = Gray, 1 = Johnson |
| DFI_STRB_WIDTH | DFI_DATA_WIDTH / 8 | Byte/mask/DBI width |
| DFI_EN_WIDTH | DFI_RATE | dfi_*data_en width |
| DFI_VALID_WIDTH | DFI_RATE | dfi_rddata_valid width |
| BANK_WIDTH | $clog2(NUM_BANKS) | Bank address width |
| BG_WIDTH | 2 | Bank-group width |
| RKW | (NUM_RANKS > 1) ? $clog2(NUM_RANKS) : 1 | Rank index width |
| BKW | $clog2(NUM_BANKS) | Bank index in command word |
| BGW | (NUM_BG > 1) ? $clog2(NUM_BG) : 1 | Bank-group index in command word |
| PHW | (DFI_RATE > 1) ? $clog2(DFI_RATE) : 1 | Phase index width |
| CMD_DW | $bits(dram_op_e) + RKW + BKW + BGW + ROW_WIDTH + COL_WIDTH + 1 | Command word {ap,col,row,bg,bank,rank,op} |
| WD_DW | 1 + DFI_STRB_WIDTH + DFI_STRB_WIDTH + DFI_DATA_WIDTH | Write-data CDC payload {last,strb,dbi,data} |
| RD_DW | 1 + 2 + DFI_STRB_WIDTH + DFI_DATA_WIDTH | Read-data CDC payload {last,resp,dbi,data} |

## 12.5	Ports

### 12.5.1	mc_dfi_4p0_plane — controller domain (ctl_clk)

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| ctl_clk | input | 1 | Controller clock |
| ctl_rstn | input | 1 | Controller reset |
| cmd_valid_i | input | 1 | Command valid |
| cmd_ready_o | output | 1 | Command ready |
| cmd_data_i | input | [CMD_DW-1:0] | Command word |
| wd_valid_i | input | 1 | Write-data valid |
| wd_ready_o | output | 1 | Write-data ready |
| wd_data_i | input | [DFI_DATA_WIDTH-1:0] | Write data |
| wd_strb_i | input | [DFI_STRB_WIDTH-1:0] | Write byte strobe |
| wd_dbi_i | input | [DFI_STRB_WIDTH-1:0] | Write DBI |
| wd_last_i | input | 1 | Last word of burst |
| rd_valid_o | output | 1 | Read-data valid |
| rd_ready_i | input | 1 | Read-data ready |
| rd_data_o | output | [DFI_DATA_WIDTH-1:0] | Read data |
| rd_dbi_o | output | [DFI_STRB_WIDTH-1:0] | Read DBI |
| rd_resp_o | output | [1:0] | Read response |
| rd_last_o | output | 1 | Last word of read return |
| init_busy_i | input | 1 | Macro init busy; crossed to dfi_init_start_o |
| init_done_i | input | 1 | Macro init done (unused) |
| rd_dbi_en_i | input | 1 | Read DBI enable |
| wr_dbi_en_i | input | 1 | Write DBI enable |
| memtype_i | input | memtype_e | Memory type |
| parity_en_i | input | 1 | CA parity enable |
| gear_i | input | [1:0] | Gear-down selector |
| rd_phase_i | input | [PHW-1:0] | Read phase (unused) |
| wr_phase_i | input | [PHW-1:0] | Write phase (unused) |
| t_phy_wrlat_i | input | [7:0] | WR cmd to dfi_wrdata_en |
| t_rddata_en_i | input | [7:0] | RD cmd to dfi_rddata_en |
| cke_i | input | 1 | CKE from P1 init sequencer |

### 12.5.2	mc_dfi_4p0_plane — PHY DFI domain (dfi_clk)

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_clk | input | 1 | DFI clock |
| dfi_rstn | input | 1 | DFI reset |
| dfi_address_o | output | [ADDR_WIDTH-1:0] | Address |
| dfi_bank_o | output | [BANK_WIDTH-1:0] | Bank |
| dfi_bg_o | output | [BG_WIDTH-1:0] | Bank group |
| dfi_act_n_o | output | 1 | Activate command |
| dfi_ras_n_o | output | 1 | RAS_n |
| dfi_cas_n_o | output | 1 | CAS_n |
| dfi_we_n_o | output | 1 | WE_n |
| dfi_cs_o | output | [NUM_RANKS-1:0] | Chip select |
| dfi_cke_o | output | 1 | CKE, registered from cke_i |
| dfi_parity_in_o | output | 1 | CA parity |
| dfi_wrdata_o | output | [DFI_DATA_WIDTH-1:0] | Write data |
| dfi_wrdata_en_o | output | [DFI_EN_WIDTH-1:0] | Write data enable, phase-masked |
| dfi_wrdata_mask_o | output | [DFI_STRB_WIDTH-1:0] | Write data mask / write DBI |
| dfi_wrdata_cs_o | output | [NUM_RANKS-1:0] | Write data chip select (zero at NUM_RANKS=1) |
| dfi_rddata_en_o | output | [DFI_EN_WIDTH-1:0] | Read data enable, phase-masked |
| dfi_rddata_i | input | [DFI_DATA_WIDTH-1:0] | Read data |
| dfi_rddata_valid_i | input | [DFI_VALID_WIDTH-1:0] | Read data valid |
| dfi_rddata_dbi_i | input | [DFI_STRB_WIDTH-1:0] | Read DBI |
| dfi_rddata_cs_o | output | [NUM_RANKS-1:0] | Read data chip select (zero at NUM_RANKS=1) |
| dfi_init_start_o | output | 1 | Init start, CDC token of init_busy_i |
| dfi_init_complete_i | input | 1 | Init complete |

### 12.5.3	mc_dfi_4p0_cmd_path — dfi_clk domain

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_clk | input | 1 | DFI clock |
| dfi_rstn | input | 1 | DFI reset |
| memtype_i | input | memtype_e | Memory type |
| parity_en_i | input | 1 | CA parity enable |
| cmd_valid_i | input | 1 | Command valid |
| cmd_ready_o | output | 1 | Command ready |
| cmd_data_i | input | [CMD_DW-1:0] | Command word |
| rd_op_ready_i | input | 1 | Read-op ready from aligner |
| wr_op_ready_i | input | 1 | Write-op ready (staged token) |
| wr_fire_o | output | 1 | Write fired, registered |
| rd_fire_o | output | 1 | Read fired, registered |
| fire_rank_o | output | [RKW-1:0] | Rank of fired command |
| wr_accept_o | output | 1 | Pop staged write-data token |
| dfi_address_o | output | [ADDR_WIDTH-1:0] | Address |
| dfi_bank_o | output | [BANK_WIDTH-1:0] | Bank |
| dfi_bg_o | output | [BG_WIDTH-1:0] | Bank group |
| dfi_act_n_o | output | 1 | Activate command |
| dfi_ras_n_o | output | 1 | RAS_n |
| dfi_cas_n_o | output | 1 | CAS_n |
| dfi_we_n_o | output | 1 | WE_n |
| dfi_cs_o | output | [NUM_RANKS-1:0] | Chip select |
| dfi_cke_o | output | 1 | Placeholder CKE from formatter |
| dfi_parity_in_o | output | 1 | CA parity |

## 12.6	Family boundary rule

The rule that assigns each controller generation to its DFI revision, and the doctrine for what belongs at the controller/PHY seam, is owned by the family architecture specification: see common_has chapter 3. The lineage summary lives at ../../03_dfi_boundary_lineage.md, and the andesite HAS clause-level DFI 4.0 surface is at ../../../research-ip/andesite-ddr4-lpddr4/docs/andesite_has/ch04_interfaces/01_dfi_v40.md.

## 12.7	Verification record

As of today, andesite’s top-level RTL instantiates mc_dfi_4p0_plane directly (research-ip/andesite-ddr4-lpddr4/rtl/top/andesite_core.sv:882) and the andesite filelists pull in common-ip/rtl/filelists/macro/mc_dfi_4p0_plane.f (research-ip/andesite-ddr4-lpddr4/rtl/filelists/andesite_all.f:19, andesite_core.f:46). The rock-local andesite_dfi_plane is no longer the top boundary; only its P1 formatter andesite_dfi_cmd_formatter remains inside mc_dfi_4p0_cmd_path.