RTL Design Sherpa

amber — Blocking MESI Snoopy L1 Cache Hardware Architecture Specification 0.5

October 6, 2026

Table of Contents

1 Amber Has Index	13

2 amber MAS Index	14

2.1 Document Organization	14

2.1.1 Front Matter	14

2.1.2 Chapter 1: Overview	14

2.1.3 Chapter 2: Functional Blocks	14

2.1.4 Chapter 3: Interfaces	14

2.1.5 Chapter 4: MonBus Observation Map	15

2.1.6 Chapter 5: Verification Bring-Up	15

2.1.7 Chapter 6: Configuration	15

2.2 Quick Reference	15

2.2.1 Core Blocks	15

2.2.2 Top Modules	16

2.3 Related Documentation	16

3 amber — Product Requirements (DRAFT)	18

3.1 1. Purpose	18

3.2 2. Background	18

3.3 3. Decisions that pick the design	19

3.4 4. Success criteria	28

3.5 5. Non-goals	28

3.6 6. Where the block sits	28

4 Global Hard Requirements for RTL Design Sherpa	29

4.1 Overview	29

4.2 Category 1: RTL Standards	29

4.2.1 1.1 Reset Handling (MANDATORY - NO EXCEPTIONS)	29

4.2.2 1.2 FPGA Synthesis Attributes (MANDATORY)	30

4.2.3 1.3 Array Syntax Standard (MANDATORY)	31

4.2.4 1.4 SRAM Module Standards (MANDATORY)	31

4.2.5 1.5 Reset Convention (STANDARD)	32

4.3 Category 2: Testbench Architecture	32

4.3.1 2.1 TB Location Requirements (MANDATORY)	32

4.3.2 2.2 Three Mandatory TB Methods (MANDATORY)	33

4.3.3 2.3 TBBase Inheritance (MANDATORY)	33

4.3.4 2.4 Three-Layer Architecture (MANDATORY)	34

4.3.5 2.5 Queue-Based Verification (PREFERRED)	35

4.4 Category 3: Test Standards	35

4.4.1 3.1 Test Naming Convention (MANDATORY)	35

4.4.2 3.2 Test Pattern - CocoTB + Pytest (MANDATORY for projects/components/)	36

4.4.3 3.3 100% Success Requirement (MANDATORY)	36

4.4.4 3.4 Simulation Timestamps in Logs (MANDATORY)	37

4.5 Category 4: Framework Usage	38

4.5.1 4.1 Search Before Creating (MANDATORY)	38

4.5.2 4.2 BFM Usage for Protocols (MANDATORY for RAPIDS)	38

4.6 Category 5: Documentation	39

4.6.1 5.1 No Emojis in Technical Docs (MANDATORY)	39

4.7 Category 6: Project-Specific Requirements	39

4.7.1 6.1 Commit Messages Carry No Attribution Trailer	39

4.7.2 6.2 STREAM Tutorial Focus	39

4.8 Summary by Priority	40

4.8.1 P0 - Critical (Enforcement Required)	40

4.8.2 P1 - High Priority (Strong Recommendation)	40

4.8.3 P2 - Standard Practice (Best Practices)	40

4.8.4 P3 - Project-Specific	40

4.9 Compliance Checklist	41

4.10 Enforcement	41

4.11 References	42

5 Document Information	43

5.1 Related documents	43

5.2 Terminology	44

5.3 Revision history	44

5.4 How to read this book	45

6 Purpose and Scope	46

6.1 Purpose	46

6.2 Scope	46

6.3 Why a blocking cache matters	46

7 Document Conventions	48

7.1 Signal naming	48

7.2 Numbering and bases	48

7.3 Citation discipline	48

7.4 Diagram conventions	48

7.5 Voice	49

8 Definitions and Acronyms	50

9 Use Cases	52

9.1 Primary use case: a correct, observable, provable L1 cache baseline	52

9.2 Secondary use cases	52

9.3 What amber is not designed for	52

10 Key Features	53

11 System Context	55

11.1 Where amber sits	55

11.2 Pair rig (amber top)	55

11.3 Onyx rig (amber_ace top)	55

11.4 Shared core, different tops	55

12 Block Diagram	56

12.1 Module roles	56

13 Top Structure and the Two Rigs	58

13.1 No rig parameter	58

13.2 amber — pair-rig top	58

13.3 amber_ace — onyx-rig top	58

13.4 What is inside amber_core	58

14 Data Flow: Hits, Misses, Fills, and Drains	60

14.1 Hit path	60

14.2 Miss path	60

14.3 Snoop path	60

14.4 Open shape: memory-side masters (D3)	61

14.5 Open shape: array construction (D11)	61

15 Control FSM: Blocking Pipeline and Pending-Fill Bypass	62

15.1 The blocking pipeline	62

15.2 Pending-fill bypass register	62

15.3 FSM oracle derivation	63

16 Coherence FSM and the Snoop Responder	64

16.1 MESI state handling	64

16.2 Snoop flow	66

16.3 Ordering convention	67

17 Arbitration and the Victim Bypass	68

17.1 Dual-port array arbitration	68

17.2 Victim bypass	68

17.3 Liveness target	68

18 CPU-Side GAXI Slave	69

18.1 Interface decision	69

18.2 Channel structure	69

18.3 No register block	70

19 Fabric-Side AXI4 Masters	71

19.1 Where they live	71

19.2 Burst behavior	71

19.3 Port summary	71

19.4 Open point: D3	71

20 Fabric-Side ACE Masters and Coherent Issue	73

20.1 Onyx-rig fabric	73

20.2 amber_ace_issue	73

20.3 Real ACE master read port (axi4ace_master_rd)	73

20.4 Real ACE master write port (axi4ace_master_wr)	75

21 Snoop Port: AC/CD/CR Adapter	77

21.1 Adapter purpose	77

21.2 Real snoop-slave port (axi4ace_snoop_slave)	77

21.3 Ordering and the family convention	78

21.4 Bus-agnostic core boundary	79

22 Observation: MonBus through amber_monlite	80

22.1 Observation decision	80

22.2 MonBus packet content	80

22.3 Observer cost	80

23 amber_pkg and Build-Time Geometry	82

23.1 Parameter philosophy	82

23.2 Core parameters	82

23.3 Derived quantities	82

23.4 Package contents	83

23.5 Rig is not a parameter	83

24 Clocking and Reset	84

24.1 Clock domain	84

24.2 Reset	84

24.3 Reset sequence	84

24.4 Clock gating	84

25 Verification Strategy	85

25.1 Pattern-B cocotb grids	85

25.2 Golden-model parity	85

25.3 Formal targets	85

25.4 Python reference model	85

25.5 MonBus coverage	85

26 Characterization	87

26.1 Board targets	87

26.2 Sweep dimensions	87

26.3 Reported metrics	87

26.4 Critical-path expectation	87

27 Future Hooks	89

27.1 Recorded future work, not blockers	89

27.2 Closing D3 and D11	90

28 amber — Pre-Hardware Architecture Specification (Pre-HAS)	91

28.1 1. Intent of this document	91

28.2 2. Feature sketches	91

28.2.1 2.1 Core datapath — the blocking bet	91

28.2.2 2.2 Coherence	92

28.2.3 2.3 Fabric side — where the two deployment rigs meet	93

28.2.4 2.4 Observation (D8 DECIDED — sketched, not re-decided)	94

28.2.5 2.5 Verification & formal surface (D9 OPEN — direction proposed)	94

28.2.6 2.6 Cost & characterisation (PRD criterion 4)	94

28.3 3. Proposed block architecture	95

28.4 4. Interface sketches (shapes, not pinouts)	96

28.5 5. Parameter sketch (illustrative — feeds the DV matrix)	97

28.6 6. Open questions this sketch surfaces	98

28.7 7. What the full HAS will add	99

28.8 8. Cross-references	99

29 jet — Product Requirements (DRAFT)	101

29.1 1. Purpose	101

29.2 2. Decisions jet adds to amber’s	101

29.3 3. Success criteria (additions to amber’s)	103

29.4 4. Non-goals (for jet, beyond amber’s)	103

30 onyx — Product Requirements (DRAFT)	104

30.1 1. Purpose	104

30.2 2. Background	104

30.3 3. Decisions that pick the design	105

30.4 4. Success criteria	113

30.5 5. Non-goals	113

30.6 6. Where the block sits	113

31 AMBA ACE — Working Interface Definition for cache-ip	114

31.1 1. What ACE is	114

31.2 2. Channel inventory	114

31.2.1 2.1 Coherent master → manager (front side, per master)	114

31.2.2 2.2 Manager → cache, cache → manager (snoop side, per cache)	115

31.3 3. Transaction types (the onyx D2 research subset in bold)	116

31.4 4. The CRRESP decision bits	117

31.5 5. ACE-Lite	118

31.6 6. What this family implements, and what it deliberately does not	118

31.7 7. Sources	118

32 gem5 Ruby protocol specs (reference extract)	120

33 Cache Associativity Simulator	121

33.1 Files	121

33.2 Model API	121

33.3 Running tests	122

33.4 UI features	122

List of Figures

No figures in this document.

List of Tables

Table 1: Table 0.1: Document information	43

Table 2: Table 0.2: Related documents	43

Table 3: Table 0.3: Revision history	44

Table 4: Table 1.0: Definitions and acronyms	50

Table 5: Table 2.0: Key features and their decisions	53

Table 6: Table 2.1: Module roles	56

Table 7: Table 3.0: MESI snoop CRRESP matrix — DT=DataTransfer, PD=PassDirty, IS=IsShared, WU=WasUnique. Snoop encodings are the six IHI0022 AC types; CleanUnique and MakeUnique are transactions a master issues (the onyx D2 subset), not snoop encodings, so they do not appear here.	64

Table 8: Table 4.0: CPU-side GAXI slave channels	69

Table 9: Table 4.1: Plain AXI4 channels in the pair rig	71

Table 10: Table 4.2: Cache events to ACE transactions	73

Table 11: Table 4.3: axi4ace_master_rd pin contract	73

Table 12: Table 4.4: axi4ace_master_wr pin contract	75

Table 13: Table 4.5: axi4ace_snoop_slave pin contract	77

Table 14: Table 4.6: MonBus event classes	80

Table 15: Table 5.0: Core elaboration parameters	82

Table 16: Table 6.0: Characterization sweep dimensions	87

Table 17: Table 6.1: Future hooks	89

# 1	Amber Has Index

Generated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	amber MAS Index

Version: 0.5 Date: 2026-10-06 Purpose: Micro-architecture specification for the amber blocking MESI snoopy L1 cache

Scope: Cycle-level behavior, per-module micro-architecture, and the pre-RTL signal-contract / kmap workbook for amber (pair-rig top) and amber_ace (onyx-rig top). The binding architecture lives in the amber HAS; this book closes the implementation decisions the HAS deferred.

## 2.1	Document Organization

Note: All chapters linked below for automated document generation.

### 2.1.1	Front Matter

- Document Information

### 2.1.2	Chapter 1: Overview

- Architecture and What the MAS Covers

- Top-Level Port List

- Clocks and Reset

### 2.1.3	Chapter 2: Functional Blocks

- amber_control: Blocking Pipeline FSM

- Pending-Fill Bypass Register

- amber_tag_array and amber_data_array

- amber_repl: Replacement Policy Engine

- amber_victim: Depth-1 Victim Buffer

- amber_fill and amber_drain

- amber_snoop_resp: Snoop Responder Adapter

- amber_ace_issue: Coherent Transaction Mapping

- amber_cpu_frontend and amber_monlite

### 2.1.4	Chapter 3: Interfaces

- CPU GAXI Request/Response Timing

- Fabric AXI4 Master Timing

- Fabric ACE Master Timing

- Snoop AC/CD/CR Timing and Ordering

- MonBus Event Emission Points

### 2.1.5	Chapter 4: MonBus Observation Map

- Event Encodings and Emission Points

- Drop-and-Count Behavior and monbus_tally_axil

### 2.1.6	Chapter 5: Verification Bring-Up

- What the K-maps Prove

- RTL Landing Diff Procedure

- gem5 FSM Cross-Check

### 2.1.7	Chapter 6: Configuration

- Elaboration Configurations

- DV Matrix Dimensions

## 2.2	Quick Reference

### 2.2.1	Core Blocks

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| amber_control | rtl/fub/amber_control.sv | Blocking pipeline FSM, state encoding, miss/replay sequencing | Pre-RTL contract |
| amber_tag_array | rtl/fub/amber_tag_array.sv | Dual-port sdpram_core tag + MESI state store | Pre-RTL contract |
| amber_data_array | rtl/fub/amber_data_array.sv | Dual-port sdpram_core data store | Pre-RTL contract |
| amber_repl | rtl/fub/amber_repl.sv | LRU / tree-PLRU / FIFO / RANDOM replacement | Pre-RTL contract |
| amber_victim | rtl/fub/amber_victim.sv | Depth-1 dirty victim buffer | Pre-RTL contract |
| amber_fill | rtl/fub/amber_fill.sv | Line-fill read master interface | Pre-RTL contract |
| amber_drain | rtl/fub/amber_drain.sv | Dirty victim write-back interface | Pre-RTL contract |
| amber_snoop_resp | rtl/fub/amber_snoop_resp.sv | ACE snoop adapter, CR/CD ordering | Pre-RTL contract |
| amber_ace_issue | rtl/fub/amber_ace_issue.sv | Cache events → ACE transactions | Pre-RTL contract |
| amber_cpu_frontend | rtl/fub/amber_cpu_frontend.sv | GAXI slave request/response latch | Pre-RTL contract |
| amber_monlite | rtl/fub/amber_monlite.sv | Drop-and-count MonBus observer | Pre-RTL contract |

### 2.2.2	Top Modules

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| amber | rtl/top/amber.sv | Pair-rig top, AXI4 memory + ACE snoop | Pre-RTL contract |
| amber_ace | rtl/top/amber_ace.sv | Onyx-rig top, full ACE coherent issue | Pre-RTL contract |
| amber_core | rtl/macro/amber_core.sv | Shared core (both rigs) | Pre-RTL contract |

## 2.3	Related Documentation

| Document | Where | Relationship |
| --- | --- | --- |
| amber PRD | ../../PRD.md | binding requirements — all D-rows and success criteria originate here |
| amber HAS | ../amber_has/amber_has_index.md | architecture specification this MAS implements |
| amber Pre-HAS | ../amber_has/amber_prehas.md | original feature sketches |
| K-map / signal-contract workbook | ../gen_amber_contracts_kmaps.py | pre-RTL combinational-contract sheets and kmaps |
| jet PRD | ../../../jet-mesi-l1/PRD.md | lockup-free follow-on; amber must not pre-decide jet’s J-rows |
| onyx PRD | ../../../onyx-ace-ccu/PRD.md | onyx D7 defines the ACE-shaped snoop port; onyx D2 defines the coherent-transaction subset |
| AMBA ACE Interface Definition | ../../../References/AMBA_ACE_Interface_Definition.md | in-repo ACE contract, verified against IHI 0022H |
| gem5 Ruby protocols | ../../../References/gem5-ruby-protocols/README.md | golden-protocol source for the Python reference model and FSM oracles |
| cache simulator | ../../../../../../bin/apps/cache_sim/README.md | golden parity model for LRU/FIFO/RANDOM |
| Global Requirements | ../../../../../../GLOBAL_REQUIREMENTS.md | reset macros, array syntax, no bespoke SRAM, filelist registry |

Last Updated: 2026-10-06 Maintained By: amber architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	amber — Product Requirements (DRAFT)

Version: 0.5 (draft, 2026-10-06) Status: nine of the eleven decision rows record a name and a date; D3 (memory-side, now shaped by the two-top structure) and D11 (arrays, house primitives by rule) remain OPEN. A decision becomes DECIDED when it records a name and a date, the way the reed-solomon PRD does it. v0.2: D8 decided — *_monlite observation wrappers (never _mon on the measured paths). v0.3: D4 decided — snoop transport is ACE-shaped (AC/CD/CR) per the onyx D7 port contract, family decision 2026-10-05. The snoop-filter (directory-lite) half of D4 stays deferred and is reopened as onyx D3. v0.4: D2 decided (Sean, 2026-10-06) — CPU-side interface is a GAXI slave. House BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach as first real consumer without an adapter. v0.5 (Sean, 2026-10-06): D1, D5, D6, D7, D9, D10 decided in one working session — geometry 32 KiB/64 B/4-way center + a tiny 16-set/2-way formal config; write-back + write-allocate; plain MESI on a 3-bit state field; replacement {LRU, tree-PLRU, FIFO, RANDOM} with LRU default (tree-PLRU the timing fallback, no sim golden model — future sim extension recorded); verification = Pattern-B grids + control-layer SymbiYosys at tiny geometry + cache_sim parity on LRU/FIFO/RANDOM + FSM oracles derived from the gem5 Ruby MESI_Two_Level SLICC tables; first consumer = TB masters bring-up, pair rig the gated deliverable, STREAM attach the documented future hook.

## 3.1	1. Purpose

A parameterised, blocking, snoopy MESI L1 data cache for the Sherpa research/teaching flow: correct against a software golden model, provable with SymbiYosys, observable through the MonBus monitor fabric, and characterised for FPGA cost like every other component in this repo. It is the first cache-ip IP and the deliberate baseline for jet, the lockup-free follow-on.

## 3.2	2. Background

A blocking cache freezes on a miss: the pending request (and everything behind it) waits for the fill to complete. That is poor for performance and excellent for correctness work — one outstanding transaction means no MSHR bookkeeping, no hit-under-miss hazards, no merge logic. MESI (Modified / Exclusive / Shared / Invalid) is the coherence layer on top: each line carries a state that says who may write it and who must be told when they hold a stale copy. Snoopy coherence broadcasts probes on a shared bus; every cache watches and answers.

Blocking and MESI are orthogonal, which is the design bet: amber nails the coherence protocol with trivial miss handling; jet keeps the protocol and upgrades the miss handling. The pair then answers the classic question with measured, on-board data: what does lockup-freedom actually buy?

Repo anchors the design must respect: resets on the ALWAYS_FF_RST macros, storage on the shared sdpram_core / gaxi FIFO primitives (no bespoke SRAM, no reset on memories), [DEPTH] array syntax, valid/ready streaming contracts, and the monitor/MonBus observation fabric from rtl/amba/.

## 3.3	3. Decisions that pick the design

| # | Decision | Candidates | What it drives |
| --- | --- | --- | --- |
| D1 | Cache geometry | DECIDED 2026-10-06 (Sean): 32 KiB / 64 B lines / 4 ways (128 sets) as the default center, fully elaboration-parameterised (4–32 KiB, 32–64 B, 2–8 ways), plus a reserved tiny config — 16 sets / 2 ways / 64 B — for SymbiYosys tractability. Board BRAM does not bind (~9 of 135 36Kb RAMs on Nexys A7 at the default), so geometry serves the DV matrix and the proofs, not the board | tag/data array dimensions, the whole DV matrix |
| D2 | CPU-side interface | DECIDED 2026-10-06 (Sean): GAXI slave. Chosen over the plain valid/ready native port and the AXI4 slave: house BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach as first real consumer without an adapter. Must express single-beat reads, write-allocate fills, and (if write-through is chosen) store-without-allocate | front-end FSM, adapter surface, first consumer |
| D3 | Memory-side interface | OPEN: AXI4 read/write masters on the house axi4_master_rd/wr wrappers (stream/rapids pattern) vs GAXI vs a simple direct SRAM port for bring-up | fill/drain engines, burst behavior on misses |
| D4 | Snoop transport | DECIDED 2026-10-05 (Sean): ACE-shaped transport — amber implements the AC/CD/CR snoop channels exactly as onyx D7 defines them (family decision: cache-ip is ACE-shaped), staying bus-agnostic internally behind a thin adapter. Chosen over a custom dedicated snoop bus and over snooping on the AXI4 fabric directly; the ACE-lite-shaped candidate was set aside because a snoop responder must drive CR responses and CD data — ACE-Lite carries no snoop channels at all. The snoop-filter (directory-lite) sub-question stays deferred, reopened as onyx D3. | the coherence bus, formal surface, gate count |
| D5 | Write policy | DECIDED 2026-10-06 (Sean): write-back + write-allocate. Write-through/no-allocate deletes the coherence traffic the research exists to measure — dirty snoop transfers (PassDirty), WriteBack drains, onyx’s memory-read elision — and leaves MESI’s M state with no job; it stays available as a bring-up mode only | M-state logic, memory traffic, formal targets |
| D6 | MESI variant | DECIDED 2026-10-06 (Sean): plain MESI on a 3-bit state field. MOESI’s O-state is a later elaboration upgrade (encoding headroom reserved now, ~512 extra tag bits); its win — dirty→shared without a memory read — is a measurement for the pair rig, not a v1 requirement | state machine count, snoop response matrix |
| D7 | Replacement policy | DECIDED 2026-10-06 (Sean): policy set {LRU, tree-PLRU, FIFO, RANDOM}, default LRU. LRU/FIFO/RANDOM match the cache simulator exactly (POLICIES = ['LRU','FIFO','RANDOM']), so golden-model parity covers every policy both sides implement; tree-PLRU is carried as the timing-tight fallback — the one policy with no sim golden model, cross-checked by RTL self-checks today and a future cache_sim extension (recorded, not silently assumed). Policy stays an elaboration parameter so the sim↔RTL cross-check holds per policy | policy engine, cross-check fidelity |
| D8 | Observation | DECIDED 2026-10-04 (Sean): *_monlite wrappers, not the heavyweight _mon. MonBus taps for hits, misses (class split if the model supplies it), snoops, evictions, and state transitions, observed through the axi4_monlite / axil4_monlite family — same 128-bit MonBus packets, UNIT/AGENT ids, and host tooling as _mon, ~1/5 the monitor gates, and a drop-and-count policy that never stalls the port. Rationale for a research cache: the observer must not perturb what it measures — a _mon wrapper that gates the port when its tracking tables fill would corrupt the very miss-latency numbers amber and jet exist to produce. The events D8 lists are emitted as MonBus packets and tallied by the standard agents (monbus_tally_axil); heavyweight _mon stays available as a DV cross-check, never on the measured paths. | perf counters, board capture, DV scoreboards |
| D9 | Verification strategy | DECIDED 2026-10-06 (Sean): Pattern-B cocotb GATE/FUNC/FULL grids over the geometry × policy × rig matrix (ACE BFMs: cocotb-framework 1.2.0); SymbiYosys scoped to the control layer — amber_control, amber_snoop_resp, the pending-fill bypass, the victim-buffer handoff — proven at the tiny 16-set/2-way geometry; cache_sim trace-replay parity across LRU/FIFO/RANDOM. The Python reference model and the FSM oracles derive from the gem5 Ruby MESI_Two_Level SLICC tables in ../References/gem5-ruby-protocols/ — L1cache.sm’s enumerated transient states are the derivation source for the amber_pkg state encoding, and every stable and transient transition is cross-checked against an executable protocol spec, not prose | DV structure, formal areas, CI time |
| D10 | First consumer | DECIDED 2026-10-06 (Sean): standalone TB masters for bring-up; the gated deliverable is the pair rig — two ambers + shared memory, the coherence research rig. STREAM attach is documented as the future integration hook (D2’s GAXI choice is what makes it cheap later); the onyx rig is sequenced per onyx D10 — onyx waits behind the pair rig | integration scope, what “done” means |
| D11 | Data/tag arrays | OPEN (direction: shared primitives): sdpram_core-based tag/data stores + house FIFOs for pending/fill queues; FPGA attributes per ../../../../GLOBAL_REQUIREMENTS.md | area/timing headroom, the no-bespoke-SRAM rule |

## 3.4	4. Success criteria

- Golden-model parity. Given the same trace + policy + geometry, amber’s hit/miss/miss-class counts match cache_sim exactly across a regression trace suite.

- Proofs. SymbiYosys: no stale data is ever served after an external write; the coherence protocol cannot deadlock under fair arbitration.

- Observation. Every event class above visible on MonBus in simulation, and captured on at least one FPGA board.

- Cost. Synthesis/implementation numbers (LUT/FF/RAM, fmax) on Nexys A7 and/or Genesys 2, recorded in docs/ the way pumice characterised.

## 3.5	5. Non-goals

Non-blocking / lockup-free miss handling (jet), directory-based coherence, L2/L3 or multi-level inclusion policies, ECC on cache arrays, virtual memory and TLB interaction, multi-port CPU issue. Each is a future IP or a future decision, not an amber requirement.

## 3.6	6. Where the block sits

Research cache on the AMBA fabric: CPU-side slave (D2), memory-side AXI4 masters (D3), snoop port toward onyx / peer caches (D4 — ACE-shaped AC/CD/CR, decided 2026-10-05), MonBus observation (D8). Standalone-testable with TB masters before any consumer exists (D10).

# 4	Global Hard Requirements for RTL Design Sherpa

Version: 1.0 Created: 2025-10-31 Purpose: Consolidated mandatory requirements extracted from all CLAUDE.md files

## 4.1	Overview

This document consolidates all MANDATORY requirements found across repository CLAUDE.md files. These are HARD REQUIREMENTS that must be followed - not suggestions or guidelines.

Sources Analyzed: - Root /CLAUDE.md - projects/components/CLAUDE.md - projects/components/dma-ip/rapids/CLAUDE.md (cited below as rapids/CLAUDE.md) - projects/components/dma-ip/stream/CLAUDE.md (cited below as stream/CLAUDE.md) - rtl/amba/CLAUDE.md (cited below as amba/CLAUDE.md) - rtl/common/CLAUDE.md - CocoTBFramework/CLAUDE.md - the DV framework guide, now maintained in the separate RTLDesignSherpa-DV repo (the framework is editable-installed into the venv)

## 4.2	Category 1: RTL Standards

### 4.2.1	1.1 Reset Handling (MANDATORY - NO EXCEPTIONS)

Requirement: ALL RTL in projects/components/ MUST use standardized reset macros

Pattern:

`include "reset_defs.svh"

`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
    end else begin
        r_state <= w_next_state;
    end
)

NOT Allowed:

// WRONG - Manual reset handling
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
    end else begin
        r_state <= w_next_state;
    end
end

Applies to: ALL synthesisable RTL – rtl/ and projects/ alike. Source: projects/components/CLAUDE.md Rule #0, widened 2026-09-06 (Sean: “All flops should use the global macro”).

Reset is asynchronous on assertion. Always. ALWAYS_FF_RST used to be conditional on USE_ASYNC_RESET and defaulted to SYNCHRONOUS, so make lint (which set the define) and simulation/synthesis (which did not) disagreed about what the design was. The define is now a no-op; passing it is harmless and changes nothing. Deassertion still has to be synchronised externally.

The one exception, and it is structural: rtl/cdc/reset_sync.sv. It is active-HIGH, which the macro cannot express without RESET_ACTIVE_HIGH set globally, so it keeps a raw always_ff. Documented at the flop itself. If you add another, document it there too and say WHY – “it has always been like that” is not a reason.

“Macro-free fork” is not a reason. Two trees claimed it and neither survived contact with evidence. timing_characterization/rtl/asic_only/ was a 37-file duplicate of rtl/ differing ONLY in reset spelling; synthesised through Yosys + slang against ASAP7 it produced a byte-identical mapped netlist (396 cells, 97 flops, every cell type matching), and the flow reading it still passed -DUSE_ASYNC_RESET, a define meaningful only to the macro tree it was cloned from. formal/cdc/cdc_handshake/cdc_handshake_formal.sv claimed Yosys could not take the macro, while 92 other .sby units stage reset_defs.svh and read it with -Iincludes; its own .sby had simply never staged it, and it now proves PASS with the macro. The macros exist to serve ASIC and FPGA both – that is the whole point of them – so a second copy of a module to spell reset differently is duplication, not portability. Deleted 2026-09-07.

Status: rtl/ converted 2026-09-06 (45 blocks / 20 files; the claim that it was “already compliant” was wrong). projects/components/utility-ip/converters/ done as projects/components/utility-ip/converters BUG-007 (was CONV-009). Hand-written RTL still outstanding elsewhere under projects/, plus ~928 blocks in GENERATED files, which must be fixed at their generators and never in the emitted .sv.

### 4.2.2	1.2 FPGA Synthesis Attributes (MANDATORY)

Requirement: ALL memory arrays MUST have FPGA synthesis hints

Pattern:

`ifdef XILINX
    (* ram_style = "auto" *)
`elsif SYNTH_PRAGMA
    /* synthesis ramstyle = "AUTO" */
`endif
logic [DATA_WIDTH-1:0] mem [DEPTH];

Attributes: - Xilinx: ram_style = "auto" | "block" | "distributed" | "ultra" - Quartus: ramstyle = "AUTO" | "M20K" | "MLAB" | "logic"

Applies to: All memory array declarations Source: projects/components/CLAUDE.md Rule #1

### 4.2.3	1.3 Array Syntax Standard (MANDATORY)

Requirement: Use [DEPTH] NOT [0:DEPTH-1]

Pattern:

// CORRECT
logic [DATA_WIDTH-1:0] mem [DEPTH];

// WRONG
logic [DATA_WIDTH-1:0] mem [0:DEPTH-1];

Reason: Iverilog compatibility, cleaner syntax Applies to: All unpacked array declarations Source: projects/components/CLAUDE.md Rule #2

### 4.2.4	1.4 SRAM Module Standards (MANDATORY)

Requirement: SRAM modules DO NOT have reset ports

Pattern:

module simple_sram #(
    parameter int DATA_WIDTH = 32,
    parameter int DEPTH = 1024
) (
    input  logic clk,
    // NO rst_n port!
    // ... other ports
);

Reason: - Real SRAM chips don’t have reset pins - FPGA block RAM doesn’t reset contents - Initialize via writes during system init

Applies to: All SRAM/memory modules Source: projects/components/CLAUDE.md Rule #3

### 4.2.5	1.5 Reset Convention (STANDARD)

Requirement: ALWAYS use active-low asynchronous reset

Signal Names: - rst_n - Active-low reset (projects/components/) - aresetn - Active-low asynchronous reset (AMBA standard) - i_rst_n - Active-low reset (rtl/common/)

NOT Allowed: - rst (positive reset) - reset (ambiguous polarity)

Applies to: All RTL modules Source: Root /CLAUDE.md, rtl/common/CLAUDE.md Rule #3

## 4.3	Category 2: Testbench Architecture

### 4.3.1	2.1 TB Location Requirements (MANDATORY)

Requirement: Project-specific TB classes MUST be in project area, NOT framework

Directory Structure:

CORRECT:
projects/components/{name}/dv/tbclasses/{module}_tb.py

WRONG:
bin/TBClasses/{name}/{module}_tb.py

Import Pattern:

# Add repo root to path
import os, sys
repo_root = os.path.abspath(os.path.join(os.path.dirname(__file__), '../../../../../..'))
sys.path.insert(0, repo_root)

# Import from PROJECT AREA
from projects.components.dma_ip.rapids.dv.tbclasses.scheduler_tb import SchedulerTB

# Shared framework utilities
from TBClasses.shared.tbbase import TBBase
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

Applies to: ALL project-specific testbenches Source: Root /CLAUDE.md, projects/components/CLAUDE.md Rule #4, rapids/CLAUDE.md Rule #0.1, stream/CLAUDE.md Rule #0.1, amba/CLAUDE.md Rule #0

### 4.3.2	2.2 Three Mandatory TB Methods (MANDATORY)

Requirement: EVERY testbench class MUST implement these three methods

Pattern:

class MyModuleTB(TBBase):
    async def setup_clocks_and_reset(self):
        """Complete initialization - starts clocks and performs reset"""
        await self.start_clock('clk', freq=10, units='ns')

        # Set config before reset (if needed)
        self.dut.cfg_param.value = initial_value

        # Reset sequence
        await self.assert_reset()
        await self.wait_clocks('clk', 10)
        await self.deassert_reset()
        await self.wait_clocks('clk', 5)

    async def assert_reset(self):
        """Assert reset signal"""
        self.dut.rst_n.value = 0  # Active-low

    async def deassert_reset(self):
        """Deassert reset signal"""
        self.dut.rst_n.value = 1

Why Required: - Consistency across all testbenches - Reusability for mid-test resets - Clear test structure and intent - Configuration control (some RTL needs signals set BEFORE reset)

Applies to: ALL testbench classes Source: Root /CLAUDE.md, rapids/CLAUDE.md Rule #0.5, stream/CLAUDE.md Rule #0.2, CocoTBFramework/CLAUDE.md Rule #3

### 4.3.3	2.3 TBBase Inheritance (MANDATORY)

Requirement: ALL testbench classes MUST inherit from TBBase

Pattern:

from TBClasses.shared.tbbase import TBBase

class MyModuleTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        # Testbench-specific initialization

What TBBase Provides: - Clock management (start_clock, wait_clocks) - Reset utilities (assert_reset, deassert_reset) - Logging (self.log) - Progress tracking (mark_progress) - Common utilities (convert_to_int, format_dec)

Applies to: ALL testbench classes Source: CocoTBFramework/CLAUDE.md Rule #2

### 4.3.4	2.4 Three-Layer Architecture (MANDATORY)

Requirement: Verification MUST be separated into three layers

Layers:

- Testbench (TB): projects/components/{name}/dv/tbclasses/{module}_tb.py

  - Infrastructure + BFMs

  - Clock/reset control

  - Base test methods

  - NO verification logic

- Scoreboard: bin/TBClasses/scoreboards/{protocol}/{module}_scoreboard.py

  - Verification logic

  - Expected vs actual comparison

  - Coverage collection

- Test: projects/components/{name}/dv/tests/test_{module}.py

  - Test intelligence

  - Imports TB class (does NOT define it)

  - Pytest wrappers

  - Scenario logic

NEVER Embed TB Classes in Test Files

Applies to: ALL verification code Source: amba/CLAUDE.md Rule #0, rapids/CLAUDE.md Rule #0.1, CocoTBFramework/CLAUDE.md Rule #4

### 4.3.5	2.5 Queue-Based Verification (PREFERRED)

Requirement: Use direct queue access for simple tests, NOT memory models

Pattern:

# CORRECT: Direct queue access
aw_pkt = self.aw_monitor._recvQ.popleft()
w_pkt = self.w_monitor._recvQ.popleft()

# Verify
assert aw_pkt.addr == expected_addr
assert w_pkt.data == expected_data

# WRONG: Memory model for simple test
memory_model = MemoryModel()
written_data = memory_model.read(addr, 4)  # Unnecessary complexity

When to Use: - Queue Access: Simple in-order tests, single-master, no overlap - Memory Models: Complex OOO scenarios, multi-master with address overlap

Applies to: Simple in-order verification scenarios Source: rapids/CLAUDE.md Scoreboard Pattern, CocoTBFramework/CLAUDE.md Rule #5

## 4.4	Category 3: Test Standards

### 4.4.1	3.1 Test Naming Convention (MANDATORY)

Requirement: Pytest function MUST match module name

Pattern:

# Module: axi4_dwidth_converter_wr.sv

CORRECT:
def test_axi4_dwidth_converter_wr(request, params):
    """Test for axi4_dwidth_converter_wr.sv"""
    ...

WRONG:
def test_converter(request, params):  # Too generic!
    ...

Why: Prevents pytest collection conflicts between related modules

Applies to: ALL pytest test functions Source: Root /CLAUDE.md

### 4.4.2	3.2 Test Pattern - CocoTB + Pytest (MANDATORY for projects/components/)

Requirement: Use two-tier pattern with cocotb_test_* prefix

Pattern:

# 1. CocoTB test functions - prefix with "cocotb_test_*"
@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic(dut):  # ← cocotb_test_* prefix!
    """CocoTB test function - NOT collected by pytest"""
    tb = SimpleSRAMTB(dut)
    await tb.setup_clocks_and_reset()
    # ... test logic

# 2. Pytest wrapper - calls specific cocotb_test_* function
@pytest.mark.parametrize("addr_width, data_width", params)
def test_basic(request, addr_width, data_width):
    """Pytest wrapper - calls cocotb_test_basic"""
    run(
        # ... setup
        testcase="cocotb_test_basic",  # ← Explicitly specify
        # ... compilation args
    )

Why: - Pytest collects only wrappers, not CocoTB functions - Each wrapper calls ONE specific CocoTB function - Allows parameter sweeps at pytest level

Applies to: ALL tests in projects/components/ Source: Root /CLAUDE.md

### 4.4.3	3.3 100% Success Requirement (MANDATORY)

Requirement: ALL tests MUST achieve 100% success rate

Pattern:

# WRONG: Accepting partial success
assert success_rate >= 70  # "Good enough" - NO!

# CORRECT: Require 100% success
assert success_rate >= 100  # 100% required

Rationale: - RTL is deterministic - 100% success is achievable - Partial success indicates bugs or timing issues - Lower thresholds mask real problems - Tests must reliably catch regressions

Applies to: ALL tests Source: rapids/CLAUDE.md, CocoTBFramework/CLAUDE.md

### 4.4.4	3.4 Simulation Timestamps in Logs (MANDATORY)

Requirement: ALL test log messages MUST include simulation time

Pattern:

# CORRECT: Include simulation timestamp
self.log.info(f"@ {cocotb.utils.get_sim_time('ns')}ns: Beat {i}: data=0x{data:X}, s_ready={ready}, occupancy={occ}")

# WRONG: No simulation timestamp
self.log.info(f"Beat {i}: data=0x{data:X}, s_ready={ready}, occupancy={occ}")

Output Example:

2025-11-06 19:11:20,688 - INFO - @ 150ns: Beat 1: data=0xA000, s_ready=1, occupancy=0
2025-11-06 19:11:20,688 - INFO - @ 160ns: Beat 2: data=0xA001, s_ready=1, occupancy=1
2025-11-06 19:11:20,688 - INFO - @ 170ns: Beat 3: data=0xA002, s_ready=1, occupancy=2
2025-11-06 19:11:20,688 - INFO - @ 180ns: Beat 4 BLOCKED: s_ready=0, occupancy=3

Why Critical: - Correlate test events with waveform timing - Debug timing-sensitive failures - Understand cycle-accurate behavior - Match log messages to specific clock edges

What Counts as “Simulation Time”: - Use cocotb.utils.get_sim_time('ns') for nanoseconds - Use cocotb.utils.get_sim_time('ps') for picoseconds - Format: @ {time}{unit}: at START of message

Anti-Pattern:

# WRONG: Only Python logging timestamp
self.log.info(f"Beat {i} completed")
# Output: "2025-11-06 19:11:20,688 - INFO - Beat 1 completed"
# Problem: Can't correlate with waveform!

Applies to: ALL test log messages showing DUT behavior Priority: P0 - CRITICAL for debugging Source: User feedback 2025-11-06

## 4.5	Category 4: Framework Usage

### 4.5.1	4.1 Search Before Creating (MANDATORY)

Requirement: ALWAYS search existing modules/components BEFORE creating new

Search Commands:

# Search for existing RTL modules
find rtl/{subsystem}/ -name "*.sv" | xargs grep -l "keyword"

# Search for existing BFMs (CocoTBFramework, editable-installed from RTLDesignSherpa-DV)
find "$(python3 -c 'import CocoTBFramework,os;print(os.path.dirname(CocoTBFramework.__file__))')/components" -name "*.py" | head

# Search for existing TB classes
find bin/TBClasses/ -name "*_tb.py"

Decision Tree: - Exact match exists → Use it - Close match exists → Adapt with parameters - No match found → Document search, propose new - Didn’t search → STOP, go back and search

Applies to: RTL modules, BFMs, TB components Source: rtl/common/CLAUDE.md Rule #1, CocoTBFramework/CLAUDE.md Rule #1

### 4.5.2	4.2 BFM Usage for Protocols (MANDATORY for RAPIDS)

Requirement: MUST use framework BFMs for all protocol interfaces

BFM Selection: - Custom valid/ready: GAXI Master/Slave (CocoTBFramework/components/gaxi/) - AXI4: AXI4 Master/Slave (CocoTBFramework/components/axi4/) - APB: APB drivers (CocoTBFramework/components/apb/) - AXI-Stream: AXIS drivers (CocoTBFramework/components/axis4/)

(BFMs come from the RTLDesignSherpa-DV framework repo; shared TB classes in this repo are bin/TBClasses/)

NEVER: - Manually drive valid/ready handshakes - Create custom protocol drivers when framework has them

Applies to: All RAPIDS FUB-level tests Source: rapids/CLAUDE.md Rule #1

## 4.6	Category 5: Documentation

### 4.6.1	5.1 No Emojis in Technical Docs (MANDATORY)

Requirement: No emojis in technical specifications

Rationale: - Break PDF generation tools (LaTeX) - Appear unprofessional in formal documentation - Use plain text for all technical documentation

Exception: User explicitly requests emojis (rare)

Applies to: All technical documentation Source: Root /CLAUDE.md

## 4.7	Category 6: Project-Specific Requirements

### 4.7.1	6.1 Commit Messages Carry No Attribution Trailer

Requirement: A commit message is the change and nothing else. No Co-Authored-By:, no Claude-Session:, no “generated with” line, and no “Documentation and implementation support by Claude.”

Applies to: every commit in this repo, every area, no exceptions. Decided: 2026-08-31 (owner). Supersedes the former per-component attribution formats that lived here as 6.1/6.2 and in six CLAUDE.md files; those required a trailer and contradicted this. Practice already matched this rule – the twenty commits before it was written carried zero trailers – but the rule itself was recorded nowhere in the repo, so every session rediscovered the conflict. It is written down here to stop that.

Not enforced by tooling. No commit-msg hook exists; bin/hooks/pre-commit checks task IDs, declaration order, protocol families, scenario verdicts, doc examples and .sv parse. This one is on the author.

Rationale and the history: vault/handbook/authoring/commit-messages.md.

### 4.7.2	6.2 STREAM Tutorial Focus

Requirement: STREAM has intentional simplifications - DO NOT “fix” them

Intentional Simplifications: 1. Aligned addresses only - No alignment fixup logic 2. Length in beats - Not bytes or chunks 3. No circular buffers - Explicit chain termination only 4. No credit management - Simple transaction limits 5. Pure memory-to-memory - No network interfaces

When users ask “Can we add alignment fixup?” - Correct answer: “STREAM intentionally keeps addresses aligned for tutorial simplicity. For complex alignment, see RAPIDS.” - Wrong answer: “Sure, let me add alignment logic…” (defeats tutorial purpose!)

Applies to: STREAM component only Source: stream/CLAUDE.md Rule #0.1

## 4.8	Summary by Priority

### 4.8.1	P0 - Critical (Enforcement Required)

- Reset macro usage (projects/components/)

- TB location (project area, not framework)

- Three mandatory TB methods

- TBBase inheritance

- Three-layer architecture

- 100% test success requirement

### 4.8.2	P1 - High Priority (Strong Recommendation)

- FPGA synthesis attributes

- Array syntax standard

- SRAM module standards

- Test naming convention

- CocoTB + Pytest pattern

- Queue-based verification

### 4.8.3	P2 - Standard Practice (Best Practices)

- Search before creating

- BFM usage for protocols

- Active-low reset convention

- No emojis in technical docs

### 4.8.4	P3 - Project-Specific

- No attribution trailer on commit messages

- STREAM tutorial focus

## 4.9	Compliance Checklist

Use this checklist when creating new RTL or testbenches:

RTL Module: - [ ] Uses ALWAYS_FF_RST macro (projects/components/) - [ ] Memory arrays have FPGA attributes - [ ] Arrays use [DEPTH] syntax - [ ] SRAM modules have no reset port - [ ] Uses active-low reset (rst_n or aresetn)

Testbench Class: - [ ] Located in projects/components/{name}/dv/tbclasses/ - [ ] Inherits from TBBase - [ ] Implements setup_clocks_and_reset() - [ ] Implements assert_reset() - [ ] Implements deassert_reset() - [ ] Uses framework BFMs (not manual handshaking)

Test File: - [ ] Imports TB class from project area - [ ] CocoTB functions prefixed with cocotb_test_* - [ ] Pytest function matches module name - [ ] Uses testcase= to specify CocoTB function - [ ] Requires 100% success rate - [ ] Uses queue-based verification (for simple tests)

Documentation: - [ ] No emojis in technical specs - [ ] Commit message carries no attribution trailer

## 4.10	Enforcement

These requirements are enforced via:

- Code Review - All PRs reviewed for compliance

- Automated Tools:

  - bin/update_resets.py - Reset macro conversion

  - bin/audit_signal_naming_conflicts.py - Signal naming checks

- CI/CD Checks:

  - Verilator linting

  - Pytest test execution

  - Test success rate validation

- Documentation:

  - CLAUDE.md files per subsystem

  - PRD.md requirements per component

Non-Compliance: - PRs will be REJECTED if they violate P0 requirements - P1 violations require justification and approval - P2/P3 violations should be corrected but may not block PR

## 4.11	References

Main Documentation: - /CLAUDE.md - Repository-wide guidance - /PRD.md - Master requirements - projects/components/CLAUDE.md - Project area standards - CLAUDE.md in the RTLDesignSherpa-DV repo - DV framework patterns (shared TB classes: bin/TBClasses/)

Subsystem Documentation: - projects/components/dma-ip/rapids/CLAUDE.md - RAPIDS guidance - projects/components/dma-ip/stream/CLAUDE.md - STREAM guidance - rtl/amba/CLAUDE.md - AMBA subsystem guidance - rtl/common/CLAUDE.md - Common RTL library guidance

Tools: - bin/update_resets.py - Reset macro conversion - bin/audit_signal_naming_conflicts.py - Signal naming audit - bin/md_to_docx.py - Documentation generation

Version: 1.0 Last Updated: 2025-10-31 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Document Information

Table 0.1: Document information

| Field | Value |
| --- | --- |
| Title | amber — Blocking MESI Snoopy L1 Cache — Hardware Architecture Specification |
| Version | 0.5 |
| Date | 2026-10-06 |
| Status | HAS ratifies the Pre-HAS sketch; PRD decisions D1, D2, D4, D5, D6, D7, D8, D9, D10 are DECIDED. D3 (memory-side shape) and D11 (array construction) remain OPEN and are recorded in the chapters they shape. |
| Scope | amber and amber_ace tops, the shared amber_core, interfaces, parameters, and integration rules for the pair rig and the onyx rig. |
| Not in scope | Lockup-free miss handling (jet), directory-based coherence, L2/L3, ECC, TLBs, multi-port CPU issue, ACE DVM and barrier semantics. |

## 5.1	Related documents

Table 0.2: Related documents

| Document | Where | Relationship |
| --- | --- | --- |
| amber PRD | ../../../PRD.md | binding requirements — all decisions and success criteria originate here |
| amber Pre-HAS | ../amber_prehas.md | architecture sketch ratified by this book |
| jet PRD | ../../../../jet-mesi-l1/PRD.md | lockup-free follow-on; amber must not pre-decide its open rows |
| onyx PRD | ../../../../onyx-ace-ccu/PRD.md | defines the ACE-shaped snoop port (D7) and the coherent-transaction subset (D2) |
| AMBA ACE Interface Definition | ../../../../References/AMBA_ACE_Interface_Definition.md | in-repo ACE contract, verified against IHI 0022H |
| gem5 Ruby protocols | ../../../../References/gem5-ruby-protocols/README.md | source for the Python reference model and FSM oracles |
| cache simulator | ../../../../../../../bin/apps/cache_sim/README.md | golden parity model for LRU/FIFO/RANDOM |
| Global Requirements | ../../../../../../../GLOBAL_REQUIREMENTS.md | reset macros, array syntax, no bespoke SRAM, filelist registry |

## 5.2	Terminology

Pair rig. Two amber instances plus shared memory; snoops travel peer-to-peer on the ACE-shaped port and misses are plain AXI4 to memory (amber D10).

Onyx rig. An amber_ace instance attached to the onyx coherency manager; misses become the onyx D2 coherent-transaction subset and snoops arrive from onyx.

Rig. The top-level instantiation you choose: amber or amber_ace. There is no RIG elaboration parameter; the rig is the top (Pre-HAS Q1, resolved 2026-10-06).

F-item. Feature sketch from the Pre-HAS (F1–F18); this HAS ratifies each decided sketch and records where it remains open.

## 5.3	Revision history

Table 0.3: Revision history

| Version | Date | Change |
| --- | --- | --- |
| 0.5 | 2026-10-06 | Full HAS. Ratifies Pre-HAS F1–F18 against PRD v0.5 decisions, fixes the two-top structure, the module set, the interface pin contracts, the parameter sketch, the verification strategy, and the open D3/D11 questions. |

## 5.4	How to read this book

Chapter 1 states purpose and vocabulary. Chapter 2 is the system shape: use cases, features, context, and one block diagram. Chapter 3 is the architecture — the shared core, data flow, the blocking control FSM, the coherence FSM, and arbitration. Chapter 4 is the pin-level interface inventory. Chapter 5 is the parameter package. Chapter 6 is integration: clock/reset, verification, characterization, and future hooks.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Purpose and Scope

## 6.1	Purpose

amber is a parameterized, blocking, snoopy MESI L1 data cache for the Sherpa research and teaching flow. It is deliberately the simplest correct cache: one outstanding transaction, no MSHRs, no hit-under-miss, no merge logic. That deletion makes it a clean baseline for the follow-on jet lockup-free cache and a tractable target for SymbiYosys proofs and cache-simulator parity.

This document specifies amber’s hardware architecture. It is written after the PRD decisions that fix geometry, write policy, MESI variant, replacement policy, CPU-side interface, snoop transport, observation, verification strategy, and first consumer, but while D3 (memory-side shape) and D11 (array construction) remain open. The open rows are recorded honestly with their current direction and consequences; nothing here is presented as decided that is not.

## 6.2	Scope

In scope:

- The two top-level wrappers, amber (pair rig) and amber_ace (onyx rig), and the shared amber_core.

- CPU-side GAXI slave interface (amber_cpu_frontend).

- Fabric-side plain AXI4 masters under amber and ACE masters plus coherent-issue logic under amber_ace.

- ACE-shaped snoop responder port (amber_snoop_resp) toward a peer cache or onyx.

- Blocking control FSM, pending-fill bypass, victim buffer, fill/drain engines, tag/data arrays, replacement policy engine, and observation wrapper.

- Parameterization, clocking/reset, verification strategy, characterization plan, and future hooks.

Out of scope (named as decisions, not omissions): lockup-free miss handling (jet), directory-based or L2/L3 coherence, ECC on arrays, virtual memory and TLB interaction, multi-port CPU issue, ACE DVM and full barrier semantics, speculative prefetching, and adaptive replacement.

## 6.3	Why a blocking cache matters

Blocking simplifies coherence because there is only one transaction in flight. A probe against a pending fill can be answered from a single bypass register (F9); there is no MSHR hazard matrix, no merge CAM, no response reorder buffer. The research value is the measured delta to jet: with arrays, policy, observation, and the MESI protocol held constant, the difference between the two IPs is strictly the miss-handling concurrency. amber makes that delta isolable.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Document Conventions

## 7.1	Signal naming

- fub_* — the flat upstream-facing side of a house AXI/ACE wrapper or adapter (e.g., fub_axi_araddr, fub_acaddr).

- m_* — the master-facing side toward the fabric or peer (e.g., m_axi_araddr, m_axi_acaddr).

- s_* / i_* / o_* — used for local module ports where the house convention prefers it.

- Active-low resets are named rst_n inside project components and aresetn on AMBA-facing pins, per the repo’s reset convention.

## 7.2	Numbering and bases

- Addresses and data widths are decimal unless prefixed with 0x.

- Bus bit ranges use Verilog-style [MSB:LSB].

- Array dimensions use the house [DEPTH] unpacked syntax, not [0:DEPTH-1].

## 7.3	Citation discipline

- PRD rows are cited as amber Dn (e.g., amber D1).

- Pre-HAS feature sketches are cited as Fn (e.g., F9).

- onyx rows are cited as onyx Dn.

- jet rows are cited as jet Jn.

- OPEN decisions are stated as open, with the current direction and the consequence for the architecture. No TBD or placeholder language is used.

## 7.4	Diagram conventions

- Block diagrams show data flow and module ownership; they are not cycle-accurate timing diagrams.

- Mermaid source is inlined in the chapter and also stored under assets/mermaid/; the trailing source line in each chapter gives the asset path.

## 7.5	Voice

This specification uses first-person-plural engineering voice: “amber does X because Y.” Every architectural claim is traceable to a PRD row, an F-item, a GLOBAL_REQUIREMENTS rule, or a real RTL port list.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Definitions and Acronyms

Table 1.0: Definitions and acronyms

| Term | Definition |
| --- | --- |
| ACE | AXI Coherency Extensions (Arm IHI 0022); the protocol shape used for the snoop port and, in the onyx rig, for coherent fabric transactions. |
| AC / CR / CD | ACE snoop channels: Address, Response, Data. |
| ARSNOOP / AWSNOOP | ACE transaction-type fields on read and write address channels. |
| CRRESP | Five-bit snoop response: DataTransfer, Error, PassDirty, IsShared, WasUnique. |
| MESI | Cache-coherence states: Modified, Exclusive, Shared, Invalid. |
| MOESI | MESI plus Owned state; amber reserves encoding headroom but does not implement O in v1.0. |
| Line | The cache allocation unit, parameterized but defaulting to 64 bytes. |
| Set | The group of ways indexed by the set portion of the address. |
| Way | One associative entry within a set. |
| Blocking | The cache accepts no new CPU request while a miss is outstanding. |
| MSHR | Miss Status Holding Register; amber deliberately has none. |
| Fill | The memory/peer response that writes a missing line into the data array. |
| Drain | The write-back of a dirty victim line to memory or the CCU. |
| Pending-fill bypass | The single register that lets a snoop probe observe the post-fill state while the fill is still in flight (F9). |
| Victim buffer | A small staging buffer for a dirty line waiting to drain; amber uses depth 1. |
| GAXI | The house generic AXI-like valid/ready streaming bus used for the CPU-side port. |
| MonBus | The 128-bit observation packet bus emitted by *_monlite wrappers. |
| Pair rig | Two amber instances plus shared memory, the gated deliverable (D10). |
| Onyx rig | An amber_ace instance attached to the onyx coherency manager. |
| Pattern-B | The repo’s cocotb GATE/FUNC/FULL verification grid naming. |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Use Cases

## 9.1	Primary use case: a correct, observable, provable L1 cache baseline

amber exists so the repo has a first cache IP whose correctness can be checked end-to-end: against a software golden model, against formal proofs, and against hardware cost measurements. The blocking design keeps the coherence protocol in the foreground and the miss-handling mechanics in the background, which is exactly the research posture.

## 9.2	Secondary use cases

- Pair-rig coherence research. Two amber instances plus shared memory form the smallest coherent system in which snoop responses, dirty write-backs, and exclusive-to-shared transitions can be observed and measured (D10).

- Onyx-rig CCU attachment. amber_ace attaches to the onyx coherency manager and exercises the onyx D2 coherent-transaction subset, so the manager side can be built and verified against a real cache (F12).

- STREAM attachment. The GAXI CPU-side port lets STREAM attach without an adapter; this is the documented future integration hook (D2).

- Teaching and trace parity. The cache_sim trace-replay cross-check makes amber a runnable illustration of how geometry and replacement policy combine into hit/miss behavior.

## 9.3	What amber is not designed for

High-throughput single-core execution, multiprocessor systems larger than the pair rig, or any workload that depends on hit-under-miss concurrency. Those belong to jet or to future IPs.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Key Features

Table 2.0: Key features and their decisions

| Feature | What amber does | PRD / F anchor |
| --- | --- | --- |
| Blocking single-outstanding pipeline | One miss at a time; the front end freezes until fill + replay complete. | F1 |
| Parameterized geometry | 4–32 KiB capacity, 32–64 B lines, 2–8 ways; default 32 KiB / 64 B / 4-way (128 sets); tiny 16-set / 2-way config reserved for proofs. | D1 |
| GAXI CPU-side slave | House valid/ready streaming port with skid/FIFO plumbing; STREAM attaches without an adapter. | D2 |
| Write-back + write-allocate | Default policy; write-through/no-allocate retained as a bring-up mode only. | D5 |
| Plain MESI on a 3-bit state field | Stable M/E/S/I; headroom for MOESI O-state as a later elaboration upgrade. | D6 |
| Pluggable replacement | LRU (default), tree-PLRU, FIFO, RANDOM; LRU/FIFO/RANDOM have cache_sim golden parity; tree-PLRU is the timing-tight fallback without a sim golden model. | D7 |
| ACE-shaped snoop responder | AC/CR/CD adapter facing peer or onyx; bus-agnostic inside. | D4 |
| Pending-fill bypass register | Probe-during-fill answered from the register, bounded and state-accurate. | F9 |
| Two tops on one core | amber for the pair rig, amber_ace for the onyx rig; no mode parameter. | F12 |
| MonBus observation | amber_monlite emits 128-bit packets; drop-and-count, never stalls the observed path. | D8 |

The feature set is intentionally narrow. Every item that is not listed is a non-goal: no MSHRs, no prefetch, no ECC, no multi-port issue, no directory filter, no barrier/DVM support. That narrowness is the design.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	System Context

## 11.1	Where amber sits

amber is a cache IP on the AMBA fabric. It consumes GAXI transactions from a CPU or DMA master on one side and issues AXI4 or ACE transactions toward memory or a coherency manager on the other. A third ACE-shaped port receives snoops from a peer cache or from onyx. Observation leaves through a fourth MonBus tap.

## 11.2	Pair rig (amber top)

CPU/STREAM --GAXI--> amber --AXI4--> memory
                        ^--AC/CR/CD-- peer amber

The pair rig is the gated deliverable (D10). Each amber has its own tag/data arrays and its own replacement state. Coherence is snoopy and peer-to-peer: when one cache misses on a line the other holds, the response is delivered through the ACE-shaped snoop port, not through memory. Dirty victims write back to memory over the plain AXI4 master.

## 11.3	Onyx rig (amber_ace top)

CPU/STREAM --GAXI--> amber_ace --ACE--> onyx --AXI4--> memory
                                      ^--AC/CR/CD--

In the onyx rig, amber_ace behaves as an ACE master toward onyx and a snoop responder toward onyx. Coherent read misses become ReadShared or ReadUnique; write promotions become CleanUnique or MakeUnique; dirty evictions become WriteBack; clean evictions become Evict (onyx D2 subset). Memory is on the far side of onyx, not directly reachable by amber_ace.

## 11.4	Shared core, different tops

Both rigs instantiate the same amber_core. The CPU-side front end, the tag/data arrays, the control FSM, the replacement engine, and the snoop responder are identical. Only the fabric master stack and the coherent-transaction issuer differ, and that difference lives in the top wrapper, not in the core.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Block Diagram

Figure 2.1 shows the shared amber_core and the two rig tops that instantiate it. The core is rig-agnostic: the CPU-side front end, control FSM, arrays, replacement engine, fill/drain engines, snoop responder, and observation wrapper are identical in both configurations. The pair-rig top adds plain AXI4 masters to memory; the onyx-rig top adds ACE masters and the amber_ace_issue coherent-transaction issuer.

Diagram 1

Source: 01_block_diagram.mmd — the fence above mirrors it.

## 12.1	Module roles

Table 2.1: Module roles

| Module | Role |
| --- | --- |
| amber | Pair-rig top: amber_core + plain AXI4 masters + snoop responder toward peer. |
| amber_ace | Onyx-rig top: amber_core + ACE masters + amber_ace_issue + snoop responder toward onyx. |
| amber_core | Shared rig-agnostic core instantiated by both tops. |
| amber_cpu_frontend | GAXI slave: request acceptance, response return, replay latch. |
| amber_control | Blocking pipeline FSM; owns tag-array control arbitration and the pending-fill bypass register. |
| amber_tag_array / amber_data_array | Dual-port sdpram_core stores; port A CPU/fill, port B snoop. |
| amber_repl | Pluggable replacement policy engine. |
| amber_victim | Dirty victim buffer + drain trigger; depth 1. |
| amber_fill / amber_drain | Read/write engines; plain AXI4 under amber, ACE under amber_ace. |
| amber_snoop_resp | ACE AC/CR/CD adapter; bus-agnostic inside. |
| amber_ace_issue | Maps cache events to the onyx D2 coherent-transaction subset. |
| amber_monlite | Observation wrapper; emits MonBus packets, never stalls. |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Top Structure and the Two Rigs

## 13.1	No rig parameter

amber does not use a RIG elaboration parameter. The rig is the top you instantiate: instantiate amber for the pair rig, instantiate amber_ace for the onyx rig (F12). Both tops compile; both are exercised in the DV matrix. This keeps the rig-specific wiring out of the shared core and avoids a mode signal that would broaden the formal surface for no benefit.

## 13.2	amber — pair-rig top

amber wraps amber_core with:

- House axi4_master_rd and axi4_master_wr for plain AXI4 memory access (D3 direction).

- The amber_snoop_resp ACE adapter facing the peer cache.

- No coherent-transaction issuer; memory-side misses are ordinary AR/AW bursts.

The snoop responder sees the peer’s AC/CR/CD even though the memory side is plain AXI4. That is exactly the pair-rig shape: coherence is handled cache-to-cache; memory only supplies cold misses and absorbs dirty write-backs.

## 13.3	amber_ace — onyx-rig top

amber_ace wraps the same amber_core with:

- axi4ace_master_rd and axi4ace_master_wr toward onyx.

- amber_ace_issue, which maps cache-side events to the onyx D2 subset: read miss → ReadShared/ReadUnique, write promotion → CleanUnique/MakeUnique, dirty eviction → WriteBack, clean eviction → Evict.

- The same amber_snoop_resp adapter, but facing onyx rather than a peer.

Memory is not directly reachable; all fabric traffic is coherent and routed through onyx.

## 13.4	What is inside amber_core

amber_core contains everything that does not change between rigs: amber_cpu_frontend, amber_control, amber_tag_array, amber_data_array, amber_repl, amber_victim, amber_fill, amber_drain, amber_snoop_resp, and amber_monlite. The fill/drain modules expose a rig-agnostic internal interface; the top selects the AXI4 or ACE transport adapter that drives it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Data Flow: Hits, Misses, Fills, and Drains

## 14.1	Hit path

A CPU request accepted by amber_cpu_frontend is latched and presented to amber_control. The control FSM drives a tag lookup on port A of amber_tag_array. On a hit, the state field is checked: read returns data from amber_data_array port A; write updates data and promotes the line to Modified. The response returns through the front end in the same transaction.

## 14.2	Miss path

On a miss, amber_repl selects the victim way. If the victim is dirty, it is moved to amber_victim and amber_drain issues a write-back burst. Once the victim path is clear, amber_fill issues a read burst. Returned beats write amber_data_array port A; the tag and MESI state are installed; the original request replays from the front-end latch. Figure 3.1 shows the sequence.

Diagram 2

Source: 02_miss_flow.mmd — the fence above mirrors it.

## 14.3	Snoop path

A snoop arrives at amber_snoop_resp, is buffered, and is presented to amber_control. The control FSM reads amber_tag_array port B. If the address matches the pending-fill bypass register, the response is composed from the register’s post-fill state and any fill beats already received. Otherwise the tag state determines CRRESP and whether amber_data_array port B supplies CD beats. The adapter returns CR and CD in order to the requester.

## 14.4	Open shape: memory-side masters (D3)

D3 is open at the time of this writing. The direction is plain AXI4 read/write masters on the house axi4_master_rd/wr wrappers for the pair rig, and the existing axi4ace_master_rd/wr for the onyx rig. The exact skid depths, ID width, and user-width defaults follow the wrappers’ own parameters; the cache does not override them except where geometry or bus width requires it. Finalizing D3 means pinning those defaults and the AW/AR burst length calculation in the fill/drain modules.

## 14.5	Open shape: array construction (D11)

D11 is open: tag and data stores will be built from the shared sdpram_core primitive, with FPGA synthesis attributes per GLOBAL_REQUIREMENTS.md and the [DEPTH] array syntax. The dual-port behavior (port A CPU/fill, port B snoop) is fixed; the exact banking, write-masking, and byte-enable plumbing are micro-architecture details for the MAS.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Control FSM: Blocking Pipeline and Pending-Fill Bypass

## 15.1	The blocking pipeline

amber_control implements the cache’s only state machine. It moves through a fixed sequence per request:

- Lookup — read tag and state for the request’s set on port A.

- Hit service — read or write the data array and return the response.

- Miss resolution — on a miss, ask amber_repl for a victim way.

- Victim drain — if the victim is dirty, stage it in amber_victim and hand it to amber_drain.

- Fill request — launch amber_fill for a whole-line burst.

- Fill completion — write data and install tag + MESI state.

- Replay — re-present the original request from the front-end latch; it now hits.

Because the pipeline is blocking, no new CPU request is accepted while a miss is in flight. That single-outstanding property is what makes the pending-fill bypass tractable.

## 15.2	Pending-fill bypass register

While a fill is outstanding, a snoop may arrive for the same line. amber_control keeps one pending-fill register holding {line_address, state_being_installed, data_available_mask}. A snoop matching the pending line is answered from the register:

- CRRESP reflects the post-fill state (e.g., Shared for a read-shared fill, Exclusive for a read-unique fill, Invalid for a line that will be invalidated by the requester’s MakeUnique).

- Required CD beats are forwarded from the fill stream as the beats arrive, so the response is bounded both for hits and for misses.

The correctness target is state-accuracy: a probe against a pending fill must observe exactly the post-fill state, never pre-fill. This is one of the SymbiYosys proof targets (D9).

## 15.3	FSM oracle derivation

The control FSM’s stable and transient states are derived from the gem5 Ruby MESI_Two_Level SLICC tables, specifically L1cache.sm (D9). The Python reference model and the formal oracles replay the same state transitions, so the protocol is cross-checked against an executable spec rather than prose. MOESI headroom is reserved in the encoding but the FSM v1.0 implements only M/E/S/I.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Coherence FSM and the Snoop Responder

## 16.1	MESI state handling

Each tag entry carries a 3-bit state field. Only M/E/S/I are used in v1.0; the extra bit is reserved for MOESI O-state as a later elaboration upgrade (D6). State transitions follow the gem5 Ruby MESI_Two_Level SLICC tables and are cross-checked by the Python reference model and formal oracles (D9).

Table 3.0: MESI snoop CRRESP matrix — DT=DataTransfer, PD=PassDirty, IS=IsShared, WU=WasUnique. Snoop encodings are the six IHI0022 AC types; CleanUnique and MakeUnique are transactions a master issues (the onyx D2 subset), not snoop encodings, so they do not appear here.

| Current state | Snoop type | Action | Next state | CRRESP (DT/PD/IS/WU/Err) |
| --- | --- | --- | --- | --- |
| Modified | ReadShared | Transfer dirty data, pass dirty, downgrade to Shared | Shared | 1 / 1 / 1 / 0 / 0 |
| Modified | ReadOnce | Transfer dirty data, pass dirty, invalidate | Invalid | 1 / 1 / 0 / 0 / 0 |
| Modified | ReadUnique | Transfer dirty data, pass dirty, invalidate | Invalid | 1 / 1 / 0 / 0 / 0 |
| Modified | CleanShared | Transfer dirty data, pass dirty, downgrade to Shared | Shared | 1 / 1 / 1 / 0 / 0 |
| Modified | CleanInvalid | Transfer dirty data, pass dirty, invalidate | Invalid | 1 / 1 / 1 / 0 / 0 |
| Modified | MakeInvalid | Invalidate without data transfer (IHI0022 forbids DT here) | Invalid | 0 / 0 / 0 / 0 / 0 |
| Exclusive | ReadShared | Transfer clean data, downgrade to Shared | Shared | 1 / 0 / 1 / 1 / 0 |
| Exclusive | ReadOnce | Transfer clean data, downgrade to Shared | Shared | 1 / 0 / 1 / 1 / 0 |
| Exclusive | ReadUnique | Transfer clean data, invalidate | Invalid | 1 / 0 / 0 / 1 / 0 |
| Exclusive | CleanShared | Already clean; no transfer, no change | Exclusive | 0 / 0 / 1 / 1 / 0 |
| Exclusive | CleanInvalid | No data; invalidate | Invalid | 0 / 0 / 0 / 0 / 0 |
| Exclusive | MakeInvalid | No data; invalidate | Invalid | 0 / 0 / 0 / 0 / 0 |
| Shared | ReadShared | No data; keep Shared | Shared | 0 / 0 / 1 / 0 / 0 |
| Shared | ReadOnce | No data; keep Shared | Shared | 0 / 0 / 1 / 0 / 0 |
| Shared | ReadUnique | No data; invalidate | Invalid | 0 / 0 / 0 / 0 / 0 |
| Shared | CleanShared | No data; keep Shared | Shared | 0 / 0 / 0 / 0 / 0 |
| Shared | CleanInvalid | No data; invalidate | Invalid | 0 / 0 / 0 / 0 / 0 |
| Shared | MakeInvalid | No data; invalidate | Invalid | 0 / 0 / 0 / 0 / 0 |
| Invalid | any | No data; miss | Invalid | 0 / 0 / 0 / 0 / 0 |

This matrix is the amber MESI interpretation of the cocotb-framework 1.2.0 ACE BFM default handler, bit for bit — including the release-corrected MakeInvalid behavior on Modified/Owned lines (invalidate, no data transfer, IHI0022 forbids DataTransfer there). Where a textbook MESI next-state could be argued either way (e.g. Exclusive + CleanShared: already clean, so the line stays Exclusive while the response carries IsShared per the family matrix), amber follows the BFM response bits and the gem5 MESI_Two_Level transient-state tables for the transitions. The DV scoreboard checks every CRRESP against this table, and CRRESP.validate_for_snoop is the in-BFM guard for the same rules.

## 16.2	Snoop flow

Figure 3.2 shows the probe path. The ACE adapter decouples ACE signaling from the cache core: inside amber_core the snoop is a bus-agnostic {address, type, response, data} transaction; only amber_snoop_resp speaks ACE.

Diagram 3

Source: 03_snoop_flow.mmd — the fence above mirrors it.

## 16.3	Ordering convention

amber’s adapter completes a transaction’s CD beats before asserting its CR. That is not amber being stricter than the protocol — IHI0022 requires the snoop response only after the final CD beat, and the cocotb-framework 1.2.0 compliance checker (check_cr_order / check_cd_order) enforces exactly this on every gather. The convention matches the family decision recorded with onyx D4 and keeps the gather logic simple to verify.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Arbitration and the Victim Bypass

## 17.1	Dual-port array arbitration

amber_tag_array and amber_data_array are dual-port sdpram_core instances:

- Port A serves CPU lookups, fill writes, and replay reads.

- Port B serves snoop lookups and snoop data readout.

The arrays themselves need no arbitration in the common case because CPU and snoop use separate ports. The arbitration problem is at the control level: the miss FSM and the snoop FSM may both need to advance tag state or start a fill/drain in the same cycle. amber_control resolves this with snoop-priority round-robin: a snoop is never starved, and a CPU miss waits a bounded number of cycles.

## 17.2	Victim bypass

amber_victim is a depth-1 buffer. When a miss evicts a dirty line, the line is staged while the fill is outstanding. If a snoop arrives for that victim line before the drain completes, the buffer can supply the data without re-reading the array. This is the victim bypass. It is shallow by design: depth 1 matches the single-outstanding miss model and keeps the formal surface small.

## 17.3	Liveness target

Under fair arbitration, neither snoops nor CPU misses can deadlock. The SymbiYosys proof set (D9) includes a no-deadlock / no-livelock target scoped to amber_control, amber_snoop_resp, the pending-fill bypass, and the victim handoff, proven at the tiny 16-set / 2-way geometry.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	CPU-Side GAXI Slave

## 18.1	Interface decision

The CPU-side port is a GAXI slave (amber D2). We chose GAXI over a plain valid/ready native port and over an AXI4 slave because the house BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach later without an adapter.

## 18.2	Channel structure

The CPU-side port is two house-GAXI streams (amber D2) — the packed-payload valid/ready pattern of rtl/amba/gaxi/ (gaxi_skid_buffer, gaxi_fifo_sync: wr_valid/wr_ready/wr_data in, rd_valid/rd_ready/rd_data out), which is exactly the house plumbing D2’s rationale points at. amber_cpu_frontend is the slave: it receives the request stream and drives the response stream.

Request stream (CPU → amber, slave input):

| Signal | Direction | Width | Payload |
| --- | --- | --- | --- |
| cpu_req_wr_valid | in | 1 | request valid |
| cpu_req_wr_ready | out | 1 | request accepted |
| cpu_req_wr_data | in | CPU_REQ_W | packed {addr, we, be, wdata} |

Response stream (amber → CPU, slave output):

Table 4.0: CPU-side GAXI slave channels

| Signal | Direction | Width | Payload |
| --- | --- | --- | --- |
| cpu_rsp_rd_valid | out | 1 | response valid |
| cpu_rsp_rd_ready | in | 1 | response accepted |
| cpu_rsp_rd_data | out | CPU_RSP_W | packed {rdata} |

Field packing follows the house field-width convention the AMBA wrappers use (CPU_REQ_W = ADDR_WIDTH + 1 + BUS_WIDTH/8 + BUS_WIDTH, CPU_RSP_W = BUS_WIDTH; the frontend’s field config fixes the bit ranges). If the CPU agent prefers named fields at the boundary, the flattened per-field view (fub_*-style, the way the AMBA wrappers expose their FUB side) carries the same payload — the packed GAXI streams are the contract; naming is RTL bring-up detail.

The frontend latches the accepted request and holds it until amber_control returns the response. Because the cache is blocking, the response stream is naturally in-order and carries no ID tags. Write-allocate fills are read-modify-write sequences inside amber_control; the GAXI slave itself sees only single-beat reads and writes.

## 18.3	No register block

amber is a parameter-only IP. There is no APB CSR or register block. All visibility — configuration, event counting, and debug — comes through the MonBus observation fabric (amber_monlite). This is a deliberate decision, not an omission, and it keeps the CPU-side port the only software-visible surface.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Fabric-Side AXI4 Masters

## 19.1	Where they live

The amber top uses the house axi4_master_rd and axi4_master_wr wrappers for plain AXI4 memory access. These masters are functionally AXI4; they carry no ACE fields and no snoop channels. Fill requests become AR bursts; dirty victim drains become AW/W/B bursts.

## 19.2	Burst behavior

amber issues whole-line fills and whole-line drains. The burst length is LINE_BYTES / (BUS_WIDTH/8) beats. The address is aligned to the line size; ARSIZE and AWSIZE are derived from BUS_WIDTH; ARBURST/AWBURST are INCR. Single-outstanding operation means there is one AR transaction and one AW transaction in flight at any time, so no reorder buffer is needed.

## 19.3	Port summary

The plain AXI4 master stack exposes the standard five AXI4 channels toward memory:

Table 4.1: Plain AXI4 channels in the pair rig

| Channel | Direction from amber | Purpose |
| --- | --- | --- |
| AR | out | read address for fill |
| R | in | read data for fill |
| AW | out | write address for dirty victim drain |
| W | out | write data for dirty victim drain |
| B | in | write response for dirty victim drain |

The exact pin names and widths follow the axi4_master_rd and axi4_master_wr module parameters. The cache supplies address, length, size, and burst type; ID and user fields pass through or are tied to defaults configurable at the top.

## 19.4	Open point: D3

The memory-side interface is D3, still open. The direction is AXI4 on the house wrappers, but the final parameter defaults (ID width, user width, skid depths) and the exact handoff from amber_fill/amber_drain to the wrappers are part of closing D3 in RTL.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Fabric-Side ACE Masters and Coherent Issue

## 20.1	Onyx-rig fabric

amber_ace uses the existing rtl/amba/ace/axi4ace_master_rd and axi4ace_master_wr modules toward onyx. These are full ACE masters with ARSNOOP[3:0] and AWSNOOP[2:0] transaction-type fields and auto-pulsed RACK/WACK outputs.

## 20.2	amber_ace_issue

amber_ace_issue maps cache-side events to the onyx D2 coherent-transaction subset:

Table 4.2: Cache events to ACE transactions

| Cache event | ACE transaction | Snoop field |
| --- | --- | --- |
| read miss, shared intent | ReadShared | fub_axi_arsnoop[3:0] = ReadShared encoding |
| read miss, exclusive intent | ReadUnique | fub_axi_arsnoop[3:0] = ReadUnique encoding |
| write to Shared line | CleanUnique | fub_axi_awsnoop[2:0] = CleanUnique encoding |
| write to Invalid line (whole line) | MakeUnique | fub_axi_awsnoop[2:0] = MakeUnique encoding |
| dirty eviction | WriteBack | fub_axi_awsnoop[2:0] = WriteBack encoding |
| clean eviction | Evict | fub_axi_awsnoop[2:0] = Evict encoding |

## 20.3	Real ACE master read port (axi4ace_master_rd)

The upstream (cache) side uses the fub_axi_* prefix; the master side uses m_axi_*:

Table 4.3: axi4ace_master_rd pin contract

| Signal | Direction | Width | Notes |
| --- | --- | --- | --- |
| fub_axi_arid | in | AXI_ID_WIDTH | upstream AR ID |
| fub_axi_araddr | in | AXI_ADDR_WIDTH | AR address |
| fub_axi_arlen | in | 8 | AR burst length |
| fub_axi_arsize | in | 3 | AR burst size |
| fub_axi_arburst | in | 2 | AR burst type |
| fub_axi_arlock | in | 1 | AR lock |
| fub_axi_arcache | in | 4 | AR cache hint |
| fub_axi_arprot | in | 3 | AR protection |
| fub_axi_arqos | in | 4 | AR QoS |
| fub_axi_arregion | in | 4 | AR region |
| fub_axi_aruser | in | AXI_USER_WIDTH | AR user |
| fub_axi_arsnoop[3:0] | in | 4 | ACE transaction type |
| fub_axi_arvalid | in | 1 | AR valid |
| fub_axi_arready | out | 1 | AR ready |
| fub_axi_rid | out | AXI_ID_WIDTH | R ID |
| fub_axi_rdata | out | AXI_DATA_WIDTH | R data |
| fub_axi_rresp | out | 2 | R response |
| fub_axi_rlast | out | 1 | R last |
| fub_axi_ruser | out | AXI_USER_WIDTH | R user |
| fub_axi_rvalid | out | 1 | R valid |
| fub_axi_rready | in | 1 | R ready |
| m_axi_ar* | out/in | — | master-side AR mirror of fub_axi_ar* |
| m_axi_r* | in/out | — | master-side R mirror of fub_axi_r* |
| m_axi_rack | out | 1 | ACE read acknowledge, auto-pulsed one cycle after last R beat |
| busy | out | 1 | activity status for clock gating |

## 20.4	Real ACE master write port (axi4ace_master_wr)

Table 4.4: axi4ace_master_wr pin contract

| Signal | Direction | Width | Notes |
| --- | --- | --- | --- |
| fub_axi_awid | in | AXI_ID_WIDTH | upstream AW ID |
| fub_axi_awaddr | in | AXI_ADDR_WIDTH | AW address |
| fub_axi_awlen | in | 8 | AW burst length |
| fub_axi_awsize | in | 3 | AW burst size |
| fub_axi_awburst | in | 2 | AW burst type |
| fub_axi_awlock | in | 1 | AW lock |
| fub_axi_awcache | in | 4 | AW cache hint |
| fub_axi_awprot | in | 3 | AW protection |
| fub_axi_awqos | in | 4 | AW QoS |
| fub_axi_awregion | in | 4 | AW region |
| fub_axi_awuser | in | AXI_USER_WIDTH | AW user |
| fub_axi_awsnoop[2:0] | in | 3 | ACE transaction type |
| fub_axi_awvalid | in | 1 | AW valid |
| fub_axi_awready | out | 1 | AW ready |
| fub_axi_wdata | in | AXI_DATA_WIDTH | W data |
| fub_axi_wstrb | in | AXI_WSTRB_WIDTH | W strobe |
| fub_axi_wlast | in | 1 | W last |
| fub_axi_wuser | in | AXI_USER_WIDTH | W user |
| fub_axi_wvalid | in | 1 | W valid |
| fub_axi_wready | out | 1 | W ready |
| fub_axi_bid | out | AXI_ID_WIDTH | B ID |
| fub_axi_bresp | out | 2 | B response |
| fub_axi_buser | out | AXI_USER_WIDTH | B user |
| fub_axi_bvalid | out | 1 | B valid |
| fub_axi_bready | in | 1 | B ready |
| m_axi_aw* / m_axi_w* / m_axi_b* | — | — | master-side AW/W/B mirrors |
| m_axi_wack | out | 1 | ACE write acknowledge, auto-pulsed one cycle after B handshake |
| busy | out | 1 | activity status for clock gating |

RACK and WACK are auto-pulsed inside the adapters because the onyx D2 subset has no ordering semantics that require explicit master-controlled acknowledges.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Snoop Port: AC/CD/CR Adapter

## 21.1	Adapter purpose

amber_snoop_resp is the ACE-shaped snoop responder (amber D4). It decouples the cache core from ACE signaling: the fabric side speaks real ACE AC/CR/CD; the core side is a bus-agnostic {address, type, response, data} transaction.

## 21.2	Real snoop-slave port (axi4ace_snoop_slave)

The module’s external (manager → cache) side uses m_axi_*; the internal (cache FSM) side uses fub_*:

Table 4.5: axi4ace_snoop_slave pin contract

| Signal | Direction | Width | Notes |
| --- | --- | --- | --- |
| m_axi_acaddr | in | ADDR_WIDTH | snoop address |
| m_axi_acsnoop[3:0] | in | 4 | snoop transaction type |
| m_axi_acprot[2:0] | in | 3 | snoop protection |
| m_axi_acvalid | in | 1 | AC valid |
| m_axi_acready | out | 1 | AC ready |
| m_axi_crresp[4:0] | out | 5 | snoop response |
| m_axi_crvalid | out | 1 | CR valid |
| m_axi_crready | in | 1 | CR ready |
| m_axi_cddata | out | DATA_WIDTH | snoop data |
| m_axi_cdlast | out | 1 | CD last beat |
| m_axi_cdvalid | out | 1 | CD valid |
| m_axi_cdready | in | 1 | CD ready |
| fub_acaddr | out | ADDR_WIDTH | internal snoop address |
| fub_acsnoop[3:0] | out | 4 | internal snoop type |
| fub_acprot[2:0] | out | 3 | internal snoop protection |
| fub_acvalid | out | 1 | internal AC valid |
| fub_acready | in | 1 | internal AC ready |
| fub_crresp[4:0] | in | 5 | internal response |
| fub_crvalid | in | 1 | internal CR valid |
| fub_crready | out | 1 | internal CR ready |
| fub_cddata | in | DATA_WIDTH | internal snoop data |
| fub_cdlast | in | 1 | internal CD last |
| fub_cdvalid | in | 1 | internal CD valid |
| fub_cdready | out | 1 | internal CD ready |
| busy | out | 1 | activity status for clock gating |

## 21.3	Ordering and the family convention

Per the in-repo ACE definition, CR and CD must not be asserted before the AC handshake, and responses are returned in the order of the AC addresses. amber strengthens this: CD beats complete before CR is asserted for the same transaction, giving onyx a single closed response at a time. Multiple outstanding snoops are buffered by the skid FIFOs inside the adapter; the cache core processes them one at a time because amber_control is a single FSM.

## 21.4	Bus-agnostic core boundary

The fub_* side is the only boundary the cache core sees. If onyx is later replaced by a real ACE interconnect, only amber_snoop_resp (and the top-level wiring) changes; amber_core does not. That was the reason D4 chose an ACE-shaped transport with a thin adapter.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Observation: MonBus through amber_monlite

## 22.1	Observation decision

amber observes through *_monlite wrappers only, not the heavyweight _mon (amber D8). The rationale is measurement integrity: a _mon wrapper that stalls the port when its tracking tables fill would corrupt the very miss-latency numbers the cache exists to produce. amber_monlite is drop-and-count: packets are emitted when possible, dropped and counted when not, and the observed path never stalls.

## 22.2	MonBus packet content

amber_monlite emits 128-bit MonBus packets with the standard UNIT/AGENT ids. Event classes:

Table 4.6: MonBus event classes

| Event class | Payload highlights |
| --- | --- |
| hit (read/write) | address set, way, MESI state before/after |
| miss | address set, way, miss class (compulsory/capacity/conflict when the golden model supplies it) |
| snoop | snoop type, hit/miss, response class |
| eviction | clean/dirty, address set/way |
| MESI transition | old state, new state, cause |
| fill/drain start+end | address, direction, transaction type |

monbus_tally_axil tallies these packets exactly like every other component in the repo. Event classification uses the same cache_sim per-access miss-class computation: compulsory, capacity, and conflict.

## 22.3	Observer cost

The cost of observation is measured in the characterization sweep (Chapter 6.3): LUT/FF/RAM and fmax are reported with amber_monlite present and with it removed, so the claim “the observer does not perturb what it measures” is quantified.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	amber_pkg and Build-Time Geometry

## 23.1	Parameter philosophy

amber is a parameter-only IP. All geometry, policy, and bus-width choices are elaboration parameters; there is no runtime register block. This matches the decision to put all visibility on MonBus rather than on an APB CSR.

## 23.2	Core parameters

Table 5.0: Core elaboration parameters

| Parameter | Default | Range / set | Meaning |
| --- | --- | --- | --- |
| SETS | 128 | 16–512 | number of sets |
| WAYS | 4 | 2–8 | associativity |
| LINE_BYTES | 64 | 32–64 | cache line size in bytes |
| BUS_WIDTH | 64 | 32–128 | CPU/fabric data width in bits |
| REPL_POLICY | "LRU" | lru, tree_plru, fifo, random | replacement policy |
| WRITE_POLICY | "wb_wa" | wb_wa, wt_na | write-back/write-allocate or write-through/no-allocate bring-up mode |
| VICTIM_DEPTH | 1 | 1–2 | dirty victim buffer depth |

## 23.3	Derived quantities

- SET_INDEX_WIDTH = $clog2(SETS)

- LINE_OFFSET_WIDTH = $clog2(LINE_BYTES)

- TAG_WIDTH = ADDR_WIDTH - SET_INDEX_WIDTH - LINE_OFFSET_WIDTH

- State field adds 3 bits to the tag store (MESI now, MOESI headroom reserved).

- FILL_BEATS = LINE_BYTES / (BUS_WIDTH/8)

## 23.4	Package contents

amber_pkg holds the parameter defaults, derived localparams, MESI state encodings, the replacement-policy enum, the write-policy enum, and the transaction-type encodings used by amber_ace_issue. The FSM oracles and the Python reference model both derive from the same gem5 Ruby MESI_Two_Level SLICC tables, so the encoding in amber_pkg is the single source of truth for RTL and DV.

## 23.5	Rig is not a parameter

There is no RIG parameter. The pair rig and the onyx rig are two separate top modules sharing amber_core (Pre-HAS Q1, resolved 2026-10-06). Elaboration selects the rig by selecting the top file to compile and instantiate.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Clocking and Reset

## 24.1	Clock domain

amber is a single-clock IP. The CPU-side GAXI port, the cache core, the fabric masters, and the snoop adapter all run on aclk. There are no CDC paths inside amber_core; the house AXI/ACE wrappers handle any external CDC if the fabric runs at a different frequency.

## 24.2	Reset

All flops use the ALWAYS_FF_RST macro with active-low asynchronous assertion, per GLOBAL_REQUIREMENTS.md Category 1.1. The reset signal inside project components is rst_n; AMBA-facing pins use aresetn. Memory arrays (sdpram_core-based tag/data stores) have no reset port; their contents are initialized by software or by the first misses after reset.

## 24.3	Reset sequence

After reset deassertion, amber_control starts in the idle state. The tag and data arrays contain undefined values until the first accesses fill them. The front end accepts no request until the control FSM is idle and ready. There is no explicit cache invalidation hardware; invalidation is the natural state of unaccessed lines because the MESI state field is not reset and the first access to any set treats a mismatching/invalid tag as a miss.

## 24.4	Clock gating

The ACE master and snoop adapter modules expose busy outputs. These can be used by the top-level clock-gating cell to shut off the fabric-side clock when the cache is quiescent. The core’s busy signal is amber_control’s non-idle state OR the pending-fill bypass register valid bit OR any skid-buffer occupancy in the adapters.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Verification Strategy

## 25.1	Pattern-B cocotb grids

The primary verification layer is Pattern-B GATE/FUNC/FULL cocotb grids over the geometry × policy × rig matrix (amber D9). The ACE BFMs are the RDS-DV cocotb-framework 1.2.0 ACE extensions: AXI4ACEMasterRead, AXI4ACEMasterWrite, and AXI4ACESnoopSlave/Master.

## 25.2	Golden-model parity

For LRU, FIFO, and RANDOM, amber’s hit/miss/miss-class counts must match the cache_sim trace replay at identical geometry and policy. tree-PLRU has no cache_sim golden model today; it is cross-checked by RTL self-checks and is recorded as a future sim extension (Chapter 6.4). Miss classes follow cache_sim: compulsory, capacity, and conflict.

## 25.3	Formal targets

SymbiYosys is scoped to the control layer and proven at the tiny 16-set / 2-way geometry:

- No stale data served after an external write (pending-fill bypass correctness).

- No deadlock or livelock under fair arbitration.

- Victim-buffer handoff correctness.

- amber_snoop_resp AC/CR/CD ordering.

The proof surface is intentionally narrow: proving the whole cache data path at default geometry is not the goal; proving the control and snoop-responder logic that correctness depends on is.

## 25.4	Python reference model

The Python reference model and the FSM oracles derive from the gem5 Ruby MESI_Two_Level SLICC tables (D9). L1cache.sm’s enumerated transient states are the source for the amber_pkg state encoding. Every stable and transient transition is cross-checked against the executable protocol spec, not prose.

## 25.5	MonBus coverage

Every event class listed in Chapter 4.5 must be visible on MonBus in simulation. The _monlite wrapper is present-vs-absent in the regression so the gate-cost delta is part of the evidence.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Characterization

## 26.1	Board targets

amber is characterized on the same FPGA boards the repo uses for other components: Nexys A7 and Genesys 2. The default geometry (32 KiB / 64 B / 4-way) is expected to consume roughly 9 of the 135 36 Kb BRAMs on the Nexys A7, leaving headroom for the rest of the system.

## 26.2	Sweep dimensions

Table 6.0: Characterization sweep dimensions

| Dimension | Values |
| --- | --- |
| Geometry | default (128 sets / 4 ways / 64 B), tiny (16 sets / 2 ways / 64 B), and boundary points from the 4–32 KiB / 2–8 way range |
| Replacement policy | LRU, tree-PLRU, FIFO, RANDOM |
| Rig | amber top, amber_ace top |
| Observation | amber_monlite present, amber_monlite absent |

## 26.3	Reported metrics

For each point, the characterization records LUT, FF, RAMB36 (and RAMB18 where relevant), and fmax. The report follows the pumice pattern: a markdown table per board, a note on the critical path, and the observer-cost delta. A single characterization run is not the goal; the sweep is, because replacement policy and observation are the variables the research questions depend on.

## 26.4	Critical-path expectation

The expected critical path is through amber_tag_array port A: address decode → tag compare → hit/miss decision → data-array enable. The dual-port split (port B for snoops) removes snoop traffic from the CPU hit path, so the path should be comparable to a non-snoopy cache of the same geometry plus the CRRESP combinatorial decode in amber_control.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Future Hooks

## 27.1	Recorded future work, not blockers

The following items are explicitly out of scope for amber v1.0 but are recorded as hooks so the architecture does not accidentally close them off.

Table 6.1: Future hooks

| Hook | Why it is future work | How amber leaves the door open |
| --- | --- | --- |
| Lockup-free miss handling | The core research delta for jet (J1–J7) | amber_core’s module boundaries are clean; MSHRs replace the pending-fill bypass and the single-outstanding FSM without changing the array or snoop adapter interfaces. |
| MOESI O-state | Measurement for the pair rig, not a v1 requirement | The 3-bit state field in amber_tag_array already reserves the encoding headroom. |
| tree-PLRU cache_sim golden model | Timing-tight policy without a sim parity model today | REPL_POLICY is an elaboration parameter; adding a tree-PLRU model to cache_sim and the DV scoreboard is a model extension, not an RTL change. |
| STREAM attachment | First real consumer after TB masters (D10) | The CPU-side GAXI slave is the attachment point; no adapter needed. |
| Directory-lite snoop filter | Deferred from amber D4, reopened as onyx D3 | The ACE-shaped snoop port is the boundary; a filter can be inserted between amber_snoop_resp and the peer/onyx without touching amber_core. |
| ACE DVM and barriers | Out of scope per onyx D8 | The snoop adapter and amber_ace_issue ignore these transaction types; adding them is an adapter/issuer change. |

## 27.2	Closing D3 and D11

D3 (memory-side interface) and D11 (array construction) remain open. Closing them will produce either a revised HAS or the MAS, depending on whether the decisions stay at architecture level or move into micro-architecture. Until then, the current directions — AXI4/ACE house wrappers and sdpram_core-based dual-port arrays — are the working defaults and the targets for the first RTL cut.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	amber — Pre-Hardware Architecture Specification (Pre-HAS)

Version: 0.3 (draft, 2026-10-06) — superseded by the full HAS (v1.0, same day) once the PRD closed its rows (v0.5): the chapter book ratifies this sketch, and this file is kept as the decision-path record. Status: architecture sketch written while the PRD decisions are still OPEN. This document records no decisions. “PROPOSED” marks a working default put forward so the decision rows can be argued with concretely; a decision becomes DECIDED only when PRD.md records a name and a date. When the decisions close, each PROPOSED item here is either ratified into the full amber_has chapter book (the stream/andesite convention) or overridden and this file edited. v0.2 (2026-10-06): D2 landed in the PRD — CPU-side is a GAXI slave. F9 now answers probe-during-fill with a pending-fill bypass instead of an unbounded stall. Q1 answered — amber_fabric carries both rig attachments and the RIG elaboration parameter generate-selects. v0.3 (2026-10-06): Q1 re-answered by the owner — both rigs as two tops, amber (pair rig) and amber_ace (onyx rig), on a shared amber_core. No RIG parameter: the rig is whichever top you instantiate.

Governing documents: PRD.md (binding) · onyx PRD D7 (snoop-port contract) · jet PRD (what must stay constant) · GLOBAL_REQUIREMENTS.md · cache simulator (golden model)

## 28.1	1. Intent of this document

The PRD carries eleven decision rows and has closed three: D2 (CPU-side is a GAXI slave), D4 (snoop transport is ACE-shaped AC/CD/CR), and D8 (observation through *_monlite wrappers only). The Pre-HAS makes the open rows concrete: for each, it sketches the feature the way a working default would build it, so the architecture can be reviewed before any RTL exists. It is deliberately cheap to change — one file, no chapter skeleton yet.

## 28.2	2. Feature sketches

### 28.2.1	2.1 Core datapath — the blocking bet

- F1 — Blocking single-outstanding pipeline. A miss freezes the front end until the fill completes and the original request replays. No MSHR, no hit-under-miss, no merge logic — that deletion is the entire point; it is jet’s delta (jet J1–J5).

- F2 — Geometry as elaboration parameters (D1 OPEN). PROPOSED default profile 32 KiB / 64 B lines / 4 ways (128 sets) as the DV matrix centre of gravity; ranges 4–32 KiB, 32–64 B, 2–8 ways.

- F3 — Pluggable replacement (D7 OPEN). PROPOSED: true-LRU as the correctness reference (the cache simulator models it exactly), tree-PLRU as the FPGA-area variant; FIFO/random kept because the sim models them too. Policy is an elaboration parameter, never a code hack, so the sim↔RTL cross-check holds for every policy both sides implement.

- F4 — Write policy (D5 OPEN). PROPOSED: write-back + write-allocate — MESI’s M state earns its keep, and dirty-victim handling is where blocking simplicity pays. Write-through/no-allocate retained as a bring-up mode because it deletes the victim path entirely.

- F5 — Whole-line fills. One burst per fill; sub-line fill granularity is a non-goal. Burst length derives from LINE_BYTES / (BUS_WIDTH/8).

### 28.2.2	2.2 Coherence

- F6 — MESI state per line (D6 OPEN). PROPOSED plain MESI; MOESI’s O-state recorded as a later elaboration upgrade, not v1 — ownership’s win (dirty-transfer reduction between caches) is a measurement for the pair rig, not a v1 requirement. Insurance: allocate a 3-bit state field now (MESI needs 2; MOESI needs 3) so the upgrade is an encoding change, not an array rebuild.

- F7 — ACE snoop responder (D4 DECIDED — sketched, not re-decided). amber_snoop_resp: a thin adapter speaking raw ACE AC/CR/CD on the outside per the onyx D7 contract (IHI 0022 the definition) and driving a bus-agnostic probe interface inside. The cache core never contains ACE — only the adapter does; that is exactly why D4 was chosen (onyx, or a real ACE interconnect, can supersede the custom bus without touching cache RTL).

- F8 — Snoop/local arbitration — the culsans problem. Probes must not starve and CPU tag latency must stay bounded. PROPOSED: tag array on the dual-port sdpram_core — port A for CPU lookups, port B for snoop lookups; data array port A for CPU/fill, port B for snoop readout feeding

  - No arbitration stall in the common case; the control collision (miss FSM vs snoop FSM wanting the same array control port) resolved with snoop-priority round-robin inside amber_control.

- F9 — Probe-during-fill, answered by a bypass (direction set 2026-10-06: add the bypass). Blocking means a fill is usually in flight, and a probe against a pending line must not observe pre-fill state. The answer is not an unbounded stall: amber keeps one pending-fill register — single outstanding by construction — recording {line address, state being installed, data availability}. A probe matching it is answered from the register: CRRESP reflects the post-fill state, and required CD beats are forwarded from the fill stream as its beats arrive. A probe that misses it takes the normal tag path. The snoop response is bounded either way. The correctness target sharpens rather than relaxes: the bypass must be state-accurate — a probe against a pending fill observes exactly the post-fill state, never pre-fill — alongside the response-bound liveness target. jet reopens the generalisation (jet J4/J6): multiple outstanding fills turn the register into a pending-fill table, which is jet’s problem, not amber’s.

- F10 — CRRESP response matrix per MESI state, mirroring the RDS-DV ACE BFM default matrix as corrected in cocotb-framework 1.2.0 (MakeInvalid on M/O invalidates without data transfer — the bug the release fixed). The matrix ships as an explicit table in the full HAS.

### 28.2.3	2.3 Fabric side — where the two deployment rigs meet

- F11 — The two rigs. Pair rig (amber D10): two ambers + shared memory, snoops peer-to-peer on the D4 port, misses are plain AXI4 to memory. Onyx rig: misses become coherent transactions toward onyx; onyx owns the memory side and drives the snoops. One cache core, two fabric attachments.

- F12 — Fabric side: two tops on one core (Q1 answered 2026-10-06: do both, as two tops). The rig-agnostic cache core is shared; the fabric difference is confined to the top wrapper and its master stack:

  - amber — the pair-rig top. Misses are plain AXI4 to memory on the house axi4_master_rd/wr wrappers; snoop encodings never driven; the ACE snoop-responder port faces the peer cache.

  - amber_ace — the onyx-rig top. Fabric side is the existing rtl/amba/ace/axi4ace_master_rd / axi4ace_master_wr modules (ACE fields and RACK/WACK auto-pulse already in place), driven by a thin amber_ace_issue coherent-transaction issuer mapping cache events to the onyx D2 subset — read miss → ReadShared / ReadUnique (by desired exclusivity), write promotion → CleanUnique / MakeUnique, dirty eviction → WriteBack, clean eviction → Evict. Memory masters not exposed; the snoop-responder port faces onyx. No parameter selects between them: the rig is whichever top is instantiated. Both tops are compiled, and both are exercised in the DV matrix — amber against a peer + memory environment, amber_ace against an onyx environment (§5).

- F13 — Fill/drain engines (D3 OPEN, direction: house wrappers). Fill = one AR burst, whole line. Drain = a small victim buffer (PROPOSED depth 1)

  - one AW burst. Single outstanding by construction — no reorder buffers.

### 28.2.4	2.4 Observation (D8 DECIDED — sketched, not re-decided)

- F14 — amber_monlite wrapper. MonBus event set: hit (read/write), miss with class split (cold/capacity/conflict, when the golden model supplies the class), snoop (type, hit/miss, response class), eviction (clean/dirty), MESI state transitions, fill/drain start+end. Tallied by monbus_tally_axil exactly like every other component; the wrapper never stalls the observed port — drop-and-count. The observer’s own gate cost is measured present vs absent (F18), so “the observer does not perturb what it measures” is a quantified claim, not an assertion.

### 28.2.5	2.5 Verification & formal surface (D9 OPEN — direction proposed)

- F15 — Pattern-B cocotb grids. GATE/FUNC/FULL over the geometry × policy × rig matrix. Instruments already exist: the RDS-DV AXI4ACESnoopMaster BFM drives amber’s snoop port and AXI4ACEMasterRead/   Write drive the fabric side (cocotb-framework 1.2.0).

- F16 — Golden-model parity. Trace replay from bin/apps/cache_sim/ at identical geometry + policy; hit/miss/miss-class counts must match exactly (PRD success criterion 1). The replacement policy parameter (F3) is what keeps this check meaningful across policies.

- F17 — SymbiYosys targets (PRD criterion 2): no stale data served after an external write (F9 ordering); no deadlock/livelock under fair arbitration. Proof surfaces named now so the RTL is built proof-shaped: amber_control FSM, amber_snoop_resp, tag-array port sharing, victim buffer handoff.

### 28.2.6	2.6 Cost & characterisation (PRD criterion 4)

- F18 — Synthesis sweep. Geometry params × replacement policy on Nexys A7 and/or Genesys 2; LUT/FF/RAM/fmax recorded in docs/ the way pumice characterised. Includes the F14 observer-present vs absent delta.

## 28.3	3. Proposed block architecture

Proposed module set (amber_* prefix, per the cache-ip convention):

| Module | Role | PRD anchor |
| --- | --- | --- |
| amber | Pair-rig top: amber_core + plain AXI4 master stack + snoop responder toward the peer | D10 |
| amber_ace | Onyx-rig top: amber_core + ACE master stack + coherent-transaction issuer + snoop responder toward onyx | D10, F12 |
| amber_core | Shared rig-agnostic core, instantiated by both tops | — |
| amber_cpu_frontend | GAXI slave (D2 decided); request accept, response return, replay latch | D2 |
| amber_control | Blocking pipeline FSM: lookup → hit / miss → evict → fill → replay; owns tag-array control arbitration (F8) and the pending-fill bypass register (F9) | D1, D5 |
| amber_tag_array | sdpram_core tag + state store (3-bit state field, F6) | D11 |
| amber_data_array | sdpram_core data store | D11 |
| amber_repl | Pluggable replacement (policy elaboration parameter) | D7 |
| amber_victim | Dirty victim buffer + drain trigger | D5 |
| amber_fill / amber_drain | AXI4 read/write engines; plain axi4_master_rd/wr under amber, axi4ace_master_rd/wr under amber_ace | D3 |
| amber_snoop_resp | ACE AC/CR/CD adapter; bus-agnostic inside — faces the peer in amber, onyx in amber_ace | D4 |
| amber_ace_issue | Coherent-transaction issuer for the onyx D2 subset — amber_ace only | F12 |
| amber_monlite | Observation wrapper (D8) | D8 |

Miss data flow: frontend accept → tag lookup (port A) → miss → amber_repl picks victim → dirty? amber_victim + drain engine WB → fill engine AR burst → data array write + state install → original request replays from the frontend latch → response.

Snoop data flow: AC arrives at amber_snoop_resp → tag lookup (port B) → line state determines CRRESP and whether CD beats follow → response driven. Conflict with an in-flight fill resolves per F9.

Observation taps sit on control-FSM decision points, not the datapath — the F8 port split means the datapath need not be touched to observe it.

## 28.4	4. Interface sketches (shapes, not pinouts)

| Side | Sketch | Anchor |
| --- | --- | --- |
| CPU-side | DECIDED (PRD D2, 2026-10-06): GAXI slave — house BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach as first real consumer without an adapter. Must express single-beat reads, write-allocate fills, byte enables. | D2 |
| Fabric-side | Two tops, shared core (F12): amber = plain AXI4 masters to memory + AC/CR/CD snoop responder toward the peer; amber_ace = axi4ace_master_rd/wr + amber_ace_issue toward the CCU (onyx D2 subset: ReadShared / ReadUnique / CleanUnique / MakeUnique / WriteBack / Evict) + the same snoop responder toward onyx. The rig is the top you instantiate — no mode parameter. | D3, D4 |
| Observation | MonBus through amber_monlite; no other debug surface. | D8 |
| Register block | None. Parameter-only IP; all visibility through MonBus tallies, unlike every other component’s APB CSR. Recorded explicitly so it is a decision and not an accident. | — |

## 28.5	5. Parameter sketch (illustrative — feeds the DV matrix)

| Parameter | PROPOSED default | Range / set |
| --- | --- | --- |
| SETS | 128 | 16–512 |
| WAYS | 4 | 2–8 |
| LINE_BYTES | 64 | 32–64 |
| REPL_POLICY | lru | lru, tree_plru, fifo, random |
| WRITE_POLICY | wb_wa | wb_wa, wt_na |
| VICTIM_DEPTH | 1 | 1–2 |
| BUS_WIDTH | 64 | 32–128 |

Rig is not a parameter: it is the top you instantiate — amber or amber_ace (F12). Derived: tag width = ADDR_WIDTH − $clog2(SETS) − $clog2(LINE_BYTES) + 3 (state field, F6); fill beats = LINE_BYTES / (BUS_WIDTH/8).

## 28.6	6. Open questions this sketch surfaces

| # | Question | Where it resolves |
| --- | --- | --- |
| Q1 | One mode-parameter amber_fabric vs separate adapters per rig — RESOLVED 2026-10-06: two tops on a shared amber_core — amber (pair rig) and amber_ace (onyx rig); no rig parameter, both compiled, both in the DV matrix (F12) | Full HAS implements it |
| Q2 | GAXI vs plain valid/ready CPU-side — RESOLVED by PRD D2 (2026-10-06, Sean): GAXI slave | amber_cpu_frontend RTL |
| Q3 | Geometry default vs Nexys A7 RAM budget (F2) | With the F18 cost sweep |
| Q4 | 3-bit state field headroom for MOESI (F6) | Cheap now, expensive later — recommend taking the insurance |
| Q5 | Probe-during-fill stall bound — RESOLVED 2026-10-06: add the pending-fill bypass (F9); formal targets become bypass state-accuracy + response bound | Full HAS implements it; PRD criterion 2 depends on it |
| Q6 | Replacement default: LRU reference vs tree-PLRU area (F3) | D7 closure, informed by F18 |

## 28.7	7. What the full HAS will add

The chapter book per the stream/andesite convention: front matter; purpose, conventions, definitions; use cases / key features / system context; graphviz block diagram; data flow; pin-level interfaces; performance targets; integration (clocking, reset, verification). The MAS follows when micro-architecture decisions land (array banking, pipeline staging). This Pre-HAS is the draft those books grow from — sections 2–6 above map onto chapters 2–5.

## 28.8	8. Cross-references

- amber PRD — binding requirements; decision record

- onyx PRD D7/D2 — snoop-port contract and coherent-transaction subset the ONYX rig speaks

- jet PRD — J-rows hold amber constant; nothing here may silently pre-decide jet (F9’s stall policy is amber’s; jet J4/J6 explicitly reopen it)

- References/AMBA_ACE_Interface_Definition.md and References/gem5-ruby-protocols/ — protocol definition and golden-protocol source

- RDS-DV cocotb-framework 1.2.0 ACE BFMs — the DV instruments F15 assumes

Last Updated: 2026-10-06 Maintained By: amber architecture (pre-decision working document)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	jet — Product Requirements (DRAFT)

Version: 0.1 (draft, 2026-10-04) Status: scaffold only — inherits every DECIDED row of amber’s PRD. Decided inherited rows so far: D8 observation (*_monlite, 2026-10-04) and D4 snoop transport (ACE-shaped AC/CD/CR per the onyx D7 contract, 2026-10-05). The rows below are the deltas jet must decide for itself; all are OPEN until they record a name and a date.

## 29.1	1. Purpose

The lockup-free (non-blocking) MESI snoopy L1 cache: amber with MSHRs. The research question is measurement — with coherence, arrays, and observation held constant, what do outstanding misses, hit-under-miss, and miss merging actually buy in latency and bandwidth, in simulation and on board?

## 29.2	2. Decisions jet adds to amber’s

| # | Decision | Candidates | What it drives |
| --- | --- | --- | --- |
| J1 | MSHR organization | OPEN: full associativity keyed by address vs direct-mapped by line index vs CAM of pending lines; capacity 2–16 entries as an elaboration parameter | the miss queue, merge logic, formal surface |
| J2 | Hit-under-miss scope | OPEN: hits to any resident line (default) vs hits only to lines not conflicting with a pending fill vs reads-only (writes still block) | hazard logic between fill port and hit port |
| J3 | Miss merging policy | OPEN: merge only exact-line aliases (default) vs merge line + offset range vs no merging (issue duplicate fetches, dedupe on return) | memory traffic, MSHR occupancy, fairness |
| J4 | Ordering guarantees | OPEN: CPU-side responses in-issue-order vs out-of-order with ID tags; snoop-during-miss handling (a probe against a pending line must see the post-fill state or stall cleanly) | response path complexity, protocol proofs, consumer expectations |
| J5 | Eviction under misses | OPEN: a dirty victim behind an outstanding fill must write back without deadlocking against the fill; victim buffer depth (0 = block like amber, 1–4 = decouple) | write-back path, the classic lockup-free deadlock corner |
| J6 | Congestion/fairness | OPEN: policy when MSHRs are full — stall new misses (strict) vs drop-and-retry vs throttle CPU-side accepts; plus snoop-vs-fill arbitration priority | starvation/liveness proofs |
| J7 | The measured claim | OPEN: define the experiment now — same trace suite + same board as amber, report hit latency distribution, miss concurrency, bandwidth, and MSHR occupancy from MonBus capture | what “jet is faster by X” must mean, reproducibly |

## 29.3	3. Success criteria (additions to amber’s)

- Correctness inheritance. Every amber proof and golden-model parity result still passes with MSHRs present (proofs are re-run, not assumed).

- The delta, measured. On the amber trace suite and at least one board: reported latency/bandwidth improvement with MonBus evidence, and the cache_sim cross-check extended so the model predicts jet’s concurrency numbers too (or the model gap is documented as a finding).

- The deadlock corner, proven. J5/J6 formal targets: no deadlock when MSHRs are full and dirty victims wait behind fills.

## 29.4	4. Non-goals (for jet, beyond amber’s)

Speculative prefetching, adaptive replacement, quality-of-service policies, multi-banked parallelism beyond what MSHRs imply. Each is a future IP or a later decision.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	onyx — Product Requirements (DRAFT)

Version: 0.3 (draft, 2026-10-05) Status: decisions pending — this page records the questions and the candidates, not answers. A decision becomes DECIDED when it records a name and a date, the way the reed-solomon PRD does it. Until then every row below is OPEN. v0.2: D7 decided 2026-10-05 (Sean) — onyx owns the ACE-shaped AC/CD/CR snoop port; amber/jet adapt to it behind a thin adapter. Decided together with amber D4, so the gate between the two is resolved. The sequencing constraint that remains: onyx still waits behind amber’s pair rig (amber D10), its first consumer. v0.3: D10 direction tightened 2026-10-05 — the ACE BFM extension of the RDS-DV axi4 family is now specified concretely (layering, four classes, build order); the row stays OPEN. RDS-DV implementation filed as RTLDesignSherpa-DV issue #86.

## 30.1	1. Purpose

A snoop-based cache coherency unit (CCU) in the shape of Arm ACE: the manager that sits between the repo’s coherent caches and memory on the house AXI4 fabric. An attached cache (amber instance, jet instance) is an ACE master; it issues coherent transactions (ReadShared, ReadUnique, CleanUnique, MakeUnique, WriteBack, Evict) instead of managing peers directly. onyx receives those transactions, broadcasts a snoop (AC request) to the other caches, gathers their CR responses (DataTransfer / IsShared / PassDirty) and CD data, decides whether the transaction may be completed from cache-supplied data alone or memory must be read, and serialises same-address traffic. The research question is the manager side itself: what do fanout, gather, adjudication, and ordering cost — in gates, in latency, and in verification effort.

## 30.2	2. Background

ACE is AXI4 plus three snoop channels (AC, CR, CD) — and this repo already speaks AXI4 (skid buffers, GAXI FIFOs, axi4_master_rd/wr wrappers, Pattern-B BFMs). The natural-protocol argument: every piece of onyx’s data path reuses existing, proven plumbing; the new logic is the coherence decision layer and the serializer. The definition is Arm IHI 0022 (the ACE chapters); the worked example is pulp-platform/culsans (ETH/PlanV), whose two halves are exactly the reading list:

- culsans’ CVA6 coherent WB cache — how a cache becomes a snoop responder: the AC/CD/CR channels, how snoop tag lookups arbitrate with the core’s own tag port, and the MOESI-ish line states.

- ace_ccu_top — the manager: broadcast snoops on a coherent read/write, collect responses, decide whether memory is read, and serialise same-address transactions.

Treat culsans as a worked example, not a dependency (last push 2024, lightly maintained). Note also what onyx is not: it is not a directory, not an L2, and not a general NoC — amber’s deferred “snoop filter / directory-lite” question (amber D4) shows up here only as one candidate row (D3).

Repo anchors the design must respect: resets on the ALWAYS_FF_RST macros, no bespoke SRAM (shared sdpram_core / house FIFO primitives), [DEPTH] array syntax, valid/ready streaming contracts, MonBus observation through *_monlite wrappers only (amber D8’s decision — the observer must never stall the measured paths), filelists registry, and GLOBAL_REQUIREMENTS.

## 30.3	3. Decisions that pick the design

| # | Decision | Candidates | What it drives |
| --- | --- | --- | --- |
| D1 | Functional scope | OPEN: CCU only — N ACE slave-side ports toward caches, one AXI4 master toward memory, one ACE-Lite IO port (the culsans shape; recommended starting scope) vs CCU + address striping / multiple memory ports (interconnect territory; non-goal today) | block boundary, what “onyx done” means |
| D2 | ACE transaction subset | OPEN: research subset — ReadShared / ReadUnique / CleanUnique / MakeUnique / WriteBack / Evict (recommended) vs subset + barrier support vs full ACE incl. DVM | front-end decoder, BFM scope, conformance matrix |
| D3 | Snoop fanout | OPEN: broadcast to all attached caches (culsans shape, simplest, recommended for v1) vs directory-lite snoop filter as an elaboration parameter (amber D4’s deferred question, reopened here) vs snoop filter mandatory | fanout logic, gate count, the coherence-traffic measurements |
| D4 | Response gather | OPEN: fixed response window (all caches respond within N beats, presumed-miss after timeout — simple, needs a proven bound) vs tagged outstanding snoop IDs with per-transaction completion (exact, more bookkeeping) | gather buffer, formal surface, ACE conformance |
| D5 | Same-address serialization | OPEN: per-address scoreboard allowing interleaving of different addresses (recommended; the interesting manager) vs full in-order single outstanding (simplest, strangles concurrency; fine as a bring-up mode) | the serializer, the deadlock/liveness proof, jet J4 interactions |
| D6 | Memory-read elision | OPEN: full adjudication — read memory only when no cache supplied data and no PassDirty (correct ACE semantics; recommended) vs always-read simplification for bring-up | memory traffic, the measurable win onyx exists to quantify |
| D7 | Cache-side interface ownership | DECIDED 2026-10-05 (Sean): onyx defines and owns an AC/CD/CR-shaped snoop port; amber/jet stay bus-agnostic internally and add a thin adapter to it. Chosen over caches natively growing ACE ports — the cache cores stay unpinned from ACE except at the adapter, so onyx (or a real ACE interconnect, e.g. culsans’ ace_ccu_top) can supersede the custom bus without touching cache RTL. | the amber D4 ↔ onyx contract, who changes when ACE supersedes the custom bus |
| D8 | Domain / barrier rules | OPEN: single shareability domain, barriers ignored (recommended non-goal, recorded here so it is a decision and not an accident) vs minimal barrier ordering vs full DVM | scope honesty, later upgrade path |
| D9 | Observation | OPEN (direction: amber D8 pattern): *_monlite taps for coherent-txn start, snoop fanout per transaction, response gather completion, memory-read elision hit/miss, serialization stalls, and WB/Evict drains; tallied by monbus_tally_axil | perf counters, board capture, DV scoreboard |
| D10 | Verification strategy | OPEN (direction: Pattern-B cocotb gate/func/full. ACE BFM — an extension of the axi4 BFM family in RDS-DV, layered the way that family is built (analysis from inspecting CocoTBFramework/components/axi4/): (1) field configs — an AXI4ACEFieldConfigHelper adds ARSNOOP[3:0] / AWSNOOP[2:0] to the AR/AW field configs; (2) signal-map patterns — new axi4ace_* protocol patterns (decide there: framework-shipped vs. user-loadable, the signal_mapping_helper question); (3) transaction/packet layer — the six D2 transaction types as an enum, CRRESP as a bitfield (DataTransfer/Error/PassDirty/IsShared/WasUnique); (4) four BFM classes — AXI4ACEMasterRead / AXI4ACEMasterWrite as subclasses (snoop type on commands; the BFM drives RACK/WACK when playing master against an onyx-port DUT, since the RTL auto-pulse in axi4ace_master_rd/wr only covers DUT-as-master), AXI4ACESnoopSlave (new, no base: receives AC, drives CR/CD in issue order — snoops carry no ID — with programmable cache-state behavior producing the correct CRRESP; this is the golden-model hook that replays gem5 traces through amber), AXI4ACESnoopMaster (new: drives AC, gathers CR/CD in order — drives onyx’s fanout in DV); (5) compliance checker — in-order CR/CD, no snoop to the requesting master, CD-before-CDLAST, CRRESP validity per snoop type; (6) main-repo TB classes under bin/TBClasses/axi4ace/ per consumer. Build order: snoop-slave BFM first (amber’s pair rig needs it), snoop-master BFM lands with onyx; ships as an RDS-DV framework release the way the irq BFM did. Python golden model derived from the gem5 Ruby MESI_Two_Level spec in ../References/gem5-ruby-protocols/; the ACE subset contract is the in-repo definition doc; SymbiYosys proofs of same-address serialisation and no-deadlock under fair arbitration; culsans behaviours as informal cross-checks, not oracles) | DV structure, formal areas, CI time |
| D11 | Master count & profile | OPEN: N coherent cache masters 2–4 as an elaboration parameter (culsans uses 2–4 CVA6) vs fixed 2 for the research rig; line size / profile locked to amber’s geometry vs independently parameterised | array dimensions, DV matrix, the measured system’s realism |

## 30.4	4. Success criteria

- Three-block coherence. Two amber instances (or amber + jet) plus onyx form a coherent system that passes the golden-model parity suite amber D9 defines — the pair rig grown by one block, same traces, same policy space.

- IHI 0022 conformance on the D2 subset. A scoreboard checks the AC/CR/CD behaviour per transaction type: the CR bits (DataTransfer, IsShared, PassDirty) of every gather are correct for the transaction and the caches’ states.

- The serializer, proven. Formal: no two same-address coherent transactions proceed incompatibly; no deadlock under fair arbitration at N ∈ {2, 3, 4} masters; the D4 response-gather mechanism cannot livelock (a presumed-miss bound exists and is proven).

- Observation. Every D9 event class visible on MonBus in simulation and captured on at least one FPGA board.

- Cost. Synthesis/implementation numbers (LUT/FF/RAM, fmax) on Nexys A7 and/or Genesys 2, recorded in docs/ the way pumice characterised.

## 30.5	5. Non-goals

Cache storage in onyx (it is a manager, not an L2 — tempting because a CCU’s gather point is the natural place to cache; explicitly out), directory-based coherence (the D3 snoop-filter candidate is the closest this IP comes), ACE DVM and full barrier semantics (D8), cache stash/dealloc features, multi-CCU / multi-domain systems, and any CPU-side or upstream-master port beyond ACE-Lite IO. Each is a future IP or a future decision, not an onyx requirement.

## 30.6	6. Where the block sits

The fabric’s coherence point, on three kinds of port: cache-side, N ACE-facing ports accepting coherent transactions from amber/jet instances acting as ACE masters (D7, decided 2026-10-05: onyx owns this port’s ACE shape; caches adapt); memory-side, one AXI4 master toward the house memory path (D6 fixes when it is read); inter-cache, the snoop fanout — AC requests driven into each peer cache, CD snoop data and CR snoop responses returned. No CPU port, no register block beyond what observation needs (D9 tallies live in monbus_tally_axil like every other component). Standalone-testable with BFM caches long before amber exists: culsans’ ace_ccu_top demonstrates exactly that decoupling.

# 31	AMBA ACE — Working Interface Definition for cache-ip

Distilled 2026-10-05 for the cache-ip family, then verified clause-by-clause against IHI 0022H on 2026-10-05 (Arm’s direct PDF, linked in §7; fetched to /tmp for comparison, not stored in-tree). Verification confirmed every transaction type, channel payload, CRRESP bit, and the ACE-Lite channel exclusions; it caught and fixed two errors in the ordering rules (the original draft claimed ACE serializes snoops one-at-a-time and mandates CD-before-CR — ACE requires only per-channel ordering and permits multiple outstanding snoops; the stricter behavior is recorded above as our own adapter convention). This document is the in-repo definition of the ACE-shaped port contract referenced by amber D4 and onyx D7 (family decision 2026-10-05: cache-ip is ACE-shaped).

Normative reference: Arm, AMBA AXI and ACE Protocol Specification, IHI 0022 (ACE chapters). Available from Arm without charge (direct PDF in §7); Arm copyright, so deliberately not mirrored here. Where this document and IHI 0022 disagree, IHI 0022 wins — file a docs bug. Worked example: pulp-platform/culsans (ace_ccu_top, Solderpad license) — read its RTL after this page, not before.

## 31.1	1. What ACE is

ACE (AXI Coherency Extensions) is AXI4 extended so that multiple caching masters can share memory coherently. An ACE master is typically a CPU’s cache; an ACE-Lite master is a non-caching agent (DMA, IO) that wants coherent access without being snooped. Between the masters and memory sits a coherency manager — in this family, onyx — which receives coherent transactions, broadcasts snoops to the peer caches, gathers their responses, decides whether DRAM must be read, and serializes same-address traffic.

ACE adds three channels to AXI4. Everything else — AW/W/B/AR/R semantics, burst rules, handshake rules — is unchanged AXI4 and is not restated here.

## 31.2	2. Channel inventory

### 31.2.1	2.1 Coherent master → manager (front side, per master)

The five AXI4 channels, with a transaction-type field added to each address channel:

| Channel | New/changed payload | Notes |
| --- | --- | --- |
| AR + R | ARSNOOP[3:0] | read-side coherent transaction type |
| AW + W + B | AWSNOOP[2:0] | write-side coherent transaction type |

Data and response channels are plain AXI4. A coherent read (e.g. ReadShared) delivers its data on the ordinary R channel, sourced by the manager from a peer cache or from memory as the adjudication decides.

Full ACE master interfaces also carry two acknowledge handshakes outside the five channels: RACK (read acknowledge — master pulses after the last read data beat of each transaction) and WACK (write acknowledge — master pulses after the write response). ACE-Lite excludes both (IHI 0022H §D11.1). Our D2 subset needs no ordering semantics from them; the family convention is to auto-pulse them promptly at the adapter and record that choice in the module docs.

### 31.2.2	2.2 Manager → cache, cache → manager (snoop side, per cache)

Three new channels. Direction below is from the point of view of the cache (amber/jet); the manager (onyx) sees the mirror image.

| Channel | Dir | Payload | Meaning |
| --- | --- | --- | --- |
| AC — snoop address | in | ACADDR, ACSNOOP[3:0], ACPROT[2:0] | “What state does this line have in you, and what should you do about it?” |
| CR — snoop response | out | CRRESP[4:0] | the answer: will you transfer data, is the line shared, is dirty responsibility passing |
| CD — snoop data | out | CDDATA, CDLAST | the line’s data, when CRRESP said it would come |

Handshake rules mirror AXI4 (VALID/READY, no combinatorial dependency of READY on VALID). Ordering rules that matter (IHI 0022H §D3.8–§D3.9, §D5.2, §D6.2):

- CR and CD only after AC. The snoop response and any snoop data must only be asserted after the ACVALID/ACREADY handshake.

- In-order responses, in-order data. CR responses must be returned in the same order as the AC addresses; CD data likewise. The spec permits a cache to have multiple outstanding snoop transactions — serialization is not required by ACE, only per-channel ordering is.

- Family convention (decided with onyx D4): the amber/jet adapter completes a transaction’s CD beats before asserting its CR, so onyx’s gather sees one fully-closed response at a time. This is stricter than ACE requires and simpler to verify; it is our contract, not the spec’s.

- Snoop vs. local access arbitration. A snoop reads the tag array and may evict/change a line’s state, so it must arbitrate with the cache’s own CPU-side tag port. This arbitration point is the classic cache-side cost of becoming a snoop responder (culsans’ CVA6 cache shows one arrangement).

## 31.3	3. Transaction types (the onyx D2 research subset in bold)

Read side (ARSNOOP): ReadOnce, ReadShared, ReadUnique, ReadClean, ReadNotSharedDirty, CleanUnique, MakeUnique, plus barrier/DVM forms (out of scope, onyx D8).

Write side (AWSNOOP): WriteUnique, WriteLineUnique, WriteBack, WriteClean, Evict, plus barrier/DVM forms (out of scope, onyx D8).

The six bolded transactions are the subset onyx D2 recommends; the definition of the subset is: everything amber or jet needs to run MESI write-back coherence against a manager, and nothing that requires DVM or barrier semantics.

Intent of each subset member, one line each:

- ReadShared — fetch a line to read; other caches may keep Shared copies; the line arrives in S (or E if nobody shared it).

- ReadUnique — fetch a line to write; all other copies must be invalidated; if a peer held it dirty, its data (or DRAM’s) is supplied.

- CleanUnique — “I already hold the line; make my copy Unique.” No data transfer; peers with Shared copies drop them.

- MakeUnique — like CleanUnique but the requester does not need the data (it will overwrite the whole line); peers invalidate.

- WriteBack — retire a dirty line to memory; the manager must merge the data into DRAM (or hand it to a subsequent reader).

- Evict — retire a clean line; a hint: no data, no DRAM write needed.

## 31.4	4. The CRRESP decision bits

CRRESP is where a snooped cache answers. The four bits the manager’s adjudication logic consumes (bit 4, WasUnique, also exists per IHI 0022):

| Bit | Name | Meaning when set |
| --- | --- | --- |
| [0] | DataTransfer | “I will drive the line’s data on CD.” Required of a Modified responder (or any responder told to transfer). |
| [1] | Error | “My lookup had an error.” Propagated, not interpreted. |
| [2] | PassDirty | “The line is/was dirty and dirty responsibility moves to whoever receives the data” — set with DataTransfer when the responder passes dirty data, and on its own when the data is already moving elsewhere. The manager uses it to decide whether DRAM must be (re)written. |
| [3] | IsShared | “Another master (or I) may hold a copy.” Tells the manager the requester must not be given Exclusive/Unique state, and that DRAM-read elision is safe when data was supplied. |

The adjudication onyx D6 specifies falls straight out of these bits: a coherent read completes from cache-supplied data alone iff some CR carried DataTransfer; DRAM must be read iff none did and no PassDirty moved dirty responsibility; DRAM must be written iff PassDirty passed dirty data that the requester did not absorb.

## 31.5	5. ACE-Lite

ACE-Lite is the non-snooped half: a master may issue coherent transaction types but never receives AC, so it never drives CR/CD. onyx’s IO port (onyx D1) is ACE-Lite — this is why the earlier “ACE-lite-shaped snoop channel” candidate in amber D4 was set aside: a snoop responder must drive CR/CD, and ACE-Lite has no snoop channels at all.

## 31.6	6. What this family implements, and what it deliberately does not

In scope (amber D4 / onyx D2 subset, single shareability domain, onyx D8): the six transaction types of §3, the AC/CR/CD contract of §2, the CRRESP semantics of §4, broadcast snoop fanout (onyx D3 v1), same-address serialization (onyx D5).

Transport for this contract exists in rtl/amba/ace/ (added 2026-10-05): axi4ace_master_rd/wr and axi4ace_slave_rd/wr (front-side movers with ARSNOOP/AWSNOOP, auto-pulsed RACK/WACK on the masters), the axi4ace_snoop_slave / axi4ace_snoop_master AC/CR/CD movers, and rtl/amba/monitor/axi4ace_snoop_monitor_lite for snoop-side observation.

Out of scope, recorded as decisions and not accidents (onyx D8): DVM (distributed virtual memory) messages, barrier ordering, multiple shareability domains, stash/dealloc hints. The repo’s port contract is ACE-shaped — same channels, signals, payload fields, and ordering rules on the pins — without claiming whole-spec conformance; see onyx D7 for why “shaped” rather than “compliant.”

## 31.7	7. Sources

- Arm, AMBA AXI and ACE Protocol Specification, IHI 0022, ACE chapters — the definition. Current issue H (2020), direct PDF: https://developer.arm.com/-/media/Arm%20Developer%20Community/PDF/IHI0022H_amba_axi_protocol_spec.pdf (landing page: https://developer.arm.com/documentation/ihi0022/). Available from Arm without charge; Arm copyright — cited, not mirrored. Sections this document was verified against: D2.2 (channels), D3.8–D3.9 (signaling, dependencies), D3.20–D3.22 (CRRESP), D5 (snoop transactions), D6.2 (sequencing), D11.1 (ACE-Lite).

- pulp-platform/culsans — rtl/src/ace_ccu_top.sv and the CVA6 coherent cache: a working ACE CCU and a working snoop-responder cache. Worked example, not a dependency. https://github.com/pulp-platform/culsans

- A Primer on Memory Consistency and Cache Coherence (Sorin, Hill, Wood, 2nd ed., stored alongside this file) — the coherence-theory background the transaction types above instantiate.

- gem5 Ruby SLICC specs (gem5-ruby-protocols/, stored alongside) — executable transient-state tables for the same protocol family; the golden-model source material, phrased in messages rather than ACE signals.

# 32	gem5 Ruby protocol specs (reference extract)

Executable, state-table-style cache-coherence protocol specifications in gem5’s SLICC language, extracted verbatim from gem5 master at commit f5c5a6e390f55dd5984977815bf9d0bd05da6945 (2026-09-07) via sparse clone.

Two protocols, as requested for the cache-ip research library:

- MESI_Two_Level — classic snoopy MESI with a shared L2. The L1 cache controller (MESI_Two_Level-L1cache.sm) is the closest executable model to amber: every transient state (IS, IM, SM, S^I, M^I, …) between a stable MESI state and a pending response is enumerated as an explicit state with its own transition table. MESI_Two_Level-msg.sm is the message-type contract (the shape a D4 snoop transport must express); MESI_Two_Level-dir.sm + -L2cache.sm show the point where a passive memory-side agent has to make coherence-visible decisions.

- MOESI_CMP_directory — the directory-based contrast set: what changes when a home/directory agent (not a broadcast medium) owns coherence serialization. Useful for ruling D4 options in or out with a working executable spec instead of prose.

Why these exist here: the Primer’s MESI diagrams show only stable states; most coherence bugs live in the transient states. These files are the enumerated transient-state tables — the raw material for deriving amber’s amber_pkg state encoding and for building the Python reference model the DV scoreboard replays against (amber D9, jet J7 cross-checks).

License: each .sm/.slicc file carries its complete copyright and BSD-3-clause license header inline (ARM Limited; Mark D. Hill and David A. Wood) — redistribution terms are inside the files themselves. LICENSE is gem5’s root file, an unrendered BSD-3 template, included as-shipped.

Upstream: https://github.com/gem5/gem5 (src/mem/ruby/protocol/). This directory is a read-only extract; track upstream for changes.

# 33	Cache Associativity Simulator

A standalone, trace-driven cache simulator for exploring set count, way count, block size, and replacement policy. Written in vanilla HTML5/CSS/ES5-ish JavaScript with no build step; open index.html directly as file://.

## 33.1	Files

bin/apps/cache_sim/
  index.html          -- UI
  style.css           -- mobile-first styles
  js/model.js         -- pure simulation model (no DOM)
  js/app.js           -- DOM glue, generators, and guided example
  test/run_tests.js   -- zero-dependency TAP-ish node test runner
  README.md           -- this file

## 33.2	Model API

var CACHESIM = require('./js/model.js');

var result = CACHESIM.simulate(
  { sets: 4, ways: 1, blockSize: 1, policy: 'LRU' },
  [0x00, 0x10, 0x20, 0x00],
  1   // seed for RANDOM policy / reproducibility
);

simulate(config, addresses, seed) returns:

{
  config: { sets, ways, blockSize, policy },
  perAccess: [
    { addr, hit, setIndex, way, missClass }, ...
  ],
  totals: {
    hits, misses, hitRate,
    compulsory, capacity, conflict
  },
  occupancy: [ [tagOrNull, ...], ... ]   // sets x ways
}

missClass is one of 'hit', 'compulsory', 'capacity', or 'conflict'. Capacity classification is computed by running a shadow fully-associative cache with the same total size and policy.

Other model helpers:

- CACHESIM.validateConfig(config) -> {ok, errors}

- CACHESIM.parseAddressText(text) -> address array (NaN for bad lines)

- CACHESIM.generateSequential(length, start)

- CACHESIM.generateStrided(length, start, stride)

- CACHESIM.generateRandom(length, seed, maxAddr) (default maxAddr = 256)

## 33.3	Running tests

node bin/apps/cache_sim/test/run_tests.js

Exit code is 0 when all tests pass.

## 33.4	UI features

- Sliders for sets, ways, and block size; dropdown for policy.

- Address trace textarea plus one-click sequential, strided, and random generators.

- Big-number hit rate, totals table, per-access log, and per-set occupancy.

- Guided example button that loads a cyclic trace and compares direct-mapped vs 2-way set-associative behavior.