Table of Contents

# Converter Index

Generated: 2025-11-15

# Converters Component Specification

Version: 1.5 Status: Production Ready Last Updated: 2025-11-15

## Document Overview

This specification provides comprehensive documentation for the Converters component, which includes data width conversion and protocol conversion modules for seamless system integration.

Document Structure: - Chapter 1: Overview and Architecture - Chapter 2: Data Width Converters - Chapter 3: Protocol Converters - Chapter 4: Usage Examples and Integration

## Quick Navigation

### Chapter 1: Overview

- 01_introduction.md - Component overview and key features

- 02_architecture.md - System architecture and module hierarchy

- 03_design_philosophy.md - Design decisions and rationale

### Chapter 2: Data Width Converters

- 01_overview.md - Data width conversion overview

- 02_axi_data_upsize.md - Narrow→Wide accumulator

- 03_axi_data_dnsize.md - Wide→Narrow splitter

- 04_dual_buffer_mode.md - High-performance dual-buffer mode

- 05_axi4_dwidth_converter_wr.md - Full write path converter

- 06_axi4_dwidth_converter_rd.md - Full read path converter

### Chapter 3: Protocol Converters

- 01_overview.md - Protocol conversion overview

- 02_axi4_to_apb.md - AXI4-to-APB bridge

- 03_peakrdl_adapter.md - PeakRDL adapter

- 04_axi4_to_axil4.md - AXI4→AXIL4 protocol downgrade

- 05_axil4_to_axi4.md - AXIL4→AXI4 protocol upgrade

- 06_uart_axil_bridge.md - UART-to-AXI4-Lite bridge (planned)

### Chapter 4: Usage Examples

- 01_configuration_guide.md - Parameter configuration guide

- 02_common_scenarios.md - Common use case examples

- 03_integration_patterns.md - System integration patterns

### Appendices

- Appendix A: Design Notes - Design analysis and rationale

  - A.1 APB Converter Analysis

  - A.2 Dual-Buffer Implementation

- Appendix B: Change Logs - Refactoring history and updates

  - B.1 Refactoring Summary

  - B.2 Documentation Update

## Component Summary

### Data Width Converters

Generic Building Blocks: - axi_data_upsize - Accumulates N narrow beats into 1 wide beat (100% throughput) - axi_data_dnsize - Splits 1 wide beat into N narrow beats (80% or 100% throughput)

Full AXI4 Converters: - axi4_dwidth_converter_wr - Complete write path (AW + W + B channels) - axi4_dwidth_converter_rd - Complete read path (AR + R channels)

Block Diagrams:

Figure 1: axi_data_upsize - Narrow-to-Wide Accumulator

Figure 2: axi_data_dnsize - Wide-to-Narrow Splitter (Single Buffer, 80% throughput)

Figure 3: axi_data_dnsize - Wide-to-Narrow Splitter (Dual Buffer, 100% throughput)

### Protocol Converters

AXI4 ↔ AXI4-Lite Converters: - axi4_to_axil4_rd - AXI4→AXIL4 read converter (burst decomposition) - axi4_to_axil4_wr - AXI4→AXIL4 write converter (burst decomposition) - axi4_to_axil4 - Full bidirectional converter (wrapper) - axil4_to_axi4_rd - AXIL4→AXI4 read converter (protocol upgrade) - axil4_to_axi4_wr - AXIL4→AXI4 write converter (protocol upgrade) - axil4_to_axi4 - Full bidirectional converter (wrapper)

Other Protocol Converters: - axi4_to_apb_convert - Full AXI4-to-APB protocol bridge - axi4_to_apb_shim - Simplified AXI4-to-APB wrapper - peakrdl_to_cmdrsp - PeakRDL register to command/response adapter - uart_axil_bridge - UART ASCII command to AXI4-Lite bridge (see rtl/uart_to_axil4/README.md)

Block Diagrams:

Figure 4: axi4_to_apb_convert - Protocol Conversion

Figure 5: peakrdl_to_cmdrsp - Register Interface Adapter

FSM Diagram:

Figure 6: AXI4-to-APB State Machine

## Performance Summary

| Module | Configuration | Throughput | Area | Use Case |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single buffer | 100% | 1× | Narrow→Wide (always optimal) |
| axi_data_dnsize | Single buffer | 80% | 1× | Wide→Narrow (area-efficient) |
| axi_data_dnsize | Dual buffer | 100% | 2× | Wide→Narrow (high-performance) |
| axi4_dwidth_converter_wr | With upsize | 100% | Standard | Full write path |
| axi4_dwidth_converter_rd | With dual dnsize | 100% | +100% | Full read path |
| axi4_to_axil4 | Burst decomposition | 100% (single) / 50% (burst) | ~450 LUTs + FFs | AXI4 master → AXIL4 slave |
| axil4_to_axi4 | Combinational upgrade | 100% | ~110 LUTs | AXIL4 slave → AXI4 fabric |
| axi4_to_apb | Protocol FSM | Sequential | Small | AXI4 → APB peripherals |

## Key Features

### Data Width Conversion

- Flexible width ratios (any integer multiple: 2:1, 4:1, 8:1, 16:1, etc.)

- Configurable sideband handling (concatenate or broadcast)

- Optional burst tracking for LAST signal generation

- Dual-buffer mode for 100% throughput (downsize only)

- Generic building blocks for custom converters

### Protocol Conversion

- AXI4 ↔ AXI4-Lite: Bidirectional protocol conversion

  - Burst decomposition (AXI4→AXIL4) with FSM-based splitting

  - Zero-overhead upgrade (AXIL4→AXI4) with combinational defaults

  - Production ready: 42/42 tests passing

- AXI4-to-APB: Full protocol translation

  - Address width adaptation (64-bit→32-bit)

  - Data width adaptation (configurable)

  - Error response mapping (PSLVERR→BRESP/RRESP)

- PeakRDL adapter: Register interface decoupling

## Documentation Assets

### Graphviz Block Diagrams

Located in assets/graphviz/: - axi_data_upsize.gv/.png - axi_data_dnsize_single.gv/.png - axi_data_dnsize_dual.gv/.png - axi4_dwidth_converter_wr.gv/.png - axi4_dwidth_converter_rd.gv/.png - axi4_to_apb.gv/.png - peakrdl_adapter.gv/.png - axi4_to_axil4.gv/.png (wrapper) - axi4_to_axil4_rd.gv/.png (read path) - axi4_to_axil4_wr.gv/.png (write path) - axil4_to_axi4.gv/.png (wrapper) - axil4_to_axi4_rd.gv/.png (read path) - axil4_to_axi4_wr.gv/.png (write path) - uart_axil_bridge.gv/.png

See assets/graphviz/README.md for regeneration instructions.

### PlantUML FSM Diagrams

Located in assets/puml/: - axi4_to_apb_fsm.puml/.png

See assets/puml/README.md for regeneration instructions.

## Related Documentation

Component-Level: - ../../README.md - Component quick start guide - ../../DUAL_BUFFER_IMPLEMENTATION.md - Dual-buffer feature deep dive - ../../ANALYSIS_APB_CONVERTER.md - APB converter analysis

Repository-Level: - RTL source: projects/components/converters/rtl/ - Test suite: projects/components/converters/dv/tests/ - Usage examples: Chapter 4 of this specification

## Version History

| Version | Date | Description |
| --- | --- | --- |
| 1.5 | 2025-11-15 | Added appendices, UART chapter, 6 new diagrams for AXI4↔AXIL4 converters |
| 1.4 | 2025-11-10 | Refactored AXI4↔AXIL4 wrappers to composition pattern (instantiation vs duplication) |
| 1.3 | 2025-11-06 | Added AXI4↔AXIL4 protocol converters (6 modules, 42 tests passing) |
| 1.2 | 2025-10-26 | Added comprehensive specification with block diagrams |
| 1.1 | 2025-10-25 | Added dual-buffer mode for axi_data_dnsize |
| 1.0 | 2025-10-24 | Initial release with generic and full converters |

## Document Conventions

Notation: - parameter_name - RTL parameters and signals - Bold - Important concepts and module names - Italic - Figure captions and references

Diagrams: - Light blue - Input interfaces - Light coral - Output interfaces - Light yellow - Control logic - Light gray - Data path elements - Arrows - Data/control flow

Author: RTL Design Sherpa Project Maintained By: Converters Component Team Last Review: 2025-11-15

# Chapter 1: Overview - Introduction

## Purpose

The Converters component provides essential data width conversion and protocol conversion modules that enable seamless integration between components with different data widths or communication protocols.

## Problem Statement

Modern SoC designs frequently encounter two integration challenges:

- Data Width Mismatch

  - CPU: 64-bit data bus

  - DDR Controller: 512-bit data bus

  - PCIe Endpoint: 128-bit data bus

  - Result: Direct connection impossible

- Protocol Incompatibility

  - AXI4 masters (CPU, DMA)

  - APB peripherals (UART, GPIO, SPI)

  - Custom register interfaces (PeakRDL)

  - Result: Protocol bridge required

## Solution Approach

### Data Width Converters

Generic Building Blocks: - axi_data_upsize - Accumulates narrow beats into wide beats - axi_data_dnsize - Splits wide beats into narrow beats

Full AXI4 Integration: - axi4_dwidth_converter_wr - Write path converter (AW + W + B) - axi4_dwidth_converter_rd - Read path converter (AR + R)

### Protocol Converters

AXI4 ↔ AXI4-Lite: - axi4_to_axil4_rd - AXI4→AXIL4 read converter (burst decomposition) - axi4_to_axil4_wr - AXI4→AXIL4 write converter (burst decomposition) - axi4_to_axil4 - Full bidirectional converter - axil4_to_axi4_rd - AXIL4→AXI4 read converter (protocol upgrade) - axil4_to_axi4_wr - AXIL4→AXI4 write converter (protocol upgrade) - axil4_to_axi4 - Full bidirectional converter

Other Protocol Converters: - axi4_to_apb_convert - Full AXI4-to-APB bridge - peakrdl_to_cmdrsp - Register interface adapter

## Key Benefits

### Reusability

- Generic modules work with any width ratio

- Configurable parameters for custom use cases

- Self-contained building blocks

### Performance

- Upsize: 100% throughput (single buffer)

- Downsize: 80% (single buffer) or 100% (dual buffer)

- Minimal latency overhead

### Flexibility

- Configurable sideband handling

- Optional burst tracking

- Dual-buffer mode for high-performance paths

## Component Scope

### In Scope

- Integer width ratios (2:1, 4:1, 8:1, 16:1, etc.)

- AXI4 and APB protocol support

- Configurable throughput vs area trade-offs

- Generic building blocks for custom converters

### Out of Scope

- Non-integer width ratios (e.g., 3:2 conversion)

- AXI4-Stream protocol (different component)

- Complex buffering beyond dual-buffer

- Clock domain crossing (use separate CDC modules)

## Target Applications

- CPU-to-DDR Integration - 64-bit CPU to 512-bit memory controller

- DMA Engines - Variable width data movers

- Mixed Protocol Systems - AXI4 to APB peripheral buses

- FPGA Fabric Interfaces - Width matching for IP integration

- Register Access - PeakRDL to custom control protocols

## Document Organization

This specification is organized into four chapters:

- Overview (This chapter) - Purpose, scope, and architecture

- Data Width Converters - Generic and full AXI4 modules

- Protocol Converters - AXI4-to-APB and PeakRDL adapters

- Usage Examples - Configuration, scenarios, and integration

Next: 02_architecture.md - System architecture and module hierarchy

# Chapter 2: Data Width Converters - Overview

## Introduction

Data width converters enable communication between components with mismatched data bus widths, a common challenge in modern SoC designs where different IP blocks operate at different data widths.

## Module Hierarchy

Data Width Converters
├── Generic Building Blocks
│   ├── axi_data_upsize.sv      - Narrow→Wide accumulator
│   └── axi_data_dnsize.sv      - Wide→Narrow splitter
│
└── Full AXI4 Converters
    ├── axi4_dwidth_converter_wr.sv - Write path (AW + W + B)
    └── axi4_dwidth_converter_rd.sv - Read path (AR + R)

## Generic Building Blocks

### axi_data_upsize - Narrow→Wide Accumulator

Purpose: Accumulate N narrow beats into 1 wide beat

Key Features: - 100% throughput (single buffer sufficient) - Configurable sideband modes (concatenate or OR) - Supports any integer width ratio - Minimal area overhead

Block Diagram:

AXI Data Upsize

Documentation: 02_axi_data_upsize.md

### axi_data_dnsize - Wide→Narrow Splitter

Purpose: Split 1 wide beat into N narrow beats

Key Features: - Two modes: Single buffer (80%) or Dual buffer (100%) throughput - Configurable sideband modes (slice or broadcast) - Optional burst tracking for LAST generation - Trade-off: throughput vs area

Block Diagrams:

Single Buffer Mode - 80% throughput, 1× area

Dual Buffer Mode - 100% throughput, 2× area

Documentation: 03_axi_data_dnsize.md | 04_dual_buffer_mode.md

## Full AXI4 Converters

### axi4_dwidth_converter_wr - Write Path Converter

Purpose: Complete write path conversion with AW, W, and B channels

Features: - Address phase management - Burst length adjustment - Skid buffers for flow control - Integrates axi_data_upsize for data conversion

Block Diagram:

AXI4 Write Converter

Documentation: 05_axi4_dwidth_converter_wr.md

### axi4_dwidth_converter_rd - Read Path Converter

Purpose: Complete read path conversion with AR and R channels

Features: - Address phase management - Burst length adjustment - Burst-aware RLAST generation - Integrates axi_data_dnsize for data conversion

Block Diagram:

AXI4 Read Converter

Documentation: 06_axi4_dwidth_converter_rd.md

## Throughput Comparison

| Module | Mode | Throughput | Area | When to Use |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single buffer | 100% | 1× | All narrow→wide conversions |
| axi_data_dnsize | Single buffer | 80% | 1× | Area-constrained designs |
| axi_data_dnsize | Dual buffer | 100% | 2× | High-bandwidth requirements |

Recommendation: - Upsize: Always use single buffer (100% throughput at minimal cost) - Downsize: Choose based on system requirements: - Single buffer for area-constrained or non-continuous traffic - Dual buffer for high-performance DMA or continuous streaming

## Common Configuration Examples

### Example 1: 64→512 bit (Write Path)

axi_data_upsize #(
    .NARROW_WIDTH(64),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(8),   // WSTRB
    .WIDE_SB_WIDTH(64),
    .SB_OR_MODE(0)         // Concatenate
) u_upsize (...);

### Example 2: 512→128 bit (Read Path, High Performance)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(2),     // RRESP
    .NARROW_SB_WIDTH(2),
    .SB_BROADCAST(1),      // Broadcast
    .DUAL_BUFFER(1)        // 100% throughput
) u_dnsize (...);

Next Sections: - 02_axi_data_upsize.md - Detailed upsize module documentation - 03_axi_data_dnsize.md - Detailed dnsize module documentation - 04_dual_buffer_mode.md - Dual-buffer architecture deep dive

# Chapter 3: Protocol Converters - Overview

## Introduction

Protocol converters enable communication between components using different communication protocols, essential for integrating diverse IP blocks in complex SoC designs.

## Available Converters

### 1. AXI4 to AXI4-Lite (Protocol Downgrade)

- Modules: axi4_to_axil4_rd.sv, axi4_to_axil4_wr.sv, axi4_to_axil4.sv

- Purpose: Burst decomposition from full AXI4 to simplified AXI4-Lite

- Features: Multi-beat burst splitting, response aggregation, zero-cycle single-beat passthrough

- Test Status: ✓ 28/28 tests passing

### 2. AXI4-Lite to AXI4 (Protocol Upgrade)

- Modules: axil4_to_axi4_rd.sv, axil4_to_axi4_wr.sv, axil4_to_axi4.sv

- Purpose: Add AXI4 burst signals with default values

- Features: Zero-overhead combinational passthrough, configurable default IDs

- Test Status: ✓ 14/14 tests passing

### 3. AXI4-to-APB Bridge

- Module: axi4_to_apb_convert.sv

- Purpose: Full protocol translation from AXI4 to APB

- Features: Address width adaptation, state machine control, error mapping

### 4. PeakRDL Adapter

- Module: peakrdl_to_cmdrsp.sv

- Purpose: Register interface to command/response protocol

- Features: Protocol decoupling, single-cycle commands, pipelined responses

### 5. UART to AXI4-Lite Bridge

- Module: uart_axil_bridge.sv (plus uart_rx.sv, uart_tx.sv)

- Purpose: ASCII command-line control of AXI4-Lite peripherals via UART

- Features: Human-readable commands, 115200 baud default, timing isolation

- Documentation: See rtl/uart_to_axil4/README.md

## AXI4 ↔ AXI4-Lite Converters

### Overview

Bidirectional protocol conversion between full AXI4 and simplified AXI4-Lite protocols.

Key Features: - Burst decomposition (AXI4→AXIL4): Multi-beat → multiple single-beat - Protocol upgrade (AXIL4→AXI4): Add default burst signals - Independent read/write paths - Data width must match (no width conversion) - Production ready with comprehensive test coverage

### AXI4 → AXI4-Lite (Downgrade)

Converts full AXI4 with burst support to AXI4-Lite single-beat transactions.

Challenges: - Decomposing multi-beat bursts into sequential single beats - Synchronizing AW and W channels (write path) - Aggregating responses across burst - Maintaining ID through burst decomposition

Block Diagram:

AXI4 Master                                    AXI4-Lite Slave
(CPU/DMA)                                      (Peripherals)
   |                                                |
   | ARLEN=3 (4-beat burst)                        |
   | ARADDR=0x1000                                 |
   +--[axi4_to_axil4_rd]-->                        |
                           AR: 0x1000 -------->    |
                           <------ R: data0        |
                           AR: 0x1004 -------->    |
                           <------ R: data1        |
                           AR: 0x1008 -------->    |
                           <------ R: data2        |
                           AR: 0x100C -------->    |
   <-- R: data0,1,2,3      <------ R: data3        |
       RLAST=1                                     |

Performance: - Single-beat: 0 cycles (passthrough) - Multi-beat: 2 × N cycles (N = burst length) - Throughput: 100% (single) / ~50% (burst)

### AXI4-Lite → AXI4 (Upgrade)

Adds AXI4 burst signals with default values for seamless AXIL4 slave integration.

Features: - Combinational logic only (no FSM) - Zero-cycle overhead - Configurable default transaction IDs - All transactions marked as single-beat (LEN=0)

Block Diagram:

AXI4-Lite Slave                                AXI4 Master
(Peripheral)                                   (Interconnect)
   |                                                |
   | Simple interface (addr, data, prot)           |
   +--[axil4_to_axi4]----------------------------> |
                                     ARADDR -----> |
                                     ARPROT -----> |
                                     ARLEN=0 -----> | (default)
                                     ARSIZE=2 ----> | (32-bit)
                                     ARBURST=INCR-> | (default)
                                     ARID=0 -------> | (default)

Performance: - Latency: 0 cycles (combinational) - Throughput: 100% - Area: ~110 LUTs (combinational only)

### Use Cases

AXI4 → AXIL4: 1. CPU with AXI4 burst master → simple AXIL4 peripherals 2. DMA controller → memory-mapped registers 3. High-performance masters → low-complexity slaves

AXIL4 → AXI4: 1. Legacy AXIL4 IP → new AXI4 system 2. Simplified peripheral design → full AXI4 fabric 3. Register blocks → burst-capable interconnect

### Documentation

- 04_axi4_to_axil4.md - Detailed AXI4→AXIL4 specification

- 05_axil4_to_axi4.md - Detailed AXIL4→AXI4 specification

## AXI4-to-APB Bridge

### Overview

Converts AXI4 master transactions (from CPU, DMA) to APB peripheral accesses.

Key Challenges: - Protocol differences (5-channel AXI4 vs 2-phase APB) - Address width mismatch (64-bit AXI4 vs 32-bit APB) - Burst support (AXI4 bursts → sequential APB transactions) - Error response mapping (PSLVERR → BRESP/RRESP)

### Block Diagram

AXI4-to-APB Converter

### State Machine

The converter uses a state machine to manage protocol translation:

AXI4-to-APB FSM

States: - IDLE - Wait for AXI4 transaction - READ - Process AXI4 read → APB read - WRITE - Process AXI4 write → APB write

### Use Cases

- CPU to APB Peripherals - Main processor accessing GPIO, UART, SPI

- DMA to Configuration Registers - DMA controller configuring peripherals

- Mixed Protocol Systems - Integrating AXI4 fabric with legacy APB devices

### Documentation

See 02_axi4_to_apb.md for detailed specification.

## PeakRDL Adapter

### Overview

Converts PeakRDL-generated register interface to a custom command/response protocol, enabling protocol decoupling and flexible register implementations.

Key Features: - APB-style register interface (input) - Command/response handshake (output) - Configurable address and data widths - Single-cycle command issue

### Block Diagram

PeakRDL Adapter

### Interface Types

Register Interface (APB-style): - reg_addr[ADDR_WIDTH-1:0] - Register address - reg_wdata[DATA_WIDTH-1:0] - Write data - reg_write - Write enable - reg_read - Read enable - reg_rdata[DATA_WIDTH-1:0] - Read data - reg_error - Error flag

Command/Response Protocol: - Command: valid/ready handshake with addr, data, write flag - Response: valid/ready handshake with data, error flag

### Use Cases

- PeakRDL Register Blocks - Decoupling register interface from implementation

- Custom Control Logic - Flexible register access mechanism

- Testbench Stimulus - Command-driven register access in verification

### Documentation

See 03_peakrdl_adapter.md for detailed specification.

## Comparison

| Feature | AXI4→AXIL4 | AXIL4→AXI4 | AXI4→APB | PeakRDL Adapter |
| --- | --- | --- | --- | --- |
| Direction | Protocol downgrade | Protocol upgrade | Protocol bridge | Interface adapter |
| Input Protocol | AXI4 (5 ch, bursts) | AXIL4 (5 ch, single) | AXI4 (5 ch, bursts) | APB-style register |
| Output Protocol | AXIL4 (5 ch, single) | AXI4 (5 ch, bursts) | APB (2 phases) | Command/response |
| Complexity | Medium (FSM, decomposition) | Very Low (combinational) | High (FSM, burst, APB) | Low (pass-through) |
| Latency | 0 (single) / 2×N (burst) | 0 cycles | 3-5 cycles per txn | 1 cycle |
| Throughput | 100% (single) / 50% (burst) | 100% | Sequential | 100% |
| Area | ~450 LUTs + FFs | ~110 LUTs | High (FSM + state) | Low |
| Use Case | Burst master → simple slave | Simple slave → full fabric | AXI4 → APB peripherals | Register decoupling |
| Test Status | ✓ 28/28 passing | ✓ 14/14 passing | In development | Production |

## Design Considerations

### When to Use Protocol Converters

Use AXI4→AXIL4 when: - CPU/DMA with burst support needs to access simple AXIL4 peripherals - Want to simplify peripheral design (no burst handling) - Need automatic burst decomposition - Data widths match between master and slave

Use AXIL4→AXI4 when: - Legacy AXIL4 IP needs to connect to AXI4 fabric - Designing simple peripheral for AXI4 system - Want zero-overhead protocol upgrade - Don’t need burst capability

Use AXI4-to-APB when: - Integrating AXI4 masters with APB peripherals - Building CPU-to-peripheral bridges - System has mixed protocol requirements

Use PeakRDL adapter when: - Decoupling register interface from implementation - Need flexible register access protocol - Building custom control/configuration logic

### Integration Guidelines

- Address Map Planning - Ensure non-overlapping regions

- Error Handling - Map error responses appropriately

- Performance Analysis - Consider latency impact

- Testing Strategy - Verify protocol compliance

Next Sections: - 02_axi4_to_apb.md - Detailed AXI4-to-APB specification - 03_peakrdl_adapter.md - Detailed PeakRDL adapter specification - 04_axi4_to_axil4.md - Detailed AXI4→AXIL4 specification - 05_axil4_to_axi4.md - Detailed AXIL4→AXI4 specification

# AXI4 to AXI4-Lite Protocol Converter

## Overview

The AXI4-to-AXI4-Lite converter provides protocol downgrade from full AXI4 to simplified AXI4-Lite by decomposing multi-beat bursts into multiple single-beat transactions.

Available Modules: - axi4_to_axil4_rd.sv - Read-only converter (AR + R channels) - axi4_to_axil4_wr.sv - Write-only converter (AW + W + B channels) - axi4_to_axil4.sv - Full bidirectional converter (wrapper)

## Key Features

### Protocol Translation

- Burst Decomposition - Multi-beat bursts → multiple single-beat transactions

- Signal Removal - Drops AXI4-specific signals (ID, USER, REGION, QOS)

- Burst Types - Supports FIXED, INCR, WRAP (all converted to single beats)

- Response Aggregation - Collects worst-case error across burst

- ID Preservation - Returns responses with original transaction ID

### Performance

- Single-Beat Passthrough - Zero-cycle overhead for single transactions (ARLEN/AWLEN = 0)

- Pipelined Decomposition - Minimizes latency for multi-beat bursts

- Independent Channels - AW and W can complete in any order (write path)

### Design Characteristics

- Data Width Match - Input and output data widths must be identical

- No Width Conversion - Pure protocol translation only

- Configurable Skid Buffers - Optional timing closure helpers

## Read Converter (axi4_to_axil4_rd)

### Module Parameters

module axi4_to_axil4_rd #(
    parameter int AXI_ID_WIDTH      = 8,      // Transaction ID width (1-16)
    parameter int AXI_ADDR_WIDTH    = 32,     // Address bus width (12-64)
    parameter int AXI_DATA_WIDTH    = 32,     // Data bus width (must match)
    parameter int AXI_USER_WIDTH    = 1,      // User signal width (0-1024)
    parameter int SKID_DEPTH_AR     = 2,      // Address skid depth (2-8)
    parameter int SKID_DEPTH_R      = 4       // Response skid depth (2-8)
) (
    input  logic                        aclk,
    input  logic                        aresetn,

    // Slave AXI4 Read Interface (Input - Full Protocol)
    input  logic [AXI_ID_WIDTH-1:0]     s_axi_arid,
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axi_araddr,
    input  logic [7:0]                  s_axi_arlen,
    input  logic [2:0]                  s_axi_arsize,
    input  logic [1:0]                  s_axi_arburst,
    input  logic                        s_axi_arlock,
    input  logic [3:0]                  s_axi_arcache,
    input  logic [2:0]                  s_axi_arprot,
    input  logic [3:0]                  s_axi_arqos,
    input  logic [3:0]                  s_axi_arregion,
    input  logic [AXI_USER_WIDTH-1:0]   s_axi_aruser,
    input  logic                        s_axi_arvalid,
    output logic                        s_axi_arready,

    output logic [AXI_ID_WIDTH-1:0]     s_axi_rid,
    output logic [AXI_DATA_WIDTH-1:0]   s_axi_rdata,
    output logic [1:0]                  s_axi_rresp,
    output logic                        s_axi_rlast,
    output logic [AXI_USER_WIDTH-1:0]   s_axi_ruser,
    output logic                        s_axi_rvalid,
    input  logic                        s_axi_rready,

    // Master AXI4-Lite Read Interface (Output - Simplified Protocol)
    output logic [AXI_ADDR_WIDTH-1:0]   m_axil_araddr,
    output logic [2:0]                  m_axil_arprot,
    output logic                        m_axil_arvalid,
    input  logic                        m_axil_arready,

    input  logic [AXI_DATA_WIDTH-1:0]   m_axil_rdata,
    input  logic [1:0]                  m_axil_rresp,
    input  logic                        m_axil_rvalid,
    output logic                        m_axil_rready
);

### Burst Decomposition Algorithm

Single Beat (ARLEN = 0): 1. Pass AR through immediately (zero-cycle overhead) 2. Pass R response back directly 3. Assert RLAST on single R beat

Multi-Beat Burst (ARLEN > 0): 1. Capture burst parameters (address, length, size, burst type) 2. Enter burst decomposition FSM 3. For each beat (0 to ARLEN): - Issue single-beat AXIL4 AR with current address - Wait for AXIL4 R response - Increment address (INCR/WRAP) or keep same (FIXED) - Accumulate worst-case response 4. Assert RLAST on final beat 5. Return to idle

FSM States: - RD_IDLE - Waiting for AR transaction, passthrough single beats - RD_BURST - Processing multi-beat burst (beats 0 to ARLEN-1) - RD_LAST_BEAT - Final beat of burst (assert RLAST)

### Response Aggregation

Responses are accumulated using worst-case priority: - SLVERR (0b10) > DECERR (0b11) > EXOKAY (0b01) > OKAY (0b00)

The worst response across all beats is returned with RLAST.

### Example Waveforms

4-Beat INCR Burst (32-bit, size=2):

Cycle  AR                 AXIL AR             AXIL R           AXI4 R
  0    addr=0x1000        -                   -                -
       len=3
  1    -                  addr=0x1000         -                -
  2    -                  -                   data=0xAAAA      data=0xAAAA, last=0
  3    -                  addr=0x1004         -                -
  4    -                  -                   data=0xBBBB      data=0xBBBB, last=0
  5    -                  addr=0x1008         -                -
  6    -                  -                   data=0xCCCC      data=0xCCCC, last=0
  7    -                  addr=0x100C         -                -
  8    -                  -                   data=0xDDDD      data=0xDDDD, last=1

## Write Converter (axi4_to_axil4_wr)

### Module Parameters

module axi4_to_axil4_wr #(
    parameter int AXI_ID_WIDTH      = 8,      // Transaction ID width (1-16)
    parameter int AXI_ADDR_WIDTH    = 32,     // Address bus width (12-64)
    parameter int AXI_DATA_WIDTH    = 32,     // Data bus width (must match)
    parameter int AXI_USER_WIDTH    = 1,      // User signal width (0-1024)
    parameter int SKID_DEPTH_AW     = 2,      // Address skid depth (2-8)
    parameter int SKID_DEPTH_W      = 4,      // Write data skid depth (2-8)
    parameter int SKID_DEPTH_B      = 4       // Response skid depth (2-8)
) (
    input  logic                        aclk,
    input  logic                        aresetn,

    // Slave AXI4 Write Interface (Input - Full Protocol)
    input  logic [AXI_ID_WIDTH-1:0]     s_axi_awid,
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axi_awaddr,
    input  logic [7:0]                  s_axi_awlen,
    input  logic [2:0]                  s_axi_awsize,
    input  logic [1:0]                  s_axi_awburst,
    input  logic                        s_axi_awlock,
    input  logic [3:0]                  s_axi_awcache,
    input  logic [2:0]                  s_axi_awprot,
    input  logic [3:0]                  s_axi_awqos,
    input  logic [3:0]                  s_axi_awregion,
    input  logic [AXI_USER_WIDTH-1:0]   s_axi_awuser,
    input  logic                        s_axi_awvalid,
    output logic                        s_axi_awready,

    input  logic [AXI_DATA_WIDTH-1:0]   s_axi_wdata,
    input  logic [STRB_WIDTH-1:0]       s_axi_wstrb,
    input  logic                        s_axi_wlast,
    input  logic [AXI_USER_WIDTH-1:0]   s_axi_wuser,
    input  logic                        s_axi_wvalid,
    output logic                        s_axi_wready,

    output logic [AXI_ID_WIDTH-1:0]     s_axi_bid,
    output logic [1:0]                  s_axi_bresp,
    output logic [AXI_USER_WIDTH-1:0]   s_axi_buser,
    output logic                        s_axi_bvalid,
    input  logic                        s_axi_bready,

    // Master AXI4-Lite Write Interface (Output - Simplified Protocol)
    output logic [AXI_ADDR_WIDTH-1:0]   m_axil_awaddr,
    output logic [2:0]                  m_axil_awprot,
    output logic                        m_axil_awvalid,
    input  logic                        m_axil_awready,

    output logic [AXI_DATA_WIDTH-1:0]   m_axil_wdata,
    output logic [STRB_WIDTH-1:0]       m_axil_wstrb,
    output logic                        m_axil_wvalid,
    input  logic                        m_axil_wready,

    input  logic [1:0]                  m_axil_bresp,
    input  logic                        m_axil_bvalid,
    output logic                        m_axil_bready
);

### Write Channel Synchronization

Key Challenge: AXI4-Lite requires AW and W to complete together, but AXI4 allows them to be independent.

Solution: 1. Track when AW has been sent for current beat (r_aw_sent flag) 2. Only consume W beat when: - AW is handshaking this cycle, OR - AW has already completed for this beat 3. Advance to next beat only when both AW and W complete

### Burst Decomposition Algorithm

Single Beat (AWLEN = 0): 1. Pass AW and W through immediately 2. Wait for B response 3. Return B with original ID

Multi-Beat Burst (AWLEN > 0): 1. Capture burst parameters (address, length, size, burst type, ID) 2. Enter burst decomposition FSM 3. For each beat (0 to AWLEN): - Issue AXIL4 AW with current address - Synchronize W data for this beat - Wait for both AW and W to complete - Collect B response - Increment address (INCR/WRAP) or keep same (FIXED) - Accumulate worst-case response 4. Return single B response with original ID after all beats complete

FSM States: - WR_IDLE - Waiting for AW transaction, passthrough single beats - WR_BURST - Processing multi-beat burst (beats 0 to AWLEN-1) - WR_LAST_BEAT - Final beat of burst

### Example Waveforms

2-Beat INCR Burst (32-bit, size=2):

Cycle  AW/W               AXIL AW/W           AXIL B           AXI4 B
  0    addr=0x2000        -                   -                -
       len=1
       data0=0xAAAA
  1    -                  addr=0x2000         -                -
                          data=0xAAAA
  2    data1=0xBBBB       -                   resp=OKAY        -
  3    -                  addr=0x2004         -                -
                          data=0xBBBB
  4    -                  -                   resp=OKAY        resp=OKAY, id=X

Note: AXI4 B response waits until all AXIL4 B responses collected.

## Full Bidirectional Converter (axi4_to_axil4)

### Module Structure

Design Philosophy: The full converter follows a composition pattern - it instantiates both read and write converters rather than duplicating their logic. This provides: - Single source of truth - Conversion logic exists only in _rd and _wr modules - Modularity - Read/write converters can be used standalone - Maintainability - Bug fixes and enhancements made in one place - Code reuse - No duplication of burst decomposition FSMs

Implementation:

module axi4_to_axil4 #(
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_DATA_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,
    parameter int SKID_DEPTH_AR     = 2,
    parameter int SKID_DEPTH_R      = 4,
    parameter int SKID_DEPTH_AW     = 2,
    parameter int SKID_DEPTH_W      = 4,
    parameter int SKID_DEPTH_B      = 4
) (
    // Clock and reset
    // Full AXI4 slave interface (all 5 channels)
    // Full AXI4-Lite master interface (all 5 channels)
);

    //==========================================================================
    // Read Path: Instantiate axi4_to_axil4_rd
    //==========================================================================

    axi4_to_axil4_rd #(
        .AXI_ID_WIDTH    (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH  (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH  (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH  (AXI_USER_WIDTH),
        .SKID_DEPTH_AR   (SKID_DEPTH_AR),
        .SKID_DEPTH_R    (SKID_DEPTH_R)
    ) u_rd_converter (
        // Connect AR and R channels
        ...
    );

    //==========================================================================
    // Write Path: Instantiate axi4_to_axil4_wr
    //==========================================================================

    axi4_to_axil4_wr #(
        .AXI_ID_WIDTH    (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH  (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH  (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH  (AXI_USER_WIDTH),
        .SKID_DEPTH_AW   (SKID_DEPTH_AW),
        .SKID_DEPTH_W    (SKID_DEPTH_W),
        .SKID_DEPTH_B    (SKID_DEPTH_B)
    ) u_wr_converter (
        // Connect AW, W, and B channels
        ...
    );

endmodule

Note: The wrapper module contains no conversion logic - it purely instantiates and connects the read and write converters. All burst decomposition, response aggregation, and protocol conversion logic resides in the _rd and _wr modules.

## Use Cases

### 1. AXI4 CPU to AXI4-Lite Peripherals

Scenario: CPU with full AXI4 master accessing simple peripherals with AXI4-Lite slaves.

Benefits: - Peripherals can use simpler AXI4-Lite interface - CPU can issue bursts for efficiency - Converter handles burst decomposition transparently

### 2. DMA to Configuration Registers

Scenario: DMA controller with AXI4 accessing memory-mapped registers.

Benefits: - Burst reads/writes decomposed automatically - Register blocks don’t need burst support - Simplified peripheral design

### 3. Protocol Bridge in SoC

Scenario: Mixed protocol system with AXI4 fabric and AXI4-Lite peripherals.

Benefits: - Clean protocol boundary - Reduced complexity in peripheral design - Easier IP integration

## Performance Characteristics

### Latency

Single-Beat Transaction: - Read: 0 cycles (passthrough) - Write: 0 cycles (passthrough)

Multi-Beat Burst: - Read: 2 × N cycles (N = burst length) - Write: 2 × N cycles + B response aggregation

### Throughput

Single-Beat: - 100% throughput (back-to-back single transactions)

Multi-Beat: - Limited by AXIL4 single-beat nature - ~50% throughput (2 cycles per beat)

### Area

Resource Usage: - Read converter: ~200 LUTs, ~100 FFs - Write converter: ~250 LUTs, ~120 FFs - Full converter: ~450 LUTs, ~220 FFs

## Integration Guidelines

### Parameter Selection

Data Width: - Must match on both sides (no width conversion) - Common values: 32, 64, 128, 256 bits

ID Width: - Sized for expected number of outstanding transactions - Typical: 4-8 bits (16-256 IDs)

Skid Buffer Depths: - Increase for timing closure in high-frequency designs - Default values usually sufficient

### Address Mapping

Example System:

AXI4 Master                AXI4-Lite Slave
(CPU/DMA)                  (Peripherals)
    |                           |
    |  64-bit addr              |  32-bit addr
    |  Bursts up to 256         |  Single beats only
    |                           |
    +--[axi4_to_axil4]----------+

Address Truncation: - Upper bits dropped (64-bit → 32-bit) - Ensure peripherals within 32-bit address space

### Error Handling

Response Mapping: - AXIL4 OKAY → AXI4 OKAY - AXIL4 SLVERR → AXI4 SLVERR

Burst Error: - Any AXIL4 beat error → entire AXI4 burst marked with error - Worst-case response returned

## Testing and Verification

### Test Coverage

Functional Tests: - Single-beat read/write (passthrough verification) - Multi-beat bursts (2, 4, 8, 16 beats) - INCR burst type (address incrementing) - FIXED burst type (same address) - WRAP burst type (address wrapping) - Error response propagation - Back-to-back transactions - Outstanding transaction handling

Performance Tests: - Maximum throughput measurement - Latency characterization - Resource utilization

### Test Results

All 28 protocol converter tests passing: - ✓ AXI4→AXIL4 Read: 7/7 tests - ✓ AXI4→AXIL4 Write: 7/7 tests - ✓ AXIL4→AXI4 Read: 7/7 tests (protocol upgrade) - ✓ AXIL4→AXI4 Write: 7/7 tests (protocol upgrade)

## Limitations

- No Data Width Conversion - Input and output data widths must match

- No Burst Optimization - All bursts decomposed, no AXIL4 burst support

- Sequential Processing - One AXIL4 beat at a time per burst

- Address Width - Upper address bits dropped if AXIL4 narrower

## Related Modules

Reverse Direction: - axil4_to_axi4_rd.sv - AXI4-Lite to AXI4 upgrade (read) - axil4_to_axi4_wr.sv - AXI4-Lite to AXI4 upgrade (write)

Data Width Conversion: - axi4_dwidth_converter_rd.sv - Width conversion with protocol - axi4_dwidth_converter_wr.sv - Width conversion with protocol

See Also: - Chapter 2: Data Width Converters - 05_axil4_to_axi4.md - Protocol upgrade specification

Version: 1.0 Status: Production Ready Last Updated: 2025-11-06 Test Status: All tests passing (28/28)

# AXI4-Lite to AXI4 Protocol Converter

## Overview

The AXI4-Lite-to-AXI4 converter provides protocol upgrade from simplified AXI4-Lite to full AXI4 by adding default values for burst-specific signals.

Available Modules: - axil4_to_axi4_rd.sv - Read-only converter (AR + R channels) - axil4_to_axi4_wr.sv - Write-only converter (AW + W + B channels) - axil4_to_axi4.sv - Full bidirectional converter (wrapper)

## Key Features

### Protocol Translation

- Signal Addition - Adds AXI4-specific signals with default values

- Single-Beat Default - All transactions marked as single-beat (LEN=0)

- Pass-Through - Minimal logic, near-zero latency

- ID Assignment - Configurable default transaction ID

### Performance

- Zero-Cycle Overhead - Direct passthrough with combinational defaults

- Full Throughput - 100% throughput maintained

- Minimal Area - Combinational logic only, no state machines

### Design Characteristics

- Data Width Match - Input and output data widths must be identical

- No Width Conversion - Pure protocol upgrade only

- Stateless - No FSM, no registered signals

## Read Converter (axil4_to_axi4_rd)

### Module Parameters

module axil4_to_axi4_rd #(
    parameter int AXI_ID_WIDTH      = 8,      // Transaction ID width
    parameter int AXI_ADDR_WIDTH    = 32,     // Address bus width
    parameter int AXI_DATA_WIDTH    = 32,     // Data bus width (must match)
    parameter int AXI_USER_WIDTH    = 1,      // User signal width
    parameter int DEFAULT_ARID      = 0       // Default transaction ID
) (
    // Slave AXI4-Lite Read Interface (Input - Simplified Protocol)
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axil_araddr,
    input  logic [2:0]                  s_axil_arprot,
    input  logic                        s_axil_arvalid,
    output logic                        s_axil_arready,

    output logic [AXI_DATA_WIDTH-1:0]   s_axil_rdata,
    output logic [1:0]                  s_axil_rresp,
    output logic                        s_axil_rvalid,
    input  logic                        s_axil_rready,

    // Master AXI4 Read Interface (Output - Full Protocol)
    output logic [AXI_ID_WIDTH-1:0]     m_axi_arid,
    output logic [AXI_ADDR_WIDTH-1:0]   m_axi_araddr,
    output logic [7:0]                  m_axi_arlen,
    output logic [2:0]                  m_axi_arsize,
    output logic [1:0]                  m_axi_arburst,
    output logic                        m_axi_arlock,
    output logic [3:0]                  m_axi_arcache,
    output logic [2:0]                  m_axi_arprot,
    output logic [3:0]                  m_axi_arqos,
    output logic [3:0]                  m_axi_arregion,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_aruser,
    output logic                        m_axi_arvalid,
    input  logic                        m_axi_arready,

    input  logic [AXI_ID_WIDTH-1:0]     m_axi_rid,
    input  logic [AXI_DATA_WIDTH-1:0]   m_axi_rdata,
    input  logic [1:0]                  m_axi_rresp,
    input  logic                        m_axi_rlast,
    input  logic [AXI_USER_WIDTH-1:0]   m_axi_ruser,
    input  logic                        m_axi_rvalid,
    output logic                        m_axi_rready
);

### Signal Mapping

AR Channel (Address Read):

// Pass-through signals
assign m_axi_araddr  = s_axil_araddr;
assign m_axi_arprot  = s_axil_arprot;
assign m_axi_arvalid = s_axil_arvalid;
assign s_axil_arready = m_axi_arready;

// Default AXI4-specific signals
assign m_axi_arid     = DEFAULT_ARID;           // Configurable default ID
assign m_axi_arlen    = 8'h00;                  // Single beat (LEN=0)
assign m_axi_arsize   = $clog2(AXI_DATA_WIDTH/8); // Full width
assign m_axi_arburst  = 2'b01;                  // INCR (don't care for single beat)
assign m_axi_arlock   = 1'b0;                   // Normal access
assign m_axi_arcache  = 4'b0000;                // Device non-bufferable
assign m_axi_arqos    = 4'h0;                   // No QoS
assign m_axi_arregion = 4'h0;                   // Default region
assign m_axi_aruser   = '0;                     // No user data

R Channel (Read Data):

// Pass-through signals
assign s_axil_rdata  = m_axi_rdata;
assign s_axil_rresp  = m_axi_rresp;
assign s_axil_rvalid = m_axi_rvalid;
assign m_axi_rready  = s_axil_rready;

// Ignore AXI4-specific signals (RID, RLAST, RUSER)

## Write Converter (axil4_to_axi4_wr)

### Module Parameters

module axil4_to_axi4_wr #(
    parameter int AXI_ID_WIDTH      = 8,      // Transaction ID width
    parameter int AXI_ADDR_WIDTH    = 32,     // Address bus width
    parameter int AXI_DATA_WIDTH    = 32,     // Data bus width (must match)
    parameter int AXI_USER_WIDTH    = 1,      // User signal width
    parameter int DEFAULT_AWID      = 0       // Default transaction ID
) (
    // Slave AXI4-Lite Write Interface (Input - Simplified Protocol)
    input  logic [AXI_ADDR_WIDTH-1:0]   s_axil_awaddr,
    input  logic [2:0]                  s_axil_awprot,
    input  logic                        s_axil_awvalid,
    output logic                        s_axil_awready,

    input  logic [AXI_DATA_WIDTH-1:0]   s_axil_wdata,
    input  logic [STRB_WIDTH-1:0]       s_axil_wstrb,
    input  logic                        s_axil_wvalid,
    output logic                        s_axil_wready,

    output logic [1:0]                  s_axil_bresp,
    output logic                        s_axil_bvalid,
    input  logic                        s_axil_bready,

    // Master AXI4 Write Interface (Output - Full Protocol)
    output logic [AXI_ID_WIDTH-1:0]     m_axi_awid,
    output logic [AXI_ADDR_WIDTH-1:0]   m_axi_awaddr,
    output logic [7:0]                  m_axi_awlen,
    output logic [2:0]                  m_axi_awsize,
    output logic [1:0]                  m_axi_awburst,
    output logic                        m_axi_awlock,
    output logic [3:0]                  m_axi_awcache,
    output logic [2:0]                  m_axi_awprot,
    output logic [3:0]                  m_axi_awqos,
    output logic [3:0]                  m_axi_awregion,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_awuser,
    output logic                        m_axi_awvalid,
    input  logic                        m_axi_awready,

    output logic [AXI_DATA_WIDTH-1:0]   m_axi_wdata,
    output logic [STRB_WIDTH-1:0]       m_axi_wstrb,
    output logic                        m_axi_wlast,
    output logic [AXI_USER_WIDTH-1:0]   m_axi_wuser,
    output logic                        m_axi_wvalid,
    input  logic                        m_axi_wready,

    input  logic [AXI_ID_WIDTH-1:0]     m_axi_bid,
    input  logic [1:0]                  m_axi_bresp,
    input  logic [AXI_USER_WIDTH-1:0]   m_axi_buser,
    input  logic                        m_axi_bvalid,
    output logic                        m_axi_bready
);

### Signal Mapping

AW Channel (Address Write):

// Pass-through signals
assign m_axi_awaddr  = s_axil_awaddr;
assign m_axi_awprot  = s_axil_awprot;
assign m_axi_awvalid = s_axil_awvalid;
assign s_axil_awready = m_axi_awready;

// Default AXI4-specific signals
assign m_axi_awid     = DEFAULT_AWID;           // Configurable default ID
assign m_axi_awlen    = 8'h00;                  // Single beat (LEN=0)
assign m_axi_awsize   = $clog2(AXI_DATA_WIDTH/8); // Full width
assign m_axi_awburst  = 2'b01;                  // INCR
assign m_axi_awlock   = 1'b0;                   // Normal access
assign m_axi_awcache  = 4'b0000;                // Device non-bufferable
assign m_axi_awqos    = 4'h0;                   // No QoS
assign m_axi_awregion = 4'h0;                   // Default region
assign m_axi_awuser   = '0;                     // No user data

W Channel (Write Data):

// Pass-through signals
assign m_axi_wdata  = s_axil_wdata;
assign m_axi_wstrb  = s_axil_wstrb;
assign m_axi_wvalid = s_axil_wvalid;
assign s_axil_wready = m_axi_wready;

// Default AXI4-specific signals
assign m_axi_wlast  = 1'b1;                     // Always last (single beat)
assign m_axi_wuser  = '0;                       // No user data

B Channel (Write Response):

// Pass-through signals
assign s_axil_bresp  = m_axi_bresp;
assign s_axil_bvalid = m_axi_bvalid;
assign m_axi_bready  = s_axil_bready;

// Ignore AXI4-specific signals (BID, BUSER)

## Full Bidirectional Converter (axil4_to_axi4)

### Module Structure

Design Philosophy: The full converter follows a composition pattern - it instantiates both read and write converters rather than duplicating their logic. This provides: - Single source of truth - Conversion logic exists only in _rd and _wr modules - Modularity - Read/write converters can be used standalone - Maintainability - Bug fixes and enhancements made in one place - Code reuse - No duplication of protocol upgrade logic

Implementation:

module axil4_to_axi4 #(
    parameter int AXI_ID_WIDTH      = 8,
    parameter int AXI_ADDR_WIDTH    = 32,
    parameter int AXI_DATA_WIDTH    = 32,
    parameter int AXI_USER_WIDTH    = 1,
    parameter int DEFAULT_ID        = 0,
    parameter int DEFAULT_REGION    = 0,
    parameter int DEFAULT_QOS       = 0,
    parameter int SKID_DEPTH_AR     = 2,
    parameter int SKID_DEPTH_AW     = 2,
    parameter int SKID_DEPTH_W      = 4,
    parameter int SKID_DEPTH_R      = 4,
    parameter int SKID_DEPTH_B      = 4
) (
    // Full AXI4-Lite slave interface (all 5 channels)
    // Full AXI4 master interface (all 5 channels)
);

    //==========================================================================
    // Read Path: Instantiate axil4_to_axi4_rd
    //==========================================================================

    axil4_to_axi4_rd #(
        .AXI_ID_WIDTH    (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH  (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH  (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH  (AXI_USER_WIDTH),
        .DEFAULT_ID      (DEFAULT_ID),
        .DEFAULT_REGION  (DEFAULT_REGION),
        .DEFAULT_QOS     (DEFAULT_QOS),
        .SKID_DEPTH_AR   (SKID_DEPTH_AR),
        .SKID_DEPTH_R    (SKID_DEPTH_R)
    ) u_rd_converter (
        // Connect AR and R channels
        ...
    );

    //==========================================================================
    // Write Path: Instantiate axil4_to_axi4_wr
    //==========================================================================

    axil4_to_axi4_wr #(
        .AXI_ID_WIDTH    (AXI_ID_WIDTH),
        .AXI_ADDR_WIDTH  (AXI_ADDR_WIDTH),
        .AXI_DATA_WIDTH  (AXI_DATA_WIDTH),
        .AXI_USER_WIDTH  (AXI_USER_WIDTH),
        .DEFAULT_ID      (DEFAULT_ID),
        .DEFAULT_REGION  (DEFAULT_REGION),
        .DEFAULT_QOS     (DEFAULT_QOS),
        .SKID_DEPTH_AW   (SKID_DEPTH_AW),
        .SKID_DEPTH_W    (SKID_DEPTH_W),
        .SKID_DEPTH_B    (SKID_DEPTH_B)
    ) u_wr_converter (
        // Connect AW, W, and B channels
        ...
    );

endmodule

Note: The wrapper module contains no conversion logic - it purely instantiates and connects the read and write converters. All protocol upgrade logic (adding default AXI4 fields) resides in the _rd and _wr modules.

## Use Cases

### 1. AXI4-Lite Peripheral to AXI4 Fabric

Scenario: Simple peripheral with AXI4-Lite interface connecting to AXI4 system interconnect.

Benefits: - Peripheral can use simpler AXIL4 interface - Connects to any AXI4 fabric - Zero overhead protocol upgrade

### 2. Legacy IP Integration

Scenario: Existing AXI4-Lite IP blocks in new AXI4-based system.

Benefits: - Reuse existing IP without modification - Seamless integration - No performance penalty

### 3. Simplified Slave Design

Scenario: New peripheral design targeting AXI4 system.

Benefits: - Design using simpler AXIL4 protocol - Automatic AXI4 compliance via converter - Reduced design complexity

## Performance Characteristics

### Latency

All Transactions: - Read: 0 cycles (combinational passthrough) - Write: 0 cycles (combinational passthrough)

### Throughput

All Transactions: - 100% throughput (no overhead)

### Area

Resource Usage: - Read converter: ~50 LUTs, 0 FFs (combinational only) - Write converter: ~60 LUTs, 0 FFs (combinational only) - Full converter: ~110 LUTs, 0 FFs (combinational only)

## Integration Guidelines

### Parameter Selection

Default IDs: - Set DEFAULT_ARID/DEFAULT_AWID to unique values if needed - Most systems can use 0 (default)

Data Width: - Must match on both sides - Common values: 32, 64, 128, 256 bits

### System Integration

Example:

AXIL4 Slave                AXI4 Master
(Peripheral)               (Interconnect)
    |                           |
    |  Simple protocol          |  Full protocol
    |  No bursts                |  Burst support
    |                           |
    +--[axil4_to_axi4]----------+

### Limitations

- Always Single-Beat - All transactions marked as LEN=0

- No Burst Support - Peripheral can’t generate bursts

- Fixed Defaults - Cache, QoS, etc. have fixed values

- No Data Width Conversion - Widths must match

## Design Rationale

### Why Protocol Upgrade?

Integration Flexibility: - Allows AXIL4 slaves in AXI4 systems - Simplifies peripheral design - Maintains AXI4 interconnect compatibility

Zero Overhead: - Pure combinational logic - No state machines - No registered signals - Minimal area impact

### Alternative Approaches

Could Use Full AXI4: - ✗ More complex peripheral design - ✗ Requires burst handling - ✗ More verification effort - ✓ Native protocol (no converter)

Could Use System-Level Bridge: - ✗ Central bottleneck - ✗ Shared resource contention - ✓ Single converter for multiple peripherals

Dedicated Per-Peripheral Converter: - ✓ Zero overhead (combinational) - ✓ No contention - ✓ Simple design - ✗ One converter per peripheral (minimal area)

## Testing and Verification

### Test Coverage

Functional Tests: - Single read/write transactions - Back-to-back transactions - Various data widths (32, 64, 128 bits) - Various ID configurations - Error response propagation

Performance Tests: - Latency verification (zero cycles) - Throughput verification (100%) - Resource utilization

### Test Results

All protocol upgrade tests passing: - ✓ AXIL4→AXI4 Read: 7/7 tests - ✓ AXIL4→AXI4 Write: 7/7 tests

## Comparison with Downgrade Converters

| Feature | AXIL4→AXI4 (Upgrade) | AXI4→AXIL4 (Downgrade) |
| --- | --- | --- |
| Complexity | Combinational only | FSM with state tracking |
| Latency | 0 cycles | 0 (single) / 2×N (burst) |
| Area | ~110 LUTs | ~450 LUTs + FFs |
| Throughput | 100% | 100% (single) / 50% (burst) |
| Use Case | Simple slave → full fabric | Full master → simple slave |

## Related Modules

Reverse Direction: - axi4_to_axil4_rd.sv - AXI4 to AXI4-Lite downgrade (read) - axi4_to_axil4_wr.sv - AXI4 to AXI4-Lite downgrade (write)

See Also: - 04_axi4_to_axil4.md - Protocol downgrade specification - Chapter 2: Data Width Converters

Version: 1.0 Status: Production Ready Last Updated: 2025-11-06 Test Status: All tests passing (14/14)

# 3.6 UART to AXI4-Lite Bridge

Module: uart_axil_bridge.sv
Supporting Modules: uart_rx.sv, uart_tx.sv
Status: Planned (Infrastructure exists, implementation pending)
Version: 1.0 (Specification)

## Overview

### Purpose

The UART-to-AXI4-Lite bridge provides human-readable ASCII command-line control of AXI4-Lite peripherals via UART serial interface. This enables:

- Debug Interface: Direct register access for system debugging

- Configuration: Setup and control peripherals without full firmware

- Testing: Manual or scripted validation of hardware functionality

- Monitoring: Read status registers and observe system state

### Key Features

- Human-Readable Protocol: ASCII text commands (e.g., “W 0x1000 0xDEADBEEF”)

- Standard Baud Rates: 115200 baud default, configurable

- Timing Isolation: Uses AXI4-Lite skid buffers to decouple UART timing from bus

- Simple Integration: Drop-in component for any AXI4-Lite system

- Bidirectional: Supports both read and write transactions

- Error Reporting: ASCII responses indicate success/failure

## Architecture

### Module Hierarchy

uart_axil_bridge (top-level)
├── uart_rx          - UART receiver (serial → parallel)
├── uart_tx          - UART transmitter (parallel → serial)
├── cmd_parser       - ASCII command decoder
├── axi_master_wr    - AXI4-Lite write transaction generator
├── axi_master_rd    - AXI4-Lite read transaction generator
└── response_encoder - Format read data as ASCII response

### Data Flow

Write Command Flow:

UART RX → ASCII Parser → Address/Data Decode → AXI4-Lite Write → Response → UART TX
   "W 0x1000 0xABCD"              ↓
                            AW: addr=0x1000
                            W:  data=0xABCD, strb=0xF
                            B:  resp → "OK" or "ERR"

Read Command Flow:

UART RX → ASCII Parser → Address Decode → AXI4-Lite Read → Format Data → UART TX
   "R 0x2000"                  ↓
                         AR: addr=0x2000
                         R:  data → "0x12345678"

## Command Protocol (Planned)

### Write Command

Format: W <address> <data>

Example:

TX: W 0x1000 0xDEADBEEF<CR><LF>
RX: OK<CR><LF>

Parameters: - <address> - Hexadecimal address (0x prefix optional) - <data> - Hexadecimal data value - Whitespace between tokens (space or tab) - Command terminated by CR, LF, or CR+LF

### Read Command

Format: R <address>

Example:

TX: R 0x1000<CR><LF>
RX: 0xDEADBEEF<CR><LF>

### Error Responses

TX: W 0xFFFFFFFF 0x1234<CR><LF>
RX: ERR: SLVERR<CR><LF>

Errors reported: - ERR: SLVERR - AXI4-Lite slave error response - ERR: DECERR - AXI4-Lite decode error - ERR: SYNTAX - Malformed command

## Parameters

### uart_axil_bridge.sv

| Parameter | Default | Description |
| --- | --- | --- |
| AXI_ADDR_WIDTH | 32 | AXI4-Lite address bus width |
| AXI_DATA_WIDTH | 32 | AXI4-Lite data bus width |
| UART_BAUD_RATE | 115200 | UART baud rate (bits per second) |
| CLK_FREQ_HZ | 100000000 | System clock frequency (for baud rate generation) |
| CMD_BUFFER_DEPTH | 256 | Command buffer size (bytes) |
| RESP_BUFFER_DEPTH | 256 | Response buffer size (bytes) |

## Interfaces

### UART Interface

// UART pins
input  logic uart_rx,           // Serial receive input
output logic uart_tx            // Serial transmit output

### AXI4-Lite Master Interface

// Write address channel
output logic [AXI_ADDR_WIDTH-1:0]    m_axil_awaddr,
output logic                         m_axil_awvalid,
input  logic                         m_axil_awready,

// Write data channel
output logic [AXI_DATA_WIDTH-1:0]    m_axil_wdata,
output logic [(AXI_DATA_WIDTH/8)-1:0] m_axil_wstrb,
output logic                         m_axil_wvalid,
input  logic                         m_axil_wready,

// Write response channel
input  logic [1:0]                   m_axil_bresp,
input  logic                         m_axil_bvalid,
output logic                         m_axil_bready,

// Read address channel
output logic [AXI_ADDR_WIDTH-1:0]    m_axil_araddr,
output logic                         m_axil_arvalid,
input  logic                         m_axil_arready,

// Read data channel
input  logic [AXI_DATA_WIDTH-1:0]    m_axil_rdata,
input  logic [1:0]                   m_axil_rresp,
input  logic                         m_axil_rvalid,
output logic                         m_axil_rready

## Timing Isolation

### Challenge

UART operates at very low data rates (115200 baud ≈ 11.52 kB/s) compared to AXI4-Lite bus speeds (typically 100+ MHz). Direct connection would create: - Long combinational paths - Timing closure difficulties - Metastability risks

### Solution

The bridge uses AXI4-Lite skid buffers (gaxi_skid_buffer.sv) to decouple UART timing from bus timing:

UART Domain            Timing Isolation         AXI4-Lite Bus
─────────────          ───────────────          ──────────────
  cmd_parser    →    Command FIFO    →    axil4_master_wr
                     ↓                ↓
              Skid Buffer        Skid Buffer
                     ↑                ↑
  resp_encoder  ←   Response FIFO   ←    axil4_master_rd

Benefits: - Clean timing domains - No timing paths between UART and AXI bus - Predictable bus behavior - Easy to meet timing at any clock frequency

## Use Cases

### 1. System Debug Interface

Scenario: Debug hardware during bring-up

# Read device ID register
> R 0x0000
0x12345678

# Write configuration register
> W 0x0010 0x00000001
OK

# Read status
> R 0x0020
0x00000003

### 2. Automated Testing

Scenario: Scripted register tests via UART

# test_script.txt
W 0x1000 0xAAAAAAAA
R 0x1000
W 0x1004 0x55555555
R 0x1004

# Send via Python
from serial import Serial
uart = Serial('/dev/ttyUSB0', 115200)
for line in open('test_script.txt'):
    uart.write(line.encode())
    response = uart.readline()
    print(response.decode())

### 3. Manufacturing Test

Scenario: Production test fixture

- UART commands sent from test jig

- Verify peripheral functionality

- Read back calibration data

- Simple pass/fail via UART responses

## Integration Example

uart_axil_bridge #(
    .AXI_ADDR_WIDTH(32),
    .AXI_DATA_WIDTH(32),
    .UART_BAUD_RATE(115200),
    .CLK_FREQ_HZ(100_000_000)
) u_uart_debug (
    .aclk           (sys_clk),
    .aresetn        (sys_rst_n),
    
    // UART pins (connect to FPGA package pins)
    .uart_rx        (uart_rxd),
    .uart_tx        (uart_txd),
    
    // AXI4-Lite master (connect to peripherals)
    .m_axil_awaddr  (debug_awaddr),
    .m_axil_awvalid (debug_awvalid),
    .m_axil_awready (debug_awready),
    // ... other AXI4-Lite signals
);

## Design Considerations

### Address Width

Standard: 32-bit addresses sufficient for most peripherals

Extended: Can configure up to 64-bit for large address spaces

### Data Width

Standard: 32-bit data matches most peripheral registers

Supported: 8, 16, 32, 64-bit data widths

Note: ASCII hex representation becomes verbose for wide data

### Command Buffer Sizing

Minimum: 64 bytes (holds ~8 commands)

Recommended: 256 bytes (holds ~32 commands)

Large Systems: 1024+ bytes for scripted test sequences

## Implementation Status

Current Status: Infrastructure and specification complete

Existing: - ✓ Filelist defined (rtl/filelists/uart_axil_bridge.f) - ✓ Test infrastructure (dv/tests/test_uart_axil_bridge.py) - ✓ Testbench class (dv/tbclasses/uart_axil_bridge_tb.py) - ✓ Python host tools (bin/uart_axi_bridge.py)

Pending: - ⏳ RTL implementation (rtl/uart_to_axil4/*.sv files) - ⏳ Dedicated module README - ⏳ Block diagram

Priority: Medium (useful for debug, not critical for core functionality)

## Future Enhancements

- Batch Commands: Support multiple commands in single UART packet

- Binary Protocol: Optional binary mode for higher throughput

- Burst Support: Extend to support AXI4 bursts (not just single beats)

- Register Macros: Named register access (“READ CONFIG” vs “R 0x1000”)

- Status Polling: Automatic periodic register monitoring

## Related Modules

- axil4_master_wr.sv - Used for AXI4-Lite write transactions

- axil4_master_rd.sv - Used for AXI4-Lite read transactions

- gaxi_skid_buffer.sv - Timing isolation buffers

## References

- UART Protocol: RS-232 serial communication standard

- AXI4-Lite: ARM AMBA AXI4-Lite specification

- Python Tools: bin/uart_axi_bridge.py - Host-side UART interface

Author: RTL Design Sherpa Project
Status: Specification Complete, Implementation Pending
Last Updated: 2025-11-14

# Appendix A: Design Notes and Analysis

This appendix contains design rationale, analysis documents, and deep-dive technical discussions that informed the converter implementation.

## A.1 APB Converter Analysis

Topic: Analysis of APB converter data width conversion patterns vs generic modules

Key Finding: The APB converter implements data width conversion using identical algorithmic patterns to our generic modules, but in a fundamentally different usage model.

Recommendation: Do NOT refactor APB converter to use generic modules due to tight coupling between protocol conversion and data width conversion.

Value: Independent validation that generic module algorithms are correct and optimal.

Full Document: a1_apb_converter_analysis.md

## A.2 Dual-Buffer Design Deep Dive

Topic: Comprehensive documentation of the dual-buffer feature implementation for high-throughput data width conversion

Achievement: Optional dual-buffer mode provides 100% throughput vs 80% for single-buffer mode

Trade-off: ~100% area increase for +25% throughput improvement

When to Use: - Single-buffer (DUAL_BUFFER=0): Area-constrained designs, throughput <100% acceptable - Dual-buffer (DUAL_BUFFER=1): Performance-critical paths, continuous streaming

Full Document: a2_dual_buffer_implementation.md

## Design Principles

These design notes capture important architectural decisions and analysis that guide the converter implementation:

- Modularity - Generic building blocks promote reuse

- Configurability - User selects optimal performance/area trade-off

- Validation - Independent implementations confirm algorithmic correctness

- Pragmatism - Not all scenarios suit the same architectural pattern

Maintained By: Converters Component Team Last Updated: 2025-11-14

# Analysis: APB Converter vs. Generic Data Width Converters

Date: 2025-10-25 Purpose: Analyze whether axi4_to_apb_convert.sv could benefit from generic axi_data_upsize and axi_data_dnsize modules

## Executive Summary

Finding: The APB converter implements data width conversion using identical algorithmic patterns to our generic modules, but in a fundamentally different usage model that makes direct integration impractical.

Recommendation: Do NOT refactor the APB converter to use generic modules. The tight coupling between protocol conversion and data width conversion requires the current inline implementation.

Value: The analysis validates that our generic module algorithms are correct - an independent implementation arrived at the same solution.

## Data Width Conversion Patterns Found

### 1. WRITE Path (Dnsize: Wide AXI → Narrow APB)

Location: axi4_to_apb_convert.sv lines 334, 337, 461

Implementation:

// Line 334 - Extract narrow data slice
w_apb_cmd_pkt_pwdata = (axi2abpratio == 1) ? r_s_axi_wdata[APBDW-1:0] :
                        r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW];

// Line 337 - Extract narrow strobe slice
w_apb_cmd_pkt_pstrb = (axi2abpratio == 1) ? r_s_axi_wstrb[APBSW-1:0] :
                       r_s_axi_wstrb[r_axi_wr_data_pointer*APBSW +: APBSW];

// Line 461 - Increment pointer through wide beat
w_axi_wr_data_pointer = r_axi_wr_data_pointer + 1;

Pattern: IDENTICAL to axi_data_dnsize module (line 210, 220) - Extract slice using pointer: data[ptr*WIDTH +: WIDTH] - Increment pointer for next narrow beat - Wrap at ratio-1

### 2. READ Path (Upsize: Narrow APB → Wide AXI)

Location: axi4_to_apb_convert.sv lines 283, 366

Implementation:

// Line 283 - Accumulate narrow data into wide shift register
r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= r_apb_rsp_pkt_prdata;

// Line 366 - Combinational accumulation (alternative path)
w_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] = r_apb_rsp_pkt_prdata;

// Lines 284-287 - Increment and wrap pointer
r_axi_rsp_data_pointer <= r_axi_rsp_data_pointer + 1;
if (r_axi_rsp_data_pointer == PTR_WIDTH'(axi2abpratio-1)) begin
    r_axi_rsp_data_pointer <= 'b0;
end

Pattern: IDENTICAL to axi_data_upsize module (line 158) - Accumulate into shift register at pointer position - Increment pointer for next narrow beat - Complete wide beat when pointer reaches ratio-1

### 3. Pointer Management

APB Converter has THREE separate pointers: - r_axi_wr_data_pointer - WRITE path dnsize (lines 315-317) - r_axi_rd_data_pointer - READ path dnsize (lines 309-311) - r_axi_rsp_data_pointer - READ response upsize (lines 284-287)

Generic Modules have ONE pointer each: - axi_data_dnsize: r_beat_ptr for splitting wide→narrow - axi_data_upsize: r_beat_ptr for accumulating narrow→wide

## Key Architectural Differences

### Generic Modules (axi_data_upsize / axi_data_dnsize)

Usage Model: Complete Beat Processing - axi_data_upsize: Wait for ALL narrow beats → output ONE complete wide beat - axi_data_dnsize: Accept ONE wide beat → output ALL narrow beats in sequence - Pure streaming pipeline with valid/ready handshaking - Standalone operation - can be inserted in any data path

State Machine: - Simple: Buffer empty/full state - No burst tracking (optional feature) - Focus solely on data width conversion

Example Flow (128→32 upsize):

1. Receive narrow beat 0 (bits [31:0])   → accumulate, ptr=1
2. Receive narrow beat 1 (bits [63:32])  → accumulate, ptr=2
3. Receive narrow beat 2 (bits [95:64])  → accumulate, ptr=3
4. Receive narrow beat 3 (bits [127:96]) → accumulate, OUTPUT wide beat, ptr=0

### APB Converter (axi4_to_apb_convert.sv)

Usage Model: Incremental Processing Within Protocol State Machine - Converts incrementally as APB transactions complete - Does NOT wait for complete wide beats - Interleaves conversion with protocol translation

State Machine: - Complex: IDLE, READ, WRITE states - Manages burst counters (r_burst_count) - Handles AXI burst to multiple APB transactions - FIRST/LAST packet generation - FIFO management for side channel data

Example Flow (128→32 write path):

1. Receive AXI AWADDR + WDATA[127:0] (full wide beat)
2. Enter WRITE state
3. APB transaction 0: Extract WDATA[31:0],   ptr=1, send to APB
4. APB transaction 1: Extract WDATA[63:32],  ptr=2, send to APB
5. APB transaction 2: Extract WDATA[95:64],  ptr=3, send to APB
6. APB transaction 3: Extract WDATA[127:96], ptr=0, complete AXI beat
7. If burst continues, repeat for next AXI beat

Critical Difference: Conversion happens within the protocol state machine, one APB transaction at a time.

## Refactoring Analysis

### Option 1: Keep Current Inline Implementation ✓ RECOMMENDED

Pros: - ✓ Already working and tested - ✓ Tight integration with protocol state machine - ✓ No additional latency - ✓ Optimal resource usage - ✓ Clear code flow for incremental conversion

Cons: - ⚠ Code duplication of conversion patterns - ⚠ Complex state machine mixing protocol + data concerns - ⚠ Harder to verify conversion logic independently

Verdict: Best choice. The tight coupling is actually necessary for the protocol conversion requirements.

### Option 2: Refactor to Use Generic Modules ✗ NOT RECOMMENDED

Conceptual Architecture:

AXI4 (DW-bit) → axi_data_dnsize → AXI4 (APBDW-bit) → axi4_to_apb_convert → APB (APBDW-bit)
                                      ↑
                              Width-matched AXI

Why This DOESN’T Work:

- Incremental Processing Conflict:

  - Generic modules process complete beats

  - APB converter processes incrementally with state machine

  - No clean insertion point for pipeline stages

- State Machine Coupling:

  - Conversion is tightly coupled with burst management

  - Pointer increments synchronized with APB transaction completions

  - FIRST/LAST generation depends on both protocol and conversion state

- Complexity Increase:

  - Would need complex glue logic to bridge generic modules to state machine

  - Additional FIFOs to buffer partial conversions

  - More difficult to reason about system behavior

- Resource Overhead:

  - Extra registers for generic module state

  - Additional control logic for coordination

  - Larger design for no functional benefit

- Verification Burden:

  - More complex interactions to verify

  - Harder to achieve coverage

  - Debugging becomes more difficult

Verdict: Refactoring would make the design more complex without providing any benefits.

## Validation: Independent Verification of Generic Module Algorithms

Finding: The APB converter independently arrived at identical data width conversion algorithms.

Validation Points:

- Slice Extraction (Dnsize):

  - APB converter line 334: r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW]

  - Generic dnsize line 210: r_data_buffer[r_beat_ptr*NARROW_WIDTH +: NARROW_WIDTH]

  - ✓ IDENTICAL pattern

- Accumulation (Upsize):

  - APB converter line 283: r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= data

  - Generic upsize line 158: r_data_accumulator[r_beat_ptr*NARROW_WIDTH +: NARROW_WIDTH] <= data

  - ✓ IDENTICAL pattern

- Pointer Management:

  - Both use same increment and wrap logic

  - Both use $clog2(RATIO) for pointer width

  - Both wrap at ratio-1

  - ✓ IDENTICAL pattern

Significance: Two independent implementations converging on the same solution provides strong evidence that our generic module algorithms are correct and optimal.

## Code Sections Reference

### APB Converter Data Width Conversion Code

Parameters:

// Line 39
parameter int AXI2APBRATIO = DW / APBDW,
parameter int PTR_WIDTH = $clog2(AXI2APBRATIO),

State Variables:

// Line 168
logic [DW-1:0] r_axi_data_shift, w_axi_data_shift;

// Lines 171-173 - THREE separate pointers
logic [PTR_WIDTH-1:0] r_axi_rd_data_pointer, w_axi_rd_data_pointer;
logic [PTR_WIDTH-1:0] r_axi_wr_data_pointer, w_axi_wr_data_pointer;
logic [PTR_WIDTH-1:0] r_axi_rsp_data_pointer, w_axi_rsp_data_pointer;

WRITE Path (Dnsize):

// Line 334 - Extract data slice
w_apb_cmd_pkt_pwdata = (axi2abpratio == 1) ? r_s_axi_wdata[APBDW-1:0] :
                        r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW];

// Line 337 - Extract strobe slice
w_apb_cmd_pkt_pstrb = (axi2abpratio == 1) ? r_s_axi_wstrb[APBSW-1:0] :
                       r_s_axi_wstrb[r_axi_wr_data_pointer*APBSW +: APBSW];

// Lines 315-317 - Increment and wrap
r_axi_wr_data_pointer <= r_axi_wr_data_pointer + 1;
if (r_axi_wr_data_pointer == PTR_WIDTH'(axi2abpratio-1))
    r_axi_wr_data_pointer <= 'b0;

READ Response Path (Upsize):

// Line 283 - Accumulate into shift register
r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= r_apb_rsp_pkt_prdata;

// Lines 284-287 - Increment and wrap
r_axi_rsp_data_pointer <= r_axi_rsp_data_pointer + 1'b1;
if (r_axi_rsp_data_pointer == PTR_WIDTH'(axi2abpratio-1)) begin
    r_axi_rsp_data_pointer <= 'b0;
end

// Line 366 - Combinational accumulation (alternative path)
w_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] = r_apb_rsp_pkt_prdata;

## Conclusion

### Summary

The APB converter implements data width conversion using patterns identical to our generic modules, but in a fundamentally different architectural context:

- Generic modules: Standalone streaming pipeline stages processing complete beats

- APB converter: Inline conversion within protocol state machine processing incrementally

### Recommendations

- Do NOT refactor APB converter to use generic modules

  - Current inline implementation is optimal for its use case

  - Refactoring would increase complexity without benefits

- Keep generic modules separate for their intended use cases:

  - Pure data width conversion in streaming pipelines

  - Protocol-agnostic width adaptation

  - Reusable components for future designs

- Document the relationship between the two implementations:

  - Both use same fundamental algorithms (validation)

  - Different usage models require different architectures

  - Not all width conversion scenarios suit pipeline approach

- Consider future opportunities:

  - Other protocol converters may benefit from generic modules

  - AXI-to-AXI width converters are prime candidates

  - Future designs should evaluate if pipeline model fits

### Value Delivered

- ✓ Validation - Independent implementation confirms algorithm correctness

- ✓ Clarity - Understand why inline vs. pipeline architectures differ

- ✓ Documentation - Captured design rationale for future reference

- ✓ Guidance - When to use generic modules vs. inline conversion

## Related Files

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi_data_upsize.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi_data_dnsize.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi4_to_apb_convert.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/USAGE.md

Author: RTL Design Sherpa Date: 2025-10-25

# Dual-Buffer Implementation for axi_data_dnsize

Date: 2025-10-25 Purpose: Document the dual-buffer feature implementation for high-throughput data width conversion

## Executive Summary

Added optional dual-buffer mode to axi_data_dnsize.sv module to achieve 100% throughput for continuous data streams.

Key Results: - Single-buffer mode (DUAL_BUFFER=0): 80% throughput, 1-cycle dead time per wide beat - Dual-buffer mode (DUAL_BUFFER=1): 100% throughput, zero dead cycles - Area cost: Approximately 2× buffer registers (+100% overhead) - Compatibility: Fully backward compatible, works with all existing test configurations

## Architecture Overview

### Single-Buffer Mode (DUAL_BUFFER=0)

Structure:

┌─────────────────┐
│  r_data_buffer  │  ← ONE wide beat buffered
│  (WIDE_WIDTH)   │
└─────────────────┘
     ↓
  Split into
  narrow beats

Throughput Limitation: - Must wait for ALL narrow beats to complete before accepting next wide beat - 1-cycle gap when transitioning between wide beats - Throughput = 4/(4+1) = 80% for 4:1 ratio

### Dual-Buffer Mode (DUAL_BUFFER=1)

Structure:

┌─────────────┐
│  r_buffer_0  │  ← Splitting into narrow beats
│              │
└─────────────┘
     ↓ (reading)

┌─────────────┐
│  r_buffer_1  │  ← Accepting NEXT wide beat
│              │
└─────────────┘
     ↑ (writing)

Ping-Pong Operation: 1. Buffer 0 splits while Buffer 1 accepts new data 2. When Buffer 0 completes, swap: Buffer 1 splits, Buffer 0 accepts 3. Continuous operation with no dead cycles

Throughput: 100% (4/4) for any ratio

## Implementation Details

### New Parameter

parameter int DUAL_BUFFER = 0,  // 0=single buffer (80% throughput, area efficient)
                                // 1=dual buffer (100% throughput, 2× area)

### State Variables

Single-Buffer Mode:

logic [WIDE_WIDTH-1:0]          r_data_buffer;
logic [WIDE_SB_PORT_WIDTH-1:0]  r_sideband_buffer;
logic                           r_wide_buffered;
logic                           r_last_buffered;

Dual-Buffer Mode:

logic [WIDE_WIDTH-1:0]          r_buffer_0, r_buffer_1;
logic [WIDE_SB_PORT_WIDTH-1:0]  r_sb_buffer_0, r_sb_buffer_1;
logic                           r_last_buffer_0, r_last_buffer_1;
logic                           r_buffer_0_valid, r_buffer_1_valid;
logic                           r_read_buffer;  // 0=reading buf0, 1=reading buf1

### Write Path (DUAL_BUFFER=1)

if (wide_valid && wide_ready) begin
    if (!gen_dual_buffer.r_buffer_0_valid) begin
        // Write to buffer 0
        gen_dual_buffer.r_buffer_0 <= wide_data;
        gen_dual_buffer.r_buffer_0_valid <= 1'b1;
    end else begin
        // Write to buffer 1 (must be empty if wide_ready=1)
        gen_dual_buffer.r_buffer_1 <= wide_data;
        gen_dual_buffer.r_buffer_1_valid <= 1'b1;
    end
end

Key: Always write to the empty buffer.

### Read Path (DUAL_BUFFER=1)

// Read from current buffer
if (current_buffer_valid && narrow_ready) begin
    if (is_last_narrow_beat) begin
        // Clear current buffer's valid flag
        if (gen_dual_buffer.r_read_buffer)
            gen_dual_buffer.r_buffer_1_valid <= 1'b0;
        else
            gen_dual_buffer.r_buffer_0_valid <= 1'b0;

        // Swap to other buffer if it has data
        if (other_buffer_valid)
            gen_dual_buffer.r_read_buffer <= ~gen_dual_buffer.r_read_buffer;
    end
end

Key: On last narrow beat, clear current buffer and swap to other if available.

### Ready Logic

Single-Buffer:

assign wide_ready = !r_wide_buffered || (narrow_ready && w_last_narrow_beat);

- Ready when buffer empty OR sending last narrow beat (1-cycle early)

Dual-Buffer:

assign wide_ready = !r_buffer_0_valid || !r_buffer_1_valid;

- Ready when at least one buffer is empty

- Allows continuous acceptance

## Burst Tracking Integration

Dual-buffer mode fully supports burst tracking (TRACK_BURSTS=1):

Challenge: Track burst position across buffer swaps

Solution: Shared burst counter applies to whichever buffer is currently being read:

// Burst tracking state (shared between buffers)
logic [BURST_LEN_WIDTH-1:0] r_slave_beat_count;
logic [BURST_LEN_WIDTH-1:0] r_slave_total_beats;
logic                       r_burst_active;

// Applies to current read buffer
if (((r_slave_beat_count + 1'b1) >= r_slave_total_beats)) begin
    // Last narrow beat of entire burst
    // Clear current buffer and end burst
end

Result: LAST signal correctly generated on final beat of burst, regardless of buffer swapping.

## Resource Impact

### Area Analysis (128-bit Wide, 32-bit Narrow, 16-bit Sideband)

| Resource | Single Buffer | Dual Buffer | Overhead |
| --- | --- | --- | --- |
| Data registers | 128 bits | 256 bits | +128 bits |
| Sideband registers | 16 bits | 32 bits | +16 bits |
| LAST flags | 1 bit | 2 bits | +1 bit |
| Valid flags | 1 bit | 2 bits | +1 bit |
| Read selector | - | 1 bit | +1 bit |
| Total FFs | 146 | 292 | +146 (+100%) |

Control Logic: ~30% increase (buffer selection, swap logic)

Overall: Expect ~100% area increase for dual-buffer mode.

### Performance Impact

| Mode | Throughput (4:1 Ratio) | Cycles/Wide Beat | Utilization |
| --- | --- | --- | --- |
| Single-Buffer | 80% | 5 (4 active + 1 dead) | 4/5 |
| Dual-Buffer | 100% | 4 (4 active + 0 dead) | 4/4 |

Improvement: +25% throughput (from 80% to 100%)

## Test Coverage

### Test Configurations Added

All existing test configurations now have dual-buffer variants:

Single-Buffer Tests (DUAL_BUFFER=0): - 128to32_wstrb_slice_simple - 256to64_wstrb_slice_simple - 128to32_rresp_broadcast_simple - 256to64_rresp_broadcast_simple - 128to32_rresp_burst_track - 256to64_rresp_burst_track - 512to128_rresp_burst_track - 128to64_no_sideband_simple

Dual-Buffer Tests (DUAL_BUFFER=1): - Same configurations with “_DUAL” suffix - Total: 16 test configurations

### Verification Results

Tested: - ✓ Basic data splitting (all ratios) - ✓ Sideband handling (broadcast and slice modes) - ✓ Burst tracking with LAST generation - ✓ Backpressure handling - ✓ Continuous streaming

Result: All tests PASS for both single and dual-buffer modes.

## Usage Recommendations

### When to Use Single-Buffer Mode (DUAL_BUFFER=0)

✓ Use when: - Area is critical - Throughput requirements are <100% - Source/sink have natural gaps in traffic - Design is already bottlenecked elsewhere

Example: Write path downsize where upstream has occasional pauses

### When to Use Dual-Buffer Mode (DUAL_BUFFER=1)

✓ Use when: - Maximum throughput required - Continuous streaming data - Sufficient area budget - Critical path in high-performance system

Example: High-bandwidth DMA engine with continuous transfers

## Code Example

### Instantiation: Single-Buffer Mode (Area Efficient)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),      // WSTRB
    .NARROW_SB_WIDTH(16),    // WSTRB
    .SB_BROADCAST(0),        // Slice mode
    .TRACK_BURSTS(0),        // Simple mode
    .DUAL_BUFFER(0)          // Single buffer (80% throughput)
) u_dnsize (
    .aclk(clk),
    .aresetn(rst_n),
    // ... ports
);

### Instantiation: Dual-Buffer Mode (High Throughput)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),
    .NARROW_SB_WIDTH(16),
    .SB_BROADCAST(0),
    .TRACK_BURSTS(0),
    .DUAL_BUFFER(1)          // Dual buffer (100% throughput)
) u_dnsize (
    .aclk(clk),
    .aresetn(rst_n),
    // ... ports
);

## Design Decisions

### Why Not Dual-Buffer for Upsize?

axi_data_upsize already achieves 100% throughput with single buffer: - Can accept narrow beat while outputting wide beat simultaneously - The || wide_ready term in narrow_ready enables overlap - No benefit from dual buffering

Conclusion: Dual-buffer only needed for dnsize module.

### Why Use Generate Blocks?

Rationale: - Complete separation of single vs. dual-buffer logic - No runtime overhead (compile-time selection) - Easier to verify each mode independently - Cleaner code structure

### Why Not Make Dual-Buffer the Default?

Considerations: - 100% area overhead is significant - Many use cases don’t need 100% throughput - Single-buffer is simpler and well-tested - User can opt-in when needed

Decision: Default to DUAL_BUFFER=0, user explicitly enables when needed.

## Lessons Learned

### SystemVerilog Constraints

Issue: Cannot declare logic variables inside procedural blocks

// ILLEGAL:
always_ff @(posedge clk) begin
    logic temp;  // ← NOT ALLOWED
    temp = ...;
end

Solution: Inline expressions or move declarations outside block

### Generate Block Naming

Best Practice: Use hierarchical naming for generate block signals:

gen_dual_buffer.r_buffer_0_valid  // Clear which mode
gen_single_buffer.r_wide_buffered // Clear which mode

### Buffer Swap Logic

Key Insight: Swap only when: 1. Current buffer completes (last narrow beat sent) 2. Other buffer has valid data waiting

This prevents unnecessary swaps and maintains correct ordering.

## Future Enhancements

### Potential Improvements

- Configurable Buffer Depth: Allow >2 buffers for even higher throughput

- Credit-Based Flow Control: Integrate with upstream credit system

- Performance Counters: Monitor buffer utilization, stalls

- Power Gating: Disable unused buffer when not needed

### Alternative Architectures

Skid Buffer Approach: - Insert skid buffers on outputs instead of dual data buffers - Lower area overhead - Similar throughput improvement

FIFO Approach: - Replace buffers with small FIFOs - More flexible depth control - Higher area cost

## Summary

The dual-buffer implementation successfully provides an optional high-throughput mode for the axi_data_dnsize module:

✓ Functionality: Proven correct through comprehensive testing ✓ Performance: 100% throughput vs. 80% for single-buffer ✓ Compatibility: Fully backward compatible ✓ Flexibility: User-selectable via parameter ✓ Robustness: Works with all modes (broadcast, slice, burst tracking)

Trade-off: ~100% area increase for +25% throughput improvement

Recommendation: Use dual-buffer mode for performance-critical paths with continuous data flow.

## Related Files

- projects/components/converters/rtl/axi_data_dnsize.sv - RTL implementation

- projects/components/converters/dv/tests/test_axi_data_dnsize.py - Test suite

- projects/components/converters/dv/tbclasses/axi_data_dnsize_tb.py - Testbench class

- projects/components/converters/USAGE.md - Usage documentation

- projects/components/converters/ANALYSIS_APB_CONVERTER.md - APB converter analysis

Author: RTL Design Sherpa Date: 2025-10-25

# Appendix B: Change Logs and Refactoring History

This appendix documents major refactorings, updates, and changes to the converter component.

## B.1 Refactoring to Composition Pattern (2025-11-10)

Summary: Refactored AXI4/AXI4-Lite converter wrappers from inline duplication to clean composition pattern

Affected Modules: - axil4_to_axi4.sv - Now instantiates read/write submodules - axi4_to_axil4.sv - Now instantiates read/write submodules

Benefits: - Single source of truth (bug fixes only needed once) - Improved modularity (can use rd/wr converters standalone) - Better maintainability (clearer separation of concerns) - Code reuse (DRY principle)

Before: 244-446 lines of duplicated conversion logic
After: 263-277 lines of clean instantiation wrappers

Full Document: b1_refactor_summary_2025-11-10.md

## B.2 Documentation Updates (2025-11-10)

Summary: Updated converter specification to reflect refactoring and add missing module documentation

Changes: - Updated Ch03 protocol converter sections for composition pattern - Added design philosophy sections explaining single source of truth - Added uart_axil_bridge to module list - Updated version to 1.4

Verification: All RTL modules now properly documented in specification

Full Document: b2_documentation_update_2025-11-10.md

## Major Version History

| Version | Date | Summary |
| --- | --- | --- |
| 1.5 | 2025-11-14 | Added appendices, UART chapter, missing diagrams |
| 1.4 | 2025-11-10 | Refactored to composition pattern |
| 1.3 | 2025-11-06 | Added AXI4↔AXIL4 protocol converters |
| 1.2 | 2025-10-26 | Added comprehensive specification with diagrams |
| 1.1 | 2025-10-25 | Added dual-buffer mode for axi_data_dnsize |
| 1.0 | 2025-10-24 | Initial release |

Maintained By: Converters Component Team Last Updated: 2025-11-14

# Converter Refactoring Summary

Date: 2025-11-10 Author: RTL Design Sherpa with Claude Code

## Overview

Refactored AXI4/AXI4-Lite converter modules to follow composition pattern instead of duplicating logic.

## Changes Made

### 1. axil4_to_axi4.sv (AXI4-Lite → AXI4)

Before: - 244 lines of inline conversion logic - Duplicated logic between top-level and read/write specific modules - Hard to maintain (changes needed in multiple places)

After: - 277 lines (wrapper with instantiations) - Simply instantiates axil4_to_axi4_rd and axil4_to_axi4_wr - Single source of truth for conversion logic

### 2. axi4_to_axil4.sv (AXI4 → AXI4-Lite)

Before: - 446 lines of inline burst decomposition logic - Duplicated logic between top-level and read/write specific modules - Complex state machines duplicated

After: - 263 lines (wrapper with instantiations) - Simply instantiates axi4_to_axil4_rd and axi4_to_axil4_wr - Single source of truth for burst decomposition logic

## Architecture Pattern

Top-level wrapper (axil4_to_axi4.sv):
├── u_rd_converter (axil4_to_axi4_rd)
│   ├── AR channel: Add AXI4 fields (ID, LEN, SIZE, etc.)
│   └── R channel: Strip AXI4 fields (ID, LAST, USER)
└── u_wr_converter (axil4_to_axi4_wr)
    ├── AW channel: Add AXI4 fields
    ├── W channel: Add WLAST=1
    └── B channel: Strip AXI4 fields (ID, USER)

Top-level wrapper (axi4_to_axil4.sv):
├── u_rd_converter (axi4_to_axil4_rd)
│   ├── AR channel: Burst decomposition FSM
│   └── R channel: Response aggregation
└── u_wr_converter (axi4_to_axil4_wr)
    ├── AW/W channels: Burst decomposition FSM
    └── B channel: Response aggregation

## Benefits

- Single Source of Truth

  - Bug fixes only need to be made once

  - Consistent behavior across standalone and combined converters

- Modularity

  - Can use read-only or write-only converters standalone

  - Easier to test individual components

- Maintainability

  - Clearer separation of concerns

  - Easier to understand and modify

- Code Reuse

  - No duplicated logic

  - Following DRY (Don’t Repeat Yourself) principle

## Verification

Lint checks pass with only expected warnings about unused signals (intentionally dropped AXI4 fields):

verilator --lint-only +incdir+rtl/amba/includes \
  projects/components/converters/rtl/axil4_to_axi4.sv \
  projects/components/converters/rtl/axil4_to_axi4_rd.sv \
  projects/components/converters/rtl/axil4_to_axi4_wr.sv

No instantiation errors - interfaces match correctly.

## Files Modified

- projects/components/converters/rtl/axil4_to_axi4.sv - Refactored to instantiate read/write modules

- projects/components/converters/rtl/axi4_to_axil4.sv - Refactored to instantiate read/write modules

## Files Unchanged (Submodules)

- projects/components/converters/rtl/axil4_to_axi4_rd.sv - Read converter (150 lines)

- projects/components/converters/rtl/axil4_to_axi4_wr.sv - Write converter (175 lines)

- projects/components/converters/rtl/axi4_to_axil4_rd.sv - Read converter with burst decomposition

- projects/components/converters/rtl/axi4_to_axil4_wr.sv - Write converter with burst decomposition

## Next Steps

Consider applying the same pattern to other converter families if they have similar duplication: - axi4_dwidth_converter_rd.sv / axi4_dwidth_converter_wr.sv - Any other protocol converters with separate read/write variants

Verification Status: ✓ Lint checks pass Code Review: Recommended before integration Testing: Existing converter tests should verify functionality

# Converter Documentation Update

Date: 2025-11-10 Author: RTL Design Sherpa with Claude Code

## Summary

Updated converter specification documentation to sync with latest RTL refactoring and add missing module descriptions.

## Changes Made

### 1. Updated Protocol Converter Documentation

#### ch03_protocol_converters/04_axi4_to_axil4.md

- Added Design Philosophy section explaining composition pattern

- Emphasized single source of truth approach

- Detailed benefits: modularity, maintainability, code reuse

- Updated module structure to show instantiation approach

- Clarified that wrapper contains NO conversion logic

#### ch03_protocol_converters/05_axil4_to_axi4.md

- Added Design Philosophy section explaining composition pattern

- Updated module structure with proper parameter list

- Fixed parameter names (DEFAULT_ID vs DEFAULT_ARID/AWID)

- Added note about pure instantiation (no logic in wrapper)

### 2. Updated Main Index (converter_index.md)

Version: - Updated from 1.3 to 1.4 - Updated “Last Updated” from 2025-11-06 to 2025-11-10 - Updated “Last Review” from 2025-11-06 to 2025-11-10

Version History: - Added v1.4 entry documenting refactoring to composition pattern

Module List: - Added uart_axil_bridge to protocol converters list with reference to dedicated README

### 3. Updated Overview (ch03_protocol_converters/01_overview.md)

Added Section: - Section 5: UART to AXI4-Lite Bridge - Module list: uart_axil_bridge.sv, uart_rx.sv, uart_tx.sv - Key features: ASCII commands, 115200 baud, timing isolation - Reference to dedicated documentation in rtl/uart_to_axil4/README.md

## Documentation Verification

### RTL Modules vs Documentation

All RTL modules are now properly documented:

Data Width Converters: - ✓ axi_data_upsize.sv - ✓ axi_data_dnsize.sv - ✓ axi4_dwidth_converter_wr.sv - ✓ axi4_dwidth_converter_rd.sv

Protocol Converters: - ✓ axi4_to_axil4.sv (wrapper - refactored) - ✓ axi4_to_axil4_rd.sv - ✓ axi4_to_axil4_wr.sv - ✓ axil4_to_axi4.sv (wrapper - refactored) - ✓ axil4_to_axi4_rd.sv - ✓ axil4_to_axi4_wr.sv - ✓ axi4_to_apb_convert.sv - ✓ axi4_to_apb_shim.sv - ✓ peakrdl_to_cmdrsp.sv - ✓ uart_axil_bridge.sv (with dedicated README)

## Refactoring Documentation

The documentation now correctly reflects the composition pattern used in the refactored code:

### Before Refactoring (Implicit)

Documentation described wrappers but didn’t emphasize design pattern.

### After Refactoring (Explicit)

Documentation now clearly states: 1. Composition Pattern - Instantiation vs duplication 2. Benefits - Single source of truth, modularity, maintainability 3. Implementation - Wrappers contain NO logic, only instantiations 4. Modularity - Read/write modules can be used standalone

## Key Documentation Principles

- Accuracy - Documentation matches actual RTL implementation

- Completeness - All modules documented

- Clarity - Design patterns explicitly explained

- Traceability - References to detailed specs and READMEs

## Related Files

Modified: - docs/converter_spec/converter_index.md - docs/converter_spec/ch03_protocol_converters/01_overview.md - docs/converter_spec/ch03_protocol_converters/04_axi4_to_axil4.md - docs/converter_spec/ch03_protocol_converters/05_axil4_to_axi4.md

Referenced (existing): - rtl/uart_to_axil4/README.md - UART bridge documentation - REFACTOR_SUMMARY_2025-11-10.md - RTL refactoring details

## Verification Checklist

- ☒ All RTL modules listed in documentation

- ☒ Refactored modules reflect composition pattern

- ☒ Version numbers updated

- ☒ Dates updated

- ☒ UART bridge referenced

- ☒ Design philosophy sections added

- ☒ No broken cross-references

Status: ✓ Complete Review: Recommended before distribution Next Steps: Consider PDF generation with updated content

# Converters Graphviz Block Diagrams

This directory contains Graphviz source files (.gv) and generated PNG diagrams for Converters component modules.

## Files

### Data Width Converters

Generic Building Blocks: - axi_data_upsize.gv - Narrow→Wide accumulator (128→512 example) - axi_data_dnsize_single.gv - Wide→Narrow splitter, single buffer (512→128, 80% throughput) - axi_data_dnsize_dual.gv - Wide→Narrow splitter, dual buffer (512→128, 100% throughput)

Full AXI4 Converters: - axi4_dwidth_converter_wr.gv - Write path converter (AW + W + B channels) - axi4_dwidth_converter_rd.gv - Read path converter (AR + R channels)

### Protocol Converters

- axi4_to_apb.gv - AXI4-to-APB protocol bridge

- peakrdl_adapter.gv - PeakRDL register interface to command/response adapter

### Generated Images (.png)

All diagrams are available as PNG files: - axi_data_upsize.png - axi_data_dnsize_single.png - axi_data_dnsize_dual.png - axi4_dwidth_converter_wr.png - axi4_dwidth_converter_rd.png - axi4_to_apb.png - peakrdl_adapter.png

## Regenerating Diagrams

### Prerequisites

- Graphviz installed (sudo apt install graphviz on Ubuntu/Debian)

- dot command available in PATH

### Generate All Diagrams

# From this directory
make all

# Or manually
dot -Tpng axi_data_upsize.gv -o axi_data_upsize.png
dot -Tpng axi_data_dnsize_single.gv -o axi_data_dnsize_single.png
# ... etc

### Generate Single Diagram

# Upsize converter
dot -Tpng axi_data_upsize.gv -o axi_data_upsize.png

# Downsize single buffer
dot -Tpng axi_data_dnsize_single.gv -o axi_data_dnsize_single.png

# Downsize dual buffer
dot -Tpng axi_data_dnsize_dual.gv -o axi_data_dnsize_dual.png

### Generate SVG (for web/docs)

dot -Tsvg axi_data_upsize.gv -o axi_data_upsize.svg
# ... for all diagrams

### Generate PDF (for documentation)

dot -Tpdf axi_data_upsize.gv -o axi_data_upsize.pdf
# ... for all diagrams

## Diagram Contents

### axi_data_upsize.gv

Shows: - Narrow input interface (128-bit beats) - Beat counter (0→3 for 4:1 ratio) - 512-bit data buffer accumulation - Wide output interface (512-bit beats) - Sideband handling (WSTRB concatenation or RRESP OR) - 100% throughput annotation

Use Case: Understanding narrow-to-wide data accumulation

### axi_data_dnsize_single.gv

Shows: - Wide input interface (512-bit beats) - Single 512-bit buffer - 4:1 multiplexer for 128-bit slices - Output counter (0→3) - Narrow output interface (128-bit beats) - Timing diagram showing 80% throughput (1-cycle gap) - Sideband handling (WSTRB slice or RRESP broadcast)

Use Case: Understanding wide-to-narrow splitting (area-efficient mode)

### axi_data_dnsize_dual.gv

Shows: - Wide input interface (512-bit beats) - Dual ping-pong buffers (Buffer 0 and Buffer 1) - Buffer select logic (write/read muxes) - Independent counters for each buffer - Output multiplexing - Timing diagram showing 100% throughput (no gaps) - Ping-pong operation (one reads while other writes)

Use Case: Understanding dual-buffer high-performance mode

### axi4_dwidth_converter_wr.gv

Shows: - AXI4 slave interface (narrow - 64-bit) - AW/W/B channel handling - Address calculator and burst length adjuster - Skid buffers for flow control - axi_data_upsize integration - AXI4 master interface (wide - 512-bit) - Example transaction (8 narrow beats → 1 wide beat)

Use Case: Understanding full write path conversion

### axi4_dwidth_converter_rd.gv

Shows: - AXI4 slave interface (wide - 512-bit) - AR/R channel handling - Address calculator and burst length adjuster - Burst tracker for RLAST generation - axi_data_dnsize integration (dual buffer) - AXI4 master interface (narrow - 64-bit) - Continuous streaming timing (100% throughput)

Use Case: Understanding full read path conversion

### axi4_to_apb.gv

Shows: - AXI4 slave interface (64-bit addr, 32-bit data) - Protocol conversion state machine - Write path (AW + W → APB write) - Read path (AR → APB read) - Address width conversion (64→32 bit) - Response converter (PSLVERR → BRESP/RRESP) - APB master interface - FSM state descriptions - Example timing diagrams

Use Case: Understanding AXI4-to-APB protocol conversion

### peakrdl_adapter.gv

Shows: - PeakRDL register interface (APB-style) - Command generator - Response handler - Command/response interface (custom protocol) - Write transaction example - Read transaction example - Error propagation

Use Case: Understanding PeakRDL adapter protocol decoupling

## Diagram Style

Colors: - Light blue - Input interfaces (slave/narrow/wide) - Light coral/pink - Output interfaces (master/narrow/wide) - Light yellow - Control logic and notes - Light gray - Data path elements - Light green - Features/highlights - Light cyan - Examples - Orange - Warnings/limitations

Nodes: - Rounded boxes - Functional blocks - Cylinders - Registers/buffers - Diamonds - Decision/control points - Trapezoids - Multiplexers - Note boxes - Annotations, examples, timing diagrams - Records - Interfaces with multiple signals

Edges: - Bold solid - Primary data flow - Dashed - Control signals - Colored - Protocol-specific paths (blue=write, green=read)

## Adding New Diagrams

To add a new converter diagram:

- Create new .gv file:

- cp axi_data_upsize.gv my_new_converter.gv

- Edit the new file:

  - Update title and module name

  - Adjust nodes for converter functionality

  - Update data flow connections

  - Add timing/example annotations

- Update Makefile SOURCES list:

- SOURCES = ... \
          my_new_converter.gv

- Generate PNG:

- make all

- Link from documentation:

- ![My Converter Block Diagram](../assets/graphviz/my_new_converter.png)

## Referenced In Documentation

These diagrams are referenced in: - converter_index.md - Main specification index - ch02_data_width_converters/*.md - Data width converter chapters - ch03_protocol_converters/*.md - Protocol converter chapters - ch04_usage_examples/*.md - Usage and integration examples

## Tools

Graphviz Version: Any recent version (tested with 2.43+)

Alternative Viewers: - Online: http://www.webgraphviz.com/ (paste .gv content) - VS Code: Install “Graphviz Preview” extension - Command line: xdg-open <diagram>.png (Linux)

## Notes

- PNG files are version controlled for easy documentation viewing

- Source .gv files are the authoritative source

- Regenerate PNGs after any .gv file changes

- Keep diagrams up-to-date with module implementations

- Use consistent color scheme across all diagrams

Last Updated: 2025-10-26 Maintainer: RTL Design Sherpa Project

# Converters PlantUML FSM Diagrams

This directory contains PlantUML source files (.puml) and generated PNG diagrams for Converters FSM documentation.

## Files

### Source Files (.puml)

- axi4_to_apb_fsm.puml - APB state machine for AXI4-to-APB protocol converter

### Generated Images (.png)

- axi4_to_apb_fsm.png - FSM diagram for AXI4-to-APB converter

## FSM Overview

### AXI4-to-APB Converter FSM

States: - IDLE - Wait for AXI4 transaction (AR or AW) - READ - Process AXI4 AR transaction → APB read - WRITE - Process AXI4 AW+W transaction → APB write

Key Features: - Read priority over write when both pending - Burst support (continues in READ/WRITE until all beats complete) - APB protocol phases (Setup → Access → Wait) - Error response handling (PSLVERR → BRESP/RRESP)

Transitions: - IDLE → READ: AXI4 read request (ARVALID && ARREADY) - IDLE → WRITE: AXI4 write request (AWVALID && AWREADY) - READ → IDLE: APB read complete (PREADY && all beats done) - WRITE → IDLE: APB write complete (PREADY && all beats done) - Self-loops: Continue burst transfers

## Note on Other Converters

Data width converters (axi_data_upsize, axi_data_dnsize) do not have FSMs: - They use simple counter-based control logic - Upsize: Beat counter (0→N-1) to accumulate narrow beats - Downsize: Output counter (0→N-1) to split wide beats - No complex state transitions needed

Only protocol converters have FSMs: - AXI4-to-APB: Protocol translation requires state machine - PeakRDL adapter: Simple passthrough, no FSM needed

## Regenerating Diagrams

### Prerequisites

- PlantUML installed

- Java runtime (required by PlantUML)

### Install PlantUML

# Ubuntu/Debian
sudo apt install plantuml

# Or download JAR
wget https://github.com/plantuml/plantuml/releases/download/v1.2023.13/plantuml-1.2023.13.jar

### Generate PNG from PlantUML

# Using installed plantuml
plantuml axi4_to_apb_fsm.puml

# Or using downloaded JAR
java -jar plantuml.jar axi4_to_apb_fsm.puml

# Output: axi4_to_apb_fsm.png

### Generate SVG (for web)

plantuml -tsvg axi4_to_apb_fsm.puml

### Generate PDF (for documentation)

plantuml -tpdf axi4_to_apb_fsm.puml

## FSM Style

Colors: - Default state colors (light blue/yellow as per PlantUML defaults) - Transition arrows with labels showing conditions

Annotations: - Entry actions described in state boxes - Transition conditions on arrows - Notes explaining protocol sequences

Format: - Standard UML state machine notation - Clear state names matching RTL enumeration - Transition guards showing trigger conditions

## Referenced In Documentation

These diagrams are referenced in: - ch03_protocol_converters/02_axi4_to_apb.md - AXI4-to-APB converter chapter - converter_index.md - Main specification index

## Adding New FSM Diagrams

If future converters include FSMs:

- Create new .puml file:

- @startuml my_converter_fsm
title My Converter FSM

[*] --> IDLE
IDLE --> ACTIVE : trigger
ACTIVE --> IDLE : done

@enduml

- Generate PNG:

- plantuml my_converter_fsm.puml

- Link from documentation:

- ![My Converter FSM](../assets/puml/my_converter_fsm.png)

## Tools

PlantUML Version: Any recent version (tested with 1.2023+)

Alternative Renderers: - Online: http://www.plantuml.com/plantuml/uml/ (paste .puml content) - VS Code: Install “PlantUML” extension - IntelliJ IDEA: Built-in PlantUML support

## Notes

- PNG files are version controlled for easy viewing

- Source .puml files are the authoritative source

- Regenerate PNGs after any .puml changes

- Keep FSM diagrams synchronized with RTL state definitions

Last Updated: 2025-10-26 Maintainer: RTL Design Sherpa Project

# Converters - Data Width and Protocol Conversion Modules

Status: Production Ready Version: 1.2 Last Updated: 2025-10-25

## Quick Start

The Converters component provides both data width conversion and protocol conversion modules, enabling seamless integration between components with different data widths or protocols.

### Key Features

Data Width Converters: - Bidirectional Conversion - Upsize (narrow→wide) and Downsize (wide→narrow) - Flexible Width Ratios - Any integer ratio (2:1, 4:1, 8:1, 16:1, etc.) - Sideband Support - Configurable handling for WSTRB (slice) and RRESP (broadcast) - Burst Tracking - Optional burst-aware LAST signal generation (read path) - High Throughput - Optional dual-buffer mode for 100% throughput (downsize) - Generic Building Blocks - Reusable axi_data_upsize and axi_data_dnsize modules

Protocol Converters: - AXI4-to-APB Bridge - Full AXI4 to APB protocol conversion with address/data width adaptation - PeakRDL Adapter - Convert PeakRDL register interface to custom command/response protocol

### Performance Summary

| Module | Mode | Throughput | Area | Use Case |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single buffer | 100% | 1× | Narrow→Wide (always optimal) |
| axi_data_dnsize | Single buffer | 80% | 1× | Wide→Narrow (area-efficient) |
| axi_data_dnsize | Dual buffer | 100% | 2× | Wide→Narrow (high-performance) |

## Architecture Overview

### Component Hierarchy

Converters Component
├── Data Width Converters:
│   ├── Generic Building Blocks:
│   │   ├── axi_data_upsize.sv      - Narrow→Wide accumulator (100% throughput)
│   │   ├── axi_data_dnsize.sv      - Wide→Narrow splitter (80% or 100% throughput)
│   │   └── Key Features:
│   │       ├── Configurable sideband handling
│   │       ├── Optional burst tracking
│   │       └── Dual-buffer mode (dnsize only)
│   │
│   └── Full AXI4 Converters:
│       ├── axi4_dwidth_converter_wr.sv  - Write path converter (AW + W + B channels)
│       ├── axi4_dwidth_converter_rd.sv  - Read path converter (AR + R channels)
│       └── Integration:
│           ├── Skid buffers for flow control
│           ├── Address phase management
│           └── Response path handling
│
└── Protocol Converters:
    ├── axi4_to_apb_convert.sv   - AXI4-to-APB bridge (full protocol conversion)
    ├── axi4_to_apb_shim.sv      - AXI4-to-APB adapter (simplified wrapper)
    └── peakrdl_to_cmdrsp.sv     - PeakRDL adapter (register→command/response)

### Data Flow Examples

Write Path Downsize (512→128 bits):

┌────────────────────────────────────────────────────┐
│ Slave Interface (512-bit)                          │
│   ┌──────────┐                                     │
│   │ AW       │ ← 1 address                         │
│   │ W        │ ← 1 wide beat (512-bit + 64-bit WSTRB)
│   │ B        │ → 1 response                        │
│   └──────────┘                                     │
│       ↓                                            │
│   ┌──────────────────┐                             │
│   │ axi_data_dnsize  │ (DUAL_BUFFER=1)             │
│   │  512→128 split   │                             │
│   └──────────────────┘                             │
│       ↓                                            │
│   ┌──────────┐                                     │
│   │ Master Interface (128-bit)                     │
│   │ AW       │ → 1 address                         │
│   │ W        │ → 4 narrow beats (128-bit + 16-bit WSTRB each)
│   │ B        │ ← 1 response                        │
│   └──────────┘                                     │
└────────────────────────────────────────────────────┘

Read Path Upsize (128→512 bits):

┌────────────────────────────────────────────────────┐
│ Master Interface (512-bit)                         │
│   ┌──────────┐                                     │
│   │ AR       │ → 1 address                         │
│   │ R        │ ← 1 wide beat (512-bit + 2-bit RRESP)
│   └──────────┘                                     │
│       ↑                                            │
│   ┌──────────────────┐                             │
│   │ axi_data_upsize  │ (TRACK_BURSTS=1)            │
│   │  128→512 accum   │                             │
│   └──────────────────┘                             │
│       ↑                                            │
│   ┌──────────┐                                     │
│   │ Slave Interface (128-bit)                      │
│   │ AR       │ ← 1 address                         │
│   │ R        │ → 4 narrow beats (128-bit + 2-bit RRESP each)
│   └──────────┘                                     │
└────────────────────────────────────────────────────┘

## Module Documentation

### 1. axi_data_upsize.sv - Narrow→Wide Accumulator

Purpose: Accumulates multiple narrow beats into a single wide beat

Throughput: 100% (single buffer sufficient)

Key Parameters:

parameter int NARROW_WIDTH    = 32;      // Input data width
parameter int WIDE_WIDTH      = 128;     // Output data width (must be integer multiple)
parameter int NARROW_SB_WIDTH = 4;       // Narrow sideband width (WSTRB: WIDTH/8)
parameter int WIDE_SB_WIDTH   = 16;      // Wide sideband width (WSTRB: WIDTH/8)
parameter int SB_OR_MODE      = 0;       // 0=concatenate (WSTRB), 1=OR (RRESP)

Sideband Modes: - SB_OR_MODE=0 (Concatenate): For WSTRB - assemble strobe bits narrow[0].wstrb = 4'b1111 → wide.wstrb[3:0]   narrow[1].wstrb = 4'b1100 → wide.wstrb[7:4]   narrow[2].wstrb = 4'b0011 → wide.wstrb[11:8]   narrow[3].wstrb = 4'b1111 → wide.wstrb[15:12]   Result: wide.wstrb = 16'b1111_0011_1100_1111

- SB_OR_MODE=1 (OR Together): For RRESP - propagate errors

- narrow[0].rresp = 2'b00 (OK)
narrow[1].rresp = 2'b10 (SLVERR)
narrow[2].rresp = 2'b00 (OK)
narrow[3].rresp = 2'b00 (OK)
Result: wide.rresp = 2'b10 (SLVERR - any error propagates)

Usage Example:

axi_data_upsize #(
    .NARROW_WIDTH(128),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(16),  // WSTRB: 128/8 = 16
    .WIDE_SB_WIDTH(64),    // WSTRB: 512/8 = 64
    .SB_OR_MODE(0)         // Concatenate WSTRB
) u_upsize (
    .aclk             (aclk),
    .aresetn          (aresetn),

    // Narrow input
    .narrow_valid     (s_wvalid),
    .narrow_ready     (s_wready),
    .narrow_data      (s_wdata),
    .narrow_sideband  (s_wstrb),
    .narrow_last      (s_wlast),

    // Wide output
    .wide_valid       (m_wvalid),
    .wide_ready       (m_wready),
    .wide_data        (m_wdata),
    .wide_sideband    (m_wstrb),
    .wide_last        (m_wlast)
);

### 2. axi_data_dnsize.sv - Wide→Narrow Splitter

Purpose: Splits single wide beat into multiple narrow beats

Throughput: - Single-buffer mode (DUAL_BUFFER=0): 80% (1-cycle gap per wide beat) - Dual-buffer mode (DUAL_BUFFER=1): 100% (continuous streaming)

Key Parameters:

parameter int WIDE_WIDTH        = 512;     // Input data width
parameter int NARROW_WIDTH      = 128;     // Output data width (must be integer divisor)
parameter int WIDE_SB_WIDTH     = 2;       // Wide sideband width (RRESP: 2 bits)
parameter int NARROW_SB_WIDTH   = 2;       // Narrow sideband width
parameter int SB_BROADCAST      = 1;       // 1=broadcast (RRESP), 0=slice (WSTRB)
parameter int TRACK_BURSTS      = 0;       // 1=track bursts for LAST, 0=simple passthrough
parameter int BURST_LEN_WIDTH   = 8;       // Burst length counter width
parameter int DUAL_BUFFER       = 0;       // 1=dual buffer (100% throughput, 2× area)
                                           // 0=single buffer (80% throughput, 1× area)

Sideband Modes: - SB_BROADCAST=1: Broadcast same value to all narrow beats (RRESP) wide.rresp = 2'b10 (SLVERR)   → narrow[0].rresp = 2'b10   → narrow[1].rresp = 2'b10   → narrow[2].rresp = 2'b10   → narrow[3].rresp = 2'b10

- SB_BROADCAST=0: Slice into narrow portions (WSTRB)

- wide.wstrb = 64'h000F_F0FF_00FF_FFFF
→ narrow[0].wstrb[15:0]  = 16'hFFFF
→ narrow[1].wstrb[15:0]  = 16'h00FF
→ narrow[2].wstrb[15:0]  = 16'hF0FF
→ narrow[3].wstrb[15:0]  = 16'h000F

Burst Tracking Mode: - TRACK_BURSTS=0: Pass wide_last to last narrow beat (simple mode) - TRACK_BURSTS=1: Generate LAST on final beat of entire burst (read path)

Dual-Buffer Mode (NEW in v1.1):

Single-buffer mode achieves 80% throughput due to 1-cycle gap when transitioning between wide beats:

Cycle: 1    2    3    4    5    6    7    8    9   10
Wide:  [====BEAT_0====]  WAIT [====BEAT_1====]  WAIT
Narrow: n0   n1   n2   n3  IDLE  n0   n1   n2   n3  IDLE
                            ↑ 1-cycle dead time

Dual-buffer mode eliminates the gap by ping-ponging between two buffers:

Cycle: 1    2    3    4    5    6    7    8    9
Wide:  [====BEAT_0====][====BEAT_1====][====BEAT_2====]
Narrow: n0   n1   n2   n3   n0   n1   n2   n3   n0
Buffer: BUF0 reading────┘   BUF1 reading────┘   BUF0...
        BUF1 writing────────┘   BUF0 writing────┘

When to Use Dual-Buffer Mode: - High-bandwidth DMA engines with continuous streaming - Performance-critical data paths where 100% utilization required - Sufficient area budget (~2× increase for buffer registers)

When to Use Single-Buffer Mode: - Area-constrained designs - Throughput requirements <100% - Natural gaps in traffic from upstream/downstream

Usage Example (Single-Buffer):

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(2),      // RRESP: 2 bits
    .NARROW_SB_WIDTH(2),
    .SB_BROADCAST(1),       // Broadcast RRESP
    .TRACK_BURSTS(1),       // Track bursts for LAST
    .BURST_LEN_WIDTH(8),
    .DUAL_BUFFER(0)         // Single buffer (80% throughput, area-efficient)
) u_dnsize (
    .aclk             (aclk),
    .aresetn          (aresetn),

    // Burst control (TRACK_BURSTS=1)
    .burst_len        (arlen),
    .burst_start      (arvalid && arready),

    // Wide input
    .wide_valid       (s_rvalid),
    .wide_ready       (s_rready),
    .wide_data        (s_rdata),
    .wide_sideband    (s_rresp),
    .wide_last        (s_rlast),

    // Narrow output
    .narrow_valid     (m_rvalid),
    .narrow_ready     (m_rready),
    .narrow_data      (m_rdata),
    .narrow_sideband  (m_rresp),
    .narrow_last      (m_rlast)
);

Usage Example (Dual-Buffer - High Performance):

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),     // WSTRB: 512/8 = 64
    .NARROW_SB_WIDTH(16),   // WSTRB: 128/8 = 16
    .SB_BROADCAST(0),       // Slice WSTRB
    .TRACK_BURSTS(0),       // Simple mode for write path
    .DUAL_BUFFER(1)         // Dual buffer (100% throughput, 2× area)
) u_dnsize_hp (
    .aclk             (aclk),
    .aresetn          (aresetn),

    // Burst control (unused in TRACK_BURSTS=0)
    .burst_len        (8'd0),
    .burst_start      (1'b0),

    // Wide input
    .wide_valid       (s_wvalid),
    .wide_ready       (s_wready),
    .wide_data        (s_wdata),
    .wide_sideband    (s_wstrb),
    .wide_last        (s_wlast),

    // Narrow output
    .narrow_valid     (m_wvalid),
    .narrow_ready     (m_wready),
    .narrow_data      (m_wdata),
    .narrow_sideband  (m_wstrb),
    .narrow_last      (m_wlast)
);

### 3. Full AXI4 Converters

axi4_dwidth_converter_wr.sv - Complete write path converter (AW + W + B)

axi4_dwidth_converter_rd.sv - Complete read path converter (AR + R)

These integrate the generic building blocks with: - Address phase management - Skid buffers for flow control - Response path handling - Full AXI4 protocol compliance

## Protocol Converters

### 4. AXI4-to-APB Bridge (axi4_to_apb_convert.sv)

Purpose: Full protocol conversion from AXI4 to APB with address/data width adaptation

Key Features: - Converts AXI4 read/write transactions to APB protocol - Handles address width conversion (AXI4 64-bit → APB 32-bit) - Data width adaptation (configurable) - State machine for AXI→APB protocol translation - Error response handling (SLVERR/DECERR)

Usage Example:

axi4_to_apb_convert #(
    .S_AXI_ADDR_WIDTH(64),
    .S_AXI_DATA_WIDTH(32),
    .M_APB_ADDR_WIDTH(32),
    .M_APB_DATA_WIDTH(32)
) u_axi_apb_bridge (
    .aclk           (clk),
    .aresetn        (rst_n),

    // AXI4 Slave Interface
    .s_axi_awaddr   (s_awaddr),
    .s_axi_awvalid  (s_awvalid),
    .s_axi_awready  (s_awready),
    // ... (full AXI4 AW/W/B/AR/R channels)

    // APB Master Interface
    .m_apb_paddr    (m_paddr),
    .m_apb_psel     (m_psel),
    .m_apb_penable  (m_penable),
    .m_apb_pwrite   (m_pwrite),
    .m_apb_pwdata   (m_pwdata),
    .m_apb_pready   (m_pready),
    .m_apb_prdata   (m_prdata),
    .m_apb_pslverr  (m_pslverr)
);

Use Cases: - Connecting AXI4 masters to APB peripherals - CPU to APB peripheral bus bridges - System integration with mixed protocols

### 5. PeakRDL-to-CmdRsp Adapter (peakrdl_to_cmdrsp.sv)

Purpose: Convert PeakRDL-generated register interface to custom command/response protocol

Key Features: - Converts APB-style register interface to command/response handshake - Supports read/write operations - Configurable command/response data widths - Single-cycle command issue - Pipelined response handling

Usage Example:

peakrdl_to_cmdrsp #(
    .ADDR_WIDTH(16),
    .DATA_WIDTH(32)
) u_peakrdl_adapter (
    .clk            (clk),
    .rst_n          (rst_n),

    // PeakRDL Register Interface (APB-style)
    .reg_addr       (reg_addr),
    .reg_wdata      (reg_wdata),
    .reg_write      (reg_write),
    .reg_read       (reg_read),
    .reg_rdata      (reg_rdata),
    .reg_error      (reg_error),

    // Command/Response Interface
    .cmd_valid      (cmd_valid),
    .cmd_ready      (cmd_ready),
    .cmd_addr       (cmd_addr),
    .cmd_data       (cmd_data),
    .cmd_write      (cmd_write),

    .rsp_valid      (rsp_valid),
    .rsp_ready      (rsp_ready),
    .rsp_data       (rsp_data),
    .rsp_error      (rsp_error)
);

Use Cases: - Interfacing PeakRDL register blocks to custom control logic - Register access through command/response protocol - Decoupling register interface from implementation

## Configuration Examples

### Example 1: Write Path Downsize (128→32 bits)

Use Case: ARM Cortex-M7 (128-bit AXI) → APB Bridge (32-bit)

axi_data_dnsize #(
    .WIDE_WIDTH(128),
    .NARROW_WIDTH(32),
    .WIDE_SB_WIDTH(16),     // WSTRB: 128/8 = 16
    .NARROW_SB_WIDTH(4),    // WSTRB: 32/8 = 4
    .SB_BROADCAST(0),       // Slice WSTRB
    .TRACK_BURSTS(0),       // Write path: simple mode
    .DUAL_BUFFER(0)         // Area-efficient
) u_wr_dnsize (
    // ... ports
);

Result: 1 wide W beat (128-bit) → 4 narrow W beats (32-bit each)

### Example 2: Read Path Upsize (256→512 bits)

Use Case: DDR4 Controller (512-bit) ← FPGA Fabric (256-bit)

axi_data_upsize #(
    .NARROW_WIDTH(256),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(2),    // RRESP: 2 bits
    .WIDE_SB_WIDTH(2),      // RRESP: 2 bits
    .SB_OR_MODE(1)          // OR together RRESP
) u_rd_upsize (
    // ... ports
);

Result: 2 narrow R beats (256-bit) → 1 wide R beat (512-bit)

### Example 3: High-Performance Write Downsize (512→128 bits)

Use Case: DMA Engine (512-bit) → PCIe Endpoint (128-bit), continuous streaming

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),     // WSTRB: 512/8 = 64
    .NARROW_SB_WIDTH(16),   // WSTRB: 128/8 = 16
    .SB_BROADCAST(0),       // Slice WSTRB
    .TRACK_BURSTS(0),       // Write path: simple mode
    .DUAL_BUFFER(1)         // HIGH-PERFORMANCE: 100% throughput
) u_wr_dnsize_hp (
    // ... ports
);

Result: 100% throughput (no dead cycles), 2× area cost

## Testing

### Test Organization

projects/components/converters/dv/tests/
├── test_axi_data_upsize.py       - Generic upsize module tests
├── test_axi_data_dnsize.py       - Generic dnsize module tests (16 configs)
├── test_axi4_dwidth_converter_wr.py  - Full write converter tests
└── test_axi4_dwidth_converter_rd.py  - Full read converter tests

### Test Configurations (axi_data_dnsize.py)

Single-Buffer Tests (DUAL_BUFFER=0): 1. 128→32 WSTRB slice (simple mode) 2. 256→64 WSTRB slice (simple mode) 3. 128→32 RRESP broadcast (simple mode) 4. 256→64 RRESP broadcast (simple mode) 5. 128→32 RRESP broadcast (burst tracking) 6. 256→64 RRESP broadcast (burst tracking) 7. 512→128 RRESP broadcast (burst tracking) 8. 128→64 no sideband (simple mode)

Dual-Buffer Tests (DUAL_BUFFER=1): 9-16. Same configurations as above with DUAL_BUFFER=1

### Running Tests

# Run all converter tests
cd $REPO_ROOT/projects/components/converters/dv/tests
make run-all-parallel          # FUNC level, 48 workers

# Run specific module tests
make run-dnsize-func           # Downsize tests
make run-upsize-func           # Upsize tests

# Run with different test levels
make run-all-gate-parallel     # Quick smoke test
make run-all-func-parallel     # Functional coverage (default)
make run-all-full-parallel     # Comprehensive validation

# Individual test
pytest test_axi_data_dnsize.py::test_axi_data_dnsize[128to32_wstrb_slice_simple_DUAL] -v

## Quality Assurance

### Lint Checks

# Run all lint tools
cd $REPO_ROOT/projects/components/converters/rtl
make lint-all

# Individual tools
make verilator    # Verilator lint
make verible      # Verible style check
make yosys        # Yosys synthesis check

# View status
make status

### Expected Lint Results

axi_data_upsize.sv and axi_data_dnsize.sv: - Clean compilation (warnings only for unused signals in certain parameter configurations) - Warnings are benign (e.g., narrow_sideband unused when NARROW_SB_WIDTH=0)

**axi4_dwidth_converter_*.sv:** - Requires rtl/amba/gaxi/ include path for skid buffer modules - PINCONNECTEMPTY warnings are expected (unused count outputs)

## Documentation

### Available Documentation

- README.md (this file) - Quick start and overview

- GENERIC_MODULES_USAGE_GUIDE.md - Detailed parameter guide for upsize/dnsize

- DUAL_BUFFER_IMPLEMENTATION.md - Comprehensive dual-buffer feature documentation

- ANALYSIS_APB_CONVERTER.md - APB protocol converter analysis

### Key Resources

For Understanding Parameters:

cat $REPO_ROOT/projects/components/converters/rtl/GENERIC_MODULES_USAGE_GUIDE.md

For Dual-Buffer Mode:

cat $REPO_ROOT/projects/components/converters/DUAL_BUFFER_IMPLEMENTATION.md

For APB Integration:

cat $REPO_ROOT/projects/components/converters/ANALYSIS_APB_CONVERTER.md

## Quick Commands

# Setup environment
source $REPO_ROOT/env_python

# Run all tests (parallel)
cd $REPO_ROOT/projects/components/converters/dv/tests
make run-all-parallel

# Lint all RTL
cd $REPO_ROOT/projects/components/converters/rtl
make lint-all

# View test status
cd $REPO_ROOT/projects/components/converters/dv/tests
make status

# Clean all artifacts
make clean-all

## Design Decisions

### Why No Dual-Buffer for Upsize?

axi_data_upsize already achieves 100% throughput with single buffer: - Can accept narrow beat while outputting wide beat simultaneously - The || wide_ready term in narrow_ready enables pipelining - No benefit from dual buffering

### Why Optional Dual-Buffer for Dnsize?

Trade-off between area and performance: - Single-buffer: 80% throughput, 1× area (good for most use cases) - Dual-buffer: 100% throughput, 2× area (high-performance paths) - User can select based on system requirements

### Why Separate Upsize/Dnsize Modules?

Promotes reuse and flexibility: - Can be used independently in custom converters - Write converter: upsize + dnsize combination - Read converter: dnsize + upsize combination - Other use cases: data width matching, FIFO interfaces, etc.

## Known Limitations

- Width Ratio Constraint: WIDE_WIDTH must be exact integer multiple of NARROW_WIDTH

- Alignment: Full converters require aligned addresses (handled by full converter modules)

- Burst Length: Burst tracking mode supports AXI4 burst lengths (BURST_LEN_WIDTH=8)

- Sideband Width: Must match data width ratios for slice mode

## Future Enhancements

- Configurable Buffer Depth: Allow >2 buffers for higher throughput

- Performance Counters: Monitor stalls, utilization, throughput

- Power Gating: Disable unused buffer in dual-buffer mode

- Credit-Based Flow Control: Integration with upstream credit systems

## Version History

- v1.1 (2025-10-25): Added dual-buffer mode for axi_data_dnsize (100% throughput)

- v1.0 (2025-10-24): Initial release with generic modules and full converters

## Related Components

- STREAM - DMA engine using width converters for descriptor fetch

- RAPIDS - Accelerator with width conversion on data paths

- APB HPET - APB peripheral using narrow interfaces

- Bridge - Protocol converters with width adaptation

Author: RTL Design Sherpa Project Last Updated: 2025-10-25

# Dual-Buffer Implementation for axi_data_dnsize

Date: 2025-10-25 Purpose: Document the dual-buffer feature implementation for high-throughput data width conversion

## Executive Summary

Added optional dual-buffer mode to axi_data_dnsize.sv module to achieve 100% throughput for continuous data streams.

Key Results: - Single-buffer mode (DUAL_BUFFER=0): 80% throughput, 1-cycle dead time per wide beat - Dual-buffer mode (DUAL_BUFFER=1): 100% throughput, zero dead cycles - Area cost: Approximately 2× buffer registers (+100% overhead) - Compatibility: Fully backward compatible, works with all existing test configurations

## Architecture Overview

### Single-Buffer Mode (DUAL_BUFFER=0)

Structure:

┌─────────────────┐
│  r_data_buffer  │  ← ONE wide beat buffered
│  (WIDE_WIDTH)   │
└─────────────────┘
     ↓
  Split into
  narrow beats

Throughput Limitation: - Must wait for ALL narrow beats to complete before accepting next wide beat - 1-cycle gap when transitioning between wide beats - Throughput = 4/(4+1) = 80% for 4:1 ratio

### Dual-Buffer Mode (DUAL_BUFFER=1)

Structure:

┌─────────────┐
│  r_buffer_0  │  ← Splitting into narrow beats
│              │
└─────────────┘
     ↓ (reading)

┌─────────────┐
│  r_buffer_1  │  ← Accepting NEXT wide beat
│              │
└─────────────┘
     ↑ (writing)

Ping-Pong Operation: 1. Buffer 0 splits while Buffer 1 accepts new data 2. When Buffer 0 completes, swap: Buffer 1 splits, Buffer 0 accepts 3. Continuous operation with no dead cycles

Throughput: 100% (4/4) for any ratio

## Implementation Details

### New Parameter

parameter int DUAL_BUFFER = 0,  // 0=single buffer (80% throughput, area efficient)
                                // 1=dual buffer (100% throughput, 2× area)

### State Variables

Single-Buffer Mode:

logic [WIDE_WIDTH-1:0]          r_data_buffer;
logic [WIDE_SB_PORT_WIDTH-1:0]  r_sideband_buffer;
logic                           r_wide_buffered;
logic                           r_last_buffered;

Dual-Buffer Mode:

logic [WIDE_WIDTH-1:0]          r_buffer_0, r_buffer_1;
logic [WIDE_SB_PORT_WIDTH-1:0]  r_sb_buffer_0, r_sb_buffer_1;
logic                           r_last_buffer_0, r_last_buffer_1;
logic                           r_buffer_0_valid, r_buffer_1_valid;
logic                           r_read_buffer;  // 0=reading buf0, 1=reading buf1

### Write Path (DUAL_BUFFER=1)

if (wide_valid && wide_ready) begin
    if (!gen_dual_buffer.r_buffer_0_valid) begin
        // Write to buffer 0
        gen_dual_buffer.r_buffer_0 <= wide_data;
        gen_dual_buffer.r_buffer_0_valid <= 1'b1;
    end else begin
        // Write to buffer 1 (must be empty if wide_ready=1)
        gen_dual_buffer.r_buffer_1 <= wide_data;
        gen_dual_buffer.r_buffer_1_valid <= 1'b1;
    end
end

Key: Always write to the empty buffer.

### Read Path (DUAL_BUFFER=1)

// Read from current buffer
if (current_buffer_valid && narrow_ready) begin
    if (is_last_narrow_beat) begin
        // Clear current buffer's valid flag
        if (gen_dual_buffer.r_read_buffer)
            gen_dual_buffer.r_buffer_1_valid <= 1'b0;
        else
            gen_dual_buffer.r_buffer_0_valid <= 1'b0;

        // Swap to other buffer if it has data
        if (other_buffer_valid)
            gen_dual_buffer.r_read_buffer <= ~gen_dual_buffer.r_read_buffer;
    end
end

Key: On last narrow beat, clear current buffer and swap to other if available.

### Ready Logic

Single-Buffer:

assign wide_ready = !r_wide_buffered || (narrow_ready && w_last_narrow_beat);

- Ready when buffer empty OR sending last narrow beat (1-cycle early)

Dual-Buffer:

assign wide_ready = !r_buffer_0_valid || !r_buffer_1_valid;

- Ready when at least one buffer is empty

- Allows continuous acceptance

## Burst Tracking Integration

Dual-buffer mode fully supports burst tracking (TRACK_BURSTS=1):

Challenge: Track burst position across buffer swaps

Solution: Shared burst counter applies to whichever buffer is currently being read:

// Burst tracking state (shared between buffers)
logic [BURST_LEN_WIDTH-1:0] r_slave_beat_count;
logic [BURST_LEN_WIDTH-1:0] r_slave_total_beats;
logic                       r_burst_active;

// Applies to current read buffer
if (((r_slave_beat_count + 1'b1) >= r_slave_total_beats)) begin
    // Last narrow beat of entire burst
    // Clear current buffer and end burst
end

Result: LAST signal correctly generated on final beat of burst, regardless of buffer swapping.

## Resource Impact

### Area Analysis (128-bit Wide, 32-bit Narrow, 16-bit Sideband)

| Resource | Single Buffer | Dual Buffer | Overhead |
| --- | --- | --- | --- |
| Data registers | 128 bits | 256 bits | +128 bits |
| Sideband registers | 16 bits | 32 bits | +16 bits |
| LAST flags | 1 bit | 2 bits | +1 bit |
| Valid flags | 1 bit | 2 bits | +1 bit |
| Read selector | - | 1 bit | +1 bit |
| Total FFs | 146 | 292 | +146 (+100%) |

Control Logic: ~30% increase (buffer selection, swap logic)

Overall: Expect ~100% area increase for dual-buffer mode.

### Performance Impact

| Mode | Throughput (4:1 Ratio) | Cycles/Wide Beat | Utilization |
| --- | --- | --- | --- |
| Single-Buffer | 80% | 5 (4 active + 1 dead) | 4/5 |
| Dual-Buffer | 100% | 4 (4 active + 0 dead) | 4/4 |

Improvement: +25% throughput (from 80% to 100%)

## Test Coverage

### Test Configurations Added

All existing test configurations now have dual-buffer variants:

Single-Buffer Tests (DUAL_BUFFER=0): - 128to32_wstrb_slice_simple - 256to64_wstrb_slice_simple - 128to32_rresp_broadcast_simple - 256to64_rresp_broadcast_simple - 128to32_rresp_burst_track - 256to64_rresp_burst_track - 512to128_rresp_burst_track - 128to64_no_sideband_simple

Dual-Buffer Tests (DUAL_BUFFER=1): - Same configurations with “_DUAL” suffix - Total: 16 test configurations

### Verification Results

Tested: - ✓ Basic data splitting (all ratios) - ✓ Sideband handling (broadcast and slice modes) - ✓ Burst tracking with LAST generation - ✓ Backpressure handling - ✓ Continuous streaming

Result: All tests PASS for both single and dual-buffer modes.

## Usage Recommendations

### When to Use Single-Buffer Mode (DUAL_BUFFER=0)

✓ Use when: - Area is critical - Throughput requirements are <100% - Source/sink have natural gaps in traffic - Design is already bottlenecked elsewhere

Example: Write path downsize where upstream has occasional pauses

### When to Use Dual-Buffer Mode (DUAL_BUFFER=1)

✓ Use when: - Maximum throughput required - Continuous streaming data - Sufficient area budget - Critical path in high-performance system

Example: High-bandwidth DMA engine with continuous transfers

## Code Example

### Instantiation: Single-Buffer Mode (Area Efficient)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),      // WSTRB
    .NARROW_SB_WIDTH(16),    // WSTRB
    .SB_BROADCAST(0),        // Slice mode
    .TRACK_BURSTS(0),        // Simple mode
    .DUAL_BUFFER(0)          // Single buffer (80% throughput)
) u_dnsize (
    .aclk(clk),
    .aresetn(rst_n),
    // ... ports
);

### Instantiation: Dual-Buffer Mode (High Throughput)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),
    .NARROW_SB_WIDTH(16),
    .SB_BROADCAST(0),
    .TRACK_BURSTS(0),
    .DUAL_BUFFER(1)          // Dual buffer (100% throughput)
) u_dnsize (
    .aclk(clk),
    .aresetn(rst_n),
    // ... ports
);

## Design Decisions

### Why Not Dual-Buffer for Upsize?

axi_data_upsize already achieves 100% throughput with single buffer: - Can accept narrow beat while outputting wide beat simultaneously - The || wide_ready term in narrow_ready enables overlap - No benefit from dual buffering

Conclusion: Dual-buffer only needed for dnsize module.

### Why Use Generate Blocks?

Rationale: - Complete separation of single vs. dual-buffer logic - No runtime overhead (compile-time selection) - Easier to verify each mode independently - Cleaner code structure

### Why Not Make Dual-Buffer the Default?

Considerations: - 100% area overhead is significant - Many use cases don’t need 100% throughput - Single-buffer is simpler and well-tested - User can opt-in when needed

Decision: Default to DUAL_BUFFER=0, user explicitly enables when needed.

## Lessons Learned

### SystemVerilog Constraints

Issue: Cannot declare logic variables inside procedural blocks

// ILLEGAL:
always_ff @(posedge clk) begin
    logic temp;  // ← NOT ALLOWED
    temp = ...;
end

Solution: Inline expressions or move declarations outside block

### Generate Block Naming

Best Practice: Use hierarchical naming for generate block signals:

gen_dual_buffer.r_buffer_0_valid  // Clear which mode
gen_single_buffer.r_wide_buffered // Clear which mode

### Buffer Swap Logic

Key Insight: Swap only when: 1. Current buffer completes (last narrow beat sent) 2. Other buffer has valid data waiting

This prevents unnecessary swaps and maintains correct ordering.

## Future Enhancements

### Potential Improvements

- Configurable Buffer Depth: Allow >2 buffers for even higher throughput

- Credit-Based Flow Control: Integrate with upstream credit system

- Performance Counters: Monitor buffer utilization, stalls

- Power Gating: Disable unused buffer when not needed

### Alternative Architectures

Skid Buffer Approach: - Insert skid buffers on outputs instead of dual data buffers - Lower area overhead - Similar throughput improvement

FIFO Approach: - Replace buffers with small FIFOs - More flexible depth control - Higher area cost

## Summary

The dual-buffer implementation successfully provides an optional high-throughput mode for the axi_data_dnsize module:

✓ Functionality: Proven correct through comprehensive testing ✓ Performance: 100% throughput vs. 80% for single-buffer ✓ Compatibility: Fully backward compatible ✓ Flexibility: User-selectable via parameter ✓ Robustness: Works with all modes (broadcast, slice, burst tracking)

Trade-off: ~100% area increase for +25% throughput improvement

Recommendation: Use dual-buffer mode for performance-critical paths with continuous data flow.

## Related Files

- projects/components/converters/rtl/axi_data_dnsize.sv - RTL implementation

- projects/components/converters/dv/tests/test_axi_data_dnsize.py - Test suite

- projects/components/converters/dv/tbclasses/axi_data_dnsize_tb.py - Testbench class

- projects/components/converters/USAGE.md - Usage documentation

- projects/components/converters/ANALYSIS_APB_CONVERTER.md - APB converter analysis

Author: RTL Design Sherpa Date: 2025-10-25

# Analysis: APB Converter vs. Generic Data Width Converters

Date: 2025-10-25 Purpose: Analyze whether axi4_to_apb_convert.sv could benefit from generic axi_data_upsize and axi_data_dnsize modules

## Executive Summary

Finding: The APB converter implements data width conversion using identical algorithmic patterns to our generic modules, but in a fundamentally different usage model that makes direct integration impractical.

Recommendation: Do NOT refactor the APB converter to use generic modules. The tight coupling between protocol conversion and data width conversion requires the current inline implementation.

Value: The analysis validates that our generic module algorithms are correct - an independent implementation arrived at the same solution.

## Data Width Conversion Patterns Found

### 1. WRITE Path (Dnsize: Wide AXI → Narrow APB)

Location: axi4_to_apb_convert.sv lines 334, 337, 461

Implementation:

// Line 334 - Extract narrow data slice
w_apb_cmd_pkt_pwdata = (axi2abpratio == 1) ? r_s_axi_wdata[APBDW-1:0] :
                        r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW];

// Line 337 - Extract narrow strobe slice
w_apb_cmd_pkt_pstrb = (axi2abpratio == 1) ? r_s_axi_wstrb[APBSW-1:0] :
                       r_s_axi_wstrb[r_axi_wr_data_pointer*APBSW +: APBSW];

// Line 461 - Increment pointer through wide beat
w_axi_wr_data_pointer = r_axi_wr_data_pointer + 1;

Pattern: IDENTICAL to axi_data_dnsize module (line 210, 220) - Extract slice using pointer: data[ptr*WIDTH +: WIDTH] - Increment pointer for next narrow beat - Wrap at ratio-1

### 2. READ Path (Upsize: Narrow APB → Wide AXI)

Location: axi4_to_apb_convert.sv lines 283, 366

Implementation:

// Line 283 - Accumulate narrow data into wide shift register
r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= r_apb_rsp_pkt_prdata;

// Line 366 - Combinational accumulation (alternative path)
w_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] = r_apb_rsp_pkt_prdata;

// Lines 284-287 - Increment and wrap pointer
r_axi_rsp_data_pointer <= r_axi_rsp_data_pointer + 1;
if (r_axi_rsp_data_pointer == PTR_WIDTH'(axi2abpratio-1)) begin
    r_axi_rsp_data_pointer <= 'b0;
end

Pattern: IDENTICAL to axi_data_upsize module (line 158) - Accumulate into shift register at pointer position - Increment pointer for next narrow beat - Complete wide beat when pointer reaches ratio-1

### 3. Pointer Management

APB Converter has THREE separate pointers: - r_axi_wr_data_pointer - WRITE path dnsize (lines 315-317) - r_axi_rd_data_pointer - READ path dnsize (lines 309-311) - r_axi_rsp_data_pointer - READ response upsize (lines 284-287)

Generic Modules have ONE pointer each: - axi_data_dnsize: r_beat_ptr for splitting wide→narrow - axi_data_upsize: r_beat_ptr for accumulating narrow→wide

## Key Architectural Differences

### Generic Modules (axi_data_upsize / axi_data_dnsize)

Usage Model: Complete Beat Processing - axi_data_upsize: Wait for ALL narrow beats → output ONE complete wide beat - axi_data_dnsize: Accept ONE wide beat → output ALL narrow beats in sequence - Pure streaming pipeline with valid/ready handshaking - Standalone operation - can be inserted in any data path

State Machine: - Simple: Buffer empty/full state - No burst tracking (optional feature) - Focus solely on data width conversion

Example Flow (128→32 upsize):

1. Receive narrow beat 0 (bits [31:0])   → accumulate, ptr=1
2. Receive narrow beat 1 (bits [63:32])  → accumulate, ptr=2
3. Receive narrow beat 2 (bits [95:64])  → accumulate, ptr=3
4. Receive narrow beat 3 (bits [127:96]) → accumulate, OUTPUT wide beat, ptr=0

### APB Converter (axi4_to_apb_convert.sv)

Usage Model: Incremental Processing Within Protocol State Machine - Converts incrementally as APB transactions complete - Does NOT wait for complete wide beats - Interleaves conversion with protocol translation

State Machine: - Complex: IDLE, READ, WRITE states - Manages burst counters (r_burst_count) - Handles AXI burst to multiple APB transactions - FIRST/LAST packet generation - FIFO management for side channel data

Example Flow (128→32 write path):

1. Receive AXI AWADDR + WDATA[127:0] (full wide beat)
2. Enter WRITE state
3. APB transaction 0: Extract WDATA[31:0],   ptr=1, send to APB
4. APB transaction 1: Extract WDATA[63:32],  ptr=2, send to APB
5. APB transaction 2: Extract WDATA[95:64],  ptr=3, send to APB
6. APB transaction 3: Extract WDATA[127:96], ptr=0, complete AXI beat
7. If burst continues, repeat for next AXI beat

Critical Difference: Conversion happens within the protocol state machine, one APB transaction at a time.

## Refactoring Analysis

### Option 1: Keep Current Inline Implementation ✓ RECOMMENDED

Pros: - ✓ Already working and tested - ✓ Tight integration with protocol state machine - ✓ No additional latency - ✓ Optimal resource usage - ✓ Clear code flow for incremental conversion

Cons: - ⚠ Code duplication of conversion patterns - ⚠ Complex state machine mixing protocol + data concerns - ⚠ Harder to verify conversion logic independently

Verdict: Best choice. The tight coupling is actually necessary for the protocol conversion requirements.

### Option 2: Refactor to Use Generic Modules ✗ NOT RECOMMENDED

Conceptual Architecture:

AXI4 (DW-bit) → axi_data_dnsize → AXI4 (APBDW-bit) → axi4_to_apb_convert → APB (APBDW-bit)
                                      ↑
                              Width-matched AXI

Why This DOESN’T Work:

- Incremental Processing Conflict:

  - Generic modules process complete beats

  - APB converter processes incrementally with state machine

  - No clean insertion point for pipeline stages

- State Machine Coupling:

  - Conversion is tightly coupled with burst management

  - Pointer increments synchronized with APB transaction completions

  - FIRST/LAST generation depends on both protocol and conversion state

- Complexity Increase:

  - Would need complex glue logic to bridge generic modules to state machine

  - Additional FIFOs to buffer partial conversions

  - More difficult to reason about system behavior

- Resource Overhead:

  - Extra registers for generic module state

  - Additional control logic for coordination

  - Larger design for no functional benefit

- Verification Burden:

  - More complex interactions to verify

  - Harder to achieve coverage

  - Debugging becomes more difficult

Verdict: Refactoring would make the design more complex without providing any benefits.

## Validation: Independent Verification of Generic Module Algorithms

Finding: The APB converter independently arrived at identical data width conversion algorithms.

Validation Points:

- Slice Extraction (Dnsize):

  - APB converter line 334: r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW]

  - Generic dnsize line 210: r_data_buffer[r_beat_ptr*NARROW_WIDTH +: NARROW_WIDTH]

  - ✓ IDENTICAL pattern

- Accumulation (Upsize):

  - APB converter line 283: r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= data

  - Generic upsize line 158: r_data_accumulator[r_beat_ptr*NARROW_WIDTH +: NARROW_WIDTH] <= data

  - ✓ IDENTICAL pattern

- Pointer Management:

  - Both use same increment and wrap logic

  - Both use $clog2(RATIO) for pointer width

  - Both wrap at ratio-1

  - ✓ IDENTICAL pattern

Significance: Two independent implementations converging on the same solution provides strong evidence that our generic module algorithms are correct and optimal.

## Code Sections Reference

### APB Converter Data Width Conversion Code

Parameters:

// Line 39
parameter int AXI2APBRATIO = DW / APBDW,
parameter int PTR_WIDTH = $clog2(AXI2APBRATIO),

State Variables:

// Line 168
logic [DW-1:0] r_axi_data_shift, w_axi_data_shift;

// Lines 171-173 - THREE separate pointers
logic [PTR_WIDTH-1:0] r_axi_rd_data_pointer, w_axi_rd_data_pointer;
logic [PTR_WIDTH-1:0] r_axi_wr_data_pointer, w_axi_wr_data_pointer;
logic [PTR_WIDTH-1:0] r_axi_rsp_data_pointer, w_axi_rsp_data_pointer;

WRITE Path (Dnsize):

// Line 334 - Extract data slice
w_apb_cmd_pkt_pwdata = (axi2abpratio == 1) ? r_s_axi_wdata[APBDW-1:0] :
                        r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW];

// Line 337 - Extract strobe slice
w_apb_cmd_pkt_pstrb = (axi2abpratio == 1) ? r_s_axi_wstrb[APBSW-1:0] :
                       r_s_axi_wstrb[r_axi_wr_data_pointer*APBSW +: APBSW];

// Lines 315-317 - Increment and wrap
r_axi_wr_data_pointer <= r_axi_wr_data_pointer + 1;
if (r_axi_wr_data_pointer == PTR_WIDTH'(axi2abpratio-1))
    r_axi_wr_data_pointer <= 'b0;

READ Response Path (Upsize):

// Line 283 - Accumulate into shift register
r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= r_apb_rsp_pkt_prdata;

// Lines 284-287 - Increment and wrap
r_axi_rsp_data_pointer <= r_axi_rsp_data_pointer + 1'b1;
if (r_axi_rsp_data_pointer == PTR_WIDTH'(axi2abpratio-1)) begin
    r_axi_rsp_data_pointer <= 'b0;
end

// Line 366 - Combinational accumulation (alternative path)
w_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] = r_apb_rsp_pkt_prdata;

## Conclusion

### Summary

The APB converter implements data width conversion using patterns identical to our generic modules, but in a fundamentally different architectural context:

- Generic modules: Standalone streaming pipeline stages processing complete beats

- APB converter: Inline conversion within protocol state machine processing incrementally

### Recommendations

- Do NOT refactor APB converter to use generic modules

  - Current inline implementation is optimal for its use case

  - Refactoring would increase complexity without benefits

- Keep generic modules separate for their intended use cases:

  - Pure data width conversion in streaming pipelines

  - Protocol-agnostic width adaptation

  - Reusable components for future designs

- Document the relationship between the two implementations:

  - Both use same fundamental algorithms (validation)

  - Different usage models require different architectures

  - Not all width conversion scenarios suit pipeline approach

- Consider future opportunities:

  - Other protocol converters may benefit from generic modules

  - AXI-to-AXI width converters are prime candidates

  - Future designs should evaluate if pipeline model fits

### Value Delivered

- ✓ Validation - Independent implementation confirms algorithm correctness

- ✓ Clarity - Understand why inline vs. pipeline architectures differ

- ✓ Documentation - Captured design rationale for future reference

- ✓ Guidance - When to use generic modules vs. inline conversion

## Related Files

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi_data_upsize.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi_data_dnsize.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi4_to_apb_convert.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/USAGE.md

Author: RTL Design Sherpa Date: 2025-10-25