RTL Design Sherpa

RAPIDS Beats Hardware Architecture Specification 0.12

July 2, 2026

Table of Contents

1 Rapids Beats Has Index	27

2 Document Information	28

2.1 Purpose	29

2.2 Scope	30

2.3 Audience	31

2.4 Document Conventions	32

2.4.1 Requirement Levels	32

2.4.2 Signal Directions	32

2.4.3 Timing Diagrams	32

2.5 References	33

3 Revision History	34

3.1 Change Summary	38

3.1.1 Version 0.8 (2026-07-13)	38

3.1.2 Version 0.7 (2026-07-13)	38

3.1.3 Version 0.6 (2026-07-02)	38

3.1.4 Version 1.0 (2026-01-17)	39

4 Acronyms and Definitions	40

4.1 Acronyms	40

4.2 Definitions	41

4.3 Signal Naming Conventions	42

5 Product Overview	43

5.1 Introduction	43

5.2 System Context	44

5.2.1 Figure 1.1.1: RAPIDS Beats in Its System	44

5.3 Target Applications	45

5.4 Architecture Highlights	46

5.4.1 Phase 1 “Beats” Simplifications	46

5.4.2 Key Capabilities	46

6 Key Features	47

6.1 Feature Summary	47

6.1.1 Multi-Channel Architecture	47

6.1.2 Descriptor-Based Control	47

6.1.3 High-Performance Data Paths	47

6.1.4 Monitoring and Debug	47

6.2 Feature Details	48

6.2.1 SRAM Buffering	49

6.2.2 AXI Burst Optimization	49

6.2.3 Error Handling	50

7 System Context	51

7.1 Integration Overview	51

7.2 System Block Diagram	52

7.2.1 Figure 1.3.1: System Integration	52

7.3 Interface Connections	53

7.3.1 Memory Interface	53

7.3.2 Network Interface	53

7.3.3 Configuration Interface	54

7.4 Clock and Reset	55

7.4.1 Clock Domains	55

7.4.2 Reset Requirements	55

7.5 Power Considerations	56

7.5.1 Clock Gating	56

7.5.2 Power States	56

8 Block Diagram	57

8.1 Top-Level Architecture	57

8.1.1 Figure 2.1.1: RAPIDS Beats Whole Unit	57

8.2 Top-Level Integration (rapids_beats_top, rapids_beats_cxs_top)	58

8.2.1 Figure 2.1.2: rapids_beats_top / rapids_beats_cxs_top Integration	59

8.3 Component Summary	60

8.3.1 Scheduler Group Array	60

8.3.2 Sink Data Path	60

8.3.3 Source Data Path	60

8.4 Hierarchy	62

8.5 Internal Buses	63

9 Data Flow	64

9.1 Overview	64

9.2 Sink Path Data Flow	65

9.3 Concurrent Operation	68

9.3.1 Deadlock Prevention	68

10 Channel Architecture	69

10.1 Multi-Channel Design	69

10.2 Channel Resource Allocation	70

10.2.1 Figure 2.3.1: Per-Channel and Shared Resources	70

10.3 Per-Channel Resources	71

10.4 Shared Resources	72

10.5 Channel Scheduling	73

10.5.1 Arbitration Policy	73

10.5.2 Figure 2.3.2: Channel Arbitration onto a Shared AXI Port	73

10.6 Channel State Machine	74

10.7 Channel Isolation	76

10.7.1 Per-Channel Error Handling	76

11 Interface Summary	77

11.1 External Interface Overview	77

11.1.1 Figure 3.1.1: External Interfaces	77

11.2 Interface Port Summary	78

11.3 Clock and Reset	79

11.4 Interface Feature Matrix	80

11.5 Signal Count Summary	81

12 AXI4 Interface Specification	82

12.1 Overview	82

12.2 Descriptor AXI Master	83

12.2.1 Purpose	83

12.2.2 Configuration	83

12.2.3 Signal List	83

12.3 Sink AXI Master (Write)	85

12.3.1 Purpose	85

12.3.2 Configuration	85

12.3.3 Signal List	85

12.4 Source AXI Master (Read)	88

12.4.1 Purpose	88

12.4.2 Configuration	88

12.4.3 Signal List	88

12.5 AXI Protocol Constraints	90

12.5.1 Address Alignment	90

12.5.2 4KB Boundary Handling	90

12.5.3 Response Handling	90

13 AXIS Interface Specification	91

13.1 Overview	91

13.2 Sink AXIS Slave	92

13.2.1 Purpose	92

13.2.2 Configuration	92

13.2.3 Signal List	92

13.2.4 Backpressure Behavior	93

13.3 Source AXIS Master	94

13.3.1 Purpose	94

13.3.2 Configuration	94

13.3.3 Signal List	94

13.3.4 Data Availability	95

13.4 AXIS Protocol Notes	96

13.4.1 Handshake Rules	96

13.4.2 TLAST Semantics	96

13.4.3 TKEEP Usage	96

13.4.4 Channel Multiplexing via TID	96

14 APB Configuration Interface	98

14.1 Overview	98

14.2 APB Slave and Address Routing	99

14.2.1 APB Slave Signal List	99

14.2.2 Descriptor Kick-Off	100

14.3 Status Interface	101

14.3.1 Per-Channel Status Signals	101

14.3.2 Channel State Encoding	101

14.4 Configuration Registers	102

14.4.1 Address Range Validation	102

14.4.2 Configuration Registers	102

14.5 Programming Sequence	103

14.5.1 Initialization	103

14.5.2 Descriptor Kick-Off	103

15 Monitor Bus Interface	105

15.1 Overview	105

15.2 MonBus Packet Format	106

15.2.1 64-bit Packet Structure	106

15.2.2 Signal List	106

15.3 Packet Types	107

15.4 Agent IDs	108

15.5 Event Codes	110

15.5.1 Scheduler Events (Agent 0x30-0x37)	110

15.5.2 Descriptor Engine Events (Agent 0x10-0x17)	110

15.6 Event Data Encoding	111

15.6.1 Completion Event Data	111

15.6.2 Error Event Data	111

15.6.3 State Transition Event Data	111

15.7 MonBus Arbitration	112

15.7.1 Figure 3.5.1: MonBus Arbitration Tree	112

15.7.2 Arbitration Policy	112

15.8 Top-Level MonBus Delivery (monbus_axil4_axil4_group)	113

15.9 Integration Notes	114

15.9.1 Downstream Connection	114

15.9.2 Packet Rate	114

16 CXS Network Interface	115

16.1 Overview	115

16.1.1 Figure 3.6.1: Network Planes	115

16.2 Link Profile	116

16.3 Credits	118

16.4 Signal List	119

16.5 Flow Control and the Sink Contract	121

16.6 Monitoring	122

16.7 Routing and Payload Type (Proposed)	123

17 Use Case: Network to Memory (Sink Path)	124

17.1 Overview	124

17.2 Buffer Management	126

17.2.1 SRAM Buffering Strategy	126

17.2.2 Figure 4.1.1: Sink Channel Buffer	126

17.2.3 Flow Control	126

17.3 Performance Considerations	128

17.3.1 Throughput	128

17.3.2 Latency	128

17.4 Error Handling	129

17.4.1 Error Sources	129

18 Use Case: Memory to Network (Source Path)	132

18.1 Overview	132

18.2 Buffer Management	134

18.2.1 SRAM Buffering Strategy	134

18.2.2 Figure 4.2.1: Source Channel Buffer	134

18.2.3 Flow Control	134

18.3 Concurrent Operation	136

18.3.1 Read-Ahead Pipeline	136

18.4 Performance Considerations	137

18.4.1 Throughput	137

18.4.2 Latency	137

18.5 Error Handling	138

18.5.1 Error Sources	138

19 Use Case: Descriptor Chaining	139

19.1 Overview	139

19.2 Chain Structure	140

19.2.1 Figure 4.3.1: Descriptor Chain Structure	140

19.3 Descriptor Fields for Chaining	143

19.3.1 Relevant Fields	143

19.3.2 Chain Termination	143

19.4 Prefetch Optimization	145

19.4.1 Descriptor Prefetching	145

19.5 Multi-Channel Chaining	146

19.5.1 Figure 4.3.2: Independent Per-Channel Chains	146

19.6 Error During Chain	147

19.6.1 Error Handling	147

19.6.2 Recovery	147

20 Use Case: Multi-Channel Operation	148

20.1 Overview	148

20.2 Channel Independence	149

20.2.1 Figure 4.4.1: Independent Channels	149

20.3 Resource Arbitration	150

20.3.1 Descriptor Port Arbitration	150

20.3.2 Sink/Source Arbitration	150

20.3.3 Figure 4.4.2: Data-Path AXI Arbitration	151

20.4 Channel Isolation	152

20.4.1 Error Isolation	152

20.4.2 Per-Channel Soft Reset	152

20.5 Use Case Examples	153

20.5.1 Mixed Traffic Pattern	153

20.5.2 Bidirectional Transfer	153

20.5.3 Figure 4.4.3: Bidirectional Transfer	154

20.5.4 Load Balancing	154

20.5.5 Figure 4.4.4: Load Balancing by tid	155

20.6 Performance Considerations	156

20.6.1 Bandwidth Sharing	156

20.6.2 Latency Impact	156

20.7 MonBus Events	157

21 Descriptor Format	158

21.1 Overview	158

21.2 Descriptor Layout	159

21.2.1 256-Bit Structure	159

21.3 Field Descriptions	162

21.3.1 Control Bits [195:192]	162

21.3.2 Direction	162

21.3.3 Transfer Length [159:128]	162

21.3.4 Next Pointer [191:160]	162

21.3.5 Address Fields [127:0]	162

21.4 Descriptor Opcodes (DATA / CTRL_READ / CTRL_WRITE)	164

21.5 Extended Descriptors (row/col-major addressing)	166

21.6 Alignment Requirements	168

21.7 Descriptor Examples	169

21.7.1 Sink Descriptor (Network to Memory)	169

21.7.2 Source Descriptor (Memory to Network)	169

21.8 Software Construction	170

21.8.1 C Structure Example	170

21.8.2 Descriptor Ring Setup	170

22 Register Map	172

22.1 Overview	172

22.2 Address Space Layout	173

22.3 Base Registers (0x100-0x3FF)	174

22.4 Monitor Registers (+0x800 within each half)	176

22.5 Key Register Fields	177

22.5.1 GLOBAL_CTRL (0x100) - Read/Write	177

22.5.2 SCHED_TIMEOUT_LIMIT (0x208) - Read/Write	177

22.5.3 DESCENG_CONFIG (0x220) - Read/Write	177

22.5.4 Tile Header Status (0x2D0-0x2D8) - byte-granular RAPIDS with TILE_HDR = 1	178

22.6 Channel Kick Sequence	180

22.7 Address Decoding	181

23 Initialization Sequence	182

23.1 Overview	182

23.2 Boot Sequence	183

23.2.1 Figure 5.3.1: Bring-Up Sequence	183

23.3 What the Hardware Requires	184

23.4 Detailed Steps	185

23.4.1 Step 1: Verify Hardware Presence	185

23.4.2 Step 2: Optional Reset	185

23.4.3 Step 3: Configure the Engine	185

23.4.4 Step 4: Optional Monitor and Interrupt Egress	186

23.4.5 Step 5: Build Descriptors	186

23.4.6 Step 6: Enable	187

23.5 Starting Transfers	188

23.5.1 Kick a Channel	188

23.6 Completion and Interrupt Handling	189

23.7 Reset and Recovery	190

23.7.1 Per-Channel Reset	190

23.7.2 Global Reset	190

24 Error Handling	191

24.1 Overview	191

24.2 Error Categories	192

24.2.1 Figure 5.4.1: Error Sources	192

24.3 Error Detection	193

24.3.1 AXI Response Errors	193

24.3.2 Descriptor Errors	193

24.3.3 Timeout Detection	194

24.4 Error Reporting	197

24.4.1 Status Registers	197

24.4.2 MonBus Error Events	197

24.4.3 Interrupt Generation	198

24.4.4 Figure 5.4.2: Error Reporting	199

24.5 Error Recovery	200

24.5.1 Recovery Flow	200

24.5.2 Figure 5.4.3: Error Recovery	200

24.5.3 Finding the Faulted Channel	201

24.5.4 AXI Error Recovery	201

24.5.5 Descriptor Error Recovery	201

24.5.6 Timeout Recovery	202

24.6 Error Prevention	203

24.6.1 Best Practices	203

24.6.2 Descriptor Validation Example	203

25 Throughput Targets	204

25.1 Theoretical Maximum	205

25.1.1 Per-Interface Bandwidth	205

25.1.2 Aggregate Bandwidth	205

25.2 Practical Throughput	206

25.2.1 Single-Channel Performance	206

25.2.2 Multi-Channel Performance	206

25.3 Throughput Limiting Factors	207

25.3.1 Memory / Network System Factors	207

25.3.2 RAPIDS Internal Factors	207

25.4 Performance Targets	209

25.4.1 Design Targets	209

25.4.2 Verification Criteria	209

25.5 On-Silicon Validation	210

26 Latency Characteristics	212

26.1 Latency Components	213

26.1.1 Kick-to-First-Beat Breakdown	213

26.1.2 Total Latency Formula	213

26.2 First-Beat Latency	215

26.3 Control-Descriptor Latency	216

26.4 Latency Variability	217

26.4.1 Sources of Variability	217

26.4.2 Worst-Case Latency	217

26.5 Latency Optimization	218

26.5.1 Software Recommendations	218

26.5.2 Hardware Configuration	218

27 Resource Estimates	219

27.1 FPGA Resource Summary	220

27.2 Resource Breakdown by Block	221

27.2.1 APB Configuration Slave + Register Block	221

27.2.2 Scheduler Group (per direction)	221

27.2.3 Control Engines (per direction)	221

27.2.4 AXI Read / Write Engines	221

27.2.5 SRAM Buffers (sink + source)	221

27.2.6 MonBus / AXI Monitors (USE_AXI_MONITORS)	222

27.3 SRAM Sizing	223

27.4 Scaling Notes	224

28 RAPIDS Beats Architecture MAS	225

28.1 Document Organization	226

28.1.1 Front Matter	226

28.1.2 Chapter 1: Overview	226

28.1.3 Chapter 2: FUB (Functional Unit Blocks)	226

28.1.4 Chapter 3: Macro (Integration Blocks)	226

28.1.5 Chapter 4: Interfaces	226

28.2 Quick Reference	228

28.2.1 FUB Modules (fub_beats/)	228

28.2.2 Macro Modules (macro_beats/)	229

28.3 Architecture Comparison: RAPIDS vs STREAM	231

28.4 Clock and Reset Summary	232

28.4.1 Clock Domains	232

28.4.2 Reset Signals	232

28.5 Interface Summary	233

28.5.1 External Interfaces	233

28.5.2 Internal Buses	233

28.6 Related Documentation	234

28.7 Specification Conventions	235

28.7.1 Signal Naming	235

28.7.2 Parameter Naming	235

28.7.3 Figure and Table Numbering	235

29 Product Requirements Document (PRD)	236

29.1 RAPIDS - Rapid AXI Programmable In-band Descriptor System	236

29.2 1. Executive Summary	237

29.2.1 1.1 Quick Stats	237

29.2.2 1.2 Subsystem Goals	237

29.3 2. Documentation Structure	239

29.3.1 Complete RAPIDS Specification	239

29.3.2 Known Issues	239

29.3.3 Other Documentation	240

29.4 2.4 Organizational Standards - RAPIDS Code Location	241

29.4.1 Code Organization Principle	241

29.4.2 RAPIDS Directory Structure	241

29.4.3 What Goes Where?	241

29.4.4 Import Pattern for RAPIDS Tests	242

29.4.5 Benefits of This Organization	242

29.4.6 Compliance Status	243

29.5 3. Architecture Overview	244

29.5.1 3.1 Top-Level Block Diagram	244

29.5.2 3.2 Data Flow	244

29.6 4. Key Features	245

29.6.1 4.1 Descriptor-Based Operation	245

29.6.2 4.2 Data Path Features	245

29.6.3 4.3 Scheduler Features	245

29.6.4 4.4 Monitoring Integration	246

29.7 5. Interfaces	247

29.7.1 5.1 External Interfaces	247

29.7.2 5.2 Configuration Parameters	247

29.8 6. Use Cases	249

29.8.1 6.1 DMA-Style Transfers	249

29.8.2 6.2 Network Packet Processing	249

29.8.3 6.3 Custom Data Path Acceleration	249

29.9 7. Test Coverage	250

29.9.1 7.1 Current Status	250

29.9.2 7.2 Test Strategy	251

29.10 8. Known Issues Summary	252

29.10.1 8.1 Critical Issues	252

29.10.2 8.2 Medium Priority Issues	252

29.11 9. Integration Guidelines	253

29.11.1 9.1 Quick Start	253

29.11.2 9.2 Configuration Steps	253

29.12 10. Development Status	255

29.12.1 10.1 Current Phase	255

29.12.2 10.2 Roadmap	255

29.13 11. Performance Characteristics	256

29.13.1 11.1 Throughput	256

29.13.2 11.2 Latency	256

29.13.3 11.3 Resource Utilization	256

29.14 12. Verification Infrastructure	257

29.14.1 12.1 Test Organization	257

29.14.2 12.2 CocoTB Framework	257

29.14.3 12.2.1 MANDATORY: BFM Usage for FUB Tests	257

29.14.4 12.3 Test File Structure (Standard Pattern)	258

29.15 13. Quick Reference	264

29.15.1 13.1 Key Files	264

29.15.2 13.2 Commands	264

29.16 14. Success Criteria	266

29.16.1 14.1 Functional	266

29.16.2 14.2 Quality	266

29.16.3 14.3 Documentation	266

29.17 15. Educational Value	267

29.18 16. Documentation Generation	268

29.18.1 16.1 Generating PDF/DOCX from Specification	268

29.19 16.2 PDF Generation Location	270

29.20 17. References	271

29.20.1 16.1 Internal Documentation	271

29.20.2 16.2 Related Subsystems	271

29.20.3 16.3 External References	271

29.21 Navigation	272

30 Address Increment Patterns for DMA and Descriptor-Based Data Movement	273

30.1 Table of Contents	274

30.2 1. Executive Summary	275

30.2.1 Pattern Categories	275

30.3 2. RAPIDS Current Implementation	276

30.3.1 2.1 Descriptor Format (256-bit)	276

30.3.2 2.2 Current Address Increment Modes	276

30.3.3 2.3 Alignment Information Structure	277

30.3.4 2.4 AXI Translation	277

30.4 3. AXI Protocol Burst Modes	278

30.4.1 3.1 FIXED Burst (AxBURST = 2’b00)	278

30.4.2 3.2 INCREMENT Burst (AxBURST = 2’b01)	278

30.4.3 3.3 WRAP Burst (AxBURST = 2’b10)	278

30.5 4. Linear Address Increment Patterns	280

30.5.1 4.1 Simple Contiguous	280

30.5.2 4.2 Unaligned Start/End	280

30.5.3 4.3 Size-Constrained Bursts	280

30.6 5. 2D Transfer Patterns (Stride-Based)	282

30.6.1 5.1 Concept	282

30.6.2 5.2 Descriptor Fields (Extended)	282

30.6.3 5.3 Address Calculation	282

30.6.4 5.4 Use Cases	282

30.6.5 5.5 Analog Devices AXI DMAC Implementation	283

30.7 6. 3D Transfer Patterns	284

30.7.1 6.1 Concept	284

30.7.2 6.2 Descriptor Fields (Extended)	284

30.7.3 6.3 Address Calculation	284

30.7.4 6.4 Use Cases	284

30.8 7. Scatter-Gather Patterns	286

30.8.1 7.1 Concept	286

30.8.2 7.2 SG Descriptor Fields	286

30.8.3 7.3 Operation Modes	286

30.8.4 7.4 AMD/Xilinx AXI DMA SG Descriptor Format	286

30.8.5 7.5 Use Cases	287

30.9 8. Circular and Ring Buffer Patterns	288

30.9.1 8.1 Concept	288

30.9.2 8.2 Descriptor Fields (Extended)	288

30.9.3 8.3 Address Calculation	288

30.9.4 8.4 Ping-Pong Double Buffering	288

30.9.5 8.5 Use Cases	289

30.10 9. Tensor Accelerator Memory Access Patterns (PRIORITY 0)	290

30.10.1 9.1 Systolic Array Dataflow Fundamentals	290

30.10.2 9.2 Data Cube Model (NVDLA Reference)	290

30.10.3 9.3 Tensor Memory Accelerator (TMA) - NVIDIA Hopper	291

30.10.4 9.4 Convolution Address Patterns	292

30.10.5 9.5 Gemmini Accelerator Architecture (UC Berkeley Reference)	294

30.10.6 9.6 Tiling for Large Matrices	295

30.10.7 9.7 Sparse Tensor Patterns	296

30.10.8 9.8 Transformer/Attention Patterns	297

30.10.9 9.9 Programmable Address Generation Unit (PAGU)	297

30.11 10. Implementation Comparison Matrix	300

30.11.1 10.1 Pattern Complexity vs. Capability	300

30.11.2 10.2 Use Case Mapping	300

30.11.3 10.3 RAPIDS Current vs. Extended Capability	300

30.12 11. RAPIDS Extension Recommendations	302

30.12.1 11.0 PRIORITY 0: Tensor Accelerator Addressing (CRITICAL)	302

30.12.2 11.1 Priority 1: 2D Stride Support	307

30.12.3 11.2 Priority 2: Circular Buffer Mode	308

30.12.4 11.3 Priority 3: Enhanced Scatter-Gather	308

30.13 12. References	310

30.13.1 Industry Documentation	310

30.13.2 GPU and Accelerator Patterns	310

30.13.3 DMA Architecture	310

30.13.4 Neural Network Accelerators	310

30.13.5 Convolution and Tensor Operations	310

30.13.6 Sparse Tensor Formats	310

30.14 Document History	312

List of Figures

Figure 1.1.1: RAPIDS Beats in Its System	44

Figure 1.3.1: System Integration	52

Figure 2.1.1: RAPIDS Beats Whole Unit	57

Figure 2.1.2: rapids_beats_top / rapids_beats_cxs_top Integration	59

Figure 2.3.1: Per-Channel and Shared Resources	70

Figure 2.3.2: Channel Arbitration onto a Shared AXI Port	73

Figure 3.1.1: External Interfaces	77

Figure 3.5.1: MonBus Arbitration Tree	112

Figure 3.6.1: Network Planes	115

Figure 4.1.1: Sink Channel Buffer	126

Figure 4.2.1: Source Channel Buffer	134

Figure 4.3.1: Descriptor Chain Structure	140

Figure 4.3.2: Independent Per-Channel Chains	146

Figure 4.4.1: Independent Channels	149

Figure 4.4.2: Data-Path AXI Arbitration	151

Figure 4.4.3: Bidirectional Transfer	154

Figure 4.4.4: Load Balancing by tid	155

Figure 5.3.1: Bring-Up Sequence	183

Figure 5.4.1: Error Sources	192

Figure 5.4.2: Error Reporting	199

Figure 5.4.3: Error Recovery	200

List of Tables

Table 1: Target Audience	31

Table 2: Reference Documents	33

Table 3: Document Revision History	34

Table 4: Acronym Definitions	40

Table 5: Term Definitions	41

Table 6: Signal Naming Conventions	42

Table 7: Target Applications	45

Table 8: Phase 1 Simplifications	46

Table 9: Data Path Summary	47

Table 10: SRAM Buffer Parameters	49

Table 11: AXI Optimization Features	49

Table 12: Error Handling Summary	50

Table 13: AXI Memory Interface Requirements	53

Table 14: AXIS Network Interface	53

Table 15: Configuration Methods	54

Table 16: Clock Domains	55

Table 17: Reset Signals	55

Table 18: Power States	56

Table 19: Scheduler Array Components	60

Table 20: Sink Path Components	60

Table 21: Source Path Components	60

Table 22: Internal Bus Summary	63

Table 23: Resource Isolation	68

Table 24: Per-Channel Resources	71

Table 25: Shared Resources	72

Table 26: Channel Isolation Features	76

Table 27: External Interface Summary	78

Table 28: Clock and Reset Signals	79

Table 29: Interface Feature Matrix	80

Table 30: Signal Count Summary	81

Table 31: Descriptor AXI Configuration	83

Table 32: Descriptor AXI Signals	83

Table 33: Sink AXI Configuration	85

Table 34: Sink AXI Signals	85

Table 35: Source AXI Configuration	88

Table 36: Source AXI Signals	88

Table 37: Address Alignment Requirements	90

Table 38: AXI Response Handling	90

Table 39: Sink AXIS Configuration	92

Table 40: Sink AXIS Signals	92

Table 41: Source AXIS Configuration	94

Table 42: Source AXIS Signals	94

Table 43: TLAST Usage	96

Table 44: TKEEP Examples	96

Table 45: APB Address Routing	99

Table 46: APB Slave Signals	99

Table 47: Status Signals	101

Table 48: Scheduler State Encoding	101

Table 49: Key Configuration Registers	102

Table 50: MonBus Signals	106

Table 51: MonBus Packet Types	107

Table 52: RAPIDS Agent IDs	108

Table 53: Scheduler Event Codes	110

Table 54: Descriptor Engine Event Codes	110

Table 55: Top-Level MonBus AXI-Lite Group	113

Table 56: MonBus Packet Rates	114

Table 57: CXS Link Profile	116

Table 58: CXS Top Parameters	118

Table 59: CXS Link Signals	119

Table 60: CXS Status Signals	120

Table 61: Sink Path Flow Control	126

Table 62: Sink Path Throughput Factors	128

Table 63: Sink Path Latency Breakdown	128

Table 64: Sink Path Error Handling	129

Table 65: Source Path Flow Control	134

Table 66: Source Path Throughput Factors	137

Table 67: Source Path Latency Breakdown	137

Table 68: Source Path Error Handling	138

Table 69: Chaining-Related Descriptor Fields	143

Table 70: Mixed Traffic Channel Assignment	153

Table 71: Bandwidth Distribution	156

Table 72: Arbitration Latency	156

Table 73: Tile Header Descriptor Fields	159

Table 74: Control Bits	162

Table 75: Address Alignment Requirements	168

Table 76: Base Registers	174

Table 77: Monitor Registers (+0x800 within each half; SRC at 0x0800, SNK at 0x1800)	176

Table 78: GLOBAL_CTRL Register	177

Table 79: SCHED_TIMEOUT_LIMIT Register	177

Table 80: DESCENG_CONFIG Register	177

Table 81: Tile Header Status Registers	178

Table 82: Conditions for a Channel to Run	184

Table 83: AXI Response Codes	193

Table 84: Descriptor Checks	193

Table 85: Timeout Mechanisms	194

Table 86: Error-Related Status Registers	197

Table 87: MonBus Error Packets	197

Table 88: Error Prevention Best Practices	203

Table 89: FUB Module Summary	228

Table 90: Macro Module Summary	229

Table 91: RAPIDS Beats vs STREAM Comparison	231

Table 92: Clock Domains	232

Table 93: Reset Signals	232

Table 94: External Interfaces	233

Table 95: Internal Buses	233

# 1	Rapids Beats Has Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

Document Title: RAPIDS Beats Hardware Architecture Specification (HAS) Document Number: RAPIDS-HAS-001 Version: 0.15 Date: 2026-10-20 Status: Released

## 2.1	Purpose

This Hardware Architecture Specification (HAS) defines the external interfaces, system integration requirements, and high-level architecture of the RAPIDS Beats accelerator. It is intended for system architects and integration engineers who need to understand RAPIDS Beats from an external perspective without requiring knowledge of internal implementation details.

## 2.2	Scope

This document covers:

- External interface specifications (AXI4, AXIS, APB, MonBus)

- System-level block diagrams and data flows

- Programming model and descriptor format

- Use cases and operational sequences

- Performance characteristics and constraints

This document does NOT cover:

- Internal module architecture (see MAS)

- RTL implementation details (see MAS)

- Verification test plans

- Physical design constraints

## 2.3	Audience

Target Audience

| Role | Relevance |
| --- | --- |
| System Architect | Primary - system integration |
| Hardware Integration Engineer | Primary - interface connection |
| Software Engineer | Primary - driver development |
| Verification Engineer | Reference - interface protocols |
| RTL Designer | Reference - external constraints |

## 2.4	Document Conventions

### 2.4.1	Requirement Levels

| Term | Meaning |
| --- | --- |
| SHALL | Mandatory requirement |
| SHOULD | Recommended but not mandatory |
| MAY | Optional feature |

### 2.4.2	Signal Directions

- Input: Signal driven by external logic into RAPIDS

- Output: Signal driven by RAPIDS to external logic

- Bidirectional: Signal can be driven by either side (rare)

### 2.4.3	Timing Diagrams

All timing diagrams use WaveDrom format with the following conventions:

- p = Positive clock edge

- n = Negative clock edge

- 0/1 = Logic low/high

- x = Unknown/don’t care

- = = Data value (with label)

- . = Previous value continues

## 2.5	References

Reference Documents

| Document | Description |
| --- | --- |
| RAPIDS Beats MAS | Module Architecture Specification |
| ARM AMBA AXI4 Specification | AXI4 protocol reference |
| ARM AMBA AXI-Stream Specification | AXIS protocol reference |
| Arm AMBA CXS Protocol Specification (IHI 0079, Issue D) | CXS protocol reference (CXS tops) |
| RAPIDS PRD | Product Requirements Document |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Revision History

Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-17 | RTL Design Sherpa | Initial release |
| 0.6 | 2026-07-02 | RTL Design Sherpa | Resynced to RTL top-level integration |
| 0.7 | 2026-07-13 | RTL Design Sherpa | Added the control-descriptor feature (CTRL_READ consumer gate / CTRL_WRITE producer doorbell) |
| 0.8 | 2026-07-13 | RTL Design Sherpa | Added Chapter 6: Performance (throughput targets, latency characteristics, measured resource estimates) |
| 0.9 | 2026-09-21 | RTL Design Sherpa | Removed a vestigial diagram-asset layer: 19 image references and 35 **Source:** links pointed at assets/mermaid/ and assets/wavedrom/ files that were never created, while a live fenced mermaid or wavedrom block sat immediately below each one carrying the same diagram. No diagram content changed – all 31 mermaid and 39 wavedrom fences render as before. |
| 0.10 | 2026-10-16 | RTL Design Sherpa | SRAM_DEPTH stated as beats per channel, per direction |
| 0.11 | 2026-10-16 | RTL Design Sherpa | Network planes and CXS: new CXS Network Interface chapter (planes, link profile, credits, signals, monitoring, planned routing / payload-type fields); interface summary, block diagram, hierarchy, system context, key features and MonBus agent table updated for the AXIS / CXS planes; the Top-Level MonBus Delivery section corrected (the data masters carry no monitors); all block diagrams updated and a network-planes figure added |
| 0.12 | 2026-10-16 | RTL Design Sherpa | Routing and payload-type PROPOSAL (not implemented): the tile packet header is defined in the compute-engine tile’s TILE_SPEC Section 11; CXS only transports it. Descriptor Format adds the proposed ptype, destination-coordinate and target-address fields; the CXS Network Interface chapter points at TILE_SPEC |
| 0.13 | 2026-10-17 | RTL Design Sherpa | Descriptor Format: the tile header fields are implemented in the byte-granular RAPIDS (TILE_HDR = 1), not in RAPIDS Beats; Register Map: tile status registers (0x2D0-0x2D8); CXS chapter names the sink and source ports |
| 0.14 | 2026-10-20 | RTL Design Sherpa | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |
| 0.15 | 2026-10-20 | RTL Design Sherpa | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |

## 3.1	Change Summary

### 3.1.1	Version 0.8 (2026-07-13)

Performance Chapter

- Added Chapter 6: Performance, mirroring the STREAM HAS performance chapter:

  - Section 6.1 Throughput Targets – per-interface and aggregate bandwidth (512-bit @ 100 MHz = 6.4 GB/s per direction, 12.8 GB/s full-duplex with concurrent sink + source), practical single/multi-channel efficiency, limiting factors, design targets, and on-silicon golden-CRC validation (48/48 configs).

  - Section 6.2 Latency Characteristics – kick-to-first-beat breakdown, control-descriptor latency (CTRL_READ retry budget, CTRL_WRITE doorbell), variability, and worst-case bounds.

  - Section 6.3 Resource Estimates – measured post-implementation utilization from the Nexys A7-100T characterization build (37,555 LUT / 28,683 FF / 22 BRAM / 0 DSP at NUM_CHANNELS=4, timing met at 100 MHz), per-block breakdown, and scaling notes.

### 3.1.2	Version 0.7 (2026-07-13)

Control-Descriptor Feature

- Documented the control-descriptor opcodes in Descriptor Format (Section 5.1): CTRL_READ (consumer gate – poll a memory location until (read & mask) == expected) and CTRL_WRITE (producer doorbell – single 32-bit write), for in-memory producer/consumer synchronization without moving payload. Added the per-opcode field layouts.

- Added CTRL_CONFIG @ 0x240 (CTRLRD_MAX_TRY[8:0], the control-read poll retry budget) to the register map (Section 5.2) and a Key Features entry.

### 3.1.3	Version 0.6 (2026-07-02)

RTL Resync

- Documented the rapids_beats_top integration: single APB slave routed to descriptor kick-off (0x000-0x03F) and the rapids_regs register block (base 0x100-0x3FF plus monitor regfile @ 0x1000).

- Refreshed the register map with the actual PeakRDL-generated addresses, including the monitor regfile at 0x1000 and SCHED_TIMEOUT_LIMIT at 0x208.

- Added the MonBus AXI-Lite group (error-drain slave s_axil_err_*, capture master m_axil_mon_*, mon_irq) and the USE_AXI_MONITORS-gated rd/wr AXI monitors to the block diagram, APB, and MonBus interface chapters.

- Reflected scheduler recoverable write-progress timeout + B-response commit gating and functional descriptor prefetch.

### 3.1.4	Version 1.0 (2026-01-17)

Initial Release

- Complete HAS for RAPIDS Beats Phase 1 architecture

- External interface specifications (AXI4, AXIS, APB, MonBus)

- System block diagrams with Mermaid

- Timing diagrams with WaveDrom

- Programming model and descriptor format

- Use case documentation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Acronyms and Definitions

## 4.1	Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface |
| AXIS | AXI-Stream |
| APB | Advanced Peripheral Bus |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| MAS | Module Architecture Specification |
| MonBus | Monitor Bus |
| RAPIDS | Rapid AXI Programmable In-band Descriptor System |
| SRAM | Static Random Access Memory |

## 4.2	Definitions

Term Definitions

| Term | Definition |
| --- | --- |
| Beat | Single data transfer unit (512 bits = 64 bytes in default configuration) |
| Channel | Independent DMA context with dedicated descriptor processing |
| Descriptor | 256-bit control structure defining a single DMA transfer |
| Descriptor Chain | Linked list of descriptors for multi-transfer operations |
| Sink Path | Data flow from network (AXIS slave) to memory (AXI write) |
| Source Path | Data flow from memory (AXI read) to network (AXIS master) |
| Fill | Operation writing data into SRAM buffer (sink path input) |
| Drain | Operation reading data from SRAM buffer (source path output) |

## 4.3	Signal Naming Conventions

Signal Naming Conventions

| Prefix/Suffix | Meaning |
| --- | --- |
| m_axi_* | AXI Master signals |
| s_axis_* | AXIS Slave signals |
| m_axis_* | AXIS Master signals |
| cfg_* | Configuration signals |
| *_valid | Handshake valid signal |
| *_ready | Handshake ready signal |
| *_n | Active-low signal |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Product Overview

## 5.1	Introduction

RAPIDS Beats is a high-performance network-to-memory accelerator designed for descriptor-based data movement operations. It bridges network interfaces (AXI-Stream) with system memory (AXI4), enabling efficient DMA transfers without CPU intervention.

The “Beats” architecture represents Phase 1 of the RAPIDS family, optimized for simplicity and throughput in streaming applications.

## 5.2	System Context

### 5.2.1	Figure 1.1.1: RAPIDS Beats in Its System

RAPIDS Beats in Its System

Source: ch01_product_context.dot

## 5.3	Target Applications

Target Applications

| Application | Description | Key Requirements |
| --- | --- | --- |
| Network Offload | Receive packets from network, store to memory | Low latency, high throughput |
| Scatter-Gather DMA | Multi-descriptor data movement | Descriptor chaining |
| Streaming Accelerator | Front-end for compute engines | Continuous data flow |
| Protocol Processing | Header/payload separation | Multi-channel isolation |

## 5.4	Architecture Highlights

### 5.4.1	Phase 1 “Beats” Simplifications

The Beats architecture prioritizes simplicity over feature completeness:

Phase 1 Simplifications

| Feature | Beats (Phase 1) | Future Phases |
| --- | --- | --- |
| Transfer Unit | Full beats only | Chunk-level alignment |
| Flow Control | Backpressure-based | Credit management |
| Address Alignment | Pre-aligned required | Hardware alignment fixup |
| Network Interface | Direct AXIS, or CXS through a swappable network plane | Protocol conversion inside the core |

### 5.4.2	Key Capabilities

- 8 Independent Channels - Concurrent transfers with dedicated resources

- Descriptor Chaining - Automatic next-descriptor fetch

- Dual Data Paths - Simultaneous sink (write) and source (read)

- Monitor Integration - Real-time event reporting via MonBus

- Swappable Network Plane - AXI-Stream or CXS (Arm IHI 0079 Issue D) at the top, same core

- Error Detection - Address range checking, AXI error propagation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Key Features

## 6.1	Feature Summary

### 6.1.1	Multi-Channel Architecture

- 8 Independent Channels with dedicated schedulers

- Per-Channel State Machines for concurrent operation

- Shared AXI Infrastructure with arbitrated access

- Channel Isolation prevents cross-channel interference

### 6.1.2	Descriptor-Based Control

- 256-bit Descriptor Format with all transfer parameters

- Automatic Chaining follows next_descriptor_ptr links

- Last Descriptor Flag terminates chain processing

- IRQ Generation via gen_irq descriptor flag

- Control Descriptors (CTRL_READ consumer gate / CTRL_WRITE producer doorbell) for in-memory producer/consumer synchronization without moving payload – see Descriptor Format (Section 5.1)

### 6.1.3	High-Performance Data Paths

Data Path Summary

| Path | Direction | Bandwidth | Features |
| --- | --- | --- | --- |
| Sink | Network to Memory | 512-bit @ clock | SRAM buffering, AXI burst writes |
| Source | Memory to Network | 512-bit @ clock | AXI burst reads, SRAM buffering |

### 6.1.4	Monitoring and Debug

- 64-bit MonBus Packets for all significant events

- State Transition Reports track FSM activity

- Error Event Codes for fault diagnosis

- Performance Metrics for throughput analysis

- Network Monitor-Lite in each network plane (AXIS or CXS), agents 0x09 / 0x0A

## 6.2	Feature Details

Descriptor Processing

### 6.2.1	SRAM Buffering

Each channel has its own buffer in each data path (sink and source), SRAM_DEPTH beats deep:

SRAM Buffer Parameters

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| SRAM_DEPTH | 4096 | 64-4096 | Beats per channel, per direction (parameter of rapids_beats_top) |
| Data Width | 512 | 256 / 512 | Bits per beat (DATA_WIDTH) |
| Buffer size | 256 KB | 2 KB-256 KB | Per channel, per direction: SRAM_DEPTH x DATA_WIDTH/8 |

The defaults are a simulation size. The Genesys 2 builds use 128 beats of 32 bytes, 4 KB per channel.

Buffer Operation:

- Sink Path: Fill from AXIS (from the RX network plane, AXIS or CXS), drain to AXI write engine

- Source Path: Fill from AXI read engine, drain to AXIS (to the TX network plane, AXIS or CXS)

- Flow Control: Backpressure when buffer full/empty

### 6.2.2	AXI Burst Optimization

RAPIDS optimizes AXI transactions for maximum bandwidth:

AXI Optimization Features

| Feature | Implementation |
| --- | --- |
| Burst Length | Up to 256 beats (AXI4 max) |
| Outstanding Transactions | Configurable (default 8) |
| Address Alignment | 64-byte aligned (beat size) |
| 4KB Boundary | Automatic burst splitting |

### 6.2.3	Error Handling

Error Handling Summary

| Error Type | Detection | Response |
| --- | --- | --- |
| Address Range | Pre-fetch validation | Descriptor rejected |
| AXI SLVERR | Response check | Transfer aborted |
| AXI DECERR | Response check | Transfer aborted |
| Timeout | Watchdog timer | Channel reset |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	System Context

## 7.1	Integration Overview

RAPIDS Beats integrates into a system as a DMA engine bridging network interfaces and system memory. This section describes the external connections and system-level requirements.

## 7.2	System Block Diagram

### 7.2.1	Figure 1.3.1: System Integration

System Integration

Source: ch01_system_integration.dot

## 7.3	Interface Connections

### 7.3.1	Memory Interface

RAPIDS Beats reaches system memory through the AXI4 masters below. Each half (src_, snk_) has its own descriptor master and its own two control masters, so the descriptor and control rows are two ports each. With monitors built, an AXI-Lite master (m_axil_mon) also writes the monitor log:

AXI Memory Interface Requirements

| Port | Access Pattern | Typical Bandwidth |
| --- | --- | --- |
| Descriptor AXI (m_axi_desc) | Random read (256-bit) | Low (<100 MB/s) |
| Control AXI (m_axi_ctrlrd, m_axi_ctrlwr) | Single 32-bit reads and writes | Very low |
| Sink AXI | Sequential write (512-bit) | High (up to interface max) |
| Source AXI | Sequential read (512-bit) | High (up to interface max) |

Interconnect Requirements:

- All of these ports MAY share a single physical AXI port with arbitration

- Descriptor port has lowest bandwidth requirement

- Sink and Source ports should have balanced bandwidth allocation

### 7.3.2	Network Interface

The network side is a swappable plane. The AXIS tops use standard AXI-Stream, below; the CXS tops (*_cxs_top) replace the two streams with one CXS link per direction, with CXSSRCID carrying the channel (see the CXS Network Interface chapter):

AXIS Network Interface

| Interface | Role | Signals |
| --- | --- | --- |
| Sink AXIS | Receives data from network | TDATA, TVALID, TREADY, TLAST |
| Source AXIS | Sends data to network | TDATA, TVALID, TREADY, TLAST |

Protocol Notes:

- Beat-granular: the beats design drives every byte valid (m_axis_tstrb all ones) and does not use the ingress s_axis_tstrb; a transfer is a whole number of beats. Partial beats (packed bytes, a contiguous tstrb on the last beat) are the byte-granular RAPIDS in rtl/top/rapids_top.sv (rapids TASK-019), which this document does not describe.

- TID/TDEST may be used for channel routing (optional)

- TUSER available for sideband information

### 7.3.3	Configuration Interface

Software configures RAPIDS through register writes:

Configuration Methods

| Method | Interface | Usage |
| --- | --- | --- |
| Direct Write | Memory-mapped | Register configuration |
| Descriptor Kick | APB-style | Start descriptor processing |

## 7.4	Clock and Reset

### 7.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | All RAPIDS logic |

Note: RAPIDS Beats uses a single clock domain. CDC (clock domain crossing) to different frequency domains must be handled externally.

### 7.4.2	Reset Requirements

Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert |

Reset Sequence:

- Assert aresetn low for minimum 4 clock cycles

- Ensure all AXI interfaces are idle before reset

- Release reset synchronously to aclk rising edge

- Wait 16 clock cycles before initiating transfers

## 7.5	Power Considerations

### 7.5.1	Clock Gating

RAPIDS supports optional clock gating for power reduction:

- Idle Detection: Automatic when no transfers pending

- Per-Channel Gating: Individual channels can be clock-gated

- External Control: cfg_clock_enable override available

### 7.5.2	Power States

Power States

| State | Description | Exit Latency |
| --- | --- | --- |
| Active | Normal operation | 0 cycles |
| Idle | No pending transfers | 1 cycle |
| Gated | Clock disabled | 4 cycles |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Block Diagram

## 8.1	Top-Level Architecture

### 8.1.1	Figure 2.1.1: RAPIDS Beats Whole Unit

RAPIDS Beats Whole Unit

Source: ch02_whole_unit.dot

## 8.2	Top-Level Integration (rapids_beats_top, rapids_beats_cxs_top)

rapids_core_beats is wrapped by rapids_beats_top, which adds the software register interface, configuration mapping, and monitoring infrastructure. A single APB slave feeds a command/response router into the rapids_regs register block (base config at 0x100-0x3FF, monitor regfile at 0x1000); the per-channel kick registers live at 0x000-0x03F inside that same block. rapids_config_block translates the register hwif_out into the core/monitor cfg_* signals.

The network endpoints are not in the core. They sit in two network planes between the core and the pins: an RX plane in front of the sink half and a TX plane behind the source half (rtl/plane/, rapids TASK-024). The AXIS top (rapids_beats_top) uses rapids_plane_axis_rx / rapids_plane_axis_tx (axis4_slave_monlite, axis4_master_monlite); the CXS top (rapids_beats_cxs_top) uses rapids_plane_cxs_rx / rapids_plane_cxs_tx (cxs_slave_monlite + cxs_to_axis, axis_to_cxs + cxs_master_monlite). Both tops are generated from rapids_beats_top_base by bin/gen_plane_tops.py; the CXS link is described in the CXS Network Interface chapter.

When USE_AXI_MONITORS = 1, each half’s descriptor read master carries an axi4_master_rd_monlite, and each plane carries a monitor-lite on its network port (AXIS or CXS; rapids TASK-015). The descriptor-monitor packets join the eight scheduler groups in the array’s 9:1 monbus_arbiter; the plane’s packets enter the core on snk_plane_mon_* / src_plane_mon_* and join the array output in the half’s 3:1 monbus_arbiter (client 1; third input reserved), the core merges the two halves, and the top delivers the stream through a monbus_axil4_axil4_group to an AXI-Lite error-drain slave, an AXI-Lite capture master, and a mon_irq interrupt. The data masters m_axi_rd and m_axi_wr carry no monitors.

### 8.2.1	Figure 2.1.2: rapids_beats_top / rapids_beats_cxs_top Integration

rapids_beats_top Integration

Source: ch02_top_integration.dot

## 8.3	Component Summary

### 8.3.1	Scheduler Group Array

The scheduler group array manages 8 independent channels:

Scheduler Array Components

| Component | Instance Count | Purpose |
| --- | --- | --- |
| scheduler_beats | 8 | Transfer coordination per channel |
| descriptor_engine_beats | 8 | Descriptor fetch and parse |
| Descriptor AXI Arbiter | 1 | Shared AXI4 for descriptor fetch |

### 8.3.2	Sink Data Path

Network-to-memory data flow:

Sink Path Components

| Component | Purpose |
| --- | --- |
| snk_sram_controller_beats | Multi-channel SRAM management |
| axi_write_engine_beats | AXI4 burst write generation |
| alloc_ctrl_beats | Space allocation tracking |
| drain_ctrl_beats | Data availability tracking |

### 8.3.3	Source Data Path

Memory-to-network data flow:

Source Path Components

| Component | Purpose |
| --- | --- |
| axi_read_engine_beats | AXI4 burst read generation |
| src_sram_controller_beats | Multi-channel SRAM management |
| alloc_ctrl_beats | Space allocation tracking |
| drain_ctrl_beats | Data availability tracking |

## 8.4	Hierarchy

rapids_beats_top | rapids_beats_cxs_top   (generated by bin/gen_plane_tops.py)
├── u_base: rapids_beats_top_base   (regs, rapids_config_block x2, monbus group)
│   └── u_core: rapids_core_beats
├── u_snk_plane: rapids_plane_axis_rx | rapids_plane_cxs_rx
│   └── axis4_slave_monlite | cxs_slave_monlite + cxs_to_axis
└── u_src_plane: rapids_plane_axis_tx | rapids_plane_cxs_tx
    └── axis4_master_monlite | axis_to_cxs + cxs_master_monlite

rapids_core_beats
├── monbus_arbiter
├── rapids_src_beats
│   ├── scheduler_group_array_beats
│   │   ├── scheduler_group_beats [0..7]
│   │   │   ├── scheduler_beats
│   │   │   ├── descriptor_engine_beats
│   │   │   ├── ctrlrd_engine
│   │   │   └── ctrlwr_engine
│   │   ├── arbiter_round_robin [x3]
│   │   ├── axi4_master_rd_monlite
│   │   └── axi_bus_meter
│   ├── monbus_arbiter (3:1; client 1 = src_plane_mon_*)
│   └── src_data_path_axis_beats
│       └── src_data_path_beats
│           ├── axi_read_engine_beats
│           └── src_sram_controller_beats
│               └── sram_controller (STREAM)
│                   └── sram_controller_unit [0..7]
│                       ├── stream_alloc_ctrl
│                       ├── gaxi_fifo_sync
│                       ├── stream_drain_ctrl
│                       └── stream_latency_bridge
└── rapids_snk_beats
    ├── scheduler_group_array_beats (same shape)
    ├── monbus_arbiter (3:1; client 1 = snk_plane_mon_*)
    └── snk_data_path_axis_beats
        └── snk_data_path_beats
            ├── axi_write_engine_beats
            └── snk_sram_controller_beats
                └── sram_controller (STREAM)
                    └── sram_controller_unit [0..7]

## 8.5	Internal Buses

Internal Bus Summary

| Bus | Width | Description |
| --- | --- | --- |
| Descriptor Bus | 256-bit | Parsed descriptor to scheduler |
| Scheduler Command | Variable | Transfer parameters to data paths |
| SRAM Data | 512-bit | Data to/from SRAM buffers |
| MonBus | 64-bit | Monitor packets (aggregated) |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Data Flow

## 9.1	Overview

RAPIDS Beats implements two independent data paths that can operate concurrently:

- Sink Path: Network to Memory (AXIS Slave -> AXI Write)

- Source Path: Memory to Network (AXI Read -> AXIS Master)

## 9.2	Sink Path Data Flow

The sink accepts a beat for channel tid only while that channel holds an SRAM reservation. snk_data_path_axis_beats drives

s_axis_tready = fill_ready && (r_pending_alloc[tid] > 0 || fill_alloc_req)

where r_pending_alloc[tid] is the number of reserved beats the channel has not yet filled. When the head beat arrives for a channel that holds no reservation, the wrapper raises fill_alloc_req in the same cycle. The request is for AXI_XFER_CONFIG.ALLOC_SIZE beats, or for whatever remains when less than that is free, and the beat is accepted on that cycle. A channel whose partition is full, or whose last allocation is still settling into fill_space_free (three cycles), gets no reservation, and tready stays low.

The reservation does not depend on the descriptor. Data can therefore reach a channel’s SRAM partition before its descriptor has been fetched, but only up to the space the partition has free. After that the channel stops accepting until the write engine drains. The write engine drains only after the scheduler has a descriptor in CH_XFER_DATA and presents sched_wr_valid, so a channel that is never kicked fills its partition once and then stalls. tready is evaluated for the tid of the beat at the head of the single s_axis port, so a stalled channel also blocks beats for every other channel behind it (head-of-line blocking). The AXIS monitor-lite’s skid stage in front of the wrapper absorbs a few beats before the stall reaches the network.

Diagram 2

Sink Path Timing

Source Path Data Flow

Source Path Timing

## 9.3	Concurrent Operation

Both data paths operate independently and can run simultaneously:

Diagram 4

### 9.3.1	Deadlock Prevention

RAPIDS prevents deadlock through independent resource allocation:

Resource Isolation

| Resource | Sink Path | Source Path | Shared |
| --- | --- | --- | --- |
| SRAM Buffer | Dedicated | Dedicated | No |
| AXI Port | Write only | Read only | No |
| Scheduler | Per-channel | Per-channel | No |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Channel Architecture

## 10.1	Multi-Channel Design

RAPIDS Beats supports 8 independent DMA channels, each with dedicated resources for concurrent operation.

## 10.2	Channel Resource Allocation

### 10.2.1	Figure 2.3.1: Per-Channel and Shared Resources

Per-Channel and Shared Resources

Source: ch02_channel_resources.dot

## 10.3	Per-Channel Resources

Each channel has dedicated:

Per-Channel Resources

| Resource | Description | Size/Capacity |
| --- | --- | --- |
| Scheduler | Transfer coordination FSM | 1 instance |
| Descriptor Engine | Descriptor fetch/parse | 8-deep FIFO |
| Sink SRAM Unit | Sink buffer, its own sram_controller | SRAM_DEPTH beats |
| Source SRAM Unit | Source buffer, its own sram_controller | SRAM_DEPTH beats |
| Configuration Registers | Per-channel config | ~16 registers |

## 10.4	Shared Resources

Resources shared across all channels:

Shared Resources

| Resource | Arbitration | Max Outstanding |
| --- | --- | --- |
| Descriptor AXI | Round-robin | 8 (1 per channel) |
| Sink AXI Write | Round-robin | 8 configurable |
| Source AXI Read | Round-robin | 8 configurable |
| MonBus Output | Round-robin (monbus_arbiter tree) | 1 (serialized) |

## 10.5	Channel Scheduling

### 10.5.1	Arbitration Policy

### 10.5.2	Figure 2.3.2: Channel Arbitration onto a Shared AXI Port

Channel Arbitration onto a Shared AXI Port

Source: ch02_channel_arbitration.dot

Arbitration Timing:

Waveform 3

## 10.6	Channel State Machine

Each channel scheduler follows this state machine:

Diagram 5

Channel State Timing

## 10.7	Channel Isolation

Channels are isolated to prevent interference:

Channel Isolation Features

| Isolation Type | Implementation |
| --- | --- |
| State Isolation | Separate FSM per channel |
| Buffer Isolation | Partitioned SRAM per channel |
| Error Isolation | Per-channel error flags |
| Reset Isolation | Per-channel soft reset |

### 10.7.1	Per-Channel Error Handling

Each channel maintains independent error state:

Waveform 5

Channel 1 error does not affect Channel 0 or Channel 2 operation.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Interface Summary

## 11.1	External Interface Overview

RAPIDS Beats exposes the following external interfaces. The network side is a swappable plane: the AXIS tops (rapids_beats_top, rapids_top) carry the s_axis_* / m_axis_* streams, and the CXS tops (rapids_beats_cxs_top, rapids_cxs_top) carry one CXS link per direction in their place (see the CXS Network Interface chapter). Everything else is identical.

### 11.1.1	Figure 3.1.1: External Interfaces

External Interfaces

Source: ch01_system_integration.dot

## 11.2	Interface Port Summary

External Interface Summary

| Interface | Type | Direction | Data Width | Address Width |
| --- | --- | --- | --- | --- |
| Descriptor AXI | AXI4 Master | Read | 256-bit | 64-bit |
| Sink AXI | AXI4 Master | Write | 512-bit | 64-bit |
| Source AXI | AXI4 Master | Read | 512-bit | 64-bit |
| Sink AXIS | AXIS Slave | Input | 512-bit | - |
| Source AXIS | AXIS Master | Output | 512-bit | - |
| Sink CXS (CXS tops only, replaces Sink AXIS) | CXS Receiver | Input | DATA_WIDTH (up to 1024-bit) | - |
| Source CXS (CXS tops only, replaces Source AXIS) | CXS Transmitter | Output | DATA_WIDTH (up to 1024-bit) | - |
| APB Config | APB-like | Input | 32-bit | 12-bit |
| MonBus | Custom | Output | 64-bit | - |

## 11.3	Clock and Reset

All interfaces operate in the same clock domain:

Clock and Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aclk | input | System clock (100-500 MHz typical) |
| aresetn | input | Active-low reset (async assert, sync deassert) |

## 11.4	Interface Feature Matrix

Interface Feature Matrix

| Feature | Desc AXI | Sink AXI | Src AXI | Sink AXIS | Src AXIS |
| --- | --- | --- | --- | --- | --- |
| Protocol | AXI4 | AXI4 | AXI4 | AXIS | AXIS |
| Direction | Read | Write | Read | Slave | Master |
| Burst Support | INCR | INCR | INCR | N/A | N/A |
| Max Burst | 1 | 256 | 256 | N/A | N/A |
| Outstanding | 8 | 8 | 8 | N/A | N/A |
| ID Width | 8-bit | 8-bit | 8-bit | Optional | Optional |

## 11.5	Signal Count Summary

Signal Count Summary

| Interface | Input Signals | Output Signals | Total |
| --- | --- | --- | --- |
| Descriptor AXI | ~20 | ~30 | ~50 |
| Sink AXI | ~15 | ~40 | ~55 |
| Source AXI | ~25 | ~25 | ~50 |
| Sink AXIS | ~20 | ~5 | ~25 |
| Source AXIS | ~5 | ~20 | ~25 |
| Sink CXS (CXS tops) | ~8 | ~7 | ~15 |
| Source CXS (CXS tops) | ~3 | ~10 | ~13 |
| APB Config | ~15 | ~5 | ~20 |
| MonBus | ~2 | ~5 | ~7 |
| Total | ~102 | ~130 | ~232 |

Note: Signal counts are approximate and vary with configuration parameters. The total counts an AXIS top; a CXS top replaces the two AXIS rows with the two CXS rows (including the link-status pins).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI4 Interface Specification

## 12.1	Overview

RAPIDS Beats uses three AXI4 master interfaces for memory access:

- Descriptor AXI - Fetches 256-bit descriptors (read-only)

- Sink AXI - Writes data to memory (write-only)

- Source AXI - Reads data from memory (read-only)

## 12.2	Descriptor AXI Master

### 12.2.1	Purpose

Fetches descriptor structures from system memory for all 8 channels.

### 12.2.2	Configuration

Descriptor AXI Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 256-bit | Fixed descriptor size |
| Address Width | 64-bit | Configurable |
| ID Width | 8-bit | Configurable |
| Burst Length | 1 | Single-beat only |
| Burst Type | INCR | Fixed |

### 12.2.3	Signal List

Descriptor AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_desc_aclk | 1 | input | Clock |
| m_axi_desc_aresetn | 1 | input | Reset (active-low) |
| m_axi_desc_arid | 8 | output | Read ID (channel ID) |
| m_axi_desc_araddr | 64 | output | Read address |
| m_axi_desc_arlen | 8 | output | Burst length (always 0) |
| m_axi_desc_arsize | 3 | output | Beat size (5 = 32 bytes) |
| m_axi_desc_arburst | 2 | output | Burst type (INCR) |
| m_axi_desc_arvalid | 1 | output | Address valid |
| m_axi_desc_arready | 1 | input | Address ready |
| m_axi_desc_rid | 8 | input | Response ID |
| m_axi_desc_rdata | 256 | input | Read data |
| m_axi_desc_rresp | 2 | input | Read response |
| m_axi_desc_rlast | 1 | input | Last beat |
| m_axi_desc_rvalid | 1 | input | Data valid |
| m_axi_desc_rready | 1 | output | Data ready |

Timing Diagram

## 12.3	Sink AXI Master (Write)

### 12.3.1	Purpose

Writes data from SRAM buffer to system memory.

### 12.3.2	Configuration

Sink AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | Match SRAM width |
| Address Width | 64-bit | Configurable | System address space |
| ID Width | 8-bit | Configurable | Transaction tracking |
| Max Burst | 256 | 1-256 | AXI4 maximum |
| Outstanding | 8 | 1-16 | Concurrent AW requests |

### 12.3.3	Signal List

Sink AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_sink_awid | 8 | output | Write ID |
| m_axi_sink_awaddr | 64 | output | Write address |
| m_axi_sink_awlen | 8 | output | Burst length - 1 |
| m_axi_sink_awsize | 3 | output | Beat size (6 = 64 bytes) |
| m_axi_sink_awburst | 2 | output | Burst type (INCR) |
| m_axi_sink_awvalid | 1 | output | Address valid |
| m_axi_sink_awready | 1 | input | Address ready |
| m_axi_sink_wdata | 512 | output | Write data |
| m_axi_sink_wstrb | 64 | output | Write strobes |
| m_axi_sink_wlast | 1 | output | Last beat |
| m_axi_sink_wvalid | 1 | output | Data valid |
| m_axi_sink_wready | 1 | input | Data ready |
| m_axi_sink_bid | 8 | input | Response ID |
| m_axi_sink_bresp | 2 | input | Write response |
| m_axi_sink_bvalid | 1 | input | Response valid |
| m_axi_sink_bready | 1 | output | Response ready |

Timing Diagram

## 12.4	Source AXI Master (Read)

### 12.4.1	Purpose

Reads data from system memory to SRAM buffer.

### 12.4.2	Configuration

Source AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | Match SRAM width |
| Address Width | 64-bit | Configurable | System address space |
| ID Width | 8-bit | Configurable | Transaction tracking |
| Max Burst | 256 | 1-256 | AXI4 maximum |
| Outstanding | 8 | 1-16 | Concurrent AR requests |

### 12.4.3	Signal List

Source AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_src_arid | 8 | output | Read ID |
| m_axi_src_araddr | 64 | output | Read address |
| m_axi_src_arlen | 8 | output | Burst length - 1 |
| m_axi_src_arsize | 3 | output | Beat size (6 = 64 bytes) |
| m_axi_src_arburst | 2 | output | Burst type (INCR) |
| m_axi_src_arvalid | 1 | output | Address valid |
| m_axi_src_arready | 1 | input | Address ready |
| m_axi_src_rid | 8 | input | Response ID |
| m_axi_src_rdata | 512 | input | Read data |
| m_axi_src_rresp | 2 | input | Read response |
| m_axi_src_rlast | 1 | input | Last beat |
| m_axi_src_rvalid | 1 | input | Data valid |
| m_axi_src_rready | 1 | output | Data ready |

Timing Diagram

## 12.5	AXI Protocol Constraints

### 12.5.1	Address Alignment

Address Alignment Requirements

| Transfer Type | Alignment Requirement |
| --- | --- |
| Descriptor | 32-byte aligned |
| Sink Data | 64-byte aligned |
| Source Data | 64-byte aligned |

### 12.5.2	4KB Boundary Handling

AXI4 requires burst transactions not to cross 4KB boundaries. RAPIDS automatically splits bursts at 4KB boundaries:

Example: 8-beat burst starting at 0x0FE0 (64 bytes/beat)
  Beat 0-1: 0x0FE0 - 0x0FFF (to 4KB boundary)
  Beat 2-7: 0x1000 - 0x117F (after boundary)

Split into two bursts:
  Burst 1: ARLEN=1, ARADDR=0x0FE0
  Burst 2: ARLEN=5, ARADDR=0x1000

### 12.5.3	Response Handling

AXI Response Handling

| Response | Code | RAPIDS Behavior |
| --- | --- | --- |
| OKAY | 2’b00 | Normal completion |
| EXOKAY | 2’b01 | Treated as OKAY |
| SLVERR | 2’b10 | Error, transfer aborted |
| DECERR | 2’b11 | Error, transfer aborted |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	AXIS Interface Specification

## 13.1	Overview

RAPIDS Beats uses AXI-Stream interfaces for network data transfer:

- Sink AXIS Slave - Receives data from network (ingress)

- Source AXIS Master - Sends data to network (egress)

These are the ports of the AXIS tops (rapids_beats_top, rapids_top). The same AXIS stream, with tid = channel, is also what every network plane presents to the core, so the contract below holds inside a CXS top too: the CXS planes convert it to and from CXS flits (see the CXS Network Interface chapter).

## 13.2	Sink AXIS Slave

### 13.2.1	Purpose

Receives streaming data from external network interface and writes to SRAM buffer.

### 13.2.2	Configuration

Sink AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | TDATA width |
| ID Width | 8-bit | 0-8 | TID width (optional) |
| DEST Width | 8-bit | 0-8 | TDEST width (optional) |
| USER Width | 1-bit | 0-16 | TUSER width (optional) |

### 13.2.3	Signal List

Sink AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_sink_tdata | 512 | input | Data payload |
| s_axis_sink_tkeep | 64 | input | Byte valid mask |
| s_axis_sink_tstrb | 64 | input | Byte position (optional) |
| s_axis_sink_tlast | 1 | input | End of packet |
| s_axis_sink_tid | 8 | input | Transaction ID (channel select) |
| s_axis_sink_tdest | 8 | input | Destination (optional) |
| s_axis_sink_tuser | 1 | input | User sideband (optional) |
| s_axis_sink_tvalid | 1 | input | Data valid |
| s_axis_sink_tready | 1 | output | Ready to accept |

Timing Diagram

### 13.2.4	Backpressure Behavior

When SRAM buffer is full, tready deasserts:

Waveform 10

## 13.3	Source AXIS Master

### 13.3.1	Purpose

Sends streaming data from SRAM buffer to external network interface.

### 13.3.2	Configuration

Source AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | TDATA width |
| ID Width | 8-bit | 0-8 | TID width (optional) |
| DEST Width | 8-bit | 0-8 | TDEST width (optional) |
| USER Width | 1-bit | 0-16 | TUSER width (optional) |

### 13.3.3	Signal List

Source AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_src_tdata | 512 | output | Data payload |
| m_axis_src_tkeep | 64 | output | Byte valid mask |
| m_axis_src_tstrb | 64 | output | Byte position (optional) |
| m_axis_src_tlast | 1 | output | End of packet |
| m_axis_src_tid | 8 | output | Transaction ID (channel) |
| m_axis_src_tdest | 8 | output | Destination (optional) |
| m_axis_src_tuser | 1 | output | User sideband (optional) |
| m_axis_src_tvalid | 1 | output | Data valid |
| m_axis_src_tready | 1 | input | Downstream ready |

Timing Diagram

### 13.3.4	Data Availability

Source AXIS only asserts tvalid when data is available in SRAM:

Waveform 12

## 13.4	AXIS Protocol Notes

### 13.4.1	Handshake Rules

Standard AXI-Stream handshake applies:

- tvalid asserts when data is available

- tready asserts when receiver can accept

- Transfer occurs on clock edge when both are high

- tvalid SHALL NOT depend on tready

- tready MAY depend on tvalid

### 13.4.2	TLAST Semantics

TLAST Usage

| Scenario | TLAST Behavior |
| --- | --- |
| Descriptor transfer complete | Assert on last beat |
| Packet boundary | Assert on last beat of packet |
| Continuous stream | Not used (always 0) |

### 13.4.3	TKEEP Usage

TKEEP indicates valid bytes within each beat:

TKEEP Examples

| TKEEP Value | Meaning |
| --- | --- |
| 64’hFFFF_FFFF_FFFF_FFFF | All 64 bytes valid |
| 64’h0000_FFFF_FFFF_FFFF | Upper 16 bytes invalid |
| 64’h0000_0000_0000_000F | Only lower 4 bytes valid |

### 13.4.4	Channel Multiplexing via TID

Multiple channels can share a single AXIS interface using TID:

Waveform 13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	APB Configuration Interface

## 14.1	Overview

At the top level (rapids_beats_top), RAPIDS Beats exposes a single APB4 slave (s_apb_*) for all software access. This interface allows software to:

- Configure channel and monitor parameters (via the register block)

- Initiate descriptor processing (kick-off)

- Read status and error information

## 14.2	APB Slave and Address Routing

The APB slave is converted to an internal command/response transaction and routed by address to two targets:

APB Address Routing

| Address Range | Target | Purpose |
| --- | --- | --- |
| 0x000-0x03F | rapids_regs (staged descriptor addresses) | Per-channel CH*_DESC_ADDR_{LOW,HIGH}, stride 0x8 |
| 0x040 | rapids_regs (KICK_ENABLE) | Per-channel launch bits (singlepulse) |
| 0x100-0x3FF | rapids_regs (base regfile) | Channel/scheduler/descriptor configuration and status |
| 0x1000+ | rapids_regs (monitor regfile) | AXI-monitor configuration and performance counters |

Because the monitor regfile is located at 0x1000, the APB address bus must be at least 13 bits wide to reach it. Configuration that was formerly imagined as discrete cfg_* ports is now driven internally by the register block through rapids_config_block; see the Register Map for the full address map, and the descriptor address ranges (DESCENG_ADDR0/1_* at 0x224-0x230).

### 14.2.1	APB Slave Signal List

APB Slave Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_apb_paddr | APB_ADDR_WIDTH (>=13) | input | Address |
| s_apb_psel | 1 | input | Select |
| s_apb_penable | 1 | input | Enable phase |
| s_apb_pwrite | 1 | input | Write / read |
| s_apb_pwdata | 32 | input | Write data |
| s_apb_pstrb | 4 | input | Write byte strobes |
| s_apb_prdata | 32 | output | Read data |
| s_apb_pready | 1 | output | Ready |
| s_apb_pslverr | 1 | output | Slave error |

### 14.2.2	Descriptor Kick-Off

To start descriptor processing on a channel:

- Write descriptor to memory

- Assert apb_valid[ch] with descriptor address on apb_addr

- Wait for apb_ready[ch] assertion

- Deassert apb_valid[ch]

Timing Diagram

## 14.3	Status Interface

### 14.3.1	Per-Channel Status Signals

Status Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| channel_idle | NC | output | Channel idle status |
| channel_error | NC | output | Channel error flag |
| scheduler_state | 4*NC | output | FSM state per channel |

### 14.3.2	Channel State Encoding

Scheduler State Encoding

| State | Value | Description |
| --- | --- | --- |
| IDLE | 4’h0 | Waiting for kick-off |
| WAIT_DESC | 4’h1 | Waiting for descriptor |
| PARSE_DESC | 4’h2 | Parsing descriptor |
| CH_XFER_DATA | 4’h3 | Transfer in progress |
| CHECK_NEXT | 4’h4 | Checking next descriptor |
| COMPLETE | 4’h5 | Transfer complete |
| ERROR | 4’hF | Error state |

## 14.4	Configuration Registers

### 14.4.1	Address Range Validation

RAPIDS validates descriptor addresses against configurable ranges:

Valid if:
  (addr >= cfg_addr0_base && addr < cfg_addr0_limit) ||
  (addr >= cfg_addr1_base && addr < cfg_addr1_limit)

### 14.4.2	Configuration Registers

Configuration and status registers are provided by the rapids_regs register block. See the Register Map for the complete, RTL-accurate address map. The registers most relevant to bring-up are:

Key Configuration Registers

| Register | Offset | Description |
| --- | --- | --- |
| GLOBAL_CTRL | 0x100 | GLOBAL_EN, GLOBAL_RST (level; write 0 to release) |
| CHANNEL_ENABLE | 0x120 | Per-channel enable bitmap |
| SCHED_TIMEOUT_CYCLES | 0x200 | Write-progress timeout window |
| SCHED_TIMEOUT_LIMIT | 0x208 | Consecutive-timeout escalation limit (0 = never) |
| DESCENG_CONFIG | 0x220 | Descriptor-engine enable / prefetch / FIFO threshold |
| DESCENG_ADDR0_BASE/_LIMIT | 0x224/0x228 | Descriptor address range 0 |

## 14.5	Programming Sequence

### 14.5.1	Initialization

// 1. Optional global reset: GLOBAL_RST is a level, not self-clearing
GLOBAL_CTRL = 0x2;           // assert
GLOBAL_CTRL = 0x0;           // release

// 2. Configure descriptor address range(s)
DESCENG_ADDR0_BASE  = 0x8000_0000;
DESCENG_ADDR0_LIMIT = 0x9000_0000;

// 3. Scheduler timeout policy
SCHED_TIMEOUT_CYCLES = 1000;
SCHED_TIMEOUT_LIMIT  = 4;    // escalate after 4 stalled windows; 0 = never

// 4. Enable channels and global enable
CHANNEL_ENABLE = 0xFF;       // enable all 8 channels
GLOBAL_CTRL    = 0x1;        // GLOBAL_EN

### 14.5.2	Descriptor Kick-Off

// Prepare descriptor in memory
desc->src_addr = src;
desc->dst_addr = dst;
desc->length = beats;
desc->last = 1;
desc->valid = 1;

// Kick off channel 0: stage the address, then launch
CH0_DESC_ADDR_LOW  = (uint32_t)desc_addr;
CH0_DESC_ADDR_HIGH = (uint32_t)(desc_addr >> 32);
KICK_ENABLE        = 0x01;   // KICK0, self-clearing

// Wait for completion: idle, or faulted (CH_ERROR is never idle)
while (!(SCHEDULER_IDLE & 0x01) && !(SCHED_ERROR & 0x01));

// Check for errors
if (SCHED_ERROR & 0x01) {
    handle_error(0);
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Monitor Bus Interface

## 15.1	Overview

RAPIDS Beats outputs a 64-bit Monitor Bus (MonBus) for real-time event reporting. The MonBus provides visibility into:

- State machine transitions

- Descriptor processing events

- Error conditions

- Performance metrics

## 15.2	MonBus Packet Format

### 15.2.1	64-bit Packet Structure

Bit Field          Width   Description
───────────────────────────────────────────────────────
[63:60]            4-bit   packet_type (error, completion, etc.)
[59:57]            3-bit   protocol (CORE for RAPIDS)
[56:53]            4-bit   event_code (specific event)
[52:47]            6-bit   channel_id
[46:43]            4-bit   unit_id (subsystem)
[42:35]            8-bit   agent_id (module)
[34:0]             35-bit  event_data (event-specific)

### 15.2.2	Signal List

MonBus Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | 1 | output | Packet valid |
| monbus_pkt_data | 64 | output | Packet data |
| monbus_pkt_ready | 1 | input | Downstream ready |

## 15.3	Packet Types

MonBus Packet Types

| Type | Code | Description |
| --- | --- | --- |
| ERROR | 4’h0 | Error event |
| COMPLETION | 4’h1 | Transfer/operation complete |
| THRESHOLD | 4’h2 | Threshold crossed |
| TIMEOUT | 4’h3 | Timeout event |
| PERF | 4’h4 | Performance metric |
| DEBUG | 4’hF | Debug/trace event |

## 15.4	Agent IDs

RAPIDS modules use the following Agent IDs:

RAPIDS Agent IDs

| Agent ID | Module | Description |
| --- | --- | --- |
| 0x08 | Descriptor AXI monitor | axi4_master_rd_monlite on the descriptor read master (per half, USE_AXI_MONITORS) |
| 0x09 | Sink-ingress network monitor | The sink RX plane’s monitor-lite: axis4_slave_monlite on s_axis_* (AXIS top) or cxs_monitor_lite in cxs_slave_monlite on s_cxs_* (CXS top) (USE_AXI_MONITORS; config SNK.MON.WRMON_*) |
| 0x0A | Source-egress network monitor | The source TX plane’s monitor-lite: axis4_master_monlite on m_axis_* (AXIS top) or cxs_monitor_lite in cxs_master_monlite on m_cxs_* (CXS top) (USE_AXI_MONITORS; config SRC.MON.RDMON_*) |
| 0x10-0x17 | Descriptor Engine | Channels 0-7 |
| 0x20 | Control Read Engine | ctrlrd (one per half) |
| 0x21 | Control Write Engine | ctrlwr (one per half) |
| 0x30-0x37 | Scheduler | Channels 0-7 |

These are the values the scheduler group array and the halves pass down (DESC_AXI_MON_AGENT_ID, DESC_MON_BASE_AGENT_ID, CTRLRD_MON_AGENT_ID, CTRLWR_MON_AGENT_ID, SCHED_MON_BASE_AGENT_ID); the module defaults inside the engines differ and are overridden at instantiation. The data paths carry no MonBus emitters (an earlier revision listed data-path agents at 0x40-0x57, which nothing emits). Agents 0x09 and 0x0A live in the network planes, outside the core; their packets enter each half’s 3:1 monbus_arbiter as client 1 on snk_plane_mon_* / src_plane_mon_*. A CXS plane’s packets carry protocol PROTOCOL_CXS, which the top’s monitor group filters through its AXIS mask slot. A CXS PACKET_END completion carries the channel (CXSSRCID) in event_data[55:48]; its channel_id field holds {PRCLTYPE[2:0], TGTID[5:0]}.

## 15.5	Event Codes

### 15.5.1	Scheduler Events (Agent 0x30-0x37)

Scheduler Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor complete |
| 0x1 | COMPLETION | Chain complete |
| 0x2 | ERROR | Timeout |
| 0x3 | ERROR | AXI error |
| 0x4 | DEBUG | State transition |

The scheduler’s and the descriptor engine’s COMPLETION packets are reported only while the half’s SCHED_CONFIG.COMPL_EN is set (reset value 1). With the bit clear the scheduler group drops them before its monitor-bus arbiter and still acknowledges the emitters, so a channel runs identically either way; ERROR packets are reported regardless of ERR_EN.

### 15.5.2	Descriptor Engine Events (Agent 0x10-0x17)

Descriptor Engine Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor fetched |
| 0x1 | ERROR | Address range error |
| 0x2 | ERROR | AXI read error |
| 0x3 | DEBUG | Fetch started |

Timing Diagram

## 15.6	Event Data Encoding

### 15.6.1	Completion Event Data

[34:32] - Reserved
[31:0]  - Transfer length (beats completed)

### 15.6.2	Error Event Data

[34:32] - Error type
[31:16] - Reserved
[15:0]  - Error details

### 15.6.3	State Transition Event Data

[34:32] - Reserved
[31:28] - Previous state
[27:24] - New state
[23:0]  - Timestamp (lower bits)

## 15.7	MonBus Arbitration

Multiple RAPIDS modules generate MonBus packets. Internal arbitration ensures ordered delivery:

### 15.7.1	Figure 3.5.1: MonBus Arbitration Tree

MonBus Arbitration Tree

Source: ch03_monbus_arbitration.dot

### 15.7.2	Arbitration Policy

- Round-robin across modules

- No packet loss (backpressure if output blocked)

- Priority boosting for ERROR packets

## 15.8	Top-Level MonBus Delivery (monbus_axil4_axil4_group)

At the top level (rapids_beats_top, rapids_beats_cxs_top), the internal MonBus is not exposed as a raw 64-bit port. Instead, when USE_AXI_MONITORS = 1, the core’s single merged stream (core_mon_*, which already carries the descriptor-monitor and network-plane packets) feeds a monbus_axil4_axil4_group. The data masters m_axi_rd and m_axi_wr carry no monitors. When USE_AXI_MONITORS = 0, the group is not built and its outputs are tied off (mon_irq = 0). The group presents three external interfaces:

Top-Level MonBus AXI-Lite Group

| Interface | Signals | Purpose |
| --- | --- | --- |
| Error-drain slave | s_axil_err_ar*, s_axil_err_r* (AXI-Lite, 32-bit read) | CPU reads captured error events from the error FIFO |
| Capture master | m_axil_mon_aw*, m_axil_mon_w* (64-bit wdata), m_axil_mon_b* | Bulk-writes the MonBus trace to system memory |
| Interrupt | mon_irq | Asserts on error/threshold events |

The capture region and flush behavior are configured by cfg_mon_base_addr, cfg_mon_limit_addr, and cfg_mon_flush_watermark.

## 15.9	Integration Notes

### 15.9.1	Downstream Connection

Internally, MonBus output connects to:

- MonBus Arbiter - Combines core + rd/wr monitor sources

- monbus_axil4_axil4_group - Error-drain slave, capture master, and mon_irq

- Debug FIFO - Buffered access for debug tools

### 15.9.2	Packet Rate

MonBus Packet Rates

| Scenario | Approximate Rate |
| --- | --- |
| Idle | 0 packets/cycle |
| Single channel active | ~1 packet/100 cycles |
| All channels active | ~1 packet/20 cycles |
| Error storm | Up to 1 packet/cycle |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	CXS Network Interface

## 16.1	Overview

The network side of RAPIDS is a swappable network plane. Each top is the tree’s base (core, registers, monitor group) plus one receive (RX) plane and one transmit (TX) plane, and the plane is chosen by the top’s module name:

- rapids_beats_top, rapids_top: AXI-Stream planes; the s_axis_* and m_axis_* ports described in the AXIS Interface chapter.

- rapids_beats_cxs_top, rapids_cxs_top: CXS planes; the sink port (s_cxs_*) into the sink half and the source port (m_cxs_*) out of the source half, one CXS link each.

The core never sees CXS. Every plane presents the same AXIS stream (tid = channel) to the core, so the descriptor engines, schedulers, SRAM buffers and AXI engines are identical in the AXIS and CXS tops. All four tops are generated from the tree’s base by bin/gen_plane_tops.py.

### 16.1.1	Figure 3.6.1: Network Planes

Network Planes

Source: ch03_network_planes.dot

## 16.2	Link Profile

The CXS planes use one fixed profile of Arm IHI 0079 (CXS) Issue D. Issue D is the first issue with the SRCID, TGTID and USER sidebands, which RAPIDS needs to carry the channel and the byte strobes.

CXS Link Profile

| Property | Value |
| --- | --- |
| Links | One per direction: the sink port s_cxs_* into the sink half, the source port m_cxs_* out of the source half |
| Channel | CXSSRCID carries the RAPIDS channel (the AXIS tid) |
| CXSTGTID | The AXIS tdest |
| CXSUSER | {tuser, tstrb}; tuser is dropped when DATA_WIDTH/8 + AXIS_USER_WIDTH > 128 |
| Packets per flit / start alignment | CXSMAXPKTPERFLIT = 2, 16-byte alignment; one AXIS packet is one CXS packet |
| Stream framing | Per stream, keyed on the channel bits of CXSSRCID: channels may interleave mid-packet |
| Link control | Explicit credit return, activation and deactivation; idle deactivation after CXS_IDLE_CYCLES |
| Parity | Off |
| Data width | DATA_WIDTH up to 1024 (CXSUSER is at most 128 bits and carries tstrb) |

Channels interleave on one link. The byte-granular source arbitrates per drain block while a packet spans its whole descriptor, so with two or more source channels active the egress switches channel in the middle of a packet. That is legal CXS (streams may interleave, IHI 0079D 2.5.2) because every block on the link frames packets per stream rather than per link.

## 16.3	Credits

One credit permits one flit (IHI 0079D 2.1). Credits are a single pool for the whole link: a credit grant carries no ID, so it cannot be reserved for one channel.

CXS Top Parameters

| Parameter | Default | Meaning |
| --- | --- | --- |
| CXS_MAX_CREDIT | 32 | Credits the receiver grants. The grant-to-flit-to-free round trip is about 3 cycles, so 32 sustains one flit per cycle with margin for link pipelining or a clock crossing |
| CXS_RX_FIFO_DEPTH | CXS_MAX_CREDIT | The sink plane’s receive FIFO; it is the credit pool, so every granted credit is backed by space |
| CXS_CREDIT_BEYOND_SPEC | CXS_MAX_CREDIT > 63 | Above the spec’s limit of 63 the link is built beyond spec, and the far end must be built the same way |
| CXS_IDLE_CYCLES | 32 | Idle cycles before the TX plane deactivates the link |

The credit count is not tied to SRAM_DEPTH. SRAM_DEPTH is the beats each channel buffers in each direction; the credit pool only has to cover the link’s round trip.

## 16.4	Signal List

The sink port (s_cxs_*) is a CXS receiver; the source port (m_cxs_*) is a CXS transmitter. Direction is given at the RAPIDS top.

CXS Link Signals

| Signal | Sink port (s_cxs_*) | Source port (m_cxs_*) | Width | Description |
| --- | --- | --- | --- | --- |
| *_cxs_valid | input | output | 1 | Flit valid |
| *_cxs_data | input | output | DATA_WIDTH | Flit data |
| *_cxs_cntl | input | output | CXS_CNTL_PW | Packet start/end control (CXSCNTL) |
| *_cxs_srcid | input | output | AXIS_ID_WIDTH | Source ID; the channel |
| *_cxs_tgtid | input | output | AXIS_DEST_WIDTH | Target ID; the AXIS tdest |
| *_cxs_user | input | output | CXS_UW | {tuser, tstrb} |
| *_cxs_crdgnt | output | input | 1 | Credit grant |
| *_cxs_crdrtn | input | output | 1 | Credit return |
| *_cxs_activereq | input | output | 1 | Link activation request |
| *_cxs_activeack | output | input | 1 | Link activation acknowledge |
| *_cxs_deacthint | output | input | 1 | Deactivation hint |

CXS Status Signals

| Signal | Direction | Description |
| --- | --- | --- |
| snk_cxs_link_state, src_cxs_link_state | output [1:0] | {ACTIVEREQ, ACTIVEACK} of each link |
| snk_cxs_protocol_error, src_cxs_protocol_error | output | Sticky: the endpoint saw a CXS protocol violation |
| snk_cxs_unsupported | output | Sticky: a received flit that AXIS framing cannot represent |

## 16.5	Flow Control and the Sink Contract

The byte-granular sink holds s_axis_tready low on an AXIS top until the channel’s descriptor has been fetched. On a CXS top the same backpressure appears as credits: the receive FIFO fills and the sink plane stops granting credits until the core drains it.

## 16.6	Monitoring

Each CXS plane carries a cxs_monitor_lite in place of the AXIS monitor-lite. It is programmed from the same CSRs (SNK.WRMON_* for the sink plane, SRC.RDMON_* for the source plane) and reports with the same agent IDs (0x09 sink, 0x0A source). Its packets are tagged PROTOCOL_CXS and join the half’s 3:1 monbus_arbiter as client 1, exactly where the AXIS plane’s packets go. See the Monitor Bus Interface chapter.

## 16.7	Routing and Payload Type (Proposed)

A proposal (2026-10-16, not implemented) gives every packet a header carrying its destination coordinate, source coordinate, payload type (cfg / prog / mem), length and target address. The header belongs to the tile network, not to CXS, and is described once in the compute-engine tile’s TILE_SPEC.md Section 11. CXS only transports it, as the first flit of the packet. Nothing on this page changes: CXSSRCID keeps the channel, so per-stream framing is the same. The descriptor fields that supply the header are in the Descriptor Format chapter.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Use Case: Network to Memory (Sink Path)

## 17.1	Overview

The Sink Path transfers data from a network interface (AXI-Stream slave) to system memory (AXI4 write master). This is the primary receive data path for network applications.

Operation Flow

Timing Diagram

## 17.2	Buffer Management

### 17.2.1	SRAM Buffering Strategy

The sink path uses internal SRAM to decouple network timing from memory timing:

### 17.2.2	Figure 4.1.1: Sink Channel Buffer

Sink Channel Buffer

Source: ch04_sink_buffering.dot

### 17.2.3	Flow Control

Sink Path Flow Control

| Condition | Behavior |
| --- | --- |
| SRAM full | Backpressure network (TREADY=0) |
| SRAM empty | Pause AXI writes |
| SRAM threshold | Optional early write start |

Watermark Timing

## 17.3	Performance Considerations

### 17.3.1	Throughput

Sink Path Throughput Factors

| Factor | Impact |
| --- | --- |
| Network bandwidth | Upper bound on receive rate |
| Memory bandwidth | Must match or exceed network |
| SRAM depth | Burst absorption capability |
| AXI outstanding | Overlapped write latency hiding |

### 17.3.2	Latency

Sink Path Latency Breakdown

| Phase | Typical Cycles |
| --- | --- |
| Descriptor fetch | 10-50 (memory dependent) |
| First beat to SRAM | 1 |
| SRAM to AXI write | 2-4 |
| AXI write completion | 10-100 (memory dependent) |

## 17.4	Error Handling

### 17.4.1	Error Sources

Sink Path Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI write error | BRESP != OKAY | Stop transfer, report via MonBus |
| SRAM overflow | Fill pointer catches drain | Should never occur (TREADY control) |
| Timeout | Watchdog timer | Abort transfer, report error |

Error Recovery Sequence

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Use Case: Memory to Network (Source Path)

## 18.1	Overview

The Source Path transfers data from system memory (AXI4 read master) to a network interface (AXI-Stream master). This is the primary transmit data path for network applications.

Operation Flow

Timing Diagram

## 18.2	Buffer Management

### 18.2.1	SRAM Buffering Strategy

The source path uses internal SRAM to prefetch data and maintain network throughput:

### 18.2.2	Figure 4.2.1: Source Channel Buffer

Source Channel Buffer

Source: ch04_source_buffering.dot

### 18.2.3	Flow Control

Source Path Flow Control

| Condition | Behavior |
| --- | --- |
| SRAM full | Pause AXI reads |
| SRAM empty | Hold TVALID low |
| Network backpressure | Stop draining SRAM |

Prefetch Strategy

## 18.3	Concurrent Operation

### 18.3.1	Read-Ahead Pipeline

The source path can issue AXI reads ahead of network consumption:

Waveform 20

## 18.4	Performance Considerations

### 18.4.1	Throughput

Source Path Throughput Factors

| Factor | Impact |
| --- | --- |
| Memory bandwidth | Upper bound on transmit rate |
| Network acceptance | Must keep pace with reads |
| SRAM depth | Read-ahead capability |
| AXI outstanding | Overlapped read latency hiding |

### 18.4.2	Latency

Source Path Latency Breakdown

| Phase | Typical Cycles |
| --- | --- |
| Descriptor fetch | 10-50 (memory dependent) |
| AXI read issue | 1-2 |
| Memory read latency | 10-100 (memory dependent) |
| SRAM to network | 1-2 |

## 18.5	Error Handling

### 18.5.1	Error Sources

Source Path Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI read error | RRESP != OKAY | Stop transfer, report via MonBus |
| Network timeout | Watchdog timer | Abort transfer, report error |
| Address range | Descriptor validation | Reject descriptor |

Backpressure Handling

When the network deasserts TREADY: 1. SRAM drain stalls immediately 2. SRAM continues filling until threshold 3. AXI reads pause when SRAM approaches full 4. Resume automatically when TREADY returns

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Use Case: Descriptor Chaining

## 19.1	Overview

Descriptor chaining enables multiple transfers to execute sequentially without software intervention. Each descriptor contains a pointer to the next descriptor, forming a linked list that RAPIDS processes automatically.

## 19.2	Chain Structure

### 19.2.1	Figure 4.3.1: Descriptor Chain Structure

Descriptor Chain Structure

Source: ch04_chain_structure.dot

Chain Execution Flow

Timing Diagram

## 19.3	Descriptor Fields for Chaining

### 19.3.1	Relevant Fields

Chaining-Related Descriptor Fields

| Field | Bits | Description |
| --- | --- | --- |
| next_ptr | [191:128] | 64-bit pointer to next descriptor |
| last | [255] | Last descriptor in chain flag |
| irq_en | [254] | Generate interrupt on completion |

### 19.3.2	Chain Termination

A chain terminates when ANY of these conditions is true:

- next_ptr == 0 (null pointer)

- last == 1 (explicit last flag)

- Error during transfer

Diagram 10

## 19.4	Prefetch Optimization

### 19.4.1	Descriptor Prefetching

RAPIDS can prefetch the next descriptor while the current transfer executes:

Waveform 23

Benefit: Zero-cycle gap between chained transfers when prefetch completes before current transfer.

Prefetch depth is controlled by DESCENG_CONFIG.PREFETCH_EN and DESCENG_CONFIG.FIFO_THRESH (register block, offset 0x220). With prefetch disabled the descriptor engine fetches on demand (roughly one descriptor ahead of the scheduler); with prefetch enabled it buffers up to FIFO_THRESH descriptors ahead. A chain fetch that is throttled (output FIFO at the threshold) is deferred and reissued automatically once the FIFO drains, so no chain link is lost.

## 19.5	Multi-Channel Chaining

Each channel maintains independent chains:

### 19.5.1	Figure 4.3.2: Independent Per-Channel Chains

Independent Per-Channel Chains

Source: ch04_multichannel_chains.dot

Channels execute their chains concurrently and independently.

## 19.6	Error During Chain

### 19.6.1	Error Handling

When an error occurs mid-chain:

Waveform 24

Error Response: 1. Current transfer aborts 2. Chain processing stops 3. Error status captures failing descriptor index 4. MonBus reports error event 5. Interrupt generated (if enabled)

### 19.6.2	Recovery

Software must: 1. Read error status to identify failing descriptor 2. Fix the issue (descriptor content, memory mapping, etc.) 3. Restart chain from failing descriptor or beginning

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Use Case: Multi-Channel Operation

## 20.1	Overview

RAPIDS Beats supports 8 independent DMA channels that can operate concurrently. This enables parallel data transfers for multiple network connections, priority-based scheduling, and efficient resource utilization.

## 20.2	Channel Independence

### 20.2.1	Figure 4.4.1: Independent Channels

Independent Channels

Source: ch04_channel_independence.dot

Concurrent Operation Timing

## 20.3	Resource Arbitration

### 20.3.1	Descriptor Port Arbitration

All channels share a single AXI port for descriptor fetches:

Waveform 26

### 20.3.2	Sink/Source Arbitration

Data path AXI ports use plain round-robin arbitration (arbiter_round_robin inside each engine). There is no priority boost: the descriptor’s priority field is carried but not used for arbitration.

### 20.3.3	Figure 4.4.2: Data-Path AXI Arbitration

Data-Path AXI Arbitration

Source: ch04_datapath_arbitration.dot

## 20.4	Channel Isolation

### 20.4.1	Error Isolation

Errors in one channel do not affect other channels:

Waveform 27

### 20.4.2	Per-Channel Soft Reset

Individual channels can be reset without affecting others:

Waveform 28

## 20.5	Use Case Examples

### 20.5.1	Mixed Traffic Pattern

Typical network application with different traffic types:

Mixed Traffic Channel Assignment

| Channel | Traffic Type | Direction | Priority | Notes |
| --- | --- | --- | --- | --- |
| 0 | Voice/RTP | Sink | High | Low latency required |
| 1 | Voice/RTP | Source | High | Low latency required |
| 2 | Video | Sink | Medium | High bandwidth |
| 3 | Control | Source | Medium | Small packets |
| 4-7 | Bulk data | Both | Low | Background transfers |

### 20.5.2	Bidirectional Transfer

Single network connection using paired channels:

### 20.5.3	Figure 4.4.3: Bidirectional Transfer

Bidirectional Transfer

Source: ch04_bidirectional.dot

### 20.5.4	Load Balancing

Multiple channels processing same traffic type:

### 20.5.5	Figure 4.4.4: Load Balancing by tid

Load Balancing by tid

Source: ch04_load_balancing.dot

## 20.6	Performance Considerations

### 20.6.1	Bandwidth Sharing

Bandwidth Distribution

| Active Channels | Per-Channel BW (% of max) | Notes |
| --- | --- | --- |
| 1 | 100% | Full bandwidth available |
| 2 | ~50% each | Round-robin sharing |
| 4 | ~25% each | Fair distribution |
| 8 | ~12.5% each | All channels active |

Note: Actual distribution depends on transfer sizes and memory latency.

### 20.6.2	Latency Impact

Multi-channel operation adds arbitration latency:

Arbitration Latency

| Scenario | Additional Latency |
| --- | --- |
| Single channel | 0 cycles |
| 2 channels contending | 1-2 cycles |
| 8 channels contending | Up to 7 cycles |

## 20.7	MonBus Events

Each channel generates independent MonBus events:

Waveform 29

Events are arbitrated and serialized on the shared MonBus output.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Descriptor Format

## 21.1	Overview

RAPIDS Beats uses 256-bit (32-byte) descriptors to define DMA transfers. Descriptors are stored in system memory and fetched by the Descriptor Engine when a channel is kicked.

## 21.2	Descriptor Layout

### 21.2.1	256-Bit Structure

Bit Range    Field Name           Width  Description
────────────────────────────────────────────────────────────────────────
[255:213]    reserved             43     Reserved (write 0)
[212:210]    desc_type            3      0 = LEGACY, 1 = EXTENDED (see below)
[209:208]    opcode               2       0 = DATA, 1 = CTRL_READ, 2 = CTRL_WRITE
[207:200]    priority             8      Transfer priority (informational)
[199:196]    channel_id           4      Channel ID (informational)
[195]        error                1      Error flag (written by hardware)
[194]        last                 1      Last descriptor in chain
[193]        gen_irq              1      Generate interrupt on completion
[192]        valid                1      Descriptor valid flag
[191:160]    next_descriptor_ptr  32     Address of next descriptor (0 = last)
[159:128]    length               32     Transfer length in BEATS (not bytes)
[127:64]     dst_addr             64     Destination address
[63:0]       src_addr             64     Source address

: 256-Bit Descriptor Layout (rapids_pkg::descriptor_t)

Tile header fields (byte-granular RAPIDS, TILE_HDR = 1, 2026-10-17): four descriptor fields supply the tile packet header defined in the compute-engine tile’s TILE_SPEC.md Section 11. They use bits that are otherwise unused, and are ignored when TILE_HDR = 0. RAPIDS Beats does not implement them.

Tile Header Descriptor Fields

| Field | Descriptor bits | Meaning |
| --- | --- | --- |
| ptype | [215:213] (from the reserved [255:213]) | 0 = cfg, 1 = prog, 2 = mem; 3 (credit) and 4 (desc) are generated by the tile, never by a descriptor |
| op | [216] | 0 = write (the source half sends length bytes from src_addr), 1 = read (header only; the target returns length bytes to the sink port) |
| Destination coordinate | dst_addr[63:50] = descriptor [127:114] | {level, X, Y, Z} (TILE_SPEC Section 11.3) |
| Target address | dst_addr[49:0] = descriptor [113:64] | Byte address inside the addressed node or cell |

The header’s tag is not a descriptor field: it is the channel that issued the descriptor. The coordinate and target address apply to SOURCE descriptors, whose dst_addr is otherwise unused. The source half builds the header from these fields and sends it as the first beat of the packet, ahead of the payload; a read (op = 1) is that header alone and reads no memory. The coordinate is passed through as written: RAPIDS does not check it.

A cell can also hand RAPIDS a complete source descriptor in this format (a desc packet through the sink port). It goes straight into the named source channel’s descriptor queue, with no fetch; see TILE_SPEC Section 11.11. An EXT descriptor cannot be handed in this way (its second chunk has no way in) and is dropped with a descriptor error.

The authoritative definition is rapids_pkg::descriptor_t in rtl/includes/rapids_pkg.sv; the same layout is restated in the scheduler’s header comment. Note the struct is declared 272 bits wide with a 16-bit pad above bit 255 – only [255:0] is fetched and used.

Visual Layout

## 21.3	Field Descriptions

### 21.3.1	Control Bits [195:192]

Control Bits

| Bit | Field | Description |
| --- | --- | --- |
| 192 | valid | Must be 1; a descriptor with valid=0 raises a descriptor error |
| 193 | gen_irq | Set to 1 to generate a completion interrupt |
| 194 | last | Set to 1 for the last descriptor in a chain |
| 195 | error | Error flag; written by hardware, write 0 |

### 21.3.2	Direction

There is no direction field in the descriptor. Direction is a property of the half that owns the channel: the SOURCE half (memory -> AXIS) reads from src_addr, the SINK half (AXIS -> memory) writes to dst_addr. A descriptor is delivered to one half’s descriptor engine by the address it was staged at, so the same layout serves both.

### 21.3.3	Transfer Length [159:128]

- 32-bit field specifying transfer size in BEATS, not bytes

- 1 beat = DATA_WIDTH bits (512 bits = 64 bytes in the default build)

- Length of 0 is reserved (no operation)

### 21.3.4	Next Pointer [191:160]

- 32-bit address of the next descriptor (zero-extended to the engine’s 64-bit address width)

- Value of 0 terminates the chain, as does last = 1

- Autonomous chaining also requires the address to fall inside one of the two configured descriptor address ranges

### 21.3.5	Address Fields [127:0]

For SINK transfers (network to memory): | Field | Usage | |——-|——-| | dst_addr [127:64] | Memory write destination address | | src_addr [63:0] | Not used (write 0) |

For SOURCE transfers (memory to network): | Field | Usage | |——-|——-| | dst_addr [127:64] | Not used (write 0) | | src_addr [63:0] | Memory read source address |

: Address Field Usage

## 21.4	Descriptor Opcodes (DATA / CTRL_READ / CTRL_WRITE)

Beyond plain DATA transfers, a descriptor carries a 2-bit opcode (rapids_pkg desc_op_e) that selects one of three behaviors. DATA descriptors move payload through the concurrent read/write engines (above); the two control opcodes let a channel synchronize with a producer/consumer in memory without moving payload – the basis of the RAPIDS producer/consumer flow.

| Opcode | Value | Behavior |
| --- | --- | --- |
| DATA | 2'b00 | Concurrent read/write payload transfer (fields as above) |
| CTRL_READ | 2'b01 | Consumer gate – poll a memory location until (read & mask) == expected, then release the descriptor/chain |
| CTRL_WRITE | 2'b10 | Producer doorbell – single 32-bit write of a value to a memory location, then continue |

CTRL_READ (consumer gate) re-interprets the descriptor fields as a poll specification. The scheduler drives the per-channel control-read engine, which reads the gate address once per retry and compares the masked value; the descriptor completes when the gate is satisfied. The retry budget is bounded by CTRL_CONFIG.CTRLRD_MAX_TRY (0-511, reset 16) so a never-satisfied gate cannot hang the channel – exhaustion raises an error instead.

| Field | Bits | Usage |
| --- | --- | --- |
| poll_addr | [63:0] | Gate address to poll |
| expected | [95:64] | Expected value after masking |
| mask | [127:96] | Compare mask ((read & mask) == expected) |
| max_try | [143:128] | Per-descriptor retry hint (capped by CTRLRD_MAX_TRY) |

CTRL_WRITE (producer doorbell) performs one single-beat 32-bit AXI write, after which the chain continues. It has no poll/retry – it is an unconditional write used to signal a consumer.

| Field | Bits | Usage |
| --- | --- | --- |
| wr_addr | [63:0] | Doorbell address |
| wr_data | [95:64] | 32-bit value to write |

The control opcodes are handled by dedicated engines (control-read / control-write), one pair per channel, driven by the scheduler’s control interface. See the MAS Scheduler (Section 2.1) and Control-Read / Control-Write Engine specifications (Sections 2.8 / 2.9).

## 21.5	Extended Descriptors (row/col-major addressing)

desc_type = 1 (EXTENDED) selects strided / 2-D / circular addressing instead of linear accumulation. It is gated at build time by USE_ROW_COL_MAJOR_ADDRESSING; when that parameter is 0 the second-chunk fetch is unreachable and synthesizes away, and an EXT descriptor is treated as legacy.

An extended descriptor is 512 bits in two 256-bit chunks. Chunk 0 is the layout above with desc_type = 1. Chunk 1 holds the address-generator configuration and is fetched by a second single-beat read at descriptor_addr + 0x20, so the two chunks must be contiguous in memory.

Chunk 1 (at descriptor_addr + 0x20)
Bit Range    Field Name        Width  Description
──────────────────────────────────────────────────────────────────────
[255:192]    reserved_hi       64     Reserved (future 3rd dimension)
[191:188]    wr_reserved       4      Reserved
[187:182]    wr_wrap1_log2     6      Write outer wrap window, log2 (0 = off)
[181:176]    wr_wrap0_log2     6      Write inner wrap window, log2 (0 = off)
[175:160]    wr_inner_count    16     Write beats per contiguous run
[159:128]    wr_stride_1       32     Write outer stride, SIGNED bytes
[127:96]     wr_stride_0       32     Write inner stride, SIGNED bytes
[95:92]      rd_reserved       4      Reserved
[91:86]      rd_wrap1_log2     6      Read outer wrap window, log2 (0 = off)
[85:80]      rd_wrap0_log2     6      Read inner wrap window, log2 (0 = off)
[79:64]      rd_inner_count    16     Read beats per contiguous run
[63:32]      rd_stride_1       32     Read outer stride, SIGNED bytes
[31:0]       rd_stride_0       32     Read inner stride, SIGNED bytes

: Extended Descriptor Chunk 1 (rapids_pkg::descriptor_ext_t)

Read and write are configured independently, which is what makes transpose possible: one side bursts while the other issues single beats.

Mode selection is implicit. The hardware compares stride_0 against the beat size (DATA_WIDTH/8):

| Condition | Mode | Behaviour |
| --- | --- | --- |
| stride_0 == beat_size | run-contiguous | bursts inner_count beats, then jumps to the next run base |
| stride_0 != beat_size | per-beat 2-D | one beat per address; transpose, gather, scatter |

Addresses follow addr = base + (i0 * stride_0) + (i1 * stride_1), with each offset masked by (1 << wrap_log2) - 1 when that wrap field is non-zero, giving circular buffers. i0 is the fast index and runs to inner_count. Strides are signed, so negative values walk backwards.

The layout is byte-compatible with STREAM’s descriptor_ext_t, so one descriptor builder serves both engines.

Descriptor Fetch Timing

## 21.6	Alignment Requirements

Address Alignment Requirements

| Field | Alignment | Notes |
| --- | --- | --- |
| Descriptor address | 32-byte | One descriptor is 32 bytes; an EXTENDED descriptor’s chunk 1 is fetched at +0x20 |
| next_ptr [191:160] | 32-byte | 32-bit descriptor address, zero-extended by the engine |
| dst_addr [127:64] | DATA_WIDTH/8 | 64-byte for 512-bit data |
| src_addr [63:0] | DATA_WIDTH/8 | 64-byte for 512-bit data |

Note: Unaligned addresses may cause unpredictable behavior or AXI protocol violations.

## 21.7	Descriptor Examples

### 21.7.1	Sink Descriptor (Network to Memory)

A 256-beat sink transfer to 0x1_0000_0000, chained to a descriptor at 0x2000, raising an interrupt on completion.

64-bit words, LSB word first:
[63:0]    = 0x0000_0000_0000_0000  // src_addr  (unused by the SINK half)
[127:64]  = 0x0000_0001_0000_0000  // dst_addr  = 0x1_0000_0000
[191:128] = 0x0000_2000_0000_0100  // next_ptr[191:160]=0x2000, length[159:128]=256
[255:192] = 0x0000_0000_0000_0007  // valid=1, gen_irq=1, last=1

Word 3 detail:
  [192] = 1        // valid
  [193] = 1        // gen_irq
  [194] = 1        // last
  [195] = 0        // error (hardware-written)
  [199:196] = 0    // channel_id (informational)
  [207:200] = 0    // priority
  [209:208] = 00   // opcode = DATA
  [212:210] = 000  // desc_type = LEGACY

### 21.7.2	Source Descriptor (Memory to Network)

A 128-beat source transfer from 0x2_0000_0000, terminated by next_ptr = 0.

64-bit words, LSB word first:
[63:0]    = 0x0000_0002_0000_0000  // src_addr = 0x2_0000_0000
[127:64]  = 0x0000_0000_0000_0000  // dst_addr (unused by the SOURCE half)
[191:128] = 0x0000_0000_0000_0080  // next_ptr = 0 (terminates), length = 128
[255:192] = 0x0000_0000_0000_0003  // valid=1, gen_irq=1, last=0

Word 3 detail:
  [192] = 1        // valid
  [193] = 1        // gen_irq
  [194] = 0        // not last -- next_ptr = 0 terminates the chain
  [209:208] = 00   // opcode = DATA
  [212:210] = 000  // desc_type = LEGACY

## 21.8	Software Construction

### 21.8.1	C Structure Example

typedef struct __attribute__((packed, aligned(32))) {
    uint64_t src_addr;      // [63:0]
    uint64_t dst_addr;      // [127:64]
    uint32_t length;        // [159:128]  transfer length in BEATS
    uint32_t next_ptr;      // [191:160]  32-bit, 0 terminates the chain
    uint32_t flags;         // [223:192]  see macros below
    uint32_t reserved[2];   // [255:224]
} rapids_descriptor_t;

// flags bit positions are relative to descriptor bit 192
#define DESC_VALID          (1u << 0)   // [192]
#define DESC_GEN_IRQ        (1u << 1)   // [193]
#define DESC_LAST           (1u << 2)   // [194]
#define DESC_ERROR          (1u << 3)   // [195] hardware-written
#define DESC_CHANNEL_ID(n)  (((n) & 0xFu) << 4)    // [199:196]
#define DESC_PRIORITY(n)    (((n) & 0xFFu) << 8)   // [207:200]
#define DESC_OPCODE(n)      (((n) & 0x3u) << 16)   // [209:208]
#define DESC_TYPE(n)        (((n) & 0x7u) << 18)   // [212:210]

#define DESC_OP_DATA        0x0
#define DESC_OP_CTRL_READ   0x1
#define DESC_OP_CTRL_WRITE  0x2

#define DESC_TYPE_LEGACY    0x0
#define DESC_TYPE_EXT       0x1

### 21.8.2	Descriptor Ring Setup

// Allocate descriptor ring (must be 32-byte aligned)
rapids_descriptor_t *desc_ring = aligned_alloc(32, NUM_DESC * sizeof(rapids_descriptor_t));

// Initialize chain
for (int i = 0; i < NUM_DESC - 1; i++) {
    desc_ring[i].next_ptr = (uint64_t)&desc_ring[i + 1];
    desc_ring[i].control = DESC_CTRL_IRQ_EN;
}

// Last descriptor
desc_ring[NUM_DESC - 1].next_ptr = 0;
desc_ring[NUM_DESC - 1].control = DESC_CTRL_LAST | DESC_CTRL_IRQ_EN;

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Register Map

## 22.1	Overview

RAPIDS Beats provides a memory-mapped register interface for configuration and status. All registers are accessed through the single top-level APB slave (s_apb_*) and are implemented by the PeakRDL-generated rapids_regs register block. One addrmap holds two independent halves of the same layout (rapids_engine_regs): SRC at 0x0000 and SNK at 0x1000, 4 KB each. Within a half, the staged descriptor addresses occupy +0x000-0x03F, the launch register KICK_ENABLE sits at +0x040, configuration and status at +0x100-0x3FF, and the nested monitor regfile (rapids_mon_regs) at +0x800. Every offset below is relative to its half; add 0x1000 for the SNK copy.

Because the SNK half lives at 0x1000, the APB address bus must be at least 13 bits wide; paddr[12] selects the half and paddr[11] the monitor window.

## 22.2	Address Space Layout

| Range (per half) | Target | Purpose |
| --- | --- | --- |
| +0x000-0x03F | rapids_engine_regs | Per-channel staged descriptor addresses |
| +0x040 | rapids_engine_regs | KICK_ENABLE per-channel launch bits |
| +0x100-0x3FF | rapids_engine_regs | Configuration and status |
| +0x800-0x9FF | rapids_mon_regs | AXI-monitor config and performance |
| 0x0000 / 0x1000 | half base | SRC half / SNK half |

The three *_PKT_MASK registers are masks: bit[type] = 1 masks (drops) that packet type, at the monitor-lite and again at the monbus group. They reset to all-masked (0xFFFF), so nothing reaches the group until the host clears the bits of the classes it wants to see. (rapids BUG-008: the description used to read “1 = enable”; the polarity and the reset are as intended.) *_TIMEOUT counts MICROSECONDS in its low 16 bits.

On a build with USE_MON_REGS = 0 (the default when USE_AXI_MONITORS = 0) the two monitor windows keep their addresses but answer every access with PSLVERR and zero read data: the config blocks drive nothing from them, and a read-back that succeeded would tell the host a monitor is configured that does not exist.

: Address Space Layout

## 22.3	Base Registers (0x100-0x3FF)

Base Registers

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | RW | GLOBAL_EN[0], GLOBAL_RST[1] (level; write 0 to release) |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE[0] |
| 0x108 | VERSION | RO | MINOR[7:0], MAJOR[15:8], NUM_CHANNELS[23:16]=8 |
| 0x120 | CHANNEL_ENABLE | RW | CH_EN[7:0] per-channel enable |
| 0x124 | CHANNEL_RESET | RW | CH_RST[7:0] per-channel reset (level; write 0 to release) |
| 0x140 | CHANNEL_IDLE | RO | CH_IDLE[7:0] – not driven at the top level, reads 0; use SCHEDULER_IDLE |
| 0x144 | DESC_ENGINE_IDLE | RO | Per-channel descriptor-engine idle |
| 0x148 | SCHEDULER_IDLE | RO | Per-channel scheduler idle (excludes CH_ERROR) |
| 0x150-0x16C | CH_STATE[0..7] | RO | Per-channel FSM state (stride 0x4) |
| 0x170 | SCHED_ERROR | RO | Per-channel sticky scheduler error |
| 0x174 | AXI_RD_COMPLETE | RO | Per-channel read-complete (SRC half only; reads 0 in SNK) |
| 0x178 | AXI_WR_COMPLETE | RO | Per-channel write-complete – not driven, reads 0 |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | Write-progress timeout window (cycles, reset 1000) |
| 0x204 | SCHED_CONFIG | RW | SCHED_EN, TIMEOUT_EN, ERR_EN, COMPL_EN, PERF_EN |
| 0x208 | SCHED_TIMEOUT_LIMIT | RW | LIMIT[7:0] escalation limit (reset 4, 0 = never) |
| 0x220 | DESCENG_CONFIG | RW | DESCENG_EN[0], PREFETCH_EN[1], FIFO_THRESH[5:2] (reset 8) |
| 0x224 | DESCENG_ADDR0_BASE | RW | Descriptor address range 0 base [31:0] |
| 0x228 | DESCENG_ADDR0_LIMIT | RW | Descriptor address range 0 limit [31:0] |
| 0x22C | DESCENG_ADDR1_BASE | RW | Descriptor address range 1 base [31:0] |
| 0x230 | DESCENG_ADDR1_LIMIT | RW | Descriptor address range 1 limit [31:0] |
| 0x240 | CTRL_CONFIG | RW | CTRLRD_MAX_TRY[8:0] control-read poll retry budget (0-511, reset 16) |
| 0x2A0 | AXI_XFER_CONFIG | RW | AXI transfer sizing configuration |
| 0x2B0 | PERF_CONFIG | RW | Performance profiler configuration |
| 0x2C0-0x2CC | OBS_CTRL/OBS_FLAGS/OBS_DATA0/OBS_DATA1 | RW/RO | Observation mux |
| 0x35C | PERF_CH_SEL | RW | Performance channel select |
| 0x378-0x380 | HIST_SEL/HIST_DATA/HIST_TOTAL | RW/RO | Latency histogram |

## 22.4	Monitor Registers (+0x800 within each half)

Monitor Registers (+0x800 within each half; SRC at 0x0800, SNK at 0x1800)

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| +0x800 | MON_FIFO_STATUS | RO | MonBus capture/error FIFO status |
| +0x804 | MON_FIFO_COUNT | RO | MonBus FIFO occupancy |
| +0x8C0-0x8DC | DAXMON_* | RW | Descriptor-monitor config (enable/timeout/latency/masks) |
| +0x8E0-0x8FC | RDMON_* | RW | Source-egress AXIS monitor-lite config (same layout; live in the SRC half, rapids TASK-015) |
| +0x900-0x91C | WRMON_* | RW | Sink-ingress AXIS monitor-lite config (same layout; live in the SNK half, rapids TASK-015; the SRC copy’s COMPRESS_EN drives the group) |
| +0x950-0x978 | DAXMON_PERF_* | RO/RW | Descriptor-monitor performance counters |
| +0x980-0x9A8 | RDMON_PERF_* | RO/RW | Read-monitor performance counters |
| +0x9B0-0x9D8 | WRMON_PERF_* | RO/RW | Write-monitor performance counters |
| +0x9E0-0x9F4 | {RD,WR}MON_PERF_CH_* | RO | Per-channel producer/backpressure/starve/idle/overflow |

## 22.5	Key Register Fields

### 22.5.1	GLOBAL_CTRL (0x100) - Read/Write

GLOBAL_CTRL Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [0] | GLOBAL_EN | 0 | Master enable for the entire engine |
| [1] | GLOBAL_RST | 0 | Global reset: holds every channel of the half in reset while 1. Not self-clearing – write 0 to release |
| [31:2] | Reserved | 0 | Reserved |

### 22.5.2	SCHED_TIMEOUT_LIMIT (0x208) - Read/Write

SCHED_TIMEOUT_LIMIT Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [7:0] | LIMIT | 4 | Consecutive write-progress timeout windows before escalating to a fatal, sticky CH_ERROR. 0 = never escalate. Total escalation time ~= LIMIT x SCHED_TIMEOUT_CYCLES. |

### 22.5.3	DESCENG_CONFIG (0x220) - Read/Write

DESCENG_CONFIG Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [0] | DESCENG_EN | 1 | Descriptor engine master enable |
| [1] | PREFETCH_EN | 0 | Prefetch chaining (0 = on-demand, ~1 ahead) |
| [5:2] | FIFO_THRESH | 8 | Descriptors buffered ahead when prefetch enabled |
| [31:6] | Reserved | 0 | Reserved |

### 22.5.4	Tile Header Status (0x2D0-0x2D8) - byte-granular RAPIDS with TILE_HDR = 1

These registers exist in both halves and read zero in RAPIDS Beats and in a TILE_HDR = 0 build. The sticky bits are write-1-to-clear.

Tile Header Status Registers

| Register | Half | Fields |
| --- | --- | --- |
| TILE_STATUS (0x2D0) | SNK | UNSUPPORTED [0]: the sink port consumed a packet it could not act on. TAG_MISMATCH [1]: a data packet’s tag differed from its stream ID |
| TILE_DESC_DROPPED (0x2D4) | SRC | DROPn [n]: channel n dropped a cell-issued descriptor (no free slot, EXT, or during channel reset) |
| TILE_COUNTS (0x2D8) | SNK | DESC_COUNT [15:0]: desc packets passed to the source half. CREDIT_COUNT [31:16]: credit packets received. Both wrap |

A drop does not error the channel; TILE_DESC_DROPPED is the only record of it. See the compute-engine tile’s TILE_SPEC Section 11.12.

Register Access Timing

Waveform 32

## 22.6	Channel Kick Sequence

Staging a channel’s descriptor address, then writing that channel’s bit in KICK_ENABLE, triggers descriptor processing:

Waveform 33

## 22.7	Address Decoding

Registers are global (not per-channel windows); per-channel fields are packed as bit lanes within a single 32-bit register (e.g. CHANNEL_ENABLE.CH_EN[7:0], SCHEDULER_IDLE.SCHED_IDLE[7:0]). The only per-channel array is CH_STATE[0..7] at 0x150-0x16C (stride 0x4). Descriptor kick-off is part of this register block: staged addresses at 0x000-0x03F (CH*_DESC_ADDR_{LOW,HIGH}, stride 0x8) and the KICK_ENABLE launch register at 0x040.

+0x000 - 0x03F: Staged descriptor addresses (CH*_DESC_ADDR_{LOW,HIGH})
+0x040        : KICK_ENABLE (per-channel launch, singlepulse)
+0x100 - 0x3FF: Base configuration / status registers
+0x800 - 0x9FF: Monitor configuration / performance registers
SRC half at 0x0000, SNK half at 0x1000 (paddr[12]); PSLVERR on the monitor
window when USE_MON_REGS = 0.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Initialization Sequence

## 23.1	Overview

This section describes how software brings up RAPIDS Beats and starts a transfer. Every register named here is a register of the PeakRDL rapids_engine_regs register file (rtl/macro_beats/rapids_engine_regs.rdl), using the names in the generated per-instance map rapids_regmap.py. The two halves share one layout: SRC at 0x0000 and SNK at 0x1000. Each half is initialized on its own, and the offsets in the Register Map are relative to the half.

The code below uses reg_wr(half, NAME, value) and reg_rd(half, NAME) as shorthand for an APB access to register NAME in the SRC or SNK half. Resolve the names through the generated map instead of hard-coding offsets.

## 23.2	Boot Sequence

### 23.2.1	Figure 5.3.1: Bring-Up Sequence

Bring-Up Sequence

Source: ch05_boot_sequence.dot

## 23.3	What the Hardware Requires

A channel runs a descriptor only when every one of these terms is true. They are ANDed in rapids_config_block, so a missing one leaves the channel idle without raising an error:

Conditions for a Channel to Run

| Term | Register.field | Reset | Effect when 0 |
| --- | --- | --- | --- |
| Global enable | GLOBAL_CTRL.GLOBAL_EN | 0 | Masks CH_EN, SCHED_EN, DESCENG_EN and every monitor enable in the half |
| Channel enable | CHANNEL_ENABLE.CH_EN[n] | 0 | The scheduler stays in CH_IDLE with a descriptor waiting |
| Scheduler enable | SCHED_CONFIG.SCHED_EN | 1 | All schedulers in the half are disabled |
| Descriptor engine enable | DESCENG_CONFIG.DESCENG_EN | 1 | No descriptor is fetched |
| Descriptor window | DESCENG_ADDR0_BASE/_LIMIT, DESCENG_ADDR1_BASE/_LIMIT | 0 / 0xFFFFFFFF | A descriptor address outside both windows is refused and the channel errors |
| Kick | KICK_ENABLE.KICKn | – | Nothing is fetched until the bit is written |

GLOBAL_EN and CH_EN both reset to 0. The other enables reset to 1, and both descriptor windows reset to the full 32-bit range, so a minimal bring-up only has to write CHANNEL_ENABLE, GLOBAL_CTRL, the staged address and KICK_ENABLE.

## 23.4	Detailed Steps

### 23.4.1	Step 1: Verify Hardware Presence

// VERSION (0x108): MINOR[7:0], MAJOR[15:8], NUM_CHANNELS[23:16]
uint32_t version = reg_rd(SRC, VERSION);
uint8_t minor    = (version >>  0) & 0xFF;   // 0x5A (90) in this release
uint8_t major    = (version >>  8) & 0xFF;   // 0x00: pre-release, version 0.90
uint8_t num_ch   = (version >> 16) & 0xFF;   // 8

if (num_ch == 0) {
    printf("ERROR: RAPIDS not detected\n");
    return -1;
}

There is no separate configuration register. The channel count is the only build parameter that software can read back. Data width and SRAM depth are not readable from the register file.

### 23.4.2	Step 2: Optional Reset

GLOBAL_CTRL.GLOBAL_RST and CHANNEL_RESET.CH_RST[n] are ordinary read/write bits. They do not self-clear. A channel is held in reset (scheduler forced to CH_IDLE, descriptor engine reset) for as long as its bit, or GLOBAL_RST, reads 1. Software must write the bit back to 0 to release it.

// Pulse the global reset: set, then clear. Keep GLOBAL_EN at 0 meanwhile.
reg_wr(half, GLOBAL_CTRL, GLOBAL_CTRL_GLOBAL_RST);   // bit 1
reg_wr(half, GLOBAL_CTRL, 0);

// Every scheduler should now report idle (SYSTEM_IDLE = AND of SCHED_IDLE).
while (!(reg_rd(half, GLOBAL_STATUS) & 1)) { }

### 23.4.3	Step 3: Configure the Engine

// Scheduler: SCHED_EN[0] TIMEOUT_EN[1] ERR_EN[2] COMPL_EN[3] PERF_EN[4]
reg_wr(half, SCHED_TIMEOUT_CYCLES, 1000);   // write-progress window, in clocks
reg_wr(half, SCHED_TIMEOUT_LIMIT,  4);      // fatal after 4 stalled windows; 0 = never
reg_wr(half, SCHED_CONFIG, SCHED_EN | TIMEOUT_EN | ERR_EN | COMPL_EN);

// Descriptor engine: DESCENG_EN[0] PREFETCH_EN[1] FIFO_THRESH[5:2]
reg_wr(half, DESCENG_CONFIG, DESCENG_EN | PREFETCH_EN | (4 << 2));

// Descriptor address windows (lower 32 bits). A fetch address outside
// both windows is refused and the channel enters CH_ERROR.
reg_wr(half, DESCENG_ADDR0_BASE,  desc_region_base);
reg_wr(half, DESCENG_ADDR0_LIMIT, desc_region_base + desc_region_size - 1);

// Burst sizing: RD_XFER_BEATS[7:0] WR_XFER_BEATS[15:8]
//               ALLOC_SIZE[23:16]  DRAIN_SIZE[31:24]
reg_wr(half, AXI_XFER_CONFIG, (16 << 24) | (16 << 16) | (8 << 8) | 8);

The SRC half uses the read-side fields and the SNK half uses the write-side fields. Writing the whole word in both halves does no harm.

### 23.4.4	Step 4: Optional Monitor and Interrupt Egress

The only interrupt output is mon_irq. It comes from the top-level monbus_axil4_axil4_group and is asserted while that group’s error FIFO is not empty. The scheduler error and completion packets travel on the CORE protocol. For both halves, the group takes its CORE filter from the SRC half’s WRMON_* registers:

// CORE packets: PKT_MASK bit[type] = 1 DROPS that type; reset is 0xFFFF.
reg_wr(SRC, WRMON_PKT_MASK, ~((1u << 0) | (1u << 1)) & 0xFFFF);  // pass Error, Completion

// ERR_SELECT[3:0] bit[type] = 1 routes that type to the error FIFO (-> mon_irq).
// Bit 0 = Error.  Bit 1 = Completion, which includes the gen_irq descriptor event.
reg_wr(SRC, WRMON_ERR_CFG, (0xFF << 8) | (1u << 0));             // keep ERR_MASK reset value

Packets that are passed but not selected for the error FIFO go to the capture path (m_axil_mon_*). The group, mon_irq and the s_axil_err_* drain exist only when USE_AXI_MONITORS = 1. With USE_AXI_MONITORS = 0, mon_irq is tied to 0, and software must poll SCHED_ERROR and SCHEDULER_IDLE instead.

### 23.4.5	Step 5: Build Descriptors

// rapids_pkg::descriptor_t, 256 bits (see Descriptor Format)
for (int i = 0; i < NUM_RX_DESC; i++) {
    desc[i].src_addr            = 0;                        // unused by a sink transfer
    desc[i].dst_addr            = rx_buffer + i * FRAME_SIZE;
    desc[i].length              = FRAME_LEN;   // BEATS in RAPIDS Beats, BYTES in byte-granular RAPIDS
    desc[i].next_descriptor_ptr = (i < NUM_RX_DESC - 1) ? (uint32_t)&desc[i + 1] : 0;
    desc[i].valid               = 1;                        // 0 -> CH_ERROR
    desc[i].last                = (i == NUM_RX_DESC - 1);
    desc[i].gen_irq             = 1;                        // completion event RAPIDS_EVENT_IRQ
}
__sync_synchronize();   // descriptors visible before the kick

The descriptors must lie inside a DESCENG_ADDR0/DESCENG_ADDR1 window.

### 23.4.6	Step 6: Enable

reg_wr(half, CHANNEL_ENABLE, 1u << ch);          // CH_EN[n]
reg_wr(half, GLOBAL_CTRL,    GLOBAL_CTRL_GLOBAL_EN);   // bit 0; GLOBAL_RST stays 0

Initialization Timing

cfg_channel_enable[n] is CH_EN[n] & GLOBAL_EN, so it asserts only after the last of the two writes.

## 23.5	Starting Transfers

### 23.5.1	Kick a Channel

Stage the descriptor address, then launch it with a single write to KICK_ENABLE. The address registers only store the value, so writing them does not start anything. Each KICKn bit is a singlepulse field and clears itself. One 32-bit write can launch several channels on the same clock.

reg_wr(half, CH0_DESC_ADDR_LOW,  (uint32_t)((uint64_t)&desc[0]));
reg_wr(half, CH0_DESC_ADDR_HIGH, (uint32_t)((uint64_t)&desc[0] >> 32));
reg_wr(half, KICK_ENABLE, 1u << 0);              // KICK0

The APB write completes as soon as the request is raised. The RTL holds the request until the descriptor engine accepts it, so a completed write does not mean the engine has accepted the kick.

Waveform 35

## 23.6	Completion and Interrupt Handling

There is no interrupt status register and no per-channel interrupt clear. Software sees completion in one of two ways:

- Polling. SCHEDULER_IDLE.SCHED_IDLE[n] returns to 1 when the scheduler is back in CH_IDLE after the last descriptor of a chain. A channel in CH_ERROR never reports idle, and SCHED_ERROR.SCHED_ERR[n] reads 1 for it instead. Poll both registers, or the loop will hang on a faulted channel. Do not poll CHANNEL_IDLE, AXI_WR_COMPLETE, or the SNK half’s AXI_RD_COMPLETE. Those fields are not driven at the top level and always read 0 (see the Register Map).

- mon_irq. If Step 4 routed Completion packets to the error FIFO, every descriptor completion lands there, and so does every error. A completion is reported as RAPIDS_EVENT_DESC_COMPLETE, or as RAPIDS_EVENT_IRQ when the descriptor’s gen_irq is set. The descriptor engine’s descriptor-loaded completions land there too. mon_irq stays high until the records are drained over s_axil_err_*.

void rapids_irq_handler(void) {
    // mon_irq == error FIFO not empty: drain every record
    while (mon_irq_pending()) {
        mon_record_t r = err_fifo_read();        // s_axil_err_* reads
        if (r.pkt_type == PKT_TYPE_ERROR) {
            handle_channel_error(r.half, r.channel_id);   // see Error Handling
        } else if (r.pkt_type == PKT_TYPE_COMPLETION) {
            process_completed_buffer(r.half, r.channel_id, r.event_data);
        }
    }
}

To run the channel again, stage the next chain’s first descriptor address and write its KICK_ENABLE bit. No re-enable is needed, because CH_EN stays set.

## 23.7	Reset and Recovery

### 23.7.1	Per-Channel Reset

void reset_channel(int half, int ch) {
    // Assert, then release: CH_RST is NOT self-clearing.
    reg_wr(half, CHANNEL_RESET, 1u << ch);
    reg_wr(half, CHANNEL_RESET, 0);

    while (!(reg_rd(half, SCHEDULER_IDLE) & (1u << ch))) { }
    // SCHED_ERROR[ch] is the CH_ERROR state, so it reads 0 again here.
}

The scheduler’s error state is sticky. The only ways out of CH_ERROR are a channel reset (CH_RST[n]) or a global reset (GLOBAL_RST). The reset leaves CH_EN[n] and the staged descriptor address unchanged. After the reset, re-stage the address if it should change, then kick.

### 23.7.2	Global Reset

void reset_rapids_half(int half) {
    reg_wr(half, GLOBAL_CTRL, GLOBAL_CTRL_GLOBAL_RST);   // GLOBAL_EN = 0, RST = 1
    reg_wr(half, GLOBAL_CTRL, 0);                        // release
    while (!(reg_rd(half, GLOBAL_STATUS) & 1)) { }       // SYSTEM_IDLE
    rapids_init_half(half);                              // Steps 3-6 again
}

GLOBAL_RST drives every channel’s reset in that half only. The other half, and the shared monitor group’s error FIFO, are not affected.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Error Handling

## 24.1	Overview

This section covers how RAPIDS Beats detects an error, how it reports one, and how software recovers. Every fatal error in a channel ends the same way. The channel’s scheduler latches it and moves to the sticky CH_ERROR state, and it stays there until software resets the channel. No per-channel error-status register holds an error code. Software has three sources of information: the SCHED_ERROR bit, the CH_STATE readback, and the MonBus error packet.

## 24.2	Error Categories

### 24.2.1	Figure 5.4.1: Error Sources

Error Sources

Source: ch05_error_sources.dot

## 24.3	Error Detection

### 24.3.1	AXI Response Errors

RAPIDS checks the response on every AXI transaction it issues: descriptor fetches, source-half data reads, sink-half data writes, and the control read/write engines.

AXI Response Codes

| Response | Code | Description | Typical Cause |
| --- | --- | --- | --- |
| OKAY | 2’b00 | Normal success | – |
| EXOKAY | 2’b01 | Exclusive OK | – |
| SLVERR | 2’b10 | Slave error | Memory protection, invalid access |
| DECERR | 2’b11 | Decode error | Address not mapped |

Waveform 36

### 24.3.2	Descriptor Errors

The descriptor engine and the scheduler apply a small, fixed set of checks. None of them has its own status code. Each one sets the channel’s SCHED_ERROR bit, and the descriptor-engine checks also emit a MonBus error packet.

Descriptor Checks

| Check | Where | Error condition |
| --- | --- | --- |
| Address window | Descriptor engine | The fetch address is outside both DESCENG_ADDR0_BASE..DESCENG_ADDR0_LIMIT and DESCENG_ADDR1_BASE..DESCENG_ADDR1_LIMIT. The fetch is refused and never issued. |
| Null pointer | Descriptor engine | The descriptor address is 0 |
| Fetch response | Descriptor engine | RRESP is not OKAY on the descriptor read |
| Valid bit | Scheduler (CH_FETCH_DESC) | The descriptor’s valid bit is 0 |

The hardware does not check length, alignment, or direction. A descriptor with a zero or oversized length is software’s responsibility. The software validation example at the end of this section shows one way to catch it.

Waveform 37

### 24.3.3	Timeout Detection

Only the scheduler’s write-progress watchdog can put a channel into CH_ERROR by itself. The control-read retry budget has a similar effect. The AXI monitors’ timeouts only report.

Timeout Mechanisms

| Mechanism | Register | Reset value | Result |
| --- | --- | --- | --- |
| Write-progress watchdog | SCHED_TIMEOUT_CYCLES.TIMEOUT_CYCLES (clocks per window), SCHED_TIMEOUT_LIMIT.LIMIT (windows) | 1000, 4 | Fatal CH_ERROR after LIMIT consecutive stalled windows. With LIMIT = 0 it never escalates. Enabled by SCHED_CONFIG.TIMEOUT_EN. |
| Control-read poll budget | CTRL_CONFIG.CTRLRD_MAX_TRY | 16 | A CTRL_READ descriptor that does not match within the budget raises ctrlrd_error, and the channel goes to CH_ERROR |
| Monitor transaction timeout | DAXMON_TIMEOUT, RDMON_TIMEOUT, WRMON_TIMEOUT | – | Emits a MonBus Timeout packet when the monitor’s TIMEOUT_EN is set. The channel state does not change. |

Recoverable write-progress timeout. Each window of SCHED_TIMEOUT_CYCLES clocks that expires without write progress adds one strike to a counter. The channel goes to the fatal, sticky CH_ERROR only after SCHED_TIMEOUT_LIMIT (register 0x208) consecutive stalled windows. Any write progress, either an AW issue or a B-response commit, clears the strike counter. A limit of 0 disables escalation entirely, which makes the timeout soft. A channel in CH_ERROR is reported through SCHED_ERROR and is deliberately left out of SCHEDULER_IDLE.

Waveform 38

## 24.4	Error Reporting

### 24.4.1	Status Registers

Error-Related Status Registers

| Register | Meaning on error |
| --- | --- |
| SCHED_ERROR.SCHED_ERR[n] | 1 while channel n is in CH_ERROR. The bit reads the state directly and clears only on channel reset. |
| CH_STATE[n].STATE | One-hot scheduler state. CH_ERROR = 7’b0100000. |
| SCHEDULER_IDLE.SCHED_IDLE[n] | 0 for a faulted channel, because CH_ERROR is not idle |
| GLOBAL_STATUS.SYSTEM_IDLE | 0 while any channel in the half is faulted |

The register file does not record which response code or which descriptor caused the error. That detail travels only in the MonBus error packet.

### 24.4.2	MonBus Error Events

Each error episode emits one error packet (PktTypeError, CORE protocol). The packet carries the half’s agent ID and the channel ID:

MonBus Error Packets

| Emitter | Event code | Event data |
| --- | --- | --- |
| Scheduler (on entering CH_ERROR) | RAPIDS_EVENT_ERROR (0x0F) | Read-side and write-side sticky error flags. Both clear means the cause was the descriptor engine, a control engine, or the timeout. |
| Descriptor engine | CORE_ERR_DESCRIPTOR_ENGINE | The descriptor fetch RRESP |

SCHED_CONFIG.ERR_EN and COMPL_EN do not gate error packets. The scheduler re-arms its one-shot error packet when the channel returns to CH_IDLE.

Waveform 39

### 24.4.3	Interrupt Generation

There is no separate interrupt status register. A channel error sets that channel’s sticky SCHED_ERROR bit and emits one MonBus error packet. The top-level monitor group routes the packet types that its err_select configuration selects into its error FIFO. For CORE packets that configuration is SRC.MON.WRMON_ERR_CFG.ERR_SELECT, and bit 0 selects Error. SRC.MON.WRMON_PKT_MASK bit 0 must also be clear, because the reset value masks every type. mon_irq is asserted while the error FIFO is not empty. Software reads the records out over s_axil_err:

### 24.4.4	Figure 5.4.2: Error Reporting

Error Reporting

Source: ch05_error_reporting.dot

## 24.5	Error Recovery

### 24.5.1	Recovery Flow

### 24.5.2	Figure 5.4.3: Error Recovery

Error Recovery

Source: ch05_error_recovery.dot

Every recovery ends the same way. Fix the cause, then reset the channel. Set its CHANNEL_RESET bit and clear it again, because the bit is not self-clearing. Re-stage the descriptor address and kick. reset_channel() is defined in Initialization.

### 24.5.3	Finding the Faulted Channel

void handle_errors(int half) {
    uint32_t faulted = reg_rd(half, SCHED_ERROR) & 0xFF;   // SCHED_ERR[7:0]

    for (int ch = 0; ch < 8; ch++) {
        if (!(faulted & (1u << ch)))
            continue;
        mon_record_t r = find_error_record(half, ch);   // from the s_axil_err drain
        recover_channel(half, ch, &r);
    }
}

### 24.5.4	AXI Error Recovery

void recover_axi_error(int half, int ch, const mon_record_t *r) {
    // The scheduler packet says which side failed; the address is not latched.
    if (r->rd_err)
        printf("%s ch%d: read-side AXI error\n", half_name(half), ch);
    if (r->wr_err)
        printf("%s ch%d: write-side AXI error\n", half_name(half), ch);
    // Check the address map and the target slave, then:
    reset_channel(half, ch);
    restage_and_kick(half, ch, first_desc_addr[half][ch]);
}

### 24.5.5	Descriptor Error Recovery

void recover_desc_error(int half, int ch, uint64_t bad_desc) {
    // Window refusal, null pointer, fetch RRESP error, or valid bit clear.
    // Fix the descriptor (or widen DESCENG_ADDR0/1) before re-kicking.
    fix_descriptor(bad_desc);
    reset_channel(half, ch);
    restage_and_kick(half, ch, bad_desc);
}

void restage_and_kick(int half, int ch, uint64_t addr) {
    reg_wr(half, CH_DESC_ADDR_LOW(ch),  (uint32_t)addr);          // CHn_DESC_ADDR_LOW
    reg_wr(half, CH_DESC_ADDR_HIGH(ch), (uint32_t)(addr >> 32));  // CHn_DESC_ADDR_HIGH
    reg_wr(half, KICK_ENABLE, 1u << ch);                          // KICKn
}

### 24.5.6	Timeout Recovery

void recover_timeout(int half, int ch) {
    // Escalated write-progress timeout: the write side stopped making progress
    // for SCHED_TIMEOUT_LIMIT windows. In the SNK half that usually means the
    // AXIS peer stopped sending or memory stopped accepting writes. In the SRC
    // half, check m_axis_tready.
    printf("%s ch%d: write-progress timeout\n", half_name(half), ch);
    reset_channel(half, ch);
    restage_and_kick(half, ch, first_desc_addr[half][ch]);
}

To ride out long stalls without a fatal error, raise SCHED_TIMEOUT_CYCLES or SCHED_TIMEOUT_LIMIT, or set the limit to 0. The timeout is then reported but never escalates.

## 24.6	Error Prevention

### 24.6.1	Best Practices

Error Prevention Best Practices

| Practice | Rationale |
| --- | --- |
| Validate descriptors before submission | The hardware checks only the window, a null pointer, and the valid bit |
| Set DESCENG_ADDR0/1 to the real descriptor region | A stray next_descriptor_ptr is refused rather than fetched |
| Size SCHED_TIMEOUT_CYCLES x SCHED_TIMEOUT_LIMIT above the worst legitimate stall | Avoids false fatal timeouts |
| Route Error packets to the error FIFO | mon_irq reports errors without polling |

### 24.6.2	Descriptor Validation Example

int validate_descriptor(const rapids_descriptor_t *desc) {
    if (!desc->valid)
        return ERR_NOT_VALID;            // the scheduler would fault the channel
    if (desc->length == 0 || desc->length > MAX_BEATS)
        return ERR_LENGTH;               // not checked by hardware
    if (desc->next_descriptor_ptr &&
        !in_desc_window(desc->next_descriptor_ptr))
        return ERR_ADDRESS;              // the descriptor engine would refuse it
    return 0;
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Throughput Targets

RAPIDS Beats is a network-to-memory accelerator: the sink path moves an AXIS ingress stream into system memory, and the source path reads system memory and drives an AXIS egress stream. The two paths are independent engines (with their own SRAM buffers, schedulers, and AXI masters), so a sink transfer and a source transfer run concurrently and their bandwidths add.

## 25.1	Theoretical Maximum

A beat is DATA_WIDTH = 512 bits = 64 bytes. Per-direction bandwidth is DATA_WIDTH x f_aclk / 8.

### 25.1.1	Per-Interface Bandwidth

| Interface | Data Width | Frequency | Bandwidth |
| --- | --- | --- | --- |
| Descriptor Fetch | 256 bits | 100 MHz | 3.2 GB/s |
| Sink AXI Write (to memory) | 512 bits | 100 MHz | 6.4 GB/s |
| Source AXI Read (from memory) | 512 bits | 100 MHz | 6.4 GB/s |
| AXIS ingress / egress (network) | 512 bits/beat | 100 MHz | 6.4 GB/s |

The table uses the Nexys A7 characterization clock (CLK100MHZ, 100 MHz). The design targets 100-500 MHz; scale linearly (e.g. 12.8 GB/s per direction at 200 MHz, 32 GB/s at 500 MHz).

### 25.1.2	Aggregate Bandwidth

- Concurrent sink + source: 12.8 GB/s at 100 MHz (6.4 sink + 6.4 source), since the two directions are separate engines against separate AXI masters.

- Descriptor Overhead: ~0.1% for large transfers; the descriptor engine prefetches (DESCENG_CONFIG.PREFETCH_EN) so chained fetches overlap the current transfer.

## 25.2	Practical Throughput

A transfer is bounded by the slower of its two ends: a sink transfer by min(AXIS ingress, AXI write), a source transfer by min(AXI read, AXIS egress). The SRAM buffer decouples the two ends so a transient stall on one side does not immediately stall the other.

### 25.2.1	Single-Channel Performance

| Scenario | Expected Efficiency | Effective Bandwidth (100 MHz) |
| --- | --- | --- |
| Large sequential (>1MB) | >95% | >6.1 GB/s per direction |
| Medium sequential (64KB-1MB) | 85-95% | 5.4-6.1 GB/s |
| Small sequential (4KB-64KB) | 70-85% | 4.5-5.4 GB/s |
| Very small (<4KB) | 40-70% | 2.6-4.5 GB/s |

### 25.2.2	Multi-Channel Performance

All channels of one direction share that direction’s AXI master, so aggregate is bounded by the shared master while per-channel throughput scales inversely with active channel count.

| Active Channels (per direction) | Per-Channel BW | Aggregate BW (per direction) |
| --- | --- | --- |
| 1 | 6.4 GB/s | 6.4 GB/s |
| 2 | 3.2 GB/s | 6.4 GB/s |
| 4 | 1.6 GB/s | 6.4 GB/s |
| 8 | 0.8 GB/s | 6.4 GB/s |

Note: figures are at 100 MHz for a single direction. Sink and source run concurrently, so full-duplex aggregate is twice the per-direction column.

## 25.3	Throughput Limiting Factors

### 25.3.1	Memory / Network System Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Memory latency | Reduces efficiency for small transfers | Deeper outstanding / larger SRAM |
| Memory or network bandwidth | Hard limit on the slower end | Match RAPIDS to the memory/network capability |
| AXIS backpressure | Stalls the network end (s_axis_tready/m_axis_tready) | SRAM buffering absorbs transient stalls |
| Interconnect contention | Variable impact | Priority / channel configuration |

### 25.3.2	RAPIDS Internal Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Descriptor fetch overhead | Fixed per descriptor | Longer transfers; descriptor prefetch (PREFETCH_EN) |
| SRAM depth | Limits outstanding beats | Configure adequate SRAM_DEPTH |
| Arbitration overhead | Multi-channel penalty on the shared AXI master | Reduce active channels per direction |
| AXI transfer sizing | Small bursts lose efficiency | AXI_XFER_CONFIG RD/WR beats, ALLOC/DRAIN sizing |
| Control descriptors | CTRL_READ/CTRL_WRITE are synchronization, not payload | Use only for producer/consumer handshakes; see below |

Control descriptors do not carry throughput. A CTRL_READ (consumer gate) polls a memory location until it matches; a CTRL_WRITE (producer doorbell) issues one 32-bit write. They synchronize a channel with a producer/consumer and occupy the channel while active, but move no payload – budget them as latency, not bandwidth (see Latency, Section 6.2).

## 25.4	Performance Targets

### 25.4.1	Design Targets

| Metric | Target | Condition |
| --- | --- | --- |
| Peak throughput (per direction) | 6.4 GB/s | Single channel, 100 MHz, ideal |
| Full-duplex aggregate | 12.8 GB/s | Sink + source concurrent, 100 MHz |
| Sustained throughput | >6.1 GB/s | Large transfers, single channel |
| Small transfer efficiency | >50% | 4KB transfers |

### 25.4.2	Verification Criteria

| Scenario | Pass Criteria |
| --- | --- |
| Sink stream (large) | >95% of AXI write bandwidth |
| Source stream (large) | >95% of AXI read bandwidth |
| Multi-channel (per direction) | Aggregate within 5% of the shared master limit |
| Data integrity (all cases) | wr_crc/rd_crc/chk_crc == golden model |

## 25.5	On-Silicon Validation

The characterization harness (rapids_char_top on the Nexys A7-100T, 100 MHz) validates data integrity end to end via golden-CRC self-checks on both paths: sink wr_crc == golden, source rd_crc == chk_crc == golden. The current harness measures correctness and beat counts.

Per-direction bus utilization is measured IN THE DESIGN, not by an external observer. rapids_beats_top instantiates the shared axi_bus_meter (rtl/amba/shared/) twice – u_rd_bus_meter on the read master and u_wr_bus_meter on the write master – giving the true per-direction split (STREAM’s shared master is aggregate-only). Aggregate PROD/BP/STARV/IDLE plus beat/byte/burst counts surface through the RDMON_PERF_* / WRMON_PERF_* CSRs, and per-channel buckets through SRC.PERF_CH_SEL / SNK.PERF_CH_SEL with RDMON_PERF_CH_* / WRMON_PERF_CH_*.

Adopting axi4_intf_master_observer here was considered and rejected (RAPIDS TASK-001, 2026-09-24). The observer is a packaging of that same axi_bus_meter plus a latency histogram and monbus taps; since the meters are already the shared primitive, it buys no measurement RAPIDS lacks while costing +208% LUTs at its smallest legal config – the same measured ground on which pumice rejected it (PUMICE-016). The one thing it would add is axi_perf_latency_hist, which RAPIDS does not instantiate; that is a separate question from this one.

As of 2026-08-05 the observer carries its OWN APB config regblock (obs_regs) instead of exporting 29 cfg_* ports for the instantiating harness to tie off, and it moved to projects/components/utility-ip/misc/rtl/ so any board flow can reach it:

-f $MISC_ROOT/rtl/filelists/axi4_intf_master_observer.f

It was renamed from axi4_dma_observer at the same time – the header always said “DMA-agnostic”, and the DMA in the name read wrong for a block a memory controller or a beats DMA would share. dma_slave_monitors is retired: module, filelist and its slvmon_regs regblock were deleted 2026-09-20 (STREAM TASK-073). The slave-side equivalent is axi4_intf_slave_observer + obs_regs.

Latest on-silicon result (full characterization suite): 48 / 48 configurations pass across channels {1, 2, 4} x beats {1, 4, 8, 16} x backpressure {off, on} x seeds {default, alternate}, sink and source, all golden-validated – including under injected AXIS backpressure.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Latency Characteristics

Latency is dominated by the one-time pipeline fill from a channel kick to the first data beat. Once running, both paths sustain the throughput of Section 6.1; control descriptors add a bounded synchronization delay.

## 26.1	Latency Components

### 26.1.1	Kick-to-First-Beat Breakdown

| Stage | Typical Cycles (100 MHz) | Notes |
| --- | --- | --- |
| Kick decode (APB write to KICK_ENABLE) | TBD – re-measure | Address is staged beforehand; the launch request is then held until the descriptor engine accepts it |
| Descriptor fetch (AXI read, 256-bit) | AXI read latency + 1 | Overlapped by prefetch on chains |
| Descriptor parse | 1-2 | Opcode + fields decoded |
| SRAM fill (sink) / AXI read issue (source) | AXI/AXIS latency | First beat into the buffer |
| First beat out | 1-2 | Drain to memory (sink) / AXIS (source) |

### 26.1.2	Total Latency Formula

L_first_beat  =  L_kick + L_desc_fetch + L_parse + L_pipeline_fill

Note: L_kick predates the staged-address/KICK_ENABLE kick and has not been re-measured. The previous figure came from a path that stalled APB until the descriptor engine accepted the kick; acceptance is now asynchronous, so the term is no longer bounded by an APB stall.

L_desc_fetch is hidden on chained descriptors when the descriptor engine has prefetched the next descriptor (DESCENG_CONFIG.PREFETCH_EN).

## 26.2	First-Beat Latency

The dominant term is the memory/network round trip to fill the pipeline. For a sink transfer the first AXIS beat must arrive and be buffered before the first AXI write can drain; for a source transfer the first AXI read must return before the first AXIS egress beat. After the first beat the pipeline is full and throughput is bandwidth-bound, not latency-bound.

## 26.3	Control-Descriptor Latency

Control descriptors are latency, not throughput (Section 6.1):

| Opcode | Latency |
| --- | --- |
| CTRL_WRITE (doorbell) | One single-beat AXI write: AW->W->B round trip |
| CTRL_READ (gate) | One poll per retry (AXI read + compare); retries paced by tick_1us, up to CTRL_CONFIG.CTRLRD_MAX_TRY (default 16, max 511) |

A CTRL_READ gate blocks its descriptor (and the chain) until the polled value matches. The retry budget bounds the worst case: an unsatisfied gate cannot hang the channel – after CTRLRD_MAX_TRY polls the engine raises ctrlrd_error instead of waiting forever.

## 26.4	Latency Variability

### 26.4.1	Sources of Variability

- Memory read latency (descriptor fetch, source reads) and write-response timing.

- AXIS backpressure (s_axis_tready / m_axis_tready) on the network end.

- Multi-channel arbitration for the shared AXI master.

- CTRL_READ retry count when a gate is not yet satisfied.

### 26.4.2	Worst-Case Latency

Bounded by: memory timeout (SCHED_TIMEOUT_CYCLES / SCHED_TIMEOUT_LIMIT escalation on the scheduler side) and the control-read retry budget (CTRL_CONFIG.CTRLRD_MAX_TRY). Both convert an otherwise-unbounded wait into a reported error so a channel is never permanently stalled.

## 26.5	Latency Optimization

### 26.5.1	Software Recommendations

- Chain descriptors so the descriptor engine prefetches the next while the current transfers (hides L_desc_fetch).

- Keep the AXIS consumer/producer ready to avoid backpressure stalls.

- Size CTRL_READ gates so the expected value is already (or soon) present to minimize poll retries.

### 26.5.2	Hardware Configuration

- Adequate SRAM_DEPTH so the buffer covers memory/network latency without stalling.

- Tune AXI_XFER_CONFIG (RD/WR beats, ALLOC/DRAIN) so bursts amortize AXI setup.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Resource Estimates

Unlike the theoretical throughput and latency numbers above, the resource figures here are measured from the actual place-and-route of the characterization build (rapids_char_harness wrapping rapids_beats_top) on the Nexys A7-100T.

## 27.1	FPGA Resource Summary

Measured, post-implementation (Vivado 2025.1, xc7a100t-csg324-1, 100 MHz, characterization build: NUM_CHANNELS = 4, DATA_WIDTH = 512, ADDR_WIDTH = 64, APB_ADDR_WIDTH = 13):

| Resource | Used | Available | Utilization |
| --- | --- | --- | --- |
| Slice LUTs | 37,555 | 63,400 | 59.24% |
| – LUT as Logic | 30,205 | 63,400 | 47.64% |
| – LUT as Memory | 7,350 | 19,000 | 38.68% |
| Slice Registers (FF) | 28,683 | 126,800 | 22.62% |
| Block RAM Tile | 22 | 135 | 16.30% |
| – RAMB36 | 20 | 135 | – |
| – RAMB18 | 4 | 270 | – |
| DSP48E1 | 0 | 240 | 0% |

Timing: all user-specified timing constraints met at 100 MHz (positive slack) – the harness plus DUT close cleanly on the -1 speed grade part.

The characterization build uses 4 channels (sink + source) to fit the harness (BRAM SRAM buffers, golden-CRC checkers, UART bridge) comfortably on the 100T. The design parameter default is NUM_CHANNELS = 8; an 8-channel build roughly scales the per-channel SRAM/scheduler logic and would target a larger part.

## 27.2	Resource Breakdown by Block

Approximate distribution of the DUT logic (excludes the char harness’s UART/CRC scaffolding). Both a sink and a source instance are present; they do not share logic.

### 27.2.1	APB Configuration Slave + Register Block

- LUTs: ~1.5K

- FFs: ~2K

- Function: APB decode, PeakRDL rapids_regs block (config + monitor regfile @ 0x1000), rapids_config_block hwif fan-out.

### 27.2.2	Scheduler Group (per direction)

- LUTs: ~3-4K per direction

- FFs: ~3K per direction

- Function: scheduler_beats (three-opcode: DATA / CTRL_READ / CTRL_WRITE), descriptor_engine_beats (256-bit fetch/parse + prefetch), channel arbitration.

### 27.2.3	Control Engines (per direction)

- LUTs: ~0.5K per direction

- FFs: ~0.5K per direction

- Function: ctrlrd_engine (poll-until-match, retry budget) and ctrlwr_engine (single-beat doorbell write). Small – one AXI read/write state machine each.

### 27.2.4	AXI Read / Write Engines

- LUTs: ~2-3K each

- FFs: ~2-3K each

- Function: 512-bit streaming AXI masters (source read, sink write) with beat-level completion.

### 27.2.5	SRAM Buffers (sink + source)

- BRAM: dominant BRAM consumer (20x RAMB36 + 4x RAMB18)

- Function: per-channel sink and source data buffers decoupling network and memory ends. BRAM scales with SRAM_DEPTH x NUM_CHANNELS x 2 directions.

### 27.2.6	MonBus / AXI Monitors (USE_AXI_MONITORS)

- LUTs: ~1-2K

- FFs: ~1-2K

- Function: rd/wr AXI monitors feeding the MonBus AXI-Lite group (error-drain + capture master), monitor perf windows @ 0x1000.

## 27.3	SRAM Sizing

BRAM is the primary scaling resource. Each direction has its own per-channel buffer, so total BRAM ~= 2 x NUM_CHANNELS x ceil(SRAM_DEPTH x DATA_WIDTH / BRAM_bits). For the 4-channel char build this lands at 22 BRAM tiles (16.30% of the 100T). Deeper buffers improve small-transfer efficiency (Section 6.1) at the cost of BRAM; wider NUM_CHANNELS multiplies both scheduler logic and BRAM.

## 27.4	Scaling Notes

| Change | LUT/FF impact | BRAM impact |
| --- | --- | --- |
| NUM_CHANNELS 4 -> 8 | ~+scheduler/arbiter per channel | ~2x buffer tiles |
| SRAM_DEPTH deeper | minimal | +tiles per direction |
| USE_AXI_MONITORS off | -1-2K LUT/FF | none |
| DATA_WIDTH 512 -> 256 | ~half datapath LUT/FF | ~half buffer width |

DSP usage is zero – RAPIDS Beats is pure control + datapath movement, no arithmetic multiply/accumulate.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	RAPIDS Beats Architecture MAS

Version: 0.13 Date: 2026-10-20 Purpose: Module Architecture Specification for RAPIDS Phase 1 “Beats” Architecture

## 28.1	Document Organization

Note: All chapters linked below for automated document generation.

### 28.1.1	Front Matter

- Document Information

### 28.1.2	Chapter 1: Overview

- Architecture Overview

- Top-Level Port List

- Clocks and Reset

### 28.1.3	Chapter 2: FUB (Functional Unit Blocks)

Control Path: - Scheduler - Descriptor Engine

AXI Engines: - AXI Read Engine - AXI Write Engine

Flow Control: - Beats Alloc Control - Beats Drain Control - Beats Latency Bridge

Control Engines: - Control-Read Engine - Control-Write Engine

### 28.1.4	Chapter 3: Macro (Integration Blocks)

Scheduler Integration: - Beats Scheduler Group - Beats Scheduler Group Array

Sink Data Path (Network to Memory): - Sink Data Path - Sink Data Path AXIS - Sink SRAM Controller

Source Data Path (Memory to Network): - Source Data Path - Source Data Path AXIS - Source SRAM Controller

Top-Level Integration: - RAPIDS Core Beats - RAPIDS Registers - RAPIDS Config Block - RAPIDS Beats Top - Network Planes

### 28.1.5	Chapter 4: Interfaces

- Interface Overview

- AXI4 Interface Specification

- AXIS Interface Specification

- MonBus Interface Specification

- CXS Interface Specification

## 28.2	Quick Reference

### 28.2.1	FUB Modules (fub_beats/)

FUB Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler_beats | scheduler_beats.sv | Transfer coordinator, descriptor processing | Implemented |
| descriptor_engine_beats | descriptor_engine_beats.sv | Descriptor fetch/parse (256-bit) | Implemented |
| axi_read_engine_beats | axi_read_engine_beats.sv | AXI read master, streaming pipeline | Implemented |
| axi_write_engine_beats | axi_write_engine_beats.sv | AXI write master, streaming pipeline | Implemented |
| alloc_ctrl_beats | alloc_ctrl_beats.sv | Space allocation tracking (virtual FIFO); kept and tested, no longer in the SRAM path since bdf4e0dff | Implemented |
| drain_ctrl_beats | drain_ctrl_beats.sv | Data availability tracking (virtual FIFO); kept and tested, no longer in the SRAM path | Implemented |
| latency_bridge_beats | latency_bridge_beats.sv | Registered-FIFO to valid/ready skid bridge; kept and tested, no longer in the SRAM path | Implemented |
| ctrlrd_engine | ctrlrd_engine.sv | Control-read consumer gate (poll-until-match, retry budget) | Implemented |
| ctrlwr_engine | ctrlwr_engine.sv | Control-write producer doorbell (single-beat write) | Implemented |

### 28.2.2	Macro Modules (macro_beats/)

Macro Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler_group_beats | scheduler_group_beats.sv | Scheduler + Descriptor Engine wrapper | Implemented |
| scheduler_group_array_beats | scheduler_group_array_beats.sv | 8-channel scheduler array with arbitration | Implemented |
| snk_data_path_beats | snk_data_path_beats.sv | Network-to-memory path integration | Implemented |
| snk_data_path_axis_beats | snk_data_path_axis_beats.sv | AXIS variant of sink path | Implemented |
| src_data_path_beats | src_data_path_beats.sv | Memory-to-network path integration | Implemented |
| src_data_path_axis_beats | src_data_path_axis_beats.sv | AXIS variant of source path | Implemented |
| snk_sram_controller_beats | snk_sram_controller_beats.sv | Sink SRAM: naming wrapper around STREAM’s sram_controller (one instance, per-channel FIFOs inside it) | Implemented |
| src_sram_controller_beats | src_sram_controller_beats.sv | Source SRAM: naming wrapper around STREAM’s sram_controller | Implemented |
| rapids_src_beats / rapids_snk_beats | rapids_src_beats.sv, rapids_snk_beats.sv | One half each: scheduler array + data path + monbus merge | Implemented |
| rapids_core_beats | rapids_core_beats.sv | Both halves + the core monbus arbiter | Implemented |
| rapids_regs | regs/generated/rtl/rapids_regs.sv (generated from rtl/macro_beats/rapids_regs.rdl) | PeakRDL register block (base + MON regfile @ 0x1000) | Implemented |
| rapids_config_block | rapids_config_block.sv | Maps register hwif_out to core/monitor cfg_* | Implemented |
| rapids_beats_top | rapids_beats_top.sv | Top-level: APB slave, kickoff, monitors, MonBus AXI-Lite group | Implemented |

## 28.3	Architecture Comparison: RAPIDS vs STREAM

The “beats” architecture is a Phase 1 implementation of RAPIDS that shares concepts with STREAM but targets network-to-memory (and vice versa) transfers rather than memory-to-memory.

RAPIDS Beats vs STREAM Comparison

| Feature | STREAM | RAPIDS Beats |
| --- | --- | --- |
| Primary Use | Memory-to-memory DMA | Network-to-memory accelerator |
| Data Paths | Single bidirectional | Separate sink/source paths |
| Network Interface | None | AXIS master/slave |
| SRAM Buffering | Shared buffer | Separate sink/source buffers |
| Descriptor Format | 256-bit | 256-bit (compatible) |
| Channel Count | 8 | 8 |
| Flow Control | alloc_ctrl/drain_ctrl | STREAM stream_alloc_ctrl/stream_drain_ctrl inside the shared sram_controller (alloc_ctrl_beats/drain_ctrl_beats kept, out of the path) |

## 28.4	Clock and Reset Summary

### 28.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | Primary - all RAPIDS logic, AXI/AXIS interfaces |

### 28.4.2	Reset Signals

Reset Signals

| Reset | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary - all RAPIDS logic |

See: Clocks and Reset for complete timing specifications

## 28.5	Interface Summary

### 28.5.1	External Interfaces

External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXI4 (Descriptor) | Master | 256-bit | Descriptor fetch |
| AXI4 (Sink Write) | Master | 512-bit (param) | Sink data write to memory |
| AXI4 (Source Read) | Master | 512-bit (param) | Source data read from memory |
| AXIS (Sink) | Slave | 512-bit (param) | Network data ingress |
| AXIS (Source) | Master | 512-bit (param) | Network data egress |
| MonBus | Master | 64-bit | Monitor packet output |

### 28.5.2	Internal Buses

Internal Buses

| Interface | Width | Purpose |
| --- | --- | --- |
| MonBus | 64-bit | Internal monitoring bus |
| Descriptor Bus | 256-bit | Descriptor distribution |
| SRAM Data Bus | 512-bit | SRAM read/write data |

## 28.6	Related Documentation

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- RAPIDS Beats HAS - Architecture specification (the original pre-beats rapids_spec/ tree was retired)

## 28.7	Specification Conventions

### 28.7.1	Signal Naming

- Clock: aclk

- Reset: aresetn (active-low)

- Valid/Ready: Standard AXI/AXIS handshake

- Registers: r_ prefix (e.g., r_state, r_counter)

- Wires: w_ prefix (e.g., w_next_state, w_grant)

### 28.7.2	Parameter Naming

- Uppercase: NUM_CHANNELS, DATA_WIDTH, ADDR_WIDTH

- Derived: CHAN_WIDTH = $clog2(NUM_CHANNELS)

### 28.7.3	Figure and Table Numbering

- Figures: Figure X.Y.Z: Title where X=chapter, Y=section, Z=figure number

- Tables: Caption line : Table Title after table markdown

Last Updated: 2026-07-13 Maintained By: RAPIDS Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Product Requirements Document (PRD)

## 29.1	RAPIDS - Rapid AXI Programmable In-band Descriptor System

Version: 1.0 Date: 2025-09-30 Status: Active Development - Validation in Progress Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 29.2	1. Executive Summary

The Rapid AXI Programmable In-band Descriptor System (RAPIDS) is a custom hardware accelerator designed for efficient memory-to-memory data movement with network interface integration. It demonstrates complex FSM coordination, descriptor-based DMA operations, and comprehensive monitoring capabilities.

Status (2026-07-22): RAPIDS was rearchitected to the “beats” design. Current RTL is in rtl/fub_beats/, rtl/macro_beats/, rtl/top_beats/ (control engines in rtl/fub/); current specs are docs/rapids_beats_has/ and docs/rapids_beats_mas/. The old rapids_fub//rapids_macro/ RTL, the docs/rapids_spec/ tree, and the program engine / network master / network slave blocks described in parts of this PRD were retired (network interfaces are now AXIS; control is APB). Sections describing the pre-beats design are kept as history and marked where practical.

Status (2026-09-29, rapids TASK-019): RAPIDS is byte-granular. The product RTL is the un-suffixed tree rtl/fub/, rtl/macro/, rtl/top/rapids_top.sv: descriptor length in BYTES, byte-granular addresses, WSTRB/TSTRB carry partial beats, one AXIS packet per descriptor. The *_beats tree is the beat-granular stepping stone, kept runnable and green as the characterized Genesys 2 design. Tests: dv/tests/fub, macro, top.

### 29.2.1	1.1 Quick Stats

- Modules: ~20 SystemVerilog files (beats architecture)

- Architecture: 3 major groups (Scheduler group, Sink path, Source path)

- Interfaces: AXI4 (memory), AXIS (network), APB (control), MonBus (monitoring)

- Test Coverage: see dv/tests/ beats suites (fub_beats/macro_beats/top_beats)

- Status: Active validation, known issues documented

### 29.2.2	1.2 Subsystem Goals

- Primary: Demonstrate complex accelerator design patterns

- Secondary: Provide DMA-style memory transfer capability

- Tertiary: Educational reference for descriptor-based engines

## 29.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 29.3.1	Complete RAPIDS Specification

Status (2026-07-22): The old docs/rapids_spec/ tree was replaced by two beats spec trees.

Byte-granular RAPIDS books (the product design): HAS Index, MAS Index. Chapters that are identical for both designs are linked from the Beats books below.

Architecture Spec (HAS), RAPIDS Beats: docs/rapids_beats_has/

- HAS Index - Complete HAS structure

- Product Overview

- Block Diagram / Data Flow / Channel Architecture

- Interfaces: Summary, AXI4, AXIS, APB, MonBus

- Use cases: docs/rapids_beats_has/ch04_use_cases/

- Programming: Descriptor Format, Register Map, Initialization, Error Handling

- Performance: docs/rapids_beats_has/ch06_performance/

Micro-Architecture Spec (MAS): docs/rapids_beats_mas/

- MAS Index - Complete MAS structure

- Overview: Architecture, Port List, Clocks and Reset

FUB blocks (docs/rapids_beats_mas/ch02_fub_blocks/): - Scheduler - Per-channel scheduler FSM - Descriptor Engine - Descriptor fetch/parsing - AXI Read Engine / AXI Write Engine - Alloc Ctrl / Drain Ctrl / Latency Bridge - CtrlRd Engine / CtrlWr Engine

Macro blocks (docs/rapids_beats_mas/ch03_macro_blocks/): - Scheduler Group / Scheduler Group Array - Sink Data Path + AXIS wrapper, SRAM controllers - Source Data Path + AXIS wrapper, SRAM controllers - RAPIDS Core / Registers / Top

Interfaces (docs/rapids_beats_mas/ch04_interfaces/): - AXI4 Interface - AXIS Interface - MonBus Interface

### 29.3.2	Known Issues

Location: projects/components/dma-ip/rapids/known_issues/

- Index - Issue tracking overview (the old scheduler.md credit-counter write-up was retired with the pre-beats RTL)

- Sink Data Path - RESOLVED 2026-09-25 (write-error flag wired; timeouts are the monitor’s job by design)

- Sink SRAM Control - RESOLVED (moved to resolved/ in d587852c2)

- Plus known_issues/active/ for current beats issues

### 29.3.3	Other Documentation

- CLAUDE - AI assistance guide for this subsystem

- TASKS - Work items (largely pre-beats history)

- Validation Report - Test results (pre-beats snapshot)

## 29.4	2.4 Organizational Standards - RAPIDS Code Location

MANDATORY: All RAPIDS-specific code must be in the project area

### 29.4.1	Code Organization Principle

“All RAPIDS-specific verification code MUST reside in projects/components/dma-ip/rapids/dv/ for easy discovery.”

This subsystem follows the repository-wide organizational standard (see /PRD.md Section 2.3) requiring all project-specific code to be located in the project area, NOT the framework area.

### 29.4.2	RAPIDS Directory Structure

projects/components/dma-ip/rapids/
├── rtl/                          # RTL source code
│   ├── includes/                 # RAPIDS packages
│   ├── fub/                      # Control engines (ctrlrd/ctrlwr)
│   ├── fub_beats/                # Beats functional unit blocks
│   ├── macro_beats/              # Beats assemblies + registers
│   ├── macro/                    # MonBus group
│   └── top_beats/                # rapids_beats_top.sv
│
└── dv/                           # Design verification (all RAPIDS-specific)
    ├── tbclasses/                # RAPIDS TB classes HERE (not framework!)
    │   ├── scheduler_tb.py       # Scheduler testbench class
    │   ├── descriptor_engine_tb.py
    │   └── rapids_core_beats_tb.py
    │
    ├── components/               # RAPIDS-specific BFMs
    │   └── data_mover_bfm.py
    │
    └── tests/                    # Test runners (import TB classes)
        ├── fub/                  # Control engine tests
        ├── fub_beats/            # Beats FUB tests (test_scheduler_beats.py, ...)
        ├── macro/                # MonBus group test
        ├── macro_beats/          # Multi-block scenarios
        └── top_beats/            # Full RAPIDS operation

### 29.4.3	What Goes Where?

| Code Type | CORRECT Location | WRONG Location |
| --- | --- | --- |
| RAPIDS TB Classes | projects/components/dma-ip/rapids/dv/tbclasses/ | bin/TBClasses/ (framework area) |
| RAPIDS-Specific BFMs | projects/components/dma-ip/rapids/dv/components/ | bin/TBClasses/ (framework area) |
| RAPIDS Scoreboards | projects/components/dma-ip/rapids/dv/ (project area) | bin/TBClasses/scoreboards/ |
| Test Runners | projects/components/dma-ip/rapids/dv/tests/ | Anywhere else |
| Shared AXI4/APB BFMs | bin/TBClasses/{protocol}/ | Project area |

### 29.4.4	Import Pattern for RAPIDS Tests

CORRECT - Import from Project Area:

# Import framework utilities (PYTHONPATH includes bin/)
import os, sys
from TBClasses.shared.utilities import get_repo_root
from TBClasses.shared.tbbase import TBBase

# Add repo root to Python path using robust git-based method
repo_root = get_repo_root()
sys.path.insert(0, repo_root)

# Import RAPIDS TB classes from PROJECT AREA
from projects.components.dma_ip.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from projects.components.dma_ip.rapids.dv.tbclasses.descriptor_engine_tb import DescriptorEngineTB

# Shared framework components
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

WRONG - Don’t Import from Framework:

# DON'T DO THIS!
from TBClasses.rapids.scheduler_tb import SchedulerTB  # WRONG!

### 29.4.5	Benefits of This Organization

- Easy Discovery - All RAPIDS code in ONE place: projects/components/dma-ip/rapids/

- Clear Ownership - RAPIDS team owns their dv/ area completely

- No Confusion - Never wonder “where does this TB class live?”

- Maintainability - Changes isolated to RAPIDS area don’t affect other projects

- Framework Stays Clean - Only truly shared cross-project code in framework

### 29.4.6	Compliance Status

RAPIDS is now compliant - All TB classes moved to project area as of 2025-10-18

Migration History: - Before: TB classes incorrectly in bin/TBClasses/rapids/ - After: TB classes correctly in projects/components/dma-ip/rapids/dv/tbclasses/ - Test Imports: Updated to import from project area

Complete Documentation: See /PRD.md Section 2.3 for repository-wide organizational standards.

## 29.5	3. Architecture Overview

### 29.5.1	3.1 Top-Level Block Diagram

RAPIDS Beats (Rapid AXI Programmable In-band Descriptor System)
├── Scheduler Group (scheduler_group_beats / scheduler_group_array_beats)
│   ├── Scheduler          (Per-channel FSM, scheduler_beats.sv)
│   ├── Descriptor Engine  (Descriptor fetch/parsing, descriptor_engine_beats.sv)
│   └── Control Engines    (ctrlrd_engine.sv / ctrlwr_engine.sv)
│
├── Sink Data Path (AXIS Network → SRAM → System Memory)
│   ├── AXIS Slave ingress   (snk_data_path_axis_beats.sv)
│   ├── Sink SRAM Controller (snk_sram_controller_beats.sv + alloc/drain ctrl)
│   └── AXI Write Engine     (axi_write_engine_beats.sv)
│
├── Source Data Path (System Memory → SRAM → AXIS Network)
│   ├── AXI Read Engine        (axi_read_engine_beats.sv)
│   ├── Source SRAM Controller (src_sram_controller_beats.sv + alloc/drain ctrl)
│   └── AXIS Master egress     (src_data_path_axis_beats.sv)
│
└── Control and Monitoring
    ├── APB CSRs           (rapids_config_block.sv, PeakRDL rapids_regs.rdl)
    └── MonBus Group       (macro/monbus_axil_group_2in.sv)

See: docs/rapids_beats_mas/ch01_overview/01_architecture.md for detailed architecture

### 29.5.2	3.2 Data Flow

Sink Path (Receive): 1. Network packets arrive via Network Slave 2. Buffered in Sink SRAM 3. DMA’d to system memory via AXI4 Write Engine 4. Completion reported via MonBus

Source Path (Transmit): 1. Descriptor specifies data location in system memory 2. Source AXI Reader fetches data to Source SRAM 3. Network Master transmits to network 4. Completion reported via MonBus

Scheduler Coordination: - Parses descriptors from Descriptor Engine - Manages credit-based flow control - Sequences program engine operations - Coordinates sink/source data paths

## 29.6	4. Key Features

### 29.6.1	4.1 Descriptor-Based Operation

| Feature | Status | Description |
| --- | --- | --- |
| Descriptor FIFO | yes | Queued descriptor processing |
| Multi-field parsing | yes | Address, length, control fields |
| Chained descriptors | ⏳ | Future enhancement |
| Completion reporting | yes | Via MonBus packets |

### 29.6.2	4.2 Data Path Features

| Feature | Status | Description |
| --- | --- | --- |
| SRAM buffering | yes | Decouple network from memory |
| AXI4 burst support | yes | Efficient memory transfers |
| Backpressure handling | yes | Flow control on all interfaces |
| Data alignment | yes | Handle unaligned transfers |

### 29.6.3	4.3 Scheduler Features

| Feature | Status | Description |
| --- | --- | --- |
| Task FSM | yes | Multi-state coordination |
| Credit management | ⏳ | Pre-beats scheduler had exponential encoding (0→1, 1→2, …, 15→∞); scheduler_beats.sv has no credit management yet (planned later phase) |
| Program sequencing | yes | Coordinated operations |
| Error detection | yes | Timeout, overflow detection |

Credit Management Details (pre-beats, historical): - Uses exponential credit encoding for compact configuration - 4-bit cfg_initial_credit decodes to actual credit counts: - 0 → 1 credit (2^0), 4 → 16 credits (2^4), 8 → 256 credits (2^8) - 15 → ∞ (unlimited credits, 0xFFFFFFFF) - Encoding applied at initialization; runtime operations are linear (increment/decrement by 1) - Provides wide range (1 to 16384) with minimal configuration overhead

See: docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md for the current scheduler specification

### 29.6.4	4.4 Monitoring Integration

| Feature | Status | Description |
| --- | --- | --- |
| MonBus packets | yes | StandardAMBA 64-bit format |
| Descriptor events | yes | Start/complete reporting |
| Error events | yes | Timeout, overflow, underflow |
| Performance metrics | ⏳ | Future enhancement |

## 29.7	5. Interfaces

### 29.7.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| APB | Slave | 32-bit | Control/status registers |
| AXI4 (Sink) | Master | Configurable | Write to system memory |
| AXI4 (Source) | Master | Configurable | Read from system memory |
| AXIS (Sink) | Slave | Configurable | Network ingress (tid = channel) |
| AXIS (Source) | Master | Configurable | Network egress (tid = channel) |
| MonBus | Master | 64-bit | Monitor packet output |

See: docs/rapids_beats_mas/ch04_interfaces/ and docs/rapids_beats_has/ch03_interfaces/ for complete interface specs

### 29.7.2	5.2 Configuration Parameters

// Example RAPIDS instantiation sketch (see rtl/top_beats/rapids_beats_top.sv for the
// full parameter/port list)
rapids_beats_top #(
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .NUM_CHANNELS(8)
) u_rapids (
    .clk                (clk),
    .rst_n              (rst_n),
    // APB control interface
    .s_apb_*            (...),
    // AXI4 memory interfaces (read + write masters)
    .m_axi_*            (...),
    // AXIS network interfaces (tid = channel)
    .s_axis_*           (...),
    .m_axis_*           (...),
    // MonBus / monitor group egress
    .mon_*              (...)
);

## 29.8	6. Use Cases

### 29.8.1	6.1 DMA-Style Transfers

Scenario: Move data from network to system memory

Flow: 1. Software writes descriptor to Descriptor Engine 2. Scheduler parses descriptor, activates Sink path 3. Network packets arrive via Network Slave 4. Data buffered in Sink SRAM 5. AXI4 Write Engine DMAs to system memory 6. Completion packet on MonBus

### 29.8.2	6.2 Network Packet Processing

Scenario: Read data from memory, transmit to network

Flow: 1. Descriptor specifies source address, length 2. Source AXI Reader fetches data to SRAM 3. Network Master transmits to network 4. Completion/error reporting via MonBus

### 29.8.3	6.3 Custom Data Path Acceleration

Educational value: Shows how to build custom accelerators - Descriptor-based control - Multi-block FSM coordination - Buffering strategies - Error handling - Performance monitoring

## 29.9	7. Test Coverage

### 29.9.1	7.1 Current Status

Status (2026-07-22): The numbers below are a pre-beats snapshot (they include the retired program engine). Current beats regression: fub 232+ and macro 182 tests passing; run the beats suites for live numbers.

Overall (pre-beats snapshot): ~85% functional coverage (basic scenarios validated, descriptor engine complete)

| Component | Test Coverage | Status |
| --- | --- | --- |
| Scheduler | ~95% | Credit encoding fixed and verified (43/43 tests passing) |
| Descriptor Engine | 100% | All tests passing (14/14 tests, 100% success rate) |
| Program Engine | ~85% | Alignment tested |
| Sink Data Path | ~75% | Basic flows working |
| Source Data Path | ~70% | Basic flows working |
| SRAM Controllers | ~80% | Buffer management tested |
| Integration | ~60% | More stress testing needed |

Test Location: projects/components/dma-ip/rapids/dv/tests/fub_beats/ and projects/components/dma-ip/rapids/dv/tests/macro_beats/ (plus fub/, macro/, top_beats/)

Recent Achievements: - Descriptor Engine (2025-10-13): Achieved 100% test pass rate using continuous background monitoring pattern - 14/14 tests passing across all test levels (basic, medium, full) - All test classes passing (APB_ONLY, MIXED) - All delay profiles passing (fast_producer, fast_consumer, fixed_delay, minimal_delay) - Applied continuous monitoring methodology for asynchronous output capture

See: vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md for detailed results

### 29.9.2	7.2 Test Strategy

FUB (Functional Unit Block) Tests: - Individual block testing - Located in projects/components/dma-ip/rapids/dv/tests/fub_beats/ (+ fub/ for control engines) - Focus on module-level functionality

Macro Tests: - Multi-block scenarios - Located in projects/components/dma-ip/rapids/dv/tests/macro_beats/ (+ macro/ for the monbus group) - End-to-end data flow validation

Top Tests: - Full RAPIDS operation - Located in projects/components/dma-ip/rapids/dv/tests/top_beats/ - Realistic traffic patterns

## 29.10	8. Known Issues Summary

### 29.10.1	8.1 Critical Issues

Scheduler Credit Counter Initialization - FIXED (2025-10-11, pre-beats) - File: the retired pre-beats scheduler.sv (replaced by rtl/fub_beats/scheduler_beats.sv, which currently has no credit management - planned for a later phase) - Issue: Credit counter was initializing to 0 instead of using exponential encoding - Fix Applied: Implemented exponential credit encoding - Status: Fixed and verified pre-beats; the known_issues/scheduler.md write-up was retired with that RTL (see known_issues/README.md)

Fix Details:

// Previous (wrong):
r_descriptor_credit_counter <= 32'h0;

// Fixed - Exponential encoding:
// 0→1, 1→2, 2→4, 3→8, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Encoding Rationale: - Compact 4-bit configuration covers 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞) - Exponential encoding applied at initialization only; runtime operations are linear

### 29.10.2	8.2 Medium Priority Issues

Descriptor Engine Edge Cases: - Some stress test failures under high load - Edge case handling needs improvement - Priority: P2

SRAM Control Timing: - Rare timing issues in back-to-back operations - Priority: P2

See: known_issues/ directory for complete issue tracking

## 29.11	9. Integration Guidelines

### 29.11.1	9.1 Quick Start

rapids_beats_top #(
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .NUM_CHANNELS(8)
) u_rapids (
    // Clocking & Reset
    .clk                (system_clk),
    .rst_n              (system_rst_n),

    // APB Control (connect to control fabric)
    .s_apb_paddr        (ctrl_paddr),
    .s_apb_psel         (ctrl_psel),
    .s_apb_penable      (ctrl_penable),
    // ... other APB signals

    // AXI4 Memory (connect to memory controller)
    // ... AXI write channels (sink), AXI read channels (source)

    // AXIS Network (connect to network fabric; tid = channel)
    .s_axis_tdata       (net_rx_data),
    .s_axis_tvalid      (net_rx_valid),
    // ... AXIS slave (receive), AXIS master (transmit)

    // Monitor group egress (AXI-Lite drain / IRQ)
    // ... mon_* signals
);

See rtl/top_beats/rapids_beats_top.sv for the complete, authoritative port list.

### 29.11.2	9.2 Configuration Steps

- Initialize via APB registers (use the generated regmap, access registers by name):

  - Configure channel enables and thresholds

  - Set timeout thresholds

- Load Descriptors:

  - Write descriptors to Descriptor Engine FIFO

  - Each descriptor specifies: address, length, control bits

- Enable Operation:

  - Set enable bits via APB registers

  - Monitor MonBus for completion/error packets

See: docs/rapids_beats_has/ch05_programming/ for register and programming details

## 29.12	10. Development Status

### 29.12.1	10.1 Current Phase

Phase: Validation and Bug Fixing (In Progress)

- Core architecture implemented

- Basic functionality verified

- Scheduler credit counter bug fixed (exponential encoding implemented)

- ⏳ Credit management tests need verification (remove workarounds)

- ⏳ Stress testing ongoing

- ⏳ Edge case refinement

See: TASKS.md for detailed work items

### 29.12.2	10.2 Roadmap

Near-Term (Q4 2025): - Fix scheduler credit counter bug (completed 2025-10-11) - ⏳ Verify credit management tests (remove workarounds) - ⏳ Complete descriptor engine stress testing - ⏳ Integration test suite expansion - ⏳ Performance benchmarking

Long-Term (2026+): - Chained descriptor support - Advanced error recovery - Performance optimizations - Multi-channel support

## 29.13	11. Performance Characteristics

### 29.13.1	11.1 Throughput

Target: Match network/memory interface bandwidth

Bottlenecks: - SRAM buffer size - AXI4 burst efficiency - Scheduler overhead

Optimization: - Increase SRAM depth for larger packets - Tune AXI4 burst parameters - Pipeline scheduler operations

### 29.13.2	11.2 Latency

Components: - Descriptor parsing: ~10 cycles - SRAM buffering: Configurable depth - AXI4 memory access: System dependent - End-to-end: Typically <100 cycles for small packets

### 29.13.3	11.3 Resource Utilization

Area: - Scheduler: ~2K LUTs - Each data path: ~3K LUTs - SRAM buffers: Configurable (dominant area) - Total: ~10K LUTs + SRAM

Power: - Clock gating opportunities in idle blocks - SRAM power depends on depth/width

## 29.14	12. Verification Infrastructure

### 29.14.1	12.1 Test Organization

Location: projects/components/dma-ip/rapids/dv/tests/

Structure:

projects/components/dma-ip/rapids/dv/tests/
├── fub/                    # Control engine tests (ctrlrd/ctrlwr)
├── fub_beats/              # Beats FUB tests (scheduler, descriptor engine,
│                           #   alloc/drain ctrl, latency bridge)
├── macro/                  # MonBus group test
├── macro_beats/            # Multi-block scenarios (scheduler group, data paths,
│                           #   SRAM controllers)
└── top_beats/              # Full RAPIDS operation

### 29.14.2	12.2 CocoTB Framework

Location: projects/components/dma-ip/rapids/dv/tbclasses/ (RAPIDS-specific) plus shared BFMs in bin/TBClasses/

Components: - RAPIDS-specific drivers - Descriptor generators - Traffic patterns - Monitor checkers

See: ../RTLDesignSherpa-DV/docs/components/<family>/ for shared framework docs

### 29.14.3	12.2.1 MANDATORY: BFM Usage for FUB Tests

CRITICAL DESIGN REQUIREMENT

All RAPIDS FUB (Functional Unit Block) level tests MUST use CocoTB Framework BFMs. Manual handshake driving is NOT allowed.

Required BFM Components:

| Interface Type | Framework Location | BFM Component |
| --- | --- | --- |
| Custom valid/ready | bin/TBClasses/gaxi/ | GAXI Master/Slave |
| AXI4 | bin/TBClasses/axi4/ | AXI4 Master/Slave |
| AXI4-Lite (AXIL) | bin/TBClasses/axil4/ | AXIL Master/Slave |
| APB | bin/TBClasses/apb/ | APB Master/Slave |
| AXI-Stream (AXIS) | bin/TBClasses/axis4/ | AXIS Master/Slave (RAPIDS network interfaces are AXIS) |
| MonBus | bin/TBClasses/monbus/ | MonBus drivers |

Rationale: 1. Consistency: All tests use standardized handshake protocols 2. Correctness: BFMs handle complex timing scenarios (backpressure, randomization) 3. Reusability: Same BFM across all RAPIDS tests 4. Maintainability: Fix once in BFM, all tests benefit 5. Coverage: BFMs include comprehensive timing profiles

Example - Program Engine:

# WRONG: Manual handshake (violates design requirement)
async def send_request(self, addr, data):
    self.dut.program_valid.value = 1
    self.dut.program_pkt_addr.value = addr
    # ... manual handshaking logic ...

# CORRECT: Use GAXI Master BFM
from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster

class ProgramEngineTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.program_master = GAXIMaster(
            dut=dut,
            clock=dut.clk,
            valid_signal='program_valid',
            ready_signal='program_ready',
            data_signals=['program_pkt_addr', 'program_pkt_data'],
            data_widths=[64, 32]
        )

    async def send_request(self, addr, data):
        await self.program_master.write({'program_pkt_addr': addr, 'program_pkt_data': data})

See: - projects/components/dma-ip/rapids/CLAUDE.md - Rule #1 for complete BFM usage guidelines - ../RTLDesignSherpa-DV/docs/components/gaxi/ - GAXI BFM documentation - bin/TBClasses/axi4/ - AXI4 BFM sources (full framework docs in the RTLDesignSherpa-DV repo)

### 29.14.4	12.3 Test File Structure (Standard Pattern)

MANDATORY: All RAPIDS tests must follow this structure

RAPIDS tests follow the same pattern as AMBA tests for consistency across the repository:

# Example: projects/components/dma-ip/rapids/dv/tests/fub_beats/test_scheduler_beats.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# Import REUSABLE testbench class (NOT defined in this file!)
from projects.components.dma_ip.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic_flow(dut):
    """Test basic descriptor flow."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_basic_descriptor_flow()
    assert result, "Basic descriptor flow test failed"

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_credit_encoding(dut):
    """Test exponential credit encoding."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_exponential_encoding_all_values()
    assert result, "Credit encoding test failed"

# ===========================================================================
# PARAMETER GENERATION - at bottom of file
# ===========================================================================

def generate_scheduler_test_params():
    """Generate test parameters for scheduler tests."""
    return [
        # (channel_id, num_channels, data_width, credit_width)
        (0, 8, 512, 8),  # Standard configuration
        # Add more parameter sets as needed
    ]

scheduler_params = generate_scheduler_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - at bottom of file
# ===========================================================================

@pytest.mark.parametrize("channel_id, num_channels, data_width, credit_width", scheduler_params)
def test_basic_flow(request, channel_id, num_channels, data_width, credit_width):
    """
    Scheduler basic flow test.

    Run with: pytest projects/components/dma-ip/rapids/dv/tests/fub_beats/test_scheduler_beats.py::test_basic_flow -v
    """
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_fub_beats': '../../rtl/fub_beats'
    })

    dut_name = "scheduler_beats"
    toplevel = dut_name

    verilog_sources = [
        os.path.join(repo_root, 'rtl', 'amba', 'includes', 'ocelus_pkg.sv'),
        # RAPIDS package(s) from the project includes area
        # (prefer get_sources_from_filelist() with rtl/filelists/ in real tests)
        os.path.join(rtl_dict['rtl_fub_beats'], '..', 'includes', 'rapids_pkg.sv'),
        os.path.join(rtl_dict['rtl_fub_beats'], f'{dut_name}.sv'),
    ]

    # Format parameters for unique test name
    cid_str = TBBase.format_dec(channel_id, 2)
    nc_str = TBBase.format_dec(num_channels, 2)
    dw_str = TBBase.format_dec(data_width, 4)
    cw_str = TBBase.format_dec(credit_width, 2)
    test_name_plus_params = f"test_{dut_name}_cid{cid_str}_nc{nc_str}_dw{dw_str}_cw{cw_str}"

    # Add worker ID for pytest-xdist parallel execution
    worker_id = os.environ.get('PYTEST_XDIST_WORKER', '')
    if worker_id:
        test_name_plus_params = f"{test_name_plus_params}_{worker_id}"

    log_path = os.path.join(log_dir, f'{test_name_plus_params}.log')
    sim_build = os.path.join(tests_dir, 'local_sim_build', test_name_plus_params)
    os.makedirs(sim_build, exist_ok=True)
    os.makedirs(log_dir, exist_ok=True)

    rtl_parameters = {
        'CHANNEL_ID': channel_id,
        'NUM_CHANNELS': num_channels,
        'DATA_WIDTH': data_width,
        'CREDIT_WIDTH': credit_width,
        # Add other RTL parameters as needed
    }

    extra_env = {
        'LOG_PATH': log_path,
        'TEST_CHANNEL_ID': str(channel_id),
        'TEST_NUM_CHANNELS': str(num_channels),
        'TEST_DATA_WIDTH': str(data_width),
    }

    compile_args = ["-Wno-TIMESCALEMOD"]
    sim_args = []
    plusargs = []

    cmd_filename = create_view_cmd(log_dir, log_path, sim_build, module, test_name_plus_params)

    try:
        run(
            python_search=[tests_dir],
            verilog_sources=verilog_sources,
            includes=[
                os.path.join(repo_root, 'projects', 'components', 'dma-ip', 'rapids', 'rtl', 'includes'),
                os.path.join(repo_root, 'rtl', 'amba', 'includes'),
            ],
            toplevel=toplevel,
            module=module,
            testcase="cocotb_test_basic_flow",  # ← cocotb test function name
            parameters=rtl_parameters,
            sim_build=sim_build,
            extra_env=extra_env,
            waves=False,
            keep_files=True,
            compile_args=compile_args,
            sim_args=sim_args,
            plusargs=plusargs,
        )

        print(f"Scheduler basic flow test completed!")
        print(f"Logs: {log_path}")

    except Exception as e:
        print(f"Scheduler basic flow test failed: {str(e)}")
        print(f"Logs preserved at: {log_path}")
        raise

Key Structure Requirements:

- Testbench Class Location:

  - ALWAYS in projects/components/dma-ip/rapids/dv/tbclasses/

  - NEVER inline in test file

  - Reusable across multiple test files

- CocoTB Test Functions:

  - Prefix with cocotb_ to prevent pytest collection

  - Located at top of test file

  - Use @cocotb.test() decorator

  - Call testbench methods

- Parameter Generation:

  - Function returns list of parameter tuples

  - Located near bottom of file (before pytest wrappers)

  - Stored in variable (e.g., scheduler_params)

- Pytest Wrapper Functions:

  - Located at bottom of file

  - Use @pytest.mark.parametrize() with parameter variable

  - Build unique test names with TBBase.format_dec()

  - Call run() with testcase="cocotb_test_function_name"

  - Handle parallel execution (PYTEST_XDIST_WORKER)

- Mandatory TB Methods:

  - async def setup_clocks_and_reset(self) - Complete initialization

  - async def assert_reset(self) - Assert reset signal(s)

  - async def deassert_reset(self) - Deassert reset signal(s)

See: - val/amba/test_apb4_slave.py - Reference example - projects/components/dma-ip/rapids/CLAUDE.md - Detailed TB requirements

## 29.15	13. Quick Reference

### 29.15.1	13.1 Key Files

| File | Purpose |
| --- | --- |
| projects/components/dma-ip/rapids/PRD.md | This document (overview) |
| projects/components/dma-ip/rapids/CLAUDE.md | AI assistance guide |
| projects/components/dma-ip/rapids/TASKS.md | Work items (largely pre-beats history) |
| projects/components/dma-ip/rapids/docs/rapids_beats_has/ | Architecture specification |
| projects/components/dma-ip/rapids/docs/rapids_beats_mas/ | Micro-architecture specification |
| projects/components/dma-ip/rapids/known_issues/ | Bug tracking |
| vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md | Test results (pre-beats snapshot) |

### 29.15.2	13.2 Commands

# Run RAPIDS tests
pytest projects/components/dma-ip/rapids/dv/tests/fub_beats/ -v     # Individual blocks
pytest projects/components/dma-ip/rapids/dv/tests/macro_beats/ -v   # Multi-block
pytest projects/components/dma-ip/rapids/dv/tests/top_beats/ -v     # Full system

# Run specific FUB test
pytest projects/components/dma-ip/rapids/dv/tests/fub_beats/test_scheduler_beats.py -v

# Lint RAPIDS RTL
verilator --lint-only projects/components/dma-ip/rapids/rtl/fub_beats/scheduler_beats.sv

# View specifications
cat projects/components/dma-ip/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md
cat projects/components/dma-ip/rapids/docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md

## 29.16	14. Success Criteria

### 29.16.1	14.1 Functional

- All major blocks implemented

- Basic data flows working

- Scheduler credit bug fixed (exponential encoding implemented)

- ⏳ Credit management tests verified (remove workarounds, run full suite)

- ⏳ 100% descriptor test pass rate (currently ~80%)

- ⏳ Stress tests passing

### 29.16.2	14.2 Quality

- ⏳ >90% functional coverage (currently ~80%)

- ⏳ >85% code coverage

- All FSMs documented

- ⏳ Integration guide complete

### 29.16.3	14.3 Documentation

- Complete specification in docs/rapids_beats_has/ + docs/rapids_beats_mas/

- Known issues documented

- ⏳ Integration examples

- ⏳ Performance characterization

## 29.17	15. Educational Value

RAPIDS demonstrates: - Complex FSM coordination (scheduler ↔ data paths) - Descriptor-based DMA design patterns - Buffer management strategies - Credit-based flow control with exponential encoding - Multi-interface integration - Comprehensive monitoring - Error detection and reporting - Compact configuration encoding strategies

Target Audience: - Advanced RTL designers - Accelerator architects - DMA engine developers - System integration engineers

## 29.18	16. Documentation Generation

### 29.18.1	16.1 Generating PDF/DOCX from Specification

Tool: bin/md_to_docx.py (driven by the wrapper scripts in docs/ - preferred)

Use the wrapper scripts to convert the linked HAS/MAS spec indexes into single all-inclusive PDF/DOCX files:

Basic Usage:

# Preferred: wrapper scripts (they call md_to_docx.py with the house style)
cd projects/components/dma-ip/rapids/docs
./generate_has_pdf.sh --rev 0.8     # builds RAPIDS_Beats_HAS_v0.8.docx/.pdf
./generate_mas_pdf.sh --rev 0.7     # builds RAPIDS_Beats_MAS_v0.7.docx/.pdf

# Direct tool invocation (from repo root), if you need custom options
python bin/md_to_docx.py \
    projects/components/dma-ip/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md \
    -o projects/components/dma-ip/rapids/docs/RAPIDS_Beats_MAS_draft.docx \
    --toc \
    --title-page \
    --pdf

Key Features: - Recursive Collection: Follows all markdown links in the index file - Heading Demotion: Automatically adjusts heading levels for included files - Table of Contents: --toc flag generates automatic ToC - Title Page: --title-page flag creates title page from first heading - PDF Export: --pdf flag generates both DOCX and PDF - Image Support: Resolves images relative to source directory - Template Support: Optional custom DOCX/DOTX template via -t flag

Common Workflow:

# 1. Update spec content under rapids_beats_has/ or rapids_beats_mas/
# 2. Generate documentation with a bumped revision
cd projects/components/dma-ip/rapids/docs
./generate_mas_pdf.sh --rev 0.8

# 3. Output files created in docs/:
#    - RAPIDS_Beats_MAS_v0.8.docx
#    - RAPIDS_Beats_MAS_v0.8.pdf

Debug Mode:

# Generate debug markdown to see combined output
python bin/md_to_docx.py \
    projects/components/dma-ip/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md \
    -o output.docx \
    --debug-md

# This creates debug.md showing the complete merged content

Tool Requirements: - Python 3.6+ - Pandoc installed and in PATH - For PDF generation: LaTeX (e.g., texlive) or use Pandoc’s built-in PDF writer

See: bin/md_to_docx.py for complete implementation details

## 29.19	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/dma-ip/rapids/docs/

Quick Command: Use the provided shell scripts:

cd projects/components/dma-ip/rapids/docs
./generate_has_pdf.sh    # Architecture spec (HAS)
./generate_mas_pdf.sh    # Micro-architecture spec (MAS)

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the rapids_beats_has / rapids_beats_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

See: bin/md_to_docx.py for complete implementation details

## 29.20	17. References

### 29.20.1	16.1 Internal Documentation

- Complete Spec: docs/rapids_beats_has/ + docs/rapids_beats_mas/ ← Primary technical reference

- Validation: vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md (pre-beats snapshot)

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 29.20.2	16.2 Related Subsystems

- AMBA: rtl/amba/ - Monitor infrastructure used in RAPIDS (monitors in rtl/amba/ocelus/)

- Common: rtl/common/ - Building blocks (counters, FIFOs, etc.)

- CocoTB Framework: bin/TBClasses/ (shared) + projects/components/dma-ip/rapids/dv/tbclasses/ (RAPIDS)

### 29.20.3	16.3 External References

- AXI4 Specification: ARM IHI0022E

- AXIL4 Specification: ARM IHI0022E (subset)

- Network interface specs (custom Network protocol)

Document Version: 1.0 Last Updated: 2025-09-30 Review Cycle: Monthly during active development Next Review: 2025-10-30 Owner: RTL Design Sherpa Project

## 29.21	Navigation

- ← Back to Root: /PRD.md

- Complete Specification: docs/rapids_beats_has/rapids_beats_has_index.md + docs/rapids_beats_mas/rapids_beats_mas_index.md

- AI Guidance: CLAUDE.md

- Tasks: TASKS.md

- Issues: known_issues/

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Address Increment Patterns for DMA and Descriptor-Based Data Movement

Document Version: 1.0 Last Updated: 2026-01-13 Scope: Comprehensive analysis of address increment patterns for descriptor-based systems Reference Implementation: RAPIDS (Rapid AXI Programmable In-band Descriptor System)

## 30.1	Table of Contents

- Executive Summary

- RAPIDS Current Implementation

- AXI Protocol Burst Modes

- Linear Address Increment Patterns

- 2D Transfer Patterns (Stride-Based)

- 3D Transfer Patterns

- Scatter-Gather Patterns

- Circular and Ring Buffer Patterns

- Tensor Memory Access Patterns

- Implementation Comparison Matrix

- RAPIDS Extension Recommendations

- References

## 30.2	1. Executive Summary

Address increment patterns determine how DMA engines and descriptor-based data movement systems calculate memory addresses during transfers. The choice of pattern significantly impacts:

- Bandwidth efficiency: Alignment and burst optimization

- Application fit: Different workloads require different patterns

- Hardware complexity: More patterns = more control logic

- Software flexibility: Richer patterns = simpler programming model

### 30.2.1	Pattern Categories

| Category | Description | Use Cases |
| --- | --- | --- |
| Linear/Contiguous | Sequential address increment | Block copy, streaming data |
| 2D Stride | Row-major with inter-row gaps | Image processing, video frames |
| 3D Volume | Multi-plane with inter-plane gaps | 3D imaging, tensor operations |
| Scatter-Gather | Non-contiguous fragment list | Protocol buffers, fragmented memory |
| Circular | Wrap-around at boundary | Audio streaming, network buffers |
| Tensor | Multi-dimensional with strides | Neural network accelerators |

## 30.3	2. RAPIDS Current Implementation

### 30.3.1	2.1 Descriptor Format (256-bit)

RAPIDS uses a fixed 256-bit descriptor structure defined in rapids_pkg.sv:

Bit Field          Width   Description
─────────────────────────────────────────────────────────
[63:0]             64-bit  src_addr - Source address (byte-aligned)
[127:64]           64-bit  dst_addr - Destination address (byte-aligned)
[159:128]          32-bit  length - Transfer length in BEATS (not bytes)
[191:160]          32-bit  next_descriptor_ptr - Next descriptor address
[207:200]           8-bit  desc_priority - Transfer priority
[199:196]           4-bit  channel_id - Channel ID
[195]               1-bit  error - Error flag
[194]               1-bit  last - Last descriptor in chain
[193]               1-bit  gen_irq - Generate interrupt
[192]               1-bit  valid - Valid descriptor
[255:208]          48-bit  reserved - Future use

### 30.3.2	2.2 Current Address Increment Modes

Mode 1: Beat-Based Linear (Default)

Address Calculation:
  next_addr = current_addr + (beat_count × 64 bytes)

  Where: RAPIDS_BYTES_PER_BEAT = 64 bytes (512-bit data width)

- All transfers aligned to 64-byte boundaries for optimal throughput

- Descriptor length field specifies count in beats

- Software converts byte count to beats: beats = (bytes + 63) >> 6

Mode 2: Chunk-Based Alignment (Sub-Beat Transfers)

For alignment/final phases handling partial beats:

Address Calculation for Alignment Phase:
  offset = address[5:0]                    // 6-bit offset (0-63)
  bytes_to_boundary = (offset == 0) ? 0 : 64 - offset

  Transfer Phases:
  Phase 1 (Alignment): Transfer bytes_to_boundary to reach 64B boundary
  Phase 2 (Streaming): Transfer full 64B beats
  Phase 3 (Final):     Transfer remaining < 64 bytes

### 30.3.3	2.3 Alignment Information Structure

typedef struct packed {
    logic [15:0] first_chunk_enables;    // Phase 1 chunk mask
    logic [15:0] streaming_chunk_enables; // Phase 2 (always 0xFFFF)
    logic [15:0] final_chunk_enables;     // Phase 3 chunk mask
    logic [31:0] first_transfer_bytes;    // Phase 1 size
    logic [31:0] streaming_bytes;         // Phase 2 size
    logic [31:0] final_transfer_bytes;    // Phase 3 size
    logic [3:0]  first_start_chunk;       // Starting chunk (0-15)
    logic [3:0]  first_valid_chunks;      // Chunks in phase 1
    logic [3:0]  final_valid_chunks;      // Chunks in phase 3
} alignment_info_t;

### 30.3.4	2.4 AXI Translation

Transfer Phase    AxSIZE    AxBURST    Address Increment
────────────────────────────────────────────────────────
Alignment         3'b010    INCR       4 bytes × (AxLEN+1)
Streaming         3'b110    INCR       64 bytes × (AxLEN+1)
Final             3'b010    INCR       4 bytes × (AxLEN+1)

## 30.4	3. AXI Protocol Burst Modes

The AXI protocol defines three burst types that constrain address generation:

### 30.4.1	3.1 FIXED Burst (AxBURST = 2’b00)

Address Formula:
  beat[n].addr = AxADDR   (constant for all beats)

Use Cases: - FIFO access (single address register) - Memory-mapped I/O ports - Audio codec sample registers

Constraints: - All beats transfer to same address - Cannot cross 4KB boundary (trivially satisfied)

### 30.4.2	3.2 INCREMENT Burst (AxBURST = 2’b01)

Address Formula:
  beat[n].addr = AxADDR + n × (1 << AxSIZE)

  Example (AxSIZE=6, 64-byte):
    beat[0] = 0x1000
    beat[1] = 0x1040
    beat[2] = 0x1080
    ...

Use Cases: - Linear memory access - Block copy operations - Stream data to/from memory

Constraints: - Cannot cross 4KB boundary - Maximum burst length varies by protocol version

### 30.4.3	3.3 WRAP Burst (AxBURST = 2’b10)

Address Formula:
  wrap_boundary = AxADDR & ~((AxLEN+1) × (1<<AxSIZE) - 1)
  wrap_size = (AxLEN+1) × (1 << AxSIZE)

  beat[n].addr = wrap_boundary + ((AxADDR + n×(1<<AxSIZE)) mod wrap_size)

  Example (AxADDR=0x1020, AxLEN=3, AxSIZE=4):
    Wrap boundary: 0x1000
    Wrap size: 64 bytes (4 × 16)
    beat[0] = 0x1020
    beat[1] = 0x1030
    beat[2] = 0x1000  (wrapped)
    beat[3] = 0x1010

Use Cases: - Cache line fills (critical-word-first) - Circular buffer access

Constraints: - AxLEN must be 1, 3, 7, or 15 - Starting address must be aligned to transfer size

## 30.5	4. Linear Address Increment Patterns

### 30.5.1	4.1 Simple Contiguous

The most basic pattern - sequential addresses:

Descriptor Fields:
  src_addr:  Starting source address
  dst_addr:  Starting destination address
  length:    Total transfer size

Address Calculation:
  for each beat b in [0, length/beat_size):
    src = src_addr + b × beat_size
    dst = dst_addr + b × beat_size

RAPIDS Implementation: Native support via beat-based transfers.

### 30.5.2	4.2 Unaligned Start/End

Handling transfers that don’t start or end on natural boundaries:

Example: Transfer 200 bytes from 0x1040
  Phase 1 (Align):  Transfer 64 bytes (0x1040-0x107F) to reach 0x1080
  Phase 2 (Stream): Transfer 128 bytes (0x1080-0x10FF) in 2 full beats
  Phase 3 (Final):  Transfer 8 bytes (0x1100-0x1107)

RAPIDS Implementation: Native support via alignment_info_t structure.

### 30.5.3	4.3 Size-Constrained Bursts

Breaking large transfers into AXI-compliant bursts:

Constraints:
  - AXI4: Max 256 beats per burst
  - No 4KB boundary crossing

Address Calculation:
  remaining = total_length
  current_addr = start_addr

  while (remaining > 0):
    burst_len = min(remaining, 256×beat_size)
    burst_len = min(burst_len, next_4kb_boundary - current_addr)
    issue_burst(current_addr, burst_len)
    current_addr += burst_len
    remaining -= burst_len

RAPIDS Implementation: Handled by AXI engine burst splitting.

## 30.6	5. 2D Transfer Patterns (Stride-Based)

### 30.6.1	5.1 Concept

2D transfers move rectangular regions where source and destination may have different memory layouts (pitches/strides).

Memory Layout:
  ┌─────────────────────────────────────┐
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 0: Transfer 4 elements
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 1: Transfer 4 elements
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 2: Transfer 4 elements
  │ . . . . . . . . . . . . . . . . . . │
  └─────────────────────────────────────┘
   ←─────── src_stride (pitch) ────────→
   ←─ width ─→

### 30.6.2	5.2 Descriptor Fields (Extended)

2D Descriptor Format:
  src_addr:    Starting source address (top-left corner)
  dst_addr:    Starting destination address
  x_length:    Bytes per row (width of transfer region)
  y_length:    Number of rows
  src_stride:  Source bytes between row starts
  dst_stride:  Destination bytes between row starts

### 30.6.3	5.3 Address Calculation

for row in [0, y_length):
  row_src = src_addr + row × src_stride
  row_dst = dst_addr + row × dst_stride
  transfer(row_src, row_dst, x_length)

### 30.6.4	5.4 Use Cases

Image Processing:

Example: Extract 640×480 ROI from 1920×1080 frame (32-bit pixels)
  src_addr:   frame_base + (y_offset × 1920 + x_offset) × 4
  dst_addr:   roi_buffer
  x_length:   640 × 4 = 2560 bytes
  y_length:   480
  src_stride: 1920 × 4 = 7680 bytes
  dst_stride: 640 × 4 = 2560 bytes (packed destination)

Video Frame Tiling:

Example: Copy 16×16 macroblock
  x_length:   16 × bytes_per_pixel
  y_length:   16
  src_stride: frame_width × bytes_per_pixel
  dst_stride: 16 × bytes_per_pixel

### 30.6.5	5.5 Analog Devices AXI DMAC Implementation

Reference implementation from ADI’s high-speed DMA controller:

Registers:
  X_LENGTH  (0x418): Row width - 1
  Y_LENGTH  (0x41C): Row count - 1
  SRC_STRIDE (0x424): Source row spacing
  DEST_STRIDE (0x420): Destination row spacing

Address Formulas:
  ROW_SRC_ADDRESS = SRC_ADDRESS + SRC_STRIDE × N
  ROW_DEST_ADDRESS = DEST_ADDRESS + DEST_STRIDE × N

## 30.7	6. 3D Transfer Patterns

### 30.7.1	6.1 Concept

3D transfers extend 2D with an additional depth/plane dimension:

Memory Layout (Z planes stacked):
  Plane 0:    Plane 1:    Plane 2:
  ┌───────┐   ┌───────┐   ┌───────┐
  │ █ █ █ │   │ █ █ █ │   │ █ █ █ │
  │ █ █ █ │   │ █ █ █ │   │ █ █ █ │
  └───────┘   └───────┘   └───────┘

### 30.7.2	6.2 Descriptor Fields (Extended)

3D Descriptor Format:
  src_addr:     Starting address (origin corner)
  dst_addr:     Destination address
  x_length:     Width in bytes
  y_length:     Height in rows
  z_length:     Depth in planes
  src_stride_x: Source row stride (unused, implicit x_length)
  src_stride_y: Source row-to-row stride
  src_stride_z: Source plane-to-plane stride
  dst_stride_y: Destination row stride
  dst_stride_z: Destination plane stride

### 30.7.3	6.3 Address Calculation

for plane in [0, z_length):
  for row in [0, y_length):
    src = src_addr + plane×src_stride_z + row×src_stride_y
    dst = dst_addr + plane×dst_stride_z + row×dst_stride_y
    transfer(src, dst, x_length)

### 30.7.4	6.4 Use Cases

3D Medical Imaging:

Example: Extract 64×64×64 VOI from 512×512×256 CT volume
  src_addr:     volume_base + z_off×512×512 + y_off×512 + x_off
  x_length:     64 bytes
  y_length:     64
  z_length:     64
  src_stride_y: 512
  src_stride_z: 512 × 512

Neural Network Tensors:

Example: Move 4D tensor slice [batch, channel, H, W]
  Requires nested 3D operations or generalized tensor addressing

## 30.8	7. Scatter-Gather Patterns

### 30.8.1	7.1 Concept

Scatter-Gather (SG) handles non-contiguous memory regions using linked lists of descriptors:

Descriptor Chain:
  ┌──────────┐     ┌──────────┐     ┌──────────┐
  │ Desc 0   │────→│ Desc 1   │────→│ Desc 2   │────→ NULL
  │ addr=A0  │     │ addr=A1  │     │ addr=A2  │
  │ len=L0   │     │ len=L1   │     │ len=L2   │
  └──────────┘     └──────────┘     └──────────┘

### 30.8.2	7.2 SG Descriptor Fields

Scatter-Gather Descriptor:
  buffer_addr:      Address of this fragment
  buffer_length:    Size of this fragment
  next_desc_ptr:    Address of next descriptor (0 = last)
  control_flags:    Completion interrupt, error handling, etc.
  status:           Completion status (written by DMA)

### 30.8.3	7.3 Operation Modes

Gather (Multiple Source -> Single Destination):

Source fragments: [A0:L0], [A1:L1], [A2:L2]
Destination: Contiguous buffer B

Result: B = concat(mem[A0:A0+L0], mem[A1:A1+L1], mem[A2:A2+L2])

Scatter (Single Source -> Multiple Destinations):

Source: Contiguous buffer B of length L0+L1+L2
Destination fragments: [A0:L0], [A1:L1], [A2:L2]

Result: mem[A0:A0+L0] = B[0:L0]
        mem[A1:A1+L1] = B[L0:L0+L1]
        mem[A2:A2+L2] = B[L0+L1:end]

### 30.8.4	7.4 AMD/Xilinx AXI DMA SG Descriptor Format

Standard Descriptor (32-bit addressing):
  Offset  Field            Description
  ──────────────────────────────────────────────
  0x00    NXTDESC          Next descriptor pointer
  0x04    Reserved
  0x08    BUFFER_ADDRESS   Data buffer pointer
  0x0C    Reserved
  0x10    Reserved
  0x14    Reserved
  0x18    CONTROL          Transfer length, SOF, EOF
  0x1C    STATUS           Completion status

### 30.8.5	7.5 Use Cases

Network Packet Assembly:

Example: Assemble TCP packet from scattered headers and payload
  Desc 0: Ethernet header (14 bytes at addr_eth)
  Desc 1: IP header (20 bytes at addr_ip)
  Desc 2: TCP header (20 bytes at addr_tcp)
  Desc 3: Payload (1460 bytes at addr_payload)

RAPIDS Implementation: Native support via next_descriptor_ptr chaining.

## 30.9	8. Circular and Ring Buffer Patterns

### 30.9.1	8.1 Concept

Circular buffers wrap addresses at a boundary, enabling continuous streaming without descriptor reload:

Memory Layout:
  ┌─────────────────────────────────────┐
  │ 7 │ 0 │ 1 │ 2 │ 3 │ 4 │ 5 │ 6 │ 7 │ (wraps)
  └─────────────────────────────────────┘
      ↑               ↑
    write           read

### 30.9.2	8.2 Descriptor Fields (Extended)

Circular Buffer Descriptor:
  base_addr:    Buffer base address
  buffer_size:  Total buffer size (must be power of 2 for efficient wrap)
  current_ptr:  Current read/write position
  wrap_enable:  Enable address wrapping

### 30.9.3	8.3 Address Calculation

Efficient wrap (power-of-2 size):
  next_addr = base_addr + ((current_ptr + increment) & (buffer_size - 1))

General wrap:
  offset = (current_ptr + increment) % buffer_size
  next_addr = base_addr + offset

### 30.9.4	8.4 Ping-Pong Double Buffering

Common implementation using two descriptors pointing to each other:

Descriptor A:               Descriptor B:
  buffer_addr = BUF_A        buffer_addr = BUF_B
  next_desc = &Desc_B        next_desc = &Desc_A

Operation:
  While DMA processes BUF_A, CPU fills BUF_B
  On completion, DMA auto-loads Desc_B, starts BUF_B
  CPU processes BUF_A while DMA runs BUF_B
  (repeat)

### 30.9.5	8.5 Use Cases

Audio Streaming:

Example: 48kHz stereo audio, 20ms buffers
  buffer_size: 48000 × 2 channels × 2 bytes × 0.02s = 3840 bytes
  Ring with 4 buffers: 15360 bytes total
  Wrap at: base + 15360

Network RX Ring:

Example: Ethernet receive ring
  descriptor_count: 256 (power of 2)
  Each descriptor points to packet buffer
  Last descriptor.next points to first descriptor
  Hardware auto-advances through ring

## 30.10	9. Tensor Accelerator Memory Access Patterns (PRIORITY 0)

Tensor accelerators for neural network inference represent the most demanding and sophisticated address generation requirements. This section provides comprehensive coverage of industry-standard approaches.

### 30.10.1	9.1 Systolic Array Dataflow Fundamentals

Systolic arrays (used in Google TPU, NVIDIA Tensor Cores, etc.) require coordinated data movement across three matrices: inputs (activations), weights, and outputs (partial sums).

Three Primary Dataflows:

| Dataflow | Stationary Data | Moving Data | Best For |
| --- | --- | --- | --- |
| Weight Stationary (WS) | Weights preloaded in PEs | Inputs + partial sums flow | Weight reuse (many inputs) |
| Input Stationary (IS) | Inputs held in PEs | Weights + partial sums flow | Input reuse (many kernels) |
| Output Stationary (OS) | Partial sums accumulate in PEs | Inputs + weights flow | Output accumulation |

Address Pattern Implications:

Weight Stationary:
  - Weights: Load once per layer, stream through array
  - Inputs: Stream row-by-row, reuse across weight tiles
  - Address pattern: 2D tiled with weight-tile-aligned boundaries

Output Stationary:
  - Outputs: Accumulate in place
  - Inputs/Weights: Double-buffered streaming
  - Address pattern: Output-tile-centric addressing

### 30.10.2	9.2 Data Cube Model (NVDLA Reference)

NVIDIA’s Deep Learning Accelerator (NVDLA) defines tensor addressing using a “data cube” model with three key stride parameters:

Data Cube Structure:

Dimensions: Width (W) × Height (H) × Channels (C)

Memory Layout:
  - Data divided into 1×1×32byte "atom cubes"
  - Scanning order: C'(32B) → W → H → C (surfaces)
  - C' changes fastest (innermost loop)

Address Calculation:
  element_addr = base_addr
                 + surface_index × surface_stride
                 + line_index × line_stride
                 + element_offset_within_line

Stride Parameters:

| Parameter | Description | Alignment |
| --- | --- | --- |
| line_stride | Bytes from line N to line N+1 | 32 bytes |
| surface_stride | Bytes from surface N to surface N+1 | 32 bytes |
| base_addr | Starting address | 32 bytes |

NVDLA Register Configuration:

D_DATAIN_FORMAT     : Data format selection
D_DATAIN_SIZE_0     : Width × Height
D_DATAIN_SIZE_1     : Channels
D_LINE_STRIDE       : Line-to-line spacing (0x5040)
D_SURF_STRIDE       : Surface-to-surface spacing

Example:
  Width=26, Height=32, Channels=3
  line_stride = 0x1A0 (aligned width)
  surface_stride = 0x1520 (line_stride × height)

### 30.10.3	9.3 Tensor Memory Accelerator (TMA) - NVIDIA Hopper

NVIDIA’s Hopper architecture introduces hardware TMA for efficient multi-dimensional tensor transfers:

TMA Descriptor Format:

CUtensorMap structure (64-bit packed descriptor):
  - Global memory base address
  - Tensor rank (1D to 5D supported)
  - Dimension sizes and strides
  - Swizzle pattern for bank conflict avoidance
  - Box dimensions (tile size for transfer)

Key Constraints:

Alignment Requirements:
  - Contiguous dimension must have stride=1
  - All other strides must be multiples of 16 bytes
  - For float tensors [M,N] with stride [N,1]: N % 4 == 0

Tile Quantization:
  - TPU/GPU systolic arrays process 128×128 or 128×8 tiles
  - Tensors must align to tile boundaries to avoid padding waste
  - Performance penalty for partial tiles (idle MACs)

Address Generation (Hardware):

TMA eliminates per-thread address calculation:
  Traditional: Each thread computes addr = base + thread_id × stride
  TMA: Single descriptor, hardware generates all addresses

Coordinate-based access:
  ArithTuple(row, col) → Hardware translates to physical address
  Automatic out-of-bounds predication (no manual boundary checks)

### 30.10.4	9.4 Convolution Address Patterns

#### 30.10.4.1	9.4.1 Direct Convolution

Input Feature Map:  [N, C_in, H_in, W_in]
Weight Kernel:      [C_out, C_in, K_h, K_w]
Output Feature Map: [N, C_out, H_out, W_out]

Output position (n, c_out, h_out, w_out):
  For each (c_in, k_h, k_w):
    input_addr = base_in + n×(C_in×H_in×W_in)
                         + c_in×(H_in×W_in)
                         + (h_out×stride_h + k_h)×W_in
                         + (w_out×stride_w + k_w)

    weight_addr = base_w + c_out×(C_in×K_h×K_w)
                         + c_in×(K_h×K_w)
                         + k_h×K_w
                         + k_w

#### 30.10.4.2	9.4.2 Im2Col Transformation

Converts convolution to GEMM for systolic array efficiency:

Original Convolution:
  Output[h,w] = sum(Input[h+kh, w+kw] × Weight[kh, kw])

Im2Col Transformation:
  - Unroll each receptive field into a column
  - Weight kernels become rows
  - Convolution becomes matrix multiplication

Address Pattern for Im2Col:
  For output position (h_out, w_out):
    col_index = h_out × W_out + w_out
    For kernel position (c_in, k_h, k_w):
      row_index = c_in × K_h × K_w + k_h × K_w + k_w
      src_addr = base + c_in×(H×W) + (h_out×s_h+k_h)×W + (w_out×s_w+k_w)
      dst_addr = im2col_base + row_index × num_output_positions + col_index

Data Expansion Factor:
  Naive: K_h × K_w × data replication
  Hardware Im2Col: Generate addresses on-the-fly, no physical expansion

#### 30.10.4.3	9.4.3 Strided and Dilated Convolution

Strided Convolution (stride > 1):
  input_h = output_h × stride + kernel_h
  input_w = output_w × stride + kernel_w
  Reduces output spatial dimensions

Dilated/Atrous Convolution:
  input_h = output_h + kernel_h × dilation
  input_w = output_w + kernel_w × dilation
  Expands receptive field without increasing parameters

Address Modification:
  base_addr + (h × stride + kh × dilation) × line_stride
            + (w × stride + kw × dilation) × element_size

#### 30.10.4.4	9.4.4 Depthwise Separable Convolution

Standard Convolution: C_in × C_out × K × K multiplications
Depthwise Separable: C_in × K × K + C_in × C_out multiplications

Phase 1 - Depthwise (per-channel):
  For each channel c:
    output[c, h, w] = conv2d(input[c], kernel[c])

  Address Pattern:
    Each channel processes independently
    No cross-channel data movement
    input_addr = base + c × H × W + h × W + w

Phase 2 - Pointwise (1×1 convolution):
  Standard GEMM across channels
  input_addr = base + c × (H × W) + position

### 30.10.5	9.5 Gemmini Accelerator Architecture (UC Berkeley Reference)

Open-source systolic array generator with well-documented DMA and addressing:

Memory Hierarchy:

┌─────────────────────────────────────────────┐
│                 Main Memory                  │
│              (Virtual Addresses)             │
└────────────────────┬────────────────────────┘
                     │ DMA (TileLink)
                     ▼
┌─────────────────────────────────────────────┐
│              Scratchpad SRAM                 │
│   (Banked, Row-Addressed, Explicit Mgmt)    │
│   Default: 256KB, 16 elements/row           │
└────────────────────┬────────────────────────┘
                     │
                     ▼
┌─────────────────────────────────────────────┐
│         Systolic Array (16×16 PEs)          │
│    Weight Stationary or Output Stationary   │
└────────────────────┬────────────────────────┘
                     │
                     ▼
┌─────────────────────────────────────────────┐
│            Accumulator SRAM                  │
│   (32-bit accumulators, in-place addition)  │
└─────────────────────────────────────────────┘

Scratchpad Address Format (32-bit):

Bit 31:    0=Scratchpad, 1=Accumulator
Bit 30:    Accumulator mode (0=overwrite, 1=accumulate)
Bit 29:    Read format (0=scaled inputType, 1=raw accType)
Bits 28:0: Row address within selected memory

Row Width:
  Scratchpad: DIM × inputType bits (default: 16 × 8 = 128 bits)
  Accumulator: DIM × accType bits (default: 16 × 32 = 512 bits)

DMA Operations:

// mvin: Move data from DRAM to scratchpad
// Configuration registers set stride
mvin(
    dram_addr,           // Virtual address (64-bit)
    sp_addr,             // Scratchpad row address
    rows,                // Number of rows to transfer
    cols                 // Elements per row (≤ DIM)
);

// mvout: Move data from accumulator to DRAM
// Supports integrated max-pooling
mvout(
    dram_addr,           // Destination address
    acc_addr,            // Accumulator row address
    rows, cols,
    pool_size,           // Optional pooling window
    pool_stride
);

Stride Configuration:

config_mvin:
  stride[0]: Source memory stride for mvin_A
  stride[1]: Source memory stride for mvin_B
  stride[2]: Source memory stride for mvin_D

config_mvout:
  stride: Destination memory stride
  pool_params: Pooling window and stride

### 30.10.6	9.6 Tiling for Large Matrices

When matrices exceed on-chip memory, tiling is essential:

Hierarchical Tiling (CUTLASS Model):

Level 1 - Thread Block Tile:
  Load tile from global memory to shared memory
  Tile size: M_tile × K_tile (A), K_tile × N_tile (B)

Level 2 - Warp Tile:
  Load from shared memory to registers
  Tile size: Warp_M × Warp_K (A), Warp_K × Warp_N (B)

Level 3 - Thread Tile:
  Individual thread's computation
  Register-to-register operations

Address Generation per Level:
  Global: base + block_id × block_tile_size
  Shared: smem_base + warp_id × warp_tile_size
  Register: Direct indexing within tile

Double-Buffering for Latency Hiding:

Buffer Structure:
  tile_buffer[2][TILE_ROWS][TILE_COLS]

Pipeline:
  Cycle N:   Compute on buffer[0], Load to buffer[1]
  Cycle N+1: Compute on buffer[1], Load to buffer[0]

Address Pattern:
  compute_addr = base + (cycle % 2) × tile_size + element_offset
  load_addr    = base + ((cycle + 1) % 2) × tile_size + element_offset

### 30.10.7	9.7 Sparse Tensor Patterns

Sparse accelerators require index-based indirect addressing:

Compressed Sparse Row (CSR) Format:

Dense: [a 0 b]    CSR: values = [a, b, c, d]
       [0 c 0]         col_idx = [0, 2, 1, 0]
       [d 0 0]         row_ptr = [0, 2, 3, 4]

Address Calculation:
  For row r, iterate col_idx[row_ptr[r]] to col_idx[row_ptr[r+1]-1]
  value_addr = values_base + idx
  col_addr = col_idx_base + idx

Indirect Access:
  actual_col = memory[col_addr]
  dense_addr = base + r × stride + actual_col × element_size

Hardware Implications:

Dense Accelerator (TPU): Initiation interval = 1 (fully pipelined)
Sparse Accelerator (EIE): Initiation interval = 17 (metadata overhead)

Trade-off:
  - Sparse saves memory bandwidth (skip zeros)
  - But adds index indirection latency
  - Effective for >90% sparsity

### 30.10.8	9.8 Transformer/Attention Patterns

Modern LLM accelerators require specialized patterns:

Q, K, V Matrix Generation:

Input: X [seq_len, d_model]
Weights: W_Q, W_K, W_V [d_model, d_k]

Q = X × W_Q  →  [seq_len, d_k]
K = X × W_K  →  [seq_len, d_k]
V = X × W_V  →  [seq_len, d_k]

Address Pattern: Standard GEMM with different weight matrices
  q_addr = q_base + seq_idx × d_k + head_idx
  k_addr = k_base + seq_idx × d_k + head_idx
  v_addr = v_base + seq_idx × d_k + head_idx

Attention Score Computation:

Attention = softmax(Q × K^T / sqrt(d_k)) × V

Memory Challenge:
  Q × K^T produces [seq_len × seq_len] matrix
  O(n^2) memory for sequence length n

FlashAttention Tiling:
  Tile Q, K, V into blocks that fit in SRAM
  Compute attention incrementally without materializing full matrix

  Tile Address Pattern:
    q_tile_addr = q_base + tile_row × tile_size × d_k
    k_tile_addr = k_base + tile_col × tile_size × d_k

  Online Softmax:
    Track running max and sum across tiles
    No need to store intermediate attention matrix

### 30.10.9	9.9 Programmable Address Generation Unit (PAGU)

Academic reference for flexible tensor addressing:

PAGU Instruction Set:

Operations Supported:
  - Standard Convolution
  - Padded Convolution
  - Strided Convolution
  - Dilated (Atrous) Convolution
  - Pooling (Max, Average)
  - Upsampled Convolution
  - Transposed Convolution

Performance: 1.7 cycles per address for convolution
Area Overhead: ~4.6× vs fixed datapath (justified by flexibility)

Configuration Registers:

tensor_config_t:
  base_addr[63:0]           // Tensor base address
  dim_sizes[4][31:0]        // Size of each dimension
  dim_strides[4][31:0]      // Stride of each dimension
  kernel_size[15:0]         // Convolution kernel size
  conv_stride[7:0]          // Convolution stride
  dilation[7:0]             // Dilation factor
  padding[7:0]              // Padding amount
  operation_mode[3:0]       // Select operation type

Address Generation FSM:

always_ff @(posedge clk) begin
    case (state)
        IDLE: begin
            if (start) begin
                dim_counters <= '0;
                state <= GENERATE;
            end
        end

        GENERATE: begin
            // Multi-dimensional address calculation
            addr <= base_addr;
            for (int d = 0; d < num_dims; d++) begin
                case (operation_mode)
                    CONV_STANDARD:
                        addr <= addr + dim_counters[d] * dim_strides[d];
                    CONV_STRIDED:
                        addr <= addr + (dim_counters[d] * conv_stride) * dim_strides[d];
                    CONV_DILATED:
                        addr <= addr + (dim_counters[d] * dilation) * dim_strides[d];
                endcase
            end

            // Increment counters (innermost first)
            increment_counters();

            if (done) state <= IDLE;
        end
    endcase
end

## 30.11	10. Implementation Comparison Matrix

### 30.11.1	10.1 Pattern Complexity vs. Capability

| Pattern | Descriptor Fields | Address Logic | Hardware Cost | SW Flexibility |
| --- | --- | --- | --- | --- |
| Linear | 3 (src, dst, len) | Simple add | Low | Limited |
| 2D Stride | 6 (+x_len, y_len, strides) | Nested loop | Medium | Good |
| 3D Volume | 9 (+z_len, z_strides) | Triple nested | Medium-High | Very Good |
| Scatter-Gather | 4 per fragment | List traversal | Medium | Excellent |
| Circular | 4 (+size, wrap) | Modulo/mask | Low-Medium | Good |
| Tensor | N (variable stride) | Sum-of-products | High | Excellent |

### 30.11.2	10.2 Use Case Mapping

| Application Domain | Primary Pattern | Secondary Pattern |
| --- | --- | --- |
| Block memory copy | Linear | - |
| Network packets | Scatter-Gather | Linear |
| Audio streaming | Circular | Linear |
| Image processing | 2D Stride | Linear |
| Video encode/decode | 2D Stride | Scatter-Gather |
| 3D graphics | 3D Volume | 2D Stride |
| Neural networks | Tensor | 2D Stride |
| Database operations | Scatter-Gather | Linear |

### 30.11.3	10.3 RAPIDS Current vs. Extended Capability

| Capability | Current RAPIDS | Industry Standard |
| --- | --- | --- |
| Linear contiguous | YES | YES |
| Unaligned transfers | YES (3-phase) | Varies |
| Descriptor chaining | YES | YES |
| 2D stride | NO | Common |
| 3D volume | NO | Rare |
| Scatter-Gather | Partial (chain) | Full SG |
| Circular wrap | NO | Common |
| Tensor addressing | NO | Emerging |

## 30.12	11. RAPIDS Extension Recommendations

### 30.12.1	11.0 PRIORITY 0: Tensor Accelerator Addressing (CRITICAL)

Rationale: Essential for AI/ML workloads. Tensor accelerators represent the highest-growth market for DMA engines. Without tensor addressing, RAPIDS cannot serve neural network inference applications.

Implementation Approach: Data Cube Model (NVDLA-style)

This approach provides the best balance of flexibility and hardware complexity, supporting convolution, pooling, and GEMM operations.

Extended Descriptor Format:

typedef struct packed {
    // === Base Fields (existing, 256 bits) ===
    logic [63:0]  src_addr;           // Base source address
    logic [63:0]  dst_addr;           // Base destination address
    logic [31:0]  length;             // For linear mode: beats
    logic [31:0]  next_descriptor_ptr;
    logic [7:0]   desc_priority;
    logic [3:0]   channel_id;
    logic         error;
    logic         last;
    logic         gen_irq;
    logic         valid;

    // === Tensor Extension (repurpose reserved bits + add 2nd descriptor word) ===
    // Mode selection
    logic [3:0]   transfer_mode;      // 0=linear, 1=2D, 2=3D/tensor, 3=conv, 4=pool

    // Data cube dimensions
    logic [15:0]  width;              // W dimension (elements)
    logic [15:0]  height;             // H dimension (lines)
    logic [15:0]  channels;           // C dimension (surfaces)

    // Stride parameters (32-byte aligned)
    logic [31:0]  line_stride;        // Bytes between lines
    logic [31:0]  surface_stride;     // Bytes between surfaces

    // Convolution parameters (when transfer_mode == CONV)
    logic [7:0]   kernel_width;       // Kw
    logic [7:0]   kernel_height;      // Kh
    logic [7:0]   conv_stride_x;      // Horizontal stride
    logic [7:0]   conv_stride_y;      // Vertical stride
    logic [7:0]   dilation_x;         // Horizontal dilation
    logic [7:0]   dilation_y;         // Vertical dilation
    logic [7:0]   pad_left;           // Left padding
    logic [7:0]   pad_right;          // Right padding
    logic [7:0]   pad_top;            // Top padding
    logic [7:0]   pad_bottom;         // Bottom padding

    // Pooling parameters (when transfer_mode == POOL)
    logic [7:0]   pool_width;
    logic [7:0]   pool_height;
    logic [7:0]   pool_stride_x;
    logic [7:0]   pool_stride_y;
    logic [1:0]   pool_type;          // 0=max, 1=avg, 2=min

    // Data type and precision
    logic [3:0]   data_type;          // 0=int8, 1=int16, 2=fp16, 3=bf16, 4=fp32
    logic [3:0]   element_size;       // Bytes per element (1, 2, 4)
} descriptor_tensor_t;

Address Generation Unit (AGU) Architecture:

module tensor_address_generator #(
    parameter int PIPE_STAGES = 3     // Pipeline for multiply-accumulate
) (
    input  logic        clk,
    input  logic        rst_n,

    // Configuration from descriptor
    input  descriptor_tensor_t desc,
    input  logic        start,

    // Address output stream
    output logic [63:0] addr,
    output logic        addr_valid,
    input  logic        addr_ready,

    // Status
    output logic        done
);

    // Dimension counters
    logic [15:0] w_cnt, h_cnt, c_cnt;
    logic [7:0]  kw_cnt, kh_cnt;       // Kernel position (for conv)

    // Pipeline registers for address calculation
    logic [63:0] addr_stage[PIPE_STAGES];

    // FSM states
    typedef enum logic [2:0] {
        IDLE,
        CALC_BASE,
        CALC_LINE,
        CALC_SURFACE,
        OUTPUT
    } state_t;
    state_t state;

    // Address calculation based on mode
    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            state <= IDLE;
            w_cnt <= '0;
            h_cnt <= '0;
            c_cnt <= '0;
        end else begin
            case (state)
                IDLE: begin
                    if (start) begin
                        w_cnt <= '0;
                        h_cnt <= '0;
                        c_cnt <= '0;
                        state <= CALC_BASE;
                    end
                end

                CALC_BASE: begin
                    // Stage 1: Base address
                    addr_stage[0] <= desc.src_addr;
                    state <= CALC_LINE;
                end

                CALC_LINE: begin
                    // Stage 2: Add line offset
                    case (desc.transfer_mode)
                        MODE_LINEAR:
                            addr_stage[1] <= addr_stage[0] + (w_cnt * desc.element_size);

                        MODE_2D, MODE_3D:
                            addr_stage[1] <= addr_stage[0]
                                           + (h_cnt * desc.line_stride)
                                           + (w_cnt * desc.element_size);

                        MODE_CONV:
                            // Convolution: account for stride and kernel position
                            addr_stage[1] <= addr_stage[0]
                                           + ((h_cnt * desc.conv_stride_y + kh_cnt * desc.dilation_y) * desc.line_stride)
                                           + ((w_cnt * desc.conv_stride_x + kw_cnt * desc.dilation_x) * desc.element_size);

                        MODE_POOL:
                            addr_stage[1] <= addr_stage[0]
                                           + ((h_cnt * desc.pool_stride_y) * desc.line_stride)
                                           + ((w_cnt * desc.pool_stride_x) * desc.element_size);
                    endcase
                    state <= CALC_SURFACE;
                end

                CALC_SURFACE: begin
                    // Stage 3: Add surface offset (for 3D/tensor)
                    if (desc.transfer_mode inside {MODE_3D, MODE_CONV}) begin
                        addr_stage[2] <= addr_stage[1] + (c_cnt * desc.surface_stride);
                    end else begin
                        addr_stage[2] <= addr_stage[1];
                    end
                    state <= OUTPUT;
                end

                OUTPUT: begin
                    if (addr_ready) begin
                        // Increment counters (innermost first)
                        if (desc.transfer_mode == MODE_CONV) begin
                            // Convolution: iterate kernel, then spatial, then channels
                            if (kw_cnt < desc.kernel_width - 1) begin
                                kw_cnt <= kw_cnt + 1;
                            end else begin
                                kw_cnt <= '0;
                                if (kh_cnt < desc.kernel_height - 1) begin
                                    kh_cnt <= kh_cnt + 1;
                                end else begin
                                    kh_cnt <= '0;
                                    // Continue to spatial/channel iteration
                                    increment_spatial_counters();
                                end
                            end
                        end else begin
                            increment_spatial_counters();
                        end

                        if (transfer_complete())
                            state <= IDLE;
                        else
                            state <= CALC_BASE;
                    end
                end
            endcase
        end
    end

    assign addr = addr_stage[PIPE_STAGES-1];
    assign addr_valid = (state == OUTPUT);
    assign done = (state == IDLE) && !start;

endmodule

Scratchpad Integration (Gemmini-style):

// Memory address format for tensor scratchpad
typedef struct packed {
    logic        mem_select;       // 0=scratchpad, 1=accumulator
    logic        acc_mode;         // 0=overwrite, 1=accumulate
    logic        read_format;      // 0=scaled, 1=raw
    logic [28:0] row_addr;         // Row address within memory
} scratchpad_addr_t;

// DMA configuration for tensor loads
typedef struct packed {
    logic [63:0] dram_addr;        // Virtual address in main memory
    scratchpad_addr_t sp_addr;     // Scratchpad destination
    logic [15:0] rows;             // Number of rows to transfer
    logic [15:0] cols;             // Elements per row
    logic [31:0] dram_stride;      // Source memory stride
    logic [15:0] sp_stride;        // Scratchpad row stride (usually 1)
} tensor_dma_config_t;

Alignment Requirements:

All tensor addresses must satisfy:
  - base_addr aligned to 32 bytes (NVDLA requirement)
  - line_stride aligned to 32 bytes
  - surface_stride aligned to 32 bytes
  - For optimal performance: align to 64 bytes (RAPIDS beat size)

Verification:
  assert((desc.src_addr & 32'h1F) == 0);
  assert((desc.line_stride & 32'h1F) == 0);
  assert((desc.surface_stride & 32'h1F) == 0);

Performance Considerations:

Address Generation Rate:
  - Target: 1 address per cycle (pipelined)
  - Convolution: 1.7 cycles/address typical (due to kernel iteration)

Memory Bandwidth:
  - TPU-class: 600+ GB/s (HBM)
  - FPGA-class: 20-80 GB/s (DDR4/5)
  - RAPIDS target: Match AXI interface bandwidth

Tiling Support:
  - Hardware computes tile boundaries
  - Double-buffering via ping-pong descriptors
  - Software provides tile dimensions, hardware handles addressing

### 30.12.2	11.1 Priority 1: 2D Stride Support

Rationale: High value for image/video processing with moderate complexity. Also serves as foundation for tensor addressing.

Descriptor Extension:

typedef struct packed {
    // Existing fields (256 bits)...

    // 2D extension (96 bits in reserved space)
    logic [31:0] x_length;        // Bytes per row
    logic [15:0] y_length;        // Number of rows
    logic [31:0] src_stride;      // Source row stride
    logic [31:0] dst_stride;      // Destination row stride
    logic        mode_2d;         // Enable 2D mode
} descriptor_2d_t;

Address Generation:

always_comb begin
    if (mode_2d) begin
        row_src_addr = src_addr + current_row * src_stride;
        row_dst_addr = dst_addr + current_row * dst_stride;
        row_length   = x_length;
    end else begin
        row_src_addr = src_addr;
        row_dst_addr = dst_addr;
        row_length   = length * BYTES_PER_BEAT;
    end
end

### 30.12.3	11.2 Priority 2: Circular Buffer Mode

Rationale: Essential for streaming applications, low complexity addition.

Descriptor Extension:

typedef struct packed {
    // Existing fields...

    // Circular buffer extension
    logic [31:0] buffer_size;     // Total buffer size
    logic [31:0] buffer_mask;     // Size-1 for power-of-2
    logic        circular_mode;   // Enable wrap-around
    logic        src_circular;    // Apply to source
    logic        dst_circular;    // Apply to destination
} descriptor_circular_t;

Address Generation:

function automatic logic [63:0] wrap_address(
    logic [63:0] base,
    logic [63:0] offset,
    logic [31:0] mask,
    logic        enable
);
    if (enable)
        return base + (offset & {32'b0, mask});
    else
        return base + offset;
endfunction

### 30.12.4	11.3 Priority 3: Enhanced Scatter-Gather

Rationale: Full SG capability for network and fragmented memory use cases.

Current Limitation: Chain-only SG requires one descriptor per fragment.

Enhancement: Add fragment list support within single descriptor:

typedef struct packed {
    logic [63:0] fragment_list_ptr;  // Pointer to fragment array
    logic [15:0] fragment_count;     // Number of fragments
    logic        sg_mode;            // Enable SG mode

    // Fragment entry format (in memory):
    // [63:0]  address
    // [31:0]  length
    // [31:0]  flags
} descriptor_sg_t;

## 30.13	12. References

### 30.13.1	Industry Documentation

- AMD AXI DMA Product Guide (PG021) - Scatter-Gather descriptor format

- Analog Devices AXI DMAC - 2D transfer implementation

- Understanding AXI Addressing (ZipCPU) - Burst mode analysis

- WRAP Address Calculation (Verification Guide) - Wrap burst formulas

### 30.13.2	GPU and Accelerator Patterns

- NVIDIA CUDA Memory Coalescing - Coalesced access patterns

- CUDA Pitch Linear Memory - 2D array allocation

- NVIDIA TMA Tutorial (CUTLASS) - Tensor Memory Accelerator

- Efficient GEMM in CUDA (CUTLASS) - Hierarchical tiling

### 30.13.3	DMA Architecture

- DMA330 Microcode - Linked-list implementation

### 30.13.4	Neural Network Accelerators

- NVDLA Hardware Architecture - Data cube model, CDMA

- NVDLA In-Memory Data Formats - Line stride, surface stride

- NVDLA Unit Description - Convolution DMA details

- Gemmini Accelerator (UC Berkeley) - Open-source systolic array

- Gemmini Documentation (Chipyard) - DMA and scratchpad

- Google TPU Architecture - Systolic array overview

- TPU System Architecture - Memory hierarchy

- Programmable AGU for DNNs - Flexible address generation

### 30.13.5	Convolution and Tensor Operations

- Im2Col Characterization - Convolution memory patterns

- ST Neural-ART Programming Model - NPU descriptor concepts

- Systolic Array Dataflows Survey - Weight/input/output stationary

- FlashAttention Analysis - Attention tiling patterns

- Depthwise Separable Convolutions - Channel-wise addressing

### 30.13.6	Sparse Tensor Formats

- FLAASH Sparse Tensor Accelerator - CSR/CSF formats

- SparTen CNN Accelerator - Sparse tensor addressing

- Indirection Stream Architecture - Hardware indirect addressing

## 30.14	Document History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-13 | RTL Design Sherpa | Initial comprehensive analysis |

This document is part of the RAPIDS (Rapid AXI Programmable In-band Descriptor System) design documentation.