RTL Design Sherpa

STREAM Subsystem Hardware Architecture Specification

January 3, 2026

Table of Contents

1 Stream Has Index	23

2 Document Information	24

2.1 STREAM Hardware Architecture Specification	24

2.2 Document Purpose	25

2.3 References	26

2.4 Terminology	27

2.5 Revision History	28

3 Purpose and Scope	33

3.1 Document Purpose	33

3.2 Scope	34

3.2.1 In Scope	34

3.2.2 Out of Scope	34

3.3 Design Philosophy	35

3.3.1 Tutorial-Focused Simplicity	35

3.3.2 Relationship to RAPIDS	35

3.4 Intended Audience	36

4 Document Conventions	37

4.1 Notation Conventions	37

4.1.1 Signal Naming	37

4.1.2 Interface Prefixes	37

4.2 Diagram Conventions	38

4.2.1 Block Diagrams	38

4.2.2 Timing Diagrams	38

4.3 Numeric Conventions	39

4.3.1 Units	39

4.3.2 Address Notation	39

4.3.3 Size Notation	39

4.4 Requirement Notation	40

4.5 Reference Format	41

5 Definitions and Acronyms	42

5.1 Acronyms	42

5.2 Definitions	43

5.2.1 Architectural Terms	43

5.2.2 Interface Terms	43

5.2.3 Transfer Terms	44

5.3 Units and Measurements	45

5.4 Document-Specific Terms	46

6 Use Cases	47

6.1 Primary Use Cases	47

6.1.1 Memory-to-Memory Data Movement	47

6.1.2 Scatter-Gather Operations	47

6.1.3 Multi-Channel Concurrent Transfers	47

6.2 Secondary Use Cases	48

6.2.1 Buffer Management	48

6.2.2 Memory Initialization	48

6.3 Target Applications	49

7 Key Features	50

7.1 Feature Summary	50

7.2 Descriptor-Based Operation	51

7.2.1 Single-Write Kick-off	51

7.2.2 Autonomous Chaining	51

7.3 Multi-Channel Architecture	52

7.3.1 Channel Independence	52

7.3.2 Resource Sharing	52

7.3.3 Priority-Based Arbitration	52

7.4 Data Path Features	53

7.4.1 Parameterizable Data Width	53

7.4.2 Aligned Address Requirement	53

7.5 Monitoring and Debug	54

7.5.1 MonBus Integration	54

7.5.2 Status Visibility	54

7.6 Error Handling	55

7.6.1 Error Detection	55

7.6.2 Error Isolation	55

8 System Context	56

8.1 System Integration Overview	56

8.1.1 Figure 2.3.1: System Context	56

8.2 External Connections	57

8.2.1 Configuration Path	57

8.2.2 Data Paths	57

8.2.3 Monitoring Path	57

8.3 Clock and Reset	58

8.3.1 Clock Domain	58

8.3.2 Reset Requirements	58

8.4 Memory Map Requirements	59

8.4.1 Descriptor Memory	59

8.4.2 Data Memory	59

8.5 Interrupt Integration	60

8.5.1 Interrupt Signals	60

8.5.2 Interrupt Conditions	60

8.6 Power Considerations	61

8.6.1 Clock Gating	61

8.6.2 Power States	61

9 Block Diagram	62

9.1 Top-Level Architecture	62

9.1.1 Figure 3.1.1: STREAM Whole Unit	62

9.2 Functional Block Summary	63

9.3 Block Interactions	65

9.3.1 Control Flow	65

9.3.2 Data Flow	65

9.3.3 Monitoring Flow	65

9.4 Parameterization	66

10 Data Flow	67

10.1 Transfer Sequence Overview	67

10.1.1 Figure 3.2.1: Transfer Sequence	67

10.2 Phase Details	68

10.2.1 Phase 1: Kick-off	68

10.2.2 Phase 2: Descriptor Fetch	68

10.2.3 Phase 3: Descriptor Parse	68

10.2.4 Phase 4: Arbitration	68

10.2.5 Phase 5: Read Phase	68

10.2.6 Phase 6: Write Phase	69

10.2.7 Phase 7: Chain Check	69

10.3 Concurrent Operation	70

10.3.1 Multi-Channel Concurrency	70

10.3.2 Pipeline Overlap	70

10.4 Data Path Bandwidth	71

10.4.1 Theoretical Maximum	71

10.4.2 Practical Considerations	71

11 Multi-Channel Architecture	72

11.1 Channel Organization	72

11.1.1 Figure 3.3.1: Channel Architecture	72

11.2 Per-Channel Resources	73

11.3 Shared Resources	74

11.4 Arbitration Scheme	75

11.4.1 Round-Robin Arbitration	75

11.4.2 Arbitration Points	75

11.5 Channel States	76

11.5.1 Figure 3.3.2: Channel FSM State Diagram	76

11.6 SRAM Buffering	78

11.7 Error Isolation	79

12 AXI Master Interfaces	80

12.1 Overview	80

12.2 Descriptor Fetch Master	81

12.2.1 Signal Summary	81

12.2.2 Transaction Characteristics	81

12.2.3 Address Alignment	82

12.3 Data Read Master	83

12.3.1 Signal Summary	83

12.3.2 Transaction Characteristics	83

12.3.3 Burst Generation	84

12.4 Data Write Master	85

12.4.1 Signal Summary	85

12.4.2 Transaction Characteristics	86

12.5 Common AXI Attributes	87

12.5.1 Supported Features	87

12.5.2 Error Handling	87

13 APB Configuration Interface	88

13.1 Overview	88

13.2 Signal Summary	89

13.3 Register Map Overview	90

13.4 Key Registers	92

13.4.1 Global Control (CTRL) - 0x000	92

13.4.2 Global Status (STATUS) - 0x004	92

13.4.3 Channel Control (CHn_CTRL) - 0x040 + n*0x10	92

13.4.4 Channel Status (CHn_STATUS) - 0x044 + n*0x10	92

13.4.5 Channel Observation Control (OBS_CTRL) - 0x0C0	93

13.4.6 Channel Observation Flags (OBS_FLAGS) - 0x0C4	93

13.4.7 Channel Observation Data (OBS_DATA0, OBS_DATA1) - 0x0C8, 0x0CC	94

13.5 Access Timing	95

13.5.1 Write Access	95

13.5.2 Read Access	95

13.6 Kick-off Blocking	96

13.7 Launching Channels Together (KICK_ENABLE)	97

14 Monitoring Interface	98

14.1 Overview	98

14.2 Signal Summary	99

14.3 Packet Format	100

14.4 Monitoring Architecture	101

14.4.1 Descriptor Path Monitoring (Both Ends)	101

14.4.2 Global Monitor Enable Methodology	101

14.5 Event Types	102

14.6 Event Configuration	103

14.6.1 Configurable Events	103

14.6.2 Event Priority	103

14.7 Bulk-Trace Compression	104

14.8 Backpressure Handling	106

14.8.1 Flow Control	106

14.8.2 Recommendations	106

14.9 Typical MonBus Downstream	107

14.10 Example Event Sequence	108

15 Throughput Targets	109

15.1 Theoretical Maximum	109

15.1.1 Per-Interface Bandwidth	109

15.1.2 Aggregate Bandwidth	109

15.2 Practical Throughput	110

15.2.1 Single-Channel Performance	110

15.2.2 Multi-Channel Performance	110

15.2.3 Measured on the Genesys 2: Shared and Per-Channel Descriptor Engines	110

15.3 Throughput Limiting Factors	112

15.3.1 Memory System Factors	112

15.3.2 STREAM Internal Factors	112

15.4 Performance Targets	113

15.4.1 Design Targets	113

15.4.2 Verification Criteria	113

15.5 Descriptor Chain Efficiency	114

15.5.1 Descriptor Overhead	114

15.5.2 Chaining Efficiency	114

15.5.3 Figure 5.1.1: Descriptor Chain Fetch Overlap	114

15.5.4 Read-Ahead Descriptor Prefetch (Cross-Descriptor Streaming)	114

16 Latency Characteristics	116

16.1 Latency Components	116

16.1.1 End-to-End Latency Breakdown	116

16.1.2 Total Latency Formula	116

16.2 First-Byte Latency	117

16.3 Latency by Transfer Size	118

16.3.1 Small Transfers	118

16.3.2 Large Transfers	118

16.4 Latency Variability	119

16.4.1 Sources of Variability	119

16.4.2 Worst-Case Latency	119

16.5 Latency Optimization	120

16.5.1 Software Recommendations	120

16.5.2 Hardware Configuration	120

16.6 Interrupt Latency	121

17 Resource Estimates	122

17.1 Measured Core Area	122

17.2 Measured Board Build (Genesys 2)	123

17.3 FPGA Resource Summary (Estimates)	125

17.4 Resource Breakdown by Block	126

17.4.1 APB Configuration Slave	126

17.4.2 Descriptor Engine	126

17.4.3 Scheduler (8 channels)	126

17.4.4 Channel Arbiter	126

17.4.5 AXI Read Engine	126

17.4.6 SRAM Buffer	127

17.4.7 AXI Write Engine	127

17.4.8 MonBus Reporter	127

17.5 SRAM Sizing	128

17.6 Scaling with Parameters	129

17.6.1 Channel Count Impact	129

17.6.2 Data Width Impact	129

17.7 ASIC Estimates	130

17.7.1 Power Estimates	130

17.8 Timing Estimates	131

17.8.1 Target Frequencies	131

17.8.2 Critical Paths	131

18 System Requirements	132

18.1 Interface Requirements	132

18.1.1 APB Interface	132

18.1.2 AXI Interfaces	132

18.1.3 MonBus Interface	132

18.2 Memory Requirements	133

18.2.1 Descriptor Memory	133

18.2.2 Data Memory	133

18.3 System Bandwidth Allocation	134

18.3.1 AXI Interconnect Requirements	134

18.3.2 Interconnect Recommendations	134

18.4 Interrupt Requirements	135

18.4.1 Interrupt Controller	135

18.4.2 Interrupt Handling	135

18.5 Address Space Requirements	136

18.5.1 STREAM Register Space	136

18.5.2 Memory Map Constraints	136

18.6 Software Requirements	137

18.6.1 Driver Requirements	137

18.6.2 Programming Sequence	137

19 Clocking and Reset	138

19.1 Clock Requirements	138

19.1.1 Primary Clock (aclk)	138

19.1.2 Clock Relationship	138

19.1.3 Figure 6.2.1: Clock Domains	138

19.2 Reset Requirements	140

19.2.1 Reset Signal (aresetn)	140

19.2.2 Reset Sequence	140

19.2.3 Reset Behavior	140

19.3 Power Management	141

19.3.1 Clock Gating Support	141

19.3.2 Low Power Recommendations	141

19.4 Timing Constraints	142

19.4.1 Clock Period Constraints	142

19.4.2 False Path Constraints	142

19.5 CDC Considerations	143

19.5.1 Single Clock Domain	143

19.5.2 CDC Recommendations	143

20 Verification Strategy	144

20.1 Verification Approach	144

20.1.1 Multi-Level Verification	144

20.2 Block-Level Verification	145

20.2.1 Test Categories	145

20.2.2 Coverage Metrics	145

20.3 Integration-Level Verification	146

20.3.1 Interface Verification	146

20.3.2 Multi-Channel Scenarios	146

20.4 System-Level Verification	147

20.4.1 End-to-End Test Cases	147

20.4.2 Performance Verification	147

20.5 Testbench Architecture	148

20.5.1 CocoTB-Based Verification	148

20.5.2 Figure 6.3.1: Testbench Architecture	148

20.5.3 Key Testbench Components	148

20.6 Verification Environment	149

20.6.1 Simulator Support	149

20.6.2 Test Execution	149

20.7 Formal Verification	150

20.7.1 Properties for Formal	150

20.7.2 Formal Tool Usage	150

20.8 Sign-Off Criteria	151

20.8.1 Block-Level Sign-Off	151

20.8.2 Integration Sign-Off	151

20.8.3 System Sign-Off	151

21 STREAM Specification Index	152

21.1 Document Organization	153

21.1.1 Front Matter	153

21.1.2 Chapter 1: Overview	153

21.1.3 Chapter 2: Functional Blocks	153

21.1.4 Chapter 3: Interfaces	153

21.1.5 Chapter 4: Registers	153

21.1.6 Chapter 5: Programming	153

21.1.7 Chapter 6: Configuration Reference	154

21.2 Quick Reference	155

21.2.1 Functional Unit Blocks (FUB)	155

21.2.2 Integration Blocks (MAC)	156

21.3 Performance Modes (AXI Engines)	158

21.3.1 Holistic Overview: V1/V2/V3 Working Together	158

21.3.2 V1 - Low Performance (Single Outstanding Transaction)	158

21.3.3 V2 - Medium Performance (Command Pipelined)	158

21.3.4 V3 - High Performance (Out-of-Order Completion)	159

21.3.5 Performance Comparison Summary	159

21.4 Clock and Reset Summary	160

21.4.1 Clock Domains	160

21.4.2 Reset Signals	160

21.5 Interface Summary	161

21.5.1 External Interfaces	161

21.5.2 Internal Buses	161

21.6 Area Estimates	162

21.6.1 By Performance Mode	162

21.6.2 Breakdown (Low Performance)	162

21.7 Related Documentation	164

21.8 Specification Conventions	165

21.8.1 Signal Naming	165

21.8.2 Parameter Naming	165

21.8.3 State Machine Naming	165

22 Product Requirements Document (PRD)	166

22.1 STREAM - Scatter-gather Transfer Rapid Engine for AXI Memory	166

22.2 1. Executive Summary	167

22.2.1 1.1 Quick Stats	167

22.2.2 1.2 Project Goals	167

22.3 2. Key Design Principles	168

22.3.1 2.1 Simplifications from RAPIDS	168

22.3.2 2.2 Tutorial-Friendly Features	168

22.4 3. Architecture Overview	169

22.4.1 3.1 Top-Level Block Diagram	169

22.4.2 3.2 Data Flow	169

22.5 4. Interfaces	170

22.5.1 4.1 External Interfaces	170

22.5.2 4.2 Descriptor Format	170

22.6 5. Key Components	172

22.6.1 5.1 Descriptor Engine (APB-Only for STREAM)	172

22.6.2 5.2 Scheduler Group (Integration Wrapper)	172

22.6.3 5.3 Scheduler (Simplified from RAPIDS)	174

22.6.4 5.3 AXI Read Engine (Streaming Pipeline - NO FSM)	174

22.6.5 5.4 AXI Write Engine (Streaming Pipeline - NO FSM)	175

22.6.6 5.5 SRAM Buffering	176

22.7 6. Configuration and Control	177

22.7.1 6.1 APB Register Map	177

22.7.2 6.2 Channel Configuration	177

22.8 7. Resource Sharing and Arbitration	178

22.8.1 7.1 Shared Resources	178

22.8.2 7.2 Arbitration Strategy	178

22.9 8. Error Detection and Recovery	179

22.9.1 8.1 Error Types	179

22.9.2 8.2 Error Recovery	179

22.10 9. MonBus Integration	180

22.10.1 9.1 Standard MonBus Format	180

22.10.2 9.2 STREAM Event Codes	180

22.10.3 9.3 Default Configuration	180

22.11 10. Design Constraints	181

22.11.1 10.1 Tutorial Constraints (Intentional Simplifications)	181

22.11.2 10.2 Implementation Constraints	181

22.12 11. Verification Strategy	182

22.12.1 11.1 Test Organization	182

22.12.2 11.2 Test Levels	182

22.13 12. Performance Characteristics	183

22.13.1 12.1 Throughput by Engine Version	183

22.13.2 12.2 Latency	183

22.13.3 12.3 Resource Utilization by Engine Version	183

22.14 13. Development Roadmap	185

22.14.1 13.1 Phase 1: Foundation (Current)	185

22.14.2 13.2 Phase 2: Core Blocks	185

22.14.3 13.3 Phase 3: Data Path	185

22.14.4 13.4 Phase 4: Integration	185

22.14.5 13.5 Phase 5: Multi-Channel	185

22.14.6 13.6 Phase 6: Advanced Engines (Future - V2/V3)	185

22.15 14. Educational Value	187

22.15.1 14.1 Learning Objectives	187

22.15.2 14.2 Progression Path	187

22.16 15. Success Criteria	188

22.16.1 15.1 Functional	188

22.16.2 15.2 Quality	188

22.16.3 15.3 Performance	188

22.17 16. Open Questions (For Review)	189

22.17.1 16.1 Descriptor Engine Adaptation	189

22.17.2 16.2 AXI Descriptor Master	189

22.17.3 16.3 Channel Arbitration	189

22.17.4 16.4 SRAM Partitioning	189

22.18 16.2 PDF Generation Location	190

22.19 17. References	191

22.19.1 17.1 Internal Documentation	191

22.19.2 17.2 External References	191

22.20 Navigation	192

23 Claude Code Guide: STREAM Subsystem	193

23.1 Quick Context	194

23.2 📖 Global Requirements Reference	195

23.3 Critical Rules for This Subsystem	196

23.3.1 Rule #0.1: TUTORIAL FOCUS - Intentional Simplifications	196

23.3.2 Rule #0.1: Testbench Location and Test Structure (MANDATORY)	196

23.3.3 Rule #0.2: Three Mandatory TB Methods (MANDATORY)	197

23.3.4 Rule #1: REUSE from RAPIDS Where Appropriate	197

23.3.5 Rule #2: Descriptor Format is DIFFERENT from RAPIDS	197

23.3.6 Rule #3: Know the Shared Resources	198

23.4 Architecture Quick Reference	199

23.4.1 Block Organization	199

23.5 Common User Questions and Responses	201

23.5.1 Q: “How is STREAM different from RAPIDS?”	201

23.5.2 Q: “How do I kick off a transfer?”	201

23.5.3 Q: “How many channels can I use?”	202

23.5.4 Q: “What’s the descriptor format?”	202

23.5.5 Q: “How do I run STREAM tests?”	203

23.6 Integration Patterns	204

23.6.1 Pattern 1: Basic STREAM Instantiation	204

23.6.2 Pattern 2: Descriptor Creation (Software Model)	205

23.6.3 Pattern 3: MonBus Integration	205

23.7 Anti-Patterns to Catch	207

23.7.1 ✗ Anti-Pattern 1: Adding Alignment Fixup	207

23.7.2 ✗ Anti-Pattern 2: Using Length in Bytes	207

23.7.3 ✗ Anti-Pattern 3: Circular Buffer Descriptors	207

23.7.4 ✗ Anti-Pattern 4: Assuming Exclusive Channel Access	207

23.8 Debugging Workflow	209

23.8.1 Issue: Descriptor Not Fetched	209

23.8.2 Issue: Data Transfer Stalls	209

23.8.3 Issue: Chained Descriptors Not Following	209

23.9 Testing Guidance	210

23.9.1 Test Organization	210

23.9.2 Test Levels	210

23.10 Key Documentation Links	211

23.10.1 Always Reference These	211

23.11 Quick Commands	212

23.12 Remember	213

23.13 PDF Generation Location	214

List of Figures

Figure 2.3.1: System Context	56

Figure 3.1.1: STREAM Whole Unit	62

Figure 3.2.1: Transfer Sequence	67

Figure 3.3.1: Channel Architecture	72

Figure 3.3.2: Channel FSM State Diagram	76

Figure 5.1.1: Descriptor Chain Fetch Overlap	114

Figure 6.2.1: Clock Domains	138

Figure 6.3.1: Testbench Architecture	148

List of Tables

Table 1: Reference Documents	26

Table 2: Terminology and Definitions	27

Table 3: Document Revision History	28

Table 4: Interrupt Signals	60

Table 5: Channel FSM States	76

Table 6: SRAM Buffering by Build	78

Table 7: Genesys 2 Board Results, SHARED_ENGINES 0 and 1	110

Table 8: Measured stream_core Area and Timing	122

Table 9: Post-Route Utilization, SHARED_ENGINES 0 and 1	123

# 1	Stream Has Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	STREAM Hardware Architecture Specification

Document Number: STREAM-HAS-001 Version: 0.99 Status: Draft Classification: Open Source - MIT License

## 2.2	Document Purpose

This Hardware Architecture Specification (HAS) provides a high-level architectural overview of the STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory) subsystem. It describes the system-level design, external interfaces, performance characteristics, and integration requirements without detailing internal implementation specifics.

Target Audience: - System architects evaluating STREAM for integration - Hardware engineers planning system-level integration - Software engineers developing drivers and firmware - Verification engineers planning system-level testing

Companion Documents: - STREAM Micro-Architecture Specification (MAS) - Detailed block-level implementation - STREAM Product Requirements Document (PRD) - Requirements and rationale

## 2.3	References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | STREAM MAS v0.90 | Micro-Architecture Specification |
| [REF-2] | STREAM PRD | Product Requirements Document |
| [REF-3] | ARM AMBA AXI4 | AXI4 Protocol Specification |
| [REF-4] | ARM AMBA APB | APB Protocol Specification |

## 2.4	Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface - ARM AMBA high-performance bus |
| APB | Advanced Peripheral Bus - ARM AMBA low-power configuration bus |
| Beat | Single data transfer on AXI bus (one clock cycle of valid data) |
| Burst | Sequence of consecutive beats forming a single AXI transaction |
| Channel | Independent DMA transfer context (STREAM is built with 8 or 16 channels) |
| Descriptor | 256-bit data structure defining a single DMA transfer operation |
| DMA | Direct Memory Access - data transfer without CPU involvement |
| HAS | Hardware Architecture Specification |
| MAS | Micro-Architecture Specification |
| MonBus | Monitor Bus - internal debug/trace event streaming interface |
| Scatter-Gather | DMA mode using linked descriptors for non-contiguous transfers |

## 2.5	Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2026-01-03 | seang | Initial HAS release |
| 0.91 | 2026-05-14 | seang | Sync to RTL changes since 2026-04-17 (commit be4e5a91); regenerate PDFs/DOCX. |
| 0.92 | 2026-06-05 | seang | Sync to RTL state at 2026-06-05 (17 commits since be4e5a91). Documents (1) the 64→128-bit monbus packet migration + new 64-bit side-band timestamp wire — referenced via docs/markdown/rtl-amba/includes/ocelus_package_spec.md, with cfg_ts_append_* removed and m_axil records locked to 3 beats; (2) new APB channel-observation register set (OBS_CTRL / OBS_FLAGS / OBS_DATA0 / OBS_DATA1) that exposes scheduler error stickies + timeout per channel; (3) BOTH-end descriptor-path monitoring with distinct (UNIT_ID, AGENT_ID) for fetch-side vs. consume-side, under the new per-port + global SV-parameter monitor methodology; (4) three RTL fixes — drain-ctrl stale-view race + post-flop wvalid gate (a82627af), registered w_arb_request for 8-channel timing closure (4e8f9e02), reset on SRAM avail outputs (b619eee9); (5) stream_core_mon duplicate cleanup (7291c4ef). |
| 0.93 | 2026-07-02 | seang | Monitor registers relocated to a separate stream_mon_regs regfile at 0x1000 (single APB slave); APB address decode widened to 13 bits / 8 KB so the monitor block is addressable (ch04_interfaces/02_apb4_slave.md). Companion MAS documents SCHED_TIMEOUT_LIMIT + the recoverable scheduler write-progress timeout and functional descriptor prefetch. |
| 0.94 | 2026-07-07 | seang | Documented the kick-burst fast-path interface (top-level i_kick_burst_mask / i_kick_burst_addr ports) that starts any subset of channels on a single clock cycle, bypassing the serial APB CHn_CTRL kick — added the interface signal table + semantics to ch04_interfaces/02_apb4_slave.md. Reference integration is the NexysA7 char harness (CH_KICK_ADDR shadow registers + KICK_GO bitmask CSR). |
| 0.95 | 2026-07-09 | seang | Documented read-ahead descriptor prefetch: the scheduler read side advances to the next chained legacy descriptor while the write side drains the current one, eliminating the per-descriptor boundary bubble (cross-descriptor streaming). Added a section to ch05_performance/01_throughput.md with on-silicon A/B (Nexys A7, single bitstream: 95.3% datapath util prefetch-on vs 76.1% off at 64-beat descriptors; row-major EXT 99.7% util bubble-free; transpose single-beat latency-bound). Runtime SCHED_CONFIG.RD_PREFETCH_EN, default on. |
| 0.96 | 2026-10-17 | seang | 8- or 16-channel builds (stream_top_ch8 / stream_top_ch16) and the 16-channel register map; one shared descriptor engine by default (SHARED_ENGINES), with measured area (8 channels 25.8k -> 20.7k LUTs; 16 channels 36.8k) in ch05_performance/03_resources.md. Corrected three pages that did not match the RTL: the channel FSM (six states, read and write concurrent in CH_XFER_DATA), SRAM buffering (SRAM_DEPTH is per channel, with no dynamic reallocation), and interrupts (one stream_irq, no per-channel vector). Diagrams ch02_system_context, ch03_whole_unit, ch03_channel_architecture and 05_channel_fsm redrawn. |
| 0.97 | 2026-10-17 | seang | Measured Genesys 2 board builds with SHARED_ENGINES 0 and 1: post-route utilization and timing in ch05_performance/03_resources.md (STREAM 30,781 -> 29,247 LUTs, 23,079 -> 20,654 flip-flops; BRAM and DSP unchanged), and the 40-configuration board sweep of both in ch05_performance/01_throughput.md (40 of 40 on both, line rate on both, the shared build one cycle sooner per run). |
| 0.98 | 2026-10-20 | seang | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |
| 0.99 | 2026-10-20 | seang | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |

Last Updated: 2026-10-17

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	Document Purpose

This Hardware Architecture Specification (HAS) defines the high-level architecture of the STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory) subsystem. It serves as the primary reference for:

- System Integration - Understanding STREAM’s external interfaces and system-level requirements

- Performance Planning - Evaluating throughput, latency, and resource characteristics

- Driver Development - Programming model and software interface requirements

- Verification Planning - System-level test scenarios and coverage requirements

This document complements the STREAM Micro-Architecture Specification (MAS), which provides detailed block-level implementation specifics.

## 3.2	Scope

### 3.2.1	In Scope

This specification covers:

- System Architecture

  - Block-level functional partitioning

  - Data flow through the subsystem

  - Multi-channel operation model

- External Interfaces

  - APB configuration slave interface

  - AXI4 master interfaces (descriptor fetch, data read, data write)

  - Monitor bus (MonBus) output interface

- Performance Characteristics

  - Throughput targets and limiting factors

  - Latency breakdown for typical operations

  - Resource estimates for FPGA/ASIC targets

- Integration Requirements

  - Clock and reset requirements

  - System-level constraints

  - Verification strategy overview

### 3.2.2	Out of Scope

This specification does not cover:

- Detailed Micro-Architecture - See STREAM MAS for block-level implementation

- RTL Implementation Details - Register-level timing, pipeline stages

- Testbench Architecture - See verification documentation

- Software Driver Implementation - See software programming guide

## 3.3	Design Philosophy

### 3.3.1	Tutorial-Focused Simplicity

STREAM is intentionally designed as a beginner-friendly DMA engine tutorial. Key simplifications include:

| Aspect | Simplification | Rationale |
| --- | --- | --- |
| Address Alignment | Aligned addresses only | Eliminates complex fixup logic |
| Transfer Length | Specified in beats | Simplifies datapath math |
| Descriptor Chaining | Linear chains only | No circular buffer management |
| Flow Control | Simple transaction limits | No credit management complexity |

These simplifications make STREAM an ideal learning platform while maintaining production-quality RTL practices.

### 3.3.2	Relationship to RAPIDS

STREAM is derived from the RAPIDS (Rapid AXI Programmable In-band Descriptor System) architecture but removes:

- Network interfaces (source/sink paths)

- Address alignment fixup logic

- Credit-based flow control

- Control read/write engines

This creates a pure memory-to-memory DMA engine suitable for understanding core DMA concepts.

## 3.4	Intended Audience

| Audience | Primary Use |
| --- | --- |
| System Architects | Evaluate STREAM for system integration |
| Hardware Engineers | Plan physical integration and timing closure |
| Software Engineers | Develop drivers and firmware |
| Verification Engineers | Plan system-level test coverage |

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Notation Conventions

### 4.1.1	Signal Naming

| Prefix/Suffix | Meaning | Example |
| --- | --- | --- |
| i_ | Input signal | i_clk, i_data |
| o_ | Output signal | o_valid, o_ready |
| r_ | Registered signal | r_state, r_count |
| w_ | Combinational wire | w_next_state |
| _n | Active-low signal | aresetn, rst_n |

### 4.1.2	Interface Prefixes

| Prefix | Protocol | Direction |
| --- | --- | --- |
| s_apb_ | APB slave | Inbound |
| m_axi_desc_ | AXI4 descriptor master | Outbound |
| m_axi_rd_ | AXI4 read master | Outbound |
| m_axi_wr_ | AXI4 write master | Outbound |
| monbus_ | Monitor bus | Outbound |

## 4.2	Diagram Conventions

### 4.2.1	Block Diagrams

- Rectangles - Functional blocks

- Arrows - Data flow direction

- Dashed Lines - Configuration/control paths

- Double Lines - High-bandwidth data paths

### 4.2.2	Timing Diagrams

- High - Logic 1

- Low - Logic 0

- X - Don’t care / undefined

- Hatched - Valid data window

## 4.3	Numeric Conventions

### 4.3.1	Units

| Unit | Meaning |
| --- | --- |
| Beat | Single data transfer (one clock cycle of valid data) |
| Burst | Sequence of consecutive beats |
| Transaction | Complete AXI transaction (address + all data beats) |

### 4.3.2	Address Notation

- All addresses are specified in bytes unless otherwise noted

- Address alignment requirements are specified relative to data width

- Example: 512-bit data width requires 64-byte (512/8) alignment

### 4.3.3	Size Notation

| Notation | Value |
| --- | --- |
| KB | 1,024 bytes |
| MB | 1,048,576 bytes |
| Kbit | 1,024 bits |

## 4.4	Requirement Notation

Requirements and constraints are identified using the following format:

| Prefix | Category |
| --- | --- |
| [REQ-x] | Functional requirement |
| [PERF-x] | Performance requirement |
| [INTEG-x] | Integration requirement |
| [CONST-x] | Design constraint |

## 4.5	Reference Format

Cross-references to other documents use the format:

- [MAS-x.y] - Reference to MAS section x.y

- [REF-n] - Reference to document in reference table

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

## 5.1	Acronyms

| Acronym | Definition |
| --- | --- |
| APB | Advanced Peripheral Bus |
| AXI | Advanced eXtensible Interface |
| AMBA | Advanced Microcontroller Bus Architecture |
| BFM | Bus Functional Model |
| CDC | Clock Domain Crossing |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out buffer |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| MAS | Micro-Architecture Specification |
| MonBus | Monitor Bus (internal debug/trace interface) |
| PRD | Product Requirements Document |
| RTL | Register Transfer Level |
| SG | Scatter-Gather |
| SRAM | Static Random Access Memory |
| STREAM | Scatter-gather Transfer Rapid Engine for AXI Memory |

## 5.2	Definitions

### 5.2.1	Architectural Terms

| Term | Definition |
| --- | --- |
| Beat | A single data transfer on an AXI bus, corresponding to one clock cycle where valid data is transferred. For a 512-bit data bus, one beat transfers 64 bytes. |
| Burst | A sequence of consecutive beats forming a single AXI transaction. STREAM supports burst lengths up to 256 beats (AXI4 maximum). |
| Channel | An independent DMA transfer context. STREAM is built with 8 or 16 channels, each capable of processing its own descriptor chain. |
| Descriptor | A 256-bit data structure in memory that defines a single DMA transfer operation, including source address, destination address, length, and chaining information. |
| Descriptor Chain | A linked list of descriptors where each descriptor points to the next, enabling complex scatter-gather transfer patterns. |
| Scatter-Gather | A DMA technique where data is transferred to/from non-contiguous memory locations using a list of descriptors. |

### 5.2.2	Interface Terms

| Term | Definition |
| --- | --- |
| APB Slave | STREAM’s configuration interface, receiving programming commands from the system processor. |
| AXI Master | STREAM’s memory interfaces for descriptor fetches and data transfers. Three masters: descriptor fetch, data read, data write. |
| MonBus | Monitor Bus - a 64-bit streaming interface for debug and performance monitoring packets. |
| Backpressure | Flow control mechanism where a receiver signals inability to accept data, causing the sender to pause. |

### 5.2.3	Transfer Terms

| Term | Definition |
| --- | --- |
| Aligned Address | An address that is a multiple of the data width in bytes. For 512-bit (64-byte) data width, aligned addresses are multiples of 64. |
| Kick-off | Initiating a DMA transfer by writing the first descriptor address to a channel’s control register. |
| Completion | The successful end of a descriptor chain, optionally generating an interrupt. |
| Chaining | Following the next_descriptor_ptr to fetch and process subsequent descriptors. |

## 5.3	Units and Measurements

| Term | Definition |
| --- | --- |
| Clock Cycle | One period of the primary system clock (aclk). |
| Latency | Time from initiating an operation to its completion, typically measured in clock cycles. |
| Throughput | Data transfer rate, typically measured in bytes per second or beats per cycle. |
| Utilization | Percentage of available bandwidth actually used for data transfer. |

## 5.4	Document-Specific Terms

| Term | Definition |
| --- | --- |
| External Interface | A signal or bus that crosses the STREAM subsystem boundary. |
| Internal Interface | A signal or bus between blocks within STREAM, not visible externally. |
| Configuration Register | An APB-accessible register for controlling STREAM behavior. |
| Status Register | An APB-readable register reporting STREAM operational state. |

Last Updated: 2026-10-17

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Use Cases

## 6.1	Primary Use Cases

### 6.1.1	Memory-to-Memory Data Movement

Description: Transfer data between two memory regions without CPU involvement.

Operation: 1. Software prepares descriptor(s) in memory with source/destination addresses 2. Software writes first descriptor address to channel control register 3. STREAM autonomously fetches descriptor and executes transfer 4. Optional interrupt notifies software of completion

Key Benefits: - CPU offloading for bulk data transfers - Deterministic transfer timing - Support for large transfer sizes via descriptor chaining

### 6.1.2	Scatter-Gather Operations

Description: Collect data from multiple non-contiguous source regions into a contiguous destination, or distribute contiguous source data to multiple destinations.

Gather Example:

Descriptor 0: src=0x1000, dst=0x8000, len=64 beats
Descriptor 1: src=0x3000, dst=0x8100, len=32 beats  (chained)
Descriptor 2: src=0x5000, dst=0x8180, len=16 beats  (chained, last)

Scatter Example:

Descriptor 0: src=0x8000, dst=0x1000, len=64 beats
Descriptor 1: src=0x8100, dst=0x3000, len=32 beats  (chained)
Descriptor 2: src=0x8180, dst=0x5000, len=16 beats  (chained, last)

### 6.1.3	Multi-Channel Concurrent Transfers

Description: Execute multiple independent transfer operations simultaneously.

Operation: - Up to 8 or 16 channels (by top) operate concurrently - Each channel processes its own descriptor chain - Shared AXI masters picked round-robin; each channel has its own SRAM FIFO

Use Cases: - Parallel DMA for multi-stream applications - Priority-based bandwidth allocation - Independent channel error isolation

## 6.2	Secondary Use Cases

### 6.2.1	Buffer Management

Description: Move data between system memory and local buffers for processing pipelines.

Pattern:

System Memory → STREAM → Processing Buffer → STREAM → System Memory

### 6.2.2	Memory Initialization

Description: Fill memory regions with pattern data using chained descriptors.

Note: Requires source buffer containing pattern data.

## 6.3	Target Applications

| Application | Channel Usage | Typical Transfer Size |
| --- | --- | --- |
| Video Frame Copy | 1-2 channels | 1-8 MB per frame |
| Audio Buffer Management | 1 channel | 4-64 KB per buffer |
| Data Collection | 4-8 channels | Variable |
| Memory Test Patterns | 1-4 channels | System-dependent |

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Key Features

## 7.1	Feature Summary

| Feature | Specification |
| --- | --- |
| Channels | 8 or 16 independent channels (stream_top_ch8 / stream_top_ch16) |
| Descriptor Size | 256 bits (32 bytes) |
| Data Width | Parameterizable (default 512 bits) |
| Address Width | 64 bits |
| Maximum Burst | 256 beats (AXI4) |
| Chaining | Unlimited descriptor chain depth |
| Arbitration | Priority-based with round-robin |

## 7.2	Descriptor-Based Operation

### 7.2.1	Single-Write Kick-off

A single APB write to a channel’s control register initiates an entire transfer sequence:

- Write descriptor address to CHn_CTRL register

- STREAM automatically fetches and processes descriptor

- Chained descriptors processed without software intervention

- Completion indicated via status register and optional interrupt

### 7.2.2	Autonomous Chaining

STREAM follows next_descriptor_ptr automatically:

- Non-zero pointer: Fetch next descriptor, continue processing

- Zero pointer: Chain complete, transition to idle

- Last flag: Explicit chain termination regardless of pointer

## 7.3	Multi-Channel Architecture

### 7.3.1	Channel Independence

Each channel maintains:

- Independent FSM state

- Separate descriptor chain pointer

- Individual error status

- Private completion interrupt

### 7.3.2	Resource Sharing

All channels share:

- Descriptor fetch AXI master

- Data read AXI master

- Data write AXI master

- Internal SRAM buffer

- MonBus reporter

### 7.3.3	Priority-Based Arbitration

- 8-bit priority field in each descriptor

- Higher priority descriptors serviced first

- Round-robin within same priority level

- Anti-starvation timeout mechanism

## 7.4	Data Path Features

### 7.4.1	Parameterizable Data Width

Compile-time configurable data width:

| Parameter | Description | Typical Values |
| --- | --- | --- |
| DATA_WIDTH | AXI data bus width | 128, 256, 512 bits |
| SRAM_DEPTH | Internal buffer depth | 1024, 2048, 4096 entries |

### 7.4.2	Aligned Address Requirement

[CONST-1] All source and destination addresses must be aligned to data width.

| Data Width | Alignment Requirement |
| --- | --- |
| 128 bits | 16-byte aligned |
| 256 bits | 32-byte aligned |
| 512 bits | 64-byte aligned |

This simplification eliminates complex alignment fixup logic for tutorial clarity.

## 7.5	Monitoring and Debug

### 7.5.1	MonBus Integration

64-bit monitor packets for:

- Transfer initiation events

- Descriptor fetch events

- Transfer completion events

- Error condition reporting

- Performance metrics

### 7.5.2	Status Visibility

APB-readable status registers provide:

- Channel state (idle, active, error)

- Descriptor count per channel

- Error codes and flags

- Performance counters (optional)

## 7.6	Error Handling

### 7.6.1	Error Detection

| Error Type | Detection | Response |
| --- | --- | --- |
| AXI read error | RRESP != OKAY | Stop chain, set error flag |
| AXI write error | BRESP != OKAY | Stop chain, set error flag |
| Invalid descriptor | Validation failure | Stop chain, set error flag |
| Address out of range | Limit check | Stop chain, set error flag |

### 7.6.2	Error Isolation

Errors in one channel do not affect other channels. Each channel has independent error status and recovery.

Last Updated: 2026-10-17

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	System Context

## 8.1	System Integration Overview

STREAM integrates into a typical SoC as a peripheral subsystem with the following connections:

### 8.1.1	Figure 2.3.1: System Context

System Context

Source: ch02_system_context.dot

## 8.2	External Connections

### 8.2.1	Configuration Path

| Connection | Protocol | Description |
| --- | --- | --- |
| APB Slave | APB3/APB4 | Configuration and control interface |

Functions: - Channel kick-off (write descriptor address) - Status monitoring - Interrupt control - Error handling

### 8.2.2	Data Paths

| Connection | Protocol | Width | Description |
| --- | --- | --- | --- |
| Descriptor Master | AXI4 | 256-bit | Fetch descriptors from memory |
| Read Data Master | AXI4 | Param | Read source data from memory |
| Write Data Master | AXI4 | Param | Write destination data to memory |

### 8.2.3	Monitoring Path

| Connection | Protocol | Width | Description |
| --- | --- | --- | --- |
| MonBus Master | MonBus | 64-bit | Debug/trace packet output |

## 8.3	Clock and Reset

### 8.3.1	Clock Domain

STREAM operates in a single clock domain:

| Signal | Description |
| --- | --- |
| aclk | Primary system clock, all interfaces synchronous |
| aresetn | Active-low asynchronous reset |

[INTEG-1] All external interfaces must be synchronous to aclk.

### 8.3.2	Reset Requirements

- Asynchronous assertion, synchronous de-assertion

- Minimum reset pulse width: 2 clock cycles

- All state machines return to idle on reset

- SRAM contents undefined after reset

## 8.4	Memory Map Requirements

### 8.4.1	Descriptor Memory

Descriptors must be placed in memory accessible via the descriptor fetch AXI master:

- Alignment: 32-byte aligned (256-bit descriptor size)

- Accessibility: Read access required

- Coherency: Software must ensure descriptors are coherent before kick-off

### 8.4.2	Data Memory

Source and destination regions must be accessible via the appropriate AXI master:

- Source: Read access via read data master

- Destination: Write access via write data master

- Alignment: Per data width (see Key Features)

## 8.5	Interrupt Integration

### 8.5.1	Interrupt Signals

Interrupt Signals

| Signal | Description |
| --- | --- |
| stream_irq | The single interrupt output: the error/interrupt FIFO of monbus_axil4_axil4_group is not empty. Tied to 0 when USE_AXI_MONITORS = 0 |

There is no per-channel interrupt vector, at 8 or at 16 channels.

### 8.5.2	Interrupt Conditions

- A MonBus packet the group classifies as an error or interrupt (for example a descriptor completion with the descriptor’s interrupt flag, or a channel error) is pushed into the error FIFO, which raises stream_irq

- Software reads the FIFO through s_axil_err to find the channel and cause, and the interrupt drops when the FIFO is empty

- Per-channel completion is also visible by polling AXI_RD_COMPLETE, AXI_WR_COMPLETE and CHANNEL_IDLE

## 8.6	Power Considerations

### 8.6.1	Clock Gating

STREAM supports clock gating when all channels are idle:

- Monitor all_channels_idle status bit

- External clock gate can disable aclk when idle

- Re-enable clock before writing kick-off register

### 8.6.2	Power States

| State | Channels | Clock | Power |
| --- | --- | --- | --- |
| Active | >= 1 busy | Running | Full |
| Idle | All idle | Running | Reduced (no switching) |
| Gated | All idle | Stopped | Minimal (leakage only) |

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Block Diagram

## 9.1	Top-Level Architecture

### 9.1.1	Figure 3.1.1: STREAM Whole Unit

STREAM Whole Unit

Source: ch03_whole_unit.dot

## 9.2	Functional Block Summary

| Block | Function | Key Interfaces |
| --- | --- | --- |
| APB Config Slave + Registers | apb4_slave_cdc (or apb4_slave), stream_regs (stream_regs_16ch in the 16-channel top), stream_config_block | APB slave, kick-off signals |
| Descriptor Engine | Fetch and parse descriptors. One shared engine serves every channel by default (SHARED_ENGINES = 1) | AXI read, per-channel descriptor output |
| Scheduler | Coordinate transfer phases | Control signals to all blocks |
| Descriptor AR Arbiter | SHARED_ENGINES = 0 only: round-robin over the per-channel descriptor engines (no priority). The shared engine makes the same pick internally | Grant signals |
| AXI Read Engine | Read source data to SRAM | AXI read master |
| SRAM Buffer | Temporary data storage | Read/write ports |
| AXI Write Engine | Write SRAM data to destination | AXI write master |
| Monitors + MonBus group | Monitor-lites, monbus_arbiter, monbus_axil4_axil4_group (USE_AXI_MONITORS = 1) | m_axil_mon, s_axil_err, stream_irq |
| Performance Profiler | Per-channel activity timestamps or elapsed times | Register read-back |

## 9.3	Block Interactions

### 9.3.1	Control Flow

APB Write -> APB Config -> Descriptor Engine -> Scheduler -> Data Path, with AXI Desc Master fetching descriptors.

### 9.3.2	Data Flow

Memory -> AXI Read Engine -> SRAM Buffer -> AXI Write Engine -> Memory

### 9.3.3	Monitoring Flow

Scheduler groups and monitor-lites -> monbus_arbiter -> monbus_axil4_axil4_group -> error FIFO (s_axil_err, stream_irq) and capture writes (m_axil_mon)

## 9.4	Parameterization

| Parameter | Description | Default | Range |
| --- | --- | --- | --- |
| NUM_CHANNELS | Number of DMA channels | 8 (stream_top_ch8), 16 (stream_top_ch16) | 1-16 (8-channel register map up to 8) |
| SHARED_ENGINES | One shared descriptor engine (1) or one per channel (0) | 1 | 0, 1 |
| DATA_WIDTH | AXI data bus width | 512 | 128, 256, 512 |
| ADDR_WIDTH | Address width | 64 | 32, 64 |
| SRAM_DEPTH | Buffer depth in entries, per channel | 4096 | 256-8192 |
| DESC_FIFO_DEPTH | Descriptor FIFO depth | 4 | 2-8 |

Last Updated: 2026-10-17

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Data Flow

## 10.1	Transfer Sequence Overview

A complete STREAM transfer follows this sequence:

### 10.1.1	Figure 3.2.1: Transfer Sequence

Transfer Sequence

Source: ch03_data_flow.dot

## 10.2	Phase Details

### 10.2.1	Phase 1: Kick-off

Software writes the first descriptor address to the channel control register:

- APB write triggers channel state transition from IDLE to ACTIVE

- Descriptor address stored in channel’s descriptor pointer register

- Kick-off signal sent to Descriptor Engine

### 10.2.2	Phase 2: Descriptor Fetch

Descriptor Engine issues AXI read transaction:

- Address: From kick-off or next_descriptor_ptr

- Size: 256 bits (single beat or 2-beat depending on bus width)

- Response: Descriptor data captured on valid read data

### 10.2.3	Phase 3: Descriptor Parse

Descriptor fields extracted and validated:

| Field | Validation |
| --- | --- |
| valid | Must be 1 |
| src_addr | Alignment check, range check |
| dst_addr | Alignment check, range check |
| length | Non-zero, within limits |

### 10.2.4	Phase 4: Arbitration

Channel requests access to shared data path:

- Multiple channels may be active simultaneously

- The read and write engines grant channels round-robin (no priority)

- Granted channel proceeds to read phase

### 10.2.5	Phase 5: Read Phase

AXI Read Engine fetches source data:

- Issues AXI read bursts to source address

- Data written to SRAM buffer

- Continues until length beats transferred

- Read/write to SRAM can overlap (ping-pong or streaming)

### 10.2.6	Phase 6: Write Phase

AXI Write Engine sends data to destination:

- Reads data from SRAM buffer

- Issues AXI write bursts to destination address

- Awaits write responses

- Continues until all data written

### 10.2.7	Phase 7: Chain Check

Scheduler determines next action:

- If last == 1 or next_descriptor_ptr == 0: Chain complete

- Otherwise: Fetch next descriptor (return to Phase 2)

## 10.3	Concurrent Operation

### 10.3.1	Multi-Channel Concurrency

Multiple channels can be at different phases simultaneously. For example: - Ch0: Kick -> Fetch -> Read -> Write -> Complete - Ch1: (offset) Kick -> Fetch -> Arb Wait -> Read -> Write - Ch2: (offset) Kick -> Fetch -> Arb Wait -> Read

### 10.3.2	Pipeline Overlap

Within a single channel, read and write phases can overlap. As read beats 0-63 complete, write beats 0-63 can begin while read continues with beats 64-127.

The SRAM buffer enables this overlap by storing in-flight data.

## 10.4	Data Path Bandwidth

### 10.4.1	Theoretical Maximum

With 512-bit data width at 200 MHz:

- Per-direction: 512 bits x 200 MHz = 102.4 Gbps = 12.8 GB/s

- Bidirectional: 25.6 GB/s (simultaneous read + write)

### 10.4.2	Practical Considerations

Actual throughput affected by:

- AXI arbitration latency

- Memory access latency

- SRAM buffer depth (limits outstanding data)

- Channel arbitration overhead

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Multi-Channel Architecture

## 11.1	Channel Organization

STREAM supports 8 or 16 independent DMA channels (stream_top_ch8 / stream_top_ch16), each processing its own descriptor chain:

### 11.1.1	Figure 3.3.1: Channel Architecture

Channel Architecture

Source: ch03_channel_architecture.dot

## 11.2	Per-Channel Resources

Each channel maintains independent state:

| Resource | Description | Size |
| --- | --- | --- |
| FSM State | Current operation phase | 4 bits |
| Descriptor Pointer | Current/next descriptor address | 64 bits |
| Transfer Counter | Beats remaining in current transfer | 32 bits |
| Error Status | Error flags and codes | 8 bits |
| Priority | Current descriptor priority | 8 bits |

## 11.3	Shared Resources

All channels share the following resources:

| Resource | Arbitration | Capacity |
| --- | --- | --- |
| Descriptor AXI Master | Round-robin pick in the shared descriptor engine | One fetch in flight per channel; fetches from different channels overlap (ARID = channel) |
| Data Read AXI Master | Round-robin over channels with SRAM space | Configurable outstanding |
| Data Write AXI Master | Round-robin over channels with SRAM data | Configurable outstanding |
| SRAM Buffer | None: one FIFO per channel | SRAM_DEPTH entries per channel |
| MonBus | Round-robin (monbus_arbiter) | One packet per grant |

## 11.4	Arbitration Scheme

### 11.4.1	Round-Robin Arbitration

Every shared master is behind a plain round-robin pick (arbiter_round_robin in acknowledge mode): the grant moves to the next requesting channel after the last winner. There is no priority logic. The descriptor’s 8-bit priority field ([207:200]) is carried but not used for arbitration, and there is no anti-starvation timer or preemption; round-robin itself bounds every channel’s wait to one turn of the other channels. The read and write engines mask a channel out of their pick until its SRAM FIFO has room (read) or data (write) for a burst.

### 11.4.2	Arbitration Points

Arbitration occurs at:

- Descriptor Fetch: When a channel has a descriptor address to fetch. In the default build (SHARED_ENGINES = 1) the pick is inside the one shared descriptor engine; with SHARED_ENGINES = 0 each channel has its own engine and an arbiter merges their AR requests

- Read Phase Start: When channel ready to read source data

- Write Phase Start: When channel ready to write destination

## 11.5	Channel States

Each channel FSM traverses these states:

### 11.5.1	Figure 3.3.2: Channel FSM State Diagram

Channel FSM State Diagram

Source: 05_channel_fsm.mmd

Channel FSM States

| State | Description | Transitions |
| --- | --- | --- |
| CH_IDLE | No active transfer | Descriptor ready and channel enabled -> CH_FETCH_DESC |
| CH_FETCH_DESC | Latch and check the descriptor (one cycle) | Valid bit set -> CH_XFER_DATA; clear -> CH_ERROR |
| CH_XFER_DATA | Read (source to SRAM) and write (SRAM to destination) run concurrently | All beats read and all writes committed -> CH_COMPLETE |
| CH_COMPLETE | Descriptor done; check for a chain | Chained -> CH_NEXT_DESC; last, once every write has committed -> CH_IDLE |
| CH_NEXT_DESC | Wait for the next chained descriptor | Descriptor ready -> CH_FETCH_DESC |
| CH_ERROR | Sticky fault (engine error, invalid descriptor, escalated timeout) | Channel reset -> CH_IDLE |

A disabled channel does not take a descriptor in CH_IDLE. A descriptor already fetched for it waits in the engine’s FIFO until the channel is enabled.

——-|————-|————-| | IDLE | No active transfer | Kick-off -> FETCH | | FETCH | Fetching descriptor | Complete -> PARSE | | PARSE | Validating descriptor | Valid -> ARB_READ, Invalid -> ERROR | | ARB_READ | Awaiting read grant | Grant -> READ | | READ | Reading source data | Complete -> ARB_WRITE | | ARB_WRITE | Awaiting write grant | Grant -> WRITE | | WRITE | Writing destination | Complete -> CHAIN | | CHAIN | Checking for next | Next -> FETCH, Last -> COMPLETE | | COMPLETE | Transfer done | IRQ sent -> IDLE | | ERROR | Error condition | SW clear -> IDLE |

## 11.6	SRAM Buffering

Each channel has its own SRAM FIFO (one sram_controller_unit per channel), SRAM_DEPTH entries deep. The top-level parameter is a per-channel depth, not a pool divided among the channels, and it is fixed at build time. An idle channel’s FIFO is not lent to a busy one.

SRAM Buffering by Build

| Build | FIFOs | Entries per channel | Total entries |
| --- | --- | --- | --- |
| stream_top_ch8 | 8 | SRAM_DEPTH | 8 x SRAM_DEPTH |
| stream_top_ch16 | 16 | SRAM_DEPTH | 16 x SRAM_DEPTH |

The read engine issues a burst for a channel only when that channel’s FIFO has room for it. The write engine issues only when the FIFO holds the data. One channel’s full or empty FIFO therefore never stalls another channel.

## 11.7	Error Isolation

Errors are isolated to the affected channel:

- Error in Channel N does not affect the other channels (0..NUM_CHANNELS-1)

- Shared resource errors logged but don’t block other channels

- Software must clear error and re-kick to resume

Last Updated: 2026-10-17

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI Master Interfaces

## 12.1	Overview

STREAM includes three AXI4 master interfaces for memory access:

| Interface | Purpose | Data Width | Address Width |
| --- | --- | --- | --- |
| m_axi_desc_* | Descriptor fetch | 256 bits | 64 bits |
| m_axi_rd_* | Source data read | Parameterizable | 64 bits |
| m_axi_wr_* | Destination write | Parameterizable | 64 bits |

## 12.2	Descriptor Fetch Master

### 12.2.1	Signal Summary

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_desc_arid | Output | ID_WIDTH | Read transaction ID |
| m_axi_desc_araddr | Output | 64 | Read address |
| m_axi_desc_arlen | Output | 8 | Burst length (0 = 1 beat) |
| m_axi_desc_arsize | Output | 3 | Beat size (5 = 32 bytes) |
| m_axi_desc_arburst | Output | 2 | Burst type (INCR) |
| m_axi_desc_arvalid | Output | 1 | Address valid |
| m_axi_desc_arready | Input | 1 | Address ready |
| m_axi_desc_rid | Input | ID_WIDTH | Read data ID |
| m_axi_desc_rdata | Input | 256 | Read data |
| m_axi_desc_rresp | Input | 2 | Read response |
| m_axi_desc_rlast | Input | 1 | Last beat |
| m_axi_desc_rvalid | Input | 1 | Read data valid |
| m_axi_desc_rready | Output | 1 | Read data ready |

### 12.2.2	Transaction Characteristics

| Characteristic | Value | Notes |
| --- | --- | --- |
| Burst Length | 1 beat | Single 256-bit descriptor |
| Burst Type | INCR | Fixed increment |
| Outstanding | 1-2 | Configurable |
| Ordering | In-order | Single ID used |

### 12.2.3	Address Alignment

Descriptor addresses must be 32-byte aligned (256-bit boundary).

## 12.3	Data Read Master

### 12.3.1	Signal Summary

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_arid | Output | ID_WIDTH | Read transaction ID |
| m_axi_rd_araddr | Output | 64 | Read address |
| m_axi_rd_arlen | Output | 8 | Burst length |
| m_axi_rd_arsize | Output | 3 | Beat size |
| m_axi_rd_arburst | Output | 2 | Burst type (INCR) |
| m_axi_rd_arvalid | Output | 1 | Address valid |
| m_axi_rd_arready | Input | 1 | Address ready |
| m_axi_rd_rid | Input | ID_WIDTH | Read data ID |
| m_axi_rd_rdata | Input | DATA_WIDTH | Read data |
| m_axi_rd_rresp | Input | 2 | Read response |
| m_axi_rd_rlast | Input | 1 | Last beat |
| m_axi_rd_rvalid | Input | 1 | Read data valid |
| m_axi_rd_rready | Output | 1 | Read data ready |

### 12.3.2	Transaction Characteristics

| Characteristic | Value | Notes |
| --- | --- | --- |
| Burst Length | 1-256 beats | AXI4 maximum |
| Burst Type | INCR | Fixed increment |
| Outstanding | Configurable | 4-16 typical |
| Ordering | In-order | Channel-based ID tagging |

### 12.3.3	Burst Generation

Large transfers are broken into maximum-length bursts:

| Transfer Size | Burst Count | Burst Lengths |
| --- | --- | --- |
| 256 beats | 1 | 256 |
| 512 beats | 2 | 256, 256 |
| 300 beats | 2 | 256, 44 |

## 12.4	Data Write Master

### 12.4.1	Signal Summary

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_awid | Output | ID_WIDTH | Write transaction ID |
| m_axi_wr_awaddr | Output | 64 | Write address |
| m_axi_wr_awlen | Output | 8 | Burst length |
| m_axi_wr_awsize | Output | 3 | Beat size |
| m_axi_wr_awburst | Output | 2 | Burst type (INCR) |
| m_axi_wr_awvalid | Output | 1 | Address valid |
| m_axi_wr_awready | Input | 1 | Address ready |
| m_axi_wr_wdata | Output | DATA_WIDTH | Write data |
| m_axi_wr_wstrb | Output | DATA_WIDTH/8 | Byte strobes |
| m_axi_wr_wlast | Output | 1 | Last beat |
| m_axi_wr_wvalid | Output | 1 | Write data valid |
| m_axi_wr_wready | Input | 1 | Write data ready |
| m_axi_wr_bid | Input | ID_WIDTH | Response ID |
| m_axi_wr_bresp | Input | 2 | Write response |
| m_axi_wr_bvalid | Input | 1 | Response valid |
| m_axi_wr_bready | Output | 1 | Response ready |

### 12.4.2	Transaction Characteristics

| Characteristic | Value | Notes |
| --- | --- | --- |
| Burst Length | 1-256 beats | Matches read bursts |
| Burst Type | INCR | Fixed increment |
| Outstanding | Configurable | 4-16 typical |
| Write Strobes | All-ones | Aligned transfers only |

## 12.5	Common AXI Attributes

### 12.5.1	Supported Features

| Feature | Descriptor | Read | Write |
| --- | --- | --- | --- |
| INCR Burst | Yes | Yes | Yes |
| WRAP Burst | No | No | No |
| FIXED Burst | No | No | No |
| Exclusive | No | No | No |
| Locked | No | No | No |

### 12.5.2	Error Handling

- SLVERR/DECERR on any channel stops the current transfer

- Error logged in channel status register

- Subsequent descriptors in chain not processed

- Software intervention required to clear and resume

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB Configuration Interface

## 13.1	Overview

STREAM provides an APB slave interface for software configuration and control. The interface supports:

- Channel kick-off (write descriptor address to start transfer)

- Status monitoring

- Interrupt control

- Error handling

## 13.2	Signal Summary

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_apb_paddr | Input | 13 | Address bus (8 KB space; reaches monitor block at 0x1000) |
| s_apb_psel | Input | 1 | Peripheral select |
| s_apb_penable | Input | 1 | Enable phase |
| s_apb_pwrite | Input | 1 | Write enable |
| s_apb_pwdata | Input | 32 | Write data |
| s_apb_pstrb | Input | 4 | Byte strobes |
| s_apb_pready | Output | 1 | Ready signal |
| s_apb_prdata | Output | 32 | Read data |
| s_apb_pslverr | Output | 1 | Slave error |

## 13.3	Register Map Overview

Two register maps exist: stream_regs for stream_top_ch8 and stream_regs_16ch for stream_top_ch16. The 16-channel map adds CH8..CH15_CTRL_{LOW,HIGH} at 0x040-0x07C, widens the per-channel fields to [15:0], and moves CH_STATE to 0x180. The full list of differences is in the MAS register-map chapter. The table below is the 8-channel map.

| Offset | Register | Access | Description |
| --- | --- | --- | --- |
| 0x000 | CH0_CTRL_LOW | RW | Channel 0 descriptor address [31:0] (staged) |
| 0x004 | CH0_CTRL_HIGH | RW | Channel 0 descriptor address [63:32] (staged) |
| 0x100 | GLOBAL_CTRL | RW | Global enable / reset |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE |
| 0x120 | CHANNEL_ENABLE | RW | Per-channel enable [7:0] ([15:0] in the 16-channel map) |
| 0x124 | CHANNEL_RESET | RW | Per-channel reset [7:0] ([15:0] in the 16-channel map) |
| 0x128 | KICK_ENABLE | RW | Launch staged addresses, one bit per channel |
| 0x140 | CHANNEL_IDLE | RO | Per-channel idle [7:0] ([15:0] in the 16-channel map) |
| 0x148 | SCHEDULER_IDLE | RO | Scheduler idle |
| 0x150 | CH_STATE0_STATE | RO | Channel 0 scheduler state [6:0], one-hot (0x180 in the 16-channel map) |
| 0x170 | SCHED_ERROR | RO | Scheduler error status |
| 0x174 | AXI_RD_COMPLETE | RO | Read completion, one bit per channel |
| 0x178 | AXI_WR_COMPLETE | RO | Write completion, one bit per channel |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | Scheduler timeout cycles |
| 0x204 | SCHED_CONFIG | RW | Scheduler enable and packet enables |
| 0x220 | DESCENG_CONFIG | RW | Descriptor engine enable / prefetch / FIFO threshold |
| 0x2A0 | AXI_XFER_CONFIG | RW | RD_XFER_BEATS, WR_XFER_BEATS |
| 0x2B0 | PERF_CONFIG | RW | Perf profiler enable / mode / clear |
| 0x2C0 | OBS_CTRL | RW | Channel observation mux select |
| 0x2D0 | PERF_DATA_LOW | RO | Perf capture low word (FIFO head) |
| 0x2D4 | PERF_DATA_HIGH | RO | Perf capture high word, same entry; the FIFO pops once BOTH halves are read |
| 0x2D8 | PERF_STATUS | RO | EMPTY / FULL / COUNT |
| 0x1000+ | Monitor / performance registers | RW/RO | AXI monitor config (0x10C0-0x111F) and per-monitor perf counters (0x1150-0x11F4), in the separate stream_mon_regs regfile |

Note: The AXI monitor and performance registers live in a separate stream_mon_regs regfile instantiated at offset 0x1000 on the same APB slave. The APB address decode is therefore 13 bits (8 KB) so those registers are addressable. Detailed monitor register layout is in the MAS (stream_mas/ch04_registers/register_map.md).

## 13.4	Key Registers

### 13.4.1	Global Control (CTRL) - 0x000

| Bits | Field | Access | Description |
| --- | --- | --- | --- |
| [0] | ENABLE | RW | Global enable |
| [1] | SOFT_RESET | RW | Soft reset (self-clearing) |
| [31:2] | Reserved | - | Reserved |

### 13.4.2	Global Status (STATUS) - 0x004

| Bits | Field | Access | Description |
| --- | --- | --- | --- |
| [0] | BUSY | RO | Any channel active |
| [7:1] | Reserved | - | Reserved |
| [15:8] | CH_ACTIVE | RO | Per-channel active mask |
| [23:16] | CH_ERROR | RO | Per-channel error mask |
| [31:24] | Reserved | - | Reserved |

### 13.4.3	Channel Control (CHn_CTRL) - 0x040 + n*0x10

| Bits | Field | Access | Description |
| --- | --- | --- | --- |
| [31:0] | DESC_ADDR | RW | Descriptor address (kick-off on write) |

Writing to CHn_CTRL initiates a transfer: 1. Descriptor address stored 2. Channel transitions from IDLE to FETCH 3. PREADY de-asserted until channel accepts kick-off

### 13.4.4	Channel Status (CHn_STATUS) - 0x044 + n*0x10

| Bits | Field | Access | Description |
| --- | --- | --- | --- |
| [3:0] | STATE | RO | Channel FSM state |
| [4] | IDLE | RO | Channel is idle |
| [5] | BUSY | RO | Channel is processing |
| [6] | ERROR | RO | Error condition |
| [7] | COMPLETE | RO | Last transfer complete |
| [15:8] | DESC_COUNT | RO | Descriptors processed |
| [23:16] | ERR_CODE | RO | Error code if ERROR set |
| [31:24] | Reserved | - | Reserved |

### 13.4.5	Channel Observation Control (OBS_CTRL) - 0x0C0

| Bits | Field | Access | Description |
| --- | --- | --- | --- |
| [2:0] | CHANNEL_SEL | RW | Channel index (0-7) for observation read |
| [31:3] | Reserved | - | Reserved |

Software writes the channel index to this register, then reads OBS_FLAGS and OBS_DATA0/DATA1 to observe that channel’s internal state.

### 13.4.6	Channel Observation Flags (OBS_FLAGS) - 0x0C4

| Bits | Field | Access | Description |
| --- | --- | --- | --- |
| [0] | SCH_ERROR_STICKY | RO | Scheduler error sticky (descriptor invalid, timeout, etc.) |
| [1] | SCH_TIMEOUT | RO | Scheduler timeout state (waiting on AXI master) |
| [7:2] | Reserved | - | Reserved |
| [31:8] | Reserved | - | Reserved |

These flags are driven by the scheduler’s internal error and timeout monitoring. See stream_mas/ch02_blocks/04_scheduler.md for detailed definitions.

### 13.4.7	Channel Observation Data (OBS_DATA0, OBS_DATA1) - 0x0C8, 0x0CC

Per-channel data such as descriptor state, current beat count, or other diagnostic fields. Exact contents vary by channel state; see the register map audit document for current field assignments.

## 13.5	Access Timing

### 13.5.1	Write Access

APB Write Access Timing

Source: apb_write_access.json

### 13.5.2	Read Access

APB Read Access Timing

Source: apb_read_access.json

## 13.6	Kick-off Blocking

When software writes to CHn_CTRL:

- If channel is IDLE: Immediate accept, PREADY asserted

- If channel is BUSY: PREADY held low until channel completes

- Prevents descriptor pointer corruption during active transfer

## 13.7	Launching Channels Together (KICK_ENABLE)

Software stages a 64-bit descriptor address per channel in CHn_CTRL_{LOW,HIGH} (0x000-0x03F, ordinary stored registers) and then launches with one write to KICK_ENABLE (0x128). Each KICKn field is a single-pulse: the write emits a one-cycle request and self-clears. Writing an address no longer kicks anything, so staging and launching are separate steps and every channel starts on the same cycle.

This matters when the APB path is reached over a slow transport (a UART-to-APB bridge on an FPGA), where eight separate two-register kicks would serialize channel starts over milliseconds.

Historical note. This replaced two earlier mechanisms: a kick block that snooped the raw APB command stream so that the address write itself kicked (the address was never readable state), and a pair of top-level i_kick_burst_mask / i_kick_burst_addr ports that existed only to work around that latency from harness_csr. Neither exists in the RTL. This page documented the i_kick_burst_* ports as a live interface until 2026-09-26; there are no such ports on any module, and KICK_ENABLE needs no port plumbing outside STREAM. See the comment at rtl/top/stream_top_ch8.sv:405-417.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Monitoring Interface

## 14.1	Overview

STREAM provides a unified Monitor Bus (MonBus) interface for debug and performance monitoring. This interface streams event packets (128-bit) with paired 64-bit side-band timestamps, capturing key operational events from all internal blocks.

## 14.2	Signal Summary

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | Output | 1 | Packet valid |
| monbus_pkt_ready | Input | 1 | Downstream ready |
| monbus_pkt_data | Output | 128 | Packet data (protocol-independent monitor packet) |
| monbus_ts_data | Output | 64 | Side-band timestamp (cycle count at packet capture) |

## 14.3	Packet Format

The 128-bit MonBus packet and 64-bit timestamp are paired atomically and transmitted as a 192-bit group through AXI-Lite skid buffering. See the canonical packet specification at docs/markdown/rtl-amba/includes/ocelus_package_spec.md for complete field definitions.

| Bits | Field | Description |
| --- | --- | --- |
| [63:56] | SUBSYSTEM_ID | STREAM subsystem identifier |
| [55:48] | EVENT_TYPE | Event category |
| [47:40] | EVENT_CODE | Specific event within category |
| [39:32] | CHANNEL_ID | Associated channel (0-7) |
| [31:0] | PAYLOAD | Event-specific data |

## 14.4	Monitoring Architecture

### 14.4.1	Descriptor Path Monitoring (Both Ends)

STREAM monitors the descriptor fetch path at both ends for observability:

- Descriptor Fetch Master (Agent ID 8): Monitors the AXI4 master that fetches descriptors from system memory

- Descriptor Consume Side (Agent ID 48 + channel): Monitors when the scheduler consumes the descriptor for processing

This dual-end monitoring allows software to attribute descriptor path stalls to either fetch delay (master side) or consume delay (slave side), enabling faster root-cause analysis.

### 14.4.2	Global Monitor Enable Methodology

All internal monitors are gated by: - Global SV parameter: USE_MONITOR (default: enabled for analysis, disabled for area-optimized builds) - Per-port SV parameters: Individual monitor enable for each monitored port - APB runtime configuration: Masks for enabling/disabling packet types at run time

## 14.5	Event Types

STREAM uses the unified MonBus event categorization defined in docs/markdown/rtl-amba/includes/ocelus_package_spec.md: - Error events (Type 0x0): Protocol violations, timeouts, invalid descriptors - Completion events (Type 0x1): Successful transfer completion, descriptor processed - Performance events (Type 0x4): Latency metrics, throughput data - Protocol-specific events: AXI address-match, debug traces

All STREAM events use Unit ID = 1 and channel-specific Agent IDs for the source block (descriptor engine, scheduler, AXI monitor, etc.).

Do not re-state the 128-bit packet layout here — refer to the canonical specification for field definitions and bit assignments.

## 14.6	Event Configuration

### 14.6.1	Configurable Events

Events can be enabled/disabled via APB registers:

| Register | Bit | Event Category |
| --- | --- | --- |
| MONBUS_CFG | [0] | Transfer events |
| MONBUS_CFG | [1] | Error events |
| MONBUS_CFG | [2] | Performance events |
| MONBUS_CFG | [7:4] | Channel mask |

### 14.6.2	Event Priority

When multiple events occur simultaneously:

- Error events (highest priority)

- Transfer completion events

- Transfer start events

- Performance events (lowest priority)

## 14.7	Bulk-Trace Compression

The MonBus output can be compressed before it leaves STREAM. Compression is a build option with a runtime enable on top of it:

| Control | Kind | Meaning |
| --- | --- | --- |
| USE_MON_COMPRESSION | parameter | Builds the compressor into the monbus group. 0 compiles it out entirely. |
| USE_MON_HALFBEAT | parameter | Packs two 30-bit slots per 64-bit beat. Only meaningful with USE_MON_COMPRESSION=1. |
| COMPRESS_EN | register field | Runtime enable. Only meaningful when the compressor was built. |

Both parameters default to 1 via CFG_USE_MON_COMPRESSION / CFG_USE_MON_HALFBEAT. A runtime COMPRESS_EN on a build that compiled the compressor out is silently inert, so read the build parameter before concluding anything from the enable bit.

What compression changes on the wire. Uncompressed, each 128-bit packet occupies three 64-bit beats ({tag, timestamp}, packet[127:64], packet[63:0]). Compressed, the group emits variable compressor slots with the tag in bits [63:60]. The two encodings are not interchangeable, which has one important consequence for integrators:

- A capture memory is the destination for compressed traffic. The host reads it back and decodes it with the reference model.

- A tally cannot consume compressed records. It reassembles raw 3-beat records with a mod-3 beat counter and has no compressor-slot decoder, so pointing compressed traffic at a tally silently produces nothing.

A useful sanity check when reading a capture back: uncompressed data lands at exactly 3.00 slots per packet. Any measurement at 3.00 is raw, whatever the enable bits say.

Measured (cosim, 2026-09-08). With the compressor built and half-beat packing on, a monitors-on DMA workload captured 36 slots for 32 decoded packets: 1.12 slots per packet, 62.5% smaller than the raw 3-beat encoding. The same workload with the compressor compiled out produced 96 slots for the same 32 packets – exactly 3.00, the raw ratio. Both decoded identically, so the difference is encoding alone.

Implementation is documented globally, not here. The encoder, its slot format, the half-beat packing and the group that carries them are specified in:

- docs/markdown/rtl-amba/monitor/monbus_compressor.md – the encoder and slot format

- docs/markdown/rtl-amba/monitor/monbus_halfbeat_packer.md – half-beat packing

- docs/markdown/rtl-amba/monitor/monbus_group_core.md – the group, its FIFOs and the beat layouts for both modes

Those are the canonical descriptions. This section states only what a STREAM integrator must decide: whether to build the compressor, and where the records are allowed to land.

## 14.8	Backpressure Handling

### 14.8.1	Flow Control

- Events generated when monbus_pkt_ready is asserted

- If monbus_pkt_ready is low, events are queued in internal FIFO

- FIFO depth: 64 entries (configurable)

- Overflow behavior: Oldest events discarded, overflow counter incremented

### 14.8.2	Recommendations

- Connect MonBus to FIFO with sufficient depth

- Monitor overflow counter in status register

- Size downstream FIFO based on expected event rate

## 14.9	Typical MonBus Downstream

STREAM MonBus → MonBus FIFO → MonBus Arbiter → System Trace
                  (64 deep)    (multi-source)    Interface

## 14.10	Example Event Sequence

A single-descriptor transfer generates these events:

Time    Event
----    -----
T0      KICKOFF     - Software kicks off channel
T1      DESC_FETCH  - Descriptor fetch initiated
T10     DESC_VALID  - Descriptor parsed successfully
T12     ARB_WAIT    - Waiting for data path grant (if applicable)
T15     READ_START  - AXI read burst initiated
T75     READ_DONE   - All source data read
T80     WRITE_START - AXI write burst initiated
T140    WRITE_DONE  - All data written, responses received
T141    COMPLETE    - Channel complete, back to IDLE

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Throughput Targets

## 15.1	Theoretical Maximum

### 15.1.1	Per-Interface Bandwidth

| Interface | Data Width | Frequency | Bandwidth |
| --- | --- | --- | --- |
| Descriptor Fetch | 256 bits | 200 MHz | 6.4 GB/s |
| Data Read | 512 bits | 200 MHz | 12.8 GB/s |
| Data Write | 512 bits | 200 MHz | 12.8 GB/s |

### 15.1.2	Aggregate Bandwidth

- Bidirectional: 25.6 GB/s (simultaneous read + write)

- Descriptor Overhead: ~0.1% for large transfers

## 15.2	Practical Throughput

### 15.2.1	Single-Channel Performance

| Scenario | Expected Efficiency | Effective Bandwidth |
| --- | --- | --- |
| Large sequential (>1MB) | >95% | >12.0 GB/s |
| Medium sequential (64KB-1MB) | 85-95% | 10.8-12.0 GB/s |
| Small sequential (4KB-64KB) | 70-85% | 9.0-10.8 GB/s |
| Very small (<4KB) | 40-70% | 5.0-9.0 GB/s |

### 15.2.2	Multi-Channel Performance

| Active Channels | Per-Channel BW | Aggregate BW |
| --- | --- | --- |
| 1 | 12.8 GB/s | 12.8 GB/s |
| 2 | 6.4 GB/s | 12.8 GB/s |
| 4 | 3.2 GB/s | 12.8 GB/s |
| 8 | 1.6 GB/s | 12.8 GB/s |

Note: Aggregate limited by shared AXI masters. Individual channel throughput scales inversely with active channel count.

### 15.2.3	Measured on the Genesys 2: Shared and Per-Channel Descriptor Engines

Source: the two bitstreams of the Resource Estimates chapter, stream_perf_genesys2_8ch_pce.bit (SHARED_ENGINES = 0) and stream_perf_genesys2_8ch.bit (SHARED_ENGINES = 1): 8 channels, 128-bit data, 100 MHz. Both ran back to back on 2026-10-17 on the same board under the same host sweep (host_characterize.py): 1, 2, 4, 8 and 16 descriptors per channel, 1 to 8 active channels, 1 MiB per channel, and no added memory latency. That is 40 configurations. The records are projects/fpga-systems/Genesys2/dma-ip/stream/reports/perf/json/stream_sweep_20261017_shared_engines0.json and ..._shared_engines1.json.

Genesys 2 Board Results, SHARED_ENGINES 0 and 1

| Measure | Per-channel engines | Shared engine |
| --- | --- | --- |
| Configurations passed | 40 of 40 | 40 of 40 |
| Data rate, first read to last read and first write to last write | 1525.9 MiB/s | 1525.9 MiB/s |
| Theoretical peak, 16 bytes x 100 MHz | 1525.9 MiB/s (1600 MB/s) | 1525.9 MiB/s (1600 MB/s) |
| Lowest end-to-end rate, kick to done | 1524.8 MiB/s | 1524.8 MiB/s |
| Total cycles, shared minus per-channel |  | -1 in every configuration |

Both builds move one beat per cycle from the first beat to the last on both AXI masters, which is the line rate. The data-phase cycle counts are identical in all 40 configurations. The only difference is that every run with the shared engine finishes exactly one cycle sooner, a fixed start-up difference in the descriptor path. Sharing the descriptor engine costs no throughput.

## 15.3	Throughput Limiting Factors

### 15.3.1	Memory System Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Memory latency | Reduces efficiency for small transfers | Use deeper outstanding |
| Memory bandwidth | Hard limit | Match STREAM to memory capability |
| Interconnect contention | Variable impact | Priority configuration |

### 15.3.2	STREAM Internal Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Descriptor fetch overhead | Fixed per descriptor | Use longer transfers |
| Per-descriptor boundary bubble | ~13 cyc/descriptor (chains) | Read-ahead prefetch (default on) — eliminates it |
| SRAM depth | Limits outstanding | Configure adequate depth |
| Arbitration overhead | Multi-channel penalty | Reduce active channels |
| Burst splitting | 4KB boundary crossings | Align transfers |

## 15.4	Performance Targets

### 15.4.1	Design Targets

| Metric | Target | Condition |
| --- | --- | --- |
| Peak throughput | 12.8 GB/s | Single direction, ideal |
| Sustained throughput | >11.0 GB/s | Large transfers, single channel |
| Multi-channel aggregate | >10.0 GB/s | 8 channels active |
| Small transfer efficiency | >50% | 4KB transfers |

### 15.4.2	Verification Criteria

| Scenario | Pass Criteria |
| --- | --- |
| 1MB sequential read | >95% efficiency |
| 1MB sequential write | >95% efficiency |
| 64KB scatter (4x16KB) | >80% efficiency |
| 4KB × 100 descriptors | >60% efficiency |

## 15.5	Descriptor Chain Efficiency

### 15.5.1	Descriptor Overhead

| Transfer Size | Descriptor Fetches | Overhead |
| --- | --- | --- |
| 64 KB | 1 | 0.05% |
| 256 KB | 1 | 0.01% |
| 1 MB | 1 | <0.01% |
| 4 KB (100 chained) | 100 | 0.5% |

### 15.5.2	Chaining Efficiency

For chained descriptors, next descriptor fetch can overlap with current transfer:

### 15.5.3	Figure 5.1.1: Descriptor Chain Fetch Overlap

Descriptor Chain Fetch Overlap

This pipelining minimizes chaining overhead for continuous transfers.

### 15.5.4	Read-Ahead Descriptor Prefetch (Cross-Descriptor Streaming)

Descriptor fetch overlaps the current transfer (above), but the datapath itself must also stream across the boundary. Without prefetch the read side stalls at each descriptor boundary while the scheduler FSM advances and the read pipeline (AR -> R) refills — a per-descriptor bubble of ~13 cycles that dominates on short-descriptor chains. Read-ahead descriptor prefetch (runtime SCHED_CONFIG.RD_PREFETCH_EN, default on; see MAS Section 2.4) advances the read side to descriptor N+1 while the write side still drains N, so the read stream never goes cold. The per-descriptor bubble drops to zero — chain throughput becomes independent of descriptor count, with only a one-time fill/drain of overhead.

Measured on silicon (Nexys A7, 100 MHz, one bitstream, prefetch on vs off):

| Descriptor size | Prefetch OFF (lockstep) | Prefetch ON |
| --- | --- | --- |
| 64 beats (16-desc chain) | 76.1% datapath util | 95.3% |
| >= 4096 beats | ~99% | 99-100% |

Applies to chained legacy descriptors. EXT (row/col-major) descriptors run lockstep, but a single row-major descriptor is internally bubble-free (its contiguous runs stream through the address-generator base FIFO — measured 99.7% util). Transpose access patterns (col-major on either side) are single-beat and latency-bound by the memory system, not by a scheduler bubble.

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Latency Characteristics

## 16.1	Latency Components

### 16.1.1	End-to-End Latency Breakdown

For a single-descriptor transfer (kick-off to completion):

| Phase | Typical Cycles | Notes |
| --- | --- | --- |
| APB write processing | 2-3 | APB transaction |
| Descriptor fetch | 10-50 | Memory latency dependent |
| Descriptor parse | 2-4 | Fixed pipeline |
| Arbitration wait | 0-100+ | Depends on contention |
| Read phase setup | 2-3 | AXI AR channel |
| Read data transfer | N + 10-50 | N beats + memory latency |
| Write phase setup | 2-3 | AXI AW channel |
| Write data transfer | N + 5-20 | N beats + response latency |
| Completion handling | 2-4 | Status update, IRQ |

### 16.1.2	Total Latency Formula

Latency = T_apb + T_desc_fetch + T_parse + T_arb + T_read + T_write + T_complete

Where:
  T_apb        = 3 cycles (fixed)
  T_desc_fetch = Memory_latency + 10 cycles
  T_parse      = 3 cycles (fixed)
  T_arb        = 0 to N (contention dependent)
  T_read       = Memory_latency + (Transfer_length / Beats_per_burst) * Burst_overhead + Transfer_length
  T_write      = Memory_latency + Transfer_length + Response_latency
  T_complete   = 3 cycles (fixed)

## 16.2	First-Byte Latency

Time from kick-off to first data beat arriving at destination:

| Component | Cycles | Cumulative |
| --- | --- | --- |
| APB processing | 3 | 3 |
| Descriptor fetch | 30 | 33 |
| Parse + setup | 5 | 38 |
| Read AR accepted | 10 | 48 |
| First read data | 20 | 68 |
| Write AW + first W | 10 | 78 |
| Total | ~80 | - |

At 200 MHz: ~400 ns first-byte latency (typical)

## 16.3	Latency by Transfer Size

### 16.3.1	Small Transfers

| Transfer Size | Total Latency | Efficiency |
| --- | --- | --- |
| 64 bytes (1 beat) | ~100 cycles | 1% |
| 256 bytes (4 beats) | ~110 cycles | 4% |
| 1 KB (16 beats) | ~130 cycles | 12% |
| 4 KB (64 beats) | ~180 cycles | 35% |

### 16.3.2	Large Transfers

| Transfer Size | Total Latency | Efficiency |
| --- | --- | --- |
| 16 KB (256 beats) | ~400 cycles | 64% |
| 64 KB (1024 beats) | ~1200 cycles | 85% |
| 256 KB (4096 beats) | ~4400 cycles | 93% |
| 1 MB (16384 beats) | ~17000 cycles | 96% |

## 16.4	Latency Variability

### 16.4.1	Sources of Variability

| Source | Impact | Range |
| --- | --- | --- |
| Memory system load | High | 10-100+ cycles |
| AXI interconnect | Medium | 5-50 cycles |
| Channel arbitration | Medium | 0-100+ cycles |
| Descriptor chaining | Low | 2-5 cycles |

### 16.4.2	Worst-Case Latency

Conditions for worst-case latency:

- All channels active (maximum arbitration delay: 8 or 16 channels, by top)

- Memory system fully loaded (maximum memory latency)

- Small transfer size (maximum overhead ratio)

- Low priority descriptor (maximum wait time)

Under these conditions, first-byte latency could exceed 1000 cycles.

## 16.5	Latency Optimization

### 16.5.1	Software Recommendations

| Recommendation | Benefit |
| --- | --- |
| Use larger transfers | Better efficiency |
| Minimize active channels | Reduce arbitration |
| Use higher priority for latency-sensitive | Earlier service |
| Pre-stage descriptors in cache | Faster fetch |

### 16.5.2	Hardware Configuration

| Setting | Effect |
| --- | --- |
| Increase outstanding limit | Hide memory latency |
| Enable descriptor prefetch | Reduce chaining overhead |
| Configure larger SRAM | Support longer bursts |

## 16.6	Interrupt Latency

Time from transfer completion to interrupt assertion:

| Phase | Cycles |
| --- | --- |
| Last write response | 0 |
| Completion FSM | 2 |
| Status register update | 1 |
| Interrupt assertion | 1 |
| Total | 4 cycles |

At 200 MHz: 20 ns from completion to interrupt.

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Resource Estimates

## 17.1	Measured Core Area

Out-of-context Vivado synthesis of stream_core at 256-bit data, with monitors off and a 100 MHz clock (2026-10-17):

Measured stream_core Area and Timing

| Build | LUTs | Flops | Worst slack |
| --- | --- | --- | --- |
| 8 channels, one descriptor engine per channel (SHARED_ENGINES = 0) | 25.8k | 14.9k | +3.24 ns |
| 8 channels, shared descriptor engine (default) | 20.7k | 11.7k | +2.50 ns |
| 16 channels, shared descriptor engine | 36.8k | 19.4k | +1.79 ns |

Sharing the descriptor engine removes about a fifth of the 8-channel core’s LUTs and flops: the per-channel AR issue logic, the address-range comparators, the 256-bit holding registers and the MonBus reporters. The 16-channel build costs about 1.8 times the 8-channel build and still closes at 100 MHz.

## 17.2	Measured Board Build (Genesys 2)

Source: two placed-and-routed Genesys 2 builds made on 2026-10-17 from the same RTL, differing only in SHARED_ENGINES. The bitstreams are stream_perf_genesys2_8ch_pce.bit (SHARED_ENGINES = 0) and stream_perf_genesys2_8ch.bit (SHARED_ENGINES = 1). Both builds use 8 channels, 128-bit data, 256 entries per channel, 8 outstanding reads and writes, AXI monitors off and GEN_MON = 0, at 100 MHz on the xc7k325tffg900-2 with Vivado 2025.1. The STREAM rows are the u_stream instance of the hierarchical post-route report; the whole-design rows include the characterization harness.

Post-Route Utilization, SHARED_ENGINES 0 and 1

| Resource | Per-channel engines | Shared engine | Change |
| --- | --- | --- | --- |
| Whole design, Slice LUTs | 63,642 | 62,111 | -1,531 (-2.4 %) |
| Whole design, Slice registers | 56,536 | 54,111 | -2,425 (-4.3 %) |
| STREAM (u_stream), LUTs | 30,781 | 29,247 | -1,534 (-5.0 %) |
| STREAM (u_stream), flip-flops | 23,079 | 20,654 | -2,425 (-10.5 %) |
| Descriptor engines, LUTs / flip-flops | 4,974 / 4,120 (8 engines) | 3,825 / 1,725 (1 engine) | -1,149 / -2,395 |
| Block RAM tiles | 26 | 26 | 0 |
| DSP | 64 | 64 | 0 |
| Worst setup slack | +0.612 ns | +0.377 ns |  |

The board saving is smaller than the out-of-context figure above for two reasons. The board build has GEN_MON = 0, so the per-channel MonBus reporters that sharing removes are not built in either case. And the shared engine keeps a descriptor FIFO and an extended-descriptor FIFO per channel; together they are 1,840 LUTs of distributed RAM, almost half the engine. Block RAM and DSP are unchanged because the descriptor engine uses neither. Both builds meet timing with no failing endpoint.

## 17.3	FPGA Resource Summary (Estimates)

The estimates below predate the measurements above and are kept for the per-block breakdown. Typical configuration (8 channels, 512-bit data width, 4096-entry SRAM per channel):

| Resource | Estimate | Notes |
| --- | --- | --- |
| LUTs | 15,000 - 25,000 | Logic and control |
| Registers | 20,000 - 30,000 | Pipeline and state |
| Block RAM | 40 - 60 | SRAM buffer |
| DSP | 0 | No DSP usage |

## 17.4	Resource Breakdown by Block

### 17.4.1	APB Configuration Slave

| Resource | Estimate |
| --- | --- |
| LUTs | 500 - 800 |
| Registers | 400 - 600 |
| BRAM | 0 |

### 17.4.2	Descriptor Engine

One shared engine for every channel in the default build.

| Resource | Estimate |
| --- | --- |
| LUTs | 1,500 - 2,500 |
| Registers | 1,000 - 1,500 |
| BRAM | 1 (descriptor FIFO) |

### 17.4.3	Scheduler (8 channels)

| Resource | Estimate |
| --- | --- |
| LUTs | 3,000 - 5,000 |
| Registers | 4,000 - 6,000 |
| BRAM | 0 |

### 17.4.4	Channel Arbiter

| Resource | Estimate |
| --- | --- |
| LUTs | 500 - 1,000 |
| Registers | 200 - 400 |
| BRAM | 0 |

### 17.4.5	AXI Read Engine

| Resource | Estimate |
| --- | --- |
| LUTs | 2,000 - 3,500 |
| Registers | 3,000 - 4,500 |
| BRAM | 0 |

### 17.4.6	SRAM Buffer

| Resource | Estimate |
| --- | --- |
| LUTs | 100 - 200 |
| Registers | 100 - 200 |
| BRAM | 32 - 64 (depth/width dependent) |

### 17.4.7	AXI Write Engine

| Resource | Estimate |
| --- | --- |
| LUTs | 2,000 - 3,500 |
| Registers | 3,000 - 4,500 |
| BRAM | 0 |

### 17.4.8	MonBus Reporter

| Resource | Estimate |
| --- | --- |
| LUTs | 800 - 1,200 |
| Registers | 600 - 900 |
| BRAM | 1 (event FIFO) |

## 17.5	SRAM Sizing

SRAM buffer size depends on configuration:

| Configuration | BRAM (18Kb) | BRAM (36Kb) |
| --- | --- | --- |
| 512b x 1024 | 16 | 8 |
| 512b x 2048 | 32 | 16 |
| 512b x 4096 | 64 | 32 |
| 256b x 4096 | 32 | 16 |
| 128b x 4096 | 16 | 8 |

## 17.6	Scaling with Parameters

### 17.6.1	Channel Count Impact

| Channels | LUT Delta | Register Delta |
| --- | --- | --- |
| 1 | Base | Base |
| 2 | +500 | +800 |
| 4 | +1,500 | +2,400 |
| 8 | +3,500 | +5,600 |
| 16 | measured: see Measured Core Area |  |

### 17.6.2	Data Width Impact

| Data Width | LUT Delta | Register Delta | BRAM Delta |
| --- | --- | --- | --- |
| 128 bits | -30% | -40% | -75% |
| 256 bits | -15% | -20% | -50% |
| 512 bits | Base | Base | Base |

## 17.7	ASIC Estimates

For 28nm technology node (typical):

| Metric | Estimate |
| --- | --- |
| Gate Count | 200K - 350K gates |
| Area | 0.2 - 0.4 mm2 |
| SRAM Area | 0.1 - 0.2 mm2 (4096x512) |
| Total Area | 0.3 - 0.6 mm2 |

### 17.7.1	Power Estimates

| Condition | Power |
| --- | --- |
| Idle (clock gated) | <1 mW |
| Idle (clock running) | 10 - 20 mW |
| Single channel active | 50 - 100 mW |
| All channels active | 150 - 300 mW |

## 17.8	Timing Estimates

### 17.8.1	Target Frequencies

| Technology | Target Fmax |
| --- | --- |
| Xilinx Ultrascale+ | 250 MHz |
| Agilex | 250 MHz |
| 28nm ASIC | 400 MHz |
| 16nm ASIC | 600 MHz |

### 17.8.2	Critical Paths

| Path | Concern Level | Mitigation |
| --- | --- | --- |
| AXI data path | Low | Fully pipelined |
| SRAM read/write | Low | Registered outputs |
| Arbiter decision | Medium | Priority encoder depth |
| Descriptor parse | Low | Simple combinational |

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	System Requirements

## 18.1	Interface Requirements

### 18.1.1	APB Interface

| Requirement | Specification |
| --- | --- |
| Protocol | APB3 or APB4 |
| Data Width | 32 bits |
| Address Width | 12 bits minimum |
| Timing | Synchronous to aclk |

### 18.1.2	AXI Interfaces

| Requirement | Descriptor | Read Data | Write Data |
| --- | --- | --- | --- |
| Protocol | AXI4 | AXI4 | AXI4 |
| Data Width | 256 bits | Parameterizable | Parameterizable |
| Address Width | 64 bits | 64 bits | 64 bits |
| ID Width | 4 bits min | 4 bits min | 4 bits min |
| Outstanding | 2 | Configurable | Configurable |

### 18.1.3	MonBus Interface

| Requirement | Specification |
| --- | --- |
| Data Width | 64 bits |
| Protocol | Valid/Ready streaming |
| Backpressure | Required (must handle ready deassertion) |

## 18.2	Memory Requirements

### 18.2.1	Descriptor Memory

| Requirement | Specification |
| --- | --- |
| Alignment | 32-byte aligned |
| Access | Read access via descriptor AXI master |
| Coherency | Software responsibility |
| Latency | <100 cycles recommended |

### 18.2.2	Data Memory

| Requirement | Specification |
| --- | --- |
| Alignment | Per data width (64-byte for 512-bit) |
| Access | Read via read master, Write via write master |
| Bandwidth | Sufficient for target throughput |

## 18.3	System Bandwidth Allocation

### 18.3.1	AXI Interconnect Requirements

To achieve maximum STREAM throughput:

| AXI Master | Required Bandwidth | Priority |
| --- | --- | --- |
| Descriptor Fetch | 0.1 GB/s | Medium |
| Data Read | 12.8 GB/s | High |
| Data Write | 12.8 GB/s | High |

### 18.3.2	Interconnect Recommendations

- Configure STREAM AXI masters with high priority

- Ensure sufficient outstanding transaction support

- Consider dedicated paths for high-bandwidth DMA

## 18.4	Interrupt Requirements

### 18.4.1	Interrupt Controller

| Requirement | Specification |
| --- | --- |
| Interrupt Lines | 8 (per-channel) or 1 (combined) |
| Type | Level-sensitive, active-high |
| Latency | <10 cycles from assertion to controller |

### 18.4.2	Interrupt Handling

- Software must read IRQ_STATUS to identify source

- Write-1-to-clear to acknowledge interrupts

- Channel remains blocked until error cleared (if applicable)

## 18.5	Address Space Requirements

### 18.5.1	STREAM Register Space

| Requirement | Specification |
| --- | --- |
| Base Address | System-defined |
| Size | 4 KB (0x1000) |
| Alignment | 4 KB aligned |

### 18.5.2	Memory Map Constraints

- Descriptor addresses must be accessible via descriptor AXI master

- Source addresses must be accessible via read AXI master

- Destination addresses must be accessible via write AXI master

- All addresses must respect configured address range limits

## 18.6	Software Requirements

### 18.6.1	Driver Requirements

| Requirement | Description |
| --- | --- |
| Descriptor Preparation | Build descriptors in coherent memory |
| Cache Management | Flush descriptor cache before kick-off |
| Interrupt Handling | Service interrupts promptly |
| Error Recovery | Clear error status, optionally retry |

### 18.6.2	Programming Sequence

- Verify STREAM is enabled (CTRL.ENABLE = 1)

- Check target channel is idle (CHn_STATUS.IDLE = 1)

- Prepare descriptor chain in memory

- Flush cache (if applicable)

- Write first descriptor address to CHn_CTRL

- Wait for interrupt or poll CHn_STATUS

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Clocking and Reset

## 19.1	Clock Requirements

### 19.1.1	Primary Clock (aclk)

| Parameter | Requirement |
| --- | --- |
| Frequency | 100 - 400 MHz (target dependent) |
| Duty Cycle | 40% - 60% |
| Jitter | <100 ps peak-to-peak |
| Domain | All logic except the APB slave side (see below) |

### 19.1.2	Clock Relationship

Everything is synchronous to aclk except the APB slave interface. With CDC_ENABLE = 1 (the stream_top_ch8 default) s_apb_* runs on pclk / presetn and apb4_slave_cdc crosses into aclk; with CDC_ENABLE = 0 apb4_slave is used and the APB interface runs on aclk:

### 19.1.3	Figure 6.2.1: Clock Domains

Clock Domains

Source: ch06_clock_distribution.dot

[INTEG-1] The AXI, AXI-Lite and interrupt interfaces must be synchronous to aclk. The APB interface may use its own pclk when CDC_ENABLE = 1; any other clock domain crossing is the system integrator’s responsibility.

## 19.2	Reset Requirements

### 19.2.1	Reset Signal (aresetn)

| Parameter | Requirement |
| --- | --- |
| Polarity | Active-low |
| Type | Asynchronous assert, synchronous deassert |
| Minimum Pulse | 2 clock cycles |
| Glitch Filter | Not required (external synchronizer assumed) |

### 19.2.2	Reset Sequence

Reset is asserted asynchronously and de-asserted synchronously:

- aresetn goes low (asynchronous assertion)

- Reset remains active for minimum 2 clock cycles

- aresetn goes high on rising edge of aclk (synchronous release)

- All blocks begin normal operation

### 19.2.3	Reset Behavior

| Block | Reset State |
| --- | --- |
| APB Config | All registers to default |
| Scheduler | All channels IDLE |
| Descriptor Engine | FSM to IDLE, FIFOs empty |
| AXI Engines | No outstanding transactions |
| SRAM | Contents undefined |
| MonBus | FIFO empty, no output |

## 19.3	Power Management

### 19.3.1	Clock Gating Support

STREAM supports external clock gating when idle:

| Signal | Purpose |
| --- | --- |
| all_channels_idle | Indicates safe to gate clock |

Clock Gating Sequence: 1. Check STATUS.BUSY = 0 2. Disable clock via external gate 3. Re-enable clock before any APB access

### 19.3.2	Low Power Recommendations

| Technique | Implementation |
| --- | --- |
| Clock gating | Gate aclk when all_channels_idle |
| Power gating | Not recommended (state loss) |
| Voltage scaling | Supported if timing met |

## 19.4	Timing Constraints

### 19.4.1	Clock Period Constraints

# Example Vivado XDC constraints
create_clock -period 5.000 -name aclk [get_ports aclk]

# APB interface (same clock)
set_input_delay -clock aclk 1.0 [get_ports s_apb_*]
set_output_delay -clock aclk 1.0 [get_ports s_apb_pready]
set_output_delay -clock aclk 1.0 [get_ports s_apb_prdata*]
set_output_delay -clock aclk 1.0 [get_ports s_apb_pslverr]

# AXI interfaces (same clock)
set_input_delay -clock aclk 1.0 [get_ports m_axi_*ready]
set_input_delay -clock aclk 1.0 [get_ports m_axi_*rdata*]
set_input_delay -clock aclk 1.0 [get_ports m_axi_*rresp*]
set_input_delay -clock aclk 1.0 [get_ports m_axi_*bresp*]
set_output_delay -clock aclk 1.0 [get_ports m_axi_*valid]
set_output_delay -clock aclk 1.0 [get_ports m_axi_*addr*]
set_output_delay -clock aclk 1.0 [get_ports m_axi_*data*]

### 19.4.2	False Path Constraints

# Reset is asynchronous
set_false_path -from [get_ports aresetn]

# Configuration registers to datapath (multi-cycle)
set_multicycle_path 2 -setup -from [get_cells *cfg_*]
set_multicycle_path 1 -hold -from [get_cells *cfg_*]

## 19.5	CDC Considerations

### 19.5.1	Single Clock Domain

STREAM operates in a single clock domain. All clock domain crossing must be handled externally:

| Interface | CDC Responsibility |
| --- | --- |
| APB Slave | External if APB on different clock |
| AXI Masters | External if AXI interconnect on different clock |
| MonBus | External if consumer on different clock |

### 19.5.2	CDC Recommendations

For systems requiring CDC:

- Use async FIFO for MonBus output

- Use AXI clock converter IP for AXI interfaces

- Use APB bridge with clock crossing for APB

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Verification Strategy

## 20.1	Verification Approach

### 20.1.1	Multi-Level Verification

STREAM verification follows a hierarchical approach:

| Level | Focus | Coverage Target |
| --- | --- | --- |
| Block | Individual module function | 100% line/toggle |
| Integration | Block-to-block interfaces | 95% functional |
| System | End-to-end operation | 90% use cases |

## 20.2	Block-Level Verification

### 20.2.1	Test Categories

| Block | Key Test Scenarios |
| --- | --- |
| APB Config | Register access, kick-off timing, error response |
| Descriptor Engine | Fetch, parse, chain, error handling |
| Scheduler | State transitions, multi-channel coordination |
| Channel Arbiter | Priority, fairness, starvation prevention |
| AXI Read Engine | Burst generation, error handling, backpressure |
| AXI Write Engine | Burst generation, response handling |
| SRAM Buffer | Read/write concurrent, full/empty |
| MonBus Reporter | Event generation, backpressure, overflow |

### 20.2.2	Coverage Metrics

| Metric | Target |
| --- | --- |
| Line coverage | 100% |
| Toggle coverage | 95% |
| FSM state coverage | 100% |
| FSM transition coverage | 100% |
| Parameter coverage | All supported values |

## 20.3	Integration-Level Verification

### 20.3.1	Interface Verification

| Interface | Test Focus |
| --- | --- |
| Kick registers to Descriptor Engine | Kick-off handshake, blocking behavior |
| Descriptor Engine to Scheduler | Descriptor passing, backpressure |
| Scheduler to AXI Engines | Grant/request, data flow |
| AXI Engines to SRAM | Concurrent access, pointer management |
| All blocks to MonBus | Event generation, arbitration |

### 20.3.2	Multi-Channel Scenarios

| Scenario | Description |
| --- | --- |
| Sequential | Channels operate one at a time |
| Concurrent | All channels active simultaneously (8 on stream_top_ch8, 16 on stream_top_ch16) |
| Priority | High priority preempts low priority |
| Fairness | Equal priority channels share fairly |
| Error isolation | Error in one channel, others continue |

## 20.4	System-Level Verification

### 20.4.1	End-to-End Test Cases

| Test Case | Description |
| --- | --- |
| Basic Transfer | Single descriptor, single channel |
| Chained Transfer | Multiple descriptors, single channel |
| Multi-Channel | Multiple channels, independent transfers |
| Scatter | Single source to multiple destinations |
| Gather | Multiple sources to single destination |
| Maximum Throughput | Sustained bandwidth test |
| Error Injection | AXI errors, invalid descriptors |
| Stress | Random traffic, long duration |

### 20.4.2	Performance Verification

| Metric | Verification Method |
| --- | --- |
| Throughput | Measure bytes/cycle over large transfer |
| Latency | Measure kick-off to completion cycles |
| Efficiency | Compare actual vs theoretical maximum |

## 20.5	Testbench Architecture

### 20.5.1	CocoTB-Based Verification

### 20.5.2	Figure 6.3.1: Testbench Architecture

Testbench Architecture

Source: ch06_testbench_stack.dot

### 20.5.3	Key Testbench Components

| Component | Purpose |
| --- | --- |
| APB Master BFM | Drive configuration interface |
| AXI Slave BFM | Model memory system |
| MonBus Monitor | Capture and verify events |
| Scoreboard | Verify data integrity |
| Coverage Collector | Track functional coverage |

## 20.6	Verification Environment

### 20.6.1	Simulator Support

| Simulator | Status |
| --- | --- |
| Verilator | Primary (open-source) |
| Icarus Verilog | Secondary |
| Commercial (VCS, Questa) | Supported |

### 20.6.2	Test Execution

# Run all STREAM tests
pytest projects/components/dma-ip/stream/dv/tests/ -v

# Run specific test category
pytest projects/components/dma-ip/stream/dv/tests/fub/ -v
pytest projects/components/dma-ip/stream/dv/tests/macro/ -v
pytest projects/components/dma-ip/stream/dv/tests/top/ -v

# Generate coverage report
pytest projects/components/dma-ip/stream/dv/tests/ --cov-report=html

## 20.7	Formal Verification

### 20.7.1	Properties for Formal

| Category | Properties |
| --- | --- |
| Protocol | AXI handshake rules, APB timing |
| Liveness | No deadlock in arbitration |
| Safety | No data corruption, proper error handling |
| Functional | Descriptor chaining correctness |

### 20.7.2	Formal Tool Usage

Recommended for: - AXI protocol compliance - Arbiter deadlock freedom - FSM reachability analysis

## 20.8	Sign-Off Criteria

### 20.8.1	Block-Level Sign-Off

- ☐ 100% line coverage

- ☐ 95% toggle coverage

- ☐ All FSM states exercised

- ☐ All FSM transitions exercised

- ☐ No simulation errors

- ☐ No lint warnings

### 20.8.2	Integration Sign-Off

- ☐ All interface scenarios passed

- ☐ Multi-channel concurrency verified

- ☐ Error handling verified

- ☐ Performance targets met

### 20.8.3	System Sign-Off

- ☐ All end-to-end test cases passed

- ☐ Throughput targets achieved

- ☐ Latency targets achieved

- ☐ 72-hour stress test passed

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	STREAM Specification Index

Version: 0.101 Date: 2026-10-20 Purpose: Complete technical specification for STREAM subsystem

## 21.1	Document Organization

Note: All chapters linked below for automated document generation.

### 21.1.1	Front Matter

- Document Information

### 21.1.2	Chapter 1: Overview

- Architecture

- Top-Level Port List

- Clocks and Reset

### 21.1.3	Chapter 2: Functional Blocks

Macro/Integration Level: - Stream Core - Scheduler Group Array - Scheduler Group

Control Path: - Scheduler - Descriptor Engine

Read Datapath: - AXI Read Engine - Stream Alloc Control

SRAM Buffering: - SRAM Controller - SRAM Controller Unit - Stream Latency Bridge

Write Datapath: - Stream Drain Control - AXI Write Engine

Configuration & Monitoring: - APB Config - Performance Profiler - MonBus AXIL Group

### 21.1.4	Chapter 3: Interfaces

- Interface Overview

- AXI4 Interface Specification

- AXIL4 Interface Specification

- APB Programming Interface

- MonBus Interface Specification

### 21.1.5	Chapter 4: Registers

- Register Map

- Complete Register Reference (generated from the RDL)

### 21.1.6	Chapter 5: Programming

- Programming Guide

### 21.1.7	Chapter 6: Configuration Reference

- Complete Configuration Guide

## 21.2	Quick Reference

### 21.2.1	Functional Unit Blocks (FUB)

| Module | File | Purpose | Lines | Status |
| --- | --- | --- | --- | --- |
| descriptor_engine | rtl/fub/descriptor_engine.sv | Descriptor fetch/parse (256-bit), one per channel (SHARED_ENGINES=0) | ~300 | [Done] |
| descriptor_engine_shared | rtl/fub/descriptor_engine_shared.sv | One descriptor fetch engine for every channel (SHARED_ENGINES=1, default; ported from RAPIDS) | ~600 | [Done] |
| scheduler | rtl/fub/scheduler.sv | Transfer coordinator | ~400 | [Done] Created (corrected) |
| axi_read_engine | rtl/fub/axi_read_engine.sv | AXI read master | ~350 | [Done] Created (no FSM - streaming pipeline) |
| axi_write_engine | rtl/fub/axi_write_engine.sv | AXI write master | ~400 | [Done] Created (no FSM - streaming pipeline) |
| sram_controller | rtl/fub/sram_controller.sv | Per-channel buffer management | ~350 | [Done] Created (no FSM - pointer arithmetic + pre-allocation) |
| sram_controller_unit | rtl/fub/sram_controller_unit.sv | Per-channel buffer unit (wraps gaxi_fifo_sync; replaced the old simple_sram.sv) | ~300 | [Done] |
| perf_profiler | rtl/fub/perf_profiler.sv | Channel performance profiling | ~400 | [Done] Dual-mode timestamp/elapsed tracking |

### 21.2.2	Integration Blocks (MAC)

| Module | File | Purpose | Lines | Status |
| --- | --- | --- | --- | --- |
| scheduler_group_array | rtl/macro/scheduler_group_array.sv | NUM_CHANNELS (8 or 16) scheduler array + descriptor fetch, shared or per channel (SHARED_ENGINES) | ~500 | [Done] |
| apb_config | rtl/macro/stream_regs.rdl + generated wrapper (regs/generated/) | Config registers | ~350 | [Done] PeakRDL-based |
| monbus_axil_group | rtl/amba/ocelus/monbus_axil4_axil4_group.sv (shared AMBA monitor block) | MonBus + AXIL | ~800 | [Done] |
| stream_top | rtl/top/stream_top_ch8.sv | Top-level (8-channel) | ~500 | [Done] |
| stream_top_ch16 | rtl/top/stream_top_ch16.sv | Top-level (16-channel); generated from stream_top_ch8.sv by bin/gen_stream_regs_16ch.py | ~500 | [Done] |
| apb_config (16ch) | rtl/macro/generated_16ch/stream_regs_16ch.rdl (derived) + regs/generated_16ch/ | 16-channel config registers | - | [Done] PeakRDL-based |

## 21.3	Performance Modes (AXI Engines)

STREAM AXI engines support three performance modes via compile-time parameters, offering area/performance trade-offs from tutorial simplicity to datacenter throughput.

### 21.3.1	Holistic Overview: V1/V2/V3 Working Together

Design Philosophy: - V1 (Low): Tutorial-focused, simple architecture, minimal area - V2 (Medium): Command pipelining hides memory latency, 6.7x throughput improvement - V3 (High): Out-of-order completion maximizes memory controller efficiency, 7.0x throughput

Key Insight: The benefit scales with memory latency. For low-latency SRAM (2-3 cycles), V1 achieves 40% throughput. For high-latency DDR4 (70-100 cycles), V1 drops to 14% but V2/V3 maintain 94-98% throughput.

Parameterization Strategy: - ENABLE_CMD_PIPELINE = 0: V1 (default, tutorial mode) - ENABLE_CMD_PIPELINE = 1: V2 (command pipelined, best area efficiency) - ENABLE_CMD_PIPELINE = 1, ENABLE_OOO_DRAIN = 1 (write) or ENABLE_OOO_READ = 1 (read): V3 (out-of-order, maximum throughput)

### 21.3.2	V1 - Low Performance (Single Outstanding Transaction)

Architecture: Single-burst-per-request, blocking on completion - Area: ~1,250 LUTs per engine - Throughput: 0.14 beats/cycle (DDR4), 0.40 beats/cycle (SRAM) - Outstanding Txns: 1 - Use Case: Tutorial examples, embedded systems, low-latency SRAM - Key Feature: Zero-bubble streaming pipeline (NO FSM!)

Blocking Behavior: - Write: Blocks on B response before accepting next request - Read: Blocks on R last before accepting next request - Simple control: 3 flags (r_ar_inflight, r_ar_valid, completion tracking)

### 21.3.3	V2 - Medium Performance (Command Pipelined)

Architecture: Command queue decouples command acceptance from data completion - Area: ~2,000 LUTs per engine (1.6x increase) - Throughput: 0.94 beats/cycle (DDR4), 0.85 beats/cycle (SRAM) - Outstanding Txns: 4-8 (command queue depth) - Use Case: General-purpose FPGA, DDR3/DDR4, balanced area/performance - Key Feature: Hides memory latency via pipelining (6.7x improvement)

Command Pipelining: - Write: AW queue + W drain FSM + B scoreboard (async response tracking) - Read: AR queue + R in-order reception (simpler than write, no B channel) - Per-command SRAM pointers enable multiple bursts from same channel

Best Area Efficiency: 4.19x throughput per unit area (6.7x throughput / 1.6x area)

### 21.3.4	V3 - High Performance (Out-of-Order Completion)

Architecture: OOO command selection maximizes memory controller efficiency - Area: ~3,500-4,000 LUTs per engine (2.8-3.2x increase) - Throughput: 0.98 beats/cycle (DDR4), 0.92 beats/cycle (SRAM) - Outstanding Txns: 8-16 (larger command queue) - Use Case: Datacenter, ASIC, HBM2, high-performance memory controllers - Key Feature: Flexible drain order based on SRAM data availability (7.0x improvement)

Out-of-Order Mechanisms: - Write: OOO W drain (select command with SRAM data ready), AXI ID matching for B responses - Read: OOO R reception (match m_axi_rid to queue entry), independent SRAM write per command - Transaction ID structure: {counter, channel_id} preserves channel routing for MonBus

Why OOO is Naturally Supported: 1. Per-channel SRAM partitioning (no cross-channel hazards) 2. Per-command pointer tracking (no pointer collision) 3. Transaction ID matching (channel ID in lower bits for routing)

### 21.3.5	Performance Comparison Summary

| Configuration | Area | DDR4 Throughput | SRAM Throughput | Area Efficiency | Use Case |
| --- | --- | --- | --- | --- | --- |
| V1 | 1.0x | 0.14 beats/cycle (1.0x) | 0.40 beats/cycle (1.0x) | 1.00 | Tutorial, embedded |
| V2 | 1.6x | 0.94 beats/cycle (6.7x) | 0.85 beats/cycle (2.1x) | 4.19 | General FPGA |
| V3 | 2.8x | 0.98 beats/cycle (7.0x) | 0.92 beats/cycle (2.3x) | 2.50 | Datacenter, ASIC |

Key Takeaway: V2 offers best area efficiency (4.19x) for typical FPGA use cases. V3 provides marginal throughput improvement (7.0x vs 6.7x) at higher area cost, justified only for high-performance memory controllers that support out-of-order responses.

## 21.4	Clock and Reset Summary

### 21.4.1	Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | Primary - all STREAM logic, AXI/AXIL interfaces |
| pclk | 50-200 MHz | APB configuration interface (may be async to aclk) |

### 21.4.2	Reset Signals

| Reset | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary - all STREAM logic |
| presetn | Active-low | Async assert, sync deassert | APB configuration interface |

See: Clocks and Reset for complete timing specifications

## 21.5	Interface Summary

### 21.5.1	External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| APB | Slave | 32-bit | Configuration registers |
| AXI (Descriptor) | Master | 256-bit | Descriptor fetch |
| AXI (Read) | Master | 512-bit (param) | Source data read |
| AXI (Write) | Master | 512-bit (param) | Destination data write |
| AXIL (Slave) | Slave | 32-bit | Error/interrupt FIFO access |
| AXIL (Master) | Master | 32-bit | MonBus packet logging to memory |
| IRQ | Output | 1-bit | Error interrupt |

### 21.5.2	Internal Buses

| Interface | Width | Purpose |
| --- | --- | --- |
| MonBus | 64-bit | Internal monitoring bus (channels -> monbus_axil_group) |

## 21.6	Area Estimates

### 21.6.1	By Performance Mode

| Configuration | Total LUTs | SRAM | Use Case |
| --- | --- | --- | --- |
| Low (Tutorial) | ~9,500 | 64 KB | Educational, area-constrained |
| Medium (Typical) | ~11,200 | 64 KB | Balanced FPGA implementations |
| High (Performance) | ~13,700 | 64 KB | High-throughput ASIC/FPGA |

### 21.6.2	Breakdown (Low Performance)

| Component | Instances | Area/Instance | Total |
| --- | --- | --- | --- |
| Descriptor Engine | 8 | ~300 LUTs | ~2,400 LUTs |
| Scheduler | 8 | ~400 LUTs | ~3,200 LUTs |
| AXI Read Engine (Low) | 1 | ~250 LUTs | ~250 LUTs |
| AXI Write Engine (Low) | 1 | ~250 LUTs | ~250 LUTs |
| SRAM Controller | 1 | ~1,600 LUTs | ~1,600 LUTs |
| Simple SRAM (internal) | 1-8 | 1024x64B total | 64 KB |
| Channel Arbiter | 3 | ~150 LUTs | ~450 LUTs |
| APB Config | 1 | ~350 LUTs | ~350 LUTs |
| MonBus AXIL Group | 1 | ~1,000 LUTs | ~1,000 LUTs |
| Total | - | - | ~9,500 LUTs + 64KB |

## 21.7	Related Documentation

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- Register Generation - PeakRDL workflow

## 21.8	Specification Conventions

### 21.8.1	Signal Naming

- Clock: aclk, pclk

- Reset: aresetn, presetn (active-low)

- Valid/Ready: Standard AXI/custom handshake

- Registers: r_ prefix (e.g., r_state, r_counter)

- Wires: w_ prefix (e.g., w_next_state, w_grant)

### 21.8.2	Parameter Naming

- Uppercase: NUM_CHANNELS, DATA_WIDTH, ADDR_WIDTH

- String parameters: PERFORMANCE (“LOW”, “MEDIUM”, “HIGH”)

### 21.8.3	State Machine Naming

typedef enum logic [3:0] {
    IDLE   = 4'h0,
    ACTIVE = 4'h1,
    // ...
} state_t;

state_t r_state, w_next_state;  // Current and next state

Last Updated: 2026-05-11 Maintained By: STREAM Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Product Requirements Document (PRD)

## 22.1	STREAM - Scatter-gather Transfer Rapid Engine for AXI Memory

Version: 1.0 Date: 2025-10-17 Status: Nearly Complete - Final Integration Pending Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 22.2	1. Executive Summary

The STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory) is a simplified DMA-style accelerator designed for efficient memory-to-memory data movement with descriptor-based scatter-gather support. STREAM serves as a beginner-friendly tutorial demonstrating descriptor-based DMA engine design patterns while maintaining production-quality RTL practices.

### 22.2.1	1.1 Quick Stats

- Modules: ~8-10 SystemVerilog files (estimated)

- Channels: Maximum 8 independent channels

- Interfaces: APB (config), AXI4 (descriptor fetch + data read/write), MonBus (monitoring)

- Architecture: Simplified from RAPIDS - pure memory-to-memory

- Tutorial Focus: Aligned addresses only, straightforward data flow

- Status: Nearly complete - config interface and top-level wrapper pending

### 22.2.2	1.2 Project Goals

- Primary: Educational DMA engine demonstrating scatter-gather descriptor chains

- Secondary: Production-quality RTL suitable for FPGA/ASIC implementation

- Tertiary: Foundation for understanding more complex DMA architectures (e.g., RAPIDS)

## 22.3	2. Key Design Principles

### 22.3.1	2.1 Simplifications from RAPIDS

STREAM is intentionally simplified for tutorial purposes:

| Feature | RAPIDS | STREAM |
| --- | --- | --- |
| Network Interfaces | Yes (Network master/slave) | ✗ No (pure memory-to-memory) |
| Address Alignment | Complex fixup logic | ✓ Aligned addresses only |
| Credit Management | Exponential encoding | ✓ Simple transaction limits |
| Control Engines | Control read/write engines | ✗ No (direct APB config) |
| Descriptor Length | Chunks (4-byte) | ✓ Beats (data width) |
| Program Engine | Complex alignment FSM | ✓ Simplified coordination |

### 22.3.2	2.2 Tutorial-Friendly Features

- Aligned Addresses: Source and destination addresses must be aligned to data width

- Length in Beats: Descriptor length specified in data beats (not bytes/chunks)

- Single APB Write: One APB register write kicks off entire descriptor chain

- No Circular Buffers: Chained descriptors with explicit termination

- Parameterized Engines: Multiple AXI engine versions (compile-time selection)

## 22.4	3. Architecture Overview

### 22.4.1	3.1 Top-Level Block Diagram

STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory)
├── APB Config Slave
│   └── Channel registers (8 channels, kick-off via write)
│
├── Descriptor Fetch
│   └── AXI Master (Read) - Fetch descriptors from memory
│
├── Scheduler Group (shared across all channels)
│   ├── Descriptor Engine   (FIFO, parsing - FROM RAPIDS)
│   ├── Scheduler           (Simplified FSM coordination)
│   └── Channel Arbitration (8 independent channels)
│
├── Data Path (shared resources)
│   ├── AXI Master (Read)  - Parameterized data width
│   ├── Simple SRAM        - Dual-port buffer (FROM RAPIDS)
│   └── AXI Master (Write) - Parameterized data width
│
└── MonBus Reporter
    └── MonBus Master - 64-bit monitoring packets

### 22.4.2	3.2 Data Flow

Descriptor-Based Transfer Sequence:

1. Software writes to APB channel register
   ↓
2. Descriptor fetch via AXI descriptor master
   ↓
3. Descriptor Engine parses descriptor fields
   ↓
4. Scheduler coordinates data transfer:
   a. AXI Read Engine fetches source data → SRAM buffer
   b. AXI Write Engine writes SRAM → destination
   ↓
5. Check for chained descriptor (next_descriptor_ptr != 0)
   ↓ (if chained)
6. Fetch next descriptor, repeat from step 3
   ↓ (if last)
7. Generate MonBus completion packet

Channel Independence: - 8 channels operate independently - All channels share: SRAM, AXI data masters, descriptor fetch master - Arbitration required for shared resources

## 22.5	4. Interfaces

### 22.5.1	4.1 External Interfaces

| Interface | Type | Width | Purpose | Notes |
| --- | --- | --- | --- | --- |
| APB Slave | Slave | 32-bit | Configuration, channel kick-off | Write to channel register starts transfer |
| AXI Master (Descriptor) | Master | 256-bit | Fetch descriptors from memory | Dedicated descriptor fetch path |
| AXI Master (Data Read) | Master | Parameterizable | Read source data | Multiple engine versions (compile-time) |
| AXI Master (Data Write) | Master | Parameterizable | Write destination data | Multiple engine versions (compile-time) |
| MonBus Master | Master | 64-bit | Monitor packet output | Standard AMBA format |

### 22.5.2	4.2 Descriptor Format

256-bit Descriptor Structure:

| Bits | Field | Description |
| --- | --- | --- |
| [63:0] | src_addr | Source address (64-bit, must be aligned to data width) |
| [127:64] | dst_addr | Destination address (64-bit, must be aligned to data width) |
| [159:128] | length | Transfer length in BEATS (not bytes!) |
| [191:160] | next_descriptor_ptr | Address of next descriptor (0 = last in chain) |
| [192] | valid | Descriptor is valid |
| [193] | interrupt | Generate interrupt on completion |
| [194] | last | Last descriptor in chain (explicit flag) |
| [195] | error | Error status (used for reporting) |
| [199:196] | channel_id | Channel ID (0-7) |
| [207:200] | priority | Transfer priority (for arbitration) |
| [255:208] | reserved | Reserved for future use |

Key Descriptor Features: - ✓ Chained descriptors: next_descriptor_ptr links to next descriptor - ✗ No circular buffers: Explicit termination (last flag or ptr=0) - ✓ Length in beats: Simplified for tutorial (no byte/chunk conversion) - ✓ Aligned addresses: Tutorial constraint (performance hidden for now)

## 22.6	5. Key Components

### 22.6.1	5.1 Descriptor Engine (APB-Only for STREAM)

Source: Adapted from RAPIDS descriptor_engine.sv

Purpose: - Autonomous descriptor fetch and chaining - APB interface for initial descriptor address - AXI read interface for descriptor memory fetches - Descriptor FIFO storage and distribution

Key Features: - ✓ Autonomous chaining: Automatically fetches next descriptor if next_descriptor_ptr != 0 AND last == 0 - ✓ Address validation: Validates next descriptor addresses against cfg_addr0/1_base/limit - ✓ APB blocking: APB blocked until channel_idle == 1 (channel fully idle) - ✓ Error handling: AXI errors stop chaining, set descriptor_error, block descriptor_valid

Adaptations from RAPIDS: - ✓ APB-only: Single APB write kicks off entire descriptor chain - ✓ Descriptor Read Address FIFO: 2-deep FIFO stores addresses for AXI fetch (APB + chaining) - ✓ Chaining logic: Descriptor engine autonomously manages next_descriptor_ptr chaining

Idle Signal: - descriptor_engine_idle asserted when: - FSM in RD_IDLE state - No pending descriptor fetches (address FIFO empty) - No active AXI transactions

### 22.6.2	5.2 Scheduler Group (Integration Wrapper)

Purpose: Wraps descriptor engine and scheduler into a single channel processing unit

Architecture:

scheduler_group (
    // APB interface (from APB config slave)
    .apb_valid         (apb_valid),
    .apb_ready         (apb_ready),   // Blocked when channel not idle
    .apb_addr          (descriptor_addr),

    // Channel idle signal composition (CRITICAL!)
    .channel_idle      (channel_idle),

    // Descriptor → Scheduler flow
    .descriptor_valid  (desc_valid),
    .descriptor_ready  (desc_ready),
    .descriptor_packet (desc_packet),

    // Data engine interfaces
    .datard_*          (datard_*),    // Read engine
    .datawr_*          (datawr_*),    // Write engine

    // Status
    .scheduler_idle    (sched_idle),
    .descriptor_idle   (desc_idle)
);

Channel Idle Signal Composition:

// Channel is idle ONLY when BOTH sub-blocks are idle
assign channel_idle = scheduler_idle && descriptor_engine_idle;

Why Both Signals Matter:

| Signal | Indicates | Used For |
| --- | --- | --- |
| scheduler_idle | No active data transfers, all descriptors processed | Prevents new APB request during active transfer |
| descriptor_engine_idle | No pending descriptor fetches (FIFO empty) | Prevents new APB request during chaining |
| channel_idle (AND of both) | Channel fully quiescent | Gates APB interface |

APB Blocking Logic:

// Descriptor engine blocks APB when channel not idle
assign apb_ready = apb_skid_ready_in &&
                   !r_channel_reset_active &&
                   w_desc_addr_fifo_empty &&    // No pending fetches
                   channel_idle;                 // Scheduler + descriptor idle

Example Scenario:

1. Software writes APB → descriptor_addr = 0x1000
   - channel_idle = 1 (both idle)
   - APB accepted

2. Descriptor engine fetches descriptor @ 0x1000
   - descriptor_engine_idle = 0 (fetch in progress)
   - channel_idle = 0
   - APB BLOCKED

3. Descriptor pushed to scheduler
   - descriptor_engine_idle = 1 (fetch complete)
   - scheduler_idle = 0 (transfer starting)
   - channel_idle = 0
   - APB BLOCKED

4. Scheduler completes data transfer
   - Descriptor has next_descriptor_ptr = 0x1100 (chained!)
   - Descriptor engine autonomously fetches @ 0x1100
   - descriptor_engine_idle = 0 (autonomous fetch)
   - channel_idle = 0
   - APB BLOCKED

5. Final descriptor completes (last = 1 OR next_ptr = 0)
   - scheduler_idle = 1 (transfer done)
   - descriptor_engine_idle = 1 (no more fetches)
   - channel_idle = 1
   - APB UNBLOCKED (ready for next transfer!)

Key Insight: The AND gate ensures software cannot interrupt a descriptor chain in progress!

### 22.6.3	5.3 Scheduler (Simplified from RAPIDS)

Purpose: - Coordinate descriptor-to-data-transfer flow - Manage 8 independent channels - Arbitrate shared resources (SRAM, AXI masters)

FSM States:

typedef enum logic [7:0] {
    SCHED_IDLE              = 8'b00000001,  // Idle, waiting for channel activation
    SCHED_FETCH_DESCRIPTOR  = 8'b00000010,  // Fetch descriptor via AXI master
    SCHED_PARSE_DESCRIPTOR  = 8'b00000100,  // Parse descriptor fields
    SCHED_READ_PHASE        = 8'b00001000,  // Coordinate read engine
    SCHED_WRITE_PHASE       = 8'b00010000,  // Coordinate write engine
    SCHED_CHAIN_CHECK       = 8'b00100000,  // Check for next descriptor
    SCHED_COMPLETE          = 8'b01000000,  // Transfer complete, report status
    SCHED_ERROR             = 8'b10000000   // Error state
} scheduler_state_t;

Key Differences from RAPIDS: - ✗ No credit management (just simple transaction limits) - ✗ No program engine coordination (no alignment fixup) - ✓ Simplified FSM (no control read/write phases)

### 22.6.4	5.3 AXI Read Engine (Streaming Pipeline - NO FSM)

Purpose: High-performance streaming reads from memory to SRAM buffer

Architecture: Pipelined streaming design (NO FSM for performance)

Key Insight: FSMs are horrible for performance! Instead, use: - Arbiter selects which channel’s datard_* interface gets access - Streaming pipeline continuously moves data when granted - Data interface (datard_valid, datard_ready, datard_beats_remaining) controls flow

Data Read Interface (per channel):

// Channel requests read access
input  logic        datard_valid;          // Channel has read request
output logic        datard_ready;          // Engine ready for request
input  logic [63:0] datard_addr;           // Source address (aligned)
input  logic [31:0] datard_beats_remaining; // Beats left to read
input  logic [7:0]  datard_burst_len;      // Preferred burst length
input  logic [3:0]  datard_channel_id;     // Channel ID for tracking

Multiple Versions (Compile-Time Selection): 1. Version 1 - Basic: Single outstanding read, fixed burst length 2. Version 2 - Pipelined: Multiple outstanding reads, configurable bursts 3. Version 3 - Adaptive: Dynamic burst sizing based on remaining beats

Operation:

1. Arbiter grants channel access based on priority
2. Engine accepts datard_* request (continuous streaming)
3. AXI AR channel issues read burst
4. AXI R channel streams data → SRAM (no FSM stalls!)
5. Engine updates beats_remaining, accepts next request
6. Different channels can have different burst lengths (e.g., CH0: 8 beats, CH1: 16 beats)

Example: Different Burst Lengths per Channel

// Channel 0 prefers 8-beat bursts
datard_burst_len[0] = 8'd8;

// Channel 1 prefers 16-beat bursts
datard_burst_len[1] = 8'd16;

// Engine adapts to requested burst length (within MAX_BURST_LEN)

### 22.6.5	5.4 AXI Write Engine (Streaming Pipeline - NO FSM)

Purpose: High-performance streaming writes from SRAM buffer to memory

Architecture: Pipelined streaming design (NO FSM for performance)

Key Insight: Same as read engine - no FSMs! Use streaming pipeline with arbiter.

Data Write Interface (per channel):

// Channel requests write access
input  logic        datawr_valid;           // Channel has write request
output logic        datawr_ready;           // Engine ready for request
input  logic [63:0] datawr_addr;            // Destination address (aligned)
input  logic [31:0] datawr_beats_remaining; // Beats left to write
input  logic [7:0]  datawr_burst_len;       // Preferred burst length
input  logic [3:0]  datawr_channel_id;      // Channel ID for tracking

Multiple Versions (Compile-Time Selection): 1. Version 1 - Basic: Single outstanding write, fixed burst length 2. Version 2 - Pipelined: Multiple outstanding writes, configurable bursts 3. Version 3 - Adaptive: Dynamic burst sizing based on remaining beats

Operation:

1. Arbiter grants channel access based on priority
2. Engine accepts datawr_* request (continuous streaming)
3. Engine reads data from SRAM
4. AXI AW channel issues write address
5. AXI W channel streams data (no FSM stalls!)
6. AXI B channel receives response, updates beats_remaining
7. Different channels can have different burst lengths (e.g., CH0: 8 beats, CH2: 32 beats)

Read/Write Asymmetry Example:

// Channel can use different burst lengths for read vs write
// Example: Read in small bursts, write in large bursts
datard_burst_len[0] = 8'd8;   // Read: 8 beats
datawr_burst_len[0] = 8'd16;  // Write: 16 beats

// Engine handles asymmetry via SRAM buffering

### 22.6.6	5.5 SRAM Buffering

Source: Originally a direct copy of RAPIDS simple_sram.sv; now implemented by rtl/fub/sram_controller.sv + sram_controller_unit.sv (wrapping the shared gaxi_fifo_sync primitive)

Purpose: - Dual-port SRAM buffer - Decouples read and write engines - Shared across all channels (arbitration required)

Why Reuse: - Standard dual-port SRAM design - Proven in RAPIDS integration tests - Parameterizable depth and width

## 22.7	6. Configuration and Control

### 22.7.1	6.1 APB Register Map

| Offset | Register | Access | Description |
| --- | --- | --- | --- |
| 0x0000 | GLOBAL_CTRL | RW | Global enable, reset |
| 0x0004 | GLOBAL_STATUS | RO | Global status, error flags |
| 0x0100 | CH0_CTRL | WO | Channel 0 kick-off (write descriptor address) |
| 0x0104 | CH0_STATUS | RO | Channel 0 status |
| 0x0108 | CH0_DESC_ADDR | RO | Channel 0 current descriptor address |
| 0x010C | CH0_BYTES_XFER | RO | Channel 0 bytes transferred |
| … | … | … | … (repeat for channels 1-7) |
| 0x0200 | CH1_CTRL | WO | Channel 1 kick-off |
| … |  |  |  |
| 0x0700 | CH7_CTRL | WO | Channel 7 kick-off |

Kick-Off Sequence: 1. Software writes descriptor address to CHx_CTRL register 2. STREAM fetches descriptor from memory 3. Transfer begins automatically 4. If chained, STREAM follows next_descriptor_ptr automatically 5. Completion reported via MonBus packet

### 22.7.2	6.2 Channel Configuration

Each channel independently configurable: - Descriptor start address: Written to CHx_CTRL - Priority: Encoded in descriptor (arbitration) - Interrupt enable: Per-descriptor flag - Status monitoring: Read CHx_STATUS

## 22.8	7. Resource Sharing and Arbitration

### 22.8.1	7.1 Shared Resources

All channels share: 1. Descriptor Fetch AXI Master - Fetches descriptors for all channels 2. Data Read AXI Master - Reads source data for all channels 3. Data Write AXI Master - Writes destination data for all channels 4. SRAM Buffer - Shared buffer (dual-port, but still arbitrated) 5. MonBus Reporter - Single monitor output

### 22.8.2	7.2 Arbitration Strategy

Priority-Based Round-Robin: - Channels have priority field in descriptor - Higher priority = serviced first - Within same priority: round-robin - Prevents starvation with timeout

Example Arbitration:

Channel 0: Priority 7 (highest)
Channel 1: Priority 5
Channel 2: Priority 5
Channel 3: Priority 3

Service order: CH0 → CH1 → CH2 (round-robin) → CH0 → CH1 → CH2 → CH3 …

## 22.9	8. Error Detection and Recovery

### 22.9.1	8.1 Error Types

| Error Type | Detection | Response |
| --- | --- | --- |
| Invalid descriptor | Valid bit = 0 | Skip, move to next |
| Alignment error | Address not aligned | Set error flag, halt channel |
| AXI SLVERR | AXI response | Set error flag, halt channel |
| AXI DECERR | AXI response | Set error flag, halt channel |
| Timeout | Transaction timeout | Set error flag, halt channel |
| SRAM overflow | Buffer full | Backpressure, wait |

### 22.9.2	8.2 Error Recovery

Per-Channel Error Handling: - Error sets channel to CH_ERROR state - Channel halts, does not affect other channels - Software must: 1. Read CHx_STATUS to identify error 2. Clear error condition 3. Re-kick channel with new descriptor

No Automatic Retry: - Tutorial design keeps error handling simple - Software responsible for retry logic

## 22.10	9. MonBus Integration

### 22.10.1	9.1 Standard MonBus Format

Uses standard 64-bit MonBus packet format: - [63:60] Packet Type (0=ERROR, 1=COMPL, etc.) - [59:57] Protocol (custom STREAM protocol) - [56:53] Event Code (STREAM-specific events) - [52:47] Channel ID (0-7) - [46:43] Unit ID (unused for STREAM) - [42:35] Agent ID (unused for STREAM) - [34:0] Event Data (address, byte count, etc.)

### 22.10.2	9.2 STREAM Event Codes

| Code | Event | Description |
| --- | --- | --- |
| 0x0 | DESC_START | Descriptor started |
| 0x1 | DESC_COMPLETE | Descriptor completed |
| 0x2 | READ_START | Read phase started |
| 0x3 | READ_COMPLETE | Read phase completed |
| 0x4 | WRITE_START | Write phase started |
| 0x5 | WRITE_COMPLETE | Write phase completed |
| 0x6 | CHAIN_FETCH | Chained descriptor fetch |
| 0xF | ERROR | Error occurred |

### 22.10.3	9.3 Default Configuration

Tutorial-Friendly Defaults: - Errors only: cfg_error_enable = 1, all others = 0 - Interrupts: Descriptor flag controls per-transfer interrupt - Reduces MonBus traffic for beginner understanding

## 22.11	10. Design Constraints

### 22.11.1	10.1 Tutorial Constraints (Intentional Simplifications)

| Constraint | Rationale |
| --- | --- |
| Aligned addresses only | Simplify data path, hide alignment complexity |
| Length in beats | Avoid byte/chunk conversion math |
| No circular buffers | Explicit termination easier to understand |
| Single APB kick-off | Simple software model |
| No credit management | Avoid exponential encoding complexity |

### 22.11.2	10.2 Implementation Constraints

| Parameter | Value | Notes |
| --- | --- | --- |
| Max channels | 8 | Compile-time parameter |
| Max burst length | 256 | AXI4 spec limit |
| Descriptor size | 256 bits | 4 × 64-bit words |
| SRAM depth | Parameterizable | Typical: 1024-4096 entries |
| Data width | Parameterizable | Typical: 512-bit |

## 22.12	11. Verification Strategy

### 22.12.1	11.1 Test Organization

projects/components/dma-ip/stream/dv/tests/
├── fub/                        # Functional Unit Block tests
│   ├── test_descriptor_engine.py
│   ├── test_scheduler.py
│   ├── test_sram_controller.py (+ test_sram_controller_alloc.py)
│   ├── test_perf_profiler.py
│   └── test_stream_latency_bridge.py
├── macro/                      # Multi-block scenarios (stream_core, datapaths)
└── top/                        # Top-level (stream_top_ch8) tests

### 22.12.2	11.2 Test Levels

Basic (Quick Smoke Tests): - Single descriptor, single channel - Aligned addresses, simple transfers - ~30 seconds runtime

Medium (Typical Scenarios): - Multiple descriptors, 2-4 channels - Chained descriptors (2-3 deep) - ~90 seconds runtime

Full (Comprehensive Validation): - All 8 channels concurrent - Long descriptor chains (10+ deep) - Error injection, stress testing - ~180 seconds runtime

## 22.13	12. Performance Characteristics

### 22.13.1	12.1 Throughput by Engine Version

V1 (Low Performance - Tutorial Mode): - Throughput: 0.14 beats/cycle (DDR4), 0.40 beats/cycle (SRAM) - Architecture: Single outstanding transaction, blocks on completion - Use Case: Tutorial examples, embedded systems, low-latency SRAM

V2 (Medium Performance - Command Pipelined): - Throughput: 0.94 beats/cycle (DDR4), 0.85 beats/cycle (SRAM) - Improvement: 6.7x over V1 (DDR4), 2.1x over V1 (SRAM) - Architecture: Command queue (4-8 deep), hides memory latency - Use Case: General-purpose FPGA, DDR3/DDR4, best area efficiency

V3 (High Performance - Out-of-Order): - Throughput: 0.98 beats/cycle (DDR4), 0.92 beats/cycle (SRAM) - Improvement: 7.0x over V1 (DDR4), 2.3x over V1 (SRAM) - Architecture: OOO command selection, maximizes memory controller efficiency - Use Case: Datacenter, ASIC, HBM2, high-performance memory

Key Insight: Benefit scales with memory latency. V1 throughput degrades from 40% (SRAM) to 14% (DDR4), while V2/V3 maintain 94-98% throughput regardless of latency.

Configuration Parameters: - ENABLE_CMD_PIPELINE = 0: V1 (default, tutorial mode) - ENABLE_CMD_PIPELINE = 1: V2 (command pipelined) - ENABLE_CMD_PIPELINE = 1, ENABLE_OOO_DRAIN = 1 (write) or ENABLE_OOO_READ = 1 (read): V3

Factors Affecting Throughput: - Memory latency (V2/V3 hide latency via pipelining) - SRAM buffer size (limits burst pipelining) - Channel arbitration overhead - Descriptor fetch latency - Engine version (V1/V2/V3 configuration)

### 22.13.2	12.2 Latency

Transfer Latency Breakdown: - Descriptor fetch: ~10-50 cycles (depends on memory) - Read phase: (length / burst_len) × burst_latency - Write phase: (length / burst_len) × burst_latency - End-to-end: Typically <200 cycles for small transfers (V1), <100 cycles (V2/V3 pipelined)

V2/V3 Latency Hiding: - Command pipelining overlaps descriptor fetch, read, write operations - Multiple outstanding transactions hide memory latency - OOO completion (V3) reduces head-of-line blocking

### 22.13.3	12.3 Resource Utilization by Engine Version

V1 Configuration (Tutorial - Minimum Area): - Total: ~9,500 LUTs + 64 KB SRAM - Descriptor Engine (8×): ~2,400 LUTs - Scheduler (8×): ~3,200 LUTs - AXI Read Engine: ~1,250 LUTs - AXI Write Engine: ~1,250 LUTs - SRAM Controller: ~1,600 LUTs - APB Config: ~350 LUTs - MonBus AXIL Group: ~1,000 LUTs

V2 Configuration (Balanced - Best Area Efficiency): - Total: ~11,000 LUTs + 64 KB SRAM (1.16x area) - AXI Read Engine: ~2,000 LUTs (1.6x increase, 6.7x throughput) - AXI Write Engine: ~2,500 LUTs (2.0x increase, 6.7x throughput) - Other blocks: Same as V1

V3 Configuration (Maximum Performance): - Total: ~14,000 LUTs + 64 KB SRAM (1.47x area) - AXI Read Engine: ~3,500 LUTs (2.8x increase, 7.0x throughput) - AXI Write Engine: ~4,000 LUTs (3.2x increase, 7.0x throughput) - Other blocks: Same as V1

Area Efficiency Comparison: - V1: 1.00 throughput / 1.00 area = 1.00 - V2: 6.70 throughput / 1.16 area = 5.78 (best efficiency) - V3: 7.00 throughput / 1.47 area = 4.76

Recommendation: V2 provides best area efficiency for most use cases. V3 justified only for high-performance memory controllers that support OOO responses.

## 22.14	13. Development Roadmap

### 22.14.1	13.1 Phase 1: Foundation (Current)

- ✓ Directory structure

- ✓ Package definitions (stream_pkg.sv)

- ✓ Imports header (stream_imports.svh)

- ⏳ Documentation (this PRD)

### 22.14.2	13.2 Phase 2: Core Blocks

- Adapt descriptor engine from RAPIDS

- Design simplified scheduler

- Create APB config interface

- Copy simple SRAM from RAPIDS

### 22.14.3	13.3 Phase 3: Data Path

- AXI read engine (version 1 - basic)

- AXI write engine (version 1 - basic)

- SRAM integration

- Channel arbitration

### 22.14.4	13.4 Phase 4: Integration

- Top-level module

- MonBus reporter

- Integration testbench

- Single-channel validation

### 22.14.5	13.5 Phase 5: Multi-Channel

- 8-channel support

- Arbiter implementation

- Multi-channel tests

- Performance tuning

### 22.14.6	13.6 Phase 6: Advanced Engines (Future - V2/V3)

Goal: Add parameterized high-performance engine variants

V2 - Command Pipelined (Medium Performance): - Command queue implementation (4-8 deep) - W drain FSM for write engine - B response scoreboard (write) or in-order R reception (read) - Per-command SRAM pointer tracking - Parameter: ENABLE_CMD_PIPELINE = 1 - Expected: 6.7x throughput improvement over V1

V3 - Out-of-Order Completion (High Performance): - OOO command selection logic - Transaction ID matching (AXI ID to queue entry) - SRAM data availability checking (write engine) - R beat matching to queue entry (read engine) - Parameters: ENABLE_CMD_PIPELINE = 1, ENABLE_OOO_DRAIN = 1 (write) or ENABLE_OOO_READ = 1 (read) - Expected: 7.0x throughput improvement over V1

Deliverables: - Updated RTL with parameterization - Performance comparison tests (V1 vs V2 vs V3) - Tutorial documentation explaining trade-offs - Area/throughput measurements on target FPGA

## 22.15	14. Educational Value

### 22.15.1	14.1 Learning Objectives

STREAM teaches: 1. Descriptor-based DMA design patterns - Descriptor structure and parsing - Chained descriptors (scatter-gather) - Descriptor fetch via AXI

- AXI4 Memory Interface Usage

  - Burst transactions

  - Address/data/response channels

  - Outstanding transactions

- Resource Sharing and Arbitration

  - Multiple channels sharing AXI masters

  - Priority-based arbitration

  - Conflict resolution

- FSM Design and Coordination

  - Multiple interconnected FSMs

  - State machine composition

  - Error handling

- Buffer Management

  - SRAM-based buffering

  - Rate matching

  - Flow control

### 22.15.2	14.2 Progression Path

Learning Sequence: 1. STREAM (this project): Memory-to-memory DMA, aligned addresses 2. STREAM Extended: Add alignment fixup, more complex scenarios 3. RAPIDS: Add network interfaces, credit management, full complexity

## 22.16	15. Success Criteria

### 22.16.1	15.1 Functional

- ✓ Single descriptor transfer working

- ✓ Chained descriptors (2-3 deep)

- ✓ Multi-channel operation (8 channels concurrent)

- ✓ Error detection and reporting

- ✓ MonBus packet generation

- ✓ >90% functional test coverage

### 22.16.2	15.2 Quality

- ✓ Verilator compiles with 0 warnings

- ✓ All tests passing (100% success rate)

- ✓ Comprehensive documentation

- ✓ Tutorial-quality code comments

### 22.16.3	15.3 Performance

- ✓ Achieves >80% of theoretical AXI bandwidth

- ✓ <5K LUT utilization (excluding SRAM)

- ✓ <200 cycle end-to-end latency for small transfers

## 22.17	16. Open Questions (For Review)

### 22.17.1	16.1 Descriptor Engine Adaptation

- Q: Should descriptor engine use APB-only, or support additional modes?

- A (pending): TBD - depends on software use case preference

### 22.17.2	16.2 AXI Descriptor Master

- Q: Fixed 256-bit width, or parameterizable?

- A (pending): Propose fixed 256-bit for simplicity

### 22.17.3	16.3 Channel Arbitration

- Q: Fixed priority, or dynamic priority based on age/fairness?

- A (pending): Propose fixed priority with round-robin for tutorial simplicity

### 22.17.4	16.4 SRAM Partitioning

- Q: Single shared SRAM, or per-channel SRAMs?

- A (pending): Propose single shared SRAM with arbitration (matches RAPIDS pattern)

## 22.18	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/dma-ip/stream/docs/

Quick Command: Use the provided shell scripts:

cd projects/components/dma-ip/stream/docs
./generate_has_pdf.sh   # STREAM HAS
./generate_mas_pdf.sh   # STREAM MAS

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the stream_has/ and stream_mas/ index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 22.19	17. References

### 22.19.1	17.1 Internal Documentation

- RAPIDS PRD: projects/components/dma-ip/rapids/PRD.md - Parent architecture

- RAPIDS Descriptor Engine: projects/components/dma-ip/rapids/rtl/fub_beats/descriptor_engine_beats.sv (beats rearchitecture of the original)

- Shared SRAM/FIFO primitives: rtl/amba/gaxi/gaxi_fifo_sync.sv, rtl/amba/shared/sdpram_core.sv (the old simple_sram.sv was removed)

- AMBA PRD: docs/markdown/rtl-amba/index.md - MonBus integration

- Repository Guide: /CLAUDE.md - Design patterns and conventions

### 22.19.2	17.2 External References

- AXI4 Specification: ARM IHI0022E

- APB Specification: ARM IHI0024C

- CocoTB Documentation: https://docs.cocotb.org/

- Verilator Manual: https://verilator.org/guide/latest/

Document Version: 1.0 Last Updated: 2025-10-17 Review Cycle: Weekly during initial design Next Review: TBD (after user feedback) Owner: RTL Design Sherpa Project

## 22.20	Navigation

- ← Back to Root: /PRD.md

- Parent Architecture: projects/components/dma-ip/rapids/PRD.md

- AI Guidance: CLAUDE.md (to be created)

- Quick Start: README.md (to be created)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Claude Code Guide: STREAM Subsystem

Version: 1.0 Last Updated: 2025-10-17 Purpose: AI-specific guidance for working with STREAM subsystem

## 23.1	Quick Context

What: STREAM - Scatter-gather Transfer Rapid Engine for AXI Memory Status: ✓ Complete - tutorial-focused DMA engine (fub/macro/top all passing; on-board characterization flows exist) Your Role: Help users build a beginner-friendly descriptor-based DMA engine

📖 Complete Specification: projects/components/dma-ip/stream/PRD.md ← Always reference this for technical details

## 23.2	📖 Global Requirements Reference

IMPORTANT: Review /GLOBAL_REQUIREMENTS.md for all mandatory requirements

All mandatory requirements are consolidated in the global requirements document: - See: /GLOBAL_REQUIREMENTS.md - Repository-wide mandatory requirements - STREAM-Specific: Attribution format, tutorial focus (intentional simplifications) - Universal: TB location, three methods, TBBase inheritance, 100% success

This CLAUDE.md provides STREAM-specific guidance. Also review: - Root /CLAUDE.md - Repository-wide patterns - projects/components/CLAUDE.md - Project area standards (reset macros, FPGA attributes) - bin/TBClasses/ - Shared TB framework (full CocoTBFramework in the RTLDesignSherpa-DV repo)

## 23.3	Critical Rules for This Subsystem

### 23.3.1	Rule #0.1: TUTORIAL FOCUS - Intentional Simplifications

⚠ STREAM is INTENTIONALLY SIMPLIFIED for educational purposes ⚠

Key Simplifications (DO NOT “fix” these): 1. ✓ Aligned addresses only - No alignment fixup logic (kept for RAPIDS) 2. ✓ Length in beats - Not bytes or chunks (simplifies math) 3. ✓ No circular buffers - Explicit chain termination only 4. ✓ No credit management - Simple transaction limits 5. ✓ Pure memory-to-memory - No network interfaces

When users ask “Can we add alignment fixup?”: - ✓ Correct answer: “STREAM intentionally keeps addresses aligned for tutorial simplicity. For complex alignment, see RAPIDS.” - ✗ Wrong answer: “Sure, let me add alignment logic…” (defeats tutorial purpose!)

### 23.3.2	Rule #0.1: Testbench Location and Test Structure (MANDATORY)

📖 See: /GLOBAL_REQUIREMENTS.md Section 2.1 for complete requirement

STREAM-Specific Directory Structure:

projects/components/dma-ip/stream/dv/
├── tbclasses/                    # ★ STREAM TB classes (project area!)
│   ├── scheduler_tb.py           # Scheduler testbench
│   ├── descriptor_engine_tb.py   # Descriptor engine testbench
│   ├── sram_controller_tb.py     # SRAM controller testbench
│   ├── stream_core_tb.py         # Macro-level core testbench
│   └── stream_top_tb.py          # Top-level testbench
└── tests/                        # Test runners import from tbclasses/
    ├── fub/                      # Per-block tests
    ├── macro/                    # Integration tests
    └── top/                      # Top-level tests

STREAM Import Pattern:

# Import STREAM TB from project area
from projects.components.dma_ip.stream.dv.tbclasses.scheduler_tb import SchedulerTB

# Shared utilities from framework
from TBClasses.shared.tbbase import TBBase

📖 Complete Pattern: projects/components/dma-ip/rapids/CLAUDE.md Rule #0.1

### 23.3.3	Rule #0.2: Three Mandatory TB Methods (MANDATORY)

📖 See: /GLOBAL_REQUIREMENTS.md Section 2.2 for complete requirement

STREAM-Specific Context:

STREAM has simpler config requirements than RAPIDS - most config can be set after reset.

Standard STREAM Pattern:

class SchedulerTB(TBBase):
    async def setup_clocks_and_reset(self):
        """Standard STREAM initialization"""
        await self.start_clock('clk', freq=10, units='ns')
        await self.assert_reset()
        await self.wait_clocks('clk', 10)
        await self.deassert_reset()
        await self.wait_clocks('clk', 5)

    async def assert_reset(self):
        self.dut.rst_n.value = 0

    async def deassert_reset(self):
        self.dut.rst_n.value = 1

Note: Unlike RAPIDS, STREAM typically doesn’t need config set before reset.

### 23.3.4	Rule #1: REUSE from RAPIDS Where Appropriate

Direct Reuse (No Modification): - ✓ descriptor_engine.sv - Works with STREAM descriptors - ✓ Shared FIFO/SRAM primitives (gaxi_fifo_sync, used inside sram_controller_unit.sv; the old simple_sram.sv was removed) - ✓ Descriptor engine tests - Adapt imports only

Adapt from RAPIDS: - ⚠ scheduler.sv - Simplify FSM (no credit management, no control engines) - ⚠ AXI engines - Create simplified versions (no alignment fixup)

Create New for STREAM: - 📝 APB config interface - PeakRDL-generated (like HPET), 8 channels, kick-off registers - 📝 Top-level integration - Different interface set

Always Ask Yourself: “Can I reuse from RAPIDS instead of creating new?”

### 23.3.5	Rule #2: Descriptor Format is DIFFERENT from RAPIDS

STREAM Descriptor (256-bit): - src_addr (64-bit), dst_addr (64-bit), length (beats), next_descriptor_ptr (32-bit) - Length is in BEATS, not chunks!

RAPIDS Descriptor: - Uses chunks (4-byte units) - Has alignment metadata

When comparing/referencing RAPIDS: - ✓ “RAPIDS uses chunks, STREAM uses beats for tutorial simplicity” - ✗ Don’t assume RAPIDS descriptor format applies to STREAM

### 23.3.6	Rule #3: Know the Shared Resources

All channels (8 or 16, by top) share: 1. Descriptor fetch AXI master – and, by default (SHARED_ENGINES=1), ONE descriptor_engine_shared that fetches for every channel (ARID = channel). SHARED_ENGINES=0 restores one descriptor_engine per channel behind an AR arbiter. 2. Data read AXI master 3. Data write AXI master 4. SRAM buffer 5. MonBus reporter

Arbitration is required! - Never assume exclusive access - All shared resources need arbiter logic - Priority-based round-robin (descriptor priority field)

## 23.4	Architecture Quick Reference

### 23.4.1	Block Organization

STREAM Architecture (as implemented)
├── rtl/fub/                        # Functional unit blocks
│   ├── descriptor_engine.sv        # Descriptor fetch/parse, one per channel (SHARED_ENGINES=0)
│   ├── descriptor_engine_shared.sv # One fetch engine for all channels (default; ported from RAPIDS)
│   ├── scheduler.sv                # Simplified per-channel scheduler
│   ├── axi_read_engine.sv          # Data read engine
│   ├── axi_write_engine.sv         # Data write engine
│   ├── sram_controller.sv          # SRAM buffering (+ sram_controller_unit.sv per channel)
│   ├── stream_alloc_ctrl.sv        # Space allocation tracking
│   ├── stream_drain_ctrl.sv        # Drain control
│   ├── stream_latency_bridge.sv    # Latency decoupling
│   ├── (moved) stream_run_addr_gen.sv -> projects/components/utility-ip/misc/rtl/  # shared with RAPIDS
│   └── perf_profiler.sv            # Performance profiling
│
├── rtl/macro/                      # Integration blocks
│   ├── scheduler_group.sv          # Per-channel scheduler group
│   ├── scheduler_group_array.sv    # NUM_CHANNELS array + descriptor fetch (SHARED_ENGINES)
│   ├── stream_core.sv              # Core integration
│   ├── stream_regs.rdl             # PeakRDL register source (8-channel, hand-written)
│   └── generated_16ch/             # 16-channel RDL, DERIVED by bin/gen_stream_regs_16ch.py
│
└── rtl/top/                        # Top level
    ├── stream_top_ch8.sv           # 8-channel top-level wrapper (hand-written)
    ├── stream_top_ch16.sv          # 16-channel top, GENERATED from stream_top_ch8.sv
    ├── generated/regsel_{8,16}ch/  # regblock/hwif/kick glue includes; the filelist picks one
    └── stream_config_block.sv      # Configuration block

16 channels: edit the 8-channel sources (stream_regs.rdl, stream_mon_regs.rdl, stream_top_ch8.sv), then run python3 bin/gen_stream_regs_16ch.py and regenerate BOTH regblocks with bin/peakrdl_generate.py; --check and bin/check_rdl_regen.py catch a stale 16-channel copy. Software and tests go by register name (rtl/stream_regmap.py / rtl/stream_regmap_16ch.py): the 16-channel map moves CH_STATE to 0x180 and OBS_CTRL.CAT_SEL to [5:4].

Monitoring uses the shared monbus group modules from rtl/amba/ocelus/ (e.g. monbus_axil4_axil4_group.sv) rather than a STREAM-local reporter.

## 23.5	Common User Questions and Responses

### 23.5.1	Q: “How is STREAM different from RAPIDS?”

A: STREAM is intentionally simplified for tutorial purposes:

Simplifications: | Feature | RAPIDS | STREAM | |———|——–|——–| | Interfaces | APB + AXI + Network | APB + AXI only | | Address Alignment | Complex fixup | Aligned only | | Credit Management | Exponential encoding | Simple limits | | Descriptor Length | Chunks (4-byte) | Beats (data width) | | Control Engines | Control read/write | None (direct APB) |

Learning Path: 1. STREAM - Basic DMA with scatter-gather 2. STREAM Extended - Add alignment fixup 3. RAPIDS - Full complexity with network + credit management

📖 See: PRD.md Section 2.1 for complete comparison table

### 23.5.2	Q: “How do I kick off a transfer?”

A: Single APB write starts descriptor chain:

// Software writes descriptor address to channel register
write_apb(ADDR_CH0_CTRL, 0x1000_0000);  // Start address of descriptor

// STREAM automatically:
// 1. Fetches descriptor from 0x1000_0000
// 2. Parses src_addr, dst_addr, length
// 3. Reads source data → SRAM
// 4. Writes SRAM → destination
// 5. Checks next_descriptor_ptr
//    - If != 0: Fetch next descriptor, repeat
//    - If == 0 or last flag set: Complete

Chained Descriptors:

Descriptor 0 @ 0x1000_0000:
  src_addr = 0x2000_0000
  dst_addr = 0x3000_0000
  length = 64 beats
  next_descriptor_ptr = 0x1000_0100  ← Points to next descriptor

Descriptor 1 @ 0x1000_0100:
  src_addr = 0x2000_1000
  dst_addr = 0x3000_1000
  length = 32 beats
  next_descriptor_ptr = 0x0000_0000  ← Last descriptor (0 = stop)

📖 See: PRD.md Section 3.2 for complete data flow

### 23.5.3	Q: “How many channels can I use?”

A: 8 or 16 independent channels, by top (stream_top_ch8 / stream_top_ch16):

Channel Operation: - Each channel has own FSM state - Each channel can have separate descriptor chain - All channels share: AXI masters, SRAM, MonBus

Arbitration: - Priority-based (descriptor priority field) - Round-robin within same priority - Prevents starvation with timeout

Example:

// Kick off 3 channels concurrently
write_apb(ADDR_CH0_CTRL, desc0_addr);  // Channel 0: Priority 7
write_apb(ADDR_CH1_CTRL, desc1_addr);  // Channel 1: Priority 5
write_apb(ADDR_CH2_CTRL, desc2_addr);  // Channel 2: Priority 5

// Service order: CH0 → CH1 → CH2 → CH0 → CH1 → CH2 ...

📖 See: PRD.md Section 7 for arbitration details

### 23.5.4	Q: “What’s the descriptor format?”

A: 256-bit descriptor (4 × 64-bit words):

typedef struct packed {
    logic [63:0]  reserved;              // [255:192] Reserved
    logic [7:0]   priority;              // [207:200] Priority
    logic [3:0]   channel_id;            // [199:196] Channel ID
    logic         error;                 // [195] Error flag
    logic         last;                  // [194] Last in chain
    logic         interrupt;             // [193] Generate interrupt
    logic         valid;                 // [192] Valid descriptor
    logic [31:0]  next_descriptor_ptr;   // [191:160] Next descriptor address
    logic [31:0]  length;                // [159:128] Length in BEATS ★
    logic [63:0]  dst_addr;              // [127:64] Destination address
    logic [63:0]  src_addr;              // [63:0] Source address
} descriptor_t;

★ CRITICAL: length is in BEATS, not bytes or chunks!

Example:

Data width = 512 bits = 64 bytes
Length = 16 beats
Total transfer = 16 × 64 = 1024 bytes

📖 See: PRD.md Section 4.2 for complete descriptor specification

### 23.5.5	Q: “How do I run STREAM tests?”

A: Multi-layered test approach (same as RAPIDS):

# 1. FUB (Functional Unit Block) Tests - Individual blocks
pytest projects/components/dma-ip/stream/dv/tests/fub/test_scheduler.py -v
pytest projects/components/dma-ip/stream/dv/tests/fub/test_descriptor_engine.py -v
pytest projects/components/dma-ip/stream/dv/tests/fub/ -v  # All FUB tests

# 2. Macro / Top Tests - Multi-block scenarios
pytest projects/components/dma-ip/stream/dv/tests/macro/ -v
pytest projects/components/dma-ip/stream/dv/tests/top/ -v

# Run with waveforms for debugging
WAVES=1 pytest projects/components/dma-ip/stream/dv/tests/fub/test_scheduler.py -v

Test Organization: - FUB tests: Focus on individual block functionality (dv/tests/fub/) - Macro/top tests: Verify block-to-block interfaces and complete data flows (dv/tests/macro/, dv/tests/top/)

## 23.6	Integration Patterns

### 23.6.1	Pattern 1: Basic STREAM Instantiation

stream_top #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .ADDR_WIDTH(64),
    .SRAM_DEPTH(4096)
) u_stream (
    // Clock and Reset
    .aclk               (system_clk),
    .aresetn            (system_rst_n),

    // APB Configuration Interface
    .s_apb_paddr        (apb_paddr),
    .s_apb_psel         (apb_psel),
    .s_apb_penable      (apb_penable),
    .s_apb_pwrite       (apb_pwrite),
    .s_apb_pwdata       (apb_pwdata),
    .s_apb_pready       (apb_pready),
    .s_apb_prdata       (apb_prdata),
    .s_apb_pslverr      (apb_pslverr),

    // AXI Master - Descriptor Fetch (256-bit)
    .m_axi_desc_araddr  (desc_araddr),
    .m_axi_desc_arlen   (desc_arlen),
    .m_axi_desc_arsize  (desc_arsize),
    .m_axi_desc_arvalid (desc_arvalid),
    .m_axi_desc_arready (desc_arready),
    .m_axi_desc_rdata   (desc_rdata),
    .m_axi_desc_rresp   (desc_rresp),
    .m_axi_desc_rlast   (desc_rlast),
    .m_axi_desc_rvalid  (desc_rvalid),
    .m_axi_desc_rready  (desc_rready),

    // AXI Master - Data Read (parameterizable width)
    .m_axi_rd_araddr    (rd_araddr),
    // ... (full AXI4 AR + R channels)

    // AXI Master - Data Write (parameterizable width)
    .m_axi_wr_awaddr    (wr_awaddr),
    // ... (full AXI4 AW + W + B channels)

    // MonBus Output
    .monbus_pkt_valid   (stream_mon_valid),
    .monbus_pkt_ready   (stream_mon_ready),
    .monbus_pkt_data    (stream_mon_data)
);

### 23.6.2	Pattern 2: Descriptor Creation (Software Model)

// C/C++ software model for descriptor creation
typedef struct {
    uint64_t src_addr;           // Source address (must be aligned!)
    uint64_t dst_addr;           // Destination address (must be aligned!)
    uint32_t length;             // Transfer length in BEATS
    uint32_t next_descriptor_ptr; // Next descriptor address (0 = last)
    uint8_t  control;            // valid | interrupt | last | error | channel_id | priority
} stream_descriptor_t;

// Create descriptor chain
stream_descriptor_t desc[2];

// Descriptor 0
desc[0].src_addr = 0x80000000ULL;  // Aligned to 64B (512-bit data)
desc[0].dst_addr = 0x90000000ULL;
desc[0].length = 64;  // 64 beats × 64 bytes = 4KB transfer
desc[0].next_descriptor_ptr = (uint32_t)&desc[1];  // Chain to next
desc[0].control = 0x01;  // valid = 1

// Descriptor 1 (last)
desc[1].src_addr = 0x80001000ULL;
desc[1].dst_addr = 0x90001000ULL;
desc[1].length = 32;  // 32 beats × 64 bytes = 2KB transfer
desc[1].next_descriptor_ptr = 0;  // Last descriptor
desc[1].control = 0x45;  // valid | last | interrupt

// Kick off transfer
write_apb_reg(CH0_CTRL, (uint32_t)&desc[0]);

### 23.6.3	Pattern 3: MonBus Integration

// Always add downstream FIFO for MonBus
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_stream_mon_fifo (
    .axi_aclk   (aclk),
    .axi_aresetn(aresetn),
    // producer side: wr_ready is "not full", so it is what backpressures MonBus
    .wr_valid   (monbus_pkt_valid),
    .wr_data    (monbus_pkt_data),
    .wr_ready   (monbus_pkt_ready),
    // consumer side
    .rd_valid   (fifo_mon_valid),
    .rd_data    (fifo_mon_data),
    .rd_ready   (consumer_ready)
);

## 23.7	Anti-Patterns to Catch

### 23.7.1	✗ Anti-Pattern 1: Adding Alignment Fixup

✗ WRONG:
"Let me add alignment fixup logic to handle unaligned addresses..."

✓ CORRECTED:
"STREAM intentionally requires aligned addresses for tutorial simplicity.
If you need unaligned transfers, that's a great learning exercise for
extending STREAM, or use RAPIDS which has full alignment support."

### 23.7.2	✗ Anti-Pattern 2: Using Length in Bytes

✗ WRONG:
descriptor.length = 4096;  // Thinking this is 4096 bytes

✓ CORRECTED:
// Length is in BEATS, not bytes!
// For 512-bit data width (64 bytes per beat):
descriptor.length = 4096 / 64;  // = 64 beats for 4096 bytes

### 23.7.3	✗ Anti-Pattern 3: Circular Buffer Descriptors

✗ WRONG:
// Descriptor chain that loops back
desc[9].next_descriptor_ptr = &desc[0];  // Circular!

✓ CORRECTED:
"STREAM does not support circular buffers (no stop condition).
Always terminate chains explicitly:
  desc[last].next_descriptor_ptr = 0;  // Stop
  desc[last].last = 1;  // Explicit last flag

### 23.7.4	✗ Anti-Pattern 4: Assuming Exclusive Channel Access

✗ WRONG:
// Assume channel has exclusive AXI master access
assign m_axi_arvalid = channel_request;  // No arbitration!

✓ CORRECTED:
// All channels share AXI masters - arbitration required
channel_arbiter u_arbiter (
    .ch_requests (channel_requests[7:0]),
    .ch_grant    (channel_grant[7:0]),
    .axi_master_available (axi_master_idle)
);

## 23.8	Debugging Workflow

### 23.8.1	Issue: Descriptor Not Fetched

Check in order: 1. ✓ APB write to CHx_CTRL register successful? 2. ✓ Descriptor address valid? 3. ✓ AXI descriptor master not stalled? 4. ✓ Descriptor memory accessible? 5. ✓ Arbiter granting descriptor fetch?

Debug commands:

pytest projects/components/dma-ip/stream/dv/tests/fub/test_scheduler.py -v -s  # Verbose test
WAVES=1 pytest projects/components/dma-ip/stream/dv/tests/fub/test_scheduler.py -v
gtkwave <logs dir>/*.fst  # Inspect FSM state transitions

### 23.8.2	Issue: Data Transfer Stalls

Check in order: 1. ✓ SRAM buffer depth sufficient? 2. ✓ Source/destination addresses aligned? 3. ✓ AXI read/write engines getting grants? 4. ✓ Downstream AXI ready signals asserted? 5. ✓ Channel not in error state?

Waveform Analysis: - Check SRAM read/write pointers - Verify AXI AR/AW/R/W/B handshakes - Inspect scheduler FSM state - Check arbiter grant signals

### 23.8.3	Issue: Chained Descriptors Not Following

Check in order: 1. ✓ next_descriptor_ptr != 0? 2. ✓ last flag not set prematurely? 3. ✓ Descriptor fetch completing successfully? 4. ✓ Scheduler transitioning to CHAIN_CHECK state? 5. ✓ MonBus showing CHAIN_FETCH event?

## 23.9	Testing Guidance

### 23.9.1	Test Organization

projects/components/dma-ip/stream/dv/tests/
├── fub/                        # Individual block tests
│   ├── test_descriptor_engine.py
│   ├── test_scheduler.py
│   ├── test_sram_controller.py (+ test_sram_controller_alloc.py)
│   ├── test_perf_profiler.py
│   └── test_stream_latency_bridge.py
├── macro/                      # Multi-block scenarios (stream_core, scheduler_group, datapaths)
└── top/                        # Top-level (stream_top_ch8) tests

### 23.9.2	Test Levels

Basic (Quick Smoke Tests): - Single descriptor, single channel - Aligned addresses, simple transfers - ~30 seconds runtime

Medium (Typical Scenarios): - Multiple descriptors, 2-4 channels - Chained descriptors (2-3 deep) - ~90 seconds runtime

Full (Comprehensive Validation): - All 8 channels concurrent - Long descriptor chains (10+ deep) - Error injection, stress testing - ~180 seconds runtime

## 23.10	Key Documentation Links

### 23.10.1	Always Reference These

This Subsystem: - projects/components/dma-ip/stream/PRD.md - Complete specification - projects/components/dma-ip/stream/README.md - Quick start guide - projects/components/dma-ip/stream/known_issues/ - Bug tracking - projects/components/dma-ip/stream/docs/stream_has/, docs/stream_mas/ - HAS/MAS spec trees

Related: - projects/components/dma-ip/rapids/PRD.md - Parent architecture (for comparison) - docs/markdown/rtl-amba/index.md - MonBus integration - /PRD.md - Master requirements - /CLAUDE.md - Repository guide

## 23.11	Quick Commands

# View complete specification
cat projects/components/dma-ip/stream/PRD.md

# Check package definition
cat projects/components/dma-ip/stream/rtl/includes/stream_pkg.sv

# Run tests
pytest projects/components/dma-ip/stream/dv/tests/fub/ -v
pytest projects/components/dma-ip/stream/dv/tests/macro/ -v
pytest projects/components/dma-ip/stream/dv/tests/top/ -v

# Lint
verilator --lint-only projects/components/dma-ip/stream/rtl/fub/scheduler.sv

# Search for modules
find projects/components/dma-ip/stream/rtl/ -name "*.sv" -exec grep -H "^module" {} \;

## 23.12	Remember

- 📖 Tutorial focus - Intentional simplifications for learning

- 🔄 Reuse from RAPIDS - Descriptor engine, SRAM, patterns

- 📏 Length in beats - Not bytes or chunks!

- 🔗 Aligned addresses - No fixup logic

- ⛓️ Chained descriptors - No circular buffers

- 🎯 8 or 16 channels - Shared resources, arbitration required

- 🔍 MonBus standard - Same format as AMBA/RAPIDS

- 🏗️ Testbench reuse - Always create TB classes in projects/components/dma-ip/stream/dv/tbclasses/ (project area, not the framework)

## 23.13	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/dma-ip/stream/docs/

Quick Command: Use the provided shell scripts:

cd projects/components/dma-ip/stream/docs
./generate_has_pdf.sh   # STREAM HAS
./generate_mas_pdf.sh   # STREAM MAS

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the stream_has/ and stream_mas/ index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

Version: 1.0 Last Updated: 2025-10-17 Maintained By: RTL Design Sherpa Project