RTL Design Sherpa

Binary BCH Codec Hardware Architecture Specification 0.1 (draft)

October 20, 2026

Table of Contents

1 Bch Has Index	11

2 Document Information	12

2.1 Binary BCH Codec Hardware Architecture Specification	12

2.2 Revision History	13

2.3 Document Purpose	15

2.4 Intended Audience	16

2.5 Related Documents	17

3 Purpose and Scope	18

3.1 Purpose	18

3.2 Scope	19

3.3 Where this document stands	20

4 Document Conventions	21

4.1 Terminology	21

4.2 Naming	22

4.3 Notation	23

4.4 Decision markers	24

5 Definitions and Acronyms	25

6 Use Cases	28

6.1 Memory controller ECC	29

6.2 CCSDS telecommand link	30

6.3 DVB outer-code link	31

6.4 Standalone codec on a fabric port	32

6.5 Memory-to-memory job engine	33

7 Key Features	34

7.1 Decided (PRD section 3)	34

7.2 Configurable, not yet chosen (chapter 6.2)	35

7.3 Explicitly not features	36

8 System Context	37

8.1 An endpoint codec, not a mid-stream insert	37

8.1.1 Figure 2.1: Where the codec sits	37

8.2 The core inside a consumer	38

8.3 The standalone tops	39

8.4 Dependencies on the rest of the repository	40

9 Block Diagram	41

9.0.1 Figure 3.1: Codec block diagram	41

9.1 The encoder core	42

9.2 The decoder core	43

9.3 The GF layer	44

9.4 The adapters	45

9.5 Hierarchy	46

10 Data Flow	47

10.1 Encoder	47

10.2 Decoder	48

10.3 Block boundaries	49

10.4 Multiple bits per beat	50

11 Solver Options	51

12 Core Interface	53

12.1 Clock and reset	54

12.2 bch_encoder_core	55

12.3 bch_decoder_core	57

12.4 Timing rules	59

12.5 Erasures (ENABLE_ERASURES, PRD D5 TBD)	60

13 AXI-Stream Adapter	61

13.1 Parameters	62

13.2 Behaviour	63

14 AXI4 Job Adapter	64

14.1 Job interface	65

14.2 AXI4 master ports	66

14.3 Parameters	67

14.4 Behaviour	68

15 Register Block	69

16 Clock and Reset	71

16.1 Single clock	71

16.2 Reset	72

16.3 Clock gating	73

16.4 Frequency	74

17 Throughput	75

17.1 Sustained rate candidates (PRD D6)	76

17.2 Measured placeholder	77

18 Latency	78

18.1 Encoder	79

18.2 Decoder	80

18.3 Buffer sizing	81

19 Resources	82

19.1 GF primitives per core	83

19.2 Storage	84

19.3 Order of magnitude	85

19.4 Scaling	86

20 System Requirements	87

20.1 The consumer provides	88

20.2 The consumer gets	89

20.3 Environment	90

21 Parameter Configuration	91

21.1 Code	92

21.2 Interface / build shape	94

21.3 Open decisions in one place	95

22 Candidate Profiles	97

23 Verification Strategy	99

23.1 Golden model	99

23.2 Test matrix	100

23.3 Test structure	102

23.4 Formal	103

23.5 Coverage	104

24 Synthesis and Implementation	105

24.1 Characterisation plan	105

24.2 Implementation notes	106

25 Fields and Polynomials	107

25.1 Why error correction needs a finite field	107

25.2 GF(2): the two-element field	108

25.3 Polynomials over GF(2)	109

25.4 Building GF(2^m)	110

25.5 Inverses and the log table	111

25.6 The freshman’s dream in characteristic 2	112

25.7 A concrete field: GF(2^4)	113

25.8 Why this is hardware-shaped	115

26 From Field to Code	116

26.1 Codewords as polynomials	116

26.2 Generator polynomial and consecutive roots	117

26.3 Minimal polynomials	118

26.4 Constructing BCH(15, 7)	120

26.5 Systematic encoding	121

26.6 Worked example: encoding one block	122

27 Syndromes, the Key Equation, and Correction	124

27.1 Received polynomial and syndromes	124

27.2 The evenness shortcut	125

27.3 The key equation	126

27.4 Chien search and binary correction	127

27.5 Worked example: correcting two errors	128

27.5.1 Computing the odd syndromes	128

27.5.2 Solving the t = 2 Peterson system	129

27.5.3 Chien search	129

27.6 When correction fails	131

27.7 From math to hardware	132

28 The Parameters n, k, t, m	133

28.1 n: block length	134

28.2 m: field dimension	135

28.3 k: message length	136

28.4 t: correctable bit errors	137

28.5 deg g: parity bits	138

28.6 Parameters in the RTL	139

28.7 Table of built profiles	140

28.8 Choosing the parameters: a worked example	141

29 Every Stage: Math to Pseudocode	142

29.1 Encoder (bch_encoder_core)	143

29.2 Syndrome unit (bch_syndrome_unit)	144

29.3 Key-equation solver (bch_key_equation_solver)	145

29.4 Chien search (bch_chien_search)	147

29.5 Verdict and correction (bch_decoder_core)	148

29.6 Trace	149

List of Figures

Figure 2.1: Where the codec sits	37

Figure 3.1: Codec block diagram	41

List of Tables

Table 1: Table 0.1: Related documents	17

Table 2: Table 1.1: Definitions and acronyms	25

Table 3: Table 2.1: Use cases and the profile and adapters each implies	33

Table 4: Table 2.2: Repository dependencies	40

Table 5: Table 3.1: Key-equation solver candidates	51

Table 6: Table 4.1: Clock and reset	54

Table 7: Table 4.2: Encoder core ports	55

Table 8: Table 4.3: Decoder core ports	57

Table 9: Table 4.4: AXI-Stream mapping (target)	61

Table 10: Table 4.5: AXI-Stream adapter parameters	62

Table 11: Table 4.6: Job fields (target)	65

Table 12: Table 4.7: AXI4 job adapter parameters	67

Table 13: Table 4.8: Register block outline (target)	69

Table 14: Table 5.1: Throughput architecture candidates	76

Table 15: Table 5.1a: Measured cycles per block (placeholder)	77

Table 16: Table 5.3: Encoder latency (placeholder)	79

Table 17: Table 5.4: Decoder latency (placeholder)	80

Table 18: Table 5.5: GF primitive counts (analytic placeholder)	83

Table 19: Table 5.6: Storage (placeholder)	84

Table 20: Table 6.1: Code parameters	92

Table 21: Table 6.2: Interface parameters	94

Table 22: Table 6.3: Open PRD decisions and their placeholders	95

Table 23: Table 6.4: Candidate profiles	97

Table 24: Table 6.5: Verification matrix	100

Table 25: Table 6.6: Characterisation steps	105

Table 26: Table 7.1: GF(2) addition and multiplication	108

Table 27: Table 7.2: Log/antilog table for GF(2^4) with p(x) = x^4 + x + 1	113

Table 28: Table 7.3: Addition examples in GF(2^4)	114

Table 29: Table 7.4: Minimal polynomials and cyclotomic cosets for alpha^1 .. alpha^4 in GF(2^4)	118

Table 30: Table 7.5: Coefficients of g(x) = x^8 + x^7 + x^6 + x^4 + 1	120

Table 31: Table 7.6: LFSR contents while encoding d(x) = x^6 + x^3 + x^2 + 1	122

Table 32: Table 7.7: Odd syndromes of the received word	128

Table 33: Table 7.8: Chien search over the 15 bit positions	130

Table 34: Table 7.9: Mapping the decode math to the decoder blocks	132

Table 35: Table 7.10: RTL parameter names for the BCH code variables	139

Table 36: Table 7.11: Built BCH profiles	140

Table 37: Table 7.12: Numeric trace of the section-7.3 example	149

# 1	Bch Has Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Binary BCH Codec Hardware Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Binary BCH Codec Hardware Architecture Specification |
| Version | 0.3 (draft) |
| Date | October 3, 2026 |
| Status | Draft – decisions D1-D6, D8, the D9 bits-per-beat sub-question, and D11 of the PRD are open; this document carries them as parameters |
| Classification | Open Source - MIT License |

## 2.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-10-03 | RTL Design Sherpa | First draft, written before any RTL exists. Captures what the PRD has decided (D7: share the reed-solomon GF layer; D9 direction: valid/ready core with optional AXIS and AXI4 adapters; D10 direction: first consumer expected from a future memory-controller project; D12: scrambler out of scope) and carries the undecided items – field dimension m, correctable bits t, shortening, encoder/decoder mix, erasures, throughput architecture, bits per beat, generator conventions, key-equation solver – as parameters tied to their PRD decision IDs. Numbers are analytic placeholders; none are measured. |
| 0.2 | 2026-10-20 | RTL Design Sherpa | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |
| 0.3 | 2026-10-20 | RTL Design Sherpa | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |

## 2.3	Document Purpose

This Hardware Architecture Specification (HAS) is the high-level view of the Binary BCH codec – what it is, where it sits, what it presents at its boundaries, what it costs and how it is configured. It is the document an integrator reads before instantiating the core in a memory controller, a compute engine or a link, and the document a verification engineer reads to know what must be proven.

It covers:

- the encoder and decoder cores and their valid/ready interfaces

- the optional AXI-Stream and AXI4 adapters for standalone use

- the configuration parameters and the candidate code profiles

- throughput, latency and resource estimates and how they scale

- integration requirements, verification strategy and synthesis plan

Micro-architecture – the GF arithmetic, the syndrome and Chien datapaths, the key-equation solver arrays, FSMs and signal-level timing – belongs to the companion Micro-Architecture Specification (MAS), which does not exist yet. Until it does, this HAS and the PRD are the block-level reference.

## 2.4	Intended Audience

- System architects deciding where error correction sits in a datapath

- Integrators instantiating the core or one of its adapters

- Verification engineers planning the golden-model comparison

- Anyone judging whether a binary BCH code, and which one, fits a channel

## 2.5	Related Documents

Table 0.1: Related documents

| Document | Location | Content |
| --- | --- | --- |
| Product Requirements | ../../PRD.md | the decision table (D1-D12) and the candidate profiles |
| Architecture sketch | ../bch_architecture_sketch.md | block diagram and reuse map (to be written) |
| References | ../../References/README.md | the standards and papers, with source and licence |
| CLAUDE.md | ../../CLAUDE.md | area facts for a session working here |
| Handbook | vault/handbook/design/ | valid/ready contract, reset and clocking, filelists, generated-RTL discipline |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	Purpose

The Binary BCH codec adds bit-level forward error correction to a data path. An encoder appends parity bits to every block of k data bits; a decoder, given the n = k + (n - k) received bits, corrects up to t bit errors anywhere in the block. The arithmetic is over GF(2^m), but the code is binary: every error value is 1, so correction is a simple bit flip and no error-value computation is needed. A binary BCH code is the choice for channels whose errors are independent bit flips or multi-bit failures without a fixed symbol framing, such as raw flash pages and memory arrays.

The component exists so that a Sherpa consumer – a memory controller, a compute engine, a serial link – can drop a proven codec into its own datapath instead of building one. That is why the deliverable is a CORE with plain valid/ready streaming ports at both ends (PRD D9 direction): the consumer keeps its own interfaces and the codec is a block inside them.

## 3.2	Scope

In scope:

- bch_encoder_core and bch_decoder_core: systematic binary BCH(n, k) over GF(2^m), parameterised in m, t and shortening

- the GF(2^m) primitive layer imported from the reed-solomon component (PRD D7), used but not duplicated

- optional boundary adapters for standalone use: AXI-Stream at either end, or an AXI4 read or write engine with a job interface at either end, chosen independently per end

- the configuration registers, status and counters of the standalone tops

- verification against a software golden model, and FPGA characterisation

Out of scope (PRD section 6 and D12):

- soft-decision, list and folded BCH decoding

- concatenation with convolutional or LDPC codes, and interleaving beyond a bit-address permutation at the boundary

- any mid-stream insertion of the codec into a fabric (chapter 2.3 says why)

- a scrambler / randomizer inside the codec (D12: out unless a named standard consumer needs one)

## 3.3	Where this document stands

No RTL exists at revision 0.1. Everything here is derived from the PRD’s decisions and the standards in References/. Where a number appears it is an analytic placeholder – a cell count from the algorithm, a latency bound from the pipeline – and is labelled so. Measured throughput, timing and resources arrive with the first synthesis run (chapter 6.5) and will replace the estimates in chapter 5.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Terminology

- Bit: one binary digit of the code word. The data and code streams are bit-oriented; there is no symbol framing inside the BCH block.

- Symbol: an m-bit field element used internally for GF(2^m) arithmetic. SYMBOL_WIDTH = m. The syndrome and key-equation hardware work in symbols; the Chien search evaluates the locator at n bit positions.

- Block: n bits on the coded side, k bits on the data side. The core delimits blocks with a last flag on the final bit of each.

- Beat: one transfer on a valid/ready port; carries BITS_PER_BEAT bits of one codeword (PRD D9 TBD).

- Profile: one fully-resolved parameter set – m, t, n, k, first root, primitive polynomial – typically named after the standard it matches (chapter 6.3).

- Core: bch_encoder_core or bch_decoder_core, valid/ready at both ends.

- Adapter: the optional AXIS or AXI4 boundary around a core.

## 4.2	Naming

Module and signal names follow the repository conventions (vault/handbook/design/): aclk / aresetn, active-low reset through the ALWAYS_FF_RST / RST_ASSERTED macros, r_ registers and w_ wires internally, valid/ready per the house contract. Parameters are upper case. Generated files carry a “Generated by” banner and live in rtl/generated/.

## 4.3	Notation

- n, k, t, m as above; n - k parity bits. The designed minimum distance is 2t + 1.

- alpha: the primitive element of GF(2^m) defined by the primitive polynomial p(x); b: the first root of the generator polynomial g(x) = lcm{ minimal polynomials of alpha^b … alpha^(b+2t-1) } over GF(2).

- Lambda(x): error-locator polynomial; S_i: syndromes.

- Counts quoted “for the reference profile” mean the CCSDS (63,56) profile, t = 1, m = 6, one bit per beat, unless a table says otherwise.

## 4.4	Decision markers

Where the PRD has decided something, this document states it as fact and cites the decision (for example “PRD D7”). Where the PRD has not, the text says TBD and names the decision; chapter 6.2 collects every TBD in one table.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

Table 1.1: Definitions and acronyms

| Term | Meaning |
| --- | --- |
| BCH(n, k) | Binary Bose-Chaudhuri-Hocquenghem code with n coded bits per block, k of them data |
| GF(2^m) | the Galois field with 2^m elements; internal symbols and all codec arithmetic live in it |
| m | field dimension; each internal symbol is m bits |
| t | the number of bit errors the code corrects per block |
| designed distance | 2t + 1, the guaranteed minimum Hamming distance of the code |
| minimal polynomial | the monic irreducible polynomial over GF(2) having alpha^i as a root in GF(2^m); g(x) is the lcm of the minimal polynomials of the consecutive roots alpha^b … alpha^(b+2t-1) |
| narrow-sense | a BCH code whose first root b = 1; non-narrow-sense has b != 1 (for example CCSDS uses b = 112 in its field representation) |
| primitive BCH | n = 2^m - 1, the full-length cyclic code |
| shortened BCH | n < 2^m - 1, derived from the full-length code by treating the missing leading bits as zero |
| evenness shortcut | in characteristic 2, S_2j = S_j^2, so only the t odd syndromes S_1, S_3, …, S_2t-1 are independent; the syndrome unit computes t GF values, not 2t |
| systematic | the k data bits pass through unchanged and the n - k parity bits follow them |
| syndrome | S_i = R(alpha^(b+i)), the received polynomial evaluated at a generator root; all zero if and only if the block is a codeword |
| key equation | the relation the solver inverts to find Lambda(x) from the odd syndromes |
| riBM | reformulated inversionless Berlekamp-Massey, the default key-equation solver candidate (Sarwate and Shanbhag 2001) |
| Euclid / ME | the modified Euclidean algorithm in its inversionless systolic form (Shao et al. 1985), the alternative solver candidate |
| step-by-step decoding | Massey’s 1965 procedure that corrects syndromes one error position at a time, attractive for small t |
| PGZ | Peterson-Gorenstein-Zierler direct matrix solve over the syndromes, another small-t candidate |
| Chien search | evaluating Lambda at every one of the n bit positions to find the roots, hence the error locations |
| Forney stage | the RS decoder stage that computes error magnitudes; absent in a binary BCH decoder because every error value is 1 |
| bit-level correction | the Chien search walks n bit positions and flips the bit wherever it finds a root; it does not evaluate symbol positions |
| erasure | a bit whose position is known to be unreliable (flagged at the input); not currently enabled (PRD D5 TBD) |
| AXIS | AXI4-Stream |
| PRD | Product Requirements Document (../../PRD.md) |
| MAS | Micro-Architecture Specification (companion, not yet written) |
| FUB | functional unit block |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Use Cases

The codec is an endpoint block (chapter 2.3). Its use cases are the endpoints a Sherpa design has or may grow, each matched to one of the PRD’s candidate profiles. None is yet a committed consumer – that is PRD decision D10 – so this chapter is the menu the first consumer chooses from.

## 6.1	Memory controller ECC

A memory controller encodes on its write path and decodes on its read path. Both cores sit inside the controller’s own datapath behind its scheduler, which already delimits bursts and so provides the block boundary. The BCH code protects a page of bits plus the spare area that holds the parity; t is chosen from the device’s raw bit-error rate and page geometry (the Cai survey and the Nabipour paper size this). This is the expected first consumer (PRD D10 direction) and the use case that argues most strongly for the bare valid/ready core.

## 6.2	CCSDS telecommand link

The CCSDS 231.0-B-4 standard specifies a (63,56) modified BCH code, t = 1, concatenated with a convolutional code inside the Command Link Transmission Unit. It is a small, fully worked example: the generator is given in figure 3-2 of the standard, the field is GF(2^6), and the correction is a single bit anywhere in the 63-bit command frame. It is the natural first RTL profile for bring-up even if no consumer adopts it.

## 6.3	DVB outer-code link

DVB-T2, DVB-S2 and DVB-C2 use a binary BCH code as the outer stage of an LDPC concatenation. The BCH block cleans the LDPC error floor. This use case matters only if a broadcast consumer appears; the relevant standards are linked, not stored, in References/.

## 6.4	Standalone codec on a fabric port

The core wrapped in AXI-Stream at both ends, inserted where a stream already has block boundaries (TLAST) and room for the parity – a framer’s output, a DMA’s stream port. This is the AXIS/AXIS adapter pairing and the simplest way to bring the block up in a system.

## 6.5	Memory-to-memory job engine

The core wrapped in an AXI4 read engine and an AXI4 write engine, driven by jobs from its register block or a descriptor stream: encode this buffer into that one, or decode. A DMA with a transform. This is the AXI4/AXI4 pairing and the natural test vehicle for larger profiles.

Table 2.1: Use cases and the profile and adapters each implies

| Use case | Profile | Adapter pairing | Erasures | Notes |
| --- | --- | --- | --- | --- |
| Memory controller ECC | m = 13-16, t per page model | none (bare cores) | optional (D5 TBD) | the expected first consumer |
| CCSDS telecommand | (63,56), m = 6, t = 1 | none, or AXIS/AXIS | no | natural first RTL profile |
| DVB outer-code link | m, t per standard | none, or AXIS/AXIS | no | only if a broadcast consumer appears |
| Standalone stream codec | any | AXIS/AXIS | optional | bring-up vehicle |
| Memory-to-memory job | any | AXI4/AXI4 | optional | test vehicle for large profiles |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Key Features

## 7.1	Decided (PRD section 3)

- Valid/ready core at both ends (D9 direction). bch_encoder_core and bch_decoder_core present the house streaming contract: valid, ready, data, a last flag on the final bit of a block; the decoder adds a per-block status out. A consumer instantiates the core and nothing else.

- Optional adapters, selectable per end (D9 direction). INTAKE_IF and OUTLET_IF each NONE (default), AXIS or AXI4, independently, so AXIS-in/AXI4-out and the other pairings are one core with different adapters generated. Only the adapters selected exist in the netlist.

- GF primitives imported, not copied (D7). gf_pkg, gf_mul, gf_mul_const and gf_inv are taken from the reed-solomon component’s rtl/gf/ tree. Nothing BCH-specific lives in the RS tree.

- Bits per beat is a new interface parameter (D9 TBD). Unlike the RS codec there is no SYMBOLS_PER_BEAT; a beat carries a slice of one codeword. The exact width and the partial-beat mask are open sub-questions of D9.

- No Forney stage (structural fact of binary BCH). Every error value is 1, so the Chien search drives a direct bit flip. The error-value hardware that the RS decoder needs is absent here.

- Evenness shortcut in the syndrome unit (structural fact of binary BCH). Only the t odd syndromes S_1, S_3, …, S_2t-1 are computed; the even syndromes are obtained by squaring in GF(2^m).

- Generated field constants (R3). The primitive polynomial, generator polynomial and log/antilog tables are emitted by a Python generator into rtl/generated/ from the profile parameters; no field constant is a literal in the datapath.

- Every block reports (R2). The decoder never silently passes a failed block: per-block status carries ok / corrected count / uncorrectable, and running counters feed the register block.

- Golden-model verification (R4). Every profile is checked against galois on random data, error patterns up to t and beyond t (to prove the uncorrectable flag), and erasures where enabled.

## 7.2	Configurable, not yet chosen (chapter 6.2)

- m, the field dimension, and t, the correctable bit count (D1, D2)

- shortening (D3) – full length or a shortened block

- whether both cores are built, or encoder-only / decoder-only (D4)

- erasure decoding present or absent (D5)

- throughput architecture and bits per beat (D6, D9)

- per-profile generator conventions: first root b, primitive polynomial (D8)

- key-equation solver algorithm (D11)

- the first consumer, which fixes all of the above (D10)

## 7.3	Explicitly not features

- Soft-decision or list decoding, interleaving beyond a boundary permutation, concatenation with other codes.

- A scrambler / randomizer inside the codec (D12: out unless a named standard consumer needs one).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	System Context

## 8.1	An endpoint codec, not a mid-stream insert

BCH protects a block, not a wire. The encoder must see k bits before it can emit the first parity bit, and the decoder must see all n bits before it can correct any of them, because the syndromes are a function of the whole block. The code therefore lives where a block boundary already exists and where the extra n - k bits have somewhere to go. The expected consumer – a flash or memory controller – places it at an endpoint: encode on the write path as the page+spare image is assembled, decode (and correct, and report) on the read path before the data leaves. The parity rides the spare area; the correction belongs right after the channel that corrupts.

Dropping the codec into the middle of a fabric fails on three counts: the parity changes the bit count, so the downstream must run faster or the upstream must leave gaps; block alignment must survive every hop, which is why every BCH standard sits behind a sync marker, a fixed frame or a page boundary; and the correction belongs immediately after the channel that corrupts, not several repackings later.

### 8.1.1	Figure 2.1: Where the codec sits

Where the codec sits

Three endpoint shapes: a memory controller with an encoder on its write path and a decoder on its read path; a telecommand link pair with the BCH code inside the CLTU; and the standalone memory-to-memory job engine built from the AXI4 adapters.

## 8.2	The core inside a consumer

With valid/ready at both ends the core is a block in the consumer’s own datapath. The consumer supplies block boundaries (its scheduler, framer or job control already knows where they are), absorbs the n/k rate change, and keeps its own external interfaces. Nothing in the consumer’s protocol handling changes; what changes is that k bits go in and n come out (or n in, k out with a status).

## 8.3	The standalone tops

When there is no consumer datapath to sit inside – a codec on a fabric port, a memory-to-memory transform – the standalone tops wrap the core in adapters (chapters 4.2 and 4.3) and a register block (chapter 4.4), and the existing AXI-Stream and AXI4 monitors observe the boundaries onto the monitor bus.

## 8.4	Dependencies on the rest of the repository

Table 2.2: Repository dependencies

| Dependency | Used for |
| --- | --- |
| projects/components/ecc-ip/reed-solomon/rtl/gf: gf_pkg, gf_mul, gf_mul_const, gf_inv | the shared GF(2^m) primitive layer (PRD D7) |
| rtl/amba/gaxi: gaxi_skid_buffer, gaxi_fifo_sync | stage decoupling; the decoder’s block buffer |
| rtl/common: counter_bin, shifter_beat_pack, shifter_lfsr_galois | positions and iterations; bit packing; any boundary LFSR |
| rtl/amba/axis4: axis4_slave, axis4_master (+ _monlite) | AXIS adapters |
| rtl/amba/axi4: axi4_master_rd, axi4_master_wr | AXI4 adapters’ timing wrappers |
| rtl/amba/ocelus: axis_monitor_lite | boundary observation onto monbus |
| bin/peakrdl_generate.py + utility-ip/converters APB-to-cpuif path | the register block and its APB attachment |
| dma-ip/stream/rtl/fub: axi_read_engine, axi_write_engine | the model for the AXI4 job engines |
| galois, aff3ct (PyPI / standalone, MIT) | DV golden model and cross-check |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Block Diagram

### 9.0.1	Figure 3.1: Codec block diagram

Codec block diagram

The two cores, the shared GF layer imported from the reed-solomon component, and the optional adapters that wrap a core for standalone use.

## 9.1	The encoder core

Three blocks in a straight line, all inside bch_encoder_core. The parity LFSR (r_lfsr) is a deg(g)-stage shift register over GF(2^m) with the generator polynomial’s coefficients on its taps; its next-state logic chains B steps of the generator so a whole beat of data bits enters in one cycle, and the keep count picks how many of those steps apply. The data / parity select passes each data beat straight through (the code is systematic), and once in_last arrives it drains the parity from the top LFSR lanes with in_ready held low, raising last on the final parity beat. The LFSR is all-zero again when the drain ends, so the next block needs no clear. A gaxi_skid_buffer on the output decouples the core from its consumer. There is no separate unpack or pack stage: the core works on B-bit beats end to end.

## 9.2	The decoder core

The received block is written into the block buffer and, in the same cycles, into bch_syndrome_unit, whose t cells each accumulate one odd syndrome by Horner’s rule as the bits arrive. An all-zero syndrome set skips the solver and releases the buffer unchanged. Otherwise the odd syndromes go to bch_key_equation_solver (riBM, the only KES_ALGO implemented), which produces the error-locator polynomial in 2t iterations and flags a degree above t. bch_chien_search then walks the n bit positions, B per cycle; each beat’s flip mask is stored beside the buffered beat, and the corrected beat feeds a second syndrome unit (ENABLE_RECHECK=1, the default) that re-checks the corrected stream.

The verdict is formed only after the walk and the re-check finish: correctable means degree <= t, root count equal to the degree, and a zero re-check. The block sequencer (IDLE, SYND, SOLVE, CHIEN, RECHECK, RELEASE) then releases the buffer, applying the stored flip masks only when the verdict is correctable; an uncorrectable or mis-framed block leaves bit-for-bit unchanged. The status outputs (ok / corrected count / uncorrectable / frame_err) are held through the released block.

Because the code is binary there is no Forney evaluator: the error value is always 1.

## 9.3	The GF layer

The GF layer is imported from projects/components/ecc-ip/reed-solomon/rtl/fub/gf/. gf_pkg holds the field arithmetic (primitive-polynomial checks, alpha powers and the gf_mul_fn multiply function); the BCH-specific generator polynomial and syndrome roots are computed in bch_pkg from the profile parameters. The cores build on it with gf_mul_const (a constant multiply, an XOR network) in the Chien search, gf_syndrome_cell in the syndrome units, and the imported riBM array key_equation_solver_ribm in the key-equation solver; the LFSR and the Chien lane sums use gf_mul_fn directly. The block counts are in chapter 5.3.

## 9.4	The adapters

The adapters come from the top that wraps the core. bch_encoder_axis4 and bch_decoder_axis4 put the house axis4_slave (intake) and axis4_master (outlet) timing wrappers around the core, with TLAST as the block boundary and the per-bit keep on tstrb or the low tuser bits. bch_encoder_axi4 and bch_decoder_axi4 are memory-to-memory job engines: rs_axi4_read_engine feeds the core and rs_axi4_write_engine drains it, sequenced by a job port (cfg_start, source and destination address, block count, cfg_done). bch_encoder_axi4 adds bch_beat_packer when k is not a multiple of B, so the written codeword has its only partial beat last. Chapter 4 specifies each.

## 9.5	Hierarchy

The cores live in rtl/macro/ (bch_encoder_core, bch_decoder_core), the fubs in rtl/fub/ (bch_syndrome_unit, bch_key_equation_solver, bch_chien_search, bch_beat_packer, bch_pkg) and the four integration tops in rtl/top/. The MAS describes each block.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Data Flow

## 10.1	Encoder

One beat in, one beat out, with the output stream (n/k) longer than the input. The exact shape depends on PRD D6 (throughput architecture) and the D9 bits-per-beat decision.

- A beat of BITS_PER_BEAT data bits arrives with in_valid; the core accepts it when the encoder can take it (in_ready).

- Each bit enters the LFSR (updating the parity registers) and is passed straight to the output mux – systematic encoding means the data are not delayed by the code, only by the pipeline.

- On the k-th bit (in_last), the mux switches to the parity registers and emits the n - k parity bits over the following cycles, then raises out_last; the LFSR clears for the next block.

- While parity drains, in_ready is low: the encoder cannot start a new block until the previous block’s parity has left. A consumer that wants full-rate back-to-back blocks provides an input FIFO of at least n - k bits upstream, or accepts the gap.

## 10.2	Decoder

One beat in, one beat out, with a block of latency in between.

- Received bits are written to the block buffer and fed to the syndrome unit. Erasure flags, if enabled, are recorded alongside each bit.

- On in_last the t odd syndromes are complete. If all are zero the block is clean: the solver is skipped and the buffer drains through the corrector unchanged, status = ok.

- Otherwise the key-equation solver runs. If the resulting locator degree exceeds t, or is zero, the block is already known to be uncorrectable.

- Chien search steps through bit positions 0 .. n-1, one (or more, per D6) per cycle, while the buffer reads out the same positions. At each root the corrector flips the bit.

- On the last position the verdict is final: uncorrectable if the root count differs from the degree, or if a re-check over the corrected stream shows a non-zero syndrome. That last check catches blocks with more than t errors whose wrong locator still has the right number of roots.

- The block is released only once its verdict exists. Corrections are applied on the way out, and not at all when the block is uncorrectable, so an uncorrectable block leaves exactly as it arrived. The status ports hold the verdict for every beat of the block and out_last marks its k-th bit.

- The next block’s bits arrive during steps 3-6 and the one after that is being solved while this one is walked; the buffers are sized for that overlap, so the decoder sustains one block per n cycles in the serial case.

## 10.3	Block boundaries

The core does not count bits to find a block: in_last is the boundary, and the core checks it. A block shorter or longer than the configured n (or k on the encoder side) is reported as a framing error in status and the core resynchronises on the next in_last. A shortened code (N_BITS < 2^m - 1) is handled by the core treating the missing leading bits as zero in the syndromes and by offsetting the Chien positions; the consumer sees only the shortened block.

## 10.4	Multiple bits per beat

With BITS_PER_BEAT = B > 1 every stage works a beat per cycle: the encoder LFSR advances by the beat’s bit count, the syndrome cells take B Horner steps, and the Chien cells evaluate B positions from one set of registers, each lane a constant multiply away from the beat’s first position. The solver works on the t odd syndromes and is unaffected. Blocks are still delimited by last; a per-beat keep mask is needed for partial final beats (the open D9 sub-question).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Solver Options

The key-equation solver is the block whose algorithm is not yet chosen (PRD D11). Three candidates are attractive for a binary BCH decoder. They all invert the same key equation from the t odd syndromes to the error-locator polynomial Lambda(x). Because there is no Forney stage, the locator is the only polynomial the Chien search needs.

Table 3.1: Key-equation solver candidates

|  | inversionless BM ("RIBM") | modified Euclidean ("EUCLID") | step-by-step / PGZ ("SMALL_T") |
| --- | --- | --- | --- |
| Algorithm | reformulated inversionless Berlekamp-Massey, Sarwate and Shanbhag 2001; seeded with the t odd syndromes | Sugiyama’s Euclidean algorithm with cross-multiplication in place of division (Shao et al. 1985) | Massey 1965 step-by-step correction, or the Peterson-Gorenstein-Zierler direct matrix solve for small t |
| Array | 3t + 1 processing elements, 2 registers each | four register arrays: R and Q of 2t + 1 symbols, the shifted locator pair of 2t + 3 | control-heavy but small datapath: one or a few GF multipliers and a syndrome update loop |
| GF multipliers | 2 per element: 6t + 2 | 2 per coefficient of each pair: 8t + 8 | O(t^2) logic or a single iterative multiplier |
| Iterations | exactly 2t | data-dependent, t + 1 .. 2t | one per located error position, up to t |
| Critical path per iteration | one multiply and one XOR, no feedback across the array | cross-multiply feeding a degree comparison that drives the swap: longer | depends on implementation; often dominated by syndrome-update MUX tree |
| Control | discrepancy select and gamma update | two nominal degree counters, the normalise / cross / swap rule | position counter, syndrome-test loop, error-counter update |
| Readability | dense; the reformulation is not obvious from the textbook | the textbook algorithm, recognisable step by step | the simplest conceptually for very small t |
| Best fit | general t; the default if no consumer names a small-t profile | a cross-check and a readable alternative | t <= 8 (flash pages often sit here); smallest area at tiny t |

No default is selected at revision 0.1. The choice is D11. Two solvers agreeing as the cross-check, per the RS D11 philosophy, is the recommended verification posture: the golden model runs every candidate on every block and requires identical roots, and the RTL is built with whichever algorithm the first consumer’s rate and area target prefers.

Nothing else in the decoder knows which solver produced Lambda. Syndromes, Chien, the block buffer and the corrector are the same modules in every candidate build.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Core Interface

Both cores present the house valid/ready streaming contract (vault/handbook/design/valid-ready-contracts.md) at each end: valid and data are held stable until ready, ready may depend on valid, valid must not depend on ready. Widths are in bits; B is BITS_PER_BEAT (PRD D9 TBD).

## 12.1	Clock and reset

Table 4.1: Clock and reset

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| aclk | in | 1 | one clock for the whole core |
| aresetn | in | 1 | active-low reset, per the component convention (chapter 4.5) |

## 12.2	bch_encoder_core

Table 4.2: Encoder core ports

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| in_valid | in | 1 | a beat of data bits is offered |
| in_ready | out | 1 | the core takes it this cycle |
| in_data | in | B | B data bits, bit 0 in the low bit TBD / D9 |
| in_keep | in | B | which bits of the beat are present: low-aligned, all ones except possibly the final beat of a block TBD / D9 |
| in_last | in | 1 | this beat carries the k-th data bit of the block |
| out_valid | out | 1 | a beat of coded bits is offered |
| out_ready | in | 1 | the consumer takes it |
| out_data | out | B | B coded bits: the data beats as received, then the n - k parity bits TBD / D9 |
| out_keep | out | B | present-bit mask of the beat: partial at the end of the data and at the end of the parity TBD / D9 |
| out_last | out | 1 | this beat carries the n-th coded bit of the block |
| frame_err | out | 1 | pulse: a block ended with other than k data bits; it was still encoded as given |

## 12.3	bch_decoder_core

Table 4.3: Decoder core ports

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| in_valid / in_ready | in / out | 1 | as the encoder |
| in_data | in | B | B received bits TBD / D9 |
| in_keep | in | B | present-bit mask TBD / D9 |
| in_last | in | 1 | this beat carries the n-th received bit |
| in_erase | in | B | (only with ENABLE_ERASURES) bit is a known erasure TBD / D5 |
| out_valid / out_ready | out / in | 1 | as the encoder |
| out_data | out | B | B corrected data bits; parity lanes are dropped, so the block is k bits TBD / D9 |
| out_keep | out | B | present-bit mask, low-aligned, partial only on the block’s final beat TBD / D9 |
| out_last | out | 1 | this beat carries the k-th data bit of the block |
| out_status_ok | out | 1 | held for every beat of the block: it had no errors |
| out_status_corrected | out | log2(t)+1 | held for every beat of the block: bits corrected (0 .. t) |
| out_status_uncorrectable | out | 1 | held for every beat of the block: correction failed; the data are exactly as received |
| out_status_frame_err | out | 1 | held for every beat of the block: block length was not n; the block passes through uncorrected, its first length - (n - k) bits as data |

## 12.4	Timing rules

- in_last is the block boundary. The core does not infer boundaries by counting; it checks the count against n (or k) and reports a mismatch.

- The encoder holds in_ready low while it drains the parity of the previous block. A consumer wanting no gap buffers that many beats upstream.

- Keep masks are low-aligned. A partial beat anywhere but a block’s last is a framing error on the decoder and a count mismatch on the encoder.

- The decoder accepts the next block while it is still correcting the previous one, up to the depth of its block buffer; in_ready falls only when the buffer is full, which under sustained back-pressure on out_ready it will be.

- Status outputs describe the block being emitted and are valid on every beat of it, first to last; a consumer may sample them with out_last or with the first beat. A block is not released until its verdict is final, so an uncorrectable verdict is never preceded by altered bits.

- Reset clears block state and the parity and syndrome registers; the block buffer is not reset (its contents are indexed by pointers that are).

## 12.5	Erasures (ENABLE_ERASURES, PRD D5 TBD)

When enabled, in_erase marks bits the consumer already knows are bad (a known-bad page region). Each erasure costs one parity bit instead of two: the decoder corrects e errors and f erasures with 2e + f <= 2t. The exact mapping of a per-beat or per-job erasure bitmap to in_erase is part of the D5 / D9 adapter work. When disabled the port does not exist and the erasure-locator logic is not generated.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	AXI-Stream Adapter

Generated at an end when its *_IF parameter is "AXIS". It mirrors the posture of the reed-solomon AXI-Stream adapter (PRD D9 direction) but is not yet designed; the table below is the target contract TBD / D9.

The adapter is the house axis4_slave (intake) or axis4_master (outlet) timing wrapper with the core’s stream mapped onto the AXI-Stream signals.

Table 4.4: AXI-Stream mapping (target)

| AXIS signal | Core signal | Notes |
| --- | --- | --- |
| TVALID / TREADY | valid / ready | one-to-one |
| TDATA[B-1:0] | data | B bits per beat |
| TKEEP[B/8-1:0] or per-bit | keep | byte keep or bit mask; the BCH-specific partial-beat encoding is part of D9 |
| TLAST | last | the block boundary |
| TUSER (intake, decoder) | in_erase[B-1:0] | erasure flags, one per bit lane, only when ENABLE_ERASURES = 1; otherwise the consumer’s TUSER passes through the intake unused |
| TUSER (outlet, decoder) | status | {frame_err, uncorrectable, corrected[..], ok}, valid with TLAST |
| TID, TDEST | – | passed through unchanged from intake to outlet when both ends are AXIS; tied off otherwise |

## 13.1	Parameters

Table 4.5: AXI-Stream adapter parameters

| Parameter | Default | Meaning |
| --- | --- | --- |
| AXIS_SKID_DEPTH | 2 | skid depth in the wrapper (2..8) |
| AXIS_ID_WIDTH, AXIS_DEST_WIDTH | 0 | pass-through sideband widths |
| AXIS_MONITOR | 0 | 1 selects the _monlite wrapper variant, which attaches axis_monitor_lite to the port and emits onto the monitor bus |

## 13.2	Behaviour

The adapter adds no protocol semantics: TLAST is the block boundary the core requires, and the rate change is visible as more beats out than in on the encoder (or fewer on the decoder). A stream that arrives without TLAST at block boundaries cannot be decoded and is reported as a framing error on every block.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	AXI4 Job Adapter

Generated at an end when its *_IF parameter is "AXI4". It mirrors the posture of the reed-solomon AXI4 job adapter (PRD D9 direction) but is not yet designed; the description below is the target contract TBD / D9.

The intake form is a read engine: a job names a source address and a byte count, the engine issues AXI4 read bursts and presents the returned data as the core’s input stream, inserting last every n (decoder) or k (encoder) bits. The outlet form is a write engine: it drains the core’s output stream into AXI4 write bursts at the job’s destination address, one block at a time, and reports each block’s status in its completion. A job controller sequences the two.

## 14.1	Job interface

Jobs arrive from the register block (chapter 4.4) – source, destination, count, kick – or, with JOB_IF = "DESC", from a small AXI-Stream descriptor port carrying the same fields for chained jobs.

Table 4.6: Job fields (target)

| Field | Width | Meaning |
| --- | --- | --- |
| src_addr | AXI_ADDR_WIDTH | read engine start address (intake = AXI4) |
| dst_addr | AXI_ADDR_WIDTH | write engine start address (outlet = AXI4) |
| byte_count | 32 | bytes to read; must be a whole number of blocks |
| kick / done | 1 / 1 | start; completion pulse with the job’s status summary |
| cfg_erasure | N_BITS | decoder only, ENABLE_ERASURES = 1: job-level erasure bitmap, sampled at cfg_start and sliced per beat by the same beat index that builds keep – the shape a known-bad column has TBD / D5 |

## 14.2	AXI4 master ports

Each engine presents one AXI4 master (read-only or write-only) through the house axi4_master_rd / axi4_master_wr timing wrapper: AXI_ID_WIDTH, AXI_ADDR_WIDTH, AXI_DATA_WIDTH (= BITS_PER_BEAT rounded up to a byte multiple), incrementing bursts up to cfg_xfer_beats, no narrow transfers. Responses other than OKAY are counted and reported in the job’s done status; the data are still passed.

## 14.3	Parameters

Table 4.7: AXI4 job adapter parameters

| Parameter | Default | Meaning |
| --- | --- | --- |
| AXI_ADDR_WIDTH | 32 | address width |
| AXI_ID_WIDTH | 4 | ID width; the engines use one ID |
| AXI_MAX_BURST_BEATS | 16 | burst cap (cfg_xfer_beats may lower it at run time) |
| JOB_IF | "REGS" | "REGS" or "DESC" |
| AXI_MONITOR | 0 | 1 attaches the _monlite wrapper variant |

## 14.4	Behaviour

The read engine is modelled on STREAM’s axi_read_engine and the write engine on its axi_write_engine – job valid/address/count in, done strobe out, a burst cap – reduced from STREAM’s multi-channel, SRAM-coupled form to one job and a FIFO. Whether that is a reduction of STREAM’s modules or a fresh instantiation is decided when the adapter is built. The job controller hands the write engine each block’s destination before the block’s first bit reaches it, so a decoder job’s output blocks land contiguously with the parity removed.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Register Block

The BCH component has no standalone tops and therefore no register block at revision 0.1. The bare cores are parameterised; their status is the per-block out_status port. When the AXI4 job adapter is built a register block will be added to the standalone top, attached through the converters’ APB-to-cpuif path exactly as the reed-solomon standalone tops do.

The target register map is the reed-solomon map adapted to bit-level status:

Table 4.8: Register block outline (target)

| Register | Access | Contents |
| --- | --- | --- |
| ID | RO | component ID, version, KES_ALGO, ENABLE_* build flags |
| PROFILE | RO | m, t, n, k, first root b, primitive polynomial index of the build |
| CTRL | RW | enable, soft reset, erasure enable (if built) |
| JOB_SRC, JOB_DST, JOB_LEN | RW | job fields (AXI4 ends only) |
| JOB_KICK | WO | start the job |
| JOB_STATUS | RO | busy, done, last job’s status summary, AXI response errors |
| STAT_BLOCKS | RO | blocks processed |
| STAT_CORRECTED_BLOCKS | RO | blocks with at least one correction |
| STAT_CORRECTED_BITS | RO | total bits corrected |
| STAT_UNCORRECTABLE | RO | blocks flagged uncorrectable |
| STAT_FRAME_ERR | RO | blocks with a length mismatch |
| IRQ_EN, IRQ_STATUS | RW / W1C | interrupt on uncorrectable, on job done, on frame error |

Registers will be accessed by name through the generated regmap in DV and host code, never by offset (vault/handbook/dv/registers-by-name.md). Counters will be 32-bit, saturating, cleared by CTRL.soft_reset.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Clock and Reset

## 16.1	Single clock

The core and its adapters run on one clock, aclk. A consumer whose data path runs on another clock crosses at the consumer’s boundary with the repository’s CDC blocks (rtl/cdc); the core does not contain a crossing. For a standalone top whose AXI4 memory side runs on a different clock than the stream side, the adapter’s timing wrapper is the place to insert the axi4_cdc_{rd,wr} pair, as the bridge does for its CDC slave ports; that is a build option for the adapter, not a property of the core.

## 16.2	Reset

aresetn is active-low and synchronous, per the component target convention. It is applied through the ALWAYS_FF_RST macros with RST_ASSERTED so the polarity follows the repository build (vault/handbook/design/reset-and-clocking.md). Reset clears control state, the parity and syndrome registers, position and iteration counters and the status. The block buffer’s storage is not reset; its pointers are, so stale contents are never read.

## 16.3	Clock gating

None inside the core. The _cg variants of the AXIS and AXI4 wrappers may be selected at the adapters for a standalone top that wants boundary clock gating, following the rest of the repository.

## 16.4	Frequency

The decoder’s clock is set by the key-equation solver’s per-iteration critical path, then by the syndrome and Chien cells (one constant multiply plus an XOR). The exact paths are not yet characterised; chapter 5.3 gives the characterisation plan that turns this into a number per part.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Throughput

All throughput numbers are TBD. The PRD requires (R5) that no number be claimed before it is measured in an out-of-context synthesis fixture per the repository’s practice.

## 17.1	Sustained rate candidates (PRD D6)

The syndrome pass is the throughput cost: n bit positions are accumulated against t GF MACs. Three architectures are candidates.

Table 5.1: Throughput architecture candidates

| Architecture | Bits per cycle | Cost | Best fit |
| --- | --- | --- | --- |
| Bit-serial syndrome pass | 1 | t parallel GF MACs, one per odd syndrome; Chien walks one position per cycle | smallest area; low clock-rate targets |
| Page-parallel unrolled syndrome tree | n / latency | unrolled GF reduction tree over the whole block; large combinational or deeply pipelined | highest throughput; large area |
| Symbol-parallel beats with per-beat serial bits | B per beat | B-bit input slice, each bit serially updates the t MACs; Chien evaluates B positions per cycle | balance; matches a byte/word memory datapath |

The rate contract – the relationship between the consumer’s clock, the block size n, the bits per beat B, and the sustained blocks per second – must be written down once D6 decides. Until then the only honest statement is that the core consumes one beat per cycle when not back-pressured, and the number of bits per beat is itself open (PRD D9).

## 17.2	Measured placeholder

No synthesis or simulation fixture exists. The table below will be replaced by measured cycles per block after the first RTL and the out-of-context fixture run.

Table 5.1a: Measured cycles per block (placeholder)

| Profile | Architecture | Bits/cycle | Cycles per block |
| --- | --- | --- | --- |
| CCSDS (63,56) | TBD | TBD | TBD |
| NAND flash shortened | TBD | TBD | TBD |
| DVB-T2/S2/C2 | TBD | TBD | TBD |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Latency

All latency numbers are TBD. The exact values depend on PRD D6 (throughput architecture), D9 (bits per beat), D11 (solver algorithm), and the pipeline depth chosen during micro-architecture.

## 18.1	Encoder

Table 5.3: Encoder latency (placeholder)

| From | To | Cycles |
| --- | --- | --- |
| first data bit in | first data bit out | pipeline only |
| k-th data bit in | first parity bit out | pipeline + 1 |
| k-th data bit in | last parity bit out | pipeline + (n - k) / B |

## 18.2	Decoder

Table 5.4: Decoder latency (placeholder)

| Phase | Cycles | Notes |
| --- | --- | --- |
| receive the block | n / B | t odd syndromes complete with the last bit |
| key equation | <= 2t (BM) or data-dependent (Euclid / small-t) | skipped when the syndromes are all zero |
| Chien walk + bit flip + re-check | n / B | one or more positions per cycle |
| drain | k / B | the data bits leave while the next block is walked |
| pipeline | TBD | descriptor skids, solver handoff, verdict stage, output FIFO |
| first bit in to first bit out | TBD | dominated by 2n/B plus solver and pipeline |
| first bit in to last bit out | TBD | dominated by 2n/B + k/B plus solver and pipeline |

For a shortened code the n above is the shortened length.

## 18.3	Buffer sizing

The block FIFO must hold the current block while the next arrives during the Chien walk. Its depth is TBD and will be sized once D6 and D9 are fixed. The output FIFO holds a block’s data bits while it waits for its verdict and the next block’s while that one is walked. The release-on-verdict rule costs this second buffer and extra first-bit latency; it is the price of never emitting an altered bit on an uncorrectable block.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Resources

All resource numbers are TBD. The PRD requires (R5) that no number be claimed before it is measured in an out-of-context synthesis fixture per the repository’s practice. Chapter 6.5 is the plan that replaces this table with measurements.

## 19.1	GF primitives per core

The BCH core reuses the reed-solomon GF primitives (gf_mul_const, gf_mul, gf_inv) without duplicating them. The counts below are the expected structural forms once the RTL exists; they are not measured.

Table 5.5: GF primitive counts (analytic placeholder)

| Primitive | Encoder | Decoder | Formula (target) |
| --- | --- | --- | --- |
| gf_mul_const | 2t | 2t + t | generator taps + syndrome constant multiplies |
| gf_mul | 0 | solver-dependent | BM: 6t + 2; Euclid: 8t + 8; small-t: O(t^2) or fewer |
| gf_inv | 0 | 0 or 1 | binary BCH needs no Forney inverse; a small-t solver may use one |
| GF symbol registers | 2t | t + solver array | encoder 2t parity; decoder t odd syndromes + locator |

## 19.2	Storage

Table 5.6: Storage (placeholder)

| Item | Size | Formula |
| --- | --- | --- |
| block FIFO | TBD | (n + latency, rounded up) x B bits, plus erasure bit if enabled |
| output FIFO | TBD | (2k + latency, rounded up) x (B + 1): corrected bit, hit, last |
| re-check syndromes | t cells | a second syndrome_unit over the corrected stream |
| log / antilog tables (gf_inv) | 2 x 2^m x m | LUT ROM at small m, BRAM or Itoh-Tsujii at large m |
| skid buffers | 2 (encoder), 3 (decoder), 2 deep | stage boundaries |

## 19.3	Order of magnitude

A gf_mul_const at m = 8 is at most 8 XOR trees of up to 8 inputs; a gf_mul is an m x m AND array with reduction. The total arithmetic cost scales with t and m, and with the D6 throughput choice. The first out-of-context run will say where the BCH decoder lands on the target parts.

## 19.4	Scaling

- m grows: every multiplier grows from m x m, and the log/antilog tables scale as 2^m x m.

- t grows: syndrome cells, solver array and Chien lanes all grow linearly with t.

- Shortening: n is smaller than 2^m - 1, so the Chien walk and buffers are shorter but the arithmetic width is unchanged.

- Bits per beat B (D9): syndrome and Chien datapaths scale with B; the solver is unaffected.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	System Requirements

What a consumer must provide, and what it gets.

## 20.1	The consumer provides

- Block boundaries. in_last on the k-th data bit (encoder) or n-th coded bit (decoder) of every block. The consumer’s scheduler, framer or job control already knows where blocks end; the core checks and reports, it does not infer.

- Room for the rate change. The encoder emits n bits for every k it takes; the decoder emits k for every n. The consumer’s downstream (encoder) or upstream (decoder) absorbs the difference, or provides a FIFO of n - k bits so the encoder’s parity drain does not stall its input.

- A single clock and the repository reset convention (chapter 4.5).

- Erasure knowledge, if ENABLE_ERASURES: which bits are known bad, per bit or per job, depending on the adapter.

- Configuration at build time. The profile is parameters; nothing about the code is set at run time in the bare core.

## 20.2	The consumer gets

- Systematic encoding: data pass through unchanged and in order, parity follows.

- A per-block verdict on the decoder: ok, corrected (with count), uncorrectable, framing error – valid with the last bit of the block, so the consumer can act on the block it has just received.

- Data as received on an uncorrectable block, never silently altered.

- Determinism: no run-time state survives a block except the counters; the same input block yields the same output block every time.

## 20.3	Environment

- SystemVerilog-2012, synthesisable, no assertions in RTL (properties live in formal/), lint-clean under Verilator per the repository gate.

- Filelists under rtl/filelists/, registered in bin/filelists.toml, one per top, from the first module.

- Python golden model dependencies (galois, aff3ct) are DV-only.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Parameter Configuration

Every parameter of the two cores and the standalone tops. TBD marks a PRD decision not yet taken; the value shown is a placeholder, not a default.

## 21.1	Code

Table 6.1: Code parameters

| Parameter | Type | Range | Default | Meaning | PRD |
| --- | --- | --- | --- | --- | --- |
| FIELD_DIM | int | 3..16 | TBD | m; the field is GF(2^m) | D1 |
| T_BITS | int | 1..(2^m-1)/2 | TBD | correctable bit errors per block | D2 |
| N_BITS | int | 2t+1 .. 2^m-1 | TBD | block length in bits; < 2^m-1 is a shortened code | D3 |
| PRIM_POLY | int | irreducible of degree m | TBD | primitive polynomial; selects the field representation | D8 |
| FIRST_ROOT | int | 0 .. 2^m-2 | TBD | b, first root of g(x); narrow-sense when b = 1 | D8 |
| ENABLE_RECHECK | bit | 0, 1 | 1 | decoder only: second syndrome unit re-checks the corrected stream and gates the verdict | – |

Neither core has an ENABLE_ERASURES or KES_ALGO parameter. Erasures (D5) are not implemented. The solver choice (D11) is bch_key_equation_solver’s own KES_ALGO, which only accepts "RIBM" and is not exposed through the decoder core.

Derived: K_BITS = N_BITS - DEGREE_G, where DEGREE_G is the degree of the generator polynomial computed by gf_pkg from m, t, PRIM_POLY and FIRST_ROOT; elaboration fails if K_BITS is below 1.

Every code parameter is fixed at elaboration (PRD D1-D3). There is no run-time t or n: the arrays are generate loops sized by t, the generator polynomial is computed by gf_pkg when the design elaborates, and a block format change is a rebuild.

## 21.2	Interface / build shape

Table 6.2: Interface parameters

| Parameter | Type | Range | Default | Meaning | PRD |
| --- | --- | --- | --- | --- | --- |
| BITS_PER_BEAT | int | 1 .. k | TBD | number of bits per valid/ready beat; the bus is a slice of one codeword | D9 |
| BUILD_ENCODER | bit |  | 1 TBD | include the encoder core | D4 |
| BUILD_DECODER | bit |  | 1 TBD | include the decoder core | D4 |
| INTAKE_IF | string | "NONE", "AXIS", "AXI4" | "NONE" | intake adapter (standalone tops) | D9 |
| OUTLET_IF | string | "NONE", "AXIS", "AXI4" | "NONE" | outlet adapter | D9 |

Adapter-specific parameters are in tables 4.5 and 4.7.

## 21.3	Open decisions in one place

Table 6.3: Open PRD decisions and their placeholders

| Decision | What fixes it | Effect if left at the placeholder |
| --- | --- | --- |
| D1 field dimension m | the first consumer’s page or frame size | GF cell width, table size, Chien length are unresolved |
| D2 correctable bits t | the consumer’s error model | parity count, solver size, Chien width are unresolved |
| D3 shortening | the consumer’s block length | zero-fill on encode, position offset on decode are unresolved |
| D4 encoder/decoder mix | transmit-only, receive-only, or both | both cores are built; a consumer instantiates what it needs |
| D5 erasures | storage / RAID consumer says yes | no erasure port, smaller decoder |
| D6 throughput architecture | the consumer’s clock and rate | the whole microarchitecture is unresolved |
| D8 generator conventions | the standard | field representation and generator taps are unresolved |
| D9 bits per beat and partial-beat mask | the consumer’s datapath width | core port widths and adapter mapping are unresolved |
| D11 key-equation solver | area / latency target | solver cell counts and cycles are unresolved |
| D10 first consumer | you | none of the above changes until it appears |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Candidate Profiles

A profile is a resolved parameter set. These three come from the standards and papers in References/; the first consumer picks one or defines its own.

Table 6.4: Candidate profiles

| Profile | m | t | n / k | PRIM_POLY | FIRST_ROOT | Source | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CCSDS telecommand | 6 | 1 | 63 / 56 | see figure 3-2 of CCSDS 231.0-B-4 section 3 | 112 in the CCSDS field representation | CCSDS 231.0-B-4 section 3 (stored in References/) | the classic command link: small t, but a fully worked generator in a free standard – the natural first RTL profile |
| NAND flash ECC | 13-16 | 12-72 class | page-plus-spare geometry, shortened by construction | per controller vendor | per controller vendor | Cai et al. 2017 (stored) for the error landscape; Nabipour & Javidan 2023 (stored) for (m,t) selection | the profile the memory-controller consumer is expected to name; n runs to thousands of bits, so D6 and the Chien length dominate |
| DVB-T2 / S2 / C2 | m per standard | per standard | per standard | per standard | per standard | ETSI EN 302 755 / 302 307 / 302 769 (linked in References/) | BCH cleans the LDPC error floor; interesting only if a broadcast consumer ever appears |

The CCSDS row is the natural first RTL profile because the generator is explicit in the stored standard. The NAND flash row is the expected consumer but its exact (m, t, n) come from the controller vendor. The DVB row is linked, not stored, because ETSI restricts redistribution.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Verification Strategy

## 23.1	Golden model

Every configuration is verified against a software binary BCH implementation on the same blocks: galois (MIT) is the first candidate because it exposes galois.BCH with explicit primitive-polynomial, generator and shortening control. aff3ct (MIT) is the cross-check. This is the model-first order from the reed-solomon TASK-002 lesson: the golden model is validated before any RTL is compared against it.

The testbench encodes with the model and checks the RTL encoder bit for bit; it injects errors into model-encoded blocks and checks the RTL decoder’s corrected output and verdict against the model’s decode.

## 23.2	Test matrix

Table 6.5: Verification matrix

| Axis | Values | Why |
| --- | --- | --- |
| profile | each row of table 6.4 the build admits | field constants and shortening differ per profile |
| errors per block | 0, 1, t-1, t, t+1, random | 0 exercises the bypass; t+1 and more must match the reference decoder’s verdict – uncorrectable, or the identical miscorrection where the code itself cannot tell – and an uncorrectable block’s data must be exactly as received |
| reference model | galois.BCH | the hardware algorithms in Python on the same field, validated against AFF3CT |
| erasures (if built) | 0 .. 2t, mixed with errors up to 2e + f = 2t | the erasure budget |
| bits per beat | 1, and the consumer’s B | the B-fold datapaths |
| solver | BM, Euclid, small-t where applicable | identical roots on every corrected block |
| adapters | NONE/NONE first; then AXIS/AXIS and AXI4/AXI4; the mixed pairs follow | the core is proven before any wrapper |
| back-pressure | none, random on both ends, sustained on the output | the valid/ready contract and the block buffer’s full behaviour |
| framing | short block, long block, missing last | the framing-error path |

## 23.3	Test structure

cocotb tests under dv/tests/ with TB classes in dv/tbclasses/, Pattern B (cocotb_test_* prefixes), the three run levels through both the REG_LEVEL grid and TEST_LEVEL depth, per the test-patterns skill. Traffic at the adapters uses the existing AXI-Stream and AXI4 BFMs; nothing drives a valid/ready port by hand. Registers are read by name through the generated regmap.

## 23.4	Formal

No assertions in RTL. Candidate properties for formal/ blocks: the valid/ready contract at both ends of each core; the encoder’s parity equals the golden LFSR for one block (bounded); the evenness shortcut S_2j = S_j^2 on the syndrome unit; the equivalence of the solver candidates on symbolic odd syndromes for small t; the no-Forney structural fact that correction is a pure bit flip.

## 23.5	Coverage

Functional coverage on the matrix above plus: every odd syndrome cell non-zero at least once, every Chien position rooted at least once across the run, the uncorrectable path taken by both the degree check and the root-count check, and the re-check syndrome path when implemented.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Synthesis and Implementation

## 24.1	Characterisation plan

Numbers in chapter 5 are placeholders. They are replaced by measurements the way the repository measures every other block: an out-of-context synthesis fixture per configuration, on the same two parts the monitor and bridge characterisations use.

Table 6.6: Characterisation steps

| Step | Fixture | Output |
| --- | --- | --- |
| 1 | bch_encoder_core and bch_decoder_core, CCSDS (63,56) profile, B = 1 | LUTs, flops, BRAM, WNS; the calibration point |
| 2 | decoder with each KES_ALGO candidate that is built | the solver comparison in table 3.1 measured |
| 3 | B = 8 and B = 64 | the bits-per-beat scaling measured once D9 is decided |
| 4 | NAND flash shortened profile | large-m, large-t scaling |
| 5 | standalone tops with each adapter pair | adapter cost; the monitors’ cost is already known |

The fixture follows projects/components/fabric-gen-ip/bridge/fpga/ (make synth BRIDGE=<fixture> on make/fpga_flow.mk) and reports through the same summary.csv shape, so the numbers land beside the bridge’s and the monitor’s.

## 24.2	Implementation notes

- All arithmetic is combinational XOR/AND networks; there are no carries, so the multiplier trees of rtl/math do not apply and the synthesiser’s DSP inference is irrelevant.

- gf_inv at small m is two 2^m-entry ROMs and infers to LUTs; at large m it becomes BRAM or Itoh-Tsujii.

- The block buffer infers BRAM once the depth passes the LUT-RAM threshold; REGISTERED = 1 on the FIFO adds a cycle and removes the read mux from the corrector’s path if timing needs it.

- The riBM array has no cross-array feedback, so its cells may be pipelined per iteration if the clock demands; the Euclid array’s degree compare cannot be pipelined without the degree-computationless variant.

- No clock gating in the core; boundary gating comes from the _cg wrapper variants at the adapters.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Fields and Polynomials

## 25.1	Why error correction needs a finite field

Every BCH decoder does arithmetic on error patterns. Before we write any hardware, we need a number system where those patterns behave predictably: a field.

A field is a set with two operations (call them addition and multiplication) that satisfy the usual rules — closure, associativity, commutativity, distributivity — plus two extra properties that make algebra work:

- Every element has an additive inverse, so subtraction is always possible.

- Every nonzero element has a multiplicative inverse, so division is always possible.

The integers modulo a prime form a field. The integers modulo a composite do not. Take mod6: the element 2 has no multiplicative inverse because gcd2,6=2, and worse, 2⋅3≡0(mod6) even though neither factor is zero. Those zero divisors break long division, and long division is exactly what polynomial encoding relies on. In a field, dividing one polynomial by another always produces a unique quotient and remainder; that remainder is what the encoder computes as parity. Without a field, the same division could have multiple remainders, and the decoder would not know which error pattern to blame. Error correction needs a finite field because the block length, the syndrome values, and the hardware datapath all have fixed widths.

## 25.2	GF(2): the two-element field

Binary BCH codes live in extension fields of GF2, the field with two elements, 0 and 1. Addition is XOR; multiplication is AND. Both are already hardware primitives.

Table 7.1: GF(2) addition and multiplication

| + | 0 | 1 | * | 0 | 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 |

Because 1+1=0, subtraction is the same as addition. This single fact drives almost every simplification in a binary BCH decoder: there are no sign changes, no borrows, and no two’s-complement corner cases.

## 25.3	Polynomials over GF(2)

A polynomial over GF2 looks like any ordinary polynomial, except its coefficients are only 0 or 1 and all arithmetic on the coefficients is mod 2.

fx=f0+f1x+f2x2+…+fdxd,fi∈{0,1}

The degree is the highest power with a nonzero coefficient. A polynomial is irreducible over GF2 if it cannot be factored into lower-degree polynomials with coefficients in GF2. Irreducible polynomials play the same role for polynomials that primes play for integers: they are the atoms from which larger structures are built.

For example, x2+x+1 is irreducible over GF2. There is no pair of degree-1 polynomials over GF2 whose product gives it, because the only degree-1 polynomials are x, x+1, and their products are x2 and x2+x and x2+1, none of which equal x2+x+1.

Watch out for polynomials that look irreducible but are not. Over GF2,

x+12=x2+2x+1=x2+1

so x2+1 is reducible even though it has no linear factor in the ordinary integer sense. The freshman’s dream from section 7.1 makes perfect squares easy to spot: any polynomial whose exponents are all even is the square of another polynomial.

## 25.4	Building GF(2^m)

We construct GF2m by taking all polynomials of degree less than m over GF2 and doing arithmetic modulo an irreducible polynomial px of degree m. That makes GF2m a finite field with 2m elements.

Pick a primitive polynomial px of degree m. The residue of x modulo px is called α:

α=xmodpx

Then every nonzero element of GF2m can be written as a power of α, and those powers cycle with period 2m−1:

GF2m={1,α,α2,…,α2m−2}

The element 0 is the extra element that makes the count 2m. Multiplication is easy in the power representation: αi⋅αj=αi+jmod2m−1. Addition is easy in the vector representation: XOR the corresponding bit vectors.

A practical implementation keeps both a log table and an antilog table so it can switch between the two in a cycle or two. To see why both forms matter, multiply 1+α by α+α2. In vector form these are 0011 and 0110 (high-order bit first).

1+αα+α2=α+α2+α2+α3=α+α3

The two α2 terms cancel because 1+1=0. The result is vector 1010, which Table 7.2 identifies as α9. In the power form the same product is α4⋅α5=α9. Both give the same answer; the vector form is how the hardware is wired, and the power form is how the human keeps track.

## 25.5	Inverses and the log table

Every nonzero element has a multiplicative inverse. In power form this is immediate:

αi−1=α−imod2m−1=α2m−1−i

So the inverse of α5 is α10, and a quick check in Table 7.2 confirms α5⋅α10=α15=1.

Division is therefore multiplication by the inverse. Solving α7⋅z=α3 for z gives z=α3α7=α3−7mod15=α11. The decoder uses this constantly: the Peterson solve in section 7.3 divides by a determinant, and every such division is one inverse lookup followed by one multiplication.

If you prefer not to store a log table, the inverse can also be computed with the extended Euclidean algorithm on polynomials over GF2. For a small fixed field the table is smaller and faster, so that is what the RTL does.

## 25.6	The freshman’s dream in characteristic 2

In a field of characteristic 2, the binomial theorem collapses because all intermediate binomial coefficients are even:

a+b2=a2+2ab+b2=a2+b2

More generally, for any non-negative integer j:

a+b2j=a2j+b2j

This identity is not a trick; it is the reason binary BCH decoders only need to compute half the syndromes. If Sj is one syndrome, then S2j=Sj2 is free. The decoder hardware computes the odd-indexed syndromes and derives the even ones by squaring. We revisit that shortcut in section 7.3.

## 25.7	A concrete field: GF(2^4)

For the worked examples in this chapter we use m=4 and the primitive polynomial

px=x4+x+1

which is 10011 in binary. The element α=xmodpx is represented by the 4-bit vector 0010. Repeated multiplication by α generates all 15 nonzero elements.

Table 7.2: Log/antilog table for GF(2^4) with p(x) = x^4 + x + 1

| i | alpha^i as polynomial | 4-bit vector |
| --- | --- | --- |
| 0 | 1 | 0001 |
| 1 | alpha | 0010 |
| 2 | alpha^2 | 0100 |
| 3 | alpha^3 | 1000 |
| 4 | 1 + alpha | 0011 |
| 5 | alpha + alpha^2 | 0110 |
| 6 | alpha^2 + alpha^3 | 1100 |
| 7 | 1 + alpha + alpha^3 | 1011 |
| 8 | 1 + alpha^2 | 0101 |
| 9 | alpha + alpha^3 | 1010 |
| 10 | 1 + alpha + alpha^2 | 0111 |
| 11 | alpha + alpha^2 + alpha^3 | 1110 |
| 12 | 1 + alpha + alpha^2 + alpha^3 | 1111 |
| 13 | 1 + alpha^2 + alpha^3 | 1101 |
| 14 | 1 + alpha^3 | 1001 |

Two checks against the table. First, α4 must satisfy pα=0, so α4=α+1, which is vector 0011 — exactly the row for i=4. Second, addition is XOR: α3 is 1000 and α5 is 0110, so their sum is 1110, which the table identifies as α11.

Table 7.3: Addition examples in GF(2^4)

| a | b | a + b (XOR) | result |
| --- | --- | --- | --- |
| alpha^3 | alpha^5 | 1000 xor 0110 = 1110 | alpha^11 |
| alpha^7 | alpha^13 | 1011 xor 1101 = 0110 | alpha^5 |
| alpha^4 | alpha^8 | 0011 xor 0101 = 0110 | alpha^5 |

Multiplication is equally mechanical. To multiply α7 by α13 we add exponents mod 15: 7+13=20≡5(mod15), so the product is α5. To multiply two vector forms without the table, multiply the polynomials and reduce modulo px. Hardware rarely does it that way; it uses the log/antilog ROM or a fixed XOR network.

## 25.8	Why this is hardware-shaped

This chapter is the only place the decoder leaves the binary world and operates on m-bit field elements, but the hardware mapping is direct:

- An m-bit register holds one field element.

- Addition is m XOR gates. There are no carries anywhere in the codec; this is field addition, not binary arithmetic.

- Multiplication by a constant is a fixed XOR network. The constant is hard-wired from the profile parameters.

- General multiplication uses an AND array followed by a reduction tree.

- Inversion is usually a table lookup.

The absence of carries is worth emphasizing. A binary adder chains carries from bit 0 to bit m−1; a field adder XORs every bit independently. That makes the GF datapath fast and easy to pipeline, which is why the encoder and decoder can afford one symbol operation per cycle even for large m.

Multiplication by the constant α — the operation the LFSR does on every clock edge — is a good illustration. In GF24 with px=x4+x+1, multiplying a vector b3,b2,b1,b0 by α shifts it left and, if the bit shifted out is 1, XORs 0011 into the low bits. The whole thing is four 2-input XOR gates and a few muxes. A full variable multiply needs more logic, but it is still pure combinatorial gates; no iteration, no microcode.

The gf_* primitives that implement these operations live in the reed-solomon component tree and are imported into the BCH core, as described in chapter 3.1’s GF layer subsection. Every arithmetic block in the encoder and decoder is built from three of them: gf_mul_const for multiplication by a known constant, gf_mul for general multiplication, and gf_inv for inversion. The gf_pkg package holds the constant functions that evaluate this arithmetic — and the generator polynomial coefficients derived from t, b, and p(x) — from the profile parameters discussed in chapter 6.2.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	From Field to Code

## 26.1	Codewords as polynomials

A binary BCH block of n bits is a polynomial over GF2:

cx=c0+c1x+c2x2+…+cn−1xn−1,ci∈{0,1}

The least significant bit is c0; the most significant bit is cn−1, which is also the first transmitted bit. This is not a general-purpose polynomial — it is just a convenient way to talk about a bit vector and its shifts at the same time.

These codes are cyclic. Multiply cx by x and reduce modulo xn−1:

x⋅cxmodxn−1

Because we are in characteristic 2, xn−1=xn+1. The reduction wraps the coefficient of xn around to the constant term, which is exactly a cyclic rotation of the n bits. If cx is a codeword, so is every cyclic shift of it.

For a concrete 15-bit example, suppose cx has a 1 at x14 and nowhere else. Then x⋅cx=x15, and reducing modulo x15+1 gives 1, which places the 1 at the constant term. The bit moved from polynomial exponent 14 to polynomial exponent 0. That cyclic property is why the encoder is a feedback shift register and why the decoder can treat a block as a single polynomial.

Every valid codeword is a multiple of the generator polynomial gx. If ux is any polynomial of degree less than k, then cx=ux⋅gx is a codeword. There are 2k such polynomials, so there are 2k codewords. The generator polynomial therefore completely defines the code; the data bits only select which multiple of gx we transmit. Systematic encoding chooses ux so that the high coefficients of the product are exactly the data bits, then adds the remainder rx to make the low coefficients the parity.

## 26.2	Generator polynomial and consecutive roots

A BCH code is defined by its generator polynomial gx, a polynomial over GF2 whose roots are 2t consecutive powers of α:

gαb=gαb+1=…=gαb+2t−1=0

The parameter b is the first root. When b=1 the code is narrow-sense, the default in most textbook treatments and in this chapter’s example. Chapter 6.3 notes that CCSDS telecommand uses b=112 in its field representation; the math is the same, only the exponents shift.

The number of consecutive roots determines the guaranteed error-correction capability. A generator with 2t consecutive roots produces a code with designed distance 2t+1, which means any two valid codewords differ in at least 2t+1 bit positions. The proof is the BCH bound; the intuition is that 2t roots give the decoder enough independent equations to solve for t error locations. For our example t=2, so the designed distance is 5: any two valid codewords differ in at least five bit positions. That guarantees correction of up to 2 errors and detection of up to 4 errors. We do not prove the bound here, but we use it to choose t for the target bit-error rate.

## 26.3	Minimal polynomials

Each root αi has a minimal polynomial mix: the monic irreducible polynomial over GF2 that has αi as a root in GF2m. Because the coefficients must lie in GF2, the other roots of mix are forced to be the conjugates of αi under squaring. Here is why. If mix has coefficients in GF2 and miαi=0, then applying the freshman’s dream from section 7.1 gives

0=miαi2=miαi2=miα2i

so α2i is also a root. Repeating the argument collects the whole cyclotomic coset.

αi,α2i,α4i,α8i,…

taken modulo 2m−1 until the sequence repeats. That set is the cyclotomic coset of i.

For i=1 in GF24 the coset is {1,2,4,8}, so

m1x=x+αx+α2x+α4x+α8

Multiplying this out and reducing every coefficient modulo 2 gives x4+x+1. For i=3 the coset is {3,6,12,9}, and the same procedure gives x4+x3+x2+x+1.

The generator is the least common multiple of the minimal polynomials of the consecutive roots:

gx=lcm{mbx,mb+1x,…,mb+2t−1x}

Since distinct minimal polynomials are irreducible, the lcm is just their product. The degree of gx is the number of parity bits:

degg=n−k≤m⋅t

For the BCH(15, 7) example in this chapter, m=4 and t=2, so degg=8=4⋅2.

Table 7.4: Minimal polynomials and cyclotomic cosets for alpha^1 .. alpha^4 in GF(2^4)

| i | cyclotomic coset | minimal polynomial m_i(x) |
| --- | --- | --- |
| 1 | {1, 2, 4, 8} | x^4 + x + 1 |
| 2 | {1, 2, 4, 8} | x^4 + x + 1 |
| 3 | {3, 6, 12, 9} | x^4 + x^3 + x^2 + x + 1 |
| 4 | {1, 2, 4, 8} | x^4 + x + 1 |

The coset for i=1 has four members, so m1x has degree 4. The same coset catches α2 and α4, which is why their minimal polynomials are identical. The coset for i=3 is disjoint, giving a second degree-4 minimal polynomial.

## 26.4	Constructing BCH(15, 7)

We build the narrow-sense code with n=15=24−1, k=7, and t=2. The consecutive roots are α1,α2,α3,α4. From Table 7.4 only two distinct minimal polynomials are needed:

gx=lcmm1x,m3x=m1x⋅m3x

gx=x4+x+1x4+x3+x2+x+1

Multiplying the two degree-4 polynomials gives terms from x8 down to x0. In characteristic 2, any coefficient that appears twice cancels. The x5, x3, x2, and x1 terms each appear an even number of times and disappear; the remaining terms are:

gx=x8+x7+x6+x4+1

The coefficients from x8 down to x0 are:

Table 7.5: Coefficients of g(x) = x^8 + x^7 + x^6 + x^4 + 1

| Power | x^8 | x^7 | x^6 | x^5 | x^4 | x^3 | x^2 | x^1 | x^0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coeff | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |

The degree is 8, exactly n−k=15−7, so this code has 8 parity bits and 7 data bits.

A full-length primitive code has n=2m−1. Most real profiles are shortened: the configured block length is smaller, and the missing leading bits are treated as zero in the syndrome computation and Chien search. Chapter 6.2 covers the parameters and chapter 6.3 lists the candidate profiles. The algebra does not change; only the position offsets and the assumption of leading zeros change.

## 26.5	Systematic encoding

A systematic codeword keeps the original k data bits unchanged and appends n−k parity bits. Write the data polynomial as dx with degree less than k. Shift it by n−k positions:

xn−k⋅dx

Divide by gx and keep the remainder rx, which has degree less than n−k:

rx=xn−k⋅dxmodgx

The codeword is then:

cx=xn−k⋅dx+rx

The high part carries the data; the low part carries the parity. Because cx differs from xn−kdx by a multiple of gx, it is a multiple of gx and therefore a valid codeword. Every root of gx is also a root of cx, which is why the syndrome test in section 7.3 works: evaluating cx at any root of gx gives zero.

The LFSR is polynomial long division in hardware. Each incoming bit shifts the partial dividend left by one power of x. If the bit shifted out of the high position is 1, the register XORs the generator coefficients into the lower positions — that is the same as subtracting gx from the current partial remainder. After the last data bit the register contains exactly the remainder rx.

The hardware in chapter 3.2 is exactly this division. The encoder core’s parity registers form a bit-level LFSR whose feedback taps are the binary coefficients of gx: each arriving bit shifts the partial remainder up one power of x, and when the bit that falls off the end is a 1, the tap positions XOR gx back in. Because the code is binary, that feedback is plain XOR — the binary encoder contains no GF multipliers at all, which is the structural difference from the RS encoder, whose symbol-level LFSR multiplies by the field-element coefficients of its gx.

## 26.6	Worked example: encoding one block

Take the 7-bit message

dx=x6+x3+x2+1

In bit order d6d5d4d3d2d1d0 this is 1001101. Feed the bits into the encoder LFSR one at a time, most significant bit first—this is the same as transmission order. The generator is gx=x8+x7+x6+x4+1, so the feedback taps are at positions 7, 6, 4, and 0. The x8 term is implicit in the shift register: it is the bit that falls off the left end and becomes the feedback decision.

Table 7.6: LFSR contents while encoding d(x) = x^6 + x^3 + x^2 + 1

| Step | Input bit | LFSR p7..p0 after update |
| --- | --- | --- |
| 1 | 1 | 11010001 |
| 2 | 0 | 01110011 |
| 3 | 0 | 11100110 |
| 4 | 1 | 11001100 |
| 5 | 1 | 10011000 |
| 6 | 0 | 11100001 |
| 7 | 1 | 11000010 |

The LFSR table and a manual polynomial long division give the same remainder, which is a useful cross-check. The final register contents 11000010 are the parity polynomial:

rx=x7+x6+x

Appending the parity to the data gives the systematic 15-bit codeword:

cx=x14+x11+x10+x8+x7+x6+x

In bit order c14…c0:

c=100110111000010

The first seven bits are the original message and the last eight bits are the parity. We can sanity-check the result by evaluating cx at a generator root such as α. Every term in cx is a power of α; using Table 7.2 from section 7.1 to convert those powers to vectors and XORing them gives 0. That is not an accident — it is the definition of a valid codeword. We use exactly this 15-bit word in section 7.3, where we corrupt two bits and walk through the decode.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Syndromes, the Key Equation, and Correction

## 27.1	Received polynomial and syndromes

The channel flips some bits. The decoder receives a polynomial Rx that is the transmitted codeword cx plus an error polynomial ex:

Rx=cx+ex

The error polynomial has a 1 at every flipped bit position and 0 everywhere else. If no errors occurred, ex=0 and Rx=cx.

The specification writes the syndromes as Si=Rαb+i for i=0…2t−1, where b is the first root. For narrow-sense codes b=1, and it is often cleaner to index by the exponent itself. In the rest of this chapter we use

Si=Rαi,i=1…2t

This is the same set of values, just renumbered. Because cx is divisible by gx, every root of gx is also a root of cx, so evaluating cx at those roots gives zero. The syndromes therefore depend only on the error polynomial:

Si=Rαi=cαi+eαi=eαi

All syndromes are zero if and only if Rx is a valid codeword. That is the first test the decoder performs, and it is the reason the syndrome unit of chapter 3.2 exists.

The unit evaluates Rx at the t independent roots as the bits stream in. It does not store the whole block and then substitute. Instead it uses Horner’s rule:

Si=…rn−1αi+rn−2αi+rn−3…αi+r0

Bits arrive most-significant first, so rn−1 is the first transmitted bit. Each arriving bit rj triggers one multiply by the constant αi and one XOR into the accumulator. After the last bit the accumulator holds Si. The multiply-by-constant is a fixed XOR network, so the syndrome cell is small and fast.

## 27.2	The evenness shortcut

In characteristic 2, the freshman’s dream from section 7.1 gives:

a+b2=a2+b2

Apply this to a syndrome. If ex has errors at polynomial exponents j1,j2,…, then

Si=αij1+αij2+…

Squaring the syndrome doubles every exponent:

Si2=α2ij1+α2ij2+…=S2i

So the even-indexed syndromes are not independent. For t=2:

S2=S12,S4=S22=S14

Once S1 and S3 are known, S2 and S4 come for free by squaring. The syndrome unit in chapter 3.2 therefore has only t cells, not 2t; each cell accumulates one odd-exponent syndrome. That is the evenness shortcut defined in chapter 1.3.

## 27.3	The key equation

The error-locator polynomial Λx is the monic polynomial whose roots are the error locators αj1,αj2,…, where each jl is a polynomial exponent:

Λx=x+αj1x+αj2⋯

It satisfies the key equation:

Λx⋅Sx≡Ωx(modx2t)

where Sx is the syndrome polynomial and Ωx is the error-evaluator polynomial. In words: Λx is the smallest polynomial that, when multiplied by the syndrome polynomial, leaves only low-degree terms. Its roots tell us where the errors are.

In a reed-solomon decoder, Ωx is also needed because the error magnitude at each location can be any field element. In a binary BCH decoder the magnitude is always 1, so once the Chien search finds the positions the corrector simply flips the bits. The solver only has to deliver Λx.

The key equation can be inverted by several algorithms. Chapter 3.3 compares three candidates for this design:

- riBM (reformulated inversionless Berlekamp-Massey) is the default general-purpose choice.

- Euclidean / ME (modified Euclidean) is a readable alternative that avoids division.

- PGZ / step-by-step is attractive for small t because it solves a small linear system directly.

For the worked example below we use the Peterson-Gorenstein-Zierler direct solve. It is the same algorithm the SMALL_T solver candidate would use, and every step is visible in the field arithmetic. Berlekamp-Massey iteratively builds Λx by inspecting one syndrome discrepancy at a time; the Euclidean algorithm repeatedly cross-multiplies two polynomials until one of them becomes the locator; PGZ writes the syndrome identities as a small linear system and solves it directly with field operations.

## 27.4	Chien search and binary correction

After the solver produces Λx, the Chien search evaluates it at every polynomial exponent:

Λα0,Λα1,…,Λαn−1

If Λαj=0, the coefficient of xj—that is, bit j in polynomial-exponent form—is in error. The Chien search walks the n positions in lock-step with the block buffer read-out, one position per cycle in the serial architecture or several positions per cycle if the throughput profile selects a parallel evaluation tree. Because the code is binary, every error value is 1, so correction is a pure flip: the corrector XORs a 1 into that bit as it leaves the block buffer. There is no Forney stage and no magnitude computation. That is the main mathematical difference between this binary BCH decoder and the reed-solomon decoder, which must also compute how much each symbol was corrupted.

## 27.5	Worked example: correcting two errors

Position convention. In this example, “bit j” means the coefficient of xj in the codeword polynomial. The RTL counts transmission-order positions p=0,1,… starting with the first transmitted bit, so coefficient xj corresponds to position

p=n−1−j

For the two flipped bits, exponents j={10,3} map to RTL positions p={4,11}.

We start with the 15-bit codeword from section 7.2:

c=100110111000010

Flip the bits whose polynomial exponents are 10 and 3—that is, add x10+x3 to cx. The received polynomial is:

Rx=cx+x10+x3

In bit order R14…R0 the received word is:

R=100100111001010

### 27.5.1	Computing the odd syndromes

We need S1=Rα and S3=Rα3. Using Table 7.2 from section 7.1 to replace each power of α by its 4-bit vector and XORing the terms that are present in Rx:

Table 7.7: Odd syndromes of the received word

| Syndrome | Exponent sum | Vector | Value |
| --- | --- | --- | --- |
| S_1 | alpha^12 | 1111 | alpha^12 |
| S_3 | alpha^7 | 1011 | alpha^7 |

The value S1=1111 comes from XORing the vectors for every set bit exponent in Rx: α14=1001, α11=1110, α8=0101, α7=1011, α6=1100, α3=1000, and α1=0010. Chasing the accumulating XOR gives 1111=α12. For S3 we use the same exponents but with tripled values mod 15, which is what the syndrome cell with root α3 accumulates.

The even syndromes follow from the shortcut:

S2=S12=α122=α9=1010

S4=S22=α92=α3=1000

So the syndrome unit only had to compute two 4-bit values, S1 and S3; the other two syndromes came from two field squarings.

### 27.5.2	Solving the t = 2 Peterson system

For two errors the recurrence is:

Sj+2+Λ1Sj+1+Λ2Sj=0,j=1,2

which gives the linear system over GF24:

Λ1S2+Λ2S1=S3

Λ1S3+Λ2S2=S4

Substitute the syndrome values S1=α12, S2=α9, S3=α7, S4=α3:

Λ1α9+Λ2α12=α7

Λ1α7+Λ2α9=α3

The determinant is:

Δ=S22+S1S3=α3+α4=α7

Using Cramer’s rule over the field:

Λ1=S2S3+S1S4Δ=α9⋅α7+α12⋅α3α7=α+1α7=α4α7=α12

Λ2=S2S4+S32Δ=α9⋅α3+α72α7=α12+α14α7=α5α7=α13

So the error-locator polynomial is:

Λx=x2+α12x+α13

Any nonzero scalar multiple of Λx has the same roots, so the Chien search and the corrector do not care about the exact scale. The riBM hardware in section 7.5 produces a scaled reciprocal of this form; the verdict logic only counts roots and checks degree, both of which are scale-invariant.

### 27.5.3	Chien search

Now evaluate Λx at αj for j=0…14.

Table 7.8: Chien search over the 15 bit positions

| j | alpha^j vector | Lambda(alpha^j) | Note |
| --- | --- | --- | --- |
| 0 | 0001 | 0011 |  |
| 1 | 0010 | 0100 |  |
| 2 | 0100 | 0111 |  |
| 3 | 1000 | 0000 | ROOT |
| 4 | 0011 | 1010 |  |
| 5 | 0110 | 1110 |  |
| 6 | 1100 | 1010 |  |
| 7 | 1011 | 0111 |  |
| 8 | 0101 | 1001 |  |
| 9 | 1010 | 1001 |  |
| 10 | 0111 | 0000 | ROOT |
| 11 | 1110 | 0011 |  |
| 12 | 1111 | 1101 |  |
| 13 | 1101 | 0100 |  |
| 14 | 1001 | 1110 |  |

The zeros occur at j=3 and j=10, exactly the exponents we flipped (RTL positions p=11 and p=4). No other position yields zero, so the decoder does not invent extra errors. The corrector XORs those two bits, restoring:

100110111000010

A final syndrome check over the corrected block gives all four syndromes S1=S2=S3=S4=0, confirming the result is a valid 15-bit codeword. The corrected bit vector matches the original codeword from section 7.2 exactly.

## 27.6	When correction fails

Not every received block can be corrected. The decoder declares a block uncorrectable when the math does not line up. Two common cases:

- The locator polynomial Λx has degree v but fewer than v distinct roots in the field. Then the error count implied by the degree does not match the actual root count.

- The locator has degree greater than t. Even if it has roots, the code was only designed to correct t errors, so those roots are not trustworthy.

Chapter 3.2 describes how the verdict logic flags these cases. Some blocks with more than t errors happen to produce a locator with the right number of roots but the wrong positions; the decoder catches those by running a second syndrome check over the corrected stream. An uncorrectable block leaves the buffer unchanged; the status port reports the failure with the final bit.

## 27.7	From math to hardware

Table 7.9: Mapping the decode math to the decoder blocks

| Math step | Hardware block | Where it lives |
| --- | --- | --- |
| Syndrome evaluation S_i = R(alpha^i) | syndrome unit | chapter 3.2 |
| Key-equation solve for Lambda(x) | key-equation solver | chapter 3.3 |
| Root search Lambda(alpha^j) = 0 | Chien search + corrector | chapter 3.2 |
| Block storage and playback | block buffer | chapter 3.2 |
| Final verdict and status | status ports | chapter 3.2 |

The syndrome unit and Chien search are pure field arithmetic pipelines. The solver is the only block whose internal algorithm varies across the candidate profiles; everything downstream sees only the locator polynomial and the syndromes. That separation is what lets the architecture in chapter 3.1 swap riBM, Euclidean, or PGZ without touching the buffer, the corrector, or the interface in chapter 4.1.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	The Parameters n, k, t, m

Every BCH code boils down to a handful of integers. Here’s what each one means, how they fit together, and where they show up in the RTL.

## 28.1	n: block length

n is the number of bits in one coded block. For a primitive BCH code it reaches the full length of the field:

nfull=2m−1

Most real profiles are shortened: the configured n is smaller than 2m−1, and the missing high-order bits are treated as zero during syndrome computation and Chien search. The algebra doesn’t change—only the position offsets and the assumption of leading zeros do. The package computes the full-field size in bch_order and the configured size is the parameter N_BITS (rtl/fub/bch_pkg.sv:50-52, rtl/fub/bch_pkg.sv:37).

## 28.2	m: field dimension

The code lives in GF2m. Each nonzero field element is a power of a primitive element α, and every element is stored as an m-bit vector. The primitive polynomial px of degree m is encoded as PRIM_POLY; bit m is the leading term and is implicit. The default profile uses m=6 and px=x6+x+1 (PRIM_POLY = 0x43) (rtl/fub/bch_pkg.sv:34-35).

## 28.3	k: message length

k is the number of data bits carried inside one n-bit block. It isn’t an independent parameter; it’s fixed by the degree of the generator polynomial:

k=n−degg

The encoder core exposes K_BITS as exactly this difference (rtl/macro/bch_encoder_core.sv:71, rtl/macro/bch_decoder_core.sv:87).

## 28.4	t: correctable bit errors

t is the guaranteed number of bit flips the decoder can fix. The generator has 2t consecutive roots:

gαb=gαb+1=…=gαb+2t−1=0

The first root b is FIRST_ROOT. When b=1 the code is narrow-sense, the usual textbook case. When b=0 the consecutive range includes α0=1, so gx gains an x+1 factor and can detect an extra odd-weight error pattern. The designed minimum distance is

d≥2t+1

This is the BCH bound; the design chooses the roots and therefore t (rtl/fub/bch_pkg.sv:56-59).

## 28.5	deg g: parity bits

The degree of the generator is the number of parity bits appended to each block:

degg=n−k

The degree counts the distinct conjugates of the consecutive roots αb…αb+2t−1. Each cyclotomic coset modulo 2m−1 contributes its size once, so the degree is bounded by

degg≤m⋅t

For many profiles the bound is loose; for the Genesys 2 profile below it’s tight: degg=13⋅8=104. The package computes the degree without expanding the whole polynomial when possible (rtl/fub/bch_pkg.sv:62-83).

## 28.6	Parameters in the RTL

Table 7.10: RTL parameter names for the BCH code variables

| Symbol | RTL parameter | Meaning |
| --- | --- | --- |
| m | FIELD_DIM | Dimension of GF2m |
| px | PRIM_POLY | Primitive polynomial of degree m |
| t | T_BITS | Correctable bit errors |
| n | N_BITS | Configured codeword length |
| b | FIRST_ROOT | First consecutive root of gx |
| k | K_BITS | Derived as n−degg |
| B | BITS_PER_BEAT | Bits per AXI-Stream/AXI4 beat |

BITS_PER_BEAT is bits per beat, not symbols. The AXI-Stream wrapper defaults it to 8 (rtl/top/bch_encoder_axis4.sv:48-69) and the AXI4 wrapper defaults the bus width to 8 (rtl/top/bch_encoder_axi4.sv:39-56).

## 28.7	Table of built profiles

Table 7.11: Built BCH profiles

| Profile | m | PRIM_POLY | t | b | n | k | degg | g (hex) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCSDS TC narrow-sense | 6 | 0x43 | 1 | 1 | 63 | 57 | 6 | 0x43 |
| CCSDS TC modified | 6 | 0x43 | 1 | 0 | 63 | 56 | 7 | 0xC5 |
| Mid-size sanity | 6 | 0x43 | 2 | 1 | 63 | 51 | 12 | 0x1539 |
| Genesys 2 board | 13 | 0x201B | 8 | 1 | 4224 | 4120 | 104 | (see package) |

The Genesys 2 board loopback design runs the shortened profile BCH4224,4120 with t=8 (projects/fpga-systems/Genesys2/ecc-ip/bch/build-loop/rtl/bch_loop_cfg_pkg.sv:26-31). The m=13 primitive polynomial is px=x13+x4+x3+x+1 (0x201B). The degree lands exactly at m⋅t=104, so the parity budget is fully used.

## 28.8	Choosing the parameters: a worked example

Say a 512-byte sector has to survive up to 8 bit flips. The data payload is 512⋅8=4096 bits.

- Pick the field. m=12 gives nfull=4095, which isn’t enough room for both data and parity. m=13 gives nfull=8191, comfortably larger than 4096.

- Pick t. With t=8 the BCH bound promises correction of up to 8 bit errors. The parity budget is at most m⋅t=104 bits.

- Choose the block length. A 4096-bit payload plus a 128-bit spare region gives n=4224.

- Check the degree. For m=13, t=8, b=1, the generator degree is exactly 104, so k=4224−104=4120. The 4096 data bits plus 24 unused bits in the data region are carried as the k message bits.

That’s the Genesys 2 profile in Table 7.11.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Every Stage: Math to Pseudocode

Here’s the bridge between the math in sections 7.1–7.3 and the Verilog in chapter 3.2. For each block you’ll see the equation it implements, pseudocode that uses the same register names as the RTL, and a one-line hardware mapping.

## 29.1	Encoder (bch_encoder_core)

The encoder keeps the first k input bits untouched and appends the parity remainder. Bits arrive most-significant first—the same order they leave the chip.

The math. Shift the message polynomial dx by n−k positions and divide by gx:

rx=xn−k⋅dxmodgx

cx=xn−k⋅dx+rx

The LFSR is polynomial long division in hardware.

The pseudocode. r_lfsr[j] is lane j of the remainder register; lane 0 is the low-order coefficient and lane DEG_G-1 is the coefficient that shifts out first during the parity drain. GEN_BITS[j] is the binary tap gj.

function lfsr_step(r_lfsr, d):
    fb = d ^ r_lfsr[DEG_G - 1]
    for j = 0 .. DEG_G - 1:
        prev = (j == 0) ? 0 : r_lfsr[j - 1]
        r_lfsr[j] = GEN_BITS[j] ? prev ^ fb : prev
    return r_lfsr

// beat unroll: B bits per cycle
w_chain[0] = r_lfsr
for u = 0 .. B - 1:
    w_chain[u + 1] = lfsr_step(w_chain[u], in_data[u])
w_lfsr_next = w_chain[B]
for u = 1 .. B:
    if w_in_count == u:
        w_lfsr_next = w_chain[u]

r_lfsr <= w_lfsr_next           // on every accepted data beat

// parity drain
w_par_beat_len = min(B, remaining parity bits)
for u = 0 .. w_par_beat_len - 1:
    w_parity_bits[u] = r_lfsr[DEG_G - 1 - u]
r_lfsr <= r_lfsr << w_par_beat_len

In the RTL. lfsr_step lives at rtl/macro/bch_encoder_core.sv:143-160; the unrolled chain and partial-beat mux are at :175-186; the parity drain reads the high-order lanes at :231-238; and frame_err fires on a bit-count mismatch at :228. The hardware is a single GF2m bit-LFSR unrolled BITS_PER_BEAT times. Here’s the part that bites: w_chain is purely combinational—only r_lfsr and the counters are clocked, so don’t try to pipeline the chain itself.

## 29.2	Syndrome unit (bch_syndrome_unit)

The syndrome unit evaluates the received polynomial Rx at the t independent odd roots. The even syndromes aren’t computed here; they’re rebuilt later by squaring.

The math. For each odd root exponent j in the consecutive range:

Sj=Rαj=p=0n−1Rp⋅αj⋅p

The even syndromes are not computed here; they are rebuilt later by squaring.

The pseudocode. One gf_syndrome_cell per odd root. ROOT is the fixed field element αroot_exp.

for each odd root index j = 0 .. T - 1:
    root_exp = FIRST_ROOT + ((FIRST_ROOT + 1) % 2) + 2*j
    ROOT     = alpha ^ root_exp
    r_s      = 0

    on every input beat:
        w_chain[0] = r_s
        for u = 0 .. B - 1:
            symbol     = in_data[u] embedded as GF element {0..0, bit}
            w_chain[u+1] = w_chain[u] * ROOT ^ symbol
        w_next = w_chain[B]
        for u = 1 .. B:
            if w_in_count == u:
                w_next = w_chain[u]
        r_s <= w_next

    out_syndromes[j] = r_s

out_no_error = (out_syndromes == 0)

In the RTL. The syndrome fub is rtl/fub/bch_syndrome_unit.sv:50; it instantiates T_BITS copies of gf_syndrome_cell (:128-144) and computes out_no_error from the packed syndrome vector (:155). The shared gf_syndrome_cell itself is one gf_mul_const and a register (projects/components/ecc-ip/reed-solomon/rtl/fub/gf/gf_syndrome_cell.sv:24-32). Don’t go looking for S2 or S4 in this block—they’re derived in the KES.

## 29.3	Key-equation solver (bch_key_equation_solver)

The solver turns the syndromes into the error-locator polynomial Λx. For a binary BCH code only Λx is needed; there’s no Forney stage because every error magnitude is 1.

The math. The key equation is

Λx⋅Sx≡Ωx(modx2t)

where Sx=S1+S2x+…+S2tx2t−1. The reformulated inversionless Berlekamp-Massey (riBM) algorithm solves it in exactly 2t iterations. One iteration uses the discrepancy δ=Δ0 and a swap decision:

Δir+1=γr⋅Δi+1r⊕δr⋅Θir

Θir+1=swap?Δi+1r:Θir

γr+1=swap?δr:γr

kr+1=swap?−kr−1:kr+1

The pseudocode. The syndrome expander reconstructs S1…S2t from the t odd input lanes; note that this expander lives in bch_key_equation_solver, not in the syndrome unit.

// syndrome expander (inside bch_key_equation_solver)
for i = 1 .. 2*t:
    if i is odd:
        w_full[i-1] = in_syndromes[(i-1)/2]
    else:
        w_full[i-1] = w_full[i/2 - 1] ^ 2   // S_{2j} = S_j^2

// riBM array (shared from the reed-solomon library)
NPE = 3*t + 1
for i = 0 .. NPE - 1:
    if i < 2*t:      init[i] = w_full[i]
    elif i == 3*t:   init[i] = 1
    else:            init[i] = 0
    Delta[i] = init[i]
    Theta[i] = init[i]

gamma = 1
k     = 0
for iter = 1 .. 2*t:
    delta = Delta[0]
    swap  = (delta != 0) && (k >= 0)
    for i = 0 .. NPE - 1:
        Delta_next[i] = gamma * Delta[i+1] ^ delta * Theta[i]
        Theta_next[i] = swap ? Delta[i+1] : Theta[i]
    Delta = Delta_next
    Theta = Theta_next
    gamma = swap ? delta : gamma
    k     = swap ? -k-1 : k+1

// BCH wrapper readout
for i = 0 .. T:
    out_lambda[i] = Delta[t + i]
out_lambda_degree = degree of Delta[t .. 3*t]
out_more_than_t   = any Delta[t+i] != 0 for i > T

In the RTL. The BCH wrapper reconstructs the full syndrome sequence at rtl/fub/bch_key_equation_solver.sv:109-134, instantiates the imported key_equation_solver_ribm at :150-167, and trims the locator to Lambda_0 .. Lambda_t at :180-185. The riBM module itself is in the reed-solomon component tree (projects/components/ecc-ip/reed-solomon/rtl/fub/key_equation_solver_ribm.sv:95,137-145 for initialization, :180-188 for the control update, and :196-213 for output readout). A sister description of the same riBM math lives at ../../../../reed-solomon/docs/reed_solomon_has/ch07_understanding_the_math/05_every_stage_math_to_pseudocode.md. The per-cell update is in projects/components/ecc-ip/reed-solomon/rtl/fub/gf/ribm_pe.sv:24-34,62-83. One hard constraint: KES_ALGO isn’t really a choice today—anything other than “RIBM” errors out at elaboration (rtl/fub/bch_key_equation_solver.sv:103-105).

## 29.4	Chien search (bch_chien_search)

The Chien search evaluates Λx at every position and flags the zeros.

The math. Position p (counted from the first transmitted bit) has locator value Xp=αn−1−p. The position is in error when

ΛXp−1=0,Xp−1=α−n−1−p

Within one beat the lane offset u evaluates position p+u:

ΛXp+u−1=i=0tΛi⋅α−in−1−p⋅αiu

The pseudocode. r_c[i] holds the coefficient contribution for the first position of the current beat; LANE_K[u,i] is the constant αiu.

for i = 0 .. T:
    r_c[i] = in_lambda[i] * alpha^{-i*(N-1)}

p = 0
while p < N:
    for u = 0 .. B - 1:
        sum = 0
        for i = 0 .. T:
            sum ^= LANE_K[u,i] * r_c[i]
        w_root[u] = (sum == 0)

    out_flip_en    = w_root & position_valid(p .. p+B-1)
    out_root_count += count(w_root & position_valid)

    p += B
    for i = 0 .. T:
        r_c[i] *= alpha^{i*B}

In the RTL. The coefficient cells are initialized and stepped at rtl/fub/bch_chien_search.sv:135-156; the per-beat lane sum and root detection are at :174-208; the running root count is at :219-231 and out_flip_en at :210-214. Because the code is binary, a root is a pure bit-flip location and no Forney stage is needed (rtl/fub/bch_chien_search.sv:9-11).

## 29.5	Verdict and correction (bch_decoder_core)

The decoder core collects the Chien results, decides whether the block is correctable, and applies the flips.

The math. A block is correctable when

degreeΛ≤tandroot count=degreeΛ

and, when enabled, the corrected stream re-check syndromes are all zero. For a binary BCH code every error value is 1, so correction is

cp=Rp⊕ep

where ep=1 for every found root position p. An uncorrectable block passes through unchanged.

The pseudocode.

// during Chien: capture beat data and flip mask
r_buf[beat]  <= in_data
r_flip[beat] <= w_chien_out_flip_en

// final verdict
w_verdict_correctable = !r_more_than_t
                        && (r_chien_root_count == r_lambda_degree)
                        && (!ENABLE_RECHECK || w_rechk_out_no_error)

// release
r_release_apply = w_verdict_correctable
out_data = r_buf[idx] ^ (r_flip[idx] & {B{r_release_apply}})

// frame-err or uncorrectable: r_release_apply = 0, block emitted unchanged

In the RTL. The block buffer and flip-mask array are at rtl/macro/bch_decoder_core.sv:148-150; the verdict expression is at :359-365; the XOR-based flip application is at :377-380; the capture of r_flip is at :520-524; and the uncorrectable/frame-err pass-through paths are at :535-547,558-569. The trap here is subtle: if the verdict is “uncorrectable,” r_release_apply stays zero and the block is emitted bit-for-bit unchanged—don’t assume the corrector always XORs something.

## 29.6	Trace

The Python script ch07_math_trace.py in this directory reproduces the section-7.3 example with the same field, the same codeword, and the same errors. Section 7.3 now states its convention explicitly: “bit j” means polynomial exponent j, and the RTL reports the complementary transmission-order position p=n−1−j. The script’s output is:

BCH(15,7) t=2 trace (bit 14 first):
  codeword           = 100110111000010
  received           = 100100111001010
  S_1                = alpha^12  (1111)
  S_3                = alpha^7   (1011)
  S_2                = alpha^9   (1010)
  S_4                = alpha^3   (1000)
  raw riBM Lambda    = alpha^1, alpha^13, alpha^14
  monic locator      = alpha^13 + alpha^12*X + X^2
  Chien roots (exp)  = [3, 10]
  RTL positions p    = [4, 11]
  corrected          = 100110111000010

Table 7.12: Numeric trace of the section-7.3 example

| Quantity | Value | Note |
| --- | --- | --- |
| S1 | α12 | matches Table 7.7 |
| S3 | α7 | matches Table 7.7 |
| raw riBM Λ0,Λ1,Λ2 | α1,α13,α14 | RTL array output (BCH wrapper keeps first three lanes) |
| monic locator | X2+α12X+α13 | reciprocal-scaled form used by the section-7.3 Chien table |
| Chien roots (exponents) | {3,10} | the flipped bits from section 7.3 |
| RTL positions p | {4,11} | transmission-order indices; see section 7.3 for the convention |
| corrected word | 100110111000010 | identical to the section-7.2 codeword |

The raw riBM array output is a scalar multiple of the reciprocal of the textbook monic locator. Evaluating that raw polynomial at α−n−1−p is why the RTL position counter reports p=4 and p=11 for the same two errors that section 7.3 labels as exponents 10 and 3.