RTL Design Sherpa

Delta Generator Architecture Specification 1.2

October 20, 2026

Table of Contents

1 Delta Gas Index	8

2 Document Information	9

2.1 Delta Generator Architecture Specification	9

2.2 Revision History	10

2.3 Document Purpose	11

2.4 Intended Audience	12

2.5 Related Documents	13

3 What the Generator Produces	14

3.1 Output artifacts	15

3.2 Architecture in the generated flat module	16

3.3 Topology status	17

4 When to Regenerate	18

4.1 Standard regeneration workflow	19

4.2 Pre-generated vs custom	20

5 Generator Entry Points	21

5.1 Flat crossbar generator	21

5.2 Tree primitives generator	22

5.3 Programmatic API	23

6 CLI and Programmatic API	24

6.1 delta_generator.py CLI	24

6.2 complete_tree_generator.py CLI	25

6.3 Programmatic API	27

6.4 Programmatic API for mesh	28

7 Parameter Semantics	29

7.1 Generation-time parameters	29

7.2 Elaboration-time parameters	30

8 AMBA 5, TSTRB, and Parity Options	31

8.1 TSTRB byte qualifiers	31

8.2 AMBA 5 sidebands (--amba5)	32

8.2.1 Parity rule	32

8.2.2 Who generates and who checks	33

8.2.3 Error flags	33

8.2.4 TWAKEUP steering	34

9 Validation Rules	35

9.1 CLI-level checks	36

9.2 Internal checks	37

9.3 Mesh topology validation	38

9.3.1 Header width budget	38

10 Pipeline Overview	40

10.0.1 Figure 3.1: Delta Generator Pipeline	40

10.1 Stages	41

11 Request Decode	42

11.1 Request matrix	43

11.2 Out-of-range TDEST	44

12 Arbitration and Routing	45

12.1 Packet-atomic round-robin arbitration	45

12.2 Crossbar mux	46

12.3 Backpressure routing	47

13 Boundary Module Instantiation	48

13.1 Flat crossbar	49

13.2 Tree primitives	50

14 Directory Layout	51

15 Module Header and Parameters	52

15.1 Header comment	53

15.2 Port naming convention	54

16 Flat Crossbar	55

16.1 Internal structures	56

16.2 Optional features	57

16.2.1 Performance counters	57

16.2.2 Assertions	57

17 Tree Primitives	58

17.1 delta_split_1to2	59

17.2 delta_merge_2to1	60

17.3 Fan-out / fan-in trees	61

17.4 Filelist convention	62

18 Full Regeneration Rule	63

18.1 Regeneration command	64

18.2 Do not hand-edit generated files	65

19 Pre-Generated Fixtures	66

19.1 Custom variants	67

20 DV Suite	68

20.1 Test focus areas	69

20.2 Running the suite	70

20.3 Adding a new test	71

21 Lint Checks	72

21.1 Common concerns	73

21.2 Synthesis smoke test	74

22 Adding a New Topology	75

22.1 Retired tree scaffolding	76

23 Adding a New Generator Option	77

23.1 Backward compatibility	78

23.2 Example: adding a QoS sideband	79

24 Adding AMBA 5 Sidebands	80

24.1 TWAKEUP design note	81

24.2 Check-signal design note	82

25 Testing an Extension	83

25.1 Minimal test plan	84

25.2 Example: adding a custom variant test	85

25.3 Commit discipline	86

List of Figures

Figure 3.1: Delta Generator Pipeline	40

List of Tables

Table 1: Document Properties	9

Table 2: Revision History	10

Table 3: Generator Output Artifacts	15

Table 4: delta_generator.py CLI Flags	24

Table 5: complete_tree_generator.py CLI Flags	25

Table 6: Generation-Time Parameters	29

Table 7: Elaboration-Time Parameters	30

Table 8: AMBA 5 check signals emitted by --amba5	32

Table 9: Check-signal ownership per port	33

Table 10: CLI-Level Validation Rules	36

Table 11: Internal Validation Rules	37

Table 12: Mesh Topology Validation Rules	38

Table 13: Implemented Head-Flit Header Fields	38

Table 14: Tree Primitives	50

Table 15: Port Naming Convention	54

Table 16: Flat Crossbar Internal Structures	56

Table 17: Sources That Force Regeneration	63

Table 18: DV Test Scenarios	69

Table 19: Lint Concerns and Mitigations	73

# 1	Delta Gas Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 2	Document Information

## 2.1	Delta Generator Architecture Specification

Document Properties

| Property | Value |
| --- | --- |
| Document Title | Delta Generator Architecture Specification |
| Version | 1.2 |
| Date | 2026-10-20 |
| Status | Released |
| Classification | Open Source - MIT License |

## 2.2	Revision History

Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-10-12 | RTL Design Sherpa | Initial GAS release — verified against delta_generator.py, complete_tree_generator.py, and emitted fixtures in rtl/. |
| 1.1 | 2026-10-12 | RTL Design Sherpa | AMBA 5 interface parity per IHI 0051B ch. 5 (issue #102): TPARITY/PW_WIDTH/--parity-width replaced by the odd-parity check signals TDATACHK … TWAKEUPCHK (spec widths, checked at every port, sticky *_parity_error), matching rtl/amba/axis5. First dv/ suite. |
| 1.2 | 2026-10-20 | RTL Design Sherpa | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |

## 2.3	Document Purpose

This Generator Architecture Specification (GAS) documents the Delta AXI-Stream crossbar code generator. It is written for the engineer who must re-run the generator, add a new topology or option, or understand why a generated module looks the way it does.

For the generated hardware, see the Delta HAS and MAS. For product requirements, see PRD.md.

## 2.4	Intended Audience

- RTL engineers extending the generator

- Verification engineers regenerating fixtures

- Integrators choosing pre-generated vs custom variants

## 2.5	Related Documents

- Delta HAS - Hardware Architecture Specification

- Delta MAS - Micro-Architecture Specification

- PRD.md - Product requirements

- CLAUDE.md - AI development guide

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 3	What the Generator Produces

The Delta generator (bin/delta_generator.py) emits a parameterized AXI-Stream flat crossbar module. A companion script (bin/complete_tree_generator.py) emits tree-structured primitives (1:2 splitters, 2:1 mergers, fan-out trees, and fan-in trees). All output is single-file SystemVerilog with no post-processing required.

## 3.1	Output artifacts

Generator Output Artifacts

| Artifact | Description |
| --- | --- |
| delta_axis_flat_<M>x<N>.sv | Core flat crossbar module. |
| delta_split_1to2.sv | 1:2 splitter primitive (when --nodes is used). |
| delta_merge_2to1.sv | 2:1 merger primitive from complete_tree_generator.py. |
| delta_fanout_1to<N>.sv | Fan-out tree scaffold (partial implementation). |
| delta_fanin_<N>to1.sv | Fan-in tree scaffold (partial implementation). |
| delta_mesh_cell.sv | Mesh cell module (emitted with --topology mesh). |
| delta_mesh_fabric_<X>x<Y>x<Z>.sv | Mesh fabric top that instantiates the cell grid. |

## 3.2	Architecture in the generated flat module

The generated delta_axis_flat_<M>x<N> module contains:

- Request decode — TDEST is decoded directly into a slave request matrix.

- Per-slave arbitration — round-robin with packet atomicity (grant held until TLAST).

- Crossbar mux — routes TDATA, TVALID, TLAST, TDEST, TID, TUSER, optional TSTRB, and with --amba5 the payload check bits.

- Optional AMBA 5 logic — TWAKEUP steering, plus the IHI 0051B ch. 5 check signals: payload checks on ingress, control checks generated or checked, and sticky per-port error flags. Built from rtl/common/dataint_parity.

- Backpressure logic — s_axis_tready driven from m_axis_tready of the granted slave.

- Optional performance counters — packet counts per master and per slave.

- Optional assertions — one-hot grant and TDEST-range checks under FORMAL.

## 3.3	Topology status

--topology has four conceptual values:

- flat (default, implemented) — emits delta_axis_flat_<M>x<N>.sv; this is the production artifact.

- tree (retired 2026-10-12) — scaffolding only, in complete_tree_generator.py; not in regression.

- mesh (implemented) — emits delta_mesh_cell.sv plus a fabric top. The hierarchy (cell / tile / confluence / fabric) is placement and documentation, not wrapper RTL.

IMPLEMENTED Mesh mode RTL is generated by delta_generator.py --topology mesh. The design is per the owner-approved delta mesh design spec dated 2026-10-13 (docs/superpowers/specs/2026-10-13-delta-mesh-design.md). The flat crossbar remains the implemented default.

Still deferred: CXS migration, MonBus observation aggregation in the tile IP, 1 cycle/hop lookahead (issue #120), and SRAM FIFO_IMPL (issue #118).

The flat crossbar and mesh fabric are both generated from the same CLI; the tree scaffolding is retired and remains only for reference.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 4	When to Regenerate

Regenerate the Delta fixtures when any of the following change:

- bin/delta_generator.py

- bin/complete_tree_generator.py

- bin/rtl_generators/verilog/module.py (the framework base class)

- The area regression flow or lint rules

## 4.1	Standard regeneration workflow

cd projects/components/fabric-gen-ip/delta/bin
python delta_generator.py --masters 2 --slaves 4 --output-dir ../rtl
python complete_tree_generator.py --type merger --output ../rtl

After regeneration, run the DV suite:

cd ../dv/tests
make clean-all && make run-all-gate

## 4.2	Pre-generated vs custom

Always check whether a pre-generated fixture exists before generating a custom one. The pre-generated fixtures are the regression baseline; custom variants are for one-off sizes.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 5	Generator Entry Points

## 5.1	Flat crossbar generator

python3 delta_generator.py --masters 2 --slaves 4 --data-width 64 --output-dir ../rtl

CLI flags are documented in the configuration chapter.

## 5.2	Tree primitives generator

python3 complete_tree_generator.py --type merger --output ../rtl
python3 complete_tree_generator.py --type fanout --size 4 --output ../rtl
python3 complete_tree_generator.py --type fanin --size 4 --output ../rtl

## 5.3	Programmatic API

from delta_generator import DeltaConfig, DeltaFlatCrossbar

cfg = DeltaConfig(
    topology="flat",
    num_masters=2,
    num_slaves=4,
    data_width=64,
    dest_width=2,
    id_width=1,
    user_width=1,
    strb_width=0,
    amba5=False,
    registered_outputs=True,
    arbiter_type="round_robin",
    add_performance_counters=True,
)
gen = DeltaFlatCrossbar(cfg)
gen.verilog("../rtl")

The verilog() method writes delta_axis_flat_<M>x<N>.sv into the requested directory using the Module framework’s write() helper.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 6	CLI and Programmatic API

## 6.1	delta_generator.py CLI

python3 delta_generator.py --masters 2 --slaves 4 --data-width 64 --output-dir ../rtl

delta_generator.py CLI Flags

| Flag | Default | Description |
| --- | --- | --- |
| --topology | flat | flat (implemented), tree (retired scaffolding), both (legacy alias; retired), or mesh (implemented). |
| --masters | required | Number of master interfaces. |
| --slaves | required | Number of slave interfaces. |
| --data-width | 64 | TDATA width in bits. |
| --user-width | 1 | TUSER width. |
| --strb-width | 0 | TSTRB width; 0 omits TSTRB ports. |
| --amba5 | False | Emit AMBA 5 TWAKEUP, the IHI 0051B ch. 5 odd-parity check signals and the per-port *_parity_error flags. Widths are fixed by the spec; requires a whole-byte --data-width. |
| --output-dir | ../rtl | Destination directory. |
| --nodes | False | Also emit the delta_split_1to2 primitive. |
| --no-counters | False | Disable the optional performance counters. |

## 6.2	complete_tree_generator.py CLI

complete_tree_generator.py CLI Flags

| Flag | Description |
| --- | --- |
| --type | merger, fanout, or fanin. |
| --size | Number of inputs/outputs for fan-out/fan-in. |
| --output | Output directory (default ../rtl). |

IMPLEMENTED The mesh CLI below is implemented and exercised in regression. It is per the 2026-10-13 mesh design spec.

python3 delta_generator.py --topology mesh \
  --cells-x 4 --cells-y 4 --cells-z 1 \
  [--torus-x] [--torus-y] [--torus-z] \
  [--torus x,y,z] \
  [--source-route] [--speculation] \
  [--fifo-impl reg|sram] \
  [--ifc axis|cxs] \
  [--data-width N] [--user-width N] [--amba5] [--no-counters] \
  --output-dir ../rtl

Mesh mode emits one cell module, delta_mesh_cell.sv, and a fabric top, delta_mesh_fabric_<X>x<Y>x<Z>.sv, that instantiates the cells-x × cells-y × cells-z grid. The hierarchy names (cell, tile, confluence, fabric) are for placement and documentation; the generator does not emit wrapper RTL for each level. The --fifo-impl knob accepts only reg (default) in mesh mode — sram is rejected at validation (issue #118: the BRAM registered-read path hangs the drop FSM). --amba5 is likewise rejected in mesh mode (flat mode only in v1).

--ifc selects the interface plane (owner decision 2026-10-16, mirroring the rapids network-plane pattern): axis (default) emits the raw AXI4-Stream ports; cxs additionally emits delta_mesh_cell_cxs.sv (a wrapper around the unchanged AXIS core), four plane modules, and delta_mesh_fabric_<X>x<Y>x<Z>_cxs.sv with native AMBA 5 CXS flit ports (Issue D framing, credit grant/return, activation handshake) so a CXS rapids attaches without adapters. CXS mode requires --data-width 256/512/1024/2048 — AMBA CXS multi-packet flits (CXSMAXPKTPERFLIT >= 2, required for TLAST framing) restrict CXSDATAWIDTH to those values (cxs_pkg::config_error rule 6); the generator rejects other widths. The §4 header content rides the flit sideband in both modes. Mesh credit sidebands remain the inter-cell flow control; CXS link credit is the plane’s port-level mechanism. axis remains the default and the checked-in delta_mesh.f artifacts are unchanged; the CXS artifacts ship under delta_mesh_cxs.f. Scope: CXS mode is validated for directed traffic (the dv suite scoreboards self/1-hop/2-hop multi-beat packets end to end); concurrent-load CXS shows a duplication/loss corner tracked as issue #122 — AXIS mode carries concurrent mesh behavior until #122 lands.

--masters and --slaves are not used in mesh mode; the fabric dimensions define the topology instead.

## 6.3	Programmatic API

The preferred programmatic entry point is the DeltaConfig dataclass plus DeltaFlatCrossbar:

from delta_generator import DeltaConfig, DeltaFlatCrossbar

cfg = DeltaConfig(
    topology="flat",
    num_masters=2,
    num_slaves=4,
    data_width=64,
    dest_width=2,
    id_width=1,
    user_width=1,
    strb_width=8,
    amba5=True,
    registered_outputs=True,
    arbiter_type="round_robin",
    add_performance_counters=True,
)

gen = DeltaFlatCrossbar(cfg)
gen.verilog("../rtl")

## 6.4	Programmatic API for mesh

from delta_mesh import DeltaMeshConfig, generate_mesh, validate_mesh

cfg = DeltaMeshConfig(
    cells_x=4,
    cells_y=4,
    cells_z=1,
    torus_x=False,
    torus_y=False,
    torus_z=False,
    source_route=False,
    speculation=False,
    fifo_impl="reg",
    data_width=64,
    user_width=44,   # must be >= 6*COORD_W + 32; here COORD_W = 2
    amba5=False,
    add_counters=True,
)
validate_mesh(cfg)
generate_mesh(cfg, "../rtl")

dest_width and id_width are normally calculated from num_slaves and num_masters but may be overridden if the caller supplies them explicitly.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 7	Parameter Semantics

## 7.1	Generation-time parameters

These are fixed in the emitted module name and structure.

Generation-Time Parameters

| Parameter | Effect in emitted RTL |
| --- | --- |
| num_masters | Number of s_axis_* input interfaces; sets NUM_MASTERS and TID width. |
| num_slaves | Number of m_axis_* output interfaces; sets NUM_SLAVES and TDEST width. |
| data_width | TDATA width; sets DATA_WIDTH. |
| user_width | TUSER width; sets USER_WIDTH. |
| strb_width | If > 0, adds TSTRB ports and sets STRB_WIDTH; otherwise omits them. |
| amba5 | Adds TWAKEUP, the IHI 0051B ch. 5 check signals and the s_/m_parity_error flags; adds the derived check-width parameters. |
| registered_outputs | Reserved for future output-register insertion; currently affects header comments. |
| arbiter_type | Reserved for future arbiter selection; currently only round-robin is emitted. |
| add_performance_counters | Adds 32-bit packet counters per master and slave. |

## 7.2	Elaboration-time parameters

The generated flat crossbar exposes the following parameters:

module delta_axis_flat_2x4 #(
    parameter int DATA_WIDTH = 64,
    parameter int DEST_WIDTH = 2,
    parameter int ID_WIDTH   = 1,
    parameter int USER_WIDTH = 1,
    parameter int NUM_MASTERS = 2,
    parameter int NUM_SLAVES  = 4
) (

When strb_width > 0, STRB_WIDTH is added. When amba5=True, TDATACHK_W, TIDCHK_W, TDESTCHK_W and TUSERCHK_W are added, plus TSTRBCHK_W with TSTRB. Each defaults to an expression of its payload width, so leave them at their defaults.

Elaboration-Time Parameters

| Parameter | Typical use |
| --- | --- |
| DATA_WIDTH | Match the datapath width of the surrounding AXI-Stream network. |
| DEST_WIDTH | Wide enough to encode NUM_SLAVES - 1. |
| ID_WIDTH | Wide enough to encode NUM_MASTERS - 1. |
| USER_WIDTH | Carry sideband metadata alongside each beat. |

The master and slave counts are not elaboration-time parameters. A 2×4 module cannot be instantiated as 2×8. Generate a new module instead.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 8	AMBA 5, TSTRB, and Parity Options

## 8.1	TSTRB byte qualifiers

By default Delta omits TSTRB to match the common AXI-Stream use case where every byte is valid. To emit byte-qualifier ports:

python3 delta_generator.py --masters 2 --slaves 4 --strb-width 8

The generator adds s_axis_tstrb and m_axis_tstrb arrays and muxes them alongside TDATA.

## 8.2	AMBA 5 sidebands (--amba5)

When --amba5 is set, the generator adds TWAKEUP and the full AXI5-Stream interface-protection set of IHI 0051B chapter 5, Check_Type = Odd_Parity_Byte_All. It also adds a sticky error flag on every port. The signal names and widths match rtl/amba/axis5/*, so a generated crossbar connects directly to those modules.

--amba5 needs a whole-byte --data-width, because TDATACHK has one bit per TDATA byte. The CLI rejects anything else.

AMBA 5 check signals emitted by --amba5

| Check signal | Width (Table 5-1) | Width parameter | Covers |
| --- | --- | --- | --- |
| TDATACHK | DATA_WIDTH/8 | TDATACHK_W | TDATA |
| TSTRBCHK | ceil(STRB_WIDTH/8) (only with --strb-width) | TSTRBCHK_W | TSTRB |
| TLASTCHK | 1 | – | TLAST |
| TIDCHK | ceil(ID_WIDTH/8) | TIDCHK_W | TID |
| TDESTCHK | ceil(DEST_WIDTH/8) | TDESTCHK_W | TDEST |
| TUSERCHK | ceil(USER_WIDTH/8) | TUSERCHK_W | TUSER |
| TVALIDCHK | 1 | – | TVALID |
| TREADYCHK | 1 | – | TREADY |
| TWAKEUPCHK | 1 | – | TWAKEUP |

Each width parameter is derived in the generated module header, for example parameter int TDATACHK_W = DATA_WIDTH / 8. If you override a payload width at instantiation, the check ports follow it. There is no separate parity-width option, because the spec fixes the widths. The old --parity-width flag has been removed.

### 8.2.1	Parity rule

Odd parity: a check bit and the bits it covers always hold an odd number of ones. Check bit [n] covers bits [8n+7:8n]. When a signal is not a whole number of bytes, the top check bit covers the short remainder (IHI 0051B section 5.3). A one-bit check is the inversion of its signal. The crossbar computes and checks every bit with one rtl/common/dataint_parity instance (CHUNKS=1, parity_type=0). The generated file therefore needs dataint_parity in its compile closure: -f $REPO_ROOT/rtl/common/filelists/dataint_parity.f, as rtl_test/filelists/delta_axis_flat_2x4_strb_amba5.f does.

### 8.2.2	Who generates and who checks

Check-signal ownership per port

| Port side | Signal | Crossbar action |
| --- | --- | --- |
| master (s_axis_*), crossbar receives | payload *CHK | Checked against the payload while TVALID is high. Then carried to the granted slave port unchanged, by the same grant mux as TDATA. |
| master | TVALIDCHK, TWAKEUPCHK | Checked every cycle out of reset (check enable is ARESETn). |
| master | TREADYCHK | Generated from the TREADY the crossbar drives. |
| slave (m_axis_*), crossbar transmits | payload *CHK | Carried from the granted master. When no master is granted, they idle at all ones, the odd parity of the all-zero idle payload. |
| slave | TVALIDCHK, TWAKEUPCHK | Generated from the TVALID / TWAKEUP the crossbar drives. |
| slave | TREADYCHK | Checked every cycle out of reset. |

The payload check bits are carried, not regenerated. A check bit that arrives bad at a master port therefore reaches the slave still bad, and both the crossbar and the downstream receiver can see it.

### 8.2.3	Error flags

s_parity_error[NUM_MASTERS] and m_parity_error[NUM_SLAVES] are sticky and are cleared only by aresetn. A master port’s flag sets on a bad payload *CHK beat, a bad TVALIDCHK, or a bad TWAKEUPCHK. A slave port’s flag sets on a bad TREADYCHK. The crossbar does not drop or alter a beat that fails its check. The flag reports the failure and the beat is still delivered.

### 8.2.4	TWAKEUP steering

TWAKEUP is treated as a hint that may arrive before TVALID. The generator decodes it directly from TDEST without waiting for a grant:

m_axis_twakeup[s] = OR over all m of (s_axis_twakeup[m] && s_axis_tdest[m] == s);

m_axis_twakeupchk[s] is generated from this OR.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 9	Validation Rules

The generator validates CLI arguments through argparse and additional checks inside main().

## 9.1	CLI-level checks

CLI-Level Validation Rules

| Rule | Behavior |
| --- | --- |
| --topology must be flat, tree, both, or mesh. | Enforced by choices. |
| --masters and --slaves are required integers. | Enforced by required=True. |
| --strb-width must be >= 0. | Custom check; negative values raise an error. |
| --amba5 requires --data-width % 8 == 0. | Custom check; TDATACHK is one bit per TDATA byte (IHI 0051B Table 5-1). |
| --cells-x, --cells-y, --cells-z must be >= 1 | Mesh-only; enforced in validate_mesh. |
| A torus wrap on a dimension requires >= 2 cells in that dimension | Mesh-only; enforced in validate_mesh. |
| --fifo-impl must be reg or sram | Mesh-only; enforced in validate_mesh. |
| --user-width must be >= the mesh header width | Mesh-only; enforced in validate_mesh; header width is 6 * COORD_W + 32. |

## 9.2	Internal checks

Internal Validation Rules

| Rule | Rationale |
| --- | --- |
| num_masters >= 1 and num_slaves >= 1 | A crossbar needs at least one master and one slave. |
| data_width > 0 | Required for meaningful TDATA. |
| user_width >= 1 | TUSER is always at least one bit. |
| dest_width = ceil(log2(num_slaves)) | TDEST must encode all slave indices. |
| id_width = ceil(log2(num_masters)) | TID must encode all master indices. |

The generator does not currently validate that strb_width matches data_width/8; callers must ensure the width is consistent with their byte-qualifier policy.

## 9.3	Mesh topology validation

IMPLEMENTED These rules apply to the mesh CLI defined in the 2026-10-13 mesh design spec. They are enforced by validate_mesh in bin/delta_mesh/config.py.

Mesh Topology Validation Rules

| Rule | Rationale |
| --- | --- |
| cells-x >= 1, cells-y >= 1, cells-z >= 1 | A fabric needs at least one cell in each dimension. cells-z is the K-stack height; K = 1 is a single-layer mesh. |
| torus wrap in a dimension requires >= 2 cells in that dimension | A one-cell dimension cannot form a ring. |
| data-width > 0 | Same baseline requirement as flat mode. |
| user-width must fit the mesh flit header | The head-flit TUSER carries dest/src coordinates, QoS class, VC, multicast flag, misroute budget, source-route pointer, and packet ID. |
| --fifo-impl must be reg or sram | Selects the input buffer implementation style. The sram choice is accepted by validation but not yet wired to the cell (issue #118). |
| --amba5 still requires whole-byte data-width | AMBA 5 sidebands carry over unchanged from flat mode. |

### 9.3.1	Header width budget

The implemented flit header uses these fields on the head beat:

Implemented Head-Flit Header Fields

| Field | Width |
| --- | --- |
| dst_x, dst_y, dst_z | 3 * COORD_W |
| src_x, src_y, src_z | 3 * COORD_W |
| QoS class | 2 |
| multicast | 1 |
| VC | 1 |
| misroute budget | 4 |
| source-route pointer | 8 |
| packet ID | 16 |

COORD_W = max(1, ceil(log2(max(cells-x, cells-y, cells-z)))). The total header width is 6 * COORD_W + 32. Validation rejects a --user-width smaller than this value. The two VCs and three QoS classes are fixed; the header must encode VC (1 bit) and QoS (2 bits) without truncation.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 10	Pipeline Overview

The Delta generator uses the Module framework from bin/rtl_generators/verilog/. It builds the RTL as a sequence of method calls that append comments, instructions, and assignments to an internal buffer, then writes the completed module to disk.

### 10.0.1	Figure 3.1: Delta Generator Pipeline

Delta Generator Pipeline

The generator parses its flags into a DeltaConfig, builds the DeltaFlatCrossbar module skeleton, and verilog() appends the sections in file order: the always-present datapath, the AMBA 5 sections (--amba5), the counters (unless --no-counters) and the FORMAL-guarded assertions. --nodes also writes the 1:2 splitter. complete_tree_generator.py, the retired tree prototype, is a separate script that writes its primitives as strings.

## 10.1	Stages

- Argument parsing — CLI flags become DeltaConfig fields.

- Width calculation — dest_width and id_width are derived from num_slaves and num_masters unless overridden.

- Object construction — DeltaFlatCrossbar(config) declares parameters and ports through the framework.

- RTL emission — verilog() calls the generator methods in order:

  - generate_header_comment()

  - generate_request_decode()

  - generate_arbiter_logic()

  - generate_crossbar_mux()

  - generate_backpressure_logic()

  - generate_wakeup_logic() (AMBA 5 only)

  - generate_check_logic() (AMBA 5 only: odd-parity *CHK checks, sticky parity_error)

  - generate_performance_counters() (unless disabled)

  - generate_assertions()

- Framework finalization — start() emits the module header; end() emits endmodule.

- File write — write() saves delta_axis_flat_<M>x<N>.sv.

The complete_tree_generator.py script is independent and emits splitter/merger/tree scaffolds directly as strings.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 11	Request Decode

Delta uses TDEST for routing instead of an address range. This is the fundamental difference between Delta and the APB crossbar generator.

## 11.1	Request matrix

The generator declares a per-slave request vector:

logic [NUM_MASTERS-1:0] request_matrix [NUM_SLAVES];

Each entry is populated in an always_comb block:

for (int s = 0; s < NUM_SLAVES; s++)
    request_matrix[s] = '0;

for (int m = 0; m < NUM_MASTERS; m++)
    if (s_axis_tvalid[m] && (s_axis_tdest[m] < NUM_SLAVES))
        request_matrix[s_axis_tdest[m]][m] = 1'b1;

Only masters with a valid beat and a TDEST inside the supported slave range contribute to the request matrix.

## 11.2	Out-of-range TDEST

The generator includes a formal assertion that flags TDEST >= NUM_SLAVES. The behavior in simulation for an out-of-range TDEST is undefined; the request is simply dropped. System-level validation must ensure that masters never present an invalid destination.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 12	Arbitration and Routing

## 12.1	Packet-atomic round-robin arbitration

Unlike the APB crossbar, which re-arbitrates every cycle, Delta locks the grant until the end of the current packet. This prevents interleaving beats from different masters on the same slave.

logic [NUM_MASTERS-1:0] grant_matrix [NUM_SLAVES];
logic [$clog2(NUM_MASTERS)-1:0] last_grant [NUM_SLAVES];
logic packet_active [NUM_SLAVES];

Arbitration state machine for each slave:

- If packet_active[s] is true, hold the grant until m_axis_tvalid[s] && m_axis_tready[s] && m_axis_tlast[s].

- If no packet is active and request_matrix[s] is non-zero, choose the next requesting master in round-robin order after last_grant[s].

- Set packet_active[s] when a new grant is awarded.

## 12.2	Crossbar mux

A per-slave always_comb block muxes the granted master’s AXI-Stream signals to the slave output:

for (int m = 0; m < NUM_MASTERS; m++)
    if (grant_matrix[s][m]) begin
        m_axis_tdata[s]  = s_axis_tdata[m];
        m_axis_tvalid[s] = s_axis_tvalid[m];
        m_axis_tlast[s]  = s_axis_tlast[m];
        m_axis_tdest[s]  = s_axis_tdest[m];
        m_axis_tid[s]    = s_axis_tid[m];
        m_axis_tuser[s]  = s_axis_tuser[m];
        // TSTRB included when enabled; with --amba5 the payload check
        // bits (TDATACHK..TUSERCHK) ride the same grant, carried unchanged
    end

## 12.3	Backpressure routing

Each master’s s_axis_tready is driven from the m_axis_tready of the slave to which it is currently granted:

for (int m = 0; m < NUM_MASTERS; m++) begin
    s_axis_tready[m] = 1'b0;
    for (int s = 0; s < NUM_SLAVES; s++)
        if (grant_matrix[s][m])
            s_axis_tready[m] = m_axis_tready[s];
end

This preserves AXI-Stream ready/valid handshake semantics across the switch.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 13	Boundary Module Instantiation

Delta does not instantiate external boundary modules for the flat crossbar. The DeltaFlatCrossbar class directly emits the full crossbar logic using the Module framework. However, the generator does produce small protocol-agnostic primitives when tree generation is requested.

## 13.1	Flat crossbar

The entire crossbar is self-contained in delta_axis_flat_<M>x<N>.sv:

- No external arbiter module is instantiated; arbitration is emitted inline.

- No external FIFOs are used; the design is purely combinational between registered arbitration state.

## 13.2	Tree primitives

When --nodes is passed, delta_split_1to2.sv is emitted by the same generator. The complete_tree_generator.py script emits additional primitives:

Tree Primitives

| Primitive | Purpose |
| --- | --- |
| delta_split_1to2 | Routes one AXI-Stream input to two outputs based on a TDEST bit. |
| delta_merge_2to1 | Arbiters two inputs to one output with packet atomicity. |
| delta_fanout_1to<N> | Schematic fan-out tree using splitters (partial implementation). |
| delta_fanin_<N>to1 | Schematic fan-in tree using mergers (partial implementation). |

The tree primitives are not yet production-complete; they exist as building blocks and documentation of the intended topology.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 14	Directory Layout

delta/
├── bin/
│   ├── delta_generator.py            # framework-based flat crossbar generator
│   ├── complete_tree_generator.py    # tree primitive generator
│   ├── delta_generator_v1_backup.py  # legacy generator (reference only)
│   └── delta_performance_model.py    # performance estimation tool
├── rtl/
│   └── delta_axis_flat_<M>x<N>.sv    # generated flat crossbar
├── dv/
│   └── tests/                        # cocotb test suite
├── PRD.md                            # product requirements
└── CLAUDE.md                         # AI development guide

The generator writes directly into the directory specified by --output-dir (default ../rtl). Unlike the APB crossbar, Delta does not currently maintain a batch script with a fixed variant table; callers generate the sizes they need.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 15	Module Header and Parameters

The Module framework emits the module header automatically from the parameter and port strings added by DeltaFlatCrossbar.__init__.

## 15.1	Header comment

The generated file begins with a block comment documenting the configuration:

//==============================================================================
// Module: delta_axis_flat_2x4
// Project: Delta - AXI-Stream Crossbar Generator
//==============================================================================
// Description: AXI-Stream 2×4 Flat Crossbar Switch
//
// Configuration:
//   - Topology: flat
//   - Masters: 2
//   - Slaves: 4
//   - Data Width: 64 bits
//   - TDEST Width: 2 bits
//   - TID Width: 1 bits
//   - Arbiter: round_robin
//
// Features:
//   - Per-slave round-robin arbitration
//   - Packet atomicity (locked grant until TLAST)
//   - Registered outputs for timing closure
//   - Full AXI-Stream compliance
//==============================================================================

## 15.2	Port naming convention

Port Naming Convention

| Direction | Naming |
| --- | --- |
| Master-side inputs | s_axis_<signal>[NUM_MASTERS] |
| Master-side outputs | s_axis_tready[NUM_MASTERS] |
| Slave-side outputs | m_axis_<signal>[NUM_SLAVES] |
| Slave-side inputs | m_axis_tready[NUM_SLAVES] |

Optional signals (tstrb, twakeup, the t*chk check signals and the s_/m_parity_error flags) are declared only when enabled. With --amba5 the header also gains derived check-width parameters (TDATACHK_W = DATA_WIDTH / 8 and so on), and the module instantiates rtl/common/dataint_parity.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 16	Flat Crossbar

The flat crossbar is the production artifact. It is a single module that connects any master to any slave based on TDEST.

## 16.1	Internal structures

Flat Crossbar Internal Structures

| Signal / Variable | Purpose |
| --- | --- |
| request_matrix[NUM_SLAVES] | One-hot request vector per slave. |
| grant_matrix[NUM_SLAVES] | One-hot grant vector per slave. |
| last_grant[NUM_SLAVES] | Round-robin pointer per slave. |
| packet_active[NUM_SLAVES] | Lock grant until TLAST. |

## 16.2	Optional features

### 16.2.1	Performance counters

When add_performance_counters=True, the generator emits 32-bit packet counters:

logic [31:0] pkt_count_master [NUM_MASTERS];
logic [31:0] pkt_count_slave [NUM_SLAVES];

Counters increment on valid && ready && tlast at the master or slave interface.

### 16.2.2	Assertions

Under `ifdef FORMAL:

- One-hot grant check per slave.

- TDEST range check per master.

These assertions are intended for formal verification flows and are stripped during normal simulation if FORMAL is not defined.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 17	Tree Primitives

Note Tree generation was retired on 2026-10-12. The delta_merge_2to1, delta_fanout_1toN, and delta_fanin_Nto1 scaffolding remains in the tree for reference but is not part of the regression baseline. Hierarchy naming going forward follows the mesh design spec: cell / tile / confluence / fabric.

Delta includes scaffolding for tree topologies in addition to the flat crossbar. These primitives are generated separately and are intended for future hierarchical switches.

## 17.1	delta_split_1to2

Generated by delta_generator.py --nodes. It routes one AXI-Stream input to two outputs based on a configurable TDEST bit:

assign route_to_1 = s_axis_tdest[SPLIT_BIT];
assign m0_axis_tvalid = s_axis_tvalid && !route_to_1;
assign m1_axis_tvalid = s_axis_tvalid && route_to_1;
assign s_axis_tready  = route_to_1 ? m1_axis_tready : m0_axis_tready;

## 17.2	delta_merge_2to1

Generated by complete_tree_generator.py --type merger. It arbitrates two inputs to one output using round-robin with packet atomicity.

## 17.3	Fan-out / fan-in trees

complete_tree_generator.py --type fanout --size N and --type fanin --size N emit structural scaffolds for cascaded splitters and mergers. The implementations are currently partial; they illustrate the intended topology and are not yet part of the regression baseline.

## 17.4	Filelist convention

A complete Delta subsystem typically needs only:

rtl/delta_axis_flat_<M>x<N>.sv

Tree-based systems add the appropriate splitter/merger files. The component-level Makefile or area flow maintains the canonical filelist.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 18	Full Regeneration Rule

Any change to the files listed below must be followed by regeneration of the affected fixtures and a full regression run.

Sources That Force Regeneration

| Source | Why it forces regeneration |
| --- | --- |
| bin/delta_generator.py | All flat crossbar output comes from this file. |
| bin/complete_tree_generator.py | All tree primitive output comes from this file. |
| bin/rtl_generators/verilog/module.py | Framework changes affect header/footer formatting. |

## 18.1	Regeneration command

cd projects/components/fabric-gen-ip/delta/bin
python delta_generator.py --masters 2 --slaves 4 --output-dir ../rtl
python complete_tree_generator.py --type merger --output ../rtl

After regeneration, the only expected git diff should be in rtl/.

## 18.2	Do not hand-edit generated files

The .sv files in rtl/ are generator output. Hand-edits will be overwritten on the next regeneration. Fix the generator instead.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 19	Pre-Generated Fixtures

Delta does not currently ship a fixed family of pre-generated sizes. The recommended baseline fixtures are generated on demand with:

python delta_generator.py --masters 2 --slaves 4 --data-width 64 --output-dir ../rtl
python delta_generator.py --masters 4 --slaves 4 --data-width 64 --output-dir ../rtl
python delta_generator.py --masters 2 --slaves 4 --data-width 64 --amba5 --output-dir ../rtl

If the project later adopts a canonical fixture set, add a generate_all.py batch script and document the fixed sizes here.

## 19.1	Custom variants

Custom sizes are fully supported. When generating a custom variant, record the exact command in the subsystem integration notes so the fixture can be reproduced.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 20	DV Suite

The component-level DV suite lives in dv/tests/ and uses cocotb.

## 20.1	Test focus areas

A complete Delta DV suite should cover:

DV Test Scenarios

| Scenario | What to verify |
| --- | --- |
| Single-packet routing | A master sends a packet to each slave; TDEST selects correctly. |
| Multi-master contention | Two masters target the same slave; arbitration is fair and packet-atomic. |
| Backpressure | Assert m_axis_tready low; s_axis_tready follows correctly. |
| TLAST handling | Grants release only after TLAST. |
| AMBA 5 sidebands | TWAKEUP steers by TDEST. Payload *CHK is carried with TDATA. TVALIDCHK/TWAKEUPCHK/TREADYCHK are the inversions of their signals. A bad check sets the right sticky flag. |
| Out-of-range TDEST | Formal assertion fires; simulation drops the request. |
| Performance counters | Counters increment on valid && ready && tlast. |

## 20.2	Running the suite

cd projects/components/fabric-gen-ip/delta/dv/tests
make clean-all && make run-all-func-parallel

Today the suite is one file, test_delta_axis_flat_2x4_strb_amba5.py, covering the AMBA 5 row above against rtl_test/delta_axis_flat_2x4_strb_amba5.sv. It has three tests at gate, func and full. The other rows are still open for the base flavor.

After a generator change, this must pass with zero failures before the regenerated RTL is committed.

## 20.3	Adding a new test

- Add a new cocotb test file in dv/tests/.

- Reference the generated module under test.

- Add the test name to the area regression list.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 21	Lint Checks

The generated RTL must pass the project’s lint flow.

## 21.1	Common concerns

Lint Concerns and Mitigations

| Concern | Mitigation in generator |
| --- | --- |
| Unpacked array lint | Arrays are declared as logic [W-1:0] name [N]; ensure the lint tool supports unpacked array ports. |
| Width mismatches | DEST_WIDTH and ID_WIDTH are computed from slave/master counts. |
| Latch inference | All always_comb blocks assign all outputs; no incomplete cases. |
| Uninitialized logic | Default assignments precede conditional overrides in every mux. |

## 21.2	Synthesis smoke test

Run a synthesis tool on the 2×4 variant after any generator change. The expected result is clean elaboration with no combinational loops and no latch inference.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 22	Adding a New Topology

The mesh mode design spec (2026-10-13) is the current reference for adding a topology. The retired tree scaffolding is still in the repo, but it is not a regression baseline and should not be used as a model for new work.

Work through the design spec topics in this order, then land the generator changes:

- Topology semantics and routing concept — read §2 (hierarchy), §5 (routing), and §7 (systolic and pattern support) of the 2026-10-13 mesh design spec. Define the coordinate space, escape/adaptive VC policy, and how standard modes switch.

- Generator CLI axis — read §12. Choose the dimension parameters, optional torus dimensions, source-route and speculation flags, data/user widths, AMBA 5 carry-over, counters, and the FIFO implementation knob.

- Emitted artifacts — read §2.1. Mesh mode emits one cell module (delta_mesh_cell) and a fabric top. Cell / tile / confluence / fabric are placement and documentation names, not wrapper RTL levels.

- Validation rules — read §4 (flit format) and §12. Enforce dimension ranges, the torus-wrap minimum, and the TUSER width needed for the header fields (dest/src coords, QoS class, VC, multicast flag, misroute budget, source-route pointer, packet ID).

- DV suites — read §13. Plan routing, QoS, multicast, deadlock soak, systolic, counter, DFT, and perf-model calibration suites.

After the design is settled, the generator mechanics are the same as any new mode:

- Add a class in delta_generator.py (e.g., DeltaMeshCrossbar) and derive it from Module or reuse existing building blocks.

- Add the topology name to the --topology choices.

- In main(), instantiate the new class when the selected topology matches.

- Add a DV test that exercises the new topology.

- Regenerate fixtures and run the full DV suite.

## 22.1	Retired tree scaffolding

DeltaSplitter1to2 and complete_tree_generator.py remain for reference. A tree crossbar would have recursively instantiated splitters and mergers using TDEST bit slices, but that path was retired on 2026-10-12.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 23	Adding a New Generator Option

To add a scalar option:

- Add the field to DeltaConfig.

- Add the CLI argument to main() in delta_generator.py.

- Pass the argument into DeltaConfig(...).

- Use the config field inside the appropriate generate_* method.

- Document the option in this GAS, the HAS, and the PRD if it changes hardware behavior.

- Add validation in main() or inside the generator method.

- Add a DV test that exercises the new option.

## 23.1	Backward compatibility

Default values must preserve existing generated output. If the new option changes the default port list, introduce it as an opt-in flag first.

## 23.2	Example: adding a QoS sideband

@dataclass
class DeltaConfig:
    ...
    qos_width: int = 0

Then emit s_axis_tqos / m_axis_tqos arrays and mux them in generate_crossbar_mux().

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 24	Adding AMBA 5 Sidebands

The current AMBA 5 support covers TWAKEUP and the full IHI 0051B chapter 5 check-signal set (TDATACHK, TSTRBCHK, TLASTCHK, TIDCHK, TDESTCHK, TUSERCHK, TVALIDCHK, TREADYCHK, TWAKEUPCHK). To add more AMBA 5 sidebands:

- Add the new signal to DeltaConfig (e.g., a width field).

- In DeltaFlatCrossbar.__init__, add the new port declarations when amba5 is enabled.

- Add routing logic in a new generate_*_logic() method.

- Call the new method from verilog().

- If the new signal is covered by a check signal (a new payload field gets a *CHK per Table 5-1), add it to _payload_chk_fields(). That one list drives the ports, the carry mux, the ingress checkers and the error OR.

- Update the DV suite (dv/tests/test_delta_axis_flat_2x4_strb_amba5.py and its harness) to exercise the new sideband.

## 24.1	TWAKEUP design note

TWAKEUP is a hint, not a data beat. It is routed by TDEST without arbitration because it may precede TVALID. If a future sideband requires grant-based routing, follow the payload check-bit pattern in generate_crossbar_mux().

## 24.2	Check-signal design note

The payload check bits ride the same grant as TDATA and are carried, not regenerated, so a beat corrupted upstream still fails downstream. The crossbar also checks them on ingress (enabled by TVALID). It generates the single-bit checks for the signals it drives (TVALIDCHK and TWAKEUPCHK on slave ports, TREADYCHK on master ports) and checks the ones it receives. Every bit is one rtl/common/dataint_parity instance, odd parity, CHUNKS=1. A new check must follow the same rule: odd parity, one bit per 8 bits, top bit covers the remainder (IHI 0051B section 5.3).

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 25	Testing an Extension

Every generator change must be exercised by at least one generated fixture and one DV test.

## 25.1	Minimal test plan

- Generator unit test — call DeltaFlatCrossbar(cfg).verilog() with the new option and verify the emitted string contains the expected constructs.

- Fixture regeneration — generate the affected fixtures and inspect the diff.

- Simulation test — add or extend a cocotb test that exercises the new behavior.

- Lint/synthesis smoke — run the lint flow on the affected variant.

## 25.2	Example: adding a custom variant test

# dv/tests/test_delta_flat_4x4.py
import cocotb

class TestDeltaFlat4x4:
    @cocotb.test()
    async def test_basic_routing(self, dut):
        # exercise all 4 masters against all 4 slaves

The test must be added to the run-all-gate target or an equivalent regression list.

## 25.3	Commit discipline

Commit generator source changes, regenerated RTL, and DV tests in a single logical change. Do not commit hand-edited generated files.