RTL Design Sherpa

Lattice Micro-Architecture Specification 1.2

October 13, 2026

Table of Contents

Lattice Mas Index	11

Lattice Micro-Architecture Specification Index	12

Document Organization	13

Main Documentation	13

Chapter 1: Architecture	14

Chapter 2: Address Decode and Arbitration	15

Chapter 3: RTL Generator	16

Quick Navigation	17

For New Users	17

For Integration	17

Common Questions	17

Visual Assets	18

Architecture Diagrams	18

Timing Diagrams	18

Component Overview	19

Key Features	19

Pre-Generated Variants	19

Design Philosophy	19

Related Documentation	20

Companion Specifications	20

Project-Level	20

Test Infrastructure	20

RTL	20

Version History	21

Lattice Visual Documentation	22

Overview	23

Documentation Structure	24

Main Documentation Files	24

Supporting Documentation	24

Visual Assets	25

Architecture Diagrams	25

Timing Diagrams	25

Quick Start	26

For New Users	26

For Existing Users	26

Directory Structure	27

Key Concepts	28

1. Architecture	28

2. Address Decode	28

3. Arbitration	28

4. RTL Generation	28

Common Questions	30

Q: Which pre-generated variant should I use?	30

Q: How do I change the base address?	30

Q: Can I change per-slave address sizes?	30

Q: How does arbitration work with 3+ masters?	30

Q: What’s the throughput?	31

Regenerating Diagrams	32

Graphviz Diagrams (Architecture, Address Decode)	32

WaveJSON Diagrams (Timing)	32

Version History	33

Maintainers	34

Lattice Architecture	35

Overview	36

Architecture Diagram	37

Figure 1.1: Lattice Top-Level Architecture	37

Functional Blocks	38

1. Master-Side Protocol Conversion	38

2. Internal Crossbar Logic	38

3. Slave-Side Protocol Conversion	38

Signal Flow Example	40

Parameter Configuration	41

APB5 Parity Across the Fabric	42

APB5 PWAKEUP Across the Fabric	43

Pre-Generated Variants	44

Design Philosophy	45

Next Steps	46

Address Decode and Arbitration	47

Overview	48

Address Decode	49

Address Map Structure	49

Decode Algorithm	49

Out-of-Range Addresses	49

Address Decode Flow Diagram	50

Figure 2.1: Address Decode Flow	50

Decode Example Walkthrough	51

Multiple Address Maps	51

Arbitration	52

Per-Slave Round-Robin	52

Round-Robin Timing Diagram	52

Arbitration Example Walkthrough	52

Multi-Slave Parallelism	53

Grant Persistence	54

Arbitration Latency	54

Integration Examples	55

Example 1: CPU to 4 Peripherals (No Contention)	55

Example 2: CPU + DMA to 4 Peripherals (Potential Contention)	55

Performance Characteristics	56

Throughput	56

Latency	56

Design Notes	57

Why 64KB Per Slave?	57

Why Round-Robin?	57

Troubleshooting	58

Issue: Wrong Slave Selected	58

Issue: Master Starved	58

Issue: Back-to-Back Transactions Stalling	58

Next Steps	59

Lattice RTL Generator	60

Overview	61

Generator Architecture	62

Two-Level Structure	62

Quick Start	63

Generate Standard Variants (1:1, 2:1, 1:4, 2:4)	63

Generate Custom Variant	63

Generate with Custom Base Address	63

Command-Line Interface	65

Full Options	65

Examples	66

Generated Code Structure	67

Template Components	67

Generator Algorithm	71

Step-by-Step Process	71

Code Generation Patterns	73

Loop-Based Generation	73

String Formatting	73

Customization Examples	74

Example 1: Large SoC with 10 Peripherals	74

Example 2: Multi-CPU System	74

Example 3: Memory-Mapped I/O Region	74

Using Generated Modules	76

Integration Example	76

Parameter Override	77

Generator Limitations and Future Work	78

Current Limitations	78

Future Enhancements	78

Testing Generated Code	80

Verification Flow	80

Recommended Test Cases	80

Advanced Usage	81

Using as Python Library	81

Batch Generation	81

Troubleshooting	82

Issue: “ModuleNotFoundError: No module named ‘amba.apbx_xbar_generator’”	82

Issue: Generated Code Won’t Lint	82

Issue: Generated Code Too Large	82

References	83

Verification Wrappers	84

List of Figures

Figure 1.1: Lattice Top-Level Architecture	37

Figure 2.1: Address Decode Flow	50

List of Tables

Table 1: Pre-Generated Variants	19

Table 2: Main Documentation Files	24

Table 3: Supporting Documentation	24

Table 4: Architecture Diagram Assets	25

Table 5: Timing Diagram Assets	25

Table 6: Generated-Variant Parameters	41

Table 7: Pre-Generated Crossbar Variants	44

Table 8: Command Line Options	65

Table 9: Python API Arguments	65

Table 10: Verification Wrapper Parameters	84

List of Waveforms

Waveform 2.1: Round-Robin Arbitration Timing	52

# Lattice Mas Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Lattice Micro-Architecture Specification Index

Component: Lattice (MxN Interconnect) Version: 1.4 Date: 2026-10-20 Purpose: Detailed micro-architecture specification for Lattice component

## Document Organization

This specification covers the Lattice: a parametric MxN interconnect that routes multiple APB masters to multiple APB slaves with automatic address-based routing and round-robin arbitration.

### Main Documentation

README.md

## Chapter 1: Architecture

chapters/01_architecture.md

## Chapter 2: Address Decode and Arbitration

chapters/02_address_and_arbitration.md

## Chapter 3: RTL Generator

chapters/03_rtl_generator.md

## Quick Navigation

### For New Users

- Start with README.md for overview and quick start

- Read 01_architecture.md to understand the design

- Study 02_address_and_arbitration.md for operational details

- Reference 03_rtl_generator.md if you need a custom configuration

### For Integration

- Pre-generated variants: See 01_architecture.md Section “Pre-Generated Variants”

- Custom generation: See 03_rtl_generator.md Section “Quick Start”

- Address mapping: See 02_address_and_arbitration.md Section “Address Decode”

- Arbitration behavior: See 02_address_and_arbitration.md Section “Arbitration”

### Common Questions

All answered in README.md, section “Common Questions”.

## Visual Assets

Every diagram in this specification is checked in as both source and rendered output:

- Source Files:

  - assets/graphviz/*.dot - Graphviz source diagrams

  - assets/wavedrom/*.json - WaveJSON timing diagrams

- Rendered Files:

  - assets/graphviz/*.png - PNG format (for document generation)

### Architecture Diagrams

- Lattice Architecture (2x4 Example)

  - Source: assets/graphviz/apbx_xbar_architecture.dot

  - Rendered: assets/graphviz/apbx_xbar_architecture.png

- Address Decode Flow

  - Source: assets/graphviz/address_decode_flow.dot

  - Rendered: assets/graphviz/address_decode_flow.png

### Timing Diagrams

- Round-Robin Arbitration

  - Source: assets/wavedrom/arbitration_round_robin.json

  - Rendered: assets/wavedrom/arbitration_round_robin.png

## Component Overview

### Key Features

- Parametric MxN Configuration: Any combination of M masters and N slaves (up to 16x16)

- Automatic Address Decode: 64KB per slave, simple offset-based routing

- Round-Robin Arbitration: Per-slave fair arbitration, no master starvation

- Back-to-Back Transactions: Accepted without master-side idle cycles, though they do not overlap inside the fabric (10 pclk cycles each for M = 1, 11 for arbitrated variants; one more than that variant’s single-transfer latency — see HAS 5.2)

- Grant Persistence: Grant held through transaction completion

- RTL Generation: Python-based generator for custom configurations

### Pre-Generated Variants

Pre-Generated Variants

| Module | M×N | Use Case |
| --- | --- | --- |
| apbx_xbar_1to1 | 1×1 | Passthrough, protocol conversion |
| apbx_xbar_2to1 | 2×1 | Multi-master arbitration |
| apbx_xbar_1to4 | 1×4 | Simple SoC peripheral bus |
| apbx_xbar_2to4 | 2×4 | Typical SoC with CPU+DMA |
| apbx_xbar_2to2_mixed | 2×2 | Mixed APB4/APB5 (m0=APB4, m1=APB5; s0=APB5, s1=APB4) |

### Design Philosophy

Proven Building Blocks: - Built from production-tested apb4_slave and apb4_master modules - No new protocol logic - pure composition - Each component independently verified

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants for fast integration - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol → cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp → protocol

## Related Documentation

### Companion Specifications

- Lattice HAS - Hardware Architecture Specification (high-level)

### Project-Level

- PRD.md: ../../PRD.md - Complete product requirements document

- CLAUDE.md: ../../CLAUDE.md - AI assistant integration guide

- README.md: ../../README.md - Quick start guide

### Test Infrastructure

- Test Directory: ../../dv/tests/ - CocoTB + pytest test suite

- Test Results: 100% passing (9/9) — all pre-generated variants plus the dedicated timing test; apbx_xbar_2to2_mixed additionally has a formal harness

### RTL

- Core Modules: ../../rtl/apbx_xbar_*.sv - Pre-generated crossbars

- Wrappers: ../../rtl/wrappers/ - testbench scaffolds (pclk/presetn only), not integration wrappers

- Generator: ../../bin/generate_xbars.py - Python generator script

## Version History

Version 1.0 (2025-10-25): - Initial specification release - Complete visual documentation (3 diagrams) - Generator documentation - All pre-generated variants verified (100% passing)

Version 1.1 (2026-10-12): - Refreshed against current RTL, generator, and DV - Updated verification status (9/9 tests passing, mixed-variant formal harness) - Added --slave-size CLI option to generator table - Updated line-of-code counts (single source of truth remains HAS ch05_performance/03_resources.md)

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Lattice Visual Documentation

Component: Lattice (MxN Interconnect) Version: 1.2 Status: Production Ready Last Updated: 2026-10-12

## Overview

This directory holds the Lattice’s documentation set: the architecture chapters, their diagrams and timing waveforms, and the generator guide.

Component Purpose: Parametric MxN APB interconnect connecting multiple masters to multiple slaves with automatic address-based routing and round-robin arbitration.

## Documentation Structure

### Main Documentation Files

Main Documentation Files

| Document | Description | Contents |
| --- | --- | --- |
| 01_architecture.md | Architecture Overview | Top-level block diagram, functional blocks, signal flow, parameters |
| 02_address_and_arbitration.md | Address Decode & Arbitration | Address map structure, decode algorithm, round-robin timing |
| 03_rtl_generator.md | RTL Generator Guide | Generator architecture, usage, customization, advanced topics |

### Supporting Documentation

Supporting Documentation

| Document | Location | Description |
| --- | --- | --- |
| PRD.md | ../../PRD.md | Complete product requirements document |
| CLAUDE.md | ../../CLAUDE.md | AI assistant integration guide |
| README.md | ../../README.md | Quick start guide |

## Visual Assets

### Architecture Diagrams

Architecture Diagram Assets

| Diagram | Format | Description |
| --- | --- | --- |
| apbx_xbar_architecture | Graphviz | 2x4 crossbar showing master-side slaves, arbitration, slave-side masters |
| address_decode_flow | Graphviz | Step-by-step address decode example (0x10023456 to Slave 2) |

Rendered Format: PNG beside each source in assets/graphviz/ (what the chapters embed).

### Timing Diagrams

Timing Diagram Assets

| Diagram | Format | Description |
| --- | --- | --- |
| arbitration_round_robin | WaveJSON | 2 masters competing for Slave 0 with round-robin arbitration |

Rendered Format: - PNG: assets/wavedrom/arbitration_round_robin.png

## Quick Start

### For New Users

- Understand the Architecture

  - Read 01_architecture.md

  - Study the architecture diagram

  - Understand master-side/slave-side protocol conversion

- Learn Address Decode & Arbitration

  - Read 02_address_and_arbitration.md

  - Study address decode flow diagram

  - Study arbitration timing diagram

- Generate Custom Crossbar (if needed)

  - Read 03_rtl_generator.md

  - Use pre-generated variants (1to1, 2to1, 1to4, 2to4, 2to2_mixed) if possible

  - Run generator for custom MxN configurations

- Integrate Into Your Design

  - See integration examples in CLAUDE.md

  - Reference complete specification in PRD.md

### For Existing Users

Need a Crossbar? - 1 master, N slaves → Use apbx_xbar_1toN or generate - 2 masters, N slaves → Use apbx_xbar_2toN or generate - M masters, N slaves → Generate with generate_xbars.py

Understanding Behavior? - Address decode issues → See 02_address_and_arbitration.md - Arbitration questions → See 02_address_and_arbitration.md

Modifying/Generating? - Generator usage → See 03_rtl_generator.md

## Directory Structure

docs/lattice_mas/
├── README.md                              ← This file
├── lattice_mas_index.md                  ← Book index (PDF build)
├── chapters/
│   ├── 01_architecture.md                 ← Architecture overview
│   ├── 02_address_and_arbitration.md      ← Decode & arbitration details
│   └── 03_rtl_generator.md                ← Generator documentation
└── assets/
    ├── graphviz/                          ← Diagram sources + rendered PNGs
    │   ├── apbx_xbar_architecture.dot / .png
    │   ├── address_decode_flow.dot / .png
    │   └── regenerate_all_graphviz.sh
    └── wavedrom/                          ← Source timing diagrams
        └── arbitration_round_robin.json

## Key Concepts

### 1. Architecture

Three-Stage Design: 1. Master-Side: APB slaves convert protocol → cmd/rsp 2. Internal Logic: Address decode + arbitration + response routing 3. Slave-Side: APB masters convert cmd/rsp → protocol

Why This Design? - Reuses proven apb4_slave and apb4_master components - Clean separation of concerns - Scalable to any MxN configuration

### 2. Address Decode

Fixed 64KB Per Slave:

Slave 0: BASE_ADDR + 0x00000 - 0x0FFFF
Slave 1: BASE_ADDR + 0x10000 - 0x1FFFF
Slave 2: BASE_ADDR + 0x20000 - 0x2FFFF
...

Decode Formula:

offset = PADDR - BASE_ADDR
slave_index = offset[16 +: $clog2(S)]  // [17:16] for S=4, [16] for S=2

See: 02_address_and_arbitration.md

### 3. Arbitration

Per-Slave Round-Robin: - Each slave has independent arbiter - Priority rotates after each grant - Grant persists through transaction completion - No master starvation

See: 02_address_and_arbitration.md

### 4. RTL Generation

All Generator Code in Component Area: - Convenience Wrapper: bin/generate_xbars.py (top-level script) - Core Generator: bin/apbx_xbar_generator.py (library implementation)

Usage:

# Generate standard variants
python generate_xbars.py

# Generate custom 3x6 crossbar
python generate_xbars.py --masters 3 --slaves 6

See: 03_rtl_generator.md

## Common Questions

### Q: Which pre-generated variant should I use?

Decision Tree:

1 master, 1 slave  → apbx_xbar_1to1 (passthrough)
1 master, 4 slaves → apbx_xbar_1to4 (address decode only)
2 masters, 1 slave → apbx_xbar_2to1 (arbitration only)
2 masters, 4 slaves → apbx_xbar_2to4 (full crossbar)
2 masters, 2 slaves (mixed APB4/APB5) → apbx_xbar_2to2_mixed
Other MxN          → Generate with generate_xbars.py

### Q: How do I change the base address?

At Instantiation:

apbx_xbar_1to4 #(
    .BASE_ADDR(32'h8000_0000)  // Override default 0x10000000
) u_xbar (...);

Or Generate with Custom Base:

python generate_xbars.py --masters 2 --slaves 4 --base-addr 0x80000000

### Q: Can I change per-slave address sizes?

No: each slave is fixed at a 64KB window. Only the map’s origin is settable, via the BASE_ADDR parameter.

Workarounds: 1. Use multiple crossbars with different BASE_ADDR 2. Modify generator’s decode logic (advanced) 3. Address translation in slaves

See: 03_rtl_generator.md#limitations

### Q: How does arbitration work with 3+ masters?

Round-Robin Example (4 Masters, Same Slave):

Initial priority: M0
Transaction 1: M0 requests → M0 granted → Priority rotates to M1
Transaction 2: M0, M1, M2 request → M1 granted → Priority rotates to M2
Transaction 3: M0, M3 request → M2 skipped (not requesting) → M3 granted → Priority rotates to M0
Transaction 4: M0, M1 request → M0 granted → Priority rotates to M1

Key: Priority rotates after each grant, ensuring fairness.

See: 02_address_and_arbitration.md#arbitration

### Q: What’s the throughput?

Single Master: - Back-to-back transactions without master-side gaps (no overlap inside the fabric; 10 pclk cycles each for M = 1, 11 when arbitrated) - Limited only by slave PREADY response time

Multiple Masters (Same Slave): - Fair sharing: Each master gets ~1/M bandwidth - Example: 2 masters = 50% each, 4 masters = 25% each

Multiple Masters (Different Slaves): - Full parallelism: Each master gets 100% of its target slave - Total system bandwidth = (number of slaves) × slave_bandwidth

See: 02_address_and_arbitration.md#performance

## Regenerating Diagrams

### Graphviz Diagrams (Architecture, Address Decode)

# Renders every .dot beside itself as PNG (150 dpi, alpha flattened, PNG8)
assets/graphviz/regenerate_all_graphviz.sh

### WaveJSON Diagrams (Timing)

cd assets/wavedrom/

# Generate SVG, then PNG
wavedrom-cli -i arbitration_round_robin.json -s arbitration_round_robin.svg
wavedrom-cli -i arbitration_round_robin.json -p arbitration_round_robin.png

## Version History

Version 1.0 (2025-10-25): - Initial visual documentation release - 3 main documentation files - 2 Graphviz architecture diagrams - 1 WaveJSON timing diagram - RTL generator documentation

Version 1.1 (2026-10-12): - Refreshed against current RTL, generator, and DV - Added mixed-variant and timing-test guidance

## Maintainers

RTL Design Sherpa Project

Questions or Issues: 1. Check this documentation first 2. Review PRD.md for complete specification 3. Review CLAUDE.md for integration guidance 4. Open a GitHub issue if the problem persists

Last Updated: 2026-10-12 Version: 1.2 Status: Production Ready

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Lattice Architecture

Component: Lattice (MxN Interconnect) Version: 1.2 Status: Production Ready

## Overview

The Lattice is a parametric interconnect that connects M APB masters to N APB slaves with automatic address-based routing and per-slave round-robin arbitration. It is a pure composition of the proven apb4_slave and apb4_master components — there is no new protocol logic in the crossbar itself.

Key Features: - Arbitrary MxN configuration (up to 16x16) - Automatic address decode (64KB per slave) - Round-robin arbitration per slave - Back-to-back transactions (no overlap; 10 pclk cycles each for M = 1, 11 when arbitrated) - Grant persistence through transaction completion

## Architecture Diagram

The 2x4 configuration below connects 2 masters (CPU and DMA) to 4 slaves (UART, GPIO, Timer, SPI):

### Figure 1.1: Lattice Top-Level Architecture

Lattice Architecture (2x4 Example)

Both masters are drawn, for one slave s. Each master’s cmd is decoded into a request bit for the slave it addresses; the slave’s arbiter_round_robin (the trapezium) grants one master and holds the grant until the response handshake (WAIT_GNT_ACK = 1); the command mux and apb4_master carry the granted cmd to the slave. The response climbs the right column, steered back by the slave index registered at cmd accept, or replaced by a local PSLVERR on a decode miss.

## Functional Blocks

### 1. Master-Side Protocol Conversion

Component: apb4_slave[M] instances (one per master)

Purpose: Convert incoming APB protocol transactions to internal cmd/rsp bus format

Features: - Full APB protocol handling (PSEL, PENABLE, PREADY) - Transaction buffering - Error response generation - Back-to-back transaction support

Dataflow:

APB Master → apb4_slave → cmd/rsp bus → Internal Crossbar Logic

### 2. Internal Crossbar Logic

Components: - Address Decode - Parallel decode to determine target slave - Per-Slave Arbiters - Round-robin arbitration for each slave - Response Routing - Registered paths back to requesting masters

Key Operations:

Address Decode:

offset = PADDR - BASE_ADDR
slave_index = offset[16 +: $clog2(S)]  // [17:16] for S=4, [16] for S=2

Arbitration: - Independent arbiter per slave - Round-robin priority rotation - Grant persistence through response - No starvation guarantee

Response Routing: - Track which master initiated each transaction - Route PRDATA/PSLVERR back to originating master - Maintain transaction ordering per master

### 3. Slave-Side Protocol Conversion

Component: apb4_master[N] instances (one per slave)

Purpose: Convert internal cmd/rsp bus format back to APB protocol for slaves

Features: - APB protocol generation (PSEL, PENABLE timing) - Response collection (PRDATA, PSLVERR, PREADY) - Wait state handling - Error propagation

Dataflow:

Internal Crossbar Logic → cmd/rsp bus → apb4_master → APB Slave

## Signal Flow Example

Transaction: Master 0 (CPU) writes to Slave 2 (Timer) at address 0x10023456

Step-by-step:

- Master 0 → apb4_slave[0]:

  - CPU asserts PSEL, PADDR=0x10023456, PWRITE=1

  - apb4_slave[0] converts to cmd/rsp format

- Address Decode:

  - offset = 0x10023456 - 0x10000000 = 0x00023456

  - slave_index = 0x00023456 >> 16 = 0x2

  - Target: Slave 2

- Arbiter[2]:

  - Check if Slave 2 is available

  - Grant to Master 0 (if no conflict)

  - Route cmd to apb4_master[2]

- apb4_master[2] → Slave 2:

  - Generate APB write transaction

  - Assert PSEL[2], PENABLE, PADDR, PWDATA

  - Wait for PREADY

- Response Path:

  - Slave 2 responds with PREADY, PSLVERR

  - apb4_master[2] captures response

  - Response routed back to apb4_slave[0]

  - apb4_slave[0] returns PREADY to CPU

Total Latency: 9 cycles SETUP-to-PREADY uncontended on a single-master variant, 10 when arbitrated; sustained back-to-back cadence is 10 and 11 respectively (measured; see 2.x — the fabric’s boundary IP and registered skid buffers dominate, not APB’s 2-cycle protocol minimum)

## Parameter Configuration

Every variant has its M×N baked in by the generator, so there is no port-count parameter to override at elaboration — a different shape means regenerating, not re-elaborating. What remains parameterizable:

Generated-Variant Parameters

| Parameter | Range | Default | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | 18-64 (decoding variants); 1-64 (1to1/2to1) | 32 | Address bus width |
| DATA_WIDTH | 8-64 | 32 | Data bus width |
| STRB_WIDTH | derived | DATA_WIDTH/8 | Write strobe width |
| BASE_ADDR | any except the top S x 64KB (where BASE+span wraps 32-bit) | 0x10000000 | Base of the slave address map |

Port counts, per-slave window size and APB version per port are generator inputs, not parameters — see chapter 3.

Derived: - Slave address range: 64KB (0x10000) per slave, as the shipped variants are generated (the generator’s own default window is 4KB) - Total address space: S × 64KB

## APB5 Parity Across the Fabric

Parity is carried only between two APB5 ports. A mixed pairing ignores parity entirely, gated by the same MST_APB5/SLV_APB5 masks as the rest of the APB5 sideband — there is no separate policy knob.

The parity pins are the AMBA APB (IHI 0024E) chapter 5 *CHK signals: PADDRCHK, PCTRLCHK, PSELCHK, PENABLECHK, PWDATACHK, PSTRBCHK, PAUSERCHK, PWUSERCHK, PWAKEUPCHK, PRDATACHK, PREADYCHK, PSLVERRCHK, PRUSERCHK and PBUSERCHK. Parity is odd, with one check bit per 8 bits of the covered signal (Table 5-1).

On an APB5→APB5 path the fabric checks and regenerates, and its architecture leaves no alternative. The boundary IP deconstructs each transfer into cmd/rsp, and the parity bits do not cross that interface: apb5_slave checks on the way in, apb5_master regenerates on the way out. The cmd/rsp fabric between them is therefore outside the protected domain — corruption inside that span is invisible, because the regenerated bit is correct by construction and masks it. That span is documented rather than hidden, and because of it each APB5 port brings its parity_error_* flag out individually: a check whose result goes nowhere is not protection.

The error flags are deliberately not folded into PSLVERR, which would make a fabric fault indistinguishable from the slave’s own error response.

None of this is present in the SHIPPED variants. All of the above describes a variant generated with enable_parity=True. Every crossbar in rtl/ is generated with parity OFF: apbx_xbar_2to2_mixed declares no parity ports at all on its module boundary, ties the boundary IP’s parity inputs to '0 and leaves its parity_error_* outputs unconnected. So there is nothing to connect and nothing to monitor unless you generate your own variant with the flag set — and a mixed APB4/APB5 pairing carries no parity in any case.

Formal coverage: formal/apbx_xbar/apbx_xbar_2to2_mixed/ proves the mixed configuration as of 2026-08-29, replacing the proof lost with the thin core on 2026-08-27. Note it proves the SIDEBAND gate, not a parity gate — a mixed pairing carries no parity at all here, so there is no parity property left to state. The generated variants are instantiated with ENABLE_PARITY=0, and the APB4-port never-sees-parity question the thin core answered does not arise when the port has no parity pins. Parity itself is covered in simulation by test_apbx_xbar_parity.py, which generates an all-APB5 2x2 variant with enable_parity=True.

## APB5 PWAKEUP Across the Fabric

PWAKEUP is a Requester output (IHI 0024E section 3.7.2, Table 3-7). It is an input on an APB5 master port. On an APB5 slave port, where the fabric is the Requester, it is an output driven by:

s<j>_apb_PWAKEUP = <apb5_master boundary IP's registered wake>
                 | m<i>_apb_PWAKEUP  (every APB5 master)

The boundary term is high while a command for that slave is pending or a transfer is in flight. It covers APB4 masters, which have no PWAKEUP, and it satisfies the 3.7.2 rule that PWAKEUP stays high until PREADY once it is high with PSEL. The master terms forward a requester’s wake ahead of its PSEL. That forwarding is not routed by decode, because every master can reach every slave and the address is not known before PSEL. The OR is unregistered, and 3.7.2 names a glitch-free OR tree as a valid construction. The slave’s side carries no wake back to any master. formal/apbx_xbar/apbx_xbar_2to2_mixed/ proves the forwarding (ap_pwakeup_forwarded), the wake under every PSEL (ap_pwakeup_covers_psel) and the quiet reset state (ap_pwakeup_not_fabricated).

## Pre-Generated Variants

Pre-Generated Crossbar Variants

| Module | M×N | Use Case |
| --- | --- | --- |
| apbx_xbar_1to1 | 1×1 | Passthrough, protocol conversion |
| apbx_xbar_2to1 | 2×1 | Multi-master arbitration |
| apbx_xbar_1to4 | 1×4 | Simple SoC peripheral bus |
| apbx_xbar_2to4 | 2×4 | Typical SoC with CPU+DMA |
| apbx_xbar_2to2_mixed | 2×2 | Mixed APB4/APB5 (m0=APB4, m1=APB5; s0=APB5, s1=APB4), per-port version gating |

LOC per variant is in HAS ch05_performance/03_resources.md, which is the single source of truth for it. The column that used to sit here went stale within two days of being reconciled.

Custom Variants: Generated on-demand via Python script

## Design Philosophy

Proven Components: - Built from production-tested apb4_slave.sv and apb4_master.sv - No new protocol logic - pure composition - Each component independently verified

Scalability: - Parametric generation for any MxN configuration - Resource usage scales linearly with M×N - No centralized bottlenecks

Predictability: - Round-robin arbitration provides deterministic behavior - Fixed address map simplifies software integration - Deterministic transfer cost: 9 pclk cycles SETUP-to-PREADY (10 back-to- back) with one master, 10 (11 back-to-back) when arbitrated

## Next Steps

- See Address Decode and Arbitration for detailed operation examples

- See PRD.md for complete specification

- See CLAUDE.md for integration guidance

- See README.md for quick start guide

Version: 1.2 Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Address Decode and Arbitration

Component: Lattice (MxN Interconnect) Version: 1.2 Status: Production Ready

## Overview

Two mechanisms do the crossbar’s work: 1. Address Decode - How incoming addresses map to specific slaves 2. Arbitration - How multiple masters share access to the same slave

## Address Decode

### Address Map Structure

The shipped variants use a 64KB (0x10000 bytes) region per slave, fixed for a given generated module and chosen at generation via --slave-size:

BASE_ADDR + 0x0000_0000 → 0x0000_FFFF : Slave 0 (64KB)
BASE_ADDR + 0x0001_0000 → 0x0001_FFFF : Slave 1 (64KB)
BASE_ADDR + 0x0002_0000 → 0x0002_FFFF : Slave 2 (64KB)
BASE_ADDR + 0x0003_0000 → 0x0003_FFFF : Slave 3 (64KB)
...
BASE_ADDR + 0x000F_0000 → 0x000F_FFFF : Slave 15 (64KB)

Default BASE_ADDR: 0x10000000

Total Address Space: (number of slaves) × 64KB

### Decode Algorithm

The crossbar extracts the slave index from the upper bits of the address offset:

offset = PADDR - BASE_ADDR
slave_index = offset[16 +: $clog2(S)]   // ceil(log2(S)) bits above the 64KB window

### Out-of-Range Addresses

This applies to the variants that DECODE — 1to4, 2to4, 2to2_mixed. In 1to1 and 2to1 there is a single slave, no address decode, and BASE_ADDR is unused: every access is forwarded to that slave whatever its address, and the slave’s own response is returned.

For the decoding variants, an access outside [BASE_ADDR, BASE_ADDR + S x 64KB) is a decode miss. The crossbar accepts it and answers locally with PSLVERR; no slave sees the transfer, and the arbiters are not involved. The master’s transaction completes normally (PREADY asserts) with the error flagged, so a bad pointer surfaces as an APB error response rather than a hang.

Earlier RTL left cmd_ready low on a miss, which wedged the master in ACCESS with PREADY low and no error signature (projects/components/fabric-gen-ip/lattice-apbx-xbar TASK-002, was APBX-002). Regression: the decode-miss scenarios in test_apbx_xbar_1to4.py, test_apbx_xbar_2to4.py, and test_apbx_xbar_2to2_mixed.py.

Why bit 16 upward? - 64KB = 0x10000 = 2^16 bytes - Lower 16 bits (offset[15:0]) are byte address within slave - ceil(log2(S)) bits starting at bit 16 select the slave: [17:16] for a 4-slave variant, [16] for 2 slaves. The shipped variants wire exactly those bits — m0_slave_sel is 2 bits wide on 1to4/2to4, 1 bit on 2to2_mixed, NOT a 4-bit index - The generator supports up to 16 slaves; bits above the index field do not select a slave, they fail the range check and become a decode miss

### Address Decode Flow Diagram

The diagram walks one concrete address — 0x10023456 — through the decode to Slave 2:

### Figure 2.1: Address Decode Flow

Address Decode Flow

### Decode Example Walkthrough

Scenario: Master accesses address 0x10023456 with BASE_ADDR = 0x10000000

Step 1: Calculate Offset

offset = PADDR - BASE_ADDR
       = 0x10023456 - 0x10000000
       = 0x00023456

Step 2: Extract Slave Index

slave_index = offset[17:16]        // S=4 -> 2 bits
            = 0x00023456 >> 16
            = 0x2

Step 3: Parallel Decode

The crossbar checks all slave ranges in parallel: - Slave 0: 0x00000 - 0x0FFFF → NO MATCH - Slave 1: 0x10000 - 0x1FFFF → NO MATCH - Slave 2: 0x20000 - 0x2FFFF → MATCH - Slave 3: 0x30000 - 0x3FFFF → NO MATCH

Step 4: Route to Arbiter

Transaction request forwarded to Arbiter[2] (Slave 2’s arbiter)

Step 5: Forward to Slave

After arbitration grant, transaction forwarded via apb4_master[2] to physical Slave 2

Final Address Sent to Slave: 0x10023456 (full address preserved)

### Multiple Address Maps

Different BASE_ADDR values create distinct address maps:

Example: Two Crossbars

// Peripheral Bus: 0x1000_0000 - 0x1003_FFFF
apbx_xbar_1to4 #(
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (...);
// Slaves at: 0x1000_0000, 0x1001_0000, 0x1002_0000, 0x1003_0000

// Memory-Mapped I/O: 0x8000_0000 - 0x8003_FFFF
apbx_xbar_1to4 #(
    .BASE_ADDR(32'h8000_0000)
) u_mmio_xbar (...);
// Slaves at: 0x8000_0000, 0x8001_0000, 0x8002_0000, 0x8003_0000

## Arbitration

### Per-Slave Round-Robin

Each slave has an independent arbiter that implements round-robin scheduling:

Key Properties: - Fair: No master can starve another - Independent: Each slave arbitrates separately - Predictable: Priority rotates after each grant - Persistent: Grant held from command through response

### Round-Robin Timing Diagram

Two masters (M0, M1) competing for access to Slave 0:

#### Waveform 2.1: Round-Robin Arbitration Timing

Round-Robin Arbitration Timing

### Arbitration Example Walkthrough

Scenario: Master 0 and Master 1 both want to access Slave 0

Initial State: - Priority: M0 (M0 has priority initially) - Slave 0: IDLE

This walkthrough shows the ORDER of arbitration events, not their timing. It is deliberately not cycle-numbered: a numbered version here showed a three-cycle transaction, which contradicted this chapter’s own measured 9-cycle SETUP-to-PREADY figure. S0_PSEL cannot assert in the grant cycle — after the REGISTERED grant the command still crosses the apb4_master cmd skid, the FSM’s IDLE launch and its SETUP state before the downstream PSEL appears. For real numbers see “Transaction Latency” below and dv/tests/test_apbx_xbar_timing.py.

Note also that M0_PENABLE is driven by the external master during its own ACCESS phase; the arbiter does not assert it, and it is high before any grant exists.

Transaction 1: M0 requests Slave 0

M0 asserts PSEL with PADDR=0x1000_0000
  Arbiter[0] grants M0 (only requester)
  ... command traverses the fabric ...
  Slave 0 responds; transaction completes
  Priority rotates: M1 now has priority

Transaction 2: M0 and M1 both request Slave 0

M0 asserts PSEL with PADDR=0x1000_0010
  M1 asserts PSEL with PADDR=0x1000_0000   -- CONFLICT
  Arbiter[0] grants M1 (has priority); M0 waits
  ... command traverses the fabric ...
  Slave 0 responds; M1's transaction completes
  Priority rotates: M0 now has priority

Transaction 3: M0 again, after rotation

M0 is still asserting (it was blocked)
  Arbiter[0] grants M0 (now has priority)
  ... command traverses the fabric ...
  Slave 0 responds; transaction completes
  Priority rotates: M1 now has priority

Result: Fair access — each master is served in turn when both request. The grant is held from command acceptance to the response handshake (WAIT_GNT_ACK(1)), so a blocked master waits one full transaction, not one cycle.

### Multi-Slave Parallelism

Key Feature: Different slaves can be accessed simultaneously by different masters

Example: Parallel Transactions

Time T0:
- Master 0 accesses Slave 0 (UART) - GRANTED
- Master 1 accesses Slave 2 (Timer) - GRANTED
Both transactions proceed in parallel (no conflict)

Time T1:
- Master 0 accesses Slave 0 (UART) - GRANTED
- Master 1 accesses Slave 0 (UART) - BLOCKED (arbitration)
Master 1 waits for Master 0 to complete

Time T2:
- Master 0 completes, releases Slave 0
- Master 1 accesses Slave 0 (UART) - GRANTED
Priority rotated for Slave 0's arbiter

Benefit: Maximum throughput when masters access different slaves

### Grant Persistence

Critical Property: Once granted, a master holds the slave until response completes

Why This Matters:

WITHOUT Grant Persistence:
- Master 0 granted Slave 0
- Master 0 asserts PENABLE
- *Grant could change here* ← BREAKS PROTOCOL!
- Slave 0 responds to wrong master

WITH Grant Persistence:
- Master 0 granted Slave 0
- Master 0 asserts PENABLE
- Grant held until PREADY asserted ← SAFE
- Slave 0 responds to correct master
- Grant released for next transaction

Implementation: Grant signal registered and held from PSEL assertion through PREADY response

### Arbitration Latency

Best Case (No Contention): - 1 cycle: the grant is REGISTERED (grant <= w_next_grant; in arbiter_round_robin), so request → grant is never same-cycle - Fabric overhead is the dominant term, not APB’s 2-cycle minimum: a full transfer measures 10 cycles SETUP-to-PREADY on an arbitrated variant (see “Transaction Latency” below). Everything in this chapter is an M > 1 figure — there is no arbiter at all when M = 1, and those variants are a cycle faster.

Worst Case (Maximum Contention): - Wait for the masters ahead in the round-robin to complete FULL transactions — the grant is held until the response handshake (WAIT_GNT_ACK(1)), so each is a full transaction, not 1 cycle. The right figure for a QUEUED transaction is the back-to-back period of 11 cycles, not the 10-cycle single-transfer latency — a waiting master inherits the same mandatory SETUP cycle that separates any two consecutive transfers - Example: 2 masters, worst case ~= one full transaction (11 cycles) of wait - Example: 4 masters, worst case = 3 full transactions ~= 33 cycles

Average Case (Random Access Pattern): - (M-1)/2 full transactions of average wait (~11 cycles each) - Statistical fairness over time

## Integration Examples

### Example 1: CPU to 4 Peripherals (No Contention)

apbx_xbar_1to4 #(
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (
    // Single master (CPU)
    // .m0_apb_PSEL(cpu_apb_PSEL), .m0_apb_PENABLE(...), ...  (elided)

    // 4 slaves (UART, GPIO, Timer, SPI)
    .s0_apb_* (uart_*),   // 0x1000_0000
    .s1_apb_* (gpio_*),   // 0x1001_0000
    .s2_apb_* (timer_*),  // 0x1002_0000
    .s3_apb_* (spi_*)     // 0x1003_0000
);

Behavior: - No arbitration needed (single master) - Pure address decode functionality - Zero arbitration overhead

### Example 2: CPU + DMA to 4 Peripherals (Potential Contention)

apbx_xbar_2to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_soc_xbar (
    // Two masters
    // .m0_apb_PSEL(cpu_apb_PSEL), .m0_apb_PENABLE(...), ...  (elided)
    // .m1_apb_PSEL(dma_apb_PSEL), ...                        (elided)

    // 4 slaves
    .s0_apb_* (mem_ctrl_*),  // 0x4000_0000
    .s1_apb_* (uart_*),      // 0x4001_0000
    .s2_apb_* (i2c_*),       // 0x4002_0000
    .s3_apb_* (adc_*)        // 0x4003_0000
);

Behavior: - Each slave has independent arbiter - CPU and DMA can access different slaves simultaneously - If both access same slave, round-robin arbitration - Fair access guaranteed

## Performance Characteristics

### Throughput

Single Master: - Back-to-back transactions supported, but NOT overlapped: apb4_slave is a one-command-at-a-time FSM, so the next command is captured only after the previous transaction completes - Measured sustained cadence: 1 transfer per 11 pclk cycles (PREADY-to-PREADY) at an always-ready slave, on an arbitrated variant. That is one MORE than its 10-cycle SETUP-to-PREADY single-transfer latency: nothing overlaps, and the next transfer’s mandatory SETUP cycle cannot share a cycle with the previous transfer’s ACCESS. (An earlier “zero bubble” claim described a pipeline this RTL does not have; a later “cadence equals latency” claim dropped the SETUP cycle.)

Multiple Masters (Same Slave): - Round-robin introduces fair sharing - Each master gets ~1/M of bandwidth - Example: 2 masters = 50% bandwidth each

Multiple Masters (Different Slaves): - Full parallelism - Each master gets 100% of its target slave - Total system bandwidth = (number of slaves) × slave_bandwidth

### Latency

Components:

- Address Decode: 0 cycles (combinational, parallel)

- Arbitration Decision: 1 cycle (grant is registered)

- Master APB phases: 2 cycles (PSEL, then PENABLE)

- Boundary IP + skid buffers: ~6 cycles — apb4_slave capture, registered cmd skid, apb4_master IDLE→SETUP→ACCESS, registered rsp skid, apb4_slave BUSY→PREADY

- Slave Response: Variable (adds to the above)

Total Minimum Latency: 10 cycles SETUP-to-PREADY (9 of them ACCESS->PREADY) on an arbitrated variant, uncontended with a zero-wait slave — measured on apbx_xbar_2to1, and the same on 2to4 and 2to2_mixed. A single-master variant has no arbiter and measures 9 (8 fabric). APB’s 2-cycle minimum applies to a directly-attached slave; it does not apply through this fabric, which converts APB→cmd/rsp→APB across registered buffers in both directions.

Sustained cadence is 11 cycles, PREADY-to-PREADY, not 10 — see “Single Master” above and HAS 5.2 for why the two differ and for the M = 1 column.

With Contention: add one full transaction (~11 cycles) per master ahead in the round-robin

## Design Notes

### Why 64KB Per Slave?

Rationale: - Sufficient for most APB peripherals (typically 256B - 4KB register space) - Simple decode logic (single shift operation) - Allows up to 16 slaves with clean byte alignment - Software-friendly (each peripheral has “round” base address)

Alternatives: - Smaller regions (4KB, 16KB) → more slaves, more complex decode - Larger regions (256KB, 1MB) → fewer slaves, wasted address space

### Why Round-Robin?

Advantages: - Fair: Prevents starvation - Simple: Minimal logic (counter + comparator) - Predictable: Software can reason about worst-case latency - No configuration: Works out-of-box

Alternatives: - Fixed priority → Can starve low-priority masters - Weighted fair queuing → More complex, requires configuration - Lottery scheduling → Unpredictable, harder to verify

## Troubleshooting

### Issue: Wrong Slave Selected

Check: 1. BASE_ADDR parameter correct? 2. Address within expected range? 3. Calculated slave_index matches expectation?

Debug:

offset = PADDR - BASE_ADDR
slave_index = offset >> 16
expected_slave = slave_index  // Should match actual PSEL

### Issue: Master Starved

Check: 1. Other masters continuously accessing same slave? 2. Round-robin priority rotating correctly? (grant is released at the RESPONSE handshake — grant_ack = grant && rsp_valid && rsp_ready with WAIT_GNT_ACK(1) — so a master that never completes a transaction holds its grant) 3. Is the starved master’s address actually decoding in range? An out-of-range access completes locally with PSLVERR and never reaches an arbiter.

There is no timeout or monitoring knob to enable: arbiter_round_robin has no timeout logic, parameter, or port. An earlier revision of this checklist referenced one.

Expected Behavior: - Each master should get grant within M transactions - No master should wait indefinitely

### Issue: Back-to-Back Transactions Stalling

Check: 1. Grant persistence working? 2. Slave asserting PREADY correctly? 3. No unintended pipeline bubbles?

Expected Behavior: - Consecutive transactions from same master should flow without gaps - Only arbitration conflicts should introduce wait states

## Next Steps

- See Architecture for top-level design overview

- See PRD.md for complete specification

- See CLAUDE.md for integration guidance

- See README.md for quick start guide

Version: 1.2 Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Lattice RTL Generator

Component: Lattice (MxN Interconnect) Version: 1.2 Status: Production Ready

## Overview

Five pre-generated variants (1to1, 2to1, 1to4, 2to4, 2to2_mixed) cover the common configurations. For anything else, the Python-based generator produces any arbitrary MxN crossbar up to 16x16.

Philosophy: - Pre-generated variants for common use cases (fast integration) - Python generator for custom configurations (flexibility) - Template-based code generation (consistency) - Built from proven components (quality)

## Generator Architecture

### Two-Level Structure

projects/components/fabric-gen-ip/lattice-apbx-xbar/
├── bin/
│   └── generate_xbars.py              ← Convenience wrapper script
│
└── projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/
    └── apbx_xbar_generator.py          ← Core generator engine

Why Two Files? 1. Core Generator (apbx_xbar_generator.py): Reusable library function - Can be imported by other scripts - Contains generate_apbx_xbar() function - Located in the component’s own bin/ directory: projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/apbx_xbar_generator.py. It is NOT under bin/rtl_generators/ — that path was claimed here until 2026-08-29 and contradicted this chapter’s own tree, its troubleshooting section, the README and the PRD. bin/rtl_generators/amba/ contains only an __init__.py.

- Convenience Wrapper (generate_xbars.py): Project-specific script

  - Easy command-line interface

  - Pre-configured for standard variants

  - Located in project bin/ directory

## Quick Start

### Generate Standard Variants (1:1, 2:1, 1:4, 2:4)

cd projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/
python generate_xbars.py

Output:

Generating 1-to-1 crossbar...
  apbx_xbar_1to1.sv
Generating 2-to-1 crossbar...
  apbx_xbar_2to1.sv
Generating 1-to-4 crossbar...
  apbx_xbar_1to4.sv
Generating 2-to-4 crossbar...
  apbx_xbar_2to4.sv
Generating 2-to-2_mixed crossbar...
  apbx_xbar_2to2_mixed.sv

Generated 5 crossbar variants

### Generate Custom Variant

cd projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

Output:

Generating 3-to-6 crossbar...
Generated apbx_xbar_3to6.sv

### Generate with Custom Base Address

cd projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/
python generate_xbars.py --masters 4 --slaves 8 --base-addr 0x80000000

Output:

Generating 4-to-8 crossbar...
Generated apbx_xbar_4to8.sv

Address Map:
  Slave 0: [0x80000000, 0x8000FFFF]
  Slave 1: [0x80010000, 0x8001FFFF]
  Slave 2: [0x80020000, 0x8002FFFF]
  ...
  Slave 7: [0x80070000, 0x8007FFFF]

## Command-Line Interface

### Full Options

python generate_xbars.py [OPTIONS]

Options:

Command Line Options

| Option | Short | Type | Default | Description |
| --- | --- | --- | --- | --- |
| --masters | -m | int | None | Number of master interfaces (1-16) |
| --slaves | -s | int | None | Number of slave interfaces (1-16) |
| --base-addr | -b | hex | 0x10000000 | Base address for slave address map |
| --slave-size | - | hex | 0x10000 | Address space per slave: 0x1000=4KB, 0x10000=64KB (matches shipped variants) |
| --help | -h | - | - | Show help message |

The CLI covers the common cases. Per-port APB version and parity are reachable only through the Python API (generate_apbx_xbar), since they take per-port lists rather than scalars:

Python API Arguments

| Argument | Type | Default | Description |
| --- | --- | --- | --- |
| slave_size | int | 0x1000 | Address window per slave. generate_xbars.py passes 0x10000 for the shipped variants |
| master_versions | list | None | Per-master APB version (4/5); None ⇒ all APB4 |
| slave_versions | list | None | Per-slave APB version (4/5); None ⇒ all APB4 |
| name_suffix | str | "" | Appended to the generated module name |
| enable_parity | bool | False | Carry APB5 parity (IHI 0024E ch. 5 *CHK signals, odd) on APB5 ports |

enable_parity checks and regenerates; it does not pass through. A generated variant checks parity at the ingress boundary and regenerates it on the far side, because the parity bits do not cross the cmd/rsp interface the boundary IP deconstructs each transfer into. The cmd/rsp fabric between the two is outside the protected domain, so with enable_parity=True each APB5 port exposes its own parity_error_* outputs (m0_parity_error_wdata, s0_parity_error_rdata, …) — a check whose result goes nowhere is not protection. APB4 ports get none, and the shipped apbx_xbar_2to2_mixed is generated with parity off, so there the boundary IP’s flags are tied off internally. A mixed APB4/APB5 pairing carries no parity at all.

(The retired apbx_xbar_thin passed parity through end to end instead, which is why this distinction used to be drawn against it. It was deleted on 2026-08-27; there is no other family to contrast with now.)

Rules: - If neither --masters nor --slaves specified → Generate all standard variants - If only one specified → Error (must specify both) - If both specified → Generate custom variant

### Examples

Example 1: Generate All Standard Variants

python generate_xbars.py

Example 2: Generate 3x6 Crossbar

python generate_xbars.py --masters 3 --slaves 6

Example 3: Generate 5x10 Crossbar with Custom Base

python generate_xbars.py -m 5 -s 10 -b 0xA0000000

Example 4: Show Help

python generate_xbars.py --help

## Generated Code Structure

### Template Components

The generator creates a complete SystemVerilog module with:

- Header with Address Map

// 2-to-4 APB crossbar with address decoding and arbitration
// 2 masters to 4 slaves using apb4_slave and apb4_master modules
//
// (This walkthrough shows the SHIPPED apbx_xbar_2to4 so every constant
//  below is real (2to4). For a 3-to-6 the shapes scale: 3-bit slave_sel,
//  a 0x00060000 range bound, and CLIENTS(3) arbiters.)
//
// Address Map (same for all masters):
//   Slave 0: [0x10000000, 0x1000FFFF]
//   Slave 1: [0x10010000, 0x1001FFFF]
//   Slave 2: [0x10020000, 0x1002FFFF]
//   Slave 3: [0x10030000, 0x1003FFFF]

- Module Declaration with Parameters

module apbx_xbar_2to4 #(
    parameter int ADDR_WIDTH = 32,
    parameter int DATA_WIDTH = 32,
    parameter int STRB_WIDTH = DATA_WIDTH / 8,
    parameter logic [ADDR_WIDTH-1:0] BASE_ADDR = 32'h10000000
) (
    // Clock and Reset
    input  logic                  pclk,
    input  logic                  presetn,

    // Master 0-1 APB interfaces
    // Slave 0-3 APB interfaces
);

- Internal Signal Declarations

// Command/Response interfaces for master 0-1 apb4_slave
    logic m0_cmd_valid, m0_cmd_ready, m0_cmd_pwrite;
    logic [ADDR_WIDTH-1:0] m0_cmd_paddr;
    // ... (full cmd/rsp bus for each master)

    // Command/Response interfaces for slave 0-3 apb4_master
    logic s0_cmd_valid, s0_cmd_ready, s0_cmd_pwrite;
    logic [ADDR_WIDTH-1:0] s0_cmd_paddr;
    // ... (full cmd/rsp bus for each slave)

- Master-Side apb4_slave Instantiations

// Master 0 apb4_slave
    apb4_slave #(
        .ADDR_WIDTH(ADDR_WIDTH),
        .DATA_WIDTH(DATA_WIDTH)
    ) u_apb4_slave_m0 (
        .pclk(pclk),
        .presetn(presetn),
        // APB interface
        .s_apb_PSEL(m0_apb_PSEL),
        .s_apb_PENABLE(m0_apb_PENABLE),
        // ... (full APB connections)
        // cmd/rsp interface
        .cmd_valid(m0_cmd_valid),
        .cmd_ready(m0_cmd_ready),
        // ... (full cmd/rsp connections)
    );

    // Master 1 apb4_slave instance...

- Address Decode Logic

// Per-master offset decode (verbatim shape of the real output --
    // there is no request-matrix signal; per-slave cmd_valid terms are
    // built directly from the decoded index)
    logic [ADDR_WIDTH-1:0] m0_cmd_offset;
    logic [1:0] m0_slave_sel;
    logic m0_addr_in_range;

    always_comb begin
        m0_cmd_offset    = m0_cmd_paddr - BASE_ADDR;
        m0_addr_in_range = (m0_cmd_paddr >= BASE_ADDR) &&
                           (m0_cmd_paddr < (BASE_ADDR + 32'h00040000));
        m0_slave_sel     = m0_cmd_offset[17:16];   // ceil(log2(S)) bits
    end

    assign s0_cmd_valid = m0_cmd_valid && m0_addr_in_range && (m0_slave_sel == 2'd0);
    assign s1_cmd_valid = m0_cmd_valid && m0_addr_in_range && (m0_slave_sel == 2'd1);
    // ... one per slave. For M>1 these terms are NOT repeated per
    // master onto the same signal -- the per-slave arbiter grant
    // selects which master drives it:
    //   case (1'b1)
    //     s0_arb_grant[0]: s0_cmd_valid = m0_cmd_valid && m0_addr_in_range && (m0_slave_sel == 2'd0);
    //     s0_arb_grant[1]: s0_cmd_valid = m1_cmd_valid && m1_addr_in_range && (m1_slave_sel == 2'd0);
    //   endcase

- Per-Slave Arbitration Logic

// One arbiter instance per slave (no hand-rolled priority
    // counters in the output -- the generator instantiates the
    // library arbiter):
    arbiter_round_robin #(
        .CLIENTS(2),          // = M (3 for a 3-to-6)
        .WAIT_GNT_ACK(1)      // hold grant until the response handshake
    ) u_s0_arbiter (
        .clk(pclk), .rst_n(presetn), .block_arb(1'b0),
        .request(s0_arb_request), .grant_ack(s0_arb_grant_ack),
        .grant(s0_arb_grant),
        .grant_valid(), .grant_id(), .last_grant()
    );

- Response Routing Logic

// Route responses back to requesting masters
    always_comb begin
        // Track which master initiated each transaction
        // Route PRDATA/PSLVERR back to correct master
    end

- Slave-Side apb4_master Instantiations

// Slave 0 apb4_master
    apb4_master #(
        .ADDR_WIDTH(ADDR_WIDTH),
        .DATA_WIDTH(DATA_WIDTH)
    ) u_s0_master (
        .pclk(pclk),
        .presetn(presetn),
        // cmd/rsp interface
        .cmd_valid(s0_cmd_valid),
        .cmd_ready(s0_cmd_ready),
        // ... (full cmd/rsp connections)
        // APB interface -- apb4_master's ports are m_apb_*, so the
        // slave-side port sN_apb_* connects to m_apb_* on the instance
        .m_apb_PSEL     (s0_apb_PSEL),
        .m_apb_PENABLE  (s0_apb_PENABLE),
        // ... (full APB connections)
    );

    // Slave 1-3 apb4_master instances...

- Module End

endmodule : apbx_xbar_2to4

## Generator Algorithm

### Step-by-Step Process

1. Validate Parameters

if M < 1 or M > 16:
    raise ValueError(f"Number of masters must be 1-16, got {M}")
if N < 1 or N > 16:
    raise ValueError(f"Number of slaves must be 1-16, got {N}")

2. Calculate Derived Parameters

slave_addr_bits = max(1, math.ceil(math.log2(N)))  # Bits for slave index
strb_width = data_width // 8                        # Byte strobe width
module_name = f"apbx_xbar_{M}to{N}"                 # Module name

3. Generate Address Map Documentation

for s in range(N):
    addr_offset = s * 0x10000  # 64KB per slave
    addr_start = base_addr + addr_offset
    addr_end = addr_start + 0xFFFF
    # Generate comment: Slave S: [START, END]

4. Generate Module Declaration - Parameters: ADDR_WIDTH, DATA_WIDTH, STRB_WIDTH, BASE_ADDR - Ports: Clock, reset - Master interfaces: M × full APB interface - Slave interfaces: N × full APB interface

5. Generate Internal Signals - Master cmd/rsp buses: M sets - Slave cmd/rsp buses: N sets - Request matrix: [M][N] - Grant matrix: [N][M]

(No priority counters are emitted. Rotation state lives inside the arbiter_round_robin instances the generator wires up, which is what the code sample further down says.)

6. Instantiate Master-Side Components

for m in range(M):
    # Generate apb4_slave instance
    # Connect APB interface to external master m
    # Connect cmd/rsp to internal buses

7. Generate Address Decode Logic

for m in range(M):
    # For each master, decode address to slave index
    # offset = paddr - BASE_ADDR
    # slave_idx = offset[16 +: ceil(log2(S))]  # S-sized index above 64KB
    # Set m_to_s_req[m][slave_idx] = 1

8. Generate Arbitration Logic

for s in range(N):
    # For each slave, arbitrate among M masters
    # Round-robin: priority rotates after each grant
    # Grant persistence: hold until transaction completes

9. Generate Response Routing

# Track transaction source (which master)
# Route responses back to originating master

10. Instantiate Slave-Side Components

for s in range(N):
    # Generate apb4_master instance
    # Connect cmd/rsp from internal buses
    # Connect APB interface to external slave s

11. Write Generated Code

with open(output_file, 'w') as f:
    f.write(generated_code)

## Code Generation Patterns

### Loop-Based Generation

Master Interfaces:

for m in range(M):
    code += f"    // Master {m} APB interface\n"
    code += f"    input  logic m{m}_apb_PSEL,\n"
    code += f"    input  logic m{m}_apb_PENABLE,\n"
    # ... (all APB signals for master m)

Slave Interfaces:

for s in range(N):
    code += f"    // Slave {s} APB interface\n"
    code += f"    output logic s{s}_apb_PSEL,\n"
    code += f"    output logic s{s}_apb_PENABLE,\n"
    # ... (all APB signals for slave s)

### String Formatting

Hex Values:

code += f"    parameter logic [ADDR_WIDTH-1:0] BASE_ADDR = {addr_width}'h{base_addr:08X}\n"
# Example: 32'h10000000

Address Map Comments:

addr_start = base_addr + (s * 0x10000)
code += f"//   Slave {s}: [0x{addr_start:08X}, 0x{addr_start + 0xFFFF:08X}]\n"
# Example: Slave 2: [0x10020000, 0x1002FFFF]

## Customization Examples

### Example 1: Large SoC with 10 Peripherals

Scenario: CPU + DMA need to access 10 peripherals

Command:

python generate_xbars.py --masters 2 --slaves 10

Generated File: apbx_xbar_2to10.sv

Address Map:

Slave 0:  0x10000000 - 0x1000FFFF (UART0)
Slave 1:  0x10010000 - 0x1001FFFF (UART1)
Slave 2:  0x10020000 - 0x1002FFFF (GPIO)
Slave 3:  0x10030000 - 0x1003FFFF (I2C)
Slave 4:  0x10040000 - 0x1004FFFF (SPI)
Slave 5:  0x10050000 - 0x1005FFFF (Timer0)
Slave 6:  0x10060000 - 0x1006FFFF (Timer1)
Slave 7:  0x10070000 - 0x1007FFFF (PWM)
Slave 8:  0x10080000 - 0x1008FFFF (ADC)
Slave 9:  0x10090000 - 0x1009FFFF (Watchdog)

LOC: ~2500 lines (extrapolated, not generated and counted — the measured anchors are in HAS ch05_performance/03_resources.md)

### Example 2: Multi-CPU System

Scenario: 4 CPUs accessing shared peripheral bus with 4 slaves

Command:

python generate_xbars.py --masters 4 --slaves 4

Generated File: apbx_xbar_4to4.sv

Benefits: - Fair arbitration among 4 CPUs - Each slave independently arbitrated - Maximum throughput when CPUs access different slaves

LOC: ~1600 lines (extrapolated, see above)

### Example 3: Memory-Mapped I/O Region

Scenario: Single master accessing I/O region at 0xA0000000

Command:

python generate_xbars.py --masters 1 --slaves 8 --base-addr 0xA0000000

Generated File: apbx_xbar_1to8.sv

Address Map:

Slave 0:  0xA0000000 - 0xA000FFFF
Slave 1:  0xA0010000 - 0xA001FFFF
...
Slave 7:  0xA0070000 - 0xA007FFFF

## Using Generated Modules

### Integration Example

// Generated file: apbx_xbar_3to6.sv

module my_soc (
    input  logic clk,
    input  logic rst_n,
    // ... other interfaces
);

    // Instantiate 3x6 crossbar
    apbx_xbar_3to6 #(
        .ADDR_WIDTH(32),
        .DATA_WIDTH(32),
        .BASE_ADDR(32'h10000000)  // Can override at instantiation
    ) u_periph_xbar (
        .pclk(clk),
        .presetn(rst_n),

        // Connect 3 masters (CPU, DMA, Debug)
        .m0_apb_PSEL(cpu_apb_psel),
        .m0_apb_PENABLE(cpu_apb_penable),
        // ... (full CPU APB interface)

        .m1_apb_PSEL(dma_apb_psel),
        .m1_apb_PENABLE(dma_apb_penable),
        // ... (full DMA APB interface)

        .m2_apb_PSEL(debug_apb_psel),
        .m2_apb_PENABLE(debug_apb_penable),
        // ... (full Debug APB interface)

        // Connect 6 slaves (UART, GPIO, Timer, SPI, I2C, ADC)
        .s0_apb_PSEL(uart_psel),
        .s0_apb_PENABLE(uart_penable),
        // ... (full UART APB interface)

        .s1_apb_PSEL(gpio_psel),
        // ... (remaining slaves)
    );

endmodule : my_soc

### Parameter Override

// Use different base address at instantiation
apbx_xbar_3to6 #(
    .BASE_ADDR(32'h80000000)  // Override default 0x10000000
) u_mmio_xbar (
    // ... connections
);
// Slaves now at: 0x80000000, 0x80010000, 0x80020000, ...

## Generator Limitations and Future Work

### Current Limitations

- Fixed 64KB Per Slave

  - Cannot customize individual slave region sizes

  - Workaround: Use multiple crossbars with different BASE_ADDR

- Max 16x16 Configuration

  - Practical limit due to arbitration complexity

  - Workaround: Hierarchical crossbars

- Single Address Map

  - All masters see same slave address map

  - Workaround: Address translation before crossbar

- No Priority Levels

  - Round-robin only, no fixed priority or weighted arbitration

  - Workaround: External priority arbitration before crossbar

### Future Enhancements

1. Variable Slave Sizes

# Potential future API
generate_apbx_xbar(
    num_masters=2,
    slave_sizes=[4096, 65536, 1024, 16384],  # Custom sizes per slave
    base_addr=0x10000000
)

2. Arbitration Policies

# Potential future API
generate_apbx_xbar(
    num_masters=4,
    num_slaves=4,
    arbitration="weighted",  # "round-robin", "fixed-priority", "weighted"
    weights=[4, 2, 1, 1]     # Master 0 gets 4x priority
)

3. Address Translation

# Potential future API
generate_apbx_xbar(
    num_masters=2,
    num_slaves=4,
    master_views=[
        {"base": 0x00000000, "remap": True},   # Master 0 sees 0x0
        {"base": 0x10000000, "remap": False}   # Master 1 sees 0x1000_0000
    ]
)

## Testing Generated Code

### Verification Flow

1. Generate Crossbar

python generate_xbars.py --masters 2 --slaves 4

2. Lint Generated RTL

cd ../../rtl/
verilator --lint-only apbx_xbar_2to4.sv

3. Run Functional Tests

cd ../../dv/tests/
make run-apbx_xbar_2to4-gate

4. View Waveforms (If Test Fails)

make run-apbx_xbar_2to4-gate-waves
# create_view_cmd() writes the gtkwave command beside the log

### Recommended Test Cases

- Single Master Access - Verify basic routing

- Multi-Master Contention - Verify arbitration

- Back-to-Back Transactions - Verify consecutive transfers complete at the expected 10/11-cycle cadence

- Address Decode Boundary - Verify slave selection at boundaries

- Error Response Propagation - Verify PSLVERR routing

- APB5 Sideband Gating - Verify apbx_xbar_2to2_mixed sideband behavior

- Timing Compliance - Verify published latency/cadence with test_apbx_xbar_timing

## Advanced Usage

### Using as Python Library

# Import generator function
# generator lives in the component's bin/, so put it on the path first
import sys; sys.path.insert(0, 'projects/components/fabric-gen-ip/lattice-apbx-xbar/bin')
from apbx_xbar_generator import generate_apbx_xbar

# Generate crossbar programmatically
code = generate_apbx_xbar(
    num_masters=3,
    num_slaves=8,
    base_addr=0xA0000000,
    addr_width=32,
    data_width=32,
    output_file="my_custom_xbar.sv"
)

# Generated code is returned as string
print(f"Generated {len(code)} bytes of SystemVerilog code")

# Can also write to file manually
with open("custom_output.sv", "w") as f:
    f.write(code)

### Batch Generation

# Generate multiple variants in one script
variants = [
    (1, 4, 0x10000000),
    (2, 4, 0x20000000),
    (3, 6, 0x30000000),
    (4, 8, 0x40000000),
]

for masters, slaves, base_addr in variants:
    code = generate_apbx_xbar(
        num_masters=masters,
        num_slaves=slaves,
        base_addr=base_addr,
        output_file=f"xbar_{masters}to{slaves}_0x{base_addr:08X}.sv"
    )
    print(f"Generated xbar_{masters}to{slaves}_0x{base_addr:08X}.sv")

## Troubleshooting

### Issue: “ModuleNotFoundError: No module named ‘amba.apbx_xbar_generator’”

Cause: Python can’t find the core generator module

Solution:

# Verify generator exists
ls -la projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/apbx_xbar_generator.py

# Run from bin directory
cd projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/
python generate_xbars.py

### Issue: Generated Code Won’t Lint

Cause: Generator bug or unsupported configuration

Solution: 1. Check parameter ranges (1-16 for M and N) 2. No span alignment is required — the slave index comes from the OFFSET. The one illegal region is the top S x 64KB of the address space, where BASE_ADDR + S*64KB wraps 32-bit and every access becomes a decode miss 3. Report bug with command that caused issue

### Issue: Generated Code Too Large

Cause: Very large MxN configuration (e.g., 16x16)

Expected Behavior: - 1x1: 167 LOC (measured) - 2x4: 792 LOC (measured) - 16x16: ~25,000 LOC (extrapolated; the practical limit)

Solution: - Consider hierarchical approach (multiple smaller crossbars) - Or use different interconnect topology (bus, tree, etc.)

## References

- Architecture Overview

- Address Decode and Arbitration

- PRD.md - Complete specification

- CLAUDE.md - Integration guidance

- README.md - Quick start guide

Version: 1.2 Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

## Verification Wrappers

rtl/wrappers/*_wrap.sv holds four modules — apbx_xbar_1to1_wrap, apbx_xbar_1to4_wrap, apbx_xbar_2to1_wrap, apbx_xbar_2to4_wrap.

The generator does not emit them. They are hand-written testbench scaffolding, and a variant gets one only when a test needs it. The clearest evidence is apbx_xbar_2to2_mixed: it is generator output like every other variant, and it has no wrapper at all, because its test drives the DUT directly. Regenerating the family produces no wrappers; generating a new custom MxN produces none either.

They are also not design entities and must not be instantiated in a system. Each declares only pclk and presetn as ports, holds every master- and slave-side APB signal as an internal net, and instantiates the crossbar against those nets. The cocotb testbenches use the wrapper as toplevel and drive the bus by hierarchical name.

Keeping the bus off the port list is deliberate: the AXI/APB factory helpers match signals by name pattern, and a wrapper that re-exported m0_apb_* and s0_apb_* at its own boundary would give those patterns two matches per signal. The internal-net form leaves exactly one.

Verification Wrapper Parameters

| Parameter | Default | On which wrappers | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | 32 | all four | Forwarded to the wrapped crossbar |
| DATA_WIDTH | 32 | all four | Forwarded to the wrapped crossbar |
| BASE_ADDR | 32'h1000_0000 | 1to4, 2to4 only | Forwarded to the crossbar’s decode base. Absent on 1to1/2to1, matching those variants, where BASE_ADDR is inert because there is nothing to decode |