RTL Design Sherpa

APB GPIO Micro-Architecture Specification 1.0

January 4, 2026

Table of Contents

1 Gpio Mas Index	15

2 APB GPIO - Overview	16

2.1 Overview	16

2.1.1 Features	16

2.1.2 Applications	16

2.1.3 Figure 1.1: APB GPIO Block Diagram	17

2.2 Parameters	18

2.2.1 Register Summary	18

2.3 Navigation	20

3 APB GPIO - Architecture	21

3.1 Overview	21

3.1.1 Figure 1.2: APB GPIO Top-Level Architecture	21

3.1.2 Figure 1.3: APB GPIO Module Hierarchy	21

3.2 Functional Description	22

3.2.1 Data Flow	22

3.2.2 Clock Domains	24

3.3 Parameters	26

3.4 Design Notes	27

3.4.1 Resource Estimates	27

3.5 Navigation	28

4 APB GPIO - Clocks and Reset	29

4.1 Overview	29

4.1.1 Clock Signals	29

4.1.2 Reset Signals	29

4.2 Functional Description	30

4.2.1 Reset Behavior	30

4.2.2 Clock Domain Crossing	30

4.2.3 Input Synchronization	31

4.3 Timing	32

4.3.1 Timing Constraints	32

4.4 Navigation	33

5 APB GPIO - Acronyms and Terminology	34

5.1 Overview	34

5.1.1 Protocol Acronyms	34

5.1.2 Signal Acronyms	34

5.1.3 GPIO-Specific Terms	34

5.1.4 Interrupt Terms	35

5.1.5 Register Terms	35

5.2 Navigation	36

6 APB GPIO - References	37

6.1 References	37

6.1.1 Internal Documentation	37

6.1.2 External References	37

6.1.3 Design References	37

6.2 Navigation	39

7 APB GPIO - Block Descriptions Overview	40

7.1 Overview	40

7.1.1 Figure 2.1: APB GPIO Module Hierarchy	40

7.1.2 Block Summary	40

7.1.3 In This Chapter	40

7.2 Navigation	42

8 APB GPIO - APB Interface Block	43

8.1 Overview	43

8.1.1 Block Diagram	43

8.2 Ports	44

8.2.1 APB Slave Interface	44

8.3 Functional Description	45

8.3.1 Read Transaction	45

8.3.2 Write Transaction	45

8.4 Timing	46

8.4.1 Figure 2.2: APB Access Timing	46

8.5 Design Notes	47

8.6 Navigation	48

9 APB GPIO - Register File Block	49

9.1 Overview	49

9.1.1 Block Diagram	49

9.2 Ports	50

9.2.1 Hardware-to-Software (hw2reg)	50

9.2.2 Software-to-Hardware (reg2hw)	50

9.3 Functional Description	52

9.3.1 Byte Enable Support	52

9.3.2 Access Types	52

9.4 Design Notes	53

9.5 Navigation	54

10 APB GPIO - GPIO Core Block	55

10.1 Overview	55

10.1.1 Block Diagram	55

10.2 Ports	56

10.3 Functional Description	57

10.3.1 Input Path	57

10.3.2 Output Path	58

10.3.3 Direction Control	58

10.4 Design Notes	59

10.5 Navigation	60

11 APB GPIO - Interrupt Controller Block	61

11.1 Overview	61

11.1.1 Figure 2.5: Interrupt Controller Block Diagram	61

11.2 Functional Description	62

11.2.1 Interrupt Modes	62

11.2.2 Edge Detection Logic	63

11.2.3 Interrupt Status	63

11.2.4 Interrupt Enable	64

11.2.5 Aggregate IRQ Output	64

11.2.6 Interrupt Handling Flow	64

11.3 Design Notes	65

11.4 Navigation	66

12 APB GPIO - CDC Logic Block	67

12.1 Overview	67

12.1.1 Figure 2.7: CDC Logic Block Diagram	67

12.2 Parameters	68

12.2.1 CDC_ENABLE	68

12.3 Functional Description	69

12.3.1 Clock Domains	69

12.3.2 CDC Implementation	70

12.3.3 The irq Output	70

12.3.4 Coherency	71

12.3.5 Reset Synchronization	71

12.4 Timing	72

12.4.1 Latency	72

12.5 Navigation	73

13 APB GPIO - Interfaces Overview	74

13.1 Overview	74

13.1.1 Interface Summary Diagram	74

13.1.2 In This Chapter	75

13.2 Navigation	76

14 APB GPIO - APB Slave Interface	77

14.1 Ports	77

14.1.1 APB Slave Signals	77

14.2 Functional Description	78

14.2.1 Protocol Compliance	78

14.2.2 Address Decoding	78

14.2.3 Byte Strobes	79

14.2.4 Error Handling	79

14.3 Timing	80

14.3.1 Access Timing	80

14.3.2 Figure 3.1: APB Access Timing (CDC_ENABLE=0)	80

14.4 Navigation	81

15 APB GPIO - GPIO Pin Interface	82

15.1 Ports	82

15.1.1 GPIO Signals	82

15.2 Functional Description	83

15.2.1 Output Mode (direction[i] = 1)	83

15.2.2 Input Mode (direction[i] = 0)	83

15.2.3 Input Synchronization	84

15.2.4 Electrical Considerations	84

15.3 Timing	86

15.3.1 Output Path	86

15.3.2 Input Path	86

15.4 Navigation	87

16 APB GPIO - Interrupt Interface	88

16.1 Ports	88

16.2 Functional Description	89

16.2.1 Interrupt Generation	89

16.2.2 Per-Pin Configuration	89

16.2.3 Interrupt Modes	89

16.2.4 Interrupt Handling	91

16.3 Timing	92

16.3.1 Edge-Triggered Latency	92

16.3.2 Level-Sensitive Latency	93

16.4 Design Notes	94

16.4.1 Connection Guidelines	94

16.5 Navigation	95

17 APB GPIO - System Interface	96

17.1 Ports	96

17.1.1 pclk - APB Clock	96

17.1.2 gpio_clk - GPIO Clock	96

17.1.3 presetn - APB Reset	96

17.1.4 gpio_rstn - GPIO Reset	96

17.2 Functional Description	98

17.2.1 Clock Configurations	98

17.2.2 Reset Sequence	99

17.2.3 Figure 3.2: Power-On Reset Sequence	100

17.3 Timing	101

17.3.1 Clock Relationship	101

17.3.2 Reset Recovery	101

17.4 Navigation	102

18 APB GPIO - Programming Model Overview	103

18.1 Overview	103

18.1.1 In This Chapter	104

18.2 Navigation	105

19 APB GPIO - Basic Operations	106

19.1 Waveforms	106

19.1.1 Direction Configuration	106

19.1.2 Output Write	106

19.1.3 Input Read	107

19.1.4 Input Synchronization	108

19.1.5 Atomic Operations	109

19.2 Usage Example	110

19.2.1 Initialization	110

19.2.2 Output Operations	110

19.2.3 Input Operations	111

19.2.4 Mixed I/O Configuration	111

19.2.5 Output Enable Behavior	111

19.3 Navigation	113

20 APB GPIO - Interrupt Configuration	114

20.1 Waveforms	114

20.1.1 Rising Edge Interrupt	114

20.1.2 Falling Edge Interrupt	114

20.1.3 Level-Sensitive Interrupt	115

20.1.4 Interrupt Clear (W1C)	115

20.2 Functional Description	117

20.2.1 Interrupt Configuration Table	117

20.3 Usage Example	118

20.3.1 Interrupt Setup	118

20.3.2 Complete Setup Examples	118

20.3.3 Interrupt Handling	119

20.3.4 Level-Sensitive Considerations	120

20.4 Navigation	121

21 APB GPIO - Programming Examples	122

21.1 Usage Example	122

21.1.1 LED Control	122

21.1.2 Button Input	123

21.1.3 DIP Switch Reading	124

21.1.4 Parallel Data Interface	124

21.1.5 PWM-Style Output	125

21.1.6 Wake-On-Change	125

21.2 Navigation	127

22 APB GPIO - Software Considerations	128

22.1 Design Notes	128

22.1.1 Performance	128

22.1.2 Synchronization	129

22.1.3 Interrupt Best Practices	129

22.1.4 Error Handling	130

22.1.5 Debug Tips	130

22.2 Navigation	132

23 APB GPIO - Register Map	133

23.1 Overview	133

23.2 GPIO_CONTROL (0x000)	135

23.3 GPIO_DIRECTION (0x004)	136

23.4 GPIO_OUTPUT (0x008)	137

23.5 GPIO_INPUT (0x00C)	138

23.6 GPIO_INT_ENABLE (0x010)	139

23.7 GPIO_INT_TYPE (0x014)	140

23.8 GPIO_INT_POLARITY (0x018)	141

23.9 GPIO_INT_BOTH (0x01C)	142

23.10 GPIO_INT_STATUS (0x020)	143

23.11 GPIO_RAW_INT (0x024)	144

23.12 GPIO_OUTPUT_SET (0x028)	145

23.13 GPIO_OUTPUT_CLR (0x02C)	146

23.14 GPIO_OUTPUT_TGL (0x030)	147

23.15 Design Notes	148

23.15.1 Atomic-Register Write Semantics (SET/CLR/TGL)	148

23.15.2 Address Calculation	148

23.16 Navigation	150

List of Figures

Figure 1.1: APB GPIO Block Diagram	17

Figure 1.2: APB GPIO Top-Level Architecture	21

Figure 1.3: APB GPIO Module Hierarchy	21

Figure 1.4: Write Transaction Flow	22

Figure 1.5: Read Transaction Flow	23

Figure 1.6: Interrupt Flow	23

Figure 1.7: Synchronous Mode Clock Domains	24

Figure 1.8: Asynchronous Mode Clock Domains	25

Figure 2.1: APB GPIO Module Hierarchy	40

Figure 2.2: APB Access Timing	46

Figure 2.5: Interrupt Controller Block Diagram	61

Figure 2.6: Edge Detection Logic	63

Figure 2.7: CDC Logic Block Diagram	67

Figure 2.8: Single Clock Domain (CDC Disabled)	69

Figure 2.9: Dual Clock Domain (CDC Enabled)	70

Figure 3.1: APB Access Timing (CDC_ENABLE=0)	80

Figure 3.2: Power-On Reset Sequence	100

List of Tables

Table 1: Table 1.1: GPIO Parameters	26

Table 2: Table 1.2: Resource Estimates	27

Table 3: Table 2.1: Block Summary	40

Table 4: Table 2.5: Edge-Triggered Modes	62

Table 5: Table 2.6: Level-Sensitive Modes	62

Table 6: Table 2.7: CDC Enable Parameter	68

Table 7: Table 2.8: CDC Latency	72

List of Waveforms

No waveforms in this document.

# 1	Gpio Mas Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	APB GPIO - Overview

## 2.1	Overview

The APB GPIO controller is a 32-bit general-purpose I/O block on the APB bus. Software gets per-bit direction control, per-bit interrupt configuration, and atomic output updates; the system gets a single interrupt line and a clean tri-state interface to the pads. Nothing exotic — which is exactly what you want from a GPIO.

### 2.1.1	Features

#### 2.1.1.1	Core Functionality

- 32-bit bidirectional GPIO port

- Per-bit direction control (input/output)

- Per-bit output enable control

- Input synchronization for metastability protection

#### 2.1.1.2	Interrupt Capabilities

- Per-bit interrupt enable

- Edge-triggered interrupts (rising, falling, or both)

- Level-triggered interrupts (high or low)

- Combined interrupt output (OR of all enabled sources)

- Write-1-to-clear interrupt status

#### 2.1.1.3	Atomic Operations

- Atomic set (OR with mask)

- Atomic clear (AND with inverted mask)

- Atomic toggle (XOR with mask)

- No read-modify-write race conditions

#### 2.1.1.4	Clock Domain Crossing

- Optional CDC support via CDC_ENABLE parameter

- Separate GPIO clock domain for async I/O

- Multi-stage input synchronization

### 2.1.2	Applications

#### 2.1.2.1	Typical Use Cases

- LED control and status indication

- Push-button and switch inputs

- External device reset control

- Interrupt generation from external events

- Bit-banged serial protocols (I2C, SPI fallback)

- Debug signals and test points

#### 2.1.2.2	System Integration

- Memory-mapped APB peripheral

- Single interrupt line to CPU/interrupt controller

- Direct connection to FPGA I/O pads via IOBUFs

- Compatible with standard GPIO software drivers

### 2.1.3	Figure 1.1: APB GPIO Block Diagram

APB GPIO Block Diagram

## 2.2	Parameters

The headline numbers. GPIO width and synchronizer depth are parameters, so treat the 32-bit default as exactly that — a default.

| Parameter | Value |
| --- | --- |
| GPIO Width | 32 bits (configurable) |
| APB Data Width | 32 bits |
| APB Address Width | 12 bits (4KB) |
| Sync Stages | 2 (configurable) |
| CDC Support | Optional |

### 2.2.1	Register Summary

The full map with field-level detail is in Chapter 5. Here’s the quick reference:

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW | 0x00000001 | Global enable and interrupt enable |
| 0x004 | GPIO_DIRECTION | RW | 0x00000000 | Per-bit direction (1=output) |
| 0x008 | GPIO_OUTPUT | RW | 0x00000000 | Output data value |
| 0x00C | GPIO_INPUT | RO | - | Input data (read-only) |
| 0x010 | GPIO_INT_ENABLE | RW | 0x00000000 | Per-bit interrupt enable |
| 0x014 | GPIO_INT_TYPE | RW | 0x00000000 | Interrupt type (1=level, 0=edge) |
| 0x018 | GPIO_INT_POLARITY | RW | 0xFFFFFFFF | Polarity (1=high/rising) |
| 0x01C | GPIO_INT_BOTH | RW | 0x00000000 | Both-edge enable |
| 0x020 | GPIO_INT_STATUS | W1C | 0x00000000 | Interrupt status (W1C) |
| 0x024 | GPIO_RAW_INT | RO | 0 (live) | Raw interrupt (pre-mask) |
| 0x028 | GPIO_OUTPUT_SET | WO | 0x00000000 | Atomic set |
| 0x02C | GPIO_OUTPUT_CLR | WO | 0x00000000 | Atomic clear |
| 0x030 | GPIO_OUTPUT_TGL | WO | 0x00000000 | Atomic toggle |

## 2.3	Navigation

Next: 02_architecture.md - High-level architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	APB GPIO - Architecture

## 3.1	Overview

Two views of the same design: the top-level block diagram, and the module hierarchy underneath it.

### 3.1.1	Figure 1.2: APB GPIO Top-Level Architecture

APB GPIO Architecture

### 3.1.2	Figure 1.3: APB GPIO Module Hierarchy

apb4_gpio                      top level
├── apb4_slave                 APB completer (CDC_ENABLE=0)
│   or apb4_slave_cdc          APB completer + crossing (CDC_ENABLE=1)
├── peakrdl_to_cmdrsp          cmd/rsp to register-block adapter
├── gpio_config_regs           write strobes, W1C merge, irq
│   ├── gpio_regs              PeakRDL register block
│   └── gpio_core              sync, output latch, edge/level
└── glitch_free_n_dff_arn      irq sync to pclk (CDC_ENABLE=1)

## 3.2	Functional Description

### 3.2.1	Data Flow

Three flows matter: APB writes, APB reads, and interrupts.

#### 3.2.1.1	Figure 1.4: Write Transaction Flow

Write Transaction Flow

#### 3.2.1.2	Figure 1.5: Read Transaction Flow

Read Transaction Flow

#### 3.2.1.3	Figure 1.6: Interrupt Flow

Interrupt Flow

### 3.2.2	Clock Domains

#### 3.2.2.1	Figure 1.7: Synchronous Mode Clock Domains

Synchronous Mode

With CDC_ENABLE = 0, everything runs on the APB clock (pclk). Input synchronization stays active on the external GPIO pins — metastability doesn’t care that your bus fabric is synchronous.

#### 3.2.2.2	Figure 1.8: Asynchronous Mode Clock Domains

Asynchronous Mode

With CDC_ENABLE = 1, the APB clock domain handles protocol conversion while the GPIO clock domain owns the registers and I/O. The crossing is the pair of asynchronous FIFOs inside apb4_slave_cdc (command in, response out), plus a 2-flop synchronizer that brings irq back to pclk.

## 3.3	Parameters

Table 1.1: GPIO Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| GPIO_WIDTH | int | 32 | Number of GPIO pins |
| SYNC_STAGES | int | 2 | Input synchronizer stages |
| CDC_ENABLE | int | 0 | Enable clock domain crossing |
| SKID_DEPTH | int | 2 | CDC skid buffer depth |
| USE_JOHNSON | int | 0 | CDC counter encoding (forwarded to apb4_slave_cdc) |

## 3.4	Design Notes

### 3.4.1	Resource Estimates

Rough synthesis numbers for planning purposes:

Table 1.2: Resource Estimates

| Component | Flip-Flops | LUTs |
| --- | --- | --- |
| gpio_core | ~200 | ~300 |
| gpio_regs | ~400 | ~200 |
| gpio_config_regs | ~50 | ~100 |
| apb4_slave (no CDC) | ~20 | ~50 |
| apb4_slave_cdc | ~100 | ~150 |
| Total (no CDC) | ~670 | ~650 |
| Total (with CDC) | ~750 | ~750 |

## 3.5	Navigation

Next: 03_clocks_and_reset.md - Clock and reset behavior

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	APB GPIO - Clocks and Reset

## 4.1	Overview

Two clocks, two resets, and one synchronizer chain you can’t turn off. Here’s what each piece does.

### 4.1.1	Clock Signals

#### 4.1.1.1	pclk (APB Clock)

- Purpose: Primary APB bus clock

- Usage: APB protocol, register access

- Typical Frequency: 50-200 MHz

#### 4.1.1.2	gpio_clk (GPIO Clock)

- Purpose: Optional separate GPIO clock domain

- Usage: Only when CDC_ENABLE=1

- Relationship: Can be asynchronous to pclk

### 4.1.2	Reset Signals

#### 4.1.2.1	presetn (APB Reset)

- Type: Active-low asynchronous reset (exception: the PeakRDL-generated register file resets SYNCHRONOUSLY – see the note in ch03_interfaces/04_system.md)

- Scope: APB interface logic

- Behavior: Resets APB state machine, clears pending transactions

#### 4.1.2.2	gpio_rstn (GPIO Reset)

- Type: Active-low asynchronous reset

- Scope: GPIO core logic

- Usage: Only when CDC_ENABLE=1

- Behavior: Resets GPIO outputs, interrupt state

## 4.2	Functional Description

### 4.2.1	Reset Behavior

#### 4.2.1.1	Register Reset Values

| Register | Reset Value | Notes |
| --- | --- | --- |
| GPIO_CONTROL | 0x00000001 | GPIO enabled; global interrupt enable clear |
| GPIO_DIRECTION | 0x00000000 | All inputs |
| GPIO_OUTPUT | 0x00000000 | Outputs low |
| GPIO_INT_ENABLE | 0x00000000 | No interrupts |
| GPIO_INT_TYPE | 0x00000000 | Edge mode |
| GPIO_INT_POLARITY | 0xFFFFFFFF | Rising/high |
| GPIO_INT_BOTH | 0x00000000 | Single edge |
| GPIO_INT_STATUS | 0x00000000 | No pending |
| GPIO_RAW_INT | 0 (live RO readback; depends on pins after reset) | No events |
| GPIO_OUTPUT_SET/CLR/TGL | 0x00000000 | No pending atomic op |

GPIO_CONTROL.ENABLE resets to 1, but don’t let that fool you into thinking pins drive out of reset: GPIO_DIRECTION resets to 0, so every pin is an input and high-Z until software programs directions. The irq output stays deasserted until software sets GPIO_CONTROL.INT_ENABLE (bit 1), which resets to 0.

#### 4.2.1.2	Output Pin Behavior During Reset

During reset: - gpio_out[31:0] = 0 - gpio_oe[31:0] = 0 (all high-Z) - irq = 0

### 4.2.2	Clock Domain Crossing

#### 4.2.2.1	When CDC_ENABLE = 0

- All logic runs on pclk

- gpio_clk input is ignored

- Connect gpio_clk = pclk for clean design

#### 4.2.2.2	When CDC_ENABLE = 1

- APB interface uses pclk

- GPIO core uses gpio_clk

- apb4_slave_cdc’s asynchronous command and response FIFOs carry the APB traffic across; irq returns to pclk through a 2-flop synchronizer

- Both resets must be asserted together at power-on

### 4.2.3	Input Synchronization

GPIO inputs are always synchronized, CDC setting or not:

Input Synchronizer

- SYNC_STAGES parameter controls depth (default: 2)

- Prevents metastability from external signal transitions

- Adds latency equal to SYNC_STAGES clock cycles

## 4.3	Timing

### 4.3.1	Timing Constraints

#### 4.3.1.1	Synchronous Mode

- Standard single-clock timing

- All paths constrained to pclk

#### 4.3.1.2	Asynchronous Mode

- Set false_path between pclk and gpio_clk domains

- Set max_delay for CDC paths

- Synchronizer FFs should have ASYNC_REG attribute

## 4.4	Navigation

Next: 04_acronyms.md - Acronyms and terminology

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	APB GPIO - Acronyms and Terminology

## 5.1	Overview

The alphabet soup, collected in one place so the rest of the document doesn’t have to stop and define itself.

### 5.1.1	Protocol Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| APB | Advanced Peripheral Bus | Low-power AMBA bus protocol |
| AMBA | Advanced Microcontroller Bus Architecture | ARM standard bus protocols |
| CDC | Clock Domain Crossing | Synchronization between clock domains |

### 5.1.2	Signal Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| CLK | Clock | Timing reference signal |
| RST | Reset | System initialization signal |
| OE | Output Enable | Tri-state buffer control |
| IRQ | Interrupt Request | Hardware interrupt signal |

### 5.1.3	GPIO-Specific Terms

| Term | Description |
| --- | --- |
| GPIO | General Purpose Input/Output - Configurable digital I/O pins |
| Pin | Individual GPIO signal (input or output) |
| Port | Group of 32 GPIO pins managed together |
| Direction | Input (0) or Output (1) configuration per pin |
| Polarity | Active-high or active-low signal interpretation |

### 5.1.4	Interrupt Terms

| Term | Description |
| --- | --- |
| Edge-triggered | Interrupt on signal transition |
| Level-sensitive | Interrupt while signal is at specified level |
| Rising edge | Low-to-high transition |
| Falling edge | High-to-low transition |
| Both edges | Either transition direction |

### 5.1.5	Register Terms

| Term | Description |
| --- | --- |
| RW | Read-Write register |
| RO | Read-Only register |
| W1C | Write-1-to-Clear register |
| HWIF | Hardware Interface (PeakRDL generated) |

## 5.2	Navigation

Next: 05_references.md - Reference documents

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	APB GPIO - References

## 6.1	References

### 6.1.1	Internal Documentation

#### 6.1.1.1	RTL Source Files

Worth having open in an editor while you read the rest of this MAS:

- rtl/gpio/apb4_gpio.sv - Top-level module (APB port, CDC generate)

- rtl/gpio/gpio_config_regs.sv - Register wrapper (atomic-op pulses, irq)

- rtl/gpio/gpio_core.sv - Pin I/O, synchronizers, interrupt detection

- rtl/gpio/gpio_regs.sv / gpio_regs_pkg.sv - PeakRDL-generated register file

- rdl/gpio/gpio_regs.rdl - Register description source

#### 6.1.1.2	Related Specifications

- APB Protocol Specification (AMBA 3)

- RLB Integration Guide

### 6.1.2	External References

#### 6.1.2.1	ARM AMBA Specifications

- AMBA 3 APB Protocol Specification

  - ARM IHI 0024E

  - Defines APB interface timing and protocol

#### 6.1.2.2	Industry Standards

- GPIO Best Practices

  - Synchronization for external inputs

  - Interrupt handling patterns

  - Tri-state buffer management

### 6.1.3	Design References

#### 6.1.3.1	Clock Domain Crossing

- Dual flip-flop synchronizer methodology

- Skid buffer for data path CDC

- Reset synchronization techniques

#### 6.1.3.2	Interrupt Handling

- Edge detection circuits

- Interrupt aggregation patterns

- Software interrupt acknowledge flows

## 6.2	Navigation

Next: Chapter 2: Block Descriptions

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	APB GPIO - Block Descriptions Overview

## 7.1	Overview

The design splits into four blocks plus an optional CDC wrapper. Small enough to hold in your head, which is how a GPIO should be.

### 7.1.1	Figure 2.1: APB GPIO Module Hierarchy

apb4_gpio                      top level
├── apb4_slave                 APB completer (CDC_ENABLE=0)
│   or apb4_slave_cdc          APB completer + crossing (CDC_ENABLE=1)
├── peakrdl_to_cmdrsp          cmd/rsp to register-block adapter
├── gpio_config_regs           write strobes, W1C merge, irq
│   ├── gpio_regs              PeakRDL register block
│   └── gpio_core              sync, output latch, edge/level
└── glitch_free_n_dff_arn      irq sync to pclk (CDC_ENABLE=1)

### 7.1.2	Block Summary

Table 2.1: Block Summary

| Block | File | Description |
| --- | --- | --- |
| APB GPIO Top | apb4_gpio.sv | Top-level module with all GPIO functionality |
| Register File | gpio_regs.sv (+ gpio_regs_pkg.sv) | PeakRDL-generated control/status registers |
| Register Wrapper | gpio_config_regs.sv | Connects registers to the core; atomic-op pulses, irq |
| GPIO Core | gpio_core.sv | Pin I/O, synchronizers, interrupt detection |

### 7.1.3	In This Chapter

#### 7.1.3.1	1. APB Interface

Handles APB protocol conversion and register access.

See: 01_apb_interface.md

#### 7.1.3.2	2. Register File

PeakRDL-generated registers for configuration and status.

See: 02_register_file.md

#### 7.1.3.3	3. GPIO Core

Main GPIO functionality including I/O control and interrupts.

See: 03_gpio_core.md

#### 7.1.3.4	4. Interrupt Controller

Edge detection, level sensing, and interrupt aggregation.

See: 04_interrupt_controller.md

#### 7.1.3.5	5. CDC Logic

Optional clock domain crossing for asynchronous GPIO clock.

See: 05_cdc_logic.md

## 7.2	Navigation

Next: 01_apb_interface.md - APB Interface details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	APB GPIO - APB Interface Block

## 8.1	Overview

The APB interface is the bridge between the system APB bus and the GPIO register file — a small FSM that turns APB’s two-phase transfers into the command/response handshake the register block speaks.

### 8.1.1	Block Diagram

APB Interface Block

## 8.2	Ports

### 8.2.1	APB Slave Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_apb_PSEL | 1 | Input | Slave select |
| s_apb_PENABLE | 1 | Input | Enable phase |
| s_apb_PWRITE | 1 | Input | Write operation |
| s_apb_PADDR | 12 | Input | Address bus |
| s_apb_PWDATA | 32 | Input | Write data |
| s_apb_PSTRB | 4 | Input | Byte strobes |
| s_apb_PPROT | 3 | Input | Protection attributes (accepted, unused) |
| s_apb_PRDATA | 32 | Output | Read data |
| s_apb_PREADY | 1 | Output | Ready response |
| s_apb_PSLVERR | 1 | Output | Error response |

## 8.3	Functional Description

### 8.3.1	Read Transaction

- Master asserts psel and paddr

- Master asserts penable on next cycle

- Slave returns prdata with pready

### 8.3.2	Write Transaction

- Master asserts psel, paddr, pwdata, pwrite

- Master asserts penable on next cycle

- Slave samples data with pready

## 8.4	Timing

Figure 2.2 shows a basic APB access, start to finish.

### 8.4.1	Figure 2.2: APB Access Timing

APB Access Timing

One SETUP cycle, then six ACCESS cycles: the register itself is read or written in cycle 3, and PREADY rises with PRDATA in the sixth ACCESS cycle.

## 8.5	Design Notes

- PREADY-gated: the bridge FSM adds five wait states per access with CDC_ENABLE=0, more with the CDC (no stalls originate in the register block itself)

- No error responses (pslverr always 0)

- 32-bit aligned access only

## 8.6	Navigation

Next: 02_register_file.md - Register File

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	APB GPIO - Register File Block

## 9.1	Overview

The register file is generated by PeakRDL from ../../rdl/gpio/gpio_regs.rdl and is the hardware/software contract for the whole block. If a bit isn’t in the RDL, it doesn’t exist.

### 9.1.1	Block Diagram

Register File Block

## 9.2	Ports

### 9.2.1	Hardware-to-Software (hw2reg)

Signals the RTL drives into the register file:

| Signal | Width | Description |
| --- | --- | --- |
| GPIO_INPUT.input_data | 32 | Current GPIO input values |
| GPIO_OUTPUT.output_data | 32 | Live output latch write-back (we-gated, the cycle after an atomic op) |
| GPIO_INT_STATUS.int_status | 32 | Interrupt status set requests (sticky; includes the deferred W1C-coincident set) |
| GPIO_RAW_INT.raw_status | 32 | Live raw event-detector status |

### 9.2.2	Software-to-Hardware (reg2hw)

Signals the register file drives out to the RTL:

| Signal | Width | Description |
| --- | --- | --- |
| GPIO_CONTROL.gpio_enable | 1 | GPIO output enable (gates gpio_oe only; resets 1) |
| GPIO_CONTROL.int_enable | 1 | Global interrupt enable (gates irq; resets 0) |
| GPIO_DIRECTION.direction | 32 | Pin direction (0=in, 1=out) |
| GPIO_OUTPUT.output_data | 32 | Output values (+ swmod write strobe) |
| GPIO_INT_ENABLE.int_enable | 32 | Interrupt enable per pin |
| GPIO_INT_TYPE.int_type | 32 | Interrupt type (0=edge, 1=level) |
| GPIO_INT_POLARITY.int_polarity | 32 | Polarity (0=fall/low, 1=rise/high) |
| GPIO_INT_BOTH.int_both | 32 | Both edges enable |
| GPIO_OUTPUT_SET.set_bits | 32 | Atomic set mask (+ swmod write strobe) |
| GPIO_OUTPUT_CLR.clear_bits | 32 | Atomic clear mask (+ swmod write strobe) |
| GPIO_OUTPUT_TGL.toggle_bits | 32 | Atomic toggle mask (+ swmod write strobe) |
| GPIO_INT_STATUS.int_status | 32 | Sticky status value (+ swmod W1C write strobe) |

## 9.3	Functional Description

### 9.3.1	Byte Enable Support

All registers support byte-granular writes via pstrb: - pstrb[0] enables bits [7:0] - pstrb[1] enables bits [15:8] - pstrb[2] enables bits [23:16] - pstrb[3] enables bits [31:24]

### 9.3.2	Access Types

| Type | Read Behavior | Write Behavior |
| --- | --- | --- |
| RW | Returns current value | Updates register |
| RO | Returns hardware value | No effect |
| W1C | Returns current value | Clears bits where 1 written |

## 9.4	Design Notes

- Generated by PeakRDL regblock

- Clocked by the core clock (w_core_clk = gpio_clk when CDC_ENABLE=1, pclk otherwise – see the CDC chapter)

- Registers reset to 0 except GPIO_CONTROL.GPIO_ENABLE (resets to 1) and GPIO_INT_POLARITY (resets to 0xFFFFFFFF); reset is synchronous inside the generated block

- swmod on GPIO_OUTPUT, GPIO_OUTPUT_SET/CLR/TGL and GPIO_INT_STATUS is a level held for the whole command-bridge transaction and leads the field storage by one cycle; gpio_config_regs edge-detects and re-aligns it before use, so every write is one operation (see Chapter 5)

## 9.5	Navigation

Next: 03_gpio_core.md - GPIO Core

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	APB GPIO - GPIO Core Block

## 10.1	Overview

The core is where the pins live: input synchronization, output driving, and direction control for all 32 GPIOs.

### 10.1.1	Block Diagram

GPIO Core Block

## 10.2	Ports

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| gpio_out | 32 | Output | Output data values |
| gpio_oe | 32 | Output | Output enables (active high) |
| gpio_in | 32 | Input | Input data values |

## 10.3	Functional Description

### 10.3.1	Input Path

#### 10.3.1.1	Synchronization

External inputs pass through a dual flip-flop synchronizer:

Input Synchronizer

- Prevents metastability from asynchronous inputs

- Configurable depth via SYNC_STAGES parameter

- Adds SYNC_STAGES clock cycles of latency

#### 10.3.1.2	Input Register

Synchronized inputs land in the GPIO_INPUT register, which is what software reads.

### 10.3.2	Output Path

#### 10.3.2.1	Output Register

Software writes to GPIO_OUTPUT to set output values. The write reaches the core as a one-cycle strobe (cfg_output_wr_stb, built in gpio_config_regs from the register’s write strobe) alongside the data, so it is a write whether or not the value changed. A direct write takes priority over an atomic operation in the same cycle (direct > toggle > set > clear):

if (cfg_output_wr_stb)        r_output_data <= cfg_output_data;
else                          // toggle, then set, then clear

r_output_data drives the pins and is written back into GPIO_OUTPUT after each atomic operation, so the register reads the live value.

#### 10.3.2.2	Output Enable

The direction register controls the tri-state buffers: - direction[i] = 0: Pin is input (high-Z output) - direction[i] = 1: Pin is output (driven)

### 10.3.3	Direction Control

#### 10.3.3.1	Per-Pin Configuration

Each pin is configured independently:

// gpio_oe = cfg_gpio_enable ? cfg_direction : '0;   (gpio_core.sv)
if (gpio_enable && direction[i]) begin
    // Output mode
    gpio_oe[i] = 1'b1;
    gpio_out[i] = output_reg[i];
end else begin
    // Input mode, or GPIO_CONTROL.ENABLE == 0 (all pins high-Z)
    gpio_oe[i] = 1'b0;
    // gpio_out[i] = don't care
end

GPIO_CONTROL.ENABLE gates ONLY the output enables: when it is 0 every pin is high-Z, but output data, input synchronization, and interrupt detection/status all keep running.

#### 10.3.3.2	Read-Back Behavior

Reading GPIO_INPUT returns: - For input pins: External signal value (synchronized) - For output pins: External signal value (may differ from output_reg if open-drain)

## 10.4	Design Notes

- All 32 pins processed in parallel

- Output updates one core-clock after the GPIO_OUTPUT field updates (the write strobe is aligned to the field, and r_output_data registers it)

- The core exports sts_raw_int and sts_int_pending only; the aggregate irq is built in gpio_config_regs from the sticky status register and the live level pins. The core has no irq output and no global-enable input

- Input synchronization always active

## 10.5	Navigation

Next: 04_interrupt_controller.md - Interrupt Controller

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	APB GPIO - Interrupt Controller Block

## 11.1	Overview

Per-pin interrupt generation with edge or level triggering and one aggregate irq. The logic is simple; the enable semantics are where people trip. There are two levels of enable and they don’t mean the same thing — read that section twice.

### 11.1.1	Figure 2.5: Interrupt Controller Block Diagram

Interrupt Controller Block

## 11.2	Functional Description

### 11.2.1	Interrupt Modes

#### 11.2.1.1	Edge-Triggered Mode

GPIO_INT_TYPE[i] = 0

Table 2.5: Edge-Triggered Modes

| GPIO_INT_POLARITY | GPIO_INT_BOTH | Trigger Condition |
| --- | --- | --- |
| 0 | 0 | Falling edge only |
| 1 | 0 | Rising edge only |
| X | 1 | Both edges |

#### 11.2.1.2	Level-Sensitive Mode

GPIO_INT_TYPE[i] = 1

Table 2.6: Level-Sensitive Modes

| GPIO_INT_POLARITY | Trigger Condition |
| --- | --- |
| 0 | Active low (interrupt while pin = 0) |
| 1 | Active high (interrupt while pin = 1) |

### 11.2.2	Edge Detection Logic

#### 11.2.2.1	Figure 2.6: Edge Detection Logic

Edge Detection Logic

### 11.2.3	Interrupt Status

- Each bit in GPIO_INT_STATUS corresponds to one pin

- Set when the interrupt condition is detected on a pin whose GPIO_INT_ENABLE bit is 1 - events on disabled pins set nothing (sts_int_pending = raw & enable in gpio_core.sv)

- Cleared by writing 1 to the bit (W1C); bits latch in both edge and level modes until cleared

- A W1C write and a hardware event in the same cycle merge per bit (next = (value | hw_set) & ~w1c_mask, gpio_config_regs.sv): the event survives on every bit the write does not clear

### 11.2.4	Interrupt Enable

Two levels of enable exist:

- GPIO_INT_ENABLE[i] = 1 enables pin i. It is sampled at EVENT time: it gates whether a status bit sets, and (for level pins) whether the pin drives irq. It is also re-applied at irq: clearing it after an edge event has latched masks that pending interrupt (the status bit stays set until W1C), as described under Aggregate IRQ Output below.

- GPIO_CONTROL[1] (global INT_ENABLE) gates the final irq output and is the only control that masks everything. It resets to 0.

### 11.2.5	Aggregate IRQ Output

per-pin effective status:
  level pin (TYPE=1): raw_detector & GPIO_INT_ENABLE   (live, bypasses STATUS)
  edge  pin (TYPE=0): GPIO_INT_STATUS & GPIO_INT_ENABLE (sticky latch, re-gated)

irq = GPIO_CONTROL[1] && (| effective_status)

(gpio_config_regs.sv.) One difference from a plain |(STATUS & ENABLE) remains: level pins bypass GPIO_INT_STATUS entirely, so W1C on a level pin does not deassert irq while the level persists. Edge pins re-apply the current per-pin enable, so clearing GPIO_INT_ENABLE[i] masks an already-latched edge interrupt on pin i (the status bit stays set until W1C and irq re-asserts if the enable is set again).

### 11.2.6	Interrupt Handling Flow

- Hardware detects condition, sets status bit

- IRQ asserted to processor

- Software reads GPIO_INT_STATUS to identify source

- Software handles interrupt

- Software writes 1 to status bit to clear

- IRQ deasserts (if no other sources active)

## 11.3	Design Notes

- Edge detection uses synchronized input

- Level-sensitive interrupts re-trigger if not cleared

- Status bits latch until software clears

## 11.4	Navigation

Next: 05_cdc_logic.md - CDC Logic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	APB GPIO - CDC Logic Block

## 12.1	Overview

When you need the GPIO core on its own clock, asynchronous to the APB bus, CDC_ENABLE moves the whole register file across the domain boundary. Everything about the crossing lives in this one block.

### 12.1.1	Figure 2.7: CDC Logic Block Diagram

CDC Logic Block

## 12.2	Parameters

### 12.2.1	CDC_ENABLE

Table 2.7: CDC Enable Parameter

| Value | Behavior |
| --- | --- |
| 0 | Single clock domain, all logic on pclk |
| 1 | Dual clock domain, GPIO core on gpio_clk |

## 12.3	Functional Description

### 12.3.1	Clock Domains

#### 12.3.1.1	Figure 2.8: Single Clock Domain (CDC Disabled)

CDC Disabled

#### 12.3.1.2	Figure 2.9: Dual Clock Domain (CDC Enabled)

CDC Enabled

### 12.3.2	CDC Implementation

When CDC_ENABLE=1 the ENTIRE register file and GPIO core sit in the gpio_clk domain; no register value is individually synchronized. The only crossing is the APB command/response stream itself, carried through an apb4_slave_cdc instance at the bus boundary (apb4_gpio.sv, gen_cdc). An APB access therefore completes with CDC handshake latency, and once it lands in the gpio_clk domain every register behaves exactly as in the single-clock configuration.

### 12.3.3	The irq Output

irq is generated in the core clock domain (gpio_config_regs.sv), which is gpio_clk when CDC_ENABLE=1. apb4_gpio.sv (gen_irq_sync) then passes it through a 2-flop synchronizer (glitch_free_n_dff_arn, FLOP_COUNT=2) clocked by pclk, so the output pin is safe to consume in the pclk domain with no external synchronizer. It is a level, so a plain synchronizer is the right primitive; there is no handshake and no pulse stretching. The cost is two pclk cycles of latency. When CDC_ENABLE=0 the synchronizer is bypassed (gen_irq_direct) and irq keeps the zero-latency behaviour of the single-clock configuration.

One contract comes with that choice. For an edge-mode pin the sticky status bit holds irq asserted until software clears it, so the synchronizer always sees it. For a level-mode pin irq is only as wide as the input level itself, and a level that de-asserts within about two pclk periods (plus the input synchronizer depth) can pass between the synchronizer’s samples and never reach the pin. The event is not lost – GPIO_INT_STATUS still latches it and polling sees it – but no interrupt fires. Hold level-mode inputs for at least two pclk periods when CDC_ENABLE=1, or use edge mode for short pulses. With CDC_ENABLE=0 every core-clock assertion is visible, so the constraint only exists in the asynchronous configuration.

### 12.3.4	Coherency

- No guaranteed atomicity across clock domains

- Software must handle potential inconsistencies

- Interrupt status always reflects gpio_clk domain

### 12.3.5	Reset Synchronization

Both resets must be asserted at power-on: 1. Assert both presetn and gpio_rstn 2. Release gpio_rstn first 3. Release presetn after gpio_clk domain stable

## 12.4	Timing

### 12.4.1	Latency

Table 2.8: CDC Latency

| Path | Latency |
| --- | --- |
| APB write to gpio_clk-domain register | APB access + apb4_slave_cdc handshake (a few cycles of each clock) |
| APB read of gpio_clk-domain state | Same crossing, in both directions |
| Interrupt detection to IRQ | gpio_clk-domain detection (Chapter 3.3) + 2 pclk cycles through the irq synchronizer |

## 12.5	Navigation

Back to: 00_overview.md - Block Descriptions Overview

Next Chapter: Chapter 3: Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB GPIO - Interfaces Overview

## 13.1	Overview

Four external interfaces: the bus, the pins, the interrupt, and the clock/reset pair.

| Interface | Type | Description |
| --- | --- | --- |
| APB Slave | Bus | Configuration and status access |
| GPIO Pins | I/O | 32 general-purpose I/O pins |
| Interrupt | Signal | Aggregate interrupt output |
| Clocks/Reset | System | Clock and reset inputs |

### 13.1.1	Interface Summary Diagram

GPIO Interfaces

### 13.1.2	In This Chapter

#### 13.1.2.1	APB Slave Interface

Complete APB protocol interface for register access.

See: 01_apb4_slave.md

#### 13.1.2.2	GPIO Pin Interface

External GPIO pin connections with tri-state control.

See: 02_gpio_pins.md

#### 13.1.2.3	Interrupt Interface

Interrupt request output signal.

See: 03_interrupt.md

#### 13.1.2.4	System Interface

Clock and reset signal requirements.

See: 04_system.md

## 13.2	Navigation

Next: 01_apb4_slave.md - APB Slave Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	APB GPIO - APB Slave Interface

## 14.1	Ports

### 14.1.1	APB Slave Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| pclk | 1 | Input | APB clock |
| presetn | 1 | Input | APB reset (active low) |
| s_apb_PSEL | 1 | Input | Peripheral select |
| s_apb_PENABLE | 1 | Input | Enable phase |
| s_apb_PWRITE | 1 | Input | Write transaction |
| s_apb_PADDR | 12 | Input | Address bus |
| s_apb_PWDATA | 32 | Input | Write data |
| s_apb_PSTRB | 4 | Input | Byte strobes |
| s_apb_PRDATA | 32 | Output | Read data |
| s_apb_PREADY | 1 | Output | Ready response |
| s_apb_PSLVERR | 1 | Output | Slave error |

## 14.2	Functional Description

### 14.2.1	Protocol Compliance

APB3/APB4 feature support:

| Feature | Support |
| --- | --- |
| PSEL | Yes |
| PENABLE | Yes |
| PWRITE | Yes |
| PADDR | 12-bit |
| PWDATA | 32-bit |
| PRDATA | 32-bit |
| PREADY | Yes (inserts wait states – see Access Timing below) |
| PSLVERR | Yes (always 0) |
| PSTRB | Yes |
| PPROT | Present on the port (s_apb_PPROT[2:0]), accepted and ignored - no protection checking is performed |

### 14.2.2	Address Decoding

| Offset | Name | Access |
| --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW |
| 0x004 | GPIO_DIRECTION | RW |
| 0x008 | GPIO_OUTPUT | RW |
| 0x00C | GPIO_INPUT | RO |
| 0x010 | GPIO_INT_ENABLE | RW |
| 0x014 | GPIO_INT_TYPE | RW |
| 0x018 | GPIO_INT_POLARITY | RW |
| 0x01C | GPIO_INT_BOTH | RW |
| 0x020 | GPIO_INT_STATUS | W1C |
| 0x024 | GPIO_RAW_INT | RO |
| 0x028 | GPIO_OUTPUT_SET | WO |
| 0x02C | GPIO_OUTPUT_CLR | WO |
| 0x030 | GPIO_OUTPUT_TGL | WO |

Only PADDR[5:0] reaches the register block, so this map aliases every 64 bytes across the 12-bit APB window; 0x034-0x03F read as zero. No address ever raises PSLVERR.

### 14.2.3	Byte Strobes

Byte-granular writes supported: - pstrb[3:0] corresponds to pwdata[31:0] - Unselected bytes retain previous values

### 14.2.4	Error Handling

- No address decode errors (all addresses valid)

- No timeout errors

- pslverr always 0

## 14.3	Timing

### 14.3.1	Access Timing

PREADY inserts wait states: the apb4_slave bridge is a small FSM that issues the command and returns the response over a registered cmd/rsp handshake, so an access takes five pclk wait states beyond the 2-cycle APB minimum (more when CDC_ENABLE=1 crosses to gpio_clk). The register block itself never stalls (cpuif_req_stall_* = 0).

### 14.3.2	Figure 3.1: APB Access Timing (CDC_ENABLE=0)

APB Access Timing

The same access as Figure 2.2: PREADY and PRDATA arrive together in the sixth ACCESS cycle, and PWDATA is held from SETUP until PREADY.

## 14.4	Navigation

Next: 02_gpio_pins.md - GPIO Pin Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	APB GPIO - GPIO Pin Interface

## 15.1	Ports

### 15.1.1	GPIO Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| gpio_out | 32 | Output | Output data values |
| gpio_oe | 32 | Output | Output enables |
| gpio_in | 32 | Input | Input data values |

## 15.2	Functional Description

### 15.2.1	Output Mode (direction[i] = 1)

One GPIO Pin

- gpio_oe[i] = 1 (output enabled)

- gpio_out[i] = GPIO_OUTPUT register value

- Pin driven to output value

### 15.2.2	Input Mode (direction[i] = 0)

The same path as the figure above with gpio_oe[i] = 0: the pad buffer is Hi-Z and only the input leg, through the synchronizer, is live.

- gpio_oe[i] = 0 (tri-state)

- gpio_out[i] = don’t care

- External value captured via synchronizer

### 15.2.3	Input Synchronization

All inputs pass through a dual flip-flop synchronizer:

Input Synchronizer

- SYNC_STAGES parameter controls depth (default: 2)

- Prevents metastability from asynchronous inputs

- All 32 inputs synchronized in parallel

Input changes show up in the GPIO_INPUT register after: - SYNC_STAGES cycles of gpio_clk (or pclk if CDC_ENABLE=0) - Plus APB read latency

### 15.2.4	Electrical Considerations

#### 15.2.4.1	Output Characteristics

| Parameter | Description |
| --- | --- |
| Drive | Standard CMOS output |
| Slew | Defined by I/O cell |
| Protection | ESD per I/O cell design |

#### 15.2.4.2	Input Characteristics

| Parameter | Description |
| --- | --- |
| Levels | CMOS compatible |
| Hysteresis | Optional per I/O cell |
| Pull-up/down | External to GPIO module |

## 15.3	Timing

### 15.3.1	Output Path

- gpio_out updates on clock edge

- gpio_oe updates on same clock edge

- No glitch-free guarantee during direction change

### 15.3.2	Input Path

- Setup/hold relative to synchronizer clock

- Metastability resolved by synchronizer

## 15.4	Navigation

Next: 03_interrupt.md - Interrupt Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	APB GPIO - Interrupt Interface

## 16.1	Ports

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| irq | 1 | Output | Interrupt request (active high) |

## 16.2	Functional Description

### 16.2.1	Interrupt Generation

per-pin effective status:
  level pin (TYPE=1): raw_detector[i] & GPIO_INT_ENABLE[i]     (live)
  edge  pin (TYPE=0): GPIO_INT_STATUS[i] & GPIO_INT_ENABLE[i]  (sticky latch)

irq = GPIO_CONTROL[1] && (| effective_status)

IRQ requires the global interrupt enable GPIO_CONTROL[1] (resets to 0). Level pins drive irq from the live detector output, bypassing GPIO_INT_STATUS; edge pins drive it from the latched status re-gated by the current per-pin enable, so clearing GPIO_INT_ENABLE[i] masks an already-latched edge interrupt on that pin without disturbing its status bit.

### 16.2.2	Per-Pin Configuration

Each GPIO pin can generate interrupts independently:

| Register | Function |
| --- | --- |
| GPIO_INT_ENABLE | Enable/disable per pin |
| GPIO_INT_TYPE | Edge (0) or Level (1) |
| GPIO_INT_POLARITY | Falling/Low (0) or Rising/High (1) |
| GPIO_INT_BOTH | Both edges (edge mode only) |
| GPIO_INT_STATUS | Current interrupt status |

### 16.2.3	Interrupt Modes

#### 16.2.3.1	Edge-Triggered

GPIO_INT_TYPE[i] = 0

Edge Detection

- Captures transitions on synchronized input

- Status bit latches until cleared by software

- Both-edge mode ignores polarity setting

#### 16.2.3.2	Level-Sensitive

GPIO_INT_TYPE[i] = 1

Level Compare

- Continuously compares input to polarity

- The STATUS bit is sticky: it latches on the (enabled) level event and stays set after the level clears, until W1C - same latching as edge mode

- What follows the input level is irq itself, because level pins drive irq from the live detector rather than from GPIO_INT_STATUS

### 16.2.4	Interrupt Handling

#### 16.2.4.1	Software Sequence

- IRQ asserts (hardware)

- CPU vectors to interrupt handler

- Read GPIO_INT_STATUS to identify sources

- Handle interrupt condition

- Write 1 to GPIO_INT_STATUS bits to clear

- IRQ deasserts if no other sources

#### 16.2.4.2	Clearing Interrupts

| Mode | Clear Method |
| --- | --- |
| Edge | Write 1 to STATUS bit |
| Level | Clear source, then write 1 to STATUS |

## 16.3	Timing

### 16.3.1	Edge-Triggered Latency

Edge-Triggered Latency

Total: 3 clock cycles typical (SYNC_STAGES=2 + 1 sticky-flop; edge detect adds no cycle)

### 16.3.2	Level-Sensitive Latency

Level-Sensitive Latency

Total: 2 clock cycles typical (level irq is combinational from the synchronizer output)

Both totals are core-clock cycles. With CDC_ENABLE=1 the core clock is gpio_clk and irq then crosses a 2-flop synchronizer into pclk (apb4_gpio.sv, gen_irq_sync), adding two pclk cycles; with CDC_ENABLE=0 there is no crossing and nothing is added.

## 16.4	Design Notes

### 16.4.1	Connection Guidelines

- Connect to interrupt controller input

- Active-high, level-sensitive recommended at controller

- Single IRQ covers all 32 GPIO pins

- Already in the pclk domain when CDC_ENABLE=1 (synchronized inside apb4_gpio); no external synchronizer needed

## 16.5	Navigation

Next: 04_system.md - System Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	APB GPIO - System Interface

## 17.1	Ports

### 17.1.1	pclk - APB Clock

| Parameter | Value |
| --- | --- |
| Purpose | APB interface clock |
| Frequency | 50-200 MHz typical |
| Domain | APB bus timing |

Used for: - APB protocol timing - Register file access - irq output (generated on pclk when CDC_ENABLE=0; synchronized into pclk when CDC_ENABLE=1)

### 17.1.2	gpio_clk - GPIO Clock

| Parameter | Value |
| --- | --- |
| Purpose | GPIO core clock |
| Frequency | Application dependent |
| Usage | Only when CDC_ENABLE=1 |

Used for: - Input synchronization - Output register updates - Interrupt detection

### 17.1.3	presetn - APB Reset

| Parameter | Value |
| --- | --- |
| Polarity | Active low |
| Type | Asynchronous assert, synchronous deassert (see note) |
| Scope | APB interface logic |

Resets: - APB state machine - Register file - Response logic

### 17.1.4	gpio_rstn - GPIO Reset

| Parameter | Value |
| --- | --- |
| Polarity | Active low |
| Type | Asynchronous assert, synchronous deassert (see note) |
| Usage | Only when CDC_ENABLE=1 |

Note: the PeakRDL-generated register file (gpio_regs.sv) is the one exception – it resets SYNCHRONOUSLY (plain always_ff @(posedge clk) with an active-high rst derived from the reset input), so its registers do not reset until a core-clock edge arrives.

Resets: - GPIO output registers - Input synchronizers - Interrupt state

## 17.2	Functional Description

### 17.2.1	Clock Configurations

#### 17.2.1.1	Single Clock Domain (CDC_ENABLE = 0)

Single Clock Domain

gpio_clk: Tie to pclk or leave unconnected

#### 17.2.1.2	Dual Clock Domain (CDC_ENABLE = 1)

Dual Clock Domain

### 17.2.2	Reset Sequence

#### 17.2.2.1	Power-On Reset

- Assert both presetn and gpio_rstn low

- Keep both clocks running: the PeakRDL register file resets synchronously (see the note above)

- Hold reset for minimum 2 clock cycles

- Release gpio_rstn first

- Wait 2 gpio_clk cycles

- Release presetn

- Wait 2 pclk cycles before APB access

### 17.2.3	Figure 3.2: Power-On Reset Sequence

Power-On Reset Sequence

## 17.3	Timing

### 17.3.1	Clock Relationship

| Mode | Constraint |
| --- | --- |
| CDC_ENABLE=0 | Single clock, no special constraints |
| CDC_ENABLE=1 | Set false_path between domains |

### 17.3.2	Reset Recovery

- Allow minimum 2 clock cycles after reset deassert

- First APB transaction may start on 3rd cycle

## 17.4	Navigation

Back to: 00_overview.md - Interfaces Overview

Next Chapter: Chapter 4: Programming Model

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	APB GPIO - Programming Model Overview

## 18.1	Overview

Thirteen registers do all the work. Everything in this chapter is a recipe built on these:

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW | 0x00000001 | Global control |
| 0x004 | GPIO_DIRECTION | RW | 0x00000000 | I/O direction |
| 0x008 | GPIO_OUTPUT | RW | 0x00000000 | Output data |
| 0x00C | GPIO_INPUT | RO | - | Input data |
| 0x010 | GPIO_INT_ENABLE | RW | 0x00000000 | Interrupt enable |
| 0x014 | GPIO_INT_TYPE | RW | 0x00000000 | Edge/level select |
| 0x018 | GPIO_INT_POLARITY | RW | 0xFFFFFFFF | Interrupt polarity |
| 0x01C | GPIO_INT_BOTH | RW | 0x00000000 | Both edges |
| 0x020 | GPIO_INT_STATUS | W1C | 0x00000000 | Interrupt status |
| 0x024 | GPIO_RAW_INT | RO | 0 (live) | Raw (unlatched) event status |
| 0x028 | GPIO_OUTPUT_SET | WO | 0x00000000 | Atomic output set (reads return 0) |
| 0x02C | GPIO_OUTPUT_CLR | WO | 0x00000000 | Atomic output clear (reads return 0) |
| 0x030 | GPIO_OUTPUT_TGL | WO | 0x00000000 | Atomic output toggle (reads return 0) |

### 18.1.1	In This Chapter

#### 18.1.1.1	Basic Operations

Fundamental GPIO read/write operations.

See: 01_basic_operations.md

#### 18.1.1.2	Interrupt Configuration

Setting up and handling GPIO interrupts.

See: 02_interrupt_config.md

#### 18.1.1.3	Programming Examples

Common use cases with code samples.

See: 03_examples.md

#### 18.1.1.4	Software Considerations

Performance tips and best practices.

See: 04_software_notes.md

## 18.2	Navigation

Next: 01_basic_operations.md - Basic Operations

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	APB GPIO - Basic Operations

## 19.1	Waveforms

Before the code, the timing. These diagrams show what actually happens inside the block for each basic operation.

### 19.1.1	Direction Configuration

When software writes to GPIO_DIRECTION, the direction register updates and controls the output enable for each pin.

GPIO Direction Write

The APB write is PREADY-gated – a few pclk beyond the two-cycle APB minimum (see Chapter 3.1). The direction register updates on the clock edge following PREADY, and the output enable (gpio_oe) reflects the new configuration immediately.

### 19.1.2	Output Write

Writing to GPIO_OUTPUT sets the output data register, which drives the external pins when direction is set to output.

GPIO Output Write

The write data flows through the APB interface to the output register. When gpio_oe[n] is high (output mode), gpio_out[n] drives the written value to the external pin. The write is strobe-driven, so writing the value the register already holds is still a write and still lands.

### 19.1.3	Input Read

Reading GPIO_INPUT returns the synchronized input values from external pins.

GPIO Input Read

External inputs pass through a 2-stage synchronizer before being captured. The synchronized value (w_gpio_sync) is returned on s_apb_PRDATA during the APB read transaction.

### 19.1.4	Input Synchronization

All GPIO inputs pass through a 2-stage synchronizer to prevent metastability.

GPIO Input Sync

The synchronizer adds 2 clock cycles of latency. External asynchronous transitions on gpio_in propagate through sync_stage1 and sync_stage2 before appearing on the internal synchronized signal w_gpio_sync.

### 19.1.5	Atomic Operations

The SET, CLEAR, and TOGGLE registers provide atomic bit manipulation without read-modify-write races.

GPIO Atomic Operations

Three consecutive APB writes demonstrate: 1. GPIO_OUTPUT_SET (0x028): Sets bits where write data is 1, leaves others unchanged 2. GPIO_OUTPUT_CLR (0x02C): Clears bits where write data is 1, leaves others unchanged 3. GPIO_OUTPUT_TGL (0x030): Inverts bits where write data is 1, leaves others unchanged

Each write performs its operation: the RTL fires the atomic pulse from the register’s write strobe, so writing the same mask twice performs the operation twice, and nothing needs to be written between operations. The registers are write-only and read as 0; GPIO_OUTPUT reads back the live result. See Chapter 5 for the full semantics.

## 19.2	Usage Example

### 19.2.1	Initialization

#### 19.2.1.1	Reset State

After reset: - GPIO_CONTROL = 0x00000001 - output stage ENABLED, global interrupt enable clear - All pins configured as inputs (GPIO_DIRECTION = 0), so every pin is high-Z despite the enable - No per-pin interrupts enabled; GPIO_INT_POLARITY resets to 0xFFFFFFFF (rising/active-high)

#### 19.2.1.2	Enable GPIO

// GPIO_CONTROL[0] (ENABLE) already resets to 1; write it explicitly if a
// previous owner may have cleared it. ENABLE gates only the output drivers.
GPIO_CONTROL = 0x00000001;

### 19.2.2	Output Operations

#### 19.2.2.1	Configure as Output

// Set pins 7:4 as outputs (bits = 1 for output)
GPIO_DIRECTION = 0x000000F0;

#### 19.2.2.2	Write Output Values

// Set pins 7:4 to value 0101
GPIO_OUTPUT = 0x00000050;

#### 19.2.2.3	Toggle Outputs

// Atomic toggle of pins 7:4; every write toggles, so a loop that
// writes this each iteration toggles on every pass
GPIO_OUTPUT_TGL = 0x000000F0;

#### 19.2.2.4	Atomic Bit Operations

// Set specific bits (pins 5 and 7)
GPIO_OUTPUT_SET = 0x000000A0;

// Clear specific bits (pins 4 and 6)
GPIO_OUTPUT_CLR = 0x00000050;

Read-modify-write of GPIO_OUTPUT (GPIO_OUTPUT |= mask) mixes safely with the atomic registers: GPIO_OUTPUT reads back the live output latch, so after GPIO_OUTPUT = 0xFF and GPIO_OUTPUT_CLR = 0x0F a read returns 0xF0 and an RMW builds on the real pin state. Prefer the atomic registers where two contexts may update outputs concurrently – that is what they are for.

### 19.2.3	Input Operations

#### 19.2.3.1	Configure as Input

// Set pins 3:0 as inputs (bits = 0 for input)
GPIO_DIRECTION &= ~0x0000000F;

#### 19.2.3.2	Read Input Values

// Read all inputs
uint32_t inputs = GPIO_INPUT;

// Check specific pin (pin 2)
if (inputs & 0x00000004) {
    // Pin 2 is high
}

#### 19.2.3.3	Read with Mask

// Read only pins 3:0
uint32_t low_nibble = GPIO_INPUT & 0x0000000F;

### 19.2.4	Mixed I/O Configuration

#### 19.2.4.1	Configure Mixed Directions

// Pins 31:16 = outputs, pins 15:0 = inputs
GPIO_DIRECTION = 0xFFFF0000;

#### 19.2.4.2	Read-Modify-Write Pattern

// Change only pins 11:8 to outputs
uint32_t dir = GPIO_DIRECTION;
dir |= 0x00000F00;    // Set pins 11:8
GPIO_DIRECTION = dir;

### 19.2.5	Output Enable Behavior

#### 19.2.5.1	Hardware Interface

When direction bit is set: - gpio_oe[i] = 1 (output enabled) - gpio_out[i] = GPIO_OUTPUT[i] value

When direction bit is clear: - gpio_oe[i] = 0 (high impedance) - gpio_out[i] = don’t care

#### 19.2.5.2	Glitch Considerations

To avoid output glitches when switching direction: 1. Set GPIO_OUTPUT to desired value 2. Then change GPIO_DIRECTION

// Safe direction change to output
GPIO_OUTPUT = desired_value;    // Set value first
GPIO_DIRECTION |= pin_mask;     // Then enable output

## 19.3	Navigation

Next: 02_interrupt_config.md - Interrupt Configuration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	APB GPIO - Interrupt Configuration

## 20.1	Waveforms

First, what the hardware does. These diagrams walk through detection and clearing, one mode at a time.

### 20.1.1	Rising Edge Interrupt

Edge-triggered interrupts detect transitions on input pins.

GPIO Rising Edge Interrupt

The detection sequence: 1. External input gpio_in[0] transitions from 0 to 1 2. 2-stage synchronizer captures the transition (w_gpio_sync) 3. Edge detector compares current vs. delayed value (r_gpio_prev) 4. Rising edge pulse (w_rising_edge) generated for one clock 5. Raw pulse (w_raw_int[0], combinational) sets the sticky GPIO_INT_STATUS bit in the register file 6. Combined irq output asserts

### 20.1.2	Falling Edge Interrupt

Falling edge detection uses inverted polarity configuration.

GPIO Falling Edge Interrupt

With cfg_int_type[0]=0 (edge mode) and cfg_int_polarity[0]=0 (falling edge), the detector triggers on 1-to-0 transitions.

### 20.1.3	Level-Sensitive Interrupt

Level-sensitive interrupts track the input state directly.

GPIO Level Interrupt

Key differences from edge mode: - irq follows the input level (not latched) - No edge detection logic involved - Interrupt re-asserts if source not cleared before ISR exit

### 20.1.4	Interrupt Clear (W1C)

Write-1-to-Clear mechanism clears latched interrupts.

GPIO Interrupt Clear

The clear sequence: 1. GPIO_INT_STATUS[0] is set (edge was detected and latched) 2. Software writes 0x01 to INT_STATUS register 3. W1C logic clears the sticky status bit 4. irq deasserts

Note: For level-sensitive interrupts, the external source must be cleared first, otherwise the interrupt immediately re-asserts.

## 20.2	Functional Description

### 20.2.1	Interrupt Configuration Table

Every combination of the three mode fields, in one place:

| INT_TYPE | INT_POLARITY | INT_BOTH | Trigger |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Falling edge |
| 0 | 1 | 0 | Rising edge |
| 0 | X | 1 | Both edges |
| 1 | 0 | X | Active low |
| 1 | 1 | X | Active high |

## 20.3	Usage Example

### 20.3.1	Interrupt Setup

Four steps, in order. Skipping step 4 is the classic mistake — the global enable resets to 0, so nothing ever interrupts.

#### 20.3.1.1	1. Configure Interrupt Type

// Edge-triggered (0) or Level-sensitive (1)
// Pins 3:0 edge, pins 7:4 level
GPIO_INT_TYPE = 0x000000F0;

#### 20.3.1.2	2. Configure Polarity

For edge mode: - 0 = falling edge - 1 = rising edge

For level mode: - 0 = active low - 1 = active high

// Rising edge / active high for pins 7:0
GPIO_INT_POLARITY = 0x000000FF;

#### 20.3.1.3	3. Configure Both-Edge Mode (Edge Mode Only)

// Enable both edges for pin 0
GPIO_INT_BOTH = 0x00000001;

#### 20.3.1.4	4. Enable Interrupts

// Enable interrupts on pins 7:0
GPIO_INT_ENABLE = 0x000000FF;

// Set the GLOBAL interrupt enable, GPIO_CONTROL[1] - it resets to 0 and
// gates the irq output, so without this step no interrupt ever asserts.
GPIO_CONTROL |= 0x00000002;

### 20.3.2	Complete Setup Examples

#### 20.3.2.1	Rising Edge Interrupt

// Configure pin 5 for rising edge interrupt
GPIO_INT_TYPE &= ~(1 << 5);      // Edge mode
GPIO_INT_POLARITY |= (1 << 5);   // Rising edge
GPIO_INT_BOTH &= ~(1 << 5);      // Single edge
GPIO_INT_ENABLE |= (1 << 5);     // Per-pin enable
GPIO_CONTROL |= 0x2;             // Global INT_ENABLE (resets to 0; irq never asserts without it)

#### 20.3.2.2	Both-Edge Interrupt

// Configure pin 3 for both-edge interrupt
GPIO_INT_TYPE &= ~(1 << 3);      // Edge mode
GPIO_INT_BOTH |= (1 << 3);       // Both edges
GPIO_INT_ENABLE |= (1 << 3);     // Per-pin enable
GPIO_CONTROL |= 0x2;             // Global INT_ENABLE

#### 20.3.2.3	Active-Low Level Interrupt

// Configure pin 7 for active-low level interrupt
GPIO_INT_TYPE |= (1 << 7);       // Level mode
GPIO_INT_POLARITY &= ~(1 << 7);  // Active low
GPIO_INT_ENABLE |= (1 << 7);     // Per-pin enable
GPIO_CONTROL |= 0x2;             // Global INT_ENABLE

### 20.3.3	Interrupt Handling

#### 20.3.3.1	Check Interrupt Status

uint32_t status = GPIO_INT_STATUS;

#### 20.3.3.2	Clear Interrupts (Write-1-to-Clear)

// Clear specific interrupt (pin 5)
GPIO_INT_STATUS = (1 << 5);

// Clear all pending interrupts
GPIO_INT_STATUS = 0xFFFFFFFF;

#### 20.3.3.3	Complete ISR Example

void gpio_isr(void) {
    // Read status
    uint32_t status = GPIO_INT_STATUS;

    // Handle each pending interrupt
    for (int i = 0; i < 32; i++) {
        if (status & (1 << i)) {
            handle_gpio_event(i);
        }
    }

    // Clear handled interrupts
    GPIO_INT_STATUS = status;
}

### 20.3.4	Level-Sensitive Considerations

#### 20.3.4.1	Avoid Interrupt Storm

For level-sensitive interrupts, the source must be cleared before the status:

void level_sensitive_isr(void) {
    uint32_t status = GPIO_INT_STATUS;

    // For level-sensitive pins, handle source first
    if (status & LEVEL_PIN_MASK) {
        clear_external_source();  // Clear what's driving pin
    }

    // Then clear status
    GPIO_INT_STATUS = status;
}

#### 20.3.4.2	Masking During Handling

Use the global enable, GPIO_CONTROL[1]. Clearing GPIO_INT_ENABLE does NOT mask an edge interrupt that has already latched into GPIO_INT_STATUS (the per-pin enable is only sampled when the event sets the sticky bit), so an enable-save/restore recipe leaves irq asserted for pending edge sources.

// Temporarily mask everything while handling
GPIO_CONTROL &= ~0x00000002;   // global interrupt enable off

// Handle interrupt source
handle_interrupt();

// Unmask
GPIO_CONTROL |= 0x00000002;

## 20.4	Navigation

Next: 03_examples.md - Programming Examples

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	APB GPIO - Programming Examples

## 21.1	Usage Example

Real recipes for the things people actually hang off a GPIO. Copy, paste, adjust the pin numbers.

### 21.1.1	LED Control

#### 21.1.1.1	Simple LED Blink

#define LED_PIN  (1 << 0)

void led_init(void) {
    GPIO_CONTROL = 1;            // Enable GPIO
    GPIO_DIRECTION |= LED_PIN;   // Set as output
}

void led_on(void) {
    GPIO_OUTPUT |= LED_PIN;
}

void led_off(void) {
    GPIO_OUTPUT &= ~LED_PIN;
}

void led_toggle(void) {
    GPIO_OUTPUT ^= LED_PIN;
}

The read-modify-write style above is fine alongside the atomic GPIO_OUTPUT_SET/CLR/TGL registers: GPIO_OUTPUT reads back the live output latch, so an RMW always starts from what the pins are actually driving (Chapter 5). Where two contexts may update outputs at once, the atomic registers avoid the RMW race entirely, and every write takes effect:

void led_on(void)     { GPIO_OUTPUT_SET = LED_PIN; }
void led_off(void)    { GPIO_OUTPUT_CLR = LED_PIN; }
void led_toggle(void) { GPIO_OUTPUT_TGL = LED_PIN; }

#### 21.1.1.2	Multiple LED Control

#define LED_MASK  0x000000FF  // LEDs on pins 7:0

void leds_write(uint8_t pattern) {
    uint32_t output = GPIO_OUTPUT;
    output = (output & ~LED_MASK) | pattern;
    GPIO_OUTPUT = output;
}

### 21.1.2	Button Input

#### 21.1.2.1	Simple Button Read

#define BUTTON_PIN  (1 << 8)

void button_init(void) {
    GPIO_CONTROL = 1;              // Enable GPIO
    GPIO_DIRECTION &= ~BUTTON_PIN; // Set as input
}

bool button_pressed(void) {
    return (GPIO_INPUT & BUTTON_PIN) != 0;
}

#### 21.1.2.2	Button with Interrupt

#define BUTTON_PIN  (1 << 8)

volatile bool button_event = false;

void button_init_irq(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION &= ~BUTTON_PIN;

    // Configure falling edge interrupt (button press)
    GPIO_INT_TYPE &= ~BUTTON_PIN;      // Edge mode
    GPIO_INT_POLARITY &= ~BUTTON_PIN;  // Falling edge
    GPIO_INT_BOTH &= ~BUTTON_PIN;      // Single edge
    GPIO_INT_ENABLE |= BUTTON_PIN;     // Enable pin
    GPIO_CONTROL |= 0x2;               // Global interrupt enable (resets to 0)
}

void button_isr(void) {
    if (GPIO_INT_STATUS & BUTTON_PIN) {
        button_event = true;
        GPIO_INT_STATUS = BUTTON_PIN;  // Clear
    }
}

### 21.1.3	DIP Switch Reading

#### 21.1.3.1	8-Bit Switch Input

#define SWITCH_MASK  0x00FF0000  // Switches on pins 23:16
#define SWITCH_SHIFT 16

void switch_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION &= ~SWITCH_MASK;  // All inputs
}

uint8_t switch_read(void) {
    return (GPIO_INPUT & SWITCH_MASK) >> SWITCH_SHIFT;
}

### 21.1.4	Parallel Data Interface

#### 21.1.4.1	8-Bit Output Port

#define DATA_MASK   0x000000FF  // Data on pins 7:0
#define STROBE_PIN  (1 << 8)    // Strobe on pin 8

void data_port_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION |= (DATA_MASK | STROBE_PIN);
    GPIO_OUTPUT &= ~STROBE_PIN;  // Strobe low
}

void data_write(uint8_t data) {
    uint32_t output = GPIO_OUTPUT;
    output = (output & ~DATA_MASK) | data;
    GPIO_OUTPUT = output;

    // Generate strobe pulse
    GPIO_OUTPUT |= STROBE_PIN;
    // Small delay if needed
    GPIO_OUTPUT &= ~STROBE_PIN;
}

#### 21.1.4.2	8-Bit Input Port with Ready

#define DATA_MASK   0x000000FF  // Data on pins 7:0
#define READY_PIN   (1 << 8)    // Ready on pin 8

void data_input_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION &= ~(DATA_MASK | READY_PIN);

    // Interrupt on ready rising edge
    GPIO_INT_TYPE &= ~READY_PIN;
    GPIO_INT_POLARITY |= READY_PIN;
    GPIO_INT_ENABLE |= READY_PIN;
}

uint8_t data_read(void) {
    return GPIO_INPUT & DATA_MASK;
}

### 21.1.5	PWM-Style Output

#### 21.1.5.1	Bit-Banged PWM (Low Frequency)

#define PWM_PIN  (1 << 0)

void pwm_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION |= PWM_PIN;
}

// Call from timer interrupt
void pwm_update(uint8_t duty, uint8_t *counter) {
    (*counter)++;
    if (*counter >= 100) *counter = 0;

    if (*counter < duty) {
        GPIO_OUTPUT |= PWM_PIN;
    } else {
        GPIO_OUTPUT &= ~PWM_PIN;
    }
}

### 21.1.6	Wake-On-Change

#### 21.1.6.1	Power Management Integration

#define WAKE_PINS  0x0000000F  // Wake sources on pins 3:0

void wake_setup(void) {
    // Configure both-edge interrupts for wake pins
    GPIO_INT_TYPE &= ~WAKE_PINS;   // Edge mode
    GPIO_INT_BOTH |= WAKE_PINS;    // Both edges
    GPIO_INT_ENABLE |= WAKE_PINS;  // Enable

    // Clear any pending before sleep
    GPIO_INT_STATUS = WAKE_PINS;
}

void enter_sleep(void) {
    wake_setup();
    // Platform-specific sleep entry
    __WFI();  // Wait for interrupt
}

## 21.2	Navigation

Next: 04_software_notes.md - Software Considerations

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	APB GPIO - Software Considerations

## 22.1	Design Notes

The stuff that separates a driver that works from a driver that works reliably. None of this is in the register map; all of it matters.

### 22.1.1	Performance

#### 22.1.1.1	Register Access Timing

| Operation | APB Cycles | Notes |
| --- | --- | --- |
| Read | 2 + wait states | PREADY-gated (bridge FSM + cmd/rsp handshake) |
| Write | 2 + wait states | PREADY-gated |
| Read-Modify-Write | one read + one write | Each PREADY-gated |

#### 22.1.1.2	Optimizing Access

Batch operations when possible:

// Inefficient - 4 separate writes
GPIO_OUTPUT |= (1 << 0);
GPIO_OUTPUT |= (1 << 1);
GPIO_OUTPUT |= (1 << 2);
GPIO_OUTPUT |= (1 << 3);

// Efficient - single write
GPIO_OUTPUT |= 0x0000000F;

Cache register values:

// Inefficient - 4 reads + 4 writes
for (int i = 0; i < 4; i++) {
    GPIO_DIRECTION |= (1 << i);
}

// Efficient - 1 read + 1 write
uint32_t dir = GPIO_DIRECTION;
dir |= 0x0000000F;
GPIO_DIRECTION = dir;

### 22.1.2	Synchronization

#### 22.1.2.1	Input Latency

GPIO inputs have inherent latency: - SYNC_STAGES clock cycles (default 2) - Plus software polling/interrupt overhead

Account for latency in timing-critical code.

#### 22.1.2.2	Volatile Registers

Always declare GPIO registers as volatile:

#define GPIO_INPUT  (*(volatile uint32_t *)0xFEC0700C)

#### 22.1.2.3	Multi-Core Considerations

If multiple cores access GPIO:

// Use atomic operations or locks
spin_lock(&gpio_lock);
uint32_t val = GPIO_OUTPUT;
val |= new_bits;
GPIO_OUTPUT = val;
spin_unlock(&gpio_lock);

### 22.1.3	Interrupt Best Practices

#### 22.1.3.1	Clear Before Return

Always clear interrupt status before ISR return:

void gpio_isr(void) {
    uint32_t status = GPIO_INT_STATUS;
    // Handle interrupts
    GPIO_INT_STATUS = status;  // Must clear!
}

#### 22.1.3.2	Avoid Spurious Interrupts

Disable interrupts during configuration:

void reconfigure_interrupt(int pin) {
    uint32_t mask = (1 << pin);

    // Disable first
    GPIO_INT_ENABLE &= ~mask;

    // Reconfigure
    // ...

    // Clear any pending
    GPIO_INT_STATUS = mask;

    // Re-enable
    GPIO_INT_ENABLE |= mask;
}

#### 22.1.3.3	Level-Sensitive Caution

Level interrupts can cause interrupt storms:

void level_isr(void) {
    // WRONG - will re-trigger immediately
    GPIO_INT_STATUS = status;

    // RIGHT - handle source first
    clear_external_interrupt_source();
    GPIO_INT_STATUS = status;
}

### 22.1.4	Error Handling

#### 22.1.4.1	No Hardware Errors

GPIO controller doesn’t generate errors: - All addresses valid - pslverr always 0

#### 22.1.4.2	Software Validation

Validate configuration in software:

bool gpio_set_direction(uint32_t pin, bool output) {
    if (pin >= 32) return false;

    if (output) {
        GPIO_DIRECTION |= (1 << pin);
    } else {
        GPIO_DIRECTION &= ~(1 << pin);
    }
    return true;
}

### 22.1.5	Debug Tips

#### 22.1.5.1	Read-Back Verification

void gpio_debug(void) {
    printf("CONTROL:   0x%08X\n", GPIO_CONTROL);
    printf("DIRECTION: 0x%08X\n", GPIO_DIRECTION);
    printf("OUTPUT:    0x%08X\n", GPIO_OUTPUT);
    printf("INPUT:     0x%08X\n", GPIO_INPUT);
    printf("INT_STAT:  0x%08X\n", GPIO_INT_STATUS);
}

#### 22.1.5.2	Loopback Testing

Connect output to input for self-test:

bool gpio_loopback_test(int out_pin, int in_pin) {
    GPIO_DIRECTION |= (1 << out_pin);
    GPIO_DIRECTION &= ~(1 << in_pin);

    // Test high
    GPIO_OUTPUT |= (1 << out_pin);
    delay_us(10);  // Allow synchronization
    if (!(GPIO_INPUT & (1 << in_pin))) return false;

    // Test low
    GPIO_OUTPUT &= ~(1 << out_pin);
    delay_us(10);
    if (GPIO_INPUT & (1 << in_pin)) return false;

    return true;
}

## 22.2	Navigation

Back to: 00_overview.md - Programming Model Overview

Next Chapter: Chapter 5: Registers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	APB GPIO - Register Map

## 23.1	Overview

Thirteen registers, verified against the RTL decode (gpio_regs.sv) and ../../rdl/gpio/gpio_regs.rdl. GPIO_REGS_SIZE = 0x34.

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW | 0x00000001 | Global enable + global interrupt enable |
| 0x004 | GPIO_DIRECTION | RW | 0x00000000 | Pin direction |
| 0x008 | GPIO_OUTPUT | RW | 0x00000000 | Output data |
| 0x00C | GPIO_INPUT | RO | - | Input data |
| 0x010 | GPIO_INT_ENABLE | RW | 0x00000000 | Per-pin interrupt enable |
| 0x014 | GPIO_INT_TYPE | RW | 0x00000000 | Interrupt type |
| 0x018 | GPIO_INT_POLARITY | RW | 0xFFFFFFFF | Interrupt polarity |
| 0x01C | GPIO_INT_BOTH | RW | 0x00000000 | Both-edge enable |
| 0x020 | GPIO_INT_STATUS | W1C | 0x00000000 | Latched interrupt status |
| 0x024 | GPIO_RAW_INT | RO | 0 (live) | Live (unlatched) event detector output – reads 0 at reset only because type/polarity/sync all reset to 0; depends on pins thereafter |
| 0x028 | GPIO_OUTPUT_SET | WO | 0x00000000 | Atomic output set (reads return 0) |
| 0x02C | GPIO_OUTPUT_CLR | WO | 0x00000000 | Atomic output clear (reads return 0) |
| 0x030 | GPIO_OUTPUT_TGL | WO | 0x00000000 | Atomic output toggle (reads return 0) |

Address decode and aliasing: only address bits [5:0] reach the register block (gpio_config_regs.sv passes regblk_addr[5:0]), so the 13-register map aliases every 64 bytes across the 4KB APB window. Offsets 0x034-0x03F within each 64-byte tile are unmapped and read as zero; no access ever raises PSLVERR.

## 23.2	GPIO_CONTROL (0x000)

Global control register.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | Reserved | RO | 0 | Reserved |
| 1 | INT_ENABLE | RW | 0 | Global interrupt enable. Gates the irq output: irq = INT_ENABLE && (any effective pending interrupt). Resets to 0, so no interrupt asserts until software sets this bit. |
| 0 | ENABLE | RW | 1 | GPIO output enable. Gates only gpio_oe: when 0, every pin is high-Z regardless of GPIO_DIRECTION. Output data, input synchronization, and interrupt detection are unaffected. Resets to 1 (enabled). |

## 23.3	GPIO_DIRECTION (0x004)

Pin direction control. Each bit controls one GPIO pin.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | DIR | RW | 0 | Direction per pin (0=input, 1=output) |

## 23.4	GPIO_OUTPUT (0x008)

Output data register. Values driven when pin configured as output.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | DATA | RW | 0 | Output values per pin |

Readback: reads return the live output latch (r_output_data in gpio_core.sv), not merely the last value software wrote. After any atomic operation (GPIO_OUTPUT_SET/CLR/TGL) the core writes its latch back into this register one cycle later, so the register and the pins stay in agreement: write 0xFF here, then 0x0F to GPIO_OUTPUT_CLR, and a read returns 0xF0. A read-modify-write of GPIO_OUTPUT therefore mixes safely with the atomic registers. The write itself is strobe-driven, so writing the value the register already holds still lands, and a software write wins over a coincident hardware write-back.

## 23.5	GPIO_INPUT (0x00C)

Input data register. Reflects synchronized external pin values.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | DATA | RO | - | Input values per pin |

Note: Value depends on external signals, not reset.

## 23.6	GPIO_INT_ENABLE (0x010)

Interrupt enable register. Controls which pins can generate interrupts.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | IE | RW | 0 | Interrupt enable per pin (1=enabled) |

## 23.7	GPIO_INT_TYPE (0x014)

Interrupt type select. Chooses edge or level sensitivity.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TYPE | RW | 0 | Type per pin (0=edge, 1=level) |

## 23.8	GPIO_INT_POLARITY (0x018)

Interrupt polarity select.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | POL | RW | 0xFFFFFFFF | Polarity per pin (resets to all rising/active-high) |

For edge mode: 0=falling, 1=rising For level mode: 0=active-low, 1=active-high

## 23.9	GPIO_INT_BOTH (0x01C)

Both-edge interrupt enable. Only applicable in edge mode.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | BOTH | RW | 0 | Both edges per pin (1=both edges) |

When set, GPIO_INT_POLARITY is ignored for that pin.

## 23.10	GPIO_INT_STATUS (0x020)

Latched interrupt status register.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | STATUS | W1C | 0 | Latched interrupt event per pin |

Access: Read returns current status. Write 1 clears the bit.

Semantics (per the RTL):

- A bit sets only when the event occurs while that pin’s GPIO_INT_ENABLE bit is 1 (sts_int_pending = raw & enable, gpio_core.sv). Events on disabled pins set nothing.

- Bits latch (stay set) until written-1-to-clear, in both edge and level modes. In level mode the bit remains set after the input level clears.

- The irq output uses this register only for edge-mode pins. Level-mode pins drive irq from the live detector output, so W1C on a level pin does not deassert irq while the level persists (see Chapter 3.3).

- A W1C write that lands in the same cycle as a new hardware event is merged per bit, next = (value | hw_set) & ~w1c_mask: bits the write is not clearing keep the coincident event (gpio_config_regs.sv holds the set in a one-cycle deferred-set register and applies it once the write completes), and a bit the write IS clearing is cleared, as in any W1C register. No event is lost on a bit software did not ask to clear.

## 23.11	GPIO_RAW_INT (0x024)

Live event-detector output, before per-pin enable gating and latching.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | RAW | RO | 0 | Raw interrupt condition per pin |

Reflects the edge/level detector output combinationally; not affected by GPIO_INT_ENABLE, GPIO_CONTROL, or W1C.

## 23.12	GPIO_OUTPUT_SET (0x028)

Atomic output set: writing 1 to a bit sets the corresponding output pin without a read-modify-write of GPIO_OUTPUT.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | SET | WO | 0 | Set output pins (1=set); reads return 0 |

## 23.13	GPIO_OUTPUT_CLR (0x02C)

Atomic output clear.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | CLR | WO | 0 | Clear output pins (1=clear); reads return 0 |

## 23.14	GPIO_OUTPUT_TGL (0x030)

Atomic output toggle.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TGL | WO | 0 | Toggle output pins (1=toggle); reads return 0 |

## 23.15	Design Notes

### 23.15.1	Atomic-Register Write Semantics (SET/CLR/TGL)

Every write performs the operation, driven by the register’s write strobe (gpio_config_regs.sv). The regblock exports swmod for each register; it is a level held for the whole command-bridge transaction and it leads the field storage by one cycle, so the wrapper takes its rising edge (one write, one event), delays it one flop to line up with the stored mask, and gates the mask with that one-cycle strobe to form the pulse gpio_core acts on:

assign w_set_wr_event = w_set_swmod & ~r_set_swmod_d;   // rising edge
r_set_stb <= w_set_wr_event;                            // align to field
assign w_output_set = r_set_stb ? GPIO_OUTPUT_SET.set_bits.value : '0;

SystemRDL restricts singlepulse to 1-bit fields, which is why the 32-bit masks are pulsed this way rather than self-clearing.

What software can rely on:

- Writing the same mask twice performs the operation twice. A toggle loop writing GPIO_OUTPUT_TGL = mask each iteration toggles on every pass.

- Alternating SET and CLR of the same mask performs every operation; nothing needs to be written in between, and no software shadow of the mask is needed.

- The registers are write-only by design (the RDL declares sw = w): reads at 0x028/0x02C/0x030 return 0. The result of an operation is observable in GPIO_OUTPUT, which reads back the live latch.

These are the conventional set/clear/toggle semantics (fixed 2026-09-08, issue #44).

### 23.15.2	Address Calculation

For system address:

Register_Address = BASE_ADDR + WINDOW_OFFSET + Register_Offset

Where:
  BASE_ADDR = 0xFEC00000 (RLB base)
  WINDOW_OFFSET = 0x7000 (GPIO window)
  Register_Offset = value from table above

Example:
  GPIO_INPUT = 0xFEC00000 + 0x7000 + 0x00C = 0xFEC0700C

## 23.16	Navigation

Back to: GPIO Specification Index