Memory Notes

Simplified DFI 4.0 Simplified edition - DFI Specification v4.0 (Denali Software)

October 4, 2026

Table of Contents

1 Simplified Dfi 4 0 Index	12

2 Introduction	13

2.1 What DFI is	13

2.2 Why the seam exists	14

2.3 Edition lineage	15

2.4 What this book covers	16

3 Organization of the DFI 4.0 Interface	17

3.1 Sub-interface map	17

3.2 Clocking	19

3.3 Phase, word and cycle replication	20

3.4 Slices and channels	21

4 Acronyms	22

5 Control Interface	24

5.1 Control signals	25

5.2 Command encoding on the control pins	28

5.3 LPDDR2/LPDDR3 CA mapping	29

5.4 LPDDR4 CA mapping	30

5.5 Control timing parameters	31

6 Write Data and Read Data Interfaces	32

6.1 Write data signals	33

6.1.1 Write DBI and write CRC	34

6.2 Read data signals	35

6.2.1 Read DBI	36

6.3 Programmable parameters for data path	37

6.4 Key data-path timing parameters	38

7 Update, Status, Error and Low-Power Interfaces	40

7.1 Update interface	41

7.1.1 Update timing parameters	42

7.2 Status interface	44

7.2.1 Status timing parameters	46

7.2.2 Programmable status parameters	47

7.3 Error interface	48

7.4 Low-power control interface	49

8 Training, DB Training, PHY Master, Geardown and LPDDR4 Channels	51

8.1 DFI training interface	52

8.1.1 Training signals	52

8.1.2 Training timing parameters	57

8.1.3 Training programmable parameters	59

8.2 DB training interface	60

8.3 PHY master interface	61

8.3.1 Inactive chip selects	62

8.4 Geardown mode	63

8.5 LPDDR4 channels	64

9 Reset and Initialization	65

9.1 Power-up and the init handshake	66

9.2 Frequency change	67

9.3 Clock disabling during init	68

9.4 Init timing parameters	69

9.5 Update before self-refresh exit	70

10 Training Operations	71

10.1 Training modes	72

10.2 Gate training	73

10.3 Read data eye training	74

10.4 Write leveling	75

10.5 CA training	76

10.6 Write DQ training	77

10.7 DB training	78

10.8 PHY master during training	79

10.9 Training chip-select renaming	80

11 Command Mapping	81

11.1 DDR4 command mapping	82

11.2 LPDDR4 CA command mapping	83

11.3 LPDDR2/LPDDR3 CA mapping	84

11.4 Chip select and rank targeting	85

11.5 CA parity	86

12 Write Carriage	87

12.1 Write timing overview	88

12.2 Write data enable	89

12.3 Write mask and write DBI	90

12.4 Write CRC	91

12.5 Write data chip select	92

13 Read Carriage	93

13.1 Read timing overview	94

13.2 Read data valid	95

13.3 Read DBI	96

13.4 Read data chip select	97

13.5 Data not valid (LPDDR2)	98

14 Wire-Level Walkthroughs	99

14.1 The simplified model	100

14.2 Walkthrough 1: activate -> read -> data return	101

14.3 Walkthrough 2: activate -> write with mask	103

14.4 Walkthrough 3: back-to-back reads with chip-select hold	104

15 Timing Parameter Families	105

15.1 Naming convention	106

15.2 Who defines each parameter	107

15.3 Parameter families	108

15.3.1 Control timing	108

15.3.2 Write data timing	108

15.3.3 Read data timing	108

15.3.4 Update timing	108

15.3.5 Status/init timing	108

15.3.6 Training timing	108

15.3.7 Low-power timing	108

15.3.8 Error timing	108

15.3.9 PHY master timing	108

15.3.10 Disconnect timing	109

16 Frequency Ratio and Phasing	110

16.1 Ratio definitions	111

16.2 What ratio does to scheduling	112

16.3 Phase replication rules	113

16.4 Timing parameter units in ratio systems	114

16.5 Ratio change	115

17 Full Timing Parameter Table	116

17.1 Programmable parameters	128

18 Low-Power Handshake and Update Mechanism	130

18.1 Low-power control	130

18.1.1 Wakeup encoding	130

18.2 Update mechanism	131

18.2.1 MC-initiated update	131

18.2.2 PHY-initiated update	131

18.2.3 DFI idle definition	131

19 Refresh Interaction and Frequency Change	132

19.1 Refresh interaction	132

19.1.1 Refresh during training	132

19.2 Frequency change	133

19.2.1 Acknowledged frequency change	133

19.2.2 Not-acknowledged frequency change	133

19.2.3 Frequency indicator	133

20 Revision Deltas and the andesite Tie-In	134

20.1 3.1 -> 4.0 revision deltas	134

20.1.1 DDR4/LPDDR4 data-path additions	134

20.1.2 Command and control additions	134

20.1.3 System-level additions	135

20.1.4 Training changes	135

20.1.5 Removed signal	136

20.2 The andesite tie-in	137

20.2.1 Inherited surface	137

20.2.2 Implemented 4.0 deltas	137

20.2.3 Study corrections recorded in the area docs	138

20.2.4 Verification posture	138

List of Figures

No figures in this document.

List of Tables

Table 1: DFI revision lineage	15

Table 2: DFI 4.0 sub-interface map	17

Table 3: Acronyms used in this book	22

Table 4: DFI 4.0 control signals	25

Table 5: Simplified command encoding on DFI control pins	28

Table 6: Control timing parameters	31

Table 7: DFI 4.0 write data signals	33

Table 8: DFI 4.0 read data signals	35

Table 9: Data-path programmable parameters	37

Table 10: Data-path timing parameters	38

Table 11: Update interface signals	41

Table 12: Update timing parameters	42

Table 13: Status interface signals	44

Table 14: Status timing parameters	46

Table 15: Status programmable parameters	47

Table 16: Error interface signals	48

Table 17: Error timing parameter	48

Table 18: Low-power control interface signals	49

Table 19: Low-power timing parameters	50

Table 20: DFI training interface signals	52

Table 21: Training timing parameters	57

Table 22: Training programmable parameters	59

Table 23: DB training interface signals	60

Table 24: DB training timing parameters	60

Table 25: PHY master interface signals	61

Table 26: PHY master timing parameters	62

Table 27: Geardown mode signal	63

Table 28: Geardown timing parameter	63

Table 29: LPDDR4 channel programmable parameter	64

Table 30: Initialization handshake phases	66

Table 31: Initialization timing parameters	69

Table 32: Simplified DDR4 command mapping on DFI control wires	82

Table 33: ACT command driven at T0/p0	101

Table 34: RD command driven at T4/p0	101

Table 35: Read enable assertion for BL8 at 1:4 ratio	102

Table 36: Read data return for BL8	102

Table 37: WR command driven at T4/p0	103

Table 38: Write data and mask for BL8	103

Table 39: Back-to-back reads at 1:4 ratio	104

Table 40: Timing parameter definition ownership	107

Table 41: DFI frequency ratios	111

Table 42: Complete DFI 4.0 timing parameter inventory	116

Table 43: Complete DFI 4.0 programmable parameter inventory	128

Table 44: Low-power wakeup encoding	130

Table 45: Data-path deltas	134

Table 46: Control deltas	134

Table 47: System-level deltas	135

Table 48: Training deltas	135

Table 49: andesite implementation of DFI 4.0 deltas	137

List of Waveforms

No waveforms in this document.

# 1	Simplified Dfi 4 0 Index

Generated: 2026-10-20

# 2	Introduction

## 2.1	What DFI is

The DDR PHY Interface (DFI) is the standard boundary between a memory controller (MC) and a DRAM PHY. The MC schedules commands, manages refresh, and buffers read/write data; the PHY clocks that information out to the DRAM devices, trains the data eyes, and returns read data. DFI splits the problem at a well-defined seam so that MC and PHY can be designed, verified and reused independently.

DFI 4.0 is the DDR4-era revision published in April 2018. It merged the 4.0 Spec Addendum into DFI 3.1 and added support for LPDDR4, DDR4-specific features (bank groups, ACT_n, CA parity, DBI, gear-down), and a set of system-level protocols (PHY master, disconnect, write DQ training, DB training). It remains backward-compatible in spirit: the core control/write/read interface from earlier revisions is still there, wrapped in the new signals.

## 2.2	Why the seam exists

A memory controller is a traffic manager. It sees banks, rows, columns, refresh counters, and arbitration. A PHY is a timing-and-electrical translator. It sees delay lines, FIFOs, phase alignment, and single-ended vs differential receivers. The two sides operate on different time scales and different abstractions. DFI standardizes the handoff:

- The MC presents commands, address, bank, and chip-select on the control interface.

- The MC presents write data and mask/DBI on the write data interface.

- The PHY returns read data and valid on the read data interface.

- A small set of status and training wires let the PHY report readiness and request calibration.

The MC does not need to know the PHY’s internal delay settings; the PHY does not need to know the MC’s scheduling policy. Both must agree on DFI timing parameters.

## 2.3	Edition lineage

DFI revision lineage

| DFI revision | Distinctives |
| --- | --- |
| DFI 1.0 | Initial MC/PHY seam for DDR1/DDR2 |
| DFI 2.0/2.1 | DDR3 support, read/write leveling, frequency change, low-power control, 1:2 and 1:4 ratios |
| DFI 3.0/3.1 | DDR4 support (CRC, CA parity, DBI, ACT_n, bank groups), LPDDR3, PHY-requested training, split low-power requests |
| DFI 4.0 | LPDDR4, DB training, per-slice read leveling, read/write data chip selects, write DQ training, PHY master, frequency indicator, disconnect protocol, geardown mode, 3DS/inactive CS support |

## 2.4	What this book covers

This book focuses on the DFI 4.0 signals, timing, and protocols that an andesite-style DDR4/LPDDR4 controller must drive. It is a study condensation, not a replacement for the spec. Every chapter cites the DFI 4.0 section numbers that the material paraphrases. Signal names are used as-is because they are the public vocabulary of the interface.

Source: DFI Specification v4.0 sections 1.0, 2.0, 2.2

# 3	Organization of the DFI 4.0 Interface

## 3.1	Sub-interface map

DFI 4.0 groups signals into sub-interfaces. Each group has its own direction, width naming, and timing parameters. The andesite controller drives most of them and ignores a few that are not required at its design point.

DFI 4.0 sub-interface map

| Sub-interface | Direction | Purpose | DFI 4.0 section |
| --- | --- | --- | --- |
| Control | MC -> PHY | Commands, address, bank, bank-group, CKE, CS, ODT, reset | 3.1 |
| Write Data | MC -> PHY | Write data, byte mask/write DBI, write enable, write CS | 3.2 |
| Read Data | PHY -> MC (data), MC -> PHY (enables) | Read data, read valid, read DBI, read enable, read CS | 3.3 |
| Update | Both | MC-initiated and PHY-initiated update handshakes | 3.4 |
| Status | Both | Init, frequency, clock disable, parity, alert | 3.5 |
| DFI Training | Both | Gate, read data eye, write leveling, CA, write DQ training | 3.6 |
| Low Power Control | MC -> PHY (req), PHY -> MC (ack) | Low-power opportunity requests | 3.7 |
| Error | PHY -> MC | PHY-detected error indication and info | 3.8 |
| DB Training | Both | DDR4 LRDIMM data-buffer training | 3.9 |
| PHY Master | PHY -> MC (req), MC -> PHY (ack) | PHY takes control of the DRAM bus | 3.10 |
| Geardown | MC -> PHY | DDR4 geardown-mode enable | 3.13 |
| LPDDR4 Channels | MC/PHY/system | Channel enable for LPDDR4 combined/independent modes | 3.14 |

## 3.2	Clocking

DFI is clocked by the DFI clock. In a matched-frequency (1:1) system the DFI clock and the DRAM clock run at the same rate. In frequency-ratio systems the DFI PHY clock is a divided version of the DRAM clock: 1:2 means one DFI clock per two DRAM clocks; 1:4 means one DFI clock per four DRAM clocks.

The andesite design point uses a 1:4 frequency ratio inherited from its predecessor. Four phases of control and write data are transferred per DFI clock; read data is returned as words that map back to those phases.

## 3.3	Phase, word and cycle replication

DFI 4.0 replicates signals with suffixes that depend on the sub-interface:

- _pN (phase): used on control, write data, read enable, parity, and training strobes. Phase 0 may drop the suffix.

- _wN (data word): used on read data, read data valid, read data DNV, read data DBI, and DB training response.

- _aN (clock cycle): used on dfi_alert_n.

For a 1:4 system, _p0 through _p3 carry the four DRAM-clock phases of each DFI clock. The MC may issue commands on any phase; the PHY must accept commands on any phase. For read data, _w0 through _w3 carry the corresponding data words.

## 3.4	Slices and channels

A slice is an independent data-logic component in the PHY. DFI minimizes slice-to-slice communication and replicates data-control signaling for point-to-point MC-PHY connectivity. In andesite, slices correspond to x8 or x16 data groups; training enables like dfi_rdlvl_en are one bit per slice.

LPDDR4 channels are a DFI 4.0 concept. LPDDR4 devices may be configured as two independent 16-bit channels or one combined 32-bit channel. The phychannel_en parameter tells the PHY which channels are enabled. The andesite design point uses one DFI bus per channel at x16 width.

Source: DFI Specification v4.0 sections 3.0, 3.11, 3.14, 4.9

# 4	Acronyms

Acronyms used in this book

| Term | Meaning |
| --- | --- |
| ACT | Activate command |
| BG | Bank group (DDR4) |
| CA | Command/Address bus |
| CRC | Cyclic redundancy check |
| CS | Chip select |
| DB | Data buffer (LRDIMM) |
| DBI | Data bus inversion |
| DFI | DDR PHY Interface |
| DNV | Data not valid |
| DQ | Data pin group (data + mask/DBI + strobe) |
| DQS | Data strobe |
| FGR | Fine granularity refresh |
| LPDDR | Low-power DDR |
| MC | Memory controller |
| MPC | Multi-purpose command (LPDDR4) |
| MR | Mode register |
| MRR | Mode register read |
| MRS | Mode register set |
| MPR | Multi-purpose register (DDR4) |
| ODT | On-die termination |
| PHY | Physical plane (the DRAM interface block) |
| PRE | Precharge command |
| RD | Read command |
| REF | Refresh command |
| SDR | Single data rate |
| SR | Self-refresh |
| WR | Write command |
| 3DS | Three-dimensional stacked DRAM |

Source: DFI Specification v4.0 section 2.2

# 5	Control Interface

The control interface carries address, command and control signals from the MC to the PHY. The PHY is responsible for preserving bit ordering and presenting the signals to the DRAM with the delay defined by tctrl_delay. In frequency-ratio systems the control bus is phase-replicated with _pN suffixes; the MC may issue commands on any phase.

Some signals are DRAM-type-specific. dfi_reset_n is used only by DDR3, DDR4 and LPDDR4. dfi_odt is used by DDR2, DDR3, DDR4 and LPDDR3. For LPDDR2/3/4 the command encoding pins (dfi_act_n, dfi_ras_n, dfi_cas_n, dfi_we_n), bank (dfi_bank), bank-group (dfi_bg) and chip-ID (dfi_cid) must be held constant; the CA bus is mapped onto dfi_address.

## 5.1	Control signals

DFI 4.0 control signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_act_n | 1 bit | - | MC -> PHY | Activate select; part of DRAM command encoding with ras/cas/we. Asserted for DDR4 ACT; polarity defines command. |
| dfi_address | DFI Address Width | - | MC -> PHY | Address bus. DDR4: column + lower row bits (A[13:0]); A[16:14] carried on ras/cas/we during ACT. LPDDR2/3: maps 20-bit DDR CA bus. LPDDR4: 6-bit SDR CA. |
| dfi_bank | DFI Bank Width | - | MC -> PHY | Bank address. PHY preserves bit ordering. Constant for LPDDR2/3/4. |
| dfi_bg | DFI Bank Group Width | - | MC -> PHY | DDR4 bank group. Constant for LPDDR2/3/4. |
| dfi_cas_n | DFI Control Width | 0x1 | MC -> PHY | Column address strobe; command encoding. During DDR4 ACT it carries row address bit A15. |
| dfi_cid | DFI Chip ID Width | 0 | MC -> PHY | Chip ID for 3D stacked (3DS) devices. Initializes to 0. Constant for LPDDR2/3/4. |
| dfi_cke | DFI CKE Width | 0x0/0x1 | MC -> PHY | Clock enable. MC drives in all phases. For LPDDR3/4 also enables DRAM output drivers during CA training. |
| dfi_cs | DFI Chip Select Width | 0x1 | MC -> PHY | Chip select; polarity matches memory signal. During LPDDR3/4 CA training it carries the calibration command on the trained rank. |
| dfi_odt | DFI ODT Width | 0x0 | MC -> PHY | On-die termination control. MC drives in all phases. |
| dfi_ras_n | DFI Control Width | 0x1 | MC -> PHY | Row address strobe; command encoding. During DDR4 ACT it carries row address bit A16. |
| dfi_reset_n | DFI Reset Width | 0x0/0x1 | MC -> PHY | DRAM reset. PHY preserves bit ordering. |
| dfi_we_n | DFI Control Width | 0x1 | MC -> PHY | Write enable; command encoding. During DDR4 ACT it carries row address bit A14. |

## 5.2	Command encoding on the control pins

For most DRAM types the command is encoded by the classic four pins:

Simplified command encoding on DFI control pins

| dfi_act_n | dfi_ras_n | dfi_cas_n | dfi_we_n | Typical command |
| --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | ACT (bank activate) |
| 1 | 1 | 0 | 1 | RD (read) |
| 1 | 1 | 0 | 0 | WR (write) |
| 1 | 0 | 1 | 1 | PRE (precharge) |
| 1 | 0 | 0 | 1 | REF (refresh) |
| 0 | x | x | x | DDR4 ACT (ACT_n asserted, ras/cas/we carry upper row bits) |

DDR4 uses dfi_act_n asserted with dfi_ras_n, dfi_cas_n and dfi_we_n carrying row address bits A16, A15 and A14. For LPDDR2/3/4 the command pins are held constant and the CA bus is carried on dfi_address.

## 5.3	LPDDR2/LPDDR3 CA mapping

For LPDDR2 and LPDDR3 the dfi_address bus is at least 20 bits wide. The PHY transmits it as a double-data-rate 10-bit CA bus: bits [9:0] on the rising edge and bits [19:10] on the falling edge of the DRAM clock.

## 5.4	LPDDR4 CA mapping

LPDDR4 uses a 6-bit single-data-rate CA bus. The command is delivered over two consecutive DRAM clock cycles, so a single LPDDR4 command spans two DFI phases. The andesite controller formats this through a dedicated CA submodule in the command formatter.

## 5.5	Control timing parameters

Control timing parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| tcmd_lat | MC | DFI PHY clocks from dfi_cs assertion until associated CA signals are driven (LPDDR4 CS-to-CA delay). |
| tctrl_delay | PHY | DFI clocks from a control signal change to the change reaching the DRAM interface. |

Source: DFI Specification v4.0 sections 3.1, 4.2, Table 5, Table 6

# 6	Write Data and Read Data Interfaces

The write data interface moves write data and mask/DBI from MC to PHY. The read data interface returns read data, data valid, and read DBI from PHY to MC. Both interfaces are phase- or word- replicated in frequency-ratio systems.

## 6.1	Write data signals

DFI 4.0 write data signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_wrdata | DFI Data Width | - | MC -> PHY | Write data bus. Data appears tphy_wrdata cycles after dfi_wrdata_en asserts. Carries MC-generated CRC word when phycrc_mode = 0. |
| dfi_wrdata_cs | DFI Physical Rank Width | - | MC -> PHY | Write data-path chip select (polarity of dfi_cs). Selects target rank for write data or, during write leveling, the rank being trained. New in 4.0. |
| dfi_wrdata_en | DFI Data Enable Width | 0x0 | MC -> PHY | Write data/mask enable. Ideally one bit per PHY data slice; all bits identical so widths may differ. |
| dfi_wrdata_mask | DFI Data Width / 8 | - | MC -> PHY | Byte mask or write DBI. Same timing as dfi_wrdata. Bit N masks dfi_wrdata[8N+7:8N]. When DBI is enabled and phydbi_mode = 0, this bus carries write DBI instead of mask. |

### 6.1.1	Write DBI and write CRC

Write DBI reuses dfi_wrdata_mask. There is no separate dfi_wrdata_dbi signal in DFI 4.0. If the PHY handles DBI (phydbi_mode = 1) the MC presents plain data and mask; if the MC handles DBI (phydbi_mode = 0) the mask bus is repurposed to carry the DBI pattern.

Write CRC may be generated by the MC (phycrc_mode = 0) or by the PHY (phycrc_mode = 1). When the MC generates CRC, the CRC word is appended on dfi_wrdata after the data burst.

## 6.2	Read data signals

DFI 4.0 read data signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_rddata | DFI Data Width | - | PHY -> MC | Read data bus. Valid within tphy_rdlat cycles after dfi_rddata_en asserts. Word-suffixed _wN. |
| dfi_rddata_cs | DFI Physical Rank Width | inactive | MC -> PHY | Read data-path chip select (polarity of dfi_cs). Selects target rank for read data or, during read training, the rank being trained. New in 4.0. |
| dfi_rddata_dbi | DFI DBI Width | - | PHY -> MC | Read DBI. Sent with dfi_rddata; valid only when phydbi_mode = 0. Word-suffixed _wN. |
| dfi_rddata_dnv | DFI Data Width / 8 | 0x0 | PHY -> MC | Data-not-valid per byte (LPDDR2). Same timing as dfi_rddata_valid. Word-suffixed _wN. |
| dfi_rddata_en | DFI Data Enable Width | 0x0 | MC -> PHY | Read data enable. Asserted trddata_en cycles after read command and held for expected contiguous read data. |
| dfi_rddata_valid | DFI Read Data Valid Width | 0x0 | PHY -> MC | Per-slice read data valid. One-to-one response to dfi_rddata_en within tphy_rdlat. Word-suffixed _wN. |

### 6.2.1	Read DBI

Read DBI has a dedicated dfi_rddata_dbi bus. The PHY returns it alongside dfi_rddata when DBI is enabled and phydbi_mode = 0. If phydbi_mode = 1 the PHY inverts data internally and the dfi_rddata_dbi signal is not used.

## 6.3	Programmable parameters for data path

Data-path programmable parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| phycrc_mode | PHY | 0 = CRC generation/validation in MC; 1 = in PHY. |
| phydbi_mode | PHY | 0 = DBI generation and data inversion in MC; 1 = in PHY. |
| dfidata_bit_enable | MC/PHY | Defines valid data bits on dfi_wrdata and dfi_rddata. Replaces the removed dfi_data_byte_disable signal. |

## 6.4	Key data-path timing parameters

Data-path timing parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| tphy_wrlat | PHY | DFI PHY clocks from write command to dfi_wrdata_en assertion. |
| tphy_wrdata | PHY | DFI PHY clocks from dfi_wrdata_en assertion to write data driven on dfi_wrdata. |
| twrdata_delay | System | DFI clocks from dfi_wrdata_en assertion to write data transfer completion on DRAM bus. |
| trddata_en | System | DFI PHY clocks from read command to dfi_rddata_en assertion. |
| tphy_rdlat | PHY | Max DFI PHY clocks from dfi_rddata_en assertion to each corresponding dfi_rddata_valid bit. |
| tphy_rdcslat | PHY | DFI PHY clocks from read command to assertion of associated dfi_rddata_cs. |
| tphy_rdcsgap | PHY | Min additional DFI PHY clocks between commands when changing target rank on dfi_rddata_cs. |
| tphy_wrcslat | PHY | DFI PHY clocks from write command to assertion of associated dfi_wrdata_cs. |
| tphy_wrcsgap | PHY | Min additional DFI PHY clocks between commands when changing target rank on dfi_wrdata_cs. |
| tphy_crcmax_lat | System | Max DFI PHY clocks from dfi_wrdata_en (CRC word cycle) to associated CRC error on dfi_alert_n. |
| tphy_crcmin_lat | System | Min DFI PHY clocks for the same path. |

Source: DFI Specification v4.0 sections 3.2, 3.3, 4.4, 4.5, 4.6, Tables 7-12

# 7	Update, Status, Error and Low-Power Interfaces

These interfaces handle housekeeping: update handshakes, initialization and frequency signaling, error reporting, and low-power opportunity requests.

## 7.1	Update interface

The update interface lets either side force a brief idle window on the DFI bus so that the PHY can update internal delay settings or the MC can update its own state.

Update interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_ctrlupd_req | 1 bit | 0x0 | MC -> PHY | MC-initiated update request. Asserted min tctrlupd_min / max tctrlupd_max cycles. Required immediately before a self-refresh exit command in DFI 4.0. |
| dfi_ctrlupd_ack | 1 bit | 0x0 | PHY -> MC | MC-initiated update acknowledge. PHY may ignore; if acknowledging, must assert before tctrlupd_min expires. DFI bus stays idle while asserted. |
| dfi_phyupd_req | 1 bit | 0x0 | PHY -> MC | PHY-initiated update request. DFI bus must idle while asserted. |
| dfi_phyupd_ack | 1 bit | 0x0 | MC -> PHY | PHY-initiated update acknowledge. MC must assert within tphyupd_resp and hold while dfi_phyupd_req is asserted. |
| dfi_phyupd_type | 2 bits | - | PHY -> MC | Selects one of four PHY update time classes (tphyupd_type0..3). Constant while request asserted. |

### 7.1.1	Update timing parameters

Update timing parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| tctrlupd_interval | MC | Max DFI clocks the MC may wait between dfi_ctrlupd_req assertions. |
| tctrlupd_min / tctrlupd_max | MC | Min/max cycles dfi_ctrlupd_req can be asserted. |
| tphyupd_resp | PHY | Max DFI clocks from dfi_phyupd_req assertion to dfi_phyupd_ack assertion. |
| tphyupd_type0..3 | PHY | Max cycles dfi_phyupd_req may stay asserted after ack for each type. |

## 7.2	Status interface

The status interface carries initialization, frequency, clock-disable and parity/alert signals.

Status interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_alert_n | DFI Alert Width | 0x1 | PHY -> MC | CRC or CA parity error indicator. Pulse width mirrors DRAM subsystem error signal. Cycle-suffixed _aN. |
| dfi_dram_clk_disable | DFI DRAM Clk Disable Width | 0x0 | MC -> PHY | When active, PHY must hold DRAM clocks constant. Timing via tdram_clk_disable / tdram_clk_enable. |
| dfi_freq_ratio | 2 bits | - | MC -> PHY | Frequency ratio indicator: 00=1:1, 01=1:2, 10=1:4, 11=reserved. Defined at init, then constant. |
| dfi_frequency | 5 bits | - | MC -> PHY | Operating-frequency indicator. Up to 32 system-defined encodings. New in 4.0. |
| dfi_init_complete | 1 bit | 0x0 | PHY -> MC | PHY ready to accept DFI transactions. All command/status signals hold defaults until it asserts. De-assertion acknowledges a frequency change. |
| dfi_init_start | 1 bit | 0x0/0x1 | MC -> PHY | At init: indicates dfi_freq_ratio and dfi_frequency are valid. During operation: requests a frequency change. |
| dfi_parity_in | 1 bit | 0x0/0x1 | MC -> PHY | Command parity, one-to-one with each DFI command. Covers address, bank, bank-group, act_n, cas_n, cid, ras_n, we_n. Phase-suffixed _pN. |

### 7.2.1	Status timing parameters

Status timing parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| tdram_clk_disable | PHY | DFI clocks from dfi_dram_clk_disable assertion until DRAM clock holds low at PHY-DRAM boundary. |
| tdram_clk_enable | PHY | DFI clocks from dfi_dram_clk_disable de-assertion to first valid DRAM clock rising edge. |
| tinit_complete | PHY | Max DFI clocks from dfi_init_start de-assertion to dfi_init_complete re-assertion during frequency change. |
| tinit_complete_min | PHY | Min DFI clocks before dfi_init_complete can be driven after a previous command or training event. |
| tinit_start | MC | DFI clocks from dfi_init_start assertion until PHY must de-assert dfi_init_complete to accept frequency change. |
| tinit_start_min | PHY | Min DFI clocks before dfi_init_start can be driven after a previous command or training event. |
| tparin_lat | MC | DFI PHY clocks between DFI command assertion and associated dfi_parity_in. |
| tphy_paritylat | PHY | Max DFI clocks between dfi_parity_in and associated dfi_alert_n return (min 0x4). |

### 7.2.2	Programmable status parameters

Status programmable parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| dfidata_bit_enable | MC/PHY | Valid data bits on dfi_wrdata and dfi_rddata. Replaces removed dfi_data_byte_disable. |
| phyfreq_range | PHY | Number of dfi_frequency values the PHY supports (1-32). |

## 7.3	Error interface

Error interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_error | DFI Error Width | 0x0 | PHY -> MC | PHY-detected error condition (DRAM errors, DLL lock loss, read DQS error, etc.). Not phased. |
| dfi_error_info | DFI Error Width x 4 | 0x0 | PHY -> MC | Additional error-source information. Valid only when dfi_error asserted. Partially spec-defined, extensible with design-specific codes. |

Error timing parameter

| Parameter | Defined by | Description |
| --- | --- | --- |
| terror_resp | PHY | Max DFI clocks from affected DFI bus transaction(s) to dfi_error assertion. |

## 7.4	Low-power control interface

Low-power control interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_lp_ack | 1 bit | 0x0 | PHY -> MC | Low-power acknowledge. PHY may ignore; if acknowledging, asserts within tlp_resp and de-asserts within tlp_wakeup after request de-asserts. |
| dfi_lp_ctrl_req | 1 bit | 0x0 | MC -> PHY | Low-power request for the control interface: no more commands will be sent. |
| dfi_lp_data_req | 1 bit | 0x0 | MC -> PHY | Low-power request for the data interface: no more data will be sent. |
| dfi_lp_wakeup | 4 bits | - | MC -> PHY | Requested wakeup time, 16 encodings from 16 cycles (0000) doubling up to 262144 (1110) and unlimited (1111). May increase but never decrease after acknowledge. |

Low-power timing parameters

| Parameter | Defined by | Description |
| --- | --- | --- |
| tlp_resp | MC | Max DFI clocks from low-power request assertion to dfi_lp_ack assertion (min 1, max 7; recommended 7). |
| tlp_wakeup | MC | Target max DFI clocks that dfi_lp_ack may stay asserted after request de-asserts (min 16). |

Source: DFI Specification v4.0 sections 3.4, 3.5, 3.7, 3.8, Tables 13-17, 21, 22

# 8	Training, DB Training, PHY Master, Geardown and LPDDR4 Channels

DFI 4.0 adds several specialized interfaces beyond the core control/data path. This chapter covers training (gate, read data eye, write leveling, CA, write DQ), DB training for LRDIMM, the PHY master protocol, geardown mode, and LPDDR4 channel configuration.

## 8.1	DFI training interface

DFI 4.0 supports four training operations: gate training, read data eye training, write leveling, and CA training. Write DQ training is new in 4.0. Training can run in DFI Training Mode (MC/PHY handshake over DFI) or PHY Independent Mode (PHY runs training on its own). The andesite design point uses DFI Training Mode.

### 8.1.1	Training signals

DFI training interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_calvl_ca_sel | Defined by tcalvl_ca_sel | 0x0 | MC -> PHY | CA foreground pattern strobe; indicates when foreground pattern should be transmitted PHY-to-DRAM. |
| dfi_calvl_capture | DFI CA Training MC I/F Width | 0x0 | MC -> PHY | One-DFI-clock pulse initiating capture of CA bus value from DQ bus. |
| dfi_calvl_data | 7 bits per slice | - | MC -> PHY | CA VREF training value to transfer to DRAM, per slice. |
| dfi_calvl_done | 1 bit | 0x0 | MC -> PHY | MC has finished iterating VREF values; PHY responds with dfi_calvl_resp. |
| dfi_calvl_en | Per slice | 0x0 | MC -> PHY | CA training logic enable per slice; assertion initiates CA training and acknowledges dfi_calvl_req. |
| dfi_calvl_req | Per slice | 0x0 | PHY -> MC | PHY-initiated CA training request per slice. |
| dfi_calvl_resp | 2 bits | 0x0 | PHY -> MC | CA training response: 00 in-process, 01 LPDDR4 VREF sequence done / LPDDR3 reserved, 10 LPDDR3 change-segment / LPDDR4 reserved, 11 complete. |
| dfi_calvl_result | 1 bit | 0x0 | PHY -> MC | VREF comparison result: 1 = current value better than previous best. |
| dfi_calvl_strobe | Per slice | 0x0 | MC -> PHY | CA VREF training strobe per slice; each DFI clock of assertion generates one DQS pulse. |
| dfi_lvl_pattern | 4 bits x read-training width | 0x0 | MC -> PHY | Read-training pattern select. For DDR4/LPDDR2/3 defines pattern directly; for LPDDR4 indexes a programmed table. |
| dfi_lvl_periodic | Leveling width | 0x0 | MC -> PHY | Global training length: 0 = full/long, 1 = periodic/short. |
| dfi_phy_calvl_cs | DFI Physical Rank Width | inactive | PHY -> MC | CA training target chip select. |
| dfi_phy_rdlvl_cs | Rank x read-training width | inactive | PHY -> MC | Read data eye training target chip select. |
| dfi_phy_rdlvl_gate_cs | Rank x read-training width | inactive | PHY -> MC | Gate training target chip select. |
| dfi_phy_wdqlvl_cs | Rank x write-DQ width | inactive | PHY -> MC | Write DQ training target chip select. |
| dfi_phy_wrlvl_cs | Rank x write-leveling width | inactive | PHY -> MC | Write leveling target chip select. |
| dfi_rdlvl_done | Per slice | 0x0 | MC -> PHY | Read data eye training: MC finished iterating training values. |
| dfi_rdlvl_en | Per slice | 0x0 | MC -> PHY | Read data eye training enable per slice; acknowledges dfi_rdlvl_req. |
| dfi_rdlvl_gate_en | Per slice | 0x0 | MC -> PHY | Gate training enable per slice; acknowledges dfi_rdlvl_gate_req. |
| dfi_rdlvl_gate_req | Per slice | 0x0 | PHY -> MC | PHY-initiated gate training request per slice. |
| dfi_rdlvl_req | Per slice | 0x0 | PHY -> MC | PHY-initiated read data eye training request per slice. |
| dfi_rdlvl_resp | Read response width | 0x0 | PHY -> MC | Read training status: 00 in-process, 01 sequence done (data-eye only), 10 reserved, 11 complete. |
| dfi_wdqlvl_done | Per slice | 0x0 | MC -> PHY | Write DQ training: MC finished iterating VREF values. New in 4.0. |
| dfi_wdqlvl_en | Per slice | 0x0 | MC -> PHY | Write DQ training enable per slice. New in 4.0. |
| dfi_wdqlvl_req | Write-DQ width | 0x0 | PHY -> MC | PHY write DQ training request. New in 4.0. |
| dfi_wdqlvl_resp | Write-DQ response width | 0x0 | PHY -> MC | Write DQ training response. New in 4.0. |
| dfi_wdqlvl_result | 1 bit | 0x0 | PHY -> MC | Write DQ VREF comparison result. New in 4.0. |
| dfi_wrlvl_en | Per slice | 0x0 | MC -> PHY | Write leveling enable per slice; acknowledges dfi_wrlvl_req. |
| dfi_wrlvl_req | Write-leveling width | 0x0 | PHY -> MC | PHY write leveling request. |
| dfi_wrlvl_resp | Per slice | 0x0 | PHY -> MC | Write leveling response: 0 in-process/not active, 1 complete. |
| dfi_wrlvl_strobe | Per slice | 0x0 | MC -> PHY | Write leveling strobe per slice; assert count per request set by syswrlvl_strobe_num. |

### 8.1.2	Training timing parameters

Training timing parameters

| Parameter | Description |
| --- | --- |
| tcalvl_ca_sel | Width generated by combining per-phase PHY inputs for dfi_calvl_ca_sel. |
| tcalvl_capture | DFI clocks after calibration command on dfi_cs before dfi_calvl_capture pulse. |
| tcalvl_cc | Min DFI clocks from one calibration command to the next. |
| tcalvl_cs_ca | PHY DFI clocks from dfi_calvl_ca_sel assertion to dfi_cs. |
| tcalvl_data | Min DFI PHY clocks dfi_calvl_data must be held for DRAM hold time. |
| tcalvl_en | Min DFI clocks from dfi_calvl_en assertion to dfi_cke de-assertion. |
| tcalvl_max | Max DFI clocks MC waits for dfi_calvl_resp after asserting dfi_calvl_en. |
| tcalvl_resp | Max DFI clocks from dfi_calvl_req to MC asserting dfi_calvl_en. |
| tcalvl_strobe | Min DFI PHY clocks from dfi_calvl_data to dfi_calvl_strobe for DRAM setup. |
| trdlvl_en | Min DFI clocks from dfi_rdlvl_en/dfi_rdlvl_gate_en to first read/MRR command. |
| trdlvl_max | Max DFI clocks MC waits for dfi_rdlvl_resp after setting read training enable. |
| trdlvl_resp | Max DFI clocks from read training request to MC asserting enable. |
| trdlvl_rr | Min DFI clocks between consecutive read-training read commands. |
| twdqlvl_en | Min DFI clocks from dfi_wdqlvl_en to first write command. |
| twdqlvl_max | Max DFI clocks MC waits for dfi_wdqlvl_resp after asserting dfi_wdqlvl_en. |
| twdqlvl_resp | Max DFI clocks from dfi_wdqlvl_req to MC asserting dfi_wdqlvl_en. |
| twdqlvl_rw | Min DFI clocks from last read to first write of next calibration commands. |
| twdqlvl_ww | Min DFI clocks between write commands during write DQ training. |
| twrlvl_en | Min DFI clocks from dfi_wrlvl_en to first dfi_wrlvl_strobe. |
| twrlvl_max | Max DFI clocks MC waits for dfi_wrlvl_resp after setting dfi_wrlvl_en. |
| twrlvl_resp | Max DFI clocks from dfi_wrlvl_req to MC asserting dfi_wrlvl_en. |
| twrlvl_ww | Min DFI clocks from one dfi_wrlvl_strobe to the next. |

### 8.1.3	Training programmable parameters

Training programmable parameters

| Parameter | Description |
| --- | --- |
| mcrdlvl_slice_group / phyrdlvl_slice_group | Per-slice grouping of dfi_rdlvl_en bits. All-1 reproduces DFI 3.1 behavior; all-0 enables fully independent per-slice leveling. |
| phycalvl_en | 1 = DFI CA training enabled in PHY. |
| phyrdlvl_en / phyrdlvl_gate_en | PHY supports read data eye / gate training on DFI bus. |
| phywrlvl_en | PHY supports write leveling on DFI bus. |
| phywdqlvl_bst | Number of consecutive write bursts per write DQ training sequence. |
| syswrlvl_strobe_num | Minimum clocks dfi_wrlvl_strobe asserts per write leveling request. |

## 8.2	DB training interface

The DB training interface supports DDR4 LRDIMM data-buffer-to-DRAM training in PHY evaluation mode.

DB training interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_db_train_en | Per slice | 0x0 | MC -> PHY | Enables DB training mode: PHY transfers commands from MC and captures DQ inputs for return on response bus. |
| dfi_db_train_resp | Per slice x DQ per slice | 0x0 | PHY -> MC | DQ data returned to MC for evaluation during DB training. Word-suffixed _wN. |

DB training timing parameters

| Parameter | Description |
| --- | --- |
| tphy_db_train_en | PHY DFI clocks after dfi_db_train_en asserts until first training command can be issued. |
| tphy_db_train_resp | Max DFI clocks from PHY DQ input to dfi_db_train_resp output. |

## 8.3	PHY master interface

The PHY master interface lets the PHY take temporary control of the DRAM bus. This is used for PHY-initiated training or other maintenance that requires the PHY to drive commands directly.

PHY master interface signals

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_phymstr_ack | 1 bit | 0x0 | MC -> PHY | PHY master acknowledge: MC has placed DRAM in a known state and PHY is master of the DRAM bus. |
| dfi_phymstr_cs_state | DFI Chip Select Width | 0x0 | PHY -> MC | Per-rank DRAM state when PHY becomes master: 0 = IDLE or self-refresh per dfi_phymstr_state_sel; 1 = MC may choose any supported state. |
| dfi_phymstr_req | 1 bit | 0x0 | PHY -> MC | PHY requests control of DFI/DRAM buses. |
| dfi_phymstr_state_sel | 1 bit | 0x0 | PHY -> MC | For CS with dfi_phymstr_cs_state=0: 0 = MC must place memory in IDLE, 1 = self-refresh. |
| dfi_phymstr_type | 2 bits | 0x0 | PHY -> MC | Selects one of four PHY master time classes (tphymstr_type0..3). |

PHY master timing parameters

| Parameter | Description |
| --- | --- |
| tphymstr_resp | Max DFI clocks from dfi_phymstr_req to dfi_phymstr_ack. |
| tphymstr_rfsh | Max DFI clocks PHY needs to forward a refresh command from MC to DRAM while master. |
| tphymstr_type0..3 | Max cycles dfi_phymstr_req may stay asserted after ack for each type. |

### 8.3.1	Inactive chip selects

DFI 4.0 introduces syscs_state, a per-chip-select state: 0 = inactive (unpopulated, uninitialized, powered off, DPD, MPSM), 1 = active. The PHY must not assert training chip-select bits for inactive ranks. This lets a controller and PHY handle partially populated systems without treating empty slots as errors.

## 8.4	Geardown mode

Geardown mode is a DDR4 feature where the CA bus runs at half the memory clock rate. The MC changes CA signals only every other DFI PHY clock, synchronized by the PHY.

Geardown mode signal

| Signal | Width | Default | Direction | What it does |
| --- | --- | --- | --- | --- |
| dfi_geardown_en | 1 bit | 0x0 | MC -> PHY | When asserted, MC and PHY operate in DDR4 geardown mode. New in 4.0. |

Geardown timing parameter

| Parameter | Description |
| --- | --- |
| tgeardown_delay | Delay from dfi_geardown_en assertion until PHY is ready to receive commands. |

## 8.5	LPDDR4 channels

LPDDR4 supports two independent 16-bit channels or one combined 32-bit channel.

LPDDR4 channel programmable parameter

| Parameter | Description |
| --- | --- |
| phychannel_en | Which DFI channels are enabled in LPDDR4 combined mode: 01 = ch0, 10 = ch1, 11 = both. Independent mode requires both. |

Source: DFI Specification v4.0 sections 3.6, 3.9, 3.10, 3.11, 3.13, 3.14, Tables 18-20, 23-26

# 9	Reset and Initialization

DFI initialization is a handshake between MC and PHY mediated by dfi_init_start and dfi_init_complete. The same two signals are reused for frequency change during normal operation.

## 9.1	Power-up and the init handshake

At power-up the PHY may optionally wait for dfi_init_start before asserting dfi_init_complete. The MC drives dfi_freq_ratio and dfi_frequency to their initial values and then asserts dfi_init_start. This tells the PHY that the frequency ratio and frequency indicator are valid.

Initialization completes when both dfi_init_start and dfi_init_complete are asserted simultaneously for at least one DFI clock. After that point the PHY accepts normal DFI transactions and all command/status signals may leave their defaults.

Initialization handshake phases

| Phase | MC behavior | PHY behavior |
| --- | --- | --- |
| Power-up | Drive dfi_freq_ratio and dfi_frequency to initial values. | May wait for dfi_init_start. |
| Init request | Assert dfi_init_start. | Accept ratio/frequency; prepare DRAM interface. |
| Init complete | May hold or de-assert dfi_init_start after both signals are high. | Assert dfi_init_complete. |
| Normal operation | Issue commands, data, refresh. | Respond to DFI transactions. |

## 9.2	Frequency change

During normal operation the MC requests a frequency change by asserting dfi_init_start again. The PHY accepts by de-asserting dfi_init_complete within tinit_start cycles. If the PHY does not de-assert dfi_init_complete within tinit_start, the MC must abort the request and release dfi_init_start.

While dfi_init_start is asserted for a frequency change, dfi_frequency must be set to the new legal value and remain unchanged. Once dfi_init_complete de-asserts, the MC holds dfi_init_start until the frequency change is complete. The PHY re-asserts dfi_init_complete within tinit_complete cycles after dfi_init_start de-asserts.

## 9.3	Clock disabling during init

When dfi_init_start is asserted during initialization, dfi_dram_clk_disable must reflect the clocks that are being used. In normal operation this signal can change dynamically to gate the DRAM clocks for power savings.

## 9.4	Init timing parameters

Initialization timing parameters

| Parameter | Description |
| --- | --- |
| tinit_start | Max DFI clocks from dfi_init_start assertion until PHY must de-assert dfi_init_complete to accept frequency change. |
| tinit_start_min | Min DFI clocks before dfi_init_start can be driven after a previous command or training event. |
| tinit_complete | Max DFI clocks from dfi_init_start de-assertion to dfi_init_complete re-assertion during frequency change. |
| tinit_complete_min | Min DFI clocks before dfi_init_complete can be driven after a previous command or training event. |

## 9.5	Update before self-refresh exit

DFI 4.0 requires a dfi_ctrlupd_req/dfi_ctrlupd_ack handshake immediately before a self- refresh exit command. This update window guarantees that the PHY has a clean point to apply any pending delay updates before the command stream resumes at full speed.

Source: DFI Specification v4.0 sections 3.5.1, 3.5.5, 4.1, 4.10

# 10	Training Operations

DFI 4.0 training moves the calibration of read gates, data eyes, write DQS/DQ timing, and CA alignment from the MC into a cooperative MC/PHY protocol. The PHY evaluates DRAM responses and adjusts delay settings; the MC sets up the DRAM, issues training commands, and responds to PHY requests.

## 10.1	Training modes

Two modes exist:

- DFI Training Mode: MC and PHY cooperate through the DFI training handshake. This is the mode described in this book.

- PHY Independent Mode: PHY runs training autonomously after initialization. The MC does not participate beyond enabling the capability.

The andesite controller implements DFI Training Mode for the training operations it supports.

## 10.2	Gate training

Gate training finds the delay at which the read DQS preamble can be reliably captured. The MC asserts dfi_rdlvl_gate_en for the target slice(s), issues the read or MRR commands required by the memory type, and the PHY adjusts the gate delay until the DQS window is centered.

When complete, the PHY drives dfi_rdlvl_resp = 'b11. The MC then de-asserts dfi_rdlvl_gate_en. The dfi_lvl_pattern signal selects the pattern for read data eye training but is not used for gate training.

## 10.3	Read data eye training

Read data eye training centers the read capture clock in the valid data eye. The MC asserts dfi_rdlvl_en for the target slice(s) and issues read/MRR commands. The PHY evaluates the data and adjusts read delay.

If the PHY returns dfi_rdlvl_resp = 'b01, the current sequence is complete but more sequences remain. The MC de-asserts dfi_rdlvl_en, sets up the next sequence (e.g., next dfi_lvl_pattern value), and re-asserts dfi_rdlvl_en. When the PHY returns 'b11, training is complete.

For LPDDR4, dfi_lvl_pattern is an index into a programmed pattern table rather than a direct pattern encoding.

## 10.4	Write leveling

Write leveling aligns the write DQS rising edge with the DRAM clock rising edge. The MC asserts dfi_wrlvl_en, then asserts dfi_wrlvl_strobe for the number of clocks defined by syswrlvl_strobe_num. The PHY samples the DQ bus response and adjusts write DQS delay.

When the PHY returns dfi_wrlvl_resp = 'b1, write leveling is complete for that slice.

## 10.5	CA training

CA training is required for LPDDR3 and LPDDR4. The MC asserts dfi_calvl_en, drives the calibration command on dfi_cs, and uses dfi_calvl_capture to tell the PHY when to sample the CA value returned on the DQ bus. For LPDDR4 CA VREF training, the MC drives dfi_calvl_data, asserts dfi_calvl_strobe to generate DQS pulses, and uses dfi_calvl_done and dfi_calvl_result to iterate to the best VREF setting.

## 10.6	Write DQ training

Write DQ training is new in DFI 4.0 and centers the clock in the write DQ eye. The MC asserts dfi_wdqlvl_en, issues a sequence of write bursts followed by matching read bursts, and iterates VREF values using dfi_wdqlvl_done and dfi_wdqlvl_result. The PHY returns dfi_wdqlvl_resp status. This training is unused unless both MC and PHY are DFI 4.0 devices.

## 10.7	DB training

DB training supports DDR4 LRDIMM data-buffer training in PHY evaluation mode. The MC asserts dfi_db_train_en, transfers commands through the PHY, and evaluates dfi_db_train_resp. This is specialized to LRDIMM systems and is not part of the andesite design point.

## 10.8	PHY master during training

The PHY master interface can be used when the PHY needs to take control of the DRAM bus for extended calibration. The PHY asserts dfi_phymstr_req; the MC places the DRAM in IDLE or self- refresh and asserts dfi_phymstr_ack. The MC may still forward refresh commands, which the PHY relays to the DRAM. This is a DFI 4.0 feature and is not required for basic DDR4/LPDDR4 operation.

## 10.9	Training chip-select renaming

A subtle but important 3.1 -> 4.0 change is the removal of the _n suffix on training chip-select signals. DFI 4.0 uses dfi_phy_wrlvl_cs, dfi_phy_rdlvl_cs, dfi_phy_rdlvl_gate_cs and dfi_phy_calvl_cs. The polarity is the same as dfi_cs; the name simply no longer embeds the active-low assumption.

Source: DFI Specification v4.0 sections 3.6, 3.9, 3.10, 3.12, 4.12, 4.15, 4.16

# 11	Command Mapping

DFI does not define DRAM commands directly. It defines the wires that carry them. The MC must map each DRAM command onto the correct pattern of dfi_act_n, dfi_ras_n, dfi_cas_n, dfi_we_n, dfi_address, dfi_bank, dfi_bg, dfi_cid, dfi_cs and dfi_cke.

## 11.1	DDR4 command mapping

For DDR4, dfi_act_n distinguishes ACT from other commands. During ACT, dfi_ras_n, dfi_cas_n and dfi_we_n carry row address bits A16, A15 and A14. During non-ACT commands they carry the classic RAS/CAS/WE encoding.

Simplified DDR4 command mapping on DFI control wires

| DRAM command | dfi_act_n | dfi_ras_n | dfi_cas_n | dfi_we_n | dfi_address | dfi_bank | dfi_bg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACT | 0 | A16 | A15 | A14 | Row[13:0] | Bank | Bank group |
| RD | 1 | 1 | 0 | 1 | Column + A10=auto-precharge | Bank | Bank group |
| WR | 1 | 1 | 0 | 0 | Column + A10=auto-precharge | Bank | Bank group |
| PRE | 1 | 0 | 1 | 1 | A10=precharge-all | Bank | Bank group |
| REF | 1 | 0 | 0 | 1 | - | - | - |
| MRS | 1 | 0 | 0 | 0 | MR data | MR address | - |
| DES | 1 | 1 | 1 | 1 | - | - | - |

## 11.2	LPDDR4 CA command mapping

LPDDR4 uses a 6-bit single-data-rate CA bus delivered over two consecutive DRAM clock cycles. The command formatter must hold the command pins constant and drive the CA encoding onto dfi_address[5:0] across two phases. The exact two-cycle encoding is defined by JESD209-4; the andesite controller generates it through a dedicated CA submodule.

Because the CA bus is SDR and commands span two cycles, the MC cannot change CA arbitrarily on every phase. The CA valid qualifier marks both cycles of a command.

## 11.3	LPDDR2/LPDDR3 CA mapping

LPDDR2 and LPDDR3 use a 10-bit double-data-rate CA bus. The MC drives a 20-bit value on dfi_address; the PHY transmits bits [9:0] on the rising edge and bits [19:10] on the falling edge. Command pins (dfi_act_n, dfi_ras_n, dfi_cas_n, dfi_we_n, dfi_bank, dfi_bg, dfi_cid) are held constant.

## 11.4	Chip select and rank targeting

dfi_cs selects the rank for control commands. DFI 4.0 adds dfi_wrdata_cs and dfi_rddata_cs to separately identify the target rank on the data path. In a single-rank andesite configuration these are driven constant. In multi-rank or 3DS configurations they follow the rank selected by the command.

## 11.5	CA parity

DDR4 supports command/address parity. The MC generates parity on dfi_parity_in covering dfi_address, dfi_bank, dfi_bg, dfi_act_n, dfi_cas_n, dfi_cid, dfi_ras_n and dfi_we_n. The PHY returns any detected parity error on dfi_alert_n. The tparin_lat and tphy_paritylat parameters define the timing relationship.

Source: DFI Specification v4.0 sections 3.1, 4.2, 4.3, 4.11, 4.19

# 12	Write Carriage

The write data interface carries write data, mask or write DBI, and enable signals from MC to PHY. The PHY forwards them to the DRAM after the programmed write latency.

## 12.1	Write timing overview

For a matched-frequency system, the sequence is:

- MC issues write command on control interface.

- After tphy_wrlat DFI clocks, MC asserts dfi_wrdata_en.

- After tphy_wrdata DFI clocks from dfi_wrdata_en, first write data word appears on dfi_wrdata.

- dfi_wrdata_en stays asserted for the duration of the burst. dfi_wrdata_mask (or write DBI when enabled) travels with the data.

In a 1:4 frequency-ratio system, the control command is issued on one phase and the phase-suffixed write data (dfi_wrdata_p0..p3) carries four DRAM-clock words per DFI clock.

## 12.2	Write data enable

dfi_wrdata_en is ideally one bit per PHY data slice. All bits are identical in normal operation, so the MC-to-PHY width may differ. The enable must be asserted early enough to satisfy tphy_wrlat and must remain asserted for the entire burst.

## 12.3	Write mask and write DBI

dfi_wrdata_mask is one bit per byte of write data. When DBI is disabled, a 1 masks the associated byte. When DBI is enabled and phydbi_mode = 0, the same bus carries the DBI pattern: a 1 tells the DRAM to invert the corresponding byte before storing it. There is no separate dfi_wrdata_dbi signal in DFI 4.0.

When phydbi_mode = 1, the PHY computes and inserts DBI internally; the MC presents plain data and mask.

## 12.4	Write CRC

Write CRC may be generated in the MC (phycrc_mode = 0) or in the PHY (phycrc_mode = 1). When the MC generates CRC, the CRC word is appended to the write burst on dfi_wrdata. The PHY reports CRC errors on dfi_alert_n with timing bounded by tphy_crcmin_lat and tphy_crcmax_lat.

## 12.5	Write data chip select

dfi_wrdata_cs identifies the target rank for the write data. During write leveling it identifies the rank being trained. The PHY may need additional rank-switching gap (tphy_wrcsgap) when the target rank changes between writes.

Source: DFI Specification v4.0 sections 3.2, 4.4, 4.5, 4.5.2, 4.5.3

# 13	Read Carriage

The read data interface returns read data from PHY to MC. The MC must tell the PHY when to expect data by asserting dfi_rddata_en; the PHY returns data and per-slice valid signals after the read latency.

## 13.1	Read timing overview

For a matched-frequency system:

- MC issues read command on control interface.

- After trddata_en DFI clocks, MC asserts dfi_rddata_en for the expected burst duration.

- Within tphy_rdlat DFI clocks from dfi_rddata_en, the PHY asserts dfi_rddata_valid and drives dfi_rddata.

In a 1:4 frequency-ratio system, dfi_rddata_en is phase-suffixed and the returned read data appears as word-suffixed _w0.._w3.

## 13.2	Read data valid

dfi_rddata_valid is one bit per PHY data slice. It is the one-to-one response to dfi_rddata_en within tphy_rdlat. The MC uses it to qualify dfi_rddata. Stray valid signals without a matching dfi_rddata_en are dropped.

## 13.3	Read DBI

When DBI is enabled and phydbi_mode = 0, the PHY returns dfi_rddata_dbi alongside dfi_rddata. A 1 indicates that the corresponding byte was inverted by the DRAM and must be re-inverted by the receiver. When phydbi_mode = 1 the PHY handles inversion internally and dfi_rddata_dbi is not used.

## 13.4	Read data chip select

dfi_rddata_cs identifies the rank for which read data is expected. During read training it identifies the rank being trained. The PHY may need additional rank-switching gap (tphy_rdcsgap) when the target rank changes between reads.

## 13.5	Data not valid (LPDDR2)

dfi_rddata_dnv is an LPDDR2-specific per-byte signal that marks bytes which do not contain valid data. It has the same timing as dfi_rddata_valid.

Source: DFI Specification v4.0 sections 3.3, 4.4, 4.6, 4.6.2

# 14	Wire-Level Walkthroughs

These walkthroughs use a deliberately tiny teaching model so the DFI signals are visible without being lost in real memory geometry.

## 14.1	The simplified model

- One rank, one x8 data slice.

- 1:4 frequency ratio: four phases (_p0.._p3) per DFI clock, four read data words (_w0.._w3) per DFI clock.

- Two banks: B0, B1.

- One row per bank: R0.

- Burst length 8 (BL8), so a single column read returns 8 contiguous data words.

- Timing assumptions (illustrative only):

  - tphy_wrlat = 2 DFI clocks

  - tphy_wrdata = 1 DFI clock

  - trddata_en = 2 DFI clocks

  - tphy_rdlat = 3 DFI clocks

The model is unrealistic in size but correct in wire-level behavior.

## 14.2	Walkthrough 1: activate -> read -> data return

The MC wants to read from bank B0, row R0, column C0.

At DFI clock T0, phase p0, the MC drives the ACT command:

ACT command driven at T0/p0

| Signal | Value | Comment |
| --- | --- | --- |
| dfi_cs_p0 | 0 (active) | Chip select asserted |
| dfi_act_n_p0 | 0 | ACT form |
| dfi_ras_n_p0 | A16 value | Row bit, plus command encoding |
| dfi_cas_n_p0 | A15 value | Row bit, plus command encoding |
| dfi_we_n_p0 | A14 value | Row bit, plus command encoding |
| dfi_address_p0 | Row[13:0] | R0 |
| dfi_bank_p0 | B0 | Bank select |
| dfi_bg_p0 | BG0 | Bank group, if present |

After DRAM timing (tRCD) the MC issues the RD command at T4, phase p0:

RD command driven at T4/p0

| Signal | Value | Comment |
| --- | --- | --- |
| dfi_cs_p0 | 0 | Chip select asserted |
| dfi_act_n_p0 | 1 | Non-ACT form |
| dfi_ras_n_p0 | 1 | RAS high for read |
| dfi_cas_n_p0 | 0 | CAS low for read |
| dfi_we_n_p0 | 1 | WE high for read |
| dfi_address_p0 | Column + A10=0 | C0, no auto-precharge |
| dfi_bank_p0 | B0 | Same bank |

Two DFI clocks after the read command, at T6, the MC asserts dfi_rddata_en for the expected burst duration (BL8 = 8 DRAM clocks = 2 DFI clocks at 1:4):

Read enable assertion for BL8 at 1:4 ratio

| Clock | dfi_rddata_en_p0 | dfi_rddata_en_p1 | dfi_rddata_en_p2 | dfi_rddata_en_p3 |
| --- | --- | --- | --- | --- |
| T6 | 1 | 1 | 1 | 1 |
| T7 | 1 | 1 | 1 | 1 |

Three DFI clocks after dfi_rddata_en asserts, at T9, the PHY returns dfi_rddata_valid and dfi_rddata:

Read data return for BL8

| Clock | dfi_rddata_valid | dfi_rddata_w0 | dfi_rddata_w1 | dfi_rddata_w2 | dfi_rddata_w3 |
| --- | --- | --- | --- | --- | --- |
| T9 | 1 | D0 | D1 | D2 | D3 |
| T10 | 1 | D4 | D5 | D6 | D7 |

The read data words D0-D7 correspond to the 8-word burst starting at column C0.

## 14.3	Walkthrough 2: activate -> write with mask

The MC wants to write to bank B1, row R0, column C0, masking byte 0 of the first data beat.

At T0 the MC issues ACT for B1/R0 as in Walkthrough 1, with dfi_bank_p0 = B1.

After tRCD, at T4 the MC issues WR:

WR command driven at T4/p0

| Signal | Value | Comment |
| --- | --- | --- |
| dfi_cs_p0 | 0 | Chip select asserted |
| dfi_act_n_p0 | 1 | Non-ACT form |
| dfi_ras_n_p0 | 1 | RAS high for write |
| dfi_cas_n_p0 | 0 | CAS low for write |
| dfi_we_n_p0 | 0 | WE low for write |
| dfi_address_p0 | Column + A10=0 | C0, no auto-precharge |
| dfi_bank_p0 | B1 | Same bank |

Two DFI clocks after the write command, at T6, the MC asserts dfi_wrdata_en. One DFI clock later, at T7, write data appears on dfi_wrdata and the mask on dfi_wrdata_mask:

Write data and mask for BL8

| Clock | dfi_wrdata_en | dfi_wrdata_w0 | dfi_wrdata_mask_w0 |
| --- | --- | --- | --- |
| T7 | 1 | beat 0 (byte 0 masked) | 0x01 |
| T8 | 1 | beat 1 | 0x00 |

The mask value 0x01 means byte 0 of beat 0 is masked; the DRAM keeps its previous value for that byte.

## 14.4	Walkthrough 3: back-to-back reads with chip-select hold

In a single-rank system dfi_rddata_cs is driven inactive-high. The PHY still expects it to be valid so it can verify the data path. This walkthrough shows two back-to-back BL8 reads to the same open row.

At T0 the MC has already activated B0/R0. At T4 it issues RD C0. At T5 it issues RD C8 (next burst-aligned column). Because both reads target the same rank, dfi_rddata_cs remains constant.

Back-to-back reads at 1:4 ratio

| Clock | Command | dfi_rddata_en start | dfi_rddata_valid start |
| --- | --- | --- | --- |
| T4 | RD C0 | T6 | T9 |
| T5 | RD C8 | T7 | T10 |

The read enable windows for the two commands overlap. The PHY must return valid for each command independently; the dfi_rddata_valid windows are T9-T10 for the first read and T10-T11 for the second read. The MC’s read aligner unions or frames the enable windows to capture each burst correctly.

Source: DFI Specification v4.0 sections 3.1, 3.2, 3.3, 4.2, 4.5, 4.6

# 15	Timing Parameter Families

DFI 4.0 timing parameters are named dfi_t* or simply t* in the spec. They are divided into families by what they constrain and by which side of the interface defines them.

## 15.1	Naming convention

- t prefix: a timing parameter.

- Middle letters identify the interface: cmd, ctrl, phy_wr, phy_rd, rd, wr, lp, calvl, rdlvl, wrlvl, wdqlvl, ctrlupd, phyupd, dram_clk, init, par, phy_mstr, etc.

- _min/_max suffixes: minimum or maximum bounds.

- _type0.._type3: per-class timing for update and PHY master.

## 15.2	Who defines each parameter

Timing parameter definition ownership

| Source | What it controls | Example parameters |
| --- | --- | --- |
| MC | Controller-side scheduling commitments | tcmd_lat, tparin_lat, tctrlupd_*, trdlvl_resp, twrlvl_resp, tcalvl_resp, twdqlvl_resp, tlp_resp, tlp_wakeup, tinit_start |
| PHY | PHY-side propagation and response delays | tctrl_delay, tphy_wrlat, tphy_wrdata, tphy_rdlat, tdram_clk_disable, tinit_complete, tphy_paritylat |
| System | Agreed timing visible to both sides | twrdata_delay, trddata_en, tcalvl_data, tcalvl_strobe, tphy_crcmin_lat, tphy_crcmax_lat |

## 15.3	Parameter families

### 15.3.1	Control timing

tcmd_lat and tctrl_delay define how commands propagate from MC to DRAM.

### 15.3.2	Write data timing

tphy_wrlat, tphy_wrdata and twrdata_delay define when write data follows a write command and when it completes on the DRAM bus.

### 15.3.3	Read data timing

trddata_en, tphy_rdlat, tphy_rdcslat, tphy_rdcsgap define when the MC asserts read enable, when valid data returns, and how rank switches are handled.

### 15.3.4	Update timing

tctrlupd_interval, tctrlupd_min/max, tphyupd_resp, tphyupd_type0..3 bound the update handshake.

### 15.3.5	Status/init timing

tdram_clk_disable, tdram_clk_enable, tinit_start, tinit_complete, tinit_start_min, tinit_complete_min, tparin_lat, tphy_paritylat bound initialization, frequency change, clock disable and parity/alert timing.

### 15.3.6	Training timing

Each training operation has its own _en, _resp, _max, _rr, _ww, _rw parameters plus the CA-specific tcalvl_* set and the write-DQ-specific twdqlvl_* set.

### 15.3.7	Low-power timing

tlp_resp and tlp_wakeup bound the low-power handshake.

### 15.3.8	Error timing

terror_resp bounds how quickly the PHY reports an error after the offending transaction.

### 15.3.9	PHY master timing

tphymstr_resp, tphymstr_rfsh, tphymstr_type0..3 bound the PHY master handshake.

### 15.3.10	Disconnect timing

DFI 4.0 adds a pair of t*_disconnect and t*_disconnect_error parameters for every disconnectable handshake: update, training, and PHY master.

Source: DFI Specification v4.0 sections 3.0-3.14, 5.0

# 16	Frequency Ratio and Phasing

DFI supports three frequency ratios between the DFI clock and the DRAM clock: 1:1, 1:2 and 1:4. The dfi_freq_ratio signal encodes the ratio at initialization and is expected to remain constant.

## 16.1	Ratio definitions

DFI frequency ratios

| dfi_freq_ratio | Ratio | Meaning |
| --- | --- | --- |
| 00 | 1:1 | DFI clock and DRAM clock are the same frequency. |
| 01 | 1:2 | One DFI clock per two DRAM clocks. |
| 10 | 1:4 | One DFI clock per four DRAM clocks. |
| 11 | Reserved | Not used. |

At 1:4, one DFI clock covers four DRAM clocks. Control and write data signals are replicated into four phase-suffixed buses (_p0.._p3). Read data is returned as four word-suffixed buses (_w0.._w3).

## 16.2	What ratio does to scheduling

In a 1:1 system, the MC issues one command per DFI clock. In a 1:2 system, it may issue two commands per DFI clock (one per phase). In a 1:4 system, it may issue up to four commands per DFI clock. The PHY must accept commands on any phase.

The andesite design point uses 1:4. This means a single ACT-RD or ACT-WR sequence can be packed tightly: the ACT on phase p0 of one DFI clock and the RD/WR on phase p0 of a later DFI clock once tRCD (measured in DRAM clocks) is satisfied.

## 16.3	Phase replication rules

- Control interface: _pN suffix gives the value for phase N of the DFI PHY clock.

- Write data interface: _pN suffix gives the write data word for phase N.

- Read data interface: _wN suffix gives the read data word N.

- dfi_alert_n: _aN suffix gives the value for clock cycle N.

- Phase 0 / word 0 / cycle 0 suffixes are optional.

## 16.4	Timing parameter units in ratio systems

For matched-frequency systems, a DFI PHY clock equals a DFI clock. For frequency-ratio systems, timing parameters that mention “DFI PHY clocks” are measured in the higher-speed PHY clock domain (one quarter of the DFI clock period at 1:4), while parameters measured in “DFI clocks” are measured in the slower DFI clock domain.

This distinction matters for tinit_start, tinit_complete, tparin_lat and several others. The MC must use the correct domain when scheduling.

## 16.5	Ratio change

A frequency change can include a ratio change. The protocol uses dfi_init_start and dfi_init_complete. The MC must drive dfi_freq_ratio and dfi_frequency to their new values while dfi_init_start is asserted, and the PHY must be ready to operate at the new ratio after the handshake completes.

Source: DFI Specification v4.0 sections 3.5.4, 4.9, 4.10

# 17	Full Timing Parameter Table

This table lists every DFI 4.0 timing parameter from the inventory, grouped by sub-interface.

Complete DFI 4.0 timing parameter inventory

| Parameter | Sub-interface | Defined by | Description |
| --- | --- | --- | --- |
| tcmd_lat | Control | MC | DFI PHY clocks from dfi_cs assertion until associated CA signals are driven (LPDDR4 CS-to-CA). |
| tctrl_delay | Control | PHY | DFI clocks from a control signal change to the change reaching the DRAM interface. |
| tphy_crcmax_lat | Write Data | System | Max DFI PHY clocks from dfi_wrdata_en (CRC word cycle) to associated CRC error on dfi_alert_n. |
| tphy_crcmin_lat | Write Data | System | Min DFI PHY clocks for the same CRC error path. |
| tphy_wrcsgap | Write Data | PHY | Min additional DFI PHY clocks between commands when changing target rank on dfi_wrdata_cs. |
| tphy_wrcslat | Write Data | PHY | DFI PHY clocks from write command to assertion of associated dfi_wrdata_cs. |
| tphy_wrdata | Write Data | PHY | DFI PHY clocks from dfi_wrdata_en assertion to write data driven on dfi_wrdata. |
| tphy_wrlat | Write Data | PHY | DFI PHY clocks from write command on control interface to dfi_wrdata_en assertion. |
| twrdata_delay | Write Data | System | DFI clocks from dfi_wrdata_en assertion to write data transfer completion on DRAM bus. |
| tphy_rdcsgap | Read Data | PHY | Min additional DFI PHY clocks between commands when changing target rank on dfi_rddata_cs. |
| tphy_rdcslat | Read Data | PHY | DFI PHY clocks from read command to assertion of associated dfi_rddata_cs. |
| tphy_rdlat | Read Data | PHY | Max DFI PHY clocks from dfi_rddata_en assertion to each corresponding dfi_rddata_valid bit. |
| trddata_en | Read Data | System | DFI PHY clocks from read command on DFI to dfi_rddata_en assertion. |
| tctrlupd_interval | Update | MC | Max DFI clocks the MC may wait between dfi_ctrlupd_req assertions. |
| tctrlupd_max | Update | MC | Max DFI clocks dfi_ctrlupd_req can be asserted. |
| tctrlupd_min | Update | MC | Min DFI clocks dfi_ctrlupd_req must be asserted. |
| tphyupd_resp | Update | PHY | Max DFI clocks from dfi_phyupd_req assertion to dfi_phyupd_ack assertion. |
| tphyupd_type0 | Update | PHY | Max DFI clocks dfi_phyupd_req may stay asserted after ack for type 0. |
| tphyupd_type1 | Update | PHY | Max DFI clocks for type 1. |
| tphyupd_type2 | Update | PHY | Max DFI clocks for type 2. |
| tphyupd_type3 | Update | PHY | Max DFI clocks for type 3. |
| tdram_clk_disable | Status | PHY | DFI clocks from dfi_dram_clk_disable assertion until DRAM clock holds low at PHY-DRAM boundary. |
| tdram_clk_enable | Status | PHY | DFI clocks from dfi_dram_clk_disable de-assertion to first valid DRAM clock rising edge. |
| tinit_complete | Status | PHY | Max DFI clocks from dfi_init_start de-assertion to dfi_init_complete re-assertion during frequency change. |
| tinit_complete_min | Status | PHY | Min DFI clocks before dfi_init_complete can be driven after a previous command/training event. |
| tinit_start | Status | MC | DFI clocks from dfi_init_start assertion until PHY must de-assert dfi_init_complete to accept frequency change. |
| tinit_start_min | Status | PHY | Min DFI clocks before dfi_init_start can be driven after a previous command/training event. |
| tparin_lat | Status | MC | DFI PHY clocks between DFI command assertion and associated dfi_parity_in. |
| tphy_paritylat | Status | PHY | Max DFI clocks between dfi_parity_in and associated dfi_alert_n return (min 0x4). |
| terror_resp | Error | PHY | Max DFI clocks from affected DFI bus transaction(s) to dfi_error assertion. |
| tcalvl_ca_sel | Training | PHY | In frequency-ratio systems, defines dfi_calvl_ca_sel width generated by combining per-phase PHY inputs. |
| tcalvl_capture | Training | PHY | DFI clocks after calibration command on dfi_cs before dfi_calvl_capture pulse. |
| tcalvl_cc | Training | PHY | Min DFI clocks from one calibration command to the next. |
| tcalvl_cs_ca | Training | PHY | PHY DFI clocks from dfi_calvl_ca_sel assertion to dfi_cs. |
| tcalvl_data | Training | System | Min DFI PHY clocks dfi_calvl_data must be held for DRAM hold time. |
| tcalvl_en | Training | PHY | Min DFI clocks from dfi_calvl_en assertion to dfi_cke de-assertion. |
| tcalvl_max | Training | MC | Max DFI clocks MC waits for dfi_calvl_resp after asserting dfi_calvl_en. |
| tcalvl_resp | Training | MC | Max DFI clocks from dfi_calvl_req to MC asserting dfi_calvl_en. |
| tcalvl_strobe | Training | System | Min DFI PHY clocks from dfi_calvl_data to dfi_calvl_strobe for DRAM setup time. |
| trdlvl_en | Training | PHY | Min DFI clocks from dfi_rdlvl_en/dfi_rdlvl_gate_en assertion to first read/MRR command. |
| trdlvl_max | Training | MC | Max DFI clocks MC waits for dfi_rdlvl_resp after setting read training enable. |
| trdlvl_resp | Training | MC | Max DFI clocks from read training request to MC asserting corresponding enable. |
| trdlvl_rr | Training | PHY | Min DFI clocks between consecutive read-training read commands. |
| twdqlvl_en | Training | PHY | Min DFI clocks from any dfi_wdqlvl_en assertion to first write command. |
| twdqlvl_max | Training | MC | Max DFI clocks MC waits for dfi_wdqlvl_resp after asserting dfi_wdqlvl_en. |
| twdqlvl_resp | Training | MC | Max DFI clocks from dfi_wdqlvl_req to MC asserting dfi_wdqlvl_en. |
| twdqlvl_rw | Training | PHY | Min DFI clocks from last read in calibration sequence to first write of next set. |
| twdqlvl_ww | Training | PHY | Min DFI clocks inserted between write commands during write DQ training. |
| twrlvl_en | Training | PHY | Min DFI clocks from dfi_wrlvl_en assertion to first dfi_wrlvl_strobe assertion. |
| twrlvl_max | Training | MC | Max DFI clocks MC waits for dfi_wrlvl_resp after setting dfi_wrlvl_en. |
| twrlvl_resp | Training | MC | Max DFI clocks from dfi_wrlvl_req to MC asserting dfi_wrlvl_en. |
| twrlvl_ww | Training | PHY | Min DFI clocks from one dfi_wrlvl_strobe assertion to the next. |
| tlp_resp | Low Power | MC | Max DFI clocks from low-power request assertion to dfi_lp_ack assertion (min 1, max 7). |
| tlp_wakeup | Low Power | MC | Target max DFI clocks that dfi_lp_ack may stay asserted after request de-asserts (min 16). |
| tphy_db_train_en | DB Training | PHY | PHY DFI clocks after dfi_db_train_en asserts until first training command can be issued. |
| tphy_db_train_resp | DB Training | PHY | Max DFI clocks from PHY DQ input to dfi_db_train_resp output. |
| tphymstr_resp | PHY Master | MC | Max DFI clocks from dfi_phymstr_req assertion to dfi_phymstr_ack assertion. |
| tphymstr_rfsh | PHY Master | PHY | Max DFI clocks PHY needs to forward a refresh command from MC to DRAM while master. |
| tphymstr_type0 | PHY Master | PHY | Max DFI clocks dfi_phymstr_req may stay asserted after ack for type 0. |
| tphymstr_type1 | PHY Master | PHY | Max for type 1. |
| tphymstr_type2 | PHY Master | PHY | Max for type 2. |
| tphymstr_type3 | PHY Master | PHY | Max for type 3. |
| tcalvl_disconnect | Disconnect | PHY | Max clocks to disconnect CA training with dfi_disconnect_error=0. |
| tcalvl_disconnect_error | Disconnect | PHY | Same with dfi_disconnect_error=1. |
| tctrlupd_disconnect | Disconnect | PHY | Max clocks to disconnect a control update. |
| tctrlupd_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| tphyupd_disconnect | Disconnect | PHY | Max clocks to disconnect a PHY update. |
| tphyupd_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| tphymstr_disconnect | Disconnect | PHY | Max clocks to disconnect a PHY master request. |
| tphymstr_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| trdlvl_disconnect | Disconnect | PHY | Max clocks to disconnect read data eye training. |
| trdlvl_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| trdlvl_gate_disconnect | Disconnect | PHY | Max clocks to disconnect gate training. |
| trdlvl_gate_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| twdqlvl_disconnect | Disconnect | PHY | Max clocks to disconnect write DQ training. |
| twdqlvl_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| twrlvl_disconnect | Disconnect | PHY | Max clocks to disconnect write leveling. |
| twrlvl_disconnect_error | Disconnect | PHY | Same with error disconnect. |
| tgeardown_delay | Geardown | PHY | Delay from dfi_geardown_en assertion until PHY is ready to receive commands. |

## 17.1	Programmable parameters

Complete DFI 4.0 programmable parameter inventory

| Parameter | Sub-interface | Defined by | Description |
| --- | --- | --- | --- |
| phycrc_mode | Write Data | PHY | 0 = CRC in MC; 1 = CRC in PHY. |
| phydbi_mode | Write/Read Data | PHY | 0 = DBI in MC; 1 = DBI in PHY. |
| dfidata_bit_enable | Status | MC/PHY | Valid data bits on dfi_wrdata and dfi_rddata. |
| phyfreq_range | Status | PHY | Number of dfi_frequency values supported (1-32). |
| mcrdlvl_slice_group | Training | MC | Per-slice grouping of dfi_rdlvl_en bits. |
| phyrdlvl_slice_group | Training | PHY | PHY-side counterpart of mcrdlvl_slice_group. |
| phycalvl_en | Training | PHY | 1 = DFI CA training enabled. |
| phyrdlvl_en | Training | PHY | PHY supports read data eye training. |
| phyrdlvl_gate_en | Training | PHY | PHY supports gate training. |
| phywdqlvl_bst | Training | PHY | Consecutive write bursts per write DQ training sequence. |
| phywrlvl_en | Training | PHY | PHY supports write leveling. |
| syswrlvl_strobe_num | Training | MC | Minimum clocks dfi_wrlvl_strobe asserts per request. |
| syscs_state | PHY Master | MC/PHY | Per-CS active/inactive state. |
| phychannel_en | LPDDR4 Channels | System/PHY | Enabled channels in LPDDR4 combined mode. |

Source: DFI Specification v4.0 sections 3.1-3.14, Tables 6, 13, 14, 16, 17, 19, 20, 22-26

# 18	Low-Power Handshake and Update Mechanism

## 18.1	Low-power control

The low-power control interface lets the MC request that the PHY enter a low-power state. It is split into separate control and data requests (dfi_lp_ctrl_req and dfi_lp_data_req) so the two sides can power down independently.

The MC asserts a request and holds dfi_lp_wakeup constant while awaiting acknowledge. The PHY may acknowledge (dfi_lp_ack within tlp_resp) or ignore the request. If acknowledged, the PHY de-asserts dfi_lp_ack within tlp_wakeup cycles after the request de-asserts.

The andesite design point exposes dfi_lp_ctrl_req and dfi_lp_data_req but does not consume acknowledges; an unacknowledged request times out and reports rather than blocking. This is the same dormant-pair disposition used by its predecessor.

### 18.1.1	Wakeup encoding

Low-power wakeup encoding

| dfi_lp_wakeup | Target max cycles |
| --- | --- |
| 0000 | 16 |
| 0001 | 32 |
| … | doubling each step |
| 1110 | 262144 |
| 1111 | Unlimited |

## 18.2	Update mechanism

### 18.2.1	MC-initiated update

The MC asserts dfi_ctrlupd_req for between tctrlupd_min and tctrlupd_max cycles. The PHY may acknowledge with dfi_ctrlupd_ack before tctrlupd_min expires. While the request is asserted, the DFI bus stays idle.

DFI 4.0 adds a requirement that a dfi_ctrlupd_req/dfi_ctrlupd_ack handshake complete immediately before any self-refresh exit command. This guarantees a clean re-entry point.

### 18.2.2	PHY-initiated update

The PHY asserts dfi_phyupd_req and drives dfi_phyupd_type to select one of four time classes. The MC must acknowledge with dfi_phyupd_ack within tphyupd_resp. The PHY holds dfi_phyupd_req for up to tphyupd_typeX cycles after the acknowledge. The DFI bus idles while the request is asserted.

### 18.2.3	DFI idle definition

The DFI bus is considered idle when no control, write data, read data, or update activity is in progress. Some training and low-power sequences require the bus to be idle before they begin.

Source: DFI Specification v4.0 sections 3.4, 3.7, 4.7, 4.8, 4.13

# 19	Refresh Interaction and Frequency Change

## 19.1	Refresh interaction

DFI does not define refresh commands specially. A refresh is just another command on the control interface: dfi_ras_n and dfi_cas_n low, dfi_we_n high, with dfi_address and dfi_bank set according to the memory type. The MC is responsible for scheduling refresh at the required rate and for precharging all banks before issuing an all-bank refresh.

The PHY may request control of the bus for its own refresh-related purposes through the PHY master interface. While the PHY is master, the MC may still forward refresh commands, which the PHY relays to the DRAM within tphymstr_rfsh cycles.

### 19.1.1	Refresh during training

DFI 4.0 allows refreshes to be issued during some training sequences, provided the DRAM timing requirements are met. The MC must not issue normal read or write traffic to slices that are actively being trained.

## 19.2	Frequency change

DFI 4.0 reuses the init handshake (dfi_init_start/dfi_init_complete) for frequency change. The MC also uses dfi_frequency to tell the PHY the target frequency encoding.

### 19.2.1	Acknowledged frequency change

- MC drives new dfi_frequency and asserts dfi_init_start.

- PHY accepts by de-asserting dfi_init_complete within tinit_start cycles.

- MC holds dfi_init_start until the frequency switch is complete.

- PHY re-asserts dfi_init_complete within tinit_complete cycles after dfi_init_start de-asserts.

### 19.2.2	Not-acknowledged frequency change

If the PHY does not de-assert dfi_init_complete within tinit_start cycles, the MC must abort the request and release dfi_init_start. The PHY must not de-assert dfi_init_complete after tinit_start expires.

### 19.2.3	Frequency indicator

dfi_frequency is a 5-bit value supporting up to 32 encodings. The mapping from encoding to clock frequency is PHY/system-defined. The phyfreq_range parameter tells the MC how many encodings the PHY supports. The MC must never drive an unsupported value.

dfi_frequency can change when dfi_init_start is low and should be ignored at that time. Once dfi_init_start asserts, dfi_frequency must be legal and stable.

Source: DFI Specification v4.0 sections 3.5.5, 3.5.6, 4.10

# 20	Revision Deltas and the andesite Tie-In

## 20.1	3.1 -> 4.0 revision deltas

DFI 4.0 is the largest single revision step in the DFI family. The changes fall into several groups.

### 20.1.1	DDR4/LPDDR4 data-path additions

Data-path deltas

| Change | DFI 4.0 handling |
| --- | --- |
| Read/write data chip selects | New dfi_rddata_cs and dfi_wrdata_cs signals let the PHY compensate data-path timing per rank. |
| Read DBI | New dedicated dfi_rddata_dbi bus. |
| Write DBI | Reuses dfi_wrdata_mask; no separate dfi_wrdata_dbi exists. |
| Write DQ training | New dfi_wdqlvl_* signals and timing parameters. |

### 20.1.2	Command and control additions

Control deltas

| Change | DFI 4.0 handling |
| --- | --- |
| Bank groups | dfi_bg added in 3.0; retained in 4.0. |
| ACT_n | dfi_act_n added in 3.0; retained. |
| CA parity | dfi_parity_in and dfi_alert_n retained from 3.0. |
| Chip ID for 3DS | dfi_cid retained from 3.0. |
| LPDDR4 SDR CA | 6-bit CA mapped onto dfi_address over two cycles. |
| tcmd_lat | New LPDDR4 CS-to-CA delay parameter. |

### 20.1.3	System-level additions

System-level deltas

| Change | DFI 4.0 handling |
| --- | --- |
| PHY master interface | New dfi_phymstr_* handshake. |
| Disconnect protocol | New dfi_disconnect_error plus disconnect timing parameters. |
| Frequency indicator | New dfi_frequency signal and phyfreq_range parameter. |
| Geardown mode | New dfi_geardown_en signal and tgeardown_delay parameter. |
| LPDDR4 channels | New phychannel_en parameter. |
| DB training | New dfi_db_train_* interface for LRDIMM. |
| Inactive CS support | New syscs_state parameter. |

### 20.1.4	Training changes

Training deltas

| Change | DFI 4.0 handling |
| --- | --- |
| Per-slice read leveling | dfi_rdlvl_en/req and dfi_rdlvl_gate_en/req became per-slice; grouping via mcrdlvl_slice_group/phyrdlvl_slice_group. |
| Training chip-select rename | _cs_n suffix removed: dfi_phy_wrlvl_cs, dfi_phy_rdlvl_cs, dfi_phy_rdlvl_gate_cs, dfi_phy_calvl_cs. |
| PHY Independent Mode | Replaces v3.1 dfi_phylvl_req_cs_n/dfi_phylvl_ack_cs_n. |
| CA training additions | dfi_calvl_ca_sel, dfi_calvl_data/done/result/strobe, and associated timing parameters. |
| Write leveling strobe | Now per-slice with assert count via syswrlvl_strobe_num. |

### 20.1.5	Removed signal

dfi_data_byte_disable was removed in DFI 4.0 and replaced by the dfidata_bit_enable programmable parameter.

## 20.2	The andesite tie-in

The andesite DDR4/LPDDR4 memory controller targets DFI 4.0 because its predecessor left the entire 3.1 -> 4.0 delta as “deliberately unimplemented.” The andesite design implements that delta.

### 20.2.1	Inherited surface

- The async-FIFO datapath, bubble-free, at 1:4 frequency ratio.

- dfi_init_start/dfi_init_complete, dfi_error/dfi_error_info, dfi_reset_n.

- dfi_lp_ctrl_req/dfi_lp_data_req presented but unacknowledged-by-default.

### 20.2.2	Implemented 4.0 deltas

The andesite HAS Chapter 4.1 and MAS pin table list the implemented changes:

andesite implementation of DFI 4.0 deltas

| DFI 4.0 feature | andesite block | Tie-in document |
| --- | --- | --- |
| dfi_act_n, dfi_bg | cmd_formatter (modified) | docs/andesite_has/ch04_interfaces/01_dfi_v40.md |
| dfi_parity_in, dfi_alert_n | cmd_formatter + new alert handling | docs/andesite_has/ch04_interfaces/01_dfi_v40.md |
| dfi_wrdata_mask as write DBI, dfi_rddata_dbi | dfi_datapath | docs/andesite_has/ch04_interfaces/01_dfi_v40.md |
| dfi_geardown_en | init_sequencer | docs/andesite_has/ch04_interfaces/01_dfi_v40.md |
| Training (dfi_rdlvl_*, dfi_calvl_*, dfi_wrlvl_*) | training block, rdlvl_ifc, ca_train_ifc, wrlvl_ifc | docs/andesite_has/ch04_interfaces/01_dfi_v40.md, docs/andesite_mas/ch03_interfaces/01_dfi40_pins.md |
| LPDDR4 CA bus | cmd_formatter LPDDR4 CA submodule | docs/andesite_has/ch04_interfaces/01_dfi_v40.md |

### 20.2.3	Study corrections recorded in the area docs

The andesite HAS and MAS record several study corrections made on 2026-10-04:

- The v3.1 per-CS training handshake dfi_phylvl_req_cs_n/dfi_phylvl_ack_cs_n does not exist in DFI 4.0; it is replaced by PHY Independent Mode.

- Training chip selects dropped the _n suffix in 4.0.

- There is no dfi_wrdata_dbi pin; write DBI reuses dfi_wrdata_mask.

- dfi_ca_capture was wrong in name and direction; the correct signal is dfi_calvl_capture (MC-to-PHY).

- dfi_init is not a DFI signal; the spec uses dfi_init_start and dfi_init_complete.

### 20.2.4	Verification posture

The andesite verification strategy targets the DFI 4.0 boundary using the in-house CocoTB framework DFI component. The BFM is DFI 4.0-partial; gaps at the andesite design point (LPDDR4 CA map, design-point JEDEC timing CSVs, gear-down validation, per-slice read leveling, CA VREF sweep feedback) are listed in docs/andesite_has/ch06_integration/01_verification_open.md. The verification plan exercises init sequences, gear-down entry, CA parity, ODT, FGR, LPDDR4 CA encoding, training protocols, and the dormant low-power pair.

Source: DFI Specification v4.0 revision history and sections 3.6.8, 3.12, 3.13, 3.14; andesite HAS ch04_interfaces/01_dfi_v40.md, andesite MAS ch03_interfaces/01_dfi40_pins.md, andesite HAS ch06_integration/01_verification_open.md