Memory Notes

Simplified DFI 2.1 Simplified edition - DFI Specification v2.1.1 (Denali Software, 17 June 2010)

October 4, 2026

Table of Contents

1 Simplified Dfi 2 1 Index	11

2 Introduction	12

2.1 What DFI is	12

2.2 Why the MC/PHY split exists	13

2.3 Edition lineage	14

2.4 Memory types covered	15

2.5 Optional features	16

3 Organization	17

3.1 Sub-interface map	17

3.2 Clocking	18

3.3 Frequency ratio	19

3.4 Slices, bytes, and data ordering	20

3.5 LPDDR2 command/address carve-out	21

4 Acronyms	22

5 Control Interface	23

5.1 Control signals	24

5.2 LPDDR2 CA mapping on dfi_address	26

5.3 Defaults and idle values	27

5.4 Timing	28

6 Write Data Interface	29

6.1 Write data signals	30

6.2 Timing model	31

7 Read Data Interface	32

7.1 Read data signals	33

7.2 Timing model	34

8 Update Interface	35

8.1 Update signals	36

8.2 MC-initiated update	37

8.3 PHY-initiated update	38

8.4 Concurrent requests	39

8.5 Update timing parameters	40

9 Status Interface	41

9.1 Status signals	42

9.2 Initialization use	44

9.3 Frequency change use	45

9.4 Clock disable	46

9.5 Parity	47

10 Training Interface	48

10.1 Training support modes	49

10.2 Read leveling signals	50

10.3 Write leveling signals	52

10.4 Key timing parameters	53

11 Low Power Control Interface	54

11.1 Low power signals	55

11.2 Wakeup encoding	56

11.3 Handshake	57

12 Initialization and Training	58

12.1 Reset and initialization	58

12.1.1 Signals held at default until init_complete	58

12.1.2 LPDDR2 special rule	58

12.1.3 init_start handshake	58

12.1.4 End-of-init ctrlupd_req	59

12.1.5 Initialization timing parameters	59

12.2 Training operations	60

12.2.1 Read leveling	60

12.2.2 Write leveling	60

12.2.3 Responsibility modes	60

12.2.4 Training is not required at init	60

13 Command Mapping	62

13.1 DDR2 command encoding	63

13.2 LPDDR2 command encoding	64

14 Write Carriage	65

14.1 Command to enable relationship	66

14.2 Enable to data relationship	67

14.3 Contiguous and interrupted writes	68

14.4 Frequency ratio packing	69

15 Read Carriage	70

15.1 Command to enable relationship	71

15.2 Enable to data relationship	72

15.3 Contiguous reads	73

15.4 Frequency ratio packing	74

16 Wire-Level Walkthroughs	75

16.1 Teaching model	76

16.2 Walkthrough 1: Activate to Read	77

16.3 Walkthrough 2: Activate to Write	78

16.4 What the traces show	79

17 Parameter Families and Ratio Phasing	80

17.1 Parameter families	80

17.1.1 Who defines what	80

17.1.2 What the families constrain	81

17.1.3 Constants, maxima, and fixed values	81

17.2 Ratio and phasing	82

17.2.1 1:1 matched frequency	82

17.2.2 1:2 frequency ratio	82

17.2.3 1:4 frequency ratio	82

17.2.4 Impact on scheduling	82

17.2.5 Ratio selection	83

18 Parameter Tables	84

18.1 Control	85

18.2 Write data	86

18.3 Read data	87

18.4 Update	88

18.5 Status	89

18.6 Training	90

18.7 Low power	92

19 System Behavior	93

19.1 Low power control	94

19.1.1 Entry sequence	94

19.1.2 Wakeup time adjustment	94

19.1.3 Exit sequence	94

19.1.4 Relationship to DRAM power modes	94

19.2 Update mechanism	95

19.2.1 MC-initiated update	95

19.2.2 PHY-initiated update	95

19.2.3 Why the bus must idle	95

19.2.4 Concurrent updates	95

19.3 Frequency change	96

19.3.1 Acknowledged frequency change	96

19.3.2 Frequency ratio reset	96

19.3.3 Pumice does not implement it	96

20 Revision Deltas	97

20.1 What DFI 2.1 added versus DFI 2.0	97

20.2 What later revisions changed	98

21 DFI 2.1 in the Pumice DDR2/LPDDR2 Controller	99

21.1 Sub-interface support	100

21.2 Clock-domain crossing	101

21.3 Command path	102

21.4 Write and read serializers	103

21.5 Known gaps and future work	104

List of Figures

No figures in this document.

List of Tables

Table 1: DFI edition lineage	14

Table 2: DFI 2.1 sub-interface summary	17

Table 3: Acronyms used in this book	22

Table 4: DFI 2.1 control interface signals	24

Table 5: LPDDR2 CA packing into dfi_address	26

Table 6: DFI 2.1 write data interface signals	30

Table 7: Write data timing parameters	31

Table 8: DFI 2.1 read data interface signals	33

Table 9: Read data timing parameters	34

Table 10: DFI 2.1 update interface signals	36

Table 11: Update interface timing parameters	40

Table 12: DFI 2.1 status interface signals	42

Table 13: Training mode encodings	49

Table 14: Read leveling signals	50

Table 15: Write leveling signals	52

Table 16: Training timing parameters	53

Table 17: DFI 2.1 low power control interface signals	55

Table 18: Low power wakeup encoding	56

Table 19: Initialization timing parameters	59

Table 20: Training responsibility modes	60

Table 21: DDR2 command encoding on DFI control signals	63

Table 22: DFI 2.1 LPDDR2 CA mapping	64

Table 23: LPDDR2 rising-edge command opcode	64

Table 24: Timing parameter families and ownership	80

Table 25: Control timing parameter	85

Table 26: Write data timing parameters	86

Table 27: Read data timing parameters	87

Table 28: Update timing parameters	88

Table 29: Status timing parameters	89

Table 30: Training timing parameters	90

Table 31: Low power timing parameters	92

Table 32: DFI 2.1 additions over DFI 2.0	97

Table 33: Brief preview of later DFI revisions	98

Table 34: Pumice DFI 2.1 sub-interface support	100

List of Waveforms

No waveforms in this document.

# 1	Simplified Dfi 2 1 Index

Generated: 2026-10-20

# 2	Introduction

## 2.1	What DFI is

The DDR PHY Interface (DFI) is the wire-level contract between a DDR memory controller (MC) and a DDR physical interface (PHY). Version 2.1.1, published by Denali Software in June 2010, defines the signals, signal relationships, and timing parameters needed to move commands, write data, and read data across that boundary for DDR1, LPDDR1, DDR2, LPDDR2, and DDR3 memory devices.

The DFI deliberately does not specify everything inside the MC or the PHY. It only standardizes the seam: what control information the MC hands off, how the PHY reports data back, how the two sides agree that initialization is done, and how optional features such as frequency ratio, low power, and training are signaled. Anything else, test logic, extra calibration, PHY-specific gearing, is left to the implementation.

## 2.2	Why the MC/PHY split exists

A modern memory subsystem usually has three planes: the controller (scheduling, bank state, refresh), the PHY (clock generation, pads, delay lines, serializers), and the DRAM devices. The controller wants a stable programming model; the PHY wants freedom to choose pad timing, delay lines, and clocking. DFI sits between them so that a controller team and a PHY team can work against a shared interface instead of sharing every implementation detail.

The DFI clock can come from either side or from the system. The only hard rule is that every DFI signal must be driven from a register clocked by a rising edge of the DFI clock. How the other side receives it, synchronously or asynchronously, is an implementation choice.

## 2.3	Edition lineage

DFI edition lineage

| Version | Year | What changed |
| --- | --- | --- |
| DFI 1.0 | 2007 | Initial release; DDR1/LPDDR1/DDR2 focus |
| DFI 2.0 | 2007-2008 | DDR3 support, read leveling, write leveling |
| DFI 2.1 | 2008-2010 | LPDDR2 support, low power, frequency ratio, parity, frequency change, update interface, tphy_wrdata |

DFI 2.1 is backward compatible with 2.0 at the signaling level, but a 2.1 MC or PHY may offer optional features that a 2.0 partner does not understand. This book focuses on the 2.1 surface and calls out the optional additions explicitly.

## 2.4	Memory types covered

DFI 2.1 lists support for DDR1, LPDDR1, DDR2, LPDDR2, and DDR3. Not every signal applies to every type. For example, dfi_reset_n is only for DDR3, dfi_odt is for DDR2 and DDR3, and the LPDDR2 CA bus rides on dfi_address while dfi_bank, dfi_ras_n, dfi_cas_n, and dfi_we_n are held idle.

## 2.5	Optional features

Several 2.1 mechanisms are optional and not required for basic DFI compliance:

- Frequency ratio (1:2 or 1:4 MC:PHY clock ratio)

- Low power control handshaking

- Command parity and dfi_parity_error

- Frequency change protocol

- PHY-initiated update (dfi_phyupd_*)

- Read and write leveling training

A minimal DDR2 controller can ignore most of these and still be DFI compliant. The pumice DDR2/LPDDR2 controller takes exactly that minimal stance: it drives the command, write-data, read-data, and init-status sub-interfaces and leaves update, training, low power, and frequency change unimplemented in its first revision.

Source: DFI Specification v2.1.1 sections 1.0, 2.0

# 3	Organization

## 3.1	Sub-interface map

DFI 2.1 partitions the MC/PHY boundary into seven sub-interfaces. Each group has its own signals, direction, and timing parameters.

DFI 2.1 sub-interface summary

| Sub-interface | MC -> PHY | PHY -> MC | Purpose |
| --- | --- | --- | --- |
| Control | address, bank, RAS/CAS/WE, CKE, CS, ODT, reset | - | DRAM command reflection |
| Write data | write data, enable, mask | - | Write payload carriage |
| Read data | read enable | read data, valid, DNV | Read payload return |
| Update | ctrlupd_req | ctrlupd_ack, phyupd_req/type, phyupd_ack | Idle-window handshakes |
| Status | data_byte_disable, dram_clk_disable, freq_ratio, init_start, parity_in | init_complete, parity_error | Init and capability signaling |
| Training | rdlvl_, wrlvl_ control | rdlvl_, wrlvl_ response | Read/write leveling |
| Low power | lp_req, lp_wakeup | lp_ack | Power-down opportunity |

The Control, Write data, and Read data sub-interfaces carry ordinary traffic. Update, Status, Training, and Low power are housekeeping groups that let the PHY pause traffic, report init completion, train delays, or enter low power states.

## 3.2	Clocking

Every DFI signal is driven from a register on the rising edge of the DFI clock. The specification does not fix where that clock comes from, only that both sides agree on it. In a matched-frequency system the DFI clock and the PHY clock are the same. In a frequency-ratio system the PHY runs at a multiple of the MC clock and the DFI clock is the slower MC clock, while individual phases are sampled on the faster PHY clock.

## 3.3	Frequency ratio

DFI 2.1 supports three ratios: 1:1, 1:2, and 1:4. The optional dfi_freq_ratio signal reports the ratio as 2'b00, 2'b01, or 2'b10; 2'b11 is reserved.

When the ratio is not 1:1, the Control interface, Write data interface, and read data enable are replicated into phase-specific copies with a _pN suffix. For example, a 1:2 system can drive dfi_address_p0 and dfi_address_p1 in a single MC clock, letting the PHY accept two commands per DFI clock. The MC may place a command on any phase; the PHY must accept commands on all phases.

Read data, read data valid, and read data not valid are different. They are replicated into DFI data word copies with a _wN suffix instead of _pN. A 1:2 ratio has _w0 and _w1; a 1:4 ratio has _w0 through _w3. The suffix for phase 0 or word 0 is optional.

## 3.4	Slices, bytes, and data ordering

DFI data signals are organized by byte lanes. dfi_wrdata_mask[0] masks dfi_wrdata[7:0], dfi_wrdata_mask[1] masks dfi_wrdata[15:8], and so on. The same ordering applies to dfi_rddata_dnv and dfi_data_byte_disable. The dfi_wrdata_en and dfi_rddata_en widths are ideally one bit per memory data slice, with bit 0 covering the lowest data segment.

## 3.5	LPDDR2 command/address carve-out

LPDDR2 does not use dfi_bank, dfi_ras_n, dfi_cas_n, or dfi_we_n. Its 10-bit Command/Address (CA) bus is mapped onto dfi_address as a flat 20-bit word: rising-edge CA bits on dfi_address[9:0] and falling-edge CA bits on dfi_address[19:10]. The PHY splits that word into the two DDR CA cycles.

Source: DFI Specification v2.1.1 sections 2.0, 3.0, 3.1, 4.7

# 4	Acronyms

Acronyms used in this book

| Term | Meaning |
| --- | --- |
| CA | Command/Address bus (used by LPDDR2) |
| CS | Chip select |
| CKE | Clock enable |
| DFI | DDR PHY Interface |
| DDR | Double data rate SDRAM (generic) |
| DNV | Data not valid |
| DRAM | Dynamic random-access memory |
| MC | Memory controller |
| MRW | Mode register write (LPDDR2) |
| MRR | Mode register read (LPDDR2) |
| ODT | On-die termination |
| PHY | Physical interface (pads, clocks, delay lines) |
| RAS/CAS/WE | Row/column/write strobe signals |
| RD/WR | Read/write commands |
| REF | Refresh command |
| PRE | Precharge command |
| ACT | Activate command |

Source: DFI Specification v2.1.1 Glossary (Section 6.0)

# 5	Control Interface

The control interface carries the command and address information that the MC wants the PHY to reflect to the DRAM devices. The PHY may delay these signals by tctrl_delay cycles, but it must preserve their timing relationship and bit ordering at the PHY-DRAM boundary.

In a frequency-ratio system every control signal becomes phase-specific: dfi_address_pN, dfi_bank_pN, dfi_cas_n_pN, and so on. Phase 0 may drop the suffix. The MC may issue commands on any phase; the PHY must accept commands on all phases.

## 5.1	Control signals

DFI 2.1 control interface signals

| Signal | Direction | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_address | MC -> PHY | DFI address width | no default | Address bus for all control commands. For LPDDR2 it carries the 20-bit CA word. |
| dfi_bank | MC -> PHY | DFI bank width | no default | Bank select for DDR1/LPDDR1/DDR2/DDR3; idle for LPDDR2. |
| dfi_cas_n | MC -> PHY | DFI control width | 0x1 | Column-address strobe. Idle for LPDDR2. |
| dfi_cke | MC -> PHY | DFI chip select width | 0x0 or 0x1 | Clock enable to DRAMs; must be driven in all phases for ratio systems. |
| dfi_cs_n | MC -> PHY | DFI chip select width | 0x1 | Chip select, active low. |
| dfi_odt | MC -> PHY | DFI chip select width | 0x0 | On-die termination control; required for DDR2/DDR3. |
| dfi_ras_n | MC -> PHY | DFI control width | 0x1 | Row-address strobe. Idle for LPDDR2. |
| dfi_reset_n | MC -> PHY | DFI chip select width | 0x0 | DRAM reset; required only for DDR3. |
| dfi_we_n | MC -> PHY | DFI control width | 0x1 | Write enable. Idle for LPDDR2. |

## 5.2	LPDDR2 CA mapping on dfi_address

For LPDDR2 the 10-bit CA bus is sent on both edges of the clock. DFI 2.1 packs the two 10-bit halves into one dfi_address word per command cycle.

LPDDR2 CA packing into dfi_address

| dfi_address bit | CA bit / edge |
| --- | --- |
| 9:0 | CA[9:0] on the rising edge |
| 19:10 | CA[9:0] on the falling edge |

So dfi_address[0] is CA0 on the rising edge, dfi_address[10] is CA0 on the falling edge, and so on up to dfi_address[19]. The minimum dfi_address width for LPDDR2 is therefore 20 bits. During initialization the LPDDR2 CA bus must be driven with a NOP until dfi_init_complete asserts.

## 5.3	Defaults and idle values

Most control signals default to the DRAM idle state: RAS, CAS, and WE are high, chip select is high, ODT is low, and reset is low. CKE is memory-dependent; most parts expect CKE low at reset, but Mobile DDR parts may expect it high. For LPDDR2 the classic RAS/CAS/WE/bank signals are unused and must remain at their idle values while dfi_address carries the CA-encoded command.

## 5.4	Timing

The only control-interface timing parameter is tctrl_delay, defined by the PHY. It specifies how many DFI clock cycles elapse between an assertion or de-assertion on the DFI control signals and the same transition appearing at the PHY-DRAM boundary. If the DFI clock and memory clock are not phase aligned, this value is rounded up to the next integer.

Source: DFI Specification v2.1.1 sections 3.1, 4.2

# 6	Write Data Interface

The write data interface moves write payloads from the MC to the PHY. It consists of the data bus dfi_wrdata, the byte mask dfi_wrdata_mask, and the enable dfi_wrdata_en. The interface is unidirectional from MC to PHY.

## 6.1	Write data signals

DFI 2.1 write data interface signals

| Signal | Direction | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_wrdata | MC -> PHY | DFI data width | no default | Write data payload. Valid while dfi_wrdata_en is asserted. |
| dfi_wrdata_en | MC -> PHY | DFI data enable width | 0x0 | Enable for write data and mask; asserted tphy_wrlat cycles after the write command and tphy_wrdata cycles before data. |
| dfi_wrdata_mask | MC -> PHY | DFI data width / 8 | no default | Per-byte mask; bit N masks dfi_wrdata[8N+7:8N]. |

In a frequency-ratio system these signals become dfi_wrdata_pN, dfi_wrdata_en_pN, and dfi_wrdata_mask_pN. Phase 0 may drop the suffix. The write data stream is still expected to begin with phase 0 unless the MC chooses to send it unaligned and uses tphy_wrdelay.

## 6.2	Timing model

Three parameters govern write data timing:

Write data timing parameters

| Parameter | Meaning |
| --- | --- |
| tphy_wrlat | Cycles from write command on control interface to dfi_wrdata_en assertion. |
| tphy_wrdata | Cycles from dfi_wrdata_en assertion to first dfi_wrdata/dfi_wrdata_mask beat. |
| tphy_wrdelay | Optional PHY-side delay between enable and data in frequency-ratio systems. |

dfi_wrdata_en is asserted tphy_wrlat cycles after the write command. It must remain asserted for the entire contiguous write data burst. The actual data and mask appear tphy_wrdata cycles after dfi_wrdata_en asserts. For back-to-back writes the enable can stay asserted across command boundaries.

The DFI 1.0/2.0 model effectively fixed tphy_wrdata to 1. DFI 2.1 made it a programmable parameter so PHYs that need earlier notification of pending data can request a larger value.

Source: DFI Specification v2.1.1 sections 3.2, 4.3

# 7	Read Data Interface

The read data interface returns read payloads from the PHY to the MC. It consists of the read enable from the MC (dfi_rddata_en) and the returned data, valid flag, and data-not-valid flag from the PHY.

## 7.1	Read data signals

DFI 2.1 read data interface signals

| Signal | Direction | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_rddata_en | MC -> PHY | DFI data enable width | 0x0 | Tells the PHY to expect read data; asserted trddata_en cycles after the read command. |
| dfi_rddata | PHY -> MC | DFI data width | no default | Returned read data. |
| dfi_rddata_valid | PHY -> MC | DFI read data valid width | 0x0 | Asserted with valid read data for each cycle of the burst. |
| dfi_rddata_dnv | PHY -> MC | DFI data width / 8 | 0x0 | Per-byte data-not-valid flag; required for LPDDR2. |

In a frequency-ratio system dfi_rddata_en becomes dfi_rddata_en_pN, while dfi_rddata, dfi_rddata_valid, and dfi_rddata_dnv become dfi_rddata_wN, dfi_rddata_valid_wN, and dfi_rddata_dnv_wN. A 1:2 ratio has two word copies; a 1:4 ratio has four. The word 0 suffix is optional.

## 7.2	Timing model

Read data timing parameters

| Parameter | Meaning |
| --- | --- |
| trddata_en | Cycles from read command on control interface to dfi_rddata_en assertion. |
| tphy_rdlat | Maximum cycles from dfi_rddata_en assertion to first dfi_rddata_valid assertion. |

The MC asserts dfi_rddata_en trddata_en cycles after the read command and holds it for the duration of contiguous read data it expects. The PHY may return data any time up to tphy_rdlat cycles after the enable; it is allowed to return data earlier by asserting dfi_rddata_valid sooner. When valid is asserted the entire DFI read data word must be valid.

dfi_rddata_dnv is used only for LPDDR2 and has the same timing as dfi_rddata_valid. Bit N correlates to the byte group dfi_rddata[8N+7:8N]. It is the PHY’s way of marking bytes that should be discarded.

Source: DFI Specification v2.1.1 sections 3.3, 4.4

# 8	Update Interface

The update interface lets either side ask the other to pause normal traffic so that internal PHY or MC settings can be updated. There are two protocols: an MC-initiated update and a PHY-initiated update. Both require the DFI bus to be idle of control, read, and write traffic while the update is in progress.

## 8.1	Update signals

DFI 2.1 update interface signals

| Signal | Direction | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_ctrlupd_req | MC -> PHY | 1 bit | 0x0 | MC-initiated update invitation; asserted for tctrlupd_min..tctrlupd_max cycles. |
| dfi_ctrlupd_ack | PHY -> MC | 1 bit | 0x0 | PHY acknowledges the MC-initiated update; may remain low to ignore. |
| dfi_phyupd_req | PHY -> MC | 1 bit | 0x0 | PHY-initiated update request; must be acknowledged by the MC. |
| dfi_phyupd_type | PHY -> MC | 2 bits | no default | Selects one of four PHY update time types. |
| dfi_phyupd_ack | MC -> PHY | 1 bit | 0x0 | MC acknowledges the PHY-initiated update. |

## 8.2	MC-initiated update

The MC asserts dfi_ctrlupd_req when it knows the DFI will be idle. The PHY is free to acknowledge or ignore the request. If it acknowledges by asserting dfi_ctrlupd_ack, the request stays asserted as long as the acknowledge is high, and the bus remains idle for update-related traffic only. If the PHY ignores the request, dfi_ctrlupd_ack stays low and the MC may de-assert the request any time after tctrlupd_min and before tctrlupd_max.

The DFI specification requires the MC to issue update requests and sets a maximum interval tctrlupd_interval between requests. The MC must also assert dfi_ctrlupd_req at the end of initialization to signal that init is complete.

## 8.3	PHY-initiated update

The PHY asserts dfi_phyupd_req when it needs the bus quieted. The request is accompanied by dfi_phyupd_type, which selects one of four timing parameters: tphyupd_type0 through tphyupd_type3. The type value must remain constant while the request is asserted.

The MC must acknowledge by asserting dfi_phyupd_ack within tphyupd_resp cycles. The PHY keeps dfi_phyupd_req asserted until the update completes, then de-asserts it; the MC de-asserts dfi_phyupd_ack on the following cycle. The entire acknowledged window is bounded by the selected tphyupd_typeX.

## 8.4	Concurrent requests

If both dfi_ctrlupd_req and dfi_phyupd_req are asserted at the same time, either side may acknowledge the other’s request. The unacknowledged request may then be de-asserted. This is the only situation in which the PHY is permitted to de-assert dfi_phyupd_req without first receiving an acknowledge.

## 8.5	Update timing parameters

Update interface timing parameters

| Parameter | Meaning |
| --- | --- |
| tctrlupd_interval | Maximum cycles the MC may wait between ctrlupd_req assertions. |
| tctrlupd_min | Minimum cycles dfi_ctrlupd_req must be asserted. |
| tctrlupd_max | Maximum cycles dfi_ctrlupd_req may be asserted. |
| tphyupd_type0..3 | Maximum cycles dfi_phyupd_req may remain asserted after ack, per type. |
| tphyupd_resp | Maximum cycles from dfi_phyupd_req to dfi_phyupd_ack. |

Source: DFI Specification v2.1.1 sections 3.4, 4.5

# 9	Status Interface

The status interface handles initialization, clock control, frequency ratio, and optional DIMM parity reporting. Several of its signals are optional and only matter if the system implements the related feature.

## 9.1	Status signals

DFI 2.1 status interface signals

| Signal | Direction | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_data_byte_disable | MC -> PHY | DFI data width / 8 | no default | Optional per-byte disable; valid only at initialization. |
| dfi_dram_clk_disable | MC -> PHY | DFI chip select width | 0x0 | When high, PHY stops clocking the DRAMs. |
| dfi_freq_ratio | MC -> PHY | 2 bits | no default | Optional static ratio: 00=1:1, 01=1:2, 10=1:4, 11=reserved. |
| dfi_init_complete | PHY -> MC | 1 bit | 0x0 | PHY is ready to respond to DFI traffic; also frequency-change ack. |
| dfi_init_start | MC -> PHY | 1 bit | 0x0 or 0x1 | MC indicates status signals are valid; also used to request frequency change. |
| dfi_parity_in | MC -> PHY | 1 bit | 0x0 or 0x1 | Even/odd parity for control interface; replicated as dfi_parity_in_pN. |
| dfi_parity_error | PHY -> MC | 1 bit | 0x0 | DIMM parity error report; correlated to ERR_OUT#. |

## 9.2	Initialization use

At power-up most command and status signals must be held at their default values until dfi_init_complete asserts. The exceptions are signals that have no meaningful default, such as dfi_address for most systems, or dfi_data_byte_disable and dfi_freq_ratio, which the MC defines before or during initialization.

dfi_init_start tells the PHY that dfi_data_byte_disable and/or dfi_freq_ratio are valid. A PHY that depends on these settings may wait for dfi_init_start before asserting dfi_init_complete. A PHY that does not may assert dfi_init_complete earlier. In either case initialization is not complete until both dfi_init_start and dfi_init_complete have been asserted simultaneously for at least one DFI clock cycle.

## 9.3	Frequency change use

During normal operation, if both sides support the optional frequency change protocol, the MC may re-assert dfi_init_start to request a frequency change. The PHY accepts by de-asserting dfi_init_complete within tinit_start cycles. The MC holds dfi_init_start until the change completes; the PHY re-asserts dfi_init_complete within tinit_complete cycles after dfi_init_start drops.

## 9.4	Clock disable

dfi_dram_clk_disable lets the MC stop the clocks to the DRAM devices. The PHY must hold the DRAM clock at a constant value within tdram_clk_disable cycles after the signal asserts, and must resume valid clock edges within tdram_clk_enable cycles after it de-asserts.

## 9.5	Parity

dfi_parity_in carries even or odd parity over dfi_address, dfi_bank, dfi_cas_n, dfi_ras_n, and dfi_we_n. In frequency-ratio systems it is replicated as dfi_parity_in_pN. dfi_parity_error is returned by the PHY within tphy_paritylat cycles when a DIMM detects a parity error.

Source: DFI Specification v2.1.1 sections 3.5, 4.1, 4.6, 4.8, 4.9

# 10	Training Interface

DFI 2.1 provides a training interface for read leveling and write leveling. Read leveling covers both data eye training and gate training. Write leveling is specific to DDR3. The interface is optional overall, but once a PHY declares support for a training operation, the MC must support all three defined modes for that operation.

## 10.1	Training support modes

Each training capability is announced by a 2-bit mode signal:

Training mode encodings

| Encoding | Mode |
| --- | --- |
| 2’b00 | Not supported |
| 2’b01 | MC Evaluation: MC enables logic, reads results, adjusts delays |
| 2’b10 | PHY Evaluation: MC enables logic, PHY evaluates and sets delays |
| 2’b11 | PHY Independent: PHY performs training without MC involvement |

The MC must be able to operate with any of these modes; the PHY typically supports only one.

## 10.2	Read leveling signals

Read leveling signals

| Signal | Direction | What it does |
| --- | --- | --- |
| dfi_rdlvl_req | PHY -> MC | PHY asks to run data eye training. |
| dfi_rdlvl_gate_req | PHY -> MC | PHY asks to run gate training. |
| dfi_rdlvl_mode | PHY -> MC | Announces data eye training support mode. |
| dfi_rdlvl_gate_mode | PHY -> MC | Announces gate training support mode. |
| dfi_rdlvl_en | MC -> PHY | Enables data eye training; also acknowledges dfi_rdlvl_req. |
| dfi_rdlvl_gate_en | MC -> PHY | Enables gate training; also acknowledges dfi_rdlvl_gate_req. |
| dfi_rdlvl_cs_n | MC -> PHY | Chip select active during read leveling. |
| dfi_rdlvl_edge | MC -> PHY | Selects positive or negative DQS edge for training. |
| dfi_rdlvl_delay_X | MC -> PHY | Per-slice delay value for data eye training. |
| dfi_rdlvl_gate_delay_X | MC -> PHY | Per-slice delay value for gate training. |
| dfi_rdlvl_load | MC -> PHY | One-cycle pulse when delay values have been updated. |
| dfi_rdlvl_resp | PHY -> MC | Training result or completion indication. |

For LPDDR2, dfi_rdlvl_resp is used only for gate training in MC Evaluation mode, not for data eye training. For DDR3 it carries sampled DQ level or gate value.

## 10.3	Write leveling signals

Write leveling signals

| Signal | Direction | What it does |
| --- | --- | --- |
| dfi_wrlvl_req | PHY -> MC | PHY asks to run write leveling. |
| dfi_wrlvl_mode | PHY -> MC | Announces write leveling support mode. |
| dfi_wrlvl_en | MC -> PHY | Enables write leveling; also acknowledges dfi_wrlvl_req. |
| dfi_wrlvl_cs_n | MC -> PHY | Chip select active during write leveling. |
| dfi_wrlvl_delay_X | MC -> PHY | Per-slice write DQS delay value. |
| dfi_wrlvl_load | MC -> PHY | One-cycle pulse when delay values have been updated. |
| dfi_wrlvl_strobe | MC -> PHY | Triggers the PHY write leveling strobe. |
| dfi_wrlvl_resp | PHY -> MC | Completion or sampled DQ result. |

Write leveling is a DDR3-specific function. A system that does not support DDR3 write leveling does not need these signals.

## 10.4	Key timing parameters

Training timing parameters

| Parameter | Meaning |
| --- | --- |
| trdlvl_resp / twrlvl_resp | Maximum cycles from request to MC enable assertion. |
| trdlvl_en / twrlvl_en | Minimum cycles from enable to first load or command. |
| trdlvl_load / twrlvl_load | Minimum cycles from delay update to next load. |
| trdlvl_dll / twrlvl_dll | Minimum cycles from load to next read or strobe. |
| trdlvl_resplat / twrlvl_resplat | Maximum cycles from command/strobe to valid response. |
| trdlvl_max / twrlvl_max | Maximum cycles the MC waits for a PHY Evaluation response. |
| trdlvl_rr | Minimum command-to-command delay for read leveling reads. |
| twrlvl_ww | Minimum strobe-to-strobe delay for write leveling. |

Source: DFI Specification v2.1.1 sections 3.6, 4.10

# 11	Low Power Control Interface

The low power control interface lets the MC tell the PHY when the memory subsystem is expected to remain idle, giving the PHY an opportunity to enter a lower power state. The interface is optional for both sides.

## 11.1	Low power signals

DFI 2.1 low power control interface signals

| Signal | Direction | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_lp_req | MC -> PHY | 1 bit | 0x0 | Informs the PHY of a low-power opportunity. |
| dfi_lp_wakeup | MC -> PHY | 4 bits | no default | Encodes how quickly the MC needs normal operation to resume. |
| dfi_lp_ack | PHY -> MC | 1 bit | 0x0 | PHY acknowledges the low-power opportunity. |

## 11.2	Wakeup encoding

dfi_lp_wakeup is valid only while dfi_lp_req is asserted. The encoding doubles the wakeup time on each step:

Low power wakeup encoding

| dfi_lp_wakeup | tlp_wakeup cycles |
| --- | --- |
| 4’b0000 | 16 |
| 4’b0001 | 32 |
| 4’b0010 | 64 |
| 4’b0011 | 128 |
| 4’b0100 | 256 |
| 4’b0101 | 512 |
| 4’b0110 | 1024 |
| 4’b0111 | 2048 |
| 4’b1000 | 4096 |
| 4’b1001 | 8192 |
| 4’b1010 | 16384 |
| 4’b1011 | 32768 |
| 4’b1100 | 65536 |
| 4’b1101 | 131072 |
| 4’b1110 | 262144 |
| 4’b1111 | unlimited |

The value must remain constant until dfi_lp_ack asserts. After that the MC may increase the value, allowing the PHY to enter a deeper low power state, but it may not decrease it. The value at the moment dfi_lp_req de-asserts sets the tlp_wakeup exit budget.

## 11.3	Handshake

When the MC detects an idle window it asserts dfi_lp_req with a chosen dfi_lp_wakeup. The PHY may acknowledge within tlp_resp cycles or ignore the request. If it acknowledges, both signals remain asserted while the PHY is in the low power state. After dfi_lp_req de-asserts, the PHY has tlp_wakeup cycles to resume normal operation and de-assert dfi_lp_ack.

The DFI clock must remain valid and constant during the entire low power handshake.

Source: DFI Specification v2.1.1 sections 3.7, 4.11

# 12	Initialization and Training

## 12.1	Reset and initialization

DFI 2.1 does not dictate a full power-on reset sequence for the MC or the PHY. It only defines when the PHY is ready to accept DFI traffic: when dfi_init_complete asserts. Until then, command and status signals must be held at their default values.

### 12.1.1	Signals held at default until init_complete

The following signals must not change from their default until dfi_init_complete asserts:

- dfi_address (except the LPDDR2 NOP requirement)

- dfi_bank, dfi_cas_n, dfi_ras_n, dfi_we_n

- dfi_cke, dfi_cs_n, dfi_odt, dfi_reset_n

- dfi_wrdata_en, dfi_rddata_en

- dfi_rddata_dnv, dfi_rddata_valid

- dfi_ctrlupd_ack, dfi_ctrlupd_req

- dfi_phyupd_req, dfi_phyupd_ack

- dfi_dram_clk_disable

- dfi_parity_error, dfi_parity_in

- dfi_rdlvl_req, dfi_rdlvl_gate_req, dfi_rdlvl_en, dfi_rdlvl_gate_en

- dfi_rdlvl_load, dfi_wrlvl_en, dfi_wrlvl_load, dfi_wrlvl_req, dfi_wrlvl_strobe

- dfi_lp_ack, dfi_lp_req

Signals with no meaningful default, such as dfi_wrdata, dfi_wrdata_mask, dfi_rddata, dfi_phyupd_type, and the per-slice delay/response signals, do not need a defined value during initialization.

### 12.1.2	LPDDR2 special rule

For LPDDR2 the dfi_address bus must carry a NOP command until dfi_init_complete asserts. The dfi_bank, dfi_cas_n, dfi_ras_n, and dfi_we_n signals are unused and must remain at constant idle values.

### 12.1.3	init_start handshake

dfi_init_start is optional. When used, it has two purposes:

- At initialization it tells the PHY that dfi_data_byte_disable and/or dfi_freq_ratio are valid.

- During normal operation it requests a frequency change.

If the PHY depends on the disable or ratio settings, it waits for dfi_init_start before asserting dfi_init_complete. If not, it may assert dfi_init_complete earlier. Either way, init is complete only when both dfi_init_start and dfi_init_complete are high for at least one DFI clock.

### 12.1.4	End-of-init ctrlupd_req

The MC must assert dfi_ctrlupd_req at the end of initialization, after all required training, to indicate that initialization is complete. This is the first update window the PHY sees.

### 12.1.5	Initialization timing parameters

Initialization timing parameters

| Parameter | Meaning |
| --- | --- |
| tinit_complete | Maximum cycles from dfi_init_start de-assertion to dfi_init_complete re-assertion during frequency change. |
| tinit_start | Maximum cycles for PHY to de-assert dfi_init_complete after a frequency change request. |

## 12.2	Training operations

DFI 2.1 supports read leveling and write leveling. Read leveling is used by both DDR3 and LPDDR2 systems; write leveling is DDR3-specific. The interface is optional overall, but if a PHY declares support for a training operation, the MC must support all three defined modes.

### 12.2.1	Read leveling

Read leveling has two parts:

- Data eye training: centers the read DQS strobe in the DQ data eye.

- Gate training: places the DQS gate at the correct point in the read preamble.

For DDR3 both operations are used. For LPDDR2 the response signal dfi_rdlvl_resp is used only for gate training in MC Evaluation mode; data eye training is not supported via that path for LPDDR2.

### 12.2.2	Write leveling

Write leveling aligns the write DQS to the memory clock. It is only relevant for DDR3. The MC either evaluates the result itself and adjusts dfi_wrlvl_delay_X, or it lets the PHY evaluate and set the delay internally.

### 12.2.3	Responsibility modes

Training responsibility modes

| Mode | MC role | PHY role |
| --- | --- | --- |
| MC Evaluation | Enables logic, reads response, writes delay values, pulses load | Provides sampled results |
| PHY Evaluation | Enables logic and waits for completion | Evaluates and sets delays |
| PHY Independent | None | Performs training autonomously |

The MC must be able to operate in all three modes, even though a given PHY only supports one.

### 12.2.4	Training is not required at init

The DFI specification does not require any training before dfi_init_complete asserts. The PHY must guarantee the integrity of the address and control path before declaring init complete, but it may perform training later under its own control or at the MC’s request.

Source: DFI Specification v2.1.1 sections 3.5, 3.6, 4.1, 4.10

# 13	Command Mapping

The DFI control interface is essentially a delayed, possibly widened, copy of the DRAM command interface. How the MC encodes a command depends on the memory type.

## 13.1	DDR2 command encoding

For DDR2 the classic command truth table applies directly on the DFI control signals.

DDR2 command encoding on DFI control signals

| Command | CS_n | RAS_n | CAS_n | WE_n | Address A10 | Address other | Bank |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOP | 0 | 1 | 1 | 1 | X | X | X |
| ACT | 0 | 0 | 1 | 1 | row bit | row | bank |
| RD | 0 | 1 | 0 | 1 | 0 | column | bank |
| RDA | 0 | 1 | 0 | 1 | 1 | column | bank |
| WR | 0 | 1 | 0 | 0 | 0 | column | bank |
| WRA | 0 | 1 | 0 | 0 | 1 | column | bank |
| PRE (one bank) | 0 | 0 | 1 | 0 | 0 | X | bank |
| PREA (all banks) | 0 | 0 | 1 | 0 | 1 | X | X |
| REF | 0 | 0 | 0 | 1 | X | X | X |
| DES | 1 | X | X | X | X | X | X |

dfi_cke is driven high during normal operation and manipulated separately for self-refresh or power-down entry. dfi_odt is driven according to the DRAM ODT programming for reads and writes.

## 13.2	LPDDR2 command encoding

LPDDR2 has no RAS/CAS/WE pins. Commands and a scrambled address are multiplexed onto a 10-bit CA bus across two clock edges. DFI 2.1 maps the entire CA word onto dfi_address as a flat 20-bit value, while dfi_bank, dfi_ras_n, dfi_cas_n, and dfi_we_n are held idle.

DFI 2.1 LPDDR2 CA mapping

| dfi_address bit | CA bit / edge |
| --- | --- |
| 9:0 | CA[9:0] on rising edge |
| 19:10 | CA[9:0] on falling edge |

The rising-edge opcode is decoded from CA0r, CA1r, CA2r, CA3r:

LPDDR2 rising-edge command opcode

| Command | CA0r | CA1r | CA2r | CA3r |
| --- | --- | --- | --- | --- |
| MRW | 0 | 0 | 0 | 0 |
| MRR | 0 | 0 | 0 | 1 |
| Refresh per-bank | 0 | 0 | 1 | 0 |
| Refresh all-bank | 0 | 0 | 1 | 1 |
| Activate | 0 | 1 | X | X |
| Write | 1 | 0 | 0 | X |
| Read | 1 | 0 | 1 | X |
| Precharge | 1 | 1 | 0 | 1 |
| BST | 1 | 1 | 0 | 0 |
| NOP/Deselect | 1 | 1 | 1 | X |

Row, column, bank, and mode-register fields are then placed on the remaining CA pins according to the JEDEC LPDDR2 command truth table. The PHY splits the 20-bit dfi_address word into the two 10-bit DDR CA cycles.

Source: DFI Specification v2.1.1 sections 3.1, 4.2; JEDEC JESD209-2F Table 60

# 14	Write Carriage

A write transaction on the DFI has three parts: the write command on the control interface, the write enable assertion, and the data/mask payload. The PHY is responsible for turning that sequence into the correct DRAM timing.

## 14.1	Command to enable relationship

The MC asserts dfi_wrdata_en tphy_wrlat cycles after the write command appears on the control interface. The enable must stay asserted for every cycle that contains write data. For contiguous back-to-back writes the enable can remain high across the command boundary.

## 14.2	Enable to data relationship

The actual data and mask appear tphy_wrdata cycles after dfi_wrdata_en asserts. In older DFI versions tphy_wrdata was effectively fixed at 1; DFI 2.1 made it programmable so the PHY can ask for earlier notification.

dfi_wrdata_mask follows the same timing as dfi_wrdata. A mask bit of 1 means “do not write this byte.” Bit N covers dfi_wrdata[8N+7:8N]. If the data bus is not a multiple of 8, the most significant mask bit covers the most significant partial byte.

## 14.3	Contiguous and interrupted writes

When two writes are separated by a bubble, the MC de-asserts dfi_wrdata_en for at least one cycle. The PHY uses that gap to separate the two bursts. When writes are back-to-back, the enable stays asserted and the data stream is contiguous.

In a frequency-ratio system the write data is split across phases. The MC may keep the data aligned to phase 0 by using tphy_wrdelay, or it may send the data unaligned and let the PHY sort it out.

## 14.4	Frequency ratio packing

In a 1:2 ratio system a single DFI clock contains two phases of write data. The MC can therefore issue one write command per DFI clock and still supply both halves of the burst on dfi_wrdata_p0 and dfi_wrdata_p1. In a 1:4 ratio system four phases fit in one DFI clock.

Source: DFI Specification v2.1.1 sections 3.2, 4.3

# 15	Read Carriage

A read transaction on the DFI has two parts: the read command on the control interface, and the returned data on the read data interface. The MC tells the PHY when to expect data; the PHY tells the MC when the data is actually valid.

## 15.1	Command to enable relationship

The MC asserts dfi_rddata_en trddata_en cycles after the read command. The enable must stay asserted for the duration of contiguous read data expected from the PHY. A single continuous assertion can cover data from multiple read commands.

## 15.2	Enable to data relationship

The PHY returns data by asserting dfi_rddata_valid with the data. The first valid beat may arrive any time up to tphy_rdlat cycles after dfi_rddata_en, but the PHY is allowed to return it earlier. When dfi_rddata_valid is high, the entire DFI read data word is valid.

For LPDDR2 the PHY also asserts dfi_rddata_dnv with the same timing. A high bit in dfi_rddata_dnv means the corresponding byte in dfi_rddata should be discarded.

## 15.3	Contiguous reads

Back-to-back reads can be covered by a single dfi_rddata_en assertion. The MC only de-asserts the enable when it no longer expects contiguous data. The PHY uses dfi_rddata_valid to mark the actual cycles that carry valid data.

## 15.4	Frequency ratio packing

In a 1:2 ratio system the read data is returned on dfi_rddata_w0 and dfi_rddata_w1, one word per half DFI clock. In a 1:4 system there are four word copies _w0 through _w3. The read data enable is still phase-specific: dfi_rddata_en_p0, dfi_rddata_en_p1, and so on.

Source: DFI Specification v2.1.1 sections 3.3, 4.4

# 16	Wire-Level Walkthroughs

These walkthroughs use a tiny teaching model so the DFI timing is visible at a glance.

## 16.1	Teaching model

- Two banks: B0 and B1.

- One rank, one chip select.

- DFI 2.1, 1:2 frequency ratio, so there are two phases per DFI clock: _p0 and _p1.

- Burst length 4, one data word per phase, so BL4 fits in one DFI clock.

- tphy_wrlat = 1, tphy_wrdata = 1 for writes; trddata_en = 2, tphy_rdlat = 5 for reads.

- The controller places ACT on phase 0 and the first subsequent RD/WR on the programmed read or write phase.

## 16.2	Walkthrough 1: Activate to Read

Assume the MC wants to read from an idle bank B0, row R3.

Cycle 0, phase 0: dfi_cs_n = 0, dfi_ras_n = 0, dfi_cas_n = 1, dfi_we_n = 1, dfi_bank = B0, dfi_address = R3. This is the ACT command. Cycle 0, phase 1 is a NOP: dfi_cs_n = 1.

After the DRAM’s tRCD has passed, the MC issues a read.

Cycle 2, phase 0: NOP. Cycle 2, phase 1: dfi_cs_n = 0, dfi_ras_n = 1, dfi_cas_n = 0, dfi_we_n = 1, dfi_bank = B0, dfi_address = column C1 with A10 = 0 for no auto-precharge. This is the RD command.

Two cycles after the RD command, at cycle 4, the MC asserts dfi_rddata_en. It stays asserted for two phases to cover the BL4 burst.

Cycle 4, phase 0: dfi_rddata_en_p0 = 1. Cycle 4, phase 1: dfi_rddata_en_p1 = 1.

The PHY returns the first valid data no later than tphy_rdlat = 5 cycles after the enable. Suppose it returns early at cycle 5.

Cycle 5, phase 0: dfi_rddata_w0 valid, dfi_rddata_valid_w0 = 1. Cycle 5, phase 1: dfi_rddata_w1 valid, dfi_rddata_valid_w1 = 1.

The MC captures the two words and delivers them as the BL4 read response.

## 16.3	Walkthrough 2: Activate to Write

Assume the MC wants to write to bank B1, row R5.

Cycle 0, phase 0: dfi_cs_n = 0, dfi_ras_n = 0, dfi_cas_n = 1, dfi_we_n = 1, dfi_bank = B1, dfi_address = R5. ACT command. Cycle 0, phase 1: NOP.

After tRCD, the MC issues the write.

Cycle 2, phase 0: NOP. Cycle 2, phase 1: dfi_cs_n = 0, dfi_ras_n = 1, dfi_cas_n = 0, dfi_we_n = 0, dfi_bank = B1, dfi_address = column C2 with A10 = 0. WR command.

One cycle after the write command, at cycle 3, the MC asserts dfi_wrdata_en_p0 and dfi_wrdata_en_p1. Because tphy_wrdata = 1, the data follows one cycle later at cycle 4.

Cycle 3, phase 0: dfi_wrdata_en_p0 = 1. Cycle 3, phase 1: dfi_wrdata_en_p1 = 1.

Cycle 4, phase 0: dfi_wrdata_p0 = first data word, dfi_wrdata_mask_p0 = 0. Cycle 4, phase 1: dfi_wrdata_p1 = second data word, dfi_wrdata_mask_p1 = 0.

If the MC wanted to mask the lower byte of the first beat, it would set dfi_wrdata_mask_p0[0] = 1.

## 16.4	What the traces show

The control interface carries the command. The write/read data enables predict when data is expected, and the data valid signal bounds the actual return. The PHY is free to delay read data up to tphy_rdlat, but the MC must have scheduled dfi_rddata_en based on trddata_en. For writes, the command leads the enable by tphy_wrlat, and the enable leads the data by tphy_wrdata.

Source: DFI Specification v2.1.1 sections 3.1, 3.2, 3.3, 4.2, 4.3, 4.4

# 17	Parameter Families and Ratio Phasing

## 17.1	Parameter families

DFI 2.1 timing parameters are named dfi_t* in public shorthand, but the specification itself uses names like tctrl_delay, tphy_wrlat, and trddata_en. Each parameter is defined by the MC, the PHY, or the system as a whole, and must remain constant while traffic is in flight. They may be changed only when the DFI bus is idle.

### 17.1.1	Who defines what

Timing parameter families and ownership

| Family | Examples | Defined by | Observable by |
| --- | --- | --- | --- |
| Control | tctrl_delay | PHY | MC sees command-to-DRAM delay |
| Write data | tphy_wrlat, tphy_wrdata, tphy_wrdelay | PHY | MC schedules enable and data |
| Read data | trddata_en, tphy_rdlat | System / PHY | MC schedules enable, PHY bounds valid |
| Update | tctrlupd_, tphyupd_ | MC / PHY | Both sides handshake idle windows |
| Status | tdram_clk_disable, tdram_clk_enable, tinit_complete, tinit_start, tphy_paritylat | PHY / MC | Both sides |
| Training | trdlvl_, twrlvl_ | PHY / MC | Both sides during leveling |
| Low power | tlp_resp, tlp_wakeup | MC | PHY responds within bounds |

### 17.1.2	What the families constrain

Control timing only has tctrl_delay: the uniform delay through the PHY to the DRAM pins. Write data timing constrains the gap from command to enable and from enable to data. Read data timing constrains the gap from command to enable and from enable to the latest valid data. Update timing bounds how long each side may keep the bus idle. Status timing bounds clock disable, init handshake, and parity latency. Training timing bounds command-to-response and delay-load delays. Low power timing bounds how quickly the PHY must acknowledge and exit.

### 17.1.3	Constants, maxima, and fixed values

A parameter may be a fixed constant, a maximum value, or a value derived from other system settings. For example, tphy_wrlat is usually a fixed PHY property, while tphy_rdlat is often a system-level maximum. The DFI does not mandate absolute numeric ranges; compatibility depends on the MC and PHY supporting overlapping ranges.

## 17.2	Ratio and phasing

DFI 2.1 supports matched frequency (1:1) and frequency ratio (1:2 and 1:4) systems. The ratio changes how command, write, and read information is packed into a single DFI clock.

### 17.2.1	1:1 matched frequency

In a 1:1 system the MC clock and the PHY clock are the same. There is one set of control signals, one write data bus, one read data bus, and one read data enable. All timing parameters are expressed in DFI clock cycles, which are also PHY clock cycles.

### 17.2.2	1:2 frequency ratio

In a 1:2 system the PHY clock runs at twice the DFI clock. Each DFI clock contains two PHY phases. The MC can therefore place commands on both phases:

- Control signals become dfi_*_p0 and dfi_*_p1.

- Write data and mask become dfi_wrdata_p0 / _p1 and dfi_wrdata_mask_p0 / _p1.

- Write data enable becomes dfi_wrdata_en_p0 / _p1.

- Read data enable becomes dfi_rddata_en_p0 / _p1.

- Read data, valid, and DNV become dfi_rddata_w0 / _w1, dfi_rddata_valid_w0 / _w1, and dfi_rddata_dnv_w0 / _w1.

The MC may issue a command on any phase. The PHY must accept commands on all phases.

### 17.2.3	1:4 frequency ratio

In a 1:4 system there are four PHY phases per DFI clock. The same suffixing applies with _p0 through _p3 and _w0 through _w3. This allows the MC to issue up to four commands per DFI clock and to receive up to four read data words per DFI clock.

### 17.2.4	Impact on scheduling

A higher ratio lets the MC run slower than the DRAM pins while still supplying full pin bandwidth. The trade-off is scheduling complexity: the controller must place each command on the correct phase, align write data to tphy_wrlat and tphy_wrdata in PHY-clock terms, and align the read data enable to trddata_en in PHY-clock terms. The tphy_wrdelay parameter exists so the MC can keep write data aligned to phase 0 in its own view while the PHY shifts it to the correct phase internally.

### 17.2.5	Ratio selection

The ratio is static for normal operation. It is communicated at initialization via dfi_freq_ratio and confirmed by the dfi_init_start handshake. A later frequency change can select a new ratio, but that requires the optional frequency change protocol.

Source: DFI Specification v2.1.1 sections 2.0, 3.1, 3.2, 3.3, 4.7, 5.0

# 18	Parameter Tables

The table below lists every timing parameter named in the DFI 2.1 inventory, grouped by sub-interface, with a plain-English meaning.

## 18.1	Control

Control timing parameter

| Parameter | Meaning |
| --- | --- |
| tctrl_delay | Cycles from DFI control transition to the same transition at the PHY-DRAM boundary. |

## 18.2	Write data

Write data timing parameters

| Parameter | Meaning |
| --- | --- |
| tphy_wrdata | Cycles from dfi_wrdata_en assertion to first dfi_wrdata beat. |
| tphy_wrdelay | Optional PHY delay between enable and data in ratio systems. |
| tphy_wrlat | Cycles from write command to dfi_wrdata_en assertion. |

## 18.3	Read data

Read data timing parameters

| Parameter | Meaning |
| --- | --- |
| tphy_rdlat | Maximum cycles from dfi_rddata_en to first dfi_rddata_valid. |
| trddata_en | Cycles from read command to dfi_rddata_en assertion. |

## 18.4	Update

Update timing parameters

| Parameter | Meaning |
| --- | --- |
| tctrlupd_interval | Maximum cycles the MC may wait between dfi_ctrlupd_req assertions. |
| tctrlupd_min | Minimum cycles dfi_ctrlupd_req must be asserted. |
| tctrlupd_max | Maximum cycles dfi_ctrlupd_req may be asserted. |
| tphyupd_type0 | Maximum cycles dfi_phyupd_req may stay asserted after ack for type 0. |
| tphyupd_type1 | Same, for type 1. |
| tphyupd_type2 | Same, for type 2. |
| tphyupd_type3 | Same, for type 3. |
| tphyupd_resp | Maximum cycles from dfi_phyupd_req to dfi_phyupd_ack. |

## 18.5	Status

Status timing parameters

| Parameter | Meaning |
| --- | --- |
| tdram_clk_disable | Cycles from dfi_dram_clk_disable assertion to DRAM clocks held constant. |
| tdram_clk_enable | Cycles from dfi_dram_clk_disable de-assertion to first valid DRAM clock edge. |
| tinit_complete | Maximum cycles from dfi_init_start de-assertion to dfi_init_complete re-assertion. |
| tinit_start | Maximum cycles for PHY to de-assert dfi_init_complete after a frequency change request. |
| tphy_paritylat | Maximum cycles from dfi_parity_in to associated dfi_parity_error. |

## 18.6	Training

Training timing parameters

| Parameter | Meaning |
| --- | --- |
| trdlvl_dll | Minimum cycles from read-leveling delay load to next read/MRR command. |
| trdlvl_en | Minimum cycles from dfi_rdlvl_en to first load or command. |
| trdlvl_load | Minimum cycles from delay update to next dfi_rdlvl_load. |
| trdlvl_max | Maximum cycles MC waits for a PHY Evaluation read-leveling response. |
| trdlvl_resp | Maximum cycles from dfi_rdlvl_req to dfi_rdlvl_en. |
| trdlvl_resplat | Maximum cycles from read/MRR command to valid dfi_rdlvl_resp. |
| trdlvl_rr | Minimum command-to-command delay during read leveling. |
| twrlvl_dll | Minimum cycles from write-leveling delay load to next dfi_wrlvl_strobe. |
| twrlvl_en | Minimum cycles from dfi_wrlvl_en to first load or strobe. |
| twrlvl_load | Minimum cycles from delay update to next dfi_wrlvl_load. |
| twrlvl_max | Maximum cycles MC waits for a PHY Evaluation write-leveling response. |
| twrlvl_resp | Maximum cycles from dfi_wrlvl_req to dfi_wrlvl_en. |
| twrlvl_resplat | Maximum cycles from dfi_wrlvl_strobe to valid dfi_wrlvl_resp. |
| twrlvl_ww | Minimum strobe-to-strobe delay during write leveling. |

## 18.7	Low power

Low power timing parameters

| Parameter | Meaning |
| --- | --- |
| tlp_resp | Maximum cycles from dfi_lp_req to dfi_lp_ack. |
| tlp_wakeup | Maximum cycles dfi_lp_ack may remain asserted after dfi_lp_req de-asserts. |

Source: DFI Specification v2.1.1 sections 3.1 through 3.7

# 19	System Behavior

This chapter covers three DFI 2.1 housekeeping protocols: low power control, the update mechanism, and the optional frequency change handshake.

## 19.1	Low power control

The DFI 2.1 low power interface is an opportunity handshake, not a forced state transition. The MC tells the PHY that the memory subsystem is idle and how quickly it may need to resume. The PHY decides whether to enter a lower power state.

### 19.1.1	Entry sequence

- The MC detects an idle window and asserts dfi_lp_req.

- The MC drives dfi_lp_wakeup with the required exit time.

- The PHY either acknowledges within tlp_resp cycles or ignores the request.

- If acknowledged, both dfi_lp_req and dfi_lp_ack remain asserted while the PHY is in the low power state.

### 19.1.2	Wakeup time adjustment

After the PHY acknowledges, the MC may increase dfi_lp_wakeup, allowing the PHY to pick a deeper state. It may not decrease the value. The final value at the moment dfi_lp_req de-asserts sets the tlp_wakeup exit budget.

### 19.1.3	Exit sequence

- The MC de-asserts dfi_lp_req when it needs normal operation to resume.

- The PHY has tlp_wakeup cycles to exit the low power state and de-assert dfi_lp_ack.

- Normal traffic may resume after dfi_lp_ack is low.

### 19.1.4	Relationship to DRAM power modes

DFI low power is about the PHY, not the DRAM. The DRAM may already be in self-refresh, power-down, or a deep power-down state through separate CKE and command sequencing. The DFI low power interface simply lets the MC and PHY coordinate so that the PHY does not try to issue traffic while the MC thinks it is allowed to sleep. The DFI clock must remain valid and at a constant frequency during the entire handshake.

## 19.2	Update mechanism

The update mechanism lets the PHY or MC pause ordinary traffic so that internal settings can be refreshed. Typical reasons include temperature compensation of delay lines or recomputing ODT settings. Both protocols require the DFI to be idle of control, read, and write traffic during the update window.

### 19.2.1	MC-initiated update

The MC asserts dfi_ctrlupd_req for at least tctrlupd_min and at most tctrlupd_max cycles. The PHY may acknowledge with dfi_ctrlupd_ack, in which case the request stays asserted as long as the acknowledge is high, or the PHY may ignore it. The MC is required to offer these update windows periodically, bounded by tctrlupd_interval.

At the end of initialization the MC must assert dfi_ctrlupd_req to mark the end of the init sequence. This is often the first update window the PHY sees.

### 19.2.2	PHY-initiated update

The PHY asserts dfi_phyupd_req with a stable dfi_phyupd_type value. The MC must acknowledge with dfi_phyupd_ack within tphyupd_resp cycles. The PHY keeps the request asserted until the update completes, then de-asserts it. The MC de-asserts dfi_phyupd_ack on the cycle after the request drops.

The selected dfi_phyupd_type determines which tphyupd_typeX bound applies. Type 0 is the shortest; type 3 is the longest.

### 19.2.3	Why the bus must idle

Updates often change delay lines or termination settings that affect how traffic is driven or sampled. If a read or write were in flight during the update, its timing assumptions could change mid-burst. Both protocols therefore require the MC to place the system in a state where no control, read, or write traffic is moving across the DFI.

### 19.2.4	Concurrent updates

If both request signals are asserted simultaneously, either side may acknowledge the other’s request. The unacknowledged request may then be dropped. This is the only case where the PHY may de-assert dfi_phyupd_req without an acknowledge.

## 19.3	Frequency change

DFI 2.1 includes an optional frequency change protocol. It reuses the initialization handshake signals dfi_init_start and dfi_init_complete. The protocol exists in the specification, but it is not required for DFI compliance.

### 19.3.1	Acknowledged frequency change

- During normal operation, with dfi_init_complete high, the MC asserts dfi_init_start to request a frequency change.

- The PHY accepts by de-asserting dfi_init_complete within tinit_start cycles.

- The MC continues to hold dfi_init_start while the system changes clocks.

- The PHY re-initializes on the new frequency and re-asserts dfi_init_complete within tinit_complete cycles after dfi_init_start de-asserts.

If the PHY does not de-assert dfi_init_complete within tinit_start cycles, the MC must abort the request and release dfi_init_start.

### 19.3.2	Frequency ratio reset

When dfi_init_start is asserted for a frequency change, both sides must reset their DFI read data word pointers to 0. This ensures that the word suffix alignment is consistent after the clock change.

### 19.3.3	Pumice does not implement it

The pumice DDR2/LPDDR2 controller does not implement the frequency change protocol. Its dfi_init_start is used only during initialization, and the DFI clock ratio is fixed by the PHY integration. A controller that needs runtime frequency scaling would need to add the acknowledged-frequency-change state machine to the DFI plane.

Source: DFI Specification v2.1.1 sections 3.4, 3.5, 3.7, 4.5, 4.8, 4.11; docs/pumice_mas/ch03_interfaces/02_dfi_v21_interface_spec.md

# 20	Revision Deltas

## 20.1	What DFI 2.1 added versus DFI 2.0

DFI 2.1 is backward compatible with 2.0, but it added several optional features. The most important additions are:

DFI 2.1 additions over DFI 2.0

| Feature | What changed |
| --- | --- |
| LPDDR2 support | CA bus mapped onto dfi_address, dfi_rddata_dnv added, read leveling extended for LPDDR2 gate training. |
| Low power control interface | New dfi_lp_req, dfi_lp_wakeup, dfi_lp_ack, plus tlp_resp and tlp_wakeup. |
| Frequency ratio | Formalized 1:2 and 1:4 ratios with _pN phase copies and _wN read data copies. |
| Parity interface | dfi_parity_in and dfi_parity_error for DDR3 DIMM command parity. |
| Frequency change protocol | Optional runtime frequency change via dfi_init_start / dfi_init_complete. |
| Update interface | Formalized dfi_ctrlupd_* and dfi_phyupd_* idle-window handshakes. |
| tphy_wrdata | Made the enable-to-data delay programmable instead of fixed at 1. |
| dfi_data_byte_disable | Optional per-byte disable at initialization. |

None of these are required for DFI compliance. A 2.1 MC and a 2.0 PHY can still interoperate if they only use the common subset.

## 20.2	What later revisions changed

DFI 3.1 and subsequent revisions moved the standard toward higher data rates and more complex memories. The brief preview below is based on the public direction of later DFI versions; this book is scoped to DFI 2.1.

Brief preview of later DFI revisions

| Area | Later change |
| --- | --- |
| Error reporting | Added dfi_error and dfi_alert_n style reporting, plus data CRC handling for DDR4/LPDDR4. |
| Multiple channels | Introduced channel indexing for multi-channel PHYs and byte groups. |
| Richer training | Added DBI training, PHY master mode, and more per-bit/per-byte training controls. |
| CA parity | Extended command parity to LPDDR4 CA buses. |
| Frequency change | Refined the frequency change handshake and added gear ratio programming. |

For the pumice DDR2/LPDDR2 controller these later additions are out of scope. The controller targets only the DFI 2.1 command, write-data, read-data, and init-status sub-interfaces.

Source: DFI Specification v2.1.1 sections 1.0, 3.4, 3.5, 3.6, 3.7, 4.7, 4.8, 4.9, 4.10, 4.11

# 21	DFI 2.1 in the Pumice DDR2/LPDDR2 Controller

This book is a simplified study guide for the DFI 2.1 interface used by the pumice DDR2/LPDDR2 memory controller. The controller’s DFI plane is implemented in rtl/macro/pumice_dfi_plane.sv and documented in the project’s microarchitecture and interface specifications.

## 21.1	Sub-interface support

Pumice DFI 2.1 sub-interface support

| Sub-interface | Used by pumice? | Notes |
| --- | --- | --- |
| Control | Yes | Per-phase command placement for DDR2 RAS/CAS/WE and LPDDR2 CA. |
| Write data | Yes | Serialized write data, mask, and enable at t_phy_wrlat. |
| Read data | Yes | Read data enable at t_rddata_en; capture on dfi_rddata_valid. |
| Status init | Yes | dfi_init_start / dfi_init_complete only. |
| Update | No | ctrlupd_* / phyupd_* not driven in the first revision. |
| Training | No | PHY self-trains at startup (a7ddrphy). |
| Frequency change | No | No use case in the current design. |
| Low power | No | No PHY-side low-power coordination. |
| Parity / error | No | Not needed for DDR2/LPDDR2 here. |

## 21.2	Clock-domain crossing

Unlike the classic model where DFI sits on the controller clock, pumice places the DFI datapath on a dedicated dfi_clk and puts the single controller-to-PHY crossing inside pumice_dfi_plane. The crossing uses gaxi_fifo_async instances for command, write data, read data, and init event tokens. There are no standalone synchronizers.

## 21.3	Command path

pumice_dfi_cmd_path pops a command FIFO and places the active JEDEC command on the programmed write or read phase, filling the other phases with NOP. For DDR2 it uses the classic RAS/CAS/WE encoding. For LPDDR2 it uses the CA-bus formatter described in docs/uarch/LPDDR2_CA_ENCODING.md, packing rising CA on dfi_address[9:0] and falling CA on dfi_address[19:10] while holding the RAS/CAS/WE/bank signals idle.

## 21.4	Write and read serializers

pumice_dfi_wr_serializer presents dfi_wrdata, dfi_wrdata_en, and dfi_wrdata_mask at the programmed write latency. pumice_dfi_rd_aligner drives dfi_rddata_en at the programmed read enable latency and captures dfi_rddata when dfi_rddata_valid asserts, packing whole words into the async FIFO back to the controller clock domain.

## 21.5	Known gaps and future work

The first pumice DFI plane intentionally omits update handshaking, training control, low-power coordination, and frequency change. These could be added in a later revision without changing the command/write/read datapath. A future DDR3/LPDDR3 controller would need a larger DFI sub-interface set and would likely replace the DFI plane rather than extend this one.

Source: docs/uarch/PUMICE_DFI_PLANE_UARCH.md, docs/pumice_mas/ch03_interfaces/02_dfi_v21_interface_spec.md, docs/uarch/LPDDR2_CA_ENCODING.md