Memory Notes

Simplified DFI 3.1 Simplified edition - DFI Specification v3.1 (Denali Software)

October 4, 2026

Table of Contents

1 Simplified Dfi 3 1 Index	13

2 Introduction	14

2.1 What DFI is	14

2.2 Why the seam exists	15

2.3 Key features of DFI 3.1	16

2.4 Edition lineage	17

3 Organization	18

3.1 Sub-interface map for DFI 3.1	18

3.2 Clocking	19

3.3 Frequency ratio and the _pN / _wN replication rule	20

3.4 PHY data slices and byte lanes	21

3.5 Simplified teaching model	22

4 Acronyms	23

5 Control Interface	25

5.1 Command/address signals	26

5.2 DRAM enable and select signals	28

5.3 LPDDR3/LPDDR2 CA-bus mapping	29

5.4 Control timing parameters	30

6 Write Data Interface	31

6.1 Write data signals	32

6.2 dfi_wrdata_en behavior	34

6.3 Write data mask and DBI	35

6.4 Write chip select	36

6.5 Write data timing parameters	37

6.6 Write data programmable parameters	38

7 Read Data Interface	39

7.1 Read data signals	40

7.2 dfi_rddata_en and dfi_rddata_valid	42

7.3 Read chip select	43

7.4 LPDDR2 data-not-valid	44

7.5 Read data timing parameters	45

8 Update Interface	46

8.1 MC-initiated update	47

8.2 PHY-initiated update	48

8.3 Update timing parameters	49

8.4 Simultaneous requests	50

9 Status Interface	51

9.1 Initialization and configuration signals	52

9.2 Parity and alert signals	54

9.3 Status timing parameters	55

10 Training Interface	56

10.1 Read training signals	57

10.2 Write leveling signals	59

10.3 CA training signals (LPDDR3)	60

10.4 PHY-requested training in non-DFI training mode	61

10.5 Common training controls	62

10.6 Training timing parameters	63

10.7 Training programmable parameters	65

11 Low Power Control Interface	66

11.1 Low power signals	67

11.2 dfi_lp_wakeup encoding	68

11.3 Low power handshake	69

11.4 Low power timing parameters	70

12 Error Interface	71

12.1 Error signals	72

12.2 Error timing	73

12.3 Spec-defined error codes	74

13 Reset and Initialization	75

13.1 Default values before dfi_init_complete	76

13.2 The dfi_init_start / dfi_init_complete handshake	77

13.3 Frequency-change reuse of the init signals	78

13.4 Initialization timing parameters	79

14 Training in DFI 3.1	80

14.1 Which training is used where	81

14.2 DFI training mode	82

14.3 Read training sequence	83

14.4 Write leveling sequence	84

14.5 CA training sequence (LPDDR3)	85

14.6 Periodic vs full training	86

14.7 Non-DFI training mode and PHY-requested training	87

15 Command Mapping	88

15.1 DDR3 command encoding on DFI	89

15.2 LPDDR3/LPDDR2 CA-bus mapping	90

15.3 Chip select and enable usage	91

15.4 Command timing relationship	92

16 Write Carriage	93

16.1 Write timing parameters in use	94

16.2 Single write transaction	95

16.3 Contiguous vs non-contiguous writes	96

16.4 Write chip select timing	97

16.5 DBI and CRC notes	98

17 Read Carriage	99

17.1 Read timing parameters in use	100

17.2 Single read transaction	101

17.3 Contiguous vs non-contiguous reads	102

17.4 Per-slice independence	103

17.5 Read chip select timing	104

17.6 LPDDR2/LPDDR3 mode-register reads	105

18 Walkthroughs	106

18.1 Teaching model	107

18.2 Walkthrough 1: activate -> read -> data return	108

18.3 Walkthrough 2: activate -> write with preamble and mask	109

18.4 Walkthrough 3: back-to-back reads on different rows	110

19 Parameter Families	111

19.1 PHY-defined vs MC/system-defined parameters	112

19.2 Parameter families	113

19.3 Minimum and maximum conventions	114

19.4 Frequency-ratio timing	115

20 Ratio and Phasing	116

20.1 Matched frequency (1:1)	117

20.2 1:2 frequency ratio	118

20.3 1:4 frequency ratio	119

20.4 Read data rotation	120

20.5 Scheduling impact	121

20.6 Practical note for scoria	122

21 Parameter Tables	123

21.1 Programmable parameters	128

22 Low Power Behavior	129

22.1 Low-power request split	130

22.2 Typical sequence	131

22.3 Relationship to DRAM self-refresh	132

22.4 Scoria note	133

23 Update Behavior	134

23.1 MC-initiated update	135

23.2 PHY-initiated update	136

23.3 Idle-bus definition	137

23.4 Update after self-refresh exit	138

24 Frequency Change	139

24.1 Acknowledged frequency change	140

24.2 Not-acknowledged frequency change	141

24.3 Stable levels during the change	142

24.4 Scoria note	143

25 Refresh Interaction	144

25.1 Refresh and training	145

25.2 Refresh and update	146

25.3 Refresh and low power	147

25.4 Scoria note	148

26 Revision Deltas	149

26.1 What carried over unchanged from v2.1.1	150

26.2 v2.1.1 -> v3.0 changes	151

26.3 v3.0 -> v3.1 changes	152

26.4 Brief v4.0 preview	153

26.5 What scoria leaves out	154

27 DFI 3.1 in the Scoria Controller	155

27.1 Why scoria chose v3.1	156

27.2 What scoria inherits	157

27.3 What scoria implements from v3.1	158

27.4 What scoria does not implement	159

27.5 Known documentation corrections	160

27.6 Study corrections carried into this book	161

List of Figures

No figures in this document.

List of Tables

Table 1: DFI 3.1 sub-interfaces	18

Table 2: Acronyms used in this book	23

Table 3: Suffix notation	24

Table 4: Control interface command and address signals	26

Table 5: Control interface enable and select signals	28

Table 6: Control interface timing parameters	30

Table 7: Write data interface signals	32

Table 8: Write data interface timing parameters	37

Table 9: Write data programmable parameters	38

Table 10: Read data interface signals	40

Table 11: Read data interface timing parameters	45

Table 12: MC-initiated update signals	47

Table 13: PHY-initiated update signals	48

Table 14: Update interface timing parameters	49

Table 15: Status interface initialization and configuration signals	52

Table 16: Status interface parity and alert signals	54

Table 17: Status interface timing parameters	55

Table 18: Training operations in DFI 3.1	56

Table 19: Read training signals	57

Table 20: Write leveling signals	59

Table 21: CA training signals	60

Table 22: PHY-requested training signals	61

Table 23: Common training control signals	62

Table 24: Training interface timing parameters	63

Table 25: Training programmable parameters	65

Table 26: Low power control interface signals	67

Table 27: Low power wakeup encoding	68

Table 28: Low power control interface timing parameters	70

Table 29: Error interface signals	72

Table 30: Error interface timing parameter	73

Table 31: DFI 3.1 error codes	74

Table 32: Initialization handshake states	77

Table 33: Initialization timing parameters	79

Table 34: Training operations by memory type	81

Table 35: DDR3 command encoding on DFI control signals	89

Table 36: LPDDR3/LPDDR2 CA mapping onto dfi_address	90

Table 37: Write carriage timing parameters	94

Table 38: Write data stream patterns	96

Table 39: Read carriage timing parameters	100

Table 40: Timing parameter ownership	112

Table 41: Timing parameter families	113

Table 42: Complete DFI 3.1 timing parameter table	123

Table 43: DFI 3.1 programmable parameters	128

Table 44: v3.0 additions over v2.1.1	151

Table 45: v3.1 additions over v3.0	152

Table 46: Scoria DFI 3.1 implementation highlights	158

Table 47: DFI 3.1 features deliberately unimplemented in scoria	159

List of Waveforms

No waveforms in this document.

# 1	Simplified Dfi 3 1 Index

Generated: 2026-10-20

# 2	Introduction

## 2.1	What DFI is

The DDR PHY Interface (DFI) is a digital boundary between a DDR memory controller (MC) and the physical plane (PHY) that drives the actual DRAM pins. The MC thinks in commands, addresses, banks and bursts; the PHY turns those abstract transactions into edge-aligned clocks, strobes and data on the DRAM bus. DFI exists so that controller architects and PHY architects can work against a common contract instead of custom-wiring every MC/PHY pair.

At its simplest, DFI carries:

- Control: command/address signals such as dfi_cs_n, dfi_ras_n, dfi_cas_n, dfi_we_n, dfi_address, dfi_bank, plus DRAM-specific enables like dfi_cke and dfi_odt.

- Write data: dfi_wrdata, dfi_wrdata_en, dfi_wrdata_mask.

- Read data: dfi_rddata, dfi_rddata_valid, plus dfi_rddata_en from the MC to tell the PHY how many words to expect.

- Status, update, training, low-power and error sidebands that let the PHY and MC coordinate initialization, calibration, power states and error reporting.

DFI 3.1 supports DDR4, DDR3, DDR2, DDR1, LPDDR3, LPDDR2 and LPDDR1 DRAMs. Not every signal is required for every DRAM class; the controller for a given memory type drives only the subset it needs.

## 2.2	Why the seam exists

The MC and PHY run at different abstraction levels and often at different clock frequencies:

- The MC schedules commands and tracks bank state in its own clock domain.

- The PHY retimes commands to the DRAM clock, generates DQS, samples read data, and absorbs the analog timing complexity of the memory channel.

DFI decouples these two domains. The MC can issue a write command and trust that, tphy_wrlat cycles later, the PHY is ready for the data. The PHY can assert dfi_rddata_valid when it has captured read data, and the MC samples it in its own domain. Timing parameters such as tctrl_delay, tphy_wrlat, tphy_wrdata, trddata_en and tphy_rdlat are the contract that lets both sides agree on when information is valid without sharing a single clock.

## 2.3	Key features of DFI 3.1

- Frequency ratios of 1:1, 1:2 and 1:4 between the MC clock (DFI clock) and the PHY clock (DFI PHY clock). Commands are replicated per phase with a _pN suffix; read data words use _wN.

- A formal initialization and frequency-change protocol over dfi_init_start and dfi_init_complete.

- Separate low-power request signals for the control and data paths (dfi_lp_ctrl_req and dfi_lp_data_req), new in 3.1.

- PHY-requested training in non-DFI training mode (dfi_phylvl_req_cs_n, dfi_phylvl_ack_cs_n), new in 3.1.

- LPDDR3 CA training (dfi_calvl_*), new in 3.1.

- Error reporting (dfi_error, dfi_error_info) and parity/CRC/alert signals, inherited from 3.0 and mostly relevant to DDR4 systems.

## 2.4	Edition lineage

This book focuses on the subset of DFI 3.1 that a DDR3/LPDDR3 controller uses. The scoria controller inherited much of its datapath and command scheduling from a v2.1.1 baseline, then adopted v3.1 for its leveling and initialization model. Chapter 7 records exactly what changed and what scoria deliberately does not implement.

Source: DFI Specification v3.1 sections 1.0, 2.0

# 3	Organization

## 3.1	Sub-interface map for DFI 3.1

DFI 3.1 organizes its signals into eight sub-interfaces. Each interface has a direction (MC to PHY or PHY to MC), a set of signals and a family of timing parameters.

DFI 3.1 sub-interfaces

| Sub-interface | Direction | Purpose |
| --- | --- | --- |
| Control | MC -> PHY | Command, address, bank, chip select, CKE, ODT, reset |
| Write data | MC -> PHY | Write data, byte mask/DBI, write enable, write chip select |
| Read data | PHY -> MC | Read data, read valid, read chip select, DNV (LPDDR2) |
| Update | Both | MC-initiated and PHY-initiated update handshakes |
| Status | Both | Init, frequency ratio, clock disable, parity, alert |
| Training | Both | Gate training, read data eye training, write leveling, CA training |
| Low power control | MC -> PHY (ack PHY -> MC) | Low-power opportunity requests |
| Error | PHY -> MC | Error indication and error information |

## 3.2	Clocking

The DFI clock is the MC clock. All DFI signals are referenced to the rising edge of this clock. The DFI PHY clock is the same frequency as the DRAM clock, which is one half of the data rate. In a matched-frequency system the MC clock and the PHY clock are the same; in a frequency-ratio system the PHY clock runs at a multiple of the MC clock.

DFI allows ratios of 1:1, 1:2 and 1:4. The ratio is conveyed on dfi_freq_ratio (2 bits): 00 for 1:1, 01 for 1:2, 10 for 1:4, 11 reserved.

## 3.3	Frequency ratio and the _pN / _wN replication rule

When the PHY clock is faster than the MC clock, the MC must supply multiple phases of command and data information per MC clock cycle. DFI 3.1 handles this with replicated signals:

- Commands and other MC-to-PHY signals that are sampled per PHY clock phase carry a _pN suffix. For example, dfi_address_p0, dfi_address_p1, dfi_cs_n_p0, dfi_cs_n_p1. The suffix for phase 0 is optional.

- Read data and read-data qualifiers from PHY to MC are delivered as DFI data words with a _wN suffix. For example, dfi_rddata_w0, dfi_rddata_w1, dfi_rddata_valid_w0, dfi_rddata_valid_w1.

- Variable-pulse-width status signals such as dfi_alert_n carry an _aN suffix to preserve pulse width across the clock-domain boundary.

The MC may issue commands on any phase. The PHY must be able to accept a command on any and all phases. For signals driven in all phases, such as dfi_cke and dfi_odt, the MC must drive every phase.

## 3.4	PHY data slices and byte lanes

DFI data width is generally twice the DRAM data width, because one DFI data word carries both edges of a DRAM data transfer. The PHY is often organized as data slices, each handling a byte or a small group of bytes.

dfi_wrdata_en and dfi_rddata_en are ideally one bit per PHY data slice, though the spec allows the width to differ between MC and PHY as long as the lowest bit maps to the lowest data segment. dfi_rddata_valid is one bit per PHY data slice so that each slice can return data independently.

dfi_data_byte_disable tells the PHY which byte lanes are unused. The bit order matches dfi_wrdata and dfi_rddata. This signal is defined at initialization and expected to remain constant.

## 3.5	Simplified teaching model

All timing examples and walkthroughs in this book use a deliberately small configuration so that phase and burst counts are obvious:

- One rank, one chip select.

- 1:2 frequency ratio: two PHY phases (p0, p1) per MC clock.

- DFI data width of 32 bits (two 16-bit PHY data words per MC clock), split into two slices.

- Burst length 8 on the DRAM, so a read or write transfers four DFI data words.

- tphy_wrlat = 2, tphy_wrdata = 1, trddata_en = 2, tphy_rdlat = 4 unless stated otherwise.

Real systems are wider and may use 1:4 ratios; the scheduling rules are the same.

Source: DFI Specification v3.1 sections 2.0, 3.5.3, 4.8, 5.0

# 4	Acronyms

Acronyms used in this book

| Term | Meaning |
| --- | --- |
| ACT | DRAM Activate command |
| BL | Burst length |
| CA | Command/Address bus (LPDDR3/LPDDR2) |
| CRC | Cyclic redundancy check |
| DBI | Data bus inversion |
| DFI | DDR PHY Interface |
| DFI PHY clock | PHY-side clock, equal to DRAM clock frequency |
| DRAM | Dynamic random-access memory |
| DQS | Data strobe |
| MC | Memory controller |
| MPR | Multi-purpose register (DRAM training mode) |
| MRR | Mode register read (LPDDR3/LPDDR2) |
| MRW | Mode register write |
| NOP | No operation command |
| ODT | On-die termination |
| PHY | Physical plane that drives DRAM pins |
| PRE | DRAM Precharge command |
| RD | DRAM Read command |
| REF | DRAM Refresh command |
| UI | Unit interval (half a data word) |
| WR | DRAM Write command |

Suffix notation

| DFI signal group | Meaning |
| --- | --- |
| _pN | Phase N of a frequency-ratio signal, MC to PHY |
| _wN | DFI data word N, PHY to MC in frequency-ratio systems |
| _aN | PHY clock cycle N for variable-pulse-width signals |

Source: DFI Specification v3.1 section 6.0, Glossary

# 5	Control Interface

The control interface carries the command, address and control signals that the MC sends to the PHY. The PHY must preserve bit ordering and timing relationships so that, after its own tctrl_delay, the signals arrive at the DRAM pins correctly aligned.

## 5.1	Command/address signals

Control interface command and address signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_address | MC | DFI Address Width | none | Address bus. For DDR3 carries row/column address; for LPDDR3/LPDDR2 it maps the double-data-rate CA bus and must be at least 20 bits. |
| dfi_bank | MC | DFI Bank Width | none | Bank select bits. |
| dfi_bg | MC | DFI Bank Group Width | none | Bank group, DDR4 only. Unused for DDR3/LPDDR3. |
| dfi_cid | MC | DFI Chip ID Width | none | Stacked-device chip ID, DDR4 only. Unused for DDR3/LPDDR3. |
| dfi_act_n | MC | 1 bit | none | DDR4 command encoding; held constant for LPDDR3/LPDDR2. |
| dfi_cas_n | MC | DFI Control Width | 0x1 | Column address strobe / command bit. |
| dfi_ras_n | MC | DFI Control Width | 0x1 | Row address strobe / command bit. |
| dfi_we_n | MC | DFI Control Width | 0x1 | Write enable / command bit. |

In a frequency-ratio system these signals become dfi_address_pN, dfi_bank_pN, dfi_cas_n_pN, and so on. The MC may place a command on any phase; the PHY must accept commands on any and all phases.

## 5.2	DRAM enable and select signals

Control interface enable and select signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_cs_n | MC | DFI Chip Select Width | 0x1 | Chip select. Low active. For LPDDR3 CA training it carries the calibration command on the bit being trained. |
| dfi_cke | MC | DFI Chip Select Width | 0x0 or 0x1 | Clock enable. Must be driven in all phases. For LPDDR3 CA training it enables DRAM output drivers. |
| dfi_odt | MC | DFI Chip Select Width | 0x0 | On-die termination control. Used by DDR3, DDR2 and LPDDR3. Must be driven in all phases. |
| dfi_reset_n | MC | DFI Chip Select Width | 0x0 or 0x1 | DRAM reset. Used by DDR4 and DDR3. |

## 5.3	LPDDR3/LPDDR2 CA-bus mapping

For LPDDR3 and LPDDR2, the DRAM CA bus is double data rate and is mapped onto the 20-bit dfi_address bus. The PHY transmits dfi_address[9:0] on the rising CA edge and dfi_address[19:10] on the falling CA edge. During these operations the signals dfi_act_n, dfi_bank, dfi_bg, dfi_cid, dfi_cas_n, dfi_ras_n and dfi_we_n must be held constant, and dfi_address must drive a NOP until dfi_init_complete asserts.

## 5.4	Control timing parameters

Control interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| tcmd_lat | MC | DFI PHY clock cycles from dfi_cs_n assertion to the associated CA signals being driven. |
| tctrl_delay | PHY | DFI clock cycles from a control signal change until that change reaches the DRAM interface. |

tctrl_delay is what makes the DFI contract practical: the MC issues signals on its clock, and the PHY promises to deliver them to the DRAM after a fixed, rounded-up delay.

Source: DFI Specification v3.1 sections 3.1, 4.2, Table 4, Table 5

# 6	Write Data Interface

The write data interface carries write data from the MC to the PHY and tells the PHY when that data is valid. The PHY then retimes the data onto the DRAM DQ/DQS pins.

## 6.1	Write data signals

Write data interface signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_wrdata | MC | DFI Data Width | none | Write data bus. Data is driven tphy_wrdata cycles after dfi_wrdata_en asserts. |
| dfi_wrdata_en | MC | DFI Data Enable Width | 0x0 | Write data and mask enable. Asserted tphy_wrlat cycles after a write command; held for the duration of the data transfer. |
| dfi_wrdata_mask | MC | DFI Data Width / 8 | none | Per-byte mask, or write DBI when DBI is enabled. Same timing as dfi_wrdata. |
| dfi_wrdata_cs_n | MC | CS Width x Data Enable Width | none | Optional write data chip select. Indicates the target rank for the data, and during write leveling indicates the rank being trained. |

In a frequency-ratio system these signals are replicated as dfi_wrdata_pN, dfi_wrdata_en_pN, dfi_wrdata_mask_pN and dfi_wrdata_cs_n_pN. The _pN suffix gives the value for each PHY clock phase.

## 6.2	dfi_wrdata_en behavior

dfi_wrdata_en is the central enable for write data. One continuous assertion may cover data for multiple back-to-back write commands. The signal is asserted tphy_wrlat cycles after the write command on the control interface, and the actual data appears tphy_wrdata cycles after the enable. The enable de-asserts tphy_wrdata cycles before the last valid data word.

For a non-contiguous write, dfi_wrdata_en de-asserts between commands. For contiguous writes it stays asserted across command boundaries.

## 6.3	Write data mask and DBI

When DBI is disabled, dfi_wrdata_mask[N] masks the byte dfi_wrdata[N*8+7:N*8]. When DBI is enabled and phydbi_mode = 0, the same bus carries the write DBI inversion bits: 1 means the corresponding byte is inverted before being driven to DRAM. Mask and DBI are mutually exclusive.

## 6.4	Write chip select

dfi_wrdata_cs_n is optional in normal operation. When it is used, the PHY may need extra spacing (tphy_wrcsgap) between commands that target different chip selects so it can re-time its data path. The signal is driven tphy_wrcslat cycles after the write command and held for at least the data transfer length plus the gap time.

## 6.5	Write data timing parameters

Write data interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| tphy_wrlat | PHY | Cycles from write command to dfi_wrdata_en assertion. |
| tphy_wrdata | PHY | Cycles from dfi_wrdata_en assertion to dfi_wrdata driven. |
| twrdata_delay | System | Cycles from dfi_wrdata_en assertion to completion of the write data transfer on the DRAM bus. |
| tphy_wrcslat | PHY | Cycles from write command to dfi_wrdata_cs_n assertion. |
| tphy_wrcsgap | PHY | Additional cycles required between commands when changing target chip select. |
| tphy_crcmin_lat, tphy_crcmax_lat | System | Min/max cycles from CRC data on dfi_wrdata_en to CRC error on dfi_alert_n. |

## 6.6	Write data programmable parameters

Write data programmable parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| phycrc_mode | PHY | 0 = MC generates CRC; 1 = PHY generates CRC. |
| phydbi_mode | PHY | 0 = MC generates DBI and inverts data; 1 = PHY does. |

CRC and DBI are DDR4-era features and are not used by the scoria DDR3/LPDDR3 controller, but the signals and parameters are part of DFI 3.1 and are noted here for completeness.

Source: DFI Specification v3.1 sections 3.2, 4.4, Table 6, Table 7, Table 8

# 7	Read Data Interface

The read data interface returns captured DRAM data from the PHY to the MC. The MC tells the PHY how many words to expect; the PHY returns data when it is ready and marks each word valid.

## 7.1	Read data signals

Read data interface signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_rddata_en | MC | DFI Data Enable Width | 0x0 | Tells the PHY a read is underway and how many contiguous words are expected. Asserted trddata_en cycles after the read command. |
| dfi_rddata | PHY | DFI Data Width | none | Read data bus. Expected within tphy_rdlat cycles after dfi_rddata_en assertion. |
| dfi_rddata_valid | PHY | DFI Read Data Valid Width | 0x0 | One bit per PHY data slice, asserted with the corresponding data. |
| dfi_rddata_dnv | PHY | DFI Data Width / 8 | 0x0 | Data-not-valid indicator, per byte. Used by LPDDR2. |
| dfi_rddata_dbi_n | PHY | DFI DBI Width | none | Read DBI inversion bits. Used when phydbi_mode = 0 and DBI is enabled. |
| dfi_rddata_cs_n | MC | CS Width x Data Enable Width | none | Optional read data chip select. During read training it indicates the rank being trained; in normal operation it identifies the target rank. |

In a frequency-ratio system, dfi_rddata_en becomes dfi_rddata_en_pN while the return data and qualifiers become dfi_rddata_wN, dfi_rddata_valid_wN, dfi_rddata_dnv_wN and dfi_rddata_dbi_n_wN. The _wN suffix identifies the DFI data word number.

## 7.2	dfi_rddata_en and dfi_rddata_valid

dfi_rddata_en is the MC’s prediction of how much read data is coming. It is asserted trddata_en cycles after the read command and held for the number of contiguous data words expected. The PHY responds with dfi_rddata_valid; there is a one-to-one correspondence between assertion cycles of dfi_rddata_en and dfi_rddata_valid.

tphy_rdlat is a maximum: the PHY may return data earlier than the programmed value. Each bit of dfi_rddata_valid corresponds to one PHY data slice, so different slices may return data on different cycles.

## 7.3	Read chip select

Like its write counterpart, dfi_rddata_cs_n is optional. When used, the PHY may require tphy_rdcsgap extra cycles between read commands that switch target chip selects. The signal is asserted tphy_rdcslat cycles after the read command.

## 7.4	LPDDR2 data-not-valid

LPDDR2 defines data words that are not all valid. In that case the PHY asserts dfi_rddata_valid for the returning word and simultaneously uses dfi_rddata_dnv to mark the invalid bytes. This is the only DRAM class in the v3.1 inventory that uses dfi_rddata_dnv.

## 7.5	Read data timing parameters

Read data interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| trddata_en | System | Cycles from read command to dfi_rddata_en assertion. |
| tphy_rdlat | PHY | Maximum cycles from dfi_rddata_en assertion to dfi_rddata_valid assertion. |
| tphy_rdcslat | PHY | Cycles from read command to dfi_rddata_cs_n assertion. |
| tphy_rdcsgap | PHY | Additional cycles required between commands when changing target chip select. |

trddata_en and tphy_rdlat are normally held constant while commands are flowing; they may only be changed when the bus is idle.

Source: DFI Specification v3.1 sections 3.3, 4.5, Table 9, Table 10, Table 11

# 8	Update Interface

The update interface lets the PHY and MC pause the DFI bus to perform calibration, timing adjustments or other maintenance. The DFI bus must be idle during an update: no commands in flight, all read and write data finished, and write data fully retired on the DRAM bus.

## 8.1	MC-initiated update

MC-initiated update signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_ctrlupd_req | MC | 1 bit | 0x0 | MC tells the PHY that the DFI bus will be idle and the PHY may update. |
| dfi_ctrlupd_ack | PHY | 1 bit | 0x0 | Optional PHY acknowledge. If asserted it must be before tctrlupd_min expires. |

dfi_ctrlupd_req must be asserted for at least tctrlupd_min cycles and no more than tctrlupd_max cycles. The maximum interval between assertions is tctrlupd_interval. The PHY is not required to acknowledge, but if it does the DFI bus stays idle while both request and acknowledge are asserted.

After self-refresh exit, dfi_ctrlupd_req must be asserted before read or write traffic resumes.

## 8.2	PHY-initiated update

PHY-initiated update signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_phyupd_req | PHY | 1 bit | 0x0 | PHY requests idle time on the DFI bus. |
| dfi_phyupd_type | PHY | 2 bits | none | Selects one of four update types. |
| dfi_phyupd_ack | MC | 1 bit | 0x0 | MC grants the request. |

When the PHY needs the bus idle, it asserts dfi_phyupd_req with a valid dfi_phyupd_type. The MC must respond with dfi_phyupd_ack within tphyupd_resp cycles. The request remains asserted until the update completes, and must de-assert before tphyupd_typeX cycles have elapsed after the acknowledge. The MC must not assert dfi_ctrlupd_req and dfi_phyupd_ack at the same time.

## 8.3	Update timing parameters

Update interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| tctrlupd_min | MC | Minimum cycles dfi_ctrlupd_req must be asserted. |
| tctrlupd_max | MC | Maximum cycles dfi_ctrlupd_req may be asserted. |
| tctrlupd_interval | MC | Maximum cycles the MC may wait between dfi_ctrlupd_req assertions. |
| tphyupd_type0 | PHY | Max cycles for dfi_phyupd_type = 0. |
| tphyupd_type1 | PHY | Max cycles for dfi_phyupd_type = 1. |
| tphyupd_type2 | PHY | Max cycles for dfi_phyupd_type = 2. |
| tphyupd_type3 | PHY | Max cycles for dfi_phyupd_type = 3. |
| tphyupd_resp | PHY | Max cycles from dfi_phyupd_req to dfi_phyupd_ack. |

## 8.4	Simultaneous requests

If dfi_ctrlupd_req and dfi_phyupd_req are asserted together, the MC is forbidden from asserting dfi_phyupd_ack at the same time as dfi_ctrlupd_req. The PHY may de-assert its own request in this case, and the acknowledged request follows its normal protocol.

Source: DFI Specification v3.1 sections 3.4, 4.6, Table 12, Table 13

# 9	Status Interface

The status interface conveys configuration and status information between the MC and the PHY. It covers initialization, frequency ratio, DRAM clock disable, command parity and the alert signal.

## 9.1	Initialization and configuration signals

Status interface initialization and configuration signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_init_start | MC | 1 bit | 0x0 or 0x1 | At init, indicates dfi_data_byte_disable and dfi_freq_ratio are valid. During normal operation, requests a frequency change. |
| dfi_init_complete | PHY | 1 bit | 0x0 | PHY is ready for normal DFI transactions. All command/status signals must be at default until this asserts. |
| dfi_data_byte_disable | MC | DFI Data Width / 8 | none | Marks unused byte lanes so the PHY may power them down. Defined at initialization only. |
| dfi_freq_ratio | MC | 2 bits | none | 00=1:1, 01=1:2, 10=1:4, 11=reserved. Required only for devices supporting multiple ratios. |
| dfi_dram_clk_disable | MC | DFI Chip Select Width | 0x0 | When asserted the PHY holds DRAM clocks at a constant value. |

Initialization completes when both dfi_init_start and dfi_init_complete are asserted simultaneously for at least one DFI clock cycle. The PHY may assert dfi_init_complete before dfi_init_start if it does not need the configuration signals.

## 9.2	Parity and alert signals

Status interface parity and alert signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_parity_in | MC | 1 bit | 0x0 or 0x1 | Command parity bit, one per command. 0=even, 1=odd over the covered CA signals. Phase-indexed in frequency-ratio systems. |
| dfi_alert_n_aN | PHY | DFI Alert Width | 1 | Active-low CRC or command-parity error indicator propagated from DRAM ALERT_N. Pulse width preserved with _aN suffix. |

dfi_parity_in covers dfi_address, dfi_bank, dfi_bg, dfi_act_n, dfi_cas_n, dfi_ras_n and dfi_we_n. It is valid for one cycle per command and is delayed by tparin_lat relative to the command. The PHY delays it by the same tctrl_delay as the command bus. dfi_alert_n asserts within tphy_paritylat cycles of the associated parity bit.

These signals are DDR4/DDR3 CA-parity and CRC features and are not used by the scoria DDR3/LPDDR3 controller.

## 9.3	Status timing parameters

Status interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| tinit_start | MC | Max cycles from dfi_init_start assertion to PHY de-asserting dfi_init_complete for a frequency change. |
| tinit_complete | PHY | Max cycles from dfi_init_start de-assertion to dfi_init_complete re-assertion after re-initialization. |
| tdram_clk_disable | PHY | Cycles from dfi_dram_clk_disable assertion to DRAM clock held constant. |
| tdram_clk_enable | PHY | Cycles from dfi_dram_clk_disable de-assertion to first valid DRAM clock edge. |
| tparin_lat | MC | Cycles from command to associated dfi_parity_in. |
| tphy_paritylat | PHY | Max cycles from dfi_parity_in to dfi_alert_n assertion. |

Source: DFI Specification v3.1 sections 3.5, 4.1, 4.7, 4.9, 4.10, Table 14, Table 15

# 10	Training Interface

DFI 3.1 defines four training operations: gate training, read data eye training, write leveling and CA training. Gate training and read data eye training together are called “read training”. The DRAM class determines which operations a system uses.

Training operations in DFI 3.1

| Operation | DRAM classes | Purpose |
| --- | --- | --- |
| Gate training | DDR4, DDR3, LPDDR3, LPDDR2 | Center the read DQS gate in the read preamble. |
| Read data eye training | DDR4, DDR3, LPDDR3, LPDDR2 | Center read DQS in the DQ data eye. |
| Write leveling | DDR4, DDR3, LPDDR3 | Align write DQS to the DRAM clock. |
| CA training | LPDDR3 only | Optimize CA bus setup/hold relative to the memory clock. |

## 10.1	Read training signals

Read training signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_rdlvl_en | MC | DFI Read Leveling MC I/F Width | 0x0 | Enables read data eye training logic in the PHY. |
| dfi_rdlvl_req | PHY | DFI Read Leveling PHY I/F Width | 0x0 | PHY requests read data eye training; MC must respond with dfi_rdlvl_en within trdlvl_resp cycles. |
| dfi_rdlvl_gate_en | MC | DFI Read Leveling MC I/F Width | 0x0 | Enables gate training logic in the PHY. |
| dfi_rdlvl_gate_req | PHY | DFI Read Leveling PHY I/F Width | 0x0 | PHY requests gate training; MC must respond with dfi_rdlvl_gate_en within trdlvl_resp cycles. |
| dfi_rdlvl_resp | PHY | DFI Read Leveling Response Width | none | Per-slice indication that training is complete. |
| dfi_phy_rdlvl_cs_n | PHY | CS Width x Read Training PHY I/F Width | 0x1 | Target chip select for read data eye training. |
| dfi_phy_rdlvl_gate_cs_n | PHY | CS Width x Read Training PHY I/F Width | 0x1 | Target chip select for gate training. |

When either read-training enable de-asserts, both the MC and the PHY must reset their DFI read data word pointers to zero.

## 10.2	Write leveling signals

Write leveling signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_wrlvl_en | MC | DFI Write Leveling MC I/F Width | 0x0 | Enables write leveling logic in the PHY. |
| dfi_wrlvl_req | PHY | DFI Write Leveling PHY I/F Width | 0x0 | PHY requests write leveling; MC must respond with dfi_wrlvl_en within twrlvl_resp cycles. |
| dfi_wrlvl_strobe | MC | DFI Write Leveling MC I/F Width | 0x0 | One-DFI-clock pulse that tells the PHY to capture the write leveling response from DQ. |
| dfi_wrlvl_resp | PHY | DFI Write Leveling Response Width | none | Per-slice indication that write leveling is complete. |
| dfi_phy_wrlvl_cs_n | PHY | CS Width x Write Leveling PHY I/F Width | 0x1 | Target chip select for write leveling. |

Write leveling is the operation that DDR3 adds over DDR2, and it is the main reason scoria moved from a v2.1.1-style interface to v3.1.

## 10.3	CA training signals (LPDDR3)

CA training signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_calvl_en | MC | DFI CA Training MC I/F Width | none | Enables LPDDR3 CA training logic. Also acknowledges dfi_calvl_req. |
| dfi_calvl_req | PHY | DFI CA Training PHY I/F Width | none | Optional PHY request for CA training. |
| dfi_calvl_capture | MC | DFI CA Training MC I/F Width | none | One-cycle pulse telling the PHY to capture the CA value from DQ. |
| dfi_calvl_resp | PHY | DFI CA Training Response Width | none | 00=not done, 01=done/do not change segment, 10=done/change segment, 11=complete. |
| dfi_phy_calvl_cs_n | PHY | DFI Chip Select Width | none | Target chip select for CA training. |

During CA training the control signals dfi_address, dfi_cke and dfi_cs_n take on training-specific roles: dfi_address carries background and calibration patterns, dfi_cke enables DRAM output drivers, and dfi_cs_n becomes the calibration command.

## 10.4	PHY-requested training in non-DFI training mode

PHY-requested training signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_phylvl_req_cs_n | PHY | DFI Rank Width | none | PHY requests control of the DRAM bus to train one chip select. |
| dfi_phylvl_ack_cs_n | MC | DFI Chip Select Width | none | MC grants the request when the bus is idle and all pages are closed. |

This interface, new in v3.1, lets the PHY ask for the DRAM bus during normal operation so it can perform its own training without using the DFI training sequences. The MC idles the bus and grants the request.

## 10.5	Common training controls

Common training control signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_lvl_pattern | MC | 4 bits (DDR4) or 1 bit (LPDDR3/LPDDR2) | 0x0 / 0x1 | Selects the training pattern for gate/read data eye training. |
| dfi_lvl_periodic | MC | DFI Leveling PHY I/F Width | 0 | 0=full/long training; 1=periodic/short training. |

## 10.6	Training timing parameters

Training interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| trdlvl_en | PHY | Minimum cycles from read training enable to first read command. |
| trdlvl_resp | MC | Max cycles from dfi_rdlvl_req/dfi_rdlvl_gate_req to enable assertion. |
| trdlvl_max | MC | Max cycles the MC waits for dfi_rdlvl_resp. |
| trdlvl_rr | PHY | Minimum cycles between read training reads. |
| twrlvl_en | PHY | Minimum cycles from write leveling enable to first dfi_wrlvl_strobe. |
| twrlvl_resp | MC | Max cycles from dfi_wrlvl_req to dfi_wrlvl_en assertion. |
| twrlvl_max | MC | Max cycles the MC waits for dfi_wrlvl_resp. |
| twrlvl_ww | PHY | Minimum cycles between dfi_wrlvl_strobe assertions. |
| tcalvl_en | PHY | Minimum cycles from dfi_calvl_en to dfi_cke de-assertion. |
| tcalvl_capture | PHY | Cycles from calibration command to dfi_calvl_capture pulse. |
| tcalvl_resp | MC | Max cycles from dfi_calvl_req to dfi_calvl_en. |
| tcalvl_max | MC | Max cycles the MC waits for dfi_calvl_resp. |
| tcalvl_cc | PHY | Minimum cycles between calibration commands. |
| tphylvl | PHY | Max cycles dfi_phylvl_req_cs_n may stay asserted after acknowledge. |
| tphylvl_resp | PHY | Max cycles from request to acknowledge for PHY-requested training. |

## 10.7	Training programmable parameters

Training programmable parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| phyrdlvl_en | PHY | PHY supports read data eye training. |
| phyrdlvl_gate_en | PHY | PHY supports gate training. |
| phywrlvl_en | PHY | PHY supports write leveling. |
| phycalvl_en | PHY | PHY supports CA training. |

Source: DFI Specification v3.1 sections 3.6, 4.11, Table 16, Table 17, Table 18

# 11	Low Power Control Interface

The low power control interface lets the MC inform the PHY of an opportunity to enter a low-power state. DFI 3.1 splits the single v2.1 dfi_lp_req into two requests: one for the control path and one for the data path. This lets the MC and PHY sleep the paths independently.

## 11.1	Low power signals

Low power control interface signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_lp_ctrl_req | MC | 1 bit | 0x0 | MC indicates no more commands will be sent on the control interface. |
| dfi_lp_data_req | MC | 1 bit | 0x0 | MC indicates no more commands will be sent on the data interface. |
| dfi_lp_ack | PHY | 1 bit | 0x0 | Optional PHY acknowledge. |
| dfi_lp_wakeup | MC | 4 bits | none | Encoded wakeup time. Valid only while a request is asserted. |

Only one low-power request may be outstanding until it is acknowledged or aborted. When exiting, if both requests are asserted they must de-assert simultaneously.

## 11.2	dfi_lp_wakeup encoding

Low power wakeup encoding

| dfi_lp_wakeup | Wakeup time (DFI clock cycles) |
| --- | --- |
| 0000 | 16 |
| 0001 | 32 |
| 0010 | 64 |
| 0011 | 128 |
| 0100 | 256 |
| 0101 | 512 |
| 0110 | 1024 |
| 0111 | 2048 |
| 1000 | 4096 |
| 1001 | 8192 |
| 1010 | 16384 |
| 1011 | 32768 |
| 1100 | 65536 |
| 1101 | 131072 |
| 1110 | 262144 |
| 1111 | unlimited |

The MC must hold dfi_lp_wakeup constant until dfi_lp_ack asserts. After acknowledge it may increase the value but never decrease it. The value at the moment the request de-asserts defines the tlp_wakeup time.

## 11.3	Low power handshake

- The MC asserts dfi_lp_ctrl_req or dfi_lp_data_req with a static dfi_lp_wakeup value.

- The PHY may assert dfi_lp_ack within tlp_resp cycles.

- While request and acknowledge are both asserted, the PHY may remain in a low-power state.

- When the request de-asserts, the PHY must return to normal operation and de-assert dfi_lp_ack within tlp_wakeup cycles.

If the PHY does not acknowledge within tlp_resp cycles, the MC may de- assert the request and treat the opportunity as declined.

## 11.4	Low power timing parameters

Low power control interface timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| tlp_resp | MC | Max cycles from request assertion to dfi_lp_ack assertion. |
| tlp_wakeup | MC | Max cycles from request de-assertion to dfi_lp_ack de-assertion. |

In the scoria DDR3/LPDDR3 controller this interface is exposed for spec compliance but is not consumed by the target PHY family.

Source: DFI Specification v3.1 sections 3.7, 4.12, Table 19, Table 20

# 12	Error Interface

The error interface lets the PHY report error conditions to the MC. It is optional for both MC and PHY, and is not phased for frequency-ratio systems.

## 12.1	Error signals

Error interface signals

| Signal | From | Width | Default | What it does |
| --- | --- | --- | --- | --- |
| dfi_error | PHY | DFI Error Width | 0 | Indicates the PHY has detected an error. Typically one bit per data slice plus one bit for control. |
| dfi_error_info | PHY | DFI Error Width x 4 | 0 | Additional information about the error source. Valid only while dfi_error is asserted. |

The PHY may implement dfi_error as a single bit or as one bit per data slice plus one control bit. dfi_error_info provides four bits per error instance. The specification defines only a few general encodings; the rest are design-specific. Undriven bits must be tied low at the MC.

## 12.2	Error timing

Error interface timing parameter

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| terror_resp | PHY | Max cycles from the affected transaction to dfi_error assertion. |

For data errors the reference point is dfi_wrdata_en or dfi_rddata_en. For command errors the reference point is the command itself. Because terror_resp is a maximum, the MC may not be able to correlate the error with a single transaction.

## 12.3	Spec-defined error codes

DFI 3.1 error codes

| dfi_error_info | Name | Meaning |
| --- | --- | --- |
| 0000 | General purpose | PHY general-purpose error. |
| 0001 | Internal PHY | PHY-specific internal error. |
| 0010-0111 | Reserved | Reserved for future definition. |
| 1000-1111 | User-defined | Available for design-specific codes. |

The scoria controller uses the error indication but ignores the DDR4- specific encodings, treating dfi_error as a generic PHY fault flag.

Source: DFI Specification v3.1 sections 3.8, 4.13, Table 21, Table 22, Table 27

# 13	Reset and Initialization

DFI 3.1 does not dictate a power-up sequence for either the MC or the PHY, but it does define the handshake that marks the boundary between initialization and normal operation.

## 13.1	Default values before dfi_init_complete

Until dfi_init_complete asserts, every command and status signal must be held at its default value. The defaults come from the interface tables:

- High-active signals default to 0 (for example dfi_odt, dfi_wrdata_en).

- Low-active signals default to 1 (for example dfi_cs_n, dfi_we_n).

- Buses have no specified default.

- State-specific signals such as dfi_dram_clk_disable and dfi_parity_in take their system-defined values.

For LPDDR3 and LPDDR2 systems, dfi_address must drive a NOP command until dfi_init_complete asserts.

## 13.2	The dfi_init_start / dfi_init_complete handshake

dfi_init_start has two functions:

- At initialization it tells the PHY that dfi_data_byte_disable and dfi_freq_ratio are valid. The PHY may wait for this assertion before asserting dfi_init_complete if it needs those values.

- During normal operation it requests a frequency change.

Initialization completes when both dfi_init_start and dfi_init_complete are asserted for at least one DFI clock cycle. After that, the MC may hold or de-assert dfi_init_start.

Initialization handshake states

| Phase | dfi_init_start | dfi_init_complete | Meaning |
| --- | --- | --- | --- |
| Power-up | driven by MC | 0 | PHY not ready. |
| Config valid | 1 | 0 | MC has defined ratio and byte disable. |
| Complete | 1 | 1 for >=1 cycle | PHY ready; normal operation may begin. |

## 13.3	Frequency-change reuse of the init signals

During normal operation the MC may assert dfi_init_start again to request a frequency change. The PHY must de-assert dfi_init_complete within tinit_start cycles to acknowledge the request. If it does not, the MC must abort and release dfi_init_start.

Once acknowledged, the MC holds dfi_init_start asserted while the clock frequency changes. When the change is done, the MC de-asserts dfi_init_start; the PHY then re-initializes and re-asserts dfi_init_complete within tinit_complete cycles.

The DFI bus must be kept at valid and stable levels throughout the frequency change. Both sides reset their DFI read data word pointers to zero when dfi_init_start asserts.

## 13.4	Initialization timing parameters

Initialization timing parameters

| Parameter | Defined by | Meaning |
| --- | --- | --- |
| tinit_start | MC | Max cycles for the PHY to acknowledge a frequency-change request. |
| tinit_complete | PHY | Max cycles for the PHY to re-assert dfi_init_complete after dfi_init_start de-asserts. |

Source: DFI Specification v3.1 sections 3.5.1, 4.1, 4.9, Figure 3, Figure 4, Figure 5

# 14	Training in DFI 3.1

DFI 3.1 supports four training operations. A system does not have to use them all; the DRAM class and the PHY capabilities determine which are required.

## 14.1	Which training is used where

Training operations by memory type

| Training | DDR3 | LPDDR3 | Notes |
| --- | --- | --- | --- |
| Gate training | Yes | Yes | Centers read DQS gate in preamble. |
| Read data eye training | Yes | Yes | Centers read DQS in DQ eye. |
| Write leveling | Yes | Yes | Aligns write DQS to DRAM clock. |
| CA training | No | Yes | LPDDR3 only; optimizes CA timing. |

For DFI compliance the MC must support the training operations that apply to the DRAM types it supports. The PHY may optionally support each operation.

## 14.2	DFI training mode

In DFI training mode the MC sets the DRAM into the appropriate training mode, issues the necessary read commands or write strobes, and lets the PHY adjust its own delays. The PHY signals completion by asserting dfi_rdlvl_resp, dfi_wrlvl_resp or dfi_calvl_resp. The MC then de- asserts the enable and returns to normal operation.

The MC must finish all normal traffic before starting training. During training the only timing parameters that matter are:

- trdlvl_rr: minimum spacing between read training reads.

- twrlvl_ww: minimum spacing between write leveling strobes.

- tcalvl_cc: minimum spacing between CA calibration commands.

## 14.3	Read training sequence

- The PHY asserts dfi_rdlvl_req (data eye) or dfi_rdlvl_gate_req (gate). It may indicate the target chip select on dfi_phy_rdlvl_cs_n or dfi_phy_rdlvl_gate_cs_n.

- The MC responds with dfi_rdlvl_en or dfi_rdlvl_gate_en within trdlvl_resp cycles.

- The MC issues training reads at least trdlvl_rr cycles apart. For DDR3 these are normal reads; for LPDDR3 they are mode-register reads.

- The PHY completes its delay search and asserts dfi_rdlvl_resp.

- The MC de-asserts the enable.

## 14.4	Write leveling sequence

- The PHY asserts dfi_wrlvl_req, optionally with dfi_phy_wrlvl_cs_n.

- The MC responds with dfi_wrlvl_en within twrlvl_resp cycles.

- The MC issues MRS commands to enter and exit write leveling mode in the DRAM.

- The MC issues dfi_wrlvl_strobe pulses at least twrlvl_ww cycles apart; the PHY captures the DQ response after each strobe.

- The PHY asserts dfi_wrlvl_resp when complete.

- The MC de-asserts dfi_wrlvl_en.

## 14.5	CA training sequence (LPDDR3)

- The PHY may assert dfi_calvl_req with dfi_phy_calvl_cs_n.

- The MC responds with dfi_calvl_en within tcalvl_resp cycles.

- The MC drives calibration patterns on dfi_address and calibration commands on dfi_cs_n, with dfi_cke used to enable DRAM outputs.

- The MC asserts dfi_calvl_capture tcalvl_capture cycles after a calibration command so the PHY captures the returned DQ value.

- The PHY reports progress on dfi_calvl_resp.

- The MC de-asserts dfi_calvl_en when dfi_calvl_resp indicates completion.

## 14.6	Periodic vs full training

dfi_lvl_periodic selects the training length:

- 0: full training, used at initialization, self-refresh restart or after large delay changes.

- 1: periodic training, a shorter tuning pass during normal operation.

## 14.7	Non-DFI training mode and PHY-requested training

If the PHY supports non-DFI training, it may request the DRAM bus with dfi_phylvl_req_cs_n. The MC grants it with dfi_phylvl_ack_cs_n only when the bus is idle and all pages are closed. The PHY then trains on its own and returns control when done. This is new in v3.1 and is separate from the DFI training sequences above.

Source: DFI Specification v3.1 sections 3.6, 4.11, Table 16, Table 17, Table 18

# 15	Command Mapping

On the DFI control interface, DRAM commands are encoded by the command signals dfi_act_n, dfi_ras_n, dfi_cas_n and dfi_we_n, together with the address, bank and chip-select buses. The PHY forwards these values to the DRAM pins with the timing controlled by tcmd_lat and tctrl_delay.

## 15.1	DDR3 command encoding on DFI

For DDR3 the command mapping is the familiar JEDEC encoding:

DDR3 command encoding on DFI control signals

| Command | dfi_act_n | dfi_ras_n | dfi_cas_n | dfi_we_n |
| --- | --- | --- | --- | --- |
| Activate (ACT) | 1 | 0 | 1 | 1 |
| Read (RD) | 1 | 1 | 0 | 1 |
| Write (WR) | 1 | 1 | 0 | 0 |
| Precharge (PRE) | 1 | 0 | 1 | 0 |
| Refresh (REF) | 1 | 0 | 0 | 1 |
| Mode Register Set (MRS) | 1 | 0 | 0 | 0 |
| NOP | 1 | 1 | 1 | 1 |
| Deselect | 1 | 1 | 1 | 0 |

dfi_act_n is held high for DDR3; it is only meaningful for DDR4, where an active-low dfi_act_n repurposes dfi_ras_n, dfi_cas_n and dfi_we_n as upper row address bits A16, A15 and A14.

## 15.2	LPDDR3/LPDDR2 CA-bus mapping

For LPDDR3 and LPDDR2 the DRAM has no separate RAS/CAS/WE pins. Instead the command, address and bank information travel on a double-data-rate CA bus. DFI maps this CA bus onto the dfi_address bus:

LPDDR3/LPDDR2 CA mapping onto dfi_address

| dfi_address bit | CA rising edge bit | CA falling edge bit |
| --- | --- | --- |
| 9:0 | CA9:CA0 | - |
| 19:10 | - | CA9:CA0 |

The PHY transmits dfi_address[9:0] on the rising CA edge and dfi_address[19:10] on the falling CA edge. During LPDDR3/LPDDR2 operation the command signals dfi_ras_n, dfi_cas_n, dfi_we_n, dfi_act_n, dfi_bank, dfi_bg and dfi_cid must be held constant.

## 15.3	Chip select and enable usage

dfi_cs_n is low active and selects the target rank. dfi_cke enables the DRAM clock. dfi_odt controls on-die termination and is relevant for DDR3, DDR2 and LPDDR3. All three are driven in every phase in a frequency- ratio system.

dfi_reset_n is used by DDR4 and DDR3. The scoria controller drives it from its init sequencer and presents it as dfi_reset_n_o at the DFI boundary.

## 15.4	Command timing relationship

The MC drives dfi_cs_n and the command/address signals according to tcmd_lat. If tcmd_lat is non-zero, the CA signals are driven that many DFI PHY clock cycles after dfi_cs_n assertion. The PHY then applies its own tctrl_delay before driving the DRAM pins.

Source: DFI Specification v3.1 sections 3.1, 4.2, Table 4

# 16	Write Carriage

A DFI write transaction has two separable delays: the time from the write command to the write-data enable, and the time from the enable to the data itself. The PHY defines both delays; the MC schedules around them.

## 16.1	Write timing parameters in use

Write carriage timing parameters

| Parameter | Meaning |
| --- | --- |
| tphy_wrlat | Cycles from write command to dfi_wrdata_en assertion. |
| tphy_wrdata | Cycles from dfi_wrdata_en assertion to dfi_wrdata driven. |
| twrdata_delay | Cycles from dfi_wrdata_en assertion to completion on the DRAM bus. |

Both tphy_wrlat and tphy_wrdata can be zero. They must be held constant while commands are flowing; they may only be changed when the bus is idle.

## 16.2	Single write transaction

- The MC issues a write command on the control interface.

- tphy_wrlat cycles later, the MC asserts dfi_wrdata_en.

- tphy_wrdata cycles after that, the MC drives the first dfi_wrdata word.

- The MC holds dfi_wrdata_en for the length of the transfer. For a burst-length-8 write this is four DFI data words.

- dfi_wrdata_en de-asserts tphy_wrdata cycles before the last valid data word.

- The write completes on the DRAM bus twrdata_delay cycles after dfi_wrdata_en first asserted.

## 16.3	Contiguous vs non-contiguous writes

For back-to-back writes with matching timing, dfi_wrdata_en can stay asserted across command boundaries and the data stream is contiguous. If the command spacing is larger or the timing parameters differ, the enable and data de-assert between commands.

Write data stream patterns

| Case | dfi_wrdata_en | Data stream |
| --- | --- | --- |
| Back-to-back, same parameters | Continuous | Contiguous |
| Interrupted first burst | Asserted for partial first burst plus full second burst | Contiguous for part of first plus all of second |
| Independent writes | De-asserts between commands | Non-contiguous |

## 16.4	Write chip select timing

When dfi_wrdata_cs_n is used, the PHY defines tphy_wrcslat and tphy_wrcsgap. The MC drives the chip select tphy_wrcslat cycles after the write command and holds it for the data transfer length plus the gap time. If the next write targets a different chip select, the MC must add tphy_wrcsgap cycles between commands.

## 16.5	DBI and CRC notes

When DBI is enabled and phydbi_mode = 0, dfi_wrdata_mask carries the per-byte inversion bits instead of mask bits. When CRC is enabled and phycrc_mode = 0, the MC sends the CRC data word on the dfi_wrdata bus and extends dfi_wrdata_en accordingly. These are DDR4 features and are not used by the scoria DDR3/LPDDR3 controller.

Source: DFI Specification v3.1 sections 3.2, 4.4, Table 6, Table 7

# 17	Read Carriage

A DFI read transaction is split into the MC’s request (dfi_rddata_en) and the PHY’s response (dfi_rddata plus dfi_rddata_valid). The MC knows how many words to expect; the PHY returns them when ready.

## 17.1	Read timing parameters in use

Read carriage timing parameters

| Parameter | Meaning |
| --- | --- |
| trddata_en | Cycles from read command to dfi_rddata_en assertion. |
| tphy_rdlat | Maximum cycles from dfi_rddata_en assertion to dfi_rddata_valid assertion. |

trddata_en and tphy_rdlat are normally held constant while commands flow. They may only be changed when the bus is idle.

## 17.2	Single read transaction

- The MC issues a read command on the control interface.

- trddata_en cycles later, the MC asserts dfi_rddata_en for the expected number of data words. For burst-length-8 this is four DFI data words.

- Within tphy_rdlat cycles, the PHY asserts dfi_rddata_valid and drives the first dfi_rddata word.

- The PHY holds dfi_rddata_valid for the same number of cycles that dfi_rddata_en was asserted.

- The MC de-asserts dfi_rddata_en once the expected data has returned.

## 17.3	Contiguous vs non-contiguous reads

Like writes, reads may be contiguous or separated by bubbles. The dfi_rddata_en signal can stay asserted across multiple read commands if no gap is expected; dfi_rddata_valid follows with the actual data. The PHY may return data earlier than the maximum tphy_rdlat, so the actual valid pattern may vary even when the enable is contiguous.

## 17.4	Per-slice independence

dfi_rddata_valid is one bit per PHY data slice. Different slices may return data on different cycles, and tphy_rdlat is the maximum across all slices. This is important for wide data buses where read capture timing varies by slice.

## 17.5	Read chip select timing

When dfi_rddata_cs_n is used, the MC drives it tphy_rdcslat cycles after the read command and holds it for the transfer length plus tphy_rdcsgap. If the next read targets a different chip select, the MC adds tphy_rdcsgap cycles.

## 17.6	LPDDR2/LPDDR3 mode-register reads

For LPDDR3 and LPDDR2, mode-register reads (MRR) use the same DFI read interface. The MC issues the MRR as a read command, asserts dfi_rddata_en for the expected return length, and the PHY returns the mode-register value on dfi_rddata. The dfi_rddata_dnv signal marks any invalid bytes in the returned word.

Source: DFI Specification v3.1 sections 3.3, 4.5, Table 9, Table 10

# 18	Walkthroughs

These walkthroughs use a deliberately tiny teaching model so that every DFI wire can be traced by hand.

## 18.1	Teaching model

- One rank, one chip select (dfi_cs_n is a single bit).

- 1:2 frequency ratio: two PHY phases (p0, p1) per MC clock.

- DFI data width 32 bits, split into two 16-bit PHY data slices.

- Burst length 8 on DRAM, so each read or write moves four DFI data words.

- Fixed timing: tcmd_lat = 0, tctrl_delay = 1, tphy_wrlat = 2, tphy_wrdata = 1, trddata_en = 2, tphy_rdlat = 4.

## 18.2	Walkthrough 1: activate -> read -> data return

The controller wants to read from bank 0, row 5, column 3.

Cycle 0: - Control interface: dfi_cs_n = 0, dfi_ras_n = 0, dfi_cas_n = 1, dfi_we_n = 1, dfi_bank = 0, dfi_address = row 5. This is an ACT. - dfi_cke = 1, dfi_odt driven to its programmed value.

Cycle 1: - Control interface: dfi_cs_n = 1 (NOP / deselect).

Cycle 4: - After tRCD DRAM timing has been satisfied, the MC issues a read. - Control interface: dfi_cs_n = 0, dfi_ras_n = 1, dfi_cas_n = 0, dfi_we_n = 1, dfi_bank = 0, dfi_address = column 3.

Cycle 6: - trddata_en = 2 cycles after the read, the MC asserts dfi_rddata_en for four cycles to request four DFI data words.

Cycle 10: - tphy_rdlat = 4 cycles after dfi_rddata_en, the PHY asserts dfi_rddata_valid and drives the first dfi_rddata word.

Cycles 10-13: - The PHY returns four consecutive data words on dfi_rddata, with dfi_rddata_valid asserted each cycle.

## 18.3	Walkthrough 2: activate -> write with preamble and mask

The controller wants to write 0xA5A5A5A5 to bank 1, row 2, column 7, masking the lowest byte.

Cycle 0: - Control interface: dfi_cs_n = 0, dfi_ras_n = 0, dfi_cas_n = 1, dfi_we_n = 1, dfi_bank = 1, dfi_address = row 2. ACT.

Cycle 4: - After row-open timing, the MC issues a write. - Control interface: dfi_cs_n = 0, dfi_ras_n = 1, dfi_cas_n = 0, dfi_we_n = 0, dfi_bank = 1, dfi_address = column 7.

Cycle 6: - tphy_wrlat = 2 cycles after the write command, the MC asserts dfi_wrdata_en for four cycles.

Cycle 7: - tphy_wrdata = 1 cycle after the enable, the MC drives the first dfi_wrdata word: 0xA5A5A5A5. - dfi_wrdata_mask is driven as 0x01 to mask the lowest byte. Bit 0 masks the byte in dfi_wrdata[7:0].

Cycles 7-10: - The MC continues driving the same data word and mask for all four cycles of the burst.

Cycle 11: - dfi_wrdata_en has de-asserted one cycle before the last data word. - The write data completes on the DRAM bus after twrdata_delay cycles.

## 18.4	Walkthrough 3: back-to-back reads on different rows

Bank 0 row 5 and bank 1 row 2 are both opened by earlier activates. The controller issues two reads, one to each bank.

Cycle 0: - Read from bank 0, column 3: dfi_cs_n = 0, dfi_cas_n = 0, dfi_we_n = 1, dfi_bank = 0, dfi_address = column 3.

Cycle 2: - dfi_rddata_en asserts for four cycles (bank 0 read data expected).

Cycle 4: - Second read from bank 1, column 7: dfi_cs_n = 0, dfi_cas_n = 0, dfi_we_n = 1, dfi_bank = 1, dfi_address = column 7. - Because the two reads are contiguous, dfi_rddata_en stays asserted for four more cycles.

Cycle 6: - dfi_rddata_valid asserts for the first read data (earliest allowed by tphy_rdlat = 4 from cycle 2).

Cycle 10: - dfi_rddata_valid asserts for the second read data (four cycles after the second dfi_rddata_en assertion at cycle 6).

The data stream on dfi_rddata is contiguous even though the two reads went to different banks, because the controller kept dfi_rddata_en asserted across both commands.

Source: DFI Specification v3.1 sections 4.4, 4.5

# 19	Parameter Families

DFI timing parameters are named dfi_t* in the specification text, though the signal names omit the dfi_ prefix for brevity. Each family constrains a different part of the MC/PHY contract.

## 19.1	PHY-defined vs MC/system-defined parameters

Timing parameter ownership

| Defined by | Examples | What the value tells you |
| --- | --- | --- |
| PHY | tctrl_delay, tphy_wrlat, tphy_wrdata, tphy_rdlat, tphy_rdcslat, tphy_wrcslat, tphy_*_gap | How much time the PHY needs internally to retime signals. |
| MC/system | tcmd_lat, trddata_en, tctrlupd_min, tctrlupd_max, tctrlupd_interval, tlp_resp, tlp_wakeup | Deadlines or guarantees the MC provides to the PHY. |
| Both/system | twrdata_delay, terror_resp, tinit_start, tinit_complete, parity/CRC latencies | Coordinated timing across both sides. |

## 19.2	Parameter families

Timing parameter families

| Family | Purpose |
| --- | --- |
| Command timing | tcmd_lat, tctrl_delay relate the command bus to chip select and to the DRAM pins. |
| Write data timing | tphy_wrlat, tphy_wrdata, twrdata_delay schedule write data relative to commands. |
| Read data timing | trddata_en, tphy_rdlat, tphy_rdcsgap, tphy_rdcslat schedule read data return. |
| Update timing | tctrlupd_*, tphyupd_type*, tphyupd_resp govern update handshakes. |
| Status/init timing | tinit_start, tinit_complete, tdram_clk_disable, tdram_clk_enable, parity/alert latencies. |
| Training timing | trdlvl_*, twrlvl_*, tcalvl_*, tphylvl* pace training sequences. |
| Low power timing | tlp_resp, tlp_wakeup bound the low-power handshake. |
| Error timing | terror_resp bounds error reporting delay. |

## 19.3	Minimum and maximum conventions

Most DFI timing parameters specify a minimum supportable value (often 0 or 1) and leave the maximum implementation-specific. The MC and PHY negotiate the actual values at integration time. The only hard rule is that the MC must meet the minimum assertion times and response deadlines, and the PHY must not exceed the maximum latencies it advertises.

## 19.4	Frequency-ratio timing

For frequency-ratio systems, timing parameters are defined in terms of DFI PHY clock cycles. In a matched-frequency system the DFI PHY clock and the DFI clock are the same, so the distinction disappears.

Source: DFI Specification v3.1 sections 3.1-3.8, 5.0

# 20	Ratio and Phasing

DFI 3.1 supports three frequency relationships between the MC clock and the PHY clock: 1:1, 1:2 and 1:4. The ratio affects command scheduling, signal naming and read data ordering.

## 20.1	Matched frequency (1:1)

In a 1:1 system the MC clock and the PHY clock are the same. There is one command opportunity per DFI clock cycle and one read/write data word per cycle. No _pN or _wN suffixes are needed, though phase 0 suffixes are optional.

## 20.2	1:2 frequency ratio

The PHY clock runs at twice the MC clock frequency. In each MC clock cycle the PHY can execute two commands or transfer two read data words. The MC therefore provides:

- dfi_address_p0, dfi_address_p1

- dfi_cs_n_p0, dfi_cs_n_p1

- dfi_wrdata_en_p0, dfi_wrdata_en_p1

- dfi_rddata_en_p0, dfi_rddata_en_p1

- dfi_rddata_w0, dfi_rddata_w1 on the return path

- dfi_rddata_valid_w0, dfi_rddata_valid_w1

The PHY must be able to accept a command on any phase.

## 20.3	1:4 frequency ratio

The PHY clock runs at four times the MC clock frequency. The same replication pattern applies with _p0 through _p3 for commands and _w0 through _w3 for read data. The 1:4 mode is explicitly defined in DFI 3.1 even though the specification text often shows only _p0/_p1 examples.

## 20.4	Read data rotation

In a frequency-ratio system, read data words return on dfi_rddata_wN in a rolling order. The MC and PHY must keep their data word pointers synchronized. A frequency change resets both pointers to zero.

## 20.5	Scheduling impact

Frequency ratio gives the MC two useful abilities:

- Multiple command slots per MC clock. A 1:4 system can issue up to four commands in one MC clock cycle.

- Higher data bandwidth at the PHY without increasing MC clock frequency.

The cost is wider buses, more complex pointer management and the need to drive enables in every phase for signals such as dfi_cke and dfi_odt.

## 20.6	Practical note for scoria

The scoria controller inherits a datapath designed around a four-phase unit of transfer (DFI_RATE = 4). The v2.1.1 specification enumerated the phase variants explicitly; v3.1 generalizes the notation to _pN. A naive signal-name diff would wrongly report the phase signals as removed. They are still present; only the naming convention changed.

Source: DFI Specification v3.1 sections 3.5.3, 4.8

# 21	Parameter Tables

This table lists every timing parameter named in the DFI 3.1 inventory, with a plain-English meaning and the interface it belongs to.

Complete DFI 3.1 timing parameter table

| Parameter | Interface | Meaning |
| --- | --- | --- |
| tcmd_lat | Control | Cycles from dfi_cs_n assertion to associated CA signals driven. |
| tctrl_delay | Control | Cycles from control signal change to that change reaching the DRAM interface. |
| tphy_wrlat | Write data | Cycles from write command to dfi_wrdata_en assertion. |
| tphy_wrdata | Write data | Cycles from dfi_wrdata_en assertion to dfi_wrdata driven. |
| twrdata_delay | Write data / Idle | Cycles from dfi_wrdata_en assertion to completion of write data transfer on DRAM bus. |
| tphy_wrcslat | Write data | Cycles from write command to dfi_wrdata_cs_n assertion. |
| tphy_wrcsgap | Write data | Additional cycles between commands when changing target write chip select. |
| tphy_crcmin_lat | Write data / CRC | Min cycles from CRC data on dfi_wrdata_en to CRC error on dfi_alert_n. |
| tphy_crcmax_lat | Write data / CRC | Max cycles from CRC data on dfi_wrdata_en to CRC error on dfi_alert_n. |
| trddata_en | Read data | Cycles from read command to dfi_rddata_en assertion. |
| tphy_rdlat | Read data | Max cycles from dfi_rddata_en assertion to dfi_rddata_valid assertion. |
| tphy_rdcslat | Read data | Cycles from read command to dfi_rddata_cs_n assertion. |
| tphy_rdcsgap | Read data | Additional cycles between commands when changing target read chip select. |
| tctrlupd_min | Update | Minimum cycles dfi_ctrlupd_req must be asserted. |
| tctrlupd_max | Update | Maximum cycles dfi_ctrlupd_req may be asserted. |
| tctrlupd_interval | Update | Maximum cycles between dfi_ctrlupd_req assertions. |
| tphyupd_type0 | Update | Max cycles for PHY-initiated update type 0. |
| tphyupd_type1 | Update | Max cycles for PHY-initiated update type 1. |
| tphyupd_type2 | Update | Max cycles for PHY-initiated update type 2. |
| tphyupd_type3 | Update | Max cycles for PHY-initiated update type 3. |
| tphyupd_resp | Update | Max cycles from dfi_phyupd_req to dfi_phyupd_ack. |
| tinit_start | Status / Init | Max cycles for PHY to acknowledge frequency-change request. |
| tinit_complete | Status / Init | Max cycles for PHY to re-assert dfi_init_complete after dfi_init_start de-asserts. |
| tdram_clk_disable | Status / Clock | Cycles from dfi_dram_clk_disable assertion to DRAM clocks held constant. |
| tdram_clk_enable | Status / Clock | Cycles from dfi_dram_clk_disable de-assertion to first valid DRAM clock edge. |
| tparin_lat | Status / Parity | Cycles from command to associated dfi_parity_in. |
| tphy_paritylat | Status / Parity | Max cycles from dfi_parity_in to dfi_alert_n assertion. |
| trdlvl_en | Training / Read | Min cycles from read training enable to first read/MRR command. |
| trdlvl_resp | Training / Read | Max cycles from dfi_rdlvl_req/dfi_rdlvl_gate_req to enable assertion. |
| trdlvl_max | Training / Read | Max cycles the MC waits for dfi_rdlvl_resp. |
| trdlvl_rr | Training / Read | Min cycles between read training reads/MRRs. |
| twrlvl_en | Training / Write | Min cycles from write leveling enable to first dfi_wrlvl_strobe. |
| twrlvl_resp | Training / Write | Max cycles from dfi_wrlvl_req to dfi_wrlvl_en. |
| twrlvl_max | Training / Write | Max cycles the MC waits for dfi_wrlvl_resp. |
| twrlvl_ww | Training / Write | Min cycles between dfi_wrlvl_strobe assertions. |
| tcalvl_en | Training / CA | Min cycles from dfi_calvl_en to dfi_cke de-assertion. |
| tcalvl_capture | Training / CA | Cycles from calibration command to dfi_calvl_capture pulse. |
| tcalvl_resp | Training / CA | Max cycles from dfi_calvl_req to dfi_calvl_en. |
| tcalvl_max | Training / CA | Max cycles the MC waits for dfi_calvl_resp. |
| tcalvl_cc | Training / CA | Min cycles between calibration commands. |
| tphylvl | Training / PHY request | Max cycles dfi_phylvl_req_cs_n stays asserted after acknowledge. |
| tphylvl_resp | Training / PHY request | Max cycles from PHY training request to MC acknowledge. |
| tlp_resp | Low power | Max cycles from low-power request to dfi_lp_ack. |
| tlp_wakeup | Low power | Max cycles from request de-assertion to dfi_lp_ack de-assertion. |
| terror_resp | Error | Max cycles from affected transaction to dfi_error assertion. |

## 21.1	Programmable parameters

DFI 3.1 programmable parameters

| Parameter | Meaning |
| --- | --- |
| phycrc_mode | 0 = MC generates CRC; 1 = PHY generates CRC. |
| phydbi_mode | 0 = MC handles DBI; 1 = PHY handles DBI. |
| phyrdlvl_en | PHY supports read data eye training. |
| phyrdlvl_gate_en | PHY supports gate training. |
| phywrlvl_en | PHY supports write leveling. |
| phycalvl_en | PHY supports CA training. |

Source: DFI Specification v3.1 sections 3.1-3.8, Tables 5-22

# 22	Low Power Behavior

DFI 3.1’s low-power interface is an opportunity request: the MC suggests that the PHY may save power, but the PHY decides whether to take the opportunity. The handshake does not directly enter DRAM self-refresh or power-down; those are DRAM commands issued through the control interface.

## 22.1	Low-power request split

v3.1 splits the v2.1 dfi_lp_req into:

- dfi_lp_ctrl_req: no more commands on the control interface.

- dfi_lp_data_req: no more activity on the data interface.

This allows the PHY to sleep the command path and data path separately. For example, a state that still refreshes memory can keep the control path awake while the data path sleeps.

## 22.2	Typical sequence

- The MC issues the DRAM command to enter the desired low-power state (power-down enter or self-refresh enter) on the control interface.

- The MC asserts dfi_lp_ctrl_req and/or dfi_lp_data_req with an appropriate dfi_lp_wakeup value.

- The PHY may acknowledge with dfi_lp_ack within tlp_resp cycles.

- The low-power state persists while request and acknowledge are both asserted.

- When the MC needs to resume, it de-asserts the request.

- The PHY de-asserts dfi_lp_ack within tlp_wakeup cycles and normal operation resumes.

## 22.3	Relationship to DRAM self-refresh

Self-refresh is a DRAM command, not a DFI state. The DFI low-power interface merely tells the PHY that the controller will not be sending commands or data for some time. The PHY may use that information to gate its own clocks or reduce power, but the DRAM refresh timing is handled by the DRAM’s self-refresh logic once the self-refresh command is issued.

## 22.4	Scoria note

The scoria controller exposes dfi_lp_ctrl_req and dfi_lp_data_req for spec compliance, but the target PHY family does not consume them. CKE is driven as a CSR bit set during initialization, and the generated core does not implement a self-refresh or power-down engine.

Source: DFI Specification v3.1 sections 3.7, 4.12

# 23	Update Behavior

Updates are short pauses in normal traffic that let the PHY perform calibration or timing adjustments. DFI 3.1 supports both MC-initiated and PHY-initiated updates.

## 23.1	MC-initiated update

The MC asserts dfi_ctrlupd_req whenever it knows the DFI bus will be idle. The PHY may acknowledge with dfi_ctrlupd_ack or ignore the request. The request must last at least tctrlupd_min and no more than tctrlupd_max cycles, and the MC must issue a new request at least every tctrlupd_interval cycles.

While the request (and any acknowledge) is asserted, the DFI bus remains idle except for update-related activity.

## 23.2	PHY-initiated update

The PHY asserts dfi_phyupd_req with a valid dfi_phyupd_type when it needs idle time. The MC must respond with dfi_phyupd_ack within tphyupd_resp cycles. The request stays asserted until the update is done and must de-assert before the selected tphyupd_typeX expires.

The MC must not assert dfi_ctrlupd_req and dfi_phyupd_ack simultaneously. If both request signals are asserted at once, the PHY may withdraw its own request.

## 23.3	Idle-bus definition

The DFI bus is idle when:

- The control interface is not sending commands.

- All read data has been transferred to the MC.

- All write data has been transferred on the DFI bus and completed on the DRAM bus.

- No pages are open.

The twrdata_delay parameter defines how long after dfi_wrdata_en the write data is guaranteed to have completed on the DRAM bus. This prevents an update from cutting off a write burst.

## 23.4	Update after self-refresh exit

After exiting self-refresh, the MC must assert dfi_ctrlupd_req before resuming normal read or write traffic. This gives the PHY a chance to re-calibrate after the low-power state.

Source: DFI Specification v3.1 sections 3.4, 4.6

# 24	Frequency Change

DFI 3.1 reuses the initialization handshake for frequency changes during normal operation. The MC requests a change with dfi_init_start; the PHY acknowledges by de-asserting dfi_init_complete.

## 24.1	Acknowledged frequency change

- The MC asserts dfi_init_start during normal operation.

- The PHY de-asserts dfi_init_complete within tinit_start cycles to accept the request.

- The MC holds dfi_init_start asserted while the clock frequency changes.

- The MC de-asserts dfi_init_start when the new frequency is stable.

- The PHY re-initializes for the new frequency and re-asserts dfi_init_complete within tinit_complete cycles.

Both sides reset their DFI read data word pointers to zero when dfi_init_start asserts.

## 24.2	Not-acknowledged frequency change

If the PHY does not de-assert dfi_init_complete within tinit_start cycles, the request is ignored. The MC must then de-assert dfi_init_start and may try again later.

## 24.3	Stable levels during the change

While dfi_init_start is asserted or dfi_init_complete is de-asserted, both the MC and the PHY must hold the DFI and memory interface signals at valid and stable levels. The DFI specification does not define a maximum overall duration for the frequency change.

## 24.4	Scoria note

The scoria controller implements the frequency-change protocol because it is part of the v3.1 initialization interface, but the DDR3/LPDDR3 target use case does not rely on dynamic frequency scaling.

Source: DFI Specification v3.1 sections 3.5.4, 4.9

# 25	Refresh Interaction

Refresh is a DRAM command issued on the DFI control interface. DFI 3.1 does not define refresh timing itself, but it does constrain when refresh can be scheduled relative to training, updates and low-power states.

## 25.1	Refresh and training

The MC must complete all in-flight transactions before starting training. Refresh therefore competes with training for idle-bus windows. DFI 3.1 adds support for refresh during training, meaning the MC is not required to disable refresh entirely while a training operation is in progress, but the details depend on the PHY and DRAM capabilities.

## 25.2	Refresh and update

An update requires the bus to be idle. A refresh command is a command, so it cannot overlap an update window. The MC must either:

- Finish the refresh before asserting dfi_ctrlupd_req or acknowledging dfi_phyupd_req, or

- Defer the refresh until the update handshake completes.

## 25.3	Refresh and low power

In self-refresh, the DRAM handles its own refresh. The DFI low-power interface only informs the PHY that the controller will be idle. When the MC exits self-refresh, it must issue dfi_ctrlupd_req before resuming normal traffic.

In power-down, refresh is suspended. The MC must ensure that the power- down duration does not violate the DRAM refresh requirements.

## 25.4	Scoria note

The scoria controller does not implement a self-refresh or power-down engine in the generated core. CKE is controlled by software through a CSR. Refresh commands are issued by the controller’s normal scheduler on the DFI control interface.

Source: DFI Specification v3.1 sections 4.6, 4.11.6, 4.12

# 26	Revision Deltas

DFI 3.1 is best understood as the v3.0 DDR4 additions plus the LPDDR3 and interface-split changes of 3.1. The scoria controller inherits the v2.1.1 baseline from pumice and upgrades to 3.1 for the leveling and initialization model.

## 26.1	What carried over unchanged from v2.1.1

- Frequency ratios and the _pN replication convention. v2.1.1 enumerated phase variants explicitly; v3.1 generalizes them to _pN. The 1:4 ratio is still defined.

- The update interface (dfi_ctrlupd_* and dfi_phyupd_*).

- The basic command/address/data interface, apart from the DDR4 additions.

- dfi_init_start and dfi_init_complete, now reused for frequency change.

- dfi_data_byte_disable and dfi_dram_clk_disable.

- Read and write leveling as concepts, though the signal names and per-CS scheme were reworked.

## 26.2	v2.1.1 -> v3.0 changes

v3.0 added the features that enable DDR4:

v3.0 additions over v2.1.1

| Area | v3.0 additions |
| --- | --- |
| Command encoding | dfi_act_n, dfi_bg, dfi_cid; dfi_ras_n/dfi_cas_n/dfi_we_n carry A16/A15/A14 when dfi_act_n is asserted. |
| Parity/CRC/alert | dfi_parity_in, dfi_alert_n_aN, CRC timing parameters. |
| DBI | dfi_rddata_dbi_n, phydbi_mode. |
| Data path chip select | dfi_wrdata_cs_n, dfi_rddata_cs_n. |
| Error interface | dfi_error, dfi_error_info, terror_resp. |
| Programmable parameters | phycrc_mode, phydbi_mode, per-training enables. |

## 26.3	v3.0 -> v3.1 changes

v3.1 focused on LPDDR3 and interface refinement:

v3.1 additions over v3.0

| Area | v3.1 additions |
| --- | --- |
| LPDDR3 CA training | dfi_calvl_*, dfi_phy_calvl_cs_n, CA training parameters. |
| LPDDR3 command support | LPDDR3 added to gate training, read data eye training and write leveling. |
| Low power split | dfi_lp_req becomes dfi_lp_ctrl_req and dfi_lp_data_req. |
| PHY-requested training | dfi_phylvl_req_cs_n, dfi_phylvl_ack_cs_n for non-DFI training mode. |
| LPDDR3 CA mapping | 20-bit dfi_address maps to LPDDR3 CA bus. |

## 26.4	Brief v4.0 preview

DFI 4.0 later extended the spec with a much larger training interface, including DBI training, PHY-master training, and additional status and frequency-change capabilities. Those features are beyond the scope of a DDR3/LPDDR3 controller and are not used by scoria.

## 26.5	What scoria leaves out

The scoria DDR3/LPDDR3 controller does not implement:

- DDR4 command encoding (dfi_act_n, dfi_bg, dfi_cid).

- CA parity, CRC and dfi_alert_n.

- DBI.

- LPDDR3 CA training (dfi_calvl_*).

These are either DDR4-era or LPDDR4-era features that do not apply to the target memory types.

Source: DFI Specification v3.1 Release Information and revision notes; projects/components/mem-ctrl-ip/research-ip/scoria-ddr3-lpddr3/docs/scoria_has/ch04_interfaces/01_dfi_v31.md

# 27	DFI 3.1 in the Scoria Controller

This chapter connects the DFI 3.1 specification to the scoria DDR3/LPDDR3 memory controller as documented in projects/components/mem-ctrl-ip/research-ip/scoria-ddr3-lpddr3/docs/scoria_has/ch04_interfaces/01_dfi_v31.md.

## 27.1	Why scoria chose v3.1

Scoria targets DDR3 and LPDDR3. Its predecessor pumice used DFI v2.1.1. The upgrade to v3.1 was driven by DDR3 write leveling: v3.1’s per-chip- select leveling scheme is what future DDR4 work will also need, so paying for it once in scoria is cheaper than re-implementing it later.

## 27.2	What scoria inherits

The datapath is inherited almost unchanged from pumice:

- One async-FIFO clock-domain crossing, bubble-free.

- The unit of transfer is a DFI word.

- DFI_RATE = 4.

A naive signal-name diff between v2.1.1 and v3.1 might claim that phase variants such as dfi_address_p2 were removed. They were not. v2.1.1 listed them explicitly; v3.1 generalized the notation to _pN. The 1:4 frequency ratio remains a defined mode.

## 27.3	What scoria implements from v3.1

Scoria DFI 3.1 implementation highlights

| Feature | Scoria implementation |
| --- | --- |
| Write leveling | scoria_wrlvl_ifc drives dfi_wrlvl_*; firmware-driven search, not a hard-coded state machine. |
| Data path chip select | dfi_wrdata_cs_n and dfi_rddata_cs_n are driven; at NUM_RANKS = 1 the value is constant rank 0. |
| dfi_reset_n | Driven from the init sequencer and presented as dfi_reset_n_o at the DFI boundary. |
| Low power requests | Exposed for spec compliance, but not consumed by the target PHY. |
| Error interface | dfi_error is used as a generic flag; DDR4-specific dfi_error_info encodings are ignored. |

## 27.4	What scoria does not implement

DFI 3.1 features deliberately unimplemented in scoria

| Group | Signals | Reason |
| --- | --- | --- |
| DDR4 command encoding | dfi_act_n*, dfi_bg*, dfi_cid* | DDR3 has no dedicated ACT pin, bank groups or chip ID. |
| CA parity, CRC, alert | dfi_alert_n*, dfi_parity_in_p* | DDR4-era features. |
| Data bus inversion | dfi_rddata_dbi* | DDR4/LPDDR4 feature. |
| CA training | dfi_calvl_*, dfi_ca_capture, dfi_phy_calvl_cs_n | LPDDR3 CA training is treated as LPDDR4-era for this controller. |

## 27.5	Known documentation corrections

The scoria tie-in document notes a few spec-related traps:

- The v2.1.1 -> v3.1 signal diff must not be trusted blindly; phase signals are renamed, not removed.

- dfi_reset_n is gated on memory type, not DFI version, so any scoria DFI claim implies this signal.

- dfi_wrdata_cs_n / dfi_rddata_cs_n are genuinely v3.1-only and are driven even in single-rank mode.

## 27.6	Study corrections carried into this book

The caveats in the DFI 3.1 inventory are worth remembering:

- Section 3.6 refers to a dfi_ca_capture pulse; the actual signal is dfi_calvl_capture.

- Section 4.11 contains typo variants such as dfi_phlvl_ack_cs_n; the defined names are dfi_phylvl_req_cs_n and dfi_phylvl_ack_cs_n.

- dfi_rdlvl_edge and timer-based signals such as dfi_rdlvl_delay_x appear only in revision-history text, not in the v3.1 signal tables.

Source: projects/components/mem-ctrl-ip/research-ip/scoria-ddr3-lpddr3/docs/scoria_has/ch04_interfaces/01_dfi_v31.md; DFI Specification v3.1 sections 3.6, 4.11