Memory Notes

Simplified LPDDR3 Simplified edition - JEDEC JESD209-3C

October 3, 2026

Table of Contents

1 Simplified Lpddr3 Index	11

2 Introduction	12

2.1 What LPDDR3 SDRAM is	12

2.2 Family positioning	13

2.3 Key features of LPDDR3	14

2.4 Supplies and voltage ranges	15

2.5 Speed grades	16

2.6 The simplified drill model	17

3 Organization	18

3.1 Banks, rows, columns	18

3.2 Prefetch and IO width	19

3.3 Burst structure	20

3.4 Channels	21

3.5 The simplified drill model	22

4 Acronyms and Conventions	23

4.1 Signal names	24

4.2 Commands	25

4.3 Architecture terms	26

4.4 Power and refresh terms	27

4.5 Conventions	28

5 Power-Up and Initialization	29

5.1 Cold-start sequence	30

5.1.1 Figure 2.1: LPDDR3 Power-Up Initialization Timeline	31

5.2 Reset without power ramp	33

5.3 Power-off sequence	34

5.4 Notes that bite	35

6 Mode Registers	36

6.1 Register map (SDRAM view)	37

6.2 Programming rules	39

6.3 MR0 - Device Information	40

6.4 MR1 - Device Feature 1	41

6.5 MR2 - RL and WL	42

6.6 MR3 - Drive Strength	43

6.7 MR4 - Refresh Rate and Temperature	44

6.8 MR10 - ZQ Calibration Codes	45

6.9 MR11 - ODT Control	46

6.10 MR16 / MR17 - PASR Masks	47

6.11 MR32 / MR40 - DQ Calibration Patterns	48

6.12 MR41 / MR42 / MR48 - CA Training	49

6.13 MR63 - Reset	50

7 Command Truth Tables	51

7.1 Pin conventions	52

7.2 Command truth table	53

7.3 CKE table	55

7.4 State table highlights	57

8 Commands: Rules and Constraints	58

8.1 ACT - Activate	59

8.2 RD - Burst read	60

8.3 WR - Burst write	61

8.4 PRE - Precharge	62

8.5 REFab / REFpb	63

8.6 Self-refresh entry and exit	64

8.7 MRR / MRW	65

8.8 Calibration and training	66

8.9 ODT	67

8.10 Power-down	68

8.11 Deep power-down	69

8.12 Clock stop and frequency change	70

8.13 NOP / Deselect	71

9 Walkthrough: AXI Read, Row Already Open	72

9.1 Scenario	73

9.2 Command trace	74

9.3 Why each command	75

9.4 AXI view	76

9.5 What to notice	77

10 Walkthrough: AXI Read, Wrong Row Open	78

10.1 Scenario	78

10.2 Command trace	79

10.3 Why each command	80

10.4 AXI view	81

10.5 What to notice	82

11 Walkthrough: AXI Write to an Idle Bank	83

11.1 Scenario	83

11.2 Command trace	84

11.3 Why each command	85

11.4 AXI view	86

11.5 What to notice	87

12 Walkthrough: Mixed Reads and Writes	88

12.1 Scenario	88

12.2 Command trace	89

12.3 Why each gap	90

12.4 AXI view	91

12.5 What to notice	92

13 Data Path	93

13.1 Prefetch and burst	93

13.2 Read and write strobes	94

13.3 Read and write latencies	95

13.4 DM - write data mask	97

13.5 What LPDDR3 does not have	98

13.6 Signaling	99

14 On-Die Termination	100

14.1 Enabling and selecting the termination value	101

14.2 Asynchronous ODT timing	102

14.3 ODT during reads	103

14.4 ODT during low-power states and training	104

14.5 ODT states truth table	105

14.6 What LPDDR3 does not have	106

15 Core Timing Parameters	107

15.1 Row-cycle parameters	108

15.2 Activate-rate parameters (cross-bank)	109

15.3 Data-to-precharge parameters	110

15.4 Why these numbers dominate	111

16 Turnaround and Burst Timing	112

16.1 Latencies and CAS-to-CAS	113

16.2 Direction changes	114

16.3 Strobe-level timing (for completeness)	115

16.4 Temperature derating	116

17 Refresh and Self-Refresh Timing	117

17.1 Refresh parameters	117

17.2 Self-refresh	119

17.3 Power-down and deep power-down	120

18 Package and Electrical Notes	121

18.1 Packages	122

18.2 Supplies and signaling	123

18.3 Speed grades	124

18.4 Derating and characterization	125

19 Test Features	126

19.1 MRR as the observation port	127

19.2 DQ calibration (MR32 / MR40)	128

19.3 ZQ calibration (MR10)	129

19.4 CA training (MR41 / MR48 / MR42)	130

19.5 Write leveling (MR2[7])	132

19.6 Temperature sensor (MR4)	133

19.7 What is not here	134

List of Figures

Figure 2.1: LPDDR3 Power-Up Initialization Timeline	31

List of Tables

No tables in this document.

List of Waveforms

No waveforms in this document.

# 1	Simplified Lpddr3 Index

Generated: 2026-10-20

# 2	Introduction

## 2.1	What LPDDR3 SDRAM is

LPDDR3 SDRAM is the third generation of JEDEC low-power double-data-rate memory, standardized as JESD209-3C. It targets mobile and battery-powered systems, typically in package-on-package (PoP) or discrete FBGA form. The interface runs at 1.2 V and the devices carry a set of power-reduction modes (power-down, self-refresh, deep power-down, per-bank refresh, clock stop) that are not found on desktop DDR3.

Its defining architectural choice is the 10-bit double-data-rate command/address (CA) bus. Instead of a wide SDR address bus with separate RAS_n, CAS_n and WE_n strobes, commands and row/column/bank addresses are packed onto two edges of a single clock cycle on CA0-CA9. CS_n and CKE are the only control inputs sampled on a single clock edge.

## 2.2	Family positioning

| Tech | Prefetch | IO voltage | DLL | Command/address style |
| --- | --- | --- | --- | --- |
| LPDDR2 (JESD209-2) | 4n (S4) / 2n (S2) | 1.2 V (HSUL_12) | No | 10-bit DDR CA bus |
| LPDDR3 (JESD209-3) | 8n | 1.2 V (HSUL_12) | No | 10-bit DDR CA bus |
| DDR3 (JESD79-3) | 8n | 1.5 V (SSTL_15) | Yes | Full parallel SDR CA + RAS/CAS/WE |

LPDDR3 is therefore best understood as an evolutionary step from LPDDR2: same CA-bus concept, wider prefetch, faster data rates, and a richer set of training/calibration features.

## 2.3	Key features of LPDDR3

- 8n prefetch: a column access moves eight words internally per DQ per access; the DDR interface delivers two words per DQ per clock, so the minimum burst occupies four clocks.

- Burst length fixed at BL8. MR1 burst-length encoding is 011B for BL8; all other encodings are reserved. There is no burst chop.

- 8 banks flat, addressed by BA0-BA2. There are no bank groups and no stack IDs.

- x16 and x32 data widths; packages support single- or dual-channel arrangements, but the protocol in this book is per-channel.

- No DLL. Read data timing is given as a broad tDQSCK window relative to the clock; tDQSCK may extend across more than one clock period.

- Read latency (RL) and write latency (WL) are programmed through mode registers (MR0/MR2), not via additive latency.

- No dedicated RESET# pin. Reset is performed with an MRW RESET command.

- MRR and MRW commands replace the single MRS command used by desktop DDR3.

- Training and calibration: CA training, write leveling, DQ calibration, and ZQ calibration are all supported.

- Asynchronous ODT: DQ-bus termination follows the ODT pin and the MR11 setting, and is turned off during reads, power-down, self-refresh, deep power-down and CA training.

- Deep power-down (DPD): lowest-power state; array contents are lost and exit requires full re-initialization.

- Per-bank refresh (REFpb) and all-bank refresh (REFab) both exist.

- Temperature sensor readable via MR4, used for refresh-rate scaling and AC timing de-rating decisions.

- Clock stop supported during CKE LOW and CKE HIGH under defined conditions.

## 2.4	Supplies and voltage ranges

| Supply | Nominal | Function |
| --- | --- | --- |
| VDD1 | 1.80 V | Core power |
| VDD2 | 1.20 V | Core power |
| VDDCA | 1.20 V | CA/CKE/CS_n/CK input-buffer power |
| VDDQ | 1.20 V | DQ/DQS/DM I/O power |
| VREF(CA) | VDDCA/2 | Reference for CA, CKE, CS_n, CK |
| VREF(DQ) | VDDQ/2 | Reference for DQ inputs |

## 2.5	Speed grades

Data-rate grades reach 1333, 1600, 1866 and 2133 MT/s at the high end (Table 64); lower data rates are also supported. Core timings (tRCD, tRP, tRAS) are specified in nanoseconds, while RL and WL are given in clock counts that depend on data rate (Table 63).

## 2.6	The simplified drill model

Real parts have thousands of rows and hundreds of columns. All examples in this book use a deliberately tiny geometry so bank state is obvious at a glance:

- Banks: B0-B7 (eight, as on 1 Gb and larger parts)

- Rows: R0-R7 per bank

- Columns: C0-C7 per row

This model is used by the ddr_drills training app. Every command trace, walkthrough and timing example in later chapters uses it. Real address maps are far larger; the scheduling rules are identical.

Source: JESD209-3C sections 1, 2.4, 3, Table 64

# 3	Organization

## 3.1	Banks, rows, columns

An LPDDR3 device is organized as eight independent banks of DRAM arrays. Each bank has its own sense-amplifier row buffer, so up to eight rows can be open at the same time (one per bank). The bank count is always eight; the spec does not define 4-bank LPDDR3 SDRAM devices.

Address geometry per density (from the spec addressing table):

| Density | Banks | Rows (x16) | Columns (x16) | tREFI (us, <=85 C) |
| --- | --- | --- | --- | --- |
| 1 Gb | 8 | R0-R12 | C0-C9 | 7.8 |
| 2 Gb | 8 | R0-R13 | C0-C9 | 3.9 |
| 4 Gb | 8 | R0-R13 | C0-C10 | 3.9 |
| 6 Gb | 8 | R0-R14* | C0-C10 | 3.9 |
| 8 Gb | 8 | R0-R14* | C0-C10 | 3.9 |
| 12 Gb | 8 | R0-R14* | C0-C11 | 3.9 |
| 16 Gb | 8 | R0-R14* | C0-C11 | 3.9 |
| 32 Gb | 8 | TBD | TBD | TBD |

*Addresses with R13 and R14 both high have no memory; an ACT to that space is ignored.

x32 parts lose one column address bit relative to x16; x16 parts expose one more column bit than x32 at the same density.

Addressing rules:

- The least-significant column bit C0 is implied zero and does not appear on the CA bus. Column addresses on the bus start at C1.

- REFpb can only target a bank that is currently idle and precharged.

## 3.2	Prefetch and IO width

- 8n prefetch: a column access fetches eight words per DQ internally. The DDR interface delivers two words per DQ per clock, so a minimum burst occupies four clocks. This is why tCCD = 4 clocks on LPDDR3.

- Data bus widths are x16 and x32. Each byte lane has its own differential strobe (DQS_t/DQS_c) and its own DM mask pin: one DQS pair and one DM per 8 DQs.

## 3.3	Burst structure

Accesses are burst-oriented: each Read or Write command transfers BL words per DQ, starting at the issued column and following the programmed burst order. BL is fixed at 8. Burst addressing follows the standard sequential pattern driven by the low-order column bits.

## 3.4	Channels

Multi-channel parts exist at the package level: a PoP or discrete package can carry two fully independent LPDDR3 channels, each with its own CA bus, clock, CKE and CS_n. Channels share nothing but the package; the protocol in this book is per-channel.

## 3.5	The simplified drill model

Per the Introduction, all examples use eight banks (B0-B7), eight rows (R0-R7) and eight columns (C0-C7). Real parts are far larger; the small model exists to make bank/row state obvious in command traces.

Source: JESD209-3C sections 2.1-2.3, 3.1

# 4	Acronyms and Conventions

Terms used precisely in this book.

## 4.1	Signal names

| Term | Meaning |
| --- | --- |
| CK_t / CK_c | Differential clock. Positive edge = CK_t rising through CK_c falling |
| CKE | Clock enable. SDR, sampled on the positive clock edge; gates all power states |
| CS_n | Chip select. SDR, sampled on the positive clock edge; contributes to the command code |
| CA0-CA9 | 10-bit DDR command/address bus; sampled on both clock edges |
| CA / CATR | Command/address bus / CA training mode entered via MR41/MR42/MR48 |
| CAxr / CAxf | CA bit x on the rising (r) / falling (f) edge of the clock |
| DQ | Bidirectional data |
| DQS_t / DQS_c | Differential data strobe, one pair per byte lane; edge-aligned on reads, center-aligned on writes |
| DM | Data mask, one per byte lane; masks write data when sampled high |
| ODT | Asynchronous on-die termination control pin for the DQ bus |
| ZQ | Pin used as reference for output drive-strength calibration |

## 4.2	Commands

| Term | Meaning |
| --- | --- |
| ACT | Activate (open a row in a bank) |
| RD / WR | Burst read / burst write |
| PRE | Precharge (close a row); per-bank or all-bank via the AB flag |
| REFab / REFpb | Refresh, all banks / one per-bank-refresh bank |
| MRR / MRW | Mode register read / mode register write |
| NOP | No operation |
| SREF / PD / DPD | Self-refresh, power-down, deep power-down |

## 4.3	Architecture terms

| Term | Meaning |
| --- | --- |
| Prefetch (8n) | Words moved internally per column access; LPDDR3 = 8n |
| BL | Burst length; fixed at 8 for LPDDR3 |
| RL / WL | Read latency / write latency, in clocks, programmed in MR0/MR2 |
| nWR | Write recovery in clocks for auto-precharge, programmed in MR1 |
| AP | Auto-precharge flag (CA0f of a RD/WR command) |
| AB | All-bank flag (CA4r of a PRE command) |
| RU{} | Round-up function; RU{x} = smallest integer >= x |
| CA training | Training mode to center CA inputs; entered/exited via MR41/MR42 |
| Write leveling | Training mode to align DQS with the clock; entered/exited via MRW |
| DQ calibration | MRR reads from MR32/MR40 return predefined calibration patterns |

## 4.4	Power and refresh terms

| Term | Meaning |
| --- | --- |
| PASR | Partial array self-refresh: refresh only masked banks/segments in self-refresh |
| RM | Refresh multiplier from MR4; tREFIM = RM x tREFI |
| DAI | Device auto-initialization status bit (MR0 OP0) |
| TUF | Temperature update flag (MR4 OP7) |
| DPD | Deep power-down; array contents lost, full re-init required on exit |
| HSUL_12 | High-speed unterminated logic, 1.2 V: the LPDDR3 IO standard |
| tREFW / tREFI / tREFIM | Refresh window / average refresh interval / multiplied refresh interval |
| tRFCab / tRFCpb | Refresh cycle time, all-bank / per-bank |
| RZQ | 240 ohm reference resistor for output driver and ODT calibration |

## 4.5	Conventions

- H/L in truth tables are logic high/low at the sampled edge; X is “don’t care” but must still be a valid logic level.

- Timing symbols are written as plain text: tRCD, tWTR, tREFIpb.

- Anything the spec leaves unspecified is illegal; after an illegal event the part has to be reset through a full power-down/re-init cycle.

Source: JESD209-3C sections 2.4 (Table 2), 3.4.1, 4.8, 4.9.1, 4.12

# 5	Power-Up and Initialization

LPDDR3 must be brought up with a fixed sequence; any other path is undefined. Reset is command-based only - the spec has no dedicated SDRAM RESET# pin (the RST_n ball in PoP ballouts is an eMMC signal), so the reset event is an MRW to MR63.

## 5.1	Cold-start sequence

- Power ramp (Ta -> Tb). Apply supplies while CKE is held LOW (<= 0.2 x VDDCA). Ta is when any supply first reaches 300 mV; Tb is when every supply and VREF is inside its operating range. The ramp (tINIT0) must finish within 20 ms. During the ramp: VDD1 must stay within 200 mV above VDD2; VDD1 and VDD2 must stay within 200 mV above VDDCA and VDDQ; VREF must be below every supply; and VSS/VSSQ/VSSCA must be within 100 mV of each other. DQ, DM, DQS_t and DQS_c must stay between VSSQ and VDDQ; CK_t, CK_c, CS_n and CA must stay between VSSCA and VDDCA, to avoid latch-up. Device outputs are High-Z while CKE is LOW.

- Clock and CKE (Tb -> Tc). Keep CKE LOW for at least tINIT1 = 100 ns after Tb. The clock must be stable for at least tINIT2 = 5 tCK before CKE’s first LOW-to-HIGH transition. If any MRR will be issued during init, the clock period must be inside the boot window tCKb = 18-100 ns. CKE, CS_n and CA must meet setup/hold from the first rising edge onward. The ODT input may be undefined until tIS before CKE is registered HIGH, then must be held static LOW or HIGH until initialization is complete (including tZQINIT).

- NOP wait (Tc -> Td). With CKE HIGH, issue NOP for at least tINIT3 = 200 us.

- MRW RESET (Td). Issue an MRW RESET (MR63). An optional PRECHARGE ALL may precede it. Keep CKE HIGH with NOPs for at least tINIT4 = 1 us; only NOPs are allowed during this time.

- DAI polling (Te -> Tf). Only MRR and power-down entry/exit are legal now. Poll MR0’s DAI bit, or simply wait tINIT5(max) = 10 us. The device is idle once DAI indicates completion or when tINIT5(max) expires. This is also the normal point to read MR8 (and MR5-MR7) to discover I/O width, density, manufacturer and revision.

- ZQ calibration / CA training (Tf -> Tg). If CA training is required, start it at Tf using MR41 (or MR48 for the second mapping) and exit with MR42; no CA commands other than RESET or NOP may be issued until training finishes (Tf’). Then issue the MRW ZQ initialization calibration command (MR10, code 0xFF). If CA training is not needed, ZQ initialization may start at Tf directly. On a shared ZQ bus, do not overlap ZQ calibrations across devices. The device is ready for normal operation after tZQINIT = 1 us.

- Configure (after Tg). MRW the operating registers: MR1 (burst length, nWR), MR2 (RL/WL, write-leveling enable, WL set), MR3 (drive strength) and MR11 (DQ ODT). The device is now idle and accepts any valid command.

### 5.1.1	Figure 2.1: LPDDR3 Power-Up Initialization Timeline

LPDDR3 Power-Up Initialization Timeline

| Parameter | Min | Max | Unit | Meaning |
| --- | --- | --- | --- | --- |
| tINIT0 | - | 20 | ms | longest allowed supply ramp |
| tINIT1 | 100 | - | ns | CKE must stay LOW after supplies are valid |
| tINIT2 | 5 | - | tCK | stable clock before CKE rises |
| tINIT3 | 200 | - | us | NOP wait after CKE goes HIGH |
| tINIT4 | 1 | - | us | NOP wait after MRW RESET |
| tINIT5 | - | 10 | us | maximum device auto-initialization time |
| tZQINIT | 1 | - | us | ZQ initialization calibration |
| tCKb | 18 | 100 | ns | boot clock period for MRR during init |

## 5.2	Reset without power ramp

An MRW RESET issued later (with all banks idle) restarts initialization at Td: mode registers return to defaults, wait tINIT4, poll DAI or wait tINIT5, redo ZQ initialization, then reconfigure MR1/MR2/MR3/MR11. SDRAM contents are undefined after MRW RESET.

## 5.3	Power-off sequence

Controlled power-off is the ramp rules in reverse. Hold CKE LOW (<= 0.2 x VDDCA); keep all other inputs between VILmin and VIHmax; outputs remain High-Z. DQ, DM, DQS_t and DQS_c must remain between VSSQ and VDDQ; CK_t, CK_c, CS_n and CA must remain between VSSCA and VDDCA. Tx is when any supply first drops below its minimum; Tz is when all supplies are below 300 mV. Between Tx and Tz the same supply-order inequalities apply as during ramp, and VSS pins must stay within 100 mV. The fall from Tx to Tz must not exceed tPOFF = 2 s.

Uncontrolled power-off is tolerated only with tighter limits: at Tx all supplies must turn off and supply current capacity must reach zero (except residual charge); VDD1 and VDD2 must fall with slope <= 0.5 V/us between Tx and Tz; and this may occur at most 400 times over the device’s life.

## 5.4	Notes that bite

- Mode-register defaults are valid only after MRW RESET and DAI completion; before that they are undefined.

- MR1, MR2 and MR3 must be written even if the default values look acceptable.

- Boot-frequency AC relaxations (tDQSCKb, etc.) apply until the device is fully configured.

- Deep power-down exit requires the full power-up initialization sequence from step 3.

Source: JESD209-3C sections 3.3.1, 3.3.2, 4.11, 4.11.1.1

# 6	Mode Registers

LPDDR3 replaces the MRS/EMRS encoding of desktop DDR with two dedicated commands: MRW (write) and MRR (read). Both carry an 8-bit register address (MA0-MA7) on the CA bus, and each register is 8 bits wide (OP0-OP7). MRW is allowed only when all banks are idle; MRR may be issued from the all-banks-idle or banks-active state.

## 6.1	Register map (SDRAM view)

| MR | Name | Access | Contents |
| --- | --- | --- | --- |
| MR0 | Device Info | R | DAI, RZQI, WL Set B support, RL3 option support |
| MR1 | Device Feature 1 | W | BL, nWR |
| MR2 | Device Feature 2 | W | RL/WL, nWRE, WL set select, WR leveling enable |
| MR3 | I/O Config 1 | W | DS (drive strength) |
| MR4 | Refresh Rate | R | Temperature-compensated refresh multiplier, TUF |
| MR5 | Basic Config 1 | R | Manufacturer ID |
| MR6 | Basic Config 2 | R | Revision ID1 |
| MR7 | Basic Config 3 | R | Revision ID2 |
| MR8 | Basic Config 4 | R | I/O width, density, type |
| MR9 | Test Mode | W | Vendor-specific test mode |
| MR10 | IO Calibration | W | ZQ calibration codes |
| MR11 | ODT Control | W | DQ ODT value, power-down ODT behavior |
| MR16 | PASR Bank Mask | W | Per-bank self-refresh mask |
| MR17 | PASR Segment Mask | W | Per-segment self-refresh mask |
| MR32 | DQ Calibration Pattern A | R | Read-only training pattern |
| MR40 | DQ Calibration Pattern B | R | Read-only training pattern |
| MR41 | CA Training 1 | W | Enter CA training |
| MR42 | CA Training 2 | W | Exit CA training |
| MR48 | CA Training 3 | W | Switch CA-to-DQ mapping during CA training |
| MR63 | Reset | W | Any write triggers MRW RESET |

Rules: RFU bits are written 0 and read as 0; reserved registers must not be written; writes to read-only registers have no effect.

## 6.2	Programming rules

- MRW can only be issued when all banks are idle.

- The MRW command period is tMRW = 10 tCK; only NOP is allowed during tMRW.

- The delay from MRW registration to the next valid non-MRW command is tMRD = max(14 ns, 10 nCK).

- The MRR command period is tMRR = 4 tCK; only NOP is allowed during tMRR.

- MRR data is valid only on DQ[7:0] on the first beat; the remaining beats are undefined.

- Minimum MRR-to-MRW latency = RL + RU(tDQSCKmax/tCK) + BL/2 + 1 clock cycles.

- Minimum MRR-to-write latency = RL + RU(tDQSCKmax/tCK) + BL/2 + 1 - WL clock cycles.

## 6.3	MR0 - Device Information

| Field | Bits | Meaning |
| --- | --- | --- |
| DAI | OP0 | 0 = auto-initialization complete; 1 = still in progress |
| RZQI | OP4:3 | ZQ-pin self-test result (optional) |
| WL Set B support | OP6 | 1 = device supports WL Set B |
| RL3 option support | OP7 | 1 = device supports RL=3/nWR=3/WL=1 at <= 166 MHz |

## 6.4	MR1 - Device Feature 1

| Field | Bits | Encodings |
| --- | --- | --- |
| BL | OP2:0 | 011 = BL8 (default); all others reserved |
| nWR (nWRE=0) | OP7:5 | 001=3 (optional), 100=6, 110=8, 111=9 |
| nWR (nWRE=1) | OP7:5 | 000=10 (default), 001=11, 010=12, 100=14, 110=16 |

Program nWR to RU(tWR/tCK); it sets when the internal precharge starts for a write with auto-precharge.

## 6.5	MR2 - RL and WL

| OP3:0 | Set A (OP6=0) | Set B (OP6=1) |
| --- | --- | --- |
| 0001 | RL=3 / WL=1 | RL=3 / WL=1 |
| 0100 | RL=6 / WL=3 | RL=6 / WL=3 |
| 0110 | RL=8 / WL=4 | RL=8 / WL=4 |
| 0111 | RL=9 / WL=5 | RL=9 / WL=5 |
| 1000 | RL=10 / WL=6 (default) | RL=10 / WL=8 |
| 1001 | RL=11 / WL=6 | RL=11 / WL=9 |
| 1010 | RL=12 / WL=6 | RL=12 / WL=9 |
| 1100 | RL=14 / WL=8 | RL=14 / WL=11 |
| 1110 | RL=16 / WL=8 | RL=16 / WL=13 |

OP4 (nWRE) selects the nWR range; OP6 selects WL Set A or B; OP7 enables write leveling.

## 6.6	MR3 - Drive Strength

| OP3:0 | Nominal impedance |
| --- | --- |
| 0001 | 34.3 ohm pull-down / pull-up |
| 0010 | 40 ohm pull-down / pull-up (default) |
| 0011 | 48 ohm pull-down / pull-up |
| 0100 | reserved for 60 ohm |
| 0110 | reserved for 80 ohm |
| 1001 | 34.3 ohm pull-down / 40 ohm pull-up |
| 1010 | 40 ohm pull-down / 48 ohm pull-up |
| 1011 | 34.3 ohm pull-down / 48 ohm pull-up |

## 6.7	MR4 - Refresh Rate and Temperature

- OP2:0 reports the refresh multiplier (RM) recommended for the current temperature: 4x, 2x, 1x (<= 85 C), 0.5x, 0.25x without AC de-rating, or 0.25x with AC de-rating. Codes 000 and 111 mean the temperature operating limit is exceeded.

- OP7 (TUF) is set when OP2:0 has changed since the last MR4 read; reading MR4 clears TUF.

- When the de-rating code is selected, add 1.875 ns to tRCD, tRC, tRAS, tRP and tRRD, and apply the tDQSCK de-rating from the AC table.

## 6.8	MR10 - ZQ Calibration Codes

| Code | Command | Latency |
| --- | --- | --- |
| 0xFF | Initialization calibration | tZQINIT = 1 us |
| 0xAB | Long calibration | tZQCL = 360 ns |
| 0x56 | Short calibration | tZQCS = 90 ns |
| 0xC3 | ZQ reset (default calibration) | tZQRESET = 50 ns |

If ZQ is tied to VDDCA, the device uses default calibration and ignores ZQ commands.

## 6.9	MR11 - ODT Control

| Field | Bits | Encodings |
| --- | --- | --- |
| DQ ODT | OP1:0 | 00 = disable (default), 01 = RZQ/4, 10 = RZQ/2, 11 = RZQ/1 |
| PD Control | OP2 | 0 = ODT disabled during power-down (default), 1 = ODT enabled during power-down |

RZQ/4 is required for LPDDR3-1866 and -2133 and optional for lower speed grades.

## 6.10	MR16 / MR17 - PASR Masks

- MR16 OP7:0 is a per-bank mask for 8-bank devices; a 1 blocks refresh to that bank.

- MR17 OP7:0 is a per-segment mask; a 1 blocks refresh to that segment of row space in every bank.

Masked regions lose data in self-refresh.

## 6.11	MR32 / MR40 - DQ Calibration Patterns

MRR to MR32 returns Pattern A; MRR to MR40 returns Pattern B. These read-only patterns are used for DQ calibration.

## 6.12	MR41 / MR42 / MR48 - CA Training

- MR41 enters CA training.

- MR48 changes the CA-to-DQ mapping so the remaining CA pins (CA4, CA9) can be calibrated.

- MR42 exits CA training.

MR41 and MR42 use special encodings whose rising-edge and falling-edge values are identical, so the device recognizes them even before CA timing has been adjusted.

## 6.13	MR63 - Reset

Any MRW to MR63 performs a device reset. The operand 0xFC is recommended before CA training because it keeps the CA bus stable across the rising and falling clock edges.

Source: JESD209-3C sections 3.4.1, 4.10, 4.11, 4.11.2, 4.11.3

# 7	Command Truth Tables

LPDDR3 uses a flat, 8-bank topology with no bank groups and no stack IDs. Commands and addresses travel multiplexed over the 10-bit CA bus on both clock edges; a single command spans one full clock cycle. A hardware RESET# pin is also provided for full device reset.

## 7.1	Pin conventions

- CKE(n-1) and CKE(n) are sampled at successive rising edges; CS_n is sampled at the rising edge.

- CAxr is CA bit x on the rising edge; CAxf is the same bit on the falling edge.

- H/L are valid logic levels; X means the pin is driven but its value does not matter.

## 7.2	Command truth table

| Command | CKE(n-1) | CKE(n) | CS_n | CA0r | CA1r | CA2r | CA3r | Falling edge payload |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MRW | H | H | L | L | L | L | L | MA6 MA7, OP0-OP7 |
| MRR | H | H | L | L | L | L | H | MA6 MA7 |
| REFpb | H | H | L | L | L | H | L | X |
| REFab | H | H | L | L | L | H | H | X |
| Enter self-refresh | H | L | L | L | L | H | X | X |
| ACT | H | H | L | L | H | R8 | R9 | R0-R7, R13, R14 |
| WR | H | H | L | H | L | L | X | AP, C3-C11 |
| RD | H | H | L | H | L | H | X | AP, C3-C11 |
| PRE (per-bank/all) | H | H | L | H | H | L | H | X |
| Enter DPD | H | L | L | H | H | L | H | X |
| NOP | H | H | L | H | H | H | X | X |
| Deselect (NOP) | H | H | H | X | X | X | X | X |
| Enter power-down | H | L | H | X | X | X | X | X |
| Exit PD/SREF/DPD | L | H | H | X | X | X | X | X |
| Maintain PD/SREF/DPD | L | L | X | X | X | X | X | X |

Notes that load-bear:

- ACT: CA4r-CA6r carry R10-R12; CA7r-CA9r select BA0-BA2; the falling edge carries R0-R7, then R13-R14.

- RD/WR: CA5r-CA6r carry C1-C2, CA7r-CA9r select the bank; CA0f is AP and CA1f-CA9f carry C3-C11. C0 is implied zero and never transmitted.

- PRE: CA4r = AB (1 = precharge all banks); when AB = 0, CA7r-CA9r select the bank.

- MRW/MRR: CA4r-CA9r carry MA0-MA5; CA0f = MA6 and CA1f = MA7.

- Self-refresh and DPD exits use an asynchronous CKE rise; power-down exit is synchronous.

- NOP can be issued either by driving CS_n high or by driving CS_n low with CA0r-CA2r high.

## 7.3	CKE table

| Current state | CKE(n-1) | CKE(n) | CS_n | Command | Next state |
| --- | --- | --- | --- | --- | --- |
| Active power-down | L | L | X | X | Active power-down |
| Active power-down | L | H | H | NOP | Active (wait tXP) |
| Idle power-down | L | L | X | X | Idle power-down |
| Idle power-down | L | H | H | NOP | Idle (wait tXP) |
| Resetting power-down | L | L | X | X | Resetting power-down |
| Resetting power-down | L | H | H | NOP | Idle if tINIT5 elapsed, else Resetting |
| Self refresh | L | L | X | X | Self refresh |
| Self refresh | L | H | H | NOP | Idle (wait tXSR) |
| Deep power-down | L | L | X | X | Deep power-down |
| Deep power-down | L | H | H | NOP | Power-on (re-init) |
| Banks active | H | L | H | NOP | Active power-down |
| All banks idle | H | L | H | NOP | Idle power-down |
| All banks idle | H | L | L | REF code | Self refresh |
| All banks idle | H | L | L | PRE code | Deep power-down |
| Any (CKE high) | H | H | - | per command table | - |

At least two clock transitions must occur while tXP or tXSR elapses. Sequences not shown are illegal.

## 7.4	State table highlights

| Current state of bank n | Command to bank n | Next state |
| --- | --- | --- |
| Idle | ACT | Row active (after tRCD) |
| Idle | REFpb/REFab | Refreshing |
| Idle | MRW/MRR/Reset/PRE | As defined |
| Row active | RD/WR | Reading/Writing |
| Row active | PRE | Precharging -> Idle |
| Row active | MRR | Active MR reading |
| Reading | RD/WR (after current burst) | Reading/Writing |
| Writing | WR/RD (after current burst) | Writing/Reading |
| Refreshing | NOP only | Idle after tRFC |
| MR writing | NOP only | Idle after tMRW |

A command other than NOP must not interrupt an in-flight burst, refresh interval, or MRW window; NOPs are required on every rising edge during those intervals.

Source: JESD209-3C sections 4.1, 4.7, 4.8, 4.9, 4.10-4.11, 4.13-4.17

# 8	Commands: Rules and Constraints

LPDDR3 has no Mode Register Set command; mode-register updates use MRW and reads use MRR. There is no DLL, no additive latency, no burst terminate, and no burst chop. BL8 is the only burst length.

## 8.1	ACT - Activate

Opens row R in bank B so column accesses can follow.

- Encoding: CS_n low, CA0r low, CA1r high; CA2r-CA3r carry R8-R9, CA4r-CA6r carry R10-R12, and CA7r-CA9r select the bank; the falling edge carries R0-R7 plus R13-R14.

- RD or WR to the same bank may follow after tRCD.

- ACT to a different bank needs tRRD; ACT to the same bank again needs a PRE first and must satisfy tRAS (ACT-to-PRE) and tRC (ACT-to-ACT).

- On 8-bank devices, no more than four ACT or REFpb operations may fall inside a rolling tFAW window.

Key timing:

| Symbol | Rule |
| --- | --- |
| tRRD | max(10 ns, 2 nCK) between ACTs to different banks |
| tFAW | max(50 ns, 8 nCK) rolling window for four bank activations |
| tRAS | min = max(42 ns, 3 nCK); max = min(70.2 us, 9 x RM x tREFI) |
| tRC | tRAS + tRPab (with PREab) or tRAS + tRPpb (with PREpb) |

## 8.2	RD - Burst read

Starts a fixed BL8 read burst from the open row.

- Encoding: CS_n low, CA0r high, CA1r low, CA2r high; CA5r-CA6r = C1-C2, CA7r-CA9r = bank, and the falling edge carries AP plus C3-C11.

- RL is programmed in the mode registers. Data is valid after RL x tCK + tDQSCK + tDQSQ from the command edge. Because there is no DLL, tDQSCK spans a wide min/max range.

- Back-to-back reads to any bank are spaced by tCCD = 4 clocks.

- A PRE to the same bank may follow after BL/2 + max(4, RU(tRTP/tCK)) - 4 clocks, and tRAS must also be satisfied.

- tRTW (book symbol): the earliest WR after a RD is RL + RU(tDQSCKmax/tCK) + BL/2 + 1 - WL clocks after the RD command.

Key timing:

| Symbol | Rule |
| --- | --- |
| tRTP | max(7.5 ns, 4 nCK) analog delay from read to PRE |

## 8.3	WR - Burst write

Starts a fixed BL8 write burst.

- Encoding: CS_n low, CA0r high, CA1r low, CA2r low; column, bank, and AP mapping matches RD.

- WL is programmed in the mode registers. Write data is expected WL x tCK + tDQSS after the command edge. The DM pin masks its byte on any beat of the burst.

- Back-to-back writes are spaced by tCCD = 4 clocks.

- A PRE to the same bank may follow after WL + BL/2 + RU(tWR/tCK) + 1 clocks.

- The earliest RD after a WR is WL + 1 + BL/2 + RU(tWTR/tCK) clocks after the WR command.

Key timing:

| Symbol | Rule |
| --- | --- |
| tWR | max(15 ns, 4 nCK) from last write data to PRE |
| tWTR | max(7.5 ns, 4 nCK) from last write data to RD |

## 8.4	PRE - Precharge

Closes the open row of one bank or all banks.

- Encoding: CS_n low, CA0r high, CA1r high, CA2r low, CA3r high; CA4r = AB (1 = all banks); when AB = 0, CA7r-CA9r select the bank.

- After a single-bank PRE wait tRPpb; after PREab wait tRPab. tRPab is longer than tRPpb on 8-bank devices.

- PRE must not violate tRAS after ACT, tRTP after RD, or tWR after WR.

## 8.5	REFab / REFpb

Refreshes rows in the storage array.

- Encoding: CS_n low, CA0r-CA2r = L L H; CA3r = H for all banks, L for per bank.

- REFab refreshes every bank and requires all banks idle; after it wait tRFCab before any ACT or further refresh.

- REFpb refreshes one bank chosen by the device’s internal 0-1-…-7 counter; the target bank must be idle. After REFpb wait tRFCpb before an ACT or refresh to that bank; only tRRD is needed before an ACT to a different bank.

- The controller must track the REFpb counter; it resets after device reset, self-refresh exit, and every REFab.

- All-bank refreshes are normally issued every tREFIM = RM x tREFI, where RM comes from MR4. Up to eight refreshes may be postponed, bounding the maximum gap to 9 x tREFIM.

## 8.6	Self-refresh entry and exit

- Entry: CKE falls while the REF code is on the bus and all banks are idle. Two NOPs are required after CKE goes low (tCPDED). The clock may be stopped or changed tCPDED after entry, but must be stable for at least two cycles before exit.

- Minimum stay is tCKESR.

- Exit: CKE rises asynchronously; issue NOPs until tXSR = max(tRFCab + 10 ns, 2 nCK) expires. At least one refresh (one REFab or eight REFpb) must be issued before re-entering self refresh.

- PASR: MR16 masks whole banks and MR17 masks eight segments per bank during self refresh. A region is retained only when both its bank mask bit and its segment mask bit are unmasked.

| Region | Mask control | Retained in self refresh when |
| --- | --- | --- |
| Whole bank n | MR16 bit n | bit n = 0 |
| Segment s of every bank | MR17 bit s | bit s = 0 |
| Bank n, segment s | MR16 bit n and MR17 bit s | both bits = 0 |

## 8.7	MRR / MRW

- MRR encoding: CS_n low, CA0r-CA3r = L L L H; register address on CA4r-CA9r and CA0f-CA1f. Burst length is fixed at 8; valid mode-register data appears on DQ0-DQ7 on the first beat, while later beats are undefined (DQ calibration patterns MR32/MR40 are the exception). DQS toggles for the full burst. tMRR = 4 clocks between MRR commands; only NOPs are allowed during tMRR.

- After idle power-down exit, wait an additional tMRRI = tRCD before MRR.

- MRW encoding: CS_n low, CA0r-CA3r = L L L L; register address on CA4r-CA9r and CA0f-CA1f, data on CA2f-CA9f. MRW is allowed only when all banks are idle; tMRW = 10 clocks, only NOPs allowed. MRW to MR63 triggers device reset and leaves array data undefined.

## 8.8	Calibration and training

- ZQ calibration: initiated by MRW. ZQINIT runs at init; ZQCL is a long recal; ZQCS is a short periodic cal; ZQRESET returns to default. All banks must be idle, ODT disabled, and the DQ bus quiet during ZQINIT/ZQCL/ZQCS.

- CA training: enter via MRW to MR41, calibrate CA0-CA3/CA5-CA8, switch CA-to-DQ mapping via MR48, calibrate CA4/CA9, then exit via MR42.

- Write leveling: enter by setting MR2[7] high. The DRAM asynchronously feeds back the CK sample on DQ for each DQS group; the controller delays DQS until the 0-to-1 transition is found. Only NOPs or the exit MRW are allowed.

- DQ calibration: MRR to MR32 returns Pattern A; MRR to MR40 returns Pattern B on the designated DQ pins.

## 8.9	ODT

LPDDR3 termination is asynchronous, controlled by the ODT pin and enabled by MR11[1:0]. ODT is automatically disabled during reads, self refresh, deep power-down, and CA training; it may optionally stay on during power-down via MR11[2].

## 8.10	Power-down

- Entry: CKE falls with CS_n high (NOP/Deselect). Active power-down has at least one row open; idle power-down has all banks idle. No refreshes occur, so dwell time is bounded by the refresh schedule.

- CKE must not fall during MRR, MRW, RD, or WR; it may fall after ACT/PRE/REF once their timing allows.

- Exit: CKE rises with CS_n high; first valid command after tXP = max(7.5 ns, 3 nCK). At least two clock transitions must occur during tXP.

## 8.11	Deep power-down

- Entry: CKE falls with the PRE code on the bus; all banks must be idle. Array contents are lost.

- The device must stay in DPD for at least tDPD = 500 us.

- Exit: CKE rises; the full power-up initialization sequence is required.

## 8.12	Clock stop and frequency change

Clock stop or frequency change is allowed only when CKE is low and no ACT/PRE/RD/WR/MRW/MRR are in flight, or when CKE is high with CS_n high and all related timings (tRCD, tRP, tWR, tMRW, etc.) are satisfied. After a CKE-high change or stop, normal operation resumes only after the clock is stable for at least 2 x tCK + tXP.

## 8.13	NOP / Deselect

Holds the bus idle and never aborts an in-progress operation. It is the only command legal during most power-state transitions and during tRFC/tMRW/tMRR windows.

Source: JESD209-3C sections 4.1-4.6, 4.7-4.8, 4.9, 4.10-4.11, 4.12, 4.13-4.16

# 9	Walkthrough: AXI Read, Row Already Open

Configuration used in all Chapter 4 traces (the drill model): BL = 8, RL = 6, WL = 3, flat 8-bank device. Banks B0-B7, rows R0-R7, columns C0-C7.

## 9.1	Scenario

An AXI4 master issues a read that maps to bank B3, row R5, column C2. Row R5 is already open in B3, so the access is a row hit.

## 9.2	Command trace

RD  B3 C2        ; row already open - issue the burst immediately
NOP              ; tCCD = 4 clocks minimum between column commands
NOP              ; read data returns after RL*tCK + tDQSCK
NOP              ; DQS may arrive over a wide window because there is no DLL

## 9.3	Why each command

- RD B3 C2 - the row is open, so no ACT is needed. CA2r=H selects a read; C2 is the burst start column; AP=0 (CA0f) so the row stays open.

- LPDDR3 uses a fixed burst length of 8. There is no burst-chop option; every RD delivers eight contiguous words from the array.

- The first DQS edge appears RL clocks after the command, plus the tDQSCK window. Because LPDDR3 has no DLL, tDQSCK is a loose min-to-max range that can span more than one clock. The controller must sample data relative to the strobe, not to a fixed CK edge.

- No PRE: leaving the row open is the open-page policy; a later access to R5 in B3 is again a hit.

## 9.4	AXI view

- AR channel: one burst request. With a 32-bit DQ bus, BL8 covers 32 bytes in one DRAM burst.

- A wider AXI transaction would be split into multiple RD commands spaced tCCD apart, e.g. RD B3 C2, then RD B3 C4 four clocks later.

## 9.5	What to notice

- A row hit costs exactly one CA cycle on the command bus. Everything else (tCCD, turnaround) is about scheduling the next access around it.

- The controller tracks one open row per bank; that bookkeeping determines whether an AXI request becomes a hit or a miss.

Source: JESD209-3C sections 3, 4.2, 4.4, 4.17.1 (derived walkthrough)

# 10	Walkthrough: AXI Read, Wrong Row Open

## 10.1	Scenario

An AXI4 read request maps to bank B3, row R6, column C0 - but row R5 is currently open in B3. This is a row conflict (miss): the wrong row must be closed before the requested row can be opened.

## 10.2	Command trace

PREpb B3         ; close the wrong row (tRAS from its ACT already met)
--- (tRPpb) ---  ; single-bank precharge time: bank unavailable
ACT B3 R6        ; open the requested row
--- (tRCD) ---   ; row-to-column delay
RD  B3 C0        ; burst read, AP=0, row stays open
NOP
NOP
NOP

## 10.3	Why each command

- PREpb B3 - a bank holds exactly one open row. AB=0, so only B3 is precharged; every other bank is untouched. This is the per-bank precharge path, which uses the shorter tRPpb recovery.

- --- (tRPpb) --- - single-bank precharge time. A PREpb to one bank uses tRPpb; a PREab would cost tRPab and close banks that might still be doing useful work.

- ACT B3 R6 - opens the requested row. The ACT-to-ACT spacing to B3 (prior ACT -> PREpb -> this ACT) satisfies tRC = tRAS + tRPpb by construction if tRAS was met before the PREpb.

- --- (tRCD) --- - row-to-column delay; the column circuitry cannot be driven until the row’s data is in the sense amps.

- RD B3 C0 - the read itself, identical to the hit case once the row is open.

## 10.4	AXI view

Same single AR request as the hit walkthrough - but the latency the master sees is now tRPpb + tRCD longer. Nothing on the AXI side changes; the penalty is entirely in DRAM scheduling.

## 10.5	What to notice

- Miss cost = tRPpb + tRCD before the first data can even start its RL journey.

- If the previous access to B3 had used AP=1, the bank would already be idle and only tRCD would be owed - that is the close-page trade in one line.

- PREpb may not be issued earlier than tRAS after the ACT that opened R5, nor earlier than tRTP after a read from R5. A scheduler that wants the earliest PREpb starts the tRAS/tRTP counters at the ACT/RD, not at the miss.

Source: JESD209-3C sections 4.1, 4.4, 4.7, 4.17.1 (derived walkthrough)

# 11	Walkthrough: AXI Write to an Idle Bank

## 11.1	Scenario

An AXI4 write request maps to bank B1, row R2, column C0. B1 is idle (no open row).

## 11.2	Command trace

ACT B1 R2        ; open the row (bank was idle - no PRE needed)
--- (tRCD) ---   ; row-to-column delay
WR  B1 C0        ; burst write, AP=0; write data starts WL*tCK + tDQSS later
NOP              ; burst occupies the DQ bus; tWR recovery runs from
NOP              ; the last data, then the bank may be precharged
NOP
NOP
NOP
NOP
NOP

## 11.3	Why each command

- ACT B1 R2 - an idle bank needs no precharge; ACT is legal as long as tRRD from the previous ACT (any bank) and the tFAW window (max 4 ACTs in a rolling tFAW on 8-bank parts) are met.

- --- (tRCD) --- - same row-to-column delay as a read.

- WR B1 C0 - CA2r=L selects a write. The controller drives DQS with a tWPRE preamble and the first data WL clocks (plus tDQSS) after the command. Each beat must meet tDS/tDH around the DQS edge; DM pins mask any beats that should not be written.

- The trailing NOPs cover tWR (write recovery from the end of the burst data): the bank may not be precharged until tWR has elapsed, because the write is still being committed to the array.

## 11.4	AXI view

- AW and W channels carry the request; the DRAM does not see them separately. The controller issues WR only when it has (or is about to have) the write data, because LPDDR3 gives it no way to stall mid-burst.

- B (write response) can be returned once the data is accepted by the DRAM schedule; the tWR window is a DRAM-internal matter.

## 11.5	What to notice

- An idle-bank write miss costs tRCD, half the read-miss penalty - no tRP because nothing was open.

- Alternative: issue WR B1 C0 with AP=1. The device then auto-precharges after the burst, and the bank returns to idle with no explicit PREpb - at the price of having programmed nWR = RU(tWR/tCK) correctly in MR1.

- tWR is the write-side analog of tRTP on the read side: both gate the PRE; both exist because the burst’s last data and the array’s precharge cannot overlap.

Source: JESD209-3C sections 4.1, 4.5, 4.7, 4.17.1 (derived walkthrough)

# 12	Walkthrough: Mixed Reads and Writes

## 12.1	Scenario

Three AXI requests arrive close together:

- Read: bank B3, row R5 (open), column C2.

- Write: bank B1, row R2 (open), column C0.

- Read: bank B1, row R2, column C4.

Both rows are already open, so this trace is pure bus scheduling: the DQ bus is shared, and read/write direction changes cost bubbles.

## 12.2	Command trace

RD  B3 C2        ; read burst 1: data out after RL + tDQSCK
--- (tRTW) ---   ; read-to-write turnaround, see below
WR  B1 C0        ; write burst: DQ direction flips
--- (tWTR) ---   ; write-to-read recovery before any new RD
RD  B1 C4        ; read burst 2, same open row as the write - a hit
PREpb B1         ; done with B1 (tRTP after the RD met); close it

## 12.3	Why each gap

- Read-to-write turnaround (book symbol tRTW): the write command may be issued no earlier than RL + RU(tDQSCK(MAX)/tCK) + BL/2 + 1 - WL clocks after the read command. With RL=6, WL=3, BL=8 and tDQSCK(MAX)=2 clocks, that is 6 + 2 + 4 + 1 - 3 = 10 clocks. The bubble exists because read data is still leaving the DRAM and the DQ bus must be idle before the controller starts driving write data. LPDDR3 has no DLL, so tDQSCK is a wide window rather than a locked edge; the controller must budget the worst-case tDQSCK(MAX).

- Write-to-read: the next read command waits WL + 1 + BL/2 + RU(tWTR/tCK) clocks after the write command (3 + 1 + 4 + 4 = 12 clocks here). tWTR lets the write data finish propagating into the array before sense amps are asked to read.

- RD B1 C4 - same bank, same row as the write: a row hit even though a write happened in between. Open-row state is per bank, not per access type.

- PREpb B1 - legal once tRAS (from the ACT that opened R2) and tRTP (from RD B1 C4) are both met. A PREab would have cost tRPab and closed every bank.

## 12.4	AXI view

AXI is oblivious to direction changes; its read and write channels are independent. The DRAM bus is not: every RD->WR or WR->RD transition on the shared DQ pins costs turnaround clocks, so a controller that groups reads with reads and writes with writes (batching) measurably outruns one that serves requests strictly in arrival order.

## 12.5	What to notice

- Turnarounds, not tRCD, dominate lightly-loaded mixed traffic.

- tCCD still applies between column commands: even without the direction change, bursts on the shared bus must not collide. With BL=8, tCCD is 4 clocks.

- tFAW limits activate parallelism: no more than four ACT or REFpb commands may fall inside a rolling tFAW window on this 8-bank device.

- Refresh appears as REFab in most schedules, but REFpb exists as a per-bank alternative that refreshes one bank while others remain active.

Source: JESD209-3C sections 4.4, 4.5, 4.7, 4.8, 4.17.1 (derived walkthrough)

# 13	Data Path

## 13.1	Prefetch and burst

LPDDR3 uses an 8n prefetch: each column access moves eight bits per DQ between the array and the IO, serialized as four UI on each of two clock cycles. The fixed burst length is BL8. There is no burst chop and no interleaved burst type; every column transaction delivers eight consecutive column locations starting from the commanded column address.

## 13.2	Read and write strobes

A single bidirectional differential pair (DQS_t/DQS_c) serves each byte lane.

| Direction | Driver | Alignment | Preamble | Postamble |
| --- | --- | --- | --- | --- |
| Read | DRAM | Edge-aligned to DQ | tRPRE >= 0.9 tCK | tRPST >= 0.3 tCK |
| Write | Controller | Centered in DQ eye | tWPRE >= 0.8 tCK | tWPST >= 0.4 tCK |

There is no DLL. The read strobe is timed with a wide access window rather than a locked phase: tDQSCK ranges from 2.5 ns (min) to 5.5 ns (max). The controller must train its capture logic to absorb this spread, using the MR32/MR40 DQ calibration patterns and the normal read eye.

## 13.3	Read and write latencies

RL and WL are programmed through MR2. Two WL sets (A and B) trade extra write latency for easier system timing at the higher speeds.

| MR2 OP<3:0> | RL | WL Set A | WL Set B | Max CK freq |
| --- | --- | --- | --- | --- |
| 0001B | 3 | 1 | 1 | <= 166 MHz (optional) |
| 0100B | 6 | 3 | 3 | <= 400 MHz |
| 0110B | 8 | 4 | 4 | <= 533 MHz |
| 0111B | 9 | 5 | 5 | <= 600 MHz |
| 1000B | 10 | 6 | 8 | <= 667 MHz (default) |
| 1001B | 11 | 6 | 9 | <= 733 MHz |
| 1010B | 12 | 6 | 9 | <= 800 MHz |
| 1100B | 14 | 8 | 11 | <= 933 MHz |
| 1110B | 16 | 8 | 13 | <= 1066 MHz |

| Symbol | Timing relationship |
| --- | --- |
| tDQSCK | CK edge to first valid DQS edge: 2.5 ns min, 5.5 ns max |
| tDQSQ | DQS-to-DQ skew: max 165/135/115/100 ps at 1333/1600/1866/2133 MT/s |
| RD cmd -> first DQ | RL x tCK + tDQSCK + tDQSQ |
| WR cmd -> first DQS | WL x tCK + tDQSS |
| tDQSS | WR cmd CK edge to first DQS latching edge: 0.75-1.25 tCK |
| tDQSH / tDQSL | DQS high/low pulse width during writes: >= 0.4 tCK |
| tDSS / tDSH | DQS falling edge setup/hold to CK: >= 0.2 tCK |
| tDS / tDH | DQ/DM setup/hold to DQS: 175/150/130/115 ps at 1333/1600/1866/2133 |

| Symbol | Turnaround relationship |
| --- | --- |
| tCCD | CAS-to-CAS delay: 4 tCK |
| tWTR | WR -> RD: max(7.5 ns, 4 tCK), measured from the CK edge after the last write datum |
| tRTW (book symbol) | RD -> WR: RL + RU(tDQSCK(MAX)/tCK) + BL/2 + 1 - WL clock cycles. JESD209-3C does not name this symbol; it is the cross-book convention. |

## 13.4	DM - write data mask

One DM pin per byte lane is sampled on both DQS edges with the write data. When DM is high for a beat, that byte is masked and not written. DM loading matches DQ/DQS loading so timing tracks the data. Partial writes (e.g. AXI W with sparse WSTRB) can use DM instead of a read-modify-write.

## 13.5	What LPDDR3 does not have

- No data bus inversion (DBI) on reads or writes.

- No parity on the CA bus and no CRC on the data bus.

- No on-die ECC.

- No burst chop, no interleaved burst order, and no additive latency.

Every DQ cycle carries payload; the controller must provide any error protection.

## 13.6	Signaling

The DQ/DQS/DM interface is HSUL_12: 1.2 V VDDQ (nominal 1.20 V, min 1.14 V, max 1.30 V), with VREFDQ near VDDQ/2 at the receiver. Unlike LPDDR2, LPDDR3 adds a dedicated ODT pin and programmable on-die termination for the data bus; termination is asynchronous and is covered in the next file.

Source: JESD209-3C sections 3.4.1 (MR1, MR2), 4.3, 4.4, 4.5, 4.6, 4.10.2, 4.11.2, 8.6.1, Table 64

# 14	On-Die Termination

LPDDR3 exposes a dedicated ODT pin that turns termination on or off for each DQ, DQS_t, DQS_c, and DM line. The pin is not sampled by the clock; the decision is asynchronous. Termination is intended to improve channel signal integrity and is controlled independently per device by the memory controller.

## 14.1	Enabling and selecting the termination value

ODT is configured through MR11.

| MR11 OP<1:0> | RTT value | Notes |
| --- | --- | --- |
| 00B | Disabled | Default |
| 01B | RZQ/4 = 60 ohm | Required for 1866/2133; optional for 1333/1600 |
| 10B | RZQ/2 = 120 ohm |  |
| 11B | RZQ/1 = 240 ohm |  |

MR11 OP<2> selects whether ODT remains functional during CKE power-down: 0B disables it in power-down (default); 1B leaves it enabled, in which case VDDQ must stay on during power-down.

The absolute accuracy of the on-die termination is maintained by ZQ calibration (MRW ZQ init, long, or short) against the external 240 ohm RZQ resistor.

## 14.2	Asynchronous ODT timing

Because the ODT pin is asynchronous, the only pin-controlled delays are the pure analog turn-on and turn-off windows.

| Symbol | Definition | Window |
| --- | --- | --- |
| tODTon | ODT pin high to RTT beginning to turn on / fully on | 1.75 ns min to 3.5 ns max |
| tODToff | ODT pin low to RTT beginning to turn off / at high-Z | 1.75 ns min to 3.5 ns max |

## 14.3	ODT during reads

The DRAM automatically disables termination during any read (RD or MRR) and re-enables it after the read data completes, even if the ODT pin stays asserted. The automatic timing is referenced from the read-data clocks:

| Symbol | Definition |
| --- | --- |
| tAODToff | Automatic RTT turn-off after READ data: min = tDQSCK,min - 300 ps, referenced from RL - 2 CK |
| tAODTon | Automatic RTT turn-on after READ data: max = tDQSCK + 1.4 x tDQSQ,max + tCK(avg,min), referenced from RL + BL/2 CK |

## 14.4	ODT during low-power states and training

| State | ODT behavior |
| --- | --- |
| CKE power-down, OP<2>=0 | Disabled within tODTd (max 12 ns); re-enabled within tODTe (max 12 ns) |
| Self-refresh entry | Disabled within tODTd (max 12 ns + 0.5 tCK) |
| Self-refresh exit | Re-enabled within tODTe (max 12 ns) |
| Deep power-down | Disabled within tODTd; full re-init required on exit |
| CA training | Disabled; ODT pin ignored |
| Write leveling | ODT pin high -> DQS_t/DQS_c termination ON, DQ termination OFF; ODT must be high if enabled |

## 14.5	ODT states truth table

| Mode | DQ termination | DQS termination |
| --- | --- | --- |
| Write | Enabled (if MR11 enabled and ODT HIGH) | Enabled |
| Read / DQ calibration | Disabled | Disabled |
| ZQ calibration | Disabled | Disabled |
| CA training | Disabled | Disabled |
| Write leveling, ODT asserted | Disabled | Enabled |

## 14.6	What LPDDR3 does not have

- No synchronous ODT latency (no ODTL-on/off as in DDR3/DDR4).

- No VTT termination rail; termination is purely on-die to VDDQ.

- No DBI, no parity, no CRC, and no on-die ECC.

Source: JESD209-3C sections 4.12, 4.12.1-4.12.7, Table 22, Table 23, Table 54, Table 64

# 15	Core Timing Parameters

The bank-state timings: what each parameter constrains, and the command transition it applies to. Values are from the AC timing table (Table 64); clock conversion is divide by tCK(avg) and round up (RU).

## 15.1	Row-cycle parameters

LPDDR3 carries TWO precharge times - one per-bank, one all-bank - and the choice follows the command, not the register:

| Symbol | Definition | Applies between | Min |
| --- | --- | --- | --- |
| tRCD | Row-to-column delay; three speed grades exist (fast/typ/slow) | ACT(bank x) -> RD/WR(bank x) | max(15/18/24 ns, 3 nCK) |
| tRPpb | Precharge recovery, single bank | PRE(bank x) -> ACT(bank x) | max(15/18/24 ns, 3 nCK) |
| tRPpab | Precharge recovery, all banks | PREA -> ACT(any) | max(18/21/27 ns, 3 nCK) |
| tRAS | Minimum (and maximum) row-open time | ACT(bank x) -> PRE(bank x) | min max(42 ns, 3 nCK); max min(70.2 us, 9 x RM x tREFI) |
| tRC | Full row cycle; built from whichever precharge applies | ACT(bank x) -> ACT(bank x) | tRAS + tRPpab, or tRAS + tRPpb |

The fast/typ/slow tRCD and tRP grades are a low-power knob: a slower setting eases the analog timing budget (and saves power) at the cost of latency. The tRAS maximum is where the refresh multiplier RM (MR4) shows up in scheduling math.

## 15.2	Activate-rate parameters (cross-bank)

| Symbol | Definition | Applies between | Min |
| --- | --- | --- | --- |
| tRRD | Activate-to-activate spacing, any two banks (limits peak array current) | ACT(bank x) -> ACT(bank y) | max(10 ns, 2 nCK) |
| tFAW | Four-activate rolling window: at most 4 ACTs (or REFpb) inside the window | any 4 ACTs within a sliding window | max(50 ns, 8 nCK) |

The spec’s own window example: if RU(tFAW/tCK) = 10 and an ACT lands in clock n, at most three more ACTs may land in clocks n+1 .. n+9. Note tFAW counts REFpb as well as ACT - per-bank refresh consumes activate budget.

## 15.3	Data-to-precharge parameters

| Symbol | Definition | Applies between | Min |
| --- | --- | --- | --- |
| tRTP | Internal analog read must finish before the row closes | last read data(bank x) -> PRE(bank x) | max(7.5 ns, 4 nCK) |
| tWR | Write recovery: last write data committed to the cells before precharge | last write data(bank x) -> PRE(bank x) | max(15 ns, 4 nCK) |

Command-spacing formulas that fall out of these (same bank):

| Transition | Earliest spacing (clocks) |
| --- | --- |
| RD -> PRE | BL/2 + max(4, RU(tRTP/tCK)) - 4 |
| WR -> PRE | WL + BL/2 + RU(tWR/tCK) + 1 |
| RDA/WRA -> ACT | precharge fires automatically after the recovery window; next ACT >= tRP after it actually starts, and >= tRC from the old ACT |

## 15.4	Why these numbers dominate

tRCD and tRP are the row-miss penalty - and LPDDR3 makes the controller choose per precharge whether it pays tRPpb or the larger tRPpab. tRAS + tRP = tRC bounds per-bank row cycling; tRRD and tFAW bound device- wide activate rate, with tFAW also metering per-bank refreshes. The scheduler hides tRCD/tRPpb behind other banks’ data and paces ACTs (and REFpBs) against tRRD/tFAW.

Source: JESD209-3C sections 4.1, 4.7, 11.4 (Table 64)

# 16	Turnaround and Burst Timing

Column-command spacing and bus-direction rules. These decide how full the DQ bus can be kept.

## 16.1	Latencies and CAS-to-CAS

LPDDR3 has no DLL and no additive latency: read and write latency are programmed directly (MR0 OP6/OP7 select the WL set and the optional RL=3/WL=1 low-latency pair) and the latencies scale with clock speed.

| Symbol | Definition | Applies between | Value |
| --- | --- | --- | --- |
| tCCD | Column command to column command, same or different banks | RD/WR -> RD/WR (same direction) | 4 clocks |
| RL | Read latency: command to first data | RD -> first DQS/DQ out | programmed: 3-16 clocks by rate (6/8/9/10/11/12/14/16 at 800-2133 MT/s) |
| WL | Write latency: command to first strobe | WR -> first DQS in | programmed set A: 1/3/4/5/6/8; set B (optional): up to 13 |

BL8 occupies 4 clocks of DQ. Seamless same-direction traffic issues a column command every BL/2 = 4 clocks, which is exactly tCCD - back-to-back BL8 bursts just saturate the bus. There is no burst chop: BL8 is the only burst length, and no interleaved burst type exists.

## 16.2	Direction changes

Note on tRTW: JESD209-3C never names the read-to-write parameter (the symbol does not appear in the spec). It does specify the spacing, and the spacing depends on tDQSCK because without a DLL the read strobe can lag the clock by more than a clock. This book calls the parameter tRTW, matching the other books in this series.

| Symbol | Definition | Applies between | Value |
| --- | --- | --- | --- |
| tRTW | Read-to-write turnaround: the loose read strobe (tDQSCK) plus burst must clear before the write preamble (book symbol; the spec gives the relation unnamed) | RD(any bank) -> WR(any bank) | RL + RU(tDQSCK(MAX)/tCK) + BL/2 + 1 - WL |
| tWTR | Write-to-read: last write data must clear the input path before a read reuses the array | last write data(any) -> RD(any) | max(7.5 ns, 4 nCK) |

Command-spacing forms:

| Transition | Spacing from first command (clocks) |
| --- | --- |
| RD -> WR | RL + RU(tDQSCK(MAX)/tCK) + BL/2 + 1 - WL |
| WR -> RD | WL + 1 + BL/2 + RU(tWTR/tCK) |
| RD -> RD | max(tCCD, BL/2) |
| WR -> WR | max(tCCD, BL/2) |

Worked example (the Chapter 4 configuration: RL = 6, WL = 3, BL8, tWTR = 4 clk, tDQSCK = 2 clk): RD->WR = 6 + 2 + 4 + 1 - 3 = 10 clocks, WR->RD = 3 + 1 + 4 + 4 = 12 clocks. The asymmetry is the write path’s long internal tail; reads turn into writes cheaper than writes turn into reads. The tDQSCK term is the LPDDR3 signature: a DLL-based DRAM replaces it with a locked value, an LPDDR3 controller must budget the whole min/max window.

## 16.3	Strobe-level timing (for completeness)

These bound the DQS/DQ relationship; the PHY must meet them. With no DLL, several are wide windows rather than tight skews:

| Symbol | Definition | Value |
| --- | --- | --- |
| tDQSCK | Read DQS access from CK_t/CK_c: a 2.5-5.5 ns window (not a locked skew) | min 2500 ps, max 5500 ps; 5620 ps derated |
| tDQSCKDS/DM/DL | DQSCK drift across 160 ns / 1.6 us / 32 ms rolling windows (guaranteed by design, untested) | 265/593/733 ps max at 1333, shrinking with rate |
| tDQSQ | DQ skew against DQS, per group | 165 ps max at 1333, down to 100 ps at 2133 |
| tQH | DQ/DQS output hold | min(tQSH, tQSL) |
| tDQSS | Write DQS first latching edge to CK | 0.75 to 1.25 tCK (wide - no DLL) |
| tWPRE / tWPST | Write DQS preamble / postamble | 0.8 / 0.4 tCK |
| tRPRE / tRPST | Read DQS preamble / postamble | 0.9 / 0.3 tCK |
| tDS / tDH | Write DQ setup / hold against DQS | 175 ps min at 1333, down to 115 ps at 2133 |
| tDQSH / tDQSL | Write DQS input high/low pulse width | 0.4 tCK |

## 16.4	Temperature derating

Above the 85 C threshold the core analog timings stretch: tRCD, tRC, tRAS, tRP and tRRD each carry a +1.875 ns derated minimum, and tDQSCK’s max becomes 5620 ps. A controller that runs hot must add the derating on top of the programmed values rather than discovering it on the bus.

Source: JESD209-3C sections 4.4, 4.5, 11.4 (Table 64)

# 17	Refresh and Self-Refresh Timing

## 17.1	Refresh parameters

LPDDR3 counts refresh the LPDDR way: a fixed number of REFRESH commands per refresh window, at either all-bank or per-bank granularity.

| Symbol | Definition | Value |
| --- | --- | --- |
| tREFW | Refresh window: every row refreshed inside it | 32 ms (16 ms at 1/2-rate, 8 ms at 1/4-rate PASR settings) |
| R | Required REFab commands per window | 4096 (8192 at 8 Gb and above) |
| tREFI | Average interval, REFab | 7.8 us |
| tREFIpb | Average interval, REFpb (per bank) | 0.975 us at 1 Gb, 0.4875 us above (8 x tREFIpb covers all banks) |
| tRFCab | All-bank refresh cycle time | 130 ns (1-4 Gb), 210 ns (6-8 Gb) |
| tRFCpb | Per-bank refresh cycle time | 60 ns (1-4 Gb), 90 ns (6-8 Gb) |

Rules:

- All banks must be idle (and tRP met) before REFab; REFpb only needs its target bank idle, and the target walks banks round-robin via an internal counter.

- REFpb buys scheduling freedom: one bank restores while the others keep serving. Eight REFpb (one per bank) equals one REFab of coverage.

- tFAW counts REFpb as an activate-class command - a cluster of per-bank refreshes eats activate budget.

- The refresh rate is set through MR4 (full, 1/2, 1/4 rate; temperature derating flag), and the RM multiplier in the tRAS maximum is the same knob seen from the row-timing side.

- Violating the refresh window silently corrupts data.

At 8 Gb, REFab at 210 ns per 7.8 us is about 2.7% of bus time; per-bank refresh cuts the all-bank stall to 60-90 ns per bank.

## 17.2	Self-refresh

The DRAM refreshes itself with the external clock stopped: the mobile sleep state. PASR (MR16 bank mask, MR17 segment mask) restricts refresh to a fraction of the array; anything outside is lost on entry.

| Symbol | Definition | Value |
| --- | --- | --- |
| tCKE | Minimum CKE pulse width, high or low | max(7.5 ns, 3 nCK) |
| tCKESR | Minimum CKE low width for self-refresh entry to exit | max(15 ns, 3 nCK) |
| tXSR | Exit self-refresh to next valid command | max(tRFCab + 10 ns, 2 nCK) |
| tCKSRE/tCKSRX | Valid clock required after entry / before exit | max(5 nCK, 10 ns) class (clock-stop rules) |

There is no separate DLL-relock wait as in DDR3: LPDDR3 has no DLL. Exit needs a stable clock, CKE high, then tXSR. The temperature sensor (MR4) doubles the internal refresh rate when the die runs hot.

## 17.3	Power-down and deep power-down

| Symbol | Definition | Value |
| --- | --- | --- |
| tXP | Exit power-down to any valid command | max(7.5 ns, 3 nCK) |
| tMRRI | After tXP, extra wait before an MRR | tRCD(min) |
| tCPDED | Command pass disable delay after PD entry | 2 tCK |
| tDPD | Minimum deep power-down time | 500 us |

Power-down keeps the array alive (refresh obligations continue: 9 x tREFI of slack at best); deep power-down does not - DPD drops the array contents and every ODT/input for minimum power, and exit is a complete re-initialization. ODT timing around the power states: disabled within 12 ns of PD entry, 12 + 0.5 tCK of SR/DPD entry, and re-enabled within 12 ns of exit. Clock stop is legal in idle and power-down, which is the other big LPDDR3 power knob alongside DPD.

Source: JESD209-3C sections 4.8, 4.9, 4.13, 4.14, 11.3 (Table 62), 11.4 (Table 64)

# 18	Package and Electrical Notes

A light pass: what matters when connecting an LPDDR3 device, deliberately skipping the full ballout grids (consult the spec figures for those).

## 18.1	Packages

- Two delivery forms: package-on-package (PoP, stacked above the application processor) and discrete FBGA; both use the same signal definitions with per-form ballouts in sec 2.1 and 2.2.

- Organizations are x16 and x32 on a single channel; 8 banks flat, no bank groups. Densities run 1 Gb to 32 Gb (the addressing table carries 6 Gb and 12 Gb steps alongside the powers of two).

- No RESET# pin: reset is a command (MRW to the reset register). There IS a dedicated ZQ pin bonding to the external 240 ohm reference resistor.

## 18.2	Supplies and signaling

| Rail | Nominal | Feeds |
| --- | --- | --- |
| VDD1 | 1.2 V class | core/array |
| VDD2 | 1.14-1.30 V | core power |
| VDDQ | 1.14-1.30 V | I/O buffers |
| VREFCA / VREFDQ | 0.5 x VDDQ | input references |

- HSUL_12 (High-Speed Unterminated Logic, 1.2 V): the bus is unterminated by design - no VTT rail, no board termination. Signal integrity comes from controlled drive strength (MR-programmed, ZQ- calibrated) plus the on-die termination for DQ, not from a terminated channel.

- CA bus: single-clock-rate command/address on a narrow, shared CA pin group (commands span two cycles); CK_t/CK_c differential; DQS_t/DQS_c differential per byte group, driven by the DRAM on reads and by the controller on writes.

## 18.3	Speed grades

Per the latency table (Table 63): data rates 333 through 2133 MT/s (tCK 6 ns down to 0.938 ns), with the programmed RL/WL pair tracking the rate - RL 6 to 16, WL 3 to 8 (set A) across the mainstream range, plus the optional RL=3/WL=1 low-latency pair and WL set B. Boot operation runs from a 10-55 MHz clock (tCKb 18-100 ns) during initialization.

## 18.4	Derating and characterization

Above 85 C the core timings take +1.875 ns derating and tDQSCK stretches to 5620 ps max (see the turnaround chapter). Section 10 carries the IDD measurement conditions (per-state current tables, the mobile power budget); section 9 the pin capacitance budgets. Neither changes scheduling; consult them when sizing the power delivery.

Source: JESD209-3C sections 2, 6.1, 11.3 (Table 63)

# 19	Test Features

LPDDR3 has no JTAG boundary scan, no DDR3-style MPR, no OCD calibration engine, and no standard write-CRC or gear-down mode. Instead it exposes a small set of calibration and observation mechanisms, all reached through MRW or MRR.

## 19.1	MRR as the observation port

A Mode Register Read returns the contents of any mode register. The command is recognized when CS_n is low with CA0-CA2 low and CA3 high; the register address is built from the remaining CA fields. Valid data appears on DQ[7:0] on the first beat after the programmed read latency, and the burst is always eight clocks long. Only NOP is legal during the tMRR interval. MRR is the normal way to read MR0 at boot to confirm device-auto-initialization status, ZQ pin self-test results, and supported write-latency/read-latency options.

## 19.2	DQ calibration (MR32 / MR40)

Because LPDDR3 has no DLL, the controller must train its read capture against a known pattern. MRR to MR32 returns Pattern A and MRR to MR40 returns Pattern B. For x16 parts the expected bits are driven on DQ0 and DQ8; for x32 parts they are driven on DQ0, DQ8, DQ16, and DQ24. The remaining data pins may either repeat DQ0 or drive zero. Pattern A toggles every bit time (1-0-1-0…), while Pattern B produces pairs of identical values (0-0-1-1-0-0-1-1). Unlike ordinary MRR bursts, the DQ calibration bursts define valid content on every beat so the host can tune capture timing on multiple edges.

## 19.3	ZQ calibration (MR10)

Output-driver and ODT calibration is initiated by MRW to MR10 with one of four opcodes:

| Opcode | Command | Typical use |
| --- | --- | --- |
| 0xFF | ZQINIT | Required once after reset; ~1 us |
| 0xAB | ZQCL | Long maintenance calibration; ~360 ns |
| 0x56 | ZQCS | Short maintenance calibration; ~90 ns |
| 0xC3 | ZQRESET | Return to default calibration; max(50 ns, 3 tCK) |

A 240 ohm, +/-1% resistor must be connected between the ZQ ball and ground. ZQ commands are only legal when all banks are precharged, ODT is disabled, and the DQ bus is quiet; only NOP may be issued during the calibration window. If several devices share one ZQ resistor, their initialization, long, and short calibrations must not overlap, although ZQRESET overlap is allowed.

## 19.4	CA training (MR41 / MR48 / MR42)

Commands travel across the narrow CA bus over two cycles, so the controller must align each CA bit to the clock before normal operation. Training is entered by MRW to MR41, performed in two passes, and exited by MRW to MR42.

- First pass (MR41): calibrate CA0, CA1, CA2, CA3, CA5, CA6, CA7, and CA8. The DRAM echoes each CA value on specific DQ pins according to the MR41 CA-to-DQ map.

- Mapping change (MR48): switch the mapping so the remaining pins can be observed.

- Second pass (MR48): calibrate CA4 and CA9.

- Exit (MR42): return to normal operation.

MR41, MR48, and MR42 use special encodings that are intentionally identical on the rising and falling clock edges so the DRAM can recognize them even before CA timing is adjusted. During training the clock phase may be intentionally varied while CS_n is high and CKE is low.

Key CA-training timings:

| Symbol | Definition | Value |
| --- | --- | --- |
| tCAMRD | First CA calibration command after mode entry | 20 tCK |
| tCAENT | First CA calibration command after CKE goes LOW | 10 tCK |
| tCACKEL | CKE LOW after CA-training mode is programmed | 10 tCK |
| tCACKEH | CKE HIGH after the last CA-training result is driven | 10 tCK |
| tCAEXT | CA-training exit command after CKE is HIGH | 10 tCK |
| tCACD | Delay from one CA calibration command to the next | RU(tADR + 2 x tCK) |
| tADR | Data-out delay after a calibration command | 20 ns max |
| tMRZ | MRW CA-training exit command to DQ tri-state | 3 ns min |

## 19.5	Write leveling (MR2[7])

Write leveling lets the controller align each DQS rising edge to the clock at the DRAM pin. The device enters write-leveling mode when MR2[7] is set HIGH and leaves when MR2[7] is cleared. Only NOP or the exit MRW is legal while in this mode.

The controller first holds DQS_t LOW and DQS_c HIGH for at least tWLDQSEN, then issues the first DQS edge no sooner than tWLMRD. The DRAM samples CK with that DQS rising edge and asynchronously drives the sampled value onto every DQ bit in the byte after tWLO. The controller iterates its DQS delay until it sees a 0-to-1 transition, which locks the DQS-to-CK relationship needed for tDQSS.

| Symbol | Definition | Value |
| --- | --- | --- |
| tWLDQSEN | DQS_t/DQS_c delay after entering write-leveling mode | 25 ns min |
| tWLMRD | First DQS edge after entering write-leveling mode | 40 ns min |
| tWLO | Write-leveling output delay | 0 to 20 ns |
| tWLS | DQS rising-edge setup to CK | 135-205 ps |
| tWLH | DQS rising-edge hold from CK | 135-205 ps |

## 19.6	Temperature sensor (MR4)

The on-die temperature sensor is read through MR4. The lower three bits report a refresh-rate multiplier that ranges from 4x down to 0.25x and also indicate whether the die is below the low-temperature limit or above the high-temperature limit. Bit 7 is the Temperature Update Flag (TUF), which is set whenever the 3-bit value has changed since the last MR4 read; reading MR4 clears TUF. The device updates the sensor at least every tTSI = 32 ms, and the value is never older than tTSI after exiting self-refresh or power-down.

## 19.7	What is not here

- No DDR3 MPR; LPDDR3 uses MRR and the DQ calibration registers instead.

- No standard boundary scan on the device.

- No OCD engine; ZQ calibration replaces it.

- No write CRC, no gear-down mode, and no per-pin loopback.

- Vendor test modes exist only as a reserved write-only register (MR9) with vendor-defined behavior.

Source: JESD209-3C sections 4.10, 4.10.1, 4.10.2, 4.11.2, 4.11.3, 4.11.4, 3.4.1