RTL Design Sherpa

RAPIDS beats on the Genesys 2 The FPGA System: Harness, Builds, Flow -- Version 1.1

September 2026

Table of Contents

1 Document Information	8

1.1 Revision History	9

1.2 How to read this book	11

2 The System	12

2.1 What it is	12

2.1.1 Figure 1.1: The bench	13

2.2 The board	14

2.3 The two USB chips	15

2.4 The stack, top to bottom	16

2.5 The design point	17

3 The Harness	18

3.0.1 Figure 2.1: The harness block map	18

3.1 The host front-end	19

3.2 The launch path: stage everything, then GO	20

3.3 Stimulus and checking	21

3.4 The run timer and the on-chip verdict	22

3.5 Memories the DUT owns	23

3.6 Measurement	24

3.7 The debug variant	25

4 One RTL, Several Bitstreams	26

4.1 The rule	26

4.1.1 Figure 3.1: One source tree, several bitstreams	26

4.2 The knobs	27

4.3 The variants and why each exists	28

4.4 The design points	30

4.5 What a build costs and where it lives	31

5 Host Tools and Flow	32

5.1 Make targets	32

5.2 The host tools	33

5.3 One measured cell, end to end	34

5.3.1 Figure 4.1: From a host command to a report cell	34

5.4 Simulation equals board	35

5.5 Reprogram between campaigns	36

List of Figures

Figure 1.1: The bench	13

Figure 2.1: The harness block map	18

Figure 3.1: One source tree, several bitstreams	26

Figure 4.1: From a host command to a report cell	34

List of Tables

Table 1: Table 0.1: Document information	8

Table 2: Table 0.2: Revision history	9

Table 3: Table 1.1: The part and what the harness uses of it	14

Table 4: Table 1.2: The stack	16

Table 5: Table 2.1: The three regions	19

Table 6: Table 2.2: Stimulus and checking	21

Table 7: Table 2.3: What the run timer compares, per half	22

Table 8: Table 2.4: Memories	23

Table 9: Table 3.1: The build knobs	27

Table 10: Table 3.2: The build variants	28

Table 11: Table 3.3: The design points	30

Table 12: Table 4.1: Make targets	32

Table 13: Table 4.2: Host tools	33

List of Waveforms

No waveforms in this document.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 1	Document Information

Table 0.1: Document information

| Field | Value |
| --- | --- |
| Title | RAPIDS beats on the Genesys 2 – The FPGA System |
| Version | 1.3 |
| Date | 2026-10-20 |
| Scope | The rapids_beats characterization system: board, harness, build variants, host flow |
| Not in scope | The RAPIDS core itself (HAS/MAS under projects/components/dma-ip/rapids/docs/); the measured numbers (reports/perf/README.md) |
| Status | Current; written against the tree at the BUG-009 close (commit 37455c992), run timer added 2026-10-15 (commit 32d8a986b) |

## 1.1	Revision History

Table 0.2: Revision history

| Version | Date | Change |
| --- | --- | --- |
| 1.0 | 2026-09-29 | First edition. Written after the 256-bit / 4 KB design point (report v2.0) and the BUG-009 close (report v2.2), so it describes the observers, monitors-in, 16 KB and ILA variants as actually built this month. |
| 1.1 | 2026-10-15 | The harness gains char_run_timer, the run timer and CRC verdict shared with the STREAM harness: Figure 2.1 adds the block, a new section and Table 2.3 describe what it compares per half, Figure 4.1 shows it closing with the meters, and the host now requires its PASS beside the golden match. |
| 1.2 | 2026-10-20 | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |
| 1.3 | 2026-10-20 | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |

## 1.2	How to read this book

Chapter 1 is the bench: what is plugged into what. Chapter 2 is the bitstream around the DUT, block by block, and the launch path a measurement takes. Chapter 3 is the reason there is more than one bitstream, and what each one buys. Chapter 4 is the flow: make targets, the host tools, and how a JSON row becomes a report cell. Every diagram is a Graphviz source under assets/graphviz/, rendered to PNG by regenerate_all_graphviz.sh.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	The System

## 2.1	What it is

rapids_beats is an on-chip characterization system. A synthesizable harness surrounds the RAPIDS beats DMA (split into a read-only SOURCE engine and a write-only SINK engine behind one APB) with seeded pattern generators, CRC checkers, descriptor and semaphore memories, a programmable memory-latency model and four per-cycle bus meters, and exposes the whole control surface to a host over one UART link. The host stages a configuration, fires one GO, and reads back a window of counts that bracket exactly the transfer that ran. Every configuration is golden-validated: the CRC the sink wrote to memory and the CRC the source produced on its stream are compared against a software model of the same LFSR pattern.

The point of the exercise is the number, not the demo: utilization per interface, per cycle, under conditions the host sets (channels, beats, burst length, memory latency, backpressure, channel schedule). Those numbers are the performance report; this book is the apparatus.

### 2.1.1	Figure 1.1: The bench

The bench: host, two USB chips, the FPGA

Source: 01_board_and_host.dot

## 2.2	The board

Digilent Genesys 2, Kintex-7 XC7K325T-2 (ffg900). The harness clock is 100 MHz: the board supplies a 200 MHz LVDS system clock, and rapids_char_genesys2_top turns it into the single-ended 100 MHz aclk through an IBUFDS and an MMCM. Everything downstream of that clock, the UART bridge, the decode, the harness and the DUT, is board-agnostic and was carried over unchanged from the Nexys A7 flow.

Why the Genesys 2 at all: the Artix-7 100T on the Nexys A7 is at its timing limit for RAPIDS beats above four channels at a 512-bit datapath. The Kintex-7 325T has roughly three times the LUTs and a faster speed grade, so the full eight-channel geometry closes 100 MHz with margin, and the instrumented variants of Chapter 3 fit at all. The BOARD knob still selects either target; the Genesys 2 defaults to eight channels, the Nexys to four.

Table 1.1: The part and what the harness uses of it

| Resource | XC7K325T-2 | Note |
| --- | --- | --- |
| Slice LUTs | 203,800 | the observers build of the 256-bit design point uses about 76,000 |
| Block RAM tiles | 445 | the same build uses 44 |
| User LEDs | 8 | the low 8 bits of the harness status bank |
| Reset | red CPU reset button, active low | synchronized inside the pin top |

## 2.3	The two USB chips

The Genesys 2 has two separate USB bridges, and the distinction matters every time a board is set up:

- The FT2232 carries JTAG. Vivado programs the bitstream and talks to the ILA through it. Its serial, 200300B818A0, is what FPGA_JTAG_SERIAL names so a two-board chain (a Nexys A7 sits on the same chain in this lab) programs the right part.

- The FT232R carries the UART. It enumerates as its own /dev/ttyUSB*, and which number it gets depends on plug order. The host tools therefore default to --port auto and probe every ttyUSB for the harness ID register rather than pinning a node.

The link is 115200-8N1 with an ASCII protocol: W addr data and R addr, one 32-bit AXI4-Lite access per line. That is slow, and the harness is built so that slowness never lands inside a measurement (Chapter 2, the launch path).

## 2.4	The stack, top to bottom

Table 1.2: The stack

| Layer | Module | Role |
| --- | --- | --- |
| Pin top | rapids_char_genesys2_top | LVDS clock in, MMCM, reset sync, LEDs; instantiates the pin-level top |
| Pin-level top | rapids_char_top | pins to harness ports; historically the host front-end, now pins plus status only |
| Harness | rapids_char_harness | UART to AXI4-Lite bridge, region decode, harness CSRs, APB master, kick sequencer, generators, checkers, memories, latency model, meters, optional observers |
| DUT | rapids_beats_top | SOURCE (memory to AXIS) and SINK (AXIS to memory), 8 channels, one APB, merged MonBus egress |

The host front-end used to live in the pin-level top. It was moved into the harness on 2026-09-23 so that simulating the harness exercises the board’s own launch path: the cocotb testbench drives the same UART bytestream the host does, and the same decode, CSRs and kick sequencer run. That is what makes “sim equals board” a checkable claim rather than a hope (Chapter 4).

## 2.5	The design point

Since 2026-09-29 the DUT is built at 256-bit AXI4 and AXIS (RAPIDS has one DATA_WIDTH for both) with 128 beats of SRAM per channel, 4 KB. The earlier 512-bit, 256-deep (16 KB) point is what report versions 1.0 to 1.5 measured. The line rate is therefore 3.20 GB/s per direction: 32 bytes per beat at 100 MHz.

The bitstream reports its own geometry. Harness CSR 0x004, BUILD, carries bytes per beat, channel count, log2 of the SRAM depth and the three instrument flags, and the host reads it on first contact and takes its byte and bandwidth arithmetic from there. A host started with a channel count the bitstream does not have refuses to run. Every results file carries that design record, so a number can never be attributed to the wrong build.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	The Harness

rapids_char_harness is the bitstream minus the pins. It has three jobs: give the host a control surface, feed and check the DUT with synthesizable stimulus, and measure without perturbing. Figure 2.1 is the block map; the sections below take the blocks in the order a transfer meets them.

### 3.0.1	Figure 2.1: The harness block map

The harness: front-end, stimulus, memories, DUT, measurement

Source: 02_harness_blocks.dot

## 3.1	The host front-end

uart_axil_bridge turns the UART bytestream into a single 32-bit AXI4-Lite master. A decode on address bits 19:16 splits that master into three regions:

Table 2.1: The three regions

| Region | Word address | What is behind it |
| --- | --- | --- |
| DUT-REG | 0x0_0000 | an APB window into the DUT through apb4_master: address bits 12:0 are the APB byte address, so the SOURCE register space at 0x0000, the SINK space at 0x1000 and the per-channel kick windows are all reachable |
| DESC-LOAD | 0x1_0000 | eight 32-bit holding words assemble one 256-bit descriptor; DESC_KICK issues a single-beat AXI4 write of it into the SOURCE or SINK descriptor RAM; DESC_STATUS reports the write landed |
| HARNESS CSR | 0x2_0000 | generator, checker, memory and monitor control, the status and CRC readback, the meter counts, the run timer and verdict (0x160-0x180), BUILD, ID |

The observers build adds a fourth window for the observers’ own APB configuration and readback; the host tools address it by name through the generated register map, never by offset.

Registers are reached by name everywhere. The harness CSR map and the DUT’s map come out of PeakRDL as Python register maps (rtl/rapids_harness_csr_regmap.py, rtl/rapids_harness_desc_regmap.py, and the DUT’s rapids_regmap.py), and rapids_char_io.py writes fields by name. Offsets churn; names do not.

## 3.2	The launch path: stage everything, then GO

UART is slow, a few hundred microseconds per access. If the measurement window opened at the first CSR write and closed at the last, the numbers would measure the link, not the DMA. Two mechanisms make the numbers trustworthy:

- Atomic launch. Every CSR, every descriptor and every per-channel kick is staged first. A single write to GO then arms the meter window, starts the AXIS generator and fires all the staged kicks on-chip within a few aclk cycles. The kick sequencer holds the kicks; the host never times anything.

- Deterministic close. The host stages OBS_TARGET, the number of productive beats the transfer will produce on the completion interface. The meters freeze the cycle after that count is reached, so the window brackets exactly the transfer, independent of the DUT’s internal idle signalling. Earlier builds keyed the close on system_idle and left the window open until the host read it, which diluted utilization toward zero.

Between runs the host pulses CHANNEL_RESET on both halves. That clears the schedulers and descriptor engines; it does not reach the SRAM controller, which is why BUG-009’s poisoned states survived it and why a campaign that must not inherit history starts from a reprogram.

## 3.3	Stimulus and checking

Everything that produces or consumes data is on-chip and seeded, so the board and the simulator see the same bytes.

Table 2.2: Stimulus and checking

| Block | Side | What it does |
| --- | --- | --- |
| axis4_master_injector | SINK ingress (s_axis) | LFSR data from GEN_SEED, GEN_NBEATS per channel, GEN_CHMASK, GEN_BPP beats per packet, GEN_TDEST; GEN_MODE.INTERLEAVE round-robins the active channels one beat at a time instead of finishing one channel before starting the next |
| axis4_slave_pattern_check | SOURCE egress (m_axis) | per-channel CRC of what arrives; CHK_CTRL.chk_ready_en is the egress backpressure knob |
| axi4_slave_rd_injector | backs m_axi_rd | LFSR read data for the SOURCE, a CRC of what it served |
| axi4_slave_wr_crc_check | backs m_axi_wr | per-channel CRC of what the SINK wrote |
| axi_response_delay x2 | in front of both slaves | RESP_DELAY: hold R and B for a programmed number of cycles, the memory-latency knob of the latency sweeps |

The golden model on the host, rapids_char_golden.py, runs the same LFSR and computes the CRCs a correct DMA must produce. PASS is anchored on the data path: the sink’s write CRC and the source’s egress CRC must equal the model.

## 3.4	The run timer and the on-chip verdict

char_run_timer (u_char_run_timer, from projects/components/utility-ip/misc/rtl/) is the same module the STREAM harness uses, so both DMAs are timed and judged the same way. A RAPIDS run tests one half, and the timer is fed that half’s input and output side, selected by KICK_CFG.HALF:

Table 2.3: What the run timer compares, per half

| Half | Input side | Output side | Stop count |
| --- | --- | --- | --- |
| SOURCE | RD_CRC, the LFSR source behind m_axi_rd | CHK_ACT_CRC, the egress pattern check on m_axis | egress beats |
| SINK | GEN_EXP_CRC, the AXIS generator | WR_CRC, the write CRC check behind m_axi_wr | write beats |

- Clear and start: GO clears the timer and starts it one cycle later.

- Stop: the output-side beat count reaching OBS_TARGET, the same target that closes the meter window. TIMER_CYC_LO/HI freeze there.

- Pass: latched 32 cycles after the stop. Every channel that ran must have both CRCs valid and equal; a channel that never ran neither passes nor fails.

- Stamps: TIMER_R_* / TIMER_W_* hold the cycle of the first and last input-side and output-side beat.

The host requires TIMER_STATUS.pass and every active channel in CRC_MATCH_MASK, as well as the golden match. A board run that fails only the on-chip check is a real mismatch between what entered a half and what left it. BUILD.RUN_TIMER says whether a bitstream carries the timer; the host skips the on-chip check on one that does not.

## 3.5	Memories the DUT owns

Table 2.4: Memories

| Block | Count | Role |
| --- | --- | --- |
| descriptor RAM (sdpram_slave_axi4_axi4) | 2 | one per half; the DUT reads descriptors over m_axi_desc, the host writes them through DESC-LOAD |
| control semaphore RAM (sdpram_slave_axi4_axi4) | 2 | one per half; the control-write master writes and the control-read master reads the same store, so a doorbell write is observable by a gate read |
| always-accept AXI4-Lite responder | 1 | terminates the DUT’s m_axil_mon MonBus egress so it never stalls |

## 3.6	Measurement

Four bus meters are always built, one per data interface: axi_bus_meter on the read and write AXI4 masters, axis_bus_meter on the AXIS in and out. Each classifies every cycle of the window into productive, backpressure, starvation or idle, and the host turns those into utilization and effective bandwidth. They are cheap, and they are the source of truth for report sections 1 to 6.

Two optional instruments, each a build variant of Chapter 3:

- Observers (USE_OBSERVERS=1): the shared axi4_intf_master_observer on the AXI masters and axis4_intf_observer on the AXIS links. Passive taps on the same wires the meters watch, with their own APB window, per channel utilization and read/write latency histograms (HIST_MAX_OUTSTANDING=32 so the latency sweeps do not lose samples). They are what measured STREAM’s five knobs on this DUT (report section 7). OBS_ENABLE_MON_TAPS additionally arms their MonBus event taps.

- Monitors-in (USE_AXI_MONITORS=1 with GEN_MON=1): the in-core AXI and descriptor monitors inside the DUT and the per-channel completion and error emitters that give their packets an egress. On by default in a product; off here by default because the bare build is tuned to close eight channels, and on when the question is what the monitors cost.

## 3.7	The debug variant

tcl/build_ila.tcl defines RAPIDS_CHAR_ILA, under which the harness and the DUT’s engines mark their observation-window nets mark_debug, and the script inserts an ILA on every marked net. The marks expand to nothing in every other build, so they cost the measured bitstreams no logic and no timing. tcl/capture_ila_bug009.tcl is the capture driver: program, arm on r_go or on a late AXI4-write beat count, drive one host run, write the trace as CSV. Its optional poison pre-run and no-reprogram modes exist because BUG-009 was history-dependent.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	One RTL, Several Bitstreams

## 4.1	The rule

There is one harness, one pin top, one board top, one set of filelists and one XDC. Every bitstream this area has ever measured came from that source at a different set of parameters. Nothing is copied to make a variant: a variant is a line in the Makefile, exported into the environment, and tcl/create_project.tcl reads it there and hands it to Vivado as a generic or a define. The alternative, a near-copy of the harness per flavour, is how two designs drift apart while everyone believes they are measuring the same thing; STREAM retired exactly such a pair, and this area never had one.

The same knobs go to the simulator. make verify-sim and make sim pass DATA_WIDTH and SRAM_DEPTH to the cocotb harness testbench as TEST_DATA_WIDTH and TEST_SRAM_DEPTH, so the pre-bitstream gate simulates the geometry the bitstream will have, and a bug that only exists at 128 beats is not hidden by a 512-beat simulation.

### 4.1.1	Figure 3.1: One source tree, several bitstreams

One RTL, the knobs, the builds

Source: 03_build_variants.dot

## 4.2	The knobs

Table 3.1: The build knobs

| Knob | Reaches the design as | Selects |
| --- | --- | --- |
| BOARD | top module, XDC, default CHANNELS | genesys2 (eight channels) or nexys (four) |
| CHANNELS | RAPIDS_NUM_CHANNELS | channel count, also handed to the host so the campaign and the bitstream cannot disagree |
| DATA_WIDTH, SRAM_DEPTH | RAPIDS_DATA_WIDTH, RAPIDS_SRAM_DEPTH | the design point; both readable back through BUILD |
| USE_AXI_MONITORS, GEN_MON | DUT generics | the in-core monitors and their MonBus emitters |
| USE_OBSERVERS, OBS_ENABLE_MON_TAPS | harness generics | the shared interface observers and their event taps |
| tcl/build_ila.tcl | `define RAPIDS_CHAR_ILA | the marked debug nets and an ILA core |

The monitor knobs deliberately carry the same names STREAM’s build-* Makefiles export, so a reader who knows one area’s builds knows the other’s.

## 4.3	The variants and why each exists

Table 3.2: The build variants

| Variant | Knobs | The question it answers | Measured |
| --- | --- | --- | --- |
| Performance (bare meters) | monitors 0, GEN_MON 0, observers 0 | how fast the datapath is, measured by the four cheap meters; the bitstream is tuned to close eight channels at 100 MHz, so this is the reference point every other variant is compared against | report sections 1 to 6, every design point |
| Monitors-in | USE_AXI_MONITORS=1, GEN_MON=1 | what the in-core monitors cost in area and timing, and whether the DUT’s MonBus egress works on silicon at all; also the first build after a monitor-lite update to check timing had not moved | 512-bit, 256-deep, 8 ch (2026-09-28): WNS +0.524 ns, 71,222 LUTs, 44 BRAM tiles; all 172 campaign configurations identical to the bare build |
| Observers | USE_OBSERVERS=1 (taps optional) | STREAM’s five characterization knobs measured by the shared interface observers instead of the bare meters, with latency histograms; the passive taps must not change a single utilization cell, and they did not | 256-bit, 4 KB, 8 ch: WNS +0.608 ns, 75,679 LUTs, 44 BRAM tiles (report v2.0); after the BUG-009 fixes WNS +0.286 ns |
| 16 KB variant | observers build with RAPIDS_SRAM_DEPTH=512 | whether the per-channel buffer depth is in the eight-channel latency knee (it is, report 7.5c) and in the single-channel window (it is not) | WNS +0.651 ns |
| ILA | build_ila.tcl on the bare build | see the engine gates, the ingress allocation and the SRAM controller’s counts during one run; the BUG-009 traces | about 1,100 marked nets; WNS +0.356 ns and +0.584 ns on the two builds this month |

Two things follow from the table. First, the instrumented variants are not redundant with each other: observers and monitors watch from opposite sides of the DUT boundary, and each has a zero the other cannot see. Second, the bare build is where timing is tuned, so any variant that closes with less margin is reporting the cost of its instrument, not a regression of the datapath.

## 4.4	The design points

Table 3.3: The design points

| Point | DATA_WIDTH / SRAM_DEPTH | Line rate | Reports |
| --- | --- | --- | --- |
| Original | 512 / 256 (16 KB per channel) | 6.40 GB/s | v1.0 to v1.5 |
| Current | 256 / 128 (4 KB per channel) | 3.20 GB/s | v2.0 onward |

Every utilization cell of the 256-bit build matched the 512-bit build at the same beat count, so the engines’ per-beat behaviour is width-independent and every gigabyte figure simply halves. Two things the smaller buffer did change: a fixed 82-cycle ingress fill stall per run, and a single-channel write window of about 80 beats instead of about 93. Those are the report’s findings; the point here is that the comparison was possible only because both points came from the same source at different generics.

## 4.5	What a build costs and where it lives

A full flow is synthesis, implementation and bitstream generation, ten to forty minutes on the lab host. make bitstream runs verify-sim first unless BITSTREAM_SKIP_VERIFY=1, refuses to start Vivado if the sink self-check fails, and writes bitstream/rapids_char.bit plus the timing, utilization and DRC reports under reports/. Bitstreams are never committed. The one or two worth keeping are copied out of the tree by make keep into the hold directory (RDS_HOLD_DIR), and make program falls back to that copy when the build directory is clean, saying which file it is programming every time.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Host Tools and Flow

## 5.1	Make targets

The flow Makefile in flows-rapids-beats/ is the one entry point. It exports the knobs of Chapter 3 and drives Vivado, the simulator and the host tools with them.

Table 4.1: Make targets

| Target | What it does |
| --- | --- |
| make sim | the cocotb harness self-check, sink and source, at the Makefile’s DATA_WIDTH and SRAM_DEPTH |
| make verify-sim | the sink self-check only; the gate make bitstream runs first |
| make bitstream | synthesis, implementation, bitstream and reports (BOARD=genesys2 USE_OBSERVERS=1 for the observers build) |
| make program | flash the board over JTAG, naming the file it programs |
| make smoke | a fast golden-validated check over UART, two channels, four beats, both halves |
| make suite | the full sweep, results to a JSON file |
| make flow | sim, bitstream, program, characterize |

## 5.2	The host tools

Table 4.2: Host tools

| Tool | Role |
| --- | --- |
| rapids_char_io.py | the UART link and register access by name through the generated register maps |
| descriptor_builder.py | assembles the 256-bit descriptors the DESC-LOAD region writes |
| rapids_char_golden.py | the software LFSR and CRC model every run is validated against |
| run_characterization.py | the campaign driver: --smoke, a single run (--active, --beats, --xfer-axlen, --interleave, --backpressure, --sink-only / --source-only), or --suite with --suite-channels, --suite-beats, --suite-descs, --suite-xfer, --suite-delay, --suite-bp, --suite-seeds; every row carries the BUILD record and the AxLEN the engines actually ran |
| dump_status.py | pretty-prints the status CSRs of a running board |
| run_sink_once.py | one sink run, for debugging by hand |
| report_tables.py, report_figures.py | rebuild the report’s tables and PNG figures from any campaign’s JSON files |
| plot_char_reports.py, gather_size_sweep.sh | the older plotting and gathering helpers |

run_characterization.py reads BUILD on first hardware contact and refuses a --channels that does not match the bitstream. It warns when a requested AxLEN exceeds the build’s cap (the SRAM depth minus one, since BUG-009) and records the effective value in every row, so a table never labels a 128-beat burst as 256.

## 5.3	One measured cell, end to end

### 5.3.1	Figure 4.1: From a host command to a report cell

The measurement path

Source: 04_measurement_path.dot

For each configuration the driver resets both halves, loads the descriptors, programs the generator and the knobs, stages the kicks and the productive beat target, writes GO, polls the completion counter, and scores: beat totals, CRCs against the golden model, the on-chip verdict from the run timer (Section 2), meter buckets, and on the observers build the histograms. The row goes into the results JSON. The report scripts turn the JSON files under reports/perf/json/ into the tables and figures of reports/perf/README.md, and reports/generate_reports_pdf.sh --rev X --only perf turns that into the versioned DOCX and PDF.

## 5.4	Simulation equals board

The harness simulation drives the same UART bytestream through the same bridge, decode, CSRs and kick sequencer that the board runs, against the same generics. dv/test_rapids_char_harness.py carries the sink and source self-checks, and the environment knobs TEST_NUM_ACTIVE, TEST_NUM_BEATS, TEST_XFER_AXLEN, TEST_RESP_DELAY_RD and TEST_RESP_DELAY_WR, TEST_GEN_INTERLEAVE and TEST_USE_OBSERVERS reach the same registers the host’s options do. A board finding is therefore reproduced by setting those knobs, not by writing a new test.

The limits are worth knowing. The self-checks run one transfer each, so a defect that needs history (a run that ends mid-segment, then a run that needs the whole buffer) is a board reproduction first and a macro-level test in the component area second. And the simulator’s slave models drain at their own rhythm, so a race that depends on a partial allocation landing on a stale view was seen only on silicon, through the ILA variant.

## 5.5	Reprogram between campaigns

A wedged channel is cleared by a reprogram, not by CHANNEL_RESET. The campaign scripts that produce report data therefore program the board before each suite, and a smoke run is treated as history: it ends mid-segment, and the BUG-009 investigation showed what a following full-depth burst did with that.