RTL Design Sherpa

RAPIDS Beats DMA Engine Performance Characterization Report

July 15, 2026

Table of Contents

Readme	5

RAPIDS Beats DMA — Performance Characterization (Genesys 2, 8 channels)	5

Characterization knobs	6

1. Headline	6

2. How it is measured (the observation hooks)	7

3. Single-axis sweep — transfer size	8

4. What we learned	10

5. Limitations & honesty notes	11

Appendix: data files & reproduce	11

List of Figures

No figures in this document.

List of Tables

Table 1: Characterization knobs — the sweep axes	6

Table 2: Headline — 8-channel line-rate at 256 KB/channel (Genesys 2)	6

Table 3: Table — engaged utilization (%) and effective bandwidth (GB/s) vs transfer size	9

# Readme

Generated: 2026-07-15

# RAPIDS Beats DMA — Performance Characterization (Genesys 2, 8 channels)

Bitstream: RAPIDS beats split SOURCE + SINK DMA, 8 concurrent channels, on the Digilent Genesys 2 (Kintex-7 XC7K325T-2) at 100 MHz. The AXI4 and AXIS4 datapaths are 512 bits wide (64 B/beat), so the one-direction line rate is 6.40 GB/s (64 B × 100 MHz). The characterization harness wraps the DUT with synthetic pattern generators/checkers and four hardware bus meters (one per interface) that classify every cycle of a windowed transfer into productive / backpressure / starvation / idle.

Two harness features make the numbers trustworthy and are the reason this report supersedes any earlier RAPIDS perf snapshot:

- Atomic launch (stage-all-then-GO). Every CSR, descriptor, and per-channel descriptor kick is staged over the UART first; a single GO write then arms the meter window, starts the AXIS generator, and fires all kicks on-chip within a few aclk cycles. No UART latency contaminates the measured window.

- Deterministic window close. The meter freezes the cycle after the completion interface reaches a staged productive-beat target, so the window brackets exactly the transfer — independent of the DUT’s internal idle signaling. Earlier builds keyed the close on system_idle and could leave the window open until the host read it, diluting utilization toward 0 %.

Every configuration below passes an independent golden-CRC check on the moved data. Source of truth: the on-chip bus meters, read over CSR.

Metric note. The headline efficiency is engaged utilization — productive / (productive + backpressure + starvation), i.e. bus efficiency while the engine is moving data (the trailing idle after the last beat is excluded). Effective bandwidth is engaged_util × 6.40 GB/s. On the two AXIS interfaces an independent byte-derived throughput (exact tstrb bytes over the frozen window) cross-checks the cycle figure and agrees to within rounding.

## Characterization knobs

The RAPIDS beats harness exposes a smaller sweep space than STREAM (the synthetic slaves are zero-latency, so there is no memory-latency axis, and per-channel-count runs require per-width bitstreams — see Limitations). The axes exercised here:

Characterization knobs — the sweep axes

| # | Knob | What it is | Set by | Values in this report |
| --- | --- | --- | --- | --- |
| 1 | Transfer size (beats/channel) | beats moved per channel per descriptor — the amortization axis | descriptor length / GEN_NBEATS | 256 B (4 beats) … 256 KB (4096 beats) |
| 2 | Path | SOURCE (mem-read → AXIS-out) vs SINK (AXIS-in → mem-write) | which half is kicked | both, every config |
| 3 | Interface | the four monitored buses | — | AXIS-in, AXI4-wr, AXI4-rd, AXIS-out |
| 4 | Channels | concurrent DMA channels | build generic | 8 (full width) |

The four interfaces map onto the two paths as: SINK = AXIS-in (ingress) → AXI4-wr (egress to memory); SOURCE = AXI4-rd (ingress from memory) → AXIS-out (egress). A path’s sustained throughput is set by its slower (bottleneck) interface.

## 1. Headline

At a 256 KB/channel transfer across all 8 channels, every interface runs at 99.8–100 % of line rate — the engine is gapless on both the memory bus and the network bus, in both directions, simultaneously.

Headline — 8-channel line-rate at 256 KB/channel (Genesys 2)

| Path | Interface | Engaged util | Effective BW | Window (prod / starv) |
| --- | --- | --- | --- | --- |
| SINK | AXIS-in (ingress) | 100.0 % | 6.40 GB/s | 32737 / 0 |
| SINK | AXI4-wr (egress) | 100.0 % | 6.40 GB/s | 32768 / 13 |
| SOURCE | AXI4-rd (ingress) | 99.8 % | 6.39 GB/s | 32768 / 50 |
| SOURCE | AXIS-out (egress) | 99.8 % | 6.39 GB/s | 32768 / 50 |

prod = 32768 = 8 channels × 4096 beats on the AXI4 side: every expected beat is accounted for, and the non-productive residue is a handful of cycles of one-time fill latency.

Figure — all four interfaces at the largest transfer sit on the 100 % line.

## 2. How it is measured (the observation hooks)

Each interface has a dedicated meter instantiated in the harness:

- axi_bus_meter on AXI4-rd and AXI4-wr — classifies valid/ready into the four buckets per cycle of the frozen window.

- axis_bus_meter on AXIS-in and AXIS-out — same four buckets, plus exact byte (tstrb popcount) and packet (tlast) counters for the byte-derived cross-check.

The window is armed by GO, opens when the active path goes busy, and freezes deterministically when the completion interface’s productive-beat count reaches the staged target (CSR_OBS_TARGET = channels × beats). Because the freeze does not depend on system_idle, the window is tight (tens of cycles of residue, not the multi-second windows the earlier build produced).

## 3. Single-axis sweep — transfer size

Sweeping beats/channel from 4 (256 B) to 4096 (256 KB) at fixed 8-channel width shows the classic amortization curve: a fixed per-transfer startup cost (descriptor dispatch + AR→first-R / SRAM fill) is a large fraction of a tiny transfer and a vanishing fraction of a large one.

Figure — engaged utilization vs transfer size, all four interfaces.

Figure — effective per-direction bandwidth vs transfer size (line rate 6.40 GB/s).

Table — engaged utilization (%) and effective bandwidth (GB/s) vs transfer size

| Transfer / ch | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out | wr BW | sout BW |
| --- | --- | --- | --- | --- | --- | --- |
| 256 B (4 b) | 0.6 % | 19.5 % | 19.0 % | 19.0 % | 1.25 | 1.22 |
| 1 KB (16 b) | 100.0 % | 72.7 % | 71.1 % | 71.1 % | 4.65 | 4.55 |
| 4 KB (64 b) | 100.0 % | 97.5 % | 92.6 % | 92.6 % | 6.24 | 5.93 |
| 16 KB (256 b) | 100.0 % | 99.1 % | 97.6 % | 97.6 % | 6.34 | 6.25 |
| 64 KB (1024 b) | 100.0 % | 99.9 % | 99.4 % | 99.4 % | 6.40 | 6.36 |
| 256 KB (4096 b) | 100.0 % | 100.0 % | 99.8 % | 99.8 % | 6.40 | 6.39 |

By 4 KB/channel every memory-bus interface is already above 92 %, and by 64 KB the whole engine is within 0.6 % of line rate. AXIS-in reads 100 % from the smallest sizes because the ingress generator is throttled by SRAM back-pressure — its engaged window is all-productive — so the sink’s amortization shows up on the AXI4-wr drain, exactly where the startup cost lives.

Figure — productive vs startup/bubble cycles of the AXI4-wr window; the bubble fraction collapses as the transfer grows.

## 4. What we learned

- The datapath is gapless. At realistic transfer sizes (≥ 64 KB/ch) all four interfaces sit at 99.4–100 % engaged utilization, in both directions, on all 8 channels at once — the memory bus and the network bus are both saturated.

- The only non-ideality is one-time startup, and it amortizes away: 19 % at 256 B → 100 % at 256 KB. There is no steady-state bubble, no per-beat gap, and no back-pressure or starvation once data is flowing.

- Invariance is the result. Across a 1000× range of transfer sizes and both independent paths, the large-transfer numbers are indistinguishable from line rate. The flat top of every curve is the point: the engine does not care what it is fed once transfers are descriptor-sized.

## 5. Limitations & honesty notes

These are harness/measurement constraints, not datapath defects — the moved data is golden-correct in every configuration, including the ones excluded below.

- Channel-count axis needs per-width bitstreams. Running active < build_width on the board wedges the sink (writes 0 beats). All data here is full-width (8/8). A channel-scaling sweep (1/2/4/8) requires a bitstream per width.

- Per-size fresh programming. The DUT wedges after a few back-to-back runs (snk_system_idle sticks), so each size was run on a freshly programmed board and the per-size results merged. A soft-reset CSR would let a single suite sweep cleanly.

- No memory-latency axis. The synthetic slaves are zero-latency, so — unlike STREAM — there is no response-delay knob to show the in-flight-window limit. Adding a RESP_DELAY CSR to the pattern slaves is the natural v1.1 extension.

## Appendix: data files & reproduce

| File | Contents |
| --- | --- |
| perf/json/genesys_8ch_size_sweep.json | merged per-size matrix (this report) |
| perf/json/genesys_8ch_*.json | raw back-to-back sweep runs (show the wedge) |
| perf/plots/*.png | figures above (host/plot_char_reports.py) |

# clean transfer-size sweep (reprograms before each size to avoid the wedge):
source env_python
./projects/NexysA7/rapids_characterization/flows-rapids-beats/host/gather_size_sweep.sh \
    /dev/ttyUSB1 \
    projects/NexysA7/rapids_characterization/reports/perf/json/genesys_8ch_size_sweep.json

# figures:
python3 projects/NexysA7/rapids_characterization/flows-rapids-beats/host/plot_char_reports.py \
    --size   projects/NexysA7/rapids_characterization/reports/perf/json/genesys_8ch_size_sweep.json \
    --outdir projects/NexysA7/rapids_characterization/reports/perf/plots

# this report (DOCX + PDF, house style):
cd projects/NexysA7/rapids_characterization/reports && ./generate_reports_pdf.sh --rev 1.0

Genesys 2 host link: JTAG on the FT2232 (200300B818A0), UART on the separate FT232R (AU05X8RM, /dev/ttyUSB1); both must be connected at once, and the board must not be power-cycled between program and run.