RTL Design Sherpa

RAPIDS Beats DMA Engine Performance Characterization Report

July 15, 2026

Table of Contents

Readme	5

RAPIDS Beats DMA — Performance Characterization (Genesys 2, 8 channels)	5

Characterization knobs	6

1. Headline	6

2. How it is measured (the observation hooks)	7

3. Single-axis sweep — transfer size	8

3.1 Channel count — per-channel independence	10

4. What we learned	11

5. Between-run state reset (root-cause found & fixed)	12

6. Other limitations	13

Appendix: data files & reproduce	13

List of Figures

No figures in this document.

List of Tables

Table 1: Characterization knobs — the sweep axes	6

Table 2: Headline — 8-channel line-rate at 256 KB/channel (Genesys 2)	6

Table 3: Table — engaged utilization (%) and effective bandwidth (GB/s) vs transfer size (8 ch)	9

Table 4: Figure — engaged utilization vs active channel count (256 KB/ch).	11

Table 5: Table — the wedge is stale scheduler state, not an in-flight fabric cycle	12

# Readme

Generated: 2026-07-16

# RAPIDS Beats DMA — Performance Characterization (Genesys 2, 8 channels)

Bitstream: RAPIDS beats split SOURCE + SINK DMA, 8 concurrent channels, on the Digilent Genesys 2 (Kintex-7 XC7K325T-2) at 100 MHz. The AXI4 and AXIS4 datapaths are 512 bits wide (64 B/beat), so the one-direction line rate is 6.40 GB/s (64 B × 100 MHz). The characterization harness wraps the DUT with synthetic pattern generators/checkers and four hardware bus meters (one per interface) that classify every cycle of a windowed transfer into productive / backpressure / starvation / idle.

Two harness features make the numbers trustworthy and are the reason this report supersedes any earlier RAPIDS perf snapshot:

- Atomic launch (stage-all-then-GO). Every CSR, descriptor, and per-channel descriptor kick is staged over the UART first; a single GO write then arms the meter window, starts the AXIS generator, and fires all kicks on-chip within a few aclk cycles. No UART latency contaminates the measured window.

- Deterministic window close. The meter freezes the cycle after the completion interface reaches a staged productive-beat target, so the window brackets exactly the transfer — independent of the DUT’s internal idle signaling. Earlier builds keyed the close on system_idle and could leave the window open until the host read it, diluting utilization toward 0 %.

Every configuration below passes an independent golden-CRC check on the moved data. Source of truth: the on-chip bus meters, read over CSR.

Metric note. The headline efficiency is engaged utilization — productive / (productive + backpressure + starvation), i.e. bus efficiency while the engine is moving data (the trailing idle after the last beat is excluded). Effective bandwidth is engaged_util × 6.40 GB/s. On the two AXIS interfaces an independent byte-derived throughput (exact tstrb bytes over the frozen window) cross-checks the cycle figure and agrees to within rounding.

## Characterization knobs

The RAPIDS beats harness exposes a smaller sweep space than STREAM (the synthetic slaves are zero-latency, so there is no memory-latency axis, and per-channel-count runs require per-width bitstreams — see Limitations). The axes exercised here:

Characterization knobs — the sweep axes

| # | Knob | What it is | Set by | Values in this report |
| --- | --- | --- | --- | --- |
| 1 | Transfer size (beats/channel) | beats moved per channel per descriptor — the amortization axis | descriptor length / GEN_NBEATS | 256 B (4 beats) … 256 KB (4096 beats) |
| 2 | Path | SOURCE (mem-read → AXIS-out) vs SINK (AXIS-in → mem-write) | which half is kicked | both, every config |
| 3 | Interface | the four monitored buses | — | AXIS-in, AXI4-wr, AXI4-rd, AXIS-out |
| 4 | Channels | concurrent DMA channels | build generic | 8 (full width) |

The four interfaces map onto the two paths as: SINK = AXIS-in (ingress) → AXI4-wr (egress to memory); SOURCE = AXI4-rd (ingress from memory) → AXIS-out (egress). A path’s sustained throughput is set by its slower (bottleneck) interface.

## 1. Headline

At a 256 KB/channel transfer across all 8 channels, every interface runs at 99.8–100 % of line rate — the engine is gapless on both the memory bus and the network bus, in both directions, simultaneously.

Headline — 8-channel line-rate at 256 KB/channel (Genesys 2)

| Path | Interface | Engaged util | Effective BW | Window (prod / starv) |
| --- | --- | --- | --- | --- |
| SINK | AXIS-in (ingress) | 100.0 % | 6.40 GB/s | 32737 / 0 |
| SINK | AXI4-wr (egress) | 100.0 % | 6.40 GB/s | 32768 / 13 |
| SOURCE | AXI4-rd (ingress) | 99.8 % | 6.39 GB/s | 32768 / 50 |
| SOURCE | AXIS-out (egress) | 99.8 % | 6.39 GB/s | 32768 / 50 |

prod = 32768 = 8 channels × 4096 beats on the AXI4 side: every expected beat is accounted for, and the non-productive residue is a handful of cycles of one-time fill latency.

Figure — all four interfaces at the largest transfer sit on the 100 % line.

## 2. How it is measured (the observation hooks)

Each interface has a dedicated meter instantiated in the harness:

- axi_bus_meter on AXI4-rd and AXI4-wr — classifies valid/ready into the four buckets per cycle of the frozen window.

- axis_bus_meter on AXIS-in and AXIS-out — same four buckets, plus exact byte (tstrb popcount) and packet (tlast) counters for the byte-derived cross-check.

The window is armed by GO, opens when the active path goes busy, and freezes deterministically when the completion interface’s productive-beat count reaches the staged target (CSR_OBS_TARGET = channels × beats). Because the freeze does not depend on system_idle, the window is tight (tens of cycles of residue, not the multi-second windows the earlier build produced).

## 3. Single-axis sweep — transfer size

Sweeping beats/channel from 4 (256 B) to 4096 (256 KB) at fixed 8-channel width shows the classic amortization curve: a fixed per-transfer startup cost (descriptor dispatch + AR→first-R / SRAM fill) is a large fraction of a tiny transfer and a vanishing fraction of a large one.

Figure — engaged utilization vs transfer size, all four interfaces.

Figure — effective per-direction bandwidth vs transfer size (line rate 6.40 GB/s).

Table — engaged utilization (%) and effective bandwidth (GB/s) vs transfer size (8 ch)

| Transfer / ch | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out | wr BW | sout BW |
| --- | --- | --- | --- | --- | --- | --- |
| 256 B (4 b) | 0.6 % | 19.5 % | 19.0 % | 19.0 % | 1.25 | 1.22 |
| 1 KB (16 b) | 55.1 % | 72.7 % | 71.1 % | 71.1 % | 4.65 | 4.55 |
| 4 KB (64 b) | 91.6 % | 97.5 % | 92.6 % | 92.6 % | 6.24 | 5.93 |
| 16 KB (256 b) | 97.6 % | 99.1 % | 97.6 % | 97.6 % | 6.34 | 6.25 |
| 64 KB (1024 b) | 99.5 % | 99.9 % | 99.4 % | 99.4 % | 6.40 | 6.36 |
| 256 KB (4096 b) | 99.9 % | 100.0 % | 99.8 % | 99.8 % | 6.40 | 6.39 |

By 4 KB/channel every interface is already above 91 %, and by 64 KB the whole engine is within 0.6 % of line rate. All four curves are the same monotonic amortization of a single fixed per-transfer startup cost (descriptor dispatch + AR→first-R / SRAM fill); there is no steady-state bubble.

Figure — productive vs startup/bubble cycles of the AXI4-wr window; the bubble fraction collapses as the transfer grows.

### 3.1 Channel count — per-channel independence

Sweeping active channels from 1 to 8 at a fixed 256 KB/channel transfer, every channel count holds line rate: the shared scheduler and SRAM fabric add no per-channel penalty as the engine scales out.

utilization vs channel count

Figure — engaged utilization vs active channel count (256 KB/ch).

| Channels | AXI4-wr | AXIS-out |
| --- | --- | --- |
| 1 | 99.9 % | 99.8 % |
| 2 | 99.8 % | 97.2 % |
| 4 | 99.9 % | 96.6 % |
| 8 | 100.0 % | 99.8 % |

: Table — utilization vs channel count (256 KB/ch)

## 4. What we learned

- The datapath is gapless. At realistic transfer sizes (≥ 64 KB/ch) all four interfaces sit at 99.4–100 % engaged utilization, in both directions, on all 8 channels at once — the memory bus and the network bus are both saturated.

- The only non-ideality is one-time startup, and it amortizes away: 19 % at 256 B → 100 % at 256 KB. There is no steady-state bubble, no per-beat gap, and no back-pressure or starvation once data is flowing.

- Per-channel independence. 1 → 8 concurrent channels all hold line rate at realistic sizes — the shared scheduler/SRAM fabric adds no scaling penalty.

- Invariance is the result. Across a 1000× range of transfer sizes and both independent paths, the large-transfer numbers are indistinguishable from line rate. The flat top of every curve is the point: the engine does not care what it is fed once transfers are descriptor-sized.

## 5. Between-run state reset (root-cause found & fixed)

Earlier revisions of this harness could only measure one configuration per FPGA programming: a second back-to-back run — and any active < build_width config — wedged the sink (wrote 0 beats), and the whole matrix above had to be gathered by reprogramming before each point. That is now fixed in the host, and the fix pinned the root cause.

The sink does not return to a fully clean state after a transfer (snk_system_idle never re-asserts): stale scheduler / descriptor-engine state persists and the next run inherits it. A discriminating board experiment isolated the mechanism:

Table — the wedge is stale scheduler state, not an in-flight fabric cycle

| Between-run action | Back-to-back result |
| --- | --- |
| none (baseline) | 1 / 4 — wedges on run 2 |
| +0.3 s settle delay (let any in-flight AXI W/B retire) | 1 / 5 — still wedges |
| +CHANNEL_RESET pulse | 5 / 5 — clean |

The 0.3 s settle (30 M cycles ≫ any outstanding-B retirement) not helping rules out a stuck datapath/fabric cycle; the existing CHANNEL_RESET.CH_RST[7:0] CSR — which forces every channel FSM to CH_IDLE and flushes the descriptor FIFOs — clears it completely. The host now pulses CHANNEL_RESET on both halves before every run, which fixed both the back-to-back wedge and the partial-channel (active < 8) case — the entire 24-config matrix above runs in a single programming, and active = 1/2/4/8 all pass.

Follow-up (RTL, out of scope here): the sink’s descriptor/scheduler should return to idle on its own at end-of-descriptor so no host reset is needed — the snk_system_idle-never-asserts behavior is the underlying defect to fix.

## 6. Other limitations

- No memory-latency axis. The synthetic slaves are zero-latency, so — unlike STREAM — there is no response-delay knob to show the in-flight-window limit. Adding a RESP_DELAY CSR to the pattern slaves is the natural v1.1 extension.

## Appendix: data files & reproduce

| File | Contents |
| --- | --- |
| perf/json/genesys_full_matrix.json | channel × size matrix (this report) |
| perf/json/genesys_8ch_*.json | earlier back-to-back runs (show the pre-fix wedge) |
| perf/plots/*.png | figures above (host/plot_char_reports.py) |

# full channel x size matrix in ONE programming (CHANNEL_RESET per run):
source env_python
python3 projects/NexysA7/rapids_characterization/flows-rapids-beats/host/run_characterization.py \
    --port /dev/ttyUSB1 --channels 8 --suite \
    --suite-channels 1,2,4,8 --suite-beats 4,16,64,256,1024,4096 --suite-bp off \
    --results projects/NexysA7/rapids_characterization/reports/perf/json/genesys_full_matrix.json

# figures:
python3 projects/NexysA7/rapids_characterization/flows-rapids-beats/host/plot_char_reports.py \
    --size   projects/NexysA7/rapids_characterization/reports/perf/json/genesys_full_matrix.json \
    --outdir projects/NexysA7/rapids_characterization/reports/perf/plots

# this report (DOCX + PDF, house style):
cd projects/NexysA7/rapids_characterization/reports && ./generate_reports_pdf.sh --rev 1.0

Genesys 2 host link: JTAG on the FT2232 (200300B818A0), UART on the separate FT232R (AU05X8RM, /dev/ttyUSB1); both must be connected at once, and the board must not be power-cycled between program and run.