RTL Design Sherpa

RAPIDS Beats DMA Engine Performance Characterization Report

July 15, 2026

Table of Contents

Readme	5

RAPIDS Beats DMA — Performance Characterization (Genesys 2, 8 channels)	5

Characterization knobs	6

1. Headline	7

2. How it is measured (the observation hooks)	8

3. Single-axis sweep — transfer size	8

3.1 Channel count — per-channel independence	11

4. What we learned	11

5. Between-run state (harness workaround + open RTL defect)	12

6. Other limitations	13

7. The STREAM knob set, measured by the interface observers (v1.2)	13

7.0 The regression this campaign found first	14

7.1 Headline: line rate on all four interfaces	14

7.2 Transfer size (phase C): the amortization knee	15

7.3 Descriptors x channels (phase A): the STREAM matrix	18

7.4 Burst length (phase B): the transaction-size knee	19

7.5 Memory latency (phase E): the in-flight window	20

7.6 What the observers add over the bare meters	22

Appendix: data files & reproduce	22

List of Figures

No figures in this document.

List of Tables

Table 1: Characterization knobs — the sweep axes	6

Table 2: Headline — 8-channel line-rate at 256 KB/channel (Genesys 2)	7

Table 3: Table — engaged utilization (%) and effective bandwidth (GB/s) vs transfer size (8 ch)	10

Table 4: Figure — engaged utilization vs active channel count (256 KB/ch).	11

Table 5: Table — the wedge is stale scheduler state, not an in-flight fabric cycle	12

Table 6: Table 7.1 – observer engaged utilization vs active channels (phase D)	14

Table 7: Table 7.2 – size sweep at 8 channels; latencies are observer-histogram means in aclk cycles	15

Table 8: Table 7.3 – AXI4-wr (sink) / AXIS-out (source) utilization over the descriptor x channel matrix	18

Table 9: Table 7.4 – burst-length sweep; latencies in aclk cycles	19

Table 10: Table 7.5 – latency sweep (v1.3 rerun, HIST_MAX_OUTSTANDING = 32, no sample loss on any row); the latency columns are log2-histogram means, so they sit on bin midpoints	20

# Readme

Generated: 2026-09-27

# RAPIDS Beats DMA — Performance Characterization (Genesys 2, 8 channels)

v1.3 (2026-09-27). Latency sweep (7.5) re-measured on a bitstream whose AXI observer keeps 32 timestamps per channel instead of 8: no histogram sample loss on any row, latency columns replaced, caveat retired (rapids TASK-012). Every utilization and bandwidth number is unchanged from v1.2.

v1.2 (2026-09-27). Section 7 adds the STREAM five-knob characterization measured by the shared interface observers (axi4_intf_master_observer on the AXI4 masters, axis4_intf_observer on both AXIS links) instead of the harness’s bare meters, on a bitstream that also fixes a source-path regression the campaign found (rapids BUG-003 / stream BUG-011). Sections 1-6 are the v1.1 bare-meter record and stand as measured.

Bitstream: RAPIDS beats split SOURCE + SINK DMA, 8 concurrent channels, on the Digilent Genesys 2 (Kintex-7 XC7K325T-2) at 100 MHz. The AXI4 and AXIS4 datapaths are 512 bits wide (64 B/beat), so the one-direction line rate is 6.40 GB/s (64 B × 100 MHz). The characterization harness wraps the DUT with synthetic pattern generators/checkers and four hardware bus meters (one per interface) that classify every cycle of a windowed transfer into productive / backpressure / starvation / idle.

Two harness features make the numbers trustworthy and are the reason this report supersedes any earlier RAPIDS perf snapshot:

- Atomic launch (stage-all-then-GO). Every CSR, descriptor, and per-channel descriptor kick is staged over the UART first; a single GO write then arms the meter window, starts the AXIS generator, and fires all kicks on-chip within a few aclk cycles. No UART latency contaminates the measured window.

- Deterministic window close. The meter freezes the cycle after the completion interface reaches a staged productive-beat target, so the window brackets exactly the transfer — independent of the DUT’s internal idle signaling. Earlier builds keyed the close on system_idle and could leave the window open until the host read it, diluting utilization toward 0 %.

Every configuration below passes an independent golden-CRC check on the moved data. Source of truth: the on-chip bus meters, read over CSR.

Metric note. The headline efficiency is engaged utilization — productive / (productive + backpressure + starvation), i.e. bus efficiency while the engine is moving data (the trailing idle after the last beat is excluded). Effective bandwidth is engaged_util × 6.40 GB/s. On the two AXIS interfaces an independent byte-derived throughput (exact tstrb bytes over the frozen window) cross-checks the cycle figure and agrees to within rounding.

## Characterization knobs

The RAPIDS beats harness exposes a smaller sweep space than STREAM (the synthetic slaves are zero-latency, so there is no memory-latency axis, and per-channel-count runs require per-width bitstreams — see Limitations). The axes exercised here:

Characterization knobs — the sweep axes

| # | Knob | What it is | Set by | Values in this report |
| --- | --- | --- | --- | --- |
| 1 | Transfer size (beats/channel) | beats moved per channel per descriptor — the amortization axis | descriptor length / GEN_NBEATS | 256 B (4 beats) … 256 KB (4096 beats) |
| 2 | Path | SOURCE (mem-read → AXIS-out) vs SINK (AXIS-in → mem-write) | which half is kicked | both, every config |
| 3 | Interface | the four monitored buses | — | AXIS-in, AXI4-wr, AXI4-rd, AXIS-out |
| 4 | Channels | concurrent DMA channels | build generic | 8 (full width) |

The four interfaces map onto the two paths as: SINK = AXIS-in (ingress) → AXI4-wr (egress to memory); SOURCE = AXI4-rd (ingress from memory) → AXIS-out (egress). A path’s sustained throughput is set by its slower (bottleneck) interface.

## 1. Headline

At a 256 KB/channel transfer across all 8 channels, every interface runs at 99.8–100 % of line rate — the engine is gapless on both the memory bus and the network bus, in both directions, simultaneously.

Headline — 8-channel line-rate at 256 KB/channel (Genesys 2)

| Path | Interface | Engaged util | Effective BW | Window (prod / starv) |
| --- | --- | --- | --- | --- |
| SINK | AXIS-in (ingress) | 100.0 % | 6.40 GB/s | 32737 / 0 |
| SINK | AXI4-wr (egress) | 100.0 % | 6.40 GB/s | 32768 / 13 |
| SOURCE | AXI4-rd (ingress) | 99.8 % | 6.39 GB/s | 32768 / 50 |
| SOURCE | AXIS-out (egress) | 99.8 % | 6.39 GB/s | 32768 / 50 |

prod = 32768 = 8 channels × 4096 beats on the AXI4 side: every expected beat is accounted for, and the non-productive residue is a handful of cycles of one-time fill latency.

Figure — all four interfaces at the largest transfer sit on the 100 % line.

## 2. How it is measured (the observation hooks)

Each interface has a dedicated meter instantiated in the harness:

- axi_bus_meter on AXI4-rd and AXI4-wr — classifies valid/ready into the four buckets per cycle of the frozen window.

- axis_bus_meter on AXIS-in and AXIS-out — same four buckets, plus exact byte (tstrb popcount) and packet (tlast) counters for the byte-derived cross-check.

The window is armed by GO, opens when the active path goes busy, and freezes deterministically when the completion interface’s productive-beat count reaches the staged target (CSR_OBS_TARGET = channels × beats). Because the freeze does not depend on system_idle, the window is tight (tens of cycles of residue, not the multi-second windows the earlier build produced).

## 3. Single-axis sweep — transfer size

Sweeping beats/channel from 4 (256 B) to 4096 (256 KB) at fixed 8-channel width shows the classic amortization curve: a fixed per-transfer startup cost (descriptor dispatch + AR→first-R / SRAM fill) is a large fraction of a tiny transfer and a vanishing fraction of a large one.

Figure — engaged utilization vs transfer size, all four interfaces.

Figure — effective per-direction bandwidth vs transfer size (line rate 6.40 GB/s).

Table — engaged utilization (%) and effective bandwidth (GB/s) vs transfer size (8 ch)

| Transfer / ch | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out | wr BW | sout BW |
| --- | --- | --- | --- | --- | --- | --- |
| 256 B (4 b) | 0.6 % | 19.5 % | 19.0 % | 19.0 % | 1.25 | 1.22 |
| 1 KB (16 b) | 55.1 % | 72.7 % | 71.1 % | 71.1 % | 4.65 | 4.55 |
| 4 KB (64 b) | 91.6 % | 97.5 % | 92.6 % | 92.6 % | 6.24 | 5.93 |
| 16 KB (256 b) | 97.6 % | 99.1 % | 97.6 % | 97.6 % | 6.34 | 6.25 |
| 64 KB (1024 b) | 99.5 % | 99.9 % | 99.4 % | 99.4 % | 6.40 | 6.36 |
| 256 KB (4096 b) | 99.9 % | 100.0 % | 99.8 % | 99.8 % | 6.40 | 6.39 |

By 4 KB/channel every interface is already above 91 %, and by 64 KB the whole engine is within 0.6 % of line rate. All four curves are the same monotonic amortization of a single fixed per-transfer startup cost (descriptor dispatch + AR→first-R / SRAM fill); there is no steady-state bubble.

Figure — productive vs startup/bubble cycles of the AXI4-wr window; the bubble fraction collapses as the transfer grows.

### 3.1 Channel count — per-channel independence

Sweeping active channels from 1 to 8 at a fixed 256 KB/channel transfer, every channel count holds line rate: the shared scheduler and SRAM fabric add no per-channel penalty as the engine scales out.

utilization vs channel count

Figure — engaged utilization vs active channel count (256 KB/ch).

| Channels | AXI4-wr | AXIS-out |
| --- | --- | --- |
| 1 | 99.9 % | 99.8 % |
| 2 | 99.8 % | 97.2 % |
| 4 | 99.9 % | 96.6 % |
| 8 | 100.0 % | 99.8 % |

: Table — utilization vs channel count (256 KB/ch)

## 4. What we learned

- The datapath is gapless. At realistic transfer sizes (≥ 64 KB/ch) all four interfaces sit at 99.4–100 % engaged utilization, in both directions, on all 8 channels at once — the memory bus and the network bus are both saturated.

- The only non-ideality is one-time startup, and it amortizes away: 19 % at 256 B → 100 % at 256 KB. There is no steady-state bubble, no per-beat gap, and no back-pressure or starvation once data is flowing.

- Per-channel independence. 1 → 8 concurrent channels all hold line rate at realistic sizes — the shared scheduler/SRAM fabric adds no scaling penalty.

- Invariance is the result. Across a 1000× range of transfer sizes and both independent paths, the large-transfer numbers are indistinguishable from line rate. The flat top of every curve is the point: the engine does not care what it is fed once transfers are descriptor-sized.

## 5. Between-run state (harness workaround + open RTL defect)

Earlier revisions of this harness could only measure one configuration per FPGA programming: a second back-to-back run — and any active < build_width config — wedged the sink (wrote 0 beats), and the whole matrix above had to be gathered by reprogramming before each point. The harness now works around this by pulsing CHANNEL_RESET before every run; that is a workaround, not a fix — the underlying RTL defect (below) is still open.

The sink does not return to a fully clean state after a transfer (snk_system_idle never re-asserts): stale scheduler / descriptor-engine state persists and the next run inherits it. A discriminating board experiment isolated the mechanism:

Table — the wedge is stale scheduler state, not an in-flight fabric cycle

| Between-run action | Back-to-back result |
| --- | --- |
| none (baseline) | 1 / 4 — wedges on run 2 |
| +0.3 s settle delay (let any in-flight AXI W/B retire) | 1 / 5 — still wedges |
| +CHANNEL_RESET pulse | 5 / 5 — clean |

The 0.3 s settle (30 M cycles ≫ any outstanding-B retirement) not helping rules out a stuck datapath/fabric cycle; the existing CHANNEL_RESET.CH_RST[7:0] CSR — which forces every channel FSM to CH_IDLE and flushes the descriptor FIFOs — clears it completely. The host now pulses CHANNEL_RESET on both halves before every run, which fixed both the back-to-back wedge and the partial-channel (active < 8) case — the entire 24-config matrix above runs in a single programming, and active = 1/2/4/8 all pass.

Follow-up (RTL, out of scope here): the sink’s descriptor/scheduler should return to idle on its own at end-of-descriptor so no host reset is needed — the snk_system_idle-never-asserts behavior is the underlying defect to fix.

## 6. Other limitations

- No memory-latency axis (v1.1). Superseded in v1.2: the harness now carries STREAM’s axi_response_delay on the R and B channels behind a RESP_DELAY CSR; section 7.5 is that sweep.

## 7. The STREAM knob set, measured by the interface observers (v1.2)

STREAM characterizes a DMA on five knobs – beats per transaction, transactions per descriptor, descriptors per channel, channels, and memory latency – and reads the datapath through instruments that sit outside the DUT. This section does the same on RAPIDS beats, with two changes to the v1.1 setup:

- The instruments are the shared observers. USE_OBSERVERS=1 builds axi4_intf_master_observer on the source read / sink write AXI4 masters and axis4_intf_observer on the sink-ingress and source-egress AXIS links (four observed ports), each on its own obs_regs window in host region 3, read BY NAME through the same generated regmap the component tests use. Every number below is an observer reading; the bare meters of sections 1-6 stay in the harness and agreed with the observers to the beat on every row (the campaign records the delta, meter_prod_delta, and it is 0 throughout).

- Two knobs the v1.1 host could not turn. Burst length (AXI_XFER_CONFIG.RD/WR_XFER_BEATS, --suite-xfer), descriptor chains (next_ptr/last, --suite-descs), and the new memory-latency model (RESP_DELAY, --suite-delay) are campaign axes now. Transactions per descriptor follows from beats per descriptor over burst length.

Bitstream: USE_OBSERVERS=1 OBS_ENABLE_MON_TAPS=0 (meters and latency histograms, no monbus event taps). v1.2 rows (7.1-7.4) are from bitstream v3: post-route WNS +1.317 ns at 100 MHz, 65.5 BRAM tiles (the 512-deep R-channel delay queue is in block RAM). The 7.5 latency sweep is from bitstream v4, the same build with the AXI observer’s timestamp FIFO at HIST_MAX_OUTSTANDING = 32: WNS +0.224 ns, 73962 LUTs, 65.5 BRAM tiles. All 67 configurations pass the golden CRC on both paths: A 20/20, B 7/7, C+D 28/28, E 12/12.

### 7.0 The regression this campaign found first

The first observer run failed every SOURCE row at >= 1024 beats (beat_total=1020 of 1024) and read the source at ~50 % where v1.1 had 96-100 %. The bare bitstream failed identically, the sim reproduced it, and a bisect over the rapids RTL tree (REPO_ROOT overlays, never a checkout) landed on bdf4e0dff, the commit that replaced rapids’ SRAM controller with STREAM’s. Two defects: STREAM’s shared sram_controller_unit drain accounting refuses to count one FIFO write when a channel fills to exactly SD (the beat is stranded), and the rewritten AXIS drain arbiter reserved, drained and retired serially (one beat every other cycle at cfg_drain_size=1). Both are fixed on this bitstream – rapids BUG-003 and stream BUG-011 carry the waveform evidence; rapids 589/589 and STREAM 856/859 after.

### 7.1 Headline: line rate on all four interfaces

At 8 channels x 256 KB the four observers read 99.4 / 100.0 / 100.0 / 100.0 % (AXIS-in, AXI4-wr, AXI4-rd, AXIS-out), i.e. 6.36 / 6.40 / 6.40 / 6.40 GB/s against the 6.40 GB/s one-direction line rate (64 B x 100 MHz); the AXIS byte-derived cross-check gives 6.36 and 6.40 GB/s. Channel scaling is flat:

Table 7.1 – observer engaged utilization vs active channels (phase D)

| Channels (4096 beats/ch) | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out |
| --- | --- | --- | --- | --- |
| 1 | 98.6 % | 99.8 % | 99.7 % | 99.7 % |
| 2 | 98.9 % | 99.8 % | 99.8 % | 99.8 % |
| 4 | 99.2 % | 99.9 % | 99.9 % | 99.9 % |
| 8 | 99.4 % | 100.0 % | 100.0 % | 100.0 % |

Figure 7.1 – observer utilization vs channel count at 256 KB/channel.

### 7.2 Transfer size (phase C): the amortization knee

Table 7.2 – size sweep at 8 channels; latencies are observer-histogram means in aclk cycles

| beats/ch | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out | wr GB/s | rd GB/s | rd bursts | AR->RLAST | AW->B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.8 % | 38.1 % | 28.6 % | 28.6 % | 2.44 | 1.83 | 8 | 6 | 6 |
| 4 | 14.0 % | 84.2 % | 71.1 % | 71.1 % | 5.39 | 4.55 | 8 | 12 | 12 |
| 16 | 39.4 % | 95.5 % | 90.8 % | 90.8 % | 6.11 | 5.81 | 16 | 23 | 23 |
| 64 | 72.0 % | 98.5 % | 97.5 % | 97.5 % | 6.30 | 6.24 | 64 | 22 | 23 |
| 256 | 91.0 % | 99.4 % | 99.4 % | 99.4 % | 6.36 | 6.36 | 232 | 24 | 24 |
| 1024 | 97.6 % | 99.8 % | 99.8 % | 99.8 % | 6.39 | 6.39 | 912 | 24 | 24 |
| 4096 | 99.4 % | 100.0 % | 100.0 % | 100.0 % | 6.40 | 6.40 | 3648 | 24 | 24 |

The three DMA-side interfaces are above 90 % from 16 beats (1 KB) per channel and within 0.6 % of line rate from 1024. The AXIS-in column lags because its window is the sink-INGRESS window (armed at GO, before the sink’s write side is busy), so it carries the generator start-up that the other three windows exclude; at 256 KB that cost has amortized to 0.6 %. rd bursts confirms the 9-beat burst shape (AxLEN 8) from 64 beats up. AR->RLAST settles at 24 cycles: the pattern slave’s fixed response plus a 9-beat burst.

Figure 7.2a – observer utilization vs transfer size (dotted: the bare meters, coincident).

Figure 7.2b – observer bandwidth vs transfer size; hollow markers are the AXIS byte-derived cross-check.

Figure 7.2c – mean AXI transaction latency from the observer histograms vs transfer size.

### 7.3 Descriptors x channels (phase A): the STREAM matrix

{1, 2, 4, 8, 16} descriptors per channel x {1, 2, 4, 8} channels, 1024 beats (64 KB) per descriptor, chained through next_ptr and closed with last:

Table 7.3 – AXI4-wr (sink) / AXIS-out (source) utilization over the descriptor x channel matrix

| descs/ch | 1 ch wr / sout | 2 ch | 4 ch | 8 ch |
| --- | --- | --- | --- | --- |
| 1 | 99.3 / 98.7 % | 99.7 / 99.4 % | 99.6 / 99.7 % | 99.8 / 99.8 % |
| 2 | 98.9 / 99.4 % | 99.0 / 99.7 % | 99.1 / 99.8 % | 99.1 / 99.9 % |
| 4 | 98.7 / 99.7 % | 98.7 / 99.8 % | 98.8 / 99.9 % | 98.8 / 100.0 % |
| 8 | 98.6 / 99.8 % | 98.6 / 99.9 % | 98.6 / 100.0 % | 98.6 / 100.0 % |
| 16 | 98.5 / 99.9 % | 98.5 / 100.0 % | 98.5 / 100.0 % | 98.5 / 100.0 % |

Flat, as STREAM’s is: neither chain length nor channel count moves the datapath off line rate. The one visible structure is the sink write side settling 1.5 % below the source as chains lengthen – the per-descriptor dispatch on the write engine costs a fixed handful of cycles per descriptor boundary, which the source egress does not pay.

Figure 7.3 – the descriptor x channel matrix from the observers.

### 7.4 Burst length (phase B): the transaction-size knee

AXI_XFER_CONFIG AxLEN {0, 1, 3, 7, 15, 31, 63} = {1 .. 64}-beat bursts at 8 channels x 1024 beats:

Table 7.4 – burst-length sweep; latencies in aclk cycles

| beats/burst | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out | AR->RLAST | AW->B |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 24.0 % | 23.2 % | 91.3 % | 91.3 % | 6 | 6 |
| 2 | 43.6 % | 42.4 % | 99.8 % | 99.8 % | 12 | 12 |
| 4 | 99.6 % | 99.0 % | 99.8 % | 99.8 % | 12 | 12 |
| 8 | 100.0 % | 99.7 % | 99.8 % | 99.8 % | 24 | 24 |
| 16 | 100.0 % | 99.9 % | 99.8 % | 99.8 % | 48 | 48 |
| 32 | 100.0 % | 99.9 % | 99.8 % | 99.8 % | 96 | 94 |
| 64 | 99.7 % | 99.1 % | 99.8 % | 99.8 % | 191 | 180 |

The knee is at 4 beats: below it the SINK path collapses (23 % at single-beat bursts, 42 % at 2) while the SOURCE path holds 91-100 %. That asymmetry is the two engines’ outstanding depth – the read engine keeps enough ARs in flight to hide a one-beat burst’s round trip, the write engine cannot, and at one beat per AW the sink is AW-issue bound. STREAM’s knee sits at the same place (~3 beats). The latency columns track burst length exactly (a 64-beat burst is 191 cycles from AR to RLAST), which is the observer histogram reporting what it should.

Figure 7.4 – utilization vs burst length; the sink write path needs >= 4-beat bursts.

### 7.5 Memory latency (phase E): the in-flight window

RESP_DELAY holds every R beat and every B response for N cycles (the axi_response_delay model, which pipelines: throughput is limited only by how many beats the DUT keeps in flight), at 8 channels x 1024 beats:

Table 7.5 – latency sweep (v1.3 rerun, HIST_MAX_OUTSTANDING = 32, no sample loss on any row); the latency columns are log2-histogram means, so they sit on bin midpoints

| delay (cyc) | AXIS-in | AXI4-wr | AXI4-rd | AXIS-out | rd GB/s | wr GB/s | AR->first R | AR->RLAST | AW->B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 100.0 % | 99.8 % | 99.8 % | 99.8 % | 6.39 | 6.39 | 24 | 24 | 24 |
| 8 | 100.0 % | 99.4 % | 99.8 % | 99.8 % | 6.38 | 6.36 | 24 | 48 | 47 |
| 16 | 98.8 % | 98.2 % | 99.7 % | 99.7 % | 6.38 | 6.28 | 48 | 48 | 48 |
| 32 | 93.6 % | 93.0 % | 99.5 % | 99.5 % | 6.37 | 5.96 | 48 | 48 | 48 |
| 48 | 91.4 % | 91.0 % | 98.7 % | 98.7 % | 6.32 | 5.82 | 95 | 96 | 94 |
| 64 | 89.7 % | 89.2 % | 97.6 % | 97.6 % | 6.25 | 5.71 | 96 | 96 | 96 |
| 96 | 78.9 % | 78.5 % | 95.0 % | 95.0 % | 6.08 | 5.02 | 96 | 96 | 96 |
| 128 | 67.5 % | 67.2 % | 88.8 % | 88.8 % | 5.68 | 4.30 | 192 | 192 | 192 |
| 192 | 60.8 % | 60.5 % | 71.5 % | 71.5 % | 4.58 | 3.87 | 192 | 192 | 192 |
| 256 | 48.0 % | 47.7 % | 57.4 % | 57.4 % | 3.67 | 3.05 | 384 | 384 | 384 |
| 384 | 33.8 % | 33.5 % | 40.3 % | 40.3 % | 2.58 | 2.14 | 384 | 384 | 384 |
| 512 | 26.1 % | 25.8 % | 31.1 % | 31.1 % | 1.99 | 1.65 | 768 | 768 | 768 |

Little’s law makes the knee readable directly: sustained beats/cycle x latency = beats in flight. The SOURCE read path holds >= 97.6 % to 64 cycles and >= 95 % to 96, then falls as 0.888 x 128 = 114, 0.715 x 192 = 137, 0.574 x 256 = 147, 0.311 x 512 = 159 – an asymptote of ~160 beats in flight across 8 channels, ~20 per channel, i.e. about two 9-beat bursts per channel. The SINK write path knees earlier (93 % at 32, 89 % at 64) and asymptotes lower (0.258 x 512 = 132, ~16 beats per channel): the write engine’s window on the B response is one to two bursts. STREAM’s window on the same knobs is ~128 beats per channel (8 outstanding x 16-beat bursts) and its knee sits at 96-112 cycles; RAPIDS beats at the shipped 9-beat bursts is window-bound roughly two to three times earlier. Widening the window – larger AxLEN, or more outstanding per channel – is the lever, and section 7.4 shows the DUT already runs 32- and 64-beat bursts at line rate.

Two notes from the instruments themselves. First, the latency columns are histogram means over log2 bins (bin b holds [2^b, 2^(b+1)), reported at its midpoint 1.5 x 2^b), and on this sweep every transaction of a row lands in one bin: the measured round trip is the injected delay plus roughly 24 cycles of DUT-plus-model overhead, so 0 lands in [16,32), 8-32 in [32,64), 48-96 in [64,128), 128-192 in [128,256), 256-384 in [256,512) and 512 in [512,1024) – the 24 / 48 / 96 / 192 / 384 / 768 staircase in the table is the histogram’s resolution, not a step in the DUT. Second, the v1.2 issue of this report carried OBS_STICKY.HIST_SAMPLE_LOST from 48 cycles up: the observer’s timestamp FIFO was sized HIST_MAX_OUTSTANDING = 8 per channel and the source read engine keeps more than that in flight once the delay exceeds one burst, so those v1.2 latency means came from a subset of transactions (and read low: 170, 264, 320, 530 where the full population sits at 192, 384, 384, 768). rapids TASK-012 raised the FIFO to 32 on the harness’s u_obs_axi (bitstream v4, post-route WNS +0.224 ns, still positive), the sweep was re-run, and OBS_STICKY reads 0 on all 12 rows for both observers. The utilization and bandwidth columns are identical between the two runs, which is what the design of the meters predicts: sample loss only ever touched the histograms.

Figure 7.5 – utilization vs injected memory latency, all four observers.

### 7.6 What the observers add over the bare meters

Same utilization numbers (to the beat), plus what a meter cannot give: burst counts, the AR->first-R / AR->RLAST / AW->B latency distributions, exact AXIS bytes and packets per port with per-tid attribution, the tap’s own packet count as a cross-check, and a sticky bit that says when a number undercounts. All of it through one register map shared with STREAM’s harness, so the host code and the report generator are the same on both DMAs.

## Appendix: data files & reproduce

| File | Contents |
| --- | --- |
| perf/json/genesys_obs_{A,B,C,E}.json | v1.2 observer campaign: descriptors x channels, burst length, size x channels, latency |
| perf/json/genesys_full_matrix.json | channel × size matrix (v1.1, bare meters) |
| perf/json/genesys_8ch_*.json | earlier back-to-back runs (show the pre-fix wedge) |
| perf/plots/*.png | figures above (flows-rapids-beats/host/plot_char_reports.py) |

# full channel x size matrix in ONE programming (CHANNEL_RESET per run):
source env_python
python3 projects/fpga-systems/Genesys2/rapids_beats/flows-rapids-beats/host/run_characterization.py \
    --port /dev/ttyUSB1 --channels 8 --suite \
    --suite-channels 1,2,4,8 --suite-beats 4,16,64,256,1024,4096 --suite-bp off \
    --results projects/fpga-systems/Genesys2/rapids_beats/reports/perf/json/genesys_full_matrix.json

# figures:
python3 projects/fpga-systems/Genesys2/rapids_beats/flows-rapids-beats/host/plot_char_reports.py \
    --size   projects/fpga-systems/Genesys2/rapids_beats/reports/perf/json/genesys_full_matrix.json \
    --outdir projects/fpga-systems/Genesys2/rapids_beats/reports/perf/plots

# this report (DOCX + PDF, house style):
cd projects/fpga-systems/Genesys2/rapids_beats/reports && ./generate_reports_pdf.sh --rev 1.0

# v1.2 observer campaign (USE_OBSERVERS=1 bitstream: make bitstream BOARD=genesys2 USE_OBSERVERS=1):
H=projects/fpga-systems/Genesys2/rapids_beats/flows-rapids-beats/host/run_characterization.py
J=projects/fpga-systems/Genesys2/rapids_beats/reports/perf/json
python3 $H --port /dev/ttyUSB0 --channels 8 --suite --suite-bp off --suite-seeds default \
    --suite-channels 1,2,4,8 --suite-beats 1024 --suite-descs 1,2,4,8,16 --results $J/genesys_obs_A.json
python3 $H --port /dev/ttyUSB0 --channels 8 --suite --suite-bp off --suite-seeds default \
    --suite-channels 8 --suite-beats 1024 --suite-xfer 0,1,3,7,15,31,63 --results $J/genesys_obs_B.json
python3 $H --port /dev/ttyUSB0 --channels 8 --suite --suite-bp off --suite-seeds default \
    --suite-channels 1,2,4,8 --suite-beats 1,4,16,64,256,1024,4096 --results $J/genesys_obs_C.json
python3 $H --port /dev/ttyUSB0 --channels 8 --suite --suite-bp off --suite-seeds default \
    --suite-channels 8 --suite-beats 1024 --suite-delay 0,8,16,32,48,64,96,128,192,256,384,512 --results $J/genesys_obs_E.json
# figures (size/channel plots from C; obs_desc_matrix / obs_xfer_knee / obs_latency_knee from A / B / E):
python3 .../host/plot_char_reports.py --size $J/genesys_obs_C.json --outdir .../reports/perf/plots
cd projects/fpga-systems/Genesys2/rapids_beats/reports && ./generate_reports_pdf.sh --rev 1.3 --only perf

Genesys 2 host link: JTAG on the FT2232 (200300B818A0), UART on the separate FT232R (AU05X8RM, /dev/ttyUSB1); both must be connected at once, and the board must not be power-cycled between program and run.