RTL Design Sherpa

STREAM Characterization DMA Performance Report

June 2026

Table of Contents

Readme	4

STREAM DMA — Performance Characterization (in-core perf-monitor build)	4

Characterization knobs	5

1. Headline	6

2. How it is measured (the observation hooks)	7

3. Single-axis sweeps	8

3.0 Knob 1 — transaction (burst) size · 1 ch, 1 MB, fixed outstanding	8

3.1 Knob 3 — descriptors per channel (1 ch, 1→16 desc, 1 MB each, delay 0)	9

3.2 Knob 4 — channels (1 desc each, 1→8 ch, 1 MB/ch, delay 0)	10

3.3 Knob 2 — transactions per descriptor (transfer size, 1 ch, 1 desc, 8 KB→1 MB, delay 0)	11

4. 2-D surfaces	12

4.1 Channels × descriptors (1 MB, delay 0) — the operating envelope	12

4.2 Channels × memory latency — the key result	14

4.3 Descriptors × memory latency (1 ch)	15

4.4 Where the cycles go — utilization pair + bus-cycle breakdown	16

5. The architecture: Little’s Law on real silicon	18

6. What we learned	19

Appendix: data files & reproduce	20

Addendum A — End-to-end utilization (system framing, not bus bubbles)	22

List of Tables

Table 1: Characterization knobs — the five sweep axes	5

Table 2: Headline — datapath utilization by knob	6

Table 3: Knob 1 — datapath utilization vs transaction (burst) size	8

Table 4: Knob 3 — datapath utilization vs descriptors per channel	9

Table 5: Knob 4 — datapath utilization vs channels	10

Table 6: Knob 2 — datapath utilization vs descriptor size (transactions per descriptor)	11

Table 7: Write-bus cycle breakdown vs memory latency (knob 5)	18

Table 8: Characterization datasets (2026-06-23, board 210292B7D46F)	20

Table 9: End-to-end vs datapath utilization by descriptor size	22

# Readme

Generated: 2026-06-23

# STREAM DMA — Performance Characterization (in-core perf-monitor build)

Bitstream: RFC Stage E in-core datapath perf-monitor build. Two correctness fixes since the 2026-06-21 report move the headline from ~94% to 100%:

- Characterization-harness slave fix. The synthetic read/write slaves (axi4_slave_rd_pattern_gen / axi4_slave_wr_crc_check) were 2-state FSMs that accepted the next transaction only after returning to idle, inserting a 1-cycle dead bubble between every burst. That ~6% was a slave-model artifact, not a DUT limitation; the slaves now accept the next AR/AW on the last beat of the current burst (gapless back-to-back).

- Data-owed meter gating. The in-core bubble buckets now count only while data is owed and flowing (first valid&&ready → outstanding drains), so the one-time AR→first-R fill latency and trailing drain are no longer mis-counted as starvation.

FST analysis confirms the datapath is 100% gapless between the first and last beat (4096 productive cycles in a 4096-cycle window; starvation = 0). The monitor footprint was trimmed to perf-only (and the per-channel completion/error MonBus emitters gated off on the FPGA build) to fit the xc7a100t; that relieved congestion enough to close 100 MHz timing at +0.031 ns WNS (was a marginal −0.097 ns). Every config below passes CRC. Source of truth: the in-core perf monitors read over CSR. Datapath: 128-bit (16 B/beat). One-direction AXI ceiling 1526 MB/s; net-bytes-moved ceiling 763 MB/s (the DMA reads and writes each byte). Date: 2026-06-23 (board 210292B7D46F). Methodology: ../../DMA_UTILIZATION_MEASUREMENT.md.

Metric note. The headline efficiency is datapath utilization (productive beats / data-owed window, from the on-chip perf monitors) — bus efficiency while the engine is moving data. End-to-end utilization (productive beats / kick→done cycles) is a system metric that also counts the one-time per-transfer startup latency; it is not bus bubbles, and is reported only in Addendum A to avoid confusion. bus throughput (mb_moved / FPGA-timer-time) is on-chip and wall-clock-independent; the host throughput_MBps is UART-dominated and shown for transparency only.

## Characterization knobs

The sweep space is five knobs — four nested workload knobs (innermost a single bus transaction, outermost the whole engine) plus the environment knob, memory latency. All byte figures assume the 128-bit datapath — 16 B / beat.

Characterization knobs — the five sweep axes

| # | Knob | What it is | Set by | Nominal values |
| --- | --- | --- | --- | --- |
| 1 | Beats per transfer (transaction size) | one AXI burst — a single read/write transaction on the bus | cfg_axi_rd_xfer_beats / cfg_axi_wr_xfer_beats (ARLEN = beats − 1) | 256 B (16 beats) · 512 B (32) · 1024 B (64) |
| 2 | Transactions per descriptor | bursts that make up one descriptor → descriptor size = transactions × transaction size | descriptor length field | 1 (256 B–1 KB) … 4096 (1 MB @ 256 B) |
| 3 | Descriptors per channel | the descriptor-chain length a channel walks | chained next_ptr | 1, 2, 4, 8, 16 |
| 4 | Channels | concurrent DMA channels | cfg_channel_enable | 1 … 8 |
| 5 | Response delay (memory latency) | modeled per-beat R/B round-trip latency of the backing memory — the environment the engine runs against | cfg_rd/wr_resp_delay_cyc (RESP_DELAY CSR) | 0 … 512 cyc |

Read the workload knobs outside-in: a channel (4) walks its descriptor chain (3); each descriptor is issued as fixed-size transactions (2); each transaction is beats per transfer (1). The datapath bubbles only when the in-flight window can’t cover the memory round trip — outstanding × beats < response delay. That single condition is reachable two ways: shrinking the transaction below ~3 beats (knob 1, §3.0) or raising the latency above the 128-beat window (knob 5, §4). Everywhere else — every real transaction size, descriptor count, and channel count, up to the BDP — the datapath is a flat 100 %; knobs 2–4 only change end-to-end framing (Addendum A).

The nominal individual transaction is 256 B / 512 B / 1024 B (16 / 32 / 64 beats); 1024 B is the spec maximum. Knob 5 models the backing memory: at delay 0 the engine runs at the full AXI ceiling; the cliff begins once latency exceeds the per-channel in-flight window (channels widen that window, §4.2).

## 1. Headline

Across the full 40-config matrix (descriptors ∈ {1,2,4,8,16} × channels ∈ {1..8}, 1 MB/descriptor, 256 B transactions), every configuration passes CRC and reads 100.0 % datapath utilization — R = 99.99–100.00 %, W = 99.94–100.00 % — at ~1525 MB/s bus throughput (the full one-direction AXI ceiling). backpressure = 0, starvation = 0: the datapath is bubble-free.

The prior report’s ~94% was not a STREAM limitation — it was the characterization-harness slave artifact (a 1-cycle per-burst gap) plus measurement-window framing (the one-time fill/drain counted as starvation), both fixed (see Bitstream note). FST analysis proves the steady-state datapath is 100% gapless.

By knob (see Characterization knobs above):

Headline — datapath utilization by knob

| knob | sweep | datapath utilization |
| --- | --- | --- |
| 1 transaction size | 1 KB → 1-beat burst (§3.0) | 100 % from 1 KB down to 64 B; falls only at 1–2 beat bursts (sub-BDP) |
| 2 transactions / descriptor | 8 KB → 1 MB descriptor (§3.3) | flat 100 % (end-to-end amortizes 91.6→99.9 %, Addendum A) |
| 3 descriptors / channel | 1 → 16 desc (§3.1) | flat 100 % — descriptor fetch overlaps the data engines |
| 4 channels | 1 → 8 ch (§3.2) | flat 100 % — shared slave caps bandwidth, not utilization |
| 5 memory latency (response delay) | 0 → 512 cyc (§4) | flat to the 128-cyc BDP per channel, then a Little’s-Law cliff |

Architectural takeaway: the datapath bubbles in exactly one regime — when the in-flight window cannot cover the memory round trip, outstanding × beats < response delay. That is reachable two ways: shrinking the transaction below ~3 beats (knob 1, §3.0) or raising memory latency above the 128-beat window (knob 5, §4) — the same bandwidth-delay-product limit from opposite directions. Across the entire real operating range — 1 KB transactions down to 64 B, at memory latency within the 128-cycle window — the engine holds the bus at 100%. Beyond the BDP throughput follows Little’s Law (AR_MAX_OUTSTANDING × burst_len beats in flight per channel), and added channels push the latency knee out, saturating past ~4 channels at the shared backing memory.

## 2. How it is measured (the observation hooks)

The harness instruments the DMA’s read/write AXI without perturbing it (the same logic now packaged standalone as axi4_dma_observer — see ../../docs/dma_observer_integration_tracker.md):

- axi_bus_meter × 2 (read R-channel, write W-channel): pure valid/ready snoop, classifies every cycle as productive / backpressure / starvation / idle. CSR counters, no SRAM — unbounded run length. Source of the datapath-utilization numbers.

- Harness timer: counts aclk from descriptor kick to write-side done (cycles_total). Bus meters freeze at the same done, so the bucket sums equal the timer window exactly.

- axi4_dma_slaves: LFSR read-source + CRC write-sink — the endpoints the DMA reads/writes, giving per-channel data-correctness CRC.

- axi_response_delay × 2: pipelined per-beat R/B hold, the knob for the memory-latency axis.

## 3. Single-axis sweeps

### 3.0 Knob 1 — transaction (burst) size · 1 ch, 1 MB, fixed outstanding

burst size util

Knob 1 — datapath utilization vs transaction (burst) size

| burst | bytes | R datapath | W datapath | starvation |
| --- | --- | --- | --- | --- |
| 64 beats | 1 KB | 100.0 % | 100.0 % | 0 % |
| 32 | 512 B | 100.0 % | 100.0 % | 0 % |
| 16 | 256 B | 100.0 % | 100.0 % | 0 % |
| 8 | 128 B | 100.0 % | 100.0 % | 0 % |
| 4 | 64 B | 100.0 % | 100.0 % | 0 % |
| 2 | 32 B | 66.9 % | 66.7 % | 33 % |
| 1 | 16 B | 33.5 % | 33.3 % | — |

The individual AXI transaction is sized by cfg_axi_rd_xfer_beats / cfg_axi_wr_xfer_beats. With a fixed outstanding depth the datapath is a flat 100 % — zero bubbles — from the 1 KB spec maximum down to 64 B (4 beats). It falls only when outstanding × beats drops below the AR→first-R fill (the bandwidth-delay product, ~3 beats here): a 2-beat burst sustains 67 %, a single beat 33 %, because the pipeline cannot stay full on bursts that small. This is the only place the datapath bubbles, and it is below any realistic transaction size.

### 3.1 Knob 3 — descriptors per channel (1 ch, 1→16 desc, 1 MB each, delay 0)

desc sweep

Knob 3 — datapath utilization vs descriptors per channel

| descriptors | total moved | bus MB/s | datapath util |
| --- | --- | --- | --- |
| 1 | 1 MB | 1525 | 100.0 % |
| 2 | 2 MB | 1525 | 100.0 % |
| 4 | 4 MB | 1525 | 100.0 % |
| 8 | 8 MB | 1525 | 100.0 % |
| 16 | 16 MB | 1525 | 100.0 % |

Flat at 100 %. The descriptor engine fetches the next descriptor concurrently with the data engines draining the current one (separate desc master), so there is no inter-descriptor bubble — chain length is free.

### 3.2 Knob 4 — channels (1 desc each, 1→8 ch, 1 MB/ch, delay 0)

channel sweep

Knob 4 — datapath utilization vs channels

| channels | total moved | bus MB/s | datapath util | starvation |
| --- | --- | --- | --- | --- |
| 1 | 1 MB | 1525 | 100.0 % | 0 % |
| 2 | 2 MB | 1525 | 100.0 % | 0 % |
| 4 | 4 MB | 1526 | 100.0 % | 0 % |
| 8 | 8 MB | 1526 | 100.0 % | 0 % |

All channel counts hold 100 % datapath at the same ~1525 MB/s. The shared read-source / write-sink is the bandwidth ceiling — adding channels splits that bandwidth across more streams rather than scaling it. Arbitration overhead 1→8 channels is zero (starvation = 0 — the combinational, bubble-free channel arbiter). Channels buy latency tolerance, not raw bandwidth (§5).

### 3.3 Knob 2 — transactions per descriptor (transfer size, 1 ch, 1 desc, 8 KB→1 MB, delay 0)

size sweep

Knob 2 — datapath utilization vs descriptor size (transactions per descriptor)

| descriptor size | 256 B transactions | bus MB/s | datapath util |
| --- | --- | --- | --- |
| 8 KB | 32 | 1398 | 99.6 % |
| 16 KB | 64 | 1459 | 99.8 % |
| 32 KB | 128 | 1492 | 99.9 % |
| 64 KB | 256 | 1509 | 100.0 % |
| 128 KB | 512 | 1517 | 100.0 % |
| 256 KB | 1024 | 1522 | 100.0 % |
| 512 KB | 2048 | 1524 | 100.0 % |
| 1 MB | 4096 | 1525 | 100.0 % |

The descriptor is issued as 32 → 4096 fixed 256 B transactions. Datapath utilization is 100 % — the bus is gapless across every burst boundary (including the 4 KB AXI boundaries a >4 KB descriptor must split on); the fractional dip at 8–32 KB is the gated window’s fixed ~2-cycle edge over a still-modest beat count, not bubbles. The end-to-end number climbs 91.6 → 99.9 % across this range as the one-time fill/drain amortizes — a framing effect, reported in Addendum A.

## 4. 2-D surfaces

### 4.1 Channels × descriptors (1 MB, delay 0) — the operating envelope

channels x desc heatmap

channels x desc lines

The entire envelope is flat at 100 % datapath. Neither axis moves efficiency: descriptors add length without bubbles (§3.1), channels share one slave (§3.2). This is the expected behaviour for a back-to-back-saturated engine in front of a single backing memory.

### 4.2 Channels × memory latency — the key result

channels x delay heatmap

channels x delay lines

At delay 0 every channel count peaks at the full ~1525 MB/s. Bus throughput holds flat until the memory round-trip latency exceeds the engine’s in-flight window (128 cycles for 1 channel = AR_MAX_OUTSTANDING × burst_len beats), then cliffs linearly per Little’s Law (BW ≈ window / L × peak). Each added channel contributes its own outstanding queue, pushing the knee out — 2 ch hold to ~256 cyc, 4 ch to ~512 — but 8 ch ≈ 4 ch: past ~4 channels the shared read-source / write-sink caps the aggregate in-flight window, so more channels stop buying latency tolerance. (Exact per-point throughput is in the line plot above.)

### 4.3 Descriptors × memory latency (1 ch)

desc x delay heatmap

desc x delay lines

Every descriptor-count curve overlaps exactly and cliffs at the same 128-cycle knee. Descriptors are sequential within a channel, so they share the single channel’s 128-beat in-flight window — they add transfer length, never latency tolerance. This is the clean complement to §4.2: channels widen the latency window; descriptors do not.

### 4.4 Where the cycles go — utilization pair + bus-cycle breakdown

The methodology (DMA_UTILIZATION_MEASUREMENT.md §5) asks for a pair of numbers — steady-state datapath utilization (§2.1) and end-to-end utilization (§2.3) — with the gap reported as overhead. On this SRAM↔SRAM engine with single-cycle descriptor turnaround the two track within the perf monitor’s resolution, so the overhead band is essentially zero: there is no descriptor- fetch bubble to amortize.

datapath vs end-to-end utilization vs latency

The §3 four-bucket classification on the write (destination) interface is the diagnostic payoff. As memory latency rises past the 128-cycle in-flight window, the lost cycles are entirely starvation — the read side cannot feed the datapath fast enough — while backpressure stays pinned at 0%:

W-bus cycle breakdown vs latency

Write-bus cycle breakdown vs memory latency (knob 5)

| delay (cyc) | productive | backpressure | starvation |
| --- | --- | --- | --- |
| 0–96 | 100% | 0% | 0% |
| 128 | 82% | 0% | 18% |
| 256 | 45% | 0% | 55% |
| 512 | 24% | 0% | 76% |

This is the engine’s signature: it is never destination-bound (the SRAM sink always keeps up); the only thing that throttles it is read-latency starvation past the outstanding-transaction window — exactly the Little’s-Law behaviour of §5. At delay 0 starvation is now 0% — the harness slave fix removed the per-burst gap that the prior report attributed to “inter-burst arbitration,” so the datapath is fully bubble-free until latency exceeds the in-flight window.

Overhead vs transfer size (§5) — the one place a real datapath/end-to-end gap appears is small transfers, where a fixed ~90-cycle startup is a larger fraction of a short run; it amortizes away by ~256 KB:

datapath vs end-to-end utilization vs size

## 5. The architecture: Little’s Law on real silicon

The cliff is the textbook signature of a multi-outstanding master in front of a pipelined memory:

- Below the knee (L ≤ 128): the in-flight window covers the round trip; the entire latency is paid once as a pipe-fill and the rest streams at line rate. Adding 64 cycles of per-beat latency costs < 0.3 % of a 1 MB run.

- At/above the knee (L ≥ 128): Little’s Law governs — BW ≈ (in_flight_beats / L) × peak, with in_flight = 128 beats and peak ≈ 1525 MB/s. The measured 1-channel cliff in §4.2 tracks (128/L) ×   1525 to within a few percent at every delay point, the fit tightening as L grows.

The window is AR_MAX_OUTSTANDING (8) × burst_len (16) = 128 beats per channel; channels scale it linearly until the shared slave caps the aggregate (≈ 4 × in this harness). This is the same BDP that bounds the per-transaction sweep in §3.0 — a burst shorter than latency / outstanding cannot keep the window full.

## 6. What we learned

- The datapath is 100 % utilized — bubble-free (starvation = 0, backpressure = 0) across every transaction size ≥ 64 B, descriptor count, and channel count at the full ~1525 MB/s ceiling. The prior ~94 % was a characterization-harness slave artifact plus measurement framing, now fixed.

- Only sub-BDP transactions bubble the bus. A burst must carry at least AR→first-R latency / outstanding (~3 beats here) to keep the pipeline full; 1–2 beat bursts are the sole place utilization drops — far below any realistic transaction (knob 1, §3.0).

- Descriptors are free. Concurrent prefetch over the dedicated descriptor master means chain length adds no per-descriptor bubble.

- Channels share bandwidth, not multiply it — in this harness, where all channels hit one read-source and one write-sink. Independent per-channel backing memory would scale N×; that’s a follow-on.

- Channels buy latency tolerance. The 128-cycle hide-window scales with channel count to the shared-slave saturation point (~4 ch); descriptors, sharing a channel’s window, buy none.

- Size matters only end-to-end. Datapath stays 100 %; only the system end-to-end metric sees the one-time fill amortize (Addendum A) — that is latency, not bus bubbles.

- The fixes cost nothing — they gained. Trimming the monitors to perf-only closed 100 MHz timing at +0.031 ns (was −0.097 ns), and the data, now correctly gated, reads a clean 100 %.

## Appendix: data files & reproduce

Characterization datasets (2026-06-23, board 210292B7D46F)

| File | Sweep |
| --- | --- |
| json/matrix_2026-06-23.json | channels × descriptors, 40 configs, 1 MB (knobs 3 × 4) |
| json/burst_{1,2,4,8,16,32,64}beat_2026-06-23.json | transaction (burst) size 1 KB → 1 beat, 1 ch (knob 1, §3.0) |
| json/size_sweep_2026-06-23.json | descriptor size 8 KB → 1 MB, 1 ch 1 desc (knob 2, §3.3) |
| json/chan_x_delay_2026-06-23.json | channels {1,2,4,8} × delay {0..512}, 1 desc |
| json/desc_x_delay_2026-06-23.json | desc {1,2,4,8,16} × delay {0..512}, 1 ch |
| json/matrix_2026-06-23.csv | flat CSV of the matrix (column dictionary below) |
| plots/*.png | figures above (host/plot_char_reports.py + burst_size_util.png) |
| old/ | superseded datasets and prior report revisions |

Every record carries the methodology primitives (§2.1 datapath + §2.3 end-to-end utilization, §3 productive/backpressure/starvation/idle buckets, aggregate and per-channel, R and W) under each config’s metrics key. Delay sweeps nest the per-run record under a result key with rd_delay/wr_delay siblings.

cd flows-stream-bridge/host && source $REPO_ROOT/env_python
P=/dev/serial/by-id/usb-Digilent_Digilent_USB_Device_210292B7D46F-if01-port0
D=0,32,64,96,112,128,144,160,192,256,384,512
# NOTE: -o paths are relative to host/; the reports dir is two levels up, so
# use an ABSOLUTE path (or ../../reports/perf) -- "../reports/perf" does NOT
# exist and the run will compute then crash on save.
R=$REPO_ROOT/projects/NexysA7/stream_characterization/reports/perf
J=$R/json   # datasets live in reports/perf/json/ ; plots in reports/perf/plots/
# IMPORTANT: re-program the board (make program) before the matrix / size
# sweeps. The --resp-delays sweep leaves the RESP_DELAY CSR set; a leftover
# value silently degrades a later no-delay run (util drops, high-load configs
# time out). Re-program also recovers a clean state if a sweep crashed.
# matrix (40-config, knobs 3 x 4):
python3 run_characterization.py --port $P --csv fpga_suite.csv -o $J/matrix_2026-06-23.json
# KNOB 1 -- individual transaction (burst) size, 1 ch, 1 MB. XFER_BEATS drives
# cfg_axi_rd/wr_xfer_beats (ARLEN = beats-1); this is the only datapath-bubbling axis:
for b in 64 32 16 8 4 2 1; do XFER_BEATS=$b python3 run_characterization.py \
    --port $P --size 1MB --channels 1 -o $J/burst_${b}beat_2026-06-23.json; done
# KNOB 2 -- descriptor size (transactions per descriptor), 1 ch 1 desc:
python3 run_characterization.py --port $P --csv size_sweep_suite.csv -o $J/size_sweep_2026-06-23.json
# channels x delay (1 desc, 512 KB):
python3 run_characterization.py --port $P --phase 1 --channels 1 2 4 8 --size 512KB \
    --resp-delays $D -o $J/chan_x_delay_2026-06-23.json
# desc x delay (1 ch, 512 KB):
python3 run_characterization.py --port $P --channels 1 --size 512KB \
    --resp-delays $D -o $J/desc_x_delay_2026-06-23.json
# plots (matrix/delay/size); burst plot is host-side matplotlib (burst_size_util.png):
python3 plot_char_reports.py --matrix $J/matrix_2026-06-23.json \
    --chan-delay $J/chan_x_delay_2026-06-23.json \
    --desc-delay $J/desc_x_delay_2026-06-23.json \
    --size $J/size_sweep_2026-06-23.json --outdir $R/plots
# CSV from a matrix JSON (column dictionary in this appendix):
python3 perf_json_to_csv.py $J/matrix_2026-06-23.json --out $J/matrix_2026-06-23.csv

CSV columns (current runner): date,time,config,channels,descriptors,desc_KB, mb_moved,bus_time_s,bus_throughput_MBps,bus_max_one_dir_MBps, bus_max_net_moved_MBps,bus_e2e_util_pct,datapath_{R,W,E2E}_pct, {R,W}_{prod,bp,starv}_pct,dma_time_s,throughput_MBps. Bus-side columns are authoritative; the two host-side columns (dma_time_s, throughput_MBps) are transparency-only. The 16desc_* configs trip trace.overflow (the 2048-beat debug-trace SRAM) — benign; it bounds only the waveform trace, not the bus-meter counters.

## Addendum A — End-to-end utilization (system framing, not bus bubbles)

End-to-end utilization (productive beats / kick→done cycles) is a system metric, not a bus metric. It counts the one-time per-transfer startup — descriptor fetch + AR→first-R pipe-fill — and the trailing drain as if they were non-productive bus cycles. They are latency, not bubbles: the bus is gapless the entire time data is moving (datapath = 100 %). E2E is reported here, apart from the headline, because that distinction is easy to misread.

end-to-end vs datapath utilization vs descriptor size

For a single isolated transfer the end-to-end number rises as the transfer grows and the fixed startup amortizes, while datapath stays pinned at 100 %:

End-to-end vs datapath utilization by descriptor size

| descriptor size | datapath util | end-to-end util |
| --- | --- | --- |
| 8 KB | 99.6 % | 91.6 % |
| 64 KB | 100.0 % | 98.9 % |
| 256 KB | 100.0 % | 99.7 % |
| 1 MB | 100.0 % | 99.9 % |

In back-to-back operation the startup of one transfer overlaps the drain of the previous, so the steady-state system rate equals the datapath rate. This number says nothing about whether the datapath bubbles — it does not (§3.0).