RTL Design Sherpa

STREAM Characterization DMA Performance Report

June 2026

Table of Contents

List of Tables

No tables in this document.

# Readme

Generated: 2026-06-16

# STREAM DMA — Performance Characterization (all-fixes bitstream)

Bitstream: all-timing-fixes build, WNS +0.048 ns @ 100 MHz, Nexys A7 (xc7a100t). Both CAM pipelines, half-beat packing, monbus compression, wrapping trace pointer, pblock_monbus floorplan. Datapath: 128-bit (16 B/beat). One-direction AXI ceiling 1526 MB/s; net-bytes-moved ceiling 763 MB/s (the DMA reads and writes each byte). Date: 2026-06-15/16. Methodology: ../../DMA_UTILIZATION_MEASUREMENT.md.

Metric note. Throughout, the headline efficiency is datapath E2E utilization (productive beats / window, from the on-chip PMU) and bus throughput (mb_moved / FPGA-timer-time). Both are on-chip, UART- and wall-clock-independent. The host wall-clock throughput_MBps is reported only for transparency — it is dominated by UART/poll overhead and is not a bus metric.

## 1. Headline

Across the full 40-config matrix (descriptors ∈ {1,2,4,8,16} × channels ∈ {1..8}, 1 MB/descriptor), every configuration passes CRC and lands in a 94.06–94.12 % datapath-E2E band at 1435–1436 MB/s net bus throughput — about 94 % of the 1526 MB/s one-direction ceiling. The residual ~6 % is steady-state inter-burst arbitration (≈1.06 cycles/beat, reported as starvation), not backpressure (backpressure ≈ 0).

| sweep | knob | result |
| --- | --- | --- |
| descriptor chain (1 ch, 1→16 desc) | descriptors | flat 94.06→94.07 %, 1435.2→1435.3 MB/s |
| multi-channel (1 desc, 1→8 ch) | channels | flat 94.06→94.11 % — shared slave, not per-channel BW scaling |
| transfer size (1 ch, 1 desc, 8 KB→1 MB) | size | 78.8 %→94.0 % as a fixed ~90-cycle startup amortizes |
| memory latency, 1 channel (0→64 cyc) | resp-delay | flat ≥94 % — pipeline fully absorbs |
| memory latency, 1 channel (128→4096 cyc) | resp-delay | linear cliff, BW ≈ 128/L × peak (Little’s Law) |
| memory latency, N channels | resp-delay | cliff position scales with channels, saturating past ~4 |

Architectural takeaway: the engine’s multi-outstanding pipeline hides 128 cycles of memory round-trip latency completely — exactly AR_MAX_OUTSTANDING (8) × burst_len (16) beats in flight. Past that, throughput degrades linearly with latency L per Little’s Law. Adding channels adds independent outstanding queues, so latency tolerance scales with channel count — up to the point the shared read-source / write-sink caps it (~4 channels in this harness).

## 2. How it is measured (the observation hooks)

The harness instruments the DMA’s read/write AXI without perturbing it (the same logic now packaged standalone as axi4_dma_observer — see ../../docs/dma_observer_integration_tracker.md):

- axi_bus_meter × 2 (read R-channel, write W-channel): pure valid/ready snoop, classifies every cycle as productive / backpressure / starvation / idle. CSR counters, no SRAM — unbounded run length. Source of the datapath-utilization numbers.

- Harness timer: counts aclk from descriptor kick to write-side done (cycles_total). Bus meters freeze at the same done, so the bucket sums equal the timer window exactly.

- axi4_dma_slaves: LFSR read-source + CRC write-sink — the endpoints the DMA reads/writes, giving per-channel data-correctness CRC.

- axi_response_delay × 2: pipelined per-beat R/B hold, the knob for the memory-latency axis.

## 3. Single-axis sweeps

### 3.1 Descriptor-chain length (1 ch, 1→16 desc, 1 MB each, delay 0)

desc sweep

| descriptors | total moved | bus MB/s | E2E util |
| --- | --- | --- | --- |
| 1 | 1 MB | 1435.2 | 94.06 % |
| 2 | 2 MB | 1435.3 | 94.06 % |
| 4 | 4 MB | 1435.3 | 94.06 % |
| 8 | 8 MB | 1435.3 | 94.07 % |
| 16 | 16 MB | 1435.3 | 94.07 % |

Flat to the third decimal. The descriptor engine fetches the next descriptor concurrently with the data engines draining the current one, so there is no inter-descriptor bubble — chain length is free.

### 3.2 Channel count (1 desc each, 1→8 ch, 1 MB/ch, delay 0)

channel sweep

| channels | total moved | bus MB/s | E2E util | starvation |
| --- | --- | --- | --- | --- |
| 1 | 1 MB | 1435.2 | 94.06 % | 5.94 % |
| 2 | 2 MB | 1435.7 | 94.09 % | 5.91 % |
| 4 | 4 MB | 1435.9 | 94.10 % | 5.90 % |
| 8 | 8 MB | 1436.0 | 94.11 % | 5.89 % |

All channel counts land at the same ~1435 MB/s. The shared read-source / write-sink is the bandwidth ceiling — adding channels splits that bandwidth across more streams rather than scaling it. Arbitration overhead going 1→8 channels is < 0.1 % (starvation even drops slightly as more channels keep the bus marginally busier). Channels buy latency tolerance, not raw bandwidth (§5).

### 3.3 Transfer size (1 ch, 1 desc, 8 KB→1 MB, delay 0)

size sweep

| size | cycles | bus MB/s | E2E util |
| --- | --- | --- | --- |
| 8 KB | 650 | 1201.9 | 78.8 % |
| 16 KB | 1,194 | 1308.6 | 85.8 % |
| 32 KB | 2,282 | 1369.4 | 89.8 % |
| 64 KB | 4,458 | 1402.0 | 91.9 % |
| 128 KB | 8,810 | 1418.8 | 93.0 % |
| 256 KB | 17,514 | 1427.4 | 93.6 % |
| 512 KB | 34,922 | 1431.8 | 93.8 % |
| 1 MB | 69,738 | 1433.9 | 94.0 % |

A fixed ~90-cycle startup/drain (pipe-fill + first-descriptor fetch) amortizes as size grows: 21 % of an 8 KB run, but < 0.2 % of a 1 MB run. Steady-state efficiency asymptotes to the ~94 % seen everywhere else. At 64 KB the software-visible loss is already under 2 %.

## 4. 2-D surfaces

### 4.1 Channels × descriptors (1 MB, delay 0) — the operating envelope

channels x desc heatmap

channels x desc lines

The entire envelope is flat at 94.1 %. Neither axis moves efficiency: descriptors add length without bubbles (§3.1), channels share one slave (§3.2). This is the expected behaviour for a back-to-back-saturated engine in front of a single backing memory.

### 4.2 Channels × memory latency — the key result

channels x delay heatmap

channels x delay lines

| delay (cyc) | 1 ch | 2 ch | 4 ch | 8 ch |
| --- | --- | --- | --- | --- |
| 0 | 1435 | 1436 | 1436 | 1436 |
| 64 | 1434 | 1435 | 1436 | 1436 |
| 128 | 1258 | 1434 | 1435 | 1436 |
| 256 | 689 | 1377 | 1435 | 1435 |
| 512 | 362 | 724 | 761 | 761 |
| 1024 | 186 | 371 | 381 | 381 |
| 4096 | 47 | 95 | 95 | 95 |

1 channel holds flat until 128 cycles, then cliffs. Each added channel contributes its own outstanding queue, pushing the knee out: 2 channels hold to ~256, 4 channels to ~512. 8 channels ≈ 4 channels — past ~4 channels the shared read-source / write-sink caps the aggregate in-flight window, so more channels stop buying tolerance.

### 4.3 Descriptors × memory latency (1 ch)

desc x delay heatmap

desc x delay lines

Every descriptor-count curve overlaps exactly and cliffs at the same 128-cycle knee. Descriptors are sequential within a channel, so they share the single channel’s 128-beat in-flight window — they add transfer length, never latency tolerance. This is the clean complement to §4.2: channels widen the latency window; descriptors do not.

## 5. The architecture: Little’s Law on real silicon

The cliff is the textbook signature of a multi-outstanding master in front of a pipelined memory:

- Below the knee (L ≤ 128): the in-flight window covers the round trip; the entire latency is paid once as a pipe-fill and the rest streams at line rate. Adding 64 cycles of per-beat latency costs < 0.3 % of a 1 MB run.

- At/above the knee (L ≥ 128): Little’s Law governs — BW ≈ (in_flight_beats / L) × peak. With in_flight = 128 and peak ≈ 1435 MB/s:

| delay | measured (1 ch) | 128/L × 1435 | match |
| --- | --- | --- | --- |
| 128 | 1258 | 1435 | knee |
| 256 | 689 | 718 | −4 % |
| 512 | 362 | 359 | exact |
| 1024 | 186 | 179 | +4 % |
| 4096 | 47 | 45 | exact |

The fit tightens as L grows (arbitration overhead becomes a smaller fraction). The window is AR_MAX_OUTSTANDING (8) × burst_len (16) = 128 beats per channel; channels scale it linearly until the shared slave caps the aggregate (≈ 4 × in this harness).

## 6. What we learned

- The engine is back-to-back-saturated at ~94 % of the one-direction AXI ceiling, flat across every descriptor count and channel count at 1 MB. The 6 % gap is steady inter-burst arbitration, not backpressure.

- Descriptors are free. Concurrent prefetch means chain length adds no per-descriptor cost.

- Channels share bandwidth, not multiply it — in this harness, where all channels hit one read-source and one write-sink. With independent backing memory per channel you’d expect N× scaling; that’s a follow-on.

- Channels buy latency tolerance. The 128-cycle hide-window scales with channel count up to the shared-slave saturation point (~4 ch).

- Descriptors buy none. They share a channel’s outstanding window.

- Size matters only through startup. A fixed ~90-cycle pipe-fill amortizes; ≥ 64 KB is within 2 % of peak.

- The timing fixes cost nothing in throughput. These numbers match the pre-fix builds — the fixes bought positive slack, not regression.

## Appendix: data files & reproduce

| File | Sweep |
| --- | --- |
| matrix_2026-06-15.json/.csv | channels × descriptors, 40 configs, 1 MB |
| chan_x_delay_2026-06-16.json | channels {1,2,4,8} × delay {0..4096} |
| desc_x_delay_2026-06-16.json | desc {1,2,4,8,16} × delay {0..4096}, 1 ch |
| size_sweep_2026-06-16.json | 1 ch, 1 desc, 8 KB→1 MB |
| plots/*.png | figures above (host/plot_char_reports.py) |

cd flows-stream-bridge/host && source $REPO_ROOT/env_python
# matrix:
python3 run_characterization.py --port /dev/ttyUSB2 --output matrix.json
# channels x delay:
python3 run_characterization.py --port /dev/ttyUSB2 --phase 1 --channels 1 2 4 8 \
    --resp-delays 0,8,32,64,128,256,512,1024,2048,4096 --output chan_x_delay.json
# desc x delay:
python3 run_characterization.py --port /dev/ttyUSB2 --channels 1 \
    --resp-delays 0,8,32,64,128,256,512,1024,2048,4096 --output desc_x_delay.json
# plots:
python3 plot_char_reports.py --matrix ../reports/perf/matrix_2026-06-15.json \
    --chan-delay ... --desc-delay ... --size ... --outdir ../reports/perf/plots
# CSV from a matrix JSON (column dictionary in this appendix):
python3 perf_json_to_csv.py ../reports/perf/matrix_2026-06-15.json --out matrix.csv

CSV columns (current runner): date,time,config,channels,descriptors,desc_KB, mb_moved,bus_time_s,bus_throughput_MBps,bus_max_one_dir_MBps, bus_max_net_moved_MBps,bus_e2e_util_pct,datapath_{R,W,E2E}_pct, {R,W}_{prod,bp,starv}_pct,dma_time_s,throughput_MBps. Bus-side columns are authoritative; the two host-side columns (dma_time_s, throughput_MBps) are transparency-only. The 16desc_* configs trip trace.overflow (the 2048-beat debug-trace SRAM) — benign; it bounds only the waveform trace, not the bus-meter counters.