Table of Contents

# Pm Acpi Index

Generated: 2025-12-06

# APB PM/ACPI Specification - Table of Contents

Component: APB Power Management / ACPI Controller Version: 1.0 Last Updated: 2025-12-01 Status: Production Ready

## Document Organization

### Chapter 1: Overview

- 01_overview.md - Component overview

- 02_architecture.md - Architecture

### Chapter 2: Blocks

- 00_overview.md - Block hierarchy

### Chapter 3: Interfaces

- 00_overview.md - Interface summary

### Chapter 4: Programming Model

- 00_overview.md - Programming overview

### Chapter 5: Registers

- 01_register_map.md - Register map

## Block Diagram

PM/ACPI Block Diagram

## Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2025-12-01 | RTL Design Sherpa | Initial specification |

# APB PM/ACPI - Overview

## Introduction

The APB PM/ACPI controller provides ACPI-compatible power management functionality with an APB interface. It handles system power states, events, and timer functionality.

## Key Features

- ACPI power management events

- PM1a/PM1b event blocks

- PM1 control block

- PM timer (24-bit, 3.579545 MHz)

- GPE (General Purpose Events) support

- System sleep state control

- Sci/SMI generation

## Applications

- System power management

- Sleep state transitions (S0-S5)

- Wake event handling

- Power button events

- Thermal events

## Block Diagram

PM/ACPI Block Diagram

## Register Summary

| Offset | Name | Description |
| --- | --- | --- |
| 0x00 | PM1_STS | PM1 Event Status |
| 0x04 | PM1_EN | PM1 Event Enable |
| 0x08 | PM1_CNT | PM1 Control |
| 0x0C | PM_TMR | PM Timer (24-bit) |
| 0x10 | GPE0_STS | GPE0 Status |
| 0x14 | GPE0_EN | GPE0 Enable |
| 0x18 | GPE1_STS | GPE1 Status |
| 0x1C | GPE1_EN | GPE1 Enable |

Next: 02_architecture.md

# APB PM/ACPI - Register Map

## Register Summary

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | PM1_STS | RO/W1C | PM1 Status |
| 0x04 | PM1_EN | RW | PM1 Enable |
| 0x08 | PM1_CNT | RW | PM1 Control |
| 0x0C | PM_TMR | RO | PM Timer |
| 0x10 | GPE0_STS | RO/W1C | GPE0 Status |
| 0x14 | GPE0_EN | RW | GPE0 Enable |
| 0x18 | GPE1_STS | RO/W1C | GPE1 Status |
| 0x1C | GPE1_EN | RW | GPE1 Enable |

## PM1_STS (0x00)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | TMR_STS | W1C | Timer overflow status |
| 4 | BM_STS | W1C | Bus master status |
| 5 | GBL_STS | W1C | Global release status |
| 8 | PWRBTN_STS | W1C | Power button status |
| 9 | SLPBTN_STS | W1C | Sleep button status |
| 10 | RTC_STS | W1C | RTC alarm status |
| 15 | WAK_STS | W1C | Wake status |

## PM1_EN (0x04)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | TMR_EN | RW | Timer overflow enable |
| 5 | GBL_EN | RW | Global release enable |
| 8 | PWRBTN_EN | RW | Power button enable |
| 9 | SLPBTN_EN | RW | Sleep button enable |
| 10 | RTC_EN | RW | RTC alarm enable |

## PM1_CNT (0x08)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | SCI_EN | RW | SCI enable |
| 1 | BM_RLD | RW | Bus master reload |
| 2 | GBL_RLS | RW | Global release |
| 12:10 | SLP_TYP | RW | Sleep type |
| 13 | SLP_EN | RW | Sleep enable |

## PM_TMR (0x0C)

| Bits | Name | Access | Description |
| --- | --- | --- | --- |
| 23:0 | TMR_VAL | RO | Timer value (3.579545 MHz) |

Back to: PM/ACPI Specification Index