RTL Design Sherpa

DDR2 / LPDDR2 Family Controller Micro-Architecture Specification 0.1

June 13, 2026

Table of Contents

1 Pumice Mas Index	24

2 Document Information	24

2.1 DDR2 / LPDDR2 Family Controller Micro-Architecture Specification	24

2.2 Document Purpose	24

2.3 Document Scope	25

2.4 Revision History	25

3 Architecture and Datapath	26

3.1 Layer Hierarchy	27

3.2 Read / Write Datapath: AXI -> split -> intake -> CAM	27

3.2.1 Why Two CAMs, Not One	28

3.2.2 Snarf (read-your-write forwarding)	28

3.3 Scheduler and Command Stream	28

3.4 Clocking Topology	29

3.5 DFI Phase / Rate Topology	29

4 Top-Level Port List	30

4.1 Parameters	30

4.2 Parameter-Dependent Widths	30

4.3 Clocks and Reset	31

4.4 Register cpuif (PeakRDL passthrough)	31

4.5 AXI4 Slave Port List	32

4.6 DFI 2.1 Master Port List	32

4.7 SystemVerilog Header Snippet	33

5 Clocks and Reset	33

5.1 Clock Domains	33

5.2 Reset Topology	34

5.3 CDC	34

5.4 Reset / Init Sequence	35

6 Top-Level Integration (pumice_core / pumice_top)	35

6.1 Purpose	36

6.2 pumice_core — the three layers	36

6.2.1 Layer instantiation inventory (pumice_core)	37

6.2.2 Inter-layer nets	37

6.3 pumice_top — CSR + by-name config	37

6.4 What the top does not do	38

7 pumice_axi4_ifc (AXI4 front-end macro)	39

7.1 Purpose	39

7.2 FUBs	39

7.3 External Boundaries	40

7.4 De-FSM’d CAMs	40

7.5 Tests	41

8 pumice_mem_cmd_scheduler (command scheduler macro)	41

8.1 Purpose	41

8.2 FUBs	41

8.3 FSM-Free Bank Timing	42

8.4 External Boundaries	42

8.5 Tests	43

9 Data Path (no standalone macro)	43

9.1 Purpose	43

9.2 Write Path (host -> DRAM)	44

9.3 Read Path (DRAM -> host)	44

9.4 Why No Beat-Sequencer FSM	44

9.5 Tests	45

10 pumice_dfi_layer (DFI v2.1 interface macro)	45

10.1 Purpose	45

10.2 FUBs	45

10.3 External Boundaries	46

10.4 Multi-Phase Pack Convention	46

10.5 LPDDR2 Command Encoding	46

10.6 Tests	47

11 AXI4 Slave + Intakes (axi4_slave_wr / axi4_slave_rd)	47

11.1 Purpose	47

11.2 pumice_wr_intake — structure	48

11.2.1 Write data flow	48

11.2.2 Ragged-burst handling	49

11.3 pumice_rd_intake — structure	49

11.3.1 Read admission and ordering	49

11.4 External interface	50

11.5 Notes	50

12 Address Mapper (addr_mapper)	51

12.1 Purpose	51

12.2 Parameters	51

12.3 Interface	52

12.4 The single-knob layout	52

12.4.1 Field extraction (variable-base shifts/masks)	53

12.5 The classic schemes are just bank_lsb settings	53

12.6 Bank XOR-hash	53

12.7 Mirror to the DV address model	54

12.8 Timing	54

12.9 Notes / flags	54

13 Read Command CAM (pumice_rd_cmd_cam)	54

13.1 Purpose	55

13.2 Parameters	55

13.3 Entry state (per slot)	55

13.4 Interfaces	56

13.4.1 Insert (from pumice_rd_intake ar_push, AR order)	56

13.4.2 Scheduler lookups (N, keyed {bank, row})	56

13.4.3 Oldest not-issued port	56

13.4.4 Issue notify	56

13.4.5 DFI read return (data in, issue order)	56

13.4.6 Drain to pumice_rd_intake (AR order, oldest-first, ready-gated)	57

13.5 De-FSM’d sequencing	57

13.6 Eviction	57

13.7 Reset	57

13.8 Notes / flags	57

14 Write Data CAM (pumice_wr_data_cam)	58

14.1 Purpose	58

14.2 Parameters	58

14.3 Entry state (per slot)	59

14.4 Age selectors	59

14.5 Interfaces	60

14.5.1 Insert (from pumice_wr_intake aw_push)	60

14.5.2 Fill (from pumice_wr_intake wdata pop)	60

14.5.3 Snarf lookup + stream (from pumice_rd_intake)	60

14.5.4 Oldest port + scheduler lookups	60

14.5.5 Commit (from arbiter) + drain to DFI write	60

14.5.6 Commit-done → B	61

14.6 De-FSM’d sequencing	61

14.7 Difference from the read CAM	61

14.8 Reset / busy	61

14.9 Notes / flags	61

15 Write-to-Read Forward — RETIRED as a standalone block	62

15.1 Why it was retired	62

15.2 Where forwarding lives now	62

15.3 Restrictions (narrower than the old block)	62

15.4 Memory Ordering	63

15.5 Tests	63

16 Command Arbiter (pumice_cmd_arbiter)	63

16.1 Purpose	64

16.2 Priority (each cycle)	64

16.2.1 Column issuability	64

16.2.2 Auto-precharge (inline page policy)	65

16.3 Per-bank ACT/PRE re-issue guard	65

16.4 Interface (arbiter)	65

16.5 Side-effects (all gated on w_fire = w_valid && cmd_ready_i)	66

16.6 v1 scope / TODO	66

16.7 pumice_mem_cmd_scheduler wrapper	66

17 Page Policy (inline in pumice_cmd_arbiter)	67

17.1 The live decision	68

17.2 Why inline	68

17.3 The retained page_predictor.sv file	69

17.4 Notes / flags	69

18 Global Timers (global_timers)	70

18.1 Purpose	70

18.2 Constraint Scope	71

18.3 Synthesis Parameters	71

18.4 Per-Rank Trackers	72

18.4.1 tFAW window — r_faw_slots[NUM_RANKS][4]	72

18.4.2 tRRD — r_trrd_cnt[NUM_RANKS]	72

18.5 Global Trackers (channel-wide)	72

18.6 Readiness Outputs (strict-flop)	73

18.7 Observability	74

18.8 Interface	74

18.8.1 Clocks / Reset	74

18.8.2 Timing config (CSR-backed)	74

18.8.3 Events from the arbiter	74

18.8.4 Readiness to the arbiter	75

18.8.5 Observability	75

18.9 Timing Budget	75

18.10 Verification Notes (cocotb test plan)	75

18.11 Open Questions / Future Work	76

19 Global Timers (global_timers)	77

19.1 Purpose	77

19.2 Timer mechanics	78

19.3 Observability	78

19.4 Parameters	78

19.5 Scope	78

19.6 Tests	78

20 Refresh Controller (refresh_ctrl)	79

20.1 Purpose	79

20.2 Synthesis Parameters	80

20.3 Behavioral Blocks	80

20.3.1 1. tREFI Counter (r_refi_cnt)	80

20.3.2 2. Pending Accumulator (r_pending)	80

20.3.3 3. Drain Quota (r_burst_remaining)	81

20.3.4 4. REFpb Bank Rotor (r_bank_rotor)	81

20.4 Interface	82

20.4.1 Parameters and Clocking	82

20.4.2 Inputs	82

20.4.3 Outputs	82

20.4.4 Observability (obs_*, future CSR readout)	82

20.5 Timing / Behavior	83

20.6 Verification Notes (cocotb test plan)	83

20.7 Open Questions / Future Work	84

21 Init Sequencer (init_sequencer)	85

21.1 Purpose	86

21.2 Synthesis Parameters	86

21.3 Interface	87

21.3.1 Clock / reset	87

21.3.2 Configuration	87

21.3.3 JEDEC init-sequence waits (CSR-backed, MC cycles)	87

21.3.4 DFI status	88

21.3.5 MR-shadow write port (muxed with CSR by pumice_mem_cmd_scheduler)	88

21.3.6 DRAM command request into the scheduler (issued while init_busy)	88

21.3.7 Legacy ZQCL handshake (DDR3+; unused for DDR2/LPDDR2)	88

21.3.8 Status	88

21.4 FSM	89

21.5 DDR2 Sequence	90

21.5.1 DDR2 MR values	91

21.6 LPDDR2 Sequence	92

21.6.1 LPDDR2 MR values	92

21.7 Init-Busy Gating	93

21.8 Verification Notes (cocotb test plan)	93

21.9 Open Questions / Future Work	94

22 Power-Down Controller (powerdown_ctrl)	95

22.1 Purpose	95

22.2 Synthesis Parameters	96

22.3 FSM	96

22.3.1 PDE-vs-SR Selection	96

22.3.2 Transitions	97

22.3.3 CKE Behavior	97

22.4 Interface	98

22.4.1 Control / Configuration Inputs	98

22.4.2 Request / Grant + CKE	98

22.4.3 Observability (future CSR readout)	98

22.5 Verification Notes (cocotb test plan)	99

22.6 Open Questions / Future Work	99

23 Mode Register (mode_register)	100

23.1 Purpose	100

23.2 Live Decoded Outputs	101

23.3 Burst Length: Fixed per Instance, Parameterized for Family Reuse	102

23.4 Scope	102

23.5 Tests	103

24 Write Data Path (pumice_wr_data_cam + pumice_dfi_wr_serializer)	103

24.1 Purpose	104

24.2 pumice_wr_data_cam — three movers over one SRAM	104

24.2.1 Entry state	104

24.2.2 SRAM slot pre-allocation	105

24.2.3 Age-based selectors	105

24.3 Snarf: write-to-read forwarding (was the standalone wr2rd_forward)	105

24.4 Commit drain and B response	106

24.5 pumice_dfi_wr_serializer — the DFI-domain endpoint	106

24.6 Block Pipeline View	107

24.7 pumice_wr_data_cam interface	108

24.7.1 Insert (from pumice_wr_intake aw_push)	108

24.7.2 Fill data (from pumice_wr_intake wdata pop)	108

24.7.3 Snarf lookup + stream (to/from pumice_rd_intake)	108

24.7.4 Oldest port + scheduler lookups (to pumice_cmd_arbiter)	108

24.7.5 Commit / drain (to pumice_dfi_layer write path)	109

24.8 pumice_dfi_wr_serializer parameters	109

24.9 Verification Notes (cocotb test plan)	109

24.10 Open Questions / Future Work	110

25 Read Data Path (pumice_rd_cmd_cam + pumice_dfi_rd_aligner)	111

25.1 Purpose	111

25.2 pumice_rd_cmd_cam — reorder buffer, two movers over one SRAM	112

25.2.1 Entry state and age	112

25.2.2 The three age selectors	112

25.2.3 Ordering guarantee	112

25.2.4 Issue-order return fill	113

25.3 pumice_dfi_rd_aligner — the DFI-domain endpoint	113

25.4 Block Pipeline View	114

25.5 pumice_rd_cmd_cam interface	114

25.5.1 Insert (from pumice_rd_intake ar_push, AR order)	114

25.5.2 Scheduler ports (to pumice_cmd_arbiter)	114

25.5.3 Issue notify (from scheduler)	115

25.5.4 DFI return (from pumice_dfi_layer read path, issue order)	115

25.5.5 Drain (to pumice_rd_intake R source, AR order)	115

25.6 pumice_dfi_rd_aligner parameters	115

25.7 Out-of-Order Completion (across AXI IDs)	116

25.8 Verification Notes (cocotb test plan)	116

25.9 Open Questions / Future Work	116

26 DFI Command Formatter (dfi_cmd_formatter + dfi_signal_pack)	117

26.1 Purpose	118

26.2 Synthesis Parameters (dfi_cmd_formatter)	118

26.3 Command Interface and Runtime Phase Placement	119

26.4 DDR2 Encoding Branch	119

26.5 LPDDR2 Encoding Branch (bit-exact JESD209-2F Table 60)	121

26.6 Multi-Phase Packing and CS_n	122

26.7 dfi_signal_pack — Final Registered DFI Stage	123

26.8 Interface (dfi_cmd_formatter outputs)	123

26.9 Timing Budget	124

26.10 Verification Notes (cocotb test plan)	124

26.11 Open Questions / Future Work	125

27 DFI Layer and Host-Width Gearing (pumice_dfi_layer + pumice_top_geared)	125

27.1 Concept 1 — The DFI Layer (pumice_dfi_layer)	126

27.1.1 Purpose	126

27.1.2 Internal structure (verified against the RTL)	126

27.1.3 Key parameters	127

27.1.4 DFI-clock runtime knobs	128

27.2 Concept 2 — Host-Width Gearing (pumice_top_geared)	128

27.2.1 Purpose	128

27.2.2 GEAR-1 bypass (bit-identical)	128

27.2.3 Geared branch	129

27.2.4 Key parameters	129

27.3 Narrow-Device (x16) Support — the Two Width Granularities	129

27.3.1 DFI_PHASE CSR (rd_phase / wr_phase)	130

27.4 Interface Summary	130

27.4.1 pumice_dfi_layer — controller side (ctl_clk)	130

27.4.2 pumice_dfi_layer — PHY side (dfi_clk)	131

27.4.3 pumice_top_geared	131

27.5 Verification Notes (cocotb test plan)	131

27.6 Open Questions / Future Work	132

28 Transaction Queue (retired — replaced by the two command CAMs)	132

RETIRED — no standalone FUB	132

28.1 What the CAMs do instead	133

28.1.1 Scheduler visibility (replaces the parallel q_entries bus)	133

28.1.2 Entry lifecycle (replaces FREE/PENDING/ISSUED/COMPLETING)	133

28.1.3 Age (replaces the saturating age counter)	134

28.1.4 Backpressure (replaces q_high_water / q_full)	134

28.2 Why the split disappeared	134

29 Bank Timer (bank_timer + pumice_bank_timers)	135

29.1 Purpose	135

29.2 Synthesis Parameters	136

29.2.1 bank_timer	136

29.2.2 pumice_bank_timers	136

29.3 Instantiation Pattern	136

29.4 The Timer Pool (per bank)	137

29.5 Row-Open Register and Auto-Precharge (no FSM)	138

29.6 Combinational safe_* Outputs	139

29.7 Derived State (observability only)	139

29.8 Interface (pumice_bank_timers)	140

29.8.1 Clocks / Reset	140

29.8.2 Timing config (CSR-backed, per-bank JEDEC cycle counts)	140

29.8.3 Command-event strobes from the arbiter	140

29.8.4 Per-bank readiness to the arbiter (combinational, single-stage)	141

29.8.5 Observability	141

29.9 Timing Budget	141

29.10 Verification Notes (cocotb test plan)	142

29.11 Open Questions / Future Work	142

30 ODT Control (absorbed — no standalone block)	143

ABSORBED — No standalone FUB	143

30.1 Where ODT Lives Today	143

30.2 Why ODT Is Off on the Board Target	144

30.3 What a Full ODT Implementation Would Add	144

30.4 Verification Notes (cocotb test plan)	145

30.5 Open Questions / Future Work	145

31 AXI4 Slave Protocol	145

31.1 Compliance Profile	146

31.2 Data Width	146

31.3 Burst Splitting	146

31.4 Supported Features	146

31.5 Unsupported / Ignored	147

31.6 Backpressure Semantics	147

31.7 Response Ordering	147

31.8 Read-Your-Write Forwarding (snarf)	147

31.9 Timing Contract	148

31.10 Error Responses	148

31.11 Open Questions / Future Work	148

32 DFI v2.1 Master Protocol	149

32.1 DFI Spec Reference	149

32.2 Sub-Interface Support Summary	149

32.3 Clock and CDC	150

32.4 Command Sub-Interface	150

32.4.1 Phase Topology	150

32.4.2 Chip-Select and ODT	150

32.4.3 DDR2 vs LPDDR2 Encoding	150

32.5 Write-Data Sub-Interface	151

32.6 Read-Data Sub-Interface	151

32.6.1 Read-Data Error Handling	151

32.7 Status / Init Sub-Interface	151

32.8 Pin Tie-Offs	151

32.9 Open Questions / Future Work	152

33 Register Access Interface (PeakRDL cpuif)	152

33.1 The cpuif Contract	152

33.2 Attaching an APB / AXI-Lite Bridge	153

33.3 Address Space	154

33.4 Behavior	154

33.4.1 Access Cycles	154

33.4.2 Sub-Word Accesses	154

33.4.3 Error Conditions	154

33.5 Config-Drive Model	155

33.6 Open Questions / Future Work	155

34 Register Map	155

34.1 Source of Truth	156

34.2 Register Summary	156

34.3 Field-Level Detail	158

34.3.1 CTRL @ 0x000 (rw)	158

34.3.2 STATUS @ 0x004 (RO, hw-written)	159

34.3.3 STATUS_HISTORY @ 0x008 (RO)	159

34.3.4 TIMINGS_RC_RCD_RP_RAS @ 0x010 (rw)	159

34.3.5 TIMINGS_RFC_REFI @ 0x014 (rw)	159

34.3.6 TIMINGS_RRD_FAW_WTR_CCD @ 0x018 (rw)	159

34.3.7 TIMINGS_CL_CWL_WR @ 0x01C (rw)	160

34.3.8 MR0..MR3 @ 0x020 / 0x024 / 0x028 / 0x02C (rw)	160

34.3.9 PASR_BANK_MASK_RANK0 @ 0x030 (rw)	160

34.3.10 PASR_SEG_MASK_RANK0 @ 0x034 (rw)	160

34.3.11 TEMP_DERATE_RANK0 @ 0x038 (RO, hw-written)	160

34.3.12 SCHED_TUNING @ 0x040 (rw)	161

34.3.13 PAGE_PRED_TUNING @ 0x044 (rw)	161

34.3.14 REFRESH_TUNING @ 0x048 (rw)	161

34.3.15 ADDR_MAP @ 0x04C (rw) — replaces the retired ADDR_MAP_TUNING	161

34.3.16 INIT_TUNING @ 0x050 (rw)	162

34.3.17 TIMINGS_RTP_RTW @ 0x054 (rw)	162

34.3.18 INIT_TIMING0 @ 0x058 (rw)	162

34.3.19 INIT_TIMING1 @ 0x05C (rw)	163

34.3.20 DFI_PHASE @ 0x060 (rw)	163

34.3.21 PHY_TIMING @ 0x064 (rw)	163

34.3.22 Observation registers (RO)	163

34.3.23 ID @ 0xFF0 (RO) — reset 0xD2020001	164

34.3.24 BUILD @ 0xFF4 (RO)	164

34.4 Multi-Rank / Multi-Bank Registers	164

34.5 Reset Values	165

34.6 Open Questions / Future Work	165

35 Runtime CSR Fields (Config-Drive Model)	165

35.1 What “Runtime” Means Here	166

35.2 Commit Semantics	167

35.3 Safe-Change Guidance	167

35.4 Recommended Programming Order	167

35.5 Open Questions / Future Work	168

36 Family Config Knobs (DDR2 / LPDDR2)	168

36.1 The Family Selector: PHY_TIMING.memtype	169

36.2 Field-by-Field Applicability	169

36.2.1 SCHED_TUNING (0x040)	169

36.2.2 REFRESH_TUNING (0x048)	170

36.2.3 ADDR_MAP (0x04C) — family-agnostic	170

36.2.4 INIT_TUNING (0x050) / INIT_TIMING0/1 (0x058/0x05C)	170

36.2.5 PHY_TIMING (0x064)	171

36.2.6 LPDDR2-only registers	171

36.3 Feature Discovery	171

36.4 Not Present in This Generation	172

36.5 Open Questions / Future Work	172

37 Initialization Sequence	172

37.1 Pre-Reset Setup	172

37.2 Init Trigger	173

37.3 Init Sequence Details	174

37.3.1 DDR2 chain (mirrors LiteDRAM’s proven Nexys A7 sequence)	174

37.3.2 LPDDR2 chain (JESD209-2F power-up + MR configuration)	175

37.4 Multi-Rank Init Differences	175

37.5 ZQ Retry	175

37.6 Post-Init Power-State	176

37.7 Software Wait Times	176

37.8 Reset Recovery	176

37.9 Open Questions / Future Work	177

38 Refresh and Power-State Programming	177

38.1 Tuning Refresh Behavior	177

38.1.1 When to Tune	178

38.2 LPDDR2 PASR (Partial Array Self-Refresh)	178

38.3 Temperature Compensation (LPDDR2)	178

38.4 Self-Refresh Entry	179

38.5 DPD (LPDDR2 Deep Power Down)	179

38.6 Open Questions / Future Work	180

39 Multi-Rank Programming	180

39.1 Discovery	181

39.2 Per-Rank Mode Register Loads	181

39.3 Per-Rank PASR (LPDDR2)	181

39.4 ODT	181

39.5 Per-Rank Disable	181

39.6 Address Mapping for Multi-Rank	181

39.7 Refresh Behavior with Multi-Rank	182

39.8 Power State Per-Rank	182

39.9 Per-Rank / Per-Bank Observation	182

39.10 Multi-Rank Bring-Up Checklist	182

39.11 Open Questions / Future Work	183

40 Error Handling	183

40.1 Error Categories	184

40.2 Software Response Patterns	184

40.2.1 Init Failure	184

40.2.2 DFI Read-Data Timeout (board bring-up)	185

40.2.3 Refresh Pressure	185

40.3 STATUS_HISTORY for Bring-Up	185

40.4 Diagnostic Sequence for Suspected Bug	185

40.5 Soft Reset vs. Re-Init	186

40.6 Open Questions / Future Work	186

41 Build-Time Configuration Reference	186

41.1 Parameters	187

41.2 Data-Width Geometry (DW = DRAM_BEAT_WIDTH x DFI_RATE)	187

41.2.1 Host-Width Freedom (pumice_top_geared)	187

41.3 Parameter Sanity	188

41.4 Filelists	188

41.5 Open Questions / Future Work	188

42 Runtime Configuration Reference	189

42.1 Bring-Up Configuration Order	189

42.2 Address-Map Tuning (single knob)	189

42.3 Characterization Sweep Order	190

42.4 Telemetry to Watch	190

42.5 Workload-Specific Recipes	191

42.5.1 Streaming (DMA, video, audio capture)	191

42.5.2 Low-Latency Bursty (CPU)	191

42.5.3 Real-Time / Safety-Critical	191

42.6 Telemetry-Driven Auto-Tuning Loop	192

42.7 Open Questions / Future Work	192

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# 1	Pumice Mas Index

Generated: 2026-07-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	DDR2 / LPDDR2 Family Controller Micro-Architecture Specification

Document Number: DDR2-LPDDR2-MAS-001 Version: 0.4 Status: Draft - reconciled with rearchitected RTL Classification: Open Source - Apache 2.0 License

## 2.2	Document Purpose

This Micro-Architecture Specification (MAS) is the implementation-level companion to the DDR2 / LPDDR2 Hardware Architecture Specification (HAS). Where the HAS answers “what is the architecture and why,” this MAS answers “what is in the RTL and how does it work.”

The MAS is the document RTL designers, verification engineers, and integrators consult when writing or reviewing SystemVerilog code, building testbenches, and stitching the controller into an SoC.

Target Audience:

- RTL designers implementing the FUBs described in this document

- Verification engineers writing per-FUB cocotb tests

- Integrators stitching the controller into an SoC

- Software engineers writing low-level memory configuration code (driver authors, bring-up engineers)

Companion Documents:

- DDR2/LPDDR2 Family Controller HAS (../pumice_has/) — high-level architecture and design rationale

- DDR2/LPDDR2 DFI BFM documentation (in RTLDesignSherpa-DV) — verification-side reference

## 2.3	Document Scope

The MAS is the micro-architecture view: per-FUB inputs/outputs, internal state, FSM diagrams, datapath timing, register-level pipeline stages, and per-block timing budgets. It assumes the reader has read the HAS for context.

This document covers:

- Top-level integration wiring (pumice_ctrl)

- Each FUB’s interface signal list, internal storage, datapath flow, FSM, and timing budget

- AXI4 and DFI v2.1 wire-level protocol details specific to this controller

- APB programming interface and the full CSR register map

- Programming sequences (init, refresh, power-down, multi-rank, error handling)

- Build-time and runtime configuration reference

This document does not cover:

- Higher-level architectural rationale (see HAS)

- Verification plans or coverage models (see dv/README.md)

- Floorplan or layout guidance (project-specific)

- Bit-level CSR YAML — that lives in regs/ and is the source of truth for CSR generation

## 2.4	Revision History

| Version | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.1 | 2026-06-14 | RTL Design Sherpa | Initial skeleton — chapter outline, FUB list, port list scaffold |
| 0.2 | 2026-07-07 | RTL Design Sherpa | Narrow-device (x16) support: DRAM_DEVICE_WIDTH param; burst-length scaling to device-word units (§15); addr_mapper column granularity = device word via BYTE_OFFSET_WIDTH (§3); DFI_PHASE CSR (rd_phase/wr_phase). Fixes the on-silicon DDR2 read failure (Nexys A7 x16). |
| 0.3 | 2026-07-07 | RTL Design Sherpa | §20 mode_register: document burst length as a fixed-per-instance init constant, decoded (not hardcoded) so the same RTL retargets DDR2 BL4 / DDR3-DDR4 BL8; BC4/burst-chop declared out of scope (always issue full BL). |
| 0.4 | 2026-07-12 | RTL Design Sherpa | Full reconciliation with the rearchitected RTL. ch02 block chapters rewritten to the live FUBs: retired txn_queue/bank_machine/xbank_timers/cmd_encoder/odt_ctrl/standalone page_predictor -> CAMs / FSM-free bank_timer / global_timers / dfi_cmd_formatter / arbiter inline open-page. addr_mapper = single ADDR_MAP.bank_lsb knob (scheme mux retired). LPDDR2 fully functional (bit-exact JESD209-2F CA + JEDEC MR init). pumice_top_geared host-width gearing. CSR map reconciled to the RDL. All block diagrams regenerated. |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Architecture and Datapath

This chapter is the implementation-level architectural orientation: the live three-layer hierarchy, the read/write datapath flow, and the clocking topology. Per-macro detail is in section 2; per-interface detail is in section 3.

## 3.1	Layer Hierarchy

The controller is a three-layer core wrapped by a top that adds the PeakRDL-generated register block. The top-of-tree is pumice_top (rtl/top/pumice_top.sv), with an optional host-width wrapper pumice_top_geared above it:

pumice_top_geared (rtl/top/pumice_top_geared.sv)   -- OPTIONAL host-width wrapper
  |- axi4_dwidth_converter_wr/_rd (formal IP; only when HOST_AXI_DATA_WIDTH != DW)
  |- pumice_top (rtl/top/pumice_top.sv)
       |- pumice_csr (regs/generated/rtl/, PeakRDL passthrough cpuif)
       |- pumice_core (rtl/top/pumice_core.sv)
            |- pumice_axi4_ifc          (rtl/macro/pumice_axi4_ifc.sv)
            |- pumice_mem_cmd_scheduler (rtl/macro/pumice_mem_cmd_scheduler.sv)
            |- pumice_dfi_layer         (rtl/macro/pumice_dfi_layer.sv)

pumice_core wires the three macros, built bottom-up:

| Macro (module) | Role | Chapter |
| --- | --- | --- |
| pumice_axi4_ifc | Host AXI4 slave + burst splitters + write/read CAMs + snarf | 2.1 |
| pumice_mem_cmd_scheduler | Arbiter + per-bank/global timers + refresh + init + mode reg | 2.2 |
| pumice_dfi_layer | Single async-FIFO CDC + DFI-clock command/write/read datapath | 2.4 |

The data path is not a standalone macro. Write and read burst buffers live in the CAMs inside pumice_axi4_ifc (SRAM-backed, de-FSM’d streaming readers); the DFI-clock serializer / aligner live in pumice_dfi_layer. Section 2.3 documents this split in place of the old data_path_macro.

Configuration (timings, DFI phases, page policy, address map) is delivered to pumice_core on ports, driven by name from the CSR hwif_out.* fields in pumice_top. There are no config ports on pumice_top other than the register cpuif – software programs every timing/phase/policy through the register bus.

## 3.2	Read / Write Datapath: AXI -> split -> intake -> CAM

The host AXI4 face is pumice_axi4_ifc. Each host burst is first split at DRAM-burst-byte boundaries by the shared axi_master_wr_splitter / axi_master_rd_splitter (one DRAM burst per split command), then handed to the dumb 1:1 intakes:

- pumice_wr_intake (axi4_slave_wr + AW-meta FIFO + wr-data FIFO + addr_mapper) pushes (bank, row, col, id) plus write data into pumice_wr_data_cam.

- pumice_rd_intake (axi4_slave_rd + addr_mapper + snarf probe) pushes (bank, row, col, id) into pumice_rd_cmd_cam, and probes the write CAM for read-your-write forwarding.

The salient property: there is exactly one stage where AXI-layer concepts (burst, ID, write strobe) cross into DRAM-layer concepts (rank, bank, row, column). That stage is addr_mapper inside each intake. Upstream everything is AXI; downstream of the CAMs everything is DRAM.

### 3.2.1	Why Two CAMs, Not One

The CAM split is between the write and read paths because the two CAMs hold different metadata, carry different data, and have different lifetimes:

| CAM | Key | Data / storage | Retired on |
| --- | --- | --- | --- |
| pumice_wr_data_cam | (bank, row) | (col, id, age, slot) + write-data SRAM; fill/commit-drain/snarf movers | commit-done (B push) |
| pumice_rd_cmd_cam | (bank, row) | (col, id, age, slot) + read-return SRAM; return-fill/drain movers | last drained R beat |

A single unified CAM would either carry both data sets (the write CAM needs a write-data SRAM the read CAM does not) or force an awkward “is_write” predicate on every scheduler lookup. Keeping them separate lets the read and write paths size independently.

Both CAMs are de-FSM’d: each stores burst data in an SRAM and exposes a streaming read engine that is FIFO-fed / oldest-pick beat-counter driven – no active/slot state latch. The r_fdone fill-complete flag gates schedulability (and, on the write side, snarf).

### 3.2.2	Snarf (read-your-write forwarding)

pumice_rd_intake probes the write CAM before scheduling a read. On a hit the read is streamed directly from the write CAM’s SRAM via the snarf mover in pumice_wr_data_cam – there is no standalone wr2rd_forward block in the live path. Snarf is limited to unscheduled writes with the same id and same burst length.

## 3.3	Scheduler and Command Stream

pumice_mem_cmd_scheduler queries both CAMs in the (bank, row) dimension via N_LU = NUM_BANKS parallel lookup ports, picks one command with pumice_cmd_arbiter (the open-page decision is inline in the arbiter – there is no separate page_predictor in the issue path), and emits a single abstract DRAM command {op, rank, bank, row, col, ap} into an output command FIFO. The scheduler is single-issue, PHY/nphases-agnostic, and runs entirely on aclk. Per-bank JEDEC readiness is stamped by pumice_bank_timers (wrapping the FSM-free bank_timer); cross-bank turnaround windows (tFAW/tRRD/tWTR/tRTW/tCCD) come from global_timers.

## 3.4	Clocking Topology

Two external clocks; exactly one CDC.

| Clock | Domain members |
| --- | --- |
| aclk | Host AXI, burst splitters, both CAMs, and the whole command scheduler |
| dfi_clk | DFI command path, write serializer, read aligner, DFI 2.1 pin bus |

The single CDC lives inside pumice_dfi_layer (pumice_dfi_cdc) and is built from async gaxi FIFOs only – the command, write-data, and read-data streams plus the init handshake cross there. No other clock crossing exists in the controller. If the SoC’s AXI master is on a different clock than aclk, an external clock converter is required upstream. See section 1.3 for the reset topology.

## 3.5	DFI Phase / Rate Topology

The internal data unit is the DFI word: DW = DRAM_BEAT_WIDTH * DFI_RATE (128 by default). One AXI beat == one DFI word == DFI_RATE DRAM beats; one AXI burst (BL/DFI_RATE beats) == one DRAM burst (BL beats). The DFI command path in pumice_dfi_layer phase-packs the abstract command onto the DFI_RATE phases, placing the command and CS/ODT on wr_phase / rd_phase as programmed by the DFI_PHASE CSR; pumice_dfi_wr_serializer and pumice_dfi_rd_aligner handle the DFI-word to per-phase data split. The scheduler never sees the phase dimension – its issue rate is one command per aclk.

The old “AXI_DATA_WIDTH == DRAM_BEAT_WIDTH” coupling is gone. Host-width freedom is a separate concern handled by the optional pumice_top_geared wrapper, which inserts the formally-verified axi4_dwidth_converter_wr/_rd between a host-width AXI slave and the fixed-DW core (HOST == DW is a bit-identical generate bypass). See docs/AXI_DRAM_GEARING_SCOPE.md.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Top-Level Port List

This chapter is the wire-level port list of the pumice_top module (rtl/top/pumice_top.sv) – the exact signal names, directions, widths, and parameter dependencies that the integration script and SoC top-level use. The optional host-width wrapper pumice_top_geared re-exports the same set with a free HOST_AXI_DATA_WIDTH on the AXI slave face; see docs/AXI_DRAM_GEARING_SCOPE.md.

## 4.1	Parameters

pumice_top is fully parameterized – there is no MEMTYPE/N_PHASES string build any more; memory type is a runtime CSR field (PHY_TIMING.memtype) and the phase count is the integer DFI_RATE.

module pumice_top #(
    parameter int AXI_ID_WIDTH    = 8,
    parameter int AXI_ADDR_WIDTH  = 32,
    parameter int NUM_RANKS       = 1,
    parameter int NUM_BANKS       = 8,
    parameter int ROW_WIDTH       = 14,
    parameter int COL_WIDTH       = 10,
    parameter int DFI_RATE        = 2,
    parameter int DRAM_BEAT_WIDTH = 64,
    parameter int BL              = 8,   // DRAM beats per burst
    parameter int NUM_ENTRIES     = 8,   // CAM depth
    parameter int N_SRAM_SLOTS    = 8,   // per-CAM burst-data SRAM slots
    parameter int CSR_ADDR_W      = 12
);

## 4.2	Parameter-Dependent Widths

Derived locally in the module (see rtl/top/pumice_top.sv):

- DW = DRAM_BEAT_WIDTH * DFI_RATE (host AXI data width == DFI word)

- SW = DW/8

- IW = AXI_ID_WIDTH

- AW = AXI_ADDR_WIDTH

- PHW = (DFI_RATE > 1) ? $clog2(DFI_RATE) : 1

- DFI_DATA_WIDTH = DW

- DFI_STRB_WIDTH = DW/8

- DFI_EN_WIDTH = DFI_RATE

- DFI_VALID_WIDTH = DFI_RATE

- DFI_ADDR_BUS_W = ROW_WIDTH * DFI_RATE

- DFI_BANK_BUS_W = $clog2(NUM_BANKS) * DFI_RATE

- DFI_CTRL_BUS_W = 1 * DFI_RATE

- DFI_CS_BUS_W = NUM_RANKS * DFI_RATE

## 4.3	Clocks and Reset

Two clocks, two active-low resets. There is no APB CSR clock – the register block runs on aclk via a passthrough cpuif.

| Port | Dir | Width | Domain | Notes |
| --- | --- | --- | --- | --- |
| aclk | in | 1 | aclk | Host AXI + CAMs + scheduler |
| aresetn | in | 1 | aclk | Active-low async reset |
| dfi_clk | in | 1 | dfi_clk | DFI datapath + PHY pin bus |
| dfi_rstn | in | 1 | dfi_clk | Active-low async reset |
| init_done_o | out | 1 | aclk | Init sequencer done (STATUS) |

## 4.4	Register cpuif (PeakRDL passthrough)

The register block pumice_csr is generated from rtl/macro/pumice_csr.rdl (via bin/peakrdl_generate.py). The top presents the raw PeakRDL passthrough cpuif – an SoC bridges its APB/AXI-Lite onto these signals.

| Port | Dir | Width |
| --- | --- | --- |
| s_cpuif_req | in | 1 |
| s_cpuif_req_is_wr | in | 1 |
| s_cpuif_addr | in | CSR_ADDR_W |
| s_cpuif_wr_data | in | 32 |
| s_cpuif_wr_biten | in | 32 |
| s_cpuif_req_stall_wr | out | 1 |
| s_cpuif_req_stall_rd | out | 1 |
| s_cpuif_rd_ack | out | 1 |
| s_cpuif_rd_err | out | 1 |
| s_cpuif_rd_data | out | 32 |
| s_cpuif_wr_ack | out | 1 |
| s_cpuif_wr_err | out | 1 |

All config (timings, DFI phases, page policy, address map) is programmed through this bus by name via the generated dv/tbclasses/pumice_regmap.py; pumice_top fans hwif_out.* into pumice_core’s config ports. See section 4 of this MAS (or the RDL) for the register map.

## 4.5	AXI4 Slave Port List

Standard AXI4 slave, single ID width IW, data width DW (= DFI word). All five channels are present (AW/W/B/AR/R) with the usual side-band (awcache/awprot/awqos/awregion/awlock/awuser and the AR equivalents). s_axi_awuser / s_axi_aruser / s_axi_wuser are single-bit at the top. Widths:

- Address: s_axi_awaddr / s_axi_araddr = AW

- Data: s_axi_wdata / s_axi_rdata = DW; s_axi_wstrb = SW

- ID: s_axi_awid / s_axi_bid / s_axi_arid / s_axi_rid = IW

The exact per-signal declaration is the AW/W/B/AR/R block in rtl/top/pumice_top.sv. AXI side-band awqos/arqos/awregion/arregion are carried through the burst splitters but do not drive scheduler priority.

## 4.6	DFI 2.1 Master Port List

The DFI pin bus is the wide (per-phase x DFI_RATE) form. Command control strobes are 1-bit-per-phase; address/bank are ROW_WIDTH/$clog2(NUM_BANKS) per phase.

| Port | Dir | Width |
| --- | --- | --- |
| dfi_address_o | out | DFI_ADDR_BUS_W |
| dfi_bank_o | out | DFI_BANK_BUS_W |
| dfi_cas_n_o | out | DFI_CTRL_BUS_W |
| dfi_ras_n_o | out | DFI_CTRL_BUS_W |
| dfi_we_n_o | out | DFI_CTRL_BUS_W |
| dfi_cs_n_o | out | DFI_CS_BUS_W |
| dfi_odt_o | out | DFI_CS_BUS_W |
| dfi_wrdata_o | out | DFI_DATA_WIDTH |
| dfi_wrdata_en_o | out | DFI_EN_WIDTH |
| dfi_wrdata_mask_o | out | DFI_STRB_WIDTH |
| dfi_rddata_en_o | out | DFI_EN_WIDTH |
| dfi_rddata_i | in | DFI_DATA_WIDTH |
| dfi_rddata_valid_i | in | DFI_VALID_WIDTH |
| dfi_init_start_o | out | 1 |
| dfi_init_complete_i | in | 1 |

Notes:

- The DDR2 ras/cas/we strobes are present in all builds. For LPDDR2 (PHY_TIMING.memtype = 1) the command rides the CA bus packed onto dfi_address_o and these strobes idle; the memtype is a runtime field, so the same SV header serves both flavors.

- The DFI bus is driven in the dfi_clk domain. a7ddrphy (LiteDRAM’s Xilinx 7-series PHY, the board target) handles the internal DFI-to-DRAM gearing; the controller does not drive per-phase read gearing beyond rd_phase.

## 4.7	SystemVerilog Header Snippet

The canonical source is rtl/top/pumice_top.sv – the module declaration there is authoritative. The parameter block above and the port groups (cpuif, AXI4, DFI 2.1) are a human-readable mirror of that file.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Clocks and Reset

## 5.1	Clock Domains

The controller has exactly two clock domains. Everything from the host AXI face through the command scheduler runs on aclk; the DFI datapath and PHY pin bus run on dfi_clk. The register block pumice_csr runs on aclk (there is no separate APB CSR clock).

| Clock | Polarity | Domain members |
| --- | --- | --- |
| aclk | Posedge | pumice_csr, pumice_axi4_ifc (AXI slave, burst splitters, both CAMs), pumice_mem_cmd_scheduler (arbiter, bank/global timers, refresh, init, mode register) |
| dfi_clk | Posedge | pumice_dfi_layer command path, write serializer, read aligner; the DFI 2.1 pin bus |

aclk and dfi_clk are independent. The DFI-word datapath unit means one FIFO word equals one dfi_clk cycle, so the crossing is bubble-free at rate.

## 5.2	Reset Topology

| Reset | Polarity | Type | Domain |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert | aclk |
| dfi_rstn | Active-low | Async assert | dfi_clk |

Each domain has its own reset. pumice_csr takes rst = ~aresetn (the PeakRDL block uses active-high internally). All flops use the repo reset macros (reset_defs.svh); SRAMs inside the CAMs have no reset port. The SoC’s PMU is expected to drive clean, de-glitched resets – there is no reset-glitch filter in the controller.

## 5.3	CDC

There is exactly one clock-domain crossing in the whole controller, and it lives in pumice_dfi_cdc inside pumice_dfi_layer. It is built from async gaxi FIFOs only (N_FLOP_CROSS = 2 by default). Four things cross it:

| Stream | Direction | Payload |
| --- | --- | --- |
| Command | aclk -> dfi_clk | {op, rank, bank, row, col, ap} (CMD_DW) |
| Write data | aclk -> dfi_clk | {last, strb, data} DFI-word (WD_DW) |
| Read data | dfi_clk -> aclk | {last, resp, data} DFI-word (RD_DW) |
| Init handshake | both | init_start out, init_complete back |

There is no APB-to-MC CSR crossing, no quiet-point override-staging crossing, and no CDC in the AXI4 datapath – pumice_axi4_ifc runs entirely on aclk. If the SoC’s AXI master is on a different clock, an external clock converter is required upstream. The register block also runs on aclk, so CSR writes take effect combinationally through hwif_out.* into the config ports of pumice_core (no staging register). Timing/phase/policy fields should be programmed while the controller is idle (before init_start, or at a quiet point) since they feed the timers and phase-packers directly.

## 5.4	Reset / Init Sequence

On power-on:

- aresetn and dfi_rstn are asserted (both low). The PHY drives dfi_init_complete_i = 0.

- The SoC deasserts both resets. pumice_csr becomes R/W-able on aclk; software programs the timing, DFI-phase, page-policy, and address-map fields by name (never by hardcoded offset – see dv/tbclasses/pumice_regmap.py). The controller idles with init_done_o = 0; AXI traffic is held off.

- The init_sequencer (inside pumice_mem_cmd_scheduler) drives dfi_init_start_o and walks the per-memtype JEDEC MRS init sequence, emitting init commands into the same arbiter path as normal traffic and waiting on the programmed init timings (t_init_wait, t_dll_wait, t_mrd_wait, t_rp_wait, t_rfc_wait) plus dfi_init_complete_i from the PHY. The mode_register shadow captures the MRS writes and exposes CL/CWL/BL to the DFI layer.

- On completion the sequencer asserts init_done_o; the controller begins honoring host AXI traffic and the refresh controller is enabled.

Because the CSR block and the scheduler share aclk, no inter-clock handshake gates step 2; software only needs both resets released and the register bus alive.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Top-Level Integration (pumice_core / pumice_top)

Modules: pumice_core.sv, pumice_top.sv Location: rtl/top/ Category: Integration (structural wiring) Status: Implemented

Note: the early SWAG used a five-*_macro hierarchy (pumice_core_macro, axi_frontend_macro, command_scheduler_macro, data_path_macro, dfi_v21_interface_macro). That naming is retired. The live controller is a three-layer stack instantiated by pumice_core, wrapped by pumice_top (which adds the PeakRDL CSR block) and optionally by pumice_top_geared (which adds a host-width AXI dwidth shim).

## 6.1	Purpose

The controller top is structural wiring only — every behavioral statement lives in a FUB or a layer macro. Two files make up the top:

- pumice_core — wires the three functional layers on their two clocks and exposes host AXI4 plus the DFI 2.1 pin bus. Config arrives on ports.

- pumice_top — instantiates pumice_core plus the PeakRDL-generated pumice_csr register block, and drives every core config port by name from the CSR hwif_out.* decode. It exposes the register cpuif, host AXI4, and the DFI pin bus.

An optional third file, pumice_top_geared, wraps pumice_top with a free HOST_AXI_DATA_WIDTH and inserts the repo’s formally-verified axi4_dwidth_converter_wr / axi4_dwidth_converter_rd between a host-width AXI slave and the fixed-DW core. HOST == DW is a generate bypass (bit-identical). See docs/AXI_DRAM_GEARING_SCOPE.md.

## 6.2	pumice_core — the three layers

pumice_core instantiates exactly three layer modules and the nets between them. There is no FSM, no CSR decode, and no arithmetic beyond the packed-bus assign for the command word.

pumice_core
├── u_ifc   : pumice_axi4_ifc          (host AXI + wr/rd CAMs)          [aclk]
├── u_sched : pumice_mem_cmd_scheduler (arbiter + timers + refresh/init)[aclk]
└── u_dfi   : pumice_dfi_layer         (single async CDC + DFI datapath)[aclk→dfi_clk]

- Host AXI, the scheduler, and both CAMs run on aclk (aresetn).

- The DFI phase-packer and PHY interface run on dfi_clk (dfi_rstn).

- The one clock crossing in the whole controller is inside pumice_dfi_layer (async gaxi FIFOs only). pumice_dfi_layer is instantiated with .ctl_clk(aclk) / .ctl_rstn(aresetn) on the control side and .dfi_clk / .dfi_rstn on the PHY side.

The internal data unit is the DFI word (DFI_DATA_WIDTH = DRAM_BEAT_WIDTH * DFI_RATE, default 128). The host AXI data width equals the DFI word; a host that runs a different AXI width uses the external pumice_top_geared shim (a separate edge concern — the core does not gear internally).

### 6.2.1	Layer instantiation inventory (pumice_core)

| Instance | Module | Count | Clock | Role |
| --- | --- | --- | --- | --- |
| u_ifc | pumice_axi4_ifc | 1 | aclk | Host AXI4 face; wr-data CAM + rd-cmd CAM |
| u_sched | pumice_mem_cmd_scheduler | 1 | aclk | Command arbiter + bank/global timers + refresh + init + mode-register shadow |
| u_dfi | pumice_dfi_layer | 1 | aclk → dfi_clk | Single async CDC + DFI cmd/wr/rd datapath |

There are no generate fan-out blocks in pumice_core. Per-(rank, bank) fan-out (the bank timers) happens one level down, inside pumice_mem_cmd_scheduler.

### 6.2.2	Inter-layer nets

pumice_core declares and wires four net groups:

- Scheduler ↔ IFC CAM ports — the per-bank scheduler lookup buses (w_wr_lu_*, w_rd_lu_*), the oldest-entry ports (w_wr_old_*, w_rd_old_*), the write commit handshake (w_wr_commit_*), and the read issue handshake (w_rd_issue_*). N_LU = NUM_BANKS (one lookup per bank).

- IFC commit-data → DFI wrdata (w_cm_*: valid/ready/data/strb/last) and DFI rddata → IFC rd-return (w_ret_*: valid/ready/data/resp/last).

- Scheduler command stream → DFI — the abstract command {op, rank, bank, row, col, ap}, flattened into w_cmd_data (width CMD_DW = 4 + RKW + BKW + ROW_WIDTH + COL_WIDTH + 1) by a single assign.

- Init handshake — w_init_start / w_init_complete between the scheduler’s init sequencer and the DFI layer’s PHY-init side.

Note the parameter mapping at the IFC boundary: pumice_core passes .BL(BL / DFI_RATE) to pumice_axi4_ifc (the IFC/CAM view of burst length is in DFI words), while pumice_dfi_layer receives the full .BL(BL) (DRAM beats).

## 6.3	pumice_top — CSR + by-name config

pumice_top adds the register block and the config fan-out. It does two things:

- Instantiates pumice_csr (PeakRDL passthrough cpuif) with the hwif_in / hwif_out struct pair. hwif_in is currently tied to '{default:'0} — status/observability readback is a follow-up; config-drive is wired first.

- Instantiates pumice_core and drives every config port by name from the decoded hwif_out.* fields. Representative mappings:

| Core port | CSR field (hwif_out.*) |
| --- | --- |
| memtype_i | PHY_TIMING.memtype (0 = DDR2, 1 = LPDDR2) |
| page_policy_i | REFRESH_TUNING.page_policy_or |
| bank_lsb_i | ADDR_MAP.bank_lsb |
| hash_en_i | ADDR_MAP.hash_en |
| hash_seed_i | ADDR_MAP.hash_seed |
| t_rcd_i/t_rp_i/t_ras_i/t_rc_i | TIMINGS_RC_RCD_RP_RAS.* |
| t_wr_i | TIMINGS_CL_CWL_WR.tWR |
| t_rtp_i/t_rtw_i | TIMINGS_RTP_RTW.* |
| t_faw_i/t_rrd_i/t_wtr_i/t_ccd_i | TIMINGS_RRD_FAW_WTR_CCD.* |
| t_refi_i | TIMINGS_RFC_REFI.tREFI |
| refresh_burst_i | PHY_TIMING.refresh_burst |
| t_init_wait_i/t_dll_wait_i | INIT_TIMING0.* |
| t_mrd_wait_i/t_rp_wait_i/t_rfc_wait_i | INIT_TIMING1.* |
| rd_phase_i/wr_phase_i | DFI_PHASE.* (sliced to [PHW-1:0]) |
| t_phy_wrlat_i/t_rddata_en_i | PHY_TIMING.* |

The CSR block is clocked on aclk with .rst(~aresetn) (PeakRDL uses an active-high reset; the top inverts aresetn). See rtl/macro/pumice_csr.rdl for the full field list.

## 6.4	What the top does not do

- No combinational logic beyond the single command-word assign in pumice_core and the by-name field selects in pumice_top.

- No CSR field expansion in pumice_core (that is pumice_csr’s job; the top only reads decoded hwif_out fields).

- No clock gating (a synthesis-script concern).

- No reset synchronization in the top — each layer takes its own domain reset (aresetn for aclk logic, dfi_rstn for the PHY domain); the CDC inside pumice_dfi_layer owns the cross-domain reset discipline.

This intentional poverty makes the top the obvious place for the structural-only sanity check: every line is an instantiation, a net declaration, or a port/field mapping.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	pumice_axi4_ifc (AXI4 front-end macro)

Module: pumice_axi4_ifc.sv Location: rtl/macro/ Category: Layer-1 macro (the AXI front-end of pumice_core) FUBs bundled: burst splitters + 2 intakes + 2 CAMs

## 7.1	Purpose

“Host AXI4 to CAM-cached DRAM requests.” The first of the three core layers. It bolts the shared AXI burst splitters onto dumb 1:1 intakes, holds the write-data CAM (write buffer + snarf source) and the read-command CAM (read reorder buffer), presents the host AXI4 face, and exposes the scheduler lookup/oldest/commit/issue ports and the DFI wr-commit-data / rd-return ports outward to the other two layers.

host AXI4 -> [wr/rd splitter] -> pumice_wr_intake -> pumice_wr_data_cam
                              -> pumice_rd_intake -> pumice_rd_cmd_cam

Runs entirely on aclk. This replaces the old axi_frontend_macro (axi_intake / addr_mapper / wr_cmd_cam / rd_cmd_cam / wr2rd_forward) – those names are retired.

## 7.2	FUBs

| FUB | Role |
| --- | --- |
| axi_master_wr_splitter / axi_master_rd_splitter | Shared AMBA IP; split each host burst at DRAM-burst-byte boundaries (ALIGN_MASK = BL*(DRAM_BEAT_WIDTH/8) - 1), one DRAM burst per split command |
| pumice_wr_intake | axi4_slave_wr + AW-meta FIFO + wr-data FIFO + addr_mapper; emits (bank,row,col,id) push plus a wr-data stream; consumes commit-done for B response |
| pumice_rd_intake | axi4_slave_rd + addr_mapper + snarf probe; emits (bank,row,col,id) push; probes the write CAM and, on a snarf hit, streams the R response from the write CAM SRAM; otherwise consumes the drained DFI return |
| pumice_wr_data_cam | Write CAM keyed on (bank,row) + write-data SRAM; fill / commit-drain / snarf movers; oldest + N_SCHED_LU lookup + commit ports; commit-data out to the DFI write path |
| pumice_rd_cmd_cam | Read CAM keyed on (bank,row) + read-return SRAM; return-fill / drain movers; oldest + N_SCHED_LU lookup + issue ports; DFI-return in, drain out to the rd intake |

Address decode (addr_mapper) is driven by bank_lsb_i / hash_en_i / hash_seed_i (the ADDR_MAP CSR); there is no scheme selector. See section 4 of this MAS for the address-map field semantics.

## 7.3	External Boundaries

- Upstream: the host AXI4 slave port (SoC-facing).

- To pumice_mem_cmd_scheduler: for each CAM, the oldest_* snapshot, the N_SCHED_LU parallel sched_lu_* lookup ports (valid/bank/row in, hit/slot/col/id/age out), and the write commit / read issue handshakes. The scheduler drives these on aclk.

- To pumice_dfi_layer: write commit-data out (wr_cm_rd_*: valid/ready/data/strb/last) to the DFI write serializer; read DFI return in (rd_dfi_ret_*: valid/ready/data/resp/last) from the DFI read aligner.

- Internal: the read intake’s snarf probe hits pumice_wr_data_cam directly (snarf_probe_* -> snarf_hit/snarf_rd_*); the write CAM’s commit_done_* retires the AW-meta FIFO entry in pumice_wr_intake to emit the B response.

The busy_o output is the OR of the four sub-block busy flags.

## 7.4	De-FSM’d CAMs

Both CAMs store burst data in an SRAM and read it back with a streaming engine that is FIFO-fed / oldest-pick beat-counter driven – there is no active/slot state machine. The r_fdone fill-complete flag gates when a slot is schedulable (and, on the write side, snarfable). The write CAM’s snarf mover is the sole read-your-write forwarding path; there is no separate forwarder.

## 7.5	Tests

FUB-level unit tests exist for each intake and each CAM (dv/tests/fub/), and a wrapper-level test drives pumice_axi4_ifc through the AXI4 BFMs (AXI4MasterWrite/AXI4MasterRead + AXI4Sequence). Drive AXI traffic through the BFMs – never hand-poke s_axi_*.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	pumice_mem_cmd_scheduler (command scheduler macro)

Module: pumice_mem_cmd_scheduler.sv Location: rtl/macro/ Category: Layer-2 macro (the decision layer of pumice_core) FUBs bundled: arbiter + bank/global timers + refresh + init + mode reg + cmd FIFO

## 8.1	Purpose

“What command should we issue this cycle?” – the decision-making layer. It wires the command arbiter to the per-bank safe timers, the global (bus/rank) turnaround timers, the refresh controller, the init sequencer plus mode-register shadow, and an output command FIFO. It emits a single abstract DRAM command stream {op, rank, bank, row, col, ap} for the DFI layer to phase-pack.

The scheduler is single-issue, PHY / nphases-agnostic, and runs on a single controller clock (aclk). The CAM sched-lookup / oldest / commit / issue ports are external – the CAMs live in pumice_axi4_ifc. This replaces the old command_scheduler_macro (scheduler / xbank_timers / page_predictor / powerdown_ctrl etc.); those names are retired.

## 8.2	FUBs

| FUB | Role |
| --- | --- |
| pumice_cmd_arbiter | The single pick core. Picks one command per cycle from the CAM lookups + oldest snapshots under page policy; open-page decision is inline here (no separate predictor). Drives the write commit / read issue handshakes and the per-command evt_* strobes; emits the abstract command. |
| pumice_bank_timers | Stamps a bank_timer per (rank,bank) and fans the evt_* strobes; produces bank_act/rdwr/pre_ready, bank_row_active, bank_open_row. |
| bank_timer | FSM-free per-bank JEDEC safe timers (tRCD/tRP/tRAS/tRC/tWR/tRTP) + a row-open register + a single auto-precharge bit. Combinational readiness off the countdown timers. |
| global_timers | Cross-bank/rank turnaround windows: tFAW + tRRD per rank, tWTR/tRTW/tCCD global. Produces _ok window flags. |
| refresh_ctrl | tREFI down-counter + 8-deep postpone; refresh_req/refresh_drain to the arbiter, refresh_grant back. Enabled after init. |
| init_sequencer | DDR2 + LPDDR2 JEDEC MRS init microprogram. Drives dfi_init_start_o, emits init commands into the arbiter, and writes the MR shadow. Gates traffic until init_done. |
| mode_register | MR shadow updated by the init MRS writes; decodes CL / CWL / BL (and AL / drive-strength / ODT). Supplies cl_o/cwl_o/bl_o to the DFI layer. |
| gaxi_fifo_sync | Output command FIFO (CMD_FIFO_DEPTH), packs {ap, col, row, bank, rank, op} (CMD_W bits) into the abstract command stream to the DFI layer. |

## 8.3	FSM-Free Bank Timing

bank_timer is deliberately stateless-in-the-control-sense: preset / decrement countdown timers (rcd/ras/rc/rp/preblk), a row-open register, and one auto-precharge bit – no multi-state bank machine. The per-command safe_act/rd/wr/pre outputs are combinational off the timers with a single register stage, so the arbiter reads readiness with one cycle of latency. This replaced the old double-registered 3-state bank FSM (retired bank_machine), which caused refresh-vs-ACT and column-vs-PRE hazards. pumice_bank_timers stamps the timer per (rank,bank).

## 8.4	External Boundaries

- CAM sched ports (to pumice_axi4_ifc): for each of write and read, the N_LU = NUM_BANKS lookup buses (lu_valid/bank/row out; hit/slot/col/id/age in), the oldest_* snapshot in, and the write commit_* / read issue_* handshake out.

- Command stream out (to pumice_dfi_layer): cmd_valid/ready + {op, rank, bank, row, col, ap}.

- Mode-register shadow out: cl_o / cwl_o / bl_o to the DFI layer.

- Init handshake: dfi_init_start_o out and dfi_init_complete_i in (routed to/from the PHY via the DFI layer); init_done_o status out.

- Config in: the JEDEC timings (tRCD/tRP/tRAS/tRC/tWR/tRTP/tFAW/tRRD/tWTR/ tRTW/tCCD/tREFI), refresh_burst, the init waits (t_init_wait/t_dll_wait/t_mrd_wait/t_rp_wait/t_rfc_wait), page_policy_i, and memtype_i – all delivered from the CSR by name.

busy_o asserts while not init-done, on a pending refresh, while a command is buffered in the FIFO, or while any bank row is active.

## 8.5	Tests

The scheduler has a macro-level test that exercises the arbiter against the two real CAMs – combinational pickers must be macro-tested, since registered feedback latency can hide double-issue hazards that a FUB-only test misses. Each constituent FUB also has its own unit test in dv/tests/fub/.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Data Path (no standalone macro)

Category: Distributed across pumice_axi4_ifc and pumice_dfi_layer Status: the old data_path_macro (wr_beat_sequencer + rd_cl_aligner) is retired – there is no data_path_macro.sv in the live tree.

## 9.1	Purpose

“Move bytes between the host AXI buffers and the DFI data lanes.” In the rearchitected core this is not a separate macro. The data path is split across two of the three core layers:

- The burst buffers live in the two CAMs inside pumice_axi4_ifc (section 2.1). Write data is buffered in pumice_wr_data_cam’s SRAM; read return data is buffered in pumice_rd_cmd_cam’s SRAM. Both use de-FSM’d streaming read engines rather than a beat-sequencer FSM.

- The DFI-clock serialize / align stages live in pumice_dfi_layer (section 2.4): pumice_dfi_wr_serializer presents write data on dfi_wrdata at t_phy_wrlat, and pumice_dfi_rd_aligner captures dfi_rddata at t_rddata_en and reassembles DFI words.

The controller-to-PHY data crossing is carried as whole DFI words through the single CDC in pumice_dfi_layer (section 1.3), so there is no separate data-path clock boundary to describe.

## 9.2	Write Path (host -> DRAM)

- pumice_wr_intake streams W beats (DFI-word granular {data, strb, last}) into pumice_wr_data_cam’s SRAM; r_fdone marks the burst fully filled.

- When the scheduler commits the write slot, the CAM’s commit-drain mover streams the burst out on the wr_cm_rd_* port (data/strb/last) into the DFI layer’s write FIFO.

- pumice_dfi_wr_serializer (on dfi_clk) drives dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask at t_phy_wrlat after the WR fire strobe from the command path. AXI wstrb = 1 (write this byte) maps to the DFI mask as-carried by the CAM strobe payload.

The write CAM’s SRAM is also the snarf source (read-your-write forwarding): pumice_rd_intake probes it and, on a hit, streams the R response straight from the write SRAM without a DRAM round-trip.

## 9.3	Read Path (DRAM -> host)

- When the scheduler issues the read slot, pumice_dfi_rd_aligner (on dfi_clk) drives dfi_rddata_en at t_rddata_en after the RD fire strobe, captures dfi_rddata when dfi_rddata_valid asserts, and packs whole DFI words ({data, resp, last}) into the read FIFO.

- The FIFO crosses to aclk (the single CDC) and lands as the dfi_ret_* stream into pumice_rd_cmd_cam’s return-fill mover, which writes the read SRAM.

- The CAM’s drain mover streams the buffered burst out oldest-first, gated on data-ready, back through pumice_rd_intake to the AXI R channel.

## 9.4	Why No Beat-Sequencer FSM

The old wr_beat_sequencer / rd_cl_aligner FUBs carried an active/slot state latch and per-beat control FSM. The live CAMs replace that with FIFO-fed / oldest-pick beat counters and a r_fdone fill flag – fewer control-path hazards and a cleaner streaming shape. The only genuinely PHY-timed logic (t_phy_wrlat / t_rddata_en alignment) is the small serializer / aligner pair in the DFI layer.

## 9.5	Tests

Covered by the CAM FUB tests and the DFI-layer FUB tests (pumice_dfi_wr_serializer, pumice_dfi_rd_aligner) in dv/tests/fub/, plus the pumice_axi4_ifc wrapper test for the end-to-end buffered data flow.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	pumice_dfi_layer (DFI v2.1 interface macro)

Module: pumice_dfi_layer.sv Location: rtl/macro/ Category: Layer-3 macro (the PHY-facing layer of pumice_core) FUBs bundled: single CDC + command path + write serializer + read aligner

## 10.1	Purpose

“Translate the internal command + data streams into DFI v2.1 wires, and hold the one clock crossing.” This layer owns the JEDEC command encoding for DDR2/LPDDR2, the multi-phase pack, and the single controller-to-PHY clock crossing. It presents the controller-clock command / wrdata / rddata streams on one side and the DFI 2.1 pin bus on the other.

Swap THIS macro when moving to DFI v3+ for newer DRAM generations – the other two core layers (AXI front-end, scheduler) are DFI-version-agnostic.

This replaces the old dfi_v21_interface_macro (dfi_cmd_formatter + dfi_signal_pack) and the retired gear_dfi block; the CDC that used to be implied elsewhere is now explicitly this layer’s pumice_dfi_cdc.

## 10.2	FUBs

| FUB | Clock | Role |
| --- | --- | --- |
| pumice_dfi_cdc | both | The single controller/PHY crossing – async gaxi FIFOs only. Carries cmd (aclk->dfi_clk), wrdata (aclk->dfi_clk), rddata (dfi_clk->aclk), plus the init handshake. |
| pumice_dfi_cmd_path | dfi_clk | Command FIFO -> DFI command bus. Uses dfi_cmd_formatter (+ dfi_signal_pack) to phase-place the JEDEC command on wr_phase / rd_phase; drives address/bank/ras/cas/we/cs/odt; emits wr_fire / rd_fire strobes. |
| pumice_dfi_wr_serializer | dfi_clk | Write-data FIFO -> dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask at t_phy_wrlat after wr_fire. |
| pumice_dfi_rd_aligner | dfi_clk | Drives dfi_rddata_en at t_rddata_en after rd_fire; captures dfi_rddata on dfi_rddata_valid; packs DFI words into the read FIFO. |

The internal datapath unit is the DFI word (dfi_wrdata width, all DFI_RATE phases). One FIFO word equals one DFI cycle, so the datapath is bubble-free at rate.

## 10.3	External Boundaries

- Controller side (ctl_clk = aclk): cmd_* in (from the scheduler, packed {op,rank,bank,row,col,ap}), wd_* in (write commit-data from pumice_wr_data_cam, {last,strb,data} DFI-word), rd_* out (read return to pumice_rd_cmd_cam, {last,resp,data}), and init_start_i / init_complete_o.

- PHY side (dfi_clk): the full DFI 2.1 pin bus – dfi_address / dfi_bank / dfi_cas_n / dfi_ras_n / dfi_we_n / dfi_cs_n / dfi_odt, dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask, dfi_rddata_en / dfi_rddata / dfi_rddata_valid, and dfi_init_start / dfi_init_complete.

- Config in (dfi_clk domain): memtype_i, rd_phase_i / wr_phase_i, t_phy_wrlat_i, t_rddata_en_i – delivered from the CSR by name.

## 10.4	Multi-Phase Pack Convention

For DFI_RATE = N, every DFI control bus is per-phase x N wide (for example, dfi_address_o = ROW_WIDTH * DFI_RATE, dfi_cs_n_o = NUM_RANKS * DFI_RATE). The command path places the active JEDEC command on the wr_phase / rd_phase slot as programmed by the DFI_PHASE CSR and drives NOP-equivalent idle on the other phases. The scheduler never sees the phase dimension; the command path absorbs all phase placement. dfi_rddata read gearing beyond rd_phase is left to the PHY (a7ddrphy handles its internal read gearing).

## 10.5	LPDDR2 Command Encoding

For memtype_i = LPDDR2, dfi_cmd_formatter takes the memtype branch and emits bit-exact JESD209-2F CA-bus commands (Table 60): the 10-bit CA bus over 2 edges, packed as a flat 20-bit word on dfi_address, with ras/cas/we idle. See rtl/LPDDR2_CA_ENCODING.md for the transcription. DDR2 uses the classic ras/cas/we/cs strobe encoding on the same pins.

## 10.6	Tests

Each FUB has its own unit test in dv/tests/fub/ (pumice_dfi_cdc, pumice_dfi_cmd_path, pumice_dfi_wr_serializer, pumice_dfi_rd_aligner), plus the layer-level test that drives the full controller-to-PHY path against the DFI slave BFM.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	AXI4 Slave + Intakes (axi4_slave_wr / axi4_slave_rd)

Modules: axi4_slave_wr / axi4_slave_rd (repo AMBA IP) inside pumice_wr_intake / pumice_rd_intake Location: intakes in rtl/fub/; slaves in the shared AMBA library Category: FUB (intakes) Parent: pumice_axi4_ifc Status: Implemented

Rearchitected: the SWAG had a single axi4_slave_fub that also owned transaction state and an R-emit FSM. That block is retired. In the live design the AXI4 protocol engine is the repo’s axi4_slave_wr / axi4_slave_rd skid IP, and each is wrapped by a thin intake FUB (pumice_wr_intake, pumice_rd_intake) that is FIFO-based and FSM-free. All in-flight transaction state lives downstream in the two CAMs (see ch02/04, ch02/05).

## 11.1	Purpose

The host-facing AXI4 protocol handling is split by direction and factored into two layers each:

- axi4_slave_wr / axi4_slave_rd — the repo’s standard AXI4 slave skid buffers. They do the channel handshakes, provide SKID_DEPTH_* buffering, and present a clean post-skid “fub” face (fub_axi_*).

- pumice_wr_intake / pumice_rd_intake — the pumice-specific intake FUBs that sit on the post-skid face. They decode the burst address via addr_mapper, push a decoded command downstream to a CAM, stream write/read data through FIFOs, and return B/R responses.

Ahead of both intakes, pumice_axi4_ifc places the repo’s axi_master_wr_splitter / axi_master_rd_splitter so that each burst delivered to an intake is exactly one DRAM burst (aligned to DRAM_BURST_BYTES = BL * DRAM_BEAT_WIDTH/8). The intakes therefore assume “one AXI burst == one DFI burst” and carry no burst-splitting logic.

## 11.2	pumice_wr_intake — structure

Exactly three things plus the address decoder, per the locked µarch spec:

- axi4_slave_wr (u_slave_wr) — AXI protocol / skid buffering. Parameters SKID_DEPTH_AW=2, SKID_DEPTH_W=4, SKID_DEPTH_B=2.

- AW-meta FIFO (u_aw_meta_fifo) — a gaxi_fifo_sync, depth AW_FIFO_DEPTH (4), width AWM_W = 1 + IW + AW, capturing {err, awid, awaddr} per burst.

- wr-data FIFO (u_wr_data_fifo) — a gaxi_fifo_sync, depth WDATA_FIFO_DEPTH (16), width WD_W = 1 + SW + DW, capturing {wlast, wstrb, wdata} per AXI beat.

- addr_mapper (u_addr_mapper) — combinational decode of the FIFO-head address into {rank, bank, row, col} for the downstream aw_push.

Plus a B-response FIFO (u_b_fifo), depth B_FIFO_DEPTH (4), width B_W = 2 + IW.

### 11.2.1	Write data flow

- AW intake — fub_awvalid writes {err, awid, awaddr} into the AW-meta FIFO. fub_awready = w_awm_wr_ready (accept while the FIFO has room).

- W intake — fub_wvalid writes {wlast, wstrb, wdata} into the wr-data FIFO; fub_wready = w_wd_wr_ready. AW and W are decoupled — each has its own FIFO.

- Decode + push — the AW-meta FIFO head address is decoded by addr_mapper; the decoded {bank, row, col, id, err} is presented on aw_push_* and valid whenever the AW-meta FIFO is non-empty. It pops on the downstream aw_push_ready_i handshake. (aw_push_rank_o is driven but left unconnected at the IFC — single-rank pick.)

- wr-data pop — the wr-data FIFO head is exposed on wdata_* and drained by the wr-data CAM (wdata_ready_i) in commit order.

- B response — a completed commit strobes wr_done_valid_i / wr_done_id_i from the CAM; that pushes {resp, id} into the B FIFO, which drives the AXI B channel.

### 11.2.2	Ragged-burst handling

The intake computes w_aw_err = ((awlen+1)*GEAR != BL) where GEAR = AXI_DATA_WIDTH / DRAM_BEAT_WIDTH. On a ragged burst the intake self-generates a SLVERR B response at the aw_push pop (err takes priority over a wr_done), and the downstream CAM drops the illegal command. A simulation $error fires when RAGGED_ASSERT != 0 (a guardrail under translate_off); a companion assertion catches a B-push collision between the err path and a same-cycle wr_done.

## 11.3	pumice_rd_intake — structure

Mirror of the write intake, plus the read snarf path:

- axi4_slave_rd (u_slave_rd) — AXI protocol / skid buffering (SKID_DEPTH_AR=2, SKID_DEPTH_R=4).

- addr_mapper (u_addr_mapper) — decode fub_araddr → {rank, bank,    row, col} at the AR inlet (combinational).

- Snarf probe — the decoded {bank, row, col, id, len} is presented on the snarf_probe_* ports every cycle fub_arvalid is high; the wr-data CAM returns a combinational snarf_hit_i.

- Order FIFO (u_order_fifo) — a gaxi_fifo_sync, depth ORDER_FIFO_DEPTH (8), width ORD_W = 1 + IW, storing {source, id} per admitted read to preserve AR order (SRC_SNARF=1, SRC_DFI=0).

- Source arbiter — a combinational mux selecting the snarf stream or the DFI read-return stream by the order-FIFO head’s source tag.

- rd-data FIFO (u_rd_data_fifo) — a gaxi_fifo_sync, depth RD_FIFO_DEPTH (16), width RD_W = 2 + 1 + IW + DW = {resp, last, id,    data}, feeding the AXI R channel.

### 11.3.1	Read admission and ordering

- An AR is admitted (w_can_admit) when the order FIFO has room and, for a MISS, ar_push_ready_i is asserted. fub_arready = w_can_admit.

- On a HIT (snarf_hit_i) the read is tagged SRC_SNARF, snarf_accept_o fires, and no ar_push is emitted (the data is streamed from the write CAM’s SRAM — DRAM is stale for the in-flight write).

- On a MISS the read is tagged SRC_DFI and ar_push_valid_o fires with the decoded {bank, row, col, id} toward the scheduler / rd CAM.

- The source arbiter drains one burst at a time in AR order: the order-FIFO head’s source selects which input stream is connected to the rd-data FIFO, and the head is popped only when the last beat of that burst is accepted. So a snarf-ready read still waits behind an earlier DFI read at the head (in-order R per the AXI contract).

## 11.4	External interface

Both intakes present a full AXI4 write (AW/W/B) or read (AR/R) slave face plus:

| Port group | Direction | Purpose |
| --- | --- | --- |
| bank_lsb_i / hash_en_i / hash_seed_i | in | ADDR_MAP config forwarded to addr_mapper |
| aw_push_* / ar_push_* | out | Decoded command to the wr/rd CAM |
| wdata_* (wr) | out | Write-data pop to the wr-data CAM |
| wr_done_* (wr) | in | Commit-completion strobe from the CAM → B response |
| snarf_probe_* / snarf_hit_i / snarf_accept_o (rd) | in/out | Read-your-write probe into the wr CAM |
| snarf_rd_* (rd) | in | Snarf data stream from the wr CAM SRAM |
| dfi_rd_* (rd) | in | DFI read-return stream (MISS reads) from the rd CAM |
| busy_o | out | Any FIFO non-empty / slave busy |

## 11.5	Notes

- There is no AXI ID side table, no w_buf/b_fifo in the SWAG sense, and no R-emit FSM. Per-ID metadata that must survive into the DRAM layer is carried in the CAM entries; everything else is dropped at the intake.

- Outstanding-count limits are structural: writes are bounded by the wr-data CAM depth (NUM_ENTRIES), reads by the rd-cmd CAM depth plus the order FIFO depth. There are no MAX_PER_ID_* parameters.

- The intakes are strict-FIFO and back-pressure cleanly: an intake stalls its AXI channel when the relevant FIFO or the downstream CAM is full.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Address Mapper (addr_mapper)

Module: addr_mapper.sv Location: rtl/fub/ Category: FUB (combinational; no clock) Parent: pumice_wr_intake / pumice_rd_intake Status: Implemented

Rearchitected: the SWAG described a three-scheme decoder (ROW_MAJOR / BANK_INTERLEAVE / XOR_HASH) selected by a runtime scheme_active_i mux. That is retired. The live addr_mapper has one placement knob — bank_lsb_i — that slides the bank field through the (byte-stripped) column region, plus an optional bank XOR-hash. The classic schemes are now just settings of bank_lsb. There is no scheme selector, no SYNTH_* param, and no un-synthesized-scheme tie-off.

## 12.1	Purpose

addr_mapper decodes a flat AXI_ADDR_WIDTH-bit address into the DRAM-layer (rank, bank, row, col) tuple. It is pure combinational — no clock, no flop, no FSM. The output is valid the same cycle as the input. One instance sits at the head-address decode of each intake (pumice_wr_intake, pumice_rd_intake), so the CAMs store the decoded tuple rather than the raw AXI address.

The mapper is driven entirely from the ADDR_MAP CSR register (see rtl/macro/pumice_csr.rdl) via three runtime inputs; there are no per-scheme build parameters.

## 12.2	Parameters

| Parameter | Default | Purpose |
| --- | --- | --- |
| AXI_ADDR_WIDTH | 32 | Flat AXI byte-address width |
| NUM_RANKS | 1 | 1, 2, or 4 (rank field width KW) |
| NUM_BANKS | 8 | 4 or 8 (bank field width BW = $clog2(NUM_BANKS)) |
| ROW_WIDTH | 14 | Row field width RW |
| COL_WIDTH | 10 | Column field width CW |
| BYTE_OFFSET_WIDTH | 3 | log2(beat byte size); low bits stripped to word addr |

## 12.3	Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_addr_i | input | AXI_ADDR_WIDTH | Flat AXI byte address |
| bank_lsb_i | input | 5 | Bank-field LSB in the word address (ADDR_MAP.bank_lsb) |
| hash_en_i | input | 1 | Enable the bank XOR-hash (ADDR_MAP.hash_en) |
| hash_seed_i | input | 8 | XOR-hash seed (ADDR_MAP.hash_seed) |
| rank_o | output | $clog2(max(NR,2)) | Decoded rank |
| bank_o | output | $clog2(NUM_BANKS) | Decoded bank |
| row_o | output | ROW_WIDTH | Decoded row |
| col_o | output | COL_WIDTH | Decoded column |

## 12.4	The single-knob layout

The address is first stripped of its byte offset to a word address:

w_word = axi_addr_i[AXI_ADDR_WIDTH-1 : BYTE_OFFSET_WIDTH]   (zero-extended to 32b)

The bank field is BW bits wide and sits at bit position bank_lsb inside the word address. The column is split around the bank into a low part below it and a high part above it; the row and rank stack above the whole column region at invariant positions:

word address (low → high):
  [ col_lo(bank_lsb) | bank(BW) | col_hi(CW - bank_lsb) | row(RW) | rank(KW) ]

  col = { col_hi, col_lo }
  row LSB is always at bit CW + BW      (only the bank slides; the column
                                         auto-splits around it)

### 12.4.1	Field extraction (variable-base shifts/masks)

The RTL clamps bank_lsb to [0, COL_WIDTH] first (w_blsb), so col_hi’s width CW - bank_lsb never goes negative and the row/rank slices stay legal. Then, over 32-bit intermediates:

col_lo = w_word & ((1 << bank_lsb) - 1)
bank   = (w_word >> bank_lsb) & ((1 << BW) - 1)
col_hi = (w_word >> (bank_lsb + BW)) & ((1 << (CW - bank_lsb)) - 1)
row    = (w_word >> (CW + BW)) & ((1 << RW) - 1)
rank   = (w_word >> (CW + BW + RW)) & ((1 << KW) - 1)   // 0 when NUM_RANKS == 1
col    = col_lo | (col_hi << bank_lsb)

## 12.5	The classic schemes are just bank_lsb settings

| Legacy scheme | Equivalent setting |
| --- | --- |
| ROW_MAJOR | bank_lsb == COL_WIDTH — bank sits above the whole column |
| max BANK_INTERLEAVE | bank_lsb == log2(cols/burst) — minimal col_lo, so a burst’s column walk stays inside one bank while consecutive lines round-robin banks |
| partial interleave | any bank_lsb in between |
| XOR_HASH | hash_en == 1, folded on top of any placement above |

The default ADDR_MAP.bank_lsb = 0x0A = COL_WIDTH is therefore ROW_MAJOR. Software is expected to keep log2(cols/burst) <= bank_lsb <= COL_WIDTH so a DRAM burst’s column walk never crosses a bank boundary; the RTL clamp enforces the upper edge.

## 12.6	Bank XOR-hash

When hash_en_i is set, each bank bit is XOR-folded with two row slices and the seed (a genvar loop of width BW):

bank_hashed[i] = bank_raw[i] ^ row[i] ^ row[MID] ^ hash_seed[i]
  where MID = (i + BW < ROW_WIDTH) ? (i + BW) : (ROW_WIDTH - 1)   // clamped
w_bank = hash_en ? bank_hashed : bank_raw

This is the identical fold to the legacy XOR_HASH scheme and defeats power-of-two-stride hot-banking. hash_seed_i is runtime-modifiable, so a bring-up engineer can change the hash without rebuilding.

## 12.7	Mirror to the DV address model

This RTL is the bit-exact mirror of the DV-side address model; both must produce identical (rank, bank, row, col) for the same input and the same bank_lsb/hash_en/hash_seed. The mirror eliminates the class of bugs where the BFM decodes one way and the controller decodes another. It is exercised by the addr_mapper FUB test, which sweeps addresses and bank_lsb values and asserts bit-equality against the model.

## 12.8	Timing

Purely combinational. The longest path is the hash fold (a few XOR levels) plus the variable-base shift/mask extraction. At the board target this is comfortably within a cycle; if a future high-frequency target cannot meet timing, the mapper can be treated as a multi-cycle path (the placement knob only changes at quiet points).

## 12.9	Notes / flags

- bank_lsb_i is 5 bits wide, matching ADDR_MAP.bank_lsb[4:0].

- rank_o width uses $clog2(NUM_RANKS > 1 ? NUM_RANKS : 2) so a single-rank build still has a legal 1-bit port; rank_o is tied to '0 when NUM_RANKS == 1.

- There is no bg_field_pos / bank-group input (DDR2/LPDDR2 have no bank groups), no scheme_active_i, and no CSR pslverr path for unsupported schemes — those SWAG concepts are gone.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Read Command CAM (pumice_rd_cmd_cam)

Module: pumice_rd_cmd_cam.sv Location: rtl/fub/ Category: FUB Parent: pumice_axi4_ifc Status: Implemented (de-FSM’d streaming drain)

Rearchitected: the SWAG CAM was keyed by AXI ID and drove a match_pending vector with per-slot beat counters and an issue FSM. The live pumice_rd_cmd_cam is keyed by {bank, row} with a free-running age, exposes N_SCHED_LU parallel scheduler lookups plus an oldest port, and is a read reorder buffer: DRAM read data returns in issue order into per-entry SRAM slots and drains to the intake in AR (insert) order. The drain has no active/slot state latch — a burst beat-counter and the combinational oldest-valid pick are the only sequencing.

## 13.1	Purpose

pumice_rd_cmd_cam tracks every outstanding DRAM read (the MISS path — snarf hits never enter this CAM). It is the mirror of pumice_wr_data_cam on the issue side, but the data direction is opposite: data comes in from the DFI read return and drains out to pumice_rd_intake. There is no snarf port (that is a write-CAM concept).

It acts as a reorder buffer because DRAM read data returns in the order the scheduler issued commands (which is row-hit reordered), whereas the AXI R channel must see reads in AR order. Data is buffered per entry, then released oldest-first once complete.

## 13.2	Parameters

| Parameter | Default | Purpose |
| --- | --- | --- |
| NUM_ENTRIES | 8 | In-flight read slots (PTRW = $clog2) |
| N_SCHED_LU | 4 | Parallel scheduler lookup ports |
| NUM_BANKS | 8 | BKW = $clog2(NUM_BANKS) |
| ROW_WIDTH | 14 |  |
| COL_WIDTH | 10 |  |
| AXI_ID_WIDTH | 8 | IW |
| AXI_DATA_WIDTH | 64 | DW (the DFI word at the IFC) |
| BL | 4 | Beats per burst (BCW = $clog2(BL)) |
| AGE_WIDTH | 16 | Free-running age counter width |
| N_SRAM_SLOTS | NUM_ENTRIES | SRAM data slots (may be < NUM_ENTRIES) |

## 13.3	Entry state (per slot)

| Field | Description |
| --- | --- |
| r_valid | Slot occupied |
| r_issued | Scheduler has issued this read to DRAM |
| r_ready | Data complete (last DFI return beat seen) |
| r_bank | Decoded bank (key) |
| r_row | Decoded row (key) |
| r_col | Decoded column |
| r_id | AXI ID (echoed on drain) |
| r_resp | Captured DFI return response |
| r_age | Insert-time snapshot of r_age_ctr |
| r_ptr / r_pv | SRAM slot index (set on first return beat) + its valid flag |

Relative age is w_rel[i] = r_age_ctr - r_age[i] (wrap-safe); larger rel = older. The burst data lives in a distributed-RAM array r_sram[N_SRAM_SLOTS*BL].

## 13.4	Interfaces

### 13.4.1	Insert (from pumice_rd_intake ar_push, AR order)

ins_valid_i / ins_ready_o with {bank, row, col, id}. ins_ready_o is w_have_free (any !r_valid[i]). On fire the slot is allocated, r_issued / r_ready cleared, key/id captured, and r_age <= r_age_ctr.

### 13.4.2	Scheduler lookups (N, keyed {bank, row})

For each of N_SCHED_LU ports the CAM returns the oldest NOT-ISSUED entry matching {bank, row} (max rel among valid && !issued), with its slot, col, id, and age. This lets the arbiter row-hit-schedule reads exactly like writes.

### 13.4.3	Oldest not-issued port

oldest_valid_o + {bank, row, col, id, slot} — the oldest valid && !issued entry, the arbiter’s fallback ACT target.

### 13.4.4	Issue notify

issue_valid_i / issue_ready_o / issue_slot_i. The scheduler tells the CAM which slot it issued. On fire, r_issued[slot] <= 1 and the slot is pushed into an issue-order FIFO (u_issue_q, depth NUM_ENTRIES) so returns fill the right slot in issue order.

### 13.4.5	DFI read return (data in, issue order)

dfi_ret_valid_i / dfi_ret_ready_o with {data, resp, last}. The return-fill engine writes each beat into the SRAM slot of the issue-FIFO head (w_iq_rd_slot):

- On the first beat (r_ret_beat == 0) it pre-allocates a free SRAM slot (w_slot_free) and records it in r_ptr[head], marking r_sram_occ.

- dfi_ret_ready_o is gated on the issue FIFO being non-empty and (first-beat only) a free SRAM slot existing.

- On dfi_ret_last_i the entry is marked r_ready, r_resp captured, and the issue FIFO popped (w_iq_rd_ready).

### 13.4.6	Drain to pumice_rd_intake (AR order, oldest-first, ready-gated)

drain_valid_o / drain_ready_i with {data, id, resp, last}. The draining slot is the combinational oldest-valid pick w_dro_slot (max rel among all valid), which is stable across a burst: the entry stays valid until its own last-beat evict, and later inserts are always younger, so no other entry can become “more oldest” mid-burst. Drain is enabled when w_dr_go = oldest-found && r_ready[oldest] (data staged). Only a burst beat-counter r_dr_beat is registered; drain_last_o = (r_dr_beat == BL-1).

## 13.5	De-FSM’d sequencing

There is no active/slot FSM on the return or drain path. The return path targets the issue-FIFO head; the drain path targets the oldest-valid pick. Both “active” slots are combinational functions of registered state and remain stable until their burst’s last beat. The only registered burst state is the two beat-counters (r_ret_beat, r_dr_beat) plus the age counter and per-entry flags. This mirrors the write CAM and removes the SWAG’s issue FSM entirely.

## 13.6	Eviction

On the drain last beat (w_dr_fire && drain_last_o): r_valid[oldest] <= 0, r_pv cleared, r_dr_beat reset, and the SRAM slot freed (r_sram_occ[r_ptr[oldest]] <= 0). AR-order release is a natural consequence of always draining the oldest entry.

## 13.7	Reset

ALWAYS_FF_RST(aclk, aresetn, ...): clears r_age_ctr, both beat-counters, r_sram_occ, and per-entry r_valid / r_issued / r_ready / r_pv. busy_o is oldest-found || issue-FIFO non-empty.

## 13.8	Notes / flags

- Reads are keyed on {bank, row} only (not AXI ID); ID is carried for the R echo and drained with the data. There is no per-ID beat table.

- The drain_id_o port is driven but left unconnected at the IFC (the intake’s order FIFO already carries the R-channel id); oldest_id_o and sched_lu_id_o are likewise informational — the arbiter’s unused sink absorbs the id/slot buses it does not consume.

- No CSR/QoS priority in this CAM — selection is purely by wrap-safe age.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Write Data CAM (pumice_wr_data_cam)

Module: pumice_wr_data_cam.sv Location: rtl/fub/ Category: FUB Parent: pumice_axi4_ifc Status: Implemented (fill / commit-drain / snarf movers)

Rearchitected: the SWAG wr_cmd_cam was an ID-keyed metadata CAM with w_buf_ptr / strb_ptr pointers into an external AXI write buffer and a b_pending / b_complete window driven by xbank_timers. The live pumice_wr_data_cam owns the write-data SRAM and is keyed on {bank, row, col} with a free-running age. It has three data movers over the SRAM — fill (on insert), snarf-stream (read-your-write), and commit-drain (to DFI write) — plus associative query ports. Commit completion drives B directly (no external write-window timer). All movers are FIFO-fed with beat-counters — no active/slot FSM.

## 14.1	Purpose

pumice_wr_data_cam holds every pending write: the decoded command key {bank, row, col} and the burst data in an SRAM slot. It presents the scheduler the same lookup/oldest interface as the read CAM so writes row-hit schedule identically, forwards data to in-flight reads (snarf), streams committed data to the DFI write path, and self-generates the B-completion strobe on evict.

## 14.2	Parameters

| Parameter | Default | Purpose |
| --- | --- | --- |
| NUM_ENTRIES | 8 | In-flight write slots (PTRW = $clog2) |
| N_SCHED_LU | 4 | Parallel scheduler lookup ports |
| NUM_BANKS | 8 | BKW = $clog2(NUM_BANKS) |
| ROW_WIDTH | 14 |  |
| COL_WIDTH | 10 |  |
| AXI_ID_WIDTH | 8 | IW |
| AXI_DATA_WIDTH | 64 | DW (the DFI word at the IFC); SW = DW/8 |
| BL | 4 | Beats per burst (BCW = $clog2(BL)) |
| AGE_WIDTH | 16 | Free-running age counter width |
| N_SRAM_SLOTS | NUM_ENTRIES | SRAM data slots (may be < NUM_ENTRIES) |

## 14.3	Entry state (per slot)

| Field | Description |
| --- | --- |
| r_valid | Slot occupied |
| r_bank / r_row / r_col | Decoded command key |
| r_id | AXI ID (echoed on commit-done → B) |
| r_age | Insert-time snapshot of r_age_ctr |
| r_ptr / r_pv | SRAM slot index (set on first fill beat) + its valid flag |
| r_fdone | Fill complete — set on the last fill beat. Only then may the entry be scheduled or snarfed (otherwise commit-drain could outrun a gapped fill and read stale SRAM beats). |
| r_sched | Arbiter has committed this slot — excludes it from sched_lu / oldest the next cycle so the arbiter can pick another entry immediately (clean 1 cmd/clock). |

Relative age w_rel[i] = r_age_ctr - r_age[i] (wrap-safe). The data and strobes live in distributed-RAM arrays r_sram / r_strb, each N_SRAM_SLOTS*BL deep.

## 14.4	Age selectors

| Port | Pick rule | Rationale |
| --- | --- | --- |
| Snarf lookup | youngest match (min rel) | Latest data for a read |
| Scheduler lookup | oldest match (max rel) | In-order commit per row |
| Oldest port | oldest valid (max rel) | Scheduler fallback |

All three selectors are gated on r_valid && r_fdone && !r_sched.

## 14.5	Interfaces

### 14.5.1	Insert (from pumice_wr_intake aw_push)

ins_valid_i / ins_ready_o with {bank, row, col, id}. ins_ready_o is w_have_free && u_fill_q.wr_ready. On fire the slot is allocated (via the priority-encoded free slot), key/id/age captured, and r_fdone cleared. The slot index is pushed into the fill FIFO (u_fill_q, depth NUM_ENTRIES).

### 14.5.2	Fill (from pumice_wr_intake wdata pop)

wd_valid_i / wd_ready_o with {data, strb, last}, written to SRAM in insert order. The fill engine reads its slot from the fill-FIFO head:

- On the first beat (r_fill_beat == 0) a free SRAM slot is pre-allocated and recorded in r_ptr[head]; wd_ready_o requires a free slot on the first beat.

- Each beat writes r_sram / r_strb at r_ptr[head]*BL + r_fill_beat.

- On wd_last_i the entry’s r_fdone is set and the fill FIFO popped.

### 14.5.3	Snarf lookup + stream (from pumice_rd_intake)

snarf_probe_* returns a combinational snarf_hit_o. Read-your-write forwarding is limited to the safe case — the hit requires all of:

- !r_sched — a scheduled write is draining/evicting to DRAM, so its CAM data is racy.

- r_id == snarf_probe_id_i — same-id W-before-R is the only AXI-ordered case where the read is required to see the write (cross-id has no ordering guarantee → that read takes the DRAM path).

- snarf_probe_len_i == BL-1 — every admitted write is exactly BL beats, so a short/long read must not snarf a full-BL write.

On snarf_accept_i (the read was admitted as a hit) the youngest matching slot is pushed into a snarf-request FIFO (u_snarf_q). The snarf read engine streams snarf_rd_* from r_sram at the FIFO-head slot, beat-counter r_sn_beat, popping on the last beat.

### 14.5.4	Oldest port + scheduler lookups

Same shape as the read CAM: oldest_* presents the oldest schedulable entry; each of N_SCHED_LU ports returns the oldest {bank, row} match with its slot, col, id, and age.

### 14.5.5	Commit (from arbiter) + drain to DFI write

commit_valid_i / commit_ready_o / commit_slot_i. Commit marks the slot r_sched (immediate exclusion) and enqueues it into the drain FIFO (u_drain_q); commit_ready_o is the drain FIFO’s write-ready. The commit-drain read engine streams cm_rd_* ({data, strb, last}) from r_sram at the drain-FIFO-head slot, beat-counter r_cm_beat, to the DFI write path. Marking is decoupled from draining so the arbiter is never stalled by the DFI back-end.

### 14.5.6	Commit-done → B

On the commit-drain last beat, commit_done_valid_o + commit_done_id_o strobe the write intake’s B-response path, and the entry is evicted: r_valid/r_pv/r_fdone/r_sched cleared and the SRAM slot freed. There is no separate b_pending/b_complete window and no xbank_timers completion driver — completion is simply “the committed burst finished streaming to DFI.”

## 14.6	De-FSM’d sequencing

Each of the three movers (fill, snarf, commit-drain) is just a burst beat-counter. The “active” slot for each is its request-FIFO head, stable until popped on the last beat. There is no active/slot latch and no per-slot beats counter beyond the shared engine counters.

## 14.7	Difference from the read CAM

| Aspect | Read CAM (pumice_rd_cmd_cam) | Write CAM (pumice_wr_data_cam) |
| --- | --- | --- |
| Data direction | DFI return in → drain out to intake | Fill in from intake → commit out to DFI |
| Movers | return-fill + drain | fill + snarf-stream + commit-drain |
| Forwarding | none | snarf (read-your-write) — the wr2rd path |
| Extra flags | r_issued, r_ready | r_fdone, r_sched |
| Completion | AR-order drain to R channel | commit-done strobe → B channel |

## 14.8	Reset / busy

ALWAYS_FF_RST clears the age counter, all three beat-counters, r_sram_occ, and per-entry r_valid/r_pv/r_fdone/r_sched. busy_o is oldest-found or any request FIFO non-empty.

## 14.9	Notes / flags

- The wr2rd (read-your-write) forwarding is entirely the snarf mover here; there is no standalone wr2rd_forward block in the live path (see ch02/21).

- N_SRAM_SLOTS may be smaller than NUM_ENTRIES: an entry can exist in the CAM (key + age) before its SRAM slot is allocated on the first fill beat, so more commands can be tracked than there are data slots.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Write-to-Read Forward — RETIRED as a standalone block

Status: RETIRED. There is no wr2rd_forward.sv in the live RTL. Replaced by: the snarf mover inside pumice_wr_data_cam (see §17, the Write Data Path chapter). Probe driver: pumice_rd_intake (the snarf probe on the AR path).

## 15.1	Why it was retired

The rearchitected AXI4 interface (pumice_axi4_ifc) folded write-to-read forwarding into the write CAM rather than keeping it as a separate FUB on the AR path. The old wr2rd_forward block sat between addr_mapper and rd_cmd_cam and compared each AR against a wr_cmd_cam snapshot bus; that snapshot bus, the standalone comparator, and the separate w_buf storage it read from no longer exist. Forwarding is now a native operation of pumice_wr_data_cam, which already holds the pending writes and their burst data in its SRAM.

## 15.2	Where forwarding lives now

Write-to-read forwarding (“snarf”) is the snarf mover of pumice_wr_data_cam. The flow is:

- pumice_rd_intake presents an incoming AR’s decoded key {bank, row, col} plus its AXI id and arlen on the CAM’s snarf_probe_* port.

- pumice_wr_data_cam combinationally searches its entries and asserts snarf_hit_o for a youngest match, subject to the safe-case restrictions.

- On snarf_accept_i, the matched slot is queued into the snarf request FIFO and the snarf mover streams the write’s SRAM beats back on snarf_rd_* (non-destructively — the write still drains to DRAM normally). Otherwise the read takes the DRAM miss path through pumice_rd_cmd_cam (§18).

## 15.3	Restrictions (narrower than the old block)

The live snarf is limited to the safe case — a write is snarfable only if all three hold:

- Unscheduled (!r_sched): a scheduled write is draining/evicting to DRAM, so its CAM data is racy.

- Same AXI id: same-id write-before-read is the only AXI-ordered case where the read is required to see the write. Cross-id reads have no ordering guarantee and take the DRAM path.

- Same burst length (arlen == BL-1): every admitted write is exactly BL beats (ragged bursts are rejected in pumice_wr_intake), so a short or long read cannot snarf a full-BL write.

The match must also be fill-complete (r_fdone); among candidates the youngest wins (latest data). This differs from the old block’s “last-write-wins by highest slot index, any matching id, conservative all-1 strb-coverage” policy — the live rule is youngest + same-id + same-BL + unscheduled.

## 15.4	Memory Ordering

Forwarding preserves AXI per-ID in-order semantics: the snarf path returns the same data a DRAM read would have, just earlier. Because it is restricted to same-id matches, it never forwards across an unordered cross-id pair.

## 15.5	Tests

Covered by the pumice_wr_data_cam FUB test and the pumice_axi4_ifc integration tests (snarf stream, snarf-vs-DRAM-path selection, and the same-id / same-BL / unscheduled exclusion scenarios). See §17 for the detailed test plan.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Command Arbiter (pumice_cmd_arbiter)

Module: pumice_cmd_arbiter.sv Location: rtl/fub/ Category: FUB (the pick core) Parent: pumice_mem_cmd_scheduler Status: Implemented (single-issue, single-rank v1)

Rearchitected: the SWAG scheduler.sv was a multi-mask priority-encoder pipeline (Stages 1–4) over a wide txn_queue snapshot with a column-op FSM (S_NEED_PRE → S_NEED_ACT → S_NEED_RDWR → S_DONE) and a round-robin W/R toggle. That block is retired. The live pick core is pumice_cmd_arbiter — a combinational, single-issue, FSM-free priority picker that reads the CAM lookup/oldest ports and the per-bank / global readiness inputs and emits ONE abstract DRAM command per cycle. It is instantiated inside pumice_mem_cmd_scheduler alongside the timers, refresh, init, and mode-register (see ch02/19, ch02/09, ch02/11, ch02/12).

## 16.1	Purpose

pumice_cmd_arbiter picks one abstract DRAM command each cycle and pushes it into the scheduler → DFI command interface. It is PHY/nphases-agnostic and single-issue; JEDEC timing is enforced by the per-bank (pumice_bank_timers) and global (global_timers) readiness inputs it consumes. Open-page vs auto-precharge is decided inline from page_policy_i — there is no standalone page predictor in the build (see ch02/08).

The wider pumice_mem_cmd_scheduler wrapper wires the arbiter to those timers, the refresh controller, the init sequencer + mode-register shadow, and an output command FIFO; the CAM sched-lookup / oldest / commit / issue ports pass through to pumice_axi4_ifc.

## 16.2	Priority (each cycle)

The pick is a single combinational if/else cascade producing an abstract command plus its side-effects (event strobes, CAM commit/issue, refresh grant), all gated on the command sink accepting the push:

- INIT (!init_done_i) — forward the init_sequencer command verbatim (init_cmd_op/bank/row).

- REFRESH (refresh_req_i || refresh_drain_i) — if any bank is active, precharge the lowest active, precharge-ready, un-guarded bank (one/cycle); once no bank is active, issue OP_REF and assert refresh_grant_o.

- COLUMN row-hit — an issuable RD/WR to an open row. READ-PRIORITY: a ready read wins over a ready write. Within each direction, oldest (max CAM relative age) wins the tie-break.

- FALLBACK — no ready row-hit: ACT the oldest pending op’s row on an idle, act-ready, un-guarded bank; or PRE a bank that is open on the wrong row.

### 16.2.1	Column issuability

For each bank j the arbiter drives the CAM lookups with {bank j, that bank's open row} (bank_open_row_i), gated valid by bank_row_active_i. A returned hit is issuable when:

- RD: rd_lu_hit && bank_rdwr_ready && tccd_ok && twtr_ok

- WR: wr_lu_hit && bank_rdwr_ready && tccd_ok && trtw_ok

The oldest issuable RD (max rd_lu_age) and oldest issuable WR are scanned in parallel across all N_LU = NUM_BANKS lookups; read-priority then selects RD over WR if both exist.

### 16.2.2	Auto-precharge (inline page policy)

w_ap = (page_policy_i == PAGE_POLICY_CLOSE);

A column op becomes OP_RDA / OP_WRA when w_ap is set, else OP_RD / OP_WR. evt_ap_o = w_ap_out tells the bank timers to model the auto-precharge. PAGE_POLICY_OPEN leaves rows open; PAGE_POLICY_HAPPY_HYBRID is treated as OPEN in v1 (the predictor hook is a TODO — see ch02/08).

## 16.3	Per-bank ACT/PRE re-issue guard

Because pumice_bank_timers register their readiness outputs (a 2-cycle latency from an evt to act/pre_ready dropping), a purely combinational arbiter would re-issue ACT/PRE to the same bank before the timers reflect it. The arbiter keeps a 2-cycle per-bank guard (r_guard0, r_guard1; w_guarded = guard0 | guard1) set on any accepted ACT/PRE, and skips guarded banks in the refresh-PRE and fallback paths.

Column ops need no guard: both CAMs exclude a just-committed/issued slot from their lookup/oldest ports the next cycle (r_sched on the write side, r_issued on the read side), so the arbiter never re-issues the same slot. The shared-DQ-bus occupancy (a BL burst owns the DQ bus for BL/DFI_RATE dfi cycles) is a dfi_clk-domain constraint enforced downstream in pumice_dfi_cmd_path (COL_BURST_CYC), not here — the CDC decouples aclk command issue from dfi_clk DQ timing.

## 16.4	Interface (arbiter)

| Group | Direction | Notes |
| --- | --- | --- |
| page_policy_i | in | OPEN / CLOSE / HAPPY_HYBRID (HAPPY == OPEN in v1) |
| init passthrough | in | init_done_i, init_cmd_{valid,op,bank,row}_i |
| refresh | in/out | refresh_req_i, refresh_drain_i, refresh_grant_o |
| per-bank readiness | in | bank_{act,rdwr,pre}_ready_i, bank_row_active_i, bank_open_row_i ([NUM_RANKS][NUM_BANKS]) |
| global readiness | in | tfaw_ok_i, trrd_ok_i (per-rank), twtr_ok_i, trtw_ok_i, tccd_ok_i |
| wr CAM lookup + oldest + commit | in/out | wr_lu_* (drive + read), wr_oldest_*, wr_commit_{valid,slot}_o |
| rd CAM lookup + oldest + issue | in/out | rd_lu_*, rd_oldest_*, rd_issue_{valid,slot}_o |
| event strobes | out | evt_{act,rd,wr,pre,ap}_o, evt_{rank,bank,row}_o (to timers) |
| command push | out/in | cmd_{valid,op,rank,bank,row,col,ap}_o, cmd_ready_i |

## 16.5	Side-effects (all gated on w_fire = w_valid && cmd_ready_i)

- evt_act/rd/wr/pre_o — strobe the bank + global timers on an accepted issue.

- wr_commit_valid_o / wr_commit_slot_o — on an accepted WR/WRA, mark the wr CAM slot scheduled (enqueue to its drain FIFO).

- rd_issue_valid_o / rd_issue_slot_o — on an accepted RD/RDA, mark the rd CAM slot issued (enqueue to its issue FIFO).

- refresh_grant_o — on an accepted OP_REF.

cmd_valid_o is asserted whenever a command is picked; cmd_rank_o is tied to rank 0 (RK0) — v1 is a single-rank pick.

## 16.6	v1 scope / TODO

- Single-rank pick (RK0 = 0); multi-rank arbitration is a TODO.

- HAPPY_HYBRID == OPEN; the per-bank page predictor is not in the build.

- Write-drain watermark and powerdown are TODO.

- Unused CAM buses (wr_lu_id_i, rd_lu_id_i, *_oldest_slot_i) are absorbed by an explicit unused sink.

## 16.7	pumice_mem_cmd_scheduler wrapper

The macro instantiates, on a single aclk:

| Instance | Module | Role |
| --- | --- | --- |
| u_init | init_sequencer | JEDEC MRS init; gates traffic until init_done |
| u_mode_reg | mode_register | MRS-updated CL/CWL/BL shadow → cl_o/cwl_o/bl_o |
| u_refresh | refresh_ctrl | tREFI + postpone; enabled after init |
| u_bank_timers | pumice_bank_timers | per-(rank,bank) safe timers + open-row |
| u_global_timers | global_timers | tFAW/tRRD (per-rank), tWTR/tRTW/tCCD (global) |
| u_arbiter | pumice_cmd_arbiter | the pick core (above) |
| u_cmd_fifo | gaxi_fifo_sync | output command FIFO, packs {op,rank,bank,row,col,ap} (CMD_W = 4 + RKW + BKW + ROW_WIDTH + COL_WIDTH + 1, depth CMD_FIFO_DEPTH=8) |

The arbiter pushes into u_cmd_fifo; the FIFO read side is the macro’s cmd_*_o command stream to the DFI layer. busy_o is !init_done || refresh_req || cmd-fifo-non-empty || any-bank-active.

Issue rate is one command per aclk. The arbiter never sees the DFI multi-phase dimension — the DFI layer packs phases downstream. Selection is oldest-first by wrap-safe CAM age; there is no QoS, no lookahead window, and no SCHEDULER_MODE OOO/INORDER synthesis switch.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Page Policy (inline in pumice_cmd_arbiter)

Live location: rtl/fub/pumice_cmd_arbiter.sv (inline decision) Standalone module: rtl/fub/page_predictor.sv — present but not in the default build Category: FUB behaviour (arbiter-embedded) Parent: pumice_mem_cmd_scheduler Status: Inline OPEN/CLOSE decision implemented; HAPPY_HYBRID hook is a TODO

Rearchitected: the SWAG planned a per-(rank, bank) HAPPY hybrid predictor as a distinct scheduler FUB feeding a predict_open / predict_hit hint into the scheduler’s auto-precharge stage. In the live build there is no standalone predictor instantiated — the open-page vs auto-precharge decision is made inline in pumice_cmd_arbiter from the page_policy_i CSR field. A page_predictor.sv file still exists in the repo (a 2-bit saturating counter, optional/reference) but is not wired into pumice_mem_cmd_scheduler or pumice_core; confirm against the filelists in rtl/filelists/.

## 17.1	The live decision

The arbiter computes a single auto-precharge bit directly from the page policy:

// pumice_cmd_arbiter.sv
logic w_ap;
assign w_ap = (page_policy_i == PAGE_POLICY_CLOSE);

and applies it to every column op it picks:

| page_policy_i | Column op emitted | Effect |
| --- | --- | --- |
| PAGE_POLICY_OPEN | OP_RD / OP_WR | Rows stay open; the arbiter row-hit-schedules subsequent column ops to the open row |
| PAGE_POLICY_CLOSE | OP_RDA / OP_WRA | Every column op auto-precharges (w_ap_out = 1) |
| PAGE_POLICY_HAPPY_HYBRID | (treated as OPEN in v1) | Predictor hook is a TODO; behaves as OPEN |

evt_ap_o is driven from w_ap_out, which tells pumice_bank_timers to model the auto-precharge (transition the bank toward precharged after the column op) rather than leaving the row active. There is no per-bank hint, no saturating counter, and no HAPPY_HYBRID-specific RTL in the live path — HAPPY currently falls through to the OPEN branch.

page_policy_i originates at pumice_top from hwif_out.REFRESH_TUNING.page_policy_or (see ch02/01), so page policy is a runtime CSR setting, not a build parameter.

## 17.2	Why inline

Open-page management in this controller is not a prediction problem: the arbiter already scans, every cycle, which banks have an open row (bank_row_active_i / bank_open_row_i) and which pending CAM entries are a row hit to that open row (the per-bank CAM lookups). Given that live row-hit visibility, the only remaining choice is the static “leave open vs auto-precharge” policy bit, which is exactly what the inline w_ap expresses. A separate predictor structure — a table, a warmup counter, per-entry hysteresis, an outcome-training broadcast — would add significant state for a marginal gain over the CSR-selected OPEN/CLOSE policy, and none of that is in the shipping build.

## 17.3	The retained page_predictor.sv file

For reference, the file that remains in the repo implements a per-(rank, bank) 2-bit saturating “predict open” hint:

| Counter | Meaning |
| --- | --- |
| 00 | strongly closed |
| 01 | weakly closed |
| 10 | weakly open |
| 11 | strongly open |

It updates on the evt_act_i strobe: the first ACT on a bank records the row with no update; a subsequent ACT to the same row saturates the counter up, a different row saturates it down. predict_open_o[r][b] is the counter MSB, all strict-flopped. This matches the module header and the two ALWAYS_FF_RST blocks in page_predictor.sv.

If HAPPY_HYBRID is promoted from TODO to a real path, this module is the intended building block: the arbiter would consult predict_open_o[rank][bank] to choose OP_RD/WR (predict open) vs OP_RDA/WRA (predict closed) when page_policy_i == PAGE_POLICY_HAPPY_HYBRID, replacing the current OPEN fall-through. Until then it is not instantiated.

## 17.4	Notes / flags

- The elaborate SWAG predictor (4 K-entry hashed table, BRAM storage, warmup counter, per-entry hysteresis, outcome-training broadcast, accuracy telemetry, PAGE_PREDICTOR_TABLE_BITS / HYSTERESIS_BITS params) does not exist in any live RTL — that was SWAG only.

- The page_predictor.sv that does exist is the simpler per-bank 2-bit counter documented above; treat it as optional/reference, gated by the filelists.

- powerdown_ctrl.sv is likewise present-but-optional and not in the default top build.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Global Timers (global_timers)

Module: global_timers.sv Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: Implemented (per-rank tFAW/tRRD; global tWTR/tRTW/tCCD; strict-flop outputs)

Replaces the retired cross-bank timer block. The original xbank_timers FUB owned a mix of per-bank AND cross-bank counters. That mix has been split:

- Per-bank timing (tRCD, tRP, tRAS, tRC, tWR/tRTP) moved into bank_timer, stamped per (rank, bank) by pumice_bank_timers.

- Cross-bank / channel-wide turnaround (tFAW, tRRD, tWTR, tRTW, tCCD) lives here in global_timers.

The retired block’s cross-rank extras (tRTRS, tCS, the last_rd_rank / last_cmd_rank chains, and the wide blocks_*[NR][NB] broadcast) are not in the live RTL. global_timers emits compact per-rank / global *_window_ok readiness bits that the arbiter ANDs with bank_timer’s safe_* outputs.

## 18.1	Purpose

global_timers holds the controller-wide DRAM constraints that span banks and therefore cannot live inside a single bank’s timer:

- tFAW — no more than four ACT commands within a rolling t_faw_i window. Tracked per rank (each rank has its own 4-deep sliding window) so a multi-rank build enforces the device-local four-activate limit independently.

- tRRD — minimum cycles between any two ACTs, per rank.

- tWTR — cycles since the last WR (global; the DQ bus is shared).

- tRTW — cycles since the last RD (global; shared DQ bus).

- tCCD — cycles since the last column command (global; paces back-to-back RD/WR across banks on the shared DQ bus).

The arbiter reads five readiness outputs — tfaw_window_ok_o[r], trrd_window_ok_o[r] (per rank), and twtr_global_ok_o, trtw_window_ok_o, tccd_window_ok_o (global) — and ANDs the relevant subset into its eligibility decision alongside the per-bank safe_* bits.

## 18.2	Constraint Scope

| Constraint | Scope | Why |
| --- | --- | --- |
| tFAW | per rank | Four-Activate-Window — device-local thermal/power limit |
| tRRD | per rank | Minimum ACT-to-ACT stagger inside one DRAM device |
| tCCD | global (channel) | Column-to-column pacing on the shared DQ bus |
| tWTR | global (channel) | Write → read turnaround on the shared DQ bus |
| tRTW | global (channel) | Read → write turnaround on the shared DQ bus |

There is no cross-rank (tRTRS / tCS) tracking in the live module. NUM_RANKS only sizes the per-rank tFAW/tRRD arrays; for NUM_RANKS == 1 those arrays are a single element and the per-rank outputs collapse to a single bit. The DQ-bus turnaround counters (tWTR/tRTW/tCCD) are single global counters shared across all ranks because the DQ bus is physically shared.

## 18.3	Synthesis Parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| NUM_RANKS | 1 | Number of per-rank tFAW windows and tRRD counters |
| NUM_BANKS | 8 | Present for interface symmetry; not used to size counters |
| RKW | clog2(NUM_RANKS) | Width of evt_act_rank_i (min 1) |
| BKW | clog2(NUM_BANKS) | Bank-select width (interface symmetry) |

All timing reload values are runtime CSR-backed 8-bit inputs (t_faw_i, t_rrd_i, t_wtr_global_i, t_rtw_i, t_ccd_i) — not parameters.

## 18.4	Per-Rank Trackers

### 18.4.1	tFAW window — r_faw_slots[NUM_RANKS][4]

Each rank owns a 4-deep pool of 8-bit down-counters (logic [NUM_RANKS-1:0][3:0][7:0]). Every cycle each non-zero slot decrements. On an ACT to a rank, the slot with the smallest remaining count is reloaded with t_faw_i:

if (evt_act_i) begin
    // pick the slot with the smallest remaining count for this rank
    automatic int unsigned slot_pick = 0;
    automatic logic [7:0] slot_min = 8'hFF;
    for (int unsigned i = 0; i < 4; i++) begin
        if (r_faw_slots[evt_act_rank_i][i] < slot_min) begin
            slot_min  = r_faw_slots[evt_act_rank_i][i];
            slot_pick = i;
        end
    end
    r_faw_slots[evt_act_rank_i][slot_pick] <= t_faw_i;
    r_trrd_cnt [evt_act_rank_i]            <= t_rrd_i;
end

The window is “OK” (an ACT may issue) when at least one of the rank’s four slots is at 0 — i.e. fewer than four of the most-recent ACTs are still inside the tFAW window:

w_tfaw_ok[k] = 1'b0;
for (int unsigned i = 0; i < 4; i++)
    if (r_faw_slots[k][i] == 8'd0) w_tfaw_ok[k] = 1'b1;

The “pick smallest count” load policy is equivalent to “replace the oldest entry,” which is the correct rolling-window behavior. It avoids a timestamp FIFO (which would need a subtractor at check time); the check is a 4-way compare-to-zero, one LUT layer.

### 18.4.2	tRRD — r_trrd_cnt[NUM_RANKS]

One 8-bit down-counter per rank, reloaded with t_rrd_i on each ACT to that rank (see the ACT block above). trrd_window_ok_o[r] is (r_trrd_cnt[r] == 0). An ACT on rank 0 does not affect r_trrd_cnt[1].

## 18.5	Global Trackers (channel-wide)

Three single 8-bit counters shared across all ranks:

| Counter | Reload trigger | Reload value | Drives |
| --- | --- | --- | --- |
| r_twtr_cnt | evt_wr_i (WR event) | t_wtr_global_i | twtr_global_ok_o |
| r_trtw_cnt | evt_rd_i (RD event) | t_rtw_i | trtw_window_ok_o |
| r_tccd_cnt | evt_wr_i OR evt_rd_i (any column command) | t_ccd_i | tccd_window_ok_o |

if (evt_wr_i) begin r_twtr_cnt <= t_wtr_global_i; r_tccd_cnt <= t_ccd_i; end
if (evt_rd_i) begin r_trtw_cnt <= t_rtw_i;        r_tccd_cnt <= t_ccd_i; end

Each *_ok output is asserted when its counter is 0. tCCD reloads on either a read or a write, so it paces every column-to-column gap on the shared bus. tWTR loads on writes and gates reads; tRTW loads on reads and gates writes.

At DDR2-800 tRTW is typically 0 cycles, but the counter is still synthesized for parameter consistency and forward-compatibility with DDR3+ where tRTW matters.

Note on the tWTR trigger. In the live RTL, tWTR reloads on the WR command event (evt_wr_i), not on a separate last-write-data-beat strobe. The CSR-supplied t_wtr_global_i value therefore carries whatever additional WL+BL/2 offset the timing programming folds in; there is no data-path wr_last_beat input to this module.

## 18.6	Readiness Outputs (strict-flop)

The *_window_ok_o outputs are registered (strict-flop house style). Each cycle the module computes the next-cycle window-ok values combinationally, then flops them:

// next-cycle tFAW-ok (combinational, per rank)
tfaw_window_ok_o <= w_tfaw_ok;
for (int unsigned k = 0; k < NUM_RANKS; k++)
    trrd_window_ok_o[k] <= (r_trrd_cnt[k] == 8'd0);
twtr_global_ok_o <= (r_twtr_cnt == 8'd0);
trtw_window_ok_o <= (r_trtw_cnt == 8'd0);
tccd_window_ok_o <= (r_tccd_cnt == 8'd0);

Out of reset, every *_ok output is driven to its permissive value ('1 / 1'b1) so the arbiter is not spuriously blocked before any command has issued.

Because the outputs are flopped, the arbiter sees a one-cycle-registered view of the counters — the same single-stage discipline as bank_timer’s combinational safe_*, one flop deeper. The arbiter accounts for this uniform one-cycle readiness latency across both timer FUBs.

## 18.7	Observability

All observability outputs are combinational “counter non-zero” flags:

| Signal | Width | Meaning |
| --- | --- | --- |
| obs_faw_nz_o | NR | Rank’s tFAW window currently full (!w_tfaw_ok[r]) |
| obs_trrd_nz_o | NR | r_trrd_cnt[r] != 0 |
| obs_twtr_nz_o | 1 | r_twtr_cnt != 0 |
| obs_trtw_nz_o | 1 | r_trtw_cnt != 0 |
| obs_tccd_nz_o | 1 | r_tccd_cnt != 0 |

## 18.8	Interface

### 18.8.1	Clocks / Reset

| Signal | Direction | Description |
| --- | --- | --- |
| mc_clk | input | Controller clock |
| mc_rst_n | input | Active-low async reset |

### 18.8.2	Timing config (CSR-backed)

| Signal | Width | Constraint |
| --- | --- | --- |
| t_faw_i | 8 | tFAW |
| t_rrd_i | 8 | tRRD |
| t_wtr_global_i | 8 | tWTR |
| t_rtw_i | 8 | tRTW |
| t_ccd_i | 8 | tCCD (CAS-to-CAS) |

### 18.8.3	Events from the arbiter

| Signal | Width | Description |
| --- | --- | --- |
| evt_act_i | 1 | ACT issued this cycle |
| evt_act_rank_i | RKW | Rank of the ACT (selects tFAW/tRRD) |
| evt_rd_i | 1 | RD issued this cycle |
| evt_wr_i | 1 | WR issued this cycle |

### 18.8.4	Readiness to the arbiter

| Signal | Width | Description |
| --- | --- | --- |
| tfaw_window_ok_o | NR | Per-rank: rank may issue another ACT |
| trrd_window_ok_o | NR | Per-rank: tRRD elapsed |
| twtr_global_ok_o | 1 | Global: tWTR elapsed (RD may follow WR) |
| trtw_window_ok_o | 1 | Global: tRTW elapsed (WR may follow RD) |
| tccd_window_ok_o | 1 | Global: tCCD elapsed (next column cmd OK) |

### 18.8.5	Observability

obs_faw_nz_o[NR], obs_trrd_nz_o[NR], obs_twtr_nz_o, obs_trtw_nz_o, obs_tccd_nz_o.

## 18.9	Timing Budget

Event → readiness is single-cycle plus the output flop:

cycle T:   arbiter asserts evt_act_i, evt_act_rank_i = r
cycle T:   r_trrd_cnt[r] <- t_rrd_i; picked tFAW slot <- t_faw_i
cycle T+1: trrd_window_ok_o[r] = 0 (registered); another ACT to rank r blocked

The window-ok comparators are one LUT layer (compare-to-zero) feeding the output flop, so the path is short. The arbiter combines these with the per-bank safe_* bits; the aggregate eligibility AND is the arbiter’s own critical path (§2.7), not this module’s.

## 18.10	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Two ACTs to same rank, gap < tRRD | Second blocked; trrd_window_ok_o[r] low |
| Two ACTs to same rank, gap == tRRD | Second issues exactly at tRRD |
| Five ACTs to one rank inside tFAW window | Fifth blocked; all four slots non-zero |
| Five ACTs spanning the tFAW boundary (4 in + 1 aged out) | Fifth issues; oldest slot returned to 0 |
| Multi-rank: ACTs on rank 0 do not block ACTs on rank 1 | Per-rank tRRD / tFAW isolation |
| WR then RD before tWTR elapses | RD blocked; twtr_global_ok_o low |
| RD then WR before tRTW elapses | WR blocked; trtw_window_ok_o low |
| Back-to-back column commands before tCCD elapses | Second column blocked; tccd_global_ok_o low |
| Reset → all *_ok outputs assert permissively | Reset default correctness |
| tFAW “pick smallest” evicts the oldest slot | Rolling-window load policy |

## 18.11	Open Questions / Future Work

- Cross-rank turnaround (tRTRS / tCS). The live module has no rank-switch DQ-handoff or chip-select-setup tracking. On the DDR2/LPDDR2 board targets (single rank, or same-rank streaming) these are met by IOB margin. A true multi-rank DDR3/DDR4 build would add a last_rd_rank / last_cmd_rank pair and cross-rank gate here — reserved but not implemented.

- tWTR from data-path event. tWTR currently reloads on the WR command event, relying on the CSR value to fold in WL+BL/2. A more precise implementation would load from the last-write-data-beat strobe; add a wr_last_beat_i input if characterization shows the command-relative approximation costs bandwidth.

- Bank-group tCCD (DDR4). DDR4 splits tCCD into same-group (tCCD_L) vs different-group (tCCD_S). DDR2/LPDDR2 have no bank groups, so the single global tCCD is exact here; the DDR4 family controller adds per-group tracking.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Global Timers (global_timers)

Module: global_timers.sv Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: implemented (v2 H)

## 19.1	Purpose

Controller-wide constraint trackers that span banks. Where pumice_bank_timers holds per-(rank, bank) JEDEC “safe” countdowns, global_timers holds the windows that apply across banks — some per-rank (device-local limits), some truly global (shared DQ bus):

| Constraint | Meaning | Tracked as |
| --- | --- | --- |
| tFAW | At most 4 ACT commands within t_faw_i cycles | Per-rank: a 4-deep sliding window of countdown timers |
| tRRD | Minimum cycles between any two ACT commands | Per-rank: single countdown, reloaded on each ACT |
| tWTR | Cycles after a WR before any RD | Global: single countdown, reloaded on each WR |
| tRTW | Cycles after a RD before any WR | Global: single countdown, reloaded on each RD |
| tCCD | Cycles between back-to-back column (RD/WR) commands | Global: single countdown, reloaded on each RD and WR |

tFAW and tRRD are per-rank because they enforce device-local thermal/power limits — each rank has its own 4-deep tFAW slot array and its own tRRD countdown, so multi-rank silicon meters each device independently. tWTR, tRTW, and tCCD are global because they gate the shared DQ bus, which every rank contends for.

The FUB exposes _window_ok flags to the scheduler:

tfaw_window_ok_o [NUM_RANKS]   // per-rank: 1 = at least one tFAW slot is at 0
trrd_window_ok_o [NUM_RANKS]   // per-rank: 1 = tRRD has elapsed
twtr_global_ok_o               // global : 1 = tWTR has elapsed
trtw_window_ok_o               // global : 1 = tRTW has elapsed
tccd_window_ok_o               // global : 1 = tCCD has elapsed

All five _window_ok outputs are strict-flop registered. Default after reset is '1 / 1'b1 (all windows open), which is correct given no ACTs/WRs/RDs have been issued.

## 19.2	Timer mechanics

All timers are 8-bit down-counters that saturate at 0 (they decrement each mc_clk while non-zero). Reload values come in on the t_*_i ports:

- evt_act_i (with evt_act_rank_i): installs t_faw_i into the targeted rank’s tFAW slot with the smallest remaining count (the freed slot), and reloads that rank’s tRRD with t_rrd_i.

- evt_wr_i: reloads the global tWTR (t_wtr_global_i) and tCCD (t_ccd_i).

- evt_rd_i: reloads the global tRTW (t_rtw_i) and tCCD (t_ccd_i).

tfaw_window_ok_o[r] is high when rank r has at least one tFAW slot at 0 (i.e. fewer than 4 ACTs are still inside the window).

## 19.3	Observability

Combinational obs_* “non-zero” flags mirror each counter for CSR/debug observation:

obs_faw_nz_o  [NUM_RANKS]   obs_trrd_nz_o [NUM_RANKS]
obs_twtr_nz_o               obs_trtw_nz_o               obs_tccd_nz_o

## 19.4	Parameters

| Parameter | Default | Purpose |
| --- | --- | --- |
| NUM_RANKS | 1 | Per-rank tFAW/tRRD array depth |
| NUM_BANKS | 8 | (BKW derivation) |

## 19.5	Scope

- Per-rank tFAW / tRRD are implemented (v2 H). Multi-rank silicon may still need the per-rank windows tightened against rank-level scheduler coordination.

- tCCD is tracked here now (it was previously baked into burst-length pacing in the scheduler).

## 19.6	Tests

Verified by dv/tests/fub/test_global_timers.py: smoke, tfaw_4_acts, trrd_spacing, twtr_after_wr, trtw_after_rd (plus tCCD coverage as the suite is extended for the v2 H additions).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Refresh Controller (refresh_ctrl)

Module: refresh_ctrl.sv Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: Implemented — FSM-free tREFI/pending/drain accumulator with a REFab/REFpb selector and REFpb bank rotor.

Scope note: this module is refresh_ctrl. It is a compact, FSM-free accumulator design — three registered counter blocks plus a bank rotor, all strict-flopped at the outputs. It tracks the tREFI interval, keeps a JEDEC-capped count of postponed refreshes, meters how many REF commands the scheduler should burst back-to-back, and (for LPDDR2 REFpb) rotates a target bank across grants.

Features that earlier drafts of this chapter described as current architecture are not implemented and are not part of this module: DARP per-bank age selection, periodic ZQCS piggyback, per-rank PASR mask propagation, self-refresh coordination, LPDDR2 temperature (MR4) scaling, per-rank REFab round-robin, a multi-state FSM, and per-(rank, bank) refresh_req/refresh_gnt handshake arrays. Refresh does not use a bank-machine handshake: it raises a single refresh_req_o and wins against other scheduler commands by priority inside pumice_cmd_arbiter. Those retired features are collected under Open Questions / Future Work.

## 20.1	Purpose

refresh_ctrl owns DRAM refresh scheduling. It produces:

- refresh_req_o — a single request line that elevates refresh to highest priority in the scheduler. It stays high while any refresh is pending.

- pending_refreshes_o — the count of postponed refreshes (JEDEC max 8).

- refresh_drain_active_o — a hint that the scheduler should keep granting REF back-to-back to work down a burst quota.

- refresh_kind_o / refresh_bank_o — the REFab-vs-REFpb selector and, in REFpb mode, the target bank produced by the bank rotor.

- obs_* — observability taps on internal state for future CSR readout.

This is the heart of the controller’s correctness story: get refresh wrong and DRAM forgets data. The design is deliberately simple — three registered counter blocks and a bank rotor, with no control FSM.

## 20.2	Synthesis Parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| NUM_BANKS | 8 | Number of banks the REFpb rotor cycles through (0..NUM_BANKS-1) |
| BA_W | $clog2(NUM_BANKS) | Width of refresh_bank_o / the bank rotor (derived) |

The module imports pumice_pkg::* and uses the standard reset macros (reset_defs.svh). Clock is mc_clk; reset is mc_rst_n (active-low).

## 20.3	Behavioral Blocks

There is no FSM. The module is four registered blocks feeding a strict-flop output stage.

### 20.3.1	1. tREFI Counter (r_refi_cnt)

A 16-bit down-counter that measures the refresh interval in MC cycles.

- Only ticks when enable_i is high (enable_i comes from init_sequencer — refresh is gated off until init completes). While enable_i is low, the counter is held reloaded at t_refi_i.

- When it reaches 0 (w_refi_expired), it reloads t_refi_i and that expiry event drives one increment of the pending accumulator.

### 20.3.2	2. Pending Accumulator (r_pending)

A 4-bit count of postponed refreshes, capped at MAX_PENDING = 8 (the JEDEC ceiling of postponed refreshes).

- A tREFI expiry (while enabled) adds 1, saturating at 8. At saturation the RTL comment flags a looming DRAM data-retention violation — the workload has blocked refresh longer than JEDEC allows.

- A grant (w_grant_accept = refresh_grant_i && r_pending > 0) subtracts 1.

- A simultaneous expiry and grant net to zero change.

- refresh_req_o is (r_pending > 0), registered.

### 20.3.3	3. Drain Quota (r_burst_remaining)

A 4-bit counter that meters how many REFs the scheduler should issue back-to-back once refresh wins arbitration.

- When the previous burst is fully drained (r_burst_remaining == 0) and there is pending work (r_pending > 0), it (re)loads min(refresh_burst_i, r_pending). If that clamp evaluates to 0 it loads 1 instead, so a burst always makes forward progress.

- Each accepted grant decrements it.

- w_drain_active = (r_burst_remaining > 0) && (r_pending > 0) is exported as refresh_drain_active_o. While it is high, the scheduler should keep granting REF back-to-back rather than yielding to reads/writes.

refresh_burst_i is a 4-bit input (1..8) that sets the desired drain count per request cycle.

### 20.3.4	4. REFpb Bank Rotor (r_bank_rotor)

Selects the target bank for LPDDR2 per-bank refresh.

- On each accepted grant, if refpb_mode_i is set, the rotor increments and wraps 0..NUM_BANKS-1.

- In REFab mode (refpb_mode_i == 0) it is held at 0.

- refresh_bank_o is the registered rotor value; it is only meaningful in REFpb mode.

- r_grants_total (16-bit) counts total accepted grants for observability.

refresh_kind_o is simply the registered copy of refpb_mode_i (0 = REFab, 1 = REFpb), forwarded to the scheduler / dfi_cmd_formatter.

All final outputs are strict-flopped (registered) in a single output stage.

## 20.4	Interface

### 20.4.1	Parameters and Clocking

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk | input | 1 | Memory-controller clock |
| mc_rst_n | input | 1 | Active-low asynchronous reset |

### 20.4.2	Inputs

| Signal | Width | Description |
| --- | --- | --- |
| t_refi_i | 16 | Refresh interval in MC cycles (tREFI). Reload value for r_refi_cnt. |
| refresh_burst_i | 4 | Desired drain count per request cycle (1..8). Loads the burst quota. |
| refpb_mode_i | 1 | 0 = REFab, 1 = REFpb (LPDDR2). Selects refresh_kind_o and enables the rotor. |
| enable_i | 1 | From init_sequencer (init done). Gates the tREFI counter and accumulation. |
| refresh_grant_i | 1 | Pulsed by the scheduler when it issues a REF on the DFI bus. |

### 20.4.3	Outputs

| Signal | Width | Description |
| --- | --- | --- |
| refresh_req_o | 1 | Refresh pending: (r_pending > 0), registered. |
| pending_refreshes_o | 4 | Current postponed-refresh count (0..8). |
| refresh_drain_active_o | 1 | Burst quota still owed and pending nonzero — keep granting REF. |
| refresh_kind_o | 1 | Registered refpb_mode_i (0 = REFab, 1 = REFpb). |
| refresh_bank_o | BA_W | REFpb target bank from the rotor (valid in REFpb mode). |

### 20.4.4	Observability (obs_*, future CSR readout)

| Signal | Width | Source |
| --- | --- | --- |
| obs_refi_cnt_o | 16 | Current r_refi_cnt value |
| obs_drain_remaining_o | 4 | Current r_burst_remaining value |
| obs_bank_rotor_o | BA_W | Current r_bank_rotor value |
| obs_grants_total_o | 16 | Total accepted grants (r_grants_total) |

These are wired out for future CSR/telemetry hookup; no CSR block consumes them yet.

## 20.5	Timing / Behavior

- Reset. All registers clear to 0. refresh_req_o, refresh_drain_active_o, refresh_kind_o, refresh_bank_o, and every obs_* output reset to 0.

- Init gating. With enable_i low, r_refi_cnt is held at t_refi_i and no pending refreshes accumulate — refresh is inert until init_sequencer releases enable_i.

- Single-cycle latency. Because there is no FSM, request/drain/bank outputs simply track the counter state one flop behind. When tREFI expires, r_pending rises the next cycle and refresh_req_o follows the cycle after (output flop stage).

- Grant handshake. The module never blocks on a grant. It exports its request and drain hint; the scheduler decides when to pulse refresh_grant_i, and the module bookkeeps against those pulses.

- Priority, not handshake. Refresh wins against other scheduler commands by priority in pumice_cmd_arbiter; there is no per-bank refresh_req/refresh_gnt array and no bank-machine handshake.

## 20.6	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| enable_i low: t_refi_i loaded, no ticks, r_pending stays 0, refresh_req_o stays low | Init gating |
| enable_i high: r_refi_cnt counts down and reloads on expiry | tREFI countdown + reload |
| One tREFI expiry with no grant: r_pending == 1, refresh_req_o asserts | Pending accumulate + request |
| Withhold grants across 8+ expiries: r_pending saturates at 8, does not overflow | JEDEC max-8 postpone cap |
| Expiry and grant in the same cycle: r_pending unchanged | Simultaneous tick+grant net-zero |
| Grants with pending > 0: r_pending decrements; refresh_req_o drops at 0 | Grant accounting |
| refresh_burst_i = N with r_pending >= N: quota loads N, refresh_drain_active_o high | Drain quota load + active hint |
| refresh_burst_i > r_pending: quota clamps to r_pending | Burst clamp (no overcount) |
| refresh_burst_i clamp resolves to 0 but pending > 0: quota loads 1 | Forward-progress floor |
| Grants during drain: r_burst_remaining counts down; refresh_drain_active_o deasserts at 0 | Drain metering |
| REFab mode (refpb_mode_i = 0): refresh_bank_o stays 0, refresh_kind_o == 0 | REFab selector + rotor held |
| REFpb mode (refpb_mode_i = 1): refresh_bank_o rotates 0..NUM_BANKS-1 across grants | REFpb bank rotor wrap |
| obs_* taps track internal counters | Observability wiring |

## 20.7	Open Questions / Future Work

The following were described in earlier SWAG/HAS drafts but are not implemented in refresh_ctrl. They are recorded here as possible future scope, not as current behavior:

- DARP per-bank age selection. REFpb currently uses a plain round-robin bank rotor. An age-aware selector (refresh the oldest idle non-masked bank first, falling back to oldest-first) would need bank-state and last-refresh-age inputs the module does not have today.

- Periodic ZQCS piggyback. No ZQCS interval counter or ZQCS sequencing exists. If added, it could piggyback on the refresh window (all banks idle) rather than triggering a separate bus-blocking event.

- Per-rank PASR mask propagation. No PASR bank/segment masks, no MR16/MR17 propagation. LPDDR2 partial-array self-refresh is unsupported.

- Self-refresh coordination. No sr_entry_req/sr_entry_gnt handshake with a power-state block; the tREFI counter is not paused for self-refresh.

- LPDDR2 temperature scaling. No MR4-driven tREFI derating. t_refi_i is the single reload value regardless of temperature class.

- Per-rank REFab round-robin. Single-rank only; there is no rank pointer or per-rank REF dispatch. NUM_BANKS (banks), not ranks, is the only structural parameter.

- Multi-state FSM / per-bank handshake. The design is FSM-free and uses a single priority-based request rather than per-(rank, bank) refresh_req/refresh_gnt arrays. Any of the above features would likely reintroduce sequencing state.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Init Sequencer (init_sequencer)

Module: init_sequencer.sv Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: Implemented — DDR2 and LPDDR2 sequences both complete (DDR2 validated on the Nexys A7 board; LPDDR2 passes the full sim suite for family reuse)

Renamed / re-scoped: an early sketch called this an init_engine_fub built on a microprogram step-record ROM (opcodes, a step interpreter FSM, SIM_INIT_SCALE, WAIT_FOR_BIT polling, ZQ retry loops, an init_error path, IRQ outputs, and a CSR MR-override path). None of that ROM machinery was built. The implemented module is a small hard-coded FSM that issues a fixed JEDEC bring-up sequence. It drives MR-write strobes into mode_register (which owns the MR shadow + live CL/CWL/BL decode), and it issues the real DRAM commands (PRECHARGE, REFRESH, MRS/MRW) through the command path while init_busy_o is high.

## 21.1	Purpose

init_sequencer walks the JEDEC cold-boot DRAM initialization sequence — the deterministic recipe of DFI init handshake, PRECHARGE-ALL, mode-register loads, and REFRESH settling that must complete before any normal traffic can issue. It is memtype-specific: memtype_i selects between the DDR2 sequence (mirrors LiteDRAM’s proven Nexys A7 recipe) and the LPDDR2 MRW sequence (JEDEC JESD209-2F).

Two things happen for each mode-register step:

- A real DRAM command is issued. The sequencer drives a single-cycle command request (init_cmd_valid_o / init_cmd_op_o / init_cmd_bank_o / init_cmd_row_o) that the parent scheduler forwards to dfi_cmd_formatter. Because the scheduler stays idle during init (it owns nothing else while init_busy_o is high) and the formatter is always cmd_ready, a single-cycle pulse issues exactly one command — no grant handshake is needed.

- The mode_register shadow is updated in lockstep (mr_seq_we_o) so the controller’s live CL/CWL/BL decode tracks what was actually programmed.

The init sequencer sits above the scheduler in the command-priority hierarchy. init_done_o also drives refresh_ctrl.enable_i — periodic refresh does not start until init reports done.

History (why the sequence is “full”). An earlier version was a “simplified” 4-MR shadow-only walk that never issued MRS/PRECHARGE/REFRESH to the DRAM. On the DFI-loopback sim the memory model stores/returns data regardless of init state, so it passed. On real DDR2 the read DLL never locked (no proper reset + refresh sequence) and no IDELAY tap found a read eye. The full sequence documented here fixed on-board bring-up.

## 21.2	Synthesis Parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| ROW_WIDTH | 14 | Width of the init_cmd_row_o request field (carries MR data; must hold DDR2 MR0=0x532 which needs bit 10, so a narrower COL-based path would truncate). |
| NUM_BANKS | 8 | Number of DRAM banks. |
| BKW | derived | $clog2(NUM_BANKS) — width of init_cmd_bank_o (bank / MR index). |

There is no MEMTYPE synthesis parameter — the sequence is selected at run-time by the memtype_i input, so one elaboration supports both families.

## 21.3	Interface

### 21.3.1	Clock / reset

| Signal | Direction | Description |
| --- | --- | --- |
| mc_clk | input | Memory-controller clock. |
| mc_rst_n | input | Active-low async reset (house macros). |

### 21.3.2	Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| memtype_i | input | enum | MEMTYPE_DDR2 selects the DDR2 sequence; anything else selects the LPDDR2 MRW chain. |

### 21.3.3	JEDEC init-sequence waits (CSR-backed, MC cycles)

All countdowns run internally as 16-bit values; the 8-bit inputs are zero-extended. Defaults live in the CSR (mentioned as 512 / 256 / 8 / 8 / 16), so these were promoted from hardcoded constants to INIT_TIMING-programmable.

| Signal | Width | Internal name | JEDEC parameter |
| --- | --- | --- | --- |
| t_init_wait_i | 16 | W_INIT | CKE / tINIT settle (also LPDDR2 tINIT4). |
| t_dll_wait_i | 16 | W_DLL | DLL lock tDLLK (also LPDDR2 tZQINIT). |
| t_mrd_wait_i | 8 | W_MRD | tMRD, post mode-register-set. |
| t_rp_wait_i | 8 | W_RP | tRP, post precharge. |
| t_rfc_wait_i | 8 | W_RFC | tRFC, post auto-refresh. |

### 21.3.4	DFI status

| Signal | Direction | Description |
| --- | --- | --- |
| dfi_init_start_o | output | Asserted (registered) once r_state != S_RESET. Tells the PHY to begin its own init. |
| dfi_init_complete_i | input | PHY reports its DLL-lock / IO training complete. Gates the exit from S_DFI_INIT. |

### 21.3.5	MR-shadow write port (muxed with CSR by pumice_mem_cmd_scheduler)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mr_seq_we_o | output | 1 | Write-enable strobe into mode_register. |
| mr_seq_index_o | output | 5 | MR index (0..3) being shadowed. |
| mr_seq_data_o | output | 16 | MR data being shadowed. |

### 21.3.6	DRAM command request into the scheduler (issued while init_busy)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| init_cmd_valid_o | output | 1 | Single-cycle command pulse. |
| init_cmd_op_o | output | dram_op_e | OP_PREA, OP_REF, or OP_MRS. |
| init_cmd_bank_o | output | BKW | MR index for MRS (DDR2) / bank field. |
| init_cmd_row_o | output | ROW_WIDTH | MR data for MRS on the wide row path (see MR tables). |

### 21.3.7	Legacy ZQCL handshake (DDR3+; unused for DDR2/LPDDR2)

| Signal | Direction | Description |
| --- | --- | --- |
| zqcl_req_o | output | Tied to 0 — DDR2 has no ZQCL command. |
| zqcl_grant_i | input | Unused (tied off via a _unused sink). |

### 21.3.8	Status

| Signal | Direction | Description |
| --- | --- | --- |
| init_busy_o | output | Registered; high whenever r_state != S_DONE. Reset value is 1. |
| init_done_o | output | Registered; high when r_state == S_DONE. Drives refresh_ctrl.enable_i. |

## 21.4	FSM

A single 5-bit state_e enum drives everything. Each command state is occupied for exactly one cycle (an unconditional transition to S_WAIT), so the combinational command decode produces a single-cycle pulse per command. S_WAIT holds for the programmed inter-command delay: it counts r_wait down and, when it reaches zero, jumps to r_next (the resume state latched by the command state).

| State | Value | Role | Wait after (into S_WAIT) |
| --- | --- | --- | --- |
| S_RESET | 0 | Reset entry; unconditionally advances. | — (→ S_DFI_INIT) |
| S_DFI_INIT | 1 | Wait for dfi_init_complete_i, then arm W_INIT. | W_INIT |
| S_PREA1 | 2 | DDR2: Precharge All (pre-EMR). | W_RP → S_EMR2 |
| S_EMR3 | 3 | DDR2: MRS EMR(3). | W_MRD → S_EMR1 |
| S_EMR2 | 4 | DDR2: MRS EMR(2). | W_MRD → S_EMR3 |
| S_EMR1 | 5 | DDR2: MRS EMR(1). | W_MRD → S_MR0_DLL |
| S_MR0_DLL | 6 | DDR2: MRS MR0 + DLL reset (0x532). | W_DLL → S_PREA2 |
| S_PREA2 | 7 | DDR2: Precharge All (pre-refresh). | W_RP → S_REF1 |
| S_REF1 | 8 | DDR2: Auto Refresh #1. | W_RFC → S_REF2 |
| S_REF2 | 9 | DDR2: Auto Refresh #2. | W_RFC → S_MR0 |
| S_MR0 | 10 | DDR2: MRS MR0, DLL-reset bit cleared (0x432). | W_MRD → S_OCD_DEF |
| S_OCD_DEF | 11 | DDR2: MRS EMR(1) + OCD default (0x380). | W_MRD → S_OCD_EXIT |
| S_OCD_EXIT | 12 | DDR2: MRS EMR(1) + OCD exit (0x000). | W_MRD → S_DONE |
| S_WAIT | 13 | Inter-command countdown; r_wait==0 → r_next. | — |
| S_DONE | 14 | Terminal; init_done_o high, self-loop. | — |
| S_L_RESET | 15 | LPDDR2: MRW(MR63) Reset. | W_INIT → S_L_ZQ |
| S_L_ZQ | 16 | LPDDR2: MRW(MR10) ZQ Init Calibration. | W_DLL → S_L_MR1 |
| S_L_MR1 | 17 | LPDDR2: MRW(MR1) BL8 / nWR3. | W_MRD → S_L_MR2 |
| S_L_MR2 | 18 | LPDDR2: MRW(MR2) RL3 / WL1. | W_MRD → S_L_MR3 |
| S_L_MR3 | 19 | LPDDR2: MRW(MR3) drive strength 40ohm. | W_MRD → S_DONE |

memtype_i only branches once: on exit from S_DFI_INIT, r_next is set to S_PREA1 for DDR2 or S_L_RESET otherwise. From there each path is a fixed chain of command states separated by S_WAIT.

S_DONE is a hard self-loop — there is no restart, no init_error, and no software-triggered re-run in the RTL. A fresh init requires mc_rst_n.

## 21.5	DDR2 Sequence

Mirrors LiteDRAM’s get_ddr2_phy_init_sequence (litedram/init.py), the reference proven on the Nexys A7:

- Assert dfi_init_start_o; wait dfi_init_complete_i (PHY runs its own DLL-lock / IO training). Then wait W_INIT (tINIT / CKE settle).

- Precharge All (S_PREA1), wait W_RP.

- Load the extended mode registers in JEDEC JESD79-2 order EMR(2), EMR(3), EMR(1) — the state chain is S_EMR2 → S_EMR3 → S_EMR1 — each followed by W_MRD. (Note the enum numbering: S_EMR2 = 4, S_EMR3 = 3, so the values are out of numeric order but the executed order is 2 → 3 → 1.)

- Load MR0 + DLL reset (S_MR0_DLL, 0x532: BL4 / CL3 / tWR3 / DLL_RESET); wait W_DLL for the DLL to lock (~200 DRAM clocks).

- Precharge All (S_PREA2), wait W_RP.

- Auto Refresh x2 (S_REF1, S_REF2), each followed by W_RFC.

- Load MR0 with the DLL-reset bit cleared (S_MR0, 0x432); wait W_MRD.

- EMR(1) + OCD Default (S_OCD_DEF, 0x380) → EMR(1) + OCD Exit (S_OCD_EXIT, 0x000).

- S_DONE: init_done_o = 1.

MR data is carried on init_cmd_row_o (ROW_WIDTH wide, MR index on init_cmd_bank_o), because MR0 = 0x532 sets bit 10 and a 10-bit column-based path would truncate it.

### 21.5.1	DDR2 MR values

Localparams, transcribed exactly:

| Symbol | Value | State | Meaning | Shadowed? |
| --- | --- | --- | --- | --- |
| DDR2_MR0_DLL | 0x0532 | S_MR0_DLL | MR0: BL4 / CL3 / tWR3 / DLL_RESET. | Yes (MR0) |
| DDR2_MR0 | 0x0432 | S_MR0 | MR0 with DLL_RESET cleared. | Yes (MR0) |
| DDR2_MR1 | 0x0000 | S_EMR1, S_OCD_EXIT | EMR(1): Rtt disabled, ODS full. | Yes (MR1) |
| DDR2_MR1_OCD | 0x0380 | S_OCD_DEF | EMR(1) + OCD calibration default. | No — transient calibration; shadow left at final EMR value. |
| DDR2_MR2 | 0x0000 | S_EMR2 | EMR(2). | Yes (MR2) |
| DDR2_MR3 | 0x0000 | S_EMR3 | EMR(3). | Yes (MR3) |

DDR2_MR0_DLL decomposes as log2(BL=4)=2 | (CL=3<<4)=0x30 | (tWR=3<<9)=0x400 = 0x432, plus reset_dll = 1<<8 = 0x100, giving 0x532. OCD default = EMR | (7<<7) = 0x380; OCD exit = EMR = 0.

## 21.6	LPDDR2 Sequence

JEDEC JESD209-2F §3.4.1 power-up + §3.5 mode registers:

- Assert dfi_init_start_o; wait dfi_init_complete_i; wait W_INIT (tINIT settle).

- MRW(MR63) = Reset (S_L_RESET, OP don’t-care); wait W_INIT (tINIT4).

- MRW(MR10) = 0xFF ZQ Init Calibration (S_L_ZQ); wait W_DLL (tZQINIT).

- Configure the device: MRW(MR1) = BL8 / nWR3 (S_L_MR1, 0x23), MRW(MR2) = RL3 / WL1 (S_L_MR2, 0x01), MRW(MR3) = DS 40ohm (S_L_MR3, 0x02) — each followed by W_MRD (tMRW).

- S_DONE.

The CSR waits are reused: W_INIT covers tINIT4, W_DLL covers tZQINIT, W_MRD covers post-MRW settling.

LPDDR2 mode-register writes must reach indices up to MR63, which a 3-bit bank port cannot express. So the index (MA) and data (OP) are packed together into the wide row request by the mrw_row() function as {MA[5:0], OP[7:0]} (init_cmd_row_o[13:8] = MA index, init_cmd_row_o[7:0] = OP data); dfi_cmd_formatter unpacks this for the LPDDR2 CA MRW word.

### 21.6.1	LPDDR2 MR values

| Symbol | OP value | State | Meaning | Shadowed? |
| --- | --- | --- | --- | --- |
| LPDDR2_MR63_OP | 0x00 | S_L_RESET | MRW(63) Reset (OP don’t-care). | No — issued, not shadowed (MR63 exceeds the 5-bit index). |
| LPDDR2_MR10_OP | 0xFF | S_L_ZQ | MR10 ZQ Init Calibration. | No — issued, not shadowed. |
| LPDDR2_MR1_OP | 0x23 | S_L_MR1 | MR1: nWR3 ¦ BL8. | Yes (MR1). |
| LPDDR2_MR2_OP | 0x01 | S_L_MR2 | MR2: RL3 / WL1. | Yes (MR2). |
| LPDDR2_MR3_OP | 0x02 | S_L_MR3 | MR3: DS 40ohm. | Yes (MR3). |

Only MR1 / MR2 / MR3 update the mode_register shadow (mr_seq_we_o), because those are the registers that feed the CL/CWL/BL decode. MR63 and MR10 are issued to the DRAM but not shadowed.

## 21.7	Init-Busy Gating

init_busy_o is high from reset (its reset value is 1) through every state except S_DONE. While busy:

- The parent pumice_mem_cmd_scheduler forwards the sequencer’s command request to dfi_cmd_formatter and issues nothing of its own — the scheduler parks so exactly one command reaches DFI per init_cmd_valid_o pulse.

- refresh_ctrl.enable_i is low (driven by init_done_o), so periodic refresh does not start until init completes.

When the FSM reaches S_DONE, init_busy_o drops and init_done_o rises; the scheduler and refresh controller take over normal operation.

## 21.8	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| DDR2 init from cold reset to init_done_o | Full DDR2 state chain executes in order. |
| Observe the DDR2 command stream: PREA, MRS×3 (EMR2/EMR3/EMR1), MRS MR0_DLL, PREA, REF×2, MRS MR0, MRS OCD_DEF, MRS OCD_EXIT | Exact JEDEC order + op/index/data on each init_cmd_* pulse. |
| DDR2 MR values on init_cmd_row_o match 0x532 / 0x432 / 0x380 / 0x000 | Wide row path carries bit 10 without truncation. |
| LPDDR2 init from cold reset to init_done_o | Full LPDDR2 MRW chain executes. |
| LPDDR2 mrw_row() packing: MR63/MR10 reach the DRAM; only MR1/MR2/MR3 set mr_seq_we_o | Index/data packing + selective shadowing. |
| dfi_init_complete_i held low → FSM stalls in S_DFI_INIT, init_busy_o stays high | DFI handshake gating. |
| Each command state produces exactly one init_cmd_valid_o cycle, then S_WAIT | Single-cycle pulse behavior. |
| Program short t_*_wait_i values → inter-command S_WAIT countdown matches | CSR-backed wait timing. |
| mr_seq_* shadow tracks the MRS writes (MR0/MR1/MR2/MR3 for DDR2) | Shadow lockstep with issued commands; OCD_DEF not shadowed. |
| After init_done_o, refresh_ctrl.enable_i asserts and refresh begins | Init → refresh handoff. |

## 21.9	Open Questions / Future Work

- No restart / error path. S_DONE is terminal and there is no init_error, timeout, or software-triggered re-run — a fresh init requires mc_rst_n. If a DFI handshake never completes, the FSM simply parks in S_DFI_INIT forever. A future variant could add a timeout on dfi_init_complete_i and a status/restart CSR, but that machinery is not in the current RTL.

- MR4 readback (LPDDR2). LPDDR2 supports reading MR4 for device temperature class. The current sequencer only writes mode registers; it does no MRR readback. Folding a temperature read into or after init is possible future work but is not implemented.

- ZQCL / ZQCS. The zqcl_req_o / zqcl_grant_i ports exist only as a legacy DDR3+ handshake and are tied off; DDR2 has no ZQCL and LPDDR2 does its ZQ init via the MR10 MRW. If the family is extended to DDR3/DDR4, this port becomes a real ZQ-calibration handshake and the FSM would gain a ZQ state.

- Per-rank MRS. The sequencer issues a single stream of commands with no rank iteration; multi-rank support would require a rank field and a per-rank MRS loop. Out of scope for the current single-rank Nexys A7 target.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Power-Down Controller (powerdown_ctrl)

Module: powerdown_ctrl.sv Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: Live but optional — a single-channel idle-detect power-down requester. Not instantiated in the default top build. Deep Power Down, per-rank control, and refresh/init interlocks are TODO (see Open Questions).

Renamed / rescoped. An earlier SWAG called this power_state_fub and described a per-rank FSM array with a separate DEEP_POWER_DOWN state, a quiet-point detector, self-refresh coordination handshake with the refresh manager, split APD/SRF idleness counters, an init-time CKE handoff mux, and a large CSR/IRQ interface. None of that is implemented. The actual RTL is a single channel-wide 4-state FSM (S_AWAKE / S_ARMING / S_REQ / S_ASLEEP) that detects controller idleness and requests either Precharge Power Down (PDE) or Self Refresh (SR). CKE is dropped for all ranks together. This chapter describes what the RTL actually does.

## 22.1	Purpose

powerdown_ctrl is a small idle-detector that asks the scheduler for permission to put the DRAM into a low-power state, then drops the DFI clock-enable (CKE) when granted. It supports two low-power flavors, selected by two enable inputs:

- PDE (Precharge Power Down) — CKE low, banks may stay precharged; wakes on any new controller activity.

- SR (Self Refresh) — CKE low while the DRAM self-refreshes internally. SR takes priority over PDE when both are enabled.

The FSM only drops and raises CKE. It does not issue the SREFE/SREFX DRAM commands, does not run any exit-timing counters (tXP / tXSDLL / tXSR), and does not coordinate a handshake with the refresh controller. All command issue, precondition enforcement (all banks precharged before SR), and exit timing are the responsibility of the parent scheduler (pumice_mem_cmd_scheduler) — this FUB is purely a request/grant + CKE-drive block. It trusts the scheduler not to issue a grant before DRAM init completes.

## 22.2	Synthesis Parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| NUM_RANKS | 1 | Number of ranks. CKE is driven identically to all ranks (see below). |
| CS_WIDTH | NUM_RANKS | Width of the dfi_cke_o output vector. |

There is no MEMTYPE parameter and no build-time DPD/PDE/SR selection — the enable inputs select behavior at run time.

## 22.3	FSM

The block is a single channel-wide FSM (state_e, 3-bit encoded), not a per-rank array. There is no separate Deep Power Down state.

| State | Encoding | Meaning | dfi_cke_o |
| --- | --- | --- | --- |
| S_AWAKE | 3'd0 | CKE high, normal operation; idle counter held at 0 | '1 |
| S_ARMING | 3'd1 | Controller idle; counting cycles toward entry | '1 |
| S_REQ | 3'd2 | pdn_req_o high, awaiting pdn_grant_i | '1 |
| S_ASLEEP | 3'd3 | Granted; CKE low, in PDE or SR per latched r_kind | '0 |

Reset state is S_AWAKE with dfi_cke_o = '1 (CKE high out of reset).

### 22.3.1	PDE-vs-SR Selection

The request is enabled when either mode is enabled: w_request_enabled = enable_pde_i || enable_sref_i.

- enable_sref_i high → on reaching the idle threshold, request SR (pdn_kind_o = 1). SR has priority over PDE.

- enable_sref_i low, enable_pde_i high → request PDE (pdn_kind_o = 0).

- Neither enabled → never request; the FSM stays in S_AWAKE.

The kind is latched into r_kind at the S_ARMING → S_REQ transition (r_kind <= enable_sref_i) and is held through S_ASLEEP.

### 22.3.2	Transitions

S_AWAKE:
  r_idle_cnt <- 0
  if (w_request_enabled && controller_idle_i) -> S_ARMING

S_ARMING:
  if (!controller_idle_i || !w_request_enabled) -> S_AWAKE, r_idle_cnt <- 0
  else if (r_idle_cnt >= idle_threshold_i)      -> S_REQ, r_kind <- enable_sref_i
  else                                          r_idle_cnt <- r_idle_cnt + 1

S_REQ:
  if (!controller_idle_i || !w_request_enabled) -> S_AWAKE, r_idle_cnt <- 0
  else if (pdn_grant_i)                         -> S_ASLEEP
                                                   (r_grants_sr++ if r_kind else r_grants_pde++)

S_ASLEEP:
  if (!controller_idle_i)                       -> S_AWAKE, r_idle_cnt <- 0
  // wakes on ANY new activity; SR exit timing (tXSDLL/tXSR) is enforced
  // by the scheduler, not here

Any loss of controller_idle_i (or de-assertion of both enables) at S_ARMING or S_REQ backs the FSM off to S_AWAKE. From S_ASLEEP, the FSM wakes on any new controller activity and returns to S_AWAKE; the scheduler is responsible for honoring exit latency before issuing commands to the DRAM.

The grant counters r_grants_pde / r_grants_sr (16-bit) increment on each successful grant and are exposed as observability outputs.

### 22.3.3	CKE Behavior

dfi_cke_o is a strict-flop output: '1 out of reset and whenever the FSM is not in S_ASLEEP; '0 in S_ASLEEP. CKE is CS_WIDTH-wide but every bit is driven identically — the block powers down all ranks together. Per-rank CKE control is a documented TODO.

All outputs are registered (strict-flop), so pdn_req_o, pdn_kind_o, sref_active_o, dfi_cke_o, and the obs_* signals lag the internal state by one cycle.

## 22.4	Interface

Clock mc_clk, active-low reset mc_rst_n.

### 22.4.1	Control / Configuration Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| idle_threshold_i | input | 16 | Idle cycles to count in S_ARMING before requesting power-down |
| enable_pde_i | input | 1 | Enable Precharge Power Down requests |
| enable_sref_i | input | 1 | Enable Self Refresh requests; takes priority over PDE |
| controller_idle_i | input | 1 | Controller idle indication from the scheduler |

### 22.4.2	Request / Grant + CKE

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| pdn_req_o | output | 1 | High while in S_REQ (requesting power-down) |
| pdn_kind_o | output | 1 | Requested kind: 0 = PDE, 1 = SR |
| pdn_grant_i | input | 1 | Scheduler grant; S_REQ → S_ASLEEP on assertion |
| sref_active_o | output | 1 | High while asleep in SR (S_ASLEEP && r_kind == 1) |
| dfi_cke_o | output | CS_WIDTH | DFI clock-enable; '0 in S_ASLEEP, else '1 |

### 22.4.3	Observability (future CSR readout)

These outputs are provided for future CSR readback; there is no CSR/APB interface wired into this FUB today.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| obs_state_o | output | 3 | Current FSM state (state_e encoding) |
| obs_idle_cnt_o | output | 16 | Current idle counter value |
| obs_grants_pde_o | output | 16 | Count of PDE grants taken |
| obs_grants_sr_o | output | 16 | Count of SR grants taken |

## 22.5	Verification Notes (cocotb test plan)

The block is small and self-contained; a directed FSM testbench is sufficient.

| Scenario | What it proves |
| --- | --- |
| Reset → S_AWAKE, dfi_cke_o = '1, all obs_* cleared | Reset behavior and CKE-high default |
| Neither enable set; hold idle → FSM never leaves S_AWAKE | No request when disabled |
| enable_pde_i only; idle for idle_threshold_i cycles → pdn_req_o, pdn_kind_o = 0 | PDE arming/threshold and request |
| enable_sref_i set (with or without PDE) → pdn_kind_o = 1 | SR priority over PDE |
| S_REQ then pdn_grant_i → S_ASLEEP, dfi_cke_o = '0, sref_active_o per kind | Grant, sleep, CKE drop, SR-active flag |
| Activity (!controller_idle_i) during S_ARMING → back to S_AWAKE, counter cleared | Arming back-off |
| Activity during S_REQ (before grant) → back to S_AWAKE | Request withdrawal on new activity |
| Wake from S_ASLEEP on !controller_idle_i → S_AWAKE, dfi_cke_o = '1 | Wake path |
| Both enables dropped mid-arming / mid-request → back-off to S_AWAKE | Enable de-assertion handling |
| Multiple sleep cycles → obs_grants_pde_o / obs_grants_sr_o increment correctly | Grant counters |
| idle_threshold_i = 0 → request on first idle cycle in S_ARMING | Threshold edge case |

## 22.6	Open Questions / Future Work

All of the following are called out in the RTL header comments and are not implemented today:

- Deep Power Down (DPD). LPDDR2-only. Would need an enable_dpd_i input and dfi_dram_clk_disable_o cooperation from dfi_signal_pack. There is no DPD state in the current FSM.

- Per-rank power-down. The block currently powers down all ranks together (identical CKE on every dfi_cke_o bit). A per-rank FSM array with independent CKE routing is future work.

- dfi_init_complete interlock. The FSM currently trusts the scheduler not to issue pdn_grant_i before DRAM init completes. A hard interlock input is future work.

- Self-refresh command / handshake ownership. SREFE/SREFX command issue, the all-banks-precharged precondition, and exit-timing enforcement (tXP / tXSDLL / tXSR) live in the scheduler, not here. There is no refresh_ctrl coordination handshake in this FUB.

- CSR / observability wiring. The obs_* outputs are intended for future CSR readback but are not connected to any APB register block yet.

- Not in the default top build. The module is live and verifiable standalone, but is not instantiated in the default pumice top today.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Mode Register (mode_register)

Module: mode_register.sv Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: implemented (DDR2 + LPDDR2 decode)

## 23.1	Purpose

Per-rank Mode Register shadow + live decode of MR-derived timing values for use by the rest of the controller.

On mr_we_i, write mr_data_i into shadow MR[mr_index_i] for mr_rank_i (indices < MAX_MR_IDX only). init_sequencer drives this during DRAM bring-up; a CSR/APB hot-update path drives it later.

The memtype_i input (memtype_e: MEMTYPE_DDR2 / MEMTYPE_LPDDR2) selects the decode branch for every output. The live decode reads from rank 0 (multi-rank designs must program matching MR values across ranks; mixed-per-rank MRs are a TODO).

## 23.2	Live Decoded Outputs

| Output | Source (DDR2) | Source (LPDDR2) |
| --- | --- | --- |
| cl_o | MR0[6:4] | RL, from the MR2[3:0] RL&WL enum |
| cwl_o | CL − 1 | WL, from the MR2[3:0] RL&WL enum |
| bl_o | MR0[2:0] | MR1[2:0] (4/8; BL16 clips to 8) |
| al_o | MR1[5:3] | 0 (not used) |
| drv_strength_o | MR1[1] | 0 (informational) |
| odt_o | MR1[6,2] | 0 |

For LPDDR2, CL and CWL are not independent MR fields — they are both derived from a single MR2[3:0] “RL & WL” enum per JESD209-2F:

| MR2[3:0] | RL (cl_o) | WL (cwl_o) |
| --- | --- | --- |
| 0001 | 3 | 1 |
| 0010 | 4 | 2 |
| 0011 | 5 | 2 |
| 0100 | 6 | 3 |
| 0101 | 7 | 4 |
| 0110 | 8 | 4 |

(default / illegal codes fall back to RL3/WL1.)

For DDR2, cl_o is the raw MR0[6:4] field and cwl_o is simply CL − 1 (saturating at 0). bl_o decodes MR0[2:0] = 010 → BL4, 011 → BL8.

All outputs are strict-flop registered. Consumed by:

- pumice_cmd_arbiter / pumice_mem_cmd_scheduler — use cl_o, cwl_o, al_o to time RD/WR latencies.

- write data path (pumice_dfi_wr_serializer via t_phy_wrlat) — sized against cwl_o for the WR-to-wrdata window.

- read data path (pumice_dfi_rd_aligner via t_rddata_en) — sized against cl_o for the RD-to-rddata window.

- dfi_cmd_formatter — uses bl_o to size column commands.

## 23.3	Burst Length: Fixed per Instance, Parameterized for Family Reuse

A memory controller supports exactly one DRAM burst length, chosen at init and fixed for the life of the instance — the datapath (beat sequencing, prefetch depth, column-stride math, tWR/tRTP-derived timing windows) is sized around that single value. pumice does not switch BL per transaction; the host/AXI burst is arbitrary and axi_intake splits it into fixed-BL DRAM commands.

bl_o is therefore a build/init constant, not a per-command control. It is decoded from the mode register rather than hardcoded so the same RTL retargets across the DDR family — the only thing that changes is the programmed value and the device/beat widths:

| Family | Burst length | Source | pumice-beat count (via bl_dram_beats) |
| --- | --- | --- | --- |
| DDR2 | BL4 (fixed) | MR0[2:0]=010 | bl_o >> log2(DRAM_BEAT_WIDTH/DRAM_DEVICE_WIDTH) |
| DDR3 | BL8 (fixed) | MR0[1:0] | same expression, bl_o=8 |
| DDR4 | BL8 (fixed) | MR0[1:0] | same expression, bl_o=8 |

Everything downstream keys off the decoded bl_o and the device/beat widths (see §15 “Narrow-Device (x16) Support”), so no burst-length constants are baked into the sequencers or the address decode. BYTE_OFFSET_WIDTH keys off device width, not BL, so the column granularity is already BL-agnostic.

BC4 / burst-chop (DDR3/DDR4): out of scope — always issue full BL8. DDR3 and DDR4 allow BC4 and BL8-on-the-fly (A12). That is the only case where one instance would see two effective burst lengths per transaction, which would break the fixed-BL datapath invariant. The design decision is to not support BC4: always transfer the full fixed BL8. Revisit only if a workload demonstrably needs the partial-burst bandwidth (it usually does not).

## 23.4	Scope

- MAX_MR_IDX=17 (indices 0..16) covers both DDR2 (MR0..MR3) and LPDDR2 (up to MR16). The shadow array is [NUM_RANKS][MAX_MR_IDX].

- LPDDR2 BL16 clips to BL8 because bl_o is 4-bit. A follow-up widens bl_o to [4:0] and updates the 3 downstream macros that consume it.

- mr_req_o is tied 0 — no hot MR updates are issued via the scheduler. Init does the MR loads directly through this FUB’s CSR write port. The mr_req_* channel is still flopped (all-zero) for port consistency and lands when the APB CSR slave provides a write-during-traffic path with a quiet-point handshake.

## 23.5	Tests

Verified by dv/tests/fub/test_mode_register.py: smoke_ddr2, ddr2_cl_sweep, reset_values, ddr2_bl_sweep, ddr2_al_sweep, multi_rank (plus LPDDR2 RL/WL-enum coverage as the suite is extended for the memtype_i LPDDR2 branch).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Write Data Path (pumice_wr_data_cam + pumice_dfi_wr_serializer)

Modules: pumice_wr_data_cam.sv, pumice_dfi_wr_serializer.sv Location: rtl/fub/ Category: FUB Parents: pumice_axi4_ifc (CAM), pumice_dfi_layer (serializer) Status: implemented

The old single-block wr_beat_sequencer / wr_data_path_fub no longer exists. In the rearchitected controller the write data path is split across two clock domains and two FUBs:

- pumice_wr_data_cam (MC domain, inside pumice_axi4_ifc) — a write-command CAM plus a wr-data SRAM with three de-FSM’d movers.

- pumice_dfi_wr_serializer (DFI domain, inside pumice_dfi_layer) — a purely mechanical DFI-word streamer that drives dfi_wrdata.

The single clock-domain crossing between them is pumice_dfi_cdc (async gaxi FIFOs). This chapter documents both halves.

Architectural context: HAS §3.7 and _SWEEP_GROUND_TRUTH.md §8. Both halves follow the repository “streaming pipeline + flags” model — no enumerated datapath FSM in the CAM movers (the DFI serializer has a tiny 3-state pacing FSM only, described below). Sequence is enforced by data-flow handshakes and burst beat-counters, not by slot state latches.

## 24.1	Purpose

The write data path stages the AXI write burst that arrives on the AW/W channels, holds it while the scheduler picks a moment to commit it to DRAM, and — when the DFI layer is ready — streams it onto the DFI write bus one DFI word per cycle.

pumice_wr_data_cam is the MC-domain staging structure. It stores each pending write’s key {bank, row, col}, its AXI id, a free-running age, and the burst’s beats in an SRAM slot. It exposes associative query ports so the pumice_cmd_arbiter can find row-hit writes, an oldest-entry port as a scheduler fallback, and a snarf (read-your-write) forwarding port for pumice_rd_intake. On commit it drains the SRAM slot into the DFI layer.

pumice_dfi_wr_serializer is the DFI-domain endpoint. On a WR command strobe (wr_fire_i) it waits t_phy_wrlat DFI cycles, then pops one DFI word per cycle from its write-data FIFO onto dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask until the burst’s last word.

## 24.2	pumice_wr_data_cam — three movers over one SRAM

The CAM never runs a datapath state machine. It has one SRAM (r_sram[N_SRAM_SLOTS*BL], plus a companion strobe array r_strb) and three movers, each of which is just a burst beat-counter reading the head of a request FIFO:

| Mover | Trigger source | Direction | Beat counter |
| --- | --- | --- | --- |
| fill | u_fill_q (insert-order slot FIFO) | wd_data_i → SRAM | r_fill_beat |
| snarf | u_snarf_q (accept-order slot FIFO) | SRAM → snarf_rd | r_sn_beat |
| commit | u_drain_q (scheduled-slot FIFO) | SRAM → cm_rd | r_cm_beat |

Each mover streams straight off its request-FIFO head, which is stable until popped. The head slot IS the “active” slot; “active” simply means the FIFO is non-empty; the FIFO is popped on the burst’s last beat. This is the de-FSM’d streaming reader pattern — no active/slot latch.

### 24.2.1	Entry state

Per entry (NUM_ENTRIES, default 8): r_valid, r_bank, r_row, r_col, r_id, r_age, plus:

- r_ptr — SRAM slot index, set on the entry’s first fill beat.

- r_pv — pointer-valid (first beat has been staged).

- r_fdone — fill COMPLETE (wd_last has been seen). An entry may be scheduled or snarfed only once r_fdone is set; otherwise the commit-drain mover could outrun a gapped fill and read stale SRAM beats.

- r_sched — the scheduler has committed this slot. It is set the cycle commit_valid_i fires and immediately excludes the entry from the sched_lu / oldest ports, so the arbiter can pick the next entry the very next cycle (clean one-command-per-clock). It clears on evict.

r_age_ctr is a free-running counter; relative age is w_rel[i] = r_age_ctr - r_age[i], which is wrap-safe.

### 24.2.2	SRAM slot pre-allocation

N_SRAM_SLOTS may be less than NUM_ENTRIES. r_sram_occ is a per-SRAM-slot occupancy bitmap. On the first fill beat of an entry a free slot is chosen (w_slot_free, highest-index-first scan), recorded into r_ptr, and marked occupied. The slot is freed on the entry’s commit evict. wd_ready_o deasserts on the first beat if no SRAM slot is free.

### 24.2.3	Age-based selectors

Three selectors run combinationally over the entry array (all skip already-scheduled and not-fill-complete entries where applicable):

- oldest port — oldest valid, fill-complete, unscheduled entry (max relative age). Scheduler fallback.

- scheduler lookups (N_SCHED_LU, default 4) — for each {bank, row} query, the oldest matching fill-complete unscheduled entry. This gives the arbiter in-order commit per row.

- snarf lookup — the youngest match (min relative age), so a read sees the latest write data.

## 24.3	Snarf: write-to-read forwarding (was the standalone wr2rd_forward)

There is no standalone wr2rd_forward FUB anymore. Write-to-read forwarding is the snarf mover inside pumice_wr_data_cam. When pumice_rd_intake presents an AR’s decoded key on the snarf probe port, the CAM combinationally searches for a matching in-flight write. The match is deliberately narrow — a write is snarfable only when all three hold:

- Not yet scheduled (!r_sched). A scheduled write is draining/evicting to DRAM, so its CAM data is racy.

- Same AXI id (r_id[i] == snarf_probe_id_i). Same-id write-before-read is the only AXI-ordered case where the read is required to observe the write; cross-id has no ordering guarantee, so a cross-id read takes the DRAM path instead.

- Same burst length — snarf_probe_len_i == BL-1. Every admitted write is exactly BL beats (ragged bursts are rejected upstream in pumice_wr_intake), so this reduces to arlen == BL-1. A short or long read must not snarf a full-BL write.

The match must also be fill-complete (r_fdone). Among candidates the youngest is chosen. snarf_hit_o asserts when a valid probe finds a match with matching length. On snarf_accept_i, the matched slot is pushed into u_snarf_q; the snarf mover then streams snarf_rd_data_o beats (non-destructively — snarf does not evict the write) in accept order.

The old chapter’s “last-write-wins highest-slot-index” rule and the “any matching in-flight write regardless of id” behavior are gone. The live policy is youngest-match, same-id, same-BL, unscheduled only.

## 24.4	Commit drain and B response

The arbiter’s commit does two things atomically: it sets r_sched on the slot (immediate exclusion from the pick ports) and enqueues the slot into u_drain_q. The commit mover works through scheduled slots at its own pace, streaming cm_rd_data_o / cm_rd_strb_o to the DFI write path and popping u_drain_q on the last beat. On that last beat the entry is evicted (r_valid, r_pv, r_fdone, r_sched cleared; SRAM slot freed) and commit_done_valid_o / commit_done_id_o strobe, which drives the pumice_wr_intake B response. Decoupling the scheduled-mark from the drain is what allows one-command-per-clock scheduling.

## 24.5	pumice_dfi_wr_serializer — the DFI-domain endpoint

This FUB is purely mechanical and lives in the DFI clock domain. Its internal datapath unit is the DFI word (DFI_DATA_WIDTH, default 128), which already carries all DFI_RATE phases; dfi_signal_pack splits it to the pins, so there is no per-beat packing here.

Interface (abridged):

| Signal | Dir | Description |
| --- | --- | --- |
| t_phy_wrlat_i | in | WR-cmd → wrdata_en delay (DFI cycles) |
| wr_fire_i | in | WR command strobe (from pumice_dfi_cmd_path) |
| wd_valid/ready/data/strb/last | in/out | Pre-staged write-data FIFO (DFI-word granular) |
| dfi_wrdata_o | out | = wd_data_i while driving |
| dfi_wrdata_en_o | out | all-ones while driving, else 0 |
| dfi_wrdata_mask_o | out | ~wd_strb_i while driving (AXI wstrb=1 write → DFI mask=1 means “mask”, i.e. mask = ~strb) |

It has a small 3-state pacing FSM (S_IDLE, S_WAIT, S_DRIVE) — the only FSM in the whole write path, and it exists to time t_phy_wrlat, not to route data:

- S_IDLE: on an available fire, if t_phy_wrlat==0 drive word 0 combinationally this cycle (via w_start_now) and go straight to S_DRIVE (or stay idle for a single-word burst); if ==1 go to S_DRIVE; else load r_wait = t_phy_wrlat-1 and go S_WAIT.

- S_WAIT: count down, enter S_DRIVE at fire+wrlat.

- S_DRIVE: pop one DFI word per cycle. On the last word, if another fire is pending continue seamlessly into the next burst (zero bubbles); else return to S_IDLE.

A r_pending counter (3-bit) absorbs wr_fire strobes that arrive while a burst is in flight. This matters: the DFI cmd path paces column commands exactly BL_WORDS DFI cycles apart (the burst’s DQ-bus occupancy), so the next burst’s first word is due the cycle right after the current burst’s last word. Because the data is pre-staged in the FIFO (the CAM drains it when the command is scheduled), at wr_fire the burst is already waiting, so the drive never stalls and bursts run back-to-back with zero bubbles.

Fixed bug (see project notes): a fire arriving mid-burst used to be dropped, stranding the write in the PHY. It is now counted in r_pending.

## 24.6	Block Pipeline View

Write Data Path — streaming CAM movers + DFI-word serializer, no datapath FSM

Source: 15_wr_data_path_pipeline.mmd

## 24.7	pumice_wr_data_cam interface

### 24.7.1	Insert (from pumice_wr_intake aw_push)

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| ins_valid_i | in | 1 | Allocate an entry |
| ins_ready_o | out | 1 | Free entry AND fill-FIFO room |
| ins_bank_i | in | BKW | Decoded bank |
| ins_row_i | in | ROW_WIDTH | Decoded row |
| ins_col_i | in | COL_WIDTH | Decoded column |
| ins_id_i | in | IW | AXI id |

### 24.7.2	Fill data (from pumice_wr_intake wdata pop)

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| wd_valid_i | in | 1 | Write-data beat valid |
| wd_ready_o | out | 1 | Fill-FIFO head present AND SRAM slot free |
| wd_data_i | in | DW | Beat data |
| wd_strb_i | in | SW | Byte strobes |
| wd_last_i | in | 1 | Last beat (sets r_fdone, pops fill FIFO) |

### 24.7.3	Snarf lookup + stream (to/from pumice_rd_intake)

| Signal | Dir | Description |
| --- | --- | --- |
| snarf_probe_* | in | Probe key {bank,row,col}, id, len |
| snarf_hit_o | out | Valid probe matched (youngest, same-id, same-BL) |
| snarf_accept_i | in | Read admitted as a snarf; push slot to snarf FIFO |
| snarf_rd_valid/ready/data/last | out/in | Snarf data stream (accept order) |

### 24.7.4	Oldest port + scheduler lookups (to pumice_cmd_arbiter)

| Signal | Dir | Description |
| --- | --- | --- |
| oldest_valid_o + oldest_{bank,row,col,id,slot}_o | out | Oldest fill-complete unscheduled entry |
| sched_lu_valid_i[N] + sched_lu_{bank,row}_i | in | Per-port {bank,row} queries |
| sched_lu_hit_o[N] + sched_lu_{slot,col,id,age}_o | out | Oldest matching entry per port |

### 24.7.5	Commit / drain (to pumice_dfi_layer write path)

| Signal | Dir | Description |
| --- | --- | --- |
| commit_valid_i | in | Arbiter commits a slot |
| commit_ready_o | out | Room in the drain FIFO |
| commit_slot_i | in | Slot to schedule |
| cm_rd_valid/ready/data/strb/last | out/in | Commit-drain data stream |
| commit_done_valid_o + commit_done_id_o | out | Evict strobe → drives B response |
| busy_o | out | Any pending fill / snarf / drain / oldest |

## 24.8	pumice_dfi_wr_serializer parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| DFI_DATA_WIDTH | 128 | DFI-word width (= DRAM_BEAT_WIDTH*DFI_RATE) |
| DFI_RATE | 2 | Phases per DFI word (= DFI_EN_WIDTH) |
| DFI_STRB_WIDTH | DW/8 | Byte-mask width |
| WRLAT_W | 8 | Width of t_phy_wrlat_i |

## 24.9	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Insert + fill a BL burst; r_fdone sets only on wd_last | Fill mover + fill-complete gate |
| Commit an entry; SRAM drains in order; evict + commit_done on last | Commit mover + B-response strobe |
| Snarf hit: same-id, same-BL, unscheduled write returns its data | Snarf youngest-match forwarding |
| Snarf miss: cross-id read, or scheduled write, or mismatched len | Snarf policy exclusions |
| Oldest / sched-lookup exclude scheduled + not-fill-complete entries | Pick-port correctness (1 cmd/clock) |
| SRAM slot exhaustion: wd_ready_o / ins_ready_o deassert | Pre-allocation backpressure |
| DFI serializer: t_phy_wrlat sweep (0, 1, N); first word at fire+wrlat | Write-latency alignment |
| Back-to-back WR fires paced BL_WORDS apart: zero-bubble drive | r_pending seamless continuation |
| wr_fire mid-burst is counted, not dropped | r_pending regression |
| dfi_wrdata_mask_o == ~wd_strb_i while driving | Strobe → mask inversion |

## 24.10	Open Questions / Future Work

- Cross-burst commit interleave. The commit mover drains one scheduled slot at a time. Interleaving beats from multiple scheduled slots would need a per-slot beat-counter shadow. Punt to a later performance mode.

- Multi-outstanding DFI reads/writes. The serializer’s r_pending counter is 3-bit; deeper pipelining is bounded by the DFI cmd path’s pacing, not by this FUB.

- Strobe-less writes. Hosts that always write full beats could elide the strobe SRAM and mask path at elaboration. Minor area win; punt.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Read Data Path (pumice_rd_cmd_cam + pumice_dfi_rd_aligner)

Modules: pumice_rd_cmd_cam.sv, pumice_dfi_rd_aligner.sv Location: rtl/fub/ Category: FUB Parents: pumice_axi4_ifc (CAM), pumice_dfi_layer (aligner) Status: implemented

The old single-block rd_cl_aligner / rd_data_path_fub no longer exists. In the rearchitected controller the read data path is split across two clock domains and two FUBs:

- pumice_dfi_rd_aligner (DFI domain, inside pumice_dfi_layer) — drives dfi_rddata_en at the right cycle for an issued READ and captures dfi_rddata words into the read return FIFO.

- pumice_rd_cmd_cam (MC domain, inside pumice_axi4_ifc) — an outstanding-read reorder buffer: DRAM data returns in issue order and buffers per entry; it drains to pumice_rd_intake in AR order.

The single clock-domain crossing between them is pumice_dfi_cdc (async gaxi FIFOs). This chapter documents both halves.

Architectural context: HAS §3.7 and _SWEEP_GROUND_TRUTH.md §8. The CAM has no datapath FSM — the movers are burst beat-counters over the SRAM. The DFI aligner has a small pacing FSM for the dfi_rddata_en window only. Sequence is enforced by data-flow handshakes, not by slot state.

## 25.1	Purpose

The read data path is the miss path: reads that cannot be satisfied by a snarf from the write CAM (see §17) go to DRAM. pumice_rd_cmd_cam is the mirror of pumice_wr_data_cam — entries keyed {bank, row, col} with a free-running age, an oldest port, and N_SCHED_LU scheduler lookups, so reads row-hit-schedule exactly like writes. It also acts as a reorder buffer: it accepts AR inserts in AR order, lets the scheduler issue them in an order it chooses (row-hit optimized), receives DRAM data in issue order, and drains it back to the host in AR/oldest order.

pumice_dfi_rd_aligner is the DFI-domain endpoint. On a RD command strobe (rd_fire_i) it drives the dfi_rddata_en capture window starting t_rddata_en DFI cycles later, and pushes one DFI word per dfi_rddata_valid cycle into the read return FIFO as {data, resp, last}.

Compared to the write CAM there is no snarf port here — snarf is a write-CAM concept (§17). Data flows in from the DFI return and out to pumice_rd_intake.

## 25.2	pumice_rd_cmd_cam — reorder buffer, two movers over one SRAM

The CAM stores each burst’s return data in an SRAM (r_sram[N_SRAM_SLOTS*BL]) and runs two de-FSM’d movers, each a burst beat-counter:

| Mover | Trigger source | Direction | Beat counter |
| --- | --- | --- | --- |
| return-fill | u_issue_q (issue-order slot FIFO) | dfi_ret → SRAM | r_ret_beat |
| drain | oldest-valid pick, data-ready gated | SRAM → drain | r_dr_beat |

### 25.2.1	Entry state and age

Per entry (NUM_ENTRIES, default 8): r_valid, r_issued (scheduler has issued it to DRAM), r_ready (return data complete), r_bank, r_row, r_col, r_id, r_resp, r_age, plus r_ptr (SRAM slot, set on the first return beat) and r_pv (pointer valid). r_age_ctr is free-running; relative age w_rel[i] = r_age_ctr - r_age[i] is wrap-safe.

SRAM slot pre-allocation is identical to the write CAM: r_sram_occ bitmap, highest-index-first free scan, allocate on first return beat, free on drain evict.

### 25.2.2	The three age selectors

- issue-side oldest — oldest valid not-yet-issued entry (max rel). Scheduler fallback; drives the oldest_* port.

- scheduler lookups (N_SCHED_LU, default 4) — per {bank, row} query, the oldest not-issued match. Row-hit read scheduling.

- drain-side oldest — oldest valid entry over all valid entries (max rel), regardless of issued/ready. This is w_dro_slot.

### 25.2.3	Ordering guarantee

Inserts happen in AR order and each new insert is younger, so the drain-side oldest pick w_dro_slot is stable across a burst: the entry stays valid until its own last-beat evict, and later inserts can never become “more oldest”. The drain mover therefore needs no active latch — the draining slot IS the oldest-valid pick. It only fires when that oldest entry’s data is staged: w_dr_go = w_dro_found && r_ready[w_dro_slot]. This is what enforces AR-order release even though DRAM returns arrive in issue order.

### 25.2.4	Issue-order return fill

When the scheduler issues a read it notifies the CAM (issue_valid_i / issue_slot_i), which sets r_issued[slot] and pushes the slot into u_issue_q. DRAM returns arrive in the same order the reads were issued, so the return-fill mover writes each return burst into the issue-FIFO head slot’s SRAM region. On the burst’s dfi_ret_last, it sets r_ready, captures r_resp, and pops u_issue_q.

## 25.3	pumice_dfi_rd_aligner — the DFI-domain endpoint

This FUB mirrors pumice_dfi_wr_serializer. Its internal unit is the DFI word (= dfi_rddata width); BL_WORDS = BL/DFI_RATE words per burst.

It has two independent concerns:

1. dfi_rddata_en window (small pacing FSM). A 3-state FSM (S_IDLE, S_WAIT, S_EN) plus a pending-fire counter r_epend:

- S_IDLE: on an available fire, if t_rddata_en==0 drive word 0’s enable combinationally (w_en_now) and seed the remaining-cycle counter for BL_WORDS-1 more en cycles; if ==1 go straight to S_EN; else load r_ewait = t_rddata_en-1 and go S_WAIT.

- S_WAIT: count down, enter S_EN at fire+t_rddata_en.

- S_EN: assert dfi_rddata_en_o for the window. On the last en-cycle, if another fire is pending reseed for a contiguous back-to-back window (zero bubbles); else return to S_IDLE.

Like the write serializer, the DFI cmd path paces column commands BL_WORDS apart, so windows abut with zero bubbles. A rd_fire arriving mid-window is counted in r_epend (previously it was dropped, leaving the second read with no capture window and stranding it in the PHY).

2. Capture (flag-and-counter only). On each dfi_rddata_valid (w_word_valid = |dfi_rddata_valid_i) a word is pushed to the read FIFO: rd_data_o = dfi_rddata_i, rd_resp_o = OKAY, and rd_last_o asserts on word BL_WORDS-1. A single word counter r_rcnt tracks progress; it wraps on the last word.

v1 note: rd_resp_o is hardwired RESP_OKAY — the DFI rddata-error signal is not yet wired. The aligner supports one outstanding read window (single-issue, tRTW/tCCD spaced by the scheduler / global_timers).

## 25.4	Block Pipeline View

Read Data Path — DFI en-window aligner + issue-order fill + AR-order drain reorder buffer

Source: 16_rd_data_path_pipeline.mmd

## 25.5	pumice_rd_cmd_cam interface

### 25.5.1	Insert (from pumice_rd_intake ar_push, AR order)

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| ins_valid_i | in | 1 | Allocate an entry |
| ins_ready_o | out | 1 | Free entry available |
| ins_bank/row/col/id_i | in | key + id | Decoded key + AXI id |

### 25.5.2	Scheduler ports (to pumice_cmd_arbiter)

| Signal | Dir | Description |
| --- | --- | --- |
| sched_lu_valid_i[N] + sched_lu_{bank,row}_i | in | Per-port {bank,row} queries |
| sched_lu_hit_o[N] + sched_lu_{slot,col,id,age}_o | out | Oldest not-issued match per port |
| oldest_valid_o + oldest_{bank,row,col,id,slot}_o | out | Oldest not-issued entry |

### 25.5.3	Issue notify (from scheduler)

| Signal | Dir | Description |
| --- | --- | --- |
| issue_valid_i | in | Scheduler issued this slot to DRAM |
| issue_ready_o | out | Room in the issue-order FIFO |
| issue_slot_i | in | Slot issued (records issue order for return fill) |

### 25.5.4	DFI return (from pumice_dfi_layer read path, issue order)

| Signal | Dir | Description |
| --- | --- | --- |
| dfi_ret_valid_i | in | Return beat valid |
| dfi_ret_ready_o | out | Issue-FIFO head present AND SRAM slot free |
| dfi_ret_data_i | in | Return beat data |
| dfi_ret_resp_i | in | Return response (captured on last) |
| dfi_ret_last_i | in | Last beat (sets r_ready, pops issue FIFO) |

### 25.5.5	Drain (to pumice_rd_intake R source, AR order)

| Signal | Dir | Description |
| --- | --- | --- |
| drain_valid_o | out | Oldest entry with data ready |
| drain_ready_i | in | Downstream accepts |
| drain_data_o | out | Beat data |
| drain_id_o | out | AXI id (rid echo) |
| drain_resp_o | out | rresp |
| drain_last_o | out | Last beat (evicts entry, frees SRAM slot) |
| busy_o | out | Oldest-valid present OR issue FIFO non-empty |

## 25.6	pumice_dfi_rd_aligner parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| DFI_DATA_WIDTH | 128 | DFI-word width |
| DFI_RATE | 2 | Phases per DFI word (= DFI_EN_WIDTH, DFI_VALID_WIDTH) |
| BL_WORDS | 4 | DFI words per read burst (= BL/DFI_RATE) |
| RDEN_W | 8 | Width of t_rddata_en_i |

## 25.7	Out-of-Order Completion (across AXI IDs)

The CAM preserves AXI ordering the way the standard requires: per-ID reads complete in issue order (a single in-flight read returns all its beats in burst order), while cross-ID reads may complete out of order because the scheduler is free to issue them in row-hit order. The reorder buffer’s AR-order drain (oldest-first, data-ready gated) makes the release order deterministic; the drain_id_o echo tags each beat with its original AXI id.

## 25.8	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Insert AR, issue, DFI return, drain: BL burst round-trips | Full miss-path smoke |
| Returns in issue order buffered into correct SRAM slots | Issue-order return fill |
| Out-of-issue-order scheduling but AR-order drain release | Reorder-buffer ordering guarantee |
| Drain gated on r_ready: oldest entry with data not yet complete stalls | Data-ready gate |
| SRAM slot exhaustion deasserts dfi_ret_ready_o | Return-side pre-allocation backpressure |
| DFI aligner t_rddata_en sweep (0, 1, N); en-window at fire+t_rddata_en | Read-latency window alignment |
| Back-to-back RD fires paced BL_WORDS apart: contiguous en windows | r_epend seamless continuation |
| rd_fire mid-window counted, not dropped | r_epend regression |
| rd_last_o on DFI word BL_WORDS-1 | Word counter / burst boundary |

## 25.9	Open Questions / Future Work

- DFI rddata error propagation. rd_resp_o is fixed OKAY in v1; wire the PHY rddata-error to rd_resp_o (and thence r_resp → AXI rresp) during bring-up.

- Multi-outstanding read windows. The aligner is single-issue in v1; the r_epend counter already tolerates paced back-to-back fires, but overlapping windows would need a deeper capture-side counter.

- Read-path parallelism for reorder / col-major. Deferred perf item — the drain mover releases one oldest burst at a time.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	DFI Command Formatter (dfi_cmd_formatter + dfi_signal_pack)

Module: dfi_cmd_formatter.sv (command decode) / dfi_signal_pack.sv (final DFI pipe stage) Location: rtl/fub/ Category: FUB Parent macro: pumice_dfi_layer Status: Implemented (DDR2 truth table + bit-exact LPDDR2 CA-bus encoding; both memtypes functional)

Replaces the retired command encoder. The original cmd_encoder_fub (with its ddr2_cmd_encoder / lpddr2_cmd_encoder generate-branch sub-modules and a separate odt_ctrl_fub) is gone. The live design is dfi_cmd_formatter.sv — a single module with a runtime memtype_i branch (not an elaboration-time generate) — followed by dfi_signal_pack.sv, the final registered DFI-bus pipeline stage.

LPDDR2 is now fully implemented. The retired chapter listed LPDDR2 as “deferred to v2.” The live formatter builds the bit-exact JESD209-2F Table 60 CA-bus word for every command; the transcription lives in rtl/LPDDR2_CA_ENCODING.md, and the DV BFM (lpddr_ca.py) encodes/decodes against the identical layout.

ODT is absorbed here. There is no standalone ODT block (see the retired ODT chapter). dfi_cmd_formatter drives dfi_odt_o; the DDR2 ODT rule follows from the same truth table. In v1 dfi_odt_o is driven to 0 (the decode leaves w_p0_odt at its NOP default for every op); the JEDEC ODT-timing hooks live in mode_register’s odt_o decode.

## 26.1	Purpose

dfi_cmd_formatter translates the scheduler’s abstract command — (cmd_op, rank, bank, row, col, len) plus an implicit auto-precharge encoded in the op (RDA/WRA vs RD/WR) — into wire-level DFI v2.1 control-bus signals, packed across DFI_RATE phases. For DDR2 it drives the classic {ras_n, cas_n, we_n} strobes plus dfi_address / dfi_bank; for LPDDR2 it packs the multiplexed 10-bit CA bus (two edges = a flat 20-bit word) onto dfi_address.

dfi_signal_pack is the final pipeline register on the DFI v2.1 bus. It latches the formatter’s command bus together with the write/read data-enable signals and drives reset-safe NOP values during reset. It owns dfi_dram_clk_disable (currently held 0).

Both modules run in the DFI clock domain inside pumice_dfi_layer (see the DFI layer / gearing chapter); the formatter is instantiated via pumice_dfi_cmd_path.

## 26.2	Synthesis Parameters (dfi_cmd_formatter)

| Parameter | Default | Effect |
| --- | --- | --- |
| NUM_RANKS | 1 | Width of the per-rank dfi_cs_n / dfi_odt output |
| NUM_BANKS | 8 | Bank-field width (BKW) |
| ROW_WIDTH | 14 | Row operand width (also carries DDR2 MR data and LPDDR2 MRW field) |
| COL_WIDTH | 10 | Column operand width |
| BURST_LEN_WIDTH | 8 | cmd_len_i width (currently unused — tied into unused_v1) |
| DFI_RATE | 2 | Phases per DFI-layer word; sets the multi-phase bus widths |
| DFI_ADDR_WIDTH | 14 | Per-phase address width; the LPDDR2 CA word needs DFI_ADDR_BUS_W >= 20 |
| DFI_BANK_WIDTH | 3 | Per-phase bank width |
| DFI_CTRL_WIDTH | 1 | Per-phase width of each ras/cas/we strobe |
| DFI_CS_WIDTH | NUM_RANKS | Per-phase CS/ODT width |

The multi-phase bus widths (DFI_*_BUS_W = DFI_*_WIDTH * DFI_RATE) are derived. memtype_i is a runtime input (MEMTYPE_DDR2 / MEMTYPE_LPDDR2), not a parameter — both encoding paths are synthesized and the branch is selected live, so one bitstream can serve either family.

## 26.3	Command Interface and Runtime Phase Placement

The formatter takes a valid/ready command handshake from the DFI-layer command FIFO:

| Signal | Width | Description |
| --- | --- | --- |
| cmd_valid_i | 1 | Command present |
| cmd_ready_o | 1 | Always 1 (registered) — formatter never stalls |
| cmd_op_i | dram_op_e | The chosen operation |
| cmd_rank_i | RKW | Target rank (selects the active CS_n) |
| cmd_bank_i | BKW | Target bank (MR index for MRS) |
| cmd_row_i | ROW_WIDTH | Row (ACT); DDR2 MR data; LPDDR2 packed MRW field |
| cmd_col_i | COL_WIDTH | Column (RD/WR) |
| cmd_len_i | BURST_LEN_WIDTH | Burst length (reserved; unused in v1) |
| rd_phase_i | PHW | DFI sub-phase carrying the READ command |
| wr_phase_i | PHW | DFI sub-phase carrying the WRITE command |

rd_phase_i / wr_phase_i are CSR-driven (DFI_PHASE CSR). The decoded command is placed on wr_phase for WR/WRA, rd_phase for RD/RDA, and phase 0 for everything else. This matches a PHY that consumes a per-command rdphase/wrphase off the DFI bus. Defaults are 0, which reproduces the legacy “everything on phase 0” behavior — notably the LiteDRAM a7ddrphy takes the command on phase 0 and applies its rdphase internally, so on the board target these stay 0. (See the gearing chapter §“DFI_PHASE CSR”.)

## 26.4	DDR2 Encoding Branch

For memtype_i == MEMTYPE_DDR2, a combinational block builds the phase-0 command fields (w_p0_*). The default is an all-deselected NOP; when a command is valid, w_p0_cs_n is set to the active-rank mask (w_active_rank_mask, bit r low for the target rank) and the strobes/address are driven per the JEDEC JESD79-2 truth table (transcribed verbatim from the RTL):

| Op | cs_n | ras_n | cas_n | we_n | bank | address |
| --- | --- | --- | --- | --- | --- | --- |
| OP_NOP | active mask | 1 | 1 | 1 | 0 | 0 |
| OP_ACT | active mask | 0 | 1 | 1 | bank | row |
| OP_RD | active mask | 1 | 0 | 1 | bank | col (A10 = 0) |
| OP_RDA | active mask | 1 | 0 | 1 | bank | col with bit 10 set (auto-PRE) |
| OP_WR | active mask | 1 | 0 | 0 | bank | col (A10 = 0) |
| OP_WRA | active mask | 1 | 0 | 0 | bank | col with bit 10 set (auto-PRE) |
| OP_PRE | active mask | 0 | 1 | 0 | bank | 0 (A10 = 0, single-bank) |
| OP_PREA | active mask | 0 | 1 | 0 | 0 | bit 10 set (all-bank) |
| OP_REF | active mask | 0 | 0 | 1 | 0 | 0 |
| OP_MRS | active mask | 0 | 0 | 0 | MR idx | MR data from cmd_row_i |

cs_n is per-rank: bit r is 0 for the selected rank, 1 elsewhere; all-1 is an all-deselected NOP. The A10 auto-precharge bit is set by OR-ing 1 << 10 into the address for RDA/WRA, and doubles as the all-bank flag for PREA.

MRS carries data on the ROW field, not the column. The RTL comment is explicit: MR0 = 0x532 needs bit 10 (tWR[1]), which a COL_WIDTH (10-bit) field would truncate — so the formatter reads MR data from cmd_row_i (ROW_WIDTH) and the MR index from cmd_bank_i. This matches init_sequencer, which drives MR data on init_cmd_row_o.

Unhandled DDR2-irrelevant ops (OP_REFPB, OP_ZQCS/ZQCL, self-refresh entry/ exit) fall through the default arm and emit NOP — they are driven via CKE / a separate sequencer, not the strobe truth table.

## 26.5	LPDDR2 Encoding Branch (bit-exact JESD209-2F Table 60)

LPDDR2 has no ras_n/cas_n/we_n. The command AND a scrambled address are multiplexed onto a 10-bit CA bus over two clock edges (rising r, falling f), carried on the DFI bus as a flat 20-bit word. The formatter builds two 10-bit half-words w_ca_r / w_ca_f and concatenates them:

assign w_lpddr2_ca = {w_ca_f, w_ca_r};   // [19:10] = falling, [9:0] = rising

This is the exact layout locked in rtl/LPDDR2_CA_ENCODING.md and enforced by a round-trip conformance test against the DV decoder (lpddr_ca.py). Per JEDEC, any other bit ordering is prohibited, so bit-exactness is a spec requirement.

Command decode is by the rising-edge opcode bits {CA0r, CA1r, CA2r, CA3r}, transcribed from the RTL unique case:

| Op | Rising opcode bits set | Payload placement |
| --- | --- | --- |
| OP_ACT | CA1r = 1 | bank → CA7r..CA9r; row hi R8..R12 → CA2r..CA6r; row lo R0..R7 → CA0f..CA7f; R13 → CA8f; R14 → CA9f |
| OP_RD / OP_RDA | CA0r = 1, CA2r = 1 (read) | bank → CA7r..CA9r; C1,C2 → CA5r,CA6r; AP → CA0f; C3..C11 → CA1f..CA9f |
| OP_WR / OP_WRA | CA0r = 1, CA2r = 0 (write) | same column/bank/AP placement as read |
| OP_PRE | CA0r=1, CA1r=1, CA3r=1 (AB=0) | bank → CA7r..CA9r |
| OP_PREA | CA0r=1, CA1r=1, CA3r=1, CA4r=1 (AB=1) | bank don’t-care |
| OP_REF | CA2r=1, CA3r=1 (all-bank) | — |
| OP_REFPB | CA2r=1 (per-bank; CA3r=0) | bank implied by device counter |
| OP_MRS (MRW) | all opcode bits 0 | MA0..MA5 → CA4r..CA9r; MA6,MA7 → CA0f,CA1f; OP0..OP7 → CA2f..CA9f |
| default (NOP) | CA0r=CA1r=CA2r=CA3r=1 | — |

Auto-precharge is w_ca_ap = (op == OP_RDA) || (op == OP_WRA), placed on CA0f. C0 is implied 0 and never transmitted (only C1..C11 ride the bus). Row/column/ bank are zero-extended (w_row15, w_col12) so wider geometries (R14, C10, C11) light up their reserved pins cleanly.

MRW field packing. LPDDR2 mode-register addresses reach MR63 (MA[5:0]), which a 3-bit bank port cannot express. The scheduler therefore carries the MRW fields in the ROW request as {MA[5:0], OP[7:0]} — w_mr_ma = {2'b0, cmd_row_i[13:8]}, w_mr_op = cmd_row_i[7:0] — matching init_sequencer’s mrw_row() packing. MA[7:6] are 0 (MR <= 63).

## 26.6	Multi-Phase Packing and CS_n

After the decode, a combinational stage packs the command into the multi-phase buses (w_dfi_*). Every phase starts at NOP (cs_n = 1, ras/cas/we = 1); then:

- DDR2: the decoded w_p0_* fields are placed on the target phase w_cmd_phase (rd_phase for reads, wr_phase for writes, phase 0 otherwise); the other phases stay NOP.

- LPDDR2: the whole 20-bit CA word is written to w_dfi_address[19:0] (low bits), ras/cas/we stay idle, and dfi_cs_n is asserted for the target rank on phase 0 when the op is not NOP. The two CA edges are already inside the word, so there is no per-DFI-phase command placement for LPDDR2.

The active-rank CS_n mask is built once (w_active_rank_mask): bit r = 0 when r == cmd_rank_i, else 1. For NUM_RANKS == 1 only bit 0 exists.

## 26.7	dfi_signal_pack — Final Registered DFI Stage

dfi_signal_pack is a pure one-cycle registered pipeline. It latches the formatter’s command bus plus the data-enable/mask signals from the write/read serializers and drives them to the PHY the next DFI cycle. Its value is the reset-safe defaults it guarantees during reset / before first issue:

| Output | Reset value | Meaning |
| --- | --- | --- |
| dfi_cs_n_o | all-1 | all-deselected |
| dfi_ras_n_o/cas_n_o/we_n_o | all-1 | NOP |
| dfi_cke_o | 0 | DRAM held CKE-low until init |
| dfi_odt_o | 0 | ODT off |
| dfi_wrdata_en_o / dfi_rddata_en_o | 0 | no data movement |
| dfi_wrdata_mask_o | all-1 | mask all bytes (write nothing) |
| dfi_dram_clk_disable_o | 0 | clock enabled (power-state TODO) |

It owns dfi_dram_clk_disable for a future power-state machine (held 0 today). Phase staggering and CKE power-down driving are v2 TODOs noted in the RTL header; today it is a transparent width-preserving flop.

## 26.8	Interface (dfi_cmd_formatter outputs)

| Signal | Width | Description |
| --- | --- | --- |
| dfi_address_o | DFI_ADDR_BUS_W | DDR2 row/col operand per phase; LPDDR2 flat CA word |
| dfi_bank_o | DFI_BANK_BUS_W | Per-phase bank (DDR2) |
| dfi_cas_n_o | DFI_CTRL_BUS_W | Per-phase CAS strobe |
| dfi_ras_n_o | DFI_CTRL_BUS_W | Per-phase RAS strobe |
| dfi_we_n_o | DFI_CTRL_BUS_W | Per-phase WE strobe |
| dfi_cs_n_o | DFI_CS_BUS_W | Per-phase per-rank chip-select |
| dfi_odt_o | DFI_CS_BUS_W | Per-phase per-rank ODT (0 in v1) |

All outputs are strict-flopped; dfi_signal_pack widens/relatches them onto the PHY pins unchanged.

## 26.9	Timing Budget

The formatter is combinational decode into a single output flop; dfi_signal_pack adds one more flop. The worst path is the LPDDR2 ACT with full row-bit spreading across CA0r/CA0f (bank + 5 rising row bits + 8 falling row bits), which is a few LUT levels feeding the output register — well within the DFI clock budget. The formatter is not the controller’s critical path; that is the arbiter upstream.

## 26.10	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| DDR2 ACT bank 3 row 0x1234 → {ras,cas,we}={0,1,1}, address=0x1234, bank=3 | DDR2 ACT truth-table row |
| DDR2 RDA bank 3 col 0x40 → {ras,cas,we}={1,0,1}, address bit 10 set | DDR2 auto-precharge via A10 |
| DDR2 PREA → {ras,cas,we}={0,1,0}, address bit 10 set | DDR2 all-bank precharge via A10 |
| DDR2 MRS MR0=0x532 → MR data on address (bit 10 present, not truncated) | MRS data on ROW field |
| LPDDR2 ACT bank 3 row 0x1234 → w_lpddr2_ca bit-exact vs lpddr_ca.py decode | LPDDR2 ACT CA packing |
| LPDDR2 WRA bank 3 col 0x40 → AP bit (CA0f) set | LPDDR2 auto-precharge via CA |
| LPDDR2 REFpb (CA2r=1, CA3r=0) vs REFab (CA2r=1, CA3r=1) | LPDDR2 refresh CA decode |
| LPDDR2 MRW MR63 → MA/OP packed on CA4r..CA9r / CA0f.. per Table 60 | LPDDR2 MRW field packing |
| Multi-rank ACT rank 1 → dfi_cs_n bit 1 low, bit 0 high | Per-rank CS_n mask |
| rd_phase=1 → RD command lands on DFI phase 1, other phases NOP | Runtime phase placement |
| NOP / !cmd_valid → all-deselected (cs_n all-1), strobes all-1 | Idle pattern |
| Reset → dfi_signal_pack drives cs_n=1, cke=0, wrdata_mask=all-1 | Reset-safe NOP defaults |

## 26.11	Open Questions / Future Work

- ODT driving. dfi_odt_o is held 0 in v1 (the decode leaves the ODT default at NOP). Multi-rank DDR2 needs the JEDEC cross-termination window (ODT-high on the non-accessed rank during a read, on the accessed rank during a write). The hooks exist (mode_register.odt_o decodes the DDR2 MR1 ODT bits); wiring the timed ODT window through the formatter is future work — see the absorbed ODT chapter.

- cmd_len_i unused. The burst-length input is reserved (tied into unused_v1); burst geometry is handled by the data-path serializers, so the formatter does not need it today.

- Phase staggering / clk-disable. dfi_signal_pack is a plain relatch; per-phase output staggering and dfi_dram_clk_disable driving (for power-down) are v2 items noted in the RTL header.

- LPDDR2 BL16. mode_register.bl_o is 4-bit and clips BL16 to BL8; only BL4/BL8 are wired end-to-end. Widening the burst path is a separate item.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	DFI Layer and Host-Width Gearing (pumice_dfi_layer + pumice_top_geared)

Module: pumice_dfi_layer.sv (macro) / pumice_top_geared.sv (top wrapper) Location: rtl/macro/ and rtl/top/ Category: Macro / Top wrapper Status: Implemented (single-CDC DFI datapath; formal-IP host-width gearing wrapper)

Replaces the retired gear_dfi_fub. The original chapter described a phase-packing FUB (gear_dfi_fub / dfi_signal_pack) that spread a single MC command and burst-of-N data beats into per-phase DFI slots. That phase-packing role is real but small — it lives inside dfi_signal_pack and the DFI-layer sub-FUBs. This chapter now covers the two live, distinct concepts that carry the word “gearing” in pumice, and it keeps them separate:

- The DFI layer (pumice_dfi_layer) — the single controller/PHY clock-domain crossing plus the DFI-clock command/write/read datapath. This is where the internal DW-wide word is split into DFI_RATE DRAM beats.

- Host-width gearing (pumice_top_geared) — an OPTIONAL top wrapper that lets a host SoC use any AXI data width, inserting the repo’s formally verified AXI data-width converters between the host and the fixed-width core.

These are orthogonal: (1) is about DRAM-beat phasing on the PHY side; (2) is about AXI beat width on the host side.

## 27.1	Concept 1 — The DFI Layer (pumice_dfi_layer)

### 27.1.1	Purpose

pumice_dfi_layer presents the controller-clock command/wrdata/rddata streams on one side and the DFI 2.1 pin bus on the other. It holds the single controller/PHY clock-domain crossing and the DFI-clock-side datapath. Its internal datapath unit is the DFI word (dfi_wrdata width = all DFI_RATE phases): one FIFO word equals one DFI cycle, which keeps the datapath bubble-free.

Two clock domains:

- ctl_clk — command from the scheduler, write data from the WR CAM, read data to the RD CAM, and the init_start / init_complete handshake.

- dfi_clk — the DFI command bus, dfi_wrdata/en/mask, dfi_rddata/en/ valid, and dfi_init_start/complete.

### 27.1.2	Internal structure (verified against the RTL)

pumice_dfi_layer instantiates exactly four sub-FUBs:

| Sub-FUB | Domain | Role |
| --- | --- | --- |
| pumice_dfi_cdc | ctl↔dfi | The single clock crossing — async gaxi FIFOs only (cmd, wrdata, rddata, plus init-start/complete bit crossings) |
| pumice_dfi_cmd_path | dfi_clk | cmd FIFO → DFI command bus + wr_fire/rd_fire strobes; instantiates dfi_cmd_formatter (§2.14) and dfi_signal_pack |
| pumice_dfi_wr_serializer | dfi_clk | wrdata FIFO → dfi_wrdata at t_phy_wrlat |
| pumice_dfi_rd_aligner | dfi_clk | dfi_rddata → rddata FIFO at t_rddata_en |

The CDC (pumice_dfi_cdc) is the only place the domains meet; everything downstream of it runs entirely in dfi_clk. The command path emits wr_fire_o / rd_fire_o strobes that time the serializer and aligner relative to the issued command.

### 27.1.3	Key parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| NUM_RANKS | 1 | CS/rank fan-out |
| NUM_BANKS | 8 | Bank-field width |
| ROW_WIDTH | 14 | Row / DFI address width |
| COL_WIDTH | 10 | Column width |
| DFI_RATE | 2 | DRAM beats per DFI word (phase count) |
| DRAM_BEAT_WIDTH | 64 | One DRAM beat’s data width |
| BL | 8 | DRAM beats per burst; BL_WORDS = BL / DFI_RATE DFI words/burst |
| CMD/WD/RD_FIFO_DEPTH | 8/16/16 | CDC FIFO depths |
| N_FLOP_CROSS | 2 | Synchronizer flop stages in the async FIFOs |

Derived DFI geometry: DFI_DATA_WIDTH = DRAM_BEAT_WIDTH * DFI_RATE, DFI_STRB_WIDTH = DFI_DATA_WIDTH/8, DFI_EN_WIDTH = DFI_VALID_WIDTH = DFI_RATE. The FIFO payloads pack the command (CMD_DW), {last,strb,data} for writes (WD_DW), and {last,resp,data} for reads (RD_DW).

### 27.1.4	DFI-clock runtime knobs

| Signal | Width | Description |
| --- | --- | --- |
| memtype_i | enum | DDR2 / LPDDR2 selector for the command path |
| rd_phase_i | PHW | DFI sub-phase carrying the READ command (see §2.14) |
| wr_phase_i | PHW | DFI sub-phase carrying the WRITE command |
| t_phy_wrlat_i | 8 | PHY write latency — when the serializer launches wrdata |
| t_rddata_en_i | 8 | When the aligner strobes dfi_rddata_en / captures data |

These are the PHY-integration knobs surfaced through the CSR (DFI_PHASE, DFI_TIMING); they carry the PHY-specific latencies rather than baking them into the datapath, which is what let on-board bring-up dial in t_rddata_en / phase placement without an RTL change.

## 27.2	Concept 2 — Host-Width Gearing (pumice_top_geared)

### 27.2.1	Purpose

The controller core is fixed at its natural width DW = DRAM_BEAT_WIDTH * DFI_RATE (default 128): one AXI beat == one DFI word == DFI_RATE DRAM beats, and one AXI burst (BL/DFI_RATE beats) == one DRAM burst (BL beats). pumice_top_geared wraps pumice_top with a free HOST_AXI_DATA_WIDTH, inserting the repo’s formally verified data-width converters (axi4_dwidth_converter_wr / _rd) between a host-width AXI slave and the fixed-DW core.

This is the family-wide gearing path (DDR2/DDR3/DDR4/LPDDR2), reusing proven + formal IP rather than re-verifying a bespoke gearbox inside the freshly stabilized controller datapath.

The old coupling is gone. Do not describe the retired “AXI_DATA_WIDTH == DRAM_BEAT_WIDTH” constraint. Internal geometry is DW = DRAM_BEAT_WIDTH * DFI_RATE; the DW→phase split happens inside the DFI layer / dfi_signal_pack, and the host AXI width is decoupled from it by this wrapper.

### 27.2.2	GEAR-1 bypass (bit-identical)

When HOST_AXI_DATA_WIDTH == DW, a generate selects the g_direct branch: the host AXI signals are wired straight through to the core with no converter and no added latency. Existing DW-width builds are therefore bit-identical whether they instantiate pumice_top directly or through pumice_top_geared.

### 27.2.3	Geared branch

When HOST_AXI_DATA_WIDTH != DW, the g_conv branch instantiates the two formal converters:

axi4_dwidth_converter_wr #(.S_AXI_DATA_WIDTH(HOST_AXI_DATA_WIDTH), .M_AXI_DATA_WIDTH(DW), ...)
axi4_dwidth_converter_rd #(.S_AXI_DATA_WIDTH(HOST_AXI_DATA_WIDTH), .M_AXI_DATA_WIDTH(DW), ...)

The host slave ports (s_axi_* at HOST_AXI_DATA_WIDTH) feed the converters; the converters’ master ports (the internal c_* nets at DW) feed pumice_top. The CSR cpuif and the DFI pin bus pass straight through the wrapper unmodified.

Burst geometry. The core still requires one AXI burst == one DRAM burst at its DW side ((awlen+1) * DFI_RATE == BL). The host issues bursts at host width; the converter translates them to DW-width bursts of that geometry. Host burst sizing is the host’s contract (the same spirit as the core’s ragged-burst check). Verified for host widths in {64, 128, 256}. Full scope in docs/AXI_DRAM_GEARING_SCOPE.md.

### 27.2.4	Key parameters

| Parameter | Default | Effect |
| --- | --- | --- |
| HOST_AXI_DATA_WIDTH | 128 | Host AXI data width (free); == DW triggers the bypass |
| DW (derived) | DRAM_BEAT_WIDTH * DFI_RATE | Fixed core width |
| core geometry params | — | AXI_ID_WIDTH, AXI_ADDR_WIDTH, NUM_RANKS, NUM_BANKS, ROW_WIDTH, COL_WIDTH, DFI_RATE, DRAM_BEAT_WIDTH, BL, … mirror pumice_top |

## 27.3	Narrow-Device (x16) Support — the Two Width Granularities

A distinction that is separate from both concepts above but which the DFI layer must respect: the pumice DRAM beat (DRAM_BEAT_WIDTH, one DFI phase’s data) is not necessarily the physical DRAM device word (DRAM_DEVICE_WIDTH, the DQ width — e.g. 16 for an x16 device). When a beat is wider than the device (a 32-bit beat over an x16 device packs K = DRAM_BEAT_WIDTH / DRAM_DEVICE_WIDTH = 2 physical DDR words), three parts of the pipeline must reason in device-word units. Getting this wrong is invisible in a DFI-level behavioral sim but corrupts real silicon — the Nexys A7 x16 bring-up hit all three:

- Burst length (beats per command). A JEDEC BL from MR0 (bl_o, BL4/BL8) is in physical device beats; the core scales it to pumice-beat units before feeding the burst-split and serializer, else an x16 BL4 is over-counted and the controller drives/captures an extra DFI cycle.

- Column address stride. DRAM columns are device-word granular, so addr_mapper’s BYTE_OFFSET_WIDTH must be log2(DRAM_DEVICE_WIDTH/8), not log2(DRAM_BEAT_WIDTH/8); using the beat width makes a split burst’s chunk overwrite the previous chunk’s tail (a ~50% read scramble on silicon).

- Read-data / valid alignment. A PHY may present read data ahead of, or on a different phase than, rddata_valid. This is handled at runtime by the t_rddata_en knob and the DFI_PHASE CSR (below) rather than baked-in latencies.

Default DRAM_DEVICE_WIDTH = DRAM_BEAT_WIDTH (K=1) makes all of the above bit-identical to the wide-device behavior.

### 27.3.1	DFI_PHASE CSR (rd_phase / wr_phase)

dfi_cmd_formatter places the READ command on rd_phase and the WRITE command on wr_phase (defaults 0; all other commands on phase 0), runtime-settable via the DFI_PHASE CSR through pumice_dfi_layer’s rd_phase_i / wr_phase_i. This matches a PHY that consumes a per-command rdphase/wrphase off the DFI bus. The LiteDRAM a7ddrphy instead takes the command on phase 0 and applies its rdphase internally, so on that PHY rd_phase stays 0 — the CSR exists for PHYs that do not.

## 27.4	Interface Summary

### 27.4.1	pumice_dfi_layer — controller side (ctl_clk)

| Signal | Direction | Description |
| --- | --- | --- |
| cmd_valid_i / cmd_ready_o / cmd_data_i[CMD_DW] | in/out/in | Command stream from the scheduler |
| wd_valid_i / wd_ready_o / wd_data_i[WD_DW] | in/out/in | Write data from the WR CAM drain ({last,strb,data}) |
| rd_valid_o / rd_ready_i / rd_data_o[RD_DW] | out/in/out | Read data to the RD CAM ({last,resp,data}) |
| init_start_i / init_complete_o | in/out | Init handshake, controller side |

### 27.4.2	pumice_dfi_layer — PHY side (dfi_clk)

DFI command bus (dfi_address_o, dfi_bank_o, dfi_cas_n_o, dfi_ras_n_o, dfi_we_n_o, dfi_cs_n_o, dfi_odt_o), write data (dfi_wrdata_o, dfi_wrdata_en_o, dfi_wrdata_mask_o), read data (dfi_rddata_en_o, dfi_rddata_i, dfi_rddata_valid_i), and the DFI init handshake (dfi_init_start_o, dfi_init_complete_i), plus the runtime knobs above.

### 27.4.3	pumice_top_geared

Host AXI4 slave at HOST_AXI_DATA_WIDTH, PeakRDL cpuif passthrough (s_cpuif_*), init_done_o, and the DFI 2.1 pin bus straight through. Internally wires either directly (g_direct) or via the two converters (g_conv) to pumice_top at DW.

## 27.5	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Command/wrdata/rddata cross the CDC bubble-free at DFI_RATE = 2 | Single-CDC datapath, one FIFO word = one DFI cycle |
| Write burst: wr_fire → serializer launches dfi_wrdata at t_phy_wrlat | Write-latency alignment |
| Read burst: rd_fire → aligner strobes dfi_rddata_en at t_rddata_en | Read-capture alignment |
| rd_phase = 1: RD command lands on DFI phase 1 | DFI_PHASE CSR routing |
| x16 device (K=2): BL4 uses one DFI cycle, columns stride by device words | Two-granularity handling |
| pumice_top_geared with HOST_AXI_DATA_WIDTH == DW: g_direct bypass | GEAR-1 bit-identical to bare pumice_top |
| Host 64 → DW 128 via axi4_dwidth_converter_wr/_rd: burst geometry preserved | Geared write/read path |
| Host 256 → DW 128: read data reassembled correctly at host width | Wide-host gearing |
| CSR cpuif and DFI pins identical whether geared or direct | Passthrough correctness |

## 27.6	Open Questions / Future Work

- Multi-command-per-cycle. The command path places one issued command per DFI word today (multi-phase content passes through, but a single command occupies a word). Emitting multiple commands per DFI cycle is a scheduler-side feature; the bus widths are already in place.

- dfi_dram_clk_disable / power-down. dfi_signal_pack holds clk-disable at 0; wiring the power-state machine through the DFI layer is future work.

- Higher gear ratios. DFI_RATE = 4 (and DDR3+ higher ratios) scale the DFI geometry linearly; validate the CDC FIFO depths and serializer/aligner timing when the family controller targets them.

- Host width coverage. Gearing is verified for host ∈ {64, 128, 256}; add 512 when a host SoC requires it. See docs/AXI_DRAM_GEARING_SCOPE.md.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	Transaction Queue (retired — replaced by the two command CAMs)

## RETIRED — no standalone FUB

The unified transaction queue (txn_queue_fub) described by the SWAG was never built as a separate block. The scheduling-view function it was meant to provide is delivered directly by the two command CAMs:

- Pending writes live in pumice_wr_data_cam — {bank, row, col} key, free-running age, and the burst data in SRAM.

- Pending reads live in pumice_rd_cmd_cam — {bank, row} key, free-running age, per-entry return buffering.

- Per-direction completion is FIFO-based in the intakes (pumice_wr_intake / pumice_rd_intake): the write B FIFO and the read order + rd-data FIFOs.

This chapter is retained to document where the queue’s responsibilities went and why. The SWAG’s entry-format, four-state lifecycle, cam_slot_idx reverse pointer, and row_hit_cached broadcast are all superseded and should not be treated as the current architecture.

Status: Retired

## 28.1	What the CAMs do instead

The SWAG separated a narrow “scheduling view” (the queue) from a wide “metadata view” (the CAMs) to keep the scheduler’s per-cycle comparator fan-in small. The live design collapses the two: each CAM entry carries both the scheduling key and the metadata, and the scheduler reads the CAMs directly through purpose-built lookup ports rather than a wide parallel-entry snapshot.

### 28.1.1	Scheduler visibility (replaces the parallel q_entries bus)

Instead of a wide per-entry snapshot bus, each CAM exposes:

- N_SCHED_LU lookup ports (N_LU = NUM_BANKS, one per bank). The arbiter drives each port with {bank j, that bank's open row} and the CAM returns the oldest matching not-yet-committed entry — slot, column, id, and age. This is the row-hit query that used to be the row_hit_cached field.

- An oldest port — the oldest schedulable entry, used as the arbiter’s ACT fallback target.

The scheduler thus gets exactly the row-hit and fallback candidates it needs, computed inside the CAM combinationally, with no separate queue structure and no row_hit_cached coherency broadcast.

### 28.1.2	Entry lifecycle (replaces FREE/PENDING/ISSUED/COMPLETING)

The four-state queue lifecycle is replaced by per-entry flags in each CAM:

| SWAG queue state | Live equivalent (write CAM) | Live equivalent (read CAM) |
| --- | --- | --- |
| FREE | !r_valid | !r_valid |
| PENDING | r_valid && r_fdone && !r_sched | r_valid && !r_issued |
| ISSUED | r_sched (enqueued to drain FIFO) | r_issued (enqueued to issue FIFO) |
| COMPLETING | commit-drain streaming, then evict | data returning, then oldest-drain |

The exclusion of already-committed/issued entries from the lookup/oldest ports (r_sched on the write side, r_issued on the read side) is what guarantees the arbiter never picks the same slot twice — the role the pick_valid_mask played in the SWAG.

### 28.1.3	Age (replaces the saturating age counter)

Both CAMs use a free-running AGE_WIDTH counter and wrap-safe relative age (rel = age_ctr - entry_age). Oldest = max rel. There is no saturation, no AGE_MAX parameter, and no age_max_runtime CSR clip; wrap-safety makes saturation unnecessary.

### 28.1.4	Backpressure (replaces q_high_water / q_full)

There is no queue occupancy watermark. Backpressure is structural: the write path stalls when the wr-data CAM has no free entry or its fill FIFO is full; the read path stalls when the rd-cmd CAM is full or the intake’s order FIFO is full. The AXI channels back-pressure through the intake FIFOs.

## 28.2	Why the split disappeared

The queue existed to make a single wide scheduling snapshot cheap. Once the scheduler was rearchitected to query per-bank (one lookup per open row) rather than scan every entry, the wide snapshot was no longer needed — the CAM’s own associative match is the scheduling view. Keeping a second copy of {rank, bank, row} in a queue, plus the cam_slot_idx reverse pointer and the row_hit_cached broadcast coherency machinery, would have been pure overhead.

See ch02/07 for the arbiter that consumes the CAM lookup ports and ch02/04 / ch02/05 for the CAM internals.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Bank Timer (bank_timer + pumice_bank_timers)

Module: bank_timer.sv (per-bank) / pumice_bank_timers.sv (aggregator) Location: rtl/fub/ Category: FUB Parent macro: pumice_mem_cmd_scheduler Status: Implemented (FSM-free countdown-timer model)

Replaces the retired bank machine. The original architecture placed a per-(rank, bank) seven-state FSM (bank_machine_fub) between the scheduler and the DFI encoder. That block no longer exists. Per-bank JEDEC timing is now enforced by bank_timer.sv — a set of preset/decrement countdown timers with a trivial row-open register and no state machine at all. pumice_bank_timers.sv stamps one bank_timer per (rank, bank) and fans the scheduler’s command-event strobes to the addressed instance.

The retirement was deliberate: the old FSM double-registered readiness (state register + next-state combinational + accepts flop), so the scheduler saw readiness two cycles stale. That staleness was the root cause of the refresh-vs-ACT and column-vs-PRE hazards observed during bring-up. The FSM-free model collapses the readiness path to a single register stage (the timers themselves), so the arbiter sees the just-issued command’s effect exactly one cycle later.

## 29.1	Purpose

bank_timer tracks one DRAM bank’s JEDEC “safe” timing. For each JEDEC per-bank constraint it holds a down-counter that is preset on the triggering command and free-runs toward zero (saturating at 0). A constraint is “safe” when its counter reads 0. The per-command readiness outputs (safe_act, safe_rd, safe_wr, safe_pre) are a combinational AND of the relevant timers plus a one-bit row-open flag — no FSM, no multi-stage lag.

The scheduler (pumice_cmd_arbiter, the single command issuer in the design) drives one set_* strobe per issued command. Each timer reloads its config value on its trigger; the safe_* outputs therefore reflect the just-issued command one cycle later. The arbiter ANDs these per-bank safe_* signals with the channel-wide turnaround windows from global_timers (§2.10) to decide which command may fire.

pumice_bank_timers is a thin aggregator: it instantiates NUM_RANKS × NUM_BANKS copies of bank_timer and routes the arbiter’s single command-event bus (evt_* + evt_rank_i / evt_bank_i) to the one addressed instance. All other instances see their set_* strobes deasserted that cycle and simply continue counting down.

## 29.2	Synthesis Parameters

### 29.2.1	bank_timer

| Parameter | Default | Effect on this FUB |
| --- | --- | --- |
| ROW_WIDTH | 14 | Width of open_row_o register and the row_i input |
| TW | 8 | Timer width (bits) for all five countdown timers |

### 29.2.2	pumice_bank_timers

| Parameter | Default | Effect |
| --- | --- | --- |
| NUM_RANKS | 1 | Rank dimension of the instance array; width of evt_rank_i sel |
| NUM_BANKS | 8 | Bank dimension of the instance array; width of evt_bank_i sel |
| ROW_WIDTH | 14 | Passed to each bank_timer |
| RKW | clog2(NUM_RANKS) | Rank-select width (min 1) |
| BKW | clog2(NUM_BANKS) | Bank-select width |

There is no per-instance rank/bank identifier parameter. Identity comes from the genvar position in pumice_bank_timers’ nested generate loop; the arbiter selects an instance by comparing evt_rank_i / evt_bank_i against that position (w_sel). All five timers share the same TW-bit width; the JEDEC cycle counts are supplied at runtime on the t_*_i ports (CSR-backed), not fixed as parameters.

## 29.3	Instantiation Pattern

From pumice_bank_timers.sv — the nested generate that stamps the array:

for (genvar k = 0; k < NUM_RANKS; k++) begin : g_rank
    for (genvar b = 0; b < NUM_BANKS; b++) begin : g_bank
        // route the scheduler's command event to THIS bank only
        logic w_sel;
        assign w_sel = (evt_rank_i == RKW'(k)) && (evt_bank_i == BKW'(b));

        bank_timer #(.ROW_WIDTH(ROW_WIDTH)) u_bt (
            .clk(aclk), .rst_n(aresetn),
            .t_rcd_i(t_rcd_i), .t_rp_i(t_rp_i), .t_ras_i(t_ras_i),
            .t_rc_i(t_rc_i), .t_wr_i(t_wr_i), .t_rtp_i(t_rtp_i),
            .set_act_i(evt_act_i && w_sel),
            .set_rd_i (evt_rd_i  && w_sel),
            .set_wr_i (evt_wr_i  && w_sel),
            .set_pre_i(evt_pre_i && w_sel),
            .set_ap_i (evt_ap_i),
            .row_i(evt_row_i),
            .safe_act_o (bank_act_ready_o [k][b]),
            .safe_rd_o  (bank_rdwr_ready_o[k][b]),
            .safe_wr_o  (/* == safe_rd */),
            .safe_pre_o (bank_pre_ready_o [k][b]),
            .row_valid_o(bank_row_active_o[k][b]),
            .open_row_o (bank_open_row_o  [k][b]),
            .state_o    (bank_state_o     [k][b]),
            /* obs_* ... */
        );
    end
end

The per-(rank, bank) readiness outputs are 2-D-packed arrays (logic [NUM_RANKS-1:0][NUM_BANKS-1:0]) consumed directly by pumice_cmd_arbiter. There is no aggregation logic beyond the packing — pure structural fan-out. Note that safe_wr_o is left unconnected in the aggregator because inside bank_timer it is identical to safe_rd_o (see the RTL: assign safe_wr_o = safe_rd_o); the arbiter uses the single bank_rdwr_ready_o bit for both RD and WR column commands.

## 29.4	The Timer Pool (per bank)

bank_timer holds five countdown registers (r_rcd, r_ras, r_rc, r_rp, r_preblk), each TW bits, plus a one-bit r_row_valid, the r_open_row register, and a single r_ap_pending auto-precharge bit. Every timer is a down-counter: reload on its trigger, else decrement while non-zero (saturate at 0).

| Timer | JEDEC constraint | Reload trigger | Reload value | Gates |
| --- | --- | --- | --- | --- |
| r_rcd | tRCD (ACT → RD/WR) | set_act_i | t_rcd_i | safe_rd/wr |
| r_ras | tRAS (ACT → PRE min) | set_act_i | t_ras_i | safe_pre |
| r_rc | tRC (ACT → ACT same bank) | set_act_i | t_rc_i | safe_act |
| r_rp | tRP (PRE → ACT) | set_pre_i OR auto-PRE fire | t_rp_i | safe_act |
| r_preblk | tRTP (RD) / tWR (WR) | set_rd_i / set_wr_i | t_rtp_i / t_wr_i | safe_pre |

The t_*_i reload values are supplied by the parent (pumice_bank_timers forwards its own 8-bit t_*_i inputs, which are CSR-backed JEDEC timing fields). They are sampled live on each reload — the timer does not stage them.

Note: the retired FSM’s separate t_cl_cnt / t_cwl_cnt (column-busy windows) are gone. Column-to-column pacing (tCCD) and the read-drives-DQ / write-recovery windows are enforced globally in global_timers (tCCD/tWTR/tRTW) and, for the write-recovery-before-precharge case, folded into the r_preblk timer here. There is no per-bank “RD_BUSY / WR_BUSY” occupancy state anymore.

## 29.5	Row-Open Register and Auto-Precharge (no FSM)

Row state is a two-element register — r_row_valid + r_open_row — plus one r_ap_pending bit. There is no enumerated state; the priority in the RTL is ACT > explicit PRE > auto-PRE fire > column:

if (set_act_i) begin
    r_row_valid  <= 1'b1;
    r_open_row   <= row_i;
    r_ap_pending <= 1'b0;
end else if (set_pre_i) begin
    r_row_valid  <= 1'b0;
    r_ap_pending <= 1'b0;
end else if (w_ap_fire) begin
    r_row_valid  <= 1'b0;
    r_ap_pending <= 1'b0;
end else if (set_rd_i || set_wr_i) begin
    r_ap_pending <= set_ap_i;   // latch the RDA/WRA qualifier
end

Auto-precharge (RDA / WRA) is a single bit, not a state. set_ap_i qualifies a RD/WR issue; if set, r_ap_pending latches. The auto-precharge “fires” combinationally once the read/write recovery window and tRAS have both elapsed:

assign w_ap_fire = r_ap_pending && (r_preblk == '0) && (r_ras == '0);

When w_ap_fire asserts, the row closes internally (r_row_valid <= 0) and r_rp reloads with t_rp_i — exactly as an explicit PRE would. No PRE command is consumed from the scheduler and no extra state is entered. This is the same degeneracy the old FSM’s “auto-pre pending” trick achieved, but here it is one flop and one AND gate.

Open-page behavior. A bare RD/WR (with set_ap_i low) only reloads r_preblk and leaves r_row_valid set, so the row stays open. Successive column commands to the open row stream at the channel-wide tCCD rate enforced by global_timers; the bank timer imposes no additional per-column pacing.

## 29.6	Combinational safe_* Outputs

The readiness outputs are pure combinational functions of the timer/flag registers — one register stage behind the arbiter’s issue decision:

// ACT: bank precharged (row closed), tRP since PRE, tRC since last ACT.
assign safe_act_o = !r_row_valid && (r_rp == '0) && (r_rc == '0);
// RD/WR: row open, tRCD met, not auto-precharging.
assign safe_rd_o  = r_row_valid && (r_rcd == '0) && !r_ap_pending;
assign safe_wr_o  = safe_rd_o;
// PRE: row open, tRAS + tRTP/tWR met, not auto-precharging.
assign safe_pre_o = r_row_valid && (r_ras == '0) && (r_preblk == '0) && !r_ap_pending;

The !r_ap_pending term on safe_rd_o / safe_pre_o is what makes an auto-precharging bank refuse further column commands and a redundant explicit PRE while its internal auto-PRE is still pending — the arbiter naturally skips the bank until the row closes and safe_act_o re-asserts.

Because these are combinational off single-stage registers, the arbiter’s per-cycle pick sees the effect of the command it issued on cycle T on cycle T+1. There is no second flop between the timer and the arbiter, so a command issued this cycle cannot be double-issued next cycle — the class of hazard that motivated retiring the FSM.

## 29.7	Derived State (observability only)

A bank_state_e (state_o) is produced purely for observability — no downstream logic reads it. It is decoded combinationally from the row-valid flag and the timers:

if (r_row_valid) state_o = (r_rcd != '0) ? BANK_ACTIVATING : BANK_ACTIVE;
else             state_o = (r_rp  != '0) ? BANK_PRECHARGING : BANK_IDLE;

This gives waveform viewers and CSR telemetry the familiar IDLE / ACTIVATING / ACTIVE / PRECHARGING names without the design actually implementing them as a sequencer. It is a decode of the timers, not the source of truth.

The remaining observability outputs are single-bit non-zero flags on the timers and the auto-pre bit: obs_rcd_nz_o, obs_preblk_nz_o, obs_ras_nz_o, obs_ap_pending_o (per bank, fanned out to obs_* arrays in the aggregator).

## 29.8	Interface (pumice_bank_timers)

### 29.8.1	Clocks / Reset

| Signal | Direction | Description |
| --- | --- | --- |
| aclk | input | Controller clock |
| aresetn | input | Active-low async reset |

### 29.8.2	Timing config (CSR-backed, per-bank JEDEC cycle counts)

| Signal | Width | Constraint |
| --- | --- | --- |
| t_rcd_i | 8 | tRCD |
| t_rp_i | 8 | tRP |
| t_ras_i | 8 | tRAS |
| t_rc_i | 8 | tRC |
| t_wr_i | 8 | tWR (WR cmd → earliest PRE, incl WL+BL/2) |
| t_rtp_i | 8 | tRTP (RD cmd → earliest PRE) |

### 29.8.3	Command-event strobes from the arbiter

| Signal | Width | Description |
| --- | --- | --- |
| evt_act_i | 1 | ACT issued this cycle |
| evt_rd_i | 1 | RD issued this cycle |
| evt_wr_i | 1 | WR issued this cycle |
| evt_pre_i | 1 | PRE issued this cycle |
| evt_ap_i | 1 | Auto-precharge qualifier (with RD/WR) |
| evt_rank_i | RKW | Rank of the issued command (selects instance) |
| evt_bank_i | BKW | Bank of the issued command (selects instance) |
| evt_row_i | ROW_WIDTH | Row operand (latched on ACT) |

### 29.8.4	Per-bank readiness to the arbiter (combinational, single-stage)

| Signal | Width | Description |
| --- | --- | --- |
| bank_act_ready_o[NR][NB] | 1 each | safe_act per bank |
| bank_rdwr_ready_o[NR][NB] | 1 each | safe_rd (== safe_wr) per bank |
| bank_pre_ready_o[NR][NB] | 1 each | safe_pre per bank |
| bank_row_active_o[NR][NB] | 1 each | row_valid per bank |
| bank_open_row_o[NR][NB] | ROW_WIDTH each | Open row per bank |
| bank_state_o[NR][NB] | bank_state_e | Decoded state (observability) |

### 29.8.5	Observability

obs_act_cnt_nz_o, obs_preblk_nz_o, obs_ras_nz_o, obs_ap_pending_o — each a [NR][NB] bit array, mirroring the per-bank obs_* outputs.

## 29.9	Timing Budget

The path from a command event to updated readiness is single-cycle:

cycle T:   arbiter asserts evt_act_i with evt_rank_i=r, evt_bank_i=b
cycle T:   w_sel picks instance (r,b); r_rcd/r_ras/r_rc load, r_row_valid <- 1
cycle T+1: safe_act_o[r][b] = 0 (r_rc/row_valid now block ACT),
           safe_rd_o[r][b]  gated by r_rcd countdown

safe_*_o is combinational from the registers, so the arbiter consumes the post-event readiness on the very next cycle — no race allowing a second issue to the same bank while its constraint is still counting. This one-cycle discipline is the whole point of the FSM-free rework: readiness is exactly one register stage deep, versus the retired FSM’s two (state + accepts).

## 29.10	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| ACT → wait tRCD → RD; RD blocked until r_rcd == 0 | tRCD gate on safe_rd |
| ACT → RDA; row auto-closes once r_preblk and r_ras clear | w_ap_fire closes row, reloads r_rp; no scheduler PRE |
| ACT → WRA; r_preblk loads tWR; auto-PRE waits for tWR AND tRAS | Write-recovery folded into r_preblk |
| Bare RD (set_ap=0) keeps row open; second RD to same row allowed | Open-page: r_row_valid stays set |
| ACT → PRE (explicit) → ACT; second ACT blocked until r_rp == 0 | tRP gate on safe_act |
| Two ACTs same bank; second blocked until r_rc == 0 | tRC gate on safe_act |
| safe_rd deasserts while r_ap_pending set (mid auto-PRE) | Auto-precharging bank refuses new column commands |
| Aggregator: evt_bank_i=b only reloads instance b; others count down | w_sel routing correctness |
| state_o decode matches timers (ACTIVATING while r_rcd != 0) | Observability decode |
| Reset during any countdown → all timers and row flag clear | Reset behavior |

## 29.11	Open Questions / Future Work

- Timer width. TW = 8 covers the DDR2/LPDDR2 board targets (tRC ≈ tens of cycles). A DDR3/DDR4 family part at a higher CK:MC ratio could need wider timers; TW is already a parameter, so this is a re-elaboration, not a rework.

- Per-bank vs shared tRC. tRC is enforced per bank here. If a future family needs same-bank-group tRC differentiation (DDR4 bank groups), that logic would land in global_timers, not here — bank_timer stays bank-local by design.

- Auto-PRE vs explicit PRE priority. The RTL priority is ACT > explicit PRE > auto-PRE. If the arbiter ever issued an explicit PRE to a bank with a pending auto-PRE, the explicit PRE wins and r_ap_pending clears. The arbiter does not do this today (it treats an auto-precharging bank as un-schedulable via safe_*), but the RTL is safe if it ever did.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	ODT Control (absorbed — no standalone block)

## ABSORBED — No standalone FUB

There is no odt_ctrl module in the live RTL. The dedicated ODT-control FUB the original architecture planned was never built as a separate block. ODT responsibility is split between two modules that already exist:

- dfi_cmd_formatter.sv (see the command-formatter chapter) owns the dfi_odt_o output. ODT follows deterministically from the issued command, so it belongs on the same output stage as the ras/cas/we truth table rather than in a separate FSM.

- mode_register.sv decodes the DDR2 ODT rule bits from the mode registers into an odt_o field (see the mode-register chapter).

This chapter is retained only to explain where ODT lives and to record the current (minimal) state honestly.

Status: Absorbed / minimal — ODT is decoded but not yet actively driven

## 30.1	Where ODT Lives Today

| Concern | Live location | State |
| --- | --- | --- |
| dfi_odt_o bus (per-rank, per-phase) | dfi_cmd_formatter.sv output | Driven to 0 in v1 (decode leaves w_p0_odt at the NOP default for every op) |
| dfi_odt_o reset value | dfi_signal_pack.sv | 0 (ODT off during reset / before init) |
| DDR2 ODT-rule decode from MRs | mode_register.sv odt_o[1:0] | Decoded from MR1 ({w_mr1[6], w_mr1[2]}); informational, not wired to the pin driver |
| LPDDR2 ODT | mode_register.sv | odt_o = 0 (LPDDR2 typically point-to-point) |

mode_register exposes odt_o as one of its live decoded outputs, but the RTL comment marks it “informational; not used” — the value is computed from the DDR2 MR1 ODT bits but is not currently consumed to time a termination window. dfi_cmd_formatter drives dfi_odt_o from w_p0_odt, which the DDR2 decode never raises above its NOP default, so on the DDR2/LPDDR2 board targets (both effectively single-rank point-to-point) ODT stays off.

## 30.2	Why ODT Is Off on the Board Target

ODT is a JEDEC impedance-matching feature: a DDR2 device contains internal termination resistors that the controller selectively enables to terminate the DQ/DQS bus. The rule is asymmetric and only matters with more than one device on the bus:

- During a read, the accessed rank drives DQ; other ranks should terminate.

- During a write, the controller drives DQ; the target rank should terminate.

For a single-rank point-to-point system there is nothing else on the bus, so ODT serves no purpose and 0 is correct. The pumice board bring-up (Nexys A7, single x16 DDR2 device) is exactly this case — which is why the v1 controller ships with ODT decoded-but-not-driven and passes on silicon.

## 30.3	What a Full ODT Implementation Would Add

If a multi-rank DDR2 target is brought up, the timed ODT window would be added inside dfi_cmd_formatter’s output stage (not as a new block), consuming mode_register.odt_o plus CL/CWL and burst length to compute per-rank turn-on/ turn-off. The JEDEC cross-termination pattern (ODT-high on the non-accessed rank during a read, on the accessed rank during a write) would drive w_p0_odt per target rank instead of the current constant 0. Because the rule follows deterministically from the issued op and the MR-decoded termination value, it stays in the formatter’s truth table — the reason the standalone block was never needed.

## 30.4	Verification Notes (cocotb test plan)

| Scenario | What it proves |
| --- | --- |
| Single-rank DDR2: dfi_odt_o stays 0 for all traffic | Current behavior (point-to-point board target) |
| Reset: dfi_signal_pack drives dfi_odt_o = 0 | Reset-safe ODT-off |
| mode_register MR1 ODT bits decode into odt_o | Decode present for future use |
| LPDDR2: mode_register.odt_o = 0 | No ODT on LPDDR2 point-to-point |

## 30.5	Open Questions / Future Work

- Timed ODT window (multi-rank DDR2). Not implemented. Would live in dfi_cmd_formatter’s output stage, driven by mode_register.odt_o + CL/CWL + BL. Add when a multi-rank DDR2 board is targeted.

- Dynamic ODT (DDR3+). DDR3 introduced Rtt_Wr vs Rtt_Nom; DDR2/LPDDR2 have only Rtt_Nom. A DDR3+ family controller would extend the decode.

- ODT during refresh/precharge. Some boards want ODT held during refresh to avoid a floating bus. Revisit only if signal-integrity characterization flags it on a multi-rank target.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	AXI4 Slave Protocol

This chapter is the wire-level contract of the controller’s host AXI4 slave face – what’s supported, what’s relaxed, what’s omitted, and the timing / ordering / backpressure semantics the integrator can rely on.

The interface is implemented by pumice_axi4_ifc (section 2.1): a pair of AMBA burst splitters feed the dumb pumice_wr_intake / pumice_rd_intake blocks (each wrapping axi4_slave_wr / axi4_slave_rd), which push into the write / read CAMs.

## 31.1	Compliance Profile

| Aspect | Support level |
| --- | --- |
| AXI4 (ARM IHI 0022) signal set | Full AW/W/B/AR/R |
| AXI4-Lite | Not supported (use the register cpuif for control) |
| AXI4-Stream | Not applicable |
| AXI5 | Not supported |

## 31.2	Data Width

The core’s AXI data width is the DFI word: DW = DRAM_BEAT_WIDTH * DFI_RATE (128 by default). One AXI beat == one DFI word == DFI_RATE DRAM beats. If the SoC master runs a different width, wrap the core in pumice_top_geared, which inserts the formally-verified axi4_dwidth_converter_wr/_rd between a host-width AXI slave and the fixed-DW core (HOST_AXI_DATA_WIDTH == DW is a bit-identical generate bypass). See docs/AXI_DRAM_GEARING_SCOPE.md.

## 31.3	Burst Splitting

Each host burst is split at DRAM-burst-byte boundaries by axi_master_wr_splitter / axi_master_rd_splitter (ALIGN_MASK = BL*(DRAM_BEAT_WIDTH/8) - 1), so every command handed to an intake is exactly one DRAM burst. This is how an arbitrary AXI awlen/arlen maps onto the fixed DRAM burst length.

## 31.4	Supported Features

- INCR burst type (mandatory)

- Arbitrary INCR burst length (split into DRAM-burst commands as above)

- ID-carried requests; per-ID in-order completion (AXI4 mandate)

- Cross-ID reordering at the DRAM layer, with AXI response ordering honored at the response side (per HAS 3.1)

- awqos/arqos/awregion/arregion carried through the splitters (observed, not yet used for scheduler priority)

- Single-bit awuser/aruser/wuser ports carried through the front-end (echoed on B/R buser/ruser)

## 31.5	Unsupported / Ignored

- AXI4 exclusive accesses (awlock, arlock observed but ignored; treated as normal accesses)

- AXI4 cache-coherent behavior (awcache/arcache observed, no behavior)

- FIXED / WRAP burst types are not a design target; the front-end is built for INCR traffic

## 31.6	Backpressure Semantics

The slave asserts backpressure via the standard AXI handshake. Backpressure originates in the intake FIFOs and CAM fill:

| Channel | Backpressure reason |
| --- | --- |
| AW | AW-meta FIFO full in pumice_wr_intake, or pumice_wr_data_cam has no free slot |
| W | Write-data FIFO / write-data SRAM full in the write path |
| AR | pumice_rd_cmd_cam has no free slot |
| B | Standard bready handshake |
| R | Standard rready handshake; the drain mover in pumice_rd_cmd_cam stalls until rready |

Total in-flight depth per direction is set by NUM_ENTRIES (CAM depth) with N_SRAM_SLOTS burst-data SRAM slots per CAM.

## 31.7	Response Ordering

- Per-ID in-order – always enforced. Two accesses with the same ID complete in issue order.

- Cross-ID – reordering is allowed at the DRAM layer (the scheduler picks by bank/row/age), but the CAMs drain and the intakes emit responses so that the AXI-visible ordering rules hold.

## 31.8	Read-Your-Write Forwarding (snarf)

pumice_rd_intake probes pumice_wr_data_cam before a read is scheduled. On a hit against an unscheduled write with the same id and same burst length, the read is streamed straight from the write CAM’s SRAM (the snarf mover) with no DRAM round-trip. On a miss the read goes through the normal DRAM path.

## 31.9	Timing Contract

| Path | Behavior |
| --- | --- |
| awvalid -> awready | <= 1 cycle when the AW-meta FIFO has space |
| awready accept -> CAM push | 1 cycle |
| CAM push -> DRAM issue | scheduler / refresh dependent (typically 0 to ~256 cycles) |
| Last W beat -> B response | commit + write-recovery window + scheduler backlog |
| arvalid -> first R beat | DRAM access latency (typically 30-100 ns) + bus rounding, or ~0 on a snarf hit |
| Successive R beats (same burst) | 1 per aclk cycle (sustained streaming) |

The slave does not embed an AXI register slice. If a target needs one for closure, add an external axi_register_slice from the AMBA library, or use pumice_top_geared (whose dwidth converters register the boundary).

## 31.10	Error Responses

| Condition | Response | Notes |
| --- | --- | --- |
| Traffic before init_done_o | held / SLVERR | Poll STATUS.init_done (or the init_done_o pin) before issuing traffic |
| DFI read return flagged bad by the aligner | SLVERR | Propagated on the affected R beats |

DECERR is never returned – the address space is monolithic.

## 31.11	Open Questions / Future Work

- QoS-driven priority. awqos/arqos are carried but not yet consumed by pumice_cmd_arbiter. A future revision would fold them into the pick.

- Wider USER signals. The front-end carries single-bit user today; wider sideband would need a USER_WIDTH parameter threaded through the intakes and CAMs.

- AXI register-slice option. Currently external / via the geared wrapper; a build-time embed could be added for very-high-frequency targets.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 32	DFI v2.1 Master Protocol

This chapter is the wire-level contract of the controller’s DFI 2.1 master face – what’s driven, what’s tied, what’s sampled, what’s deferred.

The interface is implemented by pumice_dfi_layer (section 2.4): pumice_dfi_cmd_path (with dfi_cmd_formatter + dfi_signal_pack) drives the command bus, pumice_dfi_wr_serializer drives write data, pumice_dfi_rd_aligner captures read data, and pumice_dfi_cdc is the single clock crossing to aclk.

## 32.1	DFI Spec Reference

Canonical: the DFI 2.1 specification (Denali cleartext copy at /home/seang/github/cold_storage/MemorySpecs/). The controller covers the subset of DFI 2.1 sub-interfaces needed for DDR2/LPDDR2 operation; higher-version sub-interfaces are not driven.

## 32.2	Sub-Interface Support Summary

| Sub-Interface | Direction | Supported? | Notes |
| --- | --- | --- | --- |
| Command | Controller -> PHY | Yes | Per-phase; command placed on wr_phase/rd_phase |
| Write-Data | Controller -> PHY | Yes | dfi_wrdata presented at t_phy_wrlat after WR fire |
| Read-Data | PHY -> Controller | Yes | dfi_rddata_en at t_rddata_en after RD fire; captured on dfi_rddata_valid |
| Status (init) | Both | Yes | dfi_init_start / dfi_init_complete only |
| Update | Both | No | ctrlupd_* / phyupd_* not driven in v1 |
| Training | Both | No | PHY-side (a7ddrphy self-trains at startup) |
| Frequency Change | Both | No | No use case |
| Low-Power | Controller -> PHY | No | No PHY-side low-power coordination |
| Error / CRC | - | No | Not used in DDR2/LPDDR2 here |

## 32.3	Clock and CDC

The DFI pin bus is driven in the dfi_clk domain. Unlike the classic “DFI is on the controller clock” model, this controller keeps the whole DFI datapath on a dedicated PHY clock and places the single controller-to-PHY CDC inside pumice_dfi_layer (pumice_dfi_cdc, async gaxi FIFOs only). The command, write-data, and read-data streams plus the init handshake cross there; the internal datapath unit is the whole DFI word, so one FIFO word is one dfi_clk cycle and the path is bubble-free at rate. a7ddrphy handles any further gearing to the DRAM clock.

## 32.4	Command Sub-Interface

### 32.4.1	Phase Topology

The command bus is per-phase x DFI_RATE wide. dfi_address_o is ROW_WIDTH * DFI_RATE, dfi_bank_o is $clog2(NUM_BANKS) * DFI_RATE, and each control strobe (dfi_cas_n_o / dfi_ras_n_o / dfi_we_n_o) is 1 * DFI_RATE. pumice_dfi_cmd_path places the active JEDEC command on the wr_phase / rd_phase slot (programmed by the DFI_PHASE CSR) and drives NOP-equivalent idle on the other phases.

### 32.4.2	Chip-Select and ODT

dfi_cs_n_o and dfi_odt_o are each NUM_RANKS * DFI_RATE wide (rank inner, phase outer, as the DFI_CS_BUS_W = NUM_RANKS * DFI_RATE packing implies). The formatter drives the addressed rank’s CS_n active for a single-rank command; init-time REF broadcasts. ODT is driven inside dfi_cmd_formatter / mode_register – there is no standalone odt_ctrl block.

### 32.4.3	DDR2 vs LPDDR2 Encoding

For memtype_i = DDR2, commands use the classic ras/cas/we/cs strobe encoding. For memtype_i = LPDDR2, dfi_cmd_formatter takes the memtype branch and emits bit-exact JESD209-2F CA-bus commands (Table 60): the 10-bit CA bus over 2 edges packed as a flat 20-bit word on dfi_address, with ras/cas/we idle. See rtl/LPDDR2_CA_ENCODING.md.

## 32.5	Write-Data Sub-Interface

pumice_dfi_wr_serializer presents dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask exactly t_phy_wrlat dfi_clk cycles after the wr_fire strobe from the command path. The write burst arrives from the write CAM as whole DFI words {last, strb, data}; the serializer splits into the per-phase data / enable / mask.

dfi_wrdata_en_o is DFI_RATE wide (one enable per phase). A mask bit of 1 means “do not write this byte” per DFI convention; the mask payload carried from the CAM is aligned to that convention.

## 32.6	Read-Data Sub-Interface

pumice_dfi_rd_aligner drives dfi_rddata_en_o (DFI_RATE wide) exactly t_rddata_en dfi_clk cycles after the rd_fire strobe, then captures dfi_rddata_i when dfi_rddata_valid_i asserts, packing whole DFI words ({last, resp, data}) into the read FIFO for the crossing back to aclk.

### 32.6.1	Read-Data Error Handling

dfi_rddata carries no error bit in DFI 2.1. A missing / malformed read is surfaced by the aligner’s resp field and propagated as SLVERR on the affected AXI R beats. On-silicon this path was hardened during board bring-up: t_rddata_en and the DFI read-data delay are runtime CSRs, and rd_phase = 0 is correct for a7ddrphy (which handles read gearing internally).

## 32.7	Status / Init Sub-Interface

dfi_init_start_o and dfi_init_complete_i are the only status signals. The init_sequencer (in pumice_mem_cmd_scheduler) asserts dfi_init_start and walks the JEDEC MRS init, waiting on dfi_init_complete from the PHY plus the programmed init-timing CSRs. These cross the CDC in pumice_dfi_cdc.

## 32.8	Pin Tie-Offs

| Pin | Tie-off (when applicable) |
| --- | --- |
| dfi_ras_n_o / dfi_cas_n_o / dfi_we_n_o | Idle in LPDDR2 (command rides the CA bus on dfi_address) |
| Update / frequency / low-power buses | Not present / not driven |

## 32.9	Open Questions / Future Work

- DFI error / update. ctrlupd_* / phyupd_* and a dfi_error input are not wired in v1; a later revision could add quiet-point update handshaking and route read errors to an IRQ.

- Per-rank CS width form. The controller produces the flat NUM_RANKS * DFI_RATE vector form; a PHY expecting a different packing needs a thin wrapper at the boundary.

- DFI 3+ migration. When the DDR3-LPDDR3 family controller arrives, the sub-interface set grows substantially. Swap pumice_dfi_layer and add a dedicated DFI section in that MAS; keep this one DDR2/LPDDR2-scoped.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 33	Register Access Interface (PeakRDL cpuif)

Per HAS §2.4 §3 for the per-signal port list and HAS §4.3 for the architectural view. This chapter is the wire-level contract for the controller’s register access interface.

The register block is a PeakRDL-generated module (pumice_csr, from rtl/macro/pumice_csr.rdl) instantiated directly in pumice_top. It exposes a PeakRDL passthrough CPU interface (s_cpuif_*), not a hand-written APB slave. Any APB (or AXI-Lite) bridge is external/optional — the SoC attaches whatever register transport it uses and drives the cpuif. All register storage lives in the aclk (MC) domain; there is no separate apb_pclk domain and no on-die CDC inside the register block.

## 33.1	The cpuif Contract

pumice_top presents the PeakRDL passthrough interface on aclk/aresetn:

| Direction | Signal | Meaning |
| --- | --- | --- |
| in | s_cpuif_req | Request strobe |
| in | s_cpuif_req_is_wr | 1 = write, 0 = read |
| in | s_cpuif_addr[11:0] | Byte address (12-bit, 4 KB region) |
| in | s_cpuif_wr_data[31:0] | Write data |
| in | s_cpuif_wr_biten[31:0] | Per-bit write-enable (bit strobe) |
| out | s_cpuif_req_stall_wr | Back-pressure a write request |
| out | s_cpuif_req_stall_rd | Back-pressure a read request |
| out | s_cpuif_rd_ack | Read response valid |
| out | s_cpuif_rd_err | Read decode error (unmapped address) |
| out | s_cpuif_rd_data[31:0] | Read data |
| out | s_cpuif_wr_ack | Write response valid |
| out | s_cpuif_wr_err | Write decode error (unmapped address) |

The interface is single-request/single-response with a stall handshake. Registers are 32-bit, naturally aligned on 4-byte boundaries. Sub-word writes use s_cpuif_wr_biten (per-bit granularity); a full-word write drives all 32 biten bits.

## 33.2	Attaching an APB / AXI-Lite Bridge

Because the interface is a generic passthrough cpuif, the register transport is an integration choice made outside this controller:

- An APB3/APB4 slave bridge translates psel/penable/pwrite/paddr/pwdata/prdata/pslverr to the cpuif and back. pslverr maps from s_cpuif_rd_err/s_cpuif_wr_err.

- An AXI-Lite slave bridge is equally valid; the cpuif does not assume APB.

- If the register bus is in a different clock domain than aclk, the CDC lives in that external bridge — not in pumice_csr.

No pumice_top parameter selects the transport; the bridge is a separate module in the SoC’s register fabric.

## 33.3	Address Space

The register block decodes a 4 KB region (12-bit address). Layout, matching rtl/macro/pumice_csr.rdl:

| Address range | Region |
| --- | --- |
| 0x000 – 0x008 | Control / Status / Status-history |
| 0x010 – 0x01C | Timing parameters (RC/RCD/RP/RAS, RFC/REFI, RRD/FAW/WTR/CCD, CL/CWL/WR) |
| 0x020 – 0x02C | Mode Register values (MR0..MR3) |
| 0x030 – 0x038 | LPDDR2-specific (PASR bank/segment masks, temp derate) |
| 0x040 – 0x05C | Scheduler / Page / Refresh / Addr-map / Init tuning + init timing |
| 0x054, 0x060, 0x064 | tRTP/tRTW, DFI command phase, PHY/DFI data timing |
| 0x080 – 0x09C | Per-bank row-hit observation (NUM_BANKS = 8) |
| 0x0C0 – 0x0DC | Per-bank refresh-latency observation |
| 0x100 – 0x138 | System observation / telemetry |
| 0x1C0 – 0x1E0 | Packed observation-word harvest (9 words) |
| 0xFF0 – 0xFF4 | Module identification + build hash |

The complete register/field table is in §4.2; it is generated from the RDL and must match it exactly.

## 33.4	Behavior

### 33.4.1	Access Cycles

Each request drives s_cpuif_req with s_cpuif_req_is_wr, s_cpuif_addr, and (for writes) s_cpuif_wr_data/s_cpuif_wr_biten. The block responds with s_cpuif_rd_ack/s_cpuif_wr_ack (and optionally asserts a stall while it is busy). Reads return s_cpuif_rd_data; unmapped accesses assert s_cpuif_rd_err/s_cpuif_wr_err.

### 33.4.2	Sub-Word Accesses

s_cpuif_wr_biten carries a per-bit write mask, so byte- or field-level writes are supported natively. A bridge that lacks byte strobes simply drives all biten bits on every write.

### 33.4.3	Error Conditions

The block asserts a decode error (s_cpuif_rd_err / s_cpuif_wr_err) for:

| Condition | Notes |
| --- | --- |
| Access to an unmapped address (gaps / RSVD) | rd_err/wr_err; read data is 0 |
| Write to a read-only register/field | Write is dropped; field unchanged |

Writes to reserved (sw = r) fields inside an otherwise-writable register are ignored per the RDL field access; the register keeps its value. There is no scheme/rule sub-decode error (the old ADDR_MAP_TUNING scheme decode is retired — address mapping is now the plain ADDR_MAP.bank_lsb field, see §4.2/§4.4).

## 33.5	Config-Drive Model

pumice_top wires the register block’s hwif_out.* outputs by name directly into pumice_core (see the top-level instantiation in rtl/top/pumice_top.sv). Configuration is therefore live combinational drive from the register storage into the datapath — software writes a field and the corresponding core input tracks it. There is no staging/commit or quiet-point protocol in this architecture (see §4.3). Status/observation readback (hwif_in.*) is currently tied off in pumice_top.

## 33.6	Open Questions / Future Work

- Observation readback. hwif_in is presently tied to 0; wiring the live status/telemetry counters back into the read path is a follow-up.

- Bridge selection. The passthrough cpuif is transport-agnostic. A packaged APB and AXI-Lite bridge pair (with the CDC option) would simplify SoC integration.

- Quiet-point commit. The old two-cell staging/commit model was removed with the config-drive refactor. If a future field must not change mid-transaction, a per-field commit gate can be reintroduced.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 34	Register Map

This MAS chapter is the authoritative field-level register map, transcribed directly from rtl/macro/pumice_csr.rdl and mirrored by the generated dv/tbclasses/pumice_regmap.py. If this table and the RDL ever disagree, the RDL wins. Offsets are byte addresses in the 4 KB (12-bit) region.

## 34.1	Source of Truth

The register map is a SystemRDL source, rtl/macro/pumice_csr.rdl. It is compiled by bin/peakrdl_generate.py into:

| Artifact | Consumer |
| --- | --- |
| regs/generated/rtl/pumice_csr.sv + pumice_csr_pkg.sv | The PeakRDL passthrough register block instantiated in pumice_top (hwif_in/hwif_out structs) |
| dv/tbclasses/pumice_regmap.py | The DV RegisterMap by-name access model (offset/field/default) |

pumice_top drives hwif_out.* fields by name straight into pumice_core — there is no hand-written register file and no APB slave inside the controller (see §4.1). DV programs every register by name via pumice_regmap.py; hardcoded offsets are forbidden.

## 34.2	Register Summary

| Offset | Register | Purpose |
| --- | --- | --- |
| 0x000 | CTRL | Init / power / soft-reset request bits |
| 0x004 | STATUS | Init / power / version status (RO, hw-written) |
| 0x008 | STATUS_HISTORY | Last 8 power-state transitions (RO) |
| 0x010 | TIMINGS_RC_RCD_RP_RAS | tRC / tRCD / tRP / tRAS |
| 0x014 | TIMINGS_RFC_REFI | tRFC / tREFI |
| 0x018 | TIMINGS_RRD_FAW_WTR_CCD | tRRD / tFAW / tWTR / tCCD |
| 0x01C | TIMINGS_CL_CWL_WR | CL / CWL / tWR / tRFCpb |
| 0x020 | MR0 | Mode Register 0 value |
| 0x024 | MR1 | Mode Register 1 value |
| 0x028 | MR2 | Mode Register 2 value |
| 0x02C | MR3 | Mode Register 3 value |
| 0x030 | PASR_BANK_MASK_RANK0 | LPDDR2 PASR per-bank mask (rank 0) |
| 0x034 | PASR_SEG_MASK_RANK0 | LPDDR2 PASR segment mask (rank 0) |
| 0x038 | TEMP_DERATE_RANK0 | LPDDR2 MR4 temperature class (rank 0, RO) |
| 0x040 | SCHED_TUNING | Scheduler runtime knobs |
| 0x044 | PAGE_PRED_TUNING | HAPPY-mode predictor knobs |
| 0x048 | REFRESH_TUNING | Refresh policy + page-policy override + ZQCS interval |
| 0x04C | ADDR_MAP | Address-map: bank_lsb + XOR-hash (replaces ADDR_MAP_TUNING) |
| 0x050 | INIT_TUNING | ZQ retries + per-step init timeout |
| 0x054 | TIMINGS_RTP_RTW | tRTP / tRTW |
| 0x058 | INIT_TIMING0 | Init waits: tINIT / tDLLK |
| 0x05C | INIT_TIMING1 | Init waits: tMRD / tRP / tRFC |
| 0x060 | DFI_PHASE | DFI READ/WRITE command sub-phase placement |
| 0x064 | PHY_TIMING | t_phy_wrlat / t_rddata_en / memtype / refresh_burst |
| 0x080..0x09C | OBS_ROW_HIT[8] | Per-bank row-hit count (RO, read-clear) |
| 0x0C0..0x0DC | OBS_REF_LATENCY[8] | Per-bank refresh-blocking cycles (RO) |
| 0x100..0x138 | OBS_* | System observation / telemetry (RO) |
| 0x1C0..0x1E0 | OBS_WORDS[9] | Packed obs_* harvest words (RO) |
| 0xFF0 | ID | Module ID (version / memtype / n_phases / 0xD2) |
| 0xFF4 | BUILD | Build hash |

## 34.3	Field-Level Detail

All registers are 32-bit; unlisted bits are reserved (RSVD, sw = r). “Default” is the RDL reset value.

### 34.3.1	CTRL @ 0x000 (rw)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 0 | init_start | 0 | Write 1 to start init (swmod) |
| 1 | init_force_restart | 0 | Write 1 to force re-init mid-sequence (swmod) |
| 4 | pwr_req_low_power | 0 | Request power-down |
| 5 | pwr_req_dpd | 0 | Request DPD (LPDDR2 only) |
| 6 | pwr_req_active | 0 | Request return to ACTIVE |
| 7 | pwr_req_self_refresh | 0 | Request self-refresh |
| 31 | soft_reset | 0 | Write 1 to assert internal soft reset (self-clearing, swmod) |

### 34.3.2	STATUS @ 0x004 (RO, hw-written)

| Bits | Field | Notes |
| --- | --- | --- |
| 0 | init_done | Init complete |
| 1 | init_error | Init error |
| 7:4 | power_state | Current power-state FSM state (encoded) |
| 8 | pasr_active | LPDDR2: PASR mask is non-zero |
| 23:16 | init_step_dbg | Current init step number (bring-up) |
| 31 | version_match | Build matches expected version |

### 34.3.3	STATUS_HISTORY @ 0x008 (RO)

| Bits | Field | Notes |
| --- | --- | --- |
| 31:0 | history | 8 x 4-bit power-state history; most recent in [3:0] |

### 34.3.4	TIMINGS_RC_RCD_RP_RAS @ 0x010 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 7:0 | tRC | 60 | MC cycles |
| 15:8 | tRCD | 15 |  |
| 23:16 | tRP | 15 |  |
| 31:24 | tRAS | 40 |  |

### 34.3.5	TIMINGS_RFC_REFI @ 0x014 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 15:0 | tRFC | 200 | or tRFCab |
| 31:16 | tREFI | 1950 |  |

### 34.3.6	TIMINGS_RRD_FAW_WTR_CCD @ 0x018 (rw)

| Bits | Field | Default (dec) |
| --- | --- | --- |
| 7:0 | tRRD | 6 |
| 15:8 | tFAW | 35 |
| 23:16 | tWTR | 4 |
| 31:24 | tCCD | 4 |

### 34.3.7	TIMINGS_CL_CWL_WR @ 0x01C (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 7:0 | CL | 6 | CAS latency |
| 15:8 | CWL | 4 | CAS write latency |
| 23:16 | tWR | 15 | Write recovery |
| 31:24 | tRFCpb | 70 | LPDDR2 per-bank tRFC |

### 34.3.8	MR0..MR3 @ 0x020 / 0x024 / 0x028 / 0x02C (rw)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 15:0 | VAL | 0 | Mode-register value loaded during init |

### 34.3.9	PASR_BANK_MASK_RANK0 @ 0x030 (rw)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 7:0 | pasr_banks | 0 | LPDDR2 MR16; bit N=1 masks bank N |

### 34.3.10	PASR_SEG_MASK_RANK0 @ 0x034 (rw)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 7:0 | pasr_segs | 0 | LPDDR2 segment mask |

### 34.3.11	TEMP_DERATE_RANK0 @ 0x038 (RO, hw-written)

| Bits | Field | Notes |
| --- | --- | --- |
| 1:0 | temp_class | LPDDR2 MR4: 00 nominal, 01 2x refresh, 10 4x refresh |

### 34.3.12	SCHED_TUNING @ 0x040 (rw)

| Bits | Field | Default | Access | Notes |
| --- | --- | --- | --- | --- |
| 3:0 | lookahead_active | 0 | rw | Active lookahead window (0 disables) |
| 4 | force_inorder | 0 | rw | 1 = force first-ready FIFO |
| 5 | happy_enable | 1 | rw | HAPPY predictor active (if synthesized) |
| 15:8 | age_max_runtime | 0 | rw | Runtime AGE_MAX override (0 = build default) |
| 23:16 | txn_queue_high_water | 0 | rw | Backpressure threshold |
| 27:24 | lookahead_max_obs | 0 | RO | Echo of build-time LOOKAHEAD_DEPTH_MAX |

### 34.3.13	PAGE_PRED_TUNING @ 0x044 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 15:0 | warmup_cycles | 1024 |  |
| 23:16 | hysteresis | 2 |  |

### 34.3.14	REFRESH_TUNING @ 0x048 (rw)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 1:0 | refpb_policy_or | 0 | 00 build-time, 01 RR, 10 OLDEST_FIRST, 11 DARP |
| 3:2 | page_policy_or | 0 | 00 build-time, 01 OPEN, 10 CLOSE, 11 HAPPY_HYBRID |
| 7:4 | refresh_defer_active | 1 | Active refresh deferral count |
| 31:16 | zqcs_freq_hz | 1 | Periodic ZQCS interval in Hz (0 disables) |

### 34.3.15	ADDR_MAP @ 0x04C (rw) — replaces the retired ADDR_MAP_TUNING

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 4:0 | bank_lsb | 0x0A (=COL_WIDTH, ROW_MAJOR) | Bank-field LSB in the byte-offset-stripped word address; RTL clamps to [0, COL_WIDTH] |
| 8 | hash_en | 0 | Enable bank XOR-hash: bank ^= fold(row) ^ hash_seed |
| 23:16 | hash_seed | 0 | XOR-hash seed (seed[BW-1:0]) |

There is no scheme selector, scheme_or, or synth_mask_obs. ROW_MAJOR / BANK_INTERLEAVE / XOR_HASH are just settings of this one register (see §4.4 and rtl/fub/addr_mapper.sv).

### 34.3.16	INIT_TUNING @ 0x050 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 3:0 | zq_retries | 3 | ZQ retries (1..8) |
| 15:8 | init_timeout_ms | 10 | Init timeout ms |

### 34.3.17	TIMINGS_RTP_RTW @ 0x054 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 7:0 | tRTP | 4 | Read to precharge |
| 15:8 | tRTW | 6 | Read to write |

### 34.3.18	INIT_TIMING0 @ 0x058 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 15:0 | t_init_wait | 512 | CKE/tINIT settle |
| 31:16 | t_dll_wait | 256 | DLL lock (tDLLK) |

### 34.3.19	INIT_TIMING1 @ 0x05C (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 7:0 | t_mrd_wait | 8 | post mode-reg (tMRD) |
| 15:8 | t_rp_wait | 8 | post precharge (tRP) |
| 23:16 | t_rfc_wait | 16 | post refresh (tRFC) |

### 34.3.20	DFI_PHASE @ 0x060 (rw)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 2:0 | rd_phase | 0 | READ command DFI sub-phase |
| 6:4 | wr_phase | 0 | WRITE command DFI sub-phase |

Sliced to clog2(DFI_RATE) bits downstream; upper bits ignored when DFI_RATE is small. On the Nexys A7 a7ddrphy, rd_phase=0 (the PHY handles rdphase internally).

### 34.3.21	PHY_TIMING @ 0x064 (rw)

| Bits | Field | Default (dec) | Notes |
| --- | --- | --- | --- |
| 7:0 | t_phy_wrlat | 0 | WR cmd -> dfi_wrdata_en (0 = a7ddrphy pre-pull) |
| 15:8 | t_rddata_en | 6 | RD cmd -> dfi_rddata_en window |
| 16 | memtype | 0 | 0 = DDR2, 1 = LPDDR2 |
| 23:20 | refresh_burst | 1 | REFs drained per request (1..8) |

### 34.3.22	Observation registers (RO)

| Offset | Register / array | Field | Notes |
| --- | --- | --- | --- |
| 0x080..0x09C | OBS_ROW_HIT[8] | VAL[31:0] | Per-bank row-hit count; read-clear (onread = rclr) |
| 0x0C0..0x0DC | OBS_REF_LATENCY[8] | VAL[31:0] | Per-bank refresh-blocking cycles |
| 0x100 | OBS_TXN_QUEUE_DEPTH_MAX | VAL | Max queue depth observed |
| 0x104 | OBS_TXN_QUEUE_DEPTH_AVG | VAL | Time-averaged queue depth |
| 0x108 | OBS_REFRESH_PENDING_MAX | VAL | Max refresh_pending observed |
| 0x10C..0x118 | OBS_REFRESH_DEFER_HIST_0..3 | VAL | Refresh-deferral histogram bins |
| 0x120 | OBS_PAGE_PRED_ACCURACY | VAL | HAPPY rolling prediction accuracy (%) |
| 0x130 | OBS_AXI_R_LATENCY_AVG | VAL | Avg AXI read latency (cycles) |
| 0x134 | OBS_AXI_R_LATENCY_P99 | VAL | 99th-pct AXI read latency |
| 0x138 | OBS_AXI_W_LATENCY_AVG | VAL | Avg AXI write latency |
| 0x1C0..0x1E0 | OBS_WORDS[9] | VAL | Packed obs_* harvest words |

### 34.3.23	ID @ 0xFF0 (RO) — reset 0xD2020001

| Bits | Field | Value | Notes |
| --- | --- | --- | --- |
| 7:0 | version | 0x01 | Build version |
| 15:8 | memtype | 0x00 | 0 = DDR2, 1 = LPDDR2 |
| 23:16 | n_phases | 0x02 | Gear ratio (1/2/4) |
| 31:24 | module_id | 0xD2 | Fixed 0xD2 |

### 34.3.24	BUILD @ 0xFF4 (RO)

| Bits | Field | Default | Notes |
| --- | --- | --- | --- |
| 31:0 | VAL | 0 | Build hash word |

## 34.4	Multi-Rank / Multi-Bank Registers

The RDL declares per-bank observation arrays with NUM_BANKS = 8 for rank 0 (OBS_ROW_HIT[8] at 0x080, OBS_REF_LATENCY[8] at 0x0C0). PASR / temperature registers are declared per rank as *_RANK0 for the default single-rank build. The generated pumice_regmap.py flattens arrays into indexed register names (e.g., OBS_ROW_HIT0_ROW_HIT, OBS_REF_LATENCY7_REF_LAT, OBS_WORDS8_WORD). Multi-rank builds add the corresponding *_RANK{N} registers; there is no separate capability vector register in this RDL — software reads memtype/n_phases from ID (0xFF0).

## 34.5	Reset Values

Reset defaults come straight from the RDL = <value> initializers (echoed in the default/offset entries of pumice_regmap.py). They are chosen so a “do nothing” bring-up sees a sane DDR2 baseline; DV programs the workload-specific values by name before triggering init.

## 34.6	Open Questions / Future Work

- Observation readback wiring. The RO/telemetry registers are declared and generated, but pumice_top currently ties hwif_in to 0 (see §4.1). Connecting the live counters is a follow-up.

- Multi-rank register generation. The single-rank build declares only *_RANK0. A NUM_RANKS-driven RDL loop for the PASR/temp/observation windows is the natural extension.

- RDL <-> regmap drift check. pumice_regmap.py is generated from the RDL; a CI diff would catch manual edits.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 35	Runtime CSR Fields (Config-Drive Model)

Per HAS §5.1 for the build-time-vs-runtime principle and §4.2 for the field-level register map. This MAS chapter is the implementation protocol for how runtime CSR fields reach the live datapath.

In the rearchitected controller, pumice_top wires the register block’s hwif_out.* fields by name directly into pumice_core (see rtl/top/pumice_top.sv). There is no staging/commit two-cell architecture, no CTRL.config_apply, no STATUS.config_settled, and no quiet-point drain FSM. A CSR write updates the field’s storage and the corresponding core input tracks it combinationally.

## 35.1	What “Runtime” Means Here

Most of the controller’s behavior is programmed at runtime by CSR fields, not fixed by parameters. The following are all live CSR-driven core inputs (see the u_core port map in rtl/top/pumice_top.sv):

| Domain | CSR field(s) | Core input |
| --- | --- | --- |
| Memory type | PHY_TIMING.memtype | memtype_i (0=DDR2, 1=LPDDR2) |
| Page policy | REFRESH_TUNING.page_policy_or | page_policy_i |
| Address mapping | ADDR_MAP.bank_lsb / .hash_en / .hash_seed | bank_lsb_i / hash_en_i / hash_seed_i |
| Core JEDEC timing | TIMINGS_RC_RCD_RP_RAS, TIMINGS_CL_CWL_WR.tWR, TIMINGS_RTP_RTW, TIMINGS_RRD_FAW_WTR_CCD, TIMINGS_RFC_REFI.tREFI | t_rcd_i/t_rp_i/t_ras_i/t_rc_i/t_wr_i/t_rtp_i/t_rtw_i/t_faw_i/t_rrd_i/t_wtr_i/t_ccd_i/t_refi_i |
| Refresh burst | PHY_TIMING.refresh_burst | refresh_burst_i |
| Init waits | INIT_TIMING0 / INIT_TIMING1 | t_init_wait_i/t_dll_wait_i/t_mrd_wait_i/t_rp_wait_i/t_rfc_wait_i |
| DFI phase | DFI_PHASE.rd_phase / .wr_phase | rd_phase_i/wr_phase_i (sliced to clog2(DFI_RATE)) |
| PHY data timing | PHY_TIMING.t_phy_wrlat / .t_rddata_en | t_phy_wrlat_i/t_rddata_en_i |

CL/CWL/BL are not driven from the timing CSRs at the core boundary — they are decoded from the mode-register shadow inside the scheduler layer (mode_register.sv) as the init sequencer programs MR0..MR3 (see §5.1). The TIMINGS_CL_CWL_WR.CL/.CWL fields are informational in this build.

## 35.2	Commit Semantics

Because the fields drive the core directly, a write takes effect as soon as it lands in the register storage. There is no explicit apply step. Two practical timing classes:

| Class | Fields | When it takes effect |
| --- | --- | --- |
| Sampled at an event boundary | JEDEC timings, refresh interval/burst, page policy, scheduler knobs | On the next counter reload / arbiter decision that consumes the value |
| Sampled continuously | memtype, bank_lsb/hash_*, DFI phase, PHY data timing | Combinationally; affects the next command that uses the mapper/formatter |

## 35.3	Safe-Change Guidance

The controller does not enforce a quiet point, so software is responsible for not changing a field mid-transaction when that would corrupt in-flight state:

- Timings, page policy, scheduler knobs, refresh interval: safe to change during light traffic; the new value is picked up at the next event boundary. For a clean swap, quiesce AXI traffic first (stop issuing, let outstanding responses drain) then write.

- memtype, address-map (bank_lsb/hash_*): these define how addresses decode and how commands are encoded. Change them only before init (or with the datapath fully idle). Changing address mapping under traffic re-decodes in-flight addresses inconsistently.

- DFI phase / PHY data timing: board bring-up knobs; set once during bring-up and leave fixed. rd_phase/t_rddata_en/t_phy_wrlat are matched to the attached PHY (e.g., the Nexys A7 a7ddrphy uses rd_phase=0, t_rddata_en=6, t_phy_wrlat=0).

## 35.4	Recommended Programming Order

For a clean bring-up the register writes happen while init is still gated (before CTRL.init_start):

// 1. Select memory type and address mapping (must be set before init)
csr_write(PHY_TIMING, memtype_field | t_rddata_en_field | t_phy_wrlat_field | refresh_burst_field);
csr_write(ADDR_MAP,   bank_lsb_field | hash_en_field | hash_seed_field);

// 2. Program JEDEC timings for the attached part
csr_write(TIMINGS_RC_RCD_RP_RAS, ...);
csr_write(TIMINGS_RFC_REFI,      ...);
csr_write(TIMINGS_RRD_FAW_WTR_CCD, ...);
csr_write(TIMINGS_CL_CWL_WR,     ...);
csr_write(TIMINGS_RTP_RTW,       ...);
csr_write(INIT_TIMING0, ...);  csr_write(INIT_TIMING1, ...);

// 3. Program MR0..MR3 and (LPDDR2) PASR
csr_write(MR0, ...); csr_write(MR1, ...); csr_write(MR2, ...); csr_write(MR3, ...);

// 4. Trigger init — the values above are already live at the core boundary
csr_write(CTRL, CTRL_INIT_START);

There is no apply/poll handshake between the writes — each write is already visible to the core.

## 35.5	Open Questions / Future Work

- Mid-traffic re-tuning guard. A future revision could add an optional per-field commit gate (re-introducing a lightweight quiet point) for fields that software wants to change under load. Not present in this build.

- CL/CWL CSR coupling. The TIMINGS_CL_CWL_WR.CL/.CWL fields are currently informational; the live decode comes from the MR shadow. Deciding whether the CSR or the MR shadow is authoritative is a cleanup item.

- Observation readback. STATUS/OBS_* are declared but hwif_in is tied off in pumice_top (see §4.1); wiring them back is a follow-up.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 36	Family Config Knobs (DDR2 / LPDDR2)

Per HAS §5.4 for the family-wide config philosophy. This MAS chapter is the implementation-level detail for this controller specifically: which CSR fields select DDR2-vs-LPDDR2 behavior and which fields are present-but-inert on this generation.

The controller is a two-family design (DDR2 + LPDDR2). The single most important family selector is PHY_TIMING.memtype (0 = DDR2, 1 = LPDDR2), which pumice_top maps to the core’s memtype_i and which the init sequencer and DFI command formatter branch on. There is no separate family capability vector register; ID @ 0xFF0 echoes the build’s memtype and phase count.

## 36.1	The Family Selector: PHY_TIMING.memtype

| memtype | Family | What it changes |
| --- | --- | --- |
| 0 | DDR2 | init_sequencer runs the DDR2 MRS/precharge/refresh chain; dfi_cmd_formatter drives the DDR2 RAS/CAS/WE command bus |
| 1 | LPDDR2 | init_sequencer runs the LPDDR2 MRW chain (MR63/MR10/MR1/MR2/MR3); dfi_cmd_formatter packs the JESD209-2F CA-bus word onto dfi_address |

memtype is set before init and left fixed (see §4.3). DDR2 is the board target; LPDDR2 is the family-reuse path. Both pass the full sim suite.

## 36.2	Field-by-Field Applicability

Fields below are the real CSR fields in rtl/macro/pumice_csr.rdl (see §4.2). “DDR2 / LPDDR2” notes whether the field is meaningful on each family in this build.

### 36.2.1	SCHED_TUNING (0x040)

| Field | Applicability |
| --- | --- |
| lookahead_active | Both; scheduler lookahead window (0 disables) |
| force_inorder | Both; forces first-ready FIFO ordering |
| happy_enable | Both, only meaningful when the HAPPY predictor is synthesized/selected |
| age_max_runtime | Both; anti-starvation AGE_MAX override |
| txn_queue_high_water | Both; backpressure threshold |
| lookahead_max_obs | RO echo of build-time LOOKAHEAD_DEPTH_MAX |

### 36.2.2	REFRESH_TUNING (0x048)

| Field | Applicability |
| --- | --- |
| page_policy_or | Both; drives page_policy_i (00 build-time, 01 OPEN, 10 CLOSE, 11 HAPPY_HYBRID) |
| refresh_defer_active | Both; refresh deferral / batching count |
| zqcs_freq_hz | Both; periodic ZQCS interval (DDR2 has no ZQCL, but ZQCS calibration short is JEDEC) |
| refpb_policy_or | LPDDR2-relevant (per-bank refresh); DDR2 uses all-bank REFab only |

### 36.2.3	ADDR_MAP (0x04C) — family-agnostic

| Field | Applicability |
| --- | --- |
| bank_lsb | Both; the single address-map knob. = COL_WIDTH -> ROW_MAJOR; smaller -> interleave. No scheme selector exists. |
| hash_en | Both; enables the bank XOR-hash (the old XOR_HASH “scheme”) |
| hash_seed | Both; XOR-hash seed |

DDR2/LPDDR2 have no bank groups, so there is no bank-group field. The retired ADDR_MAP_TUNING register (with scheme_or / synth_mask_obs / xor_seed_runtime) does not exist; all address mapping is the ADDR_MAP register above.

### 36.2.4	INIT_TUNING (0x050) / INIT_TIMING0/1 (0x058/0x05C)

| Field | Applicability |
| --- | --- |
| zq_retries, init_timeout_ms | Both |
| t_init_wait, t_dll_wait | Both; on LPDDR2 the sequencer reuses t_init_wait for tINIT4 and t_dll_wait for tZQINIT (see §5.1) |
| t_mrd_wait, t_rp_wait, t_rfc_wait | DDR2 uses all three; LPDDR2 reuses t_mrd_wait for post-MRW (t_rp_wait/t_rfc_wait are inert on the LPDDR2 chain) |

### 36.2.5	PHY_TIMING (0x064)

| Field | Applicability |
| --- | --- |
| memtype | The family selector (above) |
| t_phy_wrlat | Both; WR cmd -> dfi_wrdata_en (0 = a7ddrphy pre-pull) |
| t_rddata_en | Both; RD cmd -> dfi_rddata_en window |
| refresh_burst | Both; REFs drained per request |

### 36.2.6	LPDDR2-only registers

| Register | Applicability |
| --- | --- |
| PASR_BANK_MASK_RANK0 (0x030) | LPDDR2 partial-array self-refresh (MR16); inert on DDR2 |
| PASR_SEG_MASK_RANK0 (0x034) | LPDDR2 PASR segment mask; inert on DDR2 |
| TEMP_DERATE_RANK0 (0x038) | LPDDR2 MR4 temperature class (RO); inert on DDR2 |
| TIMINGS_CL_CWL_WR.tRFCpb | LPDDR2 per-bank tRFC; unused by DDR2 |
| CTRL.pwr_req_dpd | LPDDR2 Deep Power Down request; inert on DDR2 |

## 36.3	Feature Discovery

There is no 0xFF8 capability vector in this RDL. Software identifies the build via the ID register at 0xFF0:

| Bits | Field | Meaning |
| --- | --- | --- |
| 7:0 | version | Build version (0x01) |
| 15:8 | memtype | Build memtype echo (0 DDR2 / 1 LPDDR2) |
| 23:16 | n_phases | Gear ratio (1 / 2 / 4) |
| 31:24 | module_id | Fixed 0xD2 |

Note the ID.memtype field is a build-time echo (default reflects the DDR2 build); the runtime family select is PHY_TIMING.memtype, which software programs before init.

## 36.4	Not Present in This Generation

The following belong to higher DDR/LPDDR generations and have no field in this controller’s CSR map: fine-granularity refresh (FGR), write-leveling, MPR, CA training, command-bus training (CBT), bank groups, and inline ECC. They were speculative entries in an earlier family registry and are omitted here rather than tied off.

## 36.5	Open Questions / Future Work

- Multi-rank family fields. PASR/temperature registers are declared for rank 0 only; a NUM_RANKS loop in the RDL would add *_RANK{N}.

- Capability register. If a family capability vector proves useful for a generic SoC driver, it can be re-added as an RO register; for now ID carries the essentials.

- DPD power-state path. CTRL.pwr_req_dpd is defined; the LPDDR2 DPD entry/exit datapath is a follow-up (see §5.2).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 37	Initialization Sequence

Per HAS §6.2 for the architectural sequence and §2.12 for the FUB-level detail. This chapter is the software-side cookbook — what bring-up firmware actually does to get from power-on to “DRAM ready for AXI traffic.” The sequence is driven by rtl/fub/init_sequencer.sv.

## 37.1	Pre-Reset Setup

Before aresetn (the MC-domain reset) deasserts, the SoC’s PMU should:

- Apply DRAM power rails (VDD, VDDQ; LPDDR2 also VDD1/VDD2/VDDCA)

- Wait for power rails to be stable (typically 10–100 µs per JEDEC)

- Bring the register transport out of reset (the PeakRDL cpuif is R/W-able; the init sequencer sits in S_RESET/S_DFI_INIT)

- Program the family selector and address map: PHY_TIMING.memtype, ADDR_MAP.bank_lsb/hash_* (these must be set before init — see §4.3)

- Load timing values (TIMINGS_*, INIT_TIMING0/1, MR0..MR3, PASR if LPDDR2)

- Release the datapath reset

Because config is driven by name into the core (see §4.3), the values are already live at the core boundary when init runs — there is no staging/commit step.

## 37.2	Init Trigger

void start_dram_init(void) {
    // Family + address map first (must be set before init)
    csr_write(PHY_TIMING, MEMTYPE(DDR2) | T_RDDATA_EN(6) | T_PHY_WRLAT(0) | REFRESH_BURST(1));
    csr_write(ADDR_MAP,   BANK_LSB(COL_WIDTH));   // ROW_MAJOR; add HASH_EN|HASH_SEED if wanted

    // Load timings (example values for DDR2-800)
    csr_write(TIMINGS_RC_RCD_RP_RAS,   TIMING_PACK(60, 15, 15, 40)); // tRC, tRCD, tRP, tRAS
    csr_write(TIMINGS_RFC_REFI,        REFI_PACK(200, 1950));        // tRFC, tREFI
    csr_write(TIMINGS_RRD_FAW_WTR_CCD, RRD_PACK(6, 35, 4, 4));       // tRRD, tFAW, tWTR, tCCD
    csr_write(TIMINGS_CL_CWL_WR,       CL_PACK(6, 4, 15, 70));       // CL, CWL, tWR, tRFCpb
    csr_write(TIMINGS_RTP_RTW,         RTP_PACK(4, 6));              // tRTP, tRTW

    // JEDEC init-sequence waits (or leave RDL defaults 512/256/8/8/16)
    csr_write(INIT_TIMING0, INIT_T0_PACK(512, 256));                 // tINIT, tDLLK
    csr_write(INIT_TIMING1, INIT_T1_PACK(8, 8, 16));                 // tMRD, tRP, tRFC

    // MR values: for DDR2 the sequencer uses its own JEDEC-correct MR words
    // (0x0532/0x0432/0x0380/0x0000); the MR0..MR3 CSRs are the software-visible
    // shadow. For LPDDR2 the sequencer drives the MRW OP values internally.
    csr_write(MR0, mr0_value); csr_write(MR1, mr1_value);
    csr_write(MR2, mr2_value); csr_write(MR3, mr3_value);

    // (LPDDR2) PASR if needed
    csr_write(PASR_BANK_MASK_RANK0, pasr_bank_mask);

    // Optional: ZQ retry count / timeout
    csr_write(INIT_TUNING, INIT_TUNING_PACK(3, 10));  // 3 retries, 10ms timeout

    // Trigger init
    csr_write(CTRL, CTRL_INIT_START);

    // Poll for completion
    while (1) {
        uint32_t s = csr_read(STATUS);
        if (s & STATUS_INIT_DONE)  break;             // DRAM ready
        if (s & STATUS_INIT_ERROR) {                  // STATUS.init_step_dbg shows where
            log_error("DRAM init failed at step %d", STATUS_INIT_STEP_DBG(s));
            return -EIO;
        }
    }
}

Note (this build): STATUS readback (hwif_in) is currently tied off in pumice_top (see §4.1); init_done is also exposed directly as the top-level init_done_o port. Poll whichever is wired on the platform.

## 37.3	Init Sequence Details

The step-by-step JEDEC sequence is a hardware FSM in init_sequencer.sv (r_state); software does not interleave commands during init. The sequencer both issues the commands to the DRAM (through the scheduler to dfi_cmd_formatter) and updates the mode-register shadow so the live CL/CWL/BL decode tracks what was programmed. The memtype_i input selects the DDR2 or LPDDR2 chain.

### 37.3.1	DDR2 chain (mirrors LiteDRAM’s proven Nexys A7 sequence)

- Assert dfi_init_start_o; wait dfi_init_complete_i (PHY DLL-lock / IO training), then wait tINIT (t_init_wait, CKE settle)

- Precharge All (wait tRP)

- EMRS(2) = 0, EMRS(3) = 0, EMRS(1) = 0 — JEDEC MRS order EMR2 -> EMR3 -> EMR1, each followed by tMRD

- MRS(0) + DLL reset (0x0532: BL4/CL3/tWR3/DLL_RESET); wait tDLLK (DLL lock)

- Precharge All (wait tRP)

- Auto Refresh x2 (each followed by tRFC)

- MRS(0) without DLL-reset (0x0432 — clears the reset bit); wait tMRD

- EMRS(1) + OCD Default (0x0380) -> EMRS(1) + OCD Exit (0x0000)

- init_done_o = 1

### 37.3.2	LPDDR2 chain (JESD209-2F power-up + MR configuration)

- Assert dfi_init_start_o; wait dfi_init_complete_i; tINIT settle

- MRW(MR63) = Reset (wait tINIT4, reuses t_init_wait)

- MRW(MR10) = 0xFF ZQ Init Calibration (wait tZQINIT, reuses t_dll_wait)

- MRW(MR1) = BL8/nWR3 (0x23), MRW(MR2) = RL3/WL1 (0x01), MRW(MR3) = DS 40ohm (0x02) — each followed by tMRW (reuses t_mrd_wait)

- init_done_o = 1

The MR index (MA, up to MR63) and data (OP) are packed as {MA[5:0], OP[7:0]} into the row request field; dfi_cmd_formatter unpacks it for the LPDDR2 CA-bus MRW word (a 3-bit bank port cannot reach MR10/MR63). Only MR1/MR2/MR3 update the CL/CWL/BL decode shadow; MR63/MR10 are issued but not shadowed.

The init waits are the INIT_TIMING0/1 CSR fields (t_init_wait, t_dll_wait, t_mrd_wait, t_rp_wait, t_rfc_wait); RDL defaults (512/256/8/8/16 MC cycles) comfortably cover the JEDEC minimums at the board frequency. For fast sim, program small values before triggering init.

## 37.4	Multi-Rank Init Differences

The current init_sequencer.sv targets a single rank (the board build is NUM_RANKS = 1). For a NUM_RANKS > 1 build the intended extension is:

- MRS/MRW steps iterate over ranks (one rank per step iteration)

- ZQ calibration is per-rank (each rank has its own reference)

- Each rank’s PASR mask is programmed via its own PASR_*_RANK{N} register (see §4.2 open items)

Software programs the MR values once; per-rank iteration is absorbed by the sequencer. Per-rank register generation is a follow-up (the RDL declares only *_RANK0 today).

## 37.5	ZQ Retry

The INIT_TUNING.zq_retries field (default 3) is exposed for the retry count; DDR2 uses OCD calibration during init (no ZQCL) and LPDDR2 uses the MR10 ZQ Init step. Raise the retry budget if signal-integrity issues cause occasional failures.

## 37.6	Post-Init Power-State

After init_done, the controller is in ACTIVE for all ranks with CKE high. The refresh manager starts its tREFI counter and begins normal refresh scheduling. The scheduler is unblocked and any AXI bursts that accumulated during init begin issuing.

## 37.7	Software Wait Times

| Phase | Wait (real silicon, 200 MHz MC clock) |
| --- | --- |
| Power-up settling | 200 µs |
| MRS loads + per-rank iteration | ~10 cycles × NR × 4 MRs ≈ 160 cycles |
| ZQ calibration | ~1 µs |
| Initial refresh batches | ~10 µs |
| Total init duration | ~250 µs (single-rank), ~280 µs (4-rank) |

The init duration scales weakly with rank count — the per-rank serialization is dwarfed by the fixed settling delays.

## 37.8	Reset Recovery

If init fails (STATUS.init_error = 1):

void recover_init_failure(void) {
    uint8_t step = STATUS_INIT_STEP_DBG(csr_read(STATUS));
    log_error("Init failed at step %d", step);

    // Inspect step-specific telemetry (per step table)
    // ...

    // Force re-init
    csr_write(CTRL, CTRL_INIT_FORCE_RESTART);

    // Wait a few cycles for FSM to settle
    for (int i = 0; i < 10; i++) asm volatile("nop");

    // Re-init
    start_dram_init();
}

A hard fault (e.g., persistent ZQ calibration failure) requires SoC-level intervention — typically a power-cycle and re-bring-up.

## 37.9	Open Questions / Future Work

- Init telemetry capture. When init fails, telemetry beyond init_step_dbg would help (DFI signal trace, per-rank fault detection). The current debug surface is sparse; expand if bring-up calls it out.

- Init via JTAG. Some platforms init via a JTAG side-band rather than CPU+APB. The CSR map supports this through any APB-attached transport; no controller changes needed.

- Partial init for warm reset. Coming out of self-refresh doesn’t require full init. The current init engine doesn’t have a “warm reset” path. Add when DDR3+ needs it for frequency change.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 38	Refresh and Power-State Programming

Per HAS §3.4 / §3.5 for architecture and §2.11 for FUB detail. This chapter is the software-side view of refresh and power configuration. Register writes are the PeakRDL cpuif (csr_write/csr_read); there is no apply/commit handshake — fields drive the core live (see §4.3).

## 38.1	Tuning Refresh Behavior

void configure_refresh(refresh_config_t* cfg) {
    // Base tREFI / tRFC (also set during init; retunable at runtime)
    csr_write(TIMINGS_RFC_REFI, REFI_PACK(cfg->trfc, cfg->trefi));

    // REFRESH_TUNING packs deferral count, page/refpb policy, ZQCS interval.
    uint32_t v = REFRESH_DEFER_ACTIVE(cfg->defer_active);   // 1..8

    // REFpb policy override (LPDDR2 only): 01 RR, 10 OLDEST_FIRST, 11 DARP
    if (cfg->refpb_policy == DARP)             v |= REFPB_POLICY_OR(3);
    else if (cfg->refpb_policy == OLDEST)      v |= REFPB_POLICY_OR(2);
    else if (cfg->refpb_policy == ROUND_ROBIN) v |= REFPB_POLICY_OR(1);

    // Periodic ZQCS interval in Hz (0 = disable)
    v |= ZQCS_FREQ_HZ(cfg->zqcs_freq_hz);

    csr_write(REFRESH_TUNING, v);   // live on the next refresh event boundary
}

PHY_TIMING.refresh_burst (1..8) additionally controls how many REFs are drained per refresh request.

### 38.1.1	When to Tune

| Workload | Recommended config |
| --- | --- |
| Streaming (DMA, video) | defer_active = 8, ZQCS = 1 Hz |
| Low-latency bursty (CPU access) | defer_active = 1 (no batching), ZQCS = 0.1 Hz |
| Power-sensitive (sleep-heavy) | defer_active = 4, ZQCS = 0.1 Hz |
| Real-time / safety-critical | defer_active = 1, deterministic refresh latency |

## 38.2	LPDDR2 PASR (Partial Array Self-Refresh)

PASR masks DRAM regions that hold no data, reducing refresh power.

void set_pasr_rank0(uint8_t bank_mask, uint8_t seg_mask) {
    csr_write(PASR_BANK_MASK_RANK0, bank_mask);
    csr_write(PASR_SEG_MASK_RANK0,  seg_mask);
    // Propagated to DRAM via MR16/MR17 at the next self-refresh entry.
}

The single-rank build exposes PASR_BANK_MASK_RANK0 / PASR_SEG_MASK_RANK0; multi-rank *_RANK{N} registers are a follow-up (see §4.2). The PASR mask is propagated lazily — typically at the next self-refresh entry — to avoid an extra bus-blocking MR write during normal operation.

## 38.3	Temperature Compensation (LPDDR2)

LPDDR2 devices expose temperature classification in MR4. The SoC reads this via PHY-side mechanisms (out of scope for the controller; the PHY signals temperature change via interrupt or polled register).

Software updates the controller’s tREFI scaling:

TEMP_DERATE_RANK0.temp_class (0 = nominal, 1 = 2x refresh, 2 = 4x refresh) is a read-only, hardware-written field in this build — the controller captures the LPDDR2 MR4 class rather than software programming it. Software reads it to inform its own tREFI scaling via TIMINGS_RFC_REFI:

uint8_t temp = csr_read(TEMP_DERATE_RANK0) & 0x3;
// Scale tREFI down for 2x/4x refresh as temp rises
csr_write(TIMINGS_RFC_REFI, REFI_PACK(trfc, trefi >> temp));  // live next reload

## 38.4	Self-Refresh Entry

For periods of inactivity, software (or auto-detection) can put the DRAM into self-refresh:

The power-state request bits live in CTRL: pwr_req_low_power, pwr_req_self_refresh, pwr_req_active, pwr_req_dpd. STATUS.power_state[7:4] reports the current encoded state (RO, hw-written).

void enter_self_refresh(void) {
    csr_write(CTRL, CTRL_PWR_REQ_SELF_REFRESH);
    while (STATUS_POWER_STATE(csr_read(STATUS)) != POWER_STATE_SELF_REFRESH);
}

void exit_self_refresh(void) {
    csr_write(CTRL, CTRL_PWR_REQ_ACTIVE);
    while (STATUS_POWER_STATE(csr_read(STATUS)) != POWER_STATE_ACTIVE);
}

Note: any AXI traffic to a rank will auto-wake it from SR. Explicit exit is only needed when the application wants to be ready before issuing.

There is no POWER_TUNING register in this build — idle-threshold auto-APD/SRF is not a CSR knob here. Power-state entry is software-requested via CTRL. (An auto-low-power threshold register is a possible follow-up; see below.)

## 38.5	DPD (LPDDR2 Deep Power Down)

DPD is the deepest power state on LPDDR2 — DRAM is fully off; software must full-init to exit. The request bit is CTRL.pwr_req_dpd (inert on DDR2). Check the build memtype via ID (0xFF0) rather than a capability vector (there is no 0xFF8):

void enter_dpd(void) {
    if (((csr_read(ID) >> 8) & 0xFF) != MEMTYPE_LPDDR2) {
        log_error("DPD is LPDDR2-only");
        return;
    }
    csr_write(CTRL, CTRL_PWR_REQ_DPD);
    while (STATUS_POWER_STATE(csr_read(STATUS)) != POWER_STATE_DPD);
}

void exit_dpd(void) {
    start_dram_init();   // full re-init
}

DPD is rare in practice (DRAM fully unused for hours). Note the runtime family select is PHY_TIMING.memtype; ID.memtype is the build-time echo.

## 38.6	Open Questions / Future Work

- Auto power-down thresholds. This build has no POWER_TUNING register — APD/SRF is request-driven. Adding idle-threshold CSRs for automatic low-power entry is a candidate feature.

- Per-rank power control. CTRL.pwr_req_* is channel-wide. Per-rank request registers are a multi-rank follow-up.

- Observation readback. STATUS.power_state, STATUS_HISTORY, and the OBS_REFRESH_DEFER_HIST_* telemetry are declared but hwif_in is tied off in pumice_top today (see §4.1); wiring them enables telemetry-driven auto-tuning.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 39	Multi-Rank Programming

Per HAS §2.1 (multi-rank as differentiator), HAS §3.3 / §3.4 / §3.6 (architecture). FUB-level detail in MAS §2 (pumice_bank_timers per (rank,bank), refresh_ctrl round-robin, dfi_cmd_formatter CS_n / ODT).

Note: the default board build is single-rank (NUM_RANKS = 1). This chapter documents the intended multi-rank programming model; the single-rank RDL declares only *_RANK0 registers, and per-rank register generation is a follow-up (see §4.2).

## 39.1	Discovery

There is no capability vector register in this build. Software knows NUM_RANKS from the build parameter (it is a synthesis-time geometry parameter, see §6.1); the ID register (0xFF0) reports memtype and phase count but not rank count. Treat the build parameter as authoritative.

## 39.2	Per-Rank Mode Register Loads

During init, the sequencer (intended multi-rank extension) iterates the MRS/MRW loads across ranks. Software writes MR0..MR3 once:

csr_write(MR0, mr0_value);   // applied to all ranks during init
csr_write(MR1, mr1_value);
csr_write(MR2, mr2_value);
csr_write(MR3, mr3_value);

For DDR2 the sequencer uses its own JEDEC-correct MR words; the MR* CSRs are the software-visible shadow (see §5.1). Per-rank MR variation (rare) is a future feature.

## 39.3	Per-Rank PASR (LPDDR2)

Already covered in §5.2. In the single-rank build:

csr_write(PASR_BANK_MASK_RANK0, mask);

A multi-rank build adds PASR_BANK_MASK_RANK{N} registers (each rank has independent MR16/MR17).

## 39.4	ODT

ODT is handled inside dfi_cmd_formatter / mode_register (there is no standalone ODT control block and no RANK_TUNING.odt_rule_or CSR). dfi_odt_o is driven per command. Board-specific ODT rule selection is not a runtime CSR knob in this build; it is baked into the formatter’s per-command ODT logic. For boards with non-standard impedance budgets, adjust the formatter’s ODT policy at build time.

## 39.5	Per-Rank Disable

There is no RANK_TUNING rank-enable CSR in this build. Rank enable/disable is a multi-rank feature to be added alongside per-rank register generation. In the single-rank build there is one rank and it is always active.

## 39.6	Address Mapping for Multi-Rank

The rank field always sits in the high bits of the (byte-offset-stripped) word address, above the row — its position is invariant. The single ADDR_MAP.bank_lsb knob only slides the bank field within the column region (see §4.4 and rtl/fub/addr_mapper.sv); the rank field is unaffected by bank_lsb. The hash_en XOR-hash folds row bits into the bank index only — it does not touch the rank field.

There is no rank-interleave mode. Software-managed rank interleaving (the OS striping allocations across rank-aligned regions) is the workaround if consecutive-line rank striping is desired.

## 39.7	Refresh Behavior with Multi-Rank

Per HAS §3.4 and MAS §2 (refresh_ctrl, intended multi-rank extension):

- REFab dispatches per-rank in round-robin (not all-rank simultaneously)

- REFpb (LPDDR2) selects (rank, bank) tuples via the REFRESH_TUNING.refpb_policy_or policy

- Per-rank PASR masks are honored independently

A multi-rank system distributes refresh across the timeline so any single rank only blocks for tRFC per refresh — non-target ranks keep operating.

## 39.8	Power State Per-Rank

- Channel-wide CSR CTRL.pwr_req_* applies to all ranks (there is no per-rank power request CSR)

- STATUS.power_state[7:4] reports the encoded state

- Per-rank auto-low-power is not a CSR feature in this build (there is no POWER_TUNING; see §5.2)

## 39.9	Per-Rank / Per-Bank Observation

The RDL declares per-bank observation arrays for rank 0: OBS_ROW_HIT[8] (0x080..0x09C) and OBS_REF_LATENCY[8] (0x0C0..0x0DC). The generated regmap flattens them to indexed names:

uint32_t get_row_hit(uint8_t bank) {
    return csr_read(OBS_ROW_HIT0_ROW_HIT + bank * 4);   // read-clear
}

A multi-rank build adds the per-rank arrays. (Note: hwif_in observation readback is tied off in pumice_top today — see §4.1.)

## 39.10	Multi-Rank Bring-Up Checklist

| Step | Why |
| --- | --- |
| Verify the build’s NUM_RANKS matches the expected DIMM rank count | Build / board mismatch |
| Program PASR_*_RANK{N} if using LPDDR2 | PASR for LPDDR2 bring-up |
| Verify per-rank ZQ calibration succeeds during init | Each rank’s drive impedance |
| Sweep OBS_REF_LATENCY[bank] across rank workload mix | Per-rank refresh fairness |
| Stress-test rank-switching (read alternating ranks) | tRTRS / tCS timing |

## 39.11	Open Questions / Future Work

- Per-rank register generation. The RDL declares only *_RANK0; a NUM_RANKS loop is needed for PASR/temp/observation and rank-enable.

- Per-rank MR override. Some DIMMs have rank-specific tuning (rare).

- Per-rank power control. CTRL.pwr_req_* is channel-wide; per-rank request registers are a follow-up.

- Runtime ODT rule select. ODT is currently baked into dfi_cmd_formatter; a runtime CSR knob could be reintroduced if board diversity needs it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 40	Error Handling

The controller’s error-reporting paths and the recommended software response model. Register accesses are the PeakRDL cpuif (csr_write/csr_read); config fields drive the core live with no apply/commit step (see §4.3).

## 40.1	Error Categories

| Category | Detection | Reporting |
| --- | --- | --- |
| Init failure | init_sequencer timeout / ZQ retries exhausted | STATUS.init_error = 1; STATUS.init_step_dbg shows where |
| DFI rddata-valid timeout | RD issued, no rddata in the expected window | SLVERR on the AXI R (bring-up: check DFI phase / t_rddata_en) |
| AXI burst boundary violation | Per §3.1 (one AXI burst == one DRAM burst) | SLVERR on B/R |
| ZQ calibration failure (silicon) | DFI signal-integrity, caught at PHY layer | Not this controller |
| Multi-bit DRAM error | Out of scope (no inline ECC) | Would need ECC sideband |

Note: STATUS is currently a declared RO register whose hwif_in is tied off in pumice_top (see §4.1). init_done is exposed directly as the top-level init_done_o port. There is no dedicated IRQ port list wired in this build; software polls STATUS/init_done_o. A queue-overflow / refresh-miss IRQ is a possible follow-up.

## 40.2	Software Response Patterns

### 40.2.1	Init Failure

void handle_init_error(void) {
    uint32_t s = csr_read(STATUS);
    uint8_t step = STATUS_INIT_STEP_DBG(s);

    log_error("DRAM init failed at step %d", step);

    // init_sequencer step encodings (r_state), for reference:
    //   S_DFI_INIT  -> PHY init did not complete (check PHY config)
    //   S_EMR*/S_MR0* / S_L_* -> MRS/MRW issue phase (verify MR values, CA-bus)
    //   S_REF1/S_REF2 -> refresh phase
    // Force re-init (config CSRs are preserved)
    csr_write(CTRL, CTRL_INIT_FORCE_RESTART);
}

### 40.2.2	DFI Read-Data Timeout (board bring-up)

A read that returns no data almost always means the DFI read window is misaligned for the attached PHY. The knobs are DFI_PHASE.rd_phase, PHY_TIMING.t_rddata_en, and PHY_TIMING.t_phy_wrlat:

// Nexys A7 a7ddrphy known-good: rd_phase=0, t_rddata_en=6, t_phy_wrlat=0
csr_write(DFI_PHASE,   RD_PHASE(0) | WR_PHASE(0));
csr_write(PHY_TIMING,  MEMTYPE(DDR2) | T_RDDATA_EN(6) | T_PHY_WRLAT(0) | REFRESH_BURST(1));

Set these during bring-up before triggering init; they take effect immediately (config-drive, no commit step).

### 40.2.3	Refresh Pressure

Watch OBS_REFRESH_PENDING_MAX (0x108). If it approaches the deferral budget, reduce batching:

uint32_t v = csr_read(REFRESH_TUNING);
v = (v & ~REFRESH_DEFER_ACTIVE_MASK) | REFRESH_DEFER_ACTIVE(1);  // no batching
csr_write(REFRESH_TUNING, v);   // live on the next refresh event boundary

## 40.3	STATUS_HISTORY for Bring-Up

The STATUS_HISTORY register (0x008) captures the last 8 power-state transitions (4 bits each, most recent in [3:0]). Useful when the controller oscillates between power states:

void dump_state_history(void) {
    uint32_t hist = csr_read(STATUS_HISTORY);
    for (int i = 0; i < 8; i++)
        log_info("State[-%d]: %s", i, power_state_name((hist >> (i*4)) & 0xF));
}

(As with STATUS, this readback depends on hwif_in being wired — a follow-up per §4.1.)

## 40.4	Diagnostic Sequence for Suspected Bug

1. Read STATUS.init_done (or the init_done_o port) — must be 1
2. Read STATUS.power_state — should be ACTIVE
3. Read STATUS_HISTORY — look for oscillation
4. Read OBS_TXN_QUEUE_DEPTH_MAX / OBS_REFRESH_PENDING_MAX — check for backlog
5. Read OBS_AXI_R_LATENCY_P99 — tail latency telemetry
6. Read OBS_ROW_HIT[bank] — per-bank traffic distribution
7. Read OBS_REF_LATENCY[bank] — per-bank refresh fairness

This is the bring-up team’s first-pass diagnostic flow.

## 40.5	Soft Reset vs. Re-Init

CTRL.soft_reset (bit 31, self-clearing) requests an internal soft reset that re-clears datapath state without disturbing the CSR contents. Use it when the controller is in an inconsistent state but the software’s config is correct:

void soft_reset(void) {
    csr_write(CTRL, CTRL_SOFT_RESET);
    for (int i = 0; i < 16; i++) asm volatile("nop");  // self-clearing
    csr_write(CTRL, CTRL_INIT_START);                  // CSRs preserved
}

For hard cases (silicon bug, persistent failure), the SoC PMU drives the asynchronous reset and software re-brings-up from scratch.

## 40.6	Open Questions / Future Work

- IRQ port. This build polls STATUS/init_done_o. A latched error/IRQ output (init error, refresh miss, rddata timeout) is a candidate feature.

- Observation readback wiring. The STATUS/OBS_* diagnostics assume hwif_in is connected; it is tied off today (§4.1).

- Per-rank error reporting. When multi-rank lands, localizing a fault to a rank would help board bring-up.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 41	Build-Time Configuration Reference

Per HAS §5.2 for the parameter table and §5.1 for the build-vs-runtime philosophy. This chapter is the build-script-author’s view: the synthesis-time parameters and how the data-width geometry works. Most behavior is runtime CSR-driven (see §4/§5), so the build parameter set is deliberately small — mostly memory geometry.

## 41.1	Parameters

The synthesis-time parameters are the ports of pumice_top (rtl/top/pumice_top.sv). They are geometry, not policy — memtype / timings / page policy / address map are all runtime CSR fields.

pumice_top #(
    .AXI_ID_WIDTH    (8),
    .AXI_ADDR_WIDTH  (32),
    .NUM_RANKS       (1),      // board build is single-rank
    .NUM_BANKS       (8),
    .ROW_WIDTH       (14),
    .COL_WIDTH       (10),
    .DFI_RATE        (2),      // DFI phases per MC clock (gear ratio)
    .DRAM_BEAT_WIDTH (64),     // bits per DRAM beat (device data bus)
    .BL              (8),      // DRAM burst length
    .NUM_ENTRIES     (8),      // CAM depth
    .N_SRAM_SLOTS    (8)
    // DW is DERIVED: DW = DRAM_BEAT_WIDTH * DFI_RATE
) u_pumice ( ... );

There is no MEMTYPE, PAGE_POLICY, SCHEDULER_MODE, ODT_RULE_*, ADDR_MAP_SCHEMES_SYNTH, or DFI_ADDR_WIDTH parameter — those are runtime CSR fields or derived from geometry. memtype is PHY_TIMING.memtype; page policy is REFRESH_TUNING.page_policy_or; address mapping is ADDR_MAP.bank_lsb.

## 41.2	Data-Width Geometry (DW = DRAM_BEAT_WIDTH x DFI_RATE)

The controller’s internal / AXI-facing data width is derived, not a free parameter:

DW = DRAM_BEAT_WIDTH * DFI_RATE          // e.g. 64 * 2 = 128 (default)

- One AXI beat == one DFI word == DFI_RATE DRAM beats.

- One AXI burst (BL / DFI_RATE beats) == one DRAM burst (BL beats).

- The DW -> per-phase split happens in dfi_signal_pack / the DFI layer (DFI_DATA_WIDTH = DW, DFI_EN_WIDTH = DFI_RATE).

The old “AXI_DATA_WIDTH == DRAM_BEAT_WIDTH” coupling is gone — do not describe it. The host AXI data width the core presents is exactly DW.

### 41.2.1	Host-Width Freedom (pumice_top_geared)

To let a host SoC use a convenient fixed AXI width regardless of the DRAM point, wrap pumice_top in pumice_top_geared (rtl/top/pumice_top_geared.sv), which adds a free HOST_AXI_DATA_WIDTH parameter:

- It inserts the repo’s formally-verified axi4_dwidth_converter_wr / _rd between a host-width AXI slave and the fixed-DW core.

- HOST_AXI_DATA_WIDTH == DW is a generate bypass — bit-identical to bare pumice_top, no converter, no added latency.

- Verified host widths: 64, 128, 256.

- Burst geometry contract is unchanged at the core’s DW side; the converter re-sizes host bursts to DW-width bursts.

Doc: docs/AXI_DRAM_GEARING_SCOPE.md.

## 41.3	Parameter Sanity

The natural elaboration-time constraints:

generate
    if (NUM_RANKS != 1 && NUM_RANKS != 2 && NUM_RANKS != 4)
        $error("NUM_RANKS must be 1, 2, or 4");
    if (NUM_BANKS != 4 && NUM_BANKS != 8)
        $error("NUM_BANKS must be 4 or 8");
    if (DFI_RATE != 1 && DFI_RATE != 2 && DFI_RATE != 4)
        $error("DFI_RATE must be 1, 2, or 4 (gear ratio)");
    // BL must be an integer multiple of DFI_RATE so a DRAM burst is a whole
    // number of AXI beats.
endgenerate

DFI_RATE must equal the attached PHY’s phase count (e.g., the on-board 300 MT/s point is DFI_RATE = 4 at sys = 37.5 MHz / CK = 150 MHz; regenerating for a different phase count alone produces a broken hybrid — see project notes).

## 41.4	Filelists

FUB filelists live in rtl/filelists/. The live module hierarchy (top -> core -> pumice_axi4_ifc / pumice_mem_cmd_scheduler / pumice_dfi_layer) is fixed; the DDR2 vs LPDDR2 command paths coexist in dfi_cmd_formatter and branch on memtype, so a single build supports both families with dead-code elimination handling the unused branch. page_predictor.sv / powerdown_ctrl.sv are optional/not in the default top build (confirm via the filelists).

## 41.5	Open Questions / Future Work

- Multi-rank build. NUM_RANKS > 1 requires the per-rank register generation and per-rank init iteration described in §4.2 / §5.3.

- Curated small-area filelist. Excluding the optional predictor/power-down FUBs shrinks the smallest build.

- Geared-top width matrix. Extending the verified HOST_AXI_DATA_WIDTH set beyond {64,128,256} as needed.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 42	Runtime Configuration Reference

Per §4.2 for the full CSR map and §4.3 for the config-drive model (fields drive the core live; no apply/commit step). This chapter is the driver author’s cookbook for runtime tunables: what to tune, when, and what to watch for. Register access is the PeakRDL cpuif (csr_write/csr_read).

## 42.1	Bring-Up Configuration Order

When bring-up software first comes up, recommended order (address map and memtype are set before init — see §5.1):

- Family + address map (pre-init) — PHY_TIMING.memtype; ADDR_MAP.bank_lsb / .hash_en / .hash_seed.

- Set page policy — REFRESH_TUNING.page_policy_or (00 build-time, 01 OPEN, 10 CLOSE, 11 HAPPY_HYBRID).

- Set lookahead depth — SCHED_TUNING.lookahead_active (0 disables).

- Set refresh deferral — REFRESH_TUNING.refresh_defer_active; PHY_TIMING.refresh_burst.

- Set ZQCS frequency — REFRESH_TUNING.zqcs_freq_hz (1 Hz default; 0 to disable).

Each write is live immediately at the core boundary; there is no config_apply and no quiet-point drain. Quiesce AXI traffic before changing a field that would corrupt in-flight state (see §4.3).

## 42.2	Address-Map Tuning (single knob)

Address mapping is ADDR_MAP.bank_lsb alone (plus the optional XOR-hash). There is no scheme selector, scheme_or, or xor_seed_runtime:

// ROW_MAJOR: bank field above the whole column
csr_write(ADDR_MAP, BANK_LSB(COL_WIDTH));

// Max BANK_INTERLEAVE: bank field just above the burst's low column bits
csr_write(ADDR_MAP, BANK_LSB(log2_cols_per_burst));

// XOR_HASH folded on top of any placement
csr_write(ADDR_MAP, BANK_LSB(COL_WIDTH) | HASH_EN | HASH_SEED(seed));

RTL clamps bank_lsb to [0, COL_WIDTH]; keep log2(BL/DFI_RATE) <= bank_lsb <= COL_WIDTH so a DRAM burst stays inside one bank (see §4.4 and rtl/fub/addr_mapper.sv). Change address mapping only before init or with the datapath idle.

## 42.3	Characterization Sweep Order

| Sweep order | Knob | Why |
| --- | --- | --- |
| 1 | ADDR_MAP.bank_lsb | Largest impact on row-hit / bank parallelism |
| 2 | ADDR_MAP.hash_en / .hash_seed | Defeat power-of-two-stride hot-banking |
| 3 | REFRESH_TUNING.page_policy_or | OPEN vs CLOSE vs HAPPY for the workload mix |
| 4 | SCHED_TUNING.lookahead_active | Issue rate vs lookahead depth |
| 5 | SCHED_TUNING.happy_enable | A/B test the predictor (HAPPY only) |
| 6 | REFRESH_TUNING.refresh_defer_active | Refresh latency vs sustained BW |
| 7 | SCHED_TUNING.age_max_runtime | Anti-starvation tuning |
| 8 | SCHED_TUNING.txn_queue_high_water | Backpressure timing |

## 42.4	Telemetry to Watch

Observation registers per §4.2 (RO; note hwif_in readback is tied off in pumice_top today — see §4.1):

| Telemetry register | What it tells you | Tune action |
| --- | --- | --- |
| OBS_AXI_R_LATENCY_AVG / _P99 | AXI read latency (avg / tail) | scheduler / lookahead / page policy / age_max |
| OBS_AXI_W_LATENCY_AVG | AXI write latency | write-path / CWL alignment |
| OBS_ROW_HIT[bank] | Per-bank row-hit rate (read-clear) | address mapping (bank_lsb/hash), page policy |
| OBS_REF_LATENCY[bank] | Per-bank refresh blocking | refresh deferral / refpb policy |
| OBS_TXN_QUEUE_DEPTH_MAX/AVG | Queue pressure | txn_queue_high_water |
| OBS_REFRESH_PENDING_MAX | Proximity to refresh-deadline miss | lower refresh_defer_active |
| OBS_REFRESH_DEFER_HIST_0..3 | Refresh batch histogram | validate refresh_defer_active |
| OBS_PAGE_PRED_ACCURACY | HAPPY prediction accuracy | warmup_cycles / hysteresis |
| OBS_WORDS[9] | Packed obs_* harvest | FUB-internal diagnostics |

## 42.5	Workload-Specific Recipes

### 42.5.1	Streaming (DMA, video, audio capture)

// Maximize row-hit, batch refresh, interleave banks
csr_write(SCHED_TUNING,   LOOKAHEAD_ACTIVE(4) | HAPPY_ENABLE);
csr_write(REFRESH_TUNING, REFRESH_DEFER_ACTIVE(8) | PAGE_POLICY_OR(1 /*OPEN*/) | ZQCS_FREQ_HZ(1));
csr_write(ADDR_MAP,       BANK_LSB(log2_cols_per_burst));   // bank-interleave (pre-init)

### 42.5.2	Low-Latency Bursty (CPU)

csr_write(SCHED_TUNING,   LOOKAHEAD_ACTIVE(2) | HAPPY_ENABLE);
csr_write(REFRESH_TUNING, REFRESH_DEFER_ACTIVE(1) | PAGE_POLICY_OR(3 /*HAPPY*/));
csr_write(ADDR_MAP,       BANK_LSB(COL_WIDTH) | HASH_EN | HASH_SEED(seed));  // pre-init

### 42.5.3	Real-Time / Safety-Critical

csr_write(SCHED_TUNING,   FORCE_INORDER | LOOKAHEAD_ACTIVE(0));
csr_write(REFRESH_TUNING, REFRESH_DEFER_ACTIVE(1) | PAGE_POLICY_OR(2 /*CLOSE*/));

## 42.6	Telemetry-Driven Auto-Tuning Loop

For SoCs with firmware capable of background loops (once observation readback is wired):

void periodic_autotune(void) {
    static uint8_t defer = 8;
    uint32_t pending_max = csr_read(OBS_REFRESH_PENDING_MAX);

    if (pending_max > DEFER_BUDGET * 7 / 8)      { if (defer > 1) defer--; }
    else if (pending_max < DEFER_BUDGET / 4)     { if (defer < 8) defer++; }

    uint32_t v = csr_read(REFRESH_TUNING);
    v = (v & ~REFRESH_DEFER_ACTIVE_MASK) | REFRESH_DEFER_ACTIVE(defer);
    csr_write(REFRESH_TUNING, v);   // live on the next refresh event boundary
}

Run every ~100 ms. Writes are cheap (no commit drain) and workload-dependent tuning is automatic.

## 42.7	Open Questions / Future Work

- Observation readback. The telemetry recipes assume hwif_in is wired; it is tied off today (§4.1).

- Profile-select CSR. A single “workload profile” field that switches all knobs at once would simplify firmware; adds CSR area. Punt.

- QoS priority. awqos/arqos are on the AXI port but not yet consumed by the scheduler; a v2 hook.