Table of Contents

# 1	Rapids Beats Has Index

Generated: 2026-04-17

# 2	Document Information

Document Title: RAPIDS Beats Hardware Architecture Specification (HAS) Document Number: RAPIDS-HAS-001 Version: 1.0 Date: 2026-01-17 Status: Released

## 2.1	Purpose

This Hardware Architecture Specification (HAS) defines the external interfaces, system integration requirements, and high-level architecture of the RAPIDS Beats accelerator. It is intended for system architects and integration engineers who need to understand RAPIDS Beats from an external perspective without requiring knowledge of internal implementation details.

## 2.2	Scope

This document covers:

- External interface specifications (AXI4, AXIS, APB, MonBus)

- System-level block diagrams and data flows

- Programming model and descriptor format

- Use cases and operational sequences

- Performance characteristics and constraints

This document does NOT cover:

- Internal module architecture (see MAS)

- RTL implementation details (see MAS)

- Verification test plans

- Physical design constraints

## 2.3	Audience

Target Audience

| Role | Relevance |
| --- | --- |
| System Architect | Primary - system integration |
| Hardware Integration Engineer | Primary - interface connection |
| Software Engineer | Primary - driver development |
| Verification Engineer | Reference - interface protocols |
| RTL Designer | Reference - external constraints |

## 2.4	Document Conventions

### 2.4.1	Requirement Levels

| Term | Meaning |
| --- | --- |
| SHALL | Mandatory requirement |
| SHOULD | Recommended but not mandatory |
| MAY | Optional feature |

### 2.4.2	Signal Directions

- Input: Signal driven by external logic into RAPIDS

- Output: Signal driven by RAPIDS to external logic

- Bidirectional: Signal can be driven by either side (rare)

### 2.4.3	Timing Diagrams

All timing diagrams use WaveDrom format with the following conventions:

- p = Positive clock edge

- n = Negative clock edge

- 0/1 = Logic low/high

- x = Unknown/don’t care

- = = Data value (with label)

- . = Previous value continues

## 2.5	References

Reference Documents

| Document | Description |
| --- | --- |
| RAPIDS Beats MAS | Module Architecture Specification |
| ARM AMBA AXI4 Specification | AXI4 protocol reference |
| ARM AMBA AXI-Stream Specification | AXIS protocol reference |
| RAPIDS PRD | Product Requirements Document |

# 3	Revision History

Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-17 | RTL Design Sherpa | Initial release |

## 3.1	Change Summary

### 3.1.1	Version 1.0 (2026-01-17)

Initial Release

- Complete HAS for RAPIDS Beats Phase 1 architecture

- External interface specifications (AXI4, AXIS, APB, MonBus)

- System block diagrams with Mermaid

- Timing diagrams with WaveDrom

- Programming model and descriptor format

- Use case documentation

# 4	Acronyms and Definitions

## 4.1	Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface |
| AXIS | AXI-Stream |
| APB | Advanced Peripheral Bus |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| MAS | Module Architecture Specification |
| MonBus | Monitor Bus |
| RAPIDS | Rapid AXI Programmable In-band Descriptor System |
| SRAM | Static Random Access Memory |

## 4.2	Definitions

Term Definitions

| Term | Definition |
| --- | --- |
| Beat | Single data transfer unit (512 bits = 64 bytes in default configuration) |
| Channel | Independent DMA context with dedicated descriptor processing |
| Descriptor | 256-bit control structure defining a single DMA transfer |
| Descriptor Chain | Linked list of descriptors for multi-transfer operations |
| Sink Path | Data flow from network (AXIS slave) to memory (AXI write) |
| Source Path | Data flow from memory (AXI read) to network (AXIS master) |
| Fill | Operation writing data into SRAM buffer (sink path input) |
| Drain | Operation reading data from SRAM buffer (source path output) |

## 4.3	Signal Naming Conventions

Signal Naming Conventions

| Prefix/Suffix | Meaning |
| --- | --- |
| m_axi_* | AXI Master signals |
| s_axis_* | AXIS Slave signals |
| m_axis_* | AXIS Master signals |
| cfg_* | Configuration signals |
| *_valid | Handshake valid signal |
| *_ready | Handshake ready signal |
| *_n | Active-low signal |

# 5	Product Overview

## 5.1	Introduction

RAPIDS Beats is a high-performance network-to-memory accelerator designed for descriptor-based data movement operations. It bridges network interfaces (AXI-Stream) with system memory (AXI4), enabling efficient DMA transfers without CPU intervention.

The “Beats” architecture represents Phase 1 of the RAPIDS family, optimized for simplicity and throughput in streaming applications.

## 5.2	System Context

System Context Diagram

System Context Diagram

Source: 01_system_context.mmd

Diagram 1

## 5.3	Target Applications

Target Applications

| Application | Description | Key Requirements |
| --- | --- | --- |
| Network Offload | Receive packets from network, store to memory | Low latency, high throughput |
| Scatter-Gather DMA | Multi-descriptor data movement | Descriptor chaining |
| Streaming Accelerator | Front-end for compute engines | Continuous data flow |
| Protocol Processing | Header/payload separation | Multi-channel isolation |

## 5.4	Architecture Highlights

### 5.4.1	Phase 1 “Beats” Simplifications

The Beats architecture prioritizes simplicity over feature completeness:

Phase 1 Simplifications

| Feature | Beats (Phase 1) | Future Phases |
| --- | --- | --- |
| Transfer Unit | Full beats only | Chunk-level alignment |
| Flow Control | Backpressure-based | Credit management |
| Address Alignment | Pre-aligned required | Hardware alignment fixup |
| Network Interface | Direct AXIS | Protocol conversion |

### 5.4.2	Key Capabilities

- 8 Independent Channels - Concurrent transfers with dedicated resources

- Descriptor Chaining - Automatic next-descriptor fetch

- Dual Data Paths - Simultaneous sink (write) and source (read)

- Monitor Integration - Real-time event reporting via MonBus

- Error Detection - Address range checking, AXI error propagation

# 6	Key Features

## 6.1	Feature Summary

### 6.1.1	Multi-Channel Architecture

- 8 Independent Channels with dedicated schedulers

- Per-Channel State Machines for concurrent operation

- Shared AXI Infrastructure with arbitrated access

- Channel Isolation prevents cross-channel interference

### 6.1.2	Descriptor-Based Control

- 256-bit Descriptor Format with all transfer parameters

- Automatic Chaining follows next_descriptor_ptr links

- Last Descriptor Flag terminates chain processing

- IRQ Generation via gen_irq descriptor flag

### 6.1.3	High-Performance Data Paths

Data Path Summary

| Path | Direction | Bandwidth | Features |
| --- | --- | --- | --- |
| Sink | Network to Memory | 512-bit @ clock | SRAM buffering, AXI burst writes |
| Source | Memory to Network | 512-bit @ clock | AXI burst reads, SRAM buffering |

### 6.1.4	Monitoring and Debug

- 64-bit MonBus Packets for all significant events

- State Transition Reports track FSM activity

- Error Event Codes for fault diagnosis

- Performance Metrics for throughput analysis

## 6.2	Feature Details

### 6.2.1	Descriptor Processing

Descriptor Flow

Descriptor Flow

Source: 02_descriptor_flow.mmd

Diagram 2

### 6.2.2	SRAM Buffering

Each data path includes dedicated SRAM buffering:

SRAM Buffer Parameters

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| SRAM_DEPTH | 512 | 64-4096 | Entries per channel |
| Data Width | 512 | Fixed | Bits per entry |
| Total Size | 32KB | 4KB-256KB | Per data path |

Buffer Operation:

- Sink Path: Fill from AXIS, drain to AXI write engine

- Source Path: Fill from AXI read engine, drain to AXIS

- Flow Control: Backpressure when buffer full/empty

### 6.2.3	AXI Burst Optimization

RAPIDS optimizes AXI transactions for maximum bandwidth:

AXI Optimization Features

| Feature | Implementation |
| --- | --- |
| Burst Length | Up to 256 beats (AXI4 max) |
| Outstanding Transactions | Configurable (default 8) |
| Address Alignment | 64-byte aligned (beat size) |
| 4KB Boundary | Automatic burst splitting |

### 6.2.4	Error Handling

Error Handling Summary

| Error Type | Detection | Response |
| --- | --- | --- |
| Address Range | Pre-fetch validation | Descriptor rejected |
| AXI SLVERR | Response check | Transfer aborted |
| AXI DECERR | Response check | Transfer aborted |
| Timeout | Watchdog timer | Channel reset |

# 7	System Context

## 7.1	Integration Overview

RAPIDS Beats integrates into a system as a DMA engine bridging network interfaces and system memory. This section describes the external connections and system-level requirements.

## 7.2	System Block Diagram

System Integration

System Integration

Source: 03_system_integration.mmd

Diagram 3

## 7.3	Interface Connections

### 7.3.1	Memory Interface

RAPIDS requires three AXI4 master ports connected to system memory:

AXI Memory Interface Requirements

| Port | Access Pattern | Typical Bandwidth |
| --- | --- | --- |
| Descriptor AXI | Random read (256-bit) | Low (<100 MB/s) |
| Sink AXI | Sequential write (512-bit) | High (up to interface max) |
| Source AXI | Sequential read (512-bit) | High (up to interface max) |

Interconnect Requirements:

- All three ports MAY share a single physical AXI port with arbitration

- Descriptor port has lowest bandwidth requirement

- Sink and Source ports should have balanced bandwidth allocation

### 7.3.2	Network Interface

RAPIDS uses standard AXI-Stream for network connectivity:

AXIS Network Interface

| Interface | Role | Signals |
| --- | --- | --- |
| Sink AXIS | Receives data from network | TDATA, TVALID, TREADY, TLAST |
| Source AXIS | Sends data to network | TDATA, TVALID, TREADY, TLAST |

Protocol Notes:

- TKEEP/TSTRB supported for partial beat handling

- TID/TDEST may be used for channel routing (optional)

- TUSER available for sideband information

### 7.3.3	Configuration Interface

Software configures RAPIDS through register writes:

Configuration Methods

| Method | Interface | Usage |
| --- | --- | --- |
| Direct Write | Memory-mapped | Register configuration |
| Descriptor Kick | APB-style | Start descriptor processing |

## 7.4	Clock and Reset

### 7.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | All RAPIDS logic |

Note: RAPIDS Beats uses a single clock domain. CDC (clock domain crossing) to different frequency domains must be handled externally.

### 7.4.2	Reset Requirements

Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert |

Reset Sequence:

- Assert aresetn low for minimum 4 clock cycles

- Ensure all AXI interfaces are idle before reset

- Release reset synchronously to aclk rising edge

- Wait 16 clock cycles before initiating transfers

## 7.5	Power Considerations

### 7.5.1	Clock Gating

RAPIDS supports optional clock gating for power reduction:

- Idle Detection: Automatic when no transfers pending

- Per-Channel Gating: Individual channels can be clock-gated

- External Control: cfg_clock_enable override available

### 7.5.2	Power States

Power States

| State | Description | Exit Latency |
| --- | --- | --- |
| Active | Normal operation | 0 cycles |
| Idle | No pending transfers | 1 cycle |
| Gated | Clock disabled | 4 cycles |

# 8	Block Diagram

## 8.1	Top-Level Architecture

RAPIDS Beats Block Diagram

RAPIDS Beats Block Diagram

Source: 04_block_diagram.mmd

Diagram 4

## 8.2	Component Summary

### 8.2.1	Scheduler Group Array

The scheduler group array manages 8 independent channels:

Scheduler Array Components

| Component | Instance Count | Purpose |
| --- | --- | --- |
| scheduler_beats | 8 | Transfer coordination per channel |
| descriptor_engine_beats | 8 | Descriptor fetch and parse |
| Descriptor AXI Arbiter | 1 | Shared AXI4 for descriptor fetch |

### 8.2.2	Sink Data Path

Network-to-memory data flow:

Sink Path Components

| Component | Purpose |
| --- | --- |
| snk_sram_controller_beats | Multi-channel SRAM management |
| axi_write_engine_beats | AXI4 burst write generation |
| alloc_ctrl_beats | Space allocation tracking |
| drain_ctrl_beats | Data availability tracking |

### 8.2.3	Source Data Path

Memory-to-network data flow:

Source Path Components

| Component | Purpose |
| --- | --- |
| axi_read_engine_beats | AXI4 burst read generation |
| src_sram_controller_beats | Multi-channel SRAM management |
| alloc_ctrl_beats | Space allocation tracking |
| drain_ctrl_beats | Data availability tracking |

## 8.3	Hierarchy

rapids_core_beats
├── beats_scheduler_group_array
│   ├── beats_scheduler_group [0]
│   │   ├── scheduler_beats
│   │   └── descriptor_engine_beats
│   ├── beats_scheduler_group [1..6]
│   │   └── ...
│   └── beats_scheduler_group [7]
│       └── ...
├── sink_data_path_beats
│   ├── snk_sram_controller_beats
│   │   └── snk_sram_controller_unit_beats [0..7]
│   ├── axi_write_engine_beats
│   ├── alloc_ctrl_beats
│   └── drain_ctrl_beats
└── source_data_path_beats
    ├── axi_read_engine_beats
    ├── src_sram_controller_beats
    │   └── src_sram_controller_unit_beats [0..7]
    ├── alloc_ctrl_beats
    └── drain_ctrl_beats

## 8.4	Internal Buses

Internal Bus Summary

| Bus | Width | Description |
| --- | --- | --- |
| Descriptor Bus | 256-bit | Parsed descriptor to scheduler |
| Scheduler Command | Variable | Transfer parameters to data paths |
| SRAM Data | 512-bit | Data to/from SRAM buffers |
| MonBus | 64-bit | Monitor packets (aggregated) |

# 9	Data Flow

## 9.1	Overview

RAPIDS Beats implements two independent data paths that can operate concurrently:

- Sink Path: Network to Memory (AXIS Slave -> AXI Write)

- Source Path: Memory to Network (AXI Read -> AXIS Master)

## 9.2	Sink Path Data Flow

Sink Data Flow

Sink Data Flow

Source: 05_sink_flow.mmd

Diagram 5

### 9.2.1	Sink Path Timing

Sink Path Timing

Sink Path Timing

Source: sink_path_timing.json

Waveform 1

## 9.3	Source Path Data Flow

Source Data Flow

Source Data Flow

Source: 06_source_flow.mmd

Diagram 6

### 9.3.1	Source Path Timing

Source Path Timing

Source Path Timing

Source: source_path_timing.json

Waveform 2

## 9.4	Concurrent Operation

Both data paths operate independently and can run simultaneously:

Diagram 7

### 9.4.1	Deadlock Prevention

RAPIDS prevents deadlock through independent resource allocation:

Resource Isolation

| Resource | Sink Path | Source Path | Shared |
| --- | --- | --- | --- |
| SRAM Buffer | Dedicated | Dedicated | No |
| AXI Port | Write only | Read only | No |
| Scheduler | Per-channel | Per-channel | No |

# 10	Channel Architecture

## 10.1	Multi-Channel Design

RAPIDS Beats supports 8 independent DMA channels, each with dedicated resources for concurrent operation.

## 10.2	Channel Resource Allocation

Channel Architecture

Channel Architecture

Source: 07_channel_arch.mmd

Diagram 8

## 10.3	Per-Channel Resources

Each channel has dedicated:

Per-Channel Resources

| Resource | Description | Size/Capacity |
| --- | --- | --- |
| Scheduler | Transfer coordination FSM | 1 instance |
| Descriptor Engine | Descriptor fetch/parse | 8-deep FIFO |
| Sink SRAM Unit | Sink buffer partition | SRAM_DEPTH/8 entries |
| Source SRAM Unit | Source buffer partition | SRAM_DEPTH/8 entries |
| Configuration Registers | Per-channel config | ~16 registers |

## 10.4	Shared Resources

Resources shared across all channels:

Shared Resources

| Resource | Arbitration | Max Outstanding |
| --- | --- | --- |
| Descriptor AXI | Round-robin | 8 (1 per channel) |
| Sink AXI Write | Round-robin | 8 configurable |
| Source AXI Read | Round-robin | 8 configurable |
| MonBus Output | Priority | 1 (serialized) |

## 10.5	Channel Scheduling

Arbitration Policy

Arbitration Timing:

Channel Arbitration

Channel Arbitration

Source: channel_arbitration.json

Waveform 3

## 10.6	Channel State Machine

Each channel scheduler follows this state machine:

Channel FSM

Channel FSM

Source: 08_channel_fsm.mmd

Diagram 10

### 10.6.1	Channel State Timing

Channel State Timing

Channel State Timing

Source: channel_state_timing.json

Waveform 4

## 10.7	Channel Isolation

Channels are isolated to prevent interference:

Channel Isolation Features

| Isolation Type | Implementation |
| --- | --- |
| State Isolation | Separate FSM per channel |
| Buffer Isolation | Partitioned SRAM per channel |
| Error Isolation | Per-channel error flags |
| Reset Isolation | Per-channel soft reset |

### 10.7.1	Per-Channel Error Handling

Each channel maintains independent error state:

Waveform 5

Channel 1 error does not affect Channel 0 or Channel 2 operation.

# 11	Interface Summary

## 11.1	External Interface Overview

RAPIDS Beats exposes the following external interfaces:

Interface Overview

Interface Overview

Source: 09_interface_overview.mmd

Diagram 11

## 11.2	Interface Port Summary

External Interface Summary

| Interface | Type | Direction | Data Width | Address Width |
| --- | --- | --- | --- | --- |
| Descriptor AXI | AXI4 Master | Read | 256-bit | 64-bit |
| Sink AXI | AXI4 Master | Write | 512-bit | 64-bit |
| Source AXI | AXI4 Master | Read | 512-bit | 64-bit |
| Sink AXIS | AXIS Slave | Input | 512-bit | - |
| Source AXIS | AXIS Master | Output | 512-bit | - |
| APB Config | APB-like | Input | 32-bit | 12-bit |
| MonBus | Custom | Output | 64-bit | - |

## 11.3	Clock and Reset

All interfaces operate in the same clock domain:

Clock and Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aclk | input | System clock (100-500 MHz typical) |
| aresetn | input | Active-low reset (async assert, sync deassert) |

## 11.4	Interface Feature Matrix

Interface Feature Matrix

| Feature | Desc AXI | Sink AXI | Src AXI | Sink AXIS | Src AXIS |
| --- | --- | --- | --- | --- | --- |
| Protocol | AXI4 | AXI4 | AXI4 | AXIS | AXIS |
| Direction | Read | Write | Read | Slave | Master |
| Burst Support | INCR | INCR | INCR | N/A | N/A |
| Max Burst | 1 | 256 | 256 | N/A | N/A |
| Outstanding | 8 | 8 | 8 | N/A | N/A |
| ID Width | 8-bit | 8-bit | 8-bit | Optional | Optional |

## 11.5	Signal Count Summary

Signal Count Summary

| Interface | Input Signals | Output Signals | Total |
| --- | --- | --- | --- |
| Descriptor AXI | ~20 | ~30 | ~50 |
| Sink AXI | ~15 | ~40 | ~55 |
| Source AXI | ~25 | ~25 | ~50 |
| Sink AXIS | ~20 | ~5 | ~25 |
| Source AXIS | ~5 | ~20 | ~25 |
| APB Config | ~15 | ~5 | ~20 |
| MonBus | ~2 | ~5 | ~7 |
| Total | ~102 | ~130 | ~232 |

Note: Signal counts are approximate and vary with configuration parameters.

# 12	AXI4 Interface Specification

## 12.1	Overview

RAPIDS Beats uses three AXI4 master interfaces for memory access:

- Descriptor AXI - Fetches 256-bit descriptors (read-only)

- Sink AXI - Writes data to memory (write-only)

- Source AXI - Reads data from memory (read-only)

## 12.2	Descriptor AXI Master

### 12.2.1	Purpose

Fetches descriptor structures from system memory for all 8 channels.

### 12.2.2	Configuration

Descriptor AXI Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 256-bit | Fixed descriptor size |
| Address Width | 64-bit | Configurable |
| ID Width | 8-bit | Configurable |
| Burst Length | 1 | Single-beat only |
| Burst Type | INCR | Fixed |

### 12.2.3	Signal List

Descriptor AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_desc_aclk | 1 | input | Clock |
| m_axi_desc_aresetn | 1 | input | Reset (active-low) |
| m_axi_desc_arid | 8 | output | Read ID (channel ID) |
| m_axi_desc_araddr | 64 | output | Read address |
| m_axi_desc_arlen | 8 | output | Burst length (always 0) |
| m_axi_desc_arsize | 3 | output | Beat size (5 = 32 bytes) |
| m_axi_desc_arburst | 2 | output | Burst type (INCR) |
| m_axi_desc_arvalid | 1 | output | Address valid |
| m_axi_desc_arready | 1 | input | Address ready |
| m_axi_desc_rid | 8 | input | Response ID |
| m_axi_desc_rdata | 256 | input | Read data |
| m_axi_desc_rresp | 2 | input | Read response |
| m_axi_desc_rlast | 1 | input | Last beat |
| m_axi_desc_rvalid | 1 | input | Data valid |
| m_axi_desc_rready | 1 | output | Data ready |

### 12.2.4	Timing Diagram

Descriptor Fetch Timing

Descriptor Fetch Timing

Source: desc_axi_timing.json

Waveform 6

## 12.3	Sink AXI Master (Write)

### 12.3.1	Purpose

Writes data from SRAM buffer to system memory.

### 12.3.2	Configuration

Sink AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | Match SRAM width |
| Address Width | 64-bit | Configurable | System address space |
| ID Width | 8-bit | Configurable | Transaction tracking |
| Max Burst | 256 | 1-256 | AXI4 maximum |
| Outstanding | 8 | 1-16 | Concurrent AW requests |

### 12.3.3	Signal List

Sink AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_sink_awid | 8 | output | Write ID |
| m_axi_sink_awaddr | 64 | output | Write address |
| m_axi_sink_awlen | 8 | output | Burst length - 1 |
| m_axi_sink_awsize | 3 | output | Beat size (6 = 64 bytes) |
| m_axi_sink_awburst | 2 | output | Burst type (INCR) |
| m_axi_sink_awvalid | 1 | output | Address valid |
| m_axi_sink_awready | 1 | input | Address ready |
| m_axi_sink_wdata | 512 | output | Write data |
| m_axi_sink_wstrb | 64 | output | Write strobes |
| m_axi_sink_wlast | 1 | output | Last beat |
| m_axi_sink_wvalid | 1 | output | Data valid |
| m_axi_sink_wready | 1 | input | Data ready |
| m_axi_sink_bid | 8 | input | Response ID |
| m_axi_sink_bresp | 2 | input | Write response |
| m_axi_sink_bvalid | 1 | input | Response valid |
| m_axi_sink_bready | 1 | output | Response ready |

### 12.3.4	Timing Diagram

Sink AXI Write Timing

Sink AXI Write Timing

Source: sink_axi_timing.json

Waveform 7

## 12.4	Source AXI Master (Read)

### 12.4.1	Purpose

Reads data from system memory to SRAM buffer.

### 12.4.2	Configuration

Source AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | Match SRAM width |
| Address Width | 64-bit | Configurable | System address space |
| ID Width | 8-bit | Configurable | Transaction tracking |
| Max Burst | 256 | 1-256 | AXI4 maximum |
| Outstanding | 8 | 1-16 | Concurrent AR requests |

### 12.4.3	Signal List

Source AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_src_arid | 8 | output | Read ID |
| m_axi_src_araddr | 64 | output | Read address |
| m_axi_src_arlen | 8 | output | Burst length - 1 |
| m_axi_src_arsize | 3 | output | Beat size (6 = 64 bytes) |
| m_axi_src_arburst | 2 | output | Burst type (INCR) |
| m_axi_src_arvalid | 1 | output | Address valid |
| m_axi_src_arready | 1 | input | Address ready |
| m_axi_src_rid | 8 | input | Response ID |
| m_axi_src_rdata | 512 | input | Read data |
| m_axi_src_rresp | 2 | input | Read response |
| m_axi_src_rlast | 1 | input | Last beat |
| m_axi_src_rvalid | 1 | input | Data valid |
| m_axi_src_rready | 1 | output | Data ready |

### 12.4.4	Timing Diagram

Source AXI Read Timing

Source AXI Read Timing

Source: source_axi_timing.json

Waveform 8

## 12.5	AXI Protocol Constraints

### 12.5.1	Address Alignment

Address Alignment Requirements

| Transfer Type | Alignment Requirement |
| --- | --- |
| Descriptor | 32-byte aligned |
| Sink Data | 64-byte aligned |
| Source Data | 64-byte aligned |

### 12.5.2	4KB Boundary Handling

AXI4 requires burst transactions not to cross 4KB boundaries. RAPIDS automatically splits bursts at 4KB boundaries:

Example: 8-beat burst starting at 0x0FE0 (64 bytes/beat)
  Beat 0-1: 0x0FE0 - 0x0FFF (to 4KB boundary)
  Beat 2-7: 0x1000 - 0x117F (after boundary)

Split into two bursts:
  Burst 1: ARLEN=1, ARADDR=0x0FE0
  Burst 2: ARLEN=5, ARADDR=0x1000

### 12.5.3	Response Handling

AXI Response Handling

| Response | Code | RAPIDS Behavior |
| --- | --- | --- |
| OKAY | 2’b00 | Normal completion |
| EXOKAY | 2’b01 | Treated as OKAY |
| SLVERR | 2’b10 | Error, transfer aborted |
| DECERR | 2’b11 | Error, transfer aborted |

# 13	AXIS Interface Specification

## 13.1	Overview

RAPIDS Beats uses AXI-Stream interfaces for network data transfer:

- Sink AXIS Slave - Receives data from network (ingress)

- Source AXIS Master - Sends data to network (egress)

## 13.2	Sink AXIS Slave

### 13.2.1	Purpose

Receives streaming data from external network interface and writes to SRAM buffer.

### 13.2.2	Configuration

Sink AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | TDATA width |
| ID Width | 8-bit | 0-8 | TID width (optional) |
| DEST Width | 8-bit | 0-8 | TDEST width (optional) |
| USER Width | 1-bit | 0-16 | TUSER width (optional) |

### 13.2.3	Signal List

Sink AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_sink_tdata | 512 | input | Data payload |
| s_axis_sink_tkeep | 64 | input | Byte valid mask |
| s_axis_sink_tstrb | 64 | input | Byte position (optional) |
| s_axis_sink_tlast | 1 | input | End of packet |
| s_axis_sink_tid | 8 | input | Transaction ID (channel select) |
| s_axis_sink_tdest | 8 | input | Destination (optional) |
| s_axis_sink_tuser | 1 | input | User sideband (optional) |
| s_axis_sink_tvalid | 1 | input | Data valid |
| s_axis_sink_tready | 1 | output | Ready to accept |

### 13.2.4	Timing Diagram

Sink AXIS Timing

Sink AXIS Timing

Source: sink_axis_timing.json

Waveform 9

### 13.2.5	Backpressure Behavior

When SRAM buffer is full, tready deasserts:

Waveform 10

## 13.3	Source AXIS Master

### 13.3.1	Purpose

Sends streaming data from SRAM buffer to external network interface.

### 13.3.2	Configuration

Source AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | TDATA width |
| ID Width | 8-bit | 0-8 | TID width (optional) |
| DEST Width | 8-bit | 0-8 | TDEST width (optional) |
| USER Width | 1-bit | 0-16 | TUSER width (optional) |

### 13.3.3	Signal List

Source AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_src_tdata | 512 | output | Data payload |
| m_axis_src_tkeep | 64 | output | Byte valid mask |
| m_axis_src_tstrb | 64 | output | Byte position (optional) |
| m_axis_src_tlast | 1 | output | End of packet |
| m_axis_src_tid | 8 | output | Transaction ID (channel) |
| m_axis_src_tdest | 8 | output | Destination (optional) |
| m_axis_src_tuser | 1 | output | User sideband (optional) |
| m_axis_src_tvalid | 1 | output | Data valid |
| m_axis_src_tready | 1 | input | Downstream ready |

### 13.3.4	Timing Diagram

Source AXIS Timing

Source AXIS Timing

Source: source_axis_timing.json

Waveform 11

### 13.3.5	Data Availability

Source AXIS only asserts tvalid when data is available in SRAM:

Waveform 12

## 13.4	AXIS Protocol Notes

### 13.4.1	Handshake Rules

Standard AXI-Stream handshake applies:

- tvalid asserts when data is available

- tready asserts when receiver can accept

- Transfer occurs on clock edge when both are high

- tvalid SHALL NOT depend on tready

- tready MAY depend on tvalid

### 13.4.2	TLAST Semantics

TLAST Usage

| Scenario | TLAST Behavior |
| --- | --- |
| Descriptor transfer complete | Assert on last beat |
| Packet boundary | Assert on last beat of packet |
| Continuous stream | Not used (always 0) |

### 13.4.3	TKEEP Usage

TKEEP indicates valid bytes within each beat:

TKEEP Examples

| TKEEP Value | Meaning |
| --- | --- |
| 64’hFFFF_FFFF_FFFF_FFFF | All 64 bytes valid |
| 64’h0000_FFFF_FFFF_FFFF | Upper 16 bytes invalid |
| 64’h0000_0000_0000_000F | Only lower 4 bytes valid |

### 13.4.4	Channel Multiplexing via TID

Multiple channels can share a single AXIS interface using TID:

Waveform 13

# 14	APB Configuration Interface

## 14.1	Overview

RAPIDS Beats uses an APB-like interface for configuration and descriptor kick-off. This interface allows software to:

- Configure channel parameters

- Initiate descriptor processing (kick-off)

- Read status and error information

## 14.2	Configuration Interface

### 14.2.1	Signal List

APB Configuration Signals (NC = NUM_CHANNELS)

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| apb_valid | NC | input | Per-channel kick-off valid |
| apb_ready | NC | output | Per-channel ready |
| apb_addr | 64 | input | Descriptor address |
| cfg_channel_enable | NC | input | Per-channel enable |
| cfg_addr0_base | 64 | input | Address range 0 base |
| cfg_addr0_limit | 64 | input | Address range 0 limit |
| cfg_addr1_base | 64 | input | Address range 1 base |
| cfg_addr1_limit | 64 | input | Address range 1 limit |

### 14.2.2	Descriptor Kick-Off

To start descriptor processing on a channel:

- Write descriptor to memory

- Assert apb_valid[ch] with descriptor address on apb_addr

- Wait for apb_ready[ch] assertion

- Deassert apb_valid[ch]

### 14.2.3	Timing Diagram

APB Kick-Off Timing

APB Kick-Off Timing

Source: apb_kickoff_timing.json

Waveform 14

## 14.3	Status Interface

### 14.3.1	Per-Channel Status Signals

Status Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| channel_idle | NC | output | Channel idle status |
| channel_error | NC | output | Channel error flag |
| scheduler_state | 4*NC | output | FSM state per channel |

### 14.3.2	Channel State Encoding

Scheduler State Encoding

| State | Value | Description |
| --- | --- | --- |
| IDLE | 4’h0 | Waiting for kick-off |
| WAIT_DESC | 4’h1 | Waiting for descriptor |
| PARSE_DESC | 4’h2 | Parsing descriptor |
| CH_XFER_DATA | 4’h3 | Transfer in progress |
| CHECK_NEXT | 4’h4 | Checking next descriptor |
| COMPLETE | 4’h5 | Transfer complete |
| ERROR | 4’hF | Error state |

## 14.4	Configuration Registers

### 14.4.1	Address Range Validation

RAPIDS validates descriptor addresses against configurable ranges:

Valid if:
  (addr >= cfg_addr0_base && addr < cfg_addr0_limit) ||
  (addr >= cfg_addr1_base && addr < cfg_addr1_limit)

### 14.4.2	Configuration Parameters

Configuration Register Map

| Register | Offset | Width | Description |
| --- | --- | --- | --- |
| CTRL | 0x00 | 32 | Global control |
| STATUS | 0x04 | 32 | Global status |
| CH_ENABLE | 0x08 | 8 | Channel enable bitmap |
| CH_STATUS | 0x0C | 8 | Channel busy bitmap |
| ADDR0_BASE | 0x10 | 64 | Address range 0 base |
| ADDR0_LIMIT | 0x18 | 64 | Address range 0 limit |
| ADDR1_BASE | 0x20 | 64 | Address range 1 base |
| ADDR1_LIMIT | 0x28 | 64 | Address range 1 limit |
| IRQ_ENABLE | 0x30 | 8 | IRQ enable per channel |
| IRQ_STATUS | 0x34 | 8 | IRQ status per channel |

### 14.4.3	Control Register (CTRL)

CTRL Register Bits

| Bit | Name | Description |
| --- | --- | --- |
| 0 | ENABLE | Global enable |
| 1 | SOFT_RESET | Soft reset (self-clearing) |
| 7:2 | Reserved | - |

### 14.4.4	Status Register (STATUS)

STATUS Register Bits

| Bit | Name | Description |
| --- | --- | --- |
| 0 | BUSY | Any channel busy |
| 1 | ERROR | Any channel error |
| 7:2 | Reserved | - |

## 14.5	Programming Sequence

### 14.5.1	Initialization

// 1. Assert soft reset
CTRL = 0x02;
while (CTRL & 0x02);  // Wait for reset complete

// 2. Configure address ranges
ADDR0_BASE = 0x80000000;
ADDR0_LIMIT = 0x90000000;

// 3. Enable desired channels
CH_ENABLE = 0xFF;  // Enable all 8 channels

// 4. Enable IRQs if needed
IRQ_ENABLE = 0xFF;

// 5. Global enable
CTRL = 0x01;

### 14.5.2	Descriptor Kick-Off

// Prepare descriptor in memory
desc->src_addr = src;
desc->dst_addr = dst;
desc->length = beats;
desc->last = 1;
desc->valid = 1;

// Kick off channel 0
apb_kickoff(0, desc_addr);

// Wait for completion
while (!(channel_idle & 0x01));

// Check for errors
if (channel_error & 0x01) {
    handle_error(0);
}

# 15	Monitor Bus Interface

## 15.1	Overview

RAPIDS Beats outputs a 64-bit Monitor Bus (MonBus) for real-time event reporting. The MonBus provides visibility into:

- State machine transitions

- Descriptor processing events

- Error conditions

- Performance metrics

## 15.2	MonBus Packet Format

### 15.2.1	64-bit Packet Structure

Bit Field          Width   Description
───────────────────────────────────────────────────────
[63:60]            4-bit   packet_type (error, completion, etc.)
[59:57]            3-bit   protocol (CORE for RAPIDS)
[56:53]            4-bit   event_code (specific event)
[52:47]            6-bit   channel_id
[46:43]            4-bit   unit_id (subsystem)
[42:35]            8-bit   agent_id (module)
[34:0]             35-bit  event_data (event-specific)

### 15.2.2	Signal List

MonBus Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | 1 | output | Packet valid |
| monbus_pkt_data | 64 | output | Packet data |
| monbus_pkt_ready | 1 | input | Downstream ready |

## 15.3	Packet Types

MonBus Packet Types

| Type | Code | Description |
| --- | --- | --- |
| ERROR | 4’h0 | Error event |
| COMPLETION | 4’h1 | Transfer/operation complete |
| THRESHOLD | 4’h2 | Threshold crossed |
| TIMEOUT | 4’h3 | Timeout event |
| PERF | 4’h4 | Performance metric |
| DEBUG | 4’hF | Debug/trace event |

## 15.4	Agent IDs

RAPIDS modules use the following Agent IDs:

RAPIDS Agent IDs

| Agent ID | Module | Description |
| --- | --- | --- |
| 0x10-0x17 | Descriptor Engine | Channels 0-7 |
| 0x30-0x37 | Scheduler | Channels 0-7 |
| 0x40-0x47 | Sink Data Path | Channels 0-7 |
| 0x50-0x57 | Source Data Path | Channels 0-7 |

## 15.5	Event Codes

### 15.5.1	Scheduler Events (Agent 0x30-0x37)

Scheduler Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor complete |
| 0x1 | COMPLETION | Chain complete |
| 0x2 | ERROR | Timeout |
| 0x3 | ERROR | AXI error |
| 0x4 | DEBUG | State transition |

### 15.5.2	Descriptor Engine Events (Agent 0x10-0x17)

Descriptor Engine Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor fetched |
| 0x1 | ERROR | Address range error |
| 0x2 | ERROR | AXI read error |
| 0x3 | DEBUG | Fetch started |

## 15.6	Timing Diagram

MonBus Timing

MonBus Timing

Source: monbus_timing.json

Waveform 15

## 15.7	Event Data Encoding

### 15.7.1	Completion Event Data

[34:32] - Reserved
[31:0]  - Transfer length (beats completed)

### 15.7.2	Error Event Data

[34:32] - Error type
[31:16] - Reserved
[15:0]  - Error details

### 15.7.3	State Transition Event Data

[34:32] - Reserved
[31:28] - Previous state
[27:24] - New state
[23:0]  - Timestamp (lower bits)

## 15.8	MonBus Arbitration

Multiple RAPIDS modules generate MonBus packets. Internal arbitration ensures ordered delivery:

Diagram 12

### 15.8.1	Arbitration Policy

- Round-robin across modules

- No packet loss (backpressure if output blocked)

- Priority boosting for ERROR packets

## 15.9	Integration Notes

### 15.9.1	Downstream Connection

MonBus output typically connects to:

- MonBus Aggregator - Combines multiple sources

- AXIL Converter - Maps to AXI-Lite for CPU access

- Debug FIFO - Buffered access for debug tools

### 15.9.2	Packet Rate

MonBus Packet Rates

| Scenario | Approximate Rate |
| --- | --- |
| Idle | 0 packets/cycle |
| Single channel active | ~1 packet/100 cycles |
| All channels active | ~1 packet/20 cycles |
| Error storm | Up to 1 packet/cycle |

# 16	Use Case: Network to Memory (Sink Path)

## 16.1	Overview

The Sink Path transfers data from a network interface (AXI-Stream slave) to system memory (AXI4 write master). This is the primary receive data path for network applications.

## 16.2	Operation Flow

Sink Flow

Sink Flow

Source: 09_sink_flow.mmd

Diagram 13

## 16.3	Timing Diagram

Sink Transfer Timing

Sink Transfer Timing

Source: sink_transfer.json

Waveform 16

## 16.4	Buffer Management

### 16.4.1	SRAM Buffering Strategy

The sink path uses internal SRAM to decouple network timing from memory timing:

Diagram 14

### 16.4.2	Flow Control

Sink Path Flow Control

| Condition | Behavior |
| --- | --- |
| SRAM full | Backpressure network (TREADY=0) |
| SRAM empty | Pause AXI writes |
| SRAM threshold | Optional early write start |

Watermark Timing

## 16.5	Performance Considerations

### 16.5.1	Throughput

Sink Path Throughput Factors

| Factor | Impact |
| --- | --- |
| Network bandwidth | Upper bound on receive rate |
| Memory bandwidth | Must match or exceed network |
| SRAM depth | Burst absorption capability |
| AXI outstanding | Overlapped write latency hiding |

### 16.5.2	Latency

Sink Path Latency Breakdown

| Phase | Typical Cycles |
| --- | --- |
| Descriptor fetch | 10-50 (memory dependent) |
| First beat to SRAM | 1 |
| SRAM to AXI write | 2-4 |
| AXI write completion | 10-100 (memory dependent) |

## 16.6	Error Handling

### 16.6.1	Error Sources

Sink Path Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI write error | BRESP != OKAY | Stop transfer, report via MonBus |
| SRAM overflow | Fill pointer catches drain | Should never occur (TREADY control) |
| Timeout | Watchdog timer | Abort transfer, report error |

Error Recovery Sequence

# 17	Use Case: Memory to Network (Source Path)

## 17.1	Overview

The Source Path transfers data from system memory (AXI4 read master) to a network interface (AXI-Stream master). This is the primary transmit data path for network applications.

## 17.2	Operation Flow

Source Flow

Source Flow

Source: 10_source_flow.mmd

Diagram 16

## 17.3	Timing Diagram

Source Transfer Timing

Source Transfer Timing

Source: source_transfer.json

Waveform 18

## 17.4	Buffer Management

### 17.4.1	SRAM Buffering Strategy

The source path uses internal SRAM to prefetch data and maintain network throughput:

Diagram 17

### 17.4.2	Flow Control

Source Path Flow Control

| Condition | Behavior |
| --- | --- |
| SRAM full | Pause AXI reads |
| SRAM empty | Hold TVALID low |
| Network backpressure | Stop draining SRAM |

Prefetch Strategy

## 17.5	Concurrent Operation

### 17.5.1	Read-Ahead Pipeline

The source path can issue AXI reads ahead of network consumption:

Waveform 20

## 17.6	Performance Considerations

### 17.6.1	Throughput

Source Path Throughput Factors

| Factor | Impact |
| --- | --- |
| Memory bandwidth | Upper bound on transmit rate |
| Network acceptance | Must keep pace with reads |
| SRAM depth | Read-ahead capability |
| AXI outstanding | Overlapped read latency hiding |

### 17.6.2	Latency

Source Path Latency Breakdown

| Phase | Typical Cycles |
| --- | --- |
| Descriptor fetch | 10-50 (memory dependent) |
| AXI read issue | 1-2 |
| Memory read latency | 10-100 (memory dependent) |
| SRAM to network | 1-2 |

## 17.7	Error Handling

### 17.7.1	Error Sources

Source Path Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI read error | RRESP != OKAY | Stop transfer, report via MonBus |
| Network timeout | Watchdog timer | Abort transfer, report error |
| Address range | Descriptor validation | Reject descriptor |

Backpressure Handling

When the network deasserts TREADY: 1. SRAM drain stalls immediately 2. SRAM continues filling until threshold 3. AXI reads pause when SRAM approaches full 4. Resume automatically when TREADY returns

# 18	Use Case: Descriptor Chaining

## 18.1	Overview

Descriptor chaining enables multiple transfers to execute sequentially without software intervention. Each descriptor contains a pointer to the next descriptor, forming a linked list that RAPIDS processes automatically.

## 18.2	Chain Structure

Descriptor Chain

Descriptor Chain

Source: 11_descriptor_chain.mmd

Diagram 18

Chain Execution Flow

## 18.3	Timing Diagram

Chain Execution

Chain Execution

Source: chain_execution.json

Waveform 22

## 18.4	Descriptor Fields for Chaining

### 18.4.1	Relevant Fields

Chaining-Related Descriptor Fields

| Field | Bits | Description |
| --- | --- | --- |
| next_ptr | [191:128] | 64-bit pointer to next descriptor |
| last | [255] | Last descriptor in chain flag |
| irq_en | [254] | Generate interrupt on completion |

### 18.4.2	Chain Termination

A chain terminates when ANY of these conditions is true:

- next_ptr == 0 (null pointer)

- last == 1 (explicit last flag)

- Error during transfer

Diagram 20

## 18.5	Prefetch Optimization

### 18.5.1	Descriptor Prefetching

RAPIDS can prefetch the next descriptor while the current transfer executes:

Waveform 23

Benefit: Zero-cycle gap between chained transfers when prefetch completes before current transfer.

## 18.6	Multi-Channel Chaining

Each channel maintains independent chains:

Diagram 21

Channels execute their chains concurrently and independently.

## 18.7	Error During Chain

### 18.7.1	Error Handling

When an error occurs mid-chain:

Waveform 24

Error Response: 1. Current transfer aborts 2. Chain processing stops 3. Error status captures failing descriptor index 4. MonBus reports error event 5. Interrupt generated (if enabled)

### 18.7.2	Recovery

Software must: 1. Read error status to identify failing descriptor 2. Fix the issue (descriptor content, memory mapping, etc.) 3. Restart chain from failing descriptor or beginning

# 19	Use Case: Multi-Channel Operation

## 19.1	Overview

RAPIDS Beats supports 8 independent DMA channels that can operate concurrently. This enables parallel data transfers for multiple network connections, priority-based scheduling, and efficient resource utilization.

## 19.2	Channel Independence

Multi-Channel Architecture

Multi-Channel Architecture

Source: 12_multi_channel.mmd

Diagram 22

## 19.3	Concurrent Operation Timing

Multi-Channel Timing

Multi-Channel Timing

Source: multi_channel_timing.json

Waveform 25

## 19.4	Resource Arbitration

### 19.4.1	Descriptor Port Arbitration

All channels share a single AXI port for descriptor fetches:

Waveform 26

### 19.4.2	Sink/Source Arbitration

Data path AXI ports use round-robin arbitration with optional priority:

Diagram 23

## 19.5	Channel Isolation

### 19.5.1	Error Isolation

Errors in one channel do not affect other channels:

Waveform 27

### 19.5.2	Per-Channel Soft Reset

Individual channels can be reset without affecting others:

Waveform 28

## 19.6	Use Case Examples

### 19.6.1	Mixed Traffic Pattern

Typical network application with different traffic types:

Mixed Traffic Channel Assignment

| Channel | Traffic Type | Direction | Priority | Notes |
| --- | --- | --- | --- | --- |
| 0 | Voice/RTP | Sink | High | Low latency required |
| 1 | Voice/RTP | Source | High | Low latency required |
| 2 | Video | Sink | Medium | High bandwidth |
| 3 | Control | Source | Medium | Small packets |
| 4-7 | Bulk data | Both | Low | Background transfers |

### 19.6.2	Bidirectional Transfer

Single network connection using paired channels:

Diagram 24

### 19.6.3	Load Balancing

Multiple channels processing same traffic type:

Diagram 25

## 19.7	Performance Considerations

### 19.7.1	Bandwidth Sharing

Bandwidth Distribution

| Active Channels | Per-Channel BW (% of max) | Notes |
| --- | --- | --- |
| 1 | 100% | Full bandwidth available |
| 2 | ~50% each | Round-robin sharing |
| 4 | ~25% each | Fair distribution |
| 8 | ~12.5% each | All channels active |

Note: Actual distribution depends on transfer sizes and memory latency.

### 19.7.2	Latency Impact

Multi-channel operation adds arbitration latency:

Arbitration Latency

| Scenario | Additional Latency |
| --- | --- |
| Single channel | 0 cycles |
| 2 channels contending | 1-2 cycles |
| 8 channels contending | Up to 7 cycles |

## 19.8	MonBus Events

Each channel generates independent MonBus events:

Waveform 29

Events are arbitrated and serialized on the shared MonBus output.

# 20	Descriptor Format

## 20.1	Overview

RAPIDS Beats uses 256-bit (32-byte) descriptors to define DMA transfers. Descriptors are stored in system memory and fetched by the Descriptor Engine when a channel is kicked.

## 20.2	Descriptor Layout

Descriptor Format

Descriptor Format

Source: 13_descriptor_format.mmd

### 20.2.1	256-Bit Structure

Bit Range    Field Name        Width   Description
────────────────────────────────────────────────────────────────
[255]        last              1       Last descriptor in chain
[254]        irq_en            1       Generate interrupt on completion
[253:252]    reserved          2       Reserved (write 0)
[251:248]    direction         4       Transfer direction
[247:240]    reserved          8       Reserved (write 0)
[239:224]    transfer_length   16      Transfer length in beats
[223:192]    reserved          32      Reserved (write 0)
[191:128]    next_ptr          64      Pointer to next descriptor
[127:64]     dest_addr         64      Destination address (sink) or source address
[63:0]       src_addr          64      Source address (source path) or buffer addr

Visual Layout

## 20.3	Field Descriptions

### 20.3.1	Control Field [255:248]

Control Field Bits

| Bit | Field | Description |
| --- | --- | --- |
| 255 | last | Set to 1 for last descriptor in chain |
| 254 | irq_en | Set to 1 to generate completion interrupt |
| 253:252 | Reserved | Write as 0 |
| 251:248 | direction | Transfer direction encoding |

### 20.3.2	Direction Encoding

Direction Field Encoding

| Value | Name | Description |
| --- | --- | --- |
| 4’h0 | SINK | Network to memory (AXIS slave to AXI write) |
| 4’h1 | SOURCE | Memory to network (AXI read to AXIS master) |
| 4’h2-F | Reserved | Reserved for future use |

### 20.3.3	Transfer Length [239:224]

- 16-bit field specifying transfer size in beats

- 1 beat = DATA_WIDTH bits (default 512 bits = 64 bytes)

- Maximum transfer: 65,535 beats (4 MB at 64 bytes/beat)

- Length of 0 is reserved (no operation)

### 20.3.4	Next Pointer [191:128]

- 64-bit byte address of next descriptor

- Must be 32-byte aligned (bits [4:0] ignored)

- Value of 0 terminates chain (regardless of last bit)

### 20.3.5	Address Fields [127:0]

For SINK transfers (network to memory): | Field | Usage | |——-|——-| | dest_addr [127:64] | Memory write destination address | | src_addr [63:0] | Not used (write 0) |

For SOURCE transfers (memory to network): | Field | Usage | |——-|——-| | dest_addr [127:64] | Not used (write 0) | | src_addr [63:0] | Memory read source address |

: Address Field Usage

## 20.4	Descriptor Fetch Timing

Descriptor Fetch

Descriptor Fetch

Source: descriptor_fetch.json

Waveform 30

## 20.5	Alignment Requirements

Address Alignment Requirements

| Field | Alignment | Notes |
| --- | --- | --- |
| Descriptor address | 32-byte | Bits [4:0] ignored |
| next_ptr | 32-byte | Bits [4:0] ignored |
| dest_addr | DATA_WIDTH/8 | 64-byte for 512-bit data |
| src_addr | DATA_WIDTH/8 | 64-byte for 512-bit data |

Note: Unaligned addresses may cause unpredictable behavior or AXI protocol violations.

## 20.6	Descriptor Examples

### 20.6.1	Sink Descriptor (Network to Memory)

Hex representation (LSB to MSB):
[63:0]    = 0x0000_0000_0000_0000  // src_addr (unused)
[127:64]  = 0x0000_0001_0000_0000  // dest_addr = 0x1_0000_0000
[191:128] = 0x0000_0000_0000_2000  // next_ptr = 0x2000
[255:192] = 0xC000_0100_0000_0000  // last=1, irq=1, dir=SINK, len=256

Binary control field:
  [255] = 1      // Last descriptor
  [254] = 1      // Interrupt enabled
  [253:252] = 00 // Reserved
  [251:248] = 0000 // Direction = SINK

### 20.6.2	Source Descriptor (Memory to Network)

Hex representation (LSB to MSB):
[63:0]    = 0x0000_0002_0000_0000  // src_addr = 0x2_0000_0000
[127:64]  = 0x0000_0000_0000_0000  // dest_addr (unused)
[191:128] = 0x0000_0000_0000_0000  // next_ptr = 0 (last)
[255:192] = 0x4001_0080_0000_0000  // last=0, irq=1, dir=SOURCE, len=128

Binary control field:
  [255] = 0      // Not last (but next_ptr=0 terminates)
  [254] = 1      // Interrupt enabled
  [253:252] = 00 // Reserved
  [251:248] = 0001 // Direction = SOURCE

## 20.7	Software Construction

### 20.7.1	C Structure Example

typedef struct __attribute__((packed, aligned(32))) {
    uint64_t src_addr;      // [63:0]
    uint64_t dest_addr;     // [127:64]
    uint64_t next_ptr;      // [191:128]
    uint16_t reserved1;     // [207:192]
    uint16_t length;        // [223:208] - Note: shifted in actual layout
    uint8_t  reserved2;     // [231:224]
    uint8_t  direction;     // [239:232] - lower 4 bits only
    uint8_t  reserved3;     // [247:240]
    uint8_t  control;       // [255:248] - last, irq_en, etc.
} rapids_descriptor_t;

// Helper macros
#define DESC_CTRL_LAST    (1 << 7)
#define DESC_CTRL_IRQ_EN  (1 << 6)
#define DESC_DIR_SINK     0x0
#define DESC_DIR_SOURCE   0x1

### 20.7.2	Descriptor Ring Setup

// Allocate descriptor ring (must be 32-byte aligned)
rapids_descriptor_t *desc_ring = aligned_alloc(32, NUM_DESC * sizeof(rapids_descriptor_t));

// Initialize chain
for (int i = 0; i < NUM_DESC - 1; i++) {
    desc_ring[i].next_ptr = (uint64_t)&desc_ring[i + 1];
    desc_ring[i].control = DESC_CTRL_IRQ_EN;
}

// Last descriptor
desc_ring[NUM_DESC - 1].next_ptr = 0;
desc_ring[NUM_DESC - 1].control = DESC_CTRL_LAST | DESC_CTRL_IRQ_EN;

# 21	Register Map

## 21.1	Overview

RAPIDS Beats provides a memory-mapped register interface for configuration and status. Registers are accessed via the APB interface and organized by function.

## 21.2	Register Summary

### 21.2.1	Global Registers (Offset 0x000-0x0FF)

Global Registers

| Offset | Name | Width | Access | Description |
| --- | --- | --- | --- | --- |
| 0x000 | VERSION | 32 | RO | IP version register |
| 0x004 | CONFIG | 32 | RO | Configuration parameters |
| 0x008 | GLOBAL_CTRL | 32 | RW | Global control |
| 0x00C | GLOBAL_STATUS | 32 | RO | Global status |
| 0x010 | IRQ_STATUS | 32 | RW1C | Interrupt status |
| 0x014 | IRQ_ENABLE | 32 | RW | Interrupt enable |
| 0x018 | IRQ_FORCE | 32 | WO | Force interrupt (debug) |

### 21.2.2	Per-Channel Registers (Offset 0x100 + ch*0x40)

Per-Channel Registers

| Offset | Name | Width | Access | Description |
| --- | --- | --- | --- | --- |
| +0x00 | CH_CTRL | 32 | RW | Channel control |
| +0x04 | CH_STATUS | 32 | RO | Channel status |
| +0x08 | DESC_PTR_LO | 32 | RW | Descriptor pointer [31:0] |
| +0x0C | DESC_PTR_HI | 32 | RW | Descriptor pointer [63:32] |
| +0x10 | XFER_COUNT | 32 | RO | Beats transferred |
| +0x14 | ERR_STATUS | 32 | RW1C | Error status |
| +0x18 | CH_CONFIG | 32 | RW | Channel configuration |

## 21.3	Register Descriptions

### 21.3.1	VERSION (0x000) - Read Only

VERSION Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31:24] | major | 0x01 | Major version |
| [23:16] | minor | 0x00 | Minor version |
| [15:0] | patch | 0x0000 | Patch level |

### 21.3.2	CONFIG (0x004) - Read Only

CONFIG Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31:24] | num_channels | 0x08 | Number of channels |
| [23:16] | data_width | 0x40 | Data width / 8 (64 = 512-bit) |
| [15:8] | sram_depth | varies | SRAM depth / 16 |
| [7:0] | features | varies | Feature bits |

### 21.3.3	GLOBAL_CTRL (0x008) - Read/Write

GLOBAL_CTRL Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31] | soft_reset | 0 | Write 1 to reset all channels |
| [30:8] | Reserved | 0 | Reserved |
| [7:0] | clock_gate_en | 0xFF | Per-channel clock gate enable |

### 21.3.4	GLOBAL_STATUS (0x00C) - Read Only

GLOBAL_STATUS Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31:24] | Reserved | 0 | Reserved |
| [23:16] | error_channels | 0 | Channels with errors |
| [15:8] | active_channels | 0 | Channels in transfer |
| [7:0] | idle_channels | 0xFF | Channels idle |

### 21.3.5	IRQ_STATUS (0x010) - Read/Write-1-to-Clear

IRQ_STATUS Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31:24] | ch_error | 0 | Channel error interrupts |
| [23:16] | Reserved | 0 | Reserved |
| [15:8] | ch_complete | 0 | Channel completion interrupts |
| [7:0] | Reserved | 0 | Reserved |

### 21.3.6	CH_CTRL (0x100 + ch*0x40) - Read/Write

CH_CTRL Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31] | enable | 0 | Channel enable |
| [30] | kick | 0 | Write 1 to start (self-clearing) |
| [29] | abort | 0 | Write 1 to abort (self-clearing) |
| [28] | soft_reset | 0 | Write 1 to reset channel |
| [27:0] | Reserved | 0 | Reserved |

### 21.3.7	CH_STATUS (0x104 + ch*0x40) - Read Only

CH_STATUS Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31:28] | state | 0 | FSM state encoding |
| [27:24] | Reserved | 0 | Reserved |
| [23:16] | desc_count | 0 | Descriptors processed |
| [15:8] | sram_level | 0 | Current SRAM fill level |
| [7:0] | flags | 0 | Status flags |

State Encoding:

CH_STATUS State Encoding

| Value | State |
| --- | --- |
| 0x0 | IDLE |
| 0x1 | WAIT_DESC |
| 0x2 | PARSE_DESC |
| 0x3 | XFER_DATA |
| 0x4 | CHECK_NEXT |
| 0x5 | COMPLETE |
| 0xE | ERROR |
| 0xF | RESET |

### 21.3.8	ERR_STATUS (0x114 + ch*0x40) - Read/Write-1-to-Clear

ERR_STATUS Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [31:28] | axi_resp | 0 | AXI error response code |
| [27:24] | error_type | 0 | Error type encoding |
| [23:16] | error_desc_idx | 0 | Descriptor index at error |
| [15:0] | Reserved | 0 | Reserved |

## 21.4	Register Access Timing

Register Read

Register Read

Source: register_read.json

Waveform 31

Register Write

Register Write

Source: register_write.json

Waveform 32

## 21.5	Channel Kick Sequence

Writing to the kick bit triggers descriptor processing:

Waveform 33

## 21.6	Address Decoding

### 21.6.1	Address Space Layout

0x000 - 0x0FF: Global registers
0x100 - 0x13F: Channel 0 registers
0x140 - 0x17F: Channel 1 registers
0x180 - 0x1BF: Channel 2 registers
0x1C0 - 0x1FF: Channel 3 registers
0x200 - 0x23F: Channel 4 registers
0x240 - 0x27F: Channel 5 registers
0x280 - 0x2BF: Channel 6 registers
0x2C0 - 0x2FF: Channel 7 registers
0x300 - 0x3FF: Reserved

### 21.6.2	Channel Address Formula

channel_base = 0x100 + (channel_number * 0x40)
register_addr = channel_base + register_offset

Example: Channel 3 CH_STATUS = 0x100 + (3 * 0x40) + 0x04 = 0x1C4

# 22	Initialization Sequence

## 22.1	Overview

This section describes the required steps to initialize RAPIDS Beats and start DMA transfers. The initialization sequence ensures proper hardware state before beginning operations.

## 22.2	Boot Sequence

Initialization Flow

Initialization Flow

Source: 14_init_flow.mmd

Diagram 27

## 22.3	Detailed Steps

### 22.3.1	Step 1: Verify Hardware Presence

// Read VERSION register
uint32_t version = read_reg(RAPIDS_BASE + VERSION);
uint8_t major = (version >> 24) & 0xFF;
uint8_t minor = (version >> 16) & 0xFF;

if (major < 1) {
    printf("ERROR: RAPIDS not detected or incompatible version\n");
    return -1;
}

printf("RAPIDS Beats v%d.%d detected\n", major, minor);

### 22.3.2	Step 2: Read Configuration

// Read CONFIG register
uint32_t config = read_reg(RAPIDS_BASE + CONFIG);
uint8_t num_channels = (config >> 24) & 0xFF;
uint8_t data_width = ((config >> 16) & 0xFF) * 8;  // Convert to bits
uint8_t sram_depth = ((config >> 8) & 0xFF) * 16;

printf("Channels: %d, Data Width: %d bits, SRAM: %d entries\n",
       num_channels, data_width, sram_depth);

### 22.3.3	Step 3: Global Reset

// Perform soft reset to ensure clean state
write_reg(RAPIDS_BASE + GLOBAL_CTRL, GLOBAL_CTRL_SOFT_RESET);

// Wait for reset completion
while (read_reg(RAPIDS_BASE + GLOBAL_STATUS) & GLOBAL_STATUS_RESET_ACTIVE) {
    // Poll until reset complete
}

// Clear any pending interrupts
write_reg(RAPIDS_BASE + IRQ_STATUS, 0xFFFFFFFF);

### 22.3.4	Step 4: Allocate Resources

// Allocate descriptor ring (must be 32-byte aligned)
size_t desc_size = NUM_DESCRIPTORS * sizeof(rapids_descriptor_t);
rapids_descriptor_t *desc_ring = aligned_alloc(32, desc_size);
if (!desc_ring) {
    return -ENOMEM;
}

// Allocate data buffers (must be 64-byte aligned for 512-bit AXI)
size_t buf_size = BUFFER_SIZE_BYTES;
void *rx_buffer = aligned_alloc(64, buf_size);
void *tx_buffer = aligned_alloc(64, buf_size);

### 22.3.5	Step 5: Initialize Descriptors

// Initialize sink descriptor chain
for (int i = 0; i < NUM_RX_DESC - 1; i++) {
    desc_ring[i].dest_addr = (uint64_t)(rx_buffer + i * FRAME_SIZE);
    desc_ring[i].src_addr = 0;
    desc_ring[i].next_ptr = (uint64_t)&desc_ring[i + 1];
    desc_ring[i].direction = DESC_DIR_SINK;
    desc_ring[i].length = FRAME_SIZE / 64;  // Convert to beats
    desc_ring[i].control = DESC_CTRL_IRQ_EN;
}

// Last descriptor
desc_ring[NUM_RX_DESC - 1].dest_addr = (uint64_t)(rx_buffer + (NUM_RX_DESC-1) * FRAME_SIZE);
desc_ring[NUM_RX_DESC - 1].next_ptr = 0;
desc_ring[NUM_RX_DESC - 1].control = DESC_CTRL_LAST | DESC_CTRL_IRQ_EN;

// Ensure descriptors are visible to hardware
__sync_synchronize();  // Memory barrier

### 22.3.6	Step 6: Configure Channels

// Configure channel 0 for sink (receive)
uint32_t ch0_base = RAPIDS_BASE + 0x100;

// Set descriptor pointer
write_reg(ch0_base + DESC_PTR_LO, (uint32_t)((uint64_t)desc_ring & 0xFFFFFFFF));
write_reg(ch0_base + DESC_PTR_HI, (uint32_t)((uint64_t)desc_ring >> 32));

// Configure channel
write_reg(ch0_base + CH_CONFIG, CH_CONFIG_DEFAULT);

### 22.3.7	Step 7: Enable Interrupts

// Enable completion and error interrupts for channel 0
write_reg(RAPIDS_BASE + IRQ_ENABLE, IRQ_CH0_COMPLETE | IRQ_CH0_ERROR);

### 22.3.8	Step 8: Enable Channel

// Enable channel (but don't kick yet)
write_reg(ch0_base + CH_CTRL, CH_CTRL_ENABLE);

## 22.4	Initialization Timing

Initialization Timing

Initialization Timing

Source: init_timing.json

Waveform 34

## 22.5	Starting Transfers

### 22.5.1	Kick a Channel

After initialization, kick the channel to start processing:

// Kick channel 0 to start descriptor processing
write_reg(ch0_base + CH_CTRL, CH_CTRL_ENABLE | CH_CTRL_KICK);

Kick Timing

Kick Timing

Waveform 35

## 22.6	Interrupt Handling

### 22.6.1	Completion Interrupt

void rapids_irq_handler(void) {
    uint32_t irq_status = read_reg(RAPIDS_BASE + IRQ_STATUS);

    // Handle channel 0 completion
    if (irq_status & IRQ_CH0_COMPLETE) {
        // Read channel status
        uint32_t ch_status = read_reg(ch0_base + CH_STATUS);
        uint8_t desc_count = (ch_status >> 16) & 0xFF;

        // Process completed buffers
        process_rx_buffers(desc_count);

        // Clear interrupt
        write_reg(RAPIDS_BASE + IRQ_STATUS, IRQ_CH0_COMPLETE);

        // Re-kick channel for continuous operation (if desired)
        write_reg(ch0_base + CH_CTRL, CH_CTRL_ENABLE | CH_CTRL_KICK);
    }

    // Handle errors
    if (irq_status & IRQ_CH0_ERROR) {
        uint32_t err_status = read_reg(ch0_base + ERR_STATUS);
        handle_error(err_status);

        // Clear error interrupt
        write_reg(RAPIDS_BASE + IRQ_STATUS, IRQ_CH0_ERROR);
    }
}

## 22.7	Reset and Recovery

### 22.7.1	Per-Channel Reset

void reset_channel(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);

    // Soft reset the channel
    write_reg(ch_base + CH_CTRL, CH_CTRL_SOFT_RESET);

    // Wait for reset complete
    while ((read_reg(ch_base + CH_STATUS) >> 28) == 0xF) {
        // State = RESET
    }

    // Clear error status
    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);

    // Re-initialize descriptor pointer
    write_reg(ch_base + DESC_PTR_LO, saved_desc_ptr_lo[channel]);
    write_reg(ch_base + DESC_PTR_HI, saved_desc_ptr_hi[channel]);

    // Re-enable
    write_reg(ch_base + CH_CTRL, CH_CTRL_ENABLE);
}

### 22.7.2	Global Reset

void reset_rapids(void) {
    // Disable all interrupts
    write_reg(RAPIDS_BASE + IRQ_ENABLE, 0);

    // Global soft reset
    write_reg(RAPIDS_BASE + GLOBAL_CTRL, GLOBAL_CTRL_SOFT_RESET);

    // Wait for all channels idle
    while (read_reg(RAPIDS_BASE + GLOBAL_STATUS) & 0x00FF0000) {
        // Wait for active channels to clear
    }

    // Clear all interrupts
    write_reg(RAPIDS_BASE + IRQ_STATUS, 0xFFFFFFFF);

    // Re-run full initialization
    rapids_init();
}

# 23	Error Handling

## 23.1	Overview

RAPIDS Beats implements comprehensive error detection and reporting. This section describes error types, detection mechanisms, and recommended recovery procedures.

## 23.2	Error Categories

Error Hierarchy

Error Hierarchy

Source: 15_error_hierarchy.mmd

Diagram 28

## 23.3	Error Detection

### 23.3.1	AXI Response Errors

RAPIDS monitors all AXI transactions for error responses:

AXI Response Codes

| Response | Code | Description | Typical Cause |
| --- | --- | --- | --- |
| OKAY | 2’b00 | Normal success | – |
| EXOKAY | 2’b01 | Exclusive OK | – |
| SLVERR | 2’b10 | Slave error | Memory protection, invalid access |
| DECERR | 2’b11 | Decode error | Address not mapped |

AXI Error Detection

AXI Error Detection

Source: axi_error.json

Waveform 36

### 23.3.2	Descriptor Validation Errors

Descriptors are validated when parsed:

Descriptor Validation Errors

| Check | Error Condition | ERR_STATUS Code |
| --- | --- | --- |
| Address range | Address outside configured range | 0x10 |
| Alignment | Address not properly aligned | 0x11 |
| Length | Length = 0 or > max allowed | 0x12 |
| Direction | Invalid direction code | 0x13 |

Waveform 37

### 23.3.3	Timeout Detection

RAPIDS implements watchdog timers for detecting stalled operations:

Timeout Thresholds

| Timeout Type | Typical Value | Condition |
| --- | --- | --- |
| Descriptor fetch | 1024 cycles | No AXI response |
| Transfer | 65536 cycles | No progress |
| AXIS receive | 4096 cycles | TVALID stuck low |
| AXIS transmit | 4096 cycles | TREADY stuck low |

Waveform 38

## 23.4	Error Reporting

### 23.4.1	ERR_STATUS Register

When an error occurs, details are captured in the ERR_STATUS register:

ERR_STATUS [31:0]:
  [31:28] - AXI response code (BRESP or RRESP)
  [27:24] - Error type:
            0x0 = AXI write error
            0x1 = AXI read error
            0x2 = Descriptor fetch error
            0x3 = Descriptor validation error
            0x4 = Timeout error
  [23:16] - Descriptor index when error occurred
  [15:0]  - Reserved

### 23.4.2	MonBus Error Events

Errors generate MonBus packets for system-wide visibility:

Waveform 39

### 23.4.3	Interrupt Generation

Error interrupts are generated when enabled:

Diagram 29

## 23.5	Error Recovery

### 23.5.1	Recovery Flow

Error Recovery

Error Recovery

Source: 16_error_recovery.mmd

Diagram 30

### 23.5.2	AXI Error Recovery

void handle_axi_error(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);
    uint32_t err_status = read_reg(ch_base + ERR_STATUS);

    uint8_t axi_resp = (err_status >> 28) & 0xF;
    uint8_t err_type = (err_status >> 24) & 0xF;
    uint8_t desc_idx = (err_status >> 16) & 0xFF;

    if (axi_resp == 0x3) {  // DECERR
        printf("CH%d: Address decode error at descriptor %d\n", channel, desc_idx);
        // Check memory mapping, descriptor addresses
    } else if (axi_resp == 0x2) {  // SLVERR
        printf("CH%d: Slave error at descriptor %d\n", channel, desc_idx);
        // Check memory protection, peripheral status
    }

    // Clear error and reset channel
    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);
    reset_channel(channel);
}

### 23.5.3	Descriptor Error Recovery

void handle_desc_error(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);
    uint32_t err_status = read_reg(ch_base + ERR_STATUS);

    uint8_t err_type = (err_status >> 24) & 0xF;
    uint8_t desc_idx = (err_status >> 16) & 0xFF;

    switch (err_type & 0xF) {
        case 0x0:  // Address range
            printf("CH%d: Descriptor %d has invalid address\n", channel, desc_idx);
            fix_descriptor_address(channel, desc_idx);
            break;
        case 0x1:  // Alignment
            printf("CH%d: Descriptor %d has alignment issue\n", channel, desc_idx);
            fix_descriptor_alignment(channel, desc_idx);
            break;
        case 0x2:  // Length
            printf("CH%d: Descriptor %d has invalid length\n", channel, desc_idx);
            fix_descriptor_length(channel, desc_idx);
            break;
    }

    // Clear and restart from fixed descriptor
    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);
    write_reg(ch_base + DESC_PTR_LO, (uint32_t)&desc_ring[desc_idx]);
    write_reg(ch_base + DESC_PTR_HI, (uint32_t)((uint64_t)&desc_ring[desc_idx] >> 32));
    reset_channel(channel);
}

### 23.5.4	Timeout Recovery

void handle_timeout(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);
    uint32_t err_status = read_reg(ch_base + ERR_STATUS);
    uint32_t ch_status = read_reg(ch_base + CH_STATUS);

    uint8_t desc_idx = (err_status >> 16) & 0xFF;
    uint8_t sram_level = (ch_status >> 8) & 0xFF;

    printf("CH%d: Timeout at descriptor %d, SRAM level %d\n",
           channel, desc_idx, sram_level);

    // Check external interfaces
    if (sram_level == 0) {
        printf("  SRAM empty - check network input\n");
    } else if (sram_level > 0) {
        printf("  SRAM has data - check memory interface\n");
    }

    // Abort and reset
    write_reg(ch_base + CH_CTRL, CH_CTRL_ABORT);
    while ((read_reg(ch_base + CH_STATUS) >> 28) != 0x0) {
        // Wait for IDLE
    }

    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);
    reset_channel(channel);
}

## 23.6	Error Prevention

### 23.6.1	Best Practices

Error Prevention Best Practices

| Practice | Rationale |
| --- | --- |
| Validate descriptors before submission | Catch errors before hardware processing |
| Use aligned addresses | Avoid alignment errors |
| Set reasonable timeouts | Detect issues without false positives |
| Monitor SRAM levels | Identify flow control issues |
| Enable MonBus | System-wide error visibility |

### 23.6.2	Descriptor Validation Example

int validate_descriptor(rapids_descriptor_t *desc) {
    // Check alignment
    if (desc->dest_addr & 0x3F) {  // 64-byte alignment
        return ERR_ALIGNMENT;
    }

    // Check length
    if (desc->length == 0 || desc->length > MAX_BEATS) {
        return ERR_LENGTH;
    }

    // Check direction
    if (desc->direction > DESC_DIR_SOURCE) {
        return ERR_DIRECTION;
    }

    // Check address range
    if (desc->dest_addr < VALID_ADDR_MIN || desc->dest_addr > VALID_ADDR_MAX) {
        return ERR_ADDRESS;
    }

    return 0;  // Valid
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	RAPIDS Beats Architecture MAS

Version: 0.25 Date: 2025-01-10 Purpose: Module Architecture Specification for RAPIDS Phase 1 “Beats” Architecture

## 24.1	Document Organization

Note: All chapters linked below for automated document generation.

### 24.1.1	Front Matter

- Document Information

### 24.1.2	Chapter 1: Overview

- Architecture Overview

- Top-Level Port List

- Clocks and Reset

### 24.1.3	Chapter 2: FUB (Functional Unit Blocks)

Control Path: - Scheduler - Descriptor Engine

AXI Engines: - AXI Read Engine - AXI Write Engine

Flow Control: - Beats Alloc Control - Beats Drain Control - Beats Latency Bridge

### 24.1.4	Chapter 3: Macro (Integration Blocks)

Scheduler Integration: - Beats Scheduler Group - Beats Scheduler Group Array

Sink Data Path (Network to Memory): - Sink Data Path - Sink Data Path AXIS - Sink SRAM Controller - Sink SRAM Controller Unit

Source Data Path (Memory to Network): - Source Data Path - Source Data Path AXIS - Source SRAM Controller - Source SRAM Controller Unit

Top-Level Integration: - RAPIDS Core Beats

### 24.1.5	Chapter 4: Interfaces

- Interface Overview

- AXI4 Interface Specification

- AXIS Interface Specification

- MonBus Interface Specification

## 24.2	Quick Reference

### 24.2.1	FUB Modules (fub_beats/)

FUB Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler | scheduler.sv | Transfer coordinator, descriptor processing | Implemented |
| descriptor_engine | descriptor_engine.sv | Descriptor fetch/parse (256-bit) | Implemented |
| axi_read_engine | axi_read_engine.sv | AXI read master, streaming pipeline | Implemented |
| axi_write_engine | axi_write_engine.sv | AXI write master, streaming pipeline | Implemented |
| beats_alloc_ctrl | beats_alloc_ctrl.sv | Space allocation tracking (virtual FIFO) | Implemented |
| beats_drain_ctrl | beats_drain_ctrl.sv | Data availability tracking (virtual FIFO) | Implemented |
| beats_latency_bridge | beats_latency_bridge.sv | Latency compensation, backpressure management | Implemented |

### 24.2.2	Macro Modules (macro_beats/)

Macro Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| beats_scheduler_group | beats_scheduler_group.sv | Scheduler + Descriptor Engine wrapper | Implemented |
| beats_scheduler_group_array | beats_scheduler_group_array.sv | 8-channel scheduler array with arbitration | Implemented |
| sink_data_path | sink_data_path.sv | Network-to-memory path integration | Implemented |
| sink_data_path_axis | sink_data_path_axis.sv | AXIS variant of sink path | Implemented |
| source_data_path | source_data_path.sv | Memory-to-network path integration | Implemented |
| source_data_path_axis | source_data_path_axis.sv | AXIS variant of source path | Implemented |
| snk_sram_controller | snk_sram_controller.sv | Sink SRAM buffer management | Implemented |
| snk_sram_controller_unit | snk_sram_controller_unit.sv | Per-channel sink SRAM unit | Implemented |
| src_sram_controller | src_sram_controller.sv | Source SRAM buffer management | Implemented |
| src_sram_controller_unit | src_sram_controller_unit.sv | Per-channel source SRAM unit | Implemented |
| rapids_core_beats | rapids_core_beats.sv | Top-level RAPIDS integration | Implemented |

## 24.3	Architecture Comparison: RAPIDS vs STREAM

The “beats” architecture is a Phase 1 implementation of RAPIDS that shares concepts with STREAM but targets network-to-memory (and vice versa) transfers rather than memory-to-memory.

RAPIDS Beats vs STREAM Comparison

| Feature | STREAM | RAPIDS Beats |
| --- | --- | --- |
| Primary Use | Memory-to-memory DMA | Network-to-memory accelerator |
| Data Paths | Single bidirectional | Separate sink/source paths |
| Network Interface | None | AXIS master/slave |
| SRAM Buffering | Shared buffer | Separate sink/source buffers |
| Descriptor Format | 256-bit | 256-bit (compatible) |
| Channel Count | 8 | 8 |
| Flow Control | alloc_ctrl/drain_ctrl | beats_alloc_ctrl/beats_drain_ctrl |

## 24.4	Clock and Reset Summary

### 24.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | Primary - all RAPIDS logic, AXI/AXIS interfaces |

### 24.4.2	Reset Signals

Reset Signals

| Reset | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary - all RAPIDS logic |

See: Clocks and Reset for complete timing specifications

## 24.5	Interface Summary

### 24.5.1	External Interfaces

External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXI4 (Descriptor) | Master | 256-bit | Descriptor fetch |
| AXI4 (Sink Write) | Master | 512-bit (param) | Sink data write to memory |
| AXI4 (Source Read) | Master | 512-bit (param) | Source data read from memory |
| AXIS (Sink) | Slave | 512-bit (param) | Network data ingress |
| AXIS (Source) | Master | 512-bit (param) | Network data egress |
| MonBus | Master | 64-bit | Monitor packet output |

### 24.5.2	Internal Buses

Internal Buses

| Interface | Width | Purpose |
| --- | --- | --- |
| MonBus | 64-bit | Internal monitoring bus |
| Descriptor Bus | 256-bit | Descriptor distribution |
| SRAM Data Bus | 512-bit | SRAM read/write data |

## 24.6	Related Documentation

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- RAPIDS Spec - Original RAPIDS specification

## 24.7	Specification Conventions

### 24.7.1	Signal Naming

- Clock: aclk

- Reset: aresetn (active-low)

- Valid/Ready: Standard AXI/AXIS handshake

- Registers: r_ prefix (e.g., r_state, r_counter)

- Wires: w_ prefix (e.g., w_next_state, w_grant)

### 24.7.2	Parameter Naming

- Uppercase: NUM_CHANNELS, DATA_WIDTH, ADDR_WIDTH

- Derived: CHAN_WIDTH = $clog2(NUM_CHANNELS)

### 24.7.3	Figure and Table Numbering

- Figures: Figure X.Y.Z: Title where X=chapter, Y=section, Z=figure number

- Tables: Caption line : Table Title after table markdown

Last Updated: 2025-01-10 Maintained By: RAPIDS Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Product Requirements Document (PRD)

## 25.1	RAPIDS - Rapid AXI Programmable In-band Descriptor System

Version: 1.0 Date: 2025-09-30 Status: Active Development - Validation in Progress Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 25.2	1. Executive Summary

The Rapid AXI Programmable In-band Descriptor System (RAPIDS) is a custom hardware accelerator designed for efficient memory-to-memory data movement with network interface integration. It demonstrates complex FSM coordination, descriptor-based DMA operations, and comprehensive monitoring capabilities.

### 25.2.1	1.1 Quick Stats

- Modules: 17 SystemVerilog files

- Architecture: 3 major blocks (Scheduler, Sink, Source)

- Interfaces: AXI4 (memory), Network (network), AXIL4 (control), MonBus (monitoring)

- Test Coverage: ~80% functional (basic scenarios validated)

- Status: Active validation, known issues documented

### 25.2.2	1.2 Subsystem Goals

- Primary: Demonstrate complex accelerator design patterns

- Secondary: Provide DMA-style memory transfer capability

- Tertiary: Educational reference for descriptor-based engines

## 25.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 25.3.1	📚 Complete RAPIDS Specification

Location: projects/components/rapids/docs/rapids_spec/

- Index - Complete specification structure

#### 25.3.1.1	Chapter 1: Overview

- Overview

- Architecture Requirements

- Clocking and Reset

- Acronyms

- References

#### 25.3.1.2	Chapter 2: Block Specifications

- Blocks Overview

Scheduler Group: - Scheduler Group - Scheduler - Task management FSM - Descriptor Engine - Descriptor parsing - Program Engine - Program sequencing

Sink Data Path (Network → Memory): - Sink Data Path - Network Slave - Network ingress - Sink SRAM Control - Buffer management - Sink AXI Write Engine - Memory writes

Source Data Path (Memory → Network): - Source Data Path - Network Master - Network egress - Source SRAM Control - Buffer management - Source AXI Read Engine - Memory reads

Monitoring: - MonBus AXIL Group - Control/status - FSM Summary - State machine overview

#### 25.3.1.3	Chapter 3: Interfaces

- Top-Level Ports

- AXIL4 Interface

- AXI4 Interface

- Network Interface

- MonBus Interface

#### 25.3.1.4	Chapter 4 & 5: Programming

- Programming Model

- Register Definitions

### 25.3.2	🐛 Known Issues

Location: projects/components/rapids/known_issues/

- Scheduler - Credit counter initialization bug

- Sink Data Path - Minor issues

- Sink SRAM Control - Edge cases

### 25.3.3	📖 Other Documentation

- README - Quick start and integration guide

- CLAUDE - AI assistance guide for this subsystem

- TASKS - Current work items (to be created)

- Validation Report - Test results

## 25.4	2.4 Organizational Standards - RAPIDS Code Location

⚠ MANDATORY: All RAPIDS-specific code must be in the project area ⚠

### 25.4.1	Code Organization Principle

“All RAPIDS-specific verification code MUST reside in projects/components/rapids/dv/ for easy discovery.”

This subsystem follows the repository-wide organizational standard (see /PRD.md Section 2.3) requiring all project-specific code to be located in the project area, NOT the framework area.

### 25.4.2	RAPIDS Directory Structure

projects/components/rapids/
├── rtl/                          # RTL source code
│   ├── includes/                 # RAPIDS packages (rapids_pkg.sv)
│   ├── rapids_fub/               # Functional unit blocks
│   └── rapids_macro/             # Top-level integration
│
└── dv/                           # Design verification (all RAPIDS-specific)
    ├── tbclasses/                # ★ RAPIDS TB classes HERE (not framework!)
    │   ├── scheduler_tb.py       # Scheduler testbench class
    │   ├── descriptor_engine_tb.py
    │   ├── program_engine_tb.py
    │   └── rapids_integration_tb.py
    │
    ├── components/               # ★ RAPIDS-specific BFMs
    │   └── (project-specific components if needed)
    │
    ├── scoreboards/              # ★ RAPIDS-specific scoreboards
    │   └── (verification scoreboards)
    │
    └── tests/                    # Test runners (import TB classes)
        ├── fub_tests/            # Functional unit block tests
        │   ├── scheduler/
        │   │   └── test_scheduler.py
        │   ├── descriptor_engine/
        │   └── program_engine/
        ├── integration_tests/    # Multi-block scenarios
        └── system_tests/         # Full RAPIDS operation

### 25.4.3	What Goes Where?

| Code Type | ✓ CORRECT Location | ✗ WRONG Location |
| --- | --- | --- |
| RAPIDS TB Classes | projects/components/rapids/dv/tbclasses/ | bin/TBClasses/rapids/ |
| RAPIDS-Specific BFMs | projects/components/rapids/dv/components/ | bin/TBClasses/components/rapids/ |
| RAPIDS Scoreboards | projects/components/rapids/dv/scoreboards/ | bin/TBClasses/scoreboards/rapids/ |
| Test Runners | projects/components/rapids/dv/tests/ | Anywhere else |
| Shared AXI4/APB BFMs | bin/TBClasses/components/{protocol}/ | Project area |

### 25.4.4	Import Pattern for RAPIDS Tests

✓ CORRECT - Import from Project Area:

# Import framework utilities (PYTHONPATH includes bin/)
import os, sys
from TBClasses.shared.utilities import get_repo_root
from TBClasses.shared.tbbase import TBBase

# Add repo root to Python path using robust git-based method
repo_root = get_repo_root()
sys.path.insert(0, repo_root)

# Import RAPIDS TB classes from PROJECT AREA
from projects.components.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from projects.components.rapids.dv.tbclasses.descriptor_engine_tb import DescriptorEngineTB

# Shared framework components
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

✗ WRONG - Don’t Import from Framework:

# DON'T DO THIS!
from TBClasses.rapids.scheduler_tb import SchedulerTB  # ✗ WRONG!

### 25.4.5	Benefits of This Organization

- Easy Discovery - All RAPIDS code in ONE place: projects/components/rapids/

- Clear Ownership - RAPIDS team owns their dv/ area completely

- No Confusion - Never wonder “where does this TB class live?”

- Maintainability - Changes isolated to RAPIDS area don’t affect other projects

- Framework Stays Clean - Only truly shared cross-project code in framework

### 25.4.6	Compliance Status

✓ RAPIDS is now compliant - All TB classes moved to project area as of 2025-10-18

Migration History: - Before: TB classes incorrectly in bin/TBClasses/rapids/ - After: TB classes correctly in projects/components/rapids/dv/tbclasses/ - Test Imports: Updated to import from project area

📖 Complete Documentation: See /PRD.md Section 2.3 for repository-wide organizational standards.

## 25.5	3. Architecture Overview

### 25.5.1	3.1 Top-Level Block Diagram

RAPIDS (Rapid AXI Programmable In-band Descriptor System)
├── Scheduler Group
│   ├── Scheduler          (Task FSM, credit management)
│   ├── Descriptor Engine  (Descriptor FIFO, parsing)
│   └── Program Engine     (Program sequencing, alignment)
│
├── Sink Data Path (Network → SRAM → System Memory)
│   ├── Network Slave        (Network ingress interface)
│   ├── Sink SRAM Control (Buffering, flow control)
│   └── Sink AXI Writer   (AXI4 write to system memory)
│
├── Source Data Path (System Memory → SRAM → Network)
│   ├── Source AXI Reader (AXI4 read from system memory)
│   ├── Source SRAM Control (Buffering, flow control)
│   └── Network Master       (Network egress interface)
│
└── MonBus AXIL Group
    ├── AXIL4 Slave       (Control/status registers)
    └── MonBus Reporter   (Monitor packet generation)

📖 See: rapids_spec/ch02_blocks/00_overview.md for detailed architecture

### 25.5.2	3.2 Data Flow

Sink Path (Receive): 1. Network packets arrive via Network Slave 2. Buffered in Sink SRAM 3. DMA’d to system memory via AXI4 Write Engine 4. Completion reported via MonBus

Source Path (Transmit): 1. Descriptor specifies data location in system memory 2. Source AXI Reader fetches data to Source SRAM 3. Network Master transmits to network 4. Completion reported via MonBus

Scheduler Coordination: - Parses descriptors from Descriptor Engine - Manages credit-based flow control - Sequences program engine operations - Coordinates sink/source data paths

## 25.6	4. Key Features

### 25.6.1	4.1 Descriptor-Based Operation

| Feature | Status | Description |
| --- | --- | --- |
| Descriptor FIFO | ✓ | Queued descriptor processing |
| Multi-field parsing | ✓ | Address, length, control fields |
| Chained descriptors | ⏳ | Future enhancement |
| Completion reporting | ✓ | Via MonBus packets |

### 25.6.2	4.2 Data Path Features

| Feature | Status | Description |
| --- | --- | --- |
| SRAM buffering | ✓ | Decouple network from memory |
| AXI4 burst support | ✓ | Efficient memory transfers |
| Backpressure handling | ✓ | Flow control on all interfaces |
| Data alignment | ✓ | Handle unaligned transfers |

### 25.6.3	4.3 Scheduler Features

| Feature | Status | Description |
| --- | --- | --- |
| Task FSM | ✓ | Multi-state coordination |
| Credit management | ✓ | Exponential encoding (0→1, 1→2, 2→4, …, 15→∞) |
| Program sequencing | ✓ | Coordinated operations |
| Error detection | ✓ | Timeout, overflow detection |

Credit Management Details: - Uses exponential credit encoding for compact configuration - 4-bit cfg_initial_credit decodes to actual credit counts: - 0 → 1 credit (2^0), 4 → 16 credits (2^4), 8 → 256 credits (2^8) - 15 → ∞ (unlimited credits, 0xFFFFFFFF) - Encoding applied at initialization; runtime operations are linear (increment/decrement by 1) - Provides wide range (1 to 16384) with minimal configuration overhead

📖 See: rapids_spec/ch02_blocks/01_01_scheduler.md for complete encoding table

### 25.6.4	4.4 Monitoring Integration

| Feature | Status | Description |
| --- | --- | --- |
| MonBus packets | ✓ | StandardAMBA 64-bit format |
| Descriptor events | ✓ | Start/complete reporting |
| Error events | ✓ | Timeout, overflow, underflow |
| Performance metrics | ⏳ | Future enhancement |

## 25.7	5. Interfaces

### 25.7.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXIL4 | Slave | 32-bit | Control/status registers |
| AXI4 (Sink) | Master | Configurable | Write to system memory |
| AXI4 (Source) | Master | Configurable | Read from system memory |
| Network (Sink) | Slave | Configurable | Network ingress |
| Network (Source) | Master | Configurable | Network egress |
| MonBus | Master | 64-bit | Monitor packet output |

📖 See: rapids_spec/ch03_interfaces/ for complete interface specs

### 25.7.2	5.2 Configuration Parameters

// Example RAPIDS instantiation parameters
miop_top #(
    .AXI_ADDR_WIDTH(32),
    .AXI_DATA_WIDTH(64),
    .Network_DATA_WIDTH(64),
    .SRAM_DEPTH(1024),
    .MAX_DESCRIPTORS(16)
) u_miop (
    .aclk               (clk),
    .aresetn            (rst_n),
    // AXIL4 control interface
    .s_axil_*           (...),
    // AXI4 memory interfaces
    .m_axi_sink_*       (...),
    .m_axi_source_*     (...),
    // Network network interfaces
    .s_network_*           (...),
    .m_network_*           (...),
    // MonBus output
    .monbus_pkt_*       (...)
);

## 25.8	6. Use Cases

### 25.8.1	6.1 DMA-Style Transfers

Scenario: Move data from network to system memory

Flow: 1. Software writes descriptor to Descriptor Engine 2. Scheduler parses descriptor, activates Sink path 3. Network packets arrive via Network Slave 4. Data buffered in Sink SRAM 5. AXI4 Write Engine DMAs to system memory 6. Completion packet on MonBus

### 25.8.2	6.2 Network Packet Processing

Scenario: Read data from memory, transmit to network

Flow: 1. Descriptor specifies source address, length 2. Source AXI Reader fetches data to SRAM 3. Network Master transmits to network 4. Completion/error reporting via MonBus

### 25.8.3	6.3 Custom Data Path Acceleration

Educational value: Shows how to build custom accelerators - Descriptor-based control - Multi-block FSM coordination - Buffering strategies - Error handling - Performance monitoring

## 25.9	7. Test Coverage

### 25.9.1	7.1 Current Status

Overall: ~85% functional coverage (basic scenarios validated, descriptor engine complete)

| Component | Test Coverage | Status |
| --- | --- | --- |
| Scheduler | ~95% | Credit encoding fixed and verified (43/43 tests passing) |
| Descriptor Engine | ✓ 100% | All tests passing (14/14 tests, 100% success rate) |
| Program Engine | ~85% | Alignment tested |
| Sink Data Path | ~75% | Basic flows working |
| Source Data Path | ~70% | Basic flows working |
| SRAM Controllers | ~80% | Buffer management tested |
| Integration | ~60% | More stress testing needed |

Test Location: projects/components/rapids/dv/tests/fub_tests/ and projects/components/rapids/dv/tests/integration_tests/

Recent Achievements: - ✓ Descriptor Engine (2025-10-13): Achieved 100% test pass rate using continuous background monitoring pattern - 14/14 tests passing across all test levels (basic, medium, full) - All test classes passing (APB_ONLY, MIXED) - All delay profiles passing (fast_producer, fast_consumer, fixed_delay, minimal_delay) - Applied continuous monitoring methodology for asynchronous output capture

📖 See: docs/RAPIDS_Validation_Status_Report.md for detailed results

### 25.9.2	7.2 Test Strategy

FUB (Functional Unit Block) Tests: - Individual block testing - Located in projects/components/rapids/dv/tests/fub_tests/ - Focus on module-level functionality

Integration Tests: - Multi-block scenarios - Located in projects/components/rapids/dv/tests/integration_tests/ - End-to-end data flow validation

System Tests: - Full RAPIDS operation - Located in projects/components/rapids/dv/tests/system_tests/ - Realistic traffic patterns

## 25.10	8. Known Issues Summary

### 25.10.1	8.1 Critical Issues

✓ Scheduler Credit Counter Initialization - FIXED (2025-10-11) - File: projects/components/rapids/rtl/rapids_fub/scheduler.sv:567-570 - Issue: Credit counter was initializing to 0 instead of using exponential encoding - Fix Applied: Implemented exponential credit encoding - Status: Fixed, awaiting test verification - Documentation: known_issues/scheduler.md

Fix Details:

// Previous (wrong):
r_descriptor_credit_counter <= 32'h0;

// Fixed - Exponential encoding:
// 0→1, 1→2, 2→4, 3→8, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Encoding Rationale: - Compact 4-bit configuration covers 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞) - Exponential encoding applied at initialization only; runtime operations are linear

### 25.10.2	8.2 Medium Priority Issues

Descriptor Engine Edge Cases: - Some stress test failures under high load - Edge case handling needs improvement - Priority: P2

SRAM Control Timing: - Rare timing issues in back-to-back operations - Priority: P2

📖 See: known_issues/ directory for complete issue tracking

## 25.11	9. Integration Guidelines

### 25.11.1	9.1 Quick Start

miop_top #(
    .AXI_ADDR_WIDTH(32),
    .AXI_DATA_WIDTH(64),
    .Network_DATA_WIDTH(64)
) u_miop (
    // Clocking & Reset
    .aclk               (system_clk),
    .aresetn            (system_rst_n),

    // AXIL4 Control (connect to control fabric)
    .s_axil_awaddr      (ctrl_awaddr),
    .s_axil_awvalid     (ctrl_awvalid),
    .s_axil_awready     (ctrl_awready),
    // ... other AXIL signals

    // AXI4 Memory (connect to memory controller)
    .m_axi_sink_awaddr  (mem_awaddr),
    .m_axi_sink_awvalid (mem_awvalid),
    // ... AXI write channel for sink
    // ... AXI read channel for source

    // Network Network (connect to network fabric)
    .s_network_tdata       (net_rx_data),
    .s_network_tvalid      (net_rx_valid),
    // ... Network slave (receive)
    // ... Network master (transmit)

    // MonBus Output (connect to monitor aggregator)
    .monbus_pkt_valid   (miop_mon_valid),
    .monbus_pkt_ready   (miop_mon_ready),
    .monbus_pkt_data    (miop_mon_data)
);

### 25.11.2	9.2 Configuration Steps

- Initialize via AXIL4:

  - Configure SRAM depths

  - Set timeout thresholds

  - Enable credit management (or disable if using workaround)

- Load Descriptors:

  - Write descriptors to Descriptor Engine FIFO

  - Each descriptor specifies: address, length, control bits

- Enable Operation:

  - Set enable bits via AXIL4 registers

  - Monitor MonBus for completion/error packets

📖 See: rapids_spec/ch04_programming_models/01_programming.md for register details

## 25.12	10. Development Status

### 25.12.1	10.1 Current Phase

Phase: Validation and Bug Fixing (In Progress)

- ✓ Core architecture implemented

- ✓ Basic functionality verified

- ✓ Scheduler credit counter bug fixed (exponential encoding implemented)

- ⏳ Credit management tests need verification (remove workarounds)

- ⏳ Stress testing ongoing

- ⏳ Edge case refinement

📖 See: TASKS.md for detailed work items

### 25.12.2	10.2 Roadmap

Near-Term (Q4 2025): - ✓ Fix scheduler credit counter bug (completed 2025-10-11) - ⏳ Verify credit management tests (remove workarounds) - ⏳ Complete descriptor engine stress testing - ⏳ Integration test suite expansion - ⏳ Performance benchmarking

Long-Term (2026+): - Chained descriptor support - Advanced error recovery - Performance optimizations - Multi-channel support

## 25.13	11. Performance Characteristics

### 25.13.1	11.1 Throughput

Target: Match network/memory interface bandwidth

Bottlenecks: - SRAM buffer size - AXI4 burst efficiency - Scheduler overhead

Optimization: - Increase SRAM depth for larger packets - Tune AXI4 burst parameters - Pipeline scheduler operations

### 25.13.2	11.2 Latency

Components: - Descriptor parsing: ~10 cycles - SRAM buffering: Configurable depth - AXI4 memory access: System dependent - End-to-end: Typically <100 cycles for small packets

### 25.13.3	11.3 Resource Utilization

Area: - Scheduler: ~2K LUTs - Each data path: ~3K LUTs - SRAM buffers: Configurable (dominant area) - Total: ~10K LUTs + SRAM

Power: - Clock gating opportunities in idle blocks - SRAM power depends on depth/width

## 25.14	12. Verification Infrastructure

### 25.14.1	12.1 Test Organization

Location: projects/components/rapids/dv/tests/

Structure:

projects/components/rapids/dv/tests/
├── fub_tests/              # Functional Unit Block tests
│   ├── scheduler/
│   ├── descriptor_engine/
│   ├── program_engine/
│   ├── network_interfaces/
│   └── simple_sram/
├── integration_tests/      # Multi-block scenarios
└── system_tests/           # Full RAPIDS operation

### 25.14.2	12.2 CocoTB Framework

Location: bin/TBClasses/rapids/

Components: - RAPIDS-specific drivers - Descriptor generators - Traffic patterns - Monitor checkers

📖 See: docs/markdown/TBClasses/ (once created)

### 25.14.3	12.2.1 MANDATORY: BFM Usage for FUB Tests

⚠ CRITICAL DESIGN REQUIREMENT ⚠

All RAPIDS FUB (Functional Unit Block) level tests MUST use CocoTB Framework BFMs. Manual handshake driving is NOT allowed.

Required BFM Components:

| Interface Type | Framework Location | BFM Component |
| --- | --- | --- |
| Custom valid/ready | bin/TBClasses/components/gaxi/ | GAXI Master/Slave |
| AXI4 | bin/TBClasses/components/axi4/ | AXI4 Master/Slave |
| AXI4-Lite (AXIL) | bin/TBClasses/components/axil4/ | AXIL Master/Slave |
| APB | bin/TBClasses/components/apb/ | APB Master/Slave |
| AXI-Stream (AXIS) | bin/TBClasses/components/axis4/ | AXIS Master/Slave |
| Network | bin/TBClasses/components/network/ | Network Master/Slave |
| MonBus | bin/TBClasses/components/monbus/ | MonBus drivers |

Rationale: 1. Consistency: All tests use standardized handshake protocols 2. Correctness: BFMs handle complex timing scenarios (backpressure, randomization) 3. Reusability: Same BFM across all RAPIDS tests 4. Maintainability: Fix once in BFM, all tests benefit 5. Coverage: BFMs include comprehensive timing profiles

Example - Program Engine:

# ✗ WRONG: Manual handshake (violates design requirement)
async def send_request(self, addr, data):
    self.dut.program_valid.value = 1
    self.dut.program_pkt_addr.value = addr
    # ... manual handshaking logic ...

# ✓ CORRECT: Use GAXI Master BFM
from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster

class ProgramEngineTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.program_master = GAXIMaster(
            dut=dut,
            clock=dut.clk,
            valid_signal='program_valid',
            ready_signal='program_ready',
            data_signals=['program_pkt_addr', 'program_pkt_data'],
            data_widths=[64, 32]
        )

    async def send_request(self, addr, data):
        await self.program_master.write({'program_pkt_addr': addr, 'program_pkt_data': data})

📖 See: - projects/components/rapids/CLAUDE.md - Rule #1 for complete BFM usage guidelines - bin/TBClasses/components/gaxi/README.md - GAXI BFM documentation - bin/TBClasses/components/axi4/README.md - AXI4 BFM documentation

### 25.14.4	12.3 Test File Structure (Standard Pattern)

⚠ MANDATORY: All RAPIDS tests must follow this structure ⚠

RAPIDS tests follow the same pattern as AMBA tests for consistency across the repository:

# Example: projects/components/rapids/dv/tests/fub_tests/scheduler/test_scheduler.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# Import REUSABLE testbench class (NOT defined in this file!)
from TBClasses.rapids.scheduler_tb import SchedulerTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic_flow(dut):
    """Test basic descriptor flow."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_basic_descriptor_flow()
    assert result, "Basic descriptor flow test failed"

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_credit_encoding(dut):
    """Test exponential credit encoding."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_exponential_encoding_all_values()
    assert result, "Credit encoding test failed"

# ===========================================================================
# PARAMETER GENERATION - at bottom of file
# ===========================================================================

def generate_scheduler_test_params():
    """Generate test parameters for scheduler tests."""
    return [
        # (channel_id, num_channels, data_width, credit_width)
        (0, 8, 512, 8),  # Standard configuration
        # Add more parameter sets as needed
    ]

scheduler_params = generate_scheduler_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - at bottom of file
# ===========================================================================

@pytest.mark.parametrize("channel_id, num_channels, data_width, credit_width", scheduler_params)
def test_basic_flow(request, channel_id, num_channels, data_width, credit_width):
    """
    Scheduler basic flow test.

    Run with: pytest projects/components/rapids/dv/tests/fub_tests/scheduler/test_scheduler.py::test_basic_flow -v
    """
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_rapids_fub': '../../rtl/rapids_fub'
    })

    dut_name = "scheduler"
    toplevel = dut_name

    verilog_sources = [
        os.path.join(repo_root, 'rtl', 'amba', 'includes', 'monitor_pkg.sv'),
        os.path.join(repo_root, 'rtl', 'rapids', 'includes', 'rapids_pkg.sv'),
        os.path.join(rtl_dict['rtl_rapids_fub'], f'{dut_name}.sv'),
    ]

    # Format parameters for unique test name
    cid_str = TBBase.format_dec(channel_id, 2)
    nc_str = TBBase.format_dec(num_channels, 2)
    dw_str = TBBase.format_dec(data_width, 4)
    cw_str = TBBase.format_dec(credit_width, 2)
    test_name_plus_params = f"test_{dut_name}_cid{cid_str}_nc{nc_str}_dw{dw_str}_cw{cw_str}"

    # Add worker ID for pytest-xdist parallel execution
    worker_id = os.environ.get('PYTEST_XDIST_WORKER', '')
    if worker_id:
        test_name_plus_params = f"{test_name_plus_params}_{worker_id}"

    log_path = os.path.join(log_dir, f'{test_name_plus_params}.log')
    sim_build = os.path.join(tests_dir, 'local_sim_build', test_name_plus_params)
    os.makedirs(sim_build, exist_ok=True)
    os.makedirs(log_dir, exist_ok=True)

    rtl_parameters = {
        'CHANNEL_ID': channel_id,
        'NUM_CHANNELS': num_channels,
        'DATA_WIDTH': data_width,
        'CREDIT_WIDTH': credit_width,
        # Add other RTL parameters as needed
    }

    extra_env = {
        'LOG_PATH': log_path,
        'TEST_CHANNEL_ID': str(channel_id),
        'TEST_NUM_CHANNELS': str(num_channels),
        'TEST_DATA_WIDTH': str(data_width),
    }

    compile_args = ["-Wno-TIMESCALEMOD"]
    sim_args = []
    plusargs = []

    cmd_filename = create_view_cmd(log_dir, log_path, sim_build, module, test_name_plus_params)

    try:
        run(
            python_search=[tests_dir],
            verilog_sources=verilog_sources,
            includes=[
                os.path.join(repo_root, 'rtl', 'rapids', 'includes'),
                os.path.join(repo_root, 'rtl', 'amba', 'includes'),
            ],
            toplevel=toplevel,
            module=module,
            testcase="cocotb_test_basic_flow",  # ← cocotb test function name
            parameters=rtl_parameters,
            sim_build=sim_build,
            extra_env=extra_env,
            waves=False,
            keep_files=True,
            compile_args=compile_args,
            sim_args=sim_args,
            plusargs=plusargs,
        )

        print(f"✓ Scheduler basic flow test completed!")
        print(f"Logs: {log_path}")

    except Exception as e:
        print(f"✗ Scheduler basic flow test failed: {str(e)}")
        print(f"Logs preserved at: {log_path}")
        raise

Key Structure Requirements:

- Testbench Class Location:

  - ALWAYS in bin/TBClasses/rapids/

  - NEVER inline in test file

  - Reusable across multiple test files

- CocoTB Test Functions:

  - Prefix with cocotb_ to prevent pytest collection

  - Located at top of test file

  - Use @cocotb.test() decorator

  - Call testbench methods

- Parameter Generation:

  - Function returns list of parameter tuples

  - Located near bottom of file (before pytest wrappers)

  - Stored in variable (e.g., scheduler_params)

- Pytest Wrapper Functions:

  - Located at bottom of file

  - Use @pytest.mark.parametrize() with parameter variable

  - Build unique test names with TBBase.format_dec()

  - Call run() with testcase="cocotb_test_function_name"

  - Handle parallel execution (PYTEST_XDIST_WORKER)

- Mandatory TB Methods:

  - async def setup_clocks_and_reset(self) - Complete initialization

  - async def assert_reset(self) - Assert reset signal(s)

  - async def deassert_reset(self) - Deassert reset signal(s)

📖 See: - val/amba/test_apb_slave.py - Reference example - projects/components/rapids/CLAUDE.md - Detailed TB requirements

## 25.15	13. Quick Reference

### 25.15.1	13.1 Key Files

| File | Purpose |
| --- | --- |
| projects/components/rapids/PRD.md | This document (overview) |
| projects/components/rapids/README.md | Quick start guide |
| projects/components/rapids/CLAUDE.md | AI assistance guide |
| projects/components/rapids/TASKS.md | Work items (to be created) |
| projects/components/rapids/docs/rapids_spec/ | Complete specification |
| projects/components/rapids/known_issues/ | Bug tracking |
| docs/RAPIDS_Validation_Status_Report.md | Test results |

### 25.15.2	13.2 Commands

# Run RAPIDS tests
pytest projects/components/rapids/dv/tests/fub_tests/ -v          # Individual blocks
pytest projects/components/rapids/dv/tests/integration_tests/ -v   # Multi-block
pytest projects/components/rapids/dv/tests/system_tests/ -v        # Full system

# Run specific FUB test
pytest projects/components/rapids/dv/tests/fub_tests/scheduler/ -v

# Lint RAPIDS RTL
verilator --lint-only projects/components/rapids/rtl/rapids_fub/scheduler.sv

# View specifications
cat projects/components/rapids/docs/rapids_spec/miop_index.md
cat projects/components/rapids/docs/rapids_spec/ch02_blocks/01_01_scheduler.md

## 25.16	14. Success Criteria

### 25.16.1	14.1 Functional

- ✓ All major blocks implemented

- ✓ Basic data flows working

- ✓ Scheduler credit bug fixed (exponential encoding implemented)

- ⏳ Credit management tests verified (remove workarounds, run full suite)

- ⏳ 100% descriptor test pass rate (currently ~80%)

- ⏳ Stress tests passing

### 25.16.2	14.2 Quality

- ⏳ >90% functional coverage (currently ~80%)

- ⏳ >85% code coverage

- ✓ All FSMs documented

- ⏳ Integration guide complete

### 25.16.3	14.3 Documentation

- ✓ Complete specification in rapids_spec/

- ✓ Known issues documented

- ⏳ Integration examples

- ⏳ Performance characterization

## 25.17	15. Educational Value

RAPIDS demonstrates: - ✓ Complex FSM coordination (scheduler ↔ data paths) - ✓ Descriptor-based DMA design patterns - ✓ Buffer management strategies - ✓ Credit-based flow control with exponential encoding - ✓ Multi-interface integration - ✓ Comprehensive monitoring - ✓ Error detection and reporting - ✓ Compact configuration encoding strategies

Target Audience: - Advanced RTL designers - Accelerator architects - DMA engine developers - System integration engineers

## 25.18	15. Attribution and Contribution Guidelines

### 25.18.1	15.1 Git Commit Attribution

When creating git commits for RAPIDS documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: RAPIDS documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 25.19	16. Documentation Generation

### 25.19.1	16.1 Generating PDF/DOCX from Specification

Tool: /mnt/data/github/rtldesignsherpa/bin/md_to_docx.py

Use this tool to convert the linked specification index into a single all-inclusive PDF or DOCX file.

Basic Usage:

# Generate DOCX from rapids_spec index
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc \
    --title-page

# Generate both DOCX and PDF
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc \
    --title-page \
    --pdf

# With custom template (optional)
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    -t path/to/template.dotx \
    --toc \
    --title-page \
    --pdf

Key Features: - Recursive Collection: Follows all markdown links in the index file - Heading Demotion: Automatically adjusts heading levels for included files - Table of Contents: --toc flag generates automatic ToC - Title Page: --title-page flag creates title page from first heading - PDF Export: --pdf flag generates both DOCX and PDF - Image Support: Resolves images relative to source directory - Template Support: Optional custom DOCX/DOTX template via -t flag

Common Workflow:

# 1. Update version number in index file (rapids_index.md)
# 2. Generate documentation
cd /mnt/data/github/rtldesignsherpa
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o projects/components/rapids/docs/RAPIDS_Specification_v0.25.docx \
    --toc --title-page --pdf

# 3. Output files created:
#    - RAPIDS_Specification_v0.25.docx
#    - RAPIDS_Specification_v0.25.pdf (if --pdf used)

Debug Mode:

# Generate debug markdown to see combined output
python bin/md_to_docx.py \
    projects/components/rapids/docs/rapids_spec/rapids_index.md \
    -o output.docx \
    --debug-md

# This creates debug.md showing the complete merged content

Tool Requirements: - Python 3.6+ - Pandoc installed and in PATH - For PDF generation: LaTeX (e.g., texlive) or use Pandoc’s built-in PDF writer

📖 See: /mnt/data/github/rtldesignsherpa/bin/md_to_docx.py for complete implementation details

## 25.20	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/rapids/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/rapids/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the rapids_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 25.21	17. References

### 25.21.1	16.1 Internal Documentation

- Complete Spec: rapids_spec/ ← Primary technical reference

- Validation: docs/RAPIDS_Validation_Status_Report.md

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 25.21.2	16.2 Related Subsystems

- AMBA: rtl/amba/ - Monitor infrastructure used in RAPIDS

- Common: rtl/common/ - Building blocks (counters, FIFOs, etc.)

- CocoTB Framework: bin/TBClasses/rapids/

### 25.21.3	16.3 External References

- AXI4 Specification: ARM IHI0022E

- AXIL4 Specification: ARM IHI0022E (subset)

- Network interface specs (custom Network protocol)

Document Version: 1.0 Last Updated: 2025-09-30 Review Cycle: Monthly during active development Next Review: 2025-10-30 Owner: RTL Design Sherpa Project

## 25.22	Navigation

- ← Back to Root: /PRD.md

- Complete Specification: rapids_spec/miop_index.md

- Quick Start: README.md

- AI Guidance: CLAUDE.md

- Tasks: TASKS.md (to be created)

- Issues: known_issues/

# 26	Address Increment Patterns for DMA and Descriptor-Based Data Movement

Document Version: 1.0 Last Updated: 2026-01-13 Scope: Comprehensive analysis of address increment patterns for descriptor-based systems Reference Implementation: RAPIDS (Rapid AXI Programmable In-band Descriptor System)

## 26.1	Table of Contents

- Executive Summary

- RAPIDS Current Implementation

- AXI Protocol Burst Modes

- Linear Address Increment Patterns

- 2D Transfer Patterns (Stride-Based)

- 3D Transfer Patterns

- Scatter-Gather Patterns

- Circular and Ring Buffer Patterns

- Tensor Memory Access Patterns

- Implementation Comparison Matrix

- RAPIDS Extension Recommendations

- References

## 26.2	1. Executive Summary

Address increment patterns determine how DMA engines and descriptor-based data movement systems calculate memory addresses during transfers. The choice of pattern significantly impacts:

- Bandwidth efficiency: Alignment and burst optimization

- Application fit: Different workloads require different patterns

- Hardware complexity: More patterns = more control logic

- Software flexibility: Richer patterns = simpler programming model

### 26.2.1	Pattern Categories

| Category | Description | Use Cases |
| --- | --- | --- |
| Linear/Contiguous | Sequential address increment | Block copy, streaming data |
| 2D Stride | Row-major with inter-row gaps | Image processing, video frames |
| 3D Volume | Multi-plane with inter-plane gaps | 3D imaging, tensor operations |
| Scatter-Gather | Non-contiguous fragment list | Protocol buffers, fragmented memory |
| Circular | Wrap-around at boundary | Audio streaming, network buffers |
| Tensor | Multi-dimensional with strides | Neural network accelerators |

## 26.3	2. RAPIDS Current Implementation

### 26.3.1	2.1 Descriptor Format (256-bit)

RAPIDS uses a fixed 256-bit descriptor structure defined in rapids_pkg.sv:

Bit Field          Width   Description
─────────────────────────────────────────────────────────
[63:0]             64-bit  src_addr - Source address (byte-aligned)
[127:64]           64-bit  dst_addr - Destination address (byte-aligned)
[159:128]          32-bit  length - Transfer length in BEATS (not bytes)
[191:160]          32-bit  next_descriptor_ptr - Next descriptor address
[207:200]           8-bit  desc_priority - Transfer priority
[199:196]           4-bit  channel_id - Channel ID
[195]               1-bit  error - Error flag
[194]               1-bit  last - Last descriptor in chain
[193]               1-bit  gen_irq - Generate interrupt
[192]               1-bit  valid - Valid descriptor
[255:208]          48-bit  reserved - Future use

### 26.3.2	2.2 Current Address Increment Modes

Mode 1: Beat-Based Linear (Default)

Address Calculation:
  next_addr = current_addr + (beat_count × 64 bytes)

  Where: RAPIDS_BYTES_PER_BEAT = 64 bytes (512-bit data width)

- All transfers aligned to 64-byte boundaries for optimal throughput

- Descriptor length field specifies count in beats

- Software converts byte count to beats: beats = (bytes + 63) >> 6

Mode 2: Chunk-Based Alignment (Sub-Beat Transfers)

For alignment/final phases handling partial beats:

Address Calculation for Alignment Phase:
  offset = address[5:0]                    // 6-bit offset (0-63)
  bytes_to_boundary = (offset == 0) ? 0 : 64 - offset

  Transfer Phases:
  Phase 1 (Alignment): Transfer bytes_to_boundary to reach 64B boundary
  Phase 2 (Streaming): Transfer full 64B beats
  Phase 3 (Final):     Transfer remaining < 64 bytes

### 26.3.3	2.3 Alignment Information Structure

typedef struct packed {
    logic [15:0] first_chunk_enables;    // Phase 1 chunk mask
    logic [15:0] streaming_chunk_enables; // Phase 2 (always 0xFFFF)
    logic [15:0] final_chunk_enables;     // Phase 3 chunk mask
    logic [31:0] first_transfer_bytes;    // Phase 1 size
    logic [31:0] streaming_bytes;         // Phase 2 size
    logic [31:0] final_transfer_bytes;    // Phase 3 size
    logic [3:0]  first_start_chunk;       // Starting chunk (0-15)
    logic [3:0]  first_valid_chunks;      // Chunks in phase 1
    logic [3:0]  final_valid_chunks;      // Chunks in phase 3
} alignment_info_t;

### 26.3.4	2.4 AXI Translation

Transfer Phase    AxSIZE    AxBURST    Address Increment
────────────────────────────────────────────────────────
Alignment         3'b010    INCR       4 bytes × (AxLEN+1)
Streaming         3'b110    INCR       64 bytes × (AxLEN+1)
Final             3'b010    INCR       4 bytes × (AxLEN+1)

## 26.4	3. AXI Protocol Burst Modes

The AXI protocol defines three burst types that constrain address generation:

### 26.4.1	3.1 FIXED Burst (AxBURST = 2’b00)

Address Formula:
  beat[n].addr = AxADDR   (constant for all beats)

Use Cases: - FIFO access (single address register) - Memory-mapped I/O ports - Audio codec sample registers

Constraints: - All beats transfer to same address - Cannot cross 4KB boundary (trivially satisfied)

### 26.4.2	3.2 INCREMENT Burst (AxBURST = 2’b01)

Address Formula:
  beat[n].addr = AxADDR + n × (1 << AxSIZE)

  Example (AxSIZE=6, 64-byte):
    beat[0] = 0x1000
    beat[1] = 0x1040
    beat[2] = 0x1080
    ...

Use Cases: - Linear memory access - Block copy operations - Stream data to/from memory

Constraints: - Cannot cross 4KB boundary - Maximum burst length varies by protocol version

### 26.4.3	3.3 WRAP Burst (AxBURST = 2’b10)

Address Formula:
  wrap_boundary = AxADDR & ~((AxLEN+1) × (1<<AxSIZE) - 1)
  wrap_size = (AxLEN+1) × (1 << AxSIZE)

  beat[n].addr = wrap_boundary + ((AxADDR + n×(1<<AxSIZE)) mod wrap_size)

  Example (AxADDR=0x1020, AxLEN=3, AxSIZE=4):
    Wrap boundary: 0x1000
    Wrap size: 64 bytes (4 × 16)
    beat[0] = 0x1020
    beat[1] = 0x1030
    beat[2] = 0x1000  (wrapped)
    beat[3] = 0x1010

Use Cases: - Cache line fills (critical-word-first) - Circular buffer access

Constraints: - AxLEN must be 1, 3, 7, or 15 - Starting address must be aligned to transfer size

## 26.5	4. Linear Address Increment Patterns

### 26.5.1	4.1 Simple Contiguous

The most basic pattern - sequential addresses:

Descriptor Fields:
  src_addr:  Starting source address
  dst_addr:  Starting destination address
  length:    Total transfer size

Address Calculation:
  for each beat b in [0, length/beat_size):
    src = src_addr + b × beat_size
    dst = dst_addr + b × beat_size

RAPIDS Implementation: Native support via beat-based transfers.

### 26.5.2	4.2 Unaligned Start/End

Handling transfers that don’t start or end on natural boundaries:

Example: Transfer 200 bytes from 0x1040
  Phase 1 (Align):  Transfer 64 bytes (0x1040-0x107F) to reach 0x1080
  Phase 2 (Stream): Transfer 128 bytes (0x1080-0x10FF) in 2 full beats
  Phase 3 (Final):  Transfer 8 bytes (0x1100-0x1107)

RAPIDS Implementation: Native support via alignment_info_t structure.

### 26.5.3	4.3 Size-Constrained Bursts

Breaking large transfers into AXI-compliant bursts:

Constraints:
  - AXI4: Max 256 beats per burst
  - No 4KB boundary crossing

Address Calculation:
  remaining = total_length
  current_addr = start_addr

  while (remaining > 0):
    burst_len = min(remaining, 256×beat_size)
    burst_len = min(burst_len, next_4kb_boundary - current_addr)
    issue_burst(current_addr, burst_len)
    current_addr += burst_len
    remaining -= burst_len

RAPIDS Implementation: Handled by AXI engine burst splitting.

## 26.6	5. 2D Transfer Patterns (Stride-Based)

### 26.6.1	5.1 Concept

2D transfers move rectangular regions where source and destination may have different memory layouts (pitches/strides).

Memory Layout:
  ┌─────────────────────────────────────┐
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 0: Transfer 4 elements
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 1: Transfer 4 elements
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 2: Transfer 4 elements
  │ . . . . . . . . . . . . . . . . . . │
  └─────────────────────────────────────┘
   ←─────── src_stride (pitch) ────────→
   ←─ width ─→

### 26.6.2	5.2 Descriptor Fields (Extended)

2D Descriptor Format:
  src_addr:    Starting source address (top-left corner)
  dst_addr:    Starting destination address
  x_length:    Bytes per row (width of transfer region)
  y_length:    Number of rows
  src_stride:  Source bytes between row starts
  dst_stride:  Destination bytes between row starts

### 26.6.3	5.3 Address Calculation

for row in [0, y_length):
  row_src = src_addr + row × src_stride
  row_dst = dst_addr + row × dst_stride
  transfer(row_src, row_dst, x_length)

### 26.6.4	5.4 Use Cases

Image Processing:

Example: Extract 640×480 ROI from 1920×1080 frame (32-bit pixels)
  src_addr:   frame_base + (y_offset × 1920 + x_offset) × 4
  dst_addr:   roi_buffer
  x_length:   640 × 4 = 2560 bytes
  y_length:   480
  src_stride: 1920 × 4 = 7680 bytes
  dst_stride: 640 × 4 = 2560 bytes (packed destination)

Video Frame Tiling:

Example: Copy 16×16 macroblock
  x_length:   16 × bytes_per_pixel
  y_length:   16
  src_stride: frame_width × bytes_per_pixel
  dst_stride: 16 × bytes_per_pixel

### 26.6.5	5.5 Analog Devices AXI DMAC Implementation

Reference implementation from ADI’s high-speed DMA controller:

Registers:
  X_LENGTH  (0x418): Row width - 1
  Y_LENGTH  (0x41C): Row count - 1
  SRC_STRIDE (0x424): Source row spacing
  DEST_STRIDE (0x420): Destination row spacing

Address Formulas:
  ROW_SRC_ADDRESS = SRC_ADDRESS + SRC_STRIDE × N
  ROW_DEST_ADDRESS = DEST_ADDRESS + DEST_STRIDE × N

## 26.7	6. 3D Transfer Patterns

### 26.7.1	6.1 Concept

3D transfers extend 2D with an additional depth/plane dimension:

Memory Layout (Z planes stacked):
  Plane 0:    Plane 1:    Plane 2:
  ┌───────┐   ┌───────┐   ┌───────┐
  │ █ █ █ │   │ █ █ █ │   │ █ █ █ │
  │ █ █ █ │   │ █ █ █ │   │ █ █ █ │
  └───────┘   └───────┘   └───────┘

### 26.7.2	6.2 Descriptor Fields (Extended)

3D Descriptor Format:
  src_addr:     Starting address (origin corner)
  dst_addr:     Destination address
  x_length:     Width in bytes
  y_length:     Height in rows
  z_length:     Depth in planes
  src_stride_x: Source row stride (unused, implicit x_length)
  src_stride_y: Source row-to-row stride
  src_stride_z: Source plane-to-plane stride
  dst_stride_y: Destination row stride
  dst_stride_z: Destination plane stride

### 26.7.3	6.3 Address Calculation

for plane in [0, z_length):
  for row in [0, y_length):
    src = src_addr + plane×src_stride_z + row×src_stride_y
    dst = dst_addr + plane×dst_stride_z + row×dst_stride_y
    transfer(src, dst, x_length)

### 26.7.4	6.4 Use Cases

3D Medical Imaging:

Example: Extract 64×64×64 VOI from 512×512×256 CT volume
  src_addr:     volume_base + z_off×512×512 + y_off×512 + x_off
  x_length:     64 bytes
  y_length:     64
  z_length:     64
  src_stride_y: 512
  src_stride_z: 512 × 512

Neural Network Tensors:

Example: Move 4D tensor slice [batch, channel, H, W]
  Requires nested 3D operations or generalized tensor addressing

## 26.8	7. Scatter-Gather Patterns

### 26.8.1	7.1 Concept

Scatter-Gather (SG) handles non-contiguous memory regions using linked lists of descriptors:

Descriptor Chain:
  ┌──────────┐     ┌──────────┐     ┌──────────┐
  │ Desc 0   │────→│ Desc 1   │────→│ Desc 2   │────→ NULL
  │ addr=A0  │     │ addr=A1  │     │ addr=A2  │
  │ len=L0   │     │ len=L1   │     │ len=L2   │
  └──────────┘     └──────────┘     └──────────┘

### 26.8.2	7.2 SG Descriptor Fields

Scatter-Gather Descriptor:
  buffer_addr:      Address of this fragment
  buffer_length:    Size of this fragment
  next_desc_ptr:    Address of next descriptor (0 = last)
  control_flags:    Completion interrupt, error handling, etc.
  status:           Completion status (written by DMA)

### 26.8.3	7.3 Operation Modes

Gather (Multiple Source -> Single Destination):

Source fragments: [A0:L0], [A1:L1], [A2:L2]
Destination: Contiguous buffer B

Result: B = concat(mem[A0:A0+L0], mem[A1:A1+L1], mem[A2:A2+L2])

Scatter (Single Source -> Multiple Destinations):

Source: Contiguous buffer B of length L0+L1+L2
Destination fragments: [A0:L0], [A1:L1], [A2:L2]

Result: mem[A0:A0+L0] = B[0:L0]
        mem[A1:A1+L1] = B[L0:L0+L1]
        mem[A2:A2+L2] = B[L0+L1:end]

### 26.8.4	7.4 AMD/Xilinx AXI DMA SG Descriptor Format

Standard Descriptor (32-bit addressing):
  Offset  Field            Description
  ──────────────────────────────────────────────
  0x00    NXTDESC          Next descriptor pointer
  0x04    Reserved
  0x08    BUFFER_ADDRESS   Data buffer pointer
  0x0C    Reserved
  0x10    Reserved
  0x14    Reserved
  0x18    CONTROL          Transfer length, SOF, EOF
  0x1C    STATUS           Completion status

### 26.8.5	7.5 Use Cases

Network Packet Assembly:

Example: Assemble TCP packet from scattered headers and payload
  Desc 0: Ethernet header (14 bytes at addr_eth)
  Desc 1: IP header (20 bytes at addr_ip)
  Desc 2: TCP header (20 bytes at addr_tcp)
  Desc 3: Payload (1460 bytes at addr_payload)

RAPIDS Implementation: Native support via next_descriptor_ptr chaining.

## 26.9	8. Circular and Ring Buffer Patterns

### 26.9.1	8.1 Concept

Circular buffers wrap addresses at a boundary, enabling continuous streaming without descriptor reload:

Memory Layout:
  ┌─────────────────────────────────────┐
  │ 7 │ 0 │ 1 │ 2 │ 3 │ 4 │ 5 │ 6 │ 7 │ (wraps)
  └─────────────────────────────────────┘
      ↑               ↑
    write           read

### 26.9.2	8.2 Descriptor Fields (Extended)

Circular Buffer Descriptor:
  base_addr:    Buffer base address
  buffer_size:  Total buffer size (must be power of 2 for efficient wrap)
  current_ptr:  Current read/write position
  wrap_enable:  Enable address wrapping

### 26.9.3	8.3 Address Calculation

Efficient wrap (power-of-2 size):
  next_addr = base_addr + ((current_ptr + increment) & (buffer_size - 1))

General wrap:
  offset = (current_ptr + increment) % buffer_size
  next_addr = base_addr + offset

### 26.9.4	8.4 Ping-Pong Double Buffering

Common implementation using two descriptors pointing to each other:

Descriptor A:               Descriptor B:
  buffer_addr = BUF_A        buffer_addr = BUF_B
  next_desc = &Desc_B        next_desc = &Desc_A

Operation:
  While DMA processes BUF_A, CPU fills BUF_B
  On completion, DMA auto-loads Desc_B, starts BUF_B
  CPU processes BUF_A while DMA runs BUF_B
  (repeat)

### 26.9.5	8.5 Use Cases

Audio Streaming:

Example: 48kHz stereo audio, 20ms buffers
  buffer_size: 48000 × 2 channels × 2 bytes × 0.02s = 3840 bytes
  Ring with 4 buffers: 15360 bytes total
  Wrap at: base + 15360

Network RX Ring:

Example: Ethernet receive ring
  descriptor_count: 256 (power of 2)
  Each descriptor points to packet buffer
  Last descriptor.next points to first descriptor
  Hardware auto-advances through ring

## 26.10	9. Tensor Accelerator Memory Access Patterns (PRIORITY 0)

Tensor accelerators for neural network inference represent the most demanding and sophisticated address generation requirements. This section provides comprehensive coverage of industry-standard approaches.

### 26.10.1	9.1 Systolic Array Dataflow Fundamentals

Systolic arrays (used in Google TPU, NVIDIA Tensor Cores, etc.) require coordinated data movement across three matrices: inputs (activations), weights, and outputs (partial sums).

Three Primary Dataflows:

| Dataflow | Stationary Data | Moving Data | Best For |
| --- | --- | --- | --- |
| Weight Stationary (WS) | Weights preloaded in PEs | Inputs + partial sums flow | Weight reuse (many inputs) |
| Input Stationary (IS) | Inputs held in PEs | Weights + partial sums flow | Input reuse (many kernels) |
| Output Stationary (OS) | Partial sums accumulate in PEs | Inputs + weights flow | Output accumulation |

Address Pattern Implications:

Weight Stationary:
  - Weights: Load once per layer, stream through array
  - Inputs: Stream row-by-row, reuse across weight tiles
  - Address pattern: 2D tiled with weight-tile-aligned boundaries

Output Stationary:
  - Outputs: Accumulate in place
  - Inputs/Weights: Double-buffered streaming
  - Address pattern: Output-tile-centric addressing

### 26.10.2	9.2 Data Cube Model (NVDLA Reference)

NVIDIA’s Deep Learning Accelerator (NVDLA) defines tensor addressing using a “data cube” model with three key stride parameters:

Data Cube Structure:

Dimensions: Width (W) × Height (H) × Channels (C)

Memory Layout:
  - Data divided into 1×1×32byte "atom cubes"
  - Scanning order: C'(32B) → W → H → C (surfaces)
  - C' changes fastest (innermost loop)

Address Calculation:
  element_addr = base_addr
                 + surface_index × surface_stride
                 + line_index × line_stride
                 + element_offset_within_line

Stride Parameters:

| Parameter | Description | Alignment |
| --- | --- | --- |
| line_stride | Bytes from line N to line N+1 | 32 bytes |
| surface_stride | Bytes from surface N to surface N+1 | 32 bytes |
| base_addr | Starting address | 32 bytes |

NVDLA Register Configuration:

D_DATAIN_FORMAT     : Data format selection
D_DATAIN_SIZE_0     : Width × Height
D_DATAIN_SIZE_1     : Channels
D_LINE_STRIDE       : Line-to-line spacing (0x5040)
D_SURF_STRIDE       : Surface-to-surface spacing

Example:
  Width=26, Height=32, Channels=3
  line_stride = 0x1A0 (aligned width)
  surface_stride = 0x1520 (line_stride × height)

### 26.10.3	9.3 Tensor Memory Accelerator (TMA) - NVIDIA Hopper

NVIDIA’s Hopper architecture introduces hardware TMA for efficient multi-dimensional tensor transfers:

TMA Descriptor Format:

CUtensorMap structure (64-bit packed descriptor):
  - Global memory base address
  - Tensor rank (1D to 5D supported)
  - Dimension sizes and strides
  - Swizzle pattern for bank conflict avoidance
  - Box dimensions (tile size for transfer)

Key Constraints:

Alignment Requirements:
  - Contiguous dimension must have stride=1
  - All other strides must be multiples of 16 bytes
  - For float tensors [M,N] with stride [N,1]: N % 4 == 0

Tile Quantization:
  - TPU/GPU systolic arrays process 128×128 or 128×8 tiles
  - Tensors must align to tile boundaries to avoid padding waste
  - Performance penalty for partial tiles (idle MACs)

Address Generation (Hardware):

TMA eliminates per-thread address calculation:
  Traditional: Each thread computes addr = base + thread_id × stride
  TMA: Single descriptor, hardware generates all addresses

Coordinate-based access:
  ArithTuple(row, col) → Hardware translates to physical address
  Automatic out-of-bounds predication (no manual boundary checks)

### 26.10.4	9.4 Convolution Address Patterns

#### 26.10.4.1	9.4.1 Direct Convolution

Input Feature Map:  [N, C_in, H_in, W_in]
Weight Kernel:      [C_out, C_in, K_h, K_w]
Output Feature Map: [N, C_out, H_out, W_out]

Output position (n, c_out, h_out, w_out):
  For each (c_in, k_h, k_w):
    input_addr = base_in + n×(C_in×H_in×W_in)
                         + c_in×(H_in×W_in)
                         + (h_out×stride_h + k_h)×W_in
                         + (w_out×stride_w + k_w)

    weight_addr = base_w + c_out×(C_in×K_h×K_w)
                         + c_in×(K_h×K_w)
                         + k_h×K_w
                         + k_w

#### 26.10.4.2	9.4.2 Im2Col Transformation

Converts convolution to GEMM for systolic array efficiency:

Original Convolution:
  Output[h,w] = sum(Input[h+kh, w+kw] × Weight[kh, kw])

Im2Col Transformation:
  - Unroll each receptive field into a column
  - Weight kernels become rows
  - Convolution becomes matrix multiplication

Address Pattern for Im2Col:
  For output position (h_out, w_out):
    col_index = h_out × W_out + w_out
    For kernel position (c_in, k_h, k_w):
      row_index = c_in × K_h × K_w + k_h × K_w + k_w
      src_addr = base + c_in×(H×W) + (h_out×s_h+k_h)×W + (w_out×s_w+k_w)
      dst_addr = im2col_base + row_index × num_output_positions + col_index

Data Expansion Factor:
  Naive: K_h × K_w × data replication
  Hardware Im2Col: Generate addresses on-the-fly, no physical expansion

#### 26.10.4.3	9.4.3 Strided and Dilated Convolution

Strided Convolution (stride > 1):
  input_h = output_h × stride + kernel_h
  input_w = output_w × stride + kernel_w
  Reduces output spatial dimensions

Dilated/Atrous Convolution:
  input_h = output_h + kernel_h × dilation
  input_w = output_w + kernel_w × dilation
  Expands receptive field without increasing parameters

Address Modification:
  base_addr + (h × stride + kh × dilation) × line_stride
            + (w × stride + kw × dilation) × element_size

#### 26.10.4.4	9.4.4 Depthwise Separable Convolution

Standard Convolution: C_in × C_out × K × K multiplications
Depthwise Separable: C_in × K × K + C_in × C_out multiplications

Phase 1 - Depthwise (per-channel):
  For each channel c:
    output[c, h, w] = conv2d(input[c], kernel[c])

  Address Pattern:
    Each channel processes independently
    No cross-channel data movement
    input_addr = base + c × H × W + h × W + w

Phase 2 - Pointwise (1×1 convolution):
  Standard GEMM across channels
  input_addr = base + c × (H × W) + position

### 26.10.5	9.5 Gemmini Accelerator Architecture (UC Berkeley Reference)

Open-source systolic array generator with well-documented DMA and addressing:

Memory Hierarchy:

┌─────────────────────────────────────────────┐
│                 Main Memory                  │
│              (Virtual Addresses)             │
└────────────────────┬────────────────────────┘
                     │ DMA (TileLink)
                     ▼
┌─────────────────────────────────────────────┐
│              Scratchpad SRAM                 │
│   (Banked, Row-Addressed, Explicit Mgmt)    │
│   Default: 256KB, 16 elements/row           │
└────────────────────┬────────────────────────┘
                     │
                     ▼
┌─────────────────────────────────────────────┐
│         Systolic Array (16×16 PEs)          │
│    Weight Stationary or Output Stationary   │
└────────────────────┬────────────────────────┘
                     │
                     ▼
┌─────────────────────────────────────────────┐
│            Accumulator SRAM                  │
│   (32-bit accumulators, in-place addition)  │
└─────────────────────────────────────────────┘

Scratchpad Address Format (32-bit):

Bit 31:    0=Scratchpad, 1=Accumulator
Bit 30:    Accumulator mode (0=overwrite, 1=accumulate)
Bit 29:    Read format (0=scaled inputType, 1=raw accType)
Bits 28:0: Row address within selected memory

Row Width:
  Scratchpad: DIM × inputType bits (default: 16 × 8 = 128 bits)
  Accumulator: DIM × accType bits (default: 16 × 32 = 512 bits)

DMA Operations:

// mvin: Move data from DRAM to scratchpad
// Configuration registers set stride
mvin(
    dram_addr,           // Virtual address (64-bit)
    sp_addr,             // Scratchpad row address
    rows,                // Number of rows to transfer
    cols                 // Elements per row (≤ DIM)
);

// mvout: Move data from accumulator to DRAM
// Supports integrated max-pooling
mvout(
    dram_addr,           // Destination address
    acc_addr,            // Accumulator row address
    rows, cols,
    pool_size,           // Optional pooling window
    pool_stride
);

Stride Configuration:

config_mvin:
  stride[0]: Source memory stride for mvin_A
  stride[1]: Source memory stride for mvin_B
  stride[2]: Source memory stride for mvin_D

config_mvout:
  stride: Destination memory stride
  pool_params: Pooling window and stride

### 26.10.6	9.6 Tiling for Large Matrices

When matrices exceed on-chip memory, tiling is essential:

Hierarchical Tiling (CUTLASS Model):

Level 1 - Thread Block Tile:
  Load tile from global memory to shared memory
  Tile size: M_tile × K_tile (A), K_tile × N_tile (B)

Level 2 - Warp Tile:
  Load from shared memory to registers
  Tile size: Warp_M × Warp_K (A), Warp_K × Warp_N (B)

Level 3 - Thread Tile:
  Individual thread's computation
  Register-to-register operations

Address Generation per Level:
  Global: base + block_id × block_tile_size
  Shared: smem_base + warp_id × warp_tile_size
  Register: Direct indexing within tile

Double-Buffering for Latency Hiding:

Buffer Structure:
  tile_buffer[2][TILE_ROWS][TILE_COLS]

Pipeline:
  Cycle N:   Compute on buffer[0], Load to buffer[1]
  Cycle N+1: Compute on buffer[1], Load to buffer[0]

Address Pattern:
  compute_addr = base + (cycle % 2) × tile_size + element_offset
  load_addr    = base + ((cycle + 1) % 2) × tile_size + element_offset

### 26.10.7	9.7 Sparse Tensor Patterns

Sparse accelerators require index-based indirect addressing:

Compressed Sparse Row (CSR) Format:

Dense: [a 0 b]    CSR: values = [a, b, c, d]
       [0 c 0]         col_idx = [0, 2, 1, 0]
       [d 0 0]         row_ptr = [0, 2, 3, 4]

Address Calculation:
  For row r, iterate col_idx[row_ptr[r]] to col_idx[row_ptr[r+1]-1]
  value_addr = values_base + idx
  col_addr = col_idx_base + idx

Indirect Access:
  actual_col = memory[col_addr]
  dense_addr = base + r × stride + actual_col × element_size

Hardware Implications:

Dense Accelerator (TPU): Initiation interval = 1 (fully pipelined)
Sparse Accelerator (EIE): Initiation interval = 17 (metadata overhead)

Trade-off:
  - Sparse saves memory bandwidth (skip zeros)
  - But adds index indirection latency
  - Effective for >90% sparsity

### 26.10.8	9.8 Transformer/Attention Patterns

Modern LLM accelerators require specialized patterns:

Q, K, V Matrix Generation:

Input: X [seq_len, d_model]
Weights: W_Q, W_K, W_V [d_model, d_k]

Q = X × W_Q  →  [seq_len, d_k]
K = X × W_K  →  [seq_len, d_k]
V = X × W_V  →  [seq_len, d_k]

Address Pattern: Standard GEMM with different weight matrices
  q_addr = q_base + seq_idx × d_k + head_idx
  k_addr = k_base + seq_idx × d_k + head_idx
  v_addr = v_base + seq_idx × d_k + head_idx

Attention Score Computation:

Attention = softmax(Q × K^T / sqrt(d_k)) × V

Memory Challenge:
  Q × K^T produces [seq_len × seq_len] matrix
  O(n^2) memory for sequence length n

FlashAttention Tiling:
  Tile Q, K, V into blocks that fit in SRAM
  Compute attention incrementally without materializing full matrix

  Tile Address Pattern:
    q_tile_addr = q_base + tile_row × tile_size × d_k
    k_tile_addr = k_base + tile_col × tile_size × d_k

  Online Softmax:
    Track running max and sum across tiles
    No need to store intermediate attention matrix

### 26.10.9	9.9 Programmable Address Generation Unit (PAGU)

Academic reference for flexible tensor addressing:

PAGU Instruction Set:

Operations Supported:
  - Standard Convolution
  - Padded Convolution
  - Strided Convolution
  - Dilated (Atrous) Convolution
  - Pooling (Max, Average)
  - Upsampled Convolution
  - Transposed Convolution

Performance: 1.7 cycles per address for convolution
Area Overhead: ~4.6× vs fixed datapath (justified by flexibility)

Configuration Registers:

tensor_config_t:
  base_addr[63:0]           // Tensor base address
  dim_sizes[4][31:0]        // Size of each dimension
  dim_strides[4][31:0]      // Stride of each dimension
  kernel_size[15:0]         // Convolution kernel size
  conv_stride[7:0]          // Convolution stride
  dilation[7:0]             // Dilation factor
  padding[7:0]              // Padding amount
  operation_mode[3:0]       // Select operation type

Address Generation FSM:

always_ff @(posedge clk) begin
    case (state)
        IDLE: begin
            if (start) begin
                dim_counters <= '0;
                state <= GENERATE;
            end
        end

        GENERATE: begin
            // Multi-dimensional address calculation
            addr <= base_addr;
            for (int d = 0; d < num_dims; d++) begin
                case (operation_mode)
                    CONV_STANDARD:
                        addr <= addr + dim_counters[d] * dim_strides[d];
                    CONV_STRIDED:
                        addr <= addr + (dim_counters[d] * conv_stride) * dim_strides[d];
                    CONV_DILATED:
                        addr <= addr + (dim_counters[d] * dilation) * dim_strides[d];
                endcase
            end

            // Increment counters (innermost first)
            increment_counters();

            if (done) state <= IDLE;
        end
    endcase
end

## 26.11	10. Implementation Comparison Matrix

### 26.11.1	10.1 Pattern Complexity vs. Capability

| Pattern | Descriptor Fields | Address Logic | Hardware Cost | SW Flexibility |
| --- | --- | --- | --- | --- |
| Linear | 3 (src, dst, len) | Simple add | Low | Limited |
| 2D Stride | 6 (+x_len, y_len, strides) | Nested loop | Medium | Good |
| 3D Volume | 9 (+z_len, z_strides) | Triple nested | Medium-High | Very Good |
| Scatter-Gather | 4 per fragment | List traversal | Medium | Excellent |
| Circular | 4 (+size, wrap) | Modulo/mask | Low-Medium | Good |
| Tensor | N (variable stride) | Sum-of-products | High | Excellent |

### 26.11.2	10.2 Use Case Mapping

| Application Domain | Primary Pattern | Secondary Pattern |
| --- | --- | --- |
| Block memory copy | Linear | - |
| Network packets | Scatter-Gather | Linear |
| Audio streaming | Circular | Linear |
| Image processing | 2D Stride | Linear |
| Video encode/decode | 2D Stride | Scatter-Gather |
| 3D graphics | 3D Volume | 2D Stride |
| Neural networks | Tensor | 2D Stride |
| Database operations | Scatter-Gather | Linear |

### 26.11.3	10.3 RAPIDS Current vs. Extended Capability

| Capability | Current RAPIDS | Industry Standard |
| --- | --- | --- |
| Linear contiguous | YES | YES |
| Unaligned transfers | YES (3-phase) | Varies |
| Descriptor chaining | YES | YES |
| 2D stride | NO | Common |
| 3D volume | NO | Rare |
| Scatter-Gather | Partial (chain) | Full SG |
| Circular wrap | NO | Common |
| Tensor addressing | NO | Emerging |

## 26.12	11. RAPIDS Extension Recommendations

### 26.12.1	11.0 PRIORITY 0: Tensor Accelerator Addressing (CRITICAL)

Rationale: Essential for AI/ML workloads. Tensor accelerators represent the highest-growth market for DMA engines. Without tensor addressing, RAPIDS cannot serve neural network inference applications.

Implementation Approach: Data Cube Model (NVDLA-style)

This approach provides the best balance of flexibility and hardware complexity, supporting convolution, pooling, and GEMM operations.

Extended Descriptor Format:

typedef struct packed {
    // === Base Fields (existing, 256 bits) ===
    logic [63:0]  src_addr;           // Base source address
    logic [63:0]  dst_addr;           // Base destination address
    logic [31:0]  length;             // For linear mode: beats
    logic [31:0]  next_descriptor_ptr;
    logic [7:0]   desc_priority;
    logic [3:0]   channel_id;
    logic         error;
    logic         last;
    logic         gen_irq;
    logic         valid;

    // === Tensor Extension (repurpose reserved bits + add 2nd descriptor word) ===
    // Mode selection
    logic [3:0]   transfer_mode;      // 0=linear, 1=2D, 2=3D/tensor, 3=conv, 4=pool

    // Data cube dimensions
    logic [15:0]  width;              // W dimension (elements)
    logic [15:0]  height;             // H dimension (lines)
    logic [15:0]  channels;           // C dimension (surfaces)

    // Stride parameters (32-byte aligned)
    logic [31:0]  line_stride;        // Bytes between lines
    logic [31:0]  surface_stride;     // Bytes between surfaces

    // Convolution parameters (when transfer_mode == CONV)
    logic [7:0]   kernel_width;       // Kw
    logic [7:0]   kernel_height;      // Kh
    logic [7:0]   conv_stride_x;      // Horizontal stride
    logic [7:0]   conv_stride_y;      // Vertical stride
    logic [7:0]   dilation_x;         // Horizontal dilation
    logic [7:0]   dilation_y;         // Vertical dilation
    logic [7:0]   pad_left;           // Left padding
    logic [7:0]   pad_right;          // Right padding
    logic [7:0]   pad_top;            // Top padding
    logic [7:0]   pad_bottom;         // Bottom padding

    // Pooling parameters (when transfer_mode == POOL)
    logic [7:0]   pool_width;
    logic [7:0]   pool_height;
    logic [7:0]   pool_stride_x;
    logic [7:0]   pool_stride_y;
    logic [1:0]   pool_type;          // 0=max, 1=avg, 2=min

    // Data type and precision
    logic [3:0]   data_type;          // 0=int8, 1=int16, 2=fp16, 3=bf16, 4=fp32
    logic [3:0]   element_size;       // Bytes per element (1, 2, 4)
} descriptor_tensor_t;

Address Generation Unit (AGU) Architecture:

module tensor_address_generator #(
    parameter int PIPE_STAGES = 3     // Pipeline for multiply-accumulate
) (
    input  logic        clk,
    input  logic        rst_n,

    // Configuration from descriptor
    input  descriptor_tensor_t desc,
    input  logic        start,

    // Address output stream
    output logic [63:0] addr,
    output logic        addr_valid,
    input  logic        addr_ready,

    // Status
    output logic        done
);

    // Dimension counters
    logic [15:0] w_cnt, h_cnt, c_cnt;
    logic [7:0]  kw_cnt, kh_cnt;       // Kernel position (for conv)

    // Pipeline registers for address calculation
    logic [63:0] addr_stage[PIPE_STAGES];

    // FSM states
    typedef enum logic [2:0] {
        IDLE,
        CALC_BASE,
        CALC_LINE,
        CALC_SURFACE,
        OUTPUT
    } state_t;
    state_t state;

    // Address calculation based on mode
    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            state <= IDLE;
            w_cnt <= '0;
            h_cnt <= '0;
            c_cnt <= '0;
        end else begin
            case (state)
                IDLE: begin
                    if (start) begin
                        w_cnt <= '0;
                        h_cnt <= '0;
                        c_cnt <= '0;
                        state <= CALC_BASE;
                    end
                end

                CALC_BASE: begin
                    // Stage 1: Base address
                    addr_stage[0] <= desc.src_addr;
                    state <= CALC_LINE;
                end

                CALC_LINE: begin
                    // Stage 2: Add line offset
                    case (desc.transfer_mode)
                        MODE_LINEAR:
                            addr_stage[1] <= addr_stage[0] + (w_cnt * desc.element_size);

                        MODE_2D, MODE_3D:
                            addr_stage[1] <= addr_stage[0]
                                           + (h_cnt * desc.line_stride)
                                           + (w_cnt * desc.element_size);

                        MODE_CONV:
                            // Convolution: account for stride and kernel position
                            addr_stage[1] <= addr_stage[0]
                                           + ((h_cnt * desc.conv_stride_y + kh_cnt * desc.dilation_y) * desc.line_stride)
                                           + ((w_cnt * desc.conv_stride_x + kw_cnt * desc.dilation_x) * desc.element_size);

                        MODE_POOL:
                            addr_stage[1] <= addr_stage[0]
                                           + ((h_cnt * desc.pool_stride_y) * desc.line_stride)
                                           + ((w_cnt * desc.pool_stride_x) * desc.element_size);
                    endcase
                    state <= CALC_SURFACE;
                end

                CALC_SURFACE: begin
                    // Stage 3: Add surface offset (for 3D/tensor)
                    if (desc.transfer_mode inside {MODE_3D, MODE_CONV}) begin
                        addr_stage[2] <= addr_stage[1] + (c_cnt * desc.surface_stride);
                    end else begin
                        addr_stage[2] <= addr_stage[1];
                    end
                    state <= OUTPUT;
                end

                OUTPUT: begin
                    if (addr_ready) begin
                        // Increment counters (innermost first)
                        if (desc.transfer_mode == MODE_CONV) begin
                            // Convolution: iterate kernel, then spatial, then channels
                            if (kw_cnt < desc.kernel_width - 1) begin
                                kw_cnt <= kw_cnt + 1;
                            end else begin
                                kw_cnt <= '0;
                                if (kh_cnt < desc.kernel_height - 1) begin
                                    kh_cnt <= kh_cnt + 1;
                                end else begin
                                    kh_cnt <= '0;
                                    // Continue to spatial/channel iteration
                                    increment_spatial_counters();
                                end
                            end
                        end else begin
                            increment_spatial_counters();
                        end

                        if (transfer_complete())
                            state <= IDLE;
                        else
                            state <= CALC_BASE;
                    end
                end
            endcase
        end
    end

    assign addr = addr_stage[PIPE_STAGES-1];
    assign addr_valid = (state == OUTPUT);
    assign done = (state == IDLE) && !start;

endmodule

Scratchpad Integration (Gemmini-style):

// Memory address format for tensor scratchpad
typedef struct packed {
    logic        mem_select;       // 0=scratchpad, 1=accumulator
    logic        acc_mode;         // 0=overwrite, 1=accumulate
    logic        read_format;      // 0=scaled, 1=raw
    logic [28:0] row_addr;         // Row address within memory
} scratchpad_addr_t;

// DMA configuration for tensor loads
typedef struct packed {
    logic [63:0] dram_addr;        // Virtual address in main memory
    scratchpad_addr_t sp_addr;     // Scratchpad destination
    logic [15:0] rows;             // Number of rows to transfer
    logic [15:0] cols;             // Elements per row
    logic [31:0] dram_stride;      // Source memory stride
    logic [15:0] sp_stride;        // Scratchpad row stride (usually 1)
} tensor_dma_config_t;

Alignment Requirements:

All tensor addresses must satisfy:
  - base_addr aligned to 32 bytes (NVDLA requirement)
  - line_stride aligned to 32 bytes
  - surface_stride aligned to 32 bytes
  - For optimal performance: align to 64 bytes (RAPIDS beat size)

Verification:
  assert((desc.src_addr & 32'h1F) == 0);
  assert((desc.line_stride & 32'h1F) == 0);
  assert((desc.surface_stride & 32'h1F) == 0);

Performance Considerations:

Address Generation Rate:
  - Target: 1 address per cycle (pipelined)
  - Convolution: 1.7 cycles/address typical (due to kernel iteration)

Memory Bandwidth:
  - TPU-class: 600+ GB/s (HBM)
  - FPGA-class: 20-80 GB/s (DDR4/5)
  - RAPIDS target: Match AXI interface bandwidth

Tiling Support:
  - Hardware computes tile boundaries
  - Double-buffering via ping-pong descriptors
  - Software provides tile dimensions, hardware handles addressing

### 26.12.2	11.1 Priority 1: 2D Stride Support

Rationale: High value for image/video processing with moderate complexity. Also serves as foundation for tensor addressing.

Descriptor Extension:

typedef struct packed {
    // Existing fields (256 bits)...

    // 2D extension (96 bits in reserved space)
    logic [31:0] x_length;        // Bytes per row
    logic [15:0] y_length;        // Number of rows
    logic [31:0] src_stride;      // Source row stride
    logic [31:0] dst_stride;      // Destination row stride
    logic        mode_2d;         // Enable 2D mode
} descriptor_2d_t;

Address Generation:

always_comb begin
    if (mode_2d) begin
        row_src_addr = src_addr + current_row * src_stride;
        row_dst_addr = dst_addr + current_row * dst_stride;
        row_length   = x_length;
    end else begin
        row_src_addr = src_addr;
        row_dst_addr = dst_addr;
        row_length   = length * BYTES_PER_BEAT;
    end
end

### 26.12.3	11.2 Priority 2: Circular Buffer Mode

Rationale: Essential for streaming applications, low complexity addition.

Descriptor Extension:

typedef struct packed {
    // Existing fields...

    // Circular buffer extension
    logic [31:0] buffer_size;     // Total buffer size
    logic [31:0] buffer_mask;     // Size-1 for power-of-2
    logic        circular_mode;   // Enable wrap-around
    logic        src_circular;    // Apply to source
    logic        dst_circular;    // Apply to destination
} descriptor_circular_t;

Address Generation:

function automatic logic [63:0] wrap_address(
    logic [63:0] base,
    logic [63:0] offset,
    logic [31:0] mask,
    logic        enable
);
    if (enable)
        return base + (offset & {32'b0, mask});
    else
        return base + offset;
endfunction

### 26.12.4	11.3 Priority 3: Enhanced Scatter-Gather

Rationale: Full SG capability for network and fragmented memory use cases.

Current Limitation: Chain-only SG requires one descriptor per fragment.

Enhancement: Add fragment list support within single descriptor:

typedef struct packed {
    logic [63:0] fragment_list_ptr;  // Pointer to fragment array
    logic [15:0] fragment_count;     // Number of fragments
    logic        sg_mode;            // Enable SG mode

    // Fragment entry format (in memory):
    // [63:0]  address
    // [31:0]  length
    // [31:0]  flags
} descriptor_sg_t;

## 26.13	12. References

### 26.13.1	Industry Documentation

- AMD AXI DMA Product Guide (PG021) - Scatter-Gather descriptor format

- Analog Devices AXI DMAC - 2D transfer implementation

- Understanding AXI Addressing (ZipCPU) - Burst mode analysis

- WRAP Address Calculation (Verification Guide) - Wrap burst formulas

### 26.13.2	GPU and Accelerator Patterns

- NVIDIA CUDA Memory Coalescing - Coalesced access patterns

- CUDA Pitch Linear Memory - 2D array allocation

- NVIDIA TMA Tutorial (CUTLASS) - Tensor Memory Accelerator

- Efficient GEMM in CUDA (CUTLASS) - Hierarchical tiling

### 26.13.3	DMA Architecture

- Intel DMA Descriptors - PCIe DMA linked lists

- DMA330 Microcode - Linked-list implementation

### 26.13.4	Neural Network Accelerators

- NVDLA Hardware Architecture - Data cube model, CDMA

- NVDLA In-Memory Data Formats - Line stride, surface stride

- NVDLA Unit Description - Convolution DMA details

- Gemmini Accelerator (UC Berkeley) - Open-source systolic array

- Gemmini Documentation (Chipyard) - DMA and scratchpad

- Google TPU Architecture - Systolic array overview

- TPU System Architecture - Memory hierarchy

- Programmable AGU for DNNs - Flexible address generation

### 26.13.5	Convolution and Tensor Operations

- Im2Col Characterization - Convolution memory patterns

- ST Neural-ART Programming Model - NPU descriptor concepts

- Systolic Array Dataflows Survey - Weight/input/output stationary

- FlashAttention Analysis - Attention tiling patterns

- Depthwise Separable Convolutions - Channel-wise addressing

### 26.13.6	Sparse Tensor Formats

- FLAASH Sparse Tensor Accelerator - CSR/CSF formats

- SparTen CNN Accelerator - Sparse tensor addressing

- Indirection Stream Architecture - Hardware indirect addressing

## 26.14	Document History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-13 | RTL Design Sherpa | Initial comprehensive analysis |

This document is part of the RAPIDS (Rapid AXI Programmable In-band Descriptor System) design documentation.