RTL Design Sherpa

RAPIDS Hardware Architecture Specification 0.1

September 30, 2026

Table of Contents

1 Rapids Has Index	20

2 Document Information	21

2.1 Purpose	21

2.2 Scope	21

2.3 How This Book Relates to RAPIDS Beats	22

2.4 Audience	22

2.5 Document Conventions	22

2.5.1 Requirement Levels	22

2.5.2 Signal Directions	22

2.5.3 Byte and Beat Terms	23

2.5.4 Timing Diagrams	23

2.6 References	23

3 Revision History	25

3.1 Change Summary	25

3.1.1 Version 0.2 (2026-09-30)	25

3.1.2 Version 0.1 (2026-09-30)	25

4 Acronyms and Definitions	27

4.1 Acronyms	27

4.2 Definitions	27

4.3 Signal Naming Conventions	28

5 Product Overview	29

5.1 Introduction	29

5.2 System Context	29

5.2.1 Figure 1.1.1: RAPIDS in Its System	29

5.3 Target Applications	30

5.4 What the Byte Design Adds	30

5.4.1 Key Capabilities	31

5.5 Permanent Limitation	31

6 Key Features	32

6.1 Feature Summary	32

6.1.1 Multi-Channel Architecture	32

6.1.2 Byte-Granular Data Movement	32

6.1.3 Descriptor-Based Control	32

6.1.4 High-Performance Data Paths	33

6.1.5 Monitoring and Debug	33

6.2 Feature Details	34

6.2.1 SRAM Buffering	35

6.2.2 AXI Burst Optimization	35

6.2.3 Error Handling	36

7 System Context	37

7.1 Integration Overview	37

7.2 System Block Diagram	37

7.2.1 Figure 1.3.1: System Integration	37

7.3 Interface Connections	37

7.3.1 Memory Interface	37

7.3.2 Network Interface	38

7.3.3 Configuration Interface	39

7.4 Clock and Reset	39

7.4.1 Clock Domains	39

7.4.2 Reset Requirements	39

8 Block Diagram	41

8.1 Top-Level Architecture	41

8.1.1 Figure 2.1.1: RAPIDS Block Diagram	41

8.2 Top-Level Integration (rapids_top)	41

8.3 Component Summary	42

8.3.1 Scheduler Group Array	42

8.3.2 Sink Data Path	42

8.3.3 Source Data Path	43

8.4 Hierarchy	43

8.5 Internal Buses	44

9 Data Flow	45

9.1 Overview	45

9.2 Byte Placement Rules	45

9.2.1 Figure 2.2.1: Sink Byte Placement	46

9.3 The Packet Record	46

9.3.1 Figure 2.2.2: Packet Record Flow	47

9.4 Concurrent Operation	48

9.4.1 Deadlock Prevention	48

10 Channel Architecture	50

10.1 Multi-Channel Design	50

10.2 Channel Resource Allocation	50

10.2.1 Figure 2.3.1: Per-Channel and Shared Resources	50

10.3 Per-Channel Resources	50

10.4 Shared Resources	51

10.5 Channel Scheduling	51

10.5.1 Arbitration Policy	51

10.5.2 Figure 2.3.2: Channel Arbitration onto a Shared AXI Port	51

10.6 Channel State Machine	52

10.7 Channel Isolation	54

10.7.1 Per-Channel Error Handling	54

11 Interface Summary	56

11.1 External Interface Overview	56

11.1.1 Figure 3.1.1: External Interfaces	56

11.2 Interface Port Summary	56

11.3 Clock and Reset	57

11.4 Interface Feature Matrix	57

11.5 Signal Count Summary	58

12 AXI4 Interface Specification	59

12.1 Overview	59

12.1.1 Byte Granularity on the Memory Side	59

12.2 Descriptor AXI Master	59

12.3 Sink AXI Master (Write)	60

12.3.1 Purpose	60

12.3.2 Configuration	60

12.3.3 Signal List	60

12.3.4 WSTRB Contract	61

12.4 Source AXI Master (Read)	62

12.4.1 Purpose	62

12.4.2 Configuration	62

12.4.3 Signal List	63

12.4.4 Read Coverage	63

12.5 AXI Protocol Constraints	64

12.5.1 Address Alignment	64

12.5.2 4 KB Boundary Handling	64

12.5.3 Response Handling	64

13 AXIS Interface Specification	66

13.1 Overview	66

13.1.1 Byte Semantics	66

13.2 Sink AXIS Slave	66

13.2.1 Purpose	66

13.2.2 Configuration	67

13.2.3 Signal List	67

13.2.4 Packet Contract	67

13.2.5 Descriptor-First Ordering	68

13.2.6 Backpressure Behavior	69

13.3 Source AXIS Master	69

13.3.1 Purpose	69

13.3.2 Configuration	69

13.3.3 Signal List	70

13.3.4 Egress Packet Contract	70

13.3.5 Timing	71

13.3.6 Data Availability	71

13.4 AXIS Protocol Notes	71

13.4.1 Handshake Rules	71

13.4.2 TLAST Semantics	71

13.4.3 TSTRB Usage	71

13.4.4 Channel Multiplexing via TID	72

14 APB Configuration Interface	73

14.1 Overview	73

14.2 APB Slave and Address Routing	73

14.2.1 APB Slave Signal List	74

14.2.2 Descriptor Kick-Off	74

14.3 Status Interface	75

14.3.1 Per-Channel Status Signals	75

14.3.2 Channel State Encoding	75

14.4 Configuration Registers	75

14.4.1 Address Range Validation	75

14.4.2 Configuration Registers	76

14.5 Programming Sequence	76

14.5.1 Initialization	76

14.5.2 Descriptor Kick-Off	77

15 Monitor Bus Interface	78

15.1 Overview	78

15.2 MonBus Packet Format	78

15.2.1 64-bit Packet Structure	78

15.2.2 Signal List	78

15.3 Packet Types	79

15.4 Agent IDs	79

15.5 Event Codes	80

15.5.1 Scheduler Events (Agent 0x30-0x37)	80

15.5.2 Descriptor Engine Events (Agent 0x10-0x17)	80

15.6 Event Data Encoding	81

15.6.1 Completion Event Data	81

15.6.2 Error Event Data	81

15.6.3 State Transition Event Data	81

15.7 MonBus Arbitration	81

15.7.1 Figure 3.5.1: MonBus Arbitration Tree	82

15.7.2 Arbitration Policy	82

15.8 Top-Level MonBus Delivery (monbus_axil4_axil4_group)	82

15.9 Integration Notes	83

15.9.1 Downstream Connection	83

15.9.2 Packet Rate	83

16 Use Case: Network-to-Memory Transfer	85

16.1 Scenario	85

16.2 Sequence	85

16.2.1 Ordering Rule	86

16.3 Worked Example 1: 77 Bytes at Offset 5	86

16.4 Worked Example 2: 4035 Bytes Crossing a 4 KB Boundary	87

16.5 Errors	88

16.6 Related Chapters	88

17 Use Case: Memory-to-Network Transfer	89

17.1 Scenario	89

17.2 Sequence	89

17.3 Worked Example 1: 77 Bytes at Offset 5	90

17.4 Worked Example 2: 4035 Bytes Crossing a 4 KB Boundary	91

17.5 Errors	91

17.6 Related Chapters	91

18 Use Case: Descriptor Chaining	92

18.1 Overview	92

18.2 Chain Structure	92

18.2.1 Figure 4.3.1: Descriptor Chain Structure	92

18.3 Descriptor Fields for Chaining	94

18.3.1 Relevant Fields	94

18.3.2 Chain Termination	94

18.4 Prefetch Optimization	96

18.4.1 Descriptor Prefetching	96

18.5 Multi-Channel Chaining	96

18.5.1 Figure 4.3.2: Independent Per-Channel Chains	97

18.6 Error During Chain	97

18.6.1 Error Handling	97

18.6.2 Recovery	98

19 Use Case: Multi-Channel Operation	99

19.1 Overview	99

19.2 Channel Independence	99

19.2.1 Figure 4.4.1: Independent Channels	99

19.3 Resource Arbitration	100

19.3.1 Descriptor Port Arbitration	100

19.3.2 Sink/Source Arbitration	100

19.3.3 Figure 4.4.2: Data-Path AXI Arbitration	100

19.4 Channel Isolation	101

19.4.1 Error Isolation	101

19.4.2 Per-Channel Soft Reset	101

19.5 Use Case Examples	101

19.5.1 Mixed Traffic Pattern	101

19.5.2 Bidirectional Transfer	102

19.5.3 Figure 4.4.3: Bidirectional Transfer	102

19.5.4 Load Balancing	102

19.5.5 Figure 4.4.4: Load Balancing by tid	103

19.6 Performance Considerations	103

19.6.1 Bandwidth Sharing	103

19.6.2 Latency Impact	104

19.7 MonBus Events	104

20 Descriptor Format	105

20.1 Overview	105

20.2 Descriptor Layout	105

20.2.1 256-Bit Structure	105

20.3 Field Descriptions	106

20.3.1 Control Bits [195:192]	106

20.3.2 Direction	106

20.3.3 Transfer Length [159:128]	106

20.3.4 Next Pointer [191:160]	106

20.3.5 Address Fields [127:0]	107

20.4 Alignment and Offsets	107

20.4.1 How the offset reaches the bus	108

20.4.2 4 KB boundaries	108

20.4.3 Zero-length descriptors	108

20.4.4 Packet record	109

20.5 Descriptor Opcodes (DATA / CTRL_READ / CTRL_WRITE)	109

20.6 Extended Descriptors (row/col-major addressing)	110

20.6.1 EXT descriptors stay beat-aligned (permanent limitation)	110

20.7 Descriptor Fetch	111

20.8 Alignment Requirements	111

20.9 What Changed from RAPIDS Beats	112

20.10 Descriptor Examples	113

20.10.1 Sink Descriptor (Network to Memory)	113

20.10.2 Source Descriptor (Memory to Network)	113

20.11 Software Construction	113

20.11.1 C Structure Example	113

20.11.2 Descriptor Ring Setup	114

21 Register Map	116

21.1 Overview	116

21.2 Address Space Layout	116

21.3 Base Registers (0x100-0x3FF)	117

21.4 Monitor Registers (+0x800 within each half)	118

21.5 Key Register Fields	119

21.5.1 GLOBAL_CTRL (0x100) - Read/Write	119

21.5.2 SCHED_TIMEOUT_LIMIT (0x208) - Read/Write	120

21.5.3 DESCENG_CONFIG (0x220) - Read/Write	120

21.6 Channel Kick Sequence	121

21.7 Address Decoding	122

22 Initialization Sequence	123

22.1 Overview	123

22.2 Boot Sequence	124

22.2.1 Figure 5.3.1: Bring-Up Sequence	124

22.3 What the Hardware Requires	125

22.4 Detailed Steps	125

22.4.1 Step 1: Verify Hardware Presence	125

22.4.2 Step 2: Optional Reset	126

22.4.3 Step 3: Configure the Engine	126

22.4.4 Step 4: Optional Monitor and Interrupt Egress	127

22.4.5 Step 5: Build Descriptors	127

22.4.6 Step 6: Enable	128

22.5 Starting Transfers	128

22.5.1 Kick a Channel	128

22.6 Completion and Interrupt Handling	129

22.7 Reset and Recovery	130

22.7.1 Per-Channel Reset	130

22.7.2 Global Reset	130

23 Error Handling	131

23.1 Overview	131

23.2 Error Classes	131

23.3 The Byte-Granular Error: Packet Length Mismatch	134

23.3.1 Contract	134

23.3.2 Exposure	134

23.3.3 How it Clears	135

23.3.4 Prevention	135

23.4 Shared Error Classes	135

23.4.1 Invalid or Unfetchable Descriptor	135

23.4.2 Data-Path AXI Errors	135

23.4.3 Control Engine Errors	135

23.4.4 Write-Progress Timeout	136

23.5 Not Errors	136

23.5.1 Packet Record Queue Full	136

23.5.2 Sink Beat With No Packet Record	136

23.5.3 The 4 KB Burst Cap	137

23.6 Error Reporting	137

23.6.1 Registers	137

23.6.2 MonBus	138

23.7 Recovery	138

23.7.1 Channel Reset	138

23.7.2 What the Reset Does to Work in Flight	139

23.8 Software Checklist	141

23.9 Related Chapters	141

24 Throughput	143

24.1 Theoretical Maximum	143

24.2 Byte Efficiency	143

24.2.1 4 KB Boundaries	144

24.2.2 Head-of-Line Delay on the Sink Stream	144

24.3 Measured Results	145

24.3.1 Byte Campaign (Correctness)	145

24.3.2 RAPIDS Beats Performance Report	146

24.3.3 Using the Beats Numbers	147

24.4 Throughput Limiting Factors	147

24.5 Related Chapters	148

25 Latency Characteristics	150

25.1 Latency Components	150

25.1.1 Kick-to-First-Beat Breakdown	150

25.1.2 Total Latency Formula	151

25.2 First-Beat Latency	151

25.3 Control-Descriptor Latency	151

25.4 Latency Variability	152

25.4.1 Sources of Variability	152

25.4.2 Worst-Case Latency	152

25.5 Latency Optimization	152

25.5.1 Software Recommendations	152

25.5.2 Hardware Configuration	152

26 Resource Estimates	154

26.1 Build Configuration	154

26.2 FPGA Resource Summary	154

26.3 Timing	155

26.4 Board Result	156

26.5 Comparison with the Beats Build	156

26.6 What Depends on What	156

26.7 Related Chapters	157

List of Figures

Figure 1.1.1: RAPIDS in Its System	29

Figure 1.3.1: System Integration	37

Figure 2.1.1: RAPIDS Block Diagram	41

Figure 2.2.1: Sink Byte Placement	46

Figure 2.2.2: Packet Record Flow	47

Figure 2.3.1: Per-Channel and Shared Resources	50

Figure 2.3.2: Channel Arbitration onto a Shared AXI Port	51

Figure 3.1.1: External Interfaces	56

Figure 3.5.1: MonBus Arbitration Tree	82

Figure 4.3.1: Descriptor Chain Structure	92

Figure 4.3.2: Independent Per-Channel Chains	97

Figure 4.4.1: Independent Channels	99

Figure 4.4.2: Data-Path AXI Arbitration	100

Figure 4.4.3: Bidirectional Transfer	102

Figure 4.4.4: Load Balancing by tid	103

Figure 5.3.1: Bring-Up Sequence	124

List of Tables

Table 1: Target Audience	22

Table 2: Byte and Beat Terms	23

Table 3: Reference Documents	23

Table 4: Document Revision History	25

Table 5: Acronym Definitions	27

Table 6: Term Definitions	27

Table 7: Signal Naming Conventions	28

Table 8: Target Applications	30

Table 9: Byte Design Additions	30

Table 10: Data Path Summary	33

Table 11: SRAM Buffer Parameters	35

Table 12: AXI Optimization Features	35

Table 13: Error Handling Summary	36

Table 14: AXI Memory Interface Requirements	38

Table 15: AXIS Network Interface	38

Table 16: Configuration Methods	39

Table 17: Clock Domains	39

Table 18: Reset Signals	39

Table 19: Scheduler Array Components	42

Table 20: Sink Path Components	42

Table 21: Source Path Components	43

Table 22: Internal Bus Summary	44

Table 23: Byte Placement Rules	45

Table 24: Resource Isolation	48

Table 25: Per-Channel Resources	50

Table 26: Shared Resources	51

Table 27: Channel Isolation Features	54

Table 28: External Interface Summary	56

Table 29: Clock and Reset Signals	57

Table 30: Interface Feature Matrix	57

Table 31: Signal Count Summary	58

Table 32: Where byte granularity is carried on AXI4	59

Table 33: Sink AXI Configuration	60

Table 34: Sink AXI Signals	60

Table 35: WSTRB per beat	61

Table 36: Source AXI Configuration	62

Table 37: Source AXI Signals	63

Table 38: Address Alignment Requirements	64

Table 39: AXI Response Handling	64

Table 40: Byte Semantics of the RAPIDS Streams	66

Table 41: Sink AXIS Configuration	67

Table 42: Sink AXIS Signals	67

Table 43: Reasons the Sink Deasserts TREADY	69

Table 44: Source AXIS Configuration	69

Table 45: Source AXIS Signals	70

Table 46: Egress Packet Contract	70

Table 47: TLAST Usage	71

Table 48: TSTRB Examples	71

Table 49: APB Address Routing	73

Table 50: APB Slave Signals	74

Table 51: Status Signals	75

Table 52: Scheduler State Encoding	75

Table 53: Key Configuration Registers	76

Table 54: MonBus Signals	78

Table 55: MonBus Packet Types	79

Table 56: RAPIDS Agent IDs	79

Table 57: Scheduler Event Codes	80

Table 58: Descriptor Engine Event Codes	80

Table 59: Top-Level MonBus AXI-Lite Group	83

Table 60: MonBus Packet Rates	83

Table 61: Network-to-Memory Sequence	85

Table 62: Example 1 Quantities	86

Table 63: Example 1 Write Beats	86

Table 64: Example 2 Quantities	87

Table 65: Example 2 Write Bursts	87

Table 66: Network-to-Memory Error Cases	88

Table 67: Memory-to-Network Sequence	89

Table 68: Example 1 Quantities	90

Table 69: Example 1 Egress Steps	90

Table 70: Example 2 Quantities	91

Table 71: Memory-to-Network Error Cases	91

Table 72: Chaining-Related Descriptor Fields	94

Table 73: Mixed Traffic Channel Assignment	101

Table 74: Bandwidth Distribution	103

Table 75: Arbitration Latency	104

Table 76: Control Bits	106

Table 77: Address Field Usage	107

Table 78: Beat Count Examples (BYTE_LANES = 32)	107

Table 79: Descriptor Opcodes	109

Table 80: Address and Length Alignment	111

Table 81: Descriptor Differences, RAPIDS Beats to RAPIDS	112

Table 82: Base Registers	117

Table 83: Monitor Registers (+0x800 within each half; SRC at 0x0800, SNK at 0x1800)	118

Table 84: GLOBAL_CTRL Register	119

Table 85: SCHED_TIMEOUT_LIMIT Register	120

Table 86: DESCENG_CONFIG Register	120

Table 87: Conditions for a Channel to Run	125

Table 88: Error Classes	131

Table 89: Length Mismatch Cases	134

Table 90: Where the Length Mismatch Appears	134

Table 91: Timeout Configuration	136

Table 92: Error Registers	137

Table 93: Channel Reset: Handling of In-Flight Work	139

Table 94: Software Checklist	141

Table 95: Theoretical Maximum (256-bit Build)	143

Table 96: Beats and Efficiency for Small and Unaligned Transfers	144

Table 97: Sources of Measured Figures	145

Table 98: Byte Campaign on the Board (7 of 7 Pass)	145

Table 99: RAPIDS Beats Build, 8 Channels at 128 KB per Channel	146

Table 100: RAPIDS Beats Build, Transfer Size Sweep at 8 Channels	146

Table 101: RAPIDS Beats Build, Channel Scaling at 4096 Beats per Channel	147

Table 102: RAPIDS Beats Build, Burst Length Sweep at 8 Channels and 1024 Beats	147

Table 103: Throughput Limiting Factors	147

Table 104: Board Build Configuration	154

Table 105: Post-Route Utilization, 8 Channels, 256 Bits	155

Table 106: Timing at 100 MHz	155

Table 107: Beats and Byte-Granular Builds	156

Table 108: Scaling Notes	157

# 1	Rapids Has Index

Generated: 2026-10-16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

Document Title: RAPIDS Hardware Architecture Specification (HAS) Document Number: RAPIDS-HAS-002 Version: 0.2 Date: 2026-09-30 Status: Draft

## 2.1	Purpose

This Hardware Architecture Specification (HAS) defines the external interfaces, system integration requirements, and high-level architecture of RAPIDS, the byte-granular descriptor-driven DMA engine. RAPIDS counts bytes, not beats: a descriptor names a byte length and byte addresses, the write master carries byte enables, and the AXI-Stream ports carry packed bytes with a partial last beat.

RAPIDS Beats, the beat-granular design that came first, remains in the repository and remains supported. Its specification is the RAPIDS Beats HAS. The two designs share most of their external behavior, so this document links the RAPIDS Beats chapters that apply unchanged and rewrites only the chapters where the byte-granular design differs.

## 2.2	Scope

This document covers:

- External interface specifications (AXI4, AXIS, APB, MonBus) as they apply to the byte-granular design

- System-level block diagrams and data flows, including the byte placement rules

- Programming model and descriptor format (length in bytes)

- Use cases and operational sequences with unaligned transfers

- Performance characteristics and constraints

This document does NOT cover:

- Internal module architecture (see the RAPIDS MAS)

- RTL implementation details (see the RAPIDS MAS)

- The RAPIDS Beats design (see the RAPIDS Beats HAS)

- Verification test plans

- Physical design constraints

## 2.3	How This Book Relates to RAPIDS Beats

Chapters that are identical for both designs are not copied. The index links them from the RAPIDS Beats HAS, and the table of contents marks each one “(shared with RAPIDS Beats)”. When such a chapter names a module, the byte-granular module is the same name without the _beats suffix, for example rapids_top for rapids_beats_top.

Where a shared chapter says a descriptor length is a number of beats, read it as a number of bytes for RAPIDS. Every chapter that carries that distinction is rewritten in this book, so a reader following the RAPIDS chapters in order will not meet the beat unit except in the comparison tables.

## 2.4	Audience

Target Audience

| Role | Relevance |
| --- | --- |
| System Architect | Primary - system integration |
| Hardware Integration Engineer | Primary - interface connection |
| Software Engineer | Primary - driver development, descriptor construction |
| Verification Engineer | Reference - interface protocols |
| RTL Designer | Reference - external constraints |

## 2.5	Document Conventions

### 2.5.1	Requirement Levels

| Term | Meaning |
| --- | --- |
| SHALL | Mandatory requirement |
| SHOULD | Recommended but not mandatory |
| MAY | Optional feature |

### 2.5.2	Signal Directions

- Input: Signal driven by external logic into RAPIDS

- Output: Signal driven by RAPIDS to external logic

### 2.5.3	Byte and Beat Terms

Byte and Beat Terms

| Term | Meaning |
| --- | --- |
| Beat | One transfer on a data bus, DATA_WIDTH/8 bytes wide (BYTE_LANES) |
| Offset | The low log2(BYTE_LANES) bits of a byte address: the lane where a transfer starts |
| Packet | The bytes of one descriptor as they appear on an AXI-Stream port, delimited by TLAST |
| Packet record | The {bytes, offset} pair the scheduler hands each data path for one descriptor |

### 2.5.4	Timing Diagrams

Timing diagrams use WaveDrom format with the following conventions:

- p = Positive clock edge

- 0/1 = Logic low/high

- x = Unknown/don’t care

- = = Data value (with label)

- . = Previous value continues

## 2.6	References

Reference Documents

| Document | Description |
| --- | --- |
| RAPIDS MAS | Micro-architecture Specification for the byte-granular design |
| RAPIDS Beats HAS | Architecture specification for the beat-granular design; source of every shared chapter |
| ARM AMBA AXI4 Specification | AXI4 protocol reference |
| ARM AMBA AXI-Stream Specification | AXIS protocol reference |
| RAPIDS PRD | Product Requirements Document |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Revision History

Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.2 | 2026-09-30 | RTL Design Sherpa | Channel reset recovers every error class; the aresetn-only caveats are removed from Error Handling |
| 0.1 | 2026-09-30 | RTL Design Sherpa | Initial release: the byte-granular RAPIDS, split from the RAPIDS Beats HAS |

## 3.1	Change Summary

### 3.1.1	Version 0.2 (2026-09-30)

Channel reset closes the data-path error gap

- The per-channel reset now reaches the AXI read and write engines and both byte data paths. The data read and write response errors and the sink packet length error clear on a channel reset, and the channel takes a good descriptor afterwards. Error Handling no longer tells software to use aresetn for these.

- Error Handling gains the list of what a channel reset does to work in flight, including the sink tail discard and the unterminated source packet.

- Head-of-line blocking on a shared sink stream is unchanged and stays documented.

### 3.1.2	Version 0.1 (2026-09-30)

Initial Release

- The byte-granular RAPIDS (rapids TASK-019) is the product design. The RAPIDS Beats design was its stepping stone and keeps its own book, the RAPIDS Beats HAS.

- New chapters where the byte design differs: Product Overview, Key Features, System Context, Block Diagram, Data Flow, AXI4 Master Interface, AXIS Interface, both use cases, Descriptor Format, Error Handling, Throughput and Resource Estimates.

- Chapters unchanged from RAPIDS Beats are linked from the RAPIDS Beats HAS rather than copied, so a correction there reaches both books.

- Permanent limitation recorded: TYPE=EXT (row and column striding) descriptors stay beat-aligned. Linear descriptors may start at any byte.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Acronyms and Definitions

## 4.1	Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface |
| AXIS | AXI-Stream |
| APB | Advanced Peripheral Bus |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| MAS | Module Architecture Specification |
| MonBus | Monitor Bus |
| RAPIDS | Rapid AXI Programmable In-band Descriptor System |
| SRAM | Static Random Access Memory |

## 4.2	Definitions

Term Definitions

| Term | Definition |
| --- | --- |
| Beat | Single data transfer unit (512 bits = 64 bytes in default configuration) |
| Channel | Independent DMA context with dedicated descriptor processing |
| Descriptor | 256-bit control structure defining a single DMA transfer |
| Descriptor Chain | Linked list of descriptors for multi-transfer operations |
| Sink Path | Data flow from network (AXIS slave) to memory (AXI write) |
| Source Path | Data flow from memory (AXI read) to network (AXIS master) |
| Fill | Operation writing data into SRAM buffer (sink path input) |
| Drain | Operation reading data from SRAM buffer (source path output) |

## 4.3	Signal Naming Conventions

Signal Naming Conventions

| Prefix/Suffix | Meaning |
| --- | --- |
| m_axi_* | AXI Master signals |
| s_axis_* | AXIS Slave signals |
| m_axis_* | AXIS Master signals |
| cfg_* | Configuration signals |
| *_valid | Handshake valid signal |
| *_ready | Handshake ready signal |
| *_n | Active-low signal |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Product Overview

## 5.1	Introduction

RAPIDS is a high-performance network-to-memory accelerator for descriptor-based data movement. It bridges network interfaces (AXI-Stream) with system memory (AXI4), moving data without CPU intervention.

RAPIDS is byte-granular. A descriptor gives a length in bytes and byte addresses. The sink writes exactly the bytes the network sent, using byte enables on the AXI4 write data channel. The source emits exactly the bytes the descriptor names, packed from lane 0 of the AXI-Stream, with a partial last beat when the length is not a multiple of the bus width. A transfer may begin at any byte address.

The earlier RAPIDS Beats design moved whole beats only. It was the stepping stone to this design, it stays supported, and its specification is the RAPIDS Beats HAS.

## 5.2	System Context

### 5.2.1	Figure 1.1.1: RAPIDS in Its System

RAPIDS in Its System

Source: ch01_product_context.dot

## 5.3	Target Applications

Target Applications

| Application | Description | Key Requirements |
| --- | --- | --- |
| Network Offload | Receive packets from the network, store to memory | Low latency, arbitrary packet lengths |
| Scatter-Gather DMA | Multi-descriptor data movement | Descriptor chaining |
| Protocol Processing | Header and payload separation at byte boundaries | Any byte offset, one packet per descriptor |
| Streaming Accelerator | Front-end for compute engines | Continuous data flow |

## 5.4	What the Byte Design Adds

Byte Design Additions

| Capability | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Transfer unit | Full beats only | Bytes |
| Address alignment | Pre-aligned required | Any byte for linear descriptors |
| Sink write | Every lane written | Byte enables mark the bytes written |
| Source stream | Full beats | Packed bytes, partial last beat, TLAST per descriptor |
| Flow control | Backpressure-based | Backpressure-based |

Flow control is unchanged: both designs rely on backpressure, and neither has credit management.

### 5.4.1	Key Capabilities

- 8 Independent Channels - Concurrent transfers with dedicated resources

- Byte-Granular Transfers - Length in bytes, start at any byte address

- Descriptor Chaining - Automatic next-descriptor fetch

- Dual Data Paths - Simultaneous sink (write) and source (read)

- Monitor Integration - Real-time event reporting via MonBus

- Error Detection - AXI error propagation, and a packet-length check on the sink

## 5.5	Permanent Limitation

TYPE=EXT descriptors, which stride across rows and columns, stay beat-aligned by design. Their addresses SHALL be beat-aligned and their lengths SHALL be a multiple of the beat size. This is a permanent property of the design, not a phase of a schedule. Linear descriptors carry no such restriction. See the Descriptor Format chapter.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Key Features

## 6.1	Feature Summary

### 6.1.1	Multi-Channel Architecture

- 8 Independent Channels with dedicated schedulers

- Per-Channel State Machines for concurrent operation

- Shared AXI Infrastructure with arbitrated access

- Channel Isolation prevents cross-channel interference

### 6.1.2	Byte-Granular Data Movement

- Length in Bytes. The descriptor length field counts bytes. A length of zero moves nothing.

- Any Start Byte. Source and destination byte addresses may have any offset within a beat (linear descriptors).

- Byte Enables on the Write Master. WSTRB is exactly the set of bytes the descriptor writes. The first beat of a transfer strobes from the start offset, the last beat strobes up to the final byte, and every beat between strobes all lanes.

- Packed AXI-Stream. The sink accepts packed bytes with a contiguous TSTRB on the last beat. The source emits packed bytes, contiguous TSTRB, and TLAST on the last beat of each descriptor.

- One Packet per Descriptor. The source ends every descriptor with TLAST. The sink expects one TLAST-delimited packet per descriptor and checks its byte count against the descriptor length.

- Beat-Aligned Bus Traffic. AXI addresses are beat-aligned and no burst crosses a 4 KB boundary. The byte offset never reaches an AXI address: the sink carries it in WSTRB and the source drops it at the egress.

### 6.1.3	Descriptor-Based Control

- 256-bit Descriptor Format with all transfer parameters

- Automatic Chaining follows next_descriptor_ptr links

- Last Descriptor Flag terminates chain processing

- IRQ Generation via gen_irq descriptor flag

- Control Descriptors (CTRL_READ consumer gate, CTRL_WRITE producer doorbell) for in-memory producer and consumer synchronization without moving payload. They are unchanged from RAPIDS Beats; see the Descriptor Format chapter and the shared Descriptor Chaining chapter.

### 6.1.4	High-Performance Data Paths

Data Path Summary

| Path | Direction | Bandwidth | Features |
| --- | --- | --- | --- |
| Sink | Network to Memory | DATA_WIDTH per clock | Ingress byte shifter, SRAM buffering, AXI burst writes with byte enables |
| Source | Memory to Network | DATA_WIDTH per clock | AXI burst reads, SRAM buffering, egress byte re-packer |

### 6.1.5	Monitoring and Debug

- 64-bit MonBus Packets for all significant events

- State Transition Reports track FSM activity

- Error Event Codes for fault diagnosis

- Performance Metrics for throughput analysis

## 6.2	Feature Details

Descriptor Processing

The scheduler leaves the descriptor-fetch state only when every data path the descriptor uses has room for its packet record. A full record queue holds the channel there; it is backpressure, not an error.

### 6.2.1	SRAM Buffering

Each channel has its own buffer in each data path (sink and source), SRAM_DEPTH beats deep:

SRAM Buffer Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| SRAM_DEPTH | 4096 | Beats per channel, per direction (parameter of rapids_top); one beat is one DATA_WIDTH entry |
| Source entry | DATA_WIDTH bits | Data only |
| Sink entry | DATA_WIDTH + DATA_WIDTH/8 bits | Data plus one byte-enable bit per lane |

The buffer holds SRAM_DEPTH x DATA_WIDTH/8 bytes per channel per direction. At the defaults (4096 beats, 512 bits) that is 256 KB, a simulation size. The Genesys 2 builds use 128 beats of 32 bytes, 4 KB per channel.

Buffer Operation:

- Sink Path: Fill from AXIS through the ingress shifter, drain to the AXI write engine

- Source Path: Fill from the AXI read engine, drain through the egress re-packer to AXIS

- Flow Control: Backpressure when the buffer is full or empty

### 6.2.2	AXI Burst Optimization

AXI Optimization Features

| Feature | Implementation |
| --- | --- |
| Burst Length | Up to 256 beats (AXI4 maximum) |
| Outstanding Transactions | Configurable |
| Address Alignment | Beat-aligned AxADDR; the byte offset rides in WSTRB or is dropped at the egress |
| 4KB Boundary | Every burst is capped at the boundary |

### 6.2.3	Error Handling

Error Handling Summary

| Error Type | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | Response check | Transfer aborted |
| AXI DECERR | Response check | Transfer aborted |
| Timeout | Watchdog timer | Recoverable error |
| Packet length mismatch | Sink counts strobed bytes at TLAST | Sticky per-channel write error |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	System Context

## 7.1	Integration Overview

RAPIDS integrates into a system as a DMA engine bridging network interfaces and system memory. This chapter describes the external connections and the system-level requirements that differ from RAPIDS Beats. Everything not mentioned here matches the RAPIDS Beats System Context chapter.

## 7.2	System Block Diagram

### 7.2.1	Figure 1.3.1: System Integration

System Integration

Source: ch01_system_integration.dot

## 7.3	Interface Connections

### 7.3.1	Memory Interface

RAPIDS reaches system memory through the AXI4 masters below. Each half (src_, snk_) has its own descriptor master and its own two control masters (m_axi_ctrlrd, m_axi_ctrlwr). With monitors built, an AXI-Lite master (m_axil_mon) also writes the monitor log:

AXI Memory Interface Requirements

| Port | Access Pattern | Typical Bandwidth |
| --- | --- | --- |
| Descriptor AXI | Random read (256-bit) | Low (<100 MB/s) |
| Sink AXI | Sequential write with byte enables | High (up to interface max) |
| Source AXI | Sequential read | High (up to interface max) |

Interconnect Requirements:

- All of these ports MAY share a single physical AXI port with arbitration

- The descriptor port has the lowest bandwidth requirement

- Sink and Source ports should have balanced bandwidth allocation

- The memory system SHALL honor WSTRB on the sink write master. A slave that ignores byte enables corrupts the bytes outside a partial beat. Every AXI4-conformant slave honors them.

### 7.3.2	Network Interface

RAPIDS uses standard AXI-Stream for network connectivity:

AXIS Network Interface

| Interface | Role | Signals |
| --- | --- | --- |
| Sink AXIS | Receives data from the network | TDATA, TSTRB, TLAST, TID, TVALID, TREADY |
| Source AXIS | Sends data to the network | TDATA, TSTRB, TLAST, TID, TVALID, TREADY |

Protocol Notes:

- Byte-granular: the byte lanes are strobed by TSTRB, and the strobed lanes of a beat SHALL be contiguous from lane 0. RAPIDS does not use TKEEP. Only the last beat of a packet may have a partial TSTRB.

- A packet is the bytes of one descriptor, delimited by TLAST. The source ends every descriptor with TLAST. The sink expects its packet to match the descriptor’s byte length.

- TID selects the channel. TDEST and TUSER are available for sideband information.

- The sink holds TREADY low for a beat until the packet record of that beat’s channel (TID) exists, which is after that channel’s descriptor has been fetched. TREADY is one signal, so a waiting beat also holds up the beats behind it on the same stream. Software SHOULD issue a channel’s descriptor before its packet arrives, or stream one channel at a time.

### 7.3.3	Configuration Interface

Software configures RAPIDS through register writes:

Configuration Methods

| Method | Interface | Usage |
| --- | --- | --- |
| Direct Write | Memory-mapped | Register configuration |
| Descriptor Kick | APB-style | Start descriptor processing |

## 7.4	Clock and Reset

### 7.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | All RAPIDS logic |

Note: RAPIDS uses a single clock domain. Crossing into other frequency domains is handled externally.

### 7.4.2	Reset Requirements

Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert |

Reset Sequence:

- Assert aresetn low for minimum 4 clock cycles

- Ensure all AXI interfaces are idle before reset

- Release reset synchronously to aclk rising edge

- Wait 16 clock cycles before initiating transfers

Reset clears every packet record, every ingress spill, and every sticky write error.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Block Diagram

## 8.1	Top-Level Architecture

RAPIDS has the same block structure as RAPIDS Beats: a scheduler group array per direction, a data path per direction, and a top level that adds the registers, configuration mapping and monitoring. What the byte design adds sits at the edges of the two data paths: an ingress shifter on the sink, an egress re-packer on the source, and a packet record passed from each scheduler to its data path.

### 8.1.1	Figure 2.1.1: RAPIDS Block Diagram

RAPIDS block diagram

Source: 01_byte_block_diagram.dot

## 8.2	Top-Level Integration (rapids_top)

rapids_core is wrapped by rapids_top, which adds the software register interface, configuration mapping, and monitoring infrastructure. The integration is identical to rapids_beats_top: a single APB slave feeds a command/response router into the rapids_regs register block, rapids_config_block translates the register outputs into the cfg_* signals, and a monbus_axil4_axil4_group delivers the merged monitor stream to an AXI-Lite error-drain slave, an AXI-Lite capture master, and a mon_irq interrupt. The register map, the APB interface and the MonBus interface are shared with RAPIDS Beats; see the linked chapters.

The top-level ports have the same names as rapids_beats_top. The only port-level differences are the byte enables: the sink write master’s m_axi_wr_wstrb is driven from the buffered byte enables instead of all ones, and the two AXI-Stream ports use tstrb as a real byte mask.

## 8.3	Component Summary

### 8.3.1	Scheduler Group Array

The scheduler group array manages 8 independent channels per direction:

Scheduler Array Components

| Component | Instance Count | Purpose |
| --- | --- | --- |
| scheduler | 8 | Transfer coordination per channel; converts bytes to beats, issues the packet record |
| descriptor_engine | 8 | Descriptor fetch and parse (unchanged from RAPIDS Beats) |
| Descriptor AXI Arbiter | 1 | Shared AXI4 for descriptor fetch |

### 8.3.2	Sink Data Path

Network-to-memory data flow:

Sink Path Components

| Component | Purpose |
| --- | --- |
| snk_data_path_axis | Ingress: accepts packed bytes, shifts them to offset + lane, holds the spill, flushes after TLAST, counts the packet |
| snk_sram_controller | Multi-channel SRAM management; each entry is data plus byte enables |
| axi_write_engine | AXI4 burst write generation; beat-aligned AWADDR, 4 KB cap, WSTRB from the buffer |
| alloc_ctrl, drain_ctrl | Space and data availability tracking (unchanged) |

### 8.3.3	Source Data Path

Memory-to-network data flow:

Source Path Components

| Component | Purpose |
| --- | --- |
| axi_read_engine | AXI4 burst read generation; beat-aligned ARADDR, 4 KB cap |
| src_sram_controller | Multi-channel SRAM management (data only) |
| src_data_path_axis | Egress: drops the start offset, re-packs from lane 0, sets TSTRB and TLAST |
| alloc_ctrl, drain_ctrl | Space and data availability tracking (unchanged) |

## 8.4	Hierarchy

rapids_core
├── monbus_arbiter
├── rapids_src
│   ├── scheduler_group_array
│   │   └── scheduler_group [0..7]
│   │       ├── scheduler
│   │       ├── descriptor_engine
│   │       ├── ctrlrd_engine
│   │       └── ctrlwr_engine
│   ├── axis4_master_monlite
│   └── src_data_path_axis
│       └── src_data_path
│           ├── axi_read_engine
│           └── src_sram_controller
└── rapids_snk
    ├── scheduler_group_array (same shape)
    ├── axis4_slave_monlite
    └── snk_data_path_axis
        └── snk_data_path
            ├── axi_write_engine
            └── snk_sram_controller

## 8.5	Internal Buses

Internal Bus Summary

| Bus | Width | Description |
| --- | --- | --- |
| Descriptor Bus | 256-bit | Parsed descriptor to scheduler |
| Scheduler Command | Variable | Beat count and byte address to the engines |
| Packet Record | Bytes count plus offset | One per descriptor and direction, scheduler to data path |
| Source SRAM Data | DATA_WIDTH | Data to and from the source SRAM |
| Sink SRAM Data | DATA_WIDTH + DATA_WIDTH/8 | Data plus byte enables |
| MonBus | 64-bit | Monitor packets (aggregated) |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Data Flow

## 9.1	Overview

RAPIDS implements two independent data paths that can operate concurrently:

- Sink Path: Network to Memory (AXIS Slave -> AXI Write)

- Source Path: Memory to Network (AXI Read -> AXIS Master)

The flow is the RAPIDS Beats flow with three additions: a packet record that carries the byte length and offset from the scheduler to the data path, an ingress shifter on the sink, and an egress re-packer on the source.

## 9.2	Byte Placement Rules

A descriptor names a byte length L and a byte address A. With BYTE_LANES bytes per beat and OFF = A mod BYTE_LANES:

Byte Placement Rules

| Quantity | Value |
| --- | --- |
| Start offset | OFF = the low log2(BYTE_LANES) bits of A |
| Memory beats | 0 if L = 0, else ceil((OFF + L) / BYTE_LANES) |
| First memory beat | address A rounded down to a beat boundary; lanes below OFF are not written (sink) or dropped (source) |
| Last memory beat | lanes above the final byte are not written (sink) or dropped (source) |
| Beats between | all lanes carry data |
| Later addresses | rounded-down address plus the beats already moved, times BYTE_LANES |

The stream side is packed: a packet of L bytes occupies ceil(L / BYTE_LANES) stream beats, all full except possibly the last. The number of memory beats and stream beats can differ by one, because the offset shifts the boundary. The data paths absorb the difference.

### 9.2.1	Figure 2.2.1: Sink Byte Placement

Sink byte placement

Source: 02_byte_placement.dot

## 9.3	The Packet Record

For every DATA descriptor, the scheduler hands each data path it uses a record: the byte count and the start offset. The scheduler pulses the record once per descriptor and direction as it leaves the descriptor-fetch state. Each data path keeps a small per-channel queue of records (four deep). If a queue is full, the scheduler stays in descriptor fetch until there is room, so a full queue holds the channel and never drops a record. A zero-length descriptor carries no record.

The record is what lets the data path work in bytes while the engines work in beats. The sink ingress uses it to place bytes and to check the packet length. The source egress uses it to drop the offset and to set TSTRB and TLAST.

### 9.3.1	Figure 2.2.2: Packet Record Flow

Packet record flow

Source: 03_packet_record_flow.dot

Sink Path Data Flow

The ingress accepts a beat only when the channel’s packet record exists. Until then TREADY stays low for that channel. After TLAST, if the offset pushed bytes into a further memory beat, the ingress flushes it as one more beat and holds TREADY low for that cycle.

The byte-enable timing on the write master is in the AXI4 Master Interface chapter.

Source Path Data Flow

The source always reads whole beats and lets the egress discard the bytes outside the descriptor. The first memory beat only primes the egress hold when the offset is non-zero; every later beat emits one stream beat, and a final flush emits the bytes left in the hold. The result is one AXI-Stream packet per descriptor, TLAST on its last beat.

The AXI-Stream chapter carries the waveform for a packet whose last beat is partial.

## 9.4	Concurrent Operation

Both data paths operate independently and can run simultaneously. Neither shares a resource with the other.

### 9.4.1	Deadlock Prevention

RAPIDS prevents deadlock through independent resource allocation:

Resource Isolation

| Resource | Sink Path | Source Path | Shared |
| --- | --- | --- | --- |
| SRAM Buffer | Dedicated | Dedicated | No |
| AXI Port | Write only | Read only | No |
| Scheduler | Per-channel | Per-channel | No |
| Packet record queue | Per-channel, sink | Per-channel, source | No |

The sink’s TREADY is one signal, qualified by the TID of the beat on the bus. A beat for a channel whose descriptor has not been issued holds TREADY low, and it holds up any beats queued behind it on the same AXI-Stream, whatever their channel. A network source that interleaves channels on one stream SHOULD therefore issue every channel’s descriptor before it streams, or present one channel at a time. The record queues and the ingress hold are per channel, so a stalled channel never corrupts another; it can only delay it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Channel Architecture

## 10.1	Multi-Channel Design

RAPIDS Beats supports 8 independent DMA channels, each with dedicated resources for concurrent operation.

## 10.2	Channel Resource Allocation

### 10.2.1	Figure 2.3.1: Per-Channel and Shared Resources

Per-Channel and Shared Resources

Source: ch02_channel_resources.dot

## 10.3	Per-Channel Resources

Each channel has dedicated:

Per-Channel Resources

| Resource | Description | Size/Capacity |
| --- | --- | --- |
| Scheduler | Transfer coordination FSM | 1 instance |
| Descriptor Engine | Descriptor fetch/parse | 8-deep FIFO |
| Sink SRAM Unit | Sink buffer, its own sram_controller | SRAM_DEPTH beats |
| Source SRAM Unit | Source buffer, its own sram_controller | SRAM_DEPTH beats |
| Configuration Registers | Per-channel config | ~16 registers |

## 10.4	Shared Resources

Resources shared across all channels:

Shared Resources

| Resource | Arbitration | Max Outstanding |
| --- | --- | --- |
| Descriptor AXI | Round-robin | 8 (1 per channel) |
| Sink AXI Write | Round-robin | 8 configurable |
| Source AXI Read | Round-robin | 8 configurable |
| MonBus Output | Round-robin (monbus_arbiter tree) | 1 (serialized) |

## 10.5	Channel Scheduling

### 10.5.1	Arbitration Policy

### 10.5.2	Figure 2.3.2: Channel Arbitration onto a Shared AXI Port

Channel Arbitration onto a Shared AXI Port

Source: ch02_channel_arbitration.dot

Arbitration Timing:

Waveform 1

## 10.6	Channel State Machine

Each channel scheduler follows this state machine:

Diagram 4

Channel State Timing

## 10.7	Channel Isolation

Channels are isolated to prevent interference:

Channel Isolation Features

| Isolation Type | Implementation |
| --- | --- |
| State Isolation | Separate FSM per channel |
| Buffer Isolation | Partitioned SRAM per channel |
| Error Isolation | Per-channel error flags |
| Reset Isolation | Per-channel soft reset |

### 10.7.1	Per-Channel Error Handling

Each channel maintains independent error state:

Waveform 3

Channel 1 error does not affect Channel 0 or Channel 2 operation.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Interface Summary

## 11.1	External Interface Overview

RAPIDS Beats exposes the following external interfaces:

### 11.1.1	Figure 3.1.1: External Interfaces

External Interfaces

Source: ch01_system_integration.dot

## 11.2	Interface Port Summary

External Interface Summary

| Interface | Type | Direction | Data Width | Address Width |
| --- | --- | --- | --- | --- |
| Descriptor AXI | AXI4 Master | Read | 256-bit | 64-bit |
| Sink AXI | AXI4 Master | Write | 512-bit | 64-bit |
| Source AXI | AXI4 Master | Read | 512-bit | 64-bit |
| Sink AXIS | AXIS Slave | Input | 512-bit | - |
| Source AXIS | AXIS Master | Output | 512-bit | - |
| APB Config | APB-like | Input | 32-bit | 12-bit |
| MonBus | Custom | Output | 64-bit | - |

## 11.3	Clock and Reset

All interfaces operate in the same clock domain:

Clock and Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aclk | input | System clock (100-500 MHz typical) |
| aresetn | input | Active-low reset (async assert, sync deassert) |

## 11.4	Interface Feature Matrix

Interface Feature Matrix

| Feature | Desc AXI | Sink AXI | Src AXI | Sink AXIS | Src AXIS |
| --- | --- | --- | --- | --- | --- |
| Protocol | AXI4 | AXI4 | AXI4 | AXIS | AXIS |
| Direction | Read | Write | Read | Slave | Master |
| Burst Support | INCR | INCR | INCR | N/A | N/A |
| Max Burst | 1 | 256 | 256 | N/A | N/A |
| Outstanding | 8 | 8 | 8 | N/A | N/A |
| ID Width | 8-bit | 8-bit | 8-bit | Optional | Optional |

## 11.5	Signal Count Summary

Signal Count Summary

| Interface | Input Signals | Output Signals | Total |
| --- | --- | --- | --- |
| Descriptor AXI | ~20 | ~30 | ~50 |
| Sink AXI | ~15 | ~40 | ~55 |
| Source AXI | ~25 | ~25 | ~50 |
| Sink AXIS | ~20 | ~5 | ~25 |
| Source AXIS | ~5 | ~20 | ~25 |
| APB Config | ~15 | ~5 | ~20 |
| MonBus | ~2 | ~5 | ~7 |
| Total | ~102 | ~130 | ~232 |

Note: Signal counts are approximate and vary with configuration parameters.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI4 Interface Specification

## 12.1	Overview

RAPIDS uses AXI4 master interfaces for all memory access:

- Descriptor AXI - Fetches 256-bit descriptors (read-only), one master per half (src_m_axi_desc_*, snk_m_axi_desc_*)

- Sink AXI - Writes network data to memory (m_axi_wr_*)

- Source AXI - Reads memory data for the network (m_axi_rd_*)

- Control AXI - Single-word control reads and writes (*_m_axi_ctrlrd_*, *_m_axi_ctrlwr_*), unchanged from RAPIDS Beats

The descriptor and control masters behave exactly as in RAPIDS Beats; see the RAPIDS Beats AXI4 chapter for their timing. This chapter specifies what changed: the data masters carry byte-granular transfers.

### 12.1.1	Byte Granularity on the Memory Side

The descriptor length is a byte count and the source and destination addresses are byte addresses. The data masters keep AXI4 conventional by splitting the two concerns:

Where byte granularity is carried on AXI4

| Concern | Where it is carried |
| --- | --- |
| Which beats are transferred | AxADDR (beat-aligned) and AxLEN |
| Which bytes within a beat are transferred | WSTRB on the write master; the egress drops unwanted bytes on the read side |

AxSIZE is always the full beat (log2(DATA_WIDTH/8)), so the masters remain ordinary full-width INCR masters that any interconnect accepts.

## 12.2	Descriptor AXI Master

Unchanged from RAPIDS Beats. One 256-bit single-beat INCR read per descriptor, ARLEN = 0, ARSIZE = 5, ARID carries the channel. The source half and the sink half each own a master, and the descriptor address is 32-byte aligned. The descriptor’s contents (Chapter 5) differ: its length field is in bytes.

## 12.3	Sink AXI Master (Write)

### 12.3.1	Purpose

Writes packed network bytes from the SRAM buffer to system memory. The SRAM holds each beat as data plus a strobe ({strb, data}), placed by the sink ingress at the byte offset of the destination address. The write master emits that strobe on WSTRB unchanged.

### 12.3.2	Configuration

Sink AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | 512 | 32-1024 (power of two) | Beat width; 32-byte beats (256-bit) in the Genesys 2 build |
| ADDR_WIDTH | 64 | Configurable | System address space |
| AXI_ID_WIDTH | 8 | Configurable | Transaction tracking (channel in low bits) |
| Max burst | WR_XFER_BEATS register | 1-256 | Per-channel burst cap, further capped at 4 KB |
| Outstanding | 8 | 1-16 | Concurrent AW requests |

### 12.3.3	Signal List

Sink AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_wr_awid | AXI_ID_WIDTH | output | Write ID |
| m_axi_wr_awaddr | ADDR_WIDTH | output | Write address, beat-aligned |
| m_axi_wr_awlen | 8 | output | Burst length - 1 |
| m_axi_wr_awsize | 3 | output | Full beat, log2(DATA_WIDTH/8) |
| m_axi_wr_awburst | 2 | output | INCR |
| m_axi_wr_awvalid / awready | 1 | output / input | Address handshake |
| m_axi_wr_wdata | DATA_WIDTH | output | Write data |
| m_axi_wr_wstrb | DATA_WIDTH/8 | output | Byte enables: the bytes this beat writes |
| m_axi_wr_wlast | 1 | output | Last beat of the burst |
| m_axi_wr_wvalid / wready | 1 | output / input | Data handshake |
| m_axi_wr_bid | AXI_ID_WIDTH | input | Response ID |
| m_axi_wr_bresp | 2 | input | Write response |
| m_axi_wr_bvalid / bready | 1 | input / output | Response handshake |

### 12.3.4	WSTRB Contract

For a descriptor that writes length bytes starting at byte address dst, let offset = dst mod (DATA_WIDTH/8). The transfer covers ceil((offset + length) / (DATA_WIDTH/8)) beats.

WSTRB per beat

| Beat | WSTRB |
| --- | --- |
| First beat | Lanes offset to the end of the beat, or to the last byte if the transfer ends inside this beat |
| Middle beats | All ones |
| Last beat | Lanes 0 to the last byte, or up to the last lane if the transfer ends on a beat boundary |
| Single-beat transfer | Lanes offset to offset + length - 1 |

Lanes outside the run have WSTRB = 0 and their WDATA bytes are not significant. RAPIDS never writes a byte the descriptor did not name, so a destination memory that honors byte enables is left untouched outside [dst, dst + length). Every beat in the transfer carries at least one strobed byte.

#### 12.3.4.1	Waveform 3.1: Unaligned Sink Write, Byte Enables on AXI4

Byte enables on AXI4

Source: 02_axi_byte_enables.json

A 33-byte transfer to base + 31 occupies two beats of a 32-byte-beat interface. AWADDR is base, not base + 31. The first beat strobes only lane 31 (0x80000000) and carries byte 0 there; the second beat strobes all 32 lanes and carries bytes 1 to 32.

## 12.4	Source AXI Master (Read)

### 12.4.1	Purpose

Reads whole beats from system memory into the SRAM buffer. The read master does not use byte enables (AXI has none on the read side). It fetches every beat that overlaps [src, src + length), and the source egress (Chapter 3.2) drops the leading and trailing bytes that were fetched but not requested.

### 12.4.2	Configuration

Source AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | 512 | 32-1024 (power of two) | Beat width |
| ADDR_WIDTH | 64 | Configurable | System address space |
| AXI_ID_WIDTH | 8 | Configurable | Transaction tracking |
| Max burst | RD_XFER_BEATS register | 1-256 | Per-channel burst cap, further capped at 4 KB |
| Outstanding | 8 | 1-16 | Concurrent AR requests |

### 12.4.3	Signal List

Source AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_rd_arid | AXI_ID_WIDTH | output | Read ID |
| m_axi_rd_araddr | ADDR_WIDTH | output | Read address, beat-aligned |
| m_axi_rd_arlen | 8 | output | Burst length - 1 |
| m_axi_rd_arsize | 3 | output | Full beat |
| m_axi_rd_arburst | 2 | output | INCR |
| m_axi_rd_arvalid / arready | 1 | output / input | Address handshake |
| m_axi_rd_rid | AXI_ID_WIDTH | input | Response ID |
| m_axi_rd_rdata | DATA_WIDTH | input | Read data |
| m_axi_rd_rresp | 2 | input | Read response |
| m_axi_rd_rlast | 1 | input | Last beat |
| m_axi_rd_rvalid / rready | 1 | input / output | Data handshake |

### 12.4.4	Read Coverage

A read of length bytes from byte address src fetches ceil((offset + length) / (DATA_WIDTH/8)) beats starting at src rounded down to a beat boundary. Bytes fetched outside [src, src + length) are never delivered to the network.

## 12.5	AXI Protocol Constraints

### 12.5.1	Address Alignment

Address Alignment Requirements

| Transfer Type | Alignment Requirement |
| --- | --- |
| Descriptor | 32-byte aligned |
| Sink data, linear descriptor | None: any byte address |
| Source data, linear descriptor | None: any byte address |
| Sink or source data, TYPE=EXT descriptor | Beat-aligned address, beat-multiple length (permanent) |

The AXI address on the wire is always beat-aligned, whatever the descriptor holds. Only EXT (row and column striding) descriptors restrict the descriptor’s own addresses; that is a permanent design decision, not a phase limitation.

### 12.5.2	4 KB Boundary Handling

AXI4 forbids a burst crossing a 4 KB boundary. RAPIDS caps every burst at the next 4 KB boundary, computed from the beat-aligned address, on both data masters. The cap combines with the configured burst limit: the burst is the smaller of the two.

Example: 4035 bytes to dst = 0x0CB (32-byte beats, offset = 0x0B)
  Beats total = ceil((11 + 4035) / 32) = 127
  Aligned start = 0x0C0; beats from there to the 4 KB boundary = 122
  With WR_XFER_BEATS = 16 the bursts are 16-beat, the last one before the boundary
  ends at 0x1000 (a 122-beat run is 7 bursts of 16 and one of 10);
  the remaining 5 beats start at AWADDR = 0x1000.

The first burst’s WSTRB carries the offset (0xFFFFF800, lanes 11 to 31). After the first burst the working address is the aligned address plus the beats completed, so every later burst is beat-aligned.

### 12.5.3	Response Handling

AXI Response Handling

| Response | Code | RAPIDS Behavior |
| --- | --- | --- |
| OKAY | 2’b00 | Normal completion |
| EXOKAY | 2’b01 | Treated as OKAY |
| SLVERR | 2’b10 | Error, transfer aborted |
| DECERR | 2’b11 | Error, transfer aborted |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	AXIS Interface Specification

## 13.1	Overview

RAPIDS uses AXI-Stream interfaces for network data transfer:

- Sink AXIS Slave - Receives packed bytes from the network (ingress)

- Source AXIS Master - Sends packed bytes to the network (egress)

Both streams carry packed bytes: a packet is a contiguous run of bytes starting at lane 0 of its first beat, with every beat full except possibly the last. The stream is byte-granular through TSTRB, and TLAST delimits exactly one descriptor’s worth of data.

### 13.1.1	Byte Semantics

Byte Semantics of the RAPIDS Streams

| Property | Rule |
| --- | --- |
| Byte qualifier | TSTRB (there is no TKEEP on RAPIDS) |
| Packing | Bytes are packed from lane 0; no gaps inside a packet |
| Full beats | TSTRB all ones |
| Last beat | TSTRB is a contiguous run from lane 0 (2^n - 1), n = bytes in the beat |
| Packet | One descriptor’s length bytes, ending with TLAST |

The destination address offset never appears on the stream. A descriptor that writes to byte address 5 of a beat still receives its packet packed from lane 0; the sink places the bytes at the destination offset internally (Chapter 2).

## 13.2	Sink AXIS Slave

### 13.2.1	Purpose

Receives a packet, places its bytes at the descriptor’s destination offset, and writes them to the SRAM buffer for the sink AXI master.

### 13.2.2	Configuration

Sink AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | 512 | 32-1024 (power of two) | TDATA width |
| AXIS_ID_WIDTH | 8 | 1-8 | TID width; TID selects the channel |
| AXIS_DEST_WIDTH | 8 | 0-8 | TDEST width (not used to route) |
| AXIS_USER_WIDTH | 1 | 0-16 | TUSER width (not used) |

### 13.2.3	Signal List

Sink AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_tdata | DATA_WIDTH | input | Data payload |
| s_axis_tstrb | DATA_WIDTH/8 | input | Byte qualifier, contiguous from lane 0 |
| s_axis_tlast | 1 | input | Last beat of the packet |
| s_axis_tid | AXIS_ID_WIDTH | input | Channel select |
| s_axis_tdest | AXIS_DEST_WIDTH | input | Destination (accepted, not used) |
| s_axis_tuser | AXIS_USER_WIDTH | input | User sideband (accepted, not used) |
| s_axis_tvalid | 1 | input | Data valid |
| s_axis_tready | 1 | output | Ready to accept |

### 13.2.4	Packet Contract

A sink packet SHALL:

- Be delimited by TLAST, with one TID for all its beats.

- Be packed: every beat but the last has TSTRB all ones, and the last beat’s TSTRB is a contiguous run from lane 0.

- Contain exactly as many strobed bytes as the descriptor’s length for that channel.

The byte count is checked at TLAST. A packet whose count differs from the descriptor length is not silently padded or dropped: the channel’s sticky sched_wr_error bit is set and stays set until reset.

### 13.2.5	Descriptor-First Ordering

The sink cannot place bytes until it knows the destination offset, and the offset comes from the channel’s descriptor. s_axis_tready is a single signal. It is qualified by the TID of the beat currently on the bus: it is low whenever that channel has no packet record, in addition to the output register being busy or a flush beat pending.

A beat for a channel whose descriptor has not been fetched therefore stalls the whole stream, including beats for other channels queued behind it. There is no per-channel backpressure. Software SHOULD issue a channel’s descriptor before its packet arrives, or stream one channel at a time until its descriptor is known.

This is the one flow-control difference from RAPIDS Beats, whose sink buffered beats before any descriptor existed. A transmitter that blocks its own descriptor kick behind the stalled stream will deadlock; kick first.

#### 13.2.5.1	Waveform 3.1: Sink Packet With a Partial Last Beat

Packed stream, partial last beat

Source: 01_axis_partial_last_beat.json

A 33-byte packet occupies two 32-byte beats. The first beat is full (TSTRB = FFFFFFFF); the second carries one byte (TSTRB = 00000001) and TLAST. s_axis_tready rises only after the channel’s record exists, so TVALID waits for it.

### 13.2.6	Backpressure Behavior

s_axis_tready deasserts when:

Reasons the Sink Deasserts TREADY

| Cause | Meaning |
| --- | --- |
| No record for the beat’s channel | The descriptor for the TID on the bus has not been fetched; the whole stream waits |
| SRAM space | The channel’s SRAM partition is full |
| Ingress output register busy | The write into the SRAM controller has not completed |
| Flush cycle | TREADY drops for one cycle while the spill beat that follows TLAST is emitted |

The flush cycle occurs when a packet’s last bytes spill into a memory beat beyond the stream’s last beat: for example, 33 bytes to destination offset 5 occupy 38 bytes, so the stream’s second beat maps to two memory beats.

## 13.3	Source AXIS Master

### 13.3.1	Purpose

Reads memory beats through the SRAM buffer and emits the descriptor’s bytes as one packed AXIS packet.

### 13.3.2	Configuration

Source AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | 512 | 32-1024 (power of two) | TDATA width |
| AXIS_ID_WIDTH | 8 | 1-8 | TID width; carries the channel |
| AXIS_DEST_WIDTH | 8 | 0-8 | TDEST width; carries the channel |
| AXIS_USER_WIDTH | 1 | 0-16 | TUSER width; driven 0 |

### 13.3.3	Signal List

Source AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_tdata | DATA_WIDTH | output | Data payload |
| m_axis_tstrb | DATA_WIDTH/8 | output | Byte qualifier, contiguous from lane 0 |
| m_axis_tlast | 1 | output | Last beat of the packet |
| m_axis_tid | AXIS_ID_WIDTH | output | Channel |
| m_axis_tdest | AXIS_DEST_WIDTH | output | Channel (zero-extended) |
| m_axis_tuser | AXIS_USER_WIDTH | output | Driven 0 |
| m_axis_tvalid | 1 | output | Data valid |
| m_axis_tready | 1 | input | Downstream ready |

### 13.3.4	Egress Packet Contract

Egress Packet Contract

| Property | Behavior |
| --- | --- |
| Packet framing | One AXIS packet per descriptor, however many drain blocks it spans |
| Bytes | Exactly length bytes, from the descriptor’s source byte address |
| First beat | Starts at lane 0: the egress shifts memory beats down by the source offset |
| Middle beats | TSTRB all ones |
| Last beat | Contiguous TSTRB and TLAST |
| Zero-length descriptor | No packet |

RAPIDS Beats cut a packet at the end of each drain block and asserted TLAST on every one. RAPIDS asserts TLAST once, on the last byte of the descriptor.

### 13.3.5	Timing

The partial last beat is the mirror image of the sink case (Waveform 3.1 above): the final beat carries a contiguous run of n bytes and TLAST. The re-pack mechanism that produces it, including the beat that only primes the shifter, is specified in the MAS.

### 13.3.6	Data Availability

The source asserts TVALID only when packed data is available. The first memory beat of a packet with a nonzero source offset only primes the egress shifter and produces no output unless the entire packet fits in that beat.

## 13.4	AXIS Protocol Notes

### 13.4.1	Handshake Rules

Standard AXI-Stream handshake applies:

- TVALID asserts when data is available

- TREADY asserts when the receiver can accept

- A transfer occurs on a clock edge when both are high

- TVALID SHALL NOT depend on TREADY

- TREADY MAY depend on TVALID

### 13.4.2	TLAST Semantics

TLAST Usage

| Scenario | TLAST Behavior |
| --- | --- |
| Sink packet | Assert on the last beat of the packet; the byte count must equal the descriptor length |
| Source packet | Asserted once per descriptor, on its last stream beat |
| Continuous stream | Not supported: every transfer is a descriptor-sized packet |

### 13.4.3	TSTRB Usage

TSTRB Examples

| TSTRB Value (32-byte beat) | Meaning |
| --- | --- |
| FFFFFFFF | All 32 bytes valid (every beat but the last) |
| 0000FFFF | Last beat, lower 16 bytes valid |
| 00000001 | Last beat, one byte valid |
| Any non-contiguous pattern | Not produced by RAPIDS; not accepted on the sink |

### 13.4.4	Channel Multiplexing via TID

Channels share each stream through TID. A sink packet SHALL keep one TID from its first beat to TLAST. Each channel is packed independently, so packets of different channels may follow one another, but s_axis_tready does not distinguish channels: a channel that is not ready stalls the stream for all of them (see Descriptor-First Ordering). The source stream emits one packet at a time, tagged with the channel in TID and TDEST.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	APB Configuration Interface

## 14.1	Overview

At the top level (rapids_beats_top), RAPIDS Beats exposes a single APB4 slave (s_apb_*) for all software access. This interface allows software to:

- Configure channel and monitor parameters (via the register block)

- Initiate descriptor processing (kick-off)

- Read status and error information

## 14.2	APB Slave and Address Routing

The APB slave is converted to an internal command/response transaction and routed by address to two targets:

APB Address Routing

| Address Range | Target | Purpose |
| --- | --- | --- |
| 0x000-0x03F | rapids_regs (staged descriptor addresses) | Per-channel CH*_DESC_ADDR_{LOW,HIGH}, stride 0x8 |
| 0x040 | rapids_regs (KICK_ENABLE) | Per-channel launch bits (singlepulse) |
| 0x100-0x3FF | rapids_regs (base regfile) | Channel/scheduler/descriptor configuration and status |
| 0x1000+ | rapids_regs (monitor regfile) | AXI-monitor configuration and performance counters |

Because the monitor regfile is located at 0x1000, the APB address bus must be at least 13 bits wide to reach it. Configuration that was formerly imagined as discrete cfg_* ports is now driven internally by the register block through rapids_config_block; see the Register Map for the full address map, and the descriptor address ranges (DESCENG_ADDR0/1_* at 0x224-0x230).

### 14.2.1	APB Slave Signal List

APB Slave Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_apb_paddr | APB_ADDR_WIDTH (>=13) | input | Address |
| s_apb_psel | 1 | input | Select |
| s_apb_penable | 1 | input | Enable phase |
| s_apb_pwrite | 1 | input | Write / read |
| s_apb_pwdata | 32 | input | Write data |
| s_apb_pstrb | 4 | input | Write byte strobes |
| s_apb_prdata | 32 | output | Read data |
| s_apb_pready | 1 | output | Ready |
| s_apb_pslverr | 1 | output | Slave error |

### 14.2.2	Descriptor Kick-Off

To start descriptor processing on a channel:

- Write descriptor to memory

- Assert apb_valid[ch] with descriptor address on apb_addr

- Wait for apb_ready[ch] assertion

- Deassert apb_valid[ch]

Timing Diagram

## 14.3	Status Interface

### 14.3.1	Per-Channel Status Signals

Status Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| channel_idle | NC | output | Channel idle status |
| channel_error | NC | output | Channel error flag |
| scheduler_state | 4*NC | output | FSM state per channel |

### 14.3.2	Channel State Encoding

Scheduler State Encoding

| State | Value | Description |
| --- | --- | --- |
| IDLE | 4’h0 | Waiting for kick-off |
| WAIT_DESC | 4’h1 | Waiting for descriptor |
| PARSE_DESC | 4’h2 | Parsing descriptor |
| CH_XFER_DATA | 4’h3 | Transfer in progress |
| CHECK_NEXT | 4’h4 | Checking next descriptor |
| COMPLETE | 4’h5 | Transfer complete |
| ERROR | 4’hF | Error state |

## 14.4	Configuration Registers

### 14.4.1	Address Range Validation

RAPIDS validates descriptor addresses against configurable ranges:

Valid if:
  (addr >= cfg_addr0_base && addr < cfg_addr0_limit) ||
  (addr >= cfg_addr1_base && addr < cfg_addr1_limit)

### 14.4.2	Configuration Registers

Configuration and status registers are provided by the rapids_regs register block. See the Register Map for the complete, RTL-accurate address map. The registers most relevant to bring-up are:

Key Configuration Registers

| Register | Offset | Description |
| --- | --- | --- |
| GLOBAL_CTRL | 0x100 | GLOBAL_EN, GLOBAL_RST (level; write 0 to release) |
| CHANNEL_ENABLE | 0x120 | Per-channel enable bitmap |
| SCHED_TIMEOUT_CYCLES | 0x200 | Write-progress timeout window |
| SCHED_TIMEOUT_LIMIT | 0x208 | Consecutive-timeout escalation limit (0 = never) |
| DESCENG_CONFIG | 0x220 | Descriptor-engine enable / prefetch / FIFO threshold |
| DESCENG_ADDR0_BASE/_LIMIT | 0x224/0x228 | Descriptor address range 0 |

## 14.5	Programming Sequence

### 14.5.1	Initialization

// 1. Optional global reset: GLOBAL_RST is a level, not self-clearing
GLOBAL_CTRL = 0x2;           // assert
GLOBAL_CTRL = 0x0;           // release

// 2. Configure descriptor address range(s)
DESCENG_ADDR0_BASE  = 0x8000_0000;
DESCENG_ADDR0_LIMIT = 0x9000_0000;

// 3. Scheduler timeout policy
SCHED_TIMEOUT_CYCLES = 1000;
SCHED_TIMEOUT_LIMIT  = 4;    // escalate after 4 stalled windows; 0 = never

// 4. Enable channels and global enable
CHANNEL_ENABLE = 0xFF;       // enable all 8 channels
GLOBAL_CTRL    = 0x1;        // GLOBAL_EN

### 14.5.2	Descriptor Kick-Off

// Prepare descriptor in memory
desc->src_addr = src;
desc->dst_addr = dst;
desc->length = beats;
desc->last = 1;
desc->valid = 1;

// Kick off channel 0: stage the address, then launch
CH0_DESC_ADDR_LOW  = (uint32_t)desc_addr;
CH0_DESC_ADDR_HIGH = (uint32_t)(desc_addr >> 32);
KICK_ENABLE        = 0x01;   // KICK0, self-clearing

// Wait for completion: idle, or faulted (CH_ERROR is never idle)
while (!(SCHEDULER_IDLE & 0x01) && !(SCHED_ERROR & 0x01));

// Check for errors
if (SCHED_ERROR & 0x01) {
    handle_error(0);
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Monitor Bus Interface

## 15.1	Overview

RAPIDS Beats outputs a 64-bit Monitor Bus (MonBus) for real-time event reporting. The MonBus provides visibility into:

- State machine transitions

- Descriptor processing events

- Error conditions

- Performance metrics

## 15.2	MonBus Packet Format

### 15.2.1	64-bit Packet Structure

Bit Field          Width   Description
───────────────────────────────────────────────────────
[63:60]            4-bit   packet_type (error, completion, etc.)
[59:57]            3-bit   protocol (CORE for RAPIDS)
[56:53]            4-bit   event_code (specific event)
[52:47]            6-bit   channel_id
[46:43]            4-bit   unit_id (subsystem)
[42:35]            8-bit   agent_id (module)
[34:0]             35-bit  event_data (event-specific)

### 15.2.2	Signal List

MonBus Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | 1 | output | Packet valid |
| monbus_pkt_data | 64 | output | Packet data |
| monbus_pkt_ready | 1 | input | Downstream ready |

## 15.3	Packet Types

MonBus Packet Types

| Type | Code | Description |
| --- | --- | --- |
| ERROR | 4’h0 | Error event |
| COMPLETION | 4’h1 | Transfer/operation complete |
| THRESHOLD | 4’h2 | Threshold crossed |
| TIMEOUT | 4’h3 | Timeout event |
| PERF | 4’h4 | Performance metric |
| DEBUG | 4’hF | Debug/trace event |

## 15.4	Agent IDs

RAPIDS modules use the following Agent IDs:

RAPIDS Agent IDs

| Agent ID | Module | Description |
| --- | --- | --- |
| 0x08 | Descriptor AXI monitor | axi4_master_rd_monlite on the descriptor read master (per half, USE_AXI_MONITORS) |
| 0x09 | Sink-ingress AXIS monitor | axis4_slave_monlite on s_axis_* in rapids_snk_beats (USE_AXI_MONITORS; config SNK.MON.WRMON_*) |
| 0x0A | Source-egress AXIS monitor | axis4_master_monlite on m_axis_* in rapids_src_beats (USE_AXI_MONITORS; config SRC.MON.RDMON_*) |
| 0x10-0x17 | Descriptor Engine | Channels 0-7 |
| 0x20 | Control Read Engine | ctrlrd (one per half) |
| 0x21 | Control Write Engine | ctrlwr (one per half) |
| 0x30-0x37 | Scheduler | Channels 0-7 |

These are the values the scheduler group array and the halves pass down (DESC_AXI_MON_AGENT_ID, DESC_MON_BASE_AGENT_ID, CTRLRD_MON_AGENT_ID, CTRLWR_MON_AGENT_ID, SCHED_MON_BASE_AGENT_ID); the module defaults inside the engines differ and are overridden at instantiation. An earlier revision of this table listed data-path agents at 0x40-0x57, which nothing emits: the data paths carry no MonBus emitters, and the AXIS ports are monitored only by the characterization harness’s instruments until TASK-015 lands.

## 15.5	Event Codes

### 15.5.1	Scheduler Events (Agent 0x30-0x37)

Scheduler Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor complete |
| 0x1 | COMPLETION | Chain complete |
| 0x2 | ERROR | Timeout |
| 0x3 | ERROR | AXI error |
| 0x4 | DEBUG | State transition |

The scheduler’s and the descriptor engine’s COMPLETION packets are reported only while the half’s SCHED_CONFIG.COMPL_EN is set (reset value 1). With the bit clear the scheduler group drops them before its monitor-bus arbiter and still acknowledges the emitters, so a channel runs identically either way; ERROR packets are reported regardless of ERR_EN.

### 15.5.2	Descriptor Engine Events (Agent 0x10-0x17)

Descriptor Engine Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor fetched |
| 0x1 | ERROR | Address range error |
| 0x2 | ERROR | AXI read error |
| 0x3 | DEBUG | Fetch started |

Timing Diagram

## 15.6	Event Data Encoding

### 15.6.1	Completion Event Data

[34:32] - Reserved
[31:0]  - Transfer length (beats completed)

### 15.6.2	Error Event Data

[34:32] - Error type
[31:16] - Reserved
[15:0]  - Error details

### 15.6.3	State Transition Event Data

[34:32] - Reserved
[31:28] - Previous state
[27:24] - New state
[23:0]  - Timestamp (lower bits)

## 15.7	MonBus Arbitration

Multiple RAPIDS modules generate MonBus packets. Internal arbitration ensures ordered delivery:

### 15.7.1	Figure 3.5.1: MonBus Arbitration Tree

MonBus Arbitration Tree

Source: ch03_monbus_arbitration.dot

### 15.7.2	Arbitration Policy

- Round-robin across modules

- No packet loss (backpressure if output blocked)

- Priority boosting for ERROR packets

## 15.8	Top-Level MonBus Delivery (monbus_axil4_axil4_group)

At the top level (rapids_beats_top), the internal MonBus is not exposed as a raw 64-bit port. Instead, when USE_AXI_MONITORS = 1, packets are combined and delivered through a monbus_axil4_axil4_group:

- AXI monitors: axi4_master_rd_monlite and axi4_master_wr_monlite (axi_monitor_lite: error, timeout, completion, threshold and the optional address-range checker; no perf or debug class) observe the read (m_axi_rd) and write (m_axi_wr) data masters and emit monitor packets. When USE_AXI_MONITORS = 0, these taps are bypassed and the MonBus outputs below are tied off (mon_irq = 0).

- Arbitration: a 3-input monbus_arbiter merges the read-monitor packet, the write-monitor packet, and the core’s descriptor-monitor packet.

- Delivery: the combined stream feeds monbus_axil4_axil4_group, which presents three external interfaces:

Top-Level MonBus AXI-Lite Group

| Interface | Signals | Purpose |
| --- | --- | --- |
| Error-drain slave | s_axil_err_ar*, s_axil_err_r* (AXI-Lite, 32-bit read) | CPU reads captured error events from the error FIFO |
| Capture master | m_axil_mon_aw*, m_axil_mon_w* (64-bit wdata), m_axil_mon_b* | Bulk-writes the MonBus trace to system memory |
| Interrupt | mon_irq | Asserts on error/threshold events |

The capture region and flush behavior are configured by cfg_mon_base_addr, cfg_mon_limit_addr, and cfg_mon_flush_watermark.

## 15.9	Integration Notes

### 15.9.1	Downstream Connection

Internally, MonBus output connects to:

- MonBus Arbiter - Combines core + rd/wr monitor sources

- monbus_axil4_axil4_group - Error-drain slave, capture master, and mon_irq

- Debug FIFO - Buffered access for debug tools

### 15.9.2	Packet Rate

MonBus Packet Rates

| Scenario | Approximate Rate |
| --- | --- |
| Idle | 0 packets/cycle |
| Single channel active | ~1 packet/100 cycles |
| All channels active | ~1 packet/20 cycles |
| Error storm | Up to 1 packet/cycle |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Use Case: Network-to-Memory Transfer

## 16.1	Scenario

A network agent delivers a packet of L bytes on the sink AXI-Stream. RAPIDS writes exactly those bytes to system memory starting at the byte address A named by the channel’s descriptor. The packet may start anywhere in a beat, and the bytes before A and after A + L - 1 in the boundary beats are left untouched.

The examples use a 256-bit data path (32 byte lanes, DATA_WIDTH = 256, the Genesys 2 configuration). Byte lanes are numbered 0 to 31.

## 16.2	Sequence

Network-to-Memory Sequence

| Step | Actor | Action |
| --- | --- | --- |
| 1 | Software | Builds a descriptor: destination byte address A, length L in bytes, and the channel’s sink direction enabled (Descriptor Format) |
| 2 | Software | Kicks the channel so the descriptor engine fetches the descriptor |
| 3 | Scheduler | Computes the beat count and hands the sink data path a packet record: {bytes = L, offset = A mod 32} |
| 4 | Network agent | Streams the packet on s_axis_* with TID = channel. TREADY rises once the record exists |
| 5 | Sink ingress | Shifts each packed stream beat up by the offset and writes memory-aligned beats, with byte enables, into the channel’s SRAM partition |
| 6 | Sink AXI master | Drains the SRAM as AXI4 write bursts, never crossing 4 KB, with WSTRB from the stored byte enables |
| 7 | Sink ingress | At TLAST, compares the packet’s byte count with L; a mismatch sets the channel’s sticky write-error flag |
| 8 | Scheduler | On the last write response, completes the descriptor and reports it on the MonBus |

### 16.2.1	Ordering Rule

Issue the descriptor before the packet arrives, or stream one channel at a time. s_axis_tready is one signal and it stays low for a beat whose channel has no packet record yet. Until that record exists the whole stream is held, including beats for other channels queued behind it. See AXIS Interface.

## 16.3	Worked Example 1: 77 Bytes at Offset 5

Destination A = base + 5, where base is beat-aligned, and L = 77.

Example 1 Quantities

| Quantity | Value |
| --- | --- |
| Offset | 5 |
| Beats written | (5 + 77 + 31) >> 5 = 3 |
| Stream beats | 3: 32, 32 and 13 bytes |
| Last stream beat TSTRB | 0x00001FFF |

Example 1 Write Beats

| Memory beat | Address | Lanes written | Bytes | WSTRB |
| --- | --- | --- | --- | --- |
| 0 | base | 5 to 31 | 27 | 0xFFFFFFE0 |
| 1 | base + 32 | 0 to 31 | 32 | 0xFFFFFFFF |
| 2 | base + 64 | 0 to 17 | 18 | 0x0003FFFF |

The three beats carry 27 + 32 + 18 = 77 bytes. Stream byte 0 lands in lane 5 of the first memory beat. The five bytes that do not fit in that beat are held and join the next one, so each memory beat is assembled from two neighboring stream beats. The 13 bytes of the last stream beat plus the five held bytes fill lanes 0 to 17 of the last memory beat, so no extra flush beat is needed.

The write is one AXI4 burst of three beats at base. AxADDR is the aligned address and AxSIZE is the full 32-byte beat.

## 16.4	Worked Example 2: 4035 Bytes Crossing a 4 KB Boundary

Destination A = 0x0CB (byte address within a 4 KB page), L = 4035.

Example 2 Quantities

| Quantity | Value |
| --- | --- |
| Offset | 0x0CB mod 32 = 11 |
| Aligned start | 0x0C0 |
| Beats written | (11 + 4035 + 31) >> 5 = 127 |
| Beats before the 4 KB boundary | (0x1000 - 0x0C0) / 32 = 122 |
| Beats after the boundary | 5 |
| First beat WSTRB | lanes 11 to 31, 0xFFFFF800 |
| Last beat | at 0x1080, lanes 0 to 13, WSTRB = 0x00003FFF |

The 127 beats hold 21 + 125 x 32 + 14 = 4035 bytes. The sink write master cannot issue one 127-beat burst, because it would cross 0x1000. It splits at the boundary:

Example 2 Write Bursts

| Burst | AWADDR | Beats |
| --- | --- | --- |
| 1 | 0x0C0 | up to 122 |
| 2 | 0x1000 | 5 |

If the configured write burst limit (WR_XFER_BEATS) is below 122, burst 1 is itself split; the 4 KB cap and the configured limit are combined and the smaller wins. After the first burst the address continues from the aligned address plus the beats already written.

## 16.5	Errors

Network-to-Memory Error Cases

| Condition | Result |
| --- | --- |
| Packet byte count differs from L | Channel’s sticky write-error flag is set and stays set until reset (Error Handling) |
| Write response is not OKAY | AXI write error is reported through the same flag |
| Zero-length descriptor | No record, no packet expected, nothing written |

## 16.6	Related Chapters

- Descriptor Chaining, shared with RAPIDS Beats

- Multi-Channel Operation, shared with RAPIDS Beats

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Use Case: Memory-to-Network Transfer

## 17.1	Scenario

RAPIDS reads L bytes from system memory starting at the byte address A named by the channel’s descriptor and emits them on the source AXI-Stream as one packed packet. The read may start anywhere in a beat. The stream carries only the requested bytes: the first stream beat starts at lane 0 with byte A, and TLAST marks the descriptor’s last byte.

The examples use a 256-bit data path (32 byte lanes, the Genesys 2 configuration).

## 17.2	Sequence

Memory-to-Network Sequence

| Step | Actor | Action |
| --- | --- | --- |
| 1 | Software | Builds a descriptor: source byte address A, length L in bytes, the channel’s source direction enabled (Descriptor Format) |
| 2 | Software | Kicks the channel |
| 3 | Scheduler | Computes the beat count and hands the source data path a packet record: {bytes = L, offset = A mod 32} |
| 4 | Source AXI master | Reads whole aligned beats as AXI4 read bursts, never crossing 4 KB, into the channel’s SRAM partition |
| 5 | Source egress | Pops beats from the SRAM and shifts them down by the offset. The first pop only primes the shifter |
| 6 | Source egress | Emits packed stream beats with TID = TDEST = channel; the last beat carries a contiguous TSTRB and TLAST |
| 7 | Scheduler | When the packet has left, completes the descriptor and reports it on the MonBus |

Unlike the sink, the source needs no ordering rule against the network: the record is created before any read data is requested, so the egress always knows the packet length.

## 17.3	Worked Example 1: 77 Bytes at Offset 5

Source A = base + 5 with base beat-aligned, L = 77.

Example 1 Quantities

| Quantity | Value |
| --- | --- |
| Offset | 5 |
| Memory beats read | (5 + 77 + 31) >> 5 = 3 |
| Stream beats | 3: 32, 32 and 13 bytes |
| Last stream beat TSTRB | 0x00001FFF |

The read master issues one burst of three beats at base. The egress then works as follows:

Example 1 Egress Steps

| Memory beat popped | Output |
| --- | --- |
| M0 | None. Bytes 5 to 31 are held: 27 bytes |
| M1 | Stream beat 0: the 27 held bytes plus bytes 0 to 4 of M1, TSTRB = 0xFFFFFFFF |
| M2 | Stream beat 1: bytes 5 to 31 of M1 plus bytes 0 to 4 of M2, TSTRB = 0xFFFFFFFF |
| (flush) | Stream beat 2: the remaining 13 bytes, TSTRB = 0x00001FFF, TLAST |

The stream carries 32 + 32 + 13 = 77 bytes as one packet.

## 17.4	Worked Example 2: 4035 Bytes Crossing a 4 KB Boundary

Source A = 0x0CB, L = 4035.

Example 2 Quantities

| Quantity | Value |
| --- | --- |
| Offset | 11 |
| Memory beats read | 127 |
| Beats before the 4 KB boundary | 122 |
| Beats after the boundary | 5 |
| Stream beats | 127: 126 full beats and a last beat of 3 bytes |
| Last stream beat TSTRB | 0x00000007 |

The read master splits at 0x1000: one burst of up to 122 beats from 0x0C0 and one of 5 beats from 0x1000, further split if the configured read burst limit (RD_XFER_BEATS) is smaller. The egress removes the 11 leading bytes and delivers 4035 bytes as one packet with a single TLAST, however many drain blocks and read bursts the transfer spanned. RAPIDS Beats asserted TLAST at the end of every drain block; RAPIDS does not.

## 17.5	Errors

Memory-to-Network Error Cases

| Condition | Result |
| --- | --- |
| Read response is not OKAY | Channel’s read error is reported (Error Handling) |
| Zero-length descriptor | No record and no packet |

## 17.6	Related Chapters

- Descriptor Chaining, shared with RAPIDS Beats

- Multi-Channel Operation, shared with RAPIDS Beats

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Use Case: Descriptor Chaining

## 18.1	Overview

Descriptor chaining enables multiple transfers to execute sequentially without software intervention. Each descriptor contains a pointer to the next descriptor, forming a linked list that RAPIDS processes automatically.

## 18.2	Chain Structure

### 18.2.1	Figure 4.3.1: Descriptor Chain Structure

Descriptor Chain Structure

Source: ch04_chain_structure.dot

Chain Execution Flow

Timing Diagram

## 18.3	Descriptor Fields for Chaining

### 18.3.1	Relevant Fields

Chaining-Related Descriptor Fields

| Field | Bits | Description |
| --- | --- | --- |
| next_ptr | [191:128] | 64-bit pointer to next descriptor |
| last | [255] | Last descriptor in chain flag |
| irq_en | [254] | Generate interrupt on completion |

### 18.3.2	Chain Termination

A chain terminates when ANY of these conditions is true:

- next_ptr == 0 (null pointer)

- last == 1 (explicit last flag)

- Error during transfer

Diagram 6

## 18.4	Prefetch Optimization

### 18.4.1	Descriptor Prefetching

RAPIDS can prefetch the next descriptor while the current transfer executes:

Waveform 7

Benefit: Zero-cycle gap between chained transfers when prefetch completes before current transfer.

Prefetch depth is controlled by DESCENG_CONFIG.PREFETCH_EN and DESCENG_CONFIG.FIFO_THRESH (register block, offset 0x220). With prefetch disabled the descriptor engine fetches on demand (roughly one descriptor ahead of the scheduler); with prefetch enabled it buffers up to FIFO_THRESH descriptors ahead. A chain fetch that is throttled (output FIFO at the threshold) is deferred and reissued automatically once the FIFO drains, so no chain link is lost.

## 18.5	Multi-Channel Chaining

Each channel maintains independent chains:

### 18.5.1	Figure 4.3.2: Independent Per-Channel Chains

Independent Per-Channel Chains

Source: ch04_multichannel_chains.dot

Channels execute their chains concurrently and independently.

## 18.6	Error During Chain

### 18.6.1	Error Handling

When an error occurs mid-chain:

Waveform 8

Error Response: 1. Current transfer aborts 2. Chain processing stops 3. Error status captures failing descriptor index 4. MonBus reports error event 5. Interrupt generated (if enabled)

### 18.6.2	Recovery

Software must: 1. Read error status to identify failing descriptor 2. Fix the issue (descriptor content, memory mapping, etc.) 3. Restart chain from failing descriptor or beginning

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Use Case: Multi-Channel Operation

## 19.1	Overview

RAPIDS Beats supports 8 independent DMA channels that can operate concurrently. This enables parallel data transfers for multiple network connections, priority-based scheduling, and efficient resource utilization.

## 19.2	Channel Independence

### 19.2.1	Figure 4.4.1: Independent Channels

Independent Channels

Source: ch04_channel_independence.dot

Concurrent Operation Timing

## 19.3	Resource Arbitration

### 19.3.1	Descriptor Port Arbitration

All channels share a single AXI port for descriptor fetches:

Waveform 10

### 19.3.2	Sink/Source Arbitration

Data path AXI ports use plain round-robin arbitration (arbiter_round_robin inside each engine). There is no priority boost: the descriptor’s priority field is carried but not used for arbitration.

### 19.3.3	Figure 4.4.2: Data-Path AXI Arbitration

Data-Path AXI Arbitration

Source: ch04_datapath_arbitration.dot

## 19.4	Channel Isolation

### 19.4.1	Error Isolation

Errors in one channel do not affect other channels:

Waveform 11

### 19.4.2	Per-Channel Soft Reset

Individual channels can be reset without affecting others:

Waveform 12

## 19.5	Use Case Examples

### 19.5.1	Mixed Traffic Pattern

Typical network application with different traffic types:

Mixed Traffic Channel Assignment

| Channel | Traffic Type | Direction | Priority | Notes |
| --- | --- | --- | --- | --- |
| 0 | Voice/RTP | Sink | High | Low latency required |
| 1 | Voice/RTP | Source | High | Low latency required |
| 2 | Video | Sink | Medium | High bandwidth |
| 3 | Control | Source | Medium | Small packets |
| 4-7 | Bulk data | Both | Low | Background transfers |

### 19.5.2	Bidirectional Transfer

Single network connection using paired channels:

### 19.5.3	Figure 4.4.3: Bidirectional Transfer

Bidirectional Transfer

Source: ch04_bidirectional.dot

### 19.5.4	Load Balancing

Multiple channels processing same traffic type:

### 19.5.5	Figure 4.4.4: Load Balancing by tid

Load Balancing by tid

Source: ch04_load_balancing.dot

## 19.6	Performance Considerations

### 19.6.1	Bandwidth Sharing

Bandwidth Distribution

| Active Channels | Per-Channel BW (% of max) | Notes |
| --- | --- | --- |
| 1 | 100% | Full bandwidth available |
| 2 | ~50% each | Round-robin sharing |
| 4 | ~25% each | Fair distribution |
| 8 | ~12.5% each | All channels active |

Note: Actual distribution depends on transfer sizes and memory latency.

### 19.6.2	Latency Impact

Multi-channel operation adds arbitration latency:

Arbitration Latency

| Scenario | Additional Latency |
| --- | --- |
| Single channel | 0 cycles |
| 2 channels contending | 1-2 cycles |
| 8 channels contending | Up to 7 cycles |

## 19.7	MonBus Events

Each channel generates independent MonBus events:

Waveform 13

Events are arbitrated and serialized on the shared MonBus output.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Descriptor Format

## 20.1	Overview

RAPIDS uses 256-bit (32-byte) descriptors to define DMA transfers. Descriptors are stored in system memory and fetched by the Descriptor Engine when a channel is kicked. The layout is the layout RAPIDS Beats uses, bit for bit. What changed is the meaning of two things: the length field counts BYTES, and src_addr / dst_addr are byte addresses with no alignment requirement.

A descriptor therefore describes a transfer the way software thinks of it. A packet of 37 bytes at address 0x1007 is a descriptor with length = 37 and dst_addr = 0x1007. The hardware turns that into beats, byte enables and a partial last beat; software never does the arithmetic.

## 20.2	Descriptor Layout

### 20.2.1	256-Bit Structure

Bit Range    Field Name           Width  Description
────────────────────────────────────────────────────────────────────────
[255:213]    reserved             43     Reserved (write 0)
[212:210]    desc_type            3      0 = LEGACY, 1 = EXTENDED (see below)
[209:208]    opcode               2      0 = DATA, 1 = CTRL_READ, 2 = CTRL_WRITE
[207:200]    priority             8      Transfer priority (informational)
[199:196]    channel_id           4      Channel ID (informational)
[195]        error                1      Error flag (written by hardware)
[194]        last                 1      Last descriptor in chain
[193]        gen_irq              1      Generate interrupt on completion
[192]        valid                1      Descriptor valid flag
[191:160]    next_descriptor_ptr  32     Address of next descriptor (0 = last)
[159:128]    length               32     Transfer length in BYTES
[127:64]     dst_addr             64     Destination byte address
[63:0]       src_addr             64     Source byte address

: 256-Bit Descriptor Layout (rapids_pkg::descriptor_t)

The authoritative definition is rapids_pkg::descriptor_t in rtl/includes/rapids_pkg.sv. Only bits [255:0] are fetched and used. Every field position is the same as in RAPIDS Beats, so a descriptor builder needs one change: the value written to length.

## 20.3	Field Descriptions

### 20.3.1	Control Bits [195:192]

Control Bits

| Bit | Field | Description |
| --- | --- | --- |
| 192 | valid | Must be 1; a descriptor with valid=0 puts the channel in the error state |
| 193 | gen_irq | Set to 1 to generate a completion interrupt |
| 194 | last | Set to 1 for the last descriptor in a chain |
| 195 | error | Error flag; written by hardware, write 0 |

### 20.3.2	Direction

There is no direction field in the descriptor. Direction is a property of the half that owns the channel. The SOURCE half (memory to AXIS) reads from src_addr. The SINK half (AXIS to memory) writes to dst_addr. A descriptor is delivered to one half’s descriptor engine by the address it was staged at, so the same layout serves both. Each half looks only at its own address, so the two offsets never have to agree.

### 20.3.3	Transfer Length [159:128]

- 32-bit field specifying the transfer size in BYTES.

- The length is the packet length. On the sink it is the number of bytes the AXIS packet must carry. On the source it is the number of bytes in the one AXIS packet the descriptor produces.

- A length of 0 moves nothing and carries no packet record (see Alignment and Offsets).

### 20.3.4	Next Pointer [191:160]

- 32-bit address of the next descriptor, zero-extended to the engine’s 64-bit address width.

- A value of 0 terminates the chain, as does last = 1.

- Autonomous chaining also requires the address to fall inside one of the two configured descriptor address ranges.

Chain traversal is unchanged from RAPIDS Beats. See the shared Descriptor Chaining chapter.

### 20.3.5	Address Fields [127:0]

For SINK transfers (network to memory):

| Field | Usage |
| --- | --- |
| dst_addr [127:64] | Memory write destination, any byte address |
| src_addr [63:0] | Not used (write 0) |

For SOURCE transfers (memory to network):

Address Field Usage

| Field | Usage |
| --- | --- |
| dst_addr [127:64] | Not used (write 0) |
| src_addr [63:0] | Memory read source, any byte address |

## 20.4	Alignment and Offsets

Let BYTE_LANES be the beat size in bytes (DATA_WIDTH / 8, 32 for the 256-bit build) and OFF_W its log2 (5 for 256 bits). For a linear DATA descriptor:

offset      = addr[OFF_W-1:0]                    (the low bits of the byte address)
beats_total = 0                                   if length == 0
            = (offset + length + BYTE_LANES - 1) >> OFF_W   otherwise

beats_total is the number of whole memory beats the transfer touches. The sum is computed with a 33-bit intermediate, so a length near 4 GB does not overflow. Each half applies the formula to its own address: the source uses src_addr, the sink uses dst_addr.

Beat Count Examples (BYTE_LANES = 32)

| Length (bytes) | Offset | beats_total | Useful bytes / (beats x 32) |
| --- | --- | --- | --- |
| 0 | any | 0 | no transfer |
| 1 | 0 | 1 | 0.031 |
| 2 | 31 | 2 | 0.031 |
| 32 | 0 | 1 | 1.000 |
| 32 | 1 | 2 | 0.500 |
| 37 | 0 | 2 | 0.578 |
| 60 | 5 | 3 | 0.625 |
| 100 | 7 | 4 | 0.781 |
| 203 | 3 | 7 | 0.906 |

The last column is the fraction of the bus the transfer uses. A transfer that is a whole number of beats and starts on a beat boundary is 1.000. Everything else pays for the beats it touches, not the bytes it carries.

### 20.4.1	How the offset reaches the bus

- AXI address. AxADDR is always issued beat-aligned. The offset never appears in the address. Address progression inside a transfer is the aligned-down start plus a beat count times BYTE_LANES.

- Sink writes. The first beat’s WSTRB leaves lanes 0 to offset-1 clear, and the last beat’s WSTRB covers only the bytes that remain. Bytes outside the packet are never written, so a sub-beat transfer does not disturb its neighbors in memory.

- Source reads. The read master fetches whole beats from the aligned address. The egress re-packer drops the leading offset bytes and packs the rest, so the AXIS packet starts at byte lane 0.

- AXIS. The stream is packed. Only the last beat is partial: TSTRB is contiguous from lane 0 and TLAST is on that beat. See the AXIS Interface chapter.

### 20.4.2	4 KB boundaries

A burst never crosses a 4 KB boundary. The engines cut a burst at the boundary, so a transfer that straddles one is issued as more than one burst. This is a protocol rule handled in hardware, not something software arranges. Example: 203 bytes at an address whose low 12 bits are 0xFC3 has offset 3 and 7 beats. The first two beats (aligned addresses 0xFC0 and 0xFE0) end at the boundary. The remaining five beats start at the next 4 KB page.

### 20.4.3	Zero-length descriptors

A descriptor with length = 0 moves nothing. It issues no read or write request and it carries no packet record, whatever its addresses are. It still occupies its place in the chain, so last, next_descriptor_ptr and gen_irq are honored. A zero-length DATA descriptor is therefore a legal way to report a completion event or end a chain without moving data.

### 20.4.4	Packet record

Every non-zero DATA descriptor produces one packet record, {bytes, offset}, for each enabled direction. The data path uses the record to place bytes on the sink or to frame the packet on the source. The record is pulsed once, when the scheduler leaves the fetch state to start the transfer.

- Each channel’s record queue is PQ_DEPTH deep (4). A descriptor whose record does not fit waits. This is backpressure and not an error.

- The sink ingress does not accept the first beat of a packet until the channel’s record exists. Issue the descriptor before the packet arrives, or send them concurrently.

- s_axis_tready is one signal, qualified by the tid of the beat on the bus. A beat for a channel with no record stalls the whole stream, including beats for other channels queued behind it. The per-channel record queues and ingress holds keep the data intact, so the effect is delay and not corruption. A stream that must not head-of-line block should have every channel’s descriptor issued before the stream starts.

## 20.5	Descriptor Opcodes (DATA / CTRL_READ / CTRL_WRITE)

The 2-bit opcode selects one of three behaviors. Only DATA moves payload, and only DATA is affected by the byte-granular changes.

Descriptor Opcodes

| Opcode | Value | Behavior |
| --- | --- | --- |
| DATA | 2'b00 | Payload transfer; length in bytes, byte addresses |
| CTRL_READ | 2'b01 | Consumer gate: poll a memory location until (read & mask) == expected, then release the chain |
| CTRL_WRITE | 2'b10 | Producer doorbell: one 32-bit write to a memory location, then continue |

The CTRL_READ and CTRL_WRITE field layouts are unchanged from RAPIDS Beats, and they carry no packet record. See the RAPIDS Beats descriptor chapter for the field tables and the retry limit.

## 20.6	Extended Descriptors (row/col-major addressing)

desc_type = 1 (EXTENDED) selects strided, 2-D or circular addressing instead of linear accumulation. It is enabled at build time by USE_ROW_COL_MAJOR_ADDRESSING, which defaults to 1. An extended descriptor is 512 bits in two 256-bit chunks. Chunk 0 is the layout above with desc_type = 1. Chunk 1 is fetched by a second single-beat read at descriptor_addr + 0x20, so the two chunks must be contiguous in memory.

Chunk 1 (at descriptor_addr + 0x20)
Bit Range    Field Name        Width  Description
──────────────────────────────────────────────────────────────────────
[255:192]    reserved_hi       64     Reserved (future 3rd dimension)
[191:188]    wr_reserved       4      Reserved
[187:182]    wr_wrap1_log2     6      Write outer wrap window, log2 (0 = off)
[181:176]    wr_wrap0_log2     6      Write inner wrap window, log2 (0 = off)
[175:160]    wr_inner_count    16     Write beats per contiguous run
[159:128]    wr_stride_1       32     Write outer stride, SIGNED bytes
[127:96]     wr_stride_0       32     Write inner stride, SIGNED bytes
[95:92]      rd_reserved       4      Reserved
[91:86]      rd_wrap1_log2     6      Read outer wrap window, log2 (0 = off)
[85:80]      rd_wrap0_log2     6      Read inner wrap window, log2 (0 = off)
[79:64]      rd_inner_count    16     Read beats per contiguous run
[63:32]      rd_stride_1       32     Read outer stride, SIGNED bytes
[31:0]       rd_stride_0       32     Read inner stride, SIGNED bytes

: Extended Descriptor Chunk 1 (rapids_pkg::descriptor_ext_t)

The mode selection, the address equation and the signed strides are unchanged from RAPIDS Beats. The layout is byte-compatible with STREAM’s descriptor_ext_t.

### 20.6.1	EXT descriptors stay beat-aligned (permanent limitation)

TYPE=EXT is not byte-granular, and it is not going to be. This is a deliberate, permanent limitation of RAPIDS. An extended descriptor must have:

- src_addr and dst_addr aligned to BYTE_LANES (32 bytes for the 256-bit build).

- A length that is a multiple of BYTE_LANES.

Row and column striding are counted in whole beats, so a run of inner_count beats is inner_count x BYTE_LANES bytes, and the strides step between beat-aligned rows. An EXT descriptor with an unaligned address or a length that is not a beat multiple is outside the contract and is not checked. Use a linear descriptor when the transfer starts or ends mid-beat. The byte-granular path (offsets, byte enables, partial last beat, packet records) applies to linear descriptors only.

## 20.7	Descriptor Fetch

Fetch is unchanged from RAPIDS Beats. The Descriptor Engine issues a single-beat AXI4 read (arlen = 0) at the descriptor address, and again at +0x20 for an EXTENDED descriptor. The parsed descriptor is handed to the scheduler, which converts length and the address to beats_total, the record and the address working registers as described above. The descriptor address and next_descriptor_ptr are 32-byte aligned; that is the only alignment rule that remains for linear descriptors.

## 20.8	Alignment Requirements

Address and Length Alignment

| Field | Alignment | Notes |
| --- | --- | --- |
| Descriptor address | 32-byte | One descriptor is 32 bytes; an EXTENDED descriptor’s chunk 1 is fetched at +0x20 |
| next_ptr [191:160] | 32-byte | 32-bit descriptor address, zero-extended by the engine |
| dst_addr [127:64], linear | none | Any byte address |
| src_addr [63:0], linear | none | Any byte address |
| dst_addr / src_addr, EXT | DATA_WIDTH/8 | Permanent: EXT is beat-aligned |
| length, linear | none | Any byte count; 0 moves nothing |
| length, EXT | multiple of DATA_WIDTH/8 | Permanent: EXT counts whole beats |

## 20.9	What Changed from RAPIDS Beats

Descriptor Differences, RAPIDS Beats to RAPIDS

| Aspect | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Descriptor size and bit layout | 256 bits | Same |
| length unit | beats | bytes |
| length = 0 | Reserved | Legal: moves nothing, no record, chain continues |
| src_addr / dst_addr alignment (linear) | DATA_WIDTH/8 | None |
| Beats moved | length | (offset + length + BYTE_LANES - 1) >> OFF_W |
| Sink write strobes | All lanes | WSTRB marks the bytes written |
| Source packet | Full beats | One packed packet per descriptor, partial last beat |
| Per-descriptor packet record | None | {bytes, offset} per enabled direction |
| Sink accepts a packet | Before the descriptor | After the channel’s record exists |
| CTRL_READ / CTRL_WRITE | Defined | Unchanged |
| TYPE=EXT | Beat rows | Beat rows, aligned addresses and lengths (permanent) |
| Chaining, next_ptr, last, gen_irq | Defined | Unchanged |

## 20.10	Descriptor Examples

### 20.10.1	Sink Descriptor (Network to Memory)

A 100-byte sink transfer to 0x1_0000_0007, chained to a descriptor at 0x2000, raising an interrupt on completion. The destination is 7 bytes into a beat, so offset = 7 and beats_total = (7 + 100 + 31) >> 5 = 4. The first beat writes lanes 7 to 31, the last beat writes 4 bytes.

64-bit words, LSB word first:
[63:0]    = 0x0000_0000_0000_0000  // src_addr  (unused by the SINK half)
[127:64]  = 0x0000_0001_0000_0007  // dst_addr  = 0x1_0000_0007
[191:128] = 0x0000_2000_0000_0064  // next_ptr[191:160]=0x2000, length[159:128]=100
[255:192] = 0x0000_0000_0000_0003  // valid=1, gen_irq=1, last=0

### 20.10.2	Source Descriptor (Memory to Network)

A 60-byte source transfer from 0x2_0000_0005, terminating the chain. offset = 5 and beats_total = (5 + 60 + 31) >> 5 = 3 memory beats. The source emits one AXIS packet of two stream beats: 32 bytes, then 28 bytes with TLAST.

64-bit words, LSB word first:
[63:0]    = 0x0000_0002_0000_0005  // src_addr = 0x2_0000_0005
[127:64]  = 0x0000_0000_0000_0000  // dst_addr (unused by the SOURCE half)
[191:128] = 0x0000_0000_0000_003C  // next_ptr = 0, length = 60
[255:192] = 0x0000_0000_0000_0007  // valid=1, gen_irq=1, last=1

## 20.11	Software Construction

### 20.11.1	C Structure Example

typedef struct __attribute__((packed, aligned(32))) {
    uint64_t src_addr;      // [63:0]     source byte address
    uint64_t dst_addr;      // [127:64]   destination byte address
    uint32_t length;        // [159:128]  transfer length in BYTES
    uint32_t next_ptr;      // [191:160]  32-bit, 0 terminates the chain
    uint32_t flags;         // [223:192]  see macros below
    uint32_t reserved[2];   // [255:224]
} rapids_descriptor_t;

// flags bit positions are relative to descriptor bit 192
#define DESC_VALID          (1u << 0)   // [192]
#define DESC_GEN_IRQ        (1u << 1)   // [193]
#define DESC_LAST           (1u << 2)   // [194]
#define DESC_ERROR          (1u << 3)   // [195] hardware-written
#define DESC_CHANNEL_ID(n)  (((n) & 0xFu) << 4)    // [199:196]
#define DESC_PRIORITY(n)    (((n) & 0xFFu) << 8)   // [207:200]
#define DESC_OPCODE(n)      (((n) & 0x3u) << 16)   // [209:208]
#define DESC_TYPE(n)        (((n) & 0x7u) << 18)   // [212:210]

#define DESC_OP_DATA        0x0
#define DESC_OP_CTRL_READ   0x1
#define DESC_OP_CTRL_WRITE  0x2

#define DESC_TYPE_LEGACY    0x0
#define DESC_TYPE_EXT       0x1

// Beats a linear DATA descriptor touches (BYTE_LANES = 32 for 256-bit)
static inline uint32_t rapids_beats_total(uint64_t addr, uint32_t length)
{
    const uint64_t lanes = 32;
    if (length == 0)
        return 0;
    return (uint32_t)(((addr & (lanes - 1)) + length + lanes - 1) / lanes);
}

The helper is for software that wants to size buffers or predict burst counts. The hardware does not need it.

### 20.11.2	Descriptor Ring Setup

// Descriptor ring: descriptors must be 32-byte aligned
rapids_descriptor_t *ring = aligned_alloc(32, NUM_DESC * sizeof(*ring));

for (int i = 0; i < NUM_DESC; i++) {
    memset(&ring[i], 0, sizeof(ring[i]));
    ring[i].dst_addr = dst[i];               // any byte address
    ring[i].length   = len[i];               // bytes
    ring[i].flags    = DESC_VALID | DESC_GEN_IRQ;
    if (i < NUM_DESC - 1) {
        ring[i].next_ptr = (uint32_t)(uintptr_t)&ring[i + 1];
    } else {
        ring[i].next_ptr = 0;                // terminates the chain
        ring[i].flags   |= DESC_LAST;
    }
}

The descriptor buffers themselves stay 32-byte aligned. The payload addresses do not have to be.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Register Map

## 21.1	Overview

RAPIDS Beats provides a memory-mapped register interface for configuration and status. All registers are accessed through the single top-level APB slave (s_apb_*) and are implemented by the PeakRDL-generated rapids_regs register block. One addrmap holds two independent halves of the same layout (rapids_engine_regs): SRC at 0x0000 and SNK at 0x1000, 4 KB each. Within a half, the staged descriptor addresses occupy +0x000-0x03F, the launch register KICK_ENABLE sits at +0x040, configuration and status at +0x100-0x3FF, and the nested monitor regfile (rapids_mon_regs) at +0x800. Every offset below is relative to its half; add 0x1000 for the SNK copy.

Because the SNK half lives at 0x1000, the APB address bus must be at least 13 bits wide; paddr[12] selects the half and paddr[11] the monitor window.

## 21.2	Address Space Layout

| Range (per half) | Target | Purpose |
| --- | --- | --- |
| +0x000-0x03F | rapids_engine_regs | Per-channel staged descriptor addresses |
| +0x040 | rapids_engine_regs | KICK_ENABLE per-channel launch bits |
| +0x100-0x3FF | rapids_engine_regs | Configuration and status |
| +0x800-0x9FF | rapids_mon_regs | AXI-monitor config and performance |
| 0x0000 / 0x1000 | half base | SRC half / SNK half |

The three *_PKT_MASK registers are masks: bit[type] = 1 masks (drops) that packet type, at the monitor-lite and again at the monbus group. They reset to all-masked (0xFFFF), so nothing reaches the group until the host clears the bits of the classes it wants to see. (rapids BUG-008: the description used to read “1 = enable”; the polarity and the reset are as intended.) *_TIMEOUT counts MICROSECONDS in its low 16 bits.

On a build with USE_MON_REGS = 0 (the default when USE_AXI_MONITORS = 0) the two monitor windows keep their addresses but answer every access with PSLVERR and zero read data: the config blocks drive nothing from them, and a read-back that succeeded would tell the host a monitor is configured that does not exist.

: Address Space Layout

## 21.3	Base Registers (0x100-0x3FF)

Base Registers

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | RW | GLOBAL_EN[0], GLOBAL_RST[1] (level; write 0 to release) |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE[0] |
| 0x108 | VERSION | RO | MINOR[7:0], MAJOR[15:8], NUM_CHANNELS[23:16]=8 |
| 0x120 | CHANNEL_ENABLE | RW | CH_EN[7:0] per-channel enable |
| 0x124 | CHANNEL_RESET | RW | CH_RST[7:0] per-channel reset (level; write 0 to release) |
| 0x140 | CHANNEL_IDLE | RO | CH_IDLE[7:0] – not driven at the top level, reads 0; use SCHEDULER_IDLE |
| 0x144 | DESC_ENGINE_IDLE | RO | Per-channel descriptor-engine idle |
| 0x148 | SCHEDULER_IDLE | RO | Per-channel scheduler idle (excludes CH_ERROR) |
| 0x150-0x16C | CH_STATE[0..7] | RO | Per-channel FSM state (stride 0x4) |
| 0x170 | SCHED_ERROR | RO | Per-channel sticky scheduler error |
| 0x174 | AXI_RD_COMPLETE | RO | Per-channel read-complete (SRC half only; reads 0 in SNK) |
| 0x178 | AXI_WR_COMPLETE | RO | Per-channel write-complete – not driven, reads 0 |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | Write-progress timeout window (cycles, reset 1000) |
| 0x204 | SCHED_CONFIG | RW | SCHED_EN, TIMEOUT_EN, ERR_EN, COMPL_EN, PERF_EN |
| 0x208 | SCHED_TIMEOUT_LIMIT | RW | LIMIT[7:0] escalation limit (reset 4, 0 = never) |
| 0x220 | DESCENG_CONFIG | RW | DESCENG_EN[0], PREFETCH_EN[1], FIFO_THRESH[5:2] (reset 8) |
| 0x224 | DESCENG_ADDR0_BASE | RW | Descriptor address range 0 base [31:0] |
| 0x228 | DESCENG_ADDR0_LIMIT | RW | Descriptor address range 0 limit [31:0] |
| 0x22C | DESCENG_ADDR1_BASE | RW | Descriptor address range 1 base [31:0] |
| 0x230 | DESCENG_ADDR1_LIMIT | RW | Descriptor address range 1 limit [31:0] |
| 0x240 | CTRL_CONFIG | RW | CTRLRD_MAX_TRY[8:0] control-read poll retry budget (0-511, reset 16) |
| 0x2A0 | AXI_XFER_CONFIG | RW | AXI transfer sizing configuration |
| 0x2B0 | PERF_CONFIG | RW | Performance profiler configuration |
| 0x2C0-0x2CC | OBS_CTRL/OBS_FLAGS/OBS_DATA0/OBS_DATA1 | RW/RO | Observation mux |
| 0x35C | PERF_CH_SEL | RW | Performance channel select |
| 0x378-0x380 | HIST_SEL/HIST_DATA/HIST_TOTAL | RW/RO | Latency histogram |

## 21.4	Monitor Registers (+0x800 within each half)

Monitor Registers (+0x800 within each half; SRC at 0x0800, SNK at 0x1800)

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| +0x800 | MON_FIFO_STATUS | RO | MonBus capture/error FIFO status |
| +0x804 | MON_FIFO_COUNT | RO | MonBus FIFO occupancy |
| +0x8C0-0x8DC | DAXMON_* | RW | Descriptor-monitor config (enable/timeout/latency/masks) |
| +0x8E0-0x8FC | RDMON_* | RW | Source-egress AXIS monitor-lite config (same layout; live in the SRC half, rapids TASK-015) |
| +0x900-0x91C | WRMON_* | RW | Sink-ingress AXIS monitor-lite config (same layout; live in the SNK half, rapids TASK-015; the SRC copy’s COMPRESS_EN drives the group) |
| +0x950-0x978 | DAXMON_PERF_* | RO/RW | Descriptor-monitor performance counters |
| +0x980-0x9A8 | RDMON_PERF_* | RO/RW | Read-monitor performance counters |
| +0x9B0-0x9D8 | WRMON_PERF_* | RO/RW | Write-monitor performance counters |
| +0x9E0-0x9F4 | {RD,WR}MON_PERF_CH_* | RO | Per-channel producer/backpressure/starve/idle/overflow |

## 21.5	Key Register Fields

### 21.5.1	GLOBAL_CTRL (0x100) - Read/Write

GLOBAL_CTRL Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [0] | GLOBAL_EN | 0 | Master enable for the entire engine |
| [1] | GLOBAL_RST | 0 | Global reset: holds every channel of the half in reset while 1. Not self-clearing – write 0 to release |
| [31:2] | Reserved | 0 | Reserved |

### 21.5.2	SCHED_TIMEOUT_LIMIT (0x208) - Read/Write

SCHED_TIMEOUT_LIMIT Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [7:0] | LIMIT | 4 | Consecutive write-progress timeout windows before escalating to a fatal, sticky CH_ERROR. 0 = never escalate. Total escalation time ~= LIMIT x SCHED_TIMEOUT_CYCLES. |

### 21.5.3	DESCENG_CONFIG (0x220) - Read/Write

DESCENG_CONFIG Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [0] | DESCENG_EN | 1 | Descriptor engine master enable |
| [1] | PREFETCH_EN | 0 | Prefetch chaining (0 = on-demand, ~1 ahead) |
| [5:2] | FIFO_THRESH | 8 | Descriptors buffered ahead when prefetch enabled |
| [31:6] | Reserved | 0 | Reserved |

Register Access Timing

Waveform 15

## 21.6	Channel Kick Sequence

Staging a channel’s descriptor address, then writing that channel’s bit in KICK_ENABLE, triggers descriptor processing:

Waveform 16

## 21.7	Address Decoding

Registers are global (not per-channel windows); per-channel fields are packed as bit lanes within a single 32-bit register (e.g. CHANNEL_ENABLE.CH_EN[7:0], SCHEDULER_IDLE.SCHED_IDLE[7:0]). The only per-channel array is CH_STATE[0..7] at 0x150-0x16C (stride 0x4). Descriptor kick-off is part of this register block: staged addresses at 0x000-0x03F (CH*_DESC_ADDR_{LOW,HIGH}, stride 0x8) and the KICK_ENABLE launch register at 0x040.

+0x000 - 0x03F: Staged descriptor addresses (CH*_DESC_ADDR_{LOW,HIGH})
+0x040        : KICK_ENABLE (per-channel launch, singlepulse)
+0x100 - 0x3FF: Base configuration / status registers
+0x800 - 0x9FF: Monitor configuration / performance registers
SRC half at 0x0000, SNK half at 0x1000 (paddr[12]); PSLVERR on the monitor
window when USE_MON_REGS = 0.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Initialization Sequence

## 22.1	Overview

This section describes how software brings up RAPIDS Beats and starts a transfer. Every register named here is a register of the PeakRDL rapids_engine_regs register file (rtl/macro_beats/rapids_engine_regs.rdl), using the names in the generated per-instance map rapids_regmap.py. The two halves share one layout: SRC at 0x0000 and SNK at 0x1000. Each half is initialized on its own, and the offsets in the Register Map are relative to the half.

The code below uses reg_wr(half, NAME, value) and reg_rd(half, NAME) as shorthand for an APB access to register NAME in the SRC or SNK half. Resolve the names through the generated map instead of hard-coding offsets.

## 22.2	Boot Sequence

### 22.2.1	Figure 5.3.1: Bring-Up Sequence

Bring-Up Sequence

Source: ch05_boot_sequence.dot

## 22.3	What the Hardware Requires

A channel runs a descriptor only when every one of these terms is true. They are ANDed in rapids_config_block, so a missing one leaves the channel idle without raising an error:

Conditions for a Channel to Run

| Term | Register.field | Reset | Effect when 0 |
| --- | --- | --- | --- |
| Global enable | GLOBAL_CTRL.GLOBAL_EN | 0 | Masks CH_EN, SCHED_EN, DESCENG_EN and every monitor enable in the half |
| Channel enable | CHANNEL_ENABLE.CH_EN[n] | 0 | The scheduler stays in CH_IDLE with a descriptor waiting |
| Scheduler enable | SCHED_CONFIG.SCHED_EN | 1 | All schedulers in the half are disabled |
| Descriptor engine enable | DESCENG_CONFIG.DESCENG_EN | 1 | No descriptor is fetched |
| Descriptor window | DESCENG_ADDR0_BASE/_LIMIT, DESCENG_ADDR1_BASE/_LIMIT | 0 / 0xFFFFFFFF | A descriptor address outside both windows is refused and the channel errors |
| Kick | KICK_ENABLE.KICKn | – | Nothing is fetched until the bit is written |

GLOBAL_EN and CH_EN both reset to 0. The other enables reset to 1, and both descriptor windows reset to the full 32-bit range, so a minimal bring-up only has to write CHANNEL_ENABLE, GLOBAL_CTRL, the staged address and KICK_ENABLE.

## 22.4	Detailed Steps

### 22.4.1	Step 1: Verify Hardware Presence

// VERSION (0x108): MINOR[7:0], MAJOR[15:8], NUM_CHANNELS[23:16]
uint32_t version = reg_rd(SRC, VERSION);
uint8_t minor    = (version >>  0) & 0xFF;   // 0x5A (90) in this release
uint8_t major    = (version >>  8) & 0xFF;   // 0x00: pre-release, version 0.90
uint8_t num_ch   = (version >> 16) & 0xFF;   // 8

if (num_ch == 0) {
    printf("ERROR: RAPIDS not detected\n");
    return -1;
}

There is no separate configuration register. The channel count is the only build parameter that software can read back. Data width and SRAM depth are not readable from the register file.

### 22.4.2	Step 2: Optional Reset

GLOBAL_CTRL.GLOBAL_RST and CHANNEL_RESET.CH_RST[n] are ordinary read/write bits. They do not self-clear. A channel is held in reset (scheduler forced to CH_IDLE, descriptor engine reset) for as long as its bit, or GLOBAL_RST, reads 1. Software must write the bit back to 0 to release it.

// Pulse the global reset: set, then clear. Keep GLOBAL_EN at 0 meanwhile.
reg_wr(half, GLOBAL_CTRL, GLOBAL_CTRL_GLOBAL_RST);   // bit 1
reg_wr(half, GLOBAL_CTRL, 0);

// Every scheduler should now report idle (SYSTEM_IDLE = AND of SCHED_IDLE).
while (!(reg_rd(half, GLOBAL_STATUS) & 1)) { }

### 22.4.3	Step 3: Configure the Engine

// Scheduler: SCHED_EN[0] TIMEOUT_EN[1] ERR_EN[2] COMPL_EN[3] PERF_EN[4]
reg_wr(half, SCHED_TIMEOUT_CYCLES, 1000);   // write-progress window, in clocks
reg_wr(half, SCHED_TIMEOUT_LIMIT,  4);      // fatal after 4 stalled windows; 0 = never
reg_wr(half, SCHED_CONFIG, SCHED_EN | TIMEOUT_EN | ERR_EN | COMPL_EN);

// Descriptor engine: DESCENG_EN[0] PREFETCH_EN[1] FIFO_THRESH[5:2]
reg_wr(half, DESCENG_CONFIG, DESCENG_EN | PREFETCH_EN | (4 << 2));

// Descriptor address windows (lower 32 bits). A fetch address outside
// both windows is refused and the channel enters CH_ERROR.
reg_wr(half, DESCENG_ADDR0_BASE,  desc_region_base);
reg_wr(half, DESCENG_ADDR0_LIMIT, desc_region_base + desc_region_size - 1);

// Burst sizing: RD_XFER_BEATS[7:0] WR_XFER_BEATS[15:8]
//               ALLOC_SIZE[23:16]  DRAIN_SIZE[31:24]
reg_wr(half, AXI_XFER_CONFIG, (16 << 24) | (16 << 16) | (8 << 8) | 8);

The SRC half uses the read-side fields and the SNK half uses the write-side fields. Writing the whole word in both halves does no harm.

### 22.4.4	Step 4: Optional Monitor and Interrupt Egress

The only interrupt output is mon_irq. It comes from the top-level monbus_axil4_axil4_group and is asserted while that group’s error FIFO is not empty. The scheduler error and completion packets travel on the CORE protocol. For both halves, the group takes its CORE filter from the SRC half’s WRMON_* registers:

// CORE packets: PKT_MASK bit[type] = 1 DROPS that type; reset is 0xFFFF.
reg_wr(SRC, WRMON_PKT_MASK, ~((1u << 0) | (1u << 1)) & 0xFFFF);  // pass Error, Completion

// ERR_SELECT[3:0] bit[type] = 1 routes that type to the error FIFO (-> mon_irq).
// Bit 0 = Error.  Bit 1 = Completion, which includes the gen_irq descriptor event.
reg_wr(SRC, WRMON_ERR_CFG, (0xFF << 8) | (1u << 0));             // keep ERR_MASK reset value

Packets that are passed but not selected for the error FIFO go to the capture path (m_axil_mon_*). The group, mon_irq and the s_axil_err_* drain exist only when USE_AXI_MONITORS = 1. With USE_AXI_MONITORS = 0, mon_irq is tied to 0, and software must poll SCHED_ERROR and SCHEDULER_IDLE instead.

### 22.4.5	Step 5: Build Descriptors

// rapids_pkg::descriptor_t, 256 bits (see Descriptor Format)
for (int i = 0; i < NUM_RX_DESC; i++) {
    desc[i].src_addr            = 0;                        // unused by a sink transfer
    desc[i].dst_addr            = rx_buffer + i * FRAME_SIZE;
    desc[i].length              = FRAME_LEN;   // BEATS in RAPIDS Beats, BYTES in byte-granular RAPIDS
    desc[i].next_descriptor_ptr = (i < NUM_RX_DESC - 1) ? (uint32_t)&desc[i + 1] : 0;
    desc[i].valid               = 1;                        // 0 -> CH_ERROR
    desc[i].last                = (i == NUM_RX_DESC - 1);
    desc[i].gen_irq             = 1;                        // completion event RAPIDS_EVENT_IRQ
}
__sync_synchronize();   // descriptors visible before the kick

The descriptors must lie inside a DESCENG_ADDR0/DESCENG_ADDR1 window.

### 22.4.6	Step 6: Enable

reg_wr(half, CHANNEL_ENABLE, 1u << ch);          // CH_EN[n]
reg_wr(half, GLOBAL_CTRL,    GLOBAL_CTRL_GLOBAL_EN);   // bit 0; GLOBAL_RST stays 0

Initialization Timing

cfg_channel_enable[n] is CH_EN[n] & GLOBAL_EN, so it asserts only after the last of the two writes.

## 22.5	Starting Transfers

### 22.5.1	Kick a Channel

Stage the descriptor address, then launch it with a single write to KICK_ENABLE. The address registers only store the value, so writing them does not start anything. Each KICKn bit is a singlepulse field and clears itself. One 32-bit write can launch several channels on the same clock.

reg_wr(half, CH0_DESC_ADDR_LOW,  (uint32_t)((uint64_t)&desc[0]));
reg_wr(half, CH0_DESC_ADDR_HIGH, (uint32_t)((uint64_t)&desc[0] >> 32));
reg_wr(half, KICK_ENABLE, 1u << 0);              // KICK0

The APB write completes as soon as the request is raised. The RTL holds the request until the descriptor engine accepts it, so a completed write does not mean the engine has accepted the kick.

Waveform 18

## 22.6	Completion and Interrupt Handling

There is no interrupt status register and no per-channel interrupt clear. Software sees completion in one of two ways:

- Polling. SCHEDULER_IDLE.SCHED_IDLE[n] returns to 1 when the scheduler is back in CH_IDLE after the last descriptor of a chain. A channel in CH_ERROR never reports idle, and SCHED_ERROR.SCHED_ERR[n] reads 1 for it instead. Poll both registers, or the loop will hang on a faulted channel. Do not poll CHANNEL_IDLE, AXI_WR_COMPLETE, or the SNK half’s AXI_RD_COMPLETE. Those fields are not driven at the top level and always read 0 (see the Register Map).

- mon_irq. If Step 4 routed Completion packets to the error FIFO, every descriptor completion lands there, and so does every error. A completion is reported as RAPIDS_EVENT_DESC_COMPLETE, or as RAPIDS_EVENT_IRQ when the descriptor’s gen_irq is set. The descriptor engine’s descriptor-loaded completions land there too. mon_irq stays high until the records are drained over s_axil_err_*.

void rapids_irq_handler(void) {
    // mon_irq == error FIFO not empty: drain every record
    while (mon_irq_pending()) {
        mon_record_t r = err_fifo_read();        // s_axil_err_* reads
        if (r.pkt_type == PKT_TYPE_ERROR) {
            handle_channel_error(r.half, r.channel_id);   // see Error Handling
        } else if (r.pkt_type == PKT_TYPE_COMPLETION) {
            process_completed_buffer(r.half, r.channel_id, r.event_data);
        }
    }
}

To run the channel again, stage the next chain’s first descriptor address and write its KICK_ENABLE bit. No re-enable is needed, because CH_EN stays set.

## 22.7	Reset and Recovery

### 22.7.1	Per-Channel Reset

void reset_channel(int half, int ch) {
    // Assert, then release: CH_RST is NOT self-clearing.
    reg_wr(half, CHANNEL_RESET, 1u << ch);
    reg_wr(half, CHANNEL_RESET, 0);

    while (!(reg_rd(half, SCHEDULER_IDLE) & (1u << ch))) { }
    // SCHED_ERROR[ch] is the CH_ERROR state, so it reads 0 again here.
}

The scheduler’s error state is sticky. The only ways out of CH_ERROR are a channel reset (CH_RST[n]) or a global reset (GLOBAL_RST). The reset leaves CH_EN[n] and the staged descriptor address unchanged. After the reset, re-stage the address if it should change, then kick.

### 22.7.2	Global Reset

void reset_rapids_half(int half) {
    reg_wr(half, GLOBAL_CTRL, GLOBAL_CTRL_GLOBAL_RST);   // GLOBAL_EN = 0, RST = 1
    reg_wr(half, GLOBAL_CTRL, 0);                        // release
    while (!(reg_rd(half, GLOBAL_STATUS) & 1)) { }       // SYSTEM_IDLE
    rapids_init_half(half);                              // Steps 3-6 again
}

GLOBAL_RST drives every channel’s reset in that half only. The other half, and the shared monitor group’s error FIFO, are not affected.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Error Handling

## 23.1	Overview

RAPIDS reports errors per channel and per direction. The error classes are the ones RAPIDS Beats already has, and the byte-granular design adds one: the packet on the sink stream does not carry the number of bytes the descriptor asked for. This chapter lists every condition that stops a channel, how software sees it, and how to recover. It also lists three conditions that look like errors and are not.

Two facts shape the whole chapter:

- A fatal error parks the channel. The scheduler enters its error state and stays there. Only a channel reset or a global reset leaves it.

- A channel reset recovers every class. The scheduler, the descriptor and control engines, both data engines and both data paths all take the per-channel reset. No error class needs the hardware reset aresetn, and resetting one channel does not disturb the others.

## 23.2	Error Classes

Error Classes

| Class | Detected by | Fatal to the channel | Sticky until |
| --- | --- | --- | --- |
| Invalid descriptor (valid = 0) | Scheduler, on CH_FETCH_DESC | Yes | Channel reset |
| Descriptor address outside both configured ranges | Descriptor engine, before any fetch | Yes | Channel reset |
| Descriptor address of zero or otherwise invalid over APB | Descriptor engine | Yes | Channel reset |
| Descriptor fetch returns an AXI error (RRESP not OKAY) | Descriptor engine | Yes | Channel reset |
| Data read returns an AXI error (RRESP not OKAY) | AXI read engine | Yes | Channel reset |
| Data write returns an AXI error (BRESP not OKAY) | AXI write engine | Yes | Channel reset |
| Sink packet length mismatch (new) | Sink ingress, at tlast | Yes | Channel reset |
| Control read retry exhausted | Control read engine | Yes | Channel reset |
| Control write returns an AXI error | Control write engine | Yes | Channel reset |
| Write-progress timeout, first windows | Scheduler timeout counter | No | Any write progress |
| Write-progress timeout, escalated | Scheduler timeout counter | Yes | Channel reset |

The first four rows and the control-engine rows behave exactly as in RAPIDS Beats. The data-path rows are the same conditions. In RAPIDS the channel reset also clears them; the “Recovery” section lists what it does to work still in flight.

## 23.3	The Byte-Granular Error: Packet Length Mismatch

### 23.3.1	Contract

A sink descriptor of length bytes on a channel promises one AXI-Stream packet on that channel. The packet is delimited by tlast. The number of valid bytes in the packet, counted from tstrb, must equal length.

The sink ingress counts the valid bytes of every accepted beat. On the beat that carries tlast it compares the running total, including that beat, with the byte count in the channel’s packet record. If the two differ, the ingress sets a sticky per-channel bit and raises it on the channel’s sched_wr_error line.

Length Mismatch Cases

| Situation | Result |
| --- | --- |
| Packet shorter than length (early tlast) | Error |
| Packet longer than length (late tlast) | Error |
| Packet equals length | No error |
| Descriptor length = 0 | No packet record, no packet expected |

The hardware does not pad a short packet and does not drop the extra bytes of a long one. Nothing is corrected silently. The bytes that were accepted are still written, so memory that the descriptor covers must not be trusted after this error.

### 23.3.2	Exposure

The mismatch is visible in three places.

Where the Length Mismatch Appears

| Where | What software sees |
| --- | --- |
| SNK.SCHED_ERROR.SCHED_ERR | Bit for the channel set. This is the same per-channel bit that reports every fatal scheduler error, so the register does not say which class occurred. |
| MonBus | One error packet per error episode from the channel’s scheduler. Its data field carries the sticky write-error flag. |
| Scheduler state | The channel is in its error state, so SCHEDULER_IDLE.SCHED_IDLE does not show it idle. |

The error is raised on the sink half only. The source half has no counterpart: the egress re-packer builds the packet from the descriptor, so the packet it emits always has length bytes.

### 23.3.3	How it Clears

The per-channel bit is cleared by the channel reset (and therefore by the global reset, which asserts every channel’s reset). It stays clear afterwards; the next packet is checked against its own record. See “Recovery” below for what the reset does to a packet that is part-way through the stream.

### 23.3.4	Prevention

- Issue the sink descriptor for a packet before, or together with, the first beat of that packet. The stream waits for the descriptor (see “Not Errors” below), so a late descriptor costs time and nothing else.

- Set length to the exact byte count of the packet. If the length is only known at the end of the packet, write the descriptor after the length is known.

- Do not split one packet across two descriptors and do not put two packets under one descriptor. One descriptor maps to one tlast delimited packet.

## 23.4	Shared Error Classes

### 23.4.1	Invalid or Unfetchable Descriptor

The scheduler leaves CH_FETCH_DESC only for a descriptor whose valid bit is set. A descriptor with valid = 0 sends the channel to its error state.

The descriptor engine rejects a descriptor fetch whose address falls outside the two configured address ranges. The rejection happens before any AXI read is issued. It also reports an error for a fetch address of zero or otherwise invalid on the APB path, and for an AXI error response on the fetch itself.

A descriptor whose length is zero is not invalid. See “Not Errors”.

### 23.4.2	Data-Path AXI Errors

The AXI read engine sets a sticky per-channel flag when a data read returns a response other than OKAY. The AXI write engine does the same for a write response. Both flags drive the channel’s error state.

### 23.4.3	Control Engine Errors

A control read that does not match after the configured number of retries reports an error. Retry exhaustion is what stops a control read that can never match from hanging the channel. A control write that returns an AXI error reports an error. Both are fatal to the descriptor that issued them.

### 23.4.4	Write-Progress Timeout

The scheduler counts the cycles in which it holds a write request that the write engine has not accepted. A write completion or a write commit resets the count. Its configuration is shared with RAPIDS Beats:

Timeout Configuration

| Register field | Meaning | Default |
| --- | --- | --- |
| SCHED_TIMEOUT_CYCLES.TIMEOUT_CYCLES | Length of one window in cycles | 0x3E8 |
| SCHED_TIMEOUT_LIMIT.LIMIT | Consecutive expired windows that escalate to a fatal error. Zero never escalates. | 4 |

A window that expires is recoverable. Any write progress clears the strike count. Only the configured number of consecutive expired windows makes the timeout fatal. A LIMIT of zero keeps the timeout purely advisory. The timeout is unchanged by the byte-granular design.

## 23.5	Not Errors

Three conditions look like faults and are normal operation.

### 23.5.1	Packet Record Queue Full

Each channel keeps a queue of packet records, one per non-zero data descriptor per enabled direction. The queue holds four records. When it is full, the scheduler waits in CH_FETCH_DESC until a record is consumed. Software sees the channel busy and nothing else.

No error bit is set and no MonBus error is sent. The timeout counter runs only while the scheduler holds a write request the write engine has not accepted, so waiting for a record does not count toward it. Descriptor chains longer than the queue therefore run at the pace the data paths consume records.

### 23.5.2	Sink Beat With No Packet Record

The sink ingress accepts a stream beat for a channel only when that channel’s packet record exists. Until then s_axis_tready stays low.

s_axis_tready is a single signal. It is qualified by the tid of the beat that is on the bus. A beat for a channel with no record therefore stalls the whole stream, including beats for other channels that are queued behind it in the sender. The queues and holds inside RAPIDS are per channel, so no data is lost or mixed. The cost is delay, and it lands on every channel that shares the stream.

Two rules keep this delay short:

- Post the sink descriptor for a channel before the sender drives the first beat for that channel.

- A sender that interleaves channels on one stream must not assume that a waiting channel leaves the others unaffected at the AXI-Stream interface.

### 23.5.3	The 4 KB Burst Cap

An AXI burst must not cross a 4 KB boundary. The scheduler splits a transfer at each boundary. A descriptor that spans several pages issues several bursts, and every burst starts beat-aligned. The split is invisible to software, and it neither sets a flag nor counts against any timeout. The cost appears only as throughput, described in the throughput chapter.

## 23.6	Error Reporting

### 23.6.1	Registers

Software finds the failing channel from these fields. All names come from the shared register map and are the same as in RAPIDS Beats.

Error Registers

| Register field | Use |
| --- | --- |
| SRC.SCHED_ERROR.SCHED_ERR | One bit per channel, source half |
| SNK.SCHED_ERROR.SCHED_ERR | One bit per channel, sink half |
| SCHEDULER_IDLE.SCHED_IDLE | One bit per channel, set when the scheduler is idle |
| CHANNEL_RESET.CH_RST | One bit per channel, resets the scheduler and its engines |
| GLOBAL_CTRL.GLOBAL_RST | Resets every channel |
| GLOBAL_CTRL.GLOBAL_EN | Global enable |

The per-channel error bit is the summary of every fatal class in the “Error Classes” table. It does not name the class. To name the class, read the MonBus error packet or use the scheduler debug outputs.

### 23.6.2	MonBus

The scheduler sends one error packet the first time a channel enters its error state. It does not repeat the packet on every cycle. The packet carries the sticky write-error flag and the sticky read-error flag in its data field. MonBus decoding is described in the shared monitor bus chapter.

## 23.7	Recovery

### 23.7.1	Channel Reset

Writing the channel’s bit in CHANNEL_RESET.CH_RST of the half that reported the error, SNK or SRC (or GLOBAL_CTRL.GLOBAL_RST for every channel of that half), returns the channel to idle. The reset reaches the scheduler, the descriptor and control engines, the AXI read and write engines, and the sink and source data paths. Every sticky error flag in that chain clears, including the data read and write response flags and the sink packet length flag. Software should:

- Set the channel’s CH_RST bit for at least one clock, then clear it. A single-cycle pulse and a held level both work. To reset both directions of a channel, write both halves.

- Poll the SCHED_ERR bit and the SCHED_IDLE bit for the channel. The error bit stays clear and the idle bit sets.

- Rebuild the descriptor chain from a known-good state. Bytes that the failed chain already wrote stay in memory.

- Start the channel again as in the initialization sequence.

This recovers every error class in the “Error Classes” table. No hardware reset is needed, and the other channels keep running through it.

### 23.7.2	What the Reset Does to Work in Flight

A channel reset can arrive with beats, bursts and packets still moving. The hardware settles them as follows, so the channel is clean when the reset releases.

Channel Reset: Handling of In-Flight Work

| Where | In-flight work | Effect |
| --- | --- | --- |
| Sink stream, packet already started | Beats of the packet still to come | The ingress accepts and discards them up to and including tlast, so the tail of the old packet cannot be taken as the start of the next. The stream does not stall. |
| Sink stream, no packet started | None | Nothing to discard. A beat for the channel is held off while the reset is asserted. |
| Sink write path | An AXI write burst already open | The write engine finishes the burst with null beats (WSTRB = 0), so the AXI protocol stays legal and the null beats write nothing. A beat already presented on the write data channel when the reset hit is completed unchanged. No new burst opens for the channel until the open bursts and their responses have finished. Its AWADDR value is unchanged. |
| Sink write path | Write responses still due | They are consumed and ignored. A BRESP error that arrives after the reset does not set the error flag. |
| Source read path | Read bursts already issued | The read engine issues no further bursts, then drains and discards the returning read data until none is outstanding. A RRESP error in that data does not set the error flag. |
| Source stream, packet in progress | Beats not yet sent | The egress stops. A beat already on m_axis completes. The remaining beats are not sent (see below). |
| Other channels | Everything | Not affected. The data paths mask only the channel in reset. |

The sink ingress discards only the tail of a packet of which at least one beat had been accepted when the reset hit. The sender must still send that packet’s tlast; the beat is taken and dropped, and the next beat for the channel is checked against the next packet record as usual. While the reset is asserted the channel’s beats are held off (s_axis_tready low).

The source stream is left unterminated. If the reset lands between the first and last beat of a source packet, RAPIDS stops sending it and does not emit a tlast (and no null terminator). The receiver sees a partial packet. A receiver that must resynchronize after a channel reset should treat the reset as the end of the packet, or flush its own per-channel state when it resets the channel. A packet that had not started, or that had already sent its tlast, is unaffected.

Head-of-line blocking is unchanged. A sink beat for a channel with no packet record still stalls the shared s_axis_tready, as described under “Not Errors”. A channel reset does not remove this. After the reset, post the next sink descriptor before the sender starts the next packet.

## 23.8	Software Checklist

Software Checklist

| Step | Action |
| --- | --- |
| Before the run | Program SCHED_TIMEOUT_CYCLES and SCHED_TIMEOUT_LIMIT. Post the sink descriptor before the first beat of its packet. |
| During the run | Poll SCHED_ERR for the active channels, or watch MonBus for error packets. |
| On an error | Note the channel and read the MonBus packet. Decide which class from the descriptor state and the stream. |
| Recovery | Channel reset for every error class, including a length mismatch or an AXI response error. |
| After recovery | Re-post descriptors. Re-send the whole packet, not the tail. A source receiver discards the partial packet of a reset channel. |

## 23.9	Related Chapters

- Descriptor Format: the byte length, the packet record and the zero-length rule.

- Register Map: the shared register layout.

- Monitor Bus Interface: the error packet format.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Throughput

RAPIDS moves bytes. The datapath moves beats. This chapter separates the two figures because they differ for small and unaligned transfers, and it labels every measured number with the build that produced it.

The sink path moves an AXI-Stream ingress stream into memory. The source path reads memory and drives an AXI-Stream egress stream. The two paths are independent engines with their own buffers, schedulers and AXI masters, so their bandwidths add.

## 24.1	Theoretical Maximum

A beat is DATA_WIDTH / 8 bytes. The board build uses DATA_WIDTH = 256, so a beat is 32 bytes. Per-direction bandwidth is the beat size times the clock.

Theoretical Maximum (256-bit Build)

| Item | Value |
| --- | --- |
| Beat size | 32 bytes |
| Clock on the board | 100 MHz |
| Line rate per direction | 3.2 GB/s |
| Sink plus source together | 6.4 GB/s |

The line rate scales linearly with the clock and with the data width. The width does not change the per-beat behavior of the engines, so every utilization figure below holds at any width and only the GB/s scale changes.

## 24.2	Byte Efficiency

The byte-granular design accepts any start address and any length. The memory side still moves whole beats. A transfer of length bytes that starts at byte offset inside a beat occupies

beats = (offset + length + BYTE_LANES - 1) >> OFF_W

beats on that side, with BYTE_LANES = 32 and OFF_W = 5 for 256 bits. The useful fraction of those beats is

efficiency = useful_bytes / (beats * BYTE_LANES)

Here useful_bytes is length. Efficiency is 1.0 only when both the start and the end land on beat boundaries. The bus meters count a beat with partial byte enables as a productive beat, so their utilization figures measure the beat stream. The byte throughput is the beat throughput multiplied by this efficiency.

Beats and Efficiency for Small and Unaligned Transfers

| Length | Offset | Beats | Efficiency |
| --- | --- | --- | --- |
| 1 | 0 | 1 | 0.031 |
| 2 | 31 | 2 | 0.031 |
| 32 | 0 | 1 | 1.000 |
| 32 | 1 | 2 | 0.500 |
| 37 | 0 | 2 | 0.578 |
| 60 | 5 | 3 | 0.625 |
| 100 | 7 | 4 | 0.781 |
| 203 | 3 | 7 | 0.906 |

The loss is at most two partial beats per descriptor: one at the head and one at the tail. It shrinks as the length grows. A 4 KB aligned transfer is 128 full beats and loses nothing. Software that needs the full line rate should align buffers to the beat size and use lengths that are multiples of it. Software that cannot align pays at most two beats per descriptor.

The offset applies to each half separately. The source half uses the source address and the sink half uses the destination address. A copy whose source and destination have different offsets can have a different beat count on each half.

### 24.2.1	4 KB Boundaries

AXI bursts do not cross a 4 KB boundary, so a transfer is split at every boundary it crosses. A page holds 128 beats at 32 bytes. The split adds one burst per extra page, and each burst starts beat-aligned. It costs burst-start overhead and no data. For large transfers the overhead amortizes in the same way as any other burst boundary. The burst-length sweep below gives the measured cost of short bursts.

### 24.2.2	Head-of-Line Delay on the Sink Stream

The sink stream has one s_axis_tready. It is qualified by the tid of the beat on the bus. A beat for a channel with no packet record yet stalls the whole stream, including beats for other channels behind it. The queues inside RAPIDS are per channel, so the cost is delay only. A sender that posts each sink descriptor before its first beat never sees this stall. The error chapter describes the rule.

## 24.3	Measured Results

Three sets of results exist. They answer different questions and must not be mixed.

Sources of Measured Figures

| Set | Build | Question it answers |
| --- | --- | --- |
| Byte campaign | Byte-granular RAPIDS on the Genesys 2 | Are byte lengths and offsets correct on the board? |
| Beats performance report | RAPIDS Beats on the Genesys 2 | How fast does the beat datapath run? |
| Byte-build performance | None yet | How fast does the byte build run in beat mode? |

The byte checkers on the board build hold ready while they feed a transfer. A performance row measured with them therefore does not show the beat-mode backpressure behavior that the beats performance report was designed to show. The beat-mode rows in the beats performance report come from the beats build and are quoted as such below. A performance characterization of the byte build needs a build whose checkers run in beat mode, and that build does not exist yet.

### 24.3.1	Byte Campaign (Correctness)

Source: projects/fpga-systems/Genesys2/dma-ip/rapids/reports/board, files rapids_byte_bytes_20260930_072516.json and rapids_byte_smoke_20260930_072435.json. Two channels were active, one descriptor per channel.

Byte Campaign on the Board (7 of 7 Pass)

| Packet bytes | Offset | Sink beats per channel | Result |
| --- | --- | --- | --- |
| 1 | 1 | 1 | Pass |
| 2 | 31 | 2 | Pass |
| 32 | 0 | 1 | Pass |
| 37 | 0 | 2 | Pass |
| 100 | 7 | 4 | Pass |
| 96 | 17 | 4 | Pass |
| 203 | 3 (address 4035, crosses a 4 KB boundary) | 7 | Pass |

The smoke run, four beats on two channels, also passes. The beat count the sink write master issued equals the formula in every row. Golden data matched in every row on both halves. These rows prove correctness. They are runs of one to seven beats, so their utilization figures are dominated by launch and are not throughput.

### 24.3.2	RAPIDS Beats Performance Report

Source: projects/fpga-systems/Genesys2/dma-ip/rapids_beats/reports/perf/README.md, version 2.2. Build: RAPIDS Beats, 8 channels, 256 bits, 4 KB of buffer per channel, 100 MHz. Every row is golden-CRC checked. Utilization is engaged utilization: productive cycles over productive, backpressure and starvation cycles.

RAPIDS Beats Build, 8 Channels at 128 KB per Channel

| Path | Interface | Engaged utilization | Effective bandwidth |
| --- | --- | --- | --- |
| Sink | AXIS in | 99.7 % | 3.19 GB/s |
| Sink | AXI4 write | 100.0 % | 3.20 GB/s |
| Source | AXI4 read | 100.0 % | 3.20 GB/s |
| Source | AXIS out | 100.0 % | 3.20 GB/s |

The 0.3 point shortfall on the sink stream is a one-time 82-cycle fill stall of the 4 KB per-channel buffer. It does not repeat inside a run.

RAPIDS Beats Build, Transfer Size Sweep at 8 Channels

| Beats per channel | AXIS in | AXI4 write | AXI4 read | AXIS out |
| --- | --- | --- | --- | --- |
| 1 | 88.9 % | 38.1 % | 27.6 % | 27.6 % |
| 4 | 97.0 % | 84.2 % | 69.6 % | 69.6 % |
| 16 | 99.2 % | 95.5 % | 90.1 % | 90.1 % |
| 64 | 99.8 % | 98.5 % | 97.3 % | 97.3 % |
| 256 | 96.1 % | 99.7 % | 99.3 % | 99.3 % |
| 1024 | 99.0 % | 99.9 % | 99.8 % | 99.8 % |
| 4096 | 99.7 % | 100.0 % | 100.0 % | 100.0 % |

RAPIDS Beats Build, Channel Scaling at 4096 Beats per Channel

| Active channels | AXIS in | AXI4 write | AXI4 read | AXIS out |
| --- | --- | --- | --- | --- |
| 1 | 100.0 % | 99.9 % | 99.7 % | 99.7 % |
| 2 | 100.0 % | 99.9 % | 99.8 % | 99.8 % |
| 4 | 100.0 % | 100.0 % | 99.9 % | 99.9 % |
| 8 | 99.7 % | 100.0 % | 100.0 % | 100.0 % |

RAPIDS Beats Build, Burst Length Sweep at 8 Channels and 1024 Beats

| Beats per burst | AXIS in | AXI4 write | AXI4 read | AXIS out |
| --- | --- | --- | --- | --- |
| 1 | 35.9 % | 35.6 % | 99.8 % | 99.8 % |
| 2 | 71.0 % | 71.0 % | 99.8 % | 99.8 % |
| 4 | 99.0 % | 99.9 % | 99.8 % | 99.8 % |
| 8 or more | 99.0 % | 99.9 % | 99.8 % | 99.8 % |

The beats report also holds the memory-latency sweeps and the buffer-depth comparison. Those results belong to the beat datapath and are not repeated here.

### 24.3.3	Using the Beats Numbers

The byte build was not measured in beat mode, so the rows above are reference points and not measurements of it. The memory-side figures count beats. For a byte transfer, multiply the beat figure by the efficiency in the byte efficiency table to estimate the byte rate. Treat the result as an estimate until the byte build has its own performance run.

## 24.4	Throughput Limiting Factors

Throughput Limiting Factors

| Factor | Effect | Mitigation |
| --- | --- | --- |
| Partial head and tail beats | At most two beats per descriptor carry fewer than 32 useful bytes | Align buffers to the beat size and use beat-multiple lengths |
| 4 KB boundaries | One extra burst per page crossed | Place large buffers on page boundaries |
| Short bursts | Below 4 beats the write side loses efficiency in the beats measurement | Keep the burst configuration at 4 beats or more |
| Buffer fill at run start | A one-time 82-cycle stall on the sink stream for the 4 KB build | A fixed cost per run, so its share shrinks as transfers grow |
| Head-of-line stall on the sink stream | Delay on all channels when one channel lacks a packet record | Post each sink descriptor before its first beat |
| Record queue full | The scheduler waits until a record is consumed; delay only | Chains longer than four descriptors run at the pace the data paths consume records |
| Memory latency | Reduces small-transfer efficiency | Use the beats report’s latency sweeps to size outstanding depth |
| TYPE=EXT descriptors | Beat rows only, so no head or tail loss but no unaligned addressing | Keep EXT buffers aligned, which they must be anyway |

Control descriptors move no payload. A control read gates on a memory location and a control write issues a single write. Budget them as latency, not bandwidth.

## 24.5	Related Chapters

- Latency Characteristics: shared with RAPIDS Beats.

- Descriptor Format: the byte length, the offset and the beat-count formula.

- Error Handling: the length mismatch and the head-of-line rule.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Latency Characteristics

Latency is dominated by the one-time pipeline fill from a channel kick to the first data beat. Once running, both paths sustain the throughput of Section 6.1; control descriptors add a bounded synchronization delay.

## 25.1	Latency Components

### 25.1.1	Kick-to-First-Beat Breakdown

| Stage | Typical Cycles (100 MHz) | Notes |
| --- | --- | --- |
| Kick decode (APB write to KICK_ENABLE) | TBD – re-measure | Address is staged beforehand; the launch request is then held until the descriptor engine accepts it |
| Descriptor fetch (AXI read, 256-bit) | AXI read latency + 1 | Overlapped by prefetch on chains |
| Descriptor parse | 1-2 | Opcode + fields decoded |
| SRAM fill (sink) / AXI read issue (source) | AXI/AXIS latency | First beat into the buffer |
| First beat out | 1-2 | Drain to memory (sink) / AXIS (source) |

### 25.1.2	Total Latency Formula

L_first_beat  =  L_kick + L_desc_fetch + L_parse + L_pipeline_fill

Note: L_kick predates the staged-address/KICK_ENABLE kick and has not been re-measured. The previous figure came from a path that stalled APB until the descriptor engine accepted the kick; acceptance is now asynchronous, so the term is no longer bounded by an APB stall.

L_desc_fetch is hidden on chained descriptors when the descriptor engine has prefetched the next descriptor (DESCENG_CONFIG.PREFETCH_EN).

## 25.2	First-Beat Latency

The dominant term is the memory/network round trip to fill the pipeline. For a sink transfer the first AXIS beat must arrive and be buffered before the first AXI write can drain; for a source transfer the first AXI read must return before the first AXIS egress beat. After the first beat the pipeline is full and throughput is bandwidth-bound, not latency-bound.

## 25.3	Control-Descriptor Latency

Control descriptors are latency, not throughput (Section 6.1):

| Opcode | Latency |
| --- | --- |
| CTRL_WRITE (doorbell) | One single-beat AXI write: AW->W->B round trip |
| CTRL_READ (gate) | One poll per retry (AXI read + compare); retries paced by tick_1us, up to CTRL_CONFIG.CTRLRD_MAX_TRY (default 16, max 511) |

A CTRL_READ gate blocks its descriptor (and the chain) until the polled value matches. The retry budget bounds the worst case: an unsatisfied gate cannot hang the channel – after CTRLRD_MAX_TRY polls the engine raises ctrlrd_error instead of waiting forever.

## 25.4	Latency Variability

### 25.4.1	Sources of Variability

- Memory read latency (descriptor fetch, source reads) and write-response timing.

- AXIS backpressure (s_axis_tready / m_axis_tready) on the network end.

- Multi-channel arbitration for the shared AXI master.

- CTRL_READ retry count when a gate is not yet satisfied.

### 25.4.2	Worst-Case Latency

Bounded by: memory timeout (SCHED_TIMEOUT_CYCLES / SCHED_TIMEOUT_LIMIT escalation on the scheduler side) and the control-read retry budget (CTRL_CONFIG.CTRLRD_MAX_TRY). Both convert an otherwise-unbounded wait into a reported error so a channel is never permanently stalled.

## 25.5	Latency Optimization

### 25.5.1	Software Recommendations

- Chain descriptors so the descriptor engine prefetches the next while the current transfers (hides L_desc_fetch).

- Keep the AXIS consumer/producer ready to avoid backpressure stalls.

- Size CTRL_READ gates so the expected value is already (or soon) present to minimize poll retries.

### 25.5.2	Hardware Configuration

- Adequate SRAM_DEPTH so the buffer covers memory/network latency without stalling.

- Tune AXI_XFER_CONFIG (RD/WR beats, ALLOC/DRAIN) so bursts amortize AXI setup.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Resource Estimates

The figures in this chapter are measured. They come from the placed and routed board build of the byte-granular RAPIDS on the Genesys 2, not from an estimate. The chapter states the configuration first because every number depends on it.

## 26.1	Build Configuration

Board Build Configuration

| Item | Value |
| --- | --- |
| Top level | rapids_byte_genesys2_top (RAPIDS plus the characterization harness) |
| Device | xc7k325tffg900-2 (Kintex-7 325T) |
| Tool | Vivado 2025.1, design state Physopt postRoute |
| Clock | 100 MHz |
| Channels | 8, on both the sink and the source half |
| Data width | 256 bits (32-byte beats) |
| Buffer depth | 128 entries per channel, 4 KB per channel |
| Interface observers | On |
| AXI monitors | Off |

The measured design includes the characterization harness: the UART bridge, the pattern generators, the byte checkers and the bus meters. The figures are therefore an upper bound for RAPIDS alone. The harness is not separated in the reports.

## 26.2	FPGA Resource Summary

Source: projects/fpga-systems/Genesys2/dma-ip/rapids/reports/perf/json/rapids_byte_build_bytecrc.json and rapids_byte_build_wordcrc.json, extracted from the post-route reports by reports/extract_build_metrics.py.

The harness has two checker builds. BYTE_CRC=1 is the standard bitstream: the byte-wise CRC checkers take 9 cycles per 32-byte beat and verify every byte and strobe. BYTE_CRC=0 is a measurement bitstream with word-wide checkers that take one beat per cycle, so the harness does not cap the measured rate.

Post-Route Utilization, 8 Channels, 256 Bits

| Resource | Standard (BYTE_CRC=1) | Word-wide (BYTE_CRC=0) | Available |
| --- | --- | --- | --- |
| Slice LUTs | 92,874 (45.57 %) | 86,065 (42.23 %) | 203,800 |
| LUT as logic | 81,352 | 75,695 | 203,800 |
| LUT as distributed RAM | 11,522 | 10,370 | 64,000 |
| Slice registers | 82,379 (20.21 %) | 79,527 (19.51 %) | 407,600 |
| F7 muxes | 4,709 | 4,430 | 101,900 |
| F8 muxes | 106 | 107 | 50,950 |
| Block RAM tiles | 68 (15.28 %) | 52 (11.69 %) | 445 |
| DSP | 0 | 0 | 0 |

All block RAM tiles are RAMB36E1. No RAMB18 is used, and no register is a latch. DSP use is zero because RAPIDS moves data and performs no multiply-accumulate. The 16 tiles and about 6,800 LUTs that the word-wide build saves are the byte checkers’ golden storage and CRC logic, which belong to the harness and not to RAPIDS.

## 26.3	Timing

Source: the same two build JSONs.

Timing at 100 MHz

| Metric | Standard (BYTE_CRC=1) | Word-wide (BYTE_CRC=0) |
| --- | --- | --- |
| Worst negative slack, setup | +0.301 ns | +0.077 ns |
| Total negative slack | 0.000 ns | 0.000 ns |
| Worst hold slack | +0.023 ns | +0.013 ns |
| Failing endpoints | 0 of 320,415 | 0 of 301,738 |

All user timing constraints are met in both builds. The slack is small and positive; treat 100 MHz as the design point and not as a margin.

## 26.4	Board Result

On the standard bitstream (sha256 cfad34c3...) the byte-wise campaign passes 117 of 117 points and the directed sequences 4 of 4. On the word-wide bitstream (sha256 48e1282c...) the beat-aligned profile passes 28 of 28 points and reaches 3182 MB/s on the sink and 3199 MB/s on the source at 8 channels and 4096 beats, against a theoretical peak of 3200 MB/s (100 MHz x 32 bytes). The word-wide checker CRCs only slice 0 of each beat and ignores strobes, so it is compared with the beats golden; byte-level integrity comes from the byte-wise campaign. Both runs are in the throughput chapter and the characterization report (reports/perf/README.md, v0.2).

## 26.5	Comparison with the Beats Build

The RAPIDS Beats performance report lists the resources of its 256-bit build in its version 2.0 text. Both builds run 8 channels at 100 MHz on the same device with a buffer of 128 entries per channel.

Beats and Byte-Granular Builds

| Item | RAPIDS Beats, 256-bit, v2.0 | RAPIDS, byte-granular (standard build) |
| --- | --- | --- |
| Slice LUTs | 75,679 | 92,874 |
| Block RAM tiles | 44 | 68 |
| Worst negative slack | +0.608 ns | +0.301 ns |

The columns are not a like-for-like comparison of the two designs. The two bitstreams come from different versions of the RTL and the harness. The byte checkers and the byte-wise golden logic are in the byte build and not in the beats build. The beats build has since been rebuilt, and the v2.2 beats report header gives a slack of +0.286 ns for it and no new resource counts. Use the table as a size comparison of two board images, and not as the cost of byte granularity.

## 26.6	What Depends on What

The reports do not break the design down by block, and this chapter does not invent a breakdown. The relationships that follow from the design are these.

Scaling Notes

| Change | Effect |
| --- | --- |
| Channels 8 to fewer | Scheduler, descriptor and buffer resources scale with channel count |
| Buffer depth | Block RAM scales with entries per channel, per direction, and with the entry width |
| Data width | Datapath logic and buffer width scale with it. The line rate scales with it. |
| Interface observers off | Removes the observer logic and its histograms |
| AXI monitors on | Adds monitor logic. They are off in this build. |

The sink buffer entry is DATA_WIDTH + DATA_WIDTH / 8 bits wide, because each entry stores the byte enables beside the data. The source buffer entry stays DATA_WIDTH bits. The byte-granular sink therefore stores a wider entry than the beats sink for the same depth and width.

The byte-granular design adds per-channel state that the beats design lacks: a packet-record queue four deep, a sink spill register, an ingress packet byte counter and, on the source half, the re-pack hold register. The reports do not isolate their cost.

## 26.7	Related Chapters

- Throughput: the measured campaign and the beats report.

- Error Handling: the reset behavior of the sticky flags.