RTL Design Sherpa

RAPIDS Micro-Architecture Specification 0.1

September 30, 2026

Table of Contents

1 Rapids Mas Index	32

2 Document Information	33

2.1 Revision History	33

2.2 References	33

2.2.1 Related Documents	33

2.3 Terminology	34

2.4 Document Conventions	35

3 Architecture Overview	36

3.1 Overview	36

3.1.1 Key Features	36

3.1.2 What Is Shared with RAPIDS Beats	36

3.2 Block Diagram	37

3.2.1 Figure 1.1.1: RAPIDS Architecture Block Diagram	37

3.3 Data Flow Overview	38

3.3.1 Sink Path (Network to Memory)	38

3.3.2 Figure 1.1.2: Sink Path Data Flow	38

3.3.3 Source Path (Memory to Network)	38

3.3.4 Figure 1.1.3: Source Path Data Flow	38

3.4 Byte-Granular Design Points	39

3.5 Chapter Map	40

4 Top-Level Port List	41

4.1 Parameters	41

4.2 Port List (324 ports)	43

4.2.1 Clock and Reset (shared - single clock/reset for the whole wrapper)	43

4.3 SOURCE HALF (u_src) - shared-infrastructure ports (src_ prefixed)	43

4.3.1 APB Programming Interface	43

4.3.2 Per-channel configuration	43

4.3.3 Scheduler Configuration (global)	44

4.3.4 Descriptor Engine Configuration (global)	44

4.3.5 Control Engine Configuration (Phase 2, global)	45

4.3.6 Descriptor AXI Monitor Configuration	45

4.3.7 AXIS data-path monitor-lite configuration (rapids TASK-015)	46

4.3.8 Status	47

4.3.9 Descriptor AXI Monitor Status	47

4.3.10 AXIS data-path monitor-lite status (rapids TASK-015)	47

4.3.11 Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).	48

4.3.12 Descriptor Fetch AXI Master	49

4.3.13 Control Read AXI Master (32-bit) [Phase 2]	50

4.3.14 Control Write AXI Master (32-bit) [Phase 2]	51

4.4 SOURCE HALF (u_src) - direction-unique ports (no prefix)	52

4.4.1 AXI Transfer Configuration (source-only)	52

4.4.2 Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id	52

4.4.3 AXI4 Master - Data Read (Memory -> Source SRAM)	53

4.5 SINK HALF (u_snk) - shared-infrastructure ports (snk_ prefixed)	53

4.5.1 APB Programming Interface	53

4.5.2 Per-channel configuration	54

4.5.3 Scheduler Configuration (global)	54

4.5.4 Descriptor Engine Configuration (global)	54

4.5.5 Control Engine Configuration (Phase 2, global)	55

4.5.6 Descriptor AXI Monitor Configuration	55

4.5.7 AXIS data-path monitor-lite configuration (rapids TASK-015)	56

4.5.8 Status	57

4.5.9 Descriptor AXI Monitor Status	57

4.5.10 AXIS data-path monitor-lite status (rapids TASK-015)	58

4.5.11 Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).	58

4.5.12 Descriptor Fetch AXI Master	59

4.5.13 Control Read AXI Master (32-bit) [Phase 2]	60

4.5.14 Control Write AXI Master (32-bit) [Phase 2]	61

4.6 Monitor Bus (SINGLE aggregated stream for the whole core)	62

4.6.1 Monitor Bus	62

4.7 SINK HALF (u_snk) - direction-unique ports (no prefix)	62

4.7.1 AXI Transfer Configuration (sink-only)	62

4.7.2 Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id	63

4.7.3 AXI4 Master - Data Write (Sink SRAM -> Memory)	63

4.8 Debug Interface (per half, src_dbg_ / snk_dbg_ prefixed)	64

4.8.1 Source half debug	64

4.8.2 Sink half debug	65

4.8.3 Active-channel sideband for per-channel bus instrumentation	65

5 Clocks and Reset	67

5.1 Clock Domain	67

5.1.1 Clock Specifications	67

5.1.2 Clock Distribution	67

5.2 Reset Specification	67

5.2.1 Reset Signal	68

5.2.2 Reset Timing Diagram	68

5.2.3 Figure 1.3.1: Reset Timing	68

5.3 Reset Behavior	69

5.3.1 On Reset Assert	69

5.3.2 Reset Sequence	69

5.4 Configuration During Reset	69

5.5 Reset Macros	70

5.6 Power-On Reset Considerations	70

6 Scheduler	72

6.1 Overview	72

6.1.1 Key Features	72

6.1.2 Block Diagram	73

6.1.3 Figure 2.1.1: Scheduler Byte Math	73

6.2 Parameters	73

6.3 Port List	74

6.3.1 Clock, Reset and Configuration	74

6.3.2 Status	75

6.3.3 Descriptor Engine Interface	75

6.3.4 Data Read Interface (to AXI Read Engine)	76

6.3.5 Data Write Interface (to AXI Write Engine)	76

6.3.6 Completion Interface	77

6.3.7 Control Engine Interface	77

6.3.8 Monitor Bus Interface	78

6.4 Descriptor Format	78

6.5 Byte-to-Beat Math	79

6.6 Address Advance	80

6.7 Packet Records	81

6.8 Finite State Machine	81

6.9 Operation	82

6.9.1 Data Request Outputs	82

6.9.2 Write Completion	83

6.9.3 Control Descriptors	83

6.10 Error Handling	83

6.11 Recoverable Write-Progress Timeout	83

6.12 MonBus Events	84

6.13 Waveforms	84

7 Descriptor Engine Specification	86

7.1 Overview	86

7.1.1 Key Features	86

7.1.2 Block Diagram	86

7.1.3 Figure 2.2.1: Descriptor Engine Block Diagram	86

7.2 Parameters	87

7.3 Port List	87

7.3.1 Clock and Reset	87

7.3.2 APB Programming Interface	87

7.3.3 Configuration Interface	88

7.3.4 AXI4 Read Master Interface	88

7.3.5 Scheduler Interface	89

7.4 FSM States	90

7.4.1 Figure 2.2.2: Descriptor Engine FSM	90

7.5 Operation	91

7.5.1 Descriptor Chain Processing	91

7.5.2 Figure 2.2.3: Descriptor Chain Timing	91

7.6 Address Range Checking	91

7.7 Prefetch and Autonomous Chaining	92

7.7.1 Chaining Decision	92

7.7.2 Deferred (Throttle-Blocked) Chain Fetch	92

8 AXI Read Engine	94

8.1 Overview	94

8.1.1 Key Features	94

8.1.2 Block Diagram	95

8.1.3 Figure 2.3.1: AXI Read Engine Block Diagram	95

8.2 Parameters	95

8.2.1 Derived Constants	96

8.3 Port List	97

8.3.1 Clock, Reset and Configuration	97

8.3.2 Scheduler Interface	97

8.3.3 SRAM Allocation and Fill Interface	98

8.3.4 AXI4 Read Master Interface	98

8.3.5 Debug	99

8.4 Operation	99

8.4.1 Burst Length Cap (rapids BUG-009)	99

8.4.2 Byte-Granular Burst Sizing	100

8.4.3 Streaming Pipeline	100

8.4.4 Figure 2.3.2: Burst Segmentation with a Mid-Beat Start	100

8.4.5 Done Strobe	101

8.5 Error Handling	101

8.5.1 Channel Reset	101

8.6 Differences from RAPIDS Beats	102

9 AXI Write Engine	103

9.1 Overview	103

9.1.1 Key Features	103

9.1.2 Block Diagram	104

9.1.3 Figure 2.4.1: AXI Write Engine Block Diagram	104

9.2 Parameters	104

9.2.1 Derived Constants	105

9.3 Port List	106

9.3.1 Clock, Reset and Configuration	106

9.3.2 Scheduler Interface	106

9.3.3 SRAM Reservation and Drain Interface	107

9.3.4 AXI4 Write Master Interface	108

9.3.5 Debug and Status	109

9.4 Operation	109

9.4.1 Burst Length Cap (rapids BUG-009)	109

9.4.2 Byte-Granular Burst Sizing	109

9.4.3 Write Strobes	110

9.4.4 Streaming Pipeline	110

9.4.5 Two Completion Strobes	110

9.5 Error Handling	111

9.5.1 Channel Reset	111

9.6 Differences from RAPIDS Beats	112

10 Beats Alloc Control Specification	113

10.1 Overview	113

10.1.1 Key Features	113

10.1.2 Block Diagram	114

10.1.3 Figure 2.5.1: Beats Alloc Control Block Diagram	114

10.2 Concept: Virtual FIFO	114

10.2.1 Use Case: Pre-Allocation for AXI Reads	114

10.3 Parameters	115

10.4 Port List	115

10.4.1 Clock and Reset	115

10.4.2 Write Interface (Allocation Requests)	115

10.4.3 Read Interface (Data Written)	116

10.4.4 Status Outputs	116

10.5 Operation	116

10.5.1 Pointer Arithmetic	116

10.5.2 Timing Diagram	117

10.5.3 Figure 2.5.2: Allocation and Release Timing	117

10.6 Integration Example	117

11 Beats Drain Control Specification	119

11.1 Overview	119

11.1.1 Key Features	119

11.1.2 Block Diagram	120

11.1.3 Figure 2.6.1: Beats Drain Control Block Diagram	120

11.2 Concept: Data Availability Tracking	120

11.2.1 Complementary Roles	120

11.3 Parameters	121

11.4 Port List	121

11.4.1 Clock and Reset	121

11.4.2 Write Interface (Data Written to SRAM)	121

11.4.3 Read Interface (Drain Requests)	121

11.4.4 Status Outputs	122

11.5 Operation	122

11.5.1 Pointer Arithmetic	122

11.5.2 Timing Diagram	123

11.5.3 Figure 2.6.2: Data Arrival and Drain Timing	123

11.6 Integration Example	123

11.7 Relationship: alloc_ctrl vs drain_ctrl	124

12 Latency Bridge (beats) Specification	125

12.1 Overview	125

12.1.1 Key Features	125

12.1.2 Block Diagram	126

12.1.3 Figure 2.7.1: Latency Bridge Block Diagram	126

12.2 Concept: bridging a registered read	126

12.3 Parameters	127

12.4 Port List	127

12.4.1 Clock and Reset	127

12.4.2 Upstream Interface (from the registered FIFO)	127

12.4.3 Downstream Interface (to the consumer)	127

12.4.4 Status and Debug	128

12.5 Operation	129

12.5.1 Figure 2.7.2: Latency Bridge Timing (consumer always ready)	129

12.6 Integration Context	130

12.7 Design Considerations	130

13 Control-Read Engine Specification	132

13.1 Overview	132

13.1.1 Key Features	132

13.1.2 Block Diagram	133

13.1.3 Figure 2.8.1: Control-Read Engine Block Diagram	133

13.2 Parameters	133

13.3 Port List	134

13.3.1 Clock and Reset	134

13.3.2 Scheduler Interface	134

13.3.3 Configuration Interface	135

13.3.4 AXI4 Read Master Interface	135

13.3.5 MonBus Interface	136

13.4 FSM States	137

13.4.1 Figure 2.8.2: Control-Read Engine FSM	137

13.5 Operation	138

13.5.1 Masked Compare	138

13.5.2 Retry Budget and Pacing	138

13.5.3 Null Address Fast-Path	138

13.5.4 AXI Read Transaction	138

13.5.5 Channel Reset	139

13.6 MonBus Reporting	139

13.7 Descriptor Field Mapping	140

14 Control-Write Engine Specification	141

14.1 Overview	141

14.1.1 Key Features	141

14.1.2 Block Diagram	142

14.1.3 Figure 2.9.1: Control-Write Engine Block Diagram	142

14.2 Parameters	142

14.3 Port List	143

14.3.1 Clock and Reset	143

14.3.2 Configuration Interface	143

14.3.3 Scheduler Interface	143

14.3.4 AXI4 Write Master Interface	144

14.3.5 MonBus Interface	145

14.4 FSM States	145

14.4.1 Figure 2.9.2: Control-Write Engine FSM	145

14.5 Operation	147

14.5.1 Doorbell Write Sequence	147

14.5.2 Figure 2.9.3: Control-Write Doorbell Timing	147

14.5.3 Request Handshake and Skid Buffer	148

14.5.4 Address Validation	148

14.5.5 Error Reporting	148

14.5.6 Channel Reset	148

14.6 Contrast with the Control-Read Engine	149

15 Scheduler Group	151

15.1 Overview	151

15.1.1 Key Features	151

15.1.2 Block Diagram	152

15.1.3 Figure 3.1.1: Scheduler Group Block Diagram	152

15.2 Parameters	152

15.3 Port List	153

15.3.1 Clock, Reset and Kick-Off	153

15.3.2 Configuration Interface	154

15.3.3 Status	155

15.3.4 Descriptor AXI Master Interface	155

15.3.5 Control Read and Control Write AXI Interfaces	156

15.3.6 Scheduler Data Interface	156

15.3.7 Packet Record Interface	157

15.3.8 MonBus Interface	158

15.4 Internal Architecture	158

15.4.1 Descriptor Path	158

15.4.2 Control Engines	159

15.4.3 MonBus Aggregation	159

15.5 Differences from RAPIDS Beats	159

16 Scheduler Group Array	160

16.1 Overview	160

16.1.1 Key Features	160

16.1.2 Block Diagram	161

16.1.3 Figure 3.2.1: Scheduler Group Array Block Diagram	161

16.2 Parameters	161

16.3 Port List	162

16.3.1 Clock, Reset and Kick-Off	162

16.3.2 Global Scheduler and Descriptor Engine Configuration	163

16.3.3 Descriptor Monitor Configuration and Status	164

16.3.4 Descriptor-AXI Perf Window	165

16.3.5 Status	166

16.3.6 Shared AXI Masters	166

16.3.7 Per-Channel Scheduler Data Interface	166

16.3.8 Per-Channel Packet Record Interface	167

16.3.9 MonBus Interface	168

16.4 Shared Master Arbitration	168

16.4.1 Figure 3.2.2: Shared Master Arbitration	169

16.5 MonBus Aggregation	169

16.6 Differences from RAPIDS Beats	170

17 Sink Data Path	171

17.1 Overview	171

17.1.1 Key Features	171

17.1.2 Figure 3.3.1: Sink Data Path Block Diagram	172

17.2 Parameters	172

17.3 Interfaces	173

17.3.1 Fill Interface	173

17.3.2 Configuration	174

17.3.3 Scheduler Interface	174

17.3.4 AXI4 Write Master	175

17.4 Byte Enable Path	175

17.4.1 Channel Reset	175

17.5 Differences from RAPIDS Beats	175

18 Sink Data Path with AXIS (Ingress)	177

18.1 Overview	177

18.1.1 Key Features	177

18.1.2 Figure 3.4.1: Ingress Shifter	178

18.2 Parameters	178

18.3 Interfaces	179

18.3.1 AXI-Stream Slave	179

18.3.2 Scheduler Interface	179

18.3.3 Other Ports	180

18.4 Ingress Operation	180

18.4.1 Packet Records	180

18.4.2 Placement	180

18.4.3 Flush and Completion	181

18.4.4 Length Check	181

18.4.5 Channel Reset	181

18.4.6 Allocation	182

18.5 Differences from RAPIDS Beats	183

19 Sink SRAM Controller Specification	184

19.1 Overview	184

19.1.1 Key Features	184

19.1.2 Block Diagram	185

19.1.3 Figure 3.5.1: Sink SRAM Controller Block Diagram	185

19.2 Parameters	185

19.3 Port List	186

19.3.1 Clock and Reset	186

19.3.2 Fill Interface (ID-Selected)	186

19.3.3 Arbitrated Drain Interface (to AXI Write Engine)	186

19.3.4 Debug Interface	187

19.4 Drain Selection	187

19.4.1 Figure 3.5.2: Drain Selection by the Consumer	187

19.5 Integration with AXI Write Engine	188

20 Source Data Path Specification	190

20.1 Overview	190

20.1.1 Key Features	190

20.1.2 Block Diagram	191

20.1.3 Figure 3.6.1: Source Data Path Block Diagram	191

20.2 Parameters	191

20.3 Data Flow	192

20.3.1 Figure 3.6.2: Source Path Data Flow Sequence	192

20.4 Port List	193

20.4.1 Clock and Reset	193

20.4.2 Configuration	193

20.4.3 Scheduler Interface	193

20.4.4 AXI Read Master Interface	193

20.4.5 Drain Interface (Output to Network)	194

20.5 Timing Diagram	195

20.5.1 Figure 3.6.3: Source Path Transfer Timing	195

20.6 Integration Example	195

21 Source Data Path with AXIS (Egress)	197

21.1 Overview	197

21.1.1 Key Features	197

21.1.2 Figure 3.7.1: Egress Shifter	198

21.2 Parameters	198

21.3 Interfaces	199

21.3.1 Configuration	199

21.3.2 Scheduler Interface	199

21.3.3 AXI-Stream Master	200

21.3.4 Other Ports	201

21.4 Egress Operation	201

21.4.1 Reservation and Drain	201

21.4.2 Pop	201

21.4.3 Shift and Hold	201

21.4.4 Strobe and Last	202

21.4.5 Waveform 3.7: Egress Re-pack	202

21.5 Channel Reset	202

21.6 Differences from RAPIDS Beats	203

22 Source SRAM Controller Specification	205

22.1 Overview	205

22.1.1 Key Features	205

22.1.2 Block Diagram	206

22.1.3 Figure 3.8.1: Source SRAM Controller Block Diagram	206

22.2 Parameters	206

22.3 Port List	207

22.3.1 Clock and Reset	207

22.3.2 Fill Interface (ID-Selected, from AXI Read Engine)	207

22.3.3 Arbitrated Drain Interface (to Network)	207

22.3.4 Debug Interface	208

22.4 Drain Selection	208

22.4.1 Figure 3.8.2: Drain Selection by the Consumer	208

22.5 Comparison: Sink vs Source SRAM Controller	209

22.6 Integration Example	209

23 RAPIDS Core, Sink Half and Source Half	211

23.1 Overview	211

23.1.1 Figure 3.9.1: Core Block Diagram	211

23.2 Byte-Granular Content	212

23.2.1 Packet Record Nets	212

23.2.2 Port Renames	212

23.2.3 Data Ports	212

23.3 Parameters	213

23.3.1 Channel Reset Routing	214

23.4 Differences from RAPIDS Beats	215

24 RAPIDS Registers Specification	216

24.1 Overview	216

24.2 Base Register Map (0x100-0x3FF)	216

24.2.1 Key Register Fields	218

24.3 Monitor Register Map (rapids_mon_regs @ 0x1000)	219

25 RAPIDS Config Block Specification	221

25.1 Overview	221

25.2 Mapping Groups	221

25.2.1 Base Configuration	221

25.2.2 Monitor Configuration (hwif_out.MON.*)	222

25.2.3 Performance and Observation	222

26 RAPIDS Top	224

26.1 Overview	224

26.1.1 Figure 3.12.1: Top Hierarchy	224

26.2 APB Address Map	225

26.3 Parameters	225

26.4 Differences from RAPIDS Beats	226

27 AXI4 Interface Specification	228

27.1 Overview	228

27.2 Byte Address to Beat Mapping	228

27.3 Burst Rules	229

27.4 Write Strobes	229

27.5 Limitations	229

28 AXI-Stream Interface Specification	231

28.1 Overview	231

28.2 Packet Contract	231

28.3 Source Master Signals	232

28.4 Sink Slave Signals	232

28.5 Length Check	233

29 MonBus Interface Specification	234

29.1 Overview	234

29.2 MonBus Signal Interface	234

29.2.1 Signal Table	234

29.2.2 Handshaking Protocol	234

29.2.3 Timing Diagram	235

29.2.4 Figure 4.3.1: MonBus Packet Transfer Timing	235

29.3 MonBus Packet Format	235

29.3.1 64-bit Packet Structure	235

29.3.2 Figure 4.3.2: MonBus Packet Format	235

29.4 Packet Types	236

29.4.1 Type Code Definitions	236

29.4.2 Payload Formats by Type	237

29.5 MonBus Source Aggregation	238

29.5.1 RAPIDS Core MonBus Sources	238

29.5.2 Aggregation Hierarchy	239

29.5.3 Figure 4.3.3: MonBus Aggregation Tree	239

29.6 Integration Guidelines	239

29.6.1 Consumer Requirements	239

29.6.2 Recommended Consumer Patterns	239

29.7 Timing Requirements	240

30 Packet Record Interface Specification	241

30.1 Overview	241

30.1.1 Figure 4.4.1: Scheduler Byte Math	241

30.2 Signals	242

30.3 Protocol	242

30.4 Unused Direction	243

30.5 Consumer Behavior	243

31 RAPIDS Beats Architecture MAS	244

31.1 Document Organization	244

31.1.1 Front Matter	244

31.1.2 Chapter 1: Overview	244

31.1.3 Chapter 2: FUB (Functional Unit Blocks)	244

31.1.4 Chapter 3: Macro (Integration Blocks)	244

31.1.5 Chapter 4: Interfaces	245

31.2 Quick Reference	245

31.2.1 FUB Modules (fub_beats/)	245

31.2.2 Macro Modules (macro_beats/)	246

31.3 Architecture Comparison: RAPIDS vs STREAM	247

31.4 Clock and Reset Summary	248

31.4.1 Clock Domains	248

31.4.2 Reset Signals	248

31.5 Interface Summary	249

31.5.1 External Interfaces	249

31.5.2 Internal Buses	249

31.6 Related Documentation	249

31.7 Specification Conventions	250

31.7.1 Signal Naming	250

31.7.2 Parameter Naming	250

31.7.3 Figure and Table Numbering	250

List of Figures

Figure 1.1.1: RAPIDS Architecture Block Diagram	37

Figure 1.1.2: Sink Path Data Flow	38

Figure 1.1.3: Source Path Data Flow	38

Figure 1.3.1: Reset Timing	68

Figure 2.1.1: Scheduler Byte Math	73

Figure 2.2.1: Descriptor Engine Block Diagram	86

Figure 2.2.2: Descriptor Engine FSM	90

Figure 2.2.3: Descriptor Chain Timing	91

Figure 2.3.1: AXI Read Engine Block Diagram	95

Figure 2.3.2: Burst Segmentation with a Mid-Beat Start	100

Figure 2.4.1: AXI Write Engine Block Diagram	104

Figure 2.5.1: Beats Alloc Control Block Diagram	114

Figure 2.5.2: Allocation and Release Timing	117

Figure 2.6.1: Beats Drain Control Block Diagram	120

Figure 2.6.2: Data Arrival and Drain Timing	123

Figure 2.7.1: Latency Bridge Block Diagram	126

Figure 2.7.2: Latency Bridge Timing (consumer always ready)	129

Figure 2.8.1: Control-Read Engine Block Diagram	133

Figure 2.8.2: Control-Read Engine FSM	137

Figure 2.9.1: Control-Write Engine Block Diagram	142

Figure 2.9.2: Control-Write Engine FSM	145

Figure 2.9.3: Control-Write Doorbell Timing	147

Figure 3.1.1: Scheduler Group Block Diagram	152

Figure 3.2.1: Scheduler Group Array Block Diagram	161

Figure 3.2.2: Shared Master Arbitration	169

Figure 3.3.1: Sink Data Path Block Diagram	172

Figure 3.4.1: Ingress Shifter	178

Figure 3.5.1: Sink SRAM Controller Block Diagram	185

Figure 3.5.2: Drain Selection by the Consumer	187

Figure 3.6.1: Source Data Path Block Diagram	191

Figure 3.6.2: Source Path Data Flow Sequence	192

Figure 3.6.3: Source Path Transfer Timing	195

Figure 3.7.1: Egress Shifter	198

Figure 3.8.1: Source SRAM Controller Block Diagram	206

Figure 3.8.2: Drain Selection by the Consumer	208

Figure 3.9.1: Core Block Diagram	211

Figure 3.12.1: Top Hierarchy	224

Figure 4.3.1: MonBus Packet Transfer Timing	235

Figure 4.3.2: MonBus Packet Format	235

Figure 4.3.3: MonBus Aggregation Tree	239

Figure 4.4.1: Scheduler Byte Math	241

List of Tables

Table 1: Table 0.1: Revision History	33

Table 2: Table 0.2: Related Documents and Specifications	33

Table 3: Table 1.1.1: Block Roles	37

Table 4: Table 1.1.2: Design Points	39

Table 5: Table 1.2.1: Parameters (from rapids_core_beats.sv)	41

Table 6: Table 1.2.2: Clock and Reset (shared - single clock/reset for the whole wrapper)	43

Table 7: Table 1.2.3: APB Programming Interface	43

Table 8: Table 1.2.4: Per-channel configuration	43

Table 9: Table 1.2.5: Scheduler Configuration (global)	44

Table 10: Table 1.2.6: Descriptor Engine Configuration (global)	44

Table 11: Table 1.2.7: Control Engine Configuration (Phase 2, global)	45

Table 12: Table 1.2.8: Descriptor AXI Monitor Configuration	45

Table 13: Table 1.2.9: AXIS data-path monitor-lite configuration (rapids TASK-015)	46

Table 14: Table 1.2.10: Status	47

Table 15: Table 1.2.11: Descriptor AXI Monitor Status	47

Table 16: Table 1.2.12: AXIS data-path monitor-lite status (rapids TASK-015)	47

Table 17: Table 1.2.13: Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).	48

Table 18: Table 1.2.14: Descriptor Fetch AXI Master	49

Table 19: Table 1.2.15: Control Read AXI Master (32-bit) [Phase 2]	50

Table 20: Table 1.2.16: Control Write AXI Master (32-bit) [Phase 2]	51

Table 21: Table 1.2.17: AXI Transfer Configuration (source-only)	52

Table 22: Table 1.2.18: Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id	52

Table 23: Table 1.2.19: AXI4 Master - Data Read (Memory -> Source SRAM)	53

Table 24: Table 1.2.20: APB Programming Interface	53

Table 25: Table 1.2.21: Per-channel configuration	54

Table 26: Table 1.2.22: Scheduler Configuration (global)	54

Table 27: Table 1.2.23: Descriptor Engine Configuration (global)	54

Table 28: Table 1.2.24: Control Engine Configuration (Phase 2, global)	55

Table 29: Table 1.2.25: Descriptor AXI Monitor Configuration	55

Table 30: Table 1.2.26: AXIS data-path monitor-lite configuration (rapids TASK-015)	56

Table 31: Table 1.2.27: Status	57

Table 32: Table 1.2.28: Descriptor AXI Monitor Status	57

Table 33: Table 1.2.29: AXIS data-path monitor-lite status (rapids TASK-015)	58

Table 34: Table 1.2.30: Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).	58

Table 35: Table 1.2.31: Descriptor Fetch AXI Master	59

Table 36: Table 1.2.32: Control Read AXI Master (32-bit) [Phase 2]	60

Table 37: Table 1.2.33: Control Write AXI Master (32-bit) [Phase 2]	61

Table 38: Table 1.2.34: Monitor Bus	62

Table 39: Table 1.2.35: AXI Transfer Configuration (sink-only)	62

Table 40: Table 1.2.36: Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id	63

Table 41: Table 1.2.37: AXI4 Master - Data Write (Sink SRAM -> Memory)	63

Table 42: Table 1.2.38: Source half debug	64

Table 43: Table 1.2.39: Sink half debug	65

Table 44: Table 1.2.40: Active-channel sideband for per-channel bus instrumentation	65

Table 45: Table 1.3.1: Clock Specifications	67

Table 46: Table 1.3.2: Reset Specifications	68

Table 47: Table 1.3.3: Reset States	69

Table 48: Table 1.3.4: Configuration Timing Requirements	70

Table 49: Table 2.1.1: Scheduler Parameters	73

Table 50: Table 2.1.2: Clock, Reset and Configuration Ports	74

Table 51: Table 2.1.3: Status Ports	75

Table 52: Table 2.1.4: Descriptor Engine Interface	75

Table 53: Table 2.1.5: Data Read Interface	76

Table 54: Table 2.1.6: Data Write Interface	76

Table 55: Table 2.1.7: Completion and Error Interface	77

Table 56: Table 2.1.8: Control Engine Interface	77

Table 57: Table 2.1.9: Monitor Bus Interface	78

Table 58: Table 2.1.10: Descriptor Layout	78

Table 59: Table 2.1.11: Beat Totals for a 32-byte Beat	79

Table 60: Table 2.1.12: State Encoding	81

Table 61: Table 2.1.13: Transitions	82

Table 62: Table 2.1.14: MonBus Events	84

Table 63: Table 2.2.2: Clock and Reset	87

Table 64: Table 2.2.3: APB Programming Interface	87

Table 65: Table 2.2.4: Configuration Interface	88

Table 66: Table 2.2.5: AXI4 Read Master Interface	88

Table 67: Table 2.2.6: Scheduler Interface	89

Table 68: Table 2.3.1: AXI Read Engine Parameters	95

Table 69: Table 2.3.2: AXI Read Engine Derived Constants	96

Table 70: Table 2.3.3: Clock, Reset and Configuration	97

Table 71: Table 2.3.4: Scheduler Interface	97

Table 72: Table 2.3.5: SRAM Allocation and Fill Interface	98

Table 73: Table 2.3.6: AXI4 Read Master Interface	98

Table 74: Table 2.3.7: Debug Outputs	99

Table 75: Table 2.3.8: Burst Segmentation Example	100

Table 76: Table 2.3.9: Error Handling	101

Table 77: Table 2.3.10: Read Engine Delta	102

Table 78: Table 2.4.1: AXI Write Engine Parameters	104

Table 79: Table 2.4.2: AXI Write Engine Derived Constants	105

Table 80: Table 2.4.3: Clock, Reset and Configuration	106

Table 81: Table 2.4.4: Scheduler Interface	106

Table 82: Table 2.4.5: SRAM Drain Interface	107

Table 83: Table 2.4.6: AXI4 Write Master Interface	108

Table 84: Table 2.4.7: Debug and Status Outputs	109

Table 85: Table 2.4.8: Completion Strobes	110

Table 86: Table 2.4.9: Error Handling	111

Table 87: Table 2.4.10: Write Engine Delta	112

Table 88: Table 2.5.2: Clock and Reset	115

Table 89: Table 2.5.3: Write Interface	115

Table 90: Table 2.5.4: Read Interface	116

Table 91: Table 2.5.5: Status Outputs	116

Table 92: Table 2.6.2: Clock and Reset	121

Table 93: Table 2.6.3: Write Interface	121

Table 94: Table 2.6.4: Read Interface	121

Table 95: Table 2.6.5: Status Outputs	122

Table 96: Table 2.6.6: alloc_ctrl vs drain_ctrl Comparison	124

Table 97: Table 2.7.2: Clock and Reset	127

Table 98: Table 2.7.3: Upstream Interface	127

Table 99: Table 2.7.4: Downstream Interface	127

Table 100: Table 2.7.5: Status and Debug	128

Table 101: Table 2.7.6: Skid Depth Selection	130

Table 102: Table 2.8.2: Clock and Reset	134

Table 103: Table 2.8.3: Scheduler Interface	134

Table 104: Table 2.8.4: Configuration Interface	135

Table 105: Table 2.8.5: AXI4 Read Master Interface	135

Table 106: Table 2.8.6: MonBus Interface	136

Table 107: Table 2.8.7: MonBus Events	139

Table 108: Table 2.8.8: CTRL_READ Descriptor Field Mapping	140

Table 109: Table 2.9.2: Clock and Reset	143

Table 110: Table 2.9.3: Configuration Interface	143

Table 111: Table 2.9.4: Scheduler Interface	143

Table 112: Table 2.9.5: AXI4 Write Master Interface	144

Table 113: Table 2.9.6: MonBus Interface	145

Table 114: Table 3.1.1: Scheduler Group Parameters	152

Table 115: Table 3.1.2: Clock, Reset and Kick-Off	153

Table 116: Table 3.1.3: Configuration Interface	154

Table 117: Table 3.1.4: Status Interface	155

Table 118: Table 3.1.5: Descriptor AXI Master Interface	155

Table 119: Table 3.1.6: Scheduler Data Interface	156

Table 120: Table 3.1.7: Packet Record Interface	157

Table 121: Table 3.1.8: MonBus Interface	158

Table 122: Table 3.1.9: Scheduler Group Delta	159

Table 123: Table 3.2.1: Scheduler Group Array Parameters	161

Table 124: Table 3.2.2: Clock, Reset and Kick-Off	162

Table 125: Table 3.2.3: Global Configuration	163

Table 126: Table 3.2.4: Descriptor Monitor Configuration and Status	164

Table 127: Table 3.2.5: Descriptor-AXI Perf Window	165

Table 128: Table 3.2.6: Status	166

Table 129: Table 3.2.7: Per-Channel Scheduler Data Interface	166

Table 130: Table 3.2.8: Per-Channel Packet Record Interface	167

Table 131: Table 3.2.9: MonBus Interface	168

Table 132: Table 3.2.10: MonBus Source Assignment	169

Table 133: Table 3.2.11: Array Delta	170

Table 134: Table 3.3.1: Sink Data Path Parameters	172

Table 135: Table 3.3.2: Fill Interface	173

Table 136: Table 3.3.3: Configuration	174

Table 137: Table 3.3.4: Scheduler Interface	174

Table 138: Table 3.3.5: Sink Data Path Delta	175

Table 139: Table 3.4.1: Additional Parameters	178

Table 140: Table 3.4.2: AXI-Stream Slave	179

Table 141: Table 3.4.3: Packet Record Ports	179

Table 142: Table 3.4.4: Channel Reset Actions	181

Table 143: Table 3.4.5: Ingress Delta	183

Table 144: Table 3.5.2: Clock and Reset	186

Table 145: Table 3.5.3: Fill Interface (ID-Selected)	186

Table 146: Table 3.5.4: Arbitrated Drain Interface	186

Table 147: Table 3.5.5: Debug Interface	187

Table 148: Table 3.6.2: Clock and Reset	193

Table 149: Table 3.6.3: Configuration	193

Table 150: Table 3.6.4: Scheduler Interface	193

Table 151: Table 3.6.5: AXI Read Master Interface	193

Table 152: Table 3.6.6: Drain Interface	194

Table 153: Table 3.7.1: Parameters	198

Table 154: Table 3.7.2: Configuration	199

Table 155: Table 3.7.3: Scheduler Interface	199

Table 156: Table 3.7.4: AXI-Stream Master	200

Table 157: Table 3.7.5: Egress Shift Rules	201

Table 158: Table 3.7.6: Channel Reset Actions	203

Table 159: Table 3.7.7: Egress Delta	203

Table 160: Table 3.8.2: Clock and Reset	207

Table 161: Table 3.8.3: Fill Interface (ID-Selected)	207

Table 162: Table 3.8.4: Arbitrated Drain Interface	207

Table 163: Table 3.8.5: Debug Interface	208

Table 164: Table 3.8.6: Sink vs Source Controller Comparison	209

Table 165: Table 3.9.1: Packet Record Nets per Half	212

Table 166: Table 3.9.2: Core Port Renames	212

Table 167: Table 3.9.3: Data Ports	212

Table 168: Table 3.9.4: Core Parameters	213

Table 169: Table 3.9.5: Channel Reset Routing	214

Table 170: Table 3.9.6: Core Delta	215

Table 171: Table 3.10.1: Base Register Map	216

Table 172: Table 3.10.2: Monitor Register Map (base 0x1000)	219

Table 173: Table 3.11.1: Monitor Register-to-Config Mapping	222

Table 174: Table 3.12.1: APB Address Map	225

Table 175: Table 3.12.2: Top Parameters	225

Table 176: Table 3.12.3: Top Delta	226

Table 177: Table 4.1.1: Address Split	228

Table 178: Table 4.1.2: Burst Rules	229

Table 179: Table 4.1.3: Byte-Granular Write Signal	229

Table 180: Table 4.2.1: Packet Contract	231

Table 181: Table 4.2.2: Source Master Signals	232

Table 182: Table 4.2.3: Sink Slave Signals	232

Table 183: Table 4.3.1: MonBus Interface Signals	234

Table 184: Table 4.3.2: MonBus Packet Fields	235

Table 185: Table 4.3.3: MonBus Packet Types	236

Table 186: Table 4.3.4: Descriptor Packet Payload	237

Table 187: Table 4.3.5: Scheduler Packet Payload	237

Table 188: Table 4.3.6: AXI Event Packet Payload	237

Table 189: Table 4.3.7: Error Packet Payload	237

Table 190: Table 4.3.8: MonBus Source Assignment	238

Table 191: Table 4.3.9: MonBus Timing Parameters	240

Table 192: Table 4.4.1: Packet Record Signals	242

Table 193: Table 4.4.2: Record Rules	242

Table 194: FUB Module Summary	245

Table 195: Macro Module Summary	246

Table 196: RAPIDS Beats vs STREAM Comparison	247

Table 197: Clock Domains	248

Table 198: Reset Signals	248

Table 199: External Interfaces	249

Table 200: Internal Buses	249

List of Waveforms

Waveform 2.1: Byte Descriptor Start	84

Waveform 3.7: Egress Re-pack	202

# 1	Rapids Mas Index

Generated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

This document describes RAPIDS, the byte-granular Rapid AXI Programmable In-band Descriptor System. RAPIDS provides network-to-memory and memory-to-network data transfer using descriptor-based DMA with 8 channels. A descriptor length is a byte count, source and destination addresses are byte addresses, and the sink write master drives byte enables. The earlier “Beats” architecture, which counted everything in beats, was the stepping stone to this design and is documented separately in the RAPIDS Beats MAS.

## 2.1	Revision History

Table 0.1: Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.2 | 2026-09-30 | RTL Design Sherpa | Channel reset recovers every error class: cfg_channel_reset routed into the AXI read and write engines, both data paths and the SRAM wrappers; scheduler clears sticky flags on the reset. |
| 0.1 | 2026-09-30 | RTL Design Sherpa | First byte-granular MAS. Rewrites the chapters that changed from RAPIDS Beats and links the rest. |

## 2.2	References

### 2.2.1	Related Documents

Table 0.2: Related Documents and Specifications

| Source | Title | Version |
| --- | --- | --- |
| RTL Design Sherpa | RAPIDS Beats MAS (shared chapters) | 0.7 |
| RTL Design Sherpa | RAPIDS HAS | 0.2 |
| RTL Design Sherpa | RAPIDS Product Requirements Document | 1.0 |
| ARM | AMBA AXI and ACE Protocol Specification | IHI0022H |
| ARM | AMBA AXI-Stream Protocol Specification | IHI0051A |

## 2.3	Terminology

Beat

One data transfer of an AXI burst or an AXI-Stream transfer. One beat is BYTE_LANES bytes, where BYTE_LANES = DATA_WIDTH/8. The board design uses 256 bits, so a beat is 32 bytes.

Byte offset

The low OFF_W bits of a byte address, where OFF_W = $clog2(BYTE_LANES). It is the lane at which a transfer begins inside its first memory beat.

Memory beat

A beat as it appears on the AXI side, aligned to BYTE_LANES. A packet that starts at a non-zero offset occupies one more memory beat than a stream of the same length would.

Stream beat

A beat as it appears on AXI-Stream, packed from lane 0 with no offset.

Packet record

The pair {bytes, offset} that the scheduler issues once per DATA descriptor, so the AXIS shifter knows how the memory-side and stream-side layouts relate.

Spill

The bytes of a stream beat that did not fit in the current memory beat during ingress shifting. They are held for the next memory beat.

Channel

One of 8 independent DMA channels. Each has its own descriptor chain, scheduler and SRAM allocation.

Descriptor

A 256-bit structure holding the source address, destination address, length in bytes, next pointer, and control flags.

TYPE=EXT

The descriptor type that describes a row/column transfer. It stays beat-aligned by design: addresses are aligned and lengths are multiples of the beat size.

DMA

Direct Memory Access. Data movement without processor involvement.

MonBus

The 64-bit monitor bus that carries event packets out of the design.

## 2.4	Document Conventions

- Shared chapters are linked from the RAPIDS Beats MAS and marked “(shared with RAPIDS Beats)” in the index. Module names in those chapters carry a _beats suffix that this design drops.

- Requirement priorities follow the RAPIDS PRD.

- No emojis appear in this document.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Architecture Overview

Module: rapids_core.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 3.1	Overview

RAPIDS moves data between AXI-Stream and system memory in units of bytes. A descriptor’s length is a byte count in the existing 32-bit field, and its source and destination are byte addresses. A linear descriptor may start at any byte on either side. The design keeps the streaming, no-FSM data path of RAPIDS Beats and adds three things around it: byte-to-beat math in the scheduler, byte enables through the sink SRAM to the write master, and a shifter at each AXI-Stream edge that converts between packed stream beats and memory-aligned beats.

### 3.1.1	Key Features

- 8 Independent Channels: Each channel has its own descriptor chain and SRAM allocation.

- Separate Data Paths: Sink (network to memory) and source (memory to network).

- Byte Granularity: Length in bytes, byte addresses, any starting offset for linear descriptors.

- Byte Enables: The sink SRAM entry is {strb, data}; the write engine drives WSTRB from it.

- Packet Records: The scheduler issues one {bytes, offset} record per DATA descriptor to the AXIS shifter.

- One AXIS Packet per Descriptor: The source path emits exactly one packet, with TLAST on the final byte.

- Safe Bursts: AXI bursts never cross a 4 KB boundary; AxADDR is beat-aligned.

- MonBus Integration: Same monitor packets as RAPIDS Beats.

### 3.1.2	What Is Shared with RAPIDS Beats

The descriptor engine, the control-read and control-write engines, the SRAM controllers, the register block and config block, and the MonBus specification are unchanged. Those chapters are linked from the RAPIDS Beats MAS.

## 3.2	Block Diagram

### 3.2.1	Figure 1.1.1: RAPIDS Architecture Block Diagram

RAPIDS core block diagram

Source: 04_rapids_core.dot

Source: rapids_core.sv, rapids_snk.sv, rapids_src.sv

Table 1.1.1: Block Roles

| Block | Role |
| --- | --- |
| scheduler | Loads a descriptor, computes beats from offset and length, issues the packet record, tracks completion |
| scheduler_group_array | Eight scheduler groups sharing one descriptor AXI master; per-channel packet-record ports |
| snk_data_path_axis | Converts packed AXIS beats into memory-aligned beats with byte enables; checks the packet length |
| snk_data_path | SRAM of {strb, data} entries plus the AXI write engine |
| src_data_path | SRAM of data entries plus the AXI read engine; unchanged from Beats |
| src_data_path_axis | Converts memory-aligned beats into packed stream beats with a partial last beat |

## 3.3	Data Flow Overview

### 3.3.1	Sink Path (Network to Memory)

### 3.3.2	Figure 1.1.2: Sink Path Data Flow

1. Scheduler loads a descriptor, computes beats_total, pulses sched_wr_pkt_valid
        |   (record {bytes, offset} enters the per-channel record queue)
        v
2. s_axis packet arrives; tready is held low until the channel's record is known
        |
        v
3. Ingress shifter: stream beat << (offset*8), OR the spill of the previous beat
        |   low DATA_WIDTH bits = this memory beat, high bits become the new spill
        v
4. Fill logic writes {strb, data} into the sink SRAM (snk_sram_controller)
        |
        v
5. axi_write_engine: aligned AWADDR, capped burst, WSTRB from the SRAM entry
        |
        v
6. System memory; B response commits the beats and completes the descriptor

### 3.3.3	Source Path (Memory to Network)

### 3.3.4	Figure 1.1.3: Source Path Data Flow

1. Scheduler loads a descriptor, computes beats_total, pulses sched_rd_pkt_valid
        |
        v
2. axi_read_engine: aligned ARADDR, capped burst, beats into the source SRAM
        |
        v
3. src_data_path_axis reserves drain blocks (round-robin, cfg_drain_size beats)
        |
        v
4. Egress shifter: memory beat >> (offset*8), OR the next beat's low lanes
        |   the first pop only primes the hold; the last beat is partial
        v
5. m_axis: TDATA packed from lane 0, TSTRB contiguous, TLAST on the final byte

## 3.4	Byte-Granular Design Points

Table 1.1.2: Design Points

| Point | Rule |
| --- | --- |
| Length unit | Bytes, in the existing 32-bit descriptor field |
| Beats moved | (offset + length + BYTE_LANES - 1) >> OFF_W, zero when length is zero |
| First burst | May begin mid-beat; the address it presents is beat-aligned and the offset is carried by the record |
| Later bursts | Beat-aligned |
| 4 KB rule | A burst is truncated at the next 4 KB boundary |
| Sink lanes not written | WSTRB bit is 0 |
| Source last beat | Partial; TSTRB marks the valid low lanes, TLAST set |
| TYPE=EXT | Beat-aligned by design (permanent): aligned addresses, beat-multiple lengths |

The 4 KB rule and the byte math are specified in Scheduler and AXI Read Engine.

## 3.5	Chapter Map

| Topic | Chapter |
| --- | --- |
| Scheduler byte math and record issue | Scheduler |
| Record queue interface | Packet Record Interface |
| Ingress shifter | Sink Data Path AXIS |
| Egress shifter | Source Data Path AXIS |
| Everything else | Linked from the RAPIDS Beats MAS via the index |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Top-Level Port List

Module: rapids_core_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 4.1	Parameters

Every parameter of rapids_core_beats, as declared. The short aliases at the end are what the port widths below are written in (NC, AW, DW, IW, SCW, CIW, SW).

Table 1.2.1: Parameters (from rapids_core_beats.sv)

| Parameter | Type | Default | Notes |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | - |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | - |
| ADDR_WIDTH | int | 64 | - |
| DATA_WIDTH | int | 512 | - |
| AXI_ID_WIDTH | int | 8 | - |
| SRAM_DEPTH | int | 512 | - |
| SEG_COUNT_WIDTH | int | $clog2(SRAM_DEPTH) + 1 | - |
| PIPELINE | int | 1 | write-engine pipelining; 1 is the perf design point (see rapids_beats_top) |
| AR_MAX_OUTSTANDING | int | 8 | - |
| AW_MAX_OUTSTANDING | int | 8 | - |
| W_PHASE_FIFO_DEPTH | int | 64 | - |
| B_PHASE_FIFO_DEPTH | int | 16 | - |
| AXIS_ID_WIDTH | int | 8 | - |
| AXIS_DEST_WIDTH | int | 4 | - |
| AXIS_USER_WIDTH | int | 1 | - |
| DESC_MON_BASE_AGENT_ID | int | 16 | 0x10 - Descriptor Engines (16-23) |
| SCHED_MON_BASE_AGENT_ID | int | 48 | 0x30 - Schedulers (48-55) |
| DESC_AXI_MON_AGENT_ID | int | 8 | 0x08 - Descriptor AXI Master Monitor |
| SNK_AXIS_MON_AGENT_ID | int | 9 | 0x09 - Sink-ingress AXIS monitor (rapids TASK-015) |
| SRC_AXIS_MON_AGENT_ID | int | 10 | 0x0A - Source-egress AXIS monitor (rapids TASK-015) |
| ACLK_MHZ | int | 100 | clk in MHz: the AXIS monitors’ microsecond tick |
| MON_UNIT_ID | int | 1 | 0x1 |
| MON_MAX_TRANSACTIONS | int | 16 | - |
| USE_AXI_MONITORS | int | 1 | - |
| USE_ROW_COL_MAJOR_ADDRESSING | int | 1 | - |
| GEN_MON | bit | 1'b1 | - |
| NC | int | NUM_CHANNELS | - |
| AW | int | ADDR_WIDTH | - |
| DW | int | DATA_WIDTH | - |
| IW | int | AXI_ID_WIDTH | - |
| SD | int | SRAM_DEPTH | - |
| SCW | int | SEG_COUNT_WIDTH | - |
| CIW | int | (NC > 1) ? $clog2(NC) : 1 | - |
| SW | int | DW / 8 | - |

## 4.2	Port List (324 ports)

The module is two independent halves. Ports both halves carry are prefixed src_ / snk_; the direction-unique ports (the source’s AXI read master and AXIS egress, the sink’s AXI write master and AXIS ingress) carry no prefix. Sections follow the declaration order in the RTL, so a reader can walk this page and the module side by side. Widths use the parameter aliases.

### 4.2.1	Clock and Reset (shared - single clock/reset for the whole wrapper)

Table 1.2.2: Clock and Reset (shared - single clock/reset for the whole wrapper)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | - |
| rst_n | input | 1 | - |

## 4.3	SOURCE HALF (u_src) - shared-infrastructure ports (src_ prefixed)

### 4.3.1	APB Programming Interface

Table 1.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_apb_valid | input | NC | - |
| src_apb_ready | output | NC | - |
| src_apb_addr | input | NC x AW | - |

### 4.3.2	Per-channel configuration

Table 1.2.4: Per-channel configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_channel_enable | input | NC | - |
| src_cfg_channel_reset | input | NC | - |

### 4.3.3	Scheduler Configuration (global)

Table 1.2.5: Scheduler Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_sched_enable | input | 1 | - |
| src_cfg_sched_timeout_cycles | input | 32 | - |
| src_cfg_sched_timeout_limit | input | 8 | - |
| src_cfg_sched_timeout_enable | input | 1 | - |
| src_cfg_sched_err_enable | input | 1 | - |
| src_cfg_sched_compl_enable | input | 1 | - |
| src_cfg_sched_perf_enable | input | 1 | - |

### 4.3.4	Descriptor Engine Configuration (global)

Table 1.2.6: Descriptor Engine Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_desceng_enable | input | 1 | - |
| src_cfg_desceng_prefetch | input | 1 | - |
| src_cfg_desceng_fifo_thresh | input | 4 | - |
| src_cfg_desceng_addr0_base | input | AW | - |
| src_cfg_desceng_addr0_limit | input | AW | - |
| src_cfg_desceng_addr1_base | input | AW | - |
| src_cfg_desceng_addr1_limit | input | AW | - |

### 4.3.5	Control Engine Configuration (Phase 2, global)

Table 1.2.7: Control Engine Configuration (Phase 2, global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_ctrlrd_max_try | input | 9 | - |
| src_tick_1us | input | 1 | - |

### 4.3.6	Descriptor AXI Monitor Configuration

Table 1.2.8: Descriptor AXI Monitor Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_desc_mon_enable | input | 1 | - |
| src_cfg_desc_mon_err_enable | input | 1 | - |
| src_cfg_desc_mon_perf_enable | input | 1 | - |
| src_cfg_desc_mon_perf_run | input | 1 | - |
| src_cfg_desc_mon_timeout_enable | input | 1 | - |
| src_cfg_desc_mon_timeout_cycles | input | 32 | - |
| src_cfg_desc_mon_latency_thresh | input | 32 | - |
| src_cfg_desc_mon_pkt_mask | input | 16 | - |
| src_cfg_desc_mon_err_select | input | 4 | - |
| src_cfg_desc_mon_err_mask | input | 8 | - |
| src_cfg_desc_mon_timeout_mask | input | 8 | - |
| src_cfg_desc_mon_compl_mask | input | 8 | - |
| src_cfg_desc_mon_thresh_mask | input | 8 | - |
| src_cfg_desc_mon_perf_mask | input | 8 | - |
| src_cfg_desc_mon_addr_mask | input | 8 | - |
| src_cfg_desc_mon_debug_mask | input | 8 | - |

### 4.3.7	AXIS data-path monitor-lite configuration (rapids TASK-015)

Table 1.2.9: AXIS data-path monitor-lite configuration (rapids TASK-015)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_axis_mon_enable | input | 1 | - |
| src_cfg_axis_mon_err_enable | input | 1 | - |
| src_cfg_axis_mon_compl_enable | input | 1 | - |
| src_cfg_axis_mon_perf_enable | input | 1 | - |
| src_cfg_axis_mon_timeout_enable | input | 1 | - |
| src_cfg_axis_mon_timeout_cycles | input | 32 | - |
| src_cfg_axis_mon_latency_thresh | input | 32 | - |
| src_cfg_axis_mon_pkt_mask | input | 16 | - |

### 4.3.8	Status

Table 1.2.10: Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_system_idle | output | 1 | - |
| src_descriptor_engine_idle | output | NC | - |
| src_scheduler_idle | output | NC | - |
| src_scheduler_state | output | NC x 7 | - |
| src_sched_error | output | NC | - |

### 4.3.9	Descriptor AXI Monitor Status

Table 1.2.11: Descriptor AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_sts_desc_mon_busy | output | 1 | - |
| src_cfg_sts_desc_mon_active_txns | output | 8 | - |
| src_cfg_sts_desc_mon_error_count | output | 16 | - |
| src_cfg_sts_desc_mon_txn_count | output | 32 | - |
| src_cfg_sts_desc_mon_conflict_error | output | 1 | - |

### 4.3.10	AXIS data-path monitor-lite status (rapids TASK-015)

Table 1.2.12: AXIS data-path monitor-lite status (rapids TASK-015)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_sts_axis_mon_busy | output | 1 | - |
| src_cfg_sts_axis_mon_packet_count | output | 32 | - |
| src_cfg_sts_axis_mon_error_count | output | 16 | - |
| src_cfg_sts_axis_mon_dropped_count | output | 16 | - |

### 4.3.11	Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).

Table 1.2.13: Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_sts_desc_mon_win_active | output | 1 | - |
| src_sts_desc_mon_win_cycles | output | 32 | - |
| src_sts_desc_mon_prod_cycles | output | 32 | - |
| src_sts_desc_mon_bp_cycles | output | 32 | - |
| src_sts_desc_mon_starv_cycles | output | 32 | - |
| src_sts_desc_mon_idle_cycles | output | 32 | - |
| src_sts_desc_mon_beat_count | output | 32 | - |
| src_sts_desc_mon_byte_count | output | 64 | - |
| src_sts_desc_mon_burst_count | output | 32 | - |

### 4.3.12	Descriptor Fetch AXI Master

Table 1.2.14: Descriptor Fetch AXI Master

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_desc_arvalid | output | 1 | - |
| src_m_axi_desc_arready | input | 1 | - |
| src_m_axi_desc_araddr | output | AW | - |
| src_m_axi_desc_arlen | output | 8 | - |
| src_m_axi_desc_arsize | output | 3 | - |
| src_m_axi_desc_arburst | output | 2 | - |
| src_m_axi_desc_arid | output | IW | - |
| src_m_axi_desc_arlock | output | 1 | - |
| src_m_axi_desc_arcache | output | 4 | - |
| src_m_axi_desc_arprot | output | 3 | - |
| src_m_axi_desc_arqos | output | 4 | - |
| src_m_axi_desc_arregion | output | 4 | - |
| src_m_axi_desc_rvalid | input | 1 | - |
| src_m_axi_desc_rready | output | 1 | - |
| src_m_axi_desc_rdata | input | 256 | - |
| src_m_axi_desc_rresp | input | 2 | - |
| src_m_axi_desc_rlast | input | 1 | - |
| src_m_axi_desc_rid | input | IW | - |

### 4.3.13	Control Read AXI Master (32-bit) [Phase 2]

Table 1.2.15: Control Read AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_ctrlrd_arvalid | output | 1 | - |
| src_m_axi_ctrlrd_arready | input | 1 | - |
| src_m_axi_ctrlrd_araddr | output | AW | - |
| src_m_axi_ctrlrd_arlen | output | 8 | - |
| src_m_axi_ctrlrd_arsize | output | 3 | - |
| src_m_axi_ctrlrd_arburst | output | 2 | - |
| src_m_axi_ctrlrd_arid | output | IW | - |
| src_m_axi_ctrlrd_arlock | output | 1 | - |
| src_m_axi_ctrlrd_arcache | output | 4 | - |
| src_m_axi_ctrlrd_arprot | output | 3 | - |
| src_m_axi_ctrlrd_arqos | output | 4 | - |
| src_m_axi_ctrlrd_arregion | output | 4 | - |
| src_m_axi_ctrlrd_rvalid | input | 1 | - |
| src_m_axi_ctrlrd_rready | output | 1 | - |
| src_m_axi_ctrlrd_rdata | input | 32 | - |
| src_m_axi_ctrlrd_rresp | input | 2 | - |
| src_m_axi_ctrlrd_rlast | input | 1 | - |
| src_m_axi_ctrlrd_rid | input | IW | - |

### 4.3.14	Control Write AXI Master (32-bit) [Phase 2]

Table 1.2.16: Control Write AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_ctrlwr_awvalid | output | 1 | - |
| src_m_axi_ctrlwr_awready | input | 1 | - |
| src_m_axi_ctrlwr_awaddr | output | AW | - |
| src_m_axi_ctrlwr_awlen | output | 8 | - |
| src_m_axi_ctrlwr_awsize | output | 3 | - |
| src_m_axi_ctrlwr_awburst | output | 2 | - |
| src_m_axi_ctrlwr_awid | output | IW | - |
| src_m_axi_ctrlwr_awlock | output | 1 | - |
| src_m_axi_ctrlwr_awcache | output | 4 | - |
| src_m_axi_ctrlwr_awprot | output | 3 | - |
| src_m_axi_ctrlwr_awqos | output | 4 | - |
| src_m_axi_ctrlwr_awregion | output | 4 | - |
| src_m_axi_ctrlwr_wvalid | output | 1 | - |
| src_m_axi_ctrlwr_wready | input | 1 | - |
| src_m_axi_ctrlwr_wdata | output | 32 | - |
| src_m_axi_ctrlwr_wstrb | output | 4 | - |
| src_m_axi_ctrlwr_wlast | output | 1 | - |
| src_m_axi_ctrlwr_bvalid | input | 1 | - |
| src_m_axi_ctrlwr_bready | output | 1 | - |
| src_m_axi_ctrlwr_bid | input | IW | - |
| src_m_axi_ctrlwr_bresp | input | 2 | - |

## 4.4	SOURCE HALF (u_src) - direction-unique ports (no prefix)

### 4.4.1	AXI Transfer Configuration (source-only)

Table 1.2.17: AXI Transfer Configuration (source-only)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | - |
| cfg_drain_size | input | 8 | source: beats drained per AXIS packet |

### 4.4.2	Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id

Table 1.2.18: Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tdata | output | DW | - |
| m_axis_tstrb | output | SW | - |
| m_axis_tlast | output | 1 | - |
| m_axis_tid | output | AXIS_ID_WIDTH | - |
| m_axis_tdest | output | AXIS_DEST_WIDTH | - |
| m_axis_tuser | output | AXIS_USER_WIDTH | - |
| m_axis_tvalid | output | 1 | - |
| m_axis_tready | input | 1 | - |

### 4.4.3	AXI4 Master - Data Read (Memory -> Source SRAM)

Table 1.2.19: AXI4 Master - Data Read (Memory -> Source SRAM)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_arid | output | IW | - |
| m_axi_rd_araddr | output | AW | - |
| m_axi_rd_arlen | output | 8 | - |
| m_axi_rd_arsize | output | 3 | - |
| m_axi_rd_arburst | output | 2 | - |
| m_axi_rd_arvalid | output | 1 | - |
| m_axi_rd_arready | input | 1 | - |
| m_axi_rd_rid | input | IW | - |
| m_axi_rd_rdata | input | DW | - |
| m_axi_rd_rresp | input | 2 | - |
| m_axi_rd_rlast | input | 1 | - |
| m_axi_rd_rvalid | input | 1 | - |
| m_axi_rd_rready | output | 1 | - |

## 4.5	SINK HALF (u_snk) - shared-infrastructure ports (snk_ prefixed)

### 4.5.1	APB Programming Interface

Table 1.2.20: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_apb_valid | input | NC | - |
| snk_apb_ready | output | NC | - |
| snk_apb_addr | input | NC x AW | - |

### 4.5.2	Per-channel configuration

Table 1.2.21: Per-channel configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_channel_enable | input | NC | - |
| snk_cfg_channel_reset | input | NC | - |

### 4.5.3	Scheduler Configuration (global)

Table 1.2.22: Scheduler Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_sched_enable | input | 1 | - |
| snk_cfg_sched_timeout_cycles | input | 32 | - |
| snk_cfg_sched_timeout_limit | input | 8 | - |
| snk_cfg_sched_timeout_enable | input | 1 | - |
| snk_cfg_sched_err_enable | input | 1 | - |
| snk_cfg_sched_compl_enable | input | 1 | - |
| snk_cfg_sched_perf_enable | input | 1 | - |

### 4.5.4	Descriptor Engine Configuration (global)

Table 1.2.23: Descriptor Engine Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_desceng_enable | input | 1 | - |
| snk_cfg_desceng_prefetch | input | 1 | - |
| snk_cfg_desceng_fifo_thresh | input | 4 | - |
| snk_cfg_desceng_addr0_base | input | AW | - |
| snk_cfg_desceng_addr0_limit | input | AW | - |
| snk_cfg_desceng_addr1_base | input | AW | - |
| snk_cfg_desceng_addr1_limit | input | AW | - |

### 4.5.5	Control Engine Configuration (Phase 2, global)

Table 1.2.24: Control Engine Configuration (Phase 2, global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_ctrlrd_max_try | input | 9 | - |
| snk_tick_1us | input | 1 | - |

### 4.5.6	Descriptor AXI Monitor Configuration

Table 1.2.25: Descriptor AXI Monitor Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_desc_mon_enable | input | 1 | - |
| snk_cfg_desc_mon_err_enable | input | 1 | - |
| snk_cfg_desc_mon_perf_enable | input | 1 | - |
| snk_cfg_desc_mon_perf_run | input | 1 | - |
| snk_cfg_desc_mon_timeout_enable | input | 1 | - |
| snk_cfg_desc_mon_timeout_cycles | input | 32 | - |
| snk_cfg_desc_mon_latency_thresh | input | 32 | - |
| snk_cfg_desc_mon_pkt_mask | input | 16 | - |
| snk_cfg_desc_mon_err_select | input | 4 | - |
| snk_cfg_desc_mon_err_mask | input | 8 | - |
| snk_cfg_desc_mon_timeout_mask | input | 8 | - |
| snk_cfg_desc_mon_compl_mask | input | 8 | - |
| snk_cfg_desc_mon_thresh_mask | input | 8 | - |
| snk_cfg_desc_mon_perf_mask | input | 8 | - |
| snk_cfg_desc_mon_addr_mask | input | 8 | - |
| snk_cfg_desc_mon_debug_mask | input | 8 | - |

### 4.5.7	AXIS data-path monitor-lite configuration (rapids TASK-015)

Table 1.2.26: AXIS data-path monitor-lite configuration (rapids TASK-015)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_axis_mon_enable | input | 1 | - |
| snk_cfg_axis_mon_err_enable | input | 1 | - |
| snk_cfg_axis_mon_compl_enable | input | 1 | - |
| snk_cfg_axis_mon_perf_enable | input | 1 | - |
| snk_cfg_axis_mon_timeout_enable | input | 1 | - |
| snk_cfg_axis_mon_timeout_cycles | input | 32 | - |
| snk_cfg_axis_mon_latency_thresh | input | 32 | - |
| snk_cfg_axis_mon_pkt_mask | input | 16 | - |

### 4.5.8	Status

Table 1.2.27: Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_system_idle | output | 1 | - |
| snk_descriptor_engine_idle | output | NC | - |
| snk_scheduler_idle | output | NC | - |
| snk_scheduler_state | output | NC x 7 | - |
| snk_sched_error | output | NC | - |

### 4.5.9	Descriptor AXI Monitor Status

Table 1.2.28: Descriptor AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_sts_desc_mon_busy | output | 1 | - |
| snk_cfg_sts_desc_mon_active_txns | output | 8 | - |
| snk_cfg_sts_desc_mon_error_count | output | 16 | - |
| snk_cfg_sts_desc_mon_txn_count | output | 32 | - |
| snk_cfg_sts_desc_mon_conflict_error | output | 1 | - |

### 4.5.10	AXIS data-path monitor-lite status (rapids TASK-015)

Table 1.2.29: AXIS data-path monitor-lite status (rapids TASK-015)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_sts_axis_mon_busy | output | 1 | - |
| snk_cfg_sts_axis_mon_packet_count | output | 32 | - |
| snk_cfg_sts_axis_mon_error_count | output | 16 | - |
| snk_cfg_sts_axis_mon_dropped_count | output | 16 | - |

### 4.5.11	Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).

Table 1.2.30: Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_sts_desc_mon_win_active | output | 1 | - |
| snk_sts_desc_mon_win_cycles | output | 32 | - |
| snk_sts_desc_mon_prod_cycles | output | 32 | - |
| snk_sts_desc_mon_bp_cycles | output | 32 | - |
| snk_sts_desc_mon_starv_cycles | output | 32 | - |
| snk_sts_desc_mon_idle_cycles | output | 32 | - |
| snk_sts_desc_mon_beat_count | output | 32 | - |
| snk_sts_desc_mon_byte_count | output | 64 | - |
| snk_sts_desc_mon_burst_count | output | 32 | - |

### 4.5.12	Descriptor Fetch AXI Master

Table 1.2.31: Descriptor Fetch AXI Master

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_desc_arvalid | output | 1 | - |
| snk_m_axi_desc_arready | input | 1 | - |
| snk_m_axi_desc_araddr | output | AW | - |
| snk_m_axi_desc_arlen | output | 8 | - |
| snk_m_axi_desc_arsize | output | 3 | - |
| snk_m_axi_desc_arburst | output | 2 | - |
| snk_m_axi_desc_arid | output | IW | - |
| snk_m_axi_desc_arlock | output | 1 | - |
| snk_m_axi_desc_arcache | output | 4 | - |
| snk_m_axi_desc_arprot | output | 3 | - |
| snk_m_axi_desc_arqos | output | 4 | - |
| snk_m_axi_desc_arregion | output | 4 | - |
| snk_m_axi_desc_rvalid | input | 1 | - |
| snk_m_axi_desc_rready | output | 1 | - |
| snk_m_axi_desc_rdata | input | 256 | - |
| snk_m_axi_desc_rresp | input | 2 | - |
| snk_m_axi_desc_rlast | input | 1 | - |
| snk_m_axi_desc_rid | input | IW | - |

### 4.5.13	Control Read AXI Master (32-bit) [Phase 2]

Table 1.2.32: Control Read AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_ctrlrd_arvalid | output | 1 | - |
| snk_m_axi_ctrlrd_arready | input | 1 | - |
| snk_m_axi_ctrlrd_araddr | output | AW | - |
| snk_m_axi_ctrlrd_arlen | output | 8 | - |
| snk_m_axi_ctrlrd_arsize | output | 3 | - |
| snk_m_axi_ctrlrd_arburst | output | 2 | - |
| snk_m_axi_ctrlrd_arid | output | IW | - |
| snk_m_axi_ctrlrd_arlock | output | 1 | - |
| snk_m_axi_ctrlrd_arcache | output | 4 | - |
| snk_m_axi_ctrlrd_arprot | output | 3 | - |
| snk_m_axi_ctrlrd_arqos | output | 4 | - |
| snk_m_axi_ctrlrd_arregion | output | 4 | - |
| snk_m_axi_ctrlrd_rvalid | input | 1 | - |
| snk_m_axi_ctrlrd_rready | output | 1 | - |
| snk_m_axi_ctrlrd_rdata | input | 32 | - |
| snk_m_axi_ctrlrd_rresp | input | 2 | - |
| snk_m_axi_ctrlrd_rlast | input | 1 | - |
| snk_m_axi_ctrlrd_rid | input | IW | - |

### 4.5.14	Control Write AXI Master (32-bit) [Phase 2]

Table 1.2.33: Control Write AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_ctrlwr_awvalid | output | 1 | - |
| snk_m_axi_ctrlwr_awready | input | 1 | - |
| snk_m_axi_ctrlwr_awaddr | output | AW | - |
| snk_m_axi_ctrlwr_awlen | output | 8 | - |
| snk_m_axi_ctrlwr_awsize | output | 3 | - |
| snk_m_axi_ctrlwr_awburst | output | 2 | - |
| snk_m_axi_ctrlwr_awid | output | IW | - |
| snk_m_axi_ctrlwr_awlock | output | 1 | - |
| snk_m_axi_ctrlwr_awcache | output | 4 | - |
| snk_m_axi_ctrlwr_awprot | output | 3 | - |
| snk_m_axi_ctrlwr_awqos | output | 4 | - |
| snk_m_axi_ctrlwr_awregion | output | 4 | - |
| snk_m_axi_ctrlwr_wvalid | output | 1 | - |
| snk_m_axi_ctrlwr_wready | input | 1 | - |
| snk_m_axi_ctrlwr_wdata | output | 32 | - |
| snk_m_axi_ctrlwr_wstrb | output | 4 | - |
| snk_m_axi_ctrlwr_wlast | output | 1 | - |
| snk_m_axi_ctrlwr_bvalid | input | 1 | - |
| snk_m_axi_ctrlwr_bready | output | 1 | - |
| snk_m_axi_ctrlwr_bid | input | IW | - |
| snk_m_axi_ctrlwr_bresp | input | 2 | - |

## 4.6	Monitor Bus (SINGLE aggregated stream for the whole core)

The two halves’ monitor outputs are merged through a top-level monbus_arbiter, so the core exposes exactly one monitor stream.

### 4.6.1	Monitor Bus

Table 1.2.34: Monitor Bus

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | - |
| mon_ready | input | 1 | - |
| mon_packet | output | monitor_common_pkg::monitor_packet_t | - |
| mon_timestamp | output | monitor_common_pkg::monbus_timestamp_t | - |

## 4.7	SINK HALF (u_snk) - direction-unique ports (no prefix)

### 4.7.1	AXI Transfer Configuration (sink-only)

Table 1.2.35: AXI Transfer Configuration (sink-only)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_wr_xfer_beats | input | 8 | - |
| cfg_alloc_size | input | 8 | sink: SRAM alloc size per AXIS fill |

### 4.7.2	Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id

Table 1.2.36: Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tdata | input | DW | - |
| s_axis_tstrb | input | SW | - |
| s_axis_tlast | input | 1 | - |
| s_axis_tid | input | AXIS_ID_WIDTH | - |
| s_axis_tdest | input | AXIS_DEST_WIDTH | - |
| s_axis_tuser | input | AXIS_USER_WIDTH | - |
| s_axis_tvalid | input | 1 | - |
| s_axis_tready | output | 1 | - |

### 4.7.3	AXI4 Master - Data Write (Sink SRAM -> Memory)

Table 1.2.37: AXI4 Master - Data Write (Sink SRAM -> Memory)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_awid | output | IW | - |
| m_axi_wr_awaddr | output | AW | - |
| m_axi_wr_awlen | output | 8 | - |
| m_axi_wr_awsize | output | 3 | - |
| m_axi_wr_awburst | output | 2 | - |
| m_axi_wr_awlock | output | 1 | - |
| m_axi_wr_awcache | output | 4 | - |
| m_axi_wr_awprot | output | 3 | - |
| m_axi_wr_awqos | output | 4 | - |
| m_axi_wr_awregion | output | 4 | - |
| m_axi_wr_awvalid | output | 1 | - |
| m_axi_wr_awready | input | 1 | - |
| m_axi_wr_wdata | output | DW | - |
| m_axi_wr_wstrb | output | (DW/8) | - |
| m_axi_wr_wlast | output | 1 | - |
| m_axi_wr_wvalid | output | 1 | - |
| m_axi_wr_wready | input | 1 | - |
| m_axi_wr_bid | input | IW | - |
| m_axi_wr_bresp | input | 2 | - |
| m_axi_wr_bvalid | input | 1 | - |
| m_axi_wr_bready | output | 1 | - |

## 4.8	Debug Interface (per half, src_dbg_ / snk_dbg_ prefixed)

### 4.8.1	Source half debug

Table 1.2.38: Source half debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_dbg_rd_all_complete | output | NC | - |
| src_dbg_r_beats_rcvd | output | 32 | - |
| src_dbg_sram_writes | output | 32 | - |
| src_dbg_arb_request | output | NC | - |
| src_dbg_src_sram_bridge_pending | output | NC | - |
| src_dbg_src_sram_bridge_out_valid | output | NC | - |
| src_dbg_axis_beats_sent | output | 32 | - |
| src_dbg_axis_packets_sent | output | 32 | - |

### 4.8.2	Sink half debug

Table 1.2.39: Sink half debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_dbg_snk_sram_bridge_pending | output | NC | - |
| snk_dbg_snk_sram_bridge_out_valid | output | NC | - |
| snk_dbg_axis_beats_received | output | 32 | - |
| snk_dbg_axis_packets_received | output | 32 | - |

### 4.8.3	Active-channel sideband for per-channel bus instrumentation

(axi_bus_meter). The W bus carries no wid, so the write engine’s channel index must travel out of band to reach the meter at the top.

Table 1.2.40: Active-channel sideband for per-channel bus instrumentation

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_active_channel_id | output | CIW | - |
| snk_active_channel_valid | output | 1 | - |

Verified: every row above is a declared port of rapids_core_beats and every declared port has a row (324 of 324), generated from the module declaration on 2026-09-28. Regenerate rather than hand-edit when the interface changes.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Clocks and Reset

Module: rapids_core_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 5.1	Clock Domain

RAPIDS Beats operates in a single clock domain for simplified design and maximum throughput.

### 5.1.1	Clock Specifications

Table 1.3.1: Clock Specifications

| Parameter | Specification |
| --- | --- |
| Clock Signal | clk (also axi_aclk in some submodules) |
| Frequency Range | 100 - 500 MHz |
| Duty Cycle | 45% - 55% |
| Clock Jitter | < 100 ps peak-to-peak |

### 5.1.2	Clock Distribution

clk (one clock domain; rst_n active-low, async assert)
├── scheduler_group_array_beats (one per half)
│   └── descriptor_engine_beats, scheduler_beats, ctrlrd_engine, ctrlwr_engine (x8)
├── snk_data_path_axis_beats
│   └── snk_sram_controller_beats, axi_write_engine_beats
└── src_data_path_axis_beats
    └── axi_read_engine_beats, src_sram_controller_beats

## 5.2	Reset Specification

RAPIDS uses asynchronous assert, synchronous deassert reset methodology.

### 5.2.1	Reset Signal

Table 1.3.2: Reset Specifications

| Parameter | Specification |
| --- | --- |
| Reset Signal | rst_n (also axi_aresetn in some submodules) |
| Polarity | Active-low |
| Assert | Asynchronous (immediate) |
| Deassert | Synchronous (on rising clock edge) |
| Minimum Duration | 10 clock cycles |

### 5.2.2	Reset Timing Diagram

### 5.2.3	Figure 1.3.1: Reset Timing

scheduler_beats - reset release to first descriptor

Source: scheduler_reset.json, captured from dv/tests/fub_beats/test_scheduler_beats.py (basic transfer, the same run as Figure 2.1.3) with WAVES=1.

Reading it: rst_n is held low for ten cycles and released on a clock edge. Every state register is already at its reset value while reset is asserted – the FSM sits in IDLE, scheduler_idle is high, both control engines report idle and neither engine request is raised. Nothing changes at the release edge itself; the scheduler stays in IDLE until the testbench raises cfg_channel_enable ten cycles later and a descriptor becomes available, at which point it steps through FETCH_DESC to XFER_DATA and raises the read and write requests together.

## 5.3	Reset Behavior

### 5.3.1	On Reset Assert

All modules initialize to known states:

Table 1.3.3: Reset States

| Component | Reset State |
| --- | --- |
| Scheduler FSM | IDLE |
| Descriptor Engine | IDLE, FIFOs empty |
| AXI Read Engine | Idle, no outstanding transactions |
| AXI Write Engine | Idle, no outstanding transactions |
| SRAM Pointers | All zeros |
| Alloc/Drain Controllers | Empty, full space available |
| MonBus | No packets pending |

### 5.3.2	Reset Sequence

1. External reset asserted (rst_n = 0)
   - All flip-flops async reset
   - All outputs driven to safe states
   - No AXI transactions

2. System stabilization (minimum 10 cycles)
   - Clock running and stable
   - Configuration registers programmed (optional)

3. Reset deasserted (rst_n = 1, sync to clk rising edge)
   - FSMs begin operation
   - Ready to accept descriptors/data

## 5.4	Configuration During Reset

Some configuration signals MUST be stable before reset deassertion:

Table 1.3.4: Configuration Timing Requirements

| Signal | Requirement |
| --- | --- |
| src_cfg_channel_enable / snk_cfg_channel_enable | Stable before rst_n=1 (one per half) |
| src_cfg_sched_timeout_cycles / snk_cfg_sched_timeout_cycles | Stable before rst_n=1 (one per half) |
| cfg_axi_rd_xfer_beats | Stable before rst_n=1 |
| cfg_axi_wr_xfer_beats | Stable before rst_n=1 |

## 5.5	Reset Macros

RAPIDS uses standardized reset macros from reset_defs.svh:

`include "reset_defs.svh"

// Standard reset pattern
`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
        r_counter <= '0;
    end else begin
        r_state <= w_next_state;
        r_counter <= w_next_counter;
    end
)

## 5.6	Power-On Reset Considerations

- External reset should be held for minimum 10 clock cycles after clock is stable

- Configuration registers should be programmed before or during reset

- No AXI transactions until reset is deasserted and channels are enabled

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Scheduler

Module: scheduler.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2026-09-30

## 6.1	Overview

The scheduler coordinates one channel. It accepts a 256-bit descriptor from the descriptor engine, turns the descriptor’s byte length and byte addresses into beat counts for the AXI engines, tells the data paths how the packet is laid out, and tracks progress until every source beat is read and every destination beat is committed.

Compared with the RAPIDS Beats scheduler, the state machine, the timeout logic, the control-descriptor path and the MonBus events are the same. Three things change: the beat math, the address advance, and the packet record.

### 6.1.1	Key Features

- Byte length: the descriptor’s 32-bit length is a byte count.

- Byte addresses: the offset of a transfer inside its first beat is addr[OFF_W-1:0].

- Beat math: beats moved are derived from offset and length, in a 33-bit intermediate.

- Packet record: one {bytes, offset} pulse per DATA descriptor, per enabled direction, held back while the data path cannot take it.

- Concurrent read and write: both directions run inside one XFER_DATA state.

- Recoverable write-progress timeout: strikes escalate to a sticky error only when a limit is set.

- Control descriptors: CTRL_READ and CTRL_WRITE drive the control engines, unchanged.

- Directional halves: EN_READ and EN_WRITE select the source or sink half.

### 6.1.2	Block Diagram

### 6.1.3	Figure 2.1.1: Scheduler Byte Math

Scheduler byte math

Source: assets/graphviz/03_scheduler_byte_math.dot

## 6.2	Parameters

Table 2.1.1: Scheduler Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| CHANNEL_ID | int | 0 | Channel index of this scheduler |
| NUM_CHANNELS | int | 8 | Channel count |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | Channel field width |
| ADDR_WIDTH | int | 64 | Address width |
| DATA_WIDTH | int | 512 | Data width in bits; the board design uses 256 |
| MON_AGENT_ID | logic [15:0] | 16’h0040 | MonBus agent ID |
| MON_UNIT_ID | logic [7:0] | 8’h01 | MonBus unit ID |
| MON_CHANNEL_ID | logic [8:0] | 9’h000 | MonBus base channel ID |
| DESC_WIDTH | int | 256 | Descriptor width; fixed at 256 (elaboration fails otherwise) |
| EN_READ | bit | 1 | Enable the read direction for DATA descriptors |
| EN_WRITE | bit | 1 | Enable the write direction for DATA descriptors |
| USE_ROW_COL_MAJOR_ADDRESSING | int | 1 | Enable TYPE=EXT run-base generators |
| BYTE_LANES | int | DATA_WIDTH/8 | Bytes per beat |
| OFF_W | int | (BYTE_LANES>1) ? $clog2(BYTE_LANES) : 1 | Offset field width |

BYTE_LANES and OFF_W are the two parameters that do not exist in the Beats scheduler. A directional half sets exactly one of EN_READ and EN_WRITE: the source half reads only, the sink half writes only.

## 6.3	Port List

### 6.3.1	Clock, Reset and Configuration

Table 2.1.2: Clock, Reset and Configuration Ports

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | in | 1 | Clock |
| rst_n | in | 1 | Active-low reset |
| cfg_channel_enable | in | 1 | Channel enable; IDLE leaves only when set |
| cfg_channel_reset | in | 1 | Channel reset; forces IDLE and clears sticky state |
| cfg_sched_timeout_cycles | in | 32 | Write-progress timeout window in cycles |
| cfg_sched_timeout_limit | in | 8 | Consecutive windows before escalation; 0 = never |
| cfg_sched_timeout_enable | in | 1 | Enable timeout detection |

### 6.3.2	Status

Table 2.1.3: Status Ports

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| scheduler_idle | out | 1 | High in CH_IDLE with no channel reset pending |
| scheduler_state | out | 7 | One-hot state (see Table 2.1.12) |
| sched_error | out | 1 | Sticky, high in CH_ERROR |
| dbg_descriptor_error | out | 1 | Latched fatal descriptor-level error |
| dbg_read_error_sticky | out | 1 | Latched read-engine error |
| dbg_write_error_sticky | out | 1 | Latched write-engine error |
| dbg_timeout_expired | out | 1 | Live: the timeout window has elapsed |

### 6.3.3	Descriptor Engine Interface

Table 2.1.4: Descriptor Engine Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | in | 1 | A descriptor is available |
| descriptor_ready | out | 1 | High in CH_IDLE and CH_NEXT_DESC |
| descriptor_packet | in | 256 | Descriptor (layout in Table 2.1.10) |
| descriptor_ext_packet | in | 256 | Extended chunk; used only when TYPE=EXT |
| descriptor_error | in | 1 | Descriptor fetch or parse error |

### 6.3.4	Data Read Interface (to AXI Read Engine)

Table 2.1.5: Data Read Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | out | 1 | Read request |
| sched_rd_addr | out | ADDR_WIDTH | Source byte address (working register) |
| sched_rd_beats | out | 32 | Beats remaining to read |
| sched_rd_pkt_valid | out | 1 | Packet record pulse to the source egress |
| sched_rd_pkt_ready | in | 1 | Source egress record queue has room |
| sched_rd_pkt_bytes | out | 32 | Descriptor length in bytes |
| sched_rd_pkt_offset | out | OFF_W | Source byte offset inside the first beat |

### 6.3.5	Data Write Interface (to AXI Write Engine)

Table 2.1.6: Data Write Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | out | 1 | Write request |
| sched_wr_ready | in | 1 | Write engine can take the channel |
| sched_wr_addr | out | ADDR_WIDTH | Destination byte address (working register) |
| sched_wr_beats | out | 32 | Beats remaining to issue |
| sched_wr_pkt_valid | out | 1 | Packet record pulse to the sink ingress |
| sched_wr_pkt_ready | in | 1 | Sink ingress record queue has room |
| sched_wr_pkt_bytes | out | 32 | Descriptor length in bytes |
| sched_wr_pkt_offset | out | OFF_W | Destination byte offset inside the first beat |

### 6.3.6	Completion Interface

Table 2.1.7: Completion and Error Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| sched_rd_done_strobe | in | 1 | A read burst completed |
| sched_rd_beats_done | in | 32 | Beats in that burst |
| sched_wr_done_strobe | in | 1 | A write burst was issued (AW handshake) |
| sched_wr_beats_done | in | 32 | Beats in that burst |
| sched_wr_commit_strobe | in | 1 | A write burst was committed (B response) |
| sched_wr_commit_beats | in | 32 | Beats in that burst |
| sched_rd_error | in | 1 | Read engine error |
| sched_wr_error | in | 1 | Write engine error, including the sink packet length error |

### 6.3.7	Control Engine Interface

Table 2.1.8: Control Engine Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| ctrlrd_valid | out | 1 | CTRL_READ request |
| ctrlrd_ready | in | 1 | Engine accepted the request |
| ctrlrd_addr | out | ADDR_WIDTH | Poll address (descriptor source slot) |
| ctrlrd_data | out | 32 | Expected value (dst_addr[31:0]) |
| ctrlrd_mask | out | 32 | Compare mask (dst_addr[63:32]) |
| ctrlrd_error | in | 1 | Engine error (retries exhausted or AXI error) |
| ctrlrd_idle | in | 1 | Engine idle |
| ctrlwr_valid | out | 1 | CTRL_WRITE request |
| ctrlwr_ready | in | 1 | Engine accepted the request |
| ctrlwr_addr | out | ADDR_WIDTH | Doorbell address (descriptor source slot) |
| ctrlwr_data | out | 32 | Doorbell data (dst_addr[31:0]) |
| ctrlwr_error | in | 1 | Engine error |
| ctrlwr_idle | in | 1 | Engine idle |

### 6.3.8	Monitor Bus Interface

Table 2.1.9: Monitor Bus Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | in | timestamp | Time base for packets |
| mon_valid | out | 1 | Packet valid |
| mon_ready | in | 1 | Consumer ready |
| mon_packet | out | 128 | Monitor packet |
| mon_timestamp | out | 64 | Side-band timestamp |

The control interface tables do not change from RAPIDS Beats. The packet-record ports in Tables 2.1.5 and 2.1.6 are the new ports.

## 6.4	Descriptor Format

Table 2.1.10: Descriptor Layout

| Bits | Field | Notes |
| --- | --- | --- |
| [63:0] | src_addr | Byte address |
| [127:64] | dst_addr | Byte address |
| [159:128] | length | Bytes |
| [191:160] | next_descriptor_ptr | Chain pointer |
| [192] | valid | Must be 1 |
| [193] | gen_irq | Emit an IRQ event on completion |
| [194] | last | End of chain |
| [195] | error | Error flag |
| [199:196] | channel_id |  |
| [207:200] | priority |  |
| [209:208] | opcode | DATA = 0, CTRL_READ = 1, CTRL_WRITE = 2 |
| [212:210] | type | LEGACY = 0, EXT = 1 |

For CTRL_READ and CTRL_WRITE the address and data slots are reinterpreted as in Tables 2.1.8; the length field does not describe a byte count and no packet record is issued.

## 6.5	Byte-to-Beat Math

For each enabled direction the scheduler forms the offset from the low OFF_W bits of that side’s address and derives the number of memory beats the transfer touches.

w_rd_off        = src_addr[OFF_W-1:0]
w_wr_off        = dst_addr[OFF_W-1:0]
w_rd_span       = 33'(w_rd_off) + 33'(length) + 33'(BYTE_LANES-1)
w_wr_span       = 33'(w_wr_off) + 33'(length) + 33'(BYTE_LANES-1)
w_rd_beats_total = (length == 0) ? 0 : 32'(w_rd_span >> OFF_W)
w_wr_beats_total = (length == 0) ? 0 : 32'(w_wr_span >> OFF_W)

The span is 33 bits wide so a length near 4 GiB does not overflow. The read and write beat totals are computed separately because the two addresses can have different offsets; a 60-byte transfer read from offset 5 and written at offset 0 touches three source beats and two destination beats on a 32-byte design.

Table 2.1.11: Beat Totals for a 32-byte Beat

| Length (bytes) | Offset | Beats |
| --- | --- | --- |
| 0 | any | 0 |
| 1 | 0 | 1 |
| 32 | 0 | 1 |
| 32 | 1 | 2 |
| 60 | 5 | 3 |
| 60 | 0 | 2 |
| 64 | 0 | 2 |

The values load in CH_FETCH_DESC:

- r_beats_remaining loads EN_READ ? rd_total : wr_total.

- r_read_beats_remaining loads EN_READ ? rd_total : 0.

- r_write_beats_remaining and r_write_beats_to_commit load EN_WRITE ? wr_total : 0.

A disabled direction therefore loads zero, so its valid never asserts and its completion term is true from the start. Completion collapses onto the enabled direction.

For TYPE=EXT the run counters load min(run_size, beats_total) instead of the whole total, and stream_run_addr_gen.cfg_total_beats takes the beats total. TYPE=EXT is beat-aligned by design: addresses are aligned and lengths are beat multiples, so the offset is zero and the math reduces to the Beats result.

## 6.6	Address Advance

The working address registers start at the descriptor’s byte addresses. On each done strobe the address moves forward by the beats reported:

r_src_addr <= {r_src_addr[AW-1:OFF_W], {OFF_W{1'b0}}} + (AW'(sched_rd_beats_done) << OFF_W)   // on sched_rd_done_strobe
r_dst_addr <= {r_dst_addr[AW-1:OFF_W], {OFF_W{1'b0}}} + (AW'(sched_wr_beats_done) << OFF_W)   // on sched_wr_done_strobe

The low bits are cleared before the add. Only the first burst of a descriptor starts inside a beat; every later burst starts on a beat boundary, so aligning down first is correct for both. The write address advances on the AW handshake (issue), not on the B response. The B response decrements r_write_beats_to_commit instead.

The scheduler reports the working address to the engines unmodified. The engines align it (see AXI Read Engine); the offset reaches the data path through the packet record, not through the address bus.

## 6.7	Packet Records

A DATA descriptor with a non-zero length causes one record pulse per enabled direction, on the cycle the scheduler leaves CH_FETCH_DESC:

w_pkt_ok = !w_is_data || length == 0 ||
           ((!EN_READ  || sched_rd_pkt_ready) &&
            (!EN_WRITE || sched_wr_pkt_ready))

sched_{rd,wr}_pkt_valid = w_state_fetch_desc && r_descriptor.valid && w_is_data &&
                          EN_{READ,WRITE} && w_pkt_ok && (length != 0)
sched_{rd,wr}_pkt_bytes  = r_descriptor.length
sched_{rd,wr}_pkt_offset = w_{rd,wr}_off

Both directions share w_pkt_ok, so each enabled direction sees exactly one pulse. w_pkt_ok also gates the FETCH_DESC to XFER_DATA transition: while a record queue is full, the scheduler holds in CH_FETCH_DESC and the pulse does not fire. Control descriptors and zero-length transfers carry no record. See Packet Record Interface.

## 6.8	Finite State Machine

The state is one-hot and is rapids_pkg::channel_state_t.

Table 2.1.12: State Encoding

| State | Encoding | Description |
| --- | --- | --- |
| CH_IDLE | 7’b0000001 | Waiting for a descriptor and cfg_channel_enable |
| CH_FETCH_DESC | 7’b0000010 | Descriptor latched; working registers and counters load; record pulses |
| CH_XFER_DATA | 7’b0000100 | Read and write engines run concurrently; control ops issue |
| CH_COMPLETE | 7’b0001000 | Descriptor done; chain decision |
| CH_NEXT_DESC | 7’b0010000 | Waiting for the chained descriptor |
| CH_ERROR | 7’b0100000 | Sticky; only channel reset leaves it |
| CH_RESERVED | 7’b1000000 | Unused |

Transition priority is: registered channel reset to IDLE, then a hard error or timeout escalation to ERROR, then the normal case.

Table 2.1.13: Transitions

| From | To | Condition |
| --- | --- | --- |
| IDLE | FETCH_DESC | descriptor_valid && cfg_channel_enable |
| FETCH_DESC | XFER_DATA | r_descriptor.valid && w_pkt_ok |
| FETCH_DESC | FETCH_DESC | r_descriptor.valid and w_pkt_ok false (record queue full) |
| FETCH_DESC | ERROR | descriptor valid bit clear |
| XFER_DATA | COMPLETE | w_exec_complete |
| COMPLETE | NEXT_DESC | next_descriptor_ptr != 0 && !last |
| COMPLETE | IDLE | otherwise |
| NEXT_DESC | FETCH_DESC | descriptor_valid |
| ERROR | ERROR | always; channel reset is the only exit |
| any other | ERROR | default |

w_exec_complete is w_transfer_complete for a DATA descriptor and w_ctrl_complete for a control descriptor. w_transfer_complete is the read remaining count reaching zero together with the commit remaining count reaching zero. w_ctrl_complete is the control request having been accepted and the control engine having returned to idle.

scheduler_idle is CH_IDLE && !r_channel_reset_active. sched_error is the ERROR state.

## 6.9	Operation

### 6.9.1	Data Request Outputs

sched_rd_valid is high in XFER_DATA for a DATA descriptor while !w_read_complete, !w_sched_rd_completing_this_cycle and, for TYPE=EXT, !w_rd_need_base. sched_rd_beats is the read remaining count, or for EXT the current run’s remainder. sched_wr_valid is high in XFER_DATA for a DATA descriptor while r_write_beats_remaining != 0, the commit count has not reached zero, !w_sched_wr_completing_this_cycle and !w_wr_need_base.

The look-ahead terms drop the valid on the same cycle the last done strobe arrives. Without them the engine would see a still-asserted valid for one more cycle and issue an extra burst.

### 6.9.2	Write Completion

The write direction has two counters. r_write_beats_remaining decrements on the AW done strobe and drives burst sizing and the destination address. r_write_beats_to_commit decrements on the B commit strobe and gates w_write_complete. The channel therefore completes only after every issued burst has been acknowledged.

### 6.9.3	Control Descriptors

CTRL_READ and CTRL_WRITE do not use the data engines. ctrlrd_valid and ctrlwr_valid assert in XFER_DATA until the engine accepts (r_ctrl_issued), and completion is the engine returning to idle after acceptance.

## 6.10	Error Handling

w_hard_error is the OR of:

- descriptor_error, sched_rd_error, sched_wr_error

- the sticky read and write error flags

- ctrlrd_error while the descriptor is CTRL_READ

- ctrlwr_error while the descriptor is CTRL_WRITE

A hard error, or a timeout escalation, moves the FSM to CH_ERROR. The sink packet length check reports through sched_wr_error (see Sink Data Path AXIS), so a stream packet whose byte count differs from the descriptor length is a hard error for that channel. Sticky read and write flags and the descriptor error clear in CH_IDLE and also on the registered channel reset, in every state. Clearing on the reset matters: the data engines and the data paths clear their own error levels on the same channel reset, so the scheduler no longer re-enters CH_ERROR from a stale level when it reaches CH_IDLE. A channel reset therefore recovers a data read or write response error and a sink packet length error, and the channel accepts a good descriptor afterwards. The reset also clears the descriptor-loaded flag, the remaining beat counts and the control issued flag, so nothing of the failed descriptor carries into the next one.

## 6.11	Recoverable Write-Progress Timeout

The timeout counter increments while sched_wr_valid && !sched_wr_ready. It resets on sched_wr_done_strobe or sched_wr_commit_strobe, and also when the condition is false. When the counter reaches cfg_sched_timeout_cycles with cfg_sched_timeout_enable set, w_timeout_expired pulses and the counter re-arms.

Each expiry adds one strike, in an 8-bit counter that saturates. Strikes clear on write progress, in CH_IDLE, and on channel reset. Escalation is:

w_timeout_escalate = (cfg_sched_timeout_limit != 0) && (strikes >= cfg_sched_timeout_limit)

A limit of zero never escalates: the timeout is then purely informational.

## 6.12	MonBus Events

Packets are PktTypeCompletion from PROTOCOL_CORE unless noted.

Table 2.1.14: MonBus Events

| State | Event | Data |
| --- | --- | --- |
| CH_FETCH_DESC | RAPIDS_EVENT_DESC_START | descriptor length (bytes) |
| CH_COMPLETE | RAPIDS_EVENT_IRQ when gen_irq, else RAPIDS_EVENT_DESC_COMPLETE |  |
| CH_ERROR | RAPIDS_EVENT_ERROR (PktTypeError), once per error episode | {29'h0, wr_sticky, rd_sticky, 33'h0} |

There are no events during XFER_DATA. The only difference from RAPIDS Beats is that the DESC_START data field is a byte count.

## 6.13	Waveforms

#### 6.13.0.1	Waveform 2.1: Byte Descriptor Start

The beat-count arithmetic and the record pulse are combinational functions of the latched descriptor, so the start of a descriptor is short:

cycle                    : 0     1        2        3
state                    : IDLE  FETCH    XFER     XFER
descriptor_valid         : 1     0        0        0
sched_wr_pkt_ready       : 1     1        1        1
sched_wr_pkt_valid       : 0     1        0        0     (one pulse, the last FETCH cycle)
r_write_beats_remaining  : 0     0        N        N     (loads at the end of FETCH)
sched_wr_valid           : 0     0        1        1

If sched_wr_pkt_ready is low in FETCH, the state holds and the pulse is delayed until it rises.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Descriptor Engine Specification

Module: descriptor_engine_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 7.1	Overview

The Descriptor Engine fetches and manages 256-bit descriptors from memory via AXI4. It maintains a prefetch FIFO to hide memory latency and provides parsed descriptors to the scheduler.

### 7.1.1	Key Features

- AXI4 Descriptor Fetch: Single-beat 256-bit reads

- Prefetch FIFO: Configurable depth for latency hiding

- Address Range Checking: Validates descriptor addresses

- Error Detection: Address violations, AXI errors

- MonBus Integration: Fetch events and error reporting

### 7.1.2	Block Diagram

### 7.1.3	Figure 2.2.1: Descriptor Engine Block Diagram

Descriptor engine block diagram

Source: 02_descriptor_engine.dot

## 7.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int FIFO_DEPTH = 4;                    // Prefetch FIFO depth

// Monitor Bus Parameters
parameter logic [7:0] MON_AGENT_ID = 8'h10;      // Descriptor Engine Agent ID
parameter logic [3:0] MON_UNIT_ID = 4'h1;        // Unit identifier

: Table 2.2.1: Descriptor Engine Parameters

## 7.3	Port List

### 7.3.1	Clock and Reset

Table 2.2.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 7.3.2	APB Programming Interface

Table 2.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Kick-off valid (start descriptor chain) |
| apb_ready | output | 1 | Ready to accept kick-off |
| apb_addr | input | AW | First descriptor address |

### 7.3.3	Configuration Interface

Table 2.2.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_prefetch_enable | input | 1 | Enable prefetch chaining (0 = on-demand, ~1 ahead) |
| cfg_fifo_threshold | input | 4 | Prefetch depth (descriptors buffered ahead when enabled) |
| cfg_addr0_base | input | AW | Valid address range 0 base |
| cfg_addr0_limit | input | AW | Valid address range 0 limit |
| cfg_addr1_base | input | AW | Valid address range 1 base |
| cfg_addr1_limit | input | AW | Valid address range 1 limit |

### 7.3.4	AXI4 Read Master Interface

Table 2.2.5: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ar_valid | output | 1 | Address valid |
| ar_ready | input | 1 | Address ready |
| ar_addr | output | ADDR_WIDTH | Descriptor fetch address |
| ar_len | output | 8 | Burst length - 1 |
| ar_size | output | 3 | Burst size (log2 bytes) |
| ar_burst | output | 2 | Burst type (INCR) |
| ar_id | output | AXI_ID_WIDTH | Transaction ID |
| ar_lock | output | 1 | Lock type |
| ar_cache | output | 4 | Cache attributes |
| ar_prot | output | 3 | Protection attributes |
| ar_qos | output | 4 | Quality of service |
| ar_region | output | 4 | Region identifier |
| r_valid | input | 1 | Data valid |
| r_ready | output | 1 | Data ready |
| r_data | input | 256 | Descriptor payload (fixed 256-bit) |
| r_resp | input | 2 | Read response |
| r_last | input | 1 | Last beat |
| r_id | input | AXI_ID_WIDTH | Response ID |

### 7.3.5	Scheduler Interface

Table 2.2.6: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | output | 1 | Descriptor valid |
| descriptor_ready | input | 1 | Scheduler ready |
| descriptor_packet | output | 256 | Parsed descriptor (fixed 256-bit) |
| descriptor_ext_packet | output | 256 | Extended chunk 1 (0 when unused) |
| descriptor_error | output | 1 | Error flag |
| descriptor_eos | output | 1 | End of Stream |
| descriptor_eol | output | 1 | End of Line |
| descriptor_eod | output | 1 | End of Data |
| descriptor_type | output | 2 | Packet type |
| channel_idle | input | 1 | Scheduler idle (enables APB kick-off) |
| descriptor_engine_idle | output | 1 | Engine idle |

## 7.4	FSM States

### 7.4.1	Figure 2.2.2: Descriptor Engine FSM

States and transitions of the descriptor_engine_beats fetch FSM.

Diagram 1

## 7.5	Operation

### 7.5.1	Descriptor Chain Processing

### 7.5.2	Figure 2.2.3: Descriptor Chain Timing

Descriptor Engine - three-descriptor chain

Source: descriptor_engine_chain.json, captured from dv/tests/fub_beats/test_descriptor_engine_beats.py (descriptor_chaining, 32 channels, seed 7) with WAVES=1.

Reading it: one apb_valid kick carries the head address (0x10000). The engine issues an AXI read for it (ar_valid, ar_addr), the data returns on r_valid, and the parsed descriptor is presented on descriptor_valid to a ready scheduler. Because the descriptor’s next_ptr points at 0x10040 and last is clear, the engine fetches the next one on its own – and the one after that at 0x10080 – with no further APB traffic; descriptor_eol marks the last of the chain. Each fetch costs the same AR -> R -> present latency, so a chain streams at one descriptor per fetch round trip.

## 7.6	Address Range Checking

Descriptors must fall within configured valid address ranges:

Valid if: (addr >= cfg_addr0_base && addr <= cfg_addr0_limit)
       || (addr >= cfg_addr1_base && addr <= cfg_addr1_limit)

Invalid addresses generate an error and halt the descriptor chain.

## 7.7	Prefetch and Autonomous Chaining

cfg_prefetch_enable and cfg_fifo_threshold are functional prefetch controls (previously declared-but-unused, which left chaining permanently greedy). They throttle how far ahead the engine fetches chained descriptors by comparing the output descriptor FIFO occupancy (w_desc_fifo_count) against a computed w_prefetch_limit:

if      (!cfg_prefetch_enable)      w_prefetch_limit = 1;   // on-demand
else if (cfg_fifo_threshold == 0)   w_prefetch_limit = 1;   // guard (0 would stall)
else                                w_prefetch_limit = cfg_fifo_threshold;

w_prefetch_allows = (w_desc_fifo_count < w_prefetch_limit);

- Prefetch OFF (cfg_prefetch_enable = 0): limit forced to 1, so a chained fetch is only allowed when the output FIFO has drained – the engine stays roughly one descriptor ahead of the scheduler (on-demand).

- Prefetch ON: the engine buffers up to cfg_fifo_threshold descriptors ahead.

### 7.7.1	Chaining Decision

A chain fetch issues immediately when all of the following hold:

w_should_chain = w_chain_eligible     // valid non-last descriptor, in range, no error
              && w_desc_committed      // current descriptor accepted into output FIFO
              && w_prefetch_allows     // occupancy below prefetch limit
              && w_desc_addr_fifo_wr_ready;  // address FIFO has a slot

### 7.7.2	Deferred (Throttle-Blocked) Chain Fetch

If a descriptor commits and is chain-eligible but the fetch cannot issue this cycle (throttled by w_prefetch_allows, or the address FIFO is full), the owed fetch is recorded rather than dropped:

- r_chain_pending is set and the next-descriptor pointer is held in r_pending_chain_addr.

- The deferred fetch is reissued (w_pending_push_fire) once the output FIFO drains enough that w_prefetch_allows is true again and the address FIFO has a slot, clearing r_chain_pending.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	AXI Read Engine

Module: axi_read_engine.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2026-09-30

## 8.1	Overview

The AXI read engine turns per-channel read requests from the schedulers into AXI4 read bursts and routes the returning data to the SRAM controller. It is a streaming pipeline with no state machine. The channel identity travels in the AXI ID.

The engine is beat based, as in RAPIDS Beats. The scheduler has already converted the descriptor’s byte length into a beat count. Two things change for byte-granular RAPIDS: the engine must cope with a channel address that is not beat aligned, and it must never let a burst cross a 4 KB boundary.

### 8.1.1	Key Features

- Streaming pipeline: no FSM, round-robin arbitration across channels.

- Aligned AR address: ARADDR is the channel address rounded down to a beat boundary. The lanes below the offset are read and ignored by the data path.

- 4 KB burst cap: the burst length is limited so that no burst crosses a 4 KB boundary, whatever the start address.

- Configurable burst length: cfg_axi_rd_xfer_beats, clamped to what the buffer can hold (rapids BUG-009).

- Space-checked: a channel is only granted when the SRAM controller has room for the whole burst.

- Sticky error: SLVERR or DECERR on R sets sched_rd_error for the channel.

### 8.1.2	Block Diagram

### 8.1.3	Figure 2.3.1: AXI Read Engine Block Diagram

AXI read engine block diagram

Source: 05_axi_read_engine.dot

## 8.2	Parameters

Table 2.3.1: AXI Read Engine Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Number of channels. Channel id is carried in the AXI ID. |
| ADDR_WIDTH | 64 | AXI address width. |
| DATA_WIDTH | 512 | AXI data width in bits. The Genesys 2 build uses 256. |
| ID_WIDTH | 8 | AXI ID width. |
| SEG_COUNT_WIDTH | 8 | Width of the per-channel free-space count from the SRAM controller. Sets the burst clamp. |
| PIPELINE | 1 | 1 allows the next AR before the previous burst’s data has returned. 0 waits for the burst to complete. |
| AR_MAX_OUTSTANDING | 8 | Maximum outstanding AR transactions. |
| STROBE_EVERY_BEAT | 0 | Declared for compatibility with STREAM. The completion strobe in this engine is generated on the AR handshake regardless of its value. |

The module also accepts the short aliases NC, AW, DW, IW, SCW and CIW (the channel id width, $clog2(NUM_CHANNELS)).

### 8.2.1	Derived Constants

Table 2.3.2: AXI Read Engine Derived Constants

| Constant | Definition | Meaning |
| --- | --- | --- |
| BYTES_PER_BEAT | DW / 8 | Bytes per beat. |
| AXSIZE | $clog2(BYTES_PER_BEAT) | Value driven on ARSIZE. Full-width beats. |
| SD_BEATS | 1 << (SCW - 1) | Buffer depth in beats implied by SEG_COUNT_WIDTH. |
| XFER_MAX | SD_BEATS < 256 ? SD_BEATS - 1 : 254 | Largest burst length value the engine will use. |

## 8.3	Port List

### 8.3.1	Clock, Reset and Configuration

Table 2.3.3: Clock, Reset and Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | Clock. |
| rst_n | input | 1 | Active-low reset. |
| cfg_axi_rd_xfer_beats | input | 8 | Configured burst length (ARLEN value, 0 to 255). Clamped to XFER_MAX. |
| cfg_channel_reset | input | NC | Per-channel reset, level or pulse. Stops the channel’s ARs, drains and discards its in-flight R data, and clears its error flag. |

### 8.3.2	Scheduler Interface

Table 2.3.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | NC | Channel has a read request. |
| sched_rd_addr | input | NC x AW | Working source byte address. |
| sched_rd_beats | input | NC x 32 | Beats still to read for the channel. |
| sched_rd_done_strobe | output | NC | Registered pulse, one cycle after the AR handshake for the channel. |
| sched_rd_beats_done | output | NC x 32 | Beats issued by that AR (ARLEN + 1). |
| sched_rd_error | output | NC | Sticky read error for the channel, set by any non-OKAY R response. |

The address is a byte address and may carry an offset in its low bits. The scheduler advances it to the next beat boundary after each done strobe, so only the first burst of a descriptor can see a non-zero offset.

### 8.3.3	SRAM Allocation and Fill Interface

Table 2.3.5: SRAM Allocation and Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_alloc_req | output | 1 | Registered pulse, one cycle after the AR handshake, reserving space for the burst. |
| axi_rd_alloc_size | output | 8 | Beats to reserve. |
| axi_rd_alloc_id | output | IW | Channel that owns the reservation. |
| axi_rd_alloc_space_free | input | NC x SCW | Free space per channel. A channel is granted only if it fits the burst. |
| axi_rd_sram_valid | output | 1 | Follows m_axi_rvalid. |
| axi_rd_sram_ready | input | 1 | SRAM controller accepts the beat. Drives m_axi_rready. |
| axi_rd_sram_id | output | IW | Follows m_axi_rid. |
| axi_rd_sram_data | output | DW | Follows m_axi_rdata. |

### 8.3.4	AXI4 Read Master Interface

Table 2.3.6: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arid | output | IW | Read address ID (the channel). |
| m_axi_araddr | output | AW | Beat-aligned read address. |
| m_axi_arlen | output | 8 | Burst length minus one. |
| m_axi_arsize | output | 3 | AXSIZE, full-width beats. |
| m_axi_arburst | output | 2 | INCR. |
| m_axi_arvalid | output | 1 | Address valid. |
| m_axi_arready | input | 1 | Address ready. |
| m_axi_rid | input | IW | Response ID. |
| m_axi_rdata | input | DW | Read data. |
| m_axi_rresp | input | 2 | Read response. |
| m_axi_rlast | input | 1 | Last beat of the burst. |
| m_axi_rvalid | input | 1 | Data valid. |
| m_axi_rready | output | 1 | Data ready. |

### 8.3.5	Debug

Table 2.3.7: Debug Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_rd_all_complete | output | NC | Channel has no outstanding reads. |
| dbg_r_beats_rcvd | output | 32 | Total R beats received. |
| dbg_sram_writes | output | 32 | Total beats written to the SRAM controller. |
| dbg_arb_request | output | NC | Arbiter request vector. |

## 8.4	Operation

### 8.4.1	Burst Length Cap (rapids BUG-009)

cfg_axi_rd_xfer_beats is an ARLEN value, but a burst must fit in the buffer that stages it. The engine clamps the configured value to XFER_MAX, giving the working limit w_xfer_cfg. Without the clamp an ARLEN of 255 wraps the 8-bit ARLEN + 1 and a channel is granted with no space reserved. This is the same fix as in RAPIDS Beats.

### 8.4.2	Byte-Granular Burst Sizing

The engine sizes each burst from the channel’s current byte address and remaining beats. Three steps produce ARLEN:

- Distance to the next 4 KB boundary, in beats: w_beats_to_4k = (4096 - {sched_rd_addr[11:AXSIZE], AXSIZE'b0}) >> AXSIZE. The offset bits are dropped first, so the distance is measured from the aligned beat.

- Burst cap: w_cap_beats = min(w_xfer_cfg + 1, w_beats_to_4k).

- Burst length: w_transfer_size = (sched_rd_beats <= w_cap_beats) ? sched_rd_beats - 1 : w_cap_beats - 1.

m_axi_araddr is {sched_rd_addr[grant][AW-1:AXSIZE], AXSIZE'b0}. The offset never reaches the bus. The first beat of the burst carries the bytes before the offset, and the data path discards them.

Because the cap always rounds toward the boundary, a burst can be shorter than the configured length, and a later burst restarts on the boundary. The scheduler needs no knowledge of this. It only sees beats_done per burst.

### 8.4.3	Streaming Pipeline

An arbiter picks a channel that has sched_rd_valid set and enough axi_rd_alloc_space_free for its burst. When the AR handshakes, the engine pulses axi_rd_alloc_req one cycle later with the burst size so the SRAM controller reserves the space, and pulses sched_rd_done_strobe for the channel. R beats pass straight through: axi_rd_sram_valid, axi_rd_sram_id and axi_rd_sram_data follow the R channel, and m_axi_rready is axi_rd_sram_ready.

With PIPELINE = 1 the next AR for a channel can issue while earlier data is still in flight. With PIPELINE = 0 the channel waits for the burst to finish. The top and core default to 1.

### 8.4.4	Figure 2.3.2: Burst Segmentation with a Mid-Beat Start

The example uses 32-byte beats (OFF_W 5) and a configured burst of 16 beats. A descriptor reads 8,200 bytes from address 0xFF8. The scheduler computes offset 0x18 (24) and beats_total = (24 + 8200 + 31) >> 5 = 257.

Table 2.3.8: Burst Segmentation Example

| Burst | Channel address | ARADDR | Beats to 4 KB | ARLEN | Beats left after |
| --- | --- | --- | --- | --- | --- |
| 1 | 0x0FF8 | 0x0FE0 | 1 | 0 | 256 |
| 2 | 0x1000 | 0x1000 | 128 | 15 | 240 |
| 3 | 0x1200 | 0x1200 | 112 | 15 | 224 |
| … |  |  |  | 15 | continues |

The first burst is one beat because the aligned start is 32 bytes below the 4 KB boundary. From then on each burst is the configured 16 beats until the remainder runs out. The scheduler advances its address to the next beat boundary after each done strobe, which is why burst 2 starts at 0x1000, not 0x0FF8 plus a beat.

### 8.4.5	Done Strobe

The engine pulses sched_rd_done_strobe[ch] once per AR handshake, with sched_rd_beats_done[ch] equal to ARLEN + 1. The scheduler subtracts this from its remaining count and advances the address. The strobe therefore reports beats issued, not beats returned.

## 8.5	Error Handling

Table 2.3.9: Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | m_axi_rresp == 2'b10 on an accepted beat | Set sticky sched_rd_error for the channel |
| AXI DECERR | m_axi_rresp == 2'b11 on an accepted beat | Set sticky sched_rd_error for the channel |
| Any other non-zero response | m_axi_rresp != 2'b00 on an accepted beat | Same: the test is not-OKAY, and the channel is taken from RID |

The scheduler turns the sticky error into its ERROR state and a MonBus event. The engine does not abort the burst: the remaining R beats still pass through, so the interconnect is not left mid-burst.

### 8.5.1	Channel Reset

cfg_channel_reset[ch] clears the sticky sched_rd_error[ch], so a channel reset recovers a read error without aresetn. The reset also has to settle AR bursts that are already on the bus, because an AXI master cannot recall them:

- A channel in reset, or flushing, issues no new AR and does not enter the allocation pipeline.

- r_rd_flush[ch] is set by the reset and holds until the channel has no burst outstanding. While it holds, each R beat whose RID names the channel is accepted (m_axi_rready is forced high) and dropped: axi_rd_sram_valid stays low, so nothing reaches the SRAM, and its RRESP does not set the error flag.

- The completion strobe and pending allocation of the channel are cleared, so the scheduler does not see a stale done pulse after the reset.

- Other channels are not affected: every term is indexed by channel, and R beats of other channels pass as usual.

The channel’s SRAM partition is cleared by the SRAM controller wrapper (see Sink Data Path and Source Data Path AXIS). The discard is what guarantees that no beat of the reset channel reaches the partition after it has been cleared.

## 8.6	Differences from RAPIDS Beats

Table 2.3.10: Read Engine Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| ARADDR | Channel address as given (always aligned) | Aligned down to the beat |
| Burst cap | Configured length and remaining beats | Also capped at the 4 KB boundary |
| Everything else |  | Unchanged |

The beat-count chapter for the beats engine is AXI Read Engine (RAPIDS Beats). The scheduler that feeds this engine is described in Scheduler.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	AXI Write Engine

Module: axi_write_engine.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2026-09-30

## 9.1	Overview

The AXI write engine drains data from the SRAM controller and issues AXI4 write bursts to system memory. It is a streaming pipeline with no state machine. It also tracks the write responses, which is how the scheduler learns that data has been committed.

Byte-granular RAPIDS changes the engine in three places. Write strobes now come from the SRAM instead of being all ones, so a partial first or last beat writes only the bytes that belong to the packet. Burst lengths are capped at the 4 KB boundary. AWADDR is beat aligned.

### 9.1.1	Key Features

- Streaming pipeline: no FSM, round-robin arbitration across channels.

- Real write strobes: m_axi_wstrb is the strobe word stored beside each data beat (axi_wr_sram_strb). Lanes outside the packet are not written.

- Aligned AW address: AWADDR is the channel address rounded down to a beat boundary.

- 4 KB burst cap: the same cap as the read engine, applied to both the burst length and the final-burst test.

- Configurable burst length: cfg_axi_wr_xfer_beats, clamped to what the buffer can hold (rapids BUG-009).

- Two completion strobes: done on the AW handshake, commit on the B response.

- Sticky error: a non-OKAY B response sets sched_wr_error.

### 9.1.2	Block Diagram

### 9.1.3	Figure 2.4.1: AXI Write Engine Block Diagram

AXI write engine block diagram

Source: 06_axi_write_engine.dot

## 9.2	Parameters

Table 2.4.1: AXI Write Engine Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Number of channels. Channel id is carried in the AXI ID. |
| ADDR_WIDTH | 64 | AXI address width. |
| DATA_WIDTH | 512 | AXI data width in bits. The Genesys 2 build uses 256. |
| ID_WIDTH | 8 | AXI ID width. |
| USER_WIDTH | 8 | Width of m_axi_wuser, which carries the active channel. |
| SEG_COUNT_WIDTH | 8 | Width of the per-channel available-data count from the SRAM controller. Sets the burst clamp. |
| PIPELINE | 1 | 1 allows further AWs before earlier B responses return. 0 waits for the B. |
| AW_MAX_OUTSTANDING | 8 | Maximum outstanding AW transactions per channel. |
| W_PHASE_FIFO_DEPTH | 64 | Depth of the W-phase transaction FIFO, kept in order with the AW issue. |
| B_PHASE_FIFO_DEPTH | 16 | Depth of the B-phase transaction FIFO used to match responses. |

The module also accepts the short aliases NC, AW, DW, IW, UW, SCW and CIW (the channel id width).

### 9.2.1	Derived Constants

Table 2.4.2: AXI Write Engine Derived Constants

| Constant | Definition | Meaning |
| --- | --- | --- |
| BYTES_PER_BEAT | DW / 8 | Bytes per beat. |
| AXSIZE | $clog2(BYTES_PER_BEAT) | Value driven on AWSIZE. |
| XFER_MAX | min(2^(SCW-1) - 1, 254) | Largest burst length value the engine will use. |

## 9.3	Port List

### 9.3.1	Clock, Reset and Configuration

Table 2.4.3: Clock, Reset and Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | Clock. |
| rst_n | input | 1 | Active-low reset. |
| cfg_axi_wr_xfer_beats | input | 8 | Configured burst length (AWLEN value, 0 to 255). Clamped to XFER_MAX. |
| cfg_channel_reset | input | NC | Per-channel reset, level or pulse. Stops new bursts, completes open bursts with null beats, and clears the channel’s error flag. |

### 9.3.2	Scheduler Interface

Table 2.4.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | input | NC | Channel has a write request. |
| sched_wr_ready | output | NC | Pulses when the B response of the last burst of a descriptor arrives. |
| sched_wr_addr | input | NC x AW | Working destination byte address. |
| sched_wr_beats | input | NC x 32 | Beats still to write for the channel. |
| sched_wr_burst_len | input | NC x 8 | Requested burst length. Present on the port; the engine sizes bursts from cfg_axi_wr_xfer_beats and the 4 KB cap. |
| sched_wr_done_strobe | output | NC | Pulses on the AW handshake for the channel. |
| sched_wr_beats_done | output | NC x 32 | Beats issued by that AW (AWLEN + 1). |
| sched_wr_commit_strobe | output | NC | Pulses when the B response for the channel arrives. |
| sched_wr_commit_beats | output | NC x 32 | Beats confirmed by that B response, recovered from the B-phase FIFO. |
| sched_wr_error | output | NC | Sticky write error for the channel. |

The done strobe advances the scheduler’s destination address, which lets the next AW issue without waiting for a response. The commit strobe is what the scheduler counts when it decides the descriptor is complete. Both strobes are per channel and report beat counts, not bytes.

### 9.3.3	SRAM Reservation and Drain Interface

Table 2.4.5: SRAM Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_drain_req | output | NC | Pulses for the granted channel on the AW handshake. |
| axi_wr_drain_size | output | NC x 8 | Beats to reserve (AWLEN + 1). |
| axi_wr_drain_data_avail | input | NC x SCW | Data available per channel after reservations. |
| axi_wr_sram_valid | input | NC | Per-channel valid, registered. Used for arbitration. |
| axi_wr_sram_valid_comb | input | NC | Per-channel valid, combinational. Gates m_axi_wvalid. |
| axi_wr_sram_drain | output | 1 | Drain request. Equal to wvalid && wready. |
| axi_wr_sram_id | output | CIW | Channel selected for the drain. |
| axi_wr_sram_data | input | DW | Data from the selected channel. |
| axi_wr_sram_strb | input | DW/8 | Byte strobes stored with that data beat. New in RAPIDS. |

### 9.3.4	AXI4 Write Master Interface

Table 2.4.6: AXI4 Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awid | output | IW | Write address ID (the channel). |
| m_axi_awaddr | output | AW | Beat-aligned write address. |
| m_axi_awlen | output | 8 | Burst length minus one. |
| m_axi_awsize | output | 3 | AXSIZE, full-width beats. |
| m_axi_awburst | output | 2 | INCR. |
| m_axi_awvalid | output | 1 | Address valid. |
| m_axi_awready | input | 1 | Address ready. |
| m_axi_wdata | output | DW | Write data. |
| m_axi_wstrb | output | DW/8 | Write strobes, equal to axi_wr_sram_strb. |
| m_axi_wlast | output | 1 | Last beat of the burst. |
| m_axi_wuser | output | UW | Channel of the beat being written. |
| m_axi_wvalid | output | 1 | Data valid. |
| m_axi_wready | input | 1 | Data ready. |
| m_axi_bid | input | IW | Response ID. |
| m_axi_bresp | input | 2 | Write response. |
| m_axi_bvalid | input | 1 | Response valid. |
| m_axi_bready | output | 1 | Response ready. |

### 9.3.5	Debug and Status

Table 2.4.7: Debug and Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_wr_all_complete | output | NC | Channel has no outstanding writes. |
| dbg_aw_transactions | output | 32 | Total AW handshakes. |
| dbg_w_beats | output | 32 | Total W beats. |
| o_active_channel_id | output | CIW | Channel whose burst is driving, or about to drive, the W bus. |
| o_active_channel_valid | output | 1 | o_active_channel_id is meaningful. |

## 9.4	Operation

### 9.4.1	Burst Length Cap (rapids BUG-009)

cfg_axi_wr_xfer_beats is an AWLEN value, but a burst cannot be larger than the SRAM that stages it: the engine waits for the whole burst to be present before issuing the AW, and a burst the buffer cannot hold would wait forever. The engine clamps the configured value to XFER_MAX. Without the clamp an AWLEN of 255 wraps the 8-bit AWLEN + 1 and the engine issues an AW on an empty buffer. This is the same fix as in RAPIDS Beats.

### 9.4.2	Byte-Granular Burst Sizing

The sizing follows the read engine:

- w_beats_to_4k is the distance from the aligned channel address to the next 4 KB boundary, in beats.

- w_cap_beats = min(w_xfer_cfg + 1, w_beats_to_4k).

- w_transfer_size (the AWLEN) is sched_wr_beats - 1 if sched_wr_beats <= w_cap_beats, otherwise w_cap_beats - 1.

A channel may issue when the buffer holds at least w_transfer_size + 1 beats (w_has_data), or when this is the final burst. The final-burst test is sched_wr_beats > 0, sched_wr_beats <= w_cap_beats, and the available data covers all remaining beats. The 4 KB cap therefore applies to both tests, so a descriptor that ends just past a boundary still finishes on a short final burst.

m_axi_awaddr is {sched_wr_addr[ch][AW-1:AXSIZE], AXSIZE'b0}. The offset of a descriptor that starts mid-beat never reaches the bus. The first W beat of that descriptor carries WSTRB bits only for the bytes at and above the offset, and the last carries only the bytes up to the end of the packet.

### 9.4.3	Write Strobes

Each SRAM word holds a data beat and its strobe word side by side (see Sink Data Path). The write engine takes both from the drain port and presents them together, so the strobe always matches the data. In RAPIDS Beats WSTRB was tied high.

### 9.4.4	Streaming Pipeline

Each channel with sched_wr_valid and a burst that passes the data test requests arbitration. On grant the AW is issued, and on the AW handshake the engine pulses axi_wr_drain_req with the burst size so the drain controller reserves the beats. W beats stream from the drain port that axi_wr_sram_id selects. axi_wr_sram_drain equals m_axi_wvalid && m_axi_wready. m_axi_wlast closes each burst, and m_axi_wuser carries the channel.

With PIPELINE = 1 a channel may issue further AWs before earlier B responses return, up to AW_MAX_OUTSTANDING. With PIPELINE = 0 the channel waits for each B. That costs throughput, so the top and core default to 1.

### 9.4.5	Two Completion Strobes

Table 2.4.8: Completion Strobes

| Strobe | Raised when | Scheduler use |
| --- | --- | --- |
| sched_wr_done_strobe | AW handshake for the channel | Advance the destination address and the issued-beat count. |
| sched_wr_commit_strobe | B response for the channel | Count committed beats. The descriptor is complete when all beats are committed. |

sched_wr_ready is a third, coarser pulse: it fires when the B response belongs to the last burst of a descriptor.

## 9.5	Error Handling

Table 2.4.9: Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | m_axi_bresp == 2'b10 | Set sticky sched_wr_error for the channel |
| AXI DECERR | m_axi_bresp == 2'b11 | Set sticky sched_wr_error for the channel |
| Any other non-zero response | m_axi_bresp != 2'b00 | Same: the test is not-OKAY, and the channel is taken from BID |

The sink data path ORs sched_wr_error with its own packet-length error before passing it to the scheduler.

### 9.5.1	Channel Reset

cfg_channel_reset[ch] clears the sticky sched_wr_error[ch], so a channel reset recovers a B-response error without aresetn. A burst whose AW has been accepted must still receive all of its W beats, so the engine settles in-flight work rather than dropping it:

- w_kill[ch] is the reset level ORed with r_wr_flush[ch]. The flush bit is set by the reset and holds until the channel has no open burst and no B response due.

- No new AW is arbitrated for a channel with w_kill set. AWADDR comes from sched_wr_addr, which the scheduler does not clear, so the value on the bus stays stable across the reset.

- An open burst is completed with null W beats: WSTRB is zero, WDATA repeats the last captured value and the SRAM drain is not popped, so the null beats modify no byte at the destination. A beat that was presented on W and not accepted when the reset hit is the one exception: it is replayed unchanged, with its own strobes, from a captured copy, so the W channel does not change its payload while WVALID is high. At most one beat per reset is replayed this way.

- B responses of a channel with w_kill set are consumed. The commit strobe, sched_wr_ready, the done strobe and the error flag are suppressed for them.

- Other channels are not affected.

The reset therefore leaves no partial state in the engine: the B-phase FIFO drains through the flush, and the channel restarts with clean counters when the scheduler next raises sched_wr_valid.

## 9.6	Differences from RAPIDS Beats

Table 2.4.10: Write Engine Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| WSTRB | All ones | axi_wr_sram_strb |
| SRAM drain port | Data only | Data plus strobe word |
| AWADDR | Channel address as given (always aligned) | Aligned down to the beat |
| Burst cap and final-burst test | Configured length and remaining beats | Also capped at the 4 KB boundary |
| Everything else |  | Unchanged |

The beats chapter is AXI Write Engine (RAPIDS Beats). The read-side counterpart is AXI Read Engine.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Beats Alloc Control Specification

Module: alloc_ctrl_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

No longer in the SRAM datapath (2026-09-26). RAPIDS’ SRAM path now uses STREAM’s stream_alloc_ctrl / stream_drain_ctrl / stream_latency_bridge, reached through the snk_/src_sram_controller_beats naming wrappers (bdf4e0dff). This FUB is still built and still verified – it keeps its own test, its own filelist and its rapids_all.f entry – it simply is not instantiated by the SRAM controllers any more. Treat this page as the FUB’s own specification, not as a description of the current SRAM path.

## 10.1	Overview

The Beats Alloc Control is a “virtual FIFO” that tracks space allocation without storing actual data. It uses FIFO pointer logic to manage pre-allocation for AXI engines, preventing race conditions between allocation and actual data arrival.

### 10.1.1	Key Features

- Virtual FIFO: Pointer tracking without data storage

- Variable-Size Allocation: Supports multi-beat allocation requests

- Single-Beat Release: Data writes release one beat at a time

- Status Flags: Full, almost full, empty, almost empty

- Space Tracking: Reports available space for flow control

### 10.1.2	Block Diagram

### 10.1.3	Figure 2.5.1: Beats Alloc Control Block Diagram

Alloc control block diagram

Source: 05_alloc_ctrl.dot

## 10.2	Concept: Virtual FIFO

Unlike a traditional FIFO that stores data, alloc_ctrl_beats only tracks space reservations:

|  | Traditional FIFO | Virtual FIFO (alloc_ctrl_beats) |
| --- | --- | --- |
| Write side | wr_data into storage | wr_size advances a pointer |
| Read side | rd_data out of storage | one beat advances a pointer |
| Output | data | space_free |

### 10.2.1	Use Case: Pre-Allocation for AXI Reads

1. AXI engine requests N beats of space
   - alloc_ctrl.wr_valid = 1
   - alloc_ctrl.wr_size = N
   - wr_ptr advances by N

2. As AXI read data arrives (1 beat at a time)
   - alloc_ctrl.rd_valid = 1 (each beat)
   - rd_ptr advances by 1

3. Space becomes available again
   - space_free reflects available capacity

## 10.3	Parameters

parameter int DEPTH = 512;                      // Virtual FIFO depth
parameter int ALMOST_WR_MARGIN = 1;             // Almost full margin
parameter int ALMOST_RD_MARGIN = 1;             // Almost empty margin
parameter int REGISTERED = 1;                   // Registered outputs

// Derived
parameter int AW = $clog2(DEPTH);               // Address width

: Table 2.5.1: Beats Alloc Control Parameters

## 10.4	Port List

### 10.4.1	Clock and Reset

Table 2.5.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low reset |

### 10.4.2	Write Interface (Allocation Requests)

Table 2.5.3: Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | Allocate space |
| wr_size | input | 8 | Number of beats to allocate |
| wr_ready | output | 1 | Space available for allocation |

### 10.4.3	Read Interface (Data Written)

Table 2.5.4: Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | One beat of data written |
| rd_ready | output | 1 | Not full (can accept) |

### 10.4.4	Status Outputs

Table 2.5.5: Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| space_free | output | AW+1 | Available space in beats |
| wr_full | output | 1 | No space available |
| wr_almost_full | output | 1 | Near full |
| rd_empty | output | 1 | No allocations pending |
| rd_almost_empty | output | 1 | Near empty |

## 10.5	Operation

### 10.5.1	Pointer Arithmetic

space_free = DEPTH - (wr_ptr - rd_ptr)

Full condition:    wr_ptr - rd_ptr >= DEPTH
Empty condition:   wr_ptr == rd_ptr

### 10.5.2	Timing Diagram

### 10.5.3	Figure 2.5.2: Allocation and Release Timing

Allocation control - allocate then release

Source: alloc_ctrl_beats_alloc_release.json, captured from dv/tests/fub_beats/test_alloc_ctrl_beats.py (basic_alloc_drain, depth 512, seed 7) with WAVES=1.

Reading it: one wr_valid handshake with wr_size = 6 reserves six beats and space_free drops 512 to 506 the next cycle; rd_empty falls because reserved space is now outstanding. Each later rd_valid handshake releases one beat and space_free climbs back one at a time (507, 508, …). wr_full and wr_almost_full stay low: the virtual FIFO is nowhere near its depth.

## 10.6	Integration Example

// Source path: Pre-allocate SRAM space before AXI read
alloc_ctrl_beats #(
    .DEPTH(SRAM_DEPTH),
    .ALMOST_WR_MARGIN(8)
) u_src_alloc (
    .axi_aclk       (clk),
    .axi_aresetn    (rst_n),

    // Scheduler allocates space before issuing AXI read
    .wr_valid       (sched_rd_valid),
    .wr_size        (sched_rd_beats[7:0]),
    .wr_ready       (alloc_ready),

    // AXI read data arrival releases allocation
    .rd_valid       (m_axi_rvalid && m_axi_rready),
    .rd_ready       (),  // Not used for single-beat

    // Status
    .space_free     (sram_space_available),
    .wr_full        (sram_full),
    .wr_almost_full (sram_almost_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Beats Drain Control Specification

Module: drain_ctrl_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

No longer in the SRAM datapath (2026-09-26). RAPIDS’ SRAM path now uses STREAM’s stream_alloc_ctrl / stream_drain_ctrl / stream_latency_bridge, reached through the snk_/src_sram_controller_beats naming wrappers (bdf4e0dff). This FUB is still built and still verified – it keeps its own test, its own filelist and its rapids_all.f entry – it simply is not instantiated by the SRAM controllers any more. Treat this page as the FUB’s own specification, not as a description of the current SRAM path.

## 11.1	Overview

The Beats Drain Control is a “virtual FIFO” that tracks data availability without storing actual data. It complements alloc_ctrl_beats by tracking when data has been written to SRAM and is available for draining.

### 11.1.1	Key Features

- Virtual FIFO: Pointer tracking without data storage

- Single-Beat Write: Data arrivals tracked one beat at a time

- Variable-Size Drain: Supports multi-beat drain requests

- Status Flags: Full, almost full, empty, almost empty

- Data Availability: Reports beats available for drain

### 11.1.2	Block Diagram

### 11.1.3	Figure 2.6.1: Beats Drain Control Block Diagram

Drain control block diagram

Source: 06_drain_ctrl.dot

## 11.2	Concept: Data Availability Tracking

The drain_ctrl tracks when data has been written to SRAM:

alloc_ctrl:  Tracks SPACE reservations (allocation -> actual write)
drain_ctrl:  Tracks DATA availability (data write -> drain request)

### 11.2.1	Complementary Roles

Source Path:
  1. Scheduler allocates space (alloc_ctrl.wr)
  2. AXI reads data into SRAM
  3. Data written to SRAM (drain_ctrl.wr)
  4. Drain consumer reads (drain_ctrl.rd)

Sink Path:
  1. Fill allocates space (alloc_ctrl.wr)
  2. Fill writes to SRAM (alloc_ctrl.rd)
  3. Data available in SRAM (drain_ctrl.wr)
  4. AXI write drains SRAM (drain_ctrl.rd)

## 11.3	Parameters

parameter int DEPTH = 512;                      // Virtual FIFO depth
parameter int ALMOST_WR_MARGIN = 1;             // Almost full margin
parameter int ALMOST_RD_MARGIN = 1;             // Almost empty margin
parameter int REGISTERED = 1;                   // Registered outputs

// Derived
parameter int AW = $clog2(DEPTH);               // Address width

: Table 2.6.1: Beats Drain Control Parameters

## 11.4	Port List

### 11.4.1	Clock and Reset

Table 2.6.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low reset |

### 11.4.2	Write Interface (Data Written to SRAM)

Table 2.6.3: Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | One beat written to SRAM |
| wr_ready | output | 1 | Can track more data |

### 11.4.3	Read Interface (Drain Requests)

Table 2.6.4: Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | Drain request |
| rd_size | input | 8 | Number of beats to drain |
| rd_ready | output | 1 | Data available for drain |

### 11.4.4	Status Outputs

Table 2.6.5: Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| data_available | output | AW+1 | Beats available to drain |
| wr_full | output | 1 | Tracking full |
| wr_almost_full | output | 1 | Near full |
| rd_empty | output | 1 | No data to drain |
| rd_almost_empty | output | 1 | Near empty |

## 11.5	Operation

### 11.5.1	Pointer Arithmetic

data_avail = wr_ptr - rd_ptr

Full condition:    wr_ptr - rd_ptr >= DEPTH
Empty condition:   wr_ptr == rd_ptr

### 11.5.2	Timing Diagram

### 11.5.3	Figure 2.6.2: Data Arrival and Drain Timing

Drain control - single-beat arrivals then a multi-beat drain

Source: drain_ctrl_beats_arrive_drain.json, captured from dv/tests/fub_beats/test_drain_ctrl_beats.py (basic_write_drain, depth 512, seed 7) with WAVES=1.

Reading it: six wr_valid handshakes, one beat each, raise data_available 1, 2, … 6 and drop rd_empty on the first arrival. A single rd_valid handshake with rd_size = 6 then reserves all six at once: data_available returns to 0 and rd_empty rises the next cycle. The counter tracks beats, not bytes, and never touches the data itself.

## 11.6	Integration Example

// Sink path: Track data availability for AXI write engine
drain_ctrl_beats #(
    .DEPTH(SRAM_DEPTH),
    .ALMOST_RD_MARGIN(8)
) u_snk_drain (
    .axi_aclk       (clk),
    .axi_aresetn    (rst_n),

    // Fill writes data to SRAM
    .wr_valid       (fill_valid && fill_ready),
    .wr_ready       (),  // Single-beat, always ready

    // AXI write engine drains data
    .rd_valid       (axi_wr_drain_req),
    .rd_size        (axi_wr_burst_len),
    .rd_ready       (data_ready_for_drain),

    // Status
    .data_available (beats_to_write),
    .rd_empty       (no_data_to_write)
);

## 11.7	Relationship: alloc_ctrl vs drain_ctrl

Table 2.6.6: alloc_ctrl vs drain_ctrl Comparison

| Aspect | alloc_ctrl_beats | drain_ctrl_beats |
| --- | --- | --- |
| Tracks | Space reservations | Data availability |
| Write port | Multi-beat allocation | Single-beat arrival |
| Read port | Single-beat release | Multi-beat drain |
| Output | space_free | data_available |
| Use case | Pre-allocate before fill | Know when to drain |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Latency Bridge (beats) Specification

Module: latency_bridge_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented, tested, not in the SRAM path Last Updated: 2026-09-27

Not in the SRAM datapath since 2026-09-26. RAPIDS’ SRAM path is STREAM’s sram_controller, which carries its own stream_latency_bridge (the same design), reached through the snk_/src_sram_controller_beats naming wrappers (bdf4e0dff). This FUB is still built and verified – it keeps its test, its filelist and its rapids_all.f entry – but nothing in rapids_core_beats instantiates it. This page is the FUB’s own specification.

## 12.1	Overview

latency_bridge_beats turns a registered-read FIFO – one whose data appears the cycle after the read handshake – into a plain valid/ready stream the consumer can stall at will. It is a data-width bridge: it moves DATA_WIDTH bits per beat and carries nothing else. There is no beat count, no channel ID and no programmable delay in it; an earlier version of this page described such an interface, and it never existed in this module.

### 12.1.1	Key Features

- One-cycle glue, one skid buffer: a single flop (r_drain_ip) remembers that a FIFO read was accepted, so the data that lands the next cycle is pushed into a gaxi_fifo_sync skid of depth SKID_DEPTH

- Full throughput when the consumer is ready: back-to-back reads are accepted as long as the skid keeps draining

- Backpressure absorbed in the skid: s_ready drops only when a skid write is actually stalled, so the FIFO is never read into a slot that does not exist

- Occupancy reported: occupancy counts the beats the bridge holds (the one in flight plus the skid contents), which is what an occupancy-based data-available counter needs

- Debug taps: dbg_r_pending and dbg_r_out_valid expose the in-flight flop and the skid output valid, for catching stuck data from a test

### 12.1.2	Block Diagram

### 12.1.3	Figure 2.7.1: Latency Bridge Block Diagram

Latency bridge block diagram

Source: 07_latency_bridge.dot

## 12.2	Concept: bridging a registered read

A FIFO with REGISTERED=1 read (the SRAM style this repo uses so BRAM infers cleanly) answers a read handshake with data one cycle later. A consumer speaking valid/ready expects data with valid, and may deassert ready at any time. Without a bridge the FIFO has already popped a beat the consumer did not take.

cycle 0   s_valid && s_ready         FIFO read accepted        r_drain_ip <= 1
cycle 1   data arrives on s_data     skid_wr_valid = r_drain_ip, data pushed
cycle N   m_valid && m_ready         consumer drains the skid at its own pace

The bridge never guesses about downstream readiness: s_ready is derived from the skid’s room, counting only a write that is stalled this cycle as pending (a write that completes this cycle frees its slot at the same edge). That is why full throughput holds while writes complete immediately, and why the FIFO is never over-read when they do not.

## 12.3	Parameters

parameter int DATA_WIDTH = 64;   // Beat width
parameter int SKID_DEPTH = 4;    // Skid buffer depth (2-4 recommended)
parameter int DW = DATA_WIDTH;   // Short alias

: Table 2.7.1: Latency Bridge Parameters

occupancy is 3 bits wide and counts 0..5 at the default depth (one beat in flight plus four in the skid); a larger SKID_DEPTH needs the port widened.

## 12.4	Port List

### 12.4.1	Clock and Reset

Table 2.7.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 12.4.2	Upstream Interface (from the registered FIFO)

Table 2.7.3: Upstream Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_valid | input | 1 | FIFO has a beat to read (not empty) |
| s_ready | output | 1 | Bridge accepts a read this cycle; the data lands NEXT cycle |
| s_data | input | DW | Read data, valid one cycle after the s_valid && s_ready handshake |

### 12.4.3	Downstream Interface (to the consumer)

Table 2.7.4: Downstream Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_valid | output | 1 | Beat available |
| m_ready | input | 1 | Consumer takes the beat |
| m_data | output | DW | Beat data, valid with m_valid |

### 12.4.4	Status and Debug

Table 2.7.5: Status and Debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| occupancy | output | 3 | Beats held by the bridge: the in-flight read plus the skid contents (0..5) |
| dbg_r_pending | output | 1 | r_drain_ip: a read was accepted and its data is due next cycle |
| dbg_r_out_valid | output | 1 | Skid output valid (mirrors m_valid) |

## 12.5	Operation

### 12.5.1	Figure 2.7.2: Latency Bridge Timing (consumer always ready)

Latency bridge behind a registered-read FIFO

Source: latency_bridge_beats_streaming.json, captured from dv/tests/fub_beats/test_latency_bridge_beats.py (streaming, 256-bit, seed 7) with WAVES=1. The test drives a REGISTERED = 1 gaxi_fifo_sync in front of the bridge (dv/tb/latency_bridge_beats_tb_top.sv), because that is the only way to produce the bridge’s upstream contract: a valid/ready master presents data with valid, a registered FIFO presents it one cycle after the read handshake.

Reading it: each wr_valid beat lands in the FIFO and s_valid (FIFO not empty) rises a cycle later; the bridge reads it at once (s_ready stays high) and dbg_r_pending marks the cycle the data is in flight. Two cycles after the write the beat is on m_valid/m_data with its payload intact (1, 2, 3, …), and occupancy shows exactly one beat inside the bridge per transfer. With a consumer that is always ready the bridge adds the FIFO’s read latency and no more.

An earlier revision of this test drove s_* directly from the BFM and never compared m_data; every beat came out 0 and the test passed (rapids TASK-003). (dv/tests/fub_beats/test_latency_bridge_beats.py).

## 12.6	Integration Context

The bridge sits between a gaxi_fifo_sync with REGISTERED=1 and whatever consumes the stream. The same arrangement, with STREAM’s twin stream_latency_bridge, is what sram_controller_unit uses inside the shared SRAM controller today; occupancy there is what the drain accounting must allow for (stream BUG-011).

latency_bridge_beats #(
    .DATA_WIDTH (512),
    .SKID_DEPTH (4)
) u_bridge (
    .clk             (clk),
    .rst_n           (rst_n),
    // registered-read FIFO side
    .s_valid         (fifo_rd_valid),
    .s_ready         (fifo_rd_ready),
    .s_data          (fifo_rd_data),
    // consumer side
    .m_valid         (drain_valid),
    .m_ready         (drain_read),
    .m_data          (drain_data),
    // status / debug
    .occupancy       (bridge_occupancy),
    .dbg_r_pending   (dbg_bridge_pending),
    .dbg_r_out_valid (dbg_bridge_out_valid)
);

## 12.7	Design Considerations

Table 2.7.6: Skid Depth Selection

| SKID_DEPTH | Behaviour |
| --- | --- |
| 2 | Minimum: one stall of the consumer costs a bubble on the next read |
| 4 | Default: absorbs a short consumer stall with no upstream bubble |
| 8 | Deeper stall absorption; occupancy must be widened past 3 bits |

The skid does not need to cover the FIFO’s read latency (that is the one r_drain_ip flop’s job); it needs to cover how long the consumer stalls without the bridge stopping upstream reads.

Last Updated: 2026-09-27

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Control-Read Engine Specification

Module: ctrlrd_engine.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2026-07-12

## 13.1	Overview

The Control-Read Engine implements the CTRL_READ descriptor opcode (DESC_OP_CTRL_READ = 2'b01) as a consumer gate. When the scheduler decodes a CTRL_READ descriptor, it hands the engine a poll address, an expected value, and a compare mask. The engine issues an AXI4 read to the poll address, applies the mask to the returned data, and compares (read_data & mask) against the expected value. If they match, the gate is satisfied and the descriptor chain proceeds; if not, the engine retries. Retries are bounded so a gate that never matches cannot hang the channel.

One Control-Read Engine is instantiated per channel in scheduler_group_beats.sv (u_ctrlrd_engine), configured for 32-bit AXI reads.

### 13.1.1	Key Features

- Consumer Gate Semantics: Polls memory until (read_data & mask) == expected

- Masked Comparison: Only the masked bits of the polled word participate in the match

- Bounded Retry: cfg_ctrlrd_max_try (0-511) caps the number of poll attempts

- 1 microsecond Retry Pacing: Retries wait for tick_1us from the scheduler group

- Error on Exhaustion: Asserts ctrlrd_error if the gate never matches within budget

- Null Address Fast-Path: A poll address of zero completes immediately as a match

- Channel Reset Support: cfg_channel_reset returns the FSM to idle and drains any read still on the fabric

- MonBus Integration: Completion, retry, and error events

### 13.1.2	Block Diagram

### 13.1.3	Figure 2.8.1: Control-Read Engine Block Diagram

Control-read engine block diagram

Source: 08_ctrlrd_engine.dot

## 13.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 32;                 // Total channels
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Channel index width (derived)
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int AXI_DATA_WIDTH = 64;               // AXI read data width (>= 32)
parameter int AXI_ID_WIDTH = 8;                  // AXI ID width (>= CHAN_WIDTH)

// Monitor Bus Parameters
parameter logic [15:0] MON_AGENT_ID  = 16'h0030; // Control-Read Engine Agent ID
parameter logic [7:0]  MON_UNIT_ID   = 8'h02;    // Unit identifier
parameter logic [8:0]  MON_CHANNEL_ID = 9'h000;  // Base channel ID

: Table 2.8.1: Control-Read Engine Parameters

Parameter validation enforces AXI_ID_WIDTH >= CHAN_WIDTH and AXI_DATA_WIDTH >= 32 (the engine consumes only the lower 32 bits of the read data). The per-channel instance in scheduler_group_beats.sv overrides AXI_DATA_WIDTH to 32.

## 13.3	Port List

### 13.3.1	Clock and Reset

Table 2.8.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 13.3.2	Scheduler Interface

Table 2.8.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctrlrd_valid | input | 1 | Gate request valid from scheduler |
| ctrlrd_ready | output | 1 | Engine ready to accept a request |
| ctrlrd_pkt_addr | input | AW | Poll address (descriptor poll_addr[63:0]) |
| ctrlrd_pkt_data | input | 32 | Expected value (descriptor expected[95:64]) |
| ctrlrd_pkt_mask | input | 32 | Compare mask (descriptor mask[127:96]) |
| ctrlrd_error | output | 1 | Gate failed (retries exhausted or AXI error) |
| ctrlrd_result | output | 32 | Last read value (lower 32 bits) |
| ctrlrd_engine_idle | output | 1 | Engine idle (post-issue completion indicator) |

### 13.3.3	Configuration Interface

Table 2.8.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_ctrlrd_max_try | input | 9 | Poll retry budget, 0-511 (from CTRL_CONFIG.CTRLRD_MAX_TRY) |
| cfg_channel_reset | input | 1 | Per-channel reset request |
| tick_1us | input | 1 | 1 microsecond tick that paces retries |

### 13.3.4	AXI4 Read Master Interface

Table 2.8.5: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ar_valid | output | 1 | Read address valid |
| ar_ready | input | 1 | Read address ready |
| ar_addr | output | AW | Read address (poll address) |
| ar_len | output | 8 | Burst length (8'h00, single beat) |
| ar_size | output | 3 | Burst size (3'b010, 4 bytes) |
| ar_burst | output | 2 | Burst type (2'b01, INCR) |
| ar_id | output | ID | Transaction ID (channel-derived) |
| ar_lock | output | 1 | Lock (1'b0, normal access) |
| ar_cache | output | 4 | Cache (4'b0010, non-cacheable bufferable) |
| ar_prot | output | 3 | Protection (3'b000) |
| ar_qos | output | 4 | Quality of service (4'h0) |
| ar_region | output | 4 | Region (4'h0) |
| r_valid | input | 1 | Read data valid (shared channel) |
| r_ready | output | 1 | Read data ready |
| r_data | input | ADW | Read data (lower 32 bits used) |
| r_id | input | ID | Read response ID (matched to our request) |
| r_resp | input | 2 | Read response |
| r_last | input | 1 | Read last beat |

### 13.3.5	MonBus Interface

Table 2.8.6: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | 64 | Side-band monitor timestamp |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor consumer ready |
| mon_packet | output | 128 | Monitor packet (monitor_packet_t) |
| mon_timestamp | output | 64 | Monitor packet timestamp |

## 13.4	FSM States

### 13.4.1	Figure 2.8.2: Control-Read Engine FSM

States and transitions of the ctrlrd_engine poll FSM.

Diagram 2

State summary:

- READ_IDLE: Waits for a request from the scheduler (via a two-deep skid buffer). Latches poll address, expected value, mask, and the AXI ID; initializes the retry counter from cfg_ctrlrd_max_try.

- READ_ISSUE_ADDR: Drives ar_valid. A null address (64'h0) shortcuts to READ_MATCH; otherwise, once ar_ready handshakes, advances to READ_WAIT_DATA.

- READ_WAIT_DATA: Accepts the read response whose r_id matches the channel ID, capturing the lower 32 bits and the response code.

- READ_COMPARE: On an AXI error response, goes to READ_ERROR. On a masked match, goes to READ_MATCH. Otherwise, if retries remain, decrements the counter and goes to READ_RETRY_WAIT; if the budget is exhausted, goes to READ_ERROR.

- READ_RETRY_WAIT: Waits for tick_1us, then reissues the read.

- READ_MATCH: Gate satisfied. Clears the error flag and returns to READ_IDLE.

- READ_ERROR: Sets ctrlrd_error and returns to READ_IDLE.

## 13.5	Operation

### 13.5.1	Masked Compare

The engine gates on a masked equality check:

w_masked_expected = ctrlrd_pkt_data & ctrlrd_pkt_mask
w_masked_actual   = r_data[31:0]    & ctrlrd_pkt_mask
w_data_match      = (w_masked_expected == w_masked_actual)

Only the bits set in mask participate. A mask of 32'hFFFFFFFF compares the full word; a mask of a single bit turns the gate into a flag-poll.

### 13.5.2	Retry Budget and Pacing

The retry counter is loaded from cfg_ctrlrd_max_try when the request is accepted. On each non-matching, non-error compare with retries remaining, the counter decrements and the engine parks in READ_RETRY_WAIT until the next tick_1us, which paces successive polls at roughly 1 microsecond intervals. When the counter reaches zero without a match, the engine transitions to READ_ERROR and asserts ctrlrd_error, so a never-matching gate cannot stall the channel indefinitely.

cfg_ctrlrd_max_try is driven by the CTRL_CONFIG register at offset 0x240, field CTRLRD_MAX_TRY[8:0] (0-511, reset 16).

### 13.5.3	Null Address Fast-Path

If the latched poll address is 64'h0, the engine treats the gate as already satisfied: READ_ISSUE_ADDR transitions directly to READ_MATCH without issuing an AXI read. This lets software emit an unconditional (no-op) gate.

### 13.5.4	AXI Read Transaction

Polls are single-beat, 4-byte INCR reads (ar_len = 0, ar_size = 3'b010, ar_burst = 2'b01). The ar_id is derived from CHANNEL_ID, and only responses whose r_id matches are consumed on the shared read data channel. ctrlrd_result exposes the last captured 32-bit value; in the per-channel instantiation the scheduler gates on match/error rather than the raw value, so ctrlrd_result is left unconnected there.

### 13.5.5	Channel Reset

cfg_channel_reset is registered into r_channel_reset_active. While asserted, the engine refuses new requests (ctrlrd_ready is forced low), the retry and error state is cleared, and the FSM is driven back to READ_IDLE.

What the reset cannot do is cancel a read already on the fabric, because AXI has no cancel. The engine therefore drains it (rapids TASK-014, 2026-09-27):

- an AR that was raised and not yet accepted stays raised until the slave takes it (r_drain_ar), then

- the R beat that the accepted AR is owed is taken and discarded (r_drain_pending, which alone lifts r_ready outside READ_WAIT_DATA).

Until the beat is gone ctrlrd_engine_idle stays low and the request path stays closed, so the next read on this ID cannot be answered by the stale beat. Before this the reset dropped the FSM to idle with r_ready low, the beat sat on the bus, and a re-kicked channel took it as its own answer.

ctrlrd_engine_idle asserts only in READ_IDLE with no pending request, no active channel reset and nothing left to drain.

## 13.6	MonBus Reporting

The engine emits monitor packets on notable events:

Table 2.8.7: MonBus Events

| Event | Packet Type | Code | Payload |
| --- | --- | --- | --- |
| Gate satisfied (non-null) | PktTypeCompletion | CORE_COMPL_CTRLRD_COMPLETED | Poll address |
| Retry attempt | PktTypePerf | CORE_PERF_CTRLRD_RETRY | Remaining retry count |
| Retries exhausted | PktTypeError | CORE_ERR_CTRLRD_MAX_RETRIES | Poll address |
| AXI error response | PktTypeError | AXI_ERR_RESP_SLVERR / AXI_ERR_RESP_DECERR | Response code |

All packets carry MON_AGENT_ID, MON_UNIT_ID, and MON_CHANNEL_ID, and are timestamped from i_mon_time.

## 13.7	Descriptor Field Mapping

The CTRL_READ opcode reinterprets the shared 256-bit descriptor (the descriptor engine’s existing field extraction is reused):

Table 2.8.8: CTRL_READ Descriptor Field Mapping

| Descriptor Field | Bits | Engine Port |
| --- | --- | --- |
| poll_addr | [63:0] | ctrlrd_pkt_addr |
| expected | [95:64] | ctrlrd_pkt_data |
| mask | [127:96] | ctrlrd_pkt_mask |
| max_try | [143:128] | Per-descriptor budget (0 = use cfg_ctrlrd_max_try default) |

Last Updated: 2026-07-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Control-Write Engine Specification

Module: ctrlwr_engine.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2025-01-10

## 14.1	Overview

The Control-Write Engine executes a CTRL_WRITE control descriptor (DESC_OP_CTRL_WRITE = 2'b10). A CTRL_WRITE is a producer doorbell: the engine performs a single-beat AXI4 write of the descriptor’s 32-bit wr_data value to its 64-bit wr_addr target, then completes so the descriptor chain continues. One instance is dedicated per channel.

The write target address must be 4-byte aligned because the transfer is a 32-bit (aw_size = 3'b010) single-beat write. A null address (wr_addr == 0) is treated as a no-op and the request completes without issuing an AXI transaction.

### 14.1.1	Key Features

- Single-Beat AXI4 Write: One 32-bit doorbell write per request (aw_len = 0, w_last = 1)

- Producer Doorbell Semantics: Write value to target, then release the chain (no poll, no retry)

- Address Alignment Check: Requires 4-byte-aligned wr_addr; misalignment raises an error

- Null-Address Skip: wr_addr == 0 completes as a no-op

- Channel Reset Support: cfg_channel_reset returns the FSM to idle and drains any write still on the fabric

- MonBus Integration: Completion and error event reporting

### 14.1.2	Block Diagram

### 14.1.3	Figure 2.9.1: Control-Write Engine Block Diagram

Control-write engine block diagram

Source: 09_ctrlwr_engine.dot

## 14.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 32;                 // Total channels
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Derived channel index width
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int AXI_ID_WIDTH = 8;                  // AXI transaction ID width

// Monitor Bus Parameters
parameter logic [15:0] MON_AGENT_ID  = 16'h0020; // Ctrlwr Engine Agent ID
parameter logic [7:0]  MON_UNIT_ID   = 8'h01;    // Unit identifier
parameter logic [8:0]  MON_CHANNEL_ID = 9'h000;  // Base channel ID

: Table 2.9.1: Control-Write Engine Parameters

Note: AXI_ID_WIDTH must be >= CHAN_WIDTH (checked by an elaboration-time $fatal). When instantiated in scheduler_group_beats.sv as u_ctrlwr_engine, MON_AGENT_ID is overridden to 16'h0021 (CTRLWR_MON_AGENT_ID) to distinguish it from the Control-Read Engine’s 16'h0020.

## 14.3	Port List

### 14.3.1	Clock and Reset

Table 2.9.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 14.3.2	Configuration Interface

Table 2.9.3: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_reset | input | 1 | Per-channel reset: blocks new requests, drains transaction, forces idle |

### 14.3.3	Scheduler Interface

Table 2.9.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctrlwr_valid | input | 1 | Request valid (doorbell write requested) |
| ctrlwr_ready | output | 1 | Ready to accept request |
| ctrlwr_pkt_addr | input | AW | Doorbell target address (wr_addr, 4-byte aligned) |
| ctrlwr_pkt_data | input | 32 | Doorbell write value (wr_data) |
| ctrlwr_error | output | 1 | Error flag (alignment or AXI response error) |
| ctrlwr_engine_idle | output | 1 | Engine idle (in WRITE_IDLE, no active transaction, not in channel reset) |

### 14.3.4	AXI4 Write Master Interface

Table 2.9.5: AXI4 Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| aw_valid | output | 1 | Write address valid |
| aw_ready | input | 1 | Write address ready |
| aw_addr | output | AW | Write address (= wr_addr) |
| aw_len | output | 8 | Burst length (0 = single beat) |
| aw_size | output | 3 | Transfer size (3'b010 = 4 bytes / 32-bit) |
| aw_burst | output | 2 | Burst type (2'b01 = INCR) |
| aw_id | output | AXI_ID_WIDTH | Transaction ID (derived from CHANNEL_ID) |
| aw_lock | output | 1 | Lock type (0 = normal) |
| aw_cache | output | 4 | Cache type (4'b0010 = non-cacheable bufferable) |
| aw_prot | output | 3 | Protection type (3'b000) |
| aw_qos | output | 4 | Quality of service (0) |
| aw_region | output | 4 | Region identifier (0) |
| w_valid | output | 1 | Write data valid |
| w_ready | input | 1 | Write data ready |
| w_data | output | 32 | Write data (= wr_data) |
| w_strb | output | 4 | Write strobes (4'b1111 = all bytes) |
| w_last | output | 1 | Last beat (1 = single beat) |
| b_valid | input | 1 | Write response valid |
| b_ready | output | 1 | Write response ready |
| b_id | input | AXI_ID_WIDTH | Response ID (matched against expected AXI ID) |
| b_resp | input | 2 | Write response code |

### 14.3.5	MonBus Interface

Table 2.9.6: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | 64 | Monitor timestamp side-band input |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready |
| mon_packet | output | 128 | Monitor packet (monitor_packet_t) |
| mon_timestamp | output | 64 | Monitor packet timestamp |

## 14.4	FSM States

### 14.4.1	Figure 2.9.2: Control-Write Engine FSM

States and transitions of the ctrlwr_engine doorbell FSM.

Diagram 3

State summary:

- WRITE_IDLE: Wait for a request from the request skid buffer; latch wr_addr/wr_data and the expected AXI ID.

- WRITE_ISSUE_ADDR: Drive aw_valid until aw_ready. A misaligned address routes to WRITE_ERROR; a null address returns to WRITE_IDLE (no-op skip).

- WRITE_ISSUE_DATA: Drive w_valid until w_ready; advance once both address and data phases have been issued.

- WRITE_WAIT_RESP: Accept the matching b-channel response; a non-OKAY b_resp routes to WRITE_ERROR, otherwise to WRITE_COMPLETE.

- WRITE_COMPLETE: Emit a completion MonBus packet (unless null address) and return to WRITE_IDLE.

- WRITE_ERROR: Latch ctrlwr_error, emit an error MonBus packet, and return to WRITE_IDLE. The error flag persists until reset or cfg_channel_reset.

## 14.5	Operation

### 14.5.1	Doorbell Write Sequence

### 14.5.2	Figure 2.9.3: Control-Write Doorbell Timing

ctrlwr_engine - one doorbell write

Source: ctrlwr_engine_doorbell.json, captured from dv/tests/fub/test_ctrlwr_engine.py (basic_write, channel 0, 64-bit address, REG_LEVEL=GATE) with WAVES=1; the AXI side is the framework write slave.

Reading it: the request handshakes in one cycle (ctrlwr_ready is held high by the skid buffer) with the doorbell address 0x1000 and data 0x12345678, and the engine leaves idle the next cycle. AW goes out two cycles after the request, W two cycles after AW, and the slave answers B (OKAY) two cycles after W; ctrlwr_engine_idle returns the cycle after the B handshake, eight cycles after the request. Nothing is pipelined across doorbells: the engine takes one request at a time.

### 14.5.3	Request Handshake and Skid Buffer

Incoming requests are captured through a depth-2 gaxi_skid_buffer. The engine accepts a request (ctrlwr_ready) only when not in an active channel reset. The FSM pops the buffered request in WRITE_IDLE, latching {wr_addr, wr_data} and computing the expected AXI ID from CHANNEL_ID:

aw_id = {{(AXI_ID_WIDTH-CHAN_WIDTH){1'b0}}, CHANNEL_ID[CHAN_WIDTH-1:0]};

The b-channel response is matched against this expected ID (b_id == r_expected_axi_id) so a shared write master can be demultiplexed per channel.

### 14.5.4	Address Validation

Null (skip)  : wr_addr == 64'h0
Align error  : wr_addr[1:0] != 2'b00  (and not null)

A null address completes immediately as a no-op with no AXI activity and no completion packet. A misaligned address never issues on AXI; it transitions to WRITE_ERROR, asserts ctrlwr_error, and emits an error MonBus packet.

### 14.5.5	Error Reporting

ctrlwr_error is a registered, sticky flag. It is set on an alignment error or on a non-OKAY AXI write response (b_resp != 2'b00), and it persists until rst_n or cfg_channel_reset. The MonBus error packet carries either the offending address (alignment error) or the captured b_resp code (AXI response error).

### 14.5.6	Channel Reset

Asserting cfg_channel_reset registers r_channel_reset_active, which blocks new requests (ctrlwr_ready low), clears the error flag and the phase-issued state, and drives the FSM back to WRITE_IDLE.

A write already on the fabric cannot be cancelled, so the engine drains it (rapids TASK-014, 2026-09-27): a raised AW stays up until accepted (r_drain_aw), an accepted AW is followed by its W beat with the latched data (r_drain_w), and the B response is taken and discarded (r_drain_b, which alone lifts b_ready outside WRITE_WAIT_RESP). The write that was already issued therefore lands. Until B is taken ctrlwr_engine_idle stays low and the request path stays closed. Before this the reset abandoned whatever phase was in flight and left a dangling response on the channel’s ID.

ctrlwr_engine_idle asserts only in WRITE_IDLE with no pending request, no active channel reset and nothing left to drain.

## 14.6	Contrast with the Control-Read Engine

The Control-Write Engine is the producer counterpart to the Control-Read Engine (Section 2.8). Both are per-channel control-descriptor executors, but they implement opposite half of the producer/consumer handshake:

| Aspect | Control-Read Engine (2.8) | Control-Write Engine (2.9) |
| --- | --- | --- |
| Opcode | DESC_OP_CTRL_READ (2'b01) | DESC_OP_CTRL_WRITE (2'b10) |
| Role | Consumer gate | Producer doorbell |
| AXI direction | Read (poll) | Single-beat write |
| Semantics | Poll until (rd & mask) == expected | Write wr_data to wr_addr, then continue |
| Retry budget | max_try retry budget (config default) | None (single unconditional write) |
| Chain effect | Holds off the chain until the gate passes | Releases the chain after the write completes |
| Descriptor fields | poll_addr / expected / mask / max_try | wr_addr / wr_data |

A CTRL_WRITE therefore has no maximum-try configuration: it is one unconditional 32-bit write, whereas CTRL_READ polls with a bounded retry budget before either passing the gate or timing out.

Last Updated: 2026-07-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Scheduler Group

Module: scheduler_group.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 15.1	Overview

The scheduler group wraps one channel: a descriptor engine, the scheduler, the two control engines (control read and control write) and a MonBus arbiter that merges their monitor packets. It is the unit the scheduler group array instantiates once per channel.

The structure is identical to the RAPIDS Beats group. The one change is the packet-record port set: the scheduler now publishes a per-direction record of the packet’s byte length and start offset, and the group passes it through unchanged.

### 15.1.1	Key Features

- One channel, four engines: descriptor engine, scheduler, control read engine, control write engine.

- Packet-record pass-through: sched_rd_pkt_* and sched_wr_pkt_* connect the scheduler straight to the group boundary.

- MonBus aggregation: a four-client arbiter merges the four engines’ packets.

- Completion gating: cfg_sched_compl_enable drops CORE Completion packets from the descriptor engine and the scheduler at the arbiter input and acknowledges the emitters, so nothing stalls (rapids ISSUE-005). Errors are never gated.

- Status aggregation: idle, FSM state and sticky error are exported.

### 15.1.2	Block Diagram

### 15.1.3	Figure 3.1.1: Scheduler Group Block Diagram

Scheduler group block diagram

Source: 07_scheduler_group.dot

## 15.2	Parameters

Table 3.1.1: Scheduler Group Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| CHANNEL_ID | 0 | Channel identifier. |
| NUM_CHANNELS | 8 | Total channels. |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Channel id width. |
| ADDR_WIDTH | 64 | Address width. |
| DATA_WIDTH | 512 | Data width in bits. Sets the beat size seen by the scheduler. |
| AXI_ID_WIDTH | 8 | AXI ID width for the descriptor and control masters. |
| DESC_MON_AGENT_ID | 16 | MonBus agent id of the descriptor engine. |
| SCHED_MON_AGENT_ID | 48 | MonBus agent id of the scheduler. |
| CTRLRD_MON_AGENT_ID | 32 | MonBus agent id of the control read engine. |
| CTRLWR_MON_AGENT_ID | 33 | MonBus agent id of the control write engine. |
| MON_UNIT_ID | 1 | MonBus unit id. |
| MON_CHANNEL_ID | 0 | Base MonBus channel id. |
| EN_READ | 1 | Enable the read direction of the scheduler. |
| EN_WRITE | 1 | Enable the write direction of the scheduler. |
| USE_ROW_COL_MAJOR_ADDRESSING | 1 | Descriptor engine addressing mode. |
| GEN_MON | 1 | Enable monitor packet generation. |
| OFF_W | $clog2(DATA_WIDTH/8) | Width of the packet-record byte offset. New in RAPIDS. |

## 15.3	Port List

### 15.3.1	Clock, Reset and Kick-Off

Table 3.1.2: Clock, Reset and Kick-Off

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock. |
| rst_n | input | 1 | Active-low reset. |
| apb_valid | input | 1 | Channel kick-off. |
| apb_ready | output | 1 | Descriptor engine ready for kick-off. |
| apb_addr | input | AW | First descriptor address. |

### 15.3.2	Configuration Interface

Table 3.1.3: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable this channel. |
| cfg_channel_reset | input | 1 | Per-channel soft reset. |
| cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window in cycles. |
| cfg_sched_timeout_limit | input | 8 | Consecutive timeout windows before a fatal error. 0 means never. |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection. |
| cfg_sched_err_enable | input | 1 | Enable error reporting. |
| cfg_sched_compl_enable | input | 1 | 1 passes CORE Completion packets to the monitor bus; 0 drops them at the arbiter inputs. |
| cfg_sched_perf_enable | input | 1 | Enable performance monitoring. |
| cfg_ctrlrd_max_try | input | 9 | Control-read poll retry budget, 0 to 511. |
| tick_1us | input | 1 | 1 us tick for control-read retry spacing. |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch. |
| cfg_desceng_fifo_thresh | input | 4 | Prefetch threshold. |
| cfg_desceng_addr0_base | input | AW | Valid descriptor address range 0 base. |
| cfg_desceng_addr0_limit | input | AW | Valid descriptor address range 0 limit. |
| cfg_desceng_addr1_base | input | AW | Valid descriptor address range 1 base. |
| cfg_desceng_addr1_limit | input | AW | Valid descriptor address range 1 limit. |

### 15.3.3	Status

Table 3.1.4: Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle | output | 1 | Descriptor engine idle. |
| scheduler_idle | output | 1 | Scheduler idle. |
| scheduler_state | output | 7 | Scheduler FSM state, one-hot. |
| sched_error | output | 1 | Scheduler error, sticky. |

### 15.3.4	Descriptor AXI Master Interface

Table 3.1.5: Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_ar_valid / desc_ar_ready | output / input | 1 | AR handshake. |
| desc_ar_addr | output | AW | Descriptor fetch address. |
| desc_ar_len | output | 8 | Burst length minus one. |
| desc_ar_size | output | 3 | Burst size. |
| desc_ar_burst | output | 2 | Burst type. |
| desc_ar_id | output | IW | Transaction ID. |
| desc_ar_lock, _cache, _prot, _qos, _region | output | 1, 4, 3, 4, 4 | Attribute fields. |
| desc_r_valid / desc_r_ready | input / output | 1 | R handshake. |
| desc_r_data | input | 256 | Descriptor payload, fixed 256 bits. |
| desc_r_resp | input | 2 | Read response. |
| desc_r_last | input | 1 | Last beat. |
| desc_r_id | input | IW | Response ID. |

### 15.3.5	Control Read and Control Write AXI Interfaces

The control read engine fetches a 32-bit semaphore word: ctrlrd_ar_* carries the AR channel (address, length, size, burst, id and the same attribute fields as above) and ctrlrd_r_valid, ctrlrd_r_ready, ctrlrd_r_data[31:0], ctrlrd_r_id, ctrlrd_r_resp and ctrlrd_r_last the R channel. The control write engine writes a 32-bit word: ctrlwr_aw_* carries the AW channel, ctrlwr_w_valid, ctrlwr_w_ready, ctrlwr_w_data[31:0], ctrlwr_w_strb[3:0] and ctrlwr_w_last the W channel, and ctrlwr_b_valid, ctrlwr_b_ready, ctrlwr_b_id and ctrlwr_b_resp the B channel. These are unchanged from RAPIDS Beats. See Control Read Engine and Control Write Engine.

### 15.3.6	Scheduler Data Interface

Table 3.1.6: Scheduler Data Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Read request. |
| sched_rd_addr | output | AW | Working source byte address. |
| sched_rd_beats | output | 32 | Beats still to read. |
| sched_wr_valid | output | 1 | Write request. |
| sched_wr_ready | input | 1 | Write engine reports the last burst of the descriptor committed. |
| sched_wr_addr | output | AW | Working destination byte address. |
| sched_wr_beats | output | 32 | Beats still to write. |
| sched_rd_done_strobe | input | 1 | Read burst issued. |
| sched_rd_beats_done | input | 32 | Beats in that burst. |
| sched_wr_done_strobe | input | 1 | Write AW issued. |
| sched_wr_beats_done | input | 32 | Beats in that AW. |
| sched_wr_commit_strobe | input | 1 | Write B response received. |
| sched_wr_commit_beats | input | 32 | Beats committed by that B response. |
| sched_rd_error | input | 1 | Sticky read error from the read engine. |
| sched_wr_error | input | 1 | Sticky write error, including a packet-length error from the sink path. |

### 15.3.7	Packet Record Interface

Table 3.1.7: Packet Record Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_pkt_valid | output | 1 | One-cycle pulse: a read-side packet record is available. |
| sched_rd_pkt_ready | input | 1 | Consumer has room in its record queue. |
| sched_rd_pkt_bytes | output | 32 | Packet length in bytes. |
| sched_rd_pkt_offset | output | OFF_W | Byte offset of the packet start within its first memory beat. |
| sched_wr_pkt_valid | output | 1 | One-cycle pulse: a write-side packet record is available. |
| sched_wr_pkt_ready | input | 1 | Consumer has room in its record queue. |
| sched_wr_pkt_bytes | output | 32 | Packet length in bytes. |
| sched_wr_pkt_offset | output | OFF_W | Byte offset of the packet start within its first memory beat. |

The group wires these signals directly to the scheduler. The read-side record goes to the source data path and the write-side record to the sink data path. The scheduler holds a descriptor in its fetch state while a required record queue is full, using the ready inputs. See Packet Record Interface.

### 15.3.8	MonBus Interface

Table 3.1.8: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | monbus_timestamp_t | Shared monitor timebase. |
| mon_valid | output | 1 | Monitor packet valid. |
| mon_ready | input | 1 | Consumer ready. |
| mon_packet | output | monitor_packet_t | Monitor packet. |
| mon_timestamp | output | monbus_timestamp_t | Packet timestamp. |

## 15.4	Internal Architecture

### 15.4.1	Descriptor Path

The descriptor engine fetches 256-bit descriptors from the address range the configuration allows and hands each one to the scheduler with its type, end-of-stream, end-of-list and error flags. The scheduler reports scheduler_idle back to the descriptor engine as channel_idle. No byte or beat arithmetic happens in the descriptor engine.

### 15.4.2	Control Engines

When a descriptor asks the scheduler to wait on or update a semaphore, the scheduler talks to the control engines through valid and ready handshakes with an address, data and mask. Both engines share the group’s AXI_ID_WIDTH and each raises its own MonBus packets.

### 15.4.3	MonBus Aggregation

A four-client arbiter merges the descriptor engine, scheduler, control read and control write packets. cfg_sched_compl_enable gates only the CORE Completion packets of the descriptor engine and the scheduler: with it low, those packets are consumed at the arbiter input and never reach mon_packet. Error packets are never gated. GEN_MON masks every arbiter input when clear.

## 15.5	Differences from RAPIDS Beats

Table 3.1.9: Scheduler Group Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Parameter OFF_W | absent | present, $clog2(DATA_WIDTH/8) |
| sched_rd_pkt_*, sched_wr_pkt_* ports | absent | present, wired to the scheduler |
| Everything else |  | Unchanged |

The beats chapter is Beats Scheduler Group. The scheduler is described in Scheduler.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Scheduler Group Array

Module: scheduler_group_array.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 16.1	Overview

The scheduler group array instantiates one scheduler group per channel and shares three AXI masters among them: the descriptor fetch master, the control-read master and the control-write master. It also owns the descriptor-AXI monitor and merges every channel’s MonBus output with the monitor’s into one stream.

Compared with the RAPIDS Beats array, the sharing, arbitration and monitoring are unchanged. The array adds one parameter, OFF_W, and the per-channel packet-record ports, which it connects to each group by channel index.

### 16.1.1	Key Features

- One group per channel: NUM_CHANNELS scheduler groups.

- Shared masters: three round-robin arbiters with grant acknowledgment (u_desc_ar_arbiter, u_ctrlrd_ar_arbiter, u_ctrlwr_aw_arbiter).

- Descriptor-AXI monitor: a lite AXI read monitor on the shared descriptor master, plus an always-on bus meter for the perf window.

- Unified MonBus: NUM_CHANNELS + 1 sources through one round-robin arbiter.

- Direction gating: EN_READ and EN_WRITE let the source array be read-only and the sink array write-only.

- Per-channel packet records: vector ports for the byte length and offset of each channel’s packet.

### 16.1.2	Block Diagram

### 16.1.3	Figure 3.2.1: Scheduler Group Array Block Diagram

Scheduler group array block diagram

Source: 08_scheduler_group_array.dot

## 16.2	Parameters

Table 3.2.1: Scheduler Group Array Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Number of channels. |
| CHAN_WIDTH | $clog2(NUM_CHANNELS) | Channel id width. |
| ADDR_WIDTH | 64 | Address width. |
| DATA_WIDTH | 512 | Data width in bits. |
| AXI_ID_WIDTH | 8 | AXI ID width. |
| DESC_MON_BASE_AGENT_ID | 16 | Base MonBus agent id of the descriptor engines. |
| SCHED_MON_BASE_AGENT_ID | 48 | Base MonBus agent id of the schedulers. |
| DESC_AXI_MON_AGENT_ID | 8 | MonBus agent id of the descriptor AXI monitor. |
| MON_UNIT_ID | 1 | MonBus unit id. |
| MON_MAX_TRANSACTIONS | 16 | Transaction table depth of the descriptor monitor. |
| EN_READ | 1 | Enable the read direction in every scheduler. Clear for a sink array. |
| EN_WRITE | 1 | Enable the write direction in every scheduler. Clear for a source array. |
| USE_AXI_MONITORS | 1 | 0 omits the descriptor-AXI monitor hardware and ties its MonBus source to zero. |
| USE_ROW_COL_MAJOR_ADDRESSING | 1 | 0 compiles out the extended addressing path. |
| GEN_MON | 1 | 0 omits the per-channel completion and error emitters. |
| OFF_W | $clog2(DATA_WIDTH/8) | Width of the packet-record byte offset. New in RAPIDS. |

## 16.3	Port List

Vector ports are NUM_CHANNELS wide (NC), indexed by channel.

### 16.3.1	Clock, Reset and Kick-Off

Table 3.2.2: Clock, Reset and Kick-Off

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock. |
| rst_n | input | 1 | Active-low reset. |
| apb_valid | input | NC | Per-channel kick-off. |
| apb_ready | output | NC | Per-channel ready. |
| apb_addr | input | NC x AW | First descriptor address per channel. |
| cfg_channel_enable | input | NC | Per-channel enable. |
| cfg_channel_reset | input | NC | Per-channel soft reset. |

### 16.3.2	Global Scheduler and Descriptor Engine Configuration

These inputs are applied to every channel.

Table 3.2.3: Global Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_enable | input | 1 | Master scheduler enable. |
| cfg_sched_timeout_cycles | input | 32 | Timeout window in cycles. |
| cfg_sched_timeout_limit | input | 8 | Consecutive timeouts before a fatal error. 0 means never. |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection. |
| cfg_sched_err_enable | input | 1 | Enable error reporting. |
| cfg_sched_compl_enable | input | 1 | Enable CORE Completion packets. |
| cfg_sched_perf_enable | input | 1 | Enable performance monitoring. |
| cfg_desceng_enable | input | 1 | Master descriptor engine enable. |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch. |
| cfg_desceng_fifo_thresh | input | 4 | Prefetch threshold. |
| cfg_desceng_addr0_base, _limit | input | AW | Valid descriptor address range 0. |
| cfg_desceng_addr1_base, _limit | input | AW | Valid descriptor address range 1. |
| cfg_ctrlrd_max_try | input | 9 | Control-read poll retry budget. |
| tick_1us | input | 1 | 1 us tick for control-read retry spacing. |

### 16.3.3	Descriptor Monitor Configuration and Status

Table 3.2.4: Descriptor Monitor Configuration and Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desc_mon_enable | input | 1 | Enable the descriptor AXI monitor. |
| cfg_desc_mon_err_enable | input | 1 | Enable error packets. |
| cfg_desc_mon_perf_enable | input | 1 | Enable performance packets. |
| cfg_desc_mon_perf_run | input | 1 | Perf window control. |
| cfg_desc_mon_timeout_enable | input | 1 | Enable timeout packets. |
| cfg_desc_mon_timeout_cycles | input | 32 | Timeout threshold. |
| cfg_desc_mon_latency_thresh | input | 32 | Latency threshold. |
| cfg_desc_mon_pkt_mask | input | 16 | Packet type mask. |
| cfg_desc_mon_err_select | input | 4 | Error select. |
| cfg_desc_mon_err_mask, _timeout_mask, _compl_mask, _thresh_mask, _perf_mask, _addr_mask, _debug_mask | input | 8 each | Per-class event masks. |
| cfg_sts_desc_mon_busy | output | 1 | Monitor busy. |
| cfg_sts_desc_mon_active_txns | output | 8 | Active transactions. |
| cfg_sts_desc_mon_error_count | output | 16 | Error count. |
| cfg_sts_desc_mon_txn_count | output | 32 | Transaction count. |
| cfg_sts_desc_mon_conflict_error | output | 1 | Conflict error. |

### 16.3.4	Descriptor-AXI Perf Window

The perf counters come from an always-on bus meter on the descriptor R channel, not from the monitor. cfg_desc_mon_perf_run opens the window: counters clear on its rising edge, count while it is high and hold while it is low. USE_AXI_MONITORS does not gate the meter.

Table 3.2.5: Descriptor-AXI Perf Window

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sts_desc_mon_win_active | output | 1 | Window open. |
| sts_desc_mon_win_cycles | output | 32 | Cycles in the window. |
| sts_desc_mon_prod_cycles | output | 32 | R valid and ready. |
| sts_desc_mon_bp_cycles | output | 32 | R valid, not ready. |
| sts_desc_mon_starv_cycles | output | 32 | R ready, not valid. |
| sts_desc_mon_idle_cycles | output | 32 | Neither valid nor ready. |
| sts_desc_mon_beat_count | output | 32 | R handshakes. |
| sts_desc_mon_byte_count | output | 64 | Bytes moved on R (32 per descriptor beat). |
| sts_desc_mon_burst_count | output | 32 | AR handshakes. |

### 16.3.5	Status

Table 3.2.6: Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle | output | NC | Per-channel descriptor engine idle. |
| scheduler_idle | output | NC | Per-channel scheduler idle. |
| scheduler_state | output | NC x 7 | Per-channel FSM state, one-hot. |
| sched_error | output | NC | Per-channel scheduler error, sticky. |

### 16.3.6	Shared AXI Masters

The three shared masters carry the same signals as the per-group ports in Scheduler Group, prefixed desc_axi_, ctrlrd_axi_ and ctrlwr_axi_. The descriptor master returns 256-bit data. The control read master returns 32-bit data. The control write master is 32 bits wide with a 4-bit strobe. Each is a single interface: the arbiter selects one channel’s request onto it and steers the response back by ID.

### 16.3.7	Per-Channel Scheduler Data Interface

Table 3.2.7: Per-Channel Scheduler Data Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | NC | Read request per channel. |
| sched_rd_addr | output | NC x AW | Working source byte address. |
| sched_rd_beats | output | NC x 32 | Beats still to read. |
| sched_wr_valid | output | NC | Write request per channel. |
| sched_wr_ready | input | NC | Last burst of the descriptor committed. |
| sched_wr_addr | output | NC x AW | Working destination byte address. |
| sched_wr_beats | output | NC x 32 | Beats still to write. |
| sched_rd_done_strobe | input | NC | Read burst issued. |
| sched_rd_beats_done | input | NC x 32 | Beats in that burst. |
| sched_wr_done_strobe | input | NC | Write AW issued. |
| sched_wr_beats_done | input | NC x 32 | Beats in that AW. |
| sched_wr_commit_strobe | input | NC | Write B response received. |
| sched_wr_commit_beats | input | NC x 32 | Beats committed. |
| sched_rd_error | input | NC | Sticky read error. |
| sched_wr_error | input | NC | Sticky write error. |

### 16.3.8	Per-Channel Packet Record Interface

Table 3.2.8: Per-Channel Packet Record Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_pkt_valid | output | NC | Read-side record pulse per channel. |
| sched_rd_pkt_ready | input | NC | Read-side consumer has room. |
| sched_rd_pkt_bytes | output | NC x 32 | Packet length in bytes. |
| sched_rd_pkt_offset | output | NC x OFF_W | Start offset in the first memory beat. |
| sched_wr_pkt_valid | output | NC | Write-side record pulse per channel. |
| sched_wr_pkt_ready | input | NC | Write-side consumer has room. |
| sched_wr_pkt_bytes | output | NC x 32 | Packet length in bytes. |
| sched_wr_pkt_offset | output | NC x OFF_W | Start offset in the first memory beat. |

Element [ch] of each vector connects to the matching port of scheduler_group[ch]. The array adds no logic between them. A source array uses only the read set and a sink array only the write set.

### 16.3.9	MonBus Interface

Table 3.2.9: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | monbus_timestamp_t | Shared monitor timebase. |
| mon_valid | output | 1 | Packet valid. |
| mon_ready | input | 1 | Consumer ready. |
| mon_packet | output | monitor_packet_t | Packet. |
| mon_timestamp | output | monbus_timestamp_t | Timestamp. |

## 16.4	Shared Master Arbitration

Each shared master has a round-robin arbiter over NUM_CHANNELS requesters in acknowledge mode. The grant is held until the granted channel’s request is accepted (grant_ack), so a granted channel always sees its own valid and ready. The AR or AW ready seen by a channel is the downstream ready gated by that channel’s grant.

### 16.4.1	Figure 3.2.2: Shared Master Arbitration

Shared master arbitration

Source: 11_shared_master_arbitration.dot

## 16.5	MonBus Aggregation

Table 3.2.10: MonBus Source Assignment

| Source | Origin | Content |
| --- | --- | --- |
| 0 to NC-1 | scheduler_group[ch] | Descriptor engine, scheduler, control read and control write packets of the channel |
| NC | Descriptor AXI monitor | Descriptor fetch monitor packets |

One round-robin arbiter with acknowledge mode merges the NUM_CHANNELS + 1 sources into mon_*. Each source carries its own timestamp alongside the packet.

## 16.6	Differences from RAPIDS Beats

Table 3.2.11: Array Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Parameter OFF_W | absent | present |
| sched_rd_pkt_*, sched_wr_pkt_* vectors | absent | present, wired by channel |
| Everything else |  | Unchanged |

The beats chapter is Beats Scheduler Group Array.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Sink Data Path

Module: snk_data_path.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 17.1	Overview

The sink data path joins the per-channel SRAM controller to the AXI write engine. Data enters through the fill interface, waits in the channel’s SRAM partition and leaves as AXI4 write bursts.

In RAPIDS every SRAM word carries the byte enables of its data. The fill interface widens from DW to DW + SW bits, where SW = DW/8, and the write engine drives m_axi_wstrb from the stored enables. Everything else, including the allocation protocol, the scheduler interface and the burst rules, is as in RAPIDS Beats.

### 17.1.1	Key Features

- Per-channel SRAM partitions: flow control by allocation and space-free counts.

- Byte enables stored with the data: the SRAM is DW + SW bits wide.

- AXI write engine: 4 KB-safe, beat-aligned bursts. See AXI Write Engine.

- Scheduler integration: beat counts in, issue and commit strobes out.

### 17.1.2	Figure 3.3.1: Sink Data Path Block Diagram

Sink data path block diagram

Source: 09_snk_data_path.dot

## 17.2	Parameters

Table 3.3.1: Sink Data Path Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Number of channels (alias NC). |
| ADDR_WIDTH | 64 | Address width (alias AW). |
| DATA_WIDTH | 512 | Data width in bits (alias DW). The board uses 256. |
| AXI_ID_WIDTH | 8 | AXI ID width (alias IW). |
| SRAM_DEPTH | 512 | Words per channel partition (alias SD). |
| SEG_COUNT_WIDTH | $clog2(SRAM_DEPTH) + 1 | Space counter width (alias SCW). |
| PIPELINE | 1 | SRAM read pipeline setting. |
| AW_MAX_OUTSTANDING | 8 | AW commands the write engine may have in flight. |
| W_PHASE_FIFO_DEPTH | 64 | Depth of the write engine W-phase queue. |
| B_PHASE_FIFO_DEPTH | 16 | Depth of the write engine B-phase queue. |
| CIW | $clog2(NC) | Channel id width. |
| SW | DW / 8 | Byte enables per beat. New in RAPIDS. |

## 17.3	Interfaces

### 17.3.1	Fill Interface

Table 3.3.2: Fill Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Space allocation request. |
| fill_alloc_size | input | 8 | Beats to allocate. |
| fill_alloc_id | input | CIW | Channel to allocate in. |
| fill_space_free | output | NC x SCW | Free space per channel. |
| fill_valid | input | 1 | Fill beat valid. |
| fill_ready | output | 1 | Ready for a fill beat. |
| fill_id | input | CIW | Channel of the fill beat. |
| fill_data | input | DW + SW | {byte enables, data}. Bits [DW-1:0] are data. Bits [DW+SW-1:DW] are enables. |

### 17.3.2	Configuration

Table 3.3.3: Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_wr_xfer_beats | input | 8 | Write burst size cap in beats, minus one. Renamed from cfg_axi_wr_xfer. |
| cfg_channel_reset | input | NC | Per-channel reset, level or pulse. Forwarded to the write engine and the SRAM controller wrapper. |

### 17.3.3	Scheduler Interface

Table 3.3.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | input | NC | Channel requests writes. |
| sched_wr_ready | output | NC | Last burst of a descriptor committed. |
| sched_wr_addr | input | NC x AW | Working destination byte address. |
| sched_wr_beats | input | NC x 32 | Beats still to write. |
| sched_wr_burst_len | input | NC x 8 | Requested burst length. Not used for sizing. |
| sched_wr_done_strobe | output | NC | AW issued. |
| sched_wr_beats_done | output | NC x 32 | Beats in that AW. |
| sched_wr_commit_strobe | output | NC | B response received. |
| sched_wr_commit_beats | output | NC x 32 | Beats committed by that B. |
| sched_wr_error | output | NC | Sticky write error (bad B response). |

### 17.3.4	AXI4 Write Master

The write master is m_axi_aw*, m_axi_w* and m_axi_b* with IW-bit IDs and DW-bit data. See AXI4 Interface Specification. m_axi_wstrb is DW/8 bits and carries the stored byte enables.

## 17.4	Byte Enable Path

- The upstream stage presents {strb, data} on fill_data with fill_valid.

- The SRAM controller stores the whole DW + SW word. It is instantiated with DATA_WIDTH = DW + SW.

- The drain side returns the same word as drain_data.

- drain_data[DW-1:0] goes to axi_wr_sram_data and drain_data[DW+SW-1:DW] goes to axi_wr_sram_strb on the write engine.

- The write engine drives m_axi_wdata and m_axi_wstrb from those two.

The SRAM depth counts words, so capacity in beats is unchanged. Only the width grows, by one bit per data byte.

### 17.4.1	Channel Reset

cfg_channel_reset[ch] reaches two blocks. The AXI Write Engine stops issuing for the channel, completes an open burst with null beats and clears its error flag. The SRAM controller wrapper (snk_sram_controller) holds one single-channel sram_controller per channel. Each instance’s reset is a registered flop, r_ch_rst_n[ch], which goes low one cycle after cfg_channel_reset[ch] rises, so the partition’s pointers and counters return to empty. The shared STREAM sram_controller has no channel reset of its own and is not modified; giving each channel its own instance and reset is what makes the reset per channel. A reset of one channel leaves the instances of the other channels untouched.

## 17.5	Differences from RAPIDS Beats

Table 3.3.5: Sink Data Path Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| fill_data width | DW | DW + SW |
| SRAM word width | DW | DW + SW |
| m_axi_wstrb | all ones | stored per-beat enables |
| Config port name | cfg_axi_wr_xfer | cfg_axi_wr_xfer_beats |
| Scheduler port names | sched_wr, sched_wr_done, sched_wr_commit | sched_wr_beats, sched_wr_beats_done, sched_wr_commit_beats |
| Fill allocation, drain, error export |  | Unchanged |

The beats chapter is Sink Data Path. Its timing diagrams apply to the AW, W and B sequencing unchanged.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Sink Data Path with AXIS (Ingress)

Module: snk_data_path_axis.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 18.1	Overview

This module puts an AXI-Stream slave in front of the Sink Data Path and converts a packed byte stream into beat-aligned memory beats with byte enables. It is the block that makes the sink side byte-granular.

A packet arrives with its bytes packed from lane 0. Its destination in memory starts at a byte offset inside the first memory beat. The scheduler supplies that offset in a packet record. The module shifts each stream beat up by the offset, holds the bytes that spill past the beat boundary, and merges them into the next memory beat. After the last stream beat, a non-empty spill becomes one more, partial, memory beat.

### 18.1.1	Key Features

- Barrel shift by the destination offset: stream lane l lands in memory lane offset + l.

- Byte enables follow the data: the shifted strobe is stored beside the data and becomes WSTRB.

- Per-channel state: spill hold, received-byte count and length-error flag per channel, so packets of different channels interleave by beat.

- Packet record queue: four records per channel, fed by the scheduler.

- Descriptor gating: the stream is accepted only when the channel has a packet record.

- Length check: a packet whose byte count differs from the descriptor sets a sticky channel error.

- Allocation logic unchanged from RAPIDS Beats, including the settle wait.

### 18.1.2	Figure 3.4.1: Ingress Shifter

Ingress shifter

Source: 01_ingress_shifter.dot

## 18.2	Parameters

Parameters of Sink Data Path apply unchanged. The additions are:

Table 3.4.1: Additional Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| AXIS_ID_WIDTH | 8 | tid width. The low CIW bits select the channel. |
| AXIS_DEST_WIDTH | 4 | tdest width. Not used to select a channel. |
| AXIS_USER_WIDTH | 1 | tuser width. Not used. |
| SW | DW / 8 | Byte lanes per beat. |
| OFF_W | $clog2(SW) | Width of the packet-record offset. |

## 18.3	Interfaces

### 18.3.1	AXI-Stream Slave

Table 3.4.2: AXI-Stream Slave

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tdata | input | DW | Packed data. |
| s_axis_tstrb | input | SW | Byte strobes. Contiguous from lane 0. Partial only on the tlast beat. |
| s_axis_tlast | input | 1 | Last beat of the packet. |
| s_axis_tid | input | AXIS_ID_WIDTH | Channel select. |
| s_axis_tdest | input | AXIS_DEST_WIDTH | Unused. |
| s_axis_tuser | input | AXIS_USER_WIDTH | Unused. |
| s_axis_tvalid | input | 1 | Beat valid. |
| s_axis_tready | output | 1 | Beat accepted. |

s_axis_tready is high when the output register is free, no flush beat is pending and the addressed channel has a packet record queued. A channel’s stream therefore stalls until its scheduler has fetched the descriptor. s_axis_tready is a single signal qualified by the TID of the beat on the bus, so a beat for a channel that has no record blocks the stream, and any channel’s beats behind it, until that record arrives. Channels on one stream are not isolated from each other; a system that needs isolation gives each channel its own stream or guarantees the descriptor is queued before the packet is sent. s_axis_tready is also low for a beat whose TID names a channel in reset, and high without a record for a channel that is discarding the tail of a cut packet (see Channel Reset).

### 18.3.2	Scheduler Interface

The write request and completion ports are those of Sink Data Path. The packet record ports are new:

Table 3.4.3: Packet Record Ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_pkt_valid | input | NC | Record pulse from the scheduler. |
| sched_wr_pkt_ready | output | NC | Record queue not full. |
| sched_wr_pkt_bytes | input | NC x 32 | Packet length in bytes. |
| sched_wr_pkt_offset | input | NC x OFF_W | Destination offset in the first memory beat. |
| cfg_channel_reset | input | NC | Per-channel reset, level or pulse. Clears the channel’s queue, hold, byte count and length error, and discards the tail of a cut packet. |

sched_wr_error is the OR of the write engine’s B-response error and the packet length error of the channel. Both clear on cfg_channel_reset (see Channel Reset).

### 18.3.3	Other Ports

The AXI4 write master is that of Sink Data Path. cfg_alloc_size (8 bits) is the allocation segment per request, with 0 treated as 1. The debug outputs are dbg_sram_bridge_pending, dbg_sram_bridge_out_valid, dbg_axis_beats_received and dbg_axis_packets_received. o_active_channel_id and o_active_channel_valid carry the write engine’s active channel to the per-channel bus meter.

## 18.4	Ingress Operation

### 18.4.1	Packet Records

Each channel has a four-entry queue of {offset, bytes}. A record is pushed on sched_wr_pkt_valid when the queue is not full and popped when the packet completes. The head record supplies the offset for every beat of the packet and the expected length at tlast.

### 18.4.2	Placement

For an accepted stream beat, with off the head record’s offset:

- The wide word is (tdata_m << off*8) | hold_data, of width 2*DW, where tdata_m is tdata with every lane whose tstrb bit is clear forced to zero.

- The wide strobe is (tstrb << off) | hold_strb, of width 2*SW.

- The low half goes to the output register as the memory beat and its enables.

- The high half becomes the new hold for the channel.

The strobe mask matters on a partial last beat: the non-strobed lanes carry whatever the sender left there, and unmasked they would shift into the spill hold. A packet that ends without a spill never flushes the hold, so that junk would be ORed into the first memory beat of the channel’s next packet. Masking by tstrb keeps the hold to real payload bytes.

The first memory beat has its low off strobe bits clear, so the bytes below the start address are not written.

### 18.4.3	Flush and Completion

On tlast the received-byte total is compared with the head record. If the wide strobe’s high half is non-zero, a flush is scheduled and s_axis_tready drops until the hold has been emitted as the last memory beat. The packet then completes and its record pops. If the high half is empty the packet completes on the tlast beat itself.

### 18.4.4	Length Check

r_pkt_rx_bytes counts the popcount of tstrb over the packet. At tlast, a total that differs from the record’s byte count sets the channel’s sticky error. Nothing is padded or dropped. The error is visible on sched_wr_error and the scheduler treats it as fatal. It clears on cfg_channel_reset.

### 18.4.5	Channel Reset

cfg_channel_reset[ch] is a register level in the system (SNK.CHANNEL_RESET.CH_RST ORed with GLOBAL_CTRL.GLOBAL_RST), so a pulse and a held level must both work. The ingress registers it once into r_rst_d1 and uses w_rst[ch] = cfg_channel_reset[ch] | r_rst_d1[ch], which stretches a one-cycle pulse to two cycles and lets every clear see the reset on a cycle where it is asserted. The stretch is what settles an output-register beat and an allocation that were launched on the cycle of the reset. The reset is written for one channel; every term is indexed by channel, so the other channels are not affected.

Table 3.4.4: Channel Reset Actions

| State | Action on w_rst[ch] |
| --- | --- |
| Packet record queue | Write and read pointers cleared. A record arriving on the reset cycle is not pushed. |
| Spill hold, r_pkt_rx_bytes | Cleared. |
| r_pkt_len_error | Cleared. The channel’s sched_wr_error contribution drops. |
| r_pending_alloc, r_alloc_settle | Cleared, so no allocation request of the old packet is issued. |
| Output register (r_out_valid), flush beat (r_flush_valid) | Dropped when the beat belongs to the channel, so no beat of the old packet enters the SRAM after the reset. |
| s_axis_tready | Low for a beat whose TID names a channel in reset. |
| r_discard[ch] | Set when a packet was part-way through the stream (received bytes non-zero and no flush of its own pending). |

With r_discard[ch] set the ingress accepts the channel’s beats without a record and drops them: s_axis_tready does not wait for a packet record, and no state is updated. r_discard[ch] clears on the dropped beat that carries tlast. The next beat is then an ordinary first beat of a new packet. The sender is not asked to do anything special: it finishes the packet it was sending and the tail is discarded. A packet whose beats had all been accepted (the flush beat aside) is not discarded.

Head-of-line blocking is unchanged. A beat for a channel with no record, and not discarding, still holds s_axis_tready low for every channel. After a reset the next packet for the channel waits for its descriptor as before.

The channel’s write engine and SRAM partition are cleared by the blocks that own them; see AXI Write Engine and Sink Data Path. All of them settle within a few cycles of the reset, and the scheduler’s own reset returns it to CH_IDLE, so a channel reset recovers the channel without aresetn.

### 18.4.6	Allocation

The output register holds one memory beat for the fill interface. Allocation follows the beats design:

- A channel requests cfg_alloc_size beats, or the remainder when less is free.

- The next allocation of a channel waits three cycles after its previous one so fill_space_free is current.

- A beat leaves the output register only when the fill interface accepts it.

## 18.5	Differences from RAPIDS Beats

Table 3.4.5: Ingress Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Stream beat to memory beat | one to one, full lanes | shifted by the offset, spill held |
| s_axis_tstrb | full only | partial on the tlast beat |
| Byte enables into SRAM | none | {strb, data} word |
| Packet record queue | absent | four per channel |
| Stream gating | buffers before the descriptor | waits for the packet record |
| Length mismatch | not checked | sticky channel error, cleared by channel reset |
| Channel reset | not applicable | per-channel clear, cut packet tail discarded |

The beats chapter is Sink Data Path with AXIS.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Sink SRAM Controller Specification

Module: snk_sram_controller_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 19.1	Overview

The Sink SRAM Controller is a naming wrapper, with no logic of its own, around STREAM’s sram_controller – the one per-channel SRAM implementation both DMAs share since bdf4e0dff. It maps the RAPIDS fill_*/drain_* names onto STREAM’s ports; the eight per-channel units (allocation counter, FIFO, drain counter, latency bridge) live inside STREAM’s block. It manages 8 channel units, providing per-channel buffering with channel arbitration for the shared AXI write engine.

### 19.1.1	Key Features

- One shared implementation: instantiates STREAM’s sram_controller once; it holds 8 sram_controller_units, each a gaxi_fifo_sync FIFO with stream_alloc_ctrl, stream_drain_ctrl and stream_latency_bridge

- Channel Arbitration: none inside the controller – the AXI write engine picks a channel and presents it on drain_id

- Flow Control Integration: per-channel stream_alloc_ctrl / stream_drain_ctrl virtual FIFOs; the drain counter’s virtual depth is 2 x SRAM_DEPTH (stream BUG-011)

- Space Tracking: Reports available space per channel

### 19.1.2	Block Diagram

### 19.1.3	Figure 3.5.1: Sink SRAM Controller Block Diagram

Sink SRAM controller block diagram

Source: 15_snk_sram_controller.dot

## 19.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;                  // Per-channel depth
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);

: Table 3.5.1: Sink SRAM Controller Parameters

## 19.3	Port List

### 19.3.1	Clock and Reset

Table 3.5.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 19.3.2	Fill Interface (ID-Selected)

Table 3.5.3: Fill Interface (ID-Selected)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Allocation request (single, ID-selected) |
| fill_alloc_size | input | 8 | Beats to allocate |
| fill_alloc_id | input | CIW | Transaction ID selects the channel |
| fill_space_free | output | NC x SCW | Available space per channel |
| fill_valid | input | 1 | Fill data valid |
| fill_ready | output | 1 | Ready for fill data |
| fill_id | input | CIW | Transaction ID selects the channel |
| fill_data | input | DW | Fill data |

### 19.3.3	Arbitrated Drain Interface (to AXI Write Engine)

Table 3.5.4: Arbitrated Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_data_avail | output | NC x SCW | Data available per channel |
| drain_req | input | NC | Drain reservation request per channel |
| drain_size | input | NC x 8 | Beats to reserve |
| drain_valid | output | NC | Drain valid, registered (for arbitration) |
| drain_valid_comb | output | NC | Drain valid, combinational (gates m_axi_wvalid) |
| drain_read | input | 1 | Consumer read strobe |
| drain_id | input | CIW | Channel ID select for drain |
| drain_data | output | DW | Drain data (muxed from selected channel) |

### 19.3.4	Debug Interface

Table 3.5.5: Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_bridge_pending | output | NC | Bridge has a pending beat per channel |
| dbg_bridge_out_valid | output | NC | Bridge output valid per channel |

## 19.4	Drain Selection

There is no arbiter inside the controller. Every channel unit raises its own drain_valid[ch] / drain_size[ch] when it has data; the AXI write engine picks a channel, drives it on drain_id, and the controller steers that channel’s beats to the single drain_* port. Switching channels costs nothing beyond the consumer’s own decision, and a channel keeps presenting drain_valid until it is empty.

### 19.4.1	Figure 3.5.2: Drain Selection by the Consumer

snk_sram_controller_beats - consumer drains eight channels in turn

Source: snk_sram_controller_drain_select.json, captured from dv/tests/macro_beats/test_snk_sram_controller_beats.py (multi_channel, 8 channels, 512-bit data, 512-entry SRAM, REG_LEVEL=GATE) with WAVES=1.

Reading it: all eight channels have data (drain_valid = 0xff) and the consumer walks them in turn. It raises one bit of drain_req at a time; the controller answers with drain_id naming that channel and drain_data presenting its head word on the same cycle, and the consumer’s drain_read pulse pops it. Each channel’s turn lasts nine cycles here because the test releases the request and re-arms the next one; the controller itself needs no gap between channels.

## 19.5	Integration with AXI Write Engine

snk_sram_controller_beats #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_snk_sram_ctrl (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Per-channel fill interface
    .fill_alloc_req         (fill_alloc_req),
    .fill_alloc_size        (fill_alloc_size),
    .fill_space_free        (fill_space_free),
    .fill_valid             (fill_valid),
    .fill_ready             (fill_ready),
    .fill_data              (fill_data),

    // Arbitrated drain interface
    .drain_req              (sram_drain_req),
    .drain_id               (sram_drain_id),
    .drain_size             (sram_drain_beats),

    // Status
    .drain_data_avail       (channel_data_avail),
    .dbg_bridge_pending     (channel_empty),
    .dbg_bridge_out_valid   (channel_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Source Data Path Specification

Module: src_data_path_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 20.1	Overview

The Source Data Path integrates the AXI read engine and SRAM controller for memory-to-network transfers. Data is read from system memory via AXI4, buffered in SRAM, and delivered through the drain interface to the network.

### 20.1.1	Key Features

- AXI Read Engine: Streaming burst reads from system memory

- 8-Channel SRAM Controller: Per-channel buffering with flow control

- Beat-Level Tracking: Precise flow control at beat granularity

- Scheduler Integration: Receives transfer requests from scheduler

### 20.1.2	Block Diagram

### 20.1.3	Figure 3.6.1: Source Data Path Block Diagram

Source data path block diagram

Source: 16_source_data_path.dot

## 20.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int AR_MAX_OUTSTANDING = 8;
parameter int R_PHASE_FIFO_DEPTH = 64;

: Table 3.6.1: Source Data Path Parameters

## 20.3	Data Flow

### 20.3.1	Figure 3.6.2: Source Path Data Flow Sequence

1. Scheduler Read Request
   - sched_rd_valid = 1
   - sched_rd_addr = source address
   - sched_rd_beats = N
   - sched_rd_id = channel
        |
        v
2. Space Check (stream_alloc_ctrl inside STREAM's sram_controller)
   - Check src_drain_space_free[channel] >= N
   - Allocate N beats of space
        |
        v
3. AXI Read Transaction
   - axi_read_engine issues AR
   - m_axi_araddr = sched_rd_addr
   - m_axi_arlen = calculated from beats
        |
        v
4. R Channel Data
   - Data arrives on m_axi_rdata
   - Written to channel's SRAM partition
        |
        v
5. SRAM Write Complete
   - stream_drain_ctrl notified via the unit's latency bridge
   - Data available for drain
        |
        v
6. Drain Interface
   - src_drain_valid = 1
   - Data delivered to network master
        |
        v
7. Completion
   - sched_rd_done_strobe = 1
   - Scheduler notified of completion

## 20.4	Port List

### 20.4.1	Clock and Reset

Table 3.6.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 20.4.2	Configuration

Table 3.6.3: Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | AXI read transfer size in beats (all channels) |

### 20.4.3	Scheduler Interface

Table 3.6.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | NC | Channel requests read |
| sched_rd_addr | input | NC x AW | Source addresses |
| sched_rd_beats | input | NC x 32 | Beats remaining to read |
| sched_rd_done_strobe | output | NC | Burst completed (pulsed 1 cycle) |
| sched_rd_beats_done | output | NC x 32 | Beats completed in burst |
| sched_rd_error | output | NC | Sticky error flag per channel |

### 20.4.4	AXI Read Master Interface

Table 3.6.5: AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rlast | input | 1 | Last beat |

### 20.4.5	Drain Interface (Output to Network)

Table 3.6.6: Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_data_avail | output | NC x SCW | Data available per channel |
| drain_req | input | NC | Drain reservation request per channel |
| drain_size | input | NC x 8 | Beats to reserve |
| drain_valid | output | NC | Drain data valid per channel |
| drain_valid_comb | output | NC | Combinational per-channel valid (beat gate) |
| drain_read | input | 1 | Consumer read strobe |
| drain_id | input | CIW | Channel ID select for drain |
| drain_data | output | DW | Drain data (muxed from selected channel) |

## 20.5	Timing Diagram

### 20.5.1	Figure 3.6.3: Source Path Transfer Timing

src_data_path_beats - 4-beat transfer, command to done

Source: src_data_path_transfer.json, captured inside rapids_core_beats.u_src.u_source_data_path, captured from dv/tests/top_beats/test_rapids_core_beats.py (source path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1.

Reading it: the scheduler holds sched_rd_valid[0] with 4 beats at 0x10000000. The read engine issues the AR (arlen = 3) two cycles later; the four R beats land from cycle 8, rlast on the fourth, and the cycle after it the engine pulses sched_rd_done_strobe[0] with beats_done = 4 – the scheduler advances its address to 0x10000100 on that strobe. The data reaches the source SRAM two cycles after the last R beat (drain_valid[0] rises) and the AXIS side drains it one word per drain_read pulse at the pace the egress consumer allows.

## 20.6	Integration Example

src_data_path_beats #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_source_path (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Scheduler interface
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_done_strobe   (sched_rd_done_strobe),
    .sched_rd_beats_done    (sched_rd_beats_done),
    .sched_rd_error         (sched_rd_error),

    // AXI read master
    .m_axi_arvalid          (src_axi_arvalid),
    .m_axi_arready          (src_axi_arready),
    .m_axi_araddr           (src_axi_araddr),
    .m_axi_arlen            (src_axi_arlen),
    .m_axi_rvalid           (src_axi_rvalid),
    .m_axi_rready           (src_axi_rready),
    .m_axi_rdata            (src_axi_rdata),
    .m_axi_rlast            (src_axi_rlast),

    // Drain interface
    .drain_valid            (src_drain_valid),
    .drain_data             (src_drain_data),
    .drain_id               (src_drain_id),
    .drain_data_avail       (src_data_avail)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Source Data Path with AXIS (Egress)

Module: src_data_path_axis.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 21.1	Overview

This module puts an AXI-Stream master behind the source data path. It pops beat-aligned memory beats from the SRAM and re-packs them into stream beats that start at lane 0, so a packet leaves as exactly the bytes its descriptor named.

A descriptor’s source address may start in the middle of a beat. The scheduler supplies that offset and the byte length in a packet record. The module shifts each memory beat down by the offset, holds the bytes that belong to the next stream beat, and marks the last stream beat with a contiguous strobe and tlast. One descriptor produces one AXI-Stream packet.

### 21.1.1	Key Features

- Barrel shift by the source offset: memory lane offset + l goes out on stream lane l.

- Partial last beat: tstrb is the low min(bytes_left, SW) lanes and tlast marks the final beat.

- Packets follow descriptors, not drain reservations.

- Two-stage reservation and drain, unchanged from RAPIDS Beats. The drain stage advances on memory-beat pops, not on stream handshakes.

- Per-channel state: hold, bytes-left and beats-left per channel, so reservations of different channels interleave.

- Packet record queue: four records per channel, fed by the scheduler.

### 21.1.2	Figure 3.7.1: Egress Shifter

Egress shifter

Source: 02_egress_shifter.dot

## 21.2	Parameters

Table 3.7.1: Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Channels (alias NC). |
| ADDR_WIDTH | 64 | Address width (alias AW). |
| DATA_WIDTH | 512 | Data width in bits (alias DW). The board uses 256. |
| AXI_ID_WIDTH | 8 | AXI ID width (alias IW). |
| SRAM_DEPTH | 512 | Words per channel partition. |
| SEG_COUNT_WIDTH | $clog2(SRAM_DEPTH) + 1 | Space counter width. |
| PIPELINE | 1 | SRAM read pipeline setting. |
| AR_MAX_OUTSTANDING | 8 | ARs the read engine may have in flight. |
| AXIS_ID_WIDTH | 8 | tid width. |
| AXIS_DEST_WIDTH | 4 | tdest width. |
| AXIS_USER_WIDTH | 1 | tuser width. |
| SW | DW / 8 | Byte lanes per beat. |
| OFF_W | $clog2(SW) | Width of the packet-record offset. |

## 21.3	Interfaces

### 21.3.1	Configuration

Table 3.7.2: Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | Read burst size cap in beats, minus one. |
| cfg_drain_size | input | 8 | Beats per drain reservation. 0 is treated as 1. |
| cfg_channel_reset | input | NC | Per-channel reset, level or pulse. Clears the channel’s queue, hold, egress counters, drain reservations and error flag. |

### 21.3.2	Scheduler Interface

Table 3.7.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | NC | Channel requests reads. |
| sched_rd_addr | input | NC x AW | Working source byte address. |
| sched_rd_beats | input | NC x 32 | Beats still to read. |
| sched_rd_pkt_valid | input | NC | Record pulse from the scheduler. |
| sched_rd_pkt_ready | output | NC | Record queue not full. |
| sched_rd_pkt_bytes | input | NC x 32 | Packet length in bytes. |
| sched_rd_pkt_offset | input | NC x OFF_W | Source offset in the first memory beat. |
| sched_rd_done_strobe | output | NC | AR issued. |
| sched_rd_beats_done | output | NC x 32 | Beats in that AR. |
| sched_rd_error | output | NC | Sticky read error. |

### 21.3.3	AXI-Stream Master

Table 3.7.4: AXI-Stream Master

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tdata | output | DW | Packed data. |
| m_axis_tstrb | output | SW | Contiguous from lane 0. Partial only on the tlast beat. |
| m_axis_tlast | output | 1 | Last beat of the packet. |
| m_axis_tid | output | AXIS_ID_WIDTH | Channel. |
| m_axis_tdest | output | AXIS_DEST_WIDTH | Channel. |
| m_axis_tuser | output | AXIS_USER_WIDTH | Zero. |
| m_axis_tvalid | output | 1 | Beat valid. |
| m_axis_tready | input | 1 | Consumer ready. |

### 21.3.4	Other Ports

The AXI4 read master is that of the AXI Read Engine, with IW-bit IDs and DW-bit data. The debug outputs are dbg_rd_all_complete, dbg_r_beats_rcvd, dbg_sram_writes, dbg_arb_request, dbg_sram_bridge_pending, dbg_sram_bridge_out_valid, dbg_axis_beats_sent and dbg_axis_packets_sent.

## 21.4	Egress Operation

### 21.4.1	Reservation and Drain

Unchanged from RAPIDS Beats. A round-robin decision per cycle selects a channel whose available data, less reservations still in flight, reaches cfg_drain_size beats. It may also select one whose fill is finished and has any data left. Each decision is a one-cycle drain_req of up to cfg_drain_size beats. Up to four reservations queue ahead of the drain stage, which loads the next entry on the last beat of the current one.

### 21.4.2	Pop

A memory beat is popped when all of these hold:

- The drain stage is active on channel c and has a beat.

- The output register is free or being taken.

- No flush is pending.

- The channel’s packet record is known.

Each pop counts against the reservation block, so the drain stage advances on pops.

### 21.4.3	Shift and Hold

With off the head record’s offset:

Table 3.7.5: Egress Shift Rules

| Situation | Action |
| --- | --- |
| off = 0 | Every pop emits a stream beat. |
| off > 0, first pop | The pop only primes the hold with beat >> off*8. It emits only when the packet fits in that single memory beat. |
| off > 0, later pops | Emit hold ¦ (beat << (SW - off)*8). The new hold is beat >> off*8. |
| Last memory beat popped, bytes left | The flush emits the hold alone. |

The memory beat count of a packet is (off + bytes + SW - 1) >> OFF_W, computed in 33 bits.

### 21.4.4	Strobe and Last

Each emitted beat carries n = min(bytes_left, SW) bytes. tstrb is the low n lanes. tlast is set on the beat where bytes_left equals n. That beat also retires the packet record and clears the channel’s hold, started flag and counters.

### 21.4.5	Waveform 3.7: Egress Re-pack

Egress re-pack

Source: 01_egress_repack.json

Offset 5, 60 bytes and a 256-bit beat give three memory beats. The first pop, M0, only primes the hold. The second pop emits the first stream beat with 32 bytes. The third pop emits the last beat with 28 bytes, tstrb 0FFFFFFF and tlast.

## 21.5	Channel Reset

cfg_channel_reset[ch] is a register level, so a pulse and a held level must both work. As in the sink ingress the data path forms w_rst[ch] = cfg_channel_reset[ch] | r_rst_d1[ch], a two-cycle stretch of a pulse. The read engine clears its error flag and settles the reads in flight (see AXI Read Engine); the data path clears everything that would otherwise let the old descriptor keep streaming:

Table 3.7.6: Channel Reset Actions

| State | Action on w_rst[ch] |
| --- | --- |
| Packet record queue | Pointers cleared. A record arriving on the reset cycle is not pushed. |
| Egress state | r_eg_started, r_hold_valid, r_hold_data, r_eg_bytes_left and r_eg_mem_left cleared. |
| Drain reservation | Grants for the channel are masked. r_drain_tminus1[ch] is cleared. |
| Reservation queue | Entries that belong to the channel are zeroed. An entry of size 0 loads the drain stage idle, so the drain stage skips it. A reservation being drained for the channel is dropped. |
| Output register | A beat already in r_out completes, so m_axis stays stable. |

The channel’s SRAM partition is cleared by src_sram_controller, which holds one single-channel sram_controller per channel with a registered per-channel reset r_ch_rst_n[ch]. The shared STREAM controller has no channel reset of its own and is not modified. Reads of the old descriptor that are already on the bus are discarded by the read engine, so no stale data reaches the cleared partition.

A packet cut by the reset is left unterminated. If the reset lands after the first beat of a packet has gone out and before its tlast beat, the data path does not send a tlast and does not send a null terminator. The downstream receiver sees the packet stop. A packet that had not started, or whose tlast beat was already in r_out when the reset hit, is unaffected. A receiver that needs to resynchronize after a channel reset treats the reset as the end of the packet.

Other channels are unaffected: every term above is indexed by channel, the drain and reservation stages skip only the channel in reset, and their beats keep flowing through the shared m_axis.

## 21.6	Differences from RAPIDS Beats

Table 3.7.7: Egress Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Stream beat from memory beat | one to one, full lanes | shifted by the offset, hold merged |
| m_axis_tstrb | all ones | contiguous, partial on the last beat |
| m_axis_tlast | on the drain block boundary | on the last byte of the descriptor |
| Drain advance | stream handshakes | memory-beat pops |
| Packet framing | one per drain block | one per descriptor |
| Packet record queue | absent | four per channel |
| Channel reset | not applicable | per-channel clear of every stage, cut packet left unterminated |
| Reservation stage, allocation of drain |  | Unchanged |

The beats chapter is Source Data Path with AXIS.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Source SRAM Controller Specification

Module: src_sram_controller_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 22.1	Overview

The Source SRAM Controller is a naming wrapper, with no logic of its own, around STREAM’s sram_controller – the one per-channel SRAM implementation both DMAs share since bdf4e0dff. It maps the RAPIDS fill_*/drain_* names onto STREAM’s ports; the eight per-channel units (allocation counter, FIFO, drain counter, latency bridge) live inside STREAM’s block. It manages 8 channel units for the source data path, providing per-channel buffering with channel arbitration for data delivery to the network.

### 22.1.1	Key Features

- One shared implementation: instantiates STREAM’s sram_controller once; it holds 8 sram_controller_units, each a gaxi_fifo_sync FIFO with stream_alloc_ctrl, stream_drain_ctrl and stream_latency_bridge

- Channel Arbitration: none inside the controller – the AXIS egress arbiter picks a channel and presents it on drain_id

- Flow Control Integration: per-channel stream_alloc_ctrl / stream_drain_ctrl virtual FIFOs; the drain counter’s virtual depth is 2 x SRAM_DEPTH (stream BUG-011)

- Data Availability Tracking: Reports available data per channel

### 22.1.2	Block Diagram

### 22.1.3	Figure 3.8.1: Source SRAM Controller Block Diagram

Source SRAM controller block diagram

Source: 18_src_sram_controller.dot

## 22.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;                  // Per-channel depth
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);

: Table 3.8.1: Source SRAM Controller Parameters

## 22.3	Port List

### 22.3.1	Clock and Reset

Table 3.8.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 22.3.2	Fill Interface (ID-Selected, from AXI Read Engine)

Table 3.8.3: Fill Interface (ID-Selected)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Allocation request (single, ID-selected) |
| fill_alloc_size | input | 8 | Beats to allocate |
| fill_alloc_id | input | CIW | Transaction ID selects the channel |
| fill_space_free | output | NC x SCW | Available space per channel |
| fill_valid | input | 1 | Fill data valid |
| fill_ready | output | 1 | Ready for fill data |
| fill_id | input | CIW | Transaction ID selects the channel |
| fill_data | input | DW | Fill data |

### 22.3.3	Arbitrated Drain Interface (to Network)

Table 3.8.4: Arbitrated Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_data_avail | output | NC x SCW | Data available per channel |
| drain_req | input | NC | Drain reservation request per channel |
| drain_size | input | NC x 8 | Beats to reserve |
| drain_valid | output | NC | Drain data valid per channel |
| drain_valid_comb | output | NC | Combinational per-channel valid (beat gate) |
| drain_read | input | 1 | Consumer read strobe |
| drain_id | input | CIW | Channel ID select for drain |
| drain_data | output | DW | Drain data (muxed from selected channel) |

### 22.3.4	Debug Interface

Table 3.8.5: Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_bridge_pending | output | NC | Bridge has a pending beat per channel |
| dbg_bridge_out_valid | output | NC | Bridge output valid per channel |

## 22.4	Drain Selection

There is no arbiter inside the controller. Every channel unit raises its own drain_valid[ch] / drain_size[ch] when it has data; the AXIS egress arbiter picks a channel, drives it on drain_id, and the controller steers that channel’s beats to the single drain_* port. Switching channels costs nothing beyond the consumer’s own decision, and a channel keeps presenting drain_valid until it is empty.

### 22.4.1	Figure 3.8.2: Drain Selection by the Consumer

src_sram_controller_beats - consumer drains eight channels in turn

Source: src_sram_controller_drain_select.json, captured from dv/tests/macro_beats/test_src_sram_controller_beats.py (multi_channel, 8 channels, 512-bit data, 512-entry SRAM, REG_LEVEL=GATE) with WAVES=1.

Reading it: the same selection contract as the sink controller (Figure 3.5.2). With drain_valid = 0xff the consumer asks for one channel at a time through drain_req; drain_id and drain_data follow the request on the same cycle and drain_read pops the word. The nine-cycle turn per channel is the test’s pacing, not the controller’s.

## 22.5	Comparison: Sink vs Source SRAM Controller

Table 3.8.6: Sink vs Source Controller Comparison

| Aspect | Sink Controller | Source Controller |
| --- | --- | --- |
| Fill Source | Network ingress | AXI read data |
| Drain Destination | AXI write engine | Network egress |
| Fill Interface | Per-channel from network | Per-channel from AXI R |
| Drain Interface | Arbitrated SRAM read | Arbitrated drain to network |
| Primary Use | Network -> Memory | Memory -> Network |

## 22.6	Integration Example

src_sram_controller_beats #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_src_sram_ctrl (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Per-channel fill interface (from AXI read engine)
    .fill_alloc_req         (fill_alloc_req),
    .fill_alloc_size        (fill_alloc_size),
    .fill_space_free        (fill_space_free),
    .fill_valid             (fill_valid),
    .fill_ready             (fill_ready),
    .fill_data              (fill_data),

    // Arbitrated drain interface (to network)
    .drain_valid            (src_drain_valid),
    .drain_data             (src_drain_data),
    .drain_id               (src_drain_id),

    // Status
    .drain_data_avail       (channel_data_avail),
    .dbg_bridge_pending     (channel_empty),
    .dbg_bridge_out_valid   (channel_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	RAPIDS Core, Sink Half and Source Half

Module: rapids_core.sv, rapids_snk.sv, rapids_src.sv Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 23.1	Overview

rapids_core is a thin structural wrapper over two independent halves. rapids_src moves memory to the network. rapids_snk moves the network to memory. The halves share no logic and no signals. The core only presents one boundary and merges the two MonBus streams.

The structure, the port naming convention and the register and monitor plumbing are those of RAPIDS Beats. This chapter records what the byte-granular RTL contains and how it differs. The full port tables are in RAPIDS Core Beats, with the port renames below applied.

### 23.1.1	Figure 3.9.1: Core Block Diagram

Core block diagram

Source: 04_rapids_core.dot

## 23.2	Byte-Granular Content

### 23.2.1	Packet Record Nets

Each half declares the packet record nets of its direction and connects them between its scheduler group array and its AXIS data path:

Table 3.9.1: Packet Record Nets per Half

| Half | Nets | Producer | Consumer |
| --- | --- | --- | --- |
| rapids_src | sched_rd_pkt_valid, _ready, _bytes, _offset | array read set | src_data_path_axis |
| rapids_snk | sched_wr_pkt_valid, _ready, _bytes, _offset | array write set | snk_data_path_axis |

The offset nets are $clog2(DW/8) bits wide. The unused direction of each array is tied off: valid, bytes and offset are left open and ready is held at all ones so nothing stalls. The record nets are internal, so the core port list has no packet record ports.

### 23.2.2	Port Renames

The core ports that carry the beat-count unit are renamed. Everything else keeps its Beats name.

Table 3.9.2: Core Port Renames

| RAPIDS Beats | RAPIDS |
| --- | --- |
| cfg_axi_rd_xfer | cfg_axi_rd_xfer_beats |
| cfg_axi_wr_xfer | cfg_axi_wr_xfer_beats |
| src_dbg_r_rcvd | src_dbg_r_beats_rcvd |
| src_dbg_axis_sent | src_dbg_axis_beats_sent |
| snk_dbg_axis_received | snk_dbg_axis_beats_received |

### 23.2.3	Data Ports

Table 3.9.3: Data Ports

| Interface | Signals | Notes |
| --- | --- | --- |
| Source AXIS | m_axis_* | tstrb is contiguous, partial on the tlast beat. |
| Sink AXIS | s_axis_* | tid selects the channel. Partial tstrb only on tlast. |
| Source AXI read | m_axi_rd_* | ARADDR is beat-aligned, bursts stop at 4 KB. |
| Sink AXI write | m_axi_wr_* | wstrb carries the stored byte enables. |

## 23.3	Parameters

Table 3.9.4: Core Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Channels. |
| ADDR_WIDTH | 64 | Address width. |
| DATA_WIDTH | 512 | Data width in bits. The board uses 256. |
| AXI_ID_WIDTH | 8 | AXI ID width. |
| SRAM_DEPTH | 512 | Words per channel. |
| PIPELINE | 1 | Write engine pipelining. |
| AR_MAX_OUTSTANDING, AW_MAX_OUTSTANDING | 8 | Outstanding bursts. |
| W_PHASE_FIFO_DEPTH, B_PHASE_FIFO_DEPTH | 64, 16 | Write engine queues. |
| AXIS_ID_WIDTH, AXIS_DEST_WIDTH, AXIS_USER_WIDTH | 8, 4, 1 | AXIS side widths. |
| DESC_MON_BASE_AGENT_ID | 16 | Descriptor engine agents. |
| SCHED_MON_BASE_AGENT_ID | 48 | Scheduler agents. |
| DESC_AXI_MON_AGENT_ID | 8 | Descriptor AXI monitor. |
| SNK_AXIS_MON_AGENT_ID | 9 | Sink ingress AXIS monitor. |
| SRC_AXIS_MON_AGENT_ID | 10 | Source egress AXIS monitor. |
| ACLK_MHZ | 100 | Clock in MHz for the AXIS monitors’ microsecond tick. |
| MON_UNIT_ID, MON_MAX_TRANSACTIONS | 1, 16 | Monitor unit id and table depth. |
| USE_AXI_MONITORS | 1 | 0 omits the monitor hardware. |
| USE_ROW_COL_MAJOR_ADDRESSING | 1 | 0 compiles out extended addressing. |
| GEN_MON | 1 | 0 omits the completion and error emitters. |

### 23.3.1	Channel Reset Routing

Each half has its own configuration block, and that block drives the half’s per-channel reset from its CHANNEL_RESET.CH_RST field ORed with GLOBAL_CTRL.GLOBAL_RST. The core exposes it as src_cfg_channel_reset and snk_cfg_channel_reset, and each half forwards it twice: to its scheduler group array (scheduler, descriptor engine, control engines) and to its byte data path (src_data_path_axis or snk_data_path_axis, through which it reaches the AXI engine and the SRAM wrapper). A reset therefore reaches every block that holds a sticky flag or an in-flight beat of the channel, and a channel reset recovers every error class without aresetn. Table 3.9.5 lists the blocks reached.

Table 3.9.5: Channel Reset Routing

| Half | Blocks reached | Chapter |
| --- | --- | --- |
| rapids_snk | scheduler, axi_write_engine, snk_sram_controller, snk_data_path_axis | Sink Data Path AXIS |
| rapids_src | scheduler, axi_read_engine, src_sram_controller, src_data_path_axis | Source Data Path AXIS |

The two halves are independent: a reset of a sink channel does not touch the source channel with the same index, so software resets both directions of a channel by writing both halves’ reset fields. A reset of one channel never disturbs another channel of the same half.

## 23.4	Differences from RAPIDS Beats

Table 3.9.6: Core Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Half structure, monitor merge |  | Unchanged |
| Packet record nets inside each half | absent | present |
| Data path instances | *_axis_beats | byte-granular *_axis |
| Beat-unit port names | no suffix | _beats suffix, see Table 3.9.2 |

The Beats chapters for the halves are folded into the core chapter there.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	RAPIDS Registers Specification

Module: rapids_regs.sv (generated) Source: projects/components/dma-ip/rapids/rtl/macro_beats/rapids_regs.rdl, rapids_mon_regs.rdl Generated: projects/components/dma-ip/rapids/regs/generated/rtl/rapids_regs.sv Status: Implemented (PeakRDL-generated)

## 24.1	Overview

rapids_regs is the PeakRDL-generated register block for the RAPIDS Beats top-level. It exposes configuration and status through a single command/response CPU interface and drives the hardware via a hwif_out structure that rapids_config_block maps onto the core/monitor cfg_* signals.

A single addrmap (one APB slave) contains two regfiles:

- Base config/status registers at 0x100-0x3FF.

- A nested rapids_mon_regs monitor regfile at base 0x1000, decoded under hwif_out.MON.* (all DAXMON / RDMON / WRMON / MON_FIFO and per-monitor performance registers).

Because the monitor regfile lives at 0x1000, the register-block address decode requires at least 13 bits (cpuif_addr is a 13-bit compare in the generated RTL). The APB address bus feeding the block (APB_ADDR_WIDTH) must therefore be >= 13 to reach the monitor regfile.

## 24.2	Base Register Map (0x100-0x3FF)

Table 3.10.1: Base Register Map

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | RW | GLOBAL_EN[0], GLOBAL_RST[1] (self-clearing) |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE[0] (all channels idle) |
| 0x108 | VERSION | RO | MINOR[7:0]=0x5A, MAJOR[15:8], NUM_CHANNELS[23:16]=0x08 |
| 0x120 | CHANNEL_ENABLE | RW | CH_EN[7:0] per-channel enable |
| 0x124 | CHANNEL_RESET | RW | CH_RST[7:0] per-channel reset (self-clearing) |
| 0x140 | CHANNEL_IDLE | RO | CH_IDLE[7:0] per-channel idle |
| 0x144 | DESC_ENGINE_IDLE | RO | Per-channel descriptor-engine idle |
| 0x148 | SCHEDULER_IDLE | RO | Per-channel scheduler idle (excludes CH_ERROR) |
| 0x150-0x16C | CH_STATE[0..7].STATE | RO | Per-channel FSM state (stride 0x4) |
| 0x170 | SCHED_ERROR | RO | Per-channel sticky scheduler error |
| 0x174 | AXI_RD_COMPLETE | RO | Per-channel read-complete flags |
| 0x178 | AXI_WR_COMPLETE | RO | Per-channel write-complete flags |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | TIMEOUT_CYCLES[31:0] write-progress window (reset 1000) |
| 0x204 | SCHED_CONFIG | RW | SCHED_EN, TIMEOUT_EN, ERR_EN, COMPL_EN, PERF_EN |
| 0x208 | SCHED_TIMEOUT_LIMIT | RW | LIMIT[7:0] consecutive-timeout escalation limit (reset 4, 0 = never) |
| 0x220 | DESCENG_CONFIG | RW | DESCENG_EN[0], PREFETCH_EN[1], FIFO_THRESH[5:2] (reset 8) |
| 0x224 | DESCENG_ADDR0_BASE | RW | Descriptor address range 0 base [31:0] |
| 0x228 | DESCENG_ADDR0_LIMIT | RW | Descriptor address range 0 limit [31:0] |
| 0x22C | DESCENG_ADDR1_BASE | RW | Descriptor address range 1 base [31:0] |
| 0x230 | DESCENG_ADDR1_LIMIT | RW | Descriptor address range 1 limit [31:0] |
| 0x240 | CTRL_CONFIG | RW | CTRLRD_MAX_TRY[8:0] control-read poll retry budget (0-511, reset 16) |
| 0x2A0 | AXI_XFER_CONFIG | RW | AXI transfer sizing configuration |
| 0x2B0 | PERF_CONFIG | RW | Performance profiler configuration |
| 0x2C0 | OBS_CTRL | RW | Observation mux control (channel/category select) |
| 0x2C4 | OBS_FLAGS | RO | Observation flags |
| 0x2C8 | OBS_DATA0 | RO | Observation data word 0 |
| 0x2CC | OBS_DATA1 | RO | Observation data word 1 |
| 0x35C | PERF_CH_SEL | RW | Performance channel select |
| 0x378 | HIST_SEL | RW | Histogram bucket select |
| 0x37C | HIST_DATA | RO | Histogram bucket data |
| 0x380 | HIST_TOTAL | RO | Histogram total count |

### 24.2.1	Key Register Fields

SCHED_TIMEOUT_LIMIT (0x208) – LIMIT[7:0] sets the number of consecutive write-progress timeout windows a channel tolerates before the recoverable timeout escalates to a fatal, sticky CH_ERROR. 0 = never escalate (pure soft timeout). Total time to escalate is approximately LIMIT x TIMEOUT_CYCLES. Reset value is 4.

DESCENG_CONFIG (0x220) – PREFETCH_EN[1] and FIFO_THRESH[5:2] control the now-functional descriptor prefetch: prefetch off = on-demand chaining (~1 ahead), prefetch on = buffer up to FIFO_THRESH descriptors ahead.

CTRL_CONFIG (0x240) – CTRLRD_MAX_TRY[8:0] bounds the control-read poll retry budget (0-511, reset 16) fed to every channel’s ctrlrd_engine. A CTRL_READ descriptor polls its gate address once per retry; if the masked value never matches within CTRLRD_MAX_TRY attempts the engine raises ctrlrd_error so a never-satisfied gate cannot hang the channel. See the Control-Read / Control-Write Engine specs (Sections 2.8 / 2.9) and the Scheduler control interface (Section 2.1).

## 24.3	Monitor Register Map (rapids_mon_regs @ 0x1000)

The monitor regfile is instantiated at base 0x1000 under hwif_out.MON.*. It holds the MonBus FIFO status, the three AXI-monitor configuration groups (DAXMON = descriptor monitor, RDMON = read monitor, WRMON = write monitor), and per-monitor performance counters.

Table 3.10.2: Monitor Register Map (base 0x1000)

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x1000 | MON.MON_FIFO_STATUS | RO | MonBus capture/error FIFO status |
| 0x1004 | MON.MON_FIFO_COUNT | RO | MonBus FIFO occupancy |
| 0x10C0 | MON.DAXMON_ENABLE | RW | Descriptor-monitor enables (incl. COMPRESS_EN) |
| 0x10C4 | MON.DAXMON_TIMEOUT | RW | Descriptor-monitor timeout cycles |
| 0x10C8 | MON.DAXMON_LATENCY_THRESH | RW | Descriptor-monitor latency threshold |
| 0x10CC | MON.DAXMON_PKT_MASK | RW | Descriptor-monitor packet mask |
| 0x10D0 | MON.DAXMON_ERR_CFG | RW | Descriptor-monitor error select/mask |
| 0x10D4-0x10DC | MON.DAXMON_MASK1/2/3 | RW | Descriptor-monitor category masks |
| 0x10E0-0x10FC | MON.RDMON_* | RW | Source-egress AXIS monitor-lite config (same layout as DAXMON; SRC instance, rapids TASK-015) |
| 0x1100-0x111C | MON.WRMON_* | RW | Sink-ingress AXIS monitor-lite config (same layout as DAXMON; SNK instance, rapids TASK-015) |
| 0x1150-0x1178 | MON.DAXMON_PERF_* | RO/RW | Descriptor-monitor performance counters |
| 0x1180-0x11A8 | MON.RDMON_PERF_* | RO/RW | Read-monitor performance counters |
| 0x11B0-0x11D8 | MON.WRMON_PERF_* | RO/RW | Write-monitor performance counters |
| 0x11E0-0x11EC | MON.{RD,WR}MON_PERF_CH_* | RO | Per-channel producer/backpressure/starve/idle |
| 0x11F0 | MON.RDMON_PERF_CH_OVERFLOW | RO | Read-monitor per-channel overflow |
| 0x11F4 | MON.WRMON_PERF_CH_OVERFLOW | RO | Write-monitor per-channel overflow |

Each *_ENABLE register carries MON_EN, ERR_EN, COMPL_EN, TIMEOUT_EN (and COMPRESS_EN on the write monitor); each *_PERF_* group provides window, producer, backpressure, starve, idle, beat, byte (lo/hi), and burst counters.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	RAPIDS Config Block Specification

Module: rapids_config_block.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented

## 25.1	Overview

rapids_config_block is a combinational adapter that maps the register block’s hwif_out structure onto the flat cfg_* signals consumed by rapids_core_beats and the AXI monitors. It contains no state; every output is a direct assignment (with a small number of width expansions and global-enable gates).

The monitor registers arrive under hwif_out.MON.*; base configuration and status arrive at the top level of hwif_out.

## 25.2	Mapping Groups

### 25.2.1	Base Configuration

Routes scheduler, channel, and descriptor-engine configuration to the core:

- cfg_channel_enable = CHANNEL_ENABLE.CH_EN & {8{GLOBAL_CTRL.GLOBAL_EN}} (per-channel enable gated by the global master enable).

- cfg_sched_timeout_cycles = SCHED_TIMEOUT_CYCLES.TIMEOUT_CYCLES (32-bit).

- cfg_sched_timeout_limit  = SCHED_TIMEOUT_LIMIT.LIMIT (8-bit, direct).

- cfg_sched_{timeout,err,compl,perf}_enable = SCHED_CONFIG.{TIMEOUT,ERR,COMPL,PERF}_EN. COMPL_EN is the one the scheduler group acts on for its monitor packets: 0 drops the scheduler’s and descriptor engine’s CORE Completion packets at the group (emitters still acknowledged); ERR_EN and PERF_EN reach the group but gate nothing today (errors always reported, perf not implemented).

- cfg_desceng_* from DESCENG_CONFIG (enable / prefetch / FIFO threshold).

- Descriptor address ranges (cfg_desceng_addr{0,1}_{base,limit}) are zero-extended from the 32-bit register fields to the 64-bit ADDR_WIDTH.

### 25.2.2	Monitor Configuration (hwif_out.MON.*)

Three parallel groups map to the descriptor-read monitor and, since rapids TASK-015, the two AXIS data-path monitor-lites (one per half):

Table 3.11.1: Monitor Register-to-Config Mapping

| Register group | cfg_* target |
| --- | --- |
| MON.DAXMON_* | cfg_desc_mon_* (descriptor monitor) |
| MON.RDMON_* | cfg_rdeng_mon_* (source-egress AXIS monitor-lite; wired in the SRC half only) |
| MON.WRMON_* | cfg_wreng_mon_* (sink-ingress AXIS monitor-lite; wired in the SNK half only) |

The *_ENABLE bits map onto the AXIS monitor-lite’s class enables: MON_EN (gated by GLOBAL_EN), ERR_EN (error, channel-change and TSTRB checks), COMPL_EN (one Completion/STREAM_END per packet), PERF_EN (stream start/pause/resume and Credit/BACKPRESSURE stall reports), TIMEOUT_EN. *_TIMEOUT is MICROSECONDS in its low 16 bits (0 = never), *_LATENCY_THRESH the stall CYCLES that raise a backpressure report, and *_PKT_MASK is a MASK: bit[type] = 1 masks (drops) that packet type at the monitor-lite and again at the monbus group. It resets to all-masked (0xFFFF); the host clears the bits of the classes it wants to see (rapids BUG-008 corrected the description, which used to read “1 = enable”; the polarity and the reset are as intended).

Within each group the enable/timeout/latency/mask fields map 1:1, e.g. cfg_desc_mon_enable = MON.DAXMON_ENABLE.MON_EN & GLOBAL_CTRL.GLOBAL_EN, cfg_desc_mon_timeout_cycles = MON.DAXMON_TIMEOUT.TIMEOUT_CYCLES, and the category masks (timeout/compl/thresh/perf/addr/debug) pass through directly. Key enables are gated by GLOBAL_CTRL.GLOBAL_EN; masks pass through ungated.

### 25.2.3	Performance and Observation

PERF_CONFIG, PERF_CH_SEL, and OBS_CTRL map to the profiler and channel-observation-mux cfg_* signals.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	RAPIDS Top

Module: rapids_top.sv Location: projects/components/dma-ip/rapids/rtl/top/ Status: Implemented Last Updated: 2026-09-30

## 26.1	Overview

rapids_top is the synthesizable top level. It wraps RAPIDS Core with an APB4 register interface, two config blocks, the register file, per-channel bus meters and the MonBus AXI-Lite group. It has the same hierarchy as RAPIDS Beats. Only the beat-count unit names differ.

The byte-granular behavior is entirely below the top: the packet record nets are internal to the core halves, so the top port list has no new signals. The full port list is in Top-Level Port List and the top integration description is in RAPIDS Beats Top.

### 26.1.1	Figure 3.12.1: Top Hierarchy

RAPIDS top integration

Source: 10_rapids_top.dot

## 26.2	APB Address Map

The APB address is 13 bits. Bit 12 selects the half.

Table 3.12.1: APB Address Map

| Range | Content |
| --- | --- |
| 0x0000 to 0x003F | Source staged descriptor addresses. |
| 0x0040 | Source kick-off, one single-pulse bit per channel. |
| 0x0040 to 0x0FFF | Source configuration registers. |
| 0x1000 to 0x103F | Sink staged descriptor addresses. |
| 0x1040 | Sink kick-off, one single-pulse bit per channel. |
| 0x1040 to 0x1FFF | Sink configuration registers. |

The monitor register windows sit at +0x800 of each half’s 4 KB space. With USE_MON_REGS clear the windows answer with an APB error, so a host can tell a build without monitors from a zero setting.

## 26.3	Parameters

Table 3.12.2: Top Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Channels. |
| DATA_WIDTH | 512 | Data width in bits. The board uses 256. |
| ADDR_WIDTH | 64 | Address width. |
| AXI_ID_WIDTH | 8 | AXI ID width. |
| SRAM_DEPTH | 4096 | Words per channel at the top. |
| APB_ADDR_WIDTH, APB_DATA_WIDTH | 13, 32 | APB widths. |
| MON_MAX_TRANSACTIONS | 16 | Monitor table depth. |
| SNK_AXIS_MON_AGENT_ID, SRC_AXIS_MON_AGENT_ID | 9, 10 | AXIS monitor agent ids. |
| ACLK_MHZ | 100 | Clock in MHz for the AXIS monitor tick. |
| USE_AXI_MONITORS | 1 | 0 omits the monitors and the MonBus AXI-Lite group. |
| USE_ROW_COL_MAJOR_ADDRESSING | 1 | 0 compiles out extended addressing. |
| GEN_MON | 1 | 0 omits the completion and error emitters. |
| USE_MON_REGS | USE_AXI_MONITORS != 0 | Wire the monitor register windows. |
| AR_MAX_OUTSTANDING, AW_MAX_OUTSTANDING | 8 | Outstanding bursts. |
| PIPELINE | 1 | Write engine pipelining. |
| AXIS_ID_WIDTH, AXIS_DEST_WIDTH, AXIS_USER_WIDTH | 8, 4, 1 | AXIS side widths. |

Clock and reset are aclk and aresetn. cam_clear synchronously clears the MonBus group transaction tables.

Each half’s configuration block drives its per-channel reset vector (src_cfg_channel_reset, snk_cfg_channel_reset) from SRC/SNK.CHANNEL_RESET.CH_RST and GLOBAL_CTRL.GLOBAL_RST. The top wires each vector to its half of the core (see RAPIDS Core), and it reaches every block that can hold a channel error: a channel reset recovers every error class, including the AXI response and sink packet length errors, and aresetn is needed only at power-up. The top has no port change.

## 26.4	Differences from RAPIDS Beats

Table 3.12.3: Top Delta

| Item | RAPIDS Beats | RAPIDS |
| --- | --- | --- |
| Module | rapids_beats_top | rapids_top |
| Core instance | rapids_core_beats | rapids_core |
| Register file, APB map, config blocks |  | Unchanged |
| Bus meter beat counters | r_rd, r_wr | r_rd_beats, r_wr_beats |
| Config nets | *_xfer | *_xfer_beats |
| Port list |  | Unchanged |

The register-file field names RD_XFER_BEATS and WR_XFER_BEATS are the same in both designs. Their meaning is beats in both, since bursts are beat-based.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	AXI4 Interface Specification

Module: axi_read_engine.sv (source AXI read master), axi_write_engine.sv (sink AXI write master) Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2026-09-30

## 27.1	Overview

RAPIDS drives two AXI4 masters: a read master on the source half and a write master on the sink half. The channel signals, the outstanding-transaction limits and the response handling are those of RAPIDS Beats and are specified in AXI4 Interface Specification (shared with RAPIDS Beats). This chapter records only what the byte-granular design adds.

The AXI side stays beat-oriented. Every burst starts at a beat-aligned address and moves whole beats. A descriptor that starts or ends inside a beat is handled by the byte lanes of the data path, not by narrow AXI transfers. Both engines are specified in AXI Read Engine and AXI Write Engine.

## 27.2	Byte Address to Beat Mapping

The scheduler splits a byte address into a beat address and a byte offset. OFF_W is $clog2(DATA_WIDTH/8), which is 5 for the 256-bit board build.

Table 4.1.1: Address Split

| Quantity | Value |
| --- | --- |
| Offset | addr[OFF_W-1:0] |
| First beat address | addr with the low OFF_W bits cleared |
| Beats for a descriptor | (offset + length + lanes - 1) >> OFF_W, zero for zero length |

The engines receive the working byte address and the remaining beat count. ARADDR and AWADDR are driven with the low OFF_W bits at zero, so every burst is beat-aligned.

## 27.3	Burst Rules

Table 4.1.2: Burst Rules

| Rule | Read master | Write master |
| --- | --- | --- |
| Burst start | beat-aligned | beat-aligned |
| Burst size | full bus width | full bus width |
| 4 KB boundary | no burst crosses one | no burst crosses one |
| Burst cap | smaller of the configured burst and the beats left to the 4 KB boundary | same |

The number of beats left to the boundary is computed from the beat-aligned address, so a byte offset never lets a burst run past it.

## 27.4	Write Strobes

The write master carries a byte enable with every beat. The data path stores the strobe next to the data in the sink SRAM, and the write engine drives it onto WSTRB. Interior beats of a packet are fully strobed. The first beat and the last beat of a descriptor that start or end inside a beat carry a partial strobe, so only the bytes of the descriptor are written and neighbouring bytes in memory are untouched.

The read master has no byte enables. It always reads whole beats and the source data path discards the bytes outside the descriptor.

Table 4.1.3: Byte-Granular Write Signal

| Signal | Width | Meaning |
| --- | --- | --- |
| m_axi_wstrb | DATA_WIDTH/8 | Byte lanes written by the beat, from the stored strobe. |

## 27.5	Limitations

- TYPE=EXT descriptors remain beat-aligned in address and length. This is a permanent limitation.

- A descriptor never issues a narrow AXI transfer. Sub-beat behavior is entirely in the strobes.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	AXI-Stream Interface Specification

Module: src_data_path_axis.sv (source master), snk_data_path_axis.sv (sink slave) Location: projects/components/dma-ip/rapids/rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 28.1	Overview

RAPIDS has one AXI-Stream master on the source half and one AXI-Stream slave on the sink half. Both carry packed bytes. Unlike RAPIDS Beats, the stream is no longer a train of full beats: each descriptor is one packet, the last beat of a packet may be partial, and TSTRB says which byte lanes are valid. The handshake, the width parameters and the monitor tap are those of RAPIDS Beats, see AXI-Stream Interface Specification (shared with RAPIDS Beats). The two data paths are specified in Source Data Path AXIS and Sink Data Path AXIS.

## 28.2	Packet Contract

Table 4.2.1: Packet Contract

| Property | Rule |
| --- | --- |
| Packet | One descriptor is one packet, ending with TLAST. |
| Packing | Bytes are packed from lane 0 upward. The descriptor start offset in memory does not appear on the stream. |
| TSTRB | Contiguous from lane 0. All ones on every beat but the last. |
| Partial beat | Only the TLAST beat may be partial, carrying the remaining bytes. |
| Packet length | Equals the descriptor length in bytes. |
| Zero length | A zero-length descriptor produces no packet. |

## 28.3	Source Master Signals

Table 4.2.2: Source Master Signals

| Signal | Value |
| --- | --- |
| m_axis_tdata | Re-packed bytes from the drain shifter. |
| m_axis_tstrb | Low min(bytes_left, DATA_WIDTH/8) lanes set. |
| m_axis_tlast | Set when the remaining byte count equals the bytes in this beat. |
| m_axis_tid | Channel number. |
| m_axis_tdest | Channel number. |
| m_axis_tuser | Zero. |

The source path pops memory beats and re-packs them, so a stream beat is not tied one to one to a memory beat. The first pop of an offset descriptor only primes the hold register and produces no output unless the packet fits in that single beat. When the last memory beat leaves bytes in the hold register, one extra flush beat carries them.

## 28.4	Sink Slave Signals

Table 4.2.3: Sink Slave Signals

| Signal | Use |
| --- | --- |
| s_axis_tdata | Payload, shifted into the per-channel hold by the descriptor offset. |
| s_axis_tstrb | Valid byte lanes. Only the TLAST beat may be partial. |
| s_axis_tid | Channel select, low CIW bits. |
| s_axis_tdest, s_axis_tuser | Unused. |
| s_axis_tlast | Ends the packet. |
| s_axis_tready | Deasserted while the output register is busy, a flush beat is pending, or the selected channel has no packet record. |

The sink stalls a packet until the scheduler has issued the record of its descriptor. A packet for a channel whose record queue is empty is not accepted, so no byte enters the SRAM before the destination offset is known. The queue holds four records per channel.

The spill of a shifted beat is held per channel. After TLAST, if the hold register still holds bytes, TREADY drops for one flush beat that writes them to the SRAM.

## 28.5	Length Check

At TLAST the sink compares the bytes received with the length in the head record of the channel. A mismatch sets the sticky per-channel r_pkt_len_error bit, which is ORed with the write engine error into sched_wr_error. Nothing is padded or dropped silently. The bytes received are counted from the lanes set in TSTRB on each beat.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	MonBus Interface Specification

Last Updated: 2025-01-10

## 29.1	Overview

The MonBus (Monitor Bus) is a 64-bit packet-based interface for reporting internal events, status, and performance data. All RAPIDS subsystems generate MonBus packets that are aggregated and output through a unified interface.

## 29.2	MonBus Signal Interface

### 29.2.1	Signal Table

Table 4.3.1: MonBus Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | output | 1 | Packet valid |
| monbus_pkt_ready | input | 1 | Consumer ready |
| monbus_pkt_data | output | 64 | Packet data |

### 29.2.2	Handshaking Protocol

Standard valid/ready handshaking: - Data transfers when valid & ready both high - Producer holds valid until ready seen - Consumer asserts ready when able to accept

### 29.2.3	Timing Diagram

### 29.2.4	Figure 4.3.1: MonBus Packet Transfer Timing

monbus_axil_group_2in - packets from two producers through the arbiter

Source: monbus_group_packet_transfer.json, captured from dv/tests/macro/test_monbus_axil_group.py (basic_flow, error FIFO 64 x 32, TEST_LEVEL=full) with WAVES=1. The 128-bit packets are shown as their top 24 bits ([127:104]: packet type nibble, then the protocol field at [108:105], so 0x000002 is AXIS and 0x000008 is CORE).

Reading it: a MonBus link is a single-beat valid/ready handshake with the packet presented alongside valid. The group holds ready high on both inputs, so the source and sink packets are accepted the cycle they are offered, and each one reappears on the arbiter output three cycles later as another one-cycle valid/ready pair. Between packets the links idle low; nothing is held across transfers.

## 29.3	MonBus Packet Format

### 29.3.1	64-bit Packet Structure

63      56 55      48 47      40 39      32 31              0
    +--------+---------+---------+---------+-------------------+
    | Type   | Unit ID | Agent ID| Channel | Payload (32 bits) |
    +--------+---------+---------+---------+-------------------+

### 29.3.2	Figure 4.3.2: MonBus Packet Format

Table 4.3.2: MonBus Packet Fields

| Field | Bits | Description |
| --- | --- | --- |
| Type | [63:56] | Packet type code |
| Unit ID | [55:48] | Reporting unit identifier |
| Agent ID | [47:40] | Source agent within unit |
| Channel | [39:32] | Channel number (0-7) |
| Payload | [31:0] | Type-specific payload |

## 29.4	Packet Types

### 29.4.1	Type Code Definitions

Table 4.3.3: MonBus Packet Types

| Type Code | Name | Description |
| --- | --- | --- |
| 0x01 | DESC_FETCH | Descriptor fetch started |
| 0x02 | DESC_COMPLETE | Descriptor fetch completed |
| 0x03 | SCHED_START | Scheduler started transfer |
| 0x04 | SCHED_DONE | Scheduler transfer complete |
| 0x10 | AXI_RD_START | AXI read burst started |
| 0x11 | AXI_RD_DONE | AXI read burst completed |
| 0x12 | AXI_WR_START | AXI write burst started |
| 0x13 | AXI_WR_DONE | AXI write burst completed |
| 0x20 | SRAM_FILL | SRAM fill event |
| 0x21 | SRAM_DRAIN | SRAM drain event |
| 0x30 | ERROR | Error event |
| 0x31 | TIMEOUT | Timeout event |
| 0xFF | HEARTBEAT | Periodic heartbeat |

### 29.4.2	Payload Formats by Type

#### 29.4.2.1	DESC_FETCH / DESC_COMPLETE (0x01, 0x02)

Table 4.3.4: Descriptor Packet Payload

| Bits | Description |
| --- | --- |
| [31:16] | Descriptor address [47:32] |
| [15:0] | Descriptor address [31:16] |

#### 29.4.2.2	SCHED_START / SCHED_DONE (0x03, 0x04)

Table 4.3.5: Scheduler Packet Payload

| Bits | Description |
| --- | --- |
| [31:16] | Transfer ID |
| [15:0] | Beat count |

#### 29.4.2.3	AXI_RD/WR_START/DONE (0x10-0x13)

Table 4.3.6: AXI Event Packet Payload

| Bits | Description |
| --- | --- |
| [31:24] | AXI ID |
| [23:16] | Burst length |
| [15:0] | Reserved |

#### 29.4.2.4	ERROR (0x30)

Table 4.3.7: Error Packet Payload

| Bits | Description |
| --- | --- |
| [31:24] | Error code |
| [23:16] | Error source |
| [15:0] | Error details |

## 29.5	MonBus Source Aggregation

### 29.5.1	RAPIDS Core MonBus Sources

Table 4.3.8: MonBus Source Assignment

| Source ID | Agent | Description |
| --- | --- | --- |
| 0-7 | Scheduler[0:7] | Per-channel scheduler events |
| 8-15 | DescEngine[0:7] | Per-channel descriptor events |
| 16-23 | SnkSRAM[0:7] | Sink SRAM controller events |
| 24-31 | SrcSRAM[0:7] | Source SRAM controller events |
| 32 | AXI Write Engine | Write transaction events |
| 33 | AXI Read Engine | Read transaction events |
| 34 | Error Handler | Error events |

Scheduler and DescEngine COMPLETION packets are subject to the half’s SCHED_CONFIG.COMPL_EN (reset 1): the scheduler group drops them ahead of its arbiter when the bit is clear and acknowledges the emitter, so the data path is unaffected. Their ERROR packets are always forwarded (rapids ISSUE-005).

### 29.5.2	Aggregation Hierarchy

### 29.5.3	Figure 4.3.3: MonBus Aggregation Tree

MonBus aggregation tree

Source: 21_monbus_aggregation.dot

## 29.6	Integration Guidelines

### 29.6.1	Consumer Requirements

MonBus consumers must: 1. Provide sufficient ready cycles to prevent packet loss 2. Handle bursty traffic (multiple sources may have packets simultaneously) 3. Process or buffer packets at wire rate

### 29.6.2	Recommended Consumer Patterns

// Pattern 1: Direct FIFO buffering
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_gaxi_fifo_sync (
    .axi_aclk              (axi_aclk),
    .axi_aresetn           (axi_aresetn),
    .wr_valid              (wr_valid),
    .wr_ready              (wr_ready),
    .wr_data               (wr_data),
    .rd_ready              (rd_ready),
    .count                 (count),
    .rd_valid              (rd_valid),
    .rd_data               (rd_data)
);

## 29.7	Timing Requirements

Table 4.3.9: MonBus Timing Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Min Ready | 1 cycle | Must assert ready within N cycles |
| Max Latency | Configurable | Before packet dropped |
| Packet Rate | Variable | Depends on activity |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Packet Record Interface Specification

Module: scheduler.sv (producer), src_data_path_axis.sv and snk_data_path_axis.sv (consumers) Location: projects/components/dma-ip/rapids/rtl/fub/ and rtl/macro/ Status: Implemented Last Updated: 2026-09-30

## 30.1	Overview

The packet record interface is new in the byte-granular design. It tells a data path how many bytes a descriptor covers and where in its first beat the data starts. RAPIDS Beats has no equivalent, because every transfer there is a whole number of aligned beats.

The scheduler is the producer. It emits one record per data descriptor. The source or sink data path is the consumer and holds the records in a small queue per channel. The record interface is internal to each half: it does not appear on the rapids_core or rapids_top ports.

### 30.1.1	Figure 4.4.1: Scheduler Byte Math

Scheduler byte math

Source: assets/graphviz/03_scheduler_byte_math.dot

## 30.2	Signals

There is one set per direction. The source half uses sched_rd_pkt_* and the sink half uses sched_wr_pkt_*. Each signal is a vector with one entry per channel.

Table 4.4.1: Packet Record Signals

| Signal | Direction (scheduler view) | Width per channel | Description |
| --- | --- | --- | --- |
| sched_*_pkt_valid | output | 1 | One-cycle pulse: a record is presented. |
| sched_*_pkt_ready | input | 1 | The consumer queue has room. |
| sched_*_pkt_bytes | output | 32 | Descriptor length in bytes. |
| sched_*_pkt_offset | output | OFF_W | Address offset inside the first beat. |

OFF_W is $clog2(DATA_WIDTH/8), 5 for a 256-bit build.

## 30.3	Protocol

The scheduler raises valid for one cycle, in the cycle it leaves the descriptor fetch state. It does not leave that state until every enabled direction can take the record, so ready is a pre-condition of the pulse, not a wait state after it. Both directions of a channel see exactly one pulse per data descriptor.

A consumer accepts a record when valid and its queue is not full. It drives ready low when the queue is full, which is what holds the scheduler in its fetch state.

Table 4.4.2: Record Rules

| Rule | Detail |
| --- | --- |
| Records per descriptor | One, for data descriptors of nonzero length. |
| Zero-length descriptors | No record. |
| Non-data descriptors | No record. |
| Order | Records are consumed in the order pushed, per channel. |
| Queue depth | Four records per channel in each data path. |
| Pop | The record pops on the last beat of its packet. |

The offset is taken from the source address in the read direction and from the destination address in the write direction. The length is the same for both. When both directions are enabled, each data path receives its own offset.

## 30.4	Unused Direction

A scheduler group array instantiated for only one direction ties the other direction off inside the half. The unused valid, bytes and offset outputs are left open and the unused ready inputs are held at all ones, so the scheduler never waits on a queue that does not exist.

## 30.5	Consumer Behavior

- The source data path uses the record to compute the number of memory beats, to decide when a pop emits a stream beat, to set TSTRB and TLAST, and to retire the record.

- The sink data path uses the record to decide how far to shift each incoming beat, to check the received length at TLAST, and it holds off TREADY for a channel until the record exists.

Last Updated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	RAPIDS Beats Architecture MAS

Version: 0.7 Date: 2026-07-13 Purpose: Module Architecture Specification for RAPIDS Phase 1 “Beats” Architecture

## 31.1	Document Organization

Note: All chapters linked below for automated document generation.

### 31.1.1	Front Matter

- Document Information

### 31.1.2	Chapter 1: Overview

- Architecture Overview

- Top-Level Port List

- Clocks and Reset

### 31.1.3	Chapter 2: FUB (Functional Unit Blocks)

Control Path: - Scheduler - Descriptor Engine

AXI Engines: - AXI Read Engine - AXI Write Engine

Flow Control: - Beats Alloc Control - Beats Drain Control - Beats Latency Bridge

Control Engines: - Control-Read Engine - Control-Write Engine

### 31.1.4	Chapter 3: Macro (Integration Blocks)

Scheduler Integration: - Beats Scheduler Group - Beats Scheduler Group Array

Sink Data Path (Network to Memory): - Sink Data Path - Sink Data Path AXIS - Sink SRAM Controller

Source Data Path (Memory to Network): - Source Data Path - Source Data Path AXIS - Source SRAM Controller

Top-Level Integration: - RAPIDS Core Beats - RAPIDS Registers - RAPIDS Config Block - RAPIDS Beats Top

### 31.1.5	Chapter 4: Interfaces

- Interface Overview

- AXI4 Interface Specification

- AXIS Interface Specification

- MonBus Interface Specification

## 31.2	Quick Reference

### 31.2.1	FUB Modules (fub_beats/)

FUB Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler_beats | scheduler_beats.sv | Transfer coordinator, descriptor processing | Implemented |
| descriptor_engine_beats | descriptor_engine_beats.sv | Descriptor fetch/parse (256-bit) | Implemented |
| axi_read_engine_beats | axi_read_engine_beats.sv | AXI read master, streaming pipeline | Implemented |
| axi_write_engine_beats | axi_write_engine_beats.sv | AXI write master, streaming pipeline | Implemented |
| alloc_ctrl_beats | alloc_ctrl_beats.sv | Space allocation tracking (virtual FIFO); kept and tested, no longer in the SRAM path since bdf4e0dff | Implemented |
| drain_ctrl_beats | drain_ctrl_beats.sv | Data availability tracking (virtual FIFO); kept and tested, no longer in the SRAM path | Implemented |
| latency_bridge_beats | latency_bridge_beats.sv | Registered-FIFO to valid/ready skid bridge; kept and tested, no longer in the SRAM path | Implemented |
| ctrlrd_engine | ctrlrd_engine.sv | Control-read consumer gate (poll-until-match, retry budget) | Implemented |
| ctrlwr_engine | ctrlwr_engine.sv | Control-write producer doorbell (single-beat write) | Implemented |

### 31.2.2	Macro Modules (macro_beats/)

Macro Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler_group_beats | scheduler_group_beats.sv | Scheduler + Descriptor Engine wrapper | Implemented |
| scheduler_group_array_beats | scheduler_group_array_beats.sv | 8-channel scheduler array with arbitration | Implemented |
| snk_data_path_beats | snk_data_path_beats.sv | Network-to-memory path integration | Implemented |
| snk_data_path_axis_beats | snk_data_path_axis_beats.sv | AXIS variant of sink path | Implemented |
| src_data_path_beats | src_data_path_beats.sv | Memory-to-network path integration | Implemented |
| src_data_path_axis_beats | src_data_path_axis_beats.sv | AXIS variant of source path | Implemented |
| snk_sram_controller_beats | snk_sram_controller_beats.sv | Sink SRAM: naming wrapper around STREAM’s sram_controller (one instance, per-channel FIFOs inside it) | Implemented |
| src_sram_controller_beats | src_sram_controller_beats.sv | Source SRAM: naming wrapper around STREAM’s sram_controller | Implemented |
| rapids_src_beats / rapids_snk_beats | rapids_src_beats.sv, rapids_snk_beats.sv | One half each: scheduler array + data path + monbus merge | Implemented |
| rapids_core_beats | rapids_core_beats.sv | Both halves + the core monbus arbiter | Implemented |
| rapids_regs | regs/generated/rtl/rapids_regs.sv (generated from rtl/macro_beats/rapids_regs.rdl) | PeakRDL register block (base + MON regfile @ 0x1000) | Implemented |
| rapids_config_block | rapids_config_block.sv | Maps register hwif_out to core/monitor cfg_* | Implemented |
| rapids_beats_top | rapids_beats_top.sv | Top-level: APB slave, kickoff, monitors, MonBus AXI-Lite group | Implemented |

## 31.3	Architecture Comparison: RAPIDS vs STREAM

The “beats” architecture is a Phase 1 implementation of RAPIDS that shares concepts with STREAM but targets network-to-memory (and vice versa) transfers rather than memory-to-memory.

RAPIDS Beats vs STREAM Comparison

| Feature | STREAM | RAPIDS Beats |
| --- | --- | --- |
| Primary Use | Memory-to-memory DMA | Network-to-memory accelerator |
| Data Paths | Single bidirectional | Separate sink/source paths |
| Network Interface | None | AXIS master/slave |
| SRAM Buffering | Shared buffer | Separate sink/source buffers |
| Descriptor Format | 256-bit | 256-bit (compatible) |
| Channel Count | 8 | 8 |
| Flow Control | alloc_ctrl/drain_ctrl | STREAM stream_alloc_ctrl/stream_drain_ctrl inside the shared sram_controller (alloc_ctrl_beats/drain_ctrl_beats kept, out of the path) |

## 31.4	Clock and Reset Summary

### 31.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | Primary - all RAPIDS logic, AXI/AXIS interfaces |

### 31.4.2	Reset Signals

Reset Signals

| Reset | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary - all RAPIDS logic |

See: Clocks and Reset for complete timing specifications

## 31.5	Interface Summary

### 31.5.1	External Interfaces

External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXI4 (Descriptor) | Master | 256-bit | Descriptor fetch |
| AXI4 (Sink Write) | Master | 512-bit (param) | Sink data write to memory |
| AXI4 (Source Read) | Master | 512-bit (param) | Source data read from memory |
| AXIS (Sink) | Slave | 512-bit (param) | Network data ingress |
| AXIS (Source) | Master | 512-bit (param) | Network data egress |
| MonBus | Master | 64-bit | Monitor packet output |

### 31.5.2	Internal Buses

Internal Buses

| Interface | Width | Purpose |
| --- | --- | --- |
| MonBus | 64-bit | Internal monitoring bus |
| Descriptor Bus | 256-bit | Descriptor distribution |
| SRAM Data Bus | 512-bit | SRAM read/write data |

## 31.6	Related Documentation

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- RAPIDS Beats HAS - Architecture specification (the original pre-beats rapids_spec/ tree was retired)

## 31.7	Specification Conventions

### 31.7.1	Signal Naming

- Clock: aclk

- Reset: aresetn (active-low)

- Valid/Ready: Standard AXI/AXIS handshake

- Registers: r_ prefix (e.g., r_state, r_counter)

- Wires: w_ prefix (e.g., w_next_state, w_grant)

### 31.7.2	Parameter Naming

- Uppercase: NUM_CHANNELS, DATA_WIDTH, ADDR_WIDTH

- Derived: CHAN_WIDTH = $clog2(NUM_CHANNELS)

### 31.7.3	Figure and Table Numbering

- Figures: Figure X.Y.Z: Title where X=chapter, Y=section, Z=figure number

- Tables: Caption line : Table Title after table markdown

Last Updated: 2026-07-13 Maintained By: RAPIDS Architecture Team