RTL Design Sherpa

RAPIDS Byte-Granular DMA Engine Byte Characterization Report

September 30, 2026

Table of Contents

Readme	6

RAPIDS Byte-Granular DMA: Byte Characterization Report	6

1. Headline	6

2. Definitions	7

3. Beat-aligned rows against the RAPIDS Beats report	8

3.1 Word-wide checker build (the verdict)	8

3.2 Byte-wise checker build (the standard bitstream)	11

Figure 3.1: Byte minus beats utilization, beat-aligned cells	12

4. Transfer size (offset 0)	14

4.1 Sink path	15

4.2 Source path	17

Figure 4.1: Efficiency against payload size	20

Figure 4.2: Measured MB/s against payload size, peak as the dashed line	21

5. Start offset	21

5.1 Sink path	21

5.2 Source path	24

Figure 5.1: Memory-side efficiency against start offset	26

6. Descriptor chains	26

7. Source backpressure (integrity only)	27

8. Failures, caveats and coverage	28

9. Build and resources	28

10. Provenance and reproduction	29

List of Figures

Figure 3.1: Byte minus beats utilization, beat-aligned cells	12

Figure 4.1: Efficiency against payload size	20

Figure 4.2: Measured MB/s against payload size, peak as the dashed line	21

Figure 5.1: Memory-side efficiency against start offset	26

List of Tables

Table 1: Largest transfers at 8 channels	6

Table 2: Definitions	7

Table 3: Engaged utilization, byte build / RAPIDS Beats reference, beat-aligned rows	8

Table 4: Cycle counts at 4096 beats per channel (fixed start-up terms, not rates)	10

Table 5: Engaged utilization, byte build / RAPIDS Beats reference, beat-aligned rows	11

Table 6: Beat-aligned rows: bytes, beats, efficiency, MB/s and share of the 3200 MB/s peak	13

Table 7: Size sweep, sink path, offset 0	15

Table 8: Size sweep, source path, offset 0	17

Table 9: Offset sweep, sink path	21

Table 10: Offset sweep, source path	24

Table 11: Descriptor chains, 4 channels	26

Table 12: Source with backpressure, integrity rows	27

Table 13: Post-route resources and timing at 100 MHz, XC7K325T (203,800 LUTs, 445 BRAM tiles)	28

Table 14: Provenance	29

Table 15: Device readback per session	30

Table 16: Device readback per session	30

# Readme

Generated: 2026-09-30

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# RAPIDS Byte-Granular DMA: Byte Characterization Report

Version: 0.2
Date: 2026-09-30
Platform: Genesys 2, 256-bit, 8 channels, 100 MHz
Results: rapids_byte_perf_20260930_142731.json

## 1. Headline

Largest transfers at 8 channels

| Path | Point | Efficiency | MB/s measured | % of 3200 MB/s peak |
| --- | --- | --- | --- | --- |
| Sink | byte path, 4096 B x 8 ch | 1.000 | 357 | 11.1 % |
| Sink | beat path, 4096 beats x 8 ch | 1.000 | 356 | 11.1 % |
| Sink | beat path, 4096 beats x 8 ch, word-wide checker build | 1.000 | 3182 | 99.4 % |
| Source | byte path, 4096 B x 8 ch | 1.000 | 352 | 11.0 % |
| Source | beat path, 4096 beats x 8 ch | 1.000 | 356 | 11.1 % |
| Source | beat path, 4096 beats x 8 ch, word-wide checker build | 1.000 | 3199 | 100.0 % |

The byte-wise rates sit at the harness checker ceiling of 3200 / 9 = 355.6 MB/s, not at the DUT’s limit: the byte-wise CRC checkers take 9 cycles per 32-byte beat (section 3). Read every MB/s in this report against both the 3200 MB/s peak and that ceiling. Efficiency (payload over beats x lanes) does not depend on the checker.

The word-wide checker build (BYTE_CRC=0) takes one beat per cycle and is not checker-bound: its rows show the DUT’s own rate against the 3200 MB/s peak. Its data check covers 4 of the 32 bytes of each beat, so it is a rate measurement and the byte-wise build is the integrity measurement.

## 2. Definitions

Definitions

| Term | Meaning |
| --- | --- |
| Bytes moved | Total payload bytes over all active channels and descriptors; from the exact byte count of the AXIS bus meter. |
| Beats moved | Accepted beats on the interface named: AXIS beats on the stream side, memory beats on the AXI side. |
| Efficiency | payload bytes / (beats x BYTE_LANES), BYTE_LANES = 32. 1.0 means every lane of every beat carried payload. |
| MB/s | Total bytes / measurement window, windows counted in 100 MHz cycles by the harness meters, not wall clock. |
| Peak | 3200 MB/s = 100 MHz x 32 B, per direction. Shown beside every measured MB/s value. |
| Engaged utilization | prod / (prod + bp + starv) on one interface; the RAPIDS Beats headline metric, used only for the beat-aligned comparison. |

## 3. Beat-aligned rows against the RAPIDS Beats report

These rows use the beat-scaled path (pkt_bytes=None, beats per channel) on the byte DUT, which is how the RAPIDS Beats report measured its matrix: 9-beat bursts, response delay 0, backpressure off, default seed, bare bus meters. The metric is engaged utilization (prod / (prod + bp + starv)), the RAPIDS Beats headline metric. “Unchanged” is checked numerically: every cell is compared to the same cell of genesys_dw256_obs_C.json and must agree within 0.5 percentage points, the threshold the RAPIDS Beats report itself applies between builds.

### 3.1 Word-wide checker build (the verdict)

Measured on the BYTE_CRC=0 bitstream (BUILD.WORD_CRC = 1, results file rapids_byte_aligned_wordcrc_20260930.json, bitstream sha256 48e1282c46e707cd). Its checkers take one beat per cycle like the RAPIDS Beats build. They CRC one 32-bit slice of each beat (slice 0, which holds the beat’s LFSR word) and ignore the strobes, so the golden is the RAPIDS Beats golden and the data check covers 4 of the 32 bytes of each beat. That is enough for a utilization measurement of whole-beat rows; full-byte integrity is what the byte-wise build in 3.2 and the rest of this report check. The design under test is the same RTL.

Engaged utilization, byte build / RAPIDS Beats reference, beat-aligned rows

| Ch | Beats/ch | Sink AXIS-in % (byte / beats) | Sink AXI4-wr % | Source AXI4-rd % | Source AXIS-out % | Max abs delta (pp) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 50.0 / 50.0 | 10.0 / 14.3 | 6.2 / 6.7 | 6.2 / 6.7 | 4.29 |
| 1 | 4 | 100.0 / 80.0 | 25.0 / 40.0 | 21.1 / 22.2 | 21.1 / 22.2 | 20.00 |
| 1 | 16 | 25.0 / 94.1 | 48.5 / 72.7 | 51.6 / 53.3 | 51.6 / 53.3 | 69.12 |
| 1 | 64 | 57.1 / 98.5 | 79.0 / 91.4 | 81.0 / 82.1 | 81.0 / 82.1 | 41.32 |
| 1 | 256 | 84.2 / 99.6 | 93.8 / 97.7 | 94.5 / 94.8 | 94.5 / 94.8 | 15.40 |
| 1 | 1024 | 95.5 / 99.9 | 98.4 / 99.4 | 98.6 / 98.7 | 98.6 / 98.7 | 4.38 |
| 1 | 4096 | 98.8 / 100.0 | 99.6 / 99.9 | 99.6 / 99.7 | 99.6 / 99.7 | 1.13 |
| 2 | 1 | 66.7 / 66.7 | 16.7 / 22.2 | 11.1 / 11.8 | 11.1 / 11.8 | 5.56 |
| 2 | 4 | 10.5 / 88.9 | 40.0 / 57.1 | 34.8 / 36.4 | 34.8 / 36.4 | 78.36 |
| 2 | 16 | 32.0 / 97.0 | 65.3 / 84.2 | 68.1 / 69.6 | 68.1 / 69.6 | 64.97 |
| 2 | 64 | 65.3 / 99.2 | 88.3 / 95.5 | 89.5 / 90.1 | 89.5 / 90.1 | 33.92 |
| 2 | 256 | 88.3 / 99.8 | 96.8 / 98.8 | 97.2 / 97.3 | 97.2 / 97.3 | 11.53 |
| 2 | 1024 | 96.8 / 100.0 | 99.2 / 99.7 | 99.3 / 99.3 | 99.3 / 99.3 | 3.16 |
| 2 | 4096 | 99.2 / 100.0 | 99.8 / 99.9 | 99.8 / 99.8 | 99.8 / 99.8 | 0.81 |
| 4 | 1 | 100.0 / 80.0 | 25.0 / 30.8 | 18.2 / 19.0 | 18.2 / 19.0 | 20.00 |
| 4 | 4 | 12.9 / 94.1 | 57.1 / 72.7 | 51.6 / 53.3 | 51.6 / 53.3 | 81.21 |
| 4 | 16 | 37.2 / 98.5 | 79.0 / 91.4 | 81.0 / 82.1 | 81.0 / 82.1 | 61.25 |
| 4 | 64 | 70.3 / 99.6 | 93.8 / 97.3 | 94.5 / 94.8 | 94.5 / 94.8 | 29.28 |
| 4 | 256 | 90.5 / 99.7 | 98.4 / 99.4 | 98.6 / 98.7 | 98.6 / 98.7 | 9.25 |
| 4 | 1024 | 97.4 / 99.9 | 99.6 / 99.9 | 99.6 / 99.7 | 99.6 / 99.7 | 2.50 |
| 4 | 4096 | 99.3 / 100.0 | 99.9 / 100.0 | 99.9 / 99.9 | 99.9 / 99.9 | 0.64 |
| 8 | 1 | 4.0 / 88.9 | 33.3 / 38.1 | 26.7 / 27.6 | 26.7 / 27.6 | 84.85 |
| 8 | 4 | 14.5 / 97.0 | 72.7 / 84.2 | 68.1 / 69.6 | 68.1 / 69.6 | 82.42 |
| 8 | 16 | 40.5 / 99.2 | 88.3 / 95.5 | 89.5 / 90.1 | 89.5 / 90.1 | 58.72 |
| 8 | 64 | 73.1 / 99.8 | 96.8 / 98.5 | 97.2 / 97.3 | 97.2 / 97.3 | 26.66 |
| 8 | 256 | 91.6 / 96.1 | 99.2 / 99.7 | 99.3 / 99.3 | 99.3 / 99.3 | 4.51 |
| 8 | 1024 | 97.8 / 99.0 | 99.8 / 99.9 | 99.8 / 99.8 | 99.8 / 99.8 | 1.24 |
| 8 | 4096 | 99.4 / 99.7 | 99.9 / 100.0 | 100.0 / 100.0 | 100.0 / 100.0 | 0.32 |

Verdict: CHANGED. 112 of 112 beat-aligned cells were compared; 73 differ by more than 0.5 pp; the largest difference is -84.85 pp (ch 8, 1 beats, sink sin). Cells over the threshold: ch1 b1 sink wr -4.29 pp; ch1 b4 sink sin +20.00 pp; ch1 b4 sink wr -15.00 pp; ch1 b4 source rd -1.17 pp; ch1 b4 source sout -1.17 pp; ch1 b16 sink sin -69.12 pp; ch1 b16 sink wr -24.24 pp; ch1 b16 source rd -1.72 pp; ch1 b16 source sout -1.72 pp; ch1 b64 sink sin -41.32 pp; ch1 b64 sink wr -12.42 pp; ch1 b64 source rd -1.04 pp; ….

Where the differences come from. Every difference is a fixed number of cycles per run, not a change of rate. The cycle counts below are the evidence; the same counts at 16 beats per channel are identical to these, and only the 1-beat rows differ (the write side shows +3 starvation cycles there, not +11).

Cycle counts at 4096 beats per channel (fixed start-up terms, not rates)

| Ch | Sink AXIS-in bp, byte | beats | Sink AXI4-wr starv, byte | beats | Source AXI4-rd starv, byte | beats |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 47 | 0 | 17 | 6 | 15 | 14 |
| 2 | 67 | 0 | 17 | 6 | 15 | 14 |
| 4 | 107 | 2 | 17 | 6 | 15 | 14 |
| 8 | 187 | 82 | 17 | 6 | 15 | 14 |

- Sink AXIS-in. The byte ingress accepts a channel’s stream only once that channel has a packet record, because the destination offset and the expected length come from the descriptor (sink ingress chapter of the MAS). The stream is offered data, tready is low, and the meter counts those cycles as backpressure. The count grows with the channel count and does not depend on the beats per channel, which is what a serial descriptor fetch before the first accept would give. RAPIDS Beats has no such term because it buffers the stream before the descriptor arrives. The AXIS-in window opens at the first offered beat, so this wait is inside it.

- Sink AXI4-wr. The write-side window opens at the first write, so the wait above is not in it. The byte build adds a constant 11 starvation cycles at 16 beats and up (3 at 1 beat), the same for every channel count. Its source has not been isolated.

- Source. One extra starvation cycle at start-up in every cell, on both rd and sout: up to 1.7 percentage points at 1 to 64 beats, under 0.4 from 256 beats. Not isolated.

- Amortisation. Because each term is fixed, utilization converges on the RAPIDS Beats value as the transfer grows: the 4096-beat rows are within 1.2 percentage points and the 8-channel row within 0.4. The 1-to-64-beat rows, where the fixed term is a large share of the window, are the ones that move most.

- Not explained. With 1 beat per channel and 1, 2 or 4 channels the byte build shows no sink AXIS-in backpressure, while the same channels at 16 beats and up do. Not isolated.

### 3.2 Byte-wise checker build (the standard bitstream)

Engaged utilization, byte build / RAPIDS Beats reference, beat-aligned rows

| Ch | Beats/ch | Sink AXIS-in % (byte / beats) | Sink AXI4-wr % | Source AXI4-rd % | Source AXIS-out % | Max abs delta (pp) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 50.0 / 50.0 | 10.0 / 14.3 | 6.2 / 6.7 | 6.7 / 6.7 | 4.29 |
| 1 | 4 | 100.0 / 80.0 | 25.0 / 40.0 | 9.3 / 22.2 | 9.5 / 22.2 | 20.00 |
| 1 | 16 | 25.0 / 94.1 | 13.6 / 72.7 | 10.6 / 53.3 | 10.7 / 53.3 | 69.12 |
| 1 | 64 | 57.1 / 98.5 | 11.6 / 91.4 | 20.7 / 82.1 | 10.7 / 82.1 | 79.79 |
| 1 | 256 | 22.6 / 99.6 | 11.2 / 97.7 | 18.6 / 94.8 | 11.0 / 94.8 | 86.47 |
| 1 | 1024 | 12.7 / 99.9 | 11.1 / 99.4 | 12.4 / 98.7 | 11.1 / 98.7 | 88.27 |
| 1 | 4096 | 11.5 / 100.0 | 11.1 / 99.9 | 11.4 / 99.7 | 11.1 / 99.7 | 88.73 |
| 2 | 1 | 66.7 / 66.7 | 16.7 / 22.2 | 8.0 / 11.8 | 8.7 / 11.8 | 5.56 |
| 2 | 4 | 10.5 / 88.9 | 19.5 / 57.1 | 10.1 / 36.4 | 10.3 / 36.4 | 78.36 |
| 2 | 16 | 32.0 / 97.0 | 12.2 / 84.2 | 11.7 / 69.6 | 10.6 / 69.6 | 72.00 |
| 2 | 64 | 65.3 / 99.2 | 11.4 / 95.5 | 33.6 / 90.1 | 10.8 / 90.1 | 84.15 |
| 2 | 256 | 14.8 / 99.8 | 11.2 / 98.8 | 19.8 / 97.3 | 11.1 / 97.3 | 87.67 |
| 2 | 1024 | 11.8 / 100.0 | 11.1 / 99.7 | 12.5 / 99.3 | 11.1 / 99.3 | 88.58 |
| 2 | 4096 | 11.3 / 100.0 | 11.1 / 99.9 | 11.4 / 99.8 | 11.1 / 99.8 | 88.81 |
| 4 | 1 | 100.0 / 80.0 | 25.0 / 30.8 | 9.3 / 19.0 | 9.8 / 19.0 | 20.00 |
| 4 | 4 | 12.9 / 94.1 | 14.2 / 72.7 | 10.6 / 53.3 | 10.7 / 53.3 | 81.21 |
| 4 | 16 | 37.2 / 98.5 | 11.6 / 91.4 | 21.0 / 82.1 | 10.8 / 82.1 | 79.79 |
| 4 | 64 | 70.3 / 99.6 | 11.2 / 97.3 | 50.3 / 94.8 | 10.8 / 94.8 | 86.12 |
| 4 | 256 | 12.6 / 99.7 | 11.1 / 99.4 | 20.7 / 98.7 | 11.1 / 98.7 | 88.27 |
| 4 | 1024 | 11.5 / 99.9 | 11.1 / 99.9 | 12.6 / 99.7 | 11.1 / 99.7 | 88.73 |
| 4 | 4096 | 11.2 / 100.0 | 11.1 / 100.0 | 11.4 / 99.9 | 11.1 / 99.9 | 88.85 |
| 8 | 1 | 4.0 / 88.9 | 16.0 / 38.1 | 10.1 / 27.6 | 10.4 / 27.6 | 84.85 |
| 8 | 4 | 14.5 / 97.0 | 12.5 / 84.2 | 11.3 / 69.6 | 10.6 / 69.6 | 82.42 |
| 8 | 16 | 40.5 / 99.2 | 11.4 / 95.5 | 34.5 / 90.1 | 10.8 / 90.1 | 84.15 |
| 8 | 64 | 73.1 / 99.8 | 11.2 / 98.5 | 66.2 / 97.3 | 11.0 / 97.3 | 87.30 |
| 8 | 256 | 11.8 / 96.1 | 11.1 / 99.7 | 21.4 / 99.3 | 11.1 / 99.3 | 88.58 |
| 8 | 1024 | 11.3 / 99.0 | 11.1 / 99.9 | 12.6 / 99.8 | 11.1 / 99.8 | 88.81 |
| 8 | 4096 | 11.2 / 99.7 | 11.1 / 100.0 | 11.5 / 100.0 | 11.1 / 100.0 | 88.87 |

Result on this build: CHANGED. 112 of 112 beat-aligned cells were compared; 108 differ by more than 0.5 pp; the largest difference is -88.87 pp (ch 8, 4096 beats, sink wr). Cells over the threshold: ch1 b1 sink wr -4.29 pp; ch1 b4 sink sin +20.00 pp; ch1 b4 sink wr -15.00 pp; ch1 b4 source rd -12.92 pp; ch1 b4 source sout -12.70 pp; ch1 b16 sink sin -69.12 pp; ch1 b16 sink wr -59.17 pp; ch1 b16 source rd -42.74 pp; ch1 b16 source sout -42.67 pp; ch1 b64 sink sin -41.32 pp; ch1 b64 sink wr -79.79 pp; ch1 b64 source rd -61.34 pp; ….

What the byte-checker rows can and cannot show. The byte build’s on-chip checkers (BYTE_CRC=1 on axi4_slave_wr_crc_check and axis4_slave_pattern_check) fold the strobed bytes of each beat into the CRC four bytes per cycle and hold ready low while they do, so a 32-byte beat occupies 9 cycles. That caps the harness at 3200 / 9 = 355.6 MB/s (11.1 % of the 3200 MB/s peak), which is where the large-transfer rows sit. The RAPIDS Beats reference used the word-wide checkers and is not checker-bound. The differences in this table are therefore the checkers backpressuring the DUT, not evidence about the DUT’s own throughput, and this table is not comparable to the RAPIDS Beats report. The “utilization unchanged” criterion is settled in section 3.1 on the word-wide checker build (BYTE_CRC=0, BUILD.WORD_CRC = 1), where every checker takes one beat per cycle as in the RAPIDS Beats build.

### Figure 3.1: Byte minus beats utilization, beat-aligned cells

utilization delta

Beat-aligned rows: bytes, beats, efficiency, MB/s and share of the 3200 MB/s peak

| Ch | Beats/ch | Sink bytes | Sink beats | Sink eff | Sink MB/s | Sink % peak | Src bytes | Src beats | Src eff | Src MB/s | Src % peak |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 32 | 1 | 1.000 | 320 | 10.0 % | 32 | 1 | 1.000 | 200 | 6.2 % |
| 1 | 4 | 128 | 4 | 1.000 | 800 | 25.0 % | 128 | 4 | 1.000 | 298 | 9.3 % |
| 1 | 16 | 512 | 16 | 1.000 | 430 | 13.4 % | 512 | 16 | 1.000 | 339 | 10.6 % |
| 1 | 64 | 2048 | 64 | 1.000 | 372 | 11.6 % | 1056 | 33 | 1.000 | 342 | 10.7 % |
| 1 | 256 | 8192 | 256 | 1.000 | 359 | 11.2 % | 4864 | 152 | 1.000 | 353 | 11.0 % |
| 1 | 1024 | 32768 | 1024 | 1.000 | 357 | 11.1 % | 29440 | 920 | 1.000 | 355 | 11.1 % |
| 1 | 4096 | 131072 | 4096 | 1.000 | 356 | 11.1 % | 127744 | 3992 | 1.000 | 355 | 11.1 % |
| 2 | 1 | 64 | 2 | 1.000 | 533 | 16.7 % | 64 | 2 | 1.000 | 256 | 8.0 % |
| 2 | 4 | 256 | 8 | 1.000 | 337 | 10.5 % | 256 | 8 | 1.000 | 324 | 10.1 % |
| 2 | 16 | 1024 | 32 | 1.000 | 389 | 12.2 % | 928 | 29 | 1.000 | 340 | 10.6 % |
| 2 | 64 | 4096 | 128 | 1.000 | 363 | 11.4 % | 1312 | 41 | 1.000 | 344 | 10.8 % |
| 2 | 256 | 16384 | 512 | 1.000 | 357 | 11.2 % | 9152 | 286 | 1.000 | 354 | 11.1 % |
| 2 | 1024 | 65536 | 2048 | 1.000 | 356 | 11.1 % | 58304 | 1822 | 1.000 | 355 | 11.1 % |
| 2 | 4096 | 262144 | 8192 | 1.000 | 356 | 11.1 % | 254912 | 7966 | 1.000 | 356 | 11.1 % |
| 4 | 1 | 128 | 4 | 1.000 | 800 | 25.0 % | 128 | 4 | 1.000 | 298 | 9.3 % |
| 4 | 4 | 512 | 16 | 1.000 | 413 | 12.9 % | 512 | 16 | 1.000 | 339 | 10.6 % |
| 4 | 16 | 2048 | 64 | 1.000 | 372 | 11.6 % | 1056 | 33 | 1.000 | 346 | 10.8 % |
| 4 | 64 | 8192 | 256 | 1.000 | 358 | 11.2 % | 1760 | 55 | 1.000 | 346 | 10.8 % |
| 4 | 256 | 32768 | 1024 | 1.000 | 357 | 11.1 % | 17536 | 548 | 1.000 | 355 | 11.1 % |
| 4 | 1024 | 131072 | 4096 | 1.000 | 356 | 11.1 % | 115840 | 3620 | 1.000 | 355 | 11.1 % |
| 4 | 4096 | 524288 | 16384 | 1.000 | 356 | 11.1 % | 509056 | 15908 | 1.000 | 356 | 11.1 % |
| 8 | 1 | 256 | 8 | 1.000 | 129 | 4.0 % | 256 | 8 | 1.000 | 324 | 10.1 % |
| 8 | 4 | 1024 | 32 | 1.000 | 397 | 12.4 % | 960 | 30 | 1.000 | 338 | 10.6 % |
| 8 | 16 | 4096 | 128 | 1.000 | 363 | 11.4 % | 1280 | 40 | 1.000 | 345 | 10.8 % |
| 8 | 64 | 16384 | 512 | 1.000 | 357 | 11.1 % | 2720 | 85 | 1.000 | 352 | 11.0 % |
| 8 | 256 | 65536 | 2048 | 1.000 | 356 | 11.1 % | 33920 | 1060 | 1.000 | 355 | 11.1 % |
| 8 | 1024 | 262144 | 8192 | 1.000 | 356 | 11.1 % | 230528 | 7204 | 1.000 | 356 | 11.1 % |
| 8 | 4096 | 1048576 | 32768 | 1.000 | 356 | 11.1 % | 1016960 | 31780 | 1.000 | 356 | 11.1 % |

Every beat-aligned row moves whole beats, so efficiency is 1.000 by construction; measured range over 56 rows: 1.000 to 1.000.

## 4. Transfer size (offset 0)

One descriptor per channel, payload in bytes, start offset 0. Bytes moved and beats are totals over the active channels; efficiency is payload bytes over beats times 32 bytes per beat, on the AXIS side and on the memory side. MB/s is total bytes over the measurement window (the longer of the stream and memory windows) and is always shown with its share of the 3200 MB/s peak (100 MHz x 32 B).

### 4.1 Sink path

Size sweep, sink path, offset 0

| Payload B | Offset | Ch | Bytes moved | AXIS beats | Mem beats | Eff (AXIS) | Eff (mem) | MB/s | % of peak | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 | 1 | 0.031 | 0.031 | 10.0 | 0.31 % | PASS |
| 1 | 0 | 2 | 2 | 2 | 2 | 0.031 | 0.031 | 16.7 | 0.52 % | PASS |
| 1 | 0 | 4 | 4 | 4 | 4 | 0.031 | 0.031 | 25.0 | 0.78 % | PASS |
| 1 | 0 | 8 | 8 | 8 | 8 | 0.031 | 0.031 | 4.0 | 0.13 % | PASS |
| 7 | 0 | 1 | 7 | 1 | 1 | 0.219 | 0.219 | 70.0 | 2.19 % | PASS |
| 7 | 0 | 2 | 14 | 2 | 2 | 0.219 | 0.219 | 116.7 | 3.65 % | PASS |
| 7 | 0 | 4 | 28 | 4 | 4 | 0.219 | 0.219 | 175.0 | 5.47 % | PASS |
| 7 | 0 | 8 | 56 | 8 | 8 | 0.219 | 0.219 | 28.1 | 0.88 % | PASS |
| 32 | 0 | 1 | 32 | 1 | 1 | 1.000 | 1.000 | 320.0 | 10.00 % | PASS |
| 32 | 0 | 2 | 64 | 2 | 2 | 1.000 | 1.000 | 533.3 | 16.67 % | PASS |
| 32 | 0 | 4 | 128 | 4 | 4 | 1.000 | 1.000 | 800.0 | 25.00 % | PASS |
| 32 | 0 | 8 | 256 | 8 | 8 | 1.000 | 1.000 | 128.6 | 4.02 % | PASS |
| 33 | 0 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 275.0 | 8.59 % | PASS |
| 33 | 0 | 2 | 66 | 4 | 4 | 0.516 | 0.516 | 471.4 | 14.73 % | PASS |
| 33 | 0 | 4 | 132 | 8 | 8 | 0.516 | 0.516 | 113.8 | 3.56 % | PASS |
| 33 | 0 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 129.4 | 4.04 % | PASS |
| 64 | 0 | 1 | 64 | 2 | 2 | 1.000 | 1.000 | 533.3 | 16.67 % | PASS |
| 64 | 0 | 2 | 128 | 4 | 4 | 1.000 | 1.000 | 914.3 | 28.57 % | PASS |
| 64 | 0 | 4 | 256 | 8 | 8 | 1.000 | 1.000 | 220.7 | 6.90 % | PASS |
| 64 | 0 | 8 | 512 | 16 | 16 | 1.000 | 1.000 | 251.0 | 7.84 % | PASS |
| 77 | 0 | 1 | 77 | 3 | 3 | 0.802 | 0.802 | 550.0 | 17.19 % | PASS |
| 77 | 0 | 2 | 154 | 6 | 6 | 0.802 | 0.802 | 208.1 | 6.50 % | PASS |
| 77 | 0 | 4 | 308 | 12 | 12 | 0.802 | 0.802 | 256.7 | 8.02 % | PASS |
| 77 | 0 | 8 | 616 | 24 | 24 | 0.802 | 0.802 | 290.6 | 9.08 % | PASS |
| 203 | 0 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 369.1 | 11.53 % | PASS |
| 203 | 0 | 2 | 406 | 14 | 14 | 0.906 | 0.906 | 431.9 | 13.50 % | PASS |
| 203 | 0 | 4 | 812 | 28 | 28 | 0.906 | 0.906 | 386.7 | 12.08 % | PASS |
| 203 | 0 | 8 | 1624 | 56 | 56 | 0.906 | 0.906 | 367.4 | 11.48 % | PASS |
| 1024 | 0 | 1 | 1024 | 32 | 32 | 1.000 | 1.000 | 389.4 | 12.17 % | PASS |
| 1024 | 0 | 2 | 2048 | 64 | 64 | 1.000 | 1.000 | 371.7 | 11.62 % | PASS |
| 1024 | 0 | 4 | 4096 | 128 | 128 | 1.000 | 1.000 | 363.4 | 11.36 % | PASS |
| 1024 | 0 | 8 | 8192 | 256 | 256 | 1.000 | 1.000 | 359.5 | 11.23 % | PASS |
| 4035 | 0 | 1 | 4035 | 127 | 127 | 0.993 | 0.993 | 360.9 | 11.28 % | PASS |
| 4035 | 0 | 2 | 8070 | 254 | 254 | 0.993 | 0.993 | 358.0 | 11.19 % | PASS |
| 4035 | 0 | 4 | 16140 | 508 | 508 | 0.993 | 0.993 | 356.1 | 11.13 % | PASS |
| 4035 | 0 | 8 | 32280 | 1016 | 1016 | 0.993 | 0.993 | 355.6 | 11.11 % | PASS |
| 4096 | 0 | 1 | 4096 | 128 | 128 | 1.000 | 1.000 | 363.4 | 11.36 % | PASS |
| 4096 | 0 | 2 | 8192 | 256 | 256 | 1.000 | 1.000 | 359.5 | 11.23 % | PASS |
| 4096 | 0 | 4 | 16384 | 512 | 512 | 1.000 | 1.000 | 357.5 | 11.17 % | PASS |
| 4096 | 0 | 8 | 32768 | 1024 | 1024 | 1.000 | 1.000 | 356.5 | 11.14 % | PASS |

### 4.2 Source path

Size sweep, source path, offset 0

| Payload B | Offset | Ch | Bytes moved | AXIS beats | Mem beats | Eff (AXIS) | Eff (mem) | MB/s | % of peak | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 | 1 | 0.031 | 0.031 | 6.2 | 0.20 % | PASS |
| 1 | 0 | 2 | 2 | 2 | 2 | 0.031 | 0.031 | 11.1 | 0.35 % | PASS |
| 1 | 0 | 4 | 4 | 4 | 4 | 0.031 | 0.031 | 18.2 | 0.57 % | PASS |
| 1 | 0 | 8 | 8 | 8 | 8 | 0.031 | 0.031 | 26.7 | 0.83 % | PASS |
| 7 | 0 | 1 | 7 | 1 | 1 | 0.219 | 0.219 | 43.8 | 1.37 % | PASS |
| 7 | 0 | 2 | 14 | 2 | 2 | 0.219 | 0.219 | 73.7 | 2.30 % | PASS |
| 7 | 0 | 4 | 28 | 4 | 4 | 0.219 | 0.219 | 112.0 | 3.50 % | PASS |
| 7 | 0 | 8 | 56 | 8 | 8 | 0.219 | 0.219 | 151.4 | 4.73 % | PASS |
| 32 | 0 | 1 | 32 | 1 | 1 | 1.000 | 1.000 | 200.0 | 6.25 % | PASS |
| 32 | 0 | 2 | 64 | 2 | 2 | 1.000 | 1.000 | 256.0 | 8.00 % | PASS |
| 32 | 0 | 4 | 128 | 4 | 4 | 1.000 | 1.000 | 297.7 | 9.30 % | PASS |
| 32 | 0 | 8 | 256 | 8 | 8 | 1.000 | 1.000 | 324.1 | 10.13 % | PASS |
| 33 | 0 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 132.0 | 4.12 % | PASS |
| 33 | 0 | 2 | 66 | 4 | 4 | 0.516 | 0.516 | 183.3 | 5.73 % | PASS |
| 33 | 0 | 4 | 132 | 8 | 8 | 0.516 | 0.516 | 227.6 | 7.11 % | PASS |
| 33 | 0 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 258.8 | 8.09 % | PASS |
| 64 | 0 | 1 | 64 | 2 | 2 | 1.000 | 1.000 | 256.0 | 8.00 % | PASS |
| 64 | 0 | 2 | 128 | 4 | 4 | 1.000 | 1.000 | 297.7 | 9.30 % | PASS |
| 64 | 0 | 4 | 256 | 8 | 8 | 1.000 | 1.000 | 324.1 | 10.13 % | PASS |
| 64 | 0 | 8 | 512 | 16 | 16 | 1.000 | 1.000 | 339.1 | 10.60 % | PASS |
| 77 | 0 | 1 | 77 | 3 | 3 | 0.802 | 0.802 | 226.5 | 7.08 % | PASS |
| 77 | 0 | 2 | 154 | 6 | 6 | 0.802 | 0.802 | 270.2 | 8.44 % | PASS |
| 77 | 0 | 4 | 308 | 12 | 12 | 0.802 | 0.802 | 299.0 | 9.34 % | PASS |
| 77 | 0 | 8 | 616 | 24 | 24 | 0.802 | 0.802 | 315.9 | 9.87 % | PASS |
| 203 | 0 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 290.0 | 9.06 % | PASS |
| 203 | 0 | 2 | 406 | 14 | 14 | 0.906 | 0.906 | 317.2 | 9.91 % | PASS |
| 203 | 0 | 4 | 812 | 28 | 28 | 0.906 | 0.906 | 332.8 | 10.40 % | PASS |
| 203 | 0 | 8 | 1014 | 33 | 56 | 0.960 | 0.566 | 339.1 | 10.60 % | PASS |
| 1024 | 0 | 1 | 928 | 29 | 32 | 1.000 | 0.906 | 339.9 | 10.62 % | PASS |
| 1024 | 0 | 2 | 1056 | 33 | 64 | 1.000 | 0.516 | 341.7 | 10.68 % | PASS |
| 1024 | 0 | 4 | 1280 | 40 | 128 | 1.000 | 0.312 | 343.2 | 10.72 % | PASS |
| 1024 | 0 | 8 | 1760 | 55 | 256 | 1.000 | 0.215 | 348.5 | 10.89 % | PASS |
| 4035 | 0 | 1 | 1280 | 40 | 127 | 1.000 | 0.315 | 344.1 | 10.75 % | PASS |
| 4035 | 0 | 2 | 1760 | 55 | 254 | 1.000 | 0.217 | 347.1 | 10.85 % | PASS |
| 4035 | 0 | 4 | 2656 | 83 | 508 | 1.000 | 0.163 | 349.0 | 10.91 % | PASS |
| 4035 | 0 | 8 | 4512 | 141 | 1016 | 1.000 | 0.139 | 353.3 | 11.04 % | PASS |
| 4096 | 0 | 1 | 1280 | 40 | 128 | 1.000 | 0.312 | 344.1 | 10.75 % | PASS |
| 4096 | 0 | 2 | 1760 | 55 | 256 | 1.000 | 0.215 | 347.1 | 10.85 % | PASS |
| 4096 | 0 | 4 | 2688 | 84 | 512 | 1.000 | 0.164 | 352.3 | 11.01 % | PASS |
| 4096 | 0 | 8 | 4512 | 141 | 1024 | 1.000 | 0.138 | 352.0 | 11.00 % | PASS |

### Figure 4.1: Efficiency against payload size

efficiency vs size

### Figure 4.2: Measured MB/s against payload size, peak as the dashed line

MB/s vs size

Observations from this data:

- Sink, 8 channels, 4096 B: 357 MB/s = 11.1 % of the 3200 MB/s peak at efficiency 1.000.

- Sink, 8 channels: 1 B payload reaches 4.0 MB/s, 89x less than 4096 B; the window is dominated by fixed per-packet latency (199 cycles for one beat per channel).

- Source, 8 channels, 4096 B: 352 MB/s = 11.0 % of the 3200 MB/s peak at efficiency 1.000.

- Source, 8 channels: 1 B payload reaches 26.7 MB/s, 13x less than 4096 B; the window is dominated by fixed per-packet latency (30 cycles for one beat per channel).

## 5. Start offset

A descriptor may start mid-beat. The offset moves payload into an extra memory beat when offset + payload crosses a beat boundary, lowering memory-side efficiency while the AXIS side, which packs from lane 0, is unchanged. Offset 0 rows repeat the size sweep.

### 5.1 Sink path

Offset sweep, sink path

| Payload B | Offset | Ch | Bytes moved | AXIS beats | Mem beats | Eff (AXIS) | Eff (mem) | MB/s | % of peak | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | 0 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 275.0 | 8.59 % | PASS |
| 33 | 1 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 275.0 | 8.59 % | PASS |
| 33 | 5 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 275.0 | 8.59 % | PASS |
| 33 | 31 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 275.0 | 8.59 % | PASS |
| 203 | 0 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 369.1 | 11.53 % | PASS |
| 203 | 1 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 369.1 | 11.53 % | PASS |
| 203 | 5 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 369.1 | 11.53 % | PASS |
| 203 | 31 | 1 | 203 | 7 | 8 | 0.906 | 0.793 | 225.6 | 7.05 % | PASS |
| 1024 | 0 | 1 | 1024 | 32 | 32 | 1.000 | 1.000 | 389.4 | 12.17 % | PASS |
| 1024 | 1 | 1 | 1024 | 32 | 33 | 1.000 | 0.970 | 317.0 | 9.91 % | PASS |
| 1024 | 5 | 1 | 1024 | 32 | 33 | 1.000 | 0.970 | 318.0 | 9.94 % | PASS |
| 1024 | 31 | 1 | 1024 | 32 | 33 | 1.000 | 0.970 | 324.1 | 10.13 % | PASS |
| 4035 | 0 | 1 | 4035 | 127 | 127 | 0.993 | 0.993 | 360.9 | 11.28 % | PASS |
| 4035 | 1 | 1 | 4035 | 127 | 127 | 0.993 | 0.993 | 360.9 | 11.28 % | PASS |
| 4035 | 5 | 1 | 4035 | 127 | 127 | 0.993 | 0.993 | 361.2 | 11.29 % | PASS |
| 4035 | 31 | 1 | 4035 | 127 | 128 | 0.993 | 0.985 | 344.6 | 10.77 % | PASS |
| 33 | 0 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 129.4 | 4.04 % | PASS |
| 33 | 1 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 129.4 | 4.04 % | PASS |
| 33 | 5 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 129.4 | 4.04 % | PASS |
| 33 | 31 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 129.4 | 4.04 % | PASS |
| 203 | 0 | 8 | 1624 | 56 | 56 | 0.906 | 0.906 | 367.4 | 11.48 % | PASS |
| 203 | 1 | 8 | 1624 | 56 | 56 | 0.906 | 0.906 | 367.4 | 11.48 % | PASS |
| 203 | 5 | 8 | 1624 | 56 | 56 | 0.906 | 0.906 | 368.3 | 11.51 % | PASS |
| 203 | 31 | 8 | 1624 | 56 | 64 | 0.906 | 0.793 | 249.8 | 7.81 % | PASS |
| 1024 | 0 | 8 | 8192 | 256 | 256 | 1.000 | 1.000 | 359.5 | 11.23 % | PASS |
| 1024 | 1 | 8 | 8192 | 256 | 264 | 1.000 | 0.970 | 328.6 | 10.27 % | PASS |
| 1024 | 5 | 8 | 8192 | 256 | 264 | 1.000 | 0.970 | 328.7 | 10.27 % | PASS |
| 1024 | 31 | 8 | 8192 | 256 | 264 | 1.000 | 0.970 | 329.5 | 10.30 % | PASS |
| 4035 | 0 | 8 | 32280 | 1016 | 1016 | 0.993 | 0.993 | 355.6 | 11.11 % | PASS |
| 4035 | 1 | 8 | 32280 | 1016 | 1016 | 0.993 | 0.993 | 355.6 | 11.11 % | PASS |
| 4035 | 5 | 8 | 32280 | 1016 | 1016 | 0.993 | 0.993 | 355.7 | 11.12 % | PASS |
| 4035 | 31 | 8 | 32280 | 1016 | 1024 | 0.993 | 0.985 | 347.4 | 10.86 % | PASS |

### 5.2 Source path

Offset sweep, source path

| Payload B | Offset | Ch | Bytes moved | AXIS beats | Mem beats | Eff (AXIS) | Eff (mem) | MB/s | % of peak | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | 0 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 132.0 | 4.12 % | PASS |
| 33 | 1 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 126.9 | 3.97 % | PASS |
| 33 | 5 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 126.9 | 3.97 % | PASS |
| 33 | 31 | 1 | 33 | 2 | 2 | 0.516 | 0.516 | 126.9 | 3.97 % | PASS |
| 203 | 0 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 290.0 | 9.06 % | PASS |
| 203 | 1 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 285.9 | 8.93 % | PASS |
| 203 | 5 | 1 | 203 | 7 | 7 | 0.906 | 0.906 | 285.9 | 8.93 % | PASS |
| 203 | 31 | 1 | 203 | 7 | 8 | 0.906 | 0.793 | 285.9 | 8.93 % | PASS |
| 1024 | 0 | 1 | 928 | 29 | 32 | 1.000 | 0.906 | 339.9 | 10.62 % | PASS |
| 1024 | 1 | 1 | 928 | 29 | 33 | 1.000 | 0.879 | 339.9 | 10.62 % | PASS |
| 1024 | 5 | 1 | 928 | 29 | 33 | 1.000 | 0.879 | 339.9 | 10.62 % | PASS |
| 1024 | 31 | 1 | 928 | 29 | 33 | 1.000 | 0.879 | 339.9 | 10.62 % | PASS |
| 4035 | 0 | 1 | 1280 | 40 | 127 | 1.000 | 0.315 | 344.1 | 10.75 % | PASS |
| 4035 | 1 | 1 | 1280 | 40 | 127 | 1.000 | 0.315 | 344.1 | 10.75 % | PASS |
| 4035 | 5 | 1 | 1280 | 40 | 127 | 1.000 | 0.315 | 344.1 | 10.75 % | PASS |
| 4035 | 31 | 1 | 1280 | 40 | 128 | 1.000 | 0.312 | 344.1 | 10.75 % | PASS |
| 33 | 0 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 258.8 | 8.09 % | PASS |
| 33 | 1 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 256.3 | 8.01 % | PASS |
| 33 | 5 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 256.3 | 8.01 % | PASS |
| 33 | 31 | 8 | 264 | 16 | 16 | 0.516 | 0.516 | 256.3 | 8.01 % | PASS |
| 203 | 0 | 8 | 1014 | 33 | 56 | 0.960 | 0.566 | 339.1 | 10.60 % | PASS |
| 203 | 1 | 8 | 1014 | 33 | 56 | 0.960 | 0.566 | 339.1 | 10.60 % | PASS |
| 203 | 5 | 8 | 1014 | 33 | 56 | 0.960 | 0.566 | 339.1 | 10.60 % | PASS |
| 203 | 31 | 8 | 1046 | 34 | 64 | 0.961 | 0.511 | 344.1 | 10.75 % | PASS |
| 1024 | 0 | 8 | 1760 | 55 | 256 | 1.000 | 0.215 | 348.5 | 10.89 % | PASS |
| 1024 | 1 | 8 | 1760 | 55 | 264 | 1.000 | 0.208 | 345.1 | 10.78 % | PASS |
| 1024 | 5 | 8 | 1760 | 55 | 264 | 1.000 | 0.208 | 345.1 | 10.78 % | PASS |
| 1024 | 31 | 8 | 1760 | 55 | 264 | 1.000 | 0.208 | 345.1 | 10.78 % | PASS |
| 4035 | 0 | 8 | 4512 | 141 | 1016 | 1.000 | 0.139 | 353.3 | 11.04 % | PASS |
| 4035 | 1 | 8 | 4512 | 141 | 1016 | 1.000 | 0.139 | 353.3 | 11.04 % | PASS |
| 4035 | 5 | 8 | 4512 | 141 | 1016 | 1.000 | 0.139 | 353.3 | 11.04 % | PASS |
| 4035 | 31 | 8 | 4512 | 141 | 1024 | 1.000 | 0.138 | 352.0 | 11.00 % | PASS |

### Figure 5.1: Memory-side efficiency against start offset

efficiency vs offset

Offsets that cost an extra memory beat (sink, 1 channel): 203 B at offset 31: 7 -> 8 memory beats; 1024 B at offset 1: 32 -> 33 memory beats; 1024 B at offset 5: 32 -> 33 memory beats; 1024 B at offset 31: 32 -> 33 memory beats; 4035 B at offset 31: 127 -> 128 memory beats.

## 6. Descriptor chains

Several descriptors per channel back to back (4 channels), offset 0. Efficiency counts payload over all beats of the chain.

Descriptor chains, 4 channels

| Payload B | Descs | Path | Bytes moved | AXIS beats | Eff (AXIS) | MB/s | % of peak | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16 | sink | 64 | 64 | 0.031 | 5.4 | 0.17 % | PASS |
| 1 | 16 | source | 64 | 64 | 0.031 | 37.2 | 1.16 % | PASS |
| 64 | 4 | sink | 1024 | 32 | 1.000 | 223.6 | 6.99 % | PASS |
| 64 | 4 | source | 1024 | 32 | 1.000 | 347.1 | 10.85 % | PASS |
| 64 | 16 | sink | 4096 | 128 | 1.000 | 221.4 | 6.92 % | PASS |
| 64 | 16 | source | 3872 | 121 | 1.000 | 352.6 | 11.02 % | PASS |
| 1024 | 4 | sink | 16384 | 512 | 1.000 | 341.4 | 10.67 % | PASS |
| 1024 | 4 | source | 2656 | 83 | 1.000 | 350.9 | 10.96 % | PASS |
| 1024 | 16 | sink | 65536 | 2048 | 1.000 | 336.3 | 10.51 % | PASS |
| 1024 | 16 | source | 50432 | 1576 | 1.000 | 355.4 | 11.11 % | PASS |

## 7. Source backpressure (integrity only)

The source backpressure is paced by the host toggling the checker’s ready over UART, so the cycle counts measure the host, not the DUT (rapids TASK-085). These rows are integrity checks: bytes and beats must still match and the golden CRCs must agree. Their MB/s is not reported.

Source with backpressure, integrity rows

| Ch | Payload B | Offset | Expected bytes | Expected AXIS beats | MB/s | Result |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 33 | 0 | 33 | 2 | n/a (host-paced) | PASS |
| 1 | 33 | 5 | 33 | 2 | n/a (host-paced) | PASS |
| 1 | 1024 | 0 | 1024 | 32 | n/a (host-paced) | PASS |
| 1 | 1024 | 5 | 1024 | 32 | n/a (host-paced) | PASS |
| 4 | 33 | 0 | 132 | 8 | n/a (host-paced) | PASS |
| 4 | 33 | 5 | 132 | 8 | n/a (host-paced) | PASS |
| 4 | 1024 | 0 | 4096 | 128 | n/a (host-paced) | PASS |
| 4 | 1024 | 5 | 4096 | 128 | n/a (host-paced) | PASS |

## 8. Failures, caveats and coverage

No measured point failed.

Coverage: 117 of 117 planned points of profile standard are in this file (117 passed, 0 failed).

Standing limitations of the design, not of this measurement:

- The sink s_axis_tready is one signal qualified by TID, so a beat for a channel whose packet record has not arrived blocks every channel behind it on the stream (head-of-line blocking, inherent and documented).

- TYPE=EXT descriptors stay beat-aligned by design and are not part of the byte sweeps.

- AXI error responses (RRESP and BRESP other than OKAY) are not exercised by the board campaign or by the directed sequences: the harness memory model always answers OKAY and has no fault-injection hook, and no RAPIDS test in the tree drives a non-OKAY response on the read or write master. The response-error flag path is therefore unproven on silicon and in simulation. Closing this needs an injection hook in the harness, which is not built.

- Each interface has its own measurement window; the sin window runs from the first to the last stream beat (rapids ISSUE-001), so windows differ per interface and MB/s here uses the longer of the stream and memory windows.

## 9. Build and resources

Post-route resources and timing at 100 MHz, XC7K325T (203,800 LUTs, 445 BRAM tiles)

| Build | Slice LUTs | Slice regs | BRAM tiles | WNS setup | WHS hold | Failing endpoints | Bitstream sha256 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bytecrc | 92,874 | 82,379 | 68 | +0.301 ns | +0.023 ns | 0 | cfad34c3465f1110 |
| wordcrc | 86,065 | 79,527 | 52 | +0.077 ns | +0.013 ns | 0 | 48e1282c46e707cd |

Build configuration of each row:

- bytecrc: BYTE_CRC=1 USE_OBSERVERS=1 OBS_ENABLE_MON_TAPS=0 (channel-reset + hold-junk fixes).

- wordcrc: BYTE_CRC=0 USE_OBSERVERS=1 OBS_ENABLE_MON_TAPS=0. RTL commit d1d20b046.

Every build closes timing at 100 MHz with no failing endpoint. The slack is small and positive; treat 100 MHz as the design point, not as margin. The resource figures come from the post-route utilization report; the word-wide checker build is a measurement bitstream and is not the standard one.

## 10. Provenance and reproduction

Provenance

| Item | Value |
| --- | --- |
| Results file | rapids_byte_perf_20260930_142731.json |
| Timestamp | 2026-09-30T14:27:36 |
| Profile | standard |
| Status | final |
| Bitstream | rapids_byte.bit sha256 cfad34c3465f1110 |
| Design | 256-bit, 8 channels, 4096 B SRAM per channel, 100 MHz, peak 3200 MB/s per direction |
| Beats reference | genesys_dw256_obs_C.json |

Device readback per session

| Session | Bitstream sha256 | CSR_ID | BUILD | Sentinel start | Sentinel end | Stable | Aborted |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | cfad34c3465f1110 | 0x52415042 | 0x0A070820 | 0x00000000 | 0x00000000 | yes | no |

The sentinel is the MON_LIMIT register written once at configure time. It resets on any FPGA reconfiguration, so a change between readbacks means the device was reprogrammed during the run. A point measured across such a change is dropped, not recorded. Device stable for the whole run: yes.

One command runs the campaign and regenerates this report:

cd projects/fpga-systems/Genesys2/rapids/flows-rapids
./byte_perf.sh --profile full --final      # final numbers (standard byte-CRC bitstream)
./byte_perf.sh --profile standard          # quick run, writes *_prelim_*.json

The word-wide aligned results are rapids_byte_aligned_wordcrc_20260930.json; its device readback follows.

Device readback per session

| Session | Bitstream sha256 | CSR_ID | BUILD | Sentinel start | Sentinel end | Stable | Aborted |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 48e1282c46e707cd | 0x52415042 | 0x1A070820 | 0x00000000 | 0x00000000 | yes | no |

The sentinel is the MON_LIMIT register written once at configure time. It resets on any FPGA reconfiguration, so a change between readbacks means the device was reprogrammed during the run. A point measured across such a change is dropped, not recorded. Device stable for the whole run: yes.