RTL Design Sherpa

RAPIDS Beats Micro-Architecture Specification 0.10

July 2, 2026

Table of Contents

1 Rapids Beats Mas Index	39

2 Document Information	40

2.1 References	41

2.1.1 Related Documents	41

2.2 Terminology	42

2.3 “Beats” Architecture Concept	44

2.4 Revision History	45

3 Architecture Overview	51

3.1 Overview	52

3.1.1 Key Features	52

3.1.2 Block Diagram	52

3.1.3 Figure 1.1.1: RAPIDS Beats Architecture Block Diagram	52

3.2 Data Flow Overview	54

3.2.1 Sink Path (Network to Memory)	54

3.2.2 Figure 1.1.2: Sink Path Data Flow	54

3.2.3 Source Path (Memory to Network)	54

3.2.4 Figure 1.1.3: Source Path Data Flow	54

3.3 Key Architectural Decisions	56

3.3.1 Beat-Level Tracking	56

3.3.2 Concurrent Read/Write	56

3.3.3 Virtual FIFOs (Alloc/Drain)	56

3.4 Module Hierarchy	58

3.5 Timing Diagram: Basic Transfer	60

3.5.1 Figure 1.1.4: Basic Sink Path Transfer Timing	60

3.6 Related Documentation	61

4 Top-Level Port List	62

4.1 Parameters	63

4.2 Port List (324 ports)	65

4.2.1 Clock and Reset (shared - single clock/reset for the whole wrapper)	65

4.3 SOURCE HALF (u_src) - shared-infrastructure ports (src_ prefixed)	66

4.3.1 APB Programming Interface	66

4.3.2 Per-channel configuration	66

4.3.3 Scheduler Configuration (global)	66

4.3.4 Descriptor Engine Configuration (global)	67

4.3.5 Control Engine Configuration (Phase 2, global)	67

4.3.6 Descriptor AXI Monitor Configuration	67

4.3.7 Network-plane monitor input (rapids TASK-024)	68

4.3.8 Status	69

4.3.9 Descriptor AXI Monitor Status	69

4.3.10 Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).	70

4.3.11 Descriptor Fetch AXI Master	70

4.3.12 Control Read AXI Master (32-bit) [Phase 2]	71

4.3.13 Control Write AXI Master (32-bit) [Phase 2]	72

4.4 SOURCE HALF (u_src) - direction-unique ports (no prefix)	74

4.4.1 AXI Transfer Configuration (source-only)	74

4.4.2 Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id	74

4.4.3 AXI4 Master - Data Read (Memory -> Source SRAM)	74

4.5 SINK HALF (u_snk) - shared-infrastructure ports (snk_ prefixed)	76

4.5.1 APB Programming Interface	76

4.5.2 Per-channel configuration	76

4.5.3 Scheduler Configuration (global)	76

4.5.4 Descriptor Engine Configuration (global)	77

4.5.5 Control Engine Configuration (Phase 2, global)	77

4.5.6 Descriptor AXI Monitor Configuration	77

4.5.7 Network-plane monitor input (rapids TASK-024)	78

4.5.8 Status	79

4.5.9 Descriptor AXI Monitor Status	79

4.5.10 Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).	80

4.5.11 Descriptor Fetch AXI Master	80

4.5.12 Control Read AXI Master (32-bit) [Phase 2]	81

4.5.13 Control Write AXI Master (32-bit) [Phase 2]	82

4.6 Monitor Bus (SINGLE aggregated stream for the whole core)	84

4.6.1 Monitor Bus	84

4.7 SINK HALF (u_snk) - direction-unique ports (no prefix)	85

4.7.1 AXI Transfer Configuration (sink-only)	85

4.7.2 Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id	85

4.7.3 AXI4 Master - Data Write (Sink SRAM -> Memory)	85

4.8 Debug Interface (per half, src_dbg_ / snk_dbg_ prefixed)	87

4.8.1 Source half debug	87

4.8.2 Sink half debug	87

4.8.3 Active-channel sideband for per-channel bus instrumentation	88

5 Clocks and Reset	89

5.1 Clock Domain	90

5.1.1 Clock Specifications	90

5.1.2 Clock Distribution	90

5.2 Reset Specification	91

5.2.1 Reset Signal	91

5.2.2 Reset Timing Diagram	91

5.2.3 Figure 1.3.1: Reset Timing	91

5.3 Reset Behavior	93

5.3.1 On Reset Assert	93

5.3.2 Reset Sequence	93

5.4 Configuration During Reset	94

5.5 Reset Macros	95

5.6 Power-On Reset Considerations	96

6 Scheduler Specification	97

6.1 Overview	98

6.1.1 Key Features	98

6.1.2 Block Diagram	98

6.1.3 Figure 2.1.1: Scheduler Block Diagram	98

6.2 Parameters	100

6.3 Port List	101

6.3.1 Clock and Reset	101

6.3.2 Configuration Interface	101

6.3.3 Descriptor Engine Interface	101

6.3.4 Data Read Interface	102

6.3.5 Data Write Interface	102

6.3.6 Control Interface	103

6.4 FSM States	105

6.4.1 Figure 2.1.2: Scheduler FSM State Diagram	105

6.4.2 State Encoding	107

6.5 Operation	108

6.5.1 Descriptor Format	108

6.5.2 Transfer Sequence	108

6.5.3 Figure 2.1.3: Basic Transfer Timing	108

6.6 Run-Base Generator Flush	109

6.7 Error Handling	110

6.7.1 Recoverable Write-Progress Timeout	110

6.8 MonBus Events	111

7 Descriptor Engine Specification	112

7.1 Overview	113

7.1.1 Key Features	113

7.1.2 Block Diagram	113

7.1.3 Figure 2.2.1: Descriptor Engine Block Diagram	113

7.2 Parameters	114

7.3 Port List	115

7.3.1 Clock and Reset	115

7.3.2 APB Programming Interface	115

7.3.3 Configuration Interface	115

7.3.4 AXI4 Read Master Interface	116

7.3.5 Scheduler Interface	117

7.4 FSM States	118

7.4.1 Figure 2.2.2: Descriptor Engine FSM	118

7.5 Operation	119

7.5.1 Descriptor Chain Processing	119

7.5.2 Figure 2.2.3: Descriptor Chain Timing	119

7.6 Address Range Checking	120

7.7 Prefetch and Autonomous Chaining	121

7.7.1 Chaining Decision	121

7.7.2 Deferred (Throttle-Blocked) Chain Fetch	121

8 AXI Read Engine Specification	123

8.1 Overview	124

8.1.1 Key Features	124

8.1.2 Block Diagram	124

8.1.3 Figure 2.3.1: AXI Read Engine Block Diagram	124

8.2 Parameters	126

8.3 Port List	127

8.3.1 Clock and Reset	127

8.3.2 Scheduler Interface	127

8.3.3 AXI4 Read Master Interface	127

8.3.4 SRAM Allocation and Fill Interface	128

8.4 Operation	129

8.4.1 Burst Length Cap (rapids BUG-009)	129

8.4.2 Streaming Pipeline Architecture	129

8.4.3 Timing Diagram	130

8.4.4 Figure 2.3.2: AXI Read Burst Timing	130

8.5 Burst Segmentation	131

8.6 Error Handling	132

9 AXI Write Engine Specification	133

9.1 Overview	134

9.1.1 Key Features	134

9.1.2 Block Diagram	135

9.1.3 Figure 2.4.1: AXI Write Engine Block Diagram	135

9.2 Parameters	136

9.3 Port List	137

9.3.1 Clock and Reset	137

9.3.2 Scheduler Interface	137

9.3.3 AXI4 Write Master Interface	138

9.3.4 SRAM Reservation and Drain Interface	139

9.4 Operation	140

9.4.1 Burst Length Cap (rapids BUG-009)	140

9.4.2 Write Transaction Phases	140

9.4.3 Timing Diagram	141

9.4.4 Figure 2.4.2: AXI Write Burst Timing	141

9.5 Error Handling	143

10 Beats Alloc Control Specification	144

10.1 Overview	145

10.1.1 Key Features	145

10.1.2 Block Diagram	145

10.1.3 Figure 2.5.1: Beats Alloc Control Block Diagram	145

10.2 Concept: Virtual FIFO	147

10.2.1 Use Case: Pre-Allocation for AXI Reads	147

10.3 Parameters	148

10.4 Port List	149

10.4.1 Clock and Reset	149

10.4.2 Write Interface (Allocation Requests)	149

10.4.3 Read Interface (Data Written)	149

10.4.4 Status Outputs	149

10.5 Operation	151

10.5.1 Pointer Arithmetic	151

10.5.2 Timing Diagram	151

10.5.3 Figure 2.5.2: Allocation and Release Timing	151

10.6 Integration Example	152

11 Beats Drain Control Specification	153

11.1 Overview	154

11.1.1 Key Features	154

11.1.2 Block Diagram	154

11.1.3 Figure 2.6.1: Beats Drain Control Block Diagram	154

11.2 Concept: Data Availability Tracking	156

11.2.1 Complementary Roles	156

11.3 Parameters	157

11.4 Port List	158

11.4.1 Clock and Reset	158

11.4.2 Write Interface (Data Written to SRAM)	158

11.4.3 Read Interface (Drain Requests)	158

11.4.4 Status Outputs	158

11.5 Operation	160

11.5.1 Pointer Arithmetic	160

11.5.2 Timing Diagram	160

11.5.3 Figure 2.6.2: Data Arrival and Drain Timing	160

11.6 Integration Example	161

11.7 Relationship: alloc_ctrl vs drain_ctrl	162

12 Latency Bridge (beats) Specification	163

12.1 Overview	164

12.1.1 Key Features	164

12.1.2 Block Diagram	165

12.1.3 Figure 2.7.1: Latency Bridge Block Diagram	165

12.2 Concept: bridging a registered read	166

12.3 Parameters	167

12.4 Port List	168

12.4.1 Clock and Reset	168

12.4.2 Upstream Interface (from the registered FIFO)	168

12.4.3 Downstream Interface (to the consumer)	168

12.4.4 Status and Debug	169

12.5 Operation	170

12.5.1 Figure 2.7.2: Latency Bridge Timing (consumer always ready)	170

12.6 Integration Context	171

12.7 Design Considerations	172

13 Control-Read Engine Specification	173

13.1 Overview	174

13.1.1 Key Features	174

13.1.2 Block Diagram	175

13.1.3 Figure 2.8.1: Control-Read Engine Block Diagram	175

13.2 Parameters	176

13.3 Port List	177

13.3.1 Clock and Reset	177

13.3.2 Scheduler Interface	177

13.3.3 Configuration Interface	178

13.3.4 AXI4 Read Master Interface	178

13.3.5 MonBus Interface	179

13.4 FSM States	180

13.4.1 Figure 2.8.2: Control-Read Engine FSM	180

13.5 Operation	182

13.5.1 Masked Compare	182

13.5.2 Retry Budget and Pacing	182

13.5.3 Null Address Fast-Path	182

13.5.4 AXI Read Transaction	182

13.5.5 Channel Reset	182

13.6 MonBus Reporting	184

13.7 Descriptor Field Mapping	185

14 Control-Write Engine Specification	186

14.1 Overview	187

14.1.1 Key Features	187

14.1.2 Block Diagram	188

14.1.3 Figure 2.9.1: Control-Write Engine Block Diagram	188

14.2 Parameters	189

14.3 Port List	190

14.3.1 Clock and Reset	190

14.3.2 Configuration Interface	190

14.3.3 Scheduler Interface	190

14.3.4 AXI4 Write Master Interface	191

14.3.5 MonBus Interface	192

14.4 FSM States	193

14.4.1 Figure 2.9.2: Control-Write Engine FSM	193

14.5 Operation	195

14.5.1 Doorbell Write Sequence	195

14.5.2 Figure 2.9.3: Control-Write Doorbell Timing	195

14.5.3 Request Handshake and Skid Buffer	196

14.5.4 Address Validation	196

14.5.5 Error Reporting	196

14.5.6 Channel Reset	196

14.6 Contrast with the Control-Read Engine	198

15 Beats Scheduler Group Specification	199

15.1 Overview	200

15.1.1 Key Features	200

15.1.2 Block Diagram	200

15.1.3 Figure 3.1.1: Beats Scheduler Group Block Diagram	200

15.2 Parameters	201

15.3 Port List	202

15.3.1 Clock and Reset	202

15.3.2 APB Programming Interface	202

15.3.3 Configuration Interface	202

15.3.4 Descriptor AXI Master Interface	204

15.3.5 Scheduler Data Interfaces	205

15.3.6 Status	205

15.3.7 MonBus Interface	206

15.4 Internal Architecture	207

16 Beats Scheduler Group Array Specification	208

16.1 Overview	209

16.1.1 Key Features	209

16.1.2 Block Diagram	209

16.1.3 Figure 3.2.1: Beats Scheduler Group Array Block Diagram	209

16.2 Parameters	210

16.3 Port List	211

16.3.1 Clock and Reset	211

16.3.2 Per-Channel APB Programming	211

16.3.3 Per-Channel Configuration	211

16.3.4 Shared Descriptor AXI Master Interface	212

16.3.5 Per-Channel Scheduler Data Interfaces	213

16.3.6 Aggregate Status	213

16.3.7 Unified MonBus Interface	214

16.3.8 Descriptor-AXI Perf Window	214

16.4 AXI Arbitration	216

16.4.1 Figure 3.2.2: Descriptor AXI Arbitration Sequence	216

16.5 MonBus Aggregation	217

16.6 Integration Context	218

17 Sink Data Path Specification	220

17.1 Overview	221

17.1.1 Key Features	221

17.1.2 Block Diagram	221

17.1.3 Figure 3.3.1: Sink Data Path Block Diagram	221

17.2 Parameters	222

17.3 Data Flow	223

17.3.1 Figure 3.3.2: Sink Path Data Flow Sequence	223

17.4 Timing Diagram	224

17.4.1 Figure 3.3.3: Sink Path Transfer Timing	224

17.5 Interface Summary	226

17.5.1 Fill Interface (Input)	226

17.5.2 Scheduler Interface	226

18 Sink Data Path AXIS Wrapper Specification	228

18.1 Overview	229

18.1.1 Key Features	229

18.1.2 Block Diagram	229

18.1.3 Figure 3.4.1: Sink Data Path AXIS Block Diagram	229

18.2 Parameters	230

18.3 Port List	231

18.3.1 Clock and Reset	231

18.3.2 AXI-Stream Slave Interface	231

18.3.3 Allocation Configuration	231

18.3.4 Scheduler Interface	232

18.3.5 AXI Write Master Interface	232

18.4 Signal Mapping	234

18.4.1 Figure 3.4.2: AXIS to Fill Interface Mapping	234

18.5 Timing Diagram	235

18.5.1 Figure 3.4.3: AXIS Ingress Timing	235

18.6 Usage Notes	236

19 Sink SRAM Controller Specification	237

19.1 Overview	238

19.1.1 Key Features	238

19.1.2 Block Diagram	239

19.1.3 Figure 3.5.1: Sink SRAM Controller Block Diagram	239

19.2 Parameters	240

19.3 Port List	241

19.3.1 Clock and Reset	241

19.3.2 Fill Interface (ID-Selected)	241

19.3.3 Arbitrated Drain Interface (to AXI Write Engine)	241

19.3.4 Debug Interface	242

19.4 Drain Selection	243

19.4.1 Figure 3.5.2: Drain Selection by the Consumer	243

19.5 Integration with AXI Write Engine	244

20 Source Data Path Specification	245

20.1 Overview	246

20.1.1 Key Features	246

20.1.2 Block Diagram	246

20.1.3 Figure 3.6.1: Source Data Path Block Diagram	246

20.2 Parameters	248

20.3 Data Flow	249

20.3.1 Figure 3.6.2: Source Path Data Flow Sequence	249

20.4 Port List	250

20.4.1 Clock and Reset	250

20.4.2 Configuration	250

20.4.3 Scheduler Interface	250

20.4.4 AXI Read Master Interface	250

20.4.5 Drain Interface (Output to Network)	251

20.5 Timing Diagram	253

20.5.1 Figure 3.6.3: Source Path Transfer Timing	253

20.6 Integration Example	254

21 Source Data Path AXIS Wrapper Specification	255

21.1 Overview	256

21.1.1 Key Features	256

21.1.2 Block Diagram	256

21.1.3 Figure 3.7.1: Source Data Path AXIS Block Diagram	256

21.2 Parameters	257

21.3 Port List	258

21.3.1 Clock and Reset	258

21.3.2 Configuration	258

21.3.3 Scheduler Interface	258

21.3.4 AXI Read Master Interface	259

21.3.5 AXI-Stream Master Interface	259

21.4 Signal Mapping	261

21.4.1 Figure 3.7.2: Drain to AXIS Interface Mapping	261

21.5 Timing Diagram	262

21.5.1 Figure 3.7.3: AXIS Egress Timing	262

21.6 Usage Notes	263

21.7 Integration Example	264

22 Source SRAM Controller Specification	265

22.1 Overview	266

22.1.1 Key Features	266

22.1.2 Block Diagram	267

22.1.3 Figure 3.8.1: Source SRAM Controller Block Diagram	267

22.2 Parameters	268

22.3 Port List	269

22.3.1 Clock and Reset	269

22.3.2 Fill Interface (ID-Selected, from AXI Read Engine)	269

22.3.3 Arbitrated Drain Interface (to Network)	269

22.3.4 Debug Interface	270

22.4 Drain Selection	271

22.4.1 Figure 3.8.2: Drain Selection by the Consumer	271

22.5 Comparison: Sink vs Source SRAM Controller	272

22.6 Integration Example	273

23 RAPIDS Core Beats Specification	274

23.1 Overview	275

23.1.1 Key Features	275

23.1.2 Block Diagram	275

23.1.3 Figure 3.9.1: RAPIDS Core Beats Block Diagram	275

23.2 Parameters	276

23.3 Port List	277

23.3.1 Clock and Reset	277

23.3.2 Per-Channel APB Programming	277

23.3.3 Per-Channel Configuration	277

23.3.4 Descriptor AXI Master Interfaces	279

23.3.5 Sink AXI Write Master Interface	280

23.3.6 Source AXI Read Master Interface	281

23.3.7 Sink Ingress – AXIS Slave	282

23.3.8 Source Egress – AXIS Master	283

23.3.9 Unified MonBus Interface	283

23.3.10 Aggregate Status	284

23.4 Data Flow	285

23.4.1 Figure 3.9.2: RAPIDS Core Complete Data Flow	285

23.5 MonBus Aggregation	286

23.6 Integration Example	287

24 RAPIDS Registers Specification	290

24.1 Overview	291

24.2 Base Register Map (0x100-0x3FF)	292

24.2.1 Key Register Fields	294

24.3 Monitor Register Map (rapids_mon_regs @ 0x1000)	295

25 RAPIDS Config Block Specification	297

25.1 Overview	298

25.2 Mapping Groups	299

25.2.1 Base Configuration	299

25.2.2 Monitor Configuration (hwif_out.MON.*)	299

25.2.3 Performance and Observation	300

26 RAPIDS Beats Top Specification	301

26.1 Overview	302

26.2 Integration Datapath	303

26.2.1 Figure 3.12.1: RAPIDS Beats Top Integration	303

26.2.2 Address Decode	303

26.3 Write-engine pipelining (PIPELINE)	304

26.4 AXI Monitors (USE_AXI_MONITORS)	305

26.4.1 Monitor registers on a monitors-off build (USE_MON_REGS)	305

26.5 Parameters	307

26.6 Top-Level Interfaces	308

27 Network Planes Specification	310

27.1 Overview	311

27.2 Generated Tops	312

27.3 CXS Planes	313

27.3.1 Figure 3.13.1: CXS Network Planes	313

27.3.2 Receive (sink) plane	313

27.3.3 Transmit (source) plane	314

27.3.4 Per-stream framing	314

27.3.5 Parameters	314

27.3.6 Monitoring	314

27.4 Proposed Changes	316

27.5 Verification	317

28 Interface Overview	318

28.1 External Interfaces	319

28.1.1 AXI4 Master Interfaces	319

28.1.2 Streaming Interfaces	319

28.1.3 Control/Monitor Interfaces	320

28.2 Interface Documentation	321

28.3 Interface Signal Naming Conventions	322

29 AXI4 Interface Specification	323

29.1 Overview	324

29.2 Descriptor AXI Master Interface	325

29.2.1 Purpose	325

29.2.2 Signal Table	325

29.2.3 Transaction Characteristics	326

29.2.4 Timing Diagram	326

29.2.5 Figure 4.1.1: Descriptor Fetch Timing	326

29.3 Sink AXI Write Master Interface	328

29.3.1 Purpose	328

29.3.2 Signal Table	328

29.3.3 Transaction Characteristics	329

29.3.4 Timing Diagram	329

29.3.5 Figure 4.1.2: Sink AXI Write Burst Timing	329

29.4 Source AXI Read Master Interface	331

29.4.1 Purpose	331

29.4.2 Signal Table	331

29.4.3 Transaction Characteristics	331

29.4.4 Timing Diagram	332

29.4.5 Figure 4.1.3: Source AXI Read Burst Timing	332

29.5 AXI Protocol Compliance	334

29.5.1 Supported Features	334

29.5.2 Error Handling	334

30 AXI-Stream Interface Specification	335

30.1 Overview	336

30.2 AXIS Sink Slave Interface	337

30.2.1 Purpose	337

30.2.2 Signal Table	337

30.2.3 Signal Details	337

30.2.4 Timing Diagram	339

30.2.5 Figure 4.2.1: AXIS Sink Slave Timing	339

30.2.6 Backpressure	339

30.3 AXIS Source Master Interface	340

30.3.1 Purpose	340

30.3.2 Signal Table	340

30.3.3 Timing Diagram	341

30.3.4 Figure 4.2.2: AXIS Source Master Timing	341

30.3.5 Flow Control	341

30.4 AXIS vs Custom Fill/Drain Interfaces	342

30.4.1 Comparison	342

30.4.2 Selection Guide	342

30.5 Configuration Parameters	343

31 MonBus Interface Specification	344

31.1 Overview	345

31.2 MonBus Signal Interface	346

31.2.1 Signal Table	346

31.2.2 Handshaking Protocol	346

31.2.3 Timing Diagram	346

31.2.4 Figure 4.3.1: MonBus Packet Transfer Timing	346

31.3 MonBus Packet Format	348

31.3.1 64-bit Packet Structure	348

31.3.2 Figure 4.3.2: MonBus Packet Format	348

31.4 Packet Types	349

31.4.1 Type Code Definitions	349

31.4.2 Payload Formats by Type	349

31.5 MonBus Source Aggregation	351

31.5.1 RAPIDS Core MonBus Sources	351

31.5.2 Aggregation Hierarchy	352

31.5.3 Figure 4.3.3: MonBus Aggregation Tree	352

31.6 Integration Guidelines	353

31.6.1 Consumer Requirements	353

31.6.2 Recommended Consumer Patterns	353

31.7 Timing Requirements	354

32 CXS Interface Specification	355

32.1 Overview	356

32.2 Link Profile	357

32.3 Signal Table	358

32.4 Credit Flow	360

32.5 Link Activation	361

32.6 Backpressure	362

32.7 Proposed Changes	363

33 Product Requirements Document (PRD)	364

33.1 RAPIDS - Rapid AXI Programmable In-band Descriptor System	364

33.2 1. Executive Summary	365

33.2.1 1.1 Quick Stats	365

33.2.2 1.2 Subsystem Goals	365

33.3 2. Documentation Structure	367

33.3.1 Complete RAPIDS Specification	367

33.3.2 Known Issues	367

33.3.3 Other Documentation	368

33.4 2.4 Organizational Standards - RAPIDS Code Location	369

33.4.1 Code Organization Principle	369

33.4.2 RAPIDS Directory Structure	369

33.4.3 What Goes Where?	369

33.4.4 Import Pattern for RAPIDS Tests	370

33.4.5 Benefits of This Organization	370

33.4.6 Compliance Status	371

33.5 3. Architecture Overview	372

33.5.1 3.1 Top-Level Block Diagram	372

33.5.2 3.2 Data Flow	372

33.6 4. Key Features	373

33.6.1 4.1 Descriptor-Based Operation	373

33.6.2 4.2 Data Path Features	373

33.6.3 4.3 Scheduler Features	373

33.6.4 4.4 Monitoring Integration	374

33.7 5. Interfaces	375

33.7.1 5.1 External Interfaces	375

33.7.2 5.2 Configuration Parameters	375

33.8 6. Use Cases	377

33.8.1 6.1 DMA-Style Transfers	377

33.8.2 6.2 Network Packet Processing	377

33.8.3 6.3 Custom Data Path Acceleration	377

33.9 7. Test Coverage	378

33.9.1 7.1 Current Status	378

33.9.2 7.2 Test Strategy	379

33.10 8. Known Issues Summary	380

33.10.1 8.1 Critical Issues	380

33.10.2 8.2 Medium Priority Issues	380

33.11 9. Integration Guidelines	381

33.11.1 9.1 Quick Start	381

33.11.2 9.2 Configuration Steps	381

33.12 10. Development Status	383

33.12.1 10.1 Current Phase	383

33.12.2 10.2 Roadmap	383

33.13 11. Performance Characteristics	384

33.13.1 11.1 Throughput	384

33.13.2 11.2 Latency	384

33.13.3 11.3 Resource Utilization	384

33.14 12. Verification Infrastructure	385

33.14.1 12.1 Test Organization	385

33.14.2 12.2 CocoTB Framework	385

33.14.3 12.2.1 MANDATORY: BFM Usage for FUB Tests	385

33.14.4 12.3 Test File Structure (Standard Pattern)	386

33.15 13. Quick Reference	392

33.15.1 13.1 Key Files	392

33.15.2 13.2 Commands	392

33.16 14. Success Criteria	394

33.16.1 14.1 Functional	394

33.16.2 14.2 Quality	394

33.16.3 14.3 Documentation	394

33.17 15. Educational Value	395

33.18 16. Documentation Generation	396

33.18.1 16.1 Generating PDF/DOCX from Specification	396

33.19 16.2 PDF Generation Location	398

33.20 17. References	399

33.20.1 16.1 Internal Documentation	399

33.20.2 16.2 Related Subsystems	399

33.20.3 16.3 External References	399

33.21 Navigation	400

34 Claude Code Guide: RAPIDS Subsystem	401

34.1 Quick Context	402

34.2 Global Requirements Reference	408

34.3 Critical Rules for This Subsystem	409

34.3.1 Rule #0.1: Testbench Location and Test Structure (MANDATORY)	409

34.3.2 Rule #0.5: Config Before Reset (RAPIDS-specific)	409

34.3.3 Rule #0.75: Audit Signal Naming BEFORE Writing Testbenches	410

34.3.4 Rule #1: MANDATORY BFM Usage for FUB-Level Tests	410

34.3.5 Rule #2: Know the Known Issues	410

34.3.6 Rule #3: RAPIDS is Complex - Understand Block Interactions	411

34.3.7 Rule #4: Test Strategy is Multi-Layered	411

34.4 Architecture Quick Reference	413

34.4.1 Block Organization	413

34.4.2 Module Quick Reference	413

34.4.3 Interface Summary	414

34.5 Common User Questions and Responses	416

34.5.1 Q: “How does the scheduler work?”	416

34.5.2 Q: “How do I configure RAPIDS?”	416

34.5.3 Q: “What’s the data flow for network to memory transfer?”	417

34.5.4 Q: “What’s the credit counter bug and how does exponential encoding work?”	418

34.5.5 Q: “How do I run RAPIDS tests?”	419

34.6 Integration Patterns	420

34.6.1 Pattern 1: Basic RAPIDS Instantiation	420

34.6.2 Pattern 2: Configuration Sequence	421

34.6.3 Pattern 3: MonBus Integration	422

34.7 Anti-Patterns to Catch	423

34.7.1 Anti-Pattern 1: Not Understanding Exponential Credit Encoding	423

34.7.2 Anti-Pattern 2: Insufficient SRAM Depth	423

34.7.3 Anti-Pattern 3: No MonBus Downstream Handling	423

34.7.4 Anti-Pattern 4: Testing Individual Blocks in Isolation	423

34.8 Debugging Workflow	425

34.8.1 Issue: Scheduler Not Processing Descriptors	425

34.8.2 Issue: Data Path Stalls	425

34.8.3 Issue: MonBus Packets Not Generated	425

34.9 Testing Guidance	426

34.9.1 Running Tests	426

34.9.2 Test Coverage Status	426

34.10 Key Documentation Links	427

34.11 Verification Patterns	428

34.12 Documentation Generation	429

35 RAPIDS Beats Architecture HAS	430

35.1 Document Organization	431

35.1.1 Front Matter	431

35.1.2 Chapter 1: Overview	431

35.1.3 Chapter 2: Architecture	431

35.1.4 Chapter 3: Interfaces	431

35.1.5 Chapter 4: Use Cases	431

35.1.6 Chapter 5: Programming Model	431

35.1.7 Chapter 6: Performance	432

35.2 Quick Reference	433

35.2.1 RAPIDS Beats at a Glance	433

35.2.2 Interface Summary	433

35.3 Related Documentation	434

35.4 Specification Conventions	435

35.4.1 Signal Naming	435

35.4.2 Diagram Conventions	435

List of Figures

Figure 1.1.1: RAPIDS Beats Architecture Block Diagram	52

Figure 1.1.2: Sink Path Data Flow	54

Figure 1.1.3: Source Path Data Flow	54

Figure 1.1.4: Basic Sink Path Transfer Timing	60

Figure 1.3.1: Reset Timing	91

Figure 2.1.1: Scheduler Block Diagram	98

Figure 2.1.2: Scheduler FSM State Diagram	105

Figure 2.1.3: Basic Transfer Timing	108

Figure 2.2.1: Descriptor Engine Block Diagram	113

Figure 2.2.2: Descriptor Engine FSM	118

Figure 2.2.3: Descriptor Chain Timing	119

Figure 2.3.1: AXI Read Engine Block Diagram	124

Figure 2.3.2: AXI Read Burst Timing	130

Figure 2.4.1: AXI Write Engine Block Diagram	135

Figure 2.4.2: AXI Write Burst Timing	141

Figure 2.5.1: Beats Alloc Control Block Diagram	145

Figure 2.5.2: Allocation and Release Timing	151

Figure 2.6.1: Beats Drain Control Block Diagram	154

Figure 2.6.2: Data Arrival and Drain Timing	160

Figure 2.7.1: Latency Bridge Block Diagram	165

Figure 2.7.2: Latency Bridge Timing (consumer always ready)	170

Figure 2.8.1: Control-Read Engine Block Diagram	175

Figure 2.8.2: Control-Read Engine FSM	180

Figure 2.9.1: Control-Write Engine Block Diagram	188

Figure 2.9.2: Control-Write Engine FSM	193

Figure 2.9.3: Control-Write Doorbell Timing	195

Figure 3.1.1: Beats Scheduler Group Block Diagram	200

Figure 3.2.1: Beats Scheduler Group Array Block Diagram	209

Figure 3.2.2: Descriptor AXI Arbitration Sequence	216

Figure 3.3.1: Sink Data Path Block Diagram	221

Figure 3.3.2: Sink Path Data Flow Sequence	223

Figure 3.3.3: Sink Path Transfer Timing	224

Figure 3.4.1: Sink Data Path AXIS Block Diagram	229

Figure 3.4.2: AXIS to Fill Interface Mapping	234

Figure 3.4.3: AXIS Ingress Timing	235

Figure 3.5.1: Sink SRAM Controller Block Diagram	239

Figure 3.5.2: Drain Selection by the Consumer	243

Figure 3.6.1: Source Data Path Block Diagram	246

Figure 3.6.2: Source Path Data Flow Sequence	249

Figure 3.6.3: Source Path Transfer Timing	253

Figure 3.7.1: Source Data Path AXIS Block Diagram	256

Figure 3.7.2: Drain to AXIS Interface Mapping	261

Figure 3.7.3: AXIS Egress Timing	262

Figure 3.8.1: Source SRAM Controller Block Diagram	267

Figure 3.8.2: Drain Selection by the Consumer	271

Figure 3.9.1: RAPIDS Core Beats Block Diagram	275

Figure 3.9.2: RAPIDS Core Complete Data Flow	285

Figure 3.12.1: RAPIDS Beats Top Integration	303

Figure 3.13.1: CXS Network Planes	313

Figure 4.1.1: Descriptor Fetch Timing	326

Figure 4.1.2: Sink AXI Write Burst Timing	329

Figure 4.1.3: Source AXI Read Burst Timing	332

Figure 4.2.1: AXIS Sink Slave Timing	339

Figure 4.2.2: AXIS Source Master Timing	341

Figure 4.3.1: MonBus Packet Transfer Timing	346

Figure 4.3.2: MonBus Packet Format	348

Figure 4.3.3: MonBus Aggregation Tree	352

List of Tables

Table 1: Table 0.1: Related Documents and Specifications	41

Table 2: Table 0.2: RAPIDS Beats MAS Document Revision History	45

Table 3: Table 1.1.1: Beat Granularity Examples	56

Table 4: Table 1.2.1: Parameters (from rapids_core_beats.sv)	63

Table 5: Table 1.2.2: Clock and Reset (shared - single clock/reset for the whole wrapper)	65

Table 6: Table 1.2.3: APB Programming Interface	66

Table 7: Table 1.2.4: Per-channel configuration	66

Table 8: Table 1.2.5: Scheduler Configuration (global)	66

Table 9: Table 1.2.6: Descriptor Engine Configuration (global)	67

Table 10: Table 1.2.7: Control Engine Configuration (Phase 2, global)	67

Table 11: Table 1.2.8: Descriptor AXI Monitor Configuration	67

Table 12: Table 1.2.9: Network-plane monitor input (rapids TASK-024)	69

Table 13: Table 1.2.10: Status	69

Table 14: Table 1.2.11: Descriptor AXI Monitor Status	69

Table 15: Table 1.2.12: Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).	70

Table 16: Table 1.2.13: Descriptor Fetch AXI Master	70

Table 17: Table 1.2.14: Control Read AXI Master (32-bit) [Phase 2]	71

Table 18: Table 1.2.15: Control Write AXI Master (32-bit) [Phase 2]	72

Table 19: Table 1.2.16: AXI Transfer Configuration (source-only)	74

Table 20: Table 1.2.17: Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id	74

Table 21: Table 1.2.18: AXI4 Master - Data Read (Memory -> Source SRAM)	74

Table 22: Table 1.2.19: APB Programming Interface	76

Table 23: Table 1.2.20: Per-channel configuration	76

Table 24: Table 1.2.21: Scheduler Configuration (global)	76

Table 25: Table 1.2.22: Descriptor Engine Configuration (global)	77

Table 26: Table 1.2.23: Control Engine Configuration (Phase 2, global)	77

Table 27: Table 1.2.24: Descriptor AXI Monitor Configuration	77

Table 28: Table 1.2.25: Network-plane monitor input (rapids TASK-024)	79

Table 29: Table 1.2.26: Status	79

Table 30: Table 1.2.27: Descriptor AXI Monitor Status	79

Table 31: Table 1.2.28: Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).	80

Table 32: Table 1.2.29: Descriptor Fetch AXI Master	80

Table 33: Table 1.2.30: Control Read AXI Master (32-bit) [Phase 2]	81

Table 34: Table 1.2.31: Control Write AXI Master (32-bit) [Phase 2]	82

Table 35: Table 1.2.32: Monitor Bus	84

Table 36: Table 1.2.33: AXI Transfer Configuration (sink-only)	85

Table 37: Table 1.2.34: Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id	85

Table 38: Table 1.2.35: AXI4 Master - Data Write (Sink SRAM -> Memory)	85

Table 39: Table 1.2.36: Source half debug	87

Table 40: Table 1.2.37: Sink half debug	87

Table 41: Table 1.2.38: Active-channel sideband for per-channel bus instrumentation	88

Table 42: Table 1.3.1: Clock Specifications	90

Table 43: Table 1.3.2: Reset Specifications	91

Table 44: Table 1.3.3: Reset States	93

Table 45: Table 1.3.4: Configuration Timing Requirements	94

Table 46: Table 2.1.2: Clock and Reset	101

Table 47: Table 2.1.3: Configuration Interface	101

Table 48: Table 2.1.4: Descriptor Engine Interface	101

Table 49: Table 2.1.5: Data Read Interface	102

Table 50: Table 2.1.6: Data Write Interface	102

Table 51: Table 2.1.7: Control Interface (CTRL_READ / CTRL_WRITE)	103

Table 52: Table 2.1.7: FSM State Encoding	107

Table 53: Table 2.1.8: Error Handling	110

Table 54: Table 2.1.9: MonBus Event Codes	111

Table 55: Table 2.2.2: Clock and Reset	115

Table 56: Table 2.2.3: APB Programming Interface	115

Table 57: Table 2.2.4: Configuration Interface	115

Table 58: Table 2.2.5: AXI4 Read Master Interface	116

Table 59: Table 2.2.6: Scheduler Interface	117

Table 60: Table 2.3.2: Clock and Reset	127

Table 61: Table 2.3.3: Scheduler Interface	127

Table 62: Table 2.3.4: AXI4 Read Master Interface	127

Table 63: Table 2.3.5: SRAM Write Interface	128

Table 64: Table 2.3.6: Error Handling	132

Table 65: Table 2.4.2: Clock and Reset	137

Table 66: Table 2.4.3: Scheduler Interface	137

Table 67: Table 2.4.4: AXI4 Write Master Interface	138

Table 68: Table 2.4.5: SRAM Read Interface	139

Table 69: Table 2.4.6: Error Handling	143

Table 70: Table 2.5.2: Clock and Reset	149

Table 71: Table 2.5.3: Write Interface	149

Table 72: Table 2.5.4: Read Interface	149

Table 73: Table 2.5.5: Status Outputs	149

Table 74: Table 2.6.2: Clock and Reset	158

Table 75: Table 2.6.3: Write Interface	158

Table 76: Table 2.6.4: Read Interface	158

Table 77: Table 2.6.5: Status Outputs	158

Table 78: Table 2.6.6: alloc_ctrl vs drain_ctrl Comparison	162

Table 79: Table 2.7.2: Clock and Reset	168

Table 80: Table 2.7.3: Upstream Interface	168

Table 81: Table 2.7.4: Downstream Interface	168

Table 82: Table 2.7.5: Status and Debug	169

Table 83: Table 2.7.6: Skid Depth Selection	172

Table 84: Table 2.8.2: Clock and Reset	177

Table 85: Table 2.8.3: Scheduler Interface	177

Table 86: Table 2.8.4: Configuration Interface	178

Table 87: Table 2.8.5: AXI4 Read Master Interface	178

Table 88: Table 2.8.6: MonBus Interface	179

Table 89: Table 2.8.7: MonBus Events	184

Table 90: Table 2.8.8: CTRL_READ Descriptor Field Mapping	185

Table 91: Table 2.9.2: Clock and Reset	190

Table 92: Table 2.9.3: Configuration Interface	190

Table 93: Table 2.9.4: Scheduler Interface	190

Table 94: Table 2.9.5: AXI4 Write Master Interface	191

Table 95: Table 2.9.6: MonBus Interface	192

Table 96: Table 3.1.2: Clock and Reset	202

Table 97: Table 3.1.3: APB Programming Interface	202

Table 98: Table 3.1.4: Configuration Interface	202

Table 99: Table 3.1.5: Descriptor AXI Master Interface	204

Table 100: Table 3.1.6: Scheduler Data Interfaces	205

Table 101: Table 3.1.7: Status Interface	205

Table 102: Table 3.1.8: MonBus Interface	206

Table 103: Table 3.2.2: Clock and Reset	211

Table 104: Table 3.2.3: APB Programming Interface	211

Table 105: Table 3.2.4: Per-Channel Configuration	211

Table 106: Table 3.2.5: Shared Descriptor AXI Master Interface	212

Table 107: Table 3.2.6: Per-Channel Scheduler Data Interfaces	213

Table 108: Table 3.2.7: Aggregate Status	213

Table 109: Table 3.2.8: Unified MonBus Interface	214

Table 110: Table 3.2.9: Descriptor-AXI Perf Window	214

Table 111: Table 3.2.10: MonBus Source Assignment	217

Table 112: Table 3.3.2: Fill Interface	226

Table 113: Table 3.3.3: Scheduler Interface	226

Table 114: Table 3.4.2: Clock and Reset	231

Table 115: Table 3.4.3: AXI-Stream Slave Interface	231

Table 116: Table 3.4.4: Allocation Configuration	231

Table 117: Table 3.4.5: Scheduler Interface	232

Table 118: Table 3.4.6: AXI Write Master Interface	232

Table 119: Table 3.5.2: Clock and Reset	241

Table 120: Table 3.5.3: Fill Interface (ID-Selected)	241

Table 121: Table 3.5.4: Arbitrated Drain Interface	241

Table 122: Table 3.5.5: Debug Interface	242

Table 123: Table 3.6.2: Clock and Reset	250

Table 124: Table 3.6.3: Configuration	250

Table 125: Table 3.6.4: Scheduler Interface	250

Table 126: Table 3.6.5: AXI Read Master Interface	250

Table 127: Table 3.6.6: Drain Interface	251

Table 128: Table 3.7.2: Clock and Reset	258

Table 129: Table 3.7.3: Configuration	258

Table 130: Table 3.7.4: Scheduler Interface	258

Table 131: Table 3.7.5: AXI Read Master Interface	259

Table 132: Table 3.7.6: AXI-Stream Master Interface	259

Table 133: Table 3.8.2: Clock and Reset	269

Table 134: Table 3.8.3: Fill Interface (ID-Selected)	269

Table 135: Table 3.8.4: Arbitrated Drain Interface	269

Table 136: Table 3.8.5: Debug Interface	270

Table 137: Table 3.8.6: Sink vs Source Controller Comparison	272

Table 138: Table 3.9.2: Clock and Reset	277

Table 139: Table 3.9.3: APB Programming Interface	277

Table 140: Table 3.9.4: Per-Channel Configuration	278

Table 141: Table 3.9.5: Descriptor AXI Master Interfaces (src and snk)	279

Table 142: Table 3.9.6: Sink AXI Write Master Interface	280

Table 143: Table 3.9.7: Source AXI Read Master Interface	281

Table 144: Table 3.9.8: Sink Ingress AXIS Slave	282

Table 145: Table 3.9.9: Source Egress AXIS Master	283

Table 146: Table 3.9.10: Unified MonBus Interface	283

Table 147: Table 3.9.11: Aggregate Status	284

Table 148: Table 3.9.12: MonBus Source Assignment	286

Table 149: Table 3.10.1: Base Register Map	292

Table 150: Table 3.10.2: Monitor Register Map (base 0x1000)	295

Table 151: Table 3.11.1: Monitor Register-to-Config Mapping	299

Table 152: Table 3.12.1: Top-Level Address Decode	303

Table 153: Table 3.12.3: Top-Level Interfaces	308

Table 154: Table 3.13.1: RAPIDS Network Planes	311

Table 155: Table 3.13.2: Generated Tops	312

Table 156: Table 3.13.3: CXS Plane Parameters	314

Table 157: Table 3.13.4: CXS Top Test Cells	317

Table 158: Table 4.0.1: AXI4 Master Interfaces	319

Table 159: Table 4.0.2: Streaming Interfaces	319

Table 160: Table 4.0.3: Control and Monitor Interfaces	320

Table 161: Table 4.0.4: Signal Naming Conventions	322

Table 162: Table 4.1.1: Descriptor AXI Master Interface Signals	325

Table 163: Table 4.1.2: Descriptor AXI Transaction Characteristics	326

Table 164: Table 4.1.3: Sink AXI Write Master Interface Signals	328

Table 165: Table 4.1.4: Sink AXI Write Transaction Characteristics	329

Table 166: Table 4.1.5: Source AXI Read Master Interface Signals	331

Table 167: Table 4.1.6: Source AXI Read Transaction Characteristics	331

Table 168: Table 4.1.7: AXI Feature Support	334

Table 169: Table 4.2.1: AXIS Sink Slave Interface Signals	337

Table 170: Table 4.2.2: TID Channel Mapping	337

Table 171: Table 4.2.3: TSTRB Configuration	338

Table 172: Table 4.2.4: AXIS Source Master Interface Signals	340

Table 173: Table 4.2.5: Interface Comparison	342

Table 174: Table 4.2.6: Interface Selection Guide	342

Table 175: Table 4.3.1: MonBus Interface Signals	346

Table 176: Table 4.3.2: MonBus Packet Fields	348

Table 177: Table 4.3.3: MonBus Packet Types	349

Table 178: Table 4.3.4: Descriptor Packet Payload	349

Table 179: Table 4.3.5: Scheduler Packet Payload	350

Table 180: Table 4.3.6: AXI Event Packet Payload	350

Table 181: Table 4.3.7: Error Packet Payload	350

Table 182: Table 4.3.8: MonBus Source Assignment	351

Table 183: Table 4.3.9: MonBus Timing Parameters	354

Table 184: Table 4.4.1: CXS Link Profile	357

Table 185: Table 4.4.2: CXS Interface Signals	358

Table 186: Table 4.4.3: Link Activation Sequence	361

Table 187: RAPIDS Beats Feature Summary	433

Table 188: External Interface Summary	433

List of Waveforms

No waveforms in this document.

# 1	Rapids Beats Mas Index

Generated: 2026-10-21

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

This document describes the RAPIDS “Beats” architecture, a Phase 1 implementation of the Rapid AXI Programmable In-band Descriptor System. The beats architecture provides network-to-memory and memory-to-network data transfer capabilities using descriptor-based DMA with 8-channel support. It emphasizes “beat-level” tracking for precise flow control and latency management.

## 2.1	References

### 2.1.1	Related Documents

Table 0.1: Related Documents and Specifications

| Source | Title | Version |
| --- | --- | --- |
| RTL Design Sherpa | RAPIDS Product Requirements Document | 1.0 |
| RTL Design Sherpa | RAPIDS Original Specification | 0.25 |
| RTL Design Sherpa | STREAM MAS (Reference Architecture) | 0.90 |
| ARM | AMBA AXI and ACE Protocol Specification | IHI0022H |
| ARM | AMBA AXI-Stream Protocol Specification | IHI0051A |
| ARM | AMBA CXS Protocol Specification | IHI0079D |

## 2.2	Terminology

AXIS

AXI-Stream. ARM’s streaming protocol for high-bandwidth, low-latency data transfer without addressing.

AXI4

Version 4 of the AXI protocol, supporting burst transfers up to 256 beats.

Beat

A single data transfer within an AXI burst. For RAPIDS with 512-bit data width, one beat = 64 bytes.

Beats Architecture

The Phase 1 RAPIDS implementation that tracks transfers at beat granularity for precise flow control.

Burst

A group of consecutive data transfers (beats) initiated by a single address phase.

Channel

One of 8 independent DMA channels in RAPIDS. Each channel has its own descriptor chain and SRAM buffer allocation.

Descriptor

A 256-bit data structure containing transfer parameters: address, length, channel ID, and control flags.

Descriptor Chain

A linked list of descriptors where each descriptor points to the next, enabling scatter-gather operations.

DMA

Direct Memory Access. Hardware-controlled data movement between memory locations without CPU intervention.

FUB

Functional Unit Block. A self-contained RTL module with defined interfaces and testbench.

Latency Bridge

A module that provides buffering to compensate for pipeline latency between alloc/drain controllers.

MAC

Macro. An integration-level block that instantiates and connects multiple FUBs.

MonBus

Monitor Bus. A 64-bit internal bus for performance monitoring and debug event reporting.

RAPIDS

Rapid AXI Programmable In-band Descriptor System. A DMA accelerator for network-to-memory transfers.

Sink Path

The data path from network (AXIS slave) to system memory (AXI write master).

Source Path

The data path from system memory (AXI read master) to network (AXIS master).

SRAM

Static Random Access Memory. Used for internal data buffering between network and memory interfaces.

Virtual FIFO

A tracking mechanism (alloc_ctrl/drain_ctrl) that manages SRAM space without moving actual data.

## 2.3	“Beats” Architecture Concept

The “beats” architecture name reflects the key design decision to track all transfers at beat granularity (data-width units) rather than byte or chunk granularity:

- Beat-level Allocation: Space is allocated in beats (e.g., 64-byte units for 512-bit datapath)

- Beat-level Drain: Data availability is tracked in beats

- Beat-level Latency: The latency bridge compensates for pipeline delays in beat counts

- Beat-level Completion: AXI engines report completion in beats

This simplifies the math and aligns naturally with AXI burst semantics where each beat represents one data-width transfer.

## 2.4	Revision History

Revisions follow the convention x.y, where x is the major version and y is the minor version.

Table 0.2: RAPIDS Beats MAS Document Revision History

| Rev | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.25 | 2025-01-10 | seang | Initial RAPIDS Beats MAS structure |
| 0.6 | 2026-07-02 | seang | Resynced to RTL: scheduler recoverable write-progress timeout (cfg_sched_timeout_limit, strike escalation, scheduler_idle excludes CH_ERROR) and B-response commit gating; functional descriptor prefetch (cfg_prefetch_enable/cfg_fifo_threshold, deferred chaining); AXI write engine COMMIT strobes; commit/timeout-limit plumbing through rapids_core_beats; added rapids_regs register block (monitors @ 0x1000), rapids_config_block, and rapids_beats_top integration with the MonBus AXI-Lite group |
| 0.7 | 2026-07-13 | seang | Documented the control-read / control-write engine subsystem that was in the RTL but undocumented: new fub-block specs for ctrlrd_engine (Section 2.8, consumer gate: poll-until-(rd & mask)==expected, retry budget) and ctrlwr_engine (Section 2.9, producer doorbell: single-beat write); scheduler Control Interface + three-opcode (DATA/CTRL_READ/CTRL_WRITE) operation (Section 2.1); CTRL_CONFIG @ 0x240 (CTRLRD_MAX_TRY) added to the register map (Section 3.12); scheduler_group_beats control-engine instantiation. Naming cleanup: the per-instance register file is rapids_engine_regs (was rapids_half_regs). |
| 0.8 | 2026-09-21 | seang | Documentation hygiene: removed 30 **Source:** links pointing at assets/mermaid/*.mmd files that were never created (authored dead in a1760aaf6, 2026-07-17). None had an image above it, so no figure was lost; the 3 source links that resolve are unchanged. |
| 0.9 | 2026-10-16 | seang | Network planes (rapids TASK-024/025): new Section 3.13 Network Planes (the four rapids_plane_* modules, generated tops, CXS plane internals and parameters, per-stream framing, monitoring, test cells) and Section 4.4 CXS Interface Specification (link profile, signals, credit flow, activation, backpressure); rapids_beats_top page covers the base / generated split, the plane monitors on client 1 and the AXIS / CXS network ports, and its parameter and interface tables are resynced (the fill / drain rows were never top ports); core hierarchy and port list drop the monitor-lite and its three parameters, which moved to the planes. All block diagrams updated for the planes. The CXS ID mapping is marked provisional: routing and payload-type fields are planned. |
| 0.10 | 2026-10-16 | seang | Sections 3.13 and 4.4 point at the routing and payload-type proposal in the tile’s TILE_SPEC Section 11 (CXSSRCID keeps the channel) |
| 0.11 | 2026-10-17 | seang | CXS interface spec names the sink and source ports |
| 0.12 | 2026-10-20 | seang | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |
| 0.13 | 2026-10-20 | seang | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |
| 0.14 | 2026-10-21 | seang | Scheduler: run-base generators flushed by the channel reset (rapids BUG-017). Registers: per-channel CH_SCRATCH at 0x300. |

Last Updated: 2026-09-21

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Architecture Overview

Module: rapids_core_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 3.1	Overview

The RAPIDS “Beats” architecture is a Phase 1 implementation providing network-to-memory and memory-to-network data transfer capabilities. The name “beats” reflects the design decision to track all transfers at beat granularity (data-width units) for simplified flow control.

### 3.1.1	Key Features

- 8 Independent Channels: Each channel has its own descriptor chain and SRAM allocation

- Separate Data Paths: Sink (network-to-memory) and source (memory-to-network) paths

- Beat-Level Tracking: All flow control operates at beat granularity

- SRAM Buffering: Separate sink and source SRAM buffers for data staging

- MonBus Integration: Comprehensive monitoring for all subsystems

- Streaming Pipelines: No FSM in data engines - pure streaming for maximum throughput

### 3.1.2	Block Diagram

### 3.1.3	Figure 1.1.1: RAPIDS Beats Architecture Block Diagram

rapids_core_beats block diagram

Source: 19_rapids_core.dot

## 3.2	Data Flow Overview

### 3.2.1	Sink Path (Network to Memory)

The sink path receives data from an external source (via Fill interface) and writes it to system memory:

### 3.2.2	Figure 1.1.2: Sink Path Data Flow

1. External Fill Valid
        |
        v
2. sram_controller (STREAM): stream_alloc_ctrl reserves space
        |
        v
3. sram_controller: per-channel gaxi_fifo_sync takes the beats
        |
        v
4. sram_controller: stream_drain_ctrl reports data available
        |
        v
5. axi_write_engine_beats (AXI Burst Write)
        |
        v
6. System Memory

### 3.2.3	Source Path (Memory to Network)

The source path reads data from system memory and sends it to an external destination (via Drain interface):

### 3.2.4	Figure 1.1.3: Source Path Data Flow

1. Scheduler Request
        |
        v
2. sram_controller (STREAM): stream_alloc_ctrl reserves space
        |
        v
3. axi_read_engine_beats (AXI Burst Read)
        |
        v
4. sram_controller: per-channel gaxi_fifo_sync takes the beats
        |
        v
5. sram_controller: stream_drain_ctrl reports data available
        |
        v
6. External Drain Ready

## 3.3	Key Architectural Decisions

### 3.3.1	Beat-Level Tracking

All flow control uses beat granularity:

Table 1.1.1: Beat Granularity Examples

| Operation | Granularity | Example (512-bit DW) |
| --- | --- | --- |
| Space allocation | Beats | Request 8 beats = 512 bytes |
| Data availability | Beats | 16 beats ready = 1024 bytes |
| AXI burst length | Beats | ARLEN=15 = 16 beats |
| Latency compensation | Beats | 2-beat pipeline delay |

### 3.3.2	Concurrent Read/Write

To prevent deadlock with large transfers:

Example: 100MB transfer with 2KB SRAM buffer

Sequential operation (WRONG):
1. Read 100MB -> DEADLOCK at 2KB (SRAM full, can't complete read)

Concurrent operation (CORRECT):
1. Read starts filling SRAM -> SRAM becomes full (2KB)
2. Read pauses (natural backpressure)
3. Write drains SRAM -> SRAM has free space
4. Read resumes -> Both continue until 100MB complete

### 3.3.3	Virtual FIFOs (Alloc/Drain)

The alloc_ctrl and drain_ctrl modules are “virtual FIFOs” that track space/data without storing actual data:

- stream_alloc_ctrl: Tracks allocated space (write pointer advances on allocation, read pointer on actual write)

- stream_drain_ctrl: Tracks available data (write pointer advances on data arrival, read pointer on drain reservation). Its virtual depth is 2 x SRAM_DEPTH so the latency bridge’s parked beats never make it refuse a real write (stream BUG-011)

- Both live inside STREAM’s sram_controller_unit, one per channel, reached through the snk_/src_sram_controller_beats naming wrappers since bdf4e0dff. RAPIDS’ own alloc_ctrl_beats / drain_ctrl_beats / latency_bridge_beats are kept and tested but no longer in this path.

## 3.4	Module Hierarchy

The core is wrapped by a generated top: rapids_beats_top (AXIS network planes) or rapids_beats_cxs_top (CXS network planes), each rapids_beats_top_base plus an RX plane in front of the sink half and a TX plane behind the source half. See Network Planes.

rapids_beats_top | rapids_beats_cxs_top  (generated, bin/gen_plane_tops.py)
├── u_base: rapids_beats_top_base        (regs, config blocks, monbus group)
│   └── u_core: rapids_core_beats        (below)
├── u_snk_plane: rapids_plane_axis_rx | rapids_plane_cxs_rx
└── u_src_plane: rapids_plane_axis_tx | rapids_plane_cxs_tx

rapids_core_beats
├── monbus_arbiter                       (merges the two halves' monbus)
├── rapids_src_beats                     (SOURCE half)
│   ├── scheduler_group_array_beats
│   │   ├── scheduler_group_beats [0..7]
│   │   │   ├── scheduler_beats
│   │   │   ├── descriptor_engine_beats
│   │   │   ├── ctrlrd_engine
│   │   │   └── ctrlwr_engine
│   │   ├── arbiter_round_robin [x3]     (desc / ctrlrd / ctrlwr AXI masters)
│   │   ├── axi4_master_rd_monlite       (descriptor-fetch monitor)
│   │   └── axi_bus_meter
│   ├── monbus_arbiter (3:1)             (client 1 = src_plane_mon_*)
│   └── src_data_path_axis_beats
│       └── src_data_path_beats
│           ├── axi_read_engine_beats
│           └── src_sram_controller_beats    (naming wrapper)
│               └── sram_controller          (STREAM, shared)
│                   └── sram_controller_unit [0..7]
│                       ├── stream_alloc_ctrl
│                       ├── gaxi_fifo_sync   (the per-channel SRAM)
│                       ├── stream_drain_ctrl
│                       └── stream_latency_bridge
└── rapids_snk_beats                     (SINK half, same shape)
    ├── scheduler_group_array_beats
    │   └── ...
    ├── monbus_arbiter (3:1)             (client 1 = snk_plane_mon_*)
    └── snk_data_path_axis_beats
        └── snk_data_path_beats
            ├── axi_write_engine_beats
            └── snk_sram_controller_beats
                └── sram_controller (STREAM, shared)
                    └── sram_controller_unit [0..7]

alloc_ctrl_beats, drain_ctrl_beats and latency_bridge_beats (fub_beats) are not in this tree: since bdf4e0dff the SRAM path is STREAM’s controller. They keep their tests and filelists as standalone FUBs.

## 3.5	Timing Diagram: Basic Transfer

### 3.5.1	Figure 1.1.4: Basic Sink Path Transfer Timing

rapids_core_beats - sink channel 0 kick to first AW

Source: rapids_core_beats_sink_kick.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (sink path, channel 0, 32 beats, 512-bit data, TEST_LEVEL=full) with WAVES=1.

Reading it: the AXIS packet is already sitting in the sink SRAM when software kicks channel 0 (snk_apb_valid[0] for one cycle with the descriptor address 0x30000000). snk_apb_ready[0] drops the same cycle and stays low while the channel is busy. The descriptor engine leaves idle immediately to fetch the descriptor; twelve cycles later it hands the descriptor to the scheduler, its own idle returns, and snk_scheduler_idle[0] and snk_system_idle fall together. The first AW follows four cycles after that. snk_system_idle returns after the last B response of the transfer (not in this window); the busy-then-idle sequence is what the top-level tests wait on.

## 3.6	Related Documentation

- Top-Level Port List - Complete port specification

- Clocks and Reset - Timing requirements

- Beats Scheduler Group - Scheduler integration

- Sink Data Path - Sink path details

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Top-Level Port List

Module: rapids_core_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2026-10-16

## 4.1	Parameters

Every parameter of rapids_core_beats, as declared. The short aliases at the end are what the port widths below are written in (NC, AW, DW, IW, SCW, CIW, SW).

Table 1.2.1: Parameters (from rapids_core_beats.sv)

| Parameter | Type | Default | Notes |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | - |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | - |
| ADDR_WIDTH | int | 64 | - |
| DATA_WIDTH | int | 512 | - |
| AXI_ID_WIDTH | int | 8 | - |
| SRAM_DEPTH | int | 512 | - |
| SEG_COUNT_WIDTH | int | $clog2(SRAM_DEPTH) + 1 | - |
| PIPELINE | int | 1 | write-engine pipelining; 1 is the perf design point (see rapids_beats_top) |
| AR_MAX_OUTSTANDING | int | 8 | - |
| AW_MAX_OUTSTANDING | int | 8 | - |
| W_PHASE_FIFO_DEPTH | int | 64 | - |
| B_PHASE_FIFO_DEPTH | int | 16 | - |
| AXIS_ID_WIDTH | int | 8 | - |
| AXIS_DEST_WIDTH | int | 4 | - |
| AXIS_USER_WIDTH | int | 1 | - |
| DESC_MON_BASE_AGENT_ID | int | 16 | 0x10 - Descriptor Engines (16-23) |
| SCHED_MON_BASE_AGENT_ID | int | 48 | 0x30 - Schedulers (48-55) |
| DESC_AXI_MON_AGENT_ID | int | 8 | 0x08 - Descriptor AXI Master Monitor |
| MON_UNIT_ID | int | 1 | 0x1 |
| MON_MAX_TRANSACTIONS | int | 16 | - |
| USE_AXI_MONITORS | int | 1 | - |
| USE_ROW_COL_MAJOR_ADDRESSING | int | 1 | - |
| GEN_MON | bit | 1'b1 | - |
| NC | int | NUM_CHANNELS | - |
| AW | int | ADDR_WIDTH | - |
| DW | int | DATA_WIDTH | - |
| IW | int | AXI_ID_WIDTH | - |
| SD | int | SRAM_DEPTH | - |
| SCW | int | SEG_COUNT_WIDTH | - |
| CIW | int | (NC > 1) ? $clog2(NC) : 1 | - |
| SW | int | DW / 8 | - |

## 4.2	Port List (324 ports)

The module is two independent halves. Ports both halves carry are prefixed src_ / snk_; the direction-unique ports (the source’s AXI read master and AXIS egress, the sink’s AXI write master and AXIS ingress) carry no prefix. Sections follow the declaration order in the RTL, so a reader can walk this page and the module side by side. Widths use the parameter aliases.

### 4.2.1	Clock and Reset (shared - single clock/reset for the whole wrapper)

Table 1.2.2: Clock and Reset (shared - single clock/reset for the whole wrapper)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | - |
| rst_n | input | 1 | - |

## 4.3	SOURCE HALF (u_src) - shared-infrastructure ports (src_ prefixed)

### 4.3.1	APB Programming Interface

Table 1.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_apb_valid | input | NC | - |
| src_apb_ready | output | NC | - |
| src_apb_addr | input | NC x AW | - |

### 4.3.2	Per-channel configuration

Table 1.2.4: Per-channel configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_channel_enable | input | NC | - |
| src_cfg_channel_reset | input | NC | - |

### 4.3.3	Scheduler Configuration (global)

Table 1.2.5: Scheduler Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_sched_enable | input | 1 | - |
| src_cfg_sched_timeout_cycles | input | 32 | - |
| src_cfg_sched_timeout_limit | input | 8 | - |
| src_cfg_sched_timeout_enable | input | 1 | - |
| src_cfg_sched_err_enable | input | 1 | - |
| src_cfg_sched_compl_enable | input | 1 | - |
| src_cfg_sched_perf_enable | input | 1 | - |

### 4.3.4	Descriptor Engine Configuration (global)

Table 1.2.6: Descriptor Engine Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_desceng_enable | input | 1 | - |
| src_cfg_desceng_prefetch | input | 1 | - |
| src_cfg_desceng_fifo_thresh | input | 4 | - |
| src_cfg_desceng_addr0_base | input | AW | - |
| src_cfg_desceng_addr0_limit | input | AW | - |
| src_cfg_desceng_addr1_base | input | AW | - |
| src_cfg_desceng_addr1_limit | input | AW | - |

### 4.3.5	Control Engine Configuration (Phase 2, global)

Table 1.2.7: Control Engine Configuration (Phase 2, global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_ctrlrd_max_try | input | 9 | - |
| src_tick_1us | input | 1 | - |

### 4.3.6	Descriptor AXI Monitor Configuration

Table 1.2.8: Descriptor AXI Monitor Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_desc_mon_enable | input | 1 | - |
| src_cfg_desc_mon_err_enable | input | 1 | - |
| src_cfg_desc_mon_perf_enable | input | 1 | - |
| src_cfg_desc_mon_perf_run | input | 1 | - |
| src_cfg_desc_mon_timeout_enable | input | 1 | - |
| src_cfg_desc_mon_timeout_cycles | input | 32 | - |
| src_cfg_desc_mon_latency_thresh | input | 32 | - |
| src_cfg_desc_mon_pkt_mask | input | 16 | - |
| src_cfg_desc_mon_err_select | input | 4 | - |
| src_cfg_desc_mon_err_mask | input | 8 | - |
| src_cfg_desc_mon_timeout_mask | input | 8 | - |
| src_cfg_desc_mon_compl_mask | input | 8 | - |
| src_cfg_desc_mon_thresh_mask | input | 8 | - |
| src_cfg_desc_mon_perf_mask | input | 8 | - |
| src_cfg_desc_mon_addr_mask | input | 8 | - |
| src_cfg_desc_mon_debug_mask | input | 8 | - |

### 4.3.7	Network-plane monitor input (rapids TASK-024)

The network endpoint and its monitor-lite live in the plane (rtl/plane/) in the top. Its monbus packets enter here as client 1 of this half’s monbus arbiter; the plane’s monitor configuration and status come from this half’s register block in the top.

Table 1.2.9: Network-plane monitor input (rapids TASK-024)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_plane_mon_valid | input | 1 | - |
| src_plane_mon_ready | output | 1 | - |
| src_plane_mon_packet | input | 128 | monitor_packet_t |
| src_plane_mon_timestamp | input | 64 | monbus_timestamp_t |

### 4.3.8	Status

Table 1.2.10: Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_system_idle | output | 1 | - |
| src_descriptor_engine_idle | output | NC | - |
| src_scheduler_idle | output | NC | - |
| src_scheduler_state | output | NC x 7 | - |
| src_sched_error | output | NC | - |

### 4.3.9	Descriptor AXI Monitor Status

Table 1.2.11: Descriptor AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_sts_desc_mon_busy | output | 1 | - |
| src_cfg_sts_desc_mon_active_txns | output | 8 | - |
| src_cfg_sts_desc_mon_error_count | output | 16 | - |
| src_cfg_sts_desc_mon_txn_count | output | 32 | - |
| src_cfg_sts_desc_mon_conflict_error | output | 1 | - |

### 4.3.10	Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).

Table 1.2.12: Descriptor AXI Monitor perf window (feeds SRC_.MON.DAXMON_PERF_*).

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_sts_desc_mon_win_active | output | 1 | - |
| src_sts_desc_mon_win_cycles | output | 32 | - |
| src_sts_desc_mon_prod_cycles | output | 32 | - |
| src_sts_desc_mon_bp_cycles | output | 32 | - |
| src_sts_desc_mon_starv_cycles | output | 32 | - |
| src_sts_desc_mon_idle_cycles | output | 32 | - |
| src_sts_desc_mon_beat_count | output | 32 | - |
| src_sts_desc_mon_byte_count | output | 64 | - |
| src_sts_desc_mon_burst_count | output | 32 | - |

### 4.3.11	Descriptor Fetch AXI Master

Table 1.2.13: Descriptor Fetch AXI Master

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_desc_arvalid | output | 1 | - |
| src_m_axi_desc_arready | input | 1 | - |
| src_m_axi_desc_araddr | output | AW | - |
| src_m_axi_desc_arlen | output | 8 | - |
| src_m_axi_desc_arsize | output | 3 | - |
| src_m_axi_desc_arburst | output | 2 | - |
| src_m_axi_desc_arid | output | IW | - |
| src_m_axi_desc_arlock | output | 1 | - |
| src_m_axi_desc_arcache | output | 4 | - |
| src_m_axi_desc_arprot | output | 3 | - |
| src_m_axi_desc_arqos | output | 4 | - |
| src_m_axi_desc_arregion | output | 4 | - |
| src_m_axi_desc_rvalid | input | 1 | - |
| src_m_axi_desc_rready | output | 1 | - |
| src_m_axi_desc_rdata | input | 256 | - |
| src_m_axi_desc_rresp | input | 2 | - |
| src_m_axi_desc_rlast | input | 1 | - |
| src_m_axi_desc_rid | input | IW | - |

### 4.3.12	Control Read AXI Master (32-bit) [Phase 2]

Table 1.2.14: Control Read AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_ctrlrd_arvalid | output | 1 | - |
| src_m_axi_ctrlrd_arready | input | 1 | - |
| src_m_axi_ctrlrd_araddr | output | AW | - |
| src_m_axi_ctrlrd_arlen | output | 8 | - |
| src_m_axi_ctrlrd_arsize | output | 3 | - |
| src_m_axi_ctrlrd_arburst | output | 2 | - |
| src_m_axi_ctrlrd_arid | output | IW | - |
| src_m_axi_ctrlrd_arlock | output | 1 | - |
| src_m_axi_ctrlrd_arcache | output | 4 | - |
| src_m_axi_ctrlrd_arprot | output | 3 | - |
| src_m_axi_ctrlrd_arqos | output | 4 | - |
| src_m_axi_ctrlrd_arregion | output | 4 | - |
| src_m_axi_ctrlrd_rvalid | input | 1 | - |
| src_m_axi_ctrlrd_rready | output | 1 | - |
| src_m_axi_ctrlrd_rdata | input | 32 | - |
| src_m_axi_ctrlrd_rresp | input | 2 | - |
| src_m_axi_ctrlrd_rlast | input | 1 | - |
| src_m_axi_ctrlrd_rid | input | IW | - |

### 4.3.13	Control Write AXI Master (32-bit) [Phase 2]

Table 1.2.15: Control Write AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_ctrlwr_awvalid | output | 1 | - |
| src_m_axi_ctrlwr_awready | input | 1 | - |
| src_m_axi_ctrlwr_awaddr | output | AW | - |
| src_m_axi_ctrlwr_awlen | output | 8 | - |
| src_m_axi_ctrlwr_awsize | output | 3 | - |
| src_m_axi_ctrlwr_awburst | output | 2 | - |
| src_m_axi_ctrlwr_awid | output | IW | - |
| src_m_axi_ctrlwr_awlock | output | 1 | - |
| src_m_axi_ctrlwr_awcache | output | 4 | - |
| src_m_axi_ctrlwr_awprot | output | 3 | - |
| src_m_axi_ctrlwr_awqos | output | 4 | - |
| src_m_axi_ctrlwr_awregion | output | 4 | - |
| src_m_axi_ctrlwr_wvalid | output | 1 | - |
| src_m_axi_ctrlwr_wready | input | 1 | - |
| src_m_axi_ctrlwr_wdata | output | 32 | - |
| src_m_axi_ctrlwr_wstrb | output | 4 | - |
| src_m_axi_ctrlwr_wlast | output | 1 | - |
| src_m_axi_ctrlwr_bvalid | input | 1 | - |
| src_m_axi_ctrlwr_bready | output | 1 | - |
| src_m_axi_ctrlwr_bid | input | IW | - |
| src_m_axi_ctrlwr_bresp | input | 2 | - |

## 4.4	SOURCE HALF (u_src) - direction-unique ports (no prefix)

### 4.4.1	AXI Transfer Configuration (source-only)

Table 1.2.16: AXI Transfer Configuration (source-only)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | - |
| cfg_drain_size | input | 8 | source: beats drained per AXIS packet |

### 4.4.2	Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id

Table 1.2.17: Source Path - AXIS Master Interface (SRAM -> Network); tid = channel id

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tdata | output | DW | - |
| m_axis_tstrb | output | SW | - |
| m_axis_tlast | output | 1 | - |
| m_axis_tid | output | AXIS_ID_WIDTH | - |
| m_axis_tdest | output | AXIS_DEST_WIDTH | - |
| m_axis_tuser | output | AXIS_USER_WIDTH | - |
| m_axis_tvalid | output | 1 | - |
| m_axis_tready | input | 1 | - |

### 4.4.3	AXI4 Master - Data Read (Memory -> Source SRAM)

Table 1.2.18: AXI4 Master - Data Read (Memory -> Source SRAM)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_arid | output | IW | - |
| m_axi_rd_araddr | output | AW | - |
| m_axi_rd_arlen | output | 8 | - |
| m_axi_rd_arsize | output | 3 | - |
| m_axi_rd_arburst | output | 2 | - |
| m_axi_rd_arvalid | output | 1 | - |
| m_axi_rd_arready | input | 1 | - |
| m_axi_rd_rid | input | IW | - |
| m_axi_rd_rdata | input | DW | - |
| m_axi_rd_rresp | input | 2 | - |
| m_axi_rd_rlast | input | 1 | - |
| m_axi_rd_rvalid | input | 1 | - |
| m_axi_rd_rready | output | 1 | - |

## 4.5	SINK HALF (u_snk) - shared-infrastructure ports (snk_ prefixed)

### 4.5.1	APB Programming Interface

Table 1.2.19: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_apb_valid | input | NC | - |
| snk_apb_ready | output | NC | - |
| snk_apb_addr | input | NC x AW | - |

### 4.5.2	Per-channel configuration

Table 1.2.20: Per-channel configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_channel_enable | input | NC | - |
| snk_cfg_channel_reset | input | NC | - |

### 4.5.3	Scheduler Configuration (global)

Table 1.2.21: Scheduler Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_sched_enable | input | 1 | - |
| snk_cfg_sched_timeout_cycles | input | 32 | - |
| snk_cfg_sched_timeout_limit | input | 8 | - |
| snk_cfg_sched_timeout_enable | input | 1 | - |
| snk_cfg_sched_err_enable | input | 1 | - |
| snk_cfg_sched_compl_enable | input | 1 | - |
| snk_cfg_sched_perf_enable | input | 1 | - |

### 4.5.4	Descriptor Engine Configuration (global)

Table 1.2.22: Descriptor Engine Configuration (global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_desceng_enable | input | 1 | - |
| snk_cfg_desceng_prefetch | input | 1 | - |
| snk_cfg_desceng_fifo_thresh | input | 4 | - |
| snk_cfg_desceng_addr0_base | input | AW | - |
| snk_cfg_desceng_addr0_limit | input | AW | - |
| snk_cfg_desceng_addr1_base | input | AW | - |
| snk_cfg_desceng_addr1_limit | input | AW | - |

### 4.5.5	Control Engine Configuration (Phase 2, global)

Table 1.2.23: Control Engine Configuration (Phase 2, global)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_ctrlrd_max_try | input | 9 | - |
| snk_tick_1us | input | 1 | - |

### 4.5.6	Descriptor AXI Monitor Configuration

Table 1.2.24: Descriptor AXI Monitor Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_desc_mon_enable | input | 1 | - |
| snk_cfg_desc_mon_err_enable | input | 1 | - |
| snk_cfg_desc_mon_perf_enable | input | 1 | - |
| snk_cfg_desc_mon_perf_run | input | 1 | - |
| snk_cfg_desc_mon_timeout_enable | input | 1 | - |
| snk_cfg_desc_mon_timeout_cycles | input | 32 | - |
| snk_cfg_desc_mon_latency_thresh | input | 32 | - |
| snk_cfg_desc_mon_pkt_mask | input | 16 | - |
| snk_cfg_desc_mon_err_select | input | 4 | - |
| snk_cfg_desc_mon_err_mask | input | 8 | - |
| snk_cfg_desc_mon_timeout_mask | input | 8 | - |
| snk_cfg_desc_mon_compl_mask | input | 8 | - |
| snk_cfg_desc_mon_thresh_mask | input | 8 | - |
| snk_cfg_desc_mon_perf_mask | input | 8 | - |
| snk_cfg_desc_mon_addr_mask | input | 8 | - |
| snk_cfg_desc_mon_debug_mask | input | 8 | - |

### 4.5.7	Network-plane monitor input (rapids TASK-024)

The network endpoint and its monitor-lite live in the plane (rtl/plane/) in the top. Its monbus packets enter here as client 1 of this half’s monbus arbiter; the plane’s monitor configuration and status come from this half’s register block in the top.

Table 1.2.25: Network-plane monitor input (rapids TASK-024)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_plane_mon_valid | input | 1 | - |
| snk_plane_mon_ready | output | 1 | - |
| snk_plane_mon_packet | input | 128 | monitor_packet_t |
| snk_plane_mon_timestamp | input | 64 | monbus_timestamp_t |

### 4.5.8	Status

Table 1.2.26: Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_system_idle | output | 1 | - |
| snk_descriptor_engine_idle | output | NC | - |
| snk_scheduler_idle | output | NC | - |
| snk_scheduler_state | output | NC x 7 | - |
| snk_sched_error | output | NC | - |

### 4.5.9	Descriptor AXI Monitor Status

Table 1.2.27: Descriptor AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_cfg_sts_desc_mon_busy | output | 1 | - |
| snk_cfg_sts_desc_mon_active_txns | output | 8 | - |
| snk_cfg_sts_desc_mon_error_count | output | 16 | - |
| snk_cfg_sts_desc_mon_txn_count | output | 32 | - |
| snk_cfg_sts_desc_mon_conflict_error | output | 1 | - |

### 4.5.10	Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).

Table 1.2.28: Descriptor AXI Monitor perf window (feeds SNK_.MON.DAXMON_PERF_*).

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_sts_desc_mon_win_active | output | 1 | - |
| snk_sts_desc_mon_win_cycles | output | 32 | - |
| snk_sts_desc_mon_prod_cycles | output | 32 | - |
| snk_sts_desc_mon_bp_cycles | output | 32 | - |
| snk_sts_desc_mon_starv_cycles | output | 32 | - |
| snk_sts_desc_mon_idle_cycles | output | 32 | - |
| snk_sts_desc_mon_beat_count | output | 32 | - |
| snk_sts_desc_mon_byte_count | output | 64 | - |
| snk_sts_desc_mon_burst_count | output | 32 | - |

### 4.5.11	Descriptor Fetch AXI Master

Table 1.2.29: Descriptor Fetch AXI Master

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_desc_arvalid | output | 1 | - |
| snk_m_axi_desc_arready | input | 1 | - |
| snk_m_axi_desc_araddr | output | AW | - |
| snk_m_axi_desc_arlen | output | 8 | - |
| snk_m_axi_desc_arsize | output | 3 | - |
| snk_m_axi_desc_arburst | output | 2 | - |
| snk_m_axi_desc_arid | output | IW | - |
| snk_m_axi_desc_arlock | output | 1 | - |
| snk_m_axi_desc_arcache | output | 4 | - |
| snk_m_axi_desc_arprot | output | 3 | - |
| snk_m_axi_desc_arqos | output | 4 | - |
| snk_m_axi_desc_arregion | output | 4 | - |
| snk_m_axi_desc_rvalid | input | 1 | - |
| snk_m_axi_desc_rready | output | 1 | - |
| snk_m_axi_desc_rdata | input | 256 | - |
| snk_m_axi_desc_rresp | input | 2 | - |
| snk_m_axi_desc_rlast | input | 1 | - |
| snk_m_axi_desc_rid | input | IW | - |

### 4.5.12	Control Read AXI Master (32-bit) [Phase 2]

Table 1.2.30: Control Read AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_ctrlrd_arvalid | output | 1 | - |
| snk_m_axi_ctrlrd_arready | input | 1 | - |
| snk_m_axi_ctrlrd_araddr | output | AW | - |
| snk_m_axi_ctrlrd_arlen | output | 8 | - |
| snk_m_axi_ctrlrd_arsize | output | 3 | - |
| snk_m_axi_ctrlrd_arburst | output | 2 | - |
| snk_m_axi_ctrlrd_arid | output | IW | - |
| snk_m_axi_ctrlrd_arlock | output | 1 | - |
| snk_m_axi_ctrlrd_arcache | output | 4 | - |
| snk_m_axi_ctrlrd_arprot | output | 3 | - |
| snk_m_axi_ctrlrd_arqos | output | 4 | - |
| snk_m_axi_ctrlrd_arregion | output | 4 | - |
| snk_m_axi_ctrlrd_rvalid | input | 1 | - |
| snk_m_axi_ctrlrd_rready | output | 1 | - |
| snk_m_axi_ctrlrd_rdata | input | 32 | - |
| snk_m_axi_ctrlrd_rresp | input | 2 | - |
| snk_m_axi_ctrlrd_rlast | input | 1 | - |
| snk_m_axi_ctrlrd_rid | input | IW | - |

### 4.5.13	Control Write AXI Master (32-bit) [Phase 2]

Table 1.2.31: Control Write AXI Master (32-bit) [Phase 2]

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_m_axi_ctrlwr_awvalid | output | 1 | - |
| snk_m_axi_ctrlwr_awready | input | 1 | - |
| snk_m_axi_ctrlwr_awaddr | output | AW | - |
| snk_m_axi_ctrlwr_awlen | output | 8 | - |
| snk_m_axi_ctrlwr_awsize | output | 3 | - |
| snk_m_axi_ctrlwr_awburst | output | 2 | - |
| snk_m_axi_ctrlwr_awid | output | IW | - |
| snk_m_axi_ctrlwr_awlock | output | 1 | - |
| snk_m_axi_ctrlwr_awcache | output | 4 | - |
| snk_m_axi_ctrlwr_awprot | output | 3 | - |
| snk_m_axi_ctrlwr_awqos | output | 4 | - |
| snk_m_axi_ctrlwr_awregion | output | 4 | - |
| snk_m_axi_ctrlwr_wvalid | output | 1 | - |
| snk_m_axi_ctrlwr_wready | input | 1 | - |
| snk_m_axi_ctrlwr_wdata | output | 32 | - |
| snk_m_axi_ctrlwr_wstrb | output | 4 | - |
| snk_m_axi_ctrlwr_wlast | output | 1 | - |
| snk_m_axi_ctrlwr_bvalid | input | 1 | - |
| snk_m_axi_ctrlwr_bready | output | 1 | - |
| snk_m_axi_ctrlwr_bid | input | IW | - |
| snk_m_axi_ctrlwr_bresp | input | 2 | - |

## 4.6	Monitor Bus (SINGLE aggregated stream for the whole core)

The two halves’ monitor outputs are merged through a top-level monbus_arbiter, so the core exposes exactly one monitor stream.

### 4.6.1	Monitor Bus

Table 1.2.32: Monitor Bus

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | - |
| mon_ready | input | 1 | - |
| mon_packet | output | ocelus_common_pkg::monitor_packet_t | - |
| mon_timestamp | output | ocelus_common_pkg::monbus_timestamp_t | - |

## 4.7	SINK HALF (u_snk) - direction-unique ports (no prefix)

### 4.7.1	AXI Transfer Configuration (sink-only)

Table 1.2.33: AXI Transfer Configuration (sink-only)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_wr_xfer_beats | input | 8 | - |
| cfg_alloc_size | input | 8 | sink: SRAM alloc size per AXIS fill |

### 4.7.2	Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id

Table 1.2.34: Sink Path - AXIS Slave Interface (Network -> SRAM); tid = channel id

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tdata | input | DW | - |
| s_axis_tstrb | input | SW | - |
| s_axis_tlast | input | 1 | - |
| s_axis_tid | input | AXIS_ID_WIDTH | - |
| s_axis_tdest | input | AXIS_DEST_WIDTH | - |
| s_axis_tuser | input | AXIS_USER_WIDTH | - |
| s_axis_tvalid | input | 1 | - |
| s_axis_tready | output | 1 | - |

### 4.7.3	AXI4 Master - Data Write (Sink SRAM -> Memory)

Table 1.2.35: AXI4 Master - Data Write (Sink SRAM -> Memory)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_awid | output | IW | - |
| m_axi_wr_awaddr | output | AW | - |
| m_axi_wr_awlen | output | 8 | - |
| m_axi_wr_awsize | output | 3 | - |
| m_axi_wr_awburst | output | 2 | - |
| m_axi_wr_awlock | output | 1 | - |
| m_axi_wr_awcache | output | 4 | - |
| m_axi_wr_awprot | output | 3 | - |
| m_axi_wr_awqos | output | 4 | - |
| m_axi_wr_awregion | output | 4 | - |
| m_axi_wr_awvalid | output | 1 | - |
| m_axi_wr_awready | input | 1 | - |
| m_axi_wr_wdata | output | DW | - |
| m_axi_wr_wstrb | output | (DW/8) | - |
| m_axi_wr_wlast | output | 1 | - |
| m_axi_wr_wvalid | output | 1 | - |
| m_axi_wr_wready | input | 1 | - |
| m_axi_wr_bid | input | IW | - |
| m_axi_wr_bresp | input | 2 | - |
| m_axi_wr_bvalid | input | 1 | - |
| m_axi_wr_bready | output | 1 | - |

## 4.8	Debug Interface (per half, src_dbg_ / snk_dbg_ prefixed)

### 4.8.1	Source half debug

Table 1.2.36: Source half debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_dbg_rd_all_complete | output | NC | - |
| src_dbg_r_beats_rcvd | output | 32 | - |
| src_dbg_sram_writes | output | 32 | - |
| src_dbg_arb_request | output | NC | - |
| src_dbg_src_sram_bridge_pending | output | NC | - |
| src_dbg_src_sram_bridge_out_valid | output | NC | - |
| src_dbg_axis_beats_sent | output | 32 | - |
| src_dbg_axis_packets_sent | output | 32 | - |

### 4.8.2	Sink half debug

Table 1.2.37: Sink half debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_dbg_snk_sram_bridge_pending | output | NC | - |
| snk_dbg_snk_sram_bridge_out_valid | output | NC | - |
| snk_dbg_axis_beats_received | output | 32 | - |
| snk_dbg_axis_packets_received | output | 32 | - |

### 4.8.3	Active-channel sideband for per-channel bus instrumentation

(axi_bus_meter). The W bus carries no wid, so the write engine’s channel index must travel out of band to reach the meter at the top.

Table 1.2.38: Active-channel sideband for per-channel bus instrumentation

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| snk_active_channel_id | output | CIW | - |
| snk_active_channel_valid | output | 1 | - |

Verified: every row above is a declared port of rapids_core_beats and every declared port has a row (324 of 324), generated from the module declaration on 2026-09-28. Regenerate rather than hand-edit when the interface changes.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Clocks and Reset

Module: rapids_core_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 5.1	Clock Domain

RAPIDS Beats operates in a single clock domain for simplified design and maximum throughput.

### 5.1.1	Clock Specifications

Table 1.3.1: Clock Specifications

| Parameter | Specification |
| --- | --- |
| Clock Signal | clk (also axi_aclk in some submodules) |
| Frequency Range | 100 - 500 MHz |
| Duty Cycle | 45% - 55% |
| Clock Jitter | < 100 ps peak-to-peak |

### 5.1.2	Clock Distribution

clk (one clock domain; rst_n active-low, async assert)
├── scheduler_group_array_beats (one per half)
│   └── descriptor_engine_beats, scheduler_beats, ctrlrd_engine, ctrlwr_engine (x8)
├── snk_data_path_axis_beats
│   └── snk_sram_controller_beats, axi_write_engine_beats
└── src_data_path_axis_beats
    └── axi_read_engine_beats, src_sram_controller_beats

## 5.2	Reset Specification

RAPIDS uses asynchronous assert, synchronous deassert reset methodology.

### 5.2.1	Reset Signal

Table 1.3.2: Reset Specifications

| Parameter | Specification |
| --- | --- |
| Reset Signal | rst_n (also axi_aresetn in some submodules) |
| Polarity | Active-low |
| Assert | Asynchronous (immediate) |
| Deassert | Synchronous (on rising clock edge) |
| Minimum Duration | 10 clock cycles |

### 5.2.2	Reset Timing Diagram

### 5.2.3	Figure 1.3.1: Reset Timing

scheduler_beats - reset release to first descriptor

Source: scheduler_reset.json, captured from dv/tests/fub_beats/test_scheduler_beats.py (basic transfer, the same run as Figure 2.1.3) with WAVES=1.

Reading it: rst_n is held low for ten cycles and released on a clock edge. Every state register is already at its reset value while reset is asserted – the FSM sits in IDLE, scheduler_idle is high, both control engines report idle and neither engine request is raised. Nothing changes at the release edge itself; the scheduler stays in IDLE until the testbench raises cfg_channel_enable ten cycles later and a descriptor becomes available, at which point it steps through FETCH_DESC to XFER_DATA and raises the read and write requests together.

## 5.3	Reset Behavior

### 5.3.1	On Reset Assert

All modules initialize to known states:

Table 1.3.3: Reset States

| Component | Reset State |
| --- | --- |
| Scheduler FSM | IDLE |
| Descriptor Engine | IDLE, FIFOs empty |
| AXI Read Engine | Idle, no outstanding transactions |
| AXI Write Engine | Idle, no outstanding transactions |
| SRAM Pointers | All zeros |
| Alloc/Drain Controllers | Empty, full space available |
| MonBus | No packets pending |

### 5.3.2	Reset Sequence

1. External reset asserted (rst_n = 0)
   - All flip-flops async reset
   - All outputs driven to safe states
   - No AXI transactions

2. System stabilization (minimum 10 cycles)
   - Clock running and stable
   - Configuration registers programmed (optional)

3. Reset deasserted (rst_n = 1, sync to clk rising edge)
   - FSMs begin operation
   - Ready to accept descriptors/data

## 5.4	Configuration During Reset

Some configuration signals MUST be stable before reset deassertion:

Table 1.3.4: Configuration Timing Requirements

| Signal | Requirement |
| --- | --- |
| src_cfg_channel_enable / snk_cfg_channel_enable | Stable before rst_n=1 (one per half) |
| src_cfg_sched_timeout_cycles / snk_cfg_sched_timeout_cycles | Stable before rst_n=1 (one per half) |
| cfg_axi_rd_xfer_beats | Stable before rst_n=1 |
| cfg_axi_wr_xfer_beats | Stable before rst_n=1 |

## 5.5	Reset Macros

RAPIDS uses standardized reset macros from reset_defs.svh:

`include "reset_defs.svh"

// Standard reset pattern
`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
        r_counter <= '0;
    end else begin
        r_state <= w_next_state;
        r_counter <= w_next_counter;
    end
)

## 5.6	Power-On Reset Considerations

- External reset should be held for minimum 10 clock cycles after clock is stable

- Configuration registers should be programmed before or during reset

- No AXI transactions until reset is deasserted and channels are enabled

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Scheduler Specification

Module: scheduler_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 6.1	Overview

The Scheduler coordinates descriptor-based data transfers for a single RAPIDS channel. It receives descriptors from the descriptor engine, parses transfer parameters, and coordinates read/write operations through the data paths.

### 6.1.1	Key Features

- Descriptor Processing: Parses 256-bit descriptors for transfer parameters

- Beat-Based Tracking: All lengths tracked in beats (data-width units)

- Concurrent Read/Write: Simultaneous read and write to prevent deadlock

- Three descriptor opcodes: DATA (concurrent read/write), CTRL_READ (consumer gate via the control-read engine), CTRL_WRITE (producer doorbell via the control-write engine) – see Control Interface below

- Timeout Detection: Configurable watchdog for stalled transfers

- MonBus Integration: State transition and error event reporting

- Error Aggregation: Combines errors from descriptor engine and data paths

### 6.1.2	Block Diagram

### 6.1.3	Figure 2.1.1: Scheduler Block Diagram

Scheduler block diagram

Source: 01_scheduler.dot

## 6.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels in system
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Channel ID width
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width (beats)

// Monitor Bus Parameters
parameter logic [7:0] MON_AGENT_ID = 8'h30;      // RAPIDS Scheduler Agent ID
parameter logic [3:0] MON_UNIT_ID = 4'h1;        // Unit identifier
parameter logic [5:0] MON_CHANNEL_ID = 6'h0;     // Base channel ID

// Descriptor Width
parameter int DESC_WIDTH = 256;                  // RAPIDS descriptor width

: Table 2.1.1: Scheduler Parameters

## 6.3	Port List

### 6.3.1	Clock and Reset

Table 2.1.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 6.3.2	Configuration Interface

Table 2.1.3: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable this channel |
| cfg_channel_reset | input | 1 | Channel soft reset |
| cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window (cycles) |
| cfg_sched_timeout_limit | input | 8 | Consecutive-timeout windows before fatal escalation (0 = never) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |

### 6.3.3	Descriptor Engine Interface

Table 2.1.4: Descriptor Engine Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | input | 1 | Descriptor valid |
| descriptor_ready | output | 1 | Ready to accept descriptor |
| descriptor_packet | input | 256 | 256-bit descriptor |
| descriptor_error | input | 1 | Error from descriptor engine |

### 6.3.4	Data Read Interface

Table 2.1.5: Data Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Read request valid |
| sched_rd_addr | output | AW | Source address |
| sched_rd_beats | output | 32 | Beats to read |
| sched_rd_done_strobe | input | 1 | Read complete strobe |
| sched_rd_beats_done | input | 32 | Beats completed |
| sched_rd_error | input | 1 | Read error |

### 6.3.5	Data Write Interface

Table 2.1.6: Data Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | output | 1 | Write request valid (gated on remaining ISSUE beats) |
| sched_wr_addr | output | AW | Destination address |
| sched_wr_beats | output | 32 | Beats to write |
| sched_wr_done_strobe | input | 1 | Write AW-issue strobe (burst accepted on AW) |
| sched_wr_beats_done | input | 32 | Beats issued this strobe |
| sched_wr_commit_strobe | input | 1 | Write COMMIT strobe (B response received) |
| sched_wr_commit_beats | input | 32 | Beats committed this strobe |
| sched_wr_error | input | 1 | Write error |

### 6.3.6	Control Interface

For CTRL_READ / CTRL_WRITE descriptors the scheduler drives two per-channel control engines instead of the data path. Both are request/response handshakes whose completion is signalled by the engine’s *_idle going high after issue.

Table 2.1.7: Control Interface (CTRL_READ / CTRL_WRITE)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctrlrd_valid | output | 1 | Control-read request valid (to ctrlrd_engine) |
| ctrlrd_ready | input | 1 | Engine accepted the poll request |
| ctrlrd_addr | output | AW | Gate poll address (descriptor poll_addr) |
| ctrlrd_data | output | 32 | Expected value (descriptor expected) |
| ctrlrd_mask | output | 32 | Compare mask (descriptor mask) |
| ctrlrd_error | input | 1 | Engine error (poll retries exhausted / AXI error) |
| ctrlrd_idle | input | 1 | ctrlrd_engine_idle – gate satisfied (completion) |
| ctrlwr_valid | output | 1 | Control-write request valid (to ctrlwr_engine) |
| ctrlwr_ready | input | 1 | Engine accepted the doorbell request |
| ctrlwr_addr | output | AW | Doorbell address (descriptor wr_addr) |
| ctrlwr_data | output | 32 | Doorbell value (descriptor wr_data) |
| ctrlwr_error | input | 1 | Engine error (AXI error) |
| ctrlwr_idle | input | 1 | ctrlwr_engine_idle – doorbell written (completion) |

A CTRL_READ descriptor is a consumer gate: the scheduler asserts ctrlrd_valid and holds off the descriptor’s completion (and the chain) until the control-read engine reports the masked poll matched (ctrlrd_idle). The poll retry budget is bounded by CTRL_CONFIG.CTRLRD_MAX_TRY (Section 3.12) so a never-matching gate cannot hang the channel – exhaustion raises ctrlrd_error. A CTRL_WRITE descriptor is a producer doorbell: a single-beat write of ctrlwr_data to ctrlwr_addr, after which the chain continues. The scheduler decodes the opcode from the descriptor (rapids_pkg DESC_OP_DATA / DESC_OP_CTRL_READ / DESC_OP_CTRL_WRITE); DATA descriptors ignore this interface. The engines are instantiated per channel in scheduler_group_beats (u_ctrlrd_engine / u_ctrlwr_engine); see Sections 2.8 and 2.9.

Completion is gated on write-data COMMIT, not AW issue. The scheduler maintains two independent beat counters for the write side:

- r_write_beats_remaining decrements on sched_wr_done_strobe (AW handshake) and drives the destination address and sched_wr_valid (writes are requested only while ISSUE beats remain).

- r_write_beats_to_commit decrements on sched_wr_commit_strobe (B response). A transfer is considered write-complete only when this counter reaches zero (w_write_complete), so a channel is never reported done until every issued burst has been acknowledged on the B channel.

## 6.4	FSM States

### 6.4.1	Figure 2.1.2: Scheduler FSM State Diagram

States and transitions of scheduler_beats (one-hot channel_state_t in rapids_pkg).

Diagram 1

### 6.4.2	State Encoding

Table 2.1.7: FSM State Encoding

| State | Encoding | Description |
| --- | --- | --- |
| CH_IDLE | 7’b0000001 | Waiting for a descriptor |
| CH_FETCH_DESC | 7’b0000010 | Descriptor latched and decoded |
| CH_XFER_DATA | 7’b0000100 | Concurrent read/write transfer, or the control operation |
| CH_COMPLETE | 7’b0001000 | Transfer complete; chain check |
| CH_NEXT_DESC | 7’b0010000 | Waiting for the next chained descriptor |
| CH_ERROR | 7’b0100000 | Sticky error; left only by channel reset |
| CH_RESERVED | 7’b1000000 | Reserved |

## 6.5	Operation

### 6.5.1	Descriptor Format

Bits [63:0]    - src_addr: Source memory address
Bits [127:64]  - dst_addr: Destination memory address
Bits [159:128] - length: Transfer length in beats
Bits [191:160] - next_ptr: Next descriptor pointer (0 = last)
Bits [195:192] - channel_id: Channel identifier
Bits [196]     - last: Last descriptor flag
Bits [197]     - gen_irq: Generate interrupt
Bits [255:198] - reserved

### 6.5.2	Transfer Sequence

### 6.5.3	Figure 2.1.3: Basic Transfer Timing

Scheduler - one descriptor through the FSM

Source: scheduler_basic_transfer.json, captured from dv/tests/fub_beats/test_scheduler_beats.py (basic_flow, channel 0 of 8, 512-bit, seed 7) with WAVES=1.

Reading it: descriptor_valid is accepted in IDLE; the FSM steps through FETCH_DESC into XFER_DATA and scheduler_idle drops. In XFER_DATA both engines are requested at once (sched_rd_valid and sched_wr_valid high together): the read engine reports each burst issued on sched_rd_done_strobe, the write engine reports issue on sched_wr_done_strobe and the B response on sched_wr_commit_strobe (here the model answers B immediately, so the two coincide). The write side finishes its beats first and drops sched_wr_valid; when the read side has issued its last burst the FSM moves to COMPLETE and back to IDLE, and descriptor_ready re-asserts for the next descriptor.

## 6.6	Run-Base Generator Flush

The two stream_run_addr_gen instances (EXT addressing) are flushed by the registered channel reset: generation stops, results in flight in dma_address_gen are discarded and the base FIFO (a gaxi_drop_fifo_sync) is dropped whole. Before 2026-10-21 nothing emptied it, and an EXT descriptor following a channel reset that interrupted another EXT descriptor began its second run at the interrupted descriptor’s base (rapids BUG-017, found in the byte-granular RAPIDS and fixed in RAPIDS, RAPIDS Beats and STREAM together). test_*_ext_channel_reset covers it.

## 6.7	Error Handling

Table 2.1.8: Error Handling

| Error Source | Detection | Response |
| --- | --- | --- |
| Descriptor engine | descriptor_error | Transition to CH_ERROR state |
| Read engine | sched_rd_error | Set sticky error flag |
| Write engine | sched_wr_error | Set sticky error flag |
| Write-progress timeout | Recoverable, escalates after limit | See below |

### 6.7.1	Recoverable Write-Progress Timeout

The write-progress watchdog is recoverable and only escalates to a fatal fault after a configurable number of consecutive stalled windows:

- Window counting: r_timeout_counter increments while sched_wr_valid && !sched_wr_ready (a write request is stalled). The window expires when it reaches cfg_sched_timeout_cycles.

- Re-arm: On expiry the counter re-arms (resets to 0) and the scheduler keeps waiting; a single expired window is not fatal.

- Strike counting: r_timeout_strikes (8-bit) increments once per expired window. Any write progress – sched_wr_done_strobe (AW issue) or sched_wr_commit_strobe (B response) – clears both the window counter and the strike counter.

- Escalation: When cfg_sched_timeout_limit != 0 and r_timeout_strikes >= cfg_sched_timeout_limit, the channel escalates to a sticky CH_ERROR (surfaced on sched_error). Total time to escalate is approximately cfg_sched_timeout_limit * cfg_sched_timeout_cycles.

- Never-escalate mode: cfg_sched_timeout_limit == 0 is a pure soft timeout – the event may be reported, but the channel never faults.

scheduler_idle excludes CH_ERROR. Only CH_IDLE (with no active channel reset) is reported idle; a faulted channel in CH_ERROR is deliberately NOT reported idle so a wedged channel cannot read back as idle. The fault is surfaced via sched_error instead.

## 6.8	MonBus Events

Table 2.1.9: MonBus Event Codes

| Event Code | Description |
| --- | --- |
| 0x01 | Descriptor received |
| 0x02 | Transfer started |
| 0x03 | Transfer complete |
| 0x04 | Error occurred |
| 0x05 | Timeout |

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Descriptor Engine Specification

Module: descriptor_engine_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

## 7.1	Overview

The Descriptor Engine fetches and manages 256-bit descriptors from memory via AXI4. It maintains a prefetch FIFO to hide memory latency and provides parsed descriptors to the scheduler.

### 7.1.1	Key Features

- AXI4 Descriptor Fetch: Single-beat 256-bit reads

- Prefetch FIFO: Configurable depth for latency hiding

- Address Range Checking: Validates descriptor addresses

- Error Detection: Address violations, AXI errors

- MonBus Integration: Fetch events and error reporting

### 7.1.2	Block Diagram

### 7.1.3	Figure 2.2.1: Descriptor Engine Block Diagram

Descriptor engine block diagram

Source: 02_descriptor_engine.dot

## 7.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int FIFO_DEPTH = 4;                    // Prefetch FIFO depth

// Monitor Bus Parameters
parameter logic [7:0] MON_AGENT_ID = 8'h10;      // Descriptor Engine Agent ID
parameter logic [3:0] MON_UNIT_ID = 4'h1;        // Unit identifier

: Table 2.2.1: Descriptor Engine Parameters

## 7.3	Port List

### 7.3.1	Clock and Reset

Table 2.2.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 7.3.2	APB Programming Interface

Table 2.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Kick-off valid (start descriptor chain) |
| apb_ready | output | 1 | Ready to accept kick-off |
| apb_addr | input | AW | First descriptor address |

### 7.3.3	Configuration Interface

Table 2.2.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_prefetch_enable | input | 1 | Enable prefetch chaining (0 = on-demand, ~1 ahead) |
| cfg_fifo_threshold | input | 4 | Prefetch depth (descriptors buffered ahead when enabled) |
| cfg_addr0_base | input | AW | Valid address range 0 base |
| cfg_addr0_limit | input | AW | Valid address range 0 limit |
| cfg_addr1_base | input | AW | Valid address range 1 base |
| cfg_addr1_limit | input | AW | Valid address range 1 limit |

### 7.3.4	AXI4 Read Master Interface

Table 2.2.5: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ar_valid | output | 1 | Address valid |
| ar_ready | input | 1 | Address ready |
| ar_addr | output | ADDR_WIDTH | Descriptor fetch address |
| ar_len | output | 8 | Burst length - 1 |
| ar_size | output | 3 | Burst size (log2 bytes) |
| ar_burst | output | 2 | Burst type (INCR) |
| ar_id | output | AXI_ID_WIDTH | Transaction ID |
| ar_lock | output | 1 | Lock type |
| ar_cache | output | 4 | Cache attributes |
| ar_prot | output | 3 | Protection attributes |
| ar_qos | output | 4 | Quality of service |
| ar_region | output | 4 | Region identifier |
| r_valid | input | 1 | Data valid |
| r_ready | output | 1 | Data ready |
| r_data | input | 256 | Descriptor payload (fixed 256-bit) |
| r_resp | input | 2 | Read response |
| r_last | input | 1 | Last beat |
| r_id | input | AXI_ID_WIDTH | Response ID |

### 7.3.5	Scheduler Interface

Table 2.2.6: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | output | 1 | Descriptor valid |
| descriptor_ready | input | 1 | Scheduler ready |
| descriptor_packet | output | 256 | Parsed descriptor (fixed 256-bit) |
| descriptor_ext_packet | output | 256 | Extended chunk 1 (0 when unused) |
| descriptor_error | output | 1 | Error flag |
| descriptor_eos | output | 1 | End of Stream |
| descriptor_eol | output | 1 | End of Line |
| descriptor_eod | output | 1 | End of Data |
| descriptor_type | output | 2 | Packet type |
| channel_idle | input | 1 | Scheduler idle (enables APB kick-off) |
| descriptor_engine_idle | output | 1 | Engine idle |

## 7.4	FSM States

### 7.4.1	Figure 2.2.2: Descriptor Engine FSM

States and transitions of the descriptor_engine_beats fetch FSM.

Diagram 2

## 7.5	Operation

### 7.5.1	Descriptor Chain Processing

### 7.5.2	Figure 2.2.3: Descriptor Chain Timing

Descriptor Engine - three-descriptor chain

Source: descriptor_engine_chain.json, captured from dv/tests/fub_beats/test_descriptor_engine_beats.py (descriptor_chaining, 32 channels, seed 7) with WAVES=1.

Reading it: one apb_valid kick carries the head address (0x10000). The engine issues an AXI read for it (ar_valid, ar_addr), the data returns on r_valid, and the parsed descriptor is presented on descriptor_valid to a ready scheduler. Because the descriptor’s next_ptr points at 0x10040 and last is clear, the engine fetches the next one on its own – and the one after that at 0x10080 – with no further APB traffic; descriptor_eol marks the last of the chain. Each fetch costs the same AR -> R -> present latency, so a chain streams at one descriptor per fetch round trip.

## 7.6	Address Range Checking

Descriptors must fall within configured valid address ranges:

Valid if: (addr >= cfg_addr0_base && addr <= cfg_addr0_limit)
       || (addr >= cfg_addr1_base && addr <= cfg_addr1_limit)

Invalid addresses generate an error and halt the descriptor chain.

## 7.7	Prefetch and Autonomous Chaining

cfg_prefetch_enable and cfg_fifo_threshold are functional prefetch controls (previously declared-but-unused, which left chaining permanently greedy). They throttle how far ahead the engine fetches chained descriptors by comparing the output descriptor FIFO occupancy (w_desc_fifo_count) against a computed w_prefetch_limit:

if      (!cfg_prefetch_enable)      w_prefetch_limit = 1;   // on-demand
else if (cfg_fifo_threshold == 0)   w_prefetch_limit = 1;   // guard (0 would stall)
else                                w_prefetch_limit = cfg_fifo_threshold;

w_prefetch_allows = (w_desc_fifo_count < w_prefetch_limit);

- Prefetch OFF (cfg_prefetch_enable = 0): limit forced to 1, so a chained fetch is only allowed when the output FIFO has drained – the engine stays roughly one descriptor ahead of the scheduler (on-demand).

- Prefetch ON: the engine buffers up to cfg_fifo_threshold descriptors ahead.

### 7.7.1	Chaining Decision

A chain fetch issues immediately when all of the following hold:

w_should_chain = w_chain_eligible     // valid non-last descriptor, in range, no error
              && w_desc_committed      // current descriptor accepted into output FIFO
              && w_prefetch_allows     // occupancy below prefetch limit
              && w_desc_addr_fifo_wr_ready;  // address FIFO has a slot

### 7.7.2	Deferred (Throttle-Blocked) Chain Fetch

If a descriptor commits and is chain-eligible but the fetch cannot issue this cycle (throttled by w_prefetch_allows, or the address FIFO is full), the owed fetch is recorded rather than dropped:

- r_chain_pending is set and the next-descriptor pointer is held in r_pending_chain_addr.

- The deferred fetch is reissued (w_pending_push_fire) once the output FIFO drains enough that w_prefetch_allows is true again and the address FIFO has a slot, clearing r_chain_pending.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	AXI Read Engine Specification

Module: axi_read_engine_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2026-09-29

## 8.1	Overview

The AXI Read Engine performs burst reads from system memory and writes data to the SRAM buffer. It operates as a streaming pipeline with no FSM, maximizing throughput through continuous data flow.

### 8.1.1	Key Features

- Streaming Pipeline: No FSM - pure streaming architecture

- Multi-Channel Support: Channel ID passed through AXI ID field

- Configurable Burst Length: Up to 256 beats per burst

- SRAM fill only: writes R-channel beats straight into the SRAM controller’s fill_* port (the latency bridge lives in STREAM’s sram_controller_unit, not here)

- Error Handling: AXI RRESP error detection and reporting

### 8.1.2	Block Diagram

### 8.1.3	Figure 2.3.1: AXI Read Engine Block Diagram

AXI read engine block diagram

Source: 03_axi_read_engine.dot

## 8.2	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width
parameter int AXI_ID_WIDTH = 8;                  // AXI ID width
parameter int MAX_OUTSTANDING = 8;               // Max outstanding AR transactions
parameter int PIPELINE = 1;                      // Pipeline stages

// Derived
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS);

: Table 2.3.1: AXI Read Engine Parameters

## 8.3	Port List

### 8.3.1	Clock and Reset

Table 2.3.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 8.3.2	Scheduler Interface

Table 2.3.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | NC | Channel requests read |
| sched_rd_addr | input | NC x AW | Source addresses |
| sched_rd_beats | input | NC x 32 | Beats remaining to read |
| sched_rd_done_strobe | output | NC | Burst completed (pulsed 1 cycle) |
| sched_rd_beats_done | output | NC x 32 | Beats completed in burst |
| sched_rd_error | output | NC | Sticky error flag per channel (bad R response) |

### 8.3.3	AXI4 Read Master Interface

Table 2.3.4: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arid | output | IW | Read address ID |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length (beats - 1) |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type (INCR) |
| m_axi_arvalid | output | 1 | Address valid |
| m_axi_arready | input | 1 | Address ready |
| m_axi_rid | input | IW | Read data ID |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rlast | input | 1 | Last beat |
| m_axi_rvalid | input | 1 | Data valid |
| m_axi_rready | output | 1 | Data ready |

### 8.3.4	SRAM Allocation and Fill Interface

Table 2.3.5: SRAM Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_alloc_req | output | 1 | Channel requests space |
| axi_rd_alloc_size | output | 8 | Beats to reserve |
| axi_rd_alloc_id | output | IW | Transaction ID selects the channel |
| axi_rd_alloc_space_free | input | NC x SCW | Free space (beats available) per channel |
| axi_rd_sram_valid | output | 1 | Read data valid |
| axi_rd_sram_ready | input | 1 | SRAM ready to accept data |
| axi_rd_sram_id | output | IW | Transaction ID selects the channel |
| axi_rd_sram_data | output | DW | Read data payload |

## 8.4	Operation

### 8.4.1	Burst Length Cap (rapids BUG-009)

cfg_axi_rd_xfer_beats is an ARLEN (0..255), but a burst can never be larger than the SRAM it lands in: w_space_ok pre-allocates the whole burst before the AR, and a burst the buffer cannot hold would never be allocatable. The engine therefore clamps the configured value to XFER_MAX = min(2^(SCW-1) - 1, 254) – the buffer depth SEG_COUNT_WIDTH encodes, minus one, and 254 beyond that so every beat count fits the 8-bit size ports shared with sram_controller. On the Genesys 2 build (128-deep) ARLEN 255 runs as 128-beat bursts.

Before the clamp (2026-09-29), ARLEN 255 was taken literally and the 8-bit ARLEN + 1 in the gate wrapped to 0: w_space_ok passed on any free count, the AR for 256 beats allocated 0, and the engine over-fetched past the buffer (4103 R beats for a 4096-beat descriptor on the board, golden mismatch).

### 8.4.2	Streaming Pipeline Architecture

The AXI read engine uses a streaming pipeline with no FSM:

- Scheduler request: per-channel beat count, masked by SRAM space.

- AR issue: round-robin grant, burst address and length, space allocation.

- R receive: data routed to the channel by RID.

- SRAM write: one fill beat per R beat.

### 8.4.3	Timing Diagram

### 8.4.4	Figure 2.3.2: AXI Read Burst Timing

AXI Read Engine - one 8-beat burst

Source: axi_read_engine_burst.json, captured from dv/tests/fub_beats/test_axi_read_engine_beats.py (single, 4 ch, 256-bit, cfg_axi_rd_xfer_beats = 7, seed 7) with WAVES=1.

Reading it: the scheduler holds sched_rd_valid with 26 beats remaining; the arbiter grants channel 0 and the AR (arlen = 7) handshakes two cycles later. The cycle after the handshake the engine pulses sched_rd_done_strobe with beats_done = 8 (the scheduler advances address and remaining on it – note sched_rd_beats drops 26 to 18 and araddr for the next AR is already 0x100) and axi_rd_alloc_req with alloc_size = 8; the modelled space_free debits two cycles later, as the real controller does. R beats pass straight through to axi_rd_sram_valid; rlast closes the burst. The engine never waits for the data before issuing the next AR of another channel, but at PIPELINE = 0 this channel’s next AR waits for rlast.

## 8.5	Burst Segmentation

Large transfers are segmented into AXI-compliant bursts:

Total beats = 256
cfg_xfer_beats = 64

Bursts issued:
  AR[0]: addr=0x1000, len=63 (64 beats)
  AR[1]: addr=0x2000, len=63 (64 beats)
  AR[2]: addr=0x3000, len=63 (64 beats)
  AR[3]: addr=0x4000, len=63 (64 beats)

## 8.6	Error Handling

Table 2.3.6: Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | m_axi_rresp == 2'b10 | Set sched_rd_error, continue |
| AXI DECERR | m_axi_rresp == 2'b11 | Set sched_rd_error, continue |

Last Updated: 2026-09-29

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	AXI Write Engine Specification

Module: axi_write_engine_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2026-09-29

## 9.1	Overview

The AXI Write Engine reads data from SRAM and performs burst writes to system memory. It operates as a streaming pipeline and includes write response handling for completion tracking.

### 9.1.1	Key Features

- Streaming Pipeline: No FSM - pure streaming architecture

- Multi-Channel Support: Channel ID in AXI ID field

- Write Response Tracking: B channel completion handling

- Configurable Burst Length: Up to 256 beats per burst

- Registered SRAM read: consumes the SRAM controller’s registered drain_* stream (the latency bridge lives in STREAM’s sram_controller_unit, not here)

### 9.1.2	Block Diagram

### 9.1.3	Figure 2.4.1: AXI Write Engine Block Diagram

AXI write engine block diagram

Source: 04_axi_write_engine.dot

## 9.2	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width
parameter int AXI_ID_WIDTH = 8;                  // AXI ID width
parameter int MAX_OUTSTANDING = 8;               // Max outstanding AW transactions
parameter int W_FIFO_DEPTH = 64;                 // Write data FIFO depth
parameter int B_FIFO_DEPTH = 16;                 // Write response FIFO depth
parameter int PIPELINE = 1;                      // Pipeline stages

// Derived
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS);

: Table 2.4.1: AXI Write Engine Parameters

## 9.3	Port List

### 9.3.1	Clock and Reset

Table 2.4.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 9.3.2	Scheduler Interface

Table 2.4.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | input | NC | Channel requests write |
| sched_wr_ready | output | NC | Engine ready for channel |
| sched_wr_addr | input | NC x AW | Destination addresses |
| sched_wr_beats | input | NC x 32 | Beats remaining to write |
| sched_wr_burst_len | input | NC x 8 | Requested burst length |
| sched_wr_done_strobe | output | NC | Burst ISSUED on AW handshake (pulsed 1 cycle) |
| sched_wr_beats_done | output | NC x 32 | Beats issued in burst (awlen + 1) |
| sched_wr_commit_strobe | output | NC | Burst COMMITTED on B response (pulsed 1 cycle) |
| sched_wr_commit_beats | output | NC x 32 | Beats committed in burst |
| sched_wr_error | output | NC | Sticky error flag per channel (bad B response) |

Two completion strobes per channel. The engine reports write progress to the scheduler at two points, letting the scheduler gate completion on data actually landing in memory:

- sched_wr_done_strobe / sched_wr_beats_done pulse when the AW command handshakes (m_axi_awvalid && m_axi_awready); the beat count is taken directly from m_axi_awlen + 1. This advances the scheduler’s destination address (ISSUE tracking).

- sched_wr_commit_strobe / sched_wr_commit_beats pulse when the matching B response arrives (m_axi_bvalid && m_axi_bready for the channel); the beat count is recovered from the per-channel B-phase transaction FIFO. This is the COMMIT the scheduler uses to declare the transfer complete.

### 9.3.3	AXI4 Write Master Interface

Table 2.4.4: AXI4 Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awid | output | IW | Write address ID |
| m_axi_awaddr | output | AW | Write address |
| m_axi_awlen | output | 8 | Burst length (beats - 1) |
| m_axi_awsize | output | 3 | Burst size |
| m_axi_awburst | output | 2 | Burst type (INCR) |
| m_axi_awvalid | output | 1 | Address valid |
| m_axi_awready | input | 1 | Address ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat |
| m_axi_wuser | output | UW | Channel ID for transaction tracking |
| m_axi_wvalid | output | 1 | Data valid |
| m_axi_wready | input | 1 | Data ready |
| m_axi_bid | input | IW | Response ID |
| m_axi_bresp | input | 2 | Write response |
| m_axi_bvalid | input | 1 | Response valid |
| m_axi_bready | output | 1 | Response ready |

### 9.3.4	SRAM Reservation and Drain Interface

Table 2.4.5: SRAM Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_drain_req | output | NC | Channel requests to reserve data |
| axi_wr_drain_size | output | NC x 8 | Beats to reserve |
| axi_wr_drain_data_avail | input | NC x SCW | Data available after reservations |
| axi_wr_sram_valid | input | NC | Per-channel valid (registered, for arbitration) |
| axi_wr_sram_valid_comb | input | NC | Per-channel valid (combinational, gates m_axi_wvalid) |
| axi_wr_sram_drain | output | 1 | Drain request (consumer ready) |
| axi_wr_sram_id | output | CIW | Channel ID select for drain |
| axi_wr_sram_data | input | DW | Data from the selected channel (muxed) |

## 9.4	Operation

### 9.4.1	Burst Length Cap (rapids BUG-009)

cfg_axi_wr_xfer_beats is an AWLEN (0..255), but a burst can never be larger than the SRAM that stages it: w_has_data waits for the whole burst to be in the buffer, and a burst the buffer cannot hold would wait forever. The engine therefore clamps the configured value to XFER_MAX = min(2^(SCW-1) - 1, 254) – the buffer depth SEG_COUNT_WIDTH encodes, minus one, and 254 beyond that so every beat count fits the 8-bit size ports shared with sram_controller. On the Genesys 2 build (128-deep) AWLEN 255 runs as 128-beat bursts.

Before the clamp (2026-09-29), AWLEN 255 was taken literally and the 8-bit AWLEN + 1 in the gate wrapped to 0: w_has_data passed on an empty buffer, the AW went out, drain_ctrl reserved 0 beats and the W phase pulled 256 beats that were never reserved. The channel’s drain_data_available then read 0 for good (CHANNEL_RESET does not reach sram_controller), and every later run on the channel accepted one buffer of ingress and never issued an AW.

### 9.4.2	Write Transaction Phases

AXI writes have three phases that can overlap:

Phase 1: AW (Address)    Phase 2: W (Data)    Phase 3: B (Response)
    |                        |                      |
    +-- Issue address        +-- Stream data        +-- Receive response
        before data             from SRAM               track completion

### 9.4.3	Timing Diagram

### 9.4.4	Figure 2.4.2: AXI Write Burst Timing

AXI Write Engine - one 8-beat burst

Source: axi_write_engine_burst.json, captured from dv/tests/fub_beats/test_axi_write_engine_beats.py (single, 4 ch, 256-bit, cfg_axi_wr_xfer_beats = 7, seed 7) with WAVES=1.

Reading it: the SRAM model reports 26 beats available for channel 0 and the scheduler holds sched_wr_valid. The arbiter grants the channel, the AW (awlen = 7) is registered and handshakes; in that same cycle axi_wr_drain_req reserves 8 beats (data_avail drops to 18 two cycles later, the controller’s reporting latency), and the cycle after, sched_wr_done_strobe reports 8 beats issued so the scheduler advances (awaddr for the next AW is already 0x100). W beats stream from the ID-selected drain port (axi_wr_sram_drain is exactly wvalid && wready), wlast closes the burst, and the B response two cycles later raises sched_wr_commit_strobe. At PIPELINE = 0 the channel’s next AW waits for that B.

That wait is the throughput cost of PIPELINE = 0: with 8-beat bursts a channel spends the AW-to-B round trip idle, and on the Genesys 2 the 8-channel sink write path measured 49.9 % engaged utilization at 4096 beats per channel once BUG-005 made the engine honour one-in-flight (the pre-fix engine had run two by accident and read 100 %). The top and core therefore default PIPELINE = 1, which restores line rate, and since 2026-09-28 so does every module that declares the parameter, this engine included: 0 is selectable, never the default.

## 9.5	Error Handling

Table 2.4.6: Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI SLVERR | m_axi_bresp == 2'b10 | Set sched_wr_error |
| AXI DECERR | m_axi_bresp == 2'b11 | Set sched_wr_error |

Last Updated: 2026-09-29

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Beats Alloc Control Specification

Module: alloc_ctrl_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

No longer in the SRAM datapath (2026-09-26). RAPIDS’ SRAM path now uses STREAM’s stream_alloc_ctrl / stream_drain_ctrl / stream_latency_bridge, reached through the snk_/src_sram_controller_beats naming wrappers (bdf4e0dff). This FUB is still built and still verified – it keeps its own test, its own filelist and its rapids_all.f entry – it simply is not instantiated by the SRAM controllers any more. Treat this page as the FUB’s own specification, not as a description of the current SRAM path.

## 10.1	Overview

The Beats Alloc Control is a “virtual FIFO” that tracks space allocation without storing actual data. It uses FIFO pointer logic to manage pre-allocation for AXI engines, preventing race conditions between allocation and actual data arrival.

### 10.1.1	Key Features

- Virtual FIFO: Pointer tracking without data storage

- Variable-Size Allocation: Supports multi-beat allocation requests

- Single-Beat Release: Data writes release one beat at a time

- Status Flags: Full, almost full, empty, almost empty

- Space Tracking: Reports available space for flow control

### 10.1.2	Block Diagram

### 10.1.3	Figure 2.5.1: Beats Alloc Control Block Diagram

Alloc control block diagram

Source: 05_alloc_ctrl.dot

## 10.2	Concept: Virtual FIFO

Unlike a traditional FIFO that stores data, alloc_ctrl_beats only tracks space reservations:

|  | Traditional FIFO | Virtual FIFO (alloc_ctrl_beats) |
| --- | --- | --- |
| Write side | wr_data into storage | wr_size advances a pointer |
| Read side | rd_data out of storage | one beat advances a pointer |
| Output | data | space_free |

### 10.2.1	Use Case: Pre-Allocation for AXI Reads

1. AXI engine requests N beats of space
   - alloc_ctrl.wr_valid = 1
   - alloc_ctrl.wr_size = N
   - wr_ptr advances by N

2. As AXI read data arrives (1 beat at a time)
   - alloc_ctrl.rd_valid = 1 (each beat)
   - rd_ptr advances by 1

3. Space becomes available again
   - space_free reflects available capacity

## 10.3	Parameters

parameter int DEPTH = 512;                      // Virtual FIFO depth
parameter int ALMOST_WR_MARGIN = 1;             // Almost full margin
parameter int ALMOST_RD_MARGIN = 1;             // Almost empty margin
parameter int REGISTERED = 1;                   // Registered outputs

// Derived
parameter int AW = $clog2(DEPTH);               // Address width

: Table 2.5.1: Beats Alloc Control Parameters

## 10.4	Port List

### 10.4.1	Clock and Reset

Table 2.5.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low reset |

### 10.4.2	Write Interface (Allocation Requests)

Table 2.5.3: Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | Allocate space |
| wr_size | input | 8 | Number of beats to allocate |
| wr_ready | output | 1 | Space available for allocation |

### 10.4.3	Read Interface (Data Written)

Table 2.5.4: Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | One beat of data written |
| rd_ready | output | 1 | Not full (can accept) |

### 10.4.4	Status Outputs

Table 2.5.5: Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| space_free | output | AW+1 | Available space in beats |
| wr_full | output | 1 | No space available |
| wr_almost_full | output | 1 | Near full |
| rd_empty | output | 1 | No allocations pending |
| rd_almost_empty | output | 1 | Near empty |

## 10.5	Operation

### 10.5.1	Pointer Arithmetic

space_free = DEPTH - (wr_ptr - rd_ptr)

Full condition:    wr_ptr - rd_ptr >= DEPTH
Empty condition:   wr_ptr == rd_ptr

### 10.5.2	Timing Diagram

### 10.5.3	Figure 2.5.2: Allocation and Release Timing

Allocation control - allocate then release

Source: alloc_ctrl_beats_alloc_release.json, captured from dv/tests/fub_beats/test_alloc_ctrl_beats.py (basic_alloc_drain, depth 512, seed 7) with WAVES=1.

Reading it: one wr_valid handshake with wr_size = 6 reserves six beats and space_free drops 512 to 506 the next cycle; rd_empty falls because reserved space is now outstanding. Each later rd_valid handshake releases one beat and space_free climbs back one at a time (507, 508, …). wr_full and wr_almost_full stay low: the virtual FIFO is nowhere near its depth.

## 10.6	Integration Example

// Source path: Pre-allocate SRAM space before AXI read
alloc_ctrl_beats #(
    .DEPTH(SRAM_DEPTH),
    .ALMOST_WR_MARGIN(8)
) u_src_alloc (
    .axi_aclk       (clk),
    .axi_aresetn    (rst_n),

    // Scheduler allocates space before issuing AXI read
    .wr_valid       (sched_rd_valid),
    .wr_size        (sched_rd_beats[7:0]),
    .wr_ready       (alloc_ready),

    // AXI read data arrival releases allocation
    .rd_valid       (m_axi_rvalid && m_axi_rready),
    .rd_ready       (),  // Not used for single-beat

    // Status
    .space_free     (sram_space_available),
    .wr_full        (sram_full),
    .wr_almost_full (sram_almost_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Beats Drain Control Specification

Module: drain_ctrl_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented Last Updated: 2025-01-10

No longer in the SRAM datapath (2026-09-26). RAPIDS’ SRAM path now uses STREAM’s stream_alloc_ctrl / stream_drain_ctrl / stream_latency_bridge, reached through the snk_/src_sram_controller_beats naming wrappers (bdf4e0dff). This FUB is still built and still verified – it keeps its own test, its own filelist and its rapids_all.f entry – it simply is not instantiated by the SRAM controllers any more. Treat this page as the FUB’s own specification, not as a description of the current SRAM path.

## 11.1	Overview

The Beats Drain Control is a “virtual FIFO” that tracks data availability without storing actual data. It complements alloc_ctrl_beats by tracking when data has been written to SRAM and is available for draining.

### 11.1.1	Key Features

- Virtual FIFO: Pointer tracking without data storage

- Single-Beat Write: Data arrivals tracked one beat at a time

- Variable-Size Drain: Supports multi-beat drain requests

- Status Flags: Full, almost full, empty, almost empty

- Data Availability: Reports beats available for drain

### 11.1.2	Block Diagram

### 11.1.3	Figure 2.6.1: Beats Drain Control Block Diagram

Drain control block diagram

Source: 06_drain_ctrl.dot

## 11.2	Concept: Data Availability Tracking

The drain_ctrl tracks when data has been written to SRAM:

alloc_ctrl:  Tracks SPACE reservations (allocation -> actual write)
drain_ctrl:  Tracks DATA availability (data write -> drain request)

### 11.2.1	Complementary Roles

Source Path:
  1. Scheduler allocates space (alloc_ctrl.wr)
  2. AXI reads data into SRAM
  3. Data written to SRAM (drain_ctrl.wr)
  4. Drain consumer reads (drain_ctrl.rd)

Sink Path:
  1. Fill allocates space (alloc_ctrl.wr)
  2. Fill writes to SRAM (alloc_ctrl.rd)
  3. Data available in SRAM (drain_ctrl.wr)
  4. AXI write drains SRAM (drain_ctrl.rd)

## 11.3	Parameters

parameter int DEPTH = 512;                      // Virtual FIFO depth
parameter int ALMOST_WR_MARGIN = 1;             // Almost full margin
parameter int ALMOST_RD_MARGIN = 1;             // Almost empty margin
parameter int REGISTERED = 1;                   // Registered outputs

// Derived
parameter int AW = $clog2(DEPTH);               // Address width

: Table 2.6.1: Beats Drain Control Parameters

## 11.4	Port List

### 11.4.1	Clock and Reset

Table 2.6.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low reset |

### 11.4.2	Write Interface (Data Written to SRAM)

Table 2.6.3: Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | One beat written to SRAM |
| wr_ready | output | 1 | Can track more data |

### 11.4.3	Read Interface (Drain Requests)

Table 2.6.4: Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | Drain request |
| rd_size | input | 8 | Number of beats to drain |
| rd_ready | output | 1 | Data available for drain |

### 11.4.4	Status Outputs

Table 2.6.5: Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| data_available | output | AW+1 | Beats available to drain |
| wr_full | output | 1 | Tracking full |
| wr_almost_full | output | 1 | Near full |
| rd_empty | output | 1 | No data to drain |
| rd_almost_empty | output | 1 | Near empty |

## 11.5	Operation

### 11.5.1	Pointer Arithmetic

data_avail = wr_ptr - rd_ptr

Full condition:    wr_ptr - rd_ptr >= DEPTH
Empty condition:   wr_ptr == rd_ptr

### 11.5.2	Timing Diagram

### 11.5.3	Figure 2.6.2: Data Arrival and Drain Timing

Drain control - single-beat arrivals then a multi-beat drain

Source: drain_ctrl_beats_arrive_drain.json, captured from dv/tests/fub_beats/test_drain_ctrl_beats.py (basic_write_drain, depth 512, seed 7) with WAVES=1.

Reading it: six wr_valid handshakes, one beat each, raise data_available 1, 2, … 6 and drop rd_empty on the first arrival. A single rd_valid handshake with rd_size = 6 then reserves all six at once: data_available returns to 0 and rd_empty rises the next cycle. The counter tracks beats, not bytes, and never touches the data itself.

## 11.6	Integration Example

// Sink path: Track data availability for AXI write engine
drain_ctrl_beats #(
    .DEPTH(SRAM_DEPTH),
    .ALMOST_RD_MARGIN(8)
) u_snk_drain (
    .axi_aclk       (clk),
    .axi_aresetn    (rst_n),

    // Fill writes data to SRAM
    .wr_valid       (fill_valid && fill_ready),
    .wr_ready       (),  // Single-beat, always ready

    // AXI write engine drains data
    .rd_valid       (axi_wr_drain_req),
    .rd_size        (axi_wr_burst_len),
    .rd_ready       (data_ready_for_drain),

    // Status
    .data_available (beats_to_write),
    .rd_empty       (no_data_to_write)
);

## 11.7	Relationship: alloc_ctrl vs drain_ctrl

Table 2.6.6: alloc_ctrl vs drain_ctrl Comparison

| Aspect | alloc_ctrl_beats | drain_ctrl_beats |
| --- | --- | --- |
| Tracks | Space reservations | Data availability |
| Write port | Multi-beat allocation | Single-beat arrival |
| Read port | Single-beat release | Multi-beat drain |
| Output | space_free | data_available |
| Use case | Pre-allocate before fill | Know when to drain |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Latency Bridge (beats) Specification

Module: latency_bridge_beats.sv Location: projects/components/dma-ip/rapids/rtl/fub_beats/ Status: Implemented, tested, not in the SRAM path Last Updated: 2026-09-27

Not in the SRAM datapath since 2026-09-26. RAPIDS’ SRAM path is STREAM’s sram_controller, which carries its own stream_latency_bridge (the same design), reached through the snk_/src_sram_controller_beats naming wrappers (bdf4e0dff). This FUB is still built and verified – it keeps its test, its filelist and its rapids_all.f entry – but nothing in rapids_core_beats instantiates it. This page is the FUB’s own specification.

## 12.1	Overview

latency_bridge_beats turns a registered-read FIFO – one whose data appears the cycle after the read handshake – into a plain valid/ready stream the consumer can stall at will. It is a data-width bridge: it moves DATA_WIDTH bits per beat and carries nothing else. There is no beat count, no channel ID and no programmable delay in it; an earlier version of this page described such an interface, and it never existed in this module.

### 12.1.1	Key Features

- One-cycle glue, one skid buffer: a single flop (r_drain_ip) remembers that a FIFO read was accepted, so the data that lands the next cycle is pushed into a gaxi_fifo_sync skid of depth SKID_DEPTH

- Full throughput when the consumer is ready: back-to-back reads are accepted as long as the skid keeps draining

- Backpressure absorbed in the skid: s_ready drops only when a skid write is actually stalled, so the FIFO is never read into a slot that does not exist

- Occupancy reported: occupancy counts the beats the bridge holds (the one in flight plus the skid contents), which is what an occupancy-based data-available counter needs

- Debug taps: dbg_r_pending and dbg_r_out_valid expose the in-flight flop and the skid output valid, for catching stuck data from a test

### 12.1.2	Block Diagram

### 12.1.3	Figure 2.7.1: Latency Bridge Block Diagram

Latency bridge block diagram

Source: 07_latency_bridge.dot

## 12.2	Concept: bridging a registered read

A FIFO with REGISTERED=1 read (the SRAM style this repo uses so BRAM infers cleanly) answers a read handshake with data one cycle later. A consumer speaking valid/ready expects data with valid, and may deassert ready at any time. Without a bridge the FIFO has already popped a beat the consumer did not take.

cycle 0   s_valid && s_ready         FIFO read accepted        r_drain_ip <= 1
cycle 1   data arrives on s_data     skid_wr_valid = r_drain_ip, data pushed
cycle N   m_valid && m_ready         consumer drains the skid at its own pace

The bridge never guesses about downstream readiness: s_ready is derived from the skid’s room, counting only a write that is stalled this cycle as pending (a write that completes this cycle frees its slot at the same edge). That is why full throughput holds while writes complete immediately, and why the FIFO is never over-read when they do not.

## 12.3	Parameters

parameter int DATA_WIDTH = 64;   // Beat width
parameter int SKID_DEPTH = 4;    // Skid buffer depth (2-4 recommended)
parameter int DW = DATA_WIDTH;   // Short alias

: Table 2.7.1: Latency Bridge Parameters

occupancy is 3 bits wide and counts 0..5 at the default depth (one beat in flight plus four in the skid); a larger SKID_DEPTH needs the port widened.

## 12.4	Port List

### 12.4.1	Clock and Reset

Table 2.7.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 12.4.2	Upstream Interface (from the registered FIFO)

Table 2.7.3: Upstream Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_valid | input | 1 | FIFO has a beat to read (not empty) |
| s_ready | output | 1 | Bridge accepts a read this cycle; the data lands NEXT cycle |
| s_data | input | DW | Read data, valid one cycle after the s_valid && s_ready handshake |

### 12.4.3	Downstream Interface (to the consumer)

Table 2.7.4: Downstream Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_valid | output | 1 | Beat available |
| m_ready | input | 1 | Consumer takes the beat |
| m_data | output | DW | Beat data, valid with m_valid |

### 12.4.4	Status and Debug

Table 2.7.5: Status and Debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| occupancy | output | 3 | Beats held by the bridge: the in-flight read plus the skid contents (0..5) |
| dbg_r_pending | output | 1 | r_drain_ip: a read was accepted and its data is due next cycle |
| dbg_r_out_valid | output | 1 | Skid output valid (mirrors m_valid) |

## 12.5	Operation

### 12.5.1	Figure 2.7.2: Latency Bridge Timing (consumer always ready)

Latency bridge behind a registered-read FIFO

Source: latency_bridge_beats_streaming.json, captured from dv/tests/fub_beats/test_latency_bridge_beats.py (streaming, 256-bit, seed 7) with WAVES=1. The test drives a REGISTERED = 1 gaxi_fifo_sync in front of the bridge (dv/tb/latency_bridge_beats_tb_top.sv), because that is the only way to produce the bridge’s upstream contract: a valid/ready master presents data with valid, a registered FIFO presents it one cycle after the read handshake.

Reading it: each wr_valid beat lands in the FIFO and s_valid (FIFO not empty) rises a cycle later; the bridge reads it at once (s_ready stays high) and dbg_r_pending marks the cycle the data is in flight. Two cycles after the write the beat is on m_valid/m_data with its payload intact (1, 2, 3, …), and occupancy shows exactly one beat inside the bridge per transfer. With a consumer that is always ready the bridge adds the FIFO’s read latency and no more.

An earlier revision of this test drove s_* directly from the BFM and never compared m_data; every beat came out 0 and the test passed (rapids TASK-003). (dv/tests/fub_beats/test_latency_bridge_beats.py).

## 12.6	Integration Context

The bridge sits between a gaxi_fifo_sync with REGISTERED=1 and whatever consumes the stream. The same arrangement, with STREAM’s twin stream_latency_bridge, is what sram_controller_unit uses inside the shared SRAM controller today; occupancy there is what the drain accounting must allow for (stream BUG-011).

latency_bridge_beats #(
    .DATA_WIDTH (512),
    .SKID_DEPTH (4)
) u_bridge (
    .clk             (clk),
    .rst_n           (rst_n),
    // registered-read FIFO side
    .s_valid         (fifo_rd_valid),
    .s_ready         (fifo_rd_ready),
    .s_data          (fifo_rd_data),
    // consumer side
    .m_valid         (drain_valid),
    .m_ready         (drain_read),
    .m_data          (drain_data),
    // status / debug
    .occupancy       (bridge_occupancy),
    .dbg_r_pending   (dbg_bridge_pending),
    .dbg_r_out_valid (dbg_bridge_out_valid)
);

## 12.7	Design Considerations

Table 2.7.6: Skid Depth Selection

| SKID_DEPTH | Behaviour |
| --- | --- |
| 2 | Minimum: one stall of the consumer costs a bubble on the next read |
| 4 | Default: absorbs a short consumer stall with no upstream bubble |
| 8 | Deeper stall absorption; occupancy must be widened past 3 bits |

The skid does not need to cover the FIFO’s read latency (that is the one r_drain_ip flop’s job); it needs to cover how long the consumer stalls without the bridge stopping upstream reads.

Last Updated: 2026-09-27

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Control-Read Engine Specification

Module: ctrlrd_engine.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2026-07-12

## 13.1	Overview

The Control-Read Engine implements the CTRL_READ descriptor opcode (DESC_OP_CTRL_READ = 2'b01) as a consumer gate. When the scheduler decodes a CTRL_READ descriptor, it hands the engine a poll address, an expected value, and a compare mask. The engine issues an AXI4 read to the poll address, applies the mask to the returned data, and compares (read_data & mask) against the expected value. If they match, the gate is satisfied and the descriptor chain proceeds; if not, the engine retries. Retries are bounded so a gate that never matches cannot hang the channel.

One Control-Read Engine is instantiated per channel in scheduler_group_beats.sv (u_ctrlrd_engine), configured for 32-bit AXI reads.

### 13.1.1	Key Features

- Consumer Gate Semantics: Polls memory until (read_data & mask) == expected

- Masked Comparison: Only the masked bits of the polled word participate in the match

- Bounded Retry: cfg_ctrlrd_max_try (0-511) caps the number of poll attempts

- 1 microsecond Retry Pacing: Retries wait for tick_1us from the scheduler group

- Error on Exhaustion: Asserts ctrlrd_error if the gate never matches within budget

- Null Address Fast-Path: A poll address of zero completes immediately as a match

- Channel Reset Support: cfg_channel_reset returns the FSM to idle and drains any read still on the fabric

- MonBus Integration: Completion, retry, and error events

### 13.1.2	Block Diagram

### 13.1.3	Figure 2.8.1: Control-Read Engine Block Diagram

Control-read engine block diagram

Source: 08_ctrlrd_engine.dot

## 13.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 32;                 // Total channels
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Channel index width (derived)
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int AXI_DATA_WIDTH = 64;               // AXI read data width (>= 32)
parameter int AXI_ID_WIDTH = 8;                  // AXI ID width (>= CHAN_WIDTH)

// Monitor Bus Parameters
parameter logic [15:0] MON_AGENT_ID  = 16'h0030; // Control-Read Engine Agent ID
parameter logic [7:0]  MON_UNIT_ID   = 8'h02;    // Unit identifier
parameter logic [8:0]  MON_CHANNEL_ID = 9'h000;  // Base channel ID

: Table 2.8.1: Control-Read Engine Parameters

Parameter validation enforces AXI_ID_WIDTH >= CHAN_WIDTH and AXI_DATA_WIDTH >= 32 (the engine consumes only the lower 32 bits of the read data). The per-channel instance in scheduler_group_beats.sv overrides AXI_DATA_WIDTH to 32.

## 13.3	Port List

### 13.3.1	Clock and Reset

Table 2.8.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 13.3.2	Scheduler Interface

Table 2.8.3: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctrlrd_valid | input | 1 | Gate request valid from scheduler |
| ctrlrd_ready | output | 1 | Engine ready to accept a request |
| ctrlrd_pkt_addr | input | AW | Poll address (descriptor poll_addr[63:0]) |
| ctrlrd_pkt_data | input | 32 | Expected value (descriptor expected[95:64]) |
| ctrlrd_pkt_mask | input | 32 | Compare mask (descriptor mask[127:96]) |
| ctrlrd_error | output | 1 | Gate failed (retries exhausted or AXI error) |
| ctrlrd_result | output | 32 | Last read value (lower 32 bits) |
| ctrlrd_engine_idle | output | 1 | Engine idle (post-issue completion indicator) |

### 13.3.3	Configuration Interface

Table 2.8.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_ctrlrd_max_try | input | 9 | Poll retry budget, 0-511 (from CTRL_CONFIG.CTRLRD_MAX_TRY) |
| cfg_channel_reset | input | 1 | Per-channel reset request |
| tick_1us | input | 1 | 1 microsecond tick that paces retries |

### 13.3.4	AXI4 Read Master Interface

Table 2.8.5: AXI4 Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ar_valid | output | 1 | Read address valid |
| ar_ready | input | 1 | Read address ready |
| ar_addr | output | AW | Read address (poll address) |
| ar_len | output | 8 | Burst length (8'h00, single beat) |
| ar_size | output | 3 | Burst size (3'b010, 4 bytes) |
| ar_burst | output | 2 | Burst type (2'b01, INCR) |
| ar_id | output | ID | Transaction ID (channel-derived) |
| ar_lock | output | 1 | Lock (1'b0, normal access) |
| ar_cache | output | 4 | Cache (4'b0010, non-cacheable bufferable) |
| ar_prot | output | 3 | Protection (3'b000) |
| ar_qos | output | 4 | Quality of service (4'h0) |
| ar_region | output | 4 | Region (4'h0) |
| r_valid | input | 1 | Read data valid (shared channel) |
| r_ready | output | 1 | Read data ready |
| r_data | input | ADW | Read data (lower 32 bits used) |
| r_id | input | ID | Read response ID (matched to our request) |
| r_resp | input | 2 | Read response |
| r_last | input | 1 | Read last beat |

### 13.3.5	MonBus Interface

Table 2.8.6: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | 64 | Side-band monitor timestamp |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor consumer ready |
| mon_packet | output | 128 | Monitor packet (monitor_packet_t) |
| mon_timestamp | output | 64 | Monitor packet timestamp |

## 13.4	FSM States

### 13.4.1	Figure 2.8.2: Control-Read Engine FSM

States and transitions of the ctrlrd_engine poll FSM.

Diagram 3

State summary:

- READ_IDLE: Waits for a request from the scheduler (via a two-deep skid buffer). Latches poll address, expected value, mask, and the AXI ID; initializes the retry counter from cfg_ctrlrd_max_try.

- READ_ISSUE_ADDR: Drives ar_valid. A null address (64'h0) shortcuts to READ_MATCH; otherwise, once ar_ready handshakes, advances to READ_WAIT_DATA.

- READ_WAIT_DATA: Accepts the read response whose r_id matches the channel ID, capturing the lower 32 bits and the response code.

- READ_COMPARE: On an AXI error response, goes to READ_ERROR. On a masked match, goes to READ_MATCH. Otherwise, if retries remain, decrements the counter and goes to READ_RETRY_WAIT; if the budget is exhausted, goes to READ_ERROR.

- READ_RETRY_WAIT: Waits for tick_1us, then reissues the read.

- READ_MATCH: Gate satisfied. Clears the error flag and returns to READ_IDLE.

- READ_ERROR: Sets ctrlrd_error and returns to READ_IDLE.

## 13.5	Operation

### 13.5.1	Masked Compare

The engine gates on a masked equality check:

w_masked_expected = ctrlrd_pkt_data & ctrlrd_pkt_mask
w_masked_actual   = r_data[31:0]    & ctrlrd_pkt_mask
w_data_match      = (w_masked_expected == w_masked_actual)

Only the bits set in mask participate. A mask of 32'hFFFFFFFF compares the full word; a mask of a single bit turns the gate into a flag-poll.

### 13.5.2	Retry Budget and Pacing

The retry counter is loaded from cfg_ctrlrd_max_try when the request is accepted. On each non-matching, non-error compare with retries remaining, the counter decrements and the engine parks in READ_RETRY_WAIT until the next tick_1us, which paces successive polls at roughly 1 microsecond intervals. When the counter reaches zero without a match, the engine transitions to READ_ERROR and asserts ctrlrd_error, so a never-matching gate cannot stall the channel indefinitely.

cfg_ctrlrd_max_try is driven by the CTRL_CONFIG register at offset 0x240, field CTRLRD_MAX_TRY[8:0] (0-511, reset 16).

### 13.5.3	Null Address Fast-Path

If the latched poll address is 64'h0, the engine treats the gate as already satisfied: READ_ISSUE_ADDR transitions directly to READ_MATCH without issuing an AXI read. This lets software emit an unconditional (no-op) gate.

### 13.5.4	AXI Read Transaction

Polls are single-beat, 4-byte INCR reads (ar_len = 0, ar_size = 3'b010, ar_burst = 2'b01). The ar_id is derived from CHANNEL_ID, and only responses whose r_id matches are consumed on the shared read data channel. ctrlrd_result exposes the last captured 32-bit value; in the per-channel instantiation the scheduler gates on match/error rather than the raw value, so ctrlrd_result is left unconnected there.

### 13.5.5	Channel Reset

cfg_channel_reset is registered into r_channel_reset_active. While asserted, the engine refuses new requests (ctrlrd_ready is forced low), the retry and error state is cleared, and the FSM is driven back to READ_IDLE.

What the reset cannot do is cancel a read already on the fabric, because AXI has no cancel. The engine therefore drains it (rapids TASK-014, 2026-09-27):

- an AR that was raised and not yet accepted stays raised until the slave takes it (r_drain_ar), then

- the R beat that the accepted AR is owed is taken and discarded (r_drain_pending, which alone lifts r_ready outside READ_WAIT_DATA).

Until the beat is gone ctrlrd_engine_idle stays low and the request path stays closed, so the next read on this ID cannot be answered by the stale beat. Before this the reset dropped the FSM to idle with r_ready low, the beat sat on the bus, and a re-kicked channel took it as its own answer.

ctrlrd_engine_idle asserts only in READ_IDLE with no pending request, no active channel reset and nothing left to drain.

## 13.6	MonBus Reporting

The engine emits monitor packets on notable events:

Table 2.8.7: MonBus Events

| Event | Packet Type | Code | Payload |
| --- | --- | --- | --- |
| Gate satisfied (non-null) | PktTypeCompletion | CORE_COMPL_CTRLRD_COMPLETED | Poll address |
| Retry attempt | PktTypePerf | CORE_PERF_CTRLRD_RETRY | Remaining retry count |
| Retries exhausted | PktTypeError | CORE_ERR_CTRLRD_MAX_RETRIES | Poll address |
| AXI error response | PktTypeError | AXI_ERR_RESP_SLVERR / AXI_ERR_RESP_DECERR | Response code |

All packets carry MON_AGENT_ID, MON_UNIT_ID, and MON_CHANNEL_ID, and are timestamped from i_mon_time.

## 13.7	Descriptor Field Mapping

The CTRL_READ opcode reinterprets the shared 256-bit descriptor (the descriptor engine’s existing field extraction is reused):

Table 2.8.8: CTRL_READ Descriptor Field Mapping

| Descriptor Field | Bits | Engine Port |
| --- | --- | --- |
| poll_addr | [63:0] | ctrlrd_pkt_addr |
| expected | [95:64] | ctrlrd_pkt_data |
| mask | [127:96] | ctrlrd_pkt_mask |
| max_try | [143:128] | Per-descriptor budget (0 = use cfg_ctrlrd_max_try default) |

Last Updated: 2026-07-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Control-Write Engine Specification

Module: ctrlwr_engine.sv Location: projects/components/dma-ip/rapids/rtl/fub/ Status: Implemented Last Updated: 2025-01-10

## 14.1	Overview

The Control-Write Engine executes a CTRL_WRITE control descriptor (DESC_OP_CTRL_WRITE = 2'b10). A CTRL_WRITE is a producer doorbell: the engine performs a single-beat AXI4 write of the descriptor’s 32-bit wr_data value to its 64-bit wr_addr target, then completes so the descriptor chain continues. One instance is dedicated per channel.

The write target address must be 4-byte aligned because the transfer is a 32-bit (aw_size = 3'b010) single-beat write. A null address (wr_addr == 0) is treated as a no-op and the request completes without issuing an AXI transaction.

### 14.1.1	Key Features

- Single-Beat AXI4 Write: One 32-bit doorbell write per request (aw_len = 0, w_last = 1)

- Producer Doorbell Semantics: Write value to target, then release the chain (no poll, no retry)

- Address Alignment Check: Requires 4-byte-aligned wr_addr; misalignment raises an error

- Null-Address Skip: wr_addr == 0 completes as a no-op

- Channel Reset Support: cfg_channel_reset returns the FSM to idle and drains any write still on the fabric

- MonBus Integration: Completion and error event reporting

### 14.1.2	Block Diagram

### 14.1.3	Figure 2.9.1: Control-Write Engine Block Diagram

Control-write engine block diagram

Source: 09_ctrlwr_engine.dot

## 14.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 32;                 // Total channels
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Derived channel index width
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int AXI_ID_WIDTH = 8;                  // AXI transaction ID width

// Monitor Bus Parameters
parameter logic [15:0] MON_AGENT_ID  = 16'h0020; // Ctrlwr Engine Agent ID
parameter logic [7:0]  MON_UNIT_ID   = 8'h01;    // Unit identifier
parameter logic [8:0]  MON_CHANNEL_ID = 9'h000;  // Base channel ID

: Table 2.9.1: Control-Write Engine Parameters

Note: AXI_ID_WIDTH must be >= CHAN_WIDTH (checked by an elaboration-time $fatal). When instantiated in scheduler_group_beats.sv as u_ctrlwr_engine, MON_AGENT_ID is overridden to 16'h0021 (CTRLWR_MON_AGENT_ID) to distinguish it from the Control-Read Engine’s 16'h0020.

## 14.3	Port List

### 14.3.1	Clock and Reset

Table 2.9.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 14.3.2	Configuration Interface

Table 2.9.3: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_reset | input | 1 | Per-channel reset: blocks new requests, drains transaction, forces idle |

### 14.3.3	Scheduler Interface

Table 2.9.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ctrlwr_valid | input | 1 | Request valid (doorbell write requested) |
| ctrlwr_ready | output | 1 | Ready to accept request |
| ctrlwr_pkt_addr | input | AW | Doorbell target address (wr_addr, 4-byte aligned) |
| ctrlwr_pkt_data | input | 32 | Doorbell write value (wr_data) |
| ctrlwr_error | output | 1 | Error flag (alignment or AXI response error) |
| ctrlwr_engine_idle | output | 1 | Engine idle (in WRITE_IDLE, no active transaction, not in channel reset) |

### 14.3.4	AXI4 Write Master Interface

Table 2.9.5: AXI4 Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| aw_valid | output | 1 | Write address valid |
| aw_ready | input | 1 | Write address ready |
| aw_addr | output | AW | Write address (= wr_addr) |
| aw_len | output | 8 | Burst length (0 = single beat) |
| aw_size | output | 3 | Transfer size (3'b010 = 4 bytes / 32-bit) |
| aw_burst | output | 2 | Burst type (2'b01 = INCR) |
| aw_id | output | AXI_ID_WIDTH | Transaction ID (derived from CHANNEL_ID) |
| aw_lock | output | 1 | Lock type (0 = normal) |
| aw_cache | output | 4 | Cache type (4'b0010 = non-cacheable bufferable) |
| aw_prot | output | 3 | Protection type (3'b000) |
| aw_qos | output | 4 | Quality of service (0) |
| aw_region | output | 4 | Region identifier (0) |
| w_valid | output | 1 | Write data valid |
| w_ready | input | 1 | Write data ready |
| w_data | output | 32 | Write data (= wr_data) |
| w_strb | output | 4 | Write strobes (4'b1111 = all bytes) |
| w_last | output | 1 | Last beat (1 = single beat) |
| b_valid | input | 1 | Write response valid |
| b_ready | output | 1 | Write response ready |
| b_id | input | AXI_ID_WIDTH | Response ID (matched against expected AXI ID) |
| b_resp | input | 2 | Write response code |

### 14.3.5	MonBus Interface

Table 2.9.6: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | 64 | Monitor timestamp side-band input |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready |
| mon_packet | output | 128 | Monitor packet (monitor_packet_t) |
| mon_timestamp | output | 64 | Monitor packet timestamp |

## 14.4	FSM States

### 14.4.1	Figure 2.9.2: Control-Write Engine FSM

States and transitions of the ctrlwr_engine doorbell FSM.

Diagram 4

State summary:

- WRITE_IDLE: Wait for a request from the request skid buffer; latch wr_addr/wr_data and the expected AXI ID.

- WRITE_ISSUE_ADDR: Drive aw_valid until aw_ready. A misaligned address routes to WRITE_ERROR; a null address returns to WRITE_IDLE (no-op skip).

- WRITE_ISSUE_DATA: Drive w_valid until w_ready; advance once both address and data phases have been issued.

- WRITE_WAIT_RESP: Accept the matching b-channel response; a non-OKAY b_resp routes to WRITE_ERROR, otherwise to WRITE_COMPLETE.

- WRITE_COMPLETE: Emit a completion MonBus packet (unless null address) and return to WRITE_IDLE.

- WRITE_ERROR: Latch ctrlwr_error, emit an error MonBus packet, and return to WRITE_IDLE. The error flag persists until reset or cfg_channel_reset.

## 14.5	Operation

### 14.5.1	Doorbell Write Sequence

### 14.5.2	Figure 2.9.3: Control-Write Doorbell Timing

ctrlwr_engine - one doorbell write

Source: ctrlwr_engine_doorbell.json, captured from dv/tests/fub/test_ctrlwr_engine.py (basic_write, channel 0, 64-bit address, REG_LEVEL=GATE) with WAVES=1; the AXI side is the framework write slave.

Reading it: the request handshakes in one cycle (ctrlwr_ready is held high by the skid buffer) with the doorbell address 0x1000 and data 0x12345678, and the engine leaves idle the next cycle. AW goes out two cycles after the request, W two cycles after AW, and the slave answers B (OKAY) two cycles after W; ctrlwr_engine_idle returns the cycle after the B handshake, eight cycles after the request. Nothing is pipelined across doorbells: the engine takes one request at a time.

### 14.5.3	Request Handshake and Skid Buffer

Incoming requests are captured through a depth-2 gaxi_skid_buffer. The engine accepts a request (ctrlwr_ready) only when not in an active channel reset. The FSM pops the buffered request in WRITE_IDLE, latching {wr_addr, wr_data} and computing the expected AXI ID from CHANNEL_ID:

aw_id = {{(AXI_ID_WIDTH-CHAN_WIDTH){1'b0}}, CHANNEL_ID[CHAN_WIDTH-1:0]};

The b-channel response is matched against this expected ID (b_id == r_expected_axi_id) so a shared write master can be demultiplexed per channel.

### 14.5.4	Address Validation

Null (skip)  : wr_addr == 64'h0
Align error  : wr_addr[1:0] != 2'b00  (and not null)

A null address completes immediately as a no-op with no AXI activity and no completion packet. A misaligned address never issues on AXI; it transitions to WRITE_ERROR, asserts ctrlwr_error, and emits an error MonBus packet.

### 14.5.5	Error Reporting

ctrlwr_error is a registered, sticky flag. It is set on an alignment error or on a non-OKAY AXI write response (b_resp != 2'b00), and it persists until rst_n or cfg_channel_reset. The MonBus error packet carries either the offending address (alignment error) or the captured b_resp code (AXI response error).

### 14.5.6	Channel Reset

Asserting cfg_channel_reset registers r_channel_reset_active, which blocks new requests (ctrlwr_ready low), clears the error flag and the phase-issued state, and drives the FSM back to WRITE_IDLE.

A write already on the fabric cannot be cancelled, so the engine drains it (rapids TASK-014, 2026-09-27): a raised AW stays up until accepted (r_drain_aw), an accepted AW is followed by its W beat with the latched data (r_drain_w), and the B response is taken and discarded (r_drain_b, which alone lifts b_ready outside WRITE_WAIT_RESP). The write that was already issued therefore lands. Until B is taken ctrlwr_engine_idle stays low and the request path stays closed. Before this the reset abandoned whatever phase was in flight and left a dangling response on the channel’s ID.

ctrlwr_engine_idle asserts only in WRITE_IDLE with no pending request, no active channel reset and nothing left to drain.

## 14.6	Contrast with the Control-Read Engine

The Control-Write Engine is the producer counterpart to the Control-Read Engine (Section 2.8). Both are per-channel control-descriptor executors, but they implement opposite half of the producer/consumer handshake:

| Aspect | Control-Read Engine (2.8) | Control-Write Engine (2.9) |
| --- | --- | --- |
| Opcode | DESC_OP_CTRL_READ (2'b01) | DESC_OP_CTRL_WRITE (2'b10) |
| Role | Consumer gate | Producer doorbell |
| AXI direction | Read (poll) | Single-beat write |
| Semantics | Poll until (rd & mask) == expected | Write wr_data to wr_addr, then continue |
| Retry budget | max_try retry budget (config default) | None (single unconditional write) |
| Chain effect | Holds off the chain until the gate passes | Releases the chain after the write completes |
| Descriptor fields | poll_addr / expected / mask / max_try | wr_addr / wr_data |

A CTRL_WRITE therefore has no maximum-try configuration: it is one unconditional 32-bit write, whereas CTRL_READ polls with a bounded retry budget before either passing the gate or timing out.

Last Updated: 2026-07-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Beats Scheduler Group Specification

Module: scheduler_group_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 15.1	Overview

The Beats Scheduler Group wraps a single channel’s scheduler and descriptor engine together with MonBus aggregation. It provides a clean integration point for the scheduler_group_array.

### 15.1.1	Key Features

- Single Channel Integration: One scheduler + one descriptor engine

- MonBus Aggregation: Combines scheduler and descriptor engine MonBus outputs

- Configuration Pass-Through: Routes configuration to both sub-modules

- Status Aggregation: Combined idle and error status

### 15.1.2	Block Diagram

### 15.1.3	Figure 3.1.1: Beats Scheduler Group Block Diagram

Scheduler group block diagram

Source: 10_scheduler_group.dot

## 15.2	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width

// Monitor Bus Parameters
parameter int DESC_MON_AGENT_ID = 16 + CHANNEL_ID;  // Descriptor engine agent
parameter int SCHED_MON_AGENT_ID = 48 + CHANNEL_ID; // Scheduler agent
parameter int MON_UNIT_ID = 1;

: Table 3.1.1: Beats Scheduler Group Parameters

## 15.3	Port List

### 15.3.1	Clock and Reset

Table 3.1.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 15.3.2	APB Programming Interface

Table 3.1.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Channel kick-off |
| apb_ready | output | 1 | Ready for kick-off |
| apb_addr | input | AW | First descriptor address |

### 15.3.3	Configuration Interface

Table 3.1.4: Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable this channel |
| cfg_channel_reset | input | 1 | Per-channel soft reset |
| cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window (cycles) |
| cfg_sched_timeout_limit | input | 8 | Consecutive-timeout windows before fatal escalation (0 = never) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_sched_err_enable | input | 1 | Enable error reporting |
| cfg_sched_compl_enable | input | 1 | SCHED_CONFIG.COMPL_EN: 1 = the scheduler’s and descriptor engine’s CORE Completion packets reach the group’s monitor bus; 0 = the group drops them at its arbiter inputs and acknowledges the emitters, so nothing stalls (rapids ISSUE-005). Errors are never gated. |
| cfg_sched_perf_enable | input | 1 | Enable performance monitoring |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch chaining |
| cfg_desceng_fifo_thresh | input | 4 | Prefetch threshold (descriptors buffered ahead) |
| cfg_desceng_addr0_base | input | ADDR_WIDTH | Valid address range 0 base |
| cfg_desceng_addr0_limit | input | ADDR_WIDTH | Valid address range 0 limit |
| cfg_desceng_addr1_base | input | ADDR_WIDTH | Valid address range 1 base |
| cfg_desceng_addr1_limit | input | ADDR_WIDTH | Valid address range 1 limit |
| cfg_ctrlrd_max_try | input | 9 | ctrlrd poll retry budget (0-511) |
| tick_1us | input | 1 | 1 us tick for ctrlrd retry spacing |

### 15.3.4	Descriptor AXI Master Interface

Table 3.1.5: Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_ar_valid | output | 1 | AR valid |
| desc_ar_ready | input | 1 | AR ready |
| desc_ar_addr | output | ADDR_WIDTH | Descriptor fetch address |
| desc_ar_len | output | 8 | Burst length - 1 |
| desc_ar_size | output | 3 | Burst size (log2 bytes) |
| desc_ar_burst | output | 2 | Burst type |
| desc_ar_id | output | AXI_ID_WIDTH | Transaction ID |
| desc_ar_lock | output | 1 | Lock type |
| desc_ar_cache | output | 4 | Cache attributes |
| desc_ar_prot | output | 3 | Protection attributes |
| desc_ar_qos | output | 4 | Quality of service |
| desc_ar_region | output | 4 | Region identifier |
| desc_r_valid | input | 1 | R valid |
| desc_r_ready | output | 1 | R ready |
| desc_r_data | input | 256 | Descriptor payload (fixed 256-bit) |
| desc_r_resp | input | 2 | Read response |
| desc_r_last | input | 1 | Last beat |
| desc_r_id | input | AXI_ID_WIDTH | Response ID |

### 15.3.5	Scheduler Data Interfaces

Table 3.1.6: Scheduler Data Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Read request |
| sched_rd_addr | output | AW | Read address |
| sched_rd_beats | output | 32 | Read beats |
| sched_rd_done_strobe | input | 1 | Read complete |
| sched_wr_valid | output | 1 | Write request |
| sched_wr_addr | output | AW | Write address |
| sched_wr_beats | output | 32 | Write beats |
| sched_wr_done_strobe | input | 1 | Write AW-issue strobe |
| sched_wr_commit_strobe | input | 1 | Write COMMIT strobe (B response) |
| sched_wr_commit_beats | input | 32 | Beats committed this strobe |

### 15.3.6	Status

Table 3.1.7: Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| scheduler_idle | output | 1 | Scheduler idle |
| descriptor_engine_idle | output | 1 | Descriptor engine idle |
| scheduler_state | output | 7 | FSM state |
| sched_error | output | 1 | Error flag |

### 15.3.7	MonBus Interface

Table 3.1.8: MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | monbus_timestamp_t | Shared monitor timebase |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Consumer ready |
| mon_packet | output | monitor_packet_t | Monitor packet |
| mon_timestamp | output | monbus_timestamp_t | Packet timestamp |

## 15.4	Internal Architecture

The descriptor engine hands each fetched descriptor to the scheduler over a valid/ready pair, and the four MonBus emitters (descriptor engine, scheduler, ctrlrd engine, ctrlwr engine) merge in the group’s 4:1 monbus_arbiter; both are drawn in Figure 3.1.1.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Beats Scheduler Group Array Specification

Module: scheduler_group_array_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 16.1	Overview

The Beats Scheduler Group Array instantiates 8 scheduler groups with a shared descriptor AXI master and unified MonBus output. It provides the complete scheduler infrastructure for all RAPIDS channels.

### 16.1.1	Key Features

- 8-Channel Array: One scheduler_group per channel

- Shared Descriptor AXI: Round-robin arbitration for descriptor fetches

- Unified MonBus: Aggregates 9 MonBus sources (8 scheduler + 1 arbiter)

- Per-Channel Configuration: Individual enable/reset per channel

- Aggregate Status: Combined idle and error flags

### 16.1.2	Block Diagram

### 16.1.3	Figure 3.2.1: Beats Scheduler Group Array Block Diagram

Scheduler group array block diagram

Source: 11_scheduler_group_array.dot

## 16.2	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of channels
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width
parameter int DESC_DATA_WIDTH = 256;             // Descriptor width

// AXI ID Management
parameter int AXI_ID_WIDTH = 8;                  // ID field width
parameter int MAX_OUTSTANDING = 8;               // Outstanding descriptors

// Monitor Bus Parameters
parameter int MON_UNIT_ID = 1;                   // Unit ID for MonBus

: Table 3.2.1: Beats Scheduler Group Array Parameters

## 16.3	Port List

### 16.3.1	Clock and Reset

Table 3.2.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 16.3.2	Per-Channel APB Programming

Table 3.2.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | NC | Channel kick-off (per-channel) |
| apb_ready | output | NC | Ready for kick-off |
| apb_addr | input | NC*AW | First descriptor address |

### 16.3.3	Per-Channel Configuration

Table 3.2.4: Per-Channel Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | NC | Enable per channel |
| cfg_channel_reset | input | NC | Soft reset per channel |
| cfg_sched_timeout_cycles | input | NC*16 | Timeout threshold |
| cfg_sched_timeout_enable | input | NC | Enable timeout |
| cfg_desceng_enable | input | NC | Enable descriptor engine |
| cfg_desceng_prefetch | input | NC | Enable prefetching |
| cfg_desceng_fifo_thresh | input | NC*4 | Prefetch threshold |
| cfg_desceng_addr0_base | input | NC*AW | Address range 0 base |
| cfg_desceng_addr0_limit | input | NC*AW | Address range 0 limit |

### 16.3.4	Shared Descriptor AXI Master Interface

Table 3.2.5: Shared Descriptor AXI Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_axi_arvalid | output | 1 | AR valid |
| desc_axi_arready | input | 1 | AR ready |
| desc_axi_araddr | output | ADDR_WIDTH | Descriptor fetch address |
| desc_axi_arlen | output | 8 | Burst length - 1 |
| desc_axi_arsize | output | 3 | Burst size |
| desc_axi_arburst | output | 2 | Burst type |
| desc_axi_arid | output | AXI_ID_WIDTH | Transaction ID |
| desc_axi_arlock | output | 1 | Lock type |
| desc_axi_arcache | output | 4 | Cache attributes |
| desc_axi_arprot | output | 3 | Protection attributes |
| desc_axi_arqos | output | 4 | Quality of service |
| desc_axi_arregion | output | 4 | Region identifier |
| desc_axi_rvalid | input | 1 | R valid |
| desc_axi_rready | output | 1 | R ready |
| desc_axi_rdata | input | 256 | Descriptor payload (fixed 256-bit) |
| desc_axi_rresp | input | 2 | Read response |
| desc_axi_rlast | input | 1 | Last beat |
| desc_axi_rid | input | AXI_ID_WIDTH | Response ID |

### 16.3.5	Per-Channel Scheduler Data Interfaces

Table 3.2.6: Per-Channel Scheduler Data Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | NC | Read request per channel |
| sched_rd_addr | output | NC*AW | Read address |
| sched_rd_beats | output | NC*32 | Read beats |
| sched_rd_done_strobe | input | NC | Read complete |
| sched_wr_valid | output | NC | Write request per channel |
| sched_wr_addr | output | NC*AW | Write address |
| sched_wr_beats | output | NC*32 | Write beats |
| sched_wr_done_strobe | input | NC | Write complete |

### 16.3.6	Aggregate Status

Table 3.2.7: Aggregate Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle | output | NC | Per-channel descriptor engine idle |
| scheduler_idle | output | NC | Per-channel scheduler idle |
| scheduler_state | output | NC x 7 | Per-channel FSM state (one-hot) |
| sched_error | output | NC | Per-channel scheduler error (sticky) |

### 16.3.7	Unified MonBus Interface

Table 3.2.8: Unified MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i_mon_time | input | monbus_timestamp_t | Shared monitor timebase |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Consumer ready |
| mon_packet | output | monitor_packet_t | Monitor packet |
| mon_timestamp | output | monbus_timestamp_t | Packet timestamp |

### 16.3.8	Descriptor-AXI Perf Window

The descriptor monitor is axi_monitor_lite, which has no performance cone. The DAXMON_PERF_* CSRs are fed instead by an always-on axi_bus_meter on the descriptor R channel plus window/beat/byte/burst accumulators beside it, the same meter rapids_beats_top uses for RDMON/WRMON. cfg_desc_mon_perf_run (DAXMON_PERF_CTRL.RUN) opens the window: counters clear on its rising edge, count while it is high and hold while it is low. The meter is not gated by USE_AXI_MONITORS.

Table 3.2.9: Descriptor-AXI Perf Window

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desc_mon_perf_run | input | 1 | Window control (DAXMON_PERF_CTRL.RUN) |
| sts_desc_mon_win_active | output | 1 | Window open (equals RUN) |
| sts_desc_mon_win_cycles | output | 32 | Live cycle count; reads 0 while the window is closed |
| sts_desc_mon_prod_cycles | output | 32 | R valid and ready |
| sts_desc_mon_bp_cycles | output | 32 | R valid, not ready (backpressure) |
| sts_desc_mon_starv_cycles | output | 32 | R ready, not valid (starvation) |
| sts_desc_mon_idle_cycles | output | 32 | R neither valid nor ready |
| sts_desc_mon_beat_count | output | 32 | R handshakes in the window |
| sts_desc_mon_byte_count | output | 64 | 32 bytes per 256-bit descriptor beat |
| sts_desc_mon_burst_count | output | 32 | AR handshakes in the window |

## 16.4	AXI Arbitration

### 16.4.1	Figure 3.2.2: Descriptor AXI Arbitration Sequence

Two channels contending for the descriptor master; responses return by RID.

Diagram 5

## 16.5	MonBus Aggregation

The unified MonBus output aggregates 9 sources using round-robin arbitration:

Table 3.2.10: MonBus Source Assignment

| Source ID | Origin | Description |
| --- | --- | --- |
| 0-7 | scheduler_group[0:7] | Per-channel aggregated MonBus |
| 8 | Descriptor AXI monitor | axi4_master_rd_monlite packets (USE_AXI_MONITORS = 1) |

Each scheduler_group provides a single MonBus output that combines its descriptor engine, scheduler, ctrlrd engine and ctrlwr engine packets (4:1 internal arbitration).

## 16.6	Integration Context

scheduler_group_array_beats #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512)
) u_scheduler_array (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // APB programming (per-channel)
    .apb_valid              (apb_kick_valid),
    .apb_ready              (apb_kick_ready),
    .apb_addr               (apb_kick_addr),

    // Configuration (per-channel)
    .cfg_channel_enable     (cfg_ch_enable),
    .cfg_sched_timeout_cycles(cfg_timeout_cycles),
    // ... additional config ...

    // Shared descriptor AXI
    .desc_axi_arvalid       (desc_axi_arvalid),
    .desc_axi_arready       (desc_axi_arready),
    .desc_axi_araddr        (desc_axi_araddr),
    .desc_axi_rvalid        (desc_axi_rvalid),
    .desc_axi_rready        (desc_axi_rready),
    .desc_axi_rdata         (desc_axi_rdata),

    // Per-channel scheduler outputs
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_done_strobe   (sched_rd_done_strobe),
    .sched_wr_valid         (sched_wr_valid),
    .sched_wr_addr          (sched_wr_addr),
    .sched_wr_beats         (sched_wr_beats),
    .sched_wr_done_strobe   (sched_wr_done_strobe),

    // Status
    .scheduler_idle         (all_schedulers_idle),

    // Unified MonBus
    .mon_valid              (sched_array_monbus_valid),
    .mon_ready              (sched_array_monbus_ready),
    .mon_packet             (sched_array_monbus_data)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Sink Data Path Specification

Module: snk_data_path_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 17.1	Overview

The Sink Data Path integrates the SRAM controller and AXI write engine for network-to-memory transfers. Data enters through the fill interface, is buffered in SRAM, and is written to system memory via AXI4.

### 17.1.1	Key Features

- 8-Channel SRAM Controller: Per-channel buffering with flow control

- AXI Write Engine: Streaming burst writes to system memory

- Beat-Level Tracking: Precise flow control at beat granularity

- Scheduler Integration: Receives transfer requests from scheduler

### 17.1.2	Block Diagram

### 17.1.3	Figure 3.3.1: Sink Data Path Block Diagram

Sink data path block diagram

Source: 13_sink_data_path.dot

## 17.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int AW_MAX_OUTSTANDING = 8;
parameter int W_PHASE_FIFO_DEPTH = 64;
parameter int B_PHASE_FIFO_DEPTH = 16;

: Table 3.3.1: Sink Data Path Parameters

## 17.3	Data Flow

### 17.3.1	Figure 3.3.2: Sink Path Data Flow Sequence

1. Fill Request
   - snk_fill_alloc_req = 1
   - snk_fill_alloc_size = N (beats to allocate)
   - snk_fill_alloc_id = channel
        |
        v
2. Space Check (stream_alloc_ctrl inside STREAM's sram_controller)
   - Check snk_fill_space_free[channel] >= N
   - Allocate N beats of space
        |
        v
3. Fill Data Transfer
   - snk_fill_valid = 1
   - snk_fill_data = data beats
   - snk_fill_id = channel
        |
        v
4. SRAM Write
   - Data written to channel's SRAM partition
   - stream_drain_ctrl notified of data availability
        |
        v
5. AXI Write Request
   - Scheduler requests write: sched_wr_valid
   - Address and beat count provided
        |
        v
6. SRAM Read + AXI Write
   - axi_write_engine reads from SRAM
   - Issues AXI bursts to memory
        |
        v
7. Completion
   - B channel response received
   - sched_wr_done_strobe = 1

## 17.4	Timing Diagram

### 17.4.1	Figure 3.3.3: Sink Path Transfer Timing

Three windows from one transfer: the allocation and fill, the first AXI write burst being issued, and the end of that burst with its response.

(a) Allocation and fill

snk_data_path_beats - allocation and fill

(b) First AXI write burst issued

snk_data_path_beats - first AXI write burst issued

(c) Last W beats and the B response

snk_data_path_beats - last W beats and the B response

Source: snk_data_path_fill_alloc.json, snk_data_path_axi_write_aw.json, snk_data_path_axi_write_b.json, captured inside rapids_core_beats.u_snk.u_sink_data_path from dv/tests/top_beats/test_rapids_core_beats.py (sink path, channel 0, 32 beats, 512-bit data, cfg_alloc_size = 16, TEST_LEVEL=full) with WAVES=1.

Reading (a): the first beat for channel 0 arrives with no space allocated, so the path raises fill_alloc_req for 16 beats; fill_space_free[0] drops 512 to 496 three cycles later and the beat is accepted on the same cycle as the request. The remaining beats are accepted one per handshake (fill_ready stays high; the gaps are the AXIS producer’s pacing). Reading (b): once the descriptor arrives the scheduler holds sched_wr_valid with 32 beats remaining; the engine issues the AW (awlen = 8, 9 beats) and, the cycle after the AW handshake, pulses sched_wr_done_strobe with beats_done = 9, on which the scheduler advances to 23 remaining and the next AW address is already 0x20000240. Reading (c): W beats drain at the memory’s pace (wready from the AXI slave BFM), wlast marks the ninth beat, and the B response arrives two cycles later with bresp = OKAY. The done strobe is reported at issue, not at B; completion of the whole transfer is what snk_system_idle reflects.

## 17.5	Interface Summary

### 17.5.1	Fill Interface (Input)

Table 3.3.2: Fill Interface

| Signal | Direction | Description |
| --- | --- | --- |
| fill_alloc_req | input | Allocation request (single, ID-selected) |
| fill_alloc_size | input | Beats to allocate |
| fill_alloc_id | input | Transaction ID selects the channel |
| fill_space_free | output | Available space per channel |
| fill_valid | input | Fill data valid |
| fill_ready | output | Ready for fill data |
| fill_id | input | Transaction ID selects the channel |
| fill_data | input | Fill data |

### 17.5.2	Scheduler Interface

Table 3.3.3: Scheduler Interface

| Signal | Direction | Description |
| --- | --- | --- |
| sched_wr_valid | input | Channel requests write |
| sched_wr_ready | output | Path ready for channel |
| sched_wr_addr | input | Destination addresses |
| sched_wr_beats | input | Beats remaining to write |
| sched_wr_burst_len | input | Requested burst length |
| sched_wr_done_strobe | output | Burst ISSUED on AW handshake |
| sched_wr_beats_done | output | Beats issued in burst |
| sched_wr_commit_strobe | output | Burst COMMITTED on B response |
| sched_wr_commit_beats | output | Beats committed in burst |
| sched_wr_error | output | Sticky error flag per channel |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Sink Data Path AXIS Wrapper Specification

Module: snk_data_path_axis_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2026-09-29

## 18.1	Overview

The Sink Data Path AXIS wrapper adds an AXI-Stream slave interface to the sink data path, enabling standard AXIS ingress for network-to-memory transfers.

### 18.1.1	Key Features

- AXI-Stream Slave Interface: Standard AXIS for data ingress

- Per-Channel TID Mapping: AXIS TID maps to fill channel ID

- TLAST to Last Mapping: Packet boundary tracking

- Core Sink Integration: Wraps sink_data_path module

### 18.1.2	Block Diagram

### 18.1.3	Figure 3.4.1: Sink Data Path AXIS Block Diagram

Sink data path AXIS block diagram

Source: 14_sink_data_path_axis.dot

## 18.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int TID_WIDTH = 3;                     // log2(NUM_CHANNELS)
parameter int TDEST_WIDTH = 1;
parameter int TUSER_WIDTH = 1;

: Table 3.4.1: Sink Data Path AXIS Parameters

## 18.3	Port List

### 18.3.1	Clock and Reset

Table 3.4.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 18.3.2	AXI-Stream Slave Interface

Table 3.4.3: AXI-Stream Slave Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tvalid | input | 1 | Data valid |
| s_axis_tready | output | 1 | Ready for data |
| s_axis_tdata | input | DW | Data payload |
| s_axis_tstrb | input | SW | Byte strobes |
| s_axis_tlast | input | 1 | Last beat of packet |
| s_axis_tid | input | AXIS_ID_WIDTH | Stream ID (channel) |
| s_axis_tdest | input | AXIS_DEST_WIDTH | Destination |
| s_axis_tuser | input | AXIS_USER_WIDTH | User sideband |

### 18.3.3	Allocation Configuration

Table 3.4.4: Allocation Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_alloc_size | input | 8 | Default allocation size per request (beats) |

### 18.3.4	Scheduler Interface

Table 3.4.5: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | input | NC | Channel requests write |
| sched_wr_ready | output | NC | Path ready for channel |
| sched_wr_addr | input | NC x AW | Destination addresses |
| sched_wr_beats | input | NC x 32 | Beats remaining to write |
| sched_wr_burst_len | input | NC x 8 | Requested burst length |
| sched_wr_done_strobe | output | NC | Burst ISSUED on AW handshake |
| sched_wr_beats_done | output | NC x 32 | Beats issued in burst |
| sched_wr_commit_strobe | output | NC | Burst COMMITTED on B response |
| sched_wr_commit_beats | output | NC x 32 | Beats committed in burst |
| sched_wr_error | output | NC | Sticky error flag per channel |

### 18.3.5	AXI Write Master Interface

Table 3.4.6: AXI Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awvalid | output | 1 | AW channel valid |
| m_axi_awready | input | 1 | AW channel ready |
| m_axi_awaddr | output | AW | Write address |
| m_axi_awlen | output | 8 | Burst length |
| m_axi_awsize | output | 3 | Burst size |
| m_axi_awburst | output | 2 | Burst type |
| m_axi_awid | output | ID_W | Transaction ID |
| m_axi_wvalid | output | 1 | W channel valid |
| m_axi_wready | input | 1 | W channel ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat |
| m_axi_bvalid | input | 1 | B channel valid |
| m_axi_bready | output | 1 | B channel ready |
| m_axi_bresp | input | 2 | Write response |
| m_axi_bid | input | ID_W | Response ID |

## 18.4	Signal Mapping

### 18.4.1	Figure 3.4.2: AXIS to Fill Interface Mapping

AXIS to fill mapping

Source: 14b_axis_to_fill.dot

## 18.5	Timing Diagram

### 18.5.1	Figure 3.4.3: AXIS Ingress Timing

snk_data_path_axis_beats - AXIS ingress, end of a 32-beat packet

Source: snk_data_path_axis_ingress.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (sink path, channel 0, 32 beats, 512-bit data, TEST_LEVEL=full) with WAVES=1.

Reading it: each AXIS beat is passed straight through to the fill interface in the same cycle – fill_valid mirrors s_axis_tvalid, fill_id is s_axis_tid, and fill_data is s_axis_tdata (the low 16 bits are shown; the payload counts up 0x1a..0x1f). s_axis_tready is high while channel 0 has allocated space and drops the cycle after s_axis_tlast because the packet count is now complete and the next beat would need a fresh allocation (Figure 3.3.3a).

## 18.6	Usage Notes

- TID Usage: AXIS TID directly maps to channel ID for multi-channel routing

- Allocation Required: Space must be allocated via snk_alloc_* before AXIS data arrives

- Backpressure: s_axis_tready deasserts when SRAM space exhausted

- Packet Boundaries: TLAST marks end of AXIS packets, mapped to fill_last

- Partial segments (rapids BUG-009, 2026-09-29): allocation is in cfg_alloc_size segments, and a packet that ends mid-segment leaves the rest of its segment allocated for the channel’s next packet (r_pending_alloc). That shifts the allocation phase: with whole-segment-only allocation the last segment - residue slots of the buffer could never be allocated again until something drained, and a write burst equal to the buffer depth – which needs every slot – waited forever (Genesys 2: a 4-beat run, then a 128-beat-burst run accepted 124 beats and issued no AW). The ingress now allocates the remainder when less than a segment is free (fill_alloc_size = min(cfg_alloc_size, space_free)), so the buffer can always be filled to the top. Regression: test_residue_full_burst.

- Allocation settle and no dropped beats (same bug, third finding): fill_space_free reflects an allocation three cycles after its handshake. A one- or two-beat partial allocation is consumed before that, and a second allocation against the stale view over-allocated the buffer (the 8-bit space count wrapped to 254 and the ingress ran into a full FIFO). Each channel now holds its next allocation for three cycles after the last (r_alloc_settle), and s_axis_tready includes fill_ready on allocation cycles as well, so a beat is never accepted that the FIFO cannot store.

Last Updated: 2026-09-29

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Sink SRAM Controller Specification

Module: snk_sram_controller_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 19.1	Overview

The Sink SRAM Controller is a naming wrapper, with no logic of its own, around STREAM’s sram_controller – the one per-channel SRAM implementation both DMAs share since bdf4e0dff. It maps the RAPIDS fill_*/drain_* names onto STREAM’s ports; the eight per-channel units (allocation counter, FIFO, drain counter, latency bridge) live inside STREAM’s block. It manages 8 channel units, providing per-channel buffering with channel arbitration for the shared AXI write engine.

### 19.1.1	Key Features

- One shared implementation: instantiates STREAM’s sram_controller once; it holds 8 sram_controller_units, each a gaxi_fifo_sync FIFO with stream_alloc_ctrl, stream_drain_ctrl and stream_latency_bridge

- Channel Arbitration: none inside the controller – the AXI write engine picks a channel and presents it on drain_id

- Flow Control Integration: per-channel stream_alloc_ctrl / stream_drain_ctrl virtual FIFOs; the drain counter’s virtual depth is 2 x SRAM_DEPTH (stream BUG-011)

- Space Tracking: Reports available space per channel

### 19.1.2	Block Diagram

### 19.1.3	Figure 3.5.1: Sink SRAM Controller Block Diagram

Sink SRAM controller block diagram

Source: 15_snk_sram_controller.dot

## 19.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;                  // Per-channel depth
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);

: Table 3.5.1: Sink SRAM Controller Parameters

## 19.3	Port List

### 19.3.1	Clock and Reset

Table 3.5.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 19.3.2	Fill Interface (ID-Selected)

Table 3.5.3: Fill Interface (ID-Selected)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Allocation request (single, ID-selected) |
| fill_alloc_size | input | 8 | Beats to allocate |
| fill_alloc_id | input | CIW | Transaction ID selects the channel |
| fill_space_free | output | NC x SCW | Available space per channel |
| fill_valid | input | 1 | Fill data valid |
| fill_ready | output | 1 | Ready for fill data |
| fill_id | input | CIW | Transaction ID selects the channel |
| fill_data | input | DW | Fill data |

### 19.3.3	Arbitrated Drain Interface (to AXI Write Engine)

Table 3.5.4: Arbitrated Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_data_avail | output | NC x SCW | Data available per channel |
| drain_req | input | NC | Drain reservation request per channel |
| drain_size | input | NC x 8 | Beats to reserve |
| drain_valid | output | NC | Drain valid, registered (for arbitration) |
| drain_valid_comb | output | NC | Drain valid, combinational (gates m_axi_wvalid) |
| drain_read | input | 1 | Consumer read strobe |
| drain_id | input | CIW | Channel ID select for drain |
| drain_data | output | DW | Drain data (muxed from selected channel) |

### 19.3.4	Debug Interface

Table 3.5.5: Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_bridge_pending | output | NC | Bridge has a pending beat per channel |
| dbg_bridge_out_valid | output | NC | Bridge output valid per channel |

## 19.4	Drain Selection

There is no arbiter inside the controller. Every channel unit raises its own drain_valid[ch] / drain_size[ch] when it has data; the AXI write engine picks a channel, drives it on drain_id, and the controller steers that channel’s beats to the single drain_* port. Switching channels costs nothing beyond the consumer’s own decision, and a channel keeps presenting drain_valid until it is empty.

### 19.4.1	Figure 3.5.2: Drain Selection by the Consumer

snk_sram_controller_beats - consumer drains eight channels in turn

Source: snk_sram_controller_drain_select.json, captured from dv/tests/macro_beats/test_snk_sram_controller_beats.py (multi_channel, 8 channels, 512-bit data, 512-entry SRAM, REG_LEVEL=GATE) with WAVES=1.

Reading it: all eight channels have data (drain_valid = 0xff) and the consumer walks them in turn. It raises one bit of drain_req at a time; the controller answers with drain_id naming that channel and drain_data presenting its head word on the same cycle, and the consumer’s drain_read pulse pops it. Each channel’s turn lasts nine cycles here because the test releases the request and re-arms the next one; the controller itself needs no gap between channels.

## 19.5	Integration with AXI Write Engine

snk_sram_controller_beats #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_snk_sram_ctrl (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Per-channel fill interface
    .fill_alloc_req         (fill_alloc_req),
    .fill_alloc_size        (fill_alloc_size),
    .fill_space_free        (fill_space_free),
    .fill_valid             (fill_valid),
    .fill_ready             (fill_ready),
    .fill_data              (fill_data),

    // Arbitrated drain interface
    .drain_req              (sram_drain_req),
    .drain_id               (sram_drain_id),
    .drain_size             (sram_drain_beats),

    // Status
    .drain_data_avail       (channel_data_avail),
    .dbg_bridge_pending     (channel_empty),
    .dbg_bridge_out_valid   (channel_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Source Data Path Specification

Module: src_data_path_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 20.1	Overview

The Source Data Path integrates the AXI read engine and SRAM controller for memory-to-network transfers. Data is read from system memory via AXI4, buffered in SRAM, and delivered through the drain interface to the network.

### 20.1.1	Key Features

- AXI Read Engine: Streaming burst reads from system memory

- 8-Channel SRAM Controller: Per-channel buffering with flow control

- Beat-Level Tracking: Precise flow control at beat granularity

- Scheduler Integration: Receives transfer requests from scheduler

### 20.1.2	Block Diagram

### 20.1.3	Figure 3.6.1: Source Data Path Block Diagram

Source data path block diagram

Source: 16_source_data_path.dot

## 20.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int AR_MAX_OUTSTANDING = 8;
parameter int R_PHASE_FIFO_DEPTH = 64;

: Table 3.6.1: Source Data Path Parameters

## 20.3	Data Flow

### 20.3.1	Figure 3.6.2: Source Path Data Flow Sequence

1. Scheduler Read Request
   - sched_rd_valid = 1
   - sched_rd_addr = source address
   - sched_rd_beats = N
   - sched_rd_id = channel
        |
        v
2. Space Check (stream_alloc_ctrl inside STREAM's sram_controller)
   - Check src_drain_space_free[channel] >= N
   - Allocate N beats of space
        |
        v
3. AXI Read Transaction
   - axi_read_engine issues AR
   - m_axi_araddr = sched_rd_addr
   - m_axi_arlen = calculated from beats
        |
        v
4. R Channel Data
   - Data arrives on m_axi_rdata
   - Written to channel's SRAM partition
        |
        v
5. SRAM Write Complete
   - stream_drain_ctrl notified via the unit's latency bridge
   - Data available for drain
        |
        v
6. Drain Interface
   - src_drain_valid = 1
   - Data delivered to network master
        |
        v
7. Completion
   - sched_rd_done_strobe = 1
   - Scheduler notified of completion

## 20.4	Port List

### 20.4.1	Clock and Reset

Table 3.6.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 20.4.2	Configuration

Table 3.6.3: Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | AXI read transfer size in beats (all channels) |

### 20.4.3	Scheduler Interface

Table 3.6.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | NC | Channel requests read |
| sched_rd_addr | input | NC x AW | Source addresses |
| sched_rd_beats | input | NC x 32 | Beats remaining to read |
| sched_rd_done_strobe | output | NC | Burst completed (pulsed 1 cycle) |
| sched_rd_beats_done | output | NC x 32 | Beats completed in burst |
| sched_rd_error | output | NC | Sticky error flag per channel |

### 20.4.4	AXI Read Master Interface

Table 3.6.5: AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rlast | input | 1 | Last beat |

### 20.4.5	Drain Interface (Output to Network)

Table 3.6.6: Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_data_avail | output | NC x SCW | Data available per channel |
| drain_req | input | NC | Drain reservation request per channel |
| drain_size | input | NC x 8 | Beats to reserve |
| drain_valid | output | NC | Drain data valid per channel |
| drain_valid_comb | output | NC | Combinational per-channel valid (beat gate) |
| drain_read | input | 1 | Consumer read strobe |
| drain_id | input | CIW | Channel ID select for drain |
| drain_data | output | DW | Drain data (muxed from selected channel) |

## 20.5	Timing Diagram

### 20.5.1	Figure 3.6.3: Source Path Transfer Timing

src_data_path_beats - 4-beat transfer, command to done

Source: src_data_path_transfer.json, captured inside rapids_core_beats.u_src.u_source_data_path, captured from dv/tests/top_beats/test_rapids_core_beats.py (source path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1.

Reading it: the scheduler holds sched_rd_valid[0] with 4 beats at 0x10000000. The read engine issues the AR (arlen = 3) two cycles later; the four R beats land from cycle 8, rlast on the fourth, and the cycle after it the engine pulses sched_rd_done_strobe[0] with beats_done = 4 – the scheduler advances its address to 0x10000100 on that strobe. The data reaches the source SRAM two cycles after the last R beat (drain_valid[0] rises) and the AXIS side drains it one word per drain_read pulse at the pace the egress consumer allows.

## 20.6	Integration Example

src_data_path_beats #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_source_path (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Scheduler interface
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_done_strobe   (sched_rd_done_strobe),
    .sched_rd_beats_done    (sched_rd_beats_done),
    .sched_rd_error         (sched_rd_error),

    // AXI read master
    .m_axi_arvalid          (src_axi_arvalid),
    .m_axi_arready          (src_axi_arready),
    .m_axi_araddr           (src_axi_araddr),
    .m_axi_arlen            (src_axi_arlen),
    .m_axi_rvalid           (src_axi_rvalid),
    .m_axi_rready           (src_axi_rready),
    .m_axi_rdata            (src_axi_rdata),
    .m_axi_rlast            (src_axi_rlast),

    // Drain interface
    .drain_valid            (src_drain_valid),
    .drain_data             (src_drain_data),
    .drain_id               (src_drain_id),
    .drain_data_avail       (src_data_avail)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Source Data Path AXIS Wrapper Specification

Module: src_data_path_axis_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 21.1	Overview

The Source Data Path AXIS wrapper adds an AXI-Stream master interface to the source data path, enabling standard AXIS egress for memory-to-network transfers.

### 21.1.1	Key Features

- AXI-Stream Master Interface: Standard AXIS for data egress

- Per-Channel TID Mapping: Drain channel ID maps to AXIS TID

- TLAST Generation: Packet boundary marking

- Core Source Integration: Wraps source_data_path module

### 21.1.2	Block Diagram

### 21.1.3	Figure 3.7.1: Source Data Path AXIS Block Diagram

Source data path AXIS block diagram

Source: 17_source_data_path_axis.dot

## 21.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;
parameter int TID_WIDTH = 3;                     // log2(NUM_CHANNELS)
parameter int TDEST_WIDTH = 1;
parameter int TUSER_WIDTH = 1;

: Table 3.7.1: Source Data Path AXIS Parameters

## 21.3	Port List

### 21.3.1	Clock and Reset

Table 3.7.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 21.3.2	Configuration

Table 3.7.3: Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | AXI read transfer size in beats (all channels) |
| cfg_drain_size | input | 8 | Beats to drain per request |

### 21.3.3	Scheduler Interface

Table 3.7.4: Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | input | NC | Channel requests read |
| sched_rd_addr | input | NC x AW | Source addresses |
| sched_rd_beats | input | NC x 32 | Beats remaining to read |
| sched_rd_done_strobe | output | NC | Burst completed (pulsed 1 cycle) |
| sched_rd_beats_done | output | NC x 32 | Beats completed in burst |
| sched_rd_error | output | NC | Sticky error flag per channel |

### 21.3.4	AXI Read Master Interface

Table 3.7.5: AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arlen | output | 8 | Burst length |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rlast | input | 1 | Last beat |

### 21.3.5	AXI-Stream Master Interface

Table 3.7.6: AXI-Stream Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tvalid | output | 1 | Data valid |
| m_axis_tready | input | 1 | Consumer ready |
| m_axis_tdata | output | DW | Data payload |
| m_axis_tstrb | output | SW | Byte strobes |
| m_axis_tlast | output | 1 | Last beat of packet |
| m_axis_tid | output | AXIS_ID_WIDTH | Stream ID (channel) |
| m_axis_tdest | output | AXIS_DEST_WIDTH | Destination |
| m_axis_tuser | output | AXIS_USER_WIDTH | User sideband |

## 21.4	Signal Mapping

### 21.4.1	Figure 3.7.2: Drain to AXIS Interface Mapping

Drain to AXIS mapping

Source: 17b_drain_to_axis.dot

## 21.5	Timing Diagram

### 21.5.1	Figure 3.7.3: AXIS Egress Timing

rapids_core_beats - source egress on m_axis_*, 4 beats

Source: src_data_path_axis_egress.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (source path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1; the consumer is the framework AXIS slave with its default ready pacing.

Reading it: m_axis_tvalid rises with the first word and stays high for the whole burst; each beat leaves on a tready pulse from the consumer (cycles 4, 12, 16 and 19), tdata advancing 0, 1, 2, 3 in the low bits and tid naming channel 0. tlast closes each drain request’s packet (cfg_drain_size beats at most); the words arrived from the read engine one at a time here, so every request carried a single word and every beat is marked last. src_system_idle is already back high: system_idle is the AND of the schedulers’ idle flags, and the source scheduler retires a descriptor on the read engine’s done strobe (Figure 3.6.3), before the words have drained to the AXIS side. The sink half is different – its scheduler waits for the write commits, so snk_system_idle follows the last B response.

## 21.6	Usage Notes

- TID Generation: Drain channel ID directly maps to AXIS TID

- TLAST Generation: Drain last signal maps to AXIS TLAST

- Backpressure: Network TREADY backpressure propagates to SRAM drain

- Packet Boundaries: TLAST marks end of AXIS packets

## 21.7	Integration Example

src_data_path_axis_beats #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .AXIS_ID_WIDTH (3)
) u_source_axis (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Scheduler interface
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_done_strobe   (sched_rd_done_strobe),

    // AXI read master
    .m_axi_arvalid          (src_axi_arvalid),
    .m_axi_arready          (src_axi_arready),
    .m_axi_araddr           (src_axi_araddr),
    .m_axi_rvalid           (src_axi_rvalid),
    .m_axi_rready           (src_axi_rready),
    .m_axi_rdata            (src_axi_rdata),

    // AXIS master
    .m_axis_tvalid          (network_tvalid),
    .m_axis_tready          (network_tready),
    .m_axis_tdata           (network_tdata),
    .m_axis_tlast           (network_tlast),
    .m_axis_tid             (network_tid)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Source SRAM Controller Specification

Module: src_sram_controller_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 22.1	Overview

The Source SRAM Controller is a naming wrapper, with no logic of its own, around STREAM’s sram_controller – the one per-channel SRAM implementation both DMAs share since bdf4e0dff. It maps the RAPIDS fill_*/drain_* names onto STREAM’s ports; the eight per-channel units (allocation counter, FIFO, drain counter, latency bridge) live inside STREAM’s block. It manages 8 channel units for the source data path, providing per-channel buffering with channel arbitration for data delivery to the network.

### 22.1.1	Key Features

- One shared implementation: instantiates STREAM’s sram_controller once; it holds 8 sram_controller_units, each a gaxi_fifo_sync FIFO with stream_alloc_ctrl, stream_drain_ctrl and stream_latency_bridge

- Channel Arbitration: none inside the controller – the AXIS egress arbiter picks a channel and presents it on drain_id

- Flow Control Integration: per-channel stream_alloc_ctrl / stream_drain_ctrl virtual FIFOs; the drain counter’s virtual depth is 2 x SRAM_DEPTH (stream BUG-011)

- Data Availability Tracking: Reports available data per channel

### 22.1.2	Block Diagram

### 22.1.3	Figure 3.8.1: Source SRAM Controller Block Diagram

Source SRAM controller block diagram

Source: 18_src_sram_controller.dot

## 22.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int DATA_WIDTH = 512;
parameter int SRAM_DEPTH = 512;                  // Per-channel depth
parameter int ADDR_WIDTH = $clog2(SRAM_DEPTH);

: Table 3.8.1: Source SRAM Controller Parameters

## 22.3	Port List

### 22.3.1	Clock and Reset

Table 3.8.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 22.3.2	Fill Interface (ID-Selected, from AXI Read Engine)

Table 3.8.3: Fill Interface (ID-Selected)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fill_alloc_req | input | 1 | Allocation request (single, ID-selected) |
| fill_alloc_size | input | 8 | Beats to allocate |
| fill_alloc_id | input | CIW | Transaction ID selects the channel |
| fill_space_free | output | NC x SCW | Available space per channel |
| fill_valid | input | 1 | Fill data valid |
| fill_ready | output | 1 | Ready for fill data |
| fill_id | input | CIW | Transaction ID selects the channel |
| fill_data | input | DW | Fill data |

### 22.3.3	Arbitrated Drain Interface (to Network)

Table 3.8.4: Arbitrated Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| drain_data_avail | output | NC x SCW | Data available per channel |
| drain_req | input | NC | Drain reservation request per channel |
| drain_size | input | NC x 8 | Beats to reserve |
| drain_valid | output | NC | Drain data valid per channel |
| drain_valid_comb | output | NC | Combinational per-channel valid (beat gate) |
| drain_read | input | 1 | Consumer read strobe |
| drain_id | input | CIW | Channel ID select for drain |
| drain_data | output | DW | Drain data (muxed from selected channel) |

### 22.3.4	Debug Interface

Table 3.8.5: Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_bridge_pending | output | NC | Bridge has a pending beat per channel |
| dbg_bridge_out_valid | output | NC | Bridge output valid per channel |

## 22.4	Drain Selection

There is no arbiter inside the controller. Every channel unit raises its own drain_valid[ch] / drain_size[ch] when it has data; the AXIS egress arbiter picks a channel, drives it on drain_id, and the controller steers that channel’s beats to the single drain_* port. Switching channels costs nothing beyond the consumer’s own decision, and a channel keeps presenting drain_valid until it is empty.

### 22.4.1	Figure 3.8.2: Drain Selection by the Consumer

src_sram_controller_beats - consumer drains eight channels in turn

Source: src_sram_controller_drain_select.json, captured from dv/tests/macro_beats/test_src_sram_controller_beats.py (multi_channel, 8 channels, 512-bit data, 512-entry SRAM, REG_LEVEL=GATE) with WAVES=1.

Reading it: the same selection contract as the sink controller (Figure 3.5.2). With drain_valid = 0xff the consumer asks for one channel at a time through drain_req; drain_id and drain_data follow the request on the same cycle and drain_read pops the word. The nine-cycle turn per channel is the test’s pacing, not the controller’s.

## 22.5	Comparison: Sink vs Source SRAM Controller

Table 3.8.6: Sink vs Source Controller Comparison

| Aspect | Sink Controller | Source Controller |
| --- | --- | --- |
| Fill Source | Network ingress | AXI read data |
| Drain Destination | AXI write engine | Network egress |
| Fill Interface | Per-channel from network | Per-channel from AXI R |
| Drain Interface | Arbitrated SRAM read | Arbitrated drain to network |
| Primary Use | Network -> Memory | Memory -> Network |

## 22.6	Integration Example

src_sram_controller_beats #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_src_sram_ctrl (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Per-channel fill interface (from AXI read engine)
    .fill_alloc_req         (fill_alloc_req),
    .fill_alloc_size        (fill_alloc_size),
    .fill_space_free        (fill_space_free),
    .fill_valid             (fill_valid),
    .fill_ready             (fill_ready),
    .fill_data              (fill_data),

    // Arbitrated drain interface (to network)
    .drain_valid            (src_drain_valid),
    .drain_data             (src_drain_data),
    .drain_id               (src_drain_id),

    // Status
    .drain_data_avail       (channel_data_avail),
    .dbg_bridge_pending     (channel_empty),
    .dbg_bridge_out_valid   (channel_full)
);

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	RAPIDS Core Beats Specification

Module: rapids_core_beats.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented Last Updated: 2025-01-10

## 23.1	Overview

The RAPIDS Core Beats module is the top-level integration of the “beats” architecture, combining the scheduler group array with sink and source data paths.

### 23.1.1	Key Features

- Complete RAPIDS Core: Scheduler array + sink path + source path

- 8-Channel Architecture: Full multi-channel support

- Unified MonBus: Aggregated monitoring from all subsystems

- Configurable Parameters: Data width, address width, SRAM depths

### 23.1.2	Block Diagram

### 23.1.3	Figure 3.9.1: RAPIDS Core Beats Block Diagram

RAPIDS core beats block diagram

Source: 19_rapids_core.dot

## 23.2	Parameters

parameter int NUM_CHANNELS = 8;
parameter int ADDR_WIDTH = 64;
parameter int DATA_WIDTH = 512;
parameter int DESC_DATA_WIDTH = 256;
parameter int AXI_ID_WIDTH = 8;
parameter int SRAM_DEPTH = 512;

// AXI Parameters
parameter int AR_MAX_OUTSTANDING = 8;
parameter int AW_MAX_OUTSTANDING = 8;
parameter int PIPELINE = 1;             // write-engine pipelining: 1 = up to AW_MAX_OUTSTANDING bursts in flight per channel
parameter int R_PHASE_FIFO_DEPTH = 64;
parameter int W_PHASE_FIFO_DEPTH = 64;
parameter int B_PHASE_FIFO_DEPTH = 16;

// MonBus Parameters
parameter int MON_UNIT_ID = 1;

// Feature Enables
parameter bit ENABLE_AXIS_WRAPPERS = 0;          // Use AXIS interfaces

: Table 3.9.1: RAPIDS Core Beats Parameters

## 23.3	Port List

### 23.3.1	Clock and Reset

Table 3.9.2: Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low reset |

### 23.3.2	Per-Channel APB Programming

The source and sink halves each carry their own kick-off port; there is no shared apb_* port.

Table 3.9.3: APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_apb_valid | input | NC | Source channel kick-off (per-channel) |
| src_apb_ready | output | NC | Source ready for kick-off |
| src_apb_addr | input | NC x AW | Source first descriptor address |
| snk_apb_valid | input | NC | Sink channel kick-off (per-channel) |
| snk_apb_ready | output | NC | Sink ready for kick-off |
| snk_apb_addr | input | NC x AW | Sink first descriptor address |

### 23.3.3	Per-Channel Configuration

Configuration is per half as well: every knob exists as src_cfg_* and snk_cfg_* (76 configuration ports in total, the bulk of them the descriptor monitor masks). The scheduler-facing ones are:

Table 3.9.4: Per-Channel Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_cfg_channel_enable / snk_cfg_channel_enable | input | NC | Enable per channel |
| src_cfg_channel_reset / snk_cfg_channel_reset | input | NC | Soft reset per channel |
| src_cfg_sched_enable / snk_cfg_sched_enable | input | 1 | Enable the scheduler half |
| src_cfg_sched_timeout_cycles / snk_cfg_sched_timeout_cycles | input | 32 | Write-progress timeout window (cycles) |
| src_cfg_sched_timeout_limit / snk_cfg_sched_timeout_limit | input | 8 | Consecutive-timeout windows before fatal escalation (0 = never) |
| src_cfg_sched_timeout_enable / snk_cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_axi_rd_xfer_beats | input | 8 | Source read burst size (ARLEN encoding) |
| cfg_axi_wr_xfer_beats | input | 8 | Sink write burst size (ARLEN encoding) |
| cfg_alloc_size | input | 8 | Beats allocated per sink fill request |
| cfg_drain_size | input | 8 | Beats drained per source AXIS packet |

*_cfg_sched_timeout_limit is passed straight through to the scheduler_group_array_beats instance (recoverable-timeout escalation, see the Scheduler FUB). The write-side COMMIT strobes (sched_wr_commit_strobe / sched_wr_commit_beats) generated by the sink data path’s axi_write_engine_beats are routed internally from the sink data path up into the scheduler array so completion is gated on B responses.

### 23.3.4	Descriptor AXI Master Interfaces

There are TWO descriptor masters, one per half, 18 ports each. Both are read-only, 256-bit.

Table 3.9.5: Descriptor AXI Master Interfaces (src and snk)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_m_axi_desc_arvalid / snk_m_axi_desc_arvalid | output | 1 | AR channel valid |
| src_m_axi_desc_arready / snk_m_axi_desc_arready | input | 1 | AR channel ready |
| src_m_axi_desc_araddr / snk_m_axi_desc_araddr | output | AW | AR address |
| src_m_axi_desc_arid / snk_m_axi_desc_arid | output | IW | AR id |
| src_m_axi_desc_arlen / snk_m_axi_desc_arlen | output | 8 | Burst length |
| src_m_axi_desc_arsize / snk_m_axi_desc_arsize | output | 3 | Burst size |
| src_m_axi_desc_arburst / snk_m_axi_desc_arburst | output | 2 | Burst type |
| src_m_axi_desc_rvalid / snk_m_axi_desc_rvalid | input | 1 | R channel valid |
| src_m_axi_desc_rready / snk_m_axi_desc_rready | output | 1 | R channel ready |
| src_m_axi_desc_rdata / snk_m_axi_desc_rdata | input | 256 | R data (one descriptor) |
| src_m_axi_desc_rresp / snk_m_axi_desc_rresp | input | 2 | R response |
| src_m_axi_desc_rlast / snk_m_axi_desc_rlast | input | 1 | R last |
| src_m_axi_desc_rid / snk_m_axi_desc_rid | input | IW | R id |

The AR sideband (arlock, arcache, arprot, arqos, arregion) is present on both masters and driven to AXI defaults.

### 23.3.5	Sink AXI Write Master Interface

Table 3.9.6: Sink AXI Write Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_awvalid | output | 1 | AW channel valid |
| m_axi_wr_awready | input | 1 | AW channel ready |
| m_axi_wr_awaddr | output | AW | Write address |
| m_axi_wr_awid | output | IW | Write id (carries the channel index) |
| m_axi_wr_awlen | output | 8 | Burst length |
| m_axi_wr_awsize | output | 3 | Burst size |
| m_axi_wr_awburst | output | 2 | Burst type |
| m_axi_wr_wvalid | output | 1 | W channel valid |
| m_axi_wr_wready | input | 1 | W channel ready |
| m_axi_wr_wdata | output | DW | Write data |
| m_axi_wr_wstrb | output | DW/8 | Write strobes |
| m_axi_wr_wlast | output | 1 | Last beat |
| m_axi_wr_bvalid | input | 1 | B channel valid |
| m_axi_wr_bready | output | 1 | B channel ready |
| m_axi_wr_bid | input | IW | B id (routes the commit to a channel) |
| m_axi_wr_bresp | input | 2 | Write response |

The AW sideband (awlock, awcache, awprot, awqos, awregion) is present and driven to AXI defaults. 21 ports in total.

### 23.3.6	Source AXI Read Master Interface

Table 3.9.7: Source AXI Read Master Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_arvalid | output | 1 | AR channel valid |
| m_axi_rd_arready | input | 1 | AR channel ready |
| m_axi_rd_araddr | output | AW | Read address |
| m_axi_rd_arid | output | IW | Read id (carries the channel index) |
| m_axi_rd_arlen | output | 8 | Burst length |
| m_axi_rd_arsize | output | 3 | Burst size |
| m_axi_rd_arburst | output | 2 | Burst type |
| m_axi_rd_rvalid | input | 1 | R channel valid |
| m_axi_rd_rready | output | 1 | R channel ready |
| m_axi_rd_rdata | input | DW | Read data |
| m_axi_rd_rid | input | IW | R id (routes the beat to a channel) |
| m_axi_rd_rresp | input | 2 | Read response |
| m_axi_rd_rlast | input | 1 | Last beat |

### 23.3.7	Sink Ingress – AXIS Slave

The sink takes network traffic on a standard AXI-Stream slave port. In a top this port is driven by the RX network plane (fub_axis_*), whichever protocol the plane speaks on the pins, so the core is the same in the AXIS and CXS tops (see Network Planes). There is no snk_fill_* port at core level: the fill handshake is internal to snk_data_path_axis_beats, which converts AXIS into it.

Table 3.9.8: Sink Ingress AXIS Slave

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tvalid | input | 1 | Beat valid |
| s_axis_tready | output | 1 | Ready for beat |
| s_axis_tdata | input | DW | Beat data |
| s_axis_tstrb | input | SW | Byte strobes |
| s_axis_tlast | input | 1 | Last beat of packet |
| s_axis_tid | input | AXIS_ID_WIDTH | Channel id (low CIW bits select the channel) |
| s_axis_tdest | input | AXIS_DEST_WIDTH | Destination |
| s_axis_tuser | input | AXIS_USER_WIDTH | User sideband |

### 23.3.8	Source Egress – AXIS Master

Likewise there is no src_drain_* port at core level; the drain handshake is internal to src_data_path_axis_beats.

Table 3.9.9: Source Egress AXIS Master

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tvalid | output | 1 | Beat valid |
| m_axis_tready | input | 1 | Downstream ready |
| m_axis_tdata | output | DW | Beat data |
| m_axis_tstrb | output | SW | Byte strobes |
| m_axis_tlast | output | 1 | Last beat of packet |
| m_axis_tid | output | AXIS_ID_WIDTH | Channel id |
| m_axis_tdest | output | AXIS_DEST_WIDTH | Destination |
| m_axis_tuser | output | AXIS_USER_WIDTH | User sideband |

### 23.3.9	Unified MonBus Interface

One packet port, carrying a struct rather than a flat 64-bit bus.

Table 3.9.10: Unified MonBus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Packet valid |
| mon_ready | input | 1 | Consumer ready |
| mon_packet | output | ocelus_common_pkg::monitor_packet_t | Packet payload |
| mon_timestamp | output | ocelus_common_pkg::monbus_timestamp_t | Packet timestamp |

### 23.3.10	Aggregate Status

Status is reported per half. There is no all_channels_idle, sink_idle, source_idle or error_flags port.

Table 3.9.11: Aggregate Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| src_system_idle | output | 1 | Source half idle (all source schedulers idle) |
| snk_system_idle | output | 1 | Sink half idle (all sink schedulers idle) |
| src_scheduler_idle / snk_scheduler_idle | output | NC | Per-channel scheduler idle |
| src_scheduler_state / snk_scheduler_state | output | NC x 7 | Per-channel FSM state |
| src_descriptor_engine_idle / snk_descriptor_engine_idle | output | NC | Per-channel descriptor engine idle |
| src_sched_error / snk_sched_error | output | NC | Per-channel sticky scheduler error |

## 23.4	Data Flow

### 23.4.1	Figure 3.9.2: RAPIDS Core Complete Data Flow

RAPIDS core data flow

Source: 22_core_data_flow.dot

## 23.5	MonBus Aggregation

The unified MonBus output aggregates sources from all subsystems:

Table 3.9.12: MonBus Source Assignment

| Source Range | Origin | Description |
| --- | --- | --- |
| 0-15 | Scheduler Array | Per-channel scheduler + desc engine |
| 16-23 | Sink Data Path | Sink SRAM controllers |
| 24-31 | Source Data Path | Source SRAM controllers |
| 32 | AXI Write Engine | Write completions |
| 33 | AXI Read Engine | Read completions |

## 23.6	Integration Example

rapids_core_beats #(
    .NUM_CHANNELS(8),
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .SRAM_DEPTH(512)
) u_rapids_core (
    .clk                        (clk),
    .rst_n                      (rst_n),

    // APB kick-off (per half)
    .src_apb_valid              (src_kick_valid),
    .src_apb_ready              (src_kick_ready),
    .src_apb_addr               (src_kick_addr),
    .snk_apb_valid              (snk_kick_valid),
    .snk_apb_ready              (snk_kick_ready),
    .snk_apb_addr               (snk_kick_addr),

    // Configuration (per half)
    .src_cfg_channel_enable     (src_cfg_ch_enable),
    .snk_cfg_channel_enable     (snk_cfg_ch_enable),
    .src_cfg_sched_timeout_cycles(src_cfg_timeout),
    .snk_cfg_sched_timeout_cycles(snk_cfg_timeout),
    .cfg_axi_rd_xfer_beats      (cfg_rd_beats),
    .cfg_axi_wr_xfer_beats      (cfg_wr_beats),
    .cfg_alloc_size             (cfg_alloc_size),
    .cfg_drain_size             (cfg_drain_size),

    // Descriptor AXI -- one master per half
    .src_m_axi_desc_arvalid     (src_desc_arvalid),
    .src_m_axi_desc_arready     (src_desc_arready),
    .src_m_axi_desc_araddr      (src_desc_araddr),
    .src_m_axi_desc_rvalid      (src_desc_rvalid),
    .src_m_axi_desc_rready      (src_desc_rready),
    .src_m_axi_desc_rdata       (src_desc_rdata),
    .snk_m_axi_desc_arvalid     (snk_desc_arvalid),
    .snk_m_axi_desc_arready     (snk_desc_arready),
    .snk_m_axi_desc_araddr      (snk_desc_araddr),
    .snk_m_axi_desc_rvalid      (snk_desc_rvalid),
    .snk_m_axi_desc_rready      (snk_desc_rready),
    .snk_m_axi_desc_rdata       (snk_desc_rdata),

    // Sink AXI write master (memory side)
    .m_axi_wr_awvalid           (wr_awvalid),
    .m_axi_wr_awready           (wr_awready),
    .m_axi_wr_awaddr            (wr_awaddr),
    .m_axi_wr_awlen             (wr_awlen),
    .m_axi_wr_wvalid            (wr_wvalid),
    .m_axi_wr_wready            (wr_wready),
    .m_axi_wr_wdata             (wr_wdata),
    .m_axi_wr_wlast             (wr_wlast),
    .m_axi_wr_bvalid            (wr_bvalid),
    .m_axi_wr_bready            (wr_bready),
    .m_axi_wr_bresp             (wr_bresp),

    // Source AXI read master (memory side)
    .m_axi_rd_arvalid           (rd_arvalid),
    .m_axi_rd_arready           (rd_arready),
    .m_axi_rd_araddr            (rd_araddr),
    .m_axi_rd_arlen             (rd_arlen),
    .m_axi_rd_rvalid            (rd_rvalid),
    .m_axi_rd_rready            (rd_rready),
    .m_axi_rd_rdata             (rd_rdata),
    .m_axi_rd_rlast             (rd_rlast),

    // Sink ingress -- AXIS slave (network in)
    .s_axis_tvalid              (network_rx_tvalid),
    .s_axis_tready              (network_rx_tready),
    .s_axis_tdata               (network_rx_tdata),
    .s_axis_tlast               (network_rx_tlast),
    .s_axis_tid                 (network_rx_tid),

    // Source egress -- AXIS master (network out)
    .m_axis_tvalid              (network_tx_tvalid),
    .m_axis_tready              (network_tx_tready),
    .m_axis_tdata               (network_tx_tdata),
    .m_axis_tlast               (network_tx_tlast),
    .m_axis_tid                 (network_tx_tid),

    // Unified MonBus
    .mon_valid                  (mon_valid),
    .mon_ready                  (mon_ready),
    .mon_packet                 (mon_packet),
    .mon_timestamp              (mon_timestamp),

    // Status (per half)
    .src_system_idle            (src_idle),
    .snk_system_idle            (snk_idle)
);

Note: rapids_core_beats is a core-level block. The register interface, configuration mapping, and top-level integration (APB slave, descriptor kick-off, AXI monitors, and the MonBus AXI-Lite group) are provided by the rapids_regs, rapids_config_block, and rapids_beats_top modules – see RAPIDS Registers, RAPIDS Config Block, and RAPIDS Beats Top.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	RAPIDS Registers Specification

Module: rapids_regs.sv (generated) Source: projects/components/dma-ip/rapids/rtl/macro_beats/rapids_regs.rdl, rapids_mon_regs.rdl Generated: projects/components/dma-ip/rapids/regs/generated/rtl/rapids_regs.sv Status: Implemented (PeakRDL-generated)

## 24.1	Overview

rapids_regs is the PeakRDL-generated register block for the RAPIDS Beats top-level. It exposes configuration and status through a single command/response CPU interface and drives the hardware via a hwif_out structure that rapids_config_block maps onto the core/monitor cfg_* signals.

A single addrmap (one APB slave) contains two regfiles:

- Base config/status registers at 0x100-0x3FF.

- A nested rapids_mon_regs monitor regfile at base 0x1000, decoded under hwif_out.MON.* (all DAXMON / RDMON / WRMON / MON_FIFO and per-monitor performance registers).

Because the monitor regfile lives at 0x1000, the register-block address decode requires at least 13 bits (cpuif_addr is a 13-bit compare in the generated RTL). The APB address bus feeding the block (APB_ADDR_WIDTH) must therefore be >= 13 to reach the monitor regfile.

## 24.2	Base Register Map (0x100-0x3FF)

Table 3.10.1: Base Register Map

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | RW | GLOBAL_EN[0], GLOBAL_RST[1] (level; write 0 to release) |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE[0] (all channels idle) |
| 0x108 | VERSION | RO | MINOR[7:0]=0x5A, MAJOR[15:8], NUM_CHANNELS[23:16]=0x08 |
| 0x120 | CHANNEL_ENABLE | RW | CH_EN[7:0] per-channel enable |
| 0x124 | CHANNEL_RESET | RW | CH_RST[7:0] per-channel reset (level; write 0 to release) |
| 0x140 | CHANNEL_IDLE | RO | CH_IDLE[7:0] – not driven at the top level, reads 0 |
| 0x144 | DESC_ENGINE_IDLE | RO | Per-channel descriptor-engine idle |
| 0x148 | SCHEDULER_IDLE | RO | Per-channel scheduler idle (excludes CH_ERROR) |
| 0x150-0x16C | CH_STATE[0..7].STATE | RO | Per-channel FSM state (stride 0x4) |
| 0x170 | SCHED_ERROR | RO | Per-channel sticky scheduler error |
| 0x174 | AXI_RD_COMPLETE | RO | Per-channel read-complete flags (SRC half only; reads 0 in SNK) |
| 0x178 | AXI_WR_COMPLETE | RO | Per-channel write-complete flags – not driven, reads 0 |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | TIMEOUT_CYCLES[31:0] write-progress window (reset 1000) |
| 0x204 | SCHED_CONFIG | RW | SCHED_EN, TIMEOUT_EN, ERR_EN, COMPL_EN, PERF_EN |
| 0x208 | SCHED_TIMEOUT_LIMIT | RW | LIMIT[7:0] consecutive-timeout escalation limit (reset 4, 0 = never) |
| 0x220 | DESCENG_CONFIG | RW | DESCENG_EN[0], PREFETCH_EN[1], FIFO_THRESH[5:2] (reset 8) |
| 0x224 | DESCENG_ADDR0_BASE | RW | Descriptor address range 0 base [31:0] |
| 0x228 | DESCENG_ADDR0_LIMIT | RW | Descriptor address range 0 limit [31:0] |
| 0x22C | DESCENG_ADDR1_BASE | RW | Descriptor address range 1 base [31:0] |
| 0x230 | DESCENG_ADDR1_LIMIT | RW | Descriptor address range 1 limit [31:0] |
| 0x240 | CTRL_CONFIG | RW | CTRLRD_MAX_TRY[8:0] control-read poll retry budget (0-511, reset 16) |
| 0x2A0 | AXI_XFER_CONFIG | RW | AXI transfer sizing configuration |
| 0x2B0 | PERF_CONFIG | RW | Performance profiler configuration |
| 0x2C0 | OBS_CTRL | RW | Observation mux control (channel/category select) |
| 0x2C4 | OBS_FLAGS | RO | Observation flags |
| 0x2C8 | OBS_DATA0 | RO | Observation data word 0 |
| 0x2CC | OBS_DATA1 | RO | Observation data word 1 |
| 0x300-0x31C | CH_SCRATCH[n].SCRATCH | RW | Eight software bits per channel, SCRATCH[7:0] (reset 0); storage only, for flags or semaphores between the host and, in rapids-g, the half’s gatling. Not atomic across masters: give each bit one writer |
| 0x35C | PERF_CH_SEL | RW | Performance channel select |
| 0x378 | HIST_SEL | RW | Histogram bucket select |
| 0x37C | HIST_DATA | RO | Histogram bucket data |
| 0x380 | HIST_TOTAL | RO | Histogram total count |

### 24.2.1	Key Register Fields

SCHED_TIMEOUT_LIMIT (0x208) – LIMIT[7:0] sets the number of consecutive write-progress timeout windows a channel tolerates before the recoverable timeout escalates to a fatal, sticky CH_ERROR. 0 = never escalate (pure soft timeout). Total time to escalate is approximately LIMIT x TIMEOUT_CYCLES. Reset value is 4.

DESCENG_CONFIG (0x220) – PREFETCH_EN[1] and FIFO_THRESH[5:2] control the now-functional descriptor prefetch: prefetch off = on-demand chaining (~1 ahead), prefetch on = buffer up to FIFO_THRESH descriptors ahead.

CTRL_CONFIG (0x240) – CTRLRD_MAX_TRY[8:0] bounds the control-read poll retry budget (0-511, reset 16) fed to every channel’s ctrlrd_engine. A CTRL_READ descriptor polls its gate address once per retry; if the masked value never matches within CTRLRD_MAX_TRY attempts the engine raises ctrlrd_error so a never-satisfied gate cannot hang the channel. See the Control-Read / Control-Write Engine specs (Sections 2.8 / 2.9) and the Scheduler control interface (Section 2.1).

## 24.3	Monitor Register Map (rapids_mon_regs @ 0x1000)

The monitor regfile is instantiated at base 0x1000 under hwif_out.MON.*. It holds the MonBus FIFO status, the three AXI-monitor configuration groups (DAXMON = descriptor monitor, RDMON = read monitor, WRMON = write monitor), and per-monitor performance counters.

Table 3.10.2: Monitor Register Map (base 0x1000)

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x1000 | MON.MON_FIFO_STATUS | RO | MonBus capture/error FIFO status |
| 0x1004 | MON.MON_FIFO_COUNT | RO | MonBus FIFO occupancy |
| 0x10C0 | MON.DAXMON_ENABLE | RW | Descriptor-monitor enables (incl. COMPRESS_EN) |
| 0x10C4 | MON.DAXMON_TIMEOUT | RW | Descriptor-monitor timeout cycles |
| 0x10C8 | MON.DAXMON_LATENCY_THRESH | RW | Descriptor-monitor latency threshold |
| 0x10CC | MON.DAXMON_PKT_MASK | RW | Descriptor-monitor packet mask |
| 0x10D0 | MON.DAXMON_ERR_CFG | RW | Descriptor-monitor error select/mask |
| 0x10D4-0x10DC | MON.DAXMON_MASK1/2/3 | RW | Descriptor-monitor category masks |
| 0x10E0-0x10FC | MON.RDMON_* | RW | Source-egress AXIS monitor-lite config (same layout as DAXMON; SRC instance, rapids TASK-015) |
| 0x1100-0x111C | MON.WRMON_* | RW | Sink-ingress AXIS monitor-lite config (same layout as DAXMON; SNK instance, rapids TASK-015) |
| 0x1150-0x1178 | MON.DAXMON_PERF_* | RO/RW | Descriptor-monitor performance counters |
| 0x1180-0x11A8 | MON.RDMON_PERF_* | RO/RW | Read-monitor performance counters |
| 0x11B0-0x11D8 | MON.WRMON_PERF_* | RO/RW | Write-monitor performance counters |
| 0x11E0-0x11EC | MON.{RD,WR}MON_PERF_CH_* | RO | Per-channel producer/backpressure/starve/idle |
| 0x11F0 | MON.RDMON_PERF_CH_OVERFLOW | RO | Read-monitor per-channel overflow |
| 0x11F4 | MON.WRMON_PERF_CH_OVERFLOW | RO | Write-monitor per-channel overflow |

Each *_ENABLE register carries MON_EN, ERR_EN, COMPL_EN, TIMEOUT_EN (and COMPRESS_EN on the write monitor); each *_PERF_* group provides window, producer, backpressure, starve, idle, beat, byte (lo/hi), and burst counters.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	RAPIDS Config Block Specification

Module: rapids_config_block.sv Location: projects/components/dma-ip/rapids/rtl/macro_beats/ Status: Implemented

## 25.1	Overview

rapids_config_block is a combinational adapter that maps the register block’s hwif_out structure onto the flat cfg_* signals consumed by rapids_core_beats and the AXI monitors. It contains no state; every output is a direct assignment (with a small number of width expansions and global-enable gates).

The monitor registers arrive under hwif_out.MON.*; base configuration and status arrive at the top level of hwif_out.

## 25.2	Mapping Groups

### 25.2.1	Base Configuration

Routes scheduler, channel, and descriptor-engine configuration to the core:

- cfg_channel_enable = CHANNEL_ENABLE.CH_EN & {8{GLOBAL_CTRL.GLOBAL_EN}} (per-channel enable gated by the global master enable).

- cfg_sched_timeout_cycles = SCHED_TIMEOUT_CYCLES.TIMEOUT_CYCLES (32-bit).

- cfg_sched_timeout_limit  = SCHED_TIMEOUT_LIMIT.LIMIT (8-bit, direct).

- cfg_sched_{timeout,err,compl,perf}_enable = SCHED_CONFIG.{TIMEOUT,ERR,COMPL,PERF}_EN. COMPL_EN is the one the scheduler group acts on for its monitor packets: 0 drops the scheduler’s and descriptor engine’s CORE Completion packets at the group (emitters still acknowledged); ERR_EN and PERF_EN reach the group but gate nothing today (errors always reported, perf not implemented).

- cfg_desceng_* from DESCENG_CONFIG (enable / prefetch / FIFO threshold).

- Descriptor address ranges (cfg_desceng_addr{0,1}_{base,limit}) are zero-extended from the 32-bit register fields to the 64-bit ADDR_WIDTH.

### 25.2.2	Monitor Configuration (hwif_out.MON.*)

Three parallel groups map to the descriptor-read monitor and, since rapids TASK-015, the two AXIS data-path monitor-lites (one per half):

Table 3.11.1: Monitor Register-to-Config Mapping

| Register group | cfg_* target |
| --- | --- |
| MON.DAXMON_* | cfg_desc_mon_* (descriptor monitor) |
| MON.RDMON_* | cfg_rdeng_mon_* (source-egress AXIS monitor-lite; wired in the SRC half only) |
| MON.WRMON_* | cfg_wreng_mon_* (sink-ingress AXIS monitor-lite; wired in the SNK half only) |

The *_ENABLE bits map onto the AXIS monitor-lite’s class enables: MON_EN (gated by GLOBAL_EN), ERR_EN (error, channel-change and TSTRB checks), COMPL_EN (one Completion/STREAM_END per packet), PERF_EN (stream start/pause/resume and Credit/BACKPRESSURE stall reports), TIMEOUT_EN. *_TIMEOUT is MICROSECONDS in its low 16 bits (0 = never), *_LATENCY_THRESH the stall CYCLES that raise a backpressure report, and *_PKT_MASK is a MASK: bit[type] = 1 masks (drops) that packet type at the monitor-lite and again at the monbus group. It resets to all-masked (0xFFFF); the host clears the bits of the classes it wants to see (rapids BUG-008 corrected the description, which used to read “1 = enable”; the polarity and the reset are as intended).

Within each group the enable/timeout/latency/mask fields map 1:1, e.g. cfg_desc_mon_enable = MON.DAXMON_ENABLE.MON_EN & GLOBAL_CTRL.GLOBAL_EN, cfg_desc_mon_timeout_cycles = MON.DAXMON_TIMEOUT.TIMEOUT_CYCLES, and the category masks (timeout/compl/thresh/perf/addr/debug) pass through directly. Key enables are gated by GLOBAL_CTRL.GLOBAL_EN; masks pass through ungated.

### 25.2.3	Performance and Observation

PERF_CONFIG, PERF_CH_SEL, and OBS_CTRL map to the profiler and channel-observation-mux cfg_* signals.

Last Updated: 2026-07-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	RAPIDS Beats Top Specification

Module: rapids_beats_top.sv, rapids_beats_cxs_top.sv (generated), rapids_beats_top_base.sv Location: projects/components/dma-ip/rapids/rtl/top_beats/ Status: Implemented

## 26.1	Overview

rapids_beats_top is the synthesizable top-level integration of the RAPIDS Beats accelerator. It wraps the register block, config-block adapter, and core with a single APB slave for all software access, and adds optional AXI transaction monitors whose events are merged with the core’s descriptor-monitor packet and delivered through a MonBus AXI-Lite group (error-drain slave, capture master, and interrupt).

The top is generated (rapids TASK-024). rapids_beats_top_base holds everything described on this page; bin/gen_plane_tops.py wraps it with one RX and one TX network plane to make rapids_beats_top (AXIS planes) and rapids_beats_cxs_top (CXS planes). The planes are described in the Network Planes chapter; edit the base or the generator, never a generated top.

## 26.2	Integration Datapath

### 26.2.1	Figure 3.12.1: RAPIDS Beats Top Integration

RAPIDS beats top integration

Source: 20_rapids_beats_top.dot

### 26.2.2	Address Decode

Table 3.12.1: Top-Level Address Decode

| Range | Target | Purpose |
| --- | --- | --- |
| 0x000-0x03F | rapids_regs (kick) | Per-channel descriptor kick-off registers |
| 0x100-0x3FF | rapids_regs (base) | Configuration / status registers |
| 0x1000+ | rapids_regs (MON regfile) | Monitor configuration / performance |

Because the monitor regfile is at 0x1000, the APB address bus must be at least 13 bits wide (APB_ADDR_WIDTH >= 13) to reach it.

## 26.3	Write-engine pipelining (PIPELINE)

PIPELINE reaches axi_write_engine_beats through the core and both halves. 0 is the engine’s original contract, one burst in flight per channel; 1 allows up to AW_MAX_OUTSTANDING per channel. Every module that declares the parameter, from the engines to this top, defaults to 1 since 2026-09-28: once rapids BUG-005 made PIPELINE = 0 honour one-in-flight, the 8-channel Genesys 2 build measured 49.9 % AXI4-wr engaged utilization at 4096 beats per channel (59.5 % at 1024), against 100 % on the bitstream before the fix – which had reached line rate only because the pre-fix engine ran two bursts in flight by accident. The harness simulation reproduces the same numbers (82.4 % at 1024 beats with PIPELINE = 0, 99.9 % with 1). One burst in flight cannot cover the write-response round trip at 8-beat bursts, so 1 is the design point at every level; 0 is selectable for a design that must bound each channel to a single outstanding write, never the default.

## 26.4	AXI Monitors (USE_AXI_MONITORS)

When USE_AXI_MONITORS = 1 the top builds the monbus_axil4_axil4_group and feeds it the core’s MonBus stream. That stream carries the scheduler groups’ completion and error events and two monitor-lite instances per half: axi4_master_rd_monlite on the half’s descriptor read master, inside scheduler_group_array_beats (agent 0x08), and a network monitor-lite in the half’s network plane (rapids TASK-015, moved to the planes by TASK-024): the sink plane’s lite on s_axis_* or s_cxs_* (agent 0x09, configured by SNK.MON.WRMON_*) and the source plane’s lite on m_axis_* or m_cxs_* (agent 0x0A, configured by SRC.MON.RDMON_*). The plane’s packets enter the core on snk_plane_mon_* / src_plane_mon_*. Each half merges its scheduler array (client 0), its plane monitor (client 1) and a tied-off placeholder client in a 3:1 monbus_arbiter; the core merges the two halves 2:1. The AXIS planes also place an axis4_slave / axis4_master skid stage (SKID_DEPTH = 4) on the network port in every build; the CXS planes buffer in the receive FIFO instead. Only the monitor tap is gated by USE_AXI_MONITORS. The data masters m_axi_rd and m_axi_wr carry NO monitors (an earlier revision of this page said rd/wr monitor-lite blocks sat on the data masters; the RTL has never had them). Data-master observation lives in the characterization harness as external instruments (axi_bus_meter, axis_bus_meter, and the interface observers on USE_OBSERVERS builds). The group’s AXIS filter slot is fed from SRC.MON.RDMON_*, so it filters both halves’ network packets (a CXS plane’s PROTOCOL_CXS packets ride the AXIS slot); *_PKT_MASK masks on a set bit and resets to all-masked (rapids BUG-008 corrected its description). The group provides:

- s_axil_err_* – AXI-Lite (32-bit) error-drain slave: CPU reads captured error events from the error FIFO.

- m_axil_mon_* – AXI-Lite (64-bit) capture master: bulk-writes the MonBus trace to system memory (base/limit/watermark from cfg_mon_*).

- mon_irq – interrupt on error/threshold events.

When USE_AXI_MONITORS = 0 the descriptor and network monitor-lites are omitted (the AXIS skid stages and CXS receive FIFO stay), the core MonBus is dropped (always-ready), and the AXI-Lite group outputs are tied off (s_axil_err read-inactive, m_axil_mon write-inactive, mon_irq = 0).

### 26.4.1	Monitor registers on a monitors-off build (USE_MON_REGS)

USE_MON_REGS defaults to USE_AXI_MONITORS != 0 and is passed to both config blocks. When it is 0 the blocks strap every cfg_*mon_* output off, and the APB path answers the two MON register windows – SRC MON at 0x0800-0x0FFF and SNK MON at 0x1800-0x1FFF, i.e. paddr[11] set in either 4 KB half – with PSLVERR and zero read data instead of letting the registers accept a write and read it back. The address map does not change, so one host image still addresses both builds; what changes is that “not built” is distinguishable from “built and set to zero”. A blocked access is still accepted (holding cmd_ready low would hang the bus) and the guard’s error beat replaces the adapter’s response for that one transfer (rapids ISSUE-003; the shape is STREAM’s TASK-002 guard). With USE_MON_REGS = 1 the guard folds away to the straight-through wiring.

## 26.5	Parameters

parameter int NUM_CHANNELS        = 8;
parameter int DATA_WIDTH          = 512;
parameter int ADDR_WIDTH          = 64;
parameter int AXI_ID_WIDTH        = 8;
parameter int SRAM_DEPTH          = 4096;
parameter int APB_ADDR_WIDTH      = 13;   // bit 12 selects SRC (0) / SNK (1)
parameter int APB_DATA_WIDTH      = 32;
parameter int USE_AXI_MONITORS    = 1;    // 1 = descriptor + network monitor-lites + MonBus group
parameter bit USE_MON_REGS        = (USE_AXI_MONITORS != 0); // 0 = MON windows answer PSLVERR
parameter int MON_MAX_TRANSACTIONS = 16;
parameter int AR_MAX_OUTSTANDING  = 8;
parameter int AW_MAX_OUTSTANDING  = 8;
parameter int PIPELINE            = 1;   // write-engine pipelining; see the note below
parameter int AXIS_ID_WIDTH       = 8;   // network stream tid (carries the channel)
parameter int AXIS_DEST_WIDTH     = 4;
parameter int AXIS_USER_WIDTH     = 1;
// rapids_beats_cxs_top only (see the Network Planes chapter):
parameter int CXS_MAX_CREDIT      = 32;  // one credit = one flit
parameter int CXS_RX_FIFO_DEPTH   = CXS_MAX_CREDIT;
parameter bit CXS_CREDIT_BEYOND_SPEC = (CXS_MAX_CREDIT > 63);
parameter logic [7:0] CXS_IDLE_CYCLES = 8'd32;

: Table 3.12.2: RAPIDS Beats Top Parameters

## 26.6	Top-Level Interfaces

Table 3.12.3: Top-Level Interfaces

| Interface | Signals | Notes |
| --- | --- | --- |
| Clock / reset | aclk, aresetn, cam_clear | Single clock domain; cam_clear for monitor CAMs |
| APB slave | s_apb_paddr (APB_ADDR_WIDTH), s_apb_psel/penable/pwrite/pwdata/pstrb, s_apb_prdata/pready/pslverr | 32-bit data |
| Descriptor AXI (master) | m_axi_desc_ar*, m_axi_desc_r* | 256-bit read data |
| Data read AXI (master) | m_axi_rd_ar*, m_axi_rd_r* | DATA_WIDTH; no monitor |
| Data write AXI (master) | m_axi_wr_aw*, m_axi_wr_w*, m_axi_wr_b* | DATA_WIDTH; no monitor |
| Network in (AXIS top) | s_axis_tdata/tstrb/tlast/tid/tdest/tuser/tvalid/tready | Sink ingress, tid = channel |
| Network out (AXIS top) | m_axis_tdata/tstrb/tlast/tid/tdest/tuser/tvalid/tready | Source egress, tid = channel |
| Network in (CXS top) | s_cxs_*, snk_cxs_link_state, snk_cxs_protocol_error, snk_cxs_unsupported | CXS receiver, CXSSRCID = channel |
| Network out (CXS top) | m_cxs_*, src_cxs_link_state, src_cxs_protocol_error | CXS transmitter, CXSSRCID = channel |
| MonBus error-drain slave | s_axil_err_ar*, s_axil_err_r* | AXI-Lite, 32-bit read |
| MonBus capture master | m_axil_mon_aw*, m_axil_mon_w* (64-bit wdata), m_axil_mon_b* | AXI-Lite write |
| MonBus interrupt | mon_irq | Error/threshold interrupt |
| MonBus config | cfg_mon_base_addr, cfg_mon_limit_addr, cfg_mon_flush_watermark | Capture region + flush watermark |
| Status | system_idle, sched_error[NC-1:0] | Aggregate status |

Last Updated: 2026-10-16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Network Planes Specification

Modules: rapids_plane_axis_rx.sv, rapids_plane_axis_tx.sv, rapids_plane_cxs_rx.sv, rapids_plane_cxs_tx.sv Location: projects/components/dma-ip/rapids/rtl/plane/ Generator: projects/components/dma-ip/rapids/bin/gen_plane_tops.py Status: Implemented (rapids TASK-024, TASK-025)

## 27.1	Overview

A network plane is the endpoint between the core’s network-side AXIS stream and the pins: the RX plane feeds the sink half’s ingress, the TX plane takes the source half’s egress. Each plane carries the monitor-lite for its port. Both RAPIDS trees (byte-granular rtl/top, beat-granular rtl/top_beats) share the same four planes.

Every plane has the same parameters and the same inner interface, so a top selects AXIS or CXS by module name alone:

- fub_axis_*: the core-facing AXIS stream (tdata, tstrb, tlast, tid = channel, tdest, tuser)

- cfg_mon_* in, sts_mon_* out: the SNK.WRMON_* / SRC.RDMON_* monitor CSRs, from rapids_config_block

- monbus_* out: client 1 of the half’s 3:1 monbus_arbiter, entering the core on snk_plane_mon_* / src_plane_mon_*

Table 3.13.1: RAPIDS Network Planes

| Plane | Contents | Outer pins |
| --- | --- | --- |
| rapids_plane_axis_rx | axis4_slave_monlite (skid stage, SKID_DEPTH = 4, + axis_monitor_lite tap) | s_axis_* |
| rapids_plane_axis_tx | axis4_master_monlite (skid stage + tap) | m_axis_* |
| rapids_plane_cxs_rx | cxs_slave_monlite (u_endpoint) + cxs_to_axis (u_cxs_to_axis) | s_cxs_*, snk_cxs_* status |
| rapids_plane_cxs_tx | axis_to_cxs (u_axis_to_cxs) + cxs_master_monlite (u_endpoint) | m_cxs_*, src_cxs_* status |

The monitor counters of every plane are at u_snk_plane.u_endpoint and u_src_plane.u_endpoint in the top.

## 27.2	Generated Tops

Table 3.13.2: Generated Tops

| Top | Tree | Base | Planes |
| --- | --- | --- | --- |
| rapids_top | byte | rapids_top_base | AXIS |
| rapids_cxs_top | byte | rapids_top_base | CXS |
| rapids_beats_top | beats | rapids_beats_top_base | AXIS |
| rapids_beats_cxs_top | beats | rapids_beats_top_base | CXS |

gen_plane_tops.py copies the base’s parameter and port text verbatim, then instantiates the base (u_base) and the two planes (u_snk_plane, u_src_plane). The AXIS tops therefore keep exactly the interface they had before the split, and existing instantiations are unchanged. Run the generator with --check to fail on a stale top.

## 27.3	CXS Planes

### 27.3.1	Figure 3.13.1: CXS Network Planes

CXS network planes

Source: 23_cxs_planes.dot

### 27.3.2	Receive (sink) plane

cxs_slave_monlite is cxs_slave with a cxs_monitor_lite tap. cxs_slave grants credits up to CXS_MAX_CREDIT, runs the activation handshake, and holds received flits in a receive FIFO of RX_FIFO_DEPTH (default CXS_MAX_CREDIT) entries. The FIFO is the credit pool: a credit is only granted for a free entry, so a flit always has space. cxs_to_axis turns flits back into AXIS beats: CXSSRCID becomes tid, CXSTGTID becomes tdest, and CXSUSER is split into {tuser, tstrb}. When the core stops taking beats the FIFO fills, and the slave stops granting credits; that is the CXS form of the sink’s tready backpressure.

cxs_to_axis is combinational and stateless, so it can only represent the flits axis_to_cxs makes: at most one START and one END per flit, with a packet starting at byte 0. Any other flit (a transmitter that packs two packets into one flit) is still passed on, with tlast from END[0], and sets the sticky snk_cxs_unsupported.

### 27.3.3	Transmit (source) plane

axis_to_cxs packs AXIS beats into flits (tid -> CXSSRCID, tdest -> CXSTGTID, {tuser, tstrb} -> CXSUSER) and cxs_master_monlite sends them, one credit per flit. The master requests activation when it has a flit to send and deactivates the link after CXS_IDLE_CYCLES idle cycles.

### 27.3.4	Per-stream framing

axis_to_cxs and both endpoints’ control checkers track an open packet per stream, keyed on the low CXS_STREAM_SRCID_BITS = CIW bits of CXSSRCID (the channel); cxs_to_axis needs no state because each beat carries its own tid. The byte-granular source arbitrates per drain block while a packet spans the whole descriptor, so with two or more channels active the egress switches channel in the middle of a packet. Per-link framing (one open packet per link) rejects that traffic; per-stream framing accepts it, as IHI 0079D 2.5.2 allows.

### 27.3.5	Parameters

Table 3.13.3: CXS Plane Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| CXS_MAX_CREDIT | 32 | Credits granted by the receiver. One credit is one flit; the grant -> flit -> free round trip is about 3 cycles |
| CXS_RX_FIFO_DEPTH | CXS_MAX_CREDIT | Receive FIFO depth (the credit pool); need not be a power of two |
| CXS_CREDIT_BEYOND_SPEC | CXS_MAX_CREDIT > 63 | Permits more credits than IHI 0079D allows; the far end must match |
| CXS_IDLE_CYCLES | 32 | Idle cycles before the transmitter deactivates the link |
| CXS_IW, CXS_DESTW, CXS_UW, CXS_CNTL_PW | derived | Port widths; do not override |

DATA_WIDTH must be at most 1024: CXSUSER is at most 128 bits and carries tstrb (DATA_WIDTH/8 bits). tuser is dropped from CXSUSER when DATA_WIDTH/8 + AXIS_USER_WIDTH > 128.

### 27.3.6	Monitoring

The CXS planes drive cxs_monitor_lite from the same cfg_mon_* CSRs as the AXIS planes and use the same agent IDs (sink 0x09, source 0x0A). Packets are tagged PROTOCOL_CXS; the top’s monitor group filters them through its AXIS mask slot. A PACKET_END completion carries the channel in event_data[55:48]; channel_id holds {PRCLTYPE[2:0], TGTID[5:0]}. The lite’s output queue is 4 entries deep; when events outrun it the lite drops and counts them (an Error / EVENT_DROPPED packet), so a check on completions counts delivered plus dropped.

## 27.4	Proposed Changes

A proposal (2026-10-16, not implemented) puts a tile packet header at the front of every packet: destination and source coordinates, payload type (cfg / prog / mem), length and target address. It is described once in the compute-engine tile’s TILE_SPEC.md Section 11; CXS carries it as the first flit. The link signals, credits and per-stream framing on this page do not change, because CXSSRCID keeps the channel.

## 27.5	Verification

Table 3.13.4: CXS Top Test Cells

| Suite | Cells |
| --- | --- |
| dv/tests/top/test_rapids_cxs_top.py | source, sink, source_bytes, sink_bytes, source_concurrent, sink_concurrent, cxs_mon, ext_addressing |
| dv/tests/top_beats/test_rapids_beats_cxs_top.py | source, sink, source_concurrent, sink_concurrent, cxs_mon, ext_addressing |

The concurrent cells start four channels with one KICK_ENABLE write, so channels interleave on the link. Every cell also requires a clean link: no BFM or wire-monitor violation and the DUT’s protocol-error flags low. The core testbenches drive packets on plane_mon_* and require them on the MonBus output.

Last Updated: 2026-10-16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	Interface Overview

Last Updated: 2026-10-16

## 28.1	External Interfaces

The RAPIDS Beats architecture exposes the following external interfaces:

### 28.1.1	AXI4 Master Interfaces

Table 4.0.1: AXI4 Master Interfaces

| Interface | Purpose | Data Width |
| --- | --- | --- |
| Descriptor AXI | Fetch descriptors from memory | 256-bit |
| Sink AXI Write | Write sink data to memory | 512-bit (param) |
| Source AXI Read | Read source data from memory | 512-bit (param) |

### 28.1.2	Streaming Interfaces

Table 4.0.2: Streaming Interfaces

| Interface | Type | Purpose |
| --- | --- | --- |
| Fill Interface | Custom | Network data ingress (to SRAM), inside the core |
| Drain Interface | Custom | Network data egress (from SRAM), inside the core |
| AXIS Sink | AXI-Stream Slave | Network ingress on the AXIS tops; the core-facing stream of every plane |
| AXIS Source | AXI-Stream Master | Network egress on the AXIS tops; the core-facing stream of every plane |
| CXS Sink | CXS receiver | Network ingress on the CXS tops (one link, CXSSRCID = channel) |
| CXS Source | CXS transmitter | Network egress on the CXS tops (one link, CXSSRCID = channel) |

### 28.1.3	Control/Monitor Interfaces

Table 4.0.3: Control and Monitor Interfaces

| Interface | Type | Purpose |
| --- | --- | --- |
| APB | APB slave | Per-channel kick-off |
| Configuration | Custom | Global/per-channel config |
| Status | Custom | Idle, state, error flags |
| MonBus | Custom 64-bit | Monitor packet output |

## 28.2	Interface Documentation

- AXI4 Interface Specification - Descriptor, sink write, source read

- AXIS Interface Specification - Optional streaming wrappers

- MonBus Interface Specification - Monitor packet format

- CXS Interface Specification - CXS network links (CXS tops)

## 28.3	Interface Signal Naming Conventions

Table 4.0.4: Signal Naming Conventions

| Prefix | Meaning | Example |
| --- | --- | --- |
| m_axi_ | AXI master port | m_axi_arvalid |
| s_axi_ | AXI slave port | s_axi_awready |
| m_axis_ | AXIS master port | m_axis_tvalid |
| s_axis_ | AXIS slave port | s_axis_tready |
| m_cxs_ | CXS transmitter port | m_cxs_valid |
| s_cxs_ | CXS receiver port | s_cxs_crdgnt |
| cfg_ | Configuration input | cfg_enable |
| snk_ | Sink data path | snk_fill_valid |
| src_ | Source data path | src_drain_ready |
| sched_ | Scheduler interface | sched_rd_valid |
| monbus_ | Monitor bus | monbus_pkt_data |

Last Updated: 2026-10-16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	AXI4 Interface Specification

Last Updated: 2025-01-10

## 29.1	Overview

The RAPIDS Beats architecture uses three AXI4 master interfaces: 1. Descriptor AXI: Fetches descriptors from memory (256-bit data) 2. Sink AXI Write: Writes sink data to memory (512-bit data, configurable) 3. Source AXI Read: Reads source data from memory (512-bit data, configurable)

## 29.2	Descriptor AXI Master Interface

### 29.2.1	Purpose

Fetches descriptor packets from system memory. Shared across all 8 channels with round-robin arbitration.

### 29.2.2	Signal Table

Table 4.1.1: Descriptor AXI Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_arlen | output | 8 | Burst length (fixed: 0 = 1 beat) |
| m_axi_arsize | output | 3 | Burst size (fixed: 5 = 32 bytes) |
| m_axi_arburst | output | 2 | Burst type (fixed: INCR) |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | 256 | Read data (descriptor) |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rlast | input | 1 | Last beat |

### 29.2.3	Transaction Characteristics

Table 4.1.2: Descriptor AXI Transaction Characteristics

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 256 bits | Fixed descriptor size |
| Burst Length | 1 beat | Single descriptor per transaction |
| Burst Type | INCR | Incrementing burst |
| Max Outstanding | 8 | Configurable via parameter |
| ID Usage | Per-channel | ID encodes source channel |

### 29.2.4	Timing Diagram

### 29.2.5	Figure 4.1.1: Descriptor Fetch Timing

rapids_core_beats - descriptor fetch on src_m_axi_desc_*

Source: axi4_descriptor_fetch.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (source path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1.

Reading it: one single-beat read (arlen = 0) of the 256-bit descriptor at 0x30000000 with arid = channel 0. The AR handshakes when the slave raises arready two cycles after arvalid; the R beat returns two cycles after that with rlast set and rid echoing the channel. The descriptor itself is the 256-bit rdata (its low word, 0x10000000, is the source address).

## 29.3	Sink AXI Write Master Interface

### 29.3.1	Purpose

Writes sink data from SRAM to system memory. Supports burst transactions for efficient memory bandwidth.

### 29.3.2	Signal Table

Table 4.1.3: Sink AXI Write Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awvalid | output | 1 | AW channel valid |
| m_axi_awready | input | 1 | AW channel ready |
| m_axi_awaddr | output | AW | Write address |
| m_axi_awid | output | ID_W | Transaction ID |
| m_axi_awlen | output | 8 | Burst length (0-255) |
| m_axi_awsize | output | 3 | Burst size |
| m_axi_awburst | output | 2 | Burst type |
| m_axi_wvalid | output | 1 | W channel valid |
| m_axi_wready | input | 1 | W channel ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat |
| m_axi_bvalid | input | 1 | B channel valid |
| m_axi_bready | output | 1 | B channel ready |
| m_axi_bid | input | ID_W | Response ID |
| m_axi_bresp | input | 2 | Write response |

### 29.3.3	Transaction Characteristics

Table 4.1.4: Sink AXI Write Transaction Characteristics

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 512 bits | Configurable |
| Burst Length | 1-256 beats | Based on transfer size |
| Burst Type | INCR | Incrementing burst |
| Max Outstanding AW | 8 | Configurable |
| W FIFO Depth | 64 | Configurable |
| B FIFO Depth | 16 | Configurable |

### 29.3.4	Timing Diagram

### 29.3.5	Figure 4.1.2: Sink AXI Write Burst Timing

rapids_core_beats - one 4-beat sink write burst on m_axi_wr_*

Source: axi4_sink_write_burst.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (sink path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1; the memory is the framework AXI4 write slave with its default pacing.

Reading it: the AW (awlen = 3, address 0x20000000) had been held for the slave and handshakes at cycle 3; awaddr steps to the next burst’s 0x20000100 immediately. W starts two cycles later and the four beats go out on the slave’s wready pulses, the low word of wdata counting 0 to 3 and wlast on the fourth; bvalid (OKAY) comes one cycle after the last W handshake.

## 29.4	Source AXI Read Master Interface

### 29.4.1	Purpose

Reads source data from system memory into SRAM. Supports burst transactions for efficient memory bandwidth.

### 29.4.2	Signal Table

Table 4.1.5: Source AXI Read Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | AR channel valid |
| m_axi_arready | input | 1 | AR channel ready |
| m_axi_araddr | output | AW | Read address |
| m_axi_arid | output | ID_W | Transaction ID |
| m_axi_arlen | output | 8 | Burst length (0-255) |
| m_axi_arsize | output | 3 | Burst size |
| m_axi_arburst | output | 2 | Burst type |
| m_axi_rvalid | input | 1 | R channel valid |
| m_axi_rready | output | 1 | R channel ready |
| m_axi_rdata | input | DW | Read data |
| m_axi_rid | input | ID_W | Response ID |
| m_axi_rresp | input | 2 | Read response |
| m_axi_rlast | input | 1 | Last beat |

### 29.4.3	Transaction Characteristics

Table 4.1.6: Source AXI Read Transaction Characteristics

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 512 bits | Configurable |
| Burst Length | 1-256 beats | Based on transfer size |
| Burst Type | INCR | Incrementing burst |
| Max Outstanding AR | 8 | Configurable |
| R FIFO Depth | 64 | Configurable |

### 29.4.4	Timing Diagram

### 29.4.5	Figure 4.1.3: Source AXI Read Burst Timing

rapids_core_beats - one 4-beat source read burst on m_axi_rd_*

Source: axi4_source_read_burst.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (source path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1.

Reading it: AR for four 64-byte beats (arlen = 3, arsize = 6) at 0x10000000 handshakes at cycle 4; the four R beats follow back to back from cycle 6 with rready held high, rresp OKAY and rlast on the fourth. araddr and arlen return to their idle values (the next address, 0x10000100, and 0xff) once the AR is accepted.

## 29.5	AXI Protocol Compliance

### 29.5.1	Supported Features

Table 4.1.7: AXI Feature Support

| Feature | Descriptor | Sink Write | Source Read |
| --- | --- | --- | --- |
| Burst Type INCR | Yes | Yes | Yes |
| Burst Type FIXED | No | No | No |
| Burst Type WRAP | No | No | No |
| Exclusive Access | No | No | No |
| Locked Access | No | No | No |
| Unaligned Access | No | Yes | Yes |
| Narrow Transfers | No | Yes | Yes |

### 29.5.2	Error Handling

- SLVERR: Transaction aborted, error reported to scheduler

- DECERR: Transaction aborted, error reported to scheduler

- Timeout: Configurable watchdog, error reported to scheduler

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	AXI-Stream Interface Specification

Last Updated: 2025-01-10

## 30.1	Overview

The RAPIDS Beats architecture provides optional AXI-Stream (AXIS) wrappers for network interfaces: 1. AXIS Sink Slave: Receives network data for memory writes 2. AXIS Source Master: Transmits network data from memory reads

These wrappers are available when ENABLE_AXIS_WRAPPERS = 1.

## 30.2	AXIS Sink Slave Interface

### 30.2.1	Purpose

Receives streaming network data and routes to per-channel SRAM buffers based on TID.

### 30.2.2	Signal Table

Table 4.2.1: AXIS Sink Slave Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axis_tvalid | input | 1 | Data valid |
| s_axis_tready | output | 1 | Ready to accept data |
| s_axis_tdata | input | DATA_WIDTH | Data payload |
| s_axis_tstrb | input | DATA_WIDTH/8 | Byte enables (not used by the beats design) |
| s_axis_tlast | input | 1 | Last beat of packet |
| s_axis_tid | input | TID_WIDTH | Stream ID (channel select) |
| s_axis_tdest | input | TDEST_WIDTH | Destination routing |
| s_axis_tuser | input | TUSER_WIDTH | User sideband |

### 30.2.3	Signal Details

#### 30.2.3.1	TID (Transaction ID)

TID is used for per-channel routing:

Table 4.2.2: TID Channel Mapping

| TID Value | Destination |
| --- | --- |
| 0 | Channel 0 SRAM |
| 1 | Channel 1 SRAM |
| 2 | Channel 2 SRAM |
| 3 | Channel 3 SRAM |
| 4 | Channel 4 SRAM |
| 5 | Channel 5 SRAM |
| 6 | Channel 6 SRAM |
| 7 | Channel 7 SRAM |

#### 30.2.3.2	TSTRB (Byte Enables)

The ports are s_axis_tstrb / m_axis_tstrb (there is no tkeep). In the beats design they are not load-bearing: the source drives all ones and the sink ignores the ingress strobes, so every beat carries DATA_WIDTH/8 valid bytes. The byte-granular RAPIDS (rtl/top/rapids_top.sv, rapids TASK-019) packs bytes from lane 0 and uses a contiguous tstrb on the last beat of a packet.

Table 4.2.3: TSTRB Configuration

| DATA_WIDTH | TSTRB Width | Beats design | Byte-granular RAPIDS |
| --- | --- | --- | --- |
| 512 bits | 64 bits | all ones | contiguous from lane 0, partial on the last beat |
| 256 bits | 32 bits | all ones | contiguous from lane 0, partial on the last beat |

#### 30.2.3.3	TLAST (Packet Boundary)

TLAST marks the end of an AXIS packet. This is mapped to the internal fill_last signal for packet boundary tracking.

### 30.2.4	Timing Diagram

### 30.2.5	Figure 4.2.1: AXIS Sink Slave Timing

rapids_core_beats - AXIS sink slave timing, 4-beat packet

Source: axis_sink_slave.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (sink path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1; the producer is the framework AXIS master.

Reading it: s_axis_tready is high from the moment the sink has space, so each beat is accepted the cycle it is offered; the producer’s pacing leaves two idle cycles between beats. tstrb is all ones on every beat, tid selects channel 0 and tlast marks the fourth beat as the end of the packet.

### 30.2.6	Backpressure

The sink interface asserts backpressure (s_axis_tready = 0) when: - Target channel SRAM is full - No space allocated for incoming data - Internal pipeline stalls

## 30.3	AXIS Source Master Interface

### 30.3.1	Purpose

Transmits streaming network data from per-channel SRAM buffers with TID indicating source channel.

### 30.3.2	Signal Table

Table 4.2.4: AXIS Source Master Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axis_tvalid | output | 1 | Data valid |
| m_axis_tready | input | 1 | Ready to accept data |
| m_axis_tdata | output | DATA_WIDTH | Data payload |
| m_axis_tstrb | output | DATA_WIDTH/8 | Byte enables (all ones in the beats design) |
| m_axis_tlast | output | 1 | Last beat of packet |
| m_axis_tid | output | TID_WIDTH | Stream ID (source channel) |
| m_axis_tdest | output | TDEST_WIDTH | Destination routing |
| m_axis_tuser | output | TUSER_WIDTH | User sideband |

### 30.3.3	Timing Diagram

### 30.3.4	Figure 4.2.2: AXIS Source Master Timing

rapids_core_beats - AXIS source master timing

Source: axis_source_master.json, captured from dv/tests/top_beats/test_rapids_core_beats.py (source path, channel 0, 4 beats, 512-bit data, REG_LEVEL=GATE) with WAVES=1; the consumer is the framework AXIS slave with its default ready pacing.

Reading it: the master raises m_axis_tvalid and holds it, with tdata, tstrb (all ones), tid and tdest stable, until the consumer’s tready pulse takes the beat; it then presents the next word without dropping tvalid. Four beats leave on four tready pulses. tlast is high on every beat of this capture because the source packetises per drain request and each request carried one word (see Figure 3.7.3); with cfg_drain_size larger than one and data queued, a packet spans several beats and tlast marks only its final one.

### 30.3.5	Flow Control

The source interface respects network backpressure: - Data held when m_axis_tready = 0 - No data loss on backpressure - SRAM drain pauses until ready

## 30.4	AXIS vs Custom Fill/Drain Interfaces

### 30.4.1	Comparison

Table 4.2.5: Interface Comparison

| Aspect | Custom Fill/Drain | AXIS |
| --- | --- | --- |
| Standard | Proprietary | AXI-Stream |
| Channel ID | fill_id / drain_id | TID |
| Packet Boundary | fill_last / drain_last | TLAST |
| Byte Enables | fill_strb / drain_strb | TSTRB |
| Interoperability | RAPIDS only | Industry standard |

### 30.4.2	Selection Guide

Table 4.2.6: Interface Selection Guide

| Use Case | Recommended Interface |
| --- | --- |
| Internal RAPIDS testing | Custom Fill/Drain |
| Network IP integration | AXIS |
| FPGA board-level design | AXIS |
| Maximum performance | Custom Fill/Drain |

## 30.5	Configuration Parameters

// AXIS Interface Parameters
parameter int DATA_WIDTH = 512;       // Data payload width
parameter int TID_WIDTH = 3;          // Stream ID width (log2 channels)
parameter int TDEST_WIDTH = 1;        // Destination width
parameter int TUSER_WIDTH = 1;        // User sideband width

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	MonBus Interface Specification

Last Updated: 2025-01-10

## 31.1	Overview

The MonBus (Monitor Bus) is a 64-bit packet-based interface for reporting internal events, status, and performance data. All RAPIDS subsystems generate MonBus packets that are aggregated and output through a unified interface.

## 31.2	MonBus Signal Interface

### 31.2.1	Signal Table

Table 4.3.1: MonBus Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | output | 1 | Packet valid |
| monbus_pkt_ready | input | 1 | Consumer ready |
| monbus_pkt_data | output | 64 | Packet data |

### 31.2.2	Handshaking Protocol

Standard valid/ready handshaking: - Data transfers when valid & ready both high - Producer holds valid until ready seen - Consumer asserts ready when able to accept

### 31.2.3	Timing Diagram

### 31.2.4	Figure 4.3.1: MonBus Packet Transfer Timing

monbus_axil_group_2in - packets from two producers through the arbiter

Source: monbus_group_packet_transfer.json, captured from dv/tests/macro/test_monbus_axil_group.py (basic_flow, error FIFO 64 x 32, TEST_LEVEL=full) with WAVES=1. The 128-bit packets are shown as their top 24 bits ([127:104]: packet type nibble, then the protocol field at [108:105], so 0x000002 is AXIS and 0x000008 is CORE).

Reading it: a MonBus link is a single-beat valid/ready handshake with the packet presented alongside valid. The group holds ready high on both inputs, so the source and sink packets are accepted the cycle they are offered, and each one reappears on the arbiter output three cycles later as another one-cycle valid/ready pair. Between packets the links idle low; nothing is held across transfers.

## 31.3	MonBus Packet Format

### 31.3.1	64-bit Packet Structure

63      56 55      48 47      40 39      32 31              0
    +--------+---------+---------+---------+-------------------+
    | Type   | Unit ID | Agent ID| Channel | Payload (32 bits) |
    +--------+---------+---------+---------+-------------------+

### 31.3.2	Figure 4.3.2: MonBus Packet Format

Table 4.3.2: MonBus Packet Fields

| Field | Bits | Description |
| --- | --- | --- |
| Type | [63:56] | Packet type code |
| Unit ID | [55:48] | Reporting unit identifier |
| Agent ID | [47:40] | Source agent within unit |
| Channel | [39:32] | Channel number (0-7) |
| Payload | [31:0] | Type-specific payload |

## 31.4	Packet Types

### 31.4.1	Type Code Definitions

Table 4.3.3: MonBus Packet Types

| Type Code | Name | Description |
| --- | --- | --- |
| 0x01 | DESC_FETCH | Descriptor fetch started |
| 0x02 | DESC_COMPLETE | Descriptor fetch completed |
| 0x03 | SCHED_START | Scheduler started transfer |
| 0x04 | SCHED_DONE | Scheduler transfer complete |
| 0x10 | AXI_RD_START | AXI read burst started |
| 0x11 | AXI_RD_DONE | AXI read burst completed |
| 0x12 | AXI_WR_START | AXI write burst started |
| 0x13 | AXI_WR_DONE | AXI write burst completed |
| 0x20 | SRAM_FILL | SRAM fill event |
| 0x21 | SRAM_DRAIN | SRAM drain event |
| 0x30 | ERROR | Error event |
| 0x31 | TIMEOUT | Timeout event |
| 0xFF | HEARTBEAT | Periodic heartbeat |

### 31.4.2	Payload Formats by Type

#### 31.4.2.1	DESC_FETCH / DESC_COMPLETE (0x01, 0x02)

Table 4.3.4: Descriptor Packet Payload

| Bits | Description |
| --- | --- |
| [31:16] | Descriptor address [47:32] |
| [15:0] | Descriptor address [31:16] |

#### 31.4.2.2	SCHED_START / SCHED_DONE (0x03, 0x04)

Table 4.3.5: Scheduler Packet Payload

| Bits | Description |
| --- | --- |
| [31:16] | Transfer ID |
| [15:0] | Beat count |

#### 31.4.2.3	AXI_RD/WR_START/DONE (0x10-0x13)

Table 4.3.6: AXI Event Packet Payload

| Bits | Description |
| --- | --- |
| [31:24] | AXI ID |
| [23:16] | Burst length |
| [15:0] | Reserved |

#### 31.4.2.4	ERROR (0x30)

Table 4.3.7: Error Packet Payload

| Bits | Description |
| --- | --- |
| [31:24] | Error code |
| [23:16] | Error source |
| [15:0] | Error details |

## 31.5	MonBus Source Aggregation

### 31.5.1	RAPIDS Core MonBus Sources

Table 4.3.8: MonBus Source Assignment

| Source ID | Agent | Description |
| --- | --- | --- |
| 0-7 | Scheduler[0:7] | Per-channel scheduler events |
| 8-15 | DescEngine[0:7] | Per-channel descriptor events |
| 16-23 | SnkSRAM[0:7] | Sink SRAM controller events |
| 24-31 | SrcSRAM[0:7] | Source SRAM controller events |
| 32 | AXI Write Engine | Write transaction events |
| 33 | AXI Read Engine | Read transaction events |
| 34 | Error Handler | Error events |

Scheduler and DescEngine COMPLETION packets are subject to the half’s SCHED_CONFIG.COMPL_EN (reset 1): the scheduler group drops them ahead of its arbiter when the bit is clear and acknowledges the emitter, so the data path is unaffected. Their ERROR packets are always forwarded (rapids ISSUE-005).

### 31.5.2	Aggregation Hierarchy

### 31.5.3	Figure 4.3.3: MonBus Aggregation Tree

MonBus aggregation tree

Source: 21_monbus_aggregation.dot

## 31.6	Integration Guidelines

### 31.6.1	Consumer Requirements

MonBus consumers must: 1. Provide sufficient ready cycles to prevent packet loss 2. Handle bursty traffic (multiple sources may have packets simultaneously) 3. Process or buffer packets at wire rate

### 31.6.2	Recommended Consumer Patterns

// Pattern 1: Direct FIFO buffering
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_gaxi_fifo_sync (
    .axi_aclk              (axi_aclk),
    .axi_aresetn           (axi_aresetn),
    .wr_valid              (wr_valid),
    .wr_ready              (wr_ready),
    .wr_data               (wr_data),
    .rd_ready              (rd_ready),
    .count                 (count),
    .rd_valid              (rd_valid),
    .rd_data               (rd_data)
);

## 31.7	Timing Requirements

Table 4.3.9: MonBus Timing Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Min Ready | 1 cycle | Must assert ready within N cycles |
| Max Latency | Configurable | Before packet dropped |
| Packet Rate | Variable | Depends on activity |

Last Updated: 2025-01-10

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 32	CXS Interface Specification

Applies to: rapids_beats_cxs_top, rapids_cxs_top Protocol: Arm IHI 0079 (CXS) Issue D, one fixed profile Last Updated: 2026-10-16

## 32.1	Overview

The CXS tops replace the two AXIS network ports with one CXS link per direction: the sink port s_cxs_* (a CXS receiver) into the sink half and the source port m_cxs_* (a CXS transmitter) out of the source half. One link carries every channel; the channel rides in CXSSRCID. The planes that implement the links are described in the Network Planes chapter (Section 3.13).

## 32.2	Link Profile

Table 4.4.1: CXS Link Profile

| Property | Value |
| --- | --- |
| Issue | D (the issue with SRCID, TGTID and USER) |
| CXSMAXPKTPERFLIT | 2 |
| Packet start alignment | 16 bytes; RAPIDS always starts a packet at byte 0 |
| Packet mapping | One AXIS packet (tlast-delimited) is one CXS packet |
| CXSSRCID | The channel (AXIS tid) |
| CXSTGTID | AXIS tdest |
| CXSUSER | {tuser, tstrb}; tuser dropped when DATA_WIDTH/8 + AXIS_USER_WIDTH > 128 |
| Stream framing | Per stream on the channel bits of CXSSRCID |
| Link control | Explicit credit return, activation, deactivation |
| Parity | Off |

## 32.3	Signal Table

Table 4.4.2: CXS Interface Signals

| Signal | Sink port | Source port | Width | Description |
| --- | --- | --- | --- | --- |
| {s,m}_cxs_valid | in | out | 1 | Flit valid |
| {s,m}_cxs_data | in | out | DATA_WIDTH | Flit data |
| {s,m}_cxs_cntl | in | out | CXS_CNTL_PW | START / END / pointers |
| {s,m}_cxs_srcid | in | out | AXIS_ID_WIDTH | Channel |
| {s,m}_cxs_tgtid | in | out | AXIS_DEST_WIDTH | tdest |
| {s,m}_cxs_user | in | out | CXS_UW | {tuser, tstrb} |
| {s,m}_cxs_crdgnt | out | in | 1 | Credit grant (receiver to transmitter) |
| {s,m}_cxs_crdrtn | in | out | 1 | Credit return (transmitter to receiver) |
| {s,m}_cxs_activereq | in | out | 1 | Transmitter requests the link |
| {s,m}_cxs_activeack | out | in | 1 | Receiver acknowledges |
| {s,m}_cxs_deacthint | out | in | 1 | Receiver asks the transmitter to deactivate |
| snk_cxs_link_state, src_cxs_link_state | out | out | 2 | {ACTIVEREQ, ACTIVEACK} |
| snk_cxs_protocol_error, src_cxs_protocol_error | out | out | 1 | Sticky protocol violation |
| snk_cxs_unsupported | out | - | 1 | Sticky: a flit AXIS framing cannot carry |

## 32.4	Credit Flow

One credit permits one flit (IHI 0079D 2.1). The receiver grants up to CXS_MAX_CREDIT (default 32) credits, one per free entry in its receive FIFO, so every flit sent against a credit has space. The transmitter spends one credit per flit and returns the credits it still holds on deactivation. Credits are a single pool for the link; a grant carries no ID and cannot be reserved for a channel.

The grant -> flit -> free round trip between the RAPIDS endpoints is about 3 cycles, so 32 credits sustains one flit per cycle with margin for pipeline stages or a clock crossing on the link. More than 63 credits is beyond the spec and requires CXS_CREDIT_BEYOND_SPEC on both ends.

## 32.5	Link Activation

Table 4.4.3: Link Activation Sequence

| Step | Transmitter (m_cxs_*) | Receiver (s_cxs_*) |
| --- | --- | --- |
| 1 | Raises ACTIVEREQ when it has a flit to send (and only once the previous activation’s ACTIVEACK has fallen: 4-phase) | - |
| 2 | - | Raises ACTIVEACK; starts granting credits |
| 3 | Sends flits, one credit each | Frees an entry and regrants as the core drains |
| 4 | After CXS_IDLE_CYCLES idle cycles in RUN (or on DEACTHINT) drops ACTIVEREQ, then returns every held credit on CRDRTN, one per cycle | - |
| 5 | - | Drops ACTIVEACK once its credits are back |

## 32.6	Backpressure

On the sink port, a full receive FIFO means no further grants: the transmitter stalls for lack of credit. This is the CXS form of the AXIS sink’s s_axis_tready = 0, including the byte-granular sink’s hold until a channel’s descriptor has been fetched. On the source port, the plane stalls the core’s egress (fub_axis_tready = 0) while it has no credit.

## 32.7	Proposed Changes

A proposal (2026-10-16, not implemented) puts a tile packet header at the front of every packet: destination and source coordinates, payload type (cfg / prog / mem), length and target address. It is described once in the compute-engine tile’s TILE_SPEC.md Section 11; CXS carries it as the first flit. The link signals, credits and per-stream framing on this page do not change, because CXSSRCID keeps the channel.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 33	Product Requirements Document (PRD)

## 33.1	RAPIDS - Rapid AXI Programmable In-band Descriptor System

Version: 1.0 Date: 2025-09-30 Status: Active Development - Validation in Progress Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 33.2	1. Executive Summary

The Rapid AXI Programmable In-band Descriptor System (RAPIDS) is a custom hardware accelerator designed for efficient memory-to-memory data movement with network interface integration. It demonstrates complex FSM coordination, descriptor-based DMA operations, and comprehensive monitoring capabilities.

Status (2026-07-22): RAPIDS was rearchitected to the “beats” design. Current RTL is in rtl/fub_beats/, rtl/macro_beats/, rtl/top_beats/ (control engines in rtl/fub/); current specs are docs/rapids_beats_has/ and docs/rapids_beats_mas/. The old rapids_fub//rapids_macro/ RTL, the docs/rapids_spec/ tree, and the program engine / network master / network slave blocks described in parts of this PRD were retired (network interfaces are now AXIS; control is APB). Sections describing the pre-beats design are kept as history and marked where practical.

Status (2026-09-29, rapids TASK-019): RAPIDS is byte-granular. The product RTL is the un-suffixed tree rtl/fub/, rtl/macro/, rtl/top/rapids_top.sv: descriptor length in BYTES, byte-granular addresses, WSTRB/TSTRB carry partial beats, one AXIS packet per descriptor. The *_beats tree is the beat-granular stepping stone, kept runnable and green as the characterized Genesys 2 design. Tests: dv/tests/fub, macro, top.

### 33.2.1	1.1 Quick Stats

- Modules: ~20 SystemVerilog files (beats architecture)

- Architecture: 3 major groups (Scheduler group, Sink path, Source path)

- Interfaces: AXI4 (memory), AXIS (network), APB (control), MonBus (monitoring)

- Test Coverage: see dv/tests/ beats suites (fub_beats/macro_beats/top_beats)

- Status: Active validation, known issues documented

### 33.2.2	1.2 Subsystem Goals

- Primary: Demonstrate complex accelerator design patterns

- Secondary: Provide DMA-style memory transfer capability

- Tertiary: Educational reference for descriptor-based engines

## 33.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 33.3.1	Complete RAPIDS Specification

Status (2026-07-22): The old docs/rapids_spec/ tree was replaced by two beats spec trees.

Byte-granular RAPIDS books (the product design): HAS Index, MAS Index. Chapters that are identical for both designs are linked from the Beats books below.

Architecture Spec (HAS), RAPIDS Beats: docs/rapids_beats_has/

- HAS Index - Complete HAS structure

- Product Overview

- Block Diagram / Data Flow / Channel Architecture

- Interfaces: Summary, AXI4, AXIS, APB, MonBus

- Use cases: docs/rapids_beats_has/ch04_use_cases/

- Programming: Descriptor Format, Register Map, Initialization, Error Handling

- Performance: docs/rapids_beats_has/ch06_performance/

Micro-Architecture Spec (MAS): docs/rapids_beats_mas/

- MAS Index - Complete MAS structure

- Overview: Architecture, Port List, Clocks and Reset

FUB blocks (docs/rapids_beats_mas/ch02_fub_blocks/): - Scheduler - Per-channel scheduler FSM - Descriptor Engine - Descriptor fetch/parsing - AXI Read Engine / AXI Write Engine - Alloc Ctrl / Drain Ctrl / Latency Bridge - CtrlRd Engine / CtrlWr Engine

Macro blocks (docs/rapids_beats_mas/ch03_macro_blocks/): - Scheduler Group / Scheduler Group Array - Sink Data Path + AXIS wrapper, SRAM controllers - Source Data Path + AXIS wrapper, SRAM controllers - RAPIDS Core / Registers / Top

Interfaces (docs/rapids_beats_mas/ch04_interfaces/): - AXI4 Interface - AXIS Interface - MonBus Interface

### 33.3.2	Known Issues

Location: projects/components/dma-ip/rapids/known_issues/

- Index - Issue tracking overview (the old scheduler.md credit-counter write-up was retired with the pre-beats RTL)

- Sink Data Path - RESOLVED 2026-09-25 (write-error flag wired; timeouts are the monitor’s job by design)

- Sink SRAM Control - RESOLVED (moved to resolved/ in d587852c2)

- Plus known_issues/active/ for current beats issues

### 33.3.3	Other Documentation

- CLAUDE - AI assistance guide for this subsystem

- TASKS - Work items (largely pre-beats history)

- Validation Report - Test results (pre-beats snapshot)

## 33.4	2.4 Organizational Standards - RAPIDS Code Location

MANDATORY: All RAPIDS-specific code must be in the project area

### 33.4.1	Code Organization Principle

“All RAPIDS-specific verification code MUST reside in projects/components/dma-ip/rapids/dv/ for easy discovery.”

This subsystem follows the repository-wide organizational standard (see /PRD.md Section 2.3) requiring all project-specific code to be located in the project area, NOT the framework area.

### 33.4.2	RAPIDS Directory Structure

projects/components/dma-ip/rapids/
├── rtl/                          # RTL source code
│   ├── includes/                 # RAPIDS packages
│   ├── fub/                      # Control engines (ctrlrd/ctrlwr)
│   ├── fub_beats/                # Beats functional unit blocks
│   ├── macro_beats/              # Beats assemblies + registers
│   ├── macro/                    # MonBus group
│   └── top_beats/                # rapids_beats_top.sv
│
└── dv/                           # Design verification (all RAPIDS-specific)
    ├── tbclasses/                # RAPIDS TB classes HERE (not framework!)
    │   ├── scheduler_tb.py       # Scheduler testbench class
    │   ├── descriptor_engine_tb.py
    │   └── rapids_core_beats_tb.py
    │
    ├── components/               # RAPIDS-specific BFMs
    │   └── data_mover_bfm.py
    │
    └── tests/                    # Test runners (import TB classes)
        ├── fub/                  # Control engine tests
        ├── fub_beats/            # Beats FUB tests (test_scheduler_beats.py, ...)
        ├── macro/                # MonBus group test
        ├── macro_beats/          # Multi-block scenarios
        └── top_beats/            # Full RAPIDS operation

### 33.4.3	What Goes Where?

| Code Type | CORRECT Location | WRONG Location |
| --- | --- | --- |
| RAPIDS TB Classes | projects/components/dma-ip/rapids/dv/tbclasses/ | bin/TBClasses/ (framework area) |
| RAPIDS-Specific BFMs | projects/components/dma-ip/rapids/dv/components/ | bin/TBClasses/ (framework area) |
| RAPIDS Scoreboards | projects/components/dma-ip/rapids/dv/ (project area) | bin/TBClasses/scoreboards/ |
| Test Runners | projects/components/dma-ip/rapids/dv/tests/ | Anywhere else |
| Shared AXI4/APB BFMs | bin/TBClasses/{protocol}/ | Project area |

### 33.4.4	Import Pattern for RAPIDS Tests

CORRECT - Import from Project Area:

# Import framework utilities (PYTHONPATH includes bin/)
import os, sys
from TBClasses.shared.utilities import get_repo_root
from TBClasses.shared.tbbase import TBBase

# Add repo root to Python path using robust git-based method
repo_root = get_repo_root()
sys.path.insert(0, repo_root)

# Import RAPIDS TB classes from PROJECT AREA
from projects.components.dma_ip.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from projects.components.dma_ip.rapids.dv.tbclasses.descriptor_engine_tb import DescriptorEngineTB

# Shared framework components
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

WRONG - Don’t Import from Framework:

# DON'T DO THIS!
from TBClasses.rapids.scheduler_tb import SchedulerTB  # WRONG!

### 33.4.5	Benefits of This Organization

- Easy Discovery - All RAPIDS code in ONE place: projects/components/dma-ip/rapids/

- Clear Ownership - RAPIDS team owns their dv/ area completely

- No Confusion - Never wonder “where does this TB class live?”

- Maintainability - Changes isolated to RAPIDS area don’t affect other projects

- Framework Stays Clean - Only truly shared cross-project code in framework

### 33.4.6	Compliance Status

RAPIDS is now compliant - All TB classes moved to project area as of 2025-10-18

Migration History: - Before: TB classes incorrectly in bin/TBClasses/rapids/ - After: TB classes correctly in projects/components/dma-ip/rapids/dv/tbclasses/ - Test Imports: Updated to import from project area

Complete Documentation: See /PRD.md Section 2.3 for repository-wide organizational standards.

## 33.5	3. Architecture Overview

### 33.5.1	3.1 Top-Level Block Diagram

RAPIDS Beats (Rapid AXI Programmable In-band Descriptor System)
├── Scheduler Group (scheduler_group_beats / scheduler_group_array_beats)
│   ├── Scheduler          (Per-channel FSM, scheduler_beats.sv)
│   ├── Descriptor Engine  (Descriptor fetch/parsing, descriptor_engine_beats.sv)
│   └── Control Engines    (ctrlrd_engine.sv / ctrlwr_engine.sv)
│
├── Sink Data Path (AXIS Network → SRAM → System Memory)
│   ├── AXIS Slave ingress   (snk_data_path_axis_beats.sv)
│   ├── Sink SRAM Controller (snk_sram_controller_beats.sv + alloc/drain ctrl)
│   └── AXI Write Engine     (axi_write_engine_beats.sv)
│
├── Source Data Path (System Memory → SRAM → AXIS Network)
│   ├── AXI Read Engine        (axi_read_engine_beats.sv)
│   ├── Source SRAM Controller (src_sram_controller_beats.sv + alloc/drain ctrl)
│   └── AXIS Master egress     (src_data_path_axis_beats.sv)
│
└── Control and Monitoring
    ├── APB CSRs           (rapids_config_block.sv, PeakRDL rapids_regs.rdl)
    └── MonBus Group       (macro/monbus_axil_group_2in.sv)

See: docs/rapids_beats_mas/ch01_overview/01_architecture.md for detailed architecture

### 33.5.2	3.2 Data Flow

Sink Path (Receive): 1. Network packets arrive via Network Slave 2. Buffered in Sink SRAM 3. DMA’d to system memory via AXI4 Write Engine 4. Completion reported via MonBus

Source Path (Transmit): 1. Descriptor specifies data location in system memory 2. Source AXI Reader fetches data to Source SRAM 3. Network Master transmits to network 4. Completion reported via MonBus

Scheduler Coordination: - Parses descriptors from Descriptor Engine - Manages credit-based flow control - Sequences program engine operations - Coordinates sink/source data paths

## 33.6	4. Key Features

### 33.6.1	4.1 Descriptor-Based Operation

| Feature | Status | Description |
| --- | --- | --- |
| Descriptor FIFO | yes | Queued descriptor processing |
| Multi-field parsing | yes | Address, length, control fields |
| Chained descriptors | ⏳ | Future enhancement |
| Completion reporting | yes | Via MonBus packets |

### 33.6.2	4.2 Data Path Features

| Feature | Status | Description |
| --- | --- | --- |
| SRAM buffering | yes | Decouple network from memory |
| AXI4 burst support | yes | Efficient memory transfers |
| Backpressure handling | yes | Flow control on all interfaces |
| Data alignment | yes | Handle unaligned transfers |

### 33.6.3	4.3 Scheduler Features

| Feature | Status | Description |
| --- | --- | --- |
| Task FSM | yes | Multi-state coordination |
| Credit management | ⏳ | Pre-beats scheduler had exponential encoding (0→1, 1→2, …, 15→∞); scheduler_beats.sv has no credit management yet (planned later phase) |
| Program sequencing | yes | Coordinated operations |
| Error detection | yes | Timeout, overflow detection |

Credit Management Details (pre-beats, historical): - Uses exponential credit encoding for compact configuration - 4-bit cfg_initial_credit decodes to actual credit counts: - 0 → 1 credit (2^0), 4 → 16 credits (2^4), 8 → 256 credits (2^8) - 15 → ∞ (unlimited credits, 0xFFFFFFFF) - Encoding applied at initialization; runtime operations are linear (increment/decrement by 1) - Provides wide range (1 to 16384) with minimal configuration overhead

See: docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md for the current scheduler specification

rapids-g (2026-10-21, rapids TASK-034): a build of the byte-granular RAPIDS whose schedulers are sequenced by firmware on one gatling per half (projects/components/riscv-ip/gatling-rv32itetrad), sharing the scheduler datapath (rtl/fub/scheduler_core.sv) with the FSM scheduler. The firmware makes the channel decisions and computes the beat totals and the EXT run bases; it pre-validates the next descriptor while the current one runs. Register-compatible with RAPIDS; 4 or 8 channels per half, SHARED_ENGINES = 1. Trees: rtl/fub_g/, rtl/macro_g/, rtl/top_g/ (the macro and top files generated by bin/gen_rapids_g.py), firmware rtl/fw_g/sched_g.S. Specification: RAPIDS MAS Chapter 5; measured cost: MAS 5.5. Owner direction: the next-address algorithm becomes selectable by an 8-bit descriptor field (rapids TASK-035); custom gatling instructions for DMA (gatling_dma) if the firmware cost calls for them.

### 33.6.4	4.4 Monitoring Integration

| Feature | Status | Description |
| --- | --- | --- |
| MonBus packets | yes | StandardAMBA 64-bit format |
| Descriptor events | yes | Start/complete reporting |
| Error events | yes | Timeout, overflow, underflow |
| Performance metrics | ⏳ | Future enhancement |

## 33.7	5. Interfaces

### 33.7.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| APB | Slave | 32-bit | Control/status registers |
| AXI4 (Sink) | Master | Configurable | Write to system memory |
| AXI4 (Source) | Master | Configurable | Read from system memory |
| AXIS (Sink) | Slave | Configurable | Network ingress (tid = channel) |
| AXIS (Source) | Master | Configurable | Network egress (tid = channel) |
| MonBus | Master | 64-bit | Monitor packet output |

See: docs/rapids_beats_mas/ch04_interfaces/ and docs/rapids_beats_has/ch03_interfaces/ for complete interface specs

### 33.7.2	5.2 Configuration Parameters

// Example RAPIDS instantiation sketch (see rtl/top_beats/rapids_beats_top.sv for the
// full parameter/port list)
rapids_beats_top #(
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .NUM_CHANNELS(8)
) u_rapids (
    .clk                (clk),
    .rst_n              (rst_n),
    // APB control interface
    .s_apb_*            (...),
    // AXI4 memory interfaces (read + write masters)
    .m_axi_*            (...),
    // AXIS network interfaces (tid = channel)
    .s_axis_*           (...),
    .m_axis_*           (...),
    // MonBus / monitor group egress
    .mon_*              (...)
);

## 33.8	6. Use Cases

### 33.8.1	6.1 DMA-Style Transfers

Scenario: Move data from network to system memory

Flow: 1. Software writes descriptor to Descriptor Engine 2. Scheduler parses descriptor, activates Sink path 3. Network packets arrive via Network Slave 4. Data buffered in Sink SRAM 5. AXI4 Write Engine DMAs to system memory 6. Completion packet on MonBus

### 33.8.2	6.2 Network Packet Processing

Scenario: Read data from memory, transmit to network

Flow: 1. Descriptor specifies source address, length 2. Source AXI Reader fetches data to SRAM 3. Network Master transmits to network 4. Completion/error reporting via MonBus

### 33.8.3	6.3 Custom Data Path Acceleration

Educational value: Shows how to build custom accelerators - Descriptor-based control - Multi-block FSM coordination - Buffering strategies - Error handling - Performance monitoring

## 33.9	7. Test Coverage

### 33.9.1	7.1 Current Status

Status (2026-07-22): The numbers below are a pre-beats snapshot (they include the retired program engine). Current beats regression: fub 232+ and macro 182 tests passing; run the beats suites for live numbers.

Overall (pre-beats snapshot): ~85% functional coverage (basic scenarios validated, descriptor engine complete)

| Component | Test Coverage | Status |
| --- | --- | --- |
| Scheduler | ~95% | Credit encoding fixed and verified (43/43 tests passing) |
| Descriptor Engine | 100% | All tests passing (14/14 tests, 100% success rate) |
| Program Engine | ~85% | Alignment tested |
| Sink Data Path | ~75% | Basic flows working |
| Source Data Path | ~70% | Basic flows working |
| SRAM Controllers | ~80% | Buffer management tested |
| Integration | ~60% | More stress testing needed |

Test Location: projects/components/dma-ip/rapids/dv/tests/fub_beats/ and projects/components/dma-ip/rapids/dv/tests/macro_beats/ (plus fub/, macro/, top_beats/)

Recent Achievements: - Descriptor Engine (2025-10-13): Achieved 100% test pass rate using continuous background monitoring pattern - 14/14 tests passing across all test levels (basic, medium, full) - All test classes passing (APB_ONLY, MIXED) - All delay profiles passing (fast_producer, fast_consumer, fixed_delay, minimal_delay) - Applied continuous monitoring methodology for asynchronous output capture

See: vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md for detailed results

### 33.9.2	7.2 Test Strategy

FUB (Functional Unit Block) Tests: - Individual block testing - Located in projects/components/dma-ip/rapids/dv/tests/fub_beats/ (+ fub/ for control engines) - Focus on module-level functionality

Macro Tests: - Multi-block scenarios - Located in projects/components/dma-ip/rapids/dv/tests/macro_beats/ (+ macro/ for the monbus group) - End-to-end data flow validation

Top Tests: - Full RAPIDS operation - Located in projects/components/dma-ip/rapids/dv/tests/top_beats/ - Realistic traffic patterns

## 33.10	8. Known Issues Summary

### 33.10.1	8.1 Critical Issues

Scheduler Credit Counter Initialization - FIXED (2025-10-11, pre-beats) - File: the retired pre-beats scheduler.sv (replaced by rtl/fub_beats/scheduler_beats.sv, which currently has no credit management - planned for a later phase) - Issue: Credit counter was initializing to 0 instead of using exponential encoding - Fix Applied: Implemented exponential credit encoding - Status: Fixed and verified pre-beats; the known_issues/scheduler.md write-up was retired with that RTL (see known_issues/README.md)

Fix Details:

// Previous (wrong):
r_descriptor_credit_counter <= 32'h0;

// Fixed - Exponential encoding:
// 0→1, 1→2, 2→4, 3→8, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Encoding Rationale: - Compact 4-bit configuration covers 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞) - Exponential encoding applied at initialization only; runtime operations are linear

### 33.10.2	8.2 Medium Priority Issues

Descriptor Engine Edge Cases: - Some stress test failures under high load - Edge case handling needs improvement - Priority: P2

SRAM Control Timing: - Rare timing issues in back-to-back operations - Priority: P2

See: known_issues/ directory for complete issue tracking

## 33.11	9. Integration Guidelines

### 33.11.1	9.1 Quick Start

rapids_beats_top #(
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .NUM_CHANNELS(8)
) u_rapids (
    // Clocking & Reset
    .clk                (system_clk),
    .rst_n              (system_rst_n),

    // APB Control (connect to control fabric)
    .s_apb_paddr        (ctrl_paddr),
    .s_apb_psel         (ctrl_psel),
    .s_apb_penable      (ctrl_penable),
    // ... other APB signals

    // AXI4 Memory (connect to memory controller)
    // ... AXI write channels (sink), AXI read channels (source)

    // AXIS Network (connect to network fabric; tid = channel)
    .s_axis_tdata       (net_rx_data),
    .s_axis_tvalid      (net_rx_valid),
    // ... AXIS slave (receive), AXIS master (transmit)

    // Monitor group egress (AXI-Lite drain / IRQ)
    // ... mon_* signals
);

See rtl/top_beats/rapids_beats_top.sv for the complete, authoritative port list.

### 33.11.2	9.2 Configuration Steps

- Initialize via APB registers (use the generated regmap, access registers by name):

  - Configure channel enables and thresholds

  - Set timeout thresholds

- Load Descriptors:

  - Write descriptors to Descriptor Engine FIFO

  - Each descriptor specifies: address, length, control bits

- Enable Operation:

  - Set enable bits via APB registers

  - Monitor MonBus for completion/error packets

See: docs/rapids_beats_has/ch05_programming/ for register and programming details

## 33.12	10. Development Status

### 33.12.1	10.1 Current Phase

Phase: Validation and Bug Fixing (In Progress)

- Core architecture implemented

- Basic functionality verified

- Scheduler credit counter bug fixed (exponential encoding implemented)

- ⏳ Credit management tests need verification (remove workarounds)

- ⏳ Stress testing ongoing

- ⏳ Edge case refinement

See: TASKS.md for detailed work items

### 33.12.2	10.2 Roadmap

Near-Term (Q4 2025): - Fix scheduler credit counter bug (completed 2025-10-11) - ⏳ Verify credit management tests (remove workarounds) - ⏳ Complete descriptor engine stress testing - ⏳ Integration test suite expansion - ⏳ Performance benchmarking

Long-Term (2026+): - Chained descriptor support - Advanced error recovery - Performance optimizations - Multi-channel support

## 33.13	11. Performance Characteristics

### 33.13.1	11.1 Throughput

Target: Match network/memory interface bandwidth

Bottlenecks: - SRAM buffer size - AXI4 burst efficiency - Scheduler overhead

Optimization: - Increase SRAM depth for larger packets - Tune AXI4 burst parameters - Pipeline scheduler operations

### 33.13.2	11.2 Latency

Components: - Descriptor parsing: ~10 cycles - SRAM buffering: Configurable depth - AXI4 memory access: System dependent - End-to-end: Typically <100 cycles for small packets

### 33.13.3	11.3 Resource Utilization

Area: - Scheduler: ~2K LUTs - Each data path: ~3K LUTs - SRAM buffers: Configurable (dominant area) - Total: ~10K LUTs + SRAM

Power: - Clock gating opportunities in idle blocks - SRAM power depends on depth/width

## 33.14	12. Verification Infrastructure

### 33.14.1	12.1 Test Organization

Location: projects/components/dma-ip/rapids/dv/tests/

Structure:

projects/components/dma-ip/rapids/dv/tests/
├── fub/                    # Control engine tests (ctrlrd/ctrlwr)
├── fub_beats/              # Beats FUB tests (scheduler, descriptor engine,
│                           #   alloc/drain ctrl, latency bridge)
├── macro/                  # MonBus group test
├── macro_beats/            # Multi-block scenarios (scheduler group, data paths,
│                           #   SRAM controllers)
└── top_beats/              # Full RAPIDS operation

### 33.14.2	12.2 CocoTB Framework

Location: projects/components/dma-ip/rapids/dv/tbclasses/ (RAPIDS-specific) plus shared BFMs in bin/TBClasses/

Components: - RAPIDS-specific drivers - Descriptor generators - Traffic patterns - Monitor checkers

See: ../RTLDesignSherpa-DV/docs/components/<family>/ for shared framework docs

### 33.14.3	12.2.1 MANDATORY: BFM Usage for FUB Tests

CRITICAL DESIGN REQUIREMENT

All RAPIDS FUB (Functional Unit Block) level tests MUST use CocoTB Framework BFMs. Manual handshake driving is NOT allowed.

Required BFM Components:

| Interface Type | Framework Location | BFM Component |
| --- | --- | --- |
| Custom valid/ready | bin/TBClasses/gaxi/ | GAXI Master/Slave |
| AXI4 | bin/TBClasses/axi4/ | AXI4 Master/Slave |
| AXI4-Lite (AXIL) | bin/TBClasses/axil4/ | AXIL Master/Slave |
| APB | bin/TBClasses/apb/ | APB Master/Slave |
| AXI-Stream (AXIS) | bin/TBClasses/axis4/ | AXIS Master/Slave (RAPIDS network interfaces are AXIS) |
| MonBus | bin/TBClasses/monbus/ | MonBus drivers |

Rationale: 1. Consistency: All tests use standardized handshake protocols 2. Correctness: BFMs handle complex timing scenarios (backpressure, randomization) 3. Reusability: Same BFM across all RAPIDS tests 4. Maintainability: Fix once in BFM, all tests benefit 5. Coverage: BFMs include comprehensive timing profiles

Example - Program Engine:

# WRONG: Manual handshake (violates design requirement)
async def send_request(self, addr, data):
    self.dut.program_valid.value = 1
    self.dut.program_pkt_addr.value = addr
    # ... manual handshaking logic ...

# CORRECT: Use GAXI Master BFM
from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster

class ProgramEngineTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.program_master = GAXIMaster(
            dut=dut,
            clock=dut.clk,
            valid_signal='program_valid',
            ready_signal='program_ready',
            data_signals=['program_pkt_addr', 'program_pkt_data'],
            data_widths=[64, 32]
        )

    async def send_request(self, addr, data):
        await self.program_master.write({'program_pkt_addr': addr, 'program_pkt_data': data})

See: - projects/components/dma-ip/rapids/CLAUDE.md - Rule #1 for complete BFM usage guidelines - ../RTLDesignSherpa-DV/docs/components/gaxi/ - GAXI BFM documentation - bin/TBClasses/axi4/ - AXI4 BFM sources (full framework docs in the RTLDesignSherpa-DV repo)

### 33.14.4	12.3 Test File Structure (Standard Pattern)

MANDATORY: All RAPIDS tests must follow this structure

RAPIDS tests follow the same pattern as AMBA tests for consistency across the repository:

# Example: projects/components/dma-ip/rapids/dv/tests/fub_beats/test_scheduler_beats.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# Import REUSABLE testbench class (NOT defined in this file!)
from projects.components.dma_ip.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic_flow(dut):
    """Test basic descriptor flow."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_basic_descriptor_flow()
    assert result, "Basic descriptor flow test failed"

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_credit_encoding(dut):
    """Test exponential credit encoding."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_exponential_encoding_all_values()
    assert result, "Credit encoding test failed"

# ===========================================================================
# PARAMETER GENERATION - at bottom of file
# ===========================================================================

def generate_scheduler_test_params():
    """Generate test parameters for scheduler tests."""
    return [
        # (channel_id, num_channels, data_width, credit_width)
        (0, 8, 512, 8),  # Standard configuration
        # Add more parameter sets as needed
    ]

scheduler_params = generate_scheduler_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - at bottom of file
# ===========================================================================

@pytest.mark.parametrize("channel_id, num_channels, data_width, credit_width", scheduler_params)
def test_basic_flow(request, channel_id, num_channels, data_width, credit_width):
    """
    Scheduler basic flow test.

    Run with: pytest projects/components/dma-ip/rapids/dv/tests/fub_beats/test_scheduler_beats.py::test_basic_flow -v
    """
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_fub_beats': '../../rtl/fub_beats'
    })

    dut_name = "scheduler_beats"
    toplevel = dut_name

    verilog_sources = [
        os.path.join(repo_root, 'rtl', 'amba', 'includes', 'ocelus_pkg.sv'),
        # RAPIDS package(s) from the project includes area
        # (prefer get_sources_from_filelist() with rtl/filelists/ in real tests)
        os.path.join(rtl_dict['rtl_fub_beats'], '..', 'includes', 'rapids_pkg.sv'),
        os.path.join(rtl_dict['rtl_fub_beats'], f'{dut_name}.sv'),
    ]

    # Format parameters for unique test name
    cid_str = TBBase.format_dec(channel_id, 2)
    nc_str = TBBase.format_dec(num_channels, 2)
    dw_str = TBBase.format_dec(data_width, 4)
    cw_str = TBBase.format_dec(credit_width, 2)
    test_name_plus_params = f"test_{dut_name}_cid{cid_str}_nc{nc_str}_dw{dw_str}_cw{cw_str}"

    # Add worker ID for pytest-xdist parallel execution
    worker_id = os.environ.get('PYTEST_XDIST_WORKER', '')
    if worker_id:
        test_name_plus_params = f"{test_name_plus_params}_{worker_id}"

    log_path = os.path.join(log_dir, f'{test_name_plus_params}.log')
    sim_build = os.path.join(tests_dir, 'local_sim_build', test_name_plus_params)
    os.makedirs(sim_build, exist_ok=True)
    os.makedirs(log_dir, exist_ok=True)

    rtl_parameters = {
        'CHANNEL_ID': channel_id,
        'NUM_CHANNELS': num_channels,
        'DATA_WIDTH': data_width,
        'CREDIT_WIDTH': credit_width,
        # Add other RTL parameters as needed
    }

    extra_env = {
        'LOG_PATH': log_path,
        'TEST_CHANNEL_ID': str(channel_id),
        'TEST_NUM_CHANNELS': str(num_channels),
        'TEST_DATA_WIDTH': str(data_width),
    }

    compile_args = ["-Wno-TIMESCALEMOD"]
    sim_args = []
    plusargs = []

    cmd_filename = create_view_cmd(log_dir, log_path, sim_build, module, test_name_plus_params)

    try:
        run(
            python_search=[tests_dir],
            verilog_sources=verilog_sources,
            includes=[
                os.path.join(repo_root, 'projects', 'components', 'dma-ip', 'rapids', 'rtl', 'includes'),
                os.path.join(repo_root, 'rtl', 'amba', 'includes'),
            ],
            toplevel=toplevel,
            module=module,
            testcase="cocotb_test_basic_flow",  # ← cocotb test function name
            parameters=rtl_parameters,
            sim_build=sim_build,
            extra_env=extra_env,
            waves=False,
            keep_files=True,
            compile_args=compile_args,
            sim_args=sim_args,
            plusargs=plusargs,
        )

        print(f"Scheduler basic flow test completed!")
        print(f"Logs: {log_path}")

    except Exception as e:
        print(f"Scheduler basic flow test failed: {str(e)}")
        print(f"Logs preserved at: {log_path}")
        raise

Key Structure Requirements:

- Testbench Class Location:

  - ALWAYS in projects/components/dma-ip/rapids/dv/tbclasses/

  - NEVER inline in test file

  - Reusable across multiple test files

- CocoTB Test Functions:

  - Prefix with cocotb_ to prevent pytest collection

  - Located at top of test file

  - Use @cocotb.test() decorator

  - Call testbench methods

- Parameter Generation:

  - Function returns list of parameter tuples

  - Located near bottom of file (before pytest wrappers)

  - Stored in variable (e.g., scheduler_params)

- Pytest Wrapper Functions:

  - Located at bottom of file

  - Use @pytest.mark.parametrize() with parameter variable

  - Build unique test names with TBBase.format_dec()

  - Call run() with testcase="cocotb_test_function_name"

  - Handle parallel execution (PYTEST_XDIST_WORKER)

- Mandatory TB Methods:

  - async def setup_clocks_and_reset(self) - Complete initialization

  - async def assert_reset(self) - Assert reset signal(s)

  - async def deassert_reset(self) - Deassert reset signal(s)

See: - val/amba/test_apb4_slave.py - Reference example - projects/components/dma-ip/rapids/CLAUDE.md - Detailed TB requirements

## 33.15	13. Quick Reference

### 33.15.1	13.1 Key Files

| File | Purpose |
| --- | --- |
| projects/components/dma-ip/rapids/PRD.md | This document (overview) |
| projects/components/dma-ip/rapids/CLAUDE.md | AI assistance guide |
| projects/components/dma-ip/rapids/TASKS.md | Work items (largely pre-beats history) |
| projects/components/dma-ip/rapids/docs/rapids_beats_has/ | Architecture specification |
| projects/components/dma-ip/rapids/docs/rapids_beats_mas/ | Micro-architecture specification |
| projects/components/dma-ip/rapids/known_issues/ | Bug tracking |
| vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md | Test results (pre-beats snapshot) |

### 33.15.2	13.2 Commands

# Run RAPIDS tests
pytest projects/components/dma-ip/rapids/dv/tests/fub_beats/ -v     # Individual blocks
pytest projects/components/dma-ip/rapids/dv/tests/macro_beats/ -v   # Multi-block
pytest projects/components/dma-ip/rapids/dv/tests/top_beats/ -v     # Full system

# Run specific FUB test
pytest projects/components/dma-ip/rapids/dv/tests/fub_beats/test_scheduler_beats.py -v

# Lint RAPIDS RTL
verilator --lint-only projects/components/dma-ip/rapids/rtl/fub_beats/scheduler_beats.sv

# View specifications
cat projects/components/dma-ip/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md
cat projects/components/dma-ip/rapids/docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md

## 33.16	14. Success Criteria

### 33.16.1	14.1 Functional

- All major blocks implemented

- Basic data flows working

- Scheduler credit bug fixed (exponential encoding implemented)

- ⏳ Credit management tests verified (remove workarounds, run full suite)

- ⏳ 100% descriptor test pass rate (currently ~80%)

- ⏳ Stress tests passing

### 33.16.2	14.2 Quality

- ⏳ >90% functional coverage (currently ~80%)

- ⏳ >85% code coverage

- All FSMs documented

- ⏳ Integration guide complete

### 33.16.3	14.3 Documentation

- Complete specification in docs/rapids_beats_has/ + docs/rapids_beats_mas/

- Known issues documented

- ⏳ Integration examples

- ⏳ Performance characterization

## 33.17	15. Educational Value

RAPIDS demonstrates: - Complex FSM coordination (scheduler ↔ data paths) - Descriptor-based DMA design patterns - Buffer management strategies - Credit-based flow control with exponential encoding - Multi-interface integration - Comprehensive monitoring - Error detection and reporting - Compact configuration encoding strategies

Target Audience: - Advanced RTL designers - Accelerator architects - DMA engine developers - System integration engineers

## 33.18	16. Documentation Generation

### 33.18.1	16.1 Generating PDF/DOCX from Specification

Tool: bin/md_to_docx.py (driven by the wrapper scripts in docs/ - preferred)

Use the wrapper scripts to convert the linked HAS/MAS spec indexes into single all-inclusive PDF/DOCX files:

Basic Usage:

# Preferred: wrapper scripts (they call md_to_docx.py with the house style)
cd projects/components/dma-ip/rapids/docs
./generate_has_pdf.sh --rev 0.8     # builds RAPIDS_Beats_HAS_v0.8.docx/.pdf
./generate_mas_pdf.sh --rev 0.7     # builds RAPIDS_Beats_MAS_v0.7.docx/.pdf

# Direct tool invocation (from repo root), if you need custom options
python bin/md_to_docx.py \
    projects/components/dma-ip/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md \
    -o projects/components/dma-ip/rapids/docs/RAPIDS_Beats_MAS_draft.docx \
    --toc \
    --title-page \
    --pdf

Key Features: - Recursive Collection: Follows all markdown links in the index file - Heading Demotion: Automatically adjusts heading levels for included files - Table of Contents: --toc flag generates automatic ToC - Title Page: --title-page flag creates title page from first heading - PDF Export: --pdf flag generates both DOCX and PDF - Image Support: Resolves images relative to source directory - Template Support: Optional custom DOCX/DOTX template via -t flag

Common Workflow:

# 1. Update spec content under rapids_beats_has/ or rapids_beats_mas/
# 2. Generate documentation with a bumped revision
cd projects/components/dma-ip/rapids/docs
./generate_mas_pdf.sh --rev 0.8

# 3. Output files created in docs/:
#    - RAPIDS_Beats_MAS_v0.8.docx
#    - RAPIDS_Beats_MAS_v0.8.pdf

Debug Mode:

# Generate debug markdown to see combined output
python bin/md_to_docx.py \
    projects/components/dma-ip/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md \
    -o output.docx \
    --debug-md

# This creates debug.md showing the complete merged content

Tool Requirements: - Python 3.6+ - Pandoc installed and in PATH - For PDF generation: LaTeX (e.g., texlive) or use Pandoc’s built-in PDF writer

See: bin/md_to_docx.py for complete implementation details

## 33.19	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/dma-ip/rapids/docs/

Quick Command: Use the provided shell scripts:

cd projects/components/dma-ip/rapids/docs
./generate_has_pdf.sh    # Architecture spec (HAS)
./generate_mas_pdf.sh    # Micro-architecture spec (MAS)

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the rapids_beats_has / rapids_beats_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

See: bin/md_to_docx.py for complete implementation details

## 33.20	17. References

### 33.20.1	16.1 Internal Documentation

- Complete Spec: docs/rapids_beats_has/ + docs/rapids_beats_mas/ ← Primary technical reference

- Validation: vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md (pre-beats snapshot)

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 33.20.2	16.2 Related Subsystems

- AMBA: rtl/amba/ - Monitor infrastructure used in RAPIDS (monitors in rtl/amba/ocelus/)

- Common: rtl/common/ - Building blocks (counters, FIFOs, etc.)

- CocoTB Framework: bin/TBClasses/ (shared) + projects/components/dma-ip/rapids/dv/tbclasses/ (RAPIDS)

### 33.20.3	16.3 External References

- AXI4 Specification: ARM IHI0022E

- AXIL4 Specification: ARM IHI0022E (subset)

- Network interface specs (custom Network protocol)

Document Version: 1.0 Last Updated: 2025-09-30 Review Cycle: Monthly during active development Next Review: 2025-10-30 Owner: RTL Design Sherpa Project

## 33.21	Navigation

- ← Back to Root: /PRD.md

- Complete Specification: docs/rapids_beats_has/rapids_beats_has_index.md + docs/rapids_beats_mas/rapids_beats_mas_index.md

- AI Guidance: CLAUDE.md

- Tasks: TASKS.md

- Issues: known_issues/

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 34	Claude Code Guide: RAPIDS Subsystem

Version: 1.0 Last Updated: 2025-10-11 Purpose: AI-specific guidance for working with RAPIDS subsystem

## 34.1	Quick Context

What: Rapid AXI Programmable In-band Descriptor System - Custom DMA-style accelerator with AXIS network interfaces Status: Active development - “beats” rearchitecture is the current RTL Your Role: Help users understand architecture, fix bugs, extend functionality

Status (2026-07-22): RAPIDS was rearchitected to the “beats” design. Current RTL lives in rtl/fub_beats/, rtl/macro_beats/, and rtl/top_beats/rapids_beats_top.sv (plus control engines in rtl/fub/). The old rapids_fub//rapids_macro/ modules (scheduler.sv, program_engine.sv, network_slave.sv, …) and the old docs/rapids_spec/ tree are gone. Sections below that describe the pre-beats design are marked accordingly.

Status (2026-09-29, rapids TASK-019): RAPIDS is byte-granular. The product tree is the un-suffixed rtl/fub/, rtl/macro/, rtl/top/rapids_top.sv (descriptor length in BYTES, byte addresses, WSTRB/TSTRB carry partial beats, one AXIS packet per descriptor), tested in dv/tests/fub, macro, top. The *_beats tree is the beat-granular stepping stone: it stays runnable and green (the characterized Genesys 2 design) and is not maintained forward. The two trees share rapids_config_block, the regs, rapids_pkg and the control engines. Byte-specific contracts: the sink ingress holds s_axis_tready until the channel’s descriptor is fetched (issue the descriptor first, or stream concurrently); a tlast packet whose byte count differs from the descriptor sets that channel’s sticky sched_wr_error. TYPE=EXT (row/column striding) descriptors stay beat-aligned by design: aligned addresses, beat-multiple lengths. Linear descriptors may start at any byte on either side.

Silicon (2026-09-30, projects/fpga-systems/Genesys2/dma-ip/rapids/reports/perf/README.md v0.2): 117/117 byte-wise points on the standard bitstream; beat-aligned rows on the word-wide checker bitstream reach 3182 MB/s sink and 3199 MB/s source of the 3200 MB/s peak. Traps: the word-wide checker CRCs only slice 0 of each beat and ignores strobes (beats golden, not byte-wise); the aligned utilization is NOT within 0.5 pp of RAPIDS Beats because of fixed start-up terms from the packet-record gating, accepted as by design (rapids TASK-021); head-of-line blocking on a shared AXIS stream is documented, not fixed; RRESP/BRESP injection is unexercised (rapids TASK-020).

Status (2026-10-15, rapids TASK-024): network planes. The network endpoints moved out of rapids_src/rapids_snk into swappable planes in rtl/plane/ (rapids_plane_{axis,cxs}_{rx,tx}), shared by both trees. The four tops rapids_top, rapids_cxs_top, rapids_beats_top and rapids_beats_cxs_top are GENERATED by bin/gen_plane_tops.py from rtl/top/rapids_top_base.sv and rtl/top_beats/rapids_beats_top_base.sv. Edit the base or the generator, never a top, and run --check. The AXIS tops keep their old interface. CXS profile, limits (DATA_WIDTH <= 1024) and tests: docs/rapids_planes.md. Traps: the core’s plane monitor rides src_/snk_plane_mon_* (monbus_arbiter client 1); the monitor counters live at u_{src,snk}_plane.u_endpoint.

rapids TASK-025 (2026-10-16): ONE CXS link per direction, CXSSRCID = channel. Packets are framed per stream on the channel bits (the byte source interleaves channels mid-packet). CXS_MAX_CREDIT defaults to 32 (one credit = one flit; ~3-cycle credit round trip), owner 2026-10-16; above 63 needs CXS_CREDIT_BEYOND_SPEC. It is not tied to SRAM_DEPTH, which is beats per channel. CXS cells must include a concurrent-channel case: single-channel cells hid the framing bug.

Shared engines (2026-10-16, byte tree): scheduler_group_array has SHARED_ENGINES (default 1, threaded through rapids_src/rapids_snk/rapids_core/rapids_top_base and the generated tops): ONE descriptor_engine_shared, ctrlrd_engine_shared, ctrlwr_engine_shared per half (rtl/fub/), arbitrating on the scheduler side and owning their AXI masters; 0 restores the per-channel engines and AXI-side arbiters. Measured OOC synthesis, rapids_core 256-bit, monitors off: 16 channels = 77.6k LUTs shared vs 109.5k per-channel. Traps: a shared ctrl engine keeps its error until the next success, so the wrapper clears a stale error before handing the engine to another channel (test_..._ctrl_error_handoff guards it, mutation-checked); the descriptor engine registers its pick in an issue slot because the unregistered pick -> 16:1 mux -> range compare path missed 100 MHz at 16 channels.

Tile header (2026-10-17, byte tree, rapids TASK-026): TILE_HDR (default 0) on the core / base / generated tops puts the compute-eng-ip tile packet header (TILE_SPEC 11.5; status table 11.12) on both network ports as the FIRST BEAT of every packet, bits [127:0]. Needs SHARED_ENGINES = 1 and DW >= 256. Source: the scheduler latches ptype/op from the descriptor and the packet record carries {op, ptype, dst_addr} to src_data_path_axis, which emits the header; op = 1 is a header-only packet with no memory read. Sink: rtl/fub/rapids_tile_hdr_rx.sv (in rapids_snk) strips headers and sends desc packets over desc_inj_* into descriptor_engine_shared’s per-channel one-entry slot. Traps: the planes are untouched (CXS is just transport); a disabled channel’s scheduler still pops and discards descriptors from its queue in IDLE (pre-existing, also hits host kicks), so a slot-overflow test must hold the channel busy, not disabled.

Tile credits (2026-10-18, rapids TASK-027 part 1): TILE_CREDITS (default 0, needs TILE_HDR) adds rtl/fub/rapids_tile_credits.sv in rapids_src: 64 destination counters (rapids_pkg::tile_dest_index = {ext, n}: n 0-15 cell-level nodes, 16-19 tetrads, 20-21 roots; ext 1 = what is attached, so cells are 32-47) filled by set/add credit packets the sink’s rapids_tile_hdr_rx passes over credit_*. When SRC.TILE_ADV_WINDOW cycles from reset elapse, a counter still at 0 is illegal (SRC.TILE_LEGAL_DEST_LO/HI, SRC.TILE_CREDIT_STATUS.ADV_DONE). delta nodes only transport and never originate, so a node’s internal ID (0-21) is legal by topology, never charged, its credits are ignored, and a read request to it is refused (rapids_pkg::tile_dest_is_node). The scheduler holds tile descriptors until then and refuses one to an illegal destination. src_data_path_axis charges each head packet’s flits (tile_acq_*) before its header or further reservations. Traps: the window counts from RESET, so a TB must advertise during bring-up; the tile coordinate is 15 bits {ext, level, X, Y, Z}; a channel reset does not refund charged credits.

Tile straps (2026-10-19, rapids TASK-027 part 2): the same TILE_CREDITS builds the straps TOWARD RAPIDS (TILE_SPEC 11.9-11.11). Sink: rapids_tile_hdr_rx (STRAPS) tags each data packet with its sender’s cell slot (rapids_pkg::tile_ext_slot, slot = index - 32) and rapids_tile_landing holds TILE_LAND_FLITS beats per cell, draining one whole packet at a time only when snk_data_path_axis chan_can_take (record + TILE_LAND_FLITS+4 free). Core desc packets go through rapids_tile_desc_intake (4 x TILE_DESC_PER_CORE deep) into descriptor_engine_shared with INJ_HOLD (a full slot waits, never drops). rapids_tile_credit_tx returns every consumed flit / entry as coalesced add packets on the top-level m_tile_credit_* port (its own interface), and on SRC.TILE_RESYNC sends one set per legal sender while tile_quiescent is high; channel resets of both halves are gated by tile_quiescent too. rapids_tile_stall_mon on client 2 of each half’s MonBus arbiter flags data stuck for SRC.TILE_STALL_CYCLES (sink SRAM data with no W beat; source reads with no R beat). Traps: the byte TB drives tile_quiescent = 1 and m_tile_credit_ready = 1 by default; a data packet must come from a CELL coordinate (ext = 1, level 0) or it is dropped as unsupported; SRAM_DEPTH per channel >= TILE_LAND_FLITS + 4; an allocated-but-unfilled SRAM segment is free space, not data (the stall check’s first cut got this wrong). Tests: test_rapids_cxs_top_tile_straps[...] (dv/tbclasses/rapids_cxs_tile_straps_tb.py). TILE_STRAPS (rapids TASK-031) reports the two build parameters at 0x7FC, the last config word of each half (owner: top or bottom of the config space, monitor block excluded); keep it there. The runner exports TEST_TILE_{CREDITS,LAND_FLITS,DESC_PER_CORE} from the build parameters, so the sender models follow a non-default strap build (tile_straps_alt).

8- and 16-channel builds (2026-10-17, byte tree): one rapids_top_base, two register maps. The 8-channel map is today’s hand-written RDL (rtl/macro_beats/rapids_{regs,engine_regs,mon_regs,regmap}.rdl, regblock rapids_regs, rtl/rapids_regmap.py). The 16-channel map is DERIVED by bin/gen_rapids_regs_16ch.py into rtl/macro_beats/generated_16ch/ (regblock rapids_regs_16ch in regs/generated_16ch/, rtl/rapids_regmap_16ch.py); edit the 8-ch files, rerun it (--check), then regenerate both regblocks with bin/peakrdl_generate.py (check_rdl_regen.py covers both). The base picks its regblock and the per-channel-by-name glue (KICKn, CHn_DESC_ADDR) from rtl/top/generated/regsel_{8,16}ch/, chosen by +incdir in rtl/filelists/top/rapids_top_base{,_16ch}.f. Tops: rapids_top/rapids_cxs_top (8, unchanged, used by the FPGA flows) plus generated rapids_{8,16}ch_top and rapids_{8,16}ch_cxs_top. The 16-ch map moves KICK_ENABLE to 0x080 and CH_STATE to 0x180: by-name access only. HAS: rapids_has/ch05_programming/02_register_map_16ch.md. Measured OOC synthesis, rapids_core 256-bit, 16 channels shared: 77.1k LUTs, +0.56 ns at 100 MHz once the descriptor engine’s chunk-1 +32 moved to the R side.

rapids-g (2026-10-21, rapids TASK-034): the byte tree with its schedulers sequenced by firmware on one gatling per half. rtl/fub/scheduler.sv is now the FSM + ceilings + run-base generators around rtl/fub/scheduler_core.sv (the shared datapath); rtl/fub_g/scheduler_g.sv wraps the same core with a command window. rtl/macro_g/rapids_seq_g.sv (gatling + ROM + scratch) and the hand-written rtl/macro_g/scheduler_group_array_g_seq.svh; everything else in macro_g/generated/ and top_g/generated/ is DERIVED by bin/gen_rapids_g.py (--check) from the product files – edit the product, rerun. Firmware: rtl/fw_g/sched_g.S -> bin/gen_rapids_g_fw.py -> rtl/macro_g/generated/fw/sched_g_fw.svh (never edit). 4 or 8 channels, SHARED_ENGINES = 1. Tests: dv/tests/fub_g (FSM-vs-firmware equivalence bench with an independent golden model; edge table), macro_g (product array tests by module name), top_g (product top wrappers re-hosted, rapids_g_rehost.py). Traps: the TB’s SCHED_WAIT_FLOOR env floors every wait for a scheduler reaction (firmware takes hundreds of cycles a step; the -g runners set it); cocotb matches a testcase name as a prefix pattern, so no cell may be another’s prefix (ext ran ext_reset too); the firmware’s per-slot scratch is 64 words, so SCRATCH_WORDS < 64 x slots aliases slot 1 onto slot 0 (elaboration refuses it); CMD_ERR counts the firmware’s stale commands after a channel reset, so it is checked only on channels never reset. Pre-validation (the firmware ARMs a descriptor and STAGEs the next; the hardware completes and starts it) has three ordering rules, each a race the equivalence bench found: an EXT descriptor auto-completes only with both bases-done bits set; GEN_OK is written BEFORE the EXT set-up; refill_both judges both directions on one STATUS snapshot (a re-read can already be the next descriptor’s). Register path: each half’s gatling reaches its own half of rapids_regs through a per-hart mailbox (rapids_seq_g 0x1000_3000) and rapids_g_reg_arb in front of the MON guard (host + SRC + SNK, one outstanding, offset pinned per half); CH_SCRATCH[n] (0x300 per half, 8 bits, storage only) is shared by host and gatling. MAS Chapter 5.

Complete Specification: - projects/components/dma-ip/rapids/docs/rapids_beats_has/ - Architecture spec (HAS); index: rapids_beats_has_index.md - projects/components/dma-ip/rapids/docs/rapids_beats_mas/ - Micro-architecture spec (MAS); index: rapids_beats_mas_index.md - projects/components/dma-ip/rapids/docs/rapids_has/ and rapids_mas/ - the byte-granular RAPIDS books (index: rapids_has_index.md, rapids_mas_index.md); chapters identical to Beats are linked from the rapids_beats_* books, not copied - Built PDFs: docs/RAPIDS_HAS_v0.7.pdf, docs/RAPIDS_MAS_v0.6.pdf (byte design; shared engines, 16 channels, tile header, measured board A/B); docs/RAPIDS_Beats_HAS_v0.13.pdf, docs/RAPIDS_Beats_MAS_v0.11.pdf (beats design) - Network planes / CXS: HAS rapids_beats_has/ch03_interfaces/06_cxs_interface.md, MAS rapids_beats_mas/ch03_macro_blocks/13_network_planes.md and ch04_interfaces/04_cxs_interface_spec.md (all shared by the byte books). Routing / payload-type PROPOSAL (2026-10-16, not implemented) is defined ONLY in compute-eng-ip/tile/docs/TILE_SPEC.md §11 (CXS is just the transport); RAPIDS docs hold only the descriptor fields (beats HAS Descriptor Format) and pointers

## 34.2	Global Requirements Reference

IMPORTANT: Check /GLOBAL_REQUIREMENTS.md before starting RAPIDS work

All mandatory requirements are consolidated in the global requirements document: - See: /GLOBAL_REQUIREMENTS.md - Repository-wide mandatory requirements - RAPIDS-Specific: Attribution format, BFM usage requirements - Universal: TB location, three methods, TBBase inheritance, 100% success

This CLAUDE.md provides RAPIDS-specific guidance. Also review: - Root /CLAUDE.md - Repository-wide patterns - projects/components/CLAUDE.md - Project area standards (reset macros, FPGA attributes) - bin/TBClasses/ - Shared TB framework (flat protocol dirs: gaxi/, axi4/, apb/, …); the full CocoTBFramework (components/, tbclasses/) lives in the separate RTLDesignSherpa-DV repo

## 34.3	Critical Rules for This Subsystem

### 34.3.1	Rule #0.1: Testbench Location and Test Structure (MANDATORY)

See: /GLOBAL_REQUIREMENTS.md Section 2.1. Test structure itself – Pattern B, the cocotb_test_* prefix, pytest wrapper naming – is the test-patterns skill.

RAPIDS keeps all of its verification in the project area:

projects/components/dma-ip/rapids/dv/
├── tbclasses/     # 22 TB classes: scheduler_tb, descriptor_engine_tb,
│                  # rapids_core_beats_tb, rapids_beats_top_tb, ...
├── components/    # RAPIDS-specific BFMs (data_mover_bfm.py)
└── tests/         # runners by layer: fub/ fub_beats/ macro/ macro_beats/ top_beats/

Scoring choice (GR 2.4). The descriptor and program engines are in-order, so they verify by queue access – monitor._recvQ.popleft() compared against an expected value. The memory model is for integration tests, where several masters are in flight and order is not guaranteed. Reaching for the memory model on an in-order engine buys nothing and hides ordering bugs.

### 34.3.2	Rule #0.5: Config Before Reset (RAPIDS-specific)

The three mandatory TB methods are /GLOBAL_REQUIREMENTS.md Section 2.2. What is particular to RAPIDS is when configuration is applied: several modules latch config during reset, so a value written after deassert_reset() is simply never seen.

Set these in setup_clocks_and_reset() before asserting reset:

| Signal | Why it must be early |
| --- | --- |
| cfg_initial_credit | the credit counter initialises to (1 << cfg) during reset |
| cfg_use_credit | selects the flow-control mode the counter comes up in |
| cfg_timeout_threshold | watchdog loads at reset |
| cfg_sram_depth | must be stable before the memory controllers leave reset |

(The credit signals belong to the retired pre-beats scheduler.sv; rtl/fub_beats/scheduler_beats.sv has no credit management yet. The config-before-reset rule still holds for the beats modules.)

### 34.3.3	Rule #0.75: Audit Signal Naming BEFORE Writing Testbenches

Why factory prefix collisions break BFM discovery, and the audit tool that catches them, are in vault/handbook/design/naming-and-style.md. Run bin/audit_signal_naming_conflicts.py over rtl/ before writing a TB. What is RAPIDS-specific is below.

Known RAPIDS Conflicts: None (audited 2026-10-19: rtl/ audits clean). The pre-beats desc_valid/desc_ar_valid collision is now desceng_to_sched_* in both scheduler groups; history in known_issues/resolved/scheduler_group_signal_naming_conflicts.md.

Recommended Workflow: 1. Write RTL module with signals 2. Run audit script to detect conflicts 3. Fix naming conflicts (rename internal signals with _to_sched suffix) 4. Write testbench using factory pattern matching

Three Solutions When Conflicts Found: 1. Rename internal signals (recommended): desc_valid → desc_to_sched_valid 2. Use explicit signal_map: Bypass pattern matching with manual signal mapping 3. Test at higher level: Where internal signals aren’t visible (e.g., rapids_top)

Complete Guide: bin/SIGNAL_NAMING_AUDIT.md

### 34.3.4	Rule #1: MANDATORY BFM Usage for FUB-Level Tests

Never hand-drive a valid/ready handshake and never write a custom protocol driver. The interface-to-BFM map, the factory list, the trap list and the extract-vs-embed criteria are in vault/handbook/dv/bfm-usage.md.

RAPIDS-specific: the network interfaces are AXIS, so they take the axis4 factories; the custom valid/ready interfaces (program, descriptor) take GAXI. dv/components/data_mover_bfm.py is the one extracted RAPIDS BFM.

Manual driving is acceptable for throwaway debug and for clock/reset init – never in a production testbench.

### 34.3.5	Rule #2: Know the Known Issues

Check projects/components/dma-ip/rapids/known_issues/ (and active/) before diagnosing anything.

Historical: the pre-beats scheduler.sv credit counter was hardcoded to 0, then fixed with exponential encoding (0->1, 1->2, …, 15->inf). That scheduler was replaced by rtl/fub_beats/scheduler_beats.sv, which has no credit management yet, and its known_issues/scheduler.md write-up was retired with it. Do not chase that bug in beats code.

### 34.3.6	Rule #3: RAPIDS is Complex - Understand Block Interactions

Key Interaction Patterns:

- Descriptor Flow:

- Software → AXIL4 → Descriptor Engine → Scheduler → Data Paths

- Sink Data Path (Network → Memory):

- Network Slave → Sink SRAM Control → Sink AXI Write Engine → System Memory

- Source Data Path (Memory → Network):

- System Memory → Source AXI Read Engine → Source SRAM Control → Network Master

- Monitoring:

- All Blocks → MonBus Reporter → MonBus Output

Never work on one block in isolation without understanding its upstream/downstream dependencies!

### 34.3.7	Rule #4: Test Strategy is Multi-Layered

RAPIDS testing follows this hierarchy:

- FUB (Functional Unit Block) Tests: projects/components/dma-ip/rapids/dv/tests/fub_beats/ (+ fub/ for control engines)

  - Individual block testing

  - Focus: Module-level functionality

  - Example: Scheduler FSM, Descriptor Engine FIFO

- Macro Tests: projects/components/dma-ip/rapids/dv/tests/macro_beats/ (+ macro/ for the monbus group)

  - Multi-block scenarios

  - Focus: Block-to-block interfaces

  - Example: Scheduler group, sink/source data path end-to-end

- Top Tests: projects/components/dma-ip/rapids/dv/tests/top_beats/

  - Full RAPIDS operation

  - Focus: Realistic traffic patterns

  - Example: Complete DMA transfer with monitoring

When creating/modifying tests, ensure appropriate test layer is used!

## 34.4	Architecture Quick Reference

### 34.4.1	Block Organization

RAPIDS Beats Architecture (rtl/)
├── fub_beats/                       (per-block engines)
│   ├── scheduler_beats.sv           (per-channel scheduler FSM)
│   ├── descriptor_engine_beats.sv   (descriptor fetch, parsing)
│   ├── axi_read_engine_beats.sv     (AXI4 read from system memory)
│   ├── axi_write_engine_beats.sv    (AXI4 write to system memory)
│   ├── alloc_ctrl_beats.sv          (SRAM space allocation)
│   ├── drain_ctrl_beats.sv          (SRAM drain control)
│   └── latency_bridge_beats.sv      (latency-hiding bridge)
├── fub/                             (control engines)
│   ├── ctrlrd_engine.sv             (control read engine)
│   └── ctrlwr_engine.sv             (control write engine)
├── macro_beats/                     (assemblies)
│   ├── scheduler_group_beats.sv / scheduler_group_array_beats.sv
│   ├── snk_data_path_beats.sv / snk_data_path_axis_beats.sv   (AXIS network → SRAM → memory)
│   ├── src_data_path_beats.sv / src_data_path_axis_beats.sv   (memory → SRAM → AXIS network)
│   ├── snk_sram_controller_beats.sv / src_sram_controller_beats.sv
│   ├── rapids_config_block.sv + rapids_*_regs.rdl             (APB CSRs via PeakRDL)
│   └── rapids_core_beats.sv         (core integration)
├── macro/
│   └── monbus_axil_group_2in.sv     (MonBus aggregation / AXI-Lite drain)
└── top_beats/
    └── rapids_beats_top.sv          (top level: APB config, AXI4 masters, AXIS in/out, MonBus)

See: docs/rapids_beats_mas/ch02_fub_blocks/ and docs/rapids_beats_mas/ch03_macro_blocks/ for detailed block descriptions

### 34.4.2	Module Quick Reference

| Module | Location | Purpose | Documentation (docs/rapids_beats_mas/) |
| --- | --- | --- | --- |
| scheduler_beats.sv | fub_beats/ | Per-channel scheduler FSM | ch02_fub_blocks/01_scheduler.md |
| descriptor_engine_beats.sv | fub_beats/ | Descriptor fetch, parsing | ch02_fub_blocks/02_descriptor_engine.md |
| axi_read_engine_beats.sv | fub_beats/ | Memory reads | ch02_fub_blocks/03_axi_read_engine.md |
| axi_write_engine_beats.sv | fub_beats/ | Memory writes | ch02_fub_blocks/04_axi_write_engine.md |
| alloc_ctrl_beats.sv | fub_beats/ | SRAM allocation | ch02_fub_blocks/05_beats_alloc_ctrl.md |
| drain_ctrl_beats.sv | fub_beats/ | SRAM drain control | ch02_fub_blocks/06_beats_drain_ctrl.md |
| latency_bridge_beats.sv | fub_beats/ | Latency-hiding bridge | ch02_fub_blocks/07_beats_latency_bridge.md |
| snk_data_path_beats.sv | macro_beats/ | Sink data path | ch03_macro_blocks/03_sink_data_path.md |
| src_data_path_beats.sv | macro_beats/ | Source data path | ch03_macro_blocks/06_source_data_path.md |
| rapids_core_beats.sv | macro_beats/ | Core integration | ch03_macro_blocks/09_rapids_core_beats.md |
| rapids_beats_top.sv | top_beats/ | Top-level integration | ch03_macro_blocks/12_rapids_beats_top.md |

### 34.4.3	Interface Summary

| Interface | Type | Width | Purpose | Specification |
| --- | --- | --- | --- | --- |
| APB4 | Slave | 32-bit | Control/status registers | docs/rapids_beats_has/ch03_interfaces/04_apb_interface.md |
| AXI4 (Sink) | Master | Configurable | Write to system memory | docs/rapids_beats_mas/ch04_interfaces/01_axi4_interface_spec.md |
| AXI4 (Source) | Master | Configurable | Read from system memory | docs/rapids_beats_mas/ch04_interfaces/01_axi4_interface_spec.md |
| AXIS (Sink) | Slave | Configurable | Network ingress (tid = channel) | docs/rapids_beats_mas/ch04_interfaces/02_axis_interface_spec.md |
| AXIS (Source) | Master | Configurable | Network egress (tid = channel) | docs/rapids_beats_mas/ch04_interfaces/02_axis_interface_spec.md |
| CXS (Sink / Source) | Receiver / Transmitter | DATA_WIDTH <= 1024 | Network on the *_cxs_top (one link per direction, CXSSRCID = channel) | docs/rapids_beats_mas/ch04_interfaces/04_cxs_interface_spec.md |
| MonBus | Master | 64-bit | Monitor packet output | docs/rapids_beats_mas/ch04_interfaces/03_monbus_interface_spec.md |

## 34.5	Common User Questions and Responses

### 34.5.1	Q: “How does the scheduler work?”

A: Direct answer:

The scheduler is a complex FSM that coordinates RAPIDS operations:

- Idle State: Waits for descriptors from Descriptor Engine

- Parse State: Extracts descriptor fields (address, length, control)

- Credit Check: Verifies credit availability (if credit mode enabled)

- Execute State: Activates appropriate data path (sink or source)

- Monitor State: Tracks operation progress

- Complete State: Generates completion packets, updates credits

Key FSM States: - IDLE → PARSE → CREDIT_CHECK → EXECUTE → MONITOR → COMPLETE → IDLE

Credit Management: - Historical: the pre-beats scheduler.sv implemented exponential credit encoding (0→1, 1→2, 2→4, …, 15→∞) - see Q&A below for the historical details - Current: rtl/fub_beats/scheduler_beats.sv has no credit management yet (planned later phase)

See: - projects/components/dma-ip/rapids/docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md - Complete FSM specification - projects/components/dma-ip/rapids/known_issues/README.md - Known bugs and workarounds

### 34.5.2	Q: “How do I configure RAPIDS?”

Status (2026-07-22): The beats top (rapids_beats_top.sv) exposes an APB4 config slave (s_apb_*) with PeakRDL-generated registers (rtl/macro_beats/rapids_regs.rdl); access registers by name via the generated regmap, not hardcoded offsets. The AXIL4 write sequence below is from the pre-beats design and is kept for historical flavor only.

A: Configuration via memory-mapped register interface:

// 1. Initialize RAPIDS via AXIL4 writes
// Configure SRAM depths
write_axil(ADDR_SINK_SRAM_DEPTH, 1024);
write_axil(ADDR_SOURCE_SRAM_DEPTH, 1024);

// Set timeout thresholds
write_axil(ADDR_TIMEOUT_THRESHOLD, 1000);

// Configure initial credits (exponential encoding)
// 0→1, 1→2, 2→4, 3→8, 4→16, etc.
write_axil(ADDR_INITIAL_CREDIT, 4);  // 4 = 16 credits (2^4)
write_axil(ADDR_CREDIT_ENABLE, 1);   // Enable credit mode (now fixed!)

// 2. Load descriptors
write_descriptor(addr, length, control_bits);

// 3. Enable operation
write_axil(ADDR_ENABLE, 1);

See: - projects/components/dma-ip/rapids/docs/rapids_beats_has/ch05_programming/03_initialization.md - Programming model - projects/components/dma-ip/rapids/docs/rapids_beats_has/ch05_programming/02_register_map.md - Register definitions

### 34.5.3	Q: “What’s the data flow for network to memory transfer?”

A: Sink path data flow:

1. Network Packet Arrives
   ↓
2. Network Slave receives packet
   - Validates packet format
   - Handshakes with Network interface
   ↓
3. Sink SRAM Control buffers data
   - Writes to SRAM
   - Manages write pointers
   - Handles backpressure
   ↓
4. Sink AXI Write Engine
   - Reads from SRAM
   - Generates AXI4 write transactions
   - Bursts data to system memory
   ↓
5. Completion Reporting
   - Generates MonBus completion packet
   - Updates scheduler state

Key Considerations: - SRAM acts as buffer to decouple network from memory timing - AXI4 bursts used for efficient memory access - Backpressure propagates from memory to network

See: projects/components/dma-ip/rapids/docs/rapids_beats_mas/ch03_macro_blocks/03_sink_data_path.md

### 34.5.4	Q: “What’s the credit counter bug and how does exponential encoding work?”

Status (2026-07-22): Historical. This bug and fix lived in the retired pre-beats scheduler.sv; rtl/fub_beats/scheduler_beats.sv currently has no credit management (planned for a later phase). Kept as background for when credits return.

A: Scheduler credit counter exponential encoding (pre-beats):

The scheduler uses exponential credit encoding to provide a wide range of credit values (1 to 16384) with a compact 4-bit configuration:

Encoding Table: | cfg_initial_credit | Actual Credits | Use Case | |———————|—————-|———-| | 0 | 1 | Minimum (2^0) | | 1 | 2 | Very low traffic (2^1) | | 2 | 4 | Low traffic (2^2) | | 3 | 8 | (2^3) | | 4 | 16 | Typical (2^4) | | 5 | 32 | (2^5) | | 6 | 64 | Medium traffic (2^6) | | 7 | 128 | (2^7) | | 8 | 256 | High traffic (2^8) | | 10 | 1024 | Very high traffic (2^10) | | 14 | 16384 | Maximum finite (2^14) | | 15 | ∞ (0xFFFFFFFF) | Unlimited credits |

FIXED Implementation (in the retired pre-beats scheduler.sv):

// Exponential credit encoding: 0→1, 1→2, 2→4, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Was Broken (Before Fix):

r_descriptor_credit_counter <= 32'h0;  // WRONG: Hardcoded to 0

Impact (Before Fix): - Credit-based flow control didn’t work - All operations blocked if credit mode enabled - Descriptors never processed

Encoding Rationale: - Compact 4-bit config covers wide range: 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞)

Important: Exponential encoding applies only at initialization. Once running, the counter operates linearly (increment/decrement by 1).

See: - projects/components/dma-ip/rapids/docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md - Current scheduler specification - projects/components/dma-ip/rapids/known_issues/README.md - Issue tracking (the old scheduler.md write-up was retired with the pre-beats RTL)

### 34.5.5	Q: “How do I run RAPIDS tests?”

A: Multi-layered test approach:

# All of these go through the area Makefile. A bare pytest drops the level,
# the derived worker count and the reruns, and skips clean-all -- see
# vault/handbook/dv/running-regressions.md
cd projects/components/dma-ip/rapids/dv/tests

# 1. FUB tests - individual blocks. AREAS narrows the sweep;
#    the dispatcher's areas are: fub fub_beats macro macro_beats top_beats
make run-scheduler_beats-gate AREAS=fub_beats
make run-all-gate AREAS=fub_beats      # every beats FUB test
make run-all-gate AREAS=fub            # control engines

# 2. Macro tests - multi-block scenarios
make run-all-func AREAS=macro_beats

# 3. Top tests - full RAPIDS operation
make run-all-func AREAS=top_beats

# Waves: WAVES=1 via the target, not --vcd. create_view_cmd() writes a
# ready-made gtkwave command beside the log.
make run-scheduler_beats-gate-waves AREAS=fub_beats

Test Organization: - FUB tests: Focus on individual block functionality - Macro tests: Verify block-to-block interfaces - Top tests: Validate complete data flows

Current status: regenerate it – dv/tests/analyze_beats_coverage.py writes to dv/tests/coverage_reports/. The ~80% figure that sat here was a pre-beats snapshot, as is vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md, which remains the written-up version of that era.

## 34.6	Integration Patterns

### 34.6.1	Pattern 1: Basic RAPIDS Instantiation

The byte-granular top (rtl/top/rapids_top.sv, rapids TASK-019). The beat-granular stepping stone rapids_beats_top has the same ports with the same names; only the descriptor’s length unit differs (bytes here, beats there).

rapids_top #(
    .NUM_CHANNELS       (8),
    .DATA_WIDTH         (256),
    .ADDR_WIDTH         (64),
    .AXI_ID_WIDTH       (8),
    .SRAM_DEPTH         (1024),
    .USE_AXI_MONITORS   (1)
) u_rapids (
    .aclk               (system_clk),
    .aresetn            (system_rst_n),

    // APB4 configuration slave (SRC block at 0x0000, SNK block at 0x1000)
    .s_apb_paddr        (cfg_paddr),
    .s_apb_psel         (cfg_psel),
    .s_apb_penable      (cfg_penable),
    .s_apb_pwrite       (cfg_pwrite),
    .s_apb_pwdata       (cfg_pwdata),
    .s_apb_pstrb        (cfg_pstrb),
    .s_apb_prdata       (cfg_prdata),
    .s_apb_pready       (cfg_pready),
    .s_apb_pslverr      (cfg_pslverr),

    // AXI4 read master (source: memory -> m_axis)
    .m_axi_rd_arvalid   (mem_arvalid),
    .m_axi_rd_arready   (mem_arready),
    .m_axi_rd_araddr    (mem_araddr),
    .m_axi_rd_arlen     (mem_arlen),
    .m_axi_rd_rdata     (mem_rdata),
    .m_axi_rd_rvalid    (mem_rvalid),
    .m_axi_rd_rready    (mem_rready),
    // ... remaining m_axi_rd_* AR/R signals

    // AXI4 write master (sink: s_axis -> memory), WSTRB carries the byte enables
    .m_axi_wr_awvalid   (mem_awvalid),
    .m_axi_wr_awready   (mem_awready),
    .m_axi_wr_awaddr    (mem_awaddr),
    .m_axi_wr_awlen     (mem_awlen),
    .m_axi_wr_wdata     (mem_wdata),
    .m_axi_wr_wstrb     (mem_wstrb),
    .m_axi_wr_wvalid    (mem_wvalid),
    .m_axi_wr_wready    (mem_wready),
    // ... remaining m_axi_wr_* AW/W/B signals

    // AXI-Stream: ingress to the sink, egress from the source (tid = channel)
    .s_axis_tdata       (net_in_tdata),
    .s_axis_tstrb       (net_in_tstrb),
    .s_axis_tlast       (net_in_tlast),
    .s_axis_tid         (net_in_tid),
    .s_axis_tvalid      (net_in_tvalid),
    .s_axis_tready      (net_in_tready),
    .m_axis_tdata       (net_out_tdata),
    .m_axis_tstrb       (net_out_tstrb),
    .m_axis_tlast       (net_out_tlast),
    .m_axis_tid         (net_out_tid),
    .m_axis_tvalid      (net_out_tvalid),
    .m_axis_tready      (net_out_tready),
    // ... tdest/tuser, the descriptor and control AXI masters, MonBus
    .mon_irq            (dma_irq)
);

### 34.6.2	Pattern 2: Configuration Sequence

// Recommended initialization sequence
initial begin
    // 1. Assert reset
    aresetn = 0;
    repeat(10) @(posedge aclk);
    aresetn = 1;

    // 2. Configure RAPIDS via AXIL4
    axil_write(ADDR_SINK_SRAM_DEPTH, 1024);
    axil_write(ADDR_SOURCE_SRAM_DEPTH, 1024);
    axil_write(ADDR_TIMEOUT_THRESHOLD, 1000);
    axil_write(ADDR_INITIAL_CREDIT, 4);  // 4 = 16 credits (2^4, exponential encoding)
    axil_write(ADDR_CREDIT_ENABLE, 1);  // Enable credit mode (now fixed!)

    // 3. Load descriptors
    for (int i = 0; i < num_descriptors; i++) begin
        axil_write(ADDR_DESC_ADDR, descriptor[i].addr);
        axil_write(ADDR_DESC_LEN, descriptor[i].length);
        axil_write(ADDR_DESC_CTRL, descriptor[i].control);
        axil_write(ADDR_DESC_COMMIT, 1);
    end

    // 4. Enable RAPIDS operation
    axil_write(ADDR_ENABLE, 1);
end

### 34.6.3	Pattern 3: MonBus Integration

// Always add downstream FIFO for MonBus
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_rapids_mon_fifo (
    .axi_aclk   (aclk),
    .axi_aresetn(aresetn),
    // producer side: wr_ready is "not full", so it is what backpressures MonBus
    .wr_valid   (monbus_pkt_valid),
    .wr_data    (monbus_pkt_data),
    .wr_ready   (monbus_pkt_ready),
    // consumer side
    .rd_valid   (fifo_mon_valid),
    .rd_data    (fifo_mon_data),
    .rd_ready   (consumer_ready)
);

## 34.7	Anti-Patterns to Catch

### 34.7.1	Anti-Pattern 1: Not Understanding Exponential Credit Encoding

WRONG:
.cfg_initial_credit(4'd16),  // Thinks this gives 16 credits - it doesn't!

CORRECTED:
"Credits use exponential encoding:
- cfg_initial_credit = 4 → 16 credits (2^4)
- cfg_initial_credit = 8 → 256 credits (2^8)
- cfg_initial_credit = 15 → ∞ credits (unlimited)
(Pre-beats behavior; scheduler_beats.sv has no credit management yet. See the encoding
table in the credit-counter Q&A above.)"

### 34.7.2	Anti-Pattern 2: Insufficient SRAM Depth

WRONG:
.SRAM_DEPTH(16)  // Too small for realistic packets

CORRECTED:
"SRAM depth should match typical packet sizes.
Recommended: 1024-4096 entries depending on data width and packet sizes."

### 34.7.3	Anti-Pattern 3: No MonBus Downstream Handling

WRONG:
assign monbus_pkt_ready = 1'b1;  // Always ready = potential packet loss

CORRECTED:
"Connect to FIFO or proper consumer:
gaxi_fifo_sync #(.DATA_WIDTH(64), .DEPTH(256)) u_mon_fifo (
    .wr_valid(monbus_pkt_valid),
    .wr_data (monbus_pkt_data),
    .wr_ready(monbus_pkt_ready),
    ...
);"

### 34.7.4	Anti-Pattern 4: Testing Individual Blocks in Isolation

WRONG:
"Only test scheduler without descriptor engine"

CORRECTED:
"RAPIDS blocks are tightly coupled. Always test:
1. FUB tests for basic block functionality
2. Integration tests for block interactions
3. System tests for complete flows"

## 34.8	Debugging Workflow

### 34.8.1	Issue: Scheduler Not Processing Descriptors

Check in order: 1. Credit configuration correct? (Remember exponential encoding!) 2. Descriptors loaded via AXIL4? 3. RAPIDS enabled? (ADDR_ENABLE = 1) 4. Reset properly deasserted? 5. Descriptor engine FIFO not empty?

Debug commands:

cd projects/components/dma-ip/rapids/dv/tests
make run-scheduler_beats-gate AREAS=fub_beats          # -v --tb=short by default
make run-scheduler_beats-gate-waves AREAS=fub_beats    # WAVES=1, not --vcd
# create_view_cmd() writes the gtkwave command beside the log

### 34.8.2	Issue: Data Path Stalls

Check in order: 1. SRAM depth sufficient? 2. Downstream interfaces ready? 3. AXI4 backpressure handling? 4. Network flow control? 5. Buffer overflow/underflow detection?

Waveform Analysis: - Check SRAM read/write pointers - Verify AXI4 handshakes - Inspect Network TREADY signals

### 34.8.3	Issue: MonBus Packets Not Generated

Check in order: 1. Operations completing successfully? 2. MonBus ready signal asserted? 3. MonBus reporter enabled? 4. Downstream FIFO not full?

## 34.9	Testing Guidance

### 34.9.1	Running Tests

Tests are organised by layer under dv/tests/: fub/ and fub_beats/ for single blocks, macro/ and macro_beats/ for multi-block scenarios, top_beats/ for full RAPIDS operation. make list enumerates the roots.

This area’s Makefile takes an AREAS variable (default fub fub_beats macro macro_beats top_beats) to scope a run to some layers:

cd projects/components/dma-ip/rapids/dv/tests

make run-all-gate AREAS=fub_beats          # one layer
make run-scheduler_beats-gate AREAS=fub_beats
make clean-all && make run-all-full-parallel   # everything; clean-all first

Target grammar, the levels and why clean-all is not optional: vault/handbook/dv/running-regressions.md.

### 34.9.2	Test Coverage Status

Generate it, do not read it from here – a pasted table is stale the day after it is written. dv/tests/analyze_beats_coverage.py writes to dv/tests/coverage_reports/.

## 34.10	Key Documentation Links

Specification (projects/components/dma-ip/rapids/docs/): - rapids_beats_has/rapids_beats_has_index.md - architecture spec (HAS) - rapids_beats_mas/rapids_beats_mas_index.md - micro-architecture spec (MAS) - MAS ch02_fub_blocks/, ch03_macro_blocks/, ch04_interfaces/ - per-block and interface detail; HAS ch05_programming/ - the programming model

This component: PRD.md, TASKS.md, known_issues/.

Validation: vault/tracking/projects/components/dma-ip/rapids/RAPIDS_Validation_Status_Report.md – test results, but a pre-beats snapshot (last updated 2026-07-22; it still discusses program_engine, network_slave and the retired scheduler.sv). PRD.md labels it the same way. For current numbers run dv/tests/analyze_beats_coverage.py.

Framework BFM docs: ../RTLDesignSherpa-DV/docs/components/<family>/ (published at sean-galloway.github.io/RTLDesignSherpa-DV).

## 34.11	Verification Patterns

These were ~270 lines of general DV method sitting in a component file. They now live in the handbook, where every area can find them:

- Asynchronous outputs need a background monitor – vault/handbook/dv/async-output-capture.md. This is RAPIDS’ own lesson: the descriptor-engine test found 5 of 12 descriptors and filed it as a 42% engine defect, when the engine was correct and the test was not watching.

- Delay profiles – vault/handbook/dv/randomization.md. RAPIDS’ DelayProfile enum re-implements DEFAULT_PROFILES; name a catalogue profile instead.

- Extract a BFM or embed it – vault/handbook/dv/bfm-usage.md, which uses RAPIDS’ own data_mover_bfm.py (extracted) and the embedded AXI responder in descriptor_engine_tb.py as the worked pair.

- 100% success is required – /GLOBAL_REQUIREMENTS.md 3.3, which names this file as its source. A 70% threshold hides a defect rather than tolerating it.

## 34.12	Documentation Generation

Both specs build from their index file into DOCX and PDF, written into docs/. Use the wrapper scripts – they apply the house style:

cd projects/components/dma-ip/rapids/docs
./generate_has_pdf.sh --rev 0.8     # RAPIDS_Beats_HAS_v0.8.docx/.pdf
./generate_mas_pdf.sh --rev 0.7     # RAPIDS_Beats_MAS_v0.7.docx/.pdf

They call bin/md_to_docx.py, which follows the markdown links out of the index, demotes headings and builds the ToC. Pipeline details, the LaTeX/PDF engine choice and the caption traps: vault/handbook/authoring/doc-pipeline.md.

Version: 1.2 Last Updated: 2025-10-14 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 35	RAPIDS Beats Architecture HAS

Version: 0.16 Date: 2026-10-20 Purpose: Hardware Architecture Specification for RAPIDS Phase 1 “Beats” Architecture Classification: External Interface Specification

## 35.1	Document Organization

Note: All chapters linked below for automated document generation.

### 35.1.1	Front Matter

- Document Information

- Revision History

- Acronyms and Definitions

### 35.1.2	Chapter 1: Overview

- Product Overview

- Key Features

- System Context

### 35.1.3	Chapter 2: Architecture

- Block Diagram

- Data Flow

- Channel Architecture

### 35.1.4	Chapter 3: Interfaces

- Interface Summary

- AXI4 Master Interface

- AXIS Interface

- APB Configuration Interface

- Monitor Bus Interface

- CXS Network Interface

### 35.1.5	Chapter 4: Use Cases

- Network-to-Memory Transfer

- Memory-to-Network Transfer

- Descriptor Chaining

- Multi-Channel Operation

### 35.1.6	Chapter 5: Programming Model

- Descriptor Format

- Register Map

- Initialization Sequence

- Error Handling

### 35.1.7	Chapter 6: Performance

- Throughput Targets

- Latency Characteristics

- Resource Estimates

## 35.2	Quick Reference

### 35.2.1	RAPIDS Beats at a Glance

RAPIDS Beats Feature Summary

| Feature | Specification |
| --- | --- |
| Architecture | Network-to-Memory Accelerator |
| Channels | 8 independent DMA channels |
| Data Width | 512-bit (configurable) |
| Address Width | 64-bit |
| Descriptor Size | 256-bit |
| AXI Protocol | AXI4 (Read/Write Masters) |
| Network Protocol | AXI-Stream (Sink/Source) |
| Monitoring | 64-bit MonBus packets |

### 35.2.2	Interface Summary

External Interface Summary

| Interface | Type | Direction | Width | Purpose |
| --- | --- | --- | --- | --- |
| Descriptor AXI | AXI4 Master | Bidirectional | 256-bit | Descriptor fetch |
| Sink AXI | AXI4 Master | Write | 512-bit | Memory write |
| Source AXI | AXI4 Master | Read | 512-bit | Memory read |
| Sink AXIS | AXIS Slave | Input | 512-bit | Network ingress |
| Source AXIS | AXIS Master | Output | 512-bit | Network egress |
| MonBus | Custom | Output | 64-bit | Monitoring |

## 35.3	Related Documentation

- RAPIDS Beats MAS - Module Architecture Specification

- PRD.md - Product requirements

- ADDRESS_INCREMENT_PATTERNS.md - DMA addressing patterns

## 35.4	Specification Conventions

### 35.4.1	Signal Naming

- Clock: aclk (AXI clock domain)

- Reset: aresetn (active-low, async assert, sync deassert)

- AXI Signals: Standard ARM AMBA naming

- AXIS Signals: Standard AXI-Stream naming

### 35.4.2	Diagram Conventions

- Mermaid: Block diagrams and flowcharts

- WaveDrom: Timing diagrams

- Tables: Interface specifications and register maps

Last Updated: 2026-07-02 Maintained By: RAPIDS Architecture Team