RTL Design Sherpa

DDR2 / LPDDR2 Family Controller Hardware Architecture Specification 0.2

June 13, 2026

Table of Contents

1 Pumice Has Index	18

2 FUB Breakdown	18

2.1 Component Layout	18

2.2 Layer Hierarchy	18

2.3 FUB Inventory	20

2.3.1 pumice_axi4_ifc	20

2.3.2 pumice_mem_cmd_scheduler	21

2.3.3 pumice_dfi_layer	22

2.4 Data Width and Gearing	23

2.5 What Changed vs the Earlier Architecture	24

2.6 Integration	24

3 Document Information	25

3.1 DDR2 / LPDDR2 Family Controller Hardware Architecture Specification	25

3.2 Document Purpose	25

3.3 Revision History	26

3.4 Scope of This Document	26

3.5 Out of Scope	27

4 Purpose and Scope	27

4.1 Document Purpose	27

4.2 Scope	28

4.3 Design Philosophy	29

4.4 Audience	29

5 Document Conventions	30

5.1 Typographic Conventions	30

5.2 Numbering Conventions	30

5.3 Signal Direction Conventions	30

5.4 Module Diagram Conventions	30

5.5 Cross-Reference Conventions	31

6 Definitions and Acronyms	31

6.1 Acronyms	31

6.2 Defined Terms	33

7 Scope and Goals	34

7.1 High-Level Goals	34

7.2 In Scope	35

7.3 Out of Scope	36

7.4 Target Performance Envelope	37

7.5 Differentiating Features	37

7.5.1 Features Adopted from Standard Memory-Controller Patterns	39

8 Block Diagram	40

8.1 Top-Level Block Diagram	40

8.2 Data Flow Summary	41

9 Module Hierarchy	42

9.1 Hierarchy Tree	42

9.2 Layered Organization	42

9.3 Layer Groupings	43

9.4 Memtype-Conditional Logic	43

9.5 Single Clock-Domain Crossing	43

9.6 What Changed vs the Earlier Architecture	44

10 Top-Level Interfaces	44

10.1 Parameter-Dependent Widths	45

10.2 1. AXI4 Slave (Host-side)	45

10.2.1 Write Address Channel (AW)	45

10.2.2 Write Data Channel (W)	46

10.2.3 Write Response Channel (B)	46

10.2.4 Read Address Channel (AR)	46

10.2.5 Read Data Channel (R)	47

10.3 2. DFI v2.1 Master (PHY-side)	47

10.3.1 DFI Command Bus	48

10.3.2 DFI Write-Data Bus	48

10.3.3 DFI Read-Data Bus	48

10.3.4 DFI Init Handshake	49

10.4 3. CSR Register Interface (Configuration / Observation)	49

10.5 4. Clocks, Resets, and Status	49

10.6 Hierarchical Top-Level View	50

11 AXI4 Frontend	50

11.1 Burst Splitters	51

11.2 pumice_wr_intake / pumice_rd_intake	51

11.2.1 Purpose	51

11.2.2 Write intake	51

11.2.3 Read intake	51

11.2.4 Per-ID Ordering	51

11.2.5 Backpressure	52

11.3 addr_mapper	52

11.3.1 Purpose	52

11.3.2 Interfaces	52

11.3.3 Mapping Function (single knob: bank_lsb)	52

11.3.4 Classic Schemes as bank_lsb Settings	53

11.3.5 Optional Bank XOR-Hash	53

11.3.6 Relationship to the DFI BFM	53

12 Command Scheduler	54

12.1 Composition	54

12.2 pumice_cmd_arbiter	54

12.2.1 Purpose	54

12.2.2 Inputs	54

12.2.3 Outputs	55

12.2.4 Priority Function	55

12.2.5 ACT/PRE Re-Issue Guard	56

12.2.6 Page Policy (Auto-Precharge)	56

12.2.7 Issue Rate	56

12.3 Mode Register Shadow	56

12.4 v1 Scope Notes	57

13 Bank Timers and Cross-Bank Timers	57

13.1 bank_timer	57

13.1.1 Purpose	57

13.1.2 Instantiation	57

13.1.3 Timers	57

13.1.4 Row State (minimal — not an FSM)	58

13.1.5 Combinational Safe Outputs	58

13.1.6 Observability State	59

13.1.7 Refresh Interaction	59

13.2 global_timers	59

13.2.1 Purpose	59

13.2.2 Tracked Constraints	59

13.2.3 Interface	59

14 Refresh Controller	60

14.1 refresh_ctrl	60

14.1.1 Purpose	60

14.1.2 Interface	60

14.1.3 tREFI Counter and Pending Accumulator	61

14.1.4 Drain Quota	61

14.1.5 REFpb Bank Rotor	61

14.2 Arbiter-Side Refresh Sequence	61

14.3 Notes and Scope	62

15 Init Sequencer and Power-Down	62

15.1 init_sequencer	62

15.1.1 Purpose	62

15.1.2 Command Path	63

15.1.3 CSR-Backed Waits	63

15.1.4 DDR2 Sequence	63

15.1.5 LPDDR2 Sequence (fully functional)	64

15.1.6 Memtype Selection	64

15.1.7 Status Outputs	64

15.2 powerdown_ctrl	64

15.2.1 Purpose	64

15.2.2 Modes	65

15.2.3 Selection	65

15.2.4 Scope / TODO	65

16 Command Formatter and Signal Pack	65

16.1 dfi_cmd_formatter	65

16.1.1 Purpose	65

16.1.2 Input	66

16.1.3 Multi-Phase Output	66

16.1.4 DDR2 Encoding	66

16.1.5 LPDDR2 Encoding (bit-exact CA bus)	67

16.1.6 Idle Defaults	67

16.2 dfi_signal_pack	67

16.2.1 Purpose	67

16.2.2 Behavior	68

16.2.3 Multi-Cycle Commands	68

16.2.4 Verification	68

17 Write and Read Data Paths	68

17.1 Controller-Side CAMs	68

17.1.1 pumice_wr_data_cam	68

17.1.2 pumice_rd_cmd_cam	69

17.2 DFI-Side Data Movers	69

17.2.1 pumice_dfi_wr_serializer	69

17.2.2 pumice_dfi_rd_aligner	69

17.3 Response Generation	70

17.4 Runtime PHY Timing	70

18 DFI Layer and CSR	70

18.1 pumice_dfi_layer	70

18.1.1 Purpose	70

18.1.2 The Single CDC	71

18.1.3 DFI-Domain Datapath	71

18.1.4 DFI Sub-Interfaces	71

18.1.5 Runtime PHY Knobs	72

18.2 pumice_csr	72

18.2.1 Purpose	72

18.2.2 Interface	72

18.2.3 Access by Name	72

18.2.4 Address Mapping Register (ADDR_MAP @ 0x04C)	73

18.2.5 Key Configuration Registers	73

18.2.6 Narrow-Device (x16) Width Granularities	74

19 AXI4 Slave Interface	74

19.1 Data Width Is Fixed at DW	74

19.1.1 Hard width rule (compile-enforced)	74

19.2 Channels	75

19.2.1 AW (Write Address) — SoC master → controller slave	75

19.2.2 W (Write Data)	75

19.2.3 B (Write Response)	76

19.2.4 AR (Read Address)	76

19.2.5 R (Read Data)	77

19.3 Ordering Rules	77

19.4 Burst Type Support	77

19.5 Burst Length and DRAM-Burst Geometry	77

19.6 Narrow Transfers	78

19.7 Write Response Timing	78

19.8 Frontend and Backpressure	78

19.9 Host-Width Gearing	78

19.10 AXI4 Reset	78

20 DFI v2.1 Master Interface	79

20.1 Gearing and Per-Phase Packing	79

20.2 Command Interface (MC → PHY)	79

20.2.1 DDR2 Command Encoding (JESD79-2)	80

20.2.2 LPDDR2 Command Encoding (JESD209-2F)	81

20.3 Write Data Interface (MC → PHY)	81

20.4 Read Data Interface (PHY ↔ MC)	81

20.5 Status / Init Interface	81

20.6 Timing Parameters Honored	82

20.7 Clock Domains and CDC	82

21 CSR Register Interface (PeakRDL cpuif)	82

21.1 cpuif Signals	83

21.2 Access Semantics	83

21.3 Clock Domain	84

21.4 Register Map	84

21.5 Configuration Sequencing	84

22 Build-Time vs. Runtime Configuration	85

22.1 Principle	85

22.2 Why This Distinction Matters	85

22.3 The Hybrid Pattern: “Build-Time MAX + Runtime Active”	86

22.4 The “Build-Time + Runtime Mux” Pattern	86

22.5 The “Runtime Placement” Pattern	87

22.6 The “Runtime Only” Pattern	88

22.7 Configuration Quiet Points	88

22.8 Summary	88

23 Top-Level Parameter Table	89

23.1 Parameter Table	89

23.2 Memtype-Dependent Constraints	92

23.3 Timing-Configuration Sanity Checks	93

23.4 Parameter Categories	93

24 Characterization Knobs	94

24.1 Characterization Parameters	94

24.2 Sweep Plan	95

24.2.1 Benchmark Suite	96

24.2.2 Metrics Collected	96

24.2.3 Default Selection	96

24.3 Runtime Overrides via CSR	97

24.4 Observability for Sweep	97

25 Family-Wide Config Bits	98

25.1 Why a Family-Wide Registry	98

25.2 Applicability Notation	98

25.3 SCHED_TUNING (Scheduling Family Bits)	99

25.4 REFRESH_TUNING (Refresh Family Bits)	99

25.5 ADDR_MAP (Address-Mapping Family Bits)	100

25.6 INIT_TUNING (Init Family Bits)	100

25.7 POWER_TUNING (Power-State Family Bits)	101

25.8 ECC_TUNING (Future — Inline ECC)	101

25.9 RANK_TUNING (Multi-Rank Family Bits)	102

25.10 Quiet-Point Behavior	102

25.11 Bit Discovery via the ID Register	102

25.12 Per-Flavor Applicability Matrix	103

26 Clocks and Reset	104

26.1 Clock Domains	104

26.1.1 aclk	104

26.1.2 dfi_clk	104

26.2 Reset Signals	104

26.2.1 aresetn (active low, controller / AXI domain)	104

26.2.2 dfi_rstn (active low, DFI / PHY domain)	105

26.3 Reset Sequencing	105

26.3.1 Cold Boot	105

26.3.2 Warm Reset (Clock-Gated)	105

26.4 Clock Domain Crossing Summary	105

26.5 Reset and Power Considerations	106

27 Init Sequences	106

27.1 DDR2 Cold Init	106

27.2 LPDDR2 Cold Init	108

27.3 Inter-Command Waits (CSR-Backed)	108

27.4 Power-State Transitions	109

28 CSR Register Map	109

28.1 Control and Status (0x000 – 0x00F)	110

28.1.1 CTRL (0x000, R/W)	110

28.1.2 STATUS (0x004, R only)	110

28.1.3 STATUS_HISTORY (0x008, R only)	111

28.2 Timing Parameters (0x010 – 0x01F)	111

28.2.1 TIMINGS_RC_RCD_RP_RAS (0x010, R/W)	111

28.2.2 TIMINGS_RFC_REFI (0x014, R/W)	111

28.2.3 TIMINGS_RRD_FAW_WTR_CCD (0x018, R/W)	111

28.2.4 TIMINGS_CL_CWL_WR (0x01C, R/W)	111

28.3 Mode Register Values (0x020 – 0x02F)	112

28.3.1 MR0, MR1, MR2, MR3 (0x020, 0x024, 0x028, 0x02C, R/W)	112

28.4 LPDDR2-Specific (0x030 – 0x03F)	112

28.4.1 PASR_BANK_MASK_RANK0 (0x030, R/W)	112

28.4.2 PASR_SEG_MASK_RANK0 (0x034, R/W)	112

28.4.3 TEMP_DERATE_RANK0 (0x038, R only)	112

28.5 Scheduler / Refresh / Page / Addr / Init Tuning (0x040 – 0x05F)	113

28.5.1 SCHED_TUNING (0x040, R/W)	113

28.5.2 PAGE_PRED_TUNING (0x044, R/W)	113

28.5.3 REFRESH_TUNING (0x048, R/W)	113

28.5.4 ADDR_MAP (0x04C, R/W)	114

28.5.5 INIT_TUNING (0x050, R/W)	114

28.5.6 INIT_TIMING0 (0x058, R/W)	114

28.5.7 INIT_TIMING1 (0x05C, R/W)	115

28.5.8 TIMINGS_RTP_RTW (0x054, R/W)	115

28.6 DFI / PHY Timing (0x060 – 0x067)	115

28.6.1 DFI_PHASE (0x060, R/W)	115

28.6.2 PHY_TIMING (0x064, R/W)	116

28.7 Per-Bank Observation (0x080 – 0x0DF)	116

28.7.1 OBS_ROW_HIT[0..7] (0x080 + N×4, R/W with clear-on-read)	116

28.7.2 OBS_REF_LATENCY[0..7] (0x0C0 + N×4, R only)	116

28.8 System Observation (0x100 – 0x1FF)	116

28.8.1 OBS_WORDS[0..8] (0x1C0 + N×4, R only)	117

28.9 Module Identification (0xFF0 – 0xFFC)	117

28.9.1 ID (0xFF0, R only, reset = 0xD2020001)	117

28.9.2 BUILD (0xFF4, R only)	118

28.10 Notes on This Revision	118

29 Verification Strategy	118

29.1 Layered Approach	118

29.1.1 Layer 1: Module-Level Unit Tests	118

29.1.2 Layer 2: Subsystem Tests	119

29.1.3 Layer 3: End-to-End with DFI BFM Slave	120

29.1.4 Layer 4: External-Reference Cross-Validation	120

29.2 Characterization Sweep	120

29.2.1 Sweep Matrix	121

29.2.2 Sweep Outputs	121

29.2.3 Out-of-Order Scheduler Trade-Offs	122

29.3 Verification Hooks	123

29.4 Coverage Targets	123

30 Open Issues and TODOs	123

30.1 Open Issues	124

30.1.1 1. AXI4 Narrow-Burst Handling Confirmation	124

30.1.2 2. AXI4 Read-Modify-Write Strobe Handling	124

30.1.3 3. CSR Address Space Allocation	124

30.1.4 4. Per-Bank Refresh Book-Keeping Cost	124

30.1.5 5. HAPPY Predictor Hash Function	124

30.1.6 6. Self-Refresh Exit Latency	124

30.1.7 7. Init Sequencer Retry on Error	124

30.1.8 8. CSR Write Atomicity	125

30.1.9 9. Burst Splitting at Row Boundaries	125

30.1.10 10. Multi-Rank	125

30.1.11 11. ECC	125

30.1.12 12. DFI Training Sub-Interface	125

30.1.13 13. AXI Quality of Service (QoS)	125

30.1.14 14. Multi-Master AXI	125

30.1.15 15. Observation Counter Overflow	126

30.1.16 16. Retired-Architecture Modules Still on Disk (OLD/)	126

30.1.17 17. Open-Page Read-Fetches-Zero Under Gapped Reads	126

30.2 TODOs Before v0.2	126

30.3 Feature Roadmap — planned advanced modes (ordered)	126

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# 1	Pumice Has Index

Generated: 2026-07-21

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	FUB Breakdown

This section documents the actual Functional Unit Block decomposition as implemented in the rearchitected RTL: three integration layers under pumice_core, each built from leaf FUBs, driven by a PeakRDL CSR block in pumice_top.

## 2.1	Component Layout

Per the standard component layout:

pumice/
├── docs/                      # this HAS, MAS, generated PDFs
├── dv/                        # cocotb tests, BFM glue, tbclasses
└── rtl/
    ├── top/                   # pumice_top, pumice_core, pumice_top_geared
    ├── macro/                 # the three layer macros + pumice_csr.rdl + generated/
    ├── fub/                   # leaf FUBs (this section)
    ├── includes/              # shared `defines, package files (pumice_pkg)
    └── filelists/
        ├── top/               # one .f per top
        ├── macro/             # one .f per layer
        └── fub/               # one .f per FUB

Each FUB has one top SystemVerilog file named <fub>.sv with module <fub>, a filelist under filelists/fub/, and a cocotb testbench under dv/tests/.

## 2.2	Layer Hierarchy

pumice_top instantiates the PeakRDL pumice_csr block (config by name) and pumice_core, which wires the three layers:

pumice_top                                (CSR block + core)
└── pumice_core
    ├── pumice_axi4_ifc                        ("host AXI4 -> CAM-buffered requests")
    │   ├── pumice_wr_intake                   AXI4 slave wr + AW-meta FIFO + wr-data FIFO
    │   ├── pumice_rd_intake                   AXI4 slave rd + snarf probe
    │   ├── addr_mapper                        flat AXI addr -> (rank, bank, row, col)
    │   ├── pumice_wr_data_cam                 WR CAM + wr-data SRAM (fill/commit-drain/snarf)
    │   └── pumice_rd_cmd_cam                  RD CAM + rd SRAM (return-fill/drain)
    ├── pumice_mem_cmd_scheduler               ("what command to issue this cycle")
    │   ├── pumice_cmd_arbiter                 single pick core (open-page inline)
    │   ├── pumice_bank_timers (bank_timer)    per-(rank,bank) FSM-free JEDEC "safe" timers
    │   ├── global_timers                      tFAW / tRRD / tWTR / tRTW / tCCD turnaround
    │   ├── refresh_ctrl                       tREFI postponer, REFab/REFpb dispatch
    │   ├── init_sequencer                     DDR2 + LPDDR2 JEDEC MR init
    │   └── mode_register                      MR shadow -> live CL/CWL/BL/AL
    └── pumice_dfi_layer                       ("single CDC + DFI v2.1 datapath")
        ├── pumice_dfi_cdc                     the ONE clock crossing (async gaxi FIFOs)
        ├── pumice_dfi_cmd_path                unpack cmd, drive DFI via dfi_cmd_formatter
        │       (uses dfi_cmd_formatter + dfi_signal_pack)
        ├── pumice_dfi_wr_serializer           commit-drain WR CAM -> dfi_wrdata + mask
        └── pumice_dfi_rd_aligner              dfi_rddata capture -> return-fill RD CAM

Also present but not in the default top build: page_predictor and powerdown_ctrl (referenced / optional; verify against the filelists).

## 2.3	FUB Inventory

### 2.3.1	pumice_axi4_ifc

#### 2.3.1.1	pumice_wr_intake

- Purpose: AXI4 slave write engine. AW/W/B handshakes with an AW-meta FIFO and a wr-data FIFO; splits each host burst at DRAM-burst byte boundaries so each command maps to one DRAM burst.

- Key params: AXI_DATA_WIDTH, AXI_ID_WIDTH, AXI_ADDR_WIDTH, BL, NUM_ENTRIES.

- Downstream: addr_mapper, pumice_wr_data_cam.

#### 2.3.1.2	pumice_rd_intake

- Purpose: AXI4 slave read engine; splits the read burst; probes the write-data CAM for a read-your-write snarf hit (unscheduled, same-id, same-BL) before committing the read command.

- Key params: AXI_DATA_WIDTH, AXI_ID_WIDTH, AXI_ADDR_WIDTH, BL, NUM_ENTRIES.

#### 2.3.1.3	addr_mapper

- Purpose: Decode a flat AXI address into (rank, bank, row, col) using the single bank_lsb knob: the bank field slides within the byte-offset-stripped word address and the column auto-splits below (col_lo) and above (col_hi) it, with the row/rank positions invariant. An optional bank XOR-hash folds row bits (+ a seed) into the bank index. Combinational, single stage.

- Key params: AXI_ADDR_WIDTH, NUM_RANKS, NUM_BANKS, ROW_WIDTH, COL_WIDTH.

- Runtime inputs: bank_lsb_i, hash_en_i, hash_seed_i (ADDR_MAP.bank_lsb / hash_en / hash_seed).

- Notable: Mirrors the Python address-mapping class in the DV repo; the same decode drives RTL and BFM. There is no scheme selector.

#### 2.3.1.4	pumice_wr_data_cam

- Purpose: Write-data CAM. Fills each write burst into an SRAM and records the command; provides scheduler lookup / oldest / commit ports; commit-drains the burst to the DFI write serializer; and is the snarf source for read forwarding. An r_fdone fill-complete flag gates schedulability and snarf. Streaming read engine is FIFO-fed / oldest-pick — no active-slot state latch.

- Key params: NUM_ENTRIES, N_SRAM_SLOTS, NUM_RANKS, NUM_BANKS, ROW_WIDTH, BL.

#### 2.3.1.5	pumice_rd_cmd_cam

- Purpose: Read-command CAM / reorder buffer. Records read commands; return-fills DRAM read beats into an SRAM; oldest-first drain engine (gated on data-ready) streams beats back to pumice_rd_intake. No active-slot latch.

- Key params: NUM_ENTRIES, N_SRAM_SLOTS, NUM_RANKS, NUM_BANKS, ROW_WIDTH, BL.

### 2.3.2	pumice_mem_cmd_scheduler

#### 2.3.2.1	pumice_cmd_arbiter

- Purpose: The single command-pick core. Picks one abstract command per cycle from {ACT, RD/RDA, WR/WRA, PRE, REF, MRS, NOP} from the CAM lookups, gated by the per-(rank,bank) safe_* from pumice_bank_timers and the turnaround windows from global_timers. The open-page decision is inline.

- Page policy: OPEN (row-hit reuse + explicit PRE on miss), CLOSE (auto-pre on every column op via RDA/WRA), or HAPPY_HYBRID (predictor-selected). Programmed via REFRESH_TUNING.page_policy_or.

- Key params: NUM_ENTRIES, NUM_RANKS, NUM_BANKS, AGE_WIDTH.

- Notable: A combinational picker with registered feedback — always macro-test it, since registered-feedback latency created double-issue hazards caught only at the layer level.

#### 2.3.2.2	pumice_bank_timers (instantiates bank_timer)

- Purpose: Stamps one bank_timer per (rank, bank) and fans the arbiter’s command-event strobes to the addressed instance. bank_timer is FSM-free: preset/decrement countdown timers (tRCD, tRAS, tRC, tRP, precharge-block) + a row-open register + a single auto-precharge bit. The per-command safe_* outputs are combinational off the timers (one register stage), so the arbiter sees the just-issued command’s effect one cycle later with no multi-stage lag.

- Key params: NUM_RANKS, NUM_BANKS, ROW_WIDTH.

- Runtime inputs: t_rcd_i, t_rp_i, t_ras_i, t_rc_i, t_wr_i, t_rtp_i.

#### 2.3.2.3	global_timers

- Purpose: Cross-bank / cross-rank turnaround windows: tFAW (4-ACT window), tRRD, tWTR, tRTW, tCCD. ANDed by the arbiter.

- Runtime inputs: t_faw_i, t_rrd_i, t_wtr_i, t_rtw_i, t_ccd_i.

#### 2.3.2.4	refresh_ctrl

- Purpose: tREFI down-counter; postponed-refresh accumulator (JEDEC max 8); refresh-pending signal to the arbiter; REFab / REFpb dispatch (REFRESH_TUNING.refpb_policy_or).

- Runtime inputs: t_refi_i, refresh_burst_i.

#### 2.3.2.5	init_sequencer

- Purpose: Cold-boot engine that runs the memtype-specific JEDEC MR/init sequence (DDR2 and LPDDR2), driving CKE / RESET_N and MR-write strobes into mode_register, and holds off traffic until init completes.

- Runtime inputs: t_init_wait_i, t_dll_wait_i, t_mrd_wait_i, t_rp_wait_i, t_rfc_wait_i, memtype_i.

#### 2.3.2.6	mode_register

- Purpose: Per-rank Mode Register shadow + live decode to CL / CWL / BL / AL (memtype-dependent), plus drive-strength and ODT-rule outputs consumed by the DFI layer.

- Key params: NUM_RANKS, MAX_MR_IDX (17; covers DDR2 MR0..3 and LPDDR2 MR0..16).

### 2.3.3	pumice_dfi_layer

#### 2.3.3.1	pumice_dfi_cdc

- Purpose: The single controller-to-PHY clock crossing, built from asynchronous gaxi FIFOs (command, write-data, read-data). One FIFO word is one DFI cycle, so the datapaths are bubble-free.

- Key params: CMD_FIFO_DEPTH, WD_FIFO_DEPTH, RD_FIFO_DEPTH, N_FLOP_CROSS.

#### 2.3.3.2	pumice_dfi_cmd_path (uses dfi_cmd_formatter + dfi_signal_pack)

- Purpose: DFI-domain command path. Pops the abstract command stream from the CDC FIFO, unpacks {op, rank, bank, row, col, ap}, and drives the multi-phase DFI command bus via dfi_cmd_formatter; emits wr_fire / rd_fire strobes so the serializer / aligner can schedule their data phases.

- dfi_cmd_formatter: JEDEC command encoding into DFI wires — DDR2 drives cs_n / ras_n / cas_n / we_n / address / bank (including the A10 auto-precharge bit and ODT rule); LPDDR2 packs the 10-bit CA-bus command (two edges) onto dfi_address, per a memtype branch. Swap this when moving DFI generations.

- dfi_signal_pack: multi-phase aggregation — widens each DFI control bus to per-phase × DFI_RATE.

#### 2.3.3.3	pumice_dfi_wr_serializer

- Purpose: On wr_fire, commit-drains the write burst from pumice_wr_data_cam’s SRAM and drives dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask with PHY alignment.

- AXI ↔ DFI mask polarity: AXI wstrb=1 means write; DFI mask=1 means do-not-write → dfi_wrdata_mask = ~wstrb.

#### 2.3.3.4	pumice_dfi_rd_aligner

- Purpose: Drives dfi_rddata_en t_rddata_en cycles after a READ command; captures dfi_rddata beats; return-fills them into pumice_rd_cmd_cam.

- Runtime inputs: t_rddata_en_i, t_phy_wrlat_i, rd_phase_i, wr_phase_i.

## 2.4	Data Width and Gearing

Two distinct concepts:

- Internal DFI geometry. DW = DRAM_BEAT_WIDTH * DFI_RATE (128 default). One AXI beat is one DFI word is DFI_RATE DRAM beats; one AXI burst is one DRAM burst. The DW → per-phase split happens in dfi_signal_pack / the DFI layer. The host AXI data width equals DW at pumice_top.

- Host-width freedom. The optional pumice_top_geared wrapper adds a free HOST_AXI_DATA_WIDTH and inserts the repository’s formally-verified axi4_dwidth_converter_wr / _rd between a host-width slave and the fixed-DW core. HOST_AXI_DATA_WIDTH == DW is a bit-identical generate bypass. Verified host ∈ {64, 128, 256}.

## 2.5	What Changed vs the Earlier Architecture

The controller was rearchitected from an earlier FSM-based decomposition. The following blocks were retired; their behavior now lives in the FUBs shown:

| Retired block | Replaced by |
| --- | --- |
| txn_queue | the two CAMs: pumice_wr_data_cam + pumice_rd_cmd_cam |
| bank_machine | FSM-free bank_timer, stamped by pumice_bank_timers |
| xbank_timers | global_timers (turnaround) + pumice_bank_timers (per-bank) |
| cmd_encoder | dfi_cmd_formatter (+ dfi_signal_pack) |
| odt_ctrl | ODT inside dfi_cmd_formatter / mode_register (no standalone block) |
| page_predictor (standalone) | open-page decision inline in pumice_cmd_arbiter (page_predictor.sv optional) |
| wr_cmd_cam | pumice_wr_data_cam |
| scheduler | pumice_mem_cmd_scheduler (pumice_cmd_arbiter + timers + refresh + init) |
| gear_dfi | pumice_dfi_layer |
| axi_frontend / axi_intake / *_macro-as-architecture | pumice_axi4_ifc, generated pumice_csr, pumice_core |

The old names may still appear in retired sentinel tests; they are not part of the current architecture.

## 2.6	Integration

The three layer macros are structural wiring; behavioral logic lives in the leaf FUBs. The principal wiring concerns:

- Arbiter ↔ timing fan-out: pumice_bank_timers exposes per-(rank,bank) safe_* off single-stage timers; global_timers exposes turnaround-OK signals. pumice_cmd_arbiter reduces these into one command per cycle.

- CAM ↔ DFI-datapath coupling: pumice_wr_data_cam’s commit-drain feeds pumice_dfi_wr_serializer; pumice_dfi_rd_aligner’s return-fill advances pumice_rd_cmd_cam.

- The single CDC: everything up to pumice_dfi_cdc is on aclk; the command path, serializer, and aligner are on dfi_clk.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Document Information

## 3.1	DDR2 / LPDDR2 Family Controller Hardware Architecture Specification

Document Number: DDR2-LPDDR2-HAS-001 Version: 0.4 Status: Draft - reconciled with rearchitected RTL Classification: Open Source - Apache 2.0 License

## 3.2	Document Purpose

This Hardware Architecture Specification (HAS) provides a high-level architectural overview of the DDR2 / LPDDR2 unified memory controller family. It describes the system-level design, external interfaces, characterization parameters, and integration requirements without detailing internal implementation specifics.

Target Audience:

- System architects evaluating the controller for SoC integration

- Hardware engineers planning RTL implementation

- Verification engineers planning system-level testing

- Software engineers developing low-level memory configuration

Companion Documents:

- DDR2/LPDDR2 Family Controller Pre-Architecture Spec (../pre-aspec.md) — bullet-form architecture rationale

- DDR2/LPDDR2 Paging and Refresh Notes (../paging-refresh-notes.md) — research-derived parameter sources

## 3.3	Revision History

| Version | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.1 | 2026-06-13 | RTL Design Sherpa | First sketch — module decomposition |
| 0.2 | 2026-06-14 | RTL Design Sherpa | Top-level interface port list (§2.4), FUB SWAG (§2.5), family-wide config-bit registry (§5.4), multi-rank support across §2 / §3.3 / §3.4 / §3.6 / §5.2 / §6.3 |
| 0.3 | 2026-07-07 | RTL Design Sherpa | Narrow-device (x16) support: distinguish DRAM beat vs physical device word (DRAM_DEVICE_WIDTH); device-word column granularity + burst-length scaling; DFI_PHASE (rd/wr phase) CSR. Fixes on-silicon DDR2 read failure (Nexys A7 x16 bring-up). |
| 0.4 | 2026-07-12 | RTL Design Sherpa | Full reconciliation with the rearchitected RTL: three-layer core (pumice_axi4_ifc / pumice_mem_cmd_scheduler / pumice_dfi_layer); FSM-free bank_timer; de-FSM’d CAMs; single ADDR_MAP.bank_lsb address knob (scheme selector retired); fully-functional LPDDR2 (bit-exact JESD209-2F CA + JEDEC MR init); host-width pumice_top_geared gearing; block/hierarchy diagrams regenerated. |

## 3.4	Scope of This Document

This HAS describes a single parameterized memory controller covering DDR2 and LPDDR2. It defines:

- External interfaces (AXI4 slave, DFI v2.1 master, APB CSR slave)

- Module-level decomposition and behavioral specification

- Build-time and run-time configuration parameters

- Initialization, power-state, and refresh policies

- Verification strategy and characterization plan

It does not define:

- Bit-level pin assignments (the SystemVerilog port list in regs/pumice_ctrl_ports.svh is the canonical wire-level source)

- Detailed timing diagrams (deferred to v0.3)

- Verilog package skeletons or RTL stubs

- Floorplan or layout guidance

## 3.5	Out of Scope

The following features are explicitly out of scope for this version of the controller and will be addressed in higher-generation family controllers or future revisions:

- Inline ECC

- DFI training, frequency-change, and low-power sub-interfaces

- AXI exclusive access semantics

- Bank groups (not present in DDR2 / LPDDR2)

- Command/Address parity, CRC, Data Bus Inversion (not present in DDR2 / LPDDR2)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Purpose and Scope

## 4.1	Document Purpose

This Hardware Architecture Specification (HAS) defines the high-level architecture of the DDR2 / LPDDR2 Family Memory Controller. It serves as the primary reference for:

- System Integration — understanding the controller’s external interfaces and system-level requirements

- RTL Implementation Planning — module decomposition, interface contracts, and parameter sweeps

- Verification Planning — system-level test scenarios and the characterization sweep matrix

- Driver Development — programming model and APB register interface

This document complements the upcoming Micro-Architecture Specification (MAS), which will provide detailed block-level implementation specifics, signal-level pinouts, and timing diagrams.

## 4.2	Scope

This HAS covers a single unified controller that supports both DDR2 and LPDDR2 memory. The memory type is a runtime CSR selection (PHY_TIMING.memtype: 0 = DDR2, 1 = LPDDR2). The controller is built as three layers under pumice_core:

- pumice_axi4_ifc — AXI4 slave host interface: dumb write/read intakes, an address mapper, a write-data CAM (write buffer + read-your-write snarf source), and a read-command CAM (read reorder buffer).

- pumice_mem_cmd_scheduler — the command-scheduling layer: a single command arbiter (pumice_cmd_arbiter, open-page decision inline), per-(rank,bank) FSM-free JEDEC “safe” timers (pumice_bank_timers / bank_timer), global turnaround timers (tFAW / tRRD / tWTR / tRTW / tCCD), a refresh controller, an init sequencer (DDR2 and LPDDR2 JEDEC MR init), and a mode-register shadow (CL / CWL / BL / AL decode).

- pumice_dfi_layer — a single controller-to-PHY clock crossing (pumice_dfi_cdc, async FIFOs) plus the DFI-domain command path (dfi_cmd_formatter / dfi_signal_pack), write serializer, and read aligner, presenting the DFI v2.1 pin bus.

Configuration (timings, phases, page policy, address map, memtype) is delivered by name from a PeakRDL-generated CSR register block (pumice_csr) instantiated in pumice_top. An optional outer wrapper, pumice_top_geared, gives the host a free AXI data width by inserting the repository’s formally-verified axi4_dwidth_converter_wr / _rd between a host-width slave and the fixed-width core.

The differences between DDR2 and LPDDR2 are the command encoding (DDR2 ras/cas/we vs the LPDDR2 10-bit CA bus, both in dfi_cmd_formatter), the init MR sequence in init_sequencer, and a small number of mode-register decode differences. Both memtypes pass the full simulation suite; everything else is shared.

## 4.3	Design Philosophy

This controller is intended as a characterization-first design. Algorithmic choices that have meaningful research-backed alternatives — page policy, refresh policy, refresh deferral depth, address mapping — are exposed as CSR fields rather than hardcoded. The verification plan includes a benchmark sweep over representative AXI traffic patterns to pick defaults from data, not assumption.

The principal characterization knobs, all programmed by name through the CSR block, are:

- Page policy (REFRESH_TUNING.page_policy_or: OPEN / CLOSE / HAPPY_HYBRID) — where HAPPY_HYBRID is the address-bit-based page-closure predictor from Ghasempour et al. (2015). The open-page decision itself is inline in pumice_cmd_arbiter.

- Refresh policy (REFRESH_TUNING.refpb_policy_or: ROUND_ROBIN / OLDEST_FIRST / DARP) — where DARP is the dynamic access refresh parallelization scheme from Chang et al. (HPCA 2014).

- Address map (ADDR_MAP.bank_lsb, hash_en, hash_seed) — a single knob that slides the bank field within the word address, subsuming the classic ROW_MAJOR / BANK_INTERLEAVE schemes, with an optional bank XOR-hash.

These are described in detail in Chapter 5.

## 4.4	Audience

This document assumes the reader is familiar with:

- AXI4 protocol semantics (handshakes, burst types, ID-based ordering)

- DFI v2.1 specification (control / write-data / read-data sub-interfaces)

- JEDEC DDR2 (JESD79-2F) and LPDDR2 (JESD209-2F) command tables

- Basic memory-controller concepts (open vs closed page policy, refresh deadlines, bank state machines)

Readers new to these topics should first consult the companion paging-refresh-notes.md in the family working area for an entry-point bibliography.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Document Conventions

## 5.1	Typographic Conventions

| Style | Meaning |
| --- | --- |
| monospace | Module names, parameter names, signal names, register names |
| bold | Defined terms on first introduction; important warnings |
| italic | Emphasis; placeholder values to be replaced |
| NUM_RANKS | Top-level build-time parameter (all caps) |
| bank_timer | RTL module name (lowercase with underscores) |
| tRC, tRCD | JEDEC timing parameters (as in JESD79-2 / JESD209-2) |

## 5.2	Numbering Conventions

- Decimal numbers have no prefix: 16, 256.

- Hexadecimal numbers are prefixed with 0x: 0x800, 0xFF.

- Binary literals use Verilog notation when needed: 8'b1011_0010.

- Bit ranges use the convention [MSB:LSB], e.g., dfi_address[19:0].

## 5.3	Signal Direction Conventions

For DFI signals, direction is from the memory controller’s perspective:

- MC → PHY — driven by the controller

- PHY → MC — driven by the PHY (sampled by controller)

For AXI signals, direction follows the standard AXI4 convention with the controller as the slave.

## 5.4	Module Diagram Conventions

ASCII block diagrams in this document use the following conventions:

All diagrams in this document are rendered from Mermaid source files in assets/mermaid/. The Mermaid source files (.mmd) are linked alongside each rendered image so the diagrams can be edited and re-rendered.

- Rectangular boxes with sharp corners denote RTL modules.

- Subgraph rectangles group modules by functional layer.

- Solid arrows show primary data or control flow.

- Dashed arrows show configuration or observation paths (typically from the CSR slave to other modules).

- Trapezoidal nodes denote external interface ports (AXI, DFI, APB).

## 5.5	Cross-Reference Conventions

- References to other chapters: “see Chapter 4”

- References to specific sections: “see §4.1” or “see Section 4.1”

- References to other documents: “see pre-aspec.md” (relative path)

- References to external standards: “per JESD79-2F §x.y”

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Definitions and Acronyms

## 6.1	Acronyms

| Acronym | Expansion |
| --- | --- |
| AHB | Advanced High-performance Bus (ARM AMBA) |
| APB | Advanced Peripheral Bus (ARM AMBA) |
| AP | Auto-Precharge (DRAM column command modifier) |
| APD | Active Power Down |
| AXI | Advanced eXtensible Interface (ARM AMBA) |
| AxLEN | AXI burst length — the AWLEN / ARLEN signals (beats = AxLEN+1). NOT the DRAM BL: one AXI burst at core width is (AxLEN+1)/DFI_RATE DFI words and splits into fixed-BL DRAM commands. Use “AxLEN” whenever burst length refers to the AXI side, to avoid confusion with the JEDEC BL below. |
| BL | Burst Length — JEDEC/DRAM burst (MR0 device beats: BL4/BL8). Distinct from AxLEN. |
| CA | Command/Address (LPDDR2/3/4 multiplexed bus) |
| CDC | Clock Domain Crossing |
| CL | CAS Latency |
| CMD | Command |
| CSR | Control/Status Register |
| CWL | CAS Write Latency |
| DARP | Dynamic Access Refresh Parallelization |
| DBI | Data Bus Inversion |
| DDR | Double Data Rate |
| DDR2 | Double Data Rate 2 (JESD79-2) |
| DFI | DDR PHY Interface |
| DPD | Deep Power Down (LPDDR2) |
| FSM | Finite State Machine |
| FR-FCFS | First-Ready, First-Come-First-Served |
| HAPPY | Hybrid Address-based Page Policy in DRAMs |
| HAS | Hardware Architecture Specification |
| LPDDR2 | Low Power DDR 2 (JESD209-2) |
| MAS | Micro-Architecture Specification |
| MC | Memory Controller |
| MR / MRS | Mode Register / Mode Register Set command |
| EMR / EMRS | Extended Mode Register / Extended MRS |
| NOP | No Operation |
| ODT | On-Die Termination |
| OOO | Out-of-Order |
| PASR | Partial Array Self-Refresh (LPDDR2 feature) |
| PHY | Physical Layer (DRAM electrical interface) |
| PRE | Precharge command |
| PREA | Precharge All Banks |
| RD | Read column command |
| RDA | Read with Auto-Precharge |
| REF | Refresh command |
| REFab | All-Bank Refresh (LPDDR2) |
| REFpb | Per-Bank Refresh (LPDDR2) |
| SDRAM | Synchronous Dynamic RAM |
| SoC | System on Chip |
| SR | Self-Refresh |
| TCSR | Temperature-Compensated Self-Refresh (LPDDR2) |
| WR | Write column command |
| WRA | Write with Auto-Precharge |
| ZQ | Impedance Calibration (ZQ Calibration) |

## 6.2	Defined Terms

Bank Timer — A per-bank JEDEC “safe” timing tracker (bank_timer). It is not a finite state machine: it is a set of preset/decrement countdown timers (tRCD, tRAS, tRC, tRP, precharge-block) plus a small open-row register and a single auto-precharge bit. The per-command safe_* outputs are combinational off the timers (one register stage). pumice_bank_timers stamps one instance per (rank, bank).

Characterization Knob — A build-time or runtime CSR field intentionally exposed for performance sweeping during system characterization. Page policy, refresh policy, and address map are the principal knobs in this controller.

Command Formatter — dfi_cmd_formatter: the module that translates the controller’s abstract command record into DFI wire signals. A single module with a memtype branch — DDR2 drives ras/cas/we; LPDDR2 packs the 10-bit CA bus (two edges) onto dfi_address. dfi_signal_pack performs the final per-phase wire packing.

Command Arbiter — pumice_cmd_arbiter: the single command-pick core. It scans CAM readiness against the bank and global timers, applies the page policy (the open-page decision is inline here), and emits one abstract DRAM command per cycle.

FR-FCFS — First-Ready, First-Come-First-Served. The baseline DRAM scheduling policy: ready commands beat unready, row-hits beat row-misses, older entries beat younger on ties. Established by Rixner et al. (ISCA 2000).

Gearing (two senses) — (1) The internal DFI-rate split: one AXI/DFI word is spread across DFI_RATE DRAM beats and packed to per-phase buses in dfi_signal_pack / the DFI layer. (2) Host-width gearing: pumice_top_geared inserts formally-verified AXI data-width converters so the host AXI data width can differ from the core width.

Init Sequencer — init_sequencer: the cold-boot engine that issues the memtype-specific JEDEC MR/init sequence, driving MR loads and commands and holding off traffic until init completes.

Page Hit / Row Hit — An access whose target row matches the currently open row in the target bank. Avoids the tRP + tRCD penalty.

Page Conflict / Row Miss — An access whose target row differs from the currently open row in the target bank. Pays tRP + tRCD before the access can begin.

Page Policy — The strategy for when to close an open row. Programmed via REFRESH_TUNING.page_policy_or (OPEN / CLOSE / HAPPY_HYBRID); the decision is inline in pumice_cmd_arbiter.

Command CAMs — The two content-addressable buffers that hold in-flight requests between the AXI interface and the scheduler: pumice_wr_data_cam (write data buffer + snarf source) and pumice_rd_cmd_cam (read reorder buffer). Both store burst data in an SRAM with FIFO-fed / oldest-pick streaming read engines and no active-slot state latch.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Scope and Goals

## 7.1	High-Level Goals

This controller provides a single parameterized RTL implementation supporting both DDR2 and LPDDR2 memory devices via DFI v2.1. Primary design goals:

- Unified engine, one command formatter with a memtype branch. DDR2 and LPDDR2 differ only at the wire-encoding layer (ras/cas/we vs the 10-bit CA bus, both in dfi_cmd_formatter) and in the init MR sequence; the command arbiter, bank/global timers, refresh policy, init sequencer, address mapper, and AXI interface are shared between both targets. Memtype is a runtime CSR selection (PHY_TIMING.memtype).

- Embedded / mobile / mid-range SoC targets. Not optimized for high-end server DRAM (DDR5, RDIMM, multi-rank). Optimized for power efficiency and area economy.

- Characterization-first parameterization. Algorithmic choices with research-backed alternatives are exposed as build-time parameters so the design can be swept on real workloads before defaults are locked.

- Verification-friendly. Mirrors the data structures and conventions of the DDR2 / LPDDR2 DFI BFM in the DV repository so the same address mapping, timing parameters, and command encodings can drive both RTL and simulation.

## 7.2	In Scope

The following features are explicitly within scope for this controller:

- AXI4 slave at the fixed core data width DW = DRAM_BEAT_WIDTH * DFI_RATE; an optional pumice_top_geared wrapper gives the host a free AXI data width (verified host ∈ {64, 128, 256}) via formally-verified data-width converters

- DFI v2.1 master driving the command, write-data, and read-data sub-interfaces

- CSR register block (PeakRDL-generated pumice_csr, passthrough cpuif) for configuration and observation, programmed by name

- DDR2 and LPDDR2 device support selected at runtime by the PHY_TIMING.memtype CSR field

- DFI frequency ratio of 1, 2, or 4 phases via the DFI_RATE parameter

- Multi-rank operation — 1, 2, or 4 ranks via NUM_RANKS parameter; per-rank CS_n, CKE, ODT, and refresh tracking; rank-aware ODT cross-termination rules per JEDEC

- Page policies: OPEN, CLOSE, HAPPY hybrid predictor (CSR-selected; open-page decision inline in pumice_cmd_arbiter)

- Per-bank refresh for LPDDR2 with DARP-style scheduling (parameterized)

- All-bank refresh for DDR2 (and as LPDDR2 fallback)

- Elastic refresh deferral up to JEDEC ceiling of 8× tREFI (parameterized)

- Partial-array self-refresh (PASR) for LPDDR2 with CSR-configurable mask

- Temperature-compensated refresh (LPDDR2 MR4 derating)

- Cold-boot initialization via microprogram step tables

- Power state management (Active, Active Power-Down, Self-Refresh, Deep Power Down)

- Bring-up observability via CSR-exposed counters (row-hit rate, refresh latency, queue depth)

## 7.3	Out of Scope

Explicitly not addressed in this controller version:

| Feature | Reason |
| --- | --- |
| Inline ECC | Out-of-controller responsibility (SoC sideband) |
| DFI training sub-interface | Not required for v2.1 DDR2 / LPDDR2 operation |
| DFI frequency-change protocol | No use case for this generation |
| DFI low-power sub-interface | Power-state FSM uses CKE only |
| AXI exclusive accesses | Embedded targets don’t require |
| Bank groups | Not present in DDR2 / LPDDR2 (introduced in DDR4 / LPDDR4) |
| Command/Address parity | Not present in DDR2 / LPDDR2 (introduced in DDR4 / LPDDR4) |
| Write CRC | Not present in DDR2 / LPDDR2 |
| Data Bus Inversion (DBI) | Not present in DDR2 / LPDDR2 |
| DDR3 write leveling | Not present in DDR2 / LPDDR2 (LPDDR3 introduces) |
| DDR3 ZQCS / ZQCL on column | DDR2 uses OCD calibration; LPDDR2 uses MR10 ZQ |
| Higher DDR / LPDDR generations | Will be addressed in DDR3-LPDDR3 and DDR4-LPDDR4 controllers |

## 7.4	Target Performance Envelope

| Metric | Target |
| --- | --- |
| DRAM data rate | Up to DDR2-800 / LPDDR2-1066 |
| AXI clock | Up to 200 MHz (typical embedded SoC) |
| Sustained read bandwidth | ≥ 70% peak on stream workloads |
| 99th-percentile read latency | ≤ 200 ns (typical) |
| Refresh blocking overhead | < 5% (LPDDR2 with DARP); < 8% (DDR2) |
| Area | < 50K gates (excluding HAPPY predictor) |

These are target envelopes for v1; not guarantees. They will be validated and refined during the characterization sweep.

## 7.5	Differentiating Features

This controller is informed by published memory-controller literature and the open-source memory-controller ecosystem (LiteDRAM in particular — used as the cross-validation reference; see §6.4), but it is not a re-implementation of any existing controller. The design’s distinguishing features:

- Centralized FR-FCFS scheduler with full out-of-order issue. A single scheduler scans the full transaction queue every cycle, considering all banks’ readiness and row-hit state in one decision. This is closer to the commercial-vendor pattern (Synopsys, Cadence, Xilinx MIG) than to most open-source RTL controllers — open-source controllers typically expose OoO only at the cross-bank level (a per-bank FIFO model where bank parallelism is the only source of reordering). True per-port within-stream reordering — where a younger row-hit request bypasses an older row-conflict request — is uncommon outside vendor IP. AXI4 per-ID ordering is honored at the response side regardless. For real-time / safety-critical / formally-verifiable systems, OoO reordering can be disabled at runtime via the SCHED_TUNING.force_inorder CSR field (§3.2); the scheduler degrades to a first-ready FIFO that preserves arrival order at the cost of 10–25% sustained bandwidth on streaming workloads.

- Multi-rank support with JEDEC-compliant cross-rank ODT. Most open-source DDR2/3 controllers are single-rank only — LiteDRAM’s default config, UberDDR3, and most academic prototypes assume one rank per channel. This controller supports 1, 2, or 4 ranks via the NUM_RANKS build parameter. Per-rank CS_n, CKE, and ODT signals are driven; the rank-aware refresh manager tracks last_ref_age per (rank, bank); and the cross-rank ODT termination rules (read from rank N → ODT-high on rank ≠N during the read window; write to rank N → ODT-high on rank N during the write window) are handled inside dfi_cmd_formatter / mode_register (no standalone ODT block) per JESD79-2 §x.y. Multi-rank is what makes the controller usable on real DIMM-class platforms rather than just on-board point-to-point DRAM.

- Unified DDR2 + LPDDR2 engine with one command formatter. A single controller core supports both memtypes via the runtime PHY_TIMING.memtype CSR field. The LPDDR2 path includes the 10-bit CA bus encoding (JESD209-2F Table 60; packed two edges onto dfi_address in dfi_cmd_formatter), PASR mask handling (MR16/17), per-bank refresh (REFpb), temperature-compensated refresh (MR4 derating), and Deep Power Down — none of which are present in the open-source DDR2/3/4 reference controllers. LPDDR2 is fully functional (reads and writes) and passes the full simulation suite.

- DARP-style per-bank refresh scheduling — picks the idle bank with the longest time since last refresh, falling back to oldest-first when no banks are idle. Based on Chang et al. (HPCA 2014); not present in available open-source controllers.

- Two-stage page management — exact lookahead first, history-based fallback second.

  - Stage 1: a lookahead window (SCHED_TUNING.lookahead_active, 0 disables) provides exact auto-precharge decisions when the next same-bank request is already visible to the arbiter.

  - Stage 2: when lookahead is inconclusive (shallow queue or bursty traffic), the HAPPY hybrid page predictor (Ghasempour et al. 2015) uses address-bit-hashed saturating counters to predict the next access. Open-source controllers typically have either lookahead OR a fixed page policy. The two-stage combination is original to this design.

- Characterization-first parameterization. Page policy, lookahead depth, refresh policy, refresh deferral, and address mapping are all runtime CSR fields with a defined sweep matrix (see §5 and §6.4). Most published controllers expose only a subset of these; few expose them all.

- Closed-page policy as an explicit option. REFRESH_TUNING.page_policy_or ∈ {OPEN, CLOSE, HAPPY_HYBRID}. Most open-source controllers expose only on/off auto-precharge (effectively OPEN with optional lookahead). The CLOSE option is useful for adversarial / security-sensitive workloads where row-state side channels matter.

- Single-knob address mapping. ADDR_MAP.bank_lsb slides the bank field within the byte-offset-stripped word address; the column auto-splits below (col_lo) and above (col_hi) it while the row/rank positions stay invariant. The classic schemes are just settings of this one knob (bank_lsb == COL_WIDTH = ROW_MAJOR; bank_lsb == log2(cols/burst) = maximum bank interleave), and an optional bank XOR-hash (ADDR_MAP.hash_en / hash_seed) folds row bits into the bank index to defeat pathological row-conflict strides. There is no separate scheme selector.

- AXI4-first interface with ID-aware out-of-order completion. The AXI integration is a first-class design constraint rather than a wrapper on a native protocol.

- Mirrored verification address-mapping object. The addr_mapper mirrors a Python address-mapping class used by the DFI BFM, so the same mapping function drives both RTL and simulation. This eliminates a common source of address-decode bugs between RTL and verification.

### 7.5.1	Features Adopted from Standard Memory-Controller Patterns

These design patterns are widespread in the memory-controller literature and the open-source controllers we examined. Adopting them is the right call; they are noted here so the design rationale is transparent:

- Per-bank FSM-free JEDEC “safe” timers (bank_timer, stamped per (rank,bank) by pumice_bank_timers) + a central cross-bank/rank timer pool (global_timers: tRRD / tFAW / tWTR / tRTW / tCCD) — the standard bank/global split, here without a per-bank state machine.

- FR-FCFS scheduler baseline (Rixner et al. ISCA 2000) — the academic baseline; realized in pumice_cmd_arbiter.

- Elastic Refresh deferral (Stuecheli et al. MICRO 2010) — postponer with deterministic back-to-back drain, in refresh_ctrl.

- Refresh arbitrated against bank readiness — the arbiter waits for the addressed banks to be safe before issuing REF; established pattern.

- Periodic ZQCS piggybacked on the refresh window — established pattern for systems with thermal drift.

- Init sequencer issuing the memtype-specific JEDEC MR/init sequence (init_sequencer) — established pattern.

The design rationale for adopting versus extending each is documented in the relevant section.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Block Diagram

## 8.1	Top-Level Block Diagram

The controller’s top-level data and control flow is shown below. AXI4 traffic enters at top-left into pumice_axi4_ifc, which maps addresses and pushes per-direction records into its two CAMs (pumice_wr_data_cam, pumice_rd_cmd_cam). The pumice_mem_cmd_scheduler layer queries the CAMs each cycle and picks the next abstract command (pumice_cmd_arbiter against the bank and global timers). The pumice_dfi_layer crosses the single controller-to-PHY clock boundary (pumice_dfi_cdc) and formats the chosen command into DFI v2.1 wires (dfi_cmd_formatter / dfi_signal_pack), while its write serializer and read aligner move write beats out and return read beats in alignment with the scheduled commands. pumice_top instantiates pumice_core (these three layers) plus the PeakRDL pumice_csr block that drives all configuration by name.

Top-Level Block Diagram

Source: 01_block_diagram.mmd

## 8.2	Data Flow Summary

Write path: 1. AXI master issues an AW/W transaction; pumice_wr_intake (an AXI4 slave write engine + AW-meta FIFO + wr-data FIFO) accepts it and splits the host burst at DRAM-burst boundaries. 2. addr_mapper translates the flat AXI address into (rank, bank, row, col) using bank_lsb (+ optional hash). 3. pumice_wr_data_cam fills the write burst into its SRAM and records the command; the r_fdone fill-complete flag gates schedulability and snarf. 4. pumice_cmd_arbiter queries the wr/rd CAMs against the per-(rank,bank) safe_* outputs from pumice_bank_timers and the turnaround windows from global_timers; it picks the next abstract command (ACT, WR/WRA, RD/RDA, PRE, REF, MRS, NOP) and applies the page policy (open-page decision inline). 5. In pumice_dfi_layer, the command crosses the CDC and dfi_cmd_formatter encodes it into DFI cs_n / ras_n / cas_n / we_n / address / bank wires (DDR2) or the packed CA bus (LPDDR2); closed-page uses WRA (auto-precharge). 6. dfi_signal_pack aggregates the per-phase DFI control bus. 7. On the write-fire strobe, the DFI write serializer commit-drains the burst from pumice_wr_data_cam’s SRAM and drives dfi_wrdata / dfi_wrdata_en / dfi_wrdata_mask with PHY alignment. 8. On retire, the write CAM slot is freed and pumice_wr_intake returns the B-response to the AXI master.

Read path: 1. AXI master issues an AR transaction; pumice_rd_intake accepts and splits the burst. 2. Address mapping identical to the write path; pumice_rd_cmd_cam records the read command. pumice_rd_intake probes the write-data CAM: on a snarf hit (unscheduled, same-id, same-BL) the read is served directly from the write CAM SRAM without going to DRAM (read-your-write forwarding). 3. pumice_cmd_arbiter issues ACT, then RD / RDA via the DFI layer. 4. The DFI read aligner drives dfi_rddata_en t_rddata_en cycles after the RD command and captures dfi_rddata beats for the burst. 5. Returned beats fill pumice_rd_cmd_cam’s SRAM; its oldest-first drain engine (gated on data-ready) streams them back out, tagged with the original AXI ID, on the R channel.

Refresh path: 1. refresh_ctrl’s tREFI counter elapses, incrementing the refresh_pending accumulator (JEDEC max 8 postponed). 2. When refresh_pending exceeds the soft threshold, refresh becomes highest-priority in the arbiter. 3. For REFab: the arbiter waits for the addressed banks to be safe (via pumice_bank_timers), then issues REF; the affected timers reload. 4. LPDDR2 per-bank refresh (REFpb) selection is driven by REFRESH_TUNING.refpb_policy_or.

Init / power path: 1. On cold reset, init_sequencer holds off AXI traffic. 2. init_sequencer executes the memtype-specific JEDEC MR/init sequence, driving CKE / RESET_N and issuing MR-write strobes into mode_register. mode_register propagates live CL / CWL / BL / AL to the DFI layer. 3. On completion, init_done_o asserts and the arbiter begins servicing AXI traffic. 4. Power-down / self-refresh entry (Active / APD / SR / DPD) is managed by powerdown_ctrl (present but optional; not in the default top build), with SR-entry coordinated with refresh_ctrl.

Detailed per-module behavior follows in Chapter 3.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Module Hierarchy

The controller is decomposed into three layers under pumice_core, each built from leaf FUBs. Each FUB is independently verified at the unit level; each layer is verified as an integration (“macro”) unit. See FUB Breakdown for the per-FUB role descriptions and Chapter 3 for the behavioral details.

## 9.1	Hierarchy Tree

Module Hierarchy

Source: 02_module_hierarchy.mmd

## 9.2	Layered Organization

SoC level
└── pumice_top_geared            ← OPTIONAL host-width wrapper
    │                              (axi4_dwidth_converter_wr/_rd when HOST != DW)
    └── pumice_top               ← instantiated by the SoC
        ├── pumice_csr           ← PeakRDL passthrough cpuif register block
        └── pumice_core          ← the controller proper
            ├── pumice_axi4_ifc          ← host AXI + wr/rd CAMs
            ├── pumice_mem_cmd_scheduler ← "what command to issue this cycle"
            └── pumice_dfi_layer         ← single CDC + DFI v2.1 datapath

The single controller-to-PHY clock crossing lives inside pumice_dfi_layer (pumice_dfi_cdc, async FIFOs only). The host AXI interface, CAMs, and command scheduler all run on aclk; the DFI command path, write serializer, and read aligner run on dfi_clk.

## 9.3	Layer Groupings

| Layer / macro | FUBs |
| --- | --- |
| pumice_axi4_ifc | pumice_wr_intake, pumice_rd_intake, addr_mapper, pumice_wr_data_cam, pumice_rd_cmd_cam |
| pumice_mem_cmd_scheduler | pumice_cmd_arbiter, pumice_bank_timers (bank_timer), global_timers, refresh_ctrl, init_sequencer, mode_register |
| pumice_dfi_layer | pumice_dfi_cdc, pumice_dfi_cmd_path (dfi_cmd_formatter, dfi_signal_pack), pumice_dfi_wr_serializer, pumice_dfi_rd_aligner |
| pumice_core | (wraps the three layers above) |

Also present in the tree but not in the default top build: page_predictor and powerdown_ctrl (referenced / optional; verify against the filelists in rtl/filelists/).

Chapter 3 follows the layer ordering (one section per architectural layer, with module names hyperlinked to their MAS chapters).

## 9.4	Memtype-Conditional Logic

Memtype (DDR2 vs LPDDR2) is a runtime CSR selection (PHY_TIMING.memtype), not an elaboration parameter. The memtype-dependent logic lives in:

- dfi_cmd_formatter — a memtype branch: DDR2 drives ras/cas/we; LPDDR2 packs the 10-bit JESD209-2F CA-bus command (two edges) onto dfi_address.

- init_sequencer — runs the DDR2 or LPDDR2 JEDEC MR/init sequence.

- mode_register — CL / CWL / BL / AL decode differs by memtype.

Both memtypes pass the full simulation suite.

## 9.5	Single Clock-Domain Crossing

The design has exactly one clock-domain crossing: pumice_dfi_cdc inside pumice_dfi_layer, built from asynchronous gaxi FIFOs. One FIFO word is one DFI cycle, so the command / write-data / read-data datapaths are bubble-free. Everything up to the CDC is on aclk; everything past it is on dfi_clk.

## 9.6	What Changed vs the Earlier Architecture

The controller was rearchitected from an earlier FSM-based, elaboration-time decomposition into the current three-layer, CSR-driven design. Notably:

- Retired: txn_queue, bank_machine, xbank_timers, cmd_encoder, odt_ctrl, scheme selector, the *_macro-as-architecture names, and axi_intake/axi_frontend.

- Replaced by: the two CAMs (pumice_wr_data_cam + pumice_rd_cmd_cam), FSM-free bank_timer (stamped by pumice_bank_timers), global_timers, dfi_cmd_formatter (+ dfi_signal_pack), and the single ADDR_MAP.bank_lsb knob.

- Introduced: the single-CDC DFI layer, the PeakRDL pumice_csr block (configuration by name), and the optional pumice_top_geared host-width wrapper.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Top-Level Interfaces

This section is the integrator’s quick-reference for the controller’s top-level module port list. Detailed protocol semantics for each interface are in Chapter 4 (Interfaces); this is the wire-level view — names, directions, widths, parameter dependencies.

The top module is pumice_top and has four external interfaces:

- AXI4 slave (host request path), signals prefixed s_axi_*

- DFI v2.1 master (DRAM PHY path), signals suffixed _o / _i

- CSR register cpuif (configuration and observation) — a PeakRDL passthrough bus, not APB

- Clocks and resets (two domains) plus init_done_o

All four are exposed at the top level as flat per-signal ports. SystemVerilog interfaces are not used at the top boundary because the framework’s interface convention (see CocoTBFramework BFM components) drives flat-port pin-level handshakes. Internal sub-blocks use interface objects, but the top-of-design is flat for synthesis and BFM compatibility.

An optional outer wrapper, pumice_top_geared, adds a HOST_AXI_DATA_WIDTH parameter and inserts formally-verified AXI data-width converters ahead of pumice_top; when HOST_AXI_DATA_WIDTH == DW the converters are bypassed and the build is bit-identical. This section describes pumice_top itself.

## 10.1	Parameter-Dependent Widths

The core data width is fixed by the DFI geometry: DW = DRAM_BEAT_WIDTH * DFI_RATE. There is no separate AXI_DATA_WIDTH parameter on pumice_top — the host AXI data width equals DW. Throughout this section:

- AW = AXI_ADDR_WIDTH (default 32)

- DFI_RATE = DFI phase count / frequency ratio (default 2)

- DRAM_BEAT_WIDTH = per-beat DRAM data width (default 64)

- DW = DRAM_BEAT_WIDTH * DFI_RATE (host AXI + DFI word width; default 128)

- SW = DW/8 (byte-strobe width)

- IW = AXI_ID_WIDTH (default 8)

- NR = NUM_RANKS (default 1)

- NB = NUM_BANKS (default 8)

- RW = ROW_WIDTH (default 14)

- CW = COL_WIDTH (default 10)

- BKW = $clog2(NUM_BANKS)

- CSR_ADDR_W = CSR address width (default 12)

## 10.2	1. AXI4 Slave (Host-side)

All host AXI ports carry the s_axi_ prefix.

### 10.2.1	Write Address Channel (AW)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_awid | input | IW | Write transaction ID |
| s_axi_awaddr | input | AW | Write start address |
| s_axi_awlen | input | 8 | Burst length – 1 |
| s_axi_awsize | input | 3 | Beat size encoding |
| s_axi_awburst | input | 2 | Burst type (INCR mandatory; FIXED / WRAP optional) |
| s_axi_awlock | input | 1 | Lock (ignored — exclusives not supported) |
| s_axi_awcache | input | 4 | Cache hint (observed; no behavior) |
| s_axi_awprot | input | 3 | Protection bits (observed) |
| s_axi_awqos | input | 4 | QoS hint (boosts scheduler priority — see §3.2) |
| s_axi_awregion | input | 4 | Region (observed) |
| s_axi_awuser | input | 1 | User sideband (observed) |
| s_axi_awvalid | input | 1 | Address-valid |
| s_axi_awready | output | 1 | Address-ready |

### 10.2.2	Write Data Channel (W)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_wdata | input | DW | Write data |
| s_axi_wstrb | input | SW | Byte enables |
| s_axi_wlast | input | 1 | Last beat of burst |
| s_axi_wuser | input | 1 | User sideband |
| s_axi_wvalid | input | 1 | Data-valid |
| s_axi_wready | output | 1 | Data-ready |

### 10.2.3	Write Response Channel (B)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_bid | output | IW | Response ID |
| s_axi_bresp | output | 2 | Response code (OKAY / SLVERR) |
| s_axi_buser | output | 1 | User sideband |
| s_axi_bvalid | output | 1 | Response-valid |
| s_axi_bready | input | 1 | Response-ready |

### 10.2.4	Read Address Channel (AR)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_arid | input | IW | Read transaction ID |
| s_axi_araddr | input | AW | Read start address |
| s_axi_arlen | input | 8 | Burst length – 1 |
| s_axi_arsize | input | 3 | Beat size encoding |
| s_axi_arburst | input | 2 | Burst type |
| s_axi_arlock | input | 1 | Lock (ignored) |
| s_axi_arcache | input | 4 | Cache hint |
| s_axi_arprot | input | 3 | Protection |
| s_axi_arqos | input | 4 | QoS hint |
| s_axi_arregion | input | 4 | Region |
| s_axi_aruser | input | 1 | User sideband |
| s_axi_arvalid | input | 1 | Address-valid |
| s_axi_arready | output | 1 | Address-ready |

### 10.2.5	Read Data Channel (R)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_rid | output | IW | Read data ID |
| s_axi_rdata | output | DW | Read data |
| s_axi_rresp | output | 2 | Response code |
| s_axi_rlast | output | 1 | Last beat |
| s_axi_ruser | output | 1 | User sideband |
| s_axi_rvalid | output | 1 | Data-valid |
| s_axi_rready | input | 1 | Data-ready |

## 10.3	2. DFI v2.1 Master (PHY-side)

The DFI v2.1 sub-interfaces present on this controller are limited to command, write-data, read-data, and the init handshake. Training, frequency-change, low-power, update, and CRC sub-interfaces are not driven — see §2.1 Out of Scope.

The DFI ports are flat, per-phase-packed buses (the DFI_RATE phases are concatenated into one vector) rather than SystemVerilog arrays. All DFI ports carry the _o (output) or _i (input) suffix. Command formatting runs on dfi_clk, past the single CDC in pumice_dfi_layer.

### 10.3.1	DFI Command Bus

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_address_o | output | RW * DFI_RATE | Address operand (row / column); LPDDR2 CA-bus command is packed here |
| dfi_bank_o | output | BKW * DFI_RATE | Bank operand |
| dfi_cs_n_o | output | NR * DFI_RATE | Active-low chip-select, per rank per phase |
| dfi_ras_n_o | output | DFI_RATE | RAS strobe per phase (DDR2; idle for LPDDR2) |
| dfi_cas_n_o | output | DFI_RATE | CAS strobe per phase (DDR2; idle for LPDDR2) |
| dfi_we_n_o | output | DFI_RATE | WE strobe per phase (DDR2; idle for LPDDR2) |
| dfi_odt_o | output | NR * DFI_RATE | ODT, per rank per phase |

### 10.3.2	DFI Write-Data Bus

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_wrdata_o | output | DW | Write data (all phases packed; = one DFI word) |
| dfi_wrdata_en_o | output | DFI_RATE | Per-phase write-data enable |
| dfi_wrdata_mask_o | output | SW | Per-byte write mask (DFI polarity: 1 = do-not-write) |

### 10.3.3	DFI Read-Data Bus

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_rddata_en_o | output | DFI_RATE | Per-phase read-data enable |
| dfi_rddata_i | input | DW | Read data (all phases packed) |
| dfi_rddata_valid_i | input | DFI_RATE | Per-phase read-data valid |

### 10.3.4	DFI Init Handshake

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_init_start_o | output | 1 | Init handshake — start of DRAM init |
| dfi_init_complete_i | input | 1 | PHY signals init complete |

## 10.4	3. CSR Register Interface (Configuration / Observation)

Configuration is driven by name from a PeakRDL-generated register block (pumice_csr) via a passthrough cpuif request bus (not APB). Address width is CSR_ADDR_W (default 12). Detailed register map in §6.3.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_cpuif_req | input | 1 | Request valid |
| s_cpuif_req_is_wr | input | 1 | 1 = write, 0 = read |
| s_cpuif_addr | input | CSR_ADDR_W | Register address |
| s_cpuif_wr_data | input | 32 | Write data |
| s_cpuif_wr_biten | input | 32 | Per-bit write enable |
| s_cpuif_req_stall_wr | output | 1 | Write back-pressure |
| s_cpuif_req_stall_rd | output | 1 | Read back-pressure |
| s_cpuif_rd_ack | output | 1 | Read data valid |
| s_cpuif_rd_err | output | 1 | Read error |
| s_cpuif_rd_data | output | 32 | Read data |
| s_cpuif_wr_ack | output | 1 | Write accepted |
| s_cpuif_wr_err | output | 1 | Write error |

## 10.5	4. Clocks, Resets, and Status

The design has two clock domains — the controller/AXI domain (aclk) and the DFI/PHY domain (dfi_clk) — with the single crossing inside pumice_dfi_layer.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| aclk | input | 1 | Controller clock (host AXI, CAMs, scheduler) |
| aresetn | input | 1 | Controller reset, active low |
| dfi_clk | input | 1 | DFI/PHY clock |
| dfi_rstn | input | 1 | DFI reset, active low |
| init_done_o | output | 1 | Asserted when DRAM init completes |

## 10.6	Hierarchical Top-Level View

The top-level pinout in shorthand:

pumice_top
    ├── AXI4 slave        (host)         s_axi_awid, s_axi_awaddr, ..., s_axi_rvalid, s_axi_rready, ...
    ├── DFI v2.1 master   (PHY)          dfi_address_o, dfi_bank_o, dfi_cs_n_o, ..., dfi_rddata_i, ...
    ├── CSR cpuif         (config)       s_cpuif_req, s_cpuif_addr, ..., s_cpuif_rd_data, ...
    └── Clocks + status                  aclk, aresetn, dfi_clk, dfi_rstn, init_done_o

The canonical SystemVerilog port list is rtl/top/pumice_top.sv. This section is the integrator’s overview, not the build-time canonical list.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	AXI4 Frontend

The AXI frontend is pumice_axi4_ifc (rtl/macro/pumice_axi4_ifc.sv). It bolts the repository’s common AXI burst splitters onto two “dumb” 1:1 intakes and holds the two CAMs that decouple AXI-side handshake from the DRAM-side command scheduler:

host AXI4 -> [axi_master_wr_splitter] -> pumice_wr_intake -> pumice_wr_data_cam
          -> [axi_master_rd_splitter] -> pumice_rd_intake -> pumice_rd_cmd_cam

The intakes each contain a common axi4_slave (write or read side) plus an addr_mapper that translates the flat AXI address into (rank, bank, row, col). The write intake also carries an AW-meta FIFO and a write-data FIFO; the read intake carries a snarf (read-your-write) probe into the write CAM. This chapter covers the splitters, the intakes, and addr_mapper. The two CAMs are covered in the data-paths chapter (07_data_paths.md).

## 11.1	Burst Splitters

axi_master_wr_splitter and axi_master_rd_splitter are formally-verified common modules. Each host burst is split at DRAM-burst-byte boundaries so every burst that reaches an intake maps to exactly one DRAM burst. The alignment mask is DRAM_BURST_BYTES - 1, where DRAM_BURST_BYTES = BL * (DRAM_BEAT_WIDTH / 8). Splitting is transparent to the AXI master: a host burst that crosses a DRAM-burst boundary is decomposed into multiple aligned sub-bursts, each of which becomes one CAM entry / one DRAM command.

## 11.2	pumice_wr_intake / pumice_rd_intake

### 11.2.1	Purpose

Terminate the AXI channels (via the embedded axi4_slave), decode each transaction’s address, and push a {rank, bank, row, col, id} insert request downstream into the corresponding CAM. The intakes are deliberately “dumb”: there is no reordering or scheduling here — that lives in the scheduler layer reading the CAMs.

### 11.2.2	Write intake

- Accepts AW transactions and issues an aw_push insert into pumice_wr_data_cam.

- Streams W beats into the write-data FIFO tagged for the CAM SRAM fill mover.

- Returns the B response when the CAM signals commit-done for the transaction ID (wr_done_valid / wr_done_id), matching AXI4 posted-write semantics.

### 11.2.3	Read intake

- Accepts AR transactions and issues an ar_push insert into pumice_rd_cmd_cam.

- Before committing a miss to the read CAM, it probes the write CAM with a snarf query (snarf_probe_*). On a snarf hit the read is serviced directly from the write CAM’s SRAM (read-your-write forwarding) rather than being scheduled to DRAM.

- Drains the read CAM (in AR order) onto the AXI R channel with the correct ID and rlast.

### 11.2.4	Per-ID Ordering

AXI4 requires reads from the same ID to return in order and writes from the same ID to commit in order. Because each intake inserts in AR/AW arrival order and the read CAM drains in insert order, per-ID ordering is preserved. The CAMs allow row-hit scheduling to reorder DRAM commands across entries, but the AXI completion layer honors per-ID order.

### 11.2.5	Backpressure

- AW/AR .ready deassert when the corresponding CAM has no free entry (ins_ready low).

- W .ready deasserts when the write-data FIFO/SRAM fill path is full.

- R and B .valid follow standard AXI protocol; a stalled consumer stalls only the drain path and does not corrupt the controller core.

## 11.3	addr_mapper

### 11.3.1	Purpose

Translate a flat AXI address into DRAM coordinates (rank, bank, row, col). RTL: rtl/fub/addr_mapper.sv. It is combinational and single-stage, instantiated inside each intake.

### 11.3.2	Interfaces

Inputs:

- axi_addr_i — AXI_ADDR_WIDTH bits

- bank_lsb_i[4:0] — the ADDR_MAP.bank_lsb CSR field

- hash_en_i — the ADDR_MAP.hash_en CSR field

- hash_seed_i[7:0] — the ADDR_MAP.hash_seed CSR field

Outputs:

- rank_o — $clog2(NUM_RANKS) bits

- bank_o — $clog2(NUM_BANKS) bits

- row_o — ROW_WIDTH bits

- col_o — COL_WIDTH bits

### 11.3.3	Mapping Function (single knob: bank_lsb)

There is no scheme selector anymore. The mapping is driven by one runtime knob, ADDR_MAP.bank_lsb, which places the bank field within the byte-offset-stripped word address (word = axi_addr >> BYTE_OFFSET_WIDTH). The column auto-splits around the bank; row and rank stack above the column region and their positions are invariant:

word address, low -> high:
[ col_lo(bank_lsb) | bank(BW) | col_hi(CW - bank_lsb) | row(RW) | rank(KW) ]
col = { col_hi, col_lo }
row LSB is always at CW + BW (invariant — only the bank slides)

The RTL clamps bank_lsb to [0, COL_WIDTH] so the col_hi width stays non-negative and the row/rank slices land where the geometry expects.

### 11.3.4	Classic Schemes as bank_lsb Settings

The former named schemes are just settings of the one knob:

| Effect | Setting | Notes |
| --- | --- | --- |
| ROW_MAJOR | bank_lsb == COL_WIDTH | Bank above the whole column; default (0x0A) |
| max BANK_INTERLEAVE | bank_lsb == log2(cols/burst) | Minimal col_lo keeps a DRAM burst inside one bank |
| partial interleave | any value in between | Column splits around the bank |

Software keeps log2(cols/burst) <= bank_lsb <= COL_WIDTH so a DRAM burst’s column walk stays inside one bank.

### 11.3.5	Optional Bank XOR-Hash

When hash_en is set, the bank index is XOR-folded with row bits and the seed to defeat power-of-two-stride hot-banking (the former XOR_HASH scheme, now an orthogonal overlay on any bank_lsb placement):

bank[i] ^= row[i] ^ row[i + BW] ^ hash_seed[i]

The mid-slice index is clamped so it never runs past ROW_WIDTH.

### 11.3.6	Relationship to the DFI BFM

This module mirrors the AddressMapping class in the DFI BFM (CocoTBFramework/components/dfi/). The same bank_lsb / hash configuration drives both RTL elaboration-time verification and BFM checking, so the address decode is consistent between simulation and silicon.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Command Scheduler

The command-scheduling layer is pumice_mem_cmd_scheduler (rtl/macro/pumice_mem_cmd_scheduler.sv). It is a single controller-clock (aclk) layer that wires together the pick core and all the timing / bring-up support blocks, and emits one abstract DRAM command stream {op, rank, bank, row, col, ap} into a command FIFO for the DFI layer to pack onto phases.

The scheduler does not hold the transaction queue — pending requests live in the two CAMs inside pumice_axi4_ifc. The scheduler reads them through external lookup / oldest / commit / issue ports.

## 12.1	Composition

pumice_mem_cmd_scheduler instantiates:

- pumice_cmd_arbiter — the single pick core (open-page decision is inline).

- pumice_bank_timers (per-rank/bank bank_timer instances) — FSM-free per-bank JEDEC “safe” timers (see 03_bank_machines.md).

- global_timers — cross-bank / bus turnaround (tFAW, tRRD, tWTR, tRTW, tCCD).

- refresh_ctrl — tREFI tracking + postpone accumulator (see 04_refresh.md).

- init_sequencer — JEDEC MRS cold-boot init; gates traffic until done (see 05_init_power.md).

- mode_register — CL / CWL / BL decode shadow, updated by init MRS writes.

- an output command FIFO (gaxi_fifo_sync).

## 12.2	pumice_cmd_arbiter

### 12.2.1	Purpose

Pick exactly one abstract DRAM command per cycle and push it into the scheduler-to-DFI command interface. It is PHY/nphases-agnostic and single-issue. JEDEC timing readiness is supplied by the per-bank (pumice_bank_timers) and global (global_timers) inputs; the arbiter never re-derives timing itself.

### 12.2.2	Inputs

- Per-bank readiness from pumice_bank_timers: bank_act_ready, bank_rdwr_ready, bank_pre_ready, bank_row_active, bank_open_row.

- Global readiness from global_timers: tfaw_ok, trrd_ok (per-rank), twtr_ok, trtw_ok, tccd_ok.

- Per-bank scheduler lookups into both CAMs (query each bank’s open row), plus the CAMs’ oldest ports.

- Refresh request / drain from refresh_ctrl; init command passthrough from init_sequencer.

### 12.2.3	Outputs

- Command push {op, rank, bank, row, col, ap} with cmd_valid / cmd_ready.

- Event strobes to the timers (evt_act, evt_rd, evt_wr, evt_pre, evt_ap, evt_rank, evt_bank, evt_row) — fired only on an accepted issue.

- CAM side effects: write-CAM commit, read-CAM issue, refresh grant — all gated on accepted issue.

op is drawn from dram_op_e (OP_NOP, OP_ACT, OP_RD, OP_RDA, OP_WR, OP_WRA, OP_PRE, OP_PREA, OP_REF, OP_REFPB, OP_MRS, …).

### 12.2.4	Priority Function

Evaluated combinationally each cycle (descending priority):

- Init in progress (!init_done) — forward the init_sequencer command verbatim; block all normal traffic.

- Refresh (refresh_req or drain active) — precharge active banks one per cycle, then — only under w_ref_safe (all rows closed in the registered view, nothing row-affecting in flight or guarded, prior REF’s tRFC recovery elapsed) — issue REF and assert the refresh grant. Each fired REF loads the arbiter’s tRFC down-counter (TIMINGS_RFC_REFI.tRFC); while non-zero, ACTs and further REFs are blocked.

- Column row-hit — a RD/WR to an already-open row whose bank is column-ready (bank_rdwr_ready) and whose bus turnaround permits it (tccd_ok + twtr_ok for reads / trtw_ok for writes). Reads have priority over writes; within each, the oldest entry (max CAM relative age) wins the tie-break.

- Fallback — no ready row-hit: ACT the oldest pending op’s row on an idle bank (subject to tfaw_ok / trrd_ok and the ACT/PRE guard), or PRE a bank that is open on the wrong row.

The fallback target is read-priority: the read CAM’s oldest port is consulted first, then the write CAM’s.

### 12.2.5	ACT/PRE Re-Issue Guard

The bank timers register their readiness outputs (there is a two-cycle latency from an event to act_ready / pre_ready dropping). A stateless combinational arbiter would otherwise re-issue ACT/PRE to the same bank before the timers reflect it. The arbiter therefore keeps a two-cycle per-bank guard (r_guard0 / r_guard1) that blocks a bank for two cycles after an accepted ACT, PRE or column to it — columns are included because a just-fired column has not yet dropped the bank’s registered pre_ready (tRTP/tWR load), so an unguarded PRE (normal or refresh-drain) could precharge on stale readiness. Columns still self-limit against each other (both CAMs exclude a just-committed / just-issued slot from the next cycle’s lookup), so the guard never gates column-vs-column. The shared-DQ-bus burst-occupancy constraint (a BL burst owns the bus for BL/DFI_RATE DFI cycles) is enforced downstream in the DFI command path, not here; the CDC decouples aclk command issue from the dfi_clk DQ timing.

### 12.2.6	Page Policy (Auto-Precharge)

The column auto-precharge bit is set directly from page_policy_i:

- OPEN — ap = 0; rows stay open. Column ops stream to an open row at tCCD rate.

- CLOSE — ap = 1; every column op auto-precharges (issues RDA / WRA).

- HAPPY_HYBRID — in v1 this is treated as OPEN. The page_predictor hook is a documented TODO; rtl/fub/page_predictor.sv exists but is not wired into the default build.

The open-page “keep the row open” decision therefore lives inline in the arbiter and the per-bank bank_timer (which keeps the row open on RD/WR), not in a separate predictor or lookahead unit.

### 12.2.7	Issue Rate

One command issue per controller clock cycle. Multi-phase / multi-cycle placement (including LPDDR2’s 2-edge CA word) happens downstream in the DFI layer, so the arbiter always sees a single issue.

## 12.3	Mode Register Shadow

mode_register maintains the live CL / CWL / BL decode. Its shadow is written in lockstep by the init_sequencer MRS writes (mr_seq_we / mr_seq_index / mr_seq_data), so the controller’s timing decode tracks exactly what was programmed into the DRAM during init. It exposes cl_o / cwl_o / bl_o to the DFI layer.

## 12.4	v1 Scope Notes

The arbiter picks from a single rank (rank 0) in v1; write-drain watermarking, multi-rank pick, and power-down coordination are documented TODOs. The busy_o output aggregates “init not done”, “refresh pending”, “command in the FIFO”, and “any bank row active”.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Bank Timers and Cross-Bank Timers

Per-bank JEDEC timing is tracked by an FSM-free bank_timer per (rank, bank); cross-bank / bus-turnaround timing is tracked by global_timers. There is no multi-state bank machine in this design.

## 13.1	bank_timer

### 13.1.1	Purpose

Track one DRAM bank’s JEDEC “safe” timing. RTL: rtl/fub/bank_timer.sv. It is not a state machine — it is a set of preset/decrement countdown timers plus a trivial row-open register and a single auto-precharge bit. The arbiter drives one set_* strobe per issued command; each timer loads its config value on its trigger, free-runs decrementing (saturating at 0), and reports its constraint as “safe” when the count is 0.

Because the per-command safe_* outputs are a purely combinational AND of the relevant timers behind a single register stage (the counter), a just-issued command is reflected one cycle later with no multi-stage lag. This was the point of retiring the old 3-state FSM, whose double-registered state was the root cause of the refresh-vs-ACT and column-vs-PRE hazards.

### 13.1.2	Instantiation

pumice_bank_timers (rtl/fub/pumice_bank_timers.sv) stamps one bank_timer per (rank, bank) pair — for the default (NUM_RANKS=1, NUM_BANKS=8) that is 8 instances. It routes the arbiter’s evt_* event strobes to the addressed bank and fans the per-bank readiness back out to the arbiter.

### 13.1.3	Timers

Each timer loads on its event, counts down, and saturates at 0:

| Timer | JEDEC | Loaded on | Gates |
| --- | --- | --- | --- |
| r_rcd | tRCD | ACT | safe_rd/safe_wr |
| r_ras | tRAS | ACT | safe_pre |
| r_rc | tRC | ACT | safe_act |
| r_rp | tRP | explicit PRE or auto-PRE fire | safe_act |
| r_preblk | tRTP (RD) / tWR (WR) | RD / WR | safe_pre only |

The timer config inputs (t_rcd_i, t_rp_i, t_ras_i, t_rc_i, t_wr_i, t_rtp_i) are controller-cycle, command-relative values supplied by the scheduler layer from CSRs.

### 13.1.4	Row State (minimal — not an FSM)

Two elements track row occupancy:

- r_row_valid + r_open_row — set on ACT (captures row_i), cleared on an explicit PRE or when an auto-precharge completes.

- r_ap_pending — a single bit set from the set_ap_i qualifier on a RD/WR. When both r_preblk and r_ras reach 0, the auto-precharge “fires” (w_ap_fire): the row closes internally and tRP loads — with no scheduler PRE and no extra states.

### 13.1.5	Combinational Safe Outputs

The readiness outputs are combinational off the timers (single register stage behind):

- safe_act_o = !r_row_valid && (r_rp == 0) && (r_rc == 0) — bank precharged, tRP since PRE, tRC since last ACT.

- safe_rd_o = r_row_valid && (r_rcd == 0) && !r_ap_pending — row open, tRCD met, not auto-precharging.

- safe_wr_o = safe_rd_o — same condition.

- safe_pre_o = r_row_valid && (r_ras == 0) && (r_preblk == 0) && !r_ap_pending — row open, tRAS and tRTP/tWR met, not auto-precharging.

Open-page behavior falls out naturally: RD/WR only load r_preblk and (optionally) the auto-PRE bit, so column commands stream to an open row. The tCCD / tWTR / tRTW turnaround constraints are enforced globally in global_timers and ANDed by the arbiter, not here.

### 13.1.6	Observability State

For debug only, state_o is derived combinationally from the timers into a bank_state_e value (BANK_IDLE, BANK_ACTIVATING, BANK_ACTIVE, BANK_PRECHARGING). No downstream logic depends on it — it is purely an observation output alongside the obs_*_nz timer-nonzero flags.

### 13.1.7	Refresh Interaction

There is no per-bank refresh handshake. The arbiter serializes refresh: on a refresh request it precharges active banks one per cycle (using safe_pre / bank_row_active), then issues REF once no bank has an open row AND w_ref_safe holds (no in-flight/guarded row op; prior tRFC recovery elapsed — see the scheduler chapter). REF recovery (tRFC) is an arbiter-side down-counter; the bank_timer never sees REF events. The bank_timer participates only through its standard row-open and safe_pre outputs.

## 13.2	global_timers

### 13.2.1	Purpose

Enforce the cross-bank and shared-bus turnaround constraints that a single bank cannot see locally. RTL: rtl/fub/global_timers.sv. Implemented once (not per bank) because these constraints are inherently global.

### 13.2.2	Tracked Constraints

| Constraint | Description | Scope |
| --- | --- | --- |
| tFAW | At most 4 ACTs in any rolling tFAW window | per-rank |
| tRRD | Minimum gap between any two ACT commands | per-rank |
| tWTR | Write-to-read data-bus turnaround | global |
| tRTW | Read-to-write data-bus turnaround | global |
| tCCD | Column-to-column spacing | global |

### 13.2.3	Interface

The timers consume the arbiter’s evt_act (with evt_act_rank), evt_rd, and evt_wr strobes and produce the readiness window signals the arbiter ANDs into its pick:

- tfaw_window_ok_o / trrd_window_ok_o — per-rank vectors, gate ACT.

- twtr_global_ok_o / trtw_window_ok_o / tccd_window_ok_o — gate column commands.

tFAW / tRRD are kept per-rank because they are device-local activate-stagger limits; the data-bus turnaround windows (tWTR / tRTW / tCCD) are global because the DQ bus is shared across ranks.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Refresh Controller

refresh_ctrl (rtl/fub/refresh_ctrl.sv) owns tREFI timing and refresh request accounting. It does not issue commands itself — it raises a request to the command arbiter, which serializes the precharge-then-REF sequence and pulses a grant once REF reaches the DFI bus.

## 14.1	refresh_ctrl

### 14.1.1	Purpose

Track the interval between mandatory refreshes and tell the scheduler when refresh is owed. JEDEC permits up to 8 postponed refreshes; refresh_ctrl uses an 8-deep accumulator plus a drain-quota mechanism so a batch of owed refreshes can be granted back-to-back.

### 14.1.2	Interface

| Signal | Direction | Purpose |
| --- | --- | --- |
| t_refi_i[15:0] | input | Refresh interval in MC cycles (CSR-backed) |
| refresh_burst_i[3:0] | input | 1..8 REFs to drain per request cycle (CSR-backed) |
| refpb_mode_i | input | 0 = REFab, 1 = REFpb (LPDDR2) bank-rotor select |
| enable_i | input | Gates tREFI counting; driven by init_done |
| refresh_req_o | output | High while pending refreshes remain |
| refresh_grant_i | input | Pulsed by the arbiter when REF issues on the bus |
| refresh_drain_active_o | output | High during a back-to-back drain burst |
| refresh_kind_o | output | Registered REFab/REFpb selector |
| refresh_bank_o | output | Bank rotor value (valid in REFpb mode) |
| pending_refreshes_o | output | Current accumulator value |
| obs_* | output | Internal state harvested for CSR readout |

### 14.1.3	tREFI Counter and Pending Accumulator

r_refi_cnt counts down from t_refi_i. It only ticks while enable_i is high (i.e. after init completes); before that it is held reloaded at t_refi_i. On expiry it reloads and the pending accumulator r_pending increments by 1 (saturating at the JEDEC maximum of 8). Each accepted grant decrements r_pending. A simultaneous expiry and grant is a net-zero change. refresh_req_o stays high whenever r_pending > 0.

Saturating at 8 is JEDEC-conformant: the controller is expected to keep at most 8 postponed refreshes outstanding before it must catch up. If the system stays bandwidth-starved past 8 x tREFI, the counter saturates (a DRAM retention violation looming) rather than growing unbounded.

### 14.1.4	Drain Quota

r_burst_remaining implements back-to-back draining. When the previous burst is fully drained and pending work exists, it (re)loads min(refresh_burst_i, r_pending). Each grant decrements it. While r_burst_remaining > 0 and r_pending > 0, refresh_drain_active_o is asserted, which the arbiter reads as “keep granting REF back-to-back without yielding to reads/writes”. Setting refresh_burst_i = 1 disables batching (one REF per tREFI).

### 14.1.5	REFpb Bank Rotor

When refpb_mode_i is set (LPDDR2 per-bank refresh), r_bank_rotor advances 0..NUM_BANKS-1 on each grant and is exposed on refresh_bank_o. In REFab mode it stays at 0. The current default build wires refpb_mode_i = 0 (REFab) in the scheduler.

## 14.2	Arbiter-Side Refresh Sequence

refresh_ctrl only raises refresh_req / refresh_drain. The precharge-then-REF sequence is performed by pumice_cmd_arbiter at refresh priority (second only to init):

- While any bank on the target rank has an open row, precharge the active banks one per cycle (lowest ready bank first, honoring the ACT/PRE guard).

- Once no bank has an open row and w_ref_safe holds — nothing row-affecting in flight or inside its 2-cycle guard window (the registered bank view alone is 2-3 cycles stale, which once let a REFab collide with a just-issued ACT), and the previous REF’s tRFC recovery elapsed — issue OP_REF and assert refresh_grant_o to refresh_ctrl, decrementing the accumulator / drain quota. On each fired REF the arbiter loads a tRFC down-counter from TIMINGS_RFC_REFI.tRFC (t_rfc_i); while non-zero, ACT picks and further REFs are blocked (mission-mode refresh recovery — init-time refreshes wait INIT_TIMING1.t_rfc_wait separately).

- Repeat back-to-back while refresh_drain_active is high (each REF spaced by tRFC), until the accumulator is drained.

This “wait until all banks are precharged” behavior is implicit in the arbiter’s readiness gating rather than an explicit bank-grant handshake.

## 14.3	Notes and Scope

- Self-refresh / power-down are handled by the separate powerdown_ctrl block (see 05_init_power.md), not by refresh_ctrl.

- PASR, DARP idle-bank-first selection, temperature-compensated tREFI, and periodic ZQCS are not part of the current refresh_ctrl RTL. REFpb selection is a simple bank rotor; the richer selection policies remain future work.

- REFab vs REFpb for LPDDR2 is selectable via refpb_mode_i and the bank rotor, but the default build uses REFab.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Init Sequencer and Power-Down

Cold-boot bring-up is handled by init_sequencer; low-power entry/exit by powerdown_ctrl.

## 15.1	init_sequencer

### 15.1.1	Purpose

Sequence post-reset DRAM bring-up: the DFI init handshake, JEDEC mode-register loads, precharge, and refresh. RTL: rtl/fub/init_sequencer.sv. It is a compact hard-coded FSM (not a microprogram ROM). Its commands are issued to the DRAM, not merely shadowed — this was the fix for the on-board bring-up failure where an earlier shadow-only walk left the read DLL unlocked.

### 15.1.2	Command Path

While init_busy_o is high, the scheduler stays idle and forwards the sequencer’s single-cycle command pulses (init_cmd_valid_o / init_cmd_op_o / init_cmd_bank_o / init_cmd_row_o) straight to dfi_cmd_formatter. Each command state is occupied for exactly one cycle, then the FSM parks in S_WAIT for the JEDEC inter-command delay before advancing. Because dfi_cmd_formatter is always cmd_ready, a single-cycle pulse issues exactly one command with no grant handshake.

The mode_register shadow is updated in lockstep (mr_seq_we_o / mr_seq_index_o / mr_seq_data_o) so the live CL/CWL/BL decode tracks what was programmed.

### 15.1.3	CSR-Backed Waits

The inter-command delays are CSR-backed (INIT_TIMING registers), zero-extended into a 16-bit countdown, rather than hardcoded:

| Input | JEDEC | Default |
| --- | --- | --- |
| t_init_wait_i | CKE / tINIT | 512 |
| t_dll_wait_i | DLL lock tDLLK | 256 |
| t_mrd_wait_i | tMRD | 8 |
| t_rp_wait_i | tRP | 8 |
| t_rfc_wait_i | tRFC | 8 |

### 15.1.4	DDR2 Sequence

Mirrors LiteDRAM’s DDR2 PHY init (the reference proven on the Nexys A7 board):

- Assert dfi_init_start_o; wait dfi_init_complete_i (the PHY runs its own DLL-lock / IO training), then wait tINIT.

- Precharge All.

- EMRS in JEDEC order EMRS(2) then EMRS(3) then EMRS(1) — all defaults 0.

- MRS(0) + DLL reset (MR0.VAL | 0x100; reset default 0x533 = BL8/CL3/tWR3/DLL_RESET); wait tDLLK.

- Precharge All.

- Auto Refresh x2 (each followed by tRFC).

- MRS(0) without DLL reset (MR0.VAL, reset default 0x433) — clears the reset bit.

- EMRS(1) + OCD default (0x380) then EMRS(1) + OCD exit (0x000).

- init_done_o = 1.

The DDR2 MR values are CSR-backed (MR0..MR3.VAL; MR0 reset 0x433, MR1 0x0000, MR2/MR3 0x0000) with the OCD default (0x380) ORed in by the sequencer; software may retune them before an init_restart.

### 15.1.5	LPDDR2 Sequence (fully functional)

Per JESD209-2F power-up and mode-register configuration:

- Assert dfi_init_start_o; wait dfi_init_complete_i; tINIT settle.

- MRW(MR63) = Reset (OP don’t-care).

- MRW(MR10) = 0xFF ZQ Init Calibration.

- MRW(MR1) = 0x23 (BL8 / nWR3), MRW(MR2) = 0x01 (RL3 / WL1), MRW(MR3) = 0x02 (DS 40 ohm).

- init_done_o = 1.

Only MR1/MR2/MR3 update the CL/CWL/BL decode shadow; MR63/MR10 are issued to the DRAM but not shadowed (they exceed the 5-bit shadow index). The MR index (MA, up to MR63) and data (OP) are carried packed as {MA[5:0], OP[7:0]} in the ROW request field, so dfi_cmd_formatter can build the full LPDDR2 CA MRW word — a 3-bit bank port alone could not reach MR10/MR63.

### 15.1.6	Memtype Selection

memtype_i (from the PHY_TIMING CSR: 0 = DDR2, 1 = LPDDR2) chooses the sequence at runtime. After the shared DFI-init step, DDR2 branches to S_PREA1 and LPDDR2 to S_L_RESET.

### 15.1.7	Status Outputs

| Signal | Purpose |
| --- | --- |
| dfi_init_start_o | High after leaving reset |
| init_busy_o | High until the FSM reaches S_DONE |
| init_done_o | Asserted at S_DONE |
| zqcl_req_o | Tied off — DDR2 has no ZQCL |

## 15.2	powerdown_ctrl

### 15.2.1	Purpose

Idle-detect into a low-power state. RTL: rtl/fub/powerdown_ctrl.sv. It is available but not in the default top build; it is documented here for completeness.

### 15.2.2	Modes

- PDE (Precharge Power Down) — CKE low, banks may stay active. Wakes on new activity within ~tXPDLL cycles.

- SR (Self Refresh) — CKE low plus an SRE command issued by the scheduler; the DRAM self-refreshes internally and the controller stops issuing REFs. Exit requires SRX plus tXSDLL (DDR2) / tXSR (LPDDR2) before new commands. SR entry requires all banks precharged first (the scheduler enforces the precondition).

### 15.2.3	Selection

- enable_sref_i high: on the idle threshold, request SR (pdn_kind_o = 1).

- enable_sref_i low, enable_pde_i high: request PDE (pdn_kind_o = 0).

- neither: never request.

### 15.2.4	Scope / TODO

Deep Power Down (LPDDR2), per-rank power-down, and the dfi_dram_clk_disable cooperation with dfi_signal_pack are documented TODOs. The block currently powers down all ranks together and trusts the scheduler not to grant before init completes.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Command Formatter and Signal Pack

Two modules translate the scheduler’s abstract command into DFI wire signals: dfi_cmd_formatter (memtype-specific encoding and phase placement) and dfi_signal_pack (the final registered pipeline stage). dfi_cmd_formatter is instantiated inside pumice_dfi_cmd_path in the DFI layer.

## 16.1	dfi_cmd_formatter

### 16.1.1	Purpose

Translate the scheduler’s chosen dram_op_e plus (rank, bank, row, col) into the multi-phase DFI v2.1 control bus. RTL: rtl/fub/dfi_cmd_formatter.sv. It is the only place where DDR2 and LPDDR2 differ at the wire layer, and its outputs are strict-flopped.

### 16.1.2	Input

The command arrives on cmd_valid_i / cmd_ready_o (always ready) with cmd_op_i (dram_op_e), cmd_rank_i, cmd_bank_i, cmd_row_i, cmd_col_i, cmd_len_i, and the runtime phase-placement knobs rd_phase_i / wr_phase_i.

### 16.1.3	Multi-Phase Output

For DFI_RATE = N, every control signal is packed as N phases side by side on the bus (dfi_address_o, dfi_bank_o, dfi_cas_n_o, dfi_ras_n_o, dfi_we_n_o, dfi_cs_n_o, dfi_odt_o). The decoded command is placed on its target phase and the other phases emit selected/deselected NOP. The target phase is:

- rd_phase_i for OP_RD / OP_RDA,

- wr_phase_i for OP_WR / OP_WRA,

- phase 0 for everything else (ACT/PRE/REF/MRS have no data-phase contract).

The R/W phase knobs match the PHY’s rdphase/wrphase contract (e.g. a7ddrphy DDR2 CL3 nphases=2). Defaults of 0/0 preserve the legacy “everything on phase 0” behavior. cs_n is per-rank: cs_n[r]=0 selects rank r; all-ones is a deselected NOP.

### 16.1.4	DDR2 Encoding

Combinational truth table per JESD79-2 Table 49:

| op | cs_n | ras_n | cas_n | we_n | bank | addr |
| --- | --- | --- | --- | --- | --- | --- |
| NOP | sel | 1 | 1 | 1 | - | - |
| ACT | sel | 0 | 1 | 1 | BA | row |
| RD | sel | 1 | 0 | 1 | BA | col (A10=0) |
| RDA | sel | 1 | 0 | 1 | BA | col + (1<<10) |
| WR | sel | 1 | 0 | 0 | BA | col |
| WRA | sel | 1 | 0 | 0 | BA | col + (1<<10) |
| PRE | sel | 0 | 1 | 0 | BA | A10=0 |
| PREA | sel | 0 | 1 | 0 | - | (1<<10) |
| REF | sel | 0 | 0 | 1 | - | - |
| MRS | sel | 0 | 0 | 0 | MR | MR data |

The auto-precharge / all-bank bit is A10. MRS data rides cmd_row_i (ROW_WIDTH), not cmd_col_i — MR0 = 0x533 needs bit 10, which a 10-bit column field would truncate.

### 16.1.5	LPDDR2 Encoding (bit-exact CA bus)

For LPDDR2 the command rides the multiplexed 10-bit CA bus over 2 edges, packed as a flat 20-bit word carried on dfi_address (low bits); ras_n/cas_n/we_n stay idle and cs_n asserts for the target rank. The two CA edges are already inside the word, so there is no per-DFI-phase command placement.

The CA word is built bit-exact to JESD209-2F Table 60, matching the DV BFM’s lpddr_ca encoder. Layout: w_lpddr2_ca[i] = CA{i} rising edge (i = 0..9), w_lpddr2_ca[10+i] = CA{i} falling edge. Encoded commands include ACT, RD/RDA, WR/WRA, PRE, PREA, REF (all-bank), REFPB (per-bank), and MRW; NOP/Deselect drives CA0r..CA3r high. Column bit C0 is implied 0 and never transmitted; the auto-precharge flag lands on CA0f. The transcription reference is rtl/LPDDR2_CA_ENCODING.md.

For MRW, MA0..MA5 map to CA4r..CA9r, MA6/MA7 to CA0f/CA1f, and OP0..OP7 to CA2f..CA9f. The init sequencer supplies the full MR index by packing {MA[5:0], OP[7:0]} into the ROW field, so MR10/MR63 are reachable.

Note: the module’s top-of-file header comment still carries a stale “LPDDR2 (TODO)” line from an earlier revision. The body implements the full bit-exact CA encoding above; LPDDR2 reads and writes are functional and pass the sim suite.

### 16.1.6	Idle Defaults

When not issuing, every phase is driven to its deselected idle: cs_n=1, ras_n=1, cas_n=1, we_n=1, bank=0, address=0, odt=0 (DFI v2.1 Table 2 defaults). ODT is driven from this module — there is no standalone odt_ctrl block.

## 16.2	dfi_signal_pack

### 16.2.1	Purpose

Final pipeline-register stage on the DFI v2.1 bus. RTL: rtl/fub/dfi_signal_pack.sv. It latches every command / write-data / read-data-enable input and drives it out the next MC cycle, owning dfi_dram_clk_disable and reset-safe output values.

### 16.2.2	Behavior

- v1 is a pure one-cycle registered pipeline. Bus widths are DFI_*_WIDTH * DFI_RATE, so the multi-phase content from dfi_cmd_formatter passes through unchanged. This is where the internal DW = DRAM_BEAT_WIDTH * DFI_RATE word is presented across the DFI_RATE phases.

- dram_clk_disable is held at 0 in v1; per-phase staggering and power-down assertion are documented TODOs.

### 16.2.3	Multi-Cycle Commands

LPDDR2’s 2-edge CA command is not split here. The 20-bit CA word is already packed onto dfi_address by dfi_cmd_formatter; the PHY splits it into two DRAM cycles per DFI v2.1.

### 16.2.4	Verification

The RTL output round-trips against the DFI BFM: the DDR2 truth table against the JESD79-2 reference, and the LPDDR2 CA bus against the shared lpddr_ca encoder/decoder (the same layout the RTL encodes).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Write and Read Data Paths

The data path spans two clock domains. On the controller (aclk) side the two CAMs in pumice_axi4_ifc buffer burst data; on the DFI (dfi_clk) side the write serializer and read aligner drive/capture the DFI data bus. All four are de-FSM’d streaming readers — they are FIFO-fed / beat-counter datapaths with no active/slot state latch.

## 17.1	Controller-Side CAMs

### 17.1.1	pumice_wr_data_cam

RTL: rtl/fub/pumice_wr_data_cam.sv. A write-command CAM plus a write-data SRAM. Entries are keyed on {bank, row, col} with a free-running age; the burst payload lives in an SRAM slot. Three data movers stream over the SRAM:

- fill — captures W beats into the slot on insert.

- commit-drain — streams the slot to the DFI write path when the scheduler commits the entry.

- snarf — forwards a slot to a matching read (read-your-write), limited to unscheduled entries with a matching id and burst length.

Age is compared wrap-safe as a relative age (age_ctr - entry_age). The snarf lookup returns the youngest match (latest data), the scheduler lookup returns the oldest match (in-order commit per row), and the oldest port returns the oldest valid entry for the scheduler fallback. A fill-complete flag gates schedulability and snarf so a partially-filled burst is never committed or forwarded.

There is no standalone wr2rd_forward block — write-to-read forwarding is exactly the snarf mover in this CAM.

### 17.1.2	pumice_rd_cmd_cam

RTL: rtl/fub/pumice_rd_cmd_cam.sv. The read miss path — an outstanding-read tracker acting as a reorder buffer. Entries are keyed {bank, row, col} with a free-running age and N scheduler lookups so reads row-hit-schedule like writes. Data flows the opposite direction from the write CAM: it comes in from the DFI read return and drains out to pumice_rd_intake. DRAM read data returns in issue order and is buffered per entry; it drains in AR (insert) order, which is what enforces per-ID read ordering.

The issue-side oldest/lookups pick the oldest not-yet-issued entry; the drain side releases the oldest valid entry gated on data-ready. There is no snarf port (that is a write-CAM concept).

## 17.2	DFI-Side Data Movers

Both movers live in the DFI layer on dfi_clk. Their internal unit is the DFI word (DRAM_BEAT_WIDTH * DFI_RATE wide), which already carries all DFI_RATE phases; dfi_signal_pack splits it to the pins. One FIFO pop is one DFI cycle, so both are bubble-free.

### 17.2.1	pumice_dfi_wr_serializer

RTL: rtl/fub/pumice_dfi_wr_serializer.sv. On a wr_fire from the command path it waits t_phy_wrlat_i DFI cycles, then streams one DFI-word per cycle from the write-data FIFO onto dfi_wrdata (with dfi_wrdata_en and dfi_wrdata_mask) until the burst’s last word. Because the write CAM pre-stages the burst into the FIFO when the command is scheduled, the burst is already waiting at wr_fire and the drive never stalls. The DFI byte mask is the inverted AXI strobe (mask = ~strb; AXI wstrb=1 = write byte, DFI mask=1 = mask).

### 17.2.2	pumice_dfi_rd_aligner

RTL: rtl/fub/pumice_dfi_rd_aligner.sv. The mirror of the write serializer. On a rd_fire it drives dfi_rddata_en for the read window starting t_rddata_en_i DFI cycles later, captures dfi_rddata whenever the PHY asserts dfi_rddata_valid, and pushes one DFI-word per valid cycle into the read FIFO as {last, resp, data}. BL_WORDS = BL/DFI_RATE words per burst; last marks the final word so the read intake can split words back into AXI R beats. v1 supports one outstanding read window (single-issue, tRTW/tCCD spaced).

## 17.3	Response Generation

- B response — the write intake returns B when the write CAM signals commit-done for the transaction id, matching AXI4 posted-write semantics and decoupling B latency from DRAM-side delays.

- R response — the read intake drains the read CAM onto the R channel in AR order with the correct id and rlast. In v1 all reads return OKAY (the DFI resp field is carried through but DDR2/LPDDR2 have no CA parity to surface).

## 17.4	Runtime PHY Timing

The write and read timing offsets are runtime CSR knobs rather than hard-coded PHY latencies, so one build ports across PHYs: t_phy_wrlat (WR command to dfi_wrdata_en) and t_rddata_en (RD command to dfi_rddata_en), plus the DFI_PHASE CSR (rd_phase / wr_phase) covered in the DFI/CSR chapter.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	DFI Layer and CSR

The bottom of the controller is the DFI layer (pumice_dfi_layer), which owns the single controller-to-PHY clock crossing, and the generated register block (pumice_csr), which carries all runtime configuration.

## 18.1	pumice_dfi_layer

### 18.1.1	Purpose

Present the controller-clock command / write-data / read-data streams on one side and the DFI v2.1 pin bus on the other, with exactly one clock-domain crossing between them. RTL: rtl/macro/pumice_dfi_layer.sv. The controller runs on ctl_clk; the PHY-facing datapath runs on dfi_clk.

### 18.1.2	The Single CDC

pumice_dfi_cdc (rtl/fub/pumice_dfi_cdc.sv) is the only clock-domain crossing in the whole controller. It is built from asynchronous gaxi FIFOs — three streams cross it:

- cmd (ctl_clk -> dfi_clk): the abstract command stream from the scheduler.

- wrdata (ctl_clk -> dfi_clk): DFI-word-granular {last, strb, data} drained from the write CAM.

- rddata (dfi_clk -> ctl_clk): {last, resp, data} captured from the PHY.

- plus the init handshake (init_start across, init_complete back).

The internal datapath unit is the DFI word (dfi_wrdata width, all DFI_RATE phases), so one FIFO word equals one DFI cycle and the crossing is bubble-free. Keeping the CDC in one place is what makes the rest of the controller a single-clock design.

### 18.1.3	DFI-Domain Datapath

On the dfi_clk side the layer instantiates three mechanical stages:

- pumice_dfi_cmd_path — pops the command FIFO and drives the multi-phase DFI command bus via dfi_cmd_formatter (see 06_encoder_output.md). It emits wr_fire / rd_fire strobes to time the data movers and enforces the shared-DQ-bus burst occupancy (COL_BURST_CYC = BL/DFI_RATE).

- pumice_dfi_wr_serializer — drives dfi_wrdata at t_phy_wrlat (see 07_data_paths.md).

- pumice_dfi_rd_aligner — captures dfi_rddata at t_rddata_en (see 07_data_paths.md).

### 18.1.4	DFI Sub-Interfaces

The layer drives the three core DFI v2.1 sub-interfaces needed for DDR2 / LPDDR2 operation: the command bus, the write-data interface (wrdata / wrdata_en / wrdata_mask), and the read-data interface (rddata_en / rddata / rddata_valid), plus the DFI init handshake. Update / training / low-power / status sub-interfaces beyond init_complete are not driven; the controller manages power via CKE directly.

### 18.1.5	Runtime PHY Knobs

Rather than hard-coded PHY latencies, the layer takes runtime inputs so one build ports across PHYs: rd_phase_i / wr_phase_i (per-command DFI sub-phase placement), t_phy_wrlat_i, and t_rddata_en_i, plus memtype_i. These are driven from the CSR (see below).

## 18.2	pumice_csr

### 18.2.1	Purpose

Hold all runtime configuration and observation registers. The register block is generated by PeakRDL from rtl/macro/pumice_csr.rdl (via bin/peakrdl_generate.py), with a passthrough cpuif — it is not a hand-written APB3 slave.

### 18.2.2	Interface

The generated block exposes a simple request/response passthrough cpuif rather than a full bus protocol:

| Signal | Direction | Purpose |
| --- | --- | --- |
| s_cpuif_req | input | Request valid |
| s_cpuif_req_is_wr | input | Write (1) vs read (0) |
| s_cpuif_addr | input | Register address |
| s_cpuif_wr_data[31:0] | input | Write data |
| s_cpuif_wr_biten[31:0] | input | Per-bit write enable |
| s_cpuif_rd_data[31:0] | output | Read data |
| s_cpuif_*_ack / *_err / *_stall_* | output | Response / flow-control |

Registers are regwidth = 32 / accesswidth = 32. pumice_top connects the cpuif and fans the decoded hwif_out struct fields to the core; the core’s status feeds back through hwif_in. Any external bus (APB, AXI-Lite) is adapted to this cpuif outside the block.

### 18.2.3	Access by Name

DV never uses hard-coded offsets. Configuration is programmed by field name through the generated dv/tbclasses/pumice_regmap.py, which stays in sync with the RDL. This is split-proof — if a register moves, the by-name access still resolves.

### 18.2.4	Address Mapping Register (ADDR_MAP @ 0x04C)

This register replaced the retired ADDR_MAP_TUNING (the old scheme_or / synth_mask_obs scheme selector is gone). It drives addr_mapper (see 01_axi_frontend.md):

| Field | Bits | Default | Purpose |
| --- | --- | --- | --- |
| bank_lsb | [4:0] | 0x0A | Bank field LSB in the word address (= COL_WIDTH -> ROW_MAJOR) |
| hash_en | [8] | 0 | Enable bank XOR-hash bank ^= fold(row) ^ hash_seed |
| hash_seed | [23:16] | 0x00 | XOR-hash seed |

There is no scheme mux; bank_lsb alone selects row-major vs interleaved placement, and hash_en is an orthogonal overlay.

### 18.2.5	Key Configuration Registers

The RDL is authoritative; notable runtime-config registers include:

| Register | Offset | Notable fields |
| --- | --- | --- |
| DFI_PHASE | 0x060 | rd_phase[2:0], wr_phase[6:4] — DFI sub-phase placement (default 0/0) |
| PHY_TIMING | 0x064 | t_phy_wrlat[7:0] (reset 0; Nexys A7 tuple programs 1), t_rddata_en[15:8] (reset 6), memtype[16] (0=DDR2, 1=LPDDR2), refresh_burst[23:20] (1..8) |
| ADDR_MAP | 0x04C | bank_lsb, hash_en, hash_seed (above) |
| INIT_TUNING | 0x050 | zq_retries[3:0], init_timeout_ms[15:8] |
| INIT_TIMING0 | 0x054 area | t_init_wait / t_dll_wait (JEDEC init waits, formerly hardcoded) |

memtype selects DDR2 vs LPDDR2 at runtime and is consumed by init_sequencer, dfi_cmd_formatter, and mode_register. Confirm every register and field against the RDL and the generated regmap.

### 18.2.6	Narrow-Device (x16) Width Granularities

The controller keeps two width concepts distinct: the DRAM beat (one DFI phase’s data, DRAM_BEAT_WIDTH) and the physical device word (the DQ width, DRAM_DEVICE_WIDTH, e.g. 16 for a x16 device). When a beat is wider than the device, burst length (beats per command) and the column-address stride are device-word quantities. The RTL scales the MR0 burst length by DRAM_BEAT_WIDTH / DRAM_DEVICE_WIDTH and sizes the column granularity to the device word; missing either produced the on-silicon DDR2 read failure on the Nexys A7 x16 bring-up (over-read plus column-overlap scramble). PHY read-path integration (data-vs-rddata_valid skew and per-command read/write phase) is handled by the runtime DFI_PHASE / PHY_TIMING knobs above rather than hard-coded PHY latencies, so one controller build ports across PHYs.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	AXI4 Slave Interface

The controller exposes a standard AXI4 slave (pumice_top / pumice_core) for SoC connectivity. All slave ports use the s_axi_* prefix with no i_/o_ decoration (e.g. s_axi_awid, s_axi_awaddr, s_axi_wdata, s_axi_rdata).

The AXI side runs on aclk / aresetn.

## 19.1	Data Width Is Fixed at DW

The core’s AXI data width is not a free parameter. It is fixed at

DW = DRAM_BEAT_WIDTH * DFI_RATE

i.e. one AXI beat carries exactly one DFI word (= DFI_RATE DRAM beats). There is no independent AXI_DATA_WIDTH parameter on pumice_top; s_axi_wdata / s_axi_rdata / s_axi_wstrb are always DW / DW / DW/8 bits.

To attach a host AXI bus of a different width, use the optional wrapper pumice_top_geared (see Host-Width Gearing below).

### 19.1.1	Hard width rule (compile-enforced)

pumice_top_geared’s HOST_AXI_DATA_WIDTH and the DFI word DW MUST relate by an exact power-of-two ratio — AXI:DFI = G:1 or 1:G, G ∈ {1,2,4,8,…}. An initial assert … $fatal in pumice_top_geared fails elaboration / Vivado synthesis on any other pairing, so a bad width choice is a compile error, never a silent broken build. Rationale (identical to LiteDRAM, whose AXI frontend is 1:1 with its native port and routes all width change through a power-of-two stride converter): the DFI word is the atomic memory-side transfer, so an AXI beat must be a whole power-of-two number of DFI words. DW and HOST_AXI_DATA_WIDTH are the only compile-time width parameters — gear (active phases) and burst length are runtime CSRs (build-for-max).

## 19.2	Channels

Standard AXI4 — all five channels (AW, W, B, AR, R) are mandatory. No exclusive access support; s_axi_awlock / s_axi_arlock are present but ignored.

### 19.2.1	AW (Write Address) — SoC master → controller slave

| Signal | Width | Notes |
| --- | --- | --- |
| s_axi_awid | AXI_ID_WIDTH | Transaction ID |
| s_axi_awaddr | AXI_ADDR_WIDTH | Flat byte address |
| s_axi_awlen | 8 | Burst length minus 1 |
| s_axi_awsize | 3 | Beat size; up to bus width |
| s_axi_awburst | 2 | INCR mandatory; FIXED/WRAP optional |
| s_axi_awlock | 1 | Present; ignored (no exclusive access) |
| s_axi_awcache | 4 | Present; ignored |
| s_axi_awprot | 3 | Present; ignored |
| s_axi_awqos | 4 | Present; ignored |
| s_axi_awregion | 4 | Present; ignored |
| s_axi_awuser | 1 | Present; ignored |
| s_axi_awvalid | 1 | Master asserts |
| s_axi_awready | 1 | Controller asserts when the WR intake / CAM can accept |

### 19.2.2	W (Write Data)

| Signal | Width | Notes |
| --- | --- | --- |
| s_axi_wdata | DW | Beat data (DW = DRAM_BEAT_WIDTH*DFI_RATE) |
| s_axi_wstrb | DW/8 | Byte enables; inverted to dfi_wrdata_mask |
| s_axi_wlast | 1 | Final beat of burst |
| s_axi_wuser | 1 | Present; ignored |
| s_axi_wvalid | 1 | Master asserts |
| s_axi_wready | 1 | Controller asserts while the WR data path can accept |

Note: AXI4 W channel has no wid; beats are presumed in the order their AW transactions were issued. The WR intake associates write data with its AW by arrival order.

### 19.2.3	B (Write Response)

| Signal | Width | Notes |
| --- | --- | --- |
| s_axi_bid | AXI_ID_WIDTH | Matches awid |
| s_axi_bresp | 2 | OKAY or SLVERR |
| s_axi_buser | 1 | Present; ignored |
| s_axi_bvalid | 1 | Controller asserts on response ready |
| s_axi_bready | 1 | Master asserts |

### 19.2.4	AR (Read Address)

| Signal | Width | Notes |
| --- | --- | --- |
| s_axi_arid | AXI_ID_WIDTH | Transaction ID |
| s_axi_araddr | AXI_ADDR_WIDTH | Flat byte address |
| s_axi_arlen | 8 | Burst length minus 1 |
| s_axi_arsize | 3 | Beat size |
| s_axi_arburst | 2 | INCR mandatory; others optional |
| s_axi_arlock | 1 | Present; ignored |
| s_axi_arcache | 4 | Present; ignored |
| s_axi_arprot | 3 | Present; ignored |
| s_axi_arqos | 4 | Present; ignored |
| s_axi_arregion | 4 | Present; ignored |
| s_axi_aruser | 1 | Present; ignored |
| s_axi_arvalid | 1 | Master asserts |
| s_axi_arready | 1 | Controller asserts when the RD intake / CAM can accept |

### 19.2.5	R (Read Data)

| Signal | Width | Notes |
| --- | --- | --- |
| s_axi_rdata | DW | Beat data |
| s_axi_rid | AXI_ID_WIDTH | Matches arid |
| s_axi_rresp | 2 | OKAY (v1 always) |
| s_axi_rlast | 1 | Final beat of burst |
| s_axi_ruser | 1 | Present; ignored |
| s_axi_rvalid | 1 | Controller asserts when beat ready |
| s_axi_rready | 1 | Master asserts |

## 19.3	Ordering Rules

- Within an ID: AXI4 ordering preserved. Reads from arid = X return in AR order; writes to awid = X commit in AW order.

- Across IDs: Out-of-order completion permitted by default (AXI_OOO_ACROSS_IDS = true). Disabling forces in-order across IDs at the cost of bandwidth.

## 19.4	Burst Type Support

| Burst | Mandatory | Notes |
| --- | --- | --- |
| FIXED | optional | Enable with SUPPORT_FIXED_BURST; rarely useful for DRAM |
| INCR | mandatory | Standard incrementing burst |
| WRAP | optional | Enable with SUPPORT_WRAP_BURST |

## 19.5	Burst Length and DRAM-Burst Geometry

AXI4 allows up to 256 beats. The frontend pumice_axi4_ifc splits every host burst at DRAM-burst-byte boundaries (axi_master_wr_splitter / axi_master_rd_splitter) so each split request maps to exactly one DRAM burst.

At the DW side the required geometry is:

(awlen + 1) * DFI_RATE == BL

i.e. one AXI burst at DW equals one DRAM burst of BL beats. (The older “AXI_DATA_WIDTH == DRAM_BEAT_WIDTH” coupling no longer applies — the AXI beat is a full DFI word of DFI_RATE DRAM beats.)

## 19.6	Narrow Transfers

AXI4 supports awsize < bus width. The narrow case is handled via wstrb — the unused byte lanes are masked. dfi_wrdata_mask = ~wstrb, so the DRAM commits only the unmasked bytes.

## 19.7	Write Response Timing

The B response is commit-driven: it is returned once the write is accepted and committed by the controller (pumice_wr_intake raises B from the downstream commit-done handshake). A SLVERR B is self-generated by the intake for a malformed request; otherwise OKAY.

## 19.8	Frontend and Backpressure

The AXI frontend is pumice_axi4_ifc: dumb 1:1 write and read intakes (pumice_wr_intake, pumice_rd_intake) feeding a write-data CAM (pumice_wr_data_cam, the write buffer / snarf source) and a read reorder CAM (pumice_rd_cmd_cam), with in-order commit. There is no monolithic txn_queue.

- AW / AR .ready deassert when the corresponding intake or CAM cannot accept a new request (insertion ready low).

- W .ready deasserts when the write-data path (CAM buffer) is full.

- R / B .valid follow standard protocol; consumers may stall indefinitely without controller misbehavior.

## 19.9	Host-Width Gearing

To use a host AXI data width different from DW, instantiate the optional wrapper pumice_top_geared (parameter HOST_AXI_DATA_WIDTH, verified at {64, 128, 256}). It inserts the repo’s formally-verified axi4_dwidth_converter_wr / axi4_dwidth_converter_rd between the host-width slave and the DW-width core. When HOST_AXI_DATA_WIDTH == DW, a generate branch bypasses the converters entirely, so the build is bit-identical to pumice_top (no converter, no added latency). See docs/AXI_DRAM_GEARING_SCOPE.md.

## 19.10	AXI4 Reset

aresetn is the AXI reset (active-low). When asserted, all AXI-side state is cleared. aclk / aresetn clock and reset the controller/AXI domain; the DFI side uses the independent dfi_clk / dfi_rstn (see the DFI chapter).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	DFI v2.1 Master Interface

The controller drives DFI v2.1 master signals to the PHY. DFI v2.1 is the first DFI revision that supports LPDDR2; this controller targets that revision.

## 20.1	Gearing and Per-Phase Packing

The gearing / phase count is set by the parameter DFI_RATE (default 2), not N_PHASES. Rather than the DFI _pN suffix convention, this controller packs the DFI_RATE sub-phases flat into each bus. All DFI pins carry the _o/_i direction suffix (e.g. dfi_address_o, dfi_rddata_i).

Widths (top-level pumice_top, derived from geometry):

- dfi_address_o — DFI_ADDR_BUS_W = ROW_WIDTH * DFI_RATE

- dfi_bank_o — $clog2(NUM_BANKS) * DFI_RATE

- dfi_cas_n_o / dfi_ras_n_o / dfi_we_n_o — 1 * DFI_RATE

- dfi_cs_n_o / dfi_odt_o — NUM_RANKS * DFI_RATE

- dfi_wrdata_o / dfi_rddata_i — DFI_DATA_WIDTH = DRAM_BEAT_WIDTH * DFI_RATE

- dfi_wrdata_mask_o — DFI_DATA_WIDTH/8

- dfi_wrdata_en_o / dfi_rddata_en_o — DFI_RATE

- dfi_rddata_valid_i — DFI_RATE

Per-phase slice p of a bus occupies [p*W_phase +: W_phase].

Command placement (see dfi_cmd_formatter.sv): the decoded DDR2 command is placed on a target sub-phase — READ on rd_phase, WRITE on wr_phase (both CSR-driven via the DFI_PHASE register), and all other commands (ACT / PRE / REF / MRS) on phase 0. Non-driven phases emit NOP. This aligns R/W commands with the PHY’s rdphase / wrphase contract.

## 20.2	Command Interface (MC → PHY)

| Signal | Width | Notes |
| --- | --- | --- |
| dfi_address_o | ROW_WIDTH*DFI_RATE | DDR2: row / column per phase; LPDDR2: 20-bit packed CA word on the low bits |
| dfi_bank_o | clog2(NUM_BANKS)*DFI_RATE | DDR2: bank; LPDDR2: held idle |
| dfi_cas_n_o | DFI_RATE | DDR2: CAS encoding; LPDDR2: held idle |
| dfi_ras_n_o | DFI_RATE | DDR2: RAS encoding; LPDDR2: held idle |
| dfi_we_n_o | DFI_RATE | DDR2: WE encoding; LPDDR2: held idle |
| dfi_cs_n_o | NUM_RANKS*DFI_RATE | Per-rank chip select; cs_n[r]=0 selects rank r |
| dfi_odt_o | NUM_RANKS*DFI_RATE | On-die termination |

There is no dfi_cke or dfi_reset_n port on the controller top; CKE / reset sequencing is not part of this pin bus.

### 20.2.1	DDR2 Command Encoding (JESD79-2)

The DDR2 truth table implemented by dfi_cmd_formatter.sv:

| op | cs_n | ras_n | cas_n | we_n | bank | addr[10] | addr[..0] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOP | 0 | 1 | 1 | 1 | X | X | X |
| ACT | 0 | 0 | 1 | 1 | BA | row | row |
| RD | 0 | 1 | 0 | 1 | BA | 0 | col |
| RDA | 0 | 1 | 0 | 1 | BA | 1 (auto-PRE) | col |
| WR | 0 | 1 | 0 | 0 | BA | 0 | col |
| WRA | 0 | 1 | 0 | 0 | BA | 1 (auto-PRE) | col |
| PRE | 0 | 0 | 1 | 0 | BA | 0 (single bank) | X |
| PREA | 0 | 0 | 1 | 0 | X | 1 (all banks) | X |
| REF | 0 | 0 | 0 | 1 | X | X | X |
| MRS | 0 | 0 | 0 | 0 | MR idx | MR data | MR data |

cs_n is per-rank: cs_n[r]=0 selects rank r; all-ones deselects (NOP). A10 (addr[10]) is the auto-precharge bit for RD/WR and the all-bank bit for PRE. MRS carries its mode-register data on the row field (cmd_row_i, ROW_WIDTH), not the column field, so the full DDR2 MR bit range is preserved.

### 20.2.2	LPDDR2 Command Encoding (JESD209-2F)

LPDDR2 is fully functional (reads and writes). LPDDR2 has no RAS/CAS/WE strobes: the command and a scrambled address are multiplexed onto the 10-bit CA bus over two clock edges. dfi_cmd_formatter.sv builds the bit-exact JESD209-2F Table 60 encoding as a flat 20-bit word (10 bits rising + 10 bits falling) placed on the low 20 bits of dfi_address_o; dfi_ras_n_o / dfi_cas_n_o / dfi_we_n_o are held idle and dfi_cs_n_o is asserted for the target rank. The full pin layout is the locked spec in rtl/LPDDR2_CA_ENCODING.md, which both the RTL and the DV BFM encode against.

## 20.3	Write Data Interface (MC → PHY)

| Signal | Width | Notes |
| --- | --- | --- |
| dfi_wrdata_o | DFI_DATA_WIDTH | Write data (one DFI word = all phases) |
| dfi_wrdata_en_o | DFI_RATE | High during valid write-data phases |
| dfi_wrdata_mask_o | DFI_DATA_WIDTH/8 | mask = ~wstrb (1 = mask byte) |

Write data is driven by pumice_dfi_wr_serializer: on a WR command it waits t_phy_wrlat DFI cycles, then streams one DFI word per cycle from the write-data FIFO until the burst’s last word (bubble-free, back-to-back bursts).

## 20.4	Read Data Interface (PHY ↔ MC)

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| dfi_rddata_i | DFI_DATA_WIDTH | PHY → MC | Read data (one DFI word) |
| dfi_rddata_valid_i | DFI_RATE | PHY → MC | High when a read word is valid |
| dfi_rddata_en_o | DFI_RATE | MC → PHY | Read capture window |

Read alignment is done by pumice_dfi_rd_aligner: on a RD command it asserts dfi_rddata_en_o for the burst’s word count starting t_rddata_en DFI cycles later, and captures dfi_rddata_i on each dfi_rddata_valid_i. There is no dfi_rddata_dnv port.

## 20.5	Status / Init Interface

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| dfi_init_start_o | 1 | MC → PHY | Controller requests PHY init start |
| dfi_init_complete_i | 1 | PHY → MC | High when the PHY has finished init |

These are the only status handshake pins on the controller top. Update and training sub-interfaces are not present on this pin bus.

## 20.6	Timing Parameters Honored

The DFI datapath (pumice_dfi_layer) honors these CSR-driven knobs:

- t_phy_wrlat — delay from the WR command to dfi_wrdata_en (write serializer)

- t_rddata_en — delay from the RD command to the dfi_rddata_en capture window (read aligner)

- rd_phase / wr_phase — the DFI sub-phase carrying the READ / WRITE command (DFI_PHASE register)

## 20.7	Clock Domains and CDC

The controller/AXI side runs on aclk; the PHY/DFI side runs on the independent input clock dfi_clk (with dfi_rstn). The two are not related by a fixed frequency ratio and there is no combinational phase replication.

The single clock-domain crossing lives inside pumice_dfi_layer (in pumice_dfi_cdc) and is built from asynchronous gaxi FIFOs — one each for the command, write-data, and read-data streams (plus the init handshake). The internal datapath unit is one DFI word (= one DFI cycle), so one FIFO word maps to one DFI cycle and the crossing is bubble-free.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	CSR Register Interface (PeakRDL cpuif)

Configuration and observation registers are exposed through a PeakRDL-generated register block (pumice_csr), not a hand-written APB slave. The register map is authored in rtl/macro/pumice_csr.rdl and generated with the passthrough cpuif (bin/peakrdl_generate.py); pumice_top instantiates the generated pumice_csr and drives the core by name from hwif_out.* (see §6.3 for the full map). The DV mirror used for by-name access is dv/tbclasses/pumice_regmap.py.

The SoC-facing bus is therefore the PeakRDL passthrough cpuif, a simple request/ack register interface. An SoC that natively speaks APB / AXI-Lite places a thin protocol adapter in front of this cpuif; the controller itself does not embed one in this generation.

## 21.1	cpuif Signals

CSR_ADDR_W = 12 (4 KB, 32-bit registers). The block runs on aclk / ~aresetn (the controller clock/reset — see below).

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| s_cpuif_req | 1 | input | Request valid |
| s_cpuif_req_is_wr | 1 | input | 1 = write, 0 = read |
| s_cpuif_addr | 12 | input | Register byte address (4 KB space) |
| s_cpuif_wr_data | 32 | input | Write data |
| s_cpuif_wr_biten | 32 | input | Per-bit write enable |
| s_cpuif_req_stall_wr | 1 | output | Back-pressure a write request |
| s_cpuif_req_stall_rd | 1 | output | Back-pressure a read request |
| s_cpuif_rd_ack | 1 | output | Read data valid |
| s_cpuif_rd_err | 1 | output | Read error (e.g. unmapped address) |
| s_cpuif_rd_data | 32 | output | Read data |
| s_cpuif_wr_ack | 1 | output | Write accepted |
| s_cpuif_wr_err | 1 | output | Write error |

## 21.2	Access Semantics

Standard PeakRDL passthrough behavior:

- Single-cycle accepts for the typical case; the *_stall_* outputs allow the block to hold a request when needed.

- s_cpuif_rd_err / s_cpuif_wr_err are raised for unmapped addresses; the error terminates the access and does not disturb controller state.

- Read-only fields ignore writes; write-only / self-modifying fields (e.g. CTRL.init_start, CTRL.soft_reset) pulse swmod.

## 21.3	Clock Domain

The register block is clocked by aclk (the controller / host-AXI clock) and reset by ~aresetn. It is not on a separate management clock in this generation — there is no independent apb_pclk. Config fields (hwif_out.*, hw = r) are read combinationally by pumice_top and fanned out to the core; the single clock-domain crossing to the PHY side (dfi_clk) lives inside pumice_dfi_layer (async gaxi FIFOs), not in the register block. Observation fields (hwif_in.*, hw = w) are written from the core clock domain.

## 21.4	Register Map

The full address map is documented in §6.3, reconciled against rtl/macro/pumice_csr.rdl and dv/tbclasses/pumice_regmap.py.

## 21.5	Configuration Sequencing

Recommended bring-up order (see §6.1 for reset details):

- Hold aresetn asserted.

- Program timing parameters (TIMINGS_*), MR overrides, and PHY_TIMING (including memtype and t_phy_wrlat / t_rddata_en).

- Program ADDR_MAP (bank_lsb / hash_en / hash_seed) and DFI_PHASE.

- Configure PASR mask (LPDDR2 only).

- Release aresetn.

- Wait for STATUS.init_done.

- Begin AXI traffic.

Runtime tuning writes (scheduler / refresh / page policy) take effect at the next configuration quiet point; the SoC owns the drain — see §5.4.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Build-Time vs. Runtime Configuration

This controller distinguishes between build-time parameters that set the silicon footprint and runtime CSR registers that select behavior within that footprint. The two layers serve different purposes and should not be confused. This chapter applies the principle to every configurable knob in the design.

## 22.1	Principle

| Layer | What it controls | Frozen at | Cost of changing |
| --- | --- | --- | --- |
| Build-time parameter | Synthesized silicon footprint: signal widths, table depths, queue sizes, instantiation counts, encoder selection, MAX bounds of runtime values | Elaboration time | Full RTL rebuild + place-and-route + tape-out (silicon) or bitstream rebuild (FPGA) |
| Runtime CSR register | Behavior selection within the synthesized footprint: policy choices, threshold values, on/off toggles, active counts up to the build-time MAX | Programmable any time the configuration quiet point is reached (typically during init or under SoC orchestration) | A single APB write |

The architectural rule is straightforward: if a knob is changing silicon area, it must be build-time. If a knob is only changing behavior of already-synthesized hardware, it must be runtime.

## 22.2	Why This Distinction Matters

A controller designed without this distinction tends to collapse one of two ways:

- Too much build-time — every algorithmic choice is a parameter that requires a new bitstream / silicon revision to evaluate. Characterization becomes impossibly slow; post-silicon debug becomes impossible. Most academic prototype controllers fall here.

- Too much runtime — every option is always synthesized, including ones that will rarely be used. Area balloons; timing closure suffers; the design becomes harder to verify because every code path is live. This is the failure mode of “ultra-configurable” commercial IP.

This controller targets the middle: build-time picks what’s physically present; runtime picks what’s active.

## 22.3	The Hybrid Pattern: “Build-Time MAX + Runtime Active”

Many of this controller’s knobs follow a hybrid pattern where the build-time parameter sets a synthesized MAX, and a paired runtime CSR field selects the actually-used value within that MAX.

Concrete example for LOOKAHEAD_DEPTH:

Build-time:    LOOKAHEAD_DEPTH_MAX = 4
               → synthesizes 4 row-address comparators per bank machine

Runtime CSR:   sched_tuning.lookahead_active [3:0]
               → scheduler peeks min(LOOKAHEAD_DEPTH_MAX, lookahead_active)
                 same-bank requests per cycle

For debug:     Write lookahead_active = 0 to force pure HAPPY/page-policy mode
               Write lookahead_active = 4 to use the full synthesized window
               No rebuild required.

The same pattern applies to TXN_QUEUE_DEPTH, AGE_MAX, REFRESH_DEFER_MAX, INIT_ZQ_RETRIES, ZQCS_FREQ_HZ, and others — area is paid once at synthesis for the MAX; runtime selects how much of the synthesized hardware is actively gated.

## 22.4	The “Build-Time + Runtime Mux” Pattern

For algorithmic choices where multiple alternatives have similar area cost, the build-time parameter selects which code paths are synthesized, and a runtime CSR field picks which of the synthesized paths is active.

Concrete example for PAGE_POLICY:

Build-time:    PAGE_POLICY = HAPPY_HYBRID
               → synthesizes OPEN, CLOSE, and HAPPY decision logic
                 (plus the HAPPY predictor table)

Runtime CSR:   refresh_tuning.page_policy_or [1:0]
               00 = use build-time default (HAPPY_HYBRID here)
               01 = force OPEN
               10 = force CLOSE
               11 = force HAPPY_HYBRID

For debug:     Sweep policies on real workloads with no rebuilds.

The build-time choice PAGE_POLICY = OPEN would have omitted the HAPPY predictor table from the synthesized design entirely, saving area but losing the ability to switch to HAPPY at runtime. The build-time choice PAGE_POLICY = HAPPY_HYBRID synthesizes everything, paying the area cost for the predictor table, in exchange for the ability to switch among any of the three at runtime.

The same pattern applies to REFPB_POLICY, SCHEDULER_MODE, and HAPPY enable/disable.

Note that address mapping is not an example of this pattern. Earlier revisions exposed an ADDR_MAP_SCHEME build-time set plus an ADDR_MAP_TUNING.scheme_or runtime mux; both have been retired. Address mapping is now a single runtime placement knob — see “The ‘Runtime Placement’ Pattern” below.

## 22.5	The “Runtime Placement” Pattern

Some knobs neither gate synthesized alternatives nor cap a synthesized MAX — they simply reposition a field within already-present combinational logic. There is no mux and no area choice; the same decode hardware produces different behavior depending on a runtime value.

Address mapping is the canonical case. addr_mapper.sv decodes a flat AXI address into {rank, bank, row, col} from one runtime CSR field, ADDR_MAP.bank_lsb, which selects where the bank field sits in the byte-offset-stripped word address. The column fills below (col_lo) and above (col_hi) the bank; row/rank stack above at invariant positions. The classic “schemes” are just settings of bank_lsb:

Runtime CSR:   ADDR_MAP.bank_lsb [4:0]   (reset = 0x0A = COL_WIDTH = ROW_MAJOR)
               = COL_WIDTH               -> bank above whole column = ROW_MAJOR
               = log2(BL/DFI_RATE)       -> minimal col_lo          = BANK_INTERLEAVE
               in between                -> partial interleave

Runtime CSR:   ADDR_MAP.hash_en [8], ADDR_MAP.hash_seed [23:16]
               folds row bits + seed into the bank index (= XOR_HASH),
               defeating power-of-two-stride hot-banking.

There is no build-time parameter and no scheme selector: ROW_MAJOR, BANK_INTERLEAVE, and XOR_HASH are all reachable by writing bank_lsb (and optionally hash_en/hash_seed) at runtime. The address-decode logic is combinational and always present.

MEMTYPE is likewise a runtime knob in this generation, not a build-time string. The core decodes memtype_e from the CSR field PHY_TIMING.memtype (0 = DDR2, 1 = LPDDR2) — see pumice_top.sv w_memtype. There is no MEMTYPE build parameter that gates an encoder or step table; the DDR2 vs LPDDR2 behavior is selected at runtime within the synthesized datapath.

## 22.6	The “Runtime Only” Pattern

A small number of knobs have zero area cost and are runtime-only: simple counter compare values, timeout thresholds, retry counts. Examples include ZQCS_FREQ_HZ, INIT_ZQ_RETRIES, and the various observation-counter clear-on-read controls.

These have no build-time presence at all.

## 22.7	Configuration Quiet Points

Runtime CSR writes that change scheduler or refresh-manager behavior take effect at the next configuration quiet point — defined as: no DRAM commands in flight, no refresh sequence in progress, and the relevant FSM in its idle state. The CSR slave does not enforce quiet-point writes; the SoC firmware is responsible for sequencing. Writing during normal operation is well-defined but the change does not apply until the next quiet point. There is no config_apply strobe in this generation — the SoC owns the drain and must quiesce host AXI traffic (and, if needed, wait out any in-flight refresh) before the change is guaranteed to take effect.

Writes to register-file CSRs that have no FSM impact (timing parameter overrides, MR override values, observation counters) take effect immediately and have no quiet-point requirement.

## 22.8	Summary

The following two chapters document this controller’s complete configuration interface:

- §5.2 Top-Level Parameter Table — every build-time parameter, with its type, range, governing section, and (where applicable) the paired runtime CSR field.

- §5.3 Characterization Knobs — the subset of build-time + runtime knobs intentionally exposed for performance sweeping during silicon / FPGA characterization.

The full runtime CSR register map is in §6.3.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Top-Level Parameter Table

All build-time parameters for the controller, with type, default, valid range, the section that governs their use, and a one-line purpose summary.

## 23.1	Parameter Table

The parameters below are the actual elaboration parameters of pumice_top / pumice_core (with the host-width parameter added by pumice_top_geared). Several knobs that earlier revisions listed as build-time — MEMTYPE, WRPHASE, RDPHASE, and the address-map scheme set — are runtime CSR fields in this generation and are called out under “Runtime CSR knobs (not build parameters)” below.

| Parameter | Type | Default | Range / valid set | Section | Purpose |
| --- | --- | --- | --- | --- | --- |
| AXI_ID_WIDTH | int | 8 | 1-16 | §3.1 | AXI ID tag width (IW). |
| AXI_ADDR_WIDTH | int | 32 | 24-40 | §3.1 | AXI flat address width (AW). |
| NUM_RANKS | int | 1 | {1, 2, 4} | §3.3 | Number of physical ranks (per-rank CS_n, CKE, ODT). 1 = single-rank point-to-point; 2 / 4 = DIMM-class. Rank bits stack at the top of the address map (§3.1) and refresh state is tracked per (rank, bank). |
| NUM_BANKS | int | 8 | {4, 8} | §3.3 | Per-device bank count (per rank). Sets the bank-machine / lookup count. |
| ROW_WIDTH | int | 14 | 12-16 | §3.3 | Row-address width. |
| COL_WIDTH | int | 10 | 9-12 | §3.3 | Column-address width. Also the ROW_MAJOR setting of ADDR_MAP.bank_lsb. |
| DFI_RATE | int | 2 | {1, 2, 4} | §3.6 | DFI frequency-ratio gear (phase count). Drives all DFI bus widths and the internal data-unit width. This is the gear parameter formerly called N_PHASES. |
| DRAM_BEAT_WIDTH | int | 64 | {16, 32, 64} | §3.6 | Width of one DRAM data beat (per DFI phase). |
| BL | int | 8 | {4, 8} | §3.6 | DRAM burst length in beats. The AXI intake splits host bursts into fixed-BL commands; one AXI burst at the core (DW) side is (awlen+1)*DFI_RATE == BL. |
| NUM_ENTRIES | int | 8 | 4-32 (power of 2) | §3.2 | Depth of the read/write command CAMs (in-flight transaction slots). Pointer width is clog2(NUM_ENTRIES). |
| N_SRAM_SLOTS | int | 8 | 4-32 (power of 2) | §3.2 | Write-data SRAM slot count (buffered write payloads awaiting commit). |
| BYTE_OFFSET_WIDTH | int | 3 | clog2(beat byte size) | §3.1 | Low byte-offset bits stripped before address decode (log2 of beat byte size). |
| AGE_WIDTH | int | 16 | 8-24 | §3.2 | Width of the per-transaction age counter used for FR-FCFS anti-starvation tie-breaking. |
| HOST_AXI_DATA_WIDTH | int | 128 | {64, 128, 256} (verified) | §3.1 | pumice_top_geared only. Free host-side AXI data width. Formally-verified axi4_dwidth_converter_wr/_rd shims bridge the host width to the fixed core width DW. HOST_AXI_DATA_WIDTH == DW is a generate-bypassed, bit-identical direct connection (no converter, no added latency). |

Derived parameters (computed at elaboration, not overridable independently):

- DW = DRAM_BEAT_WIDTH * DFI_RATE — the fixed core AXI data width and the DFI word width (default 64 * 2 = 128). DFI_DATA_WIDTH = DW. One core AXI beat == one DFI word == DFI_RATE DRAM beats.

- SW = DW / 8 — core AXI / DFI strobe width.

- PHW = clog2(DFI_RATE) (min 1) — DFI sub-phase index width.

- DFI_STRB_WIDTH = DW/8, DFI_EN_WIDTH = DFI_VALID_WIDTH = DFI_RATE, DFI_ADDR_BUS_W = ROW_WIDTH*DFI_RATE, DFI_BANK_BUS_W = clog2(NUM_BANKS)*DFI_RATE, DFI_CS_BUS_W = NUM_RANKS*DFI_RATE — the DFI 2.1 pin-bus widths, all scaled by DFI_RATE.

Runtime CSR knobs (not build parameters):

These select behavior at runtime and have no build-time parameter in this generation. Full field detail is in §6.3.

- PHY_TIMING.memtype (1-bit) — 0 = DDR2, 1 = LPDDR2. The core decodes memtype_e from this field (pumice_top.sv w_memtype); there is no build-time MEMTYPE string.

- DFI_PHASE.wr_phase / DFI_PHASE.rd_phase (3-bit each, sliced to PHW downstream) — which DFI sub-phase carries the WRITE / READ command. Defaults 0/0. These replace the former build-time WRPHASE / RDPHASE.

- ADDR_MAP.bank_lsb (5-bit, reset 0x0A = COL_WIDTH = ROW_MAJOR), ADDR_MAP.hash_en (1-bit), ADDR_MAP.hash_seed (8-bit) — the single address-map placement knob (+ optional bank XOR-hash). The classic ROW_MAJOR / BANK_INTERLEAVE / XOR_HASH “schemes” are just settings of these fields; there is no ADDR_MAP_SCHEMES_SYNTH / ADDR_MAP_SCHEME_DEFAULT build parameter and no scheme mux. See addr_mapper.sv.

- REFRESH_TUNING.page_policy_or (2-bit) — 00 = build-time default, 01 = OPEN, 10 = CLOSE, 11 = HAPPY_HYBRID.

- REFRESH_TUNING.refpb_policy_or (2-bit), REFRESH_TUNING.refresh_defer_active (4-bit), REFRESH_TUNING.zqcs_freq_hz (16-bit, reset 1 Hz).

- SCHED_TUNING.lookahead_active (4-bit), force_inorder (1-bit), happy_enable (1-bit, reset 1), age_max_runtime (8-bit), txn_queue_high_water (8-bit); lookahead_max_obs (4-bit, RO echo of the build MAX).

- PAGE_PRED_TUNING.warmup_cycles (16-bit, reset 1024), hysteresis (8-bit, reset 2).

- Timing CSRs — TIMINGS_RC_RCD_RP_RAS (tRC/tRCD/tRP/tRAS), TIMINGS_RFC_REFI (tRFC/tREFI), TIMINGS_RRD_FAW_WTR_CCD (tRRD/tFAW/tWTR/tCCD), TIMINGS_CL_CWL_WR (CL/CWL/tWR/tRFCpb), TIMINGS_RTP_RTW (tRTP/tRTW).

- PHY_TIMING.refresh_burst (4-bit, 1..8), PHY_TIMING.t_phy_wrlat (8-bit), PHY_TIMING.t_rddata_en (8-bit).

- Init timings — INIT_TIMING0 (t_init_wait/t_dll_wait), INIT_TIMING1 (t_mrd_wait/t_rp_wait/t_rfc_wait), INIT_TUNING.zq_retries (4-bit, reset 3), INIT_TUNING.init_timeout_ms (8-bit, reset 10).

These cost zero silicon area beyond the register flops; making them build-time parameters would have been wrong (and was, in earlier revisions of this document).

## 23.2	Memtype-Dependent Constraints

MEMTYPE is a runtime CSR field (PHY_TIMING.memtype) rather than an elaboration parameter, so the DDR2-vs-LPDDR2 selection does not gate elaboration-time asserts. The memtype-dependent behavior is instead enforced at run time within the synthesized datapath:

- PHY_TIMING.memtype == 1 (LPDDR2) enables the LPDDR2-only features (PASR bank/segment masks, DPD request, temperature-derate readback, per-bank tRFC via TIMINGS_CL_CWL_WR.tRFCpb).

- PHY_TIMING.memtype == 0 (DDR2) uses the DDR2 command encoding and all-bank refresh.

- The DFI address bus is ROW_WIDTH * DFI_RATE wide (DFI_ADDR_BUS_W) and carries the LPDDR2 CA packing when memtype == 1; it must be wide enough for whichever memtype is programmed, which is guaranteed by ROW_WIDTH >= 14.

## 23.3	Timing-Configuration Sanity Checks

The controller expects the following relationships to hold on the loaded timing CSR values. Because timings are runtime CSRs, these are boot-time programming contracts the SoC firmware must honor (they are documented here so the sweep tool and bring-up scripts can assert them):

- tREFI_cycles >= 100 — the MC clock must be fast enough relative to tREFI that the refresh timer is well-resolved.

- tRCD_cycles >= 2, tRP_cycles >= 2, tRC_cycles >= tRAS + tRP — basic JEDEC consistency checks.

- tRFC_cycles > tRP_cycles — the refresh-cycle time must exceed the precharge time.

These are configuration bugs that should be caught at boot time before traffic starts, not preventable run-time faults.

## 23.4	Parameter Categories

| Category | Parameters |
| --- | --- |
| Geometry | NUM_RANKS, NUM_BANKS, ROW_WIDTH, COL_WIDTH |
| Multi-rank | NUM_RANKS |
| Bus widths | AXI_ID_WIDTH, AXI_ADDR_WIDTH, DRAM_BEAT_WIDTH, BYTE_OFFSET_WIDTH, HOST_AXI_DATA_WIDTH; derived DW/DFI_DATA_WIDTH |
| Gear / DFI | DFI_RATE, BL (with runtime DFI_PHASE.rd_phase / DFI_PHASE.wr_phase) |
| Capacity | NUM_ENTRIES, N_SRAM_SLOTS, AGE_WIDTH |
| Characterization | Runtime CSR knobs paired with the geometry above — SCHED_TUNING.lookahead_active, REFRESH_TUNING.page_policy_or / refpb_policy_or / refresh_defer_active, PAGE_PRED_TUNING.*, ADDR_MAP.bank_lsb / hash_en (see §5.3 and §6.3) |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Characterization Knobs

A subset of the build-time parameters is intentionally left configurable for characterization sweeping. These are the algorithmic choices that have research-backed alternatives where the best choice depends on workload. Rather than baking in a one-size-fits-all default, we expose them as parameters so the verification plan can sweep them on representative AXI traffic and pick defaults from data.

## 24.1	Characterization Parameters

| Parameter | Choices | Recommended default | Rationale |
| --- | --- | --- | --- |
| LOOKAHEAD_DEPTH | 0 / 1 / 2 / 4 | 1 | Same-bank lookahead window for exact auto-precharge decisions when the next same-bank request is already in the queue (typical for streaming / DMA). Falls back to PAGE_POLICY when the queue is shallow. 0 disables; 2 or 4 buys diminishing returns at modest comparator cost. |
| PAGE_POLICY | OPEN / CLOSE / HAPPY_HYBRID | HAPPY_HYBRID (LPDDR2), OPEN (DDR2) | Fallback for when lookahead is inconclusive. Streaming workloads favor OPEN; mobile-mixed favors HAPPY_HYBRID. Ghasempour 2015 reports near-optimal accuracy with small storage. |
| PAGE_PREDICTOR_TABLE_BITS | 8 – 16 | 12 | 4 KB predictor is sufficient for typical mobile workloads. Larger tables marginal improvement. |
| REFPB_POLICY | ROUND_ROBIN / OLDEST_FIRST / DARP | DARP | Chang HPCA 2014: DARP wins over round-robin on uneven bank activity (real workloads). |
| REFRESH_DEFER_MAX | 1 – 8 | 8 (JEDEC ceiling) | Stuecheli MICRO 2010: deferring up to 8 maximizes Elastic Refresh benefit. Set to 1 for real-time predictability. |
| TXN_QUEUE_DEPTH | 4 – 64 | 16 | Larger queues help bandwidth on multi-master AXI; smaller queues reduce area. |
| AGE_MAX | 32 – 1024 | 256 | Caps FR-FCFS starvation; larger values let row-hits dominate longer at the cost of tail latency. |
| DFI_RATE | 1 / 2 / 4 | depends on PHY | Build-time DFI gear (phase count). 4 typical for FPGA SoCs; 2 for slower mobile platforms; 1 for simulation-only. Formerly named N_PHASES. |
| ADDR_MAP.bank_lsb (+ hash_en) | log2(BL/DFI_RATE) .. COL_WIDTH, hash off / on | bank_lsb = COL_WIDTH (ROW_MAJOR), hash off | Runtime address-map placement knob (not a build parameter). bank_lsb = COL_WIDTH = ROW_MAJOR (best for streaming); lowering bank_lsb toward log2(BL/DFI_RATE) interleaves banks (best for random); hash_en folds row bits into the bank index to defeat power-of-two-stride hot-banking (adversarial). Sweep bank_lsb across the legal range with hash off, then repeat the best point with hash on. See addr_mapper.sv. |

## 24.2	Sweep Plan

The verification plan (see §6.4) includes a characterization sweep that runs each parameter through its choices on a small benchmark suite:

### 24.2.1	Benchmark Suite

A key open question for the sweep is the interaction between LOOKAHEAD_DEPTH and PAGE_POLICY. With deep queues and conclusive lookahead, the choice of PAGE_POLICY barely matters because the fallback rarely fires. With shallow queues, PAGE_POLICY dominates. The sweep matrix should cover both extremes.

| Workload | Pattern |
| --- | --- |
| stream-read | Sequential 4 KB read bursts |
| stream-write | Sequential 4 KB write bursts |
| stream-mixed | Interleaved sequential read and write |
| random-narrow | Random small AXI bursts (1–4 beats) |
| random-wide | Random large AXI bursts (16–64 beats) |
| mobile-mixed | Bursty traffic with periodic large writes (modeled on smartphone GPU + display) |
| multi-master | Two AXI masters issuing concurrent traffic with different patterns |

### 24.2.2	Metrics Collected

For each (workload, parameter) combination, the sweep records:

- Sustained read bandwidth (% peak)

- Sustained write bandwidth (% peak)

- Average read latency

- 99th-percentile read latency

- Refresh blocking time (% of cycles where the bus was blocked by refresh)

- Row-hit rate (per-bank, averaged)

- Refresh deferral utilization (avg refresh_pending over time)

### 24.2.3	Default Selection

After the sweep, each parameter’s default is set to the value that:

- Maximizes sustained bandwidth across the benchmark suite

- Subject to: 99th-percentile latency does not regress by more than 20%

- Subject to: area cost is within budget

The defaults shown in the parameter table above are the expected outcomes based on the cited research. The actual defaults will be confirmed by the sweep.

## 24.3	Runtime Overrides via CSR

Some characterization parameters are also accessible at runtime via CSR (see §6.3). This allows in-system tuning without rebuild:

- PAGE_POLICY runtime override (REFRESH_TUNING.page_policy_or)

- REFPB_POLICY runtime override (REFRESH_TUNING.refpb_policy_or)

- Address-map placement (ADDR_MAP.bank_lsb, hash_en, hash_seed) — this is a runtime knob outright, not an override of a build default

- Page predictor warm-up cycles and hysteresis (PAGE_PRED_TUNING.warmup_cycles / hysteresis)

Runtime overrides take effect at the next “quiet point” (no commands in flight). The SoC is responsible for ensuring no DRAM traffic during the transition.

## 24.4	Observability for Sweep

The CSR exposes the following observation counters (described in §6.3):

- OBS_ROW_HIT[8] — per-bank rolling row-hit count (clear-on-read).

- OBS_REF_LATENCY[8] — per-bank average refresh-blocking cycles.

- OBS_TXN_QUEUE_DEPTH_MAX / OBS_TXN_QUEUE_DEPTH_AVG — max and time-averaged transaction-queue depth.

- OBS_PAGE_PRED_ACCURACY — HAPPY-mode rolling prediction accuracy (%).

- OBS_REFRESH_DEFER_HIST_0..3 — refresh-deferral histogram bins (plus OBS_REFRESH_PENDING_MAX for the max refresh_pending observed).

- OBS_AXI_R_LATENCY_AVG / OBS_AXI_R_LATENCY_P99 / OBS_AXI_W_LATENCY_AVG — AXI read/write latency counters.

These feed back into the sweep tool for in-loop default selection.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Family-Wide Config Bits

This section is the family-wide config-bit registry — a catalog of runtime configuration bits used across the DDR2 / LPDDR2, DDR3 / LPDDR3, and DDR4 / LPDDR4 memory controllers. Some bits apply to all generations; some are introduced in one generation and absent in earlier ones; some are defined across the family but ignored in flavors where the underlying mechanism does not exist. That last case is intentional. Allocating the bit family-wide — even when unused — keeps the CSR map mechanically compatible across controllers and lets software treat the family as a single product line.

## 25.1	Why a Family-Wide Registry

Each controller in the family has its own CSR map (per §6.3 of each generation’s HAS). But the field encodings, bit positions within shared registers, and the names are aligned across the family. The intent:

- Software portability: a single device driver template understands the bit layout and only needs per-flavor capability checks.

- Bring-up portability: bring-up software written for DDR3 should mostly work on DDR2 with the same register reads.

- Verification portability: the cocotb test suite’s CSR-poke helpers can be reused.

- Debug consistency: the same “config dump” script produces a parseable output regardless of which flavor is running.

A bit may be marked N/A in a flavor; reads return 0 and writes are ignored (no pslverr, just silently ignored — because software portability requires that writing a fully-portable config word does not error on flavors that don’t implement every bit).

## 25.2	Applicability Notation

The Apply column uses this shorthand:

- all — applies to every flavor in the family

- DDR-only — applies to DDR2/3/4 only; LPDDR variants ignore

- LP-only — applies to LPDDR2/3/4 only; DDR variants ignore

- DDR3+ — applies to DDR3, DDR4, and forward; DDR2 ignores

- DDR4+ — applies to DDR4 only (introduced for bank groups); earlier ignore

- LPDDR3+ — LPDDR3 and LPDDR4 only

- LPDDR4-only — LPDDR4 only

## 25.3	SCHED_TUNING (Scheduling Family Bits)

Register block at the same offset (0x040) in every controller. These bits govern the FR-FCFS scheduler’s runtime behavior.

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| lookahead_active | 4 | all | Runtime lookahead window (0 .. build-time MAX). 0 disables lookahead. |
| force_inorder | 1 | all | Forces FR-FCFS into first-ready FIFO mode for debug / real-time. |
| happy_enable | 1 | all | Runtime kill-switch for the HAPPY page predictor. Reads as 0 in flavors built without HAPPY. |
| age_max_runtime | 8 | all | Anti-starvation age cap; runtime override of AGE_MAX. |
| txn_queue_high_water | 8 | all | Backpressure threshold; AXI awready deasserts when queue ≥ threshold. |
| lookahead_max_obs | 4 | all (R only) | Echo of build-time LOOKAHEAD_DEPTH_MAX. Software discovery. |
| qos_high_priority | 1 | all | PLANNED — not in the RDL yet (roadmap Axis 1, QoS step): when 1, scheduler boosts requests with awqos/arqos ≥ 8. |
| bank_group_balance | 1 | DDR4+ | Round-robin across bank groups in scheduler tie-break (DDR4/LPDDR4 only); N/A elsewhere. |

## 25.4	REFRESH_TUNING (Refresh Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| refpb_policy_or | 2 | LP-only | REFpb selection-policy override (LPDDR2/3/4); N/A for DDR-only flavors |
| page_policy_or | 2 | all | Runtime override for fallback page policy |
| refresh_defer_active | 4 | all | Active deferral count (1 .. build-time MAX). 1 = no batching. |
| zqcs_freq_hz | 16 | all | Periodic ZQCS interval. 0 = init-only. |
| fgr_mode | 2 | DDR3+ | Fine-Granularity Refresh mode (DDR3 1x/2x/4x; DDR4 fixed-rate, on-the-fly). N/A in DDR2. |
| refresh_per_rank | 1 | all | Force per-rank dispatch even when NUM_RANKS=1 (debug). Reset = 1 always when multi-rank. |

## 25.5	ADDR_MAP (Address-Mapping Family Bits)

The address-map register at 0x04C is a single placement knob, not a scheme mux. The old ADDR_MAP_TUNING register (with a scheme_or selector and a synth_mask_obs synthesized-scheme bitmask) has been retired in this generation: the classic ROW_MAJOR / BANK_INTERLEAVE / XOR_HASH schemes are just settings of bank_lsb plus the optional hash_en fold, so there is nothing to mux (see §5.4 §3/§4 and rtl/fub/addr_mapper.sv).

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| bank_lsb | 5 | all | Bank-field LSB in the word address. = COL_WIDTH -> ROW_MAJOR; lower -> interleave. RTL clamps to [0, COL_WIDTH]. |
| hash_en | 1 | all | Enable bank XOR-hash fold (bank ^= fold(row) ^ hash_seed) = the old XOR_HASH scheme. |
| hash_seed | 8 | all | XOR-hash seed. Runtime seed change without rebuild. |

Family note: for DDR4+ the bank-group field is placed by extending the same bank_lsb-relative stack rather than by a separate bg_field_position selector; that generation’s RDL documents the exact field split.

## 25.6	INIT_TUNING (Init Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| zq_retries | 4 | all | ZQ calibration retry count (DDR2 uses OCD; LPDDR2 uses MR10 ZQ). |
| init_timeout_ms | 8 | all | Per-step init timeout (ms; scaled by SIM_INIT_SCALE in sim). |
| wl_retries | 4 | DDR3+ | Write-leveling retry count. N/A in DDR2 (no write-leveling). |
| mpr_enable | 1 | DDR3+ | Multi-Purpose Register read-back during init. N/A pre-DDR3. |
| ca_train_enable | 1 | LPDDR3+ | LPDDR3/4 CA-bus training. N/A in LPDDR2 (no CA training). |
| cbt_enable | 1 | LPDDR4-only | Command-Bus-Training. N/A pre-LPDDR4. |

## 25.7	POWER_TUNING (Power-State Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| apd_idle_threshold | 16 | all | Cycles of bank-idleness before APD entry |
| srf_idle_threshold | 24 | all | Cycles before Self-Refresh entry |
| dpd_enable | 1 | LP-only | Deep-Power-Down enable. N/A for DDR-only flavors. |
| pasr_active | 1 | LP-only (R) | Reflects whether any rank has a non-zero PASR mask. R-only. |
| ckedis_after_sr | 1 | DDR3+ | Drop CKE after SR-entry for sub-mW power floor. N/A in DDR2. |
| low_freq_mode | 1 | DDR3+ | Reduced-frequency operation hint to PHY. N/A in DDR2. |

## 25.8	ECC_TUNING (Future — Inline ECC)

This block is reserved for future inline-ECC controllers. The bits are N/A in all current controllers (DDR2/3/4 and LPDDR2/3/4 v1) because inline ECC is out of scope for the current generation. Future revisions of any flavor that adopt inline ECC will populate this block.

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| ecc_enable | 1 | (reserved) | Enable inline ECC. Reads as 0 in all v1 controllers. |
| ecc_correct_mode | 2 | (reserved) | SECDED / Chipkill / off |

## 25.9	RANK_TUNING (Multi-Rank Family Bits)

These are present in any flavor with NUM_RANKS > 1. For NUM_RANKS = 1 builds, the bits read as 0 and writes are silently ignored.

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| rank_enable_mask | 4 | all | Per-rank enable; clearing a bit suppresses all commands to that rank. Up to 4 ranks. |
| odt_rule_or | 2 | all | Runtime override for ODT_RULE_MULTIRANK (00 = build-time default; 01 = JEDEC_DDR2; 10 = JEDEC_LPDDR2; 11 = OFF). |
| num_ranks_obs | 3 | all (R only) | Echo of build-time NUM_RANKS. Software discovery. |
| cs_assert_cycles | 4 | all | Number of cycles to hold CS_n asserted per command; tunable for PHY timing margin. |

## 25.10	Quiet-Point Behavior

Runtime config bits take effect at the next configuration quiet point — no DRAM commands in flight, no refresh sequence in progress. The register block does not enforce quiet points automatically; SoC firmware is responsible for sequencing writes at a drain point (typically during init or an orchestrated idle window). This generation’s RDL does not implement a config_apply strobe or a config_settled status bit — a future family revision may add one; today the SoC owns the drain.

The quiet-point requirement is family-wide because every flavor has hand-off state between scheduler / refresh / power-state that would be corrupted by mid-flight reconfiguration.

## 25.11	Bit Discovery via the ID Register

This generation does not implement a dedicated capability vector; software discovers the build via the ID register at 0xFF0 (see §6.3): memtype, n_phases (gear ratio), and version are readable there, and module_id is the fixed 0xD2 family tag. Echo fields embedded in the tuning registers (SCHED_TUNING.lookahead_max_obs, etc.) expose the remaining synthesized ceilings. A future family revision may add a packed capability vector along the lines below:

| Bits | Field | Description |
| --- | --- | --- |
| 0 | cap_happy | 1 = HAPPY predictor synthesized |
| 1 | cap_bg_balance | 1 = bank-group balance scheduler logic present (DDR4+) |
| 2 | cap_fgr | 1 = Fine-Granularity Refresh supported (DDR3+) |
| 3 | cap_pasr | 1 = PASR mask is implemented (LP-only) |
| 4 | cap_dpd | 1 = Deep-Power-Down supported (LP-only) |
| 5 | cap_wl | 1 = Write-leveling implemented (DDR3+) |
| 6 | cap_cbt | 1 = CBT implemented (LPDDR4-only) |
| 7 | cap_xor_hash | 1 = bank XOR-hash implemented |
| 11:8 | cap_max_ranks | Build-time NUM_RANKS (echo of geometry) |
| 15:12 | cap_n_phases | Build-time gear ratio (DFI_RATE) |
| 19:16 | cap_memtype | 0 = DDR2, 1 = DDR3, 2 = DDR4, 4 = LPDDR2, 5 = LPDDR3, 6 = LPDDR4 |

## 25.12	Per-Flavor Applicability Matrix

| Bit | DDR2 | DDR3 | DDR4 | LPDDR2 | LPDDR3 | LPDDR4 |
| --- | --- | --- | --- | --- | --- | --- |
| lookahead_active | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| force_inorder | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| happy_enable | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| bank_group_balance | — | — | ✓ | — | — | ✓ |
| refpb_policy_or | — | — | — | ✓ | ✓ | ✓ |
| fgr_mode | — | ✓ | ✓ | — | — | — |
| bg_field_position | — | — | ✓ | — | — | ✓ |
| wl_retries | — | ✓ | ✓ | — | — | — |
| ca_train_enable | — | — | — | — | ✓ | ✓ |
| cbt_enable | — | — | — | — | — | ✓ |
| dpd_enable | — | — | — | ✓ | ✓ | ✓ |
| ckedis_after_sr | — | ✓ | ✓ | — | — | — |
| rank_enable_mask | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| odt_rule_or | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| hash_en / hash_seed | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |

The “—” entries are present in the CSR map at the documented offsets but read 0 and ignore writes in that flavor. Software treats these as “soft-N/A” — not an error condition, just an absent feature.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Clocks and Reset

## 26.1	Clock Domains

The controller uses two independent clocks. The register block is a PeakRDL passthrough cpuif clocked by aclk — there is no separate management clock.

### 26.1.1	aclk

Controller clock. All host-AXI4 logic, the scheduler, the bank timers, the write/read CAMs, and the register block (pumice_csr) run on aclk. The controller-side of the command / write-data / read-data streams into the DFI layer is on aclk.

Typical frequency: 100 – 200 MHz on FPGA SoCs; higher on silicon.

### 26.1.2	dfi_clk

PHY / DFI-side clock. The DFI 2.1 command bus, write serializer, and read aligner run on dfi_clk; the PHY consumes and drives the DFI pin bus on this clock. The gear ratio (DFI_RATE) sets how many DRAM phases each dfi_clk cycle carries.

aclk and dfi_clk are asynchronous to each other. The single crossing between them lives inside pumice_dfi_layer (see the CDC summary below).

## 26.2	Reset Signals

Both resets are active-low.

### 26.2.1	aresetn (active low, controller / AXI domain)

Controller reset, tied to the aclk domain. The register block resets on ~aresetn (pumice_csr .rst(~aresetn)). Resets:

- Host AXI4 interface state (pumice_axi4_ifc)

- Write-data and read-command CAMs

- Command scheduler and bank timers (pumice_mem_cmd_scheduler)

- Refresh manager and init sequencer

- Register block (pumice_csr) and observation state

### 26.2.2	dfi_rstn (active low, DFI / PHY domain)

DFI-domain reset, tied to the dfi_clk domain. Resets the DFI command path, the write serializer, the read aligner, and the DFI-side of the CDC FIFOs inside pumice_dfi_layer.

Note: init_sequencer names its ports mc_clk / mc_rst_n internally, but the top level (pumice_top → pumice_core) ties these to aclk / aresetn.

## 26.3	Reset Sequencing

### 26.3.1	Cold Boot

- Both resets asserted (aresetn, dfi_rstn)

- Clocks stable

- dfi_rstn deasserted; aresetn deasserted (software programs the CSR timings/phases/policy via the register cpuif, or defaults are used)

- The init sequencer asserts dfi_init_start_o and waits on dfi_init_complete_i (the PHY runs its own DLL-lock / IO training)

- The init sequencer walks the JEDEC MRS / precharge / refresh sequence (see the Init Sequences chapter)

- On init_done_o, the controller begins servicing AXI traffic

### 26.3.2	Warm Reset (Clock-Gated)

The SoC can warm-reset the controller without losing DRAM content:

- SoC requests self-refresh entry (via powerdown_ctrl, optional)

- Controller acknowledges; DRAM is in self-refresh

- SoC asserts aresetn; clocks may be gated

- SoC deasserts aresetn; clocks resume

- Controller re-runs the init sequence

- Controller resumes normal operation

## 26.4	Clock Domain Crossing Summary

There is exactly one clock-domain crossing in the design: the aclk ↔ dfi_clk crossing inside pumice_dfi_layer (pumice_dfi_cdc.sv). It is built from asynchronous gaxi FIFOs only — command, write-data, and read-data each get their own async FIFO, plus the init_start / init_complete handshake. The register cpuif shares the aclk domain, so there is no CSR-clock CDC.

| Crossing | Mechanism | Latency |
| --- | --- | --- |
| aclk → dfi_clk | Async gaxi FIFOs (cmd / wrdata) in pumice_dfi_cdc | FIFO + N_FLOP_CROSS-flop sync |
| dfi_clk → aclk | Async gaxi FIFO (rddata) in pumice_dfi_cdc | FIFO + N_FLOP_CROSS-flop sync |

## 26.5	Reset and Power Considerations

DFI v2.1 requires dfi_init_start to be asserted by the controller as part of init; the init sequencer drives dfi_init_start_o out of reset (once it leaves S_RESET) and waits on the PHY’s dfi_init_complete_i before proceeding to the MRS / precharge / refresh walk. The init_start / init_complete handshake crosses the aclk ↔ dfi_clk boundary through the same CDC as the command and data streams.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Init Sequences

The cold-boot init sequences for DDR2 and LPDDR2 are implemented as a single FSM in rtl/fub/init_sequencer.sv (not separate step-package files). The FSM issues real MRS / precharge / refresh commands to the DRAM through the command scheduler while init_busy_o is high, and updates the mode-register shadow in lockstep so the live CL / CWL / BL decode tracks what was programmed. The memtype_i input selects the DDR2 or the LPDDR2 branch.

The DDR2 branch mirrors LiteDRAM’s get_ddr2_phy_init_sequence and is proven on the Nexys A7 board.

## 27.1	DDR2 Cold Init

Per JESD79-2F. FSM states from init_sequencer.sv; MR data rides the ROW request field (init_cmd_row_o), and the MR / EMR number is carried in the bank index (init_cmd_bank_o).

| Step | State | Command | MR data | Post-wait |
| --- | --- | --- | --- | --- |
| 1 | S_DFI_INIT | assert dfi_init_start, wait dfi_init_complete, then tINIT | — | t_init_wait |
| 2 | S_PREA1 | PRECHARGE all | — | t_rp_wait (tRP) |
| 3 | S_EMR2 | MRS EMR(2) | 0x0000 | t_mrd_wait (tMRD) |
| 4 | S_EMR3 | MRS EMR(3) | 0x0000 | t_mrd_wait |
| 5 | S_EMR1 | MRS EMR(1) | 0x0000 | t_mrd_wait |
| 6 | S_MR0_DLL | MRS MR0 + DLL reset | MR0.VAL \¦ 0x100 (reset default 0x0533 = BL8 / CL3 / tWR3 / DLL_RESET) | t_dll_wait (tDLLK) |
| 7 | S_PREA2 | PRECHARGE all | — | t_rp_wait |
| 8 | S_REF1 | AUTO REFRESH | — | t_rfc_wait (tRFC) |
| 9 | S_REF2 | AUTO REFRESH | — | t_rfc_wait |
| 10 | S_MR0 | MRS MR0 (DLL reset cleared) | MR0.VAL (reset default 0x0433) | t_mrd_wait |
| 11 | S_OCD_DEF | MRS EMR(1) + OCD default | 0x0380 | t_mrd_wait |
| 12 | S_OCD_EXIT | MRS EMR(1) + OCD exit | 0x0000 | t_mrd_wait |
| 13 | S_DONE | assert init_done | — | — |

Note the JEDEC extended-mode-register order: EMR(2), then EMR(3), then EMR(1), before MR0. The OCD-default state (S_OCD_DEF) leaves the MR1 shadow unchanged; S_OCD_EXIT restores it to the final 0x0000 value so the live decode is stable.

## 27.2	LPDDR2 Cold Init

Per JESD209-2F. LPDDR2 uses the Mode Register Write (MRW) chain. Because the MR address (MA) can reach MR63 / MR10 — beyond a 3-bit bank port — the sequencer packs {MA[5:0], OP[7:0]} into the ROW request field, and dfi_cmd_formatter.sv unpacks it (row[13:8] = MA, row[7:0] = OP) to build the bit-exact LPDDR2 CA-bus MRW word. Only MR1 / MR2 / MR3 update the CL / CWL / BL decode shadow; MR63 and MR10 are issued to the DRAM but not shadowed.

| Step | State | Command | OP data | Post-wait |
| --- | --- | --- | --- | --- |
| 1 | S_DFI_INIT | assert dfi_init_start, wait dfi_init_complete, then tINIT | — | t_init_wait |
| 2 | S_L_RESET | MRW(MR63) Reset | 0x00 | t_init_wait (tINIT4) |
| 3 | S_L_ZQ | MRW(MR10) ZQ Init Calibration | 0xFF | t_dll_wait (tZQINIT) |
| 4 | S_L_MR1 | MRW(MR1) BL8 / nWR3 | 0x23 | t_mrd_wait (tMRW) |
| 5 | S_L_MR2 | MRW(MR2) RL3 / WL1 | 0x01 | t_mrd_wait |
| 6 | S_L_MR3 | MRW(MR3) DS 40 ohm | 0x02 | t_mrd_wait |
| 7 | S_DONE | assert init_done | — | — |

The LPDDR2 branch reuses the same CSR waits as DDR2: t_init_wait covers both tINIT and tINIT4, t_dll_wait covers tZQINIT, and t_mrd_wait covers the post-MRW tMRW delay.

LPDDR2 is now fully functional in sim: reads and writes, bit-exact JESD209-2F CA encoding, and the full MR init chain above.

## 27.3	Inter-Command Waits (CSR-Backed)

The JEDEC inter-command delays were previously hardcoded; they are now driven by CSR fields (zero-extended to the 16-bit internal countdown). All counts are in MC (aclk) cycles.

| CSR field | Purpose | Reset value |
| --- | --- | --- |
| INIT_TIMING0.t_init_wait | CKE / tINIT settle | 512 |
| INIT_TIMING0.t_dll_wait | DLL lock (tDLLK) | 256 |
| INIT_TIMING1.t_mrd_wait | post mode-register (tMRD) | 8 |
| INIT_TIMING1.t_rp_wait | post precharge (tRP) | 8 |
| INIT_TIMING1.t_rfc_wait | post auto-refresh (tRFC) | 16 |

Init is a one-time event, so generous margins are fine — the reset defaults comfortably cover the JEDEC minimums at the on-board clock rate. Simulation programs shorter INIT_TIMING* values to keep test runtimes practical; there is no separate scaling parameter — the shorter waits are just smaller CSR values.

## 27.4	Power-State Transitions

Self-refresh and deep-power-down transitions are handled by the optional powerdown_ctrl.sv power-state FSM, not by the init sequencer. On an idle threshold it requests a low-power state and drops CKE (dfi_cke_o); it wakes on any new controller activity. The two supported entry modes are:

- Precharge Power Down (PDE) — CKE low, banks may stay active.

- Self Refresh (SR) — CKE low plus the SRE command; the DRAM self-refreshes internally and the controller stops issuing REFs.

Self-refresh entry requires all banks precharged first (the scheduler enforces this precondition before granting). Self-refresh exit timing (tXSDLL for DDR2 / tXSR for LPDDR2) is enforced by the scheduler; the power-state FSM itself only drops and restores CKE.

LPDDR2 Deep Power Down (DPD) is supported as a planned power-state entry (CKE low after precharge-all). DPD exit loses DRAM content, so the full LPDDR2 cold init sequence above must run again.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	CSR Register Map

The controller’s configuration and observation registers are defined in rtl/macro/pumice_csr.rdl and generated with PeakRDL into a passthrough-cpuif register block (bin/peakrdl_generate.py). The DV mirror is dv/tbclasses/pumice_regmap.py; software addresses every field by name through that generated map — hardcoded offsets are forbidden. This chapter is the authoritative narrative for that RDL; every register, offset, and reset value below is reconciled against it.

The map occupies a 4 KB space (12-bit address, CSR_ADDR_W = 12). All registers are 32 bits wide, little-endian, 4-byte aligned. The register bus is the PeakRDL passthrough cpuif (s_cpuif_*), not a hand-written APB slave — see §4.3. Config fields are consumed hw = r (hwif_out.*) and drive the core by name; status/observation fields are hw = w (hwif_in.*).

## 28.1	Control and Status (0x000 – 0x00F)

### 28.1.1	CTRL (0x000, R/W)

| Bit | Field | Reset | Description |
| --- | --- | --- | --- |
| 0 | init_start | 0 | Write 1 to start init (self-modifying strobe) |
| 1 | init_force_restart | 0 | Write 1 to force re-init even mid-sequence |
| 3:2 | RSVD_3_2 | 0 | Reserved |
| 4 | pwr_req_low_power | 0 | Request power-down state |
| 5 | pwr_req_dpd | 0 | Request DPD (LPDDR2 only) |
| 6 | pwr_req_active | 0 | Request return to ACTIVE |
| 7 | pwr_req_self_refresh | 0 | Request self-refresh |
| 30:8 | RSVD_30_8 | 0 | Reserved |
| 31 | soft_reset | 0 | Write 1 to assert internal soft reset (self-clearing) |

### 28.1.2	STATUS (0x004, R only)

| Bit | Field | Description |
| --- | --- | --- |
| 0 | init_done | 1 when init complete |
| 1 | init_error | 1 on init failure |
| 7:4 | power_state | Current power-state FSM state (encoded) |
| 8 | pasr_active | LPDDR2: PASR mask is non-zero |
| 23:16 | init_step_dbg | Current init step number (for bring-up) |
| 31 | version_match | 1 when build matches expected version |

### 28.1.3	STATUS_HISTORY (0x008, R only)

Last 8 power-state transitions, 4 bits each. Most recent in [3:0]; new transitions push the oldest off. Useful for bring-up debugging of power-state oscillation.

## 28.2	Timing Parameters (0x010 – 0x01F)

Packed timing parameters in MC (aclk) cycles. All fields are sw = rw, hw = r and drive the core scheduler / DFI layer.

### 28.2.1	TIMINGS_RC_RCD_RP_RAS (0x010, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | tRC | 60 | tRC in cycles |
| 15:8 | tRCD | 15 | tRCD in cycles |
| 23:16 | tRP | 15 | tRP in cycles |
| 31:24 | tRAS | 40 | tRAS in cycles |

### 28.2.2	TIMINGS_RFC_REFI (0x014, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 15:0 | tRFC | 16 | tRFC (or tRFCab) in MC cycles — mission-mode REF recovery, consumed by the arbiter |
| 31:16 | tREFI | 1950 | tREFI in cycles |

### 28.2.3	TIMINGS_RRD_FAW_WTR_CCD (0x018, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | tRRD | 6 | tRRD |
| 15:8 | tFAW | 35 | tFAW |
| 23:16 | tWTR | 4 | tWTR |
| 31:24 | tCCD | 4 | tCCD |

### 28.2.4	TIMINGS_CL_CWL_WR (0x01C, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | CL | 6 | CAS latency |
| 15:8 | CWL | 4 | CAS write latency |
| 23:16 | tWR | 15 | Write recovery |
| 31:24 | tRFCpb | 70 | LPDDR2 per-bank tRFC |

## 28.3	Mode Register Values (0x020 – 0x02F)

### 28.3.1	MR0, MR1, MR2, MR3 (0x020, 0x024, 0x028, 0x02C, R/W)

Low 16 bits (VAL) contain the MR value; upper 16 bits reserved. These hold override MR values; the default init walk uses the JEDEC-standard values built into init_sequencer.sv (§6.2). Reset = 0.

## 28.4	LPDDR2-Specific (0x030 – 0x03F)

### 28.4.1	PASR_BANK_MASK_RANK0 (0x030, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | pasr_banks | 0 | LPDDR2 PASR per-bank mask for rank 0 (MR16); bit N = 1 masks bank N |

### 28.4.2	PASR_SEG_MASK_RANK0 (0x034, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | pasr_segs | 0 | LPDDR2 PASR segment mask for rank 0 |

### 28.4.3	TEMP_DERATE_RANK0 (0x038, R only)

| Bits | Field | Description |
| --- | --- | --- |
| 1:0 | temp_class | From rank 0’s MR4: 00 = nominal, 01 = 2x refresh, 10 = 4x refresh |

The current RDL defines the single-rank instances only (NUM_BANKS = 8, rank 0). Multi-rank builds would extend this block; the map as generated today does not allocate per-rank PASR/temperature registers beyond rank 0.

## 28.5	Scheduler / Refresh / Page / Addr / Init Tuning (0x040 – 0x05F)

### 28.5.1	SCHED_TUNING (0x040, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 3:0 | lookahead_active | 0 | Active lookahead window (0..LOOKAHEAD_DEPTH_MAX). 0 disables lookahead. |
| 4 | force_inorder | 0 | 1 = force first-ready FIFO (disable row-hit reordering). |
| 5 | happy_enable | 1 | 1 = HAPPY predictor active (only meaningful if synthesized). |
| 15:8 | age_max_runtime | 0 | Runtime AGE_MAX override (0 = use build-time default). |
| 23:16 | txn_queue_high_water | 0 | Backpressure-assertion threshold for the txn queue. |
| 27:24 | lookahead_max_obs | — | (R only, hw = w) Echo of build-time LOOKAHEAD_DEPTH_MAX. |

### 28.5.2	PAGE_PRED_TUNING (0x044, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 15:0 | warmup_cycles | 1024 | Warmup cycles before predictor is trusted |
| 23:16 | hysteresis | 2 | Saturating-counter hysteresis |

### 28.5.3	REFRESH_TUNING (0x048, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 1:0 | refpb_policy_or | 0 | 00 = build-time; 01 = RR; 10 = OLDEST_FIRST; 11 = DARP. |
| 3:2 | page_policy_or | 0 | 00 = build-time; 01 = OPEN; 10 = CLOSE; 11 = HAPPY_HYBRID. |
| 7:4 | refresh_defer_active | 1 | Active refresh deferral count (1..REFRESH_DEFER_MAX). 1 = no batching. |
| 31:16 | zqcs_freq_hz | 1 | Periodic ZQCS interval in Hz. 0 disables periodic ZQCS. |

### 28.5.4	ADDR_MAP (0x04C, R/W)

Replaces the retired ADDR_MAP_TUNING register. The AXI-to-DRAM mapping is driven by a single knob rather than a scheme selector — see §5.4 and §3.1, and rtl/fub/addr_mapper.sv.

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 4:0 | bank_lsb | 0x0A | Bank-field LSB in the word address. Default 10 = COL_WIDTH = ROW_MAJOR placement. RTL clamps to [0, COL_WIDTH]. |
| 8 | hash_en | 0 | Enable bank XOR-hash: bank ^= fold(row) ^ hash_seed. |
| 23:16 | hash_seed | 0 | XOR-hash seed (seed[BW-1:0]). |

The register reset value is 0x0000000A (bank_lsb = 10, hash off). Setting bank_lsb = COL_WIDTH gives ROW_MAJOR; bank_lsb = log2(cols/burst) gives maximum bank interleave with burst locality preserved; hash_en folds an XOR-hash on top of any placement (the old XOR_HASH scheme). There is no separate scheme-selector field.

### 28.5.5	INIT_TUNING (0x050, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 3:0 | zq_retries | 3 | ZQ calibration retry count before raising init_error. Range 1–8. |
| 15:8 | init_timeout_ms | 10 | Per-step init timeout in ms. Range 1–255. |

### 28.5.6	INIT_TIMING0 (0x058, R/W)

JEDEC init-sequence waits (MC cycles), consumed by init_sequencer.sv. Previously hardcoded.

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 15:0 | t_init_wait | 512 | CKE / tINIT settle |
| 31:16 | t_dll_wait | 256 | DLL lock (tDLLK) |

### 28.5.7	INIT_TIMING1 (0x05C, R/W)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | t_mrd_wait | 8 | tMRD (post mode-reg) |
| 15:8 | t_rp_wait | 8 | tRP (post precharge) |
| 23:16 | t_rfc_wait | 16 | tRFC (post refresh) |

### 28.5.8	TIMINGS_RTP_RTW (0x054, R/W)

Read-to-precharge and read-to-write turn-around. Previously tRTP was hardcoded (8’d4) and tRTW was tied to it; both are now independent configs.

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | tRTP | 4 | Read to precharge |
| 15:8 | tRTW | 6 | Read to write |

## 28.6	DFI / PHY Timing (0x060 – 0x067)

### 28.6.1	DFI_PHASE (0x060, R/W)

Which DFI sub-phase carries the READ vs WRITE command, to match the PHY’s rdphase / wrphase contract (see §4.2). Fields are sliced to clog2(DFI_RATE) bits downstream; upper bits are ignored when DFI_RATE is narrower than the field. Defaults 0/0 preserve the legacy all-on-phase-0 behavior.

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 2:0 | rd_phase | 0 | READ command DFI sub-phase |
| 6:4 | wr_phase | 0 | WRITE command DFI sub-phase |

### 28.6.2	PHY_TIMING (0x064, R/W)

PHY / DFI data timing plus memory-type selection. All fields hw = r so they drive the controller core.

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | t_phy_wrlat | 0 | WRITE command -> dfi_wrdata_en (Nexys A7 bring-up tuple programs 1) |
| 15:8 | t_rddata_en | 6 | RD command -> dfi_rddata_en window |
| 16 | memtype | 0 | 0 = DDR2, 1 = LPDDR2 (drives the whole core memtype_e path) |
| 23:20 | refresh_burst | 1 | REFs drained per request (1..8) |

PHY_TIMING.memtype is the runtime memory-type select. It is decoded into the memtype_e enum at the top and fanned out to the addr/scheduler/DFI/init logic; geometry (DFI_RATE, DRAM_BEAT_WIDTH, NUM_BANKS, ROW/COL width, BL) remains build-time — see §5.1.

## 28.7	Per-Bank Observation (0x080 – 0x0DF)

Single-rank (rank 0), NUM_BANKS = 8. Each is a regfile of 8 word-wide registers striding by 4 bytes.

### 28.7.1	OBS_ROW_HIT[0..7] (0x080 + N×4, R/W with clear-on-read)

Rolling row-hit count per bank. VAL[31:0], onread = rclr (reset on read or soft_reset).

### 28.7.2	OBS_REF_LATENCY[0..7] (0x0C0 + N×4, R only)

Average refresh-blocking cycles per bank. VAL[31:0], hw = w.

## 28.8	System Observation (0x100 – 0x1FF)

All hw = w, sw = r.

| Offset | Register | Description |
| --- | --- | --- |
| 0x100 | OBS_TXN_QUEUE_DEPTH_MAX | Max queue depth observed |
| 0x104 | OBS_TXN_QUEUE_DEPTH_AVG | Time-averaged depth |
| 0x108 | OBS_REFRESH_PENDING_MAX | Max refresh_pending value observed |
| 0x10C | OBS_REFRESH_DEFER_HIST_0 | Refresh deferral histogram bin 0 |
| 0x110 | OBS_REFRESH_DEFER_HIST_1 | Histogram bin 1 |
| 0x114 | OBS_REFRESH_DEFER_HIST_2 | Histogram bin 2 |
| 0x118 | OBS_REFRESH_DEFER_HIST_3 | Histogram bin 3 |
| 0x120 | OBS_PAGE_PRED_ACCURACY | HAPPY mode: rolling prediction accuracy (%) |
| 0x130 | OBS_AXI_R_LATENCY_AVG | Avg AXI read latency in cycles |
| 0x134 | OBS_AXI_R_LATENCY_P99 | 99th-percentile read latency |
| 0x138 | OBS_AXI_W_LATENCY_AVG | Avg AXI write latency |

### 28.8.1	OBS_WORDS[0..8] (0x1C0 + N×4, R only)

Nine 32-bit read-only words carrying the packed obs_* signal harvest from the FUB internals (see docs/csr_obs_layout.md). VAL[31:0], hw = w.

## 28.9	Module Identification (0xFF0 – 0xFFC)

### 28.9.1	ID (0xFF0, R only, reset = 0xD2020001)

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| 7:0 | version | 0x01 | Build version |
| 15:8 | memtype | 0x00 | 0 = DDR2, 1 = LPDDR2 |
| 23:16 | n_phases | 0x02 | Gear ratio (1, 2, or 4) |
| 31:24 | module_id | 0xD2 | Fixed 0xD2 |

### 28.9.2	BUILD (0xFF4, R only)

Build hash word. VAL[31:0], reset = 0.

## 28.10	Notes on This Revision

- ADDR_MAP_TUNING (with scheme_or / synth_mask_obs) has been retired and replaced by ADDR_MAP at 0x04C. No scheme-selector field exists in RTL.

- TIMINGS_RTP_RTW (0x054), INIT_TIMING0 (0x058), INIT_TIMING1 (0x05C), DFI_PHASE (0x060), and PHY_TIMING (0x064) expose knobs that were formerly hardcoded in the FUBs; they are now runtime CSRs.

- PHY_TIMING.memtype and PHY_TIMING.refresh_burst are new runtime fields.

- The observation block is single-rank as generated; the multi-rank per-rank layout described in earlier revisions is not present in the current RDL.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Verification Strategy

A layered verification approach with the existing DFI BFM as the primary reference.

## 29.1	Layered Approach

### 29.1.1	Layer 1: Module-Level Unit Tests

Each live FUB gets its own cocotb unit test (see dv/tests/fub/):

| Module | Test focus |
| --- | --- |
| pumice_wr_intake | AW-meta + wr-data FIFO handshakes; backpressure; burst splitting into fixed-BL commands |
| pumice_rd_intake | AR channel handshakes; address mapping; snarf (read-your-write) probe |
| pumice_wr_data_cam | WR CAM fill / commit-drain / snarf movers; r_fdone fill-complete gating |
| pumice_rd_cmd_cam | RD CAM return-fill / drain movers; oldest-first, data-ready gating |
| addr_mapper | Address translation across ADDR_MAP.bank_lsb settings (ROW_MAJOR .. interleave) and hash on/off |
| pumice_cmd_arbiter | Single-pick priority order; inline open-page decision; refresh prioritization |
| pumice_bank_timers / bank_timer | Per-(rank,bank) preset/decrement timers; combinational safe_act/rd/wr/pre readiness |
| global_timers | tFAW / tRRD / tWTR / tRTW / tCCD against JESD79-2 / JESD209-2 truth tables |
| refresh_ctrl | tREFI counter; refresh-deferral; refresh request/ack handshake |
| init_sequencer | Step-by-step DDR2 and LPDDR2 JEDEC MR init sequences |
| mode_register | CL / CWL / BL / AL decode (DDR2 and LPDDR2 MR2 RL/WL enum) |
| dfi_cmd_formatter | Round-trip table tests against JEDEC encoding; LPDDR2 CA-bus branch |
| dfi_signal_pack | DW-to-phase split; idle-phase handling |
| pumice_dfi_cdc | The single async-FIFO clock-domain crossing; bubble-free cmd/wr/rd flow |
| pumice_dfi_wr_serializer | CWL alignment; write-data serialization onto DFI phases |
| pumice_dfi_rd_aligner | CL alignment; read-data assembly; back-pressure |

The four macro wrappers and the integration levels are exercised at Layer 2/3: pumice_axi4_ifc, pumice_mem_cmd_scheduler, pumice_dfi_layer, pumice_core, and pumice_top (the PeakRDL pumice_csr register block is verified with the top).

### 29.1.2	Layer 2: Subsystem Tests

Multi-module integration tests, framed around the three macro layers (see dv/tests/macro/):

- AXI interface (pumice_axi4_ifc = wr/rd intakes + the two CAMs): full AXI transaction flow with diverse traffic, including read-your-write snarf forwarding

- Command scheduler (pumice_mem_cmd_scheduler = cmd arbiter + bank/global timers + refresh + init + mode register): scheduler decisions on representative queue contents, inline open-page reordering, refresh interleave

- DFI layer (pumice_dfi_layer = single CDC + cmd path + wr serializer + rd aligner): command/data flow across the clock-domain crossing with the DFI BFM slave

### 29.1.3	Layer 3: End-to-End with DFI BFM Slave

The full controller drives the DFI BFM slave through our existing co-sim harness (tests/sim/dfi/test_litedram/... patterns in the DV repo).

- AXI4 stimulus generators issue representative traffic

- DFI BFM slave captures the controller’s command stream

- Assertions check: command sequencing matches AXI, timing constraints are honored, data integrity round-trips through the slave’s MemoryModel

### 29.1.4	Layer 4: External-Reference Cross-Validation

For DDR2: cross-validate against LiteDRAM’s DDR2 model.

- Generate LiteDRAM controller alongside ours

- Both drive the same DFI BFM slave with the same AXI stimulus

- Compare command streams; differences flag potential design issues

This is a real, proven path. A LiteDRAM DDR2 memtest passes on the Nexys A7 board (128 MiB), which confirmed the board, PHY, pin-out, and DRAM are good. pumice itself is now board-proven on the same Nexys A7: DDR2 reads and writes are clean (0 mismatches across the characterization traffic). Both memtypes pass the full sim suite.

LPDDR2 has no LiteDRAM analog, so cross-validation for LPDDR2 is limited to the BFM master to BFM slave path. LPDDR2 is nonetheless fully functional in sim (reads and writes, bit-exact JESD209-2F CA encoding, full MR init) and passes the sim suite.

## 29.2	Characterization Sweep

Post-functional verification, the sweep runs each characterization parameter through its choices on the benchmark suite from §5.2:

### 29.2.1	Sweep Matrix

| Parameter | Sweep values |
| --- | --- |
| LOOKAHEAD_DEPTH | 0, 1, 2, 4 |
| PAGE_POLICY | OPEN, CLOSE, HAPPY_HYBRID |
| REFPB_POLICY | ROUND_ROBIN, OLDEST_FIRST, DARP |
| REFRESH_DEFER_MAX | 1, 2, 4, 8 |
| TXN_QUEUE_DEPTH | 8, 16, 32 |
| DFI_RATE | (build-determined; usually 4) |
| ADDR_MAP.bank_lsb | ROW_MAJOR (== COL_WIDTH), interleave, hash on/off |

The full cross-product would be 4 × 3 × 3 × 4 × 3 = 432 builds; in practice we sweep one parameter at a time holding others at recommended defaults, which reduces to ~17 builds per memtype. The exception is the LOOKAHEAD_DEPTH × PAGE_POLICY pair — these interact strongly (the fallback policy only matters when lookahead is inconclusive), so this pair is swept as a full 4 × 3 = 12-point grid.

### 29.2.2	Sweep Outputs

Each (parameter, value) run produces a JSON report:

{
  "memtype": "LPDDR2",
  "page_policy": "HAPPY_HYBRID",
  "refpb_policy": "DARP",
  "workload": "mobile-mixed",
  "metrics": {
    "read_bw_pct": 73.2,
    "write_bw_pct": 68.1,
    "read_latency_avg_ns": 142,
    "read_latency_p99_ns": 198,
    "refresh_block_pct": 4.3,
    "row_hit_rate_avg": 0.81,
    "page_predictor_accuracy": 0.91
  }
}

These feed into a sweep visualization tool that selects defaults from data.

### 29.2.3	Out-of-Order Scheduler Trade-Offs

The centralized FR-FCFS scheduler proposed in §1 (Differentiating Features) is the most architecturally aggressive choice in this controller and warrants explicit characterization beyond just the sweep matrix above. Three concrete concerns:

- Timing-critical path scales with TXN_QUEUE_DEPTH. Scanning the full queue every cycle for the best candidate is a comparator pyramid: priority resolves over (ready-flag, row-hit-flag, age) for every entry simultaneously. At TXN_QUEUE_DEPTH = 16 this is well within an embedded-SoC clock budget; at 32 it stays comfortable; at 64 it becomes the critical path on the controller side. The characterization sweep should report scheduler combinational depth at synthesis time alongside the performance numbers — the right TXN_QUEUE_DEPTH is the largest value that doesn’t push the scheduler off the clock budget.

- Area cost is super-linear in queue depth. Each queue entry needs its full metadata replicated (bank, row, col, ID, age, row-hit cache). At depth 16, ~2K gates; at depth 64, ~10K gates — comparable to the rest of the scheduler combined. The HAPPY predictor’s gate cost is a separate line item.

- Verification cost is significantly higher than FIFO. OoO issue means assertions like “request N landed at cycle X” can’t be used. The DV plan relies on protocol-level scoreboarding: the BFM master records arrival order per ID, the BFM slave records issue order, and the scoreboard verifies the AXI4 per-ID ordering invariant is preserved. Within-ID ordering violations are a higher-severity bug than performance regressions and should be caught at Layer 1 unit tests, not deferred to integration.

- Fairness tail matters under adversarial workloads. With AGE_MAX as the only anti-starvation cap, pathological patterns can drive 99th-percentile latency well above mean. The random-narrow and multi-master workloads in the benchmark suite intentionally exercise this; the mobile-mixed workload simulates the common case. The recommended default for AGE_MAX should be picked based on tail-latency targets, not throughput targets.

These concerns are unique to the OoO scheduler design choice — a simpler per-bank FIFO controller (the typical open-source pattern) would not face them, at the cost of leaving row-hit reordering performance on the table.

## 29.3	Verification Hooks

Internal observation outputs are exposed via CSR (see §6.3) for in-system characterization. Additionally, the controller emits internal “event” signals that can be tied to waveform-dump triggers:

- ev_init_done

- ev_refresh_issued

- ev_page_conflict_detected

- ev_queue_full

- ev_refresh_pending_critical

Specific events can be wired to a test-bench-controlled waveform-capture trigger during bring-up.

## 29.4	Coverage Targets

Functional coverage (collected during regression):

- All FSM states reached in each module that has one (the bank timers and CAMs are FSM-free; their timer/counter states are covered instead)

- All page policies exercised

- All refresh policies exercised

- All scheduler priority levels exercised

- All AXI burst types exercised (if enabled)

- All ADDR_MAP.bank_lsb settings (ROW_MAJOR .. interleave) exercised, with hash on and off

Code coverage targets: 100% line / branch coverage for all modules.

Assertion coverage: all design assertions (registered protocol checks) hit during regression.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Open Issues and TODOs

Items where design choices were made provisionally and should be reviewed before the HAS is promoted to formal status.

## 30.1	Open Issues

### 30.1.1	1. AXI4 Narrow-Burst Handling Confirmation

AXI4 allows narrow transfers (data width less than bus width). The current plan is to handle them via byte-mask through wrdata_mask. Confirm the wstrb-to-wrdata-mask wiring honors the AXI partial-write semantics correctly, particularly for unaligned bursts.

### 30.1.2	2. AXI4 Read-Modify-Write Strobe Handling

Partial writes in DDR2 / LPDDR2 use wrdata_mask to mask byte lanes; this is straightforward. Confirm that per-byte masking is wired all the way through the write path (pumice_wr_intake -> pumice_wr_data_cam -> pumice_dfi_wr_serializer) and that pumice_wr_intake doesn’t widen narrow writes implicitly.

### 30.1.3	3. CSR Address Space Allocation

The config bus is a PeakRDL passthrough cpuif on aclk (no APB slave, no separate apb_pclk). CSR_ADDR_W = 12 provides a 4 KB register region. Confirm the SoC’s address-map allocation has at least 4 KB available for the cpuif window, and that there’s no conflict with adjacent peripherals.

### 30.1.4	4. Per-Bank Refresh Book-Keeping Cost

Keeping last_ref_age per bank costs one counter per bank. For 8 banks this is 8 small counters. Confirm the synthesis area impact is acceptable; we expect ~200 LUTs for the per-bank refresh tracking on a typical FPGA.

### 30.1.5	5. HAPPY Predictor Hash Function

Ghasempour 2015 suggests bank-XOR-low-row-bits as the hash function. Confirm with our typical workloads. If the workloads have systematic bank-row correlations, a different hash may be needed.

### 30.1.6	6. Self-Refresh Exit Latency

JESD79-2 requires tXSNR (~200 ns) before any command after SR exit. Confirm the optional powerdown_ctrl enforces this correctly and that the scheduler is gated for the full duration.

### 30.1.7	7. Init Sequencer Retry on Error

If ZQ fails, do we retry up to 3 times then halt, or just halt? Current plan: 3 retries (parameter INIT_ZQ_RETRIES) then raise init_error. Confirm with the system architect — some systems prefer fail-fast. (Note: DDR2 ZQ init differs from LPDDR2 — init_sequencer handles both JEDEC sequences.)

### 30.1.8	8. CSR Write Atomicity

Multi-register parameter changes (e.g., updating both MR0 and a related timing) need a “config quiet period” to take effect atomically. Today the SoC owns the drain: software is expected to quiesce AXI traffic before reprogramming timing/mode CSRs. A dedicated config_apply strobe (hardware-enforced quiet period) is not implemented and is a possible future addition. Document the software drain protocol regardless.

### 30.1.9	9. Burst Splitting at Row Boundaries

AXI4 allows up to 256 beats per burst; a DRAM row contains 2^COL_WIDTH columns. Burst splitting into fixed-BL commands happens in pumice_wr_intake / pumice_rd_intake. Confirm the split logic correctly tracks partial-burst completion for ID-based ordering.

### 30.1.10	10. Multi-Rank

Multi-rank scaffolding now exists: NUM_RANKS is a real build parameter (default 1), the DFI dfi_cs_n_o / dfi_odt_o buses are per-rank (width NUM_RANKS * DFI_RATE), and addr_mapper carries a rank field (rank_o) at the top of the address stack. The command path and bank timers are stamped per (rank, bank). Remaining gap: per-rank refresh coordination, PASR (partial-array self-refresh), and per-rank observation registers are not yet implemented. Confirm the multi-rank data path with a NUM_RANKS > 1 build before relying on it.

### 30.1.11	11. ECC

Out of scope for this controller. ECC is expected to be handled at the SoC level or in a sideband wrapper, not the controller itself.

### 30.1.12	12. DFI Training Sub-Interface

Not driven in v1; the assumption is that the PHY handles its own training or training is not required for this DDR2 / LPDDR2 target. Confirm with the PHY vendor before silicon tape-out.

### 30.1.13	13. AXI Quality of Service (QoS)

Currently the awqos / arqos fields are forwarded to the scheduler as priority hints, but the scheduler does not implement explicit QoS classes. Decide whether to add proper QoS support (multi-class scheduler) or leave QoS as a future enhancement.

### 30.1.14	14. Multi-Master AXI

Currently single AXI port. Decide whether to add a multi-port AXI crossbar internally (simplifies SoC integration but adds area) or leave as a single-port with the SoC responsible for AXI arbitration upstream.

### 30.1.15	15. Observation Counter Overflow

Observation counters (row-hit, queue depth max, etc.) are 32-bit. At high traffic rates they could wrap. Decide on overflow handling: saturate, clear-on-read, or rely on SoC reading frequently.

### 30.1.16	16. Retired-Architecture Modules Still on Disk (OLD/)

The pre-rearchitecture modules still live under rtl/fub/OLD/, rtl/macro/OLD/, and rtl/top/OLD/ (e.g. scheduler.sv, wr_cmd_cam.sv, rd_cmd_cam.sv, xbank_timers.sv, axi_intake.sv, the *_macro.sv blocks, pumice_csr_slave.sv). They are referenced only by retired sentinel tests in dv/tests/ (test_scheduler.py, test_wr_cmd_cam.py, test_xbank_timers.py, test_axi_intake.py, and the *_macro tests). Remove the OLD/ trees and their sentinel tests once the new architecture is fully signed off, so the live module set is the only thing that builds.

### 30.1.17	17. Open-Page Read-Fetches-Zero Under Gapped Reads

A known scheduler/CAM interaction: under open-page policy with gapped read traffic, a read can be fetched before its data is valid (returns zero) in a narrow timing window (reproduces at PUMICE_SEED=2, the burst-pause / hit-miss-oscillation pattern). Root-cause and fix the read-fetch gating in the open-page path before promoting the HAS to formal status. The r_fdone fill-complete gating in the CAMs is the relevant mechanism.

## 30.2	TODOs Before v0.2

- Add bit-level pinout tables for the AXI, DFI, and cpuif (register) interfaces

- Add timing diagrams for the WR / RD command issue, including CWL / CL alignment

- Add sequence diagrams for init_sequencer (DDR2 and LPDDR2 MR init) and, where present, the optional powerdown_ctrl; document the FSM-free bank_timer countdown timers rather than an FSM diagram

- Add a draft of the Verilog package skeletons for ddr2_init_steps_pkg.sv and lpddr2_init_steps_pkg.sv

- Cross-reference each section to the corresponding pre-aspec.md bullet

- Add quantitative area / power estimates per module (synthesis pass needed)

- Add waveform examples for the canonical init sequences

## 30.3	Feature Roadmap — planned advanced modes (ordered)

The full catalog of planned, config-bit-selectable advanced modes lives in docs/design-requirements.md (“Advanced modes — selectable scheduling / paging / refresh”); the family-level split (commodity-legal here vs model-only parked for DDR3/DDR4) is in projects/components/memory-controllers/ADVANCED_MODES_ROADMAP.md. Entry gate (TASKS.md TASK-FEATURES): board reads validated at the bring-up tuple, refresh-collision fix re-soaked on silicon.

Serial pre-silicon implementation order (each step OFF-by-default with its own red-to-green model test):

- Foundation — SCHED_POLICY / PAGE_POLICY_CFG / REFRESH_MODE mode-select CSRs + *_STATS telemetry + PHY capability straps (no behavior change; defaults bit-identical).

- Scheduling (Axis 1) — in_order -> fr_fcfs (confirm current) -> age_threshold -> most/fewest_pending -> ACCESS_PREF -> write-batching -> QoS (AxQOS-aware pick, QOS_EN).

- Paging (Axis 2) — static_open/close (confirm) -> fixed_open -> adapt_time -> rbl_static -> rbl_dyn -> adapt_access.

- Refresh (Axis 3, commodity) — JEDEC pull-in/postpone sweep -> refpb_rr.

None of the mode-select CSRs above exist in the RDL yet; open issue 13 (QoS) is subsumed by step 2 of this order.