RTL Design Sherpa

RAPIDS Beats Hardware Architecture Specification 0.6

July 2, 2026

Table of Contents

1 Rapids Beats Has Index	27

2 Document Information	27

2.1 Purpose	27

2.2 Scope	27

2.3 Audience	28

2.4 Document Conventions	28

2.4.1 Requirement Levels	28

2.4.2 Signal Directions	28

2.4.3 Timing Diagrams	28

2.5 References	29

3 Revision History	29

3.1 Change Summary	30

3.1.1 Version 0.8 (2026-07-13)	30

3.1.2 Version 0.7 (2026-07-13)	30

3.1.3 Version 0.6 (2026-07-02)	30

3.1.4 Version 1.0 (2026-01-17)	31

4 Acronyms and Definitions	31

4.1 Acronyms	31

4.2 Definitions	32

4.3 Signal Naming Conventions	33

5 Product Overview	33

5.1 Introduction	33

5.2 System Context	33

5.3 Target Applications	34

5.4 Architecture Highlights	34

5.4.1 Phase 1 “Beats” Simplifications	34

5.4.2 Key Capabilities	35

6 Key Features	35

6.1 Feature Summary	35

6.1.1 Multi-Channel Architecture	35

6.1.2 Descriptor-Based Control	35

6.1.3 High-Performance Data Paths	36

6.1.4 Monitoring and Debug	36

6.2 Feature Details	36

6.2.1 Descriptor Processing	36

6.2.2 SRAM Buffering	38

6.2.3 AXI Burst Optimization	38

6.2.4 Error Handling	38

7 System Context	39

7.1 Integration Overview	39

7.2 System Block Diagram	39

7.3 Interface Connections	41

7.3.1 Memory Interface	41

7.3.2 Network Interface	41

7.3.3 Configuration Interface	41

7.4 Clock and Reset	42

7.4.1 Clock Domains	42

7.4.2 Reset Requirements	42

7.5 Power Considerations	42

7.5.1 Clock Gating	42

7.5.2 Power States	43

8 Block Diagram	43

8.1 Top-Level Architecture	43

8.2 Top-Level Integration (rapids_beats_top)	44

8.3 Component Summary	46

8.3.1 Scheduler Group Array	46

8.3.2 Sink Data Path	46

8.3.3 Source Data Path	46

8.4 Hierarchy	47

8.5 Internal Buses	47

9 Data Flow	48

9.1 Overview	48

9.2 Sink Path Data Flow	48

9.2.1 Sink Path Timing	48

9.3 Source Path Data Flow	49

9.3.1 Source Path Timing	50

9.4 Concurrent Operation	51

9.4.1 Deadlock Prevention	51

10 Channel Architecture	51

10.1 Multi-Channel Design	51

10.2 Channel Resource Allocation	51

10.3 Per-Channel Resources	52

10.4 Shared Resources	52

10.5 Channel Scheduling	53

10.6 Channel State Machine	54

10.6.1 Channel State Timing	55

10.7 Channel Isolation	56

10.7.1 Per-Channel Error Handling	56

11 Interface Summary	57

11.1 External Interface Overview	57

11.2 Interface Port Summary	59

11.3 Clock and Reset	59

11.4 Interface Feature Matrix	59

11.5 Signal Count Summary	60

12 AXI4 Interface Specification	60

12.1 Overview	60

12.2 Descriptor AXI Master	61

12.2.1 Purpose	61

12.2.2 Configuration	61

12.2.3 Signal List	61

12.2.4 Timing Diagram	62

12.3 Sink AXI Master (Write)	63

12.3.1 Purpose	63

12.3.2 Configuration	63

12.3.3 Signal List	63

12.3.4 Timing Diagram	64

12.4 Source AXI Master (Read)	65

12.4.1 Purpose	65

12.4.2 Configuration	65

12.4.3 Signal List	66

12.4.4 Timing Diagram	66

12.5 AXI Protocol Constraints	67

12.5.1 Address Alignment	67

12.5.2 4KB Boundary Handling	67

12.5.3 Response Handling	68

13 AXIS Interface Specification	68

13.1 Overview	68

13.2 Sink AXIS Slave	68

13.2.1 Purpose	68

13.2.2 Configuration	69

13.2.3 Signal List	69

13.2.4 Timing Diagram	70

13.2.5 Backpressure Behavior	70

13.3 Source AXIS Master	70

13.3.1 Purpose	70

13.3.2 Configuration	71

13.3.3 Signal List	71

13.3.4 Timing Diagram	71

13.3.5 Data Availability	72

13.4 AXIS Protocol Notes	72

13.4.1 Handshake Rules	72

13.4.2 TLAST Semantics	73

13.4.3 TKEEP Usage	73

13.4.4 Channel Multiplexing via TID	73

14 APB Configuration Interface	74

14.1 Overview	74

14.2 APB Slave and Address Routing	74

14.2.1 APB Slave Signal List	74

14.2.2 Descriptor Kick-Off	75

14.2.3 Timing Diagram	75

14.3 Status Interface	76

14.3.1 Per-Channel Status Signals	76

14.3.2 Channel State Encoding	76

14.4 Configuration Registers	76

14.4.1 Address Range Validation	76

14.4.2 Configuration Registers	77

14.5 Programming Sequence	77

14.5.1 Initialization	77

14.5.2 Descriptor Kick-Off	78

15 Monitor Bus Interface	78

15.1 Overview	78

15.2 MonBus Packet Format	78

15.2.1 64-bit Packet Structure	78

15.2.2 Signal List	79

15.3 Packet Types	79

15.4 Agent IDs	79

15.5 Event Codes	80

15.5.1 Scheduler Events (Agent 0x30-0x37)	80

15.5.2 Descriptor Engine Events (Agent 0x10-0x17)	80

15.6 Timing Diagram	80

15.7 Event Data Encoding	81

15.7.1 Completion Event Data	81

15.7.2 Error Event Data	81

15.7.3 State Transition Event Data	81

15.8 MonBus Arbitration	81

15.8.1 Arbitration Policy	82

15.9 Top-Level MonBus Delivery (monbus_axil_axil_group)	82

15.10 Integration Notes	83

15.10.1 Downstream Connection	83

15.10.2 Packet Rate	83

16 Use Case: Network to Memory (Sink Path)	84

16.1 Overview	84

16.2 Operation Flow	84

16.3 Timing Diagram	84

16.4 Buffer Management	85

16.4.1 SRAM Buffering Strategy	85

16.4.2 Flow Control	86

16.5 Performance Considerations	86

16.5.1 Throughput	86

16.5.2 Latency	86

16.6 Error Handling	87

16.6.1 Error Sources	87

17 Use Case: Memory to Network (Source Path)	89

17.1 Overview	89

17.2 Operation Flow	89

17.3 Timing Diagram	90

17.4 Buffer Management	90

17.4.1 SRAM Buffering Strategy	90

17.4.2 Flow Control	91

17.5 Concurrent Operation	91

17.5.1 Read-Ahead Pipeline	91

17.6 Performance Considerations	92

17.6.1 Throughput	92

17.6.2 Latency	92

17.7 Error Handling	93

17.7.1 Error Sources	93

18 Use Case: Descriptor Chaining	93

18.1 Overview	93

18.2 Chain Structure	94

18.3 Timing Diagram	95

18.4 Descriptor Fields for Chaining	96

18.4.1 Relevant Fields	96

18.4.2 Chain Termination	96

18.5 Prefetch Optimization	98

18.5.1 Descriptor Prefetching	98

18.6 Multi-Channel Chaining	98

18.7 Error During Chain	98

18.7.1 Error Handling	98

18.7.2 Recovery	99

19 Use Case: Multi-Channel Operation	99

19.1 Overview	99

19.2 Channel Independence	99

19.3 Concurrent Operation Timing	100

19.4 Resource Arbitration	100

19.4.1 Descriptor Port Arbitration	100

19.4.2 Sink/Source Arbitration	101

19.5 Channel Isolation	101

19.5.1 Error Isolation	101

19.5.2 Per-Channel Soft Reset	102

19.6 Use Case Examples	102

19.6.1 Mixed Traffic Pattern	102

19.6.2 Bidirectional Transfer	103

19.6.3 Load Balancing	103

19.7 Performance Considerations	104

19.7.1 Bandwidth Sharing	104

19.7.2 Latency Impact	104

19.8 MonBus Events	104

20 Descriptor Format	105

20.1 Overview	105

20.2 Descriptor Layout	105

20.2.1 256-Bit Structure	105

20.3 Field Descriptions	107

20.3.1 Control Field [255:248]	107

20.3.2 Direction Encoding	107

20.3.3 Transfer Length [239:224]	107

20.3.4 Next Pointer [191:128]	107

20.3.5 Address Fields [127:0]	108

20.4 Descriptor Opcodes (DATA / CTRL_READ / CTRL_WRITE)	108

20.5 Descriptor Fetch Timing	109

20.6 Alignment Requirements	110

20.7 Descriptor Examples	110

20.7.1 Sink Descriptor (Network to Memory)	110

20.7.2 Source Descriptor (Memory to Network)	110

20.8 Software Construction	111

20.8.1 C Structure Example	111

20.8.2 Descriptor Ring Setup	111

21 Register Map	112

21.1 Overview	112

21.2 Address Space Layout	112

21.3 Base Registers (0x100-0x3FF)	112

21.4 Monitor Registers (0x1000)	114

21.5 Key Register Fields	115

21.5.1 GLOBAL_CTRL (0x100) - Read/Write	115

21.5.2 SCHED_TIMEOUT_LIMIT (0x208) - Read/Write	115

21.5.3 DESCENG_CONFIG (0x220) - Read/Write	115

21.6 Register Access Timing	116

21.7 Channel Kick Sequence	117

21.8 Address Decoding	117

22 Initialization Sequence	117

22.1 Overview	117

22.2 Boot Sequence	117

22.3 Detailed Steps	120

22.3.1 Step 1: Verify Hardware Presence	120

22.3.2 Step 2: Read Configuration	120

22.3.3 Step 3: Global Reset	120

22.3.4 Step 4: Allocate Resources	120

22.3.5 Step 5: Initialize Descriptors	121

22.3.6 Step 6: Configure Channels	121

22.3.7 Step 7: Enable Interrupts	121

22.3.8 Step 8: Enable Channel	122

22.4 Initialization Timing	122

22.5 Starting Transfers	122

22.5.1 Kick a Channel	122

22.6 Interrupt Handling	123

22.6.1 Completion Interrupt	123

22.7 Reset and Recovery	124

22.7.1 Per-Channel Reset	124

22.7.2 Global Reset	124

23 Error Handling	125

23.1 Overview	125

23.2 Error Categories	125

23.3 Error Detection	125

23.3.1 AXI Response Errors	125

23.3.2 Descriptor Validation Errors	126

23.3.3 Timeout Detection	127

23.4 Error Reporting	128

23.4.1 ERR_STATUS Register	128

23.4.2 MonBus Error Events	128

23.4.3 Interrupt Generation	128

23.5 Error Recovery	129

23.5.1 Recovery Flow	129

23.5.2 AXI Error Recovery	131

23.5.3 Descriptor Error Recovery	131

23.5.4 Timeout Recovery	132

23.6 Error Prevention	133

23.6.1 Best Practices	133

23.6.2 Descriptor Validation Example	133

24 Throughput Targets	134

24.1 Theoretical Maximum	134

24.1.1 Per-Interface Bandwidth	134

24.1.2 Aggregate Bandwidth	134

24.2 Practical Throughput	135

24.2.1 Single-Channel Performance	135

24.2.2 Multi-Channel Performance	135

24.3 Throughput Limiting Factors	135

24.3.1 Memory / Network System Factors	135

24.3.2 RAPIDS Internal Factors	136

24.4 Performance Targets	137

24.4.1 Design Targets	137

24.4.2 Verification Criteria	137

24.5 On-Silicon Validation	137

25 Latency Characteristics	138

25.1 Latency Components	138

25.1.1 Kick-to-First-Beat Breakdown	138

25.1.2 Total Latency Formula	139

25.2 First-Beat Latency	139

25.3 Control-Descriptor Latency	139

25.4 Latency Variability	140

25.4.1 Sources of Variability	140

25.4.2 Worst-Case Latency	140

25.5 Latency Optimization	140

25.5.1 Software Recommendations	140

25.5.2 Hardware Configuration	140

26 Resource Estimates	141

26.1 FPGA Resource Summary	141

26.2 Resource Breakdown by Block	142

26.2.1 APB Configuration Slave + Register Block	142

26.2.2 Scheduler Group (per direction)	142

26.2.3 Control Engines (per direction)	142

26.2.4 AXI Read / Write Engines	142

26.2.5 SRAM Buffers (sink + source)	142

26.2.6 MonBus / AXI Monitors (USE_AXI_MONITORS)	143

26.3 SRAM Sizing	143

26.4 Scaling Notes	143

27 RAPIDS Beats Architecture MAS	144

27.1 Document Organization	144

27.1.1 Front Matter	144

27.1.2 Chapter 1: Overview	144

27.1.3 Chapter 2: FUB (Functional Unit Blocks)	144

27.1.4 Chapter 3: Macro (Integration Blocks)	144

27.1.5 Chapter 4: Interfaces	144

27.2 Quick Reference	145

27.2.1 FUB Modules (fub_beats/)	145

27.2.2 Macro Modules (macro_beats/)	146

27.3 Architecture Comparison: RAPIDS vs STREAM	147

27.4 Clock and Reset Summary	147

27.4.1 Clock Domains	147

27.4.2 Reset Signals	148

27.5 Interface Summary	148

27.5.1 External Interfaces	148

27.5.2 Internal Buses	148

27.6 Related Documentation	149

27.7 Specification Conventions	149

27.7.1 Signal Naming	149

27.7.2 Parameter Naming	149

27.7.3 Figure and Table Numbering	149

28 Product Requirements Document (PRD)	150

28.1 RAPIDS - Rapid AXI Programmable In-band Descriptor System	150

28.2 1. Executive Summary	150

28.2.1 1.1 Quick Stats	150

28.2.2 1.2 Subsystem Goals	150

28.3 2. Documentation Structure	151

28.3.1 📚 Complete RAPIDS Specification	151

28.3.2 🐛 Known Issues	151

28.3.3 📖 Other Documentation	152

28.4 2.4 Organizational Standards - RAPIDS Code Location	152

28.4.1 Code Organization Principle	152

28.4.2 RAPIDS Directory Structure	152

28.4.3 What Goes Where?	153

28.4.4 Import Pattern for RAPIDS Tests	153

28.4.5 Benefits of This Organization	154

28.4.6 Compliance Status	154

28.5 3. Architecture Overview	154

28.5.1 3.1 Top-Level Block Diagram	154

28.5.2 3.2 Data Flow	155

28.6 4. Key Features	155

28.6.1 4.1 Descriptor-Based Operation	155

28.6.2 4.2 Data Path Features	155

28.6.3 4.3 Scheduler Features	156

28.6.4 4.4 Monitoring Integration	156

28.7 5. Interfaces	157

28.7.1 5.1 External Interfaces	157

28.7.2 5.2 Configuration Parameters	157

28.8 6. Use Cases	158

28.8.1 6.1 DMA-Style Transfers	158

28.8.2 6.2 Network Packet Processing	158

28.8.3 6.3 Custom Data Path Acceleration	158

28.9 7. Test Coverage	158

28.9.1 7.1 Current Status	158

28.9.2 7.2 Test Strategy	159

28.10 8. Known Issues Summary	160

28.10.1 8.1 Critical Issues	160

28.10.2 8.2 Medium Priority Issues	160

28.11 9. Integration Guidelines	160

28.11.1 9.1 Quick Start	160

28.11.2 9.2 Configuration Steps	161

28.12 10. Development Status	162

28.12.1 10.1 Current Phase	162

28.12.2 10.2 Roadmap	162

28.13 11. Performance Characteristics	162

28.13.1 11.1 Throughput	162

28.13.2 11.2 Latency	162

28.13.3 11.3 Resource Utilization	162

28.14 12. Verification Infrastructure	163

28.14.1 12.1 Test Organization	163

28.14.2 12.2 CocoTB Framework	163

28.14.3 12.2.1 MANDATORY: BFM Usage for FUB Tests	163

28.14.4 12.3 Test File Structure (Standard Pattern)	164

28.15 13. Quick Reference	169

28.15.1 13.1 Key Files	169

28.15.2 13.2 Commands	170

28.16 14. Success Criteria	170

28.16.1 14.1 Functional	170

28.16.2 14.2 Quality	170

28.16.3 14.3 Documentation	171

28.17 15. Educational Value	171

28.18 15. Attribution and Contribution Guidelines	171

28.18.1 15.1 Git Commit Attribution	171

28.19 16. Documentation Generation	172

28.19.1 16.1 Generating PDF/DOCX from Specification	172

28.20 16.2 PDF Generation Location	173

28.21 17. References	173

28.21.1 16.1 Internal Documentation	173

28.21.2 16.2 Related Subsystems	174

28.21.3 16.3 External References	174

28.22 Navigation	174

29 Address Increment Patterns for DMA and Descriptor-Based Data Movement	174

29.1 Table of Contents	175

29.2 1. Executive Summary	175

29.2.1 Pattern Categories	175

29.3 2. RAPIDS Current Implementation	176

29.3.1 2.1 Descriptor Format (256-bit)	176

29.3.2 2.2 Current Address Increment Modes	176

29.3.3 2.3 Alignment Information Structure	177

29.3.4 2.4 AXI Translation	177

29.4 3. AXI Protocol Burst Modes	177

29.4.1 3.1 FIXED Burst (AxBURST = 2’b00)	177

29.4.2 3.2 INCREMENT Burst (AxBURST = 2’b01)	178

29.4.3 3.3 WRAP Burst (AxBURST = 2’b10)	178

29.5 4. Linear Address Increment Patterns	178

29.5.1 4.1 Simple Contiguous	178

29.5.2 4.2 Unaligned Start/End	179

29.5.3 4.3 Size-Constrained Bursts	179

29.6 5. 2D Transfer Patterns (Stride-Based)	179

29.6.1 5.1 Concept	179

29.6.2 5.2 Descriptor Fields (Extended)	180

29.6.3 5.3 Address Calculation	180

29.6.4 5.4 Use Cases	180

29.6.5 5.5 Analog Devices AXI DMAC Implementation	180

29.7 6. 3D Transfer Patterns	181

29.7.1 6.1 Concept	181

29.7.2 6.2 Descriptor Fields (Extended)	181

29.7.3 6.3 Address Calculation	181

29.7.4 6.4 Use Cases	182

29.8 7. Scatter-Gather Patterns	182

29.8.1 7.1 Concept	182

29.8.2 7.2 SG Descriptor Fields	182

29.8.3 7.3 Operation Modes	182

29.8.4 7.4 AMD/Xilinx AXI DMA SG Descriptor Format	183

29.8.5 7.5 Use Cases	183

29.9 8. Circular and Ring Buffer Patterns	183

29.9.1 8.1 Concept	183

29.9.2 8.2 Descriptor Fields (Extended)	184

29.9.3 8.3 Address Calculation	184

29.9.4 8.4 Ping-Pong Double Buffering	184

29.9.5 8.5 Use Cases	184

29.10 9. Tensor Accelerator Memory Access Patterns (PRIORITY 0)	185

29.10.1 9.1 Systolic Array Dataflow Fundamentals	185

29.10.2 9.2 Data Cube Model (NVDLA Reference)	186

29.10.3 9.3 Tensor Memory Accelerator (TMA) - NVIDIA Hopper	186

29.10.4 9.4 Convolution Address Patterns	187

29.10.5 9.5 Gemmini Accelerator Architecture (UC Berkeley Reference)	189

29.10.6 9.6 Tiling for Large Matrices	190

29.10.7 9.7 Sparse Tensor Patterns	191

29.10.8 9.8 Transformer/Attention Patterns	192

29.10.9 9.9 Programmable Address Generation Unit (PAGU)	193

29.11 10. Implementation Comparison Matrix	194

29.11.1 10.1 Pattern Complexity vs. Capability	194

29.11.2 10.2 Use Case Mapping	194

29.11.3 10.3 RAPIDS Current vs. Extended Capability	195

29.12 11. RAPIDS Extension Recommendations	195

29.12.1 11.0 PRIORITY 0: Tensor Accelerator Addressing (CRITICAL)	195

29.12.2 11.1 Priority 1: 2D Stride Support	201

29.12.3 11.2 Priority 2: Circular Buffer Mode	201

29.12.4 11.3 Priority 3: Enhanced Scatter-Gather	202

29.13 12. References	203

29.13.1 Industry Documentation	203

29.13.2 GPU and Accelerator Patterns	203

29.13.3 DMA Architecture	203

29.13.4 Neural Network Accelerators	203

29.13.5 Convolution and Tensor Operations	203

29.13.6 Sparse Tensor Formats	204

29.14 Document History	204

List of Figures

No figures in this document.

List of Tables

Table 1: Target Audience	28

Table 2: Reference Documents	29

Table 3: Document Revision History	29

Table 4: Acronym Definitions	31

Table 5: Term Definitions	32

Table 6: Signal Naming Conventions	33

Table 7: Target Applications	34

Table 8: Phase 1 Simplifications	35

Table 9: Data Path Summary	36

Table 10: SRAM Buffer Parameters	38

Table 11: AXI Optimization Features	38

Table 12: Error Handling Summary	38

Table 13: AXI Memory Interface Requirements	41

Table 14: AXIS Network Interface	41

Table 15: Configuration Methods	42

Table 16: Clock Domains	42

Table 17: Reset Signals	42

Table 18: Power States	43

Table 19: Scheduler Array Components	46

Table 20: Sink Path Components	46

Table 21: Source Path Components	46

Table 22: Internal Bus Summary	47

Table 23: Resource Isolation	51

Table 24: Per-Channel Resources	52

Table 25: Shared Resources	53

Table 26: Channel Isolation Features	56

Table 27: External Interface Summary	59

Table 28: Clock and Reset Signals	59

Table 29: Interface Feature Matrix	59

Table 30: Signal Count Summary	60

Table 31: Descriptor AXI Configuration	61

Table 32: Descriptor AXI Signals	61

Table 33: Sink AXI Configuration	63

Table 34: Sink AXI Signals	63

Table 35: Source AXI Configuration	65

Table 36: Source AXI Signals	66

Table 37: Address Alignment Requirements	67

Table 38: AXI Response Handling	68

Table 39: Sink AXIS Configuration	69

Table 40: Sink AXIS Signals	69

Table 41: Source AXIS Configuration	71

Table 42: Source AXIS Signals	71

Table 43: TLAST Usage	73

Table 44: TKEEP Examples	73

Table 45: APB Address Routing	74

Table 46: APB Slave Signals	74

Table 47: Status Signals	76

Table 48: Scheduler State Encoding	76

Table 49: Key Configuration Registers	77

Table 50: MonBus Signals	79

Table 51: MonBus Packet Types	79

Table 52: RAPIDS Agent IDs	79

Table 53: Scheduler Event Codes	80

Table 54: Descriptor Engine Event Codes	80

Table 55: Top-Level MonBus AXI-Lite Group	83

Table 56: MonBus Packet Rates	83

Table 57: Sink Path Flow Control	86

Table 58: Sink Path Throughput Factors	86

Table 59: Sink Path Latency Breakdown	86

Table 60: Sink Path Error Handling	87

Table 61: Source Path Flow Control	91

Table 62: Source Path Throughput Factors	92

Table 63: Source Path Latency Breakdown	92

Table 64: Source Path Error Handling	93

Table 65: Chaining-Related Descriptor Fields	96

Table 66: Mixed Traffic Channel Assignment	102

Table 67: Bandwidth Distribution	104

Table 68: Arbitration Latency	104

Table 69: Control Field Bits	107

Table 70: Direction Field Encoding	107

Table 71: Address Alignment Requirements	110

Table 72: Address Space Layout	112

Table 73: Base Registers	112

Table 74: Monitor Registers (base 0x1000)	114

Table 75: GLOBAL_CTRL Register	115

Table 76: SCHED_TIMEOUT_LIMIT Register	115

Table 77: DESCENG_CONFIG Register	115

Table 78: AXI Response Codes	125

Table 79: Descriptor Validation Errors	126

Table 80: Timeout Thresholds	127

Table 81: Error Prevention Best Practices	133

Table 82: FUB Module Summary	145

Table 83: Macro Module Summary	146

Table 84: RAPIDS Beats vs STREAM Comparison	147

Table 85: Clock Domains	147

Table 86: Reset Signals	148

Table 87: External Interfaces	148

Table 88: Internal Buses	148

# 1	Rapids Beats Has Index

Generated: 2026-08-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

Document Title: RAPIDS Beats Hardware Architecture Specification (HAS) Document Number: RAPIDS-HAS-001 Version: 0.8 Date: 2026-07-13 Status: Released

## 2.1	Purpose

This Hardware Architecture Specification (HAS) defines the external interfaces, system integration requirements, and high-level architecture of the RAPIDS Beats accelerator. It is intended for system architects and integration engineers who need to understand RAPIDS Beats from an external perspective without requiring knowledge of internal implementation details.

## 2.2	Scope

This document covers:

- External interface specifications (AXI4, AXIS, APB, MonBus)

- System-level block diagrams and data flows

- Programming model and descriptor format

- Use cases and operational sequences

- Performance characteristics and constraints

This document does NOT cover:

- Internal module architecture (see MAS)

- RTL implementation details (see MAS)

- Verification test plans

- Physical design constraints

## 2.3	Audience

Target Audience

| Role | Relevance |
| --- | --- |
| System Architect | Primary - system integration |
| Hardware Integration Engineer | Primary - interface connection |
| Software Engineer | Primary - driver development |
| Verification Engineer | Reference - interface protocols |
| RTL Designer | Reference - external constraints |

## 2.4	Document Conventions

### 2.4.1	Requirement Levels

| Term | Meaning |
| --- | --- |
| SHALL | Mandatory requirement |
| SHOULD | Recommended but not mandatory |
| MAY | Optional feature |

### 2.4.2	Signal Directions

- Input: Signal driven by external logic into RAPIDS

- Output: Signal driven by RAPIDS to external logic

- Bidirectional: Signal can be driven by either side (rare)

### 2.4.3	Timing Diagrams

All timing diagrams use WaveDrom format with the following conventions:

- p = Positive clock edge

- n = Negative clock edge

- 0/1 = Logic low/high

- x = Unknown/don’t care

- = = Data value (with label)

- . = Previous value continues

## 2.5	References

Reference Documents

| Document | Description |
| --- | --- |
| RAPIDS Beats MAS | Module Architecture Specification |
| ARM AMBA AXI4 Specification | AXI4 protocol reference |
| ARM AMBA AXI-Stream Specification | AXIS protocol reference |
| RAPIDS PRD | Product Requirements Document |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Revision History

Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-17 | RTL Design Sherpa | Initial release |
| 0.6 | 2026-07-02 | RTL Design Sherpa | Resynced to RTL top-level integration |
| 0.7 | 2026-07-13 | RTL Design Sherpa | Added the control-descriptor feature (CTRL_READ consumer gate / CTRL_WRITE producer doorbell) |
| 0.8 | 2026-07-13 | RTL Design Sherpa | Added Chapter 6: Performance (throughput targets, latency characteristics, measured resource estimates) |

## 3.1	Change Summary

### 3.1.1	Version 0.8 (2026-07-13)

Performance Chapter

- Added Chapter 6: Performance, mirroring the STREAM HAS performance chapter:

  - Section 6.1 Throughput Targets – per-interface and aggregate bandwidth (512-bit @ 100 MHz = 6.4 GB/s per direction, 12.8 GB/s full-duplex with concurrent sink + source), practical single/multi-channel efficiency, limiting factors, design targets, and on-silicon golden-CRC validation (48/48 configs).

  - Section 6.2 Latency Characteristics – kick-to-first-beat breakdown, control-descriptor latency (CTRL_READ retry budget, CTRL_WRITE doorbell), variability, and worst-case bounds.

  - Section 6.3 Resource Estimates – measured post-implementation utilization from the Nexys A7-100T characterization build (37,555 LUT / 28,683 FF / 22 BRAM / 0 DSP at NUM_CHANNELS=4, timing met at 100 MHz), per-block breakdown, and scaling notes.

### 3.1.2	Version 0.7 (2026-07-13)

Control-Descriptor Feature

- Documented the control-descriptor opcodes in Descriptor Format (Section 5.1): CTRL_READ (consumer gate – poll a memory location until (read & mask) == expected) and CTRL_WRITE (producer doorbell – single 32-bit write), for in-memory producer/consumer synchronization without moving payload. Added the per-opcode field layouts.

- Added CTRL_CONFIG @ 0x240 (CTRLRD_MAX_TRY[8:0], the control-read poll retry budget) to the register map (Section 5.2) and a Key Features entry.

### 3.1.3	Version 0.6 (2026-07-02)

RTL Resync

- Documented the rapids_beats_top integration: single APB slave routed to descriptor kick-off (0x000-0x03F) and the rapids_regs register block (base 0x100-0x3FF plus monitor regfile @ 0x1000).

- Refreshed the register map with the actual PeakRDL-generated addresses, including the monitor regfile at 0x1000 and SCHED_TIMEOUT_LIMIT at 0x208.

- Added the MonBus AXI-Lite group (error-drain slave s_axil_err_*, capture master m_axil_mon_*, mon_irq) and the USE_AXI_MONITORS-gated rd/wr AXI monitors to the block diagram, APB, and MonBus interface chapters.

- Reflected scheduler recoverable write-progress timeout + B-response commit gating and functional descriptor prefetch.

### 3.1.4	Version 1.0 (2026-01-17)

Initial Release

- Complete HAS for RAPIDS Beats Phase 1 architecture

- External interface specifications (AXI4, AXIS, APB, MonBus)

- System block diagrams with Mermaid

- Timing diagrams with WaveDrom

- Programming model and descriptor format

- Use case documentation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Acronyms and Definitions

## 4.1	Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface |
| AXIS | AXI-Stream |
| APB | Advanced Peripheral Bus |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| MAS | Module Architecture Specification |
| MonBus | Monitor Bus |
| RAPIDS | Rapid AXI Programmable In-band Descriptor System |
| SRAM | Static Random Access Memory |

## 4.2	Definitions

Term Definitions

| Term | Definition |
| --- | --- |
| Beat | Single data transfer unit (512 bits = 64 bytes in default configuration) |
| Channel | Independent DMA context with dedicated descriptor processing |
| Descriptor | 256-bit control structure defining a single DMA transfer |
| Descriptor Chain | Linked list of descriptors for multi-transfer operations |
| Sink Path | Data flow from network (AXIS slave) to memory (AXI write) |
| Source Path | Data flow from memory (AXI read) to network (AXIS master) |
| Fill | Operation writing data into SRAM buffer (sink path input) |
| Drain | Operation reading data from SRAM buffer (source path output) |

## 4.3	Signal Naming Conventions

Signal Naming Conventions

| Prefix/Suffix | Meaning |
| --- | --- |
| m_axi_* | AXI Master signals |
| s_axis_* | AXIS Slave signals |
| m_axis_* | AXIS Master signals |
| cfg_* | Configuration signals |
| *_valid | Handshake valid signal |
| *_ready | Handshake ready signal |
| *_n | Active-low signal |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Product Overview

## 5.1	Introduction

RAPIDS Beats is a high-performance network-to-memory accelerator designed for descriptor-based data movement operations. It bridges network interfaces (AXI-Stream) with system memory (AXI4), enabling efficient DMA transfers without CPU intervention.

The “Beats” architecture represents Phase 1 of the RAPIDS family, optimized for simplicity and throughput in streaming applications.

## 5.2	System Context

System Context Diagram

System Context Diagram

Source: 01_system_context.mmd

Diagram 1

## 5.3	Target Applications

Target Applications

| Application | Description | Key Requirements |
| --- | --- | --- |
| Network Offload | Receive packets from network, store to memory | Low latency, high throughput |
| Scatter-Gather DMA | Multi-descriptor data movement | Descriptor chaining |
| Streaming Accelerator | Front-end for compute engines | Continuous data flow |
| Protocol Processing | Header/payload separation | Multi-channel isolation |

## 5.4	Architecture Highlights

### 5.4.1	Phase 1 “Beats” Simplifications

The Beats architecture prioritizes simplicity over feature completeness:

Phase 1 Simplifications

| Feature | Beats (Phase 1) | Future Phases |
| --- | --- | --- |
| Transfer Unit | Full beats only | Chunk-level alignment |
| Flow Control | Backpressure-based | Credit management |
| Address Alignment | Pre-aligned required | Hardware alignment fixup |
| Network Interface | Direct AXIS | Protocol conversion |

### 5.4.2	Key Capabilities

- 8 Independent Channels - Concurrent transfers with dedicated resources

- Descriptor Chaining - Automatic next-descriptor fetch

- Dual Data Paths - Simultaneous sink (write) and source (read)

- Monitor Integration - Real-time event reporting via MonBus

- Error Detection - Address range checking, AXI error propagation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Key Features

## 6.1	Feature Summary

### 6.1.1	Multi-Channel Architecture

- 8 Independent Channels with dedicated schedulers

- Per-Channel State Machines for concurrent operation

- Shared AXI Infrastructure with arbitrated access

- Channel Isolation prevents cross-channel interference

### 6.1.2	Descriptor-Based Control

- 256-bit Descriptor Format with all transfer parameters

- Automatic Chaining follows next_descriptor_ptr links

- Last Descriptor Flag terminates chain processing

- IRQ Generation via gen_irq descriptor flag

- Control Descriptors (CTRL_READ consumer gate / CTRL_WRITE producer doorbell) for in-memory producer/consumer synchronization without moving payload – see Descriptor Format (Section 5.1)

### 6.1.3	High-Performance Data Paths

Data Path Summary

| Path | Direction | Bandwidth | Features |
| --- | --- | --- | --- |
| Sink | Network to Memory | 512-bit @ clock | SRAM buffering, AXI burst writes |
| Source | Memory to Network | 512-bit @ clock | AXI burst reads, SRAM buffering |

### 6.1.4	Monitoring and Debug

- 64-bit MonBus Packets for all significant events

- State Transition Reports track FSM activity

- Error Event Codes for fault diagnosis

- Performance Metrics for throughput analysis

## 6.2	Feature Details

### 6.2.1	Descriptor Processing

Descriptor Flow

Descriptor Flow

Source: 02_descriptor_flow.mmd

Diagram 2

### 6.2.2	SRAM Buffering

Each data path includes dedicated SRAM buffering:

SRAM Buffer Parameters

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| SRAM_DEPTH | 512 | 64-4096 | Entries per channel |
| Data Width | 512 | Fixed | Bits per entry |
| Total Size | 32KB | 4KB-256KB | Per data path |

Buffer Operation:

- Sink Path: Fill from AXIS, drain to AXI write engine

- Source Path: Fill from AXI read engine, drain to AXIS

- Flow Control: Backpressure when buffer full/empty

### 6.2.3	AXI Burst Optimization

RAPIDS optimizes AXI transactions for maximum bandwidth:

AXI Optimization Features

| Feature | Implementation |
| --- | --- |
| Burst Length | Up to 256 beats (AXI4 max) |
| Outstanding Transactions | Configurable (default 8) |
| Address Alignment | 64-byte aligned (beat size) |
| 4KB Boundary | Automatic burst splitting |

### 6.2.4	Error Handling

Error Handling Summary

| Error Type | Detection | Response |
| --- | --- | --- |
| Address Range | Pre-fetch validation | Descriptor rejected |
| AXI SLVERR | Response check | Transfer aborted |
| AXI DECERR | Response check | Transfer aborted |
| Timeout | Watchdog timer | Channel reset |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	System Context

## 7.1	Integration Overview

RAPIDS Beats integrates into a system as a DMA engine bridging network interfaces and system memory. This section describes the external connections and system-level requirements.

## 7.2	System Block Diagram

System Integration

System Integration

Source: 03_system_integration.mmd

Diagram 3

## 7.3	Interface Connections

### 7.3.1	Memory Interface

RAPIDS requires three AXI4 master ports connected to system memory:

AXI Memory Interface Requirements

| Port | Access Pattern | Typical Bandwidth |
| --- | --- | --- |
| Descriptor AXI | Random read (256-bit) | Low (<100 MB/s) |
| Sink AXI | Sequential write (512-bit) | High (up to interface max) |
| Source AXI | Sequential read (512-bit) | High (up to interface max) |

Interconnect Requirements:

- All three ports MAY share a single physical AXI port with arbitration

- Descriptor port has lowest bandwidth requirement

- Sink and Source ports should have balanced bandwidth allocation

### 7.3.2	Network Interface

RAPIDS uses standard AXI-Stream for network connectivity:

AXIS Network Interface

| Interface | Role | Signals |
| --- | --- | --- |
| Sink AXIS | Receives data from network | TDATA, TVALID, TREADY, TLAST |
| Source AXIS | Sends data to network | TDATA, TVALID, TREADY, TLAST |

Protocol Notes:

- TKEEP/TSTRB supported for partial beat handling

- TID/TDEST may be used for channel routing (optional)

- TUSER available for sideband information

### 7.3.3	Configuration Interface

Software configures RAPIDS through register writes:

Configuration Methods

| Method | Interface | Usage |
| --- | --- | --- |
| Direct Write | Memory-mapped | Register configuration |
| Descriptor Kick | APB-style | Start descriptor processing |

## 7.4	Clock and Reset

### 7.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | All RAPIDS logic |

Note: RAPIDS Beats uses a single clock domain. CDC (clock domain crossing) to different frequency domains must be handled externally.

### 7.4.2	Reset Requirements

Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert |

Reset Sequence:

- Assert aresetn low for minimum 4 clock cycles

- Ensure all AXI interfaces are idle before reset

- Release reset synchronously to aclk rising edge

- Wait 16 clock cycles before initiating transfers

## 7.5	Power Considerations

### 7.5.1	Clock Gating

RAPIDS supports optional clock gating for power reduction:

- Idle Detection: Automatic when no transfers pending

- Per-Channel Gating: Individual channels can be clock-gated

- External Control: cfg_clock_enable override available

### 7.5.2	Power States

Power States

| State | Description | Exit Latency |
| --- | --- | --- |
| Active | Normal operation | 0 cycles |
| Idle | No pending transfers | 1 cycle |
| Gated | Clock disabled | 4 cycles |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Block Diagram

## 8.1	Top-Level Architecture

RAPIDS Beats Block Diagram

RAPIDS Beats Block Diagram

Source: 04_block_diagram.mmd

Diagram 4

## 8.2	Top-Level Integration (rapids_beats_top)

rapids_core_beats is wrapped by rapids_beats_top, which adds the software register interface, configuration mapping, and monitoring infrastructure. A single APB slave feeds a command/response router that splits accesses between per-channel descriptor kick-off (apb4todescr, 0x000-0x03F) and the rapids_regs register block (base config at 0x100-0x3FF, monitor regfile at 0x1000). rapids_config_block translates the register hwif_out into the core/monitor cfg_* signals.

When USE_AXI_MONITORS = 1, AXI transaction monitors observe the read and write data masters; their packets are merged with the core descriptor-monitor packet by a monbus_arbiter and delivered through a monbus_axil_axil_group to an AXI-Lite error-drain slave, an AXI-Lite capture master, and a mon_irq interrupt.

Diagram 5

## 8.3	Component Summary

### 8.3.1	Scheduler Group Array

The scheduler group array manages 8 independent channels:

Scheduler Array Components

| Component | Instance Count | Purpose |
| --- | --- | --- |
| scheduler_beats | 8 | Transfer coordination per channel |
| descriptor_engine_beats | 8 | Descriptor fetch and parse |
| Descriptor AXI Arbiter | 1 | Shared AXI4 for descriptor fetch |

### 8.3.2	Sink Data Path

Network-to-memory data flow:

Sink Path Components

| Component | Purpose |
| --- | --- |
| snk_sram_controller_beats | Multi-channel SRAM management |
| axi_write_engine_beats | AXI4 burst write generation |
| alloc_ctrl_beats | Space allocation tracking |
| drain_ctrl_beats | Data availability tracking |

### 8.3.3	Source Data Path

Memory-to-network data flow:

Source Path Components

| Component | Purpose |
| --- | --- |
| axi_read_engine_beats | AXI4 burst read generation |
| src_sram_controller_beats | Multi-channel SRAM management |
| alloc_ctrl_beats | Space allocation tracking |
| drain_ctrl_beats | Data availability tracking |

## 8.4	Hierarchy

rapids_core_beats
├── beats_scheduler_group_array
│   ├── beats_scheduler_group [0]
│   │   ├── scheduler_beats
│   │   └── descriptor_engine_beats
│   ├── beats_scheduler_group [1..6]
│   │   └── ...
│   └── beats_scheduler_group [7]
│       └── ...
├── sink_data_path_beats
│   ├── snk_sram_controller_beats
│   │   └── snk_sram_controller_unit_beats [0..7]
│   ├── axi_write_engine_beats
│   ├── alloc_ctrl_beats
│   └── drain_ctrl_beats
└── source_data_path_beats
    ├── axi_read_engine_beats
    ├── src_sram_controller_beats
    │   └── src_sram_controller_unit_beats [0..7]
    ├── alloc_ctrl_beats
    └── drain_ctrl_beats

## 8.5	Internal Buses

Internal Bus Summary

| Bus | Width | Description |
| --- | --- | --- |
| Descriptor Bus | 256-bit | Parsed descriptor to scheduler |
| Scheduler Command | Variable | Transfer parameters to data paths |
| SRAM Data | 512-bit | Data to/from SRAM buffers |
| MonBus | 64-bit | Monitor packets (aggregated) |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Data Flow

## 9.1	Overview

RAPIDS Beats implements two independent data paths that can operate concurrently:

- Sink Path: Network to Memory (AXIS Slave -> AXI Write)

- Source Path: Memory to Network (AXI Read -> AXIS Master)

## 9.2	Sink Path Data Flow

Sink Data Flow

Sink Data Flow

Source: 05_sink_flow.mmd

Diagram 6

### 9.2.1	Sink Path Timing

Sink Path Timing

Sink Path Timing

Source: sink_path_timing.json

Waveform 1

## 9.3	Source Path Data Flow

Source Data Flow

Source Data Flow

Source: 06_source_flow.mmd

Diagram 7

### 9.3.1	Source Path Timing

Source Path Timing

Source Path Timing

Source: source_path_timing.json

Waveform 2

## 9.4	Concurrent Operation

Both data paths operate independently and can run simultaneously:

Diagram 8

### 9.4.1	Deadlock Prevention

RAPIDS prevents deadlock through independent resource allocation:

Resource Isolation

| Resource | Sink Path | Source Path | Shared |
| --- | --- | --- | --- |
| SRAM Buffer | Dedicated | Dedicated | No |
| AXI Port | Write only | Read only | No |
| Scheduler | Per-channel | Per-channel | No |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Channel Architecture

## 10.1	Multi-Channel Design

RAPIDS Beats supports 8 independent DMA channels, each with dedicated resources for concurrent operation.

## 10.2	Channel Resource Allocation

Channel Architecture

Channel Architecture

Source: 07_channel_arch.mmd

Diagram 9

## 10.3	Per-Channel Resources

Each channel has dedicated:

Per-Channel Resources

| Resource | Description | Size/Capacity |
| --- | --- | --- |
| Scheduler | Transfer coordination FSM | 1 instance |
| Descriptor Engine | Descriptor fetch/parse | 8-deep FIFO |
| Sink SRAM Unit | Sink buffer partition | SRAM_DEPTH/8 entries |
| Source SRAM Unit | Source buffer partition | SRAM_DEPTH/8 entries |
| Configuration Registers | Per-channel config | ~16 registers |

## 10.4	Shared Resources

Resources shared across all channels:

Shared Resources

| Resource | Arbitration | Max Outstanding |
| --- | --- | --- |
| Descriptor AXI | Round-robin | 8 (1 per channel) |
| Sink AXI Write | Round-robin | 8 configurable |
| Source AXI Read | Round-robin | 8 configurable |
| MonBus Output | Priority | 1 (serialized) |

## 10.5	Channel Scheduling

Arbitration Policy

Arbitration Timing:

Channel Arbitration

Channel Arbitration

Source: channel_arbitration.json

Waveform 3

## 10.6	Channel State Machine

Each channel scheduler follows this state machine:

Channel FSM

Channel FSM

Source: 08_channel_fsm.mmd

Diagram 11

### 10.6.1	Channel State Timing

Channel State Timing

Channel State Timing

Source: channel_state_timing.json

Waveform 4

## 10.7	Channel Isolation

Channels are isolated to prevent interference:

Channel Isolation Features

| Isolation Type | Implementation |
| --- | --- |
| State Isolation | Separate FSM per channel |
| Buffer Isolation | Partitioned SRAM per channel |
| Error Isolation | Per-channel error flags |
| Reset Isolation | Per-channel soft reset |

### 10.7.1	Per-Channel Error Handling

Each channel maintains independent error state:

Waveform 5

Channel 1 error does not affect Channel 0 or Channel 2 operation.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Interface Summary

## 11.1	External Interface Overview

RAPIDS Beats exposes the following external interfaces:

Interface Overview

Interface Overview

Source: 09_interface_overview.mmd

Diagram 12

## 11.2	Interface Port Summary

External Interface Summary

| Interface | Type | Direction | Data Width | Address Width |
| --- | --- | --- | --- | --- |
| Descriptor AXI | AXI4 Master | Read | 256-bit | 64-bit |
| Sink AXI | AXI4 Master | Write | 512-bit | 64-bit |
| Source AXI | AXI4 Master | Read | 512-bit | 64-bit |
| Sink AXIS | AXIS Slave | Input | 512-bit | - |
| Source AXIS | AXIS Master | Output | 512-bit | - |
| APB Config | APB-like | Input | 32-bit | 12-bit |
| MonBus | Custom | Output | 64-bit | - |

## 11.3	Clock and Reset

All interfaces operate in the same clock domain:

Clock and Reset Signals

| Signal | Type | Description |
| --- | --- | --- |
| aclk | input | System clock (100-500 MHz typical) |
| aresetn | input | Active-low reset (async assert, sync deassert) |

## 11.4	Interface Feature Matrix

Interface Feature Matrix

| Feature | Desc AXI | Sink AXI | Src AXI | Sink AXIS | Src AXIS |
| --- | --- | --- | --- | --- | --- |
| Protocol | AXI4 | AXI4 | AXI4 | AXIS | AXIS |
| Direction | Read | Write | Read | Slave | Master |
| Burst Support | INCR | INCR | INCR | N/A | N/A |
| Max Burst | 1 | 256 | 256 | N/A | N/A |
| Outstanding | 8 | 8 | 8 | N/A | N/A |
| ID Width | 8-bit | 8-bit | 8-bit | Optional | Optional |

## 11.5	Signal Count Summary

Signal Count Summary

| Interface | Input Signals | Output Signals | Total |
| --- | --- | --- | --- |
| Descriptor AXI | ~20 | ~30 | ~50 |
| Sink AXI | ~15 | ~40 | ~55 |
| Source AXI | ~25 | ~25 | ~50 |
| Sink AXIS | ~20 | ~5 | ~25 |
| Source AXIS | ~5 | ~20 | ~25 |
| APB Config | ~15 | ~5 | ~20 |
| MonBus | ~2 | ~5 | ~7 |
| Total | ~102 | ~130 | ~232 |

Note: Signal counts are approximate and vary with configuration parameters.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI4 Interface Specification

## 12.1	Overview

RAPIDS Beats uses three AXI4 master interfaces for memory access:

- Descriptor AXI - Fetches 256-bit descriptors (read-only)

- Sink AXI - Writes data to memory (write-only)

- Source AXI - Reads data from memory (read-only)

## 12.2	Descriptor AXI Master

### 12.2.1	Purpose

Fetches descriptor structures from system memory for all 8 channels.

### 12.2.2	Configuration

Descriptor AXI Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| Data Width | 256-bit | Fixed descriptor size |
| Address Width | 64-bit | Configurable |
| ID Width | 8-bit | Configurable |
| Burst Length | 1 | Single-beat only |
| Burst Type | INCR | Fixed |

### 12.2.3	Signal List

Descriptor AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_desc_aclk | 1 | input | Clock |
| m_axi_desc_aresetn | 1 | input | Reset (active-low) |
| m_axi_desc_arid | 8 | output | Read ID (channel ID) |
| m_axi_desc_araddr | 64 | output | Read address |
| m_axi_desc_arlen | 8 | output | Burst length (always 0) |
| m_axi_desc_arsize | 3 | output | Beat size (5 = 32 bytes) |
| m_axi_desc_arburst | 2 | output | Burst type (INCR) |
| m_axi_desc_arvalid | 1 | output | Address valid |
| m_axi_desc_arready | 1 | input | Address ready |
| m_axi_desc_rid | 8 | input | Response ID |
| m_axi_desc_rdata | 256 | input | Read data |
| m_axi_desc_rresp | 2 | input | Read response |
| m_axi_desc_rlast | 1 | input | Last beat |
| m_axi_desc_rvalid | 1 | input | Data valid |
| m_axi_desc_rready | 1 | output | Data ready |

### 12.2.4	Timing Diagram

Descriptor Fetch Timing

Descriptor Fetch Timing

Source: desc_axi_timing.json

Waveform 6

## 12.3	Sink AXI Master (Write)

### 12.3.1	Purpose

Writes data from SRAM buffer to system memory.

### 12.3.2	Configuration

Sink AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | Match SRAM width |
| Address Width | 64-bit | Configurable | System address space |
| ID Width | 8-bit | Configurable | Transaction tracking |
| Max Burst | 256 | 1-256 | AXI4 maximum |
| Outstanding | 8 | 1-16 | Concurrent AW requests |

### 12.3.3	Signal List

Sink AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_sink_awid | 8 | output | Write ID |
| m_axi_sink_awaddr | 64 | output | Write address |
| m_axi_sink_awlen | 8 | output | Burst length - 1 |
| m_axi_sink_awsize | 3 | output | Beat size (6 = 64 bytes) |
| m_axi_sink_awburst | 2 | output | Burst type (INCR) |
| m_axi_sink_awvalid | 1 | output | Address valid |
| m_axi_sink_awready | 1 | input | Address ready |
| m_axi_sink_wdata | 512 | output | Write data |
| m_axi_sink_wstrb | 64 | output | Write strobes |
| m_axi_sink_wlast | 1 | output | Last beat |
| m_axi_sink_wvalid | 1 | output | Data valid |
| m_axi_sink_wready | 1 | input | Data ready |
| m_axi_sink_bid | 8 | input | Response ID |
| m_axi_sink_bresp | 2 | input | Write response |
| m_axi_sink_bvalid | 1 | input | Response valid |
| m_axi_sink_bready | 1 | output | Response ready |

### 12.3.4	Timing Diagram

Sink AXI Write Timing

Sink AXI Write Timing

Source: sink_axi_timing.json

Waveform 7

## 12.4	Source AXI Master (Read)

### 12.4.1	Purpose

Reads data from system memory to SRAM buffer.

### 12.4.2	Configuration

Source AXI Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | Match SRAM width |
| Address Width | 64-bit | Configurable | System address space |
| ID Width | 8-bit | Configurable | Transaction tracking |
| Max Burst | 256 | 1-256 | AXI4 maximum |
| Outstanding | 8 | 1-16 | Concurrent AR requests |

### 12.4.3	Signal List

Source AXI Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axi_src_arid | 8 | output | Read ID |
| m_axi_src_araddr | 64 | output | Read address |
| m_axi_src_arlen | 8 | output | Burst length - 1 |
| m_axi_src_arsize | 3 | output | Beat size (6 = 64 bytes) |
| m_axi_src_arburst | 2 | output | Burst type (INCR) |
| m_axi_src_arvalid | 1 | output | Address valid |
| m_axi_src_arready | 1 | input | Address ready |
| m_axi_src_rid | 8 | input | Response ID |
| m_axi_src_rdata | 512 | input | Read data |
| m_axi_src_rresp | 2 | input | Read response |
| m_axi_src_rlast | 1 | input | Last beat |
| m_axi_src_rvalid | 1 | input | Data valid |
| m_axi_src_rready | 1 | output | Data ready |

### 12.4.4	Timing Diagram

Source AXI Read Timing

Source AXI Read Timing

Source: source_axi_timing.json

Waveform 8

## 12.5	AXI Protocol Constraints

### 12.5.1	Address Alignment

Address Alignment Requirements

| Transfer Type | Alignment Requirement |
| --- | --- |
| Descriptor | 32-byte aligned |
| Sink Data | 64-byte aligned |
| Source Data | 64-byte aligned |

### 12.5.2	4KB Boundary Handling

AXI4 requires burst transactions not to cross 4KB boundaries. RAPIDS automatically splits bursts at 4KB boundaries:

Example: 8-beat burst starting at 0x0FE0 (64 bytes/beat)
  Beat 0-1: 0x0FE0 - 0x0FFF (to 4KB boundary)
  Beat 2-7: 0x1000 - 0x117F (after boundary)

Split into two bursts:
  Burst 1: ARLEN=1, ARADDR=0x0FE0
  Burst 2: ARLEN=5, ARADDR=0x1000

### 12.5.3	Response Handling

AXI Response Handling

| Response | Code | RAPIDS Behavior |
| --- | --- | --- |
| OKAY | 2’b00 | Normal completion |
| EXOKAY | 2’b01 | Treated as OKAY |
| SLVERR | 2’b10 | Error, transfer aborted |
| DECERR | 2’b11 | Error, transfer aborted |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	AXIS Interface Specification

## 13.1	Overview

RAPIDS Beats uses AXI-Stream interfaces for network data transfer:

- Sink AXIS Slave - Receives data from network (ingress)

- Source AXIS Master - Sends data to network (egress)

## 13.2	Sink AXIS Slave

### 13.2.1	Purpose

Receives streaming data from external network interface and writes to SRAM buffer.

### 13.2.2	Configuration

Sink AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | TDATA width |
| ID Width | 8-bit | 0-8 | TID width (optional) |
| DEST Width | 8-bit | 0-8 | TDEST width (optional) |
| USER Width | 1-bit | 0-16 | TUSER width (optional) |

### 13.2.3	Signal List

Sink AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_axis_sink_tdata | 512 | input | Data payload |
| s_axis_sink_tkeep | 64 | input | Byte valid mask |
| s_axis_sink_tstrb | 64 | input | Byte position (optional) |
| s_axis_sink_tlast | 1 | input | End of packet |
| s_axis_sink_tid | 8 | input | Transaction ID (channel select) |
| s_axis_sink_tdest | 8 | input | Destination (optional) |
| s_axis_sink_tuser | 1 | input | User sideband (optional) |
| s_axis_sink_tvalid | 1 | input | Data valid |
| s_axis_sink_tready | 1 | output | Ready to accept |

### 13.2.4	Timing Diagram

Sink AXIS Timing

Sink AXIS Timing

Source: sink_axis_timing.json

Waveform 9

### 13.2.5	Backpressure Behavior

When SRAM buffer is full, tready deasserts:

Waveform 10

## 13.3	Source AXIS Master

### 13.3.1	Purpose

Sends streaming data from SRAM buffer to external network interface.

### 13.3.2	Configuration

Source AXIS Configuration

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| Data Width | 512-bit | Configurable | TDATA width |
| ID Width | 8-bit | 0-8 | TID width (optional) |
| DEST Width | 8-bit | 0-8 | TDEST width (optional) |
| USER Width | 1-bit | 0-16 | TUSER width (optional) |

### 13.3.3	Signal List

Source AXIS Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m_axis_src_tdata | 512 | output | Data payload |
| m_axis_src_tkeep | 64 | output | Byte valid mask |
| m_axis_src_tstrb | 64 | output | Byte position (optional) |
| m_axis_src_tlast | 1 | output | End of packet |
| m_axis_src_tid | 8 | output | Transaction ID (channel) |
| m_axis_src_tdest | 8 | output | Destination (optional) |
| m_axis_src_tuser | 1 | output | User sideband (optional) |
| m_axis_src_tvalid | 1 | output | Data valid |
| m_axis_src_tready | 1 | input | Downstream ready |

### 13.3.4	Timing Diagram

Source AXIS Timing

Source AXIS Timing

Source: source_axis_timing.json

Waveform 11

### 13.3.5	Data Availability

Source AXIS only asserts tvalid when data is available in SRAM:

Waveform 12

## 13.4	AXIS Protocol Notes

### 13.4.1	Handshake Rules

Standard AXI-Stream handshake applies:

- tvalid asserts when data is available

- tready asserts when receiver can accept

- Transfer occurs on clock edge when both are high

- tvalid SHALL NOT depend on tready

- tready MAY depend on tvalid

### 13.4.2	TLAST Semantics

TLAST Usage

| Scenario | TLAST Behavior |
| --- | --- |
| Descriptor transfer complete | Assert on last beat |
| Packet boundary | Assert on last beat of packet |
| Continuous stream | Not used (always 0) |

### 13.4.3	TKEEP Usage

TKEEP indicates valid bytes within each beat:

TKEEP Examples

| TKEEP Value | Meaning |
| --- | --- |
| 64’hFFFF_FFFF_FFFF_FFFF | All 64 bytes valid |
| 64’h0000_FFFF_FFFF_FFFF | Upper 16 bytes invalid |
| 64’h0000_0000_0000_000F | Only lower 4 bytes valid |

### 13.4.4	Channel Multiplexing via TID

Multiple channels can share a single AXIS interface using TID:

Waveform 13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	APB Configuration Interface

## 14.1	Overview

At the top level (rapids_beats_top), RAPIDS Beats exposes a single APB4 slave (s_apb_*) for all software access. This interface allows software to:

- Configure channel and monitor parameters (via the register block)

- Initiate descriptor processing (kick-off)

- Read status and error information

## 14.2	APB Slave and Address Routing

The APB slave is converted to an internal command/response transaction and routed by address to two targets:

APB Address Routing

| Address Range | Target | Purpose |
| --- | --- | --- |
| 0x000-0x03F | Descriptor kick-off (apb4todescr) | Per-channel descriptor chain start |
| 0x100-0x3FF | rapids_regs (base regfile) | Channel/scheduler/descriptor configuration and status |
| 0x1000+ | rapids_regs (monitor regfile) | AXI-monitor configuration and performance counters |

Because the monitor regfile is located at 0x1000, the APB address bus must be at least 13 bits wide to reach it. Configuration that was formerly imagined as discrete cfg_* ports is now driven internally by the register block through rapids_config_block; see the Register Map for the full address map, and the descriptor address ranges (DESCENG_ADDR0/1_* at 0x224-0x230).

### 14.2.1	APB Slave Signal List

APB Slave Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_apb_paddr | APB_ADDR_WIDTH (>=13) | input | Address |
| s_apb_psel | 1 | input | Select |
| s_apb_penable | 1 | input | Enable phase |
| s_apb_pwrite | 1 | input | Write / read |
| s_apb_pwdata | 32 | input | Write data |
| s_apb_pstrb | 4 | input | Write byte strobes |
| s_apb_prdata | 32 | output | Read data |
| s_apb_pready | 1 | output | Ready |
| s_apb_pslverr | 1 | output | Slave error |

### 14.2.2	Descriptor Kick-Off

To start descriptor processing on a channel:

- Write descriptor to memory

- Assert apb_valid[ch] with descriptor address on apb_addr

- Wait for apb_ready[ch] assertion

- Deassert apb_valid[ch]

### 14.2.3	Timing Diagram

APB Kick-Off Timing

APB Kick-Off Timing

Source: apb_kickoff_timing.json

Waveform 14

## 14.3	Status Interface

### 14.3.1	Per-Channel Status Signals

Status Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| channel_idle | NC | output | Channel idle status |
| channel_error | NC | output | Channel error flag |
| scheduler_state | 4*NC | output | FSM state per channel |

### 14.3.2	Channel State Encoding

Scheduler State Encoding

| State | Value | Description |
| --- | --- | --- |
| IDLE | 4’h0 | Waiting for kick-off |
| WAIT_DESC | 4’h1 | Waiting for descriptor |
| PARSE_DESC | 4’h2 | Parsing descriptor |
| CH_XFER_DATA | 4’h3 | Transfer in progress |
| CHECK_NEXT | 4’h4 | Checking next descriptor |
| COMPLETE | 4’h5 | Transfer complete |
| ERROR | 4’hF | Error state |

## 14.4	Configuration Registers

### 14.4.1	Address Range Validation

RAPIDS validates descriptor addresses against configurable ranges:

Valid if:
  (addr >= cfg_addr0_base && addr < cfg_addr0_limit) ||
  (addr >= cfg_addr1_base && addr < cfg_addr1_limit)

### 14.4.2	Configuration Registers

Configuration and status registers are provided by the rapids_regs register block. See the Register Map for the complete, RTL-accurate address map. The registers most relevant to bring-up are:

Key Configuration Registers

| Register | Offset | Description |
| --- | --- | --- |
| GLOBAL_CTRL | 0x100 | GLOBAL_EN, GLOBAL_RST (self-clearing) |
| CHANNEL_ENABLE | 0x120 | Per-channel enable bitmap |
| SCHED_TIMEOUT_CYCLES | 0x200 | Write-progress timeout window |
| SCHED_TIMEOUT_LIMIT | 0x208 | Consecutive-timeout escalation limit (0 = never) |
| DESCENG_CONFIG | 0x220 | Descriptor-engine enable / prefetch / FIFO threshold |
| DESCENG_ADDR0_BASE/_LIMIT | 0x224/0x228 | Descriptor address range 0 |

## 14.5	Programming Sequence

### 14.5.1	Initialization

// 1. Optional global reset (self-clearing)
GLOBAL_CTRL = 0x2;
while (GLOBAL_CTRL & 0x2);   // wait for GLOBAL_RST to clear

// 2. Configure descriptor address range(s)
DESCENG_ADDR0_BASE  = 0x8000_0000;
DESCENG_ADDR0_LIMIT = 0x9000_0000;

// 3. Scheduler timeout policy
SCHED_TIMEOUT_CYCLES = 1000;
SCHED_TIMEOUT_LIMIT  = 4;    // escalate after 4 stalled windows; 0 = never

// 4. Enable channels and global enable
CHANNEL_ENABLE = 0xFF;       // enable all 8 channels
GLOBAL_CTRL    = 0x1;        // GLOBAL_EN

### 14.5.2	Descriptor Kick-Off

// Prepare descriptor in memory
desc->src_addr = src;
desc->dst_addr = dst;
desc->length = beats;
desc->last = 1;
desc->valid = 1;

// Kick off channel 0
apb_kickoff(0, desc_addr);

// Wait for completion
while (!(channel_idle & 0x01));

// Check for errors
if (channel_error & 0x01) {
    handle_error(0);
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Monitor Bus Interface

## 15.1	Overview

RAPIDS Beats outputs a 64-bit Monitor Bus (MonBus) for real-time event reporting. The MonBus provides visibility into:

- State machine transitions

- Descriptor processing events

- Error conditions

- Performance metrics

## 15.2	MonBus Packet Format

### 15.2.1	64-bit Packet Structure

Bit Field          Width   Description
───────────────────────────────────────────────────────
[63:60]            4-bit   packet_type (error, completion, etc.)
[59:57]            3-bit   protocol (CORE for RAPIDS)
[56:53]            4-bit   event_code (specific event)
[52:47]            6-bit   channel_id
[46:43]            4-bit   unit_id (subsystem)
[42:35]            8-bit   agent_id (module)
[34:0]             35-bit  event_data (event-specific)

### 15.2.2	Signal List

MonBus Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| monbus_pkt_valid | 1 | output | Packet valid |
| monbus_pkt_data | 64 | output | Packet data |
| monbus_pkt_ready | 1 | input | Downstream ready |

## 15.3	Packet Types

MonBus Packet Types

| Type | Code | Description |
| --- | --- | --- |
| ERROR | 4’h0 | Error event |
| COMPLETION | 4’h1 | Transfer/operation complete |
| THRESHOLD | 4’h2 | Threshold crossed |
| TIMEOUT | 4’h3 | Timeout event |
| PERF | 4’h4 | Performance metric |
| DEBUG | 4’hF | Debug/trace event |

## 15.4	Agent IDs

RAPIDS modules use the following Agent IDs:

RAPIDS Agent IDs

| Agent ID | Module | Description |
| --- | --- | --- |
| 0x10-0x17 | Descriptor Engine | Channels 0-7 |
| 0x30-0x37 | Scheduler | Channels 0-7 |
| 0x40-0x47 | Sink Data Path | Channels 0-7 |
| 0x50-0x57 | Source Data Path | Channels 0-7 |

## 15.5	Event Codes

### 15.5.1	Scheduler Events (Agent 0x30-0x37)

Scheduler Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor complete |
| 0x1 | COMPLETION | Chain complete |
| 0x2 | ERROR | Timeout |
| 0x3 | ERROR | AXI error |
| 0x4 | DEBUG | State transition |

### 15.5.2	Descriptor Engine Events (Agent 0x10-0x17)

Descriptor Engine Event Codes

| Event Code | Type | Description |
| --- | --- | --- |
| 0x0 | COMPLETION | Descriptor fetched |
| 0x1 | ERROR | Address range error |
| 0x2 | ERROR | AXI read error |
| 0x3 | DEBUG | Fetch started |

## 15.6	Timing Diagram

MonBus Timing

MonBus Timing

Source: monbus_timing.json

Waveform 15

## 15.7	Event Data Encoding

### 15.7.1	Completion Event Data

[34:32] - Reserved
[31:0]  - Transfer length (beats completed)

### 15.7.2	Error Event Data

[34:32] - Error type
[31:16] - Reserved
[15:0]  - Error details

### 15.7.3	State Transition Event Data

[34:32] - Reserved
[31:28] - Previous state
[27:24] - New state
[23:0]  - Timestamp (lower bits)

## 15.8	MonBus Arbitration

Multiple RAPIDS modules generate MonBus packets. Internal arbitration ensures ordered delivery:

Diagram 13

### 15.8.1	Arbitration Policy

- Round-robin across modules

- No packet loss (backpressure if output blocked)

- Priority boosting for ERROR packets

## 15.9	Top-Level MonBus Delivery (monbus_axil_axil_group)

At the top level (rapids_beats_top), the internal MonBus is not exposed as a raw 64-bit port. Instead, when USE_AXI_MONITORS = 1, packets are combined and delivered through a monbus_axil_axil_group:

- AXI monitors: axi4_master_rd_mon and axi4_master_wr_mon observe the read (m_axi_rd) and write (m_axi_wr) data masters and emit monitor packets. When USE_AXI_MONITORS = 0, these taps are bypassed and the MonBus outputs below are tied off (mon_irq = 0).

- Arbitration: a 3-input monbus_arbiter merges the read-monitor packet, the write-monitor packet, and the core’s descriptor-monitor packet.

- Delivery: the combined stream feeds monbus_axil_axil_group, which presents three external interfaces:

Top-Level MonBus AXI-Lite Group

| Interface | Signals | Purpose |
| --- | --- | --- |
| Error-drain slave | s_axil_err_ar*, s_axil_err_r* (AXI-Lite, 32-bit read) | CPU reads captured error events from the error FIFO |
| Capture master | m_axil_mon_aw*, m_axil_mon_w* (64-bit wdata), m_axil_mon_b* | Bulk-writes the MonBus trace to system memory |
| Interrupt | mon_irq | Asserts on error/threshold events |

The capture region and flush behavior are configured by cfg_mon_base_addr, cfg_mon_limit_addr, and cfg_mon_flush_watermark.

## 15.10	Integration Notes

### 15.10.1	Downstream Connection

Internally, MonBus output connects to:

- MonBus Arbiter - Combines core + rd/wr monitor sources

- monbus_axil_axil_group - Error-drain slave, capture master, and mon_irq

- Debug FIFO - Buffered access for debug tools

### 15.10.2	Packet Rate

MonBus Packet Rates

| Scenario | Approximate Rate |
| --- | --- |
| Idle | 0 packets/cycle |
| Single channel active | ~1 packet/100 cycles |
| All channels active | ~1 packet/20 cycles |
| Error storm | Up to 1 packet/cycle |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Use Case: Network to Memory (Sink Path)

## 16.1	Overview

The Sink Path transfers data from a network interface (AXI-Stream slave) to system memory (AXI4 write master). This is the primary receive data path for network applications.

## 16.2	Operation Flow

Sink Flow

Sink Flow

Source: 09_sink_flow.mmd

Diagram 14

## 16.3	Timing Diagram

Sink Transfer Timing

Sink Transfer Timing

Source: sink_transfer.json

Waveform 16

## 16.4	Buffer Management

### 16.4.1	SRAM Buffering Strategy

The sink path uses internal SRAM to decouple network timing from memory timing:

Diagram 15

### 16.4.2	Flow Control

Sink Path Flow Control

| Condition | Behavior |
| --- | --- |
| SRAM full | Backpressure network (TREADY=0) |
| SRAM empty | Pause AXI writes |
| SRAM threshold | Optional early write start |

Watermark Timing

## 16.5	Performance Considerations

### 16.5.1	Throughput

Sink Path Throughput Factors

| Factor | Impact |
| --- | --- |
| Network bandwidth | Upper bound on receive rate |
| Memory bandwidth | Must match or exceed network |
| SRAM depth | Burst absorption capability |
| AXI outstanding | Overlapped write latency hiding |

### 16.5.2	Latency

Sink Path Latency Breakdown

| Phase | Typical Cycles |
| --- | --- |
| Descriptor fetch | 10-50 (memory dependent) |
| First beat to SRAM | 1 |
| SRAM to AXI write | 2-4 |
| AXI write completion | 10-100 (memory dependent) |

## 16.6	Error Handling

### 16.6.1	Error Sources

Sink Path Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI write error | BRESP != OKAY | Stop transfer, report via MonBus |
| SRAM overflow | Fill pointer catches drain | Should never occur (TREADY control) |
| Timeout | Watchdog timer | Abort transfer, report error |

Error Recovery Sequence

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Use Case: Memory to Network (Source Path)

## 17.1	Overview

The Source Path transfers data from system memory (AXI4 read master) to a network interface (AXI-Stream master). This is the primary transmit data path for network applications.

## 17.2	Operation Flow

Source Flow

Source Flow

Source: 10_source_flow.mmd

Diagram 17

## 17.3	Timing Diagram

Source Transfer Timing

Source Transfer Timing

Source: source_transfer.json

Waveform 18

## 17.4	Buffer Management

### 17.4.1	SRAM Buffering Strategy

The source path uses internal SRAM to prefetch data and maintain network throughput:

Diagram 18

### 17.4.2	Flow Control

Source Path Flow Control

| Condition | Behavior |
| --- | --- |
| SRAM full | Pause AXI reads |
| SRAM empty | Hold TVALID low |
| Network backpressure | Stop draining SRAM |

Prefetch Strategy

## 17.5	Concurrent Operation

### 17.5.1	Read-Ahead Pipeline

The source path can issue AXI reads ahead of network consumption:

Waveform 20

## 17.6	Performance Considerations

### 17.6.1	Throughput

Source Path Throughput Factors

| Factor | Impact |
| --- | --- |
| Memory bandwidth | Upper bound on transmit rate |
| Network acceptance | Must keep pace with reads |
| SRAM depth | Read-ahead capability |
| AXI outstanding | Overlapped read latency hiding |

### 17.6.2	Latency

Source Path Latency Breakdown

| Phase | Typical Cycles |
| --- | --- |
| Descriptor fetch | 10-50 (memory dependent) |
| AXI read issue | 1-2 |
| Memory read latency | 10-100 (memory dependent) |
| SRAM to network | 1-2 |

## 17.7	Error Handling

### 17.7.1	Error Sources

Source Path Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| AXI read error | RRESP != OKAY | Stop transfer, report via MonBus |
| Network timeout | Watchdog timer | Abort transfer, report error |
| Address range | Descriptor validation | Reject descriptor |

Backpressure Handling

When the network deasserts TREADY: 1. SRAM drain stalls immediately 2. SRAM continues filling until threshold 3. AXI reads pause when SRAM approaches full 4. Resume automatically when TREADY returns

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Use Case: Descriptor Chaining

## 18.1	Overview

Descriptor chaining enables multiple transfers to execute sequentially without software intervention. Each descriptor contains a pointer to the next descriptor, forming a linked list that RAPIDS processes automatically.

## 18.2	Chain Structure

Descriptor Chain

Descriptor Chain

Source: 11_descriptor_chain.mmd

Diagram 19

Chain Execution Flow

## 18.3	Timing Diagram

Chain Execution

Chain Execution

Source: chain_execution.json

Waveform 22

## 18.4	Descriptor Fields for Chaining

### 18.4.1	Relevant Fields

Chaining-Related Descriptor Fields

| Field | Bits | Description |
| --- | --- | --- |
| next_ptr | [191:128] | 64-bit pointer to next descriptor |
| last | [255] | Last descriptor in chain flag |
| irq_en | [254] | Generate interrupt on completion |

### 18.4.2	Chain Termination

A chain terminates when ANY of these conditions is true:

- next_ptr == 0 (null pointer)

- last == 1 (explicit last flag)

- Error during transfer

Diagram 21

## 18.5	Prefetch Optimization

### 18.5.1	Descriptor Prefetching

RAPIDS can prefetch the next descriptor while the current transfer executes:

Waveform 23

Benefit: Zero-cycle gap between chained transfers when prefetch completes before current transfer.

Prefetch depth is controlled by DESCENG_CONFIG.PREFETCH_EN and DESCENG_CONFIG.FIFO_THRESH (register block, offset 0x220). With prefetch disabled the descriptor engine fetches on demand (roughly one descriptor ahead of the scheduler); with prefetch enabled it buffers up to FIFO_THRESH descriptors ahead. A chain fetch that is throttled (output FIFO at the threshold) is deferred and reissued automatically once the FIFO drains, so no chain link is lost.

## 18.6	Multi-Channel Chaining

Each channel maintains independent chains:

Diagram 22

Channels execute their chains concurrently and independently.

## 18.7	Error During Chain

### 18.7.1	Error Handling

When an error occurs mid-chain:

Waveform 24

Error Response: 1. Current transfer aborts 2. Chain processing stops 3. Error status captures failing descriptor index 4. MonBus reports error event 5. Interrupt generated (if enabled)

### 18.7.2	Recovery

Software must: 1. Read error status to identify failing descriptor 2. Fix the issue (descriptor content, memory mapping, etc.) 3. Restart chain from failing descriptor or beginning

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Use Case: Multi-Channel Operation

## 19.1	Overview

RAPIDS Beats supports 8 independent DMA channels that can operate concurrently. This enables parallel data transfers for multiple network connections, priority-based scheduling, and efficient resource utilization.

## 19.2	Channel Independence

Multi-Channel Architecture

Multi-Channel Architecture

Source: 12_multi_channel.mmd

Diagram 23

## 19.3	Concurrent Operation Timing

Multi-Channel Timing

Multi-Channel Timing

Source: multi_channel_timing.json

Waveform 25

## 19.4	Resource Arbitration

### 19.4.1	Descriptor Port Arbitration

All channels share a single AXI port for descriptor fetches:

Waveform 26

### 19.4.2	Sink/Source Arbitration

Data path AXI ports use round-robin arbitration with optional priority:

Diagram 24

## 19.5	Channel Isolation

### 19.5.1	Error Isolation

Errors in one channel do not affect other channels:

Waveform 27

### 19.5.2	Per-Channel Soft Reset

Individual channels can be reset without affecting others:

Waveform 28

## 19.6	Use Case Examples

### 19.6.1	Mixed Traffic Pattern

Typical network application with different traffic types:

Mixed Traffic Channel Assignment

| Channel | Traffic Type | Direction | Priority | Notes |
| --- | --- | --- | --- | --- |
| 0 | Voice/RTP | Sink | High | Low latency required |
| 1 | Voice/RTP | Source | High | Low latency required |
| 2 | Video | Sink | Medium | High bandwidth |
| 3 | Control | Source | Medium | Small packets |
| 4-7 | Bulk data | Both | Low | Background transfers |

### 19.6.2	Bidirectional Transfer

Single network connection using paired channels:

Diagram 25

### 19.6.3	Load Balancing

Multiple channels processing same traffic type:

Diagram 26

## 19.7	Performance Considerations

### 19.7.1	Bandwidth Sharing

Bandwidth Distribution

| Active Channels | Per-Channel BW (% of max) | Notes |
| --- | --- | --- |
| 1 | 100% | Full bandwidth available |
| 2 | ~50% each | Round-robin sharing |
| 4 | ~25% each | Fair distribution |
| 8 | ~12.5% each | All channels active |

Note: Actual distribution depends on transfer sizes and memory latency.

### 19.7.2	Latency Impact

Multi-channel operation adds arbitration latency:

Arbitration Latency

| Scenario | Additional Latency |
| --- | --- |
| Single channel | 0 cycles |
| 2 channels contending | 1-2 cycles |
| 8 channels contending | Up to 7 cycles |

## 19.8	MonBus Events

Each channel generates independent MonBus events:

Waveform 29

Events are arbitrated and serialized on the shared MonBus output.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Descriptor Format

## 20.1	Overview

RAPIDS Beats uses 256-bit (32-byte) descriptors to define DMA transfers. Descriptors are stored in system memory and fetched by the Descriptor Engine when a channel is kicked.

## 20.2	Descriptor Layout

Descriptor Format

Descriptor Format

Source: 13_descriptor_format.mmd

### 20.2.1	256-Bit Structure

Bit Range    Field Name        Width   Description
────────────────────────────────────────────────────────────────
[255]        last              1       Last descriptor in chain
[254]        irq_en            1       Generate interrupt on completion
[253:252]    reserved          2       Reserved (write 0)
[251:248]    direction         4       Transfer direction
[247:240]    reserved          8       Reserved (write 0)
[239:224]    transfer_length   16      Transfer length in beats
[223:192]    reserved          32      Reserved (write 0)
[191:128]    next_ptr          64      Pointer to next descriptor
[127:64]     dest_addr         64      Destination address (sink) or source address
[63:0]       src_addr          64      Source address (source path) or buffer addr

Visual Layout

## 20.3	Field Descriptions

### 20.3.1	Control Field [255:248]

Control Field Bits

| Bit | Field | Description |
| --- | --- | --- |
| 255 | last | Set to 1 for last descriptor in chain |
| 254 | irq_en | Set to 1 to generate completion interrupt |
| 253:252 | Reserved | Write as 0 |
| 251:248 | direction | Transfer direction encoding |

### 20.3.2	Direction Encoding

Direction Field Encoding

| Value | Name | Description |
| --- | --- | --- |
| 4’h0 | SINK | Network to memory (AXIS slave to AXI write) |
| 4’h1 | SOURCE | Memory to network (AXI read to AXIS master) |
| 4’h2-F | Reserved | Reserved for future use |

### 20.3.3	Transfer Length [239:224]

- 16-bit field specifying transfer size in beats

- 1 beat = DATA_WIDTH bits (default 512 bits = 64 bytes)

- Maximum transfer: 65,535 beats (4 MB at 64 bytes/beat)

- Length of 0 is reserved (no operation)

### 20.3.4	Next Pointer [191:128]

- 64-bit byte address of next descriptor

- Must be 32-byte aligned (bits [4:0] ignored)

- Value of 0 terminates chain (regardless of last bit)

### 20.3.5	Address Fields [127:0]

For SINK transfers (network to memory): | Field | Usage | |——-|——-| | dest_addr [127:64] | Memory write destination address | | src_addr [63:0] | Not used (write 0) |

For SOURCE transfers (memory to network): | Field | Usage | |——-|——-| | dest_addr [127:64] | Not used (write 0) | | src_addr [63:0] | Memory read source address |

: Address Field Usage

## 20.4	Descriptor Opcodes (DATA / CTRL_READ / CTRL_WRITE)

Beyond plain DATA transfers, a descriptor carries a 2-bit opcode (rapids_pkg desc_op_e) that selects one of three behaviors. DATA descriptors move payload through the concurrent read/write engines (above); the two control opcodes let a channel synchronize with a producer/consumer in memory without moving payload – the basis of the RAPIDS producer/consumer flow.

| Opcode | Value | Behavior |
| --- | --- | --- |
| DATA | 2'b00 | Concurrent read/write payload transfer (fields as above) |
| CTRL_READ | 2'b01 | Consumer gate – poll a memory location until (read & mask) == expected, then release the descriptor/chain |
| CTRL_WRITE | 2'b10 | Producer doorbell – single 32-bit write of a value to a memory location, then continue |

CTRL_READ (consumer gate) re-interprets the descriptor fields as a poll specification. The scheduler drives the per-channel control-read engine, which reads the gate address once per retry and compares the masked value; the descriptor completes when the gate is satisfied. The retry budget is bounded by CTRL_CONFIG.CTRLRD_MAX_TRY (0-511, reset 16) so a never-satisfied gate cannot hang the channel – exhaustion raises an error instead.

| Field | Bits | Usage |
| --- | --- | --- |
| poll_addr | [63:0] | Gate address to poll |
| expected | [95:64] | Expected value after masking |
| mask | [127:96] | Compare mask ((read & mask) == expected) |
| max_try | [143:128] | Per-descriptor retry hint (capped by CTRLRD_MAX_TRY) |

CTRL_WRITE (producer doorbell) performs one single-beat 32-bit AXI write, after which the chain continues. It has no poll/retry – it is an unconditional write used to signal a consumer.

| Field | Bits | Usage |
| --- | --- | --- |
| wr_addr | [63:0] | Doorbell address |
| wr_data | [95:64] | 32-bit value to write |

The control opcodes are handled by dedicated engines (control-read / control-write), one pair per channel, driven by the scheduler’s control interface. See the MAS Scheduler (Section 2.1) and Control-Read / Control-Write Engine specifications (Sections 2.8 / 2.9).

## 20.5	Descriptor Fetch Timing

Descriptor Fetch

Descriptor Fetch

Source: descriptor_fetch.json

Waveform 30

## 20.6	Alignment Requirements

Address Alignment Requirements

| Field | Alignment | Notes |
| --- | --- | --- |
| Descriptor address | 32-byte | Bits [4:0] ignored |
| next_ptr | 32-byte | Bits [4:0] ignored |
| dest_addr | DATA_WIDTH/8 | 64-byte for 512-bit data |
| src_addr | DATA_WIDTH/8 | 64-byte for 512-bit data |

Note: Unaligned addresses may cause unpredictable behavior or AXI protocol violations.

## 20.7	Descriptor Examples

### 20.7.1	Sink Descriptor (Network to Memory)

Hex representation (LSB to MSB):
[63:0]    = 0x0000_0000_0000_0000  // src_addr (unused)
[127:64]  = 0x0000_0001_0000_0000  // dest_addr = 0x1_0000_0000
[191:128] = 0x0000_0000_0000_2000  // next_ptr = 0x2000
[255:192] = 0xC000_0100_0000_0000  // last=1, irq=1, dir=SINK, len=256

Binary control field:
  [255] = 1      // Last descriptor
  [254] = 1      // Interrupt enabled
  [253:252] = 00 // Reserved
  [251:248] = 0000 // Direction = SINK

### 20.7.2	Source Descriptor (Memory to Network)

Hex representation (LSB to MSB):
[63:0]    = 0x0000_0002_0000_0000  // src_addr = 0x2_0000_0000
[127:64]  = 0x0000_0000_0000_0000  // dest_addr (unused)
[191:128] = 0x0000_0000_0000_0000  // next_ptr = 0 (last)
[255:192] = 0x4001_0080_0000_0000  // last=0, irq=1, dir=SOURCE, len=128

Binary control field:
  [255] = 0      // Not last (but next_ptr=0 terminates)
  [254] = 1      // Interrupt enabled
  [253:252] = 00 // Reserved
  [251:248] = 0001 // Direction = SOURCE

## 20.8	Software Construction

### 20.8.1	C Structure Example

typedef struct __attribute__((packed, aligned(32))) {
    uint64_t src_addr;      // [63:0]
    uint64_t dest_addr;     // [127:64]
    uint64_t next_ptr;      // [191:128]
    uint16_t reserved1;     // [207:192]
    uint16_t length;        // [223:208] - Note: shifted in actual layout
    uint8_t  reserved2;     // [231:224]
    uint8_t  direction;     // [239:232] - lower 4 bits only
    uint8_t  reserved3;     // [247:240]
    uint8_t  control;       // [255:248] - last, irq_en, etc.
} rapids_descriptor_t;

// Helper macros
#define DESC_CTRL_LAST    (1 << 7)
#define DESC_CTRL_IRQ_EN  (1 << 6)
#define DESC_DIR_SINK     0x0
#define DESC_DIR_SOURCE   0x1

### 20.8.2	Descriptor Ring Setup

// Allocate descriptor ring (must be 32-byte aligned)
rapids_descriptor_t *desc_ring = aligned_alloc(32, NUM_DESC * sizeof(rapids_descriptor_t));

// Initialize chain
for (int i = 0; i < NUM_DESC - 1; i++) {
    desc_ring[i].next_ptr = (uint64_t)&desc_ring[i + 1];
    desc_ring[i].control = DESC_CTRL_IRQ_EN;
}

// Last descriptor
desc_ring[NUM_DESC - 1].next_ptr = 0;
desc_ring[NUM_DESC - 1].control = DESC_CTRL_LAST | DESC_CTRL_IRQ_EN;

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Register Map

## 21.1	Overview

RAPIDS Beats provides a memory-mapped register interface for configuration and status. All registers are accessed through the single top-level APB slave (s_apb_*) and are implemented by the PeakRDL-generated rapids_regs register block. A single addrmap (one APB slave) contains a base regfile at 0x100-0x3FF and a nested monitor regfile (rapids_mon_regs) at 0x1000. Descriptor kick-off uses a separate address range (0x000-0x03F) routed to apb4todescr.

Because the monitor regfile lives at 0x1000, the APB address bus must be at least 13 bits wide.

## 21.2	Address Space Layout

Address Space Layout

| Range | Target | Purpose |
| --- | --- | --- |
| 0x000-0x03F | apb4todescr | Per-channel descriptor kick-off |
| 0x100-0x3FF | rapids_regs base regfile | Configuration and status |
| 0x1000+ | rapids_regs monitor regfile | AXI-monitor config and performance |

## 21.3	Base Registers (0x100-0x3FF)

Base Registers

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | RW | GLOBAL_EN[0], GLOBAL_RST[1] (self-clearing) |
| 0x104 | GLOBAL_STATUS | RO | SYSTEM_IDLE[0] |
| 0x108 | VERSION | RO | MINOR[7:0], MAJOR[15:8], NUM_CHANNELS[23:16]=8 |
| 0x120 | CHANNEL_ENABLE | RW | CH_EN[7:0] per-channel enable |
| 0x124 | CHANNEL_RESET | RW | CH_RST[7:0] per-channel reset (self-clearing) |
| 0x140 | CHANNEL_IDLE | RO | CH_IDLE[7:0] |
| 0x144 | DESC_ENGINE_IDLE | RO | Per-channel descriptor-engine idle |
| 0x148 | SCHEDULER_IDLE | RO | Per-channel scheduler idle (excludes CH_ERROR) |
| 0x150-0x16C | CH_STATE[0..7] | RO | Per-channel FSM state (stride 0x4) |
| 0x170 | SCHED_ERROR | RO | Per-channel sticky scheduler error |
| 0x174 | AXI_RD_COMPLETE | RO | Per-channel read-complete |
| 0x178 | AXI_WR_COMPLETE | RO | Per-channel write-complete |
| 0x200 | SCHED_TIMEOUT_CYCLES | RW | Write-progress timeout window (cycles, reset 1000) |
| 0x204 | SCHED_CONFIG | RW | SCHED_EN, TIMEOUT_EN, ERR_EN, COMPL_EN, PERF_EN |
| 0x208 | SCHED_TIMEOUT_LIMIT | RW | LIMIT[7:0] escalation limit (reset 4, 0 = never) |
| 0x220 | DESCENG_CONFIG | RW | DESCENG_EN[0], PREFETCH_EN[1], FIFO_THRESH[5:2] (reset 8) |
| 0x224 | DESCENG_ADDR0_BASE | RW | Descriptor address range 0 base [31:0] |
| 0x228 | DESCENG_ADDR0_LIMIT | RW | Descriptor address range 0 limit [31:0] |
| 0x22C | DESCENG_ADDR1_BASE | RW | Descriptor address range 1 base [31:0] |
| 0x230 | DESCENG_ADDR1_LIMIT | RW | Descriptor address range 1 limit [31:0] |
| 0x240 | CTRL_CONFIG | RW | CTRLRD_MAX_TRY[8:0] control-read poll retry budget (0-511, reset 16) |
| 0x2A0 | AXI_XFER_CONFIG | RW | AXI transfer sizing configuration |
| 0x2B0 | PERF_CONFIG | RW | Performance profiler configuration |
| 0x2C0-0x2CC | OBS_CTRL/OBS_FLAGS/OBS_DATA0/OBS_DATA1 | RW/RO | Observation mux |
| 0x35C | PERF_CH_SEL | RW | Performance channel select |
| 0x378-0x380 | HIST_SEL/HIST_DATA/HIST_TOTAL | RW/RO | Latency histogram |

## 21.4	Monitor Registers (0x1000)

Monitor Registers (base 0x1000)

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x1000 | MON_FIFO_STATUS | RO | MonBus capture/error FIFO status |
| 0x1004 | MON_FIFO_COUNT | RO | MonBus FIFO occupancy |
| 0x10C0-0x10DC | DAXMON_* | RW | Descriptor-monitor config (enable/timeout/latency/masks) |
| 0x10E0-0x10FC | RDMON_* | RW | Read-monitor config (same layout) |
| 0x1100-0x111C | WRMON_* | RW | Write-monitor config (same layout, incl. COMPRESS_EN) |
| 0x1150-0x1178 | DAXMON_PERF_* | RO/RW | Descriptor-monitor performance counters |
| 0x1180-0x11A8 | RDMON_PERF_* | RO/RW | Read-monitor performance counters |
| 0x11B0-0x11D8 | WRMON_PERF_* | RO/RW | Write-monitor performance counters |
| 0x11E0-0x11F4 | {RD,WR}MON_PERF_CH_* | RO | Per-channel producer/backpressure/starve/idle/overflow |

## 21.5	Key Register Fields

### 21.5.1	GLOBAL_CTRL (0x100) - Read/Write

GLOBAL_CTRL Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [0] | GLOBAL_EN | 0 | Master enable for the entire engine |
| [1] | GLOBAL_RST | 0 | Global reset (self-clearing) |
| [31:2] | Reserved | 0 | Reserved |

### 21.5.2	SCHED_TIMEOUT_LIMIT (0x208) - Read/Write

SCHED_TIMEOUT_LIMIT Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [7:0] | LIMIT | 4 | Consecutive write-progress timeout windows before escalating to a fatal, sticky CH_ERROR. 0 = never escalate. Total escalation time ~= LIMIT x SCHED_TIMEOUT_CYCLES. |

### 21.5.3	DESCENG_CONFIG (0x220) - Read/Write

DESCENG_CONFIG Register

| Bits | Field | Reset | Description |
| --- | --- | --- | --- |
| [0] | DESCENG_EN | 1 | Descriptor engine master enable |
| [1] | PREFETCH_EN | 0 | Prefetch chaining (0 = on-demand, ~1 ahead) |
| [5:2] | FIFO_THRESH | 8 | Descriptors buffered ahead when prefetch enabled |
| [31:6] | Reserved | 0 | Reserved |

## 21.6	Register Access Timing

Register Read

Register Read

Source: register_read.json

Waveform 31

Register Write

Register Write

Source: register_write.json

Waveform 32

## 21.7	Channel Kick Sequence

Writing to the kick bit triggers descriptor processing:

Waveform 33

## 21.8	Address Decoding

Registers are global (not per-channel windows); per-channel fields are packed as bit lanes within a single 32-bit register (e.g. CHANNEL_ENABLE.CH_EN[7:0], CHANNEL_IDLE.CH_IDLE[7:0]). The only per-channel array is CH_STATE[0..7] at 0x150-0x16C (stride 0x4). Descriptor kick-off is a separate address range (0x000-0x03F) handled by apb4todescr, not part of the register regfile.

0x000 - 0x03F: Descriptor kick-off (apb4todescr)
0x100 - 0x3FF: Base configuration / status registers
0x1000+      : Monitor configuration / performance registers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Initialization Sequence

## 22.1	Overview

This section describes the required steps to initialize RAPIDS Beats and start DMA transfers. The initialization sequence ensures proper hardware state before beginning operations.

## 22.2	Boot Sequence

Initialization Flow

Initialization Flow

Source: 14_init_flow.mmd

Diagram 28

## 22.3	Detailed Steps

### 22.3.1	Step 1: Verify Hardware Presence

// Read VERSION register
uint32_t version = read_reg(RAPIDS_BASE + VERSION);
uint8_t major = (version >> 24) & 0xFF;
uint8_t minor = (version >> 16) & 0xFF;

if (major < 1) {
    printf("ERROR: RAPIDS not detected or incompatible version\n");
    return -1;
}

printf("RAPIDS Beats v%d.%d detected\n", major, minor);

### 22.3.2	Step 2: Read Configuration

// Read CONFIG register
uint32_t config = read_reg(RAPIDS_BASE + CONFIG);
uint8_t num_channels = (config >> 24) & 0xFF;
uint8_t data_width = ((config >> 16) & 0xFF) * 8;  // Convert to bits
uint8_t sram_depth = ((config >> 8) & 0xFF) * 16;

printf("Channels: %d, Data Width: %d bits, SRAM: %d entries\n",
       num_channels, data_width, sram_depth);

### 22.3.3	Step 3: Global Reset

// Perform soft reset to ensure clean state
write_reg(RAPIDS_BASE + GLOBAL_CTRL, GLOBAL_CTRL_SOFT_RESET);

// Wait for reset completion
while (read_reg(RAPIDS_BASE + GLOBAL_STATUS) & GLOBAL_STATUS_RESET_ACTIVE) {
    // Poll until reset complete
}

// Clear any pending interrupts
write_reg(RAPIDS_BASE + IRQ_STATUS, 0xFFFFFFFF);

### 22.3.4	Step 4: Allocate Resources

// Allocate descriptor ring (must be 32-byte aligned)
size_t desc_size = NUM_DESCRIPTORS * sizeof(rapids_descriptor_t);
rapids_descriptor_t *desc_ring = aligned_alloc(32, desc_size);
if (!desc_ring) {
    return -ENOMEM;
}

// Allocate data buffers (must be 64-byte aligned for 512-bit AXI)
size_t buf_size = BUFFER_SIZE_BYTES;
void *rx_buffer = aligned_alloc(64, buf_size);
void *tx_buffer = aligned_alloc(64, buf_size);

### 22.3.5	Step 5: Initialize Descriptors

// Initialize sink descriptor chain
for (int i = 0; i < NUM_RX_DESC - 1; i++) {
    desc_ring[i].dest_addr = (uint64_t)(rx_buffer + i * FRAME_SIZE);
    desc_ring[i].src_addr = 0;
    desc_ring[i].next_ptr = (uint64_t)&desc_ring[i + 1];
    desc_ring[i].direction = DESC_DIR_SINK;
    desc_ring[i].length = FRAME_SIZE / 64;  // Convert to beats
    desc_ring[i].control = DESC_CTRL_IRQ_EN;
}

// Last descriptor
desc_ring[NUM_RX_DESC - 1].dest_addr = (uint64_t)(rx_buffer + (NUM_RX_DESC-1) * FRAME_SIZE);
desc_ring[NUM_RX_DESC - 1].next_ptr = 0;
desc_ring[NUM_RX_DESC - 1].control = DESC_CTRL_LAST | DESC_CTRL_IRQ_EN;

// Ensure descriptors are visible to hardware
__sync_synchronize();  // Memory barrier

### 22.3.6	Step 6: Configure Channels

// Configure channel 0 for sink (receive)
uint32_t ch0_base = RAPIDS_BASE + 0x100;

// Set descriptor pointer
write_reg(ch0_base + DESC_PTR_LO, (uint32_t)((uint64_t)desc_ring & 0xFFFFFFFF));
write_reg(ch0_base + DESC_PTR_HI, (uint32_t)((uint64_t)desc_ring >> 32));

// Configure channel
write_reg(ch0_base + CH_CONFIG, CH_CONFIG_DEFAULT);

### 22.3.7	Step 7: Enable Interrupts

// Enable completion and error interrupts for channel 0
write_reg(RAPIDS_BASE + IRQ_ENABLE, IRQ_CH0_COMPLETE | IRQ_CH0_ERROR);

### 22.3.8	Step 8: Enable Channel

// Enable channel (but don't kick yet)
write_reg(ch0_base + CH_CTRL, CH_CTRL_ENABLE);

## 22.4	Initialization Timing

Initialization Timing

Initialization Timing

Source: init_timing.json

Waveform 34

## 22.5	Starting Transfers

### 22.5.1	Kick a Channel

After initialization, kick the channel to start processing:

// Kick channel 0 to start descriptor processing
write_reg(ch0_base + CH_CTRL, CH_CTRL_ENABLE | CH_CTRL_KICK);

Kick Timing

Kick Timing

Waveform 35

## 22.6	Interrupt Handling

### 22.6.1	Completion Interrupt

void rapids_irq_handler(void) {
    uint32_t irq_status = read_reg(RAPIDS_BASE + IRQ_STATUS);

    // Handle channel 0 completion
    if (irq_status & IRQ_CH0_COMPLETE) {
        // Read channel status
        uint32_t ch_status = read_reg(ch0_base + CH_STATUS);
        uint8_t desc_count = (ch_status >> 16) & 0xFF;

        // Process completed buffers
        process_rx_buffers(desc_count);

        // Clear interrupt
        write_reg(RAPIDS_BASE + IRQ_STATUS, IRQ_CH0_COMPLETE);

        // Re-kick channel for continuous operation (if desired)
        write_reg(ch0_base + CH_CTRL, CH_CTRL_ENABLE | CH_CTRL_KICK);
    }

    // Handle errors
    if (irq_status & IRQ_CH0_ERROR) {
        uint32_t err_status = read_reg(ch0_base + ERR_STATUS);
        handle_error(err_status);

        // Clear error interrupt
        write_reg(RAPIDS_BASE + IRQ_STATUS, IRQ_CH0_ERROR);
    }
}

## 22.7	Reset and Recovery

### 22.7.1	Per-Channel Reset

void reset_channel(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);

    // Soft reset the channel
    write_reg(ch_base + CH_CTRL, CH_CTRL_SOFT_RESET);

    // Wait for reset complete
    while ((read_reg(ch_base + CH_STATUS) >> 28) == 0xF) {
        // State = RESET
    }

    // Clear error status
    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);

    // Re-initialize descriptor pointer
    write_reg(ch_base + DESC_PTR_LO, saved_desc_ptr_lo[channel]);
    write_reg(ch_base + DESC_PTR_HI, saved_desc_ptr_hi[channel]);

    // Re-enable
    write_reg(ch_base + CH_CTRL, CH_CTRL_ENABLE);
}

### 22.7.2	Global Reset

void reset_rapids(void) {
    // Disable all interrupts
    write_reg(RAPIDS_BASE + IRQ_ENABLE, 0);

    // Global soft reset
    write_reg(RAPIDS_BASE + GLOBAL_CTRL, GLOBAL_CTRL_SOFT_RESET);

    // Wait for all channels idle
    while (read_reg(RAPIDS_BASE + GLOBAL_STATUS) & 0x00FF0000) {
        // Wait for active channels to clear
    }

    // Clear all interrupts
    write_reg(RAPIDS_BASE + IRQ_STATUS, 0xFFFFFFFF);

    // Re-run full initialization
    rapids_init();
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Error Handling

## 23.1	Overview

RAPIDS Beats implements comprehensive error detection and reporting. This section describes error types, detection mechanisms, and recommended recovery procedures.

## 23.2	Error Categories

Error Hierarchy

Error Hierarchy

Source: 15_error_hierarchy.mmd

Diagram 29

## 23.3	Error Detection

### 23.3.1	AXI Response Errors

RAPIDS monitors all AXI transactions for error responses:

AXI Response Codes

| Response | Code | Description | Typical Cause |
| --- | --- | --- | --- |
| OKAY | 2’b00 | Normal success | – |
| EXOKAY | 2’b01 | Exclusive OK | – |
| SLVERR | 2’b10 | Slave error | Memory protection, invalid access |
| DECERR | 2’b11 | Decode error | Address not mapped |

AXI Error Detection

AXI Error Detection

Source: axi_error.json

Waveform 36

### 23.3.2	Descriptor Validation Errors

Descriptors are validated when parsed:

Descriptor Validation Errors

| Check | Error Condition | ERR_STATUS Code |
| --- | --- | --- |
| Address range | Address outside configured range | 0x10 |
| Alignment | Address not properly aligned | 0x11 |
| Length | Length = 0 or > max allowed | 0x12 |
| Direction | Invalid direction code | 0x13 |

Waveform 37

### 23.3.3	Timeout Detection

RAPIDS implements watchdog timers for detecting stalled operations:

Timeout Thresholds

| Timeout Type | Typical Value | Condition |
| --- | --- | --- |
| Descriptor fetch | 1024 cycles | No AXI response |
| Transfer | 65536 cycles | No progress |
| AXIS receive | 4096 cycles | TVALID stuck low |
| AXIS transmit | 4096 cycles | TREADY stuck low |

Recoverable write-progress timeout. The scheduler’s write-progress watchdog is recoverable: each expired window (SCHED_TIMEOUT_CYCLES) increments a strike counter, and the channel only escalates to a fatal, sticky CH_ERROR after SCHED_TIMEOUT_LIMIT (register 0x208) consecutive stalled windows. Any write progress – an AW issue or a B-response commit – clears the strike counter. A limit of 0 disables escalation entirely (soft timeout). A faulted channel in CH_ERROR is reported via SCHED_ERROR and is deliberately excluded from SCHEDULER_IDLE.

Waveform 38

## 23.4	Error Reporting

### 23.4.1	ERR_STATUS Register

When an error occurs, details are captured in the ERR_STATUS register:

ERR_STATUS [31:0]:
  [31:28] - AXI response code (BRESP or RRESP)
  [27:24] - Error type:
            0x0 = AXI write error
            0x1 = AXI read error
            0x2 = Descriptor fetch error
            0x3 = Descriptor validation error
            0x4 = Timeout error
  [23:16] - Descriptor index when error occurred
  [15:0]  - Reserved

### 23.4.2	MonBus Error Events

Errors generate MonBus packets for system-wide visibility:

Waveform 39

### 23.4.3	Interrupt Generation

Error interrupts are generated when enabled:

Diagram 30

## 23.5	Error Recovery

### 23.5.1	Recovery Flow

Error Recovery

Error Recovery

Source: 16_error_recovery.mmd

Diagram 31

### 23.5.2	AXI Error Recovery

void handle_axi_error(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);
    uint32_t err_status = read_reg(ch_base + ERR_STATUS);

    uint8_t axi_resp = (err_status >> 28) & 0xF;
    uint8_t err_type = (err_status >> 24) & 0xF;
    uint8_t desc_idx = (err_status >> 16) & 0xFF;

    if (axi_resp == 0x3) {  // DECERR
        printf("CH%d: Address decode error at descriptor %d\n", channel, desc_idx);
        // Check memory mapping, descriptor addresses
    } else if (axi_resp == 0x2) {  // SLVERR
        printf("CH%d: Slave error at descriptor %d\n", channel, desc_idx);
        // Check memory protection, peripheral status
    }

    // Clear error and reset channel
    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);
    reset_channel(channel);
}

### 23.5.3	Descriptor Error Recovery

void handle_desc_error(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);
    uint32_t err_status = read_reg(ch_base + ERR_STATUS);

    uint8_t err_type = (err_status >> 24) & 0xF;
    uint8_t desc_idx = (err_status >> 16) & 0xFF;

    switch (err_type & 0xF) {
        case 0x0:  // Address range
            printf("CH%d: Descriptor %d has invalid address\n", channel, desc_idx);
            fix_descriptor_address(channel, desc_idx);
            break;
        case 0x1:  // Alignment
            printf("CH%d: Descriptor %d has alignment issue\n", channel, desc_idx);
            fix_descriptor_alignment(channel, desc_idx);
            break;
        case 0x2:  // Length
            printf("CH%d: Descriptor %d has invalid length\n", channel, desc_idx);
            fix_descriptor_length(channel, desc_idx);
            break;
    }

    // Clear and restart from fixed descriptor
    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);
    write_reg(ch_base + DESC_PTR_LO, (uint32_t)&desc_ring[desc_idx]);
    write_reg(ch_base + DESC_PTR_HI, (uint32_t)((uint64_t)&desc_ring[desc_idx] >> 32));
    reset_channel(channel);
}

### 23.5.4	Timeout Recovery

void handle_timeout(int channel) {
    uint32_t ch_base = RAPIDS_BASE + 0x100 + (channel * 0x40);
    uint32_t err_status = read_reg(ch_base + ERR_STATUS);
    uint32_t ch_status = read_reg(ch_base + CH_STATUS);

    uint8_t desc_idx = (err_status >> 16) & 0xFF;
    uint8_t sram_level = (ch_status >> 8) & 0xFF;

    printf("CH%d: Timeout at descriptor %d, SRAM level %d\n",
           channel, desc_idx, sram_level);

    // Check external interfaces
    if (sram_level == 0) {
        printf("  SRAM empty - check network input\n");
    } else if (sram_level > 0) {
        printf("  SRAM has data - check memory interface\n");
    }

    // Abort and reset
    write_reg(ch_base + CH_CTRL, CH_CTRL_ABORT);
    while ((read_reg(ch_base + CH_STATUS) >> 28) != 0x0) {
        // Wait for IDLE
    }

    write_reg(ch_base + ERR_STATUS, 0xFFFFFFFF);
    reset_channel(channel);
}

## 23.6	Error Prevention

### 23.6.1	Best Practices

Error Prevention Best Practices

| Practice | Rationale |
| --- | --- |
| Validate descriptors before submission | Catch errors before hardware processing |
| Use aligned addresses | Avoid alignment errors |
| Set reasonable timeouts | Detect issues without false positives |
| Monitor SRAM levels | Identify flow control issues |
| Enable MonBus | System-wide error visibility |

### 23.6.2	Descriptor Validation Example

int validate_descriptor(rapids_descriptor_t *desc) {
    // Check alignment
    if (desc->dest_addr & 0x3F) {  // 64-byte alignment
        return ERR_ALIGNMENT;
    }

    // Check length
    if (desc->length == 0 || desc->length > MAX_BEATS) {
        return ERR_LENGTH;
    }

    // Check direction
    if (desc->direction > DESC_DIR_SOURCE) {
        return ERR_DIRECTION;
    }

    // Check address range
    if (desc->dest_addr < VALID_ADDR_MIN || desc->dest_addr > VALID_ADDR_MAX) {
        return ERR_ADDRESS;
    }

    return 0;  // Valid
}

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Throughput Targets

RAPIDS Beats is a network-to-memory accelerator: the sink path moves an AXIS ingress stream into system memory, and the source path reads system memory and drives an AXIS egress stream. The two paths are independent engines (with their own SRAM buffers, schedulers, and AXI masters), so a sink transfer and a source transfer run concurrently and their bandwidths add.

## 24.1	Theoretical Maximum

A beat is DATA_WIDTH = 512 bits = 64 bytes. Per-direction bandwidth is DATA_WIDTH x f_aclk / 8.

### 24.1.1	Per-Interface Bandwidth

| Interface | Data Width | Frequency | Bandwidth |
| --- | --- | --- | --- |
| Descriptor Fetch | 256 bits | 100 MHz | 3.2 GB/s |
| Sink AXI Write (to memory) | 512 bits | 100 MHz | 6.4 GB/s |
| Source AXI Read (from memory) | 512 bits | 100 MHz | 6.4 GB/s |
| AXIS ingress / egress (network) | 512 bits/beat | 100 MHz | 6.4 GB/s |

The table uses the Nexys A7 characterization clock (CLK100MHZ, 100 MHz). The design targets 100-500 MHz; scale linearly (e.g. 12.8 GB/s per direction at 200 MHz, 32 GB/s at 500 MHz).

### 24.1.2	Aggregate Bandwidth

- Concurrent sink + source: 12.8 GB/s at 100 MHz (6.4 sink + 6.4 source), since the two directions are separate engines against separate AXI masters.

- Descriptor Overhead: ~0.1% for large transfers; the descriptor engine prefetches (DESCENG_CONFIG.PREFETCH_EN) so chained fetches overlap the current transfer.

## 24.2	Practical Throughput

A transfer is bounded by the slower of its two ends: a sink transfer by min(AXIS ingress, AXI write), a source transfer by min(AXI read, AXIS egress). The SRAM buffer decouples the two ends so a transient stall on one side does not immediately stall the other.

### 24.2.1	Single-Channel Performance

| Scenario | Expected Efficiency | Effective Bandwidth (100 MHz) |
| --- | --- | --- |
| Large sequential (>1MB) | >95% | >6.1 GB/s per direction |
| Medium sequential (64KB-1MB) | 85-95% | 5.4-6.1 GB/s |
| Small sequential (4KB-64KB) | 70-85% | 4.5-5.4 GB/s |
| Very small (<4KB) | 40-70% | 2.6-4.5 GB/s |

### 24.2.2	Multi-Channel Performance

All channels of one direction share that direction’s AXI master, so aggregate is bounded by the shared master while per-channel throughput scales inversely with active channel count.

| Active Channels (per direction) | Per-Channel BW | Aggregate BW (per direction) |
| --- | --- | --- |
| 1 | 6.4 GB/s | 6.4 GB/s |
| 2 | 3.2 GB/s | 6.4 GB/s |
| 4 | 1.6 GB/s | 6.4 GB/s |
| 8 | 0.8 GB/s | 6.4 GB/s |

Note: figures are at 100 MHz for a single direction. Sink and source run concurrently, so full-duplex aggregate is twice the per-direction column.

## 24.3	Throughput Limiting Factors

### 24.3.1	Memory / Network System Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Memory latency | Reduces efficiency for small transfers | Deeper outstanding / larger SRAM |
| Memory or network bandwidth | Hard limit on the slower end | Match RAPIDS to the memory/network capability |
| AXIS backpressure | Stalls the network end (s_axis_tready/m_axis_tready) | SRAM buffering absorbs transient stalls |
| Interconnect contention | Variable impact | Priority / channel configuration |

### 24.3.2	RAPIDS Internal Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Descriptor fetch overhead | Fixed per descriptor | Longer transfers; descriptor prefetch (PREFETCH_EN) |
| SRAM depth | Limits outstanding beats | Configure adequate SRAM_DEPTH |
| Arbitration overhead | Multi-channel penalty on the shared AXI master | Reduce active channels per direction |
| AXI transfer sizing | Small bursts lose efficiency | AXI_XFER_CONFIG RD/WR beats, ALLOC/DRAIN sizing |
| Control descriptors | CTRL_READ/CTRL_WRITE are synchronization, not payload | Use only for producer/consumer handshakes; see below |

Control descriptors do not carry throughput. A CTRL_READ (consumer gate) polls a memory location until it matches; a CTRL_WRITE (producer doorbell) issues one 32-bit write. They synchronize a channel with a producer/consumer and occupy the channel while active, but move no payload – budget them as latency, not bandwidth (see Latency, Section 6.2).

## 24.4	Performance Targets

### 24.4.1	Design Targets

| Metric | Target | Condition |
| --- | --- | --- |
| Peak throughput (per direction) | 6.4 GB/s | Single channel, 100 MHz, ideal |
| Full-duplex aggregate | 12.8 GB/s | Sink + source concurrent, 100 MHz |
| Sustained throughput | >6.1 GB/s | Large transfers, single channel |
| Small transfer efficiency | >50% | 4KB transfers |

### 24.4.2	Verification Criteria

| Scenario | Pass Criteria |
| --- | --- |
| Sink stream (large) | >95% of AXI write bandwidth |
| Source stream (large) | >95% of AXI read bandwidth |
| Multi-channel (per direction) | Aggregate within 5% of the shared master limit |
| Data integrity (all cases) | wr_crc/rd_crc/chk_crc == golden model |

## 24.5	On-Silicon Validation

The characterization harness (rapids_char_top on the Nexys A7-100T, 100 MHz) validates data integrity end to end via golden-CRC self-checks on both paths: sink wr_crc == golden, source rd_crc == chk_crc == golden. The current harness measures correctness and beat counts.

Per-direction bus utilization uses the same instrument the STREAM char does: the shared axi4_intf_master_observer (projects/components/misc/rtl/) dropped inline on the harness AXI masters, auto-windowed in hardware, with aggregate PROD/BP/STARV/ IDLE buckets + beat/byte/burst counts surfaced at harness CSR 0x100-0x11C and read verbatim by read_bus_meters.py. RAPIDS maps to it cleanly – a read tap on the source master and a write tap on the sink master give a true per-direction split (STREAM’s shared master is aggregate-only). Wiring the observer into rapids_char_harness is the remaining step to report measured GB/s here.

As of 2026-08-05 the observer carries its OWN APB config regblock (obs_regs) instead of exporting 29 cfg_* ports for the instantiating harness to tie off, and it moved to projects/components/misc/rtl/ so any board flow can reach it:

-f $MISC_ROOT/rtl/filelists/axi4_intf_master_observer.f

It was renamed from axi4_dma_observer at the same time – the header always said “DMA-agnostic”, and the DMA in the name read wrong for a block a memory controller or a beats DMA would share. dma_slave_monitors (the monitored slave wrapper, also used here) sits alongside it on the same terms.

Latest on-silicon result (full characterization suite): 48 / 48 configurations pass across channels {1, 2, 4} x beats {1, 4, 8, 16} x backpressure {off, on} x seeds {default, alternate}, sink and source, all golden-validated – including under injected AXIS backpressure.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Latency Characteristics

Latency is dominated by the one-time pipeline fill from a channel kick to the first data beat. Once running, both paths sustain the throughput of Section 6.1; control descriptors add a bounded synchronization delay.

## 25.1	Latency Components

### 25.1.1	Kick-to-First-Beat Breakdown

| Stage | Typical Cycles (100 MHz) | Notes |
| --- | --- | --- |
| Kick decode (APB / kick window) | 2-4 | Descriptor address latched |
| Descriptor fetch (AXI read, 256-bit) | AXI read latency + 1 | Overlapped by prefetch on chains |
| Descriptor parse | 1-2 | Opcode + fields decoded |
| SRAM fill (sink) / AXI read issue (source) | AXI/AXIS latency | First beat into the buffer |
| First beat out | 1-2 | Drain to memory (sink) / AXIS (source) |

### 25.1.2	Total Latency Formula

L_first_beat  =  L_kick + L_desc_fetch + L_parse + L_pipeline_fill

L_desc_fetch is hidden on chained descriptors when the descriptor engine has prefetched the next descriptor (DESCENG_CONFIG.PREFETCH_EN).

## 25.2	First-Beat Latency

The dominant term is the memory/network round trip to fill the pipeline. For a sink transfer the first AXIS beat must arrive and be buffered before the first AXI write can drain; for a source transfer the first AXI read must return before the first AXIS egress beat. After the first beat the pipeline is full and throughput is bandwidth-bound, not latency-bound.

## 25.3	Control-Descriptor Latency

Control descriptors are latency, not throughput (Section 6.1):

| Opcode | Latency |
| --- | --- |
| CTRL_WRITE (doorbell) | One single-beat AXI write: AW->W->B round trip |
| CTRL_READ (gate) | One poll per retry (AXI read + compare); retries paced by tick_1us, up to CTRL_CONFIG.CTRLRD_MAX_TRY (default 16, max 511) |

A CTRL_READ gate blocks its descriptor (and the chain) until the polled value matches. The retry budget bounds the worst case: an unsatisfied gate cannot hang the channel – after CTRLRD_MAX_TRY polls the engine raises ctrlrd_error instead of waiting forever.

## 25.4	Latency Variability

### 25.4.1	Sources of Variability

- Memory read latency (descriptor fetch, source reads) and write-response timing.

- AXIS backpressure (s_axis_tready / m_axis_tready) on the network end.

- Multi-channel arbitration for the shared AXI master.

- CTRL_READ retry count when a gate is not yet satisfied.

### 25.4.2	Worst-Case Latency

Bounded by: memory timeout (SCHED_TIMEOUT_CYCLES / SCHED_TIMEOUT_LIMIT escalation on the scheduler side) and the control-read retry budget (CTRL_CONFIG.CTRLRD_MAX_TRY). Both convert an otherwise-unbounded wait into a reported error so a channel is never permanently stalled.

## 25.5	Latency Optimization

### 25.5.1	Software Recommendations

- Chain descriptors so the descriptor engine prefetches the next while the current transfers (hides L_desc_fetch).

- Keep the AXIS consumer/producer ready to avoid backpressure stalls.

- Size CTRL_READ gates so the expected value is already (or soon) present to minimize poll retries.

### 25.5.2	Hardware Configuration

- Adequate SRAM_DEPTH so the buffer covers memory/network latency without stalling.

- Tune AXI_XFER_CONFIG (RD/WR beats, ALLOC/DRAIN) so bursts amortize AXI setup.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Resource Estimates

Unlike the theoretical throughput and latency numbers above, the resource figures here are measured from the actual place-and-route of the characterization build (rapids_char_harness wrapping rapids_beats_top) on the Nexys A7-100T.

## 26.1	FPGA Resource Summary

Measured, post-implementation (Vivado 2025.1, xc7a100t-csg324-1, 100 MHz, characterization build: NUM_CHANNELS = 4, DATA_WIDTH = 512, ADDR_WIDTH = 64, APB_ADDR_WIDTH = 13):

| Resource | Used | Available | Utilization |
| --- | --- | --- | --- |
| Slice LUTs | 37,555 | 63,400 | 59.24% |
| – LUT as Logic | 30,205 | 63,400 | 47.64% |
| – LUT as Memory | 7,350 | 19,000 | 38.68% |
| Slice Registers (FF) | 28,683 | 126,800 | 22.62% |
| Block RAM Tile | 22 | 135 | 16.30% |
| – RAMB36 | 20 | 135 | – |
| – RAMB18 | 4 | 270 | – |
| DSP48E1 | 0 | 240 | 0% |

Timing: all user-specified timing constraints met at 100 MHz (positive slack) – the harness plus DUT close cleanly on the -1 speed grade part.

The characterization build uses 4 channels (sink + source) to fit the harness (BRAM SRAM buffers, golden-CRC checkers, UART bridge) comfortably on the 100T. The design parameter default is NUM_CHANNELS = 8; an 8-channel build roughly scales the per-channel SRAM/scheduler logic and would target a larger part.

## 26.2	Resource Breakdown by Block

Approximate distribution of the DUT logic (excludes the char harness’s UART/CRC scaffolding). Both a sink and a source instance are present; they do not share logic.

### 26.2.1	APB Configuration Slave + Register Block

- LUTs: ~1.5K

- FFs: ~2K

- Function: APB decode, PeakRDL rapids_regs block (config + monitor regfile @ 0x1000), rapids_config_block hwif fan-out.

### 26.2.2	Scheduler Group (per direction)

- LUTs: ~3-4K per direction

- FFs: ~3K per direction

- Function: scheduler_beats (three-opcode: DATA / CTRL_READ / CTRL_WRITE), descriptor_engine_beats (256-bit fetch/parse + prefetch), channel arbitration.

### 26.2.3	Control Engines (per direction)

- LUTs: ~0.5K per direction

- FFs: ~0.5K per direction

- Function: ctrlrd_engine (poll-until-match, retry budget) and ctrlwr_engine (single-beat doorbell write). Small – one AXI read/write state machine each.

### 26.2.4	AXI Read / Write Engines

- LUTs: ~2-3K each

- FFs: ~2-3K each

- Function: 512-bit streaming AXI masters (source read, sink write) with beat-level completion.

### 26.2.5	SRAM Buffers (sink + source)

- BRAM: dominant BRAM consumer (20x RAMB36 + 4x RAMB18)

- Function: per-channel sink and source data buffers decoupling network and memory ends. BRAM scales with SRAM_DEPTH x NUM_CHANNELS x 2 directions.

### 26.2.6	MonBus / AXI Monitors (USE_AXI_MONITORS)

- LUTs: ~1-2K

- FFs: ~1-2K

- Function: rd/wr AXI monitors feeding the MonBus AXI-Lite group (error-drain + capture master), monitor perf windows @ 0x1000.

## 26.3	SRAM Sizing

BRAM is the primary scaling resource. Each direction has its own per-channel buffer, so total BRAM ~= 2 x NUM_CHANNELS x ceil(SRAM_DEPTH x DATA_WIDTH / BRAM_bits). For the 4-channel char build this lands at 22 BRAM tiles (16.30% of the 100T). Deeper buffers improve small-transfer efficiency (Section 6.1) at the cost of BRAM; wider NUM_CHANNELS multiplies both scheduler logic and BRAM.

## 26.4	Scaling Notes

| Change | LUT/FF impact | BRAM impact |
| --- | --- | --- |
| NUM_CHANNELS 4 -> 8 | ~+scheduler/arbiter per channel | ~2x buffer tiles |
| SRAM_DEPTH deeper | minimal | +tiles per direction |
| USE_AXI_MONITORS off | -1-2K LUT/FF | none |
| DATA_WIDTH 512 -> 256 | ~half datapath LUT/FF | ~half buffer width |

DSP usage is zero – RAPIDS Beats is pure control + datapath movement, no arithmetic multiply/accumulate.

Last Updated: 2026-07-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	RAPIDS Beats Architecture MAS

Version: 0.7 Date: 2026-07-13 Purpose: Module Architecture Specification for RAPIDS Phase 1 “Beats” Architecture

## 27.1	Document Organization

Note: All chapters linked below for automated document generation.

### 27.1.1	Front Matter

- Document Information

### 27.1.2	Chapter 1: Overview

- Architecture Overview

- Top-Level Port List

- Clocks and Reset

### 27.1.3	Chapter 2: FUB (Functional Unit Blocks)

Control Path: - Scheduler - Descriptor Engine

AXI Engines: - AXI Read Engine - AXI Write Engine

Flow Control: - Beats Alloc Control - Beats Drain Control - Beats Latency Bridge

Control Engines: - Control-Read Engine - Control-Write Engine

### 27.1.4	Chapter 3: Macro (Integration Blocks)

Scheduler Integration: - Beats Scheduler Group - Beats Scheduler Group Array

Sink Data Path (Network to Memory): - Sink Data Path - Sink Data Path AXIS - Sink SRAM Controller - Sink SRAM Controller Unit

Source Data Path (Memory to Network): - Source Data Path - Source Data Path AXIS - Source SRAM Controller - Source SRAM Controller Unit

Top-Level Integration: - RAPIDS Core Beats - RAPIDS Registers - RAPIDS Config Block - RAPIDS Beats Top

### 27.1.5	Chapter 4: Interfaces

- Interface Overview

- AXI4 Interface Specification

- AXIS Interface Specification

- MonBus Interface Specification

## 27.2	Quick Reference

### 27.2.1	FUB Modules (fub_beats/)

FUB Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler | scheduler.sv | Transfer coordinator, descriptor processing | Implemented |
| descriptor_engine | descriptor_engine.sv | Descriptor fetch/parse (256-bit) | Implemented |
| axi_read_engine | axi_read_engine.sv | AXI read master, streaming pipeline | Implemented |
| axi_write_engine | axi_write_engine.sv | AXI write master, streaming pipeline | Implemented |
| beats_alloc_ctrl | beats_alloc_ctrl.sv | Space allocation tracking (virtual FIFO) | Implemented |
| beats_drain_ctrl | beats_drain_ctrl.sv | Data availability tracking (virtual FIFO) | Implemented |
| beats_latency_bridge | beats_latency_bridge.sv | Latency compensation, backpressure management | Implemented |
| ctrlrd_engine | ctrlrd_engine.sv | Control-read consumer gate (poll-until-match, retry budget) | Implemented |
| ctrlwr_engine | ctrlwr_engine.sv | Control-write producer doorbell (single-beat write) | Implemented |

### 27.2.2	Macro Modules (macro_beats/)

Macro Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| beats_scheduler_group | beats_scheduler_group.sv | Scheduler + Descriptor Engine wrapper | Implemented |
| beats_scheduler_group_array | beats_scheduler_group_array.sv | 8-channel scheduler array with arbitration | Implemented |
| sink_data_path | sink_data_path.sv | Network-to-memory path integration | Implemented |
| sink_data_path_axis | sink_data_path_axis.sv | AXIS variant of sink path | Implemented |
| source_data_path | source_data_path.sv | Memory-to-network path integration | Implemented |
| source_data_path_axis | source_data_path_axis.sv | AXIS variant of source path | Implemented |
| snk_sram_controller | snk_sram_controller.sv | Sink SRAM buffer management | Implemented |
| snk_sram_controller_unit | snk_sram_controller_unit.sv | Per-channel sink SRAM unit | Implemented |
| src_sram_controller | src_sram_controller.sv | Source SRAM buffer management | Implemented |
| src_sram_controller_unit | src_sram_controller_unit.sv | Per-channel source SRAM unit | Implemented |
| rapids_core_beats | rapids_core_beats.sv | Core RAPIDS integration (scheduler array + data paths) | Implemented |
| rapids_regs | rapids_regs.sv (generated) | PeakRDL register block (base + MON regfile @ 0x1000) | Implemented |
| rapids_config_block | rapids_config_block.sv | Maps register hwif_out to core/monitor cfg_* | Implemented |
| rapids_beats_top | rapids_beats_top.sv | Top-level: APB slave, kickoff, monitors, MonBus AXI-Lite group | Implemented |

## 27.3	Architecture Comparison: RAPIDS vs STREAM

The “beats” architecture is a Phase 1 implementation of RAPIDS that shares concepts with STREAM but targets network-to-memory (and vice versa) transfers rather than memory-to-memory.

RAPIDS Beats vs STREAM Comparison

| Feature | STREAM | RAPIDS Beats |
| --- | --- | --- |
| Primary Use | Memory-to-memory DMA | Network-to-memory accelerator |
| Data Paths | Single bidirectional | Separate sink/source paths |
| Network Interface | None | AXIS master/slave |
| SRAM Buffering | Shared buffer | Separate sink/source buffers |
| Descriptor Format | 256-bit | 256-bit (compatible) |
| Channel Count | 8 | 8 |
| Flow Control | alloc_ctrl/drain_ctrl | beats_alloc_ctrl/beats_drain_ctrl |

## 27.4	Clock and Reset Summary

### 27.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | Primary - all RAPIDS logic, AXI/AXIS interfaces |

### 27.4.2	Reset Signals

Reset Signals

| Reset | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary - all RAPIDS logic |

See: Clocks and Reset for complete timing specifications

## 27.5	Interface Summary

### 27.5.1	External Interfaces

External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXI4 (Descriptor) | Master | 256-bit | Descriptor fetch |
| AXI4 (Sink Write) | Master | 512-bit (param) | Sink data write to memory |
| AXI4 (Source Read) | Master | 512-bit (param) | Source data read from memory |
| AXIS (Sink) | Slave | 512-bit (param) | Network data ingress |
| AXIS (Source) | Master | 512-bit (param) | Network data egress |
| MonBus | Master | 64-bit | Monitor packet output |

### 27.5.2	Internal Buses

Internal Buses

| Interface | Width | Purpose |
| --- | --- | --- |
| MonBus | 64-bit | Internal monitoring bus |
| Descriptor Bus | 256-bit | Descriptor distribution |
| SRAM Data Bus | 512-bit | SRAM read/write data |

## 27.6	Related Documentation

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- RAPIDS Beats HAS - Architecture specification (the original pre-beats rapids_spec/ tree was retired)

## 27.7	Specification Conventions

### 27.7.1	Signal Naming

- Clock: aclk

- Reset: aresetn (active-low)

- Valid/Ready: Standard AXI/AXIS handshake

- Registers: r_ prefix (e.g., r_state, r_counter)

- Wires: w_ prefix (e.g., w_next_state, w_grant)

### 27.7.2	Parameter Naming

- Uppercase: NUM_CHANNELS, DATA_WIDTH, ADDR_WIDTH

- Derived: CHAN_WIDTH = $clog2(NUM_CHANNELS)

### 27.7.3	Figure and Table Numbering

- Figures: Figure X.Y.Z: Title where X=chapter, Y=section, Z=figure number

- Tables: Caption line : Table Title after table markdown

Last Updated: 2026-07-13 Maintained By: RAPIDS Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	Product Requirements Document (PRD)

## 28.1	RAPIDS - Rapid AXI Programmable In-band Descriptor System

Version: 1.0 Date: 2025-09-30 Status: Active Development - Validation in Progress Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 28.2	1. Executive Summary

The Rapid AXI Programmable In-band Descriptor System (RAPIDS) is a custom hardware accelerator designed for efficient memory-to-memory data movement with network interface integration. It demonstrates complex FSM coordination, descriptor-based DMA operations, and comprehensive monitoring capabilities.

Status (2026-07-22): RAPIDS was rearchitected to the “beats” design. Current RTL is in rtl/fub_beats/, rtl/macro_beats/, rtl/top_beats/ (control engines in rtl/fub/); current specs are docs/rapids_beats_has/ and docs/rapids_beats_mas/. The old rapids_fub//rapids_macro/ RTL, the docs/rapids_spec/ tree, and the program engine / network master / network slave blocks described in parts of this PRD were retired (network interfaces are now AXIS; control is APB). Sections describing the pre-beats design are kept as history and marked where practical.

### 28.2.1	1.1 Quick Stats

- Modules: ~20 SystemVerilog files (beats architecture)

- Architecture: 3 major groups (Scheduler group, Sink path, Source path)

- Interfaces: AXI4 (memory), AXIS (network), APB (control), MonBus (monitoring)

- Test Coverage: see dv/tests/ beats suites (fub_beats/macro_beats/top_beats)

- Status: Active validation, known issues documented

### 28.2.2	1.2 Subsystem Goals

- Primary: Demonstrate complex accelerator design patterns

- Secondary: Provide DMA-style memory transfer capability

- Tertiary: Educational reference for descriptor-based engines

## 28.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 28.3.1	📚 Complete RAPIDS Specification

Status (2026-07-22): The old docs/rapids_spec/ tree was replaced by two beats spec trees.

Architecture Spec (HAS): docs/rapids_beats_has/

- HAS Index - Complete HAS structure

- Product Overview

- Block Diagram / Data Flow / Channel Architecture

- Interfaces: Summary, AXI4, AXIS, APB, MonBus

- Use cases: docs/rapids_beats_has/ch04_use_cases/

- Programming: Descriptor Format, Register Map, Initialization, Error Handling

- Performance: docs/rapids_beats_has/ch06_performance/

Micro-Architecture Spec (MAS): docs/rapids_beats_mas/

- MAS Index - Complete MAS structure

- Overview: Architecture, Port List, Clocks and Reset

FUB blocks (docs/rapids_beats_mas/ch02_fub_blocks/): - Scheduler - Per-channel scheduler FSM - Descriptor Engine - Descriptor fetch/parsing - AXI Read Engine / AXI Write Engine - Alloc Ctrl / Drain Ctrl / Latency Bridge - CtrlRd Engine / CtrlWr Engine

Macro blocks (docs/rapids_beats_mas/ch03_macro_blocks/): - Scheduler Group / Scheduler Group Array - Sink Data Path + AXIS wrapper, SRAM controllers - Source Data Path + AXIS wrapper, SRAM controllers - RAPIDS Core / Registers / Top

Interfaces (docs/rapids_beats_mas/ch04_interfaces/): - AXI4 Interface - AXIS Interface - MonBus Interface

### 28.3.2	🐛 Known Issues

Location: projects/components/dmas/rapids/known_issues/

- Index - Issue tracking overview (the old scheduler.md credit-counter write-up was retired with the pre-beats RTL)

- Sink Data Path - Minor issues

- Sink SRAM Control - Edge cases

- Plus known_issues/active/ for current beats issues

### 28.3.3	📖 Other Documentation

- CLAUDE - AI assistance guide for this subsystem

- TASKS - Work items (largely pre-beats history)

- Validation Report - Test results (pre-beats snapshot)

## 28.4	2.4 Organizational Standards - RAPIDS Code Location

⚠ MANDATORY: All RAPIDS-specific code must be in the project area ⚠

### 28.4.1	Code Organization Principle

“All RAPIDS-specific verification code MUST reside in projects/components/dmas/rapids/dv/ for easy discovery.”

This subsystem follows the repository-wide organizational standard (see /PRD.md Section 2.3) requiring all project-specific code to be located in the project area, NOT the framework area.

### 28.4.2	RAPIDS Directory Structure

projects/components/dmas/rapids/
├── rtl/                          # RTL source code
│   ├── includes/                 # RAPIDS packages
│   ├── fub/                      # Control engines (ctrlrd/ctrlwr)
│   ├── fub_beats/                # Beats functional unit blocks
│   ├── macro_beats/              # Beats assemblies + registers
│   ├── macro/                    # MonBus group
│   └── top_beats/                # rapids_beats_top.sv
│
└── dv/                           # Design verification (all RAPIDS-specific)
    ├── tbclasses/                # ★ RAPIDS TB classes HERE (not framework!)
    │   ├── scheduler_tb.py       # Scheduler testbench class
    │   ├── descriptor_engine_tb.py
    │   └── rapids_core_beats_tb.py
    │
    ├── components/               # ★ RAPIDS-specific BFMs
    │   └── data_mover_bfm.py
    │
    └── tests/                    # Test runners (import TB classes)
        ├── fub/                  # Control engine tests
        ├── fub_beats/            # Beats FUB tests (test_scheduler_beats.py, ...)
        ├── macro/                # MonBus group test
        ├── macro_beats/          # Multi-block scenarios
        └── top_beats/            # Full RAPIDS operation

### 28.4.3	What Goes Where?

| Code Type | ✓ CORRECT Location | ✗ WRONG Location |
| --- | --- | --- |
| RAPIDS TB Classes | projects/components/dmas/rapids/dv/tbclasses/ | bin/TBClasses/ (framework area) |
| RAPIDS-Specific BFMs | projects/components/dmas/rapids/dv/components/ | bin/TBClasses/ (framework area) |
| RAPIDS Scoreboards | projects/components/dmas/rapids/dv/ (project area) | bin/TBClasses/scoreboards/ |
| Test Runners | projects/components/dmas/rapids/dv/tests/ | Anywhere else |
| Shared AXI4/APB BFMs | bin/TBClasses/{protocol}/ | Project area |

### 28.4.4	Import Pattern for RAPIDS Tests

✓ CORRECT - Import from Project Area:

# Import framework utilities (PYTHONPATH includes bin/)
import os, sys
from TBClasses.shared.utilities import get_repo_root
from TBClasses.shared.tbbase import TBBase

# Add repo root to Python path using robust git-based method
repo_root = get_repo_root()
sys.path.insert(0, repo_root)

# Import RAPIDS TB classes from PROJECT AREA
from projects.components.dmas.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from projects.components.dmas.rapids.dv.tbclasses.descriptor_engine_tb import DescriptorEngineTB

# Shared framework components
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

✗ WRONG - Don’t Import from Framework:

# DON'T DO THIS!
from TBClasses.rapids.scheduler_tb import SchedulerTB  # ✗ WRONG!

### 28.4.5	Benefits of This Organization

- Easy Discovery - All RAPIDS code in ONE place: projects/components/dmas/rapids/

- Clear Ownership - RAPIDS team owns their dv/ area completely

- No Confusion - Never wonder “where does this TB class live?”

- Maintainability - Changes isolated to RAPIDS area don’t affect other projects

- Framework Stays Clean - Only truly shared cross-project code in framework

### 28.4.6	Compliance Status

✓ RAPIDS is now compliant - All TB classes moved to project area as of 2025-10-18

Migration History: - Before: TB classes incorrectly in bin/TBClasses/rapids/ - After: TB classes correctly in projects/components/dmas/rapids/dv/tbclasses/ - Test Imports: Updated to import from project area

📖 Complete Documentation: See /PRD.md Section 2.3 for repository-wide organizational standards.

## 28.5	3. Architecture Overview

### 28.5.1	3.1 Top-Level Block Diagram

RAPIDS Beats (Rapid AXI Programmable In-band Descriptor System)
├── Scheduler Group (scheduler_group_beats / scheduler_group_array_beats)
│   ├── Scheduler          (Per-channel FSM, scheduler_beats.sv)
│   ├── Descriptor Engine  (Descriptor fetch/parsing, descriptor_engine_beats.sv)
│   └── Control Engines    (ctrlrd_engine.sv / ctrlwr_engine.sv)
│
├── Sink Data Path (AXIS Network → SRAM → System Memory)
│   ├── AXIS Slave ingress   (snk_data_path_axis_beats.sv)
│   ├── Sink SRAM Controller (snk_sram_controller_beats.sv + alloc/drain ctrl)
│   └── AXI Write Engine     (axi_write_engine_beats.sv)
│
├── Source Data Path (System Memory → SRAM → AXIS Network)
│   ├── AXI Read Engine        (axi_read_engine_beats.sv)
│   ├── Source SRAM Controller (src_sram_controller_beats.sv + alloc/drain ctrl)
│   └── AXIS Master egress     (src_data_path_axis_beats.sv)
│
└── Control and Monitoring
    ├── APB CSRs           (rapids_config_block.sv, PeakRDL rapids_regs.rdl)
    └── MonBus Group       (macro/monbus_axil_group_2in.sv)

📖 See: docs/rapids_beats_mas/ch01_overview/01_architecture.md for detailed architecture

### 28.5.2	3.2 Data Flow

Sink Path (Receive): 1. Network packets arrive via Network Slave 2. Buffered in Sink SRAM 3. DMA’d to system memory via AXI4 Write Engine 4. Completion reported via MonBus

Source Path (Transmit): 1. Descriptor specifies data location in system memory 2. Source AXI Reader fetches data to Source SRAM 3. Network Master transmits to network 4. Completion reported via MonBus

Scheduler Coordination: - Parses descriptors from Descriptor Engine - Manages credit-based flow control - Sequences program engine operations - Coordinates sink/source data paths

## 28.6	4. Key Features

### 28.6.1	4.1 Descriptor-Based Operation

| Feature | Status | Description |
| --- | --- | --- |
| Descriptor FIFO | ✓ | Queued descriptor processing |
| Multi-field parsing | ✓ | Address, length, control fields |
| Chained descriptors | ⏳ | Future enhancement |
| Completion reporting | ✓ | Via MonBus packets |

### 28.6.2	4.2 Data Path Features

| Feature | Status | Description |
| --- | --- | --- |
| SRAM buffering | ✓ | Decouple network from memory |
| AXI4 burst support | ✓ | Efficient memory transfers |
| Backpressure handling | ✓ | Flow control on all interfaces |
| Data alignment | ✓ | Handle unaligned transfers |

### 28.6.3	4.3 Scheduler Features

| Feature | Status | Description |
| --- | --- | --- |
| Task FSM | ✓ | Multi-state coordination |
| Credit management | ⏳ | Pre-beats scheduler had exponential encoding (0→1, 1→2, …, 15→∞); scheduler_beats.sv has no credit management yet (planned later phase) |
| Program sequencing | ✓ | Coordinated operations |
| Error detection | ✓ | Timeout, overflow detection |

Credit Management Details (pre-beats, historical): - Uses exponential credit encoding for compact configuration - 4-bit cfg_initial_credit decodes to actual credit counts: - 0 → 1 credit (2^0), 4 → 16 credits (2^4), 8 → 256 credits (2^8) - 15 → ∞ (unlimited credits, 0xFFFFFFFF) - Encoding applied at initialization; runtime operations are linear (increment/decrement by 1) - Provides wide range (1 to 16384) with minimal configuration overhead

📖 See: docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md for the current scheduler specification

### 28.6.4	4.4 Monitoring Integration

| Feature | Status | Description |
| --- | --- | --- |
| MonBus packets | ✓ | StandardAMBA 64-bit format |
| Descriptor events | ✓ | Start/complete reporting |
| Error events | ✓ | Timeout, overflow, underflow |
| Performance metrics | ⏳ | Future enhancement |

## 28.7	5. Interfaces

### 28.7.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| APB | Slave | 32-bit | Control/status registers |
| AXI4 (Sink) | Master | Configurable | Write to system memory |
| AXI4 (Source) | Master | Configurable | Read from system memory |
| AXIS (Sink) | Slave | Configurable | Network ingress (tid = channel) |
| AXIS (Source) | Master | Configurable | Network egress (tid = channel) |
| MonBus | Master | 64-bit | Monitor packet output |

📖 See: docs/rapids_beats_mas/ch04_interfaces/ and docs/rapids_beats_has/ch03_interfaces/ for complete interface specs

### 28.7.2	5.2 Configuration Parameters

// Example RAPIDS instantiation sketch (see rtl/top_beats/rapids_beats_top.sv for the
// full parameter/port list)
rapids_beats_top #(
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .NUM_CHANNELS(8)
) u_rapids (
    .clk                (clk),
    .rst_n              (rst_n),
    // APB control interface
    .s_apb_*            (...),
    // AXI4 memory interfaces (read + write masters)
    .m_axi_*            (...),
    // AXIS network interfaces (tid = channel)
    .s_axis_*           (...),
    .m_axis_*           (...),
    // MonBus / monitor group egress
    .mon_*              (...)
);

## 28.8	6. Use Cases

### 28.8.1	6.1 DMA-Style Transfers

Scenario: Move data from network to system memory

Flow: 1. Software writes descriptor to Descriptor Engine 2. Scheduler parses descriptor, activates Sink path 3. Network packets arrive via Network Slave 4. Data buffered in Sink SRAM 5. AXI4 Write Engine DMAs to system memory 6. Completion packet on MonBus

### 28.8.2	6.2 Network Packet Processing

Scenario: Read data from memory, transmit to network

Flow: 1. Descriptor specifies source address, length 2. Source AXI Reader fetches data to SRAM 3. Network Master transmits to network 4. Completion/error reporting via MonBus

### 28.8.3	6.3 Custom Data Path Acceleration

Educational value: Shows how to build custom accelerators - Descriptor-based control - Multi-block FSM coordination - Buffering strategies - Error handling - Performance monitoring

## 28.9	7. Test Coverage

### 28.9.1	7.1 Current Status

Status (2026-07-22): The numbers below are a pre-beats snapshot (they include the retired program engine). Current beats regression: fub 232+ and macro 182 tests passing; run the beats suites for live numbers.

Overall (pre-beats snapshot): ~85% functional coverage (basic scenarios validated, descriptor engine complete)

| Component | Test Coverage | Status |
| --- | --- | --- |
| Scheduler | ~95% | Credit encoding fixed and verified (43/43 tests passing) |
| Descriptor Engine | ✓ 100% | All tests passing (14/14 tests, 100% success rate) |
| Program Engine | ~85% | Alignment tested |
| Sink Data Path | ~75% | Basic flows working |
| Source Data Path | ~70% | Basic flows working |
| SRAM Controllers | ~80% | Buffer management tested |
| Integration | ~60% | More stress testing needed |

Test Location: projects/components/dmas/rapids/dv/tests/fub_beats/ and projects/components/dmas/rapids/dv/tests/macro_beats/ (plus fub/, macro/, top_beats/)

Recent Achievements: - ✓ Descriptor Engine (2025-10-13): Achieved 100% test pass rate using continuous background monitoring pattern - 14/14 tests passing across all test levels (basic, medium, full) - All test classes passing (APB_ONLY, MIXED) - All delay profiles passing (fast_producer, fast_consumer, fixed_delay, minimal_delay) - Applied continuous monitoring methodology for asynchronous output capture

📖 See: docs/RAPIDS_Validation_Status_Report.md for detailed results

### 28.9.2	7.2 Test Strategy

FUB (Functional Unit Block) Tests: - Individual block testing - Located in projects/components/dmas/rapids/dv/tests/fub_beats/ (+ fub/ for control engines) - Focus on module-level functionality

Macro Tests: - Multi-block scenarios - Located in projects/components/dmas/rapids/dv/tests/macro_beats/ (+ macro/ for the monbus group) - End-to-end data flow validation

Top Tests: - Full RAPIDS operation - Located in projects/components/dmas/rapids/dv/tests/top_beats/ - Realistic traffic patterns

## 28.10	8. Known Issues Summary

### 28.10.1	8.1 Critical Issues

✓ Scheduler Credit Counter Initialization - FIXED (2025-10-11, pre-beats) - File: the retired pre-beats scheduler.sv (replaced by rtl/fub_beats/scheduler_beats.sv, which currently has no credit management - planned for a later phase) - Issue: Credit counter was initializing to 0 instead of using exponential encoding - Fix Applied: Implemented exponential credit encoding - Status: Fixed and verified pre-beats; the known_issues/scheduler.md write-up was retired with that RTL (see known_issues/README.md)

Fix Details:

// Previous (wrong):
r_descriptor_credit_counter <= 32'h0;

// Fixed - Exponential encoding:
// 0→1, 1→2, 2→4, 3→8, ..., 14→16384, 15→∞
r_descriptor_credit_counter <= (cfg_initial_credit == 4'hF) ? 32'hFFFFFFFF :
                              (cfg_initial_credit == 4'h0) ? 32'h00000001 :
                              (32'h1 << cfg_initial_credit);

Encoding Rationale: - Compact 4-bit configuration covers 1 to 16384 credits - Fine-grained control for low traffic (1, 2, 4, 8) - High-throughput support (256, 1024, 16384) - Special unlimited mode (15 → ∞) - Exponential encoding applied at initialization only; runtime operations are linear

### 28.10.2	8.2 Medium Priority Issues

Descriptor Engine Edge Cases: - Some stress test failures under high load - Edge case handling needs improvement - Priority: P2

SRAM Control Timing: - Rare timing issues in back-to-back operations - Priority: P2

📖 See: known_issues/ directory for complete issue tracking

## 28.11	9. Integration Guidelines

### 28.11.1	9.1 Quick Start

rapids_beats_top #(
    .ADDR_WIDTH(64),
    .DATA_WIDTH(512),
    .NUM_CHANNELS(8)
) u_rapids (
    // Clocking & Reset
    .clk                (system_clk),
    .rst_n              (system_rst_n),

    // APB Control (connect to control fabric)
    .s_apb_paddr        (ctrl_paddr),
    .s_apb_psel         (ctrl_psel),
    .s_apb_penable      (ctrl_penable),
    // ... other APB signals

    // AXI4 Memory (connect to memory controller)
    // ... AXI write channels (sink), AXI read channels (source)

    // AXIS Network (connect to network fabric; tid = channel)
    .s_axis_tdata       (net_rx_data),
    .s_axis_tvalid      (net_rx_valid),
    // ... AXIS slave (receive), AXIS master (transmit)

    // Monitor group egress (AXI-Lite drain / IRQ)
    // ... mon_* signals
);

See rtl/top_beats/rapids_beats_top.sv for the complete, authoritative port list.

### 28.11.2	9.2 Configuration Steps

- Initialize via APB registers (use the generated regmap, access registers by name):

  - Configure channel enables and thresholds

  - Set timeout thresholds

- Load Descriptors:

  - Write descriptors to Descriptor Engine FIFO

  - Each descriptor specifies: address, length, control bits

- Enable Operation:

  - Set enable bits via APB registers

  - Monitor MonBus for completion/error packets

📖 See: docs/rapids_beats_has/ch05_programming/ for register and programming details

## 28.12	10. Development Status

### 28.12.1	10.1 Current Phase

Phase: Validation and Bug Fixing (In Progress)

- ✓ Core architecture implemented

- ✓ Basic functionality verified

- ✓ Scheduler credit counter bug fixed (exponential encoding implemented)

- ⏳ Credit management tests need verification (remove workarounds)

- ⏳ Stress testing ongoing

- ⏳ Edge case refinement

📖 See: TASKS.md for detailed work items

### 28.12.2	10.2 Roadmap

Near-Term (Q4 2025): - ✓ Fix scheduler credit counter bug (completed 2025-10-11) - ⏳ Verify credit management tests (remove workarounds) - ⏳ Complete descriptor engine stress testing - ⏳ Integration test suite expansion - ⏳ Performance benchmarking

Long-Term (2026+): - Chained descriptor support - Advanced error recovery - Performance optimizations - Multi-channel support

## 28.13	11. Performance Characteristics

### 28.13.1	11.1 Throughput

Target: Match network/memory interface bandwidth

Bottlenecks: - SRAM buffer size - AXI4 burst efficiency - Scheduler overhead

Optimization: - Increase SRAM depth for larger packets - Tune AXI4 burst parameters - Pipeline scheduler operations

### 28.13.2	11.2 Latency

Components: - Descriptor parsing: ~10 cycles - SRAM buffering: Configurable depth - AXI4 memory access: System dependent - End-to-end: Typically <100 cycles for small packets

### 28.13.3	11.3 Resource Utilization

Area: - Scheduler: ~2K LUTs - Each data path: ~3K LUTs - SRAM buffers: Configurable (dominant area) - Total: ~10K LUTs + SRAM

Power: - Clock gating opportunities in idle blocks - SRAM power depends on depth/width

## 28.14	12. Verification Infrastructure

### 28.14.1	12.1 Test Organization

Location: projects/components/dmas/rapids/dv/tests/

Structure:

projects/components/dmas/rapids/dv/tests/
├── fub/                    # Control engine tests (ctrlrd/ctrlwr)
├── fub_beats/              # Beats FUB tests (scheduler, descriptor engine,
│                           #   alloc/drain ctrl, latency bridge)
├── macro/                  # MonBus group test
├── macro_beats/            # Multi-block scenarios (scheduler group, data paths,
│                           #   SRAM controllers)
└── top_beats/              # Full RAPIDS operation

### 28.14.2	12.2 CocoTB Framework

Location: projects/components/dmas/rapids/dv/tbclasses/ (RAPIDS-specific) plus shared BFMs in bin/TBClasses/

Components: - RAPIDS-specific drivers - Descriptor generators - Traffic patterns - Monitor checkers

📖 See: docs/markdown/TBClasses/ for shared framework docs

### 28.14.3	12.2.1 MANDATORY: BFM Usage for FUB Tests

⚠ CRITICAL DESIGN REQUIREMENT ⚠

All RAPIDS FUB (Functional Unit Block) level tests MUST use CocoTB Framework BFMs. Manual handshake driving is NOT allowed.

Required BFM Components:

| Interface Type | Framework Location | BFM Component |
| --- | --- | --- |
| Custom valid/ready | bin/TBClasses/gaxi/ | GAXI Master/Slave |
| AXI4 | bin/TBClasses/axi4/ | AXI4 Master/Slave |
| AXI4-Lite (AXIL) | bin/TBClasses/axil4/ | AXIL Master/Slave |
| APB | bin/TBClasses/apb/ | APB Master/Slave |
| AXI-Stream (AXIS) | bin/TBClasses/axis4/ | AXIS Master/Slave (RAPIDS network interfaces are AXIS) |
| MonBus | bin/TBClasses/monbus/ | MonBus drivers |

Rationale: 1. Consistency: All tests use standardized handshake protocols 2. Correctness: BFMs handle complex timing scenarios (backpressure, randomization) 3. Reusability: Same BFM across all RAPIDS tests 4. Maintainability: Fix once in BFM, all tests benefit 5. Coverage: BFMs include comprehensive timing profiles

Example - Program Engine:

# ✗ WRONG: Manual handshake (violates design requirement)
async def send_request(self, addr, data):
    self.dut.program_valid.value = 1
    self.dut.program_pkt_addr.value = addr
    # ... manual handshaking logic ...

# ✓ CORRECT: Use GAXI Master BFM
from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster

class ProgramEngineTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.program_master = GAXIMaster(
            dut=dut,
            clock=dut.clk,
            valid_signal='program_valid',
            ready_signal='program_ready',
            data_signals=['program_pkt_addr', 'program_pkt_data'],
            data_widths=[64, 32]
        )

    async def send_request(self, addr, data):
        await self.program_master.write({'program_pkt_addr': addr, 'program_pkt_data': data})

📖 See: - projects/components/dmas/rapids/CLAUDE.md - Rule #1 for complete BFM usage guidelines - docs/markdown/TBClasses/gaxi/ - GAXI BFM documentation - bin/TBClasses/axi4/ - AXI4 BFM sources (full framework docs in the RTLDesignSherpa-DV repo)

### 28.14.4	12.3 Test File Structure (Standard Pattern)

⚠ MANDATORY: All RAPIDS tests must follow this structure ⚠

RAPIDS tests follow the same pattern as AMBA tests for consistency across the repository:

# Example: projects/components/dmas/rapids/dv/tests/fub_beats/test_scheduler_beats.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# Import REUSABLE testbench class (NOT defined in this file!)
from projects.components.dmas.rapids.dv.tbclasses.scheduler_tb import SchedulerTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic_flow(dut):
    """Test basic descriptor flow."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_basic_descriptor_flow()
    assert result, "Basic descriptor flow test failed"

@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_credit_encoding(dut):
    """Test exponential credit encoding."""
    tb = SchedulerTB(dut)
    await tb.setup_clocks_and_reset()  # Mandatory init method
    await tb.initialize_test()
    result = await tb.test_exponential_encoding_all_values()
    assert result, "Credit encoding test failed"

# ===========================================================================
# PARAMETER GENERATION - at bottom of file
# ===========================================================================

def generate_scheduler_test_params():
    """Generate test parameters for scheduler tests."""
    return [
        # (channel_id, num_channels, data_width, credit_width)
        (0, 8, 512, 8),  # Standard configuration
        # Add more parameter sets as needed
    ]

scheduler_params = generate_scheduler_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - at bottom of file
# ===========================================================================

@pytest.mark.parametrize("channel_id, num_channels, data_width, credit_width", scheduler_params)
def test_basic_flow(request, channel_id, num_channels, data_width, credit_width):
    """
    Scheduler basic flow test.

    Run with: pytest projects/components/dmas/rapids/dv/tests/fub_beats/test_scheduler_beats.py::test_basic_flow -v
    """
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_fub_beats': '../../rtl/fub_beats'
    })

    dut_name = "scheduler_beats"
    toplevel = dut_name

    verilog_sources = [
        os.path.join(repo_root, 'rtl', 'amba', 'includes', 'monitor_pkg.sv'),
        # RAPIDS package(s) from the project includes area
        # (prefer get_sources_from_filelist() with rtl/filelists/ in real tests)
        os.path.join(rtl_dict['rtl_fub_beats'], '..', 'includes', 'rapids_pkg.sv'),
        os.path.join(rtl_dict['rtl_fub_beats'], f'{dut_name}.sv'),
    ]

    # Format parameters for unique test name
    cid_str = TBBase.format_dec(channel_id, 2)
    nc_str = TBBase.format_dec(num_channels, 2)
    dw_str = TBBase.format_dec(data_width, 4)
    cw_str = TBBase.format_dec(credit_width, 2)
    test_name_plus_params = f"test_{dut_name}_cid{cid_str}_nc{nc_str}_dw{dw_str}_cw{cw_str}"

    # Add worker ID for pytest-xdist parallel execution
    worker_id = os.environ.get('PYTEST_XDIST_WORKER', '')
    if worker_id:
        test_name_plus_params = f"{test_name_plus_params}_{worker_id}"

    log_path = os.path.join(log_dir, f'{test_name_plus_params}.log')
    sim_build = os.path.join(tests_dir, 'local_sim_build', test_name_plus_params)
    os.makedirs(sim_build, exist_ok=True)
    os.makedirs(log_dir, exist_ok=True)

    rtl_parameters = {
        'CHANNEL_ID': channel_id,
        'NUM_CHANNELS': num_channels,
        'DATA_WIDTH': data_width,
        'CREDIT_WIDTH': credit_width,
        # Add other RTL parameters as needed
    }

    extra_env = {
        'LOG_PATH': log_path,
        'TEST_CHANNEL_ID': str(channel_id),
        'TEST_NUM_CHANNELS': str(num_channels),
        'TEST_DATA_WIDTH': str(data_width),
    }

    compile_args = ["-Wno-TIMESCALEMOD"]
    sim_args = []
    plusargs = []

    cmd_filename = create_view_cmd(log_dir, log_path, sim_build, module, test_name_plus_params)

    try:
        run(
            python_search=[tests_dir],
            verilog_sources=verilog_sources,
            includes=[
                os.path.join(repo_root, 'projects', 'components', 'dmas', 'rapids', 'rtl', 'includes'),
                os.path.join(repo_root, 'rtl', 'amba', 'includes'),
            ],
            toplevel=toplevel,
            module=module,
            testcase="cocotb_test_basic_flow",  # ← cocotb test function name
            parameters=rtl_parameters,
            sim_build=sim_build,
            extra_env=extra_env,
            waves=False,
            keep_files=True,
            compile_args=compile_args,
            sim_args=sim_args,
            plusargs=plusargs,
        )

        print(f"✓ Scheduler basic flow test completed!")
        print(f"Logs: {log_path}")

    except Exception as e:
        print(f"✗ Scheduler basic flow test failed: {str(e)}")
        print(f"Logs preserved at: {log_path}")
        raise

Key Structure Requirements:

- Testbench Class Location:

  - ALWAYS in projects/components/dmas/rapids/dv/tbclasses/

  - NEVER inline in test file

  - Reusable across multiple test files

- CocoTB Test Functions:

  - Prefix with cocotb_ to prevent pytest collection

  - Located at top of test file

  - Use @cocotb.test() decorator

  - Call testbench methods

- Parameter Generation:

  - Function returns list of parameter tuples

  - Located near bottom of file (before pytest wrappers)

  - Stored in variable (e.g., scheduler_params)

- Pytest Wrapper Functions:

  - Located at bottom of file

  - Use @pytest.mark.parametrize() with parameter variable

  - Build unique test names with TBBase.format_dec()

  - Call run() with testcase="cocotb_test_function_name"

  - Handle parallel execution (PYTEST_XDIST_WORKER)

- Mandatory TB Methods:

  - async def setup_clocks_and_reset(self) - Complete initialization

  - async def assert_reset(self) - Assert reset signal(s)

  - async def deassert_reset(self) - Deassert reset signal(s)

📖 See: - val/amba/test_apb4_slave.py - Reference example - projects/components/dmas/rapids/CLAUDE.md - Detailed TB requirements

## 28.15	13. Quick Reference

### 28.15.1	13.1 Key Files

| File | Purpose |
| --- | --- |
| projects/components/dmas/rapids/PRD.md | This document (overview) |
| projects/components/dmas/rapids/CLAUDE.md | AI assistance guide |
| projects/components/dmas/rapids/TASKS.md | Work items (largely pre-beats history) |
| projects/components/dmas/rapids/docs/rapids_beats_has/ | Architecture specification |
| projects/components/dmas/rapids/docs/rapids_beats_mas/ | Micro-architecture specification |
| projects/components/dmas/rapids/known_issues/ | Bug tracking |
| docs/RAPIDS_Validation_Status_Report.md | Test results (pre-beats snapshot) |

### 28.15.2	13.2 Commands

# Run RAPIDS tests
pytest projects/components/dmas/rapids/dv/tests/fub_beats/ -v     # Individual blocks
pytest projects/components/dmas/rapids/dv/tests/macro_beats/ -v   # Multi-block
pytest projects/components/dmas/rapids/dv/tests/top_beats/ -v     # Full system

# Run specific FUB test
pytest projects/components/dmas/rapids/dv/tests/fub_beats/test_scheduler_beats.py -v

# Lint RAPIDS RTL
verilator --lint-only projects/components/dmas/rapids/rtl/fub_beats/scheduler_beats.sv

# View specifications
cat projects/components/dmas/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md
cat projects/components/dmas/rapids/docs/rapids_beats_mas/ch02_fub_blocks/01_scheduler.md

## 28.16	14. Success Criteria

### 28.16.1	14.1 Functional

- ✓ All major blocks implemented

- ✓ Basic data flows working

- ✓ Scheduler credit bug fixed (exponential encoding implemented)

- ⏳ Credit management tests verified (remove workarounds, run full suite)

- ⏳ 100% descriptor test pass rate (currently ~80%)

- ⏳ Stress tests passing

### 28.16.2	14.2 Quality

- ⏳ >90% functional coverage (currently ~80%)

- ⏳ >85% code coverage

- ✓ All FSMs documented

- ⏳ Integration guide complete

### 28.16.3	14.3 Documentation

- ✓ Complete specification in docs/rapids_beats_has/ + docs/rapids_beats_mas/

- ✓ Known issues documented

- ⏳ Integration examples

- ⏳ Performance characterization

## 28.17	15. Educational Value

RAPIDS demonstrates: - ✓ Complex FSM coordination (scheduler ↔ data paths) - ✓ Descriptor-based DMA design patterns - ✓ Buffer management strategies - ✓ Credit-based flow control with exponential encoding - ✓ Multi-interface integration - ✓ Comprehensive monitoring - ✓ Error detection and reporting - ✓ Compact configuration encoding strategies

Target Audience: - Advanced RTL designers - Accelerator architects - DMA engine developers - System integration engineers

## 28.18	15. Attribution and Contribution Guidelines

### 28.18.1	15.1 Git Commit Attribution

When creating git commits for RAPIDS documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: RAPIDS documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 28.19	16. Documentation Generation

### 28.19.1	16.1 Generating PDF/DOCX from Specification

Tool: bin/md_to_docx.py (driven by the wrapper scripts in docs/ - preferred)

Use the wrapper scripts to convert the linked HAS/MAS spec indexes into single all-inclusive PDF/DOCX files:

Basic Usage:

# Preferred: wrapper scripts (they call md_to_docx.py with the house style)
cd projects/components/dmas/rapids/docs
./generate_has_pdf.sh --rev 0.8     # builds RAPIDS_Beats_HAS_v0.8.docx/.pdf
./generate_mas_pdf.sh --rev 0.7     # builds RAPIDS_Beats_MAS_v0.7.docx/.pdf

# Direct tool invocation (from repo root), if you need custom options
python bin/md_to_docx.py \
    projects/components/dmas/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md \
    -o projects/components/dmas/rapids/docs/RAPIDS_Beats_MAS_draft.docx \
    --toc \
    --title-page \
    --pdf

Key Features: - Recursive Collection: Follows all markdown links in the index file - Heading Demotion: Automatically adjusts heading levels for included files - Table of Contents: --toc flag generates automatic ToC - Title Page: --title-page flag creates title page from first heading - PDF Export: --pdf flag generates both DOCX and PDF - Image Support: Resolves images relative to source directory - Template Support: Optional custom DOCX/DOTX template via -t flag

Common Workflow:

# 1. Update spec content under rapids_beats_has/ or rapids_beats_mas/
# 2. Generate documentation with a bumped revision
cd projects/components/dmas/rapids/docs
./generate_mas_pdf.sh --rev 0.8

# 3. Output files created in docs/:
#    - RAPIDS_Beats_MAS_v0.8.docx
#    - RAPIDS_Beats_MAS_v0.8.pdf

Debug Mode:

# Generate debug markdown to see combined output
python bin/md_to_docx.py \
    projects/components/dmas/rapids/docs/rapids_beats_mas/rapids_beats_mas_index.md \
    -o output.docx \
    --debug-md

# This creates debug.md showing the complete merged content

Tool Requirements: - Python 3.6+ - Pandoc installed and in PATH - For PDF generation: LaTeX (e.g., texlive) or use Pandoc’s built-in PDF writer

📖 See: bin/md_to_docx.py for complete implementation details

## 28.20	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/dmas/rapids/docs/

Quick Command: Use the provided shell scripts:

cd projects/components/dmas/rapids/docs
./generate_has_pdf.sh    # Architecture spec (HAS)
./generate_mas_pdf.sh    # Micro-architecture spec (MAS)

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the rapids_beats_has / rapids_beats_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 28.21	17. References

### 28.21.1	16.1 Internal Documentation

- Complete Spec: docs/rapids_beats_has/ + docs/rapids_beats_mas/ ← Primary technical reference

- Validation: docs/RAPIDS_Validation_Status_Report.md (pre-beats snapshot)

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 28.21.2	16.2 Related Subsystems

- AMBA: rtl/amba/ - Monitor infrastructure used in RAPIDS (monitors in rtl/amba/monitor/)

- Common: rtl/common/ - Building blocks (counters, FIFOs, etc.)

- CocoTB Framework: bin/TBClasses/ (shared) + projects/components/dmas/rapids/dv/tbclasses/ (RAPIDS)

### 28.21.3	16.3 External References

- AXI4 Specification: ARM IHI0022E

- AXIL4 Specification: ARM IHI0022E (subset)

- Network interface specs (custom Network protocol)

Document Version: 1.0 Last Updated: 2025-09-30 Review Cycle: Monthly during active development Next Review: 2025-10-30 Owner: RTL Design Sherpa Project

## 28.22	Navigation

- ← Back to Root: /PRD.md

- Complete Specification: docs/rapids_beats_has/rapids_beats_has_index.md + docs/rapids_beats_mas/rapids_beats_mas_index.md

- AI Guidance: CLAUDE.md

- Tasks: TASKS.md

- Issues: known_issues/

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Address Increment Patterns for DMA and Descriptor-Based Data Movement

Document Version: 1.0 Last Updated: 2026-01-13 Scope: Comprehensive analysis of address increment patterns for descriptor-based systems Reference Implementation: RAPIDS (Rapid AXI Programmable In-band Descriptor System)

## 29.1	Table of Contents

- Executive Summary

- RAPIDS Current Implementation

- AXI Protocol Burst Modes

- Linear Address Increment Patterns

- 2D Transfer Patterns (Stride-Based)

- 3D Transfer Patterns

- Scatter-Gather Patterns

- Circular and Ring Buffer Patterns

- Tensor Memory Access Patterns

- Implementation Comparison Matrix

- RAPIDS Extension Recommendations

- References

## 29.2	1. Executive Summary

Address increment patterns determine how DMA engines and descriptor-based data movement systems calculate memory addresses during transfers. The choice of pattern significantly impacts:

- Bandwidth efficiency: Alignment and burst optimization

- Application fit: Different workloads require different patterns

- Hardware complexity: More patterns = more control logic

- Software flexibility: Richer patterns = simpler programming model

### 29.2.1	Pattern Categories

| Category | Description | Use Cases |
| --- | --- | --- |
| Linear/Contiguous | Sequential address increment | Block copy, streaming data |
| 2D Stride | Row-major with inter-row gaps | Image processing, video frames |
| 3D Volume | Multi-plane with inter-plane gaps | 3D imaging, tensor operations |
| Scatter-Gather | Non-contiguous fragment list | Protocol buffers, fragmented memory |
| Circular | Wrap-around at boundary | Audio streaming, network buffers |
| Tensor | Multi-dimensional with strides | Neural network accelerators |

## 29.3	2. RAPIDS Current Implementation

### 29.3.1	2.1 Descriptor Format (256-bit)

RAPIDS uses a fixed 256-bit descriptor structure defined in rapids_pkg.sv:

Bit Field          Width   Description
─────────────────────────────────────────────────────────
[63:0]             64-bit  src_addr - Source address (byte-aligned)
[127:64]           64-bit  dst_addr - Destination address (byte-aligned)
[159:128]          32-bit  length - Transfer length in BEATS (not bytes)
[191:160]          32-bit  next_descriptor_ptr - Next descriptor address
[207:200]           8-bit  desc_priority - Transfer priority
[199:196]           4-bit  channel_id - Channel ID
[195]               1-bit  error - Error flag
[194]               1-bit  last - Last descriptor in chain
[193]               1-bit  gen_irq - Generate interrupt
[192]               1-bit  valid - Valid descriptor
[255:208]          48-bit  reserved - Future use

### 29.3.2	2.2 Current Address Increment Modes

Mode 1: Beat-Based Linear (Default)

Address Calculation:
  next_addr = current_addr + (beat_count × 64 bytes)

  Where: RAPIDS_BYTES_PER_BEAT = 64 bytes (512-bit data width)

- All transfers aligned to 64-byte boundaries for optimal throughput

- Descriptor length field specifies count in beats

- Software converts byte count to beats: beats = (bytes + 63) >> 6

Mode 2: Chunk-Based Alignment (Sub-Beat Transfers)

For alignment/final phases handling partial beats:

Address Calculation for Alignment Phase:
  offset = address[5:0]                    // 6-bit offset (0-63)
  bytes_to_boundary = (offset == 0) ? 0 : 64 - offset

  Transfer Phases:
  Phase 1 (Alignment): Transfer bytes_to_boundary to reach 64B boundary
  Phase 2 (Streaming): Transfer full 64B beats
  Phase 3 (Final):     Transfer remaining < 64 bytes

### 29.3.3	2.3 Alignment Information Structure

typedef struct packed {
    logic [15:0] first_chunk_enables;    // Phase 1 chunk mask
    logic [15:0] streaming_chunk_enables; // Phase 2 (always 0xFFFF)
    logic [15:0] final_chunk_enables;     // Phase 3 chunk mask
    logic [31:0] first_transfer_bytes;    // Phase 1 size
    logic [31:0] streaming_bytes;         // Phase 2 size
    logic [31:0] final_transfer_bytes;    // Phase 3 size
    logic [3:0]  first_start_chunk;       // Starting chunk (0-15)
    logic [3:0]  first_valid_chunks;      // Chunks in phase 1
    logic [3:0]  final_valid_chunks;      // Chunks in phase 3
} alignment_info_t;

### 29.3.4	2.4 AXI Translation

Transfer Phase    AxSIZE    AxBURST    Address Increment
────────────────────────────────────────────────────────
Alignment         3'b010    INCR       4 bytes × (AxLEN+1)
Streaming         3'b110    INCR       64 bytes × (AxLEN+1)
Final             3'b010    INCR       4 bytes × (AxLEN+1)

## 29.4	3. AXI Protocol Burst Modes

The AXI protocol defines three burst types that constrain address generation:

### 29.4.1	3.1 FIXED Burst (AxBURST = 2’b00)

Address Formula:
  beat[n].addr = AxADDR   (constant for all beats)

Use Cases: - FIFO access (single address register) - Memory-mapped I/O ports - Audio codec sample registers

Constraints: - All beats transfer to same address - Cannot cross 4KB boundary (trivially satisfied)

### 29.4.2	3.2 INCREMENT Burst (AxBURST = 2’b01)

Address Formula:
  beat[n].addr = AxADDR + n × (1 << AxSIZE)

  Example (AxSIZE=6, 64-byte):
    beat[0] = 0x1000
    beat[1] = 0x1040
    beat[2] = 0x1080
    ...

Use Cases: - Linear memory access - Block copy operations - Stream data to/from memory

Constraints: - Cannot cross 4KB boundary - Maximum burst length varies by protocol version

### 29.4.3	3.3 WRAP Burst (AxBURST = 2’b10)

Address Formula:
  wrap_boundary = AxADDR & ~((AxLEN+1) × (1<<AxSIZE) - 1)
  wrap_size = (AxLEN+1) × (1 << AxSIZE)

  beat[n].addr = wrap_boundary + ((AxADDR + n×(1<<AxSIZE)) mod wrap_size)

  Example (AxADDR=0x1020, AxLEN=3, AxSIZE=4):
    Wrap boundary: 0x1000
    Wrap size: 64 bytes (4 × 16)
    beat[0] = 0x1020
    beat[1] = 0x1030
    beat[2] = 0x1000  (wrapped)
    beat[3] = 0x1010

Use Cases: - Cache line fills (critical-word-first) - Circular buffer access

Constraints: - AxLEN must be 1, 3, 7, or 15 - Starting address must be aligned to transfer size

## 29.5	4. Linear Address Increment Patterns

### 29.5.1	4.1 Simple Contiguous

The most basic pattern - sequential addresses:

Descriptor Fields:
  src_addr:  Starting source address
  dst_addr:  Starting destination address
  length:    Total transfer size

Address Calculation:
  for each beat b in [0, length/beat_size):
    src = src_addr + b × beat_size
    dst = dst_addr + b × beat_size

RAPIDS Implementation: Native support via beat-based transfers.

### 29.5.2	4.2 Unaligned Start/End

Handling transfers that don’t start or end on natural boundaries:

Example: Transfer 200 bytes from 0x1040
  Phase 1 (Align):  Transfer 64 bytes (0x1040-0x107F) to reach 0x1080
  Phase 2 (Stream): Transfer 128 bytes (0x1080-0x10FF) in 2 full beats
  Phase 3 (Final):  Transfer 8 bytes (0x1100-0x1107)

RAPIDS Implementation: Native support via alignment_info_t structure.

### 29.5.3	4.3 Size-Constrained Bursts

Breaking large transfers into AXI-compliant bursts:

Constraints:
  - AXI4: Max 256 beats per burst
  - No 4KB boundary crossing

Address Calculation:
  remaining = total_length
  current_addr = start_addr

  while (remaining > 0):
    burst_len = min(remaining, 256×beat_size)
    burst_len = min(burst_len, next_4kb_boundary - current_addr)
    issue_burst(current_addr, burst_len)
    current_addr += burst_len
    remaining -= burst_len

RAPIDS Implementation: Handled by AXI engine burst splitting.

## 29.6	5. 2D Transfer Patterns (Stride-Based)

### 29.6.1	5.1 Concept

2D transfers move rectangular regions where source and destination may have different memory layouts (pitches/strides).

Memory Layout:
  ┌─────────────────────────────────────┐
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 0: Transfer 4 elements
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 1: Transfer 4 elements
  │ █ █ █ █ . . . . . . . . . . . . . . │  Row 2: Transfer 4 elements
  │ . . . . . . . . . . . . . . . . . . │
  └─────────────────────────────────────┘
   ←─────── src_stride (pitch) ────────→
   ←─ width ─→

### 29.6.2	5.2 Descriptor Fields (Extended)

2D Descriptor Format:
  src_addr:    Starting source address (top-left corner)
  dst_addr:    Starting destination address
  x_length:    Bytes per row (width of transfer region)
  y_length:    Number of rows
  src_stride:  Source bytes between row starts
  dst_stride:  Destination bytes between row starts

### 29.6.3	5.3 Address Calculation

for row in [0, y_length):
  row_src = src_addr + row × src_stride
  row_dst = dst_addr + row × dst_stride
  transfer(row_src, row_dst, x_length)

### 29.6.4	5.4 Use Cases

Image Processing:

Example: Extract 640×480 ROI from 1920×1080 frame (32-bit pixels)
  src_addr:   frame_base + (y_offset × 1920 + x_offset) × 4
  dst_addr:   roi_buffer
  x_length:   640 × 4 = 2560 bytes
  y_length:   480
  src_stride: 1920 × 4 = 7680 bytes
  dst_stride: 640 × 4 = 2560 bytes (packed destination)

Video Frame Tiling:

Example: Copy 16×16 macroblock
  x_length:   16 × bytes_per_pixel
  y_length:   16
  src_stride: frame_width × bytes_per_pixel
  dst_stride: 16 × bytes_per_pixel

### 29.6.5	5.5 Analog Devices AXI DMAC Implementation

Reference implementation from ADI’s high-speed DMA controller:

Registers:
  X_LENGTH  (0x418): Row width - 1
  Y_LENGTH  (0x41C): Row count - 1
  SRC_STRIDE (0x424): Source row spacing
  DEST_STRIDE (0x420): Destination row spacing

Address Formulas:
  ROW_SRC_ADDRESS = SRC_ADDRESS + SRC_STRIDE × N
  ROW_DEST_ADDRESS = DEST_ADDRESS + DEST_STRIDE × N

## 29.7	6. 3D Transfer Patterns

### 29.7.1	6.1 Concept

3D transfers extend 2D with an additional depth/plane dimension:

Memory Layout (Z planes stacked):
  Plane 0:    Plane 1:    Plane 2:
  ┌───────┐   ┌───────┐   ┌───────┐
  │ █ █ █ │   │ █ █ █ │   │ █ █ █ │
  │ █ █ █ │   │ █ █ █ │   │ █ █ █ │
  └───────┘   └───────┘   └───────┘

### 29.7.2	6.2 Descriptor Fields (Extended)

3D Descriptor Format:
  src_addr:     Starting address (origin corner)
  dst_addr:     Destination address
  x_length:     Width in bytes
  y_length:     Height in rows
  z_length:     Depth in planes
  src_stride_x: Source row stride (unused, implicit x_length)
  src_stride_y: Source row-to-row stride
  src_stride_z: Source plane-to-plane stride
  dst_stride_y: Destination row stride
  dst_stride_z: Destination plane stride

### 29.7.3	6.3 Address Calculation

for plane in [0, z_length):
  for row in [0, y_length):
    src = src_addr + plane×src_stride_z + row×src_stride_y
    dst = dst_addr + plane×dst_stride_z + row×dst_stride_y
    transfer(src, dst, x_length)

### 29.7.4	6.4 Use Cases

3D Medical Imaging:

Example: Extract 64×64×64 VOI from 512×512×256 CT volume
  src_addr:     volume_base + z_off×512×512 + y_off×512 + x_off
  x_length:     64 bytes
  y_length:     64
  z_length:     64
  src_stride_y: 512
  src_stride_z: 512 × 512

Neural Network Tensors:

Example: Move 4D tensor slice [batch, channel, H, W]
  Requires nested 3D operations or generalized tensor addressing

## 29.8	7. Scatter-Gather Patterns

### 29.8.1	7.1 Concept

Scatter-Gather (SG) handles non-contiguous memory regions using linked lists of descriptors:

Descriptor Chain:
  ┌──────────┐     ┌──────────┐     ┌──────────┐
  │ Desc 0   │────→│ Desc 1   │────→│ Desc 2   │────→ NULL
  │ addr=A0  │     │ addr=A1  │     │ addr=A2  │
  │ len=L0   │     │ len=L1   │     │ len=L2   │
  └──────────┘     └──────────┘     └──────────┘

### 29.8.2	7.2 SG Descriptor Fields

Scatter-Gather Descriptor:
  buffer_addr:      Address of this fragment
  buffer_length:    Size of this fragment
  next_desc_ptr:    Address of next descriptor (0 = last)
  control_flags:    Completion interrupt, error handling, etc.
  status:           Completion status (written by DMA)

### 29.8.3	7.3 Operation Modes

Gather (Multiple Source -> Single Destination):

Source fragments: [A0:L0], [A1:L1], [A2:L2]
Destination: Contiguous buffer B

Result: B = concat(mem[A0:A0+L0], mem[A1:A1+L1], mem[A2:A2+L2])

Scatter (Single Source -> Multiple Destinations):

Source: Contiguous buffer B of length L0+L1+L2
Destination fragments: [A0:L0], [A1:L1], [A2:L2]

Result: mem[A0:A0+L0] = B[0:L0]
        mem[A1:A1+L1] = B[L0:L0+L1]
        mem[A2:A2+L2] = B[L0+L1:end]

### 29.8.4	7.4 AMD/Xilinx AXI DMA SG Descriptor Format

Standard Descriptor (32-bit addressing):
  Offset  Field            Description
  ──────────────────────────────────────────────
  0x00    NXTDESC          Next descriptor pointer
  0x04    Reserved
  0x08    BUFFER_ADDRESS   Data buffer pointer
  0x0C    Reserved
  0x10    Reserved
  0x14    Reserved
  0x18    CONTROL          Transfer length, SOF, EOF
  0x1C    STATUS           Completion status

### 29.8.5	7.5 Use Cases

Network Packet Assembly:

Example: Assemble TCP packet from scattered headers and payload
  Desc 0: Ethernet header (14 bytes at addr_eth)
  Desc 1: IP header (20 bytes at addr_ip)
  Desc 2: TCP header (20 bytes at addr_tcp)
  Desc 3: Payload (1460 bytes at addr_payload)

RAPIDS Implementation: Native support via next_descriptor_ptr chaining.

## 29.9	8. Circular and Ring Buffer Patterns

### 29.9.1	8.1 Concept

Circular buffers wrap addresses at a boundary, enabling continuous streaming without descriptor reload:

Memory Layout:
  ┌─────────────────────────────────────┐
  │ 7 │ 0 │ 1 │ 2 │ 3 │ 4 │ 5 │ 6 │ 7 │ (wraps)
  └─────────────────────────────────────┘
      ↑               ↑
    write           read

### 29.9.2	8.2 Descriptor Fields (Extended)

Circular Buffer Descriptor:
  base_addr:    Buffer base address
  buffer_size:  Total buffer size (must be power of 2 for efficient wrap)
  current_ptr:  Current read/write position
  wrap_enable:  Enable address wrapping

### 29.9.3	8.3 Address Calculation

Efficient wrap (power-of-2 size):
  next_addr = base_addr + ((current_ptr + increment) & (buffer_size - 1))

General wrap:
  offset = (current_ptr + increment) % buffer_size
  next_addr = base_addr + offset

### 29.9.4	8.4 Ping-Pong Double Buffering

Common implementation using two descriptors pointing to each other:

Descriptor A:               Descriptor B:
  buffer_addr = BUF_A        buffer_addr = BUF_B
  next_desc = &Desc_B        next_desc = &Desc_A

Operation:
  While DMA processes BUF_A, CPU fills BUF_B
  On completion, DMA auto-loads Desc_B, starts BUF_B
  CPU processes BUF_A while DMA runs BUF_B
  (repeat)

### 29.9.5	8.5 Use Cases

Audio Streaming:

Example: 48kHz stereo audio, 20ms buffers
  buffer_size: 48000 × 2 channels × 2 bytes × 0.02s = 3840 bytes
  Ring with 4 buffers: 15360 bytes total
  Wrap at: base + 15360

Network RX Ring:

Example: Ethernet receive ring
  descriptor_count: 256 (power of 2)
  Each descriptor points to packet buffer
  Last descriptor.next points to first descriptor
  Hardware auto-advances through ring

## 29.10	9. Tensor Accelerator Memory Access Patterns (PRIORITY 0)

Tensor accelerators for neural network inference represent the most demanding and sophisticated address generation requirements. This section provides comprehensive coverage of industry-standard approaches.

### 29.10.1	9.1 Systolic Array Dataflow Fundamentals

Systolic arrays (used in Google TPU, NVIDIA Tensor Cores, etc.) require coordinated data movement across three matrices: inputs (activations), weights, and outputs (partial sums).

Three Primary Dataflows:

| Dataflow | Stationary Data | Moving Data | Best For |
| --- | --- | --- | --- |
| Weight Stationary (WS) | Weights preloaded in PEs | Inputs + partial sums flow | Weight reuse (many inputs) |
| Input Stationary (IS) | Inputs held in PEs | Weights + partial sums flow | Input reuse (many kernels) |
| Output Stationary (OS) | Partial sums accumulate in PEs | Inputs + weights flow | Output accumulation |

Address Pattern Implications:

Weight Stationary:
  - Weights: Load once per layer, stream through array
  - Inputs: Stream row-by-row, reuse across weight tiles
  - Address pattern: 2D tiled with weight-tile-aligned boundaries

Output Stationary:
  - Outputs: Accumulate in place
  - Inputs/Weights: Double-buffered streaming
  - Address pattern: Output-tile-centric addressing

### 29.10.2	9.2 Data Cube Model (NVDLA Reference)

NVIDIA’s Deep Learning Accelerator (NVDLA) defines tensor addressing using a “data cube” model with three key stride parameters:

Data Cube Structure:

Dimensions: Width (W) × Height (H) × Channels (C)

Memory Layout:
  - Data divided into 1×1×32byte "atom cubes"
  - Scanning order: C'(32B) → W → H → C (surfaces)
  - C' changes fastest (innermost loop)

Address Calculation:
  element_addr = base_addr
                 + surface_index × surface_stride
                 + line_index × line_stride
                 + element_offset_within_line

Stride Parameters:

| Parameter | Description | Alignment |
| --- | --- | --- |
| line_stride | Bytes from line N to line N+1 | 32 bytes |
| surface_stride | Bytes from surface N to surface N+1 | 32 bytes |
| base_addr | Starting address | 32 bytes |

NVDLA Register Configuration:

D_DATAIN_FORMAT     : Data format selection
D_DATAIN_SIZE_0     : Width × Height
D_DATAIN_SIZE_1     : Channels
D_LINE_STRIDE       : Line-to-line spacing (0x5040)
D_SURF_STRIDE       : Surface-to-surface spacing

Example:
  Width=26, Height=32, Channels=3
  line_stride = 0x1A0 (aligned width)
  surface_stride = 0x1520 (line_stride × height)

### 29.10.3	9.3 Tensor Memory Accelerator (TMA) - NVIDIA Hopper

NVIDIA’s Hopper architecture introduces hardware TMA for efficient multi-dimensional tensor transfers:

TMA Descriptor Format:

CUtensorMap structure (64-bit packed descriptor):
  - Global memory base address
  - Tensor rank (1D to 5D supported)
  - Dimension sizes and strides
  - Swizzle pattern for bank conflict avoidance
  - Box dimensions (tile size for transfer)

Key Constraints:

Alignment Requirements:
  - Contiguous dimension must have stride=1
  - All other strides must be multiples of 16 bytes
  - For float tensors [M,N] with stride [N,1]: N % 4 == 0

Tile Quantization:
  - TPU/GPU systolic arrays process 128×128 or 128×8 tiles
  - Tensors must align to tile boundaries to avoid padding waste
  - Performance penalty for partial tiles (idle MACs)

Address Generation (Hardware):

TMA eliminates per-thread address calculation:
  Traditional: Each thread computes addr = base + thread_id × stride
  TMA: Single descriptor, hardware generates all addresses

Coordinate-based access:
  ArithTuple(row, col) → Hardware translates to physical address
  Automatic out-of-bounds predication (no manual boundary checks)

### 29.10.4	9.4 Convolution Address Patterns

#### 29.10.4.1	9.4.1 Direct Convolution

Input Feature Map:  [N, C_in, H_in, W_in]
Weight Kernel:      [C_out, C_in, K_h, K_w]
Output Feature Map: [N, C_out, H_out, W_out]

Output position (n, c_out, h_out, w_out):
  For each (c_in, k_h, k_w):
    input_addr = base_in + n×(C_in×H_in×W_in)
                         + c_in×(H_in×W_in)
                         + (h_out×stride_h + k_h)×W_in
                         + (w_out×stride_w + k_w)

    weight_addr = base_w + c_out×(C_in×K_h×K_w)
                         + c_in×(K_h×K_w)
                         + k_h×K_w
                         + k_w

#### 29.10.4.2	9.4.2 Im2Col Transformation

Converts convolution to GEMM for systolic array efficiency:

Original Convolution:
  Output[h,w] = sum(Input[h+kh, w+kw] × Weight[kh, kw])

Im2Col Transformation:
  - Unroll each receptive field into a column
  - Weight kernels become rows
  - Convolution becomes matrix multiplication

Address Pattern for Im2Col:
  For output position (h_out, w_out):
    col_index = h_out × W_out + w_out
    For kernel position (c_in, k_h, k_w):
      row_index = c_in × K_h × K_w + k_h × K_w + k_w
      src_addr = base + c_in×(H×W) + (h_out×s_h+k_h)×W + (w_out×s_w+k_w)
      dst_addr = im2col_base + row_index × num_output_positions + col_index

Data Expansion Factor:
  Naive: K_h × K_w × data replication
  Hardware Im2Col: Generate addresses on-the-fly, no physical expansion

#### 29.10.4.3	9.4.3 Strided and Dilated Convolution

Strided Convolution (stride > 1):
  input_h = output_h × stride + kernel_h
  input_w = output_w × stride + kernel_w
  Reduces output spatial dimensions

Dilated/Atrous Convolution:
  input_h = output_h + kernel_h × dilation
  input_w = output_w + kernel_w × dilation
  Expands receptive field without increasing parameters

Address Modification:
  base_addr + (h × stride + kh × dilation) × line_stride
            + (w × stride + kw × dilation) × element_size

#### 29.10.4.4	9.4.4 Depthwise Separable Convolution

Standard Convolution: C_in × C_out × K × K multiplications
Depthwise Separable: C_in × K × K + C_in × C_out multiplications

Phase 1 - Depthwise (per-channel):
  For each channel c:
    output[c, h, w] = conv2d(input[c], kernel[c])

  Address Pattern:
    Each channel processes independently
    No cross-channel data movement
    input_addr = base + c × H × W + h × W + w

Phase 2 - Pointwise (1×1 convolution):
  Standard GEMM across channels
  input_addr = base + c × (H × W) + position

### 29.10.5	9.5 Gemmini Accelerator Architecture (UC Berkeley Reference)

Open-source systolic array generator with well-documented DMA and addressing:

Memory Hierarchy:

┌─────────────────────────────────────────────┐
│                 Main Memory                  │
│              (Virtual Addresses)             │
└────────────────────┬────────────────────────┘
                     │ DMA (TileLink)
                     ▼
┌─────────────────────────────────────────────┐
│              Scratchpad SRAM                 │
│   (Banked, Row-Addressed, Explicit Mgmt)    │
│   Default: 256KB, 16 elements/row           │
└────────────────────┬────────────────────────┘
                     │
                     ▼
┌─────────────────────────────────────────────┐
│         Systolic Array (16×16 PEs)          │
│    Weight Stationary or Output Stationary   │
└────────────────────┬────────────────────────┘
                     │
                     ▼
┌─────────────────────────────────────────────┐
│            Accumulator SRAM                  │
│   (32-bit accumulators, in-place addition)  │
└─────────────────────────────────────────────┘

Scratchpad Address Format (32-bit):

Bit 31:    0=Scratchpad, 1=Accumulator
Bit 30:    Accumulator mode (0=overwrite, 1=accumulate)
Bit 29:    Read format (0=scaled inputType, 1=raw accType)
Bits 28:0: Row address within selected memory

Row Width:
  Scratchpad: DIM × inputType bits (default: 16 × 8 = 128 bits)
  Accumulator: DIM × accType bits (default: 16 × 32 = 512 bits)

DMA Operations:

// mvin: Move data from DRAM to scratchpad
// Configuration registers set stride
mvin(
    dram_addr,           // Virtual address (64-bit)
    sp_addr,             // Scratchpad row address
    rows,                // Number of rows to transfer
    cols                 // Elements per row (≤ DIM)
);

// mvout: Move data from accumulator to DRAM
// Supports integrated max-pooling
mvout(
    dram_addr,           // Destination address
    acc_addr,            // Accumulator row address
    rows, cols,
    pool_size,           // Optional pooling window
    pool_stride
);

Stride Configuration:

config_mvin:
  stride[0]: Source memory stride for mvin_A
  stride[1]: Source memory stride for mvin_B
  stride[2]: Source memory stride for mvin_D

config_mvout:
  stride: Destination memory stride
  pool_params: Pooling window and stride

### 29.10.6	9.6 Tiling for Large Matrices

When matrices exceed on-chip memory, tiling is essential:

Hierarchical Tiling (CUTLASS Model):

Level 1 - Thread Block Tile:
  Load tile from global memory to shared memory
  Tile size: M_tile × K_tile (A), K_tile × N_tile (B)

Level 2 - Warp Tile:
  Load from shared memory to registers
  Tile size: Warp_M × Warp_K (A), Warp_K × Warp_N (B)

Level 3 - Thread Tile:
  Individual thread's computation
  Register-to-register operations

Address Generation per Level:
  Global: base + block_id × block_tile_size
  Shared: smem_base + warp_id × warp_tile_size
  Register: Direct indexing within tile

Double-Buffering for Latency Hiding:

Buffer Structure:
  tile_buffer[2][TILE_ROWS][TILE_COLS]

Pipeline:
  Cycle N:   Compute on buffer[0], Load to buffer[1]
  Cycle N+1: Compute on buffer[1], Load to buffer[0]

Address Pattern:
  compute_addr = base + (cycle % 2) × tile_size + element_offset
  load_addr    = base + ((cycle + 1) % 2) × tile_size + element_offset

### 29.10.7	9.7 Sparse Tensor Patterns

Sparse accelerators require index-based indirect addressing:

Compressed Sparse Row (CSR) Format:

Dense: [a 0 b]    CSR: values = [a, b, c, d]
       [0 c 0]         col_idx = [0, 2, 1, 0]
       [d 0 0]         row_ptr = [0, 2, 3, 4]

Address Calculation:
  For row r, iterate col_idx[row_ptr[r]] to col_idx[row_ptr[r+1]-1]
  value_addr = values_base + idx
  col_addr = col_idx_base + idx

Indirect Access:
  actual_col = memory[col_addr]
  dense_addr = base + r × stride + actual_col × element_size

Hardware Implications:

Dense Accelerator (TPU): Initiation interval = 1 (fully pipelined)
Sparse Accelerator (EIE): Initiation interval = 17 (metadata overhead)

Trade-off:
  - Sparse saves memory bandwidth (skip zeros)
  - But adds index indirection latency
  - Effective for >90% sparsity

### 29.10.8	9.8 Transformer/Attention Patterns

Modern LLM accelerators require specialized patterns:

Q, K, V Matrix Generation:

Input: X [seq_len, d_model]
Weights: W_Q, W_K, W_V [d_model, d_k]

Q = X × W_Q  →  [seq_len, d_k]
K = X × W_K  →  [seq_len, d_k]
V = X × W_V  →  [seq_len, d_k]

Address Pattern: Standard GEMM with different weight matrices
  q_addr = q_base + seq_idx × d_k + head_idx
  k_addr = k_base + seq_idx × d_k + head_idx
  v_addr = v_base + seq_idx × d_k + head_idx

Attention Score Computation:

Attention = softmax(Q × K^T / sqrt(d_k)) × V

Memory Challenge:
  Q × K^T produces [seq_len × seq_len] matrix
  O(n^2) memory for sequence length n

FlashAttention Tiling:
  Tile Q, K, V into blocks that fit in SRAM
  Compute attention incrementally without materializing full matrix

  Tile Address Pattern:
    q_tile_addr = q_base + tile_row × tile_size × d_k
    k_tile_addr = k_base + tile_col × tile_size × d_k

  Online Softmax:
    Track running max and sum across tiles
    No need to store intermediate attention matrix

### 29.10.9	9.9 Programmable Address Generation Unit (PAGU)

Academic reference for flexible tensor addressing:

PAGU Instruction Set:

Operations Supported:
  - Standard Convolution
  - Padded Convolution
  - Strided Convolution
  - Dilated (Atrous) Convolution
  - Pooling (Max, Average)
  - Upsampled Convolution
  - Transposed Convolution

Performance: 1.7 cycles per address for convolution
Area Overhead: ~4.6× vs fixed datapath (justified by flexibility)

Configuration Registers:

tensor_config_t:
  base_addr[63:0]           // Tensor base address
  dim_sizes[4][31:0]        // Size of each dimension
  dim_strides[4][31:0]      // Stride of each dimension
  kernel_size[15:0]         // Convolution kernel size
  conv_stride[7:0]          // Convolution stride
  dilation[7:0]             // Dilation factor
  padding[7:0]              // Padding amount
  operation_mode[3:0]       // Select operation type

Address Generation FSM:

always_ff @(posedge clk) begin
    case (state)
        IDLE: begin
            if (start) begin
                dim_counters <= '0;
                state <= GENERATE;
            end
        end

        GENERATE: begin
            // Multi-dimensional address calculation
            addr <= base_addr;
            for (int d = 0; d < num_dims; d++) begin
                case (operation_mode)
                    CONV_STANDARD:
                        addr <= addr + dim_counters[d] * dim_strides[d];
                    CONV_STRIDED:
                        addr <= addr + (dim_counters[d] * conv_stride) * dim_strides[d];
                    CONV_DILATED:
                        addr <= addr + (dim_counters[d] * dilation) * dim_strides[d];
                endcase
            end

            // Increment counters (innermost first)
            increment_counters();

            if (done) state <= IDLE;
        end
    endcase
end

## 29.11	10. Implementation Comparison Matrix

### 29.11.1	10.1 Pattern Complexity vs. Capability

| Pattern | Descriptor Fields | Address Logic | Hardware Cost | SW Flexibility |
| --- | --- | --- | --- | --- |
| Linear | 3 (src, dst, len) | Simple add | Low | Limited |
| 2D Stride | 6 (+x_len, y_len, strides) | Nested loop | Medium | Good |
| 3D Volume | 9 (+z_len, z_strides) | Triple nested | Medium-High | Very Good |
| Scatter-Gather | 4 per fragment | List traversal | Medium | Excellent |
| Circular | 4 (+size, wrap) | Modulo/mask | Low-Medium | Good |
| Tensor | N (variable stride) | Sum-of-products | High | Excellent |

### 29.11.2	10.2 Use Case Mapping

| Application Domain | Primary Pattern | Secondary Pattern |
| --- | --- | --- |
| Block memory copy | Linear | - |
| Network packets | Scatter-Gather | Linear |
| Audio streaming | Circular | Linear |
| Image processing | 2D Stride | Linear |
| Video encode/decode | 2D Stride | Scatter-Gather |
| 3D graphics | 3D Volume | 2D Stride |
| Neural networks | Tensor | 2D Stride |
| Database operations | Scatter-Gather | Linear |

### 29.11.3	10.3 RAPIDS Current vs. Extended Capability

| Capability | Current RAPIDS | Industry Standard |
| --- | --- | --- |
| Linear contiguous | YES | YES |
| Unaligned transfers | YES (3-phase) | Varies |
| Descriptor chaining | YES | YES |
| 2D stride | NO | Common |
| 3D volume | NO | Rare |
| Scatter-Gather | Partial (chain) | Full SG |
| Circular wrap | NO | Common |
| Tensor addressing | NO | Emerging |

## 29.12	11. RAPIDS Extension Recommendations

### 29.12.1	11.0 PRIORITY 0: Tensor Accelerator Addressing (CRITICAL)

Rationale: Essential for AI/ML workloads. Tensor accelerators represent the highest-growth market for DMA engines. Without tensor addressing, RAPIDS cannot serve neural network inference applications.

Implementation Approach: Data Cube Model (NVDLA-style)

This approach provides the best balance of flexibility and hardware complexity, supporting convolution, pooling, and GEMM operations.

Extended Descriptor Format:

typedef struct packed {
    // === Base Fields (existing, 256 bits) ===
    logic [63:0]  src_addr;           // Base source address
    logic [63:0]  dst_addr;           // Base destination address
    logic [31:0]  length;             // For linear mode: beats
    logic [31:0]  next_descriptor_ptr;
    logic [7:0]   desc_priority;
    logic [3:0]   channel_id;
    logic         error;
    logic         last;
    logic         gen_irq;
    logic         valid;

    // === Tensor Extension (repurpose reserved bits + add 2nd descriptor word) ===
    // Mode selection
    logic [3:0]   transfer_mode;      // 0=linear, 1=2D, 2=3D/tensor, 3=conv, 4=pool

    // Data cube dimensions
    logic [15:0]  width;              // W dimension (elements)
    logic [15:0]  height;             // H dimension (lines)
    logic [15:0]  channels;           // C dimension (surfaces)

    // Stride parameters (32-byte aligned)
    logic [31:0]  line_stride;        // Bytes between lines
    logic [31:0]  surface_stride;     // Bytes between surfaces

    // Convolution parameters (when transfer_mode == CONV)
    logic [7:0]   kernel_width;       // Kw
    logic [7:0]   kernel_height;      // Kh
    logic [7:0]   conv_stride_x;      // Horizontal stride
    logic [7:0]   conv_stride_y;      // Vertical stride
    logic [7:0]   dilation_x;         // Horizontal dilation
    logic [7:0]   dilation_y;         // Vertical dilation
    logic [7:0]   pad_left;           // Left padding
    logic [7:0]   pad_right;          // Right padding
    logic [7:0]   pad_top;            // Top padding
    logic [7:0]   pad_bottom;         // Bottom padding

    // Pooling parameters (when transfer_mode == POOL)
    logic [7:0]   pool_width;
    logic [7:0]   pool_height;
    logic [7:0]   pool_stride_x;
    logic [7:0]   pool_stride_y;
    logic [1:0]   pool_type;          // 0=max, 1=avg, 2=min

    // Data type and precision
    logic [3:0]   data_type;          // 0=int8, 1=int16, 2=fp16, 3=bf16, 4=fp32
    logic [3:0]   element_size;       // Bytes per element (1, 2, 4)
} descriptor_tensor_t;

Address Generation Unit (AGU) Architecture:

module tensor_address_generator #(
    parameter int PIPE_STAGES = 3     // Pipeline for multiply-accumulate
) (
    input  logic        clk,
    input  logic        rst_n,

    // Configuration from descriptor
    input  descriptor_tensor_t desc,
    input  logic        start,

    // Address output stream
    output logic [63:0] addr,
    output logic        addr_valid,
    input  logic        addr_ready,

    // Status
    output logic        done
);

    // Dimension counters
    logic [15:0] w_cnt, h_cnt, c_cnt;
    logic [7:0]  kw_cnt, kh_cnt;       // Kernel position (for conv)

    // Pipeline registers for address calculation
    logic [63:0] addr_stage[PIPE_STAGES];

    // FSM states
    typedef enum logic [2:0] {
        IDLE,
        CALC_BASE,
        CALC_LINE,
        CALC_SURFACE,
        OUTPUT
    } state_t;
    state_t state;

    // Address calculation based on mode
    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            state <= IDLE;
            w_cnt <= '0;
            h_cnt <= '0;
            c_cnt <= '0;
        end else begin
            case (state)
                IDLE: begin
                    if (start) begin
                        w_cnt <= '0;
                        h_cnt <= '0;
                        c_cnt <= '0;
                        state <= CALC_BASE;
                    end
                end

                CALC_BASE: begin
                    // Stage 1: Base address
                    addr_stage[0] <= desc.src_addr;
                    state <= CALC_LINE;
                end

                CALC_LINE: begin
                    // Stage 2: Add line offset
                    case (desc.transfer_mode)
                        MODE_LINEAR:
                            addr_stage[1] <= addr_stage[0] + (w_cnt * desc.element_size);

                        MODE_2D, MODE_3D:
                            addr_stage[1] <= addr_stage[0]
                                           + (h_cnt * desc.line_stride)
                                           + (w_cnt * desc.element_size);

                        MODE_CONV:
                            // Convolution: account for stride and kernel position
                            addr_stage[1] <= addr_stage[0]
                                           + ((h_cnt * desc.conv_stride_y + kh_cnt * desc.dilation_y) * desc.line_stride)
                                           + ((w_cnt * desc.conv_stride_x + kw_cnt * desc.dilation_x) * desc.element_size);

                        MODE_POOL:
                            addr_stage[1] <= addr_stage[0]
                                           + ((h_cnt * desc.pool_stride_y) * desc.line_stride)
                                           + ((w_cnt * desc.pool_stride_x) * desc.element_size);
                    endcase
                    state <= CALC_SURFACE;
                end

                CALC_SURFACE: begin
                    // Stage 3: Add surface offset (for 3D/tensor)
                    if (desc.transfer_mode inside {MODE_3D, MODE_CONV}) begin
                        addr_stage[2] <= addr_stage[1] + (c_cnt * desc.surface_stride);
                    end else begin
                        addr_stage[2] <= addr_stage[1];
                    end
                    state <= OUTPUT;
                end

                OUTPUT: begin
                    if (addr_ready) begin
                        // Increment counters (innermost first)
                        if (desc.transfer_mode == MODE_CONV) begin
                            // Convolution: iterate kernel, then spatial, then channels
                            if (kw_cnt < desc.kernel_width - 1) begin
                                kw_cnt <= kw_cnt + 1;
                            end else begin
                                kw_cnt <= '0;
                                if (kh_cnt < desc.kernel_height - 1) begin
                                    kh_cnt <= kh_cnt + 1;
                                end else begin
                                    kh_cnt <= '0;
                                    // Continue to spatial/channel iteration
                                    increment_spatial_counters();
                                end
                            end
                        end else begin
                            increment_spatial_counters();
                        end

                        if (transfer_complete())
                            state <= IDLE;
                        else
                            state <= CALC_BASE;
                    end
                end
            endcase
        end
    end

    assign addr = addr_stage[PIPE_STAGES-1];
    assign addr_valid = (state == OUTPUT);
    assign done = (state == IDLE) && !start;

endmodule

Scratchpad Integration (Gemmini-style):

// Memory address format for tensor scratchpad
typedef struct packed {
    logic        mem_select;       // 0=scratchpad, 1=accumulator
    logic        acc_mode;         // 0=overwrite, 1=accumulate
    logic        read_format;      // 0=scaled, 1=raw
    logic [28:0] row_addr;         // Row address within memory
} scratchpad_addr_t;

// DMA configuration for tensor loads
typedef struct packed {
    logic [63:0] dram_addr;        // Virtual address in main memory
    scratchpad_addr_t sp_addr;     // Scratchpad destination
    logic [15:0] rows;             // Number of rows to transfer
    logic [15:0] cols;             // Elements per row
    logic [31:0] dram_stride;      // Source memory stride
    logic [15:0] sp_stride;        // Scratchpad row stride (usually 1)
} tensor_dma_config_t;

Alignment Requirements:

All tensor addresses must satisfy:
  - base_addr aligned to 32 bytes (NVDLA requirement)
  - line_stride aligned to 32 bytes
  - surface_stride aligned to 32 bytes
  - For optimal performance: align to 64 bytes (RAPIDS beat size)

Verification:
  assert((desc.src_addr & 32'h1F) == 0);
  assert((desc.line_stride & 32'h1F) == 0);
  assert((desc.surface_stride & 32'h1F) == 0);

Performance Considerations:

Address Generation Rate:
  - Target: 1 address per cycle (pipelined)
  - Convolution: 1.7 cycles/address typical (due to kernel iteration)

Memory Bandwidth:
  - TPU-class: 600+ GB/s (HBM)
  - FPGA-class: 20-80 GB/s (DDR4/5)
  - RAPIDS target: Match AXI interface bandwidth

Tiling Support:
  - Hardware computes tile boundaries
  - Double-buffering via ping-pong descriptors
  - Software provides tile dimensions, hardware handles addressing

### 29.12.2	11.1 Priority 1: 2D Stride Support

Rationale: High value for image/video processing with moderate complexity. Also serves as foundation for tensor addressing.

Descriptor Extension:

typedef struct packed {
    // Existing fields (256 bits)...

    // 2D extension (96 bits in reserved space)
    logic [31:0] x_length;        // Bytes per row
    logic [15:0] y_length;        // Number of rows
    logic [31:0] src_stride;      // Source row stride
    logic [31:0] dst_stride;      // Destination row stride
    logic        mode_2d;         // Enable 2D mode
} descriptor_2d_t;

Address Generation:

always_comb begin
    if (mode_2d) begin
        row_src_addr = src_addr + current_row * src_stride;
        row_dst_addr = dst_addr + current_row * dst_stride;
        row_length   = x_length;
    end else begin
        row_src_addr = src_addr;
        row_dst_addr = dst_addr;
        row_length   = length * BYTES_PER_BEAT;
    end
end

### 29.12.3	11.2 Priority 2: Circular Buffer Mode

Rationale: Essential for streaming applications, low complexity addition.

Descriptor Extension:

typedef struct packed {
    // Existing fields...

    // Circular buffer extension
    logic [31:0] buffer_size;     // Total buffer size
    logic [31:0] buffer_mask;     // Size-1 for power-of-2
    logic        circular_mode;   // Enable wrap-around
    logic        src_circular;    // Apply to source
    logic        dst_circular;    // Apply to destination
} descriptor_circular_t;

Address Generation:

function automatic logic [63:0] wrap_address(
    logic [63:0] base,
    logic [63:0] offset,
    logic [31:0] mask,
    logic        enable
);
    if (enable)
        return base + (offset & {32'b0, mask});
    else
        return base + offset;
endfunction

### 29.12.4	11.3 Priority 3: Enhanced Scatter-Gather

Rationale: Full SG capability for network and fragmented memory use cases.

Current Limitation: Chain-only SG requires one descriptor per fragment.

Enhancement: Add fragment list support within single descriptor:

typedef struct packed {
    logic [63:0] fragment_list_ptr;  // Pointer to fragment array
    logic [15:0] fragment_count;     // Number of fragments
    logic        sg_mode;            // Enable SG mode

    // Fragment entry format (in memory):
    // [63:0]  address
    // [31:0]  length
    // [31:0]  flags
} descriptor_sg_t;

## 29.13	12. References

### 29.13.1	Industry Documentation

- AMD AXI DMA Product Guide (PG021) - Scatter-Gather descriptor format

- Analog Devices AXI DMAC - 2D transfer implementation

- Understanding AXI Addressing (ZipCPU) - Burst mode analysis

- WRAP Address Calculation (Verification Guide) - Wrap burst formulas

### 29.13.2	GPU and Accelerator Patterns

- NVIDIA CUDA Memory Coalescing - Coalesced access patterns

- CUDA Pitch Linear Memory - 2D array allocation

- NVIDIA TMA Tutorial (CUTLASS) - Tensor Memory Accelerator

- Efficient GEMM in CUDA (CUTLASS) - Hierarchical tiling

### 29.13.3	DMA Architecture

- Intel DMA Descriptors - PCIe DMA linked lists

- DMA330 Microcode - Linked-list implementation

### 29.13.4	Neural Network Accelerators

- NVDLA Hardware Architecture - Data cube model, CDMA

- NVDLA In-Memory Data Formats - Line stride, surface stride

- NVDLA Unit Description - Convolution DMA details

- Gemmini Accelerator (UC Berkeley) - Open-source systolic array

- Gemmini Documentation (Chipyard) - DMA and scratchpad

- Google TPU Architecture - Systolic array overview

- TPU System Architecture - Memory hierarchy

- Programmable AGU for DNNs - Flexible address generation

### 29.13.5	Convolution and Tensor Operations

- Im2Col Characterization - Convolution memory patterns

- ST Neural-ART Programming Model - NPU descriptor concepts

- Systolic Array Dataflows Survey - Weight/input/output stationary

- FlashAttention Analysis - Attention tiling patterns

- Depthwise Separable Convolutions - Channel-wise addressing

### 29.13.6	Sparse Tensor Formats

- FLAASH Sparse Tensor Accelerator - CSR/CSF formats

- SparTen CNN Accelerator - Sparse tensor addressing

- Indirection Stream Architecture - Hardware indirect addressing

## 29.14	Document History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-13 | RTL Design Sherpa | Initial comprehensive analysis |

This document is part of the RAPIDS (Rapid AXI Programmable In-band Descriptor System) design documentation.