RTL Design Sherpa

STREAM Subsystem Micro-Architecture Specification 0.90

January 2, 2026

Table of Contents

1 Stream Index	61

2 Document Information	61

2.1 References	61

2.1.1 Related Documents	61

2.2 Terminology	62

2.3 Revision History	63

3 STREAM Architecture Overview	64

3.1 Introduction	64

3.1.1 Design Philosophy	64

3.2 System Architecture	65

3.2.1 High-Level Block Diagram	65

3.2.2 Figure 1.1.1: STREAM Architecture Overview	65

3.3 Key Architectural Concepts	65

3.3.1 1. Multi-Channel Design	65

3.3.2 2. Descriptor-Based Operation	66

3.3.3 3. Concurrent Read/Write	66

3.3.4 4. Space-Aware Flow Control	66

3.3.5 5. Priority-Based Arbitration	66

3.4 Data Flow Example	67

3.4.1 Single Channel Transfer	67

3.4.2 Figure 1.1.2: Single Channel Transfer Sequence	67

3.5 Component Hierarchy	67

3.5.1 Top-Level Integration (stream_top_ch8.sv)	67

3.5.2 Core DMA Engine (stream_core.sv)	68

3.6 Interface Summary	68

3.6.1 External Interfaces (stream_top_ch8.sv)	68

3.6.2 Internal MonBus Interface (stream_core.sv)	69

3.6.3 Configuration Registers (APB)	69

3.7 Resource Utilization	69

3.7.1 Area Breakdown (Typical 512-bit Data Width)	69

3.8 Performance Characteristics	70

3.8.1 Throughput	70

3.8.2 Latency	70

3.9 Design Decisions	70

3.9.1 Why These Simplifications?	70

3.9.2 Production Enhancements (Future)	71

3.10 Testing Strategy	71

3.10.1 Verification Layers	71

3.10.2 Coverage Targets	71

3.11 Related Documentation	72

3.12 Revision History	72

4 STREAM Top-Level Port List	72

4.1 Overview	73

4.2 Clock and Reset	73

4.3 APB Programming Interface	73

4.4 Configuration Interface	74

4.4.1 Per-Channel Configuration	74

4.4.2 Global Scheduler Configuration	75

4.4.3 Descriptor Engine Configuration	75

4.4.4 AXI Monitor Configuration	76

4.4.5 AXI Transfer Configuration	77

4.4.6 Performance Profiler Configuration	77

4.5 Status Interface	78

4.5.1 Per-Channel Status	78

4.6 Performance Profiler Interface	79

4.7 AXI4 Master - Descriptor Fetch (256-bit)	80

4.7.1 AR Channel (Read Address)	80

4.7.2 R Channel (Read Data)	81

4.8 AXI4 Master - Data Read (Parameterizable Width)	81

4.8.1 AR Channel (Read Address)	81

4.8.2 R Channel (Read Data)	82

4.9 AXI4 Master - Data Write (Parameterizable Width)	83

4.9.1 AW Channel (Write Address)	83

4.9.2 W Channel (Write Data)	84

4.9.3 B Channel (Write Response)	84

4.10 Status/Debug Outputs	85

4.10.1 Descriptor AXI Monitor Status	85

4.10.2 Read Engine AXI Monitor Status	85

4.10.3 Write Engine AXI Monitor Status	86

4.11 Unified Monitor Bus Interface	86

4.12 Port Count Summary	87

4.13 Default Parameter Values	88

4.14 Related Documentation	89

4.15 Revision History	89

5 Clocks and Reset Specification	89

5.1 Overview	90

5.2 Clock Domain	90

5.2.1 Primary Clock: aclk	90

5.2.2 Secondary Clock: pclk (APB Clock)	90

5.3 Reset	90

5.3.1 Primary Reset: aresetn	90

5.3.2 Secondary Reset: presetn	91

5.4 Reset Sequencing	91

5.4.1 Power-On Reset	91

5.4.2 Functional Reset	92

5.4.3 Reset Recovery	92

5.5 Clock Requirements by Module	92

5.5.1 Functional Unit Blocks (FUB)	92

5.5.2 Integration Blocks (MAC)	93

5.6 Timing Constraints	93

5.6.1 Setup and Hold Times	93

5.6.2 Critical Paths	93

5.7 Clock Domain Crossing (CDC)	94

5.7.1 APB Configuration CDC	94

5.7.2 No CDC Required	95

5.8 Reset State Initialization	95

5.8.1 Register Reset Values	95

5.8.2 SRAM Reset	96

5.9 Clock Gating (Optional)	96

5.9.1 Per-Channel Clock Gating	96

5.10 Verification Requirements	97

5.10.1 Clock Checks	97

5.10.2 Reset Checks	97

5.10.3 CDC Checks	97

5.11 Example Reset Testbench	97

5.12 Related Documentation	98

Top-Level: mac_04_stream_top.md - Clock/reset integration	98

5.13 Revision History	98

6 STREAM Core Specification	99

6.1 Overview	99

6.1.1 Key Features	99

6.1.2 Block Diagram	99

6.1.3 Figure 2.1.1: STREAM Core Block Diagram	100

6.2 Architecture	102

6.2.1 Component Hierarchy	102

6.2.2 Data Flow	102

6.3 Parameters	103

6.3.1 Primary Configuration	103

6.3.2 Monitor Control	104

6.3.3 Outstanding Transaction Limits	104

6.3.4 AXI Skid Buffer Depths	104

6.3.5 MonBus Agent IDs	105

6.4 Port List	105

6.4.1 Clock and Reset	105

6.4.2 APB Programming Interface	105

6.4.3 Configuration Interface	106

6.4.4 Status Interface	109

6.4.5 AXI4 Master - Descriptor Fetch (256-bit)	110

6.4.6 AXI4 Master - Data Read (Parameterizable Width)	111

6.4.7 AXI4 Master - Data Write (Parameterizable Width)	113

6.4.8 Status/Debug Outputs	114

6.4.9 Unified Monitor Bus Interface	116

6.5 Operation	116

6.5.1 Transfer Initialization	116

6.5.2 Monitoring Transfer Progress	117

6.6 Timing Diagrams	117

6.6.1 Complete Data Flow Timing	117

6.7 Testing	120

6.8 Resource Utilization	120

6.9 Integration Example	121

6.10 Related Documentation	122

MonBus AXI-Lite Group: 16_monbus_axil_group.md - Monitor bus arbitration	123

6.11 Revision History	123

7 Scheduler Group Array	123

7.1 Overview	123

7.1.1 Key Features	123

7.1.2 Simplified from RAPIDS	124

7.2 Architecture	124

7.2.1 Component Hierarchy	124

7.2.2 Descriptor AXI Arbitration	124

7.2.3 Data Path Interfaces	125

7.3 Parameters	125

7.3.1 Monitor Bus Agent IDs	125

7.4 Port List	126

7.4.1 Clock and Reset	126

7.4.2 APB Programming Interface (Per-Channel)	126

7.4.3 Configuration Interface (Per-Channel)	126

7.4.4 Global Scheduler Configuration	127

7.4.5 Descriptor Engine Configuration	127

7.4.6 Status Interface (Per-Channel)	128

7.4.7 Shared Descriptor AXI4 Master Read Interface	128

7.4.8 Shared Data Read Interface (Per-Channel Arrays)	129

7.4.9 Shared Data Write Interface (Per-Channel Arrays)	130

7.4.10 Unified Monitor Bus Interface	130

7.5 Operation	131

7.5.1 Descriptor Fetch Arbitration	131

7.5.2 Data Path Flow	131

7.5.3 MonBus Aggregation	131

7.6 Integration Example	131

7.7 Related Documentation	132

MonBus Arbiter: 16_monbus_axil_group.md - Monitor bus aggregation	133

7.8 Revision History	133

8 Scheduler Group	133

8.1 Overview	134

8.1.1 Key Features	134

8.1.2 Simplified from RAPIDS	134

8.2 Architecture	134

8.2.1 Component Hierarchy	134

8.2.2 Data Flow	134

8.3 Parameters	135

8.3.1 Monitor Bus Agent IDs	135

8.4 Port List	136

8.4.1 Clock and Reset	136

8.4.2 APB Programming Interface	136

8.4.3 Configuration Interface	136

8.4.4 Scheduler Configuration	137

8.4.5 Descriptor Engine Configuration	137

8.4.6 Status Interface	138

8.4.7 Descriptor AXI Interface (to external arbiter)	138

8.4.8 Data Read Interface (to shared read engine)	139

8.4.9 Data Write Interface (to shared write engine)	139

8.4.10 Monitor Bus Interface	140

8.5 Operation	140

8.5.1 Transfer Sequence	140

8.5.2 MonBus Sources	141

8.6 Integration Example	141

8.7 Related Documentation	142

AXI Write Engine: 12_axi_write_engine.md - Shared write engine	142

8.8 Revision History	142

9 Scheduler Specification	143

9.1 Overview	143

9.1.1 Key Features	143

9.1.2 Block Diagram	144

9.1.3 Figure 2.4.1: Scheduler Block Diagram	144

9.2 CRITICAL: Concurrent Read/Write Design	144

9.3 Parameters	145

9.4 Port List	145

9.4.1 Clock and Reset	145

9.4.2 Configuration Interface	146

9.4.3 Status Interface	146

9.4.4 Descriptor Engine Interface	146

9.4.5 Data Read Interface	147

9.4.6 Data Write Interface	147

9.4.7 Error Signals	148

9.4.8 Monitor Bus Interface	148

9.5 Interface	149

9.5.1 Clock and Reset	149

9.5.2 Configuration Interface	149

9.5.3 Status Interface	149

9.5.4 Descriptor Engine Interface	149

9.5.5 Data Read Interface (to AXI Read Engine)	149

9.5.6 Data Write Interface (to AXI Write Engine)	150

9.5.7 Monitor Bus Interface	151

9.6 Descriptor Format	151

9.6.1 STREAM Descriptor (256-bit)	151

9.7 FSM Operation	152

9.7.1 State Machine	152

9.7.2 Figure 2.4.2: Scheduler FSM	153

9.7.3 State Transitions	153

9.8 Beat Tracking	154

9.8.1 Independent Counters	154

9.8.2 Completion Detection	154

9.8.3 Multiple Requests per Descriptor	155

9.9 Address Management	155

9.9.1 Static Base Address	155

9.10 Timeout Detection	156

9.10.1 Timeout Counter	156

9.11 Error Handling	156

9.11.1 Error Sources	156

9.11.2 Sticky Error Flags	157

9.11.3 Error Recovery	157

9.12 Interrupt Generation	158

9.12.1 IRQ via MonBus	158

9.13 Descriptor Chaining	158

9.13.1 Chain Detection	158

9.13.2 Chain Termination	158

9.14 MonBus Integration	159

9.14.1 Event Types	159

9.14.2 MonBus Packet Format	159

9.15 Timing Diagrams	160

9.15.1 Normal Transfer (No Chaining)	160

9.15.2 Figure 2.4.3: Scheduler Normal Transfer Timing	160

9.15.3 Descriptor Chaining	161

9.15.4 Figure 2.4.4: Scheduler Descriptor Chaining Timing	161

9.15.5 Single-Descriptor Transfer (Detailed)	161

9.15.6 Chained Descriptors (Detailed)	162

9.16 Testing	162

9.17 Performance Considerations	163

9.17.1 Concurrent Operation Benefit	163

9.17.2 Example Performance	163

9.18 Related Documentation	163

9.19 Revision History	164

10 Descriptor Engine	165

10.1 Overview	165

10.1.1 Key Features	165

10.2 Architecture	166

10.2.1 Operating Flow	166

10.2.2 FSM States	166

10.2.3 Descriptor Format (256-bit STREAM)	166

10.3 Parameters	167

10.3.1 Monitor Bus Parameters	167

10.4 Port List	168

10.4.1 Clock and Reset	168

10.4.2 APB Programming Interface	168

10.4.3 Scheduler Interface	168

10.4.4 Enhanced Control Outputs	169

10.4.5 AXI4 AR Channel	169

10.4.6 AXI4 R Channel (FIXED 256-bit)	170

10.4.7 Configuration Interface	170

10.4.8 Status Interface	171

10.4.9 Monitor Bus Interface	171

10.5 Operation	171

10.5.1 APB Acceptance Policy	171

10.5.2 Autonomous Chaining Decision Tree	171

10.5.3 AXI Transaction	172

10.6 Timing Diagrams	172

10.6.1 APB Kick-off Sequence	172

10.7 Integration Example	174

10.8 Common Issues	175

10.8.1 Issue 1: APB Write Not Accepted	175

10.8.2 Issue 2: Descriptor Chain Stops Early	175

10.9 Related Documentation	175

APB Interface: 13_apbtodescr.md - APB to descriptor kick-off	175

10.10 Revision History	175

11 AXI Read Engine	176

11.1 Overview	176

11.1.1 Key Features	176

11.2 Architecture	177

11.2.1 Operation Flow	177

11.2.2 Key Design Decisions	177

11.2.3 System Idle Behavior	177

11.3 Parameters	177

11.3.1 Derived Parameters	178

11.4 Port List	178

11.4.1 Clock and Reset	178

11.4.2 Configuration Interface	178

11.4.3 Scheduler Interface (Per-Channel)	179

11.4.4 Completion Interface (Per-Channel)	179

11.4.5 SRAM Allocation Interface	179

11.4.6 SRAM Write Interface	180

11.4.7 AXI4 AR Channel	180

11.4.8 AXI4 R Channel	180

11.4.9 Error Interface	181

11.4.10 Debug Interface	181

11.5 Operation	181

11.5.1 Space-Aware Request Masking	181

11.5.2 Transfer Size Calculation	182

11.5.3 Outstanding Transaction Tracking	182

11.5.4 Completion Strobe	182

11.6 Timing Diagrams	182

11.6.1 Perfect Streaming - AXI Read Transaction	182

11.6.2 Multi-Channel Streaming	184

11.7 Integration Example	184

11.8 Related Documentation	186

Write Engine: 12_axi_write_engine.md - Complementary write datapath	186

11.9 Revision History	186

12 Stream Allocation Controller	187

12.1 Overview	187

12.1.1 What Makes This a “Virtual FIFO”	187

12.2 The Allocation Problem	187

12.2.1 Without Allocation Controller	187

12.2.2 With Allocation Controller	187

12.3 Architecture	189

12.3.1 Two-Pointer System	189

12.3.2 Figure 2.7.1: Stream Allocation Controller Block Diagram	189

12.3.3 CRITICAL: Confusing Naming Convention	191

12.4 Parameters	191

12.5 Port List	192

12.5.1 Clock and Reset	192

12.5.2 Write Interface (Allocation Requests)	192

12.5.3 Read Interface (Actual Data Written)	193

12.5.4 Status Outputs	193

12.6 Interfaces	193

12.6.1 Write Interface (Allocation Requests)	193

12.6.2 Read Interface (Actual Data Written)	194

12.6.3 Status Outputs	194

12.7 Operation	195

12.7.1 Allocation Flow	195

12.7.2 Pointer Arithmetic	195

12.8 Timing Behavior	196

12.8.1 Allocation Latency	196

12.8.2 Release Latency	196

12.9 Integration Example	197

12.9.1 In sram_controller_unit.sv	197

12.10 Debug Support	197

12.10.1 Display Statements	197

12.10.2 Waveform Analysis	198

12.11 Common Issues	198

12.11.1 Issue 1: Space Not Released	198

12.11.2 Issue 2: Overflow Despite Allocation	198

12.11.3 Issue 3: Allocation Pointer Overflow	199

12.12 Comparison with Drain Controller	199

12.13 Resource Utilization	199

12.14 Related Modules	200

12.15 Related Documentation	200

AXI Read Engine: 06_axi_read_engine.md	200

12.16 Revision History	200

13 SRAM Controller Specification	200

13.1 Overview	201

13.1.1 Key Features	201

13.1.2 Design Rationale	201

13.2 Architecture	202

13.2.1 Block Diagram	202

13.2.2 Figure 2.8.1: SRAM Controller Block Diagram	202

13.2.3 Per-Channel Architecture	202

13.3 Parameters	202

13.4 Port List	203

13.4.1 Clock and Reset	203

13.4.2 Allocation Interface	203

13.4.3 Write Interface	204

13.4.4 Drain Interface	204

13.4.5 Read Interface	204

13.4.6 Debug Interface	205

13.5 Interface	205

13.5.1 Clock and Reset	205

13.5.2 Allocation Interface (AXI Read Engine Flow Control)	205

13.5.3 Write Interface (AXI Read Engine → FIFO)	206

13.5.4 Drain Interface (AXI Write Engine Flow Control)	206

13.5.5 Read Interface (FIFO → AXI Write Engine)	207

13.5.6 Debug Interface	207

13.6 ID Decode Logic	208

13.6.1 Write Valid Decode	208

13.6.2 Read/Drain Decode	208

13.6.3 Allocation Decode	209

13.7 Per-Channel Unit	209

13.8 Operation Flows	210

13.8.1 Write Flow (AXI Read Data → FIFO)	210

13.8.2 Read Flow (FIFO → AXI Write Data)	211

13.9 Allocation vs. Drain Controllers	211

13.9.1 Why Separate Controllers?	211

13.9.2 Flow Control Comparison	212

13.10 Timing Diagrams	212

13.10.1 Combined Write and Read Timing	212

13.10.2 Write Path (R Data → FIFO)	213

13.10.3 Read Path (FIFO → W Data)	214

13.11 Error Conditions	214

13.11.1 Invalid Channel ID	214

13.11.2 FIFO Overflow	214

13.11.3 FIFO Underflow	215

13.12 Performance Considerations	215

13.12.1 Per-Channel FIFO Benefits	215

13.12.2 SRAM Resource Usage	215

13.13 Testing	215

13.14 Related Documentation	216

13.15 Revision History	216

14 SRAM Controller Unit	217

14.1 Overview	217

14.1.1 Key Features	217

14.2 Architecture	218

14.2.1 Block Diagram	218

14.2.2 Figure 1: SRAM Controller Unit Block Diagram	218

14.2.3 Component Hierarchy	218

14.2.4 Data Flow	218

14.2.5 Controller Naming Convention (CRITICAL)	218

14.3 Parameters	219

14.3.1 Derived Parameters	219

14.4 Port List	219

14.4.1 Clock and Reset	219

14.4.2 Allocation Interface (Read Engine Flow Control)	219

14.4.3 Write Interface (AXI Read Engine to FIFO)	220

14.4.4 Drain Interface (Write Engine Flow Control)	220

14.4.5 Read Interface (FIFO to AXI Write Engine)	220

14.4.6 Debug Interface	221

14.5 Operation	221

14.5.1 Allocation Flow	221

14.5.2 Drain Flow	221

14.5.3 Data Available Calculation	221

14.6 Integration Example	222

14.7 Common Issues	222

14.7.1 Issue 1: Space Accounting Mismatch	222

14.7.2 Issue 2: Data Available Undercount	223

14.8 Related Documentation	223

Write Engine: 12_axi_write_engine.md - Data consumer	223

14.9 Revision History	223

15 Stream Latency Bridge	224

15.1 Overview	224

15.1.1 Key Features	224

15.2 Architecture	225

15.2.1 Block Diagram	225

15.2.2 Figure 2.10.1: Stream Latency Bridge Block Diagram	225

15.2.3 Operation Flow	225

15.2.4 Component Diagram	225

15.2.5 Backpressure Logic	226

15.3 Parameters	227

15.3.1 Derived Parameters	227

15.4 Port List	227

15.4.1 Clock and Reset	227

15.4.2 Upstream Interface (from registered FIFO)	227

15.4.3 Downstream Interface (to consumer)	228

15.4.4 Status Interface	228

15.4.5 Debug Interface	228

15.5 Operation	228

15.5.1 Glue Logic	228

15.5.2 Skid Buffer	229

15.5.3 Occupancy Calculation	229

15.6 Timing Diagrams	229

15.6.1 Backpressure Handling	229

15.7 Integration Example	230

15.8 Design Rationale	231

15.8.1 Why Use a Skid Buffer?	231

15.8.2 Why 4-Deep?	231

15.9 Related Documentation	231

Producer: 06_axi_read_engine.md - Feeds upstream FIFO	232

15.10 Revision History	232

16 Stream Drain Controller	232

16.1 Overview	232

16.1.1 Key Features	232

16.2 Architecture	233

16.2.1 Block Diagram	233

16.2.2 Figure 1: Stream Drain Controller Block Diagram	233

16.2.3 Virtual FIFO Concept	233

16.2.4 Pointer Operations	233

16.3 Parameters	234

16.3.1 Derived Parameters	234

16.4 Port List	234

16.4.1 Clock and Reset	234

16.4.2 Write Interface (Data Entry)	234

16.4.3 Read Interface (Drain Requests)	235

16.4.4 Status Outputs	235

16.5 Operation	235

16.5.1 Data Entry (Write Side)	235

16.5.2 Drain Requests (Read Side)	236

16.5.3 Status Generation	236

16.6 Comparison with stream_alloc_ctrl	237

16.7 Integration Example	237

16.8 Common Issues	238

16.8.1 Issue 1: Data Available Undercount	238

16.8.2 Issue 2: Drain Request Overflow	238

16.9 Related Documentation	238

Control Block: See fifo_control in rtl/amba/gaxi/	238

16.10 Revision History	238

17 AXI Write Engine	239

17.1 Overview	239

17.1.1 Key Features	239

17.2 Architecture	240

17.2.1 Block Diagram	240

17.2.2 Figure 2.12.1: AXI Write Engine Block Diagram	240

17.2.3 Operation Flow	240

17.2.4 Key Design Decisions	240

17.3 Parameters	241

17.3.1 Derived Parameters	241

17.4 Port List	242

17.4.1 Clock and Reset	242

17.4.2 Configuration Interface	242

17.4.3 Scheduler Interface (Per-Channel)	242

17.4.4 Completion Interface (Per-Channel)	242

17.4.5 SRAM Drain Interface	243

17.4.6 SRAM Read Interface	243

17.4.7 AXI4 AW Channel	243

17.4.8 AXI4 W Channel	244

17.4.9 AXI4 B Channel	244

17.4.10 Error Interface	244

17.4.11 Debug Interface	245

17.5 Operation	245

17.5.1 Data-Aware Request Masking	245

17.5.2 W-Phase Transaction FIFO	245

17.5.3 B-Phase Transaction FIFOs	246

17.5.4 Outstanding Transaction Tracking	246

17.6 Timing Diagrams	246

17.6.1 Perfect Streaming - AXI Write Transaction	246

17.6.2 Multi-Channel Streaming	248

17.7 Integration Example	249

17.8 Related Documentation	250

Read Engine: 06_axi_read_engine.md - Complementary read datapath	250

17.9 Revision History	250

18 APB to Descriptor Router	251

18.1 Overview	251

18.1.1 Key Features	251

18.2 Architecture	252

18.2.1 Address Map	252

18.2.2 Block Diagram	252

18.2.3 Figure 2.13.1: APB to Descriptor Block Diagram	252

18.2.4 FSM Diagram	253

18.2.5 Figure 2.13.2: APB to Descriptor FSM	253

18.2.6 Write Flow	253

18.2.7 FSM States	254

18.3 Parameters	254

18.4 Port List	254

18.4.1 Clock and Reset	254

18.4.2 APB Slave CMD Interface	255

18.4.3 APB Slave RSP Interface	255

18.4.4 Descriptor Engine APB Ports	255

18.4.5 Integration Control	256

18.5 Operation	256

18.5.1 Address Decode	256

18.5.2 64-bit Address Assembly	256

18.5.3 Error Conditions	256

18.6 Timing Diagrams	257

18.6.1 Normal Write Sequence	257

18.6.2 Backpressure Handling	257

18.6.3 Channel Kick-off Examples	259

18.6.4 Multi-Channel Operation	260

18.7 Integration Example	261

18.8 Common Issues	262

18.8.1 Issue 1: APB Response Not Returning	262

18.8.2 Issue 2: Wrong Channel Kicked Off	262

18.9 Related Documentation	262

Register File: PeakRDL-generated stream_regs.sv	262

18.10 Revision History	262

19 APB Configuration Block	263

19.1 Overview	263

19.1.1 Key Features	263

19.2 Architecture	264

19.2.1 Block Diagram	264

19.2.2 Figure 1: Stream Config Block Diagram	264

19.2.3 Configuration Groups	264

19.3 Parameters	265

19.4 Port List	265

19.4.1 Clock and Reset	265

19.4.2 PeakRDL Register Inputs	266

19.4.3 Configuration Outputs	267

19.5 Operation	269

19.5.1 Global Enable Gating	269

19.5.2 Global Reset OR	269

19.5.3 Address Zero Extension	269

19.6 Integration Example	269

19.7 Related Documentation	270

Monitors: Monitor documentation for each AXI monitor	270

19.8 Revision History	270

20 Performance Profiler	271

20.1 Overview	271

20.1.1 Key Features	271

20.2 Architecture	272

20.2.1 Block Diagram	272

20.2.2 Figure 2.15.1: Performance Profiler Block Diagram	272

20.2.3 Profiling Modes	272

20.2.4 FIFO Entry Format (36-bit)	273

20.3 Parameters	273

20.4 Port List	274

20.4.1 Clock and Reset	274

20.4.2 Channel Monitoring	274

20.4.3 Configuration Interface	274

20.4.4 FIFO Read Interface	274

20.5 Operation	275

20.5.1 Edge Detection	275

20.5.2 Timestamp Counter	275

20.5.3 Start Time Capture (Elapsed Mode)	275

20.5.4 Two-Register Read Sequence	276

20.6 Integration Example	276

20.7 Software Usage	277

20.7.1 Example: Measure Channel 0 Transfer Time	277

20.8 Common Issues	277

20.8.1 Issue 1: FIFO Overflow	277

20.8.2 Issue 2: Timestamp Rollover	278

20.9 Related Documentation	278

Configuration: 14_apb_config.md - Config register mapping	278

20.10 Revision History	278

21 MonBus AXI-Lite Group	279

21.1 Overview	279

21.1.1 Key Features	279

21.1.2 Simplified from RAPIDS	279

21.2 Architecture	280

21.2.1 Block Diagram	280

21.2.2 Figure 2.16.1: MonBus AXI-Lite Group Block Diagram	280

21.2.3 Filter Decision Tree	281

21.3 Parameters	281

21.4 Port List	281

21.4.1 Clock and Reset	281

21.4.2 Monitor Bus Input	282

21.4.3 AXI-Lite Slave Read Interface	282

21.4.4 AXI-Lite Master Write Interface	282

21.4.5 Interrupt Output	283

21.4.6 Configuration Interface	283

21.4.7 Protocol Configuration (per protocol)	283

21.4.8 Debug/Status	284

21.5 Operation	285

21.5.1 Packet Type Filtering	285

21.5.2 Master Write Address Generation	285

21.5.3 Master Write FSM	285

21.6 Integration Example	286

21.7 Common Issues	287

21.7.1 Issue 1: Missing Monitor Packets	287

21.7.2 Issue 2: Master Writes Stall	287

21.8 Related Documentation	287

Configuration: 14_apb_config.md - Register mapping	287

21.9 Revision History	287

22 Chapter 3: External Interfaces	288

22.1 Interface Specifications	288

22.1.1 01_axi4_interface_spec.md	288

22.1.2 02_axil4_interface_spec.md	288

22.1.3 03_apb_interface_spec.md	288

22.1.4 05_monbus_interface_spec.md	288

22.2 STREAM Interface Summary (stream_top_ch8.sv)	289

23 AXI4 Interface Specification for STREAM	290

23.1 Overview	290

23.2 STREAM AXI4 Interface Summary	290

23.2.1 Number of Interfaces	290

23.2.2 Interface Parameters	291

23.2.3 Interface Types and Transfer Modes	291

23.2.4 Interface Group Parameter Settings	292

23.2.5 Interface Configuration Summary	293

23.3 Transfer Mode Specifications	294

23.3.1 Mode 1: Simplified Transfer Mode (Control Interfaces)	294

23.3.2 Mode 2: Flexible Transfer Mode (Data Interfaces)	294

23.4 Mode 1: Simplified Transfer Mode Specification	295

23.4.1 Assumption 1: Address Alignment to Data Bus Width	295

23.4.2 Assumption 2: Fixed Transfer Size	295

23.5 Mode 2: Flexible Transfer Mode Specification	296

23.5.1 Assumption 1: 4-Byte Address Alignment	296

23.5.2 Assumption 2: Multiple Transfer Sizes	296

23.5.3 Assumption 3: Progressive Alignment Strategy	296

23.5.4 Assumption 4: Chunk Enable Support	297

23.6 Common Protocol Assumptions (Both Modes)	297

23.6.1 Assumption 1: Incrementing Bursts Only	297

23.6.2 Assumption 2: No Address Wraparound	298

23.7 Flexible Mode: Address Calculation Examples	298

23.7.1 Progressive Alignment Examples	298

23.7.2 Chunk Enable Pattern Examples	299

23.8 Master Read Interface Specification	300

23.8.1 Read Address Channel (AR)	300

23.8.2 Read Data Channel (R)	301

23.9 Master Write Interface Specification	301

23.9.1 Write Address Channel (AW)	301

23.9.2 Write Data Channel (W)	302

23.9.3 Write Response Channel (B)	303

23.10 Address Calculation Rules	303

23.10.1 Simplified Mode Address Generation	303

23.10.2 Flexible Mode Address Generation	303

23.10.3 4KB Boundary Considerations (Both Modes)	304

23.11 Write Strobe Generation	304

23.11.1 Simplified Mode Strobe Generation	304

23.11.2 Flexible Mode Strobe Generation	304

23.12 Response Codes	305

23.12.1 Response Code Specification	305

23.13 Implementation Benefits	306

23.13.1 Simplified Mode Benefits	306

23.13.2 Flexible Mode Benefits	306

23.13.3 Mode Selection Guidelines	306

23.14 Validation Requirements	307

23.14.1 Simplified Mode Validation	307

23.14.2 Flexible Mode Validation	307

23.14.3 Common Validation	308

23.15 Performance Characteristics	308

23.15.1 Simplified Mode Performance	308

23.15.2 Flexible Mode Performance	309

23.15.3 Performance Optimization Strategy	309

23.16 This dual-mode approach provides the best of both worlds: simplified, predictable operation for control interfaces and flexible, high-performance operation for data interfaces.	309

23.17 Revision History	309

24 AXI4-Lite Interface Specification and Assumptions	310

24.1 Overview	310

24.2 Interface Summary	310

24.2.1 Number of Interfaces	310

24.2.2 Interface Parameters	310

24.3 Core Protocol Assumptions	310

24.3.1 Inherent AXI4-Lite Simplifications	310

24.3.2 Implementation Assumptions	311

24.4 Master Read Interface Specification	312

24.4.1 Read Address Channel (AR)	312

24.4.2 Read Data Channel (R)	313

24.4.3 AXI4-Lite Simplifications (Read)	313

24.5 Master Write Interface Specification	314

24.5.1 Write Address Channel (AW)	314

24.5.2 Write Data Channel (W)	314

24.5.3 Write Response Channel (B)	314

24.5.4 AXI4-Lite Simplifications (Write)	315

24.6 Address Requirements	315

24.6.1 Address Alignment Rules	315

24.6.2 Address Validation Examples	316

24.7 Response Codes	316

24.7.1 Response Code Specification	316

24.7.2 Response Usage Guidelines	316

24.8 Protection Signal Usage	317

24.8.1 Protection Signal Encoding	317

24.8.2 Common Protection Patterns	317

24.9 Implementation Benefits	317

24.9.1 Simplified Control Register Interface	317

24.9.2 Address Decode Implementation	318

24.9.3 Error Generation Logic	318

24.10 Timing Requirements	319

24.10.1 Handshake Protocol	319

24.10.2 Channel Dependencies	319

24.10.3 Reset Behavior	319

24.11 Validation Requirements	320

24.11.1 Functional Validation	320

24.11.2 Timing Validation	320

24.11.3 Error Injection Testing	321

24.12 Example Transactions	321

24.12.1 64-bit Register Write	321

24.12.2 AW Transaction Flow	321

24.12.3 64-bit Register Read	322

24.12.4 AR Transaction Flow	322

24.12.5 Misaligned Address Example	322

24.13 Common Use Cases	323

24.13.1 Typical Applications	323

24.13.2 Performance Considerations	323

24.14 Revision History	324

25 APB Programming Interface Specification for STREAM	324

25.1 Overview	324

25.2 Interface Summary	324

25.2.1 STREAM Programming Interface	324

25.2.2 Interface Parameters	325

25.3 Core Protocol Assumptions	325

25.3.1 Inherent APB Simplifications	325

25.3.2 Implementation Assumptions	326

25.4 Interface Signal Specification	327

25.4.1 Clock and Reset Signals	327

25.4.2 Data Signals	328

25.4.3 Response Signals	328

25.5 Address Space Configuration	328

25.5.1 Address Range Specification	328

25.5.2 Address Decode Implementation	328

25.6 Transaction Protocol	329

25.6.1 2-Phase Transaction States	329

25.6.2 State Transitions	329

25.6.3 Transaction Timing Requirements	330

25.7 Advanced Features	330

25.7.1 Dynamic Clock Gating	330

25.7.2 Clock Domain Crossing (CDC) Handshaking	330

25.7.3 Buffering and Flow Control	331

25.8 Error Handling	331

25.8.1 Error Response Specification	331

25.8.2 Error Response Timing	332

25.9 Reset Behavior	332

25.9.1 Reset Requirements	332

25.10 Implementation Variants	332

25.10.1 Available Implementations	332

25.11 Performance Characteristics	333

25.11.1 Latency Specifications	333

25.11.2 Throughput Specifications	333

25.11.3 Power Consumption	333

25.12 Validation Requirements	334

25.12.1 Functional Validation	334

25.12.2 Timing Validation	334

25.13 Example Transactions	335

25.13.1 32-bit Register Write	335

25.13.2 32-bit Register Read with Wait State	335

25.14 Revision History	335

26 Monitor Bus Architecture and Event Code Organization	336

26.1 STREAM-Specific Context	336

26.2 Overview	336

26.3 Interface Summary	336

26.3.1 Number of Interfaces	336

26.3.2 Interface Parameters	337

26.4 Core Design Assumptions	337

26.4.1 Assumption 1: Hierarchical Event Organization	337

26.4.2 Assumption 2: Protocol Isolation	338

26.4.3 Assumption 3: Two-Tier Memory Architecture	338

26.4.4 Assumption 4: Configurable Packet Routing	338

26.5 Interface Signal Specification	339

26.5.1 Monitor Bus Output Interface	339

26.5.2 Protocol Input Interfaces	339

26.5.3 Control and Status Signals	340

26.6 Packet Format and Field Allocation	341

26.6.1 64-bit Monitor Bus Packet Structure	341

26.6.2 Packet Type Definitions	341

26.7 Protocol-Specific Event Codes	342

26.7.1 AXI Protocol Events	342

26.7.2 APB Protocol Events	345

26.7.3 MNOC Protocol Events	348

26.7.4 ARB Protocol Events	351

26.7.5 CORE Protocol Events	355

26.8 Memory Architecture and Packet Routing	360

26.8.1 Two-Tier Memory Architecture	360

26.8.2 Routing Configuration	361

26.8.3 Address Space Management	362

26.8.4 Transaction State and Bus Transaction Structure	362

26.8.5 APB Transaction Phases	365

26.8.6 APB Protection Types	365

26.8.7 MNOC Payload Types	365

26.8.8 MNOC ACK Types	366

26.8.9 ARB State Types	366

26.8.10 CORE State Types	366

26.9 Configuration and Control	367

26.9.1 Monitor Configuration Registers	367

26.9.2 Protocol-Specific Configuration	368

26.10 Validation Requirements	370

26.10.1 Functional Validation	370

26.10.2 Performance Validation	371

26.10.3 Error Handling Validation	371

26.11 Usage Examples	372

26.11.1 Creating Monitor Packets	372

26.11.2 Packet Decoding	372

26.11.3 Monitor Bus Packet Helper Functions	372

26.12 Debug and Monitoring Signals	376

26.12.1 Essential Debug Signals	376

26.12.2 Performance Counters	376

26.13 Protocol Coverage Summary	376

26.13.1 Complete Protocol Event Matrix	376

26.14 Total Event Codes: 544 defined across all protocols and packet types.	377

26.15 Revision History	377

27 STREAM Register Map	378

27.1 Overview	378

27.2 Address Space Layout	378

27.3 Register Details	379

27.3.1 Channel Kick-off Registers (0x000 - 0x03F)	379

27.3.2 Global Control and Status (0x100 - 0x11F)	380

27.3.3 Per-Channel Control and Status (0x120 - 0x17F)	382

27.3.4 Engine Completion and Error Status (0x170 - 0x17F)	384

27.3.5 Monitor FIFO Status (0x180 - 0x1FF)	385

27.3.6 Scheduler Configuration (0x200 - 0x21F)	386

27.3.7 Descriptor Engine Configuration (0x220 - 0x23F)	386

27.3.8 Descriptor AXI Monitor Configuration (0x240 - 0x25F)	388

27.3.9 Read Engine AXI Monitor Configuration (0x260 - 0x27F)	390

27.3.10 Write Engine AXI Monitor Configuration (0x280 - 0x29F)	392

27.3.11 AXI Transfer Configuration (0x2A0 - 0x2AF)	395

27.3.12 Performance Profiler Configuration (0x2B0 - 0x2BF)	395

27.4 Typical Usage Flow	396

27.4.1 Initialization	396

27.4.2 Start Transfer	397

27.4.3 Poll for Completion	397

27.4.4 Error Handling	397

27.5 Register Summary Table	397

27.6 PeakRDL Generation	398

28 Chapter 5: Programming Models	399

28.1 Contents	399

28.1.1 01_initialization.md	399

28.1.2 02_single_transfer.md	399

28.1.3 03_chained_transfers.md	399

28.1.4 04_multi_channel.md	399

28.1.5 05_error_handling.md	400

28.2 Planned (Future)	400

28.2.1 06_performance_tuning.md	400

28.2.2 07_software_examples.md	400

29 Chapter 6: Configuration Reference	401

29.1 Overview	401

29.1.1 Configuration Categories	401

29.2 1. Channel Control Configuration	401

29.2.1 cfg_channel_enable[NUM_CHANNELS-1:0]	401

29.2.2 cfg_channel_reset[NUM_CHANNELS-1:0]	402

29.3 2. Scheduler Configuration	403

29.3.1 cfg_sched_timeout_cycles[15:0]	403

29.3.2 cfg_sched_enable	403

29.3.3 cfg_sched_timeout_enable	403

29.3.4 cfg_sched_err_enable	403

29.3.5 cfg_sched_compl_enable	404

29.3.6 cfg_sched_perf_enable	404

29.4 3. Descriptor Engine Configuration	404

29.4.1 cfg_desceng_enable	404

29.4.2 cfg_desceng_prefetch	404

29.4.3 cfg_desceng_fifo_thresh[3:0]	405

29.4.4 cfg_desceng_addr0_base[31:0]	405

29.4.5 cfg_desceng_addr0_limit[31:0]	405

29.4.6 cfg_desceng_addr1_base[31:0]	405

29.4.7 cfg_desceng_addr1_limit[31:0]	405

29.5 4. AXI Monitor Configuration	406

29.5.1 4.1 Descriptor AXI Monitor (cfg_desc_mon_*)	406

29.5.2 4.2 Read Engine Monitor (cfg_rdeng_mon_*)	409

29.5.3 4.3 Write Engine Monitor (cfg_wreng_mon_*)	410

29.6 5. AXI Transfer Configuration	410

29.6.1 cfg_axi_rd_xfer_beats[7:0]	410

29.6.2 cfg_axi_wr_xfer_beats[7:0]	410

29.7 6. Performance Profiler Configuration	411

29.7.1 cfg_perf_enable	411

29.7.2 cfg_perf_mode	411

29.7.3 cfg_perf_clear	411

29.8 7. Configuration Presets	411

29.8.1 7.1 Minimal Configuration (Tutorial/Embedded)	411

29.8.2 7.2 Balanced Configuration (Typical FPGA)	412

29.8.3 7.3 High-Performance Configuration (ASIC/Datacenter)	413

29.8.4 7.4 Debug Configuration (Verbose Monitoring)	414

29.9 8. Configuration Best Practices	415

29.9.1 8.1 Monitor Configuration Guidelines	415

29.9.2 8.2 Timeout Configuration	416

29.9.3 8.3 Prefetch Configuration	416

29.9.4 8.4 Burst Size Selection	416

29.10 9. Configuration Register Map Summary	417

29.11 10. Software Configuration Examples	418

29.11.1 10.1 C/C++ Initialization (Minimal)	418

29.11.2 10.2 C/C++ Initialization (Balanced)	419

29.12 11. Troubleshooting Configuration Issues	420

29.12.1 Problem: No transfers occurring	420

29.12.2 Problem: Timeout errors	420

29.12.3 Problem: MonBus overflow	420

29.12.4 Problem: Low throughput	420

29.13 Related Documentation	420

30 Product Requirements Document (PRD)	421

30.1 STREAM - Scatter-gather Transfer Rapid Engine for AXI Memory	421

30.2 1. Executive Summary	421

30.2.1 1.1 Quick Stats	421

30.2.2 1.2 Project Goals	421

30.3 2. Key Design Principles	422

30.3.1 2.1 Simplifications from RAPIDS	422

30.3.2 2.2 Tutorial-Friendly Features	422

30.4 3. Architecture Overview	422

30.4.1 3.1 Top-Level Block Diagram	422

30.4.2 3.2 Data Flow	423

30.5 4. Interfaces	423

30.5.1 4.1 External Interfaces	423

30.5.2 4.2 Descriptor Format	424

30.6 5. Key Components	425

30.6.1 5.1 Descriptor Engine (APB-Only for STREAM)	425

30.6.2 5.2 Scheduler Group (Integration Wrapper)	425

30.6.3 5.3 Scheduler (Simplified from RAPIDS)	427

30.6.4 5.3 AXI Read Engine (Streaming Pipeline - NO FSM)	428

30.6.5 5.4 AXI Write Engine (Streaming Pipeline - NO FSM)	429

30.6.6 5.5 Simple SRAM	430

30.7 6. Configuration and Control	430

30.7.1 6.1 APB Register Map	430

30.7.2 6.2 Channel Configuration	431

30.8 7. Resource Sharing and Arbitration	431

30.8.1 7.1 Shared Resources	431

30.8.2 7.2 Arbitration Strategy	431

30.9 8. Error Detection and Recovery	431

30.9.1 8.1 Error Types	431

30.9.2 8.2 Error Recovery	432

30.10 9. MonBus Integration	432

30.10.1 9.1 Standard MonBus Format	432

30.10.2 9.2 STREAM Event Codes	432

30.10.3 9.3 Default Configuration	433

30.11 10. Design Constraints	433

30.11.1 10.1 Tutorial Constraints (Intentional Simplifications)	433

30.11.2 10.2 Implementation Constraints	433

30.12 11. Verification Strategy	433

30.12.1 11.1 Test Organization	433

30.12.2 11.2 Test Levels	434

30.13 12. Performance Characteristics	434

30.13.1 12.1 Throughput by Engine Version	434

30.13.2 12.2 Latency	435

30.13.3 12.3 Resource Utilization by Engine Version	435

30.14 13. Development Roadmap	435

30.14.1 13.1 Phase 1: Foundation (Current)	435

30.14.2 13.2 Phase 2: Core Blocks	436

30.14.3 13.3 Phase 3: Data Path	436

30.14.4 13.4 Phase 4: Integration	436

30.14.5 13.5 Phase 5: Multi-Channel	436

30.14.6 13.6 Phase 6: Advanced Engines (Future - V2/V3)	436

30.15 14. Educational Value	437

30.15.1 14.1 Learning Objectives	437

30.15.2 14.2 Progression Path	437

30.16 15. Success Criteria	438

30.16.1 15.1 Functional	438

30.16.2 15.2 Quality	438

30.16.3 15.3 Performance	438

30.17 16. Open Questions (For Review)	438

30.17.1 16.1 Descriptor Engine Adaptation	438

30.17.2 16.2 AXI Descriptor Master	438

30.17.3 16.3 Channel Arbitration	438

30.17.4 16.4 SRAM Partitioning	439

30.18 16. Attribution and Contribution Guidelines	439

30.18.1 16.1 Git Commit Attribution	439

30.19 16.2 PDF Generation Location	439

30.20 17. References	440

30.20.1 17.1 Internal Documentation	440

30.20.2 17.2 External References	440

30.21 Navigation	440

31 Claude Code Guide: STREAM Subsystem	440

31.1 Quick Context	441

31.2 📖 Global Requirements Reference	441

31.3 Critical Rules for This Subsystem	441

31.3.1 Rule #0: Attribution Format for Git Commits	441

31.3.2 Rule #0.1: TUTORIAL FOCUS - Intentional Simplifications	441

31.3.3 Rule #0.1: Testbench Location and Test Structure (MANDATORY)	442

31.3.4 Rule #0.2: Three Mandatory TB Methods (MANDATORY)	442

31.3.5 Rule #1: REUSE from RAPIDS Where Appropriate	443

31.3.6 Rule #2: Descriptor Format is DIFFERENT from RAPIDS	443

31.3.7 Rule #3: Know the Shared Resources	443

31.4 Architecture Quick Reference	444

31.4.1 Block Organization	444

31.4.2 Module Status	444

31.5 Common User Questions and Responses	445

31.5.1 Q: “How is STREAM different from RAPIDS?”	445

31.5.2 Q: “How do I kick off a transfer?”	445

31.5.3 Q: “How many channels can I use?”	446

31.5.4 Q: “What’s the descriptor format?”	446

31.5.5 Q: “How do I run STREAM tests?”	447

31.6 Integration Patterns	447

31.6.1 Pattern 1: Basic STREAM Instantiation	447

31.6.2 Pattern 2: Descriptor Creation (Software Model)	448

31.6.3 Pattern 3: MonBus Integration	449

31.7 Anti-Patterns to Catch	450

31.7.1 ✗ Anti-Pattern 1: Adding Alignment Fixup	450

31.7.2 ✗ Anti-Pattern 2: Using Length in Bytes	450

31.7.3 ✗ Anti-Pattern 3: Circular Buffer Descriptors	450

31.7.4 ✗ Anti-Pattern 4: Assuming Exclusive Channel Access	450

31.8 Debugging Workflow	451

31.8.1 Issue: Descriptor Not Fetched	451

31.8.2 Issue: Data Transfer Stalls	451

31.8.3 Issue: Chained Descriptors Not Following	451

31.9 Testing Guidance	451

31.9.1 Test Organization	451

31.9.2 Test Levels	452

31.10 Key Documentation Links	452

31.10.1 Always Reference These	452

31.11 Quick Commands	452

31.12 Remember	453

31.13 PDF Generation Location	453

32 STREAM Register Definitions	454

32.1 Directory Structure	454

32.2 Register Generation Workflow	454

32.2.1 Phase 1: Define Register Map (Future)	454

32.2.2 Phase 2: PeakRDL-Generated Registers (Future)	454

32.3 Register Map (Planned)	455

32.3.1 Global Registers	455

32.3.2 Channel Registers (8 × 0x10 bytes)	455

32.4 Integration Pattern	456

32.4.1 PeakRDL-Generated Implementation (Future):	456

32.5 Reference Examples	457

32.6 Status	457

List of Figures

Figure 1.1.1: STREAM Architecture Overview	65

Figure 1.1.2: Single Channel Transfer Sequence	67

Figure 2.1.1: STREAM Core Block Diagram	100

Figure 2.4.1: Scheduler Block Diagram	144

Figure 2.4.2: Scheduler FSM	153

Figure 2.4.3: Scheduler Normal Transfer Timing	160

Figure 2.4.4: Scheduler Descriptor Chaining Timing	161

Figure 2.7.1: Stream Allocation Controller Block Diagram	189

Figure 2.8.1: SRAM Controller Block Diagram	202

Figure 1: SRAM Controller Unit Block Diagram	218

Figure 2.10.1: Stream Latency Bridge Block Diagram	225

Figure 1: Stream Drain Controller Block Diagram	233

Figure 2.12.1: AXI Write Engine Block Diagram	240

Figure 2.13.1: APB to Descriptor Block Diagram	252

Figure 2.13.2: APB to Descriptor FSM	253

Figure 1: Stream Config Block Diagram	264

Figure 2.15.1: Performance Profiler Block Diagram	272

Figure 2.16.1: MonBus AXI-Lite Group Block Diagram	280

List of Tables

Table 1: Related Documents and Specifications	61

Table 2: STREAM MAS Document Revision History	63

Table 3: STREAM External Interfaces	68

Table 4: STREAM Internal MonBus Interface	69

Table 5: STREAM Resource Utilization Breakdown	69

Table 6: STREAM Architecture Overview Revision History	72

Table 7: Clock and Reset Signals	73

Table 8: APB Programming Interface Signals	74

Table 9: Per-Channel Configuration Signals	74

Table 10: Global Scheduler Configuration Signals	75

Table 11: Descriptor Engine Configuration Signals	75

Table 12: AXI Monitor Signal Prefixes	76

Table 13: AXI Monitor Configuration Signal Suffixes	76

Table 14: AXI Transfer Configuration Signals	77

Table 15: Performance Profiler Configuration Signals	77

Table 16: Per-Channel Status Signals	78

Table 17: Performance Profiler Interface Signals	79

Table 18: Descriptor AXI Master AR Channel Signals	80

Table 19: R Channel	81

Table 20: AR Channel	81

Table 21: R Channel	82

Table 22: AW Channel	83

Table 23: W Channel	84

Table 24: B Channel	84

Table 25: Descriptor AXI Monitor Status	85

Table 26: Read Engine AXI Monitor Status	85

Table 27: Write Engine AXI Monitor Status	86

Table 28: Unified Monitor Bus Interface	86

Table 29: Port Count Summary	87

Table 30: Default Parameter Values	88

Table 31: STREAM Top-Level Port List Revision History	89

Table 32: Reset Recovery	92

Table 33: Functional Unit Blocks	92

Table 34: Integration Blocks	93

Table 35: Clocks and Reset Specification Revision History	98

Table 36: Primary Configuration	103

Table 37: Monitor Control	104

Table 38: Outstanding Transaction Limits	104

Table 39: AXI Skid Buffer Depths	104

Table 40: MonBus Agent IDs	105

Table 41: Clock and Reset	105

Table 42: APB Programming Interface	105

Table 43: Configuration Interface	106

Table 44: Configuration Interface	106

Table 45: Configuration Interface	107

Table 46: Configuration Interface	107

Table 47: Status Interface	109

Table 48: Status Interface	109

Table 49: Status Interface	110

Table 50: AXI4 Master - Descriptor Fetch	110

Table 51: AXI4 Master - Descriptor Fetch	111

Table 52: AXI4 Master - Data Read	111

Table 53: AXI4 Master - Data Read	112

Table 54: AXI4 Master - Data Write	113

Table 55: AXI4 Master - Data Write	114

Table 56: AXI4 Master - Data Write	114

Table 57: Status/Debug Outputs	115

Table 58: Status/Debug Outputs	115

Table 59: Status/Debug Outputs	115

Table 60: Unified Monitor Bus Interface	116

Table 61: Resource Utilization	120

Table 62: STREAM Core Specification Revision History	123

Table 63: Parameters	125

Table 64: Monitor Bus Agent IDs	125

Table 65: Clock and Reset	126

Table 66: APB Programming Interface	126

Table 67: Configuration Interface	126

Table 68: Global Scheduler Configuration	127

Table 69: Descriptor Engine Configuration	127

Table 70: Status Interface	128

Table 71: Shared Descriptor AXI4 Master Read Interface	128

Table 72: Shared Descriptor AXI4 Master Read Interface	129

Table 73: Shared Data Read Interface	129

Table 74: Shared Data Write Interface	130

Table 75: Unified Monitor Bus Interface	130

Table 76: Scheduler Group Array Revision History	133

Table 77: Parameters	135

Table 78: Monitor Bus Agent IDs	135

Table 79: Clock and Reset	136

Table 80: APB Programming Interface	136

Table 81: Configuration Interface	136

Table 82: Scheduler Configuration	137

Table 83: Descriptor Engine Configuration	137

Table 84: Status Interface	138

Table 85: Descriptor AXI Interface	138

Table 86: Descriptor AXI Interface	139

Table 87: Data Read Interface	139

Table 88: Data Write Interface	139

Table 89: Monitor Bus Interface	140

Table 90: Scheduler Group Revision History	142

Table 91: Clock and Reset	145

Table 92: Configuration Interface	146

Table 93: Status Interface	146

Table 94: Descriptor Engine Interface	146

Table 95: Data Read Interface	147

Table 96: Data Read Interface	147

Table 97: Data Write Interface	147

Table 98: Data Write Interface	148

Table 99: Error Signals	148

Table 100: Monitor Bus Interface	148

Table 101: Revision History	164

Table 102: Descriptor Format	166

Table 103: Parameters	167

Table 104: Monitor Bus Parameters	167

Table 105: Clock and Reset	168

Table 106: APB Programming Interface	168

Table 107: Scheduler Interface	168

Table 108: Enhanced Control Outputs	169

Table 109: AXI4 AR Channel	169

Table 110: AXI4 R Channel	170

Table 111: Configuration Interface	170

Table 112: Status Interface	171

Table 113: Monitor Bus Interface	171

Table 114: Descriptor Engine Revision History	175

Table 115: Parameters	177

Table 116: Derived Parameters	178

Table 117: Clock and Reset	178

Table 118: Configuration Interface	178

Table 119: Scheduler Interface	179

Table 120: Completion Interface	179

Table 121: SRAM Allocation Interface	179

Table 122: SRAM Write Interface	180

Table 123: AXI4 AR Channel	180

Table 124: AXI4 R Channel	180

Table 125: Error Interface	181

Table 126: Debug Interface	181

Table 127: AXI Read Engine Revision History	186

Table 128: Parameters	191

Table 129: Clock and Reset	192

Table 130: Write Interface	192

Table 131: Read Interface	193

Table 132: Status Outputs	193

Table 133: Write Interface	193

Table 134: Read Interface	194

Table 135: Status Outputs	194

Table 136: Comparison with Drain Controller	199

Table 137: Stream Allocation Controller Revision History	200

Table 138: Clock and Reset	203

Table 139: Allocation Interface	203

Table 140: Write Interface	204

Table 141: Drain Interface	204

Table 142: Read Interface	205

Table 143: Debug Interface	205

Table 144: Revision History	216

Table 145: Parameters	219

Table 146: Derived Parameters	219

Table 147: Clock and Reset	219

Table 148: Allocation Interface	219

Table 149: Write Interface	220

Table 150: Drain Interface	220

Table 151: Read Interface	220

Table 152: Debug Interface	221

Table 153: SRAM Controller Unit Revision History	223

Table 154: Parameters	227

Table 155: Derived Parameters	227

Table 156: Clock and Reset	227

Table 157: Upstream Interface	227

Table 158: Downstream Interface	228

Table 159: Status Interface	228

Table 160: Debug Interface	228

Table 161: Stream Latency Bridge Revision History	232

Table 162: Parameters	234

Table 163: Derived Parameters	234

Table 164: Clock and Reset	234

Table 165: Write Interface	234

Table 166: Read Interface	235

Table 167: Status Outputs	235

Table 168: Comparison with stream_alloc_ctrl	237

Table 169: Stream Drain Controller Revision History	238

Table 170: Parameters	241

Table 171: Derived Parameters	241

Table 172: Clock and Reset	242

Table 173: Configuration Interface	242

Table 174: Scheduler Interface	242

Table 175: Completion Interface	242

Table 176: SRAM Drain Interface	243

Table 177: SRAM Read Interface	243

Table 178: AXI4 AW Channel	243

Table 179: AXI4 W Channel	244

Table 180: AXI4 B Channel	244

Table 181: Error Interface	244

Table 182: Debug Interface	245

Table 183: AXI Write Engine Revision History	250

Table 184: Parameters	254

Table 185: Clock and Reset	254

Table 186: APB Slave CMD Interface	255

Table 187: APB Slave RSP Interface	255

Table 188: Descriptor Engine APB Ports	255

Table 189: Integration Control	256

Table 190: APB to Descriptor Router Revision History	262

Table 191: Parameters	265

Table 192: Clock and Reset	265

Table 193: PeakRDL Register Inputs	266

Table 194: PeakRDL Register Inputs	266

Table 195: PeakRDL Register Inputs	266

Table 196: PeakRDL Register Inputs	267

Table 197: PeakRDL Register Inputs	267

Table 198: Configuration Outputs	267

Table 199: Configuration Outputs	267

Table 200: Configuration Outputs	268

Table 201: Configuration Outputs	268

Table 202: Configuration Outputs	268

Table 203: APB Configuration Block Revision History	270

Table 204: FIFO Entry Format	273

Table 205: Parameters	273

Table 206: Clock and Reset	274

Table 207: Channel Monitoring	274

Table 208: Configuration Interface	274

Table 209: FIFO Read Interface	274

Table 210: Performance Profiler Revision History	278

Table 211: Parameters	281

Table 212: Clock and Reset	281

Table 213: Monitor Bus Input	282

Table 214: AXI-Lite Slave Read Interface	282

Table 215: AXI-Lite Master Write Interface	282

Table 216: Interrupt Output	283

Table 217: Configuration Interface	283

Table 218: Protocol Configuration	284

Table 219: Protocol Configuration	284

Table 220: Protocol Configuration	284

Table 221: Debug/Status	284

Table 222: MonBus AXI-Lite Group Revision History	287

Table 223: Number of Interfaces	290

Table 224: Interface Parameters	291

Table 225: Interface Types and Transfer Modes	291

Table 226: Interface Group Parameter Settings	292

Table 227: Interface Configuration Summary	293

Table 228: Simplified Mode Assumptions	294

Table 229: Flexible Mode Assumptions	294

Table 230: Assumption 1: Address Alignment to Data Bus Width	295

Table 231: Assumption 2: Fixed Transfer Size	295

Table 232: Assumption 1: 4-Byte Address Alignment	296

Table 233: Assumption 2: Multiple Transfer Sizes	296

Table 234: Assumption 3: Progressive Alignment Strategy	296

Table 235: Assumption 4: Chunk Enable Support	297

Table 236: Assumption 1: Incrementing Bursts Only	297

Table 237: Assumption 2: No Address Wraparound	298

Table 238: Progressive Alignment Examples	298

Table 239: Progressive Alignment Examples	299

Table 240: Chunk Enable Pattern Examples	299

Table 241: Read Address Channel	300

Table 242: Read Data Channel	301

Table 243: Write Address Channel	301

Table 244: Write Data Channel	302

Table 245: Write Response Channel	303

Table 246: Simplified Mode Address Generation	303

Table 247: Flexible Mode Address Generation	303

Table 248: 4KB Boundary Considerations	304

Table 249: Simplified Mode Strobe Generation	304

Table 250: Flexible Mode Strobe Generation	305

Table 251: Response Code Specification	305

Table 252: Simplified Mode Benefits	306

Table 253: Flexible Mode Benefits	306

Table 254: Mode Selection Guidelines	306

Table 255: Simplified Mode Validation	307

Table 256: Flexible Mode Validation	307

Table 257: Common Validation	308

Table 258: Simplified Mode Performance	308

Table 259: Flexible Mode Performance	309

Table 260: AXI4 Interface Specification for STREAM Revision History	309

Table 261: Interface Parameters	310

Table 262: Inherent AXI4-Lite Simplifications	311

Table 263: Assumption 1: Address Alignment to Data Bus Width	311

Table 264: Assumption 2: Fixed Transfer Size	311

Table 265: Assumption 3: No Address Wraparound	312

Table 266: Assumption 4: Standard Protection Attributes	312

Table 267: Read Address Channel	312

Table 268: Read Data Channel	313

Table 269: AXI4-Lite Simplifications	313

Table 270: Write Address Channel	314

Table 271: Write Data Channel	314

Table 272: Write Response Channel	314

Table 273: AXI4-Lite Simplifications	315

Table 274: Address Alignment Rules	315

Table 275: Address Validation Examples	316

Table 276: Response Code Specification	316

Table 277: Response Usage Guidelines	316

Table 278: Protection Signal Encoding	317

Table 279: Common Protection Patterns	317

Table 280: Simplified Control Register Interface	317

Table 281: Address Decode Implementation	318

Table 282: Error Generation Logic	318

Table 283: Handshake Protocol	319

Table 284: Channel Dependencies	319

Table 285: Reset Behavior	319

Table 286: Functional Validation	320

Table 287: Timing Validation	320

Table 288: Error Injection Testing	321

Table 289: 64-bit Register Write	321

Table 290: AW Transaction Flow	321

Table 291: 64-bit Register Read	322

Table 292: AR Transaction Flow	322

Table 293: Misaligned Address Example	322

Table 294: Typical Applications	323

Table 295: Performance Considerations —	323

Table 296: AXI4-Lite Interface Specification and Assumptions Revision History	324

Table 297: STREAM Programming Interface	325

Table 298: Interface Parameters	325

Table 299: Assumption 1: Word-Aligned Access Only	326

Table 300: Assumption 2: Standard Transfer Sizes	326

Table 301: Assumption 3: Single-Cycle Default Operation	326

Table 302: Clock and Reset Signals ### Address and Control Signals	327

Table 303: Address and Control Signals	327

Table 304: Data Signals	328

Table 305: Response Signals	328

Table 306: Address Range Specification	328

Table 307: Address Decode Implementation	328

Table 308: 2-Phase Transaction States	329

Table 309: State Transitions	329

Table 310: Transaction Timing Requirements	330

Table 311: Dynamic Clock Gating	330

Table 312: Clock Domain Crossing (CDC) Handshaking	330

Table 313: Buffering and Flow Control	331

Table 314: Error Response Specification	331

Table 315: Error Response Timing	332

Table 316: Reset Requirements	332

Table 317: Available Implementations	332

Table 318: Latency Specifications	333

Table 319: Throughput Specifications	333

Table 320: Power Consumption	333

Table 321: Functional Validation	334

Table 322: Timing Validation	334

Table 323: 32-bit Register Write	335

Table 324: 32-bit Register Read with Wait State —	335

Table 325: APB Programming Interface Specification for STREAM Revision History	335

Table 326: Interface Parameters	337

Table 327: Assumption 1: Hierarchical Event Organization	337

Table 328: Assumption 2: Protocol Isolation	338

Table 329: Assumption 3: Two-Tier Memory Architecture	338

Table 330: Assumption 4: Configurable Packet Routing	338

Table 331: Monitor Bus Output Interface	339

Table 332: Protocol Input Interfaces	339

Table 333: Control and Status Signals	340

Table 334: 64-bit Monitor Bus Packet Structure	341

Table 335: Packet Type Definitions	341

Table 336: Error Events	342

Table 337: Timeout Events	343

Table 338: Performance Events	344

Table 339: Error Events	345

Table 340: Timeout Events	345

Table 341: Completion Events	346

Table 342: Threshold Events	346

Table 343: Performance Events	347

Table 344: Debug Events	347

Table 345: Error Events	348

Table 346: Credit Events	349

Table 347: Channel Events	350

Table 348: Stream Events	350

Table 349: Error Events	351

Table 350: Timeout Events	352

Table 351: Completion Events	352

Table 352: Threshold Events	353

Table 353: Performance Events	354

Table 354: Debug Events	354

Table 355: Error Events	355

Table 356: Timeout Events	356

Table 357: Completion Events	357

Table 358: Threshold Events	358

Table 359: Performance Events	358

Table 360: Debug Events	359

Table 361: Local Error/Interrupt Memory	360

Table 362: Configurable External Memory	361

Table 363: Base and Limit Registers	361

Table 364: Routing Decision Logic	361

Table 365: Memory Layout Example	362

Table 366: Transaction State Enumeration	362

Table 367: Enhanced Transaction Structure	363

Table 368: Phase Completion Flags	363

Table 369: Protocol-Specific Tracking Fields	364

Table 370: APB Transaction Phases	365

Table 371: APB Protection Types	365

Table 372: MNOC Payload Types	365

Table 373: MNOC ACK Types	366

Table 374: ARB State Types	366

Table 375: CORE State Types	366

Table 376: Global Configuration	367

Table 377: Packet Type Enable Mapping	367

Table 378: AXI Monitor Configuration	368

Table 379: MNOC Monitor Configuration	369

Table 380: ARB Monitor Configuration	369

Table 381: CORE Monitor Configuration	370

Table 382: Functional Validation	370

Table 383: Performance Validation	371

Table 384: Error Handling Validation	371

Table 385: Creating Monitor Packets	372

Table 386: Packet Decoding	372

Table 387: Packet Field Extraction	372

Table 388: Packet Creation Function	373

Table 389: Event Code Creation Functions	373

Table 390: Validation Functions	375

Table 391: String Functions for Debugging	375

Table 392: Essential Debug Signals	376

Table 393: Performance Counters	376

Table 394: Complete Protocol Event Matrix	376

Table 395: Monitor Bus Architecture and Event Code Organization Revision History	377

Table 396: Channel Kick-off Registers	379

Table 397: GLOBAL_CTRL	381

Table 398: GLOBAL_STATUS	381

Table 399: VERSION	381

Table 400: CHANNEL_ENABLE	382

Table 401: CHANNEL_RESET	382

Table 402: CHANNEL_IDLE	382

Table 403: DESC_ENGINE_IDLE	383

Table 404: SCHEDULER_IDLE	383

Table 405: CH_STATE[0..7]	383

Table 406: SCHED_ERROR	384

Table 407: AXI_RD_COMPLETE	384

Table 408: AXI_WR_COMPLETE	385

Table 409: MON_FIFO_STATUS	385

Table 410: MON_FIFO_COUNT	385

Table 411: SCHED_TIMEOUT_CYCLES	386

Table 412: SCHED_CONFIG	386

Table 413: DESCENG_CONFIG	386

Table 414: DESCENG_ADDR0_BASE	387

Table 415: DESCENG_ADDR0_LIMIT	387

Table 416: DESCENG_ADDR1_BASE	387

Table 417: DESCENG_ADDR1_LIMIT	387

Table 418: DAXMON_ENABLE	388

Table 419: DAXMON_TIMEOUT	388

Table 420: DAXMON_LATENCY_THRESH	388

Table 421: DAXMON_PKT_MASK	388

Table 422: DAXMON_ERR_CFG	389

Table 423: DAXMON_MASK1	389

Table 424: DAXMON_MASK2	389

Table 425: DAXMON_MASK3	390

Table 426: RDMON_ENABLE	390

Table 427: RDMON_TIMEOUT	390

Table 428: RDMON_LATENCY_THRESH	391

Table 429: RDMON_PKT_MASK	391

Table 430: RDMON_ERR_CFG	391

Table 431: RDMON_MASK1	391

Table 432: RDMON_MASK2	392

Table 433: RDMON_MASK3	392

Table 434: WRMON_ENABLE	392

Table 435: WRMON_TIMEOUT	393

Table 436: WRMON_LATENCY_THRESH	393

Table 437: WRMON_PKT_MASK	393

Table 438: WRMON_ERR_CFG	393

Table 439: WRMON_MASK1	394

Table 440: WRMON_MASK2	394

Table 441: WRMON_MASK3	394

Table 442: AXI_XFER_CONFIG	395

Table 443: PERF_CONFIG	395

Table 444: Register Summary Table	397

Table 445: PeakRDL Generation	398

Table 446: Configuration Categories	401

Table 447: 9. Configuration Register Map Summary	417

List of Waveforms

Waveform 2.1.1: STREAM Core Data Flow	118

Waveform 2.5.1: Descriptor Engine APB Basic Kick-off	173

Waveform 2.6.1: AXI Read Engine - Perfect Streaming	183

Waveform 2.6.2: Datapath Read - Multi-Channel	184

Waveform 2.8.1: SRAM Controller Write/Read Operation	213

Waveform 2.10.1: Stream Latency Bridge Backpressure	230

Waveform 2.12.1: AXI Write Engine - Perfect Streaming	247

Waveform 2.12.2: Datapath Write - Multi-Channel	248

Waveform 2.13.1: APB Normal Write Sequence	257

Waveform 2.13.2: APB Write with Backpressure	258

Waveform 2.13.3: Channel 0 Kick-off	259

Waveform 2.13.4: Channel 7 Kick-off	260

Waveform 2.13.5: Multi-Channel Kick-off Sequence	261

# 1	Stream Index

Generated: 2026-04-17

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

This document describes the STREAM subsystem, a Scatter-gather Transfer Rapid Engine for AXI Memory. STREAM provides descriptor-based DMA functionality with multi-channel support, enabling efficient memory-to-memory data transfers via AXI4 interfaces. The subsystem is designed as a tutorial-focused implementation with intentional simplifications for educational purposes.

## 2.1	References

### 2.1.1	Related Documents

Related Documents and Specifications

| Source | Title | Version |
| --- | --- | --- |
| RTL Design Sherpa | STREAM Product Requirements Document | 1.0 |
| RTL Design Sherpa | RAPIDS MAS (Parent Architecture) | 0.90 |
| ARM | AMBA AXI and ACE Protocol Specification | IHI0022H |
| ARM | AMBA APB Protocol Specification | IHI0024C |

## 2.2	Terminology

AXI

Advanced eXtensible Interface. ARM’s high-performance, high-frequency protocol for interconnect.

AXI4

Version 4 of the AXI protocol, supporting burst transfers up to 256 beats.

AXIL

AXI4-Lite. A simplified subset of AXI4 for low-throughput register access.

APB

Advanced Peripheral Bus. ARM’s simple, low-power bus protocol for peripheral register access.

Beat

A single data transfer within an AXI burst. For STREAM with 512-bit data width, one beat = 64 bytes.

Burst

A group of consecutive data transfers (beats) initiated by a single address phase.

Channel

One of 8 independent DMA channels in STREAM. Each channel has its own descriptor chain and SRAM buffer allocation.

Descriptor

A 256-bit data structure containing transfer parameters: source address, destination address, length, and chain pointer.

Descriptor Chain

A linked list of descriptors where each descriptor points to the next, enabling scatter-gather operations.

DMA

Direct Memory Access. Hardware-controlled data movement between memory locations without CPU intervention.

FUB

Functional Unit Block. A self-contained RTL module with defined interfaces and testbench.

MAC

Macro. An integration-level block that instantiates and connects multiple FUBs.

MonBus

Monitor Bus. A 64-bit internal bus for performance monitoring and debug event reporting.

Outstanding Transaction

A transaction that has been issued but not yet completed. V2/V3 performance modes support multiple outstanding transactions.

Scatter-Gather

A DMA technique where non-contiguous memory regions are transferred using a descriptor chain.

SRAM

Static Random Access Memory. Used for internal data buffering between read and write datapaths.

V1/V2/V3

STREAM performance modes: - V1 (Low): Single outstanding transaction, tutorial-focused - V2 (Medium): Command pipelined, best area efficiency - V3 (High): Out-of-order completion, maximum throughput

## 2.3	Revision History

Revisions follow the convention x.y, where x is the major version and y is the minor version. Minor versions indicate incremental updates; major versions indicate significant architectural changes.

STREAM MAS Document Revision History

| Rev | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial STREAM MAS structure and block documentation |
| 0.91 | 2026-01-02 | seang | Added front matter, figure/table captions, document styling |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	STREAM Architecture Overview

Component: STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory) Version: 0.90 Status: Pre-release

## 3.1	Introduction

STREAM is a high-performance, multi-channel descriptor-based DMA engine designed for memory-to-memory scatter-gather transfers. It provides an educational yet production-capable architecture demonstrating key DMA concepts while maintaining professional design practices.

### 3.1.1	Design Philosophy

- Tutorial Focus - Intentional simplifications for learning

- Production Quality - Industry-standard interfaces and verification

- Scalability - 8 independent channels with shared resource arbitration

- Modularity - Clear separation of concerns across functional blocks

## 3.2	System Architecture

### 3.2.1	High-Level Block Diagram

### 3.2.2	Figure 1.1.1: STREAM Architecture Overview

Diagram

## 3.3	Key Architectural Concepts

### 3.3.1	1. Multi-Channel Design

8 Independent Channels: - Each channel operates autonomously - Separate descriptor chains per channel - Independent FSM state per channel - Concurrent transfers across all channels

Resource Sharing: - Descriptor AXI master (shared via round-robin arbiter) - Data read AXI master (shared via priority arbiter) - Data write AXI master (shared via priority arbiter) - SRAM buffer (per-channel FIFOs, no arbitration)

### 3.3.2	2. Descriptor-Based Operation

Descriptor Format (256-bit):

[255:192] Reserved
[191:160] Next Descriptor Pointer
[159:128] Length (in beats)
[127:64]  Destination Address
[63:0]    Source Address

Descriptor Chain: - Single APB write kicks off chain - Automatic chaining via next_descriptor_ptr - Explicit termination (ptr = 0 or last flag)

### 3.3.3	3. Concurrent Read/Write

CRITICAL Design Pattern:

STREAM uses concurrent read and write operations to handle transfers larger than the SRAM buffer:

Example: 100MB transfer with 64KB SRAM buffer

Sequential (WRONG):
  Read 100MB → DEADLOCK at 64KB (SRAM full)

Concurrent (CORRECT):
  Read fills SRAM → SRAM full → Read pauses
  Write drains SRAM → SRAM space freed → Read resumes
  Both continue until 100MB complete

Implementation: - Scheduler runs read and write FSMs concurrently in XFER_DATA state - Independent beat counters for read vs write - Transfer completes when BOTH counters reach zero

### 3.3.4	4. Space-Aware Flow Control

Allocation Controller (Read Path): - Reserves SRAM space BEFORE issuing AXI AR - Prevents overflow due to AR/R latency gap - 2x space margin (accounts for in-flight allocations)

Drain Controller (Write Path): - Reserves SRAM data BEFORE issuing AXI AW - Prevents underflow due to AW/W latency gap - Includes latency bridge occupancy in count

### 3.3.5	5. Priority-Based Arbitration

Descriptor Fetch: - Round-robin arbitration (fair access) - All channels equal priority

Data Read/Write: - Priority-based arbitration - Priority from descriptor - Round-robin within same priority tier - Timeout prevention for low-priority channels

## 3.4	Data Flow Example

### 3.4.1	Single Channel Transfer

### 3.4.2	Figure 1.1.2: Single Channel Transfer Sequence

Diagram

## 3.5	Component Hierarchy

### 3.5.1	Top-Level Integration (stream_top_ch8.sv)

stream_top_ch8 (Top-Level Wrapper)
├── APB Interface Path
│   ├── apb_slave_cdc        (CDC_ENABLE=1: APB clock domain crossing)
│   └── cmdrsp_router        (Address-based routing)
│       ├── apbtodescr       (0x000-0x03F: Channel kick-off routing)
│       ├── perf_profiler    (0x040-0x0FF: Performance profiling)
│       └── peakrdl_to_cmdrsp (0x100-0x3FF: APB → CMD/RSP conversion)
│           └── stream_regs  (PeakRDL-generated register file)
├── stream_config_block      (Register → config signal mapping)
├── stream_core              (Main DMA engine - see below)
└── monbus_axil_group        (USE_AXI_MONITORS=1: MonBus → AXI-Lite)
    ├── Error FIFO (s_axil_err_* slave read)
    └── Master Writer (m_axil_mon_* master write)

### 3.5.2	Core DMA Engine (stream_core.sv)

.1.3: Core DMA Engine Hierarchy

## 3.6	Interface Summary

### 3.6.1	External Interfaces (stream_top_ch8.sv)

STREAM External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| APB | Slave | 32-bit | Configuration, control, status (with optional CDC) |
| Descriptor AXI | Master | 256-bit | Fetch descriptors from memory |
| Data Read AXI | Master | 512-bit | Read source data |
| Data Write AXI | Master | 512-bit | Write destination data |
| Error FIFO AXIL | Slave | 32-bit | Read monitor error/interrupt FIFO |
| Monitor Write AXIL | Master | 32-bit | Write monitor data to memory |
| IRQ | Output | 1-bit | Interrupt (error FIFO not empty) |

### 3.6.2	Internal MonBus Interface (stream_core.sv)

STREAM Internal MonBus Interface

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| MonBus | Output | 64-bit | Debug/trace event stream to monbus_axil_group |

### 3.6.3	Configuration Registers (APB)

APB Address Map:

0x000-0x03F: Channel kick-off registers (apbtodescr routing)
0x040-0x0FF: Performance profiler interface
0x100-0x3FF: PeakRDL configuration registers

Key Register Groups: - 0x100: Global Control (enable, reset, version) - 0x120: Per-Channel Control (enable, reset) - 0x140: Per-Channel Status (idle, state, completion) - 0x180: Monitor FIFO Status - 0x200: Scheduler Configuration - 0x220: Descriptor Engine Configuration - 0x240-0x29F: AXI Monitor Configuration (DAXMON, RDMON, WRMON) - 0x2A0: AXI Transfer Configuration - 0x2B0: Performance Profiler Configuration

See: Register Map for complete documentation.

## 3.7	Resource Utilization

### 3.7.1	Area Breakdown (Typical 512-bit Data Width)

STREAM Resource Utilization Breakdown

| Component | Quantity | Logic (LUTs) | Memory |
| --- | --- | --- | --- |
| Descriptor Engine | 8 | ~300 each | Minimal |
| Scheduler | 8 | ~400 each | Minimal |
| AXI Read Engine | 1 | ~800 | Minimal |
| AXI Write Engine | 1 | ~800 | Minimal |
| SRAM Controller | 1 | ~600 | 8 × 64KB = 512KB |
| Arbiters | 3 | ~150 each | Minimal |
| APB Config | 1 | ~400 | Register file |
| MonBus Aggregator | 1 | ~200 | Small FIFO |
| Total | - | ~10K LUTs | ~512KB RAM |

Notes: - SRAM depth configurable (typical: 512 entries × 512 bits × 8 channels = 512KB) - Logic utilization scales with NUM_CHANNELS - Memory utilization scales with SRAM_DEPTH

## 3.8	Performance Characteristics

### 3.8.1	Throughput

Theoretical Maximum (512-bit @ 250MHz): - Single channel: 16 GB/s (512 bits × 250 MHz) - 8 channels concurrent: Limited by memory bandwidth, not DMA

Practical Performance: - Dependent on: - Memory controller latency - Burst sizes (larger = better efficiency) - Transfer alignment - Channel contention

Typical Real-World: - 8-12 GB/s sustained (single channel) - 50-75% of theoretical maximum

### 3.8.2	Latency

Descriptor Fetch Latency: - AR issue to R data: ~10-50 cycles (memory dependent) - Descriptor parsing: 1 cycle - Total: ~11-51 cycles per descriptor

Transfer Initiation: - APB write to first AR: ~5-10 cycles - AR to first R data: ~10-50 cycles - Total: ~15-60 cycles from kickoff to first data

Transfer Completion: - Last W beat to B response: ~5-20 cycles - B response to IRQ: 1 cycle - Total: ~6-21 cycles from last data to interrupt

## 3.9	Design Decisions

### 3.9.1	Why These Simplifications?

- Aligned Addresses Only:

  - Eliminates complex alignment fixup logic

  - Clear data path with no byte shifting

  - Focus: Core DMA operation, not edge cases

- Length in Beats:

  - Direct mapping to AXI burst length

  - No unit conversion overhead

  - Focus: AXI protocol understanding

- No Credit Management:

  - Simpler resource arbitration

  - Transaction limits via configuration

  - Focus: Arbitration basics, not complex flow control

- No Circular Buffers:

  - Explicit chain termination

  - Clear end-of-transfer detection

  - Focus: Descriptor chaining, not circular logic

### 3.9.2	Production Enhancements (Future)

If extending STREAM for production use, consider: - [ ] Alignment fixup (byte-level granularity) - [ ] Length in bytes/chunks - [ ] Credit-based flow control - [ ] Circular buffer support - [ ] Advanced error recovery - [ ] Power management - [ ] Multiple SRAM segments

## 3.10	Testing Strategy

### 3.10.1	Verification Layers

1. Unit Tests (FUB Level): - Descriptor engine: Fetch, parse, chain - Scheduler: FSM states, concurrent read/write - AXI engines: AR/R and AW/W/B channels - SRAM controller: Allocation, drain, FIFO - Individual tests per module

2. Integration Tests (Multi-Block): - Scheduler + engines: Data flow - Descriptor + scheduler: Descriptor chaining - Full path: APB → Descriptor → Transfer → IRQ

3. System Tests (Full Core): - Single channel end-to-end - Multi-channel concurrent - Error injection and recovery - Performance validation - Long-duration stress tests

### 3.10.2	Coverage Targets

- Code Coverage: >95%

- Functional Coverage: >90%

- Corner Cases: 100% tested

- Error Paths: 100% tested

## 3.11	Related Documentation

Chapter 2 - Block Specifications: - Scheduler - Core FSM controller - Descriptor Engine - Descriptor fetch/parse - AXI Read Engine - Source data read - AXI Write Engine - Destination data write - SRAM Controller - Buffering and flow control - Scheduler Group - Per-channel wrapper - Scheduler Group Array - 8-channel array

Other Resources: - STREAM PRD - Product requirements - Test Plan - Verification strategy

Last Updated: 2025-12-01 Document Version: 0.91 —

## 3.12	Revision History

STREAM Architecture Overview Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	STREAM Top-Level Port List

Module: stream_core.sv Location: projects/components/stream/rtl/macro/ Last Updated: 2025-12-01

## 4.1	Overview

This document provides a complete reference for all top-level ports of the STREAM Core module, organized by interface type. All port names, directions, widths, and descriptions are extracted directly from the RTL implementation.

Note: This documents stream_core.sv which is instantiated inside stream_top_ch8.sv. The top-level wrapper adds: - APB slave interface (replaces apb_valid/ready/addr with standard APB protocol) - AXI-Lite interfaces for monitor bus (s_axil_err_, m_axil_mon_) - Optional APB CDC crossing (CDC_ENABLE parameter)

For top-level integration, see the Architecture Overview.

Quick Navigation: - Clock and Reset - APB Programming Interface - Configuration Interface - Status Interface - Performance Profiler Interface - AXI4 Master - Descriptor Fetch - AXI4 Master - Data Read - AXI4 Master - Data Write - Status/Debug Outputs - Unified Monitor Bus

## 4.2	Clock and Reset

Clock and Reset Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock (100-500 MHz typical) |
| rst_n | input | 1 | Active-low asynchronous reset |

Notes: - All STREAM logic operates in the clk domain - Reset is asynchronous assert, synchronous deassert - All registers use reset macros from reset_defs.svh

## 4.3	APB Programming Interface

Per-channel descriptor kick-off interface. Each channel has independent valid/ready/address signals for descriptor chain start.

APB Programming Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid[ch] | input | NUM_CHANNELS | Channel descriptor address valid |
| apb_ready[ch] | output | NUM_CHANNELS | Channel ready to accept descriptor address |
| apb_addr[ch] | input | NUM_CHANNELS × ADDR_WIDTH | Descriptor address per channel (64-bit default) |

Usage Pattern:

// Kick off channel 0 with descriptor at 0x1000_0000
apb_valid[0] = 1'b1;
apb_addr[0] = 64'h0000_0000_1000_0000;
// Wait for handshake
wait (apb_ready[0]);
apb_valid[0] = 1'b0;

Notes: - Standard valid/ready handshake protocol - Descriptor address must be aligned to descriptor size (32 bytes) - Channel starts operation immediately after handshake - Default: NUM_CHANNELS = 8, ADDR_WIDTH = 64

## 4.4	Configuration Interface

### 4.4.1	Per-Channel Configuration

Per-Channel Configuration Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable[ch] | input | NUM_CHANNELS | Enable channel (0=disabled, 1=enabled) |
| cfg_channel_reset[ch] | input | NUM_CHANNELS | Soft reset channel (FSM → IDLE state) |

Notes: - Channel must be enabled before accepting descriptors - Soft reset clears channel FSM but preserves config

### 4.4.2	Global Scheduler Configuration

Global Scheduler Configuration Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_enable | input | 1 | Global scheduler enable |
| cfg_sched_timeout_cycles | input | 16 | Timeout threshold (clock cycles) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_sched_err_enable | input | 1 | Enable error event reporting |
| cfg_sched_compl_enable | input | 1 | Enable completion event reporting |
| cfg_sched_perf_enable | input | 1 | Enable performance event reporting |

Notes: - cfg_sched_enable is master enable for all schedulers - Timeout measured from descriptor fetch to completion - Event enables control MonBus traffic

### 4.4.3	Descriptor Engine Configuration

Descriptor Engine Configuration Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desceng_enable | input | 1 | Enable descriptor engine |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch |
| cfg_desceng_fifo_thresh | input | 4 | FIFO threshold for prefetch trigger |
| cfg_desceng_addr0_base | input | ADDR_WIDTH | Address range 0 base (protection) |
| cfg_desceng_addr0_limit | input | ADDR_WIDTH | Address range 0 limit (protection) |
| cfg_desceng_addr1_base | input | ADDR_WIDTH | Address range 1 base (protection) |
| cfg_desceng_addr1_limit | input | ADDR_WIDTH | Address range 1 limit (protection) |

Notes: - Descriptor engine shared across all channels - Prefetch improves latency for chained descriptors - Address ranges provide optional descriptor memory protection

### 4.4.4	AXI Monitor Configuration

Three identical sets of monitor config signals for descriptor, read, and write AXI masters:

AXI Monitor Signal Prefixes

| Signal Prefix | Applies To | Description |
| --- | --- | --- |
| cfg_desc_mon_* | Descriptor AXI | Descriptor fetch monitoring |
| cfg_rdeng_mon_* | Read AXI | Data read monitoring |
| cfg_wreng_mon_* | Write AXI | Data write monitoring |

Each monitor has the following configuration signals:

AXI Monitor Configuration Signal Suffixes

| Signal Suffix | Width | Description |
| --- | --- | --- |
| _enable | 1 | Enable monitor |
| _err_enable | 1 | Enable error packet reporting |
| _perf_enable | 1 | Enable performance packet reporting |
| _timeout_enable | 1 | Enable timeout detection |
| _timeout_cycles | 32 | Timeout threshold (cycles) |
| _latency_thresh | 32 | Latency threshold for events |
| _pkt_mask | 16 | Packet type mask (filter events) |
| _err_select | 4 | Error type selector |
| _err_mask | 8 | Error event mask |
| _timeout_mask | 8 | Timeout event mask |
| _compl_mask | 8 | Completion event mask |
| _thresh_mask | 8 | Threshold event mask |
| _perf_mask | 8 | Performance event mask |
| _addr_mask | 8 | Address event mask |
| _debug_mask | 8 | Debug event mask |

Example Signals:

cfg_desc_mon_enable          // Descriptor monitor enable
cfg_desc_mon_timeout_cycles  // Descriptor timeout threshold
cfg_rdeng_mon_err_enable     // Read engine error reporting enable
cfg_wreng_mon_perf_enable    // Write engine perf reporting enable

Notes: - Monitors generate MonBus packets for events - Masks control which event types are reported - Timeout measured from AR/AW valid to last R/B response

### 4.4.5	AXI Transfer Configuration

AXI Transfer Configuration Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | Read burst size (beats, 1-256) |
| cfg_axi_wr_xfer_beats | input | 8 | Write burst size (beats, 1-256) |

Notes: - Configures AXI burst length for read/write engines - Larger bursts improve bandwidth but increase latency - Typical: 16 beats for DDR4, 8 beats for low-latency SRAM

### 4.4.6	Performance Profiler Configuration

Performance Profiler Configuration Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_perf_enable | input | 1 | Enable profiler |
| cfg_perf_mode | input | 1 | Profiler mode (0=timestamp, 1=elapsed) |
| cfg_perf_clear | input | 1 | Clear profiler counters and FIFO |

Notes: - Timestamp mode: Records start/end timestamps (software calculates elapsed) - Elapsed mode: Hardware calculates elapsed time directly - Clear is a single-cycle pulse

## 4.5	Status Interface

### 4.5.1	Per-Channel Status

Per-Channel Status Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle[ch] | output | NUM_CHANNELS | Descriptor engine idle (no pending fetch) |
| scheduler_idle[ch] | output | NUM_CHANNELS | Scheduler in IDLE state (waiting for descriptor) |
| scheduler_state[ch] | output | NUM_CHANNELS × 7 | Scheduler FSM state (ONE-HOT encoding) |
| sched_error[ch] | output | NUM_CHANNELS | Scheduler error flag (sticky, cleared by reset) |
| axi_rd_all_complete[ch] | output | NUM_CHANNELS | All read transactions complete for channel |
| axi_wr_all_complete[ch] | output | NUM_CHANNELS | All write transactions complete for channel |

Scheduler State Encoding (ONE-HOT):

7'b0000001  // CH_IDLE - Waiting for descriptor
7'b0000010  // CH_FETCH_DESC - Fetching descriptor
7'b0000100  // CH_XFER_DATA - Transfer in progress
7'b0001000  // CH_COMPLETE - Transfer complete
7'b0010000  // CH_NEXT_DESC - Chaining to next descriptor
7'b0100000  // CH_ERROR - Error occurred
7'b1000000  // (Reserved)

Transfer Complete Condition:

channel_complete = scheduler_idle[ch] &&
                   axi_rd_all_complete[ch] &&
                   axi_wr_all_complete[ch];

## 4.6	Performance Profiler Interface

Performance Profiler Interface Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| perf_fifo_empty | output | 1 | Profiler FIFO empty flag |
| perf_fifo_full | output | 1 | Profiler FIFO full flag |
| perf_fifo_count | output | 16 | Profiler FIFO occupancy (0-256) |
| perf_fifo_rd | input | 1 | Read profiler entry (pop FIFO) |
| perf_fifo_data_low | output | 32 | Profiler data [31:0] (timestamp or elapsed) |
| perf_fifo_data_high | output | 32 | Profiler data [63:32] (metadata) |

FIFO Entry Format:

Low Word [31:0]: - Timestamp mode: Timestamp in clock cycles - Elapsed mode: Elapsed time in clock cycles

High Word [31:0]: - Bits [31:4]: Reserved (0) - Bit [3]: Event type (0=start, 1=end) - Bits [2:0]: Channel ID (0-7)

Read Sequence:

// Check FIFO not empty
if (!perf_fifo_empty) begin
    // Read 36-bit entry (two registers)
    perf_fifo_rd = 1'b1;  // Pulse to pop FIFO
    @(posedge clk);
    perf_fifo_rd = 1'b0;

    // Sample data on next cycle
    timestamp = perf_fifo_data_low;
    metadata = perf_fifo_data_high;
    channel_id = metadata[2:0];
    event_type = metadata[3];
end

## 4.7	AXI4 Master - Descriptor Fetch (256-bit)

Fixed 256-bit width AXI4 master for descriptor fetch from memory.

### 4.7.1	AR Channel (Read Address)

Descriptor AXI Master AR Channel Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_desc_arid | output | AXI_ID_WIDTH | Transaction ID |
| m_axi_desc_araddr | output | ADDR_WIDTH | Read address |
| m_axi_desc_arlen | output | 8 | Burst length - 1 (AXI encoding) |
| m_axi_desc_arsize | output | 3 | Burst size = log2(bytes per beat) |
| m_axi_desc_arburst | output | 2 | Burst type (01=INCR) |
| m_axi_desc_arlock | output | 1 | Lock type (0=normal) |
| m_axi_desc_arcache | output | 4 | Cache attributes |
| m_axi_desc_arprot | output | 3 | Protection attributes |
| m_axi_desc_arqos | output | 4 | QoS value |
| m_axi_desc_arregion | output | 4 | Region identifier |
| m_axi_desc_aruser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_desc_arvalid | output | 1 | Address valid |
| m_axi_desc_arready | input | 1 | Address ready |

### 4.7.2	R Channel (Read Data)

R Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_desc_rid | input | AXI_ID_WIDTH | Transaction ID |
| m_axi_desc_rdata | input | 256 | Read data (FIXED 256-bit descriptor) |
| m_axi_desc_rresp | input | 2 | Response (00=OKAY, 01=EXOKAY, 10=SLVERR, 11=DECERR) |
| m_axi_desc_rlast | input | 1 | Last beat of burst |
| m_axi_desc_ruser | input | CHAN_WIDTH | User signal (channel ID) |
| m_axi_desc_rvalid | input | 1 | Read data valid |
| m_axi_desc_rready | output | 1 | Read data ready |

Notes: - Data width is FIXED at 256 bits (descriptor size) - Typical burst: 1 beat (arlen=0) for single descriptor fetch - arsize = 3’b101 (32 bytes = 2^5) - ID encodes channel number for response routing

## 4.8	AXI4 Master - Data Read (Parameterizable Width)

Parameterizable width AXI4 master for reading source data from memory. Default 512-bit.

### 4.8.1	AR Channel (Read Address)

AR Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_arid | output | AXI_ID_WIDTH | Transaction ID |
| m_axi_rd_araddr | output | ADDR_WIDTH | Read address |
| m_axi_rd_arlen | output | 8 | Burst length - 1 |
| m_axi_rd_arsize | output | 3 | Burst size = log2(bytes per beat) |
| m_axi_rd_arburst | output | 2 | Burst type (01=INCR) |
| m_axi_rd_arlock | output | 1 | Lock type (0=normal) |
| m_axi_rd_arcache | output | 4 | Cache attributes |
| m_axi_rd_arprot | output | 3 | Protection attributes |
| m_axi_rd_arqos | output | 4 | QoS value |
| m_axi_rd_arregion | output | 4 | Region identifier |
| m_axi_rd_aruser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_rd_arvalid | output | 1 | Address valid |
| m_axi_rd_arready | input | 1 | Address ready |

### 4.8.2	R Channel (Read Data)

R Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_rid | input | AXI_ID_WIDTH | Transaction ID |
| m_axi_rd_rdata | input | DATA_WIDTH | Read data (default 512-bit) |
| m_axi_rd_rresp | input | 2 | Response |
| m_axi_rd_rlast | input | 1 | Last beat of burst |
| m_axi_rd_ruser | input | CHAN_WIDTH | User signal (channel ID) |
| m_axi_rd_rvalid | input | 1 | Read data valid |
| m_axi_rd_rready | output | 1 | Read data ready |

Notes: - Data width configurable via DATA_WIDTH parameter (default 512) - Burst length configured via cfg_axi_rd_xfer_beats (default 16) - arsize = log2(DATA_WIDTH/8) automatically calculated - AXI skid buffers on external interface for timing closure

## 4.9	AXI4 Master - Data Write (Parameterizable Width)

Parameterizable width AXI4 master for writing destination data to memory. Default 512-bit.

### 4.9.1	AW Channel (Write Address)

AW Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_awid | output | AXI_ID_WIDTH | Transaction ID |
| m_axi_wr_awaddr | output | ADDR_WIDTH | Write address |
| m_axi_wr_awlen | output | 8 | Burst length - 1 |
| m_axi_wr_awsize | output | 3 | Burst size = log2(bytes per beat) |
| m_axi_wr_awburst | output | 2 | Burst type (01=INCR) |
| m_axi_wr_awlock | output | 1 | Lock type (0=normal) |
| m_axi_wr_awcache | output | 4 | Cache attributes |
| m_axi_wr_awprot | output | 3 | Protection attributes |
| m_axi_wr_awqos | output | 4 | QoS value |
| m_axi_wr_awregion | output | 4 | Region identifier |
| m_axi_wr_awuser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_wr_awvalid | output | 1 | Address valid |
| m_axi_wr_awready | input | 1 | Address ready |

### 4.9.2	W Channel (Write Data)

W Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_wdata | output | DATA_WIDTH | Write data (default 512-bit) |
| m_axi_wr_wstrb | output | DATA_WIDTH/8 | Write strobes (byte enables, all 1s) |
| m_axi_wr_wlast | output | 1 | Last beat of burst |
| m_axi_wr_wuser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_wr_wvalid | output | 1 | Write data valid |
| m_axi_wr_wready | input | 1 | Write data ready |

### 4.9.3	B Channel (Write Response)

B Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_bid | input | AXI_ID_WIDTH | Transaction ID |
| m_axi_wr_bresp | input | 2 | Response (00=OKAY, 01=EXOKAY, 10=SLVERR, 11=DECERR) |
| m_axi_wr_buser | input | CHAN_WIDTH | User signal (channel ID) |
| m_axi_wr_bvalid | input | 1 | Response valid |
| m_axi_wr_bready | output | 1 | Response ready |

Notes: - Data width configurable via DATA_WIDTH parameter (default 512) - Burst length configured via cfg_axi_wr_xfer_beats (default 16) - awsize = log2(DATA_WIDTH/8) automatically calculated - wstrb typically all 1s (full data width writes) - AXI skid buffers on external interface for timing closure

## 4.10	Status/Debug Outputs

### 4.10.1	Descriptor AXI Monitor Status

Descriptor AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sts_desc_mon_busy | output | 1 | Monitor busy processing transaction |
| cfg_sts_desc_mon_active_txns | output | 8 | Active transaction count (0-255) |
| cfg_sts_desc_mon_error_count | output | 16 | Cumulative error count |
| cfg_sts_desc_mon_txn_count | output | 32 | Total transaction count |
| cfg_sts_desc_mon_conflict_error | output | 1 | ID conflict detected (sticky) |

### 4.10.2	Read Engine AXI Monitor Status

Read Engine AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sts_rdeng_skid_busy | output | 1 | Skid buffer busy (not empty) |
| cfg_sts_rdeng_mon_active_txns | output | 8 | Active transaction count |
| cfg_sts_rdeng_mon_error_count | output | 16 | Cumulative error count |
| cfg_sts_rdeng_mon_txn_count | output | 32 | Total transaction count |
| cfg_sts_rdeng_mon_conflict_error | output | 1 | ID conflict detected (sticky) |

### 4.10.3	Write Engine AXI Monitor Status

Write Engine AXI Monitor Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sts_wreng_skid_busy | output | 1 | Skid buffer busy (not empty) |
| cfg_sts_wreng_mon_active_txns | output | 8 | Active transaction count |
| cfg_sts_wreng_mon_error_count | output | 16 | Cumulative error count |
| cfg_sts_wreng_mon_txn_count | output | 32 | Total transaction count |
| cfg_sts_wreng_mon_conflict_error | output | 1 | ID conflict detected (sticky) |

Notes: - Status signals for debug and performance analysis - Transaction counts never roll over (use for profiling) - Error counts increment on any AXI error response - ID conflicts indicate internal RTL bug (should never occur)

## 4.11	Unified Monitor Bus Interface

Single output interface for all STREAM monitoring events.

Unified Monitor Bus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready (from downstream consumer) |
| mon_packet | output | 64 | Monitor packet data (64-bit standard format) |

MonBus Packet Format (64-bit):

[63:56] - Packet type (event code)
[55:48] - Agent ID (source identifier)
[47:40] - Unit ID (1 for STREAM)
[39:32] - Channel ID (0-7)
[31:0]  - Event-specific data

MonBus Sources: - Descriptor engines: 8 sources, agent IDs 16-23 (0x10-0x17) - Schedulers: 8 sources, agent IDs 48-55 (0x30-0x37) - Descriptor AXI monitor: agent ID 8 (0x08) - Read AXI monitor: configurable agent ID - Write AXI monitor: configurable agent ID

Downstream Integration:

// Connect to MonBus FIFO for buffering
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_mon_fifo (
    .i_clk    (clk),
    .i_rst_n  (rst_n),
    .i_data   (mon_packet),
    .i_valid  (mon_valid),
    .o_ready  (mon_ready),
    // ... downstream connection
);

Notes: - Standard AMBA monitor bus protocol - Always buffer with FIFO to prevent backpressure to STREAM - Packet format documented in rtl/amba/includes/monitor_pkg.sv - Event codes defined in STREAM package

## 4.12	Port Count Summary

Port Count Summary

| Interface Type | Input Ports | Output Ports | Bidirectional |
| --- | --- | --- | --- |
| Clock/Reset | 2 | 0 | 0 |
| APB Programming | NUM_CHANNELS × (1 + ADDR_WIDTH) | NUM_CHANNELS | 0 |
| Configuration | ~150 | 0 | 0 |
| Status | 1 (perf_fifo_rd) | NUM_CHANNELS × 10 + 30 | 0 |
| Perf Profiler | 1 | 4 | 0 |
| AXI Desc Master | 7 | 14 | 0 |
| AXI Read Master | 7 | 14 | 0 |
| AXI Write Master | 10 | 19 | 0 |
| MonBus | 1 | 2 | 0 |

Approximate Total: ~350 ports (varies with NUM_CHANNELS and ADDR_WIDTH/DATA_WIDTH)

## 4.13	Default Parameter Values

Default Parameter Values

| Parameter | Default | Description |
| --- | --- | --- |
| NUM_CHANNELS | 8 | Number of DMA channels |
| CHAN_WIDTH | 3 | Channel ID width (log2(NUM_CHANNELS)) |
| ADDR_WIDTH | 64 | Address bus width |
| DATA_WIDTH | 512 | Data bus width |
| AXI_ID_WIDTH | 8 | AXI transaction ID width |
| FIFO_DEPTH | 512 | Per-channel FIFO depth |
| AR_MAX_OUTSTANDING | 8 | Max concurrent read requests |
| AW_MAX_OUTSTANDING | 8 | Max concurrent write requests |

## 4.14	Related Documentation

- Stream Core Block Spec - Detailed block documentation

- Clocks and Reset - Timing specifications

- Architecture - System architecture overview

Last Updated: 2025-12-01 Maintained By: STREAM Architecture Team —

## 4.15	Revision History

STREAM Top-Level Port List Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Clocks and Reset Specification

Chapter: 01 Version: 0.90 Last Updated: 2025-11-22

## 5.1	Overview

STREAM operates in a single clock domain with a single asynchronous active-low reset. This chapter defines clock requirements, reset behavior, and timing constraints for the STREAM subsystem.

## 5.2	Clock Domain

### 5.2.1	Primary Clock: aclk

Specification: - Name: aclk (AXI clock) - Type: Synchronous, single-edge (rising edge) - Frequency: Configurable (typical: 100 MHz - 500 MHz) - Duty Cycle: 50% 5% - Jitter: < 100 ps peak-to-peak

Usage: - All STREAM internal logic - All AXI master interfaces - All AXIL interfaces - MonBus output - SRAM

### 5.2.2	Secondary Clock: pclk (APB Clock)

Specification: - Name: pclk (Peripheral clock) - Type: Synchronous, single-edge (rising edge) - Frequency: Configurable (typical: 50 MHz - 200 MHz) - Relation to aclk: May be asynchronous

Usage: - APB configuration interface only

Clock Domain Crossing (CDC): - If pclk aclk: CDC logic required in apb_config.sv - If pclk = aclk: Direct connection (no CDC)

CDC Implementation: - Use apb_slave_cdc wrapper (like HPET example) - apb_slave_cdc implements handshake-based CDC using cdc_handshake modules - One cdc_handshake for command interface (APB → core) - One cdc_handshake for response interface (core → APB) - Full req/ack handshake protocol (NOT async FIFO) - Works across all frequency ratios (slow-to-fast, fast-to-slow, any ratio)

## 5.3	Reset

### 5.3.1	Primary Reset: aresetn

Specification: - Name: aresetn (AXI reset, active-low) - Type: Asynchronous assertion, synchronous deassertion - Polarity: Active-low (0 = reset, 1 = normal operation) - Assertion: Asynchronous (can occur at any time) - Deassertion: Synchronous to aclk rising edge - Duration: Minimum 10 aclk cycles

Reset Behavior:

// Standard reset pattern for all STREAM modules
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        // Asynchronous reset assertion
        r_state <= IDLE;
        r_counter <= '0;
        r_valid <= 1'b0;
        // ... all registers to known state
    end else begin
        // Synchronous operation
        r_state <= w_next_state;
        // ... normal logic
    end
end

### 5.3.2	Secondary Reset: presetn

Specification: - Name: presetn (APB reset, active-low) - Type: Asynchronous assertion, synchronous deassertion - Polarity: Active-low (0 = reset, 1 = normal operation) - Synchronization: May be asynchronous to aresetn

Usage: - APB configuration interface only - Typically tied to aresetn if pclk = aclk

## 5.4	Reset Sequencing

### 5.4.1	Power-On Reset

Recommended sequence:

1. Assert aresetn (LOW)
2. Assert presetn (LOW)
3. Apply stable clocks (aclk, pclk)
4. Wait   10 aclk cycles
5. Deassert presetn (HIGH) on pclk rising edge
6. Deassert aresetn (HIGH) on aclk rising edge
7. Wait   5 aclk cycles for stabilization
8. Begin APB configuration

Timing diagram:

aclk     __| |_| |_| |_| |_| |_| |_| |_| |_| |_| |_| |_
pclk     __| |_| |_| |_| |_| |_| |_| |_| |_| |_| |_| |_
aresetn  __________|                                  
presetn  __________|                                  
         <--10 cyc-->

### 5.4.2	Functional Reset

Software-initiated reset (per channel):

// Reset specific channel via APB
write_apb(ADDR_GLOBAL_CTRL, CHANNEL_0_RESET);  // Auto-clears after 1 cycle

// Hardware response:
// - Channel FSM returns to IDLE
// - Channel registers cleared
// - Outstanding transactions flushed
// - MonBus error packet generated (if mid-transfer)

### 5.4.3	Reset Recovery

After reset deassertion:

Reset Recovery

| Cycle | Event |
| --- | --- |
| 0 | aresetn deasserted (rising edge) |
| 1-5 | Internal state stabilization |
| 6+ | Ready for APB configuration |
| 10+ | Ready for descriptor transfers |

## 5.5	Clock Requirements by Module

### 5.5.1	Functional Unit Blocks (FUB)

Functional Unit Blocks

| Module | Clock | Reset | Frequency | Notes |
| --- | --- | --- | --- | --- |
| descriptor_engine | aclk | aresetn | 100-500 MHz | AXI master timing |
| scheduler | aclk | aresetn | 100-500 MHz | Single cycle FSM |
| axi_read_engine | aclk | aresetn | 100-500 MHz | AXI master timing |
| axi_write_engine | aclk | aresetn | 100-500 MHz | AXI master timing |
| simple_sram | aclk | aresetn | 100-500 MHz | Synchronous SRAM |

### 5.5.2	Integration Blocks (MAC)

Integration Blocks

| Module | Clock(s) | Reset(s) | Frequency | Notes |
| --- | --- | --- | --- | --- |
| channel_arbiter | aclk | aresetn | 100-500 MHz | Single cycle arbitration |
| apb_config | pclk, (aclk) | presetn, (aresetn) | 50-200 MHz (APB) | CDC if async |
| monbus_axil_group | aclk | aresetn | 100-500 MHz | AXIL timing |
| stream_top | aclk, pclk | aresetn, presetn | Mixed | Top-level |

## 5.6	Timing Constraints

### 5.6.1	Setup and Hold Times

Internal registers (relative to aclk): - Setup time: 0.5 ns (typical) - Hold time: 0.1 ns (typical) - Clock-to-Q: 0.3 ns (typical)

External interfaces: - AXI/AXIL: Per ARM IHI0022E specification - APB: Per ARM IHI0024C specification

### 5.6.2	Critical Paths

Identified critical paths:

- Arbiter -> Scheduler grant:

  - Latency: 1 cycle

  - Path: Priority encoder -> One-hot grant

- AXI read -> SRAM write:

  - Latency: 1 cycle

  - Path: R data -> SRAM write port

- SRAM read -> AXI write:

  - Latency: 1 cycle

  - Path: SRAM read port -> W data

Maximum frequency estimation: - Typical FPGA (Xilinx 7-series): 250 MHz - High-end FPGA (UltraScale+): 400 MHz - ASIC (28nm): 500 MHz

## 5.7	Clock Domain Crossing (CDC)

### 5.7.1	APB Configuration CDC

When required: pclk aclk (asynchronous APB interface)

CDC Implementation:

// APB to STREAM domain (pclk -> aclk)
apb_slave_cdc #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .SYNC_STAGES(2)  // Dual-flop synchronizer
) u_apb_cdc (
    // APB side (pclk domain)
    .s_pclk(pclk),
    .s_presetn(presetn),
    .s_paddr(paddr),
    .s_pwrite(pwrite),
    .s_pwdata(pwdata),
    .s_prdata(prdata),

    // STREAM side (aclk domain)
    .m_pclk(aclk),
    .m_presetn(aresetn),
    .m_paddr(paddr_sync),
    .m_pwrite(pwrite_sync),
    .m_pwdata(pwdata_sync),
    .m_prdata(prdata_sync)
);

CDC Mechanism: - apb_slave_cdc uses handshake-based CDC via cdc_handshake modules (NOT async FIFOs) - Command path (pclk → aclk): APB write/read commands cross via req/ack handshake - Response path (aclk → pclk): APB read data crosses back via req/ack handshake - Internal synchronizers: Dual-flop synchronizers (2-3 stages) for handshake signals - ASYNC_REG attribute: Applied to synchronizer stages for timing tools - Timing constraints: Proper constraints required in SDC/XDC for synchronizer paths

Handshake Protocol Benefits: - Works across all frequency ratios (slow-to-fast, fast-to-slow, arbitrary ratios) - Guaranteed data integrity (req/ack ensures data stability before sampling) - No FIFO depth management or gray code pointer complexity - Latency: 4-6 APB clock cycles for register access (handshake round-trip)

### 5.7.2	No CDC Required

Single clock domain: If pclk = aclk and presetn = aresetn:

// Direct connection (no CDC wrapper)
apb_config #(
    .NUM_CHANNELS(8)
) u_apb_config (
    .pclk(aclk),        // Same clock
    .presetn(aresetn),  // Same reset
    // ... direct APB signals
);

## 5.8	Reset State Initialization

### 5.8.1	Register Reset Values

All STREAM modules must initialize to known state on reset:

// Descriptor Engine
if (!aresetn) begin
    r_desc_fifo_wr_ptr <= '0;
    r_desc_valid <= 1'b0;
    r_desc_error <= 1'b0;
end

// Scheduler
if (!aresetn) begin
    r_current_state <= CH_IDLE;
    r_read_beats_remaining <= '0;
    r_write_beats_remaining <= '0;
    r_timeout_counter <= '0;
end

// AXI Engines
if (!aresetn) begin
    r_burst_counter <= '0;
    m_axi_arvalid <= 1'b0;
    m_axi_awvalid <= 1'b0;
end

// Arbiter
if (!aresetn) begin
    r_last_grant_id <= '0;
    r_grant_valid <= 1'b0;
end

### 5.8.2	SRAM Reset

SRAM contents: Undefined after reset (no initialization required)

SRAM pointers:

if (!aresetn) begin
    wr_ptr <= '0;
    rd_ptr <= '0;
end

## 5.9	Clock Gating (Optional)

For power optimization in ASIC implementations:

### 5.9.1	Per-Channel Clock Gating

// Clock gate when channel idle
clock_gate_ctrl u_ch0_clk_gate (
    .clk_in(aclk),
    .enable(ch0_enable),
    .clk_out(ch0_gated_clk)
);

// Use gated clock for channel logic
scheduler #(.CHANNEL_ID(0)) u_ch0_sched (
    .aclk(ch0_gated_clk),  // Gated clock
    .aresetn(aresetn),
    // ...
);

Note: Clock gating typically not used in FPGA implementations (tutorial focus).

## 5.10	Verification Requirements

### 5.10.1	Clock Checks

Testbench must verify: - [Done] Clock period consistent - [Done] Clock duty cycle 50% tolerance - [Done] No glitches on clock - [Done] Setup/hold times met

### 5.10.2	Reset Checks

Testbench must verify: - [Done] All registers initialize to known state - [Done] Reset assertion clears FSMs to IDLE - [Done] Reset deassertion synchronous to clock - [Done] Minimum reset duration (10 cycles) enforced - [Done] Operations don’t start until stabilization complete

### 5.10.3	CDC Checks

For APB CDC (if present): - [Done] No metastability violations - [Done] Data integrity across domains - [Done] Proper flag synchronization

## 5.11	Example Reset Testbench

# CocoTB testbench pattern
class StreamTB(TBBase):
    async def setup_clocks_and_reset(self):
        """Complete clock and reset initialization"""
        # Start clocks
        await self.start_clock('aclk', freq=10, units='ns')  # 100 MHz
        await self.start_clock('pclk', freq=20, units='ns')  # 50 MHz (async)

        # Assert reset
        await self.assert_reset()

        # Hold reset for 10 aclk cycles
        await self.wait_clocks('aclk', 10)

        # Deassert reset (synchronous to aclk)
        await self.deassert_reset()

        # Stabilization period
        await self.wait_clocks('aclk', 5)

        # Ready for operation

    async def assert_reset(self):
        """Assert both resets"""
        self.dut.aresetn.value = 0
        self.dut.presetn.value = 0

    async def deassert_reset(self):
        """Deassert both resets synchronously"""
        # Wait for rising edge of aclk
        await RisingEdge(self.dut.aclk)
        self.dut.aresetn.value = 1

        # Wait for rising edge of pclk
        await RisingEdge(self.dut.pclk)
        self.dut.presetn.value = 1

## 5.12	Related Documentation

- Scheduler FSM: fub_02_scheduler.md - Reset behavior

- APB Config: mac_02_apb_config.md - CDC implementation

## Top-Level: mac_04_stream_top.md - Clock/reset integration

## 5.13	Revision History

Clocks and Reset Specification Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	STREAM Core Specification

Module: stream_core.sv Location: projects/components/stream/rtl/macro/ Status: Implemented Last Updated: 2025-11-30

## 6.1	Overview

The STREAM Core is the top-level integration module that combines all STREAM components into a complete scatter-gather DMA engine. It provides 8 independent channels with descriptor-based memory-to-memory transfers.

### 6.1.1	Key Features

- 8 Independent Channels: Concurrent descriptor-based transfers

- Shared AXI Masters: Three shared AXI4 masters for efficiency

  - Descriptor fetch (256-bit fixed)

  - Data read (parameterizable, default 512-bit)

  - Data write (parameterizable, default 512-bit)

- Per-Channel Buffering: Independent FIFO per channel (512 entries default)

- Performance Monitoring: Integrated profiler with FIFO readout

- AXI Skid Buffers: Timing closure on all external interfaces

- Unified MonBus: Single monitor bus output for all events

### 6.1.2	Block Diagram

The STREAM Core integrates the following major components:

### 6.1.3	Figure 2.1.1: STREAM Core Block Diagram

STREAM Core Block Diagram

Source: 01_stream_core_block.mmd

## 6.2	Architecture

### 6.2.1	Component Hierarchy

- scheduler_group_array - Top scheduler layer

  - 8 × scheduler_group instances

  - Shared descriptor engine

  - Descriptor fetch arbitration

- sram_controller - Buffering layer

  - 8 × independent FIFOs (gaxi_fifo_sync)

  - Per-channel allocation controllers

  - Per-channel drain controllers

- axi_read_engine - Read datapath

  - Shared AXI master for all channels

  - ID-based routing to SRAM

  - Space allocation flow control

- axi_write_engine - Write datapath

  - Shared AXI master for all channels

  - ID-based routing from SRAM

  - Drain reservation flow control

- perf_profiler - Performance monitoring

  - Transaction counting

  - Bandwidth tracking

  - FIFO readout interface

- AXI Skid Buffers - Timing closure

  - Descriptor AXI (AR/R)

  - Read AXI (AR/R)

  - Write AXI (AW/W/B)

### 6.2.2	Data Flow

Descriptor Fetch Flow:

.1.2: Descriptor Fetch Flow

Data Transfer Flow:

.1.3: Data Transfer Flow

## 6.3	Parameters

### 6.3.1	Primary Configuration

Primary Configuration

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | Number of independent DMA channels |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | Channel ID width (3 for 8 channels) |
| ADDR_WIDTH | int | 64 | Address bus width |
| DATA_WIDTH | int | 512 | Data bus width (must match memory interface) |
| AXI_ID_WIDTH | int | 8 | AXI transaction ID width |
| FIFO_DEPTH | int | 512 | Per-channel FIFO depth |

### 6.3.2	Monitor Control

Monitor Control

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| USE_AXI_MONITORS | int | 1 | Enable (1) or disable (0) AXI transaction monitors |

Note: When USE_AXI_MONITORS = 0: - All monitor configuration inputs are tied off internally - MonBus output remains present but inactive (no packets generated) - Reduces resource usage for production systems

### 6.3.3	Outstanding Transaction Limits

Outstanding Transaction Limits

| Parameter | Default | Description |
| --- | --- | --- |
| AR_MAX_OUTSTANDING | 8 | Maximum concurrent read address requests |
| AW_MAX_OUTSTANDING | 8 | Maximum concurrent write address requests |

### 6.3.4	AXI Skid Buffer Depths

AXI Skid Buffer Depths

| Parameter | Default | Purpose |
| --- | --- | --- |
| SKID_DEPTH_AR | 2 | AR channel timing closure |
| SKID_DEPTH_R | 4 | R channel timing closure |
| SKID_DEPTH_AW | 2 | AW channel timing closure |
| SKID_DEPTH_W | 4 | W channel timing closure |
| SKID_DEPTH_B | 2 | B channel timing closure |

Note: Deeper buffers on data channels (R/W) improve throughput.

### 6.3.5	MonBus Agent IDs

MonBus Agent IDs

| Parameter | Default | Description |
| --- | --- | --- |
| DESC_MON_BASE_AGENT_ID | 16 (0x10) | Descriptor engines base (16-23) |
| SCHED_MON_BASE_AGENT_ID | 48 (0x30) | Schedulers base (48-55) |
| DESC_AXI_MON_AGENT_ID | 8 (0x08) | Descriptor AXI master monitor |
| MON_UNIT_ID | 1 (0x1) | Unit ID for all STREAM events |

## 6.4	Port List

### 6.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 6.4.2	APB Programming Interface

Per-channel descriptor kick-off interface:

APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid[ch] | input | NUM_CHANNELS | Channel descriptor address valid |
| apb_ready[ch] | output | NUM_CHANNELS | Channel ready to accept descriptor address |
| apb_addr[ch] | input | NUM_CHANNELS × ADDR_WIDTH | Descriptor address per channel |

Usage:

// Kick off channel 0 with descriptor at 0x1000_0000
apb_valid[0] = 1'b1;
apb_addr[0] = 64'h0000_0000_1000_0000;
// Wait for handshake
wait (apb_ready[0]);
apb_valid[0] = 1'b0;

### 6.4.3	Configuration Interface

Per-Channel Configuration:

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable[ch] | input | NUM_CHANNELS | Enable channel |
| cfg_channel_reset[ch] | input | NUM_CHANNELS | Soft reset channel (FSM → IDLE) |

Global Scheduler Configuration:

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_enable | input | 1 | Global scheduler enable |
| cfg_sched_timeout_cycles | input | 16 | Timeout threshold (cycles) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_sched_err_enable | input | 1 | Enable error event reporting |
| cfg_sched_compl_enable | input | 1 | Enable completion event reporting |
| cfg_sched_perf_enable | input | 1 | Enable performance event reporting |

Descriptor Engine Configuration:

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desceng_enable | input | 1 | Enable descriptor engine |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch |
| cfg_desceng_fifo_thresh | input | 4 | FIFO threshold for prefetch |
| cfg_desceng_addr0_base | input | ADDR_WIDTH | Base address limit 0 |
| cfg_desceng_addr0_limit | input | ADDR_WIDTH | Limit address limit 0 |
| cfg_desceng_addr1_base | input | ADDR_WIDTH | Base address limit 1 |
| cfg_desceng_addr1_limit | input | ADDR_WIDTH | Limit address limit 1 |

AXI Monitor Configuration:

Three identical sets of monitor config signals (descriptor, read, write):

Configuration Interface

| Signal Prefix | Applies To | Description |
| --- | --- | --- |
| cfg_desc_mon_* | Descriptor AXI | Descriptor fetch monitoring |
| cfg_rdeng_mon_* | Read AXI | Data read monitoring |
| cfg_wreng_mon_* | Write AXI | Data write monitoring |

Each monitor has:

| Signal Suffix | Width | Description |
| --- | --- | --- |
| _enable | 1 | Enable monitor |
| _err_enable | 1 | Enable error reporting |
| _perf_enable | 1 | Enable performance reporting |
| _timeout_enable | 1 | Enable timeout detection |
| _timeout_cycles | 32 | Timeout threshold |
| _latency_thresh | 32 | Latency threshold for events |
| _pkt_mask | 16 | Packet type mask |
| _err_select | 4 | Error type selector |
| _err_mask | 8 | Error event mask |
| _timeout_mask | 8 | Timeout event mask |
| _compl_mask | 8 | Completion event mask |
| _thresh_mask | 8 | Threshold event mask |
| _perf_mask | 8 | Performance event mask |
| _addr_mask | 8 | Address event mask |
| _debug_mask | 8 | Debug event mask |

AXI Transfer Configuration:

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | Read burst size (beats) |
| cfg_axi_wr_xfer_beats | input | 8 | Write burst size (beats) |

Performance Profiler Configuration:

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_perf_enable | input | 1 | Enable profiler |
| cfg_perf_mode | input | 1 | Profiler mode (0=count, 1=latency) |
| cfg_perf_clear | input | 1 | Clear profiler counters |

### 6.4.4	Status Interface

System-Level Status:

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| system_idle | output | 1 | All channels idle (AND of all scheduler_idle) |

Per-Channel Status:

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle[ch] | output | NUM_CHANNELS | Descriptor engine idle |
| scheduler_idle[ch] | output | NUM_CHANNELS | Scheduler in IDLE state |
| scheduler_state[ch] | output | NUM_CHANNELS × 7 | Scheduler FSM state (ONE-HOT) |
| sched_error[ch] | output | NUM_CHANNELS | Scheduler error (sticky) |
| axi_rd_all_complete[ch] | output | NUM_CHANNELS | All read transactions complete |
| axi_wr_all_complete[ch] | output | NUM_CHANNELS | All write transactions complete |

Performance Profiler Status:

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| perf_fifo_empty | output | 1 | Profiler FIFO empty |
| perf_fifo_full | output | 1 | Profiler FIFO full |
| perf_fifo_count | output | 16 | Profiler FIFO occupancy |
| perf_fifo_rd | input | 1 | Read profiler entry |
| perf_fifo_data_low | output | 32 | Profiler data [31:0] |
| perf_fifo_data_high | output | 32 | Profiler data [63:32] |

### 6.4.5	AXI4 Master - Descriptor Fetch (256-bit)

AR Channel:

AXI4 Master - Descriptor Fetch

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_desc_arid | output | AXI_ID_WIDTH | Transaction ID |
| m_axi_desc_araddr | output | ADDR_WIDTH | Address |
| m_axi_desc_arlen | output | 8 | Burst length - 1 |
| m_axi_desc_arsize | output | 3 | Burst size (log2 bytes) |
| m_axi_desc_arburst | output | 2 | Burst type (INCR) |
| m_axi_desc_arlock | output | 1 | Lock type |
| m_axi_desc_arcache | output | 4 | Cache attributes |
| m_axi_desc_arprot | output | 3 | Protection attributes |
| m_axi_desc_arqos | output | 4 | QoS value |
| m_axi_desc_arregion | output | 4 | Region identifier |
| m_axi_desc_aruser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_desc_arvalid | output | 1 | Address valid |
| m_axi_desc_arready | input | 1 | Address ready |

R Channel:

AXI4 Master - Descriptor Fetch

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_desc_rid | input | AXI_ID_WIDTH | Transaction ID |
| m_axi_desc_rdata | input | 256 | Read data (FIXED 256-bit) |
| m_axi_desc_rresp | input | 2 | Response (OKAY/EXOKAY/SLVERR/DECERR) |
| m_axi_desc_rlast | input | 1 | Last beat of burst |
| m_axi_desc_ruser | input | CHAN_WIDTH | User signal (channel ID) |
| m_axi_desc_rvalid | input | 1 | Read data valid |
| m_axi_desc_rready | output | 1 | Read data ready |

### 6.4.6	AXI4 Master - Data Read (Parameterizable Width)

AR Channel:

AXI4 Master - Data Read

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_arid | output | AXI_ID_WIDTH | Transaction ID |
| m_axi_rd_araddr | output | ADDR_WIDTH | Address |
| m_axi_rd_arlen | output | 8 | Burst length - 1 |
| m_axi_rd_arsize | output | 3 | Burst size (log2 bytes) |
| m_axi_rd_arburst | output | 2 | Burst type (INCR) |
| m_axi_rd_arlock | output | 1 | Lock type |
| m_axi_rd_arcache | output | 4 | Cache attributes |
| m_axi_rd_arprot | output | 3 | Protection attributes |
| m_axi_rd_arqos | output | 4 | QoS value |
| m_axi_rd_arregion | output | 4 | Region identifier |
| m_axi_rd_aruser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_rd_arvalid | output | 1 | Address valid |
| m_axi_rd_arready | input | 1 | Address ready |

R Channel:

AXI4 Master - Data Read

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rd_rid | input | AXI_ID_WIDTH | Transaction ID |
| m_axi_rd_rdata | input | DATA_WIDTH | Read data (default 512-bit) |
| m_axi_rd_rresp | input | 2 | Response |
| m_axi_rd_rlast | input | 1 | Last beat of burst |
| m_axi_rd_ruser | input | CHAN_WIDTH | User signal (channel ID) |
| m_axi_rd_rvalid | input | 1 | Read data valid |
| m_axi_rd_rready | output | 1 | Read data ready |

### 6.4.7	AXI4 Master - Data Write (Parameterizable Width)

AW Channel:

AXI4 Master - Data Write

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_awid | output | AXI_ID_WIDTH | Transaction ID |
| m_axi_wr_awaddr | output | ADDR_WIDTH | Address |
| m_axi_wr_awlen | output | 8 | Burst length - 1 |
| m_axi_wr_awsize | output | 3 | Burst size (log2 bytes) |
| m_axi_wr_awburst | output | 2 | Burst type (INCR) |
| m_axi_wr_awlock | output | 1 | Lock type |
| m_axi_wr_awcache | output | 4 | Cache attributes |
| m_axi_wr_awprot | output | 3 | Protection attributes |
| m_axi_wr_awqos | output | 4 | QoS value |
| m_axi_wr_awregion | output | 4 | Region identifier |
| m_axi_wr_awuser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_wr_awvalid | output | 1 | Address valid |
| m_axi_wr_awready | input | 1 | Address ready |

W Channel:

AXI4 Master - Data Write

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_wdata | output | DATA_WIDTH | Write data (default 512-bit) |
| m_axi_wr_wstrb | output | DATA_WIDTH/8 | Write strobes (byte enables) |
| m_axi_wr_wlast | output | 1 | Last beat of burst |
| m_axi_wr_wuser | output | CHAN_WIDTH | User signal (channel ID) |
| m_axi_wr_wvalid | output | 1 | Write data valid |
| m_axi_wr_wready | input | 1 | Write data ready |

B Channel:

AXI4 Master - Data Write

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wr_bid | input | AXI_ID_WIDTH | Transaction ID |
| m_axi_wr_bresp | input | 2 | Response |
| m_axi_wr_buser | input | CHAN_WIDTH | User signal (channel ID) |
| m_axi_wr_bvalid | input | 1 | Response valid |
| m_axi_wr_bready | output | 1 | Response ready |

### 6.4.8	Status/Debug Outputs

Descriptor AXI Monitor:

Status/Debug Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sts_desc_mon_busy | output | 1 | Monitor busy |
| cfg_sts_desc_mon_active_txns | output | 8 | Active transaction count |
| cfg_sts_desc_mon_error_count | output | 16 | Error count |
| cfg_sts_desc_mon_txn_count | output | 32 | Total transaction count |
| cfg_sts_desc_mon_conflict_error | output | 1 | ID conflict detected |

Read Engine AXI Monitor:

Status/Debug Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sts_rdeng_skid_busy | output | 1 | Skid buffer busy |
| cfg_sts_rdeng_mon_active_txns | output | 8 | Active transaction count |
| cfg_sts_rdeng_mon_error_count | output | 16 | Error count |
| cfg_sts_rdeng_mon_txn_count | output | 32 | Total transaction count |
| cfg_sts_rdeng_mon_conflict_error | output | 1 | ID conflict detected |

Write Engine AXI Monitor:

Status/Debug Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sts_wreng_skid_busy | output | 1 | Skid buffer busy |
| cfg_sts_wreng_mon_active_txns | output | 8 | Active transaction count |
| cfg_sts_wreng_mon_error_count | output | 16 | Error count |
| cfg_sts_wreng_mon_txn_count | output | 32 | Total transaction count |
| cfg_sts_wreng_mon_conflict_error | output | 1 | ID conflict detected |

### 6.4.9	Unified Monitor Bus Interface

Unified Monitor Bus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready |
| mon_packet | output | 64 | Monitor packet data |

MonBus Sources: - Descriptor engines (8 sources, agent IDs 16-23) - Schedulers (8 sources, agent IDs 48-55) - Descriptor AXI monitor (agent ID 8) - Read AXI monitor (configurable) - Write AXI monitor (configurable)

## 6.5	Operation

### 6.5.1	Transfer Initialization

Step 1: Configuration

1. Configure global settings (timeouts, monitors, transfer sizes)
2. Enable descriptor engine (cfg_desceng_enable = 1)
3. Enable scheduler (cfg_sched_enable = 1)
4. Enable target channel (cfg_channel_enable[ch] = 1)

Step 2: Descriptor Kick-off

1. Assert apb_valid[ch] = 1
2. Provide descriptor address on apb_addr[ch]
3. Wait for apb_ready[ch] handshake
4. Deassert apb_valid[ch]

Step 3: Automatic Transfer

1. Descriptor engine fetches descriptor via m_axi_desc_*
2. Scheduler receives descriptor, starts transfer
3. Read engine: memory → SRAM (via m_axi_rd_*)
4. Write engine: SRAM → memory (via m_axi_wr_*)
5. Scheduler reports completion via MonBus

### 6.5.2	Monitoring Transfer Progress

Check Scheduler State:

// Monitor scheduler state
case (scheduler_state[ch])
    7'b0000001: // CH_IDLE - waiting for descriptor
    7'b0000010: // CH_FETCH_DESC - fetching descriptor
    7'b0000100: // CH_XFER_DATA - transfer in progress
    7'b0001000: // CH_COMPLETE - transfer complete
    7'b0010000: // CH_NEXT_DESC - chaining to next
    7'b0100000: // CH_ERROR - error occurred
endcase

Check Completion:

// All complete when:
complete = scheduler_idle[ch] &&
           axi_rd_all_complete[ch] &&
           axi_wr_all_complete[ch];

## 6.6	Timing Diagrams

### 6.6.1	Complete Data Flow Timing

The following diagram shows the complete data flow through the STREAM core, from descriptor kick-off through data transfer completion:

#### 6.6.1.1	Waveform 2.1.1: STREAM Core Data Flow

STREAM Core Data Flow

Source: stream_core_data_flow.json

## 6.7	Testing

Test Location: projects/components/stream/dv/tests/top/

Key Test Scenarios:

- Single channel transfer - Basic end-to-end operation

- Multi-channel concurrent - 2-8 channels simultaneously

- Descriptor chaining - 2-5 descriptors linked

- FIFO overflow prevention - Large transfer with small FIFO

- Error handling - AXI errors, timeouts

- Performance profiling - Bandwidth measurements

- MonBus event checking - Verify all events reported

Test Configuration:

# Basic configuration
NUM_CHANNELS = 4
DATA_WIDTH = 128
FIFO_DEPTH = 512
cfg_axi_rd_xfer_beats = 16
cfg_axi_wr_xfer_beats = 16

## 6.8	Resource Utilization

Estimated Resources (8 channels, 512-bit data, 512-deep FIFOs):

Resource Utilization

| Component | Quantity | Est. Size |
| --- | --- | --- |
| Schedulers | 8 | 8 × ~500 FFs |
| Descriptor Engine | 1 | ~1000 FFs |
| SRAM FIFOs | 8 | 8 × 512 × 512-bit = 256KB |
| AXI Engines | 2 | 2 × ~2000 FFs |
| Skid Buffers | 3 sets | ~2000 FFs |
| Monitors | 3 | 3 × ~1000 FFs |
| Total |  | ~20K FFs + 256KB SRAM |

Critical Paths: - AXI handshake paths (improved by skid buffers) - SRAM address decode - MonBus arbiter

## 6.9	Integration Example

stream_core #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .FIFO_DEPTH(512)
) u_stream (
    .clk                    (system_clk),
    .rst_n                  (system_rst_n),

    // APB kick-off
    .apb_valid              (stream_apb_valid),
    .apb_ready              (stream_apb_ready),
    .apb_addr               (stream_apb_addr),

    // Configuration
    .cfg_channel_enable     (stream_ch_enable),
    .cfg_sched_enable       (1'b1),
    .cfg_axi_rd_xfer_beats  (8'd16),
    .cfg_axi_wr_xfer_beats  (8'd16),
    // ... other config

    // AXI Descriptor Master
    .m_axi_desc_arid        (desc_arid),
    .m_axi_desc_araddr      (desc_araddr),
    // ... full AXI AR/R

    // AXI Read Master
    .m_axi_rd_arid          (rd_arid),
    .m_axi_rd_araddr        (rd_araddr),
    // ... full AXI AR/R

    // AXI Write Master
    .m_axi_wr_awid          (wr_awid),
    .m_axi_wr_awaddr        (wr_awaddr),
    // ... full AXI AW/W/B

    // MonBus
    .mon_valid              (stream_mon_valid),
    .mon_ready              (stream_mon_ready),
    .mon_packet             (stream_mon_packet)
);

## 6.10	Related Documentation

- Scheduler Group Array: 02_scheduler_group_array.md - Multi-channel scheduler integration

- Scheduler Group: 03_scheduler_group.md - Single channel scheduler + descriptor engine

- Scheduler: 04_scheduler.md - Channel state machine and transfer coordination

- Descriptor Engine: 05_descriptor_engine.md - Descriptor fetch and parsing

- AXI Read Engine: 06_axi_read_engine.md - Read datapath

- Stream Alloc Ctrl: 07_stream_alloc_ctrl.md - Space allocation controller

- SRAM Controller: 08_sram_controller.md - Per-channel buffering

- SRAM Controller Unit: 09_sram_controller_unit.md - Single channel SRAM unit

- Stream Latency Bridge: 10_stream_latency_bridge.md - Timing bridge

- Stream Drain Ctrl: 11_stream_drain_ctrl.md - Drain flow controller

- AXI Write Engine: 12_axi_write_engine.md - Write datapath

- APB to Descriptor: 13_apbtodescr.md - APB configuration interface

- Performance Profiler: 15_perf_profiler.md - Performance monitoring

## MonBus AXI-Lite Group: 16_monbus_axil_group.md - Monitor bus arbitration

## 6.11	Revision History

STREAM Core Specification Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Scheduler Group Array

Module: scheduler_group_array.sv Location: projects/components/stream/rtl/macro/ Category: MACRO (Top-level Integration) Parent: stream_core.sv Status: Implemented Last Updated: 2025-11-30

## 7.1	Overview

The scheduler_group_array module is the top-level array of 8 scheduler_group instances that provides multi-channel DMA with shared resources. It serves as the scheduler layer of the STREAM engine, handling descriptor fetch arbitration and routing data path interfaces.

### 7.1.1	Key Features

- 8 Independent Channels: Each channel has its own scheduler_group instance

- Shared Descriptor AXI Master: Round-robin arbitration for descriptor fetches

- Per-Channel Data Interfaces: Direct passthrough to AXI engines (no arbitration)

- Unified MonBus Aggregation: 9 sources (8 channels + descriptor AXI monitor)

- ID-Based R Channel Routing: Channel ID embedded in AXI ID for response demux

### 7.1.2	Simplified from RAPIDS

This module is intentionally simplified from RAPIDS for tutorial focus: - 8 channels (vs 32 in RAPIDS) - No control read/write engines - No program engine - No network interfaces (EOS completion) - No alignment bus - Direct data path interfaces (beats-based, no chunks)

## 7.2	Architecture

### 7.2.1	Component Hierarchy

scheduler_group_array
 scheduler_group[0..7]          # 8 channel instances
    scheduler                  # Channel FSM
    descriptor_engine          # Descriptor fetch
 arbiter_round_robin            # AR channel arbitration
 axi4_master_rd_mon             # Descriptor AXI monitor
 monbus_arbiter                 # 9-source MonBus aggregation

### 7.2.2	Descriptor AXI Arbitration

AR Channel Flow:

Channel 0..7 AR Requests -> Round-Robin Arbiter -> Single AR to External AXI
                        |
              Embed channel_id in AXI ID lower bits

R Channel Demux:

External R Response -> Extract channel_id from RID[CHAN_WIDTH-1:0] -> Route to correct channel

Critical: Channel ID is embedded in the lower bits of the AXI ID field:

// In AR mux
desc_axi_int_arid = {{(AXI_ID_WIDTH-CHAN_WIDTH){1'b0}}, ch[CHAN_WIDTH-1:0]};

// In R demux
desc_r_channel_id = desc_axi_int_rid[CHAN_WIDTH-1:0];

### 7.2.3	Data Path Interfaces

IMPORTANT: Data read and write interfaces are NOT arbitrated at this level!

- sched_rd_* - Per-channel arrays passed directly to axi_read_engine

- sched_wr_* - Per-channel arrays passed directly to axi_write_engine

The AXI engines themselves handle channel arbitration internally.

## 7.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | Number of DMA channels |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | Channel ID width (3 bits for 8 channels) |
| ADDR_WIDTH | int | 64 | Address bus width |
| DATA_WIDTH | int | 512 | Data bus width |
| AXI_ID_WIDTH | int | 8 | AXI transaction ID width |

### 7.3.1	Monitor Bus Agent IDs

Monitor Bus Agent IDs

| Parameter | Default | Description |
| --- | --- | --- |
| DESC_MON_BASE_AGENT_ID | 16 (0x10) | Descriptor engine base (16-23) |
| SCHED_MON_BASE_AGENT_ID | 48 (0x30) | Scheduler base (48-55) |
| DESC_AXI_MON_AGENT_ID | 8 (0x08) | Descriptor AXI monitor |
| MON_UNIT_ID | 1 | Unit ID for all STREAM events |
| MON_MAX_TRANSACTIONS | 16 | Max outstanding transactions for monitor |

## 7.4	Port List

### 7.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 7.4.2	APB Programming Interface (Per-Channel)

APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid[ch] | input | NUM_CHANNELS | Descriptor address valid |
| apb_ready[ch] | output | NUM_CHANNELS | Ready to accept descriptor |
| apb_addr[ch] | input | NUM_CHANNELS x ADDR_WIDTH | Descriptor address |

### 7.4.3	Configuration Interface (Per-Channel)

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable[ch] | input | NUM_CHANNELS | Enable channel |
| cfg_channel_reset[ch] | input | NUM_CHANNELS | Soft reset channel |

### 7.4.4	Global Scheduler Configuration

Global Scheduler Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_enable | input | 1 | Master scheduler enable |
| cfg_sched_timeout_cycles | input | 16 | Timeout threshold |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_sched_err_enable | input | 1 | Enable error reporting |
| cfg_sched_compl_enable | input | 1 | Enable completion reporting |
| cfg_sched_perf_enable | input | 1 | Enable performance monitoring |

### 7.4.5	Descriptor Engine Configuration

Descriptor Engine Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desceng_enable | input | 1 | Enable descriptor engine |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch |
| cfg_desceng_fifo_thresh | input | 4 | FIFO threshold for prefetch |
| cfg_desceng_addr0_base | input | ADDR_WIDTH | Address range 0 base |
| cfg_desceng_addr0_limit | input | ADDR_WIDTH | Address range 0 limit |
| cfg_desceng_addr1_base | input | ADDR_WIDTH | Address range 1 base |
| cfg_desceng_addr1_limit | input | ADDR_WIDTH | Address range 1 limit |

### 7.4.6	Status Interface (Per-Channel)

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle[ch] | output | NUM_CHANNELS | Descriptor engine idle |
| scheduler_idle[ch] | output | NUM_CHANNELS | Scheduler idle |
| scheduler_state[ch] | output | NUM_CHANNELS x 7 | Scheduler FSM state (ONE-HOT) |
| sched_error[ch] | output | NUM_CHANNELS | Scheduler error (sticky) |

### 7.4.7	Shared Descriptor AXI4 Master Read Interface

AR Channel:

Shared Descriptor AXI4 Master Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_axi_arvalid | output | 1 | Address valid |
| desc_axi_arready | input | 1 | Address ready |
| desc_axi_araddr | output | ADDR_WIDTH | Address |
| desc_axi_arlen | output | 8 | Burst length - 1 |
| desc_axi_arsize | output | 3 | Burst size (log2 bytes) |
| desc_axi_arburst | output | 2 | Burst type (INCR) |
| desc_axi_arid | output | AXI_ID_WIDTH | Transaction ID |
| desc_axi_arlock | output | 1 | Lock type |
| desc_axi_arcache | output | 4 | Cache attributes |
| desc_axi_arprot | output | 3 | Protection attributes |
| desc_axi_arqos | output | 4 | QoS value |
| desc_axi_arregion | output | 4 | Region identifier |

R Channel (FIXED 256-bit):

Shared Descriptor AXI4 Master Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_axi_rvalid | input | 1 | Read data valid |
| desc_axi_rready | output | 1 | Read data ready |
| desc_axi_rdata | input | 256 | Read data (FIXED 256-bit descriptor) |
| desc_axi_rresp | input | 2 | Response |
| desc_axi_rlast | input | 1 | Last beat of burst |
| desc_axi_rid | input | AXI_ID_WIDTH | Transaction ID |

### 7.4.8	Shared Data Read Interface (Per-Channel Arrays)

Shared Data Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid[ch] | output | NUM_CHANNELS | Read request valid |
| sched_rd_addr[ch] | output | NUM_CHANNELS x ADDR_WIDTH | Source address |
| sched_rd_beats[ch] | output | NUM_CHANNELS x 32 | Beats to read |
| sched_rd_done_strobe[ch] | input | NUM_CHANNELS | Read completion strobe |
| sched_rd_beats_done[ch] | input | NUM_CHANNELS x 32 | Beats completed |
| sched_rd_error[ch] | input | NUM_CHANNELS | Read error |

### 7.4.9	Shared Data Write Interface (Per-Channel Arrays)

Shared Data Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid[ch] | output | NUM_CHANNELS | Write request valid |
| sched_wr_ready[ch] | input | NUM_CHANNELS | Write ready |
| sched_wr_addr[ch] | output | NUM_CHANNELS x ADDR_WIDTH | Destination address |
| sched_wr_beats[ch] | output | NUM_CHANNELS x 32 | Beats to write |
| sched_wr_done_strobe[ch] | input | NUM_CHANNELS | Write completion strobe |
| sched_wr_beats_done[ch] | input | NUM_CHANNELS x 32 | Beats completed |
| sched_wr_error[ch] | input | NUM_CHANNELS | Write error |

### 7.4.10	Unified Monitor Bus Interface

Unified Monitor Bus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready |
| mon_packet | output | 64 | Monitor packet data |

## 7.5	Operation

### 7.5.1	Descriptor Fetch Arbitration

- Request Phase: Channels assert desc_ar_valid[ch] when needing descriptor fetch

- Grant Phase: Round-robin arbiter selects one channel

- AR Issue: Granted channel’s AR signals muxed to external interface

- ID Embedding: Channel ID placed in lower bits of arid

- Grant ACK: Arbiter acknowledged when arready received

- R Routing: Response routed to correct channel based on rid[CHAN_WIDTH-1:0]

### 7.5.2	Data Path Flow

Data interfaces pass directly through without arbitration:

scheduler_group[ch].sched_rd_* -> sched_rd_*[ch] -> axi_read_engine (handles arbitration)
scheduler_group[ch].sched_wr_* -> sched_wr_*[ch] -> axi_write_engine (handles arbitration)

### 7.5.3	MonBus Aggregation

9 sources aggregated via monbus_arbiter: - Sources 0-7: Per-channel mon_* from scheduler_groups - Source 8: Descriptor AXI master monitor

## 7.6	Integration Example

scheduler_group_array #(
    .NUM_CHANNELS           (8),
    .ADDR_WIDTH             (64),
    .DATA_WIDTH             (512),
    .AXI_ID_WIDTH           (8)
) u_scheduler_group_array (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // APB kick-off
    .apb_valid              (apb_valid),
    .apb_ready              (apb_ready),
    .apb_addr               (apb_addr),

    // Configuration
    .cfg_channel_enable     (cfg_channel_enable),
    .cfg_channel_reset      (cfg_channel_reset),
    .cfg_sched_enable       (cfg_sched_enable),
    // ... other config

    // Descriptor AXI master
    .desc_axi_arvalid       (desc_axi_arvalid),
    .desc_axi_arready       (desc_axi_arready),
    // ... full AR/R interface

    // Data read interface (to axi_read_engine)
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),
    .sched_rd_done_strobe   (rd_done_strobe),
    .sched_rd_beats_done    (rd_beats_done),

    // Data write interface (to axi_write_engine)
    .sched_wr_valid         (sched_wr_valid),
    .sched_wr_ready         (sched_wr_ready),
    .sched_wr_addr          (sched_wr_addr),
    .sched_wr_beats         (sched_wr_beats),
    .sched_wr_done_strobe   (wr_done_strobe),
    .sched_wr_beats_done    (wr_beats_done),

    // Monitor bus
    .mon_valid              (sga_mon_valid),
    .mon_ready              (sga_mon_ready),
    .mon_packet             (sga_mon_packet)
);

## 7.7	Related Documentation

- Parent: 01_stream_core.md - Top-level integration

- Child: 03_scheduler_group.md - Single channel scheduler + descriptor engine

- Scheduler: 04_scheduler.md - Channel state machine

- Descriptor Engine: 05_descriptor_engine.md - Descriptor fetch and parsing

- AXI Read Engine: 06_axi_read_engine.md - Read datapath (receives sched_rd_*)

- AXI Write Engine: 12_axi_write_engine.md - Write datapath (receives sched_wr_*)

## MonBus Arbiter: 16_monbus_axil_group.md - Monitor bus aggregation

## 7.8	Revision History

Scheduler Group Array Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Scheduler Group

Module: scheduler_group.sv Location: projects/components/stream/rtl/macro/ Category: MACRO (Channel Wrapper) Parent: scheduler_group_array.sv Status: Implemented Last Updated: 2025-11-30

## 8.1	Overview

The scheduler_group module is a wrapper combining the scheduler and descriptor_engine for a single STREAM channel. It provides a simplified interface compared to RAPIDS by removing unused features while maintaining the core descriptor-based DMA functionality.

### 8.1.1	Key Features

- Single Channel Wrapper: Combines scheduler + descriptor_engine

- Simplified from RAPIDS: No program engine, no control engines, no alignment bus

- MonBus Aggregation: Combines 2 internal sources (scheduler, descriptor_engine)

- Direct Data Interfaces: Per-channel read/write interfaces to shared engines

### 8.1.2	Simplified from RAPIDS

Removed from RAPIDS scheduler_group: - Program engine - Control read/write engines - Alignment bus - Network interfaces (EOS completion) - Credit management

## 8.2	Architecture

### 8.2.1	Component Hierarchy

scheduler_group
 scheduler                      # Channel state machine
 descriptor_engine              # Descriptor fetch and parsing
 arbiter_round_robin_monbus     # 2-source MonBus arbiter

### 8.2.2	Data Flow

APB kick-off -> descriptor_engine -> scheduler
                       |                  |

: Data Flow
                       v                  v
              Descriptor AXI        sched_rd_* / sched_wr_*
              (to arbiter)          (to shared engines)

Descriptor Flow: 1. APB kick-off provides descriptor address 2. Descriptor engine fetches via shared AXI (arbitrated externally) 3. Parsed descriptor delivered to scheduler 4. Scheduler coordinates read/write phases

Data Path Flow: 1. Scheduler asserts sched_rd_valid with source address/beats 2. External read engine issues AXI reads, returns completion strobe 3. Scheduler asserts sched_wr_valid with destination address/beats 4. External write engine issues AXI writes, returns completion strobe 5. Scheduler advances to next descriptor or completes

## 8.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| CHANNEL_ID | int | 0 | Channel identifier (0-7) |
| NUM_CHANNELS | int | 8 | Total number of channels |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | Channel ID width |
| ADDR_WIDTH | int | 64 | Address bus width |
| DATA_WIDTH | int | 512 | Data bus width |
| AXI_ID_WIDTH | int | 8 | AXI transaction ID width |

### 8.3.1	Monitor Bus Agent IDs

Monitor Bus Agent IDs

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| DESC_MON_AGENT_ID | 8-bit | 0x10+ch | Descriptor engine agent ID |
| SCHED_MON_AGENT_ID | 8-bit | 0x30+ch | Scheduler agent ID |
| MON_UNIT_ID | 4-bit | 0x1 | Unit ID |
| MON_CHANNEL_ID | 6-bit | ch | Channel ID in packets |

## 8.4	Port List

### 8.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 8.4.2	APB Programming Interface

APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Descriptor address valid |
| apb_ready | output | 1 | Ready to accept descriptor |
| apb_addr | input | ADDR_WIDTH | Descriptor address |

### 8.4.3	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable this channel |
| cfg_channel_reset | input | 1 | Soft reset this channel |

### 8.4.4	Scheduler Configuration

Scheduler Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_timeout_cycles | input | 16 | Timeout threshold |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |
| cfg_sched_err_enable | input | 1 | Enable error reporting |
| cfg_sched_compl_enable | input | 1 | Enable completion reporting |
| cfg_sched_perf_enable | input | 1 | Enable performance monitoring |

### 8.4.5	Descriptor Engine Configuration

Descriptor Engine Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desceng_prefetch | input | 1 | Enable descriptor prefetch |
| cfg_desceng_fifo_thresh | input | 4 | FIFO threshold for prefetch |
| cfg_desceng_addr0_base | input | ADDR_WIDTH | Address range 0 base |
| cfg_desceng_addr0_limit | input | ADDR_WIDTH | Address range 0 limit |
| cfg_desceng_addr1_base | input | ADDR_WIDTH | Address range 1 base |
| cfg_desceng_addr1_limit | input | ADDR_WIDTH | Address range 1 limit |

### 8.4.6	Status Interface

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle | output | 1 | Descriptor engine idle |
| scheduler_idle | output | 1 | Scheduler in IDLE state |
| scheduler_state | output | 7 | Scheduler FSM state (ONE-HOT) |
| sched_error | output | 1 | Scheduler error (sticky) |

### 8.4.7	Descriptor AXI Interface (to external arbiter)

AR Channel:

Descriptor AXI Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_ar_valid | output | 1 | Address valid |
| desc_ar_ready | input | 1 | Address ready |
| desc_ar_addr | output | ADDR_WIDTH | Address |
| desc_ar_len | output | 8 | Burst length - 1 |
| desc_ar_size | output | 3 | Burst size |
| desc_ar_burst | output | 2 | Burst type |
| desc_ar_id | output | AXI_ID_WIDTH | Transaction ID |
| desc_ar_lock | output | 1 | Lock type |
| desc_ar_cache | output | 4 | Cache attributes |
| desc_ar_prot | output | 3 | Protection attributes |
| desc_ar_qos | output | 4 | QoS value |
| desc_ar_region | output | 4 | Region identifier |

R Channel (256-bit fixed):

Descriptor AXI Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_r_valid | input | 1 | Read data valid |
| desc_r_ready | output | 1 | Read data ready |
| desc_r_data | input | 256 | Read data (256-bit descriptor) |
| desc_r_resp | input | 2 | Response |
| desc_r_last | input | 1 | Last beat |
| desc_r_id | input | AXI_ID_WIDTH | Transaction ID |

### 8.4.8	Data Read Interface (to shared read engine)

Data Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Read request valid |
| sched_rd_addr | output | ADDR_WIDTH | Source address |
| sched_rd_beats | output | 32 | Beats to read |
| sched_rd_done_strobe | input | 1 | Read completion strobe |
| sched_rd_beats_done | input | 32 | Beats completed |
| sched_rd_error | input | 1 | Read error |

### 8.4.9	Data Write Interface (to shared write engine)

Data Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | output | 1 | Write request valid |
| sched_wr_ready | input | 1 | Write ready |
| sched_wr_addr | output | ADDR_WIDTH | Destination address |
| sched_wr_beats | output | 32 | Beats to write |
| sched_wr_done_strobe | input | 1 | Write completion strobe |
| sched_wr_beats_done | input | 32 | Beats completed |
| sched_wr_error | input | 1 | Write error |

### 8.4.10	Monitor Bus Interface

Monitor Bus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready |
| mon_packet | output | 64 | Monitor packet data |

## 8.5	Operation

### 8.5.1	Transfer Sequence

- APB Kick-off: apb_valid with descriptor address

- Descriptor Fetch: descriptor_engine fetches via AXI

- Descriptor Parse: 256-bit descriptor parsed into fields

- Read Phase: scheduler asserts sched_rd_valid

- Read Completion: engine returns sched_rd_done_strobe

- Write Phase: scheduler asserts sched_wr_valid

- Write Completion: engine returns sched_wr_done_strobe

- Chain Check: if next_descriptor_ptr != 0, fetch next

- Complete: scheduler returns to IDLE

### 8.5.2	MonBus Sources

The scheduler_group aggregates 2 MonBus sources: 1. Descriptor Engine: Fetch events, errors 2. Scheduler: State transitions, completions, errors

## 8.6	Integration Example

scheduler_group #(
    .CHANNEL_ID             (ch),
    .NUM_CHANNELS           (8),
    .ADDR_WIDTH             (64),
    .DATA_WIDTH             (512),
    .AXI_ID_WIDTH           (8),
    .DESC_MON_AGENT_ID      (8'h10 + ch),
    .SCHED_MON_AGENT_ID     (8'h30 + ch)
) u_scheduler_group (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // APB interface
    .apb_valid              (apb_valid[ch]),
    .apb_ready              (apb_ready[ch]),
    .apb_addr               (apb_addr[ch]),

    // Configuration
    .cfg_channel_enable     (cfg_channel_enable[ch]),
    .cfg_channel_reset      (cfg_channel_reset[ch]),
    // ... other config

    // Descriptor AXI (to arbiter)
    .desc_ar_valid          (desc_ar_valid[ch]),
    .desc_ar_ready          (desc_ar_ready[ch]),
    // ... full AR/R interface

    // Data interfaces (direct passthrough)
    .sched_rd_valid         (sched_rd_valid[ch]),
    .sched_rd_addr          (sched_rd_addr[ch]),
    .sched_rd_beats         (sched_rd_beats[ch]),
    .sched_rd_done_strobe   (sched_rd_done_strobe[ch]),
    .sched_rd_beats_done    (sched_rd_beats_done[ch]),

    .sched_wr_valid         (sched_wr_valid[ch]),
    .sched_wr_ready         (sched_wr_ready[ch]),
    .sched_wr_addr          (sched_wr_addr[ch]),
    .sched_wr_beats         (sched_wr_beats[ch]),
    .sched_wr_done_strobe   (sched_wr_done_strobe[ch]),
    .sched_wr_beats_done    (sched_wr_beats_done[ch]),

    // Monitor bus
    .mon_valid              (mon_valid_ch[ch]),
    .mon_ready              (mon_ready_ch[ch]),
    .mon_packet             (mon_packet_ch[ch])
);

## 8.7	Related Documentation

- Parent: 02_scheduler_group_array.md - Multi-channel array

- Scheduler: 04_scheduler.md - Channel state machine details

- Descriptor Engine: 05_descriptor_engine.md - Descriptor fetch logic

- AXI Read Engine: 06_axi_read_engine.md - Shared read engine

## AXI Write Engine: 12_axi_write_engine.md - Shared write engine

## 8.8	Revision History

Scheduler Group Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Scheduler Specification

Module: scheduler.sv Location: projects/components/stream/rtl/fub/ Status: Implemented Last Updated: 2025-11-30

## 9.1	Overview

The Scheduler coordinates descriptor-based memory-to-memory DMA transfers for a single channel. It receives descriptors, manages concurrent read/write operations, and handles descriptor chaining.

### 9.1.1	Key Features

- Concurrent read/write: Read and write engines run simultaneously (prevents deadlock)

- Beat-based tracking: Length in data width units (STREAM simplification)

- Aligned addresses: No alignment fixup logic (must be pre-aligned)

- Descriptor chaining: Follows next_descriptor_ptr for multi-buffer transfers

- Interrupt generation: MonBus IRQ event when gen_irq flag set

- Error handling: Timeout detection, error aggregation from engines

- MonBus integration: State transition and IRQ event reporting

### 9.1.2	Block Diagram

### 9.1.3	Figure 2.4.1: Scheduler Block Diagram

Diagram

Source: 02_scheduler_block.mmd

## 9.2	CRITICAL: Concurrent Read/Write Design

Why Concurrent Operation is Essential:

The scheduler runs read and write engines CONCURRENTLY in CH_XFER_DATA state. This prevents deadlock when transfer size exceeds SRAM buffer capacity:

Example: 100MB transfer with 2KB SRAM buffer

Sequential operation (WRONG):
1. Read 100MB → DEADLOCK at 2KB (SRAM full, can't complete read)

Concurrent operation (CORRECT):
1. Read starts filling SRAM → SRAM becomes full (2KB)
2. Read pauses (natural backpressure)
3. Write drains SRAM → SRAM has free space
4. Read resumes → Both continue until 100MB complete

Implementation: - Both sched_rd_valid and sched_wr_valid asserted in CH_XFER_DATA - Independent beat counters: r_read_beats_remaining, r_write_beats_remaining - Exit when BOTH counters reach zero

## 9.3	Parameters

parameter int CHANNEL_ID = 0;                    // Channel identifier
parameter int NUM_CHANNELS = 8;                  // Total channels in system
parameter int CHAN_WIDTH = $clog2(NUM_CHANNELS); // Channel ID width
parameter int ADDR_WIDTH = 64;                   // Address bus width
parameter int DATA_WIDTH = 512;                  // Data bus width (beats)

// Monitor Bus Parameters
parameter logic [7:0] MON_AGENT_ID = 8'h40;      // STREAM Scheduler Agent ID
parameter logic [3:0] MON_UNIT_ID = 4'h1;        // Unit identifier
parameter logic [5:0] MON_CHANNEL_ID = 6'h0;     // Base channel ID

// Descriptor Width (FIXED at 256-bit for STREAM)
parameter int DESC_WIDTH = 256;

Validation:

// Scheduler only supports 256-bit STREAM descriptors
if (DESC_WIDTH != 256)
    $fatal("DESC_WIDTH must be 256 for STREAM scheduler");

## 9.4	Port List

### 9.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 9.4.2	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | input | 1 | Enable this channel |
| cfg_channel_reset | input | 1 | Channel soft reset (FSM → IDLE) |
| cfg_sched_timeout_cycles | input | 16 | Timeout threshold in clock cycles (runtime config) |
| cfg_sched_timeout_enable | input | 1 | Enable timeout detection |

### 9.4.3	Status Interface

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| scheduler_idle | output | 1 | Scheduler idle flag |
| scheduler_state | output | 7 | Current FSM state (one-hot encoding) |

### 9.4.4	Descriptor Engine Interface

Descriptor Engine Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_valid | input | 1 | Descriptor valid from descriptor engine |
| descriptor_ready | output | 1 | Scheduler ready to accept descriptor |
| descriptor_packet | input | 256 | 256-bit STREAM descriptor (see format below) |
| descriptor_error | input | 1 | Error signal from descriptor engine |

### 9.4.5	Data Read Interface

To AXI Read Engine:

Data Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid | output | 1 | Channel requests read |
| sched_rd_addr | output | ADDR_WIDTH | Source address (aligned, static during burst) |
| sched_rd_beats | output | 32 | Beats remaining to read |

Completion from Read Engine:

Data Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_done_strobe | input | 1 | Read burst completed (1-cycle pulse) |
| sched_rd_beats_done | input | 32 | Number of beats completed in burst |
| sched_rd_error | input | 1 | Read engine error (sticky) |

### 9.4.6	Data Write Interface

To AXI Write Engine:

Data Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid | output | 1 | Channel requests write |
| sched_wr_ready | input | 1 | Engine ready for channel (completion handshake) |
| sched_wr_addr | output | ADDR_WIDTH | Destination address (aligned, static during burst) |
| sched_wr_beats | output | 32 | Beats remaining to write |

Completion from Write Engine:

Data Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_done_strobe | input | 1 | Write burst completed (1-cycle pulse) |
| sched_wr_beats_done | input | 32 | Number of beats completed in burst |
| sched_wr_error | input | 1 | Write engine error (sticky) |

### 9.4.7	Error Signals

Error Signals

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_error | output | 1 | Scheduler error output (aggregates rd/wr errors, sticky) |

### 9.4.8	Monitor Bus Interface

Monitor Bus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor bus ready |
| mon_packet | output | 64 | 64-bit monitor bus packet |

## 9.5	Interface

### 9.5.1	Clock and Reset

input  logic                        clk;
input  logic                        rst_n;      // Active-low asynchronous reset

### 9.5.2	Configuration Interface

input  logic                        cfg_channel_enable;       // Enable this channel
input  logic                        cfg_channel_reset;        // Channel reset (soft reset)
input  logic [15:0]                 cfg_sched_timeout_cycles; // Timeout threshold (runtime config)
input  logic                        cfg_sched_timeout_enable; // Enable timeout detection

Channel Reset Behavior: - cfg_channel_reset forces FSM to CH_IDLE immediately - Clears descriptor_loaded flag - Resets beat counters - Independent of global rst_n

Timeout Configuration: - Runtime configurable via cfg_sched_timeout_cycles (replaces compile-time TIMEOUT_CYCLES parameter) - Can be disabled/enabled dynamically via cfg_sched_timeout_enable

### 9.5.3	Status Interface

output logic                        scheduler_idle;         // Scheduler in CH_IDLE
output logic [6:0]                  scheduler_state;        // Current state (ONE-HOT)

State Encoding (ONE-HOT): - [0] = CH_IDLE - [1] = CH_FETCH_DESC - [2] = CH_XFER_DATA - [3] = CH_COMPLETE - [4] = CH_NEXT_DESC - [5] = CH_ERROR - [6] = Reserved

### 9.5.4	Descriptor Engine Interface

input  logic                        descriptor_valid;
output logic                        descriptor_ready;
input  logic [DESC_WIDTH-1:0]       descriptor_packet;     // 256-bit STREAM descriptor
input  logic                        descriptor_error;       // Error from descriptor engine

Descriptor Handshake: - descriptor_ready asserted in CH_IDLE or CH_NEXT_DESC - Descriptor captured when valid && ready - Supports descriptor chaining (next_descriptor_ptr)

### 9.5.5	Data Read Interface (to AXI Read Engine)

Request:

output logic                        sched_rd_valid;         // Request read access
output logic [ADDR_WIDTH-1:0]       sched_rd_addr;          // Source address (static base)
output logic [31:0]                 sched_rd_beats;         // Total beats to read

Completion:

input  logic                        sched_rd_done_strobe;   // Read engine completed beats
input  logic [31:0]                 sched_rd_beats_done;    // Number of beats completed

Error:

input  logic                        sched_rd_error;         // Read engine error

Address Management: - Scheduler provides static base address in sched_rd_addr - Read engine handles address increment internally - Scheduler does NOT update sched_rd_addr after each burst

Interface Timing Notes (November 2025 Updates): - Scheduler holds sched_rd_valid high while sched_rd_beats > 0 - Engine reports completion via sched_rd_done_strobe (pulsed) - Scheduler decrements r_read_beats_remaining by sched_rd_beats_done - Process repeats until r_read_beats_remaining == 0

### 9.5.6	Data Write Interface (to AXI Write Engine)

Request:

output logic                        sched_wr_valid;         // Request write access
input  logic                        sched_wr_ready;         // Engine grants access
output logic [ADDR_WIDTH-1:0]       sched_wr_addr;          // Destination address (static base)
output logic [31:0]                 sched_wr_beats;         // Total beats to write

Completion:

input  logic                        sched_wr_done_strobe;   // Write engine completed beats
input  logic [31:0]                 sched_wr_beats_done;    // Number of beats completed

Error:

input  logic                        sched_wr_error;         // Write engine error

Interface Timing Notes (November 2025 Updates):

sched_wr_ready Timing: - REGISTERED OUTPUT from write engine - Not combinatorial from completion signals - Asserts 1 cycle after channel becomes ready to accept new request - Cleared 1 cycle after handshake (valid && ready) - Do not expect immediate deassertion on handshake - takes 1 cycle

Example Timing:

Cycle  sched_wr_valid  sched_wr_ready  sched_wr_done_strobe  Notes
-----  --------------  --------------  --------------------  -----
  0    0               0               0                     Idle
  1    1               0               0                     Request (not ready yet)
  2    1               1               0                     Handshake!
  3    1               0               0                     Ready cleared (registered)
 ...   (W-phase executes)
 50    1               0               1                     B response, done strobe
 51    1               1               0                     Ready re-asserts (next cycle)

### 9.5.7	Monitor Bus Interface

output logic                        mon_valid;
input  logic                        mon_ready;
output logic [63:0]                 mon_packet;

MonBus Events Generated: - State transitions (IDLE → FETCH_DESC, etc.) - IRQ event (when descriptor.gen_irq set) - Error events

## 9.6	Descriptor Format

### 9.6.1	STREAM Descriptor (256-bit)

typedef struct packed {
    logic [47:0]  reserved;              // [255:208] Reserved
    logic [7:0]   desc_priority;         // [207:200] Transfer priority
    logic [3:0]   channel_id;            // [199:196] Channel ID (informational)
    logic         error;                 // [195] Error flag
    logic         last;                  // [194] Last in chain flag
    logic         gen_irq;               // [193] Generate interrupt on completion
    logic         valid;                 // [192] Valid descriptor
    logic [31:0]  next_descriptor_ptr;   // [191:160] Next descriptor address (0 = last)
    logic [31:0]  length;                // [159:128] Length in BEATS
    logic [63:0]  dst_addr;              // [127:64] Destination address (aligned)
    logic [63:0]  src_addr;              // [63:0] Source address (aligned)
} descriptor_t;

Field Constraints: - src_addr / dst_addr: Must be aligned to DATA_WIDTH (e.g., 64-byte aligned for 512-bit data) - length: Transfer size in BEATS (not bytes or chunks) - next_descriptor_ptr: 0 or address of next descriptor - valid: Must be 1 for descriptor to be accepted - last: Terminates chain (overrides next_descriptor_ptr) - gen_irq: Generates STREAM_EVENT_IRQ via MonBus when transfer completes

Descriptor Bit Positions:

DESC_SRC_ADDR:  [63:0]
DESC_DST_ADDR:  [127:64]
DESC_LENGTH:    [159:128]
DESC_NEXT_PTR:  [191:160]
DESC_VALID_BIT: [192]
DESC_GEN_IRQ:   [193]
DESC_LAST:      [194]

## 9.7	FSM Operation

### 9.7.1	State Machine

States (ONE-HOT encoded):

CH_IDLE        - Waiting for descriptor
CH_FETCH_DESC  - Latch and validate descriptor
CH_XFER_DATA   - Concurrent read/write transfer
CH_COMPLETE    - Transfer done, check chaining
CH_NEXT_DESC   - Fetch next chained descriptor
CH_ERROR       - Error condition

FSM Flow:

### 9.7.2	Figure 2.4.2: Scheduler FSM

Diagram

Source: 02_scheduler_block.mmd

### 9.7.3	State Transitions

CH_IDLE: - Wait for: descriptor_valid && cfg_channel_enable - Action: Assert descriptor_ready - Next: CH_FETCH_DESC (when handshake occurs)

CH_FETCH_DESC: - Action: - Latch descriptor fields into r_descriptor - Initialize working registers (r_src_addr, r_dst_addr, r_*_beats_remaining) - Validate descriptor.valid bit - Next: - CH_XFER_DATA (if valid) - CH_ERROR (if invalid)

CH_XFER_DATA: - Action: - Assert BOTH sched_rd_valid and sched_wr_valid (concurrent operation!) - Decrement r_read_beats_remaining on sched_rd_done_strobe - Decrement r_write_beats_remaining on sched_wr_done_strobe - Monitor timeout counter - Exit When: r_read_beats_remaining == 0 && r_write_beats_remaining == 0 - Next: CH_COMPLETE

CH_COMPLETE: - Action: - Generate MonBus IRQ event (if gen_irq set) - Check next_descriptor_ptr and last flag - Clear descriptor_loaded flag - Next: - CH_NEXT_DESC (if chaining) - CH_IDLE (if last or no chain)

CH_NEXT_DESC: - Wait for: descriptor_valid (descriptor engine fetches next) - Action: Assert descriptor_ready - Next: CH_FETCH_DESC

CH_ERROR: - STICKY STATE: Once in error, stays here until channel reset - Action: Report error via MonBus - Recovery: Requires cfg_channel_reset assertion (or global reset) - Note: scheduler_idle asserts in this state (allows external monitoring)

## 9.8	Beat Tracking

### 9.8.1	Independent Counters

Initialization (CH_FETCH_DESC):

r_read_beats_remaining  <= r_descriptor.length;
r_write_beats_remaining <= r_descriptor.length;

Decrement (CH_XFER_DATA):

// Read progress (independent)
if (sched_rd_done_strobe) begin
    r_read_beats_remaining <= (r_read_beats_remaining >= sched_rd_beats_done) ?
                             (r_read_beats_remaining - sched_rd_beats_done) : 32'h0;
end

// Write progress (independent)
if (sched_wr_done_strobe) begin
    r_write_beats_remaining <= (r_write_beats_remaining >= sched_wr_beats_done) ?
                              (r_write_beats_remaining - sched_wr_beats_done) : 32'h0;
end

Saturation: - Counters saturate at 0 (prevent underflow) - Safety check for engine misbehavior

### 9.8.2	Completion Detection

Combinational Flags:

w_read_complete     = (r_read_beats_remaining == 0);
w_write_complete    = (r_write_beats_remaining == 0);
w_transfer_complete = w_read_complete && w_write_complete;

State Exit:

// In CH_XFER_DATA:
if (w_transfer_complete) begin
    w_next_state = CH_COMPLETE;
end

### 9.8.3	Multiple Requests per Descriptor

For large transfers, scheduler issues multiple requests as engines complete work:

Descriptor: length = 256 beats (16KB @ 512-bit data)
Engine burst size: 16 beats (configured via cfg_axi_wr_xfer_beats)

Request sequence:
  Request 1: sched_wr_valid=1, sched_wr_beats=256, ready → handshake
             Engine executes 16-beat burst
             sched_wr_done_strobe=1, sched_wr_beats_done=16
             Scheduler updates: 256 - 16 = 240 beats remaining

  Request 2: sched_wr_valid=1, sched_wr_beats=240, ready → handshake
             Engine executes 16-beat burst
             sched_wr_done_strobe=1, sched_wr_beats_done=16
             Scheduler updates: 240 - 16 = 224 beats remaining

  ... (continues for 16 requests total)

  Request 16: sched_wr_valid=1, sched_wr_beats=16, ready → handshake
              Engine executes 16-beat burst
              sched_wr_done_strobe=1, sched_wr_beats_done=16
              Scheduler updates: 16 - 16 = 0 beats remaining → COMPLETE

Key Point: Scheduler holds sched_wr_valid high and keeps reissuing requests as long as sched_wr_beats > 0. Engines handle each request independently.

## 9.9	Address Management

### 9.9.1	Static Base Address

Scheduler Provides:

assign sched_rd_addr = r_src_addr;  // Static base, set in CH_FETCH_DESC
assign sched_wr_addr = r_dst_addr;  // Static base, set in CH_FETCH_DESC

Scheduler Does NOT: - Increment addresses after each burst - Calculate byte offsets - Handle alignment

Engine Responsibility: - Read engine: m_axi_araddr = sched_rd_addr + (beats_issued << AXSIZE) - Write engine: m_axi_awaddr = sched_wr_addr + (beats_issued << AXSIZE)

## 9.10	Timeout Detection

### 9.10.1	Timeout Counter

Configuration: - Runtime configurable via cfg_sched_timeout_cycles (16-bit) - Can be enabled/disabled via cfg_sched_timeout_enable - Replaces compile-time TIMEOUT_CYCLES parameter

Increment:

// In CH_XFER_DATA when waiting for engines
if (cfg_sched_timeout_enable && (!sched_rd_ready || !sched_wr_ready)) begin
    r_timeout_counter <= r_timeout_counter + 1;
end

Timeout Flag:

assign w_timeout_expired = (r_timeout_counter >= cfg_sched_timeout_cycles);

Reset:

// Clear when state changes or engines respond
if (state_change || (sched_rd_ready && sched_wr_ready)) begin
    r_timeout_counter <= 0;
end

Action on Timeout: - FSM transitions to CH_ERROR - MonBus timeout event generated - Channel must be reset to recover

## 9.11	Error Handling

### 9.11.1	Error Sources

External: - descriptor_error - Descriptor engine reports error - sched_rd_error - Read engine error (AXI RRESP != OKAY, etc.) - sched_wr_error - Write engine error (AXI BRESP != OKAY, etc.)

Internal: - w_timeout_expired - Timeout counter exceeded threshold - !r_descriptor.valid - Invalid descriptor in CH_FETCH_DESC

### 9.11.2	Sticky Error Flags

logic r_read_error_sticky;   // Set on sched_rd_error, cleared in CH_IDLE
logic r_write_error_sticky;  // Set on sched_wr_error, cleared in CH_IDLE
logic r_descriptor_error;    // Set on descriptor_error or validation failure

Set Condition:

if (sched_rd_error)
    r_read_error_sticky <= 1'b1;

if (sched_wr_error)
    r_write_error_sticky <= 1'b1;

Clear Condition:

if (r_current_state == CH_IDLE)
    r_*_error_sticky <= 1'b0;

### 9.11.3	Error Recovery

Error Transition:

// Any state with error condition
if (descriptor_error || sched_rd_error || sched_wr_error ||
    r_read_error_sticky || r_write_error_sticky || w_timeout_expired) begin
    w_next_state = CH_ERROR;
end

CH_ERROR is STICKY:

CH_ERROR: begin
    // Error state - STICKY, stay here until reset
    // Once in error, only way out is through reset
    w_next_state = CH_ERROR;
end

Recovery: - CH_ERROR is a sticky state - does NOT auto-recover - Software must assert cfg_channel_reset (or global rst_n) - On channel reset: FSM → CH_IDLE, sticky flags cleared

## 9.12	Interrupt Generation

### 9.12.1	IRQ via MonBus

Trigger: - Descriptor completes (CH_COMPLETE state) - r_descriptor.gen_irq flag set

MonBus Event:

// In CH_COMPLETE state with gen_irq set
mon_packet = {
    MON_AGENT_ID,           // [63:56] Agent ID (0x40 = STREAM Scheduler)
    MON_UNIT_ID,            // [55:52] Unit ID
    MON_CHANNEL_ID,         // [51:46] Channel ID
    STREAM_EVENT_IRQ,       // [45:40] Event code (IRQ)
    descriptor_fields       // [39:0]  Descriptor info
};

No Separate IRQ Signal: - IRQ communicated via MonBus only - Software monitors MonBus for IRQ events - Event includes channel ID for routing

## 9.13	Descriptor Chaining

### 9.13.1	Chain Detection

In CH_COMPLETE:

if (r_descriptor.next_descriptor_ptr != 32'h0 && !r_descriptor.last) begin
    w_next_state = CH_NEXT_DESC;  // Chain to next descriptor
end else begin
    w_next_state = CH_IDLE;       // Complete (last or no chain)
end

### 9.13.2	Chain Termination

Explicit Termination: - next_descriptor_ptr == 0 → Stop - last == 1 → Stop (overrides next_descriptor_ptr)

Example Chain:

Descriptor 0 @ 0x1000:
  src_addr = 0x2000, dst_addr = 0x3000, length = 64
  next_descriptor_ptr = 0x1040, last = 0
  → Chains to next

Descriptor 1 @ 0x1040:
  src_addr = 0x2100, dst_addr = 0x3100, length = 32
  next_descriptor_ptr = 0x0000, last = 1
  → Last in chain

## 9.14	MonBus Integration

### 9.14.1	Event Types

State Transitions: - IDLE → FETCH_DESC: Descriptor fetch start - FETCH_DESC → XFER_DATA: Transfer start - XFER_DATA → COMPLETE: Transfer complete - COMPLETE → NEXT_DESC: Chain fetch - Any → ERROR: Error occurred

Special Events: - STREAM_EVENT_IRQ: Interrupt generation (gen_irq flag) - STREAM_EVENT_TIMEOUT: Timeout expired - STREAM_EVENT_ERROR: Error condition

### 9.14.2	MonBus Packet Format

Generic Event:

[63:56] - MON_AGENT_ID (0x40)
[55:52] - MON_UNIT_ID
[51:46] - MON_CHANNEL_ID + CHANNEL_ID
[45:40] - Event code
[39:0]  - Event-specific data

## 9.15	Timing Diagrams

### 9.15.1	Normal Transfer (No Chaining)

### 9.15.2	Figure 2.4.3: Scheduler Normal Transfer Timing

Scheduler Normal Transfer Timing

Source: 04_scheduler_normal_transfer.mmd

Notes: - Both read and write run concurrently in XFER state - Independent done strobes decrement separate counters

### 9.15.3	Descriptor Chaining

### 9.15.4	Figure 2.4.4: Scheduler Descriptor Chaining Timing

Scheduler Descriptor Chaining Timing

Source: 04_scheduler_chaining.mmd

### 9.15.5	Single-Descriptor Transfer (Detailed)

Cycle  State        sched_wr_valid  sched_wr_ready  sched_wr_beats  Notes
-----  -----------  --------------  --------------  --------------  -----
  0    IDLE         0               0               0               Waiting
  1    FETCH_DESC   0               0               0               Request descriptor
  2    FETCH_DESC   0               0               0               (fetch latency)
  3    XFER_DATA    0               0               0               Descriptor received
 ...   (read phase)
 50    XFER_DATA    1               0               256             Assert write request
 51    XFER_DATA    1               1               256             Engine ready, handshake!
 52    XFER_DATA    1               0               256             Ready cleared (registered)
 ...   (engine executes burst)
100    XFER_DATA    1               0               256             done_strobe=1, beats_done=16
101    XFER_DATA    1               1               240             Ready re-asserts, new beats_remaining
102    XFER_DATA    1               0               240             Handshake again
...    (continues until beats_remaining == 0)

### 9.15.6	Chained Descriptors (Detailed)

Descriptor chain:
  Desc 0: length=128, next_ptr=0x1000_0100
  Desc 1: length=64,  next_ptr=0

Cycle  State        Descriptor  sched_wr_beats  Notes
-----  -----------  ----------  --------------  -----
  0    IDLE         -           0               Start
  1    FETCH_DESC   Req @0x1000 0               Fetch first
  3    XFER_DATA    Desc 0      128             Transfer desc 0
 ...   (8 bursts × 16 beats)
200    COMPLETE     Desc 0      0               Transfer done
201    NEXT_DESC    Check ptr   0               next_ptr = 0x1000_0100
202    FETCH_DESC   Req @0x1100 0               Fetch second
204    XFER_DATA    Desc 1      64              Transfer desc 1
 ...   (4 bursts × 16 beats)
400    COMPLETE     Desc 1      0               Transfer done
401    NEXT_DESC    Check ptr   0               next_ptr = 0 → terminate
402    IDLE         -           0               Chain complete

## 9.16	Testing

Test Location: projects/components/stream/dv/tests/fub_tests/scheduler/

Key Test Scenarios:

- Single descriptor transfer - Basic operation

- Descriptor chaining - 2-4 descriptors linked

- Concurrent read/write - Verify no deadlock with small SRAM

- Large transfer (> SRAM) - 100MB transfer with 2KB SRAM

- IRQ generation - gen_irq flag set

- Error handling - Descriptor, read, write errors

- Timeout detection - Engine stall scenarios

- Channel reset - cfg_channel_reset during transfer

- Runtime timeout config - Change cfg_sched_timeout_cycles dynamically

- sched_rd_ready / sched_wr_ready timing validation - Verify registered ready behavior

## 9.17	Performance Considerations

### 9.17.1	Concurrent Operation Benefit

Without Concurrency (Sequential): - Max transfer size = SRAM buffer size - Deadlock when transfer > buffer - Throughput = min(read_bw, write_bw)

With Concurrency: - No transfer size limit - Natural flow control via SRAM full/empty - Throughput = max(read_bw, write_bw) (pipeline overlap)

### 9.17.2	Example Performance

Configuration: - DATA_WIDTH = 512 bits (64 bytes/beat) - SRAM = 2KB (32 beats) - Transfer = 100MB (1,562,500 beats)

Sequential (hypothetical): - DEADLOCK at 2KB (can’t complete read)

Concurrent: - Read fills SRAM (32 beats) - Write drains SRAM concurrently - Both engines sustain ~0.9 beats/cycle - Total time: ~1.7M cycles

## 9.18	Related Documentation

- Descriptor Engine: 05_descriptor_engine.md - Descriptor fetch

- AXI Read Engine: 06_axi_read_engine.md - Source data read

- AXI Write Engine: 12_axi_write_engine.md - Destination data write

- SRAM Controller: 08_sram_controller.md - Buffer management

- Scheduler Group: 03_scheduler_group.md - Single-channel integration

- Scheduler Group Array: 02_scheduler_group_array.md - Multi-channel integration

## 9.19	Revision History

Revision History

| Date | Version | Changes |
| --- | --- | --- |
| 2025-10-17 | 1.0 | Initial documentation with old signal names (datard_, datawr_) |
| 2025-11-16 | 1.5 | Enhanced documentation with detailed sections |
| 2025-11-21 | 2.0 | Merged documentation:- Updated all signal names (sched_rd_, sched_wr_)- Added runtime timeout configuration (cfg_sched_timeout_cycles/enable)- Registered ready signal timing clarification- Added multiple requests per descriptor section- Enhanced beat tracking and error handling details- Updated all code examples and timing diagrams- Added timing examples for chained descriptors- Combined best content from multiple documentation sources |
| 2025-11-30 | 2.1 | RTL Sync Update:- CH_ERROR is now STICKY (requires reset to recover)- scheduler_idle asserts in CH_ERROR state- Updated related documentation references |

Last Updated: 2025-11-30 (matched to current RTL implementation)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Descriptor Engine

Module: descriptor_engine.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: scheduler_group.sv Status: Implemented Last Updated: 2025-11-30

## 10.1	Overview

The descriptor_engine module is an autonomous descriptor fetch engine with chaining support. It fetches 256-bit STREAM descriptors from memory via AXI4 master interface and provides two operating modes for kick-off.

### 10.1.1	Key Features

- Two Operating Modes:

  - APB-initiated: Software writes descriptor address, engine fetches

  - Autonomous chaining: Engine automatically fetches next_descriptor_ptr

- Two-FIFO Architecture:

  - Descriptor address FIFO: Holds addresses to fetch

  - Descriptor data FIFO: Buffers fetched descriptors for scheduler

- Address Range Validation: Two configurable ranges for security

- MonBus Event Reporting: Fetch complete and error events

## 10.2	Architecture

### 10.2.1	Operating Flow

APB Mode:

APB write -> skid buffer -> desc addr FIFO -> AXI fetch -> desc data FIFO -> scheduler

Chaining Mode:

Descriptor complete -> extract next_ptr -> validate -> desc addr FIFO ->
AXI fetch -> desc data FIFO -> scheduler (repeat until last=1 or next_ptr=0)

### 10.2.2	FSM States

typedef enum logic [2:0] {
    RD_IDLE       = 3'b000,  // Waiting for descriptor address
    RD_ISSUE_ADDR = 3'b001,  // Issue AXI AR transaction
    RD_WAIT_DATA  = 3'b010,  // Wait for AXI R response
    RD_COMPLETE   = 3'b011,  // Descriptor fetched, push to FIFO
    RD_ERROR      = 3'b100   // Error occurred
} read_engine_state_t;

### 10.2.3	Descriptor Format (256-bit STREAM)

Descriptor Format

| Bits | Field | Description |
| --- | --- | --- |
| [63:0] | src_addr | Source memory address (64-bit, must be aligned) |
| [127:64] | dst_addr | Destination memory address (64-bit, must be aligned) |
| [159:128] | length | Transfer length in BEATS (not bytes!) |
| [191:160] | next_descriptor_ptr | Address of next descriptor (32-bit, 0 = last) |
| [192] | valid | Descriptor valid flag |
| [193] | gen_irq | Generate interrupt on completion |
| [194] | last | Last descriptor in chain (explicit termination) |
| [195] | error | Descriptor error flag |
| [199:196] | channel_id | Channel identifier |
| [207:200] | priority | Descriptor priority |
| [255:208] | reserved | Reserved for future use |

## 10.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| CHANNEL_ID | int | 0 | Channel identifier |
| NUM_CHANNELS | int | 32 | Total number of channels |
| CHAN_WIDTH | int | $clog2(NUM_CHANNELS) | Channel ID width |
| ADDR_WIDTH | int | 64 | Address bus width |
| AXI_ID_WIDTH | int | 8 | AXI transaction ID width |
| FIFO_DEPTH | int | 8 | Descriptor data FIFO depth |
| DESC_ADDR_FIFO_DEPTH | int | 2 | Descriptor address FIFO depth |
| TIMEOUT_CYCLES | int | 1000 | AXI timeout threshold |

### 10.3.1	Monitor Bus Parameters

Monitor Bus Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| MON_AGENT_ID | 8-bit | 0x10 | Descriptor Engine Agent ID |
| MON_UNIT_ID | 4-bit | 0x1 | Unit identifier |
| MON_CHANNEL_ID | 6-bit | 0x0 | Base channel ID |

## 10.4	Port List

### 10.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 10.4.2	APB Programming Interface

APB Programming Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_valid | input | 1 | Descriptor address valid |
| apb_ready | output | 1 | Ready to accept address |
| apb_addr | input | ADDR_WIDTH | Descriptor address |

### 10.4.3	Scheduler Interface

Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| channel_idle | input | 1 | Scheduler idle (enables APB) |
| descriptor_valid | output | 1 | Descriptor available |
| descriptor_ready | input | 1 | Scheduler ready to accept |
| descriptor_packet | output | 256 | FIXED 256-bit descriptor |
| descriptor_error | output | 1 | Fetch error flag |

### 10.4.4	Enhanced Control Outputs

Enhanced Control Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_eos | output | 1 | End of Stream (future use) |
| descriptor_eol | output | 1 | End of Line (future use) |
| descriptor_eod | output | 1 | End of Data (future use) |
| descriptor_type | output | 2 | Packet type (future use) |

### 10.4.5	AXI4 AR Channel

AXI4 AR Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ar_valid | output | 1 | Address valid |
| ar_ready | input | 1 | Address ready |
| ar_addr | output | ADDR_WIDTH | Address |
| ar_len | output | 8 | Burst length - 1 |
| ar_size | output | 3 | Burst size (log2 bytes) |
| ar_burst | output | 2 | Burst type (INCR) |
| ar_id | output | AXI_ID_WIDTH | Transaction ID |
| ar_lock | output | 1 | Lock type |
| ar_cache | output | 4 | Cache attributes |
| ar_prot | output | 3 | Protection attributes |
| ar_qos | output | 4 | QoS value |
| ar_region | output | 4 | Region identifier |

### 10.4.6	AXI4 R Channel (FIXED 256-bit)

AXI4 R Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| r_valid | input | 1 | Read data valid |
| r_ready | output | 1 | Read data ready |
| r_data | input | 256 | Read data (FIXED 256-bit) |
| r_resp | input | 2 | Response |
| r_last | input | 1 | Last beat |
| r_id | input | AXI_ID_WIDTH | Transaction ID |

### 10.4.7	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_prefetch_enable | input | 1 | Enable descriptor prefetch |
| cfg_fifo_threshold | input | 4 | FIFO threshold for prefetch |
| cfg_addr0_base | input | ADDR_WIDTH | Address range 0 base |
| cfg_addr0_limit | input | ADDR_WIDTH | Address range 0 limit |
| cfg_addr1_base | input | ADDR_WIDTH | Address range 1 base |
| cfg_addr1_limit | input | ADDR_WIDTH | Address range 1 limit |
| cfg_channel_reset | input | 1 | Channel reset |

### 10.4.8	Status Interface

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| descriptor_engine_idle | output | 1 | Engine idle |

### 10.4.9	Monitor Bus Interface

Monitor Bus Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor packet ready |
| mon_packet | output | 64 | Monitor packet data |

## 10.5	Operation

### 10.5.1	APB Acceptance Policy

APB writes are accepted ONLY when ALL conditions met: 1. !r_channel_reset_active - Not in channel reset 2. w_desc_addr_fifo_empty - No pending descriptor fetches 3. channel_idle - Scheduler completed all prior descriptors 4. !r_apb_ip - No APB transaction currently in progress 5. apb_addr != 0 - Address 0 is invalid

### 10.5.2	Autonomous Chaining Decision Tree

Level 1: Basic Eligibility (w_chain_condition) - next_descriptor_ptr != 0 (non-null pointer) - descriptor.last == 0 (not explicitly marked as last) - descriptor.valid == 1 (valid descriptor)

Level 2: Address Validation (w_next_addr_valid) - next_addr within cfg_addr0_base..cfg_addr0_limit OR - next_addr within cfg_addr1_base..cfg_addr1_limit

Level 3: Final Decision (w_should_chain) - Level 1 + Level 2 conditions met - !r_descriptor_error (no fetch error) - w_desc_fifo_wr_ready (descriptor FIFO has space)

### 10.5.3	AXI Transaction

ar_len = 8'h00;           // Single beat transfer
ar_size = 3'b110;         // 64 bytes (512-bit for 256-bit descriptor)
ar_burst = 2'b01;         // INCR burst type
ar_id = {{padding}, CHANNEL_ID};  // Channel ID in lower bits
ar_cache = 4'b0010;       // Normal non-cacheable bufferable
ar_prot = 3'b000;         // Data, secure, unprivileged

## 10.6	Timing Diagrams

### 10.6.1	APB Kick-off Sequence

The following diagram shows the APB-initiated descriptor kick-off sequence:

#### 10.6.1.1	Waveform 2.5.1: Descriptor Engine APB Basic Kick-off

Descriptor Engine APB Basic

Source: descriptor_engine_apb_basic.json

## 10.7	Integration Example

descriptor_engine #(
    .CHANNEL_ID         (ch),
    .NUM_CHANNELS       (8),
    .ADDR_WIDTH         (64),
    .AXI_ID_WIDTH       (8),
    .FIFO_DEPTH         (8),
    .MON_AGENT_ID       (8'h10 + ch)
) u_descriptor_engine (
    .clk                (clk),
    .rst_n              (rst_n),

    // APB interface
    .apb_valid          (apb_valid),
    .apb_ready          (apb_ready),
    .apb_addr           (apb_addr),

    // Scheduler interface
    .channel_idle       (scheduler_idle),
    .descriptor_valid   (desc_valid),
    .descriptor_ready   (desc_ready),
    .descriptor_packet  (desc_packet),
    .descriptor_error   (desc_error),

    // AXI interface
    .ar_valid           (ar_valid),
    .ar_ready           (ar_ready),
    .ar_addr            (ar_addr),
    // ... full AR/R interface

    // Configuration
    .cfg_prefetch_enable    (cfg_desceng_prefetch),
    .cfg_addr0_base         (cfg_desceng_addr0_base),
    .cfg_addr0_limit        (cfg_desceng_addr0_limit),
    .cfg_addr1_base         (cfg_desceng_addr1_base),
    .cfg_addr1_limit        (cfg_desceng_addr1_limit),
    .cfg_channel_reset      (cfg_channel_reset),

    // Status
    .descriptor_engine_idle (desc_engine_idle),

    // Monitor bus
    .mon_valid          (desceng_mon_valid),
    .mon_ready          (desceng_mon_ready),
    .mon_packet         (desceng_mon_packet)
);

## 10.8	Common Issues

### 10.8.1	Issue 1: APB Write Not Accepted

Symptom: apb_ready never asserts

Root Causes: 1. Channel not idle (channel_idle = 0) 2. Previous APB transaction still in progress (r_apb_ip = 1) 3. Descriptor address FIFO not empty 4. Channel reset active

Solution: Wait for channel to complete current work before next kick-off.

### 10.8.2	Issue 2: Descriptor Chain Stops Early

Symptom: Chaining stops before expected

Root Causes: 1. next_descriptor_ptr is 0 or out of valid range 2. last flag set in descriptor 3. AXI error during fetch

Debug: Check descriptor memory contents and address range configuration.

## 10.9	Related Documentation

- Parent: 03_scheduler_group.md - Channel wrapper

- Consumer: 04_scheduler.md - Scheduler that receives descriptors

- Arbiter: 02_scheduler_group_array.md - Descriptor AXI arbitration

## APB Interface: 13_apbtodescr.md - APB to descriptor kick-off

## 10.10	Revision History

Descriptor Engine Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	AXI Read Engine

Module: axi_read_engine.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: stream_core.sv Status: Implemented Last Updated: 2025-11-30

## 11.1	Overview

The axi_read_engine module is a high-performance multi-channel AXI4 read engine with space-aware arbitration. It serves all 8 STREAM channels through a single AXI master interface with intelligent flow control.

### 11.1.1	Key Features

- Round-Robin Arbitration: Fair scheduling across channels

- Space-Aware Masking: Only arbitrate channels with sufficient SRAM space

- Pre-Allocation Handshake: Reserve SRAM space before data arrives

- Streaming Data Path: Direct passthrough to SRAM controller

- Channel ID in AXI ID: Enables per-channel response routing

- Pipelined/Non-Pipelined Modes: Configurable outstanding transaction depth

## 11.2	Architecture

### 11.2.1	Operation Flow

1. Scheduler Interface: Each channel can request read bursts
2. Space Checking: Mask channels without sufficient SRAM space
3. Arbitration: Round-robin arbiter selects next channel to service
4. AXI AR Issue: Issue read command to AXI, assert rd_alloc to SRAM controller
5. AXI R Response: Stream read data directly to SRAM controller

### 11.2.2	Key Design Decisions

Combinational AR Outputs: AR outputs are driven combinationally from the arbiter to avoid 1-cycle delay. When axi_rd_alloc_space_free goes to 0, arvalid drops in the same cycle.

No Internal Buffering: The engine is a streaming pipeline with no internal data storage. Data flows directly from AXI R channel to SRAM controller.

### 11.2.3	System Idle Behavior

When ALL channels complete their work (w_arb_request == 0), the engine becomes idle until new requests arrive. This is NOT a bubble - it’s legitimate system idle.

Why This Matters for Testing: - With few active channels and short bursts, there are brief periods when all channels are in their WRITE phase - During these periods, m_axi_arvalid = 0 (no R channel activity) - This is EXPECTED behavior, not a performance bug - The dbg_arb_request signal exposes w_arb_request to help testbenches distinguish: - TRUE BUBBLE: arvalid=0 while arb_request!=0 (channels waiting, arbiter stalled) - SYSTEM IDLE: arvalid=0 while arb_request==0 (no channels need service)

## 11.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | Number of channels |
| ADDR_WIDTH | int | 64 | AXI address width |
| DATA_WIDTH | int | 512 | AXI data width |
| ID_WIDTH | int | 8 | AXI ID width |
| SEG_COUNT_WIDTH | int | 8 | Width of space/count signals |
| PIPELINE | int | 0 | 0: non-pipelined, 1: pipelined |
| AR_MAX_OUTSTANDING | int | 8 | Maximum outstanding AR requests (PIPELINE=1) |
| STROBE_EVERY_BEAT | int | 0 | 0: strobe on last beat, 1: strobe every beat |

### 11.3.1	Derived Parameters

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| NC | NUM_CHANNELS | Short alias |
| AW | ADDR_WIDTH | Short alias |
| DW | DATA_WIDTH | Short alias |
| IW | ID_WIDTH | Short alias |
| SCW | SEG_COUNT_WIDTH | Segment count width |
| CIW | $clog2(NC) | Channel ID width (min 1 bit) |

## 11.4	Port List

### 11.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 11.4.2	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | input | 8 | Transfer size in beats (all channels) |

### 11.4.3	Scheduler Interface (Per-Channel)

Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_valid[ch] | input | NC | Channel requests read |
| sched_rd_addr[ch] | input | NC x AW | Source addresses |
| sched_rd_beats[ch] | input | NC x 32 | Beats remaining to read |

### 11.4.4	Completion Interface (Per-Channel)

Completion Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_done_strobe[ch] | output | NC | Burst completed (1 cycle pulse) |
| sched_rd_beats_done[ch] | output | NC x 32 | Number of beats completed |

### 11.4.5	SRAM Allocation Interface

SRAM Allocation Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_alloc_req | output | 1 | Request space allocation |
| axi_rd_alloc_size | output | 8 | Beats to reserve |
| axi_rd_alloc_id | output | IW | Transaction ID (channel) |
| axi_rd_alloc_space_free[ch] | input | NC x SCW | Free space per channel |

### 11.4.6	SRAM Write Interface

SRAM Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_sram_valid | output | 1 | Read data valid |
| axi_rd_sram_ready | input | 1 | Ready to accept data |
| axi_rd_sram_id | output | IW | Transaction ID (channel) |
| axi_rd_sram_data | output | DW | Read data payload |

### 11.4.7	AXI4 AR Channel

AXI4 AR Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arvalid | output | 1 | Address valid |
| m_axi_arready | input | 1 | Address ready |
| m_axi_arid | output | IW | Transaction ID |
| m_axi_araddr | output | AW | Address |
| m_axi_arlen | output | 8 | Burst length - 1 |
| m_axi_arsize | output | 3 | Burst size (log2 bytes) |
| m_axi_arburst | output | 2 | Burst type (INCR) |

### 11.4.8	AXI4 R Channel

AXI4 R Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_rvalid | input | 1 | Read data valid |
| m_axi_rready | output | 1 | Read data ready |
| m_axi_rid | input | IW | Transaction ID |
| m_axi_rdata | input | DW | Read data |
| m_axi_rresp | input | 2 | Response |
| m_axi_rlast | input | 1 | Last beat of burst |

### 11.4.9	Error Interface

Error Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_rd_error[ch] | output | NC | Sticky error flag per channel |

### 11.4.10	Debug Interface

Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_rd_all_complete[ch] | output | NC | All reads complete |
| dbg_r_beats_rcvd | output | 32 | Total R beats received |
| dbg_sram_writes | output | 32 | Total writes to SRAM |
| dbg_arb_request[ch] | output | NC | Arbiter request vector |

## 11.5	Operation

### 11.5.1	Space-Aware Request Masking

// Only request arbitration if:
// 1. Scheduler is requesting (sched_rd_valid)
// 2. Sufficient SRAM space available (w_space_ok)
// 3. Below outstanding limit (w_below_outstanding_limit)
w_arb_request[i] = sched_rd_valid[i] && w_space_ok[i] && w_below_outstanding_limit[i];

### 11.5.2	Transfer Size Calculation

// Calculate actual transfer size for this channel
// Min of remaining beats or configured max
w_transfer_size[i] = (sched_rd_beats[i] <= cfg_axi_rd_xfer_beats + 1) ?
                     (sched_rd_beats[i] - 1) : cfg_axi_rd_xfer_beats;

### 11.5.3	Outstanding Transaction Tracking

PIPELINE=0 (Non-Pipelined): - Binary flag per channel (0 or 1 outstanding) - Set when AR issues for channel - Clear when R last arrives for channel

PIPELINE=1 (Pipelined): - Counter per channel (0 to AR_MAX_OUTSTANDING) - Increment on AR handshake - Decrement on R last handshake

### 11.5.4	Completion Strobe

Strobes fire when AR transaction is accepted (not when R completes):

// Pulse when AR transaction is accepted (arvalid && arready)
// This tells scheduler that request was issued to AXI bus
if (m_axi_arvalid && m_axi_arready) begin
    r_done_strobe[w_arb_grant_id] <= 1'b1;
    r_beats_done[w_arb_grant_id] <= w_transfer_size[w_arb_grant_id] + 1;
end

## 11.6	Timing Diagrams

### 11.6.1	Perfect Streaming - AXI Read Transaction

The following timing diagram shows the AXI read engine operating at maximum throughput with perfect streaming - where both rvalid and rready remain HIGH for consecutive clock cycles, achieving one data beat per cycle.

#### 11.6.1.1	Waveform 2.6.1: AXI Read Engine - Perfect Streaming

AXI Read Engine - Perfect Streaming

Transaction Flow:

- AR Channel Handshake (Address Phase)

  - m_axi_arvalid rises to initiate a read request

  - m_axi_arready is already HIGH (slave ready)

  - Address (araddr), ID (arid), and length (arlen=7 for 8 beats) are captured

  - Handshake completes in a single cycle when both valid and ready are HIGH

- R Channel Streaming (Data Phase)

  - After AR acceptance, the AXI slave begins returning data

  - m_axi_rvalid rises and stays HIGH for all 8 beats

  - m_axi_rready remains HIGH throughout (no backpressure from SRAM controller)

  - Perfect streaming: One data beat transferred every clock cycle

  - m_axi_rlast rises on the final beat (beat 7) to mark burst completion

- Next Transaction

  - After rlast, the engine can immediately issue the next AR request

  - The cycle repeats for the next channel or next burst

Key Performance Indicators: - No bubbles: rvalid and rready both HIGH during data phase - Full bandwidth: Data width (256/512 bits) x clock frequency - Zero-wait states: SRAM controller accepts data at line rate

### 11.6.2	Multi-Channel Streaming

For multi-channel operation showing channel switching while maintaining streaming performance, see:

#### 11.6.2.1	Waveform 2.6.2: Datapath Read - Multi-Channel

Datapath Read - Multi-Channel

This diagram shows how the engine arbitrates between channels while maintaining high throughput.

## 11.7	Integration Example

axi_read_engine #(
    .NUM_CHANNELS       (8),
    .ADDR_WIDTH         (64),
    .DATA_WIDTH         (512),
    .ID_WIDTH           (8),
    .SEG_COUNT_WIDTH    (10),
    .PIPELINE           (0),
    .AR_MAX_OUTSTANDING (8)
) u_axi_read_engine (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Configuration
    .cfg_axi_rd_xfer_beats  (cfg_axi_rd_xfer_beats),

    // Scheduler interface
    .sched_rd_valid         (sched_rd_valid),
    .sched_rd_addr          (sched_rd_addr),
    .sched_rd_beats         (sched_rd_beats),

    // Completion interface
    .sched_rd_done_strobe   (sched_rd_done_strobe),
    .sched_rd_beats_done    (sched_rd_beats_done),

    // SRAM allocation interface
    .axi_rd_alloc_req       (axi_rd_alloc_req),
    .axi_rd_alloc_size      (axi_rd_alloc_size),
    .axi_rd_alloc_id        (axi_rd_alloc_id),
    .axi_rd_alloc_space_free(axi_rd_alloc_space_free),

    // SRAM write interface
    .axi_rd_sram_valid      (axi_rd_sram_valid),
    .axi_rd_sram_ready      (axi_rd_sram_ready),
    .axi_rd_sram_id         (axi_rd_sram_id),
    .axi_rd_sram_data       (axi_rd_sram_data),

    // AXI master
    .m_axi_arvalid          (m_axi_rd_arvalid),
    .m_axi_arready          (m_axi_rd_arready),
    .m_axi_arid             (m_axi_rd_arid),
    .m_axi_araddr           (m_axi_rd_araddr),
    .m_axi_arlen            (m_axi_rd_arlen),
    .m_axi_arsize           (m_axi_rd_arsize),
    .m_axi_arburst          (m_axi_rd_arburst),

    .m_axi_rvalid           (m_axi_rd_rvalid),
    .m_axi_rready           (m_axi_rd_rready),
    .m_axi_rid              (m_axi_rd_rid),
    .m_axi_rdata            (m_axi_rd_rdata),
    .m_axi_rresp            (m_axi_rd_rresp),
    .m_axi_rlast            (m_axi_rd_rlast),

    // Error and debug
    .sched_rd_error         (sched_rd_error),
    .dbg_rd_all_complete    (dbg_rd_all_complete),
    .dbg_arb_request        (dbg_arb_request)
);

## 11.8	Related Documentation

- Parent: 01_stream_core.md - Top-level integration

- Scheduler Array: 02_scheduler_group_array.md - Provides sched_rd_* signals

- Allocation Controller: 07_stream_alloc_ctrl.md - Space tracking for read engine

- SRAM Controller: 08_sram_controller.md - Receives read data

## Write Engine: 12_axi_write_engine.md - Complementary write datapath

## 11.9	Revision History

AXI Read Engine Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Stream Allocation Controller

Module: stream_alloc_ctrl.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: sram_controller_unit.sv Status: Implemented Last Updated: 2025-11-30

## 12.1	Overview

The stream_alloc_ctrl module is a virtual FIFO without data storage that tracks space allocation and availability using FIFO pointer logic. It provides pre-allocation support for AXI read engines to prevent race conditions between burst requests and data arrival.

### 12.1.1	What Makes This a “Virtual FIFO”

Virtual FIFO: - Has write pointer (allocation pointer) - Has read pointer (actual write pointer) - Calculates full/empty/space_free - NO data storage - only pointer arithmetic

Use Case: Reserve FIFO space BEFORE AXI read data arrives

## 12.2	The Allocation Problem

### 12.2.1	Without Allocation Controller

Problem: Race condition between space check and data arrival

Cycle 0: Read engine checks FIFO space
         space_free = 100 beats → OK to issue 16-beat burst

Cycle 5: AR handshake completes, AXI read starts

Cycle 10: Meanwhile, write engine drains 90 beats from FIFO
          space_free = 10 beats (NOT ENOUGH!)

Cycle 15: AXI read data starts arriving (16 beats)
          → OVERFLOW! Only 10 beats of space available

### 12.2.2	With Allocation Controller

Solution: Reserve space when issuing AR, not when data arrives

Cycle 0: Read engine checks space_free = 100 beats
         Allocate 16 beats:
           rd_alloc_req = 1, rd_alloc_size = 16
           → space_free = 84 beats (reserved!)

Cycle 5: AR handshake completes

Cycle 10: Write engine checks space_free = 84 beats
          → Sees reduced space, won't overdrain

Cycle 15: AXI read data arrives, enters FIFO
          → Space was reserved, guaranteed to fit

## 12.3	Architecture

### 12.3.1	Two-Pointer System

### 12.3.2	Figure 2.7.1: Stream Allocation Controller Block Diagram

Diagram

–>

Write Pointer (Allocation Pointer): - Advances when space is allocated (burst request) - Variable-size increment (rd_alloc_size) - Represents “reserved space”

Read Pointer (Actual Write Pointer): - Advances when data exits the controller (output handshake) - Single-beat increment - Represents “released space”

Space Calculation:

space_free = DEPTH - (wr_ptr - rd_ptr)

### 12.3.3	CRITICAL: Confusing Naming Convention

WARNING: Allocation controller uses OPPOSITE naming from normal FIFO!

Normal FIFO: | Signal | Meaning | |——–|———| | wr_* | Write data → Consumes space → space_free decreases | | rd_* | Read data → Frees space → space_free increases |

: CRITICAL: Confusing Naming Convention

Allocation Controller: | Signal | Meaning | |——–|———| | wr_* | ALLOCATE space → Reserves space → space_free decreases | | rd_* | RELEASE space → Data exits controller → space_free increases |

: CRITICAL: Confusing Naming Convention

Why This Matters:

// "Write" side = ALLOCATE (reserve space for upcoming burst)
// Read engine says "I need 16 beats"
.wr_valid (rd_alloc_req),
.wr_size  (rd_alloc_size),  // Variable size (16 in this example)

// "Read" side = RELEASE (data exits controller, free space)
// Must monitor OUTPUT handshake (after latency bridge!)
.rd_valid (axi_wr_sram_valid && axi_wr_sram_ready),  // Output side!

Key Insight: The “read” side of allocation controller connects to the OUTPUT of the FIFO + latency bridge, NOT the FIFO input!

## 12.4	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| DEPTH | int | 512 | Virtual FIFO depth (must match physical FIFO) |
| ALMOST_WR_MARGIN | int | 1 | Almost full threshold |
| ALMOST_RD_MARGIN | int | 1 | Almost empty threshold |
| REGISTERED | int | 1 | Register outputs for timing |

## 12.5	Port List

### 12.5.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low asynchronous reset |

### 12.5.2	Write Interface (Allocation Requests)

Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | Allocate space request |
| wr_size | input | 8 | Number of entries to allocate |
| wr_ready | output | 1 | Space available (!wr_full) |

### 12.5.3	Read Interface (Actual Data Written)

Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | Data exits controller (release space) |
| rd_ready | output | 1 | Not empty (!rd_empty) |

### 12.5.4	Status Outputs

Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| space_free | output | AW+1 | Available space (beats) |
| wr_full | output | 1 | Full flag (no space available) |
| wr_almost_full | output | 1 | Almost full flag |
| rd_empty | output | 1 | Empty flag (no allocations) |
| rd_almost_empty | output | 1 | Almost empty flag |

## 12.6	Interfaces

### 12.6.1	Write Interface (Allocation Requests)

Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | Input | 1 | Allocate space |
| wr_size | Input | 8 | Number of entries to allocate |
| wr_ready | Output | 1 | Space available (!wr_full) |

Usage:

// Read engine allocates space before issuing AR
rd_alloc_req = (space_check_ok && !ar_pending);
rd_alloc_size = cfg_axi_rd_xfer_beats;

### 12.6.2	Read Interface (Actual Data Written)

Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | Input | 1 | Data exits controller |
| rd_ready | Output | 1 | Not empty (!rd_empty) |

Usage:

// Connect to OUTPUT handshake (after latency bridge)
assign rd_valid = (axi_wr_sram_valid && axi_wr_sram_ready);

### 12.6.3	Status Outputs

Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| space_free | Output | AW+1 | Available unreserved space |
| wr_full | Output | 1 | No space available |
| wr_almost_full | Output | 1 | Almost full |
| rd_empty | Output | 1 | No allocations pending |
| rd_almost_empty | Output | 1 | Almost empty |

Note: space_free is the most important output - used by read engine for space checking.

## 12.7	Operation

### 12.7.1	Allocation Flow

Step 1: Check Space

// Read engine checks space availability
if (space_free >= (cfg_axi_rd_xfer_beats << 1)) begin  // 2x margin
    // Space OK, proceed to allocation
end

Step 2: Allocate Space

rd_alloc_req = 1'b1;
rd_alloc_size = 8'd16;  // Reserve 16 beats

// Next cycle: wr_ptr advances
r_wr_ptr_bin <= r_wr_ptr_bin + 16;
space_free decreases by 16

Step 3: AXI Read Executes

// Some cycles later, AXI AR handshake completes
m_axi_arvalid = 1, m_axi_arready = 1
// AXI read starts

Step 4: Data Arrives at FIFO

// Many cycles later, AXI read data arrives
axi_rd_sram_valid = 1, axi_rd_sram_ready = 1
// Data enters FIFO (allocation controller doesn't see this directly)

Step 5: Data Exits Controller (After Latency Bridge)

// Eventually, data traverses FIFO + latency bridge
axi_wr_sram_valid = 1, axi_wr_sram_ready = 1

// THIS triggers allocation controller release!
rd_valid = 1
r_rd_ptr_bin <= r_rd_ptr_bin + 1
space_free increases by 1

### 12.7.2	Pointer Arithmetic

Write Pointer (Allocation):

// Variable-size increment
if (w_write && !r_wr_full) begin
    r_wr_ptr_bin <= r_wr_ptr_bin + (AW+1)'(wr_size);
end

Read Pointer (Release):

// Single-beat increment (uses counter_bin utility)
counter_bin #(
    .WIDTH (AW + 1),
    .MAX   (D)
) read_pointer_inst (
    .clk              (axi_aclk),
    .rst_n            (axi_aresetn),
    .enable           (w_read && !r_rd_empty),
    .counter_bin_curr (r_rd_ptr_bin),
    .counter_bin_next (w_rd_ptr_bin_next)
);

Space Calculation:

// Occupancy = wr_ptr - rd_ptr
w_count = w_wr_ptr_bin_next - w_rd_ptr_bin_next;

// Space free = total depth - occupancy
space_free = (AW+1)'(D) - w_count;

## 12.8	Timing Behavior

### 12.8.1	Allocation Latency

Allocation is IMMEDIATE (combinational + 1 cycle):

Cycle N:   rd_alloc_req = 1, rd_alloc_size = 16
Cycle N+1: r_wr_ptr_bin = old_value + 16
           space_free = old_value - 16

Read engine sees updated space_free on next cycle.

### 12.8.2	Release Latency

Release is IMMEDIATE (combinational + 1 cycle):

Cycle N:   axi_wr_sram_valid = 1, axi_wr_sram_ready = 1
           → rd_valid = 1
Cycle N+1: r_rd_ptr_bin = old_value + 1
           space_free = old_value + 1

Write engine sees updated space_free on next cycle.

## 12.9	Integration Example

### 12.9.1	In sram_controller_unit.sv

stream_alloc_ctrl #(
    .DEPTH(SD),
    .REGISTERED(1)
) u_alloc_ctrl (
    .axi_aclk           (clk),
    .axi_aresetn        (rst_n),

    // ALLOCATE (reserve space for upcoming burst)
    .wr_valid           (rd_alloc_req),      // From read engine
    .wr_size            (rd_alloc_size),     // Burst size
    .wr_ready           (),                  // Unused (space check uses space_free)

    // RELEASE (data exits controller - OUTPUT handshake!)
    .rd_valid           (axi_wr_sram_valid && axi_wr_sram_ready),  // After latency bridge!
    .rd_ready           (),                  // Unused

    // Space tracking
    .space_free         (alloc_space_free),  // To read engine (via register)

    // Unused status
    .wr_full            (),
    .wr_almost_full     (),
    .rd_empty           (),
    .rd_almost_empty    ()
);

// Register output to break long paths
`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        rd_space_free <= SCW'(SD);  // Full space on reset
    end else begin
        rd_space_free <= alloc_space_free;
    end
)

## 12.10	Debug Support

### 12.10.1	Display Statements

Allocation:

$display("ALLOC @ %t: allocated %0d beats, wr_ptr: %0d -> %0d, space_free will be %0d",
        $time, wr_size, r_wr_ptr_bin, r_wr_ptr_bin + wr_size,
        D - (r_wr_ptr_bin + wr_size - r_rd_ptr_bin));

Release (Drain):

$display("DRAIN @ %t: drained 1 beat, rd_ptr: %0d -> %0d, space_free will be %0d",
        $time, r_rd_ptr_bin, w_rd_ptr_bin_next,
        D - (r_wr_ptr_bin - w_rd_ptr_bin_next));

### 12.10.2	Waveform Analysis

Key Signals to Monitor: - r_wr_ptr_bin - Allocation pointer - r_rd_ptr_bin - Release pointer - space_free - Available space - rd_alloc_req, rd_alloc_size - Allocation requests - axi_wr_sram_valid, axi_wr_sram_ready - Release handshake

## 12.11	Common Issues

### 12.11.1	Issue 1: Space Not Released

Symptom: space_free decreases but never increases

Root Cause: rd_valid not connected to output handshake

Wrong:

.rd_valid (axi_rd_sram_valid && axi_rd_sram_ready)  // FIFO input - WRONG!

Correct:

.rd_valid (axi_wr_sram_valid && axi_wr_sram_ready)  // After latency bridge - CORRECT!

### 12.11.2	Issue 2: Overflow Despite Allocation

Symptom: FIFO overflows even with allocation controller

Root Cause: Read engine uses wrong space value

Wrong:

if (space_free >= cfg_axi_rd_xfer_beats) begin  // Exact match - WRONG!
    allocate();
end

Correct:

if (space_free >= (cfg_axi_rd_xfer_beats << 1)) begin  // 2x margin - CORRECT!
    allocate();
end

Why 2x: Accounts for in-flight allocations and pipeline delays.

### 12.11.3	Issue 3: Allocation Pointer Overflow

Symptom: r_wr_ptr_bin wraps at unexpected value

Root Cause: Pointer width insufficient

Check:

parameter int AW = $clog2(D);  // Address width
logic [AW:0] r_wr_ptr_bin;     // AW+1 bits (for full detection)

Extra bit allows distinguishing full (wr_ptr = rd_ptr + DEPTH) from empty (wr_ptr = rd_ptr).

## 12.12	Comparison with Drain Controller

Comparison with Drain Controller

| Aspect | Allocation Controller | Drain Controller |
| --- | --- | --- |
| Purpose | Reserve FIFO space | Reserve FIFO data |
| User | AXI read engine | AXI write engine |
| Write side | Allocation request (reserve) | Data enters FIFO (increment) |
| Read side | Data exits controller (release) | Drain request (reserve) |
| Naming | OPPOSITE of normal FIFO | SAME as normal FIFO |
| Output | space_free | data_available |

## 12.13	Resource Utilization

Per Instance: - 2 × (AW+1)-bit counters (wr_ptr, rd_ptr) - 1 × (AW+1)-bit space_free calculation - FIFO control block (full/empty logic) - ~50-100 flip-flops total

Example (DEPTH=512, AW=9): - 2 × 10-bit counters = 20 FFs - Control logic = ~30 FFs - Total: ~50 FFs

Very lightweight - pointer logic only, no data storage.

## 12.14	Related Modules

- Counterpart: stream_drain_ctrl.sv - Drain-side flow control

- Parent: sram_controller_unit.sv - Instantiates allocation controller

- User: axi_read_engine.sv - Checks rd_space_free before issuing AR

## 12.15	Related Documentation

- Drain Controller: 11_stream_drain_ctrl.md

- SRAM Controller Unit: 09_sram_controller_unit.md

- SRAM Controller: 08_sram_controller.md

## AXI Read Engine: 06_axi_read_engine.md

## 12.16	Revision History

Stream Allocation Controller Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	SRAM Controller Specification

Module: sram_controller.sv Location: projects/components/stream/rtl/fub/ Status: Implemented Last Updated: 2025-11-30

## 13.1	Overview

The SRAM Controller provides per-channel buffering between AXI read and write engines using independent FIFO structures. Each channel has its own FIFO with dedicated allocation controller (write side) and drain controller (read side).

### 13.1.1	Key Features

- Per-channel FIFOs: Independent gaxi_fifo_sync per channel (no segmentation complexity)

- ID-based routing: Transaction ID selects channel for write/read operations

- Allocation controller: Reserves space before AXI read data arrives

- Drain controller: Manages data availability for AXI write engine

- Latency bridge: Aligns FIFO read latency (registered output)

- Saturating counters: 8-bit space/count reporting per channel

### 13.1.2	Design Rationale

Why Per-Channel FIFOs Instead of Segmented SRAM?

The implementation uses independent FIFOs rather than a monolithic SRAM divided into segments:

Advantages of Per-Channel FIFOs: - Simpler logic - FIFOs are standard, well-tested components - Better isolation - Channel failures don’t affect others - Easier timing - No cross-channel paths or arbitration - Modular - Easy to add/remove channels - No pointer arithmetic - FIFO handles internally

Trade-offs: - More SRAM resources (NC × DEPTH vs. shared pool) - Potential waste if channels idle (unused FIFO space)

Decision: Simplicity and isolation outweigh SRAM efficiency for STREAM’s tutorial focus.

## 13.2	Architecture

### 13.2.1	Block Diagram

### 13.2.2	Figure 2.8.1: SRAM Controller Block Diagram

Diagram

Source: 05_sram_controller_block.mmd

### 13.2.3	Per-Channel Architecture

Each channel contains three components (in sram_controller_unit):

- Allocation Controller (stream_alloc_ctrl):

  - Receives rd_alloc_req from read engine

  - Tracks reserved vs. committed space

  - Provides rd_space_free to read engine

- FIFO (gaxi_fifo_sync):

  - Stores data between read and write engines

  - Depth = SRAM_DEPTH parameter

  - Standard valid/ready handshaking

- Drain Controller + Latency Bridge (stream_drain_ctrl):

  - Receives wr_drain_req from write engine

  - Provides wr_drain_data_avail to write engine

  - Latency bridge aligns FIFO read latency

## 13.3	Parameters

parameter int NUM_CHANNELS = 8;                  // Number of independent channels
parameter int DATA_WIDTH = 512;                  // Data width in bits
parameter int SRAM_DEPTH = 512;                  // Depth per channel FIFO
parameter int SEG_COUNT_WIDTH = $clog2(SRAM_DEPTH) + 1;  // Width of count signals

// Short aliases (internal use)
parameter int NC = NUM_CHANNELS;
parameter int DW = DATA_WIDTH;
parameter int SD = SRAM_DEPTH;
parameter int SCW = SEG_COUNT_WIDTH;             // FIFO depth counter width
parameter int CIW = (NC > 1) ? $clog2(NC) : 1;   // Channel ID width (min 1 bit)

Note: “SEG_COUNT_WIDTH” refers to the FIFO depth counter width (historical name from segmented design consideration).

## 13.4	Port List

### 13.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 13.4.2	Allocation Interface

AXI Read Engine Flow Control (Space Reservation):

Allocation Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_alloc_req | input | 1 | Allocation request (single request with ID) |
| axi_rd_alloc_size | input | 8 | Number of beats to allocate |
| axi_rd_alloc_id | input | CIW | Channel ID for allocation |
| axi_rd_alloc_space_free[ch] | output | NUM_CHANNELS × SEG_COUNT_WIDTH | Free space per channel FIFO |

### 13.4.3	Write Interface

AXI Read Engine → FIFO (ID-based routing):

Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_sram_valid | input | 1 | Write data valid (single valid with ID) |
| axi_rd_sram_ready | output | 1 | Ready (muxed from selected channel) |
| axi_rd_sram_id | input | CIW | Channel ID select for write |
| axi_rd_sram_data | input | DATA_WIDTH | Write data (common bus) |

### 13.4.4	Drain Interface

Write Engine Flow Control (Data Availability):

Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_drain_data_avail[ch] | output | NUM_CHANNELS × SEG_COUNT_WIDTH | Available data after reservations per channel |
| axi_wr_drain_req[ch] | input | NUM_CHANNELS | Per-channel drain request |
| axi_wr_drain_size[ch] | input | NUM_CHANNELS × 8 | Per-channel drain size (beats to reserve) |

### 13.4.5	Read Interface

FIFO → AXI Write Engine (ID-based routing):

Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_sram_valid[ch] | output | NUM_CHANNELS | Per-channel valid (data available) |
| axi_wr_sram_drain | input | 1 | Drain request (consumer ready) |
| axi_wr_sram_id | input | CIW | Channel ID select for read |
| axi_wr_sram_data | output | DATA_WIDTH | Read data from selected channel (muxed) |

### 13.4.6	Debug Interface

Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_bridge_pending[ch] | output | NUM_CHANNELS | Latency bridge pending per channel |
| dbg_bridge_out_valid[ch] | output | NUM_CHANNELS | Latency bridge output valid per channel |

## 13.5	Interface

### 13.5.1	Clock and Reset

input  logic                    clk;
input  logic                    rst_n;      // Active-low asynchronous reset

### 13.5.2	Allocation Interface (AXI Read Engine Flow Control)

Space Reservation:

input  logic                                axi_rd_alloc_req;       // Single allocation request
input  logic [7:0]                          axi_rd_alloc_size;      // Beats to allocate
input  logic [CIW-1:0]                      axi_rd_alloc_id;        // Channel ID for allocation
output logic [NC-1:0][SCW-1:0]              axi_rd_alloc_space_free; // Free space per channel

Allocation Protocol: 1. Read engine issues AR transaction 2. Before R data arrives: Engine asserts axi_rd_alloc_req with size and ID 3. Allocation controller reserves space in selected channel 4. Engine tracks reserved space, prevents over-issuing AR commands 5. Space commits when actual data arrives (FIFO write)

Why Allocation is Critical:

Without pre-allocation, multiple AR commands could be issued before data arrives, causing FIFO overflow:

Problem without allocation:
  Cycle 0: Issue AR (16 beats) - FIFO has 32 free
  Cycle 1: Issue AR (16 beats) - FIFO still shows 32 free (data hasn't arrived!)
  Cycle 2: Issue AR (16 beats) - FIFO still shows 32 free
  Cycle 10: R data starts arriving (48 beats total)
  → OVERFLOW! Only 32 beats of space

Solution with allocation:
  Cycle 0: Issue AR (16 beats), allocate 16 - FIFO shows 16 free (reserved)
  Cycle 1: Issue AR (16 beats), allocate 16 - FIFO shows 0 free (reserved)
  Cycle 2: Cannot issue AR - no space available
  Cycle 10: R data arrives - space is guaranteed

### 13.5.3	Write Interface (AXI Read Engine → FIFO)

ID-Based Write:

input  logic                    axi_rd_sram_valid;      // Single valid for all channels
input  logic [CIW-1:0]          axi_rd_sram_id;         // Channel ID select
output logic                    axi_rd_sram_ready;      // Ready (muxed from selected channel)
input  logic [DW-1:0]           axi_rd_sram_data;       // Shared data bus

Write Protocol: 1. Read engine asserts axi_rd_sram_valid with data 2. axi_rd_sram_id selects which channel FIFO receives data 3. Controller decodes ID to per-channel valid_decoded[id] 4. Selected channel’s FIFO ready muxed to axi_rd_sram_ready

### 13.5.4	Drain Interface (AXI Write Engine Flow Control)

Data Availability:

input  logic [NC-1:0]                       axi_wr_drain_req;       // Per-channel drain request
input  logic [NC-1:0][7:0]                  axi_wr_drain_size;      // Per-channel drain size
output logic [NC-1:0][SCW-1:0]              axi_wr_drain_data_avail;  // Available data after reservations

Drain Protocol: 1. Write engine checks axi_wr_drain_data_avail[ch] for available data 2. Engine asserts axi_wr_drain_req[ch] to reserve data for W burst 3. Drain controller updates available count (subtracts reserved) 4. Data commits when actually read from FIFO

Why Drain Reservation is Critical:

Similar to allocation, drain prevents under-reporting data availability:

Problem without drain reservation:
  Cycle 0: FIFO has 32 beats available
  Cycle 1: Issue AW (16 beats) - FIFO still shows 32 available
  Cycle 2: Issue AW (16 beats) - FIFO still shows 32 available
  Cycle 3: Issue AW (16 beats) - FIFO still shows 32 available
  Cycle 10: W bursts start draining (48 beats expected)
  → UNDERFLOW! Only 32 beats available

Solution with drain reservation:
  Cycle 0: FIFO has 32 beats available
  Cycle 1: Issue AW (16 beats), drain 16 - shows 16 available (reserved)
  Cycle 2: Issue AW (16 beats), drain 16 - shows 0 available (reserved)
  Cycle 3: Cannot issue AW - no data available
  Cycle 10: W bursts drain - data is guaranteed

### 13.5.5	Read Interface (FIFO → AXI Write Engine)

ID-Based Read:

output logic [NC-1:0]           axi_wr_sram_valid;      // Per-channel valid (data available)
input  logic                    axi_wr_sram_drain;      // Single drain (consumer requests data)
input  logic [CIW-1:0]          axi_wr_sram_id;         // Channel ID select
output logic [DW-1:0]           axi_wr_sram_data;       // Data from selected channel

Read Protocol: 1. Write engine checks axi_wr_sram_valid[ch] (per-channel) 2. Engine asserts axi_wr_sram_drain with axi_wr_sram_id 3. Controller decodes ID to per-channel drain_decoded[id] 4. Selected channel’s data muxed to axi_wr_sram_data

### 13.5.6	Debug Interface

output logic [NC-1:0]           dbg_bridge_pending;     // Latency bridge pending per channel
output logic [NC-1:0]           dbg_bridge_out_valid;   // Latency bridge output valid per channel

Purpose: Monitor latency bridge state to catch bugs in read timing.

## 13.6	ID Decode Logic

### 13.6.1	Write Valid Decode

Decode axi_rd_sram_id to per-channel valid:

always_comb begin
    axi_rd_sram_valid_decoded = '0;
    if (axi_rd_sram_valid && axi_rd_sram_id < NC) begin
        axi_rd_sram_valid_decoded[axi_rd_sram_id] = 1'b1;
    end
end

Mux ready from selected channel:

always_comb begin
    if (axi_rd_sram_id < NC) begin
        axi_rd_sram_ready = axi_rd_sram_ready_per_channel[axi_rd_sram_id];
    end else begin
        axi_rd_sram_ready = 1'b0;  // Invalid ID → not ready
    end
end

### 13.6.2	Read/Drain Decode

Decode axi_wr_sram_id to per-channel drain:

always_comb begin
    axi_wr_sram_drain_decoded = '0;
    if (axi_wr_sram_drain && axi_wr_sram_id < NC) begin
        axi_wr_sram_drain_decoded[axi_wr_sram_id] = 1'b1;
    end
end

Mux data from selected channel:

always_comb begin
    if (axi_wr_sram_id < NC) begin
        axi_wr_sram_data = axi_wr_sram_data_per_channel[axi_wr_sram_id];
    end else begin
        axi_wr_sram_data = '0;  // Invalid ID → zero data
    end
end

### 13.6.3	Allocation Decode

Decode axi_rd_alloc_id to per-channel allocation:

always_comb begin
    axi_rd_alloc_req_decoded = '0;
    if (axi_rd_alloc_req && axi_rd_alloc_id < NC) begin
        axi_rd_alloc_req_decoded[axi_rd_alloc_id] = 1'b1;
    end
end

## 13.7	Per-Channel Unit

Each channel instantiates sram_controller_unit (separate module):

sram_controller_unit #(
    .DATA_WIDTH(DW),
    .SRAM_DEPTH(SRAM_DEPTH),
    .SEG_COUNT_WIDTH(SEG_COUNT_WIDTH)
) u_channel_unit (
    .clk                (clk),
    .rst_n              (rst_n),

    // Write interface (decoded valid from ID)
    .axi_rd_sram_valid  (axi_rd_sram_valid_decoded[i]),
    .axi_rd_sram_ready  (axi_rd_sram_ready_per_channel[i]),
    .axi_rd_sram_data   (axi_rd_sram_data),      // SHARED

    // Read interface (decoded drain)
    .axi_wr_sram_valid  (axi_wr_sram_valid[i]),
    .axi_wr_sram_ready  (axi_wr_sram_drain_decoded[i]),
    .axi_wr_sram_data   (axi_wr_sram_data_per_channel[i]),

    // Allocation interface (decoded req from ID)
    .rd_alloc_req       (axi_rd_alloc_req_decoded[i]),
    .rd_alloc_size      (axi_rd_alloc_size),     // SHARED
    .rd_space_free      (axi_rd_alloc_space_free[i]),

    // Drain interface (per-channel)
    .wr_drain_req       (axi_wr_drain_req[i]),
    .wr_drain_size      (axi_wr_drain_size[i]),
    .wr_drain_data_avail(axi_wr_drain_data_avail[i]),

    // Debug
    .dbg_bridge_pending     (dbg_bridge_pending[i]),
    .dbg_bridge_out_valid   (dbg_bridge_out_valid[i])
);

Key Points: - axi_rd_sram_data is shared - all units see same data bus - Only unit with valid_decoded[i] = 1 writes to its FIFO - rd_alloc_size is shared - all see same size value - Only unit with alloc_req_decoded[i] = 1 reserves space

## 13.8	Operation Flows

### 13.8.1	Write Flow (AXI Read Data → FIFO)

Allocation Phase (before data arrives):

1. Read engine issues AR command (araddr, arlen, arid)
2. Engine extracts channel ID from arid[CIW-1:0]
3. Engine asserts:
     axi_rd_alloc_req = 1
     axi_rd_alloc_id = channel_id
     axi_rd_alloc_size = arlen + 1  (burst size in beats)
4. Allocation controller (in sram_controller_unit):
     - Checks rd_space_free[channel_id] >= alloc_size
     - Reserves space (doesn't commit yet)
5. Engine proceeds with AR transaction

Data Arrival Phase:

1. AXI R data arrives:
     m_axi_rvalid = 1
     m_axi_rid = transaction_id (contains channel_id in lower bits)
     m_axi_rdata = data

2. Read engine forwards to SRAM controller:
     axi_rd_sram_valid = 1
     axi_rd_sram_id = rid[CIW-1:0]  (extract channel ID)
     axi_rd_sram_data = rdata

3. SRAM controller decodes:
     axi_rd_sram_valid_decoded[channel_id] = 1

4. Selected channel FIFO:
     - Writes data if ready
     - Commits 1 beat of reserved space
     - Decrements reserved counter

5. Ready mux:
     axi_rd_sram_ready = fifo_ready[channel_id]

### 13.8.2	Read Flow (FIFO → AXI Write Data)

Drain Reservation Phase (before W data drains):

1. Write engine checks drain_data_avail[channel_id]
2. If sufficient data available:
     axi_wr_drain_req[channel_id] = 1
     axi_wr_drain_size[channel_id] = burst_size
3. Drain controller:
     - Reserves data (decrements available count)
     - Prevents other channels from seeing this data

Data Drain Phase:

1. Write engine ready to consume data:
     axi_wr_sram_drain = 1
     axi_wr_sram_id = channel_id

2. SRAM controller decodes:
     axi_wr_sram_drain_decoded[channel_id] = 1

3. Selected channel FIFO:
     - Reads data (1-cycle latency through bridge)
     - Commits 1 beat of reserved data
     - Decrements drain reservation

4. Data mux:
     axi_wr_sram_data = fifo_data[channel_id]

5. Valid output:
     axi_wr_sram_valid[channel_id] = bridge_valid

## 13.9	Allocation vs. Drain Controllers

### 13.9.1	Why Separate Controllers?

Allocation Controller (Write Side): - Problem: AXI read AR issued before R data arrives - Solution: Pre-allocate space when AR issues, commit when R arrives - Prevents: Over-issuing AR commands when FIFO full

Drain Controller (Read Side): - Problem: AXI write AW issued before W data drains from FIFO - Solution: Reserve data when AW issues, commit when W drains - Prevents: Under-reporting available data when burst in progress

### 13.9.2	Flow Control Comparison

Read Engine (uses allocation):

// Check space BEFORE issuing AR
if (rd_space_free[ch] >= burst_size) begin
    issue_ar_command();
    assert_rd_alloc_req();  // Reserve space
end
// Later: R data arrives → commits reservation

Write Engine (uses drain):

// Check data availability BEFORE issuing AW
if (wr_drain_data_avail[ch] >= burst_size) begin
    issue_aw_command();
    assert_wr_drain_req();  // Reserve data
end
// Later: W beats drain → commits reservation

## 13.10	Timing Diagrams

### 13.10.1	Combined Write and Read Timing

The following diagram shows the complete SRAM controller write and read flow:

#### 13.10.1.1	Waveform 2.8.1: SRAM Controller Write/Read Operation

SRAM Controller Write/Read

Source: sram_controller_wr_rd.json

### 13.10.2	Write Path (R Data → FIFO)

Cycle:  0    1    2    3    4    5
        |    |    |    |    |    |

: Write Path
Alloc:  [REQ][---grant---]
        (ch2, size=4)

R Data:                [V][V][V][V]
R ID:                  [2][2][2][2]
        |              |  |  |  |

: Write Path
Decode:                [1][1][1][1] (channel 2)
FIFO:                  [W][W][W][W]
Ready:  [1][1][1][1][1][1][1]...

Notes:
- Allocation at cycle 0 reserves 4 beats
- R data arrives cycles 3-6
- Each beat commits 1 reserved entry

### 13.10.3	Read Path (FIFO → W Data)

Cycle:  0    1    2    3    4    5    6
        |    |    |    |    |    |    |

: Read Path
Drain:  [REQ][---reserve---]
        (ch2, size=2)

Drain:            [D][D]
Drain ID:         [2][2]
        |         |  |

: Read Path
Decode:           [1][1] (channel 2)
Bridge:           [-][V][V] (1-cycle latency)
W Data:               [D0][D1]

Notes:
- Drain request at cycle 0 reserves 2 beats
- Actual drain at cycles 3-4
- Latency bridge adds 1 cycle (data at 4-5)

## 13.11	Error Conditions

### 13.11.1	Invalid Channel ID

Condition: axi_rd_sram_id >= NUM_CHANNELS or axi_wr_sram_id >= NUM_CHANNELS

Handling: - Write: axi_rd_sram_ready = 0 (not ready, data dropped) - Read: axi_wr_sram_data = 0 (zero data output) - No error signal - engines should not generate invalid IDs

### 13.11.2	FIFO Overflow

Condition: Write when FIFO full

Prevention: - Allocation controller tracks reserved + committed space - Read engine checks rd_space_free before issuing AR - Only issues AR when sufficient space available

Should never happen if read engine follows protocol!

### 13.11.3	FIFO Underflow

Condition: Read when FIFO empty

Prevention: - Drain controller tracks available - reserved data - Write engine checks wr_drain_data_avail before issuing AW - Only issues AW when sufficient data available

Should never happen if write engine follows protocol!

## 13.12	Performance Considerations

### 13.12.1	Per-Channel FIFO Benefits

Isolation: - Channel failures don’t affect others - No cross-channel contention - Simplified debug (each channel independent)

Timing: - No shared arbitration delays - Predictable latency per channel - Easier timing closure

Scalability: - Easy to add/remove channels - Parameterizable depth per channel - No global SRAM redesign needed

### 13.12.2	SRAM Resource Usage

Total SRAM: NUM_CHANNELS × SRAM_DEPTH × DATA_WIDTH

Example (8 channels, 512 depth, 512-bit data): - Per channel: 512 × 512 bits = 32KB - Total: 8 × 32KB = 256KB

Comparison to Segmented Approach: - Segmented: 1 × (8 × 512) × 512 bits = 256KB (same total) - But segmented requires complex pointer arithmetic and arbitration - Per-channel FIFOs trade complexity for slight area increase (FIFO overhead)

## 13.13	Testing

Test Location: projects/components/stream/dv/tests/fub_tests/sram_controller/

Key Test Scenarios:

- Single channel fill/drain - Basic FIFO operation

- All channels concurrent - Independent operation

- ID decode - Correct channel selection

- Allocation before data - Space reserved correctly

- Drain before read - Data reserved correctly

- Latency bridge - 1-cycle delay verified

- Invalid ID - Graceful handling

- Back-to-back - No bubbles between bursts

## 13.14	Related Documentation

- SRAM Controller Unit: 09_sram_controller_unit.md - Per-channel implementation

- Allocation Controller: 07_stream_alloc_ctrl.md - Write-side flow control

- Drain Controller: 11_stream_drain_ctrl.md - Read-side flow control

- AXI Read Engine: 06_axi_read_engine.md - Write interface usage

- AXI Write Engine: 12_axi_write_engine.md - Read interface usage

- Stream Core: 01_stream_core.md - Top-level integration

## 13.15	Revision History

Revision History

| Date | Version | Changes |
| --- | --- | --- |
| 2025-10-18 | 0.5 | Initial draft documenting segmented SRAM approach |
| 2025-11-16 | 1.0 | Updated to document per-channel FIFO implementation (actual RTL) |
| 2025-11-21 | 1.5 | Merged documentation:- Consolidated duplicate files- Added design rationale section- Enhanced allocation/drain explanations with examples- Clarified per-channel FIFO architecture- Added performance comparison- Verified all content matches current RTL implementation |
| 2025-11-30 | 1.6 | Updated related documentation references |

Last Updated: 2025-11-30 (verified against RTL implementation)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	SRAM Controller Unit

Module: sram_controller_unit.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: sram_controller.sv Status: Implemented Last Updated: 2025-11-30

## 14.1	Overview

The sram_controller_unit module is a single-channel SRAM controller unit containing allocation controller, FIFO buffer, and latency bridge. It handles one channel’s data flow from AXI read engine through buffering to AXI write engine with proper flow control.

### 14.1.1	Key Features

- Three-Component Architecture:

  - Allocation controller (stream_alloc_ctrl) - Space tracking for reads

  - FIFO buffer (gaxi_fifo_sync) - Physical data storage

  - Latency bridge (stream_latency_bridge) - Timing compensation

- Drain Controller: Tracks data availability for write engine

- Virtual FIFO Pattern: Pointer arithmetic without data storage for flow control

- Registered Outputs: Breaks combinatorial paths for timing closure

## 14.2	Architecture

### 14.2.1	Block Diagram

### 14.2.2	Figure 1: SRAM Controller Unit Block Diagram

SRAM Controller Unit Block Diagram

Source: 06_sram_controller_unit_block.mmd

### 14.2.3	Component Hierarchy

sram_controller_unit
 stream_alloc_ctrl          # Allocation tracking (space availability)
 stream_drain_ctrl          # Drain tracking (data availability)
 gaxi_fifo_sync             # Physical data storage FIFO
 stream_latency_bridge      # 1-cycle latency compensation

### 14.2.4	Data Flow

AXI Read Engine � FIFO Write Port � FIFO Storage � FIFO Read Port � Latency Bridge � AXI Write Engine
                                                                            
         Allocation Controller (space tracking) 
                                                                             
                                          Drain Controller (data tracking)

### 14.2.5	Controller Naming Convention (CRITICAL)

Allocation Controller Perspective: - wr side = ALLOCATE (reserve space, advance wr_ptr) - rd side = FULFILL (data arrives, advance rd_ptr, FREE space)

Drain Controller Perspective: - wr side = DATA WRITTEN (increment occupancy) - rd side = DRAIN REQUEST (reserve data for write burst)

This is OPPOSITE of normal FIFO naming conventions!

## 14.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | int | 512 | Data width in bits |
| SRAM_DEPTH | int | 512 | FIFO depth in entries |
| SEG_COUNT_WIDTH | int | $clog2(SRAM_DEPTH)+1 | Width for count signals |

### 14.3.1	Derived Parameters

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| DW | DATA_WIDTH | Short alias |
| SD | SRAM_DEPTH | Short alias |
| SCW | SEG_COUNT_WIDTH | Segment count width |
| ADDR_WIDTH | $clog2(SD) | FIFO address width |

## 14.4	Port List

### 14.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 14.4.2	Allocation Interface (Read Engine Flow Control)

Allocation Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_alloc_req | input | 1 | Request space allocation |
| axi_rd_alloc_size | input | 8 | Beats to reserve |
| axi_rd_alloc_space_free | output | SCW | Free space available |

### 14.4.3	Write Interface (AXI Read Engine to FIFO)

Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_rd_sram_valid | input | 1 | Write data valid |
| axi_rd_sram_ready | output | 1 | Ready to accept data |
| axi_rd_sram_data | input | DW | Write data payload |

### 14.4.4	Drain Interface (Write Engine Flow Control)

Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_drain_data_avail | output | SCW | Data available for drain |
| axi_wr_drain_req | input | 1 | Request to drain data |
| axi_wr_drain_size | input | 8 | Beats to drain |

### 14.4.5	Read Interface (FIFO to AXI Write Engine)

Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_sram_valid | output | 1 | Read data valid |
| axi_wr_sram_ready | input | 1 | Ready to accept data |
| axi_wr_sram_data | output | DW | Read data payload |

### 14.4.6	Debug Interface

Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_bridge_pending | output | 1 | Data in flight in bridge |
| dbg_bridge_out_valid | output | 1 | Bridge output valid |

## 14.5	Operation

### 14.5.1	Allocation Flow

- Space Check: Read engine checks axi_rd_alloc_space_free

- Reservation: Read engine asserts axi_rd_alloc_req with burst size

- Space Decrement: Allocation controller decrements space_free

- Data Arrival: Data enters FIFO via axi_rd_sram_* interface

- Space Release: When data exits (output handshake), space_free increments

### 14.5.2	Drain Flow

- Data Check: Write engine checks axi_wr_drain_data_avail

- Reservation: Write engine asserts axi_wr_drain_req with burst size

- Data Decrement: Drain controller decrements data_available

- Data Consumption: Write engine reads via axi_wr_sram_* interface

### 14.5.3	Data Available Calculation

// Total data available = drain controller data + latency bridge occupancy
assign axi_wr_drain_data_avail = drain_data_available + SCW'(bridge_occupancy);

The latency bridge can hold 1 beat in flight plus up to 4 beats in its skid buffer, which must be accounted for in the data availability count.

## 14.6	Integration Example

sram_controller_unit #(
    .DATA_WIDTH     (512),
    .SRAM_DEPTH     (512),
    .SEG_COUNT_WIDTH(10)
) u_sram_controller_unit (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Allocation interface
    .axi_rd_alloc_req       (axi_rd_alloc_req[ch]),
    .axi_rd_alloc_size      (axi_rd_alloc_size),
    .axi_rd_alloc_space_free(axi_rd_alloc_space_free[ch]),

    // Write interface (from AXI Read Engine)
    .axi_rd_sram_valid      (axi_rd_sram_valid[ch]),
    .axi_rd_sram_ready      (axi_rd_sram_ready[ch]),
    .axi_rd_sram_data       (axi_rd_sram_data),

    // Drain interface
    .axi_wr_drain_data_avail(axi_wr_drain_data_avail[ch]),
    .axi_wr_drain_req       (axi_wr_drain_req[ch]),
    .axi_wr_drain_size      (axi_wr_drain_size[ch]),

    // Read interface (to AXI Write Engine)
    .axi_wr_sram_valid      (axi_wr_sram_valid[ch]),
    .axi_wr_sram_ready      (axi_wr_sram_ready[ch]),
    .axi_wr_sram_data       (axi_wr_sram_data[ch]),

    // Debug
    .dbg_bridge_pending     (dbg_bridge_pending[ch]),
    .dbg_bridge_out_valid   (dbg_bridge_out_valid[ch])
);

## 14.7	Common Issues

### 14.7.1	Issue 1: Space Accounting Mismatch

Symptom: Read engine sees space but FIFO overflows

Root Causes: 1. Allocation controller not receiving fulfillment signals 2. Output handshake not connected to allocation controller rd_valid

Solution: Ensure axi_wr_sram_valid && axi_wr_sram_ready connects to allocation controller.

### 14.7.2	Issue 2: Data Available Undercount

Symptom: Write engine stalls despite data in FIFO

Root Causes: 1. Bridge occupancy not included in data_available calculation 2. Drain controller not receiving write handshakes

Solution: Verify axi_wr_drain_data_avail = drain_data_available + bridge_occupancy.

## 14.8	Related Documentation

- Parent: 08_sram_controller.md - Multi-channel SRAM controller

- Allocation Controller: 07_stream_alloc_ctrl.md - Space tracking details

- Drain Controller: 11_stream_drain_ctrl.md - Data tracking details

- Latency Bridge: 10_stream_latency_bridge.md - Timing compensation

- Read Engine: 06_axi_read_engine.md - Data producer

## Write Engine: 12_axi_write_engine.md - Data consumer

## 14.9	Revision History

SRAM Controller Unit Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Stream Latency Bridge

Module: stream_latency_bridge.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: sram_controller_unit.sv Status: Implemented Last Updated: 2025-11-30

## 15.1	Overview

The stream_latency_bridge module is a simple latency-1 bridge that compensates for registered FIFO output latency. It uses glue logic plus a skid buffer to handle backpressure from downstream consumers.

### 15.1.1	Key Features

- 1-Cycle Latency Compensation: Bridges registered FIFO output to consumer

- Skid Buffer Architecture: 4-deep FIFO absorbs consumer backpressure

- Full Throughput: Maintains back-to-back transfers when consumer is ready

- Simple Design: No complex ready calculations - skid buffer handles it all

- Occupancy Tracking: Reports total beats held for flow control

## 15.2	Architecture

### 15.2.1	Block Diagram

### 15.2.2	Figure 2.10.1: Stream Latency Bridge Block Diagram

Stream Latency Bridge Block Diagram

Source: 06_stream_latency_bridge_block.mmd

### 15.2.3	Operation Flow

Cycle 0: Drain FIFO (s_valid && skid_ready) � r_drain_ip = 1
Cycle 1: Data arrives from FIFO � Push to skid (skid_valid = r_drain_ip)
Cycle N: Consumer drains skid buffer at its own pace

### 15.2.4	Component Diagram

+-------------------+
                    |   Glue Logic      |

: Component Diagram
   s_valid -------->|  (r_drain_ip)     |
   s_ready <--------|                   |
   s_data  -------->|                   |
                    +--------+----------+
                             |
                             v
                    +-------------------+
                    |   Skid Buffer     |
                    |  (gaxi_fifo_sync) |
                    |  Depth: 4         |

: Component Diagram
                    +--------+----------+
                             |
                             v
   m_valid <-----------------|
   m_ready ---------------->||
   m_data  <-----------------|

### 15.2.5	Backpressure Logic

The bridge uses a conservative backpressure approach: - Track stalled writes (wr_valid && !wr_ready) - Calculate pending count = skid_count + stalled - Allow accept when pending < SKID_DEPTH OR consumer is draining

Truth Table (SKID_DEPTH=4):
count | wr_valid | wr_ready | stalled | pending | room | s_ready
------|----------|----------|---------|---------|------|--------
0     | 0        | 1        | 0       | 0       | 1    | 1      � Empty
0     | 1        | 1        | 0       | 0       | 1    | 1      � Writing, completes
3     | 0        | 1        | 0       | 3       | 1    | 1      � Room for 1
3     | 1        | 1        | 0       | 3       | 1    | 1      � Writing, will be 4
3     | 1        | 0        | 1       | 4       | 0    | 0      � Write stalled
4     | 0        | 0        | 0       | 4       | 0    | 0      � Full

## 15.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | int | 64 | Data width in bits |
| SKID_DEPTH | int | 4 | Skid buffer depth (2-4 recommended) |

### 15.3.1	Derived Parameters

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| DW | DATA_WIDTH | Short alias |

## 15.4	Port List

### 15.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 15.4.2	Upstream Interface (from registered FIFO)

Upstream Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_valid | input | 1 | Upstream valid (FIFO not empty) |
| s_ready | output | 1 | Ready to accept from FIFO |
| s_data | input | DW | Data from FIFO (valid 1 cycle after handshake) |

### 15.4.3	Downstream Interface (to consumer)

Downstream Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_valid | output | 1 | Data valid to consumer |
| m_ready | input | 1 | Consumer ready |
| m_data | output | DW | Data to consumer |

### 15.4.4	Status Interface

Status Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| occupancy | output | 3 | Beats in bridge (0-5: 1 in flight + 4 in skid) |

### 15.4.5	Debug Interface

Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_r_pending | output | 1 | Data in flight (r_drain_ip) |
| dbg_r_out_valid | output | 1 | Output valid (m_valid) |

## 15.5	Operation

### 15.5.1	Glue Logic

The glue logic consists of a single flop (r_drain_ip) that tracks when a FIFO drain is in progress:

// Drain FIFO when upstream has data AND we can accept
wire w_drain_fifo = s_valid && s_ready;

// Flop the drain signal to track data in flight
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_drain_ip <= 1'b0;
    end else begin
        r_drain_ip <= w_drain_fifo;
    end
end

// When r_drain_ip asserted, data arrived from FIFO � push to skid
assign skid_wr_valid = r_drain_ip;
assign skid_wr_data = s_data;

### 15.5.2	Skid Buffer

A 4-deep gaxi_fifo_sync instance with: - Non-registered mode (REGISTERED=0) for minimum latency - Auto memory style selection

### 15.5.3	Occupancy Calculation

// Total occupancy = data in skid buffer
// Note: r_drain_ip not counted separately (conservative)
assign occupancy = skid_count;

## 15.6	Timing Diagrams

### 15.6.1	Backpressure Handling

The following diagram shows how the latency bridge handles downstream backpressure:

#### 15.6.1.1	Waveform 2.10.1: Stream Latency Bridge Backpressure

Latency Bridge Backpressure

Source: latency_bridge_backpressure.json

## 15.7	Integration Example

stream_latency_bridge #(
    .DATA_WIDTH (512),
    .SKID_DEPTH (4)
) u_latency_bridge (
    .clk            (clk),
    .rst_n          (rst_n),

    // Slave (from FIFO)
    .s_data         (fifo_rd_data),
    .s_valid        (fifo_rd_valid),
    .s_ready        (fifo_rd_ready),

    // Master (to AXI Write Engine)
    .m_data         (axi_wr_sram_data),
    .m_valid        (axi_wr_sram_valid),
    .m_ready        (axi_wr_sram_ready),

    // Status
    .occupancy      (bridge_occupancy),

    // Debug
    .dbg_r_pending  (dbg_bridge_pending),
    .dbg_r_out_valid(dbg_bridge_out_valid)
);

## 15.8	Design Rationale

### 15.8.1	Why Use a Skid Buffer?

Previous designs attempted complex ready signal calculations to handle backpressure. This led to: - Timing issues from deep combinatorial paths - Edge cases with multi-cycle backpressure - Difficult debugging of flow control bugs

The skid buffer approach: - Simple glue logic (single flop) - FIFO handles all backpressure complexity - Proven, reusable component - Easy to verify and debug

### 15.8.2	Why 4-Deep?

- 2-deep: Minimum for back-to-back (1 in flight + 1 buffered)

- 4-deep: Allows 2-3 cycles of consumer stall without upstream backpressure

- 8-deep: Overkill for most use cases

4-deep provides good balance between: - Latency tolerance (handles brief stalls) - Resource usage (small FIFO) - Throughput (back-to-back capable)

## 15.9	Related Documentation

- Parent: 09_sram_controller_unit.md - Single-channel SRAM controller

- Consumer: 12_axi_write_engine.md - Receives bridge output

## Producer: 06_axi_read_engine.md - Feeds upstream FIFO

## 15.10	Revision History

Stream Latency Bridge Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Stream Drain Controller

Module: stream_drain_ctrl.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: sram_controller_unit.sv Status: Implemented Last Updated: 2025-11-30

## 16.1	Overview

The stream_drain_ctrl module is a virtual FIFO for write engine flow control. It tracks data availability using FIFO pointer logic without storing any data. This allows the write engine to pre-reserve data before issuing AXI write commands.

### 16.1.1	Key Features

- Virtual FIFO Pattern: Pointer arithmetic only, no data storage

- Pre-Reservation: Write engine reserves data before AXI AW command

- Variable Size Drains: Supports burst-sized reservations (not single beat)

- Registered Outputs: Optional output registration for timing

- Underflow Prevention: Ensures data exists before write engine commits

## 16.2	Architecture

### 16.2.1	Block Diagram

### 16.2.2	Figure 1: Stream Drain Controller Block Diagram

Stream Drain Controller Block Diagram

Source: 09_stream_drain_ctrl_block.mmd

### 16.2.3	Virtual FIFO Concept

Unlike a real FIFO that stores data, the drain controller only tracks pointers:

Real FIFO:           Virtual FIFO (Drain Controller):
      
 Data[0]            wr_ptr: 5     � Tracks entries written
 Data[1]            rd_ptr: 2     � Tracks entries drained
 Data[2]            available:3   � Difference
 ...               


Actual data stored in gaxi_fifo_sync (separate component)

### 16.2.4	Pointer Operations

Write Side (Data Entry):

When FIFO receives data:
  wr_ptr += 1
  data_available += 1

Read Side (Drain Request):

When write engine requests drain:
  rd_ptr += rd_size  (variable burst size!)
  data_available -= rd_size

## 16.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| DEPTH | int | 512 | Virtual FIFO depth |
| ALMOST_WR_MARGIN | int | 1 | Almost full margin |
| ALMOST_RD_MARGIN | int | 1 | Almost empty margin |
| REGISTERED | int | 1 | Registered outputs |

### 16.3.1	Derived Parameters

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| D | DEPTH | Short alias |
| AW | $clog2(D) | Address width |

## 16.4	Port List

### 16.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | System clock |
| axi_aresetn | input | 1 | Active-low asynchronous reset |

### 16.4.2	Write Interface (Data Entry)

Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wr_valid | input | 1 | Data written to FIFO |
| wr_ready | output | 1 | Not full (can accept) |

### 16.4.3	Read Interface (Drain Requests)

Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rd_valid | input | 1 | Request to drain data |
| rd_size | input | 8 | Number of entries to drain |
| rd_ready | output | 1 | Data available (not empty) |

### 16.4.4	Status Outputs

Status Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| data_available | output | AW+1 | Available data count |
| wr_full | output | 1 | Full (no space) |
| wr_almost_full | output | 1 | Almost full |
| rd_empty | output | 1 | Empty (no data) |
| rd_almost_empty | output | 1 | Almost empty |

## 16.5	Operation

### 16.5.1	Data Entry (Write Side)

Data entry is single-beat increments, tracking each beat written to the physical FIFO:

// Write pointer uses counter_bin for single-beat increments
counter_bin #(
    .WIDTH (AW + 1),
    .MAX   (D)
) write_pointer_inst (
    .clk              (axi_aclk),
    .rst_n            (axi_aresetn),
    .enable           (w_write && !r_wr_full),
    .counter_bin_curr (r_wr_ptr_bin),
    .counter_bin_next (w_wr_ptr_bin_next)
);

### 16.5.2	Drain Requests (Read Side)

Drain requests support variable-size bursts:

// Read pointer advances by rd_size on each drain request
always_ff @(posedge axi_aclk or negedge axi_aresetn) begin
    if (!axi_aresetn) begin
        r_rd_ptr_bin <= '0;
    end else begin
        if (w_read && !r_rd_empty) begin
            r_rd_ptr_bin <= r_rd_ptr_bin + (AW+1)'(rd_size);
        end
    end
end

### 16.5.3	Status Generation

Uses fifo_control block for full/empty/almost flags:

fifo_control #(
    .DEPTH             (D),
    .ADDR_WIDTH        (AW),
    .ALMOST_RD_MARGIN  (ALMOST_RD_MARGIN),
    .ALMOST_WR_MARGIN  (ALMOST_WR_MARGIN),
    .REGISTERED        (REGISTERED)
) fifo_control_inst (
    // ... pointer inputs
    .count            (w_count),
    .wr_full          (r_wr_full),
    .wr_almost_full   (r_wr_almost_full),
    .rd_empty         (r_rd_empty),
    .rd_almost_empty  (r_rd_almost_empty)
);

## 16.6	Comparison with stream_alloc_ctrl

Comparison with stream_alloc_ctrl

| Aspect | stream_alloc_ctrl | stream_drain_ctrl |
| --- | --- | --- |
| Purpose | Track space for reads | Track data for writes |
| Write Side | Reservation (burst) | Data entry (single beat) |
| Read Side | Fulfillment (single beat) | Drain request (burst) |
| Consumer | AXI Read Engine | AXI Write Engine |

Both use virtual FIFO pattern but with opposite semantics for read/write operations.

## 16.7	Integration Example

stream_drain_ctrl #(
    .DEPTH            (512),
    .REGISTERED       (1)
) u_drain_ctrl (
    .axi_aclk         (clk),
    .axi_aresetn      (rst_n),

    // Write interface: Connected to FIFO write handshake
    .wr_valid         (fifo_wr_valid && fifo_wr_ready),
    .wr_ready         (),  // Not used (tracks, doesn't control)

    // Read interface: Connected to write engine drain requests
    .rd_valid         (axi_wr_drain_req),
    .rd_size          (axi_wr_drain_size),
    .rd_ready         (),  // Not used (polling interface)

    // Status
    .data_available   (drain_data_available),
    .wr_full          (),
    .wr_almost_full   (),
    .rd_empty         (),
    .rd_almost_empty  ()
);

## 16.8	Common Issues

### 16.8.1	Issue 1: Data Available Undercount

Symptom: Write engine sees 0 available when FIFO has data

Root Causes: 1. wr_valid not connected to FIFO write handshake 2. Bridge occupancy not added to data_available

Solution: Ensure connection: wr_valid = fifo_wr_valid && fifo_wr_ready

### 16.8.2	Issue 2: Drain Request Overflow

Symptom: rd_ptr advances past wr_ptr

Root Causes: 1. Write engine requesting more than available 2. Multiple simultaneous drain requests

Solution: Write engine must check data_available >= requested_size before draining.

## 16.9	Related Documentation

- Parent: 09_sram_controller_unit.md - Integration context

- Counterpart: 07_stream_alloc_ctrl.md - Space tracking (read side)

- Consumer: 12_axi_write_engine.md - Uses drain interface

## Control Block: See fifo_control in rtl/amba/gaxi/

## 16.10	Revision History

Stream Drain Controller Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	AXI Write Engine

Module: axi_write_engine.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: stream_core.sv Status: Implemented Last Updated: 2025-11-30

## 17.1	Overview

The axi_write_engine module is a high-performance multi-channel AXI4 write engine with data-aware arbitration. It serves all 8 STREAM channels through a single AXI master interface with intelligent flow control.

### 17.1.1	Key Features

- Round-Robin Arbitration: Fair scheduling across channels

- Data-Aware Masking: Only arbitrate channels with sufficient SRAM data

- Pre-Drain Handshake: Reserve SRAM data before AW command issues

- FIFO-Based W Tracking: Single shared FIFO for W-phase order preservation

- Per-Channel B Tracking: Separate FIFOs for out-of-order B responses

- Channel ID in AXI ID: Enables per-channel response routing

- Pipelined/Non-Pipelined Modes: Configurable outstanding transaction depth

## 17.2	Architecture

### 17.2.1	Block Diagram

### 17.2.2	Figure 2.12.1: AXI Write Engine Block Diagram

AXI Write Engine Block Diagram

Source: 10_axi_write_engine_block.mmd

### 17.2.3	Operation Flow

1. Scheduler Interface: Each channel can request write bursts
2. Data Checking: Mask channels without sufficient SRAM data
3. Arbitration: Round-robin arbiter selects next channel to service
4. AXI AW Issue: Issue write command to AXI, assert wr_drain to reserve data
5. W-Phase FIFO: Push transaction to shared FIFO (preserves AW order)
6. AXI W Stream: Stream write data from SRAM controller (ID-based mux)
7. AXI B Response: Pop per-channel B-phase FIFO, complete transaction

### 17.2.4	Key Design Decisions

FIFO-Based W Tracking (No FSM): Instead of a state machine, uses a single shared FIFO to track pending W-phase transactions. This preserves the AW command order which is critical for W-phase correctness.

Separate W and B Phase FIFOs: - W-Phase FIFO: Single shared FIFO (in-order with AW) - B-Phase FIFOs: Per-channel FIFOs (out-of-order responses allowed by AXI4)

ID-Based SRAM Interface: The write engine drives axi_wr_sram_id to select which channel’s data to drain. The SRAM controller muxes the appropriate channel’s data to axi_wr_sram_data.

## 17.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | Number of channels |
| ADDR_WIDTH | int | 64 | AXI address width |
| DATA_WIDTH | int | 512 | AXI data width |
| ID_WIDTH | int | 8 | AXI ID width |
| USER_WIDTH | int | 8 | AXI USER width |
| SEG_COUNT_WIDTH | int | 8 | Width of space/count signals |
| PIPELINE | int | 0 | 0: non-pipelined, 1: pipelined |
| AW_MAX_OUTSTANDING | int | 8 | Maximum outstanding AW requests (PIPELINE=1) |
| W_PHASE_FIFO_DEPTH | int | 64 | W-phase transaction FIFO depth |
| B_PHASE_FIFO_DEPTH | int | 16 | B-phase transaction FIFO depth per channel |

### 17.3.1	Derived Parameters

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| NC | NUM_CHANNELS | Short alias |
| AW | ADDR_WIDTH | Short alias |
| DW | DATA_WIDTH | Short alias |
| IW | ID_WIDTH | Short alias |
| UW | USER_WIDTH | Short alias |
| SCW | SEG_COUNT_WIDTH | Segment count width |
| CIW | $clog2(NC) | Channel ID width (min 1 bit) |
| AXSIZE | $clog2(DW/8) | AXI burst size |

## 17.4	Port List

### 17.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 17.4.2	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_wr_xfer_beats | input | 8 | Transfer size in beats (all channels) |

### 17.4.3	Scheduler Interface (Per-Channel)

Scheduler Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_valid[ch] | input | NC | Channel requests write |
| sched_wr_ready[ch] | output | NC | Engine ready (descriptor complete) |
| sched_wr_addr[ch] | input | NC x AW | Destination addresses |
| sched_wr_beats[ch] | input | NC x 32 | Beats remaining to write |
| sched_wr_burst_len[ch] | input | NC x 8 | Requested burst length |

### 17.4.4	Completion Interface (Per-Channel)

Completion Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_done_strobe[ch] | output | NC | Burst completed (1 cycle pulse) |
| sched_wr_beats_done[ch] | output | NC x 32 | Number of beats completed |

### 17.4.5	SRAM Drain Interface

SRAM Drain Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_drain_req[ch] | output | NC | Request to reserve data |
| axi_wr_drain_size[ch] | output | NC x 8 | Beats to reserve |
| axi_wr_drain_data_avail[ch] | input | NC x SCW | Data available per channel |

### 17.4.6	SRAM Read Interface

SRAM Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_wr_sram_valid[ch] | input | NC | Per-channel valid |
| axi_wr_sram_drain | output | 1 | Drain request |
| axi_wr_sram_id | output | CIW | Channel ID select |
| axi_wr_sram_data | input | DW | Muxed data from selected channel |

### 17.4.7	AXI4 AW Channel

AXI4 AW Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_awvalid | output | 1 | Address valid |
| m_axi_awready | input | 1 | Address ready |
| m_axi_awid | output | IW | Transaction ID |
| m_axi_awaddr | output | AW | Address |
| m_axi_awlen | output | 8 | Burst length - 1 |
| m_axi_awsize | output | 3 | Burst size (log2 bytes) |
| m_axi_awburst | output | 2 | Burst type (INCR) |

### 17.4.8	AXI4 W Channel

AXI4 W Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_wvalid | output | 1 | Write data valid |
| m_axi_wready | input | 1 | Write data ready |
| m_axi_wdata | output | DW | Write data |
| m_axi_wstrb | output | DW/8 | Write strobes |
| m_axi_wlast | output | 1 | Last beat of burst |
| m_axi_wuser | output | UW | Channel ID for tracking |

### 17.4.9	AXI4 B Channel

AXI4 B Channel

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_bvalid | input | 1 | Response valid |
| m_axi_bready | output | 1 | Response ready |
| m_axi_bid | input | IW | Transaction ID |
| m_axi_bresp | input | 2 | Response |

### 17.4.10	Error Interface

Error Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| sched_wr_error[ch] | output | NC | Sticky error flag per channel |

### 17.4.11	Debug Interface

Debug Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dbg_wr_all_complete[ch] | output | NC | All writes complete |
| dbg_aw_transactions | output | 32 | Total AW transactions issued |
| dbg_w_beats | output | 32 | Total W beats written to AXI |

## 17.5	Operation

### 17.5.1	Data-Aware Request Masking

// Only request arbitration if:
// 1. Scheduler is requesting (sched_wr_valid)
// 2. Sufficient SRAM data available (w_data_ok)
// 3. Below outstanding limit (w_no_outstanding)
w_arb_request[i] = sched_wr_valid[i] && w_data_ok[i] && w_no_outstanding[i];

// Data check includes final burst handling
w_data_ok[i] = w_has_data[i] || w_final_burst[i];

### 17.5.2	W-Phase Transaction FIFO

Single shared FIFO preserves AW command order:

typedef struct packed {
    logic [7:0]    beats;       // Number of beats for this W transaction
    logic [CIW-1:0] channel_id;  // Channel ID for this transaction
} w_phase_txn_t;

// Push on AW handshake (preserves order)
if (m_axi_awvalid && m_axi_awready) begin
    w_phase_txn_fifo_wr = 1'b1;
    w_phase_txn_fifo_din.beats = m_axi_awlen + 8'd1;
    w_phase_txn_fifo_din.channel_id = r_aw_channel_id;
end

### 17.5.3	B-Phase Transaction FIFOs

Per-channel FIFOs track beats and last flag for completion:

typedef struct packed {
    logic [7:0]  beats;     // Number of beats in this transaction
    logic        last;      // Is this the last transfer for descriptor?
} b_phase_txn_t;

// last flag determines when sched_wr_ready asserts
if (m_axi_bvalid && m_axi_bready) begin
    if (b_phase_txn_fifo_dout[ch_id].last) begin
        r_sched_ready[ch_id] <= 1'b1;
    end
end

### 17.5.4	Outstanding Transaction Tracking

PIPELINE=0 (Non-Pipelined): - Boolean flag per channel (0 or 1 outstanding) - Set when AW issues, clear when B arrives

PIPELINE=1 (Pipelined): - Counter per channel (0 to AW_MAX_OUTSTANDING) - Increment on AW, decrement on B

## 17.6	Timing Diagrams

### 17.6.1	Perfect Streaming - AXI Write Transaction

The following timing diagram shows the AXI write engine operating at maximum throughput with perfect streaming - where both wvalid and wready remain HIGH for consecutive clock cycles, achieving one data beat per cycle.

#### 17.6.1.1	Waveform 2.12.1: AXI Write Engine - Perfect Streaming

AXI Write Engine - Perfect Streaming

Transaction Flow:

- AW Channel Handshake (Address Phase)

  - m_axi_awvalid rises to initiate a write request

  - m_axi_awready is already HIGH (slave ready)

  - Address (awaddr), ID (awid), and length (awlen=7 for 8 beats) are captured

  - Handshake completes in a single cycle when both valid and ready are HIGH

  - Simultaneously, axi_wr_drain_req reserves data from SRAM controller

- W Channel Streaming (Data Phase)

  - After AW acceptance, the engine begins streaming data from SRAM

  - m_axi_wvalid rises and stays HIGH for all 8 beats

  - m_axi_wready remains HIGH throughout (no backpressure from AXI slave)

  - Perfect streaming: One data beat transferred every clock cycle

  - m_axi_wlast rises on the final beat (beat 7) to mark burst completion

  - wstrb is all-ones (0xFF) indicating full data width valid

- B Channel Response (Completion Phase)

  - After wlast, the AXI slave returns a write response

  - m_axi_bvalid rises with bid matching the transaction ID

  - m_axi_bready is HIGH (engine always ready for responses)

  - bresp=OKAY indicates successful write

  - Response triggers sched_wr_done_strobe to notify scheduler

- Next Transaction

  - After B response, the engine can immediately issue the next AW request

  - The cycle repeats for the next channel or next burst

Key Performance Indicators: - No bubbles: wvalid and wready both HIGH during data phase - Full bandwidth: Data width (256/512 bits) x clock frequency - Zero-wait states: SRAM controller provides data at line rate - Minimal latency: B response arrives shortly after wlast

### 17.6.2	Multi-Channel Streaming

For multi-channel operation showing channel switching while maintaining streaming performance, see:

#### 17.6.2.1	Waveform 2.12.2: Datapath Write - Multi-Channel

Datapath Write - Multi-Channel

This diagram shows how the engine arbitrates between channels while maintaining high throughput. Note how: - Different channel IDs (awid) appear in sequence - W-phase FIFO preserves AW order for correct data association - B responses can arrive out-of-order (per AXI4 spec)

## 17.7	Integration Example

axi_write_engine #(
    .NUM_CHANNELS       (8),
    .ADDR_WIDTH         (64),
    .DATA_WIDTH         (512),
    .ID_WIDTH           (8),
    .SEG_COUNT_WIDTH    (10),
    .PIPELINE           (0),
    .AW_MAX_OUTSTANDING (8),
    .W_PHASE_FIFO_DEPTH (64),
    .B_PHASE_FIFO_DEPTH (16)
) u_axi_write_engine (
    .clk                    (clk),
    .rst_n                  (rst_n),

    // Configuration
    .cfg_axi_wr_xfer_beats  (cfg_axi_wr_xfer_beats),

    // Scheduler interface
    .sched_wr_valid         (sched_wr_valid),
    .sched_wr_ready         (sched_wr_ready),
    .sched_wr_addr          (sched_wr_addr),
    .sched_wr_beats         (sched_wr_beats),
    .sched_wr_burst_len     (sched_wr_burst_len),

    // Completion interface
    .sched_wr_done_strobe   (sched_wr_done_strobe),
    .sched_wr_beats_done    (sched_wr_beats_done),

    // SRAM drain interface
    .axi_wr_drain_req       (axi_wr_drain_req),
    .axi_wr_drain_size      (axi_wr_drain_size),
    .axi_wr_drain_data_avail(axi_wr_drain_data_avail),

    // SRAM read interface
    .axi_wr_sram_valid      (axi_wr_sram_valid),
    .axi_wr_sram_drain      (axi_wr_sram_drain),
    .axi_wr_sram_id         (axi_wr_sram_id),
    .axi_wr_sram_data       (axi_wr_sram_data),

    // AXI master
    .m_axi_awvalid          (m_axi_wr_awvalid),
    .m_axi_awready          (m_axi_wr_awready),
    .m_axi_awid             (m_axi_wr_awid),
    .m_axi_awaddr           (m_axi_wr_awaddr),
    .m_axi_awlen            (m_axi_wr_awlen),
    .m_axi_awsize           (m_axi_wr_awsize),
    .m_axi_awburst          (m_axi_wr_awburst),

    .m_axi_wvalid           (m_axi_wr_wvalid),
    .m_axi_wready           (m_axi_wr_wready),
    .m_axi_wdata            (m_axi_wr_wdata),
    .m_axi_wstrb            (m_axi_wr_wstrb),
    .m_axi_wlast            (m_axi_wr_wlast),
    .m_axi_wuser            (m_axi_wr_wuser),

    .m_axi_bvalid           (m_axi_wr_bvalid),
    .m_axi_bready           (m_axi_wr_bready),
    .m_axi_bid              (m_axi_wr_bid),
    .m_axi_bresp            (m_axi_wr_bresp),

    // Error and debug
    .sched_wr_error         (sched_wr_error),
    .dbg_wr_all_complete    (dbg_wr_all_complete),
    .dbg_aw_transactions    (dbg_aw_transactions),
    .dbg_w_beats            (dbg_w_beats)
);

## 17.8	Related Documentation

- Parent: 01_stream_core.md - Top-level integration

- Scheduler Array: 02_scheduler_group_array.md - Provides sched_wr_* signals

- SRAM Controller: 08_sram_controller.md - Data source

- Drain Controller: 11_stream_drain_ctrl.md - Data availability tracking

## Read Engine: 06_axi_read_engine.md - Complementary read datapath

## 17.9	Revision History

AXI Write Engine Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	APB to Descriptor Router

Module: apbtodescr.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: stream_top.sv Status: Implemented Last Updated: 2025-11-30

## 18.1	Overview

The apbtodescr module is an APB-to-descriptor engine router that handles address-based routing of APB writes to descriptor engine kick-off ports. It decodes the APB address to determine the target channel and routes the 64-bit descriptor address using a two-write sequence.

### 18.1.1	Key Features

- Address-Based Routing: APB address bits select target channel

- 64-bit Address Assembly: Two 32-bit APB writes combine into 64-bit descriptor address

- Backpressure Handling: Delays APB response if descriptor engine busy

- Address Range Checking: Error on out-of-range addresses

- Integration Signal: apb_descriptor_kickoff_hit for response muxing

## 18.2	Architecture

### 18.2.1	Address Map

Relative to BASE_ADDR:
  BASE + 0x00: Channel 0 descriptor address LOW  [31:0]
  BASE + 0x04: Channel 0 descriptor address HIGH [63:32]
  BASE + 0x08: Channel 1 descriptor address LOW  [31:0]
  BASE + 0x0C: Channel 1 descriptor address HIGH [63:32]
  ...
  BASE + 0x38: Channel 7 descriptor address LOW  [31:0]
  BASE + 0x3C: Channel 7 descriptor address HIGH [63:32]

### 18.2.2	Block Diagram

### 18.2.3	Figure 2.13.1: APB to Descriptor Block Diagram

APB to Descriptor Block Diagram

Source: 02_apbtodescr_block.mmd

### 18.2.4	FSM Diagram

### 18.2.5	Figure 2.13.2: APB to Descriptor FSM

APB to Descriptor FSM

Source: 02_apbtodescr_fsm.mmd

### 18.2.6	Write Flow

1. Software writes LOW word to CHx_CTRL_LOW register
2. PeakRDL APB slave presents write on cmd/rsp interface
3. Module captures LOW word, responds with OKAY
4. Software writes HIGH word to CHx_CTRL_HIGH register
5. Module assembles 64-bit address from LOW + HIGH
6. Module asserts desc_apb_valid[channel_id]
7. Module drives desc_apb_addr[channel_id]
8. Waits for desc_apb_ready[channel_id]
9. Completes APB transaction (asserts apb_rsp_valid)

### 18.2.7	FSM States

typedef enum logic [2:0] {
    IDLE          = 3'b000,    // Waiting for APB command (LOW write)
    RESPOND_LOW   = 3'b001,    // Sending response after LOW write
    WAIT_HIGH     = 3'b010,    // Waiting for HIGH write
    ROUTE         = 3'b011,    // Routing to descriptor engine
    RESPOND_HIGH  = 3'b100     // Sending final response after HIGH write
} state_t;

## 18.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | APB address width |
| DATA_WIDTH | int | 32 | APB data width |
| NUM_CHANNELS | int | 8 | Number of DMA channels |

## 18.4	Port List

### 18.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 18.4.2	APB Slave CMD Interface

APB Slave CMD Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_cmd_valid | input | 1 | Command valid |
| apb_cmd_ready | output | 1 | Ready to accept command |
| apb_cmd_addr | input | ADDR_WIDTH | Command address |
| apb_cmd_wdata | input | DATA_WIDTH | Write data |
| apb_cmd_write | input | 1 | Write enable (1=write, 0=read) |

### 18.4.3	APB Slave RSP Interface

APB Slave RSP Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_rsp_valid | output | 1 | Response valid |
| apb_rsp_ready | input | 1 | Response ready |
| apb_rsp_rdata | output | DATA_WIDTH | Read data (always 0) |
| apb_rsp_error | output | 1 | Error flag |

### 18.4.4	Descriptor Engine APB Ports

Descriptor Engine APB Ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| desc_apb_valid[ch] | output | NUM_CHANNELS | Per-channel valid |
| desc_apb_ready[ch] | input | NUM_CHANNELS | Per-channel ready |
| desc_apb_addr[ch] | output | NUM_CHANNELS x 64 | 64-bit descriptor addresses |

### 18.4.5	Integration Control

Integration Control

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| apb_descriptor_kickoff_hit | output | 1 | Indicates kick-off in progress |

## 18.5	Operation

### 18.5.1	Address Decode

// Extract channel ID from address (dword-aligned: addr[5:3])
// Address bits [1:0] are ignored (word-aligned)
// Address bit  [2]   selects LOW (0) or HIGH (1) register
// Address bits [5:3] select channel 0-7
assign channel_id = apb_cmd_addr[5:3];
assign r_is_high_write = apb_cmd_addr[2];

// Valid range: 0x00 to 0x3F (8 channels x 8 bytes)
assign addr_in_range = ({20'h0, apb_cmd_addr[11:0]} < (NUM_CHANNELS * 8));

### 18.5.2	64-bit Address Assembly

// LOW write captured in IDLE state
if (r_state == IDLE && apb_cmd_valid) begin
    r_wdata_low <= apb_cmd_wdata;
end

// HIGH write captured in WAIT_HIGH state
if (r_state == WAIT_HIGH && apb_cmd_valid) begin
    r_wdata_high <= apb_cmd_wdata;
end

// Assembled 64-bit address
desc_apb_addr[ch] = {r_wdata_high, r_wdata_low};

### 18.5.3	Error Conditions

- Read Operation: APB reads not supported

- Address Out of Range: Beyond channel address space

- HIGH Before LOW: HIGH write without preceding LOW write

- Wrong Channel: HIGH write to different channel than LOW

## 18.6	Timing Diagrams

### 18.6.1	Normal Write Sequence

The following diagram shows the normal write flow where a LOW word followed by HIGH word kicks off a channel:

#### 18.6.1.1	Waveform 2.13.1: APB Normal Write Sequence

APB Normal Write

Source: apbtodescr_normal_write.json

### 18.6.2	Backpressure Handling

When the descriptor engine is busy, the APB response is delayed until the engine accepts the kick-off:

#### 18.6.2.1	Waveform 2.13.2: APB Write with Backpressure

APB Backpressure Write

Source: apbtodescr_backpressure_write.json

### 18.6.3	Channel Kick-off Examples

#### 18.6.3.1	Waveform 2.13.3: Channel 0 Kick-off

Channel 0 Kick-off

Source: apbtodescr_ch0_kickoff.json

#### 18.6.3.2	Waveform 2.13.4: Channel 7 Kick-off

Channel 7 Kick-off

Source: apbtodescr_ch7_kickoff.json

### 18.6.4	Multi-Channel Operation

The following diagram shows multiple channels being kicked off in sequence:

#### 18.6.4.1	Waveform 2.13.5: Multi-Channel Kick-off Sequence

Multi-Channel Kick-off

Source: apbtodescr_multi_channel.json

## 18.7	Integration Example

apbtodescr #(
    .ADDR_WIDTH     (32),
    .DATA_WIDTH     (32),
    .NUM_CHANNELS   (8)
) u_apbtodescr (
    .clk                        (clk),
    .rst_n                      (rst_n),

    // APB CMD/RSP from PeakRDL
    .apb_cmd_valid              (apb_kickoff_cmd_valid),
    .apb_cmd_ready              (apb_kickoff_cmd_ready),
    .apb_cmd_addr               (apb_kickoff_cmd_addr),
    .apb_cmd_wdata              (apb_kickoff_cmd_wdata),
    .apb_cmd_write              (apb_kickoff_cmd_write),

    .apb_rsp_valid              (apb_kickoff_rsp_valid),
    .apb_rsp_ready              (apb_kickoff_rsp_ready),
    .apb_rsp_rdata              (apb_kickoff_rsp_rdata),
    .apb_rsp_error              (apb_kickoff_rsp_error),

    // To descriptor engines
    .desc_apb_valid             (desc_apb_valid),
    .desc_apb_ready             (desc_apb_ready),
    .desc_apb_addr              (desc_apb_addr),

    // Integration
    .apb_descriptor_kickoff_hit (kickoff_hit)
);

## 18.8	Common Issues

### 18.8.1	Issue 1: APB Response Not Returning

Symptom: APB write hangs without response

Root Causes: 1. Descriptor engine not asserting ready 2. FSM stuck in ROUTE state 3. apb_rsp_ready not being asserted

Solution: Check descriptor engine acceptance and FSM state.

### 18.8.2	Issue 2: Wrong Channel Kicked Off

Symptom: Different channel starts than expected

Root Cause: HIGH write sent to different channel than LOW

Solution: Ensure software writes LOW then HIGH to same channel address.

## 18.9	Related Documentation

- Parent: 01_stream_core.md - Top-level integration

- Consumer: 05_descriptor_engine.md - Receives kick-off

## Register File: PeakRDL-generated stream_regs.sv

## 18.10	Revision History

APB to Descriptor Router Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	APB Configuration Block

Module: stream_config_block.sv Location: projects/components/stream/rtl/top/ Category: TOP (Integration) Parent: stream_top.sv Status: Implemented Last Updated: 2025-11-30

## 19.1	Overview

The stream_config_block module maps PeakRDL-generated register outputs to STREAM core configuration inputs. It provides a clean interface layer between the register file and the STREAM DMA engine core, handling field extraction, width conversion, and global enable gating.

### 19.1.1	Key Features

- Field Extraction: Extracts fields from packed register values

- Width Conversion: Converts register fields to configuration signal widths

- Global Enable Gating: Gates enables by global enable register

- Address Extension: Zero-extends 32-bit register addresses to 64-bit

- Clean Interface: Isolates register naming from core logic

## 19.2	Architecture

### 19.2.1	Block Diagram

### 19.2.2	Figure 1: Stream Config Block Diagram

Stream Config Block Diagram

Source: 14_stream_config_block.mmd

### 19.2.3	Configuration Groups

stream_config_block
 Global and Channel Control
    cfg_channel_enable (gated by global_en)
    cfg_channel_reset (OR'd with global_rst)
 Scheduler Configuration
    cfg_sched_enable (gated by global_en)
    cfg_sched_timeout_cycles
    cfg_sched_*_enable flags
 Descriptor Engine Configuration
    cfg_desceng_enable (gated by global_en)
    cfg_desceng_prefetch
    cfg_desceng_addr*_base/limit (zero-extended)
 Monitor Configurations (3 sets)
    Descriptor AXI Monitor (cfg_desc_mon_*)
    Read Engine Monitor (cfg_rdeng_mon_*)
    Write Engine Monitor (cfg_wreng_mon_*)
 AXI Transfer Configuration
    cfg_axi_rd_xfer_beats
    cfg_axi_wr_xfer_beats
 Performance Profiler
     cfg_perf_enable (gated by global_en)
     cfg_perf_mode
     cfg_perf_clear

## 19.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | Number of DMA channels |
| ADDR_WIDTH | int | 64 | Address width for config outputs |

## 19.4	Port List

### 19.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 19.4.2	PeakRDL Register Inputs

Global Control:

PeakRDL Register Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| reg_global_ctrl_global_en | input | 1 | Global enable |
| reg_global_ctrl_global_rst | input | 1 | Global reset |
| reg_channel_enable_ch_en | input | 8 | Per-channel enable bits |
| reg_channel_reset_ch_rst | input | 8 | Per-channel reset bits |

Scheduler Configuration:

PeakRDL Register Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| reg_sched_timeout_cycles_* | input | 16 | Timeout threshold |
| reg_sched_config_* | input | 1 | Various enables |

Descriptor Engine Configuration:

PeakRDL Register Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| reg_desceng_config_* | input | varies | Enable and threshold |
| reg_desceng_addr*_* | input | 32 | Address range limits |

Monitor Configurations (3 sets):

Each monitor (DAXMON, RDMON, WRMON) has: - reg_*_enable_* - Enable flags - reg_*_timeout_* - Timeout configuration - reg_*_latency_thresh_* - Latency threshold - reg_*_pkt_mask_* - Packet type mask - reg_*_err_cfg_* - Error configuration - reg_*_mask*_* - Event masks

AXI Transfer Configuration:

PeakRDL Register Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| reg_axi_xfer_config_rd_xfer_beats | input | 8 | Read burst size |
| reg_axi_xfer_config_wr_xfer_beats | input | 8 | Write burst size |

Performance Profiler:

PeakRDL Register Inputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| reg_perf_config_perf_en | input | 1 | Profiler enable |
| reg_perf_config_perf_mode | input | 1 | Profiling mode |
| reg_perf_config_perf_clear | input | 1 | Clear profiler |

### 19.4.3	Configuration Outputs

Global and Channel:

Configuration Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_channel_enable | output | NUM_CHANNELS | Gated channel enables |
| cfg_channel_reset | output | NUM_CHANNELS | Combined channel resets |

Scheduler:

Configuration Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_sched_enable | output | 1 | Gated scheduler enable |
| cfg_sched_timeout_cycles | output | 16 | Timeout value |
| cfg_sched_*_enable | output | 1 | Feature enables |

Descriptor Engine:

Configuration Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_desceng_enable | output | 1 | Gated desceng enable |
| cfg_desceng_prefetch | output | 1 | Prefetch enable |
| cfg_desceng_fifo_thresh | output | 4 | FIFO threshold |
| cfg_desceng_addr*_base | output | ADDR_WIDTH | Zero-extended address |
| cfg_desceng_addr*_limit | output | ADDR_WIDTH | Zero-extended address |

Monitor Outputs (3 sets):

Each monitor outputs ~16 configuration signals covering: - Enable flags - Timeout values - Thresholds - Masks

AXI Transfer:

Configuration Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_rd_xfer_beats | output | 8 | Read burst size |
| cfg_axi_wr_xfer_beats | output | 8 | Write burst size |

Performance Profiler:

Configuration Outputs

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_perf_enable | output | 1 | Gated profiler enable |
| cfg_perf_mode | output | 1 | Profiling mode |
| cfg_perf_clear | output | 1 | Clear signal |

## 19.5	Operation

### 19.5.1	Global Enable Gating

// Gate all channel enables by global enable
assign cfg_channel_enable = reg_channel_enable_ch_en & {NUM_CHANNELS{reg_global_ctrl_global_en}};

// Gate scheduler enable
assign cfg_sched_enable = reg_sched_config_sched_en & reg_global_ctrl_global_en;

// Gate monitor enables
assign cfg_desc_mon_enable = reg_daxmon_enable_mon_en & reg_global_ctrl_global_en;

### 19.5.2	Global Reset OR

// Channel resets are OR'd with global reset
assign cfg_channel_reset = reg_channel_reset_ch_rst | {NUM_CHANNELS{reg_global_ctrl_global_rst}};

### 19.5.3	Address Zero Extension

// Zero-extend 32-bit register addresses to ADDR_WIDTH (typically 64-bit)
assign cfg_desceng_addr0_base = {{(ADDR_WIDTH-32){1'b0}}, reg_desceng_addr0_base_addr0_base};
assign cfg_desceng_addr0_limit = {{(ADDR_WIDTH-32){1'b0}}, reg_desceng_addr0_limit_addr0_limit};

## 19.6	Integration Example

stream_config_block #(
    .NUM_CHANNELS   (8),
    .ADDR_WIDTH     (64)
) u_config_block (
    .clk            (clk),
    .rst_n          (rst_n),

    // From PeakRDL register file
    .reg_global_ctrl_global_en      (regs_global_ctrl_global_en),
    .reg_global_ctrl_global_rst     (regs_global_ctrl_global_rst),
    .reg_channel_enable_ch_en       (regs_channel_enable_ch_en),
    // ... more register inputs

    // To stream_core
    .cfg_channel_enable             (cfg_channel_enable),
    .cfg_channel_reset              (cfg_channel_reset),
    .cfg_sched_enable               (cfg_sched_enable),
    // ... more config outputs
);

## 19.7	Related Documentation

- Parent: 01_stream_core.md - Receives configuration signals

- Register File: PeakRDL-generated stream_regs.sv

- Scheduler: 04_scheduler.md - Uses sched_* config

- Descriptor Engine: 05_descriptor_engine.md - Uses desceng_* config

## Monitors: Monitor documentation for each AXI monitor

## 19.8	Revision History

APB Configuration Block Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Performance Profiler

Module: perf_profiler.sv Location: projects/components/stream/rtl/fub/ Category: FUB (Functional Unit Block) Parent: stream_top.sv Status: Implemented Last Updated: 2025-11-30

## 20.1	Overview

The perf_profiler module captures timing information for channel activity to enable performance analysis and bottleneck identification. It monitors channel idle signals and records either timestamps or elapsed times to a FIFO for software retrieval.

### 20.1.1	Key Features

- Two Profiling Modes:

  - Mode 0 (TIMESTAMP): Captures timestamps on idle signal edges

  - Mode 1 (ELAPSED): Captures elapsed time directly

- Per-Channel Profiling: Tracks all 8 channels independently

- 256-Entry FIFO: Buffers performance data for software polling

- Channel ID Tagging: Each entry includes source channel

- Configurable via APB: Enable, mode, and clear controls

## 20.2	Architecture

### 20.2.1	Block Diagram

### 20.2.2	Figure 2.15.1: Performance Profiler Block Diagram

Performance Profiler Block Diagram

Source: 15_perf_profiler_block.mmd

### 20.2.3	Profiling Modes

Mode 0 - TIMESTAMP:

Records timestamp when:
  - Channel transitions idle � active (start)
  - Channel transitions active � idle (end)

Software calculates: elapsed = timestamp_end - timestamp_start

Advantages:
  - Simpler hardware
  - More flexible analysis
  - Can detect partial transfers

Mode 1 - ELAPSED:

Records elapsed time when:
  - Channel returns to idle

elapsed_time = current_timestamp - start_timestamp

Advantages:
  - Hardware calculates duration
  - Simpler software processing
  - Direct performance metric

### 20.2.4	FIFO Entry Format (36-bit)

FIFO Entry Format

| Bits | Field | Description |
| --- | --- | --- |
| [31:0] | timestamp/elapsed | Timestamp or elapsed time |
| [34:32] | channel_id | Source channel (0-7) |
| [35] | event_type | 0=start, 1=end (timestamp mode) |

## 20.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| NUM_CHANNELS | int | 8 | Number of channels to monitor |
| CHANNEL_WIDTH | int | $clog2(NUM_CHANNELS) | Channel ID width |
| TIMESTAMP_WIDTH | int | 32 | Timestamp counter width |
| FIFO_DEPTH | int | 256 | Performance FIFO depth |
| FIFO_ADDR_WIDTH | int | $clog2(FIFO_DEPTH) | FIFO address width |

## 20.4	Port List

### 20.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | input | 1 | System clock |
| rst_n | input | 1 | Active-low asynchronous reset |

### 20.4.2	Channel Monitoring

Channel Monitoring

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| channel_idle | input | NUM_CHANNELS | Per-channel idle signals |

### 20.4.3	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_enable | input | 1 | Enable profiling |
| cfg_mode | input | 1 | 0=timestamp, 1=elapsed |
| cfg_clear | input | 1 | Clear FIFO and counters |

### 20.4.4	FIFO Read Interface

FIFO Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| perf_fifo_rd | input | 1 | Read strobe (pops FIFO) |
| perf_fifo_data_low | output | 32 | Timestamp/elapsed [31:0] |
| perf_fifo_data_high | output | 32 | {28’b0, event_type, channel_id} |
| perf_fifo_empty | output | 1 | FIFO empty flag |
| perf_fifo_full | output | 1 | FIFO full flag |
| perf_fifo_count | output | 16 | Number of entries |

## 20.5	Operation

### 20.5.1	Edge Detection

// Detect idle signal transitions
assign w_idle_rising  = channel_idle & ~r_idle_prev;  // Active � Idle (end)
assign w_idle_falling = ~channel_idle & r_idle_prev;  // Idle � Active (start)

### 20.5.2	Timestamp Counter

// Free-running 32-bit counter
// Increments every cycle when profiling enabled
// Wraps at 2^32 - 1 (software handles rollover)
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_timestamp_counter <= '0;
    end else if (cfg_clear) begin
        r_timestamp_counter <= '0;
    end else if (cfg_enable) begin
        r_timestamp_counter <= r_timestamp_counter + 1'b1;
    end
end

### 20.5.3	Start Time Capture (Elapsed Mode)

// Per-channel start time tracking
generate
    for (ch = 0; ch < NUM_CHANNELS; ch++) begin
        always_ff @(posedge clk) begin
            if (cfg_enable && cfg_mode == MODE_ELAPSED) begin
                if (w_idle_falling[ch]) begin
                    r_start_time[ch] <= r_timestamp_counter;
                    r_channel_active[ch] <= 1'b1;
                end
                else if (w_idle_rising[ch]) begin
                    r_channel_active[ch] <= 1'b0;
                end
            end
        end
    end
endgenerate

### 20.5.4	Two-Register Read Sequence

Software reads FIFO via APB:
1. Check perf_fifo_empty == 0
2. Read PERF_FIFO_DATA_LOW:
   - Triggers perf_fifo_rd strobe
   - Pops FIFO, latches 36-bit entry
   - Returns timestamp/elapsed [31:0]
3. Read PERF_FIFO_DATA_HIGH:
   - Returns {28'b0, event_type, channel_id}
   - No FIFO pop
4. Parse data and repeat

## 20.6	Integration Example

perf_profiler #(
    .NUM_CHANNELS       (8),
    .TIMESTAMP_WIDTH    (32),
    .FIFO_DEPTH         (256)
) u_perf_profiler (
    .clk                (clk),
    .rst_n              (rst_n),

    // From schedulers
    .channel_idle       (scheduler_idle),

    // Configuration
    .cfg_enable         (cfg_perf_enable),
    .cfg_mode           (cfg_perf_mode),
    .cfg_clear          (cfg_perf_clear),

    // FIFO read interface (to APB)
    .perf_fifo_rd       (perf_fifo_rd),
    .perf_fifo_data_low (perf_fifo_data_low),
    .perf_fifo_data_high(perf_fifo_data_high),
    .perf_fifo_empty    (perf_fifo_empty),
    .perf_fifo_full     (perf_fifo_full),
    .perf_fifo_count    (perf_fifo_count)
);

## 20.7	Software Usage

### 20.7.1	Example: Measure Channel 0 Transfer Time

// Enable timestamp mode
write_reg(PERF_CONFIG, PERF_EN | MODE_TIMESTAMP);

// Start transfer on channel 0
write_reg(CH0_CTRL_LOW, desc_addr_low);
write_reg(CH0_CTRL_HIGH, desc_addr_high);

// Wait for completion
while (!read_reg(CH0_STATUS) & COMPLETE);

// Read performance data
while (!(read_reg(PERF_STATUS) & FIFO_EMPTY)) {
    uint32_t low  = read_reg(PERF_FIFO_DATA_LOW);   // Pops FIFO
    uint32_t high = read_reg(PERF_FIFO_DATA_HIGH);  // Reads latched

    uint32_t timestamp = low;
    uint8_t  channel   = high & 0x7;
    uint8_t  event     = (high >> 3) & 0x1;

    if (channel == 0) {
        if (event == 0)  // Start
            start_time = timestamp;
        else             // End
            elapsed = timestamp - start_time;
    }
}

## 20.8	Common Issues

### 20.8.1	Issue 1: FIFO Overflow

Symptom: Performance data lost

Root Cause: Software not polling fast enough

Solution: - Poll more frequently - Use larger FIFO depth - Consider interrupt on half-full

### 20.8.2	Issue 2: Timestamp Rollover

Symptom: Negative elapsed times calculated

Root Cause: 32-bit counter wrapped during measurement

Solution: Handle rollover in software:

if (end_time < start_time)
    elapsed = (0xFFFFFFFF - start_time) + end_time + 1;

## 20.9	Related Documentation

- Parent: 01_stream_core.md - Top-level integration

- Data Source: 04_scheduler.md - Provides idle signals

## Configuration: 14_apb_config.md - Config register mapping

## 20.10	Revision History

Performance Profiler Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	MonBus AXI-Lite Group

Module: monbus_axil_group.sv Location: projects/components/stream/rtl/macro/ Category: MACRO (Integration) Parent: stream_top.sv Status: Implemented Last Updated: 2025-11-30

## 21.1	Overview

The monbus_axil_group module receives monitor bus packets from STREAM channels, applies configurable filtering, and routes filtered packets to either an error/interrupt FIFO (accessible via AXI-Lite slave) or a master write interface (writes to memory).

### 21.1.1	Key Features

- Single Monitor Bus Input: STREAM is memory-to-memory (no network paths)

- Per-Protocol Filtering: Configurable packet type masking

- Dual Output Paths:

  - Error/Interrupt FIFO - generates interrupt when not empty

  - Master Write FIFO - writes to configurable address range

- Protocol Support: AXI, AXIS, CORE (3 protocols)

- Built-in AXI-Lite Skid Buffering: For timing closure

### 21.1.2	Simplified from RAPIDS

RAPIDS has source + sink data paths requiring TWO monitor buses. STREAM has memory-to-memory only, so this module has ONE monitor bus input (no arbitration needed).

## 21.2	Architecture

### 21.2.1	Block Diagram

### 21.2.2	Figure 2.16.1: MonBus AXI-Lite Group Block Diagram

MonBus AXI-Lite Group Block Diagram

Source: 16_monbus_axil_group_block.mmd

### 21.2.3	Filter Decision Tree

For each incoming packet:
1. Extract: pkt_type, pkt_protocol, pkt_event_code
2. Check protocol-specific masks:
   - pkt_mask: Drop if bit[pkt_type] = 1
   - err_select: Route to error FIFO if bit[pkt_type] = 1
   - event_mask: Drop if bit[event_code] = 1
3. Route:
   - Dropped packets: Consumed silently
   - Error-selected: To error/interrupt FIFO
   - Remaining: To master write FIFO

## 21.3	Parameters

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| FIFO_DEPTH_ERR | int | 64 | Error/interrupt FIFO depth |
| FIFO_DEPTH_WRITE | int | 32 | Master write FIFO depth |
| ADDR_WIDTH | int | 32 | AXI address width |
| DATA_WIDTH | int | 32 | AXI data width |
| NUM_PROTOCOLS | int | 3 | AXI, AXIS, CORE |

## 21.4	Port List

### 21.4.1	Clock and Reset

Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| axi_aclk | input | 1 | AXI clock |
| axi_aresetn | input | 1 | AXI active-low reset |

### 21.4.2	Monitor Bus Input

Monitor Bus Input

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| monbus_valid | input | 1 | Packet valid |
| monbus_ready | output | 1 | Ready to accept |
| monbus_packet | input | 64 | 64-bit monitor packet |

### 21.4.3	AXI-Lite Slave Read Interface

AXI-Lite Slave Read Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axil_arvalid | input | 1 | Read address valid |
| s_axil_arready | output | 1 | Read address ready |
| s_axil_araddr | input | ADDR_WIDTH | Read address |
| s_axil_arprot | input | 3 | Protection bits |
| s_axil_rvalid | output | 1 | Read data valid |
| s_axil_rready | input | 1 | Read data ready |
| s_axil_rdata | output | DATA_WIDTH | Read data |
| s_axil_rresp | output | 2 | Read response |

### 21.4.4	AXI-Lite Master Write Interface

AXI-Lite Master Write Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axil_awvalid | output | 1 | Write address valid |
| m_axil_awready | input | 1 | Write address ready |
| m_axil_awaddr | output | ADDR_WIDTH | Write address |
| m_axil_awprot | output | 3 | Protection bits |
| m_axil_wvalid | output | 1 | Write data valid |
| m_axil_wready | input | 1 | Write data ready |
| m_axil_wdata | output | DATA_WIDTH | Write data |
| m_axil_wstrb | output | DATA_WIDTH/8 | Write strobes |
| m_axil_bvalid | input | 1 | Write response valid |
| m_axil_bready | output | 1 | Write response ready |
| m_axil_bresp | input | 2 | Write response |

### 21.4.5	Interrupt Output

Interrupt Output

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| irq_out | output | 1 | Interrupt (error FIFO not empty) |

### 21.4.6	Configuration Interface

Configuration Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_base_addr | input | ADDR_WIDTH | Base for master writes |
| cfg_limit_addr | input | ADDR_WIDTH | Limit for master writes |

### 21.4.7	Protocol Configuration (per protocol)

AXI Protocol:

Protocol Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axi_pkt_mask | input | 16 | Drop mask for packet types |
| cfg_axi_err_select | input | 16 | Error FIFO select |
| cfg_axi_*_mask | input | 16 | Per-event-type masks |

AXIS Protocol:

Protocol Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_axis_pkt_mask | input | 16 | Drop mask |
| cfg_axis_err_select | input | 16 | Error FIFO select |
| cfg_axis_*_mask | input | 16 | Per-event-type masks |

CORE Protocol:

Protocol Configuration

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cfg_core_pkt_mask | input | 16 | Drop mask |
| cfg_core_err_select | input | 16 | Error FIFO select |
| cfg_core_*_mask | input | 16 | Per-event-type masks |

### 21.4.8	Debug/Status

Debug/Status

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| err_fifo_full | output | 1 | Error FIFO full |
| write_fifo_full | output | 1 | Write FIFO full |
| err_fifo_count | output | 8 | Error FIFO count |
| write_fifo_count | output | 8 | Write FIFO count |

## 21.5	Operation

### 21.5.1	Packet Type Filtering

// Protocol-specific filtering (example: AXI)
case (pkt_protocol)
    3'b000: begin // AXI
        // Check if packet type is dropped
        pkt_drop = cfg_axi_pkt_mask[pkt_type];

        // Check if packet goes to error FIFO
        pkt_to_err_fifo = cfg_axi_err_select[pkt_type] && !pkt_drop;

        // Check individual event masking
        case (pkt_type)
            PktTypeError:     pkt_event_masked = cfg_axi_error_mask[pkt_event_code];
            PktTypeTimeout:   pkt_event_masked = cfg_axi_timeout_mask[pkt_event_code];
            // ... more packet types
        endcase
    end
endcase

### 21.5.2	Master Write Address Generation

// Address wraps within configured range
always_comb begin
    next_write_addr = current_write_addr + (DATA_WIDTH == 64 ? 8 : 4);
    if (next_write_addr > cfg_limit_addr) begin
        next_write_addr = cfg_base_addr;
    end
end

### 21.5.3	Master Write FSM

typedef enum logic [2:0] {
    WRITE_IDLE       = 3'b000,
    WRITE_ADDR       = 3'b001,
    WRITE_DATA_LOW   = 3'b010,
    WRITE_DATA_HIGH  = 3'b011,  // For 32-bit data width only
    WRITE_RESP       = 3'b100
} write_state_t;

## 21.6	Integration Example

monbus_axil_group #(
    .FIFO_DEPTH_ERR     (64),
    .FIFO_DEPTH_WRITE   (32),
    .ADDR_WIDTH         (32),
    .DATA_WIDTH         (32),
    .NUM_PROTOCOLS      (3)
) u_monbus_axil_group (
    .axi_aclk           (clk),
    .axi_aresetn        (rst_n),

    // Monitor bus input
    .monbus_valid       (stream_mon_valid),
    .monbus_ready       (stream_mon_ready),
    .monbus_packet      (stream_mon_packet),

    // AXI-Lite slave read (for error FIFO access)
    .s_axil_arvalid     (axil_slave_arvalid),
    .s_axil_arready     (axil_slave_arready),
    // ... more slave signals

    // AXI-Lite master write (for logging)
    .m_axil_awvalid     (axil_master_awvalid),
    .m_axil_awready     (axil_master_awready),
    // ... more master signals

    // Interrupt
    .irq_out            (monbus_interrupt),

    // Configuration
    .cfg_base_addr      (cfg_monbus_base_addr),
    .cfg_limit_addr     (cfg_monbus_limit_addr),
    .cfg_axi_pkt_mask   (cfg_axi_pkt_mask),
    // ... more protocol configuration
);

## 21.7	Common Issues

### 21.7.1	Issue 1: Missing Monitor Packets

Symptom: Expected events not appearing in FIFO

Root Causes: 1. Packet type masked in cfg_pkt_mask 2. Event masked in cfg_*_mask 3. FIFO full (packets dropped)

Solution: Check mask configuration and FIFO status.

### 21.7.2	Issue 2: Master Writes Stall

Symptom: Write FIFO fills up, backpressure to monitor bus

Root Causes: 1. AXI-Lite master target not responding 2. Address range exhausted (wrapping too fast)

Solution: Increase address range or reduce logging rate via masks.

## 21.8	Related Documentation

- Parent: 01_stream_core.md - Provides monitor bus packets

- Packet Format: Monitor Bus Protocol documentation

## Configuration: 14_apb_config.md - Register mapping

## 21.9	Revision History

MonBus AXI-Lite Group Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Chapter 3: External Interfaces

This chapter documents the external interfaces of the STREAM DMA engine (stream_top_ch8.sv).

## 22.1	Interface Specifications

STREAM has seven primary external interfaces:

### 22.1.1	01_axi4_interface_spec.md

- AXI4 Master Interfaces (3 total)

  - Descriptor fetch (256-bit, read-only)

  - Data read (parameterizable, default 512-bit)

  - Data write (parameterizable, default 512-bit)

- Protocol specifications and assumptions

- Transfer modes and alignment requirements

- Timing and performance characteristics

### 22.1.2	02_axil4_interface_spec.md

- AXI4-Lite Interfaces (2 total, in monbus_axil_group)

  - Error FIFO slave read (s_axil_err_*): 32-bit, read-only

  - Monitor data master write (m_axil_mon_*): 32-bit, write-only

- Used for monitor bus packet access and logging

- Part of monbus_axil_group integration (USE_AXI_MONITORS=1)

### 22.1.3	03_apb_interface_spec.md

- APB4 Slave Interface

- Full APB4 protocol with PSEL/PENABLE

- Optional CDC for asynchronous clock domains (CDC_ENABLE parameter)

- Address map: 0x000-0x03F kick-off, 0x100-0x3FF registers

### 22.1.4	05_monbus_interface_spec.md

- MonBus Internal Protocol

- Unified 64-bit monitoring bus (stream_core → monbus_axil_group)

- Event packet format and encoding

- Agent ID assignments for STREAM components

- Converted to AXI-Lite at top level

## 22.2	STREAM Interface Summary (stream_top_ch8.sv)

| Interface | Type | Direction | Width | Purpose |
| --- | --- | --- | --- | --- |
| APB4 | Slave | Input | 32-bit data, 12-bit addr | Configuration and kick-off |
| AXI4 Descriptor | Master Read | Output | 256-bit | Descriptor fetch |
| AXI4 Data Read | Master Read | Output | 512-bit (param) | Source data read |
| AXI4 Data Write | Master Write | Output | 512-bit (param) | Destination data write |
| AXIL Error FIFO | Slave Read | Input | 32-bit | Monitor error/interrupt FIFO |
| AXIL Monitor Write | Master Write | Output | 32-bit | Monitor data to memory |
| IRQ | Output | Output | 1-bit | Interrupt (error FIFO not empty) |

Note: AXIL interfaces are active when USE_AXI_MONITORS=1. MonBus is internal (stream_core → monbus_axil_group).

Last Updated: 2025-12-01

Dependencies: - AMBA AXI4 Protocol Specification v4.0 - AMBA Monitor Bus Protocol (internal spec) - STREAM Architecture Overview

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	AXI4 Interface Specification for STREAM

## 23.1	Overview

This document defines the AXI4 interface specification for the STREAM DMA engine. STREAM uses three AXI4 master interfaces for memory access:

- Descriptor Fetch Master - 256-bit read-only interface for fetching descriptors

- Data Read Master - Parameterizable width (default 512-bit) for reading source data

- Data Write Master - Parameterizable width (default 512-bit) for writing destination data

Note: This document focuses on STREAM-specific implementation details. For generic AXI4 protocol information, refer to the AMBA AXI4 Protocol Specification v4.0.

## 23.2	STREAM AXI4 Interface Summary

### 23.2.1	Number of Interfaces

STREAM implements 3 AXI4 Master Interfaces:

Number of Interfaces

| Interface | Type | Width | Channels | Purpose |
| --- | --- | --- | --- | --- |
| Descriptor Fetch | Master Read | 256-bit (fixed) | AR, R | Fetch descriptors from memory |
| Data Read | Master Read | 512-bit (param) | AR, R | Read source data to SRAM |
| Data Write | Master Write | 512-bit (param) | AW, W, B | Write SRAM data to destination |

### 23.2.2	Interface Parameters

Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| DATA_WIDTH | AXI data bus width in bits | 32, 64, 128, 256, 512, 1024 | 32 |
| ADDR_WIDTH | AXI address bus width in bits | 32, 64 | 37 |
| ID_WIDTH | AXI ID tag width in bits | 1-16 | 8 |
| USER_WIDTH | AXI user signal width in bits (optional) | 0-16 | 1 |

### 23.2.3	Interface Types and Transfer Modes

Interface Types and Transfer Modes

| Interface Group | Channels | Transfer Mode | Address Alignment | Monitor | DCG | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AXI4 Master Read-Split | AR, R | Flexible | 4-byte | Yes | Yes | Data interfaces with chunk enables |
| AXI4 Master Write-Split | AW, W, B | Flexible | 4-byte | Yes | Yes | Data interfaces with chunk enables |
| AXI4 Master Read | AR, R | Simplified | Bus-width | No | Yes | Control interfaces, fully aligned |
| AXI4 Master Write | AW, W, B | Simplified | Bus-width | Yes | Yes | Control interfaces, fully aligned |

### 23.2.4	Interface Group Parameter Settings

Interface Group Parameter Settings

| Interface Group | Data Width | Address Width | ID Width | User Width | Transfer Mode |
| --- | --- | --- | --- | --- | --- |
| AXI4 Master Read-Split | 512 bits | 37 bits | 8 bits | 1 bit | Flexible |
| AXI4 Master Write-Split | 512 bits | 37 bits | 8 bits | 1 bit | Flexible |
| AXI4 Master Read | 32 bits | 37 bits | 8 bits | 1 bit | Simplified |
| AXI4 Master Write | 32 bits | 37 bits | 8 bits | 1 bit | Simplified |

### 23.2.5	Interface Configuration Summary

Interface Configuration Summary

| Interface Type | Interface Group | Transfer Mode | Alignment | Notes |
| --- | --- | --- | --- | --- |
| Descriptor Sink | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |
| Descriptor Source | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |
| Data Source | AXI4 Master Read-Split | Flexible | 4-byte aligned | High-bandwidth data |
| Data Sink | AXI4 Master Write-Split | Flexible | 4-byte aligned | High-bandwidth data |
| Program Sink | AXI4 Master Write | Simplified | 32-bit aligned | Control interface |
| Program Source | AXI4 Master Write | Simplified | 32-bit aligned | Control interface |
| Flag Sink | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |
| Flag Source | AXI4 Master Read | Simplified | 32-bit aligned | Control interface |

## 23.3	Transfer Mode Specifications

This specification defines two distinct transfer modes to optimize different interface types:

### 23.3.1	Mode 1: Simplified Transfer Mode (Control Interfaces)

Used for control interfaces (descriptors, programs, flags) that prioritize simplicity and predictable timing.

#### 23.3.1.1	Simplified Mode Assumptions

Simplified Mode Assumptions

| Aspect | Requirement |
| --- | --- |
| Address Alignment | All addresses aligned to full data bus width |
| Transfer Size | All transfers use maximum size equal to bus width |
| Burst Type | Incrementing bursts only (AxBURST = 2’b01) |
| Transfer Complexity | Maximum simplicity for predictable operation |

### 23.3.2	Mode 2: Flexible Transfer Mode (Data Interfaces)

Used for high-bandwidth data interfaces that need to handle arbitrary address alignment while maintaining efficiency.

#### 23.3.2.1	Flexible Mode Assumptions

Flexible Mode Assumptions

| Aspect | Requirement |
| --- | --- |
| Address Alignment | 4-byte aligned addresses (minimum alignment) |
| Transfer Sizes | Multiple sizes supported: 4, 8, 16, 32, 64 bytes |
| Burst Type | Incrementing bursts only (AxBURST = 2’b01) |
| Alignment Strategy | Progressive alignment to optimize bus utilization |

## 23.4	Mode 1: Simplified Transfer Mode Specification

### 23.4.1	Assumption 1: Address Alignment to Data Bus Width

Assumption 1: Address Alignment to Data Bus Width

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All AXI transactions aligned to data bus width |
| 32-bit bus (4 bytes) | Address[1:0] must be 2’b00 |
| 64-bit bus (8 bytes) | Address[2:0] must be 3’b000 |
| 128-bit bus (16 bytes) | Address[3:0] must be 4’b0000 |
| 256-bit bus (32 bytes) | Address[4:0] must be 5’b00000 |
| 512-bit bus (64 bytes) | Address[5:0] must be 6’b000000 |
| 1024-bit bus (128 bytes) | Address[6:0] must be 7’b0000000 |
| Rationale | Maximizes bus efficiency and eliminates unaligned access complexity |

### 23.4.2	Assumption 2: Fixed Transfer Size

Assumption 2: Fixed Transfer Size

| Aspect | Requirement |
| --- | --- |
| Transfer Size Rule | All transfers use maximum size equal to bus width |
| 32-bit bus | AxSIZE = 3’b010 (4 bytes) |
| 64-bit bus | AxSIZE = 3’b011 (8 bytes) |
| 128-bit bus | AxSIZE = 3’b100 (16 bytes) |
| 256-bit bus | AxSIZE = 3’b101 (32 bytes) |
| 512-bit bus | AxSIZE = 3’b110 (64 bytes) |
| 1024-bit bus | AxSIZE = 3’b111 (128 bytes) |
| Rationale | Maximizes bus utilization and simplifies address alignment |

## 23.5	Mode 2: Flexible Transfer Mode Specification

### 23.5.1	Assumption 1: 4-Byte Address Alignment

Assumption 1: 4-Byte Address Alignment

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All AXI transactions aligned to 4-byte boundaries |
| Address Constraint | Address[1:0] must be 2’b00 |
| Rationale | Balances flexibility with AXI protocol requirements |
| Benefit | Supports arbitrary data placement while maintaining AXI compliance |

### 23.5.2	Assumption 2: Multiple Transfer Sizes

Assumption 2: Multiple Transfer Sizes

| Transfer Size | AxSIZE Value | Use Case |
| --- | --- | --- |
| 4 bytes | 3’b010 | Initial alignment, small transfers |
| 8 bytes | 3’b011 | Progressive alignment |
| 16 bytes | 3’b100 | Progressive alignment |
| 32 bytes | 3’b101 | Progressive alignment |
| 64 bytes | 3’b110 | Optimal full-width transfers |
| 128 bytes | 3’b111 | Maximum efficiency (1024-bit bus) |

### 23.5.3	Assumption 3: Progressive Alignment Strategy

Assumption 3: Progressive Alignment Strategy

| Aspect | Requirement |
| --- | --- |
| Alignment Goal | Align to 64-byte boundaries for optimal bus utilization |
| Alignment Sequence | Use progressive sizes: 4 → 8 → 16 → 32 → 64 bytes |
| Optimization | Choose largest possible transfer size at each step |
| Example | Address 0x1004: 4-byte transfer → aligned to 0x1008, then larger transfers |

### 23.5.4	Assumption 4: Chunk Enable Support

Assumption 4: Chunk Enable Support

| Aspect | Requirement |
| --- | --- |
| Chunk Granularity | 16 chunks of 32-bits each (512-bit bus) |
| Write Strobes | Generated from chunk enables for precise byte control |
| Alignment Transfers | Chunk patterns optimized for alignment sequences |
| Benefits | Precise data validity, optimal memory utilization |

## 23.6	Common Protocol Assumptions (Both Modes)

### 23.6.1	Assumption 1: Incrementing Bursts Only

Assumption 1: Incrementing Bursts Only

| Aspect | Requirement |
| --- | --- |
| Burst Type | All AXI bursts use incrementing address mode (AxBURST = 2’b01) |
| Excluded Types | No FIXED (2’b00) or WRAP (2’b10) bursts supported |
| Rationale | Simplifies address generation logic and covers most use cases |
| Benefit | Eliminates wrap boundary calculations and fixed address handling |

### 23.6.2	Assumption 2: No Address Wraparound

Assumption 2: No Address Wraparound

| Aspect | Requirement |
| --- | --- |
| Wraparound Rule | Transactions never wrap around top of address space |
| Example | No 0xFFFFFFFF → 0x00000000 transitions |
| Rationale | Real systems never allow this due to memory layout |
| Benefit | Dramatically simplified boundary crossing detection logic |

## 23.7	Flexible Mode: Address Calculation Examples

### 23.7.1	Progressive Alignment Examples

Example 1: Address 0x1004 → 0x1040 (64-byte boundary)

Progressive Alignment Examples

| Step | Address | Size | AxSIZE | Length | Bytes Transferred | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0x1004 | 4 bytes | 3’b010 | 1 beat | 4 | Initial alignment |
| 2 | 0x1008 | 8 bytes | 3’b011 | 1 beat | 8 | Progressive alignment |
| 3 | 0x1010 | 16 bytes | 3’b100 | 1 beat | 16 | Progressive alignment |
| 4 | 0x1020 | 32 bytes | 3’b101 | 1 beat | 32 | Progressive alignment |
| 5 | 0x1040 | 64 bytes | 3’b110 | N beats | 64×N | Optimal transfers |

Example 2: Address 0x1010 → 0x1040 (64-byte boundary)

Progressive Alignment Examples

| Step | Address | Size | AxSIZE | Length | Bytes Transferred | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0x1010 | 16 bytes | 3’b100 | 1 beat | 16 | Optimal initial size |
| 2 | 0x1020 | 32 bytes | 3’b101 | 1 beat | 32 | Progressive alignment |
| 3 | 0x1040 | 64 bytes | 3’b110 | N beats | 64×N | Optimal transfers |

### 23.7.2	Chunk Enable Pattern Examples

512-bit Bus with 16×32-bit chunks

Chunk Enable Pattern Examples

| Transfer Size | Address Offset | Chunk Pattern | Description |
| --- | --- | --- | --- |
| 4 bytes | 0x04 | 16’h0002 | Chunk 1 only |
| 8 bytes | 0x08 | 16’h000C | Chunks 2-3 |
| 16 bytes | 0x10 | 16’h00F0 | Chunks 4-7 |
| 32 bytes | 0x20 | 16’hFF00 | Chunks 8-15 |
| 64 bytes | 0x00 | 16’hFFFF | All chunks |

## 23.8	Master Read Interface Specification

### 23.8.1	Read Address Channel (AR)

Read Address Channel

| Signal | Width | Direction | Simplified Mode | Flexible Mode | Description |
| --- | --- | --- | --- | --- | --- |
| ar_addr | ADDR_WIDTH | Master→Slave | Bus-width aligned | 4-byte aligned | Read address |
| ar_len | 8 | Master→Slave | 0-255 | 0-255 | Burst length - 1 |
| ar_size | 3 | Master→Slave | Fixed per bus | Variable: 4-64 bytes | Transfer size |
| ar_burst | 2 | Master→Slave | 2’b01 (INCR only) | 2’b01 (INCR only) | Burst type |
| ar_id | ID_WIDTH | Master→Slave | Any | Any | Transaction ID |
| ar_lock | 1 | Master→Slave | 1’b0 | 1’b0 | Lock type (normal) |
| ar_cache | 4 | Master→Slave | Implementation specific | 4’b0011 | Cache attributes |
| ar_prot | 3 | Master→Slave | Implementation specific | 3’b000 | Protection attributes |
| ar_qos | 4 | Master→Slave | 4’b0000 | 4’b0000 | Quality of Service |
| ar_region | 4 | Master→Slave | 4’b0000 | 4’b0000 | Region identifier |
| ar_user | USER_WIDTH | Master→Slave | Optional | Optional | User-defined |
| ar_valid | 1 | Master→Slave | 0 or 1 | 0 or 1 | Address valid |
| ar_ready | 1 | Slave→Master | 0 or 1 | 0 or 1 | Address ready |

### 23.8.2	Read Data Channel (R)

Read Data Channel

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| r_data | DATA_WIDTH | Slave→Master | Read data |
| r_id | ID_WIDTH | Slave→Master | Transaction ID |
| r_resp | 2 | Slave→Master | Read response |
| r_last | 1 | Slave→Master | Last transfer in burst |
| r_user | USER_WIDTH | Slave→Master | User-defined (optional) |
| r_valid | 1 | Slave→Master | Read data valid |
| r_ready | 1 | Master→Slave | Read data ready |

## 23.9	Master Write Interface Specification

### 23.9.1	Write Address Channel (AW)

Write Address Channel

| Signal | Width | Direction | Simplified Mode | Flexible Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aw_addr | ADDR_WIDTH | Master→Slave | Bus-width aligned | 4-byte aligned | Write address |
| aw_len | 8 | Master→Slave | 0-255 | 0-255 | Burst length - 1 |
| aw_size | 3 | Master→Slave | Fixed per bus | Variable: 4-64 bytes | Transfer size |
| aw_burst | 2 | Master→Slave | 2’b01 (INCR only) | 2’b01 (INCR only) | Burst type |
| aw_id | ID_WIDTH | Master→Slave | Any | Any | Transaction ID |
| aw_lock | 1 | Master→Slave | 1’b0 | 1’b0 | Lock type (normal) |
| aw_cache | 4 | Master→Slave | Implementation specific | 4’b0011 | Cache attributes |
| aw_prot | 3 | Master→Slave | Implementation specific | 3’b000 | Protection attributes |
| aw_qos | 4 | Master→Slave | 4’b0000 | 4’b0000 | Quality of Service |
| aw_region | 4 | Master→Slave | 4’b0000 | 4’b0000 | Region identifier |
| aw_user | USER_WIDTH | Master→Slave | Optional | Optional | User-defined |
| aw_valid | 1 | Master→Slave | 0 or 1 | 0 or 1 | Address valid |
| aw_ready | 1 | Slave→Master | 0 or 1 | 0 or 1 | Address ready |

### 23.9.2	Write Data Channel (W)

Write Data Channel

| Signal | Width | Direction | Simplified Mode | Flexible Mode | Description |
| --- | --- | --- | --- | --- | --- |
| w_data | DATA_WIDTH | Master→Slave | Write data | Write data | Write data |
| w_strb | DATA_WIDTH/8 | Master→Slave | All 1’s | From chunk enables | Write strobes |
| w_last | 1 | Master→Slave | Last transfer | Last transfer | Last transfer in burst |
| w_user | USER_WIDTH | Master→Slave | Optional | Optional | User-defined |
| w_valid | 1 | Master→Slave | 0 or 1 | 0 or 1 | Write data valid |
| w_ready | 1 | Slave→Master | 0 or 1 | 0 or 1 | Write data ready |

### 23.9.3	Write Response Channel (B)

Write Response Channel

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| b_id | ID_WIDTH | Slave→Master | Transaction ID |
| b_resp | 2 | Slave→Master | Write response |
| b_user | USER_WIDTH | Slave→Master | User-defined (optional) |
| b_valid | 1 | Slave→Master | Response valid |
| b_ready | 1 | Master→Slave | Response ready |

## 23.10	Address Calculation Rules

### 23.10.1	Simplified Mode Address Generation

Simplified Mode Address Generation

| Parameter | Formula | Description |
| --- | --- | --- |
| First Address | Must be bus-width aligned | Starting address |
| Address N | First_Address + (N × Bus_Width_Bytes) | Address for beat N |
| Alignment Check | (Address % Bus_Width_Bytes) == 0 | Must always be true |

### 23.10.2	Flexible Mode Address Generation

Flexible Mode Address Generation

| Parameter | Formula | Description |
| --- | --- | --- |
| First Address | Must be 4-byte aligned | Starting address |
| Address N | First_Address + (N × Transfer_Size) | Address for beat N |
| Alignment Check | (Address % 4) == 0 | Must always be true |
| Progressive Alignment | Choose largest size ≤ bytes_to_boundary | Optimization strategy |

### 23.10.3	4KB Boundary Considerations (Both Modes)

4KB Boundary Considerations

| Validation Rule | Formula | Description |
| --- | --- | --- |
| 4KB Boundary | Bursts cannot cross 4KB (0x1000) boundaries | AXI specification |
| Max Burst Calculation | Max_Beats = (4KB - (Start_Address % 4KB)) / Transfer_Size | Burst limit |
| Boundary Check | Verify no 4KB crossings in burst | Mandatory validation |

## 23.11	Write Strobe Generation

### 23.11.1	Simplified Mode Strobe Generation

Simplified Mode Strobe Generation

| Bus Width | Strobe Pattern | Description |
| --- | --- | --- |
| 32-bit | 4’b1111 | All bytes valid |
| 512-bit | 64’hFFFFFFFFFFFFFFFF | All bytes valid |

### 23.11.2	Flexible Mode Strobe Generation

From Chunk Enables (512-bit bus example):

// Convert 16×32-bit chunk enables to 64×8-bit write strobes
for (int chunk = 0; chunk < 16; chunk++) begin
    if (chunk_enable[chunk]) begin
        w_strb[chunk*4 +: 4] = 4'hF; // 4 bytes per chunk
    end
end

Alignment Transfer Examples:

Flexible Mode Strobe Generation

| Transfer Size | Chunk Pattern | Strobe Pattern | Description |
| --- | --- | --- | --- |
| 4 bytes | 16’h0001 | 64’h000000000000000F | First 4 bytes |
| 16 bytes | 16’h000F | 64’h00000000000000FF | First 16 bytes |
| 32 bytes | 16’h00FF | 64’h0000000000FFFFFF | First 32 bytes |
| 64 bytes | 16’hFFFF | 64’hFFFFFFFFFFFFFFFF | All 64 bytes |

## 23.12	Response Codes

### 23.12.1	Response Code Specification

Response Code Specification

| Value | Name | Description | Simplified Mode Usage | Flexible Mode Usage |
| --- | --- | --- | --- | --- |
| 2’b00 | OKAY | Normal access success | Bus-width aligned access | 4-byte aligned access |
| 2’b01 | EXOKAY | Exclusive access success | Bus-width aligned exclusive | 4-byte aligned exclusive |
| 2’b10 | SLVERR | Slave error | Slave-specific error | Slave-specific error |
| 2’b11 | DECERR | Decode error | Bus-width misalignment | 4-byte misalignment |

## 23.13	Implementation Benefits

### 23.13.1	Simplified Mode Benefits

Simplified Mode Benefits

| Benefit Area | Simplification | Impact |
| --- | --- | --- |
| Address Generation | Simple increment by bus width | Minimal logic complexity |
| Size Checking | No dynamic size validation | No validation logic needed |
| Strobe Generation | All strobes always high | Trivial implementation |
| Timing | Predictable single-size transfers | Optimal timing closure |

### 23.13.2	Flexible Mode Benefits

Flexible Mode Benefits

| Benefit Area | Capability | Impact |
| --- | --- | --- |
| Data Placement | Arbitrary 4-byte aligned placement | Maximum flexibility |
| Bus Utilization | Progressive alignment optimization | High efficiency achieved |
| Chunk Control | Precise byte-level validity | Optimal memory utilization |
| Alignment Strategy | Automatic alignment to boundaries | Performance optimization |

### 23.13.3	Mode Selection Guidelines

Mode Selection Guidelines

| Interface Type | Recommended Mode | Rationale |
| --- | --- | --- |
| High-bandwidth data | Flexible | Maximize throughput, handle arbitrary alignment |
| Control/status | Simplified | Predictable timing, minimal complexity |
| Descriptors | Simplified | Fixed-size structures, simple implementation |
| Programs | Simplified | Single-word writes, minimal overhead |
| Flags | Simplified | Fixed-size status, predictable behavior |

## 23.14	Validation Requirements

### 23.14.1	Simplified Mode Validation

Simplified Mode Validation

| Validation Area | Requirements |
| --- | --- |
| Address Alignment | Verify all addresses aligned to full bus width |
| Fixed Size | Verify AxSIZE always matches DATA_WIDTH |
| Full Strobes | Verify w_strb is always all 1’s |
| Burst Type | Verify AxBURST is always 2’b01 |

### 23.14.2	Flexible Mode Validation

Flexible Mode Validation

| Validation Area | Requirements |
| --- | --- |
| Address Alignment | Verify all addresses are 4-byte aligned |
| Size Validation | Verify AxSIZE matches actual transfer size |
| Chunk Consistency | Verify chunk enables match transfer size |
| Strobe Generation | Verify strobes generated correctly from chunks |
| Progressive Alignment | Verify alignment strategy optimization |
| Boundary Checking | Verify no 4KB boundary crossings |

### 23.14.3	Common Validation

Common Validation

| Validation Area | Requirements |
| --- | --- |
| No Wraparound | Verify addresses never wrap around |
| Incrementing Only | Verify AxBURST is always 2’b01 |
| Response Handling | Verify proper response generation |
| Error Conditions | Verify alignment violation responses |

## 23.15	Performance Characteristics

### 23.15.1	Simplified Mode Performance

Simplified Mode Performance

| Metric | Typical Value | Description |
| --- | --- | --- |
| Latency | 3 cycles | Address + Data + Response |
| Throughput | 1 transfer per clock | Sustained rate |
| Efficiency | 100% | Perfect bus utilization |
| Complexity | Minimal | Simple implementation |

### 23.15.2	Flexible Mode Performance

Flexible Mode Performance

| Metric | Alignment Phase | Optimized Phase | Description |
| --- | --- | --- | --- |
| Latency | 3-15 cycles | 3 cycles | Variable based on alignment |
| Throughput | Variable | 1 transfer per clock | Depends on alignment pattern |
| Efficiency | 25-100% | 100% | Improves with alignment |
| Complexity | Moderate | Minimal | Progressive optimization |

### 23.15.3	Performance Optimization Strategy

Flexible Mode Alignment Strategy: 1. Initial Phase: Use largest possible transfer size for current alignment 2. Progressive Phase: Incrementally align to larger boundaries
3. Optimized Phase: Use full bus-width transfers once aligned 4. Result: Achieve maximum efficiency while handling arbitrary starting addresses

## 23.16	This dual-mode approach provides the best of both worlds: simplified, predictable operation for control interfaces and flexible, high-performance operation for data interfaces.

## 23.17	Revision History

AXI4 Interface Specification for STREAM Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	AXI4-Lite Interface Specification and Assumptions

## 24.1	Overview

This document defines the formal specification and assumptions for an AXI4-Lite interface implementation. AXI4-Lite is a subset of AXI4 optimized for simple, lightweight control register interfaces with inherent protocol simplifications.

## 24.2	Interface Summary

### 24.2.1	Number of Interfaces

- 2 Master Read Interface: Single read channel for Monitor Packets (one for each Source and Sink)

- 2 Master Write Interface: Single write channel for Monitor Packets plus a timestamp (one for each Source and Sink)

### 24.2.2	Interface Parameters

Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| DATA_WIDTH | AXI data bus width in bits | 32, 64 | 32, 64 |
| ADDR_WIDTH | AXI address bus width in bits | 32, 64 | 37 |
| STRB_WIDTH | Write strobe width | DATA_WIDTH/8 | 8 |

## 24.3	Core Protocol Assumptions

### 24.3.1	Inherent AXI4-Lite Simplifications

AXI4-Lite protocol inherently provides the following constraints:

Inherent AXI4-Lite Simplifications

| Constraint | Description |
| --- | --- |
| Single Transfers Only | No burst transactions supported |
| No Transaction IDs | All transactions are in-order |
| Fixed Transfer Size | Always uses full data bus width |
| No User Signals | Simplified interface without user-defined extensions |

### 24.3.2	Implementation Assumptions

#### 24.3.2.1	Assumption 1: Address Alignment to Data Bus Width

Assumption 1: Address Alignment to Data Bus Width

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All AXI4-Lite transactions aligned to data bus width |
| 32-bit bus alignment | Address[1:0] must be 2’b00 (4-byte aligned) |
| 64-bit bus alignment | Address[2:0] must be 3’b000 (8-byte aligned) |
| Rationale | Maximizes bus efficiency and eliminates unaligned access complexity |
| Benefit | Simplifies address decode and data steering logic |

#### 24.3.2.2	Assumption 2: Fixed Transfer Size

Assumption 2: Fixed Transfer Size

| Aspect | Requirement |
| --- | --- |
| Transfer Size Rule | All transfers use maximum size equal to bus width |
| 32-bit bus | AxSIZE = 3’b010 (4 bytes) |
| 64-bit bus | AxSIZE = 3’b011 (8 bytes) |
| Rationale | Maximizes bus utilization and simplifies control logic |
| Benefit | No size decode logic required |

#### 24.3.2.3	Assumption 3: No Address Wraparound

Assumption 3: No Address Wraparound

| Aspect | Requirement |
| --- | --- |
| Wraparound Rule | Transactions never wrap around top of address space |
| Rationale | Control register accesses never require wraparound behavior |
| Benefit | Simplified address boundary checking |

#### 24.3.2.4	Assumption 4: Standard Protection Attributes

Assumption 4: Standard Protection Attributes

| Access Type | AxPROT Value | Description |
| --- | --- | --- |
| Normal Access | 3’b000 | Data, secure, unprivileged |
| Privileged Access | 3’b001 | Data, secure, privileged |
| Rationale |  | Covers the majority of control register access patterns |

## 24.4	Master Read Interface Specification

### 24.4.1	Read Address Channel (AR)

Read Address Channel

| Signal | Width | Direction | Required Values | Description |
| --- | --- | --- | --- | --- |
| ar_addr | ADDR_WIDTH | Master→Slave | 8-byte aligned | Read address |
| ar_prot | 3 | Master→Slave | Implementation specific | Protection attributes |
| ar_valid | 1 | Master→Slave | 0 or 1 | Address valid |
| ar_ready | 1 | Slave→Master | 0 or 1 | Address ready |

### 24.4.2	Read Data Channel (R)

Read Data Channel

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| r_data | 64 | Slave→Master | Read data |
| r_resp | 2 | Slave→Master | Read response |
| r_valid | 1 | Slave→Master | Read data valid |
| r_ready | 1 | Master→Slave | Read data ready |

### 24.4.3	AXI4-Lite Simplifications (Read)

AXI4-Lite Simplifications

| Removed Signal | AXI4 Usage | AXI4-Lite Reason |
| --- | --- | --- |
| ar_id | Transaction ID | Single transfers, no transaction IDs |
| ar_len | Burst length | Single transfers only |
| ar_size | Transfer size | Fixed to bus width |
| ar_burst | Burst type | Single transfers only |
| ar_lock | Lock type | Simplified access model |
| ar_cache | Cache attributes | Simplified memory model |
| ar_qos | Quality of Service | Simplified priority model |
| ar_region | Region identifier | Simplified address space |
| ar_user | User-defined | Simplified interface |
| r_id | Transaction ID | No transaction IDs needed |
| r_last | Last transfer | Single transfers only |
| r_user | User-defined | Simplified interface |

## 24.5	Master Write Interface Specification

### 24.5.1	Write Address Channel (AW)

Write Address Channel

| Signal | Width | Direction | Required Values | Description |
| --- | --- | --- | --- | --- |
| aw_addr | ADDR_WIDTH | Master→Slave | 8-byte aligned | Write address |
| aw_prot | 3 | Master→Slave | Implementation specific | Protection attributes |
| aw_valid | 1 | Master→Slave | 0 or 1 | Address valid |
| aw_ready | 1 | Slave→Master | 0 or 1 | Address ready |

### 24.5.2	Write Data Channel (W)

Write Data Channel

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| w_data | 32 | Master→Slave | Write data |
| w_strb | 4 | Master→Slave | Write strobes (byte enables) |
| w_valid | 1 | Master→Slave | Write data valid |
| w_ready | 1 | Slave→Master | Write data ready |

### 24.5.3	Write Response Channel (B)

Write Response Channel

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| b_resp | 2 | Slave→Master | Write response |
| b_valid | 1 | Slave→Master | Response valid |
| b_ready | 1 | Master→Slave | Response ready |

### 24.5.4	AXI4-Lite Simplifications (Write)

AXI4-Lite Simplifications

| Removed Signal | AXI4 Usage | AXI4-Lite Reason |
| --- | --- | --- |
| aw_id | Transaction ID | Single transfers, no transaction IDs |
| aw_len | Burst length | Single transfers only |
| aw_size | Transfer size | Fixed to bus width |
| aw_burst | Burst type | Single transfers only |
| aw_lock | Lock type | Simplified access model |
| aw_cache | Cache attributes | Simplified memory model |
| aw_qos | Quality of Service | Simplified priority model |
| aw_region | Region identifier | Simplified address space |
| aw_user | User-defined | Simplified interface |
| w_last | Last transfer | Single transfers only |
| w_user | User-defined | Simplified interface |
| b_id | Transaction ID | No transaction IDs needed |
| b_user | User-defined | Simplified interface |

## 24.6	Address Requirements

### 24.6.1	Address Alignment Rules

Address Alignment Rules

| Alignment Type | Formula | Description |
| --- | --- | --- |
| Valid Address | (Address % 4) == 0 | Must be 8-byte aligned |
| Mandatory Alignment | Address[2:0] must be 3’b000 | Per Assumption 1 |

### 24.6.2	Address Validation Examples

Address Validation Examples

| Address Category | Examples | Status |
| --- | --- | --- |
| Valid (84byte aligned) | 0x1000, 0x1004, 0x1008, 0x100C | Accepted |
| Invalid (unaligned) | 0x1001, 0x1002, 0x1003 | DECERR response |

## 24.7	Response Codes

### 24.7.1	Response Code Specification

Response Code Specification

| Value | Name | Description | Usage in Control Registers |
| --- | --- | --- | --- |
| 2’b00 | OKAY | Normal access success | Successful register access |
| 2’b01 | EXOKAY | Exclusive access success | Not used in AXI4-Lite |
| 2’b10 | SLVERR | Slave error | Invalid register access |
| 2’b11 | DECERR | Decode error | Address decode failure or misalignment |

### 24.7.2	Response Usage Guidelines

Response Usage Guidelines

| Response Type | Usage | Description |
| --- | --- | --- |
| OKAY | Normal completion | Successful register access |
| EXOKAY | Not applicable | AXI4-Lite doesn’t support exclusive accesses |
| SLVERR | Register error | Invalid register operation |
| DECERR | Address error | Misalignment or decode failure per Assumption 1 |

## 24.8	Protection Signal Usage

### 24.8.1	Protection Signal Encoding

Protection Signal Encoding

| Bit | Name | Description | Recommended Usage |
| --- | --- | --- | --- |
| [0] | Privileged | 0=Normal, 1=Privileged | Set based on processor mode |
| [1] | Non-secure | 0=Secure, 1=Non-secure | Set based on security domain |
| [2] | Instruction | 0=Data, 1=Instruction | Always 0 for control registers |

### 24.8.2	Common Protection Patterns

Common Protection Patterns

| Pattern | AxPROT Value | Description |
| --- | --- | --- |
| Normal Data Access | 3’b000 | Standard register access |
| Privileged Data Access | 3’b001 | Privileged register access |
| Debug Access | 3’b010 | Debug register access |
| Privileged Debug | 3’b011 | Privileged debug access |

## 24.9	Implementation Benefits

### 24.9.1	Simplified Control Register Interface

Simplified Control Register Interface

| Benefit Area | Simplification | Impact |
| --- | --- | --- |
| Address Decode | Simple 8-byte aligned address comparison | Reduced decode logic |
| Transaction Handling | No burst or ID tracking required | Simplified state machines |
| Flow Control | Straightforward valid-ready handshakes | Reduced complexity |
| Response Generation | Simple OKAY/SLVERR/DECERR responses | Minimal response logic |
| Size Handling | Fixed 64-bit transfers only | No size decode needed |

### 24.9.2	Address Decode Implementation

Address Decode Implementation

| Implementation Aspect | Method | Benefit |
| --- | --- | --- |
| 4-byte Alignment Check | addr[1:0] == 2’b00 | Simple bit masking |
| Address Range Check | addr >= base && addr <= limit | Simple comparisons |
| Combined Check | alignment_ok && range_ok | Single decode decision |

### 24.9.3	Error Generation Logic

Error Generation Logic

| Error Condition | Check | Response |
| --- | --- | --- |
| Address Misalignment | addr[1:0] != 2’b00 | Generate DECERR |
| Address Out of Range | !addr_in_range(addr) | Generate DECERR |
| Register Error | register_error_condition | Generate SLVERR |
| Normal Access | All checks pass | Generate OKAY |

## 24.10	Timing Requirements

### 24.10.1	Handshake Protocol

Handshake Protocol

| Protocol Rule | Requirement | Description |
| --- | --- | --- |
| Valid-Ready Transfer | Transfer occurs when both VALID and READY are high | Standard AXI handshake |
| Valid Independence | VALID can be asserted independently of READY | Master controls valid |
| Ready Dependency | READY can depend on VALID state | Slave controls ready |
| Signal Stability | Once VALID asserted, all signals stable until READY | Data integrity |

### 24.10.2	Channel Dependencies

Channel Dependencies

| Dependency | Requirement | Description |
| --- | --- | --- |
| Write Channels | AW and W channels are independent | Can be presented in any order |
| Write Response | B channel waits for both AW and W completion | Response dependency |
| Read Channels | R channel waits for AR channel completion | Response dependency |
| Transaction Ordering | Multiple outstanding transactions not supported | Inherent AXI4-Lite limitation |

### 24.10.3	Reset Behavior

Reset Behavior

| Reset Phase | Requirement | Description |
| --- | --- | --- |
| Active Reset | aresetn is active-low reset signal | Standard AXI reset |
| Reset Requirements | All VALID signals deasserted during reset | Clean reset state |
| Reset Recovery | All VALID signals low after reset deassertion | Proper startup |

## 24.11	Validation Requirements

### 24.11.1	Functional Validation

Functional Validation

| Validation Area | Requirements |
| --- | --- |
| Address Alignment | Verify all accesses are 8-byte aligned per Assumption 1 |
| Fixed Size | Verify all transfers are full 64-bit width per Assumption 2 |
| Response Correctness | Verify appropriate response codes (DECERR for misaligned access) |
| Handshake Compliance | Verify all valid-ready handshakes |
| Register Behavior | Verify read/write register functionality |
| No Wraparound | Verify no address wraparound scenarios per Assumption 3 |

### 24.11.2	Timing Validation

Timing Validation

| Validation Area | Requirements |
| --- | --- |
| Setup/Hold | Verify signal timing requirements |
| Reset Behavior | Verify proper reset sequence |
| Back-pressure | Verify ready signal behavior under load |

### 24.11.3	Error Injection Testing

Error Injection Testing

| Test Type | Injection Method | Expected Response |
| --- | --- | --- |
| Misaligned Address | Inject addresses with addr[2:0] != 0 | DECERR response |
| Out of Range | Inject addresses outside valid range | DECERR response |
| Register Errors | Inject register-specific errors | SLVERR response |

## 24.12	Example Transactions

### 24.12.1	64-bit Register Write

64-bit Register Write

| Parameter | Value | Description |
| --- | --- | --- |
| Bus Width | 64 bits (8 bytes) | Data bus configuration |
| Target Address | 0x1000 (8-byte aligned) | Valid aligned address |
| Write Data | 0xDEADBEEFCAFEBABE | 64-bit data value |
| Required Settings | aw_addr=0x1000, aw_prot=3’b000, w_data=0xDEADBEEFCAFEBABE, w_strb=8’b11111111 | Transaction configuration |

### 24.12.2	AW Transaction Flow

AW Transaction Flow

| Step | Action | Signal States |
| --- | --- | --- |
| 1 | Assert aw_valid with address | aw_valid=1, aw_addr=0x1000 |
| 2 | Assert w_valid with data | w_valid=1, w_data=0xDEADBEEFCAFEBABE |
| 3 | Wait for handshakes | aw_ready=1, w_ready=1 |
| 4 | Wait for response | b_valid=1, b_resp=OKAY |
| 5 | Complete transaction | b_ready=1 |

### 24.12.3	64-bit Register Read

64-bit Register Read

| Parameter | Value | Description |
| --- | --- | --- |
| Bus Width | 64 bits (8 bytes) | Data bus configuration |
| Target Address | 0x1008 (8-byte aligned) | Valid aligned address |
| Required Settings | ar_addr=0x1008, ar_prot=3’b000 | Transaction configuration |

### 24.12.4	AR Transaction Flow

AR Transaction Flow

| Step | Action | Signal States |
| --- | --- | --- |
| 1 | Assert ar_valid with address | ar_valid=1, ar_addr=0x1008 |
| 2 | Wait for address handshake | ar_ready=1 |
| 3 | Wait for data response | r_valid=1, r_resp=OKAY |
| 4 | Complete transaction | r_ready=1 |
| 5 | Capture data | r_data (64 bits) |

### 24.12.5	Misaligned Address Example

Misaligned Address Example

| Parameter | Value | Description |
| --- | --- | --- |
| Bus Width | 64 bits (8 bytes) | Data bus configuration |
| Target Address | 0x1004 (misaligned) | Invalid address |
| Expected Behavior | Address decode detects misalignment → DECERR response → No register access | Error handling |

## 24.13	Common Use Cases

### 24.13.1	Typical Applications

Typical Applications

| Application | Description |
| --- | --- |
| Control/Status Registers | 64-bit device configuration and monitoring |
| Memory-Mapped Peripherals | Simple register-based devices |
| Debug Interfaces | Debug and trace control registers |
| Configuration Space | PCIe configuration space access |
| Performance Counters | 64-bit performance monitoring registers |

### 24.13.2	Performance Considerations

Performance Considerations —

| Consideration | Impact | Description |
| --- | --- | --- |
| Latency | Single-cycle responses preferred | Simple registers |
| Throughput | Limited by single outstanding transaction | AXI4-Lite constraint |
| Efficiency | 64-bit transfers maximize data efficiency | Modern system optimization |

## 24.14	Revision History

AXI4-Lite Interface Specification and Assumptions Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	APB Programming Interface Specification for STREAM

## 25.1	Overview

This document defines the APB programming interface for the STREAM DMA engine.

Important: STREAM uses a simplified programming interface for descriptor kick-off, NOT a full APB slave. The interface uses standard valid/ready handshaking per channel to initiate descriptor-based transfers.

This interface is separate from the optional APB configuration interface that may be added in future implementations for runtime configuration of STREAM parameters.

## 25.2	Interface Summary

### 25.2.1	STREAM Programming Interface

STREAM provides a simplified per-channel descriptor programming interface:

STREAM Programming Interface

| Signal Group | Per-Channel Signals | Protocol | Purpose |
| --- | --- | --- | --- |
| Programming | apb_valid[ch], apb_ready[ch], apb_addr[ch] | Valid/Ready handshake | Descriptor address input |

Note: Despite the “apb_” prefix, this is NOT a full AMBA APB interface. It’s a simplified programming interface using APB naming convention for historical reasons.

### 25.2.2	Interface Parameters

Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| DATA_WIDTH | APB data bus width in bits | 8, 16, 32 | 32 |
| ADDR_WIDTH | APB address bus width in bits | 16, 24, 32 | 32 |
| STRB_WIDTH | Write strobe width (APB4 only) | DATA_WIDTH/8 | 4 |
| NUM_SLAVES | Number of peripheral slaves | 1-32 | 1 |
| DEPTH | Internal buffer depth | 2+ | 2 |

## 25.3	Core Protocol Assumptions

### 25.3.1	Inherent APB Simplifications

APB protocol inherently provides the following constraints:

- Single Master Only: Only one bus master supported

- Non-Pipelined: One transaction completes before next begins

- Simple 2-Phase Protocol: Setup phase followed by access phase

- No Burst Transfers: Only single transfers supported

- In-Order Completion: No out-of-order transaction capability

### 25.3.2	Implementation Assumptions

#### 25.3.2.1	Assumption 1: Word-Aligned Access Only

Assumption 1: Word-Aligned Access Only

| Aspect | Requirement |
| --- | --- |
| Alignment Rule | All APB transfers are aligned to natural word boundaries |
| 8-bit transfers | Byte aligned (no restriction) |
| 16-bit transfers | 2-byte aligned (PADDR[0] = 0) |
| 32-bit transfers | 4-byte aligned (PADDR[1:0] = 2’b00) |
| Rationale | Simplifies peripheral decode logic and ensures efficient access |

#### 25.3.2.2	Assumption 2: Standard Transfer Sizes

Assumption 2: Standard Transfer Sizes

| Bus Width | Supported Transfer Sizes | Rationale |
| --- | --- | --- |
| 32-bit bus | 8-bit, 16-bit, 32-bit | Covers typical register access patterns |
| 16-bit bus | 8-bit, 16-bit | Keeps decode logic simple |
| 8-bit bus | 8-bit only | Minimal complexity |

#### 25.3.2.3	Assumption 3: Single-Cycle Default Operation

Assumption 3: Single-Cycle Default Operation

| Aspect | Requirement |
| --- | --- |
| Default Behavior | Most peripherals respond in a single cycle (PREADY tied high) |
| Optimization | PREADY optimization for simple registers |
| Exception | Complex peripherals may use PREADY for wait states |
| Rationale | Minimizes latency for control register access |

## 25.4	Interface Signal Specification

### 25.4.1	Clock and Reset Signals

Clock and Reset Signals ### Address and Control Signals

| Signal | IO | Description |
| --- | --- | --- |
| src_clk | I | Source APB clock signal |
| snk_rdata[31:0] | I | Read data |
| src_wdata[31:0] | O | Write data |

Address and Control Signals

| Signal | Width | Direction | Required Values | Description |
| --- | --- | --- | --- | --- |
| src_addr | 11 bits | Master→Slave | Aligned per transfer size | Peripheral address (2KB space) |
| src_sel | 1 | Master→Slave | 0 or 1 | Peripheral select |
| src_enable | 1 | Master→Slave | 0 or 1 | Enable signal |
| src_write | 1 | Master→Slave | 0 or 1 | Write enable (1=write, 0=read) |
| snk_addr | 11 bits | Master→Slave | Aligned per transfer size | Peripheral address (2KB space) |
| snk_sel | 1 | Master→Slave | 0 or 1 | Peripheral select |
| snk_enable | 1 | Master→Slave | 0 or 1 | Enable signal |
| snk_write | 1 | Master→Slave | 0 or 1 | Write enable (1=write, 0=read) |

### 25.4.2	Data Signals

Data Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| src_wdata | 32 | Master→Slave | Write data |
| src_rdata | 32 | Slave→Master | Read data |
| snk_wdata | 32 | Master→Slave | Write data |
| snk_rdata | 32 | Slave→Master | Read data |

### 25.4.3	Response Signals

Response Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| src_ready | 1 | Slave→Master | Transfer ready |
| src_slverr | 1 | Slave→Master | Slave error |
| snk_ready | 1 | Slave→Master | Transfer ready |
| snk_slverr | 1 | Slave→Master | Slave error |

## 25.5	Address Space Configuration

### 25.5.1	Address Range Specification

Address Range Specification

| Parameter | Value | Description |
| --- | --- | --- |
| Address Width | 11 bits (src_addr[10:0]) | Full address space |
| Address Space | 2KB (2048 bytes) | Total addressable space |
| Register Alignment | 32-bit aligned (4-byte boundaries) | Address constraints |
| Usable Addresses | 0x000 to 0x7FF | Valid address range |

### 25.5.2	Address Decode Implementation

Address Decode Implementation

| Register | Address Bits | Address Value | Description |
| --- | --- | --- | --- |
| reg0 | src_addr[10:2] == 9’h000 | 0x000 | First register |
| reg1 | src_addr[10:2] == 9’h001 | 0x004 | Second register |
| reg2 | src_addr[10:2] == 9’h002 | 0x008 | Third register |
| … | … | … | Up to 512 32-bit registers |

## 25.6	Transaction Protocol

### 25.6.1	2-Phase Transaction States

2-Phase Transaction States

| Phase | State | Duration | Signal Requirements |
| --- | --- | --- | --- |
| Setup (T1) | SETUP | 1 clock cycle | src_sel=1, src_enable=0, address/data stable |
| Access (T2+) | ACCESS | 1+ clock cycles | src_sel=1, src_enable=1, wait for src_ready |
| Idle | IDLE | Variable | src_sel=0, src_enable=0 |

### 25.6.2	State Transitions

State Transitions

| Current State | Next State | Condition | Description |
| --- | --- | --- | --- |
| IDLE | SETUP | Transaction start | Master drives address and control |
| SETUP | ACCESS | Clock edge | Master asserts src_enable |
| ACCESS | IDLE | src_ready=1 | Transaction completes |
| ACCESS | ACCESS | src_ready=0 | Wait state continues |

### 25.6.3	Transaction Timing Requirements

Transaction Timing Requirements

| Timing Parameter | Requirement | Description |
| --- | --- | --- |
| Setup Time | All master signals stable before rising src_clk | Signal stability |
| Hold Time | All master signals held after rising src_clk | Signal stability |
| Output Delay | Slave outputs valid after rising src_clk | Response timing |

## 25.7	Advanced Features

### 25.7.1	Dynamic Clock Gating

Dynamic Clock Gating

| Feature | Description | Benefit |
| --- | --- | --- |
| Automatic Power Reduction | Clocks gated when no activity detected | Significant power savings |
| Configurable Idle Threshold | Programmable idle count before gating | Tunable power/performance |
| Multi-Domain Support | Independent gating for APB (PCLK) and backend (ACLK) | Flexible power management |
| Graceful Handoff | Ready signals forced to zero during gating | Protocol compliance |
| Activity Detection | Monitors valid signals across interfaces | Intelligent gating control |

### 25.7.2	Clock Domain Crossing (CDC) Handshaking

Clock Domain Crossing (CDC) Handshaking

| Feature | Description | Benefit |
| --- | --- | --- |
| Arbitrary Frequency Support | Works across any clock frequency ratios | Design flexibility |
| Robust Reliability | Proven handshaking with proper synchronization | Zero data loss |
| Command/Response Separation | Independent CDC paths | Maximum throughput |
| Skid Buffer Integration | Internal buffering prevents backpressure | Smooth operation |
| Deterministic Latency | Predictable timing characteristics | System predictability |

### 25.7.3	Buffering and Flow Control

Buffering and Flow Control

| Feature | Description | Benefit |
| --- | --- | --- |
| Internal Skid Buffers | Configurable depth buffering | Improved performance |
| Backpressure Handling | Proper ready/valid handshaking | Flow control |
| Command Pipelining | Backend processes while APB handles responses | Efficiency |
| Response Queuing | Responses buffered for varying latencies | Latency tolerance |

## 25.8	Error Handling

### 25.8.1	Error Response Specification

Error Response Specification

| Condition | src_slverr | Description |
| --- | --- | --- |
| Successful access | 0 | Normal completion |
| Address decode error | 1 | Invalid address |
| Protection violation | 1 | Insufficient privilege |
| Backend error | 1 | Downstream error condition |

### 25.8.2	Error Response Timing

Error Response Timing

| Timing Requirement | Description |
| --- | --- |
| src_slverr Validity | Must be valid when src_ready is asserted |
| Error Completion | Error response completes transaction normally |
| Master Responsibility | Master must check src_slverr status |

## 25.9	Reset Behavior

### 25.9.1	Reset Requirements

Reset Requirements

| Reset Phase | Signal Requirements | Description |
| --- | --- | --- |
| Reset Assertion | All outputs to known states | Deterministic reset |
| Reset Deassertion | src_sel=0, src_enable=0 in first cycle | Clean startup |
| Reset Values | src_ready=0, src_slverr=0, src_rdata=0 | Default states |

## 25.10	Implementation Variants

### 25.10.1	Available Implementations

Available Implementations

| Variant | Features | Use Case |
| --- | --- | --- |
| Basic APB Slave | Single clock domain, internal buffering | Simple peripherals |
| CDC-Enabled APB Slave | Dual clock domain (PCLK/ACLK) | Mixed-frequency systems |
| Power-Optimized APB Slave | Dynamic clock gating | Power-sensitive designs |

## 25.11	Performance Characteristics

### 25.11.1	Latency Specifications

Latency Specifications

| Metric | Value | Description |
| --- | --- | --- |
| Minimum Read Latency | 2 clock cycles | Setup + access phases |
| Minimum Write Latency | 2 clock cycles | Setup + access phases |
| CDC Additional Latency | 2-6 clock cycles | Depends on clock relationships |
| Buffer Latency | Minimal | Skid buffer design |

### 25.11.2	Throughput Specifications

Throughput Specifications

| Metric | Value | Description |
| --- | --- | --- |
| Single Transaction | 2 clocks minimum | Basic transaction time |
| Back-to-back Transactions | Limited by src_ready | Depends on peripheral |
| CDC Throughput | Maintained across domains | Proper buffering |

### 25.11.3	Power Consumption

Power Consumption

| Power Category | Characteristics | Description |
| --- | --- | --- |
| Active Power | Standard CMOS logic | Normal operation |
| Idle Power | Significantly reduced with clock gating | Power optimization |
| Gating Overhead | Minimal additional logic | Efficient implementation |

## 25.12	Validation Requirements

### 25.12.1	Functional Validation

Functional Validation

| Validation Area | Requirements |
| --- | --- |
| Protocol Compliance | Verify 2-phase setup/access sequence |
| Address Decode | Verify 11-bit address handling and alignment |
| Error Response | Verify src_slverr generation and handling |
| Wait States | Verify src_ready functionality |
| CDC Operation | Verify cross-clock domain transfers |
| Clock Gating | Verify power management behavior |
| Buffer Operation | Verify skid buffer and flow control |

### 25.12.2	Timing Validation

Timing Validation

| Validation Area | Requirements |
| --- | --- |
| Setup/Hold | Verify signal timing requirements |
| Reset Sequence | Verify proper reset behavior |
| Multi-Clock | Verify CDC timing across frequency ranges |
| Gating Transitions | Verify clock enable/disable timing |

## 25.13	Example Transactions

### 25.13.1	32-bit Register Write

32-bit Register Write

| Clock Cycle | src_sel | src_enable | src_write | src_addr | src_wdata | src_ready | Phase |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 1 | 0x100 | 0xABCD | 0 | Setup |
| 2 | 1 | 1 | 1 | 0x100 | 0xABCD | 1 | Access |
| 3 | 0 | 0 | - | - | - | 0 | Idle |

### 25.13.2	32-bit Register Read with Wait State

32-bit Register Read with Wait State —

| Clock Cycle | src_sel | src_enable | src_write | src_addr | src_rdata | src_ready | Phase |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0x104 | - | 0 | Setup |
| 2 | 1 | 1 | 0 | 0x104 | - | 0 | Access |
| 3 | 1 | 1 | 0 | 0x104 | 0x1234 | 1 | Complete |
| 4 | 0 | 0 | - | - | - | 0 | Idle |

## 25.14	Revision History

APB Programming Interface Specification for STREAM Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Monitor Bus Architecture and Event Code Organization

## 26.1	STREAM-Specific Context

For STREAM DMA Engine: This document describes the generic Monitor Bus (MonBus) protocol used across all AMBA-based components. STREAM uses MonBus for unified event reporting from: - Descriptor engines (8 sources, agent IDs 16-23) - Schedulers (8 sources, agent IDs 48-55) - Descriptor AXI monitor (agent ID 8) - Read/Write AXI monitors (configurable agent IDs)

All STREAM events use Unit ID = 1 and follow the standard 64-bit packet format defined below.

## 26.2	Overview

The Monitor Bus architecture provides a unified, scalable framework for monitoring and error reporting across multiple bus protocols in complex SoC designs. This system supports AXI, APB, MNOC (Mesh Network on Chip), ARB (Arbiter), CORE, and custom protocols through a standardized 64-bit packet format with protocol-aware event categorization.

## 26.3	Interface Summary

### 26.3.1	Number of Interfaces

- 1 Monitor Bus Output Interface: Unified 64-bit packet stream

- Multiple Protocol Input Interfaces: AXI, APB, MNOC, ARB, CORE, Custom protocol monitors

- Local Memory Interface: Error/interrupt packet storage

- External Memory Interface: Bulk packet storage

### 26.3.2	Interface Parameters

Interface Parameters

| Parameter | Description | Valid Values | Default |
| --- | --- | --- | --- |
| PACKET_WIDTH | Monitor bus packet width | 64 | 64 |
| PROTOCOL_WIDTH | Protocol identifier width | 3 | 3 |
| EVENT_CODE_WIDTH | Event code width | 4 | 4 |
| PACKET_TYPE_WIDTH | Packet type width | 4 | 4 |
| CHANNEL_ID_WIDTH | Channel identifier width | 6 | 6 |
| UNIT_ID_WIDTH | Unit identifier width | 4 | 4 |
| AGENT_ID_WIDTH | Agent identifier width | 8 | 8 |
| EVENT_DATA_WIDTH | Event data width | 35 | 35 |

## 26.4	Core Design Assumptions

### 26.4.1	Assumption 1: Hierarchical Event Organization

Assumption 1: Hierarchical Event Organization

| Aspect | Requirement |
| --- | --- |
| Organization Rule | Protocol → Packet Type → Event Code hierarchy |
| Event Space | Each protocol × packet type combination has exactly 16 event codes |
| Mapping | 1:1 mapping between packet types and event codes |
| Rationale | Provides clear, scalable event organization |

### 26.4.2	Assumption 2: Protocol Isolation

Assumption 2: Protocol Isolation

| Aspect | Requirement |
| --- | --- |
| Isolation Rule | Each protocol owns its event space |
| Conflict Prevention | No cross-protocol event conflicts |
| Independent Evolution | Protocols can evolve independently |
| Rationale | Prevents interference and enables protocol-specific optimization |

### 26.4.3	Assumption 3: Two-Tier Memory Architecture

Assumption 3: Two-Tier Memory Architecture

| Aspect | Requirement |
| --- | --- |
| Local Storage | Critical events (errors/interrupts) stored locally |
| External Storage | Non-critical events routed to external memory |
| Routing Decision | Based on packet type configuration |
| Rationale | Balances immediate access with bulk storage needs |

### 26.4.4	Assumption 4: Configurable Packet Routing

Assumption 4: Configurable Packet Routing

| Aspect | Requirement |
| --- | --- |
| Routing Rule | Different packet types can route to different destinations |
| Configuration | Base/limit registers define routing per packet type |
| Priority Support | Configurable priority levels per packet type |
| Rationale | Enables flexible memory allocation and access patterns |

## 26.5	Interface Signal Specification

### 26.5.1	Monitor Bus Output Interface

Monitor Bus Output Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| mon_packet | 64 | Monitor→System | Monitor packet data |
| mon_valid | 1 | Monitor→System | Packet valid signal |
| mon_ready | 1 | System→Monitor | Ready to accept packet |
| mon_error | 1 | Monitor→System | Monitor error condition |

### 26.5.2	Protocol Input Interfaces

Protocol Input Interfaces

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| axi_event | 64 | AXI Monitor→Bus | AXI event packet |
| axi_event_valid | 1 | AXI Monitor→Bus | AXI event valid |
| axi_event_ready | 1 | Bus→AXI Monitor | Ready for AXI event |
| apb_event | 64 | APB Monitor→Bus | APB event packet |
| apb_event_valid | 1 | APB Monitor→Bus | APB event valid |
| apb_event_ready | 1 | Bus→APB Monitor | Ready for APB event |
| mnoc_event | 64 | MNOC Monitor→Bus | MNOC event packet |
| mnoc_event_valid | 1 | MNOC Monitor→Bus | MNOC event valid |
| mnoc_event_ready | 1 | Bus→MNOC Monitor | Ready for MNOC event |
| arb_event | 64 | ARB Monitor→Bus | ARB event packet |
| arb_event_valid | 1 | ARB Monitor→Bus | ARB event valid |
| arb_event_ready | 1 | Bus→ARB Monitor | Ready for ARB event |
| core_event | 64 | CORE Monitor→Bus | CORE event packet |
| core_event_valid | 1 | CORE Monitor→Bus | CORE event valid |
| core_event_ready | 1 | Bus→CORE Monitor | Ready for CORE event |

### 26.5.3	Control and Status Signals

Control and Status Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | System clock |
| resetn | 1 | Input | Active-low reset |
| monitor_enable | 1 | Input | Global monitor enable |
| packet_type_enables | 16 | Input | Per-type enable bits |
| local_memory_full | 1 | Output | Local memory full flag |
| external_memory_error | 1 | Output | External memory error |

## 26.6	Packet Format and Field Allocation

### 26.6.1	64-bit Monitor Bus Packet Structure

64-bit Monitor Bus Packet Structure

| Field | Bits | Width | Description |
| --- | --- | --- | --- |
| Packet Type | [63:60] | 4 | Event category (Error, Completion, etc.) |
| Protocol | [59:57] | 3 | Bus protocol (AXI=0, MNOC=1, APB=2, ARB=3, CORE=4) |
| Event Code | [56:53] | 4 | Specific events within category |
| Channel ID | [52:47] | 6 | Transaction/channel identifier |
| Unit ID | [46:43] | 4 | Subsystem identifier |
| Agent ID | [42:35] | 8 | Module identifier |
| Event Data | [34:0] | 35 | Event-specific payload |

### 26.6.2	Packet Type Definitions

Packet Type Definitions

| Value | Name | Purpose | Applicable Protocols |
| --- | --- | --- | --- |
| 0x0 | Error | Protocol violations, response errors | All |
| 0x1 | Completion | Successful transaction completion | All |
| 0x2 | Threshold | Threshold crossed events | All |
| 0x3 | Timeout | Timeout conditions | All |
| 0x4 | Performance | Performance metrics | All |
| 0x5 | Credit | Credit management | MNOC only |
| 0x6 | Channel | Channel status | MNOC only |
| 0x7 | Stream | Stream events | MNOC only |
| 0x8 | Address Match | Address matching | AXI only |
| 0x9 | APB Specific | APB protocol events | APB only |
| 0xA-0xE | Reserved | Future expansion | - |
| 0xF | Debug | Debug and trace events | All |

## 26.7	Protocol-Specific Event Codes

### 26.7.1	AXI Protocol Events

#### 26.7.1.1	Error Events (PktTypeError + PROTOCOL_AXI)

Error Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | AXI_ERR_RESP_SLVERR | Slave error response |
| 0x1 | AXI_ERR_RESP_DECERR | Decode error response |
| 0x2 | AXI_ERR_DATA_ORPHAN | Data without command |
| 0x3 | AXI_ERR_RESP_ORPHAN | Response without transaction |
| 0x4 | AXI_ERR_PROTOCOL | Protocol violation |
| 0x5 | AXI_ERR_BURST_LENGTH | Invalid burst length |
| 0x6 | AXI_ERR_BURST_SIZE | Invalid burst size |
| 0x7 | AXI_ERR_BURST_TYPE | Invalid burst type |
| 0x8 | AXI_ERR_ID_COLLISION | ID collision detected |
| 0x9 | AXI_ERR_WRITE_BEFORE_ADDR | Write data before address |
| 0xA | AXI_ERR_RESP_BEFORE_DATA | Response before data complete |
| 0xB | AXI_ERR_LAST_MISSING | Missing LAST signal |
| 0xC | AXI_ERR_STROBE_ERROR | Write strobe error |
| 0xD | AXI_ERR_RESERVED_D | Reserved |
| 0xE | AXI_ERR_RESERVED_E | Reserved |
| 0xF | AXI_ERR_USER_DEFINED | User-defined error |

#### 26.7.1.2	Timeout Events (PktTypeTimeout + PROTOCOL_AXI)

Timeout Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | AXI_TIMEOUT_CMD | Command/Address timeout |
| 0x1 | AXI_TIMEOUT_DATA | Data timeout |
| 0x2 | AXI_TIMEOUT_RESP | Response timeout |
| 0x3 | AXI_TIMEOUT_HANDSHAKE | Handshake timeout |
| 0x4 | AXI_TIMEOUT_BURST | Burst completion timeout |
| 0x5 | AXI_TIMEOUT_EXCLUSIVE | Exclusive access timeout |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | AXI_TIMEOUT_USER_DEFINED | User-defined timeout |

#### 26.7.1.3	Performance Events (PktTypePerf + PROTOCOL_AXI)

Performance Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | AXI_PERF_ADDR_LATENCY | Address phase latency |
| 0x1 | AXI_PERF_DATA_LATENCY | Data phase latency |
| 0x2 | AXI_PERF_RESP_LATENCY | Response phase latency |
| 0x3 | AXI_PERF_TOTAL_LATENCY | Total transaction latency |
| 0x4 | AXI_PERF_THROUGHPUT | Transaction throughput |
| 0x5 | AXI_PERF_ERROR_RATE | Error rate |
| 0x6 | AXI_PERF_ACTIVE_COUNT | Active transaction count |
| 0x7 | AXI_PERF_BANDWIDTH_UTIL | Bandwidth utilization |
| 0x8 | AXI_PERF_QUEUE_DEPTH | Average queue depth |
| 0x9 | AXI_PERF_BURST_EFFICIENCY | Burst efficiency metric |
| 0xA-0xE | Reserved | Future expansion |
| 0xF | AXI_PERF_USER_DEFINED | User-defined performance |

### 26.7.2	APB Protocol Events

#### 26.7.2.1	Error Events (PktTypeError + PROTOCOL_APB)

Error Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_ERR_PSLVERR | Peripheral slave error |
| 0x1 | APB_ERR_SETUP_VIOLATION | Setup phase protocol violation |
| 0x2 | APB_ERR_ACCESS_VIOLATION | Access phase protocol violation |
| 0x3 | APB_ERR_STROBE_ERROR | Write strobe error |
| 0x4 | APB_ERR_ADDR_DECODE | Address decode error |
| 0x5 | APB_ERR_PROT_VIOLATION | Protection violation (PPROT) |
| 0x6 | APB_ERR_ENABLE_ERROR | Enable phase error |
| 0x7 | APB_ERR_READY_ERROR | PREADY protocol error |
| 0x8-0xE | Reserved | Future expansion |
| 0xF | APB_ERR_USER_DEFINED | User-defined error |

#### 26.7.2.2	Timeout Events (PktTypeTimeout + PROTOCOL_APB)

Timeout Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_TIMEOUT_SETUP | Setup phase timeout |
| 0x1 | APB_TIMEOUT_ACCESS | Access phase timeout |
| 0x2 | APB_TIMEOUT_ENABLE | Enable phase timeout (PREADY stuck) |
| 0x3 | APB_TIMEOUT_PREADY_STUCK | PREADY stuck low |
| 0x4 | APB_TIMEOUT_TRANSFER | Overall transfer timeout |
| 0x5-0xE | Reserved | Future expansion |
| 0xF | APB_TIMEOUT_USER_DEFINED | User-defined timeout |

#### 26.7.2.3	Completion Events (PktTypeCompletion + PROTOCOL_APB)

Completion Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_COMPL_TRANS_COMPLETE | Transaction completed |
| 0x1 | APB_COMPL_READ_COMPLETE | Read transaction complete |
| 0x2 | APB_COMPL_WRITE_COMPLETE | Write transaction complete |
| 0x3-0xE | Reserved | Future expansion |
| 0xF | APB_COMPL_USER_DEFINED | User-defined completion |

#### 26.7.2.4	Threshold Events (PktTypeThreshold + PROTOCOL_APB)

Threshold Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_THRESH_LATENCY | APB latency threshold |
| 0x1 | APB_THRESH_ERROR_RATE | APB error rate threshold |
| 0x2 | APB_THRESH_ACCESS_COUNT | Access count threshold |
| 0x3 | APB_THRESH_BANDWIDTH | Bandwidth threshold |
| 0x4-0xE | Reserved | Future expansion |
| 0xF | APB_THRESH_USER_DEFINED | User-defined threshold |

#### 26.7.2.5	Performance Events (PktTypePerf + PROTOCOL_APB)

Performance Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_PERF_READ_LATENCY | Read transaction latency |
| 0x1 | APB_PERF_WRITE_LATENCY | Write transaction latency |
| 0x2 | APB_PERF_THROUGHPUT | Transaction throughput |
| 0x3 | APB_PERF_ERROR_RATE | Error rate |
| 0x4 | APB_PERF_ACTIVE_COUNT | Active transaction count |
| 0x5 | APB_PERF_COMPLETED_COUNT | Completed transaction count |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | APB_PERF_USER_DEFINED | User-defined performance |

#### 26.7.2.6	Debug Events (PktTypeDebug + PROTOCOL_APB)

Debug Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | APB_DEBUG_STATE_CHANGE | APB state changed |
| 0x1 | APB_DEBUG_SETUP_PHASE | Setup phase event |
| 0x2 | APB_DEBUG_ACCESS_PHASE | Access phase event |
| 0x3 | APB_DEBUG_ENABLE_PHASE | Enable phase event |
| 0x4 | APB_DEBUG_PSEL_TRACE | PSEL trace |
| 0x5 | APB_DEBUG_PENABLE_TRACE | PENABLE trace |
| 0x6 | APB_DEBUG_PREADY_TRACE | PREADY trace |
| 0x7 | APB_DEBUG_PPROT_TRACE | PPROT trace |
| 0x8 | APB_DEBUG_PSTRB_TRACE | PSTRB trace |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | APB_DEBUG_USER_DEFINED | User-defined debug |

### 26.7.3	MNOC Protocol Events

#### 26.7.3.1	Error Events (PktTypeError + PROTOCOL_MNOC)

Error Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | MNOC_ERR_PARITY | Parity error |
| 0x1 | MNOC_ERR_PROTOCOL | Protocol violation |
| 0x2 | MNOC_ERR_OVERFLOW | Buffer/Credit overflow |
| 0x3 | MNOC_ERR_UNDERFLOW | Buffer/Credit underflow |
| 0x4 | MNOC_ERR_ORPHAN | Orphaned packet/ACK |
| 0x5 | MNOC_ERR_INVALID | Invalid type/channel/payload |
| 0x6 | MNOC_ERR_HEADER_CRC | Header CRC error |
| 0x7 | MNOC_ERR_PAYLOAD_CRC | Payload CRC error |
| 0x8 | MNOC_ERR_SEQUENCE | Sequence number error |
| 0x9 | MNOC_ERR_ROUTE | Routing error |
| 0xA | MNOC_ERR_DEADLOCK | Deadlock detected |
| 0xB-0xE | Reserved | Future expansion |
| 0xF | MNOC_ERR_USER_DEFINED | User-defined error |

#### 26.7.3.2	Credit Events (PktTypeCredit + PROTOCOL_MNOC)

Credit Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | MNOC_CREDIT_ALLOCATED | Credits allocated |
| 0x1 | MNOC_CREDIT_CONSUMED | Credits consumed |
| 0x2 | MNOC_CREDIT_RETURNED | Credits returned |
| 0x3 | MNOC_CREDIT_OVERFLOW | Credit overflow detected |
| 0x4 | MNOC_CREDIT_UNDERFLOW | Credit underflow detected |
| 0x5 | MNOC_CREDIT_EXHAUSTED | All credits exhausted |
| 0x6 | MNOC_CREDIT_RESTORED | Credits restored |
| 0x7 | MNOC_CREDIT_EFFICIENCY | Credit efficiency metric |
| 0x8 | MNOC_CREDIT_LEAK | Credit leak detected |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | MNOC_CREDIT_USER_DEFINED | User-defined credit event |

#### 26.7.3.3	Channel Events (PktTypeChannel + PROTOCOL_MNOC)

Channel Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | MNOC_CHANNEL_OPEN | Channel opened |
| 0x1 | MNOC_CHANNEL_CLOSE | Channel closed |
| 0x2 | MNOC_CHANNEL_STALL | Channel stalled |
| 0x3 | MNOC_CHANNEL_RESUME | Channel resumed |
| 0x4 | MNOC_CHANNEL_CONGESTION | Channel congestion detected |
| 0x5 | MNOC_CHANNEL_PRIORITY | Channel priority change |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | MNOC_CHANNEL_USER_DEFINED | User-defined channel event |

#### 26.7.3.4	Stream Events (PktTypeStream + PROTOCOL_MNOC)

Stream Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | MNOC_STREAM_START | Stream started |
| 0x1 | MNOC_STREAM_END | Stream ended (EOS) |
| 0x2 | MNOC_STREAM_PAUSE | Stream paused |
| 0x3 | MNOC_STREAM_RESUME | Stream resumed |
| 0x4 | MNOC_STREAM_OVERFLOW | Stream buffer overflow |
| 0x5 | MNOC_STREAM_UNDERFLOW | Stream buffer underflow |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | MNOC_STREAM_USER_DEFINED | User-defined stream event |

### 26.7.4	ARB Protocol Events

#### 26.7.4.1	Error Events (PktTypeError + PROTOCOL_ARB)

Error Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_ERR_STARVATION | Client request starvation |
| 0x1 | ARB_ERR_ACK_TIMEOUT | Grant ACK timeout |
| 0x2 | ARB_ERR_PROTOCOL_VIOLATION | ACK protocol violation |
| 0x3 | ARB_ERR_CREDIT_VIOLATION | Credit system violation |
| 0x4 | ARB_ERR_FAIRNESS_VIOLATION | Weighted fairness violation |
| 0x5 | ARB_ERR_WEIGHT_UNDERFLOW | Weight credit underflow |
| 0x6 | ARB_ERR_CONCURRENT_GRANTS | Multiple simultaneous grants |
| 0x7 | ARB_ERR_INVALID_GRANT_ID | Invalid grant ID detected |
| 0x8 | ARB_ERR_ORPHAN_ACK | ACK without pending grant |
| 0x9 | ARB_ERR_GRANT_OVERLAP | Overlapping grant periods |
| 0xA | ARB_ERR_MASK_ERROR | Round-robin mask error |
| 0xB | ARB_ERR_STATE_MACHINE | FSM state error |
| 0xC | ARB_ERR_CONFIGURATION | Invalid configuration |
| 0xD-0xE | Reserved | Future expansion |
| 0xF | ARB_ERR_USER_DEFINED | User-defined error |

#### 26.7.4.2	Timeout Events (PktTypeTimeout + PROTOCOL_ARB)

Timeout Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_TIMEOUT_GRANT_ACK | Grant ACK timeout |
| 0x1 | ARB_TIMEOUT_REQUEST_HOLD | Request held too long |
| 0x2 | ARB_TIMEOUT_WEIGHT_UPDATE | Weight update timeout |
| 0x3 | ARB_TIMEOUT_BLOCK_RELEASE | Block release timeout |
| 0x4 | ARB_TIMEOUT_CREDIT_UPDATE | Credit update timeout |
| 0x5 | ARB_TIMEOUT_STATE_CHANGE | State machine timeout |
| 0x6-0xE | Reserved | Future expansion |
| 0xF | ARB_TIMEOUT_USER_DEFINED | User-defined timeout |

#### 26.7.4.3	Completion Events (PktTypeCompletion + PROTOCOL_ARB)

Completion Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_COMPL_GRANT_ISSUED | Grant successfully issued |
| 0x1 | ARB_COMPL_ACK_RECEIVED | ACK successfully received |
| 0x2 | ARB_COMPL_TRANSACTION | Complete transaction (grant+ack) |
| 0x3 | ARB_COMPL_WEIGHT_UPDATE | Weight update completed |
| 0x4 | ARB_COMPL_CREDIT_CYCLE | Credit cycle completed |
| 0x5 | ARB_COMPL_FAIRNESS_PERIOD | Fairness analysis period |
| 0x6 | ARB_COMPL_BLOCK_PERIOD | Block period completed |
| 0x7-0xE | Reserved | Future expansion |
| 0xF | ARB_COMPL_USER_DEFINED | User-defined completion |

#### 26.7.4.4	Threshold Events (PktTypeThreshold + PROTOCOL_ARB)

Threshold Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_THRESH_REQUEST_LATENCY | Request-to-grant latency threshold |
| 0x1 | ARB_THRESH_ACK_LATENCY | Grant-to-ACK latency threshold |
| 0x2 | ARB_THRESH_FAIRNESS_DEV | Fairness deviation threshold |
| 0x3 | ARB_THRESH_ACTIVE_REQUESTS | Active request count threshold |
| 0x4 | ARB_THRESH_GRANT_RATE | Grant rate threshold |
| 0x5 | ARB_THRESH_EFFICIENCY | Grant efficiency threshold |
| 0x6 | ARB_THRESH_CREDIT_LOW | Low credit threshold |
| 0x7 | ARB_THRESH_WEIGHT_IMBALANCE | Weight imbalance threshold |
| 0x8 | ARB_THRESH_STARVATION_TIME | Starvation time threshold |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | ARB_THRESH_USER_DEFINED | User-defined threshold |

#### 26.7.4.5	Performance Events (PktTypePerf + PROTOCOL_ARB)

Performance Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_PERF_GRANT_ISSUED | Grant issued event |
| 0x1 | ARB_PERF_ACK_RECEIVED | ACK received event |
| 0x2 | ARB_PERF_GRANT_EFFICIENCY | Grant completion efficiency |
| 0x3 | ARB_PERF_FAIRNESS_METRIC | Fairness compliance metric |
| 0x4 | ARB_PERF_THROUGHPUT | Arbitration throughput |
| 0x5 | ARB_PERF_LATENCY_AVG | Average latency measurement |
| 0x6 | ARB_PERF_WEIGHT_COMPLIANCE | Weight compliance metric |
| 0x7 | ARB_PERF_CREDIT_UTILIZATION | Credit utilization efficiency |
| 0x8 | ARB_PERF_CLIENT_ACTIVITY | Per-client activity metric |
| 0x9 | ARB_PERF_STARVATION_COUNT | Starvation event count |
| 0xA | ARB_PERF_BLOCK_EFFICIENCY | Block/unblock efficiency |
| 0xB-0xE | Reserved | Future expansion |
| 0xF | ARB_PERF_USER_DEFINED | User-defined performance |

#### 26.7.4.6	Debug Events (PktTypeDebug + PROTOCOL_ARB)

Debug Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | ARB_DEBUG_STATE_CHANGE | Arbiter state machine change |
| 0x1 | ARB_DEBUG_MASK_UPDATE | Round-robin mask update |
| 0x2 | ARB_DEBUG_WEIGHT_CHANGE | Weight configuration change |
| 0x3 | ARB_DEBUG_CREDIT_UPDATE | Credit level update |
| 0x4 | ARB_DEBUG_CLIENT_MASK | Client enable/disable mask |
| 0x5 | ARB_DEBUG_PRIORITY_CHANGE | Priority level change |
| 0x6 | ARB_DEBUG_BLOCK_EVENT | Block/unblock event |
| 0x7 | ARB_DEBUG_QUEUE_STATUS | Request queue status |
| 0x8 | ARB_DEBUG_COUNTER_SNAPSHOT | Counter values snapshot |
| 0x9 | ARB_DEBUG_FIFO_STATUS | FIFO status change |
| 0xA | ARB_DEBUG_FAIRNESS_STATE | Fairness tracking state |
| 0xB | ARB_DEBUG_ACK_STATE | ACK protocol state |
| 0xC-0xE | Reserved | Future expansion |
| 0xF | ARB_DEBUG_USER_DEFINED | User-defined debug |

### 26.7.5	CORE Protocol Events

#### 26.7.5.1	Error Events (PktTypeError + PROTOCOL_CORE)

Error Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_ERR_DESCRIPTOR_MALFORMED | Missing magic number (0x900dc0de) |
| 0x1 | CORE_ERR_DESCRIPTOR_BAD_ADDR | Invalid descriptor address |
| 0x2 | CORE_ERR_DATA_BAD_ADDR | Invalid data address (fetch or runtime) |
| 0x3 | CORE_ERR_FLAG_COMPARISON | Flag mask/compare mismatch |
| 0x4 | CORE_ERR_CREDIT_UNDERFLOW | Credit system violation |
| 0x5 | CORE_ERR_STATE_MACHINE | Invalid FSM state transition |
| 0x6 | CORE_ERR_DESCRIPTOR_ENGINE | Descriptor engine FSM error |
| 0x7 | CORE_ERR_FLAG_ENGINE | Flag engine FSM error |
| 0x8 | CORE_ERR_PROGRAM_ENGINE | Program engine FSM error |
| 0x9 | CORE_ERR_DATA_ENGINE | Data engine error |
| 0xA | CORE_ERR_CHANNEL_INVALID | Invalid channel ID |
| 0xB | CORE_ERR_CONTROL_VIOLATION | Control register violation |
| 0xC-0xE | Reserved | Future expansion |
| 0xF | CORE_ERR_USER_DEFINED | User-defined error |

#### 26.7.5.2	Timeout Events (PktTypeTimeout + PROTOCOL_CORE)

Timeout Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_TIMEOUT_DESCRIPTOR_FETCH | Descriptor fetch timeout |
| 0x1 | CORE_TIMEOUT_FLAG_RETRY | Flag comparison retry timeout |
| 0x2 | CORE_TIMEOUT_PROGRAM_WRITE | Program write timeout |
| 0x3 | CORE_TIMEOUT_DATA_TRANSFER | Data transfer timeout |
| 0x4 | CORE_TIMEOUT_CREDIT_WAIT | Credit wait timeout |
| 0x5 | CORE_TIMEOUT_CONTROL_WAIT | Control enable wait timeout |
| 0x6 | CORE_TIMEOUT_ENGINE_RESPONSE | Sub-engine response timeout |
| 0x7 | CORE_TIMEOUT_STATE_TRANSITION | FSM state transition timeout |
| 0x8-0xE | Reserved | Future expansion |
| 0xF | CORE_TIMEOUT_USER_DEFINED | User-defined timeout |

#### 26.7.5.3	Completion Events (PktTypeCompletion + PROTOCOL_CORE)

Completion Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_COMPL_DESCRIPTOR_LOADED | Descriptor successfully loaded |
| 0x1 | CORE_COMPL_DESCRIPTOR_CHAIN | Descriptor chain completed |
| 0x2 | CORE_COMPL_FLAG_MATCHED | Flag comparison successful |
| 0x3 | CORE_COMPL_PROGRAM_COMPLETED | Post-programming completed |
| 0x4 | CORE_COMPL_DATA_TRANSFER | Data transfer completed |
| 0x5 | CORE_COMPL_CREDIT_CYCLE | Credit cycle completed |
| 0x6 | CORE_COMPL_CHANNEL_COMPLETE | Channel processing complete |
| 0x7 | CORE_COMPL_ENGINE_READY | Sub-engine ready |
| 0x8-0xE | Reserved | Future expansion |
| 0xF | CORE_COMPL_USER_DEFINED | User-defined completion |

#### 26.7.5.4	Threshold Events (PktTypeThreshold + PROTOCOL_CORE)

Threshold Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_THRESH_DESCRIPTOR_QUEUE | Descriptor queue depth threshold |
| 0x1 | CORE_THRESH_CREDIT_LOW | Credit low threshold |
| 0x2 | CORE_THRESH_FLAG_RETRY_COUNT | Flag retry count threshold |
| 0x3 | CORE_THRESH_LATENCY | Processing latency threshold |
| 0x4 | CORE_THRESH_ERROR_RATE | Error rate threshold |
| 0x5 | CORE_THRESH_THROUGHPUT | Throughput threshold |
| 0x6 | CORE_THRESH_ACTIVE_CHANNELS | Active channel count threshold |
| 0x7 | CORE_THRESH_PROGRAM_LATENCY | Program write latency threshold |
| 0x8 | CORE_THRESH_DATA_RATE | Data transfer rate threshold |
| 0x9-0xE | Reserved | Future expansion |
| 0xF | CORE_THRESH_USER_DEFINED | User-defined threshold |

#### 26.7.5.5	Performance Events (PktTypePerf + PROTOCOL_CORE)

Performance Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_PERF_END_OF_DATA | Stream continuation signal |
| 0x1 | CORE_PERF_END_OF_STREAM | Stream termination signal |
| 0x2 | CORE_PERF_ENTERING_IDLE | FSM returning to idle |
| 0x3 | CORE_PERF_CREDIT_INCREMENTED | Credit added by software |
| 0x4 | CORE_PERF_CREDIT_EXHAUSTED | Credit blocking execution |
| 0x5 | CORE_PERF_STATE_TRANSITION | FSM state change |
| 0x6 | CORE_PERF_DESCRIPTOR_ACTIVE | Data processing started |
| 0x7 | CORE_PERF_FLAG_RETRY | Flag comparison retry |
| 0x8 | CORE_PERF_CHANNEL_ENABLE | Channel enabled by software |
| 0x9 | CORE_PERF_CHANNEL_DISABLE | Channel disabled by software |
| 0xA | CORE_PERF_CREDIT_UTILIZATION | Credit utilization metric |
| 0xB | CORE_PERF_PROCESSING_LATENCY | Total processing latency |
| 0xC | CORE_PERF_QUEUE_DEPTH | Current queue depth |
| 0xD-0xE | Reserved | Future expansion |
| 0xF | CORE_PERF_USER_DEFINED | User-defined performance |

#### 26.7.5.6	Debug Events (PktTypeDebug + PROTOCOL_CORE)

Debug Events

| Code | Event Name | Description |
| --- | --- | --- |
| 0x0 | CORE_DEBUG_FSM_STATE_CHANGE | Descriptor FSM state change |
| 0x1 | CORE_DEBUG_DESCRIPTOR_CONTENT | Descriptor content trace |
| 0x2 | CORE_DEBUG_FLAG_ENGINE_STATE | Flag engine state trace |
| 0x3 | CORE_DEBUG_PROGRAM_ENGINE_STATE | Program engine state trace |
| 0x4 | CORE_DEBUG_CREDIT_OPERATION | Credit system operation |
| 0x5 | CORE_DEBUG_CONTROL_REGISTER | Control register access |
| 0x6 | CORE_DEBUG_ENGINE_HANDSHAKE | Engine handshake trace |
| 0x7 | CORE_DEBUG_QUEUE_STATUS | Queue status change |
| 0x8 | CORE_DEBUG_COUNTER_SNAPSHOT | Counter values snapshot |
| 0x9 | CORE_DEBUG_ADDRESS_TRACE | Address progression trace |
| 0xA | CORE_DEBUG_PAYLOAD_TRACE | Payload content trace |
| 0xB-0xE | Reserved | Future expansion |
| 0xF | CORE_DEBUG_USER_DEFINED | User-defined debug |

## 26.8	Memory Architecture and Packet Routing

### 26.8.1	Two-Tier Memory Architecture

#### 26.8.1.1	Local Error/Interrupt Memory

Local Error/Interrupt Memory

| Characteristic | Description |
| --- | --- |
| Storage Types | Error Packets (Type 0x0) and Timeout Packets (Type 0x3) |
| Access Method | Immediate CPU access without memory subsystem delays |
| Capacity | Large enough to prevent overflow during error bursts |
| Priority | Critical events requiring immediate attention |
| Indexing | Fast search and retrieval mechanisms |

#### 26.8.1.2	Configurable External Memory

Configurable External Memory

| Characteristic | Description |
| --- | --- |
| Storage Types | Performance, Completion, Threshold, Debug packets |
| Access Method | Base and limit registers define memory regions |
| Capacity | Bulk storage for non-critical events |
| DMA Support | Can be accessed via DMA for efficient transfer |
| Time Stamping | 32-bit timestamp appended when routing externally |

### 26.8.2	Routing Configuration

#### 26.8.2.1	Base and Limit Registers

Base and Limit Registers

| Register Set | Purpose | Configuration |
| --- | --- | --- |
| Completion Config | Type 0x1 routing | base_addr, limit_addr, enable, priority |
| Threshold Config | Type 0x2 routing | base_addr, limit_addr, enable, priority |
| Performance Config | Type 0x4 routing | base_addr, limit_addr, enable, priority |
| Debug Config | Type 0xF routing | base_addr, limit_addr, enable, priority |

#### 26.8.2.2	Routing Decision Logic

Routing Decision Logic

| Packet Type | Destination | Address Calculation |
| --- | --- | --- |
| Error (0x0) | Local Memory | local_error_write_pointer |
| Timeout (0x3) | Local Memory | local_error_write_pointer |
| Completion (0x1) | External Memory | completion_config.base_addr + offset |
| Performance (0x4) | External Memory | performance_config.base_addr + offset |
| Debug (0xF) | External Memory | debug_config.base_addr + offset |

### 26.8.3	Address Space Management

#### 26.8.3.1	Memory Layout Example

Memory Layout Example

| Address Range | Usage | Description |
| --- | --- | --- |
| 0x1000_0000 - 0x1000_FFFF | Local Error Memory | Immediate access storage |
| 0x2000_0000 - 0x2001_FFFF | Performance Packets | External bulk storage |
| 0x2010_0000 - 0x2011_FFFF | Completion Packets | External bulk storage |
| 0x2020_0000 - 0x202F_FFFF | Debug Packets | External bulk storage |

### 26.8.4	Transaction State and Bus Transaction Structure

#### 26.8.4.1	Transaction State Enumeration

Transaction State Enumeration

| State | Value | Description | Usage |
| --- | --- | --- | --- |
| TRANS_EMPTY | 3’b000 | Unused entry | Available slot |
| TRANS_ADDR_PHASE | 3’b001 | Address phase active (AXI) / Packet sent (MNOC) / Setup phase (APB) | Initial phase |
| TRANS_DATA_PHASE | 3’b010 | Data phase active (AXI) / Waiting for ACK (MNOC) / Access phase (APB) | Data transfer |
| TRANS_RESP_PHASE | 3’b011 | Response phase active (AXI) / ACK received (MNOC) / Enable phase (APB) | Response handling |
| TRANS_COMPLETE | 3’b100 | Transaction complete | Successful completion |
| TRANS_ERROR | 3’b101 | Transaction has error | Error condition |
| TRANS_ORPHANED | 3’b110 | Orphaned transaction | Missing components |
| TRANS_CREDIT_STALL | 3’b111 | Credit stall (MNOC only) | MNOC-specific stall |

#### 26.8.4.2	Enhanced Transaction Structure

Enhanced Transaction Structure

| Field | Width | Description | Protocol Usage |
| --- | --- | --- | --- |
| valid | 1 | Entry is valid | All protocol’s |
| protocol | 3 | Protocol type (AXI/MNOC/APB/ARB/CORE) | All protocols |
| state | 3 | Transaction state | All protocols |
| id | 32 | Transaction ID (AXI) / Sequence (MNOC) / PSEL encoding (APB) | All protocols |
| addr | 64 | Transaction address / Channel addr / PADDR | All protocols |
| len | 8 | Burst length (AXI) / Packet count (MNOC) / Always 0 (APB) | AXI, MNOC |
| size | 3 | Access size (AXI) / Reserved (MNOC) / Transfer size (APB) | AXI, APB |
| burst | 2 | Burst type (AXI) / Payload type (MNOC) / PPROT[1:0] (APB) | All protocols |

#### 26.8.4.3	Phase Completion Flags

Phase Completion Flags

| Flag | Description | Protocol Usage |
| --- | --- | --- |
| cmd_received | Address phase received / Packet sent / Setup phase | All protocols |
| data_started | Data phase started / ACK expected / Access phase | All protocols |
| data_completed | Data phase completed / ACK received / Enable phase | All protocols |
| resp_received | Response received / Final ACK / PREADY asserted | All protocols |

#### 26.8.4.4	Protocol-Specific Tracking Fields

Protocol-Specific Tracking Fields

| Field | Width | Description | Protocol |
| --- | --- | --- | --- |
| channel | 6 | Channel ID (AXI ID / MNOC channel / PSEL bit position) | All protocols |
| eos_seen | 1 | EOS marker seen | MNOC only |
| parity_error | 1 | Parity error detected | MNOC only |
| credit_at_start | 8 | Credits available at start | MNOC only |
| retry_count | 3 | Number of retries | MNOC only |
| desc_addr_match | 1 | Descriptor address match detected | AXI only |
| data_addr_match | 1 | Data address match detected | AXI only |
| apb_phase | 2 | Current APB phase | APB only |
| pslverr_seen | 1 | PSLVERR detected | APB only |
| pprot_value | 3 | PPROT value | APB only |
| pstrb_value | 4 | PSTRB value for writes | APB only |
| arb_grant_id | 8 | Current grant ID | ARB only |
| arb_weight | 8 | Current weight value | ARB only |
| core_fsm_state | 3 | Current CORE FSM state | CORE only |
| core_channel_id | 6 | CORE channel identifier | CORE only |

### 26.8.5	APB Transaction Phases

APB Transaction Phases

| Phase | Value | Description |
| --- | --- | --- |
| APB_PHASE_IDLE | 2’b00 | Bus idle |
| APB_PHASE_SETUP | 2’b01 | Setup phase (PSEL asserted) |
| APB_PHASE_ACCESS | 2’b10 | Access phase (PENABLE asserted) |
| APB_PHASE_ENABLE | 2’b11 | Enable phase (waiting for PREADY) |

### 26.8.6	APB Protection Types

APB Protection Types

| Protection | Value | Description |
| --- | --- | --- |
| APB_PROT_NORMAL | 3’b000 | Normal access |
| APB_PROT_PRIVILEGED | 3’b001 | Privileged access |
| APB_PROT_SECURE | 3’b010 | Secure access |
| APB_PROT_INSTRUCTION | 3’b100 | Instruction access |

### 26.8.7	MNOC Payload Types

MNOC Payload Types

| Payload | Value | Description |
| --- | --- | --- |
| MNOC_PAYLOAD_CONFIG | 2’b00 | CONFIG_PKT |
| MNOC_PAYLOAD_TS | 2’b01 | TS_PKT |
| MNOC_PAYLOAD_RAW | 2’b11 | RAW_PKT |

### 26.8.8	MNOC ACK Types

MNOC ACK Types

| ACK Type | Value | Description |
| --- | --- | --- |
| MNOC_ACK_STOP | 2’b00 | MSAP_STOP |
| MNOC_ACK_START | 2’b01 | MSAP_START |
| MNOC_ACK_CREDIT_ON | 2’b10 | MSAP_CREDIT_ON |
| MNOC_ACK_STOP_AT_EOS | 2’b11 | MSAP_STOP_AT_EOS |

### 26.8.9	ARB State Types

ARB State Types

| State | Value | Description |
| --- | --- | --- |
| ARB_STATE_IDLE | 3’b000 | Idle state |
| ARB_STATE_ARBITRATE | 3’b001 | Performing arbitration |
| ARB_STATE_GRANT | 3’b010 | Grant issued, waiting for ACK |
| ARB_STATE_BLOCKED | 3’b011 | Arbitration blocked |
| ARB_STATE_WEIGHT_UPD | 3’b100 | Weight update in progress |
| ARB_STATE_ERROR | 3’b101 | Error state |

### 26.8.10	CORE State Types

CORE State Types

| State | Value | Description |
| --- | --- | --- |
| CORE_STATE_IDLE | 3’b000 | Idle state |
| CORE_STATE_DESC_FETCH | 3’b001 | Fetching descriptor |
| CORE_STATE_FLAG_CHECK | 3’b010 | Checking flag condition |
| CORE_STATE_PROGRAM_WRITE | 3’b011 | Writing program |
| CORE_STATE_DATA_TRANSFER | 3’b100 | Transferring data |
| CORE_STATE_CREDIT_WAIT | 3’b101 | Waiting for credits |
| CORE_STATE_ERROR | 3’b110 | Error state |

## 26.9	Configuration and Control

### 26.9.1	Monitor Configuration Registers

#### 26.9.1.1	Global Configuration

Global Configuration

| Field | Width | Description |
| --- | --- | --- |
| monitor_enable | 1 | Global monitor enable |
| error_local_enable | 1 | Enable local error storage |
| external_route_enable | 1 | Enable external routing |
| unit_id | 4 | Unit identifier |
| agent_id | 8 | Agent identifier |
| packet_type_enables | 16 | Per-type enable bits |

#### 26.9.1.2	Packet Type Enable Mapping

Packet Type Enable Mapping

| Bit | Enable | Description |
| --- | --- | --- |
| 0 | PKT_ENABLE_ERROR | Enable error packets |
| 1 | PKT_ENABLE_COMPLETION | Enable completion packets |
| 2 | PKT_ENABLE_THRESHOLD | Enable threshold packets |
| 3 | PKT_ENABLE_TIMEOUT | Enable timeout packets |
| 4 | PKT_ENABLE_PERF | Enable performance packets |
| 5 | PKT_ENABLE_CREDIT | Enable credit packets (MNOC) |
| 6 | PKT_ENABLE_CHANNEL | Enable channel packets (MNOC) |
| 7 | PKT_ENABLE_STREAM | Enable stream packets (MNOC) |
| 8 | PKT_ENABLE_ADDR_MATCH | Enable address match (AXI) |
| 9 | PKT_ENABLE_APB | Enable APB packets |
| 15 | PKT_ENABLE_DEBUG | Enable debug packets |

### 26.9.2	Protocol-Specific Configuration

#### 26.9.2.1	AXI Monitor Configuration

AXI Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| active_trans_threshold | 16 | Active transaction threshold |
| latency_threshold | 32 | Latency threshold (cycles) |
| addr_timeout_cnt | 4 | Address timeout count |
| data_timeout_cnt | 4 | Data timeout count |
| resp_timeout_cnt | 4 | Response timeout count |
| burst_boundary_check | 1 | Enable burst boundary checking |
| address_match_enable | 1 | Enable address matching |
| desc_addr_match_base | 64 | Descriptor address match base |
| desc_addr_match_mask | 64 | Descriptor address match mask |
| data_addr_match_base | 64 | Data address match base |
| data_addr_match_mask | 64 | Data address match mask |

#### 26.9.2.2	MNOC Monitor Configuration

MNOC Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| credit_low_threshold | 8 | Credit low threshold |
| packet_rate_threshold | 16 | Packet rate threshold |
| max_route_hops | 8 | Maximum routing hops |
| enable_credit_tracking | 1 | Enable credit tracking |
| enable_deadlock_detect | 1 | Enable deadlock detection |
| deadlock_timeout | 4 | Deadlock detection timeout |

#### 26.9.2.3	ARB Monitor Configuration

ARB Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| grant_timeout_cnt | 16 | Grant ACK timeout count |
| fairness_window | 32 | Fairness analysis window |
| weight_update_enable | 1 | Enable weight tracking |
| starvation_threshold | 16 | Starvation detection threshold |
| efficiency_threshold | 8 | Grant efficiency threshold |

#### 26.9.2.4	CORE Monitor Configuration

CORE Monitor Configuration

| Field | Width | Description |
| --- | --- | --- |
| descriptor_timeout_cnt | 16 | Descriptor fetch timeout count |
| flag_retry_limit | 8 | Maximum flag retry count |
| credit_low_threshold | 8 | Credit low threshold |
| processing_timeout_cnt | 32 | Processing timeout count |
| enable_descriptor_trace | 1 | Enable descriptor content tracing |
| enable_fsm_trace | 1 | Enable FSM state tracing |

## 26.10	Validation Requirements

### 26.10.1	Functional Validation

Functional Validation

| Validation Area | Requirements |
| --- | --- |
| Packet Format | Verify 64-bit packet structure and field encoding |
| Event Organization | Verify hierarchical event code organization |
| Protocol Isolation | Verify independent protocol event spaces |
| Routing Logic | Verify packet routing based on type and configuration |
| Memory Management | Verify local and external memory operations |
| Configuration | Verify register configuration and enable controls |

### 26.10.2	Performance Validation

Performance Validation

| Validation Area | Requirements |
| --- | --- |
| Throughput | Verify monitor bus can handle peak event rates |
| Latency | Verify low-latency path for critical events |
| Memory Efficiency | Verify efficient memory usage patterns |
| Power Consumption | Verify power-efficient operation |

### 26.10.3	Error Handling Validation

Error Handling Validation

| Validation Area | Requirements |
| --- | --- |
| Error Injection | Verify error detection and reporting |
| Overflow Handling | Verify behavior when memories fill |
| Configuration Errors | Verify invalid configuration detection |
| Recovery Mechanisms | Verify error recovery procedures |

## 26.11	Usage Examples

### 26.11.1	Creating Monitor Packets

Creating Monitor Packets

| Packet Type | Example Usage |
| --- | --- |
| AXI Error | Protocol=AXI, Type=Error, Code=AXI_ERR_RESP_SLVERR |
| MNOC Credit | Protocol=MNOC, Type=Credit, Code=MNOC_CREDIT_EXHAUSTED |
| APB Performance | Protocol=APB, Type=Performance, Code=APB_PERF_TOTAL_LATENCY |
| ARB Threshold | Protocol=ARB, Type=Threshold, Code=ARB_THRESH_FAIRNESS_DEV |
| CORE Completion | Protocol=CORE, Type=Completion, Code=CORE_COMPL_DESCRIPTOR_LOADED |

### 26.11.2	Packet Decoding

Packet Decoding

| Decoding Step | Method |
| --- | --- |
| Extract Type | packet[63:60] |
| Extract Protocol | packet[59:57] |
| Extract Event Code | packet[56:53] |
| Extract Channel ID | packet[52:47] |
| Extract Event Data | packet[34:0] |

### 26.11.3	Monitor Bus Packet Helper Functions

#### 26.11.3.1	Packet Field Extraction

Packet Field Extraction

| Function | Return Type | Description |
| --- | --- | --- |
| get_packet_type(pkt) | logic [3:0] | Extract packet type [63:60] |
| get_protocol_type(pkt) | protocol_type_t | Extract protocol [59:57] |
| get_event_code(pkt) | logic [3:0] | Extract event code [56:53] |
| get_channel_id(pkt) | logic [5:0] | Extract channel ID [52:47] |
| get_unit_id(pkt) | logic [3:0] | Extract unit ID [46:43] |
| get_agent_id(pkt) | logic [7:0] | Extract agent ID [42:35] |
| get_event_data(pkt) | logic [34:0] | Extract event data [34:0] |

#### 26.11.3.2	Packet Creation Function

Packet Creation Function

| Function | Parameters | Description |
| --- | --- | --- |
| create_monitor_packet() | packet_type, protocol, event_code, channel_id, unit_id, agent_id, event_data | Create complete 64-bit packet |

#### 26.11.3.3	Event Code Creation Functions

Event Code Creation Functions

| Function | Parameter | Description |
| --- | --- | --- |
| create_axi_error_event() | axi_error_code_t | Create AXI error event code |
| create_axi_timeout_event() | axi_timeout_code_t | Create AXI timeout event code |
| create_axi_completion_event() | axi_completion_code_t | Create AXI completion event code |
| create_axi_threshold_event() | axi_threshold_code_t | Create AXI threshold event code |
| create_axi_performance_event() | axi_performance_code_t | Create AXI performance event code |
| create_axi_addr_match_event() | axi_addr_match_code_t | Create AXI address match event code |
| create_axi_debug_event() | axi_debug_code_t | Create AXI debug event code |
| create_apb_error_event() | apb_error_code_t | Create APB error event code |
| create_apb_timeout_event() | apb_timeout_code_t | Create APB timeout event code |
| create_apb_completion_event() | apb_completion_code_t | Create APB completion event code |
| create_mnoc_error_event() | mnoc_error_code_t | Create MNOC error event code |
| create_mnoc_timeout_event() | mnoc_timeout_code_t | Create MNOC timeout event code |
| create_mnoc_completion_event() | mnoc_completion_code_t | Create MNOC completion event code |
| create_mnoc_credit_event() | mnoc_credit_code_t | Create MNOC credit event code |
| create_mnoc_channel_event() | mnoc_channel_code_t | Create MNOC channel event code |
| create_mnoc_stream_event() | mnoc_stream_code_t | Create MNOC stream event code |
| create_arb_error_event() | arb_error_code_t | Create ARB error event code |
| create_arb_timeout_event() | arb_timeout_code_t | Create ARB timeout event code |
| create_arb_completion_event() | arb_completion_code_t | Create ARB completion event code |
| create_arb_threshold_event() | arb_threshold_code_t | Create ARB threshold event code |
| create_arb_performance_event() | arb_performance_code_t | Create ARB performance event code |
| create_arb_debug_event() | arb_debug_code_t | Create ARB debug event code |
| create_core_error_event() | core_error_code_t | Create CORE error event code |
| create_core_timeout_event() | core_timeout_code_t | Create CORE timeout event code |
| create_core_completion_event() | core_completion_code_t | Create CORE completion event code |
| create_core_threshold_event() | core_threshold_code_t | Create CORE threshold event code |
| create_core_performance_event() | core_performance_code_t | Create CORE performance event code |
| create_core_debug_event() | core_debug_code_t | Create CORE debug event code |

#### 26.11.3.4	Validation Functions

Validation Functions

| Function | Parameters | Description |
| --- | --- | --- |
| is_valid_event_for_packet_type() | packet_type, protocol, event_code | Validate event code for packet type and protocol |

#### 26.11.3.5	String Functions for Debugging

String Functions for Debugging

| Function | Parameter | Description |
| --- | --- | --- |
| get_axi_error_name() | axi_error_code_t | Get human-readable AXI error name |
| get_arb_error_name() | arb_error_code_t | Get human-readable ARB error name |
| get_core_error_name() | core_error_code_t | Get human-readable CORE error name |
| get_packet_type_name() | logic [3:0] | Get packet type name string |
| get_protocol_name() | protocol_type_t | Get protocol name string |
| get_event_name() | packet_type, protocol, event_code | Get comprehensive event name |

## 26.12	Debug and Monitoring Signals

### 26.12.1	Essential Debug Signals

Essential Debug Signals

| Signal | Width | Purpose |
| --- | --- | --- |
| debug_packet_counts | 32 × 16 | Packet count per type |
| debug_protocol_counts | 32 × 5 | Packet count per protocol |
| debug_error_counts | 32 | Total error packet count |
| debug_local_memory_level | 16 | Local memory usage level |
| debug_external_memory_level | 16 | External memory usage level |

### 26.12.2	Performance Counters

Performance Counters

| Counter | Width | Purpose |
| --- | --- | --- |
| total_packets_processed | 32 | Total packets processed |
| packets_dropped | 32 | Packets dropped due to overflow |
| routing_errors | 32 | Routing configuration errors |
| memory_full_events | 32 | Memory full occurrences |

## 26.13	Protocol Coverage Summary

### 26.13.1	Complete Protocol Event Matrix

Complete Protocol Event Matrix

| Protocol | Error | Timeout | Completion | Threshold | Performance | Debug | Protocol-Specific |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AXI | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | AddrMatch YES 16 |
| MNOC | YES 16 | YES 16 | YES 16 | NO 0 | NO 0 | NO 0 | Credit/Channel/Stream YES 48 |
| APB | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | None |
| ARB | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | None |
| CORE | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | YES 16 | None |

## 26.14	Total Event Codes: 544 defined across all protocols and packet types.

## 26.15	Revision History

Monitor Bus Architecture and Event Code Organization Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.90 | 2025-11-22 | seang | Initial block specification |
| 0.91 | 2026-01-02 | seang | Added table captions and figure numbers |

Last Updated: 2026-01-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	STREAM Register Map

## 27.1	Overview

The STREAM DMA engine register interface consists of two distinct regions:

- Channel Kick-off Registers (0x000 - 0x03F) - Direct routing to descriptor engines

- Configuration and Status Registers (0x100 - 0xFFF) - PeakRDL-generated register file

## 27.2	Address Space Layout

Base Address (configurable parameter)
│
├─ 0x000 - 0x03F: Channel Kick-off (apbtodescr.sv routing)
│  ├─ 0x000: CH0_CTRL_LOW  - Channel 0 descriptor address [31:0]
│  ├─ 0x004: CH0_CTRL_HIGH - Channel 0 descriptor address [63:32]
│  ├─ 0x008: CH1_CTRL_LOW  - Channel 1 descriptor address [31:0]
│  ├─ 0x00C: CH1_CTRL_HIGH - Channel 1 descriptor address [63:32]
│  ├─ 0x010: CH2_CTRL_LOW  - Channel 2 descriptor address [31:0]
│  ├─ 0x014: CH2_CTRL_HIGH - Channel 2 descriptor address [63:32]
│  ├─ 0x018: CH3_CTRL_LOW  - Channel 3 descriptor address [31:0]
│  ├─ 0x01C: CH3_CTRL_HIGH - Channel 3 descriptor address [63:32]
│  ├─ 0x020: CH4_CTRL_LOW  - Channel 4 descriptor address [31:0]
│  ├─ 0x024: CH4_CTRL_HIGH - Channel 4 descriptor address [63:32]
│  ├─ 0x028: CH5_CTRL_LOW  - Channel 5 descriptor address [31:0]
│  ├─ 0x02C: CH5_CTRL_HIGH - Channel 5 descriptor address [63:32]
│  ├─ 0x030: CH6_CTRL_LOW  - Channel 6 descriptor address [31:0]
│  ├─ 0x034: CH6_CTRL_HIGH - Channel 6 descriptor address [63:32]
│  ├─ 0x038: CH7_CTRL_LOW  - Channel 7 descriptor address [31:0]
│  └─ 0x03C: CH7_CTRL_HIGH - Channel 7 descriptor address [63:32]
│
├─ 0x040 - 0x0FF: Reserved
│
└─ 0x100 - 0x3FF: Configuration and Status Registers
   ├─ 0x100 - 0x11F: Global Control and Status
   ├─ 0x120 - 0x13F: Per-Channel Control
   ├─ 0x140 - 0x16F: Per-Channel Status
   ├─ 0x170 - 0x17F: Engine Completion and Error Status
   ├─ 0x180 - 0x1FF: Monitor FIFO Status
   ├─ 0x200 - 0x21F: Scheduler Configuration
   ├─ 0x220 - 0x23F: Descriptor Engine Configuration
   ├─ 0x240 - 0x25F: Descriptor AXI Monitor Configuration
   ├─ 0x260 - 0x27F: Read Engine AXI Monitor Configuration
   ├─ 0x280 - 0x29F: Write Engine AXI Monitor Configuration
   ├─ 0x2A0 - 0x2AF: AXI Transfer Configuration
   └─ 0x2B0 - 0x2BF: Performance Profiler Configuration

## 27.3	Register Details

### 27.3.1	Channel Kick-off Registers (0x000 - 0x03F)

These registers are NOT traditional registers. Writes are routed directly to descriptor engine APB ports via apbtodescr.sv.

Note: Descriptor addresses are 64-bit (ADDR_WIDTH parameter, default 64). On 32-bit APB bus, each channel requires TWO registers (LOW/HIGH).

Channel Kick-off Registers

| Offset | Register | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | CH0_CTRL_LOW | WO | N/A | Channel 0 descriptor address [31:0] |
| 0x004 | CH0_CTRL_HIGH | WO | N/A | Channel 0 descriptor address [63:32] |
| 0x008 | CH1_CTRL_LOW | WO | N/A | Channel 1 descriptor address [31:0] |
| 0x00C | CH1_CTRL_HIGH | WO | N/A | Channel 1 descriptor address [63:32] |
| 0x010 | CH2_CTRL_LOW | WO | N/A | Channel 2 descriptor address [31:0] |
| 0x014 | CH2_CTRL_HIGH | WO | N/A | Channel 2 descriptor address [63:32] |
| 0x018 | CH3_CTRL_LOW | WO | N/A | Channel 3 descriptor address [31:0] |
| 0x01C | CH3_CTRL_HIGH | WO | N/A | Channel 3 descriptor address [63:32] |
| 0x020 | CH4_CTRL_LOW | WO | N/A | Channel 4 descriptor address [31:0] |
| 0x024 | CH4_CTRL_HIGH | WO | N/A | Channel 4 descriptor address [63:32] |
| 0x028 | CH5_CTRL_LOW | WO | N/A | Channel 5 descriptor address [31:0] |
| 0x02C | CH5_CTRL_HIGH | WO | N/A | Channel 5 descriptor address [63:32] |
| 0x030 | CH6_CTRL_LOW | WO | N/A | Channel 6 descriptor address [31:0] |
| 0x034 | CH6_CTRL_HIGH | WO | N/A | Channel 6 descriptor address [63:32] |
| 0x038 | CH7_CTRL_LOW | WO | N/A | Channel 7 descriptor address [31:0] |
| 0x03C | CH7_CTRL_HIGH | WO | N/A | Channel 7 descriptor address [63:32] |

Write Behavior: - Descriptor address is 64-bit, split across LOW and HIGH registers - Write to HIGH register triggers descriptor engine kick-off - LOW register write is buffered, HIGH register write initiates transfer - Both registers must be written in order (LOW then HIGH) - Write blocks until descriptor engine accepts (back-pressure) - Read not supported (returns error)

Example:

// Start DMA transfer on channel 0
// Descriptor at physical address 0x0000_0001_8000_0000 (64-bit)

// Write lower 32 bits first (buffered)
write32(BASE + CH0_CTRL_LOW, 0x8000_0000);

// Write upper 32 bits second (triggers kick-off)
write32(BASE + CH0_CTRL_HIGH, 0x0000_0001);  // Blocks until accepted

// For descriptors in lower 4GB (typical case):
write32(BASE + CH0_CTRL_LOW, 0x8000_0000);
write32(BASE + CH0_CTRL_HIGH, 0x0000_0000);  // Upper 32 bits = 0

### 27.3.2	Global Control and Status (0x100 - 0x11F)

#### 27.3.2.1	GLOBAL_CTRL (0x100)

Master control register for entire STREAM engine.

GLOBAL_CTRL

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | Reserved | RO | 0x0 | Reserved |
| 1 | GLOBAL_RST | RW | 0 | Global reset (self-clearing) |
| 0 | GLOBAL_EN | RW | 0 | Global enable (1=enabled, 0=disabled) |

Usage:

// Enable STREAM engine
write32(BASE + GLOBAL_CTRL, 0x1);

// Reset all channels
write32(BASE + GLOBAL_CTRL, 0x3);  // Set both EN and RST
// RST self-clears after one cycle

#### 27.3.2.2	GLOBAL_STATUS (0x104)

Overall system status.

GLOBAL_STATUS

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | RO | Reserved |
| 0 | SYSTEM_IDLE | RO | System idle (all channels idle) |

#### 27.3.2.3	VERSION (0x108)

Version and configuration information (read-only).

VERSION

| Bits | Field | Type | Value | Description |
| --- | --- | --- | --- | --- |
| 31:24 | Reserved | RO | 0x00 | Reserved |
| 23:16 | NUM_CHANNELS | RO | 0x08 | Number of channels (8) |
| 15:8 | MAJOR | RO | 0x00 | Major version (0) |
| 7:0 | MINOR | RO | 0x5A | Minor version (90 decimal = 0.90) |

### 27.3.3	Per-Channel Control and Status (0x120 - 0x17F)

#### 27.3.3.1	CHANNEL_ENABLE (0x120)

Per-channel enable control (bit vector).

CHANNEL_ENABLE

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:8 | Reserved | RO | 0x0 | Reserved |
| 7:0 | CH_EN | RW | 0x00 | Channel enable [7:0] (1=enabled) |

Usage:

// Enable channels 0, 1, 2
write32(BASE + CHANNEL_ENABLE, 0x07);

// Disable channel 1, keep others
uint32_t val = read32(BASE + CHANNEL_ENABLE);
val &= ~(1 << 1);  // Clear bit 1
write32(BASE + CHANNEL_ENABLE, val);

#### 27.3.3.2	CHANNEL_RESET (0x124)

Per-channel reset control (bit vector, self-clearing).

CHANNEL_RESET

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:8 | Reserved | RO | 0x0 | Reserved |
| 7:0 | CH_RST | RW | 0x00 | Channel reset [7:0] (write 1 to reset) |

Usage:

// Reset channels 0 and 3
write32(BASE + CHANNEL_RESET, 0x09);  // Bits 0 and 3
// Self-clears after reset completes

#### 27.3.3.3	CHANNEL_IDLE (0x140)

Per-channel idle status (bit vector, read-only).

CHANNEL_IDLE

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved | RO | Reserved |
| 7:0 | CH_IDLE | RO | Channel idle [7:0] (1=idle, 0=active) |

#### 27.3.3.4	DESC_ENGINE_IDLE (0x144)

Per-channel descriptor engine idle status.

DESC_ENGINE_IDLE

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved | RO | Reserved |
| 7:0 | DESC_IDLE | RO | Descriptor engine idle [7:0] (1=idle, 0=active) |

#### 27.3.3.5	SCHEDULER_IDLE (0x148)

Per-channel scheduler idle status.

SCHEDULER_IDLE

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved | RO | Reserved |
| 7:0 | SCHED_IDLE | RO | Scheduler idle [7:0] (1=idle, 0=active) |

#### 27.3.3.6	CH_STATE[0..7] (0x150 - 0x16C)

Per-channel scheduler FSM state (8 registers, stride 0x4).

CH_STATE[0..7]

| Offset | Register | Bits | Field | Type | Description |
| --- | --- | --- | --- | --- | --- |
| 0x150 | CH0_STATE | 31:7 | Reserved | RO | Reserved |
|  |  | 6:0 | STATE | RO | Channel 0 scheduler state (one-hot) |
| 0x154 | CH1_STATE | 6:0 | STATE | RO | Channel 1 scheduler state (one-hot) |
| 0x158 | CH2_STATE | 6:0 | STATE | RO | Channel 2 scheduler state (one-hot) |
| 0x15C | CH3_STATE | 6:0 | STATE | RO | Channel 3 scheduler state (one-hot) |
| 0x160 | CH4_STATE | 6:0 | STATE | RO | Channel 4 scheduler state (one-hot) |
| 0x164 | CH5_STATE | 6:0 | STATE | RO | Channel 5 scheduler state (one-hot) |
| 0x168 | CH6_STATE | 6:0 | STATE | RO | Channel 6 scheduler state (one-hot) |
| 0x16C | CH7_STATE | 6:0 | STATE | RO | Channel 7 scheduler state (one-hot) |

State Encoding (One-Hot):

Bit 0 (0x01) = CH_IDLE         - Channel idle, waiting for descriptor
Bit 1 (0x02) = CH_FETCH_DESC   - Fetching descriptor from memory
Bit 2 (0x04) = CH_XFER_DATA    - Concurrent read AND write transfer
Bit 3 (0x08) = CH_COMPLETE     - Transfer complete
Bit 4 (0x10) = CH_NEXT_DESC    - Fetching next chained descriptor
Bit 5 (0x20) = CH_ERROR        - Error state
Bit 6 (0x40) = CH_RESERVED     - Reserved for future use

Note: Only ONE bit should be set at a time (one-hot encoding). Multiple bits set indicates a logic error.

### 27.3.4	Engine Completion and Error Status (0x170 - 0x17F)

#### 27.3.4.1	SCHED_ERROR (0x170)

Per-channel scheduler error flags.

SCHED_ERROR

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved | RO | Reserved |
| 7:0 | SCHED_ERR | RO | Scheduler error bits [7:0] (1=error, 0=no error) |

#### 27.3.4.2	AXI_RD_COMPLETE (0x174)

Per-channel AXI read engine completion status.

AXI_RD_COMPLETE

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved | RO | Reserved |
| 7:0 | RD_COMPLETE | RO | Read completion bits [7:0] (1=complete, 0=pending) |

#### 27.3.4.3	AXI_WR_COMPLETE (0x178)

Per-channel AXI write engine completion status.

AXI_WR_COMPLETE

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved | RO | Reserved |
| 7:0 | WR_COMPLETE | RO | Write completion bits [7:0] (1=complete, 0=pending) |

### 27.3.5	Monitor FIFO Status (0x180 - 0x1FF)

These registers are active when USE_AXI_MONITORS=1.

#### 27.3.5.1	MON_FIFO_STATUS (0x180)

Monitor bus FIFO status indicators.

MON_FIFO_STATUS

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:4 | Reserved | RO | Reserved |
| 3 | MON_FIFO_UNFL | RO | FIFO underflow detected (1=error) |
| 2 | MON_FIFO_OVFL | RO | FIFO overflow detected (1=error) |
| 1 | MON_FIFO_EMPTY | RO | FIFO empty (1=empty, 0=data available) |
| 0 | MON_FIFO_FULL | RO | FIFO full (1=full, 0=space available) |

#### 27.3.5.2	MON_FIFO_COUNT (0x184)

Monitor bus FIFO entry count.

MON_FIFO_COUNT

| Bits | Field | Type | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | RO | Reserved |
| 15:0 | FIFO_COUNT | RO | Number of entries in FIFO [15:0] |

### 27.3.6	Scheduler Configuration (0x200 - 0x21F)

#### 27.3.6.1	SCHED_TIMEOUT_CYCLES (0x200)

Timeout threshold for scheduler (global for all channels).

SCHED_TIMEOUT_CYCLES

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:0 | TIMEOUT_CYCLES | RW | 1000 | Timeout in clock cycles |

#### 27.3.6.2	SCHED_CONFIG (0x204)

Scheduler feature enables (global for all channels).

SCHED_CONFIG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | Reserved | RO | 0x0 | Reserved |
| 4 | PERF_EN | RW | 0 | Performance monitoring enable |
| 3 | COMPL_EN | RW | 1 | Completion reporting enable |
| 2 | ERR_EN | RW | 1 | Error reporting enable |
| 1 | TIMEOUT_EN | RW | 1 | Timeout detection enable |
| 0 | SCHED_EN | RW | 1 | Scheduler enable |

### 27.3.7	Descriptor Engine Configuration (0x220 - 0x23F)

#### 27.3.7.1	DESCENG_CONFIG (0x220)

Descriptor engine feature enables (global for all channels).

DESCENG_CONFIG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:6 | Reserved | RO | 0x0 | Reserved |
| 5:2 | FIFO_THRESH | RW | 0x8 | Prefetch FIFO threshold (4 bits) |
| 1 | PREFETCH_EN | RW | 0 | Prefetch enable |
| 0 | DESCENG_EN | RW | 1 | Descriptor engine enable |

#### 27.3.7.2	DESCENG_ADDR0_BASE (0x224)

Base address for descriptor address range 0 (lower 32 bits).

DESCENG_ADDR0_BASE

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | ADDR0_BASE | RW | 0x00000000 | Address range 0 base |

#### 27.3.7.3	DESCENG_ADDR0_LIMIT (0x228)

Limit address for descriptor address range 0 (lower 32 bits).

DESCENG_ADDR0_LIMIT

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | ADDR0_LIMIT | RW | 0xFFFFFFFF | Address range 0 limit |

#### 27.3.7.4	DESCENG_ADDR1_BASE (0x22C)

Base address for descriptor address range 1 (lower 32 bits).

DESCENG_ADDR1_BASE

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | ADDR1_BASE | RW | 0x00000000 | Address range 1 base |

#### 27.3.7.5	DESCENG_ADDR1_LIMIT (0x230)

Limit address for descriptor address range 1 (lower 32 bits).

DESCENG_ADDR1_LIMIT

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | ADDR1_LIMIT | RW | 0xFFFFFFFF | Address range 1 limit |

### 27.3.8	Descriptor AXI Monitor Configuration (0x240 - 0x25F)

#### 27.3.8.1	DAXMON_ENABLE (0x240)

Descriptor AXI master monitor enable controls.

DAXMON_ENABLE

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | Reserved | RO | 0x0 | Reserved |
| 4 | PERF_EN | RW | 0 | Performance packet enable |
| 3 | TIMEOUT_EN | RW | 1 | Timeout detection enable |
| 2 | COMPL_EN | RW | 0 | Completion packet enable |
| 1 | ERR_EN | RW | 1 | Error detection enable |
| 0 | MON_EN | RW | 1 | Master enable for descriptor monitor |

#### 27.3.8.2	DAXMON_TIMEOUT (0x244)

Descriptor AXI monitor timeout threshold.

DAXMON_TIMEOUT

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TIMEOUT_CYCLES | RW | 10000 | Timeout in clock cycles |

#### 27.3.8.3	DAXMON_LATENCY_THRESH (0x248)

Descriptor AXI monitor latency threshold.

DAXMON_LATENCY_THRESH

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | LATENCY_THRESH | RW | 5000 | Latency threshold in clock cycles |

#### 27.3.8.4	DAXMON_PKT_MASK (0x24C)

Descriptor AXI monitor packet type filtering.

DAXMON_PKT_MASK

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:0 | PKT_MASK | RW | 0xFFFF | Packet type mask (1=enable, 0=disable) |

#### 27.3.8.5	DAXMON_ERR_CFG (0x250)

Descriptor AXI monitor error selection and filtering.

DAXMON_ERR_CFG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | ERR_MASK | RW | 0xFF | Error type filtering mask |
| 7:4 | Reserved | RO | 0x0 | Reserved |
| 3:0 | ERR_SELECT | RW | 0x0 | Error type selection |

#### 27.3.8.6	DAXMON_MASK1 (0x254)

Descriptor AXI monitor timeout and completion masks.

DAXMON_MASK1

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | COMPL_MASK | RW | 0x00 | Completion mask |
| 7:0 | TIMEOUT_MASK | RW | 0xFF | Timeout mask |

#### 27.3.8.7	DAXMON_MASK2 (0x258)

Descriptor AXI monitor threshold and performance masks.

DAXMON_MASK2

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | PERF_MASK | RW | 0x00 | Performance mask |
| 7:0 | THRESH_MASK | RW | 0xFF | Threshold mask |

#### 27.3.8.8	DAXMON_MASK3 (0x25C)

Descriptor AXI monitor address and debug masks.

DAXMON_MASK3

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | DEBUG_MASK | RW | 0x00 | Debug mask |
| 7:0 | ADDR_MASK | RW | 0xFF | Address mask |

### 27.3.9	Read Engine AXI Monitor Configuration (0x260 - 0x27F)

#### 27.3.9.1	RDMON_ENABLE (0x260)

Read engine AXI master monitor enable controls.

RDMON_ENABLE

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | Reserved | RO | 0x0 | Reserved |
| 4 | PERF_EN | RW | 0 | Performance packet enable |
| 3 | TIMEOUT_EN | RW | 1 | Timeout detection enable |
| 2 | COMPL_EN | RW | 0 | Completion packet enable |
| 1 | ERR_EN | RW | 1 | Error detection enable |
| 0 | MON_EN | RW | 1 | Master enable for read monitor |

#### 27.3.9.2	RDMON_TIMEOUT (0x264)

Read engine AXI monitor timeout threshold.

RDMON_TIMEOUT

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TIMEOUT_CYCLES | RW | 10000 | Timeout in clock cycles |

#### 27.3.9.3	RDMON_LATENCY_THRESH (0x268)

Read engine AXI monitor latency threshold.

RDMON_LATENCY_THRESH

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | LATENCY_THRESH | RW | 5000 | Latency threshold in clock cycles |

#### 27.3.9.4	RDMON_PKT_MASK (0x26C)

Read engine AXI monitor packet type filtering.

RDMON_PKT_MASK

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:0 | PKT_MASK | RW | 0xFFFF | Packet type mask (1=enable, 0=disable) |

#### 27.3.9.5	RDMON_ERR_CFG (0x270)

Read engine AXI monitor error selection and filtering.

RDMON_ERR_CFG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | ERR_MASK | RW | 0xFF | Error type filtering mask |
| 7:4 | Reserved | RO | 0x0 | Reserved |
| 3:0 | ERR_SELECT | RW | 0x0 | Error type selection |

#### 27.3.9.6	RDMON_MASK1 (0x274)

Read engine AXI monitor timeout and completion masks.

RDMON_MASK1

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | COMPL_MASK | RW | 0x00 | Completion mask |
| 7:0 | TIMEOUT_MASK | RW | 0xFF | Timeout mask |

#### 27.3.9.7	RDMON_MASK2 (0x278)

Read engine AXI monitor threshold and performance masks.

RDMON_MASK2

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | PERF_MASK | RW | 0x00 | Performance mask |
| 7:0 | THRESH_MASK | RW | 0xFF | Threshold mask |

#### 27.3.9.8	RDMON_MASK3 (0x27C)

Read engine AXI monitor address and debug masks.

RDMON_MASK3

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | DEBUG_MASK | RW | 0x00 | Debug mask |
| 7:0 | ADDR_MASK | RW | 0xFF | Address mask |

### 27.3.10	Write Engine AXI Monitor Configuration (0x280 - 0x29F)

#### 27.3.10.1	WRMON_ENABLE (0x280)

Write engine AXI master monitor enable controls.

WRMON_ENABLE

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | Reserved | RO | 0x0 | Reserved |
| 4 | PERF_EN | RW | 0 | Performance packet enable |
| 3 | TIMEOUT_EN | RW | 1 | Timeout detection enable |
| 2 | COMPL_EN | RW | 0 | Completion packet enable |
| 1 | ERR_EN | RW | 1 | Error detection enable |
| 0 | MON_EN | RW | 1 | Master enable for write monitor |

#### 27.3.10.2	WRMON_TIMEOUT (0x284)

Write engine AXI monitor timeout threshold.

WRMON_TIMEOUT

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TIMEOUT_CYCLES | RW | 10000 | Timeout in clock cycles |

#### 27.3.10.3	WRMON_LATENCY_THRESH (0x288)

Write engine AXI monitor latency threshold.

WRMON_LATENCY_THRESH

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | LATENCY_THRESH | RW | 5000 | Latency threshold in clock cycles |

#### 27.3.10.4	WRMON_PKT_MASK (0x28C)

Write engine AXI monitor packet type filtering.

WRMON_PKT_MASK

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:0 | PKT_MASK | RW | 0xFFFF | Packet type mask (1=enable, 0=disable) |

#### 27.3.10.5	WRMON_ERR_CFG (0x290)

Write engine AXI monitor error selection and filtering.

WRMON_ERR_CFG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | ERR_MASK | RW | 0xFF | Error type filtering mask |
| 7:4 | Reserved | RO | 0x0 | Reserved |
| 3:0 | ERR_SELECT | RW | 0x0 | Error type selection |

#### 27.3.10.6	WRMON_MASK1 (0x294)

Write engine AXI monitor timeout and completion masks.

WRMON_MASK1

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | COMPL_MASK | RW | 0x00 | Completion mask |
| 7:0 | TIMEOUT_MASK | RW | 0xFF | Timeout mask |

#### 27.3.10.7	WRMON_MASK2 (0x298)

Write engine AXI monitor threshold and performance masks.

WRMON_MASK2

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | PERF_MASK | RW | 0x00 | Performance mask |
| 7:0 | THRESH_MASK | RW | 0xFF | Threshold mask |

#### 27.3.10.8	WRMON_MASK3 (0x29C)

Write engine AXI monitor address and debug masks.

WRMON_MASK3

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | DEBUG_MASK | RW | 0x00 | Debug mask |
| 7:0 | ADDR_MASK | RW | 0xFF | Address mask |

### 27.3.11	AXI Transfer Configuration (0x2A0 - 0x2AF)

#### 27.3.11.1	AXI_XFER_CONFIG (0x2A0)

AXI read and write transfer burst sizes.

AXI_XFER_CONFIG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | Reserved | RO | 0x0 | Reserved |
| 15:8 | WR_XFER_BEATS | RW | 15 | AXI write transfer beats (AWLEN: 0-255 = 1-256) |
| 7:0 | RD_XFER_BEATS | RW | 15 | AXI read transfer beats (ARLEN: 0-255 = 1-256) |

Usage:

// Configure for 16-beat bursts (default)
write32(BASE + AXI_XFER_CONFIG, 0x0F0F);

// Configure for 64-beat bursts
write32(BASE + AXI_XFER_CONFIG, 0x3F3F);

// Configure for maximum 256-beat bursts
write32(BASE + AXI_XFER_CONFIG, 0xFFFF);

### 27.3.12	Performance Profiler Configuration (0x2B0 - 0x2BF)

#### 27.3.12.1	PERF_CONFIG (0x2B0)

Performance profiler enable and mode controls.

PERF_CONFIG

| Bits | Field | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | Reserved | RO | 0x0 | Reserved |
| 2 | PERF_CLEAR | RW | 0 | Clear counters (write 1, self-clearing) |
| 1 | PERF_MODE | RW | 0 | Mode: 0=count, 1=histogram |
| 0 | PERF_EN | RW | 0 | Performance profiler enable |

Usage:

// Enable performance profiler in count mode
write32(BASE + PERF_CONFIG, 0x01);

// Enable in histogram mode
write32(BASE + PERF_CONFIG, 0x03);

// Clear counters
write32(BASE + PERF_CONFIG, 0x05);  // EN + CLEAR
// PERF_CLEAR self-clears after one cycle

## 27.4	Typical Usage Flow

### 27.4.1	Initialization

// 1. Global enable
write32(BASE + GLOBAL_CTRL, 0x1);

// 2. Configure scheduler
write32(BASE + SCHED_TIMEOUT_CYCLES, 10000);
write32(BASE + SCHED_CONFIG, 0x1F);  // All features enabled

// 3. Configure descriptor engine
write32(BASE + DESCENG_CONFIG, 0x01);  // Enable, no prefetch
write32(BASE + DESCENG_ADDR0_BASE, 0x8000_0000);
write32(BASE + DESCENG_ADDR0_LIMIT, 0x8FFF_FFFF);

// 4. Configure monitors (minimal reporting)
write32(BASE + DAXMON_CONFIG, 0x05);  // Error + timeout only
write32(BASE + RDMON_CONFIG, 0x05);
write32(BASE + WRMON_CONFIG, 0x05);

// 5. Enable desired channels
write32(BASE + CHANNEL_ENABLE, 0xFF);  // All 8 channels

### 27.4.2	Start Transfer

// Write 64-bit descriptor address to channel kick-off registers
// Descriptor at address 0x0000_0000_8000_0100 (64-bit)
write32(BASE + CH0_CTRL_LOW, 0x8000_0100);   // Lower 32 bits (buffered)
write32(BASE + CH0_CTRL_HIGH, 0x0000_0000);  // Upper 32 bits (triggers kick-off)
// Transfer starts immediately (blocks until descriptor engine ready)

### 27.4.3	Poll for Completion

// Check channel 0 idle status
while (!(read32(BASE + CHANNEL_IDLE) & 0x01)) {
    // Wait for channel 0 to become idle
}

// Or check scheduler state (one-hot encoding)
while ((read32(BASE + CH0_STATE) & 0x7F) != 0x01) {
    // Wait for CH_IDLE state (bit 0 = 0x01)
}

### 27.4.4	Error Handling

// Check all channel states for errors (one-hot encoding)
for (int ch = 0; ch < 8; ch++) {
    uint32_t state = read32(BASE + CH0_STATE + (ch * 4)) & 0x7F;
    if (state & 0x20) {  // CH_ERROR (bit 5)
        // Reset channel
        write32(BASE + CHANNEL_RESET, 1 << ch);
    }
}

## 27.5	Register Summary Table

Register Summary Table

| Offset Range | Description | Count | Type |
| --- | --- | --- | --- |
| 0x000-0x03F | Channel kick-off registers (LOW/HIGH) | 16 | Write-routing |
| 0x100-0x11F | Global control and status | 3 | RW/RO |
| 0x120-0x13F | Per-channel control | 2 | RW |
| 0x140-0x16F | Per-channel status | 11 | RO |
| 0x170-0x17F | Engine completion and error status | 3 | RO |
| 0x180-0x1FF | Monitor FIFO status | 2 | RO |
| 0x200-0x21F | Scheduler configuration | 2 | RW |
| 0x220-0x23F | Descriptor engine configuration | 5 | RW |
| 0x240-0x25F | Descriptor AXI monitor configuration | 8 | RW |
| 0x260-0x27F | Read engine AXI monitor configuration | 8 | RW |
| 0x280-0x29F | Write engine AXI monitor configuration | 8 | RW |
| 0x2A0-0x2AF | AXI transfer configuration | 1 | RW |
| 0x2B0-0x2BF | Performance profiler configuration | 1 | RW |

Total: 70 registers (16 kick-off + 54 config/status)

## 27.6	PeakRDL Generation

To generate SystemVerilog from the register definition:

cd projects/components/stream/rtl/stream_macro/
peakrdl regblock stream_regs.rdl -o generated/

This generates: - stream_regs_pkg.sv - Register definitions package - stream_regs.sv - APB slave register interface

Revision History:

PeakRDL Generation

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2025-10-20 | sean galloway | Initial creation |
| 1.1 | 2025-12-01 | sean galloway | Added complete monitor registers from RDL |
|  |  |  | Added engine completion/error status (0x170) |
|  |  |  | Added monitor FIFO status (0x180) |
|  |  |  | Added AXI transfer config (0x2A0) |
|  |  |  | Added performance profiler config (0x2B0) |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	Chapter 5: Programming Models

This chapter provides software developer guidance for using the STREAM DMA engine.

## 28.1	Contents

### 28.1.1	01_initialization.md

- Power-on initialization sequence

- Register configuration

- Channel setup

- Configuration presets (minimal, high-performance)

### 28.1.2	02_single_transfer.md

- Descriptor format (256-bit)

- Kick-off address map and write sequence

- Simple single-descriptor transfer

- C code examples with complete workflow

### 28.1.3	03_chained_transfers.md

- Multi-descriptor chains

- Descriptor linking via next_descriptor_ptr

- Chain termination methods

- Scatter-gather operations

- Prefetch mode for performance

### 28.1.4	04_multi_channel.md

- 8-channel concurrent operations

- Priority-based scheduling

- Resource sharing strategies

- Channel pooling and load balancing

- Performance considerations

### 28.1.5	05_error_handling.md

- Error types (AXI, timeout, internal)

- Error detection registers

- Recovery procedures

- Interrupt-based error handling

- Debug monitoring

## 28.2	Planned (Future)

### 28.2.1	06_performance_tuning.md

- Burst size selection

- Priority tuning

- SRAM depth considerations

- Maximizing throughput

### 28.2.2	07_software_examples.md

- Complete working examples

- Linux driver skeleton

- Bare-metal usage

- Common use cases

Status: Core programming guides complete (01-05)

Target Audience: - Software engineers integrating STREAM - Driver developers - System architects - Application developers

Last Updated: 2025-12-01

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Chapter 6: Configuration Reference

Version: 0.90 Last Updated: 2025-11-22 Purpose: Complete reference for all STREAM configuration signals

## 29.1	Overview

STREAM provides comprehensive runtime configuration through APB registers and compile-time parameters. This chapter documents all configuration signals, their valid ranges, default values, and recommended settings for different use cases.

### 29.1.1	Configuration Categories

Configuration Categories

| Category | Signals | Purpose |
| --- | --- | --- |
| Channel Control | 2 | Enable/reset individual channels |
| Scheduler | 6 | Transfer scheduling and timeouts |
| Descriptor Engine | 7 | Descriptor fetch behavior |
| AXI Monitors | 45 (15 per monitor × 3) | Debug/trace filtering |
| AXI Transfer | 2 | Burst configuration |
| Performance | 3 | Profiling and metrics |
| Total | 65 configuration signals | Full system control |

## 29.2	1. Channel Control Configuration

### 29.2.1	cfg_channel_enable[NUM_CHANNELS-1:0]

Type: Per-channel enable Width: 1 bit × NUM_CHANNELS (default 8) Default: 8’hFF (all enabled) Register: CHANNEL_ENABLE @ 0x120

Description: Enables or disables individual DMA channels. When disabled, the channel: - Ignores descriptor kick-off requests - Completes current transfer if in progress - Enters idle state - Remains in idle until re-enabled

Valid Values: - 1'b1: Channel enabled (can accept transfers) - 1'b0: Channel disabled (ignores new transfers)

Use Cases:

// Enable only channels 0, 2, 4 (even channels)
cfg_channel_enable = 8'b01010101;

// Disable all channels (emergency stop)
cfg_channel_enable = 8'b00000000;

// Enable all channels (normal operation)
cfg_channel_enable = 8'b11111111;

Interaction with Global Enable: The final channel enable is: cfg_channel_enable[i] & reg_global_ctrl_global_en

### 29.2.2	cfg_channel_reset[NUM_CHANNELS-1:0]

Type: Per-channel reset Width: 1 bit × NUM_CHANNELS (default 8) Default: 8’h00 (no resets active) Register: CHANNEL_RESET @ 0x124

Description: Asserts reset for individual channels without affecting other channels or global state. When asserted: - Channel FSM returns to IDLE - Pending transfers aborted - SRAM buffers flushed - Descriptor engine reset

Valid Values: - 1'b1: Channel in reset (clears state) - 1'b0: Channel operating normally

Use Cases:

// Reset channel 3 after error
cfg_channel_reset = 8'b00001000;
wait_cycles(10);
cfg_channel_reset = 8'b00000000;  // Release reset

// Reset all channels (soft reset)
cfg_channel_reset = 8'hFF;
wait_cycles(10);
cfg_channel_reset = 8'h00;

IMPORTANT: Assert reset for minimum 10 clock cycles to ensure complete FSM reset.

## 29.3	2. Scheduler Configuration

### 29.3.1	cfg_sched_timeout_cycles[15:0]

Type: Timeout threshold Width: 16 bits Default: 16’d1000 (1,000 cycles) Register: SCHED_TIMEOUT_CYCLES @ 0x200[15:0]

Description: Number of clock cycles before a channel operation times out. Applies to: - Descriptor fetch latency - AXI read/write response latency - Scheduler state transitions

Valid Range: 100 to 65535 cycles Typical Values: - Fast SRAM: 100-500 cycles - DDR3/DDR4: 1000-10000 cycles - High-latency: 10000-65535 cycles

Calculation:

Timeout cycles = (Expected latency × 10) + margin

Example for DDR4 @ 200 MHz:
- Expected read latency: 100 cycles (500 ns)
- Safety margin: 10x
- Timeout = 100 × 10 = 1000 cycles

### 29.3.2	cfg_sched_enable

Type: Global scheduler enable Width: 1 bit Default: 1’b1 (enabled) Register: SCHED_CONFIG @ 0x204[0]

Description: Master enable for all schedulers. When disabled, all channels stop scheduling new operations.

### 29.3.3	cfg_sched_timeout_enable

Type: Timeout detection enable Width: 1 bit Default: 1’b1 (enabled) Register: SCHED_CONFIG @ 0x204[1]

Description: Enables timeout detection for scheduler operations. Disable for simulation or known slow memory.

### 29.3.4	cfg_sched_err_enable

Type: Error detection enable Width: 1 bit Default: 1’b1 (enabled) Register: SCHED_CONFIG @ 0x204[2]

Description: Enables error packet generation for scheduler errors (AXI SLVERR, DECERR, timeouts).

### 29.3.5	cfg_sched_compl_enable

Type: Completion event enable Width: 1 bit Default: 1’b1 (enabled by default) Register: SCHED_CONFIG @ 0x204[3]

Description: Enables MonBus packets for transfer completion events. Generate high traffic, use sparingly.

### 29.3.6	cfg_sched_perf_enable

Type: Performance monitoring enable Width: 1 bit Default: 1’b0 (disabled by default) Register: SCHED_CONFIG @ 0x204[4]

Description: Enables performance profiling packets (latency, throughput metrics).

## 29.4	3. Descriptor Engine Configuration

### 29.4.1	cfg_desceng_enable

Type: Descriptor engine global enable Width: 1 bit Default: 1’b1 (enabled) Register: DESCENG_CTRL @ 0x220[0]

Description: Global enable for all descriptor engines. When disabled, descriptor fetch stops.

### 29.4.2	cfg_desceng_prefetch

Type: Prefetch enable Width: 1 bit Default: 1’b0 (disabled) Register: DESCENG_CTRL @ 0x220[1]

Description: Enables descriptor prefetching to hide fetch latency.

Prefetch Behavior: - Enabled: Fetch next descriptor while current transfer executes - Disabled: Wait for current transfer completion before fetching next

Performance Impact: - Prefetch ON: +15-30% throughput for chained descriptors - Prefetch OFF: Simpler logic, lower area

### 29.4.3	cfg_desceng_fifo_thresh[3:0]

Type: FIFO threshold Width: 4 bits Default: 4’h8 (8 entries) Register: DESCENG_CTRL @ 0x220[7:4]

Description: Number of entries in descriptor FIFO before asserting backpressure.

Valid Range: 1-15 entries Typical Values: - Low latency: 2-4 entries - Balanced: 8 entries (default) - High throughput: 12-15 entries

### 29.4.4	cfg_desceng_addr0_base[31:0]

Type: Base address for descriptor region 0 Width: 32 bits Default: 32’h0000_0000 Register: DESCENG_ADDR0_BASE @ 0x224

Description: Base address of first descriptor memory region. Descriptors fetched from this region if within range.

Alignment: Must be aligned to descriptor size (256 bits = 32 bytes)

### 29.4.5	cfg_desceng_addr0_limit[31:0]

Type: Limit address for descriptor region 0 Width: 32 bits Default: 32’hFFFF_FFFF (no limit) Register: DESCENG_ADDR0_LIMIT @ 0x228

Description: Upper limit of first descriptor region. Descriptors beyond this address use region 1.

### 29.4.6	cfg_desceng_addr1_base[31:0]

Type: Base address for descriptor region 1 Width: 32 bits Default: 32’h0000_0000 Register: DESCENG_ADDR1_BASE @ 0x22C

Description: Base address of second descriptor memory region (optional).

### 29.4.7	cfg_desceng_addr1_limit[31:0]

Type: Limit address for descriptor region 1 Width: 32 bits Default: 32’hFFFF_FFFF Register: DESCENG_ADDR1_LIMIT @ 0x230

Description: Upper limit of second descriptor region.

## 29.5	4. AXI Monitor Configuration

STREAM includes three independent AXI monitors with identical configuration sets:

- Descriptor AXI Monitor (cfg_desc_mon_*) - Monitors descriptor fetch AXI master

- Read Engine Monitor (cfg_rdeng_mon_*) - Monitors data read AXI master

- Write Engine Monitor (cfg_wreng_mon_*) - Monitors data write AXI master

Each monitor has 15 configuration signals with the same structure.

### 29.5.1	4.1 Descriptor AXI Monitor (cfg_desc_mon_*)

Register Base: 0x240-0x25F

#### 29.5.1.1	cfg_desc_mon_enable

Type: Monitor master enable Width: 1 bit Default: 1’b1 (enabled) Register: DAXMON_ENABLE @ 0x240[0]

Description: Master enable for descriptor AXI monitor. All monitor packets disabled when this is 0.

#### 29.5.1.2	cfg_desc_mon_err_enable

Type: Error packet enable Width: 1 bit Default: 1’b1 (enabled) Register: DAXMON_ENABLE @ 0x240[1]

Description: Enables error packet generation (SLVERR, DECERR, protocol violations).

#### 29.5.1.3	cfg_desc_mon_perf_enable

Type: Performance packet enable Width: 1 bit Default: 1’b0 (disabled) Register: DAXMON_ENABLE @ 0x240[2]

Description: Enables performance monitoring packets (latency, bandwidth).

WARNING: High packet rate - use only during debug.

#### 29.5.1.4	cfg_desc_mon_timeout_enable

Type: Timeout detection enable Width: 1 bit Default: 1’b1 (enabled) Register: DAXMON_ENABLE @ 0x240[3]

Description: Enables timeout packet generation when transactions exceed threshold.

#### 29.5.1.5	cfg_desc_mon_timeout_cycles[31:0]

Type: Timeout threshold Width: 32 bits Default: 32’d10000 Register: DAXMON_TIMEOUT @ 0x244

Description: Number of cycles before transaction times out.

#### 29.5.1.6	cfg_desc_mon_latency_thresh[31:0]

Type: Latency threshold Width: 32 bits Default: 32’d1000 Register: DAXMON_LATENCY @ 0x248

Description: Latency threshold for performance warnings.

#### 29.5.1.7	cfg_desc_mon_pkt_mask[15:0]

Type: Packet type filter Width: 16 bits (1 bit per packet type) Default: 16’h00FF (errors + completions) Register: DAXMON_PKT_MASK @ 0x24C[15:0]

Description: Bit mask to filter packet types. Only packet types with corresponding bit set are generated.

Packet Type Mapping:

Bit  | Packet Type
-----|-------------
0    | AR command
1    | AW command
2    | R completion
3    | W data
4    | B response
5    | Error
6    | Timeout
7    | Completion
8-15 | Reserved

Examples:

// Only errors
cfg_desc_mon_pkt_mask = 16'h0020;  // Bit 5

// Errors + timeouts
cfg_desc_mon_pkt_mask = 16'h0060;  // Bits 5-6

// All packets
cfg_desc_mon_pkt_mask = 16'hFFFF;

#### 29.5.1.8	cfg_desc_mon_err_select[3:0]

Type: Error type selector Width: 4 bits Default: 4’hF (all errors) Register: DAXMON_ERR_SELECT @ 0x24C[19:16]

Description: Selects which error types to monitor.

Error Type Bits:

Bit | Error Type
----|------------
0   | SLVERR
1   | DECERR
2   | Protocol violation
3   | Reserved

#### 29.5.1.9	cfg_desc_mon_err_mask[7:0]

Type: Error event filter Width: 8 bits Default: 8’hFF (all errors) Register: DAXMON_MASK1 @ 0x250[7:0]

Description: Bit mask for error packet generation (channel-specific filtering).

#### 29.5.1.10	cfg_desc_mon_timeout_mask[7:0]

Type: Timeout channel filter Width: 8 bits Default: 8’hFF (all channels) Register: DAXMON_MASK1 @ 0x250[15:8]

Description: Channel mask for timeout packets. Set bit enables timeout detection for corresponding channel.

#### 29.5.1.11	cfg_desc_mon_compl_mask[7:0]

Type: Completion channel filter Width: 8 bits Default: 8’h00 (no channels) Register: DAXMON_MASK1 @ 0x250[23:16]

Description: Channel mask for completion packets.

WARNING: Completion packets are high volume - enable only for specific channels.

#### 29.5.1.12	cfg_desc_mon_thresh_mask[7:0]

Type: Threshold event filter Width: 8 bits Default: 8’hFF (all channels) Register: DAXMON_MASK2 @ 0x254[7:0]

Description: Channel mask for latency threshold exceedance packets.

#### 29.5.1.13	cfg_desc_mon_perf_mask[7:0]

Type: Performance packet filter Width: 8 bits Default: 8’h00 (no channels) Register: DAXMON_MASK2 @ 0x254[15:8]

Description: Channel mask for performance monitoring packets.

#### 29.5.1.14	cfg_desc_mon_addr_mask[7:0]

Type: Address-based filter Width: 8 bits Default: 8’hFF (all addresses) Register: DAXMON_MASK2 @ 0x254[23:16]

Description: Channel mask for address-range-based packet filtering.

#### 29.5.1.15	cfg_desc_mon_debug_mask[7:0]

Type: Debug event filter Width: 8 bits Default: 8’h00 (no debug packets) Register: DAXMON_MASK2 @ 0x254[31:24]

Description: Channel mask for debug-level packets (verbose trace).

### 29.5.2	4.2 Read Engine Monitor (cfg_rdeng_mon_*)

Register Base: 0x260-0x27F

All signals identical to Descriptor Monitor (Section 4.1), but applied to data read AXI master.

Key Differences: - Monitors data transfers (not descriptors) - Higher throughput → more packets - Recommended: Keep cfg_rdeng_mon_compl_enable = 0 unless debugging

### 29.5.3	4.3 Write Engine Monitor (cfg_wreng_mon_*)

Register Base: 0x280-0x29F

All signals identical to Descriptor Monitor (Section 4.1), but applied to data write AXI master.

Key Differences: - Monitors write transactions (AW/W/B channels) - B response timing critical for performance - Recommended: Enable only error packets by default

## 29.6	5. AXI Transfer Configuration

### 29.6.1	cfg_axi_rd_xfer_beats[7:0]

Type: Read transfer size Width: 8 bits Default: 8’d16 (16 beats) Register: AXI_RD_XFER @ 0x2A0[7:0]

Description: Default number of beats per AXI read burst. Actual burst size may be less to respect 4KB boundaries.

Valid Range: 1-256 beats (AXI4 standard) Typical Values: - Small transfers: 4-8 beats - Balanced: 16 beats (default) - Large transfers: 32-64 beats - Maximum throughput: 128-256 beats

Calculation:

Transfer size (bytes) = beats × (DATA_WIDTH / 8)

Example for DATA_WIDTH = 512 bits:
- 16 beats = 16 × 64 = 1024 bytes (1 KB)
- 64 beats = 64 × 64 = 4096 bytes (4 KB)

### 29.6.2	cfg_axi_wr_xfer_beats[7:0]

Type: Write transfer size Width: 8 bits Default: 8’d16 (16 beats) Register: AXI_WR_XFER @ 0x2A0[15:8]

Description: Default number of beats per AXI write burst.

Same constraints as cfg_axi_rd_xfer_beats

## 29.7	6. Performance Profiler Configuration

### 29.7.1	cfg_perf_enable

Type: Profiler enable Width: 1 bit Default: 1’b0 (disabled) Register: PERF_CTRL @ 0x2B0[0]

Description: Enables performance profiling for all channels. When enabled, profiler captures: - Transfer start/end timestamps - Latency per channel - Throughput measurements - Channel utilization

### 29.7.2	cfg_perf_mode

Type: Profiling mode Width: 1 bit Default: 1’b0 (timestamp mode) Register: PERF_CTRL @ 0x2B0[1]

Description: Selects profiling mode: - 1'b0: Timestamp mode - Record absolute timestamps - 1'b1: Elapsed time mode - Record delta times

Use Cases: - Timestamp: Correlate events across multiple blocks - Elapsed: Measure operation latencies

### 29.7.3	cfg_perf_clear

Type: Clear profiler state Width: 1 bit (write-only) Default: 1’b0 Register: PERF_CTRL @ 0x2B0[2]

Description: Write 1'b1 to clear profiler FIFOs and counters. Self-clearing (automatically returns to 0).

## 29.8	7. Configuration Presets

### 29.8.1	7.1 Minimal Configuration (Tutorial/Embedded)

Use Case: Educational, minimal logic, single-channel operation

// Channel control
cfg_channel_enable = 8'b00000001;  // Only channel 0

// Scheduler
cfg_sched_enable = 1'b1;
cfg_sched_timeout_cycles = 16'd500;      // Short timeout (SRAM)
cfg_sched_timeout_enable = 1'b1;
cfg_sched_err_enable = 1'b1;
cfg_sched_compl_enable = 1'b0;           // Disable completion packets
cfg_sched_perf_enable = 1'b0;            // Disable performance packets

// Descriptor engine
cfg_desceng_enable = 1'b1;
cfg_desceng_prefetch = 1'b0;             // No prefetch (simpler)
cfg_desceng_fifo_thresh = 4'h4;          // Small FIFO

// All monitors DISABLED (reduce logic)
cfg_desc_mon_enable = 1'b0;
cfg_rdeng_mon_enable = 1'b0;
cfg_wreng_mon_enable = 1'b0;

// AXI transfer
cfg_axi_rd_xfer_beats = 8'd8;            // Small bursts
cfg_axi_wr_xfer_beats = 8'd8;

// Performance profiler
cfg_perf_enable = 1'b0;                  // Disabled

### 29.8.2	7.2 Balanced Configuration (Typical FPGA)

Use Case: General-purpose DMA, moderate channels, balanced performance/area

// Channel control
cfg_channel_enable = 8'hFF;              // All 8 channels

// Scheduler
cfg_sched_enable = 1'b1;
cfg_sched_timeout_cycles = 16'd5000;     // DDR4 timeout
cfg_sched_timeout_enable = 1'b1;
cfg_sched_err_enable = 1'b1;
cfg_sched_compl_enable = 1'b0;           // Errors only
cfg_sched_perf_enable = 1'b0;

// Descriptor engine
cfg_desceng_enable = 1'b1;
cfg_desceng_prefetch = 1'b1;             // Enable prefetch
cfg_desceng_fifo_thresh = 4'h8;          // Balanced FIFO

// Descriptor monitor (errors only)
cfg_desc_mon_enable = 1'b1;
cfg_desc_mon_err_enable = 1'b1;
cfg_desc_mon_perf_enable = 1'b0;
cfg_desc_mon_timeout_enable = 1'b1;
cfg_desc_mon_timeout_cycles = 32'd10000;
cfg_desc_mon_pkt_mask = 16'h0060;        // Errors + timeouts

// Read/write monitors (errors only)
cfg_rdeng_mon_enable = 1'b1;
cfg_rdeng_mon_err_enable = 1'b1;
cfg_rdeng_mon_perf_enable = 1'b0;

cfg_wreng_mon_enable = 1'b1;
cfg_wreng_mon_err_enable = 1'b1;
cfg_wreng_mon_perf_enable = 1'b0;

// AXI transfer
cfg_axi_rd_xfer_beats = 8'd32;           // Moderate bursts
cfg_axi_wr_xfer_beats = 8'd32;

// Performance profiler
cfg_perf_enable = 1'b1;                  // Enable profiling
cfg_perf_mode = 1'b1;                    // Elapsed time mode

### 29.8.3	7.3 High-Performance Configuration (ASIC/Datacenter)

Use Case: Maximum throughput, all channels active, full monitoring

// Channel control
cfg_channel_enable = 8'hFF;              // All channels

// Scheduler
cfg_sched_enable = 1'b1;
cfg_sched_timeout_cycles = 16'd20000;    // High latency tolerance
cfg_sched_timeout_enable = 1'b1;
cfg_sched_err_enable = 1'b1;
cfg_sched_compl_enable = 1'b1;           // Full monitoring
cfg_sched_perf_enable = 1'b1;

// Descriptor engine
cfg_desceng_enable = 1'b1;
cfg_desceng_prefetch = 1'b1;
cfg_desceng_fifo_thresh = 4'hF;          // Max FIFO depth

// All monitors ENABLED with full profiling
cfg_desc_mon_enable = 1'b1;
cfg_desc_mon_err_enable = 1'b1;
cfg_desc_mon_perf_enable = 1'b1;         // Performance monitoring
cfg_desc_mon_timeout_enable = 1'b1;
cfg_desc_mon_timeout_cycles = 32'd20000;
cfg_desc_mon_pkt_mask = 16'hFFFF;        // All packet types

// Same for read/write monitors
cfg_rdeng_mon_enable = 1'b1;
cfg_rdeng_mon_err_enable = 1'b1;
cfg_rdeng_mon_perf_enable = 1'b1;

cfg_wreng_mon_enable = 1'b1;
cfg_wreng_mon_err_enable = 1'b1;
cfg_wreng_mon_perf_enable = 1'b1;

// AXI transfer
cfg_axi_rd_xfer_beats = 8'd128;          // Large bursts
cfg_axi_wr_xfer_beats = 8'd128;

// Performance profiler
cfg_perf_enable = 1'b1;
cfg_perf_mode = 1'b0;                    // Timestamp mode (correlation)

### 29.8.4	7.4 Debug Configuration (Verbose Monitoring)

Use Case: Debugging integration issues, detailed trace analysis

// Enable specific channel for debug
cfg_channel_enable = 8'b00000001;        // Channel 0 only

// Scheduler
cfg_sched_enable = 1'b1;
cfg_sched_timeout_cycles = 16'd65535;    // Long timeout for debug
cfg_sched_timeout_enable = 1'b1;
cfg_sched_err_enable = 1'b1;
cfg_sched_compl_enable = 1'b1;           // All events
cfg_sched_perf_enable = 1'b1;

// Descriptor engine
cfg_desceng_enable = 1'b1;
cfg_desceng_prefetch = 1'b0;             // Simpler for debug

// All monitors ENABLED with verbose trace
cfg_desc_mon_enable = 1'b1;
cfg_desc_mon_err_enable = 1'b1;
cfg_desc_mon_perf_enable = 1'b1;
cfg_desc_mon_timeout_enable = 1'b1;
cfg_desc_mon_pkt_mask = 16'hFFFF;        // ALL packets
cfg_desc_mon_compl_mask = 8'h01;         // Channel 0 completions
cfg_desc_mon_debug_mask = 8'h01;         // Channel 0 debug packets

// Same for read/write monitors
cfg_rdeng_mon_enable = 1'b1;
cfg_rdeng_mon_pkt_mask = 16'hFFFF;       // Verbose
cfg_rdeng_mon_compl_mask = 8'h01;

cfg_wreng_mon_enable = 1'b1;
cfg_wreng_mon_pkt_mask = 16'hFFFF;
cfg_wreng_mon_compl_mask = 8'h01;

// AXI transfer (small for debug)
cfg_axi_rd_xfer_beats = 8'd4;
cfg_axi_wr_xfer_beats = 8'd4;

// Performance profiler
cfg_perf_enable = 1'b1;
cfg_perf_mode = 1'b0;                    // Timestamps

## 29.9	8. Configuration Best Practices

### 29.9.1	8.1 Monitor Configuration Guidelines

General Rules:

- Start with errors only: Enable only cfg_*_mon_err_enable initially. Add other packets as needed.

- Completion packets are expensive: Only enable cfg_*_mon_compl_enable for specific channels during debug.

- Performance packets flood MonBus: Enable cfg_*_mon_perf_enable sparingly (1-2 channels maximum).

- Use masks aggressively: Set channel masks to enable monitoring only on channels of interest.

### 29.9.2	8.2 Timeout Configuration

Calculation Method:

Recommended timeout = (Expected latency × Safety factor) + Margin

Safety factor:
- SRAM: 2-5x
- DDR3/DDR4: 5-10x
- High-latency: 10-20x

Margin: +100 cycles minimum

Examples:

SRAM @ 200 MHz:
- Expected: 20 cycles (100 ns)
- Safety: 5x
- Timeout: 20 × 5 + 100 = 200 cycles

DDR4 @ 200 MHz:
- Expected: 100 cycles (500 ns)
- Safety: 10x
- Timeout: 100 × 10 + 100 = 1100 cycles

### 29.9.3	8.3 Prefetch Configuration

Enable prefetch when: - Descriptor chains > 2 descriptors - Memory latency > 50 cycles - Throughput is priority

Disable prefetch when: - Area is constrained - Single descriptors only - Simplicity is priority

### 29.9.4	8.4 Burst Size Selection

Read Burst Size:

Optimal burst size = min(
    Memory controller page size,
    4KB (AXI limit),
    SRAM FIFO depth / 2
)

Example for DDR4 (8KB page), FIFO depth 512 entries:
- Page size: 8192 bytes = 128 beats (512-bit)
- AXI limit: 4096 bytes = 64 beats
- FIFO limit: 512/2 = 256 beats
- Optimal: min(128, 64, 256) = 64 beats

Write Burst Size: - Usually same as read burst size - May be smaller if write FIFO depth is limited

## 29.10	9. Configuration Register Map Summary

9. Configuration Register Map Summary

| Address | Register Name | Fields | Section |
| --- | --- | --- | --- |
| 0x100 | GLOBAL_CTRL | global_en, global_rst | - |
| 0x120 | CHANNEL_ENABLE | ch_en[7:0] | 1 |
| 0x124 | CHANNEL_RESET | ch_rst[7:0] | 1 |
| 0x200 | SCHED_TIMEOUT_CYCLES | timeout_cycles[15:0] | 2 |
| 0x204 | SCHED_CONFIG | enable, timeout_en, err_en, compl_en, perf_en | 2 |
| 0x220 | DESCENG_CONFIG | enable, prefetch, fifo_thresh | 3 |
| 0x224 | DESCENG_ADDR0_BASE | addr0_base[31:0] | 3 |
| 0x228 | DESCENG_ADDR0_LIMIT | addr0_limit[31:0] | 3 |
| 0x22C | DESCENG_ADDR1_BASE | addr1_base[31:0] | 3 |
| 0x230 | DESCENG_ADDR1_LIMIT | addr1_limit[31:0] | 3 |
| 0x240-0x25F | DAXMON_* | Descriptor monitor (15 signals) | 4.1 |
| 0x260-0x27F | RDMON_* | Read engine monitor (15 signals) | 4.2 |
| 0x280-0x29F | WRMON_* | Write engine monitor (15 signals) | 4.3 |
| 0x2A0 | AXI_XFER_CFG | rd_xfer_beats, wr_xfer_beats | 5 |
| 0x2B0 | PERF_CTRL | perf_enable, perf_mode, perf_clear | 6 |

Total Address Space: 0x000-0x3FF (1KB)

## 29.11	10. Software Configuration Examples

### 29.11.1	10.1 C/C++ Initialization (Minimal)

// Minimal configuration for single-channel operation
void stream_init_minimal(volatile uint32_t *base_addr) {
    // Global enable
    base_addr[0x100/4] = 0x00000001;  // GLOBAL_CTRL.global_en

    // Enable channel 0 only
    base_addr[0x120/4] = 0x00000001;  // CHANNEL_ENABLE.ch_en

    // Scheduler config
    base_addr[0x200/4] = 500;         // SCHED_TIMEOUT_CYCLES (SRAM)
    base_addr[0x204/4] = 0x00000007;  // SCHED_CONFIG: enable | timeout_en | err_en

    // Descriptor engine
    base_addr[0x220/4] = 0x00000041;  // enable | fifo_thresh=4

    // Disable all monitors (minimal)
    base_addr[0x240/4] = 0x00000000;  // DAXMON_ENABLE
    base_addr[0x260/4] = 0x00000000;  // RDMON_ENABLE
    base_addr[0x280/4] = 0x00000000;  // WRMON_ENABLE

    // AXI transfer config
    base_addr[0x2A0/4] = 0x00000808;  // 8 beats read + write
}

### 29.11.2	10.2 C/C++ Initialization (Balanced)

// Balanced configuration for typical FPGA
void stream_init_balanced(volatile uint32_t *base_addr) {
    // Global enable
    base_addr[0x100/4] = 0x00000001;

    // Enable all 8 channels
    base_addr[0x120/4] = 0x000000FF;

    // Scheduler config
    base_addr[0x200/4] = 5000;        // SCHED_TIMEOUT_CYCLES (DDR4)
    base_addr[0x204/4] = 0x00000007;  // SCHED_CONFIG: enable | timeout_en | err_en

    // Descriptor engine
    base_addr[0x220/4] = 0x00000083;  // enable | prefetch | fifo_thresh=8

    // Descriptor AXI monitor (errors only)
    base_addr[0x240/4] = 0x0000000B;  // enable | err_en | timeout_en
    base_addr[0x244/4] = 10000;       // timeout_cycles
    base_addr[0x24C/4] = 0x00000060;  // pkt_mask: errors + timeouts

    // Read/write monitors (errors only)
    base_addr[0x260/4] = 0x0000000B;  // RDMON: enable | err_en | timeout_en
    base_addr[0x280/4] = 0x0000000B;  // WRMON: enable | err_en | timeout_en

    // AXI transfer config
    base_addr[0x2A0/4] = 0x00002020;  // 32 beats read + write

    // Enable performance profiler
    base_addr[0x2B0/4] = 0x00000003;  // enable | elapsed mode
}

## 29.12	11. Troubleshooting Configuration Issues

### 29.12.1	Problem: No transfers occurring

Check: 1. cfg_channel_enable[n] set for channel n? 2. cfg_sched_enable = 1? 3. cfg_desceng_enable = 1? 4. Global enable set?

### 29.12.2	Problem: Timeout errors

Check: 1. cfg_sched_timeout_cycles too small? 2. Memory latency higher than expected? 3. AXI backpressure not handled?

Solution: - Increase timeout: cfg_sched_timeout_cycles = 20000 - Disable temporarily: cfg_sched_timeout_enable = 0

### 29.12.3	Problem: MonBus overflow

Check: 1. Too many completion packets enabled? 2. Performance packets enabled on all channels? 3. Debug packets enabled?

Solution: - Disable completion: cfg_*_mon_compl_enable = 0 - Use channel masks: cfg_*_mon_compl_mask = 8'h01 (channel 0 only) - Reduce packet types: cfg_*_mon_pkt_mask = 16'h0060 (errors + timeouts)

### 29.12.4	Problem: Low throughput

Check: 1. Prefetch disabled? 2. Burst size too small? 3. FIFO threshold too conservative?

Solution: - Enable prefetch: cfg_desceng_prefetch = 1 - Increase bursts: cfg_axi_rd_xfer_beats = 64 - Increase FIFO: cfg_desceng_fifo_thresh = 12

## 29.13	Related Documentation

- Register Map - Complete APB register specification

- Clocks and Reset - Timing requirements

- Programming Guide - Software API examples

Last Updated: 2025-12-01 Maintained By: STREAM Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Product Requirements Document (PRD)

## 30.1	STREAM - Scatter-gather Transfer Rapid Engine for AXI Memory

Version: 1.0 Date: 2025-10-17 Status: Nearly Complete - Final Integration Pending Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 30.2	1. Executive Summary

The STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory) is a simplified DMA-style accelerator designed for efficient memory-to-memory data movement with descriptor-based scatter-gather support. STREAM serves as a beginner-friendly tutorial demonstrating descriptor-based DMA engine design patterns while maintaining production-quality RTL practices.

### 30.2.1	1.1 Quick Stats

- Modules: ~8-10 SystemVerilog files (estimated)

- Channels: Maximum 8 independent channels

- Interfaces: APB (config), AXI4 (descriptor fetch + data read/write), MonBus (monitoring)

- Architecture: Simplified from RAPIDS - pure memory-to-memory

- Tutorial Focus: Aligned addresses only, straightforward data flow

- Status: Nearly complete - config interface and top-level wrapper pending

### 30.2.2	1.2 Project Goals

- Primary: Educational DMA engine demonstrating scatter-gather descriptor chains

- Secondary: Production-quality RTL suitable for FPGA/ASIC implementation

- Tertiary: Foundation for understanding more complex DMA architectures (e.g., RAPIDS)

## 30.3	2. Key Design Principles

### 30.3.1	2.1 Simplifications from RAPIDS

STREAM is intentionally simplified for tutorial purposes:

| Feature | RAPIDS | STREAM |
| --- | --- | --- |
| Network Interfaces | Yes (Network master/slave) | ✗ No (pure memory-to-memory) |
| Address Alignment | Complex fixup logic | ✓ Aligned addresses only |
| Credit Management | Exponential encoding | ✓ Simple transaction limits |
| Control Engines | Control read/write engines | ✗ No (direct APB config) |
| Descriptor Length | Chunks (4-byte) | ✓ Beats (data width) |
| Program Engine | Complex alignment FSM | ✓ Simplified coordination |

### 30.3.2	2.2 Tutorial-Friendly Features

- Aligned Addresses: Source and destination addresses must be aligned to data width

- Length in Beats: Descriptor length specified in data beats (not bytes/chunks)

- Single APB Write: One APB register write kicks off entire descriptor chain

- No Circular Buffers: Chained descriptors with explicit termination

- Parameterized Engines: Multiple AXI engine versions (compile-time selection)

## 30.4	3. Architecture Overview

### 30.4.1	3.1 Top-Level Block Diagram

STREAM (Scatter-gather Transfer Rapid Engine for AXI Memory)
├── APB Config Slave
│   └── Channel registers (8 channels, kick-off via write)
│
├── Descriptor Fetch
│   └── AXI Master (Read) - Fetch descriptors from memory
│
├── Scheduler Group (shared across all channels)
│   ├── Descriptor Engine   (FIFO, parsing - FROM RAPIDS)
│   ├── Scheduler           (Simplified FSM coordination)
│   └── Channel Arbitration (8 independent channels)
│
├── Data Path (shared resources)
│   ├── AXI Master (Read)  - Parameterized data width
│   ├── Simple SRAM        - Dual-port buffer (FROM RAPIDS)
│   └── AXI Master (Write) - Parameterized data width
│
└── MonBus Reporter
    └── MonBus Master - 64-bit monitoring packets

### 30.4.2	3.2 Data Flow

Descriptor-Based Transfer Sequence:

1. Software writes to APB channel register
   ↓
2. Descriptor fetch via AXI descriptor master
   ↓
3. Descriptor Engine parses descriptor fields
   ↓
4. Scheduler coordinates data transfer:
   a. AXI Read Engine fetches source data → SRAM buffer
   b. AXI Write Engine writes SRAM → destination
   ↓
5. Check for chained descriptor (next_descriptor_ptr != 0)
   ↓ (if chained)
6. Fetch next descriptor, repeat from step 3
   ↓ (if last)
7. Generate MonBus completion packet

Channel Independence: - 8 channels operate independently - All channels share: SRAM, AXI data masters, descriptor fetch master - Arbitration required for shared resources

## 30.5	4. Interfaces

### 30.5.1	4.1 External Interfaces

| Interface | Type | Width | Purpose | Notes |
| --- | --- | --- | --- | --- |
| APB Slave | Slave | 32-bit | Configuration, channel kick-off | Write to channel register starts transfer |
| AXI Master (Descriptor) | Master | 256-bit | Fetch descriptors from memory | Dedicated descriptor fetch path |
| AXI Master (Data Read) | Master | Parameterizable | Read source data | Multiple engine versions (compile-time) |
| AXI Master (Data Write) | Master | Parameterizable | Write destination data | Multiple engine versions (compile-time) |
| MonBus Master | Master | 64-bit | Monitor packet output | Standard AMBA format |

### 30.5.2	4.2 Descriptor Format

256-bit Descriptor Structure:

| Bits | Field | Description |
| --- | --- | --- |
| [63:0] | src_addr | Source address (64-bit, must be aligned to data width) |
| [127:64] | dst_addr | Destination address (64-bit, must be aligned to data width) |
| [159:128] | length | Transfer length in BEATS (not bytes!) |
| [191:160] | next_descriptor_ptr | Address of next descriptor (0 = last in chain) |
| [192] | valid | Descriptor is valid |
| [193] | interrupt | Generate interrupt on completion |
| [194] | last | Last descriptor in chain (explicit flag) |
| [195] | error | Error status (used for reporting) |
| [199:196] | channel_id | Channel ID (0-7) |
| [207:200] | priority | Transfer priority (for arbitration) |
| [255:208] | reserved | Reserved for future use |

Key Descriptor Features: - ✓ Chained descriptors: next_descriptor_ptr links to next descriptor - ✗ No circular buffers: Explicit termination (last flag or ptr=0) - ✓ Length in beats: Simplified for tutorial (no byte/chunk conversion) - ✓ Aligned addresses: Tutorial constraint (performance hidden for now)

## 30.6	5. Key Components

### 30.6.1	5.1 Descriptor Engine (APB-Only for STREAM)

Source: Adapted from RAPIDS descriptor_engine.sv

Purpose: - Autonomous descriptor fetch and chaining - APB interface for initial descriptor address - AXI read interface for descriptor memory fetches - Descriptor FIFO storage and distribution

Key Features: - ✓ Autonomous chaining: Automatically fetches next descriptor if next_descriptor_ptr != 0 AND last == 0 - ✓ Address validation: Validates next descriptor addresses against cfg_addr0/1_base/limit - ✓ APB blocking: APB blocked until channel_idle == 1 (channel fully idle) - ✓ Error handling: AXI errors stop chaining, set descriptor_error, block descriptor_valid

Adaptations from RAPIDS: - ✓ APB-only: Single APB write kicks off entire descriptor chain - ✓ Descriptor Read Address FIFO: 2-deep FIFO stores addresses for AXI fetch (APB + chaining) - ✓ Chaining logic: Descriptor engine autonomously manages next_descriptor_ptr chaining

Idle Signal: - descriptor_engine_idle asserted when: - FSM in RD_IDLE state - No pending descriptor fetches (address FIFO empty) - No active AXI transactions

### 30.6.2	5.2 Scheduler Group (Integration Wrapper)

Purpose: Wraps descriptor engine and scheduler into a single channel processing unit

Architecture:

scheduler_group (
    // APB interface (from APB config slave)
    .apb_valid         (apb_valid),
    .apb_ready         (apb_ready),   // Blocked when channel not idle
    .apb_addr          (descriptor_addr),

    // Channel idle signal composition (CRITICAL!)
    .channel_idle      (channel_idle),

    // Descriptor → Scheduler flow
    .descriptor_valid  (desc_valid),
    .descriptor_ready  (desc_ready),
    .descriptor_packet (desc_packet),

    // Data engine interfaces
    .datard_*          (datard_*),    // Read engine
    .datawr_*          (datawr_*),    // Write engine

    // Status
    .scheduler_idle    (sched_idle),
    .descriptor_idle   (desc_idle)
);

Channel Idle Signal Composition:

// Channel is idle ONLY when BOTH sub-blocks are idle
assign channel_idle = scheduler_idle && descriptor_engine_idle;

Why Both Signals Matter:

| Signal | Indicates | Used For |
| --- | --- | --- |
| scheduler_idle | No active data transfers, all descriptors processed | Prevents new APB request during active transfer |
| descriptor_engine_idle | No pending descriptor fetches (FIFO empty) | Prevents new APB request during chaining |
| channel_idle (AND of both) | Channel fully quiescent | Gates APB interface |

APB Blocking Logic:

// Descriptor engine blocks APB when channel not idle
assign apb_ready = apb_skid_ready_in &&
                   !r_channel_reset_active &&
                   w_desc_addr_fifo_empty &&    // No pending fetches
                   channel_idle;                 // Scheduler + descriptor idle

Example Scenario:

1. Software writes APB → descriptor_addr = 0x1000
   - channel_idle = 1 (both idle)
   - APB accepted

2. Descriptor engine fetches descriptor @ 0x1000
   - descriptor_engine_idle = 0 (fetch in progress)
   - channel_idle = 0
   - APB BLOCKED

3. Descriptor pushed to scheduler
   - descriptor_engine_idle = 1 (fetch complete)
   - scheduler_idle = 0 (transfer starting)
   - channel_idle = 0
   - APB BLOCKED

4. Scheduler completes data transfer
   - Descriptor has next_descriptor_ptr = 0x1100 (chained!)
   - Descriptor engine autonomously fetches @ 0x1100
   - descriptor_engine_idle = 0 (autonomous fetch)
   - channel_idle = 0
   - APB BLOCKED

5. Final descriptor completes (last = 1 OR next_ptr = 0)
   - scheduler_idle = 1 (transfer done)
   - descriptor_engine_idle = 1 (no more fetches)
   - channel_idle = 1
   - APB UNBLOCKED (ready for next transfer!)

Key Insight: The AND gate ensures software cannot interrupt a descriptor chain in progress!

### 30.6.3	5.3 Scheduler (Simplified from RAPIDS)

Purpose: - Coordinate descriptor-to-data-transfer flow - Manage 8 independent channels - Arbitrate shared resources (SRAM, AXI masters)

FSM States:

typedef enum logic [7:0] {
    SCHED_IDLE              = 8'b00000001,  // Idle, waiting for channel activation
    SCHED_FETCH_DESCRIPTOR  = 8'b00000010,  // Fetch descriptor via AXI master
    SCHED_PARSE_DESCRIPTOR  = 8'b00000100,  // Parse descriptor fields
    SCHED_READ_PHASE        = 8'b00001000,  // Coordinate read engine
    SCHED_WRITE_PHASE       = 8'b00010000,  // Coordinate write engine
    SCHED_CHAIN_CHECK       = 8'b00100000,  // Check for next descriptor
    SCHED_COMPLETE          = 8'b01000000,  // Transfer complete, report status
    SCHED_ERROR             = 8'b10000000   // Error state
} scheduler_state_t;

Key Differences from RAPIDS: - ✗ No credit management (just simple transaction limits) - ✗ No program engine coordination (no alignment fixup) - ✓ Simplified FSM (no control read/write phases)

### 30.6.4	5.3 AXI Read Engine (Streaming Pipeline - NO FSM)

Purpose: High-performance streaming reads from memory to SRAM buffer

Architecture: Pipelined streaming design (NO FSM for performance)

Key Insight: FSMs are horrible for performance! Instead, use: - Arbiter selects which channel’s datard_* interface gets access - Streaming pipeline continuously moves data when granted - Data interface (datard_valid, datard_ready, datard_beats_remaining) controls flow

Data Read Interface (per channel):

// Channel requests read access
input  logic        datard_valid;          // Channel has read request
output logic        datard_ready;          // Engine ready for request
input  logic [63:0] datard_addr;           // Source address (aligned)
input  logic [31:0] datard_beats_remaining; // Beats left to read
input  logic [7:0]  datard_burst_len;      // Preferred burst length
input  logic [3:0]  datard_channel_id;     // Channel ID for tracking

Multiple Versions (Compile-Time Selection): 1. Version 1 - Basic: Single outstanding read, fixed burst length 2. Version 2 - Pipelined: Multiple outstanding reads, configurable bursts 3. Version 3 - Adaptive: Dynamic burst sizing based on remaining beats

Operation:

1. Arbiter grants channel access based on priority
2. Engine accepts datard_* request (continuous streaming)
3. AXI AR channel issues read burst
4. AXI R channel streams data → SRAM (no FSM stalls!)
5. Engine updates beats_remaining, accepts next request
6. Different channels can have different burst lengths (e.g., CH0: 8 beats, CH1: 16 beats)

Example: Different Burst Lengths per Channel

// Channel 0 prefers 8-beat bursts
datard_burst_len[0] = 8'd8;

// Channel 1 prefers 16-beat bursts
datard_burst_len[1] = 8'd16;

// Engine adapts to requested burst length (within MAX_BURST_LEN)

### 30.6.5	5.4 AXI Write Engine (Streaming Pipeline - NO FSM)

Purpose: High-performance streaming writes from SRAM buffer to memory

Architecture: Pipelined streaming design (NO FSM for performance)

Key Insight: Same as read engine - no FSMs! Use streaming pipeline with arbiter.

Data Write Interface (per channel):

// Channel requests write access
input  logic        datawr_valid;           // Channel has write request
output logic        datawr_ready;           // Engine ready for request
input  logic [63:0] datawr_addr;            // Destination address (aligned)
input  logic [31:0] datawr_beats_remaining; // Beats left to write
input  logic [7:0]  datawr_burst_len;       // Preferred burst length
input  logic [3:0]  datawr_channel_id;      // Channel ID for tracking

Multiple Versions (Compile-Time Selection): 1. Version 1 - Basic: Single outstanding write, fixed burst length 2. Version 2 - Pipelined: Multiple outstanding writes, configurable bursts 3. Version 3 - Adaptive: Dynamic burst sizing based on remaining beats

Operation:

1. Arbiter grants channel access based on priority
2. Engine accepts datawr_* request (continuous streaming)
3. Engine reads data from SRAM
4. AXI AW channel issues write address
5. AXI W channel streams data (no FSM stalls!)
6. AXI B channel receives response, updates beats_remaining
7. Different channels can have different burst lengths (e.g., CH0: 8 beats, CH2: 32 beats)

Read/Write Asymmetry Example:

// Channel can use different burst lengths for read vs write
// Example: Read in small bursts, write in large bursts
datard_burst_len[0] = 8'd8;   // Read: 8 beats
datawr_burst_len[0] = 8'd16;  // Write: 16 beats

// Engine handles asymmetry via SRAM buffering

### 30.6.6	5.5 Simple SRAM

Source: Direct copy from RAPIDS simple_sram.sv

Purpose: - Dual-port SRAM buffer - Decouples read and write engines - Shared across all channels (arbitration required)

Why Reuse: - Standard dual-port SRAM design - Proven in RAPIDS integration tests - Parameterizable depth and width

## 30.7	6. Configuration and Control

### 30.7.1	6.1 APB Register Map

| Offset | Register | Access | Description |
| --- | --- | --- | --- |
| 0x0000 | GLOBAL_CTRL | RW | Global enable, reset |
| 0x0004 | GLOBAL_STATUS | RO | Global status, error flags |
| 0x0100 | CH0_CTRL | WO | Channel 0 kick-off (write descriptor address) |
| 0x0104 | CH0_STATUS | RO | Channel 0 status |
| 0x0108 | CH0_DESC_ADDR | RO | Channel 0 current descriptor address |
| 0x010C | CH0_BYTES_XFER | RO | Channel 0 bytes transferred |
| … | … | … | … (repeat for channels 1-7) |
| 0x0200 | CH1_CTRL | WO | Channel 1 kick-off |
| … |  |  |  |
| 0x0700 | CH7_CTRL | WO | Channel 7 kick-off |

Kick-Off Sequence: 1. Software writes descriptor address to CHx_CTRL register 2. STREAM fetches descriptor from memory 3. Transfer begins automatically 4. If chained, STREAM follows next_descriptor_ptr automatically 5. Completion reported via MonBus packet

### 30.7.2	6.2 Channel Configuration

Each channel independently configurable: - Descriptor start address: Written to CHx_CTRL - Priority: Encoded in descriptor (arbitration) - Interrupt enable: Per-descriptor flag - Status monitoring: Read CHx_STATUS

## 30.8	7. Resource Sharing and Arbitration

### 30.8.1	7.1 Shared Resources

All channels share: 1. Descriptor Fetch AXI Master - Fetches descriptors for all channels 2. Data Read AXI Master - Reads source data for all channels 3. Data Write AXI Master - Writes destination data for all channels 4. SRAM Buffer - Shared buffer (dual-port, but still arbitrated) 5. MonBus Reporter - Single monitor output

### 30.8.2	7.2 Arbitration Strategy

Priority-Based Round-Robin: - Channels have priority field in descriptor - Higher priority = serviced first - Within same priority: round-robin - Prevents starvation with timeout

Example Arbitration:

Channel 0: Priority 7 (highest)
Channel 1: Priority 5
Channel 2: Priority 5
Channel 3: Priority 3

Service order: CH0 → CH1 → CH2 (round-robin) → CH0 → CH1 → CH2 → CH3 …

## 30.9	8. Error Detection and Recovery

### 30.9.1	8.1 Error Types

| Error Type | Detection | Response |
| --- | --- | --- |
| Invalid descriptor | Valid bit = 0 | Skip, move to next |
| Alignment error | Address not aligned | Set error flag, halt channel |
| AXI SLVERR | AXI response | Set error flag, halt channel |
| AXI DECERR | AXI response | Set error flag, halt channel |
| Timeout | Transaction timeout | Set error flag, halt channel |
| SRAM overflow | Buffer full | Backpressure, wait |

### 30.9.2	8.2 Error Recovery

Per-Channel Error Handling: - Error sets channel to CH_ERROR state - Channel halts, does not affect other channels - Software must: 1. Read CHx_STATUS to identify error 2. Clear error condition 3. Re-kick channel with new descriptor

No Automatic Retry: - Tutorial design keeps error handling simple - Software responsible for retry logic

## 30.10	9. MonBus Integration

### 30.10.1	9.1 Standard MonBus Format

Uses standard 64-bit MonBus packet format: - [63:60] Packet Type (0=ERROR, 1=COMPL, etc.) - [59:57] Protocol (custom STREAM protocol) - [56:53] Event Code (STREAM-specific events) - [52:47] Channel ID (0-7) - [46:43] Unit ID (unused for STREAM) - [42:35] Agent ID (unused for STREAM) - [34:0] Event Data (address, byte count, etc.)

### 30.10.2	9.2 STREAM Event Codes

| Code | Event | Description |
| --- | --- | --- |
| 0x0 | DESC_START | Descriptor started |
| 0x1 | DESC_COMPLETE | Descriptor completed |
| 0x2 | READ_START | Read phase started |
| 0x3 | READ_COMPLETE | Read phase completed |
| 0x4 | WRITE_START | Write phase started |
| 0x5 | WRITE_COMPLETE | Write phase completed |
| 0x6 | CHAIN_FETCH | Chained descriptor fetch |
| 0xF | ERROR | Error occurred |

### 30.10.3	9.3 Default Configuration

Tutorial-Friendly Defaults: - Errors only: cfg_error_enable = 1, all others = 0 - Interrupts: Descriptor flag controls per-transfer interrupt - Reduces MonBus traffic for beginner understanding

## 30.11	10. Design Constraints

### 30.11.1	10.1 Tutorial Constraints (Intentional Simplifications)

| Constraint | Rationale |
| --- | --- |
| Aligned addresses only | Simplify data path, hide alignment complexity |
| Length in beats | Avoid byte/chunk conversion math |
| No circular buffers | Explicit termination easier to understand |
| Single APB kick-off | Simple software model |
| No credit management | Avoid exponential encoding complexity |

### 30.11.2	10.2 Implementation Constraints

| Parameter | Value | Notes |
| --- | --- | --- |
| Max channels | 8 | Compile-time parameter |
| Max burst length | 256 | AXI4 spec limit |
| Descriptor size | 256 bits | 4 × 64-bit words |
| SRAM depth | Parameterizable | Typical: 1024-4096 entries |
| Data width | Parameterizable | Typical: 512-bit |

## 30.12	11. Verification Strategy

### 30.12.1	11.1 Test Organization

projects/components/stream/dv/tests/
├── fub_tests/                  # Functional Unit Block tests
│   ├── descriptor_engine/      # Copy from RAPIDS (adapt imports)
│   ├── scheduler/              # Simplified scheduler tests
│   ├── axi_engines/            # Read/write engine tests
│   └── sram/                   # SRAM tests
│
└── integration_tests/          # Multi-block scenarios
    ├── single_channel/         # Single channel transfers
    ├── multi_channel/          # 8-channel concurrent
    ├── chained_descriptors/    # Descriptor chain tests
    └── error_handling/         # Error recovery tests

### 30.12.2	11.2 Test Levels

Basic (Quick Smoke Tests): - Single descriptor, single channel - Aligned addresses, simple transfers - ~30 seconds runtime

Medium (Typical Scenarios): - Multiple descriptors, 2-4 channels - Chained descriptors (2-3 deep) - ~90 seconds runtime

Full (Comprehensive Validation): - All 8 channels concurrent - Long descriptor chains (10+ deep) - Error injection, stress testing - ~180 seconds runtime

## 30.13	12. Performance Characteristics

### 30.13.1	12.1 Throughput by Engine Version

V1 (Low Performance - Tutorial Mode): - Throughput: 0.14 beats/cycle (DDR4), 0.40 beats/cycle (SRAM) - Architecture: Single outstanding transaction, blocks on completion - Use Case: Tutorial examples, embedded systems, low-latency SRAM

V2 (Medium Performance - Command Pipelined): - Throughput: 0.94 beats/cycle (DDR4), 0.85 beats/cycle (SRAM) - Improvement: 6.7x over V1 (DDR4), 2.1x over V1 (SRAM) - Architecture: Command queue (4-8 deep), hides memory latency - Use Case: General-purpose FPGA, DDR3/DDR4, best area efficiency

V3 (High Performance - Out-of-Order): - Throughput: 0.98 beats/cycle (DDR4), 0.92 beats/cycle (SRAM) - Improvement: 7.0x over V1 (DDR4), 2.3x over V1 (SRAM) - Architecture: OOO command selection, maximizes memory controller efficiency - Use Case: Datacenter, ASIC, HBM2, high-performance memory

Key Insight: Benefit scales with memory latency. V1 throughput degrades from 40% (SRAM) to 14% (DDR4), while V2/V3 maintain 94-98% throughput regardless of latency.

Configuration Parameters: - ENABLE_CMD_PIPELINE = 0: V1 (default, tutorial mode) - ENABLE_CMD_PIPELINE = 1: V2 (command pipelined) - ENABLE_CMD_PIPELINE = 1, ENABLE_OOO_DRAIN = 1 (write) or ENABLE_OOO_READ = 1 (read): V3

Factors Affecting Throughput: - Memory latency (V2/V3 hide latency via pipelining) - SRAM buffer size (limits burst pipelining) - Channel arbitration overhead - Descriptor fetch latency - Engine version (V1/V2/V3 configuration)

### 30.13.2	12.2 Latency

Transfer Latency Breakdown: - Descriptor fetch: ~10-50 cycles (depends on memory) - Read phase: (length / burst_len) × burst_latency - Write phase: (length / burst_len) × burst_latency - End-to-end: Typically <200 cycles for small transfers (V1), <100 cycles (V2/V3 pipelined)

V2/V3 Latency Hiding: - Command pipelining overlaps descriptor fetch, read, write operations - Multiple outstanding transactions hide memory latency - OOO completion (V3) reduces head-of-line blocking

### 30.13.3	12.3 Resource Utilization by Engine Version

V1 Configuration (Tutorial - Minimum Area): - Total: ~9,500 LUTs + 64 KB SRAM - Descriptor Engine (8×): ~2,400 LUTs - Scheduler (8×): ~3,200 LUTs - AXI Read Engine: ~1,250 LUTs - AXI Write Engine: ~1,250 LUTs - SRAM Controller: ~1,600 LUTs - APB Config: ~350 LUTs - MonBus AXIL Group: ~1,000 LUTs

V2 Configuration (Balanced - Best Area Efficiency): - Total: ~11,000 LUTs + 64 KB SRAM (1.16x area) - AXI Read Engine: ~2,000 LUTs (1.6x increase, 6.7x throughput) - AXI Write Engine: ~2,500 LUTs (2.0x increase, 6.7x throughput) - Other blocks: Same as V1

V3 Configuration (Maximum Performance): - Total: ~14,000 LUTs + 64 KB SRAM (1.47x area) - AXI Read Engine: ~3,500 LUTs (2.8x increase, 7.0x throughput) - AXI Write Engine: ~4,000 LUTs (3.2x increase, 7.0x throughput) - Other blocks: Same as V1

Area Efficiency Comparison: - V1: 1.00 throughput / 1.00 area = 1.00 - V2: 6.70 throughput / 1.16 area = 5.78 (best efficiency) - V3: 7.00 throughput / 1.47 area = 4.76

Recommendation: V2 provides best area efficiency for most use cases. V3 justified only for high-performance memory controllers that support OOO responses.

## 30.14	13. Development Roadmap

### 30.14.1	13.1 Phase 1: Foundation (Current)

- ✓ Directory structure

- ✓ Package definitions (stream_pkg.sv)

- ✓ Imports header (stream_imports.svh)

- ⏳ Documentation (this PRD)

### 30.14.2	13.2 Phase 2: Core Blocks

- Adapt descriptor engine from RAPIDS

- Design simplified scheduler

- Create APB config interface

- Copy simple SRAM from RAPIDS

### 30.14.3	13.3 Phase 3: Data Path

- AXI read engine (version 1 - basic)

- AXI write engine (version 1 - basic)

- SRAM integration

- Channel arbitration

### 30.14.4	13.4 Phase 4: Integration

- Top-level module

- MonBus reporter

- Integration testbench

- Single-channel validation

### 30.14.5	13.5 Phase 5: Multi-Channel

- 8-channel support

- Arbiter implementation

- Multi-channel tests

- Performance tuning

### 30.14.6	13.6 Phase 6: Advanced Engines (Future - V2/V3)

Goal: Add parameterized high-performance engine variants

V2 - Command Pipelined (Medium Performance): - Command queue implementation (4-8 deep) - W drain FSM for write engine - B response scoreboard (write) or in-order R reception (read) - Per-command SRAM pointer tracking - Parameter: ENABLE_CMD_PIPELINE = 1 - Expected: 6.7x throughput improvement over V1

V3 - Out-of-Order Completion (High Performance): - OOO command selection logic - Transaction ID matching (AXI ID to queue entry) - SRAM data availability checking (write engine) - R beat matching to queue entry (read engine) - Parameters: ENABLE_CMD_PIPELINE = 1, ENABLE_OOO_DRAIN = 1 (write) or ENABLE_OOO_READ = 1 (read) - Expected: 7.0x throughput improvement over V1

Deliverables: - Updated RTL with parameterization - Performance comparison tests (V1 vs V2 vs V3) - Tutorial documentation explaining trade-offs - Area/throughput measurements on target FPGA

## 30.15	14. Educational Value

### 30.15.1	14.1 Learning Objectives

STREAM teaches: 1. Descriptor-based DMA design patterns - Descriptor structure and parsing - Chained descriptors (scatter-gather) - Descriptor fetch via AXI

- AXI4 Memory Interface Usage

  - Burst transactions

  - Address/data/response channels

  - Outstanding transactions

- Resource Sharing and Arbitration

  - Multiple channels sharing AXI masters

  - Priority-based arbitration

  - Conflict resolution

- FSM Design and Coordination

  - Multiple interconnected FSMs

  - State machine composition

  - Error handling

- Buffer Management

  - SRAM-based buffering

  - Rate matching

  - Flow control

### 30.15.2	14.2 Progression Path

Learning Sequence: 1. STREAM (this project): Memory-to-memory DMA, aligned addresses 2. STREAM Extended: Add alignment fixup, more complex scenarios 3. RAPIDS: Add network interfaces, credit management, full complexity

## 30.16	15. Success Criteria

### 30.16.1	15.1 Functional

- ✓ Single descriptor transfer working

- ✓ Chained descriptors (2-3 deep)

- ✓ Multi-channel operation (8 channels concurrent)

- ✓ Error detection and reporting

- ✓ MonBus packet generation

- ✓ >90% functional test coverage

### 30.16.2	15.2 Quality

- ✓ Verilator compiles with 0 warnings

- ✓ All tests passing (100% success rate)

- ✓ Comprehensive documentation

- ✓ Tutorial-quality code comments

### 30.16.3	15.3 Performance

- ✓ Achieves >80% of theoretical AXI bandwidth

- ✓ <5K LUT utilization (excluding SRAM)

- ✓ <200 cycle end-to-end latency for small transfers

## 30.17	16. Open Questions (For Review)

### 30.17.1	16.1 Descriptor Engine Adaptation

- Q: Should descriptor engine use APB-only, or support additional modes?

- A (pending): TBD - depends on software use case preference

### 30.17.2	16.2 AXI Descriptor Master

- Q: Fixed 256-bit width, or parameterizable?

- A (pending): Propose fixed 256-bit for simplicity

### 30.17.3	16.3 Channel Arbitration

- Q: Fixed priority, or dynamic priority based on age/fairness?

- A (pending): Propose fixed priority with round-robin for tutorial simplicity

### 30.17.4	16.4 SRAM Partitioning

- Q: Single shared SRAM, or per-channel SRAMs?

- A (pending): Propose single shared SRAM with arbitration (matches RAPIDS pattern)

## 30.18	16. Attribution and Contribution Guidelines

### 30.18.1	16.1 Git Commit Attribution

When creating git commits for STREAM documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: STREAM documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 30.19	16.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/stream/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/stream/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the stream_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 30.20	17. References

### 30.20.1	17.1 Internal Documentation

- RAPIDS PRD: projects/components/rapids/PRD.md - Parent architecture

- RAPIDS Descriptor Engine: projects/components/rapids/rtl/rapids_fub/descriptor_engine.sv

- RAPIDS Simple SRAM: projects/components/rapids/rtl/rapids_fub/simple_sram.sv

- AMBA PRD: rtl/amba/PRD.md - MonBus integration

- Repository Guide: /CLAUDE.md - Design patterns and conventions

### 30.20.2	17.2 External References

- AXI4 Specification: ARM IHI0022E

- APB Specification: ARM IHI0024C

- CocoTB Documentation: https://docs.cocotb.org/

- Verilator Manual: https://verilator.org/guide/latest/

Document Version: 1.0 Last Updated: 2025-10-17 Review Cycle: Weekly during initial design Next Review: TBD (after user feedback) Owner: RTL Design Sherpa Project

## 30.21	Navigation

- ← Back to Root: /PRD.md

- Parent Architecture: projects/components/rapids/PRD.md

- AI Guidance: CLAUDE.md (to be created)

- Quick Start: README.md (to be created)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	Claude Code Guide: STREAM Subsystem

Version: 1.0 Last Updated: 2025-10-17 Purpose: AI-specific guidance for working with STREAM subsystem

## 31.1	Quick Context

What: STREAM - Scatter-gather Transfer Rapid Engine for AXI Memory Status: 🟡 Initial design - tutorial-focused DMA engine Your Role: Help users build a beginner-friendly descriptor-based DMA engine

📖 Complete Specification: projects/components/stream/PRD.md ← Always reference this for technical details

## 31.2	📖 Global Requirements Reference

IMPORTANT: Review /GLOBAL_REQUIREMENTS.md for all mandatory requirements

All mandatory requirements are consolidated in the global requirements document: - See: /GLOBAL_REQUIREMENTS.md - Repository-wide mandatory requirements - STREAM-Specific: Attribution format, tutorial focus (intentional simplifications) - Universal: TB location, three methods, TBBase inheritance, 100% success

This CLAUDE.md provides STREAM-specific guidance. Also review: - Root /CLAUDE.md - Repository-wide patterns - projects/components/CLAUDE.md - Project area standards (reset macros, FPGA attributes) - bin/TBClasses/CLAUDE.md - Framework usage patterns

## 31.3	Critical Rules for This Subsystem

### 31.3.1	Rule #0: Attribution Format for Git Commits

IMPORTANT: When creating git commit messages for STREAM documentation or code:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: STREAM receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

### 31.3.2	Rule #0.1: TUTORIAL FOCUS - Intentional Simplifications

⚠ STREAM is INTENTIONALLY SIMPLIFIED for educational purposes ⚠

Key Simplifications (DO NOT “fix” these): 1. ✓ Aligned addresses only - No alignment fixup logic (kept for RAPIDS) 2. ✓ Length in beats - Not bytes or chunks (simplifies math) 3. ✓ No circular buffers - Explicit chain termination only 4. ✓ No credit management - Simple transaction limits 5. ✓ Pure memory-to-memory - No network interfaces

When users ask “Can we add alignment fixup?”: - ✓ Correct answer: “STREAM intentionally keeps addresses aligned for tutorial simplicity. For complex alignment, see RAPIDS.” - ✗ Wrong answer: “Sure, let me add alignment logic…” (defeats tutorial purpose!)

### 31.3.3	Rule #0.1: Testbench Location and Test Structure (MANDATORY)

📖 See: /GLOBAL_REQUIREMENTS.md Section 2.1 for complete requirement

STREAM-Specific Directory Structure:

projects/components/stream/dv/
├── tbclasses/                    # ★ STREAM TB classes (project area!)
│   ├── scheduler_tb.py           # Scheduler testbench
│   ├── descriptor_engine_tb.py   # Descriptor engine testbench
│   └── axi_engine_tb.py          # AXI engine testbenches
└── tests/fub_tests/              # Test runners import from tbclasses/

STREAM Import Pattern:

# Import STREAM TB from project area
from projects.components.stream.dv.tbclasses.scheduler_tb import StreamSchedulerTB

# Shared utilities from framework
from TBClasses.shared.tbbase import TBBase

📖 Complete Pattern: projects/components/rapids/CLAUDE.md Rule #0.1

### 31.3.4	Rule #0.2: Three Mandatory TB Methods (MANDATORY)

📖 See: /GLOBAL_REQUIREMENTS.md Section 2.2 for complete requirement

STREAM-Specific Context:

STREAM has simpler config requirements than RAPIDS - most config can be set after reset.

Standard STREAM Pattern:

class StreamSchedulerTB(TBBase):
    async def setup_clocks_and_reset(self):
        """Standard STREAM initialization"""
        await self.start_clock('clk', freq=10, units='ns')
        await self.assert_reset()
        await self.wait_clocks('clk', 10)
        await self.deassert_reset()
        await self.wait_clocks('clk', 5)

    async def assert_reset(self):
        self.dut.rst_n.value = 0

    async def deassert_reset(self):
        self.dut.rst_n.value = 1

Note: Unlike RAPIDS, STREAM typically doesn’t need config set before reset.

### 31.3.5	Rule #1: REUSE from RAPIDS Where Appropriate

Direct Reuse (No Modification): - ✓ descriptor_engine.sv - Works with STREAM descriptors - ✓ simple_sram.sv - Standard dual-port SRAM - ✓ Descriptor engine tests - Adapt imports only

Adapt from RAPIDS: - ⚠ scheduler.sv - Simplify FSM (no credit management, no control engines) - ⚠ AXI engines - Create simplified versions (no alignment fixup)

Create New for STREAM: - 📝 APB config interface - PeakRDL-generated (like HPET), 8 channels, kick-off registers - 📝 Top-level integration - Different interface set

Always Ask Yourself: “Can I reuse from RAPIDS instead of creating new?”

### 31.3.6	Rule #2: Descriptor Format is DIFFERENT from RAPIDS

STREAM Descriptor (256-bit): - src_addr (64-bit), dst_addr (64-bit), length (beats), next_descriptor_ptr (32-bit) - Length is in BEATS, not chunks!

RAPIDS Descriptor: - Uses chunks (4-byte units) - Has alignment metadata

When comparing/referencing RAPIDS: - ✓ “RAPIDS uses chunks, STREAM uses beats for tutorial simplicity” - ✗ Don’t assume RAPIDS descriptor format applies to STREAM

### 31.3.7	Rule #3: Know the Shared Resources

All 8 channels share: 1. Descriptor fetch AXI master 2. Data read AXI master 3. Data write AXI master 4. SRAM buffer 5. MonBus reporter

Arbitration is required! - Never assume exclusive access - All shared resources need arbiter logic - Priority-based round-robin (descriptor priority field)

## 31.4	Architecture Quick Reference

### 31.4.1	Block Organization

STREAM Architecture (Estimated: 8-10 modules)
├── APB Config
│   └── apb_config_slave.sv        (To be created)
│
├── Scheduler Group
│   ├── descriptor_engine.sv       (FROM RAPIDS - adapt imports)
│   ├── scheduler.sv               (Simplified from RAPIDS)
│   └── channel_arbiter.sv         (To be created)
│
├── Data Path
│   ├── axi_read_engine_v1.sv     (Version 1: Basic)
│   ├── axi_read_engine_v2.sv     (Version 2: Pipelined)
│   ├── axi_write_engine_v1.sv    (Version 1: Basic)
│   ├── axi_write_engine_v2.sv    (Version 2: Pipelined)
│   └── simple_sram.sv             (FROM RAPIDS - no changes)
│
└── MonBus Reporter
    └── monbus_reporter.sv         (To be created)

### 31.4.2	Module Status

| Module | Source | Status | Notes |
| --- | --- | --- | --- |
| descriptor_engine.sv | RAPIDS (copy) | ✓ Copied | Adapt #include only |
| simple_sram.sv | RAPIDS (copy) | ✓ Copied | No changes needed |
| stream_pkg.sv | New | ✓ Created | Descriptor format defined |
| stream_imports.svh | New | ✓ Created | Package imports |
| scheduler.sv | RAPIDS (simplify) | ⏳ Pending | Remove credit mgmt, control engines |
| apb_config_slave.sv | New | ⏳ Pending | 8 channel registers |
| axi_read_engine_v1.sv | New | ⏳ Pending | Basic version |
| axi_write_engine_v1.sv | New | ⏳ Pending | Basic version |
| channel_arbiter.sv | New | ⏳ Pending | Priority-based round-robin |
| monbus_reporter.sv | New | ⏳ Pending | STREAM event codes |
| stream_top.sv | New | ⏳ Pending | Top-level integration |

## 31.5	Common User Questions and Responses

### 31.5.1	Q: “How is STREAM different from RAPIDS?”

A: STREAM is intentionally simplified for tutorial purposes:

Simplifications: | Feature | RAPIDS | STREAM | |———|——–|——–| | Interfaces | APB + AXI + Network | APB + AXI only | | Address Alignment | Complex fixup | Aligned only | | Credit Management | Exponential encoding | Simple limits | | Descriptor Length | Chunks (4-byte) | Beats (data width) | | Control Engines | Control read/write | None (direct APB) |

Learning Path: 1. STREAM - Basic DMA with scatter-gather 2. STREAM Extended - Add alignment fixup 3. RAPIDS - Full complexity with network + credit management

📖 See: PRD.md Section 2.1 for complete comparison table

### 31.5.2	Q: “How do I kick off a transfer?”

A: Single APB write starts descriptor chain:

// Software writes descriptor address to channel register
write_apb(ADDR_CH0_CTRL, 0x1000_0000);  // Start address of descriptor

// STREAM automatically:
// 1. Fetches descriptor from 0x1000_0000
// 2. Parses src_addr, dst_addr, length
// 3. Reads source data → SRAM
// 4. Writes SRAM → destination
// 5. Checks next_descriptor_ptr
//    - If != 0: Fetch next descriptor, repeat
//    - If == 0 or last flag set: Complete

Chained Descriptors:

Descriptor 0 @ 0x1000_0000:
  src_addr = 0x2000_0000
  dst_addr = 0x3000_0000
  length = 64 beats
  next_descriptor_ptr = 0x1000_0100  ← Points to next descriptor

Descriptor 1 @ 0x1000_0100:
  src_addr = 0x2000_1000
  dst_addr = 0x3000_1000
  length = 32 beats
  next_descriptor_ptr = 0x0000_0000  ← Last descriptor (0 = stop)

📖 See: PRD.md Section 3.2 for complete data flow

### 31.5.3	Q: “How many channels can I use?”

A: Maximum 8 independent channels:

Channel Operation: - Each channel has own FSM state - Each channel can have separate descriptor chain - All channels share: AXI masters, SRAM, MonBus

Arbitration: - Priority-based (descriptor priority field) - Round-robin within same priority - Prevents starvation with timeout

Example:

// Kick off 3 channels concurrently
write_apb(ADDR_CH0_CTRL, desc0_addr);  // Channel 0: Priority 7
write_apb(ADDR_CH1_CTRL, desc1_addr);  // Channel 1: Priority 5
write_apb(ADDR_CH2_CTRL, desc2_addr);  // Channel 2: Priority 5

// Service order: CH0 → CH1 → CH2 → CH0 → CH1 → CH2 ...

📖 See: PRD.md Section 7 for arbitration details

### 31.5.4	Q: “What’s the descriptor format?”

A: 256-bit descriptor (4 × 64-bit words):

typedef struct packed {
    logic [63:0]  reserved;              // [255:192] Reserved
    logic [7:0]   priority;              // [207:200] Priority
    logic [3:0]   channel_id;            // [199:196] Channel ID
    logic         error;                 // [195] Error flag
    logic         last;                  // [194] Last in chain
    logic         interrupt;             // [193] Generate interrupt
    logic         valid;                 // [192] Valid descriptor
    logic [31:0]  next_descriptor_ptr;   // [191:160] Next descriptor address
    logic [31:0]  length;                // [159:128] Length in BEATS ★
    logic [63:0]  dst_addr;              // [127:64] Destination address
    logic [63:0]  src_addr;              // [63:0] Source address
} descriptor_t;

★ CRITICAL: length is in BEATS, not bytes or chunks!

Example:

Data width = 512 bits = 64 bytes
Length = 16 beats
Total transfer = 16 × 64 = 1024 bytes

📖 See: PRD.md Section 4.2 for complete descriptor specification

### 31.5.5	Q: “How do I run STREAM tests?”

A: Multi-layered test approach (same as RAPIDS):

# 1. FUB (Functional Unit Block) Tests - Individual blocks
pytest projects/components/stream/dv/tests/fub_tests/scheduler/ -v
pytest projects/components/stream/dv/tests/fub_tests/descriptor_engine/ -v
pytest projects/components/stream/dv/tests/fub_tests/ -v  # All FUB tests

# 2. Integration Tests - Multi-block scenarios
pytest projects/components/stream/dv/tests/integration_tests/ -v

# Run with waveforms for debugging
pytest projects/components/stream/dv/tests/fub_tests/scheduler/ --vcd=debug.vcd
gtkwave debug.vcd

Test Organization: - FUB tests: Focus on individual block functionality - Integration tests: Verify block-to-block interfaces and complete data flows

## 31.6	Integration Patterns

### 31.6.1	Pattern 1: Basic STREAM Instantiation

stream_top #(
    .NUM_CHANNELS(8),
    .DATA_WIDTH(512),
    .ADDR_WIDTH(64),
    .SRAM_DEPTH(4096)
) u_stream (
    // Clock and Reset
    .aclk               (system_clk),
    .aresetn            (system_rst_n),

    // APB Configuration Interface
    .s_apb_paddr        (apb_paddr),
    .s_apb_psel         (apb_psel),
    .s_apb_penable      (apb_penable),
    .s_apb_pwrite       (apb_pwrite),
    .s_apb_pwdata       (apb_pwdata),
    .s_apb_pready       (apb_pready),
    .s_apb_prdata       (apb_prdata),
    .s_apb_pslverr      (apb_pslverr),

    // AXI Master - Descriptor Fetch (256-bit)
    .m_axi_desc_araddr  (desc_araddr),
    .m_axi_desc_arlen   (desc_arlen),
    .m_axi_desc_arsize  (desc_arsize),
    .m_axi_desc_arvalid (desc_arvalid),
    .m_axi_desc_arready (desc_arready),
    .m_axi_desc_rdata   (desc_rdata),
    .m_axi_desc_rresp   (desc_rresp),
    .m_axi_desc_rlast   (desc_rlast),
    .m_axi_desc_rvalid  (desc_rvalid),
    .m_axi_desc_rready  (desc_rready),

    // AXI Master - Data Read (parameterizable width)
    .m_axi_rd_araddr    (rd_araddr),
    // ... (full AXI4 AR + R channels)

    // AXI Master - Data Write (parameterizable width)
    .m_axi_wr_awaddr    (wr_awaddr),
    // ... (full AXI4 AW + W + B channels)

    // MonBus Output
    .monbus_pkt_valid   (stream_mon_valid),
    .monbus_pkt_ready   (stream_mon_ready),
    .monbus_pkt_data    (stream_mon_data)
);

### 31.6.2	Pattern 2: Descriptor Creation (Software Model)

// C/C++ software model for descriptor creation
typedef struct {
    uint64_t src_addr;           // Source address (must be aligned!)
    uint64_t dst_addr;           // Destination address (must be aligned!)
    uint32_t length;             // Transfer length in BEATS
    uint32_t next_descriptor_ptr; // Next descriptor address (0 = last)
    uint8_t  control;            // valid | interrupt | last | error | channel_id | priority
} stream_descriptor_t;

// Create descriptor chain
stream_descriptor_t desc[2];

// Descriptor 0
desc[0].src_addr = 0x80000000ULL;  // Aligned to 64B (512-bit data)
desc[0].dst_addr = 0x90000000ULL;
desc[0].length = 64;  // 64 beats × 64 bytes = 4KB transfer
desc[0].next_descriptor_ptr = (uint32_t)&desc[1];  // Chain to next
desc[0].control = 0x01;  // valid = 1

// Descriptor 1 (last)
desc[1].src_addr = 0x80001000ULL;
desc[1].dst_addr = 0x90001000ULL;
desc[1].length = 32;  // 32 beats × 64 bytes = 2KB transfer
desc[1].next_descriptor_ptr = 0;  // Last descriptor
desc[1].control = 0x45;  // valid | last | interrupt

// Kick off transfer
write_apb_reg(CH0_CTRL, (uint32_t)&desc[0]);

### 31.6.3	Pattern 3: MonBus Integration

// Always add downstream FIFO for MonBus
gaxi_fifo_sync #(
    .DATA_WIDTH(64),
    .DEPTH(256)
) u_stream_mon_fifo (
    .i_clk      (aclk),
    .i_rst_n    (aresetn),
    .i_data     (monbus_pkt_data),
    .i_valid    (monbus_pkt_valid),
    .o_ready    (monbus_pkt_ready),
    .o_data     (fifo_mon_data),
    .o_valid    (fifo_mon_valid),
    .i_ready    (consumer_ready)
);

## 31.7	Anti-Patterns to Catch

### 31.7.1	✗ Anti-Pattern 1: Adding Alignment Fixup

✗ WRONG:
"Let me add alignment fixup logic to handle unaligned addresses..."

✓ CORRECTED:
"STREAM intentionally requires aligned addresses for tutorial simplicity.
If you need unaligned transfers, that's a great learning exercise for
extending STREAM, or use RAPIDS which has full alignment support."

### 31.7.2	✗ Anti-Pattern 2: Using Length in Bytes

✗ WRONG:
descriptor.length = 4096;  // Thinking this is 4096 bytes

✓ CORRECTED:
// Length is in BEATS, not bytes!
// For 512-bit data width (64 bytes per beat):
descriptor.length = 4096 / 64;  // = 64 beats for 4096 bytes

### 31.7.3	✗ Anti-Pattern 3: Circular Buffer Descriptors

✗ WRONG:
// Descriptor chain that loops back
desc[9].next_descriptor_ptr = &desc[0];  // Circular!

✓ CORRECTED:
"STREAM does not support circular buffers (no stop condition).
Always terminate chains explicitly:
  desc[last].next_descriptor_ptr = 0;  // Stop
  desc[last].last = 1;  // Explicit last flag

### 31.7.4	✗ Anti-Pattern 4: Assuming Exclusive Channel Access

✗ WRONG:
// Assume channel has exclusive AXI master access
assign m_axi_arvalid = channel_request;  // No arbitration!

✓ CORRECTED:
// All channels share AXI masters - arbitration required
channel_arbiter u_arbiter (
    .ch_requests (channel_requests[7:0]),
    .ch_grant    (channel_grant[7:0]),
    .axi_master_available (axi_master_idle)
);

## 31.8	Debugging Workflow

### 31.8.1	Issue: Descriptor Not Fetched

Check in order: 1. ✓ APB write to CHx_CTRL register successful? 2. ✓ Descriptor address valid? 3. ✓ AXI descriptor master not stalled? 4. ✓ Descriptor memory accessible? 5. ✓ Arbiter granting descriptor fetch?

Debug commands:

pytest projects/components/stream/dv/tests/fub_tests/scheduler/ -v -s  # Verbose test
pytest projects/components/stream/dv/tests/fub_tests/scheduler/ --vcd=debug.vcd
gtkwave debug.vcd  # Inspect FSM state transitions

### 31.8.2	Issue: Data Transfer Stalls

Check in order: 1. ✓ SRAM buffer depth sufficient? 2. ✓ Source/destination addresses aligned? 3. ✓ AXI read/write engines getting grants? 4. ✓ Downstream AXI ready signals asserted? 5. ✓ Channel not in error state?

Waveform Analysis: - Check SRAM read/write pointers - Verify AXI AR/AW/R/W/B handshakes - Inspect scheduler FSM state - Check arbiter grant signals

### 31.8.3	Issue: Chained Descriptors Not Following

Check in order: 1. ✓ next_descriptor_ptr != 0? 2. ✓ last flag not set prematurely? 3. ✓ Descriptor fetch completing successfully? 4. ✓ Scheduler transitioning to CHAIN_CHECK state? 5. ✓ MonBus showing CHAIN_FETCH event?

## 31.9	Testing Guidance

### 31.9.1	Test Organization

projects/components/stream/dv/tests/
├── fub_tests/                  # Individual block tests
│   ├── descriptor_engine/      # Adapt from RAPIDS tests
│   ├── scheduler/              # Simplified scheduler tests
│   ├── axi_engines/            # Read/write engine tests
│   └── sram/                   # SRAM tests (from RAPIDS)
│
└── integration_tests/          # Multi-block scenarios
    ├── single_channel/         # Single channel transfers
    ├── multi_channel/          # 8-channel concurrent
    ├── chained_descriptors/    # Descriptor chain tests
    └── error_handling/         # Error recovery tests

### 31.9.2	Test Levels

Basic (Quick Smoke Tests): - Single descriptor, single channel - Aligned addresses, simple transfers - ~30 seconds runtime

Medium (Typical Scenarios): - Multiple descriptors, 2-4 channels - Chained descriptors (2-3 deep) - ~90 seconds runtime

Full (Comprehensive Validation): - All 8 channels concurrent - Long descriptor chains (10+ deep) - Error injection, stress testing - ~180 seconds runtime

## 31.10	Key Documentation Links

### 31.10.1	Always Reference These

This Subsystem: - projects/components/stream/PRD.md - Complete specification - projects/components/stream/README.md - Quick start guide (to be created) - projects/components/stream/known_issues/ - Bug tracking

Related: - projects/components/rapids/PRD.md - Parent architecture (for comparison) - rtl/amba/PRD.md - MonBus integration - /PRD.md - Master requirements - /CLAUDE.md - Repository guide

## 31.11	Quick Commands

# View complete specification
cat projects/components/stream/PRD.md

# Check package definition
cat projects/components/stream/rtl/includes/stream_pkg.sv

# Run tests (once created)
pytest projects/components/stream/dv/tests/fub_tests/ -v
pytest projects/components/stream/dv/tests/integration_tests/ -v

# Lint (once RTL created)
verilator --lint-only projects/components/stream/rtl/stream_fub/scheduler.sv

# Search for modules
find projects/components/stream/rtl/ -name "*.sv" -exec grep -H "^module" {} \;

## 31.12	Remember

- 📖 Tutorial focus - Intentional simplifications for learning

- 🔄 Reuse from RAPIDS - Descriptor engine, SRAM, patterns

- 📏 Length in beats - Not bytes or chunks!

- 🔗 Aligned addresses - No fixup logic

- ⛓️ Chained descriptors - No circular buffers

- 🎯 8 channels - Shared resources, arbitration required

- 🔍 MonBus standard - Same format as AMBA/RAPIDS

- 🏗️ Testbench reuse - Always create TB classes in bin/TBClasses/stream/

## 31.13	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/stream/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/stream/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the stream_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

Version: 1.0 Last Updated: 2025-10-17 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 32	STREAM Register Definitions

Purpose: PeakRDL register files and generated RTL for STREAM configuration

## 32.1	Directory Structure

regs/
├── README.md                 # This file
├── stream_regs.rdl           # Register definition (to be created)
└── generated/                # PeakRDL-generated outputs (to be created)
    ├── rtl/
    │   ├── stream_regs.sv
    │   └── stream_regs_pkg.sv
    └── docs/
        └── stream_regs.html

## 32.2	Register Generation Workflow

### 32.2.1	Phase 1: Define Register Map (Future)

Create stream_regs.rdl following the same pattern as apb_hpet.

### 32.2.2	Phase 2: PeakRDL-Generated Registers (Future)

Following the same pattern as projects/components/apb_hpet/:

- Define Register Map (stream_regs.rdl)

- addrmap stream_regs {
    name = "STREAM Configuration Registers";

    // Global control
    reg {
        field { sw=rw; hw=r; } enable;
        field { sw=rw; hw=r; } reset;
    } GLOBAL_CTRL @ 0x00;

    // Channel registers (8 channels × 16 bytes)
    regfile channel_regs {
        reg { ... } CH_CTRL;
        reg { ... } CH_STATUS;
        reg { ... } CH_RD_BURST;
        reg { ... } CH_WR_BURST;
    };

    channel_regs CH[8] @ 0x10 += 0x10;
};

- Generate RTL

- cd projects/components/stream/regs
../../bin/peakrdl_generate.py stream_regs.rdl --copy-rtl ../rtl/stream_macro

- Create APB Config Wrapper

  - Instantiate generated register block (similar to apb_hpet.sv)

  - Add apb_slave_cdc wrapper if clock domain crossing needed

  - Wire register outputs to STREAM control signals

## 32.3	Register Map (Planned)

### 32.3.1	Global Registers

| Address | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | GLOBAL_CTRL | RW | Global enable, reset |
| 0x04 | GLOBAL_STATUS | RO | Global status, channel idle/error |
| 0x08 | GLOBAL_CONFIG | RW | Global configuration |
| 0x0C | Reserved | - | - |

### 32.3.2	Channel Registers (8 × 0x10 bytes)

Each channel (0-7) has a 16-byte register block starting at 0x10 + (channel_id × 0x10):

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| +0x00 | CHx_CTRL | WO | Descriptor address (write to kick off) |
| +0x04 | CHx_STATUS | RO | Channel status, idle, error |
| +0x08 | CHx_RD_BURST | RW | Read burst length (for engine config) |
| +0x0C | CHx_WR_BURST | RW | Write burst length (for engine config) |

Example: - Channel 0 registers: 0x10, 0x14, 0x18, 0x1C - Channel 1 registers: 0x20, 0x24, 0x28, 0x2C - Channel 7 registers: 0x80, 0x84, 0x88, 0x8C

## 32.4	Integration Pattern

### 32.4.1	PeakRDL-Generated Implementation (Future):

// rtl/stream_macro/apb_config.sv
module apb_config (
    // APB interface
    input  logic [ADDR_WIDTH-1:0] paddr,
    // ...
);
    // Instantiate PeakRDL-generated registers
    stream_regs u_regs (
        .pclk    (pclk),
        .presetn (presetn),
        .paddr   (paddr),
        .psel    (psel),
        // ... APB signals

        // Register field outputs
        .global_ctrl_enable (ch_enable_internal),
        .ch0_ctrl_desc_addr (ch_desc_addr[0]),
        .ch0_rd_burst       (ch_read_burst_len[0]),
        // ... generated field outputs
    );

    // Optional: Add CDC wrapper if crossing clock domains
    // (like HPET's apb_slave_cdc)
endmodule

## 32.5	Reference Examples

HPET PeakRDL Implementation: - projects/components/apb_hpet/peakrdl/hpet_regs.rdl - Register definition - projects/components/apb_hpet/peakrdl/generated/ - Generated outputs - projects/components/apb_hpet/rtl/apb_hpet.sv - Wrapper with CDC

HPET Generation Command:

../../bin/peakrdl_generate.py hpet_regs.rdl --copy-rtl ..

## 32.6	Status

- ⏳ Phase 1: Manual apb_config.sv (placeholder)

- ⏳ Phase 2: Create stream_regs.rdl (deferred to after Phase 2 RTL implementation)

- ⏳ Phase 3: Generate register RTL with PeakRDL

- ⏳ Phase 4: Update apb_config.sv wrapper to use generated registers

Note: Per user feedback, “The apb config will be the last thing done. They will have configs done along the lines of how they are done for the hpet.”

Last Updated: 2025-10-17 Related Documentation: - ../docs/ARCHITECTURAL_NOTES.md Section 6 - APB Configuration (Deferred) - ../PRD.md Section 3.1 - APB Configuration Interface - ../../apb_hpet/peakrdl/README.md - HPET PeakRDL example