RTL Design Sherpa

Pre-Bitstream Timing Prediction on FPGA Artix-7 Companion to the ASIC Methodology White Paper

June 12, 2026

Table of Contents

1 Readme Fpga	5

2 Pre-Bitstream Timing Prediction on FPGA (Artix-7)	5

2.1 Abstract	5

2.2 1. What changes between the ASIC paper and this one	6

2.3 2. The harness on FPGA	7

2.4 3. The Vivado flow	7

2.5 4. The spreadsheet	8

2.5.1 4.1 characterization	8

2.5.2 4.2 building blocks	9

2.6 5. The validation loop (planned)	10

2.7 6. Recommended workflow	11

2.8 7. Limitations / intentional non-features	11

2.9 8. Navigation	11

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# 1	Readme Fpga

Generated: 2026-06-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Pre-Bitstream Timing Prediction on FPGA (Artix-7)

The FPGA companion to the ASIC pre-synth methodology. Same harness, same nine FUBs, same spreadsheet shape – but the lookup numbers come from Vivado primitive datasheets (and eventually from real post-route reports off the Nexys A7), and the validation loop runs through an on-board clock sweep instead of an STA tool.

Version: 0.1 (initial publication; calibration runs pending) Last Updated: 2026-06-12 Companion to: README.md (ASIC / ASAP7 paper) Status: Vivado flow live; spreadsheet seeded with datasheet numbers; hardware validation loop planned, not built.

## 2.1	Abstract

This document carries the same purpose as the ASIC paper: predict and close timing before the bitstream build ever starts. On ASIC, that prediction is grounded in Yosys + ABC + OpenSTA per-gate-delay measurements. On FPGA the inputs are different but the workflow is the same:

- characterize the relevant Xilinx primitives (LUT6, CARRY4, DSP48, BRAM, MUXF7/F8, LUTRAM, SRL) at the speed grade you’ll ship at,

- record a per-primitive ps-per-level row on the characterization sheet,

- the building-blocks sheet looks those numbers up via VLOOKUP and reports a slack column for every block you might instantiate,

- a designer at the spreadsheet sees fail / pass per block as they tweak the planned clock frequency, before committing to a 10-30 minute Vivado place-and-route round trip.

The deliverable is work/artix7_characterization.xlsx, generated by work/build_fpga_characterization_xlsx.py. The Vivado flow lives under fpga/ – a Nexys A7-100T project that builds the same char_top harness, wrapped for a single board clock and eight observation LEDs.

This methodology is explicitly proactive. It is not a debugger to reach for after Vivado throws a negative WNS at you; it is a design-time budget table the designer consults before writing the first always_ff. The point is to flip the loop – characterize once per (device, speed grade), then build with a known-good budget – so place and route becomes a confirmation step rather than the first time anyone finds out the pipeline doesn’t close.

## 2.2	1. What changes between the ASIC paper and this one

| Topic | ASIC paper | FPGA paper |
| --- | --- | --- |
| Process | TSMC-class ASAP7 PDK (open-source) | Xilinx Artix-7 (xc7a100tcsg324-1 on Nexys A7-100T) |
| Toolchain | Yosys + slang + ABC + OpenSTA | Vivado (synth + impl + bitgen) |
| Corners | TT / FF / SS (Liberty PVT corners) | Speed grade -1 / -2 / -3 |
| Frequency range | 2.0 GHz - 3.0 GHz (where 7nm starts to choke) | 100 - 300 MHz (where Artix-7 typically choke) |
| Per-level primitives | NAND, XOR, MUX, CARRY, MULT, GRAY_CTR, QUEUE | LUT6, MUXF7, MUXF8, CARRY4, DSP48, LUTRAM, SRL |
| Mixed-bag proxy | MULT (multiplier tree) | DSP48 (DSP macro) |
| Reference single-point | INV, CLK_DIV | BRAM (RAMB36 sync read) |
| Tflop source | Tcq + Tsu derived from NAND chain probe | FDRE Tcq + Tsu (~400 ps -1, ~280 ps -3) |
| Wire delay model | per-metal-layer ps/mm (M2/M3 … M8/M9) | single FPGA-routing ps/hop (default 200 ps -1) |
| Ground truth | ABC stime -c per chain | Vivado post-route report_timing_summary |
| Validation loop | corner data already there (same Liberty) | on-board MMCM clock sweep + host-side golden model |

The spreadsheet’s building blocks sheet is identical RTL with FPGA- specific numbers swapped in. The stream sheet (per-FUB port walks) is intentionally omitted from this v0.1 – the per-port hand-walks port over 1:1 from the ASIC sheet, only the looked-up per-level numbers change.

## 2.3	2. The harness on FPGA

char_top is the same nine-FUB harness described in README.md Section 5. All anti-optimisation defences (LFSR-driven inputs, outputs routed to top-level ports, dont_touch attributes) work identically under Vivado.

For board-level use a thin wrapper trims the wide observation ports down to something the Nexys A7’s 8 user LEDs can drive:

- fpga/rtl/char_top_fpga.sv instantiates char_top with all 9 FUBs enabled and XOR-reduces each FUB’s output bus to a single bit.

- 8 LEDs drive one bit per FUB; the 9th FUB folds into LED[0] via an XOR with the NAND output so both chains reach a top-level port without losing an LED.

- A 100 MHz onboard oscillator (board pin E3) feeds clk; the CPU_RESETN button (C12) feeds rst_n; four switches (J15 / L16 / M13 / R15) provide the LFSR seed nibble.

Every chain still reaches an output port, so Vivado optimisation can’t delete anything – the FPGA equivalent of the ASIC scaffold.

## 2.4	3. The Vivado flow

The flow lives under fpga/, shaped like stream_characterization/flows-stream-bridge:

fpga/
├── Makefile                      project | synth | bitstream | program | clean
├── rtl/char_top_fpga.sv          thin wrapper around char_top
├── constraints/char_top_fpga.xdc Nexys A7-100T pin map + 100 MHz clock
└── tcl/
    ├── create_project.tcl        project gen from rtl/filelists/char_top.f
    ├── synth_only.tcl            synth + utilization_synth.txt
    ├── build_all.tcl             synth + impl + bitgen + reports
    ├── program_fpga.tcl          JTAG flash via Vivado hw_server
    └── filelist_utils.tcl        env-var + +incdir + nested -f support

Quick start (with REPO_ROOT exported via the repo’s env_python):

cd projects/NexysA7/timing_characterization/fpga
make help            # see all targets
make synth           # ~2 min, posts utilization + post-synth timing
make bitstream       # ~10-30 min, full flow + all reports
make utilization     # cat the latest utilization report
make timing          # cat the latest post-route timing summary
make program         # flash if board is plugged in

Reports land under fpga/reports/:

| File | Contents |
| --- | --- |
| utilization_synth.txt | post-synth LUT/FF/BRAM/DSP totals |
| utilization_impl.txt | post-route same |
| timing_summary.txt | per-path-group WNS, the calibration source |
| timing_worst.txt | top 50 worst paths |
| power.txt | Vivado’s power estimate |

The bitstream itself lands at fpga/bitstream/timing_char.bit.

## 2.5	4. The spreadsheet

work/artix7_characterization.xlsx is structured exactly like the ASIC companion. Two sheets:

### 2.5.1	4.1 characterization

Per-primitive ps-per-level rows for the seven primitives that scale with chain depth (LUT6, MUXF7, MUXF8, CARRY4, DSP48, LUTRAM, SRL), plus a single-point reference for BRAM access time. Speed grades -1 / -2 / -3 across the five-frequency sweep (100 / 150 / 200 / 250 / 300 MHz).

Sample TT-like numbers (Artix-7 -1 ballparks):

| primitive | per-level ps (-1) | notes |
| --- | --- | --- |
| LUT6 | 200 | single-LUT propagation |
| MUXF7 | 250 | F7 cascade |
| MUXF8 | 280 | F8 cascade |
| CARRY4 | 80 | per-bit CIN -> COUT (dedicated chain) |
| DSP48 | 2500 | PROXY for mixed-bag logic |
| LUTRAM | 600 | distributed RAM read |
| SRL | 400 | SRL16 / SRL32 shift |
| BRAM (ref) | 2400 | RAMB36 sync read, 1-cycle |

Tflop (FDRE Tcq + Tsu): 400 ps at -1, 340 ps at -2, 280 ps at -3.

The Design clocks table at the bottom is the single planning point. One row per clock the design uses:

| clock name | freq (MHz) | target sg | period | input 60% | output 40% | wire/hop |
| --- | --- | --- | --- | --- | --- | --- |
| clk_main | 100 | sg-1 | derived | derived | derived | 200 |
| (5 empty slots) |  |  |  |  |  |  |

Yellow cells are editable; everything else derives. The “wire/hop” column defaults higher than the ASIC sheet (200 ps -1) because FPGA routing dominates timing more than ASIC standard-cell routing.

### 2.5.2	4.2 building blocks

One row per reusable RTL block, with editable params and corner-aware target columns:

- gaxi_skid_buffer

- gaxi_fifo_sync (REG=0)

- gaxi_fifo_sync (REG=1, BRAM-backed)

- arbiter_round_robin

- axi4_master_rd

- axi4_master_wr

- BRAM (raw RAMB36)

Each row’s slack cell pulls from the Design clocks table via VLOOKUP – change clk_main’s freq or speed grade and slack flips per block. Negative slack renders bold red, three-decimal precision (0.000).

## 2.6	5. The validation loop (planned)

The ASIC paper has STA as the ground truth – run Yosys + ABC + OpenSTA and the delay numbers come out of stime -c. On FPGA the equivalent is Vivado’s post-route report_timing_summary, which is already what make bitstream emits.

The genuinely interesting closure – “does the spreadsheet match the chip?” – needs an on-board experiment. Sketch:

- MMCM wrapper. An extra wrapper char_top_fpga_mmcm.sv takes the 100 MHz board clock and synthesises N test points (e.g. 200 / 250 / 300 / 350 MHz) through a Mixed-Mode Clock Manager. Each test clock feeds an isolated char_top instance with the same per-FUB enables the spreadsheet predicted at.

- Host-side golden model. The same LFSR reference model that the ASIC TB (dv/tbclasses/timing_char_tb.py) already implements gets replayed in software for every clock test point. If the FPGA stops producing the golden LED pattern at clock X, that’s the chip announcing functional failure at clock X.

- Calibration table. Three columns: predicted-fail-freq (spreadsheet), measured-fail-freq (chip), error (%). A small error means the spreadsheet’s per-LUT / per-CARRY4 numbers are honest; a big error means the seed values need updating with the post-route data Vivado gave you.

None of this is built yet. It’s the natural next step once a real make bitstream run produces a real timing_summary.txt we can calibrate the spreadsheet against.

## 2.7	6. Recommended workflow

- Plan. Open work/artix7_characterization.xlsx. On characterization, set clk_main freq + speed grade to your target. Flip to building blocks – any red rows are the blocks you’ll need to pipeline harder or use sparingly.

- Iterate the plan. Lower the freq until everything’s green (the design’s actual ceiling on this device); switch the speed grade to model a faster-graded part; bump wire/hop to model longer inter-CLB routes.

- Build. cd fpga; make synth for a quick utilisation check; make bitstream for the full place-and-route + reports.

- Calibrate. Read reports/timing_summary.txt after make bitstream. Compare the per-FUB WNS against what the spreadsheet predicted. Update work/build_fpga_characterization_xlsx.py’s PRIM_PS dict with the calibrated numbers and regenerate the workbook.

The methodology pays for itself the first time it tells you “your plan doesn’t fit at 300 MHz on -1 silicon” before you spend 30 minutes on a Vivado run to learn the same thing.

## 2.8	7. Limitations / intentional non-features

- Seed data are datasheet ballparks, not calibrated measurements. Treat the v0.1 workbook as a methodology demo; the per-LUT / per-CARRY4 numbers are reasonable to start a discussion but should be replaced with calibrated values from real Vivado runs.

- Wire delay is one number. ASIC has four metal layers in the workbook; FPGA’s interconnect is more uniform and the inter-CLB routing dominates. One ps/hop with a per-clock override is enough resolution for a SWAG.

- No MMCM sweep, no host golden model. The on-board validation loop in section 5 is sketched but not wired up.

- No power numbers. Vivado emits report_power; the workbook doesn’t yet model it.

## 2.9	8. Navigation

- ASIC companion paper: README.md

- Vivado flow Makefile: fpga/Makefile

- Board pin map: fpga/constraints/char_top_fpga.xdc

- FPGA wrapper RTL: fpga/rtl/char_top_fpga.sv

- FPGA spreadsheet: work/artix7_characterization.xlsx

- Spreadsheet generator: work/build_fpga_characterization_xlsx.py

- Components area: ../../components/README.md

- Repository guide: /CLAUDE.md

Version: 0.1 Last Updated: 2026-06-12 Maintained By: RTL Design Sherpa Project