RTL Design Sherpa

RLB Top Micro-Architecture Specification 1.0

September 30, 2026

Table of Contents

1 Rlb Top Mas Index	15

2 RLB Top - Overview	16

2.1 What This Block Is	16

2.2 Why It Is a Separate Specification	16

2.3 Block Inventory	16

2.4 Features	17

2.5 Deliberate Scope Boundaries	18

2.6 Applications	18

2.7 Related Documents	19

3 RLB Top - Architecture	20

3.1 Hierarchy	20

3.2 The Address Path	20

3.3 The Interrupt Fabric	21

3.3.1 Source to line	21

3.3.2 The three destinations	22

3.3.3 OR-ed in, never substituted	23

3.4 The Cascade Cross-Connect	23

3.5 The Aggregated Output	23

3.6 Boot-Interrupt Rerouting	23

3.7 Related Documents	24

4 RLB Top - Clocks and Reset	25

4.1 The Short Version	25

4.2 Clock and Reset Ports	25

4.3 Why the Extra Clocks Are Unused	25

4.4 Reset Behaviour	26

4.4.1 Reset state of the interrupt fabric	26

4.4.2 Resetting to clear an interrupt	27

4.5 Related Documents	27

5 RLB Top - Acronyms and Terminology	28

5.1 Acronyms	28

5.2 Terminology	29

5.3 Related Documents	30

6 RLB Top - References	31

6.1 External Specifications	31

6.2 In-Repository References	32

6.2.1 This subsystem	32

6.2.2 Per-block specifications	32

6.2.3 Repository practice	32

6.3 Related Documents	33

7 RLB Top - Block Hierarchy Overview	34

7.1 The Twelve Instances	34

7.2 u_apbx_xbar - the address decode	35

7.3 The two 8259 instances	35

7.4 u_ioapic - and what it is not wired to	36

7.5 u_ioapic_boot_intx - the boot-interrupt companion	36

7.6 The glue	36

7.7 Related Documents	37

8 RLB Top - The APB Crossbar Block	38

8.1 Overview	38

8.2 Generation	38

8.3 Slave Index to Window	38

8.4 Address Forwarding	39

8.5 Decode Miss Behaviour	39

8.6 What This Block Does Not Do	39

8.7 Related Documents	40

9 RLB Top - The Interrupt Fabric Block	41

9.1 Overview	41

9.2 The Routing Table	41

9.2.1 Convention versus choice	42

9.2.2 The HPET legacy pair	42

9.3 Distribution	42

9.4 What the Fabric Does Not Do	43

9.5 Verification Heritage	43

9.6 Related Documents	43

10 RLB Top - The 8259 Cascade Cross-Connect	45

10.1 Overview	45

10.2 Why the Pair Exists At All	45

10.3 The Three-Wire Cross-Connect	45

10.4 The Master: IR2 Is Forced, Not OR-ed	45

10.5 The Slave: An Inline Input Worth Naming	46

10.6 The Boot-Interrupt Side Effect	46

10.7 Verification Heritage	46

10.8 Related Documents	46

11 RLB Top - Top-Level Interface	48

11.1 Overview	48

11.2 Parameters	48

11.2.1 Why only two interrupt lines are parameters	49

11.3 Ports	50

11.3.1 Clock and Reset	50

11.3.2 APB4 Slave Interface	51

11.3.3 Peripheral External Interfaces	51

11.3.4 Ports that need care	53

11.4 Address Decode Contract	54

11.5 Integration Checklist	54

11.6 Related Documents	54

12 RLB Top - APB4 Protocol at the Subsystem Boundary	56

12.1 Overview	56

12.2 Protocol Feature Support	56

12.3 Transfer Phases	56

12.4 Wait States	57

12.5 Write Strobes and Protection	57

12.6 Error Responses	57

12.7 Rules an Integrator Must Honour	58

12.8 Related Documents	58

13 RLB Top - Interrupt Pin Reference	59

13.1 Overview	59

13.2 Block Interrupt Outputs, Pin by Pin	59

13.3 The Sixteen Legacy Lines As the Controllers See Them	62

13.4 External Interrupt Input Pins	63

13.5 The Aggregate	64

13.6 Scope Boundaries Restated	64

13.7 Related Documents	64

14 RLB Top - Initialization and Programming Guide	66

14.1 Scope	66

14.2 Address Helper	66

14.3 Bring-Up Order	67

14.4 Step 1: Confirm the Subsystem Answers	67

14.5 Step 2: Choose the Interrupt Topology	68

14.6 Step 3a: Single-Controller Configuration	68

14.7 Step 3b: Cascaded-Pair Configuration	69

14.8 Step 4: IOAPIC Redirection Entries	70

14.9 Step 5: Enable the Blocks	71

14.10 Step 6: Verify	71

14.11 Clearing an Interrupt	71

14.12 Related Documents	72

15 RLB Top - Use Cases	73

15.1 Use Case 1: A UART Interrupt Through the Master Controller	73

15.2 Use Case 2: A GPIO Interrupt Through the Cascade	73

15.3 Use Case 3: The Same Source Through the IOAPIC	74

15.4 Use Case 4: HPET Legacy Replacement	74

15.5 Use Case 5: Boot-Interrupt Rerouting	75

15.6 Use Case 6: One Interrupt Line for the Whole Subsystem	75

15.7 Coincident Sources	75

15.8 Related Documents	76

16 RLB Top - Power Management: Sleep and Wake Across Blocks	77

16.1 Overview	77

16.2 Who Participates	77

16.3 Step 1: Bring the PM Block Up	78

16.4 Step 2: Arm the Wake Sources	78

16.5 Step 3: Enter Sleep	79

16.6 Step 4: Wake	80

16.7 Step 5: The Interrupt Path for ACPI Events	80

16.8 What Sleep Does Not Disturb	81

16.9 Related Documents	81

17 RLB Top - Address Map	82

17.1 rlb_top Has No Registers	82

17.2 Decode	82

17.3 Window Map	82

17.4 Read-Safe Probe Registers	83

17.5 The IOAPIC Is Indirect	84

17.6 Interrupt Line Assignments	84

17.7 Related Documents	85

18 Retro Legacy Blocks - Product Requirements Document	86

18.1 1. Overview	86

18.1.1 1.1 Purpose	86

18.1.2 1.2 Design Philosophy	86

18.1.3 1.3 Target Applications	86

18.2 2. Implemented Blocks	87

18.2.1 2.1 HPET - High Precision Event Timer	87

18.3 3. Planned Blocks	88

18.3.1 3.1 8259 - Programmable Interrupt Controller (PIC)	88

18.3.2 3.2 8254 - Programmable Interval Timer (PIT)	88

18.3.3 3.3 GPIO - General Purpose I/O	88

18.3.4 3.4 RTC - Real-Time Clock	89

18.3.5 3.5 SMBus Controller	89

18.3.6 3.6 UART - Universal Asynchronous Receiver/Transmitter	89

18.3.7 3.7 SPI Controller	89

18.3.8 3.8 I2C Controller	90

18.3.9 3.9 Watchdog Timer	90

18.3.10 3.10 Power Management / ACPI Controller	90

18.3.11 3.11 IOAPIC - I/O Advanced Programmable Interrupt Controller	90

18.3.12 3.12 Interconnect ID / Version Registers	90

18.4 4. Integration and Wrapper Goals	91

18.4.1 4.1 Individual Block Integration	91

18.4.2 4.2 RLB Wrapper Architecture	92

18.5 5. Design Standards	94

18.5.1 5.1 Reset Handling	94

18.5.2 5.2 Register Generation	94

18.5.3 5.3 Testbench Architecture	94

18.5.4 5.4 FPGA Synthesis Attributes	95

18.5.5 5.5 Documentation Requirements	95

18.6 6. Quality Metrics	95

18.6.1 6.1 Production Readiness Criteria	95

18.6.2 6.2 Current Status	96

18.7 7. Development Roadmap	97

18.7.1 7.1 Phase 1: Foundation (Complete)	97

18.7.2 7.2 Phase 2: Core Peripherals (Next 6-9 Months)	97

18.7.3 7.3 Phase 3: Advanced Peripherals (9-15 Months)	97

18.7.4 7.4 Phase 4: System Integration (15+ Months)	98

18.8 8. References	98

18.8.1 8.1 External Standards	98

18.8.2 8.2 Internal Documentation	98

18.8.3 8.3 Block-Specific Documentation	98

18.9 9. Success Criteria	98

18.9.1 9.1 Individual Block Success	98

18.9.2 9.2 Collection Success	99

18.9.3 9.3 Long-Term Vision	99

19 RLB (Retro Legacy Blocks) - FPGA Implementation Guide	100

19.1 What Are RLB (Retro Legacy Blocks)?	100

19.1.1 Overview	100

19.1.2 Purpose	100

19.2 Implemented RLB Modules	100

19.2.1 Complete Modules	100

19.3 FPGA Implementation - Nexys A7 / Genesys2	102

19.3.1 Can RLB Be Implemented on These Boards? YES!	102

19.3.2 Target FPGA Specifications	102

19.3.3 Resource Usage Estimates	102

19.3.4 What Can Fit on These Boards?	103

19.4 RLB System Integration on FPGA	103

19.4.1 Typical FPGA System Architecture	103

19.4.2 Example Use Cases on FPGAs	104

19.5 Nexys A7 Specific Integration	105

19.5.1 Available Resources on Nexys A7	105

19.5.2 How RLB Modules Map to Nexys A7	105

19.5.3 Example Nexys A7 Top-Level	105

19.6 Implementation Recommendations	106

19.6.1 For Nexys A7-100T	106

19.6.2 For Genesys 2	106

19.7 FPGA-Specific Considerations	107

19.7.1 Clock Management	107

19.7.2 Power Management	107

19.7.3 Interrupt Routing	107

19.7.4 External Interfaces	107

19.8 Soft Processor Options	108

19.8.1 MicroBlaze (Xilinx)	108

19.8.2 RISC-V (Open Source)	108

19.8.3 Custom Soft Core	108

19.9 Step-by-Step FPGA Implementation	108

19.9.1 Phase 1: Basic System (Week 1-2)	108

19.9.2 Phase 2: Expand Peripherals (Week 3-4)	109

19.9.3 Phase 3: Advanced Features (Week 5-6)	109

19.9.4 Phase 4: Application Development (Week 7+)	109

19.10 Example Address Map for FPGA	109

19.11 Software Development	110

19.11.1 Bare-Metal Firmware	110

19.11.2 Operating System Support	110

19.12 Advantages of RLB on FPGA	110

19.12.1 Development Benefits	110

19.12.2 Technical Benefits	111

19.12.3 Vs. Microcontroller	111

19.13 Conclusion	111

19.13.1 Can RLB Be Used on Nexys A7/Genesys2? ABSOLUTELY!	111

List of Figures

No figures in this document.

List of Tables

No tables in this document.

List of Waveforms

No waveforms in this document.

# 1	Rlb Top Mas Index

Generated: 2026-10-13

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	RLB Top - Overview

## 2.1	What This Block Is

rlb_top is the integration level of the Retro Legacy Blocks subsystem. It takes one APB4 slave port and presents, behind it, a complete set of PC-compatible legacy peripherals: two interrupt controllers, an IOAPIC, two timers, a real-time clock, an SMBus controller, a GPIO controller, a UART, and a power-management block.

It is not a peripheral. It contains no registers of its own, implements no protocol, and every address it answers belongs to something it instantiates. What it does contribute is three things that exist nowhere else in the subsystem:

- An address map. A 1-to-10 APB crossbar splits one slave port into ten 4KB windows from BASE_ADDR, and answers an unmapped address with PSLVERR rather than hanging.

- An interrupt fabric. Each block’s interrupt output is routed internally onto its conventional legacy line and delivered to both 8259s and to the IOAPIC, so a board does not have to wire ten interrupt pins back in.

- A cascade. The two 8259 instances are cross-connected as a PC/AT master and slave pair, with the vector and acknowledge path between them.

## 2.2	Why It Is a Separate Specification

Each of the nine other books in this directory specifies one block. None of them can specify the above, because none of them can see it: a block knows its own interrupt output pin, not which controller that pin reaches, and certainly not what happens when two blocks assert at once.

Before this book existed, the routing table and the bring-up order lived only in the testbench under dv/tbclasses/rlb_top/. That is a workable place to verify behaviour and a poor place to look it up.

## 2.3	Block Inventory

Ten windows, all assigned. There is no reserved window.

| Window | Address range | Block | Instance | Specification |
| --- | --- | --- | --- | --- |
| 0 | 0xFEC0_0000 - 0xFEC0_0FFF | HPET | u_hpet | hpet_mas |
| 1 | 0xFEC0_1000 - 0xFEC0_1FFF | 8259 PIC (master) | u_pic | pic_8259_mas |
| 2 | 0xFEC0_2000 - 0xFEC0_2FFF | 8254 PIT | u_pit | pit_8254_mas |
| 3 | 0xFEC0_3000 - 0xFEC0_3FFF | RTC | u_rtc | rtc_mas |
| 4 | 0xFEC0_4000 - 0xFEC0_4FFF | SMBus | u_smbus | smbus_mas |
| 5 | 0xFEC0_5000 - 0xFEC0_5FFF | PM/ACPI | u_pm_acpi | pm_acpi_mas |
| 6 | 0xFEC0_6000 - 0xFEC0_6FFF | IOAPIC | u_ioapic | ioapic_mas |
| 7 | 0xFEC0_7000 - 0xFEC0_7FFF | GPIO | u_gpio | gpio_mas |
| 8 | 0xFEC0_8000 - 0xFEC0_8FFF | UART 16550 | u_uart | uart_16550_mas |
| 9 | 0xFEC0_9000 - 0xFEC0_9FFF | 8259 PIC (cascade slave) | u_pic_slave | pic_8259_mas |

Two further instances carry no address window:

| Instance | Module | Contribution |
| --- | --- | --- |
| u_apbx_xbar | apbx_xbar_1to10 | The address decode. Generated, not hand-written. |
| u_ioapic_boot_intx | ioapic_boot_intx | Turns “the IOAPIC is not delivering this pin” into a legacy PIC input. |

Twelve instances in total.

## 2.4	Features

- Single APB4 slave entry point, 32-bit address and data, with PSTRB and PPROT carried through to every block

- Ten 4KB windows on a clean power-of-two decode: the slave index is PADDR[15:12]

- Unmapped addresses complete with PSLVERR instead of stalling the bus

- Interrupt fabric covering six sourcing blocks, presented on the conventional legacy IRQ lines

- Both 8259s cross-connected as a cascaded pair, with master IR2 protected

- A single aggregated rlb_irq_out for a system that wants one interrupt input rather than ten

- Boot-interrupt rerouting available and disabled by default

- External pic_irq_in and ioapic_irq_in retained as inputs and OR-ed with the internal fabric, so an existing board keeps its interrupt path

## 2.5	Deliberate Scope Boundaries

These are decisions, not defects. Each is recorded here because each one looks like a bug to somebody reading the RTL for the first time.

IRQ2 is never driven by the fabric. In a PC/AT pair, master IR2 belongs to the cascade: it carries the slave controller’s INT, not a device. rlb_top masks IR2 off every other source and forces it from the slave, so an interrupt routed to IRQ2 reaches nothing. This is also why IRQ8 exists as a line at all.

The general HPET timers are not routed. hpet_timer_irq has no conventional legacy line, which is the same reason the HPET’s TIMER_INT_ROUTE_CAP field reads 0. Those timers reach rlb_irq_out and their own output pins. Only the two legacy-replacement outputs, hpet_legacy_irq0 and hpet_legacy_irq8, enter the fabric.

ioapic_msi_emit is not instantiated. Software can program cfg_msi_addr and cfg_msi_data through the IOAPIC’s IOWIN, but nothing in this subsystem consumes them. The pins are connected and deliberately left open rather than omitted, because an omitted pin is a PINMISSING warning that hides the gap.

Boot-interrupt rerouting has a default map, not a board map. The pin to legacy-input mapping is a board and chipset decision. The default is the published identity mapping for pins 0-7 with no reroute above that, and rerouting is disabled at reset.

Every block is instantiated with CDC_ENABLE(0). The per-block clock and reset ports are present on rlb_top’s boundary but unused in this configuration. See Clocks and Reset.

## 2.6	Applications

- A drop-in legacy peripheral subsystem for a soft-core SoC on FPGA

- A target for firmware and driver development against PC-compatible peripherals without the hardware

- A worked example of an integration level: generated address decode, an interrupt fabric with convention-versus-choice documented, and a cascade

## 2.7	Related Documents

- Architecture - how the pieces fit together

- Top-Level Interface - the integration contract

- Initialization - bring-up order

- Window Map - the address map in detail

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	RLB Top - Architecture

## 3.1	Hierarchy

rlb_top
├── u_apbx_xbar         apbx_xbar_1to10     one master, ten slaves (GENERATED)
├── u_hpet              apb4_hpet           window 0
├── u_pic               apb4_pic_8259       window 1   master of the pair
├── u_pic_slave         apb4_pic_8259       window 9   slave of the pair
├── u_pit               apb4_pit_8254       window 2
├── u_rtc               apb4_rtc            window 3
├── u_smbus             apb4_smbus          window 4
├── u_pm_acpi           apb4_pm_acpi        window 5
├── u_ioapic            apb4_ioapic         window 6
├── u_gpio              apb4_gpio           window 7
├── u_uart              apb4_uart_16550     window 8
└── u_ioapic_boot_intx  ioapic_boot_intx    no window; combinational companion

RLB Top Integration Hierarchy

Everything else in the module is wiring and two pieces of combinational logic: the interrupt fabric and the cascade cross-connect.

## 3.2	The Address Path

One APB4 slave port enters the crossbar. The crossbar decodes PADDR[15:12] into a slave index and forwards the transaction, including PSTRB and PPROT, to that slave’s port.

The crossbar is generated. It comes from the shared crossbar generator, configured as one master, ten slaves, base 0xFEC00000, 32-bit address and data, 4KB per slave. Do not hand-edit apbx_xbar_1to10.sv. The generator owns the decode-miss path, where an unmapped address completes with PSLVERR instead of hanging, and a hand-rolled copy is exactly how that behaviour gets lost - it already happened once to the crossbar this one replaced.

The crossbar routes the full address; each block takes the low 12 bits. Every peripheral instantiation slices PADDR[11:0] off its window’s channel. A block therefore sees offsets from 0x000 within its own window and needs no knowledge of BASE_ADDR. This is the one detail most likely to trip up a reader comparing the top-level port width (32 bits) against a block’s (12 bits).

| Property | Value |
| --- | --- |
| Base address | BASE_ADDR, default 0xFEC0_0000 |
| Window size | 4KB (0x1000) |
| Windows | 10, indices 0-9, none reserved |
| Slave index | PADDR[15:12] |
| Address seen by a block | PADDR[11:0] of its own window |
| Unmapped address | completes with PSLVERR |

## 3.3	The Interrupt Fabric

Each block emits its interrupt on its own output pin, and those pins remain on the boundary. The fabric additionally collects them into an internal 16-bit vector, w_fabric_irq, on the conventional legacy line numbers, and feeds that vector to three destinations.

### 3.3.1	Source to line

| Source signal | Line | Basis | Reaches |
| --- | --- | --- | --- |
| pit_timer_irq[0] OR hpet_legacy_irq0 | IRQ0 | convention (system timer) | master 8259 IR0, IOAPIC pin 0 |
| uart_irq | IRQ4 | convention (COM1) | master 8259 IR4, IOAPIC pin 4 |
| rtc_alarm_irq OR rtc_second_irq OR hpet_legacy_irq8 | IRQ8 | convention (RTC alarm) | slave 8259 IR0, IOAPIC pin 8 |
| pm_interrupt | IRQ9 | convention (ACPI) | slave 8259 IR1, IOAPIC pin 9 |
| smb_interrupt | IRQ10 | choice, parameter IRQ_SMBUS | slave 8259 IR2, IOAPIC pin 10 |
| gpio_irq | IRQ11 | choice, parameter IRQ_GPIO | slave 8259 IR3, IOAPIC pin 11 |
| hpet_timer_irq[*] | none | no conventional line exists | rlb_irq_out and its own pins only |
| - | IRQ2 | reserved for the cascade | never driven by the fabric |

Convention versus choice is a real distinction here, and it is reflected in the RTL. IRQ0, IRQ4, IRQ8 and IRQ9 come from the published typical-assignment table reproduced in the IOAPIC specification, so they are fixed as localparam inside the module: making them overridable would invite silently diverging from what every driver already assumes. SMBus and GPIO have no traditional assignment - the table offers only “Available” - so those two are this subsystem’s choice and are exposed as module parameters an integrator can move.

Two sources share IRQ0 and three share IRQ8. That is intentional and safe: HPET legacy-replacement mode exists precisely to replace the 8254 tick and the RTC periodic interrupt, and the HPET core suppresses timers 0 and 1 on hpet_timer_irq while that mode is active, so the same event cannot arrive twice.

### 3.3.2	The three destinations

Master 8259. The master sees

w_master_pic_irq = ((pic_irq_in[7:0] | w_boot_intx_pic_irq | w_fabric_irq[7:0]) & 8'hFB)
                 | {5'b0, w_spic_int, 2'b0}

Note the order: the external input, the boot-interrupt reroute and the fabric are OR-ed together first, then & 8'hFB clears bit 2, and only then is bit 2 driven from the slave’s INT. The mask is what makes the cascade trustworthy - without it an external IRQ2, or a boot-interrupt reroute onto pin 2, could impersonate the slave controller. Every other master level passes through unchanged.

Slave 8259. The slave’s irq_in is the inline expression pic_irq_in[15:8] | w_fabric_irq[15:8]. It is not a named signal, which is worth knowing when probing: with pic_irq_in held at zero, the slave’s IR line for a given IRQ is w_fabric_irq[irq].

IOAPIC. w_ioapic_irq = ioapic_irq_in | w_fabric_irq, zero-extended to the IOAPIC’s pin count. The fabric defines 16 legacy lines; the IOAPIC has IOAPIC_NUM_IRQS pins, default 24, and the upper pins are for additional devices this subsystem does not source.

### 3.3.3	OR-ed in, never substituted

pic_irq_in and ioapic_irq_in remain inputs and are OR-ed with the fabric. This is deliberate: an integrator keeps the external interrupt path, and every test that drove those pins directly still works. Replacing them would have been a silent change to the port contract.

pic_int_out is not a fabric source. Feeding the controller’s own output back into its inputs is a combinational loop. It appears only in the aggregate below, which drives nothing internally.

## 3.4	The Cascade Cross-Connect

The two 8259 instances are wired as a PC/AT pair.

| Direction | Signal | Purpose |
| --- | --- | --- |
| slave to master | w_spic_int | the slave’s INT, forced onto master IR2 |
| master to slave | w_pic_cas_ack | the master’s cascade acknowledge |
| slave to master | w_spic_vector | the slave’s vector, returned by the master in place of its own |

The master’s cas_ack_in ties low, because nothing acknowledges the master from above. The invariant this creates is that master IR2 always equals w_spic_int - not that IR2 never moves. For any IRQ8-15 source, IR2 should rise; that is the cascade working.

## 3.5	The Aggregated Output

rlb_irq_out is a single line asserted while any block interrupt is asserted:

rlb_irq_out = |{hpet_timer_irq, hpet_legacy_irq0, hpet_legacy_irq8,
                pic_int_out, pit_timer_irq,
                rtc_alarm_irq, rtc_second_irq, smb_interrupt,
                pm_interrupt, gpio_irq, uart_irq}

It exists for a system that wants one interrupt input rather than ten pins. It is a pure OR, it includes pic_int_out because that is what a single-line consumer wants to see, and it takes no part in the routing above - which is precisely why including the controller’s own output is safe here.

## 3.6	Boot-Interrupt Rerouting

The IOAPIC exports which pins it is not delivering. u_ioapic_boot_intx turns that into legacy PIC inputs, so a masked IOAPIC pin can still reach the 8259.

The map is the published identity mapping - IOAPIC pin n to legacy input n for n in 0 to 7 - narrowed to the eight inputs the master controller has, with a no-reroute code for pins 8 and above. Pin 2 is a dead entry in that table: it still names legacy input 2, but master IR2 is masked off the external inputs and driven from the slave, so a reroute onto pin 2 reaches nothing. It is left in the map so the table still reads as the identity mapping; an integrator who needs pin 2 rerouted picks a different legacy input.

Rerouting is safe by default: the enable bit resets to 0, so w_boot_intx_pic_irq is all zeros and the master’s OR is exactly pic_irq_in until software opts in.

## 3.7	Related Documents

- Overview

- Clocks and Reset

- Top-Level Interface

- Initialization

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	RLB Top - Clocks and Reset

## 4.1	The Short Version

There is one functional clock domain in this configuration: pclk. Every other clock port exists on the boundary and is unused.

That needs stating plainly, because the port list suggests otherwise.

## 4.2	Clock and Reset Ports

| Clock | Reset | Intended domain | Used in this configuration |
| --- | --- | --- | --- |
| pclk | presetn | APB, primary | Yes - everything runs here |
| hpet_clk | hpet_resetn | HPET counter | No |
| pit_clk | pit_resetn | PIT counting clock | No |
| rtc_clk | rtc_resetn | RTC time base | No |
| smbus_clk | smbus_resetn | SMBus bit clock | No |
| pm_clk | pm_resetn | PM/ACPI | No |
| ioapic_clk | ioapic_resetn | IOAPIC | No |
| gpio_clk | gpio_rstn | GPIO sampling | No |
| uart_clk | uart_rstn | UART baud reference | No |

Note the naming inconsistency in the last two: GPIO and UART use gpio_rstn and uart_rstn, not _resetn. That is the blocks’ own port naming, carried through.

## 4.3	Why the Extra Clocks Are Unused

rlb_top instantiates every CDC-capable block with CDC_ENABLE(0):

| Instance | Parameterisation |
| --- | --- |
| u_hpet | CDC_ENABLE(0) |
| u_pit | CDC_ENABLE(0) |
| u_smbus | CDC_ENABLE(0) |
| u_pm_acpi | CDC_ENABLE(0) |
| u_ioapic | CDC_ENABLE(0) |
| u_gpio | CDC_ENABLE(0) |
| u_uart | CDC_ENABLE(0) |

With CDC_ENABLE=0 a block uses the plain APB slave and ignores its second clock and reset entirely. The ports remain on rlb_top so the boundary does not change if a future configuration enables CDC on one or more blocks, but today nothing downstream of them is clocked.

What an integrator should do: tie every unused clock to pclk and every unused reset to presetn. Leaving them unconnected or tied to a constant is legal in this configuration but becomes a real bug the moment any block is rebuilt with CDC_ENABLE=1, and the failure then looks like a dead peripheral rather than a missing clock.

The RTC is the case worth flagging: a real system usually wants its time base on a separate always-on 32.768 kHz domain, which in this subsystem means instantiating apb4_rtc with CDC enabled rather than relying on rtc_clk as wired here.

## 4.4	Reset Behaviour

- Active low, asynchronous, throughout. There is no positive-polarity reset anywhere in this subsystem.

- Reset is applied through the repository’s reset macros from reset_defs.svh, which rlb_top includes. A hand-written always_ff @(posedge clk or negedge rst_n) is not used and would be rejected.

- rlb_top itself holds no state. It has no always_ff of its own: the interrupt fabric is a single always_comb, and the cascade and aggregate are continuous assignments. All reset behaviour belongs to the instantiated blocks.

### 4.4.1	Reset state of the interrupt fabric

Because the fabric is purely combinational, its reset state is whatever its sources are, and at reset those are all deasserted. Two consequences matter for bring-up:

- w_boot_intx_pic_irq is all zeros after reset, because the IOAPIC’s boot-interrupt enable resets to 0. The master’s interrupt input is therefore exactly pic_irq_in until software enables rerouting.

- IOAPIC redirection entries reset masked. A source can reach the IOAPIC’s pin and produce no delivery at all, which looks identical to a broken route. Unmasking the relevant entry is a required bring-up step, not an optimisation. See Initialization.

### 4.4.2	Resetting to clear an interrupt

The 8259 in edge-triggered mode latches INT high until it is acknowledged. The integration test suite resets the whole subsystem between phases rather than acknowledging, because the acknowledge and EOI paths have their own known limitations at block level. That is a verification convenience and not a recommendation for firmware; software should acknowledge properly. It is mentioned here because it explains why the testbench re-programs both controllers and re-arms the IOAPIC after every such phase - a full reset drops all of that configuration.

## 4.5	Related Documents

- Architecture

- Top-Level Interface

- Repository reset and clocking rules: vault/handbook/design/reset-and-clocking.md

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	RLB Top - Acronyms and Terminology

## 5.1	Acronyms

| Term | Expansion | In this subsystem |
| --- | --- | --- |
| ACPI | Advanced Configuration and Power Interface | The power-management block on window 5 |
| APB | Advanced Peripheral Bus | The bus every block speaks; APB4 here |
| APB4 | AMBA 4 APB | The revision in use, with PSTRB and PPROT |
| CDC | Clock Domain Crossing | Parameter CDC_ENABLE, 0 in this configuration |
| EOI | End Of Interrupt | Acknowledge written back to a controller |
| GPE | General Purpose Event | ACPI wake and status events, pm_gpe_events |
| GPIO | General Purpose Input/Output | The controller on window 7 |
| HPET | High Precision Event Timer | The timer on window 0 |
| ICW | Initialization Command Word | 8259 configuration writes, ICW1-ICW4 |
| IOAPIC | I/O Advanced Programmable Interrupt Controller | The controller on window 6 |
| IR | Interrupt Request line | An individual input of an 8259, IR0-IR7 |
| IRQ | Interrupt Request | A legacy line number, IRQ0-IRQ15 |
| MSI | Message Signalled Interrupt | Configurable in the IOAPIC, not emitted here |
| OCW | Operation Command Word | 8259 runtime writes, OCW1-OCW3 |
| PIC | Programmable Interrupt Controller | The 8259 pair, windows 1 and 9 |
| PIT | Programmable Interval Timer | The 8254 on window 2 |
| PM | Power Management | Used interchangeably with ACPI here |
| RLB | Retro Legacy Blocks | This subsystem |
| RTC | Real-Time Clock | The block on window 3 |
| RTE | Redirection Table Entry | An IOAPIC per-pin configuration entry |
| SMBus | System Management Bus | The controller on window 4 |
| UART | Universal Asynchronous Receiver/Transmitter | The 16550 on window 8 |

## 5.2	Terminology

Window. One of the ten 4KB address regions behind the crossbar. Window n starts at BASE_ADDR + n * 0x1000. The window number equals the crossbar’s slave index, which equals PADDR[15:12].

Fabric. The internal interrupt routing in rlb_top: the w_fabric_irq vector and the logic that builds it and distributes it. Not a bus.

Line. A legacy IRQ number, 0 to 15. A line is a position in the fabric vector, and separately an input on one of the two controllers.

Convention versus choice. A line assignment is convention if it comes from the published typical-assignment table, and a choice if this subsystem picked it because no traditional assignment exists. Convention assignments are fixed in the RTL; choices are module parameters.

Master and slave. The two 8259 instances. The master is on window 1 and drives pic_int_out; the slave is on window 9 and reaches the CPU only through the master’s IR2. Unrelated to the APB sense of master and slave.

Cascade. The connection between the two controllers. Master IR2 carries the slave’s INT, the master’s acknowledge reaches the slave, and the slave’s vector is returned in place of the master’s.

Boot interrupt. A source whose IOAPIC pin is masked, rerouted to the legacy controller so it is not lost during boot before the IOAPIC is configured.

Legacy replacement. An HPET mode in which HPET timers 0 and 1 take over the 8254 tick and the RTC periodic interrupt, appearing on hpet_legacy_irq0 and hpet_legacy_irq8 while being suppressed on hpet_timer_irq.

## 5.3	Related Documents

- Overview

- References

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	RLB Top - References

## 6.1	External Specifications

Every peripheral in this subsystem implements a published specification. Copies of the specifications used are kept in the component’s References/ directory.

| Block | Specification |
| --- | --- |
| HPET | High Precision Event Timer specification, revision 1.0a |
| 8259 PIC | 8259A programmable interrupt controller datasheet |
| 8254 PIT | 82C54 interval timer datasheet |
| RTC | MC146818 real-time clock plus RAM datasheet |
| SMBus | System Management Bus specification, version 3.3 |
| PM/ACPI | Advanced Configuration and Power Interface specification |
| IOAPIC | I/O advanced programmable interrupt controller datasheet |
| UART 16550 | 16550 asynchronous communications element datasheet |
| Bus | AMBA APB protocol specification, APB4 |

The interrupt line assignments this subsystem treats as convention - IRQ0 for the system timer, IRQ4 for COM1, IRQ8 for the RTC alarm and IRQ9 for ACPI - come from the typical-assignment table reproduced in ioapic_mas ch05. That table is the authority cited in the RTL, and it is also the evidence that IRQ10 and IRQ11 have no traditional assignment, which is why SMBus and GPIO are parameters rather than fixed.

## 6.2	In-Repository References

### 6.2.1	This subsystem

| Item | Location |
| --- | --- |
| Integration RTL | rtl/rlb_top/rlb_top.sv |
| Integration filelist | rtl/rlb_top/filelists/rlb_top.f |
| Generated crossbar | rtl/apbx_xbar/apbx_xbar_1to10.sv |
| Test runner | dv/tests/test_rlb_top.py |
| Testbench classes | dv/tbclasses/rlb_top/ |
| Subsystem requirements | PRD.md |
| Subsystem overview | README.md |
| Area facts and traps | CLAUDE.md |
| FPGA integration guide | docs/RLB_FPGA_IMPLEMENTATION_GUIDE.md |
| Implementation status | docs/IMPLEMENTATION_STATUS.md |
| Driver development reading list | docs/LegacyBlocksAndDriverGuide.md |

### 6.2.2	Per-block specifications

| Block | Specification |
| --- | --- |
| HPET | hpet_mas |
| 8259 PIC | pic_8259_mas |
| 8254 PIT | pit_8254_mas |
| RTC | rtc_mas |
| SMBus | smbus_mas |
| PM/ACPI | pm_acpi_mas |
| IOAPIC | ioapic_mas |
| GPIO | gpio_mas |
| UART 16550 | uart_16550_mas |

### 6.2.3	Repository practice

The methods this subsystem follows are recorded in the handbook rather than restated here:

| Topic | Note |
| --- | --- |
| Generated RTL discipline | vault/handbook/design/generated-rtl-discipline.md |
| Reset and clocking | vault/handbook/design/reset-and-clocking.md |
| Filelists | vault/handbook/design/filelists.md |
| Running regressions | vault/handbook/dv/running-regressions.md |
| Register map links | vault/handbook/authoring/register-map-links.md |

## 6.3	Related Documents

- Overview

- Acronyms

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	RLB Top - Block Hierarchy Overview

## 7.1	The Twelve Instances

rlb_top contains no logic blocks of its own. It instantiates twelve modules and wires them together, with two pieces of combinational glue: the interrupt fabric and the cascade cross-connect.

| Instance | Module | Window | Parameters passed |
| --- | --- | --- | --- |
| u_apbx_xbar | apbx_xbar_1to10 | - | ADDR_WIDTH, DATA_WIDTH, STRB_WIDTH, BASE_ADDR |
| u_hpet | apb4_hpet | 0 | VENDOR_ID, REVISION_ID, NUM_TIMERS, CDC_ENABLE(0) |
| u_pic | apb4_pic_8259 | 1 | none |
| u_pit | apb4_pit_8254 | 2 | NUM_COUNTERS, CDC_ENABLE(0) |
| u_rtc | apb4_rtc | 3 | none |
| u_smbus | apb4_smbus | 4 | FIFO_DEPTH, CDC_ENABLE(0) |
| u_pm_acpi | apb4_pm_acpi | 5 | CDC_ENABLE(0) |
| u_ioapic | apb4_ioapic | 6 | NUM_IRQS, CDC_ENABLE(0) |
| u_gpio | apb4_gpio | 7 | GPIO_WIDTH, SYNC_STAGES, CDC_ENABLE(0), SKID_DEPTH(2) |
| u_uart | apb4_uart_16550 | 8 | FIFO_DEPTH, SYNC_STAGES, CDC_ENABLE(0), SKID_DEPTH(2) |
| u_pic_slave | apb4_pic_8259 | 9 | none |
| u_ioapic_boot_intx | ioapic_boot_intx | - | NUM_IRQS, NUM_PIC(8), PIC_IDX_W(4), PIC_MAP |

## 7.2	u_apbx_xbar - the address decode

A generated 1-to-10 APB crossbar. It is produced by the shared crossbar generator, configured as one master, ten slaves, base 0xFEC00000, 32-bit address and data, and 4KB per slave.

Do not hand-edit it. The generator owns the decode-miss path - an unmapped address completes with PSLVERR rather than hanging the bus - and the hand-rolled crossbar this one replaced had already lost that behaviour. It is regenerated, not patched.

The crossbar forwards the full 32-bit address. Each peripheral instantiation slices PADDR[11:0] from its own channel, so a block sees offsets from 0x000 within its window.

## 7.3	The two 8259 instances

The same module appears twice, and the difference is entirely in the wiring.

|  | u_pic (master, window 1) | u_pic_slave (slave, window 9) |
| --- | --- | --- |
| irq_in | w_master_pic_irq (a named signal) | pic_irq_in[15:8] \¦ w_fabric_irq[15:8] (inline) |
| int_out | pic_int_out, a module output | w_spic_int, internal only |
| cas_vector | w_spic_vector, from the slave | 8'h00, tied off |
| cas_ack_in | 1'b0 - nothing acknowledges the master | w_pic_cas_ack, from the master |
| cas_ack | w_pic_cas_ack, to the slave | unconnected |
| inta_vector_o | unconnected | w_spic_vector, to the master |

Two details are worth carrying away. First, the slave’s irq_in is an inline expression, not a named signal, so there is nothing to probe by name; with pic_irq_in held at zero the slave’s IR line for a given IRQ is exactly w_fabric_irq[irq]. Second, the master’s IR2 is not OR-ed with anything - it is masked off every other source and then forced from the slave.

## 7.4	u_ioapic - and what it is not wired to

The IOAPIC receives w_ioapic_irq, which is the external ioapic_irq_in OR-ed with the zero-extended fabric vector. Its delivery handshake, EOI input and retry input are all on the module boundary.

Four of its configuration outputs are connected and deliberately left open:

| Pin | Why it is open |
| --- | --- |
| cfg_msi_addr | ioapic_msi_emit is not instantiated here, so nothing consumes MSI configuration |
| cfg_msi_data | as above |
| cfg_mask_vec | consumed by u_ioapic_boot_intx |
| cfg_boot_intx_en | consumed by u_ioapic_boot_intx |

The first two are the interesting case. Software can program them through IOWIN, and nothing will happen. They are wired to an empty connection rather than omitted from the instantiation because an omitted pin produces a PINMISSING warning, which is how a gap like this becomes invisible.

## 7.5	u_ioapic_boot_intx - the boot-interrupt companion

A small combinational module. It takes the raw IOAPIC pin inputs, the IOAPIC’s mask vector and its boot-interrupt enable, and produces legacy PIC inputs for the pins the IOAPIC is not delivering.

Its map, BOOT_INTX_PIC_MAP, is built as a localparam in rlb_top: the identity mapping for pins 0 through 7, and a no-reroute code for every pin above that. Field 0 is the least significant, so the identity entries are written last in the concatenation.

Pin 2 is a dead entry. The map still names legacy input 2, but master IR2 is masked off the external inputs and driven from the slave controller, so a reroute onto pin 2 reaches nothing. It is kept in the map so the table still reads as the published identity mapping; an integrator who needs pin 2 rerouted must choose a different legacy input.

Because the IOAPIC’s boot-interrupt enable resets to 0, this module’s output is all zeros after reset and the master’s interrupt input is exactly pic_irq_in until software opts in.

## 7.6	The glue

Neither of these is a module, and both are worth naming because they are where the integration behaviour lives.

The interrupt fabric is one always_comb block that builds w_fabric_irq[15:0] from the blocks’ interrupt outputs, plus three continuous assignments that distribute it: to the IOAPIC, to the master controller with the IR2 mask, and into the rlb_irq_out aggregate.

The cascade cross-connect is three wires between the two controllers, listed in the table above.

rlb_top has no always_ff of its own. It holds no state.

## 7.7	Related Documents

- Architecture - the fabric and cascade in detail

- Top-Level Interface - the port list

- Window Map

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	RLB Top - The APB Crossbar Block

## 8.1	Overview

u_apbx_xbar is the only address decoder in the subsystem. Every APB transaction that enters on s_apb_* is steered to exactly one of ten peripheral channels here, and every decode decision a debugger needs to make — which block is this address in? — is answered by this module’s window map and nothing else.

It is not hand-written. It is generated by the shared crossbar generator and treated as a build artifact: regenerated when the map changes, never patched.

## 8.2	Generation

Produced by projects/components/fabric-gen-ip/lattice-apbx-xbar/bin/apbx_xbar_generator.py:

generate_apbx_xbar(num_masters=1, num_slaves=10, base_addr=0xFEC00000,
                   addr_width=32, data_width=32, slave_size=0x1000)

rlb_top instantiates the result as apbx_xbar_1to10 and passes only the global bus parameters through: ADDR_WIDTH, DATA_WIDTH, STRB_WIDTH, BASE_ADDR. The slave count and window size are baked into the generated RTL, not parameterised at the instantiation.

## 8.3	Slave Index to Window

The generated module names its peripheral channels s0_apb_* through s9_apb_*. The mapping to the ten 4KB windows is fixed at generation time:

| Slave | Window | Block | Base offset |
| --- | --- | --- | --- |
| s0 | 0 | HPET | 0x0000 |
| s1 | 1 | Master 8259 PIC | 0x1000 |
| s2 | 2 | 8254 PIT | 0x2000 |
| s3 | 3 | RTC | 0x3000 |
| s4 | 4 | SMBus | 0x4000 |
| s5 | 5 | PM/ACPI | 0x5000 |
| s6 | 6 | IOAPIC | 0x6000 |
| s7 | 7 | GPIO | 0x7000 |
| s8 | 8 | UART 16550 | 0x8000 |
| s9 | 9 | Slave 8259 PIC | 0x9000 |

The full decode table, including probe registers for each window, lives in Chapter 5 and Chapter 3.1. This page owns the mechanism; those pages own the map.

Two facts about the map are worth recording where the crossbar is described:

- There is no reserved slave. Window 9 decoded as “Reserved” when the crossbar was generated. When the slave 8259 moved into that window (RLB/pic_8259 TASK-001) the decode already existed — only the wires on s9_apb_* changed owner. No regeneration was needed, and no reserved window remains in the address map.

- The window number is the address nibble. With 4KB windows, the slave select is exactly PADDR[15:12] above BASE_ADDR, so the window number, the slave index, and the address nibble are the same value.

## 8.4	Address Forwarding

The crossbar forwards the full 32-bit address on each slave channel. Each peripheral instantiation slices PADDR[11:0] from its own channel, so every block sees register offsets starting at 0x000 within its own window and never learns the absolute address. A block that needs no more than 12 bits of offset is therefore insulated from the base address; only rlb_top knows BASE_ADDR (default 0xFEC00000, a parameter).

## 8.5	Decode Miss Behaviour

An address that matches no slave completes with PSLVERR asserted instead of hanging the bus. This behaviour is owned by the generator, not by this instance: the generator emits the decode-miss path, and the hand-rolled crossbar this one replaced had already lost it. That history is why the rule is regenerate, don’t edit — a patch on the generated file is exactly how the fix disappears. The unmapped-address error response is exercised by the func suite (see the book index, “What the suite proves”, item 4).

## 8.6	What This Block Does Not Do

- No interrupt involvement. The crossbar is a pure address/data path. Interrupts route around it entirely — see The Interrupt Fabric.

- No clock or reset generation. It runs on pclk / presetn like every block in this configuration; all CDC_ENABLE(0).

- No slave-side wait-state merging policy beyond what the generator emits: each peripheral’s PREADY is returned on its own window.

## 8.7	Related Documents

- Block Hierarchy Overview - the twelve instances

- Top-Level Interface - the address decode contract this block implements

- Window Map - the decode table

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	RLB Top - The Interrupt Fabric Block

## 9.1	Overview

The interrupt fabric is not a module. It is one always_comb block that builds w_fabric_irq[15:0] from the blocks’ interrupt outputs, plus three continuous assignments that distribute the result: to the two 8259s, to the IOAPIC, and into the rlb_irq_out aggregate. It is listed here as a block because the integration behaviour lives in this glue, and a reader who skips it understands the address map but not the subsystem.

Its job: put every block interrupt on its conventional legacy line so that standard driver software and either interrupt controller see a recognisable PC/AT wiring, without the board having to wire ten interrupt pins back in.

RLB Top Interrupt Fabric

## 9.2	The Routing Table

Seven block-level sources drive the fabric’s six lines — PIT channel 0 and HPET’s legacy pair both land on IRQ0. The six driven lines:

| Line | Driver(s) | Origin |
| --- | --- | --- |
| IRQ0 | pit_timer_irq[0] ¦ hpet_legacy_irq0 | 8254 channel-0 tick, replaced by HPET timer 0 in legacy-replacement mode |
| IRQ4 | uart_irq | UART 16550 (COM1 by convention) |
| IRQ8 | rtc_alarm_irq ¦ rtc_second_irq ¦ hpet_legacy_irq8 | RTC alarm + periodic, replaced by HPET timer 1 in legacy mode |
| IRQ9 | pm_interrupt | PM/ACPI |
| IRQ10 | smb_interrupt | SMBus — choice, parameter IRQ_SMBUS |
| IRQ11 | gpio_irq | GPIO — choice, parameter IRQ_GPIO |

Every other fabric line is constant zero, including IRQ2, which is deliberately never driven: it is the 8259 cascade input, carried from the slave controller’s INT (see The Cascade Cross-Connect). Anything routed onto IRQ2 reaches nothing — QEMU shipped exactly that bug, which is why the line is commented in the RTL rather than left implicit.

### 9.2.1	Convention versus choice

IRQ0/4/8/9 come straight from the “Typical IRQ Assignments” table in the IOAPIC book’s register chapter, and are fixed localparams inside rlb_top. Making them overridable would invite silently diverging from the wiring every driver already assumes. SMBus and GPIO have no traditional line — the table offers only “Available” — so those two are this subsystem’s choice and are exposed as module parameters, meant to be changed by an integrator who needs them elsewhere.

### 9.2.2	The HPET legacy pair

hpet_legacy_irq0 and hpet_legacy_irq8 are not the general hpet_timer_irq outputs. They are asserted only while HPET_CONFIG.legacy_replacement is set, and while it is set the HPET core suppresses timers 0/1 on their own pins — so the HPET and the device it replaces can never both drive the same line for the same event. The general hpet_timer_irq lines are deliberately not routed into the fabric: they have no conventional legacy line (the same reason TIMER_INT_ROUTE_CAP reads 0), and they reach rlb_irq_out and their own pins only.

## 9.3	Distribution

The built vector goes to three consumers:

Both 8259s. The master sees the low half through the IR2 mask and cascade force described in the cascade page; the slave OR-es the upper half into its external input inline: pic_irq_in[15:8] | w_fabric_irq[15:8]. The fabric is OR-ed into the external inputs, not substituted for them — pic_irq_in stays an input, so a board keeps its external interrupt path and every existing test keeps its direct drive.

The IOAPIC. w_ioapic_irq = ioapic_irq_in | {zero-extend of w_fabric_irq} — the same sources on the same pin numbers, zero-extended to IOAPIC_NUM_IRQS (24). The upper eight pins are PCI or additional devices this subsystem does not source; they remain for the board.

The aggregate. rlb_irq_out is a pure OR of every block interrupt, including pic_int_out — the delivered controller output, which is exactly what a single-line consumer wants. pic_int_out appears only here: feeding the 8259’s own output back into its inputs would be a combinational loop, and the aggregate drives no internal logic, so it is safe to include.

## 9.4	What the Fabric Does Not Do

- No masking, latching, or priority. It is wires and one OR tree. All policy — masking, vector selection, delivery — belongs to the 8259s and the IOAPIC, and to the software that programs them (Chapter 4).

- No CDC. The fabric is combinational in the pclk domain. Every block runs CDC_ENABLE(0) in this configuration, so block interrupts are already synchronous to pclk before they reach the fabric.

- No boot-interrupt rerouting. That is the separate u_ioapic_boot_intx companion, described in the Block Hierarchy Overview — it produces its own PIC inputs, which the master’s input OR then combines with the fabric’s.

## 9.5	Verification Heritage

The routing table and the “OR-ed, not substituted” contract are the subject of RLB TASK-015 (the fabric), TASK-017/TASK-018 (per-line and per-block routing proof) and TASK-019 (the follow-up batch). The full suite proves each of the six sourcing blocks reaches its own master or slave IR line, per line rather than in aggregate, and its IOAPIC pin, and passes coincident-assert cases across both PICs.

## 9.6	Related Documents

- Block Hierarchy Overview - where the fabric sits among the twelve instances

- The Cascade Cross-Connect - why IRQ2 is never driven

- Per-Block Interrupt Pin Reference - pin-level table including the pins the fabric does not touch

- Use Cases - routing an interrupt end to end

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	RLB Top - The 8259 Cascade Cross-Connect

## 10.1	Overview

The subsystem carries a PC/AT interrupt pair: u_pic is the master (window 1), u_pic_slave is the slave (window 9), and the two are wired together inside rlb_top exactly as the cascaded configuration the 8259 was designed for. The cross-connect between them is three named signals — the only internal state path rlb_top owns besides the interrupt fabric. This page is the wiring in detail; the software sequences that bring the pair up are in Chapter 4.1.

## 10.2	Why the Pair Exists At All

Sixteen interrupt inputs need two 8259s: the master covers IRQ0-7, the slave covers IRQ8-15. The slave’s window at 0xFEC09000 is what makes IRQ8-15 exist — and IRQ8 in particular is the line HPET timer 1 replaces when legacy-replacement mode is on (RLB/hpet TASK-003).

## 10.3	The Three-Wire Cross-Connect

| Signal | From | To | Carries |
| --- | --- | --- | --- |
| w_spic_int | slave int_out | master IR2 (forced) | The slave’s interrupt request |
| w_pic_cas_ack | master cas_ack | slave cas_ack_in | The cascade acknowledge |
| w_spic_vector | slave inta_vector_o | master cas_vector | The slave’s vector, returned by the master in place of its own |

The remaining cascade pins are tied or left open by role: the master is cas_ack_in = 1'b0 because nothing above it acknowledges its INT, and its inta_vector_o is unconnected because nobody above consumes the vector. The slave’s cas_vector input is 8'h00 — it never forwards a deeper cascade — and its cas_ack output is unconnected.

## 10.4	The Master: IR2 Is Forced, Not OR-ed

The master’s interrupt input is:

assign w_master_pic_irq = ((pic_irq_in[7:0] | w_boot_intx_pic_irq
                            | w_fabric_irq[7:0]) & 8'hFB)
                        | {5'b0, w_spic_int, 2'b0};

Reading it inside out: the external inputs, the boot-interrupt reroute, and the fabric’s low half are OR-ed together, then bit 2 is masked off (& 8'hFB), and then the slave’s INT is forced onto that bit. In a PC/AT pair, master IR2 belongs to the cascade — it carries the slave’s INT, not an external device — so an external IRQ2, or a boot-interrupt reroute onto pin 2, would otherwise impersonate the slave. Mask-then-force makes that impossible: every other master level is unchanged, and IR2 is exactly the slave’s request.

This is also why IRQ2 reaches nothing from the outside. The address map and the interrupt map both still show the line; it is consumed internally by construction.

## 10.5	The Slave: An Inline Input Worth Naming

The slave’s interrupt input is an inline expression, not a named signal:

.irq_in (pic_irq_in[15:8] | w_fabric_irq[15:8]),

There is nothing to probe by name. With pic_irq_in held at zero, the slave’s IR line for a given IRQ is exactly w_fabric_irq[irq] — which is how the full suite verifies the slave-routed sources per line: drive the block, watch w_fabric_irq[15:8], and the slave’s view is known by construction.

On the cascade side the slave exports its vector upward (inta_vector_o → w_spic_vector → master’s cas_vector), so when the master acknowledges a slave-sourced interrupt, the vector the CPU eventually reads is the slave’s — the master returns it in place of its own. That vector path is the entire reason w_spic_vector exists as a named wire rather than a port-to-port connection.

## 10.6	The Boot-Interrupt Side Effect

u_ioapic_boot_intx can reroute IOAPIC pins onto legacy PIC inputs, and its identity map still names legacy input 2 for pin 2. That entry has been dead since the cascade took IR2: a reroute onto pin 2 is masked off the master’s input before the slave’s INT is forced on, so it reaches nothing. The map keeps the entry so it still reads as the published 82093AA identity mapping; an integrator who genuinely needs pin 2 rerouted must choose a different legacy input via PIC_MAP.

## 10.7	Verification Heritage

The cross-connect is the subject of RLB/pic_8259 TASK-001. The func suite proves the slave answers on window 9 and that the aggregated IRQ reaches rlb_irq_out; the full suite adds the cascade invariant — a slave-routed source delivered end to end through the master’s int_out — and coincident asserts spanning both controllers. The bring-up sequences (single controller, cascaded pair) transcribed into Chapter 4.1 come from the cocotb helpers init_pic and init_pic_cascade, which are known to work because this suite passes.

## 10.8	Related Documents

- Block Hierarchy Overview - the two-instance wiring table

- The Interrupt Fabric - what reaches the slave’s IR lines

- Initialization - programming the pair (ICW3 master/slave values)

- Window Map - window 9 decode

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	RLB Top - Top-Level Interface

## 11.1	Overview

rlb_top is the integration contract for the whole subsystem: one APB4 slave port, the peripherals’ external pins, and one aggregated interrupt output. This chapter is the pin list, the parameters, and the address decode rules an integrator has to honour.

## 11.2	Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | APB address width. The subsystem decodes a full system address, not a windowed one. |
| DATA_WIDTH | int | 32 | APB data width. |
| STRB_WIDTH | int | DATA_WIDTH/8 | Write-strobe width. Derived; do not override independently. |
| BASE_ADDR | logic [ADDR_WIDTH-1:0] | 32'hFEC00000 | Base of the ten-window region. |
| HPET_VENDOR_ID | int | 1 | Driven into the HPET capability register. |
| HPET_REVISION | int | 1 | Driven into the HPET capability register. |
| HPET_NUM_TIMERS | int | 2 | HPET comparator count. Sets the width of hpet_timer_irq. |
| PIT_NUM_COUNTERS | int | 3 | 8254 counter count. Sets the width of pit_gate_in and pit_timer_irq. |
| IOAPIC_NUM_IRQS | int | 24 | IOAPIC pin count. Sets the width of ioapic_irq_in. |
| GPIO_WIDTH | int | 32 | GPIO pin count. |
| GPIO_SYNC_STAGES | int | 2 | GPIO input synchroniser depth. |
| UART_FIFO_DEPTH | int | 16 | UART FIFO depth. |
| UART_SYNC_STAGES | int | 2 | UART input synchroniser depth. |
| SMBUS_FIFO_DEPTH | int | 32 | SMBus FIFO depth. |
| IRQ_SMBUS | int | 10 | Choice, not convention. The legacy line SMBus is routed to. |
| IRQ_GPIO | int | 11 | Choice, not convention. The legacy line GPIO is routed to. |

### 11.2.1	Why only two interrupt lines are parameters

IRQ0, IRQ4, IRQ8 and IRQ9 are fixed as localparam inside the module because they come from the published typical-assignment table: the system timer, COM1, the RTC alarm and ACPI respectively. Exposing them would invite silently diverging from what every driver already assumes.

SMBus and GPIO have no traditional assignment - the table lists IRQ10 and IRQ11 as available - so those two are this subsystem’s choice and are parameters an integrator can move.

## 11.3	Ports

### 11.3.1	Clock and Reset

| Port | Dir | Width | Notes |
| --- | --- | --- | --- |
| pclk | in | 1 | APB clock. The only functional clock in this configuration. |
| presetn | in | 1 | APB reset, active low, asynchronous. |
| hpet_clk, hpet_resetn | in | 1 each | Unused: u_hpet is CDC_ENABLE(0). Tie to pclk / presetn. |
| pit_clk, pit_resetn | in | 1 each | Unused: CDC_ENABLE(0). Tie to pclk / presetn. |
| rtc_clk, rtc_resetn | in | 1 each | Unused in this configuration. Tie to pclk / presetn. |
| smbus_clk, smbus_resetn | in | 1 each | Unused: CDC_ENABLE(0). Tie to pclk / presetn. |
| pm_clk, pm_resetn | in | 1 each | Unused: CDC_ENABLE(0). Tie to pclk / presetn. |
| ioapic_clk, ioapic_resetn | in | 1 each | Unused: CDC_ENABLE(0). Tie to pclk / presetn. |
| gpio_clk, gpio_rstn | in | 1 each | Unused: CDC_ENABLE(0). Note _rstn, not _resetn. |
| uart_clk, uart_rstn | in | 1 each | Unused: CDC_ENABLE(0). Note _rstn, not _resetn. |

See Clocks and Reset for why the secondary clocks are present but unused, and what to do about it.

### 11.3.2	APB4 Slave Interface

The single entry point to the subsystem.

| Port | Dir | Width | Notes |
| --- | --- | --- | --- |
| s_apb_PSEL | in | 1 |  |
| s_apb_PENABLE | in | 1 |  |
| s_apb_PADDR | in | ADDR_WIDTH | Full system address, not a window offset. |
| s_apb_PWRITE | in | 1 |  |
| s_apb_PWDATA | in | DATA_WIDTH |  |
| s_apb_PSTRB | in | STRB_WIDTH | Carried through to the selected block. |
| s_apb_PPROT | in | 3 | Carried through to the selected block. |
| s_apb_PRDATA | out | DATA_WIDTH |  |
| s_apb_PSLVERR | out | 1 | Also the decode-miss response. |
| s_apb_PREADY | out | 1 |  |

### 11.3.3	Peripheral External Interfaces

| Block | Inputs | Outputs |
| --- | --- | --- |
| HPET | - | hpet_timer_irq[HPET_NUM_TIMERS-1:0], hpet_legacy_irq0, hpet_legacy_irq8 |
| 8259 PIC | pic_irq_in[15:0] | pic_int_out |
| 8254 PIT | pit_gate_in[PIT_NUM_COUNTERS-1:0] | pit_timer_irq[PIT_NUM_COUNTERS-1:0] |
| RTC | - | rtc_alarm_irq, rtc_second_irq |
| SMBus | smb_scl_i, smb_sda_i | smb_scl_o, smb_scl_t, smb_sda_o, smb_sda_t, smb_interrupt |
| PM/ACPI | pm_gpe_events[31:0], pm_gpe1_events[31:0], pm_power_button_n, pm_sleep_button_n, pm_rtc_alarm, pm_ext_wake_n, pm_wdt_reset_n, pm_ext_reset_n, pm_power_domain_ack[7:0] | pm_clock_gate_en[31:0], pm_power_domain_en[7:0], pm_sys_reset_req, pm_periph_reset_req, pm_interrupt |
| IOAPIC | ioapic_irq_in[IOAPIC_NUM_IRQS-1:0], ioapic_irq_out_ready, ioapic_irq_out_retry, ioapic_eoi_in, ioapic_eoi_vector[7:0] | ioapic_irq_out_valid, ioapic_irq_out_vector[7:0], ioapic_irq_out_dest[7:0], ioapic_irq_out_deliv_mode[2:0], ioapic_irq_out_dest_mode |
| GPIO | gpio_in[GPIO_WIDTH-1:0] | gpio_out[GPIO_WIDTH-1:0], gpio_oe[GPIO_WIDTH-1:0], gpio_irq |
| UART 16550 | uart_rx, uart_cts_n, uart_dsr_n, uart_ri_n, uart_dcd_n | uart_tx, uart_dtr_n, uart_rts_n, uart_rxrdy_n, uart_txrdy_n, uart_out1_n, uart_out2_n, uart_irq |
| Subsystem | - | rlb_irq_out |

### 11.3.4	Ports that need care

| Port | What to know |
| --- | --- |
| pic_irq_in[2] | Ignored. Master IR2 is the cascade input, forced from the slave controller. Driving this bit reaches nothing. |
| pic_irq_in[15:8] | Goes to the slave controller, not the master. |
| ioapic_irq_out_retry | Tie low if the receiver always accepts. It is the IOAPIC’s half of delegated lowest-priority delivery. |
| pm_power_domain_ack[7:0] | Tie high unless the rail sequencer is enabled with an acknowledge requirement. |
| pm_wdt_reset_n, pm_ext_reset_n | Tie high (inactive) if the system has no watchdog or reset button. They are reported in the PM reset-status register. |
| pm_gpe1_events[31:0] | Tie to zero when the system has only one GPE bank. |
| uart_rxrdy_n, uart_txrdy_n | DMA handshake pins. Leave unconnected in a programmed-I/O system. |
| hpet_timer_irq | Not routed into the interrupt fabric. Wire it yourself if you need it delivered. |
| rlb_irq_out | A pure OR of every block interrupt including pic_int_out. Drives nothing internally. |

## 11.4	Address Decode Contract

| Property | Value |
| --- | --- |
| Base | BASE_ADDR, default 0xFEC0_0000 |
| Window size | 0x1000 (4KB) |
| Window count | 10, indices 0-9 |
| Slave index | PADDR[15:12] |
| Offset seen by a block | PADDR[11:0] |
| Reserved windows | none - all ten are assigned |
| Unmapped address | completes normally with PSLVERR asserted; it does not hang |

The decode-miss behaviour is a property of the generated crossbar, and it is the reason the crossbar must be regenerated rather than hand-edited.

## 11.5	Integration Checklist

- Drive pclk and presetn. Tie every secondary clock to pclk and every secondary reset to presetn unless you have rebuilt a block with CDC_ENABLE=1.

- Connect the APB4 slave port, passing the full system address.

- Decide whether you want the internal interrupt fabric, the external pins, or both. Both is the default and costs nothing: they are OR-ed.

- If you are not driving external legacy interrupts, tie pic_irq_in and ioapic_irq_in to zero. The fabric then supplies everything.

- Tie off the PM pins listed above according to what your system actually has.

- Choose IRQ_SMBUS and IRQ_GPIO if the defaults clash with your platform.

- Decide how the IOAPIC’s delivery handshake is consumed, and tie ioapic_irq_out_retry low if your receiver always accepts.

- Use rlb_irq_out if you want a single interrupt input; otherwise use pic_int_out and the IOAPIC delivery interface.

## 11.6	Related Documents

- Architecture

- Clocks and Reset

- Initialization

- Window Map

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	RLB Top - APB4 Protocol at the Subsystem Boundary

## 12.1	Overview

This page is the protocol contract on the single APB slave port s_apb_* — the boundary a SoC fabric must honour to talk to the subsystem. The signal list and the address decode table live in Top-Level Interface; this page owns the rules: phases, wait states, strobes, protection attributes, and the two ways a transaction can complete with an error.

The boundary is the master side of the generated crossbar (Chapter 2.1). Everything below is a property of that generated fabric plus the blocks behind it, not of any hand-written glue in rlb_top.

## 12.2	Protocol Feature Support

| Feature | Support |
| --- | --- |
| PSEL / PENABLE / PWRITE | Yes — standard two-phase APB |
| PADDR | 32-bit, full system address (not a window offset) |
| PWDATA / PRDATA | 32-bit |
| PREADY | Yes — wait states come from the addressed block (see below) |
| PSLVERR | Yes — two error sources (see below) |
| PSTRB | Yes — carried through to the selected block |
| PPROT | Yes — carried through to the selected block; checking is a per-block decision |

The interface is APB4. A master that speaks only APB3 can still use it: tie PSTRB to all-ones on writes and all-zero on reads (the APB4 rule — PSTRB must be zero during a read), and tie PPROT to zero.

## 12.3	Transfer Phases

A transaction is the standard APB pair:

- Setup. The master asserts PSEL, presents the full system address on PADDR, the direction on PWRITE, and (for writes) PWDATA, PSTRB, PPROT.

- Access. The master asserts PENABLE. The transfer completes on the rising edge where the slave’s PREADY is high; read data is sampled on PRDATA in the same cycle.

The minimum transaction length is therefore two pclk cycles. There is no back-to-back packing and no burst concept — APB4 is single-transfer, and the crossbar does not add one.

## 12.4	Wait States

PREADY returned to the master is the selected block’s own PREADY. The fabric between the boundary and the block is combinational: it adds no wait states of its own, so the wait-state profile of a transaction is entirely the addressed block’s, as documented in that block’s book. Software that polls a slow register (a UART line-status register after a transmit, for example) sees that block’s documented latency, unchanged by the subsystem layer.

A consequence worth knowing for debug: while PREADY is low, the address and control must be held stable per the APB spec — the crossbar holds the decode stable with them, so a stalled transaction cannot migrate to a different window mid-wait.

## 12.5	Write Strobes and Protection

PSTRB and PPROT are forwarded to the selected block on every transaction. What the block does with them is the block’s own documented behaviour:

- PSTRB — byte lane enables for writes. Blocks that implement byte-granular register writes honour it; blocks with word-only registers document that in their own book. Masters should not assume more than the target block promises.

- PPROT — protection attributes. The RLB blocks accept and ignore it (no protection checking at this level), as each per-block book states. It exists on the boundary because APB4 requires the signals to be carried; a system-level protection scheme would be enforced above rlb_top.

## 12.6	Error Responses

A transaction completes with PSLVERR asserted for exactly two reasons:

- Decode miss. The address matches no window. The generated crossbar’s decode-miss path completes the transaction with PSLVERR rather than holding PREADY low — an unmapped access can never hang the bus. Read data on a miss is undefined; software must treat the transaction as failed. This path is a property of the generator and is the reason the crossbar is regenerated rather than hand-edited.

- Block error. The addressed block asserts its own PSLVERR (for example, an access to a register offset that block does not implement). The block’s book owns the specifics; the boundary simply returns whatever the block raises.

The two are distinguishable in hardware only by address: if the address is inside a decoded window, the error came from the block; if it is outside all ten windows, it came from the decode miss. The func suite proves the decode-miss case completes instead of hanging.

## 12.7	Rules an Integrator Must Honour

- Drive the full system address; the subsystem decodes PADDR[15:12] against BASE_ADDR internally and blocks see only PADDR[11:0].

- Never issue a new transaction to a different window while PREADY is low on the current one — standard APB, restated here because the crossbar’s decode follows the held address.

- Tie PSTRB/PPROT per the APB4 rules if your master does not drive them.

- Expect errors to be fast, never absent: both failure modes complete with PSLVERR in bounded time.

## 12.8	Related Documents

- Top-Level Interface - the port list and the decode table this protocol serves

- The APB Crossbar Block - the generated fabric that implements this boundary

- Window Map - the ten windows the decode resolves to

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	RLB Top - Interrupt Pin Reference

## 13.1	Overview

This page answers the pin-level question: which line does each block’s interrupt land on, and where does each external interrupt pin go? The routing intent — convention versus choice, OR-ed rather than substituted, IRQ2 never driven — is documented in The Interrupt Fabric. This page is the table a firmware or board engineer looks up a single pin in. Nothing here invents behaviour: every row is a signal on rlb_top’s port list, and the port list is in Top-Level Interface.

## 13.2	Block Interrupt Outputs, Pin by Pin

| RTL pin | Fabric line | Master 8259 | Slave 8259 | IOAPIC pin | rlb_irq_out | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| pit_timer_irq[0] | IRQ0 | IR0 | — | 0 | yes | 8254 channel-0 tick |
| pit_timer_irq[2:1] | none | — | — | — | yes | Channels 1-2 leave on their pins and the aggregate only |
| hpet_legacy_irq0 | IRQ0 | IR0 | — | 0 | yes | Only while HPET_CONFIG.legacy_replacement is set; suppresses hpet_timer_irq[0] while set |
| hpet_legacy_irq8 | IRQ8 | — | IR0 | 8 | yes | Only while legacy replacement is set; suppresses hpet_timer_irq[1] while set |
| hpet_timer_irq[1:0] | none | — | — | — | yes | Deliberately not routed — no conventional legacy line; TIMER_INT_ROUTE_CAP reads 0 for the same reason |
| uart_irq | IRQ4 | IR4 | — | 4 | yes | COM1 by convention |
| rtc_alarm_irq | IRQ8 | — | IR0 | 8 | yes | Shares IRQ8 with rtc_second_irq and hpet_legacy_irq8 |
| rtc_second_irq | IRQ8 | — | IR0 | 8 | yes | Periodic interrupt |
| smb_interrupt | IRQ10 | — | IR2 | 10 | yes | Choice — parameter IRQ_SMBUS |
| pm_interrupt | IRQ9 | — | IR1 | 9 | yes | ACPI by convention; PM wake inputs (pm_rtc_alarm, pm_ext_wake_n, GPE banks) feed the PM block, not the fabric |
| gpio_irq | IRQ11 | — | IR3 | 11 | yes | Choice — parameter IRQ_GPIO |
| pic_int_out | none | — | — | — | yes | The master’s delivered output. Fed back to nothing internally — a loop — it appears only in the aggregate |
| ioapic_irq_out_* | n/a | — | — | — | no | A message interface (valid, vector, dest, deliv_mode, dest_mode), not a level line; consumed by the receiver above the subsystem |

Rows saying “IR0” for the slave mean the slave’s interrupt input bit 0, which is IRQ8 in system numbering — the slave covers IRQ8-15. Rows saying “none” under Fabric reach the aggregate and their own pins only.

## 13.3	The Sixteen Legacy Lines As the Controllers See Them

| IRQ | Master IR | Slave IR | IOAPIC pin | Source(s) on the fabric | Fixed or parameter |
| --- | --- | --- | --- | --- | --- |
| 0 | IR0 | — | 0 | pit_timer_irq[0], hpet_legacy_irq0 | convention |
| 1 | IR1 | — | 1 | none | — |
| 2 | — | — | — | never driven — cascade input, forced from the slave’s INT | — |
| 3 | IR3 | — | 3 | none | — |
| 4 | IR4 | — | 4 | uart_irq | convention |
| 5 | IR5 | — | 5 | none | — |
| 6 | IR6 | — | 6 | none | — |
| 7 | IR7 | — | 7 | none | — |
| 8 | — | IR0 | 8 | rtc_alarm_irq, rtc_second_irq, hpet_legacy_irq8 | convention |
| 9 | — | IR1 | 9 | pm_interrupt | convention |
| 10 | — | IR2 | 10 | smb_interrupt | parameter IRQ_SMBUS |
| 11 | — | IR3 | 11 | gpio_irq | parameter IRQ_GPIO |
| 12 | — | IR4 | 12 | none | — |
| 13 | — | IR5 | 13 | none | — |
| 14 | — | IR6 | 14 | none | — |
| 15 | — | IR7 | 15 | none | — |

IOAPIC pins 16-23 exist on the boundary (IOAPIC_NUM_IRQS = 24) and are sourced only by the board, through ioapic_irq_in — the subsystem drives no fabric line above 15.

## 13.4	External Interrupt Input Pins

| Pin | Feeds | Notes |
| --- | --- | --- |
| pic_irq_in[7:0] | Master, OR-ed with boot-interrupt reroute and fabric lines 0-7 | Bit 2 is ignored: masked off before the slave’s INT is forced onto IR2, so driving it reaches nothing |
| pic_irq_in[15:8] | Slave, OR-ed with fabric lines 8-15 | System IRQ8-15 |
| ioapic_irq_in[23:0] | IOAPIC pins, OR-ed with the zero-extended fabric; also feeds u_ioapic_boot_intx | Pins 0-7 can be rerouted onto legacy inputs when the IOAPIC is not delivering them (see below); a reroute onto pin 2 is a dead entry |
| ioapic_eoi_in, ioapic_eoi_vector[7:0] | IOAPIC EOI input | End-of-interrupt from the local APIC side |
| ioapic_irq_out_ready, ioapic_irq_out_retry | Delivery handshake inputs | Tie retry low if the receiver always accepts |

## 13.5	The Aggregate

rlb_irq_out is a pure OR of: hpet_timer_irq, hpet_legacy_irq0, hpet_legacy_irq8, pic_int_out, pit_timer_irq, rtc_alarm_irq, rtc_second_irq, smb_interrupt, pm_interrupt, gpio_irq, uart_irq. It exists for a SoC that wants one interrupt input rather than ten pins; it drives nothing inside the subsystem, which is why including pic_int_out in it is safe. The IOAPIC’s message-style delivery output is not part of it.

## 13.6	Scope Boundaries Restated

Three integration decisions live at this boundary and are choices, not defects:

- hpet_timer_irq is not routed into the fabric (first table, third row).

- The two MSI configuration outputs (cfg_msi_addr, cfg_msi_data) are connected and left open — ioapic_msi_emit is not instantiated.

- Boot-interrupt rerouting exists (u_ioapic_boot_intx) but its pin-to-legacy map defaults to the identity mapping for pins 0-7 and no reroute above that; the pin-2 entry is dead because IRQ2 is the cascade input.

## 13.7	Related Documents

- The Interrupt Fabric - why the table above looks the way it does

- The Cascade Cross-Connect - why IRQ2 is never driven

- Initialization - programming the controllers to receive these lines

- Use Cases - one interrupt routed end to end

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	RLB Top - Initialization and Programming Guide

## 14.1	Scope

This chapter is the subsystem bring-up: the order in which the pieces have to be configured, and the parts that only exist at the integration level - the interrupt controllers, the IOAPIC redirection table, and the cascade. Programming a block’s own function (setting a timer period, configuring a UART’s baud rate) belongs to that block’s specification.

The sequences below are the ones the integration test suite executes, which is why they are known to work rather than merely plausible.

There is also an executable transcription of this chapter: dv/host/rlb_bringup_programs.py runs steps 1-4 against any write32/read32 bus (a board port binds the same program to a UART bridge), and dv/tests/test_rlb_top_bringup.py proves the book sufficient by driving bring-up through that program alone — it never calls the DV helpers this chapter was transcribed from. Edit this chapter, then run that test: it caught two wrong register offsets in this chapter’s first edition (IOWIN, PIC_STATUS), which is exactly the drift it exists to catch.

## 14.2	Address Helper

Every register access in this chapter goes through a window. One helper keeps the rest readable:

#define RLB_BASE      0xFEC00000u
#define RLB_WINDOW    0x1000u

#define WIN_HPET      0
#define WIN_PIC       1      /* master 8259 */
#define WIN_PIT       2
#define WIN_RTC       3
#define WIN_SMBUS     4
#define WIN_PM        5
#define WIN_IOAPIC    6
#define WIN_GPIO      7
#define WIN_UART      8
#define WIN_PIC_SLAVE 9      /* slave 8259 */

static inline volatile uint32_t *rlb_reg(unsigned window, unsigned offset)
{
    return (volatile uint32_t *)(RLB_BASE + window * RLB_WINDOW + offset);
}

## 14.3	Bring-Up Order

| Step | What | Why here |
| --- | --- | --- |
| 1 | Release reset, confirm the subsystem answers | Nothing below is meaningful if decode is broken |
| 2 | Decide the interrupt topology | Determines whether step 3 is the single or cascade sequence |
| 3 | Configure the 8259 (single or cascaded pair) | The controllers must be initialised before any source is enabled |
| 4 | Configure IOAPIC redirection entries | Entries reset masked; an unconfigured entry silently delivers nothing |
| 5 | Enable the peripheral blocks | Only now can a source assert without being lost |
| 6 | Verify | Read back status rather than assuming |

The ordering constraint that actually matters is steps 3 and 4 before step 5. A block enabled before its controller is configured asserts into a controller that is still in its initialisation state, and in edge-triggered mode the result is a latched interrupt nobody is ready to service.

## 14.4	Step 1: Confirm the Subsystem Answers

Each window has a read-safe probe register. Reading all ten is the cheapest possible confirmation that the crossbar and every block are alive.

/* Read-safe probe per window. */
static const struct { unsigned window, offset; const char *name; } probes[] = {
    { WIN_HPET,      0x000, "hpet"      },  /* HPET_ID, read-only identity */
    { WIN_PIC,       0x000, "pic"       },  /* PIC_CONFIG */
    { WIN_PIT,       0x000, "pit"       },  /* PIT_CONFIG */
    { WIN_RTC,       0x000, "rtc"       },  /* RTC_CONFIG */
    { WIN_SMBUS,     0x000, "smbus"     },  /* SMBUS_CONTROL */
    { WIN_PM,        0x000, "pm"        },  /* ACPI_CONTROL */
    { WIN_IOAPIC,    0x000, "ioapic"    },  /* IOREGSEL */
    { WIN_GPIO,      0x000, "gpio"      },  /* GPIO_CONTROL */
    { WIN_UART,      0x020, "uart"      },  /* UART_SCR -- see the warning */
    { WIN_PIC_SLAVE, 0x000, "pic_slave" },  /* PIC_CONFIG on the slave */
};

Do not probe the UART at offset 0x000. That is the receive buffer, and reading it pops a byte off the RX FIFO. Use the scratch register at 0x020. This is a real trap: a “harmless” identity read that destroys received data.

An unmapped address is also a useful probe in the other direction: it must complete with PSLVERR asserted rather than hang. If it hangs, the crossbar is not the generated one.

## 14.5	Step 2: Choose the Interrupt Topology

| Topology | Use when | Configure |
| --- | --- | --- |
| Single 8259 | Nothing you care about is on IRQ8-15 | Master only, SNGL set |
| Cascaded pair | Anything on IRQ8-15 - which includes the RTC, ACPI, SMBus and GPIO | Both controllers, SNGL clear |
| IOAPIC | You want vectored delivery with per-pin destination control | Redirection entries, step 4 |

The fabric feeds all of these simultaneously. Choosing one does not disable the others.

Note which sources land where: IRQ0 and IRQ4 reach the master; IRQ8, IRQ9, IRQ10 and IRQ11 reach the slave. With four of the six sourcing blocks on the slave controller, the single-controller topology is rarely what you want.

## 14.6	Step 3a: Single-Controller Configuration

Master only, in single mode. This is enough to get INT asserting.

#define PIC_CONFIG 0x000
#define PIC_ICW1   0x004
#define PIC_ICW2   0x008
#define PIC_ICW3   0x00C
#define PIC_ICW4   0x010
#define PIC_OCW1   0x014
#define PIC_STATUS 0x028

int pic_init_single(uint8_t vector_base)   /* vector_base e.g. 0x20 */
{
    *rlb_reg(WIN_PIC, PIC_CONFIG) = 0x1;          /* enable, NOT init_mode */
    *rlb_reg(WIN_PIC, PIC_ICW1)   = 0x10 | 0x02 | 0x01;  /* marker | SNGL | IC4 */
    *rlb_reg(WIN_PIC, PIC_ICW2)   = vector_base;
    /* No ICW3: SNGL is set, so the controller does not expect one. */
    *rlb_reg(WIN_PIC, PIC_ICW4)   = 0x01;         /* 8086 mode */
    *rlb_reg(WIN_PIC, PIC_OCW1)   = 0x00;         /* unmask all */

    return (*rlb_reg(WIN_PIC, PIC_STATUS) & 1) != 0;   /* init complete */
}

PIC_CONFIG must set the enable bit without init_mode. Setting init_mode here sends the initialisation state machine back to its idle state once the sequence completes, and the controller ends up unconfigured. This is the single most likely way to get a controller that accepts every write and then does nothing.

ICW1 here is 0x13: the marker bit, SNGL, and IC4. Level-triggered mode (LTIM) is left clear, so the controller is edge-triggered - which matters when clearing an interrupt, because INT latches until acknowledged.

## 14.7	Step 3b: Cascaded-Pair Configuration

Both controllers, as a master and slave pair. Anything arriving on IRQ8-15 requires this.

int pic_init_cascade(uint8_t master_base, uint8_t slave_base)  /* e.g. 0x20, 0x28 */
{
    /* ---- MASTER, window 1 */
    *rlb_reg(WIN_PIC, PIC_CONFIG) = 0x1;
    *rlb_reg(WIN_PIC, PIC_ICW1)   = 0x10 | 0x01;  /* marker | IC4, SNGL CLEAR */
    *rlb_reg(WIN_PIC, PIC_ICW2)   = master_base;
    *rlb_reg(WIN_PIC, PIC_ICW3)   = 0x04;         /* a slave is attached on IR2 */
    *rlb_reg(WIN_PIC, PIC_ICW4)   = 0x01;
    *rlb_reg(WIN_PIC, PIC_OCW1)   = 0x00;         /* unmask all, IR2 included */

    /* ---- SLAVE, window 9 */
    *rlb_reg(WIN_PIC_SLAVE, PIC_CONFIG) = 0x1;
    *rlb_reg(WIN_PIC_SLAVE, PIC_ICW1)   = 0x10 | 0x01;
    *rlb_reg(WIN_PIC_SLAVE, PIC_ICW2)   = slave_base;
    *rlb_reg(WIN_PIC_SLAVE, PIC_ICW3)   = 0x02;   /* I hang off master IR2 */
    *rlb_reg(WIN_PIC_SLAVE, PIC_ICW4)   = 0x01;
    *rlb_reg(WIN_PIC_SLAVE, PIC_OCW1)   = 0x00;

    return (*rlb_reg(WIN_PIC,       PIC_STATUS) & 1)
        && (*rlb_reg(WIN_PIC_SLAVE, PIC_STATUS) & 1);
}

Four things must all be true for a slave interrupt to reach the CPU, and omitting any one of them looks exactly like a broken interrupt fabric:

- The master is in cascade mode - SNGL clear in ICW1

- The master’s ICW3 marks IR2 as carrying a slave (0x04)

- The slave’s ICW3 states which master line it hangs off (0x02)

- Master IR2 is unmasked in OCW1

With SNGL clear, ICW3 is mandatory. The initialisation state machine waits for it. Skipping it leaves the controller stuck mid-sequence.

Note that master_base and slave_base differ by 8 (0x20 and 0x28): the slave’s eight lines get their own vector block.

## 14.8	Step 4: IOAPIC Redirection Entries

The IOAPIC is programmed indirectly: write the register index to IOREGSEL, then the value to IOWIN.

#define IOAPIC_IOREGSEL  0x000
#define IOAPIC_IOWIN     0x004
#define IOAPIC_BOOTINTX  /* see ioapic_mas ch05 for the offset */

static void ioapic_write(unsigned selector, uint32_t value)
{
    *rlb_reg(WIN_IOAPIC, IOAPIC_IOREGSEL) = selector;
    *rlb_reg(WIN_IOAPIC, IOAPIC_IOWIN)    = value;
}

/* Redirection entries are two 32-bit halves per pin. */
static unsigned rte_lo(unsigned irq) { return 0x10 + irq * 2; }
static unsigned rte_hi(unsigned irq) { return 0x11 + irq * 2; }

void ioapic_arm(unsigned irq, uint8_t vector, int masked)
{
    ioapic_write(rte_lo(irq), vector | ((masked ? 1u : 0u) << 16));
    ioapic_write(rte_hi(irq), 0);      /* destination */
}

Redirection entries reset masked. This is the step most often missed, and its failure mode is the worst kind: the source reaches the IOAPIC’s pin, the pin is correct, and no message is ever delivered. That is indistinguishable from a broken route unless you know to check the mask bit.

On vectors: the assignment is yours. The published typical-assignment table suggests 0x20 through 0x2F for the legacy lines. The integration test suite uses 0x40 + irq, which is a test convention, not a hardware property - convenient because the vector then says which line produced it. Nothing in the RTL constrains the choice.

## 14.9	Step 5: Enable the Blocks

Per-block, from that block’s specification. The only integration-level rule is that this comes last.

If you enable the HPET’s legacy-replacement mode, be aware it takes over IRQ0 and IRQ8: HPET timers 0 and 1 then drive hpet_legacy_irq0 and hpet_legacy_irq8 into those lines and are suppressed on hpet_timer_irq. You should silence the 8254 channel 0 and the RTC periodic interrupt to match, or two sources will be competing for one line - legally, since the fabric ORs them, but confusingly.

## 14.10	Step 6: Verify

| Check | How |
| --- | --- |
| Decode is sound | All ten probes read without error; an unmapped address sets PSLVERR |
| Controllers initialised | PIC_STATUS bit 0 set on each configured controller |
| Cascade consistent | Master IR2 follows the slave’s INT. It should rise for an IRQ8-15 source - that is the cascade working, not a fault |
| A route works | Assert a source, confirm pic_int_out, and confirm an IOAPIC delivery with the vector you programmed |
| Anything at all is pending | rlb_irq_out is asserted while any block interrupt is |

## 14.11	Clearing an Interrupt

In edge-triggered mode pic_int_out latches until the controller is acknowledged. Software should acknowledge and issue EOI normally.

The integration test suite instead resets the whole subsystem between phases, because the acknowledge and EOI paths carry their own block-level limitations that would make an integration test fail on somebody else’s known defect. That is a verification convenience, not guidance for firmware. It is mentioned here only because it explains a real consequence: a reset drops all controller and IOAPIC configuration, so anything that resets must redo steps 3 and 4.

## 14.12	Related Documents

- Use Cases - complete routing paths, end to end

- Architecture - what the fabric does with a source

- Window Map - where each block’s registers are specified

- pic_8259_mas, ioapic_mas - the controllers’ own specifications

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	RLB Top - Use Cases

Six worked paths through the subsystem. Each one is exercised by the integration test suite, so each is known to work as described.

## 15.1	Use Case 1: A UART Interrupt Through the Master Controller

The simplest routing path: a source on a master line.

| Stage | Signal |
| --- | --- |
| Source | uart_irq from u_uart |
| Fabric line | IRQ4 (IRQ_UART, convention: COM1) |
| Master input | w_master_pic_irq[4] |
| Output | pic_int_out |

pic_init_cascade(0x20, 0x28);       /* or pic_init_single(0x20) -- IRQ4 is a master line */
uart_enable_interrupts();           /* see uart_16550_mas */
/* uart_irq -> w_fabric_irq[4] -> master IR4 -> pic_int_out */

The vector the CPU sees is master_base + 4, so 0x24 with a base of 0x20.

## 15.2	Use Case 2: A GPIO Interrupt Through the Cascade

A source on a slave line. This is the common case, not the exception - four of the six sourcing blocks land on the slave controller.

| Stage | Signal |
| --- | --- |
| Source | gpio_irq from u_gpio |
| Fabric line | IRQ11 (IRQ_GPIO, a choice, parameterised) |
| Slave input | slave IR3, because 11 - 8 = 3 |
| Slave output | w_spic_int |
| Master input | w_master_pic_irq[2], forced from the slave |
| Output | pic_int_out, returning the slave’s vector |

pic_init_cascade(0x20, 0x28);       /* mandatory: IRQ11 is a slave line */
gpio_enable_interrupt(pin);         /* see gpio_mas */

The vector is slave_base + 3, so 0x2B with a slave base of 0x28. The master returns the slave’s vector in place of its own - that is what the cascade acknowledge path is for.

Master IR2 rising here is correct behaviour, not a spurious cascade interrupt. The invariant to check is that IR2 equals the slave’s INT, never that IR2 stays low.

## 15.3	Use Case 3: The Same Source Through the IOAPIC

The fabric feeds the IOAPIC in parallel with the controllers. No source change is needed - only IOAPIC configuration.

ioapic_arm(11, 0x40 + 11, /* masked = */ 0);   /* unmask IRQ11's entry */
gpio_enable_interrupt(pin);
/* Delivery appears on ioapic_irq_out_valid with vector 0x4B. */

Both paths are live at once: the same GPIO assertion reaches the cascade and produces an IOAPIC delivery. If you want only one, mask the other - mask the IOAPIC entry, or mask the line in the controller’s OCW1.

Remember the mask default. An entry you never programmed is masked, and a masked entry delivers nothing while looking perfectly correct from the source side.

## 15.4	Use Case 4: HPET Legacy Replacement

Replacing the 8254 tick and the RTC periodic interrupt with HPET timers.

| HPET output | Line | Replaces |
| --- | --- | --- |
| hpet_legacy_irq0 | IRQ0 | 8254 channel 0 tick |
| hpet_legacy_irq8 | IRQ8 | RTC periodic interrupt |

hpet_set_legacy_replacement(1);     /* see hpet_mas */
pit_disable_counter(0);             /* avoid two sources on IRQ0 */
rtc_disable_periodic();             /* avoid two sources on IRQ8 */

The HPET core suppresses timers 0 and 1 on hpet_timer_irq while this mode is active, so a single event cannot arrive twice from the HPET itself. Silencing the 8254 and the RTC is still your job: the fabric ORs all three onto those lines, so leaving them enabled is legal and produces interrupts from sources you thought you had replaced.

The general HPET timers are unaffected, and remain unrouted - they reach rlb_irq_out and their own pins only.

## 15.5	Use Case 5: Boot-Interrupt Rerouting

Catching a source whose IOAPIC entry is masked, so it is not lost during boot before the IOAPIC is configured.

ioapic_arm(irq, vector, /* masked = */ 1);     /* IOAPIC will NOT deliver */
ioapic_write(IOAPIC_BOOTINTX, 1);              /* reroute masked pins to the PIC */
/* The source now reaches the legacy controller instead. */

The map is the identity mapping for IOAPIC pins 0-7 onto legacy inputs 0-7, and no reroute above pin 7.

Pin 2 cannot be rerouted. The map still names legacy input 2, but master IR2 is masked off every external source and driven from the slave controller, so a reroute onto pin 2 reaches nothing. If you need pin 2’s source on the legacy path, pick a different legacy input.

Rerouting is disabled at reset, so the master’s interrupt input is exactly pic_irq_in until you perform the second write above.

## 15.6	Use Case 6: One Interrupt Line for the Whole Subsystem

For a system that would rather not model ten interrupt sources.

/* Wire rlb_irq_out to your controller. On assertion, poll to find the source. */
if (rlb_irq_out_asserted()) {
    /* Check each block's status register to identify the source. */
}

rlb_irq_out asserts while any block interrupt is asserted, including pic_int_out itself. It carries no vector and no priority - it is a pure OR, and identifying the source means polling. It drives nothing inside the subsystem, which is precisely why including the controller’s own output in it is safe.

This is also the path that sees the general HPET timers, which no other path does.

## 15.7	Coincident Sources

The fabric is combinational and per-line, so simultaneous sources are handled by construction: each lands on its own line, and lines that share a source are OR-ed. The integration suite exercises two, three and four coincident sources, spanning both controllers - for example the UART on master IR4 while SMBus, ACPI and GPIO assert together under the cascade.

Two practical notes from that testing:

- Some sources are slow to assert. ACPI takes tens of clocks from pm_gpe_events to pm_interrupt, and SMBus considerably longer. Code that expects a source to be asserted a few cycles after its stimulus will sample too early.

- Priority is the controller’s job, not the fabric’s. The fabric presents every asserted line at once; which one the CPU services is decided by the 8259 priority logic, or by the IOAPIC’s delivery configuration.

## 15.8	Related Documents

- Initialization - the bring-up sequences these use cases assume

- Architecture - the fabric, cascade and aggregate

- Window Map

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	RLB Top - Power Management: Sleep and Wake Across Blocks

## 16.1	Overview

Sleep in this subsystem belongs to exactly one block: the PM/ACPI controller on window 5. The other nine blocks do not implement power states, are not clock-gated by the PM block inside rlb_top, and keep running on pclk through a sleep. What the PM block does produce on its output pins — pm_clock_gate_en[31:0] and pm_power_domain_en[7:0] — is a board-level clock-and-power policy that a consuming SoC may honour, ignore, or wire to nothing. This page sequences the part software controls: arming wake sources, entering S1/S3/S5, and reading the subsystem back to S0. Register-level field detail is the PM/ACPI book’s; this page is the order of operations and the subsystem-level facts that change it.

All addresses below are in window 5 (base RLB_BASE + 0x5000); see Chapter 5 and the pm_acpi register chapter for the field tables.

## 16.2	Who Participates

| Block | Role in sleep |
| --- | --- |
| PM/ACPI | Owns it. The sleep FSM, wake latches, and the gate/domain outputs all live here |
| 8259 pair, IOAPIC | Unchanged. Their programming survives sleep — an interrupt controller does not need re-initialization after wake |
| HPET, PIT, RTC, GPIO, UART, SMBus | Ignore sleep inside the subsystem. Their *_clk/*_resetn pins are unused (CDC_ENABLE(0)) and nothing gates their pclk. If a system wants them quiet in S3, the board consumes the PM gate outputs above the subsystem |
| APB fabric | Stays live. The crossbar decodes regardless of power state, which is what lets software read WAKE_STATUS after a wake |

Two consequences follow. First, there is no per-block quiesce sequence inside rlb_top — software does not park the UART or silence the PIT before sleeping; it simply stops caring, because nothing in the subsystem asserts their interrupts into the fabric unless their own event occurs. Second, pm_clock_gate_en / pm_power_domain_en are outputs for a board, not a mechanism this subsystem enforces: in simulation they can be observed and ignored.

## 16.3	Step 1: Bring the PM Block Up

The PM block is one of the ten windows brought up by Initialization step 5 — enable blocks — with one extra gate before it matters: ACPI_CONTROL.acpi_enable (0x000 bit 0) is the SCI_EN-like master switch. While it is 0, pm_interrupt is held low and no GPE event is captured, though PM1_STATUS and WAKE_STATUS still record their events. Set it before arming anything that must interrupt.

## 16.4	Step 2: Arm the Wake Sources

A source wakes the machine only when both halves are programmed:

- Global wake mask — WAKE_ENABLE (0x064): bits for GPE, power button, RTC alarm, and external wake.

- Per-bank enables — for GPE sources, program GPE0_ENABLE / GPE1_ENABLE (0x038-0x03C, 0x09C-0x0A0) and the matching GPE0_WAKE_EN / GPE1_WAKE_EN registers, and choose edge or level per source with GPE0_TRIGGER / GPE1_TRIGGER. The PM/ACPI book’s GPE section owns the interrupt/wake split; the short version is that a source can be armed to wake a sleeping machine without interrupting a running one.

The non-GPE wake inputs are pins, and three of them need board attention, stated here because they are the usual “sleep works but never wakes” causes:

| Wake input | Pin | Board responsibility |
| --- | --- | --- |
| Power button | pm_power_button_n | Drive from the chassis switch; debounce window in BUTTON_TIMING (0x070) |
| Sleep button | pm_sleep_button_n | Optional; mask with PM1_ENABLE.slpbtn_en if absent |
| RTC alarm | pm_rtc_alarm | Not wired to the RTC inside rlb_top — rtc_alarm_irq is an output pin and pm_rtc_alarm is an input pin. Connect them at the board/SoC level if the RTC should wake the system |
| External wake | pm_ext_wake_n | Tie inactive if unused |

## 16.5	Step 3: Enter Sleep

Program PM1_CONTROL (0x010):

- Write sleep_type (bits 2:0): 1 = S1, 3 = S3, 5 = S5. Other values are treated as 0 and the FSM stays in S0.

- Write sleep_enable (bit 3) = 1. This is a one-shot: the bit self-clears, and the core rising-edge-detects it, so one write is one sleep request and a wake that returns the machine to S0 is not undone by the still-programmed sleep_type.

On a board with real power rails, enable the rail sequencer first (PWR_SEQ_CONFIG, 0x07C): seq_enable turns the clock/rail moves into an ordered walk — clocks gated first on the way down, rails 7 to 0; rails 0 to 7 first on the way up, clocks ungated last — with optional per-rail acknowledge and a programmable inter-step delay. Without it the transitions are instant, which is correct in simulation and wrong on a board. A rail that never acknowledges stalls the walk deliberately; PWR_SEQ_STATUS (0x080) is where to look (seq_busy parked on a rail index). There is no timeout, by design.

The hardware then walks the FSM through TRANSITION into the programmed state. What the state means at the outputs:

| State | pm_clock_gate_en | pm_power_domain_en | Leaving it |
| --- | --- | --- | --- |
| S1 | CLOCK_GATE_CTRL & 0x3 — only blocks 0-1 may stay clocked | POWER_DOMAIN_CTRL unchanged | Any armed wake source |
| S3 | 0 — everything gated | POWER_DOMAIN_CTRL & 0x1 — domain 0 always on | Any armed wake source |
| S5 | 0 | POWER_DOMAIN_CTRL & 0x1 | Pulses pm_sys_reset_req: a wake from S5 is a boot, not a resume |

ACPI_CONTROL.current_state (bits 5:4) reports the encoding, not the ACPI number: 0 = S0, 1 = S1, 2 = S5, 3 = S3. It reads 0 while the FSM is in TRANSITION.

## 16.6	Step 4: Wake

An armed source fires while the FSM is in S1 or S3 and the core latches the wake request — the latch outranks the still-programmed sleep_type while the FSM is in TRANSITION, which is why a button press landing during entry does not bounce the machine back into sleep. The FSM returns through TRANSITION to S0, the latch drops, and the walk (if enabled) re-powers rails in reverse order.

Software then:

- Reads WAKE_STATUS (0x060) to identify the source — gpe_wake, pwrbtn_wake, rtc_wake, ext_wake — and writes 1s to clear it.

- Clears the matching status that fed the interrupt: ACPI_STATUS (0x004), and PM1_STATUS (0x014) for the button/RTC paths, and the GPE status registers for a GPE wake. pm_interrupt is a level — the OR of the enabled sticky bits — and deasserts only when every enabled source has been cleared.

- If the sleep was S5, treats the wake as a boot: RESET_STATUS reads differently, and whatever the system wired to pm_sys_reset_req has reset the machine.

One corner the PM book records: a wake event in the exact cycle of the sleep request is not latched as a wake. The sticky status bits and the level interrupt still record it, the machine enters the programmed state, and the next wake event brings it back. Software that must not sleep through such an event checks the status registers after entry.

## 16.7	Step 5: The Interrupt Path for ACPI Events

pm_interrupt leaves the PM block on the fabric’s IRQ9 line — ACPI by convention — and therefore reaches the slave 8259 (IR1) and IOAPIC pin 9 like any other routed source (see Interrupt Pin Reference). The SCI handler is just a legacy interrupt handler: program the controllers during initialization — step 3 for the 8259s, step 4 for the IOAPIC — to deliver IRQ9, and the PM block needs no special routing. GPE events additionally depend on ACPI_INT_ENABLE.gpe_int_enable and the GPE per-source enables before they contribute to the pin.

## 16.8	What Sleep Does Not Disturb

- Interrupt controller programming. The 8259 pair and IOAPIC hold their configuration across sleep; there is no re-init in the wake path.

- The other blocks’ state. UART FIFOs, PIT counters, RTC time, GPIO configuration — all of it survives, because nothing inside the subsystem gates them. Only a board that consumes the PM gate outputs can change that, and then it is the board’s contract, not this book’s.

- The address map. Every window answers the same way after wake as before; the subsystem performs no remap on sleep or wake.

## 16.9	Related Documents

- Initialization - the bring-up these steps assume

- Use Cases - routing an interrupt end to end (the IRQ9 path)

- The Interrupt Fabric - how IRQ9 reaches both controllers

- PM/ACPI register detail - pm_acpi_mas book, chapter 5

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	RLB Top - Address Map

## 17.1	rlb_top Has No Registers

rlb_top implements no registers of its own. Every address it answers belongs to a block it instantiates. This chapter is therefore the window map - how an address is decoded, and which specification documents the registers inside each window.

## 17.2	Decode

| Property | Value |
| --- | --- |
| Base | BASE_ADDR, default 0xFEC0_0000 |
| Window size | 0x1000 (4KB) |
| Windows | 10, indices 0-9 |
| Slave index | PADDR[15:12] |
| Offset within a block | PADDR[11:0] |
| Reserved | none - all ten windows are assigned |
| Unmapped | completes with PSLVERR; does not hang |

The decode is a clean power of two: the window number is PADDR[15:12], and the address a block sees is the low twelve bits. A block’s own register map is therefore documented in offsets from 0x000, and the subsystem address is BASE_ADDR + window * 0x1000 + offset.

## 17.3	Window Map

| Window | Address range | Block | Register map |
| --- | --- | --- | --- |
| 0 | 0xFEC0_0000 - 0xFEC0_0FFF | HPET | hpet_mas ch05_registers/01_register_map.md |
| 1 | 0xFEC0_1000 - 0xFEC0_1FFF | 8259 PIC (master) | pic_8259_mas ch05_registers/01_register_map.md |
| 2 | 0xFEC0_2000 - 0xFEC0_2FFF | 8254 PIT | pit_8254_mas ch05_registers/01_register_map.md |
| 3 | 0xFEC0_3000 - 0xFEC0_3FFF | RTC | rtc_mas ch05_registers/01_register_map.md |
| 4 | 0xFEC0_4000 - 0xFEC0_4FFF | SMBus | smbus_mas ch05_registers/01_register_map.md |
| 5 | 0xFEC0_5000 - 0xFEC0_5FFF | PM/ACPI | pm_acpi_mas ch05_registers/01_register_map.md |
| 6 | 0xFEC0_6000 - 0xFEC0_6FFF | IOAPIC | ioapic_mas ch05_registers/01_register_map.md |
| 7 | 0xFEC0_7000 - 0xFEC0_7FFF | GPIO | gpio_mas ch05_registers/01_register_map.md |
| 8 | 0xFEC0_8000 - 0xFEC0_8FFF | UART 16550 | uart_16550_mas ch05_registers/01_register_map.md |
| 9 | 0xFEC0_9000 - 0xFEC0_9FFF | 8259 PIC (cascade slave) | pic_8259_mas ch05_registers/01_register_map.md |

Windows 1 and 9 are the same block with the same register map, wired as a master and slave pair. The registers are identical; what differs is the cascade configuration written into them. See Initialization.

## 17.4	Read-Safe Probe Registers

For confirming a window is alive without disturbing it.

| Window | Block | Probe offset | Register |
| --- | --- | --- | --- |
| 0 | HPET | 0x000 | HPET_ID, read-only identity |
| 1 | PIC (master) | 0x000 | PIC_CONFIG |
| 2 | PIT | 0x000 | PIT_CONFIG |
| 3 | RTC | 0x000 | RTC_CONFIG |
| 4 | SMBus | 0x000 | SMBUS_CONTROL |
| 5 | PM/ACPI | 0x000 | ACPI_CONTROL |
| 6 | IOAPIC | 0x000 | IOREGSEL |
| 7 | GPIO | 0x000 | GPIO_CONTROL |
| 8 | UART 16550 | 0x020 | UART_SCR scratch - not 0x000 |
| 9 | PIC (slave) | 0x000 | PIC_CONFIG |

The UART is the exception and it matters. Offset 0x000 in the UART window is the receive buffer; reading it pops a byte off the RX FIFO. Probe the scratch register at 0x020 instead. An identity read that quietly consumes received data is a genuinely difficult bug to find.

## 17.5	The IOAPIC Is Indirect

The IOAPIC occupies a 4KB window but exposes only two directly addressed registers. Everything else is reached by writing an index to IOREGSEL and then the data to IOWIN. The redirection table in particular is two 32-bit halves per pin, reached through that indirection - so an IOAPIC “register offset” in the IOAPIC specification is a selector value, not a window offset.

## 17.6	Interrupt Line Assignments

Not registers, but the other half of the programming surface. The line numbers are fixed in hardware for convention assignments and parameterised for choices.

| Line | Source | Basis | Controller input |
| --- | --- | --- | --- |
| IRQ0 | pit_timer_irq[0] OR hpet_legacy_irq0 | convention | master IR0 |
| IRQ2 | cascade only - never driven by the fabric | convention | master IR2, from the slave’s INT |
| IRQ4 | uart_irq | convention | master IR4 |
| IRQ8 | rtc_alarm_irq OR rtc_second_irq OR hpet_legacy_irq8 | convention | slave IR0 |
| IRQ9 | pm_interrupt | convention | slave IR1 |
| IRQ10 | smb_interrupt | choice, parameter IRQ_SMBUS | slave IR2 |
| IRQ11 | gpio_irq | choice, parameter IRQ_GPIO | slave IR3 |

Each line also reaches the IOAPIC pin of the same number. The IOAPIC’s vector for a pin is whatever software programs into that pin’s redirection entry; there is no hardware-fixed vector.

## 17.7	Related Documents

- Top-Level Interface - the decode contract and port list

- Initialization - the address helper and bring-up order

- Architecture - how a line reaches its destinations

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Retro Legacy Blocks - Product Requirements Document

Component: Retro Legacy Blocks (RLB) - Production-Quality Legacy Peripherals Version: 1.0 Status: Active Development - HPET Production Ready Last Updated: 2025-10-29

## 18.1	1. Overview

### 18.1.1	1.1 Purpose

The Retro Legacy Blocks (RLB) component provides production-quality implementations of legacy peripheral blocks based on proven peripheral designs. These blocks are designed to be reusable, well-tested, and suitable for both FPGA and ASIC implementation.

### 18.1.2	1.2 Design Philosophy

“Retro” - Proven Architectures: - Implements time-tested peripheral designs from successful platforms - Focuses on simplicity, reliability, and well-understood behavior - Prioritizes production-readiness over experimental features

“Legacy” - Time-Tested Interfaces: - Based on proven peripheral interface specifications - Suitable for systems requiring retro-compatible peripheral compatibility - APB-based interface for easy integration

“Blocks” - Modular Collection: - Each peripheral is independent and self-contained - Clear separation between different blocks (rtl/hpet/, rtl/gpio/, etc.) - Can be used individually or wrapped into integrated subsystem

### 18.1.3	1.3 Target Applications

- Retro-compatible platform compatibility layers

- Embedded systems requiring legacy peripheral interfaces

- FPGA-based system emulation

- Educational platforms demonstrating classic peripheral designs

- Mixed-vintage SoC integration (modern + legacy interfaces)

## 18.2	2. Implemented Blocks

### 18.2.1	2.1 HPET - High Precision Event Timer

Status: Production Ready (5/6 configurations 100% passing) RTL Location: rtl/hpet/ Documentation: docs/hpet_mas/

Key Features: - Configurable timer count: 2, 3, or 8 independent timers - 64-bit main counter for high-resolution timestamps - 64-bit comparators per timer - Operating modes: One-shot and periodic - Clock domain crossing: Optional CDC for timer/APB clock independence - APB4 interface: Standard AMBA APB protocol - PeakRDL integration: Register map generated from SystemRDL specification

Applications: - System tick generation - Real-time OS scheduling - Precise event timing - Performance profiling - Watchdog timers - Multi-rate timing domains

Test Coverage: - 6 configurations tested (2/3/8 timers, CDC on/off) - 5/6 configurations at 100% pass rate - 1 configuration at 92% (minor stress test timeout) - 12 test cases per configuration (gate/func/full)

Architecture:

┌─────────────────────────────────────────────────────────────┐
│                        APB HPET                              │
├─────────────────────────────────────────────────────────────┤
│                                                               │
│  ┌──────────────┐     ┌──────────────┐    ┌──────────────┐ │
│  │  APB Slave   │────▶│ Config Regs  │───▶│  HPET Core   │ │
│  │  (Optional   │     │ (PeakRDL)    │    │              │ │
│  │   CDC)       │     │              │    │ - Counter    │ │
│  └──────────────┘     └──────────────┘    │ - Timers[N]  │ │
│                                            │ - Comparators│ │
│                                            └──────────────┘ │
│                                                    │         │
│                                            Timer IRQs [N:0]  │
└────────────────────────────────────────────────────────────┘

Design Highlights: - Reset macro standardization (FPGA-friendly) - Per-timer data buses prevent corruption - Edge-triggered register write strobes (not level) - W1C status register for interrupt clearing - Optional asynchronous clock domains with handshake CDC

See: docs/hpet_mas/hpet_mas_index.md for complete HPET specification

## 18.3	3. Planned Blocks

### 18.3.1	3.1 8259 - Programmable Interrupt Controller (PIC)

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: High Address: 0xFEC0_1000 - 0xFEC0_1FFF (4KB window, master of the cascaded pair)

Planned Features: - 8259A-compatible register interface - 8 interrupt request (IRQ) inputs - Cascadable (master/slave configuration) - Priority resolver (fixed and rotating priority) - Edge and level triggered modes - Interrupt mask register - End-of-Interrupt (EOI) handling - APB register interface

Applications: - Legacy interrupt management - PC-compatible systems - Hardware interrupt aggregation - Priority-based interrupt handling - Cascaded multi-level interrupt systems

### 18.3.2	3.2 8254 - Programmable Interval Timer (PIT)

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: High Address: 0xFEC0_2000 - 0xFEC0_2FFF (4KB window)

Planned Features: - 8254-compatible register interface - 3 independent 16-bit counters - 6 programmable counter modes - Binary and BCD counting - Read-back command - Configurable clock input - Interrupt/output generation per counter - APB register interface

Counter Modes: - Mode 0: Interrupt on terminal count - Mode 1: Hardware retriggerable one-shot - Mode 2: Rate generator - Mode 3: Square wave mode - Mode 4: Software triggered strobe - Mode 5: Hardware triggered strobe

Applications: - System tick generation - Periodic timer interrupts - Square wave generation - Event counting - Legacy PC timer compatibility

### 18.3.3	3.3 GPIO - General Purpose I/O

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_7000 - 0xFEC0_7FFF (4KB window)

Planned Features: - Configurable pin count (8, 16, 32 pins) - Per-pin direction control (input/output/bidirectional) - Input debouncing logic - Interrupt generation (rising/falling/both edges, level) - Output drive strength configuration - Pull-up/pull-down control - APB register interface

Applications: - LED control - Button inputs - Hardware control signals - Chip-select generation - Status monitoring

### 18.3.4	3.4 RTC - Real-Time Clock

Status: Implemented (apb4_rtc + MAS; see docs/rtc_mas – binary 24-hour timekeeping validated; BCD/12-hour modes and the production-mode time-set path are broken, tracked in issue #56) Priority: Medium Effort: 3-4 weeks Address: 0xFEC0_3000 - 0xFEC0_3FFF (4KB window)

Planned Features: - 32.768 kHz clock input (typical RTC crystal frequency) - Seconds, minutes, hours, day, month, year tracking - Alarm functionality - Battery backup support (power domain considerations) - 24-hour or 12-hour (AM/PM) mode - Leap year handling - APB register interface

Applications: - System time-of-day tracking - Wake-on-alarm functionality - Timestamp generation - Power-aware applications

### 18.3.5	3.5 SMBus Controller

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_4000 - 0xFEC0_4FFF (4KB window)

Planned Features: - SMBus 2.0 compliance - Master and slave modes - Clock stretching support - Packet Error Checking (PEC) - Alert response address - Configurable clock speed - APB register interface

Applications: - System management bus communication - Sensor interfaces (temperature, voltage) - EEPROM access - Battery management - Fan control

### 18.3.6	3.6 UART - Universal Asynchronous Receiver/Transmitter

Status: Implemented (apb4_uart_16550 + MAS; see docs/uart_16550_mas – note the shipped block has 1/2 stop bits only and no automatic flow control, and issue #60 tracks its RTL defects) Priority: Medium Address: 0xFEC0_8000 - 0xFEC0_8FFF (4KB window)

Planned Features: - 16550-compatible register interface - Configurable baud rate generation - 5/6/7/8 data bits - Parity: none, even, odd, mark, space - Stop bits: 1, 1.5, 2 - Hardware flow control (RTS/CTS) - FIFO buffers (16-byte TX/RX) - Interrupt generation

Applications: - Debug console - Serial communication - Modem interfaces - Legacy peripheral communication

### 18.3.7	3.7 SPI Controller

Status: Planned Priority: Low Effort: 5-6 weeks Address: TBD (not in primary ILB address map)

Planned Features: - Master mode (initially; slave mode future) - Configurable clock polarity and phase (CPOL/CPHA) - Multiple chip selects - Configurable word size (8/16/32 bits) - TX/RX FIFOs - DMA support (future) - APB register interface

Applications: - Flash memory access - ADC/DAC interfaces - Display controllers - SD card communication

### 18.3.8	3.8 I2C Controller

Status: Planned Priority: Low Effort: 5-7 weeks Address: TBD (not in primary ILB address map)

Planned Features: - I2C standard (100 kHz), fast (400 kHz), fast-plus (1 MHz) modes - Multi-master arbitration - 7-bit and 10-bit addressing - Clock stretching - General call support - APB register interface

Applications: - Sensor interfaces - EEPROM access - Multi-chip communication - System configuration

### 18.3.9	3.9 Watchdog Timer

Status: Planned Priority: Low Effort: 2-3 weeks Address: TBD (not in primary ILB address map)

Planned Features: - Configurable timeout period - Countdown counter with reload - Reset generation on timeout - Lock mechanism to prevent accidental disable - Interrupt before reset (optional warning) - APB register interface

Applications: - System fault recovery - Software hang detection - Periodic system reset - Safety-critical applications

### 18.3.10	3.10 Power Management / ACPI Controller

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_5000 - 0xFEC0_5FFF (4KB window)

Planned Features: - Clock gating control per block - Power domain sequencing - Reset generation and distribution - Wake event handling - Sleep/idle mode control - ACPI-compatible registers - APB register interface

Applications: - Low-power system design - Battery-powered devices - Dynamic power management - Thermal management - OS power management interface

### 18.3.11	3.11 IOAPIC - I/O Advanced Programmable Interrupt Controller

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_6000 - 0xFEC0_6FFF (4KB window)

Planned Features: - I/O APIC CSR model (register-based interface) - Multiple interrupt inputs (24+) - Programmable interrupt routing - Edge and level triggered modes - Priority-based arbitration - Interrupt masking per input - APB register interface for configuration

Applications: - Advanced interrupt routing - Multi-processor interrupt distribution - Flexible interrupt mapping - Legacy IRQ redirection - PC-compatible systems

### 18.3.12	3.12 Interconnect ID / Version Registers

Status: Planned – NOT present in the shipped design Priority: Low Effort: 1-2 weeks Address: unallocated. The shipped decoder is the generated apbx_xbar_1to10, whose ten windows (0x0000-0x9FFF) are fully assigned, so adding this block needs a window assignment AND a crossbar regeneration – it cannot simply occupy 0xF000 as earlier drafts of this document assumed.

Planned Features: - Vendor ID register - Device ID register - Revision ID register - Block presence/capability bits - Configuration status registers - Debug/diagnostic registers - APB register interface

Applications: - Software block discovery - Version checking - Feature detection - Debug and diagnostics - Platform identification

## 18.4	4. Integration and Wrapper Goals

### 18.4.1	4.1 Individual Block Integration

Each block is designed to be used standalone:

Example - HPET Integration:

apb4_hpet #(
    .NUM_TIMERS(3),
    .VENDOR_ID(16'h4747),
    .REVISION_ID(16'h0001),
    .CDC_ENABLE(0)
) u_hpet (
    .pclk           (apb_clk),
    .presetn        (apb_rst_n),
    // APB interface -- the ports are s_apb_P*, not bare p*
    .s_apb_PSEL     (psel_hpet),
    .s_apb_PENABLE  (penable),
    .s_apb_PREADY   (pready_hpet),
    .s_apb_PADDR    (paddr[11:0]),
    .s_apb_PWRITE   (pwrite),
    .s_apb_PWDATA   (pwdata),
    .s_apb_PSTRB    (pstrb),
    .s_apb_PPROT    (pprot),
    .s_apb_PRDATA   (prdata_hpet),
    .s_apb_PSLVERR  (pslverr_hpet),
    // HPET-specific
    .hpet_clk       (timer_clk),
    .hpet_resetn    (timer_rst_n),
    .timer_irq    (timer_irq[2:0])
);

### 18.4.2	4.2 RLB Wrapper Architecture

Goal: Create top-level wrapper combining multiple legacy blocks into unified retro-compatible subsystem.

System Architecture:

┌─────────────────────────────────────────┐
                    │         RLB Wrapper                      │
                    │  (Single APB Slave Entry Point)         │
                    └──────────────┬──────────────────────────┘
                                   │
                    ┌──────────────▼──────────────────┐
                    │     APB Decoder/Bridge           │
                    │  (Address decode + routing)      │
                    └──────────────┬──────────────────┘
                                   │
        ┌──────────────────────────┴──────────────────────────┐
        │                                                       │
        ▼                 ▼              ▼              ▼      ▼
    ┌──────┐         ┌──────┐       ┌──────┐      ┌──────┐  ┌──────┐
    │ HPET │         │ 8259 │       │ 8254 │      │ RTC  │  │ ...  │
    │ 4KB  │         │ 4KB  │       │ 4KB  │      │ 4KB  │  │      │
    └──────┘         └──────┘       └──────┘      └──────┘  └──────┘

Address Map:

Base address: 0xFEC0_0000 – the BASE_ADDR parameter default in rlb_top.sv, and the conventional location for this class of block. It is a PARAMETER: an integrator may relocate the whole window set. Window size: 4KB per block (clean power-of-2 decode)

| Address Range | Block | Size | Function |
| --- | --- | --- | --- |
| 0xFEC0_0000 - 0xFEC0_0FFF | HPET | 4KB | High Precision Event Timer |
| 0xFEC0_1000 - 0xFEC0_1FFF | 8259 PIC (master) | 4KB | Programmable Interrupt Controller |
| 0xFEC0_2000 - 0xFEC0_2FFF | 8254 PIT | 4KB | Programmable Interval Timer |
| 0xFEC0_3000 - 0xFEC0_3FFF | RTC | 4KB | Real-Time Clock |
| 0xFEC0_4000 - 0xFEC0_4FFF | SMBus | 4KB | SMBus Host Controller |
| 0xFEC0_5000 - 0xFEC0_5FFF | PM/ACPI | 4KB | Power Management / ACPI Registers |
| 0xFEC0_6000 - 0xFEC0_6FFF | IOAPIC | 4KB | I/O Advanced PIC (CSR model) |
| 0xFEC0_7000 - 0xFEC0_7FFF | GPIO | 4KB | General Purpose I/O |
| 0xFEC0_8000 - 0xFEC0_8FFF | UART 16550 | 4KB | Serial port |
| 0xFEC0_9000 - 0xFEC0_9FFF | 8259 PIC (slave) | 4KB | Cascade slave of the PC/AT pair |
| All other addresses | decode-miss agent | - | Accepted and answered locally with PSLVERR |

The decoder is the GENERATED apbx_xbar_1to10 (one master, ten slaves), so the window count and the base both come from the generator invocation recorded in rlb_top.sv. There is no window above 0x9FFF.

Decoder Implementation:

// Address decode logic (simplified)
localparam BASE_ADDR = 32'hFEC0_0000;
localparam BLOCK_SIZE = 12;  // 4KB = 2^12

logic [3:0] block_sel;
assign block_sel = paddr[15:12];  // Extract window number

always_comb begin
    psel_hpet      = (block_sel == 4'h0) & psel;  // 0xFEC0_0xxx
    psel_pic8259   = (block_sel == 4'h1) & psel;  // 0xFEC0_1xxx
    psel_pit8254   = (block_sel == 4'h2) & psel;  // 0xFEC0_2xxx
    psel_rtc       = (block_sel == 4'h3) & psel;  // 0xFEC0_3xxx
    psel_smbus     = (block_sel == 4'h4) & psel;  // 0xFEC0_4xxx
    psel_pm        = (block_sel == 4'h5) & psel;  // 0xFEC0_5xxx
    psel_ioapic    = (block_sel == 4'h6) & psel;  // 0xFEC0_6xxx
    psel_gpio      = (block_sel == 4'h7) & psel;  // 0xFEC0_7xxx
    psel_uart      = (block_sel == 4'h8) & psel;  // 0xFEC0_8xxx
    psel_pic_slave = (block_sel == 4'h9) & psel;  // 0xFEC0_9xxx
    psel_error     = !(|{psel_hpet, psel_pic8259, psel_pit8254,
                         psel_rtc, psel_smbus, psel_pm, psel_ioapic,
                         psel_gpio, psel_uart, psel_pic_slave}) & psel;
end

Interface: - Single APB slave port at base address 0xFEC0_0000 (the BASE_ADDR parameter) - Aggregated interrupt output combining all block IRQs - Per-block clock/reset control for power management - External I/O signals (GPIO, UART, I2C/SMBus, etc.) - Error slave returns SLVERR for unmapped addresses

Benefits: - Simplified system integration (single APB slave) - Consistent 4KB window addressing - Clean power-of-2 address decode - Ten 4KB windows, fully assigned; expansion needs a crossbar regeneration - Single verification target - Drop-in retro-compatible peripheral subsystem

## 18.5	5. Design Standards

### 18.5.1	5.1 Reset Handling

MANDATORY: All blocks must use standardized reset macros from rtl/amba/includes/reset_defs.svh

Pattern:

`include "reset_defs.svh"

`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
        r_counter <= '0;
    end else begin
        r_state <= w_next_state;
        r_counter <= r_counter + 1'b1;
    end
)

Why: - FPGA-friendly reset inference - Consistent synthesis behavior - Single-point reset polarity control - Better timing closure

### 18.5.2	5.2 Register Generation

Preferred: Use PeakRDL for register map generation

Process: 1. Define registers in SystemRDL (.rdl file) 2. Generate RTL using PeakRDL regblock 3. Create wrapper module connecting registers to core logic 4. Use edge detection for write strobes (not level)

Benefits: - Consistent register interface - Auto-generated documentation - Reduced manual RTL errors - Easy register map changes

### 18.5.3	5.3 Testbench Architecture

MANDATORY: Follow project testbench organization pattern

Structure:

dv/
├── tbclasses/{block}/          # Block-specific TB classes
│   ├── {block}_tb.py          # Main testbench
│   ├── {block}_tests_basic.py # Gate test suite
│   ├── {block}_tests_medium.py # Func test suite
│   └── {block}_tests_full.py  # Full test suite
└── tests/{block}/             # Test runners
    ├── test_apb4_{block}.py   # Pytest wrapper
    └── conftest.py            # Pytest configuration

Import Pattern:

# Always import from PROJECT AREA
from projects.components.retro_legacy_blocks.dv.tbclasses.{block}.{block}_tb import {Block}TB

Test Levels: - Gate: Core functionality (register access, basic operation) - Func: Extended features (modes, configurations, edge cases) - Full: Stress testing, CDC variants, corner cases

Target: 100% pass rate at all levels

### 18.5.4	5.4 FPGA Synthesis Attributes

MANDATORY: Add FPGA synthesis hints for memory arrays

`ifdef XILINX
    (* ram_style = "auto" *)
`elsif SYNTH_PRAGMA
    /* synthesis ramstyle = "AUTO" */
`endif
logic [DATA_WIDTH-1:0] mem [DEPTH];

### 18.5.5	5.5 Documentation Requirements

Each block must have: - RTL comments (inline) - Register map specification - Block-level specification in docs/{block}_mas/ - Integration guide - Test plan and results

## 18.6	6. Quality Metrics

### 18.6.1	6.1 Production Readiness Criteria

A block is considered “Production Ready” when:

- All gate tests pass 100%

- All func tests pass 100%

- All full tests pass ≥95%

- Complete register map specification

- RTL lint clean (Verilator)

- Reset macros used throughout

- FPGA synthesis attributes applied

- Integration guide written

- Known issues documented

### 18.6.2	6.2 Current Status

| Block | Priority | Status | Test Pass Rate | Documentation | Production Ready |
| --- | --- | --- | --- | --- | --- |
| HPET | High | Implemented | see dv/tests | MAS (ch1/2/5) | RTL defects open (#46) |
| 8259 PIC | High | Implemented | see dv/tests | MAS (ch1/5) | ISR/EOI half inert (#50) |
| 8254 PIT | High | Implemented | 6/6 both CDC configs | MAS (full) | Mode 0 only (#52) |
| GPIO | Medium | Implemented | see dv/tests | MAS (full) | Functional; quirks tracked (#44) |
| RTC | Medium | Implemented | see dv/tests | MAS (ch1/5) | Binary 24h only; time-set broken in production mode (#56) |
| SMBus | Medium | Implemented | see dv/tests | MAS (ch1/5) | Transfer engine non-functional (#58) |
| PM/ACPI | Medium | Implemented | see dv/tests | MAS (ch1/5) | Status/W1C/wake defects (#54) |
| IOAPIC | Medium | Implemented | see dv/tests | MAS (ch1/2/5) | Double delivery + EOI hazards (#48) |
| UART | Medium | Implemented | see dv/tests | MAS (full) | FE/BI dead, IER unwired (#60) |
| SPI | Low | Planned | N/A | N/A | No |
| I2C | Low | Planned | N/A | N/A | No |
| Watchdog | Low | Planned | N/A | N/A | No |
| Interconnect | Low | Planned | N/A | N/A | No |

## 18.7	7. Development Roadmap

### 18.7.1	7.1 Phase 1: Foundation (Complete)

- HPET implementation

- Directory structure for multiple blocks

- Testbench architecture established

- Documentation templates

- Build and test infrastructure

### 18.7.2	7.2 Phase 2: Core Peripherals (Next 6-9 Months)

Q1 2026 (High Priority): - 8259 PIC (6-8 weeks) - Interrupt controller - 8254 PIT (4-5 weeks) - Interval timer - RTC (3-4 weeks) - Real-time clock

Q2 2026 (Medium Priority): - GPIO Controller (4-6 weeks) - SMBus Controller (6-8 weeks) - PM/ACPI Controller (8-10 weeks)

Q3 2026: - UART (4-5 weeks) - IOAPIC (6-8 weeks)

### 18.7.3	7.3 Phase 3: Advanced Peripherals (9-15 Months)

Q4 2026: - SPI Controller (5-6 weeks) - I2C Controller (5-7 weeks) - Watchdog Timer (2-3 weeks)

Q1 2027: - Interconnect ID/Version Registers (1-2 weeks) - ILB Wrapper integration starts

### 18.7.4	7.4 Phase 4: System Integration (15+ Months)

Q2-Q4 2027: - Complete ILB wrapper with all blocks - System-level integration examples - Performance characterization - FPGA reference designs - Application notes - Software driver examples

## 18.8	8. References

### 18.8.1	8.1 External Standards

Peripheral Specifications: - ACPI HPET Specification 1.0a - SMBus Specification Version 2.0 - 16550 UART Datasheet - I2C Specification (NXP) - SPI Protocol Specification

Bus Protocols: - AMBA APB Protocol Specification (ARM) - AMBA 3 APB Protocol v1.0

### 18.8.2	8.2 Internal Documentation

- /CLAUDE.md - Repository AI guide

- /PRD.md - Master repository requirements

- projects/components/legacy-ip/retro_legacy_blocks/CLAUDE.md - Component AI guide

- projects/components/legacy-ip/retro_legacy_blocks/README.md - Component overview

- vault/Tasks/RLB/ - task tracking (cross-block); vault/Tasks/RLB/hpet/ for HPET

### 18.8.3	8.3 Block-Specific Documentation

HPET: - docs/hpet_mas/hpet_mas_index.md - HPET specification - docs/IMPLEMENTATION_STATUS.md - HPET test results - known_issues/ - HPET issue tracking

## 18.9	9. Success Criteria

### 18.9.1	9.1 Individual Block Success

Each block must: - Pass all gate/func tests at 100% - Pass full tests at ≥95% - Have complete register map specification - Include integration guide with examples - Be lint-clean (Verilator) - Use reset macros throughout - Include FPGA synthesis attributes

### 18.9.2	9.2 Collection Success

The retro_legacy_blocks component is successful when: - At least 6 blocks production-ready (HPET + 5 high/medium priority blocks) - All blocks follow consistent architecture (reset macros, PeakRDL, APB interface) - RLB wrapper integrates all blocks seamlessly with clean 4KB addressing - System-level integration example provided - Complete documentation for all blocks - FPGA reference design available - Address map covers all essential retro-compatible peripherals

### 18.9.3	9.3 Long-Term Vision

Ultimate goal: - Production-quality retro-compatible peripheral subsystem - Complete peripheral coverage for legacy platform requirements - Used in production FPGA designs - Educational resource for classic peripheral design - Foundation for mixed-vintage SoC designs

Version: 1.0 Last Review: 2025-10-29 Next Review: After each new block completion Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	RLB (Retro Legacy Blocks) - FPGA Implementation Guide

Date: 2025-11-16 Purpose: Explain RLB blocks and FPGA implementation compatibility

## 19.1	What Are RLB (Retro Legacy Blocks)?

### 19.1.1	Overview

Retro Legacy Blocks are modern implementations of classic PC/embedded system peripherals, designed as reusable IP cores with APB interfaces. They provide functionality compatible with legacy systems but implemented in clean, modern SystemVerilog.

### 19.1.2	Purpose

Educational: - Learn how classic PC peripherals work internally - Understand interrupt controllers, timers, power management - Study register-based peripheral design - Practice verification of complex protocols

Practical: - Build soft-core SoC systems on FPGAs - Create retro computer replicas (PC-compatible) - Implement embedded systems with familiar peripherals - Rapid prototyping of control systems

Industrial: - Reusable IP cores for custom SoCs - Known-good peripheral implementations - Well-documented, tested components - Modern coding standards with legacy compatibility

## 19.2	Implemented RLB Modules

### 19.2.1	Complete Modules

#### 19.2.1.1	1. HPET (High Precision Event Timer)

- Purpose: High-resolution timer (sub-microsecond precision)

- Features: Multiple timers, one-shot/periodic modes, 64-bit counters

- PC Equivalent: Modern replacement for PIT

- Use Cases: Precise timing, event scheduling, benchmarking

#### 19.2.1.2	2. PIT_8254 (Programmable Interval Timer)

- Purpose: Classic PC timer (3 channels)

- Features: Mode 0-5 operation, binary/BCD counting

- PC Equivalent: 8254 PIT

- Use Cases: System tick, speaker control, legacy PC compatibility

#### 19.2.1.3	3. RTC (Real-Time Clock)

- Purpose: Calendar and alarm functions

- Features: Time/date tracking, alarm, periodic interrupts

- PC Equivalent: MC146818 / DS12887

- Use Cases: Wall-clock time, wake-on-alarm, timestamping

#### 19.2.1.4	4. PIC_8259 (Programmable Interrupt Controller)

- Purpose: Legacy interrupt management

- Features: 8 IRQ inputs, priority, cascading, masking

- PC Equivalent: 8259A PIC

- Use Cases: Legacy PC interrupts, simple systems

#### 19.2.1.5	5. SMBus (System Management Bus)

- Purpose: Low-speed system management communication

- Features: I2C/SMBus 2.0, master/slave, PEC, FIFOs

- PC Equivalent: Modern motherboard SMBus controller

- Use Cases: Sensor monitoring, battery management, system config

#### 19.2.1.6	6. PM_ACPI (Power Management ACPI)

- Purpose: Advanced power management

- Features: PM timer, power states (S0/S1/S3), GPE, clock gating, power domains

- PC Equivalent: ACPI PM controller

- Use Cases: Low-power systems, battery devices, sleep/wake, clock control

#### 19.2.1.7	7. IOAPIC (I/O Advanced Programmable Interrupt Controller)

- Purpose: Advanced interrupt routing

- Features: 24 IRQs, redirection table, edge/level, priority, EOI

- PC Equivalent: 82093AA I/O APIC

- Use Cases: Multi-processor systems, flexible IRQ routing, modern PCs

## 19.3	FPGA Implementation - Nexys A7 / Genesys2

### 19.3.1	Can RLB Be Implemented on These Boards? YES!

All RLB modules are 100% compatible with Nexys A7 and Genesys2 FPGAs.

### 19.3.2	Target FPGA Specifications

#### 19.3.2.1	Nexys A7 (Artix-7)

- FPGA: XC7A100T-1CSG324C (or XC7A50T)

- Logic Cells: 101,440 (A100T) or 52,160 (A50T)

- Block RAM: 4,860 Kb

- Clock: 100 MHz oscillator

- I/O: Extensive (PMOD, switches, LEDs, UART, etc.)

- Verdict: Excellent fit for full RLB system

#### 19.3.2.2	Genesys 2 (Kintex-7)

- FPGA: XC7K325T-2FFG900C

- Logic Cells: 326,080

- Block RAM: 16,020 Kb

- Clock: 200 MHz oscillator

- I/O: Very extensive

- Verdict: More than sufficient, ideal for complex SoC

### 19.3.3	Resource Usage Estimates

Per Module (Approximate):

| Module | LUTs | FFs | BRAM | Notes |
| --- | --- | --- | --- | --- |
| HPET | ~800-1200 | ~600-900 | 0 | Multi-timer, 64-bit counters |
| PIT_8254 | ~400-600 | ~300-500 | 0 | 3 channels, modes |
| RTC | ~300-500 | ~250-400 | 0 | Time/date logic |
| PIC_8259 | ~200-400 | ~150-300 | 0 | Simple priority logic |
| SMBus | ~600-900 | ~500-700 | 0.5 | FIFOs, protocol FSM |
| PM_ACPI | ~500-800 | ~400-600 | 0 | Power FSM, GPE |
| IOAPIC | ~800-1200 | ~600-900 | 0 | 24 IRQs, redirection |

Total RLB System (~5500 LUTs, ~4300 FFs): - Nexys A7-100T: ~5% logic utilization Plenty of room - Nexys A7-50T: ~11% logic utilization Still comfortable - Genesys 2: ~2% logic utilization Massive headroom

### 19.3.4	What Can Fit on These Boards?

Nexys A7-100T Can Support: - All 7 RLB modules - MicroBlaze or RISC-V soft processor - DDR2 controller - Ethernet MAC - VGA/HDMI display controller - USB interface - Audio codec - Result: Complete retro-PC or modern embedded system

Genesys 2 Can Support: - Everything above PLUS: - Multiple soft processors - PCIe endpoint - High-speed interfaces - Complex video processing - Result: Professional-grade SoC development platform

## 19.4	RLB System Integration on FPGA

### 19.4.1	Typical FPGA System Architecture

┌─────────────────────────────────────────────────────────┐
│ FPGA Top Level (Nexys A7 or Genesys 2)                 │
│                                                          │
│  ┌──────────────┐          ┌──────────────┐            │
│  │ Soft CPU     │          │ DDR2/3       │            │
│  │ (MicroBlaze/ │◄────────►│ Controller   │◄───► DRAM │
│  │  RISC-V)     │   AXI    │              │            │
│  └──────┬───────┘          └──────────────┘            │
│         │ APB                                            │
│         ▼                                                │
│  ┌──────────────────────────────────────┐              │
│  │ APB Crossbar (apbx_xbar)              │              │
│  └──┬───┬───┬───┬───┬───┬───┬──────────┘              │
│     │   │   │   │   │   │   │                           │
│   ┌─▼─┐ │   │   │   │   │   │   [More peripherals]     │
│   │HPT│ │   │   │   │   │   │                           │
│   │ ET│ │   │   │   │   │   │                           │
│   └─┬─┘ │   │   │   │   │   │                           │
│     │IRQ│   │   │   │   │   │                           │
│  ┌──▼───▼───▼───▼───▼───▼───▼──────┐                   │
│  │ IOAPIC or PIC_8259               │                   │
│  │ (Interrupt Aggregation)          │                   │
│  └──────────┬────────────────────────┘                   │
│            IRQ                                            │
│             └──────────► CPU Interrupt                    │
│                                                           │
│  External Connections:                                    │
│  ├─ GPIO ◄───► PMOD ports, switches, LEDs               │
│  ├─ UART ◄───► USB-UART bridge                          │
│  ├─ SMBus ◄──► I2C sensors, EEPROMs                     │
│  └─ PM_ACPI ◄► Power management signals                  │
└───────────────────────────────────────────────────────────┘

### 19.4.2	Example Use Cases on FPGAs

#### 19.4.2.1	1. Retro PC Replica

Build a PC-compatible system on FPGA: - Soft x86 core (or compatible RISC-V with x86 emulation) - All RLB peripherals for PC compatibility - VGA output for display - PS/2 keyboard/mouse - Run DOS, early Windows, Linux

#### 19.4.2.2	2. Embedded Control System

Industrial/automotive control: - RISC-V processor - HPET for precise timing - GPIO for sensors/actuators - SMBus for temperature/voltage monitoring - PM_ACPI for power optimization - UART for debug/communication

#### 19.4.2.3	3. Education Platform

Learn computer architecture: - Study peripheral internals - Practice interrupt handling - Understand power management - Experiment with different configurations - Debug with real hardware

#### 19.4.2.4	4. SoC Prototyping

Rapid prototyping: - Test peripheral configurations - Validate interrupt routing - Optimize power consumption - Benchmark performance - Hardware/software co-development

## 19.5	Nexys A7 Specific Integration

### 19.5.1	Available Resources on Nexys A7

Peripherals Already on Board: - 16 switches (GPIO inputs) - 16 LEDs (GPIO outputs) - 5 push buttons - 7-segment displays (4 digits) - USB-UART bridge - PMOD connectors - VGA output - Microphone - Speaker - Temperature sensor - Accelerometer

### 19.5.2	How RLB Modules Map to Nexys A7

| RLB Module | Nexys A7 Resource | Connection |
| --- | --- | --- |
| GPIO | Switches, LEDs, Buttons | Direct mapping |
| UART_16550 | USB-UART bridge | FT2232HQ chip |
| HPET | Internal timing | No external pins needed |
| PIT_8254 | Internal timing | Timer/speaker |
| RTC | Internal clock | Optional ext crystal |
| PIC/IOAPIC | Internal routing | IRQ aggregation |
| SMBus | I2C PMOD | Via I2C PMOD adapter |
| PM_ACPI | Power control | Clock gates, power logic |

### 19.5.3	Example Nexys A7 Top-Level

module nexys_a7_rlb_system (
    // Clock and reset
    input  wire         clk_100mhz,    // 100 MHz oscillator
    input  wire         cpu_resetn,    // Reset button

    // GPIO
    input  wire [15:0]  sw,            // Switches → GPIO inputs
    output wire [15:0]  led,           // LEDs ← GPIO outputs
    input  wire [4:0]   btn,           // Buttons → GPIO/interrupts

    // UART
    input  wire         uart_rxd,      // USB-UART RX
    output wire         uart_txd,      // USB-UART TX

    // I2C/SMBus (via PMOD)
    inout  wire         smbus_sda,
    inout  wire         smbus_scl,

    // 7-segment display
    output wire [6:0]   seg,
    output wire [7:0]   an,

    // VGA (optional)
    output wire [3:0]   vga_r, vga_g, vga_b,
    output wire         vga_hs, vga_vs
);

    // Instantiate soft processor (MicroBlaze, RISC-V, etc.)
    // Instantiate APB crossbar
    // Instantiate all RLB modules
    // Connect to board resources

endmodule

## 19.6	Implementation Recommendations

### 19.6.1	For Nexys A7-100T

Recommended RLB Configuration:

HPET        - System timing
PIT_8254    - Legacy timer for compatibility
RTC         - Real-time clock
PIC_8259    - Simple interrupt controller (OR IOAPIC)
GPIO        - Board I/O (switches, LEDs, buttons)
UART_16550  - Serial communication
SMBus       - I2C peripherals
PM_ACPI     - Optional (if power features needed)

Resource Budget: - RLB modules: ~6,000 LUTs, ~5,000 FFs - MicroBlaze: ~2,000-3,000 LUTs - DDR2 Controller: ~2,000 LUTs - VGA/Display: ~1,000 LUTs - Total: ~11,000-12,000 LUTs (11% of XC7A100T)

Plenty of room for: - User applications - Additional peripherals - Debug logic - Custom IP cores

### 19.6.2	For Genesys 2

Can Support Everything: - All RLB modules simultaneously - Multiple soft processors - Complex memory hierarchy - High-speed peripherals - Video processing - Network interfaces

Recommended for: - Professional SoC development - Multi-core systems - High-performance applications - Complex retro systems (full PC replica)

## 19.7	FPGA-Specific Considerations

### 19.7.1	Clock Management

Nexys A7: - 100 MHz main clock → Can generate all needed frequencies - Use MMCM/PLL for: - CPU clock (50-200 MHz) - Peripheral clocks (25-100 MHz) - Display clocks (25.175 MHz for VGA) - USB clock (60 MHz)

RLB Modules: Most work at system clock (50-100 MHz), some support CDC for different clocks

### 19.7.2	Power Management

PM_ACPI Use on FPGA: - Clock gating controls → Xilin x clock enables - Power domains → Can’t control FPGA power, but can simulate - Useful for: Power-aware designs, testing, learning ACPI - Can control external power via GPIO pins

### 19.7.3	Interrupt Routing

IOAPIC vs PIC_8259 Choice: - Simple systems (single CPU): Use PIC_8259 (smaller) - Advanced systems: Use IOAPIC (more flexible, PC-compatible) - Maximum flexibility: Include both, use jumpers/config

### 19.7.4	External Interfaces

How to Connect RLB to Board Resources:

- GPIO Module:

  - Inputs: Switches, buttons

  - Outputs: LEDs, 7-segment displays

  - Bidirectional: PMOD I/O pins

- UART_16550:

  - Connect to FT2232HQ USB-UART

  - Baud rate generator uses system clock

  - CTS/RTS for flow control (optional)

- SMBus:

  - Connect to I2C PMOD

  - On-board: Temperature sensor (ADT7420)

  - On-board: Accelerometer (ADXL362)

  - External: I2C EEPROMs, sensors via PMOD

- Timers (HPET/PIT):

  - Internal only, no external pins

  - Can drive speaker output

  - Can generate PWM via GPIO

- RTC:

  - Internal timekeeping

  - Optional: External 32.768 kHz crystal

  - Battery backup simulation

## 19.8	Soft Processor Options

### 19.8.1	MicroBlaze (Xilinx)

- Pros: Well-supported, debugger, IP integration

- Cons: Proprietary, Vivado-specific

- RLB Compatibility: Excellent (APB via AXI interconnect)

### 19.8.2	RISC-V (Open Source)

- Options: VexRiscv, PicoRV32, Rocket, BOOM

- Pros: Open-source, portable, modern ISA

- Cons: Need to build toolchain

- RLB Compatibility: Excellent (APB native or via AXI)

### 19.8.3	Custom Soft Core

- Can design custom CPU specifically for RLB peripherals

- Full control over instruction set

- Educational value

## 19.9	Step-by-Step FPGA Implementation

### 19.9.1	Phase 1: Basic System (Week 1-2)

- Create FPGA project (Vivado for Xilinx)

- Add simple processor (MicroBlaze or RISC-V)

- Add APB crossbar components/fabric-gen-ip/lattice-apbx-xbar/

- Add one RLB module (start with GPIO)

- Connect to LEDs/switches

- Test basic register access

### 19.9.2	Phase 2: Expand Peripherals (Week 3-4)

- Add UART_16550 → Test serial communication

- Add HPET → Test precise timing

- Add PIC_8259 or IOAPIC → Test interrupts

- Add RTC → Test time keeping

### 19.9.3	Phase 3: Advanced Features (Week 5-6)

- Add SMBus → Test I2C communication

- Add PM_ACPI → Test power management

- Add all remaining RLB modules

- System integration testing

### 19.9.4	Phase 4: Application Development (Week 7+)

- Develop firmware using RLB peripherals

- Test real-world applications

- Optimize and debug

- Hardware/software co-verification

## 19.10	Example Address Map for FPGA

Base Address | Module      | Size | Description
-------------|-------------|------|------------------------
0xFEC0_0000  | HPET        | 4KB  | High Precision Timer
0xFEC0_1000  | PIC_8259    | 4KB  | Interrupt Controller (master)
0xFEC0_2000  | PIT_8254    | 4KB  | Programmable Interval Timer
0xFEC0_3000  | RTC         | 4KB  | Real-Time Clock
0xFEC0_4000  | SMBus       | 4KB  | System Management Bus
0xFEC0_5000  | PM_ACPI     | 4KB  | Power Management
0xFEC0_6000  | IOAPIC      | 4KB  | I/O APIC
0xFEC0_7000  | GPIO        | 4KB  | General Purpose I/O
0xFEC0_8000  | UART_16550  | 4KB  | Serial Port
0xFEC0_9000  | PIC_8259    | 4KB  | Interrupt Controller (cascade slave)

All accessible via APB from soft processor. The base is the BASE_ADDR parameter of rlb_top (default 0xFEC00000); the window order is the generated apbx_xbar_1to10’s and is not free to rearrange.

## 19.11	Software Development

### 19.11.1	Bare-Metal Firmware

Using RLB Modules in C:

#include "rlb_peripherals.h"

// Initialize system
void system_init(void) {
    // Configure GPIO
    gpio_set_direction(0xFFFF0000);  // Upper 16 = outputs

    // Initialize UART
    uart_init(115200);
    uart_puts("System starting...\n");

    // Configure HPET timer
    hpet_init();
    hpet_start_timer(0, 1000000);  // 1ms periodic

    // Setup interrupts via IOAPIC
    ioapic_configure_irq(14, 0x2E, IRQ_EDGE, 0);  // IDE
    ioapic_configure_irq(15, 0x2F, IRQ_EDGE, 0);  // IDE

    // Enable power management
    pm_acpi_init();
}

### 19.11.2	Operating System Support

Can Run: - Bare-metal: Direct hardware access - RTOS: FreeRTOS, Zephyr with RLB drivers - Linux: With proper device drivers for RLB - DOS/DOS-compatible: If using x86-compatible core - Custom OS: Educational OS development

## 19.12	Advantages of RLB on FPGA

### 19.12.1	Development Benefits

- Rapid Prototyping: Change peripheral config instantly (no PCB)

- Debugging: ChipScope/ILA for internal signals

- Flexibility: Easy to add/remove peripherals

- Cost-Effective: One board, infinite configurations

- Educational: See everything, change everything

### 19.12.2	Technical Benefits

- Known-Good IP: Well-documented, tested peripherals

- Standard Interfaces: APB everywhere, easy integration

- Scalable: Start simple, add complexity

- Portable: Pure SystemVerilog, vendor-independent

- Modern Design: Clean code, proper methodology

### 19.12.3	Vs. Microcontroller

Why FPGA + RLB instead of MCU?

| Feature | MCU | FPGA + RLB |
| --- | --- | --- |
| Peripherals | Fixed | Infinite flexibility |
| Interrupts | Fixed routing | Programmable (IOAPIC) |
| Timers | Limited count | As many as needed |
| Clock domains | 1-2 | Unlimited with CDC |
| Custom logic | Limited | Full flexibility |
| Learning value | Black box | Full visibility |
| Cost | Lower | Higher but more capable |

## 19.13	Conclusion

### 19.13.1	Can RLB Be Used on Nexys A7/Genesys2? ABSOLUTELY!

Perfect Match Because: - Plenty of logic resources - Rich I/O for peripheral connections - Clock resources for multi-domain design - Development boards with extensive support - Educational and professional use

Recommended Starting Point: 1. Nexys A7-100T for most projects 2. Genesys 2 for complex/multi-core systems 3. Start with GPIO + UART + HPET 4. Add peripherals incrementally 5. Full RLB system fits comfortably with room to spare

The RLB modules are specifically designed to be FPGA-friendly and work excellently on these development boards!

Last Updated: 2025-11-16 Recommendation: Nexys A7-100T is ideal for complete RLB-based SoC development