RTL Design Sherpa

APB Crossbar Micro-Architecture Specification 1.0

January 3, 2026

Table of Contents

Apb Xbar Mas Index	16

APB Crossbar Micro-Architecture Specification Index	16

Document Organization	16

Main Documentation	16

Chapter 1: Architecture	16

Chapter 2: Address Decode and Arbitration	16

Chapter 3: RTL Generator	16

Quick Navigation	17

For New Users	17

For Integration	17

Common Questions	17

Visual Assets	17

Architecture Diagrams	17

Timing Diagrams	18

Component Overview	18

Key Features	18

Pre-Generated Variants	18

Design Philosophy	18

Related Documentation	19

Companion Specifications	19

Project-Level	19

Test Infrastructure	19

RTL	19

Version History	19

APB Crossbar Visual Documentation	20

Overview	20

Documentation Structure	20

Main Documentation Files	20

Supporting Documentation	20

Visual Assets	21

Architecture Diagrams	21

Timing Diagrams	21

Quick Start	21

For New Users	21

For Existing Users	22

Directory Structure	22

Key Concepts	23

1. Architecture	23

2. Address Decode	23

3. Arbitration	23

4. RTL Generation	23

Common Questions	24

Q: Which pre-generated variant should I use?	24

Q: How do I change the base address?	24

Q: Can I change per-slave address sizes?	24

Q: How does arbitration work with 3+ masters?	24

Q: What’s the throughput?	25

Regenerating Diagrams	25

Graphviz Diagrams (Architecture, Address Decode)	25

WaveJSON Diagrams (Timing)	25

Version History	25

Maintainers	26

APB Crossbar Architecture	26

Overview	26

Architecture Diagram	26

Figure 1.1: APB Crossbar Top-Level Architecture	27

Functional Blocks	27

1. Master-Side Protocol Conversion	27

2. Internal Crossbar Logic	28

3. Slave-Side Protocol Conversion	28

Signal Flow Example	28

Parameter Configuration	29

Pre-Generated Variants	30

Design Philosophy	30

Next Steps	30

Address Decode and Arbitration	31

Overview	31

Address Decode	31

Address Map Structure	31

Decode Algorithm	32

Address Decode Flow Diagram	32

Figure 2.1: Address Decode Flow	33

Decode Example Walkthrough	33

Multiple Address Maps	34

Arbitration	35

Per-Slave Round-Robin	35

Round-Robin Timing Diagram	35

Waveform 2.1: Round-Robin Arbitration Timing	35

Arbitration Example Walkthrough	35

Multi-Slave Parallelism	36

Grant Persistence	37

Arbitration Latency	37

Integration Examples	37

Example 1: CPU to 4 Peripherals (No Contention)	37

Example 2: CPU + DMA to 4 Peripherals (Potential Contention)	38

Performance Characteristics	38

Throughput	38

Latency	38

Design Notes	39

Why 64KB Per Slave?	39

Why Round-Robin?	39

Troubleshooting	39

Issue: Wrong Slave Selected	39

Issue: Master Starved	39

Issue: Back-to-Back Transactions Stalling	40

Next Steps	40

APB Crossbar RTL Generator	40

Overview	40

Generator Architecture	41

Two-Level Structure	41

Quick Start	41

Generate Standard Variants (1:1, 2:1, 1:4, 2:4)	41

Generate Custom Variant	41

Generate with Custom Base Address	42

Command-Line Interface	42

Full Options	42

Examples	42

Generated Code Structure	43

Template Components	43

Generator Algorithm	46

Step-by-Step Process	46

Code Generation Patterns	47

Loop-Based Generation	47

String Formatting	48

Customization Examples	48

Example 1: Large SoC with 10 Peripherals	48

Example 2: Multi-CPU System	48

Example 3: Memory-Mapped I/O Region	49

Using Generated Modules	49

Integration Example	49

Parameter Override	50

Generator Limitations and Future Work	50

Current Limitations	50

Future Enhancements	51

Testing Generated Code	52

Verification Flow	52

Recommended Test Cases	52

Advanced Usage	52

Using as Python Library	52

Batch Generation	53

Troubleshooting	53

Issue: “ModuleNotFoundError: No module named ‘amba.apb_xbar_generator’”	53

Issue: Generated Code Won’t Lint	53

Issue: Generated Code Too Large	54

References	54

APB Crossbar Hardware Architecture Specification Index	54

Document Organization	54

Front Matter	54

Chapter 1: Introduction	55

Chapter 2: System Overview	55

Chapter 3: Architecture	55

Chapter 4: Interfaces	55

Chapter 5: Performance	55

Chapter 6: Integration	55

Related Documentation	55

Product Requirements Document (PRD)	56

APB Crossbar Generator	56

1. Executive Summary	56

1.1 Quick Stats	56

1.2 Project Goals	56

2. Key Design Principles	57

2.1 Architecture Philosophy	57

2.2 Design Trade-offs	57

3. Architecture Overview	57

3.1 Top-Level Block Diagram	57

3.2 Address Mapping	58

3.3 Arbitration Strategy	58

4. Available Variants	59

4.1 Pre-Generated Modules	59

4.2 Wrapper Modules	59

5. Functional Requirements	60

FR-1: Arbitrary MxN Configuration	60

FR-2: Round-Robin Arbitration	60

FR-3: Address-Based Routing	60

FR-4: Zero-Bubble Throughput	60

FR-5: Configurable Address Map	60

6. Interface Specifications	61

6.1 Master-Side Interface (APB Slave)	61

6.2 Slave-Side Interface (APB Master)	61

7. Parameter Configuration	61

8. Generator Usage	62

8.1 Pre-Generated Variants	62

8.2 Custom Generation	62

8.3 Generator Options	62

9. Performance Characteristics	62

9.1 Latency	62

9.2 Throughput	63

9.3 Resource Utilization (Estimated)	63

10. Verification Status	63

10.1 Test Coverage	63

10.2 Test Methodology	64

11. Integration Guide	64

11.1 Simple Integration (1to4)	64

11.2 Multi-Master Integration (2to4)	65

12. Design Constraints	65

12.1 Limitations	65

12.2 Assumptions	65

13. Future Enhancements	66

13.1 Potential Features	66

13.2 Generator Improvements	66

14. References	66

14.1 Internal Documentation	66

14.2 External Standards	66

14.3 Related Components	66

Claude Code Guide: APB Crossbar Component	67

Quick Context	67

Critical Rules for This Component	67

Rule #0: This is a GENERATOR, Not Just Modules	67

Rule #1: Address Map is Fixed Per-Slave	68

Rule #2: Know the Pre-Generated Variants	68

Rule #3: Generator Syntax	68

Architecture Quick Reference	69

Module Organization	69

Block Diagram (2×4 Example)	70

Common User Questions and Responses	70

Q: “How do I connect a CPU to 4 peripherals?”	70

Q: “I need CPU and DMA to access peripherals. Which crossbar?”	71

Q: “What if I need 3 masters and 8 slaves?”	71

Q: “How does arbitration work?”	72

Q: “Can I change the address map?”	72

Q: “How do I run tests?”	73

Q: “What’s the thin variant for?”	73

Integration Patterns	74

Pattern 1: Simple Peripheral Interconnect	74

Pattern 2: Multi-Master System	74

Pattern 3: Hierarchical Interconnect	75

Anti-Patterns to Catch	76

✗ Anti-Pattern 1: Generating When Pre-Generated Exists	76

✗ Anti-Pattern 2: Assuming Custom Per-Slave Sizes	76

✗ Anti-Pattern 3: Not Mentioning Address Map	76

✗ Anti-Pattern 4: Forgetting About Wrappers	76

Debugging Workflow	77

Issue: Address Not Routing Correctly	77

Issue: Arbitration Not Fair	77

Issue: Back-to-Back Transactions Stalling	77

Quick Commands	77

Remember	78

APB Crossbar Modules	79

Architecture	79

Key Features	79

Available Modules	79

Generating Crossbars	79

Method 1: Convenience Script (Recommended)	79

Method 2: Direct Generator	80

Address Map	80

Parameters	80

Usage Example	80

Testing	81

Design Notes	81

Arbitration Strategy	81

Address Decoding	82

Timing	82

Known Limitations	82

Generating Custom Variants	82

Files	82

References	83

List of Figures

Figure 1.1: APB Crossbar Top-Level Architecture	27

Figure 2.1: Address Decode Flow	33

List of Tables

Table 1: Top-Level Parameters	29

Table 2: Pre-Generated Crossbar Variants	30

Table 3: Command Line Options	42

List of Waveforms

Waveform 2.1: Round-Robin Arbitration Timing	35

# Apb Xbar Mas Index

Generated: 2026-01-04

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Micro-Architecture Specification Index

Component: APB Crossbar (MxN Interconnect) Version: 1.0 Date: 2026-01-03 Purpose: Detailed micro-architecture specification for APB Crossbar component

## Document Organization

This specification covers the APB Crossbar component - a parametric MxN interconnect for connecting multiple APB masters to multiple APB slaves with automatic address-based routing and round-robin arbitration.

### Main Documentation

README.md

## Chapter 1: Architecture

chapters/01_architecture.md

## Chapter 2: Address Decode and Arbitration

chapters/02_address_and_arbitration.md

## Chapter 3: RTL Generator

chapters/03_rtl_generator.md

## Quick Navigation

### For New Users

- Start with README.md for overview and quick start

- Read 01_architecture.md to understand the design

- Study 02_address_and_arbitration.md for operational details

- Reference 03_rtl_generator.md if custom configuration needed

### For Integration

- Pre-generated variants: See 01_architecture.md Section “Pre-Generated Variants”

- Custom generation: See 03_rtl_generator.md Section “Quick Start”

- Address mapping: See 02_address_and_arbitration.md Section “Address Decode”

- Arbitration behavior: See 02_address_and_arbitration.md Section “Arbitration”

### Common Questions

All answered in README.md Section “Common Questions”

## Visual Assets

All diagrams referenced in the documentation are available in:

- Source Files:

  - assets/graphviz/*.gv - Graphviz source diagrams

  - assets/wavedrom/*.json - WaveJSON timing diagrams

- Rendered Files:

  - assets/png/*.png - PNG format (for document generation)

### Architecture Diagrams

- APB Crossbar Architecture (2x4 Example)

  - Source: assets/graphviz/apb_xbar_architecture.gv

  - Rendered: assets/png/apb_xbar_architecture.png

- Address Decode Flow

  - Source: assets/graphviz/address_decode_flow.gv

  - Rendered: assets/png/address_decode_flow.png

### Timing Diagrams

- Round-Robin Arbitration

  - Source: assets/wavedrom/arbitration_round_robin.json

  - Rendered: assets/wavedrom/arbitration_round_robin.png

## Component Overview

### Key Features

- Parametric MxN Configuration: Any combination of M masters and N slaves (up to 16x16)

- Automatic Address Decode: 64KB per slave, simple offset-based routing

- Round-Robin Arbitration: Per-slave fair arbitration, no master starvation

- Zero-Bubble Throughput: Back-to-back transactions without idle cycles

- Grant Persistence: Hold grant through transaction completion

- RTL Generation: Python-based generator for custom configurations

### Pre-Generated Variants

| Module | M×N | Use Case |
| --- | --- | --- |
| apb_xbar_1to1 | 1×1 | Passthrough, protocol conversion |
| apb_xbar_2to1 | 2×1 | Multi-master arbitration |
| apb_xbar_1to4 | 1×4 | Simple SoC peripheral bus |
| apb_xbar_2to4 | 2×4 | Typical SoC with CPU+DMA |
| apb_xbar_thin | 1×1 | Minimal overhead passthrough |

### Design Philosophy

Proven Building Blocks: - Built from production-tested apb_slave and apb_master modules - No new protocol logic - pure composition - Each component independently verified

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants for fast integration - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol → cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp → protocol

## Related Documentation

### Companion Specifications

- APB Crossbar HAS - Hardware Architecture Specification (high-level)

### Project-Level

- PRD.md: ../../PRD.md - Complete product requirements document

- CLAUDE.md: ../../CLAUDE.md - AI assistant integration guide

- README.md: ../../README.md - Quick start guide

### Test Infrastructure

- Test Directory: ../../dv/tests/ - CocoTB + pytest test suite

- Test Results: All pre-generated variants 100% passing

### RTL

- Core Modules: ../../rtl/apb_xbar_*.sv - Pre-generated crossbars

- Wrappers: ../../rtl/wrappers/ - Pre-configured wrappers

- Generator: ../../bin/generate_xbars.py - Python generator script

## Version History

Version 1.0 (2025-10-25): - Initial specification release - Complete visual documentation (3 diagrams) - Comprehensive generator documentation - All pre-generated variants verified (100% passing)

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Visual Documentation

Component: APB Crossbar (MxN Interconnect) Version: 1.0 Status: Production Ready Last Updated: 2025-10-25

## Overview

This directory contains visual documentation for the APB Crossbar component, including architecture diagrams, timing waveforms, and generator documentation.

Component Purpose: Parametric MxN APB interconnect connecting multiple masters to multiple slaves with automatic address-based routing and round-robin arbitration.

## Documentation Structure

### Main Documentation Files

| Document | Description | Contents |
| --- | --- | --- |
| 01_architecture.md | Architecture Overview | Top-level block diagram, functional blocks, signal flow, parameters |
| 02_address_and_arbitration.md | Address Decode & Arbitration | Address map structure, decode algorithm, round-robin timing |
| 03_rtl_generator.md | RTL Generator Guide | Generator architecture, usage, customization, advanced topics |

### Supporting Documentation

| Document | Location | Description |
| --- | --- | --- |
| PRD.md | ../../PRD.md | Complete product requirements document |
| CLAUDE.md | ../../CLAUDE.md | AI assistant integration guide |
| README.md | ../../README.md | Quick start guide |

## Visual Assets

### Architecture Diagrams

| Diagram | Format | Description |
| --- | --- | --- |
| apb_xbar_architecture | Graphviz | 2x4 crossbar showing master-side slaves, arbitration, slave-side masters |
| address_decode_flow | Graphviz | Step-by-step address decode example (0x10023456 → Slave 2) |

Rendered Formats: - PNG: assets/svg/*.svg (for markdown embedding) - SVG: assets/svg/*.svg (for web viewing, scalable)

### Timing Diagrams

| Diagram | Format | Description |
| --- | --- | --- |
| arbitration_round_robin | WaveJSON | 2 masters competing for Slave 0 with round-robin arbitration |

Rendered Format: - PNG: assets/svg/arbitration_round_robin.svg

## Quick Start

### For New Users

- Understand the Architecture

  - Read 01_architecture.md

  - Study the architecture diagram

  - Understand master-side/slave-side protocol conversion

- Learn Address Decode & Arbitration

  - Read 02_address_and_arbitration.md

  - Study address decode flow diagram

  - Study arbitration timing diagram

- Generate Custom Crossbar (if needed)

  - Read 03_rtl_generator.md

  - Use pre-generated variants (1to1, 2to1, 1to4, 2to4) if possible

  - Run generator for custom MxN configurations

- Integrate Into Your Design

  - See integration examples in CLAUDE.md

  - Reference complete specification in PRD.md

### For Existing Users

Need a Crossbar? - 1 master, N slaves → Use apb_xbar_1toN or generate - 2 masters, N slaves → Use apb_xbar_2toN or generate - M masters, N slaves → Generate with generate_xbars.py

Understanding Behavior? - Address decode issues → See 02_address_and_arbitration.md - Arbitration questions → See 02_address_and_arbitration.md

Modifying/Generating? - Generator usage → See 03_rtl_generator.md

## Directory Structure

docs/apb_xbar_spec/
├── README.md                              ← This file
├── 01_architecture.md                     ← Architecture overview
├── 02_address_and_arbitration.md          ← Decode & arbitration details
├── 03_rtl_generator.md                    ← Generator documentation
└── assets/
    ├── graphviz/                          ← Source diagrams
    │   ├── apb_xbar_architecture.gv
    │   └── address_decode_flow.gv
    ├── wavedrom/                          ← Source timing diagrams
    │   └── arbitration_round_robin.json
    ├── svg/                               ← Rendered SVG (scalable)
    │   ├── apb_xbar_architecture.svg
    │   └── address_decode_flow.svg
    └── png/                               ← Rendered PNG (embedded)
        ├── apb_xbar_architecture.svg
        ├── address_decode_flow.svg
        └── arbitration_round_robin.svg

## Key Concepts

### 1. Architecture

Three-Stage Design: 1. Master-Side: APB slaves convert protocol → cmd/rsp 2. Internal Logic: Address decode + arbitration + response routing 3. Slave-Side: APB masters convert cmd/rsp → protocol

Why This Design? - Reuses proven apb_slave and apb_master components - Clean separation of concerns - Scalable to any MxN configuration

### 2. Address Decode

Fixed 64KB Per Slave:

Slave 0: BASE_ADDR + 0x00000 - 0x0FFFF
Slave 1: BASE_ADDR + 0x10000 - 0x1FFFF
Slave 2: BASE_ADDR + 0x20000 - 0x2FFFF
...

Decode Formula:

offset = PADDR - BASE_ADDR
slave_index = offset[19:16]  // Divide by 64KB

See: 02_address_and_arbitration.md

### 3. Arbitration

Per-Slave Round-Robin: - Each slave has independent arbiter - Priority rotates after each grant - Grant persists through transaction completion - No master starvation

See: 02_address_and_arbitration.md

### 4. RTL Generation

All Generator Code in Component Area: - Convenience Wrapper: bin/generate_xbars.py (top-level script) - Core Generator: bin/apb_xbar_generator.py (library implementation)

Usage:

# Generate standard variants
python generate_xbars.py

# Generate custom 3x6 crossbar
python generate_xbars.py --masters 3 --slaves 6

See: 03_rtl_generator.md

## Common Questions

### Q: Which pre-generated variant should I use?

Decision Tree:

1 master, 1 slave  → apb_xbar_1to1 (passthrough)
1 master, 4 slaves → apb_xbar_1to4 (address decode only)
2 masters, 1 slave → apb_xbar_2to1 (arbitration only)
2 masters, 4 slaves → apb_xbar_2to4 (full crossbar)
Other MxN          → Generate with generate_xbars.py

### Q: How do I change the base address?

At Instantiation:

apb_xbar_1to4 #(
    .BASE_ADDR(32'h8000_0000)  // Override default 0x10000000
) u_xbar (...);

Or Generate with Custom Base:

python generate_xbars.py --masters 2 --slaves 4 --base-addr 0x80000000

### Q: Can I change per-slave address sizes?

No (Current Limitation): Each slave is fixed at 64KB.

Workarounds: 1. Use multiple crossbars with different BASE_ADDR 2. Modify generator’s decode logic (advanced) 3. Address translation in slaves

See: 03_rtl_generator.md#limitations

### Q: How does arbitration work with 3+ masters?

Round-Robin Example (4 Masters, Same Slave):

Initial priority: M0
Transaction 1: M0 requests → M0 granted → Priority rotates to M1
Transaction 2: M0, M1, M2 request → M1 granted → Priority rotates to M2
Transaction 3: M0, M3 request → M2 skipped (not requesting) → M3 granted → Priority rotates to M0
Transaction 4: M0, M1 request → M0 granted → Priority rotates to M1

Key: Priority rotates after each grant, ensuring fairness.

See: 02_address_and_arbitration.md#arbitration

### Q: What’s the throughput?

Single Master: - Zero-bubble (back-to-back transactions without gaps) - Limited only by slave PREADY response time

Multiple Masters (Same Slave): - Fair sharing: Each master gets ~1/M bandwidth - Example: 2 masters = 50% each, 4 masters = 25% each

Multiple Masters (Different Slaves): - Full parallelism: Each master gets 100% of its target slave - Total system bandwidth = NUM_SLAVES × slave_bandwidth

See: 02_address_and_arbitration.md#performance

## Regenerating Diagrams

### Graphviz Diagrams (Architecture, Address Decode)

cd assets/graphviz/

# Generate SVG
dot -Tsvg apb_xbar_architecture.gv -o ../svg/apb_xbar_architecture.svg
dot -Tsvg address_decode_flow.gv -o ../svg/address_decode_flow.svg

# Generate PNG
dot -Tsvg apb_xbar_architecture.gv -o ../svg/apb_xbar_architecture.svg
dot -Tsvg address_decode_flow.gv -o ../svg/address_decode_flow.svg

### WaveJSON Diagrams (Timing)

cd assets/wavedrom/

# Generate PNG
wavedrom-cli -i arbitration_round_robin.json -s ../svg/arbitration_round_robin.svg

## Version History

Version 1.0 (2025-10-25): - Initial visual documentation release - 3 main documentation files - 2 Graphviz architecture diagrams - 1 WaveJSON timing diagram - RTL generator documentation

## Maintainers

RTL Design Sherpa Project

Questions or Issues: 1. Check this documentation first 2. Review PRD.md for complete specification 3. Review CLAUDE.md for integration guidance 4. Open GitHub issue if problem persists

Last Updated: 2025-10-25 Version: 1.0 Status: Production Ready

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Architecture

Component: APB Crossbar (MxN Interconnect) Version: 1.0 Status: Production Ready

## Overview

The APB Crossbar is a parametric interconnect that connects M APB masters to N APB slaves with automatic address-based routing and per-slave round-robin arbitration. Built from proven apb_slave and apb_master components, the crossbar provides a clean, scalable solution for SoC peripheral interconnect.

Key Features: - Arbitrary MxN configuration (up to 16x16) - Automatic address decode (64KB per slave) - Round-robin arbitration per slave - Zero-bubble throughput - Grant persistence through transaction completion

## Architecture Diagram

The following diagram shows a 2x4 crossbar configuration connecting 2 masters (CPU and DMA) to 4 slaves (UART, GPIO, Timer, SPI):

### Figure 1.1: APB Crossbar Top-Level Architecture

APB Crossbar Architecture (2x4 Example)

The figure shows 2 masters connected to 4 slaves via master-side protocol conversion, internal arbitration logic, and slave-side protocol conversion.

## Functional Blocks

### 1. Master-Side Protocol Conversion

Component: apb_slave[M] instances (one per master)

Purpose: Convert incoming APB protocol transactions to internal cmd/rsp bus format

Features: - Full APB protocol handling (PSEL, PENABLE, PREADY) - Transaction buffering - Error response generation - Back-to-back transaction support

Dataflow:

APB Master → apb_slave → cmd/rsp bus → Internal Crossbar Logic

### 2. Internal Crossbar Logic

Components: - Address Decode - Parallel decode to determine target slave - Per-Slave Arbiters - Round-robin arbitration for each slave - Response Routing - Registered paths back to requesting masters

Key Operations:

Address Decode:

offset = PADDR - BASE_ADDR
slave_index = offset[19:16]  // Upper 4 bits of 20-bit offset (64KB regions)

Arbitration: - Independent arbiter per slave - Round-robin priority rotation - Grant persistence through response - No starvation guarantee

Response Routing: - Track which master initiated each transaction - Route PRDATA/PSLVERR back to originating master - Maintain transaction ordering per master

### 3. Slave-Side Protocol Conversion

Component: apb_master[N] instances (one per slave)

Purpose: Convert internal cmd/rsp bus format back to APB protocol for slaves

Features: - APB protocol generation (PSEL, PENABLE timing) - Response collection (PRDATA, PSLVERR, PREADY) - Wait state handling - Error propagation

Dataflow:

Internal Crossbar Logic → cmd/rsp bus → apb_master → APB Slave

## Signal Flow Example

Transaction: Master 0 (CPU) writes to Slave 2 (Timer) at address 0x10023456

Step-by-step:

- Master 0 → apb_slave[0]:

  - CPU asserts PSEL, PADDR=0x10023456, PWRITE=1

  - apb_slave[0] converts to cmd/rsp format

- Address Decode:

  - offset = 0x10023456 - 0x10000000 = 0x00023456

  - slave_index = 0x00023456 >> 16 = 0x2

  - Target: Slave 2

- Arbiter[2]:

  - Check if Slave 2 is available

  - Grant to Master 0 (if no conflict)

  - Route cmd to apb_master[2]

- apb_master[2] → Slave 2:

  - Generate APB write transaction

  - Assert PSEL[2], PENABLE, PADDR, PWDATA

  - Wait for PREADY

- Response Path:

  - Slave 2 responds with PREADY, PSLVERR

  - apb_master[2] captures response

  - Response routed back to apb_slave[0]

  - apb_slave[0] returns PREADY to CPU

Total Latency: Typically 2-3 cycles for uncontended access

## Parameter Configuration

Top-Level Parameters:

Top-Level Parameters

| Parameter | Range | Default | Description |
| --- | --- | --- | --- |
| NUM_MASTERS | 1-16 | 2 | Number of APB masters |
| NUM_SLAVES | 1-16 | 4 | Number of APB slaves |
| ADDR_WIDTH | 8-64 | 32 | Address bus width |
| DATA_WIDTH | 8-64 | 32 | Data bus width |
| BASE_ADDR | Any | 0x10000000 | Base address for address map |

Derived Parameters: - Slave address range: 64KB (0x10000) per slave - Total address space: NUM_SLAVES × 64KB

## Pre-Generated Variants

Pre-Generated Crossbar Variants

| Module | M×N | LOC | Use Case |
| --- | --- | --- | --- |
| apb_xbar_1to1 | 1×1 | ~200 | Passthrough, protocol conversion |
| apb_xbar_2to1 | 2×1 | ~400 | Multi-master arbitration |
| apb_xbar_1to4 | 1×4 | ~500 | Simple SoC peripheral bus |
| apb_xbar_2to4 | 2×4 | ~1000 | Typical SoC with CPU+DMA |
| apb_xbar_thin | 1×1 | ~150 | Minimal overhead passthrough |

Custom Variants: Generated on-demand via Python script

## Design Philosophy

Proven Components: - Built from production-tested apb_slave.sv and apb_master.sv - No new protocol logic - pure composition - Each component independently verified

Scalability: - Parametric generation for any MxN configuration - Resource usage scales linearly with M×N - No centralized bottlenecks

Predictability: - Round-robin arbitration provides deterministic behavior - Fixed address map simplifies software integration - Zero-bubble design ensures maximum throughput

## Next Steps

- See Address Decode and Arbitration for detailed operation examples

- See PRD.md for complete specification

- See CLAUDE.md for integration guidance

- See README.md for quick start guide

Version: 1.0 Last Updated: 2025-10-25 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Address Decode and Arbitration

Component: APB Crossbar (MxN Interconnect) Version: 1.0 Status: Production Ready

## Overview

This document details the two core mechanisms of the APB Crossbar: 1. Address Decode - How incoming addresses map to specific slaves 2. Arbitration - How multiple masters share access to the same slave

## Address Decode

### Address Map Structure

The crossbar uses a fixed 64KB (0x10000 bytes) region per slave:

BASE_ADDR + 0x0000_0000 → 0x0000_FFFF : Slave 0 (64KB)
BASE_ADDR + 0x0001_0000 → 0x0001_FFFF : Slave 1 (64KB)
BASE_ADDR + 0x0002_0000 → 0x0002_FFFF : Slave 2 (64KB)
BASE_ADDR + 0x0003_0000 → 0x0003_FFFF : Slave 3 (64KB)
...
BASE_ADDR + 0x000F_0000 → 0x000F_FFFF : Slave 15 (64KB)

Default BASE_ADDR: 0x10000000

Total Address Space: NUM_SLAVES × 64KB

### Decode Algorithm

The crossbar extracts the slave index from the upper bits of the address offset:

offset = PADDR - BASE_ADDR
slave_index = offset[19:16]  // Upper 4 bits of 20-bit offset

Why offset[19:16]? - 64KB = 0x10000 = 2^16 bytes - Lower 16 bits (offset[15:0]) are byte address within slave - Upper 4 bits (offset[19:16]) select which of 16 slaves - Supports up to 16 slaves (0x0 through 0xF)

### Address Decode Flow Diagram

The following diagram shows a concrete example of how address 0x10023456 routes to Slave 2:

### Figure 2.1: Address Decode Flow

Address Decode Flow

The figure shows step-by-step routing of address 0x10023456 to Slave 2.

### Decode Example Walkthrough

Scenario: Master accesses address 0x10023456 with BASE_ADDR = 0x10000000

Step 1: Calculate Offset

offset = PADDR - BASE_ADDR
       = 0x10023456 - 0x10000000
       = 0x00023456

Step 2: Extract Slave Index

slave_index = offset[19:16]
            = 0x00023456 >> 16
            = 0x2

Step 3: Parallel Decode

The crossbar checks all slave ranges in parallel: - Slave 0: 0x00000 - 0x0FFFF → NO MATCH - Slave 1: 0x10000 - 0x1FFFF → NO MATCH - Slave 2: 0x20000 - 0x2FFFF → MATCH ✓ - Slave 3: 0x30000 - 0x3FFFF → NO MATCH

Step 4: Route to Arbiter

Transaction request forwarded to Arbiter[2] (Slave 2’s arbiter)

Step 5: Forward to Slave

After arbitration grant, transaction forwarded via apb_master[2] to physical Slave 2

Final Address Sent to Slave: 0x10023456 (full address preserved)

### Multiple Address Maps

You can create multiple distinct address maps by using different BASE_ADDR values:

Example: Two Crossbars

// Peripheral Bus: 0x1000_0000 - 0x1003_FFFF
apb_xbar_1to4 #(
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (...);
// Slaves at: 0x1000_0000, 0x1001_0000, 0x1002_0000, 0x1003_0000

// Memory-Mapped I/O: 0x8000_0000 - 0x8003_FFFF
apb_xbar_1to4 #(
    .BASE_ADDR(32'h8000_0000)
) u_mmio_xbar (...);
// Slaves at: 0x8000_0000, 0x8001_0000, 0x8002_0000, 0x8003_0000

## Arbitration

### Per-Slave Round-Robin

Each slave has an independent arbiter that implements round-robin scheduling:

Key Properties: - Fair: No master can starve another - Independent: Each slave arbitrates separately - Predictable: Priority rotates after each grant - Persistent: Grant held from command through response

### Round-Robin Timing Diagram

The following timing diagram shows 2 masters (M0, M1) competing for access to Slave 0:

### Waveform 2.1: Round-Robin Arbitration Timing

Round-Robin Arbitration Timing

### Arbitration Example Walkthrough

Scenario: Master 0 and Master 1 both want to access Slave 0

Initial State: - Priority: M0 (M0 has priority initially) - Slave 0: IDLE

Transaction 1: M0 Requests Slave 0

Cycle 1: M0 asserts request (M0_PSEL, M0_PADDR=0x1000_0000)
Cycle 2: Arbiter[0] grants to M0 (only requester)
         M0_PENABLE asserted
         S0_PSEL asserted to Slave 0
Cycle 3: Slave 0 responds (S0_PREADY)
         Transaction completes
         Priority rotates: M1 now has priority

Transaction 2: M0 and M1 Both Request Slave 0

Cycle 4: M0 asserts request (M0_PADDR=0x1000_0010)
         M1 asserts request (M1_PADDR=0x1000_0000) -- CONFLICT!
Cycle 5: Arbiter[0] grants to M1 (has priority)
         M1_PENABLE asserted
         S0_PSEL asserted to Slave 0
         M0 blocked, waits
Cycle 6: Slave 0 responds (S0_PREADY)
         M1 transaction completes
         Priority rotates: M0 now has priority

Transaction 3: M0 Request (After Rotation)

Cycle 7: M0 still asserting request (was blocked)
Cycle 8: Arbiter[0] grants to M0 (now has priority)
         M0_PENABLE asserted
         S0_PSEL asserted to Slave 0
Cycle 9: Slave 0 responds (S0_PREADY)
         Transaction completes
         Priority rotates: M1 now has priority

Result: Fair access - each master gets served in turn when both request

### Multi-Slave Parallelism

Key Feature: Different slaves can be accessed simultaneously by different masters

Example: Parallel Transactions

Time T0:
- Master 0 accesses Slave 0 (UART) - GRANTED
- Master 1 accesses Slave 2 (Timer) - GRANTED
Both transactions proceed in parallel (no conflict)

Time T1:
- Master 0 accesses Slave 0 (UART) - GRANTED
- Master 1 accesses Slave 0 (UART) - BLOCKED (arbitration)
Master 1 waits for Master 0 to complete

Time T2:
- Master 0 completes, releases Slave 0
- Master 1 accesses Slave 0 (UART) - GRANTED
Priority rotated for Slave 0's arbiter

Benefit: Maximum throughput when masters access different slaves

### Grant Persistence

Critical Property: Once granted, a master holds the slave until response completes

Why This Matters:

WITHOUT Grant Persistence:
- Master 0 granted Slave 0
- Master 0 asserts PENABLE
- *Grant could change here* ← BREAKS PROTOCOL!
- Slave 0 responds to wrong master

WITH Grant Persistence:
- Master 0 granted Slave 0
- Master 0 asserts PENABLE
- Grant held until PREADY asserted ← SAFE
- Slave 0 responds to correct master
- Grant released for next transaction

Implementation: Grant signal registered and held from PSEL assertion through PREADY response

### Arbitration Latency

Best Case (No Contention): - 0 cycles arbitration delay - Request → Grant in same cycle - APB protocol overhead: 2 cycles minimum (PSEL + PENABLE)

Worst Case (Maximum Contention): - M-1 cycles wait time (other masters ahead in round-robin) - Example: 2 masters, worst case = 1 cycle wait - Example: 4 masters, worst case = 3 cycles wait

Average Case (Random Access Pattern): - (M-1)/2 cycles average wait time - Statistical fairness over time

## Integration Examples

### Example 1: CPU to 4 Peripherals (No Contention)

apb_xbar_1to4 #(
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (
    // Single master (CPU)
    .m0_apb_* (cpu_apb_*),

    // 4 slaves (UART, GPIO, Timer, SPI)
    .s0_apb_* (uart_*),   // 0x1000_0000
    .s1_apb_* (gpio_*),   // 0x1001_0000
    .s2_apb_* (timer_*),  // 0x1002_0000
    .s3_apb_* (spi_*)     // 0x1003_0000
);

Behavior: - No arbitration needed (single master) - Pure address decode functionality - Zero arbitration overhead

### Example 2: CPU + DMA to 4 Peripherals (Potential Contention)

apb_xbar_2to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_soc_xbar (
    // Two masters
    .m0_apb_* (cpu_apb_*),
    .m1_apb_* (dma_apb_*),

    // 4 slaves
    .s0_apb_* (mem_ctrl_*),  // 0x4000_0000
    .s1_apb_* (uart_*),      // 0x4001_0000
    .s2_apb_* (i2c_*),       // 0x4002_0000
    .s3_apb_* (adc_*)        // 0x4003_0000
);

Behavior: - Each slave has independent arbiter - CPU and DMA can access different slaves simultaneously - If both access same slave, round-robin arbitration - Fair access guaranteed

## Performance Characteristics

### Throughput

Single Master: - Back-to-back transactions supported - Zero bubble (no idle cycles between transactions) - Limited only by slave PREADY response time

Multiple Masters (Same Slave): - Round-robin introduces fair sharing - Each master gets ~1/M of bandwidth - Example: 2 masters = 50% bandwidth each

Multiple Masters (Different Slaves): - Full parallelism - Each master gets 100% of its target slave - Total system bandwidth = NUM_SLAVES × slave_bandwidth

### Latency

Components:

- Address Decode: 0 cycles (combinational, parallel)

- Arbitration Decision: 0 cycles (combinational)

- APB Protocol: 2 cycles minimum (PSEL + PENABLE)

- Slave Response: Variable (depends on slave)

Total Minimum Latency: 2 cycles (uncontended access)

With Contention: +1 to +(M-1) cycles arbitration wait time

## Design Notes

### Why 64KB Per Slave?

Rationale: - Sufficient for most APB peripherals (typically 256B - 4KB register space) - Simple decode logic (single shift operation) - Allows up to 16 slaves with clean byte alignment - Software-friendly (each peripheral has “round” base address)

Alternatives: - Smaller regions (4KB, 16KB) → more slaves, more complex decode - Larger regions (256KB, 1MB) → fewer slaves, wasted address space

### Why Round-Robin?

Advantages: - Fair: Prevents starvation - Simple: Minimal logic (counter + comparator) - Predictable: Software can reason about worst-case latency - No configuration: Works out-of-box

Alternatives: - Fixed priority → Can starve low-priority masters - Weighted fair queuing → More complex, requires configuration - Lottery scheduling → Unpredictable, harder to verify

## Troubleshooting

### Issue: Wrong Slave Selected

Check: 1. BASE_ADDR parameter correct? 2. Address within expected range? 3. Calculated slave_index matches expectation?

Debug:

offset = PADDR - BASE_ADDR
slave_index = offset >> 16
expected_slave = slave_index  // Should match actual PSEL

### Issue: Master Starved

Check: 1. Other masters continuously accessing same slave? 2. Arbitration timeout monitoring enabled? 3. Round-robin priority rotating correctly?

Expected Behavior: - Each master should get grant within M transactions - No master should wait indefinitely

### Issue: Back-to-Back Transactions Stalling

Check: 1. Grant persistence working? 2. Slave asserting PREADY correctly? 3. No unintended pipeline bubbles?

Expected Behavior: - Consecutive transactions from same master should flow without gaps - Only arbitration conflicts should introduce wait states

## Next Steps

- See Architecture for top-level design overview

- See PRD.md for complete specification

- See CLAUDE.md for integration guidance

- See README.md for quick start guide

Version: 1.0 Last Updated: 2025-10-25 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar RTL Generator

Component: APB Crossbar (MxN Interconnect) Version: 1.0 Status: Production Ready

## Overview

The APB Crossbar uses parametric RTL generation to create custom MxN configurations. While several pre-generated variants exist (1to1, 2to1, 1to4, 2to4, thin), the Python-based generator enables creation of any arbitrary MxN crossbar up to 16x16.

Philosophy: - Pre-generated variants for common use cases (fast integration) - Python generator for custom configurations (flexibility) - Template-based code generation (consistency) - Built from proven components (quality)

## Generator Architecture

### Two-Level Structure

projects/components/apb_xbar/
├── bin/
│   └── generate_xbars.py              ← Convenience wrapper script
│
└── (repo root)/bin/rtl_generators/amba/
    └── apb_xbar_generator.py          ← Core generator engine

Why Two Files? 1. Core Generator (apb_xbar_generator.py): Reusable library function - Can be imported by other scripts - Contains generate_apb_xbar() function - Located in central bin/rtl_generators/ directory

- Convenience Wrapper (generate_xbars.py): Project-specific script

  - Easy command-line interface

  - Pre-configured for standard variants

  - Located in project bin/ directory

## Quick Start

### Generate Standard Variants (1:1, 2:1, 1:4, 2:4)

cd projects/components/apb_xbar/bin/
python generate_xbars.py

Output:

Generating 1-to-1 crossbar...
  ✓ apb_xbar_1to1.sv
Generating 2-to-1 crossbar...
  ✓ apb_xbar_2to1.sv
Generating 1-to-4 crossbar...
  ✓ apb_xbar_1to4.sv
Generating 2-to-4 crossbar...
  ✓ apb_xbar_2to4.sv

✓ Generated 4 crossbar variants

### Generate Custom Variant

cd projects/components/apb_xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

Output:

Generating 3-to-6 crossbar...
✓ Generated apb_xbar_3to6.sv

### Generate with Custom Base Address

cd projects/components/apb_xbar/bin/
python generate_xbars.py --masters 4 --slaves 8 --base-addr 0x80000000

Output:

Generating 4-to-8 crossbar...
✓ Generated apb_xbar_4to8.sv

Address Map:
  Slave 0: [0x80000000, 0x8000FFFF]
  Slave 1: [0x80010000, 0x8001FFFF]
  Slave 2: [0x80020000, 0x8002FFFF]
  ...
  Slave 7: [0x80070000, 0x8007FFFF]

## Command-Line Interface

### Full Options

python generate_xbars.py [OPTIONS]

Options:

Command Line Options

| Option | Short | Type | Default | Description |
| --- | --- | --- | --- | --- |
| --masters | -m | int | None | Number of master interfaces (1-16) |
| --slaves | -s | int | None | Number of slave interfaces (1-16) |
| --base-addr | -b | hex | 0x10000000 | Base address for slave address map |
| --help | -h | - | - | Show help message |

Rules: - If neither --masters nor --slaves specified → Generate all standard variants - If only one specified → Error (must specify both) - If both specified → Generate custom variant

### Examples

Example 1: Generate All Standard Variants

python generate_xbars.py

Example 2: Generate 3x6 Crossbar

python generate_xbars.py --masters 3 --slaves 6

Example 3: Generate 5x10 Crossbar with Custom Base

python generate_xbars.py -m 5 -s 10 -b 0xA0000000

Example 4: Show Help

python generate_xbars.py --help

## Generated Code Structure

### Template Components

The generator creates a complete SystemVerilog module with:

- Header with Address Map

// 3-to-6 APB crossbar with address decoding and arbitration
// 3 masters to 6 slaves using apb_slave and apb_master modules
//
// Address Map (same for all masters):
//   Slave 0: [0x10000000, 0x1000FFFF]
//   Slave 1: [0x10010000, 0x1001FFFF]
//   Slave 2: [0x10020000, 0x1002FFFF]
//   Slave 3: [0x10030000, 0x1003FFFF]
//   Slave 4: [0x10040000, 0x1004FFFF]
//   Slave 5: [0x10050000, 0x1005FFFF]

- Module Declaration with Parameters

module apb_xbar_3to6 #(
    parameter int ADDR_WIDTH = 32,
    parameter int DATA_WIDTH = 32,
    parameter int STRB_WIDTH = DATA_WIDTH / 8,
    parameter logic [ADDR_WIDTH-1:0] BASE_ADDR = 32'h10000000
) (
    // Clock and Reset
    input  logic                  pclk,
    input  logic                  presetn,

    // Master 0-2 APB interfaces
    // Slave 0-5 APB interfaces
);

- Internal Signal Declarations

// Command/Response interfaces for master 0-2 apb_slave
    logic m0_cmd_valid, m0_cmd_ready, m0_cmd_pwrite;
    logic [ADDR_WIDTH-1:0] m0_cmd_paddr;
    // ... (full cmd/rsp bus for each master)

    // Command/Response interfaces for slave 0-5 apb_master
    logic s0_cmd_valid, s0_cmd_ready, s0_cmd_pwrite;
    logic [ADDR_WIDTH-1:0] s0_cmd_paddr;
    // ... (full cmd/rsp bus for each slave)

- Master-Side apb_slave Instantiations

// Master 0 apb_slave
    apb_slave #(
        .ADDR_WIDTH(ADDR_WIDTH),
        .DATA_WIDTH(DATA_WIDTH)
    ) u_m0_slave (
        .pclk(pclk),
        .presetn(presetn),
        // APB interface
        .apb_PSEL(m0_apb_PSEL),
        .apb_PENABLE(m0_apb_PENABLE),
        // ... (full APB connections)
        // cmd/rsp interface
        .cmd_valid(m0_cmd_valid),
        .cmd_ready(m0_cmd_ready),
        // ... (full cmd/rsp connections)
    );

    // Master 1, 2 apb_slave instances...

- Address Decode Logic

// Address decode for each master
    logic [M-1:0][N-1:0] m_to_s_req;  // Request matrix

    // Master 0 address decode
    always_comb begin
        m_to_s_req[0] = '0;
        if (m0_cmd_valid) begin
            logic [ADDR_WIDTH-1:0] offset = m0_cmd_paddr - BASE_ADDR;
            logic [3:0] slave_idx = offset[19:16];
            case (slave_idx)
                4'd0: m_to_s_req[0][0] = 1'b1;
                4'd1: m_to_s_req[0][1] = 1'b1;
                // ... (case for each slave)
            endcase
        end
    end

    // Master 1, 2 address decode...

- Per-Slave Arbitration Logic

// Arbitration for each slave
    logic [N-1:0][M-1:0] s_grant;  // Grant matrix
    logic [N-1:0][$clog2(M)-1:0] s_arb_priority;  // Round-robin priority

    // Slave 0 arbiter (round-robin among M masters)
    always_ff @(posedge pclk or negedge presetn) begin
        if (!presetn) begin
            s_arb_priority[0] <= '0;
            s_grant[0] <= '0;
        end else begin
            // Round-robin arbitration logic
            // Priority rotation after each grant
        end
    end

    // Slave 1-5 arbiters...

- Response Routing Logic

// Route responses back to requesting masters
    always_comb begin
        // Track which master initiated each transaction
        // Route PRDATA/PSLVERR back to correct master
    end

- Slave-Side apb_master Instantiations

// Slave 0 apb_master
    apb_master #(
        .ADDR_WIDTH(ADDR_WIDTH),
        .DATA_WIDTH(DATA_WIDTH)
    ) u_s0_master (
        .pclk(pclk),
        .presetn(presetn),
        // cmd/rsp interface
        .cmd_valid(s0_cmd_valid),
        .cmd_ready(s0_cmd_ready),
        // ... (full cmd/rsp connections)
        // APB interface
        .apb_PSEL(s0_apb_PSEL),
        .apb_PENABLE(s0_apb_PENABLE),
        // ... (full APB connections)
    );

    // Slave 1-5 apb_master instances...

- Module End

endmodule : apb_xbar_3to6

## Generator Algorithm

### Step-by-Step Process

1. Validate Parameters

if M < 1 or M > 16:
    raise ValueError(f"Number of masters must be 1-16, got {M}")
if N < 1 or N > 16:
    raise ValueError(f"Number of slaves must be 1-16, got {N}")

2. Calculate Derived Parameters

slave_addr_bits = max(1, math.ceil(math.log2(N)))  # Bits for slave index
strb_width = data_width // 8                        # Byte strobe width
module_name = f"apb_xbar_{M}to{N}"                 # Module name

3. Generate Address Map Documentation

for s in range(N):
    addr_offset = s * 0x10000  # 64KB per slave
    addr_start = base_addr + addr_offset
    addr_end = addr_start + 0xFFFF
    # Generate comment: Slave S: [START, END]

4. Generate Module Declaration - Parameters: ADDR_WIDTH, DATA_WIDTH, STRB_WIDTH, BASE_ADDR - Ports: Clock, reset - Master interfaces: M × full APB interface - Slave interfaces: N × full APB interface

5. Generate Internal Signals - Master cmd/rsp buses: M sets - Slave cmd/rsp buses: N sets - Request matrix: [M][N] - Grant matrix: [N][M] - Priority counters: [N]

6. Instantiate Master-Side Components

for m in range(M):
    # Generate apb_slave instance
    # Connect APB interface to external master m
    # Connect cmd/rsp to internal buses

7. Generate Address Decode Logic

for m in range(M):
    # For each master, decode address to slave index
    # offset = paddr - BASE_ADDR
    # slave_idx = offset[19:16]  # Upper 4 bits
    # Set m_to_s_req[m][slave_idx] = 1

8. Generate Arbitration Logic

for s in range(N):
    # For each slave, arbitrate among M masters
    # Round-robin: priority rotates after each grant
    # Grant persistence: hold until transaction completes

9. Generate Response Routing

# Track transaction source (which master)
# Route responses back to originating master

10. Instantiate Slave-Side Components

for s in range(N):
    # Generate apb_master instance
    # Connect cmd/rsp from internal buses
    # Connect APB interface to external slave s

11. Write Generated Code

with open(output_file, 'w') as f:
    f.write(generated_code)

## Code Generation Patterns

### Loop-Based Generation

Master Interfaces:

for m in range(M):
    code += f"    // Master {m} APB interface\n"
    code += f"    input  logic m{m}_apb_PSEL,\n"
    code += f"    input  logic m{m}_apb_PENABLE,\n"
    # ... (all APB signals for master m)

Slave Interfaces:

for s in range(N):
    code += f"    // Slave {s} APB interface\n"
    code += f"    output logic s{s}_apb_PSEL,\n"
    code += f"    output logic s{s}_apb_PENABLE,\n"
    # ... (all APB signals for slave s)

### String Formatting

Hex Values:

code += f"    parameter logic [ADDR_WIDTH-1:0] BASE_ADDR = {addr_width}'h{base_addr:08X}\n"
# Example: 32'h10000000

Address Map Comments:

addr_start = base_addr + (s * 0x10000)
code += f"//   Slave {s}: [0x{addr_start:08X}, 0x{addr_start + 0xFFFF:08X}]\n"
# Example: Slave 2: [0x10020000, 0x1002FFFF]

## Customization Examples

### Example 1: Large SoC with 10 Peripherals

Scenario: CPU + DMA need to access 10 peripherals

Command:

python generate_xbars.py --masters 2 --slaves 10

Generated File: apb_xbar_2to10.sv

Address Map:

Slave 0:  0x10000000 - 0x1000FFFF (UART0)
Slave 1:  0x10010000 - 0x1001FFFF (UART1)
Slave 2:  0x10020000 - 0x1002FFFF (GPIO)
Slave 3:  0x10030000 - 0x1003FFFF (I2C)
Slave 4:  0x10040000 - 0x1004FFFF (SPI)
Slave 5:  0x10050000 - 0x1005FFFF (Timer0)
Slave 6:  0x10060000 - 0x1006FFFF (Timer1)
Slave 7:  0x10070000 - 0x1007FFFF (PWM)
Slave 8:  0x10080000 - 0x1008FFFF (ADC)
Slave 9:  0x10090000 - 0x1009FFFF (Watchdog)

LOC: ~2500 lines (estimated)

### Example 2: Multi-CPU System

Scenario: 4 CPUs accessing shared peripheral bus with 4 slaves

Command:

python generate_xbars.py --masters 4 --slaves 4

Generated File: apb_xbar_4to4.sv

Benefits: - Fair arbitration among 4 CPUs - Each slave independently arbitrated - Maximum throughput when CPUs access different slaves

LOC: ~1600 lines (estimated)

### Example 3: Memory-Mapped I/O Region

Scenario: Single master accessing I/O region at 0xA0000000

Command:

python generate_xbars.py --masters 1 --slaves 8 --base-addr 0xA0000000

Generated File: apb_xbar_1to8.sv

Address Map:

Slave 0:  0xA0000000 - 0xA000FFFF
Slave 1:  0xA0010000 - 0xA001FFFF
...
Slave 7:  0xA0070000 - 0xA007FFFF

## Using Generated Modules

### Integration Example

// Generated file: apb_xbar_3to6.sv

module my_soc (
    input  logic clk,
    input  logic rst_n,
    // ... other interfaces
);

    // Instantiate 3x6 crossbar
    apb_xbar_3to6 #(
        .ADDR_WIDTH(32),
        .DATA_WIDTH(32),
        .BASE_ADDR(32'h10000000)  // Can override at instantiation
    ) u_periph_xbar (
        .pclk(clk),
        .presetn(rst_n),

        // Connect 3 masters (CPU, DMA, Debug)
        .m0_apb_PSEL(cpu_apb_psel),
        .m0_apb_PENABLE(cpu_apb_penable),
        // ... (full CPU APB interface)

        .m1_apb_PSEL(dma_apb_psel),
        .m1_apb_PENABLE(dma_apb_penable),
        // ... (full DMA APB interface)

        .m2_apb_PSEL(debug_apb_psel),
        .m2_apb_PENABLE(debug_apb_penable),
        // ... (full Debug APB interface)

        // Connect 6 slaves (UART, GPIO, Timer, SPI, I2C, ADC)
        .s0_apb_PSEL(uart_psel),
        .s0_apb_PENABLE(uart_penable),
        // ... (full UART APB interface)

        .s1_apb_PSEL(gpio_psel),
        // ... (remaining slaves)
    );

endmodule : my_soc

### Parameter Override

// Use different base address at instantiation
apb_xbar_3to6 #(
    .BASE_ADDR(32'h80000000)  // Override default 0x10000000
) u_mmio_xbar (
    // ... connections
);
// Slaves now at: 0x80000000, 0x80010000, 0x80020000, ...

## Generator Limitations and Future Work

### Current Limitations

- Fixed 64KB Per Slave

  - Cannot customize individual slave region sizes

  - Workaround: Use multiple crossbars with different BASE_ADDR

- Max 16x16 Configuration

  - Practical limit due to arbitration complexity

  - Workaround: Hierarchical crossbars

- Single Address Map

  - All masters see same slave address map

  - Workaround: Address translation before crossbar

- No Priority Levels

  - Round-robin only, no fixed priority or weighted arbitration

  - Workaround: External priority arbitration before crossbar

### Future Enhancements

1. Variable Slave Sizes

# Potential future API
generate_apb_xbar(
    num_masters=2,
    slave_sizes=[4096, 65536, 1024, 16384],  # Custom sizes per slave
    base_addr=0x10000000
)

2. Arbitration Policies

# Potential future API
generate_apb_xbar(
    num_masters=4,
    num_slaves=4,
    arbitration="weighted",  # "round-robin", "fixed-priority", "weighted"
    weights=[4, 2, 1, 1]     # Master 0 gets 4x priority
)

3. Address Translation

# Potential future API
generate_apb_xbar(
    num_masters=2,
    num_slaves=4,
    master_views=[
        {"base": 0x00000000, "remap": True},   # Master 0 sees 0x0
        {"base": 0x10000000, "remap": False}   # Master 1 sees 0x1000_0000
    ]
)

## Testing Generated Code

### Verification Flow

1. Generate Crossbar

python generate_xbars.py --masters 2 --slaves 4

2. Lint Generated RTL

cd ../../rtl/
verilator --lint-only apb_xbar_2to4.sv

3. Run Functional Tests

cd ../../dv/tests/
pytest test_apb_xbar_2to4.py -v

4. View Waveforms (If Test Fails)

gtkwave waves.vcd

### Recommended Test Cases

- Single Master Access - Verify basic routing

- Multi-Master Contention - Verify arbitration

- Back-to-Back Transactions - Verify zero-bubble throughput

- Address Decode Boundary - Verify slave selection at boundaries

- Error Response Propagation - Verify PSLVERR routing

## Advanced Usage

### Using as Python Library

# Import generator function
from bin.rtl_generators.amba.apb_xbar_generator import generate_apb_xbar

# Generate crossbar programmatically
code = generate_apb_xbar(
    num_masters=3,
    num_slaves=8,
    base_addr=0xA0000000,
    addr_width=32,
    data_width=32,
    output_file="my_custom_xbar.sv"
)

# Generated code is returned as string
print(f"Generated {len(code)} bytes of SystemVerilog code")

# Can also write to file manually
with open("custom_output.sv", "w") as f:
    f.write(code)

### Batch Generation

# Generate multiple variants in one script
variants = [
    (1, 4, 0x10000000),
    (2, 4, 0x20000000),
    (3, 6, 0x30000000),
    (4, 8, 0x40000000),
]

for masters, slaves, base_addr in variants:
    code = generate_apb_xbar(
        num_masters=masters,
        num_slaves=slaves,
        base_addr=base_addr,
        output_file=f"xbar_{masters}to{slaves}_0x{base_addr:08X}.sv"
    )
    print(f"✓ Generated xbar_{masters}to{slaves}_0x{base_addr:08X}.sv")

## Troubleshooting

### Issue: “ModuleNotFoundError: No module named ‘amba.apb_xbar_generator’”

Cause: Python can’t find the core generator module

Solution:

# Verify generator exists
ls -la bin/apb_xbar_generator.py

# Run from bin directory
cd projects/components/apb_xbar/bin/
python generate_xbars.py

### Issue: Generated Code Won’t Lint

Cause: Generator bug or unsupported configuration

Solution: 1. Check parameter ranges (1-16 for M and N) 2. Verify base address alignment (should be aligned to 64KB × N) 3. Report bug with command that caused issue

### Issue: Generated Code Too Large

Cause: Very large MxN configuration (e.g., 16x16)

Expected Behavior: - 1x1: ~200 LOC - 2x4: ~1000 LOC - 16x16: ~25,000 LOC (practical limit)

Solution: - Consider hierarchical approach (multiple smaller crossbars) - Or use different interconnect topology (bus, tree, etc.)

## References

- Architecture Overview

- Address Decode and Arbitration

- PRD.md - Complete specification

- CLAUDE.md - Integration guidance

- README.md - Quick start guide

Version: 1.0 Last Updated: 2025-10-25 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Hardware Architecture Specification Index

Version: 1.0 Date: 2026-01-03 Purpose: High-level hardware architecture specification for APB Crossbar component

## Document Organization

Note: All chapters linked below for automated document generation.

### Front Matter

- Document Information

### Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### Chapter 3: Architecture

- Block Diagram

- Data Flow

- Address Mapping

### Chapter 4: Interfaces

- Master-Side APB Interfaces

- Slave-Side APB Interfaces

- Clock and Reset

### Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resource Estimates

### Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

## Related Documentation

- APB Crossbar MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Product Requirements Document (PRD)

## APB Crossbar Generator

Version: 1.0 Date: 2025-10-19 Status: Production Ready Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 1. Executive Summary

The APB Crossbar is a parametric APB interconnect generator that creates configurable MxN crossbar fabrics for connecting multiple APB masters to multiple APB slaves. Built using proven apb_slave and apb_master modules, the crossbar provides independent round-robin arbitration per slave and automatic address-based routing.

### 1.1 Quick Stats

- Modules: 5 pre-generated variants + generator for custom sizes

- Max Capacity: Up to 16x16 (configurable)

- Architecture: apb_slave → arbitration + decode → apb_master

- Status: Production ready, all tests passing

- Generator: Python-based parametric code generation

### 1.2 Project Goals

- Primary: Provide production-quality APB interconnect for SoC integration

- Secondary: Demonstrate parametric RTL generation best practices

- Tertiary: Support both fixed and dynamically-generated variants

## 2. Key Design Principles

### 2.1 Architecture Philosophy

Proven Building Blocks: - Uses apb_slave.sv and apb_master.sv from rtl/amba/apb - Each module independently tested and production-proven - Crossbar = composition of proven components

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants (1to1, 2to1, 1to4, 2to4, thin) - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol to cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp back to protocol

### 2.2 Design Trade-offs

| Feature | Choice | Rationale |
| --- | --- | --- |
| Arbitration | Round-robin per slave | Fair, simple, predictable |
| Address Map | 64KB per slave | Sufficient for most peripherals |
| Grant Persistence | Hold through response | Zero-bubble throughput |
| Address Decode | Parallel decode | Low latency, simple logic |

## 3. Architecture Overview

### 3.1 Top-Level Block Diagram

APB Crossbar (M masters × N slaves)
┌─────────────────────────────────────────────────────────────────┐
│                                                                  │
│  Master-Side (APB slaves)                                        │
│  ┌──────────────┐  ┌──────────────┐       ┌──────────────┐     │
│  │ apb_slave[0] │  │ apb_slave[1] │  ...  │ apb_slave[M-1]│     │
│  │ (Master 0)   │  │ (Master 1)   │       │ (Master M-1)  │     │
│  └──────┬───────┘  └──────┬───────┘       └──────┬────────┘     │
│         │                  │                      │              │
│         └──────────────────┴──────────────────────┘              │
│                            │                                     │
│                     cmd/rsp buses                                │
│                            │                                     │
│         ┌──────────────────┴──────────────────────┐              │
│         │                                          │              │
│  ┌──────▼──────┐  ┌──────────────┐  ┌────────────▼──┐          │
│  │ Address     │  │ Arbitration  │  │ Response       │          │
│  │ Decode      │  │ per Slave    │  │ Routing        │          │
│  │ (parallel)  │  │ (round-robin)│  │ (registered)   │          │
│  └──────┬──────┘  └──────┬───────┘  └────────┬───────┘          │
│         │                 │                   │                  │
│         └─────────────────┴───────────────────┘                  │
│                            │                                     │
│                     cmd/rsp buses                                │
│                            │                                     │
│         ┌──────────────────┴──────────────────────┐              │
│         │                  │                      │              │
│  ┌──────▼───────┐  ┌──────▼───────┐       ┌──────▼────────┐    │
│  │ apb_master[0]│  │ apb_master[1]│  ...  │ apb_master[N-1]│    │
│  │ (Slave 0)    │  │ (Slave 1)    │       │ (Slave N-1)    │    │
│  └──────────────┘  └──────────────┘       └───────────────┘    │
│                                                                  │
│  Slave-Side (APB masters)                                        │
└──────────────────────────────────────────────────────────────────┘

APB Crossbar Architecture (2x4 Example) Figure: APB Crossbar top-level architecture showing 2 masters connected to 4 slaves. Source: docs/apb_xbar_spec/assets/graphviz/apb_xbar_architecture.gv | SVG

### 3.2 Address Mapping

BASE_ADDR + 0x0000_0000 → 0x0000_FFFF : Slave 0 (64KB)
BASE_ADDR + 0x0001_0000 → 0x0001_FFFF : Slave 1 (64KB)
BASE_ADDR + 0x0002_0000 → 0x0002_FFFF : Slave 2 (64KB)
BASE_ADDR + 0x0003_0000 → 0x0003_FFFF : Slave 3 (64KB)
...
BASE_ADDR + 0x000F_0000 → 0x000F_FFFF : Slave 15 (64KB)

Default BASE_ADDR: 0x1000_0000

Address Decode Example:

The crossbar extracts the slave index from the upper bits of the address offset:

slave_index = (address - BASE_ADDR) >> 16  // Divide by 64KB (0x10000)

Address Decode Flow Figure: Address decode flow showing how address 0x10023456 routes to Slave 2. Source: docs/apb_xbar_spec/assets/graphviz/address_decode_flow.gv | SVG

### 3.3 Arbitration Strategy

Per-Slave Round-Robin: - Each slave has independent arbiter - Master priority rotates after each grant - Grant held from command acceptance through response completion - No master can starve another master

Example:

Slave 0 accessed by M0, M1, M0 → Next grant goes to M1
Slave 1 accessed by M1, M1, M0 → Next grant goes to M0

Round-Robin Arbitration Timing Figure: Round-robin arbitration timing showing 2 masters competing for Slave 0. Master priority rotates after each grant to ensure fair access. Source: docs/apb_xbar_spec/assets/wavedrom/arbitration_round_robin.json

## 4. Available Variants

### 4.1 Pre-Generated Modules

| Module | Masters | Slaves | Use Case | File Size |
| --- | --- | --- | --- | --- |
| apb_xbar_1to1 | 1 | 1 | Protocol conversion, testing | ~200 LOC |
| apb_xbar_2to1 | 2 | 1 | Multi-master arbitration | ~400 LOC |
| apb_xbar_1to4 | 1 | 4 | Address decode, simple SoC | ~500 LOC |
| apb_xbar_2to4 | 2 | 4 | Full crossbar, typical SoC | ~1000 LOC |
| apb_xbar_thin | 1 | 1 | Minimal passthrough | ~150 LOC |

### 4.2 Wrapper Modules

Pre-configured wrappers for common topologies:

| Wrapper | Configuration | Purpose |
| --- | --- | --- |
| apb_xbar_1to1_wrap | 1×1 | Simple connection |
| apb_xbar_2to1_wrap | 2×1 | Dual-master arbitration |
| apb_xbar_1to4_wrap | 1×4 | Single-master decode |
| apb_xbar_2to4_wrap | 2×4 | Full SoC crossbar |
| apb_xbar_wrap_m10_s10 | 10×10 | Large crossbar |
| apb_xbar_thin_wrap_m10_s10 | 10×10 | Minimal version |

## 5. Functional Requirements

### FR-1: Arbitrary MxN Configuration

Priority: P0 (Critical) Status: ✓ Implemented and verified

Description: Generate crossbar for any M masters and N slaves (up to 16x16)

Verification: Generator creates valid RTL for all tested configurations

### FR-2: Round-Robin Arbitration

Priority: P0 (Critical) Status: ✓ Implemented and verified

Description: Fair per-slave arbitration with rotating master priority

Verification: Arbitration stress tests (130+ transactions for 2to1)

### FR-3: Address-Based Routing

Priority: P0 (Critical) Status: ✓ Implemented and verified

Description: Automatic slave selection based on address ranges

Verification: Address decode validation (200+ transactions for 1to4)

### FR-4: Zero-Bubble Throughput

Priority: P1 (High) Status: ✓ Implemented and verified

Description: Back-to-back transactions supported without idle cycles

Verification: Performance tests show consecutive transactions

### FR-5: Configurable Address Map

Priority: P1 (High) Status: ✓ Implemented and verified

Description: BASE_ADDR parameter sets base of address map

Verification: Tests with multiple BASE_ADDR values

## 6. Interface Specifications

### 6.1 Master-Side Interface (APB Slave)

Per-master APB slave interface:

input  logic                    m<i>_apb_PSEL
input  logic                    m<i>_apb_PENABLE
input  logic [ADDR_WIDTH-1:0]   m<i>_apb_PADDR
input  logic                    m<i>_apb_PWRITE
input  logic [DATA_WIDTH-1:0]   m<i>_apb_PWDATA
input  logic [STRB_WIDTH-1:0]   m<i>_apb_PSTRB
input  logic [2:0]              m<i>_apb_PPROT
output logic [DATA_WIDTH-1:0]   m<i>_apb_PRDATA
output logic                    m<i>_apb_PSLVERR
output logic                    m<i>_apb_PREADY

### 6.2 Slave-Side Interface (APB Master)

Per-slave APB master interface:

output logic                    s<j>_apb_PSEL
output logic                    s<j>_apb_PENABLE
output logic [ADDR_WIDTH-1:0]   s<j>_apb_PADDR
output logic                    s<j>_apb_PWRITE
output logic [DATA_WIDTH-1:0]   s<j>_apb_PWDATA
output logic [STRB_WIDTH-1:0]   s<j>_apb_PSTRB
output logic [2:0]              s<j>_apb_PPROT
input  logic [DATA_WIDTH-1:0]   s<j>_apb_PRDATA
input  logic                    s<j>_apb_PSLVERR
input  logic                    s<j>_apb_PREADY

## 7. Parameter Configuration

| Parameter | Type | Default | Range | Description |
| --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | 1-64 | Address bus width |
| DATA_WIDTH | int | 32 | 8,16,32,64 | Data bus width |
| STRB_WIDTH | int | DATA_WIDTH/8 | - | Strobe width (auto-calc) |
| BASE_ADDR | logic[31:0] | 0x10000000 | Any | Base address for slave map |

## 8. Generator Usage

### 8.1 Pre-Generated Variants

Use existing modules directly:

# Available in projects/components/apb_xbar/rtl/
apb_xbar_1to1.sv
apb_xbar_2to1.sv
apb_xbar_1to4.sv
apb_xbar_2to4.sv
apb_xbar_thin.sv

### 8.2 Custom Generation

Generate custom MxN crossbar:

cd projects/components/apb_xbar/bin/
python generate_xbars.py --masters 3 --slaves 6
python generate_xbars.py --masters 4 --slaves 8 --base-addr 0x80000000

### 8.3 Generator Options

python generate_xbars.py --help

Options:
  --masters M       Number of masters (1-16)
  --slaves N        Number of slaves (1-16)
  --base-addr ADDR  Base address (default: 0x10000000)
  --output FILE     Output file path
  --thin            Generate thin variant (minimal logic)

## 9. Performance Characteristics

### 9.1 Latency

| Path | Latency | Notes |
| --- | --- | --- |
| Command path | 1 cycle | APB slave → decode → APB master |
| Response path | 1 cycle | APB master → routing → APB slave |
| Total | 2 cycles min | APB protocol overhead |

### 9.2 Throughput

- Back-to-back transactions: Supported

- Zero-bubble overhead: Yes (with grant persistence)

- Maximum rate: 1 transaction per 2 APB cycles per master

### 9.3 Resource Utilization (Estimated)

| Configuration | LUTs | FFs | Notes |
| --- | --- | --- | --- |
| 1to1 | ~50 | ~20 | Passthrough |
| 2to1 | ~150 | ~80 | Arbitration |
| 1to4 | ~200 | ~100 | Address decode |
| 2to4 | ~400 | ~200 | Full crossbar |
| 10to10 | ~5K | ~2K | Large crossbar |

## 10. Verification Status

### 10.1 Test Coverage

| Test | Transactions | Status | Coverage |
| --- | --- | --- | --- |
| test_apb_xbar_1to1 | 100+ | ✓ Pass | Basic connectivity |
| test_apb_xbar_2to1 | 130+ | ✓ Pass | Arbitration stress |
| test_apb_xbar_1to4 | 200+ | ✓ Pass | Address decode |
| test_apb_xbar_2to4 | 350+ | ✓ Pass | Full crossbar stress |

Overall: 100% passing, >750 total transactions tested

### 10.2 Test Methodology

Test Structure: - CocoTB framework - Random transaction generation - Variable delay profiles - Address range validation - Arbitration fairness checks

Verification Points: - Correct address routing - Round-robin arbitration - Response integrity - Back-to-back transactions - Error propagation

## 11. Integration Guide

### 11.1 Simple Integration (1to4)

apb_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master interface
    .m0_apb_PSEL(cpu_psel),
    .m0_apb_PENABLE(cpu_penable),
    .m0_apb_PADDR(cpu_paddr),
    .m0_apb_PWRITE(cpu_pwrite),
    .m0_apb_PWDATA(cpu_pwdata),
    .m0_apb_PSTRB(cpu_pstrb),
    .m0_apb_PPROT(cpu_pprot),
    .m0_apb_PRDATA(cpu_prdata),
    .m0_apb_PSLVERR(cpu_pslverr),
    .m0_apb_PREADY(cpu_pready),

    // Slave 0: UART (0x1000_0000 - 0x1000_FFFF)
    .s0_apb_PSEL(uart_psel),
    .s0_apb_PENABLE(uart_penable),
    // ... (similar connections)

    // Slave 1: GPIO (0x1001_0000 - 0x1001_FFFF)
    .s1_apb_PSEL(gpio_psel),
    // ... (similar connections)

    // Slave 2: Timer (0x1002_0000 - 0x1002_FFFF)
    // Slave 3: SPI (0x1003_0000 - 0x1003_FFFF)
);

### 11.2 Multi-Master Integration (2to4)

apb_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    // ... (full interface)

    // Master 1: DMA
    .m1_apb_PSEL(dma_psel),
    // ... (full interface)

    // Slaves 0-3: Peripherals
    // ... (slave connections)
);

## 12. Design Constraints

### 12.1 Limitations

| Constraint | Value | Rationale |
| --- | --- | --- |
| Max masters | 16 | Generator limit (configurable) |
| Max slaves | 16 | Generator limit (configurable) |
| Slave region size | 64KB | Fixed per slave |
| No slave disable | N/A | All slaves always active |
| No timeout | N/A | Assumes slaves always respond |

### 12.2 Assumptions

- Slave responses: All slaves respond eventually (no timeout handling)

- Address ranges: Slaves occupy 64KB regions starting at BASE_ADDR

- APB compliance: All masters and slaves follow APB protocol

- Single clock domain: All signals synchronous to pclk

## 13. Future Enhancements

### 13.1 Potential Features

- Configurable slave region sizes: Not just 64KB

- Slave enable/disable: Dynamic slave activation

- Timeout detection: Watchdog for hung slaves

- Transaction ordering: Optional in-order completion

- Priority arbitration: Weighted instead of round-robin

### 13.2 Generator Improvements

- HDL output formats: Support Verilog, VHDL, Chisel

- Automated testing: Generate tests alongside RTL

- Documentation generation: Auto-generate integration guides

- Synthesis scripts: Generate vendor-specific scripts

## 14. References

### 14.1 Internal Documentation

- README.md - Quick start guide

- CLAUDE.md - AI assistant guidelines

- bin/generate_xbars.py - Generator script

- dv/tests/ - Test implementations

### 14.2 External Standards

- AMBA APB Protocol v2.0 - ARM IHI 0024C

- APB4 Extensions - AMBA 5 specification

### 14.3 Related Components

- rtl/amba/apb/apb_slave.sv - Master-side building block

- rtl/amba/apb/apb_master.sv - Slave-side building block

- projects/components/apb_xbar/bin/apb_xbar_generator.py - Main generator

Document Version: 1.0 Last Review: 2025-10-19 Next Review: 2026-01-01 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Claude Code Guide: APB Crossbar Component

Version: 1.0 Last Updated: 2025-10-19 Purpose: AI-specific guidance for working with APB Crossbar component

## Quick Context

What: APB Crossbar Generator - Parametric APB interconnect for connecting M masters to N slaves Status: ✓ Production Ready (all tests passing) Your Role: Help users generate, integrate, and customize APB crossbars

📖 Complete Specification: projects/components/apb_xbar/PRD.md ← Always reference this for technical details

## Critical Rules for This Component

### Rule #0: This is a GENERATOR, Not Just Modules

IMPORTANT: APB Crossbar is both: 1. Pre-generated modules (1to1, 2to1, 1to4, 2to4, thin) in rtl/ 2. Python generator (bin/generate_xbars.py) for custom configurations

When users ask for crossbar: - ✓ Check if pre-generated variant exists first - ✓ Only suggest generation if custom size needed

Decision Tree:

User needs crossbar?
├─ 1x1, 2x1, 1x4, 2x4? → Use pre-generated module
├─ Thin/minimal? → Use apb_xbar_thin
└─ Custom MxN? → Run generator script

### Rule #1: Address Map is Fixed Per-Slave

Each slave occupies 64KB region:

Slave 0: BASE_ADDR + 0x0000_0000 → 0x0000_FFFF
Slave 1: BASE_ADDR + 0x0001_0000 → 0x0001_FFFF
Slave 2: BASE_ADDR + 0x0002_0000 → 0x0002_FFFF
...

Users CANNOT change per-slave size (current limitation)

If user asks for different sizes:

✗ WRONG: "Let me modify the generator to support custom sizes per slave"

✓ CORRECT: "Current design uses fixed 64KB per slave. You can:
1. Use BASE_ADDR parameter to shift entire map
2. For custom sizes, modify generator's addr_offset calculation
3. Or use multiple crossbars with different BASE_ADDR values"

### Rule #2: Know the Pre-Generated Variants

Available in rtl/ directory:

| Module | M×N | Use Case | When to Recommend |
| --- | --- | --- | --- |
| apb_xbar_1to1 | 1×1 | Passthrough | Protocol conversion, testing |
| apb_xbar_2to1 | 2×1 | Arbitration | Multi-master to single peripheral |
| apb_xbar_1to4 | 1×4 | Decode | Single CPU to multiple peripherals |
| apb_xbar_2to4 | 2×4 | Full crossbar | CPU + DMA to peripherals |
| apb_xbar_thin | 1×1 | Minimal | Low-overhead passthrough |

Always suggest pre-generated first!

### Rule #3: Generator Syntax

Generate custom crossbar:

cd projects/components/apb_xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

Options:

--masters M         Number of masters (1-16)
--slaves N          Number of slaves (1-16)
--base-addr ADDR    Base address (default: 0x10000000)
--output FILE       Output file path
--thin              Generate thin variant (minimal logic)

Example:

# Generate 3x8 crossbar with custom base
python generate_xbars.py --masters 3 --slaves 8 --base-addr 0x80000000

# Generate thin 5x5 variant
python generate_xbars.py --masters 5 --slaves 5 --thin

## Architecture Quick Reference

### Module Organization

projects/components/apb_xbar/
├── rtl/
│   ├── apb_xbar_1to1.sv        Core crossbar modules
│   ├── apb_xbar_2to1.sv
│   ├── apb_xbar_1to4.sv
│   ├── apb_xbar_2to4.sv
│   ├── apb_xbar_thin.sv
│   └── wrappers/               Pre-configured wrappers
│       ├── apb_xbar_1to1_wrap.sv
│       ├── apb_xbar_2to1_wrap.sv
│       ├── apb_xbar_1to4_wrap.sv
│       ├── apb_xbar_2to4_wrap.sv
│       ├── apb_xbar_wrap.sv
│       ├── apb_xbar_wrap_m10_s10.sv
│       └── apb_xbar_thin_wrap_m10_s10.sv
├── bin/
│   └── generate_xbars.py       Generator script
├── dv/
│   └── tests/
│       ├── test_apb_xbar_1to1.py
│       ├── test_apb_xbar_2to1.py
│       ├── test_apb_xbar_1to4.py
│       ├── test_apb_xbar_2to4.py
│       └── GTKW/                Waveform configs
├── PRD.md                       Complete specification
├── CLAUDE.md                    This file
└── README.md                    Quick start guide

### Block Diagram (2×4 Example)

Master 0      Master 1
               │             │
               ▼             ▼
         ┌─────────────────────┐
         │  APB Slaves (M-side) │
         └──────────┬────────────┘
                    │
         ┌──────────┴────────────┐
         │ Address Decode        │
         │ + Arbitration         │
         │ + Response Routing    │
         └──────────┬────────────┘
                    │
         ┌──────────┴────────────┐
         │  APB Masters (S-side) │
         └──────┬────────┬───────┘
                │        │   (more)
           Slave 0   Slave 1 ... Slave 3

## Common User Questions and Responses

### Q: “How do I connect a CPU to 4 peripherals?”

A: Use the pre-generated 1to4 crossbar:

apb_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)  // Start of peripheral space
) u_peripheral_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // CPU connection (master 0)
    .m0_apb_PSEL(cpu_psel),
    .m0_apb_PENABLE(cpu_penable),
    .m0_apb_PADDR(cpu_paddr),
    .m0_apb_PWRITE(cpu_pwrite),
    .m0_apb_PWDATA(cpu_pwdata),
    .m0_apb_PSTRB(cpu_pstrb),
    .m0_apb_PPROT(cpu_pprot),
    .m0_apb_PRDATA(cpu_prdata),
    .m0_apb_PSLVERR(cpu_pslverr),
    .m0_apb_PREADY(cpu_pready),

    // Peripheral 0: UART @ 0x1000_0000
    .s0_apb_PSEL(uart_psel),
    .s0_apb_PENABLE(uart_penable),
    // ... (full interface)

    // Peripheral 1: GPIO @ 0x1001_0000
    // Peripheral 2: Timer @ 0x1002_0000
    // Peripheral 3: SPI @ 0x1003_0000
);

Address map: - UART: 0x1000_0000 - 0x1000_FFFF - GPIO: 0x1001_0000 - 0x1001_FFFF - Timer: 0x1002_0000 - 0x1002_FFFF - SPI: 0x1003_0000 - 0x1003_FFFF

📖 See: PRD.md Section 11.1

### Q: “I need CPU and DMA to access peripherals. Which crossbar?”

A: Use the 2to4 crossbar:

apb_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_soc_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    // ... (full interface)

    // Master 1: DMA
    .m1_apb_PSEL(dma_psel),
    // ... (full interface)

    // Slaves 0-3: Peripherals
    // ... (slave connections)
);

Key feature: Round-robin arbitration per slave ensures fair access between CPU and DMA.

📖 See: PRD.md Section 11.2

### Q: “What if I need 3 masters and 8 slaves?”

A: Generate custom crossbar:

cd projects/components/apb_xbar/bin/
python generate_xbars.py --masters 3 --slaves 8 --output ../rtl/apb_xbar_3to8.sv

# Check generated file
ls -lh ../rtl/apb_xbar_3to8.sv

Then instantiate like any other crossbar:

apb_xbar_3to8 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_custom_xbar (
    // ... connections for 3 masters and 8 slaves
);

📖 See: PRD.md Section 8

### Q: “How does arbitration work?”

A: Per-slave round-robin:

Each slave has its own arbiter that rotates master priority:

Example with 2 masters accessing Slave 0:

Transaction 1: M0 requests → M0 granted
Transaction 2: M0 and M1 request → M1 granted (rotated)
Transaction 3: M1 requests → M1 granted
Transaction 4: M0 and M1 request → M0 granted (rotated)

Key points: - Independent per slave: Each slave arbitrates independently - Fair: No master can starve another - Grant persistence: Once granted, master holds slave until response completes - Zero-bubble: Back-to-back transactions supported

📖 See: PRD.md Section 3.3

### Q: “Can I change the address map?”

A: BASE_ADDR parameter shifts entire map, but per-slave size is fixed:

// Default: 0x1000_0000
apb_xbar_1to4 #(.BASE_ADDR(32'h1000_0000)) u_xbar1 (...);
// Slaves at: 0x1000_0000, 0x1001_0000, 0x1002_0000, 0x1003_0000

// Shifted: 0x8000_0000
apb_xbar_1to4 #(.BASE_ADDR(32'h8000_0000)) u_xbar2 (...);
// Slaves at: 0x8000_0000, 0x8001_0000, 0x8002_0000, 0x8003_0000

Limitation: Each slave always occupies 64KB (0x10000 bytes)

Workaround for different sizes: 1. Use multiple crossbars with different BASE_ADDR 2. Modify generator’s addr_offset calculation 3. Use address masking in slaves

📖 See: PRD.md Section 3.2

### Q: “How do I run tests?”

A: Use pytest:

cd projects/components/apb_xbar/dv/tests/

# Run specific test
pytest test_apb_xbar_1to4.py -v

# Run all tests
pytest test_apb_xbar_*.py -v

# Run with waveforms
pytest test_apb_xbar_2to4.py --vcd=waves.vcd
gtkwave waves.vcd

Test coverage: - test_apb_xbar_1to1: 100+ transactions (passthrough) - test_apb_xbar_2to1: 130+ transactions (arbitration) - test_apb_xbar_1to4: 200+ transactions (address decode) - test_apb_xbar_2to4: 350+ transactions (full stress)

All tests passing ✓

📖 See: PRD.md Section 10

### Q: “What’s the thin variant for?”

A: Minimal overhead passthrough:

apb_xbar_thin #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32)
) u_thin_xbar (
    // ... 1×1 connection
);

Use cases: - Protocol conversion - Timing boundary - Clock domain crossing preparation - Testing/debugging

Difference from apb_xbar_1to1: - Fewer internal registers - Lower latency - Smaller area (~30% reduction) - No arbitration overhead

📖 See: README.md and PRD.md Section 4

## Integration Patterns

### Pattern 1: Simple Peripheral Interconnect

// CPU to 4 peripherals
apb_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (
    .pclk(sys_clk),
    .presetn(sys_rst_n),

    // CPU side
    .m0_apb_PSEL(cpu_apb_psel),
    .m0_apb_PENABLE(cpu_apb_penable),
    .m0_apb_PADDR(cpu_apb_paddr),
    .m0_apb_PWRITE(cpu_apb_pwrite),
    .m0_apb_PWDATA(cpu_apb_pwdata),
    .m0_apb_PSTRB(cpu_apb_pstrb),
    .m0_apb_PPROT(cpu_apb_pprot),
    .m0_apb_PRDATA(cpu_apb_prdata),
    .m0_apb_PSLVERR(cpu_apb_pslverr),
    .m0_apb_PREADY(cpu_apb_pready),

    // Peripherals
    .s0_apb_PSEL(uart_psel), /* ... full interface ... */
    .s1_apb_PSEL(gpio_psel), /* ... */
    .s2_apb_PSEL(timer_psel), /* ... */
    .s3_apb_PSEL(spi_psel) /* ... */
);

### Pattern 2: Multi-Master System

// CPU + DMA to peripherals
apb_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h4000_0000)
) u_soc_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    /* ... full CPU interface ... */

    // Master 1: DMA Controller
    .m1_apb_PSEL(dma_psel),
    /* ... full DMA interface ... */

    // Slave 0-3: Memory-mapped peripherals
    .s0_apb_PSEL(mem_ctrl_psel), /* ... */
    .s1_apb_PSEL(uart_psel), /* ... */
    .s2_apb_PSEL(i2c_psel), /* ... */
    .s3_apb_PSEL(adc_psel) /* ... */
);

### Pattern 3: Hierarchical Interconnect

// Top-level crossbar
apb_xbar_1to4 u_top_xbar (
    .m0_apb_* (cpu_apb_*),
    .s0_apb_* (periph_bus0_*),  // To sub-crossbar 0
    .s1_apb_* (periph_bus1_*),  // To sub-crossbar 1
    .s2_apb_* (mem_ctrl_*),     // Direct to memory controller
    .s3_apb_* (dma_ctrl_*)      // Direct to DMA
);

// Sub-crossbar 0 for low-speed peripherals
apb_xbar_1to4 u_periph_xbar0 (
    .m0_apb_* (periph_bus0_*),
    .s0_apb_* (uart0_*),
    .s1_apb_* (gpio0_*),
    .s2_apb_* (i2c0_*),
    .s3_apb_* (spi0_*)
);

// Sub-crossbar 1 for high-speed peripherals
apb_xbar_1to4 u_periph_xbar1 (
    .m0_apb_* (periph_bus1_*),
    .s0_apb_* (uart1_*),
    .s1_apb_* (timer_*),
    .s2_apb_* (pwm_*),
    .s3_apb_* (adc_*)
);

## Anti-Patterns to Catch

### ✗ Anti-Pattern 1: Generating When Pre-Generated Exists

✗ WRONG:
User: "I need a 2×4 crossbar"
You: "Let me generate that for you..."
python generate_xbars.py --masters 2 --slaves 4

✓ CORRECTED:
"Use the pre-generated apb_xbar_2to4.sv in the rtl/ directory.
No generation needed!"

### ✗ Anti-Pattern 2: Assuming Custom Per-Slave Sizes

✗ WRONG:
User: "Can I make slave 0 256KB and slave 1 4KB?"
You: "Sure, let me modify the parameters..."

✓ CORRECTED:
"Current design uses fixed 64KB per slave. For custom sizes:
1. Modify generator's addr_offset calculation
2. Or use multiple crossbars with different BASE_ADDR
3. Or implement address masking in slaves"

### ✗ Anti-Pattern 3: Not Mentioning Address Map

✗ WRONG:
User: "How do I integrate the crossbar?"
You: *Shows port connections only*

✓ CORRECTED:
"Here's the integration with address map:
apb_xbar_1to4 #(.BASE_ADDR(32'h1000_0000)) u_xbar (...);

Address map:
- Slave 0: 0x1000_0000 - 0x1000_FFFF
- Slave 1: 0x1001_0000 - 0x1001_FFFF
- Slave 2: 0x1002_0000 - 0x1002_FFFF
- Slave 3: 0x1003_0000 - 0x1003_FFFF"

### ✗ Anti-Pattern 4: Forgetting About Wrappers

✗ WRONG:
User: "I need a quick 10×10 crossbar"
You: "Run the generator with --masters 10 --slaves 10"

✓ CORRECTED:
"We have pre-configured wrappers in rtl/wrappers/:
- apb_xbar_wrap_m10_s10.sv (full version)
- apb_xbar_thin_wrap_m10_s10.sv (thin version)

Use those for faster integration!"

## Debugging Workflow

### Issue: Address Not Routing Correctly

Check in order: 1. ✓ BASE_ADDR parameter set correctly? 2. ✓ Address within slave’s 64KB region? 3. ✓ PSEL signal asserted by master? 4. ✓ All APB signals properly connected?

Calculate expected slave:

slave_index = (address - BASE_ADDR) >> 16  // Divide by 64KB

Debug commands:

pytest dv/tests/test_apb_xbar_1to4.py --vcd=debug.vcd -v
gtkwave debug.vcd  # Check address decode logic

### Issue: Arbitration Not Fair

Symptoms: - One master dominates - Other masters starved

Check: 1. ✓ Arbiters instantiated per slave? 2. ✓ Round-robin logic correct? 3. ✓ Grant persistence working?

Verify with tests:

pytest dv/tests/test_apb_xbar_2to1.py -v  # Arbitration stress test

### Issue: Back-to-Back Transactions Stalling

Check: 1. ✓ Grant persistence enabled? 2. ✓ Slaves responding with PREADY? 3. ✓ No unintended pipeline bubbles?

View waveforms:

pytest dv/tests/test_apb_xbar_2to4.py --vcd=perf.vcd
gtkwave perf.vcd  # Check for idle cycles

## Quick Commands

# List available crossbars
ls projects/components/apb_xbar/rtl/*.sv

# Generate custom crossbar
cd projects/components/apb_xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

# Run all tests
cd projects/components/apb_xbar/dv/tests/
pytest test_apb_xbar_*.py -v

# Run specific test with waveforms
pytest test_apb_xbar_2to4.py --vcd=debug.vcd -v

# View waveforms
gtkwave debug.vcd

# Check documentation
cat projects/components/apb_xbar/PRD.md
cat projects/components/apb_xbar/README.md

## Remember

- 🔍 Check pre-generated first - Don’t generate unnecessarily

- 📍 Address map matters - Always mention BASE_ADDR + 64KB regions

- ⚖️ Fair arbitration - Round-robin per slave

- 🔗 Complete connections - All APB signals must be wired

- ✓ Tests available - 100% passing, comprehensive coverage

- 📚 Wrappers exist - Check rtl/wrappers/ for common configs

- 🎯 Generator limits - Up to 16×16 (configurable)

Version: 1.0 Last Updated: 2025-10-19 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Modules

This directory contains APB crossbar modules generated using the proven apb_slave and apb_master architecture.

## Architecture

All crossbars follow a consistent design:

Master Side:     apb_slave modules convert APB → cmd/rsp interface
Internal:        Round-robin arbitration + address decoding
Slave Side:      apb_master modules convert cmd/rsp → APB interface

### Key Features

- Independent arbitration per slave: Each slave has its own round-robin arbiter

- Grant persistence: Grants held from command acceptance through response completion

- Address decoding: Automatic slave selection based on address ranges

- Proven architecture: Uses production-tested apb_slave/apb_master modules

## Available Modules

| Module | Description | Masters | Slaves | Use Case |
| --- | --- | --- | --- | --- |
| apb_xbar_1to1.sv | Simple passthrough | 1 | 1 | Protocol conversion, testing |
| apb_xbar_2to1.sv | Arbitration only | 2 | 1 | Multi-master to single slave |
| apb_xbar_1to4.sv | Address decode only | 1 | 4 | Single master to multi-slave |
| apb_xbar_2to4.sv | Full crossbar | 2 | 4 | Multi-master to multi-slave |

## Generating Crossbars

### Method 1: Convenience Script (Recommended)

Generate all standard variants:

cd rtl/amba/apb/xbar/
python generate_xbars.py

Generate custom variant:

python generate_xbars.py --masters 3 --slaves 6
python generate_xbars.py --masters 4 --slaves 8 --base-addr 0x80000000

### Method 2: Direct Generator

Use the main generator for more control:

cd bin/rtl_generators/amba/
python apb_xbar_generator.py --masters 2 --slaves 4 --output ../../../rtl/amba/apb/xbar/apb_xbar_2to4.sv

## Address Map

All crossbars use a uniform address mapping (configurable via BASE_ADDR parameter):

Slave 0: [BASE_ADDR + 0x0000_0000, BASE_ADDR + 0x0000_FFFF]  (64KB)
Slave 1: [BASE_ADDR + 0x0001_0000, BASE_ADDR + 0x0001_FFFF]  (64KB)
Slave 2: [BASE_ADDR + 0x0002_0000, BASE_ADDR + 0x0002_FFFF]  (64KB)
Slave 3: [BASE_ADDR + 0x0003_0000, BASE_ADDR + 0x0003_FFFF]  (64KB)
...

Default BASE_ADDR: 0x1000_0000

## Parameters

All modules support these parameters:

| Parameter | Default | Description |
| --- | --- | --- |
| ADDR_WIDTH | 32 | Address bus width |
| DATA_WIDTH | 32 | Data bus width |
| STRB_WIDTH | DATA_WIDTH/8 | Strobe width (auto-calculated) |
| BASE_ADDR | 0x10000000 | Base address for slave address map |

## Usage Example

apb_xbar_2to4 #(
    .ADDR_WIDTH (32),
    .DATA_WIDTH (32),
    .BASE_ADDR  (32'h8000_0000)
) u_xbar (
    .pclk       (apb_clk),
    .presetn    (apb_rst_n),

    // Master 0 interface
    .m0_apb_PSEL    (m0_psel),
    .m0_apb_PENABLE (m0_penable),
    .m0_apb_PADDR   (m0_paddr),
    .m0_apb_PWRITE  (m0_pwrite),
    .m0_apb_PWDATA  (m0_pwdata),
    .m0_apb_PSTRB   (m0_pstrb),
    .m0_apb_PPROT   (m0_pprot),
    .m0_apb_PRDATA  (m0_prdata),
    .m0_apb_PSLVERR (m0_pslverr),
    .m0_apb_PREADY  (m0_pready),

    // Master 1 interface
    .m1_apb_PSEL    (m1_psel),
    // ... (similar connections)

    // Slave 0-3 interfaces
    .s0_apb_PSEL    (s0_psel),
    // ... (similar connections)
);

## Testing

All generated crossbars have corresponding test files in val/integ_amba/:

- test_apb_xbar_1to1.py - 100+ transactions, variable delay profiles

- test_apb_xbar_2to1.py - 130+ transactions, arbitration stress tests

- test_apb_xbar_1to4.py - 200+ transactions, address decode validation

- test_apb_xbar_2to4.py - 350+ transactions, full crossbar stress

Run tests:

pytest val/integ_amba/test_apb_xbar_2to4.py -v
pytest val/integ_amba/test_apb_xbar_*.py -v  # All variants

## Design Notes

### Arbitration Strategy

- Round-robin per slave: Master priority rotates (M0→M1→M0…)

- Grant persistence: Once granted, master owns slave until transaction completes

- Fairness: No master can starve another master

### Address Decoding

- Parallel decode: All masters decode addresses simultaneously

- Registered routing: Slave selection registered at command acceptance

- Response routing: Based on registered slave selection

### Timing

- Zero bubble overhead: Back-to-back transactions supported

- Single-cycle arbitration: New grants issued in same cycle as previous completion

- Pipelined datapath: Command and response phases overlap different transactions

## Known Limitations

- Fixed address map: 64KB regions per slave

  - Can be changed by modifying generator’s addr_offset calculation

- No slave disable: All slaves always active

  - Could add enable parameter if needed

- No timeout handling: Assumes slaves always respond

  - Add watchdog if needed for unreliable slaves

## Generating Custom Variants

For variants beyond 16x16, modify generator limits in apb_xbar_generator.py:

if M < 1 or M > 16:  # Change 16 to desired max
    raise ValueError(f"Number of masters must be 1-16, got {M}")

## Files

- generate_xbars.py - Convenience script for generation

- apb_xbar_1to1.sv - 1-to-1 passthrough

- apb_xbar_2to1.sv - 2-to-1 with arbitration

- apb_xbar_1to4.sv - 1-to-4 with address decode

- apb_xbar_2to4.sv - 2-to-4 full crossbar

- README.md - This file

## References

- APB Specification: ARM IHI 0024C (AMBA APB Protocol v2.0)

- Generator: projects/components/apb_xbar/bin/apb_xbar_generator.py

- Base modules: rtl/amba/apb/apb_slave.sv, rtl/amba/apb/apb_master.sv

- Tests: val/integ_amba/test_apb_xbar_*.py

Generated by RTL Design Sherpa | Last Updated: 2025-10-14