RTL Design Sherpa

Bridge Generator Architecture Specification 1.2

October 16, 2026

Table of Contents

1 Bridge Gas Index	11

2 Document Information	12

2.1 Bridge Generator Architecture Specification	12

2.2 Revision History	13

2.3 Document Purpose	16

2.4 Intended Audience	17

2.5 Related Documents	18

3 What the Generator Produces	19

3.1 Generated Artifact Family	20

3.2 What Each Variant Means	21

3.3 Scope of Generation	22

4 When to Regenerate	23

4.1 Triggers for Full Regeneration	24

4.2 Regeneration Workflow	25

4.3 When Inputs (Not Code) Change	26

4.4 Identifying Generated Files	27

5 Generator Entry Points	28

5.1 Single-Bridge Mode	29

5.2 Bulk Mode	30

5.3 Makefile Targets	31

5.4 Return Value	32

6 Bridge-Level TOML Keys	33

6.1 [bridge] scalar keys	34

6.2 [bridge.clock] table	35

6.3 Crossbar and arbitration keys	36

6.4 Monitor and cfg subsystem keys	38

6.5 [bridge.mon_group] table	40

6.6 [bridge.defaults] table	41

7 Port Specification Schema	42

7.1 Common port fields	43

7.2 Slave-only fields	45

7.3 Optional behavior fields	46

7.4 axi5_features values	48

7.5 Channel semantics	49

7.6 Example	50

8 Interface Wrapper Configuration	51

8.1 interface.type values	52

8.2 interface.skid_depths table	53

8.3 latency (port key)	54

8.4 How the wrapper selection flows	55

8.5 Protocol-specific wrapper notes	56

9 Monitor and CFG Subsystem Keys	57

9.1 Monitor subsystem	58

9.1.1 Bridge-level monitor keys	58

9.1.2 Per-port monitor keys	58

9.1.3 [bridge.mon_group] table	59

9.2 CFG register block	60

9.2.1 What changes	60

9.2.2 Per-port cfg fields	60

10 Validation Rules	62

10.1 Field validation (validate_required_fields)	63

10.2 Channel and protocol validation	64

10.3 Protocol-specific constraints	65

10.3.1 APB / APB5 (validate_apb_constraints)	65

10.3.2 Wishbone B4 (validate_wb4_constraints)	65

10.3.3 AXI3 (validate_axi3)	65

10.3.4 AXI5 (validate_axi5)	65

10.4 Master-slave compatibility (validate_master_slave_compatibility)	66

10.5 Address map validation (validate_address_map)	67

10.6 Slave ID width (validate_slave_id_widths)	68

10.7 CDC constraints (validate_cdc_constraints)	69

10.8 Warning for dropped sideband (warn_axi5_dropped_sideband)	70

11 Pipeline Overview	71

11.1 Pipeline diagram	72

11.1.1 Figure 3.1: Bridge Generator Pipeline	72

11.2 Key files by stage	73

12 Configuration Loading	75

12.1 TOML parsing	76

12.2 Connectivity CSV	77

12.3 BridgeConfig construction	78

12.4 Normalization	79

13 Validation	80

13.1 Validation order	81

13.2 Error behavior	82

13.3 Examples of validation failures	83

13.4 Adding a new validation rule	84

14 Adapter and Crossbar Emission	85

14.1 Translation to emitter structs	86

14.2 BridgeModuleGenerator construction	87

14.3 Sub-generators	88

14.4 Master adapter (AdapterGenerator)	89

14.5 Slave adapter (SlaveAdapterGenerator)	90

14.6 Crossbar (CrossbarGenerator)	91

15 Filelist and Test Generation	92

15.1 Filelist generation	93

15.1.1 Filelist sections	93

15.2 Test generation	94

16 Directory Layout	95

16.1 Variant directories	96

17 Package File	97

17.1 What it contains	98

17.2 Excerpt from a generated package	99

17.3 Why it matters	100

18 Top Module	101

18.1 Module signature	102

18.2 Internal structure	103

18.3 Excerpt: subtractive catch-all instantiation	104

18.4 Excerpt: adapter wiring pattern	105

19 Adapters and Crossbar	106

19.1 Master adapter (<name>_adapter.sv)	106

19.1.1 Per-width paths	106

19.2 Slave adapter (<name>_adapter.sv)	107

19.2.1 Response routing	107

19.3 Crossbar (<bridge_name>_xbar.sv)	108

20 Filelist	109

20.1 Path convention	110

20.2 Example filelist head	111

20.3 Filelist is generated, not hand-maintained	112

21 Generated Tests	113

21.1 TB class	114

21.2 Test runner	115

21.3 Monitor stress test	116

21.4 Regeneration discipline for tests	117

22 Full Regeneration Rule	118

22.1 Why partial regeneration is dangerous	119

22.2 Required workflow	120

22.3 Code changes that trigger full regeneration	121

22.4 Files that are never regenerated	122

22.5 Version-mismatch symptoms	123

23 Generated Examples	124

23.1 Representative fixtures	125

23.2 How to verify a regeneration	126

23.3 Fixture as documentation	127

24 DV Suite	128

24.1 Test structure	129

24.2 Running tests	130

24.3 Test levels	131

24.4 Verification coverage	132

24.5 When generator changes affect DV	133

25 Lint and Synthesis Checks	134

25.1 Verilator lint	135

25.2 Out-of-context synthesis	136

25.3 When to run checks	137

26 Adding a Protocol	138

26.1 Step 1: Add the protocol to the validator	139

26.2 Step 2: Add converter/wrapper selection	140

26.3 Step 3: Update the filelist	141

26.4 Step 4: Add a representative fixture	142

26.5 Step 5: Run the full regeneration and test	143

26.6 Example: how APB was added	144

27 Adding a Bridge-Level Option	145

27.1 Step 1: Add the dataclass field	146

27.2 Step 2: Add the TOML loader mapping	147

27.3 Step 3: Validate if necessary	148

27.4 Step 4: Thread it into the emitter	149

27.5 Step 5: Use the option in BridgeModuleGenerator	150

27.6 Step 6: Document and test	151

27.7 Example: xbar_pipeline	152

28 Adding a Port-Level Option	153

28.1 Step 1: Add the PortSpec field	154

28.2 Step 2: Load it from TOML	155

28.3 Step 3: Validate it	156

28.4 Step 4: Thread it into MasterConfig / SlaveInfo	157

28.5 Step 5: Use it in the adapter generator	158

28.6 Step 6: Update the filelist if needed	159

28.7 Example: cdc	160

29 Testing an Extension	161

29.1 Minimum test plan	162

29.2 Regression fixtures	163

29.3 Documentation	164

List of Figures

Figure 3.1: Bridge Generator Pipeline	72

List of Tables

Table 1: Document information	12

Table 2: Revision history	13

Table 3: Generated Artifact Family	20

Table 4: Identifying Generated Files	27

Table 5: Single-Bridge Mode	29

Table 6: Makefile Targets	31

Table 7: [bridge] scalar keys	34

Table 8: [bridge.clock] table	35

Table 9: Crossbar and arbitration keys	36

Table 10: Monitor and cfg subsystem keys	38

Table 11: [bridge.mon_group] table	40

Table 12: Common port fields	43

Table 13: Slave-only fields	45

Table 14: Optional behavior fields	46

Table 15: Channel semantics	49

Table 16: interface.type values	52

Table 17: interface.skid_depths table	53

Table 18: latency modes	54

Table 19: Bridge-level monitor keys	58

Table 20: Per-port monitor keys	58

Table 21: Per-port cfg fields	60

Table 22: Key files by stage	73

Table 23: Examples of validation failures	83

Table 24: Translation to emitter structs	86

Table 25: Sub-generators	88

Table 26: Test generation	94

Table 27: Representative fixtures	125

Table 28: Test levels	131

# 1	Bridge Gas Index

Generated: 2026-10-16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Bridge Generator Architecture Specification

Document information

| Property | Value |
| --- | --- |
| Document Title | Bridge Generator Architecture Specification |
| Version | 1.2 |
| Date | 2026-10-16 |
| Status | Released |
| Classification | Open Source - MIT License |

## 2.2	Revision History

Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-10-12 | RTL Design Sherpa | Initial GAS release — generator code paths verified against bridge_generator.py, bridge_pkg/config.py, bridge_pkg/config_validator.py, bridge_pkg/config_loader.py, and emitted artifacts in rtl/generated/bridge_mix_e/. |
| 1.1 (renumbered) | 2026-10-15 | RTL Design Sherpa | The bridge document set (GAS, HAS, MAS) is renumbered to one common rev, 1.1; rows above keep the numbers they were published under. Content: the one-target-per-master limitation is fixed (GitHub #119 L1) – per-ID target tracking ([bridge].id_track_bits), B/R response arbiters per master width path, the crossed-W hold; the generated TB carries its connectivity map and the generator emits the AXIL narrow-to-wide alignment test (GitHub #119 L8); per-port outstanding limits, max_outstanding (power of two 2..64, default 16), size every tracker – the fixed 16 removed (GitHub #119 L5); per-port latency modes (cut_all / cut_ax / cut_none) and honoured TOML skid depths (GitHub #119 L7); read-return-atomic and trace-echo ports no longer limited to one target (GitHub #119 L1b). |
| 1.2 | 2026-10-16 | RTL Design Sherpa | Slave-adapter CAM pipelining (GitHub #119): bridge_cam gains PIPELINE_ALLOC (registered insert with a pending-entry bypass; the full flag counts the pending entry) and PIPELINE_FREE (registered free; lookups see the post-free state), both on in every CAM-tracked slave adapter – MAS Figure 4.2. HAS 5.1 adds the Genesys 2 fabric-benchmark results against the Xilinx AXI Crossbar and SmartConnect; HAS 5.3 / 6.4 reference synthesis tables re-measured on the current generator. CDC on master ports (GitHub #119 L2): cdc = true on an AXI4 master puts axi4_cdc_{wr,rd} between the port and its timing wrapper, with <master>_aclk / <master>_aresetn on the top (HAS 4.5a, MAS 6.1); new fixture bridge_3x2_rw_mcdc and test_bridge_3x2_rw_mcdc_ratio. Generated TBs build their protocol checkers after reset. Read width converters are built with R_INTERLEAVE = 0 (no reassembly pool; it had grown every converter fixture past synthesis), so a slave may not interleave R beats across IDs (HAS 4.1, PRD FR-9). |

## 2.3	Document Purpose

This Generator Architecture Specification (GAS) is the manual for the Bridge code generator. It is written for the engineer who must:

- re-run the generator after a bugfix,

- add a new protocol or port option,

- debug a generated module,

- or audit why a particular file was emitted.

It covers the full generator surface: CLI flags, TOML keys, Python data classes, validation rules, emission stages, output files, and the extension points that are safe to touch.

For the hardware that the generator produces, see the Bridge HAS and Bridge MAS. For product requirements, see PRD.md.

## 2.4	Intended Audience

- RTL engineers modifying or extending the generator

- Verification engineers regenerating fixtures after a generator change

- System architects adding new bridge configurations to the batch

- Integrators debugging a generated bridge top

## 2.5	Related Documents

- Bridge HAS - Hardware Architecture Specification

- Bridge MAS - Micro-Architecture Specification

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide and subsystem rules

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	What the Generator Produces

The Bridge generator (bin/bridge_generator.py) emits a complete, synthesizable AXI4/AXI5 crossbar from a TOML port description and a CSV connectivity matrix. The output is not a parameterized Verilog template: every width, count, and protocol conversion is resolved at generation time, and the top module exposes no parameters.

## 3.1	Generated Artifact Family

For each requested variant (no or mon), the generator writes a directory under rtl/generated/<bridge_name>/ containing:

Generated Artifact Family

| File | Stamped generator |
| --- | --- |
| <bridge_name>_pkg.sv | PackageGenerator |
| <bridge_name>.sv | BridgeModuleGenerator |
| <bridge_name>_xbar.sv | CrossbarGenerator |
| <master>_adapter.sv | AdapterGenerator (per master) |
| <slave>_adapter.sv | SlaveAdapterGenerator (per slave) |
| <bridge_name>.f | bridge_generator.py (filelist) |
| <bridge_name>.toml | copied source config |
| <bridge_name>_connectivity.csv | copied connectivity matrix |
| __cfg_*_pkg.sv / <bridge_name>_cfg.sv | CfgRdlGenerator (only when use_cfg_regblock = true) |

The header comment in each .sv file names the generator class that produced it. This is the fastest way to tell whether a file is generated and which part of the pipeline owns it.

## 3.2	What Each Variant Means

The TOML [bridge].variants list selects which variants to emit:

- "no" — bare bridge, no monitor wrapper instances.

- "mon" — bridge with _monlite wrappers on every enabled port, plus the monbus arbiter and an internal AXIL4/AXIL4 group (unless internal_axil_group = false).

Both variants are fully independent; each gets its own directory and its own filelist.

## 3.3	Scope of Generation

The generator resolves:

- Port prefixes, channel subsets (rw/wr/rd), widths, and protocol variants.

- Width converters (axi4_dwidth_converter_{wr,rd}) inserted only where master and slave data widths differ.

- Protocol shims (axi4_to_apb4_shim, axi4_to_apb5_shim, axi4_to_axil4_*, axi4_to_axil5_*, axi4_to_axi3, axi4_to_wb4, AXI5 boundary wrappers) selected by the port protocol field.

- The subtractive catch-all slave (axi4_subtractive_slave), always present and wired to unmapped addresses.

- Optional monitor wrappers and a cfg register block.

The generator does not emit testbench code unless --generate-tests is passed.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	When to Regenerate

Any change to the generator or its inputs can change every emitted file. Bridge enforces a full regeneration discipline.

## 4.1	Triggers for Full Regeneration

Regenerate every generated bridge in rtl/generated/ when any of the following change:

- bin/bridge_generator.py

- Any Python file under bin/bridge_pkg/

- Any Jinja template under bin/bridge_pkg/jinja_templates/

- Any shared converter or wrapper IP the filelist pulls in, if its interface changes

Do not regenerate a single bridge and leave the rest untouched. Generated files share package definitions, converter selection logic, and filelist conventions; partial regeneration creates version skew.

## 4.2	Regeneration Workflow

cd projects/components/fabric-gen-ip/bridge/bin
make clean          # removes rtl/generated/
make all            # regenerates every bridge in bridge_batch.csv

After regenerating RTL, also regenerate tests if the generator’s test templates changed:

python bridge_generator.py --bulk bridge_batch.csv --generate-tests

Then run the DV suite from a clean build directory:

cd ../dv/tests
make clean-all && make run-all-func

## 4.3	When Inputs (Not Code) Change

If only the TOML or CSV for one bridge changes, regenerate that bridge alone is safe only when no generator code changed:

python bridge_generator.py --ports test_configs/my_bridge.toml

If in doubt, run make clean && make all.

## 4.4	Identifying Generated Files

Each generator class stamps its own header. Grepping for the generator name is more reliable than grepping for bridge_generator.py:

Identifying Generated Files

| Generator class | Header string |
| --- | --- |
| BridgeModuleGenerator | Generated by: BridgeModuleGenerator |
| PackageGenerator | Generated by: PackageGenerator |
| CrossbarGenerator | Generated by: CrossbarGenerator |
| AdapterGenerator | Generated by: AdapterGenerator |
| SlaveAdapterGenerator | Generated by: SlaveAdapterGenerator |

Hand-written files in rtl/ (for example bridge_cam.sv) are never regenerated and must not be deleted by make clean.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Generator Entry Points

Bridge has two invocation modes: single-bridge and bulk.

## 5.1	Single-Bridge Mode

cd projects/components/fabric-gen-ip/bridge/bin
python3 bridge_generator.py \
    --ports test_configs/bridge_2x2_rw.toml \
    [--connectivity test_configs/bridge_2x2_rw_connectivity.csv] \
    [--name bridge_2x2_rw] \
    [--output-dir ../rtl/generated]

For TOML inputs, --connectivity is optional: the loader auto-detects <bridge_name>_connectivity.csv next to the TOML file. CSV port files are no longer supported.

CLI flags:

Single-Bridge Mode

| Flag | Default | Purpose |
| --- | --- | --- |
| --ports | required | Path to the TOML port configuration |
| --connectivity | auto | Path to the CSV connectivity matrix |
| --name | from TOML or auto | Output module base name |
| --output-dir | ../rtl/generated | Where to write the bridge directory |
| --generate-tests | off | Also emit cocotb TB class and test runner |
| --output-tb | ../dv/tbclasses | TB class output directory |
| --output-test | ../dv/tests | Test runner output directory |

## 5.2	Bulk Mode

python3 bridge_generator.py --bulk bridge_batch.csv

The batch CSV columns are:

name,ports,connectivity,output_dir,output_tb,output_test

Comment lines starting with # are ignored. The name and connectivity fields may be empty; empty connectivity triggers auto-detection from the TOML filename.

## 5.3	Makefile Targets

The bin/Makefile wraps the bulk path:

Makefile Targets

| Target | Action |
| --- | --- |
| make all | bridge_generator.py --bulk bridge_batch.csv |
| make clean | remove rtl/generated/ and rtl/filelists/ |
| make regen | alias for make clean && make all |

## 5.4	Return Value

generate_bridge() returns a tuple (success: bool, emitted_variants: list[tuple[str, bool]]). A non-empty emitted_variants list means at least one variant was written. The bulk driver exits non-zero if any bridge or test generation fails.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 6	Bridge-Level TOML Keys

The TOML [bridge] table configures the bridge as a whole. It is parsed by bridge_pkg.config_loader.load_config() into the BridgeConfig dataclass (bridge_pkg/config.py).

## 6.1	[bridge] scalar keys

[bridge] scalar keys

| Key | Type | Default | Dataclass field | Description |
| --- | --- | --- | --- | --- |
| name | string | "" | BridgeConfig.name | Output base name. If empty, the generator auto-names from topology (bridge_<M>x<S>_<channels>). |
| variants | list of strings | ["no"] | BridgeConfig.variants | Build variants to emit. Each entry must be "no" or "mon". At least one entry is required in TOML. |
| description | string | "" | ignored | Human-readable comment; copied into generated header only. |

## 6.2	[bridge.clock] table

[bridge.clock] table

| Key | Type | Default | Used by | Description |
| --- | --- | --- | --- | --- |
| name | string | "aclk" | header comment | Clock signal name (informational). |
| frequency_mhz | int | none | header comment | Clock frequency (informational). |

## 6.3	Crossbar and arbitration keys

Crossbar and arbitration keys

| Key | Type | Default | Dataclass field | Description |
| --- | --- | --- | --- | --- |
| xbar_pipeline | boolean | false | BridgeConfig.xbar_pipeline | Add skid register stage on every slave-side channel inside the crossbar. |
| arbitration | string | "rr" | BridgeConfig.arbitration | Per-slave arbitration policy. "rr" = round-robin; "qos" = QoS + aging. |
| qos_aging_shift | int | 4 | BridgeConfig.qos_aging_shift | For "qos", a waiting request gains one priority level every 2**shift cycles. |
| id_track_bits | int 0..8 | 4 | BridgeConfig.id_track_bits | Per-ID target table in every master adapter: 2**id_track_bits entries by the ID’s low bits (never more than the port’s ID). A request waits only while its ID is in flight to a different slave. Fixes the one-target-per-master limitation (GitHub #119 L1). Out of range is a ValueError at load. |
| max_outstanding | int, power of two 2..64 | 16 | PortSpec.max_outstanding (default for every port) | Outstanding-transaction limit per direction for every port that does not set its own: master-adapter tracker depth and per-ID count width; slave-adapter bridge_cam / in-order FIFO depth; the crossbar’s W-destination (per master path) and W-owner (per slave, plus 2 on a registered crossbar) FIFOs follow. Fixes the fixed 16 (GitHub #119 L5). |

## 6.4	Monitor and cfg subsystem keys

Monitor and cfg subsystem keys

| Key | Type | Default | Dataclass field | Description |
| --- | --- | --- | --- | --- |
| mon_preset | string | "lite" | BridgeConfig.mon_preset | Baseline monitor reporter enables. Accepted aliases: "all", "error_only", "functional", "none", "lite". Since 2026-09-26 every monitored port uses axi_monitor_lite, so the preset resolves to the lite set regardless. |
| use_all_monitors | boolean | false | BridgeConfig.use_all_monitors | Force USE_MONITOR=1 on every port. Mutually exclusive with use_no_monitors. |
| use_no_monitors | boolean | false | BridgeConfig.use_no_monitors | Force USE_MONITOR=0 on every port. Mutually exclusive with use_all_monitors. |
| internal_axil_group | boolean | true | BridgeConfig.internal_axil_group | For "mon" variants, instantiate the internal monbus_axil4_axil4_group. When false, expose the aggregated monbus stream at the top. |
| use_cfg_regblock | boolean | false | BridgeConfig.use_cfg_regblock | Replace the 350+ individual cfg_* top-level pins with a PeakRDL-generated cfg register block. |

## 6.5	[bridge.mon_group] table

[bridge.mon_group] table

| Key | Type | Default | Dataclass field | Description |
| --- | --- | --- | --- | --- |
| slave_protocol | string | "axil4" | MonGroupConfig.slave_protocol | Protocol of the monbus group slave-read port. "axil4" or "axi4". Deprecated spelling "axil" is normalized to "axil4". |
| master_protocol | string | "axil4" | MonGroupConfig.master_protocol | Protocol of the monbus group master-write port. "axil4" or "axi4". |
| axi_id_width | int | 8 | MonGroupConfig.axi_id_width | AXI ID width when either side is axi4. |
| max_burst_beats | int | 64 | MonGroupConfig.max_burst_beats | Max write burst when master side is axi4. |
| fifo_depth_err | int | 64 | MonGroupConfig.fifo_depth_err | Error FIFO depth. |
| fifo_depth_write | int | 96 | MonGroupConfig.fifo_depth_write | Write-data FIFO depth in beats. |
| flush_timeout_cycles | int | 1024 | MonGroupConfig.flush_timeout_cycles | Flush timeout. |
| use_compression | int | 0 | MonGroupConfig.use_compression | Compression enable. |

## 6.6	[bridge.defaults] table

The defaults.skid_depths sub-table provides default skid depths for interface wrappers:

[bridge.defaults]
skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

Per-port interface.skid_depths overrides these defaults, and a port’s latency mode then sets the channels it cuts to 0 (pass-through). Valid per-channel depths are integers in the range 2..8 inclusive. Until GitHub #119 L7 (2026-10-15) both tables were parsed and ignored – every wrapper was built 2/4/2/2/2; they take effect now. Any other key under [bridge.defaults] is reported and ignored.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 7	Port Specification Schema

Each master and slave is described by a TOML table under [[bridge.masters]] or [[bridge.slaves]]. The loader converts each table into a PortSpec dataclass (bridge_pkg/config.py).

## 7.1	Common port fields

Common port fields

| Field | Type | Default | PortSpec attribute | Description |
| --- | --- | --- | --- | --- |
| name | string | required | port_name | Unique identifier used in generated module names and file names. |
| prefix | string | required | prefix | Signal prefix. The generator normalizes a missing trailing underscore (cpu_m_axi → cpu_m_axi_). |
| protocol | string | required | protocol | One of axi3, axi4, axi5, apb, apb5, axil, axil5, wb4. |
| channels | string | required | channels | One of rw, wr, rd. Slaves must declare what they support; the generator trims unneeded channels. |
| data_width | int | required | data_width | Data bus width in bits. |
| addr_width | int | required | addr_width | Address bus width in bits. |
| id_width | int | 0 | id_width | AXI ID width. Required > 0 for axi4/axi5 masters; must be 0 for APB, AXI-Lite, and Wishbone. |
| user_width | int | 0 | (passed to wrapper) | AXI4 user signal width. |
| cdc | boolean | false | cdc | Run this port on its own clock (<name>_aclk / <name>_aresetn) behind axi4_cdc_{wr,rd}: on a slave between the timing wrapper and the port, on a master between the port and the timing wrapper (GitHub #119 L2). protocol="axi4" only. |

## 7.2	Slave-only fields

Slave-only fields

| Field | Type | Default | PortSpec attribute | Description |
| --- | --- | --- | --- | --- |
| base_addr | hex string/int | required for slaves | base_addr | Start of the slave address window. Must be 4 KiB-aligned. |
| addr_range | hex string/int | required for slaves | addr_range | Size of the slave window. Must be a non-zero multiple of 4 KiB. |

## 7.3	Optional behavior fields

Optional behavior fields

| Field | Type | Default | PortSpec attribute | Description |
| --- | --- | --- | --- | --- |
| interface | table | none | see below | Timing-isolation wrapper configuration. |
| enable_ooo | boolean | false | enable_ooo | Mark slave as out-of-order capable. (No generated fixture currently uses this.) |
| max_outstanding | int, power of two 2..64 | [bridge].max_outstanding (16) | max_outstanding | This port’s outstanding limit per direction (master: tracker depth; slave: response-tracker depth). The internal subtractive slave stays at 16. |
| use_monitor | boolean | true | use_monitor | Per-port monitor enable; meaningful only for "mon" variants. |
| mon_add | list of strings | [] | mon_add | Additive reporter cones beyond the preset. Names come from MON_CONES: error, timeout, compl, threshold, perf, debug. |
| mon_remove | list of strings | [] | mon_remove | Reporter cones to remove from the preset. |
| axi5_features | list of strings | [] | axi5_features | AXI5 sideband feature list. Legal only on axi5 or axil5 ports. |
| internal | boolean | false | internal | Port has no top-level pins. Used automatically for the subtractive catch-all. |

## 7.4	axi5_features values

The validator (bridge_pkg/config_validator.py) accepts:

- Droppable sideband: nsaid, trace, mpam, mecid, unique, chunking.

- Connectivity-gated (every connected path must be native AXI5, same feature, same width): poison, atomic, mte.

On axil5 ports, only user and exclusive are forwardable; trace, loop, mpam, mecid, nsaid, poison are rejected because they have no AXI4 source/destination.

## 7.5	Channel semantics

Channel semantics

| channels | Master gets | Slave must support |
| --- | --- | --- |
| rw | AW, W, B, AR, R | AW, W, B, AR, R |
| wr | AW, W, B only | AW, W, B |
| rd | AR, R only | AR, R |

A wr master may connect to a wr or rw slave; a rd master may connect to a rd or rw slave. A wr master cannot connect to a rd slave, and vice versa.

## 7.6	Example

[[bridge.masters]]
name       = "cpu"
prefix     = "cpu_m_axi_"
protocol   = "axi4"
channels   = "rw"
id_width   = 4
addr_width = 32
data_width = 64
user_width = 1

[[bridge.slaves]]
name       = "ddr"
prefix     = "ddr_s_axi_"
protocol   = "axi4"
channels   = "rw"
id_width   = 6
addr_width = 32
data_width = 512
user_width = 1
base_addr  = "0x00000000"
addr_range = "0x80000000"

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 8	Interface Wrapper Configuration

The optional per-port interface table selects a timing-isolation wrapper and its skid depths. If omitted, the port is connected directly to the crossbar (no wrapper).

## 8.1	interface.type values

interface.type values

| Value | Wrapper instantiated |
| --- | --- |
| axi4_master | axi4_slave_wr / axi4_slave_rd (master port acts as AXI slave to external master) |
| axi4_slave | axi4_master_wr / axi4_master_rd (slave port acts as AXI master to external slave) |
| axi4_master_mon | _monlite variant of the master wrapper |
| axi4_slave_mon | _monlite variant of the slave wrapper |

The _monlite wrappers are used unconditionally since 2026-09-26; the old non-lite wrappers were retired.

## 8.2	interface.skid_depths table

interface.skid_depths table

| Key | Type | Default | Valid range | Description |
| --- | --- | --- | --- | --- |
| ar | int | from [bridge.defaults.skid_depths] | 2..8 | Read address skid depth. |
| r | int | from defaults | 2..8 | Read data skid depth. |
| aw | int | from defaults | 2..8 | Write address skid depth. |
| w | int | from defaults | 2..8 | Write data skid depth. |
| b | int | from defaults | 2..8 | Write response skid depth. |

## 8.3	latency (port key)

latency selects which channels of the port’s timing wrapper are registered (GitHub #119 L7). A cut channel’s skid depth becomes 0, which gaxi_skid_buffer builds as a combinational pass-through.

latency modes

| Value | Registered channels | Port adds |
| --- | --- | --- |
| cut_all (default) | AW, W, B, AR, R | 1 cycle per direction |
| cut_ax | AW, AR | 1 cycle on requests, 0 on W / B / R |
| cut_none | none | 0 cycles: the port boundary is wires |

Legal only on a same-clock axi4 / axi5 port; a converted port (AXI-Lite, APB, Wishbone, AXI3) or a cdc = true port has buffers of its own that the mode does not reach, so the loader rejects it there. The master adapter keeps the resolved depths as module parameters (SKID_DEPTH_*), so a single generated adapter can still be re-timed by overriding them at elaboration. Measured on bridge_2x2_rw_lat by test_bridge_2x2_rw_lat_modes.

Example:

[[bridge.masters]]
name = "dma"
prefix = "dma_m_axi_"
protocol = "axi4"
channels = "rw"
id_width = 4
addr_width = 32
data_width = 512
user_width = 1
interface = {type = "axi4_master", skid_depths = {ar = 4, r = 8, aw = 4, w = 8, b = 4}}

## 8.4	How the wrapper selection flows

- bridge_pkg/config_loader.py reads the interface table into PortSpec attributes.

- bridge_generator.py passes the port protocol and use_monitor flag to BridgeModuleGenerator.

- AdapterGenerator (bridge_pkg/generators/adapter_generator.py) and SlaveAdapterGenerator (bridge_pkg/generators/slave_adapter_generator.py) instantiate the appropriate wrapper module based on protocol and monitor state.

- The generated adapter module wires the wrapper’s fub_axi_* internal face to the crossbar.

## 8.5	Protocol-specific wrapper notes

- axi5 ports use axi5_slave_{wr,rd} on master sides and axi5_master_{wr,rd} on slave sides.

- apb/apb5 master ports use apb{4,5}_to_axi4 front-end converters inside the adapter.

- wb4 master ports use wb4_to_axi4.

- axi3 master ports use axi3_to_axi4.

- apb, apb5, axil, axil5, wb4, and axi3 slave ports use the corresponding completer shim (axi4_to_apb4_shim, axi4_to_wb4, etc.).

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 9	Monitor and CFG Subsystem Keys

Bridge can optionally emit a monitor infrastructure and a cfg register block. This section documents the keys that control those subsystems.

## 9.1	Monitor subsystem

The "mon" variant wraps each enabled port with a _monlite timing wrapper. The wrapper exposes a monbus_* packet interface that feeds a per-bridge monbus_arbiter and, by default, a monbus_axil4_axil4_group.

### 9.1.1	Bridge-level monitor keys

Bridge-level monitor keys

| Key | Type | Default | Description |
| --- | --- | --- | --- |
| mon_preset | string | "lite" | Baseline reporter enables. Since 2026-09-26 all monitored ports use axi_monitor_lite, so every preset resolves to the same hardware; the field is kept for backward-compatible config loading. |
| use_all_monitors | boolean | false | Override every port’s use_monitor to true. |
| use_no_monitors | boolean | false | Override every port’s use_monitor to false. Mutually exclusive with use_all_monitors. |

### 9.1.2	Per-port monitor keys

Per-port monitor keys

| Key | Type | Default | Description |
| --- | --- | --- | --- |
| use_monitor | boolean | true | Include the _monlite wrapper on this port in a "mon" variant. |
| mon_add | list | [] | Extra reporter cones to enable beyond the preset. |
| mon_remove | list | [] | Reporter cones to disable from the preset. |

MON_CONES in bridge_pkg/config.py defines the valid cone names: error, timeout, compl, threshold, perf, debug.

### 9.1.3	[bridge.mon_group] table

This table selects the internal monbus aggregation module. See “Bridge-Level TOML Keys” for the full schema. The selected module is:

f"monbus_{slave_protocol}_{master_protocol}_group"

Examples: monbus_axil4_axil4_group, monbus_axi4_axi4_group.

When internal_axil_group = false, the bridge still instantiates the monbus_arbiter but exposes its aggregated stream as monbus_agg_* top-level outputs, letting an external consumer provide the AXIL group.

## 9.2	CFG register block

Setting use_cfg_regblock = true replaces the 350+ individual cfg_<port>_<idx>_<chan>_* top-level pins with a PeakRDL-generated register block.

### 9.2.1	What changes

- CfgRdlGenerator (bridge_pkg/generators/cfg_rdl_generator.py) builds an RDL schema from the adapter list and BridgeModuleGenerator._MON_GROUP_CFG.

- PeakRDL emits <bridge_name>_cfg_pkg.sv and <bridge_name>_cfg.sv into the bridge directory.

- bridge_generator.py patches the top module to narrow the 32-bit s_cfg_axil_*addr into the regblock’s actual address width.

- The filelist gains the cfg package and body, plus a Verilator waiver for PeakRDL’s field_combo MULTIDRIVEN warning.

### 9.2.2	Per-port cfg fields

CfgAdapterPort is constructed in bridge_generator.py for every master and every non-internal slave. Fields include:

Per-port cfg fields

| Field | Meaning |
| --- | --- |
| name | Port name |
| idx | Index within direction |
| has_wr / has_rd | Which directions exist after channel trimming |
| perfmon_enabled | Whether perf counters exist for this port |
| n_addr_ranges | Number of address ranges (currently 0) |
| addr_width | Address width for this adapter |

The subtractive catch-all slave is deliberately excluded from the cfg map so that adding or removing it does not shift register offsets.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 10	Validation Rules

Configuration validation is centralized in bridge_pkg/config_validator.py. The loader calls validate_config(masters, slaves, connectivity) before any RTL is emitted. Validation failures raise ValidationError and stop generation.

## 10.1	Field validation (validate_required_fields)

- Every port must have name and prefix.

- data_width and addr_width must be positive.

- axi4 masters must have id_width > 0.

- axil/axil5 masters must have id_width == 0.

- apb/apb5/wb4/axi3 masters must have addr_width == 32.

- Slaves must have base_addr and a positive addr_range.

- Slave base_addr and addr_range must be 4 KiB-aligned / a multiple of 4 KiB.

## 10.2	Channel and protocol validation

- channels must be one of rw, wr, rd (validate_channels).

- protocol must be one of axi3, axi4, axi5, apb, apb5, axil, axil5, wb4 (validate_protocol).

- Slaves must explicitly specify channels (validate_slave_channels_explicit).

## 10.3	Protocol-specific constraints

### 10.3.1	APB / APB5 (validate_apb_constraints)

- channels must be rw.

- data_width must be 8, 16, or 32.

- id_width must be 0.

### 10.3.2	Wishbone B4 (validate_wb4_constraints)

- channels must be rw.

- data_width must be 8, 16, 32, or 64.

- id_width must be 0.

### 10.3.3	AXI3 (validate_axi3)

- AXI3 masters must have id_width > 0.

- data_width must be 32, 64, 128, or 256.

- CDC is not supported.

### 10.3.4	AXI5 (validate_axi5)

- axi5_features may appear only on axi5 or axil5 ports.

- Droppable features: nsaid, trace, mpam, mecid, unique, chunking.

- Connectivity-gated features: poison, atomic, mte.

- mte and chunking require data_width >= 128 (validate_axi5_wide_features).

- poison, atomic, mte require every connected path to be native AXI5 with the feature enabled and the same data width (validate_axi5_poison_connectivity).

- An rw AXI5 master with atomic may only connect to rw AXI5 slaves (validate_axi5_atomic_read_return).

- axil5 ports accept only user and exclusive as forwardable features (validate_axil5_features).

## 10.4	Master-slave compatibility (validate_master_slave_compatibility)

- A wr master cannot connect to a rd slave.

- A rd master cannot connect to a wr slave.

- Compatibility is checked only for connected pairs (per the CSV connectivity matrix).

## 10.5	Address map validation (validate_address_map)

- Positively-decoded slave windows must not overlap.

- The subtractive catch-all (internal=true) is excluded from overlap checks.

## 10.6	Slave ID width (validate_slave_id_widths)

With more than one master, every AXI slave port’s ID is widened by $clog2(NUM_MASTERS) master-index bits. The validator checks that each slave’s declared id_width is at least:

widest_master_id_width + clog2(num_masters)

## 10.7	CDC constraints (validate_cdc_constraints)

- cdc=true is legal only on AXI4 ports, master (GitHub #119 L2) or external slave.

- Internal slaves cannot be CDC ports.

## 10.8	Warning for dropped sideband (warn_axi5_dropped_sideband)

Droppable AXI5 sideband that terminates mid-path prints a generation-time warning naming the path and the reason (non-AXI5 peer, missing feature, or width converter).

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 11	Pipeline Overview

The Bridge generation pipeline has five stages:

- Parse — TOML and CSV files are read into Python data structures.

- Validate — validate_config() checks semantic and protocol rules.

- Translate — PortSpec objects are converted to MasterConfig/SlaveInfo objects consumed by the emitter.

- Emit RTL — BridgeModuleGenerator.generate_all() writes package, adapters, crossbar, and top module.

- Emit collateral — filelist, copied configs, optional regblock, optional tests.

Each variant (no, mon) runs stages 3–5 independently and writes to its own directory.

## 11.1	Pipeline diagram

### 11.1.1	Figure 3.1: Bridge Generator Pipeline

Bridge generator pipeline

Configuration enters at the top and passes down the center column: load, validate, then one pass per variant into the RTL emitter, which fans out to one sub-generator per file kind. Every output lands on the emitted-artifacts bar at the bottom. The dashed boxes run only when asked for: the cocotb test generator under --generate-tests, and the PeakRDL config regblock for a use_cfg_regblock bridge’s mon variant. Nothing in the generator arbitrates, so the figure has no trapezium.

## 11.2	Key files by stage

Key files by stage

| Stage | File | Function |
| --- | --- | --- |
| Parse | bridge_pkg/config_loader.py | load_config() reads TOML and CSV. |
| Validate | bridge_pkg/config_validator.py | validate_config() runs all checks. |
| Translate | bridge_generator.py | _emit_bridge_variant() builds MasterConfig/SlaveInfo. |
| Emit RTL | bridge_pkg/components/bridge_module_generator.py | BridgeModuleGenerator.generate_all() orchestrates sub-generators. |
| Emit adapters | bridge_pkg/generators/adapter_generator.py | AdapterGenerator emits master adapters. |
| Emit slave adapters | bridge_pkg/generators/slave_adapter_generator.py | SlaveAdapterGenerator emits slave adapters. |
| Emit crossbar | bridge_pkg/generators/crossbar_generator.py | CrossbarGenerator emits the internal crossbar. |
| Emit package | bridge_pkg/generators/package_generator.py | PackageGenerator emits the SV package. |
| Emit filelist | bridge_generator.py | _emit_bridge_variant() writes the .f filelist. |
| Emit tests | bridge_generator.py | generate_tests() / generate_monitor_tests() write cocotb files. |
| Emit cfg | bridge_pkg/generators/cfg_rdl_generator.py | CfgRdlGenerator emits the PeakRDL register block. |

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 12	Configuration Loading

Configuration loading is implemented in bridge_pkg/config_loader.py. The entry point is load_config(ports_file, connectivity_file=None).

## 12.1	TOML parsing

The loader uses Python’s tomllib (or tomli on older Pythons) to read the TOML file. It expects:

- A [bridge] table with scalar keys and [[bridge.masters]] / [[bridge.slaves]] arrays.

- An optional [bridge.mon_group] table.

- An optional [bridge.defaults] table with a skid_depths sub-table.

Hex strings such as "0x10000000" are accepted for base_addr and addr_range.

## 12.2	Connectivity CSV

The connectivity matrix is a CSV with the first column labeled master\slave and subsequent columns named after slaves. Example:

master\slave,ddr,sram,apb_periph
cpu,1,1,0
dma,1,0,1

If connectivity_file is None, find_connectivity_csv(ports_file) searches for <stem>_connectivity.csv next to the TOML file.

The loader builds a dictionary connectivity[master_name] = set(slave_names).

## 12.3	BridgeConfig construction

load_config() returns a BridgeConfig dataclass (bridge_pkg/config.py) populated with:

- masters: list of PortSpec (direction=“master”)

- slaves: list of PortSpec (direction=“slave”)

- connectivity: dict mapping master names to sets of slave names

- bridge-level fields such as variants, xbar_pipeline, arbitration, mon_preset, use_cfg_regblock, mon_group

The subtractive catch-all slave is not added by the loader; BridgeModuleGenerator appends it internally during emission.

## 12.4	Normalization

- Port prefixes are not normalized by the loader; normalization happens later in bridge_generator.py via _normalize_prefix().

- MonGroupConfig normalizes the deprecated "axil" spelling to "axil4" in __post_init__.

- dest_width and id_width are not relevant to Bridge; those are Delta-specific.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 13	Validation

Validation runs immediately after loading. The function validate_config(masters, slaves, connectivity) in bridge_pkg/config_validator.py is the single gatekeeper.

## 13.1	Validation order

- Per-port required fields — validate_required_fields(port) for every master and slave.

- Channel and protocol values — validate_channels() and validate_protocol().

- Protocol constraints — APB, WB4, AXI3, CDC.

- AXI5 feature validation — validate_axi5(), wide-feature checks, poison connectivity, atomic read-return rules, axil5 feature checks.

- Slave ID width — validate_slave_id_widths().

- Address map — validate_address_map().

- Master-slave compatibility — validate_master_slave_compatibility().

- Dropped-sideband warnings — warn_axi5_dropped_sideband() (non-fatal).

## 13.2	Error behavior

Every validator raises ValidationError(message) on failure. bridge_generator.py catches the exception, prints the traceback, and returns (False, []) so the bulk driver can count the failure.

## 13.3	Examples of validation failures

Examples of validation failures

| Scenario | Validator | Message highlights |
| --- | --- | --- |
| channels = "rx" | validate_channels | Invalid channels 'rx'... Must be one of {'rw', 'rd', 'wr'} |
| protocol = "axi6" | validate_protocol | Invalid protocol 'axi6'... Must be one of {'axi3', 'axi4', 'axi5', 'apb', 'apb5', 'axil', 'axil5', 'wb4'} |
| APB with id_width = 4 | validate_apb_constraints | must have id_width=0 |
| AXI5 poison on a path to an AXI4 slave | validate_axi5_poison_connectivity | connected slave cannot carry it natively |
| Overlapping slave windows | validate_address_map | Address overlap detected |
| wr master to rd slave | validate_master_slave_compatibility | cannot connect to read-only slave |

## 13.4	Adding a new validation rule

To add a rule:

- Write a validator function that accepts (masters, slaves, connectivity) or a single PortSpec.

- Call it from validate_config() at the appropriate point in the order above.

- Update this chapter and the configuration chapter with the new rule.

Keep validators stateless and raise ValidationError only; do not mutate the config.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 14	Adapter and Crossbar Emission

After validation, bridge_generator.py calls _emit_bridge_variant() for each variant in BridgeConfig.variants. That function translates PortSpec objects into MasterConfig/SlaveInfo objects and drives BridgeModuleGenerator.

## 14.1	Translation to emitter structs

MasterConfig and SlaveInfo are lightweight structs consumed by the generators. Key fields:

Translation to emitter structs

| Field | MasterConfig | SlaveInfo |
| --- | --- | --- |
| name | port name | port name |
| prefix | normalized prefix | normalized prefix |
| data_width | int | int |
| addr_width | int | int |
| id_width | int | int |
| channels | rw/wr/rd | rw/wr/rd |
| slave_connections | list of slave indices | — |
| protocol | string | string |
| use_monitor | bool | bool |
| mon_enables | dict | dict |
| axi5_features | list | list |
| cdc | bool | bool |
| enable_ooo | — | bool |
| internal | — | bool |

## 14.2	BridgeModuleGenerator construction

_emit_bridge_variant() instantiates BridgeModuleGenerator with:

BridgeModuleGenerator(
    bridge_name=output_name,
    enable_monitoring=use_monitor,
    internal_axil_group=internal_axil_group,
    use_all_monitors=...,
    use_no_monitors=...,
    use_cfg_regblock=...,
    mon_group=...,
    xbar_pipeline=...,
    arbitration=...,
    qos_aging_shift=...,
    id_track_bits=...,
)

Then it calls gen.add_master(master) and gen.add_slave(slave) for every port, and finally gen.generate_all(bridge_dir).

## 14.3	Sub-generators

BridgeModuleGenerator.generate_all() returns a dict mapping a tag to a file path. The tags are:

Sub-generators

| Tag | Generator class | Output file |
| --- | --- | --- |
| package | PackageGenerator | <bridge>_pkg.sv |
| adapter_<name> | AdapterGenerator | <name>_adapter.sv |
| slave_adapter_<name> | SlaveAdapterGenerator | <name>_adapter.sv |
| bridge | BridgeModuleGenerator itself | <bridge>.sv |
| crossbar | CrossbarGenerator | <bridge>_xbar.sv |

The package is always emitted first so later modules can import it. Adapter file names are based on the port name, not the prefix.

## 14.4	Master adapter (AdapterGenerator)

The master adapter:

- Declares the external port with the configured prefix and channel subset.

- Optionally instantiates a timing wrapper (axi4_slave_wr/rd, _monlite variants, or AXI5 boundary wrappers).

- Optionally instantiates a protocol converter (apb4_to_axi4, axi3_to_axi4, wb4_to_axi4, axi5_atomic_filter, etc.).

- Generates per-width output paths (32b, 64b, 128b, 256b, 512b) and inserts axi4_dwidth_converter_{wr,rd} or axil_to_axi4_wide_align_{wr,rd} only where needed.

- Emits address decode logic producing <name>_slave_select_aw / ar and a bridge_id sideband.

## 14.5	Slave adapter (SlaveAdapterGenerator)

The slave adapter:

- Declares the external slave port with the configured prefix and protocol.

- Optionally instantiates a timing wrapper or protocol shim (axi4_to_apb4_shim, axi4_to_axi3, axi4_to_wb4, AXI5 boundary wrappers).

- Carries the bridge_id response tracker: bridge_cam for an AXI4/AXI5 slave of a multi-master bridge or an enable_ooo slave (SlaveAdapterGenerator.use_cam), an in-order FIFO otherwise.

- Handles CDC if cdc=true by inserting axi4_cdc_{wr,rd}.

## 14.6	Crossbar (CrossbarGenerator)

The crossbar module wires all master-adapter outputs to all slave-adapter inputs. It contains:

- AW and AR request matrices.

- Per-slave round-robin arbiters (or QoS arbiters when arbitration="qos").

- W-owner FIFOs and response routing by bridge_id.

- Optional registered crossbar stage when xbar_pipeline=true.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 15	Filelist and Test Generation

After RTL emission, _emit_bridge_variant() writes the filelist and optionally generates cocotb tests.

## 15.1	Filelist generation

The filelist is written to rtl/filelists/<bridge_name>.f, two levels above the bridge directory. It is a complete compile closure using $REPO_ROOT-relative paths.

### 15.1.1	Filelist sections

- Include directory — +incdir+$REPO_ROOT/rtl/amba/includes.

- Reset macro header — -f $REPO_ROOT/rtl/amba/filelists/reset_defs.f.

- Monitor packages — -f $REPO_ROOT/rtl/amba/filelists/monitor_pkgs.f (always included, even for "no" variants, because the subtractive catch-all references them).

- Subtractive catch-all — -f $REPO_ROOT/rtl/amba/filelists/axi4_subtractive_slave.f.

- Generated files — package first, then other generated files sorted by tag.

- AXI4/AXI5 wrapper closures — axi4_slave_wr.f, axi4_slave_rd.f, axi4_master_wr.f, axi4_master_rd.f, plus AXI5 variants when needed.

- Converter closures — axi_data_upsize.f, axi_data_dnsize.f, axi4_dwidth_converter_{wr,rd}.f, axil_to_axi4_wide_align_{wr,rd}.f.

- Protocol shim closures — APB, AXI-Lite, AXI3, Wishbone, AXI5 atomic filters/read-return trackers as needed.

- CDC closures — axi4_cdc_wr.f, axi4_cdc_rd.f if any port, master or slave, has cdc=true.

- Monitor infrastructure — _monlite wrapper filelists and monbus_arbiter.f, plus the selected monbus_<p1>_<p2>_group.f for "mon" variants.

- Verilator waiver — bridge_regblock.vlt when a cfg regblock is present.

The filelist is generated by string construction in bridge_generator.py, not by a template. Every dependency is pulled in via the owning component’s closure filelist rather than by hand-listing individual .sv files.

## 15.2	Test generation

When --generate-tests is passed:

- For "no" variants, generate_tests() emits a TB class in dv/tbclasses/ and a test runner in dv/tests/.

- For "mon" variants, generate_monitor_tests() emits a TB class and a monitor-stress test runner.

Both functions use Jinja2 templates from bin/bridge_pkg/jinja_templates/:

Test generation

| Template | Output |
| --- | --- |
| bridge_tb_class.py.j2 | dv/tbclasses/<class_module>.py |
| bridge_test_file.py.j2 | dv/tests/test_<bridge>.py |
| bridge_monitor_test.py.j2 | dv/tests/test_<bridge>_monitor.py |

The TB class name is derived from the bridge name by build_tb_class_name(). The import block is selected by _tb_import_block(); if the output path is not dotted-importable (for example, because of the hyphen in fabric-gen-ip), the generated test falls back to a sys.path insertion.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 16	Directory Layout

For each variant the generator creates a directory under the requested output dir:

rtl/generated/<bridge_name>/
├── <bridge_name>_pkg.sv
├── <bridge_name>.sv
├── <bridge_name>_xbar.sv
├── <master0>_adapter.sv
├── <master1>_adapter.sv
├── ...
├── <slave0>_adapter.sv
├── <slave1>_adapter.sv
├── ...
├── <bridge_name>.toml              (copied source config)
├── <bridge_name>_connectivity.csv  (copied connectivity matrix)
├── <bridge_name>_cfg_pkg.sv        (when use_cfg_regblock=true)
└── <bridge_name>_cfg.sv            (when use_cfg_regblock=true)

rtl/filelists/
└── <bridge_name>.f

If --generate-tests is used, the following are also written:

dv/tbclasses/
└── <tb_class_module>.py
dv/tests/
└── test_<bridge_name>.py

For "mon" variants the test runner is named test_<bridge_name>_monitor.py.

## 16.1	Variant directories

The "no" variant writes to <base_name>/. The "mon" variant writes to <base_name>_mon/.

Example for a config with name = "bridge_mix_e" and variants = ["no"]:

rtl/generated/bridge_mix_e/
├── bridge_mix_e_pkg.sv
├── bridge_mix_e.sv
├── bridge_mix_e_xbar.sv
├── cpu_axi3_adapter.sv
├── dma_axi4_adapter.sv
├── host_apb_adapter.sv
├── ddr_axi3_adapter.sv
├── sram_axi4_adapter.sv
├── apb_periph_adapter.sv
├── subtractive_adapter.sv
├── bridge_mix_e.toml
└── bridge_mix_e_connectivity.csv

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 17	Package File

The package file (<bridge_name>_pkg.sv) is generated by PackageGenerator and is always the first file compiled.

## 17.1	What it contains

- BRIDGE_ID_WIDTH — width of the internal bridge_id sideband.

- NUM_MASTERS / NUM_SLAVES — port counts.

- Localparam constants derived from the topology.

- Typedefs for the internal AXI4 structs used by the crossbar and adapters, such as axi4_aw_t, axi4_w_<width>_t, axi4_b_t, axi4_ar_t, axi4_r_<width>_t.

The exact set of typedefs depends on the unique data widths present in the bridge. For example, bridge_mix_e uses 32-bit and 64-bit paths, so its package defines axi4_w_32b_t, axi4_r_32b_t, axi4_w_64b_t, and axi4_r_64b_t.

## 17.2	Excerpt from a generated package

package bridge_mix_e_pkg;

    localparam int NUM_MASTERS = 3;
    localparam int NUM_SLAVES  = 4;  // includes subtractive catch-all
    localparam int BRIDGE_ID_WIDTH = 2;

    typedef struct packed {
        logic [3:0]  id;
        logic [31:0] addr;
        logic [7:0]  len;
        logic [2:0]  size;
        logic [1:0]  burst;
        logic        lock;
        logic [3:0]  cache;
        logic [2:0]  prot;
        logic [3:0]  qos;
        logic [3:0]  region;
        logic        user;
    } axi4_aw_t;

    // ... additional structs for W, B, AR, R at each used width ...

endpackage : bridge_mix_e_pkg

## 17.3	Why it matters

Because the top module imports this package, any change to widths or counts changes the package and therefore every generated file in the same variant. This is one reason partial regeneration is unsafe.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 18	Top Module

The top module (<bridge_name>.sv) is the integration shell. It instantiates the subtractive catch-all, every master adapter, the crossbar, and every slave adapter.

## 18.1	Module signature

The top module takes no parameters. All widths are fixed at generation time and come from the package:

module bridge_mix_e
    import bridge_mix_e_pkg::*;
(
    input  logic aclk,
    input  logic aresetn,

    // Master 0: cpu_axi3 (rw)
    input  logic [3:0] cpu_axi3_awid,
    input  logic [31:0] cpu_axi3_awaddr,
    // ... all AXI3/AXI4/APB/Wishbone channels per port protocol ...

    // Slave 0: ddr_axi3
    output logic [5:0] ddr_axi3_awid,
    // ...

    // Unmapped-address status (subtractive slave)
    output logic        unmapped_irq,
    output logic [31:0] unmapped_addr,
    output logic [7:0]  unmapped_count,
    input  logic        unmapped_clear
);

The exact port list is derived from the port protocol, channel subset, and widths.

## 18.2	Internal structure

The top module contains:

- axi4_subtractive_slave — always present, terminates unmapped addresses with DECERR and exposes the unmapped_* status pins.

- Master adapter instances — one per master, named <name>_adapter.

- Crossbar instance — <bridge_name>_xbar, wired to all adapter outputs.

- Slave adapter instances — one per external slave plus one subtractive_adapter for the catch-all.

## 18.3	Excerpt: subtractive catch-all instantiation

axi4_subtractive_slave #(
    .AXI_ID_WIDTH   (6),
    .AXI_ADDR_WIDTH (32),
    .AXI_DATA_WIDTH (64),
    .AXI_USER_WIDTH (1),
    .UNIT_ID        (8'd3),
    .AGENT_ID       (16'h5B00)
) u_subtractive (
    .aclk          (aclk),
    .aresetn       (aresetn),
    // ... AXI channels ...
    .o_hit_irq     (unmapped_irq),
    .o_hit_addr    (subtractive_hit_addr),
    .o_hit_count   (unmapped_count),
    .i_hit_clear   (subtractive_hit_clear)
);

## 18.4	Excerpt: adapter wiring pattern

cpu_axi3_adapter u_cpu_axi3_adapter (
    .aclk(aclk),
    .aresetn(aresetn),
    .cpu_axi3_awid(cpu_axi3_awid),
    // ... external AXI3 pins ...
    .slave_select_aw(cpu_axi3_slave_select_aw),
    .bridge_id_aw(cpu_axi3_bridge_id_aw),
    .slave_select_ar(cpu_axi3_slave_select_ar),
    .bridge_id_ar(cpu_axi3_bridge_id_ar),
    // ... width-specific internal paths ...
);

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 19	Adapters and Crossbar

## 19.1	Master adapter (<name>_adapter.sv)

The master adapter sits between an external master and the internal crossbar. Its responsibilities are:

- Expose the external port with the configured prefix and protocol.

- Insert timing-isolation wrappers when requested.

- Handle CDC if cdc=true: axi4_cdc_{wr,rd} between the port (on ext_aclk) and the timing wrapper (on aclk).

- Insert protocol converters for non-AXI4 master protocols (apb, apb5, axil, axil5, wb4, axi3, axi5).

- Generate per-width internal paths and instantiate width converters only where master and slave widths differ.

- Decode addresses and emit slave_select_aw/ar plus bridge_id_aw/ar.

### 19.1.1	Per-width paths

For each unique slave width the master connects to, the adapter emits a separate internal AXI4 path. If the master width matches the slave width, the path is direct; otherwise it uses a converter. Example from bridge_mix_e:

// cpu_axi3 Adapter outputs
logic [NUM_SLAVES-1:0] cpu_axi3_slave_select_aw;
logic [BRIDGE_ID_WIDTH-1:0] cpu_axi3_bridge_id_aw;
// 32b path
axi4_aw_t     cpu_axi3_32b_aw;
axi4_w_32b_t  cpu_axi3_32b_w;
// 64b path
axi4_aw_t     cpu_axi3_64b_aw;
axi4_w_64b_t  cpu_axi3_64b_w;

## 19.2	Slave adapter (<name>_adapter.sv)

The slave adapter sits between the crossbar and an external slave. Its responsibilities are:

- Convert the internal AXI4 face to the external protocol.

- Carry the bridge_id response tracker: bridge_cam or an in-order FIFO.

- Handle CDC if cdc=true.

- Expose APB, AXI-Lite, AXI3, AXI5, or Wishbone pins as configured.

### 19.2.1	Response routing

Each slave adapter stores the issuing master’s bridge_id on the address handshake and hands it back with the response. For an AXI4/AXI5 slave in a bridge with more than one master (or any slave with enable_ooo) the store is a pair of bridge_cam instances keyed by the returned BID/RID, so the slave may complete different IDs out of order. Single-master bridges, protocol-shim slaves and the subtractive slave use an in-order FIFO whose head names the master. With more than one master, slave-side AXI IDs are {BRIDGE_ID, id}, clog2(NUM_MASTERS) bits wider than the master’s.

SlaveAdapterGenerator emits each bridge_cam with DEPTH set to the slave’s max_outstanding, ALLOW_DUPLICATES(1), PIPELINE_EVICT(0), PIPELINE_ALLOC(1) and PIPELINE_FREE(1). The last two register the CAM’s insert and free (GitHub #119), so the crossbar’s arbitrated ID and the returning response ID no longer drive the CAM’s tag match and count update in their own cycle. Responses are still routed in the cycle they arrive. The mechanism is MAS 4.1. Changing these emitted parameters changes every CAM-tracked slave adapter, so regenerate all bridges (make regen) and re-run the full regression (Chapter 5).

## 19.3	Crossbar (<bridge_name>_xbar.sv)

The crossbar module:

- Accepts per-master decode outputs (slave_select_*, bridge_id_*).

- Builds AW/AR request matrices.

- Instantiates per-slave round-robin arbiters with grant locked to the address handshake.

- Routes W beats via a per-slave W-owner FIFO.

- Muxes responses back to masters using the bridge_id sideband.

- Optionally inserts a registered stage when xbar_pipeline=true.

The crossbar is protocol-agnostic internally; all protocol conversion happens in the adapters.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 20	Filelist

The filelist (<bridge_name>.f) declares the complete compile closure for one variant. It is generated in bridge_generator.py by _emit_bridge_variant().

## 20.1	Path convention

All paths inside the filelist are anchored to $REPO_ROOT. This allows the filelist to be included from another filelist regardless of the including file’s base directory.

## 20.2	Example filelist head

# Include directories
+incdir+$REPO_ROOT/rtl/amba/includes

# Reset macro header (`ALWAYS_FF_RST / `RST_ASSERTED)
-f $REPO_ROOT/rtl/amba/filelists/reset_defs.f

# Monitor packages (must precede any module that references them)
-f $REPO_ROOT/rtl/amba/filelists/monitor_pkgs.f

# Subtractive catch-all slave (unmapped-address terminator)
-f $REPO_ROOT/rtl/amba/filelists/axi4_subtractive_slave.f

# Bridge RTL files (generated)
$REPO_ROOT/projects/components/fabric-gen-ip/bridge/rtl/generated/bridge_mix_e/bridge_mix_e_pkg.sv
$REPO_ROOT/projects/components/fabric-gen-ip/bridge/rtl/generated/bridge_mix_e/bridge_mix_e_xbar.sv
$REPO_ROOT/projects/components/fabric-gen-ip/bridge/rtl/generated/bridge_mix_e/bridge_mix_e.sv
$REPO_ROOT/projects/components/fabric-gen-ip/bridge/rtl/generated/bridge_mix_e/apb_periph_adapter.sv
...

# AXI4 Wrapper modules (timing isolation)
-f $REPO_ROOT/rtl/amba/filelists/axi4_slave_wr.f
-f $REPO_ROOT/rtl/amba/filelists/axi4_slave_rd.f
-f $REPO_ROOT/rtl/amba/filelists/axi4_master_wr.f
-f $REPO_ROOT/rtl/amba/filelists/axi4_master_rd.f

# Width converters
-f $REPO_ROOT/projects/components/utility-ip/converters/rtl/filelists/axi_data_upsize.f
-f $REPO_ROOT/projects/components/utility-ip/converters/rtl/filelists/axi_data_dnsize.f
-f $REPO_ROOT/projects/components/utility-ip/converters/rtl/filelists/axi4_dwidth_converter_rd.f
-f $REPO_ROOT/projects/components/utility-ip/converters/rtl/filelists/axi4_dwidth_converter_wr.f

## 20.3	Filelist is generated, not hand-maintained

Because the converter and wrapper selections depend on the port protocols, the filelist is regenerated every time the bridge is regenerated. Do not hand-edit a generated filelist; instead, change the generator’s selection logic and regenerate.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 21	Generated Tests

When --generate-tests is passed, the generator emits a cocotb testbench class and a pytest runner for each variant.

## 21.1	TB class

The TB class is written to dv/tbclasses/<tb_class_module>.py. Its name is derived from the bridge name:

bridge_2x2_rw  → Bridge2x2RwTB
bridge_mix_e   → BridgeMixETB

The class inherits from TBBase and implements:

- setup_clocks_and_reset() — drives aclk and aresetn.

- assert_reset() — drives self.dut.aresetn.value = 0.

- deassert_reset() — releases reset.

It constructs GAXIMaster / GAXISlave BFMs per port using the AXI4 field configs derived from the generated widths.

## 21.2	Test runner

The test runner (dv/tests/test_<bridge>.py) contains cocotb.test functions and pytest wrappers. Each wrapper names its testcase= explicitly. Example shape:

@cocotb.test()
async def basic_read_write(dut):
    tb = Bridge2x2RwTB(dut)
    await tb.run_basic_read_write()

@pytest.mark.parametrize("seed", [0, 1, 2])
def test_bridge_2x2_rw_basic(seed):
    run_test(
        module="test_bridge_2x2_rw",
        testcase="basic_read_write",
        extra_env={"SEED": str(seed)}
    )

## 21.3	Monitor stress test

For "mon" variants, generate_monitor_tests() emits:

- A TB class with the full monbus/cfg surface.

- test_<bridge>_monitor.py driving backpressure and error injection phases scaled by TEST_LEVEL.

## 21.4	Regeneration discipline for tests

Tests are generated artifacts. After any change to the test templates or to widths that affect BFM construction, regenerate tests with --generate-tests from a clean state. Hand-written tests for generated DUTs should live in separate files (for example, test_bridge_2x2_rw_tracking.py) and not be overwritten by the generator.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 22	Full Regeneration Rule

Generated RTL must be treated like compiler output: when the compiler changes, rebuild everything.

## 22.1	Why partial regeneration is dangerous

- The package file defines widths used by every other generated file.

- Converter selection logic affects adapter port lists and the filelist.

- The bridge_id width is global; a change changes the crossbar and every adapter.

- The filelist pulls in external IP closures; an interface change there must be reflected in every generated bridge.

## 22.2	Required workflow

cd projects/components/fabric-gen-ip/bridge/bin
make clean
make all

If tests are generated:

python bridge_generator.py --bulk bridge_batch.csv --generate-tests
cd ../dv/tests
make clean-all && make run-all-func

## 22.3	Code changes that trigger full regeneration

- bin/bridge_generator.py

- Any file under bin/bridge_pkg/

- Any Jinja template under bin/bridge_pkg/jinja_templates/

- Any shared converter/wrapper IP whose interface changed

## 22.4	Files that are never regenerated

- rtl/bridge_cam.sv — hand-written CAM, instantiated by the slave adapters of multi-master AXI4/AXI5 slaves.

- Anything outside rtl/generated/ and rtl/filelists/.

## 22.5	Version-mismatch symptoms

- Tests that previously passed now fail.

- Simulation errors about missing signals or mismatched widths.

- Address decode routing to wrong slaves.

- Verilator warnings about undeclared identifiers in the package.

If any of these appear after a generator change, the first recovery step is make clean && make all.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 23	Generated Examples

The checked-in rtl/generated/ directory contains a variety of fixtures that serve as regression anchors and integration examples.

## 23.1	Representative fixtures

Representative fixtures

| Fixture | Masters | Slaves | Notable features |
| --- | --- | --- | --- |
| bridge_2x2_rw | 2 | 2 | Plain AXI4, 32-bit addresses, per-port widths. |
| bridge_2x2_axi5 | 2 | 2 | Native AXI5 fabric with MTE/chunking. |
| bridge_1x2_rw_apb5 | 1 | 2 | APB5 slave, single master. |
| bridge_2x2_axi3_masters | 2 | 2 | AXI3 masters on AXI4 fabric. |
| bridge_mix_e | 3 | 3 | AXI3 + AXI4 + APB masters → AXI3 + AXI4 + APB slaves. |
| bridge_1x2_rd_regblock_mon | 1 | 2 | use_cfg_regblock=true, "mon" variant. |
| bridge_2x2_rw_qos_pipe | 2 | 2 | arbitration="qos", xbar_pipeline=true. |

## 23.2	How to verify a regeneration

After make all, every fixture in rtl/generated/ should be byte-identical to the checked-in version if the generator and inputs have not changed. Because the generated files carry a generation-date-free header, a clean regeneration should not introduce spurious diffs.

To compare:

cd projects/components/fabric-gen-ip/bridge
# Regenerate everything
make -C bin clean && make -C bin all
# Check for unexpected diffs
git diff -- rtl/generated/ | head -n 200

(Note: the user instruction forbids running git commands; the command above is for illustration only and is the responsibility of the agent that owns the shared tree.)

If the generator code changed intentionally, the diffs are expected and must be reviewed.

## 23.3	Fixture as documentation

The bridge_mix_e fixture is the canonical mixed-protocol example. Its TOML and generated top module are referenced throughout this specification. To understand how a new protocol or width combination is emitted, generate a minimal fixture and compare it to the closest existing one.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 24	DV Suite

Bridge uses a cocotb/pytest DV suite in dv/tests/. Test classes live in dv/tbclasses/; runners live in dv/tests/.

## 24.1	Test structure

dv/
├── tbclasses/
│   ├── bridge2x2_rw_tb.py          # per-config TB class
│   ├── bridge_mix_e_tb.py
│   └── ...
└── tests/
    ├── conftest.py
    ├── test_bridge_2x2_rw.py       # generated
    ├── test_bridge_mix_e.py        # generated
    ├── test_bridge_2x2_rw_tracking.py   # hand-written
    ├── test_bridge_1x2_rw_axi5a_atomics.py
    └── ...

## 24.2	Running tests

Use the area Makefile, not bare pytest:

cd projects/components/fabric-gen-ip/bridge/dv/tests

# Specific test, gate level
make run-bridge_2x2_rw-gate

# All functional tests
make clean-all && make run-all-func

# Full regression
make clean-all && make run-all-full

Bridge tests run serially because each test consumes approximately 1 GB of memory.

## 24.3	Test levels

Each generated wrapper exports REG_LEVEL cells:

Test levels

| Level | Cells | Description |
| --- | --- | --- |
| GATE | 72 | Smoke / gate-level depth |
| FUNC | 144 | Functional depth |
| FULL | 216 | Stress depth |

The TB reads the depth profile from dv/tbclasses/bridge_levels.py.

## 24.4	Verification coverage

The DV suite covers:

- Basic read/write connectivity per master-slave pair.

- Boundary probe (address decode miss and subtractive catch-all).

- Arbitration fairness and multi-master contention.

- Burst transfers and packet atomicity.

- Width conversion paths.

- APB/AXI-Lite/AXI3/Wishbone slave conversions.

- AXI5 sideband propagation and atomics.

- Monitor stress (*_mon_monitor.py) and cfg regblock programming.

## 24.5	When generator changes affect DV

After any generator change that affects the generated port list, width, or filelist, regenerate the tests and run make clean-all && make run-all-func before declaring the change safe.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 25	Lint and Synthesis Checks

Every generated bridge is expected to pass Verilator lint and out-of-context synthesis.

## 25.1	Verilator lint

The rtl/Makefile runs lint:

cd projects/components/fabric-gen-ip/bridge/rtl
make verilator

This lints every generated fixture using its filelist. The generator aims for lint-clean output; any new warning class must be either fixed in the generator or documented with a scoped waiver.

## 25.2	Out-of-context synthesis

The FPGA flow in projects/components/fabric-gen-ip/bridge/fpga/ supports out-of-context synthesis:

cd projects/components/fabric-gen-ip/bridge/fpga
make synth BRIDGE=bridge_2x2_rw PART=xc7a100tcsg324-1 CLK_NS=10

Resource and timing tables in the HAS/MAS are produced by this flow.

## 25.3	When to run checks

Run lint and at least one synthesis check after:

- Adding a new protocol or option.

- Changing the adapter or crossbar emission templates.

- Updating converter/wrapper dependencies.

A generator change that passes simulation but fails synthesis is not done.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 26	Adding a Protocol

Adding a new protocol to Bridge means adding a new value to the protocol enum, validating it, selecting the right converter/wrapper in the adapters, and updating the filelist logic.

## 26.1	Step 1: Add the protocol to the validator

In bridge_pkg/config_validator.py:

- Add the protocol string to valid_protocols in validate_protocol().

- Add a new validate_<proto>_constraints() function if the protocol has special rules (for example, allowed widths, channel restrictions, ID width rules).

- Call the new validator from validate_config().

## 26.2	Step 2: Add converter/wrapper selection

In bridge_pkg/generators/adapter_generator.py:

- For a master protocol, decide which front-end converter module to instantiate.

- Add the protocol to the width-converter path selection.

- Emit the correct external port signals.

In bridge_pkg/generators/slave_adapter_generator.py:

- For a slave protocol, decide which completer shim to instantiate.

- Wire the internal AXI4 face to the shim.

## 26.3	Step 3: Update the filelist

In bridge_generator.py _emit_bridge_variant():

- Add if any(m.protocol.lower() == '<proto>' for m in config.masters): block to pull in the master-side converter closure.

- Add the corresponding slave-side block.

Always -f the converter’s own closure filelist; do not hand-list individual .sv files.

## 26.4	Step 4: Add a representative fixture

Add a TOML/CSV pair to bin/test_configs/ and a line to bridge_batch.csv so the new protocol is exercised in regression.

## 26.5	Step 5: Run the full regeneration and test

cd projects/components/fabric-gen-ip/bridge/bin
make clean && make all
python bridge_generator.py --bulk bridge_batch.csv --generate-tests
cd ../dv/tests
make clean-all && make run-<new_fixture>-func

## 26.6	Example: how APB was added

APB master support uses apb4_to_axi4 / apb5_to_axi4 front-end converters. APB slave support uses axi4_to_apb4_shim / axi4_to_apb5_shim. The validator enforces channels="rw", data_width in {8,16,32}, and id_width=0. The filelist pulls in the converter closures conditionally.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 27	Adding a Bridge-Level Option

Bridge-level options live in the [bridge] TOML table and become fields of BridgeConfig.

## 27.1	Step 1: Add the dataclass field

In bridge_pkg/config.py, add a field to BridgeConfig with a sensible default:

@dataclass
class BridgeConfig:
    ...
    my_option: bool = False

## 27.2	Step 2: Add the TOML loader mapping

In bridge_pkg/config_loader.py, read the new key from the TOML tree and assign it to the BridgeConfig instance.

## 27.3	Step 3: Validate if necessary

If the option has legal-value restrictions or conflicts with other options, add a validator in bridge_pkg/config_validator.py and call it from validate_config().

## 27.4	Step 4: Thread it into the emitter

In bridge_generator.py _emit_bridge_variant(), pass the option to BridgeModuleGenerator:

gen = BridgeModuleGenerator(
    ...
    my_option=getattr(config, 'my_option', False),
)

## 27.5	Step 5: Use the option in BridgeModuleGenerator

In bridge_pkg/components/bridge_module_generator.py:

- Accept the option in __init__.

- Store it as an instance attribute.

- Use it to conditionally emit logic, instantiate modules, or modify the port list.

## 27.6	Step 6: Document and test

- Add the key to this GAS chapter 2.

- Add a fixture to bridge_batch.csv that exercises the option.

- Regenerate and run the DV suite.

## 27.7	Example: xbar_pipeline

xbar_pipeline is a bridge-level option that adds a registered skid stage inside the crossbar. It is loaded from TOML, stored in BridgeConfig, passed to BridgeModuleGenerator, and used by CrossbarGenerator to emit extra pipeline registers.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 28	Adding a Port-Level Option

Port-level options are fields of PortSpec. They affect one port at a time.

## 28.1	Step 1: Add the PortSpec field

In bridge_pkg/config.py, add a field to PortSpec with a default:

@dataclass
class PortSpec:
    ...
    my_port_option: bool = False

## 28.2	Step 2: Load it from TOML

In bridge_pkg/config_loader.py, copy the TOML key into the PortSpec constructor for both masters and slaves.

## 28.3	Step 3: Validate it

If the option has constraints, add a validator in bridge_pkg/config_validator.py. For example, cdc is validated by validate_cdc_constraints() to ensure it is only used on AXI4 slave ports.

## 28.4	Step 4: Thread it into MasterConfig / SlaveInfo

In bridge_generator.py _emit_bridge_variant(), pass the option through when building MasterConfig or SlaveInfo:

master_config = MasterConfig(
    ...
    my_port_option=getattr(master_spec, 'my_port_option', False),
)

## 28.5	Step 5: Use it in the adapter generator

In bridge_pkg/generators/adapter_generator.py or slave_adapter_generator.py, read the option from the config struct and emit conditional logic.

## 28.6	Step 6: Update the filelist if needed

If the option requires additional IP, add conditional filelist lines in bridge_generator.py.

## 28.7	Example: cdc

The cdc port option is a PortSpec field. It is validated as AXI4-only, passed through SlaveInfo and MasterConfig, and used by SlaveAdapterGenerator and AdapterGenerator to insert axi4_cdc_{wr,rd} (behind the slave wrapper, ahead of the master wrapper); the bridge top adds the port clock pins and the TB template starts and resets each port clock. The filelist conditionally pulls in axi4_cdc_wr.f and axi4_cdc_rd.f.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 29	Testing an Extension

Any generator extension must be proven by regeneration, lint, and DV.

## 29.1	Minimum test plan

- Single-bridge generation

- python bridge_generator.py --ports test_configs/my_extension.toml

- Verify the output directory contains the expected files and that the new logic appears in the generated .sv.

- Validation failure paths

- Create deliberately invalid TOML files and confirm the generator raises ValidationError with a clear message.

- Bulk regeneration

- make clean && make all

- Confirm no fixture regresses.

- Lint

- cd ../rtl && make verilator

- DV smoke

- cd ../dv/tests
make clean-all && make run-<fixture>-func

- Synthesis sanity

- For area-sensitive options, run out-of-context synthesis on the smallest fixture that exercises the option.

## 29.2	Regression fixtures

Add at least one fixture to bridge_batch.csv that exercises the new protocol or option. If the option is off by default, also add a fixture with the option enabled.

## 29.3	Documentation

Update this GAS and CLAUDE.md if the extension changes the way users configure or extend the generator. Keep the validator rule list, the configuration tables, and the extension-guide steps in sync.