RTL Design Sherpa

STREAM Characterization Monbus Compression Report

June 2026

Table of Contents

Readme	4

Monbus Compression — Characterization (in-core perf-monitor build)	4

1. What compression means here	4

2. Headline (real capture, 682 records, h = 0.944)	5

3. Per-record classification	6

4. Half-beat wire format	7

5. Reduction vs stream length — CAM warm-up	7

6. Live on-board confirmation	8

6.1 Reproducibility — root-caused (was a reset bug, now fixed)	9

6.2 OPEN — cluster state-accumulation wedge (not a clean-state bug)	9

7. What we learned	10

Appendix: data files & reproduce	10

List of Tables

No tables in this document.

# Readme

Generated: 2026-06-22

# Monbus Compression — Characterization (in-core perf-monitor build)

Companion to ../perf/. Where the perf report measures how hard the DMA drives the bus, this measures how well the monbus compressor + half-beat packer shrink the monitor-event stream on real captured traffic.

Provenance. The compressor RTL and packet format are unchanged by the RFC Stage E in-core perf-monitor work / arm-gap fix, so the locked 682-record golden capture and its reductions (current 63.0%, half-beat 75.7%) still stand. Re-confirmed live on the 2026-06-21 in-core perf-monitor build (board 210292B7D46F, 8desc_1ch_1MB --mon-config debug-compl): CRC ok, decode OK, 63.5% reduction — consistent with the golden capture.

Bitstream: all-fixes build (USE_MON_COMPRESSION = USE_MON_HALFBEAT = USE_MON_CAM_PIPELINE = 1), WNS +0.048 ns @ 100 MHz, xc7a100t. Primary data: the committed encoder CAM logic replayed bit-exact and offline on the real 682-record on-board capture (../compression_dataset/desc_axi_16desc_8ch_1MB.json). The on-chip packer is verified bit-identical to this model in cosim (val/amba/test_monbus_halfbeat.py), so the offline reduction is what the FPGA emits for the same record stream — it is the codec, not an estimate.

## 1. What compression means here

Each monitor record is a 128-bit packet + 64-bit timestamp. A raw (Tier-0) escape costs 3 × 64-bit beats. The codec removes that two ways:

- Tier-1 templating (64-bit slot): a CAM-hit record collapses to 1 beat. Structural ceiling 1 − 1/3 = 66.7 %.

- Half-beat packing (32-bit slot): two compressible records share one beat (0.5 beat/record), raising the ceiling to 1 − 0.5/3 = 83.3 %.

Realized reduction is dominated by the Tier-1 hit rate h, not slot size:

reduction = h · (1 − b_t/3)
    current  (b_t = 1.0):  h · 0.667     ceiling 66.7 %
    half-beat(b_t = 0.5):  h · 0.833     ceiling 83.3 %

So 80 % needs h ≈ 0.96. The point of measuring on real traffic is to find the actual h and the fraction of hits that fit a 32-bit half-slot.

## 2. Headline (real capture, 682 records, h = 0.944)

codec comparison

| codec | beats | reduction |
| --- | --- | --- |
| raw baseline | 2046 | — |
| current 64-bit | 758 | 63.0 % |
| half-beat 32-bit | 498 | 75.7 % |

Half-beat packing turns the 63.0 % codec into 75.7 % on this trace — a 12.7-percentage-point gain, 260 fewer beats drained over UART for identical monitor coverage.

Why 75.7 % and not the 83.3 % ceiling: h = 0.944 caps half-beat at 0.944 × 0.833 = 78.6 %, and of the 644 hits, 123 fit only a full 64-bit beat (event_data or Δts too wide for a 23-bit half-payload), plus lone-half round-up. The gap to ceiling is real-traffic structure, not codec overhead.

## 3. Per-record classification

tier composition

| class | records | share | cost |
| --- | --- | --- | --- |
| half-fit (Tier-1, 32-bit slot) | 521 | 76 % | 0.5 beat |
| full-only (Tier-1, 64-bit slot) | 123 | 18 % | 1 beat |
| raw escape (Tier-0, CAM miss / overflow) | 38 | 6 % | 3 beats |

Three quarters of records pack into half-slots. The 18 % full-only records are Tier-1 hits whose event_data/Δts exceed the 23-bit half-payload; the 6 % raw escapes are first-occurrence CAM misses (one per distinct template) plus a handful of Δts overflows.

## 4. Half-beat wire format

A half-pair beat is {tag[63:60]=0x4, slotA[59:30], slotB[29:0]} — two 30-bit slots, each sub[1:0] + idx[4:0] + 23-bit payload:

HALF-A (absolute):  sub=1 | idx[4:0] | delta_ts[9:0] | event_data[12:0]
HALF-C (delta):     sub=2 | idx[4:0] | delta_ts[9:0] | ed_delta[11:0] signed
NOP (lone pad):     sub=0

delta_ts < 1024 cyc (~10.2 µs @ 100 MHz) for either form; HALF-A carries event_data < 2^13, HALF-C a signed ed_delta |·| < 2^12 for counters/sequential addresses. Tier-1 records that fit neither stay a full 64-bit beat; a lone trailing half rounds up to a full beat. Widths locked in host/monbus_halfbeat_model.py (lines 50–62), mirrored bit-exact by rtl/amba/shared/monbus_halfbeat_packer.sv.

## 5. Reduction vs stream length — CAM warm-up

reduction vs length

Replaying prefixes of the real capture shows the dominant real-world effect: warm-up amortization. Each distinct template costs one 3-beat raw escape on first sight; until those are paid off they dominate a short stream. As records accumulate the escapes amortize and reduction climbs toward the steady 75.7 %. This is why short captures under-report compression and the full-stream number is the representative one.

## 6. Live on-board confirmation

live confirmation

The same codec is synthesized into the bitstream. hb_measure.py runs the compressed workload, drains the on-chip trace SRAM, and decodes the FPGA-emitted slots with the golden decoder — every clean point below round-trips bit-exact (decode=OK), confirming the HW slot stream is correct. 1-channel descriptor sweep (debug-compl monitor preset):

| records | half-beat beats | reduction |
| --- | --- | --- |
| 4 | 12 | 0.0 % (all first-occurrence escapes) |
| 8 | 17 | 29.2 % |
| 16 | 23 | 52.1 % |
| 32 | 35 | 63.5 % |

The live curve tracks the offline warm-up curve (§5): reduction climbs with record count toward the full-capture 75.7 %.

### 6.1 Reproducibility — root-caused (was a reset bug, now fixed)

An earlier hb_measure run reported 8desc_8ch as 261 beats one time and 405 another for the identical config. This was proven to be a measurement bug, not the codec:

- Localized on HW: 3 properly-reset runs give bit-identical packets and identical per-template timestamp deltas (only the absolute timer base shifts — it cancels in deltas), so the deterministic model yields the same compression every time.

- Root cause: hb_measure fired only the soft-reset pulse, leaving per-channel + stat state stale between back-to-back invocations, which perturbed the captured event stream. Fixed (hb_measure now does the full cluster reset: soft-reset → per-channel reset → clear-stats); with the fix, repeated 8desc_8ch runs are bit-identical (212 rec / 405 beats / 36.3 %, CRC ok, every run).

- The compressor RTL is bit-exact to the golden model in cosim on the exact captured stream (val/amba/test_monbus_halfbeat.py, Phase 2 + DATASET_OVERRIDE). The codec was never implicated.

### 6.2 OPEN — cluster state-accumulation wedge (not a clean-state bug)

Correction. An earlier draft here called this a reproducible 4–7-channel DMA hang / RTL bug. That was wrong. On a freshly reprogrammed board the full 1–8 channel debug-compl sweep passes (all CRC=ok), 8desc_4ch passes 8/8 consecutive runs, and the hb_measure path passes. The failures were an accumulated-state wedge.

After a long session of many mixed runs (wide resp-delay sweeps + the hb_measure compression sweeps here), the cluster reaches a wedged state in which some configs hang (we saw 4–7 active channels under debug-compl time out at 120 s, zero beats, AXI_RD_COMPLETE=AXI_WR_COMPLETE=0). The per-run soft-reset / cluster-reset does not clear it — only a full FPGA reprogram does (make program). So it is a reset-completeness / state-accumulation gap in the harness, not a clean-state config or codec bug, and not data corruption. Tracked in rtl/amba/KNOWN_ISSUES/axi_monitor_blockready_hang_partial_channels.md. The compression numbers in §2–§6.1 are from clean-state runs and are unaffected.

## 7. What we learned

- Half-beat is worth it on real traffic: 63.0 % → 75.7 % (+12.7 pp) on the captured stream, for two 30-bit slots per beat.

- h dominates, not slot size. At h = 0.944 the half-beat ceiling is 78.6 %; we hit 75.7 % of the theoretical 83.3 %.

- Three quarters of records half-pack; 18 % need a full beat (wide payload), 6 % escape (cold templates).

- Warm-up is the real-world tax. Short streams under-report; the cost is one 3-beat escape per distinct template, amortized over the run.

- The HW codec is correct. Live FPGA slots decode bit-exact and track the model; the codec was never the bug. The earlier non-reproducibility was a hb_measure reset bug (fixed, §6.1).

- Separate open issue (§6.2): a long mixed-run session can wedge the cluster so some configs hang — but a freshly reprogrammed board passes the full sweep, so it’s a reset-completeness / state-accumulation gap in the harness, not a clean-state config, codec, or data-corruption bug. Cleared by make program.

## Appendix: data files & reproduce

| File | Contents |
| --- | --- |
| ../compression_dataset/desc_axi_16desc_8ch_1MB.json | real 682-record on-board capture (locked dataset) |
| plots/*.png | figures above (host/plot_compression_report.py) |
| plots/summary.json | numeric summary (records, h, tier split, beats, reductions) |

cd flows-stream-bridge/host && source $REPO_ROOT/env_python
# Authoritative, offline, bit-exact (no board needed):
python3 plot_compression_report.py \
    --capture ../reports/compression_dataset/desc_axi_16desc_8ch_1MB.json \
    --outdir ../reports/compression/plots
# Live on-board confirmation (keep workloads small; trace SRAM = 2048 beats;
# reset the cluster between runs):
python3 hb_measure.py --port /dev/ttyUSB2 --channels 1 \
    --config 8desc_1ch_1MB --mon-config debug-compl

Codec spec: bin/TBClasses/monbus/monbus_compressor.py; RTL: rtl/amba/shared/monbus_{compressor,halfbeat_packer}.sv; cosim: val/amba/test_monbus_{compressor,halfbeat}.py.